3GPP TSG-RAN WG3 Meeting #ad-hoc2	R3-172505

Qingdao, P.R. China, 27th – 29th June 2017

Agenda Item:	10.1

Source:	Ericsson

Title:	TP of PDU session resource

Document for:	Discussions & Approval

# 1	Text Proposal for NGAP TS 38.413

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

---------------------------------

Skip unchanged text

----------------------------------

## 8.2	PDU Session Management Procedures

Editor’s Note:	The message names for PDU Session Management may need further review.

### 8.2.1	PDU Session Resource Setup

#### 8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU Session Resources and the corresponding QoS flows, and to setup corresponding Data Radio Bearers for a given UE. The procedure uses UE-associated signalling.

Editor’s Note:	Further details are FFS.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the gNB.

-	The PDU SESSION RESOURCE REQUEST message shall contain the information required by the gNB to setup PDU session related NG-RAN configuration consisting of at least one PDU Session Resource for each PDU Session Resource to setup included in PDU Session Resource To Be Setup Item IE.

The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the gNB.

Upon reception of the PDU SESSION RESOURCE SETUP REQUEST message, and if resources are available for the requested configuration, the gNB shall execute the requested NG-RAN configuration and allocate associated resources over NG and over Uu for each PDU session listed in the PDU Session Resource To Be Setup Item IE.

For each requested PDU session, if resources are available for the requested configuration, the gNB shall establish at least one or several Data Radio Bearers and associate each accepted QoS flow of the PDU session to a Data Radio Bearer established.

For each PDU session successfully established the gNB shall pass to the UE the NAS-PDU IE and the value contained in the PDU Session ID IE received for the PDU Session. The gNB shall not send to the UE the NAS PDUs associated to the failed PDU sessions.

For each PDU session the gNB shall store the transport layer address and the tunnel endpoint included in the PDU Session Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and use it as the uplink termination point for the user plane data for this PDU session.

For each PDU session for which the PDU Session Type IE is included in the PDU SESSION RESOURCE SETUP REQUEST message and is not set to “IPv4” nor “IPv6”, the gNB shall not perform header compression for the concerned PDU session [FFS].

For each PDU session for which the S-NSSAI IE is included in the PDU SESSION RESOURCE SETUP REQUEST message the gNB shall store the Network Slice Assistance Information corresponding to the concerned PDU session and use it as specified in TS 38.300 [8].

For each PDU session in the PDU SESSION RESOURCE SETUP REQUEST message the gNB shall enforce the traffic corresponding to the received PDU Session Resource Aggregated Maximum Bit Rate IE. The gNB shall use the received Aggregate Maximum Bit Rate for the concerned PDU session and concerned UE as specified in TS 23.501 [9].

For each QoS flow requested to be setup the gNB shall take into account the received QoS Flow Level QoS Parameters IE. For each QoS flow the gNB shall establish or modify the resources according to the values of the Allocation and Retention Priority IE (priority level and pre-emption indicators) and the resource situation as follows:

-	The gNB shall consider the priority level of the requested QoS flow, when deciding on the resource allocation.

-	The priority levels and the pre-emption indicators may (individually or in combination) be used to determine whether the QoS flow setup has to be performed unconditionally and immediately. If the requested QoS flow is marked as “may trigger pre-emption” and the resource situation requires so, the gNB may trigger the pre-emption procedure which may then cause the forced release of a lower priority QoS flow which is marked as “pre-emptable”. Whilst the process and the extent of the pre-emption procedure are operator-dependent, the pre-emption indicators shall be treated as follows:

1.	The values of the last received Pre-emption Vulnerability IE and Priority Level IE shall prevail.

2.	If the Pre-emption Capability IE is set to “may trigger pre-emption”, then this allocation request may trigger the pre-emption procedure.

3.	If the Pre-emption Capability IE is set to “shall not trigger pre-emption”, then this allocation request shall not trigger the pre-emption procedure.

4.	If the Pre-emption Vulnerability IE is set to “pre-emptable”, then this QoS flow shall be included in the pre-emption process.

5.	If the Pre-emption Vulnerability IE is set to “not pre-emptable”, then this QoS flow shall not be included in the pre-emption process.

6.	If the Priority Level IE is set to “no priority” the given values for the Pre-emption Capability IE and Pre-emption Vulnerability IE shall not be considered. Instead the values “shall not trigger pre-emption” and “not pre-emptable” shall prevail.

-	The gNB pre-emption process shall keep the following rules [FFS]:

1.	The gNB shall only pre-empt QoS flows with lower priority, in ascending order of priority.

2.	The pre-emption may be done for QoS flows belonging to the same UE or to other UEs.

The gNB shall report to the AMF in the PDU SESSION RESOURCE SETUP RESPONSE message the result for each individual PDU session in the PDU Session Setup Response Transfer IE. In particular it shall report for each PDU session [FFS]:

-	A list of QoS flows which are successfully established shall be included in the QoS Flows Setup List IE.

-	A list of QoS flows which failed to be established, if any, shall be included in the QoS Flows Failed to Setup List IE.

-	The transport layer address and tunnel endpoint to be used for the PDU session.

Upon reception of the PDU SESSION RESOURCE SETUP RESPONSE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Setup Response Transfer IE to each SMF associated with the concerned PDU session. [FFS]

When the gNB reports unsuccessful establishment of a QoS flow, the cause value should be precise enough to enable the SMF to know the reason for an unsuccessful establishment. [FFS]

Editor’s Note:	Further details are FFS.

---------------------------------

Skip unchanged text

----------------------------------

#### 9.3.1.5	PDU Session Resource Setup List

Editor’s Note: 	The grouping of SM-related IEs using an NGAP SM container transparent to the AMF is subject to confirmation. IE structure needs further checking and completion. Further details FFS.

Editor’s Note:	The name PDU Session Resource is to be confirmed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Setup Item IEs |  | 1 .. <maxnoof PDUSessionResources> |  |  | EACH | reject |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>S-NSSAI | O |  | <ref> |  | - |  |
| >>PDU Session Setup Request Transfer | O |  | <ref> | This IE includes a container from the SMF to the RAN node transparently to the AMF as per section <ref>. | - |  |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>