3GPP TSG-RAN WG3 AH #2	Tdoc R3-172512

Qingdao, P. R. China, 27th – 29th June 2017

Agenda Item:	10.10.1

Source:	Ericsson

Title:	TP for resilience and scalability in a disaggregated gNB

Document for:	pCR

# Introduction

In the following we propose a pCR to capture the proposal in R3-172511.

# pCR for TS 38.472

Start of Text Proposal for TS 38.472

## 7	Transport layer

SCTP (IETF RFC 4960 [5]) shall be supported as the transport layer of F1-c signalling bearer. The Payload Protocol Identifier assigned by IANA to be used by SCTP for the application layer protocol F1AP is xx (FFS).

SCTP refers to the Stream Control Transmission Protocol developed by the Sigtran working group of the IETF for the purpose of transporting various signalling protocols over IP network.

There shall be one or several SCTP associations established between one gNB-CU and gNB-DU pair.

The gNB-DU shall establish the SCTP association. The SCTP Destination Port number value assigned by IANA to be used for F1AP is yyyyy (FFS).

Within the set of SCTP associations established between one gNB-CU and gNB-DU pair a single SCTP association shall be used for the sole use of F1AP elementary procedures that utilize F1AP signalling not related to a UE, utilising a single pair of stream identifiers. This SCTP association is denoted as the “primary” SCTP association.

Within an SCTP association established between one gNB-CU and gNB-DU pair utilised for UE-associated signalling:

-	At least one pair of stream identifiers shall be reserved for the sole use of F1AP elementary procedures that utilize UE-associated signallings. However, a few pairs (i.e. more than one) should be reserved.

-	A single UE-associated signalling shall use one SCTP stream and the stream should not be changed during the communication of the UE-associated signalling as long as the same SCTP associations is used.

Transport network redundancy may be achieved by SCTP multi-homing between two end-points, of which one or both is assigned with multiple IP addresses. SCTP end-points shall support a multi-homed remote SCTP end-point. For SCTP endpoint redundancy an INIT may be sent from gNB-CU or gNB-DU, at any time for an already established SCTP association, which shall be handled as defined in IETF RFC 4960 [5] in sub clause 5.2.

The SCTP congestion control may, using an implementation specific mechanism, initiate higher layer protocols to reduce the signalling traffic at the source and prioritise certain messages.

End of Text Proposal for TS 38.472