3GPP TSG-RAN WG3 NR AdHoc                            R3-172546

Qingdao, China, 27-29 June 2017

Agenda item:		10.12.5.2

Source:	Altiostar Networks, Vodafone, Orange, SKT, Intel

Title:	F1 User Plane Enhancement for Flow Control

Document for:		Discussion and Decision

# 1	Introduction

User plane enhancement for flow control was discussed in RAN3#96 [1-5]. In this contribution, we compare the flow control approaches in CU-DU split architecture and propose enhancements to the F1 User Plane specification.

# 2	General Principle

Figure 1 illustrates the overall flow control principle in CU-DU split architecture. The incoming PDCP PDUs are divided into multiple bearers for multiple cells. The exact proportion of split depends on cell loading, radio condition of the link, and the RLC buffer status. Flow control function optimizes the traffic pattern depending on policy to ensure service quality while avoiding congestion to the network.

In 5G network deployments with CU-DU split architecture, larger number of DUs may be aggregated to CU than previously envisioned due to virtualization and centralization of PDCP layer. It is envisioned to have 1000’s of cells aggregated to CU in a central location. Therefore, the number of traffic flows through the midhaul may be orders of magnitude larger than in conventional deployments where typically a few 10’s of cells are supported in a base station. This results in large bandwidth requirements and increased fluctuation in traffic loading in the midhaul.

Figure 1: Flow control in CU-DU split architecture

There are two requirements for flow control algorithm:

- Traffic management

  - Flow control makes fair utilization of radio resources among cells (eg 20MHz cell vs 100 MHz cell), while ensuring packet delivery under instantaneous variations such as changing transport and radio conditions. Queuing delays between the primary and secondary cells should remain similar.

- Buffer management

  - Flow control reduces the probability of buffer under-run and overrun, while limiting the buffering time at the DU RLC to a desired level. Buffer under-run causes scheduler starvation while buffer overrun causes packet drop, reducing the throughput significantly.

Buffer status reporting and radio resource capability at the DU is critical for flow control. Without buffer status information, over-provisioning of DU RLC buffer is required to avoid packet drop.

Observation 1: Flow control with RLC buffer status reporting allows fair usage of radio resources and buffer management at the RLC layer.

Observation 2: Without accurate flow control, RLC buffer size needs to be over-provisioned resulting in expensive DU implementation.

Various F1 design approaches for flow control were proposed in RAN3#96 [1-5]. In [3], a polling based reporting mechanism is proposed.

- Node controlling PDCP requests PDCP PDU delivery status from the DU.

- PDCP status is reported from DU:

  - The highest SN of PDCP PDU that was successfully transferred from RLC to MAC

  - Queuing delay of PDCP PDU inside DU

- CU may estimate the PDCP PDU RTT based on the delay between the PDCP PDU transmission and reception of the report.

However, queuing delay at DU is not sufficient to support traffic management function required to handle 5G data rate and multi-vendor CU-DU operation. To compute the split ratio of the forwarded packet, current and maximum buffer size and RLC throughput at the DU is required. Furthermore, RLC buffer under-run or overrun cannot be estimated from the queuing delay alone. It is also noted that measurement of queuing delay requires time stamp and tracking of the packets at the RLC layer, which may increase implementation complexity.

# 3	F1 User Plane Specification Aspects

For flow control operation described in [2], following enhancements are required in the F1 User Plane:

Maximum RLC buffer size

During cell setup stage, maximum RLC buffer size is sent from DU to CU.

RLC buffer status reporting

DU sends data forward request to CU. At the time of data forward request, average RLC buffer size and RLC buffer drain rate is reported to CU.

- Average RLC buffer size: Average RLC queue depth in terms of PDCP PDU packets at the time of reporting n. Averaging may be computed based on moving average or by IIR filtering.

- RLC buffer drain rate: Number of RLC SDUs successfully delivered to MAC between reporting time n and previous reporting time n-1

- The data forward request may be event-driven based on RLC buffer status, or configured periodically.

  - In case of peirodic reporting, periodicity is configured by CU

  - Alternativley, polling may be considered with the RLC buffer status reports embedded with PDCP PDU status reporting.

If the above enhancements are agreed for F1, similar enhancement may be applied for X2 and Xn interfaces for Dual Connectivity in 5G.

Proposal 1: It is proposed to introduce enhancements to the F1 User Plane specification for flow control based on RLC buffer status reporting.

Proposal 2: It is proposed to consider similar enhancements in X2 and Xn interfaces for Dual Connectivity.

# 4	Conclusion

In this contribution, we analysed flow control enhancements for F1 user plane. It is observed that

Observation 1: Flow control with RLC buffer status reporting allows fair usage of radio resources and buffer management at the RLC layer.

Observation 2: Without accurate flow control, RLC buffer size needs to be over-provisioned resulting in expensive DU implementation.

Based on these observations, we make the following proposals:

Proposal 1: Introduce enhancements to the F1 User Plane specification for flow control based on RLC buffer status reporting.

Proposal 2: Consider similar enhancements in the X2 and Xn interfaces for Dual Connectivity.

# References

- RAN3 Chairman’s Notes, 3GPP TSG RAN WG3 #96, 3GPP MCC.

- R3-171953, “Flow Control in CU-DU Split Architecture,” Altiostar, Orange, SK Telecom

- R3-171438, “Introduction of F1 User Plane protocol ,” Nokia, Alcatel-Lucent Shanghai Bell

- R3-171814, “Discussions on enhancement of retransmissions over F1 interface,” Huawei

- R3-171599, “Discussion on flow control over F1-U,” ZTE