3GPP TSG RAN WG3 NR AdHoc	R3-172623

Qingdao, China, 27th -29th June 2017

Agenda item:		10.10.1

Source:	Huawei.

Title:					TP on F1 interface management to 38.470

Document for:		pCR

# Introduction

This is the TP for capturing the stage 2 procedure descriptions to 38.470 based on the offline discussion in RAN3 AH NR#2  based on the following minutes from the online session:

CB: # 16_F1_IntMgmt

-  remove “peer”

- check level of detail appropriate to 38.470

(Huawei)

Rev in R3-172623

Compared to the previous version, the term “peer entity” is replaced by “receiving entity”.

# Text proposal for 38.470

# 5	Procedures of the F1 interface

## 5.1	Control plane procedures

### 5.1.x	Interface Management procedures

#### 5.1.x.y1	Reset procedure

The purpose of the Reset procedure is to re-initialize the receiving entity or part of the receiving entity after node setup and after a failure event occurred. This procedure is initiated by both the gNB-DU and gNB-CU. The receiving node releases the corresponding references and resources and replies with an acknowledgement to confirm that the resources and references are cleared.

#### 5.1.x.y2		Error Indication procedure

The Error Indication procedure is initiated by the gNB-DU and the gNB-CU, to report detected errors in one incoming message, if an appropriate failure message cannot be reported to the sending entity.

#### 5.1.x.y3	F1 Setup procedure

The F1 Setup procedure is used to exchange configured data which is required in the gNB-CU and in the gNB-DU respectively to ensure a proper interoperation. The F1 Setup procedure is triggered by the gNB-DU. The gNB-CU responds to indicate the successful or unsuccessful outcome of the procedure.