# 3GPP TSG-RAN WG3 NR#2 Adhoc	R3-172151

# Qingdao, China, 27-29 June 2017

3GPP TSG CT WG1 Meeting #104						C1-172648

Zhangjiajie, P.R of China, 15-19 May 2017

Title:	Reply LS on Applying Extended NAS timers based on UE's operation in CE Mode B

Response to:	LS on Applying Extended NAS timers based on UE's operation in CE Mode B (C1-172113, S2-172744)

Release:	Rel-14

Work Item:	CIoT

Source:	CT1

To:	SA2

Cc:	CT4, RAN3

Contact person:	Vivek Gupta

vivek.g.gupta@intel.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT1 thanks SA2 for their LS on Applying Extended NAS timers based on UE's operation in CE Mode B.

With regards to the following question to CT1;

Does CT1 see any issue in using CE level information at MME to determine if the UE is operating in CE mode B and applies extended NAS timer at MME only if CE level corresponds to CE mode B? SA2 would like CT1 to avoid any unexpected expiry of NAS timer due to transition between CE mode B and CE mode A.

Yes, CT1 does see an issue in the above. CT1 would like to point out that the UE CE mode can change frequently and the MME may not always have up to date information regarding the CE mode the UE is operating in, even if the UE CE mode is communicated to MME in every UL NAS transport message. As such even if the MME applies extended NAS timers using the most up to date information regarding the CE mode the UE is operating in, CT1 would like to indicate that during transition across CE modes when a NAS procedure is ongoing, the NAS timer value is calculated at the start of a NAS procedure, and is not re-calculated until the NAS procedure is completed, restarted or aborted.

As such NAS timers can expire during transition between different CE modes, if NAS timers corresponding to CE mode B are applied only if the UE is operating in CE mode B.

# 2	Actions

To SA2:

ACTION: 	CT1 kindly asks SA2 to take the above information into account.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 105	21 - 25 Aug 2017	Krakow, Poland

TSG CT WG1 Meeting 106	23 - 27 Oct 2017    	India