3GPP TS 36.133 V14.9.0 (2018-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	78

1	Scope	79

2	References	79

3	Definitions, symbols and abbreviations	80

3.1	Definitions	80

3.2	Symbols	82

3.3	Abbreviations	83

3.4	Test tolerances	85

3.5	Additional notation	85

3.5.1	Groups of bands	85

3.6	General	87

3.6.1	Applicability of requirements in this specification version	87

4	E-UTRAN RRC_IDLE state mobility	93

4.1	Cell Selection	93

4.2	Cell Re-selection	93

4.2.1	Introduction	93

4.2.2	Requirements	93

4.2.2.1	Measurement and evaluation of serving cell	94

4.2.2.2	Void	95

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	95

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	97

4.2.2.5	Measurements of inter-RAT cells	99

4.2.2.5.1	Measurements of UTRAN FDD cells	99

4.2.2.5.2	Measurements of UTRAN TDD cells	101

4.2.2.5.3	Measurements of GSM cells	102

4.2.2.5.4	Measurements of HRPD cells	103

4.2.2.5.5	Measurements of cdma2000 1X	104

4.2.2.6	Evaluation of cell re-selection criteria	105

4.2.2.7	Maximum interruption in paging reception	106

4.2.2.8	void	106

4.2.2.9	UE measurement capability	106

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	107

4.2.2.10	Reselection to CSG cells	107

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	107

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	108

4.2.2.11	Void	109

4.2.2.12	Void	109

4.2.2.13	Void	109

4.3	Minimization of Drive Tests (MDT)	109

4.3.1	Introduction	110

4.3.2	Measurements	110

4.3.2.1	Requirements	110

4.3.3	Relative Time Stamp Accuracy	110

4.3.3.1	Requirements	110

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	110

4.3.4.1	 Requirements	110

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	111

4.3.5.1	Requirements for timeSinceFailure	111

4.4	MBSFN Measurements	111

4.4.1	Introduction	111

4.4.2	MBSFN RSRP measurements	111

4.4.3	MBSFN RSRQ measurements	111

4.4.4	MCH BLER measurements	112

4.5	Proximity-based Services	112

4.5.1	Introduction	112

4.5.2	Requirements	112

4.5.2.1	Interruptions with ProSe Direct Discovery	112

4.5.2.2	Interruptions with ProSe Direct Communication	112

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	112

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	113

4.6	Cell Selection and Re-selection Requirements for UE category NB1	113

4.6.1	Cell Selection	113

4.6.2	Cell Re-selection	113

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	114

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	114

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	116

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	116

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	118

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	119

4.6.2.7	Maximum interruption in paging reception in normal coverage	120

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	120

4.6.2.8	UE measurement capability	121

4.7	Cell Selection and Re-selection Requirements for UE category M1	121

4.7.1	Cell Selection	121

4.7.2	Cell Re-selection	121

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	121

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	121

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	121

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	123

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	124

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	124

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	125

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	125

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	126

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	127

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	129

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	129

4.8	Idle State Positioning Measurement Requirements for UE category NB1	130

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	130

4.8.1.1	RSTD Measurement Reporting Delay	131

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	132

4.8.2.1	RSTD Measurement Reporting Delay	133

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	133

4.8.3.1	RSTD Measurement Reporting Delay	135

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	135

4.8.4.1	RSTD Measurement Reporting Delay	136

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	137

4.8.5.1	Measurement Reporting Delay	138

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	138

4.8.6.1	Measurement Reporting Delay	139

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	139

4.8.7.1	Measurement Reporting Delay	140

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	141

4.8.8.1	Measurement Reporting Delay	142

5	E-UTRAN RRC_CONNECTED state mobility	142

5.1	E-UTRAN Handover	143

5.1.1	Introduction	143

5.1.2	Requirements	143

5.1.2.1	E-UTRAN FDD – FDD	143

5.1.2.1.1	Handover delay	143

5.1.2.1.2	Interruption time	144

5.2.2.2	E-UTRAN FDD – TDD	145

5.2.2.2.1	(Void)	145

5.2.2.2.2	(Void)	145

5.2.2.3	E-UTRAN TDD – FDD	145

5.2.2.3.1	(Void)	145

5.2.2.3.2	(Void)	145

5.2.2.4	E-UTRAN TDD – TDD	145

5.2.2.4.1	Handover delay	145

5.2.2.4.2	Interruption time	146

5.2.2.5	E-UTRAN HD–FDD	147

5.2.2.5.1	Handover delay	147

5.2.2.5.2	Interruption time	147

5.3	Handover to other RATs	149

5.3.1	E-UTRAN - UTRAN FDD Handover	149

5.3.1.1	Introduction	149

5.3.1.1.1	Handover delay	149

5.3.1.1.2	Interruption time	149

5.3.2	E-UTRAN - UTRAN TDD Handover	150

5.3.2.1	Introduction	150

5.3.2.2	Requirements	150

5.3.2.2.1	Handover delay	150

5.3.2.2.2	Interruption time	150

5.3.3	E-UTRAN - GSM Handover	151

5.3.3.1	Introduction	151

5.3.3.2	Requirements	151

5.3.3.2.1	Handover delay	151

5.3.3.2.2	Interruption time	151

5.4	Handover to Non-3GPP RATs	151

5.4.1	E-UTRAN – HRPD Handover	151

5.4.1.1	Introduction	151

5.4.1.1.1	Handover delay	152

5.4.1.1.2	Interruption time	152

5.4.2	E-UTRAN – cdma2000 1X Handover	152

5.4.2.1	Introduction	152

5.4.2.1.1	Handover delay	152

5.4.2.1.2	Interruption time	152

5.5	E-UTRAN Handover for Cat-M1 UEs	153

5.5.1	Introduction	153

5.5.2	Requirements in CEModeA	153

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	153

5.5.2.1.1	Handover delay	153

5.5.2.1.2	Interruption time	153

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	154

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	154

5.5.2.3.1	Void	154

5.5.2.3.2	Void	154

5.5.3	Requirements in CEModeB	154

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	154

5.5.3.1.1	Handover delay	154

5.5.3.1.2	Interruption time	155

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	155

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	155

5.6	Void	155

6	RRC Connection Mobility Control	155

6.1	RRC Re-establishment	155

6.1.1	Introduction	156

6.1.2	Requirements	156

6.1.2.1	UE Re-establishment delay requirement	156

6.2	Random Access	156

6.2.1	Introduction	156

6.2.2	Requirements	156

6.2.2.1	Contention based random access	157

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	157

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	157

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	157

6.2.2.1.4	Void	157

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	157

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	157

6.2.2.2	Non-Contention based random access	157

6.2.2.2.1	Correct behaviour when receiving Random Access Response	157

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	157

6.2.3	Requirements for Cat-M1 UEs	158

6.3	RRC Connection Release with Redirection	158

6.3.1	Introduction	158

6.3.2	Requirements	158

6.3.2.1	RRC connection release with redirection to UTRAN FDD	158

6.3.2.2	RRC connection release with redirection to GERAN	158

6.3.2.3	RRC connection release with redirection to UTRAN TDD	159

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	159

6.4.1	Introduction	159

6.4.2	Requirements	159

6.5	RRC Re-establishment for NB-IoT UEs	160

6.5.1	Introduction	160

6.5.2	Requirements	160

6.5.2.1	UE Re-establishment delay requirement in normal coverage	160

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	161

6.6	Random Access for UE category NB1	161

6.6.1	Introduction	161

6.6.2	Requirements	161

6.6.2.1	Correct behaviour when receiving Random Access Response reception	161

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	162

6.6.2.3	Correct behaviour when receiving a NACK on msg3	162

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	162

6.6.2.5	Correct behaviour when contention Resolution timer expires	162

6.6.3	Requirements for NPRACH configuration	162

6.7	RRC Re-establishment for Cat-M1 UEs	162

6.7.1	Introduction	162

6.7.2	Requirements	162

6.7.2.1	UE Re-establishment delay requirement for CEModeA	163

6.7.2.2	UE Re-establishment delay requirement for CEModeB	163

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	164

6.8.1	Introduction	164

6.8.2	Requirements	164

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	164

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	164

6.9.1	Introduction	164

6.9.2	Requirements	165

7	Timing and signalling characteristics	165

7.1	UE transmit timing	165

7.1.1	Introduction	165

7.1.2	Requirements	166

7.2	UE timer accuracy	167

7.2.1	Introduction	167

7.2.2	Requirements	167

7.3	Timing Advance	167

7.3.1	Introduction	167

7.3.2	Requirements	167

7.3.2.1	Timing Advance adjustment delay	167

7.3.2.2	Timing Advance adjustment accuracy	167

7.4	Cell phase synchronization accuracy (TDD)	168

7.4.1	Definition	168

7.4.2	Minimum requirements	168

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	168

7.5.1	Introduction	168

7.5.2	eNodeB Synchronization Requirements	169

7.5.2.1	Synchronized E-UTRAN	169

7.5.2.2	Non-Synchronized E-UTRAN	169

7.6	Radio Link Monitoring	169

7.6.1	Introduction	169

7.6.2	Requirements	170

7.6.2.1	Minimum requirement when no DRX is used	170

7.6.2.2	Minimum requirement when DRX is used	171

7.6.2.3	Minimum requirement at transitions	172

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	172

7.6.2.5	Minimum requirement under IDC Interference	173

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	173

7.7.1	Introduction	173

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	173

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	174

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells	174

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	175

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	175

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	176

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	176

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	177

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	177

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	178

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	178

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	179

7.8	Interruptions with Carrier Aggregation	179

7.8.1	Introduction	179

7.8.2	Requirements	179

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	179

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	180

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	180

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	180

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	180

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	180

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	180

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	181

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	181

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	182

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	182

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	182

7.8.2.13	Interruptions at SRS carrier based switching	183

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	184

7.9.1	Introduction	184

7.9.2	Minimum Requirements for Interband Carrier Aggregation	184

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	184

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	185

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	185

7.10.1	Introduction	185

7.10.2	Requirements	185

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	185

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	185

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	186

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	186

7.11	Radio Link Monitoring for UE Category 0	186

7.11.1	Introduction	186

7.11.2	Requirements for FD-FDD and TDD	188

7.11.2.1	Minimum requirement when no DRX is used	188

7.11.2.2	Minimum requirement when DRX is used	188

7.11.2.3	Minimum requirement at transitions	189

7.11.3	Requirements for HD-FDD	189

7.11.3.1	Minimum requirement when no DRX is used	189

7.11.3.2	Minimum requirement when DRX is used	189

7.11.3.3	Minimum requirement at transitions	190

7.12	Interruptions with Dual Connectivity	190

7.12.1	Introduction	190

7.12.2	Requirements	190

7.12.2.1	Interruptions at PSCell addition/release	190

7.12.2.2	Interruptions at transitions between active and non-active during DRX	191

7.12.2.3	Interruptions at transitions from non-DRX to DRX	191

7.12.2.4	Interruptions at SCell addition/release	191

7.12.2.5	Interruptions at SCell activation/deactivation	191

7.12.2.6	Interruptions during measurements on SCC	192

7.12.2.7	Interruptions at SRS carrier based switching	192

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	193

7.13.1	Definition	193

7.13.2	Minimum requirements	193

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	193

7.14.1	Introduction	193

7.14.2	PSCell Addition Delay Requirement	193

7.14.3	PSCell Release Delay Requirement	194

7.15	Maximum Receive Timing Difference in Dual Connectivity	194

7.15.1	Introduction	194

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	194

7.16	Proximity-based Services	195

7.16.1	Introduction	195

7.16.2	Requirements	195

7.16.2.1	ProSe UE transmission timing	195

7.16.2.1.1	Serving cell or PCell as timing reference	195

7.16.2.1.2	SCell or non-serving cell as timing reference	195

7.16.3	Interruptions with ProSe	195

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	196

7.16.3.2	Interruptions at ProSe Direct Communication configuration	196

7.16.3.3	Interruptions during ProSe Direct Discovery	196

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	196

7.16.3.5	Interruptions during ProSe Direct Communication	197

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	197

7.16.4.1	Measurement and evaluation of selected cell	197

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	197

7.16.5	Selection / Reselection of ProSe relay UE	198

7.16.6	ProSe operation under deactivated SCell	198

7.17	Maximum Transmission Timing Difference in Dual Connectivity	199

7.17.1	Introduction	199

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	199

7.18.1	Introduction	199

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	199

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	199

7.19	Radio Link Monitoring for UE Category M1	199

7.19.1	Introduction	199

7.19.2	Requirements for FD-FDD and TDD CE mode A	199

7.19.2.1	Minimum requirement when no DRX is used	200

7.19.2.2	Minimum requirement when DRX is used	201

7.19.2.3	Minimum requirement at transitions	202

7.19.3	Requirements for HD-FDD with CE mode A	203

7.19.3.1	Minimum requirement when no DRX is used	203

7.19.3.2	Minimum requirement when DRX is used	203

7.19.3.3	Minimum requirement at transitions	204

7.19.4	Requirements for FD-FDD and TDD with CE mode B	204

7.19.4.1	Minimum requirement when no DRX is used	205

7.19.4.2	Minimum requirement when DRX is used	206

7.19.4.3	Minimum requirement at transitions	207

7.19.5	Requirements for HD-FDD with CE mode B	207

7.19.5.1	Minimum requirement when no DRX is used	208

7.19.5.2	Minimum requirement when DRX is used	208

7.19.5.3	Minimum requirement at transitions	209

7.20	UE transmit timing for NB-IoT	209

7.20.1	Introduction	209

7.20.2	Requirements	209

7.21	 UE timer accuracy for NB-IoT	210

7.21.1	Introduction	210

7.21.2	Requirements	210

7.22	Timing Advance for NB-IoT	210

7.22.1	Introduction	210

7.22.2	Requirements	210

7.22.2.1	Timing Advance adjustment delay	210

7.22.2.2	Timing Advance adjustment accuracy	211

7.23	Radio Link Monitoring for Category NB1 UE	211

7.23.1	Introduction	211

7.23.2	Requirements for HD-FDD Category NB1 UE	211

7.23.2.1	Minimum requirement when no DRX is used	211

7.23.2.2	Minimum requirement when DRX is used	212

7.23.2.3	Minimum requirement at transitions	212

7.24	UE transmit timing for Category M1	213

7.24.1	Introduction	213

7.24.2	Requirements	213

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	214

7.25.1	Definition	214

7.25.2	Minimum requirements	214

7.26	UE transmit timing for Category M2	214

7.26.1	Introduction	214

7.26.2	Requirements	214

7.27	UE timer accuracy for category M1	215

7.27.1	Introduction	215

7.27.2	Requirements	215

7.28	Timing Advance for Category M1	215

7.28.1	Introduction	215

7.28.2	Requirements	215

7.29	Interruptions requirements with FeMBMS	215

7.29.1	Introduction	215

7.29.2	Requirements	215

7.30	Numerology switching delay requirements with FeMBMS	215

7.30.1	Introduction	215

7.30.2	Requirements	215

8	UE Measurements Procedures in RRC_CONNECTED State	216

8.1	General Measurement Requirements	216

8.1.1	Introduction	216

8.1.2	Requirements	216

8.1.2.1	UE measurement capability	216

8.1.2.1.1	Monitoring of multiple layers using gaps	220

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	221

8.1.2.1.2	Network controlled small gap	222

8.1.2.2	E-UTRAN intra frequency measurements	224

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	224

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	228

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	232

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	233

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	234

8.1.2.3	E-UTRAN inter frequency measurements	234

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	235

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	240

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	247

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	247

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	247

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	248

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	250

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	251

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	252

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	258

8.1.2.4	Inter RAT measurements	258

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	258

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	263

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	263

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	267

8.1.2.4.5	E-UTRAN FDD – GSM measurements	267

8.1.2.4.6	E-UTRAN TDD – GSM measurements	272

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	272

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	274

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	274

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	274

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	275

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	275

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	275

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	275

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	277

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	277

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	277

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	277

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	278

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	278

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	280

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	280

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	280

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	282

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	284

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	285

8.1.2.6.5	(Void)	287

8.1.2.6.6	(Void)	287

8.1.2.6.7	(Void)	287

8.1.2.6.8	(Void)	287

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	287

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	288

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	289

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	291

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	293

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	294

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	296

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	298

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	299

8.1.2.7	E-UTRAN E-CID Measurements	301

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	301

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	302

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	304

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	304

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	305

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	305

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	308

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	311

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	314

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	318

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	318

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	318

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	318

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	319

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	319

8.2.1	Introduction	319

8.2.2	Requirements	320

8.3	Measurements for E-UTRA carrier aggregation	322

8.3.1	Introduction	322

8.3.2	Measurements of the primary component carrier	322

8.3.3	Measurements of a secondary component carrier	322

8.3.3.1	Measurements of a secondary component carrier with active SCell	322

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	323

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	323

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	324

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	325

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	325

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	326

8.4.1	Introduction	326

8.4.2	Measurements on the primary component carrier	326

8.4.3	Measurements on a secondary component carrier	327

8.4.4	Measurements on both primary component carrier and a secondary component carrier	327

8.4.5	Measurements on different secondary component carriers	328

8.5	Measurements for UE category 0	329

8.5.1	Introduction	329

8.5.2 	Requirements	330

8.5.2.1 	E-UTRAN intra frequency measurements	330

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	330

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	333

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	335

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	339

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	340

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	340

8.6	Discovery signal measurements	341

8.6.1	Introduction	341

8.6.2	Requirements for CRS based discovery signal measurements	341

8.6.2.1	E-UTRAN intra frequency measurements	341

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	341

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	344

8.6.2.2	E-UTRAN inter frequency measurements	346

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	347

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	349

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	352

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	352

8.6.3	Requirements for CSI-RS based discovery signal measurements	352

8.6.3.1	E-UTRAN intra frequency measurements	352

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	352

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	355

8.6.3.2	E-UTRAN inter frequency measurements	357

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	357

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	360

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	362

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	363

8.7	Discovery signal measurements for E-UTRA carrier aggregation	363

8.7.1	Introduction	363

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	363

8.7.2.1	Measurements of the primary component carrier	363

8.7.2.2	Measurements of a secondary component carrier	363

8.7.2.3	Measurements of a secondary component carrier with active SCell	363

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	363

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	364

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	365

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	366

8.7.3.1	Measurements of the primary component carrier	366

8.7.3.2	Measurements of a secondary component carrier	366

8.7.3.3	Measurements of a secondary component carrier with active SCell	367

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	367

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	367

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	368

8.8	Measurements for E-UTRA dual connectivity	370

8.8.1	Introduction	370

8.8.2	Intra-frequency measurements requirements on PCell	370

8.8.3	Intra-frequency measurements requirements on PSCell	370

8.8.4	Inter-frequency and inter-RAT measurement requirements	370

8.8.5	Intra-frequency measurements with autonomous gaps	370

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	370

8.8.5.2	ECGI reporting delay	372

8.8.6	Inter-frequency measurements with autonomous gaps	372

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	372

8.8.6.2	ECGI reporting delay	373

8.8.7	SSTD Measurements	373

8.8.7.1	Introduction	373

8.8.7.2	SSTD Measurement requirements	373

8.8.7.3	SSTD Measurement Reporting Delay	373

8.8.8	Intra-frequency measurements requirements on SCell	374

8.9	MBSFN Measurements	374

8.9.1	Introduction	374

8.9.2	MBSFN RSRP Measurements	374

8.9.3	MBSFN RSRQ Measurements	374

8.9.4	MCH BLER Measurements	374

8.10	Proximity-based Services	375

8.10.1	Introduction	375

8.10.2	Requirements	375

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	375

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	375

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	376

8.11.1	Introduction	376

8.11.2	CRS based discovery signal measurements	376

8.11.2.1	E-UTRAN intra-frequency measurements	376

8.11.2.1.1	Requirements	376

8.11.2.1.1.1	Requirements when no DRX is used	376

8.11.2.1.1.1.1	Measurement Reporting Requirements	378

8.11.2.1.1.2	Requirements when DRX is used	379

8.11.2.1.1.2.1	Measurement Reporting Requirements	381

8.11.2.2	E-UTRAN inter-frequency measurements	381

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	381

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	385

8.11.3	CSI-RS based discovery signal measurements	385

8.11.3.1	E-UTRAN intra-frequency measurements	385

8.11.3.1.1	Requirements	386

8.11.3.1.1.1	Requirements when no DRX is used	386

8.11.3.1.1.1.1	Measurement Reporting Requirements	387

8.11.3.1.1.2	Requirements when DRX is used	388

8.11.3.1.1.2.1	Measurement Reporting Requirements	389

8.11.3.2	E-UTRAN inter-frequency measurements	390

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	390

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	394

8.11.4	RSSI measurements	394

8.11.4.1	E-UTRAN intra-frequency measurements	394

8.11.4.2	E-UTRAN inter-frequency measurements	394

8.11.5	Channel occupancy measurements	394

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	394

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	394

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	395

8.12.1	Introduction	395

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	395

8.12.2.1	Introduction	395

8.12.2.2	Measurements of a secondary component carrier	395

8.12.2.3	Measurements of a secondary component carrier with active SCell	395

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	395

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	395

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	398

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	400

8.12.3.1	Introduction	400

8.12.3.2	Measurements of a secondary component carrier	400

8.12.3.3	Measurements of a secondary component carrier with active SCell	400

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	400

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	400

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	402

8.13	Measurements for UE Category M1	404

8.13.1	Introduction	404

8.13.2	Requirements for UE category M1 with CE mode A	404

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	405

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	405

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	409

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	411

8.13.2.2	Void	414

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	414

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	414

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	417

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	419

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	420

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	420

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	422

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	424

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	425

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	425

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	425

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	427

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	427

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	428

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	429

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	429

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	429

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	433

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	435

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	439

8.13.3	Requirements for UE category M1 with CE mode B	439

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	440

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	440

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	444

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	447

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	452

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	453

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	453

8.13.3.2	Void	454

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	454

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	454

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	457

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	459

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	460

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	460

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	460

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	460

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	461

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	461

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	462

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	462

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	462

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	466

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	468

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	473

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	474

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	474

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	476

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	478

8.14	Measurements for UE category NB1	479

8.14.1	Introduction	479

8.14.2	NB-IoT intra frequency measurements under normal coverage	479

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	479

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	479

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	479

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	479

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	479

8.15	Void	480

8.16	Measurements for UE Category M2	480

8.16.1	Introduction	480

8.16.2	Requirements for UE category M2 with CE mode A	480

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	480

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	481

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	481

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	481

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	482

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	482

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	482

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	482

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	485

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	487

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	488

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	488

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	490

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	492

8.16.3	Requirements for UE category M2 with CE mode B	493

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	493

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	493

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	495

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	498

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	498

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	498

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	500

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	503

8.17	Void	503

8.18	Measurements for non-BL/CE UE	503

8.18.1	Introduction	503

8.18.2	Requirements for non-BL/CE UE with CE Mode B	503

8.18.2.1	E-UTRAN intra frequency measurements	503

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	503

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	504

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	504

9	Measurements performance requirements for UE	505

9.1	E-UTRAN measurements	506

9.1.1	Introduction	506

9.1.2	Intra-frequency RSRP Accuracy Requirements	506

9.1.2.1	Absolute RSRP Accuracy	506

9.1.2.2	Relative Accuracy of RSRP	507

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	507

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	508

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	509

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	510

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	511

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	512

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	513

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	513

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	513

9.1.3	Inter-frequency RSRP Accuracy Requirements	514

9.1.3.1	Absolute RSRP Accuracy	514

9.1.3.2	Relative Accuracy of RSRP	515

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	515

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	516

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	517

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	517

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	517

9.1.4	RSRP Measurement Report Mapping	518

9.1.5	Intra-frequency RSRQ Accuracy Requirements	518

9.1.5.1	Absolute RSRQ Accuracy	518

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	519

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	520

9.1.5.4	Absolute WB-RSRQ Accuracy	521

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	522

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	523

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	523

9.1.6	Inter-frequency RSRQ Accuracy Requirements	523

9.1.6.1	Absolute RSRQ Accuracy	523

9.1.6.2	Relative Accuracy of RSRQ	524

9.1.6.3	Absolute WB-RSRQ Accuracy	525

9.1.6.4	Relative WB-RSRQ Accuracy	525

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	526

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	527

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	527

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	527

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	528

9.1.7	RSRQ Measurement Report Mapping	529

9.1.8	Power Headroom	529

9.1.8.1	Period	529

9.1.8.2	Reporting Delay	529

9.1.8.3	Void	530

9.1.8.4	Report Mapping	530

9.1.9	UE Rx – Tx time difference	530

9.1.9.1	Measurement Requirement	530

9.1.9.2	Measurement Report mapping	531

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	531

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	532

9.1.10	Reference Signal Time Difference (RSTD)	533

9.1.10.1	Intra-Frequency Accuracy Requirement	533

9.1.10.2	Inter-Frequency Accuracy Requirement	535

9.1.10.3	RSTD Measurement Report Mapping	536

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	537

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	537

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	538

9.1.11	Carrier aggregation measurement accuracy	539

9.1.11.1	Primary component carrier accuracy requirement	540

9.1.11.2	Secondary component carrier accuracy requirement	540

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	540

9.1.11.4	Secondary component carrier relative accuracy requirement	540

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	540

9.1.13	Measurement accuracy for UE category 0	541

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	541

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	541

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	542

9.1.14	Accuracy requirements for Discovery Signal Measurements	543

9.1.14.1	Introduction	543

9.1.14.2	RSRP measurements in discovery signal occasions	543

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	543

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	543

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	543

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	544

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	545

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	545

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	546

9.1.14.3.3	CSI-RSRP measurement report mapping	547

9.1.14.4	RSRQ measurements in discovery signal occasions	547

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	547

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	547

9.1.15.1.1	Primary component carrier accuracy requirement	547

9.1.15.1.2	Secondary component carrier accuracy requirement	547

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	548

9.1.15.1.4	Secondary component carrier relative accuracy requirement	548

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	548

9.1.15.2.1	Primary component carrier accuracy requirement	548

9.1.15.2.2	Secondary component carrier accuracy requirement	548

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	548

9.1.15.2.4	Secondary component carrier relative accuracy requirement	548

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	548

9.1.17	RS-SINR Measurements	549

9.1.17.1	Measurement Report Mapping	549

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	549

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	549

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	550

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	550

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	551

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	551

9.1.18.1	Introduction	551

9.1.18.2	RSRP measurements	551

9.1.18.2.1	RSRP measurement report mapping	551

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	551

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	552

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	552

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	553

9.1.18.3	RSRQ measurements	553

9.1.18.3.1	RSRQ measurement report mapping	553

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	554

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	554

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	555

9.1.18.4	CSI-RSRP measurements	555

9.1.18.4.1	CSI-RSRP measurement report mapping	555

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	555

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	556

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	556

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	557

9.1.18.5	RSSI measurements	557

9.1.18.5.1	RSSI measurement report mapping	557

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	558

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	558

9.1.18.6	Channel occupancy measurements	558

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	558

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	559

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	559

9.1.19.1	Introduction	559

9.1.19.2	Accuracy requirements for measurements on SCC	559

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	559

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	560

9.1.20	SFN and Subframe Time Difference (SSTD)	560

9.1.20.1	SSTD Accuracy Requirement	560

9.1.20.2	SSTD Measurement Report Mapping	561

9.1.21	Measurement accuracy for UE category M1	562

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	562

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	563

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	564

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	565

9.1.21.5	RSRP Measurement Report Mapping	566

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	566

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	567

9.1.21.8	RSRQ Measurement Report Mapping	568

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	568

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	569

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	570

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	571

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	572

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	573

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	574

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	575

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	576

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	577

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	578

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	579

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	581

9.1.22	Measurement accuracy for UE Category NB1	583

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	583

9.1.22.2	Void	583

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode	583

9.1.22.4	Void	584

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	584

9.1.22.6	Void	584

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	584

9.1.22.8	Void	585

9.1.22.9	NRSRP Measurement Report Mapping	585

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	585

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	586

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	587

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	588

9.1.22.14	NRSRQ Measurement Report Mapping	589

9.1.23	Power Headroom for UE Category NB1	589

9.1.23.1	Period	590

9.1.23.2	Reporting Delay	590

9.1.23.3	Report Mapping for UE Category NB1	590

9.1.23.3.1	Void	590

9.1.23.3.2	Void	590

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	591

9.1.24	Void	591

9.1.25	Measurement accuracy for UE category M2	591

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	591

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	592

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	593

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	594

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	596

9.1.26	Measurement Accuracy for non-BL CE UE	598

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	598

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	599

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	599

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	600

9.1.26.5	RSRP Measurement Report Mapping	601

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	601

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	601

9.1.26.8	RSRQ Measurement Report Mapping	602

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	602

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	603

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	604

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	605

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	606

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	606

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	606

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	607

9.2	UTRAN FDD Measurements	608

9.2.1	UTRAN FDD CPICH RSCP	609

9.2.2	Void	609

9.2.3	UTRAN FDD CPICH Ec/No	609

9.3	UTRAN TDD Measurements	610

9.3.1	UTRAN TDD P-CCPCH RSCP	610

9.3.2	Void	610

9.3.3	Void	610

9.4	GSM Measurements	610

9.4.1	GSM carrier RSSI	610

9.5	CDMA2000 1x RTT Measurements	611

9.5.1	CDMA2000 1x RTT Pilot Strength	611

9.6	PCMAX,c	611

9.6.1	Report Mapping	611

9.6.2 	Estimation Period	611

9.6.3	Reporting Delay	611

9.7	IEEE802.11 Measurements	611

9.7.1	WLAN RSSI	612

9.7.2	WLAN RSSI Measurement Report Mapping	612

9.8	MBSFN Measurements	612

9.8.1	Introduction	612

9.8.2	MBSFN RSRP	612

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	612

9.8.2.2	MBSFN RSRP measurement report mapping	613

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	613

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	614

9.8.3	MBSFN RSRQ	614

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	614

9.8.3.2	MBSFN RSRQ measurement report mapping	614

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	615

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	615

9.8.4	MCH BLER	615

9.8.4.1	Measurement report mapping for MCH BLER	615

9.8.4.2	Measurement report mapping for MCH Block Number	616

9.9	ProSe Measurements	617

9.9.1	Introduction	617

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	617

9.9.2.1	Absolute S-RSRP Accuracy	617

9.9.2.2	Relative Accuracy of S-RSRP	618

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	619

9.9.3.1	Absolute SD-RSRP Accuracy	619

9.9.3.2	Relative Accuracy of SD-RSRP	619

9.10	V2X Measurements	620

9.10.1	Introduction	620

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	620

9.10.2.1	Absolute S-RSRP Accuracy	620

9.10.2.2	Relative Accuracy of S-RSRP	621

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	621

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	621

9.10.4	S-RSSI Measurement Accuracy Requirements	622

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	622

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	622

10	Measurements Performance Requirements for E-UTRAN	623

10.1	Received Interference Power	623

10.1.1	Absolute accuracy requirement	623

10.1.2	Relative accuracy requirement	623

10.1.3	Received Interference Power measurement report mapping	623

10.2	Angle of Arrival (AOA)	624

10.2.1	Range/mapping	624

10.3	Timing Advance (TADV)	624

10.3.1	Report mapping	624

11	ProSe Requirements in Any Cell Selection state	625

11.1	Introduction	625

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	625

11.2.1	Introduction	625

11.2.2	ProSe UE transmission timing	625

11.3	Initiation/Cease of SLSS Transmissions	625

11.3.1	Introduction	625

11.3.2	Requirements	625

11.4	Measurements for ProSe in Any Cell Selection State	626

11.4.1	Introduction	626

11.4.2	Requirements	626

11.4.2.1	E-UTRA FDD	626

11.4.2.2	E-UTRA TDD	626

11.5	Selection / Reselection of ProSe Synchronization Reference	627

11.5.1	Introduction	627

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	627

11.5.2.1	Introduction	627

11.5.2.2	Requirements	627

11.6	Void	627

11.7	Selection / Reselection of ProSe relay UE	627

11.7.1	Introduction	627

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	627

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	628

12.1	Introduction	628

12.2	Transmit Timing	628

12.2.1	GNSS as timing reference	628

12.3	Interruption	628

12.4	Reliability of GNSS signal	629

13	V2X Requirements	629

13.1	Introduction	629

13.2	UE Transmit Timing	629

13.2.1	Introduction	629

13.2.2	GNSS as synchronization reference source	629

13.2.3	Serving cell/PCell as synchronization reference source	630

13.2.4	SyncRef UE as synchronization reference source	630

13.3	Initiation/Cease of SLSS Transmissions	630

13.3.1	Introduction	630

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	630

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	631

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	631

13.4	Selection / Reselection of V2X Synchronization Reference Source	631

13.5	Autonomous Resource Selection/Reselection measurements	632

13.5.1	Introduction	632

13.5.2	PSSCH-RSRP measurements	632

13.5.3	S-RSSI measurements	632

13.6	Congestion Control measurements	632

13.7	Interruption	633

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	633

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	633

13.8	Reliability of GNSS signal	633

Annex A (normative):	Test Cases	635

A.1	Purpose of annex	635

A.2	Requirement classification for statistical testing	635

A.2.1	Types of requirements in TS 36.133	635

A.2.1.1	Time and delay requirements on UE higher layer actions	635

A.2.1.2	Measurements of power levels, relative powers and time	635

A.2.1.3	Implementation requirements	636

A.2.1.4	Physical layer timing requirements	636

A.3	RRM test configurations	637

A.3.1	Reference Measurement Channels	637

A.3.1.1	PDSCH	637

A.3.1.1.1	FDD	637

A.3.1.1.2	TDD	638

A.3.1.1.3	FDD for UE category 0	640

A.3.1.1.4	HD-FDD for UE category 0	641

A.3.1.1.5	TDD for UE category 0	642

A.3.1.1.6	Frame Structure 3	643

A.3.1.2	PCFICH/PDCCH/PHICH	644

A.3.1.2.1	FDD	644

A.3.1.2.2	TDD	644

A.3.1.2.3	HD-FDD for UE category 0	645

A.3.1.2.4	FS 3	645

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	645

A.3.1.3.1	FDD in CEModeA	646

A.3.1.3.2	HD-FDD in CEModeA	646

A.3.1.3.3	TDD in CEModeA	647

A.3.1.3.4	FDD in CEModeB	647

A.3.1.3.5	HD-FDD in CEModeB	648

A.3.1.3.6	TDD in CEModeB	648

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	649

A.3.1.4.1	FDD in CEModeA	649

A.3.1.4.2	HD-FDD in CEModeA	650

A.3.1.4.3	TDD in CEModeA	651

A.3.1.4.4	FDD in CEModeB	652

A.3.1.4.5	 HD-FDD in CEModeB	653

A.3.1.4.6	TDD in CEModeB	654

A.3.1.5	NPDSCH Reference Channel for UE category NB1	655

A.3.1.5.1	HD-FDD in-band operation	655

A.3.1.5.2	Void	656

A.3.1.5.3	HD-FDD standalone operation	656

A.3.1.5.4	Void	657

A.3.1.5.5	HD-FDD guard band operation	657

A.3.1.5.6	Void	657

A.3.1.6	NPDCCH Reference Channel for UE category NB1	657

A.3.1.6.1	HD-FDD in-band operation	657

A.3.1.6.2	Void	658

A.3.1.6.3	HD-FDD standalone operation	658

A.3.1.6.4	Void	658

A.3.1.6.5	HD-FDD guard band operation	658

A.3.1.6.6	Void	659

A.3.2	OFDMA Channel Noise Generator (OCNG)	659

A.3.2.1	OCNG Patterns for FDD	659

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	660

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	660

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	661

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	662

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	663

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	663

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	664

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	664

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	665

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	665

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	666

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	667

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	667

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	668

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	668

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	669

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	669

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	670

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	670

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	671

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	671

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	672

A.3.2.2	OCNG Patterns for TDD	672

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	673

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	673

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	674

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	674

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	675

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	676

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	677

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	678

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	678

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	679

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	680

A.3.2.3	OCNG Patterns for Narrowband IoT	680

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	682

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	683

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	683

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	684

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	685

A.3.2.4	OCNG Patterns for V2X sidelink	686

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	687

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	687

A.3.3	Reference DRX Configurations	687

A.3.4	ABS Transmission Configurations	688

A.3.4.1	Non-MBSFN ABS Transmission Configurations	688

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	688

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	688

A.3.4.2	MBSFN ABS Transmission Configurations	689

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	689

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	690

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	690

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	690

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	691

A.3.6.1	Introduction	691

A.3.7	Test Cases with Different Channel Bandwidths	691

A.3.7.1	Introduction	691

A.3.7.2	Principle of testing	691

A.3.8	Antenna Configuration	691

A.3.8.1	Antenna connection for 4 Rx capable UEs	691

A.3.8.1.1 Introduction	691

A.3.8.1.2 Principle of testing	691

A.3.8.1.2.1 Single carrier tests	691

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	692

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	693

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	693

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	693

A.3.9.1	Introduction	693

A.3.9.2	Principle of testing	693

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	694

A.3.10.1	Introduction	694

A.3.10.2	Principle of testing	694

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	694

A.3.11.1	Introduction	694

A.3.11.2	Principle of Testing	694

A.3.12	Proximity-based Services	694

A.3.12.1	Introduction	694

A.3.12.2	Reference DRX configurations for ProSe tests	694

A.3.12.3	Test Cases with Different Channel Bandwidths	695

A.3.12.3.1	Introduction	695

A.3.12.3.2	Principle of testing	695

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	695

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	701

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	703

A.3.12.6.1	FDD	703

A.3.12.7	Reference measurement channels for ProSe Direct Communication	703

A.3.12.7.1	FDD	703

A.3.12.8	ProSe Receive Traffic Generator	704

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	704

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	704

A.3.13	Time Offset between Cells	705

A.3.13.1	Introduction	705

A.3.13.2	Definition	705

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	705

A.3.14.1	Introduction	705

A.3.14.2	Principle of testing	705

A.3.15	Dual connectivity test cases with different combination of duplex mode	705

A.3.15.1	Introduction	705

A.3.15.2	Principle of testing	705

A.3.16	Reference PRACH Configurations	706

A.3.17	Listen before talk model	706

A.3.17.1	Introduction	706

A.3.17.2	Definition	706

A.3.18	Reference NPRACH Configurations	707

A.3.19	Dual connectivity test cases with different bandwidth combinations	708

A.3.19.1	Introduction	708

A.3.19.2	Principle of testing	708

A.3.20	Category M1 UE Test Cases	708

A.3.20.1	Introduction	708

A.3.20.2	Principle of Cat-M1 UE Testing	708

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	709

A.3.21.1	Introduction	709

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	710

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	711

A.3.22	Category 1bis UE Test Cases	712

A.3.22.1	Introduction	712

A.3.22.2	Principle of Category 1bis UE Testing	712

A.3.23	Category NB2 UE Test Cases	715

A.3.23.1	Introduction	715

A.3.23.2	Principle of Category NB2 UE Testing	715

A.3.24	V2X sidelink communication	715

A.3.24.1	Introduction	715

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	716

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	719

A.3.25	Category M2 UE Test Cases	720

A.3.25.1	Introduction	720

A.3.25.2	Principle of Cat-M2 UE Testing	720

A.4	E-UTRAN RRC_IDLE state	722

A.4.2	Cell Re-Selection	722

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	722

A.4.2.1.1	Test Purpose and Environment	722

A.4.2.1.2	Test Requirements	723

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	724

A.4.2.2.1	Test Purpose and Environment	724

A.4.2.2.2	Test Requirements	725

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	726

A.4.2.3.1	Test Purpose and Environment	726

A.4.2.3.2	Test Requirements	727

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	728

A.4.2.4.1	Test Purpose and Environment	728

A.4.2.4.2	Test Requirements	729

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	730

A.4.2.5.1	Test Purpose and Environment	730

A.4.2.5.2	Test Requirements	731

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	732

A.4.2.6.1	Test Purpose and Environment	732

A.4.2.6.2	Test Requirements	733

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	734

A.4.2.7.1	Test Purpose and Environment	734

A.4.2.7.2	Test Requirements	735

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	736

A.4.2.8.1	Test Purpose and Environment	736

A.4.2.8.2	Test Requirements	737

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	738

A.4.2.9.1	Test Purpose and Environment	738

A.4.2.9.2	Test Requirements	738

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	738

A.4.2.10.1	Test Purpose and Environment	738

A.4.2.10.2	Test Requirements	741

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	741

A.4.2.11.1	Test Purpose and Environment	741

A.4.2.11.2	Test Requirements	744

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	744

A.4.2.12.1	Test Purpose and Environment	744

A.4.2.12.2	Test Requirements	746

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	747

A.4.2.13.1	Test Purpose and Environment	747

A.4.2.13.2	Test Requirements	748

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	749

A.4.2.14.1	Test Purpose and Environment	749

A.4.2.14.2	Test Requirements	750

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	751

A.4.2.15.1	Test Purpose and Environment	751

A.4.2.15.2	Test Requirements	752

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	753

A.4.2.16.1	Test Purpose and Environment	753

A.4.2.16.2	Test Requirements	754

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	755

A.4.2.17.1	Test Purpose and Environment	755

A.4.2.17.2	Test Requirements	756

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	757

A.4.2.18.1	Test Purpose and Environment	757

A.4.2.18.2	Test Requirements	759

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	760

A.4.2.19.1	Test Purpose and Environment	760

A.4.2.19.2	Test Requirements	762

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	763

A.4.2.20.1	Test Purpose and Environment	763

A.4.2.20.2	Test Requirements	764

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	765

A.4.2.21.1	Test Purpose and Environment	765

A.4.2.21.2	Test Requirements	766

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	767

A.4.2.22.1	Test Purpose and Environment	767

A.4.2.22.2	Test Requirements	768

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	769

A.4.2.23.1	Test Purpose and Environment	769

A.4.2.23.2	Test Requirements	770

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	771

A.4.2.24.1	Test Purpose and Environment	771

A.4.2.24.2	Test Requirements	773

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	774

A.4.2.25.1	Test Purpose and Environment	774

A.4.2.25.2	Test Requirements	775

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	776

A.4.2.26.1	Test Purpose and Environment	776

A.4.2.26.2	Test Requirements	777

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	778

A.4.2.27.1	Test Purpose and Environment	778

A.4.2.27.2	Test Requirements	779

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	780

A.4.2.28.1	Test Purpose and Environment	780

A.4.2.28.2	Test Requirements	781

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	782

A.4.2.29.1	Test Purpose and Environment	782

A.4.2.29.2	Test Requirements	783

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	784

A.4.2.30.1	Test Purpose and Environment	784

A.4.2.30.2	Test Requirements	785

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	786

A.4.2.31.1	Test Purpose and Environment	786

A.4.2.31.2	Test Requirements	787

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	788

A.4.2.32.1	Test Purpose and Environment	788

A.4.2.32.2	Test Requirements	789

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	790

A.4.2.33.1	Test Purpose and Environment	790

A.4.2.33.2	Test Requirements	791

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	792

A.4.2.34.1	Test Purpose and Environment	792

A.4.2.34.2	Test Requirements	793

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	794

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	794

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	794

A.4.3.1.1.1	Test Purpose and Environment	794

A.4.3.1.1.2	Test Requirements	796

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	796

A.4.3.1.2.1	Test Purpose and Environment	796

A.4.3.1.2.2	Test Requirements	799

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	799

A.4.3.1.3.1	Test Purpose and Environment	799

A.4.3.1.3.2	Test Requirements	802

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	802

A.4.3.1.4.1	Test Purpose and Environment	802

A.4.3.1.4.2	Test Requirements	803

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	803

A.4.3.1.5.1	Test Purpose and Environment	803

A.4.3.1.5.2	Test Requirements	806

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	807

A.4.3.2.1	Test Purpose and Environment	807

A.4.3.2.1.1	Void	807

A.4.3.2.1.2	1.28Mcps TDD option	807

A.4.3.2.1.3	Void	809

A.4.3.2.2	Test Requirements	809

A.4.3.2.2.1	1.28Mcps TDD option	809

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	809

A.4.3.2A.1	Test Purpose and Environment	809

A.4.3.2A.2	Test Requirements	813

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	813

A.4.3.3.1	Test Purpose and Environment	813

A.4.3.3.2	Test Requirements	816

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	816

A.4.3.3A.1	Test Purpose and Environment	816

A.4.3.3A.2	Test Requirements	820

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	821

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	821

A.4.3.4.1.1	Test Purpose and Environment	821

A.4.3.4.1.2	Test Requirements	823

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	823

A.4.3.4.2.1	Test Purpose and Environment	823

A.4.3.4.2.2	Test Requirements	825

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	825

A.4.3.4.3.1	Test Purpose and Environment	825

A.4.3.4.3.2	Test Requirements	828

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	828

A.4.3.4.4.1	Test Purpose and Environment	828

A.4.3.4.4.2	Test Requirements	832

A.4.4	E-UTRAN to GSM Cell Re-Selection	832

A.4.4.1	E-UTRAN FDD – GSM:	832

A.4.4.1.1	Test Purpose and Environment	832

A.4.4.1.2	Test Requirements	834

A.4.4.2	E-UTRAN TDD – GSM:	834

A.4.4.2.1	Test Purpose and Environment	834

A.4.4.2.2	Test Requirements	836

A.4.5	E-UTRAN to HRPD Cell Re-Selection	837

A.4.5.1	E-UTRAN FDD – HRPD	837

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	837

A.4.5.1.1.1	Test Purpose and Environment	837

A.4.5.1.1.2	Test Requirements	839

A.4.5.2	E-UTRAN TDD – HRPD	839

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	839

A.4.5.2.1.1	Test Purpose and Environment	839

A.4.5.2.1.2	Test Requirements	842

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	842

A.4.6.1	E-UTRAN FDD – cdma2000 1X	842

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	842

A.4.6.1.1.1	Test Purpose and Environment	842

A.4.6.1.1.2	Test Requirements	845

A.4.6.2	E-UTRAN TDD – cdma2000 1X	845

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	845

A.4.6.2.1.1	Test Purpose and Environment	845

A.4.6.2.1.2	Test Requirements	848

A.4.7	Idle State Positioning Measurement for UE category NB1	848

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	848

A.4.7.1.1	Test Purpose and Environment	848

A.4.7.1.2	Test Requirements	852

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	853

A.4.7.2.1	Test Purpose and Environment	853

A.4.7.4.2	Test Requirements	856

A.5	E-UTRAN RRC CONNECTED Mode Mobility	857

A.5.1	E-UTRAN Handover	857

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	857

A.5.1.1.1	Test Purpose and Environment	857

A.5.1.1.2	Test Requirements	859

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	859

A.5.1.2.1	Test Purpose and Environment	859

A.5.1.2.2	Test Requirements	861

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	861

A.5.1.3.1	Test Purpose and Environment	861

A.5.1.3.2	Test Requirements	863

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	863

A.5.1.4.1	Test Purpose and Environment	863

A.5.1.4.2	Test Requirements	865

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	865

A.5.1.5.1	Test Purpose and Environment	865

A.5.1.5.2	Test Requirements	867

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	867

A.5.1.6.1	Test Purpose and Environment	867

A.5.1.6.2	Test Requirements	869

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	869

A.5.1.7.1	Test Purpose and Environment	869

A.5.1.7.2	Test Requirements	872

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	872

A.5.1.8.1	Test Purpose and Environment	872

A.5.1.8.2	Test Requirements	875

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	875

A.5.1.9.1	Test Purpose and Environment	875

A.5.1.9.2	Test Requirements	876

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	876

A.5.1.10.1	Test Purpose and Environment	876

A.5.1.10.2	Test Requirements	878

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	878

A.5.1.11.1	Test Purpose and Environment	878

A.5.1.11.2	Test Requirements	880

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	880

A.5.1.12.1	Test Purpose and Environment	880

A.5.1.12.2	Test Requirements	882

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	882

A.5.1.13.1	Test Purpose and Environment	882

A.5.1.13.2	Test Requirements	884

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	885

A.5.1.14.1	Test Purpose and Environment	885

A.5.1.14.2	Test Requirements	886

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	887

A.5.1.15.1	Test Purpose and Environment	887

A.5.1.15.2	Test Requirements	888

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	889

A.5.1.16.1	Test Purpose and Environment	889

A.5.1.16.2	Test Requirements	890

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	891

A.5.1.17.1	Test Purpose and Environment	891

A.5.1.17.2	Test Requirements	892

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	893

A.5.1.18.1	Test Purpose and Environment	893

A.5.1.18.2	Test Requirements	894

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	895

A.5.1.19.1	Test Purpose and Environment	895

A.5.1.19.2	Test Requirements	896

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	897

A.5.1.20.1	Test Purpose and Environment	897

A.5.1.20.2	Test Requirements	898

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	899

A.5.1.21.1	Test Purpose and Environment	899

A.5.1.21.2	Test Requirements	900

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	900

A.5.1.22.1	Test Purpose and Environment	900

A.5.1.22.2	Test Requirements	902

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	902

A.5.1.23.1	Test Purpose and Environment	902

A.5.1.23.2	Test Requirements	904

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	904

A.5.1.24.1	Test Purpose and Environment	904

A.5.1.24.2	Test Requirements	906

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	906

A.5.1.25.1	Test Purpose and Environment	906

A.5.1.25.2	Test Requirements	908

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	908

A.5.1.26.1	Test Purpose and Environment	908

A.5.1.26.2	Test Requirements	910

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	911

A.5.1.27.1	Test Purpose and Environment	911

A.5.1.27.2	Test Requirements	912

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	913

A.5.1.28.1	Test Purpose and Environment	913

A.5.1.28.2	Test Requirements	914

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	915

A.5.1.29.1	Test Purpose and Environment	915

A.5.1.29.2	Test Requirements	916

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	917

A.5.1.30.1	Test Purpose and Environment	917

A.5.1.30.2	Test Requirements	918

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	919

A.5.1.31.1	Test Purpose and Environment	919

A.5.1.31.2	Test Requirements	920

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	921

A.5.1.32.1	Test Purpose and Environment	921

A.5.1.32.2	Test Requirements	922

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	923

A.5.1.33.1	Test Purpose and Environment	923

A.5.1.13.2	Test Requirements	924

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	925

A.5.1.34.1	Test Purpose and Environment	925

A.5.1.34.2	Test Requirements	926

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	927

A.5.1.35.1	Test Purpose and Environment	927

A.5.1.35.2	Test Requirements	928

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	929

A.5.1.36.1	Test Purpose and Environment	929

A.5.1.36.2	Test Requirements	930

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	931

A.5.1.37.1	Test Purpose and Environment	931

A.5.1.37.2	Test Requirements	932

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	933

A.5.1.38.1	Test Purpose and Environment	933

A.5.1.38.2	Test Requirements	934

A.5.2	E-UTRAN Handover to other RATs	935

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	935

A.5.2.1.1	Test Purpose and Environment	935

A.5.2.1.2	Test Requirements	936

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	937

A.5.2.2.1	Test Purpose and Environment	937

A.5.2.2.2	Test Requirements	940

A.5.2.3 E-UTRAN FDD- GSM Handover	940

A.5.2.3.1	Test Purpose and Environment	940

A.5.2.3.2	Test Requirements	941

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	942

A.5.2.4.1	Test Purpose and Environment	942

A.5.2.4.1.1	Void	942

A.5.2.4.1.2	1.28 Mcps TDD option	942

A.5.2.4.1.3	Void	944

A.5.2.4.2	Test Requirements	944

A.5.2.4.2.1	Void	944

A.5.2.4.2.2	1.28 Mcps TDD option	944

A.5.2.4.2.3	Void	944

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	944

A.5.2.5.1	Test Purpose and Environment	944

A.5.2.5.1.3	Void	947

A.5.2.5.2	Test Requirements	947

A.5.2.5.2.1	Void	947

A.5.2.5.2.2	1.28 Mcps TDD option	947

A.5.2.5.2.3	Void	947

A.5.2.6	E-UTRAN TDD - GSM Handover	947

A.5.2.6.1	Test Purpose and Environment	947

A.5.2.6.2	Test Requirements	949

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	950

A.5.2.7.1	Test Purpose and Environment	950

A.5.2.7.2	Test Requirements	952

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	952

A.5.2.8.1	Test Purpose and Environment	952

A.5.2.8.2	Test Requirements	953

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	954

A.5.2.9.1	Test Purpose and Environment	954

A.5.2.9.2	Test Requirements	955

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	956

A.5.2.10.1	Test Purpose and Environment	956

A.5.2.10.2	Test Requirements	958

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	958

A.5.2.10A.1	Test Purpose and Environment	958

A.5.2.10A.2	Test Requirements	961

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	961

A.5.2.10B.1	Test Purpose and Environment	961

A.5.2.10B.2	Test Requirements	964

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	964

A.5.2.11.1	Test Purpose and Environment	964

A.5.2.11.2	Test Requirements	964

A.5.3	E-UTRAN Handover to Non-3GPP RATs	965

A.5.3.1	E-UTRAN FDD – HRPD Handover	965

A.5.3.1.1	Test Purpose and Environment	965

A.5.3.1.2	Test Requirements	967

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	967

A.5.3.2.1	Test Purpose and Environment	967

A.5.3.2.2	Test Requirements	970

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	970

A.5.3.3.1	Test Purpose and Environment	970

A.5.3.3.2	Test Requirements	972

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	973

A.5.3.4.1	Test Purpose and Environment	973

A.5.3.4.2	Test Requirements	974

A.5.3.5	E-UTRAN TDD – HRPD Handover	975

A.5.3.5.1	Test Purpose and Environment	975

A.5.3.5.2	Test Requirements	977

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	978

A.5.3.6.1	Test Purpose and Environment	978

A.5.3.6.2	Test Requirements	981

A.6	RRC Connection Control	981

A.6.1	RRC Re-establishment	981

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	981

A.6.1.1.1	Test Purpose and Environment	981

A.6.1.1.2	Test Requirements	982

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	983

A.6.1.2.1	Test Purpose and Environment	983

A.6.1.2.2	Test Requirements	984

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	985

A.6.1.3.1	Test Purpose and Environment	985

A.6.1.3.2	Test Requirements	986

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	987

A.6.1.4.1	Test Purpose and Environment	987

A.6.1.4.2	Test Requirements	988

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	989

A.6.1.5.1	Test Purpose and Environment	989

A.6.1.5.2	Test Requirements	989

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	989

A.6.1.6.1	Test Purpose and Environment	989

A.6.1.6.2	Test Requirements	991

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	992

A.6.1.7.1	Test Purpose and Environment	992

A.6.1.7.2	Test Requirements	993

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	994

A.6.1.8.1	Test Purpose and Environment	994

A.6.1.8.2	Test Requirements	995

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	996

A.6.1.9.1	Test Purpose and Environment	996

A.6.1.9.2	Test Requirements	997

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	998

A.6.1.10.1	Test Purpose and Environment	998

A.6.1.10.2	Test Requirements	999

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1000

A.6.1.11.1	Test Purpose and Environment	1000

A.6.1.11.2	Test Requirements	1001

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1002

A.6.1.12.1	Test Purpose and Environment	1002

A.6.1.12.2	Test Requirements	1003

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1004

A.6.1.13.1	Test Purpose and Environment	1004

A.6.1.13.2	Test Requirements	1005

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1006

A.6.1.14.1	Test Purpose and Environment	1006

A.6.1.14.2	Test Requirements	1007

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1008

A.6.1.15.1	Test Purpose and Environment	1008

A.6.1.15.2	Test Requirements	1010

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1011

A.6.1.16.1	Test Purpose and Environment	1011

A.6.1.16.2	Test Requirements	1013

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1014

A.6.1.17.1	Test Purpose and Environment	1014

A.6.1.17.2	Test Requirements	1015

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1016

A.6.1.18.1	Test Purpose and Environment	1016

A.6.1.18.2	Test Requirements	1017

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1018

A.6.1.19.1	Test Purpose and Environment	1018

A.6.1.19.2	Test Requirements	1019

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1020

A.6.1.20.1	Test Purpose and Environment	1020

A.6.1.20.2	Test Requirements	1021

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1022

A.6.1.21.1	Test Purpose and Environment	1022

A.6.1.21.2	Test Requirements	1023

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1024

A.6.1.22.1	Test Purpose and Environment	1024

A.6.1.22.2	Test Requirements	1025

A.6.2	Random Access	1026

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1026

A.6.2.1.1	Test Purpose and Environment	1026

A.6.2.1.2	Test Requirements	1028

A.6.2.1.2.1	Random Access Response Reception	1028

A.6.2.1.2.2	No Random Access Response Reception	1028

A.6.2.1.2.3	Receiving a NACK on msg3	1028

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1028

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1029

A.6.2.1.2.6	Contention Resolution Timer expiry	1029

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1029

A.6.2.2.1	Test Purpose and Environment	1029

A.6.2.2.2	Test Requirements	1030

A.6.2.2.2.1	Random Access Response Reception	1031

A.6.2.2.2.2	No Random Access Response Reception	1031

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1031

A.6.2.3.1	Test Purpose and Environment	1031

A.6.2.3.2	Test Requirements	1033

A.6.2.3.2.1	Random Access Response Reception	1033

A.6.2.3.2.2	No Random Access Response reception	1033

A.6.2.3.2.3	Receiving a NACK on msg3	1033

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1034

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1034

A.6.2.3.2.6	Contention Resolution Timer expiry	1034

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1034

A.6.2.4.1	Test Purpose and Environment	1034

A.6.2.4.2	Test Requirements	1036

A.6.2.4.2.1	Random Access Response Reception	1036

A.6.2.4.2.2	No Random Access Response Reception	1036

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1036

A.6.2.5.1	Test Purpose and Environment	1036

A.6.2.5.2	Test Requirements	1037

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1037

A.6.2.6.1	Test Purpose and Environment	1037

A.6.2.6.2	Test Requirements	1037

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1038

A.6.2.7.1	Test Purpose and Environment	1038

A.6.2.7.2	Test Requirements	1040

A.6.2.7.2.1	Random Access Response Reception	1040

A.6.2.7.2.2	No Random Access Response Reception	1040

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1040

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1041

A.6.2.8.1	Test Purpose and Environment	1041

A.6.2.8.2	Test Requirements	1043

A.6.2.8.2.1	Random Access Response Reception	1043

A.6.2.8.2.2	No Random Access Response Reception	1043

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1043

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1044

A.6.2.9.1	Test Purpose and Environment	1044

A.6.2.9.2	Test Requirements	1046

A.6.2.9.2.1	Random Access Response Reception	1046

A.6.2.9.2.2	No Random Access Response Reception	1047

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1047

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1048

A.6.2.10.1	Test Purpose and Environment	1048

A.6.2.10.2	Test Requirements	1050

A.6.2.10.2.1	Random Access Response Reception	1050

A.6.2.10.2.2	No Random Access Response Reception	1050

A.6.2.10.2.3	Receiving a NACK on msg3	1051

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1051

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1051

A.6.2.10.2.6	Contention Resolution Timer expiry	1051

A.6.2.10.2.7	PRACH Resource Selection	1051

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1051

A.6.2.11.1	Test Purpose and Environment	1051

A.6.2.11.2	Test Requirements	1053

A.6.2.11.2.1	Random Access Response Reception	1053

A.6.2.11.2.2	No Random Access Response Reception	1054

A.6.2.11.2.3	Receiving a NACK on msg3	1054

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1054

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1054

A.6.2.11.2.6	Contention Resolution Timer expiry	1054

A.6.2.11.2.7	PRACH Resource Selection	1055

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1055

A.6.2.12.1	Test Purpose and Environment	1055

A.6.2.12.2	Test Requirements	1057

A.6.2.12.2.1	Random Access Response Reception	1057

A.6.2.12.2.2	No Random Access Response Reception	1058

A.6.2.12.2.3	Receiving a NACK on msg3	1058

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1058

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1058

A.6.2.12.2.6	Contention Resolution Timer expiry	1058

A.6.2.12.2.7	PRACH Resource Selection	1059

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1059

A.6.2.13.1	Test Purpose and Environment	1059

A.6.2.13.2	Test Requirements	1061

A.6.2.13.2.1	Random Access Response Reception	1061

A.6.2.13.2.2	No Random Access Response Reception	1061

A.6.2.13.2.3	Receiving a NACK on msg3	1062

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1062

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1062

A.6.2.13.2.6	Contention Resolution Timer expiry	1062

A.6.2.13.2.7	PRACH Resource Selection	1062

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1062

A.6.2.14.1	Test Purpose and Environment	1062

A.6.2.14.2	Test Requirements	1064

A.6.2.14.2.1	Random Access Response Reception	1064

A.6.2.14.2.2	No Random Access Response Reception	1065

A.6.2.14.2.3	Receiving a NACK on msg3	1065

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1065

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1065

A.6.2.14.2.6	Contention Resolution Timer expiry	1065

A.6.2.14.2.7	PRACH Resource Selection	1066

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1066

A.6.2.15.1	Test Purpose and Environment	1066

A.6.2.15.2	Test Requirements	1068

A.6.2.15.2.1	Random Access Response Reception	1068

A.6.2.15.2.2	No Random Access Response Reception	1069

A.6.2.15.2.3	Receiving a NACK on msg3	1069

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1069

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1069

A.6.2.15.2.6	Contention Resolution Timer expiry	1069

A.6.2.15.2.7	PRACH Resource Selection	1070

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1070

A.6.2.16.1	Test Purpose and Environment	1070

A.6.2.16.2	Test Requirements	1073

A.6.2.16.2.1	Random Access Response Reception	1073

A.6.2.16.2.2	No Random Access Response Reception	1074

A.6.2.16.2.3	Receiving a NACK on msg3	1074

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1074

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1074

A.6.2.16.2.6	Contention Resolution Timer expiry	1074

A.6.2.16.2.7	NPRACH Resource Selection	1074

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1075

A.6.2.17.1	Test Purpose and Environment	1075

A.6.2.17.2	Test Requirements	1078

A.6.2.17.2.1	Random Access Response Reception	1078

A.6.2.17.2.2	No Random Access Response Reception	1079

A.6.2.17.2.3	Receiving a NACK on msg3	1079

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1079

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1079

A.6.2.17.2.6	Contention Resolution Timer expiry	1079

A.6.2.17.2.7	NPRACH Resource Selection	1079

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1080

A.6.2.18.1	Test Purpose and Environment	1080

A.6.2.18.2	Test Requirements	1083

A.6.2.18.2.1	Random Access Response Reception	1083

A.6.2.18.2.2	No Random Access Response Reception	1084

A.6.2.18.2.3	Receiving a NACK on msg3	1084

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1084

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1084

A.6.2.18.2.6	Contention Resolution Timer expiry	1084

A.6.2.18.2.7	NPRACH Resource Selection	1084

A.6.3	RRC Connection Release with Redirection	1085

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1085

A.6.3.1.1	Test Purpose and Environment	1085

A.6.3.1.2	Test Requirements	1087

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1087

A.6.3.2.1	Test Purpose and Environment	1087

A.6.3.2.2	Test Requirements	1089

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1089

A.6.3.3.1	Test Purpose and Environment	1089

A.6.3.3.2	Test Requirements	1090

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1091

A.6.3.4.1	Test Purpose and Environment	1091

A.6.3.4.2	Test Requirements	1092

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1093

A.6.3.5.1	Test Purpose and Environment	1093

A.6.3.5.2	Test Requirements	1095

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1095

A.6.3.6.1	Test Purpose and Environment	1095

A. 6.3.6.2	Test Requirements	1097

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1097

A.6.3.7.1	Test Purpose and Environment	1097

A.6.3.7.2	Test Requirements	1100

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1100

A.6.3.8.1	Test Purpose and Environment	1100

A.6.3.8.2	Test Requirements	1103

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1103

A.6.3.9.1	Test Purpose and Environment	1103

A.6.3.9.2	Test Requirements	1105

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1105

A.6.3.10.1	Test Purpose and Environment	1105

A.6.3.10.2	Test Requirements	1107

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1107

A.6.3.11.1	Test Purpose and Environment	1107

A.6.3.11.2	Test Requirements	1109

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1109

A.6.3.12.1	Test Purpose and Environment	1109

A.6.3.12.2	Test Requirements	1112

A.7	Timing and Signalling Characteristics	1113

A.7.1	UE Transmit Timing	1113

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1113

A.7.1.1.1	Test Purpose and Environment	1113

A.7.1.1.2	Test Requirements	1114

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1115

A.7.1.2.1	Test Purpose and Environment	1115

A.7.1.2.2	Test Requirements	1117

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1118

A.7.1.3.1	Test Purpose and Environment	1118

A.7.1.3.2	Test Requirements	1120

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1121

A.7.1.4.1	Test Purpose and Environment	1121

A.7.1.4.2	Test Requirements	1123

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1124

A.7.1.4A.1	Test Purpose and Environment	1124

A.7.1.4A.2	Test Requirements	1124

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1124

A.7.1.5.1	Test Purpose and Environment	1124

A.7.1.5.2	Test Requirements	1125

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1125

A.7.1.6.1	Test Purpose and Environment	1125

A.7.1.6.2	Test Requirements	1127

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1128

A.7.1.7.1	Test Purpose and Environment	1128

A.7.1.7.2	Test Requirements	1129

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1130

A.7.1.7A.1	Test Purpose and Environment	1130

A.7.1.7A.2	Test Requirements	1130

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1130

A.7.1.7B.1	Test Purpose and Environment	1130

A.7.1.7B.2	Test Requirements	1130

A.7.1.8	Void	1131

A.7.1.8.1	Void	1131

A.7.1.8.2	Void	1131

A.7.1.9	Void	1131

A.7.1.9.1	Void	1131

A.7.1.9.2	Void	1131

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1131

A.7.1.10.1	Test Purpose and Environment	1131

A.7.1.10.2	Test Requirements	1133

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1133

A.7.1.11.1	Test Purpose and Environment	1133

A.7.1.11.2	Test Requirements	1135

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1135

A.7.1.12.1	Test Purpose and Environment	1135

A.7.1.12.2	Test Requirements	1137

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1137

A.7.1.13.1	Test Purpose and Environment	1137

A.7.1.13.2	Test Requirements	1139

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1140

A.7.1.14.1	Test Purpose and Environment	1140

A.7.1.14.2	Test Requirements	1141

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1141

A.7.1.15.1	Test Purpose and Environment	1141

A.7.1.15.2	Test Requirements	1142

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1143

A.7.1.16.1	Test Purpose and Environment	1143

A.7.1.16.2	Test Requirements	1144

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1145

A.7.1.17.1	Test Purpose and Environment	1145

A.7.1.17.2	Test Requirements	1147

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1148

A.7.1.18.1	Test Purpose and Environment	1148

A.7.1.18.2	Test Requirements	1150

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1151

A.7.1.19.1	Test Purpose and Environment	1151

A.7.1.19.2	Test Requirements	1152

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1152

A.7.1.20.1	Test Purpose and Environment	1152

A.7.1.20.2	Test Requirements	1154

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1154

A.7.1.21.1	Test Purpose and Environment	1154

A.7.1.21.2	Test Requirements	1156

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1156

A.7.1.22.1	Test Purpose and Environment	1156

A.7.1.22.2	Test Requirements	1158

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1158

A.7.1.23.1	Test Purpose and Environment	1158

A.7.1.23.2	Test Requirements	1160

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1160

A.7.1.24.1	Test Purpose and Environment	1160

A.7.1.24.2	Test Requirements	1161

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1162

A.7.1.25.1	Test Purpose and Environment	1162

A.7.1.25.2	Test Requirements	1163

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1164

A.7.1.26.1	Test Purpose and Environment	1164

A.7.1.26.2	Test Requirements	1165

A.7.2	UE Timing Advance	1166

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1166

A.7.2.1.1	Test Purpose and Environment	1166

A.7.2.1.2	Test Requirements	1168

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1168

A.7.2.2.1	Test Purpose and Environment	1168

A.7.2.2.2	Test Requirements	1170

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1170

A.7.2.3.1	Test Purpose and Environment	1170

A.7.2.3.2	Test Requirements	1171

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1171

A.7.2.4.1	Test Purpose and Environment	1171

A.7.2.4.2	Test Requirements	1173

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1173

A.7.2.5.1	Test Purpose and Environment	1173

A.7.2.5.2	Test Requirements	1175

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1175

A.7.2.5A.1	Test Purpose and Environment	1175

A.7.2.5A.2	Test Requirements	1176

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1176

A.7.2.5B.1	Test Purpose and Environment	1176

A.7.2.5B.2	Test Requirements	1176

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1176

A.7.2.6.1	Test Purpose and Environment	1176

A.7.2.6.2	Test Requirements	1179

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1179

A.7.2.7.1	Test Purpose and Environment	1179

A.7.2.7.2	Test Requirements	1181

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1181

A.7.2.8.1	Test Purpose and Environment	1181

A.7.2.8.2	Test Requirements	1183

A.7.2.9.2	Test Requirements	1185

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1185

A.7.2.10.1	Test Purpose and Environment	1185

A.7.2.10.2	Test Requirements	1186

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1186

A.7.2.11.1	Test Purpose and Environment	1186

A.7.2.11.2	Test Requirements	1188

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1188

A.7.2.12.1	Test Purpose and Environment	1188

A.7.2.12.2	Test Requirements	1190

A.7.3	Radio Link Monitoring	1190

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1191

A.7.3.1.1	Test Purpose and Environment	1191

A.7.3.1.2	Test Requirements	1195

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1195

A.7.3.2.1	Test Purpose and Environment	1195

A.7.3.2.2	Test Requirements	1198

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1198

A.7.3.3.1	Test Purpose and Environment	1198

A.7.3.3.2	Test Requirements	1202

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1202

A.7.3.4.1	Test Purpose and Environment	1202

A.7.3.4.2	Test Requirements	1205

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1205

A.7.3.5.1	Test Purpose and Environment	1205

A.7.3.5.2	Test Requirements	1209

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1210

A.7.3.6.1	Test Purpose and Environment	1210

A.7.3.6.2	Test Requirements	1213

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1213

A.7.3.7.1	Test Purpose and Environment	1213

A.7.3.7.2	Test Requirements	1216

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1216

A.7.3.8.1	Test Purpose and Environment	1216

A.7.3.8.2	Test Requirements	1219

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1219

A.7.3.9.2	Test Requirements	1222

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1222

A.7.3.10.1	Test Purpose and Environment	1222

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1225

A.7.3.11.1	Test Purpose and Environment	1225

A.7.3.11.2	Test Requirements	1229

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1229

A.7.3.12.1	Test Purpose and Environment	1229

A.7.3.12.2	Test Requirements	1233

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1233

A.7.3.13.1	Test Purpose and Environment	1233

A.7.3.13.2	Test Requirements	1236

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1236

A.7.3.14.1	Test Purpose and Environment	1236

A.7.3.14.2	Test Requirements	1239

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1239

A.7.3.15.1	Test Purpose and Environment	1239

A.7.3.15.2	Test Requirements	1242

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1242

A.7.3.16.1	Test Purpose and Environment	1242

A.7.3.16.2	Test Requirements	1246

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1246

A.7.3.17.1	Test Purpose and Environment	1246

A.7.3.17.2	Test Requirements	1249

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1249

A.7.3.18.1	Test Purpose and Environment	1249

A.7.3.18.2	Test Requirements	1253

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1253

A.7.3.19.1	Test Purpose and Environment	1253

A.7.3.19.2	Test Requirements	1257

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1257

A.7.3.20.1	Test Purpose and Environment	1257

A.7.3.20.2	Test Requirements	1261

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1261

A.7.3.21.1	Test Purpose and Environment	1261

A.7.3.21.2	Test Requirements	1265

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1265

A.7.3.22.1	Test Purpose and Environment	1265

A.7.3.22.2	Test Requirements	1269

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1269

A.7.3.23.1	Test Purpose and Environment	1269

A.7.3.23.2	Test Requirements	1270

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1270

A.7.3.24.1	Test Purpose and Environment	1270

A.7.3.24.2	Test Requirements	1271

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1271

A.7.3.25.1	Test Purpose and Environment	1271

A.7.3.25.2	Test Requirements	1272

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1272

A.7.3.26.1	Test Purpose and Environment	1272

A.7.3.26.2	Test Requirements	1275

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1275

A.7.3.27.1	Test Purpose and Environment	1275

A.7.3.27.2	Test Requirements	1278

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1278

A.7.3.28.1	Test Purpose and Environment	1278

A.7.3.28.2	Test Requirements	1281

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1281

A.7.3.29.1	Test Purpose and Environment	1281

A.7.3.29.2	Test Requirements	1284

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1284

A.7.3.30.1	Test Purpose and Environment	1284

A.7.3.30.2	Test Requirements	1287

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1287

A.7.3.31.1	Test Purpose and Environment	1287

A.7.3.31.2	Test Requirements	1290

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1290

A.7.3.32.1	Test Purpose and Environment	1290

A.7.3.32.2	Test Requirements	1293

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1293

A.7.3.33.1	Test Purpose and Environment	1293

A.7.3.33.2	Test Requirements	1296

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1296

A.7.3.34.1	Test Purpose and Environment	1296

A.7.3.34.2	Test Requirements	1299

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1299

A.7.3.35.1	Test Purpose and Environment	1299

A.7.3.35.2	Test Requirements	1302

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1302

A.7.3.36.1	Test Purpose and Environment	1302

A.7.3.36.2	Test Requirements	1305

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1305

A.7.3.37.1	Test Purpose and Environment	1305

A.7.3.37.2	Test Requirements	1308

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1308

A.7.3.38.1	Test Purpose and Environment	1308

A.7.3.38.2	Test Requirements	1311

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1311

A.7.3.39.1	Test Purpose and Environment	1311

A.7.3.39.2	Test Requirements	1314

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1315

A.7.3.40.1	Test Purpose and Environment	1315

A.7.3.40.2	Test Requirements	1317

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1318

A.7.3.41.1	Test Purpose and Environment	1318

A.7.3.41.2	Test Requirements	1321

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1321

A.7.3.42.1	Test Purpose and Environment	1321

A.7.3.42.2	Test Requirements	1324

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1324

A.7.3.43.1	Test Purpose and Environment	1324

A.7.3.43.2	Test Requirements	1327

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1327

A.7.3.44.1	Test Purpose and Environment	1327

A.7.3.44.2	Test Requirements	1330

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1331

A.7.3.45.1	Test Purpose and Environment	1331

A.7.3.45.2	Test Requirements	1333

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1334

A.7.3.46.1	Test Purpose and Environment	1334

A.7.3.46.2	Test Requirements	1337

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1337

A.7.3.47.1	Test Purpose and Environment	1337

A.7.3.47.2	Test Requirements	1340

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1340

A.7.3.48.1	Test Purpose and Environment	1340

A.7.3.48.2	Test Requirements	1343

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1343

A.7.3.49.1	Test Purpose and Environment	1343

A.7.3.49.2	Test Requirements	1346

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1346

A.7.3.50.1	Test Purpose and Environment	1346

A.7.3.50.2	Test Requirements	1349

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1349

A.7.3.51.1	Test Purpose and Environment	1349

A.7.3.51.2	Test Requirements	1352

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1352

A.7.3.52.1	Test Purpose and Environment	1352

A.7.3.52.2	Test Requirements	1355

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1355

A.7.3.53.1	Test Purpose and Environment	1355

A.7.3.53.2	Test Requirements	1358

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1358

A.7.3.54.1	Test Purpose and Environment	1358

A.7.3.54.2	Test Requirements	1361

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1361

A.7.3.55.1	Test Purpose and Environment	1361

A.7.3.55.2	Test Requirements	1364

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1364

A.7.3.56.1	Test Purpose and Environment	1364

A.7.3.56.2	Test Requirements	1367

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1367

A.7.3.57.1	Test Purpose and Environment	1367

A.7.3.57.2	Test Requirements	1370

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1370

A.7.3.58.1	Test Purpose and Environment	1370

A.7.3.58.2	Test Requirements	1373

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1373

A.7.3.59.1	Test Purpose and Environment	1373

A.7.3.59.2	Test Requirements	1376

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1376

A.7.3.60.1	Test Purpose and Environment	1376

A.7.3.60.2	Test Requirements	1380

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1380

A.7.3.61.1	Test Purpose and Environment	1380

A.7.3.61.2	Test Requirements	1384

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1384

A.7.3.62.1	Test Purpose and Environment	1384

A.7.3.62.2	Test Requirements	1388

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1388

A.7.3.63.1	Test Purpose and Environment	1388

A.7.3.63.2	Test Requirements	1392

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1392

A.7.3.64.1	Test Purpose and Environment	1392

A.7.3.64.2	Test Requirements	1395

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1396

A.7.3.65.1	Test Purpose and Environment	1396

A.7.3.65.2	Test Requirements	1399

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1400

A.7.3.66.1	Test Purpose and Environment	1400

A.7.3.66.2	Test Requirements	1402

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1403

A.7.3.67.1	Test Purpose and Environment	1403

A.7.3.67.2	Test Requirements	1406

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1407

A.7.3.68.1	Test Purpose and Environment	1407

A.7.3.68.2	Test Requirements	1409

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1409

A.7.3.69.1	Test Purpose and Environment	1409

A.7.3.69.2	Test Requirements	1411

The rate of correct events observed during repeated tests shall be at least 90%.A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1411

A.7.3.70.1	Test Purpose and Environment	1411

A.7.3.70.2	Test Requirements	1413

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1413

A.7.3.71.1	Test Purpose and Environment	1413

A.7.3.71.2	Test Requirements	1416

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1416

A.7.3.72.1	Test Purpose and Environment	1416

A.7.3.72.2	Test Requirements	1418

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1418

A.7.3.73.1	Test Purpose and Environment	1418

A.7.3.73.2	Test Requirements	1420

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1420

A.7.3.74.1	Test Purpose and Environment	1420

A.7.3.74.2	Test Requirements	1423

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1423

A.7.3.75.1	Test Purpose and Environment	1423

A.7.3.75.2	Test Requirements	1425

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1426

A.7.3.76.1	Test Purpose and Environment	1426

A.7.3.76.2	Test Requirements	1429

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1429

A.7.3.77.1	Test Purpose and Environment	1429

A.7.3.77.2	Test Requirements	1432

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1432

A.7.3.78.1	Test Purpose and Environment	1432

A.7.3.78.2	Test Requirements	1435

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1435

A.7.3.79.1	Test Purpose and Environment	1435

A.7.3.79.2	Test Requirements	1438

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1438

A.7.3.80.1	Test Purpose and Environment	1438

A.7.3.80.2	Test Requirements	1441

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1441

A.7.3.81.1	Test Purpose and Environment	1441

A.7.3.81.2	Test Requirements	1444

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1444

A.7.3.82.1	Test Purpose and Environment	1444

A.7.3.82.2	Test Requirements	1447

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1447

A.7.3.83.1	Test Purpose and Environment	1447

A.7.3.83.2	Test Requirements	1448

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1449

A.7.3.84.1	Test Purpose and Environment	1449

A.7.3.84.2	Test Requirements	1451

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1451

A.7.3.85.1	Test Purpose and Environment	1451

A.7.3.85.2	Test Requirements	1452

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1453

A.7.3.86.1	Test Purpose and Environment	1453

A.7.3.86.2	Test Requirements	1455

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1455

A.7.3.87.1	Test Purpose and Environment	1455

A.7.3.87.2	Test Requirements	1457

A.7.4	Interruption for Dual Connectivity	1458

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1458

A.7.4.1.1	Test Purpose and Environment	1458

A.7.4.1.2	Test Requirements	1460

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1460

A.7.4.2.1	Test Purpose and Environment	1460

A.7.4.2.2	Test Requirements	1462

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1462

A.7.4.3.1	Test Purpose and Environment	1462

A.7.4.3.2	Test Requirements	1464

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1464

A.7.4.4.1	Test Purpose and Environment	1464

A.7.4.4.2	Test Requirements	1467

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1467

A.7.4.5.1	Test Purpose and Environment	1467

A.7.4.5.2	Test Requirements	1469

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1469

A.7.4.6.1	Test Purpose and Environment	1469

A.7.4.6.2	Test Requirements	1471

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1471

A.7.4.7.1	Test Purpose and Environment	1471

A.7.4.7.2	Test Requirements	1474

A.7.5	Proximity-based Services	1474

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1474

A.7.5.1.1	Test Purpose and Environment	1474

A.7.5.1.2	Test Requirements	1475

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1476

A.7.5.2.1	Test Purpose and Environment	1476

A.7.5.1.2	Test Requirements	1476

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1477

A.7.5.3.1	Test Purpose and Environment	1477

A.7.5.3.2	Test Requirements	1478

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1479

A.7.5.4.1	Test Purpose and Environment	1479

A.7.5.4.2	Test Requirements	1480

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1481

A.7.5.5.1	Test Purpose and Environment	1481

A.7.5.5.2	Test Requirements	1483

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1484

A.7.5.6.1	Test Purpose and Environment	1484

A.7.5.6.2	Test Requirements	1485

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1486

A.7.5.7.1	Test Purpose and Environment	1486

A.7.5.7.2	Test Requirements	1488

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1488

A.7.5.8.1	Test Purpose and Environment	1488

A.7.5.8.2	Test Requirements	1490

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1491

A.7.5.9.1	Test Purpose and Environment	1491

A.7.5.9.2	Test Requirements	1493

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1494

A.7.5.10.1	Test Purpose and Environment	1494

A.7.5.10.2	Test Requirements	1496

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1497

A.7.5.11.1	Test Purpose and Environment	1497

A.7.5.11.2	Test Requirements	1499

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1499

A.7.5.12.1	Test Purpose and Environment	1499

A.7.5.12.2	Test Requirements	1503

A.7.6	Interruption for carrier aggregation	1504

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1504

A.7.6.1.1	Test Purpose and Environment	1504

A.7.6.1.2	Test Requirements	1506

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1506

A.7.6.2.1	Test Purpose and Environment	1506

A.7.6.2.2	Test Requirements	1509

A.8	UE Measurements Procedures	1509

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1509

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1509

A.8.1.1.1	Test Purpose and Environment	1509

A.8.1.1.2	Test Requirements	1511

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1511

A.8.1.2.1	Test Purpose and Environment	1511

A.8.1.2.2	Test Requirements	1513

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1513

A.8.1.3.1	Test Purpose and Environment	1513

A.8.1.3.2	Test Requirements	1516

A.8.1.4	Void	1516

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1516

A.8.1.5.1	Test Purpose and Environment	1516

A.8.1.5.2	Test Requirements	1518

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1518

A.8.1.6.1	Test Purpose and Environment	1518

A.8.1.6.2	Test Requirements	1520

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1520

A.8.1.7.1	Test Purpose and Environment	1520

A.8.1.7.2	Test Requirements	1522

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1523

A.8.1.8.1	Test Purpose and Environment	1523

A.8.1.8.2	Test Requirements	1525

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1526

A.8.1.9.1	Test Purpose and Environment	1526

A.8.1.9.2	Test Requirements	1526

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1526

A.8.1.10.1	Test Purpose and Environment	1526

A.8.1.10.2	Test Requirements	1527

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1527

A.8.1.11.1	Test Purpose and Environment	1527

A.8.1.11.2	Test Requirements	1529

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1530

A.8.1.12.1	Test Purpose and Environment	1530

A.8.1.12.2	Test Requirements	1531

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1532

A.8.1.13.1	Test Purpose and Environment	1532

A.8.1.13.2	Test Requirements	1534

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1534

A.8.1.14.1	Test Purpose and Environment	1534

A.8.1.14.2	Test Requirements	1536

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1537

A.8.1.15.1	Test Purpose and Environment	1537

A.8.1.15.2	Test Requirements	1538

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1539

A.8.1.16.1	Test Purpose and Environment	1539

A.8.1.16.2	Test Requirements	1541

A.8.1.17	Void	1541

A.8.1.18	Void	1541

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1541

A.8.1.19.1	Test Purpose and Environment	1541

A.8.1.19.2	Test Requirements	1543

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1544

A.8.1.20.1	Test Purpose and Environment	1544

A.8.1.20.2	Test Requirements	1546

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1546

A.8.1.21.1	Test Purpose and Environment	1546

A.8.1.21.2	Test Requirements	1548

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1549

A.8.1.22.1	Test Purpose and Environment	1549

A.8.1.22.2	Test Requirements	1551

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1551

A.8.1.23.1	Test Purpose and Environment	1551

A.8.1.23.2	Test Requirements	1552

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1553

A.8.1.24.1	Test Purpose and Environment	1553

A.8.1.24.2	Test Requirements	1555

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1555

A.8.1.25.1	Test Purpose and Environment	1555

A.8.1.25.2	Test Requirements	1557

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1557

A.8.1.26.1	Test Purpose and Environment	1557

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1560

A.8.1.27.1	Test Purpose and Environment	1560

A.8.1.27.2	Test Requirements	1562

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1562

A.8.1.28.1	Test Purpose and Environment	1562

A.8.1.28.2	Test Requirements	1564

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1564

A.8.1.29.1	Test Purpose and Environment	1564

A.8.1.29.2	Test Requirements	1566

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1567

A.8.1.30.1	Test Purpose and Environment	1567

A.8.1.30.2	Test Requirements	1569

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1569

A.8.1.31.1	Test Purpose and Environment	1569

A.8.1.31.2	Test Requirements	1571

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1572

A.8.1.32.1	Test Purpose and Environment	1572

A.8.1.32.2	Test Requirements	1573

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1574

A.8.1.33.1	Test Purpose and Environment	1574

A.8.1.33.2	Test Requirements	1575

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1576

A.8.1.34.1	Test Purpose and Environment	1576

A.8.1.34.2	Test Requirements	1577

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1578

A.8.1.35.1	Test Purpose and Environment	1578

A.8.1.35.2	Test Requirements	1579

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1580

A.8.1.36.1	Test Purpose and Environment	1580

A.8.1.36.2	Test Requirements	1581

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1582

A.8.1.37.1	Test Purpose and Environment	1582

A.8.1.37.2	Test Requirements	1584

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1584

A.8.1.38.1	Test Purpose and Environment	1584

A.8.1.38.2	Test Requirements	1585

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1586

A.8.1.39.1	Test Purpose and Environment	1586

A.8.1.39.2	Test Requirements	1588

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1588

A.8.1.40.1	Test Purpose and Environment	1588

A.8.1.40.2	Test Requirements	1590

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1590

A.8.1.41.1	Test Purpose and Environment	1590

A.8.1.41.2	Test Requirement	1592

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1592

A.8.1.42.1	Test Purpose and Environment	1592

A.8.1.42.2	Test Requirement	1593

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1594

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1594

A.8.2.1.1	Test Purpose and Environment	1594

A.8.2.1.2	Test Requirements	1595

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1595

A.8.2.2.1	Test Purpose and Environment	1595

A.8.2.2.2	Test Requirements	1598

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1598

A.8.2.3.1	Test Purpose and Environment	1598

A.8.2.3.2	Test Requirements	1600

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1601

A.8.2.4.1	Test Purpose and Environment	1601

A.8.2.4.2	Test Requirements	1603

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1603

A.8.2.5.1	Test Purpose and Environment	1603

A.8.2.5.2	Test Requirements	1605

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1606

A.8.2.6.1	Test Purpose and Environment	1606

A.8.2.6.2	Test Requirements	1608

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1609

A.8.2.7.1	Test Purpose and Environment	1609

A.8.2.7.2	Test Requirements	1610

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1611

A.8.2.8.1	Test Purpose and Environment	1611

A.8.2.8.2	Test Requirements	1613

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1613

A.8.2.9.1	Test Purpose and Environment	1613

A.8.2.9.2	Test Requirements	1615

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1616

A.8.2.10.1	Test Purpose and Environment	1616

A.8.2.10.2	Test Requirements	1618

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1618

A.8.2.11.1	Test Purpose and Environment	1618

A.8.2.11.2	Test Requirements	1620

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1620

A.8.2.12.1	Test Purpose and Environment	1620

A.8.2.12.2	Test Requirements	1622

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1623

A.8.2.13.1	Test Purpose and Environment	1623

A.8.2.13.2	Test Requirements	1625

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1625

A.8.2.14.1	Test Purpose and Environment	1625

A.8.2.14.2	Test Requirement	1627

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1628

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1628

A.8.3.1.1	Test Purpose and Environment	1628

A.8.3.1.2	Test Requirements	1629

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1629

A.8.3.2.1	Test Purpose and Environment	1629

A.8.3.2.2	Test Requirements	1632

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1632

A.8.3.3.1	Test Purpose and Environment	1632

A.8.3.3.2	Test Requirements	1635

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1635

A.8.3.4.1	Test Purpose and Environment	1635

A.8.3.4.2	Test Requirements	1637

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1637

A.8.3.5.2	Test Requirements	1639

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1639

A.8.3.6.1	Test Purpose and Environment	1639

A.8.3.6.2	Test Requirements	1641

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1642

A.8.3.7.1	Test Purpose and Environment	1642

A.8.3.7.2	Test Requirements	1644

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1645

A.8.3.8.1	Test Purpose and Environment	1645

A.8.3.8.2	Test Requirements	1648

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1648

A.8.3.9.1	Test Purpose and Environment	1648

A.8.3.9.2	Test Requirements	1651

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1652

A.8.3.10.1	Test Purpose and Environment	1652

A.8.3.10.2	Test Requirements	1653

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1653

A.8.3.11.1	Test Purpose and Environment	1653

A.8.3.11.2	Test Requirement	1655

A.8.3.12	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1656

A.8.3.12.1	Test Purpose and Environment	1656

A.8.3.12.2	Test Requirement	1657

A.8.3.13	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1658

A.8.3.13.1	Test Purpose and Environment	1658

A.8.3.13.2	Test Requirement	1659

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1660

A.8.3.14.1	Test Purpose and Environment	1660

A.8.3.14.2	Test Requirement	1661

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1662

A.8.3.15.1	Test Purpose and Environment	1662

A.8.3.15.2	Test Requirement	1663

A.8.3.16	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1664

A.8.3.16.1	Test Purpose and Environment	1664

A.8.3.16.2	Test Requirement	1666

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1666

A.8.3.17.1	Test Purpose and Environment	1666

A.8.3.17.2	Test Requirement	1669

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1669

A.8.3.18.1	Test Purpose and Environment	1669

A.8.3.18.2	Test Requirement	1672

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1672

A.8.3.19.1	Test Purpose and Environment	1672

A.8.3.19.2	Test Requirement	1675

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1675

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1675

A.8.4.1.1	Test Purpose and Environment	1675

A.8.4.1.2	Test Requirements	1677

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1677

A.8.4.2.1	Test Purpose and Environment	1677

A.8.4.2.2	Test Requirements	1680

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1680

A.8.4.3.1	Test Purpose and Environment	1680

A.8.4.3.2	Test Requirements	1682

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1683

A.8.4.4.1	Test Purpose and Environment	1683

A.8.4.4.2	Test Requirements	1685

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1686

A.8.4.5.1	Test Purpose and Environment	1686

A.8.4.5.2	Test Requirements	1688

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1688

A.8.4.6.1	Test Purpose and Environment	1688

A.8.4.6.2	Test Requirements	1689

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1689

A.8.4.7.1	Test Purpose and Environment	1689

A.8.4.7.2	Test Requirements	1693

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1693

A.8.4.8.1	Test Purpose and Environment	1693

A.8.4.8.2	Test Requirements	1696

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1696

A.8.4.9.1	Test Purpose and Environment	1696

A.8.4.9.2	Test Requirements	1700

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1700

A.8.4.10.1	Test Purpose and Environment	1700

A.8.4.10.2	Test Requirements	1702

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1702

A.8.4.11.1	Test Purpose and Environment	1702

A.8.4.11.2	Test Requirement	1704

A.8.4.12	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1704

A.8.4.12.1	Test Purpose and Environment	1704

A.8.4.12.2	Test Requirement	1706

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1707

A.8.4.13.1	Test Purpose and Environment	1707

A.8.4.13.2	Test Requirement	1708

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1709

A.8.4.14.1	Test Purpose and Environment	1709

A.8.4.14.2	Test Requirement	1711

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1711

A.8.4.15.1	Test Purpose and Environment	1711

A.8.4.15.2	Test Requirement	1714

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1714

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1714

A.8.5.1.1	Test Purpose and Environment	1714

A.8.5.1.2	Test Requirements	1716

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1716

A.8.5.2.1	Test Purpose and Environment	1716

A.8.5.2.2	Test Requirements	1718

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1718

A.8.5.3.1	Test Purpose and Environment	1718

A.8.5.3.2	Test Requirements	1721

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1721

A.8.5.4.1	Test Purpose and Environment	1721

A.8.5.4.2	Test Requirements	1724

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1724

A.8.5.5.1	Test Purpose and Environment	1724

A.8.5.5.2	Test Requirements	1726

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1726

A.8.5.6.1	Test Purpose and Environment	1726

A.8.5.6.2	Test Requirements	1727

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1728

A.8.5.7.1	Test Purpose and Environment	1728

A.8.5.7.2	Test Requirements	1728

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1728

A.8.5.8.1	Test Purpose and Environment	1728

A.8.5.8.2	Test Requirements	1731

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1731

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1731

A.8.6.1.1	Test Purpose and Environment	1731

A.8.6.1.2	Test Requirements	1733

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1734

A.8.6.2.1	Test Purpose and Environment	1734

A.8.6.2.2	Test Requirements	1736

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1736

A.8.6.3.1	Test Purpose and Environment	1736

A.8.6.3.2	Test Requirements	1739

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1739

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1739

A.8.7.1.1	Test Purpose and Environment	1739

A.8.7.1.1.1	Void	1739

A.8.7.1.1.2	1.28 Mcps TDD option	1739

A.8.7.1.1.3	Void	1741

A.8.7.1.2	Test Requirements	1741

A.8.7.1.2.1	Void	1741

A.8.7.1.2.2	1.28 Mcps TDD option	1741

A.8.7.1.2.3	Void	1741

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1741

A.8.7.2.1	Test Purpose and Environment	1741

A.8.7.2.2	Test Requirements	1744

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1745

A.8.7.3.1	Test Purpose and Environment	1745

A.8.7.3.2	Test Parameters	1745

A.8.7.3.3	Test Requirements	1746

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1747

A.8.7.4.1	Test Purpose and Environment	1747

A.8.7.4.2	Test Requirements	1749

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1749

A.8.7.5.1	Test Purpose and Environment	1749

A.8.7.5.2	Test Requirements	1751

A.8.7A	TBD	1751

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1751

A.8.7A.1.1	Test Purpose and Environment	1751

A.8.7A.1.2	Test Requirements	1754

A.8.8	E-UTRAN FDD – GSM Measurements	1754

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1754

A.8.8.1.1	Test Purpose and Environment	1754

A.8.8.1.2	Test Requirements	1756

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1756

A.8.8.2.1	Test Purpose and Environment	1756

A.8.8.2.2	Test Requirements	1758

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1759

A.8.8.3.1	Test Purpose and Environment	1759

A.8.8.3.2	Test Requirements	1760

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1761

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1761

A.8.9.1.1	Test Purpose and Environment	1761

A.8.9.1.2	Test Requirements	1762

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1763

A.8.9.2.1	Test Purpose and Environment	1763

A.8.9.2.2	Test Requirements	1765

A.8.10	E-UTRAN TDD – GSM Measurements	1765

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1765

A.8.10.1.1	Test Purpose and Environment	1765

A.8.10.1.2	Test Requirements	1766

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1767

A.8.10.2.1	Test Purpose and Environment	1767

A.8.10.2.2	Test Requirements	1769

A.8.11	Monitoring of Multiple Layers	1769

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1769

A.8.11.1.1	Test Purpose and Environment	1769

A.8.11.1.2	Test Requirements	1771

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1772

A.8.11.2.1	Test Purpose and Environment	1772

A.8.11.2.2	Test Requirements	1773

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1773

A.8.11.3.1	Test Purpose and Environment	1773

A.8.11.3.2	Test Requirements	1776

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1776

A.8.11.4.1	Test Purpose and Environment	1776

A.8.11.4.2	Test Requirements	1779

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1779

A.8.11.5.1	Test Purpose and Environment	1779

A.8.11.5.2	Test Requirements	1781

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1782

A.8.11.6.1	Test Purpose and Environment	1782

A.8.11.6.2	Test Requirements	1784

A.8.12	RSTD Intra-frequency Measurements	1785

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1785

A.8.12.1.1	Test Purpose and Environment	1785

A.8.12.1.2	Test Requirements	1788

A.8.12.1.2A	Test Requirements for UE Category 1bis	1789

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1789

A.8.12.2.1	Test Purpose and Environment	1789

A.8.12.2.2	Test Requirements	1793

A.8.12.2.2A	Test Requirements for UE Category 1bis	1794

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	1794

A.8.12.3.1	Test Purpose and Environment	1794

A.8.12.3.2	Test Requirements	1798

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	1799

A.8.12.4.1	Test Purpose and Environment	1799

A.8.12.4.2	Test Requirements	1802

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	1803

A.8.12.5.1	Test Purpose and Environment	1803

A.8.12.5.2	Test Requirements	1806

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	1807

A.8.12.6.1	Test Purpose and Environment	1807

A.8.12.6.2	Test Requirements	1810

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	1811

A.8.12.7.1	Test Purpose and Environment	1811

A.8.12.7.2	Test Requirements	1814

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	1815

A.8.12.8.1	Test Purpose and Environment	1815

A.8.12.8.2	Test Requirements	1818

A.8.13	RSTD Inter-frequency Measurements	1819

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1819

A.8.13.1.1	Test Purpose and Environment	1819

A.8.13.1.2	Test Requirements	1822

A.8.13.1.2A	Test Requirements for UE Category 1bis	1823

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1823

A.8.13.2.1	Test Purpose and Environment	1823

A.8.13.2.2	Test Requirements	1827

A.8.13.2.2A	Test Requirements for UE Category 1bis	1828

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	1828

A.8.13.3.1	Test Purpose and Environment	1828

A.8.13.3.2	Test Requirements	1832

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	1833

A.8.13.4.1	Test Purpose and Environment	1833

A.8.13.4.2	Test Requirements	1836

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	1837

A.8.13.5.1	Test Purpose and Environment	1837

A.8.13.5.2	Test Requirements	1841

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	1842

A.8.13.6.1	Test Purpose and Environment	1842

A.8.13.6.2	Test Requirements	1845

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	1846

A.8.13.7.1	Test Purpose and Environment	1846

A.8.13.7.2	Test Requirements	1849

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	1850

A.8.13.8.1	Test Purpose and Environment	1850

A.8.13.8.2	Test Requirements	1854

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	1855

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1855

A.8.14.1.1	Test Purpose and Environment	1855

A.8.14.1.2	Test Requirements	1857

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1857

A.8.14.2.1	Test Purpose and Environment	1857

A.8.14.2.2	Test Requirements	1860

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1860

A.8.14.3.1	Test Purpose and Environment	1860

A.8.14.3.2	Test Requirements	1862

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	1863

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1863

A.8.15.1.1	Test Purpose and Environment	1863

A.8.15.1.2	Test Requirements	1864

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1864

A.8.15.2.1	Test Purpose and Environment	1864

A.8.15.2.2	Test Requirements	1867

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1867

A.8.15.3.1	Test Purpose and Environment	1867

A.8.15.3.2	Test Requirements	1869

A.8.16	E-UTRAN Carrier Aggregation Measurements	1870

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	1870

A.8.16.1.1	Test Purpose and Environment	1870

A.8.16.1.2	Test Requirements	1872

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	1872

A.8.16.2.1	Test Purpose and Environment	1872

A.8.16.2.2	Test Requirements	1874

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1874

A.8.16.3.1	Test Purpose and Environment	1874

A.8.16.3.2	Test Requirements	1876

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	1877

A.8.16.3A.1	Test Purpose and Environment	1877

A.8.16.3A.2	Test Requirements	1878

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	1879

A.8.16.4.1	Test Purpose and Environment	1879

A.8.16.4.2	Test Requirements	1880

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1881

A.8.16.4A.1	Test Purpose and Environment	1881

A.8.16.4A.2	Test Requirements	1883

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1884

A.8.16.5.1	Test Purpose and Environment	1884

A.8.16.5.2	Test Requirements	1884

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1884

A.8.16.6.1	Test Purpose and Environment	1884

A.8.16.6.2	Test Requirements	1885

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1885

A.8.16.7.1	 Test Purpose and Environment	1885

A.8.16.7.2	 Test Requirements	1886

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1886

A.8.16.8.1	 Test Purpose and Environment	1886

A.8.16.8.2	 Test Requirements	1887

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1887

A.8.16.9.1	Test Purpose and Environment	1887

A.8.16.9.2	Test Requirements	1888

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1888

A.8.16.10.1	Test Purpose and Environment	1888

A.8.16.10.2	Test Requirements	1889

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1889

A.8.16.11.1	 Test Purpose and Environment	1889

A.8.16.11.2	 Test Requirements	1890

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1890

A.8.16.12.1	 Test Purpose and Environment	1890

A.8.16.12.2	 Test Requirements	1890

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	1890

A.8.16.13.1	Test Purpose and Environment	1890

A.8.16.13.2	Test Requirements	1891

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	1891

A.8.16.14.1	Test Purpose and Environment	1891

A.8.16.14.2	Test Requirements	1892

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	1892

A.8.16.15.1	 Test Purpose and Environment	1892

A.8.16.7.2	 Test Requirements	1892

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	1892

A.8.16.16.1	 Test Purpose and Environment	1892

A.8.16.16.2	 Test Requirements	1893

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	1893

A.8.16.17.1	Test Purpose and Environment	1893

A.8.16.17.2	Test Requirements	1895

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	1896

A.8.16.17A.1	Test Purpose and Environment	1896

A.8.16.17A.2	Test Requirements	1896

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1896

A.8.16.17B.1	Test Purpose and Environment	1896

A.8.16.17B.2	Test Requirements	1897

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1897

A.8.16.17C.1	Test Purpose and Environment	1897

A.8.16.17C.2	Test Requirements	1897

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	1897

A.8.16.18.1	Test Purpose and Environment	1897

A.8.16.18.2	Test Requirements	1899

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	1900

A.8.16.18A.1	Test Purpose and Environment	1900

A.8.16.18A.2	Test Requirements	1900

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1900

A.8.16.18B.1	Test Purpose and Environment	1900

A.8.16.18B.2	Test Requirements	1901

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1901

A.8.16.18C.1	Test Purpose and Environment	1901

A.8.16.18C.2	Test Requirements	1901

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	1901

A.8.16.18D.1	Test Purpose and Environment	1901

A.8.16.18D.2	Test Requirements	1902

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	1902

A.8.16.19.1	Test Purpose and Environment	1902

A.8.16.19.2	Test Requirements	1904

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1904

A.8.16.19A.1	Test Purpose and Environment	1904

A.8.16.19A.2	Test Requirements	1905

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1905

A.8.16.19B.1	Test Purpose and Environment	1905

A.8.16.19B.2	Test Requirements	1905

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1905

A.8.16.19C.1	Test Purpose and Environment	1905

A.8.16.19C.2	Test Requirements	1906

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	1906

A.8.16.20.1	Test Purpose and Environment	1906

A.8.16.20.2	Test Requirements	1908

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1909

A.8.16.20A.1	Test Purpose and Environment	1909

A.8.16.20A.2	Test Requirements	1909

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1909

A.8.16.20B.1	Test Purpose and Environment	1909

A.8.16.20B.2	Test Requirements	1910

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1910

A.8.16.20C.1	Test Purpose and Environment	1910

A.8.16.20C.2	Test Requirements	1910

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	1910

A.8.16.20D.1	Test Purpose and Environment	1910

A.8.16.20D.2	Test Requirements	1911

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	1911

A.8.16.21.1	Test Purpose and Environment	1911

A.8.16.21.2	Test Requirements	1913

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	1913

A.8.16.22.1	Test Purpose and Environment	1913

A.8.16.22.2	Test Requirements	1915

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	1915

A.8.16.23.1	Test Purpose and Environment	1915

A.8.16.23.2	Test Requirements	1918

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	1918

A.8.16.24.1	Test Purpose and Environment	1918

A.8.16.24.2	Test Requirements	1921

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	1921

A.8.16.25.1	Test Purpose and Environment	1921

A.8.16.25.2	Test Requirements	1923

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	1923

A.8.16.26.1	Test Purpose and Environment	1923

A.8.16.26.2	Test Requirements	1926

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1926

A.8.16.27.1	Test Purpose and Environment	1926

A.8.16.27.2	Test Requirements	1930

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1930

A.8.16.28.1	Test Purpose and Environment	1930

A.8.16.28.2	Test Requirements	1934

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1934

A.8.16.29.1	Test Purpose and Environment	1934

A.8.16.29.2	Test Requirements	1938

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1938

A.8.16.30.1	Test Purpose and Environment	1938

A.8.16.30.2	Test Requirements	1942

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	1942

A.8.16.31.1	Test Purpose and Environment	1942

A.8.16.31.2	Test Requirements	1946

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	1946

A.8.16.32.1	Test Purpose and Environment	1946

A.8.16.32.2	Test Requirements	1950

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1950

A.8.16.33.1	Test Purpose and Environment	1950

A.8.16.33.2	Test Requirements	1954

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1954

A.8.16.34.1	Test Purpose and Environment	1954

A.8.16.34.2	Test Requirements	1958

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	1958

A.8.16.35.1	Test Purpose and Environment	1958

A.8.16.35.2	Test Requirements	1960

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	1961

A.8.16.36.1	Test Purpose and Environment	1961

A.8.16.36.2	Test Requirements	1963

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	1964

A.8.16.37.1	Test Purpose and Environment	1964

A.8.16.37.2	Test Requirements	1966

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	1967

A.8.16.38.1	Test Purpose and Environment	1967

A.8.16.38.2	Test Requirements	1969

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	1970

A.8.16.39.1	Test Purpose and Environment	1970

A.8.16.39.2	Test Requirements	1973

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	1973

A.8.16.40.1	Test Purpose and Environment	1973

A.8.16.40.2	Test Requirements	1976

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	1976

A.8.16.41.1	Test Purpose and Environment	1976

A.8.16.41.2	Test Requirements	1978

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	1979

A.8.16.42.1	Test Purpose and Environment	1979

A.8.16.42.2	Test Requirements	1981

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	1982

A.8.16.43.1	Test Purpose and Environment	1982

A.8.16.43.2	Test Requirements	1985

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	1985

A.8.16.44.1	Test Purpose and Environment	1985

A.8.16.44.2	Test Requirements	1988

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	1988

A.8.16.45.1	Test Purpose and Environment	1988

A.8.16.45.2	Test Requirements	1991

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	1991

A.8.16.46.1	Test Purpose and Environment	1991

A.8.16.46.2	Test Requirements	1993

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	1994

A.8.16.47.1	Test Purpose and Environment	1994

A.8.16.47.2	Test Requirements	1996

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	1997

A.8.16.48.1	Test Purpose and Environment	1997

A.8.16.48.2	Test Requirements	1999

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2000

A.8.16.49.1	Test Purpose and Environment	2000

A.8.16.49.2	Test Requirements	2003

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2003

A.8.16.50.1	Test Purpose and Environment	2003

A.8.16.50.2	Test Requirements	2005

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2006

A.8.16.51.1	Test Purpose and Environment	2006

A.8.16.51.2	Test Requirements	2010

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2010

A.8.16.52.1	Test Purpose and Environment	2010

A.8.16.52.2	Test Requirements	2014

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2014

A.8.16.53.1	Test Purpose and Environment	2014

A.8.16.53.2	Test Requirements	2018

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2018

A.8.16.54.1	Test Purpose and Environment	2018

A.8.16.54.2	Test Requirements	2022

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2022

A.8.16.55.1	Test Purpose and Environment	2022

A.8.16.55.2	Test Requirements	2027

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2027

A.8.16.56.1	Test Purpose and Environment	2027

A.8.16.56.2	Test Requirements	2032

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2032

A.8.16.57.1	Test Purpose and Environment	2032

A.8.16.57.2	Test Requirements	2034

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2035

A.8.16.58.1	Test Purpose and Environment	2035

A.8.16.58.2	Test Requirements	2037

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2038

A.8.16.59.1	Test Purpose and Environment	2038

A.8.16.59.2	Test Requirements	2042

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2042

A.8.16.60.1	Test Purpose and Environment	2042

A.8.16.60.2	Test Requirements	2045

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2046

A.8.16.61.1	Test Purpose and Environment	2046

A.8.16.61.2	Test Requirements	2049

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2049

A.8.16.62.1	Test Purpose and Environment	2049

A.8.16.62.2	Test Requirements	2051

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2052

A.8.16.63.1	Test Purpose and Environment	2052

A.8.16.63.2	Test Requirements	2055

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2056

A.8.16.64.1	Test Purpose and Environment	2056

A.8.16.64.2	Test Requirements	2060

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2060

A.8.16.65.1	Test Purpose and Environment	2060

A.8.16.65.2	Test Requirements	2065

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2065

A.8.16.66.1	Test Purpose and Environment	2065

A.8.16.66.2	Test Requirements	2070

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2070

A.8.16.67.1	Test Purpose and Environment	2070

A.8.16.67.2	Test Requirements	2075

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2075

A.8.16.68.1	Test Purpose and Environment	2075

A.8.16.68.2	Test Requirements	2080

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2080

A.8.16.69.1	Test Purpose and Environment	2080

A.8.16.69.2	Test Requirements	2083

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2083

A.8.16.70.1	Test Purpose and Environment	2083

A.8.16.70.2	Test Requirements	2086

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2086

A.8.16.71.1	Test Purpose and Environment	2086

A.8.16.71.2	Test Requirements	2090

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2090

A.8.16.72.1	Test Purpose and Environment	2090

A.8.16.72.2	Test Requirements	2094

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2094

A.8.16.73.1	Test Purpose and Environment	2094

A.8.16.73.2	Test Requirements	2099

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2099

A.8.16.74.1	Test Purpose and Environment	2099

A.8.16.74.2	Test Requirements	2104

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2104

A.8.16.75.1	Test Purpose and Environment	2104

A.8.16.75.2	Test Requirements	2109

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2109

A.8.16.76.1	Test Purpose and Environment	2109

A.8.16.76.2	Test Requirements	2114

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2114

A.8.16.77.1	Test Purpose and Environment	2114

A.8.16.77.2	Test Requirements	2117

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2117

A.8.16.78.1	Test Purpose and Environment	2117

A.8.16.78.2	Test Requirements	2120

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2121

A.8.16.79.1	Test Purpose and Environment	2121

A.8.16.79.2	Test Requirements	2126

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2126

A.8.16.80.1	Test Purpose and Environment	2126

A.8.16.80.2	Test Requirements	2131

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2131

A.8.16.81.1	Test Purpose and Environment	2131

A.8.16.81.2	Test Requirements	2136

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2136

A.8.16.82.1	Test Purpose and Environment	2136

A.8.16.82.2	Test Requirements	2141

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2141

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2141

A.8.17.1.1	Test Purpose and Environment	2141

A.8.17.1.2	Test Requirements	2146

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2146

A.8.17.2.1	Test Purpose and Environment	2146

A.8.17.2.2	Test Requirements	2152

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2152

A.8.17.3.1	Test Purpose and Environment	2152

A.8.17.3.2	Test Requirements	2153

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2153

A.8.17.4.1	Test Purpose and Environment	2153

A.8.17.4.2	Test Requirements	2154

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2154

A.8.17.5.1	Test Purpose and Environment	2154

A.8.17.5.2	Test Requirements	2155

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2155

A.8.17.6.1	Test Purpose and Environment	2155

A.8.17.6.2	Test Requirements	2156

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2157

A.8.17.7.1	Test Purpose and Environment	2157

A.8.17.7.2	Test Requirements	2157

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2158

A.8.17.8.1	Test Purpose and Environment	2158

A.8.17.8.2	Test Requirements	2158

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2159

A.8.17.9.1	Test Purpose and Environment	2159

A.8.17.9.2	Test Requirements	2160

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2160

A.8.17.10.1	Test Purpose and Environment	2160

A.8.17.10.2	Test Requirements	2165

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2166

A.8.17.11.1	Test Purpose and Environment	2166

A.8.17.11.2	Test Requirements	2172

A.8.18	E-UTRAN TDD – HRPD Measurements	2173

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2173

A.8.18.1.1	Test Purpose and Environment	2173

A.8.18.1.2	Test Requirements	2175

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2175

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2175

A.8.19.1.1	Test Purpose and Environment	2175

A.8.19.1.2	Test Requirements	2176

A.8.20	Inter-frequency/RAT Measurements in CA mode	2177

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2177

A.8.20.1.1	Test Purpose and Environment	2177

A.8.20.1.2	Test Requirements	2178

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2178

A.8.20.2.1	Test Purpose and Environment	2179

A.8.20.2.2	Test Requirements	2180

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2181

A.8.20.2A.1	Test Purpose and Environment	2181

A.8.20.2A.2	Test Requirements	2181

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2181

A.8.20.2B.1	Test Purpose and Environment	2181

A.8.20.2B.2	Test Requirements	2184

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2184

A.8.20.3.1	Test Purpose and Environment	2184

A.8.20.3.2	Test Requirements	2186

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2186

A.8.20.4.1	Test Purpose and Environment	2186

A.8.20.4.1.1	1.28 Mcps TDD option	2186

A.8.20.4.2	Test Requirements	2188

A.8.20.4.2.1	1.28 Mcps TDD option	2188

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2188

A.8.20.4A.1	Test Purpose and Environment	2188

A.8.20.4A.1.1	1.28 Mcps TDD option	2188

A.8.20.4A.2	Test Requirements	2188

A.8.20.4A.2.1	1.28 Mcps TDD option	2188

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2189

A.8.20.4B.1	Test Purpose and Environment	2189

A.8.20.4B.1.1	1.28 Mcps TDD option	2189

A.8.20.4B.2	Test Requirements	2191

A.8.20.4B.2.1	1.28 Mcps TDD option	2191

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2191

A.8.21.1	Test Purpose and Environment	2191

A.8.21.2 Test Requirements	2195

A.8.22	E-UTRAN Discovery Signal Measurements	2195

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2195

A.8.22.1.1	Test Purpose and Environment	2195

A.8.22.1.2	Test Requirements	2198

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2198

A.8.22.2.1	Test Purpose and Environment	2198

A.8.22.2.2	Test Requirements	2201

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2201

A.8.22.3.1	Test Purpose and Environment	2201

A.8.22.3.2	Test Requirements	2204

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2204

A.8.22.4.1	Test Purpose and Environment	2204

A.8.22.4.2	Test Requirements	2207

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2207

A.8.22.5.1	Test Purpose and Environment	2207

A.8.22.5.2	Test Requirements	2210

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2210

A.8.22.6.1	Test Purpose and Environment	2210

A.8.22.6.2	Test Requirements	2213

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2213

A.8.22.7.1	Test Purpose and Environment	2213

A.8.22.7.2	Test Requirements	2216

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2216

A.8.22.8.1	Test Purpose and Environment	2216

A.8.22.8.2	Test Requirements	2219

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2219

A.8.22.9.1	Test Purpose and Environment	2219

A.8.22.9.2	Test Requirements	2221

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2222

A.8.22.10.1	Test Purpose and Environment	2222

A.8.22.10.2	Test Requirements	2224

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2225

A.8.22.11.1	Test Purpose and Environment	2225

A.8.22.11.2	Test Requirements	2227

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2227

A.8.22.12.1	Test Purpose and Environment	2227

A.8.22.12.2	Test Requirements	2230

A.8.23	E-UTRAN Dual Connectivity Measurements	2230

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2230

A.8.23.1.1	Test Purpose and Environment	2230

A.8.23.1.2	Test Requirements	2233

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2233

A.8.23.2.1	Test Purpose and Environment	2233

A.8.23.2.2	Test Requirements	2236

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2236

A.8.23.3.1	Test Purpose and Environment	2236

A.8.23.3.2	Test Requirements	2239

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2239

A.8.23.4.1	Test Purpose and Environment	2239

A.8.23.4.2	Test Requirements	2242

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2242

A.8.23.5.1	Test Purpose and Environment	2242

A.8.23.5.2	Test Requirements	2245

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2245

A.8.23.6.1	Test Purpose and Environment	2245

A.8.23.6.2	Test Requirements	2248

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2248

A.8.23.7.1	Test Purpose and Environment	2248

A.8.23.7.2	Test Requirements	2250

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2251

A.8.23.8.1	Test Purpose and Environment	2251

A.8.23.8.2	Test Requirements	2253

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2254

A.8.23.9.1	Test Purpose and Environment	2254

A.8.23.9.2	Test Requirements	2257

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2257

A.8.23.10.1	Test Purpose and Environment	2257

A.8.23.10.2	Test Requirements	2260

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2260

A.8.23.11.1	Test Purpose and Environment	2260

A.8.23.11.2	Test Requirements	2263

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2263

A.8.23.12.1	Test Purpose and Environment	2263

A.8.23.12.2	Test Requirements	2266

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2266

A.8.23.13.1	Test Purpose and Environment	2266

A.8.23.13.2	Test Requirements	2269

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2269

A.8.23.14.1	Test Purpose and Environment	2269

A.8.23.14.2	Test Requirements	2272

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2272

A.8.23.15.1	Test Purpose and Environment	2272

A.8.23.15.2	Test Requirements	2275

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2275

A.8.23.16.1	Test Purpose and Environment	2275

A.8.23.16.2	Test Requirements	2276

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2277

A.8.23.17.1	Test Purpose and Environment	2277

A.8.23.17.2	Test Requirements	2279

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2279

A.8.23.18.1	Test Purpose and Environment	2279

A.8.23.18.2	Test Requirements	2280

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2281

A.8.23.19.1	Test Purpose and Environment	2281

A.8.23.19.2	Test Requirements	2282

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2283

A.8.23.20.1	Test Purpose and Environment	2283

A.8.23.20.2	Test Requirements	2284

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2285

A.8.23.21.1	Test Purpose and Environment	2285

A.8.23.21.2	Test Requirements	2286

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2287

A.8.23.22.1	Test Purpose and Environment	2287

A.8.23.22.2	Test Requirements	2288

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2289

A.8.23.23.1	Test Purpose and Environment	2289

A.8.23.23.2	Test Requirements	2290

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2291

A.8.23.24.1	Test Purpose and Environment	2291

A.8.23.24.2	Test Requirements	2293

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2294

A.8.23.25.1	Test Purpose and Environment	2294

A.8.23.25.2	Test Requirements	2296

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2297

A.8.23.26.1	Test Purpose and Environment	2297

A.8.23.26.2	Test Requirements	2299

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2300

A.8.23.27.1	Test Purpose and Environment	2300

A.8.23.27.2	Test Requirements	2303

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2303

A.8.23.28.1	Test Purpose and Environment	2303

A.8.23.28.2	Test Requirements	2306

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2306

A.8.23.29.1	Test Purpose and Environment	2306

A.8.23.29.2	Test Requirements	2309

A.8.24	Proximity-based Services	2309

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2309

A.8.24.1.1	Test Purpose and Environment	2309

A.8.24.1.2	Test Requirements	2310

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2311

A.8.24.2.1	Test Purpose and Environment	2311

A.8.24.2.2	Test Requirements	2312

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2312

A.8.24.3.1	Test Purpose and Environment	2313

A.8.24.3.2	Test Requirements	2314

A.8.25	E-UTRAN-WLAN Measurements	2315

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2315

A.8.25.1.1	Test Purpose and Environment	2315

A.8.25.1.2	Test Requirements	2317

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2317

A.8.25.2.1	Test Purpose and Environment	2317

A.8.25.2.2	Test Requirements	2319

A.8.26 Frame Structure 3 (FS3)	2319

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2319

A.8.26.1.1	Test Purpose and Environment	2319

A.8.26.1.2	Test Requirements	2321

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2322

A.8.26.2.1	Test Purpose and Environment	2322

A.8.26.2.2	Test Requirements	2324

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2325

A.8.26.3.1	Test Purpose and Environment	2325

A.8.26.3.2	Test Requirements	2328

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2328

A.8.26.3A.1	Test Purpose and Environment	2328

A.8.26.3A.2	Test Requirements	2331

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2331

A.8.26.4.1	Test Purpose and Environment	2331

A.8.26.4.2	Test Requirements	2334

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2334

A.8.26.4A.1	Test Purpose and Environment	2334

A.8.26.4A.2	Test Requirements	2337

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2337

A.8.26.5.1	Test Purpose and Environment	2337

A.8.26.5.2	Test Requirements	2340

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2340

A.8.26.5A.1	Test Purpose and Environment	2340

A.8.26.5A.2	Test Requirements	2343

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2343

A.8.26.6.1	Test Purpose and Environment	2343

A.8.26.6.2	Test Requirements	2346

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2346

A.8.26.6A.1	Test Purpose and Environment	2346

A.8.26.6A.2	Test Requirements	2349

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2349

A.8.26.7.1	Test Purpose and Environment	2349

A.8.26.7.2	Test Requirements	2352

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2352

A.8.26.8.1	Test Purpose and Environment	2352

A.8.26.8.2	Test Requirements	2355

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2355

A.8.26.9.1	Test Purpose and Environment	2355

A.8.26.9.2	Test Requirements	2358

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2358

A.8.26.10.1	Test Purpose and Environment	2358

A.8.26.10.2	Test Requirements	2360

A.9	Measurement Performance Requirements	2361

A.9.1	RSRP	2361

A.9.1.1	FDD Intra frequency case	2361

A.9.1.1.1	Test Purpose and Environment	2361

A.9.1.1.2	Test parameters	2361

A.9.1.1.3	Test Requirements	2363

A.9.1.2	TDD Intra frequency case	2363

A.9.1.2.1	Test Purpose and Environment	2363

A.9.1.2.2	Test parameters	2363

A.9.1.2.3	Test Requirements	2364

A.9.1.3	FDD—FDD Inter frequency case	2365

A.9.1.3.1	Test Purpose and Environment	2365

A.9.1.3.2	Test parameters	2365

A.9.1.3.3	Test Requirements	2367

A.9.1.4	TDD—TDD Inter frequency case	2367

A.9.1.4.1	Test Purpose and Environment	2367

A.9.1.4.2	Test parameters	2367

A.9.1.4.3	Test Requirements	2369

A.9.1.5	FDD—TDD Inter frequency case	2370

A.9.1.5.1	Test Purpose and Environment	2370

A.9.1.5.2	Test parameters	2370

A.9.1.5.3	Test Requirements	2371

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2372

A.9.1.6.1	Test Purpose and Environment	2372

A.9.1.6.2	Test parameters	2372

A.9.1.6.3	Test Requirements	2374

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2374

A.9.1.7.1	Test Purpose and Environment	2374

A.9.1.7.2	Test parameters	2374

A.9.1.7.3	Test Requirements	2376

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2376

A.9.1.8.1	Test Purpose and Environment	2376

A.9.1.8.2	Test parameters	2376

A.9.1.8.3	Test Requirements	2379

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2379

A.9.1.9.1	Test Purpose and Environment	2379

A.9.1.9.2	Test parameters	2379

A.9.1.9.3	Test Requirements	2382

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2382

A.9.1.10.1	Test Purpose and Environment	2382

A.9.1.10.2	Test parameters	2382

A.9.1.10.3	Test Requirements	2384

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2384

A.9.1.11.1	Test Purpose and Environment	2384

A.9.1.11.2	Test parameters	2384

A.9.1.11.3	Test Requirements	2387

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2387

A.9.1.12.1	Test Purpose and Environment	2387

A.9.1.12.2	Test parameters	2387

A.9.1.12.3	Test Requirements	2388

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2388

A.9.1.13.1	Test Purpose and Environment	2388

A.9.1.13.2	Test parameters	2388

A.9.1.13.3	Test Requirements	2389

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2389

A.9.1.14.1	Test Purpose and Environment	2389

A.9.1.14.2	Test parameters	2389

A.9.1.14.3	Test Requirements	2392

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2392

A.9.1.15.1	Test Purpose and Environment	2392

A.9.1.15.2	Test parameters	2392

A.9.1.15.3	Test Requirements	2394

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2395

A.9.1.16.1	Test Purpose and Environment	2395

A.9.1.16.2	Test parameters	2395

A.9.1.16.3	Test Requirements	2396

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2396

A.9.1.17.1	Test Purpose and Environment	2396

A.9.1.17.2	Test parameters	2397

A.9.1.17.3	Test Requirements	2398

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2398

A.9.1.18.1	Test Purpose and Environment	2398

A.9.1.18.2	Test parameters	2398

A.9.1.18.3	Test Requirements	2399

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2399

A.9.1.19.1	Test Purpose and Environment	2400

A.9.1.19.2	Test parameters	2400

A.9.1.19.3	Test Requirements	2400

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2400

A.9.1.20.1	Test Purpose and Environment	2400

A.9.1.20.2	Test parameters	2400

A.9.1.20.3	Test Requirements	2401

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2402

A.9.1.21.1	Test Purpose and Environment	2402

A.9.1.21.2	Test parameters	2402

A.9.1.21.3	Test Requirements	2402

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2402

A.9.1.22.1	Test Purpose and Environment	2402

A.9.1.22.2	Test parameters	2403

A.9.1.22.3	Test Requirements	2406

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2406

A.9.1.23.1	Test Purpose and Environment	2406

A.9.1.23.2	Test parameters	2406

A.9.1.23.3	Test Requirements	2408

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2409

A.9.1.24.1	Test Purpose and Environment	2409

A.9.1.24.2	Test parameters	2409

A.9.1.24.3	Test Requirements	2409

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2410

A.9.1.25.1	Test Purpose and Environment	2410

A.9.1.25.2	Test parameters	2410

A.9.1.25.3	Test Requirements	2412

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2412

A.9.1.26.1	Test Purpose and Environment	2412

A.9.1.26.2	Test parameters	2412

A.9.1.26.3	Test Requirements	2414

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2414

A.9.1.27.1	Test Purpose and Environment	2414

A.9.1.27.2	Test parameters	2414

A.9.1.27.3	Test Requirements	2416

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2416

A.9.1.28.1	Test Purpose and Environment	2416

A.9.1.28.2	Test parameters	2416

A.9.1.28.3	Test Requirements	2418

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2418

A.9.1.29.1	Test Purpose and Environment	2418

A.9.1.29.2	Test parameters	2418

A.9.1.29.3	Test Requirements	2420

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2420

A.9.1.30.1	Test Purpose and Environment	2420

A.9.1.30.2	Test parameters	2420

A.9.1.30.3	Test Requirements	2422

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2422

A.9.1.31.1	Test Purpose and Environment	2422

A.9.1.31.2	Test parameters	2422

A.9.1.31.3	Test Requirements	2424

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2424

A.9.1.32.1	Test Purpose and Environment	2424

A.9.1.32.2	Test parameters	2425

A.9.1.32.3	Test Requirements	2427

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2427

A.9.1.33.1	Test Purpose and Environment	2427

A.9.1.33.2	Test parameters	2427

A.9.1.33.3	Test Requirements	2429

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2429

A.9.1.34.1	Test Purpose and Environment	2429

A.9.1.34.2	Test parameters	2429

A.9.1.34.3	Test Requirements	2431

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2431

A.9.1.35.1	Test Purpose and Environment	2431

A.9.1.35.2	Test parameters	2431

A.9.1.35.3	Test Requirements	2433

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2434

A.9.1.36.1	Test Purpose and Environment	2434

A.9.1.36.2	Test parameters	2434

A.9.1.36.3	Test Requirements	2436

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2436

A.9.1.37.1	Test Purpose and Environment	2436

A.9.1.37.2	Test parameters	2436

A.9.1.37.3	Test Requirements	2438

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2439

A.9.1.38.1	Test Purpose and Environment	2439

A.9.1.38.2	Test parameters	2439

A.9.1.38.3	Test Requirements	2441

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2442

A.9.1.39.1	Test Purpose and Environment	2442

A.9.1.39.2	Test parameters	2442

A.9.1.39.3	Test Requirements	2446

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2446

A.9.1.40.1	Test Purpose and Environment	2446

A.9.1.40.2	Test parameters	2446

A.9.1.40.3	Test Requirements	2451

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2451

A.9.1.41.1	Test Purpose and Environment	2451

A.9.1.41.2	Test parameters	2451

A.9.1.41.3	Test Requirements	2453

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2453

A.9.1.42.1	Test Purpose and Environment	2453

A.9.1.42.2	Test parameters	2453

A.9.1.42.3	Test Requirements	2455

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2455

A.9.1.43.1	Test Purpose and Environment	2455

A.9.1.43.2	Test parameters	2455

A.9.1.43.3	Test Requirements	2456

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2457

A.9.1.44.1	Test Purpose and Environment	2457

A.9.1.44.2	Test parameters	2457

A.9.1.44.3	Test Requirements	2461

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2461

A.9.1.45.1	Test Purpose and Environment	2461

A.9.1.45.2	Test parameters	2461

A.9.1.45.3	Test Requirements	2465

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2465

A.9.1.46.1	Test Purpose and Environment	2465

A.9.1.46.2	Test parameters	2465

A.9.1.46.3	Test Requirements	2471

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2471

A.9.1.47.1	Test Purpose and Environment	2471

A.9.1.47.2	Test parameters	2472

A.9.1.47.3	Test Requirements	2478

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2478

A.9.1.48.1	Test Purpose and Environment	2478

A.9.1.48.2	Test parameters	2479

A.9.1.48.3	Test Requirements	2486

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2487

A.9.1.49.1	Test Purpose and Environment	2487

A.9.1.49.2	Test parameters	2487

A.9.1.49.3	Test Requirements	2494

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2495

A.9.1.50.1	Test Purpose and Environment	2495

A.9.1.50.2	Test parameters	2495

A.9.1.50.3	Test Requirements	2500

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2500

A.9.1.51.1	Test Purpose and Environment	2500

A.9.1.51.2	Test parameters	2500

A.9.1.51.3	Test Requirements	2504

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2505

A.9.1.52.1	Test Purpose and Environment	2505

A.9.1.52.2	Test parameters	2505

A.9.1.52.3	Test Requirements	2507

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2507

A.9.1.52A.1	Test Purpose and Environment	2507

A.9.1.52A.2	Test parameters	2507

A.9.1.52A.3	Test Requirements	2508

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2508

A.9.1.53.1	Test Purpose and Environment	2508

A.9.1.53.2	Test parameters	2509

A.9.1.53.3	Test Requirements	2511

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2511

A.9.1.53A.1	Test Purpose and Environment	2511

A.9.1.53A.2	Test parameters	2511

A.9.1.53A.3	Test Requirements	2512

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.54.1	Test Purpose and Environment	2512

A.9.1.54.2	Test parameters	2513

A.9.1.54.3	Test Requirements	2515

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2515

A.9.1.55.1	Test Purpose and Environment	2515

A.9.1.55.2	Test parameters	2515

A.9.1.55.3	Test Requirements	2518

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2518

A.9.1.56.1	Test Purpose and Environment	2518

A.9.1.56.2	Test parameters	2518

A.9.1.56.3	Test Requirements	2520

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2521

A.9.1.57.1	Test Purpose and Environment	2521

A.9.1.57.2	Test parameters	2521

A.9.1.57.3	Test Requirements	2523

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2523

A.9.1.57A.1	Test Purpose and Environment	2523

A.9.1.57A.2	Test parameters	2523

A.9.1.57A.3	Test Requirements	2524

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2524

A.9.1.58.1	Test Purpose and Environment	2524

A.9.1.58.2	Test parameters	2525

A.9.1.58.3	Test Requirements	2527

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2527

A.9.1.58A.1	Test Purpose and Environment	2527

A.9.1.58A.2	Test parameters	2527

A.9.1.58A.3	Test Requirements	2528

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2528

A.9.1.59.1	Test Purpose and Environment	2528

A.9.1.59.2	Test parameters	2529

A.9.1.59.3	Test Requirements	2531

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2531

A.9.1.60.1	Test Purpose and Environment	2531

A.9.1.60.2	Test parameters	2531

A.9.1.60.3	Test Requirements	2533

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2534

A.9.1.61.1	Test Purpose and Environment	2534

A.9.1.61.2	Test parameters	2534

A.9.1.61.3	Test Requirements	2536

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2536

A.9.1.62.1	Test Purpose and Environment	2536

A.9.1.62.2	Test parameters	2536

A.9.1.62.3	Test Requirements	2538

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2538

A.9.1.63.1	Test Purpose and Environment	2538

A.9.1.63.2	Test parameters	2538

A.9.1.63.3	Test Requirements	2540

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2540

A.9.1.64.1	Test Purpose and Environment	2540

A.9.1.64.2	Test parameters	2540

A.9.1.64.3	Test Requirements	2541

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2542

A.9.1.65.1	Test Purpose and Environment	2542

A.9.1.65.2	Test parameters	2542

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2546

A.9.1.66.1	Test Purpose and Environment	2546

A.9.1.66.2	Test parameters	2546

A.9.1.66.3	Test Requirements	2550

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2550

A.9.1.67.1	Test Purpose and Environment	2550

A.9.1.67.2	Test parameters	2550

A.9.1.67.3	Test Requirements	2551

A.9.2	RSRQ	2552

A.9.2.1	FDD Intra frequency case	2552

A.9.2.1.1	Test Purpose and Environment	2552

A.9.2.1.2	Test parameters	2552

A.9.2.1.3	Test Requirements	2554

A.9.2.2	TDD Intra frequency case	2554

A.9.2.2.1	Test Purpose and Environment	2554

A.9.2.2.2	Test parameters	2554

A.9.2.2.3	Test Requirements	2556

A.9.2.3	FDD—FDD Inter frequency case	2556

A.9.2.3.1	Test Purpose and Environment	2556

A.9.2.3.2	Test parameters	2556

A.9.2.3.3	Test Requirements	2558

A.9.2.4	TDD—TDD Inter frequency case	2558

A.9.2.4.1	Test Purpose and Environment	2558

A.9.2.4.2	Test parameters	2558

A.9.2.4.3	Test Requirements	2560

A.9.2.4A	FDD—TDD Inter frequency case	2561

A.9.2.4A.1	Test Purpose and Environment	2561

A.9.2.4A.2	Test parameters	2561

A.9.2.4A.3	Test Requirements	2563

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2563

A.9.2.5.1	Test Purpose and Environment	2563

A.9.2.5.2	Test parameters	2564

A.9.2.5.3	Test Requirements	2566

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2566

A.9.2.6.1	Test Purpose and Environment	2566

A.9.2.6.2	Test parameters	2566

A.9.2.6.3	Test Requirements	2568

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2568

A.9.2.7.1	Test Purpose and Environment	2568

A.9.2.7.2	Test parameters	2568

A.9.2.7.3	Test Requirements	2571

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2571

A.9.2.8.1	Test Purpose and Environment	2571

A.9.2.8.2	Test parameters	2571

A.9.2.8.3	Test Requirements	2574

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2574

A.9.2.9.1	Test Purpose and Environment	2574

A.9.2.9.2	Test parameters	2574

A.9.2.9.3	Test Requirements	2577

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2577

A.9.2.10.1	Test Purpose and Environment	2577

A.9.2.10.2	Test parameters	2577

A.9.2.10.3	Test Requirements	2580

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2580

A.9.2.11.1	Test Purpose and Environment	2580

A.9.2.11.2	Test parameters	2580

A.9.2.11.3	Test Requirements	2581

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2581

A.9.2.12.1	Test Purpose and Environment	2581

A.9.2.12.2	Test parameters	2581

A.9.2.12.3	Test Requirements	2582

A.9.2.13	Void	2583

A.9.2.13.1	Void	2583

A.9.2.13.2	Void	2583

A.9.2.13.3	Void	2583

A.9.2.14	Void	2583

A.9.2.14.1	Void	2583

A.9.2.14.2	Void	2583

A.9.2.14.3	Void	2583

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2583

A.9.2.15.1	Test Purpose and Environment	2583

A.9.2.15.2	Test parameters	2583

A.9.2.15.3	Test Requirements	2586

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2586

A.9.2.16.1	Test Purpose and Environment	2586

A.9.2.16.2	Test parameters	2586

A.9.2.16.3	Test Requirements	2589

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2589

A.9.2.17.1	Test Purpose and Environment	2589

A.9.2.17.2	Test parameters	2589

A.9.2.17.3	Test Requirements	2591

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2591

A.9.2.18.1	Test Purpose and Environment	2591

A.9.2.18.2	Test parameters	2591

A.9.2.18.3	Test Requirements	2593

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2593

A.9.2.19.1	Test Purpose and Environment	2593

A.9.2.19.2	Test parameters	2593

A.9.2.19.3	Test Requirements	2595

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2595

A.9.2.20.1	Test Purpose and Environment	2595

A.9.2.20.2	Test parameters	2595

A.9.2.20.3	Test Requirements	2597

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2597

A.9.2.21.1	Test Purpose and Environment	2597

A.9.2.21.2	Test parameters	2597

A.9.2.21.3	Test Requirements	2599

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2599

A.9.2.22.1	Test Purpose and Environment	2599

A.9.2.22.2	Test parameters	2599

A.9.2.22.3	Test Requirements	2600

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2600

A.9.2.23.1	Test Purpose and Environment	2600

A.9.2.23.2	Test parameters	2600

A.9.2.23.3	Test Requirements	2602

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2602

A.9.2.24.1	Test Purpose and Environment	2602

A.9.2.24.2	Test parameters	2602

A.9.2.24.3	Test Requirements	2602

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2603

A.9.2.25.1	Test Purpose and Environment	2603

A.9.2.25.2	Test parameters	2603

A.9.2.25.3	Test Requirements	2606

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2606

A.9.2.26.1	Test Purpose and Environment	2606

A.9.2.26.2	Test parameters	2606

A.9.2.26.3	Test Requirements	2609

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2609

A.9.2.27.1	Test Purpose and Environment	2609

A.9.2.27.2	Test parameters	2609

A.9.2.27.3	Test Requirements	2610

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2610

A.9.2.28.1	Test Purpose and Environment	2610

A.9.2.28.2	Test parameters	2610

A.9.2.28.3	Test Requirements	2612

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2612

A.9.2.29.1	Test Purpose and Environment	2612

A.9.2.29.2	Test parameters	2612

A.9.2.29.3	Test Requirements	2614

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2614

A.9.2.30.1	Test Purpose and Environment	2614

A.9.2.30.2	Test parameters	2614

A.9.2.30.3	Test Requirements	2616

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2616

A.9.2.31.1	Test Purpose and Environment	2616

A.9.2.31.2	Test parameters	2616

A.9.2.31.3	Test Requirements	2618

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2618

A.9.2.32.1	Test Purpose and Environment	2618

A.9.2.32.2	Test parameters	2618

A.9.2.32.3	Test Requirements	2620

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2620

A.9.2.33.1	Test Purpose and Environment	2620

A.9.2.33.2	Test parameters	2620

A.9.2.33.3	Test Requirements	2622

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2622

A.9.2.34.1	Test Purpose and Environment	2622

A.9.2.34.2	Test parameters	2622

A.9.2.34.3	Test Requirements	2624

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2624

A.9.2.35.1	Test Purpose and Environment	2624

A.9.2.35.2	Test parameters	2624

A.9.2.35.3	Test Requirements	2626

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2626

A.9.2.36.1	Test Purpose and Environment	2626

A.9.2.36.2	Test parameters	2626

A.9.2.36.3	Test Requirements	2627

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2628

A.9.2.37.1	Test Purpose and Environment	2628

A.9.2.37.2	Test parameters	2628

A.9.2.37.3	Test Requirements	2629

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2630

A.9.2.38.1	Test Purpose and Environment	2630

A.9.2.38.2	Test parameters	2630

A.9.2.38.3	Test Requirements	2633

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2633

A.9.2.39.1	Test Purpose and Environment	2633

A.9.2.39.2	Test parameters	2633

A.9.2.39.3	Test Requirements	2636

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2636

A.9.2.40.1	Test Purpose and Environment	2636

A.9.2.40.2	Test parameters	2636

A.9.2.40.3	Test Requirements	2638

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2638

A.9.2.41.1	Test Purpose and Environment	2638

A.9.2.41.2	Test parameters	2639

A.9.2.41.3	Test Requirements	2641

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2641

A.9.2.42.1	Test Purpose and Environment	2641

A.9.2.42.2	Test parameters	2641

A.9.2.42.3	Test Requirements	2643

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2643

A.9.2.43.1	Test Purpose and Environment	2643

A.9.2.43.2	Test parameters	2643

A.9.2.43.3	Test Requirements	2645

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2645

A.9.2.44.1	Test Purpose and Environment	2645

A.9.2.44.2	Test parameters	2645

A.9.2.44.3	Test Requirements	2647

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2647

A.9.2.45.1	Test Purpose and Environment	2647

A.9.2.45.2	Test parameters	2647

A.9.2.45.3	Test Requirements	2650

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2650

A.9.2.46.1	Test Purpose and Environment	2651

A.9.2.46.2	Test parameters	2651

A.9.2.46.3	Test Requirements	2654

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2654

A.9.2.47.1	Test Purpose and Environment	2655

A.9.2.47.2	Test parameters	2655

A.9.2.47.3	Test Requirements	2658

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2658

A.9.2.48.1	Test Purpose and Environment	2658

A.9.2.48.2	Test parameters	2658

A.9.2.48.3	Test Requirements	2661

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2661

A.9.2.49.1	Test Purpose and Environment	2661

A.9.2.49.2	Test parameters	2661

A.9.2.49.3	Test Requirements	2665

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2666

A.9.2.50.1	Test Purpose and Environment	2666

A.9.2.50.2	Test parameters	2666

A.9.2.50.3	Test Requirements	2670

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2670

A.9.2.51.1	Test Purpose and Environment	2670

A.9.2.51.2	Test parameters	2671

A.9.2.51.3	Test Requirements	2674

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2674

A.9.2.52.1	Test Purpose and Environment	2674

A.9.2.52.2	Test parameters	2674

A.9.2.52.3	Test Requirements	2676

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2677

A.9.2.53.1	Test Purpose and Environment	2677

A.9.2.53.2	Test parameters	2677

A.9.2.53.3	Test Requirements	2681

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2682

A.9.2.54.1	Test Purpose and Environment	2682

A.9.2.54.2	Test parameters	2682

A.9.2.54.3	Test Requirements	2686

A.9.3	UTRAN FDD CPICH RSCP	2686

A.9.3.1	E-UTRAN FDD	2686

A.9.3.1.1	Test Purpose and Environment	2686

A.9.3.1.2	Parameters	2686

A.9.3.1.3	Test Requirements	2688

A.9.3.2 E-UTRAN TDD	2688

A.9.3.2.1	Test Purpose and Environment	2688

A.9.3.2.2	Parameters	2688

A.9.3.2.3	Test Requirements	2691

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2691

A.9.3.3.1	Test Purpose and Environment	2691

A.9.3.3.2	Parameters	2691

A.9.3.3.3	Test Requirements	2692

A.9.4	UTRAN FDD CPICH Ec/No	2693

A.9.4.1	E-UTRAN FDD	2693

A.9.4.1.1	Test Purpose and Environment	2693

A.9.4.1.2	Parameters	2693

A.9.4.1.3	Test Requirements	2695

A.9.4.2	E-UTRAN TDD	2696

A.9.4.2.1	Test Purpose and Environment	2696

A.9.4.2.2	Parameters	2696

A.9.4.2.3	Test Requirements	2698

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2699

A.9.4.3.1	Test Purpose and Environment	2699

A.9.4.3.2	Parameters	2699

A.9.4.3.3	Test Requirements	2700

A.9.5	UTRAN TDD measurement	2700

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2700

A.9.5.1.1	Test Purpose and Environment	2700

A.9.5.1.2	Test parameters	2700

A.9.5.1.3	Test Requirements	2702

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2702

A.9.5.2.1	Test Purpose and Environment	2702

A.9.5.2.2	Test parameters	2702

A.9.5.2.3	Test Requirements	2704

A.9.6	GSM Carrier RSSI	2705

A.9.6.1	E-UTRAN FDD	2705

A.9.6.1.1	Test Purpose and Environment	2705

A.9.6.1.2	Test Requirements	2706

A.9.6.2	E-UTRAN TDD	2706

A.9.6.2.1	Test Purpose and Environment	2706

A.9.6.2.2	Test Requirements	2709

A.9.7	UE Rx – Tx Time Difference	2709

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2709

A.9.7.1.1	Test Purpose and Environment	2709

A.9.7.1.2	Test parameters	2709

A.9.7.1.3	Test Requirements	2711

A.9.7.2	E-UTRA TDD	2711

A.9.7.2.1	Test Purpose and Environment	2711

A.9.7.2.2	Test parameters	2711

A.9.7.2.3	Test Requirements	2713

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2713

A.9.7.3.1	Test Purpose and Environment	2713

A.9.7.3.2	Test parameters	2713

A.9.7.3.3	Test Requirements	2716

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2716

A.9.7.4.1	Test Purpose and Environment	2716

A.9.7.4.2	Test Parameters	2716

A.9.7.4.3	Test Requirements	2719

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2719

A.9.7.5.1	Test Purpose and Environment	2719

A.9.7.5.2	Test parameters	2719

A.9.7.5.3	Test Requirements	2722

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2722

A.9.7.6.1	Test Purpose and Environment	2722

A.9.7.6.2	Test Parameters	2722

A.9.7.6.3	Test Requirements	2725

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2725

A.9.7.7.1	Test Purpose and Environment	2725

A.9.7.7.2	Test parameters	2725

A.9.7.7.3	Test Requirements	2726

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2727

A.9.7.8.1	Test Purpose and Environment	2727

A.9.7.8.2	Test parameters	2727

A.9.7.8.3	Test Requirements	2728

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2728

A.9.7.9.1	Test Purpose and Environment	2728

A.9.7.9.2	Test parameters	2728

A.9.7.9.3	Test Requirements	2729

A.9.8	RSTD	2730

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2730

A.9.8.1.1	Test Purpose and Environment	2730

A.9.8.1.2	Test Requirements	2732

A.9.8.1.2A	Test Requirements for UE Category 1bis	2732

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2733

A.9.8.2.1	Test Purpose and Environment	2733

A.9.8.2.2	Test Requirements	2735

A.9.8.2.2A	Test Requirements for UE Category 1bis	2735

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2736

A.9.8.3.1	Test Purpose and Environment	2736

A.9.8.3.2	Test Requirements	2738

A.9.8.3.2A	Test Requirements for UE Category 1bis	2738

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2739

A.9.8.4.1	Test Purpose and Environment	2739

A.9.8.4.2	Test Requirements	2741

A.9.8.4.2A	Test Requirements for UE Category 1bis	2741

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2742

A.9.8.5.1	Test Purpose and Environment	2742

A.9.8.5.2	Test Requirements	2744

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2744

A.9.8.6.1	Test Purpose and Environment	2744

A.9.8.6.2	Test Requirements	2747

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2747

A.9.8.7.1	Test Purpose and Environment	2747

A.9.8.7.2	Test Requirements	2748

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2748

A.9.8.8.1	Test Purpose and Environment	2748

A.9.8.8.2	Test Requirements	2749

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2749

A.9.8.9.1	Test Purpose and Environment	2749

A.9.8.9.2	Test Requirements	2750

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2750

A.9.8.10.1	Test Purpose and Environment	2750

A.9.8.10.2	Test Requirements	2750

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2750

A.9.8.11.1	Test Purpose and Environment	2750

A.9.8.11.2	Test Requirements	2751

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2751

A.9.8.12.1	Test Purpose and Environment	2751

A.9.8.12.2	Test Requirements	2752

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2752

A.9.8.13.1	Test Purpose and Environment	2752

A.9.8.13.2	Test Requirements	2753

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2753

A.9.8.14.1	Test Purpose and Environment	2753

A.9.8.14.2	Test Requirements	2758

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2758

A.9.8.15.1	Test Purpose and Environment	2758

A.9.8.15.2	Test Requirements	2762

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2762

A.9.8.16.1	Test Purpose and Environment	2762

A.9.8.16.2	Test Requirements	2765

A.9.8.17 HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2765

A.9.8.17.1	Test Purpose and Environment	2765

A.9.8.17.2	Test Requirements	2768

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2768

A.9.8.18.1	Test Purpose and Environment	2768

A.9.8.18.2	Test Requirements	2771

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2771

A.9.8.19.1	Test Purpose and Environment	2771

A.9.8.19.2	Test Requirements	2774

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2774

A.9.8.20.1	Test Purpose and Environment	2774

A.9.8.20.2	Test Requirements	2776

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2776

A.9.8.21.1	Test Purpose and Environment	2776

A.9.8.21.2	Test Requirements	2779

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2779

A.9.8.22.1	Test Purpose and Environment	2779

A.9.8.22.2	Test Requirements	2782

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2783

A.9.8.23.1	Test Purpose and Environment	2783

A.9.8.23.2	Test Requirements	2785

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2785

A.9.8.24.1	Test Purpose and Environment	2785

A.9.8.24.2	Test Requirements	2788

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2788

A.9.8.25.1	Test Purpose and Environment	2788

A.9.8.25.2	Test Requirements	2791

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2792

A.9.8.26.1	Test Purpose and Environment	2792

A.9.8.26.2	Test Requirements	2794

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2794

A.9.8.27.1	Test Purpose and Environment	2794

A.9.8.27.2	Test Requirements	2797

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2797

A.9.8.28.1	Test Purpose and Environment	2797

A.9.8.28.2	Test Requirements	2800

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2800

A.9.8.29.1	Test Purpose and Environment	2800

A.9.8.29.2	Test Requirements	2803

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2804

A.9.8.30.1	Test Purpose and Environment	2804

A.9.8.30.2	Test Requirements	2806

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2807

A.9.8.31.1	Test Purpose and Environment	2807

A.9.8.31.2	Test Requirements	2810

A.9.9	RSRP and RSRQ on the serving cell	2810

A.9.9.1	FDD Intra frequency serving cell case	2810

A.9.9.1.1	Test Purpose and Environment	2810

A.9.9.1.2	Test parameters	2810

A.9.9.1.3	Test Requirements	2812

A.9.9.2	TDD Intra frequency serving cell case	2812

A.9.9.2.1	Test Purpose and Environment	2812

A.9.9.2.2	Test parameters	2812

A.9.9.2.3	Test Requirements	2814

A.9.10	SSTD	2814

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2814

A.9.10.1.1	Test Purpose and Environment	2814

A.9.10.1.2	Test parameters	2814

A.9.10.1.3	Test Requirements	2816

A.9.10.3	FDD—FDD Inter frequency case	2816

A.9.10.3.1	Test Purpose and Environment	2816

A.9.10.3.2	Test parameters	2816

A.9.10.3.3	Test Requirements	2820

A.9.10.4	TDD—TDD Inter frequency case	2820

A.9.10.4.1	Test Purpose and Environment	2820

A.9.10.4.2	Test parameters	2820

A.9.10.4.3	Test Requirements	2825

A.9.11 RSSI	2825

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2825

A.9.11.1.1	Test Purpose and Environment	2825

A.9.11.1.2	Test parameters	2825

A.9.11.1.3	Test Requirements	2827

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2827

A.9.11.2.1	Test Purpose and Environment	2827

A.9.11.2.2	Test parameters	2827

A.9.12	Channel occupancy	2829

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2829

A.9.12.1.1	Test Purpose and Environment	2829

A.9.12.1.2	Test parameters	2829

A.9.12.1.3	Test Requirements	2831

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2831

A.9.12.2.1	Test Purpose and Environment	2831

A.9.12.2.2	Test parameters	2831

A.9.12.2.3	Test Requirements	2833

A.9.13	RS-SINR	2833

A.9.13.1	FDD Intra-Frequency Case	2833

A.9.13.1.1	Test Purpose and Environment	2833

A.9.13.1.2	Test parameters	2833

A.9.13.1.3	Test Requirements	2835

A.9.13.2	TDD Intra-Frequency Case	2835

A.9.13.2.1	Test Purpose and Environment	2835

A.9.13.2.2	Test parameters	2835

A.9.13.2.3	Test Requirements	2838

A.9.13.5	FDD—TDD Inter frequency case	2838

A.9.13.5.1	Test Purpose and Environment	2838

A.9.13.5.2	Test parameters	2838

A.9.13.5.3	Test Requirements	2842

A.9.13.6	TDD—FDD Inter frequency case	2842

A.9.13.6.1	Test Purpose and Environment	2842

A.9.13.6.2	Test parameters	2842

A.9.13.6.3	Test Requirements	2845

A.10	Proximity-based Services in Any Cell Selection State	2845

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2845

A.10.1.1	Test Purpose and Environment	2845

A.10.1.2	Test Requirements	2846

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2847

A.10.2.1	Test Purpose and Environment	2847

A.10.2.2	Test Requirements	2848

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2848

A.10.3.1	Test Purpose and Environment	2848

A.10.3.2	Test Requirements	2850

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2850

A.10.4.1	Test Purpose and Environment	2850

A.10.4.2	Test Requirements	2852

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2852

A.11.1	V2V UE Transmission Timing Accuracy Test	2852

A.11.1.1	Test Purpose and Environment	2852

A.11.1.2	Test requirements	2853

A.11.2	Interruptions due to V2V sidelink communication	2853

A.11.2.1	Test Purpose and Environment	2853

A.11.2.2	Test Requirements	2855

A.12	2855

A.12.1	V2X UE Transmission Timing Accuracy Test	2855

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2855

A.12.1.1.1	Test Purpose and Environment	2855

A.12.1.1.2	Test requirements	2856

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	2857

A.12.1.2.1	Test Purpose and Environment	2857

A.12.1.2.2	Test Requirements	2857

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2858

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	2858

A.12.2.1.1	Test Purpose and Environment	2858

A.12.2.1.2	Test Requirements	2859

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	2860

A.12.2.2.1	Test Purpose and Environment	2860

A.12.2.1.2	Test Requirements	2861

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	2861

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	2861

A.12.3.1.1	Test Purpose and Environment	2861

A.12.3.1.2	Test Requirements	2863

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	2864

A.12.3.2.1	Test Purpose and Environment	2864

A.12.3.1.2	Test Requirements	2866

A.12.4	Congestion Control Measurement Test for V2X UE	2866

A.12.4.1	Test Purpose and Environment	2866

A.12.4.2	Test Requirements	2868

A.12.5	Interruptions due to V2X Sidelink Communication	2868

A.12.5.1	Test Purpose and Environment	2868

A.12.5.1	Test Requirements	2870

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	2870

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	2870

A.12.6.1.1	Test Purpose and Environment	2870

A.12.6.1.2	Test Requirements	2872

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	2872

A.12.6.2.1	Test Purpose and Environment	2872

A.12.6.1.2	Test Requirements	2874

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	2875

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	2875

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	2875

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	2875

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE supporting coverage enhancement	2875

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	2877

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	2877

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	2878

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	2878

B.1.7.1 Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	2878

B.1.7.2 Condition for measurements of inter-frequence E-UTRAN cells for cell selection	2879

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE supporting coverage enhancement	2881

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	2881

B.2.1	Conditions for E-UTRAN intra-frequency measurements	2881

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	2882

B.2.3	Conditions for E-UTRAN inter-frequency measurements	2882

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	2883

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	2883

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	2883

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	2884

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	2884

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2885

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	2885

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	2885

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	2885

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	2886

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2886

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2886

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	2887

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	2887

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2887

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2888

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	2888

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	2889

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	2890

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	2890

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	2890

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	2892

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	2892

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	2892

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	2892

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	2893

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	2893

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	2895

B.3	Conditions for measurements performance requirements for UE	2895

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	2895

B.3.2	Void	2895

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	2895

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	2895

B.3.5	Conditions for UE Rx – Tx time difference	2895

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	2896

B.3.7	Conditions for inter-frequency RSTD measurements	2896

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	2896

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	2896

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	2896

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2897

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2897

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	2897

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2897

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	2897

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	2897

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2898

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	2898

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	2898

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2899

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	2899

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	2899

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2899

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	2899

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	2899

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	2899

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	2899

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	2899

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2900

B.3.21.1	Conditions for RSRP measurements	2900

B.3.21.2	Conditions for RSRQ measurements	2900

B.3.21.3	Conditions for CSI-RSRP measurements	2900

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2900

B.3.22.1	Conditions for RSRP measurements	2900

B.3.22.2	Void	2901

B.3.22.3	Conditions for CSI-RSRP measurements	2901

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2901

B.3.23.1	Conditions for RSRP measurements	2901

B.3.23.2	Conditions for RSRQ measurements	2901

B.3.23.3	Conditions for CSI-RSRP measurements	2901

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2902

B.3.24.1	Conditions for RSRP measurements	2902

B.3.24.2	Conditions for RSRQ measurements	2902

B.3.24.3	Conditions for CSI-RSRP measurements	2902

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2902

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2902

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	2902

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	2903

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	2903

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	2903

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	2903

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	2903

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	2904

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	2904

B.4	RRM Requirements Exceptions	2904

B.4.1	General	2904

B.4.2	Receiver sensitivity relaxation for UE supporting CA	2904

B.4.3	Receiver sensitivity relaxation for UE configured with CA	2904

B.4.3.1	Inter-band carrier aggregation	2904

B.4.3.2	Intra-band non-contiguous carrier aggregation	2904

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	2905

B.5	Conditions for Measurement Performance Requirements for ProSe UE	2905

B.5.1	Conditions for S-RSRP Accuracy Requirements	2905

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	2905

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2906

B.5.4	Conditions for SD-RSRP Accuracy Requirements	2906

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	2907

B.6	Conditions for V2X	2907

B.6.1	Test parameters for GNSS signals	2907

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	2907

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	2907

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2908

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	2908

Annex C (informative):	Change history:	2909