3GPP TS 37.145-2 V15.2.0 (2018-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Active Antenna System (AAS) Base Station (BS)

conformance testing;

Part 2: radiated conformance testing

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	13

1	Scope	14

2	References	14

3	Definitions, symbols and abbreviations	16

3.1	Definitions	16

3.2	Symbols	18

3.3	Abbreviations	21

4	General test conditions and declarations	22

4.1	Measurement uncertainties and test requirements	22

4.1.1	General	22

4.1.2	Acceptable uncertainty of Test System	24

4.1.2.1	General	24

4.1.2.2	Measurement of transmitter	25

4.1.2.3	Measurement of receiver	27

4.1.2.4	Measurement of performance requirement	27

4.1.3	Interpretation of measurement results	27

4.2	Conducted and radiated requirement reference points	28

4.3	Base station classes for AAS BS	29

4.4	Regional requirements	29

4.5	Operating bands and band categories	31

4.6	Channel arrangements	31

4.7	Requirements for AAS BS capable of multi-band operation	31

4.8	AAS BS configurations	31

4.8.1	Transmit configurations	31

4.8.2	Receive configurations	33

4.8.3	Power supply options	34

4.8.4	BS with integrated Iuant BS modem	34

4.9	Capability sets	34

4.10	Manufacturer declarations	34

4.11	Test signal configurations for testing	40

4.11.1	General	40

4.11.2	Test signal configurations	40

4.11.2.1	ATCR1: UTRA multicarrier operation	40

4.11.2.1.1	General	40

4.11.2.1.2	ATCR1a generation	40

4.11.2.1.3	ATCR1b generation	41

4.11.2.1.4	ATCR1 power allocation	41

4.11.2.2	ANTCR1: UTRA FDD multicarrier non-contiguous operation	41

4.11.2.2.1	General	41

4.11.2.2.2	ANTCR1 generation	41

4.11.2.2.3	ANTCR1 power allocation	41

4.11.2.3	ATCR2: E-UTRA multicarrier operation	42

4.11.2.3.1	General	42

4.11.2.3.2	ATCR2a generation	42

4.11.2.3.3	ATCR2b generation	42

4.11.2.3.4	ATCR2 power allocation	42

4.11.2.4	ANTCR2: E-UTRA multicarrier non-contiguous operation	43

4.11.2.4.1	General	43

4.11.2.4.2	ANTCR2 generation	43

4.11.2.4.3	ANTCR2 power allocation	43

4.11.2.5	ATCR3: UTRA and E-UTRA multi RAT operation	43

4.11.2.5.1	General	43

4.11.2.5.2	ATCR3a generation	44

4.11.2.5.3	ATCR3b generation	44

4.11.2.5.4	ATCR3 power allocation	44

4.11.2.6	ANTCR3: UTRA and E-UTRA multi RAT non-contiguous operation	44

4.11.2.6.1	General	44

4.11.2.6.2	ANTCR3a generation	45

4.11.2.6.3	ANTCR3 power allocation	45

4.11.2.7	ATCR4: Single carrier for receiver tests	45

4.11.2.7.1	ATCR4a generation	45

4.11.2.7.2	ATCR4b generation	45

4.11.2.7.3	ATCR4c generation	46

4.11.2.7.4	ATCR4 power allocation	46

4.11.2.8	Generation of MB-MSR test configurations	46

4.11.2.8.1	ATCR5a: MB-MSR test configuration for full carrier allocation	46

4.11.2.8.1.1	General	46

4.11.2.8.1.2	ATCR5a generation	46

4.11.2.8.1.3	ATCR5a power allocation	47

4.11.2.8.2	ATCR5b: MB-MSR test configuration with high PSD per carrier	47

4.11.2.8.2.1	General	47

4.11.2.8.2.2	ATCR5b generation	47

4.11.2.8.2.3	ATCR5b power allocation	48

4.11.2.9	ATCR6: Single carrier for Transmitter tests	48

4.11.2.9.1	ATCR6a generation	48

4.11.2.9.2	ATCR6b generation	48

4.11.2.9.3	ATCR6c generation	48

4.11.2.9.4	ATCR6 power allocation	48

4.12	RF channels and test models	48

4.12.1	RF channels	48

4.12.2	Test models	49

4.13	Format and interpretation of tests	50

4.14	Reference coordinate system	51

4.15	Co-location requirements	51

4.15.1	General	51

4.15.2.2	Co-location test antenna characteristics	52

5	Applicability of Requirements	53

5.1	General	53

5.2	Test configurations for AAS BS for operating bands where MSR with more than 1 RAT is supported	54

5.3	Test configurations for multi-carrier capable AAS BS in operating bands where one RAT capability sets are supported	57

5.3.1	General	57

5.3.2	AAS BS supporting one RAT only MSR in the operating band	57

5.3.3	AAS BS supporting Single-RAT UTRA in the operating band	61

5.3.4	AAS BS supporting Single-RAT E-UTRA in the operating band	62

5.4	Test configurations for AAS BS operating bands with multi-band dependencies	64

5.4.1	AAS BS operating bands with multi-band dependencies supporting MSR operation	64

5.4.2	AAS BS operating bands with multi-band dependencies supporting Single-RAT only	66

6	Radiated transmitter characteristics	67

6.1	General	67

6.2	Radiated Transmit Power	68

6.2.1	Definition and applicability	68

6.2.2	Minimum Requirement	68

6.2.3	Test purpose	68

6.2.4	Method of test	68

6.2.4.1	Initial conditions	68

6.2.4.2	Procedure	69

6.2.5	Test Requirement	69

6.3	OTA Base Station output power	69

6.3.1	General	69

6.3.2	OTA Maximum output power	70

6.3.2.1	Definition and applicability	70

6.3.2.2	Minimum Requirement	70

6.3.2.3	Test purpose	70

6.3.2.4	Method of test	70

6.3.2.4.1	Initial conditions	70

6.3.2.4.2	Procedure	70

6.3.2.5	Test Requirement	71

6.3.3	OTA E-UTRA DL RS power	71

6.3.3.1	Definition and applicability	71

6.3.3.2	Minimum Requirement	71

6.3.3.3	Test purpose	71

6.3.3.4	Method of test	72

6.3.3.4.1	Initial conditions	72

6.3.3.4.2	Procedure	72

6.3.3.5	Test Requirement	72

6.4	OTA Output power dynamics	72

6.4.1	General	72

6.4.2	OTA UTRA Inner loop power control in the downlink	73

6.4.2.1	Definition and applicability	73

6.4.2.2	Minimum Requirement	73

6.4.2.3	Test purpose	73

6.4.2.4	Method of test	73

6.4.2.4.1	Initial conditions	73

6.4.2.4.2	Procedure	73

6.4.2.5	Test Requirement	74

6.4.3	OTA Power control dynamic range	75

6.4.3.1	Definition and applicability	75

6.4.3.2	Minimum Requirement	75

6.4.3.3	Test purpose	75

6.4.3.4	Method of test	75

6.4.3.4.1	Initial conditions	75

6.4.3.4.2	Procedure	75

6.4.3.5	Test Requirement	76

6.4.4	OTA Total power dynamic range	76

6.4.4.1	Definition and applicability	76

6.4.4.2	Minimum Requirement	76

6.4.4.3	Test purpose	76

6.4.4.4	Method of test	77

6.4.4.4.1	Initial conditions	77

6.4.4.4.2	Procedure	77

6.4.4.4.2.1	General procedure	77

6.4.4.4.2.2	UTRA FDD	77

6.4.4.4.2.3	E-UTRA	77

6.4.4.5	Test Requirement	78

6.4.4.5.1	UTRA FDD	78

6.4.4.5.2	E-UTRA	78

6.4.5	OTA IPDL time mask	78

6.4.5.1	Definition and applicability	78

6.4.5.2	Minimum Requirement	78

6.4.5.3	Test purpose	78

6.4.5.4	Method of test	79

6.4.5.4.1	Initial conditions	79

6.4.5.4.2	Procedure	79

6.4.5.5	Test Requirement	79

6.4.6	OTA RE Power control dynamic range	80

6.4.6.1	Definition and applicability	80

6.4.6.2	Minimum Requirement	80

6.4.6.3	Method of test	80

6.5	OTA Transmit ON/OFF power	80

6.5.1	General	80

6.5.2	OTA Transmitter OFF power	80

6.5.2.1	Definition and applicability	80

6.5.2.2	Minimum Requirement	81

6.5.2.3	Test purpose	81

6.5.2.4	Method of test	81

6.5.2.4.1	Initial conditions	81

6.5.2.4.2	Procedure	81

6.5.2.5	Test Requirement	82

6.5.3	OTA Transmitter transient period	82

6.5.3.1	Definition and applicability	82

6.5.3.2	Minimum Requirement	82

6.5.3.3	Test purpose	82

6.5.3.4	Method of test	83

6.5.3.4.1	Initial conditions	83

6.5.3.4.2	Procedure	83

6.5.3.5	Test Requirement	83

6.6	OTA Transmitted signal quality	84

6.6.1	General	84

6.6.2	OTA Frequency Error	84

6.6.2.1	Definition and applicability	84

6.6.2.2	Minimum Requirement	84

6.6.2.3	Test purpose	84

6.6.2.4	Method of test	84

6.6.2.5	Test Requirement	84

6.6.2.5.1	UTRA FDD test requirement	84

6.6.2.5.2	E-UTRA test requirement	84

6.6.3	OTA Time alignment error	85

6.6.3.1	Definition and applicability	85

6.6.3.2	Minimum Requirement	85

6.6.3.3	Test purpose	85

6.6.3.4	Method of test	85

6.6.3.4.1	Initial conditions	85

6.6.3.4.1.1	General test conditions	85

6.6.3.4.1.2	UTRA FDD	86

6.6.3.4.1.3	E-UTRA	86

6.6.3.4.2	Procedure	86

6.6.3.4.2.1	General Procedure	86

6.6.3.4.2.2	UTRA FDD Procedure	86

6.6.3.4.2.3	E-UTRA Procedure	86

6.6.3.5	Test Requirement	87

6.6.3.5.1	UTRA FDD test requirement	87

6.6.3.5.2	E-UTRA test requirement	87

6.6.4	OTA Modulation quality	87

6.6.4.1	Definition and applicability	87

6.6.4.2	Minimum Requirement	87

6.6.4.3	Test purpose	88

6.6.4.4	Method of test	88

6.6.4.4.1	UTRA Method of test	88

6.6.4.4.1.1	Initial conditions	88

6.6.4.4.1.2	Procedure	88

6.6.4.4.1.2.1	General procedure	88

6.6.4.4.1.2.3	PCDE  procedure	89

6.6.4.4.1.2.4	RCDE  procedure	89

6.6.4.4.2	E-UTRA Method of test	89

6.6.4.4.2.1	Initial conditions	89

6.6.4.4.2.2	Procedure	89

6.6.4.5	Test Requirement	90

6.6.4.5.1	UTRA test requirement	90

6.6.4.5.2	E-UTRA test requirement	90

6.7	OTA Unwanted Emissions	91

6.7.1	General	91

6.7.2	OTA occupied bandwidth	92

6.7.2.1	Definition and applicability	92

6.7.2.2	Minimum Requirement	92

6.7.2.3	Test purpose	92

6.7.2.4	Method of test	92

6.7.2.4.1	Initial conditions	92

6.7.2.4.1.1	General test conditions	92

6.7.2.4.1.2	UTRA FDD	92

6.7.2.4.1.3	E-UTRA	92

6.7.2.4.2	Procedure	93

6.7.2.5	Test Requirement	94

6.7.2.5.1	MSR	94

6.7.2.5.2	UTRA FDD	94

6.7.2.5.3	E-UTRA	94

6.7.3	OTA Adjacent Channel Leakage power Ratio	94

6.7.3.1	Definition and applicability	94

6.7.3.2	Minimum Requirement	94

6.7.3.3	Test purpose	94

6.7.3.4	Method of test	95

6.7.3.4.1	Initial conditions	95

6.7.3.4.1.1	General test conditions	95

6.7.3.4.1.2	MSR	95

6.7.3.4.1.3	UTRA FDD	95

6.7.3.4.1.4	E-UTRA	95

6.7.3.4.2	Procedure	95

6.7.3.4.2.1	General	95

6.7.3.4.2.2	MSR	96

6.7.3.4.2.3	UTRA FDD	96

6.7.3.4.2.4	E-UTRA	96

6.7.3.5	Test Requirement	97

6.7.3.5.1	MSR	97

6.7.3.5.1.1 MSR E-UTRA test requirement	97

6.7.3.5.1.2 MSR UTRA FDD test requirement	99

6.7.3.5.1.3 OTA Cumulative ACLR test requirement in non-contiguous spectrum	100

6.7.3.5.2	UTRA FDD	101

6.7.3.5.2.1	OTA ACLR	101

6.7.3.5.2.2	OTA Cumulative ACLR test requirement in non-contiguous spectrum or multiple bands	102

6.7.3.5.3	E-UTRA	103

6.7.3.5.3.1	OTA ACLR	103

6.7.3.5.3.2	OTA Cumulative ACLR test requirement in non-contiguous spectrum	104

6.7.4	OTA Spectrum emission mask	105

6.7.4.1	Definition and applicability	105

6.7.4.2	Minimum Requirement	106

6.7.4.3	Test purpose	106

6.7.4.4	Method of test	106

6.7.4.4.1	Initial conditions	106

6.7.4.4.1.1	General test conditions	106

6.7.4.4.1.2	UTRA FDD	106

6.7.4.4.2	Procedure	106

6.7.4.5	Test Requirement	107

6.7.4.5.1	UTRA FDD	107

6.7.5	OTA Operating band unwanted emission	117

6.7.5.1	Definition and applicability	117

6.7.5.2	Minimum Requirement	117

6.7.5.3	Test purpose	117

6.7.5.4	Method of test	117

6.7.5.4.1	Initial conditions	117

6.7.5.4.2	Procedure	118

6.7.5.5	Test Requirement	118

6.7.5.5.1	General	118

6.7.5.5.2	MSR Band categories 1 and 3	119

6.7.5.5.3	MSR Band Category 2	123

6.7.5.5.4	MSR Additional requirements	128

6.7.5.5.4.1	Limits in FCC Title 47	128

6.7.5.5.4.2	Unsynchronized operation for BC3	128

6.7.5.5.4.3	Protection of DTT	129

6.7.5.5.5	E-UTRA	129

6.7.5.5.5.1	General	129

6.7.5.5.5.2	Wide Area BS (Category A)	130

6.7.5.5.5.3	Wide Area BS Category B (Option1)	135

6.7.5.5.5.4	Wide Area BS Category B (Option 2)	139

6.7.5.5.5.5	Local Area BS (Category A and B)	141

6.7.5.5.5.6	Medium Range BS (Category A and B)	144

6.7.5.5.5.7	Additional requirements	150

6.7.6	OTA Spurious emission	151

6.7.6.1	General	151

6.7.6.2	Mandatory Requirements	152

6.7.6.2.1	Definition and applicability	152

6.7.6.2.2	Minimum Requirement	152

6.7.6.2.3	Test purpose	152

6.7.6.2.4	Method of test	152

6.7.6.2.4.1	Initial conditions	152

6.7.6.2.4.2	Procedure	153

6.7.6.2.5	Test Requirement	153

6.7.6.2.5.1	MSR operation	153

6.7.6.2.5.2	Single RAT UTRA operation	154

6.7.6.2.5.3	Single RAT E-UTRA operation	155

6.7.6.3	Protection of the BS receiver of own or different BS	156

6.7.6.3.1	Definition and applicability	156

6.7.6.3.2	Minimum Requirement	156

6.7.6.3.3	Test purpose	156

6.7.6.3.4	Method of test	156

6.7.6.3.4.1	Initial conditions	156

6.7.6.3.4.2	Procedure	157

6.7.6.3.5	Test Requirement	158

6.7.6.3.5.1	MSR operation	158

6.7.6.3.5.2	Single RAT UTRA operation	158

6.7.6.3.5.3	Single RAT E-UTRA operation	158

6.7.6.4	Additional spurious emissions requirements	158

6.7.6.4.1	Definition and applicability	158

6.7.6.4.2	Minimum Requirement	159

6.7.6.4.3	Test purpose	159

6.7.6.4.4	Method of test	159

6.7.6.4.4.1	Initial conditions	159

6.7.6.4.4.2	Procedure	159

6.7.6.4.5	Test Requirement	160

6.7.6.4.5.1	MSR operation	160

6.7.6.4.5.2	Single RAT UTRA operation	160

6.7.6.4.5.3	Single RAT E-UTRA operation	165

6.7.6.5	Co-location with other base stations	173

6.7.6.5.1	Definition and applicability	173

6.7.6.5.2	Minimum Requirement	173

6.7.6.5.3	Test purpose	174

6.7.6.5.4	Method of test	174

6.7.6.5.4.1	Initial conditions	174

6.7.6.5.4.2	Procedure	174

6.7.6.5.5	Test Requirement	174

6.8	OTA Transmitter intermodulation	182

6.8.1	Definition and applicability	182

6.8.2	Minimum Requirement	182

6.8.3	Test purpose	182

6.8.4	Method of test	182

6.8.4.1	Initial conditions	182

6.8.4.2	Procedure	183

6.8.5	Test Requirement	185

7	Radiated receiver characteristics	188

7.1	General	188

7.2	OTA sensitivity	189

7.2.1	Definition and applicability	189

7.2.2	Minimum Requirement	190

7.2.3	Test Purpose	190

7.2.4	Method of test	190

7.2.4.1	Initial conditions	190

7.2.4.2	Procedure	190

7.2.5	Test Requirements	191

7.2.5.1	General	191

7.2.5.2	UTRA FDD Test Requirements	191

7.2.5.3	UTRA TDD 1,28Mcp option Test Requirements	191

7.2.5.4	E-UTRA Test Requirements	191

7.3	OTA Reference sensitivity level	192

7.3.1	Definition and applicability	192

7.3.2	Minimum Requirement	192

7.3.3	Test purpose	192

7.3.4	Method of test	192

7.3.4.1	Initial conditions	192

7.3.4.2	Procedure	192

7.3.5	Test Requirement	193

7.4	OTA Dynamic range	194

7.4.1	Definition and applicability	194

7.4.2	Minimum Requirement	194

7.4.3	Test purpose	194

7.4.4	Method of test	194

7.4.4.1	Initial conditions	194

7.4.4.2	Procedure	194

7.4.5	Test Requirement	195

7.4.5.1	UTRA FDD operation	195

7.4.5.2	E-UTRA operation	195

7.5	OTA Adjacent channel selectivity, general blocking, and narrowband blocking	197

7.5.1	Definition and applicability	197

7.5.2	Minimum Requirement	198

7.5.3	Test purpose	198

7.5.4	Method of test	198

7.5.4.1	Initial conditions	198

7.5.4.2	Procedure	198

7.5.4.2.1	General procedure	198

7.5.4.2.2	MSR operation	198

7.5.4.2.2.1	Procedure for general blocking	198

7.5.4.2.2.2	Procedure for narrowband blocking	199

7.5.4.2.2.3	Procedure for additional BC3 blocking requirement	199

7.5.4.2.3	Single RAT UTRA FDD operation	199

7.5.4.2.4	Single RAT E-UTRA operation	199

7.4.4.2.4.1	Procedure for adjacent channel selectivity	199

7.5.4.2.4.2	Procedure for narrow-band blocking	200

7.5.5	Test Requirement	200

7.5.5.1	MSR operation	200

7.5.5.1.1	General blocking test requirement	200

7.5.5.1.2	General narrowband blocking test requirement	201

7.5.5.1.3	Additional BC3 blocking test requirement	202

7.5.5.2	Single RAT UTRA FDD operation	203

7.5.5.3	Single RAT E-UTRA operation	203

7.6	OTA Blocking	206

7.6.1	General	206

7.6.2	General Requirement	206

7.6.2.1	Definition and applicability	206

7.6.2.2	Minimum Requirement	206

7.6.2.3	Test purpose	206

7.6.2.4	Method of test	206

7.6.2.4.1	Initial conditions	206

7.6.2.4.2	Procedure	207

7.6.2.4.2.1	General procedure	207

7.6.2.5	Test Requirement	209

7.6.3	Co-location Requirement	212

7.6.3.1	Definition and applicability	212

7.6.3.2	Minimum Requirement	212

7.6.3.3	Test purpose	213

7.6.3.4	Method of test	213

7.6.3.4.1	Initial conditions	213

7.6.3.4.2	Procedure	213

7.6.3.5	Test Requirement	213

7.7	OTA Receiver spurious emissions	217

7.7.1	Definition and applicability	217

7.7.2	Minimum Requirement	217

7.7.3	Test purpose	217

7.7.4	Method of test	217

7.7.4.1	Initial conditions	217

7.7.4.2	Procedure	218

7.7.5	Test Requirement	218

7.8	OTA Receiver intermodulation	219

7.8.1	Definition and applicability	219

7.8.2	Minimum Requirement	219

7.8.3	Test purpose	219

7.8.4	Method of test	220

7.8.4.1	Initial conditions	220

7.8.4.2	Procedure	220

7.8.4.2.1	General procedure	220

7.8.4.2.2	MSR operation	220

7.8.4.2.3	Single RAT UTRA FDD operation	221

7.7.4.2.4	Single RAT E-UTRA operation	221

7.8.5	Test Requirement	221

7.8.5.1	MSR operation	221

7.8.5.1.1	General intermodulation test requirement	221

7.8.5.1.2	General narrowband intermodulation test requirement	222

7.8.5.2	Single RAT UTRA operation	224

7.8.5.3	Single RAT E- UTRA operation	225

7.9	OTA In-channel selectivity	229

7.9.1	Definition and applicability	229

7.9.2	Minimum Requirement	229

7.9.3	Test purpose	230

7.9.4	Method of test	230

7.9.4.1	Initial conditions	230

7.9.4.2	Procedure	230

7.9.5	Test Requirement	230

8	Radiated performance requirements	232

8.1	General	232

8.1.1	OTA demodulation branches	232

8.2	Radiated performance requirements for MSR	233

8.3	Radiated performance requirements for UTRA FDD	233

8.3.1	General	233

8.3.2	Definitions and applicability	234

8.3.3	Minimum requirements	234

8.3.4	Test purposes	234

8.3.5	Method of test	235

8.3.5.1	Initial conditions	235

8.3.5.2	Procedure	235

8.3.6	Test requirements	236

8.4	Radiated performance requirements for E-UTRA	236

8.4.1	General	236

8.4.2	Definitions and applicability	237

8.4.3	Minimum requirements	238

8.4.4	Test purposes	238

8.4.5	Method of test	238

8.4.5.1	Initial conditions	238

8.4.5.2	Procedure	239

8.4.6	Test requirements	240

Annex A (normative):	Test system characterization	241

Annex B (normative):	Calibration	242

Annex C (informative):	Test tolerances and derivation of test requirements	243

C.1	General	243

C.2	Measurement of transmitter (OTA)	244

C.3	Measurement of receiver (OTA)	245

Annex D (informative):	Test system set-up	246

D.1	Transmitter	246

D.1.1	Radiated Transmit Power, OTA E-UTRA DL RS power, output power dynamics and Transmitter signal quality	246

D.1.2	OTA Base Station output power, ACLR, OTA spectrum emissions mask, OTA operating band unwanted emissions	247

D.1.3	OTA spurious emissions	247

D.1.4	OTA Co-location emissions, TX OFF power	248

D.1.5	OTA Transmitter Intermodulation	248

D.2	Receiver	249

D.2.1	OTA sensitivity and OTA Reference sensitivity	249

D.2.2	OTA Dynamic range	249

D.2.3	OTA Adjacent channel selectivity, general blocking, and narrowband blocking	250

D.2.4	OTA Blocking	251

D.2.5	OTA Receiver spurious emissions	252

D.2.6	OTA Receiver intermodulation	252

D.2.7	OTA In-channel selectivity	253

Annex E (normative):	Estimation of Measurement Uncertainty	254

E.1	General	254

E.2	Measurement methodology descriptions	254

E.3	Measurement uncertainty budget format	254

E.4	Measurement uncertainty budgets	254

E.5	Measurement error contribution descriptions	254

Annex F (normative):	 TRP measurement grids	254

F.1	General	254

F.2	Spherical equal angle grid	254

F.2.1	General	254

F.2.2	Rayleigh sampling criteria	255

F.3	Spherical equal area grid	255

F.4	Spherical Fibonacci grid	256

F.5	Orthogonal 2 cut grid	256

F.6	Wave vector space grid	256

Annex G (normative):	 Environmental requirements for the BS equipment	257

G.1	General	257

G.2	Normal test environment	257

G.3	Extreme test environment	257

G.3.1	General	257

G.3.2	Extreme temperature	257

G.4	Vibration	258

G.5	Power supply	258

G.6	Measurement of test environments	258

G.7	OTA extreme test methods	258

G.7.1	Direct far field method	258

G.7.2	Relative method	259

Annex H (informative):	 Measuring noise close to noise-floor	261

Annex I (informative):	Change history	262

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies radiated test methods and conformance requirements for 2 types of AAS BS; hybrid requirements set which specify requirements for a hybrid AAS BS with both a conducted and a radiated interface and OTA requirements set which specify requirements for an OTA AAS BS which has a radiated interface only.

The hybrid AAS BS requirements are specified for E-UTRA AAS Base Station (BS), the FDD mode of UTRA AAS Base Station (BS), the 1,28 Mcps TDD mode of UTRA AAS Base Station (BS) in single RAT and any MSR AAS Base Station (BS) implementation of these RATs including NR operation.

The OTA AAS BS requirements are specified for E-UTRA AAS Base Station (BS), the FDD mode of UTRA AAS Base Station (BS), in single RAT and any MSR AAS Base Station (BS) implementation of these RATs and/or NR.

The requirements have been derived from, and are consistent with non-AAS BS specifications in 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] or 3GPP TS 37.104 [5] and where applicable with the NR specification 38.104 [33]. The technical specification 3GPP TS 37.145 is in 2 parts, part 3GPP TS 37.145-1 [9] covers conducted requirements and part 3GPP TS 37.145-2 (the present document) covers radiated requirements.

The present document does not establish radiated test methods and conformance requirements for Band 46 operation as it is not supported by AAS BS. Conducted Band 46 test requirements are still applicable for AAS BS for protection of and against Band 46 operation, as specified in 3GPP TS 37.145-1 [9].

The present document does not establish minimum RF characteristics or minimum performance requirements for Narrow-Band Internet of Things (NB-IoT) in band, NB-IoT guard band, or standalone NB-IoT operation, for AAS BS in single RAT E-UTRA operation or in MSR operation using E-UTRA.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD)".

[3]	3GPP TS 25.105: "Base Station (BS) radio transmission and reception (TDD)".

[4]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[5]	3GPP TS 37.104: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception".

[6]	3GPP TS 37.105: "Active Antenna System (AAS) Base Station (BS) transmission and reception".

[7]	3GPP TR 37.842: "E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)".

[8]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[9]	3GPP TS 37.145-1: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing".

[10]	3GPP TS 25.141: "Base Station (BS) conformance testing (FDD)".

[11]	3GPP TS 25.142: "Base Station (BS) conformance testing (TDD)".

[12]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing".

[13]	3GPP TS 37.141: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing".

[14]	Recommendation ITU-R M.328: "Spectra and bandwidth of emissions".

[15]	3GPP TS 25.331 (V14.3.0): "Radio Resource Control (RRC); Protocol specification (Release 14)".

[16]	Recommendation ITU-R SM.329-10: "Unwanted emissions in the spurious domain".

[17]	FCC publication number 662911: "Emissions Testing of Transmitters with Multiple Outputs in the Same Band"

[18]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission

[19]	CEPT ECC Decision (13)03, "The harmonised use of the frequency band 1452-1492 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)".

[20]	IEC 60721: "Classification of environmental conditions"

[21]	IEC 60721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[22]	IEC 60721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 4: Stationary use at non-weather protected locations"

[23]	ETSI EN 300 019-1-3: "Environmental Engineering (EE); Environmental conditions and environmental tests for telecommunications equipment; Part 1-3: Classification of environmental conditions; Stationary use at weatherprotected locations"

[24]	ETSI EN 300 019-1-4: "Environmental Engineering (EE); Environmental conditions and environmental tests for telecommunications equipment; Part 1-4: Classification of environmental conditions; Stationary use at non-weatherprotected locations"

[25]	IEC 60068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[26]	IEC 60068-2-2 (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[27]	IEC 60068-2-6 (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[28]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation"

[29]	3GPP TR 37.843: "Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements"

[30]	Recommendation ITU-T O.153: "Basic parameters for the measurement of error performance at bit rates below the primary rate"

[31]	3GPP TR 25.942: "Radio Frequency (RF) system scenarios".

[32]	3GPP TS 45.004: "Digital cellular telecommunications system (Phase 2+); Modulation".

[33]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception".

[34]	3GPP TS 38.141-2: "Base Station (BS) conformance testing Part 2: Radiated conformance testing".

[35]	3GPP TS 38.141-1: "NR;Base Station (BS) conformance testing; Part 1: Conducted conformance testing".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

AAS BS receiver: composite receiver function of an AAS BS receiving in an operating band

active antenna system base station: base station system which combines an Antenna Array with an Active transceiver unit array and a Radio Distribution Network

band category: group of operating bands for which the same MSR scenarios apply

Base Station RF Bandwidth: bandwidth in which a base station transmits and/or receives single or multiple carrier(s) and/or RATs simultaneously within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth

beam: main lobe of a radiation pattern from an AAS BS

NOTE:	For certain AAS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the -3 dB EIRP contour of the beam

beam direction pair: data set consisting of  the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is supposed to be found

beamwidth: angles describing the major and minor axes of an ellipsoid closest fit to an essentially elliptic half-power contour of a beam

carrier: modulated waveform conveying the physical channels

carrier aggregation: aggregation of two or more E-UTRA component carriers in order to support wider transmission bandwidths

channel bandwidth: RF bandwidth supporting a single RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell

NOTE 1:	The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	For UTRA FDD, the channel bandwidth is the nominal channel spacing specified in 3GPP TS 25.104 [2], For UTRA TDD 1,28 Mcps, the channel bandwidth is the nominal channel spacing specified in 3GPP TS 25.105 [3].

NOTE 3:	For E-UTRA, the channel bandwidths are specified in 3GPP TS 36.104 [4].

NOTE 4:  In TS 38.104 [33] for NR, channel bandwidths are referred to as BS channel bandwidths, since for NR BS and UE channel bandwidths may differ.

channel edge: lowest or highest frequency of the NR carrier, separated by the BS channel bandwidth

contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s)

demodulation branch: single input of the AAS BS receiver to the demodulation algorithms

NOTE 1:	For UTRA FDD, non-AAS BS a demodulation branch is referred to as a receive diversity branch or an UL MIMO branch. For E-UTRA non-AAS BS a demodulation branch is referred to as an RX antenna in the performance requirement tables.

NOTE 2:	The term "RX antenna" in clause 8 (i.e. Performance requirements) of the E-UTRA specification 3GPP TS 36.104 [4] does not refer to physical receiver antennas.

co-location reference antenna: a passive antenna used as reference for base station to base station co-location requirements

contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s)

downlink operating band: part of the (FDD) operating band designated for downlink

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE 1:	The sensitivity is the minimum received power level at which a RAT specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (0 dBi).

fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%

highest carrier: the carrier with the highest carrier frequency transmitted/received in a specified frequency band

hybrid AAS BS:	AAS BS which has both a conducted RF interface and a radiated RF interface in the far field and conforms to a hybrid requirements set

NOTE: 	For NR operation, a hybrid AAS BS corresponds to NR type 1-H in [34].

hybrid requirements set: complete set of requirements applied to a hybrid AAS BS with both conducted and radiated  requirements

inter-band carrier aggregation: carrier aggregation of component carriers in different operating bands

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

intra-band contiguous carrier aggregation: contiguous carriers aggregated in the same operating band

intra-band non-contiguous carrier aggregation: non-contiguous carriers aggregated in the same operating band

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

maximum carrier TRP output power: mean power level measured per RIB during the transmitter ON period for a specific carrier in a specified reference condition and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

maximum total TRP output power: mean power level measured per RIB during the transmitter ON period in a specified reference condition and corresponding to the declared rated total TRP output power (Prated,t,TRP)

minSENS: the lowest declared EIS value for the OSDD’s declared for OTA sensitivity requirement.

minSENS RoAoA: the reference RoAoA associated with the OSDD with the lowest declared EIS value.

MSR operation: operation of AAS BS declared to be MSR in particular operating band(s)

multi-band RIB: operating band specific RIB which is paired with one or more additional operating band specific RIBs where the multiple bands are supported through common active electronic component(s)

non-AAS BS: BS conforming to one of  the RF requirement specifications 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] or 3GPP TS 37.104 [5]

non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s)

operating band: frequency range in which the AAS BS operates (paired or unpaired), that is defined with a specific set of technical requirements

OTA AAS BS: AAS BS which has ≥8 transceiver units for E-UTRA or MSR and ≥4 transceiver units for UTRA per cell and has a radiated RF interface only and conforms to the OTA requirements set.

NOTE: For NR operation, an OTA AAS BS corresponds to an NR type 1-O BS in [34].

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE 1:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

NOTE 2:	OTA peak directions set definition (applicabile to multiple directional requirements) is replacing the Rel-13/14 EIRP accuracy directions set definition (which was applicable to EIRP requirement only).

OTA REFSENS RoAoA: Is the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction.

NOTE: This contour will be related to the average element/sub-array radiation pattern 3dB beam width.

OTA requirements set:	complete set of OTA requirements applied to an OTA AAS BS.

OTA sensitivity directions declaration: set of manufacturer declarations comprising one or more EIS values (with related RAT and channel bandwidth), and the directions where it (they) applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction

radiated interface boundary: operating band specific reference point for radiated requirements, defined in the near field or in the far field depending on the radiated requirement’s testability feasibility

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier.

radio distribution network: passive network which distributes radio signals generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

rated beam EIRP: EIRP that is declared as being achieved in the beam peak direction associated with a particular beam direction pair

rated carrier TRP output power: mean power level declared by the manufacturer per carrier, for BS operating in single carrier, multi-carrier, or carrier aggregation configurations that the manufacturer has declared to be available at the RIB during the transmitter ON period

rated total TRP output power: mean power level declared by the manufacturer, that the manufacturer has declared to be available at the RIB during the transmitter ON period

receiver target: angles of arrival in which reception is performed

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD

receiver target reference direction: direction inside the receiver target redirection range declared by the manufacturer for conformance testing.

NOTE	For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA.

reference beam direction: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy compliance directions set

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the OTA peak directions set

reference RoAoA: the sensitivity RoAoA associated with the receiver target reference direction for each OSDD.

sensitivity RoAoA: RoAoA within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific AAS BS direction setting

single-band RIB: operating band specific RIB without any common active electronic component(s) shared with other operating bands

single RAT E-UTRA operation: operation of AAS BS declared to be single RAT E-UTRA in the operating band

NOTE: 	Single RAT E-UTRA operation does not cover in-band NB-IoT, nor guardband NB-IoT operation.

single RAT UTRA operation: operation of AAS BS declared to be single RAT UTRA in the operating band

sub-block: one contiguous allocated block of spectrum for use by the same Base Station

NOTE	There may be multiple instances of sub-blocks within a Base Station RF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within an Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

TAB connector: transceiver array boundary connector

throughput: number of payload bits successfully received per second for a reference measurement channel in a specified reference condition

total radiated power: is the total power radiated by the antenna.

NOTE 1:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.

NOTE 2:	total radiated power is defined in both the near-field region and the far-field region.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: bandwidth of an instantaneous E-UTRA transmission from a UE or BS, measured in Resource Block units

single-band RIB: operating band specific RIB without any common active electronic component(s) shared with other operating bands

total radiated power: the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.  Total radiated power is defined in both the near-field region and the far-field region.

uplink operating band: part of the (FDD) operating band designated for uplink

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ	The Beam width in θ

BeWϕ	The Beam width in ϕ

BeWθ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees.

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees.

BWChannel	Channel bandwidth (for E-UTRA or NR)

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz. BWChannel_CA= Fedge_high- Fedge_low.

BWConfig	Transmission bandwidth configuration (for E-UTRA), expressed in MHz, where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink.

DwPTS	Downlink part of the special subframe (for E-UTRA TDD) operation

Eb	Average energy per information bit (for UTRA)

f	Frequency

f	Separation between the Base Station RF bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax	The largest value of f used for defining the requirement

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔminSENS	Difference between conducted reference sensitivity and EISminsens

ΔOTAREFSENS	Difference between conducted reference sensitivity and OTA REFSENS

Δsample	The difference between the nominal and extreme power measurements dueing extreme EIRP testing, Pmax,sample,nom  - Pmax,sample,ex

EISminsens	The EIS declared for the minSENS RoAoA

FC	Carrier centre frequency

Ffilter	Filter centre frequency

f_offset	Separation between the Base Station RF bandwidth edge frequency and the centre of the measuring filter

f_offsetmax	The maximum value of f_offset used for defining the requirement

FBW RF,high	Upper RF bandwidth edge, where FBW RF,high = FC,high + Foffset, RAT

FBW RF,low	Lower RF bandwidth edge, where FBW RF,low = FC,low - Foffset, RAT

FC	RF reference frequency on the channel raster

FC,high	Centre frequency of the highest transmitted/received carrier.

FC,low	Centre frequency of the lowest transmitted/received carrier.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

Fedge_low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset.

Fedge_high	The upper edge of aggregated channel bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset. Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper RF bandwidth edge, sub-block edge or inter-RF bandwidth edge, or from the centre frequency of the lowest transmitted/received carrier to the lower RF bandwidth edge, sub-block edge or inter-RF bandwidth edge for a specific RAT.

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fuw	Frequency offset of unwanted signal

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all transceiver units supporting the operating band

N0	Total one-sided noise power spectral density due to all noise sources (for UTRA)

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

NRB	Transmission bandwidth configuration, expressed in units of resource blocks (for E-UTRA)

Pmax,c,EIRP	The maximum carrier EIRP when the AAS BS is configured at the rated carrier output TRP (PRated,c,TRP)

Pmax,c,EIRP, extreme	The maximum carrier EIRP when the AAS BS is configured at the rated carrier output TRP (PRated,c,TRP) under extreme conditions, either measured directly or calculated.

Pmax,c,TRP	The maximum carrier output TRP per cell

Pmax,sample,nom	The measured sample power in extreme conditionals chamber when the AAS BS is configured at the rated carrier output TRP (Prated,c,TRP), under nominal conditions.

Pmax,sample,ext	The measured sample power in extreme conditionals chamber when the AAS BS is configured at the rated carrier output TRP (Prated,c,TRP), under extreme conditions.Pmax,t,TRP	The maximum total output power per cell

Prated,c,EIRP	The rated carrier output EIRP when the AAS BS is configured at the rated carrier output TRP (Prated,c,TRP)

Prated,c,FBWhigh	The rated carrier EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,FBWlow	The rated carrier EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,TRP	The rated carrier output TRP

Prated,t,TRP	Rated total TRP declared per RIB

Wgap	Sub-block gap size or Inter RF Bandwidth gap size

The angle in the reference coordinate system between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

The angle in the reference coordinate system between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°.

PREFSENS	Conducted reference Sensitivity power level

TRPEstimate	Numerically approximated TRP

Figure 3.2-1: Illustration of RF bandwidth related symbols and definitions for Multi-standard Radio

Figure 3.2-2: Illustration of RF bandwidth related symbols and definitions
for non-contiguous Multi-standard Radio

Figure 3.2-3: Illustration of maximum radio bandwidth and Total RF bandwidth
for Multi-band Multi-standard Radio

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS BS	Active Antenna System Base Station

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

BC	Band Category

BER	Bit Error Rate

BLER	Block Error Rate

CA	Carrier Aggregation

CACLR	Cumulative ACLR

CLTA 	Co-Location Test Antenna

DTT	Digital Terrestrial Television

DUT	Device Under Test

DIP	Dominant Interferer Proportion

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

FBW	Fractional Bandwidth

FDD	Frequency Division Duplex

FRC	Fixed Reference Channel

ICS	In-Channel Selectivity

ITU	International Telecommunication Union

ITU-R	Radio communication Sector of the ITU

MB-MSR	Multi-Band Multi-Standard Radio

MBT	Multi-Band Testing

MC	Multi-Carrier in a Single RAT

NR	New Radio

OBW	Occupied Band Width

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

RAT	Radio Access Technology

RB	Resource Block (for E-UTRA)

RDN	Radio Distribution Network

REFSENS	Reference Sensitivity

RIB	Radiated Interface Boundary

RF	Radio Frequency

RoAoA	Range of Angles of Arrival

SBT	Single Band Testing

SC	Single-Carrier

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TDD	Time Division Duplex

TRP	Total Radiated Power

TT	Test Tolerance

UE	User Equipment

# 4	General test conditions and declarations

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in part 2 of this specification, i.e. to all AAS BS radiated tests.

The minimum requirements for AAS BS radiated requirements are given in 3GPP TS 37.105 [14] subclause 9 and 10 for the radiated transmitter and radiated receiver characteristics, respectively. Test Tolerances for the radiated test requirements explicitly stated in part 2 of the present specification are given in annex C of this specification.

Test Tolerances are individually calculated for each test. The Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

Table 4.1.1-1: Overview of radiated Tx requirements

| AAS BS requirement |  | OTA requirement type | Coverage range | Notes |
| --- | --- | --- | --- | --- |
| Base station output power | Output power accuracy for EIRP | Directional | OTA peak directions set | Output power accuracy for EIRP requirement is already included as a core requirement in TS 37.105 []. |
|  | Output power accuracy for TRP | TRP | N/A |  |
| E-UTRA DL RS power |  | Directional | OTA peak directions set | Conformance testing is carried out in the reference direction |
| Output power dynamics |  | Directional | OTA peak directions set | Conformance testing is carried out in the reference direction. |
| Transmitter OFF power |  | Co-location | N/A |  |
| Frequency Error |  | Directional | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Time Alignment Error |  | Directional | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Modulation Quality (EVM) |  | Directional | OTA coverage range | Conformance testing is carried out in the reference direction and the maximum directions of the OTA coverage range on each axis. |
| Unwanted emissions | Occupied Bandwidth | Directional | OTA coverage range | Conformance testing is carried out in the reference direction. |
|  | Adjacent Channel Leakage Radio (ACLR) | TRP | N/A |  |
|  | Spectrum emission mask | TRP | N/A |  |
| Spurious emissions | Mandatory Requirements | TRP | N/A |  |
|  | Protection of the BS receiver of own or different BS | Co-location | N/A |  |
|  | Additional spurious emissions requirements | TRP | N/A | Includes co-existence in same geographical area |
|  | Co-location with other base stations | Co-location | N/A |  |
| Transmitter intermodulation |  | Co-location | N/A | The interferer is applied as a co-location requirements, the radiated emissions requirements are specified in the appropriated referenced sub-clause, generally TRP |
| NOTE: Directional does not imply one compliance direction only. The requirement applies to a single direction at a time. |  |  |  |  |

Table 4.1.1-2: Overview of radiated Rx requirements

| AAS BS requirement |  | OTA requirement type | Applicability levels | Coverage range | Number of conformance directions |
| --- | --- | --- | --- | --- | --- |
| OTA sensitivity |  | Directional | N/A | Receiver target redirection range (D10.8) | 5 |
| OTA reference sensitivity |  | Directional | OTA REFSENS | OTA REFSENS RoAoA | 5 |
| Dynamic range |  | Directional | OTA REFSENS | OTA REFSENS RoAoA | 1 |
| In-band selectivity and blocking |  | Directional | OTA REFSENS and minSENS | OTA REFSENS RoAoA and minSENS RoAoA | 5 |
| ACS and narrowband blocking |  | Directional | OTA REFSENS (NB blocking only) and minSENS | OTA REFSENS RoAoA (NB blocking only) minSENS RoAoA (NB blocking and ACS) | 5 (blocking) 1 (ACS) |
| Out-of-band blocking | Mandatory | Directional | minSENS | minSENS RoAoA | 1 |
|  | Co-location with other base stations | Co-location | N/A | N/A |  |
| Receiver spurious emissions |  | TRP | N/A | N/A | - |
| Receiver intermodulation |  | Directional | OTA REFSENS and minSENS | OTA REFSENS RoAoA and minSENS RoAoA | 1 |
| In-channel selectivity |  | Directional | minSENS | minSENS RoAoA | 1 |
| NOTE: Directional does not imply one compliance direction only. The requirement applies to a single direction at a time. |  |  |  |  |  |

### 4.1.2	Acceptable uncertainty of Test System

#### 4.1.2.1	General

The maximum acceptable uncertainty of the Test System is specified below for each test defined explicitly in the present specification, where appropriate.

The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

For details on measurement uncertainty budget calculation, OTA measurement methodology description (including calibration and measurement stage for each test range), MU budget format and its contributions, refer to TR 37.843 [29].

#### 4.1.2.2	Measurement of transmitter

Table 4.1.2.2-1: Maximum Test System uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power (Normal conditions) | ±1.1 dB, f ≤ 3.0 GHz ±1.3 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.842 [7], subclause 10.3.2.2. Uncertainty budget contributors as well as uncertainty budget assessment described in annex E. |
| 6.2 Radiated transmit power (extreme conditions) | ±2.5 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 | See 3GPP TR 37.843 [28], subclause 10.2.8. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.3.2 OTA Maximum output power | ±1.4 dB, f ≤ 3.0 GHz ±1.5 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.843 [28], subclause 10.4.1. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C |
| 6.3.3 OTA E-UTRA DL RS power | 1.3 dB, f ≤ 3.0 GHz 1.5 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.843 [28], subclause 10.2.2. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.4.2 OTA UTRA Inner loop power control in the downlink | 0.1 dB | See 3GPP TR 37.843 [28], subclause 10.2.3. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.4.3 OTA Power control dynamic range | 1.1 dB | See 3GPP TR 37.843 [28], subclause 10.2.3. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.4.4 OTA Total power dynamic range | 0.3 dB UTRA 0.4 dB E-UTRA & NR | See 3GPP TR 37.843 [28], subclause 10.2.3. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.4.5 OTA IPDL time mask | 0.7 dB | See 3GPP TR 37.843 [28], subclause 10.2.3. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.5 OTA Transmit ON/OFF power | ±3.4 dB, f ≤ 3.0 GHz ±3.6 dB, 3.0 GHz < f ≤ 4.2 GHz (NOTE 1) | See 3GPP TR 37.843 [28], subclause 10.6.2. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.6.2 OTA Frequency Error | 12 Hz | See 3GPP TR 37.843 [28], subclause 10.2.4. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.6.3 OTA Time Alignment Error | 25 ns | See 3GPP TR 37.843 [28], subclause 10.2.6. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.6.4 OTA Modulation Quality | 1 % | See 3GPP TR 37.843 [28], subclause 10.2.5. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.7.2 OTA occupied bandwidth | 1.4MHz, 3MHz Channel BW: 30kHz 5MHz, 10MHz Channel BW: 100kHz 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 50MHz: Channel BW: 300kHz 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz Channel BW: 600 kHz | See 3GPP TR 37.843 [28], subclause 10.2.7. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.7.3 OTA ACLR | ACLR / CACLR ±1.0 dB, f ≤ 3.0 GHz ±1.2 dB, 3.0 GHz < f ≤ 4.2 Absolute limit ±2.2 dB, f ≤ 3.0GHz ±2.7 dB, 3.0GHz < f ≤ 4.2GHz | See 3GPP TR 37.843 [28], subclause 10.4.2. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.7.4 OTA Spectrum emission mask | ±1.8 dB, f ≤ 3.0GHz ±2.0 dB, 3.0GHz < f ≤ 4.2GHz | See 3GPP TR 37.843 [28], subclause 10.4.4. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.7.5 OTA Operating band unwanted emissions | ±1.8 dB, f ≤ 3.0GHz ±2.0 dB, 3.0GHz < f ≤ 4.2GHz | See 3GPP TR 37.843 [28], subclause 10.4.3. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.7.6.2 OTA Transmitter spurious emissions, Mandatory Requirements | 30MHz < f ≤ 6 GHz: ±2.3 dB 6 GHz < f ≤ 19 GHz: ±4.2 dB | See 3GPP TR 37.843 [28], subclause 10.5. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.7.6.3 Transmitter spurious emissions, Protection of BS receiver | ±3.1 dB, f ≤ 3.0GHz ±3.3 dB, 3.0GHz < f ≤ 4.2GHz | See 3GPP TR 37.843 [28], subclause 10.6.3. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.7.6.4 Transmitter spurious emissions, Additional spurious emission requirements | ±2.6 dB, f ≤ 3.0GHz ±3.0 dB, 3.0GHz < f ≤ 4.2GHz | See 3GPP TR 37.843 [28], subclause 10.5. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.7.6.5 Transmitter spurious emissions, Co-location | ±3.1 dB, f ≤ 3.0GHz ±3.3 dB, 3.0GHz < f ≤ 4.2GHz (NOTE 1) | See 3GPP TR 37.843 [28], subclause 10.6.3. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 6.8 OTA Transmitter intermodulation (interferer requirements) This tolerance applies to the stimulus and not the measurements defined in 6.8 | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests (6.6.1, 6.6.2 and 6.6.4) which have to be carried out in the presence of the interferer. ±3.2 dB, f ≤ 3.0GHz ±3.4 dB, 3.0GHz < f ≤ 4.2GHz (NOTE 1) | See 3GPP TR 37.843 [28], subclause 10.6.4. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| NOTE 1: Fulfilling the criteria for CLTA selection and placement in subclause 4.15 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate measurement uncertainty budget in TR 37.843 [28], subclause 10.6 shall be used for evaluating the test system uncertainty. |  |  |

#### 4.1.2.3	Measurement of receiver

Table 4.1.2.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.2 OTA Sensitivity | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.842 [7], subclause 10.3.2.2. Uncertainty budget contributors as well as uncertainty budget assessment described in annex E. |
| 7.3 OTA Reference Sensitivity | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.842 [7], subclause 10.3.2.2. Uncertainty budget contributors as well as uncertainty budget assessment described in annex E. |
| 7.4 OTA Dynamic range | ±0.3 dB | See 3GPP TR 37.843 [28], subclause 10.3.4. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 7.5 OTA Adjacent channel selectivity, general blocking, and narrowband blocking | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.843 [28], subclause 10.3.5. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 7.5 OTA In-band general blocking, | ±1.9 dB, f ≤ 3.0 GHz ±2.2 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.843 [28], subclause 10.3.5. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 7.6.2 OTA Blocking | fwanted ≤ 3 GHz 1 MHz < finterferer ≤ 3 GHz: ±2.0 dB 3 GHz < finterferer ≤ 6 GHz: ±2.1 dB 6 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB 3 GHz < fwanted ≤ 4.2GHz: 1 MHz < finterferer ≤ 3 GHz: ±2.0 dB 3 GHz < finterferer ≤ 6 GHz: ±2.1 dB 6 GHz < finterferer ≤ 12.75 GHz: ±3.6 dB | See 3GPP TR 37.843 [28], subclause 10.8. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 7.6.3 Co-location blocking | ±3.2 dB, f ≤ 3.0 GHz ±3.4 dB, 3.0 GHz < f ≤ 4.2 GHz (NOTE 2) | See 3GPP TR 37.843 [28], subclause 10.6.5. Uncertainty budget contributors as well as uncertainty budget assessment described in annex |
| 7.7 Receiver spurious emissions | ±2.5 dB, 30MHz < f ≤ 6 GHz: dB ±4.2 dB, 6 GHz < f ≤ 19 GHz | See 3GPP TR 37.843 [28], subclause 10.5. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 7.8 OTA Receiver intermodulation (General requirements) | ±2.0 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.843 [28], subclause 10.3.6. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 7.8 OTA Receiver intermodulation (Narrowband requirements) | ±2.0 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.843 [28], subclause 10.3.6. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| 7.9 OTA In-channel selectivity | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz | See 3GPP TR 37.843 [28], subclause 10.3.7. Uncertainty budget contributors as well as uncertainty budget assessment described in annex |
| NOTE 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements or the BER/FER due to finite test duration is not considered. NOTE 2: Fulfilling the criteria for CLTA selection and placement in subclause 4.15 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate measurement uncertainty budget in TR 37.843 [28], subclause 10.6 shall be used for evaluating the test system uncertainty. |  |  |

#### 4.1.2.4	Measurement of performance requirement

The measurement uncertainties for the performance requirements are the same as those quoted in TS 36.141 [12] subclause 4.2.1.3 and TS 25.141 [10] subclause 4.1.4.

### 4.1.3	Interpretation of measurement results

The measurement results returned by the Test System are compared - without any modification - against the test requirements as defined by the Shared Risk principle.

The Shared Risk principle is defined in Recommendation ITU-R M.1545 [8].

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in subclause 4.1.2 of the present document.

If the Test System for a test is known to have a measurement uncertainty greater than that specified in subclause 4.1.2, it is still permitted to use this apparatus provided that an adjustment is made as follows.

Any additional uncertainty in the Test System over and above that specified in subclause 4.1.2 shall be used to tighten the test requirement, making the test harder to pass. (For some tests e.g. receiver tests, this may require modification of stimulus signals). This procedure (defined in annex C) will ensure that a Test System not compliant with subclause 4.1.2 does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with subclause 4.1.2 had been used.

## 4.2	Conducted and radiated requirement reference points

AAS BS requirements are defined for two points of reference, signified by radiated requirements (RIB) and conducted requirements (TAB).

Figure 4.2-1: Radiated and conducted points of reference of hybrid AAS BS

Figure 4.3-2: Radiated points of reference of OTA AAS BS

Radiated characteristics are defined over the air (OTA) at the radiated interface boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) directions in which the OTA requirements apply are detailed for each requirement.

Some OTA requirements are specified as co-location requirements where the requirements are specified at the conducted interface of the co-location reference antenna, co-location requirements are further defined in subclause 4.15

Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to receive/send parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network that distributes the RF power between the transceiver array boundary and the antenna array, in an implementation specific way.

How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement subclause.

The present document details the test requirements of the radiated requirements only and hence only requires the radiated reference points.

## 4.3	Base station classes for AAS BS

The requirements in this specification apply to AAS BS of Wide Area BS, Medium Range BS and Local Area BS classes unless otherwise stated.

The base station classes are defined in 3GPP TS 37.105 [6].

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions. Non-AAS requirements are applicable as defined in the present document. In many cases, such requirements include regional requirements that are implicitly referenced from the present specification, and listed in the specification for the specifications concerned [2] [5].

Table 4.4-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 4.6 | Operating bands and Band Categories | Some bands may be applied regionally. |
| 6.3 | OTA Base Station output power | These requirements apply in Japan for an E-UTRA BS operating in band 34 and Band 41. |
| 6.7.2 | OTA Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the Occupied bandwidth according to the definition. |
| 6.7.4 | OTA Spectrum emission mask | The mask specified may be mandatory in certain regions. In other regions this mask may not be applied. Additional spectrum protection requirements may apply regionally. |
| 6.7.5 | OTA Operating band unwanted emissions | The BS may have to comply with the applicable emission limits established by FCC Title 47 [18], when deployed in regions where those limits are applied and under the conditions declared by the manufacturer. |
| 6.7.5 | OTA Operating band unwanted emissions | The requirements for unsynchronized TDD co-existence may apply regionally. |
| 6.7.5 | OTA Operating band unwanted emissions | The requirements for protection of DTT may apply regionally. |
| 6.7.5 | OTA Operating band unwanted emissions | Regional requirement as defined in 3GPP TS 37.104, subclause 6.6.2.4.4 [5] may be applied for the protection of systems operating in frequency bands adjacent to band 1 as defined in 3GPP TS 37.104, subclause 4.5, [5] in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed. |
| 6.7.5 | OTA Operating band unwanted emissions | Additional requirements for band 41 may apply in certain regions as additional operating band unwanted emission limits. |
| 6.7.5 | OTA Operating band unwanted emissions | Additional band 32 unwanted emissions requirements may apply in certain regions |
| 6.7.6 | OTA Spurious emissions | Category A limits are mandatory for regions where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [16] apply. Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [16] apply. |
| 6.7.6 | OTA Spurious emissions | Additional spurious emissions requirements may be applied for the protection of system operating in frequency ranges other than the AAS BS operating band as described in 3GPP TS 37.104 [5] subclause 6.6.1.3 (NOTE). |
| 6.7.6 | OTA Spurious emissions | In addition to 3GPP requirements, the BS may have to comply with the applicable emission limits established by FCC Title 47 [18], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.7.6 | OTA Spurious emissions | Co-location spurious emissions requirements may be applied for the protection of other BS receivers when an MSR BS operating in another frequency band is co-located with an AAS BS. |
| 6.8 | OTA Transmitter intermodulation | Additional requirements may apply in certain regions. |
| 7.6 | OTA Blocking | Co-location blocking requirements may be applied for the protection of the BS receiver when a BS operating in another frequency band is co-located with an AAS BS. |
| 7.6 | OTA Blocking | For the Public Safety LTE BS in Korea from 718 to 728 MHz in Band 28, regional blocking requirement is specified in TS 36.104 [4], subclause 7.6.3. |
| NOTE: AAS BS does not support Band 46 operation, but additional spurious emissions requirements for Band 46 as described in 3GPP TS 37.104 [5] subclause 6.6.1.3, are still applicable for AAS BS for protection of Band 46 operation. |  |  |

| Subclause number | Requirement | Comments |
| --- | --- | --- |
| 4.5 | Operating bands and Band Categories | Some bands may be applied regionally. |
| 6.2.2.2.1 | Base station output power | These requirements apply in Japan for an E-UTRA BS operating in band 34 and Band 41. |
| 6.6.2 | Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the Occupied bandwidth according to the definition. |
| 6.6.4 | Spectrum emission mask | The mask specified may be mandatory in certain regions. In other regions this mask may not be applied. Additional spectrum protection requirements may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | The BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied and under the conditions declared by the manufacturer. |
| 6.6.5 | Operating band unwanted emissions | The requirements for unsynchronized TDD co-existence may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | The requirements for protection of DTT may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | Regional requirement as defined in TS37.104, subclause 6.6.2.4.4 [12] may be applied for the protection of systems operating in frequency bands adjacent to band 1 as defined in TS37.104, subclause 4.5, [12] in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed. |
| 6.6.5 | Operating band unwanted emissions | Additional requirements for band 41 may apply in certain regions as additional Operating band unwanted emission limits. |
| 6.6.5 | Operating band unwanted emissions | Additional band 32 unwanted emissions requirements may apply in certain regions. |
| 6.6.6 | Spurious emissions | Category A limits are mandatory for regions where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [35] apply. Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [35] apply. |
| 6.6.6 | Spurious emissions | Additional spurious emissions requirements may be applied for the protection of system operating in frequency ranges other than the AAS BS operating band as described in TS37.104 [12] subclause 6.6.1.3. |
| 6.6.6 | Spurious emissions | In addition to 3GPP requirements, the BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.6 | Spurious emissions | Co-location spurious emissions requirements may be applied for the protection of other BS receivers when an MSR BS operating in another frequency band is co-located with an AAS BS. |
| 6.7 | Transmitter intermodulation | Additional requirements may apply in certain regions. |
| 7.5 | Blocking | Co-location requirements may be applied for the protection of the BS receiver when a BS operating in another frequency band is co-located with an AAS BS. |

## 4.5	Operating bands and band categories

The operating bands and band categories for AAS BS are the same as for non-AAS BS, as described in 3GPP TS 37.104 [6].

NOTE 1:	AAS BS does not support GSM, but BC2 is still applicable for protection of/against GSM operation in BC2 operating bands.

NOTE 2:	AAS BS does not support Band 46 (and all its sub-bands defined in 3GPP TS 36.104 [4]) operation. Conducted Band 46 test requirements are still applicable for AAS BS for protection of and against Band 46 operation, as specified in 3GPP TS 37.145-1 [9].

## 4.6	Channel arrangements

The channel arrangements for AAS BS are the same as those for UTRA non-AAS BS and E-UTRA non-AAS BS as described in 3GPP TS 37.104 [5].

## 4.7	Requirements for AAS BS capable of multi-band operation

For AAS BS capable of operation in multiple operating bands, the RF requirements in clause 6 and 7 apply separately to each supported operating band unless otherwise stated.

## 4.8	AAS BS configurations

### 4.8.1	Transmit configurations

Unless otherwise stated, the radiated transmitter characteristics in clause 6 are specified at the radiated interface boundary (RIB). The AAS BS shall have a full complement of transceiver units for the configuration in normal operating conditions.

Figure 4.8.1-1: Transmitter test interfaces

Figure 4.8.1-2: Transmitter test interfaces for co-location concept

### 4.8.2	Receive configurations

Unless otherwise stated, the radiated receiver characteristics in clause 7 are specified at the radiated interface boundary (RIB). The AAS BS shall have a full complement of transceiver units for the configuration in normal operating conditions.

Figure 4.8.2-1: Receiver test interfaces

Figure 4.8.2-2: Receiver test interfaces for co-location concept

### 4.8.3	Power supply options

If the AAS BS is supplied with a number of different power supply configurations, it may not be necessary to test RF parameters for each of the power supply options, provided that it can be demonstrated that the range of conditions over which the equipment is tested is at least as great as the range of conditions due to any of the power supply configurations.

### 4.8.4	BS with integrated Iuant BS modem

Unless otherwise stated, for the tests in the present document, the integrated Iuant BS modem shall be switched OFF.

## 4.9	Capability sets

A radiated capability set is defined as the AAS BS capability to support certain RAT combinations in an operating band.

The manufacturer shall declare the supported radiated capability set(s) according to table 4.9-1 for each supported operating band.

Table 4.9-1 Radiated capability sets

| Radiated capability Set supported by the AAS BS | RCSA1 | RCSA2 | RCSA3 | RCSA3A | RCSA4 | RCSA5 |
| --- | --- | --- | --- | --- | --- | --- |
| Supported RATs | AAS BS supports MSR operation of UTRA only in the band | AAS BS supports MSR operation of E-UTRA only in the band | AAS BS supports MSR E-UTRA and UTRA in the band | AAS BS supports NR and E-UTRA MSR in the band | AAS BS supports single-RAT UTRA in the band | AAS BS supports single-RAT E-UTRA in the band |
| Supported configurations | SR UTRA (SC, MC) | SR E-UTRA (SC, MC, CA) | MR UTRA + E-UTRA SR UTRA (SC, MC) SR E-UTRA (SC, MC, CA) | MR E-UTRA + NR SR NR (SC, MC, CA) SR E-UTRA (SC, MC, CA) | SR UTRA (SC, MC) | SR E-UTRA (SC, MC, CA) |
| Applicable BC | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 |

The applicable test configurations for each RF requirement are defined in subclause 5.1, 5.2 and 5.3 for the declared radiated capability set(s). For beams with multi-band beam dependencies the applicable test configurations for each RF requirement are defined in subclause 5.4 for the declared radiated capability set(s).

NOTE:	Not every supported configuration within a capability set is tested, but the tables in subclauses 5.2, 5.3 and 5.4 provide a judicious choice among the supported configurations and test configurations to ensure proper test coverage.

## 4.10	Manufacturer declarations

The AAS BS declarations categories D9.x and D10.x listed in table 4.10-1 are required to be provided by the manufacturer for the radiated requirements testing of the hybrid AAS BS or the OTA AAS BS.

For the hybrid AAS BS declarations required for the conducted requirements testing, refer to 3GPP TS 37.145-1 [9], subclause 4.10.

NOTE 1:	D9.x declarations are related to the radiated Tx requirements, while D10.x declarations are related to the radiated Rx requirements.

NOTE 2:	From Rel-15 onwards, additional D11.x declarations are introduced in table 4.10-2 for OTA AAS BS, in order to easily distinguish from the Rel-13/14 OTA declarations which are also applicable for hybrid AAS BS. Declarations in table 4.10-2 are applicable to OTA AAS BS only.

Table 4.10-1: Hybrid AAS BS and OTA AAS BS manufacturer declarations for radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D9.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the AAS BS enclosure. |
| D9.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the AAS BS enclosure. |
| D9.3 | Beam identifier | A unique title to identify a beam. e.g. a, b, c or 1,2,3. The vendor may declare any number of beams, the minimum requirement to declare for conformance are the beams with the highest intend EIRP for each of the beams widths below: 1) Narrowest intended BeWθ, narrowest intended BeWϕ (possible when narrowest intended BeWθ) at the reference beam direction. 2) Narrowest intended BeWϕ, narrowest intended BeWθ (possible when narrowest intended BeWϕ) at the reference beam direction. 3) Widest intended BeWθ , widest intended BeWϕ (possible when widest intended BeWθ at the reference beam direction. 4) Widest intended BeWϕ – widest intended BeWθ (possible when widest intended BeWϕ) at the reference beam direction. 5) BeWθ and BeWϕ which provide highest intended EIRP of all possible beams at the reference beam direction. NOTE 1: Depending on the capability of the system some of these beams may be the same. When selecting the above five beam widths for declaration, all beams that the AAS BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. |
| D9.4 | Operating band support | UTRA or E-UTRA operating band(s) supported by AAS BS. Supported bands declared for every beam in D9.3. NOTE 2: these operating bands are related to their respective sigle-band RIBs. NOTE 3: this declaration in-directly provide information on the RAT’s supported by the AAS BS. |
| D9.5 | Beam RAT support | RAT(s) supported by each beam for each supported operating band, declared for every beam identified in D9.3. |
| D9.6 | E-UTRA channel band width support | E-UTRA channel bandwidth supported. Declared for each beam (D9.3) and each E-UTRA operating band (D9.4). |
| D9.7 | Reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam |
| D9.8 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam identified in D9.3. NOTE 4: In Rel-13/14 version of this specification, this declaration was called EIRP accuracy directions set. |
| D9.9 | Maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam identified in D9.3. |
| D9.10 | Rated beam EIRP | The rated EIRP level per carrier (PRated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared for every beam identified in D9.3. |
| D9.11 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam identified in D9.3. |
| D9.12 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D9.4-D9.11) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. |
| D9.13 | Parallel beams | List of beams which have been declared equivalent (D9.12) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. |
| D9.14 | Number of carriers at maximum TRP | The number of carriers per operating band the AAS BS is capable of generating at maximum TRP declared each RAT (and multi-RAT) for every beam identified in D9.3. |
| D9.15 | Multi-band transceiver units | Declared if an operating band is generated using transceiver units supporting operation in multiple operating bands through common active RF components. |
| D9.16 | Operating bands with multi-band dependencies | List operating bands which are generated by multi-band transceiver units. Declared for each operating band for which multi-band transceiver units (D9.15) have been declared, |
| D9.15 | Maximum radiated Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band, declared for each supported operating band identified in D9.4. |
| D9.18 | Maximum radiated Base Station RF Bandwidth for contiguous operation. | Largest Base Station RF Bandwidth for contiguous spectrum operation, declared for each supported operating band (D9.4). |
| D9.19 | Maximum radiated Base Station RF Bandwidth for non- contiguous operation. | Maximum Base Station RF Bandwidth for non-contiguous spectrum operation, declared for each supported operating band (D9.4). |
| D9.20 | Inter-band CA bands | Declared inter-band CA bands supported per operating band(D9.4). |
| D9.21 | CA only operation | Declared per operating band identified in D9.4. |
| D9.22 | Multi-carrier HSPA only operation | Declared per each supported UTRA operating band (D9.4). |
| D9.23 | Reduced number of supported carriers at maximum TRP in multi-RAT operations | Declared for each supported operating (D9.4). |
| D9.24 | Reduced maximum TRP at the total number of supported carriers in multi-RAT operations | Declared for each supported operating band (D9.4). |
| D9.25 | Radiated capability set (RCSA) | The manufacturer shall declare the supported radiated capability set(s) according to table 4.9-1 for each supported operating band (D9.4). |
| D9.26 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D9.16) |
| D9.27 | Total number of supported carriers for operating bands with multi-band dependencies | Total number of supported carriers for operating bands declared to have multi-band dependencies (D9.16). |
| D9.28 | Contiguous or non-contiguous spectrum support | Ability of AAS BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. |
| D9.29 | Non-contiguous parameters | If non-contiguous operation is supported in operating band () and parameters (e.g. frequency range, maximum Base Station RF Bandwidth, rated total output power, etc.) differ from the contiguous spectrum operation, then this declaration provided parameters for the non-contiguous operation. Otherwise, parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same. |
| D9.30 | DL RS EIRP for conformance test | The DL RS EIRP transmitted during the DL RS power conformance test derived from the power broadcast on the DL-SCH and the AAS BS directivity in the direction to be tested. |
| D9.31 | NR BS channel band width and SCS support | NR BS channel bandwidth and SCS supported. Declared for each beam () and each operating band (). |
| D9.32 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (). |
| D9.33 | Inter-band CA bands | Declared inter-band CA bands supported by the beam. Declared per beam (D.3). |
| D9.34 | CA only operation | Declared of CA-only but not multiple carriers operation, declared per operating band (D.4) and per beam (D.3). |
| D10.1 | OSDD identifier | A unique identifier for the OSDD. |
| D10.2 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD identified in D10.1. NOTE 2: As each identified OSDD has a declared minimum EIS value (D10.6), multiple operating band can be only be declared if they have the same minimum EIS declaration. |
| D10.3 | OSDD RAT support | RAT(s) supported by the OSDD for each supported operating band, declared for every OSDD identified in D10.1. NOTE 3: If the OSDD supports multiple RAT's with different minimum EIS value (D10.6) if all other parameters are the same then different EIS values for different RATS and signal BW's may be declared for an OSDD. |
| D10.4 | OTA sensitivity E-UTRA supported channel bandwidths | The E-UTRA channel bandwidths supported by each OSDD. |
| D10.5 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD |
| D10.6 | Minimum EIS | The minimum EIS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared for per RAT and E-UTRA supported channel BW for the OSDD (10.4). The lowest EIS value for all the declared OSDD’s is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. NOTE 4: If the AAS BS is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. |
| D10.7 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D10.9) for each OSDD. |
| D10.8 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD |
| D10.9 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D10.8). NOTE 5: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. |
| D10.10 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D10.11). |
| D10.11 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. |
| D10.12 | OTA sensitivity supported NR BS channel bandwidth and SCS | The NR BS channel bandwidths and SCS supported by each OSDD. |

Table 4.10-2: OTA AAS BS manufacturers declarations for radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D11.1 | AAS BS requirements set | Declaration of either hybrid AAS BS architecture conforming to the hybrid requirement set, or OTA AAS BS architecture conforming to the OTA requirement set. |
| D11.2 | BS class | BS Class of the AAS BS, declared as Wide Area BS, Medium Range BS, or Local Area BS. |
| D11.3 | OTA coverage range | Declared as a single range within which selected TX OTA requirements are intended to be met. NOTE 1: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. |
| D11.4 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D11.2). NOTE 2: The OTA coverage reference direction may be the same as the Reference beam direction pair (D9.7) but does not have to be. |
| D11.5 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. |
| D11.6 | The rated carrier OTA BS power, Pmax,c,TRP | Pmax,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band, per supported RAT. |
| D11.7 | Worst-case side of the AAS BS on which the co-location test antenna is placed | Declare the worst-case side of the AAS BS on which the co-location test antenna is placed and test will be done only on the declared side. |
| D11.8 | Spurious emission category | Declare the OTA AAS BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [16]. |
| D11.9 | Geographic area support | The manufacturer shall declare the regions the OTA AAS BS may operate in. e.g. CEPT. |
| D11.10 | Band 20 or Band XX support, operating in geographical areas allocated to broadcasting (DTT) | If the OTA AAS BS supports Band 20/XX or Band 32/XXXII, the manufacturer shall declare if the OTA AAS BS may operate in geographical areas allocated to broadcasting (DTT). |
| D11.11 | Band 20 or Band XX support, emission level for channel N (PEM,N) | If the OTA AAS BS supports Band 20 or Band XX and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the emission level for channel N (as defined in annex G of 3GPP TS 36.104 [4]) shall be declared. |
| D11.12 | Band 20 or Band XX support, Maximum output Power in 10 MHz (P10MHz) | If the OTA AAS BS supports Band 20 or Band XX and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the maximum output power in 10 MHz (annex G of 3GPP TS 36.104 [4]) shall be declared. |
| D11.13 | Band 32 or Band XXXII support, Declared emission level in Band 32/XXXII (PEM,B32,ind) | If the OTA AAS BS supports Band 32 or Band XXXII and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the emission level in Band 32/XXXII (PEM,B32,ind, ind = a, b, c, d, e) shall be declared. |
| D11.14 | Co-existence with other systems | The manufacturer shall declare whether the OTA AAS BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another operating band are deployed. |
| D11.15 | Co-location with other base stations | The manufacturer shall declare whether the OTA AAS BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD and/or E-UTRA operating in another operating band. |
| D11.16 | Single-band RIB or multi-band RIB | List of single-band RIB and/or multi-band RIB resulting from the supported operating bands (D9.4), and operating bands with multi-band dependencies (D9.16). |
| D11.17 | Single or multiple carrier | OTA AAS BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RAT, per RIB. |
| D11.18 | Maximum number of supported carriers per band | Maximum number of supported carriers. Declared per supported operating band, per RAT, per RIB. |
| D11.19 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared per RIB. |
| D11.20 | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (D9.16), which have any impact on the test configuration generation. |
| D11.21 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by an OTA AAS BS in a particular operating band. Declared per RIB (D11.13). |
| D11.22 | Maximum supported power difference between carriers | Maximum supported TRP power difference between carriers in each supported operating band. Declared per RIB. |
| D11.23 | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (D9.16, D11.16). |
| D11.24 | UTRA FDD MIMO support | Number of 'antennas' supported by the UTRA FDD MIMO mode (i.e. 2 or 4). Declared per supported UTRA FDD operating band (D9.4). NOTE 3: The concept of "antenna 2", "antenna 3" and "antenna 4" is described in 3GPP TS 25.104 [2]. |
| D11.25 | UTRA Inner loop power control dynamic range | Power control dynamic range for UTRA inner loop power control. Declared per supported UTRA FDD operating band, per RIB. |
| D11.26 | Inter-band CA or inter-band HSDPA | Declaration of operating band combinations supporting inter-band CA or multi-band HSDPA. Declared per operating band combination (D9.16, D11.16). |
| D11.27 | Intra-band contiguous CA or intra-band contiguous HSDPA | Declaration of operating band(s) supporting intra-band contiguous CA, or intra-band contiguous HSDPA. Declared per operating band with CA support. |
| D11.28 | Intra-band non-contiguous CA or intra-band contiguous HSDPA | Declaration of operating band(s) supporting intra-band non-contiguous CA, or intra-band non-contiguous HSDPA. Declared per operating band with CA support. |
| D11.29 | OTA REFSENS RoAoA | The REFSENS RoAoA associated with the receiver target reference direction (D11.30). |
| D11.30 | OTA REFSENS receiver target reference direction | An associated direction inside the OTA REFSENS RoAoA (D11.29). |
| D11.31 | OTA REFSENS conformance test directions | Four conformance test directions for the OTA REFSENS: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. |
| D11.32 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. |
| D11.33 | Rated beam EIRP at lower frequency range of the fractional bandwidth (Prated,c,FBWlow ) | The rated EIRP level per carrier at lower frequency range of the fractional bandwidth (Prated,c,FBWlow ), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared per beam for all supported frequency ranges (D11.32). NOTE 13: if D11.33 is declared for certain frequency range (D11.32), there shall be no “Rated beam EIRP” declaration (D9.10) for the operating band containing that particular frequency range. |
| D11.34 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh ) | The rated EIRP level per carrier at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared per beam for all supported frequency ranges in (D11.32). NOTE 14: if D11.34 is declared for certain frequency range (D11.32), there shall be no “Rated beam EIRP” declaration (D9.10) for the operating band containing that particular frequency range. |

## 4.11	Test signal configurations for testing

### 4.11.1	General

The test configurations shall be constructed using the methods defined below subject to the parameters declared by the manufacturer as listed in subclause 4.10.

For test contiguous spectrum operation configurations used in receiver tests only the carriers in the outermost frequency positions in the Base Station RF Bandwidth need to be generated by the test equipment. For non-contiguous spectrum operation test configurations used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment.

The applicable test models for generation of the carrier transmit test signal are defined in subclause 4.12.2.

NOTE:	In case carriers are shifted to align with the channel raster Foffset.

### 4.11.1A	NR Test signal used to build Test Configurations

The signal’s Channel Bandwidth and Subcarrier spacing used to build NR Test Configurations shall be selected according to table 4.11.1A-1.

Table 4.11.1A-1: Signal to be used to build NR TCs

| Operating Band characteristics |  | FDL_high – FDL_low < 100 MHz | FDL_high – FDL_low ≥ 100 MHz |
| --- | --- | --- | --- |
| TC signal characteristics | BWchannel | 5 MHz (Note 1) | 20 MHz (Note 1) |
|  | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| Note 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

### 4.11.2	Test signal configurations

#### 4.11.2.1	ATCR1: UTRA multicarrier operation

##### 4.11.2.1.1	General

The purpose of ATCR1 is to test UTRA OTA multi-carrier aspects.

##### 4.11.2.1.2	ATCR1a generation

ATCR1a should be constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum radiated Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D9.18).

-	Place one UTRA FDD carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA FDD carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset shall apply.

-	For transmitter tests, alternately place a UTRA FDD carrier adjacent to the already placed carriers at the low and high Base Station RF Bandwidth edges until there is no more space to fit a carrier or the beam does not support more carriers. The nominal carrier spacing defined in subclause 4.6 shall apply.

-	The carrier(s) may be shifted maximum 100 kHz towards lower frequencies for BRFBW and MRFBW and towards higher frequencies for TRFBW to align with the channel raster.

##### 4.11.2.1.3	ATCR1b generation

ATCR1b is constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum radiated Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D6.20).

-	Place one UTRA TDD carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA TDD carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset shall apply.

-	For transmitter tests, alternately place a UTRA TDD carrier adjacent to the already placed carriers at the low and high Base Station RF Bandwidth edges until there is no more space to fit a carrier or the beam does not support more carriers. The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.1.4	ATCR1 power allocation

Set the number of carriers to the number of carriers at maximum TRP (see table 4.10-1, D9.14).

For EIRP accuracy requirements set each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

#### 4.11.2.2	ANTCR1: UTRA FDD multicarrier non-contiguous operation

##### 4.11.2.2.1	General

The purpose of ANTCR1 is to test UTRA FDD multicarrier non-contiguous aspects.

##### 4.11.2.2.2	ANTCR1 generation

ANTCR1 is constructed as NTC1a in 3GPP TS 37.141 [13], subclause 4.8.1a.1

ANTCR1is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D6.21). The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth for non-contiguous operation.

-	For transmitter tests, place one UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	For receiver tests, place one UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. For single-band operation, if the maximum Base Station RF Bandwidth for non-contiguous operation is at least 35 MHz and the beam supports at least 4 UTRA FDD carriers, place a UTRA FDD carrier adjacent to each already placed carrier for each sub-block. The nominal carrier spacing defined in subclause 4.6 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset-RAT for the carrier adjacent to the sub-block gap.

-	The UTRA FDD carrier in the lower sub-block may be shifted maximum100 kHz towards lower frequencies and the UTRA FDD carrier in the upper sub-block may be shifted maximum100 kHz towards higher frequencies to align with the channel raster.

##### 4.11.2.2.3	ANTCR1 power allocation

Set the number of carriers to the number of carriers at maximum TRP (see table 4.10-1, D9.14).

For EIRP accuracy requirements set each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

#### 4.11.2.3	ATCR2: E-UTRA multicarrier operation

##### 4.11.2.3.1	General

The purpose of ATCR2a is to test E-UTRA multi-carrier aspects excluding CA occupied bandwidth.

The purpose of ATCR2b is to test E-UTRA Contiguous CA occupied bandwidth.

##### 4.11.2.3.2	ATCR2a generation

ATCR2a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D9.18).

-	Select the narrowest supported E-UTRA carrier and place it adjacent to the low Base Station RF Bandwidth edge. Place a 5 MHz E-UTRA carrier adjacent to the high Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	For transmitter tests, select as many 5 MHz E-UTRA carriers that the beam supports and that fit in the rest of the Base Station RF Bandwidth. Place the carriers adjacent to each other starting from the high Base Station RF Bandwidth edge. The nominal carrier spacing defined in subclause 4.6 shall apply. The specified FOffset-RAT shall apply.

-	If 5 MHz E-UTRA carriers are not supported by the beam the narrowest supported channel bandwidth (see table 4.10-1, D9.6) shall be selected instead.

The test configuration should be constructed on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the beam (see table 4.10-1, D9.20). All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

##### 4.11.2.3.3	ATCR2b generation

ATCR2b is constructed on a per band basis using the following method:

-	All component carrier combinations supported by the beam, which have different sum of channel bandwidth of component carrier, shall be tested. For all component carrier combinations which have the same sum of channel bandwidth of component carriers, only one of the component carrier combinations shall be tested.

-	Of all component carrier combinations which have same sum of channel bandwidth of component carrier, select those with the narrowest carrier at the lower Base Station RF Bandwidth edge.

-	Of the combinations selected in the previous step, select one with the narrowest carrier at the upper Base Station RF Bandwidth edge.

-	If there are multiple combinations fulfilling previous steps, select the one with the smallest number of component carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the lowest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the highest carrier

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the carrier which has been selected in the previous step.

-	If there are multiple combinations fulfilling previous steps, repeat the previous step until there is only one combination left.

-	The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.3.4	ATCR2 power allocation

Set the number of carriers to the number of carriers at maximum TRP (see table 4.10-1, D9.14).

For EIRP accuracy requirements set each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

For a beam declared to support only CA operation (see table 4.10-1, D6.23), set the power spectral density of of each carrier to the same level so that the sum of the carrier power equals the same value as above.

#### 4.11.2.4	ANTCR2: E-UTRA multicarrier non-contiguous operation

##### 4.11.2.4.1	General

The purpose of ANTCR2 is to test E-UTRA multicarrier non-contiguous aspects.

##### 4.11.2.4.2	ANTCR2 generation

ANTCR2 is constructed as NTC2 in 3GPP TS 37.141 [13], subclause 4.8.2a.1

ANTCR2 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D9.19). The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum radiated Base Station RF Bandwidth (see table 4.10-1, D9.17).

-	For transmitter tests, place a 5MHz E-UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and a 5MHz E-UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the beam, the narrowest supported channel bandwidth shall be selected instead.

-	For receiver tests, place a 5MHz E-UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and a 5MHz E-UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. If 5 MHz E-UTRA carriers are not supported by the beam, the narrowest supported channel bandwidth shall be selected instead.

-	For single-band operation receiver tests, if the remaining gap is at least 15 MHz plus two times the channel bandwidth used in the previous step and the beam supports at least 4 E-UTRA carriers, place a E-UTRA carrier of this channel bandwidth adjacent to each already placed carrier for each sub-block. The nominal carrier spacing defined in subclause 4.5 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset-RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.4.3	ANTCR2 power allocation

Set the number of carriers to the number of carriers at maximum EIRP (see table 4.10-1, D9.14).

For EIRP accuracy requirementsset each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

#### 4.11.2.5	ATCR3: UTRA and E-UTRA multi RAT operation

##### 4.11.2.5.1	General

The purpose of ATCR3 is to test UTRA and E-UTRA multi-RAT aspects.

If the maximum EIRP and total number of supported carriers at maximum EIRP are not simultaneously supported in Multi-RAT operations, two instances of ATCR3 shall be generated using the following values for rated total output power and the total number of supported carriers:

1)	The maximum EIRP and the reduced number of supported carriers at the maximum EIRP in Multi-RAT operations.

2)	The reduced maximum EIRP at the total number of supported carriers in Multi-RAT operations and the total number of supported carriers.

Tests that use ATCR3 shall be performed using both instances 1) and 2) of ATCR3.

##### 4.11.2.5.2	ATCR3a generation

ATCR3a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth (see table 4.10-1 D9.17).

-	Select an FDD UTRA carrier to be placed at the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	Place a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. If that is not possible use the narrowest E-UTRA carrier supported by the beam. The specified FOffset-RAT shall apply.

-	For transmitter tests, alternately add FDD UTRA carriers at the low end and 5 MHz E-UTRA carriers at the high end adjacent to the already placed carriers until the Base Station RF Bandwidth is filled or the total number of supported carriers (see table 4.10-1, D9.14) is reached. The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.5.3	ATCR3b generation

ATCR3b is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth (see table 4.10-1 D9.17).

-	Select a UTRA TDD carrier to be placed at the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	Place a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. If that is not possible use the narrowest E-UTRA carrier supported by the beam. The specified FOffset-RAT shall apply.

-	For transmitter tests, alternately add UTRA TDD carriers at the low end and 5 MHz E-UTRA carriers at the high end adjacent to the already placed carriers until the Base Station RF Bandwidth is filled or the total number of supported carriers is reached. The nominal carrier spacing defined in clause 4.6 shall apply.

##### 4.11.2.5.4	ATCR3 power allocation

For ATCR3a set the number of carriers to the reduced number of carriers at maximum TRP in multi-RAT operations (see table 4.10-1, D9.23) and set each carrier to maximum EIRP (see table 4.10-1, D9.11).

For EIRP accuracy requirementsset each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

#### 4.11.2.6	ANTCR3: UTRA and E-UTRA multi RAT non-contiguous operation

##### 4.11.2.6.1	General

The purpose of ANTCR3 is to test UTRA and E-UTRA multi RAT non-contiguous aspects.

If the maximum EIRP and total number of supported carriers at maximum EIRP are not simultaneously supported in Multi-RAT operations, two instances of ANTCR3 shall be generated using the following values for rated total output power and the total number of supported carriers:

1)	The maximum EIRP and the reduced number of supported carriers at the maximum EIRP in Multi-RAT operations.

2)	The reduced maximum EIRP at the total number of supported carriers in Multi-RAT operations and the total number of supported carriers.

If the reduced number of supported carriers is 4 or more, only instance 1) of ANTCR3 shall be used in the tests, otherwise both instances 1) and 2) of ANTCR3 shall be used in the tests.

##### 4.11.2.6.2	ANTCR3a generation

ANTCR3a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D6.21). The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth for non-contiguous operation.

-	For transmitter tests, place an UTRA carrier at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the beam, the narrowest supported channel bandwidth shall be selected instead. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	For receiver tests, place an UTRA carrier at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the beam, the narrowest supported channel bandwidth shall be selected instead. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	For single-band operation receiver tests, if the remaining gap is at least 20 MHz plus the channel bandwidth of the E-UTRA carrier used in the previous step and the beam supports at least 2 UTRA and 2 E-UTRA carriers, place a E-UTRA carrier of this channel bandwidth adjacent to the carrier at the lower Base Station RF Bandwidth edge and UTRA carrier adjacent to the carrier at the upper Base Station RF Bandwidth edge. The nominal carrier spacing defined in subclause 4.6 shall apply. The UTRA FDD may be shifted maximum 100 kHz towards higher frequencies to align with the channel raster.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset-RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.6.3	ANTCR3 power allocation

For case (1) in subclause 4.11.2.6.1 set the number of carriers to the reduced  number of carriers at maximum TRP in multi-RAT operations (see table 4.10-1, D9.23).

For EIRP accuracy requirements set each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested  beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

For case (2) in subclause 4.11.2.6.1  set the number of carriers to the reduced  number of carriers at maximum TRP (see table 4.10-1, D9.14) and set each carrier to the reduced maximum TRP at the total number of supported carriers in Multi-RAT operations (see table 4.10-1, D9.24) for the tested beam direction pair.

#### 4.11.2.7	ATCR4: Single carrier for receiver tests

##### 4.11.2.7.1	ATCR4a generation

ATCR4a is constructed using the following method:

-	Place a single (UTRA FDD) carrier in the middle of the maximum radiated Base Station RF Bandwidth. The carrier may be shifted maximum 100 kHz towards lower frequencies for BRFBW and MRFBW and towards higher frequencies for TRFBW to align with the channel raster.

##### 4.11.2.7.2	ATCR4b generation

ATCR4b is constructed using the following method:

-	Place the narrowest supported E-UTRA carrier in the middle of the maximum radiated Base Station RF Bandwidth.

##### 4.11.2.7.3	ATCR4c generation

ATCR4c is constructed using the following method:

-	Place a single UTRA TDD carrier in the middle of the maximum radiated Base Station RF Bandwidth.

##### 4.11.2.7.3A	ATCR4d generation

ATCR4d is constructed using the following method:

-	Place a single NR carrier as specified in subclause 4.11.1A in the middle of the maximum radiated Base Station RF Bandwidth.

##### 11.2.7.4	ATCR4 power allocation

Set the beam EIRP on the carrier such that it's EIRP level is equal to the sum of rated beam EIRPs (see table 4.10-1, D9.12) when transmitting the maximum supported carriers at the beam peak direction (see table 4.10-1, D9.16).

#### 4.11.2.8	Generation of MB-MSR test configurations

##### 4.11.2.8.1	ATCR5a: MB-MSR test configuration for full carrier allocation

###### 4.11.2.8.1.1	General

The purpose of ATCR5a is to test beams which have been generated using transceiver units supporting operation in multiple operating bands through common active electronic components(s), considering maximum supported number of carriers.

###### 4.11.2.8.1.2	ATCR5a generation

ATCR5a is based on re-using the existing test configurations applicable per band on beams generated using Multi-band transceiver units and hence have declared multi-band dependencies (see table 4.10-1, D9.16). ATCR5a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth (see table 4.10-1, D9.17).

-	The number of carriers of each supported operating band shall be the declared maximum number of supported carriers by the multi-band dependencies in each band (see table 4.10-1, D9.16). Carriers shall first be placed at the outermost edges of the declared maximum radiated Radio Bandwidth (see table 4.10-1, D9.17). Additional carriers shall next be placed at the edges of the Base Station RF Bandwidths, if possible.

-	The allocated Base Station RF Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum radiated Radio Bandwidth (see table 4.10-1, D9.17).

-	Each concerned band shall be considered as an independent band and the corresponding test configuration shall be generated in each band. The mirror image of the single band test configuration shall be used in the highest band being tested for the beam.

-	Band category and declared per band capability set (see table 4.10-1, D9.25) shall be used to generate per band RAT/carrier allocation according to table 4.11.2.8.1.2-1 for each band category and radiated capability set. If an operating band with multi-band dependencies supports three carriers only, two carriers shall be placed in one band according to the relevant test configuration while the remaining carrier shall be placed at the edge of the maximum Radio Bandwidth (DUID9) in the other band.

-	If the sum of the base Station RF bandwidths of each of the supported operating bands is greater than the declared maximum Radio Bandwidth of the operating band with multi-band dependencies (see table 4.10-1, D9.26, D9.16) then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared maximum Radio Bandwidth is not exceeded and vice versa.

-	If the sum of the maximum number of supported carrier of each supported operating bands with multi-band dependencies (see table 4.10-1, D9.16) is larger than the declared t Total number of supported carriers for operating bands with multi-band dependencies (see table 4.10-1, D9.27), repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not be exceeded and vice versa.

Table 4.11.2.8.1.2-1: The applicability of test configuration in each band

| BC | RCSA 1 | RCSA 2 | RCSA 3 | RCSA 3A | RCSA 4 | RCSA 5 |
| --- | --- | --- | --- | --- | --- | --- |
| BC1 | ATCR1a | ATCR2a | ATCR3a | ATCR7 | ATCR1a | ATCR2a |
| BC2 | ATCR1a | ATCR2a | ATCR3a | ATCR7 | ATCR1a | ATCR2a |
| BC3 | ATCR1b | ATCR2a | ATCR3b | ATCR7 | ATCR1b | ATCR2a |

###### 4.11.2.8.1.3	ATCR5a power allocation

Set the number of carriers to the total number of supported carriers for the declared multi-band dependencies (see table 4.10-1, D9.27).

For EIRP accuracy requirements set each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

If the allocated number of carriers in an operating band exceeds the declared number of carriers at maximum TRP in an operating band (see table 4.10-1, D9.14) the carriers should if possible be allocated to a different operating band.

##### 4.11.2.8.2	ATCR5b: MB-MSR test configuration with high PSD per carrier

###### 4.11.2.8.2.1	General

The purpose of ATCR5b is to test multi-band operation aspects considering higher PSD cases with reduced number of carriers and non-contiguous operation (if supported) in multi-band mode.

###### 4.11.2.8.2.2	ATCR5b generation

ATCR5b is based on re-using the existing test configurations applicable for operating bands using multi-band transceiver units and hence have declared multi-band dependencies (see table 4.10-1, D9.16). ATCR5b is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth (see table 4.10-1, D9.17).

-	The allocated Radio Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth of the operating band with multi-band dependencies (see table 4.10-1, D9.26).

-	The maximum number of carriers is limited to two per band.  Carriers shall be placed at the outermost edges of the declared maximum Radio Bandwidth of the operating band with multi-band dependencies (see table 4.10-1, D9.26).

-	Each concerned band shall be considered as an independent band and the corresponding test configuration for non-contiguous operation shall be generated in each band according to table 4.11.2.8.2.2-1. Narrowest supported E-UTRA channel bandwidth shall be used in the test configuration. The mirror image of the single band test configuration shall be used in the highest band being tested.

-	For AAS BS supporting CSA4 in the band and supports three carriers only, two carriers shall be placed in one band according to AUTC2 while the remaining carrier shall be placed at the edge of the Maximum Base Station RF Bandwidth in the other band.

-	If the sum of the base Station RF bandwidths of each of the supported operating bands is greater than the declared maximum Radio Bandwidth of the operating band with multi-band dependencies (see table 4.10-1, D9.26) for the declared multi-band dependencies (see table 4.10-1, D9.16) then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared maximum Radio Bandwidth of the operating band with multi-band dependencies (see table 4.10-1, D9.26) is not exceeded and vice versa.

Table 4.11.2.8.2.2-1: The applicability of test configuration in each band

| BC | RCSA 1 | RCSA 2 | RCSA 3 | RCSA 3A | RCSA 4 | RCSA 5 |
| --- | --- | --- | --- | --- | --- | --- |
| BC1 | ANTCR1a | ANTCR2 | ANTCR3a | ANTCR7 | ANTCR1 | ANTCR2 |
| BC2 | ANTCR1a | ANTCR2 | ANTCR3a | ANTCR7 | ANTCR1 | ANTCR2 |
| BC3 | ATCR1b | ANTCR2 | ANTCR3a | ANTCR7 | N/A | ANTCR2 |

###### 4.11.2.8.2.3	ATCR5b power allocation

Set the number of carriers to the total number of supported carriers for the declared multi-band dependencies (see table 4.10-1, D9.27).

For EIRP accuracy requirements set each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

If the sum of the TRP for all carriers in an operating band(s) exceeds the sum of the maximum TRP per carrier (see table 4.10-1, D9.14) for the number of carriers transmitted in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the TRP allocated for a carrier exceeds the declared maximum TRP, the exceeded power shall, if possible, be reallocated into the other carriers.

#### 4.11.2.9	ATCR6: Single carrier for Transmitter tests

##### 4.11.2.9.1	ATCR6a generation

ATCR6a is constructed using the following method:

-	Place a single UTRA carrier at the RF channel to be tested.

##### 4.11.2.9.2	ATCR6b generation

ATCR6b is constructed using the following method:

-	Place a 5 MHz E-UTRA carrier i at the RF channel to be tested. If 5 MHz carriers are not supported by the beam the narrowest supported channel BW shall be selected instead.

##### 4.11.2.9.3	Void

##### 4.11.2.9.3A	ATCR6d generation

ATCR6d is constructed using the following method:

##### -	Place a single NR carrier as specified in subclause 4.11.1A at the RF channel to be tested.

##### 4.11.2.9.4	ATCR6 power allocation

Set the number of carriers to 1. Set the beam parameters to those appropriate for the beam identifier of the beam under test and to the direction to be tested from the beam declarations (see table 4.10-1, D9.3 - 13).

#### 4.11.2.10	ATCR7: E-UTRA and NR multi RAT operation

##### 4.11.2.10.1	General

The purpose of ATCR7 is to test E-UTRA and NR multi-RAT aspects.

If the maximum EIRP and total number of supported carriers at maximum EIRP are not simultaneously supported in Multi-RAT operations, two instances of ATCR7 shall be generated using the following values for rated total output power and the total number of supported carriers:

1)	The maximum EIRP and the reduced number of supported carriers at the maximum EIRP in Multi-RAT operations.

2)	The reduced maximum EIRP at the total number of supported carriers in Multi-RAT operations and the total number of supported carriers.

## Tests that use ATCR7 shall be performed using both instances 1) and 2) of ATCR7.

##### 4.11.2.10.2	ATCR7 generation

ATCR7 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth (see table 4.10-1 D9.17).

-	Select a NR carrier as specified in subclause 4.11.1A to be placed at the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	Place a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. If that is not possible use the narrowest E-UTRA carrier supported by the beam. The specified FOffset-RAT shall apply.

-	For transmitter tests, alternately add NR carriers as specified in subclause 4.11.1A at the low end and 5 MHz E-UTRA carriers at the high end adjacent to the already placed carriers until the Base Station RF Bandwidth is filled or the total number of supported carriers (see table 4.10-1, D9.14) is reached. The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.10.3	ATCR7 power allocation

Set the number of carriers to the reduced  number of carriers at maximum TRP in multi-RAT operations (see table 4.10-1, D9.23) and set each carrier to maximum EIRP (see table 4.10-1, D9.11).

For EIRP accuracy requirementsset each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

#### 4.11.2.11	ANTCR7: E-UTRA and NR multi RAT non-contiguous operation

##### 4.11.2.11.1	General

The purpose of ANTCR7 is to test E-UTRA and NR multi RAT non-contiguous aspects.

If the maximum EIRP and total number of supported carriers at maximum EIRP are not simultaneously supported in Multi-RAT operations, two instances of ANTCR7 shall be generated using the following values for rated total output power and the total number of supported carriers:

1)	The maximum EIRP and the reduced number of supported carriers at the maximum EIRP in Multi-RAT operations.

2)	The reduced maximum EIRP at the total number of supported carriers in Multi-RAT operations and the total number of supported carriers.

If the reduced number of supported carriers is 4 or more, only instance 1) of ANTCR7 shall be used in the tests, otherwise both instances 1) and 2) of ANTCR7 shall be used in the tests.

##### 4.11.2.11.2	ANTCR7 generation

ANTRC7 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D6.21). The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth for non-contiguous operation.

-	For transmitter tests, place an NR carrier as specified in subclause 4.11.1A at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the beam, the narrowest supported channel bandwidth shall be selected instead.

-	For receiver tests, place a NR carrier as specified in subclause 4.11.1A at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the beam, the narrowest supported channel bandwidth shall be selected instead.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset-RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.11.3	ANTCR7 power allocation

For case (1) in subclause 4.11.2.6.1 set the number of carriers to the reduced  number of carriers at maximum TRP in multi-RAT operations (see table 4.10-1, D9.23).

For EIRP accuracy requirements set each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested  beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

For case (2) in subclause 4.11.2.6.1  set the number of carriers to the reduced  number of carriers at maximum TRP (see table 4.10-1, D9.14) and set each carrier to the reduced maximum TRP at the total number of supported carriers in Multi-RAT operations (see table 4.10-1, D9.24) for the tested beam direction pair.

#### 4.11.12.3	ATCR8: NR multicarrier operation

##### 4.11.2.12.1	General

The purpose of ATCR8a is to test NR multi-carrier aspects excluding CA occupied bandwidth.

The purpose of ATCR8b is to test NR Contiguous CA occupied bandwidth.

##### 4.11.2.12.2	ATCR8a generation

ATCR8 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared radiated Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D9.18).

-	Select the NR carrier as specified in subclause 4.11.1A and place it adjacent to the low Base Station RF Bandwidth edge. Place a similar NR carrier adjacent to the high Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	For transmitter tests, select as many similar NR carriers that the beam supports and that fit in the rest of the Base Station RF Bandwidth. Place the carriers adjacent to each other starting from the high Base Station RF Bandwidth edge. The nominal carrier spacing defined in subclause 4.6 shall apply. The specified FOffset-RAT shall apply.

The test configuration should be constructed on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the beam (see table 4.10-1, D9.20). All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

##### 4.11.2.3.3	ATCR8b generation

ATCR8b is constructed on a per band basis using the following method:

-	All component carrier combinations supported by the beam, which have different sum of channel bandwidth of component carrier, shall be tested. For all component carrier combinations which have the same sum of channel bandwidth of component carriers, only one of the component carrier combinations shall be tested.

-	Of all component carrier combinations which have same sum of channel bandwidth of component carrier, select those with the narrowest carrier at the lower Base Station RF Bandwidth edge.

-	Of the combinations selected in the previous step, select one with the narrowest carrier at the upper Base Station RF Bandwidth edge.

-	If there are multiple combinations fulfilling previous steps, select the one with the smallest number of component carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the lowest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the highest carrier

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the carrier which has been selected in the previous step.

-	If there are multiple combinations fulfilling previous steps, repeat the previous step until there is only one combination left.

-	The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.12.4	ATCR8 power allocation

Set the number of carriers to the number of carriers at maximum TRP (see table 4.10-1, D9.14).

For EIRP accuracy requirements set each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

For a beam declared to support only CA operation (see table 4.10-1, D6.23), set the power spectral density of of each carrier to the same level so that the sum of the carrier power equals the same value as above.

#### 4.11.2.13	ANTCR8: NR multicarrier non-contiguous operation

##### 4.11.2.13.1	General

The purpose of ANTCR8 is to test NR multicarrier non-contiguous aspects.

##### 4.11.2.13.2	ANTCR8 generation

ANTCR8 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum radiated Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D9.19). The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum radiated Base Station RF Bandwidth (see table 4.10-1, D9.17).

-	For transmitter tests, place a NR carrier as specified in subclause 4.11.1A adjacent to the upper Base Station RF Bandwidth edge and a similar NR carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	For receiver tests, place a NR carrier as specified in subclause 4.11.1A adjacent to the upper Base Station RF Bandwidth edge and a similar NR carrier adjacent to the lower Base Station RF Bandwidth edge. -	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset-RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.13.3	ANTCR8 power allocation

Set the number of carriers to the number of carriers at maximum EIRP (see table 4.10-1, D9.14).

For EIRP accuracy requirementsset each beam to maximum EIRP (see table 4.10-1, D9.10) for the tested beam direction pair.

## For all other requirements ensure the total radiated power is PRated,c,TRP (see table 4.10-2, D11.6).

## 4.12	RF channels and test models

### 4.12.1	RF channels

For single carrier tests unless otherwise stated the tests shall be performed with a single carrier at each of the RF channels B, M and T.

Many tests in this TS are performed with the maximum radiated Base Station RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BRFBW (bottom), MRFBW (middle) and TRFBW (top).

-	Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum Base Station RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW: maximum Base Station RF Bandwidth located in the middle of the supported frequency range in the operating band. MRFBW may be shifted maximum 100 kHz towards lower frequencies to align carriers with the channel raster.

-	TRFBW: maximum Base Station RF Bandwidth located at the top of the supported frequency range in the operating band.

For the test of certain RF requirements the present specification refers to test procedures defined in the single-RAT specifications [2], [3], [4], [5]. In this case, the interpretation of the RF channels to be tested shall be according to the definitions in the corresponding single-RAT specifications [2], [3], [4], [5].

For an operating band which has multi-band beam dependencies capable of dual-band operation, unless otherwise stated, the test shall be performed at BRFBW_T'RFBW and B'RFBW_TRFBW defined as following:

-	BRFBW_ T'RFBW: the Base Station RF Bandwidths located at the bottom of the supported frequency range in the lower operating band and at the highest possible simultaneous frequency position, within the maximum Radio Bandwidth, in the upper operating band.

-	B'RFBW_TRFBW: the Base Station RF Bandwidths located at the top of the supported frequency range in the upper operating band and at the lowest possible simultaneous frequency position, within the maximum Radio Bandwidth, in the lower operating band.

NOTE:	BRFBW_T'RFBW = B'RFBW_TRFBW = BRFBW_TRFBW when the declared maximum radiated Radio Bandwidth (see table 4.10-1, D9.17) spans both operating bands. BRFBW_TRFBW means the Base Station RF Bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band.

When a test is performed by a test laboratory, the position of BRFBW, MRFBW and TRFBW in each supported operating band, the position of BRFBW_T'RFBW and B'RFBW_TRFBW in the supported operating band combinations shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

### 4.12.2	Test models

a) Unless otherwise stated, carriers used for transmitter tests shall be configured as follows:

-	UTRA FDD carriers shall be configured according to TM1 as defined in TS 25.141 [10], subclause 6.1.1.1.

-	UTRA TDD carriers shall be configured according to table 6.1A as defined in TS 25.142 [11], subclause 6.2.4.1.2.

-	E-UTRA carriers shall be configured according to E-TM1.1 as defined in TS 36.141 [12], subclause 6.1.1.1. For BC3 RCS3 BS testing, E-UTRA carriers shall be configured according to E-TM1_BC3CS3 defined in annex E of TS 37.141 [13].

-	NR carriers shall be configured according to NR-FR1-TM1 as defined in 3GPP TS 38.141-2 [34], subclause  4.9.2.2. For BC3 BS testing, NR carriers shall be configured according to NR-FR1-TM1_CS3 defined in annex E of 3GPP TS 37.141 [13].

b)	The configuration of the carriers in test configurations used for testing modulation quality and frequency error shall be as follows:

-	For the case that modulation accuracy is measured for UTRA FDD, the UTRA FDD carriers shall be configured according to the supported TM1, TM4 and if HS-PDSCH transmission using 16QAM is supported also TM5 as defined in TS 25.141 [10], subclauses 6.1.1.1, 6.1.1.4 and 6.1.1.4A whilst any remaining carriers from other RAT(s) shall be configured according to a).

-	For the case that modulation accuracy is measured for E-UTRA, the E-UTRA carriers shall be configured according to the supported E-TM3.1, E-TM3.2, E-TM3.3 and E-TM2 as defined in TS 36.141 [12] subclauses 6.1.1.4, 6.1.1.5, 6.1.1.6 and 6.1.1.3 whilst any remaining carriers from other RAT(s) shall be configured according to a). For BC3 CS3 BS testing, E-UTRA carriers shall be configured according to E-TM3.1_BC3CS3, E-TM3.2_BC3CS3, E-TM3.3_BC3CS3 and E-TM2_BC3CS3 defined in annex E of TS 37.141 [13].

-	For the case that modulation accuracy is measured for NR, the NR carriers shall be configured according to the supported NR-FR1-TM3.1, NR-FR1-TM3.1a, NR-FR1-TM3.2, NR-FR1-TM3.3, NR-FR1-TM2 and NR-FR1-TM2a as defined in 3GPP TS 38.141-2 [34] subclause 4.9.2.2 whilst any remaining carriers from other RAT(s) shall be configured according to a). For BC3 BS testing, NR carriers shall be configured according to NR-FR1-TM3.1_BC3, NR-FR1-TM3.1a_BC3, NR-FR1-TM3.2_BC3, NR-FR1-TM3.3_BC3, NR-FR1-TM2_BC3 and NR-FR1-TM2a_BC3 defined in annex E of 3GPP TS 37.141 [13].

c) Unless otherwise stated, transmitter carriers used for receiver tests shall be configured as follows:

-	UTRA FDD carriers shall be configured according to TM1 as defined in TS 25.141 [10], subclause 6.1.1.1.

-	UTRA TDD carriers shall be configured according to table 6.1A as defined in TS 25.142 [11], subclause 6.2.4.1.2.

-	E-UTRA carriers shall be configured according to E-TM1.1 as defined in TS 36.141 [12], subclause 6.1.1.1. For BC3 CS3 BS testing, E-UTRA carriers shall be configured according to E-TM1_BC3CS3 defined in annex E of TS 37.141 [13].

-	NR carriers shall be configured according to NR-FR1-TM1 as defined in 3GPP TS 38.141-1 [37], subclause 4.9.2.2. For BC3 BS testing, NR carriers shall be configured according to NR-FR1-TM1_CS3 defined in annex E of 3GPP TS 37.141 [16].3GPP TS 37.141 [13].

For the test of certain RF requirements clause 5 refers to the test configurations as defined in the single-RAT specifications. In this case, the transmitter test signals and test models as defined within the referred test specification for the RF requirement shall be used.

## 4.13	Format and interpretation of tests

Each test in the following clauses has a standard format:

X	Title

All tests are applicable to all equipment within the scope of the present document, unless otherwise stated.

X.1	Definition and applicability

This subclause gives the general definition of the parameter under consideration and specifies whether the test is applicable to all equipment or only to a certain subset. Required manufacturer declarations may be included here.

X.2	Minimum requirement

This subclause contains the reference to the subclause to the 3GPP reference (or core) specification which defines the minimum requirement. For each requirement, there are separate references for MSR and single RAT, where applicable in the core requirement. If the requirement does not apply to a particular RAT, this is explicitly stated here (rather than through a reference).

X.3	Test purpose

This subclause defines the purpose of the test.

X.4	Method of test

X.4.1	General

In some cases there are alternative test procedures or initial conditions. In such cases, guidance for which initial conditions and test procedures can be applied are stated here. In the case only one test procedure is applicable, that is stated here.

X.4.2y	First test method

X.4.2y.1	Initial conditions

This subclause defines the initial conditions for each test, including the test environment, the RF channels to be tested and the basic measurement set-up. The test system is assumed to be correctly calibrated as part of the initial conditions. Calibration is not explicitly mentioned.

X.4.2y.2	Procedure

This subclause describes the steps necessary to perform the test and provides further details of the test definition like point of access (e.g. test port), domain (e.g. frequency-span), range, weighting (e.g. bandwidth), and algorithms (e.g. averaging). The procedure may comprise data processing of the measurement result before comparison with the test requirement (e.g. average result from several measurement positions).

X.4.3y	Alternative test method (if any)

If there are alternative test methods, each is described with its initial conditions and procedures.

X.5	Test requirement

This subclause defines the pass/fail criteria for the equipment under test, see subclause 4.1.3 Interpretation of measurement results. Test requirements for every minimum requirement referred in subclause X.2 are listed here. Cases where minimum requirements do not apply need not be mentioned.

The test requirements may be different depending on the test method applied. A test requirement for each test method applicable to the respective MSR/Single RAT requirement is given in separate subclauses where applicable.

## 4.14	Reference coordinate system

Radiated requirements are stated in terms of electromagnetic characteristics (e.g. EIRP and EIS) at certain angles with respect to the base station. To be able to declare radiated characteristics part of radiated requirements a reference coordinate system is required. The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure.. The location of the origin and the orientation of the reference coordinate system are for the base station manufacturer to declare.

The reference coordinate system is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () as showed in figure 4.14-1.

Figure 4.14-1: Reference coordinate system

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°, inclusive. Note that  is defined as positive along the down-tilt angle.

## 4.15	Co-location requirements

### 4.15.1	General

Co-location requirements are requirements which are based on assuming the AAS BS is co-located with another BS of the same base station class. They ensure that both co-located systems can operate with minimal degradation to each other.

The co-location requirements in Table 4.15-1-1 rely on a co-location reference antenna used to mimic a base station to base station co-location scenario.

Table 4.15-1-1: Co-location requirements

| Clause number | Requirement | Co-location reference antenna operation | Type |
| --- | --- | --- | --- |
| 6.5 | OTA Transmit ON/OFF power | Measure emission | Mandatory |
| 6.7.6.3 6.7.6.5 | OTA Spurious emission | Measure emission | Optional based on declaration |
| 6.8 | OTA Transmitter intermodulation | Inject the interferer signal | Mandatory |
| 7.6.3 | OTA Blocking | Inject the interferer signal | Optional based on declaration |

The OTA Transmit ON/OFF power requirement and OTA Transmitter intermodulation requirement are mandatory requirements where the test requirement is derived using the co-location reference antenna, which represents the worst-case scenario.

The co-location reference antenna is defined in 3GPP TS 37.105[6].

### 4.15.2	Co-location test antenna

#### 4.15.2.1	General

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

A CLTA is a practical antenna which can be used to test conformance to the co-location requirements.

#### 4.15.2.2	Co-location test antenna characteristics

A co-location test antenna is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. A CLTA shall comply to the requirements specified in Table 4.15.2.2-1. Translation of the requirements to other test antennas is not precuded but suitable translations between the co-location reference antenna and test antennas must be provided to demonstrate that the method is within the specified MU.

The currently defined CLTAs are suitable for testing AAS BSs implemented with a planar antenna array. The method for testing AAS BS with other antenna array implementations is FFS.

Table 4.15.2.2-1: CLTA characteristics

| Parameter | in-band CLTA | out-of-band CLTAs |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | N/A |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | The half-power vertical beam width of the CLTA equals the narrowest declared vertical beamwidth ±3° |
| Polarization | Match | Match to in-band |
| Conducted interface return loss | > 10dB | > 10dB |
| NOTE 1: If a multi-column or multi-band antenna is used the column closest to the AAS BS shall be selected while other columns are terminated during testing. |  |  |

#### 4.15.2.3	Co-location test antenna alignment

The alignment between the AAS BS under test and the co-location test antenna is specified in Table 4.15.2.3-1 and Figure 4.15.2.3-1.

Table 4.15.2.3-1: CLTA alignment tolerances

| Parameter | in-band | out-of-band |
| --- | --- | --- |
| Edge-to-edge separation between the AAS BS and the CLTA, d | 0.1 m ± 0.01 m | 0.1 m ± 0.01 m |
| Vertical alignment | Centre ± 0.01 m | Centre ± 0.01 m |
| Front alignment | Radome front ± 0.01 m | Radome front ± 0.01 m |

Figure 4.15.2.3-1 Alignment of AAS BS and CLTA

# 5	Applicability of Requirements

## 5.1	General

The present clause defines for each radiated test requirement the set of mandatory test configurations which shall be used for demonstrating conformance for radiated requirement.

Test configurations for beams supporting multiple RAT in the tested operating band are specified in subclause 5.1.

Test configurations for radiated requirements where the operating band has been declared to support single RAT requirements (see table 4.10-1, D9.5) by either  MSR requirements for UTRA only or E-UTRA only or with a single-RAT UTRA requirements  or single RAT E-UTRA requirements are specified in subclause 5.2.

Test configurations for an AAS BS with operating bands which have multi-band dependencies are specified in subclause 5.3.

Requirements apply to AAS BS according to the declared RAT radiated Capability Set (see table 4.10-1, D9.25) within each supported operating band  the Band Category of the declared operating band  (see table 4.10-1, D9.4), as listed in the heading of each table. Some RF requirements listed in the tables may not be mandatory or they may apply only regionally. This is further specified for each requirement in clauses 6 and 7, and in table 4.4-1.

For a declared RAT radiated Capability Set (see table 4.10-1, D9.25) in tables 5.2-1, 5.3.2-1, 5.3.3-1, 5.3.4-1, 5.4.1-1 or 5.4.2-1 only the requirements listed in the column for that radiated Capability Set apply. Requirements listed under RCSA other than the declared RCSA(s) need not be tested.

An AAS BS where the operating band is declared: to support multiple RATs (MSR) and to be capable of contiguous spectrum operation only, the test configuration(s) in tables 5.2-1 and 5.3.2-1 denoted by a "C" and entries that refer to single-RAT specifications shall be used for testing.

An AAS BS where the operating band is declared: to support multiple RATs (MSR), to be capable of contiguous and non-contiguous spectrum operation (see table 4.10-1, D9.28), where the parameters according to subclause 4.10 are identical for contiguous and non-contiguous operation (see table 4.10-1, D9.29). Shall use for each declared operating band the test configuration(s) in tables 5.2-1 and 5.3.2-1 denoted by "CNC" and entries that refer to single-RAT specifications shall be used.

An AAS BS where the operating band is declared: to support multiple RATs (MSR), to be capable of contiguous and non-contiguous spectrum operation (see table 4.10-1, D9.28), where the parameters according to subclause 4.10 are not identical for contiguous and non-contiguous operation (see table 4.10-1, D9.29). Shall use for each declared operating band the test configuration(s) in tables 5.2-1 and 5.3.2-1 denoted by "C/NC" and entries that refer to single-RAT specifications shall be used.

For an AAS BS operating bands which have multi-band dependencies which is MSR capable the applicability of the requirement for each operating band is determined by the RAT configuration within that operating band as identified in tables 5.2-1 and 5.3.2-1, unless otherwise stated. The testing of an AAS BS in operating bands with multi-band dependencies which are MSR capable shall be according to table 5.4.1-1 as follows:

-	For requirements test denoted by SBT (Single Band Test), the test configuration (s) in tables 5.2-1 and 5.3.2-1 shall be used for each operating band depending on the RAT configuration within that band.

-	For requirements test denoted by MBT (Multi-Band Test), the test configuration (s) in table 5.4.1-1 shall be used depending on the Band Category of the declared operating band combination.

For a single–RAT UTRA only operating bands subclause 5.3.3 defines for each radiated test requirement the set of mandatory test configurations which shall be used for demonstrating conformance. The applicable test configurations are specified in table 5.3.3-1 for each supported RF configuration, which shall be declared according to subclause 4.10. The generation and EIRP allocation for each test configuration is defined in subclause 4.11.2.

For a single-RAT E-UTRA only operating bands subclause 5.3.4 defines for each radiated test requirement the set of mandatory test configurations which shall be used for demonstrating conformance. The applicable test configurations are specified in table 5.3.4-1 for each supported RF configuration, which shall be declared according to subclause 4.10. The generation and EIRP allocation for each test configuration is defined in subclause 4.11.3.

For an AAS BS operating band declared to be capable of single carrier operation only (see table 4.10-1, D9.4), a single carrier (SC) shall be used for testing.

## 5.2	Test configurations for AAS BS for operating bands where MSR with more than 1 RAT is supported

Table 5.2-1: Test configuration applicability to requirements
and capability sets for AAS BS supporting MSR operation

| Test case |  |  |  | UTRA + E-UTRA (RCSA 3) |  |  |  |  |  | E-UTRA + NR (RCSA 3A) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BC1 |  | BC2 |  | BC3 |  | BC1 |  | BC2 |  | BC3 |  |
| 6.2 |  | Radiated transmit power |  | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3b |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  |
| 6.3 |  | OTA Base Station output power |  | - |  | - |  | - |  | - |  | - |  | - |  |
| 6.3.2 |  | OTA Maximum output power |  | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  |
| 6.3.3 |  | OTA E-UTRA DL RS power |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  |
| 6.4 |  | OTA Output power dynamics |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  |
|  |  | UTRA FDD |  | Subclause 5.3.3 |  | Subclause 5.3.3 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | NR – RE power control dynamic range |  | N/A |  | N/A |  | N/A |  | Tested with Error Vector Magnitude |  | Tested with Error Vector Magnitude |  | Tested with Error Vector Magnitude |  |
|  |  | NR – total power dynamic range |  | N/A |  | N/A |  | N/A |  | SC |  | SC |  | SC |  |
| 6.5 |  | OTA Transmit ON/OFF power |  | - |  | - |  | - |  | - |  | - |  | - |  |
| 6.5.1 |  | OTA Transmitter OFF power |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  |
| 6.5.2 |  | OTA Transmitter transient period |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  |
| 6.6 |  | Transmitted signal quality |  | - |  | - |  | - |  | - |  | - |  | - |  |
| 6.6.2 |  | OTA Frequency error |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA |  | Same TC as used in subclause 6.6.4 |  | Same TC as used in subclause 6.6.4 |  | Same TC as used in subclause 6.6.4 |  | Same TC as used in subclause 6.6.4 |  | Same TC as used in subclause 6.6.4 |  | Same TC as used in subclause 6.6.4 |  |
|  |  | UTRA FDD |  | Same TC as used in subclause 6. 6.4 |  | Same TC as used in subclause 6. 6.4 |  | N/A |  | N/A |  | N/A |  | N/A |  |
| NR |  | NR |  | N/A |  | N/A |  |  |  | Same TC as used in subclause 6.6.4 |  | Same TC as used in subclause 6.6.4 |  | Same TC as used in subclause 6.6.4 |  |
| 6.6.3 |  | OTA Time alignment error |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  |
|  |  | UTRA FDD |  | Subclause 5.3.3 |  | Subclause 5.3.3 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | NR |  | N/A |  | N/A |  | N/A |  | C: ATCR8 CNC: ATCR8 C/NC: ATCR8, ANTCR8 |  | C: ATCR8 CNC: ATCR8 C/NC: ATCR8, ANTCR8 |  | C: ATCR8 CNC: ATCR8 C/NC: ATCR8, ANTCR8 |  |
| 6.6.4 |  | OTA Modulation quality - EVM |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA |  | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | UTRA FDD |  | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ATCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  | NR |  | N/A |  | N/A |  | N/A |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ATCR7 C/NC: ATCR7, ANTCR7 |  |
| 6.7 |  | OTA Unwanted Emissions |  | - |  | - |  | - |  | - |  | - |  | - |  |
| 6.7.2 |  | OTA Occupied bandwidth |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | Minimum requirement |  | Subclause 5.3.3 Subclause 5.3.4 |  | Subclause 5.3.3 Subclause 5.3.4 |  | Subclause 5.3.3 Subclause 5.3.4 |  | Subclause 5.3.4 SC, ATCR8b (Note) |  | Subclause 5.3.4 SC, ATCR8b (Note) |  | Subclause 5.3.4 SC, ATCR8b (Note) |  |
| 6.7.3 |  | OTA Adjacent Channel Leakage power Ratio |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA |  | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 |  | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 |  | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 |  | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 |  | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 |  | C: ATCR2a CNC: ANTCR2 C/NC:ATCR2a, ANTCR2 |  |
|  |  | UTRA FDD |  | Subclause 5.3.3 |  | Subclause 5.3.3 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | NR |  | N/A |  | N/A |  | N/A |  | C: ATCR8a CNC: ANTCR8 C/NC: ATCR8a, ANTCR8 |  | C: ATCR8a CNC: ANTCR8 C/NC: ATCR8a, ANTCR8 |  | C: ATCR8a CNC: ANTCR8 C/NC: ATCR8a, ANTCR8 |  |
|  |  | Cumulative ACLR |  | CNC: ANTCR3a C/NC:ANTCR3a |  | CNC: ANTCR3a C/NC:ANTCR3a |  |  |  | CNC: ANTCR7 C/NC:ANTCR7 |  | CNC: ANTCR7 C/NC:ANTCR7 |  | CNC: ANTCR7 C/NC:ANTCR7 |  |
| 6.6.5 |  | OTA Operating band unwanted emission |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | General requirement for Band Categories 1 and 3 |  | Subclause 5.3.3 Subclause 5.3.4 C: ATCR3a CNC: ATCR3a, ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | Subclause 5.3.3 Subclause 5.3.4 |  | Subclause 5.3.4 C: ATCR7 CNC: ATCR7, ANTCR7 C/NC: ATCR7, ANTCR7 |  | N/A |  | Subclause 5.3.4 C: ATCR7 CNC: ATCR7, ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | General requirement for Band Category 2 |  | N/A |  | Subclause 5.3.3 Subclause 5.3.4 C: ATCR3a CNC: ATCR3a, ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | N/A |  | Subclause 5.3.4 C: ATCR7 CNC: ATCR7, ANTCR7 C/NC: ATCR7, ANTCR7 |  | N/A |  |
|  |  | Additional requirements |  | (note) |  | (note) |  | (note) |  | (note) |  | (note) |  | (note) |  |
| 6.7.6 |  | OTA Spurious emission |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | (Category A) |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | (Category B) |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | Protection of the BS receiver of own or different BS |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | Additional spurious emissions requirements |  | C: ATCR3a, CNC: ANTCR3a, C/NC: ATCR3a, ATCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7, CNC: ANTCR7, C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | Co-location with other Base Stations |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
| 6.8 |  | OTA Transmitter intermodulation |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | General requirement |  | Same TC as used in subclause 6.7 |  | Same TC as used in subclause 6.7 |  | Same TC as used in subclause 6.7 |  | Same TC as used in subclause 6.7 |  | Same TC as used in subclause 6.7 |  | Same TC as used in subclause 6.7 |  |
|  |  | Additional requirement (BC1 and BC2) |  | CNC: ANTCR3a C/NC:ANTCR3a |  | Same TC as used in subclause 6.7 |  | N/A |  | CNC: ANTCR7 C/NC:ANTCR7a |  | Same TC as used in subclause 6.7 |  | N/A |  |
|  |  | Additional requirement (BC3) |  | N/A |  | N/A |  | Same TC as used in subclause 6.7 |  | N/A |  | N/A |  | Same TC as used in subclause 6.7 |  |
| 7.2 |  | OTA sensitivity |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA requirement |  | subclause 5.3.4 |  | subclause 5.3.4 |  | subclause 5.3.4 |  | subclause 5.3.4 |  | subclause 5.3.4 |  | subclause 5.3.4 |  |
|  |  | UTRA FDD requirement |  | subclause 5.3.3 |  | subclause 5.3.3 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | UTRA TDD requirement |  | N/A |  | N/A |  | subclause 5.3.3 |  | N/A |  | N/A |  | N/A |  |
|  |  | NR requirement |  | N/A |  | N/A |  | N/A |  | ATCR4d |  | ATRC4d |  | ATCR4d |  |
| 7.3 |  | OTA reference sensitivity level |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA requirement |  | subclause 5.3.4 |  | subclause 5.3.4 |  | subclause 5.3.4 |  | subclause 5.3.4 |  | subclause 5.3.4 |  | subclause 5.3.4 |  |
|  |  | UTRA FDD requirement |  | subclause 5.3.3 |  | subclause 5.3.3 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | NR requirement |  | N/A |  | N/A |  | N/A |  | ATRC4d |  | ATCR4d |  | ATCR4d |  |
| 7.4 |  | OTA Dynamic range |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  |
|  |  | UTRA FDD |  | Subclause 5.3.3 |  | Subclause 5.3.3 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | NR |  | N/A |  | N/A |  | N/A |  | ATCR4d |  | ATCR4d |  | ATCR4d |  |
| 7.5 |  | OTA Adjacent channel selectivity and narrowband blocking |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | General blocking requirement |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | General narrowband blocking requirement |  | C: ATCR3a, ATCR4b CNC:ANTCR3a, ATCR4b C/NC: ATCR3a, ANTCR3a,ATCR4b |  | C: ATCR3a, ATCR4b CNC:ANTCR3a, ATCR4b C/NC: ATCR3a, ANTCR3a,ATCR4b |  | C: ATCR4b |  | C: ATCR7, ATCR4b, ATCR4d CNC:ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7,ATCR4b, ATCR4d |  | C: ATCR7, ATCR4b, ATCR4d CNC:ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7,ATCR4b, ATCR4d |  | C: ATCR7, ATCR4b, ATCR4d CNC: ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7, ATCR4b, ATCR4d |  |
|  |  | Additional BC3 blocking minimum requirement |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
| 7.6 |  | OTA Blocking |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | General requirement |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | Co-location requirement |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
| 7.7 |  | OTA Receiver spurious emissions |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | General requirement |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | Additional requirement for BC2 (Category B) |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| 7.8 |  | OTA Receiver intermodulation |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | General intermodulation requirement |  | C: ATCR3a CNC: ANTCR3a C/NC: ATCR3a, ANTCR3a |  | C: ATCR3a CNC ANTCR3a C/NC: ATCR3a, ANTCR3a |  | N/A |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC ANTCR7 C/NC: ATCR7, ANTCR7 |  | C: ATCR7 CNC: ANTCR7 C/NC: ATCR7, ANTCR7 |  |
|  |  | General narrowband intermodulation requirement |  | C: ATCR3a, ATCR4b CNC:ANTCR3a, ATCR4b C/NC: ATCR3a, ANTCR3a, ATCR4b |  | C: ATCR3a ATCR4b CNC:ANTCR3a,ATCR4b C/NC: ATCR3a, ANTCR3a; ATCR4b |  | C: ATCR4b |  | C: ATCR7, ATCR4b, ATCR4d CNC:ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7, ATCR4b, ATCR4d |  | C: ATCR7 ATCR4b, ATCR4d CNC:ANTCR7,ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7; ATCR4b, ATCR4d |  | C: ATCR7, ATCR4b, ATCR4d CNC: ANTCR7, ATCR4b, ATCR4d C/NC: ATCR7, ANTCR7, ATCR4b, ATCR4d |  |
| 7.9 |  | OTA In-channel selectivity |  | - |  | - |  | - |  | - |  | - |  | - |  |
|  |  | E-UTRA requirement |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  | Subclause 5.3.4 |  |
|  |  | NR requirement |  | N/A |  | N/A |  | N/A |  | ATCR4d |  | ATCR4d |  | ATCR4d |  |
| NOTE: ATCR8b is only applicable when contiguous CA is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.3	Test configurations for multi-carrier capable AAS BS in operating bands where one RAT capability sets are supported

### 5.3.1	General

An AAS BS may support only one RAT operation in an operating band by fulfilling different sets of requirements. Both UTRA and E-UTRA have two complete sets of requirements that may be fulfilled depending on whether the beam is declared to be MSR or single RAT in the operating band. MSR and single RAT requirements are addressed separately by separate test requirements (and corresponding core requirements). They are also identified by different capability sets as described in subclauses 4.11 and 5.2.

### 5.3.2	AAS BS supporting one RAT only MSR in the operating band

This subclause contains test configuration applicability to requirements and capability sets for AAS BS supporting one RAT only MSR operation operating with multiple carriers (MC).

Table 5.3.2-1: Test configuration applicability to requirements
and capability sets for operating bands supporting one RAT only MSR operation

| Capability Set |  | UTRA (MC) capable BS (RCSA1) |  |  |  | E-UTRA (MC) capable BS (RCSA2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case |  | BC1 |  | BC2 | BC3 | BC1 | BC2 | BC3 |
| 6.2 | Radiated transmit power | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a |  | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1b | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 |
| 6.3 | OTA Base Station output power |  | - | - | - | - | - | - |
| 6.3.2 | OTA maximum output power |  | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 |
| 6.3.3 | OTA E-UTRA DL RS power |  | N/A | N/A | N/A | Subclause 5.3.4 | Subclause 5.3.4 | Subclause 5.3.4 |
| 6.4 | OTA Output power dynamics |  | - | - | - | - | - | - |
| 6.4.2 | OTA UTRA Inner loop power control in the downlink |  | N/A | N/A | N/A | Subclause 5.3.4 | Subclause 5.3.4 | Subclause 5.3.4 |
| 6.4.3 | OTA Power control dynamic range |  | Subclause 5.3.3 | Subclause 5.3.3 | N/A | N/A | N/A | N/A |
| 6.4.4 | OTA Total power dynamic range |  | N/A | N/A | Subclause 5.3.3 | N/A | N/A | N/A |
| 6.4.5 | OTA IPDL time mask |  | Subclause 5.3.3 | Subclause 5.3.3 | N/A |  |  |  |
| 6.4.6 | OTA RE Power control dynamic range |  | N/A | N/A | N/A | Subclause 5.3.4 | Subclause 5.3.4 | Subclause 5.3.4 |
| 6.5 | OTA Transmit ON/OFF power |  | - | - | - | - | - | - |
| 6.5.1 | OTA Transmitter OFF power |  | N/A | N/A | N/A | N/A | N/A | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 |
| 6.5.2 | OTA Transmitter transient period |  | N/A | N/A | N/A | N/A | N/A | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 |
| 6.6 | OTA signal quality |  | - | - | - | - | - | - |
| 6.6.2 | OTA Frequency error |  | - | - | - | - | - | - |
|  | E-UTRA |  | N/A | N/A | N/A | Same TC as used in subclause 6.5.4 | Same TC as used in subclause 6.5.4 | Same TC as used in subclause 6.5.4 |
|  | UTRA FDD |  | Same TC as used in subclause 6.5.4 | Same TC as used in subclause 6.5.4 | N/A | N/A | N/A | N/A |
| 6.6.3 | OTA Time alignment error |  | - | - | - | - | - | - |
|  | E-UTRA |  | N/A | N/A | N/A | Subclause 5.3.4 | Subclause 5.3.4 | Subclause 5.3.4 |
|  | UTRA FDD |  | Subclause 5.3.3 | Subclause 5.3.3 | N/A | N/A | N/A | N/A |
| 6.6.4 | OTA Modulation quality - EVM |  | - | - | - | - | - | - |
|  | E-UTRA |  | N/A | N/A | N/A | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2a C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ATCR2aC/NC: ATCR2a, ANTCR2 |
|  | UTRA FDD |  | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ATCR1a C/NC: ATCR1a, ANTCR1a | N/A | N/A | N/A | N/A |
| 6.7 | OTA Unwanted Emissions |  | - | - | - | - | - | - |
| 6.7.2 | OTA Occupied bandwidth |  | - | - | - | - | - | - |
|  | Minimum requirement |  | Subclause 5.3.3 | Subclause 5.3.3 | Subclause 5.3.3 | Subclause 5.3.4 | Subclause 5.3.4 | Subclause 5.3.4 |
| 6.7.3 | OTA Adjacent Channel Leakage power Ratio |  |  | - | - | - | - | - |
|  | E-UTRA |  | N/A | N/A | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | UTRA FDD |  | Subclause 5.3.3 | Subclause 5.3.3 | N/A | N/A | N/A | N/A |
|  | Cumulative ACLR |  | CNC: ANTCR1a C/NC: ANTCR1a | CNC: ANTCR1a C/NC: ANTCR1a | - | CNC: ANTCR2 C/NC: ANTCR2 | CNC: ANTCR2 C/NC: ANTCR2 | CNC: ANTCR2 C/NC: ANTCR2 |
| 6.7.4 | OTA Spectrum emission mask |  |  |  |  |  |  |  |
| 6.7.5 | OTA Operating band unwanted emission |  | - | - | - | - | - | - |
|  | General requirement for Band Categories 1 and 3 |  | Subclause 5.3.3 C: ATCR1a CNC: ATCR1a, ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | Subclause 5.3.3 | Subclause 5.3.4 C: ATCR2a CNC: ATCR2a, ANTCR2 C/NC: ATCR2a, ANTCR2 | N/A | Subclause 5.3.4 C: ATCR2a CNC: ATCR2a, ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | General requirement for Band Category 2 |  | N/A | Subclause 5.3.3 C: ATCR1a CNC: ATCR1a, ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | N/A | Subclause 5.3.4 C: ATCR2a CNC: ATCR2a, ANTCR2 C/NC: ATCR2a, ANTCR2 | N/A |
|  | Additional requirements |  | (note) | (note) | (note) | (note) | (note) | (note) |
| 6.7.6 | OTA Spurious emission |  | - | - | - | - | - | - |
|  | (Category A) |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | (Category B) |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | Protection of the BS receiver of own or different BS |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | Additional spurious emissions requirements |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | Co-location with other Base Stations |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2C/NC: ATCR2a, ANTCR2 |
| 6.8 | OTA Transmitter intermodulation |  | - | - | - | - | - | - |
|  | General requirement |  | Same TC as used in subclause 6.6 | Same TC as used in subclause 6.6 | Same TC as used in subclause 6.6 | Same TC as used in subclause 6.6 | Same TC as used in subclause 6.6 | Same TC as used in subclause 6.6 |
|  | Additional requirement (BC1 and BC2) |  | CNC: ANTCR1a C/NC:ANTCR1a | Same TC as used in 6.6 | N/A | CNC: ANTCR2 C/NC: ANTCR2 | Same TC as used in 6.6 | N/A |
|  | Additional requirement (BC3) |  | N/A | N/A | Same TC as used in subclause 6.6 | N/A | N/A | Same TC as used in 6.6 |
| 7.2 | OTA sensitivity | - |  | - | - | - | - | - |
|  | E-UTRA requirement | N/A |  | N/A | N/A | subclause 5.3.4 | subclause 5.3.4 | subclause 5.3.4 |
|  | UTRA FDD requirement | subclause 5.3.3 |  | subclause 5.3.3 | N/A | N/A | N/A | N/A |
|  | UTRA TDD requirement | N/A |  | N/A | subclause 5.3.3 | N/A | N/A | N/A |
| 7.3 | OTA Reference sensitivity level | - |  | - | - | - | - | - |
|  | E-UTRA requirement | N/A |  | N/A | N/A | Subclause 5.3.4 | Subclause 5.3.4 | Subclause 5.3.4 |
|  | UTRA FDD requirement | Subclause 5.3.3 |  | Subclause 5.3.3 | N/A | N/A | N/A | N/A |
| 7.4 | OTA Dynamic range | - |  | - | - | - | - | - |
|  | E-UTRA | N/A |  | N/A | N/A | Subclause 5.3.4 | Subclause 5.3.4 | Subclause 5.3.4 |
|  | UTRA FDD | Subclause 5.3.3 |  | Subclause 5.3.3 | N/A | N/A | N/A | N/A |
| 7.5 | OTA Adjacent channel selectivity and narrowband blocking | - |  | - | - | - | - | - |
|  | General blocking requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a , ANTCR1a |  | C: ATCR1a, NC: ANTCR1a C/NC: ATCR1a , ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | General narrowband blocking requirement | C: ATCR1a, ATCR4a CNC: ANTCR1a, ATCR4a C/NC: ATCR1a, ANTCR1a, ATCR4a |  | C: ATCR1a, ATCR4a CNC:ANTCR1a, ATCR4a C/NC: ATCR1a, ANTCR1a, ATCR4a | N/A | C: ATCR2a, ATCR4b CNC:ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2 | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b |
|  | Additional BC3 blocking minimum requirement | N/A |  | N/A | N/A | N/A | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
| 7.6 | OTA Blocking | - |  | - | - | - | - | - |
|  | General requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | Co-location requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
| 7.7 | OTA Receiver spurious emissions | - |  | - | - | - | - | - |
|  | General requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a, ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
| 7.8 | OTA Receiver intermodulation | - |  | - | - | - | - | - |
|  | General intermodulation requirement | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a , ANTCR1a |  | C: ATCR1a CNC: ANTCR1a C/NC: ATCR1a , ANTCR1a | N/A | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 | C: ATCR2a CNC: ANTCR2 C/NC: ATCR2a, ANTCR2 |
|  | General narrowband intermodulation requirement | C: ATCR1a, ATCR4a CNC:ANTCR1a, ATCR4a C/NC: ATCR1a, ANTCR1a, ATCR4a |  | C: ATCR1a, ATCR4a CNC:ANTCR1a, ATCR4a C/NC: ATCR1a, ANTCR1a, ATCR4a | N/A | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b | C: ATCR2a, ATCR4b CNC: ANTCR2, ATCR4b C/NC: ATCR2a, ANTCR2, ATCR4b |
| 7.9 | OTA In-channel selectivity | - |  | - | - | - | - | - |
|  | E-UTRA requirement | N/A |  | N/A | N/A | Subclause 5.3.4 | Subclause 5.3.4 | Subclause 5.3.4 |

### 5.3.3	AAS BS supporting Single-RAT UTRA in the operating band

This subclause contains the test configurations for AAS BS supporting single-RAT UTRA in the operating band. The test configurations apply to beams operating with multiple carriers (MC).

For an AAS BS declared to support multi-carrier operation in contiguous spectrum operation in single band only, the test configurations in the second column of table 5.3.3-1 for FDD, and in the fifth column of table 5.3.3-1 for TDD, shall be used for testing.

NOTE:	The applicability of test configurations for TDD in this subclause is only applicable to UTRA TDD 1,28 Mcps option.

For FDD an AAS BS declared to support multi-carrier operation in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to subclause 4.10 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the third column of table 5.3.3-1 shall be used for testing.

For FDD an AAS BS declared to support multi-carrier operation in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to subclause 4.10 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the fourth column of table 5.3.3-1 shall be used for testing.

Table 5.3.3-1: Test configurations for a AAS BS supporting single-RAT UTRA operation

| test case |  | Single-RAT UTRA FDD MC capable AAS BS operating band (RCSA4) C capable only | Single-RAT UTRA FDD MC capable AAS BS operating band (RCSA4) C and NC capable with identical parameters | Single-RAT UTRA FDD MC capable AAS BS operating band (RCSA4) C and NC capable with different parameters | Single-RAT UTRA TDD MC AAS BS operating band (RCSA4) C capable only |
| --- | --- | --- | --- | --- | --- |
| 6.2 | Radiated Transmit Power | ATCR1a | ATCR1a | ATCR1a, ANTCR1 | ATCR1b |
| 6.3 | OTA Base Station output power | - | - | - | N/A |
| 6.3.2 | Base Station maximum output power | ATC1a | ATC1a | ATC1a, ANTC1 | N/A |
| 6.4 | OTA Output power dynamics | - | - | - | N/A |
| 6.4.2 | OTA UTRA Inner loop power control in the downlink | SC | SC | SC | N/A |
| 6.4.3 | OTA Power control dynamic range | SC | SC | SC | N/A |
| 6.4.4 | OTA Total power dynamic range | SC or ATC1a | SC or ATC1a | SC or ATC1a | N/A |
| 6.4.5 | OTA IPDL time mask | SC | SC | SC | N/A |
| 6.6 | OTA Transmitted signal quality | - | - | - | N/A |
| 6.6.2 | OTA Frequency error | ATC1a | ATC1a | ATC1a, ANTC1 | N/A |
| 6.6.3 | OTA Time alignment error | ATC1a | ATC1a | ATC1a, ANTC1 | N/A |
| 6.6.4 | OTA Modulation quality - EVM | ATC1a | ATC1a | ATC1a, ANTC1 | N/A |
| 6.6.4 | OTA Modulation quality - PCDE | ATC1a | ATC1a | ATC1a, ANTC1 | N/A |
| 6.6.4 | OTA Modulation quality - RCDE | ATC1a | ATC1a | ATC1a | N/A |
| 6.7 | OTA Unwanted Emissions | - | - | - | N/A |
| 6.7.2 | OTA Occupied bandwidth | SC | SC | SC | N/A |
| 6.7.3 | OTA Adjacent Channel Leakage power Ratio | ATC1a | ANTC1 | ATC1a, ANTC1 | N/A |
|  | Cumulative ACLR | - | ANTC1 | ANTC1 | N/A |
| 6.7.4 | OTA Spectrum emission mask | ATC1a | ATC1a, ANTC1 | ATC1a, ANTC1 | N/A |
| 6.7.6 | OTA Spurious emission | ATC1a | ANTC1 | ATC1a, ANTC1 | N/A |
| 6.8 | OTA Transmitter intermodulation | ATC1a | ATC1a, ANTC1 | ATC1a, ANTC1 | N/A |
| 7.2 | OTA sensitivity | ATCR4a | ATCR4a | ATCR4a | ATCR4c |
| 7.3 | OTA Reference sensitivity level | ATCR4a | ATCR4a | ATCR4a | N/A |
| 7.4 | OTA Dynamic range | ATCR4a | ATCR4a | ATCR4a | N/A |
| 7.5 | OTA Adjacent channel selectivity and narrowband blocking | ATCR1a | ANTCR1 | ATCR1a, ANTCR1 | N/A |
| 7.6 | OTA Blocking | ATCR1a | ANTCR1 | ATCR1a, ANTCR1 | N/A |
| 7.7 | OTA Receiver spurious emissions | ATCR1a | ANTCR1 | ATCR1a, ANTCR1 | N/A |
| 7.8 | OTA Receiver intermodulation | ATCR1a | ANTCR1 | ATCR1a, ANTCR1 | N/A |

### 5.3.4	AAS BS supporting Single-RAT E-UTRA in the operating band

This subclause contains the test configurations for AAS BS supporting single-RAT E-UTRA in the operating band. The test configurations apply to AAS BS operating bands operating with multiple carriers (MC).

For an AAS BS declared to support multi-carrier and/or CA operation in contiguous spectrum operation in single band only, the test configurations in the second column of table 5.3.4-1 shall be used for testing.

For an AAS BS declared to support multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to subclause 4.10 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the third column of table 5.3.4-1 shall be used for testing.

For an AAS BS declared to support multi-carrier and/or CA in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to subclause 4.10 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the fourth column of table 5.3.4-1 shall be used for testing.

Table 5.3.4-1: Test configurations for a AAS BS supporting single-RAT E-UTRA operation capable of
both contiguous and non-contiguous spectrum in multi-carrier and/or CA operation in single band

| test case |  | Single-RAT E-UTRA MC capable AAS BS operating band (RCSA5) C capable only | Single-RAT E-UTRA MC capable AAS BS operating band (RCSA5) C and NC capable BS with identical parameters | Single-RAT E-UTRA MC capable AAS BS operating band (RCSA5) C and NC capable BS with different parameters |
| --- | --- | --- | --- | --- |
| 6.2 | Radiated Transmit Power | ATCR2a | ATCR2a | ATCR2a, ANTCR2 |
| 6.3 | OTA Base Station output power | - | - | - |
| 6.3.2 | OTA Base Station maximum output power | ATCR2a | ATCR2a | ATCR2a, ANTCR2 |
| 6.3.3 | OTA E-UTRA DL RS power | SC | SC | SC |
| 6.4 | OTA Output power dynamics | - | - | - |
| 6.4.4 | OTA Total power dynamic range | SC | SC | SC |
| 6.4.6 | OTA RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.5 | OTA Transmit ON/OFF power | ATCR2a | ATCR2a | ATCR2a, ANTCR2 |
| 6.6 | OTA Transmitted signal quality | - | - | - |
| 6.6.2 | OTA Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.6.3 | OTA Time alignment error | ATCR2a | ATCR2a | ATCR2a, ANTCR2 |
| 6.6.4 | OTA Modulation quality - EVM | ATCR2a | ATCR2a | ATCR2a, ANTCR2 |
| 6.7 | OTA Unwanted Emissions | - | - | - |
| 6.7.2 | OTA Occupied bandwidth | SC, ATCR2b (Note) | SC, ATCR2b (Note) | SC, ATCR2b (Note) |
| 6.7.3 | OTA Adjacent Channel Leakage power Ratio | ATCR2a | ATCR2a, ANTCR2 | ATCR2a, ANTCR2 |
| 6.7.3 | OTA Cumulative ACLR | - | ANTCR2 | ANTCR2 |
| 6.7.5 | OTA Operating band unwanted emission | ATCR2a | ATCR2a, ANTCR2 | ATCR2a, ANTCR2 |
| 6.7.6 | OTA Spurious emission | ATCR2a | ANTCR2 | ATCR2a, ANTCR2 |
| 6.8 | OTA Transmitter intermodulation | ATCR2a | Same TC as used in 6.6 | Same TC as used in 6.6 |
| 7.2 | OTA sensitivity | ATCR4b | ATCR4b | ATCR4b |
| 7.3 | OTA Reference sensitivity level | ATCR4b | ATCR4b | ATCR4b |
| 7.4 | OTA Dynamic range | ATCR4b | ATCR4b | ATCR4b |
| 7.5 | OTA Adjacent channel selectivity and narrowband blocking | ATCR2a | ANTCR2 | ATCR1, ANTCR2 |
| 7.6 | OTA Blocking | ATCR2a | ANTCR2 | ATCR2a, ANTCR2 |
| 7.7 | OTA Receiver spurious emissions | ATCR2a | ANTCR2 | ATCR2a, ANTCR2 |
| 7.8 | OTA Receiver intermodulation | ATCR2a | ANTCR2 | ATCR2a, ANTCR2 |
| 7.9 | OTA In-channel selectivity | ATCR4b | ATCR4b | ATCR4b |
| NOTE: ATCR2b is only applicable when contiguous CA is supported. |  |  |  |  |

## 5.4	Test configurations for AAS BS operating bands with multi-band dependencies

### 5.4.1	AAS BS operating bands with multi-band dependencies supporting MSR operation

Table 5.4.1-1: Test configuration for AAS BS operating bands containing beams
with multi-band dependencies supporting MSR operation

| test case |  |  | Test for AAS BS operating bands with multi-band dependencies RCSA1,RCSA2, RCSA3, RCSA3A | Test configuration for MBT |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | BC1/BC2 | BC3 |
| 6.2 | Radiated Transmit Power |  | SBT, MBT | ATCR5a | ATCR5a |
| 6.3 |  | OTA Base Station output power | - | - | - |
| 6.3.2 |  | OTA Base Station maximum output power | SBT, MBT | ATCR5a | ATCR5a |
| 6.3.3 |  | OTA E-UTRA DL RS power | E-UTRA for DL RS power | SBT | - |
| 6.4 |  | OTA Output power dynamics | - | - | - |
| 6.4.2 |  | OTA UTRA Inner loop power control in the downlink | SBT | - | - |
| 6.4.3 |  | OTA Power control dynamic range | SBT | - | - |
| 6.4.4 |  | OTA Total power dynamic range | SBT | - | N/A |
| 6.4.5 |  | OTA IPDL time mask | SBT | - | N/A |
| 6.4.6 |  | OTA RE Power control dynamic range | SBT | - | N/A |
| 6.5 |  | OTA Transmit ON/OFF power | - | - | - |
| 6.5.1 |  | OTA Transmitter OFF power | MBT, SBT (note 3) | N/A | ATCR5a |
| 6.5.2 |  | OTA Transmitter transient period | MBT, SBT (note 3) | N/A | ATCR5a |
| 6.6 |  | OTA Transmitted signal quality | - | - | - |
| 6.6.2 |  | OTA Frequency error | - | - | - |
|  |  | E-UTRA | SBT, MBT | ATCR5a | ATCR5a |
|  |  | UTRA FDD | SBT, MBT | ATCR5a | N/A |
|  |  | NR | SBT, MBT | ATCR5a | ATCR5a |
| 6.6.3 |  | OTA Time alignment error | - | - | - |
|  |  | E-UTRA | SBT, MBT (note 1) | ATCR5b | ATCR5b |
|  |  | UTRA FDD | SBT, MBT (note 1) | ATCR5b | N/A |
|  |  | NR | SBT, MBT (note 1) | ATCR5b | ATCR5b |
| 6.6.4 |  | OTA Modulation quality - EVM | - | - | - |
|  |  | E-UTRA | SBT, MBT | ATCR5a | ATCR5a |
|  |  | UTRA FDD | SBT, MBT | ATCR5a | N/A |
|  |  | NR | SBT, MBT | ATCR5a | ATCR5a |
| 6.7 |  | OTA Unwanted Emissions | - | - | - |
| 6.6.2 |  | OTA Occupied bandwidth | - | - | - |
|  |  | Minimum requirement | SBT | - | - |
| 6.7.3 |  | OTA Adjacent Channel Leakage power Ratio | - | - | - |
|  |  | E-UTRA | SBT, MBT (note 2) | ATCR5b | ATCR5b |
|  |  | UTRA FDD | SBT, MBT (note 2) | ATCR5b | N/A |
|  |  | NR | SBT, MBT (note 2) | ATCR5b | ATCR5b |
|  |  | Cumulative ACLR | SBT, MBT (note 2) | ATCR5b | ATCR5b |
| 6.7.4 |  | OTA Spectrum emission mask | - | - | - |
| 6.7.5 |  | OTA Operating band unwanted emission | - | - | - |
|  |  | General requirement for Band Categories 1 and 3 | SBT, MBT | ATCR5b | ATCR5b |
|  |  | General requirement for Band Category 2 | SBT, MBT | ATCR5b | N/A |
|  |  | Additional requirements | SBT, MBT | - | - |
| 6.7.6 |  | OTA Spurious emission | - | - | - |
|  |  | (Category A) | SBT, MBT | ATCR5b | ATCR5b |
|  |  | (Category B) | SBT, MBT | ATCR5b | ATCR5b |
|  |  | Protection of the BS receiver of own or different BS | SBT, MBT | ATCR5b | ATCR5b |
|  |  | Additional spurious emissions requirements | SBT, MBT | ATCR5b | ATCR5b |
|  |  | Co-location with other Base Stations | - | - | - |
| 6.8 |  | OTA Transmitter intermodulation | - | - | - |
|  |  | General requirement | SBT | - | - |
|  |  | Additional requirement (BC1 and BC2) | SBT | - | N/A |
|  |  | Additional requirement (BC3) | SBT | N/A | - |
| 7.2 | OTA sensitivity |  | SBT | ATCR4a,ATCR4b | None |
| 7.3 | OTA Reference sensitivity level |  | - | - | - |
|  | E-UTRA requirement |  | SBT | - | - |
|  | UTRA FDD requirement |  | SBT | - | - |
|  |  | NR requirement |  |  | SBT |  | - |  |  | - |
| 7.4 | OTA Dynamic range |  | - | - | - |
|  | E-UTRA |  | SBT | - | - |
|  | UTRA FDD |  | SBT | - | - |
|  |  | NR |  |  | SBT |  | - |  |  | - |
| 7.5 | OTA Adjacent channel selectivity and narrowband blocking |  | - | - | - |
|  | General blocking requirement |  | MBT, SBT (note 3) | ATCR5b | ATCR5b |
|  | General narrowband blocking requirement |  | MBT, SBT (note 3) | ATCR5b | ATCR5b |
| 7.6 | OTA Blocking |  | - | - | - |
|  | General requirement |  | MBT, SBT(note 3) | ATCR5b | ATCR5b |
|  | Co-location requirement |  | MBT, SBT(note 3) | ATCR5b | ATCR5b |
| 7.7 | OTA Receiver spurious emissions |  | - | - | - |
|  | General requirement |  | SBT, MBT | ATCR5b | ATCR5b |
| 7.8 | OTA Receiver intermodulation |  | - | - | - |
|  | General intermodulation requirement |  | MBT, SBT(note 3) | ATCR5b | ATCR5b |
|  | General narrowband intermodulation requirement |  | MBT, SBT(note 3) | ATCR5b | ATCR5b |
| 7.9 | OTA In-channel selectivity |  | - | - | - |
|  | E-UTRA requirement |  | SBT | - | - |
|  |  | NR requirement |  |  | SBT |  | - |  |  | - |
| NOTE 1: MBT is only applicable when DB-DC-HSDPA or inter-band CA is supported. NOTE 2: For ACLR, MBT shall be applied for the Inter RF bandwidth gap only. NOTE 3: SBT is only applicable if different Capability Sets are declared for single-band and multi-band operation. |  |  |  |  |  |

### 5.4.2	AAS BS operating bands with multi-band dependencies supporting Single-RAT only

For an AAS BS operating bands with multi-band dependencies supporting single-RAT only in the operational band, the test configurations in table 5.4.2-1, shall be used for testing.

Table 5.4.2-1: Test configuration for AAS BS operating bands with multi-band dependencies supporting Single-RAT only

| test case |  |  | UTRA FDD RCSA4 |  | UTRA TDD RCSA4 | E-UTRA Test RCSA5 |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2 | Radiated Transmit Power |  | ATCR1a/ANTCR1 (Note 1) ATCR5a |  | ATCR1b (Note 3), ATCR5a | ATCR2a/ANTCR2 (Note 5), ATCR5a |
| 6.3 |  | OTA Base Station output power |  | - | - | - |
| 6.3.2 |  | OTA Base Station maximum output power |  | ATCR1a/ANTCR1 (note 1) ATCR5a | N/A | ATCR2a/ANTCR2 (note 6), ATCR5a |
| 6.3.3 |  | OTA E-UTRA DL RS power |  | N/A | N/A | SC |
| 6.4 |  | OTA Output power dynamics |  | - | - | - |
| 6.4.2 |  | OTA UTRA Inner loop power control in the downlink |  | SC | N/A | N/A |
| 6.4.3 |  | OTA Power control dynamic range |  | SC | N/A | SC |
| 6.4.4 |  | OTA Total power dynamic range |  | SC or ATCR1a | N/A | SC |
| 6.4.5 |  | OTA IPDL time mask |  | SC | N/A | N/A |
| 6.4.6 |  | OTA RE Power control dynamic range |  | N/A | N/A | Tested with Error Vector Magnitude |
| 6.5 |  | OTA Transmit ON/OFF power |  | - | - | - |
| 6.5.1 |  | OTA Transmitter OFF power |  | N/A | N/A | ATCR5a (only applied for E-UTRA TDD BS) |
| 6.5.2 |  | OTA Transmitter transient period |  | N/A | N/A | SC |
| 6.6 |  | OTA Transmitted signal quality |  | - | - | - |
| 6.6.2 |  | OTA Frequency error |  | Tested with EVM | N/A | Tested with Error Vector Magnitude |
| 6.6.3 |  | OTA Time alignment error |  | ATCR1a/ANTCR1 (note 1) ATCR5b | N/A | ATCR2a/ANTCR2 (note 6), ATCR5b (note 7) |
| 6.6.4 |  | OTA Modulation quality - EVM |  | ATCR1a/ANTCR1 (note 1), ATCR5a | N/A | ATCR2a/ANTCR2 (note 6), ATCR5a |
| 6.6.4 |  | OTA Modulation quality - PCDE |  | ATCR1a/ANTCR1 (note 1) | N/A | N/A |
| 6.6.4 |  | OTA Modulation quality - RCDE |  | ATCR1 | N/A | N/A |
| 6.7 |  | OTA Unwanted Emissions |  | - | - | - |
| 6.7.2 |  | OTA Occupied bandwidth |  | SC | N/A | SC, ATCR2b (note 8) |
| 6.7.3 |  | OTA Adjacent Channel Leakage power Ratio |  | ATCR1a/ANTCR1 (note 1) ATCR5b (note 2) | N/A | ATCR2a/ANTCR2 (note 6), ATCR5b (note 9) |
| 6.7.4 |  | OTA Spectrum emission mask |  | ATCR1a/ANTCR1 (note 1) ATCR5b | N/A | N/A |
| 6.7.5 |  | OTA Operating band unwanted emission |  | N/A | N/A | ATCR2a/ANTCR2 (note 6), ATCR5b |
| 6.7.6 |  | OTA Spurious emission |  | ATCR1a/ANTCR1 (note 1) ATCR5b | N/A | ATCR2a/ANTCR2 (note 6), ATCR5b |
| 6.8 |  | OTA Transmitter intermodulation |  | ATCR1a/ANTCR1 (note 1) | N/A | ATCR2a/ANTCR2 (note 6) |
| 7.2 | Reference sensitivity level |  | ATCR4a |  | ATCR4c | ATCR4b |
| 7.3 |  | OTA Reference sensitivity level |  | ATCR4a | N/A | ATCR4a |
| 7.4 |  | OTA Dynamic range |  | ATCR4a | N/A | ATCR4a |
| 7.5 |  | OTA Adjacent channel selectivity and narrowband blocking |  | ATCR5b | N/A | ATCR5b |
| 7.6 |  | OTA Blocking |  | ATCR5b | N/A | ATCR5b |
| 7.7 |  | OTA Receiver spurious emissions |  | ATCR1a/ANTC1 (note 1) ATCR5b | N/A | ATCR2a/ANTC2 (note 6), ATCR5b |
| 7.8 |  | OTA Receiver intermodulation |  | ATCR5b | N/A | ATCR5b |
| 7.9 |  | OTA In-channel selectivity |  | N/A | N/A | ATCR4a |
| NOTE 1: ATCR1a and/or ANTCR1 shall be applied in each supported operating band according to table 5.3.3-1. NOTE 2: ATCR5b may be applied for Inter RF Bandwidth gap only. NOTE 3: ATCR1b shall be applied in each supported operating band according to table 5.3.3-1. NOTE 4: Void NOTE 5: ATCR5a may be applied for Inter RF bandwidth gap only. NOTE 6: ATCR2 and/or ANTCR2 shall be applied in each supported operating band according to table 5.3.4-1. NOTE 7: ATCR5b is only applicable when inter-band CA is supported. NOTE 8: ATCR2b is only applicable when contiguous CA is supported. NOTE 9: ATCR5b may be applied for Inter RF bandwidth gap only. |  |  |  |  |  |  |

# 6	Radiated transmitter characteristics

## 6.1	General

General test conditions for transmitter tests are given in clause 4, including interpretation of measurement results and configuration for testing. AAS BS configurations for the tests are defined in subclause 4.8.

If beams have been declared equivalent and parallel (see table 4.10-1, D9.12, D9.13), only a representative beam is necessary to demonstrate conformance. The manufacturer shall declare the minimum number of supported geographical cells (i.e. geographical areas). The minimum number of supported geographical cells (Ncells) relates to the AAS BS setting with the minimum amount of cell splitting supported.

OTA AAS BS transmitter requirements apply per geographical cell.

Any radiated transmitter test requirement specified in 3GPP TS 37.105 [6] is not applicable for AAS BS operation in Band 46.

For OTA base station output power (sub-clause 6.3), OTA transmit ON/OFF power (sub-clause 6.5), OTA unwanted emissions requirements (sub-clause 6.7) and OTA receiver spurious emissions (sub-clause 7.7), TRP is defined in spherical coordinates as:

, where PD(r,,) is the power density in W/m2 of two orthogonal polarizations at a distance r (meters).

NOTE: General TRP expression that is obtained by integrating the Poynting vector over a spherical surface.

## 6.2	Radiated Transmit Power

### 6.2.1	Definition and applicability

An AAS BS is declared to support one or more beams. Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each beam, the requirement is based on declarations (see table 4.10-1) of a beam identifier (D9.3), reference beam direction pair (D9.7), rated beam EIRP (D9.10) at the reference beam direction pair, OTA peak directions set (D9.8), the beam direction pairs at the maximum steering directions (D9.9) and their associated rated beam EIRP  and beamwidth(s) (D9.11) for reference beam direction pair and maximum steering directions.

For a declared beam identifier and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period.

For each beam peak direction associated with a beam direction pair within the OTA peak directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in subclause 6.2.4.1.

NOTE 1:	The OTA peak directions set for a beam is the complete continuous or discrete set of all beam direction for which the EIRP accuracy is intended to be achieved for the beam.

NOTE 2:	A beam direction pair consists of a beam centre direction and an associated beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

For operating bands where the supported fractional bandwidth (FBW) is larger than 6%, two rated carrier EIRP may be declared by manufacturer:

- Prated,c,FBWlow for lower supported frequency range, and

- Prated,c,FBWhigh for higher supported frequency range.

For frequencies in between FFBWlow and FFBWhigh the rated carrier EIRP is:

-	Prated,c,FBWlow, for the carrier whose carrier frequency is within frequency range FFBWlow ≤ f < (FFBWlow +FFBWhigh) / 2,

-	Prated,c,FBWhigh, for the carrier whose carrier frequency is within frequency range (FFBWlow +FFBWhigh) / 2 ≤ f ≤FFBWhigh.

### 6.2.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.2.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.2.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.2.4.

### 6.2.3	Test purpose

The test purpose is to verify the ability to accurately generate and direct radiated power per beam, across the frequency range and under normal conditions, for all declared beams of the AAS BS.

### 6.2.4	Method of test

#### 6.2.4.1	Initial conditions

Test environment: Normal; see annex G.2.

RF bandwidth positions to be tested: BRFBW, MRFBW and TRFBW in single-band operation, see subclause 4.12.1.

BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

Directions to be tested: The reference beam direction pair (see table 4.10-1, D9.7) and the maximum steering directions (see table 4.10-1, D9.9).

In addition for a OTA AAS BS, a single test case shall be performed under extreme test environment as defined in annex clause G.3 In this case, it is sufficient to test on a single combination of one ARFCN, UARFCN or EARFCN, one RF bandwidth position and with only one applicable test configuration defined in clause 5.

NOTE:	Tests under extreme power supply also test extreme temperature.

#### 6.2.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna

4)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair.

5)	Set the base station to transmit according to the applicable test configuration in clause 5 using the corresponding test model(s) in subclause 4.12.2.

In addition, for an AAS BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.11.

6)	Measure EIRP by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Test steps 3 to 6 are repeated for all declared beams (see table 4.10-1, D9.3) and their reference beam direction pairs and maximum steering directions (see table 4.10-1, D9.7 and D9.11).

For multi-band capable AAS BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

8)	For extreme conditions tests the methods in annex X may be used where a representative power measurement is taken at both nominal conditions (Pmax,sample,nom) and extreme conditions (Pmax,sample,ext) and the delta (Δsample) is added to the nominal measurement from step 6 such that Pmax,c,EIRP, extreme = Pmax,c,EIRP + Δsample.

### 6.2.5	Test Requirement

For each declared conformance beam direction pair, in normal conditions, the EIRP measurement result in step 6 of subclause 6.2.4.2 shall for UTRA, E-UTRA and NR remain:

-	within +3.3 dB and –3.3 dB of the manufacturer's declared rated beam EIRP value for carrier frequency f  ≤ 3.0 GHz;

-	within +3.5 dB and –3.5 dB of the manufacturer's declared rated beam EIRP value for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

For a OTA AAS BS in extreme conditions, the result in step 8 (Pmax,c,EIRP, extreme) of subclause 6.2.4.2 shall be:

-	within +5.2 dB and –5.2 dB of the manufacturer's declared rated beam EIRP value for carrier frequency f  ≤ 3.0 GHz;

-	within +5.3 dB and –5.3 dB of the manufacturer's declared rated beam EIRP value for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

## 6.3	OTA Base Station output power

### 6.3.1	General

The OTA AAS BS base station output power is declared as TRP.

### 6.3.2	OTA Maximum output power

#### 6.3.2.1	Definition and applicability

The rated carrier output power of the OTA AAS BS shall be as specified for UTRA in table 6.3.2.1-1, and for E-UTRA and NR in table 6.3.2.1-2

Table 6.3.2.1-1: UTRA OTA AAS Base Station rated output power limits for BS classes

| OTA AAS BS class | PRated,c,TRP |
| --- | --- |
| Wide Area BS | (NOTE) |
| Medium Range BS | ≤ 44 dBm |
| Local Area BS | ≤ 30 dBm |
| NOTE: There is no upper limit for the PRated,c,TRP of the Wide Area Base Station. |  |

Table 6.3.2.1-2: E-UTRA and NR OTA AAS Base Station rated output power limits for BS classes

| OTA AAS BS class | PRated,c,TRP |
| --- | --- |
| Wide Area BS | (NOTE) |
| Medium Range BS | ≤ 47 dBm |
| Local Area BS | ≤ 33 dBm |
| NOTE: There is no upper limit for the PRated,c,TRP of the Wide Area Base Station. |  |

#### 6.3.2.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.3.2.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.3.2.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.3.2.4.

#### 6.3.2.3	Test purpose

The test purpose is to verify the accuracy of the maximum carrier TRP (Pmax,c,TRP) across the frequency range for all RIBs  in the AAS BS.

#### 6.3.2.4	Method of test

##### 6.3.2.4.1	Initial conditions

Test environment: Normal; see annex G.2.

RF bandwidth positions to be tested: BRFBW, MRFBW and TRFBW in single-band operation, see subclause 4.12.1.

BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex F) as long as the required TRP output power level is achieved.

##### 6.3.2.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3) Configure theAAS BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

4)	Set the AAS BS to transmit according to the applicable test configuration in clause 5 using the corresponding test model(s) in subclause 4.12.2.

In addition, for an AAS BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.11.

5)	Align the BS and the test antenna such that measurements to determine TRP can be performed.(see annex F)

6)	Measure power by either a) or b) below:

a)	If the test facility only supports single polarization, then measure power with the test facility's test antenna/probe polarization matched to the AAS BS.

b)	If the test facility supports dual polarization then measure total power for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as power = powerp1 + powerp2.

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex F).

8)	Calculate TRP using thepower measurements.

For multi-band capable AAS BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

#### 6.3.2.5	Test Requirement

The TRP measurement result in step 9 of subclause 6.3.2.4.2  shall for UTRA, E-UTRA and NR remain:

-	within +3.4 dB and -3.4dB of the manufacturer's declared rated TRP value for carrier frequency f  ≤ 3.0 GHz;

-	within +3.5 dB and –3.5 dB of the manufacturer's declared rated TRP value for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

### 6.3.3	OTA E-UTRA DL RS power

#### 6.3.3.1	Definition and applicability

This requirement applies to the RIB(s) transmitting primary DL RS.

The DL RS power is the resource element power of the Downlink Reference Symbol at the RIB transmitting the DL RS for a cell.

The absolute DL RS power is indicated on the DL-SCH. The absolute accuracy is defined as the maximum deviation between the DL RS power indicated on the DL-SCH and the DL RS power of each E-UTRA carrier.

#### 6.3.3.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.3.3.2.

There is no DL RS power requirement for single RAT UTRA operation.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.3.3.4.

#### 6.3.3.3	Test purpose

The test purpose is to verify that the E-UTRA FDD DL RS power is within the limits specified by the minimum requirement.

#### 6.3.3.4	Method of test

##### 6.3.3.4.1	Initial conditions

Test environment: normal:	see annex G.2.

RF channels to be tested:	B, M and T; see subclause 4.12.1.

Beams to be tested:	The narrowest declared beam (see table 4.10-1, D9.3, D9.11).

Directions to be tested:	The reference beam direction pair (see table 4.10-1, D9.7).

##### 6.3.3.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair

5)	Set the AAS BS to transmit using E-TM 1.1, in 36.141 [12]3GPP TS 36.141 [12] subclause 6.1.1.1 at manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

6)	Measure DL RS power EIRP by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

The DL RS power is measured according to annex F in 36.141 [12]3GPP TS 36.141 [12].

In addition, for multi-band RIB(s), the following steps shall apply:

7)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.3.3.5	Test Requirement

The DL RS EIRP of each E-UTRA carrier shall be:

within  3,4 dB of the declared DL RS EIRP (see table 4.10-1, D9.30) for carrier frequency f ≤ 3.0GHz.

within  3,6 dB of the declared DL RS EIRP (see table 4.10-1, D9.30)  for carrier frequency 3.0GHz < f ≤ 4.2GHz.

## 6.4	OTA Output power dynamics

### 6.4.1	General

The requirements in subclause 9.4 apply during the transmitter ON period. Transmit signal quality (as specified in subclause 9.6) shall be maintained for the output power dynamics requirements. Power control is used to limit the interference level. The TA output power requirements are directional requirements and apply to the beam peak directions associated with the beam direction pairs over the OTA peak directions set.

### 6.4.2	OTA UTRA Inner loop power control in the downlink

#### 6.4.2.1	Definition and applicability

Inner loop power control in the downlink is the ability of the AAS BS transmitter to adjust the transmitter output power of a code channel in accordance with the corresponding TPC symbols received in the uplink.

#### 6.4.2.2	Minimum requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.2.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.2.3.

This requirement does not apply to single RAT E-UTRA or NR operation.

#### 6.4.2.3	Test purpose

The test purpose is to verify that the Inner loop power control in the downlink is within the limits specified by the minimum requirement.

#### 6.4.2.4	Method of test

##### 6.4.2.4.1	Initial conditions

Test environment:	normal; see annex G.2.

RF channels to be tested:	 M; see subclause 4.12.1.

Beams to be tested:	The narrowest declared beam (see table 4.10-1, D9.3, D9.11).

Directions to be tested: The reference beam direction pair (see table 4.10-1, D9.7).

Disable closed loop power control.

The DPCH intended for power control is on channel 120 starting at -3 dB.

Establish downlink power control with parameters as specified in table 6.4.2.4.1-1.

Table 6.4.2.4.1-1: DL power control parameters

| Parameter | Level/status | Unit |
| --- | --- | --- |
| UL signal mean power | PREFSENS + 10 dB | dBm |
| Data sequence | PN9 |  |

##### 6.4.2.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna

4)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair

5)	Set the AAS BS to transmit using TM2, in 3GPP TS 25.141 [10],  subclause 6.1.1.2 at the manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

6)	Set and send alternating TPC bits from the UE simulator or UL signal generator.

7)	Measure mean power level of the code under the test each time TPC command is transmitted by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

All steps within power control dynamic range declared by manufacturer (see table 4.10-1, D6.57) shall be measured. Use the code domain power measurement method defined in annex E in 3GPP TS 25.141 [10].

8)	Measure the 10 highest and the 10 lowest power step levels within the power control dynamic range declared by manufacturer by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

Measure by sending 10 consecutive equal commands as described in 3GPP TS 37.105 [6], subclause 6.3.2. Table 6.3.2.3-2

In addition, for multi-band RIB(s), the following steps shall apply:

9)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.4.2.5	Test Requirement

For UTRA FDD the test requirement Inner loop power control is:

a)	AAS BS shall fulfil step size requirement shown in table 6.4.2.5-1 for all power control steps declared by manufacture in subclause 4.10.

b)	For all measured Up/Down cycles, the difference of code domain power between before and after 10 equal commands (Up and Down), derived in step (3), shall not exceed the prescribed tolerance in table 6.4.2.5.1-2.

Table 6.4.2.5-1: UTRA FDD power control step tolerance

| Power control commands in the down link | Transmitter power control step tolerance |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 dB step size |  | 1,5 dB step size |  | 1 dB step size |  | 0,5 dB step size |  |
|  | Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| Up (TPC command "1") | +0.9 dB | +3.1 dB | +0.65 dB | +2.35 dB | +0.4 dB | +1.6 dB | +0.15 dB | +0.85 dB |
| Down (TPC command "0") | -0.9 dB | -3.1 dB | -0.65 dB | -2.35 dB | -0.4 dB | -1.6 dB | -0.15 dB | -0.85 dB |

Table 6.4.2.5-2: UTRA FDD aggregated power control step range

| Power control commands in the down link | Transmitter aggregated power control step change after 10 consecutive equal commands (up or down) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 dB step size |  | 1,5 dB step size |  | 1 dB step size |  | 0,5 dB step size |  |
|  | Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| Up (TPC command "1") | +15.9 dB | +24.1 dB | +11.9 dB | +18.1 dB | +7.9 dB | +12.1 dB | +3.9 dB | +6.1 dB |
| Down (TPC command "0") | -15.9 dB | -24.1 dB | -11.9 dB | -18.1 dB | -7.9 dB | -12.1 dB | -3.9 dB | -6.1 dB |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.4.3	OTA Power control dynamic range

#### 6.4.3.1	Definition and applicability

The power control dynamic range is the difference between the maximum and the minimum code domain power of a code channel for a specified reference condition.

This requirement applies at each RIB supporting transmission in the operating band.

This requirement applies to UTRA operation only.

#### 6.4.3.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.3.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.3.3.

This requirement does not apply to single RAT E-UTRA or NR operation.

#### 6.4.3.3	Test purpose

The test purpose is to verify that the power control dynamic range is within the limits specified by the minimum requirement.

#### 6.4.3.4	Method of test

##### 6.4.3.4.1	Initial conditions

Test environment:	normal; see annex G.2.

RF channels to be tested:	B, M and T; see subclause 4.12.1.

Beams to be tested:	The narrowest declared beam (see table 4.10-1, D9.3, D9.11).

Directions to be tested: The reference beam direction pair (see table 4.10-1, D9.7).

##### 6.4.3.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna

4)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair.

5)	Set the AAS BS to transmit using TM2, in 3GPP TS 25.141 [10],  subclause 6.1.1.2 at the manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

6)	Using TM2, set the code domain EIRP of the DPCH under test to Pmax,c,EIRP - 3 dB. Power levels for other code channels may be adjusted if necessary.

7)	Measure the code domain EIRP of the code channel under test by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

Use the code domain power measurement method defined in annex E in 3GPP TS 25.141 [10].

8)	Set the code domain EIRP of the DPCH under test to Pmax,c,EIRP - 28 dB by means determined by the manufacturer. The power levels for the other code channels used in step 2 shall remain unchanged (the overall output power will drop by approximately 3 dB).

9)	Measure the code domain EIRP of the code channel under test by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.4.3.5	Test Requirement

For UTRA FDD the test requirement Inner loop power control is:

Downlink (DL) power control dynamic range:

-	maximum code domain power: maximum output EIRP (Pmax,c,EIRP) –4.1 dB or greater;

-	minimum code domain power: maximum output EIRP (Pmax,c,EIRP) –26.9 dB or less.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.4.4	OTA Total power dynamic range

#### 6.4.4.1	Definition and applicability

The total power dynamic range is the difference between the maximum and the minimum output power for a specified reference condition.

This requirement applies at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the dynamic range is the BS maximum carrier EIRP (Pmax,c,EIRP). The lower limit of the dynamic range is the lowest minimum power from the AAS BS when no traffic channels are activated in the same direction using the same beam.

Particularly for E-UTRA, the total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

NOTE 2:	The upper limit of the dynamic range at a RIB is the OFDM symbol power at maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the dynamic range at a RIB is the OFDM symbol power when one resource block is transmitted. The OFDM symbol carries PDSCH and not contain RS, PBCH or synchronization signals in the same direction using the same beam.

#### 6.4.4.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.4.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.4.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.4.4.

#### 6.4.4.3	Test purpose

The test purpose is to verify that the total power dynamic range is within the limits specified by the minimum requirement.

#### 6.4.4.4	Method of test

##### 6.4.4.4.1	Initial conditions

Test environment:	normal; see annex G.2.

RF channels to be tested:	B, M and T; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier: BRFBW, MRFBW and TRFBW in single band operation; see subclause 4.12.1.

Beams to be tested:	The narrowest declared beam (see table 4.10-1, D9.3, D9.11).

Directions to be tested: The reference beam direction pair (see table 4.10-1, D9.7).

##### 6.4.4.4.2	Procedure

###### 6.4.4.4.2.1	General procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair.

###### 6.4.4.4.2.2	UTRA FDD

5)	Set the AAS BS to transmit using TM2, in 3GPP TS 25.141 [10],  subclause 6.1.1.2 at the manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

The downlink total dynamic range is computed as the difference of the maximum EIRP, measured as defined in step 6 in subclause 6.2.4.2 and the EIRP measured at step 3 of the Error Vector Magnitude test, as described in subclause 6.6.4.4.2.1.

In addition, for multi-band RIB(s), the following steps shall apply:

6)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 6.4.4.4.2.3	E-UTRA

5)	Set the AAS BS to transmit using E-TM 3.1, in 36.141 [12]3GPP TS 36.141 [12] subclause 6.1.1.1 at the manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

6)	Measure the average OFDM symbol EIRP as defined in annex F in 36.141 [12]3GPP TS 36.141 [12].

7)	Set the AAS BS to transmit using E-TM 2, in 36.141 [12]3GPP TS 36.141 [12] subclause 6.1.1.1 at the manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

8)	Measure the average OFDM symbol power as defined in annex F 36.141 [12]3GPP TS 36.141 [12] by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

The measured OFDM symbols shall not contain RS, PBCH or synchronisation signals.

9)	If BS supports 256QAM, set the channel set-up of the transmitted signal according to E-TM 3.1a.and repeat step 1. Set to transmit a signal according to E-TM 2a and repeat step 3.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 6.4.4.4.2.4	NR

5)	set the BS to transmit a signal according to:

- NR-FR1-TM3.1a in TS 38.141-1 [35] subclause 4.9.2.2.6 if 256QAM is supported by BS without power back off;

- or NR-FR1-TM3.1 in TS 38.141-1 [35] subclause 4.9.2.2.5 if 256QAM is not supported by BS;

- or NR-FR1-TM3.1in TS 38.141-1 [35] subclause 4.9.2.2.5 if 256QAM is supported by BS with power back off;

6)	Measure the average OFDM symbol power as defined  by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	set the BS to transmit a signal according to:

- NR-FR1-TM2a in TS 38.141-1 [35] subclause 4.9.2.2.4 if 256QAM is supported by BS;

- or NR-FR1-TM2 in TS 38.141-1 [35] subclause 4.9.2.2.3 if 256QAM is not supported by BS;

8)	Measure the average OFDM symbol power as defined by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

The measured OFDM symbols shall not contain RS or SSB.

In addition, for multi-band RIB(s), the following steps shall apply:

#### 9)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.4.4.5	Test Requirement

##### 6.4.4.5.1	UTRA FDD

For UTRA FDD the downlink total power dynamic range shall be 17.7 dB or greater.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.4.4.5.2	E-UTRA

The downlink (DL) total power dynamic range for each E-UTRA carrier shall be larger than or equal to the level in table 6.4.4.5.1-1.

Table 6.4.4.5.2-1 E-UTRA total power dynamic range, paired spectrum

| E-UTRA channel bandwidth (MHz) | Total power dynamic range (dB) |
| --- | --- |
| 1.4 | 7.3 |
| 3 | 11.3 |
| 5 | 13.5 |
| 10 | 16.5 |
| 15 | 18.3 |
| 20 | 19.6 |

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

NOTE 2:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in subclause 6.6.4.5.

##### 6.4.4.5.2	NR

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.4.4.5.2-1.

Table 6.4.4.5.2-1: Total power dynamic range

| BS channel bandwidth [MHz] | Total power dynamic range [dB] |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |
| 25 | 20.8 | 17.7 | 14.5 |
| 30 | 21.6 | 18.5 | 15.3 |
| 40 | 22.9 | 19.8 | 16.6 |
| 50 | 23.9 | 20.8 | 17.7 |
| 60 | N/A | 21.8 | 18.5 |
| 70 | N/A | 22.3 | 19.1 |
| 80 | N/A | 22.9 | 19.8 |
| 90 | N/A | 23.4 | 20.4 |
| 100 | N/A | 23.9 | 20.9 |

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### NOTE 2:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in subclause 6.6.4.5.

### 6.4.5	OTA IPDL time mask

#### 6.4.5.1	Definition and applicability

To support IPDL location method in UTRA FDD operation, the AAS BS shall interrupt all transmitted signals in the downlink (i.e. common and dedicated channels). The IPDL time mask specifies the limits at the RIB output power during these idle periods.

This requirement applies only to AAS BS supporting IPDL. The requirement applies at each RIB supporting transmission in the operating band.

#### 6.4.5.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.5.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.5.3.

This requirement does not apply to single RAT E-UTRA or MSR E-UTRA/NR operation.

#### 6.4.5.3	Test purpose

The test purpose is to verify the ability of the AAS BS to temporarily reduce its output power below a specified value to improve time difference measurements made by UE for location services.

#### 6.4.5.4	Method of test

##### 6.4.5.4.1	Initial conditions

Test environment:	normal; see annex G.2.

RF channels to be tested:	B, M and T; see subclause 4.12.1.

Beams to be tested:	The narrowest declared beam (see table 4.10-1, D9.3, D9.11).

Directions to be tested: The reference beam direction pair (see table 4.10-1, D9.7).

Configure the AAS BS to produce idle periods in continuous mode. The IPDL parameters as defined in 3GPP TS 25.214 [23] shall have the following values:

-	IP_Spacing = 5

-	IP_Length = 10 CPICH symbols

-	Seed = 0

##### 6.4.5.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair.

5)	Set the AAS BS to transmit using TM1, in 3GPP TS 25.141 [10],  subclause 6.1.1.2 at the manufacturers declared rated carrier output EIRP(PRated,c,EIRP).

6)	Measure the mean EIRP over a period starting 27 chips after the beginning of the IPDL period and ending 27 chips before the expiration of the IPDL period by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

In addition, for multi-band RIB(s), the following steps shall apply:

7)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.4.5.5	Test Requirement

The mean EIRP measured according to step (3) in clause 6.4.5.4.2 shall be equal to or less than

maximum output EIRP (Pmax,c,EIRP) – 34.3 dB.

See also figure 6.4.5.5-1.

Figure 6.4.5.5-1: IPDL Time Mask

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.4.6	OTA RE Power control dynamic range

#### 6.4.6.1	Definition and applicability

The RE power control dynamic range is the difference between the power of an RE and the average RE power for an AAS BS at maximum output power (PRated,c,TRP) for a specified reference condition.

This requirement applies at each RIB supporting transmission in the operating band.

#### 6.4.6.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.6.2.

This requirement does not apply to single RAT UTRA operation.

For AAS BS single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.4.6.4.

#### 6.4.6.3	Method of test

No specific test or test requirements are defined for RE Power control dynamic range. The Error Vector Magnitude test, as described in subclause 6.6.4 provides sufficient test coverage for this requirement.

## 6.5	OTA Transmit ON/OFF power

### 6.5.1	General

OTA transmitter ON/OFF power requirements apply only to TDD operation of E-UTRA and NR.

The OTA Transmit ON/OFF power requirements are co-location requirements and specified as the power sum of the supported polarization(s) at the co-location reference antenna conducted output(s), see subclause 4.15.

### 6.5.2	OTA Transmitter OFF power

#### 6.5.2.1	Definition and applicability

OTA transmitter OFF power is defined as the mean power measured over 70/N s filtered with a square filter of bandwidth equal to the Base Station RF Bandwidth(s) centred on the central frequency of the Base Station RF Bandwidth (s) during the transmitter OFF period. For UTRA and E-UTRA, N=1. For NR, N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For multi-band RIBs and single band RIBs supporting transmission in multiple bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

#### 6.5.2.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.5.2.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.5.2.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.5.2.4.

#### 6.5.2.3	Test purpose

The purpose of this test is to verify the OTA transmitter OFF power is within the limits of the minimum requirements.

#### 6.5.2.4	Method of test

##### 6.5.2.4.1	Initial conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested:

-	M; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested:

-	MRFBW in single band operation, see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.12.1.

##### 6.5.2.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Set the AAS BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Place the co-location test antenna as specified in subclause 4.15.

5)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair.

6)	Set the AAS BS to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.12.2. For single carrier set the AAS BS to transmit at manufacturers declared rated carrier output power per cell (PRated,c,TRP).

7)	For E-UTRA and UTRA, measure the mean power spectral density at the output(s) of co-location test antennaas power sum over all supported polarizations over 70μs filtered with a square filter of bandwidth equal to the RF bandwidth of the AAS BS centred on the central frequency of the RF bandwidth. 70μs average window centre is set from 35μs after end of one transmitter ON period + 17μs to 35μs before start of next transmitter ON period - 17μs.

For NR, measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the TAB connector centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period – 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

8)	For an AAS BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70 μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70 μs average window centre is set from 35 μs after end of one transmitter ON period + 17 μs to 35μs before start of next transmitter ON period - 17 μs.

In addition, for a multi-band RIB, the following steps shall apply:

9)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.5.2.5	Test Requirement

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

For a multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance given in annex C.

### 6.5.3	OTA Transmitter transient period

#### 6.5.3.1	Definition and applicability

The OTA transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The OTA transmitter transient period is illustrated in figure 6.5.3.1-1.

Figure 6.5.3.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at RIB supporting reception in the operating band and is measured at the co-location reference antenna conducted outputs.

#### 6.5.3.2	Minimum Requirement

This requirement does not apply to single RAT UTRA operation.

For AAS BS in single RAT E-UTRA and NR operation, the minimum requirement is defined in TS 37.105 [6], subclause 9.5.3.2.

#### 6.5.3.3	Test purpose

The purpose of this test is to verify that the OTA transmitter transient periods are within the limits of the minimum requirements.

#### 6.5.3.4	Method of test

##### 6.5.3.4.1	Initial conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	M; see subclause 4.12.1.

RF bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

##### 6.5.3.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Set the AAS BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Place the co-location test antenna as specified in subclause 4.15.

5)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair.

6)	Set the AAS BS to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.12.2. For single carrier set the AAS BS to transmit at manufacturers declared rated carrier output power per cell (PRated,c,TRP).

7)	Measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70μs filtered with a square filter of bandwidth equal to the RF bandwidth of the AAS BS centred on the central frequency of the RF bandwidth. 70μs average window centre is set from 35μs after end of one transmitter ON period + 17μs to 35μs before start of next transmitter ON period - 17μs.

8)	For an AAS BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70 μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70 μs average window centre is set from 35 μs after end of one transmitter ON period + 17 μs to 35μs before start of next transmitter ON period - 17 μs.

In addition, for a multi-band RIB, the following steps shall apply:

9)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.5.3.5	Test Requirement

The mean power spectral density measured according to subclause 6.5.3.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.3.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

For a multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 6.6	OTA Transmitted signal quality

### 6.6.1	General

Unless otherwise stated, the requirements in clause 6.6 apply during the transmitter ON period.

### 6.6.2	OTA Frequency Error

#### 6.6.2.1	Definition and applicability

OTA frequency error is the measure of the difference between the actual AAS BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

The OTA frequency error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 6.6.2.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.6.2.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.6.2.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.6.2.4.

#### 6.6.2.3	Test purpose

The test purpose is to verify that OTA frequency error is within the limit specified by the minimum requirement.

#### 6.6.2.4	Method of test

Requirement is tested together with OTA modulation quality test, as described in subclause 6.6.4.

NOTE:	Measurement only in the OTA coverage range reference direction (see table 4.10-1, D11.4) is enough for OTA frequency error measurement.

#### 6.6.2.5	Test Requirement

##### 6.6.2.5.1	UTRA FDD test requirement

The OTA frequency error for every measured slot shall be between the minimum and maximum value specified in table 6.6.2.5.1-1.

Table 6.6.2.5.1-1: OTA frequency error test requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±(0.05 ppm + 12 Hz) |
| Medium Range BS | ±(0.1 ppm + 12 Hz) |
| Local Area BS | ±(0.1 ppm + 12 Hz) |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.6.2.5.2	E-UTRA and NR test requirement

The modulated carrier frequency of each E-UTRA and NR carrier configured by the AAS BS shall be accurate to within the accuracy range given in table 6.6.2.5.2-1 observed over a period of one subframe (1 ms).

Table 6.6.2.5.2-1: OTA frequency error test requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±(0.05 ppm + 12 Hz) |
| Medium Range BS | ±(0.1 ppm + 12 Hz) |
| Local Area BS | ±(0.1 ppm + 12 Hz) |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.6.3	OTA Time alignment error

#### 6.6.3.1	Definition and applicability

This requirement applies to frame timing in:

-	UTRA single/multi-carrier transmissions and their combinations with MIMO or TX diversity.

-	E-UTRA and/or NR single/multi-carrier transmissions and their combinations with MIMO or TX diversity.

-	E-UTRA and /or NR carrier aggregation, with or without MIMO or TX diversity.

Frames of the UTRA/E-UTRA/NR signals present in the radiated domain are not perfectly aligned in time. In relation to each other, the RF signals present in the radiated domain may experience certain timing differences.

For a specific set of signals/transmitter configuration/transmission mode, the OTA Time Alignment Error (OTA TAE) is defined as the largest timing difference between any two different E-UTRA signals or any two different UTRA or any two different NR signals belonging to different reference symbols (e.g. CRS0 or CRS1) in the radiated domain. The OTA time alignment error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 6.6.3.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.6.3.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.6.3.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.6.3.4.

#### 6.6.3.3	Test purpose

The test purpose is to verify that the OTA time alignment error is within the limit specified by the minimum requirement.

#### 6.6.3.4	Method of test

##### 6.6.3.4.1	Initial conditions

###### 6.6.3.4.1.1	General test conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	M; see subclause 4.12.1.

Directions to be tested: The OTA coverage range reference direction (see table 4.10-1, D11.4).

For dual polarized systems the requirement shall be tested and met considering both polarisations. If the measurement antenna does not support dual polarization, time alignment error shall be measured under the condition that measurement antenna is aligned between the AAS BS polarisations such that it receives half the power from each polarisation.

###### 6.6.3.4.1.2	UTRA FDD

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

Refer to subclause D.1.3 for a functional block diagram of the test set-up.

###### 6.6.3.4.1.3	E-UTRA and NR

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

##### 6.6.3.4.2	Procedure

###### 6.6.3.4.2.1	General Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna.

4)	Configure the beamforming settings of the AAS BS according to the direction to be tested.

###### 6.6.3.4.2.2	UTRA FDD Procedure

5)	If the AAS BS supports TX diversity or MIMO, set the AAS BS to transmit TM1, subclause 4.12.2, at manufacturer's declared rated output power, PRated,c,TRP using TX diversity or MIMO.

6)	Measure the time alignment error between the signals using different P-CPICH and CPICH signals on different beams.

7)	If the AAS BS supports DC-HSDPA, 4C-HSDPA, NC-4C-HSDPA or 8C-HSDPA set the AAS BS to transmit according to TM1, without using TX diversity or MIMO, on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

8)	Measure the time alignment error between the signals using the P-CPICH  and CPICH signals on another beam.

9) If the AAS BS supports DB-DC-HSDPA or any of the multi-band 4C-HSDPA or 8C-HSDPA configurations set the AAS BS to transmit TM1 on two carriers belonging to different frequency bands, without using TX diversity or MIMO on any of the carriers.

10) Measure the time alignment error between the signals using different P-CPICH and CPICH signals on different beams.

In addition, for a multi-band RIB, the following steps shall apply:

11)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 6.6.3.4.2.3	E-UTRA and NR Procedure

5)	Set the AAS BS to transmit E-TM1.1 or NR-FR1-TM1.1 or any DL signal using TX diversity, MIMO transmission or carrier aggregation, using the configuration with the minimum number of cells and reference signals.

NOTE:	For TX diversity and MIMO transmission, different ports may be configured in E-TM (using p = 0 and 1).

For an AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit according to manufacturer's declared rated output power, PRated,c,TRP.

If the AAS BS supports intra band contiguous or non-contiguous Carrier Aggregation set the AAS BS to transmit using the applicable test configuration and corresponding power setting specified in subclauses 4.11.

If the AAS BS supports inter band carrier aggregation set the AAS BS to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in subclauses 4.11.

6)	Measure the time alignment error between the different reference symbols on different beams on the carrier(s).

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.3.5	Test Requirement

##### 6.6.3.5.1	UTRA FDD test requirement

For Tx diversity and MIMO transmission, in the tested cell, TAE shall not exceed 0.35 Tc.

For transmission of multiple cells within a frequency band TAE shall not exceed 0.6 Tc.

For transmission of multiple cells in different frequency bands TAE shall not exceed 5.1 Tc.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.6.3.5.2	E-UTRA test requirement

For MIMO or TX diversity transmissions, at each carrier frequency, TAE shall not exceed 90 ns.

For intra-band carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 155 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 285 ns.

For inter-band carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 285 ns.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.6.3.5.3	NR test requirement

For MIMO or TX diversity transmissions, at each carrier frequency, OTA TAE shall not exceed 90 ns.

For intra-band contiguous carrier aggregation, with or without MIMO or TX diversity, OTA TAE shall not exceed 285 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO or TX diversity, OTA TAE shall not exceed 3.025 µs.

For inter-band carrier aggregation, with or without MIMO or TX diversity, OTA TAE shall not exceed 3.025 µs.

### NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.6.4	OTA Modulation quality

#### 6.6.4.1	Definition and applicability

OTA modulation quality is defined by the difference between the measured carrier signal and a reference signal. Modulation quality can be expressed e.g. as Peak Code Domain Error (PCDE) or Relative Code Domain Error (RCDE) or Error Vector Magnitude (EVM) for UTRA and Error Vector Magnitude (EVM) for E-UTRA.

The OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 6.6.4.2	Minimum Requirement

For AAS BS the in MSR operation minimum requirement is defined in TS 37.105 [6], subclause 9.6.4.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.6.4.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.6.4.4.

#### 6.6.4.3	Test purpose

The test purpose is to verify that OTA modulation quality is within the limit specified by the minimum requirement.

#### 6.6.4.4	Method of test

##### 6.6.4.4.1	UTRA Method of test

###### 6.6.4.4.1.1	Initial conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	B, M and T; see subclause 4.12.1.

Base Station RF Bandwidth position to be tested:

-	BRFBW, MRFBW and TRFBW in single-band operation, see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

Directions to be tested:

-	The OTA coverage range reference direction (see table 4.10-2, D11.4) and the OTA coverage range maximumdirections (see table 4.10-2, D11.5).

-	The EVM test is performed once using the narrowest beamwidth supported by the AAS BS

For dual polarisaed systems the requirement shall be tested and met for both polarisations.

###### 6.6.4.4.1.2	Procedure

6.6.4.4.1.2.1	General procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna.

4)	Configure the beamforming settings of the AAS BS according to the direction to be tested. 6.6.4.4.1.2.2	EVM procedure

5)	Set the AAS BS to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.12.2. For single carrier set the AAS BS to transmit at manufacturers declared rated carrier output power per cell (PRated,c,EIRP).6)	For each carrier, measure the Error Vector Magnitude and frequency error as defined in annex D.1.1 and the mean EIRP (in the conformance direction)  of the signal. The measurement shall be performed on all 15 slots of the frame defined by the Test Model.

7)	Using the same setting as in step 5), set the AAS BS to transmit a signal according to TM4, subclause 4.12.2, with X value equal to 18, and repeat step 6). If the requirement in subclause 6.6.4.5 is not fulfilled, decrease the total output power by setting the base station to transmit a signal according to TM4 with X greater than 18, and repeat step 6).

The following test shall be additionally performed if the base station supports HS-PDSCH transmission using 16QAM:

8)	Using the same setting as in step 5), set the base station to transmit according to TM5, subclause 4.12.2.

9)	Repeat step 6).

In addition, for a multi-band RIB, the following steps shall apply:

10)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.6.4.4.1.2.3	PCDE  procedure

5)	Set the AAS BS to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.12.2. For single carrier set the AAS BS to transmit at manufacturers declared rated carrier output power per cell (PRated,c,EIRP).6)	Measure Peak code domain error according to annex D.1.1. The measurement shall be performed on all 15 slots of the frame defined by TM3. For an AAS BS declared to be capable of multi-carrier operation the measurement is performed on one of the carriers under test.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.6.4.4.1.2.4	RCDE  procedure

5)	Set the AAS BS to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.12.2. For single carrier set the AAS BS to transmit at manufacturers declared rated carrier output power per cell (PRated,c,EIRP).6)	Measure average Relative code domain error according to annex E. The measurement shall be performed over one frame defined by TM6 and averaged as specified in 3GPP TS 25.141 [10] subclause 6.7.4.4.2. For an AAS BS declared to be capable of multi-carrier operation the measurement is performed on one of the carriers under test.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.6.4.4.2	E-UTRA and NR Method of test

###### 6.6.4.4.2.1	Initial conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	B and T; see subclause 4.12.1.

RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW and TRFBW in single-band operation, see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

Directions to be tested:

-	The  OTA coverage range reference direction (see table 4.10-2, D11.4) and the OTA coverage range maximum directions (see table 4.10-2, D11.5).

For dual polarisaed systems the requirement shall be [tested and met for both polarisations.

###### 6.6.4.4.2.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Move the AAS BS on the positioner in order that the direction to be tested aligns with the test antenna

4)	Configure the beamforming settings of the AAS BS according to the direction to be tested.

5)	Set the AAS BS to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.12.2. For single carrier set the AAS BS to transmit at manufacturers declared rated carrier output power per cell (PRated,c,EIRP).6)	Measure the EVM and frequency error as defined in annex D.

7)	For E-UTRA, Repeat steps 5) and 6) for E-TM 3.2, E-TM 3.3 and E-TM 2. Repeat steps 5) and 6) for E-TM3.1a and E-TM 2a for 256QAM, if supported by the AAS BS. For E-TM2 and E-TM2a the OFDM symbol EIRP (in the conformance direction)  shall be at the lower limit of the dynamic range according to the test procedure in subclause 6.4.4.4.2 and test requirements in subclause 6.4.4.5.2.

8)	For NR, Repeat steps 5) and 6) for NR-FR1-TM 3.2, NR-FR1-TM 3.3 and NR-FR1-TM 2. Repeat steps 5) and 6) for NR-FR1-TM3.1a and NR-FR1-TM 2a for 256QAM, if supported by the AAS BS. For NR-FR1-TM2 and E-TM2a the OFDM symbol EIRP (in the conformance direction) shall be at the lower limit of the dynamic range according to the test procedure in subclause 6.4.4.4.2 and test requirements in subclause 6.4.4.5.2.

In addition, for multi-band RIB, the following steps shall apply:

9)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.4.5	Test Requirement

##### 6.6.4.5.1	UTRA test requirement

The Error Vector Magnitude for each UTRA carrier and every measured slot shall be less than 18.5 % when the AAS BS is transmitting a composite signal using only QPSK modulation and shall be less than 13.5% when the AAS BS is transmitting a composite signal that includes 16QAM modulation.

The peak code domain error for every measured slot shall not exceed 32 dB at spreading factor 256.

The average Relative Code Domain Error for 64QAM modulated codes shall not exceed 20 dB at spreading factor 16.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.6.4.5.2	E-UTRA and NR test requirement

The EVM of each E-UTRA and NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.6.4.5.2-1.

Table 6.6.4.5.2-1: EVM requirements

| Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 % |
| 16QAM | 13.5 % |
| 64QAM | 9 % |
| 256QAM | 4.5 % |

The EVM requirement shall be applicable within a time period around the centre of the CP therefore the EVM requirement is tested against the maximum of the RMS average of 10 subframes at the two window W extremities.

Table 6.6.4.5.2-2 specifies EVM window length (W) for normal CP, the cyclic prefix length  is 160 for symbols 0 and 144 for symbols 1-6.

Table 6.6.4.5.2-2: EVM window length for normal CP

| Channel Bandwidth MHz | FFT size | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6 (%) (Note) |
| --- | --- | --- | --- | --- | --- |
| 1.4 | 128 | 10 | 9 | 5 | 55.6 |
| 3 | 256 | 20 | 18 | 12 | 66.7 |
| 5 | 512 | 40 | 36 | 32 | 88.9 |
| 10 | 1024 | 80 | 72 | 66 | 91.7 |
| 15 | 1536 | 120 | 108 | 102 | 94.4 |
| 20 | 2048 | 160 | 144 | 136 | 94.4 |
| NOTE: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 6.7	OTA Unwanted Emissions

### 6.7.1	General

Unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions 3GPP TS 25.331 [15]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

OTA unwanted emissions for OTA AAS BS in single RAT E-UTRA operation and MSR operation using E-UTRA consist of an OTA operating band unwanted emissions requirement and OTA spurious emissions requirement.  OTA operating band unwanted emissions requirement  defines limits for emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band, where ΔfOBUE is the maximum offset of the operating band unwanted emission mask from the operating band edge. Emissions outside of this frequency range are limited by OTA spurious emissions requirement.

The values of ΔfOBUE are defined for OTA AAS BS for E-UTRA, NR and UTRA FDD operating bands in Table 6.7.1-1.

Table 6.7.1-1: Maximum offset of OBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE [MHz] |
| --- | --- | --- |
| OTA AAS BS | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL_high – FDL_low ≤ 900 MHz | 40 |

OTA unwanted emissions for OTA AAS BS in single UTRA operation and MSR operation using UTRA consist of OTA  spectrum emission mask requirement and OTA spurious emissions requirement.

NOTE: for definitions of conducted unwanted emissions requirements refer to clause 6.6 in TS 37.145-1 [9].

The unwanted emission requirements are applied per cell for all the configurations supported by OTA AAS BS.  Requirements for OTA unwanted emissions are captured using TRP, directional requirements or  co-location requirements as described per requirement.

There is in addition a requirement for occupied bandwidth and an ACLR requirement.

### 6.7.2	OTA occupied bandwidth

#### 6.7.2.1	Definition and applicability

The OTA occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also recommendation ITU-R SM.328 [14].

The value of /2 shall be taken as 0.5%.

The OTA occupied bandwidth requirement applies during the transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the OTA occupied bandwidth according to the definition in the present clause.

The OTA occupied bandwidth is defined as a directional requirement and shall be met in the manufacturer’s declared OTA coverage range at the RIB.

#### 6.7.2.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.2.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.2.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.2.4.

#### 6.7.2.3	Test purpose

The test purpose is to verify that the emission at the RIB does not occupy an excessive bandwidth for the service to be provided and is, therefore, not likely to create interference to other users of the spectrum beyond undue limits.

#### 6.7.2.4	Method of test

##### 6.7.2.4.1	Initial conditions

###### 6.7.2.4.1.1	General test conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested:

-	M; see subclause 4.12.1.

Aggregated Channel Bandwidth positions to be tested for contiguous carrier aggregation: MBW Channel CA

Directions to be tested:

-	The reference beam direction pair (see table 4.10-1, D9.7).

###### 6.7.2.4.1.2	UTRA FDD

-	Set the AAS BS to transmit a signal in accordance to TM1in subclause 4.12.2 at manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

###### 6.7.2.4.1.3	E-UTRA and NR

-	Aggregated Channel Bandwidth positions to be tested for contiguous carrier aggregation:

-	BBW Channel CA, MBW Channel CA and TBW Channel CA.

-	For a AAS BS declared to be capable of single carrier operation, start transmission according to E- TM1.1 or N- TM1.1, subclause 4.12.2 at manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

-	For a AAS BS declared to be capble of contiguas carrier aggregation operation, set the base station to transmit according to E-TM1.1 or N- TM1.1, on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 5

-	For an AAS BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.11.

##### 6.7.2.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3) Set the AAS BS so that  the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested is toward the test antenna.

4)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair.

5)	Set the AAS BS to transmit signal.

6)	For UTRA FDD, measure the spectrum of the transmitted signal across a span of 10 MHz, based on an occupied bandwidth requirement of 5 MHz. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less. The spectrum shall be measured at 400 or more points across the measurement span.

For E-UTRA and NR measure the spectrum emission of the transmitted signal using at least the number of measurement points, and across a span, as listed in table 6.7.2.4.2-1. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less.

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode is power responding. There are at least two ways to be power responding. The spectrum analyser can be set to "sample" detection, with its video bandwidth setting at least three times its RBW setting. Or the analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

Table 6.7.2.4.2-1: Span and number of measurement points for OBW measurements for E-UTRA/NR

| Bandwidth | Channel bandwidth BWChannel (MHz) |  |  |  |  |  | Aggregated channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 | 3 | 5 | 10 | 15 | > 20 | > 20 |
| Span (MHz) | 10 | 10 | 10 | 20 | 30 | 2×BWChannel |  |
| Minimum number of measurement points | 1429 | 667 | 400 | 400 | 400 | 2×BWChannel100kHz |  |

7)	Compute the total of the EIRP, P0, (in power units, not decibel units) of all the measurement cells in the measurement span. Compute P1, the EIRP outside the occupied bandwidth on each side. P1 is half of the total EIRP outside the bandwidth. P1 is half of (100 % - (occupied percentage)) of P0. For the occupied percentage of 99 %, P1 is 0.005 times P0. The EIRP calculation depends on whether the test facility supports dual polarization:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS. Measure and sum the EIRP on both polarizations to obtain P0 or P1.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power as the sum over both polarizations to obtain P0 or P1

8)	Determine the lowest frequency, f1, for which the sum of all EIRP in the measurement cells from the beginning of the span to f1 exceeds P1.

9)	Determine the highest frequency, f2, for which the sum of all EIRP in the measurement cells from the end of the span to f2 exceeds P1.

10)	Compute the OTA occupied bandwidth as f2 - f1.

In addition, for multi-band RIB(s), the following steps shall apply:

11) For multi-band RIBs and single band tests, repeat the steps 6) - 10) above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

#### 6.7.2.5	Test Requirement

##### 6.7.2.5.1	MSR

The OTA occupied bandwidth of a single carrier shall be less than the values listed in table 6.7.2.5.1-1. In addition, for E-UTRA/NR intra-band contiguous carrier aggregation, test requirement in clause 6.6.1.5 of 3GPP TS 36.141 [12] applies for the E-UTRA/NR component carriers that are aggregated.

Table 6.7.2.5.1-1: OTA Occupied bandwidth

| RAT | OTA Occupied bandwidth limit |
| --- | --- |
| E-UTRA/NR | BWChannel |
| UTRA FDD | 5 MHz |

##### 6.7.2.5.2	UTRA FDD

The OTA occupied bandwidth shall be less than 5 MHz based on a chip rate of 3,84 Mcps.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.7.2.5.3	E-UTRA

The OTA occupied bandwidth for each E-UTRA carrier shall be less than the channel bandwidth. For contiguous CA, the occupied bandwidth shall be less than or equal to the Aggregated Channel Bandwidth as defined in 3GPP TS 36.141 [12] subclause 5.6.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.7.3	OTA Adjacent Channel Leakage power Ratio

#### 6.7.3.1	Definition and applicability

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

#### 6.7.3.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.3.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.3.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.3.4.

#### 6.7.3.3	Test purpose

To verify that the adjacent channel leakage power ratio requirement shall be met as specified by the minimum requirement.

#### 6.7.3.4	Method of test

##### 6.7.3.4.1	Initial conditions

###### 6.7.3.4.1.1	General test conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	B and T; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW and TRFBW in single-band operation; see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

###### 6.7.3.4.1.2	MSR

For E-UTRA ACLR requirement outside the Base Station RF Bandwidth edges and the ACLR requirement applied inside sub-block gap, in addition, for non-contiguous spectrum operation or Inter RF Bandwidth gap for multi-band operation using, the test configurations defined in subclause 4.8, the method of test described in subclause 6.7.3.4.2 applies.

###### 6.7.3.4.1.3	UTRA FDD

Set the AAS BS to transmit a signal modulated in accordance to TM1, in subclause 4.12.2.

For an AAS BS declared to be capable of multi-carrier operation, set the base station to transmit according to TM1 on all carriers configured.

###### 6.7.3.4.1.4	E-UTRA

For an AAS BS declared to be capable of single carrier operation only set to transmit a signal according to E-TM1.1 in subclause 4.12.2.

For an AAS BS declared to be capable of multi-carrier and/or CA operation, set to transmit according to E-TM1.1 on all carriers configured.

###### 6.7.3.4.1.5	NR

For an AAS BS declared to be capable of single carrier operation only set to transmit a signal according to NR-FR1-TM1.1 in subclause 4.12.2.

For an AAS BS declared to be capable of multi-carrier and/or CA operation, set to transmit according to NR-FR1-TM1.1 on all carriers configured.

##### 6.7.3.4.2	Procedure

##### 6.7.3.4.2.1	General Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Configure theAAS BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

The measurement devices characteristics shall be:

- measurement filter bandwidth: defined in subclause 6.7.3.5.

- detection mode: true RMS voltage or true power averaging.

4)	For single carrier operation, set the AAS BS to transmit according to the applicable test configuration in clause 5 using the corresponding test model(s) in subclause 4.12.2 at manufacturers declared rated carrier output power (PRated,c,TRP).

For an AAS BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.11.

5)	Align the BS and the test antenna such that measurements to determine TRP can be performed (see Annex F)

6)	Measure the absolute total power of the assigned channel frequency and the (adjacent channel frequency)

7)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for TRPEstimate for each of the assigned channel frequency and the adjacent channel frequency (see Annex F).

8)	Calculate TRPEstimate for the absolute total radiated power of the wanted channel and the adjacent channel and the ACLR estimate using the measurements made in Step 7.

NOTE:	ACLR is calculated by the ratio of the absolute TRP of the assigned channel frequency and the absolute TRP of the adjacent frequency channel.

##### 6.7.3.4.2.2	MSR

1)	For E-UTRA and NR, measure OTA ACLR:

-	outside the Base Station RF Bandwidth edges;

-	inside sub-block gap for non-contiguous spectrum operation as specified in subclause 6.7.3.5.1.1;

-	inside Inter RF Bandwidth gap for multi-band operation.

2)	For UTRA FDD, measure ACLR inside sub-block gap or Inter RF Bandwidth gap as specified in subclause 6.7.3.5.1.2.

3)	Measure Cumulative Adjacent Channel Leakage power Ratio (CACLR) inside sub-block gap or the Inter RF Bandwidth gap as specified in subclause 6.7.3.5.1.3.

In addition, for multi-band RIB, the following steps shall apply:

4)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.3.4.2.3	UTRA FDD

1)	Measure OTA ACLR for 5 MHz and 10 MHz offsets both side of channel frequency. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

2)	For the OTA ACLR requirement applied inside sub-block gap for non-contiguous spectrum operation or inside Inter RF Bandwidth gap for multi-band operation:

a)	Measure OTA ACLR inside sub-block gap or Inter RF Bandwidth gap as specified in clause 6.7.3.5.2.1, if applicable.

b)	Measure OTA CACLR inside sub-block gap or Inter RF Bandwidth gap as specified in clause 6.7.3.5.2.2, if applicable.

In addition, for multi-band RIB, the following steps shall apply:

3)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.3.4.2.4	E-UTRA

1)	Measure OTA ACLR for the frequency offsets both side of channel frequency as specified in table 6.7.3.5.3.1-1 (Paired spectrum case) or Table 6.7.3.5.3.1-2 (Unpaired spectrum case) respectively. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

2)	For the OTA ACLR requirement applied inside sub-block gap for non-contiguous spectrum operation: or inside Inter RF Bandwidth gap for multi-band operation:

a)	Measure OTA ACLR inside sub-block gap or Inter RF Bandwidth gap as specified in subclause 6.7.3.5.3.1, if applicable.

b)	Measure OTA CACLR inside sub-block gap or Inter RF Bandwidth gap as specified in subclause 6.7.3.5.3.2, if applicable.

3)	Repeat the test with the channel set-up according to E- TM1.2 in subclause 4.12.2.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.7.3.5	Test Requirement

##### 6.7.3.5.1	MSR

###### 6.7.3.5.1.1 MSR E-UTRA test requirement

For E-UTRA, the test requirement is specified in tables 6.7.3.5.1.1-1 and 6.7.3.5.1.1-2, and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

For an AAS BS operating in non-contiguous spectrum, the OTA ACLR also applies for the first adjacent channel inside any sub-block gap with a gap size Wgap ≥ 15 MHz. The OTA ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz. The CACLR test requirement in subclause 6.6.3.5.6.2 applies in sub-block gaps for the frequency ranges defined in table 6.6.3.5.6.2-1.

For a multi-band RIB, the ACLR also applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15 MHz. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz. The OTA CACLR requirement in subclause 6.7.3.5.3.2 applies in Inter RF Bandwidth gaps for the frequency ranges defined in Table 6.7.3.5.3.2-1.

The requirement applies during the transmitter on period.

The OTA ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For operation in paired spectrum, the measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.1.1-1.

Table 6.7.3.5.1.1-1: OTA ACLR in paired spectrum

| Channel bandwidth of E-UTRA Lowest/ Highest Carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lower or above the upper Base Station RF Bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| 1.4, 3.0, 5, 10, 15, 20 | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |
|  | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |
|  | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
|  | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA Lowest/Highest Carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |  |

For operation in unpaired spectrum, the measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.1.1-2.

Table 6.7.3.5.1.1-2: OTA ACLR in unpaired spectrum with synchronized operation

| Channel bandwidth of E-UTRA Lowest/ Highest Carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lower or above the upper Base StationRF Bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| 1.4, 3 | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |
|  | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |
|  | 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44 dB | 43.8dB |
|  | 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44 dB | 43.8dB |
| 5, 10, 15, 20 | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |
|  | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |
|  | 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44 dB | 43.8dB |
|  | 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44 dB | 43.8dB |
|  | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
|  | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
|  | 5 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44 dB | 43.8dB |
|  | 15 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44 dB | 43.8dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA Lowest/Highest Carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.105 [3], with a chip rate as defined in this table. |  |  |  |  |  |

For operation in non-contiguous paired spectrum, the measurement results shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.1.1-3.

Table 6.7.3.5.1.1-3: OTA ACLR in non-contiguous paired spectrum

| Sub-block gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |  |

For operation in non-contiguous unpaired spectrum, the measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.1.1-4.

Table 6.7.3.5.1.1-4: OTA ACLR in non-contiguous unpaired spectrum

| Sub-block gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44 dB | 43.8dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44 dB | 43.8dB |

For Category A, an E-UTRA AAS BS of Wide Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.1.1-1/2/3/4 or OTA ACLR absolute limit of -4 dBm/MHz shall apply, whichever is less stringent.

For Category B, an E-UTRA AAS BS of Wide Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.1.1-1/2/3/4 or OTA ACLR absolute limit of -6 dBm/MHz shall apply, whichever is less stringent.

For an E-UTRA AAS BS of Medium Range BS class, either the OTA ACLR limits in Tables 6.7.3.5.1.1-1/2/3/4 or OTA ACLR absolute limit of -16 dBm/MHz shall apply, whichever is less stringent.

For an E-UTRA AAS BS of Local Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.1.1-1/2/3/4 or OTA ACLR absolute limit of -23 dBm/MHz shall apply, whichever is less stringent.

###### 6.7.3.5.1.2 MSR UTRA FDD test requirement

For UTRA FDD, the test requirement is specified in Tables 6.7.3.5.1.2-1 and 6.7.3.5.1.2-2, and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

The measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.1.2-1.

Table 6.7.3.5.1.2-1: OTA ACLR

| BS channel offset below the first or above the last carrier frequency used | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- |
| 5 MHz | 44 dB | 43.8dB |
| 10 MHz | 44 dB | 43.8dB |
| Note 1: In certain regions, the adjacent channel power (the RRC filtered mean power centered on an adjacent channel frequency) shall be less than or equal to -7.2 dBm/3.84 MHz (for Band I, III, IX, XI and XXI) or ＋2.8 dBm/3.84MHz (for Band VI, VIII and XIX) or as specified by the ACLR limit, whichever is the higher. This note is not applicable for Home BS. Note 2: For Home BS, the adjacent channel power (the RRC filtered mean power centered on an adjacent channel frequency) shall be less than or equal to -42.7 dBm/3.84MHz f≤ 3.0 GHz and -42.4 dBm/3.84MHz for 3.0 GHz < f ≤ 4.2 GHz or as specified by the ACLR limit, whichever is the higher. |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex F.

The measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.1.2-2.

Table 6.7.3.5.1.2-2: OTA ACLR in non-contiguous spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104, with a chip rate as defined in this table. |  |  |  |  |  |

For an AAS BS operating in non-contiguous spectrum, OTA ACLR requirement also applies for the first adjacent channel, inside any sub-block gap with a gap size Wgap ≥ 15 MHz. The OTA ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz. The OTA CACLR test requirement in subclause 6.7.3.5.3.2 applies in sub block gaps for the frequency ranges defined in Table 6.7.3.5.3.2-1.

For a multi-band capable AAS BS OTA ACLR requirement also applies for the first adjacent channel, inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15 MHz. The OTA ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz. The OTA CACLR requirement in subclause 6.7.3.5.3.2 applies in Inter RF Bandwidth gaps for the frequency ranges defined in Table 6.7.3.5.3.2-1.

For Category A, a UTRA AAS BS of Wide Area BS class, either the OTA ACLR limits in the Tables 6.7.3.5.1.2-1/2 or the absolute limit of -7 dBm/MHz shall apply, whichever is less stringent.

For Category B, a UTRA AAS BS of Wide Area BS class, either the OTA ACLR limits in the Tables 6.7.3.5.1.2-1/2 or the absolute limit of -9 dBm/MHz shall apply, whichever is less stringent.

For a UTRA AAS BS of Medium Range BS class, either the OTA ACLR limits in the Tables 6.7.3.5.1.2-1/2 or the absolute limit of -19 dBm/MHz shall apply, whichever is less stringent.

For a UTRA AAS BS of Local Area BS class, either the OTA ACLR limits in the Tables 6.7.3.5.1.2-1/2 or the absolute limit of -26 dBm/MHz shall apply, whichever is less stringent.

###### 6.7.3.5.1.3 OTA Cumulative ACLR test requirement in non-contiguous spectrum

The following test requirement applies for sub-block or Inter RF Bandwidth gap sizes listed in Table 6.7.3.5.1.3-1:

-	Inside a sub-block gap within an operating band for a BS operating in non-contiguous spectrum.

-	Inside an Inter RF Bandwidth gap for a BS operating in multiple bands, where multiple bands are mapped on the same antenna connector.

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or the Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap; and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The requirement applies to adjacent channels of E-UTRA or UTRA carriers allocated adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap. The assumed filter for the adjacent channel frequency is defined in Table 6.7.3.5.1.3-1 and the filters on the assigned channels are defined in Table 6.7.3.5.1.3-2.

NOTE:	If the RAT on the assigned channel frequencies is different, the filters used are also different.

The CACLR for E-UTRA and UTRA carriers located on either side of the sub-block gap or the Inter RF Bandwidth gap shall not be less than the value specified in Table 6.7.3.5.1.3-1.

Table 6.7.3.5.1.3-1: OTA CACLR in non-contiguous spectrum or multiple bands

| Band Category | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit for bands below 3GHz | OTA CACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- | --- |
| BC1, BC2 | 5 ≤ Wgap < 15 (Note 3) | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| BC1, BC2 | 10 < Wgap < 20 (Note 3) | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| BC3 | 5 ≤ Wgap < 15 (Note 3) | 2.5 MHz | 5MHz E-UTRA | Square (BWConfig) | 44 dB | 43.8dB |
| BC3 | 10 < Wgap < 20 (Note 3) | 7.5 MHz | 5MHz E-UTRA | Square (BWConfig) | 44 dB | 43.8dB |
| BC1, BC2, BC3 | 5 ≤ Wgap < 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8dB |
| BC1, BC2, BC3 | 10 ≤ Wgap < 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8dB |
| BC1, BC2, BC3 | 20 ≤ Wgap < 30 (Note 3, 5) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8dB |
| BC1, BC2, BC3 | 20 ≤ Wgap < 60 (Note 4) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 45 dB | 43.8dB |
| BC1, BC2, BC3 | 40 ≤ Wgap < 50 (Note 3, 5) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8dB |
| BC1, BC2, BC3 | 40 ≤ Wgap < 80 (Note 4) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8dB |
| NOTE 1: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the channel bandwidth of the carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. NOTE 5: Applicable in case the channel bandwidth of the lowest/highest NR carrier transmitted is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |

Table 6.7.3.5.1.3-2: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| E-UTRA | E-UTRA of same BW |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |
| UTRA FDD | RRC (3.84 Mcps) |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.104 [2], with a chip rate as defined in this table. |  |

For Category A, an E-UTRA or NR AAS BS of Wide Area BS class, either the OTA CACLR limits in Table 6.7.3.5.1.3-1 or OTA ACLR absolute limit of -4 dBm/MHz shall apply, whichever is less stringent.

For Category B, an E-UTRA or NR AAS BS of Wide Area BS class, either the OTA CACLR limits in Table 6.7.3.5.1.3-1 or OTA ACLR absolute limit of -6 d Bm/MHz shall apply, whichever is less stringent.

For an E-UTRA or NR AAS BS of Medium Range BS class, either the OTA CACLR limits in Table 6.7.3.5.1.3-1 or OTA ACLR absolute limit of -16 dBm/MHz shall apply, whichever is less stringent.

For an E-UTRA or NR AAS BS of Local Area BS class, either the OTA CACLR limits in Table 6.7.3.5.1.3-1 or OTA ACLR absolute limit of -23 dBm/MHz shall apply, whichever is less stringent.

For Category A, a UTRA AAS BS of Wide Area BS class, either the OTA CACLR limits in Table 6.7.3.5.1.3-1 or OTA ACLR absolute limit of -7 dBm/MHz shall apply, whichever is less stringent.

For Category B, a UTRA AAS BS of Wide Area BS class, either the OTA CACLR limits in Table 6.7.3.5.1.3-1 or OTA ACLR absolute limit of -9 dBm/MHz shall apply, whichever is less stringent.

For a UTRA AAS BS of Medium Range BS class, either the OTA CACLR limits in Table 6.7.3.5.1.3-1 or OTA ACLR absolute limit of -19 dBm/MHz shall apply, whichever is less stringent.

For a UTRA AAS BS of Local Area BS class, either the OTA CACLR limits in Table 6.7.3.5.1.3-1 or OTA ACLR absolute limit of -26 dBm/MHz shall apply, whichever is less stringent.

###### 6.7.3.5.1.4	NR test requirement

For the NR OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.1.4-1/2a or the OTA ACLR absolute limits in tables 6.7.3.5.1.4-2 shall apply, whichever is less stringent. The OTA CACLR limits in subclause 6.7.3.5.1.3 shall also apply.

For a RIB operating in non-contiguous spectrum, the OTA ACLR requirement applies inside sub block gaps for the frequency ranges defined in table 6.7.3.5.1.4-2a, while the CACLR requirement applies inside sub block gaps for the frequency ranges defined in table 6.7.3.5.1.3-1.

For a multi-band RIB, the OTA ACLR test requirement applies inside Inter RF Bandwidth gaps for the frequency ranges defined in table 6.7.3.5.1.4-2a, while the OTA CACLR requirement applies inside Inter RF Bandwidth gaps for the frequency ranges defined in table 6.7.3.5.1.3-1.

For operation in paired and unpaired spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1.4-1.

Table 6.7.3.5.1.4-1: NR ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0 – 3 GHz) | OTA ACLR limit (3 – 6 GHz) |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.1.4-2.

Table 6.7.3.5.1.4-2: NR ACLR absolute limit

| BS category / BS class | OTA ACLR absolute limit |
| --- | --- |
| Category A Wide Area BS | -4 dBm/MHz |
| Category B Wide Area BS | -6 dBm/MHz |
| Medium Range BS | -16 dBm/MHz |
| Local Area BS | -23 dBm/MHz |

For operation in non-contiguous spectrum or multiple bands, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1.4-2a.

Table 6.7.3.5.1.4-2a: NR ACLR limit in non-contiguous spectrum or multiple bands

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0-3GHz) | OTA ACLR limit (3-6GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |

The OTA CACLR measurement result shall not less than the OTA CACLR limit specified in table 6.7.3.5.1.4-1.

##### 6.7.3.5.2	UTRA FDD

###### 6.7.3.5.2.1	OTA ACLR

The measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.2.1-1.

Table 6.7.3.5.2.1-1: OTA BS ACLR

| BS channel offset below the first or above the last carrier frequency used | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- |
| 5 MHz | 44 dB | 43.8dB |
| 10 MHz | 44 dB | 43.8dB |
| NOTE 1: In certain regions, the adjacent channel power (the RRC filtered mean power centred on an adjacent channel frequency) shall be less than or equal to -7.2 dBm/3.84 MHz (for Band I, III, IX, XI and XXI) or +2.8 dBm/3.84 MHz (for Band VI, VIII and XIX) or as specified by the ACLR limit, whichever is the higher. This note is not applicable for Home BS. NOTE 2: For Home BS, the adjacent channel power (the RRC filtered mean power centred on an adjacent channel frequency) shall be less than or equal to -42.7 dBm/3.84 MHz f≤ 3.0 GHz and -42.4 dBm/3.84 MHz for 3.0 GHz < f ≤ 4.2 GHz or as specified by the ACLR limit, whichever is the higher. |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

The measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.2.1-2.

Table 6.7.3.5.2.1-2: OTA ACLR in non-contiguous spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |  |

For Category A, a UTRA AAS BS of Wide Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.2.1-1/2 or OTA ACLR absolute limit of -7 dBm/MHz shall apply, whichever is less stringent.

For Category B, a UTRA AAS BS of Wide Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.2.1-1/2 or OTA ACLR absolute limit of -9 dBm/MHz shall apply, whichever is less stringent.

For a UTRA AAS BS of Medium Range BS class, either the OTA ACLR limits in Tables 6.7.3.5.2.1-1/2 or OTA ACLR absolute limit of -19 dBm/MHz shall apply, whichever is less stringent.

For a UTRA AAS BS of Local Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.2.1-1/2 or OTA ACLR absolute limit of -26 dBm/MHz shall apply, whichever is less stringent.

###### 6.7.3.5.2.2	OTA Cumulative ACLR test requirement in non-contiguous spectrum or multiple bands

The following test requirement applies for an AAS BS operating in non-contiguous spectrum or multiple bands.

The following requirement applies for the gap sizes listed in Table 6.7.3.5.2.2-1:

-	inside a sub-block gap within an operating band for an AAS BS operating in non-contiguous spectrum;

-	inside an Inter RF Bandwidth gap for a multi-band capable AAS BS.

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or Inter RF Bandwidth gap; and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in Table 6.7.3.5.2.2-1 and the filters on the assigned channels are defined in Table 6.7.3.5.2.2-2.

The CACLR for UTRA carriers located on either side of the sub-block gap or Inter RF Bandwidth gap shall not be less than the value specified in Table 6.7.3.5.2.2-1.

Table 6.7.3.5.2.2-1: OTA CACLR in non-contiguous spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit for bands below 3GHz | OTA CACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| 10 MHz < Wgap < 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |  |

Table 6.7.3.5.2.2-2: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| UTRA FDD | RRC (3.84 Mcps) |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.104 [2],with a chip rate as defined in this table. |  |

For Category A, a UTRA AAS BS of Wide Area BS class, either the OTA CACLR limits in Table 6.7.3.5.2.2-1 or OTA ACLR absolute limit of -7 dBm/MHz shall apply, whichever is less stringent.

For Category B, a UTRA AAS BS of Wide Area BS class, either the OTA CACLR limits in Table 6.7.3.5.2.2-1 or OTA ACLR absolute limit of -9 dBm/MHz shall apply, whichever is less stringent.

For a UTRA AAS BS of Medium Range BS class, either the OTA CACLR limits in Table 6.7.3.5.2.2-1 or OTA ACLR absolute limit of -19 dBm/MHz shall apply, whichever is less stringent.

For a UTRA AAS BS of Local Area BS class, either the OTA CACLR limits in Table 6.7.3.5.2.2-1 or OTA ACLR absolute limit of -26 dBm/MHz shall apply, whichever is less stringent.

##### 6.7.3.5.3	E-UTRA

###### 6.7.3.5.3.1	OTA ACLR

For operation in paired spectrum, the OTA ACLR shall be higher than the value specified in Table 6.7.3.5.3.1-1.

Table 6.7.3.5.3.1-1: OTA ACLR in paired spectrum

| Channel bandwidth of E-UTRA lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| 1.4, 3.0, 5, 10, 15, 20 | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |
|  | BWChannel /2 + 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
|  | BWChannel /2 + 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA lowest/highest/ carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.014 [3] with a chip rate as defined in this table. |  |  |  |  |  |

For operation in unpaired spectrum, the measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.3.1-2.

Table 6.7.3.5.3.1-2: OTA ACLR in unpaired spectrum with synchronized operation

| Channel bandwidth of E-UTRA lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz |  | OTA ACLR limit for bands between 3 and 4.2GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4, 3.0 | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB |  | 43.8dB |  |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |  |
|  | BWChannel /2 + 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44 dB | 43.8dB |  |
|  | BWChannel /2 + 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44 dB | 43.8dB |  |
| 5, 10, 15, 20 | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB |  | 43.8dB |  |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44 dB | 43.8dB |  |
|  | BWChannel /2 + 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44 dB | 43.8dB |  |
|  | BWChannel /2 + 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44 dB | 43.8dB |  |
|  | BWChannel /2 + 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |  |
|  | BWChannel /2 + 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |  |
|  | BWChannel /2 + 5 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44 dB | 43.8dB |  |
|  | BWChannel /2 + 15 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44 dB | 43.8dB |  |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.014 [3] with a chip rate as defined in this table. |  |  |  |  |  |  |  |

For operation in non-contiguous paired spectrum or multiple bands, the measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.3.1-3.

Table 6.7.3.5.3.1-3: OTA ACLR in non-contiguous paired spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |  |

For operation in non-contiguous unpaired spectrum or multiple bands, the measurement result shall not be less than the OTA ACLR limit specified in Table 6.7.3.5.3.1-4.

Table 6.7.3.5.3.1-4: OTA ACLR in non-contiguous unpaired spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit for bands below 3GHz | OTA ACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44 dB | 43.8dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44 dB | 43.8dB |

For Category A, an E-UTRA AAS BS of Wide Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.3.1-1/2/3/4 or OTA ACLR absolute limit of -4 dBm/MHz shall apply, whichever is less stringent.

For Category B, an E-UTRA AAS BS of Wide Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.3.1-1/2/3/4 or OTA ACLR absolute limit of -6 dBm/MHz shall apply, whichever is less stringent.

For an E-UTRA AAS BS of Medium Range BS class, either the OTA ACLR limits in Tables 6.7.3.5.3.1-1/2/3/4 or OTA ACLR absolute limit of -16 dBm/MHz shall apply, whichever is less stringent.

For an E-UTRA AAS BS of Local Area BS class, either the OTA ACLR limits in Tables 6.7.3.5.3.1-1/2/3/4 or OTA ACLR absolute limit of -23 dBm/MHz shall apply, whichever is less stringent.

###### 6.7.3.5.3.2	OTA Cumulative ACLR test requirement in non-contiguous spectrum

The following test requirement applies for the sub-block or Inter RF Bandwidth gap sizes listed in Table 6.7.3.5.3.2-1,

-	Inside a sub-block gap within an operating band for a BS operating in non-contiguous spectrum.

-	Inside an Inter RF Bandwidth gap for a multi-band capable AAS BS.

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or Inter RF Bandwidth gap; and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in Tables 6.7.3.5.3.2-1 and 6.7.3.5.3.2-2. Filters on the assigned channels are defined in Table 6.7.3.5.3.2-3.

For operation in non-contiguous spectrum or multiple bands, the CACLR for E-UTRA carriers located on either side of the sub-block gap or Inter RF Bandwidth gap shall not be less than the value specified in Tables 6.7.3.5.3.2-1 and 6.7.3.5.3.2-2.

Table 6.7.3.5.3.2-1: OTA CACLR in non-contiguous paired spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit for bands below 3GHz | OTA CACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| 10 MHz < Wgap < 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44 dB | 43.8dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in 3GPP TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |  |

Table 6.7.3.5.3.2-2: OTA CACLR in non-contiguous unpaired spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit for bands below 3GHz | OTA CACLR limit for bands between 3 and 4.2GHz |
| --- | --- | --- | --- | --- | --- |
| 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 5 MHz E-UTRA carrier | Square (BWConfig) | 44 dB | 43.8dB |
| 10 MHz < Wgap < 20 MHz | 7.5 MHz | 5 MHz E-UTRA carrier | Square (BWConfig) | 44 dB | 43.8dB |

Table 6.7.3.5.3.2-3: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| E-UTRA | E-UTRA of same BW |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

For Category A, an E-UTRA AAS BS of Wide Area BS class, either the OTA CACLR limits in Tables 6.7.3.5.3.2-1/2 or OTA ACLR absolute limit of -4 dBm/MHz shall apply, whichever is less stringent.

For Category B, an E-UTRA AAS BS of Wide Area BS class, either the OTA CACLR limits in Tables 6.7.3.5.3.2-1/2 or OTA ACLR absolute limit of -6 dBm/MHz shall apply, whichever is less stringent.

For an E-UTRA AAS BS of Medium Range BS class, either the OTA CACLR limits in Tables 6.7.3.5.3.2-1/2 or OTA ACLR absolute limit of -16 dBm/MHz shall apply, whichever is less stringent.

For an E-UTRA AAS BS of Local Area BS class, either the OTA CACLR limits in Tables 6.7.3.5.3.2-1/2 or OTA ACLR absolute limit of -23 dBm/MHz shall apply, whichever is less stringent.

### 6.7.4	OTA Spectrum emission mask

#### 6.7.4.1	Definition and applicability

This requirement is applicable for AAS BS in single RAT UTRA operation only.

The spectrum emission mask minimum requirements are quoted as TRP unless otherwise stated.

#### 6.7.4.2	Minimum requirement

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.4.3.

This requirement does not apply to single RAT E-UTRA operation or MSR operation.

#### 6.7.4.3	Test purpose

This test measures the emissions of the AAS BS, close to the assigned channel bandwidth of the wanted signal, while the AAS BS is in operation.

#### 6.7.4.4	Method of test

##### 6.7.4.4.1	Initial conditions

###### 6.7.4.4.1.1	General test conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	B, M and T; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1.

###### 6.7.4.4.1.2	UTRA FDD

For an AAS BS declared to be capable of single carrier operation only, set to transmit a signal according to TM1, in subclause 4.12.2.

For a multi-carrier capable AAS BS, set to transmit according to TM1 on all carriers configured using the applicable test configuration.

##### 6.7.4.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Configure theAAS BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

The measurement devices characteristics shall be:

-	a 30 kHz measurement bandwidth.

-	Measurements with an offset from the carrier centre frequency between 2,515 MHz and 4.0 MHz shall use Measurements with an offset from the carrier centre frequency between 4.0 MHz and (f_offsetmax - 500 kHz) shall use a 1 MHz measurement bandwidth.

-	detection mode: true RMS voltage or true power averaging.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

4)	For single carrier operation, set the AAS BS to transmit according to the applicable test configuration in clause 5 using the corresponding test model(s) in subclause 4.12.2 at manufacturers declared rated carrier output power (PRated,c,TRP).

For an AAS BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.11.

5) For UTRA FDD multi-band RIB or RIB operating in non-contiguous spectrum, the emission within the Inter RF Bandwidth or sub-block gap shall be measured using the specified measurement bandwidth from the closest Base Station RF Bandwidth or sub block edge.

In addition, for multi-band RIB, the following steps shall apply:

6)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

7)	Align the BS and the test antenna such that measurements to determine TRP can be performed (see Annex xx)

8)	Sweep the centre frequency of the measurement filter in contiguous steps and measure emission power within the specified frequency ranges with the specified measurement bandwidth.

9)	Repeat step 8-9 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see Annex F).

10)	Calculate TRPEstimate using the measurements made in Step 7.

#### 6.7.4.5	Test Requirement

##### 6.7.4.5.1	UTRA FDD

The measurement result shall not exceed the test requirements specified in Tables 6.7.4.5.1-1 to 6.7.4.5.1-11 for the appropriate Prated,c,TRP, where

-	f is the separation between the carrier frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the carrier frequency and the centre of the measurement filter;

-	f_offsetmax is either 12.5 MHz or the offset to the UMTS Tx band edge as defined in clause 3.4.1, whichever is the greater.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Inside any Inter RF Bandwidth gaps with Wgap < 2×ΔfOBUE for a multi-band RIB, emissions shall not exceed the cumulative sum of the minimum requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The minimum requirements for Base Station RF Bandwidth edge is specified in tables 9.7.3.4.2-1 to 9.7.3.4.2-10 below, where in this case:

-	f is equal to 2.5MHz plus the separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the Base Station RF Bandwidth edge.

-	f_offset is equal to 2.5MHz plus the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is either 12.5 MHz or the offset to the UMTS Tx band edge as defined in section 5.2, whichever is the greater.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band RIB, the operating band unwanted emission minimum requirements apply also in a supported operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported operating band. In this case, no cumulative limit is applied in the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted and

-	In case the inter-band gap between a downlink band with carrier(s) transmitted and a downlink band without any carrier transmitted is less than 2×ΔfOBUE , f_offsetmax shall be the offset to the frequency ΔfOBUE  outside the outermost edges of the two downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from ΔfOBUE  below the lowest frequency, up to ΔfOBUE  above the highest frequency of the downlink operating band without any carrier transmitted.

Inside any sub-block gap for a RIB operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the minimum requirements specified for the adjacent sub blocks on each side of the sub-block gap. The minimum requirement for each sub block is specified in Tables 6.7.4.5.1-1 to 6.7.4.5.1-11 below, where in this case:

-	f is equal to 2.5MHz plus the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is equal to 2.5MHz plus the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub-block gap bandwidth minus half of the bandwidth of the measuring filter plus 2.5MHz.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.7.4.5.1-1: Spectrum emission mask values, P max,c,TRP  49 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -6.2 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz | -6.2 + 15.(f_offset/MHz-2.715)dBm | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -18.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -5.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -5.2 dBm | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirement in subclauses 6.7.6 shall be met. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-2: Spectrum emission mask values, P max,c,TRP  49 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -6 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz | -6 + 15.(f_offset/MHz-2.715)dBm | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -18 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -5 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -5 dBm | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirement in clause 6.7.6 shall be met. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-3: Spectrum emission mask values, 45 dBm  P rated,c,TRP < 49 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -6.2 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz | -6.2 + 15(f_offset/MHz-2.715)dBm | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -18.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -5.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | Prated,c,TRP - 54.2 dB | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirement in subclauses 6.7.6 shall be met. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-4: Spectrum emission mask values, 45 dBm  P rated,c,TRP < 49 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -6 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz | -6 + 15(f_offset/MHz-2.715)dBm | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -18 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -5 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | Prated,c,TRP - 54 dB | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirement in clause 6.7.6 shall be met. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-5: Spectrum emission mask values, 37 dBm  Prated,c,TRP < 45 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point,f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | Prated,c,TRP – 51.2 dB | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz | Prated,c,TRP – 51.2 – 15(f_offset/MHz-2.715) dB | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | Prated,c,TRP – 64.2 dB | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | Prated,c,TRP – 50.2 dB | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | Prated,c,TRP – 54.2 dB | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirement in subclauses 6.7.6 shall be met. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-6: Spectrum emission mask values, 37 dBm  Prated,c,TRP < 45 dBm for UTRA FDD bands >  3 GHz

| Frequency offset of measurement filter -3 dB point,f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | Prated,c,TRP – 51dB | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz | Prated,c,TRP – 51 – 15(f_offset/MHz-2.715) dB | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | Prated,c,TRP – 63 dB | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | Prated,c,TRP – 50 dB | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | Prated,c,TRP – 54 dB | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirement in subclauses 6.7.6 shall be met. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-7: Spectrum emission mask values, Prated,c,TRP < 37 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -14.2 dBm | 30 kHz |
| 2.7 f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz | -14.2 dBm -15(f_offset/MHz-2.715)dB | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -26.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -13.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -17.2 dBm | 1 MHz |
| NOTE 1: For RIB supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirement in subclauses 6.7.6 shall be met. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-8: Spectrum emission mask values, Prated,c,TRP < 37 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -14 dBm | 30 kHz |
| 2.7 f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz | -14 dBm -15(f_offset/MHz-2.715)dB | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -26 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -13 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -17 dBm | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirement in subclauses 6.7.6 shall be met. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

For operation in band II, IV, V, X, XII, XIII, XIV, XXV and XXVI, the applicable additional requirement in tables 6.7.4.5.1-9 to 6.7.4.5.1-11 apply in addition to the test requirements in tables 6.7.4.5.1-1 to 6.7.4.5.1-8.

Table 6.7.4.5.1-9: Additional spectrum emission limits for Bands II, IV, X, XXV

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 3.5 MHz | 2.515 MHz f_offset < 3.515 MHz | -7.2 dBm | 30 kHz |
| 3.5 MHz f fmax | 4.0 MHz f_offset < f_offsetmax | -5.2 dBm | 1 MHz |
| NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-10: Additional spectrum emission limits for Bands V, XXVI

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 3.5 MHz | 2.515 MHz f_offset < 3.515 MHz | -7.2 dBm | 30 kHz |
| 3.5 MHz f fmax | 3.55 MHz f_offset < f_offsetmax | -5.2 dBm | 100 kHz |
| NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.7.4.5.1-11: Additional spectrum emission limits for Bands XII, XIII,  XIV

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.6 MHz | 2.515 MHz f_offset < 2.615 MHz | -5.2 dBm | 30 kHz |
| 2.6 MHz f fmax | 2.65 MHz f_offset < f_offsetmax | -5.2 dBm | 100 kHz |
| NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

In certain regions the following test requirement may apply for protection of DTT. For a RIB operating in Band XX, the level of emissions in the band 470-790 MHz, measured in an 8MHz filter bandwidth on centre frequencies Ffilter according to Table 6.7.4.5.1-12, shall not exceed the maximum emission level TRP in Table 6.7.4.5.1-12.

Table 6.7.4.5.1-12: Emissions levels for protection of DTT

| Case | Measurement filter centre frequency | Condition on BS maximum aggregate EIRP / 10 MHz, PEIRP_10MHz (NOTE) | Maximum Level PEIRP,N,MAX | Measurement Bandwidth |
| --- | --- | --- | --- | --- |
| A: for DTT frequencies where broadcasting is protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz 59 dBm | 1.8 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PEIRP_10MHz < 59 dBm | PEIRP_10MHz – 57.2 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz < 36 dBm | -21.2 dBm | 8 MHz |
| B: for DTT frequencies where broadcasting is subject to an intermediate level of protection | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz 59 dBm | 11.8 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PEIRP_10MHz < 59 dBm | PEIRP_10MHz – 47.2 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz < 36 dBm | -11.2 dBm | 8 MHz |
| C: for DTT frequencies where broadcasting is not protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | N.A. | 23.8 dBm | 8 MHz |
| NOTE: PEIRP_10MHz (dBm) is defined by the expression PEIRP_10MHz = P10MHz + Gant + 6dB for UTRA and PEIRP_10MHz = P10MHz + Gant + 9dB for E-UTRA, where Gant is [17] dBi |  |  |  |  |

NOTE 1:	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The method outlined in annex B1, TS 37.105 [6] indicates how the limit in Table 6.7.4.5.1-12 demonstrates compliance to the regional requirement

In certain regions, the following test requirements may apply to a RIB operating in Band XXXII within 1452-1492 MHz. The level of unwanted emissions, measured on centre frequencies f_offset with filter bandwidth, according to Table 6.7.4.5.1-13, shall not exceed the maximum TRP limits indicated in the table.

Table 6.7.4.5.1-13: Declared frequency band XXXII unwanted emission
within 1452-1492 MHz

| Frequency offset of measurement filter centre frequency, f_offset | Maximum level (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 5 MHz | PEIRP – 17 dBi + 7.8 dB | 5 MHz |
| 10 MHz | PEIRP – 17 dBi + 7.8 dB | 5 MHz |
| 15 MHz ≤ f_offset ≤ f_offsetmax, B32 | PEIRP – 17 dBi + 7.8 dB | 5 MHz |
| NOTE: f_offsetmax, B32 denotes the frequency difference between the lower channel carrier frequency and 1454.5 MHz, and the frequency difference between the upper channel carrier frequency and 1489.5 MHz for the set channel position. |  |  |

NOTE 2:	The regional requirement, included in CEPT ECC Decision (13)03 [xx], is defined in terms of EIRP per antenna, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The method outlined in annex B, TS 37.105 [6] indicates how the limit in Table 6.7.4.5.1-13 demonstrates compliance to the regional requirement.

In certain regions, the following test requirement may apply to RIB operating in Band XXXII within 1452-1492MHz for the protection of services in spectrum adjacent to the frequency range 1452-1492 MHz. The level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to Table 6.7.4.5.1-14, shall not exceed the maximum emission TRP limits in the table. This requirement applies in the frequency range 1429-1518MHz even though part of the range falls in the spurious domain.

Table 6.7.4.5.1-14: Frequency band XXXII declared emission outside 1452-1492 MHz

| Filter centre frequency, Ffilter | Declared emission level (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 1429.5 MHz ≤ Ffilter ≤ 1448.5 MHz | PEIRP – 17 dBi + 7.8dB | 1 MHz |
| Ffilter = 1450.5 MHz | PEIRP – 17 dBi + 7.8dB | 3 MHz |
| Ffilter = 1493.5 MHz | PEIRP – 17 dBi + 7.8dB | 3 MHz |
| 1495.5 MHz ≤ Ffilter ≤ 1517.5 MHz | PEIRP – 17 dBi + 7.8dB | 1 MHz |

NOTE 3:	The regional requirement, included in CEPT ECC Decision (13)03 [19], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The method outlined in annex B, TS 37.105 indicates how the limit in Table 6.7.4.5.1-14 demonstrates compliance to the regional requirement.

### 6.7.5	OTA Operating band unwanted emission

#### 6.7.5.1	Definition and applicability

Unless otherwise stated, for E-UTRA single band and MSR the operating band unwanted emission limits are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band to the lower Base Station RF Bandwidth edge located at FBW RF,low and from the upper Base Station RF Bandwidth edge located at FBW RF,high  up to ΔfOBUE above the highest frequency of each supported downlink operating band.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification.

#### 6.7.5.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.5.2

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.5.4.

This requirement does not apply to single RAT UTRA operation.

#### 6.7.5.3	Test purpose

This test measures the emissions of the AAS BS, close to the assigned channel bandwidth of the wanted signal, while the AAS BS is in operation.

#### 6.7.5.4	Method of test

##### 6.7.5.4.1	Initial conditions

Test environment:

-	normal; annex G.2.

RF channels to be tested for single carrier:

-	B, M and T; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.12.1

##### 6.7.5.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Configure theAAS BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

The measurement devices characteristics shall be:

-	detection mode: true RMS voltage or true power averaging.

4)	Set the AAS BS to transmit:

a)	For MSR:

-	Set the AAS BS to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.12.

b)	For E-UTRA:

-	AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit a signal according to E-TM1.1 (subclause 4.12.2) at manufacturer's declared rated carrier output power (PRated,c,TRP).

-	For an AAS BS declared to be capable of multi-carrier and/or CA operation, set the set the AAS BS to transmit according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

5)	Align the BS and the test antenna such that measurements to determine TRP can be performed (see Annex xx)

6)	Sweep the centre frequency of the measurement filter in contiguous steps and measure emission power within the specified frequency ranges with the specified measurement bandwidth.

7)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex F).

8)	Calculate TRPEstimate using the measurements made in Step 7.

9)	Repeat the test for the remaining test cases:

a)	For MSR with channel set-up according to clause 5 and subclause 4.12.2.

b)	For E-UTRA with the channel set-up according to E-TM 1.2

In addition, for multi-band RIB, the following steps shall apply:

10)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.7.5.5	Test Requirement

##### 6.7.5.5.1	General

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.7.5.5.2	MSR Band categories 1 and 3

For an AAS BS of Wide Area BS class operating in Band Category 1 or Band Category 3, the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Wide Area BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Wide Area BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

For an AAS BS of Medium Range BS class operating in Band Category 1 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Medium Range BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Medium Range BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

For an AAS BS of Local Area BS class operating in Band Category 1 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Local Area BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Local Area BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

Outside the Base Station RF Bandwidth edges, emissions shall not exceed the maximum levels specified in Tables 6.7.5.5.2-1 to 6.7.5.5.2-8 below, where:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB RIB, inside any Inter RF Bandwidth gaps with Wgap < 2×ΔfOBUE MHz, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The minimum requirement for Base Station RF Bandwidth edge is specified in Tables 6.7.5.5.2-1 to 6.7.5.5.2-8, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band RIB, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in another operating band. In this case where there is no carrier transmitted in an operating band, no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted and

-	In case the Inter RF Bandwidth gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted is less than 2×ΔfOBUE MHz, f_offsetmax shall be the offset to the frequency ΔfOBUE MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both supported downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from ΔfOBUE MHz below the lowest frequency, up to ΔfOBUE MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

Inside any sub-block gap for a RIB operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The minimum requirement for each sub block is specified in Tables 6.7.5.5.2-1 to 6.7.5.5.2-8, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge frequency.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.7.5.5.2-1: Wide Area operating band unwanted emission mask (UEM) for BC1 and BC3 bands ≤ 3GHz for BS not supporting NR (except for BS operating in Band n1)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015 MHz f_offset < 0.215 MHz | -3.2 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz | -3.2-15(f_offset/MHz-0.215)dBm | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | -15.2 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -2.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (NOTE 5) | 1 MHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/MHz. NOTE 2: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.2-2: Wide Area operating band unwanted emission mask (UEM) for BC1 and BC3 bands > 3GHz for BS not supporting NR (except for BS operating in Band n1)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015 MHz f_offset < 0.215 MHz | -3 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz | -3-15(f_offset/MHz-0.215)dBm | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | -15 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (NOTE 5) | 1 MHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/MHz. NOTE 2: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.2-2a: Wide Area operating band unwanted emission mask (UEM) for BS supporting NR and not supporting UTRA in BC1 and BC3 bands below 1GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm – 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (Note 5) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -7dBm/100kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.7.5.5.2-2b: Wide Area operating band unwanted emission mask (UEM) for BS supporting NR (except operation in Band n1) and not supporting UTRA in BC1 and BC3 bands > 3GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -3.8 dBm – 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 5) | 1MHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -6dBm/1MHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.7.5.5.2-3: Medium Range BS operating band unwanted emission mask (UEM)
for BC1 for bands ≤ 3 GHz, 40 < Prated,c,TRP  47 dBm for BS not supporting NR

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c,TRP-56.2dB-(7/5)*(f_offset-0,015)dB | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz | Prated,c,TRP-51.2dB-(7/5)*(f_offset-0,015)dB | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | Prated,c,TRP – 63.2 dB | 30 kHz |
| 1 MHz f 2.6 MHz | 1.5 MHz f_offset < 3.1 MHz | Prated,c,TRP –50.2 dB | 1 MHz |
| 2.6 MHz f 5 MHz | 3.1 MHz f_offset < 5.5 MHz | min(Prated,c,TRP – 50.2 dB, -4.2dBm) | 1 MHz |
| 5 MHz f min(fmax, 10 MHz) | 5.5 MHz f_offset < min (f_offsetmax, 10.5 MHz) | Prated,c,TRP –54.2 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Prated,c,TRP-56 dB | 1 MHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be (Prated,c,TRP - 56 dB)/MHz. NOTE 2: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.2-3a: Medium Range BS operating band unwanted emission mask (UEM) for BS supporting NR and not supporting UTRA in BC1 bands ≤ 3GHz, BS maximum output power 40 < Prated,c,TRP  47 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP-51.2dB-(7/5)*(f_offset-0,05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP-58.2dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP-60dB, -16dBm) (Note 5) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be Min(Prated,c,TRP -60dB, -16dBm)/100kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. |  |  |  |

Table 6.7.5.5.2-4: Medium Range BS operating band unwanted emission mask (UEM)
for BC1 for bands > 3 GHz, 40 < Prated,c,TRP  47 dBm for BS not supporting NR

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c,TRP-56dB-(7/5)*(f_offset-0,015)dB | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz | Prated,c,TRP-51dB-(7/5)*(f_offset-0,015)dB | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | Prated,c,TRP – 63 dB | 30 kHz |
| 1 MHz f 2.6 MHz | 1.5 MHz f_offset < 3.1 MHz | Prated,c,TRP –50 dB | 1 MHz |
| 2.6 MHz f 5 MHz | 3.1 MHz f_offset < 5.5 MHz | min(Prated,c,TRP – 50 dB, -4dBm) | 1 MHz |
| 5 MHz f min(fmax, 10 MHz) | 5.5 MHz f_offset < min(f_offsetmax ,10.5 MHz) | Prated,c,TRP –54 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Prated,c,TRP-56 dB | 1 MHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be (Prated,c,TRP - 56 dB)/MHz. NOTE 2: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.2-4a: Medium Range BS operating band unwanted emission mask (UEM) for BS supporting NR and not supporting UTRA in BC1 bands > 3GHz, BS maximum output power 40 < Prated,c,TRP  47 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP – 51dB - 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP -58 dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP -60dB, -16dBm) (Note 5) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be Min(Prated,c,TRP -60dB, -16dBm)/100kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. |  |  |  |

Table 6.7.5.5.2-5: Medium Range BS operating band unwanted emission mask (UEM)
for BC1 for bands≤ 3 GHz, Prated,c,TRP  40 dBm for BS not supporting NR

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | -7.2-5/3(f_offset/MHz-0.015)dBm | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz | -2.2-15(f_offset/MHz-0.015)dBm | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | -23.2 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -10.2 dBm | 1 MHz |
| 5 MHz f min(fmax,10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5 MHz) | -14.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -16dBm (Note 5) | 1 MHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -16 dBm/MHz. NOTE 2: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.2-5a: Medium Range BS operating band unwanted emission mask (UEM) for BS supporting NR and not supporting UTRA in BC1 bands ≤ 3GHz, BS maximum output power Prated,c,TRP  40 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -11.2 dBm – 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -18.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (Note 8) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -20dBm/100kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. |  |  |  |

Table 6.7.5.5.2-6: Medium Range BS operating band unwanted emission mask (UEM)
for BC1 for bands> 3 GHz, Prated,c,TRP   40 dBm for BS not supporting NR

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | -7-5/3(f_offset/MHz-0.015)dBm | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz | -2-15(f_offset/MHz-0.015)dBm | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | -23 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -10 dBm | 1 MHz |
| 5 MHz f min(fmax,10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5 MHz) | -14 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -16dBm (Note 5) | 1 MHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -16 dBm/MHz. NOTE 2: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.2-6a: Medium Range BS operating band unwanted emission mask (UEM) for BS supporting NR and not supporting UTRA in BC1 bands > 3GHz, BS maximum output power Prated,c,TRP  40 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -11 dBm – 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -18 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (Note 5) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -20dBm/100kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. |  |  |  |

Table 6.7.5.5.2-7: Local Area operating band unwanted emission mask (UEM)
for BC1 for bands  3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -19.2-7/5(f_offset/MHz – 0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -26.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -28dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -28 dBm/100 kHz. NOTE 2: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.2-8: Local Area operating band unwanted emission mask (UEM)
for BC1 for bands > 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -19-7/5(f_offset/MHz – 0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -26 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -28dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -28 dBm/100 kHz. NOTE 2: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

##### 6.7.5.5.3	MSR Band Category 2

For a RIB operating in Band Category 2 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a RIB operating in non-contiguous spectrum, it applies inside any sub-block gap.

Outside the Base Station RF Bandwidth edges, emissions shall not exceed the maximum levels specified in Tables 6.7.5.5.3-1 to 6.7.5.5.3-8, where:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band RIB, inside any Inter RF Bandwidth gaps with Wgap < 2×ΔfOBUE MHz, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The minimum requirement for Base Station RF Bandwidth edge is specified in Tables 6.7.5.5.3-1 to 6.7.5.5.3-8, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band RIB and where there is no carrier transmitted in an operating band, no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted and

-	In case the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted less than is 2×ΔfOBUE MHz, f_offsetmax shall be the offset to the frequency ΔfOBUE MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both supported downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from ΔfOBUE MHz below the lowest frequency, up to ΔfOBUE MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

Inside any sub-block gap for a RIB operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the test requirement specified for the adjacent sub blocks on each side of the sub block gap. The minimum requirement for each sub block is specified in Tables 6.7.5.5.3-1 to 6.7.5.5.3-8, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.7.5.5.3-1: Wide Area operating band unwanted emission mask (UEM) for BC2 for BS not supporting NR (except for BS operating in Band n3 or n8) Wide

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz (Note 1) | 0.015 MHz f_offset < 0.215 MHz | -3.2 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz | -3.2-15(f_offset/MHz-0.215) dBm | 30 kHz |
| (Note 8) | 1.015 MHz f_offset < 1.5 MHz | -15.2 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -2.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 10) | 1 MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.7.5.5.3-2 apply for 0 MHz f < 0.15 MHz. NOTE 2: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -6 dBm/MHz. NOTE 3: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz operation the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.7.5.5.3-2: Wide Area BS operating band unwanted emission limits for operation in BC2
with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 2, 3 and 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz | Max(15.8dBm-60(f_offset/MHz-0.015)+XdB, -3.2 dBm) | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz | Max(12.8dBm-160(f_offset/MHz-0.065)+XdB, -3.2 dBm) | 30 kHz |
| NOTE 1: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 3: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 4: In case the carrier adjacent to the Base Station RF Bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier - 43, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the Base Station RF Bandwidth edge. In other cases, X = 0. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.7.5.5.3-2a: Wide Area operating band unwanted emission mask (UEM) for BS supporting NR (except operation in Band n8) and not supporting UTRA in BC2 bands below 1GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm – 7/5(f_offset/MHz – 0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (Note 10) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -7dBm/100kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: For operation with an E-UTRA 1.4 or 3MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in Table 6.7.5.5.3-2 apply for 0 MHz f < 0.15 MHz. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.3-2b: Wide Area operating band unwanted emission mask (UEM) for BS supporting NR (except operation in Band n3) and not supporting UTRA in BC2 bands above 1GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm – 7/5(f_offset/MHz – 0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 10) | 1MHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -6dBm/1MHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: For operation with an E-UTRA 1.4 or 3MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in Table 6.7.5.5.3-2 apply for 0 MHz f < 0.15 MHz. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.3-3: Medium Range BS operating band unwanted emission mask (UEM)
for BC2, 40 < Prated,c,TRP   47 dBm for BS not supporting NR

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz (Note 1) | 0.015 MHz f_offset < 0.615 MHz | Prated,c,TRP-56.2dB-(5/3)*(f_offset-0,015)dB | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz | Prated,c,TRP-51.2dB-15*(f_offset-0,215)dB | 30 kHz |
| (Note 8) | 1.015 MHz f_offset < 1.5 MHz | Prated,c,TRP - 63.2 dB | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | Prated,c,TRP - 50.2 dB | 1 MHz |
| 2.8 MHz f 5 MHz | 3.3 MHz f_offset < 5.5 MHz | min(Prated,c,TRP - 50.2 dB, -4.2dBm) | 1 MHz |
| 5 MHz f min(fmax, 10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5 MHz) | Prated,c,TRP - 54.2 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Prated,c,TRP -56 dB (Note 10) | 1 MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in Table 6.7.5.5.3-5 apply for 0 MHz f < 0.15 MHz. NOTE 2: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be (Prated,c,TRP - 56 dB)/MHz. NOTE 3: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.7.5.5.3-3a: Medium Range BS operating band unwanted emission mask (UEM) for BS supporting NR and not supporting UTRA in BC2 bands, BS maximum output power 40 < Prated,c,TRP   47 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP – 51.2dB - 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP -58.2dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP -60dB, -16dBm) (Note 10) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be Min(Prated,c,TRP -60dB, -16dBm)/100kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For operation with an E-UTRA 1.4 or 3MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in Table 6.7.5.5.3-5 apply for 0 MHz f < 0.15 MHz. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.3-4: Medium Range BS operating band unwanted emission mask (UEM)
for BC2, Prated,c,TRP  40 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz (Note 1) | 0.015 MHz f_offset < 0.615 MHz | -16.2dBm-5/3(f_offset/MHz-0.015)dB | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz | -11.2dBm-15(f_offset/MHz-0.215)dB | 30 kHz |
| (Note 8) | 1.015 MHz f_offset < 1.5 MHz | -23.2 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -10.2 dBm | 1 MHz |
| 5 MHz f min(fmax,10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5 MHz) | -14.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -16 dBm (Note 10) | 1 MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.7.5.5.3-6 apply for 0 MHz f < 0.15 MHz. NOTE 2: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -16 dBm/MHz. NOTE 3: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.7.5.5.3-4a: Medium Range BS operating band unwanted emission mask (UEM) for BS supporting NR and not supporting UTRA in BC2 bands, BS maximum output power Prated,c,TRP  40 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -11.2 dBm – 7/5(f_offset/MHz-0.05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -18.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (Note 8) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -20dBm/100kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For operation with an E-UTRA 1.4 or 3MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in Table 6.7.5.5.3-6 apply for 0 MHz f < 0.15 MHz. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.3-5: Medium Range operating band unwanted emission limits for operation in BC2
with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge,
40 < Prated,c,TRP  47 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz | Prated,c,TRP-36.2-60(f_offset-0.015 | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz | Prated,c,TRP-39.2-160(f_offset-0.065) | 30 kHz |
| NOTE 1: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 3: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.7.5.5.3-6: Medium Range operating band unwanted emission limits for operation in BC2
with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge,
Prated,c,TRP  40 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 2, 3 and 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz | Max(3.8-60(f_offset/MHz-0.015)+X, -16.2) dBm | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz | Max(-0.8-160(f_offset/MHz-0.065)+X, -16.2) dBm | 30 kHz |
| NOTE 1: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 3: For multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 4: In case the carrier adjacent to the Base Station RF Bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier - 31, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the Base Station RF Bandwidth edge. In other cases, X = 0. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.7.5.5.3-7: Local Area operating band unwanted emission mask (UEM) for BC2

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz (Note 1) | 0.05 MHz f_offset < 5.05 MHz | -19.2-7/5(f_offset/MHz-0.05) dBm | 100 kHz |
| 5 MHz f < min (10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -26.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -28 dBm (Note 7) | 100 kHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.6.5.5.3-8 apply for 0 MHz f < 0.16 MHz. NOTE 2: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -28 dBm/100 kHz. NOTE 3: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.7.5.5.3-8: Local Area operating band unwanted emission limits for operation in BC2 with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 2, 3 and 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz | Max(-3.2dBm-60(f_offset/MHz-0.015)dB + X dB, -24.2dBm) | 30 kHz |
| 0.05 MHz f < 0.16 MHz | 0.065 MHz f_offset < 0.175 MHz | max(-6.2dBm-160(f_offset/MHz-0.065)dB + X dB, -24.2 dBm) | 30 kHz |
| NOTE 1: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For MSR RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 3: For MSR multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 4: In case the carrier adjacent to the Base Station RF Bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier - 24, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the Base Station RF Bandwidth edge. In other cases, X = 0. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

NOTE 9:	As a general rule for the requirements in the present subclause, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.7.5.5.4	MSR Additional requirements

###### 6.7.5.5.4.1	Limits in FCC Title 47

In addition to the requirements in subclauses 6.7.5.5.2 and 6.7.5.5.3, the AAS BSW may have to comply with the applicable emission limits established by FCC Title 47 [18], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

###### 6.7.5.5.4.2	Unsynchronized operation for BC3

In certain regions, the following requirements may apply to a TDD AAS BS operating in BC3 in the same geographic area and in the same operating band as another TDD system without synchronisation. For this case the emissions shall not exceed -52 dBm/MHz in each supported downlink operating band except in:

-	The frequency range from 10 MHz below the lower Base Station RF Bandwidth edge to the frequency 10 MHz above the upper Base Station RF Bandwidth edge of each supported band.

NOTE 1:	Local or regional regulations may specify another excluded frequency range, which may include frequencies where synchronised TDD systems operate.

NOTE 2:	TDD Base Stations that are synchronized and operating in BC3 can transmit without these additional co-existence requirements.

###### 6.7.5.5.4.3	Protection of DTT

In certain regions the following requirement may apply for protection of DTT. For an AAS BS operating in Band 20/n20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to table 6.7.5.5.4.3-1, shall not exceed the maximum emission level TRP level shown in the table. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.7.5.5.4.3-1: Declared emissions levels for protection of DTT

| Case | Measurement filter centre frequency | Condition on BS maximum aggregate TRP / 10 MHz, PTRP_10MHz (NOTE) | Maximum Level PTRP,N,MAX | Measurement Bandwidth |
| --- | --- | --- | --- | --- |
| A: for DTT frequencies where broadcasting is protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PTRP_10MHz 59 dBm | 1.8dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PTRP_10MHz < 59 dBm | PTRP_10MHz – 57.2 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PTRP_10MHz < 36 dBm | -21.2dBm | 8 MHz |
| B: for DTT frequencies where broadcasting is subject to an intermediate level of protection | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PTRP_10MHz 59 dBm | 11.8 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PTRP_10MHz < 59 dBm | PTRP_10MHz – 47.2 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PTRP_10MHz < 36 dBm | -11.2dBm | 8 MHz |
| C: for DTT frequencies where broadcasting is not protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | N.A. | 23.8 dBm | 8 MHz |
| NOTE: PTRP_10MHz (dBm) is defined by the expression PTRP_10MHz = P10MHz + Gant + 6dB for UTRA and PTRP_10MHz = P10MHz + Gant + 9dB for E-UTRA, where Gant is 17 dBi |  |  |  |  |

NOTE:	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions and the deployment (including antenna gain and feeder loss). The method outlined in annex B1, 3GPP TS 37.105 [6] indicates how the limit in Table 6.7.5.5.4.3-1 demonstrates compliance to the regional requirement.

##### 6.7.5.5.5	E-UTRA

###### 6.7.5.5.5.1	General

The measurement results in subclause 6.7.5.4 shall not exceed the maximum levels specified in the tables below, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band RIB, inside any Inter RF Bandwidth gaps with Wgap < 2×ΔfOBUE MHz, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The test requirement for Base Station RF Bandwidth edge is specified in Tables 6.7.5.5.5.2-1 to 6.7.5.5.5.2-9, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the Base Station RF Bandwidth edge.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For multi-band RIB, the operating band unwanted emission limits apply also in a supported operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported operating band. In this case, no cumulative limit is applied in the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted and:

-	In case the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted is less than 2×ΔfOBUE MHz, f_offsetmax shall be the offset to the frequency ΔfOBUE MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from ΔfOBUE MHz below the lowest frequency, up to ΔfOBUE MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

For a multicarrier E-UTRA TAB connector or a RIB configured for intra-band contiguous or non-contiguous carrier aggregation the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the upper edge of the carrier transmitted at the highest carrier frequency within a specified frequency band.

In addition inside any sub-block gap for a RIB operating in non-contiguous spectrum, measurement results shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The minimum requirement for each sub block is specified in Tables 6.7.5.5.5.2-1 to 6.7.5.5.5.2-9, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

###### 6.7.5.5.5.2	Wide Area BS (Category A)

For E-UTRA RIB operating in Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 26, 27, 28, 29, 31, 44, 68 emissions shall not exceed the maximum levels specified in Tables 6.7.5.5.5.2-1 to 6.7.5.5.5.2-3.

Table 6.7.5.5.5.2-1: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | 9.8dBm-10/1.4(f_offset/MHz-0.05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -0.2 dBm | 100 kHz |
| 2.8 MHz f min(10 MHz, fmax) | 2.85 MHz f_offset < min(10.05 MHz, f_offsetmax) | -2.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -4 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.2-2: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | 5.8dBm-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -4.2 dBm | 100 kHz |
| 6 MHz f min(10 MHz, fmax) | 6.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -2.2 dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -4 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.2-3: Wide Area BS operating band unwanted emission limits for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -4 dBm (NOTE 9) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For E-UTRA RIB operating in Bands 1, 2, 3, 4, 7, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 45, 48, 65, 66, 69, 70, emissions shall not exceed the maximum levels specified in tables 6.6.5.5.5.2-4, 6.6.5.5.5.2-6 and 6.6.5.5.5.2-8.

For E-UTRA RIB operating in Bands 22, 42, 43, emissions shall not exceed the maximum levels specified in tables 6.6.5.5.5.2-5, 6.6.5.5.5.2-7 and 6.6.5.5.5.2-9.

Table 6.7.5.5.5.2-4: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | 9.8dBm-10/1.4(f_offset/MHz-0.05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -0.2 dBm | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 3.3 MHz f_offset < min(10.5 MHz, f_offsetmax) | -2.2 (Note 8) dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -4 dBm (NOTE 8) | 1MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.2-5: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | 10dBm-10/1.4(f_offset/MHz-0.05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | 0 dBm | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 3.3 MHz f_offset < min(10.5 MHz, f_offsetmax) | -2 (Note 8) dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -4 dBm (NOTE 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.2-6: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | 5.8dBm-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -4.2 dBm | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -2.2 dBm (Note 8) | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | 4 dBm (Note 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirmeent within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.2-7: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | 6dBm-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -4.0 dBm | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -2.0 dBm (Note 8) | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -4 dBm (Note 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.2-8: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -4 dBm (NOTE 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.2-9: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 4dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -4 dBm (NOTE 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

###### 6.7.5.5.5.3	Wide Area BS Category B (Option1)

For Category B Operating band unwanted emissions, there are two options for the limits that may be applied regionally, option 1 is as follows.  For E-UTRA AAS BS operating in Bands 5, 8, 12, 13, 14, 17, 20, 26, 27, 28, 29, 31, 44, 67, 68 emissions shall not exceed the maximum levels specified in Tables 6.7.5.5.5.3-1 to 6.7.5.5.5.3-3.

Table 6.7.5.5.5.3-1: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | 9.8dBm-10/1.4(f_offset/MHz-0.05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -0.2 dBm | 100 kHz |
| 2.8 MHz f min(10 MHz, fmax) | 2.85 MHz f_offset < min(10.05 MHz, f_offsetmax) | -5.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -7 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.3-2: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | 5.8-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -4.2 dBm | 100 kHz |
| 6 MHz f min(10 MHz, fmax) | 6.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -5.2 dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (Note 8) | 100kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -7 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

Table 6.7.5.5.5.3-3: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (NOTE 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -7 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For E-UTRA RIB operating in Bands 1, 2, 3, 4, 7, 10, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 45, 48, 65, 66, 69, 70, emissions shall not exceed the maximum levels specified in tables 6.6.5.5.5.3-4, 6.6.5.5.5.3-6 and 6.6.5.5.5.3-8.

For E-UTRA RIB operating in Bands 22, 42, 43, emissions shall not exceed the maximum levels specified in tables 6.6.5.5.5.3-5, 6.6.5.5.5.3-7 and 6.6.5.5.5.3-9.

Table 6.7.5.5.5.3-4: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | 9.8dBm-10/1.4(f_offset/MHz-0,05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -0.2 dBm | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 3.3 MHz f_offset < min(10.5 MHz, f_offsetmax) | -4.2 dBm (Note 8) | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 8) | 1MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.3-5: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | 10dBm-10/1.4(f_offset/MHz-0,05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | 0 dBm | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 3.3 MHz f_offset < min(10.5 MHz, f_offsetmax) | -4 dBm (Note 8) | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 8) | 1MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.3-6: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | 5.8 dBm-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -4.2 dBm | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -4.2 dBm (Note 8( | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 8) | 1MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.3-7: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | 6 dBm-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -4 dBm | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -4 dBm (Note 8) | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.3-8: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (NOTE 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.3-9: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 4dBm-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (NOTE 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

###### 6.7.5.5.5.4	Wide Area BS Category B (Option 2)

For Category B Operating band unwanted emissions, there are two options for the limits that may be applied regionally, option 1 is as follows.

The limits in this subclause are intended for Europe and may be applied regionally for a RIB operating in band 1, 3, 8, 32, 33, 34 or 65.

For a RIB operating in band 1, 3, 8, 32, 33, 34 or 65, emissions shall not exceed the maximum levels specified in table 6.6.5.5.5.4-1 below for 5, 10, 15 and 20 MHz channel bandwidth.

Table 6.7.5.5.5.4-1: Regional Wide Area BS operating band unwanted emission limits
in band 1, 3, 8, 32, 33, 34 or 65 for 5, 10, 15 and 20 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015 MHz f_offset < 0.215 MHz | -3.2 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz | -3.2-15(f_offset/MHz-0.215)dB | 30 kHz |
| (Note 7) | 1.015 MHz f_offset < 1.5 MHz | -15.2 dBm | 30 kHz |
| 1 MHz f min( 10 MHz , fmax) | 1.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -2.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (NOTE 7) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 7: This frequency range ensures that the range of values of f_offset is continuous. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For a RIB operating in band 3, 8 or 65, emissions shall not exceed the maximum levels specified in table 6.7.5.5.5.4-2 for 3 MHz channel bandwidth.

Table 6.7.5.5.5.4-2: Regional Wide Area BS operating band unwanted emission limits
in band 3, 8 or 65 for 3 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz | 15.8dBm-60(f_offset_MHz-0.015)dB | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0. 065 MHz f_offset < 0.165 MHz | 12.8dBm-160(f_offset/MHz-0.065) dB | 30 kHz |
| 0.15 MHz f < 0.2 MHz | 0.165 MHz f_offset < 0.215 MHz | -3.2 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz | -3.2 dBm-15(f_offset/MHz-0.215)dB | 30 kHz |
| (Note 7) | 1.015 MHz f_offset < 1.5 MHz | -15.2 dBm | 30 kHz |
| 1 MHz f 6 MHz | 1.5 MHz f_offset < 6.5 MHz, | -2.2 dBm | 1 MHz |
| 6 MHz f < min(10 MHz, fmax) | 6.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -4.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 8) |  |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 7: This frequency range ensures that the range of values of f_offset is continuous. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For a BS operating in band 3, 8 or 65, emissions shall not exceed the maximum levels specified in table 6.7.5.5.5.4-3 for 1.4 MHz channel bandwidth.

Table 6.7.5.5.5.4-3: Regional Wide Area BS operating band unwanted emission limits
in band 3, 8 or 65 for 1.4 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz | 15.8dBm-60(f_offset/MHz-0.015)dB | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0. 065 MHz f_offset < 0.165 MHz | 12.8dBm-160(f_offset/MHz-0.065)dB | 30 kHz |
| 0.15 MHz f < 0.2 MHz | 0.165 MHz f_offset < 0.215 MHz | -3.2 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz | -3.2-15(f_offset/MHz-0.215)dB | 30 kHz |
| (Note 7) | 1.015 MHz f_offset < 1.5 MHz | -15.2 dBm | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | -2.2 dBm | 1 MHz |
| 2.8 MHz f < min(10 MHz, fmax) | 3.3 MHz f_offset < min(10.5 MHz, f_offsetmax) | -4.2 dBm (Note 8) | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 8) | 1 MHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 7: This frequency range ensures that the range of values of f_offset is continuous. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

###### 6.7.5.5.5.5	Local Area BS (Category A and B)

For an AAS BS of Local Area BS class in E-UTRA bands ≤ 3 GHz, emissions shall not exceed the maximum levels specified in Tables 6.7.5.5.5.5-1, 6.7.5.5.5.5-3 and 6.7.5.5.5.5-5.

For an AAS BS of Local Area BS class in E-UTRA bands > 3 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.5.5.5.5-2, 6.7.5.5.5.5-4 and 6.7.5.5.5.5-6.

Table 6.7.5.5.5.5-1: Local Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | -10.2-10/1.4(f_offset/MHz-0.05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -20.2 dBm | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 2.85 MHz f_offset < min(10.05 MHz, f_offsetmax) | -20.2 dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -22 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -22 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.5-2: Local Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | -10-10/1.4(f_offset/MHz-0.05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -20 dBm | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 2.85 MHz f_offset < min(10.05 MHz, f_offsetmax) | -20 dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -22 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -22 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.5-3: Local Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | -14.2-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -24.2 dBm | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -24.2 dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -26 (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -26 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.5-4: Local Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | -14-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -24 dBm | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -24 dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -26 (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -26 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.5-5: Local Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -19.2-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -26.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -28 dBm (NOTE 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -28 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.5-6: Local Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -19-7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -26 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -28 dBm (NOTE 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -28 dBm/100 kHz. NOTE 2: For BS a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

###### 6.7.5.5.5.6	Medium Range BS (Category A and B)

For an AAS BS of Medium Range BS class in E-UTRA bands ≤ 3 GHz, emissions shall not exceed the maximum levels specified in Tables 6.7.5.5.5.6-1, 6.7.5.5.5.6-3, 6.7.5.5.5.6-5, 6.7.5.5.5.6-7, 6.7.5.5.5.6-9 and 6.7.5.5.5.6-11.

For an AAS BS of Medium Range BS class in E-UTRA bands > 3 GHz, emissions shall not exceed the maximum levels specified in Tables 6.7.5.5.5.6-2, 6.7.5.5.5.6-4, 6.7.5.5.5.6-6, 6.7.5.5.5.6-8, 6.7.5.5.5.6-10 and 6.7.5.5.5.6-12.

Table 6.7.5.5.5.6-1: Medium Range BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth, 40 < Prated,c,TRP  47 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | Prated,c,TRP-43.2dB-(10/1.4)*(f_offset-0,05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | Prated,c,TRP -53.2dB | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 2.85 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14.2dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -16 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-2: Medium Range BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth, 40 < Prated,c,TRP  47 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | Prated,c,TRP-43dB-(10/1.4)*(f_offset-0,05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | Prated,c,TRP -53dB | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 2.85 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -25 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-3: Medium Range BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth, Prated,c,TRP  40 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | -3.2 dBm-10/1.4(f_offset/MHz-0.05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -13.2 dBm | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 2.85 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14.2dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -16 dBm/100 kHz. NOTE 2: For BS a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-4: Medium Range BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth, Prated,c,TRP  40 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz | -3 dBm-10/1.4(f_offset/MHz-0.05)dB | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -13 dBm | 100 kHz |
| 2.8 MHz f < min(10 MHz, fmax) | 2.85 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -16 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-5: Medium Range BS operating band unwanted emission limits
for 3 MHz channel bandwidth, 40 < Prated,c,TRP  47 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | Prated,c,TRP-47.2dB-(10/3)*(f_offset-0,05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | Prated,c,TRP –57.2dB | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Min(Prated,c,TRP –57.2dB, -14.2dBm) (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP –59dB, -16dBm) (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be Min(Prated,c,TRP -59dB, -16 dBm)/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-6: Medium Range BS operating band unwanted emission limits
for 3 MHz channel bandwidth, 40 < Prated,c,TRP  47 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | Prated,c,TRP-47dB-(10/3)*(f_offset-0,05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | Prated,c,TRP –57dB | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Min(Prated,c,TRP –57dB, -14dBm) (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP –59dB, -16dBm) (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be Min(Prated,c,TRP -59dB, -16 dBm)/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-7: Medium Range BS operating band unwanted emission limits
for 3 MHz channel bandwidth, Prated,c,TRP  40 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | -7.2 dBm-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -17.2 dBm | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -17.2 dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -19 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -19 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-8: Medium Range BS operating band unwanted emission limits
for 3 MHz channel bandwidth, Prated,c,TRP  40 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz | -7 dBm-10/3(f_offset/MHz-0.05)dB | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -17 dBm | 100 kHz |
| 6 MHz f < min(10 MHz, fmax) | 6.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -17 dBm (Note 8) | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -19 dBm (Note 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -19 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-9: Medium Range BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth, 40 < Prated,c,TRP  47 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP-51.2dB-(7/5)*(f_offset-0,05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP – 58.2dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP – 60dB, -16dBm) (NOTE 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be Min(Prated,c,TRP -60 dB, -16 dBm)/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-10: Medium Range BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth, 40 < Prated,c,TRP  47 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP-51dB-(7/5)*(f_offset-0,05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP – 58dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP – 60dB, -16dBm) (NOTE 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be Min(Prated,c,TRP -60 dB, -16 dBm)/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-11: Medium Range BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth, Prated,c,TRP  40 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP-11.2dB-(7/5)*(f_offset-0,05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -18.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (NOTE 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -20dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.7.5.5.5.6-12: Medium Range BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth, Prated,c,TRP  40 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP-11dB-(7/5)*(f_offset-0,05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -18 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (NOTE 8) | 100 kHz |
| NOTE 1: For a RIB supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -20 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

###### 6.7.5.5.5.7	Additional requirements

In certain regions the following test requirement may apply. For E-UTRA RIB operating in Bands 5, 26, 27 or 28, emissions shall not exceed the maximum levels specified in Table 6.7.5.5.5.7-1.

Table 6.7.5.5.5.7-1: Additional operating band unwanted emission limits for E-UTRA bands < 1 GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz f < 1 MHz | 0.005 MHz f_offset < 0.995 MHz | -12.2 dBm | 10 kHz |
| 3 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -11.2dBm | 30 kHz |
| 5 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -13.2 dBm | 30 kHz |
| 10 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -11.2 dBm | 100 kHz |
| 15 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -11.2 dBm | 100 kHz |
| 20 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -11.2 dBm | 100 kHz |
| All | 1 MHz f < min(10 MHz, fmax) | 1.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -11.2dBm (Note 8) | 100 kHz |
| All | 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 100 kHz |
| NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |  |

In certain regions the following test requirement may apply. For E-UTRA a RIB operating in Bands 2, 4, 10, 23, 25, 30, 35, 36, 41, 66, emissions shall not exceed the maximum levels specified in Table 6.7.5.5.5.7-2.

Table 6.7.5.5.5.7-2: Additional operating band unwanted emission limits for E-UTRA bands > 1GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz f < 1 MHz | 0.005 MHz f_offset < 0.995 MHz | -12.2 dBm | 10 kHz |
| 3 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -11.2 dBm | 30 kHz |
| 5 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -13.2 dBm | 30 kHz |
| 10 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -11.2 dBm | 100 kHz |
| 15 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13.2 dBm | 100 kHz |
| 20 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -14.2 dBm | 100 kHz |
| All | 1 MHz f < min(10 MHz, fmax) | 1.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -11.2 dBm (Note 8) | 1 MHz |
| All | 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 1 Mhz |
| NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |  |

In certain regions the following test requirement may apply. For E-UTRA a RIB operating in Bands 12, 13, 14, 17, 29 emissions shall not exceed the maximum levels specified in Table 6.7.5.5.5.7-3.

Table 6.7.5.5.5.7-3: Additional operating band unwanted emission limits
for E-UTRA (bands 12, 13, 14, 17 and 29)

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| All | 0 MHz f < 100 kHz | 0.015 MHz f_offset < 0.085 MHz | -11.2 dBm | 30 kHz |
| All | 100 kHz f < min(10 MHz, fmax) | 150 kHz f_offset < min(10.05 MHz, f_offsetmax) | -11.2 dBm (Note 8) | 100 kHz |
| All | 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 100 kHz |
| NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |  |

In certain regions, the following test requirements may apply to an E-UTRA TDD RIB operating in the same geographic area and in the same operating band as another E-UTRA TDD system without synchronisation. For this case the emissions shall not exceed -52 dBm/MHz in each supported downlink operating band, except in:

-	The frequency range from ΔfOBUE MHz below the lower channel edge to the frequency ΔfOBUE MHz above the upper channel edge of each supported band.

In certain regions the following test requirement may apply for protection of DTT. For E-UTRA a RIB operating in Band 20, the level of emissions in the band 470-790 MHz, measured in an 8MHz filter bandwidth on centre frequencies Ffilter according to Table 6.7.5.5.5.7-4, shall not exceed the maximum emission TRP level shown in the table.  This test requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.7.5.5.5.7-4: Declared emissions levels for protection of DTT

| Case | Measurement filter centre frequency | Condition on BS maximum aggregate EIRP / 10 MHz, PEIRP_10MHz (NOTE) | Maximum Level PEIRP,N,MAX | Measurement Bandwidth |
| --- | --- | --- | --- | --- |
| A: for DTT frequencies where broadcasting is protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz 59 dBm | 1.8dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PEIRP_10MHz < 59 dBm | PEIRP_10MHz – 57.2 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz < 36 dBm | -21.2dBm | 8 MHz |
| B: for DTT frequencies where broadcasting is subject to an intermediate level of protection | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz 59 dBm | 11.8dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PEIRP_10MHz < 59 dBm | PEIRP_10MHz – 47.2 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PEIRP_10MHz < 36 dBm | -11,2dBm | 8 MHz |
| C: for DTT frequencies where broadcasting is not protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | N.A. | 23.8dBm | 8 MHz |
| NOTE: PEIRP_10MHz (dBm) is defined by the expression PEIRP_10MHz = P10MHz + Gant + 6dB for UTRA and PEIRP_10MHz = P10MHz + Gant + 9dB for E-UTRA, where Gant is 17 dBi |  |  |  |  |

NOTE 1:	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions and the deployment (including antenna gain and feeder loss). The method outlined in annex B1, 3GPP TS 37.105 [6] indicates how the limit in Table 6.7.5.5.5.7-4 demonstrates compliance to the regional requirement.

### 6.7.6	OTA Spurious emission

#### 6.7.6.1	General

The OTA spurious emissions limits are specified as TRP per cell unless otherwise specified.

The OTA transmitter spurious emission limits apply from 30 MHz to 12.75 GHz, excluding the following RAT-specific frequency ranges:

-	UTRA FDD BS as specified in TS 25.104 [2]: from 12.5MHz below the lowest carrier frequency used up to 12.5MHz above the highest carrier frequency used.

-	E-UTRA BS as specified in TS 36.104 [4]: from ΔfOBUE below the lowest frequency of the downlink operating band up to ΔfOBUE above the highest frequency of the downlink operating band, where ΔfOBUE is defined in subclause 6.7.1.

-	MSR BS as specified in TS 37.104 [5]: from ΔfOBUE below the lowest frequency of the downlink operating band up to ΔfOBUE above the highest frequency of the downlink operating band, where ΔfOBUE is defined in subclause 6.7.1. For some operating bands the upper frequency limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [16]. In some exceptional cases, requirements apply also closer than 10 MHz from the downlink operating band; these cases are highlighted in the requirement tables in respective referenced UTRA, E-UTRA, NR or MSR specifications. For operating bands supported by multi-band RIB each supported band including the ΔfOBUE around the band are excluded from the spurious emissions requirements.

The requirements apply for both single band RIBs and multi-band RIBs (except for frequencies at which exclusion bands or other multi-band provisions apply) and for all transmission modes foreseen by the manufacturer's specification. Unless otherwise stated, all requirements are measured as mean power.

For operation in Region 2, where the FCC guidance for MIMO systems in [17] is applicable, the emissions limits are the same regardless of the number of transceiver units so the limits are equivalent to those for a single transceiver unit as specified in the as the corresponding applicable non-AAS BS per transmitter requirement specified in 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4], 3GPP TS 37.104 [5] or 3GPP TS 38.104 [33]. For E-UTRA and NR the limits will be 9dB lower and for UTRA FDD the limits will be 6dB lower.

The AAS BS requirements for spurious emissions limits which are specified for Band 46 in 3GPP TS 37.104 [5], are applicable for AAS BS.

#### 6.7.6.2	Mandatory Requirements

##### 6.7.6.2.1	Definition and applicability

The OTA spurious emissions mandatory requirements include the CAT A, CAT B and additional minimum requirements for BC2, limits are specified as TRP per cell unless otherwise specified.

##### 6.7.6.2.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.6.2.1.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.6.3.1.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.6.4.1.

##### 6.7.6.2.3	Test purpose

The test purpose is to verify the radiated spurious emissions from the AAS BS at the RIB are within the specified minimum requirements.

##### 6.7.6.2.4	Method of test

###### 6.7.6.2.4.1	Initial conditions

Test environment: normal, see annex G.2.

RF channels to be tested for single carrier:

B when testing from 30 MHz to FDL_low  - ΔfOBUE

T when testing from FDL_high  + ΔfOBUE to 12.75GHz (or to 5th harmonic)

RF bandwidth positions to be tested: [FFS] in single-band operation, see subclause 4.12.1.

BRFBW when testing from 30 MHz to FDL_low  - ΔfOBUE

TRFBW when testing from FDL_high  + ΔfOBUE to 12.75GHz (or 5th harmonic)

RF bandwidth positions to be tested in multi-band operation, see subclause 4.12.1.

BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low  - ΔfOBUE

B'RFBW_TRFBW when testing from FDL_Bhigh_high  + ΔfOBUE to 12.75GHz (or to 5th harmonic)

BRFBW_T'RFBW and  B'RFBW_TRFBW when testing from FDL_Blow_high  + ΔfOBUE to FDL_Bhigh_low  - ΔfOBUE

Directions to be tested

As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex F) as long as the required TRP output power level is achieved.

###### 6.7.6.2.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.7.6.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the AAS BS to transmit

a)	For MSR:

-	Set the RIB to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.11.

b)	For UTRA:

-	For a RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to TM1, subclause 4.12.2, at the manufacturer's declared rated output power, Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier operation, set the set the RIB to transmit according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

c)	For E-UTRA:

-	RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to E-TM1.1 in subclause 4.12.2, at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier and/or CA operation, set the set the RIB to transmit according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

6)	Align the BS and the test antenna such that measurements to determine TRP can be performed (see annex F)

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex F).

Note 1: the TRP measurement grid may not be the same for all measurement frequencies.

Note 2: the frequency sweep or the TRP measurement grid sweep may be done in any order

9)	Calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.6.2.5	Test Requirement

###### 6.7.6.2.5.1	MSR operation

Category A requirements

For an AAS BS meeting category A the TRP of any spurious emission shall not exceed the limits in table 6.7.6.2.5.1-1

Table 6.7.6.2.5.1-1: AAS BS OTA Spurious emission limits, Category A

| Frequency range | Maximum level | Measurement Bandwidth | NOTE |
| --- | --- | --- | --- |
| 30MHz - 1GHz | -13 + X dBm NOTE 4, | 100 kHz | NOTE 1 |
| 1GHz - 12.75 GHz |  | 1 MHz | NOTE 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | NOTE 2, NOTE 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [16] , s4.1 NOTE 2: Bandwidth as in ITU-R SM.329 [16] , s4.1. Upper frequency as in ITU-R SM.329 [16] , s2.5 table 1 NOTE 3: Applies only for Bands 22, 42 and 43. NOTE 4: X = 9 dB for E-UTRA, X = 6 dB for UTRA. With the exception of operation in Region 2 where the FCC guidance for MIMO systems in [17] is applicable and any other territories where regulation requires, X=0dB. |  |  |  |

Category B requirements

For UTRA, the minimum requirement is specified in subclause 6.7.6.2.5.2

For E-UTRA and NR, the minimum requirement is specified in subclause 6.7.6.2.5.3

Additional requirements for BC2 (category B)

For AAS BS operating in Band Category 2 when GSM/EDGE is configured, the power of any spurious emission shall not exceed the limits in table 6.7.6.2.5.1-2.

For multi-band RIBs the limits in table 6.7.6.2.5.1-2are only applicable when all supported operating bands belong to BC2 and GSM/EDGE is configured in all bands.

Table 6.7.6.2.5.1-2: Additional AAS BS OTA Spurious emissions limits for BC2, Category B

| Frequency range | Frequency offset from downlink operating band edge (NOTE) | Maximum Level | Measurement Bandwidth |
| --- | --- | --- | --- |
| 500 MHz 1 GHz | 10 – 20 MHz | -25 dBm | 300 kHz |
|  | 20 – 30 MHz | -25 dBm | 1 MHz |
|  | ≥ 30 MHz | -25 dBm | 3 MHz |
| 1 GHz 12.75 GHz | ≥ 30 MHz | -22 dBm | 3 MHz |
| NOTE: For multi-band RIBs, the frequency offset is relative to the closest operating band. |  |  |  |

###### 6.7.6.2.5.2	Single RAT UTRA operation

Category A requirements

For an AAS BS meeting category A the TRP of any spurious emission shall not exceed the limits in table 6.7.6.2.5.1-1

Category B requirements

The following limits shall be met in cases where Category B limits for spurious emissions, as defined in ITU-R Recommendation SM.329 [17], are applied.

The TRP of any spurious emission shall not exceed the limits in tables  6.7.6.2.5.2-1 ans 6.7.6.2.5.2-2

Table 6.7.6.2.5.2-1: OTA AAS BS Mandatory spurious emissions limits, operating band I, II, III, IV, VII, X, XXII, XXV, XXXII (Category B)

| Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -30 dBm | 100 kHz | NOTE 1 |
| 1 GHz Flow - 10 MHz | -24 dBm | 1 MHz | NOTE 1 |
| Flow - 10 MHz Fhigh + 10 MHz | -9 dBm | 1 MHz | NOTE 2 |
| Fhigh + 10 MHz 12.75 GHz | -24 dBm | 1 MHz | NOTE 3 |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the DL operating band in GHz | -24 dBm | 1 MHz | NOTE 3, NOTE 4 |
| NOTE 1: Bandwidth as in ITU-R Recommendation SM.329 [16], s4.1 NOTE 2: Limit based on ITU-R Recommendation SM.329 [16], s4.3 and Annex 7 NOTE 3: Bandwidth as in ITU-R Recommendation SM.329 [16], s4.1. Upper frequency as in ITU-R SM.329 [16], s2.5 table 1 NOTE 4: Applies only for Band XXII |  |  |  |
| Key: Flow: The lowest downlink frequency of the operating band as defined in subclause 4.7 Fhigh: The highest downlink frequency of the operating band as defined in subclause 4.7 |  |  |  |

Table 6.7.6.2.5.2-2: BS Mandatory spurious emissions limits, operating band V, VIII, XII, XIII, XIV, XX, XXVI (Category B)

| Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz Flow - 10 MHz | -30 dBm | 100 kHz | NOTE 1 |
| Flow - 10 MHz Fhigh + 10 MHz | -10 dBm | 100 kHz | NOTE 2 |
| Fhigh + 10 MHz 1 GHz | -30 dBm | 100 kHz | NOTE 1 |
| 1GHz 12.75GHz | -24 dBm | 1 MHz | NOTE 3 |
| NOTE 1: Bandwidth as in ITU-R Recommendation SM.329 [17], s4.1 NOTE 2: Limit based on ITU-R Recommendation SM.329 [17], s4.3 and Annex 7 NOTE 3: Bandwidth as in ITU-R Recommendation SM.329 [17], s4.1. Upper frequency as in ITU-R SM.329 [17], s2.5 table 1 |  |  |  |
| Key: Flow: The lowest downlink frequency of the operating band as defined in subclause 4.7 Fhigh: The highest downlink frequency of the operating band as defined in subclause 4.7 |  |  |  |

###### 6.7.6.2.5.3	Single RAT E-UTRA operation

Category A requirements

For an AAS BS meeting category A the TRP of any spurious emission shall not exceed the limits in table 6.7.6.2.5.1-1

Category B requirements

The TRP of any spurious emission shall not exceed the limits in table 6.7.6.2.5.3-1

Table 6.7.6.2.5.3-1: AAS BS OTA Spurious emissions limits, Category B

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -27 dBm | 100 kHz | NOTE 1 |
| 1 GHz 12.75 GHz | -21 dBm | 1 MHz | NOTE 2 |
| 12.75 GHz 5th harmonic of the upper frequency edge of the DL operating band in GHz | -21 dBm | 1 MHz | NOTE 2, NOTE 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [16] , s4.1 NOTE 2: Bandwidth as in ITU-R SM.329 [16] , s4.1. Upper frequency as in ITU-R SM.329 [16] , s2.5 table 1 NOTE 3: Applies only for Bands 22, 42 and 43. |  |  |  |

#### 6.7.6.3	Protection of the BS receiver of own or different BS

##### 6.7.6.3.1	Definition and applicability

This requirement shall be applied for FDD operation in order to prevent the receivers of own or a different BS of the same band being desensitised by emissions from a OTA AAS BS.

The requirement is a co-location requirement, the power levels specified at the CLTA output(s).

##### 6.7.6.3.2	Minimum Requirement

The minimum requirement for AAS BS in MSR operation is defined in TS 37.105 [6], subclause 9.7.6.2.

The minimum requirement for AAS BS in single RAT UTRA operation is defined in TS 37.105 [6], subclause 9.7.6.3.

The minimum requirement for AAS BS in single RAT E-UTRA operation is defined in TS 37.105 [6], subclause 9.7.6.4.

##### 6.7.6.3.3	Test purpose

The test purpose of OTA spurious emission is to verify the radiated spurious emissions from the AAS BS at the RIB are within specified requirements.

For OTA co-locate spurious emission, the test purpose is to verify that the emission is within the specified requirement limits at the CLTA conducted output(s).

##### 6.7.6.3.4	Method of test

###### 6.7.6.3.4.1	Initial conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier (SC):

-	M; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier (MC):

-	MRFBW for single-band RIB, see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band RIB, see subclause 4.12.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, co-location spurious emission testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, co-location spurious emission testing below the lowest operating band may be omitted.

Directions to be tested:

-	As the general spurious emission requirement is defined on TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex F) as long as the required TRP output power level is achieved.

###### 6.7.6.3.4.2	Procedure

1)	Select a CLTA according to parameters given in Table 4.15.2.2-1 and place the CLTA according to parameters given in Table 4.15.2.3-1.

2)	Several CLTAs are required to cover the whole co-location spurious emission frequency ranges.

3)	The test antenna shall be dual (or single) polarized with the same frequency range as the AAS BS for co-location spurious emission test case.

4)	Connect test antenna and CLTA to the measurement equipment as depicted in Annex D1.4.

5)	OTA co-location spurious emission is measured at the CLTA conducted output(s).

6)	The measurement device (signal analyzer) characteristics shall be:

-	Detection mode: True RMS.

7)	Set the AAS BS to transmit:

a)	For MSR:

-	Set the AAS BS to transmit maximum power, according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.11.

b)	For UTRA FDD:

-	For a AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit full maximum power according to TM1, subclause 4.12.2, at the manufacturer's declared rated output power, PRated,c,TRP.

-	For a AAS BS declared to be capable of multi-carrier operation, set the AAS BS to transmit maximum power according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

c)	For E-UTRA:

-	For AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit maximu power according to E-TM1.1 in subclause 4.12.2, at manufacturer's declared rated output power, PRated,c,TRP.

-	For a AAS BS declared to be capable of multi-carrier and/or CA operation, set the AAS BS to transmit maximum power according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

8)	Measure the emission at the specified frequencies with specified measurement bandwidth and note that the measured value does not exceed the test requirement in subclause 6.7.6.5.

NOTE:	An alternative measurement method to be used for measureing the OTA emission is described in Annex H.

In addition, for multi-band RIB, the following steps shall apply:

9)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.6.3.5	Test Requirement

###### 6.7.6.3.5.1	MSR operation

The total power from both polarizations of the CLTA connector output(s) of any spurious emission shall not exceed the limits in table 6.7.6.3.5.1-1 depending on the declared Base Station class and Band Category.

Table 6.7.6.3.5.1-1: BS Spurious emissions limits for protection of the BS receiver

| BS-class | Band category | Frequency range | Maximum Level for bands below 3GHz | Maximum Level for bands between 3 and 4.2GHz | Measurement Bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Wide Area BS | BC1 | FUL_low – FUL_high | -113.9 dBm | -113.7 dBm | 100 kHz |  |
| Wide Area BS | BC2 | FUL_low – FUL_high | -115.9 dBm | -115.7 dBm | 100 kHz |  |
| Medium Range BS | BC1,BC2 | FUL_low – FUL_high | -108.9 dBm | -108.7 dBm | 100 kHz |  |
| Local Area BS | BC1,BC2 | FUL_low – FUL_high | -105.9 dBm | -105.7 dBm | 100 kHz |  |

###### 6.7.6.3.5.2	Single RAT UTRA operation

The total power of any spurious emission from both polarizations of the CLTA connector output(s) shall not exceed the limits in table 6.7.6.3.5.2-1.

Table 6.7.6.3.5.2-1: BS Spurious emissions limits for protection of the BS receiver

|  | Frequency range | Maximum Level for bands below 3GHz | Maximum Level for bands between 3 and 4.2GHz | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- | --- |
| Wide Area BS | FUL_low – FUL_high | - 116.9 dBm | - 116.7 dBm | 100 kHz |  |
| Medium Range BS | FUL_low – FUL_high | - 106.9 dBm | - 106.7 dBm | 100 kHz |  |
| Local Area BS | FUL_low – FUL_high | - 102.9 dBm | - 102.7 dBm | 100 kHz |  |

###### 6.7.6.3.5.3	Single RAT E-UTRA operation

The total power of any spurious emission from both polarizations of the CLTA connector output(s) shall not exceed the limits in table 6.7.6.3.5.3-1.

Table 6.7.6.3.5.3-1: BS Spurious emissions limits for protection of the BS receiver

|  | Frequency range | Maximum Level for bands below 3GHz | Maximum Level for bands between 3 and 4.2GHz | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- | --- |
| Wide Area BS | FUL_low – FUL_high | -113.9 dBm | -113.7 dBm | 100 kHz |  |
| Medium Range BS | FUL_low – FUL_high | -108.9 dBm | -108.7 dBm | 100 kHz |  |
| Local Area BS | FUL_low – FUL_high | -105.9 dBm | -105.7 dBm | 100 kHz |  |

#### 6.7.6.4	Additional spurious emissions requirements

##### 6.7.6.4.1	Definition and applicability

These requirements may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in subclause 4.5.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM/EDGE, CDMA, UTRA, E-UTRA, NR etc.) as listed below.

All additional spurious requirements are TRP unless otherwise stated.

##### 6.7.6.4.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.6.2.3.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.6.3.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 9.7.6.4.3.

##### 6.7.6.4.3	Test purpose

The test purpose is to verify the radiated spurious emissions from the AAS BS at the RIB are within the specified additional spurious emissions requirements.

##### 6.7.6.4.4	Method of test

###### 6.7.6.4.4.1	Initial conditions

Test environment: normal, see annex G.2.

RF channels to be tested for single carrier:

B when testing from 30 MHz to FDL_low  - ΔfOBUE

T when testing from FDL_high  + ΔfOBUE to 12.75GHz (or to 5th harmonic)

RF bandwidth positions to be tested: in single-band operation, see subclause 4.12.1.

BRFBW when testing from 30 MHz to FDL_low  - ΔfOBUE

TRFBW when testing from FDL_high  + ΔfOBUE to 12.75GHz (or to 5th harmonic)

RF bandwidth positions to be tested in multi-band operation, see subclause 4.12.1.

BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low  - ΔfOBUE

B'RFBW_TRFBW when testing from FDL_Bhigh_high  + ΔfOBUE to 12.75GHz (or to 5th harmonic)

BRFBW_T'RFBW and  B'RFBW_TRFBW when testing from FDL_Blow_high  + ΔfOBUE to FDL_Bhigh_low  - ΔfOBUE

Directions to be tested:

As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex F) as long as the required TRP output power level is achieved.

###### 6.7.6.4.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in 3GPP TS 37.104 [5] subclause 6.6.1.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the AAS BS to transmit

a)	For MSR:

-	Set the RIB to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.11.

b)	For UTRA:

-	For a RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to TM1, subclause 4.12.2, at the manufacturer's declared rated output power, Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier operation, set the set the RIB to transmit according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

c)	For E-UTRA:

-	RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to E-TM1.1 in subclause 4.12.2, at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier and/or CA operation, set the set the RIB to transmit according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

6)	Align the BS and the test antenna such that measurement in the direction of the appropriate TRP measurement grid point can be performed. (see annex F)

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex F).

Note 1: the TRP measurement grid may not be the same for all measurement frequencies.

Note 2: the frequency sweep or the TRP measurement grid sweep may be done in any order

9)	Calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 6.7.6.4.5	Test Requirement

###### 6.7.6.4.5.1	MSR operation

For UTRA, the minimum requirement is specified in subclause 6.7.6.4.5.2

For E-UTRA, the minimum requirement is specified in subclause 6.7.6.4.5.3.

###### 6.7.6.4.5.1.1	E-UTRA and NR MSR operation

The TRP of any spurious emission shall not exceed the limits of table 6.7.6.4.5.1.1-1 for an AAS BS where requirements for co-existence with the system listed in the first column apply. For a multi-band RIB, the exclusions and conditions in the notes column of table 6.7.6.4.5.1.1-1 apply for each supported operating band.

Table 6.7.6.4.5.1.1-1: AAS BS OTA Spurious emissions limits for co-existence with systems operating in other frequency bands

| System type to co-exist with | Frequency range for co-existence requirement | Maximum Level |  | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz | -45.4dBm |  | 100 kHz | This requirement does not apply to BS operating in band 8/n8 |
|  | 876 - 915 MHz | -49.4dBm |  | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band 8/n8, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| DCS1800 (Note 3) | 1805 - 1880 MHz | -35.4dBm |  | 100 kHz | This requirement does not apply to BS operating in band 3/n3. |
|  | 1710 - 1785 MHz | -49.4dBm |  | 100 kHz | This requirement does not apply to BS operating in band 3/n3, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| PCS1900 | 1930 - 1990 MHz | -35.4dBm |  | 100 kHz | This requirement does not apply to BS operating in band 2/n2, 25/n25, band 36 or band 70/n70. |
|  | 1850 - 1910 MHz | -49.4dBm |  | 100 kHz | This requirement does not apply to BS operating in band 2/n2 or 25/n25, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in band 35. |
| GSM850 or CDMA850 | 869 - 894 MHz | -45.4dBm |  | 100 kHz | This requirement does not apply to BS operating in band 5/n5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -49.4dBm |  | 100 kHz | This requirement does not apply to BS operating in band 5/n5 or 26, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 2110 - 2170 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 1/n1 or 65 . |
|  | 1920 - 1980 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 1/n1 or 65, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1930 - 1990 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 2/n2, 25/n25 or 70/n70. |
|  | 1850 - 1910 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 2/n2 or 25/n25, since it is already covered by the requirement in sub-clause 6.6.6.5.2.4 |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 (Note 3) | 1805 - 1880 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 3/n3 or 9. |
|  | 1710 - 1785 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 3/n3, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 4, 10 or 66. |
|  | 1710 - 1755 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 4, 10 or 66, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 869 - 894 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 5/n5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 5/n5 or 26, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 | 860 - 890 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 6, 18, 19 |
|  | 815 - 830 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 18 since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
|  | 830 - 845 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 6, 19, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2620 - 2690 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 7/n7. |
|  | 2500 - 2570 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 7/n7, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 925 - 960 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 8/n8. |
|  | 880 - 915 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 8/n8, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 3/n3 or 9. |
|  | 1749.9 - 1784.9 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 3/n3 or 9, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 4, 10 or 66/n66. |
|  | 1710 - 1770 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 10 or 66/n66, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 - 1510.9 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 11, 21, 32, 50/n50, 74 or 75/n75. |
|  | 1427.9 - 1447.9 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 11 or 74, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in band 32, 50/n50, 51/n51, 75/n75 or 76/n76. |
|  | 1447.9 – 1462.9 MHz | -37.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 21 or 74, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in band 32, 50/n50 or 75/n75. |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 729 - 746 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 12/n12 or 85. |
|  | 699 - 716 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 12/n12 or 85, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 13. |
|  | 777 - 787 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 13, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 14. |
|  | 788 - 798 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 14, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| E-UTRA Band 17 | 734 - 746 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 17. |
|  | 704 - 716 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 17, since it is already covered by the requirement in subclause 6.7.6.3.5.1 For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 791 - 821 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 20/n20 or 28/n28. |
|  | 832 - 862 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 20/n20, since it is already covered by the requirement in subclause 6.7.6.3.5.1 |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 22, 42, 48, 49, n77 or n78. |
|  | 3410 – 3490 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 22, since it is already covered by the requirement in subclause 6.7.6.3.5.1 This requirement does not apply to Band 42 |
| E-UTRA Band 24 | 1525 – 1559 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 24. |
|  | 1626.5 – 1660.5 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 24, since it is already covered by the requirement in subclause 6.7.6.3.5.1 |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1930 - 1995 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 2/n2, 25/n25 or 70/n70. |
|  | 1850 - 1915 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 25/n25, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in band 2/n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 - 894 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 5/n5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 814 - 849 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 26, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in band 5/n5, it applies for 814 MHz to 824 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| E-UTRA Band 27 | 852 – 869 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 5/n5, 26 or 27. |
|  | 807 – 824 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 27, since it is already covered by the requirement in subclause 6.7.6.3.5.1 For BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 This requirement also applies to BS operating in Band 28/n28, starting 4 MHz above the Band 28/n28 downlink operating band (Note 6). |
| E-UTRA Band 28 or NR Band n28 | 758 - 803 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 20/n20, 28/n28, 44 or 67. |
|  | 703 - 748 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 28/n28, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733MHz. |
| E-UTRA Band 29 | 717 – 728 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in Band 29 or 85. |
| E-UTRA Band 30 | 2350 - 2360 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 30 or 40/n40. |
|  | 2305 - 2315 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 30, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in Band 40. |
| E-UTRA Band 31 | 462.5 – 467.5 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 31, 72 or 73. |
|  | 452.5 – 457.5 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 31, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in band 72 or 73. |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 11, 21, 32, 50/n50, 74 or 75/n75. |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 or NR Band n34 | 2010 - 2025 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in Band 34/n34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in Band 2/n2, 25/n25 or 36 |
| UTRA TDD in Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in Band 38/n38 or 69. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR Band n39 | 1880 – 1920MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 39/n39 |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 30 or 40/n40 |
| E-UTRA Band 41 or NR Band n41 | 2496 – 2690MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 41/n41 |
| E-UTRA Band 42 | 3400 – 3600 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 22, 42, 43, 48, 49, 52, n77 or n78. |
| E-UTRA Band 43 | 3600 – 3800 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 42, 43, 48, 49, n77 or n78. |
| E-UTRA Band 44 | 703 - 803 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 28/n28 or 44 |
| E-UTRA Band 45 | 1447 - 1467 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 45 |
| E-UTRA Band 46 | 5150 - 5925 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 46 |
| E-UTRA Band 47 | 5855 - 5925 MHz | -40.4 dBm |  | 1 MHz |  |
| E-UTRA Band 48 | 3550 – 3700 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 22, 42, 43, 48, 49, n77 or n78 |
| E-UTRA Band 49 | 3550 – 3700 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 22, 42, 43, 48, 49, n77 or n78 |
| E-UTRA Band 50 or NR Band n50 | 1432 - 1517 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in Band 11, 21, 32, 45, 50/n50, 51/n51, 74, 75/n75 or 76/n76. |
| E-UTRA Band 51 or NR Band n51 | 1427 - 1432 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in Band 50/n50, 51/n51, 75/n75 or 76/n76. |
| E-UTRA Band 52 | 3300 – 3400 MHz | -40.4 dBm |  | 1 MHz | This is not applicable to BS operating in Band 42 or 52. |
| E-UTRA Band 65 | 2110 - 2200 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 1/n1 or 65, |
|  | 1920 - 2010 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 65, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 |
| E-UTRA Band 66 or NR Band n66 | 2110 - 2200 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 4, 10, 23 or 66/n66. |
|  | 1710 - 1780 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 66/n66, since it is already covered by the requirement in clause 6.7.6.3.5.1 For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in clause 6.7.6.3.5.1 For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in clause 6.7.6.3.5.1 |
| E-UTRA Band 67 | 738 – 758 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 28/n28 or 67. |
| E-UTRA Band 68 | 753 -783 MHz | -37.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 28/n28 or 68. |
|  | 698-728 MHz | -40.4dBm |  | 1 MHz | This requirement does not apply to BS operating in band 68, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in Band 28/n28, it applies between 698 MHz and 703 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 |
| E-UTRA Band 69 |  | 2570 - 2620 MHz |  | -40.4 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 38 or 69. |  |
| E-UTRA Band 70 or NR Band n70 | 1995 - 2020 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 2/n2, 25/n25 or 70/n70 |
|  | 1695 – 1710 MHz | -37.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 70/n70, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| E-UTRA Band 71 or NR Band n71 | 617 - 652 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 71/n71. |
|  | 663 – 698 MHz | -37.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 71/n71, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| E-UTRA Band 72 | 461 - 466 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 31, 72 or 73. |
|  | 451 - 456 MHz | -37.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 72, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in band 73. |
| E-UTRA Band 73 | 460 - 465 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 31, 72 or 73. |
|  | 450 - 455 MHz | -37.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 73, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |
| E-UTRA Band 74 or NR band n74 | 1475 – 1518 MHz | -40.4 dBm |  | 1 MHz | This requirement does not apply to BS operating in band 11, 21, 32, 50/n50, 74 or 75/n75. |
|  | 1427 – 1470 MHz | -37.4 dBm |  | 1MHz | This requirement does not apply to BS operating in Band 74, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in band 32, 45, 50/n50, 51/n51, 75/n75 or 76/n76. |
| E-UTRA Band 75 or NR Band n75 |  | 1432 - 1517 MHz |  | -40.4 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 11, 21, 32, 45, 50/n50, 51/n51, 74, 75/n75 or 76/n76. |  |
| E-UTRA Band 76 or NR Band n76 |  | 1427 - 1432 MHz |  | -40.4 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in Band 50/n50, 51/n51, 75/n75 or 76/n76. |  |
| NR Band n77 |  | 3300 MHz – 4200 MHz |  | -40.4 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48, 49, n77 or n78 |  |
| NR Band n78 |  | 3300 MHz – 3800 MHz |  | -40.4 dBm |  |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48, 49, n77 or n78 |  |
| NR Band n80 |  | 1710 - 1785 MHz |  | -37.4 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 3/n3, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 |  |
| NR Band n81 |  | 880 - 915 MHz |  | -37.4 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 8/n8, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |  |
| NR Band n82 |  | 832 - 862 MHz |  | -37.4 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 20/n20, since it is already covered by the requirement in subclause 6.7.6.3.5.1 |  |
| NR Band n83 |  | 703 - 748 MHz |  | -37.4 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 28/n28, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736MHz. For BS operating in Band 68, it applies for 728MHz to 733MHz. |  |
| NR Band n84 |  | 1920 - 1980 MHz |  | -37.4 dBm |  |  | 1 MHz |  | This requirement does not apply to BS operating in band 1/n1 or 65, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 |  |
| E-UTRA Band 85 | 728 - 746 MHz | -40.4 dBm | 1 MHz |  | This requirement does not apply to BS operating in band 12/n12, 29 or 85. |
|  | 698 - 716 MHz | -37.4 dBm | 1 MHz |  | This requirement does not apply to BS operating in band 85, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |
| NR Band n86 | 1710 - 1780 MHz | -37.4 dBm | 1 MHz |  | This requirement does not apply to BS operating in band 66/n66, since it is already covered by the requirement in sub-clause 6.7.6.3.5.1 For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in sub-clause 6.7.6.3.5.1 |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, except for the cases where the noted requirements apply to a BS operating in Band 25/n25, Band 27, Band 28/n28 or Band 29, the co-existence requirements in table 6.7.6.4.5.1.1-1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see subclause 6.7.1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.7.6.4.5.1.1-1 assumes that two operating bands, where the frequency ranges in subclause 4.7 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III or E-UTRA Band 3 or NR band n3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 – 1850 MHz and 1710 – 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations (except in Band 46), special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 6:	For Band 28/n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band.

NOTE 7:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band or E-UTRA Band 17 UL operating band.

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between ΔfOBUE below the lowest BS transmitter frequency of the downlink operating band and ΔfOBUE above the highest BS transmitter frequency of the downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.1.1-2: AAS BS OTA Spurious emissions limits for BS for co-existence with PHS

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -32 dBm | 300 kHz | Applicable for co-existence with PHS system operating in 1884.5-1915.7MHz |
| NOTE: The requirement is not applicable in China. |  |  |  |

The following requirement shall be applied to AAS BS operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.1.1-3: AAS BS OTA Spurious emissions limits for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 13 | 793 - 805 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 769 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 799 - 805 MHz | -37 dBm | 6.25 kHz |  |

The following requirement shall be applied to AAS BS operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.1.1-4: AAS BS OTA Spurious emissions limits for protection of 800 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -4 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

The following requirement may apply to E-UTRA AAS BS operating in Band 41 in certain regions. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.1.1-5: Additional AAS BS OTA Spurious emissions limits for Band 41

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 2505MHz – 2535MHz | -30.4 dBm | 1 MHz |  |
| 2535MHz – 2655MHz | -10.4 dBm | 1 MHz | Applicable at offsets ≥ 250% of channel bandwidth from carrier frequency |
| NOTE: This requirement applies for 10 or 20 MHz E-UTRA carriers allocated within 2545-2575MHz or 2595-2645MHz. |  |  |  |

The following requirement may apply to AAS BS operating in Band 30 in certain regions. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.1.1-6: Additional AAS BS OTA Spurious emissions limits for Band 30

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 2200MHz – 2345MHz | -33.4 dBm | 1 MHz |  |
| 2362.5MHz – 2365MHz | -13.4 dBm | 1 MHz |  |
| 2365MHz – 2367.5MHz | -28.4 dBm | 1 MHz |  |
| 2367.5MHz – 2370MHz | -30.4 dBm | 1 MHz |  |
| 2370MHz – 2395MHz | -33.4 dBm | 1 MHz |  |

The following requirement may apply to AAS BS operating in Band 48 in certain regions. The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.1.1-7: Additional AAS BS OTA Spurious emissions limits for Band 48

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 3530MHz – 3720MHz | -13.4 dBm | 1 MHz | Applicable 10MHz from the assigned channel edge |
| 3100MHz – 3530MHz 3720MHz – 4200MHz | -28.0 dBm | 1 MHz |  |

In addition to the requirements in subclauses in the present subclause, the AAS BS may have to comply with the applicable emission limits established by FCC Title 47 [18], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

The following requirement shall be applied to AAS BS operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

Table 6.7.6.4.5.1.1-8: AAS BS OTA Spurious emissions limits for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 13 | 793 - 805 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 769 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 799 - 805 MHz | -37 dBm | 6.25 kHz |  |

The following requirement shall be applied to AAS BS operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.1.1-9: AAS BS OTA Spurious emissions limits for protection of 800 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -13 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

###### 6.7.6.4.5.2	Single RAT UTRA operation

The TRP of any spurious emission shall not exceed the limits of table 6.7.6.4.5.2-1 for a AAS BS where requirements for co-existence with the system listed in the first column apply. For a multi-band RIB, the exclusions and conditions in the notes column of table 6.7.6.4.5.2-1 apply for each supported operating band.

Table 6.7.6.4.5.2-1 OTA AAS BS Spurious emissions limits for UTRA FDD BS in geographic coverage area of systems operating in other frequency bands

| System type operating in the same geographical area | Band for co-existence requirement | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz | -48.4dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in band VIII |
|  | 876 - 915 MHz | -52.4dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to UTRA FDD operating in band VIII, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| DCS1800 | 1805 - 1880 MHz | -38.4dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in band III |
|  | 1710 - 1785 MHz | -52.4dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in band III, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| PCS1900 | 1930 - 1990 MHz | -38.4dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band II or band XXV |
|  | 1850 - 1910 MHz | -52.4dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band II or band XXV, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| GSM850 or CDMA850 | 869 - 894 MHz | -48.4dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band V or XXVI |
|  | 824 - 849 MHz | -52.4dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band V or XXVI, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 - 2170 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band I, |
|  | 1920 - 1980 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band I, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 - 1990 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band II or band XXV |
|  | 1850 - 1910 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band II or band XXV, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 - 1880 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band III or band IX |
|  | 1710 - 1785 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band III, since it is already covered by the requirement in subclause 6.7.6.5.1.4. For UTRA BS operating in band IX, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in subclause 6.7.6.5.1.4. |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band IV or band X |
|  | 1710 - 1755 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band IV or band X, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 - 894 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band V or XXVI |
|  | 824 - 849 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band V or XXVI, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band VI or XIX, E-UTRA Band 6, 18 or 19 | 860 - 890 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VI or XIX |
|  | 815 - 845 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VI or XIX, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 - 2690 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VII, |
|  | 2500 - 2570 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VII, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 - 960 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VIII. |
|  | 880 - 915 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VIII, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band III or band IX |
|  | 1749.9 - 1784.9 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band III or band IX, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band IV or band X. |
|  | 1710 - 1770 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band X, since it is already covered by the requirement in subclause 6.7.6.5.1.4. For UTRA FDD BS operating in Band IV, it applies for 1755 MHz to 1770 MHz, while the rest is covered in subclause 6.7.6.5.1.4. |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 - 1510.9 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XI , XXI or XXXII. |
|  | 1427.9 - 1447.9 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XI, since it is already covered by the requirement in subclause 6.7.6.5.1.4. For UTRA BS operating in band XXXII, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. |
|  | 1447.9 - 1462.9 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXI, since it is already covered by the requirement in subclause 6.7.6.5.1.4. For UTRA BS operating in band XXXII, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. |
| UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 - 746 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII |
|  | 699 - 716 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIII |
|  | 777 - 787 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIII, since it is already covered by the requirement in sub-clause 6.7.6.5.1.4. |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIV |
|  | 788 - 798 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIV, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| E-UTRA Band 17 | 734 - 746 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII |
|  | 704 - 716 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 - 821 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XX |
|  | 832 - 862 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XX, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 -3590 MHz | -43.0dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXII. |
|  | 3410 -3490 MHz | -40.0dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXII, since it is already covered by the requirement in subclause 6.7.6.5.1.4. |
|  | 2010 – 2020 MHZ | -40.4dBm | 1 MHz |  |
| E-UTRA Band 24 | 1525 – 1559 MHz | -43.4dBm | 1 MHz |  |
|  | 1626.5 – 1660.5 MHz | -40.4dBm | 1 MHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 - 1995 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band II or band XXV |
|  | 1850 - 1915 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXV, since it is already covered by the requirement in subclause 6.7.6.5.1.4. For UTRA FDD BS operating in Band II, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 6.7.6.5.1.4. |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859-894 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band V or band XXVI |
|  | 814-849 MHz | -40.4dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXVI, since it is already covered by the requirements in subclause 6.7.6.5.1.4 For UTRA FDD BS operating in band V, it applies for 814MHz to 824MHz, while the rest is covered in subclause 6.7.6.5.1.4 |
| E-UTRA Band 27 | 852 – 869 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA BS operating in Band V or XXVI. |
|  | 807 – 824 MHz | -40.4dBm | 1 MHz | For UTRA BS operating in Band XXVI, it applies for 807 MHz to 814 MHz, while the rest is covered in subclause 6.7.6.5.1.4. |
| E-UTRA Band 28 or NR band n28 | 758 – 803 MHz | -43.4dBm | 1 MHz |  |
|  | 703 – 748 MHz | -40.4dBm | 1 MHz |  |
| E-UTRA Band 29 | 717 – 728 MHz | -43.4dBm | 1 MHz |  |
| E-UTRA Band 30 | 2350 - 2360 MHz | -43.4dBm | 1 MHz |  |
|  | 2305 - 2315 MHz | -40.4dBm | 1 MHz |  |
| E-UTRA Band 31 | 462.5 -467.5 MHz | -43.4dBm | 1 MHz |  |
|  | 452.5 -457.5 MHz | -40.4dBm | 1 MHz |  |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 – 1496 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA BS operating in Band XI, XXI, or XXXII |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 – 1920 MHz | -43.4dBm | 1 MHz |  |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 – 2025 MHz | -43.4dBm | 1 MHz |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 – 2620 MHz | -43.4dBm | 1 MHz |  |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920 MHz | -43.4dBm | 1 MHz | Applicable in China |
| UTRA TDD in Band e) or E-UTRA Band 40 or NR band n40 | 2300 – 2400 MHz | -43.4dBm | 1 MHz |  |
| E-UTRA Band 41 or NR band n41 | 2496 - 2690 MHz | -43.4dBm | 1 MHz |  |
| E-UTRA Band 42 | 3400 – 3600 MHz | -43.0dBm | 1 MHz |  |
| E-UTRA Band 43 | 3600 – 3800 MHz | -43.0dBm | 1 MHz |  |
| E-UTRA Band 44 | 703 - 803 MHz | -43.4dBm | 1 MHz |  |
| E-UTRA Band 45 | 1447 - 1467 MHz | -43.4dBm | 1 MHz |  |
| E-UTRA Band 46 | 5150 - 5925 MHz | -42.5 dBm | 1 MHz |  |
| E-UTRA Band 47 | 5855 - 5925 MHz | -42.5 dBm | 1 MHz |  |
| E-UTRA Band 48 | 3550 – 3700 MHz | -43.0dBm | 1 MHz |  |
| E-UTRA Band 50 or NR Band n50 | 1432 - 1517 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to UTRA BS operating in Band XI |
| E-UTRA Band 51 or NR Band n51 | 1427 - 1432 MHz | -40.4 dBm | 1 MHz |  |
| E-UTRA Band 65 | 2110 - 2200 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA BS operating in band I, |
|  | 1920 - 2010 MHz | -40.4dBm | 1 MHz | For UTRA BS operating in Band I, it applies for 1980 MHz to 2010 MHz, while the rest is covered in subclause 6.7.6.5.1.4 |
| E-UTRA Band 66 or NR band n66 | 2110 - 2200 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA BS operating in band IV or X . |
|  | 1710 - 1780 MHz | -40.4dBm | 1 MHz | For UTRA BS operating in Band IV, this requirement applies for 1755 MHz to 1780 MHz, while the rest is covered in subclause 6.7.6.5.1.4. For UTRA BS operating in Band X, this requirement applies for 1770 MHz to 1780 MHz, while the rest is covered in subclause 6.7.6.5.1.4. |
| E-UTRA Band 67 | 738 - 758 MHz | -43.4dBm | 1 MHz |  |
| E-UTRA Band 68 | 753 -783 MHz | -43.4dBm | 1 MHz |  |
|  | 698-728 MHz | -40.4dBm | 1 MHz |  |
| E-UTRA Band 69 | 2570 - 2620 MHz | -43.4dBm | 1 MHz |  |
| E-UTRA Band 70 or NR band n70 | 1995 – 2020 MHz | -43.4dBm | 1 MHz | This requirement does not apply to UTRA BS operating in band II or XXV. |
|  | 1695 – 1710 MHz | -40.4dBm | 1 MHz |  |
| E-UTRA Band 71 or NR Band n71 | 617 - 652 MHz | -40.4 dBm | 1 MHz |  |
|  | 663 – 698 MHz | -37.4 dBm | 1 MHz |  |
| E-UTRA Band 72 | 461 - 466 MHz | -40.4 dBm | 1 MHz |  |
|  | 451 - 456 MHz | -37.4 dBm | 1 MHz |  |
| E-UTRA Band 73 | 460 - 465 MHz | -40.4 dBm | 1 MHz |  |
|  | 450 - 455 MHz | -37.4 dBm | 1 MHz |  |
| E-UTRA Band 74 or NR band n74 | 1475 – 1518 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XI. |
|  | 1427 – 1470 MHz | -37.4 dBm | 1MHz |  |
| E-UTRA Band 75 or NR Band n75 | 1432 - 1517 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XI. |
| E-UTRA Band 76 or NR Band n76 | 1427 - 1432 MHz | -40.4 dBm | 1 MHz |  |
| NR Band n77 | 3300 MHz – 4200 MHz | -40.4 dBm | 1 MHz |  |
| NR Band n78 | 3300 MHz – 3800 MHz | -40.4 dBm | 1 MHz |  |
| NR Band n80 | 1710 - 1785 MHz | -37.4 dBm | 1 MHz | For BS operating in band IX. |
| NR Band n81 | 880 - 915 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band IIX |
| NR Band n82 | 832 - 862 MHz | -37.4 dBm | 1 MHz |  |
| NR Band n83 | 703 - 748 MHz | -37.4 dBm | 1 MHz |  |
| NR Band n84 | 1920 - 1980 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band I |
| E-UTRA Band 85 | 728 - 746 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band XII |
|  | 698 - 716 MHz | -37.4 dBm | 1 MHz |  |
| NR Band n86 | 1710 - 1780 MHz | -37.4 dBm | 1 MHz | For BS operating in Band IV, it applies for 1755 MHz to 1780 MHz, while the rest is covered in sub-clause 6.7.6.5.1.4 For BS operating in Band X, it applies for 1770 MHz to 1780 MHz, while the rest is covered in sub-clause 6.7.6.5.1.4 |
| NOTE 1: The co-existence requirements do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see subclause 6.7.1). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 2: The table above assumes that two operating bands, where the frequency ranges would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

The following requirement may be applied for the protection of PHS in geographic areas in which both PHS and UTRA FDD are deployed. This requirement is also applicable at specified frequencies falling between 12.5MHz below the first carrier frequency used and 12.5MHz above the last carrier frequency used.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.2-2: AAS BS OTA Spurious emissions limits for BS in geographic coverage area of PHS

| Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -35 dBm | 300 kHz |  |

The following requirement may be applied for the protection in bands adjacent to bands I or VII as defined in subclause 4.4, in geographic areas in which both an adjacent band service and UTRA FDD are deployed.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.2-3: AAS BS OTA Spurious emissions limits for protection of adjacent band services

| Operating Band | Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| I | 2100-2105 MHz | -24 + 6.0 (f - 2100 MHz) dBm | 1 MHz |  |
|  | 2175-2180 MHz | -24 + 6.0 (2180 MHz - f) dBm | 1 MHz |  |
| VII | 2610-2615 MHz | -24 + 6.0 (f - 2610 MHz) dBm | 1 MHz |  |
|  | 2695-2700 MHz | -24 + 6.0 (2700 MHz - f) dBm | 1 MHz |  |

NOTE:	This requirement for the frequency range 2610-2615 MHz may be applied to geographic areas in which both UTRA-TDD and UTRA-FDD are deployed.

The following requirement shall be applied to AAS BS operating in Bands XIII and XIV to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at specified frequencies falling between 12.5 MHz below the first carrier frequency used and 12.5 MHz above the last carrier frequency used.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.2-4: AAS BS OTA Spurious emissions limits

| Operating Band | Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| XIII | 763 - 775 MHz | -37.4 dBm | 6.25 kHz |  |
| XIII | 793 - 805 MHz | -37.4 dBm | 6.25 kHz |  |
| XIV | 769 - 775 MHz | -37.4 dBm | 6.25 kHz |  |
| XIV | 799 - 805 MHz | -37.4 dBm | 6.25 kHz |  |

The following requirement shall be applied to AAS BS operating in Bands XXVI to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at specified frequencies falling between 12.5 MHz below the first carrier frequency used and 12.5 MHz above the last carrier frequency used.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.2-5: AAS BS OTA Spurious emissions limits

| Operating Band | Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| XXVI | 851 - 859 MHz | -4.4 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

###### 6.7.6.4.5.3	Single RAT E-UTRA operation

The TRP of any spurious emission shall not exceed the limits of table 6.7.6.4.5.3-1 for an AAS BS where requirements for co-existence with the system listed in the first column apply. For a multi-band RIB, the exclusions and conditions in the notes column of table 6.7.6.4.5.3-1 apply for each supported operating band.

Table 6.7.6.4.5.3-1: AAS BS OTA Spurious emissions limits for co-existence with systems operating in other frequency bands

| System type to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz | -45.4dBm | 100 kHz | This requirement does not apply to BS operating in band 8 |
|  | 876 - 915 MHz | -49.4dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band 8, since it is already covered by the requirement in subclause 6.7.6.5.3.3 |
| DCS1800 (NOTE 3) | 1805 - 1880 MHz | -35.4dBm | 100 kHz | This requirement does not apply to BS operating in band 3. |
|  | 1710 - 1785 MHz | -49.4dBm | 100 kHz | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| PCS1900 | 1930 - 1990 MHz | -35.4dBm | 100 kHz | This requirement does not apply to BS operating in band 2, 25, band 36 or band 70. |
|  | 1850 - 1910 MHz | -49.4dBm | 100 kHz | This requirement does not apply to BS operating in band 2 or 25, since it is already covered by the requirement in subclause 6.7.6.5.3.3. This requirement does not apply to BS operating in band 35. |
| GSM850 or CDMA850 | 869 - 894 MHz | -45.4dBm | 100 kHz | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -49.4dBm | 100 kHz | This requirement does not apply to BS operating in band 5 or 26, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 - 2170 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 1 or 65, |
|  | 1920 - 1980 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 1 or 65, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 - 1990 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 2, 25 or 70. |
|  | 1850 - 1910 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 2 or 25, since it is already covered by the requirement in subclause 6.7.6.5.3.3 |
| UTRA FDD Band III or E-UTRA Band 3 or NR band n3 (NOTE 3) | 1805 - 1880 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 3 or 9. |
|  | 1710 - 1785 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in subclause 6.7.6.5.3.3. |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 4, 10 or 66 |
|  | 1710 - 1755 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 4, 10 or 66, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 - 894 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 5 or 26, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 | 860 - 890 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 6, 18, 19 |
|  | 815 - 830 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 18 since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
|  | 830 - 845 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 6, 19, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 - 2690 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 7. |
|  | 2500 - 2570 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 7, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 - 960 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 8. |
|  | 880 - 915 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 3 or 9. |
|  | 1749.9 - 1784.9 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 3 or 9, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 4, 10 or 66 |
|  | 1710 - 1770 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 10 or 66, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in subclause 6.7.6.5.3.3. |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 - 1510.9 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 11, 21 or 32 |
|  | 1427.9 - 1447.9 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 11, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 32, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. |
|  | 1447.9 – 1462.9 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 21, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 32, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. |
| UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 - 746 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 12. |
|  | 699 - 716 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 12, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (NOTE 7) |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 13. |
|  | 777 - 787 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 13, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 14. |
|  | 788 - 798 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 14, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| E-UTRA Band 17 | 734 - 746 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 17. |
|  | 704 - 716 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 17, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (NOTE 7) |
| UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 - 821 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 20 or 28. |
|  | 832 - 862 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 MHz | -40.0dBm | 1 MHz | This requirement does not apply to BS operating in band 22 or 42. |
|  | 3410 – 3490 MHz | -37.0dBm | 1 MHz | This requirement does not apply to BS operating in band 22, since it is already covered by the requirement in subclause 9.7.3.3. This requirement does not apply to Band 42. |
| E-UTRA Band 24 | 1525 – 1559 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 24. |
|  | 1626.5 – 1660.5 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 24, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 - 1995 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 2, 25 or 70. |
|  | 1850 - 1915 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 25, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in subclause 6.7.6.5.3.3. |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 - 894 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 814 - 849 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 26, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in subclause 6.7.6.5.3.3. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| E-UTRA Band 27 | 852 – 869 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in bands 5, 26 or 27. |
|  | 807 – 824 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 27, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in subclause 6.7.6.5.3.3. This requirement also applies to BS operating in Band 28, starting 4 MHz above the Band 28 downlink operating band (NOTE 6). |
| E-UTRA Band 28 or NR band n28 | 758 - 803 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 20, 28, 44, 67 or 68. |
|  | 703 - 748 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 28, since it is already covered by the requirement in subclause 6.7.6.5.3.3. This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733MHz. |
| E-UTRA Band 29 | 717 – 728 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in Band 29 |
| E-UTRA Band 30 | 2350 - 2360 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 30 or 40. |
|  | 2305 - 2315 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 30, since it is already covered by the requirement in subclause 6.7.6.5.3.3. This requirement does not apply to BS operating in Band 40. |
| E-UTRA Band 31 | 462.5 – 467.5 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 31. |
|  | 452.5 – 457.5 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 31, since it is already covered by the requirement in subclause 6.7.6.5.3.3. |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 11, 21 or 32. |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in Band 34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in Band 2, 25 or 36 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -40.4dBm | 1 MHz | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 – 2620 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in Band 38 or 69. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920MHz | -40.4dBm | 1 MHz | This is not applicable to BS operating in Band 39 |
| UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 – 2400MHz | -40.4dBm | 1 MHz | This is not applicable to BS operating in Band 30 or 40 |
| E-UTRA Band 41 or NR band n41 | 2496 – 2690MHz | -40.4dBm | 1 MHz | This is not applicable to BS operating in Band 41 |
| E-UTRA Band 42 | 3400 – 3600 MHz | -40.0dBm | 1 MHz | This is not applicable to BS operating in Band 22, 42 or 43 |
| E-UTRA Band 43 | 3600 – 3800 MHz | -40.0dBm | 1 MHz | This is not applicable to BS operating in Band 42 or 43 |
| E-UTRA Band 44 | 703 - 803 MHz | -40.4dBm | 1 MHz | This is not applicable to BS operating in Band 28 or 44 |
| E-UTRA Band 45 | 1447 - 1467 MHz | -40.4dBm | 1 MHz | This is not applicable to BS operating in Band 45 |
| E-UTRA Band 46 | 5150 - 5925 MHz | -39.5 dBm | 1 MHz | This is not applicable to BS operating in Band 46 |
| E-UTRA Band 50 or NR Band n50 | 1432 - 1517 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76 |
| E-UTRA Band 51 or NR Band n51 | 1427 - 1432 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 50, 51, 75 or 76. |
| E-UTRA Band 65 | 2110 - 2200 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 1 or 65, |
|  | 1920 - 2010 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 65, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in subclause 6.7.6.5.3.3. |
| E-UTRA Band 66 or NR Band n66 | 2110 - 2200 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 4, 10, 23 or 66. |
|  | 1710 - 1780 MHz | -37.4dBm | 1 MHz | This requirement does not apply to BS operating in band 66, since it is already covered by the requirement in subclause 6.7.6.5.3.3. For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in subclause 6.7.6.5.3.3. For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in subclause 6.7.6.5.3.3. |
| E-UTRA Band 67 | 738 – 758 MHz | -40.4dBm | 1 MHz | This requirement does not apply to BS operating in band 28 or 67. |
| E-UTRA Band 68 | 753 -783 MHz | -40.4dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 28, or 68. |  |
|  | 698-728 MHz | -37.4dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 68, since it is already covered by the requirement in subclause 9.7.3.3. For E-UTRA BS operating in Band 28, it applies between 698 MHz and 703 MHz, while the rest is covered in subclause 9.7.3.3. |  |
| E-UTRA Band 69 | 2570 - 2620 MHz | -40.4dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 38 or 69. |  |
| E-UTRA Band 70 or NR Band n70 | 1995 - 2020 MHz | -40.4dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 2, 25 or 70 |  |
|  | 1695 – 1710 MHz | -37.4dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 70, since it is already covered by the requirement in subclause 6.7.6.5.3.3 |  |
| E-UTRA Band 71 or NR Band n71 | 617 - 652 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 71. |  |
|  | 663 – 698 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 71/n71, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 |  |
| E-UTRA Band 72 | 461 - 466 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  | 451 - 456 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 72, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 This requirement does not apply to BS operating in band 73. |  |
| E-UTRA Band 73 | 460 - 465 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  | 450 - 455 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 73, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 |  |
| E-UTRA Band 74 or NR band n74 | 1475 – 1518 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 11, 21, 32, 50 74 or 75. |  |
|  | 1427 – 1470 MHz | -37.4 dBm | 1MHz | This requirement does not apply to BS operating in Band 74, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 This requirement does not apply to BS operating in band 32, 45, 50, 51, 75 or 76. |  |
| E-UTRA Band 75 or NR Band n75 | 1432 - 1517 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76. |  |
| E-UTRA Band 76 or NR Band n76 | 1427 - 1432 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in Band 50, 51, 75 or 76. |  |
| NR Band n77 | 3300 MHz – 4200 MHz | -40.4 dBm | 1 MHz | This is not applicable to BS operating in Band 42, 43, 48, 49 |  |
| NR Band n78 | 3300 MHz – 3800 MHz | -40.4 dBm | 1 MHz | This is not applicable to BS operating in Band 42, 43, 48, 49 |  |
| NR Band n80 | 1710 - 1785 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in sub-clause 6.7.6.3.5.3.3 |  |
| NR Band n81 | 880 - 915 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 |  |
| NR Band n82 | 832 - 862 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in subclause 6.7.6.3.5.3.3 |  |
| NR Band n83 | 703 - 748 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 28, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736MHz. For BS operating in Band 68, it applies for 728MHz to 733MHz. |  |
| NR Band n84 | 1920 - 1980 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 1 or 65, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 |  |
| E-UTRA Band 85 | 728 - 746 MHz | -40.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 12, 29 or 85. |  |
|  | 698 - 716 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 85, since it is already covered by the requirement in sub-clause 6.7.6.3.5.3.3 For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |  |
| NR Band n86 | 1710 - 1780 MHz | -37.4 dBm | 1 MHz | This requirement does not apply to BS operating in band 66/n66, since it is already covered by the requirement in sub-clause 6.7.6.3.5.13.3. For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in sub-clause 6.7.6.3.5.13.3. For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in sub-clause 6.7.6.3.5.3.3 |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in table 6.7.6.4.5.3-1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see subclause 6.7.1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.7.6.4.5.3-1 assumes that two operating bands, where the frequency ranges in subclause 4.7 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III or E-UTRA Band 3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 – 1850 MHz and 1710 – 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations (except in Band 46), special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 6:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band.

NOTE 7:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band or E-UTRA Band 17 UL operating band.

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between 10 MHz below the lowest BS transmitter frequency of the downlink operating band and 10 MHz above the highest BS transmitter frequency of the downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-2: AAS BS OTA Spurious emissions limits for BS for co-existence with PHS

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -32 dBm | 300 kHz | Applicable for co-existence with PHS system operating in 1884.5-1915.7MHz |
| NOTE: The requirement is not applicable in China. |  |  |  |

The following requirement shall be applied to AAS BS operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-3: AAS BS OTA Spurious emissions limits for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 13 | 793 - 805 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 769 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 799 - 805 MHz | -37 dBm | 6.25 kHz |  |

The following requirement shall be applied to AAS BS operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-4: AAS BS OTA Spurious emissions limits for protection of 800 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -4 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

The following requirement may apply to E-UTRA AAS BS operating in Band 41 in certain regions. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-5: Additional AAS BS OTA Spurious emissions limits for Band 41

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 2505MHz – 2535MHz | -30.4 dBm | 1 MHz |  |
| 2535MHz – 2655MHz | -10.4 dBm | 1 MHz | Applicable at offsets ≥ 250% of channel bandwidth from carrier frequency |
| NOTE: This requirement applies for 10 or 20 MHz E-UTRA carriers allocated within 2545-2575MHz or 2595-2645MHz. |  |  |  |

The following requirement may apply to AAS BS operating in Band 30 in certain regions. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-6: Additional AAS BS OTA Spurious emissions limits for Band 30

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 2200MHz – 2345MHz | -33.4 dBm | 1 MHz |  |
| 2362.5MHz – 2365MHz | -13.4 dBm | 1 MHz |  |
| 2365MHz – 2367.5MHz | -28.4 dBm | 1 MHz |  |
| 2367.5MHz – 2370MHz | -30.4 dBm | 1 MHz |  |
| 2370MHz – 2395MHz | -33.4 dBm | 1 MHz |  |

The following requirement may apply to AAS BS operating in Band 48 in certain regions. The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-7: Additional AAS BS OTA Spurious emissions limits for Band 48

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 3530MHz – 3720MHz | -13.4 dBm | 1 MHz | Applicable 10MHz from the assigned channel edge |
| 3100MHz – 3530MHz 3720MHz – 4200MHz | -28.0 dBm | 1 MHz |  |

In addition to the requirements in subclauses 6.7.6.5.3.1, 6.7.6.5.3.2, 6.7.6.5.3.3 and above in the present subclause, the AAS BS may have to comply with the applicable emission limits established by FCC Title 47 [18], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

Additional co-existence requirements in table 6.7.6.4.5.3-8 may apply for some regions. The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-8: AAS BS OTA Spurious emissions limits for E-UTRA BS for co-existence with systems operating in Band 46

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| E-UTRA Band 46a | 5150 - 5250 MHz | -27.5 dBm | 1 MHz | This is only applicable to E-UTRA BS operating in Band 46c or 46d. |
| E-UTRA Band 46b | 5250 - 5350 MHz | -27.5 dBm | 1 MHz | This is only applicable to E-UTRA BS operating in Band 46c or 46d. |
| E-UTRA Band 46c | 5470 - 5725 MHz | -27.5 dBm | 1 MHz | This is only applicable to E-UTRA BS operating in Band 46a or 46b. |
| E-UTRA Band 46d | 5725 - 5925 MHz | -27.5 dBm | 1 MHz | This is only applicable to E-UTRA BS operating in Band 46a or 46b. |
| NOTE: This requirement may apply to E-UTRA BS operating in certain regions. |  |  |  |  |

The following requirement shall be applied to AAS BS operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-9: AAS BS OTA Spurious emissions limits for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 13 | 793 - 805 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 769 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 799 - 805 MHz | -37 dBm | 6.25 kHz |  |

The following requirement shall be applied to AAS BS operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 6.7.6.4.5.3-10: AAS BS OTA Spurious emissions limits for protection of 800 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -13 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

#### 6.7.6.5	Co-location with other base stations

##### 6.7.6.5.1	Definition and applicability

##### 6.7.6.5.2	Minimum Requirement

The minimum requirement for AAS BS in MSR operation is defined in TS 37.105 [6], subclause 9.7.6.2.

The minimum requirement for AAS BS in single RAT UTRA operation is defined in TS 37.105 [6], subclause 9.7.6.3.

The minimum requirement for AAS BS in single RAT E-UTRA operation is defined in TS 37.105 [6], subclause 9.7.6.4.

##### 6.7.6.5.3	Test purpose

The test purpose of OTA spurious emission is to verify the radiated spurious emissions from the AAS BS at the RIB are within specified requirements.

For OTA co-location spurious emission, the test purpose is to verify that the emission is within the specified requirement limits at the CLTA conducted output(s).

##### 6.7.6.5.4	Method of test

###### 6.7.6.5.4.1	Initial conditions

See sub-clause 6.7.6.3.4.1.

###### 6.7.6.5.4.2	Procedure

See sub-clause 6.7.6.3.4.2.

##### 6.7.6.5.5	Test Requirement

###### 6.7.6.5.5.1	MSR operation

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

The requirements are co-location emission requirements are specified as the power sum of the supported polarization(s) at the CLTA conducted output(s).

The output of the CLTA of any spurious emission shall not exceed the limits of table 6.7.6.5.5.1-1 for a AAS BS where requirements for co-location with a BS type listed in the first column apply, depending on the declared Base Station class. For a multi-band RIB, the exclusions and conditions in the notes column of table 6.7.6.5.5.1-1 apply for each supported operating band.

Table 6.7.6.5.5.1-1: AAS BS OTA Spurious emissions E-UTRA limits for AAS BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level (WA-BS) | Maximum Level (MR-BS) | Maximum Level (LA-BS) | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876-915 MHz | -115.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| DCS1800 | 1710 - 1785 MHz | -115.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| PCS1900 | 1850 - 1910 MHz | -115.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 - 849 MHz | -115.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 - 1980 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1850 - 1910 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1710 - 1785 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 824 - 849 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 845 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2500 - 2570 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 880 - 915 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 50/n50, 51/n51, 75/n75, 76/n76 |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 - 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 - 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 18 | 815 - 830 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 832 - 862 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 – 1462.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 32, 50/n50, 75/n75 |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -113.7 dBm | -108.7dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42, n77 or n78 |
| E-UTRA Band 23 | 2000 - 2020 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1850 - 1915 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814 - 849 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 27 | 807 - 824 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 44 |
| E-UTRA Band 30 | 2305 - 2315 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 40/n40 |
| E-UTRA Band 31 | 452.5 – 457.5 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 or NR Band n34 | 2010 - 2025 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 34/n34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 2, n2 and 36 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 38/n38. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR Band n39 | 1880 – 1920MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 33 and 39/n39 |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 30 or 40/n40 |
| E-UTRA Band 41 or NR Band n41 | 2496 – 2690MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 41/n41 |
| E-UTRA Band 42 | 3400 – 3600 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 22, 42, 43, 48, 49, 52, n77 or n78 |
| E-UTRA Band 43 | 3600 – 3800 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49, n77 or n78 |
| E-UTRA Band 44 | 703 – 803 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 28/n28 or 44 |
| E-UTRA Band 45 | 1447 – 1467 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 45 |
| E-UTRA Band 46 | 5150 – 5925 MHz | N/A | -108.6 dBm | -105.6 dBm | 100 kHz | This is not applicable to BS operating in Band 46 |
| E-UTRA Band 48 | 3550 – 3700 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49, n77 or n78 |
| E-UTRA Band 49 | 3550 – 3700 MHz | N/A | N/A | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49, n77 or n78 |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 11, 21, 32, 51, n51, 74, 75/n75, 76/n76 |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | N/A | N/A | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 50/n50, 75/n75, 76/n76 |
| E-UTRA Band 52 | 3300 – 3400 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 42 or 52 |
| E-UTRA Band 65 | 1920 - 2010 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 66 or NR Band n66 | 1710 – 1780 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 68 | 698 – 728 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 70 or NR Band n70 or NR band n70 | 1695 – 1710 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 71 or NR Band n71 | 663 – 698 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 72 | 451 – 456 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 73 | 450 – 455 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 74 or NR band n74 | 1427 – 1470 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 50/n50, 51/n51 |
| NR Band n77 | 3300 MHz – 4200 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42 43, 48, 49, n77 or n78 |
| NR Band n78 | 3300 MHz – 3800 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49, n77 or n78 |
| NR band n79 | 4400 – 5000 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz |  |
| E-UTRA Band 85 | 698 - 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, the co-location requirements in table 6.7.6.5.3.5-1 do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see subclause 6.7.1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [31].

NOTE 2:	Table 6.7.6.5.3.5-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in subclause 4.6 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

###### NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

###### 6.7.6.5.5.2	Single RAT UTRA operation

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD and/or E-UTRA BS are co-located with a BS.

The requirements assume with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

The requirements are co-location emission requirements and specified as the power sum of the supported polarization(s) at the CLTA conducted output(s).

The power sum of any spurious emission is specified over all supported polarizations at the conducted output(s) of the CLTA and shall not exceed the limits of table 6.7.6.5.5.2-1 for a AAS BS where requirements for co-location with a BS type listed in the first column apply, depending on the declared Base Station class. For a multi-band RIB, the exclusions and conditions in the Notes column of table 6.7.6.5.5.2-1 apply for each supported operating band.

Table 6.7.6.5.5.2-1: UTRA AAS BS OTA Spurious emissions limits for AAS BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level (WA-BS) | Maximum Level (MR-BS) | Maximum Level (LA-BS) | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876-915 MHz | -118.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| DCS1800 | 1710 - 1785 MHz | -118.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| PCS1900 | 1850 - 1910 MHz | -118.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 - 849 MHz | -118.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 1920 - 1980 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1850 - 1910 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1710 - 1785 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 824 - 849 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 845 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2500 - 2570 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 880 - 915 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 699 - 716 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 - 716 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 18 | 815 - 830 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 832 - 862 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 – 1462.9 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -116.7 dBm | -111.7 dBm | -108.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42 |
| E-UTRA Band 23 | 2000 - 2020 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 - 1915 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814 - 849 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 27 | 807 - 824 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 28 or NR band n28 | 703 – 748 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 44 |
| E-UTRA Band 30 | 2305 - 2315 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 40 |
| E-UTRA Band 31 | 452.5 – 457.5 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 2 and 36 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 – 2620 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 38. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 33 and 39 |
| UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 – 2400MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 30 or 40 |
| E-UTRA Band 41 or NR band n41 | 2496 – 2690MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 41 |
| E-UTRA Band 42 | 3400 – 3600 MHz | -116.7 dBm | -111.7 dBm | -108.7 dBm | 100 kHz | This is not applicable to BS operating in Band 22, 42 or 43 |
| E-UTRA Band 43 | 3600 – 3800 MHz | -116.7 dBm | -111.7 dBm | -108.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42 or 43 |
| E-UTRA Band 44 | 703 – 803 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 28 or 44 |
| E-UTRA Band 45 | 1447 – 1467 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz | This is not applicable to BS operating in Band 45 |
| E-UTRA Band 46 | 5150 – 5925 MHz | N/A | -108.6 dBm | -105.6 dBm | 100 kHz |  |
| E-UTRA Band 48 | 3550 – 3700 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 49 | 3550 – 3700 MHz | N/A | N/A | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band XI |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | N/A | N/A | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 65 | 1920 - 2010 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 66 or NR band n66 | 1710 – 1780 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 68 | 698 – 728 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 70 or NR band n70 | 1695 – 1710 MHz | -116.9 dBm | -111.9 dBm | -108.9 dBm | 100 kHz |  |
| E-UTRA Band 71 or NR Band n71 | 663 – 698 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 72 | 451 – 456 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 73 | 450 – 455 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 74 or NR band n74 | 1427 – 1470 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| NR Band n77 | 3300 MHz – 4200 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz |  |
| NR Band n78 | 3300 MHz – 3800 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz |  |
| NR band n79 | 4400 – 5000 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz |  |
| E-UTRA Band 85 | 698 - 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, the co-location requirements in table 6.7.6.5.2.5-1 do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see subclause 6.7.1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [31].

NOTE 2:	Table 6.7.6.5.2.5-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in subclause 4.6 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

###### 6.7.6.5.5.3	Single RAT E-UTRA operation

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD and/or E-UTRA BS are co-located with a BS.

The requirements assume co-location with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

The requirements are co-location emission requirements and specified as the power sum of the supported polarization(s) at the CLTA conducted output(s).

The power sum of any spurious emission is specified over all supported polarizations at the conducted output(s) of the CLTA and shall not exceed the limits of table 6.7.6.5.5.3-1 for a AAS BS where requirements for co-location with a BS type listed in the first column apply, depending on the declared Base Station class. For a multi-band RIB, the exclusions and conditions in the notes column of table 6.7.6.5.5.3-1 apply for each supported operating band.

Table 6.7.6.5.5.3-1: AAS BS OTA Spurious emissions E-UTRA limits for AAS BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level (WA-BS) | Maximum Level (MR-BS) | Maximum Level (LA-BS) | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876-915 MHz | -115.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| DCS1800 | 1710 - 1785 MHz | -115.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| PCS1900 | 1850 - 1910 MHz | -115.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 - 849 MHz | -115.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 1920 - 1980 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1850 - 1910 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1710 - 1785 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 824 - 849 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 845 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2500 - 2570 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 880 - 915 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 699 - 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 - 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 18 | 815 - 830 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 832 - 862 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 – 1462.9 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -113.7 dBm | -108.7dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42 |
| E-UTRA Band 23 | 2000 - 2020 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 - 1915 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814 - 849 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 27 | 807 - 824 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 28 or NR band n28 | 703 – 748 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 44 |
| E-UTRA Band 30 | 2305 - 2315 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 40 |
| E-UTRA Band 31 | 452.5 – 457.5 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 2 and 36 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 – 2620 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 38. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 33 and 39 |
| UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 – 2400MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 30 or 40 |
| E-UTRA Band 41 or NR band n41 | 2496 – 2690MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 41 |
| E-UTRA Band 42 | 3400 – 3600 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 22, 42 or 43 |
| E-UTRA Band 43 | 3600 – 3800 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42 or 43 |
| E-UTRA Band 44 | 703 – 803 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 28 or 44 |
| E-UTRA Band 45 | 1447 – 1467 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 45 |
| E-UTRA Band 46 | 5150 – 5925 MHz | N/A | -108.6 dBm | -105.6 dBm | 100 kHz | This is not applicable to BS operating in Band 46 |
| E-UTRA Band 48 | 3550 – 3700 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49 |
| E-UTRA Band 49 | 3550 – 3700 MHz | N/A | N/A | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49 |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 11, 21, 32, 51, n51, 74 |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | N/A | N/A | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 50 |
| E-UTRA Band 52 | 3300 – 3400 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 42 or 52 |
| E-UTRA Band 65 | 1920 - 2010 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 66 | 1710 – 1780 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 68 | 698 – 728 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 70 | 1695 – 1710 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 71 or NR Band n71 | 663 – 698 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 72 | 451 – 456 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 73 | 450 – 455 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |
| E-UTRA Band 74 or NR band n74 | 1427 – 1470 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz | This is not applicable to BS operating in Band 50, 51 |
| NR Band n77 | 3300 MHz – 4200 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42 43, 48, 49 |
| NR Band n78 | 3300 MHz – 3800 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49 |
| NR band n79 | 4400 – 5000 MHz | -113.7 dBm | -108.7 dBm | -105.7 dBm | 100 kHz |  |
| E-UTRA Band 85 | 698 - 716 MHz | -113.9 dBm | -108.9 dBm | -105.9 dBm | 100 kHz |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, the co-location requirements in table 6.7.6.5.3.5-1 do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see subclause 6.7.1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [31].

NOTE 2:	Table 6.7.6.5.3.5-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in subclause 4.6 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

## 6.8	OTA Transmitter intermodulation

### 6.8.1	Definition and applicability

The OTA transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station. The requirement applies during the transmitter ON period and the transmitter transient period.

The requirement applies at each RIB supporting transmission in the operating band.

The transmitter intermodulation level is the total radiated power of the intermodulation products when an interfering signal is injected into the CLTA.

For OTA AAS BS, the transmitter intermodulation requirement is captured by the co-location transmitter intermodulation scenario case, in which the interfering signal is injected into the CLTA.

### 6.8.2	Minimum Requirement

The minimum requirement for AAS BS in MSR operation is defined in TS 37.105 [6], subclause 9.8.2.

The minimum requirement for AAS BS in single RAT UTRA operation is defined in TS 37.105 [6], subclause 9.8.3.

The minimum requirement for AAS BS in single RAT E-UTRA operation is defined in TS 37.105 [6], subclause 9.8.4.

### 6.8.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to restrict the generation of intermodulation products in its nonlinear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station to below specified levels.

### 6.8.4	Method of test

#### 6.8.4.1	Initial conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier:

-	M; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band RIB, see subclause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see subclause 4.12.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, emission testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, emission testing below the lowest operating band may be omitted.

-	FFS

Directions to be tested for:

As the requirement is based on TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex F) as long as the required TRP output power level is achieved.

#### 6.8.4.2	Procedure

1)	Select a CLTA according to parameters given in Table 4.15.2.2-1.

2)	Place the CLTA according to parameters given in Table 4.15.2.3-1.

3)	The test antenna(s) shall be dual (or single) polarized covering the same frequency range as the AAS BS and the emission frequencies.

4)	Several test antennas are required to cover both the AAS BS and the whole emission frequency range.

5)	Connect the test antenna and CLTA to the measurement equipment as shown in Annex D1.5, Figures D.1.5-1.

6)	During the OTA emission measurements at the test antenna conducted output(s), both AAS BS and CLTA are rotated around same axis.

7)	The OTA unwanted emissions measurement method shall be TRP, according to the procedure described in Annex F.

8)	The measurement device (signal analyzer) characteristics shall be:

-	Detection mode: True RMS.

9)	Set the AAS BS to transmit:

a)	For MSR:

-	Set the AAS BS to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.11.

b)	For UTRA FDD:

-	For a AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit maximum power according to TM1, subclause 4.12.2, at the manufacturer's declared rated output power, PRated,c,TRP.

-	For a AAS BS declared to be capable of multi-carrier operation, set the AAS BS to transmit maximum power according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

c)	For E-UTRA:

-	For AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit maximum power according to E-TM1.1 in subclause 4.12.2, at manufacturer's declared rated output power, PRated,c,TRP.

-	For a AAS BS declared to be capable of multi-carrier and/or CA operation, set the AAS BS to transmit maximum power according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.11.

10)	Generate the interfering signal via the CLTA. The CLTA shall be fed with a power level equal to declared Prated,t,TRP, divided over all supported polarizations, from the same signal generator source:

a)	For MSR:

-	using E-TM1.1 as defined in subclause 4.12.2, with 5 MHz channel bandwidth, at a centre frequency offset according to the conditions in table 6.8.5.1.1-1, but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

b)	For UTRA FDD:

-	in accordance to TM1, subclause 4.12.2 with a frequency offset according to the conditions of table 6.8.5.2.1-1, but exclude interfering signal frequencies that are outside of the allocated downlink operating band or interfering signal frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

c)	For E-UTRA:

-	according to E-TM1.1, as defined in subclause 4.12.2, with 5 MHz channel bandwidth and a centre frequency offset according to the conditions of table 6.8.5.3.1-1, but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

11)	Adjust the interfering signal level at the CLTA conducted input(s) as defined in:

a)	For MSR:

i.	General co-location table 6.8.5.1.1-1.

ii.	Additional co-location (BC1 and BC2) table 6.8.5.1.2-1.

iii.	Additional co-location (BC3) table 6.8.5.1.3-1.

b)	For UTRA FDD:

i.	General co-location table 6.8.5.2.1-1 .

c)	For E-UTRA:

i.	General co-location table 6.8.5.3.1-1.

ii.	Additional requirement for Band 41 table 6.8.5.3.2-1.

12)	If the interferer signal is applicable according to clause 5, perform the unwanted emission tests specified in subclauses 6.7.3 (OTA ACLR), 6.7.4 (OTA spectrum mask) and 6.7.5 (OTA OBUE), for all third and fifth order intermodulation products which appear in the frequency ranges defined in subclauses 6.7.3, 6.7.4 and 6.7.5 (NOTE 2). The width of the intermodulation products shall be taken into account.

13)	If the interferer signal is applicable according to clause 5, perform the transmitter spurious emissions test as specified in subclause 6.7.6 (OTA spurious emission), except OTA co-location spurious emission, for all third and fifth order intermodulation products which appear in the frequency ranges defined in subclause 6.7.6 (NOTE 2). The width of the intermodulation products shall be taken into account.

14)	Verify that the emission level does not exceed the required level in subclause 6.8.5 (Test requirements) with the exception of interfering signal frequencies.

15)	Repeat the test for the remaining interfering signal centre frequency offsets according to the conditions of:

a)	For MSR:

i.	General co-location table 6.8.5.1.1-1.

ii.	Additional co-location (BC1 and BC2) table 6.8.5.1.2-1.

iii.	Additional co-location (BC3) table 6.8.5.1.3-1.

b)	For UTRA FDD:

i.	General co-location table 6.8.5.2.1-1 .

c)	For E-UTRA:

i.	General co-location table 6.8.5.3.1-1.

ii.	Additional requirement for Band 41 table 6.8.5.3.2-1.

16)	Repeat the test for the remaining interfering signals defined in clause 5 for requirements 6.7.3 (OTA ACLR), 6.7.4 (OTA spectrum mask), 6.7.5 (OTA OBUE) and 6.7.6 (OTA spurious emission), except OTA co-location spurious emission.

In addition, for multi-band AAS BS, the following steps shall apply:

17)	For multi-band AAS BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

NOTE 1:	The third order intermodulation products are centred at 2F1F2 and 2F2F1. The fifth order intermodulation products are centred at 3F12F2, 3F22F1, 4F1F2, and 4F2F1 where F1 represents the test signal centre frequency or centre frequency of each sub-block and F2 represents the interfering signal centre frequency. The widths of intermodulation products are:

-	(n*BWF1 + m*1.6MHz) for the nF1mF2 products;

-	(n*1.6MHz + m* BWF1) for the nF2mF1 products;

where BWF1 represents the test signal RF bandwidth or channel bandwidth in case of single carrier, or sub-block bandwidth.

NOTE 2:	During the conformance test the interferer signal can be applied on one side of the wanted signal, while the transmitter intermodulation emission is measured only on the opposite side of the wanted signal. This applies for intermodulation products which are within the operating band or OBUE region.

### 6.8.5	Test Requirement

#### 6.8.5.1	MSR test requirements

##### 6.8.5.1.1	General test requirement

In the frequency range relevant for this test the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in subclause 6.7.6 (except co-location spurious emission), operating band unwanted emission in subclause 6.7.5 and ACLR in subclause 6.7.3 in the presence of a wanted signal and an interfering signal according to table 6.8.5.1.1-1 for an OTA AAS BS operating in BC1, BC2 and BC3.

The requirement is applicable outside the edges of the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or radio bandwidth edges. 

For RIB supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For multi-band RIBs, the requirement applies relative to the Base Station RF Bandwidth edges of each operating band. In case the inter Base Station RF Bandwidth gap is less than 15 MHz, the requirement in the gap applies only for interfering signal offsets where the interfering signal falls completely within the inter Base Station RF Bandwidth gap.

Table 6.8.5.1.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | E-UTRA or NR signal |
| Interfering signal type | E-UTRA signal of channel bandwidth 5 MHz |
| Interfering signal level applied to the CLTA | Rated total output power per RIB in the operating band (corresponding to PRated,t,TRP) |
| Interfering signal centre frequency offset from Base Station RF Bandwidth edge or edge of sub-block inside a gap | ±2.5 MHz ±7.5 MHz ±12.5 MHz |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the RIB is excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, 3GPP TS 37.141 provides further guidance regarding appropriate test requirements. NOTE 2: In certain regions, NOTE 1 is not applied in Band 1, 3, 8, 9, 11, 18, 19, 21, 28, 32 operating within 1 475.9 MHz to 1 495.9 MHz, 34. NOTE 3: The Prated,t,TRP is split between supported polarizations at the CLTA input ports. |  |

##### 6.8.5.1.2	Additional test requirement (BC1 and BC2)

In the frequency range relevant for this test the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in subclause 6.7.6 (except co-location spurious emission), operating band unwanted emission in subclause 6.7.5 and ACLR in subclause 6.7.3 in the presence of a wanted signal and an interfering signal according to table 6.8.5.1.2-1 for an OTA AAS BS operating in BC2.

The requirement is applicable outside the edges of the Base Station RF Bandwidth for BC2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum in BC1 or BC2, the requirement is also applicable inside a sub-block gap with a gap size larger than or equal to two times the interfering signal centre frequency offset. For RIBs supporting operation in non-contiguous spectrum in BC1, the requirement is not applicable inside a sub-block gap with a gap size equal to or larger than 5 MHz. The interfering signal offset is defined relative to the sub-block edges.

For multi-band RIBs, the requirement applies relative to the Base Station RF Bandwidth edges of a BC2 operating band. The requirement is also applicable for BC1 and BC2 inside an inter Base Station RF Bandwidth gap equal to or larger than two times the interfering signal centre frequency offset. For RIBs supporting operation in multiple operating bands, the requirement is not applicable for BC1 band inside an inter Base Station RF Bandwidth gap with a gap size equal to or larger than 5 MHz.

Table 6.8.5.1.2-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | E-UTRA and/or NR UTRA signal |
| Interfering signal type | CW |
| Interfering signal level applied to the CLTA | Rated total output power per RIB in the operating band (corresponding to PRated,t,TRP) |
| Interfering signal centre frequency offset from Base Station RF Bandwidth edge or edge of sub-block inside a gap | > abs(800) kHz for CW interferer |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the RIB are excluded from the requirement. NOTE 2: The Prated,t,TRP is split between polarizations at the CLTA. |  |

##### 6.8.5.1.3	Additional test requirement (BC3)

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in subclause 6.7.6 (except co-location spurious emission), operating band unwanted emission in subclause 6.7.5 and ACLR in subclause 6.7.3 in the presence of a wanted signal and an interfering signal according table 6.8.5.1.3-1 an OTA AAS BS operating in BC3.

For multi-band RIBs, the requirement applies relative to the Base Station RF Bandwidth edges of each operating band. In case the Inter RF Bandwidth gap is less than 3.2 MHz, the requirement in the gap applies only for interfering signal offsets where the interfering signal falls completely within the inter Base Station RF Bandwidth gap.

Table 6.8.5.1.3-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement (BC3)

| Parameter | Value |
| --- | --- |
| Wanted signal type | E-UTRA and/or UTRA and/or NR signal |
| Interfering signal type | 1,28 Mcps UTRA TDD signal of channel bandwidth 1,6 MHz |
| Interfering signal level applied to the CLTA | Rated total output power per RIB in the operating band (corresponding to PRated,t,TRP) |
| Interfering signal centre frequency offset from Base Station RF Bandwidth edge or edge of sub-block inside a gap | ±0,8 MHz ±1,6 MHz ±2,4 MHz |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement. NOTE 2: The Prated,t,TRP is split between polarizations at the CLTA. |  |

#### 6.8.5.2	Single RAT UTRA operation

##### 6.8.5.2.1	General test requirement for UTRA FDD

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the out of band emission or the spurious emission requirements of subclause 6.7.4 (OTA spectrum mask) and subclause 6.7.6 (OTA spurious emission, except co-location spurious emission), in the presence of interfering signal according to table 6.8.5.2.1-1.

For RIBs supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For multi-band RIBs, the requirement is also applicable inside an Inter RF Bandwidth gap for interfering signal offsets where the interfering signal falls completely within the Base Station RF Bandwidth gap.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

Table 6.8.5.2.1-1: Interfering and wanted signal frequency offset for OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | UTRA |
| Interfering signal type | UTRA |
| Interfering signal level applied to the CLTA | Rated total output power per RIB in the operating band (corresponding to PRated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | -2,5 MHz -7,5 MHz -12,5 MHz +2,5 MHz +7,5 MHz +12,5 MHz |
| NOTE 1: Interference frequencies that are outside of any allocated frequency band for UTRA-FDD downlink specified in subclause 4.6 are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: NOTE 1 is not applied in Band I, III, VI, VIII, IX, XI, XIX, XXI, and XXXII operating within 1 475.9 MHz to 1 495.9MHz, in certain regions. NOTE 3: The Prated,t,TRP is split between polarizations at the CLTA. |  |

#### 6.8.5.3	Single RAT E-UTRA operation

##### 6.8.5.3.1	General test requirement

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the unwanted emission limits in subclauses 6.7.6 (OTA spurious emission, except co-location spurious emission), 6.7.5 (OTA OBUE) and 6.7.3 (OTA ACLR) in the presence of an E-UTRA interfering signal according to according to table 6.8.5.3.1-1.

The requirement is applicable outside the Base Station RF Bandwidth or radio bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or radio bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For multi-band RIBs, the requirement applies relative to the Base Station RF Bandwidth edges of each supported operating band. In case the Inter RF Bandwidth gap is less than 15 MHz, the requirement in the gap applies only for interfering signal offsets where the interfering signal falls completely within the inter Base Station RF Bandwidth gap.

Table 6.8.5.3.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | E-UTRA single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | E-UTRA signal of channel bandwidth 5 MHz |
| Interfering signal level applied to the CLTA | Rated total output power per RIB in the operating band (corresponding to PRated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a sub-block gap | ±2,5 MHz ±7,5 MHz ±12,5 MHz |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, 3GPP TS 36.141 provides further guidance regarding appropriate test requirements. NOTE 2: In certain regions, NOTE 1 is not applied in Band 1, 3, 8, 9, 11, 18, 19, 21, 28, 32 operating within 1 475.9 MHz to 1 495.9 MHz, 34. NOTE 3: The Prated,t,TRP is split between polarizations at the CLTA. |  |

##### 6.8.5.3.2	Additional test requirement for Band 41

In certain regions, the following requirement may apply: For E-UTRA single RAT AAS BS operating in operating band 41 in the presence of an interfering signal according to table 6.8.5.3.2-1, the OTA transmitter intermodulation level shall not exceed the maximum levels for OTA spurious emission (except co-location spurious emission), and OTA operating band unwanted emission specified additionally for operating band 41 single RAT E-UTRA operation. Also, the OTA ACLR requirements for same carrier type assumed in adjacent channels shall be fulfilled in the presence of the interfering signal.

Table 6.8.5.3.2-1: Interfering and wanted signals for the additional OTA transmitter intermodulation requirement for Band 41

| Parameter | Value |
| --- | --- |
| Wanted signal | E-UTRA single carrier (NOTE) |
| Interfering signal type | E-UTRA signal of the same channel bandwidth as the wanted signal |
| Interfering signal level applied to the CLTA | Rated total output power per RIB in the operating band (corresponding to PRated,t,TRP) |
| Interfering signal centre frequency offset from the centre frequency of the wanted signal | ±BWChannel ±2 x BWChannel |
| NOTE 1: This requirement applies for 10 MHz or 20 MHz E-UTRA carriers allocated within 2 545 MHz to 2 575 MHz or 2 595 MHz to 2 645 MHz. NOTE 2: The Prated,t,TRP is split between polarizations at the CLTA. |  |

# 7	Radiated receiver characteristics

## 7.1	General

OTA receiver characteristics requirements apply to the AAS BS including all its functional components active unless otherwise stated in each requirement.

Unless otherwise stated the requirements in clause 7 apply during the AAS BS receive period.

The requirements in clause 7 shall be met for any transmitter setting.

The (E-UTRA) throughput requirements defined for the receiver characteristics in this clause do not assume HARQ retransmissions.

When the AAS BS is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

Any radiated receiver test requirement specified in 3GPP TS 37.105 [6] is not applicable for AAS BS operation in Band 46.

Each requirement shall be met over the RoAoA specified.

For requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔOTAREFSENS = 44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) (dB) for the reference direction.

And

ΔOTAREFSENS = 41.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) (dB) for all other directions.

For requirements which are to be met over the minSENS RoAoA absolute requirement values are offset by the following term:

ΔminSENS = PREFSENS – EISminSENS (dB)

## 7.2	OTA sensitivity

### 7.2.1	Definition and applicability

The OTA sensitivity requirement is based upon the declaration of one or more OTA sensitivity direction declarations (OSDD), related to an AAS BS receiver.

The AAS BS receiver may optionally be capable of redirecting/changing the receiver target by means of adjusting BS settings resulting in multiple sensitivity RoAoA. The sensitivity RoAoA resulting from the current AAS BS settings is the active sensitivity RoAoA.

If the AAS BS is capable of redirecting the receiver target related to the OSDD then the OSDD shall include:

-	The set(s) of RAT, Channel bandwidth and declared minimum EIS level applicable to any active sensitivity RoAoA inside the receiver target redirection range in the OSDD.

-	A declared receiver target redirection range, describing all the angles of arrival that can be addressed for the OSDD through alternative settings in the AAS BS.

-	Five declared sensitivity RoAoA comprising the conformance testing directions as detailed in [7].

-	The receiver target reference direction.

NOTE 1:	Some of the declared sensitivity RoAoA may coincide depending on the redirection capability.

NOTE 2:	In addition to the declared sensitivity RoAoA, several sensitivity RoAoA may be implicitly defined by the receiver target redirection range without being explicitly declared in the OSDD.

NOTE 3:	(Void)

If the AAS BS is not capable of redirecting the receiver target related to the OSDD, then the OSDD includes only:

-	The set(s) of RAT, Channel bandwidth and declared minimum EIS level applicable to the sensitivity RoAoA in the OSDD.

-	One declared active sensitivity RoAoA.

-	The receiver target reference direction.

NOTE 4:	For AAS BS without target redirection capability, the declared (fixed) sensitivity RoAoA is always the active sensitivity RoAoA.

The OTA sensitivity EIS level declaration shall apply to all supported polarizations, under the assumption of polarization match.

### 7.2.2	Minimum Requirement

### The minimum requirement for MSR & NR operation is in 3GPP TS 37.105 [6], subclause 10.2.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.2.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.2.4.

### 7.2.3	Test Purpose

The test purpose is to verify that the AAS BS can meet the BER or throughput requirement for a specified measurement channel at the EIS level and the range of angles of arrival declared in the OSDD.

### 7.2.4	Method of test

#### 7.2.4.1	Initial conditions

Test environment:

-	normal: see annex G.2.

RF channels to be tested:

-	M; see subclause 4.12.1.

Directions to be tested:

-	receiver target reference direction (see table 4.10-1, D10.9),

-	conformance test directions (see table 4.10-1, D10.10).

#### 7.2.4.2	Procedure

1)	Place the AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D1.1.

2)	Align the manufacturer declared coordinate system orientation of the AAS BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the AAS BS under test.

5)	Configure the beam peak direction of the AAS BS according to declared reference beam direction pair for the appropriate beam identifier.

6)	Set the AAS BS to transmit the beam(s) of the same operational band and RAT as the OSDD being tested according to the appropriate test configuration in clause 5.

7)	Start the signal generator for the wanted signal to transmit:

-	12.2 kbps DPCH with reference measurement channel defined in annex A in 3GPP TS 25.141 [10] (PN-9 data sequence or longer) for UTRA FDD.

-	UL reference measurement channel (12.2 kbps) defined in annex A.2.1 in 3GPP TS 25.142 [11] for UTRA TDD 1.28Mcps operation.

-	The test signal as specified in subclause 7.2.5.4 for E-UTRA.

-	The test signal as specified in subclause 7.2.5.5 for NR.

8)	Set the test signal mean power so the calibrated radiated power at the AAS BS Antenna Array coordinate system reference point is as specified in subclause 7.2.5.

9)	Measure:

-	BER according to annex C in 3GPP TS 25.141 [10] for FDD UTRA.

-	BER  according to annex F in 3GPP TS 25.142 [11] for TDD UTRA.

-	Throughput according to annex E in 3GPP TS 36.141 [12] for E-UTRA.

-	Throughput according to annex A in 3GPP TS 38.141-2 [34] for NR.

10)	Repeat steps 3 to 9 for all OSDD(s) declared for the AAS BS (see table 4.10-1, D10.1), and supported polarizations.

For multi-band capable AAS BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.2.5	Test Requirements

#### 7.2.5.1	General

The minimum EIS level is a declared figure for each OSDD (see table 4.10-1, D10.6). The test requirement is calculated from the declared value offset by the EIS Test Tolerance specified in subclause 4.1.2.3.

#### 7.2.5.2	UTRA FDD Test Requirements

The BER measurement result in step 9 of subclause 7.2.4.2 shall not be greater than the limit specified in table 7.2.5.2-1.

Table 7.2.5.2-1: EIS levels

| Reference measurement channel | Reference measurement channel data rate | OTA sensitivity ( dBm) |  | BER |
| --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |
| 12.2kbps DPCH with reference measurement channel defined in annex A in 3GPP TS 25.141 [10] (PN-9 data sequence or longer) | 12.2 kbps | Declared minimum EIS (D10.6) + 1.3 dB | Declared minimum EIS (D10.6) + 1.4 dB | BER shall not exceed 0.001 |

#### 7.2.5.3	UTRA TDD 1,28Mcp option Test Requirements

The BER measurement result in step 9 of subclause 7.2.4.2 shall not be greater than the limit specified in table 7.2.5.3-1.

Table 7.2.5.3-1: EIS levels

| Reference measurement channel | Reference measurement channel data rate | OTA sensitivity ( dBm) |  | BER |
| --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |
| UL reference measurement channel (12.2 kbps) defined in annex A.2.1 in 3GPP TS 25.142 [11] | 12.2 kbps | Declared minimum EIS (D10.6) + 1.3 dB | Declared minimum EIS (D10.6) + 1.4 dB | BER shall not exceed 0.001 |

#### 7.2.5.4	E-UTRA Test Requirements

For each measured E-UTRA carrier, the throughput measured in step 9 of subclause 7.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in 3GPP TS 36.141 [12] annex A.1 with parameters specified in table 7.2.5.4-1.

Table 7.2.5.4-1: EIS levels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | OTA sensitivity (dBm) |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 1.4 | FRC A1-1 in annex A.1 [12] | Declared minimum EIS (D10.6) + 1.3 dB | Declared minimum EIS (D10.6) + 1.4 dB |
| 3 | FRC A1-2 in annex A.1 [12] |  |  |
| 5 | FRC A1-3 in annex A.1 [12] |  |  |
| 10 | FRC A1-3 in annex A.1 [12] (Note) |  |  |
| 15 |  |  |  |
| 20 |  |  |  |
| NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 in [11] mapped to disjoint frequency ranges with a width of 25 Resource Blocks each. |  |  |  |

#### 7.2.5.5	NR Test Requirements

For each measured carrier, the throughput measured in step 9 of subclause 7.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 38.104 [33] annex A.1 with parameters specified in table 7.2.5.5-1.

Table 7.2.5.5-1: EIS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EIS level [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 | Declared minimum EIS + 1.6 |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 | Declared minimum EIS + 1.6 |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 | Declared minimum EIS + 1.6 |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 | Declared minimum EIS + 1.6 |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 | Declared minimum EIS + 1.6 |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 | Declared minimum EIS + 1.6 |

## 7.3	OTA Reference sensitivity level

### 7.3.1	Definition and applicability

The OTA REFSENS requirement is intended to ensure the OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA REFSENS requirement shall apply to each supported polarization, under the assumption of polarization match.

The OTA reference sensitivity power level EISREFSENS is the mean power received at the radiated interface at which a reference performance requirement shall be met for a specified reference measurement channel.

### The OTA REFSENS EIS level declaration shall apply to all supported polarizations, under the assumption of polarization match.

### 7.3.2	Minimum Requirement

The minimum requirement for MSR & NR operation is in 3GPP TS 37.105 [6], subclause 10.3.2.

The minimum requirement for AAS BS in single RAT UTRA operation is defined in TS 37.105 [6], subclause 10.3.3.

The minimum requirement for AAS BS in single RAT E-UTRA operation is defined in TS 37.105 [6], subclause 10.3.4.

### 7.3.3	Test purpose

The test purpose is to verify that the AAS BS can meet the BER or throughput requirement for a specified measurement channel at the EISREFSENS level and the range of angles of arrival within the OTA REFSENS RoAoA.

### 7.3.4	Method of test

#### 7.3.4.1	Initial conditions

Test environment:

-	normal: see annex G.2.

RF channels to be tested:

-	B, M and T; see subclause 4.12.1.

Directions to be tested:

-	OTA REFSENS receiver target reference direction (see table 4.10-2, D11.29),

-	OTA REFSENS conformance test directions (see table 4.10-2, D11.31).

#### 7.3.4.2	Procedure

1)	Place the AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D1.1.

2)	Align the manufacturer declared coordinate system orientation of the AAS BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the AAS BS under test.

5)	Configure the beam peak direction of the AAS BS according to declared reference beam direction pair for the appropriate beam identifier.

6)	Set the AAS BS to transmit the beam(s) of the same operational band and RAT as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in clause 5.

7)	Start the signal generator for the wanted signal to transmit:

-	12.2 kbps DPCH with reference measurement channel defined in annex A in 3GPP TS 25.141 [10] (PN-9 data sequence or longer) for UTRA FDD.

-	The test signal as specified in subclause 7.3.5.3 for E-UTRA.

-	The test signal as specified in subclause 7.3.5.4 for NR.

8)	Set the test signal mean power so the calibrated radiated power at the AAS BS Antenna Array coordinate system reference point is as specified in subclause 7.3.5.

9)	Measure:

-	BER according to annex C in 3GPP TS 25.141 [10] for FDD UTRA.

-	Throughput according to annex E in 3GPP TS 36.141 [12] for E-UTRA.

-	Throughput according to annex A in 3GPP TS 38.141-2 [34] for NR.

10)	Repeat steps 3 to 9 for all directions to be tested, and supported polarizations.

For multi-band capable AAS BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.3.5	Test Requirement

#### 7.3.5.1	General

The EISREFSENS level is the conducted REFSENS requirement value offset by ΔOTAREFSENS. The test requirement is calculated from the EISREFSENS level offset by the EISREFSENS Test Tolerance specified in subclause 4.1.2.3.

#### 7.3.5.2	UTRA FDD Test Requirements

The BER measurement result in step 9 of subclause 7.3.4.2 shall not be greater than the limit specified in table 7.3.5.2-1.

Table 7.3.5.2-1: EISREFSENS levels

| Reference measurement channel | Reference measurement channel data rate | EISREFSENS (dBm) |  | BER |
| --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |
| 12.2kbps DPCH with reference measurement channel defined in annex A in 3GPP TS 25.141 [10] (PN-9 data sequence or longer) | 12.2 kbps | -119.7 - ΔOTAREFSENS | -119.6 - ΔOTAREFSENS | BER shall not exceed 0.001 |

#### 7.3.5.3	E-UTRA Test Requirements

For each measured E-UTRA carrier, the throughput measured in step 9 of subclause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in 3GPP TS 36.141 [12] annex A.1 with parameters specified in table 7.3.5.3-1.

Table 7.3.5.3-1: EISREFSENS levels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | EISREFSENS (dBm) |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 1.4 | FRC A1-1 in annex A.1 [12] | -105.5 - ΔOTAREFSENS | -105.4 - ΔOTAREFSENS |
| 3 | FRC A1-2 in annex A.1 [12] | -101.7 - ΔOTAREFSENS | -101.6 - ΔOTAREFSENS |
| 5 | FRC A1-3 in annex A.1 [12] | -100.2 - ΔOTAREFSENS | -100.1 - ΔOTAREFSENS |
| 10 | FRC A1-3 in annex A.1 [12] (Note) | -100.2 - ΔOTAREFSENS | -100.1 - ΔOTAREFSENS |
| 15 |  |  |  |
| 20 |  |  |  |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 in [11] mapped to disjoint frequency ranges with a width of 25 Resource Blocks each. |  |  |  |

#### 7.3.5.4	NR Test Requirements

For each measured carrier, the throughput measured in step 9 of subclause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex TS 38.104 [33] A.1 with parameters specified in tables 7.3.5.4-1 to 7.3.5.4-3.

Table 7.3.5.4-1: Wide Area BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -100.4 – ΔOTAREFSENS | -100.3 – ΔOTAREFSENS | -100.1 – ΔOTAREFSENS |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -100.5 – ΔOTAREFSENS | -100.4 – ΔOTAREFSENS | -100.2 – ΔOTAREFSENS |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -97.6 – ΔOTAREFSENS | -97.5 – ΔOTAREFSENS | -97.3 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -94 – ΔOTAREFSENS | -93.9 – ΔOTAREFSENS | -93.7 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -94.3 – ΔOTAREFSENS | -94.2 – ΔOTAREFSENS | -94 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -94.4 – ΔOTAREFSENS | -94.3 – ΔOTAREFSENS | -94.1 – ΔOTAREFSENS |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.4-2: Medium Range BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -95.4 – ΔOTAREFSENS | -95.3 – ΔOTAREFSENS | -95.1 – ΔOTAREFSENS |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -95.5 – ΔOTAREFSENS | -95.4 – ΔOTAREFSENS | -95.2 – ΔOTAREFSENS |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -92.6 – ΔOTAREFSENS | -92.5 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -89 – ΔOTAREFSENS | -88.9 – ΔOTAREFSENS | -88.7 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -89.3 – ΔOTAREFSENS | -89.2 – ΔOTAREFSENS | -89 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -89.4 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS | -89.1 – ΔOTAREFSENS |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.4-3: Local Area BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -92.4 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS | -92.1 – ΔOTAREFSENS |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -92.5 – ΔOTAREFSENS | -92.4 – ΔOTAREFSENS | -92.2 – ΔOTAREFSENS |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -89.6 – ΔOTAREFSENS | -89.5 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -86 – ΔOTAREFSENS | -85.9 – ΔOTAREFSENS | -85.7 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -86.3 – ΔOTAREFSENS | -86.2 – ΔOTAREFSENS | -86 – ΔOTAREFSENS |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -86.4 – ΔOTAREFSENS | -86.3 – ΔOTAREFSENS | -86.1 – ΔOTAREFSENS |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

## 7.4	OTA Dynamic range

### 7.4.1	Definition and applicability

The OTA dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal inside the received channel bandwidth or the capability of receiving high level of wanted signal.

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

### 7.4.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 10.4.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.4.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.4.4.

### 7.4.3	Test purpose

To verify that at the dynamic range of the receiver shall fulfil the specified limit.

### 7.4.4	Method of test

#### 7.4.4.1	Initial conditions

Test environment: normal; see annex G.2.

RF channels to be tested for single carrier:	M; see subclause 4.12.1.

Directions to be tested: OTA REFSENS receiver target reference direction (see table 4.10-2, D11.30).

#### 7.4.4.2	Procedure

1)	Place the AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex D2.2

2)	Align the manufacturer declared coordinate system orientation of the AAS BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the NR BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s)..

5)	Set the test signal mean power so the calibrated radiated power at the AAS BS Antenna Array coordinate system reference point is as specified as follows:

a)	Set the signal generator for the wanted signal to transmit:

-	as specified in table 7.4.5.1-1 for UTRA.

-	as specified in table 7.4.5.2-3 to table 7.4.5.2-5 for E-UTRA.

-	as specified in table 7.4.5.3-3 to table 7.4.5.2-5 for NR.

b)	Set the Signal generator for the AWGN interfering signal at the same frequency as the wanted signal to transmit:

-	as specified in table 7.4.5.1-1 for UTRA.

-	as specified in table 7.4.5.2-3 to table 7.4.5.2-5 for E-UTRA.

-	as specified in table 7.4.5.3-3 to table 7.4.5.23-5 for NR.

6)	Measure:

-	BER according to annex C in 3GPP TS 25.141 [10] for FDD UTRA.

-	Throughput according to annex E in 3GPP TS 36.141 [12] for E-UTRA.

-	Throughput according to TS 38.141-2 [34] for NR.

7)	Repeat for all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

8)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.4.5	Test Requirement

#### 7.4.5.1	UTRA FDD operation

The BER shall not exceed 0,001 for the parameters specified in table 7.3.5.1-1.

Table 7.4.5.1-1: Dynamic range

| Parameter | Level Wide Area BS | Level Medium Range BS | Level Local Area BS | Unit |
| --- | --- | --- | --- | --- |
| Reference measurement channel data rate | 12.2 | 12.2 | 12.2 | kbps |
| Wanted signal mean power | -90.7 - ΔOTAREFSENS | -80.7 - ΔOTAREFSENS | -76.7 - ΔOTAREFSENS | dBm |
| Interfering AWGN signal | -73 - ΔOTAREFSENS | -63 - ΔOTAREFSENS | -59 - ΔOTAREFSENS | dBm/3.84 MHz |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.4.5.2	E-UTRA operation

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A in 36.141 [12]3GPP TS 36.141 [12] with parameters specified in table 7.3.5.3-1 for an AAS BS of Wide Area BS class, in Table7.3.5.3-2 for an AAS BS of Local Area BS class and in table 7.3.5.3-3 for AAS BS of Medium Range BS class.

Table 7.4.5.2-1: AAS BS of Wide Area BS class dynamic range

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in 3GPP TS 36.104 [4], annex A.2 | -76.0 - ΔOTAREFSENS | -88.7 - ΔOTAREFSENS | AWGN |
| 3 | FRC A2-2 in 3GPP TS 36.104 [4], annex A.2 | -72.1 - ΔOTAREFSENS | -84.7 - ΔOTAREFSENS | AWGN |
| 5 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 | -69.9 - ΔOTAREFSENS | -82.5 - ΔOTAREFSENS | AWGN |
| 10 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 (NOTE) | -69.9 - ΔOTAREFSENS | -79.5 - ΔOTAREFSENS | AWGN |
| 15 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 (NOTE) | -69.9 - ΔOTAREFSENS | -77.7 - ΔOTAREFSENS | AWGN |
| 20 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 (NOTE) | -69.9 - ΔOTAREFSENS | -76.4 - ΔOTAREFSENS | AWGN |
| NOTE: The wanted signal mean power is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A2-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |  |

Table 7.3.5.3-2: AAS BS of Local Area BS class  dynamic range

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in 3GPP TS 36.104 [4], annex A.2 | -68.0 - ΔOTAREFSENS | -80.7 - ΔOTAREFSENS | AWGN |
| 3 | FRC A2-2 in 3GPP TS 36.104 [4], annex A.2 | -64.1 - ΔOTAREFSENS | -76.7 - ΔOTAREFSENS | AWGN |
| 5 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 | -61.9 - ΔOTAREFSENS | -74.5 - ΔOTAREFSENS | AWGN |
| 10 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 (NOTE 1) FRC A2-4 in 3GPP TS 36.104 [4], annex A.2 (NOTE 2) | -61.9 - ΔOTAREFSENS -65.0 - ΔOTAREFSENS | -71.5 - ΔOTAREFSENS | AWGN |
| 15 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 (NOTE 1) | -61.9 - ΔOTAREFSENS | -69.7 - ΔOTAREFSENS | AWGN |
| 20 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 (NOTE 1) FRC A2-5 in 3GPP TS 36.104 [4], annex A.2 (NOTE 2) | -61.9 - ΔOTAREFSENS -65.0 - ΔOTAREFSENS | -68.4 - ΔOTAREFSENS | AWGN |
| NOTE 1: The wanted signal mean power is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A2-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. This reference measurement channel is not applied for Band 46. NOTE 2: The wanted signal mean power is the power level of a single instance of the reference measurement channel. This requirement shall be met for each single interlace of FRC A2-4 and A2-5. This reference measurement channel is only applied for Band 46. |  |  |  |  |

Table 7.3.5.3-3: AAS BS of Medium Range BS class dynamic range

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in 3GPP TS 36.104 [4], annex A.2 | -71.0 - ΔOTAREFSENS | -83.7 - ΔOTAREFSENS | AWGN |
| 3 | FRC A2-2 in 3GPP TS 36.104 [4], annex A.2 | -67.1 - ΔOTAREFSENS | -79.7 - ΔOTAREFSENS | AWGN |
| 5 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 | -64.9 - ΔOTAREFSENS | -77.5 - ΔOTAREFSENS | AWGN |
| 10 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 (NOTE 1) FRC A2-4 in 3GPP TS 36.104 [4], annex A.2 (NOTE 2) | -64.9 - ΔOTAREFSENS -68.0 - ΔOTAREFSENS | -74.5 - ΔOTAREFSENS | AWGN |
| 15 | FRC A2-3 in 3GPP TS 36.104 [4], Annex A.2 (NOTE 1) | -64.9 - ΔOTAREFSENS | -72.7 - ΔOTAREFSENS | AWGN |
| 20 | FRC A2-3 in 3GPP TS 36.104 [4], annex A.2 (NOTE 1) FRC A2-5 in 3GPP TS 36.104 [4], annex A.2 (NOTE 2) | -64.9 - ΔOTAREFSENS -68.0 - ΔOTAREFSENS | -71.4 - ΔOTAREFSENS | AWGN |
| NOTE 1: The wanted signal mean power is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A2-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. This reference measurement channel is not applied for Band 46. NOTE 2: The wanted signal mean power is the power level of a single instance of the reference measurement channel. This requirement shall be met for each single interlace of FRC A2-4 and A2-5. This reference measurement channel is only applied for Band 46. |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.4.5.3	NR operation

For each measured carrier, the throughput measured in step 6 of subclause 7.4.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 38.104 [33] annex A.2 with parameters specified in tables 7.4.5.3-1 to 7.4.5.3-3.

Table 7.4.5.3-1: Wide Area BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -82.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -79.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G- FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -76.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G- FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -75.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G- FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -74.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G- FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -73.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G- FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -72.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -71.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G- FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.3-2: Medium Range BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -74.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -72.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G- FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -71.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G- FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -70.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G- FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -69.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G- FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G- FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | --59.2 – ΔOTAREFSENS | --59.2 – ΔOTAREFSENS | --59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -66.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G- FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.3-3: Local Area BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -74.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -71.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -69.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G- FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -68.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G- FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G- FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -66.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G- FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G- FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -63.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G- FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

## 7.5	OTA Adjacent channel selectivity, general blocking, and narrowband blocking

### 7.5.1	Definition and applicability

The adjacent channel selectivity (ACS), general blocking and narrowband blocking characteristics are measures of the receiver unit ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer inside the operating band.

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS : the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS : the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

NOTE:	For Single RAT requirements, the in-band selectivity characteristics is referred to as "adjacent channel selectivity", whereas for the MSR requirements, the corresponding property is referred to as "general blocking" since the adjacent frequency range may not carry a channel addressable from the interfered carrier.

The in-band blocking requirement applies from FUL_low - ΔfOOB to FUL_high + ΔfOOB, excluding the downlink frequency range of the operating band. The ΔfOOB is defined in table 7.5-1.

Table 7.5-1: ΔfOOB offset

| Operating band characteristics | ΔfOOB [MHz] |
| --- | --- |
| 100 MHz ≥ FUL_high – FUL_low | 20 |
| 100 MHz < FUL_high – FUL_low ≤ 900 MHz | 60 |

### 7.5.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 10.5.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.5.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.5.4.

### 7.5.3	Test purpose

The test stresses the receiver unit ability to withstand high-level interference from unwanted signals at specified frequency offsets without undue degradation of its sensitivity.

### 7.5.4	Method of test

#### 7.5.4.1	Initial conditions

Test environment: normal; see annex G.2.

RF channels to be tested for single carrier:	M; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier (MC):	-	MRFBW for single-band RIB(s), see subclause 4.12.1, BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band RIB(s), see subclause 4.12.1.

Directions to be tested:

OTA minSENS receiver target reference direction (see table 4.10-2, D107.

OTA REFSENS conformance test directions (see table 4.10-2, D11.31).).

#### 7.5.4.2	Procedure

##### 7.5.4.2.1	General procedure

The general procedure steps apply to the procedures for all the RATs.

1)	Place the AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex D1.1.

2)	Align the manufacturer declared coordinate system orientation of the AAS BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the NR BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Set the test signal mean power so the calibrated radiated power at the AAS BS Antenna Array coordinate system reference point is as specified as follows:

a)	Set the signal generator for the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to transmit:

-	For E-UTRA see clause A.1 in TS 36.141 [12].

-	For UTRA FDD see clause A.2 in TS 25.141 [10].

-	For NR see clause A.1 in TS 38.141-2 [34].

##### 7.5.4.2.2	MSR operation

###### 7.5.4.2.2.1	Procedure for general blocking

1)	Adjust the signal generators to the type of interfering signal, levels and the frequency offsets as specified in table 7.5.5.1.1-1.

2)	The interfering signal shall be swept with a step size of 1 MHz starting from the minimum offset to the channel edges of the wanted signals as specified in table 7.5.5.1.1-1.

3)	Measure the performance of the wanted signal as defined in subclause 7.5.5.1, for the relevant carriers specified by the test configuration in subclause 4.11.

4)	Repeat for all the specified measurement directions.

5) Repeat for all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

6)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 7.5.4.2.2.2	Procedure for narrowband blocking

1)	Adjust the signal generators to the type of interfering signal, levels and the frequency offsets as specified in table 7.5.5.1.2-1.

2)	Set-up and sweep the interfering RB centre frequency offset to the channel edge of the wanted signal according to table 7.5.5.1.2-1.

3)	Measure the performance of the wanted signal at the receiver under test, as defined in subclause 7.5.5.1, for the relevant carriers specified by the test configuration in subclause 4.11.

4)	Repeat for all the specified measurement directions.

5) Repeat for all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

6)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 7.5.4.2.2.3	Procedure for additional BC3 blocking requirement

1)	Adjust the signal generators to the type of interfering signal, levels and the frequency offsets as specified in table 7.5.5.1.3-1.

2)	Measure the performance of the wanted signal at the receiver  under test, as defined in subclause 7.5.5, for the relevant carriers specified by the test configuration in subclause 4.8.

3)	Repeat for all the specified measurement directions and all supported polarizations.

##### 7.5.4.2.3	Single RAT UTRA FDD operation

1)	Generate the wanted signal and adjust the ATT1 to set the input level to the level specified in table 7.5.5.2-1 For a RIB supporting multi-carrier operation, generate the wanted signal according to the applicable test configuration (see clause 4.11) using applicable reference measurement channel to the RIB under test. Power settings are specified in table 7.5.5.2-1.

2)	Set-up the interfering signal at the adjacent channel frequency and adjust the ATT2 to obtain the specified level of interfering signal at the AAS BS input defined in table 7.5.5.2-1. Note that the interfering signal shall have an ACLR of at least 63 dB in order to eliminate the impact of interfering signal adjacent channel leakage power on the ACS measurement.

3)	Measure the BER of the wanted signal at the receiver under test.

4)	Repeat for all the specified measurement directions and all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

5)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.5.4.2.4	Single RAT E-UTRA operation

###### 7.4.4.2.4.1	Procedure for adjacent channel selectivity

1)	Generate the wanted signal using the applicable test configuration specified in subclause 5.3.4 and adjust the input level to the level specified in table 7.5.5.3-1 for the appropriate BS class.

2)	Set-up the interfering signal at the adjacent channel frequency and adjust the interfering signal level to the level defined in table 7.5.5.3-1 for the appropriate BS class.

3)	Measure the throughput according to annex E in 36.141 [12]3GPP TS 36.141 [12], for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclause 5.3.4.

4)	Repeat for all the specified measurement directions and all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

5)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 7.5.4.2.4.2	Procedure for narrow-band blocking

1)	Configure the beam peak direction of the AAS BS according to declared reference beam direction pair for the appropriate beam identifier.

2)	For RIB operating E-UTRA FDD declared to be capable of single carrier operation only in the operating band, set the AAS BS to transmit according to subclause 4.12.2 at manufacturers declared rated output power PRated,c,TABC.

For a RIB operating E-UTRA FDD declared to be capable of multi-carrier and/or CA operation in the operating band, set the ASA BS to transmit according to subclause 4.12.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 5.3.4.

3)	Generate the wanted signal using the applicable test configuration specified in subclause 5.3.4 and adjust the input level to the level specified in table 7.5.5.3-1.

4)	Adjust the interfering signal level to the level defined in table 7.5.5.3-1. Set-up and sweep the interfering RB centre frequency offset to the channel edge of the wanted signal according to table 7.5.5.3-2.

5)	Measure the throughput according to annex E in 36.141 [12]3GPP TS 36.141 [12], for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclause 5.3.4.

6)	Repeat for all the specified measurement directions and all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

7)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.5.5	Test Requirement

#### 7.5.5.1	MSR operation

##### 7.5.5.1.1	General blocking test requirement

For the general blocking requirement, the interfering signal shall be a UTRA FDD signal as specified in clause A.1 in 3GPP TS 25.141 [10].

For RIBs supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 15 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the RIB, using the parameters in tables 7.5.5.1.1-1 and 7.5.5.1.1-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in subclause 7.2.5.4.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in subclause 7.2.5.2.

For multi-band RIBs, the requirement applies according to table 7.5.5.1.1-1 for the in-band blocking frequency ranges of each supported operating band.

Table 7.5.5.1.1-1: General blocking requirement

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] (NOTE 1) | Centre Frequency of Interfering Signal | Interfering signal centre frequency minimum frequency offset from the Base Station RF Bandwidth edge or edge of sub-block inside a gap [MHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | -40 + y - ΔOTAREFSENS (NOTE 7, 9) | EISREFSENS + x dB (NOTE 2, 5, 9) | FUL_low - ΔfOOB to FUL_high + ΔfOOB | ±7.5 |
|  | -40 + y - ΔminSENS (NOTE 7, 10) | EISminSENS + x dB (NOTE 2, 5, 10) |  |  |
| Medium Range BS | -35 + y - ΔOTAREFSENS (NOTE 7, 9) | EISREFSENS + x dB (NOTE 3, 5, 9) |  |  |
|  | -35 + y - ΔminSENS (NOTE 7, 10) | EISminSENS + x dB (NOTE 3, 5, 10) |  |  |
| Local Area BS | -30 + y - ΔOTAREFSENS (NOTE 7, 9) | EISREFSENS + x dB (NOTE 4, 5, 9) |  |  |
|  | -30 + y - ΔminSENS (NOTE 7, 10) | EISminSENS + x dB (NOTE 4, 5, 10) |  |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: For WA BS that does not support NR, “x” is equal to 6 in case of E-UTRA or UTRA wanted signals and equal to 3 in case of GSM/EDGE wanted signal. NOTE 3: For MR BS that does not support NR, “x” is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA wanted signal and 3 in case of GSM/EDGE wanted signal. NOTE 4: For LA BS that does not support NR, “x” is equal to 11 in case of E-UTRA wanted signal, 6 in case of UTRA wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 5: For a BS that supports NR but does not support UTRA, x is equal to 6. NOTE 6: For a BS capable of multi-band operation, “x” in Note 2, 3, 4, 5 applies in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in an adjacent or overlapping band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, “x” is equal to 1.4 dB. NOTE 7: For a BS that not supporting NR, “y” is equal to zero for all BS classes. For a BS that supports NR but does not support UTRA, “y” is equal to -3 for the WA and MR BS class and -5 for the LA BS class. NOTE 8: The downlink frequency range of an FDD operating band is excluded from the general blocking requirement. NOTE 9: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 10: This test requirement is only applied in the OTA minSENS receiver target reference direction. |  |  |  |  |

Table 7.5.5.1.1-2: Void

NOTE:	The requirement in tables 7.5.5.1.1-1 and 7.5.5.1.1-2 assumes that two operating bands, where the downlink operating band (see table 4.4-1 and table 4.4-2 in 3GPP TS 37.141 [13].) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

##### 7.5.5.1.2	General narrowband blocking test requirement

For the narrowband blocking requirement, the interfering signal shall be an E-UTRA 1RB signal as specified in clause A.3 in 3GPP TS 37.141 [13].

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 3 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least 3 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the RIB, using the parameters in table 7.4.5.1.2-1 the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in subclause 7.2.5.4.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in subclause 7.2.5.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined for BS type 1-O in TS 38.104 [33], subclause 10.3.2

Table 7.5.5.1.2-1: Narrowband blocking requirement

| Base Station Type | RAT of the carrier | Wanted signal mean power [dBm] (NOTE 1,2) | Interfering signal mean power [dBm] | Interfering RB (NOTE 3) centre frequency offset from the AAS Base Station RF Bandwidth edge or edge of sub-block inside a gap [kHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | E-UTRA, NR, UTRA | EISREFSENS + x dB (NOTE 6) | -49 - ΔOTAREFSENS (NOTE 6) | ±(240 +m 180), m=0, 1, 2, 3, 4, 9, 14 (Note 4) ±(550 +m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 100 (Note 5) |
|  |  | EISminSENS + x dB (NOTE 7) | -49 – ΔminSENS (NOTE 7) |  |
| Medium Range BS |  | EISREFSENS + x dB (NOTE 6) | -44 - ΔOTAREFSENS (NOTE 6) |  |
|  |  | EISminSENS + x dB (NOTE 7) | -44 – ΔminSENS (NOTE 7) |  |
| Local Area BS |  | EISREFSENS + x dB (NOTE 6) | -41 - ΔOTAREFSENS (NOTE 6) |  |
|  |  | EISminSENS + x dB (NOTE 7) | -41 – ΔminSENS (NOTE 7) |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: "x" is equal to 6 dB in case of E-UTRA or UTRA wanted signals. NOTE 3: Interfering signal (E-UTRA 3 MHz) consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the AAS Base Station RF Bandwidth edge. NOTE 4: Applicable for channel bandwidths equal to or below 20 MHz. NOTE 5: Applicable for channel bandwidths above 20MHz. NOTE 6: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 7: This test requirement is only applied in the OTA minSENS receiver target reference direction. |  |  |  |  |

##### 7.5.5.1.3	Additional BC3 blocking test requirement

The interfering signal is a 1,28Mcps UTRA TDD modulated signal as specified in clause A.2 in 3GPP TS 37.141 [13].

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 4.8MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the RIB, using the parameters in table 7.4.5.1.3-1, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in subclause 7.2.5.3.

Table 7.5.5.1.3-1: Additional blocking requirement for Band Category 3

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] (NOTE) | Interfering signal centre frequency minimum frequency offset from the Base Station RF Bandwidth edge [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| 33 - 40 | (FUL_low - 20) | to | (FUL_high + 20) | -40 - ΔOTAREFSENS (NOTE 2) | EISREFSENS + 6 dB (NOTE 2) | ±2,4 |
|  |  |  |  | -40 – ΔminSENS (NOTE 3) | EISminSENS + 6 dB (NOTE 3) |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 3: This test requirement is only applied in the OTA minSENS receiver target reference direction. |  |  |  |  |  |  |

#### 7.5.5.2	Single RAT UTRA FDD operation

For each measured carrier, the BER shall not exceed 0,001 for the parameters specified in table 7.5.5.2-1.

For multi-carrier RIB the ACS requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the lower/upper Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 5 MHz. The interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -2.5 MHz/+2.5 MHz, respectively.

For a multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 5 MHz. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -2.5 MHz/+2.5 MHz, respectively.

Table 7.5.5.2-1: Adjacent channel selectivity

| Parameter | Level Wide Area BS | Level Medium Range BS | Level Local Area / Home BS | Unit |
| --- | --- | --- | --- | --- |
| Data rate | 12.2 | 12.2 | 12.2 | kbps |
| Wanted signal mean power | -115 – ΔminSENS | -105 – ΔminSENS | -101 – ΔminSENS | dBm |
| Interfering signal mean power | -52 – ΔminSENS | -42 – ΔminSENS | -38 – ΔminSENS | dBm |
| Fuw offset (Modulated) | ±5 | ±5 | ±5 | MHz |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.5.5.3	Single RAT E-UTRA operation

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For E-UTRA Wide Area BS, the wanted and the interfering signal coupled to the BS antenna input are specified in tables 7.5.5.3-1 and 7.5.5.3-2 for narrowband blocking and 7.5.5.3-3 for ACS. The reference measurement channel for the wanted signal is identified in table 10.3.4-1 for each channel bandwidth and further specified in 3GPP TS 36.104 [4] Annex A.

For E-UTRA Medium Range BS, the wanted and the interfering signal coupled to the BS antenna input are specified in tables 7.5.5.3-1 and 7.5.5.3-2 for narrowband blocking and in table 7.5.5.3-5 for ACS. Narrowband blocking requirements are not applied for Band 46. The reference measurement channel for the wanted signal is identified in table 10.3.4-3 for each channel bandwidth and further specified in 3GPP TS 36.104 [4] Annex A.

For E-UTRA Local Area BS, the wanted and the interfering signal coupled to the BS antenna input are specified in tables 7.5.5.3-1 and 7.5.5.3-2 for narrowband blocking and 7.5.5.3-4 for ACS. Narrowband blocking requirements are not applied for Band 46. The reference measurement channel for the wanted signal is identified in table 10.3.4-2 for each channel bandwidth and further specified in 3GPP TS 36.104 [4] Annex A.

For narrowband blocking the OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

For ACS the OTA levels are applied referenced to ΔminSENS.

The ACS and narrowband blocking requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the E-UTRA interfering signal in tables 7.5.5.3-3, 7.5.5.3-4 and 7.5.5.3-5. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap

For multi-band RIBs, the ACS requirement applies in addition inside any Inter RF Bandwidth gap at the RIB, in case the gap size is at least as wide as the E-UTRA interfering signal in tables 7.5.5.3-3, 7.5.5.3-4 and 7.5.5.3-5. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For a RIBs operating in non-contiguous spectrum within any operating band, the narrowband blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.5.5.3-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs,, the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the E-UTRA interfering signal in table 7.5.5.3-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.5.5.3-1: Narrowband blocking requirement

|  | Wanted signal mean power [dBm] (NOTE) | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB (NOTE 2) | -49 – ΔOTAREFSENS (NOTE 2) | See table 10.5.4.2-2 |
|  | EISminSENS + 6 dB (NOTE 3) | -49 – ΔminSENS (NOTE 3) |  |
| Medium Range BS | EISREFSENS + 6 dB (NOTE 2) | -44 – ΔOTAREFSENS (NOTE 2) | See table 10.5.4.2-2 |
|  | EISminSENS + 6 dB (NOTE 3) | -44 – ΔminSENS (NOTE 3) |  |
| Local Area BS | EISREFSENS + 6 dB (NOTE 2) | -41 – ΔOTAREFSENS (NOTE 2) | See table 10.5.4.2-2 |
|  | EISminSENS + 6 dB (NOTE 3) | -41 – ΔminSENS (NOTE 3) |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 3: This test requirement is only applied in the OTA minSENS receiver target reference direction. |  |  |  |

Table 7.5.5.3-2: Interfering signal for Narrowband blocking requirement

| E-UTRA channel BW of the lowest/highest carrier received [MHz] |  | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwdith edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- |
| 1.4 |  | ±(252.5+m*180), m=0, 1, 2, 3, 4, 5 | 1.4 MHz E-UTRA signal, 1 RB (NOTE) |
| 3 |  | ±(247.5+m*180), m=0, 1, 2, 3, 4, 7, 10, 13 | 3 MHz E-UTRA signal, 1 RB (NOTE) |
| 5 |  | ±(342.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB (NOTE) |
| 10 |  | ±(347.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB (NOTE) |
| 15 |  | ±(352.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB (NOTE) |
| 20 |  | ±(342.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB (NOTE) |
| NOTE: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |

Table 7.5.5.3-3: Adjacent channel selectivity for Wide Area BS

| E-UTRA channel bandwidth of the lowesthighest carrier received [MHz] | Wanted signal mean power [dBm] (NOTE) | Interfering signal mean power [dBm] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | EISminSENS + 11 dB | -52 – ΔminSENS | ±0.7025 | 1.4MHz E-UTRA signal |
| 3 | EISminSENS + 8 dB | -52 – ΔminSENS | ±1.5075 | 3MHz E-UTRA signal |
| 5 | EISminSENS + 6 dB | -52 – ΔminSENS | ±2.5025 | 5MHz E-UTRA signal |
| 10 | EISminSENS + 6 dB | -52 – ΔminSENS | ±2.5075 | 5MHz E-UTRA signal |
| 15 | EISminSENS + 6 dB | -52 – ΔminSENS | ±2.5125 | 5MHz E-UTRA signal |
| 20 | EISminSENS + 6 dB | -52 – ΔminSENS | ±2.5025 | 5MHz E-UTRA signal |
| NOTE: EISminSENS depends on the channel bandwidth as specified see subclause 7.2. |  |  |  |  |

Table 7.5.5.3-4: Adjacent channel selectivity for Local Area BS

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] (NOTE 1) | Interfering signal mean power [dBm] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | EISminSENS + 11 dB | -44 – ΔminSENS | ±0.7025 | 1.4MHz E-UTRA signal |
| 3 | EISminSENS + 8 dB | -44 – ΔminSENS | ±1.5075 | 3MHz E-UTRA signal |
| 5 | EISminSENS + 6 dB | -44 – ΔminSENS | ±2.5025 | 5MHz E-UTRA signal |
| 10 | EISminSENS + 6 dB | -44 – ΔminSENS | ±2.5075 ±10.0175 | 5MHz E-UTRA signal (NOTE 2) 20 MHz E-UTRA signal (NOTE 3) |
| 15 | EISminSENS + 6 dB | -44 – ΔminSENS | ±2.5125 | 5MHz E-UTRA signal |
| 20 | EISminSENS + 6 dB | -44 – ΔminSENS | ±2.5025 ±10.0175 | 5MHz E-UTRA signal (NOTE 2) 20 MHz E-UTRA signal (NOTE 3) |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified see subclause 7.2. NOTE 2: This type of interfering signal is not applied for Band 46. NOTE 3: This type of interfering signal is only applied for Band 46. |  |  |  |  |

Table 7.5.5.3-5: Adjacent channel selectivity for Medium Range BS

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] (NOTE 1) | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | EISminSENS + 11 dB | -47 – ΔminSENS | ±0.7025 | 1.4MHz E-UTRA signal |
| 3 | EISminSENS + 8 dB | -47 – ΔminSENS | ±1.5075 | 3MHz E-UTRA signal |
| 5 | EISminSENS + 6 dB | -47 – ΔminSENS | ±2.5025 | 5MHz E-UTRA signal |
| 10 | EISminSENS + 6 dB | -47 – ΔminSENS | ±2.5075 ±10.0175 | 5MHz E-UTRA signal (NOTE 2) 20 MHz E-UTRA signal (NOTE 3) |
| 15 | EISminSENS + 6 dB | -47 – ΔminSENS | ±2.5125 | 5MHz E-UTRA signal |
| 20 | EISminSENS + 6 dB | -47 – ΔminSENS | ±2.5025 ±10.0175 | 5MHz E-UTRA signal (NOTE 2) 20 MHz E-UTRA signa (NOTE 3) |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified see subclause 7.2. NOTE 2: This type of interfering signal is not applied for Band 46. NOTE 3: This type of interfering signal is only applied for Band 46. |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 7.6	OTA Blocking

### 7.6.1	General

The blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

The requirement applies at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted signal applies to all supported polarizations, under the assumption of polarization match. The interferer shall be polarization matched for in-band frequencies and the polarization maintained for out-of-band frequencies.

### 7.6.2	General Requirement

#### 7.6.2.1	Definition and applicability

#### 7.6.2.2	Minimum Requirement

The minimum requirement for AAS BS in MSR operation is defined in TS 37.105 [6], subclause 10.6.2.

The minimum requirement for AAS BS in single RAT UTRA operation is defined in TS 37.105 [6], subclause 10.6.3.

The minimum requirement for AAS BS in single RAT E-UTRA operation is defined in TS 37.105 [6], subclause 10.6.4.

#### 7.6.2.3	Test purpose

The test stresses the ability of the receiver unit associated with the RIB under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

#### 7.6.2.4	Method of test

##### 7.6.2.4.1	Initial conditions

Test environment:

-	normal; see annex G.2.

RF channels to be tested for single carrier (SC):

-	M; see subclause 4.12.1

Base Station RF Bandwidth positions to be tested for multi-carrier (MC):

-	MRFBW for single-band RIB, see subclause 4.12.1, BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band RIB, see subclause 4.12.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, blocking testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, blocking testing below the lowest operating band may be omitted.

Directions to be tested:

-	OTA REFSENS receiver target reference direction (see table 4.10-2, D11.30).

##### 7.6.2.4.2	Procedure

###### 7.6.2.4.2.1	General procedure

1)	Place AAS BS and the test antenna(s) according to Annex D.2.4.

2)	Place test antenna(s) in reference direction (see table 4.10-1, D10.9) at far-field distance, aligned in all supported polarizations with the AAS BS as depicted in Annex D.2.4.

3)	Connect test antenna(s) to the measurement equipment as shown in Annex D.2.4.

4)	The test antenna(s) shall be dual (or single) polarized covering the same frequency ranges as the AAS BS and the blocking frequencies. If the test antenna does not cover both the wanted and interfering signal frequencies, separate test antennas for the wanted and interfering signal are required.

5)	The OTA blocking interferer is injected into the test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

6)	The AAS BS receives the wanted signal and the interferer signal for supported polarization(s), in the reference direction (see table 4.10-1, D10.9) from the test antenna(s).

###### 7.6.2.4.2.2	MSR operation

1)	Generate the wanted signal from the test antenna, according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the RIB under test as follows:

-	For E-UTRA see clause A.1 in 3GPP TS 36.141 [12].

-	For UTRA FDD see clause A.2 in 3GPP TS 25.141 [10].

-	For NR see clause A.1 in 3GPP TS 38.141-2 [34].

2)	Set the transmitter unit(s) associated with the RIB under test to transmit in reference direction (see table 4.10-1, D10.9) with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

The transmitter unit(s) associated with the RIB under test may be turned off for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

3)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.1.1-1 and, when applicable, for co-location test requirements in table 7.6.5.1.2-1.

The distance between the test object and test antenna injecting the interferer signal is adjusted when necessary to ensure specified interferer signal level to be received.

4)	The CW interfering signal shall be swept with a step size of 1 MHz within the specified range.

5)	Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the subclause 7.6.5, for the relevant carriers specified by the test configuration in subclause 4.11.

6)	Repeat for all supported polarizations.

In addition, for multi-band RIB, the following steps shall apply:

7)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 7.6.2.4.2.3	Single RAT UTRA FDD operation

1)	Generate the wanted signal, from the test antenna, according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the RIB under test as shown in subclause A.2.1 in 3GPP TS 25.141 [10].

2)	Set the transmitter unit(s) associated with the RIB under test to transmit in reference direction (see table 4.10-1, D10.9) with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

The transmitter unit(s) associated with the RIB under test may be turned off for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

3)	Adjust the signal generators to the type of interfering signals and the frequency offsets as specified in tables 7.6.5.2.1-1 to 7.6.5.2.1-2 (in-band and narrowband blocking test requirements) and 7.6.5.2.2-1 (co-location test requirements). Note that the GMSK modulated interfering signal shall have an ACLR of at least 72 dB in order to eliminate the impact of interfering signal adjacent channel leakage power on the blocking characteristics measurement. For the tests defined in tables 7.6.5.2.1-1 to 7.6.5.2.1-2, the interfering signal shall be at a frequency offset Fuw from the assigned channel frequency of the wanted signal which is given by:

Fuw =  (n x 1 MHz),

where n shall be increased in integer steps from n = 10 up to such a value that the centre frequency of the interfering signal covers the range from 1 MHz to 12.75 GHz.

4)	Measure the BER of the wanted signal at the receiver unit associated with the RIB under test.

5)	Repeat for all supported polarizations.

In addition, for multi-band RIB, the following steps shall apply:

6)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 7.6.2.4.2.4	Single RAT E-UTRA operation

1)	Generate the wanted signal from the test antenna, according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the RIB under test as shown in clause A.1 in 3GPP TS 36.141 [6].

2)	Set the transmitter unit(s) associated with the RIB under test to transmit in reference direction (see table 4.10-1, D10.9) with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

The transmitter unit(s) associated with the RIB under test may be turned off for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

3)	Adjust the signal generators to the type of interfering signals and the frequency offsets as specified in tables 7.6.5.3.1-1 to 7.6.5.3.1-2 (in-band blocking test requirements), 7.6.5.3.2-1 (co-location test requirements).

4)	The CW interfering signal shall be swept with a step size of 1 MHz within the specified range.

5)	Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the subclause 7.6.5, for the relevant carriers specified by the test configuration in subclause 4.11.

6)	Repeat for all supported polarizations.

In addition, for multi-band RIB, the following steps shall apply:

7)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 7.6.2.5	Test Requirement

##### 7.6.2.5.1	MSR operation

The OTA interfering signal RMS field-strength shall be set to 0.36 V/m at the base station RIB per polarization.

NOTE:	The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m; for example, 0.36 V/m is equivalent to 36 dBm at fixed distance of 30 m.

For a wanted and an interfering signal specified at the RIB using the parameters in table 7.6.2.5.1-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [9], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in 3GPP TS 25.104 [2], subclause 7.2.1.

-	For any NR carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 38.104 [33], subclause 7.2.1.

For multi-band RIB, the requirement applies for each supported operating band. The in-band blocking frequency ranges of all supported operating bands according to table 7.6.2.5.1-1 shall be excluded from the requirement.

The OTA blocking requirement applies from 30 MHz to FUL_low - ΔfOOB and from FUL_high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band. ΔfOOB is defined in subclause 7.5.1.

Table 7.6.2.5.1-1: Blocking performance requirement

| Wanted signal mean power (dBm) | Interfering signal RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- |
| EISminSENS + 6 dB (Note 1) | 0.36 V/m | CW carrier |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified in TS 38.104 [33], subclause 10.2.1. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m; for example, 0.36 V/m is equivalent to 36 dBm at fixed distance of 30 m. |  |  |

##### 7.6.2.5.2	Single RAT UTRA FDD operation

In addition to the following in-band and narrowband requirements, the general minimum requirements relating to out of band blocking defined for MSR in subclause 7.6.2.5.1-1 shall also be applied for single RAT UTRA operation.

The minimum requirement for in-band blocking and narrowband blocking UTRA operation is defined below:

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges applicable to each RIB.

For RIB supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap and is equal to -7.5MHz/+7.5MHz, respectively.

For a RIB supporting operation in non-contiguous spectrum the narrowband blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 400kHz or 600kHz, depending on the operating band. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap and is equal to -200kHz/+200kHz or -300kHz/+300kHz, respectively.

For multi-band RIBs the requirement in the in-band blocking frequency range applies for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case Inter RF Bandwidth gap size is at least 15MHz. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -7.5MHz/+7.5MHz, respectively.

For multi-band RIBs the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 400kHz or 600kHz, depending on the operating band. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -200kHz/+200kHz or -300kHz/+300kHz, respectively.

For the wanted and interfering signal at the RIB, using the parameters in tables 7.6.2.5.2-1 and 7.6.2.5.2-2, the following requirements shall be met:

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in 3GPP TS 25.104 [2], subclause 7.2.1.

The OTA levels are applied referenced to two antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 7.6.2.5.2-1: In-band blocking requirement for single RAT UTRA AAS BS

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Wide Area BS | -40 – ΔOTAREFSENS | EISREFSENS + 6 dB | ±10MHz | WCDMA signal (NOTE 1) |
|  | -40 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Medium Range BS | -35 – ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -35 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Local Area BS | -30 – ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -30 – ΔminSENS | EISminSENS + 6 dB |  |  |
| NOTE 1: The characteristics of the W-CDMA interference signal are specified in Annex C of TS 25.104 [2]. NOTE 2: For multi-band RIBs, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to -119.6 – ΔOTAREFSENS dBm or -119.6 – ΔminSENS dBm as appropriate. |  |  |  |  |

NOTE:	Table 7.6.2.5.2-1 assumes that two operating bands, where the downlink frequencies (see subclause 4.6) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.2.5.2-2: Blocking performance requirement (narrowband) for single RAT UTRA AAS BS

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Wide Area BS | -47 – ΔOTAREFSENS | EISREFSENS + 6 dB | ±2.7 MHz (NOTE 2) ±2.8 MHz (NOTE 3) | GMSK modulated (NOTE 1) |
|  | -47 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Medium Range BS | -42 – ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -42 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Local Area BS | -37 – ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -37 – ΔminSENS | EISminSENS + 6 dB |  |  |
| NOTE 1: GMSK modulation as defined in TS 45.004 [32]. NOTE 2: applies for bands II,IV,V,VIII,X,XII,XIV,XXV,XXVI NOTE 3: applies for bands III,VIII |  |  |  |  |

##### 7.6.2.5.3	Single RAT E-UTRA operation

In addition to the following in-band and narrowband requirements, the general minimum requirements relating to out of band blocking defined for MSR in subclause 7.6.2.5.1-1 shall also be applied for single RAT E-UTRA operation.

The minimum requirement for in-band blocking E-UTRA operation is defined below:

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges applicable to each RIB.

For RIB supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 15 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal at the RIB, using the parameters in tables 7.6.2.5.3-1 and 7.6.2.5.3-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [4], subclause 7.2.1.

The OTA levels are applied referenced to two antenna gain offsets ΔOTAREFSENS and ΔminSENS.

For multi-band RIBs, the requirement applies according to table 7.6.5.3.1-1 for the in-band blocking frequency ranges of each supported operating band.

Table 7.6.2.5.3-1: In-band blocking requirement for single RAT E-UTRA

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] (NOTE 1,2) | Type of Interfering Signal | Interfering signal centre frequency minimum frequency offset from the Base Station RF Bandwidth edge or edge of sub-block inside a gap [MHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | -43 – ΔOTAREFSENS | EISREFSENS + 6 dB | See table 7.6.2.5.3-2 | See table 7.6.2.5.3-2 |
|  | -43 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Medium Range BS | -38 – ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -38 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Local Area BS | -35 – ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -35 – ΔminSENS | EISminSENS + 6 dB |  |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 10.3 and 10.2 in TS 37.105 [6]. NOTE 2: For multi-band RIBs, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to EISREFSENS +1.4 dB or EISminSENS +1.4 dB as appropriate. |  |  |  |  |

Table 7.6.2.5.3-2: Interfering signals for single RAT E-UTRA in-band blocking performance requirement

| E-UTRA channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | 1.4 MHz E-UTRA signal |
| 3 | ±4.5 | 3 MHz E-UTRA signal |
| 5 | ±7.5 | 5 MHz E-UTRA signal |
| 10 | ±7.5 | 5 MHz E-UTRA signal |
| 15 | ±7.5 | 5 MHz E-UTRA signal |
| 20 | ±7.5 | 5 MHz E-UTRA signal |
| 20 | ±30 | 20 MHz E-UTRA signal |

### 7.6.3	Co-location Requirement

#### 7.6.3.1	Definition and applicability

This additional blocking requirement may be applied for the protection of AAS BS receivers when E-UTRA BS, UTRA BS, NR BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with an AAS BS.

The requirement is a co-location requirement. The interferer power levels are specified at the CLTA conducted input(s).

Interfering signal shall be applied to the CLTA. The interfering power is specified per polarization.

#### 7.6.3.2	Minimum Requirement

The minimum requirement for AAS BS in MSR operation is defined in TS 37.105 [6], subclause 10.6.2.

The minimum requirement for AAS BS in single RAT UTRA operation is defined in TS 37.105 [6], subclause 10.6.3.

The minimum requirement for AAS BS in single RAT E-UTRA operation is defined in TS 37.105 [6], subclause 10.6.4.

#### 7.6.3.3	Test purpose

The test stresses the ability of the receiver unit associated with the RIB under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

#### 7.6.3.4	Method of test

##### 7.6.3.4.1	Initial conditions

Initial conditions accroding to sub-clause 7.6.2.4.1.

##### 7.6.3.4.2	Procedure

1)	Place AAS BS and CLTA as specified in sub-clause 4.15, at the distance d=0.1m.

2)	Several CLTAs are required to cover the whole co-location blocking frequency ranges.

3)	Place test antenna in reference direction (see table 4.10-1, D10.9) at far-field distance, aligned in all supported polarizations (single or dual) with the AAS BS as depicted in Annex D.2.4.

4)	The test antenna shall be dual (or single) polarized, with the same frequency range as the AAS BS, for the co-location blocking test case.

5)	Connect test antenna and CLTA to the measurement equipment as depicted in Annex D.2.4.

6)	The AAS BS receives the wanted signal in all supported polarizations (single or dual), in the reference direction (see table 4.10-1, D10.9) from the test antenna.

7)	The OTA co-location blocking interferer is injected into the CLTA. The CLTA is fed with the co-location blocking interferer.

###### 7.6.3.4.2.1	MSR operation

See sub-clause 7.6.2.4.2.2.

###### 7.6.3.4.2.2	Single RAT UTRA FDD operation

See sub-clause 7.6.2.4.2.3.

###### 7.6.3.4.2.3	Single RAT E-UTRA operation

See sub-clause 7.6.2.4.2.4.

#### 7.6.3.5	Test Requirement

##### 7.6.3.5.1	MSR operation

This additional blocking requirement may be applied for the protection of AAS BS receivers when E-UTRA BS, UTRA BS, NR BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with an AAS BS.

The requirement is a co-location requirement, the interferer power levels specified at the CLTA conducted input(s).

The requirement is valid over minSENS RoAoA.

Interfering signal shall be applied to the CLTA. The interfering power is specified per polarization.

When the wanted and an interfering signal using the parameters in table 7.6.3.5.1-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [9], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in 3GPP TS 25.104 [2], subclause 7.2.1.

-	For any NR carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 38.104 [33], subclause 7.2.1.

Table 7.6.3.5.1-1: Blocking requirement for co-location with BS in other frequency bands

| Type of co-located BS | Centre Frequency of Interfering Signal [MHz] | Interfering Signal mean power for WA BS [dBm] | Interfering Signal mean power for MR BS [dBm] | Interfering Signal mean power for LA BS [dBm] | Wanted Signal mean power [dBm] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| GSM900 | 921 – 960 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| DCS1800 | 1805 - 1880 (NOTE 4) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| PCS1900 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 – 2170 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 - 1880 (NOTE 4) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 – 2690 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 – 960 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 – 746 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIIII or E-UTRA Band 13 | 746 – 756 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 – 768 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 17 | 734 – 746 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 18 | 860 – 875 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIX or E-UTRA Band 19 | 875 – 890 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 – 821 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 - 3 590 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 23 | 2180 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 24 | 1525 – 1559 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 27 | 852 – 869 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 28 or NR band n28 | 758 – 803 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 29 | 717 – 728 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 30 | 2350 – 2360 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 31 | 462.5 - 467.5 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 (NOTE-5) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 33 | 1900 – 1920 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 34 or NR band n34 | 2010 – 2025 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 35 | 1850 – 1910 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 36 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band c) or E-UTRA TDD Band 37 | 1910 – 1930 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 – 2620 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 – 2400 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 41 or NR band n41 | 2496 – 2690 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 42 | 3400 – 3600 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 43 | 3600 – 3800 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 44 | 703 – 803 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 45 | 1447 - 1467 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 46 | 5150 - 5925 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 48 | 3550 – 3700 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 49 | 3550 – 3700 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 51 or or NR band n51 | 1427– 1432 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 65 | 2110 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 66 or or NR band n66 | 2110 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 67 | 738 - 758 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 68 | 753 - 783 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 69 | 2570-2620 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 70 or or NR band n70 | 1995 - 2020 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 71 or or NR band n71 | 617 - 652 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 72 | 461 - 466 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 73 | 460 - 465 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 74 or NR band n74 | 1475 - 1518 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 75 or or NR band n75 | 1432 - 1517 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 76 or or NR band n76 | 1427 - 1432 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n77 | 3300 - 4200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n78 | 3300 - 3800 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n79 | 4400 - 5000 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NOTE 1: EISminSENS depends on the RAT, the BS class and on the channel bandwidth, see subclauses 10.3 and 10.2 in TS 37.105; "x" is equal to 6 dB in case of E-UTRA or UTRA wanted signals. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the ΔfOOB immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768 - 797 MHz. NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD or NR TDD with E-UTRA FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in 3GPP TR 25.942 [31]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805 - 1850 MHz. NOTE 5: For an AAS BS operating in band 11 or 21, this requirement applies for interfering signal within the frequency range 1475.9 - 1495.9 MHz. |  |  |  |  |  |  |  |

##### 7.6.3.5.2	Single RAT UTRA FDD operation

This additional blocking requirement may be applied for the protection of AAS BS receivers when E-UTRA BS, UTRA BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with an AAS BS.

The requirement is a co-location requirement, the interferer power levels specified at the CLTA conducted input(s).

The requirement is valid over minSENS RoAoA.

Interfering signal shall be applied to the CLTA. The interfering power is specified per polarization.

When the wanted and an interfering signal using the parameters in table 7.6.3.5.1-1 for co-location with UTRA or E-UTRA systems and table 7.6.3.5.2-1 for co-location with GSM systems, the following requirements shall be met:

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in 3GPP TS 25.104 [2], subclause 7.2.1.

Table 7.6.3.5.2-1: UTRA additional OTA blocking requirement for co-location with BS in other frequency bands

| Type of co-located BS | Centre Frequency of Interfering Signal [MHz] | Interfering Signal mean power for WA BS [dBm] | Interfering Signal mean power for MR BS [dBm] | Interfering Signal mean power for LA BS [dBm] | Wanted Signal mean power [dBm] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| GSM900 | 921 – 960 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| DCS1800 | 1805 - 1880 (NOTE 4) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| PCS1900 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 – 2170 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 - 1880 (NOTE 4) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 – 2690 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 – 960 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 – 746 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIIII or E-UTRA Band 13 | 746 – 756 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 – 768 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 17 | 734 – 746 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 18 | 860 – 875 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIX or E-UTRA Band 19 | 875 – 890 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 – 821 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 - 3 590 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 23 | 2180 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 24 | 1525 – 1559 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 27 | 852 – 869 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 28 or NR band n28 | 758 – 803 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 29 | 717 – 728 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 30 | 2350 – 2360 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 31 | 462.5 - 467.5 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 (NOTE-5) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 33 | 1900 – 1920 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 34 or NR band n34 | 2010 – 2025 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 35 | 1850 – 1910 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 36 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band c) or E-UTRA TDD Band 37 | 1910 – 1930 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 – 2620 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 – 2400 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 41 or NR band n41 | 2496 – 2690 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 42 | 3400 – 3600 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 43 | 3600 – 3800 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 44 | 703 – 803 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 45 | 1447 - 1467 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 46 | 5150 - 5925 | NA | NA | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 48 | 3550 – 3700 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 49 | 3550 – 3700 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 51 or or NR band n51 | 1427– 1432 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 65 | 2110 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 66 or or NR band n66 | 2110 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 67 | 738 - 758 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 68 | 753 - 783 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 69 | 2570-2620 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 70 or or NR band n70 | 1995 - 2020 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 71 or or NR band n71 | 617 - 652 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 72 | 461 - 466 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 73 | 460 - 465 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 74 or NR band n74 | 1475 - 1518 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 75 or or NR band n75 | 1432 - 1517 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 76 or or NR band n76 | 1427 - 1432 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n77 | 3300 - 4200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n78 | 3300 - 3800 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n79 | 4400 - 5000 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NOTE 1: EISminSENS depends on the BS class and on the channel bandwidth, see subclauses 10.2 in TS 37.105; "x" is equal to 6 dB in case of UTRA wanted signals. NOTE 2: Except for a BS operating in Band XIII, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band XIII the requirements do not apply when the interfering signal falls within the frequency range 768 - 797 MHz. NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in 3GPP TR 25.942 [31]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805 - 1850 MHz. NOTE 5: For an AAS BS operating in band XI, this requirement applies for interfering signal within the frequency range 1475.9 - 1495.9 MHz. |  |  |  |  |  |  |  |

##### 7.6.3.5.3	Single RAT E-UTRA operation

This additional blocking requirement may be applied for the protection of AAS BS receivers when E-UTRA BS, UTRA BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with an AAS BS.

The requirement is a co-location requirement, the interferer power levels specified at the CLTA conducted input(s).

The requirement is valid over minSENS RoAoA.

Interfering signal shall be applied to the CLTA. The interfering power is specified per polarization.

When the wanted and an interfering signal using the parameters in table 7.6.3.5.1-1 for co-location with UTRA or E-UTRA systems and table 7.6.3.5.3-1 for co-location with GSM systems, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [9], subclause 7.2.1.

Table 7.6.3.5.3-1: E-UTRA additional OTA blocking requirement for co-location with BS in other frequency bands

| Type of co-located BS | Centre Frequency of Interfering Signal [MHz] | Interfering Signal mean power for WA BS [dBm] | Interfering Signal mean power for MR BS [dBm] | Interfering Signal mean power for LA BS [dBm] | Wanted Signal mean power [dBm] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| GSM900 | 921 – 960 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| DCS1800 | 1805 - 1880 (NOTE 4) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| PCS1900 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 – 2170 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 - 1880 (NOTE 4) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 – 2690 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 – 960 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 – 746 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIIII or E-UTRA Band 13 | 746 – 756 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 – 768 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 17 | 734 – 746 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 18 | 860 – 875 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XIX or E-UTRA Band 19 | 875 – 890 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 – 821 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 - 3 590 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 23 | 2180 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 24 | 1525 – 1559 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 – 894 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 27 | 852 – 869 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 28 or NR band n28 | 758 – 803 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 29 | 717 – 728 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 30 | 2350 – 2360 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 31 | 462.5 - 467.5 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 (NOTE-5) | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 33 | 1900 – 1920 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 34 or NR band n34 | 2010 – 2025 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 35 | 1850 – 1910 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 36 | 1930 – 1990 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band c) or E-UTRA TDD Band 37 | 1910 – 1930 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 – 2620 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 – 2400 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 41 or NR band n41 | 2496 – 2690 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 42 | 3400 – 3600 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 43 | 3600 – 3800 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 44 | 703 – 803 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 45 | 1447 - 1467 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 46 | 5150 - 5925 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 48 | 3550 – 3700 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 49 | 3550 – 3700 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 51 or or NR band n51 | 1427– 1432 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 65 | 2110 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 66 or or NR band n66 | 2110 – 2200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 67 | 738 - 758 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 68 | 753 - 783 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 69 | 2570-2620 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 70 or or NR band n70 | 1995 - 2020 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 71 or or NR band n71 | 617 - 652 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 72 | 461 - 466 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 73 | 460 - 465 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 74 or NR band n74 | 1475 - 1518 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 75 or or NR band n75 | 1432 - 1517 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| E-UTRA Band 76 or or NR band n76 | 1427 - 1432 | N/A | N/A | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n77 | 3300 - 4200 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n78 | 3300 - 3800 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NR band n79 | 4400 - 5000 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |
| NOTE 1: EISminSENS depends on the BS class and on the channel bandwidth, see subclauses 10.2 in TS 37.105; "x" is equal to 6 dB in case of E-UTRA wanted signals. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768 - 797 MHz. NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in 3GPP TR 25.942 [31]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805 - 1850 MHz. NOTE 5: For an AAS BS operating in band 11 or 21, this requirement applies for interfering signal within the frequency range 1475.9 - 1495.9 MHz. |  |  |  |  |  |  |  |

## 7.7	OTA Receiver spurious emissions

### 7.7.1	Definition and applicability

The receiver spurious emission requirement is the power of the emissions radiated from the antenna array from a receiver unit. For an OTA AAS BS operating in FDD, OTA RX spurious emissions requirement do not apply as they are superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain.

NOTE:	The OTA receiver spurious emission requirement applicability for the AAS BS with the RX-only capabilities is FFS.

For an OTA AAS BS operating in TDD, the OTA receiver spurious emissions requirement applies during the transmitter OFF period only.

For RX only multi-band RIB, the RX spurious emissions requirements are subject to exclusion zones in each supported operating band.

### 7.7.2	Minimum Requirement

For an MSR AAS BS the minimum requirement is in TS 37.105 [6], subclause 10.7.2

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.7.4

For single RAT UTRA AAS BS the minimum requirement is in TS 37.105 [6], subclause 10.7.3

### 7.7.3	Test purpose

The test purpose is to verify the receiver radiated spurious emissions from the AAS BS at the RIB are within the specified requirements.

### 7.7.4	Method of test

#### 7.7.4.1	Initial conditions

Test environment: normal; see annex G.2.

RF channels to be tested for single carrier: M; see subclause 4.12.1.

RF bandwidth positions to be tested in single-band operation: -	MRFBW, see subclause 4.12.1,

RF bandwidth positions to be tested in multi-band operation, see subclause 4.12.1.

-	BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low  - ΔfOBUE

-	B'RFBW_TRFBW when testing from FDL_Bhigh_high  + ΔfOBUE to 12.75GHz (or to 5th harmonic)

-	BRFBW_T'RFBW and  B'RFBW_TRFBW when testing from FDL_Blow_high  + ΔfOBUE to FDL_Bhigh_low  - ΔfOBUE

Directions to be tested: Not applicable as Rx only TRP measurement.

#### 7.7.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in 3GPP TS 37.104 [5] subclause 6.6.1.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the TDD AAS BS to receive only

6)	Set the AAS BS in the direction of the appropriated TRP measurement grid (see annex F)

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-9 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex F).

Note 1: the TRP measurement grid may not be the same for all measurement frequencies.

Note 2: the frequency sweep or the TRP measurement grid sweep may be done in any order

9)	Calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.7.5	Test Requirement

The TRP of any spurious emission shall not exceed the levels in table 7.7.5-1:

Table 7.7.5-1: Reciever spurious emission test requirement

| Frequency range | Maximum level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 30MHz - 1 GHz | -45.5 dBm | 100 kHz |  |
| 1 GHz - 6GHz | -35.5 dBm | 1 MHz |  |
| 6 GHz - 12.75 GHz | -33.8 dBm | 1 MHz |  |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the UL operating band in GHz | -33.8 dBm | 1 MHz | Applies only for Bands 22, 42 and 43. |
| NOTE: The frequency range between 2.5 * channel bandwidth below the first carrier frequency and 2.5 * channel bandwidth above the last carrier frequency transmitted by the AAS BS may be excluded from the requirement. However, frequencies that are more than 10 MHz below the lowest frequency of any of the AAS BS supported downlink operating band or more than 10 MHz above the highest frequency of any of the AAS BS supported downlink operating band shall not be excluded from the requirement. For a multiband RIB, the exclusion applies for all supported operating bands. |  |  |  |

In addition to the requirements in table 7.7.5-1, the power of any spurious emission shall not exceed the levels specified for Protection of the E-UTRA FDD BS receiver of own or different BS in subclause 6.7.6.3 and for Co-existence with other systems in the same geographical area in subclause 6.7.6.4. In addition, the co-existence requirements for co-located base stations specified in subclause 6.7.6.5 may also be applied.

## 7.8	OTA Receiver intermodulation

### 7.8.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver unit to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS : the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS : the AoA of the incident wave of a received signal and the interfering signal are are within the within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

### 7.8.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 10.8.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.8.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.8.4.

### 7.8.3	Test purpose

The test purpose is to verify the ability of the receiver to inhibit the generation of intermodulation products in its non-linear elements caused by the presence of two high-level interfering signals at frequencies with a specific relationship to the frequency of the wanted signal.

### 7.8.4	Method of test

#### 7.8.4.1	Initial conditions

Test environment: normal; see annex G.2.

RF channels to be tested for single carrier:			M; see subclause 4.12.1.

Base Station RF Bandwidth positions to be tested:	For single-band RIB(s): MRFBW if ATC4 is applicable; BRFBW and TRFBW for other ATC, see subclause 4.12.1. For multi-band RIB(s): BRFBW_T'RFBW and B'RFBW_TRFBW, see subclause 4.12.1.

Directions to be tested: OTA REFSENS reciever target reference direction (see table 4.10-20 D11.30).

#### 7.8.4.2	Procedure

##### 7.8.4.2.1	General procedure

The general procedure steps apply to the procedures for all the RATs.

1)	Place the AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex D2.6.

2)	Align the manufacturer declared coordinate system orientation of the AAS BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the NR BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s)

5)	Configure the beam peak direction of the AAS BS according to declared reference beam direction pair for the appropriate beam identifier.

6)	Set the AAS BS to transmit the beam(s) of the same operational band and RAT as the OSDD being tested according to the appropriate test configuration in clause 5.

7)	Set the test signal mean power so the calibrated radiated power at the AAS BS Antenna Array coordinate system reference point is as specified as follows:

Set the signal generator for the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to transmit:

-	For E-UTRA see clause A.1 in TS 36.141 [12].

-	For UTRA FDD see clause A.2 in TS 25.141 [10].

-	For NR see clause A.1 in TS 38.141-2 [34].

##### 7.8.4.2.2	MSR operation

7.8.4.4.2.1	Procedure for general and narrowband intermodulation

1)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified in table 7.8.5.1.1-1 and Table 7.7.5.1.1-2 for general intermodulation requirement, and Table 7.8.5.2.1-1 and Table 7.78.5.2.1-2 for narrowband intermodulation requirement.

2)	Measure the performance of the wanted signal at the receiver under test, as defined in subclause 7.8.5.1.1 and 7.8.5.1.2, for the relevant carriers specified by the test configuration in clause 5.

3)	Repeat for all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

4)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.8.4.2.3	Single RAT UTRA FDD operation

1)	Generate the wanted signal (reference signal) and adjust ATT1 to set the signal level to the level specified in table 7.8.5.2-1. For a RIB supporting multi-carrier operation, generate the wanted signal according to the applicable test configuration (see clause 4.11) using applicable reference measurement channel. Power settings are specified in table 7.8.5.2-1.

2)	Adjust the signal generators to the type of interfering signals and the frequency offsets as specified in tables 7.8.5.2-1and 7.8.5.2-2. Note that the GMSK modulated interfering signal shall have an ACLR of at least 72 dB in order to eliminate the impact of interfering signal adjacent channel leakage power on the intermodulation characteristics measurement.

3)	Adjust the ATT2 and ATT3 to obtain the specified level of interfering signal at the RIB.

4)	Measure the BER of the wanted signal. For a RIB supporting multi-carrier operation the BER shall be measured for all relevant carriers specified by the test configuration.

5)	Repeat for all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

6)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.8.4.2.4	Single RAT E-UTRA operation

1)	Generate the wanted signal using the applicable test configuration specified in clause 5 and adjust the signal level to the level specified in table 7.8.5.3-1.

2)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified in table 7.8.5.3-2 for intermodulation requirement and Table 7.8.5.3-3, Table 7.8.5.3-4 and Table 7.8.5.3-5 for narrowband intermodulation requirement.

3)	Adjust the signal generators to obtain the specified level of interfering signal.

4)	Measure the throughput according to annex E of 36.141 [12]3GPP TS 36.141 [12], for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clause 5.

5)	Repeat for all supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

6)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.8.5	Test Requirement

#### 7.8.5.1	MSR operation

##### 7.8.5.1.1	General intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA, NR or UTRA signal as specified in 3GPP TS 37.104 [5], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap at those connectors, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals at the RIB, using the parameters in tables 7.8.5.1.1-1 and 7.8.5.1.1-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.5.3.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in TS 25.104 [2], subclause 7.2.5.1.

-	For any NR carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in TS 38.104 [33], subclause 7.2.5.3.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 7.8.5.1.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE 1) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48 + y - ΔOTAREFSENS (NOTE 6) | EISREFSENS +x dB (NOTE 2, 5) | See table 7.8.5.1.1-2 |
|  | -48 + y – ΔminSENS (NOTE 6) | EISminSENS + x dB (NOTE 2, 5) |  |
| Medium Range BS | -44 + y - ΔOTAREFSENS (NOTE 6) | EISREFSENS + x dB (NOTE 3, 5) |  |
|  | -44 + y – ΔminSENS (NOTE 6) | EISminSENS + x dB (NOTE 3, 5) |  |
| Local Area BS | -38 + y - ΔOTAREFSENS (NOTE 6) | EISREFSENS +x dB (NOTE 4, 5) |  |
|  | -38 + y – ΔminSENS (NOTE 6) | EISminSENS + x dB (NOTE 4, 5) |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: For WA BS not supporting NR, “x” is equal to 6 in case of E-UTRA or UTRA or NB-IoT wanted signals and equal to 3 in case of GSM/EDGE wanted signal. NOTE 3: For MR BS not supporting NR, “x” is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA or NB-IoT wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 4: For LA BS not supporting NR, “x” is equal to 12 in case of E-UTRA or NB-IoT wanted signals, 6 in case of UTRA wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 5: For a BS supporting NR and not supporting UTRA, x is equal to 6. NOTE 6: For a BS not supporting NR, “y” is equal to zero for all BS classes. For a BS that supports NR and supporting UTRA; “y” is equal to -4 for the WA BS class, -3 for the MR BS class and -6 for the LA BS class. |  |  |  |

Table 7.8.5.1.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2,0 (BC1 and BC3) / ±2,1 (BC2) | CW |
|  | ±4,9 | 1,4 MHz E-UTRA signal |
| E-UTRA 3 MHz | ±4,4 (BC1 and BC3) / ±4,5 (BC2) | CW |
|  | ±10,5 | 3 MHz E-UTRA signal |
| UTRA FDD and E-UTRA 5 MHz | ±7,5 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 10 MHz | ±7,375 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 15 MHz | ±7,25 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 20 MHz | ±7,125 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| GSM/EDGE | ±7,575 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| 1,28 Mcps UTRA TDD | ±2,3 (BC3) | CW |
|  | ±5,6 (BC3) | 1,28 Mcps UTRA TDD signal |
| NR 5 MHz | [±7.5] | CW |
|  | [±17.5] | 5MHz E-UTRA signal |
| NR 10 MHz | [±7.45] | CW |
|  | [±17.5] | 5MHz E-UTRA signal |
| NR 15 MHz | [±7.43] | CW |
|  | [±17.5] | 5MHz E-UTRA signal |
| NR 20 MHz | [±7.38] | CW |
|  | [±17.5] | 5MHz E-UTRA signal |
| NR 25 MHz | [±7.45] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 30 MHz | [±7.43] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 40 MHz | [±7.45] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 50 MHz | [±7.35] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 60 MHz | [±7.49] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 70 MHz | [±7.42] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 80 MHz | [±7.44] | CW |
|  | [±25] | 20MHz E-UTRA signal |
| NR 90 MHz | [±25] | CW |
|  | [±7.43] | 20MHz E-UTRA signal |
| NR 100 MHz | [±7.45] | CW |
|  | [±25] | 20MHz E-UTRA signal |

##### 7.8.5.1.2	General narrowband intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal as specified in 3GPP TS 37.104 [5], annex A.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For RIB supporting operation in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 10.8.2.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least as wide as the E-UTRA interfering signal in table 7.8.5.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals at the RIB, using the parameters in tables 7.8.5.1.2-1 and 7.8.5.1.2-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.5.3.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in TS 25.104 [2], subclause 7.2.5.1.

-	For any NR carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in TS 38.104 [33], subclause 7.2.5.3.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 7.8.5.1.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 - ΔOTAREFSENS | EISREFSENS + 6 dB | See table 7.8.5.1.2-2 |
|  | -52 – ΔminSENS | EISminSENS + 6 dB |  |
| Medium Range BS | -47 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |
|  | -47 – ΔminSENS | EISminSENS + 6 dB |  |
| Local Area BS | -44 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |
|  | -44 – ΔminSENS | EISminSENS + 6 dB |  |
| NOTE EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 10.3 and 10.2. |  |  |  |

Table 7.8.5.1.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or edge of the sub-block | CW or 1RB interfering signal centre frequency offset from the Base Station RF Bandwidthedge or edge of sub-block inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3,0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 5 MHz | ±360 | CW |
|  | ±1 060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 10 MHz (NOTE 2) | ±325 | CW |
|  | ±1 240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 15 MHz (NOTE 2) | ±380 | CW |
|  | ±1 600 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 20 MHz (NOTE 2) | ±345 | CW |
|  | ±1 780 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1 780 (BC1 and BC2) | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| GSM/EDGE | ±340 | CW |
|  | ±880 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| 1,28 Mcps UTRA TDD | ±190 (BC3) | CW |
|  | ±970 (BC3) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz | [±360] | CW |
|  | [±1420] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz | [±325] | CW |
|  | [±1780] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz (Note 2) | [±380] | CW |
|  | [±1600] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz (Note 2) | [±345] | CW |
|  | [±1780] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz (Note 2) | [±325] | CW |
|  | [±1990] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz (Note 2) | [±320] | CW |
|  | [±1990] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz (Note 2) | [±310] | CW |
|  | [±2710] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz (Note 2) | [±330] | CW |
|  | [±3250] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz (Note 2) | [±350] | CW |
|  | [±3790] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz (Note 2) | [±400] | CW |
|  | [±4870] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz (Note 2) | [±390] | CW |
|  | [±4870] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz (Note 2) | [±340] | CW |
|  | [±5770] | E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz (Note 2) | [±340] | CW |
|  | [±5770] | E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

#### 7.8.5.2	Single RAT UTRA operation

The static reference performance as specified in subclause 10.3 and 10.2 shall be met for a Wide Area BS when the signals in table 7.8.5.2-1 and table 7.8.5.2-2 are at the RIB.

The static reference performance as specified in subclause 10.3 and 10.2 shall be met for a Medium range BS when the signals in table 7.8.5.2-3 and table 7.8.5.2-4 are at the RIB.

The static reference performance as specified in subclause 10.3 and 10.2 shall be met for a Local Area BS when the signals in table 7.8.5.2-5 and table 7.8.5.2-6 are at the RIB.

For RIB supporting operation in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least is at least 6.8MHz. The CW interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -1MHz/+1MHz, respectively. The GMSK modulated interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -3.4MHz/+3.4MHz, respectively.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 6.8MHz. The CW interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -1MHz/+1MHz, respectively. The GMSK modulated interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -3.4MHz/+3.4MHz, respectively.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 7.8.5.2-1: Intermodulation performance requirement (Wide Area BS)

| Operating band | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Offset | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| All bands | - 48 - ΔOTAREFSENS | - 115 - ΔOTAREFSENS | ±10 MHz | CW signal |
|  | -48 – ΔminSENS | -115 – ΔminSENS |  |  |
|  | - 48 - ΔOTAREFSENS | - 115 - ΔOTAREFSENS | ±20 MHz | WCDMA signal (NOTE) |
|  | -48 – ΔminSENS | -115 – ΔminSENS |  |  |
| NOTE: The characteristics of the WCDMA interference signal are specified in 3GPP TS 25.104 [2] Annex C. |  |  |  |  |

Table 7.8.5.2-2: Narrowband intermodulation performance requirement (Wide Area BS)

| Operating band | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Offset | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| II, III, IV, V, VIII, X, XII, XIII, XIV, XXV, XXVI | - 47 - ΔOTAREFSENS | - 115 - ΔOTAREFSENS | ±3.5 MHz | CW signal |
|  | -47 – ΔminSENS | -115 – ΔminSENS |  |  |
|  | - 47 - ΔOTAREFSENS | - 115 - ΔOTAREFSENS | ±5.9 MHz | GMSK modulated (NOTE) |
|  | -47 – ΔminSENS | -115 – ΔminSENS |  |  |
| NOTE: GMSK as defined in TS45.004 [32] |  |  |  |  |

Table 7.8.5.2-3: Intermodulation performance requirement (Medium Range BS)

| Operating band | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Offset | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| All bands | - 44 - ΔOTAREFSENS | - 105 - ΔOTAREFSENS | ±10 MHz | CW signal |
|  | -44 – ΔminSENS | -105 – ΔminSENS |  |  |
|  | - 44 - ΔOTAREFSENS | - 105 - ΔOTAREFSENS | ±20 MHz | WCDMA signal (NOTE) |
|  | -44 – ΔminSENS | -105 – ΔminSENS |  |  |
| NOTE: The characteristics of the WCDMA interference signal are specified in 3GPP TS 25.104 [2] Annex C. |  |  |  |  |

Table 7.8.5.2-4: Narrowband intermodulation performance requirement (Medium Range BS)

| Operating band | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Offset | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| II, III, IV, V, VIII, X, XII, XIII, XIV, XXV, XXVI | - 43 - ΔOTAREFSENS | - 105 - ΔOTAREFSENS | ±3.5 MHz | CW signal |
|  | -43 – ΔminSENS | -105 – ΔminSENS |  |  |
|  | - 43 - ΔOTAREFSENS | - 105 - ΔOTAREFSENS | ±5.9 MHz | GMSK modulated (NOTE) |
|  | -43 – ΔminSENS | -105 – ΔminSENS |  |  |
| NOTE: GMSK as defined in TS45.004 [32] |  |  |  |  |

Table 7.8.5.2-5: Intermodulation performance requirement (Local Area BS)

| Operating band | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Offset | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| All bands | - 38 - ΔOTAREFSENS | - 101 - ΔOTAREFSENS | ±10 MHz | CW signal |
|  | -38 – ΔminSENS | -101 – ΔminSENS |  |  |
|  | - 38 - ΔOTAREFSENS | - 101 - ΔOTAREFSENS | ±20 MHz | WCDMA signal (NOTE) |
|  | -38 – ΔminSENS | -101 – ΔminSENS |  |  |
| NOTE: The characteristics of the WCDMA interference signal are specified in 3GPP TS 25.104 [2] Annex C. |  |  |  |  |

Table 7.8.5.2-6: Narrowband intermodulation performance requirement (Local Area BS)

| Operating band | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] (NOTE) | Offset | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| II, III, IV, V, VIII, X, XII, XIII, XIV, XXV, XXVI | - 38 - ΔOTAREFSENS | - 101 - ΔOTAREFSENS | ±3.5 MHz | CW signal |
|  | -38 – ΔminSENS | -101 – ΔminSENS |  |  |
|  | - 38 - ΔOTAREFSENS | - 101 - ΔOTAREFSENS | ±5.9 MHz | GMSK modulated (NOTE) |
|  | -38 – ΔminSENS | -101 – ΔminSENS |  |  |
| NOTE GMSK as defined in TS45.004 [32] |  |  |  |  |

#### 7.8.5.3	Single RAT E- UTRA operation

For E-UTRA, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals at the RIB, with the conditions specified in tables 7.8.5.3-1 and 7.8.5.3-2 for intermodulation performance and in tables 7.8.5.3-3, 7.8.5.3-4, and 7.8.5.3-5 for narrowband intermodulation performance. Narrowband intermodulation requirements are not applied for Band 46. The reference measurement channel for the wanted signal is identified in table 10.8.4-1 to 6 for each channel bandwidth and further specified in 3GPP TS 36.104 [4] Annex A.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For RIB supporting operation in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.8.5.3-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least as wide as the E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge.

For multi-band RIBs, the narrowband intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least as wide as the E-UTRA interfering signal in tables 7.8.5.3-3, 7.8.5.3-4 and 7.8.5.3-5. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 7.8.5.3-1: Intermodulation performance requirement

| BS type | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] (NOTE) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | See table 7.8.5.3-2 |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6dB | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |
| Local Area BS | EISREFSENS + 6dB | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |
| NOTE: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. |  |  |  |

Table 7.8.5.3-2: Interfering signal for Intermodulation performance requirement

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 3 | ±4.5 | CW |
|  | ±10.5 | 3 MHz E-UTRA signal (NOTE 3) |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| 10 | ±7.375 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| 15 | ±7.25 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| 20 | ±7.125 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal (NOTE 1) |
| 20 | ±7.125 | CW |
|  | ±24 | 20 MHz E-UTRA signal (NOTE 2) |
| NOTE 1: This type of interfering signal is not applied for Band 46. NOTE 2: This type of interfering signal is only applied for Band 46. NOTE 3: 3 MHz channel bandwidth is not applicable to guard band operation. |  |  |

Table 7.8.5.3-3: Narrowband intermodulation performance requirement for Wide Area BS

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] (NOTE 1) | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±270 | CW |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±790 | 1.4 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
| 3 | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±270 | CW |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±780 | 3.0 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
| 5 | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±360 | CW |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±1060 | 5 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
| 10 (NOTE 3) | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±325 | CW |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
| 15 (NOTE 3) | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±380 | CW |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±1600 | 5MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
| 20 (NOTE 3) | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±345 | CW |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -52 - ΔOTAREFSENS | ±1780 | 5MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -52 – ΔminSENS |  |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 3: This requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals |  |  |  |  |

Table 7.8.5.3-4: Narrowband intermodulation performance requirement for Local Area BS

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] (NOTE 1) | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±270 | CW |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±790 | 1.4 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
| 3 | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±270 | CW |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±780 | 3.0 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
| 5 | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±360 | CW |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±1060 | 5 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
| 10 (NOTE 3) | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±325 | CW |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
| 15 (NOTE 3) | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±380 | CW |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±1600 | 5MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
| 20 (NOTE 3) | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±345 | CW |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -44 - ΔOTAREFSENS | ±1780 | 5MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -44 – ΔminSENS |  |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 3: This requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals |  |  |  |  |

Table 7.8.5.3-5: Narrowband intermodulation performance requirement for Medium Range BS

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] (NOTE 1) | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±270 | CW |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±790 | 1.4 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
| 3 | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±270 | CW |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±780 | 3.0 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
| 5 | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±360 | CW |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±1060 | 5 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
| 10 (NOTE 3) | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±325 | CW |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
| 15 (NOTE 3) | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±380 | CW |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±1600 | 5MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
| 20 (NOTE 3) | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±345 | CW |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
|  | EISREFSENS + 6dB | -47 - ΔOTAREFSENS | ±1780 | 5MHz E-UTRA signal, 1 RB (NOTE 2) |
|  | EISminSENS + 6 dB | -47 – ΔminSENS |  |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 7.3 and 7.2. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 3: This requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals |  |  |  |  |

## 7.9	OTA In-channel selectivity

### 7.9.1	Definition and applicability

In-channel selectivity (ICS) is a measure of the receiver unit ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel.

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

### 7.9.2	Minimum Requirement

For AAS BS in MSR operation the minimum requirement is defined in TS 37.105 [6], subclause 10.9.2.

For AAS BS in single RAT UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.9.3.

For AAS BS in single RAT E-UTRA operation the minimum requirement is defined in TS 37.105 [6], subclause 10.9.4.

### 7.9.3	Test purpose

The purpose of this test is to verify the ability of the receiver to suppress the IQ leakage.

### 7.9.4	Method of test

#### 7.9.4.1	Initial conditions

Test environment:	normal; see annex G.2

RF channels to be tested for single carrier:	M; see subclause 4.12.1.

Directions to be tested: OTA REFSENS reciever target reference direction (see table 4.10-2, D11.30).

#### 7.9.4.2	Procedure

1)	Place the AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex D1.1.

2)	Align the manufacturer declared coordinate system orientation of the AAS BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the NR BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s)

For each supported E-UTRA channel BW:

5)	Adjust the signal generator for the wanted signal as specified in table 7.9.5.1-1 for AAS BS of  Wide Area BS class, in table 7.9.5.1-2 for AAS BS of  Local Area BS class and in table 7.9.5.1-3 for AAS BS of  Medium Range BS class on one side of the FC.

6)	Adjust the signal generator for the interfering signal as specified in table 7.9.5.1-1 for AAS BS of  Wide Area BS class, in table 7.9.5.1-2 for AAS BS of  Local Area BS class and in table 7.9.5.1-3 for AAS BS of  Medium Range BS class at opposite side of the FC and adjacent to the wanted signal.

7)	Measure throughput according to annex E in TS 36.141 [12].

8)	Repeat the measurement with the wanted signal on the other side of the FC, and the interfering signal at opposite side of the FC and adjacent to the wanted signal.

9)	Repeat for all supported polarizations. 10

For each supported NR channel BW:

2)	Adjust the signal generator for the wanted signal as specified in table 7.9.5.2-1 for AAS BS of  Wide Area BS class, in table 7.9.5.2-2 for AAS BS of  Local Area BS class and in table 7.9.5.2-3 for AAS BS of  Medium Range BS class on one side of the FC.

3)	Adjust the signal generator for the interfering signal as specified in table 7.9.5.2-1 for AAS BS of  Wide Area BS class, in table 7.9.5.2-2 for AAS BS of  Local Area BS class and in table 7.9.5.2-3 for AAS BS of  Medium Range BS class at opposite side of the FC and adjacent to the wanted signal.

4)	Measure throughput according to annex A in TS 38.141-2 [34].

5)	Repeat the measurement with the wanted signal on the other side of the FC, and the interfering signal at opposite side of the FC and adjacent to the wanted signal.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.9.5	Test Requirement

### 7.9.5.1	E-UTRA test requirement

For E-UTRA, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in 3GPP 36.104 [4] Annex A with parameters specified in table 10.9.4-1 for Wide Area BS, in table 10.9.4-2 for Local Area BS and in table 10.9.4-3 for Medium Range BS.

The OTA levels are applied referenced to ΔminSENS.

Table 7.9.5-1 Wide Area BS in-channel selectivity for E-UTRA

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in 3GPP 36.104 [4] Annex A.1 | -105.2– ΔminSENS | -104.8 – ΔminSENS | -87 – ΔminSENS | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in 3GPP 36.104 [4] Annex A.1 | -100.4 – ΔminSENS | -100.0– ΔminSENS | -84 – ΔminSENS | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in 3GPP 36.104 [4] Annex A.1 | -98.3 – ΔminSENS | -97.9 – ΔminSENS | -81 – ΔminSENS | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in 3GPP 36.104 [4] Annex A.1 | -96.8 – ΔminSENS | -96.4 – ΔminSENS | -77 – ΔminSENS | 10 MHz E-UTRA signal, 25 RBs |
| 15 | A1-3 in 3GPP 36.104 [4] Annex A.1 (NOTE) | -96.8 – ΔminSENS | -96.4 – ΔminSENS | -77 – ΔminSENS | 15 MHz E-UTRA signal, 25 RBs (NOTE) |
| 20 | A1-3 in 3GPP 36.104 [4] Annex A.1 (NOTE) | -96.8 – ΔminSENS | -96.4 – ΔminSENS | -77 – ΔminSENS | 20 MHz E-UTRA signal, 25 RBs (NOTE) |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

Table 7.9.5-2 Local Area BS in-channel selectivity for E-UTRA

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in 3GPP 36.104 [4] Annex A.1 | -97.2 – ΔminSENS | -96.8 – ΔminSENS | -79 – ΔminSENS | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in 3GPP 36.104 [4] Annex A.1 | -92.4 – ΔminSENS | -92.0 – ΔminSENS | -76 – ΔminSENS | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in 3GPP 36.104 [4] Annex A.1 | -90.3 – ΔminSENS | -89.9 – ΔminSENS | -73 – ΔminSENS | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in 3GPP 36.104 [4] Annex A.1 (NOTE 3) A1-8 in 3GPP 36.104 [4] Annex A.1 (NOTE 2) | -88.8 – ΔminSENS -91.5 – ΔminSENS | -88.4 – ΔminSENS -91.1 – ΔminSENS | -69 – ΔminSENS -71.8 – ΔminSENS | 10 MHz E-UTRA signal, 25 RBs (NOTE 3) 10 MHz E-UTRA interlace signal, 10 RBs (NOTE 2) |
| 15 | A1-3 in 3GPP 36.104 [4] Annex A.1 (NOTE 1) | -88.8 – ΔminSENS | -88.4 – ΔminSENS | -69 – ΔminSENS | 15 MHz E-UTRA signal, 25 RBs (NOTE 1) |
| 20 | A1-3 in 3GPP 36.104 [4] Annex A.1 (NOTE 1) A1-9 in 3GPP 36.104 [4] Annex A.1 (NOTE 2) | -88.8 – ΔminSENS -91.5 – ΔminSENS | -88.4 – ΔminSENS -91.1 – ΔminSENS | -69 – ΔminSENS -71.8 – ΔminSENS | 20 MHz E-UTRA signal, 25 RBs (NOTE 1) 20 MHz E-UTRA interlace signal, 10 RBs (NOTE 2) |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, this reference measurement channel and interfering signal are not applied for Band 46. NOTE 2: Wanted and interfering signal interlaces are mirrored around Fc, this reference measurement channel and interfering signal are only applied for Band 46. NOTE 3: This reference measurement channel and interfering signal are not applied for Band 46. |  |  |  |  |  |

Table 7.9.5-3 Medium Range BS in-channel selectivity for E-UTRA

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in 3GPP 36.104 [4] Annex A.1 | -100.2 – ΔminSENS | -99.8 – ΔminSENS | -82 – ΔminSENS | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in 3GPP 36.104 [4] Annex A.1 | -95.4 – ΔminSENS | -95.0 – ΔminSENS | -79 – ΔminSENS | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in 3GPP 36.104 [4] Annex A.1 | -93.3 – ΔminSENS | -92.9 – ΔminSENS | -76 – ΔminSENS | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in 3GPP 36.104 [4] Annex A.1 (NOTE 3) A1-8 in 3GPP 36.104 [4] Annex A.1 (NOTE 2) | -91.8 – ΔminSENS -94.5 – ΔminSENS | -91.4– ΔminSENS -94.1 – ΔminSENS | -72 – ΔminSENS -74.8 – ΔminSENS | 10 MHz E-UTRA signal, 25 RBs (NOTE 3) 10 MHz E-UTRA interlace signal, 10 RBs (NOTE 2) |
| 15 | A1-3 in 3GPP 36.104 [4] Annex A.1 (NOTE 1) | -91.8– ΔminSENS | -91.4– ΔminSENS | -72 – ΔminSENS | 15 MHz E-UTRA signal, 25 RBs (NOTE 1) |
| 20 | A1-3 in 3GPP 36.104 [4] Annex A.1 (NOTE 1) A1-9 in 3GPP 36.104 [4] Annex A.1 (NOTE 2) | -91.8– ΔminSENS -94.5 – ΔminSENS | -91.4– ΔminSENS -94.1 – ΔminSENS | -72 – ΔminSENS -74.8 – ΔminSENS | 20 MHz E-UTRA signal, 25 RBs (NOTE 1) 20 MHz E-UTRA interlace signal, 10 RBs (NOTE 2) |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, this reference measurement channel and interfering signal are not applied for Band 46. NOTE 2: Wanted and interfering signal interlaces are mirrored around Fc, this reference measurement channel and interfering signal are only applied for Band 46. NOTE 3: This reference measurement channel and interfering signal are not applied for Band 46. |  |  |  |  |  |

#### 7.9.5.2	NR test requirement

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted and interfering signals applies to all supported polarizations, under the assumption of polarization matching.

Details of the reference measurement channels can be found in TS 38.141-2 [34] annex A.

Table 7.9.5.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth [MHz] | Subcarrier spacing [KHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -98.9-ΔminSENS | -98.5-ΔminSENS | -98.2-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -97-ΔminSENS | -96.6-ΔminSENS | -96.3-ΔminSENS | -77.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40, 50 | 15 | G-FR1-A1-4 | -90.6-ΔminSENS | -90.2-ΔminSENS | -89.9-ΔminSENS | -71.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 15 kHz, 100 PRB |
| 5 | 30 | G-FR1-A1-8 | -99.6-ΔminSENS | -99.2-ΔminSENS | -98.9-ΔminSENS | -81.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -97.1-ΔminSENS | -96.7-ΔminSENS | -96.4-ΔminSENS | -78.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 30 kHz, 10 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -71.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 30 kHz, 50 PRB |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -96.5-ΔminSENS | -96.1-ΔminSENS | -95.8-ΔminSENS | -78.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -91-ΔminSENS | -90.6-ΔminSENS | -90.3-ΔminSENS | -71.6 - ΔminSEN | DFT-s-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 and located in the interfering signal. |  |  |  |  |  |  |  |

Table 7.9.5.2-2: Medium Range BS in-channel selectivity

| BS channel bandwidth [MHz] | Subcarrier spacing [KHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -93.9-ΔminSENS | -93.5-ΔminSENS | -93.2-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -92-ΔminSENS | -91.6-ΔminSENS | -91.3-ΔminSENS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40, 50 | 15 | G-FR1-A1-4 | -85.6-ΔminSENS | -85.2-ΔminSENS | -84.9-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 100 PRB |
| 5 | 30 | G-FR1-A1-8 | -94.6-ΔminSENS | -94.2-ΔminSENS | -93.9-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -92.1-ΔminSENS | -91.7-ΔminSENS | -91.4-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 10 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -85.9-ΔminSENS | -85.5-ΔminSENS | -85.2-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 50 PRB |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -91.5-ΔminSENS | -91.1-ΔminSENS | -90.8-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86-ΔminSENS | -85.6-ΔminSENS | -85.3-ΔminSENS | -66.6 - ΔminSENS | DFT-s-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 and located in the interfering signal. |  |  |  |  |  |  |  |

Table 7.9.5.2-3: Local area BS in-channel selectivity

| BS channel bandwidth [MHz] | Subcarrier spacing [KHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -89-ΔminSENS | -88.6-ΔminSENS | -88.3-ΔminSENS | -69.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40, 50 | 15 | G-FR1-A1-4 | -82.6-ΔminSENS | -82.2-ΔminSENS | -81.9-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 100 PRB |
| 5 | 30 | G-FR1-A1-8 | -91.6-ΔminSENS | -91.2-ΔminSENS | -90.9-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -89.1-ΔminSENS | -88.7-ΔminSENS | -88.4-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 10 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -82.9-ΔminSENS | -82.5-ΔminSENS | -82.2-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 50 PRB |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -88.5-ΔminSENS | -88.1-ΔminSENS | -87.8-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -83-ΔminSENS | -82.6-ΔminSENS | -82.3-ΔminSENS | -63.6 - ΔminSENS | DFT-s-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 and located in the interfering signal. |  |  |  |  |  |  |  |

# 8	Radiated performance requirements

## 8.1	General

Radiated performance requirements specify the ability of the OTA AAS BS to correctly demodulate radiated signals in various propagation conditions and configurations.

The demodulation requirements for an OTA AAS BS are limited to two OTA demodulations branches as described in subcluse 8.1.1, and are the same as non-AAS BS demodulation requirements specified for:

-	Single RAT UTRA operation in 3GPP TS 25.104 [2] clause 8 for FDD operation,

-	Single RAT E-UTRA operation in 3GPP TS 36.104 [4], subclauses 8.1 – 8.4,

-	MSR operation in 3GPP TS 37.105 [6], based on references to the single RAT requirements in 3GPP TS 25.104 [2] and 3GPP TS 36.104 [4].

### 8.1.1	OTA demodulation branches

OTA performance requirements are only specified for up to 2 demodulation branches.

If the OTA AAS BS uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then OTA performance requirements can be tested for up to two demodulation branches (i.e. 1TX-1RX or 1TX-2RX test setup). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the OTA AAS BS does not use polarization diversity then OTA performance requirements can only be tested for a single demodulation branch (i.e. 1TX-1RX test setup).

## 8.2	Radiated performance requirements for MSR

For OTA AAS BS in single RAT UTRA operation, minimum requirements for radiated demodulation performance are specified in subclause 8.3.

For OTA AAS BS in single RAT E-UTRA operation, minimum requirements for radiated demodulation performance are specified in subclause 8.4.

NOTE:	Radiated performance requirements for MSR BS are applicable to the multi-RAT of single RAT operation, but tested only in single RAT configuration.

## 8.3	Radiated performance requirements for UTRA FDD

### 8.3.1	General

Radiated performance requirements for single RAT UTRA operation in FDD are specified for the fixed reference channels (FRC) and propagation conditions defined in TS 25.104 [2] annex A and annex B, respectively. The requirements only apply to those FRCs that are supported by the OTA AAS BS.

Unless stated otherwise, radiated performance requirements apply for a single carrier only. Performance requirements for a BS supporting DC-HSUPA or DB-DC-HSUPA are defined in terms of single carrier requirements. The requirements in subclause 8.3 shall be met with the transmitter(s) ON.

NOTE:	In normal operating conditions the transceiver units are configured to transmit and receive at the same time. The transmitter unit(s) associated with the RIB may be OFF for some of the tests in clause 8.

In the referenced conducted test requirements from TS 25.141 [10] the method to test describes connection to one or a number of BS antenna connectors. When applying these methods to the OTA AAS BS, connection shall be made to the RIB, based on one or two OTA demodulation branches.

In the referred UTRA specifications and in this section, the term “BS with RX diversity” refers to performance requirements for two demodulation branches, and the term “BS without RX diversity” refers to performance requirements for one demodulation branch. For “BS with RX diversity”, only the BS performance requirements with Rx diversity apply, otherwise only the BS performance requirements without Rx diversity apply.

For testing purposes, there is one demodulation branch per active polarization assumed.In tests performed with signal generators a synchronization signal may be provided, from the BS to the signal generator, to enable correct timing of the wanted signal.

The Eb/N0 used is defined as:

Where:

is the received total energy of DPDCH, DPCCH, S-DPCCH, HS-DPCCH, E-DPDCH, S-E-DPDCH, E-DPCCH and S-E-DPCCH per PN chip per demodulation branch from all branches

is the total one-sided noise power spectral density due to all noise sources

is the number of chips per frame

is the number of information bits in DTCH excluding CRC bits per frame

All Bit Error Ratio (BER) and Block Error ratio (BLER) measurements shall be carried out according to the general rules for statistical testing defined in ITU-T Recommendation O.153 [30] and TS 25.141 [10], annex C.

If external BLER measurement is not used then the internal BLER calculation shall be used instead. When internal BLER calculation is used, the requirements of the verification test according to TS 25.141 [10] subclause 8.6 shall be met in advance.

Table 8.3.1-1: Summary of AAS BS performance targets for single RAT UTRA operation in FDD

| Physical channel | Measurement channel | Static | Multi-path Case 1 | Multi-path Case 2 | Multi-path Case 3 | Moving | Birth / Death | High Speed Train (Note) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Performance metric |  |  |  |  |  |  |
| DCH | 12.2 kbps | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER <10-2 | BLER <10-2 | BLER <10-2 |
|  | 64 kbps | BLER< 10-1,10-2 | BLER< 10-1, 10-2 | BLER< 10-1,10-2 | BLER < 10-1, 10-2,10-3 | BLER < 10-1,10-2 | BLER < 10-1,10-2 | - |
|  | 144 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER < 10-1, 10-2,10-3 | - | - | - |
|  | 384 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER < 10-1, 10-2,10-3 | - | - | - |
| NOTE: Optional condition, not applicable for all BSs. |  |  |  |  |  |  |  |  |

### 8.3.2	Definitions and applicability

Definitions of radiated performance requirements in single RAT UTRA operation in FDD and their applicability are the same as defined in TS 25.141 [10], in requirement’s specific “Definition and applicability” subclauses within 8.2 - 8.13 subcluases.

The following limitation apply for the radiated performance requirements in single RAT UTRA operation in FDD:

-	All the radiated performance requirements referred from TS 25.141 [10] are subject to 2Rx limitation in the OTA test setup, as captured in subclause 8.1.1.

-	If the OTA AAS BS does not use polarisation diversity then performance requirements only apply to a single demodulation branch (i.e. 1TX-1RX test setup).

-	If the OTA AAS BS uses polarisation diversity and has the ability to maintain isolation between the performance requirements signals for each of the demodulation branches, then performance requirements can be applied to up to two demodulation branches (i.e. 1TX-2RX test setups).

NOTE:	for the list of BS demodulation requirements which were found to be feasible in OTA test setup with the above 2Rx limitation, refer to TR 37.843 [29], subclause 7.8.

-	For FRC8 in TS 25.104 [2] the non E-DPCCH boosting and E-DPCCH boosting requirement only apply for the option supported by the OTA AAS BS.

-	Performance of signalling detection for 4C-HSDPA HS-DPCCH test shall be performed only for the BS supporting 4C-HSDPA.

-	Performance of signalling detection for 8C-HSDPA HS-DPCCH test shall be performed only for the BS supporting 8C-HSDPA.

-	The performance requirements for the high speed train scenarios defined in TS 25.104 [2] are optional.

### 8.3.3	Minimum requirements

Minimum requirements for radiated performance requirements in single RAT UTRA operation in FDD are the same as defined in TS 25.104 [2], in requirement’s specific “Minimum requirement” subclauses within 8.2 - 8.12 subcluases, subject to limitations listed in subclause 8.3.2.

### 8.3.4	Test purposes

Test purposes for the radiated performance requirements in single RAT UTRA operation in FDD are the same as defined in TS 25.141 [10], in requirement’s specific “Test purpose” subclauses within subcluases 8.2 - 8.13.

The radiated test shall verify the OTA AAS BS receiver’s ability to achieve requirement’s specific performance metric under defined propagation conditions emulated in OTA test chamber, for a given Eb/N0 (or Ec/N0).

### 8.3.5	Method of test

#### 8.3.5.1	Initial conditions

Initial conditions of radiated performance requirements in single RAT UTRA operation in FDD are the same as defined in TS 25.141 [10], in requirement’s specific “Initial conditions” subclauses within 8.2 - 8.13 subcluases, with the exception, that instead of connecting the BS tester(s) directly to antenna connector(s), either a single or two polarizations should be transmitted via test antenna(s) in the OTA chamber, where each polarization represents a demodulation branch.

The initial conditions for the radiated performance requirements in single RAT UTRA operation in FDD are generalized as follows:

Test environment:	normal, see annex G.2.

RF channels to be tested:	B, M and T; see TS 25.141 [10], subclause 4.8.

Direction to be tested: receiver target reference direction (see table 4.10-1, D10.9).

#### 8.3.5.2	Procedure

1)	Place the OTA AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.2.1.

2)	Align the manufacturer declared coordinate system orientation of the OTA AAS BS with the test system.

3)	Set the OTA AAS BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and/or AWGN generators (depending on the required OTA test procedure) to a test antenna via a combining network in OTA test setup, as shown in appropriate subclause in annex D.2.2.

5)	Apply the conducted performance test procedure appropriate to the requirement as described in subclause 8 of TS 25.141 [10]. Instead of connection via an antenna connector, one of the RX antenna signals should be transmitted on each polarization of the test antenna(s).

6)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A in TS 25.141 [10], and according to additional test parameters listed in respective conducted performance test procedure in TS 25.141 [10].

a.	For RACH requrements: configure test signal generator to sends a preamble according to the test signal pattern defined for RACH in TS 25.141 [10].

7)	The multipath fading emulators shall be configured according to the corresponding channel model defined in TS 25.141 [10], annex D.

8)	Adjust the test signal mean power so the calibrated radiated signal level at the BS receiver is as specified in requirement’s specific subclause 8 of TS 25.141 [10].

a.	For DCH requirements: Adjust the test signal mean power so the calibrated radiated Eb/N0value at the BS receiver is as specified in requirement’s specific subclause 8 of TS 25.141 [10]. To achieve the specified Eb/N0, the ratio of the wanted signal level relative to the AWGN signal should be adjusted to: 10 * Log10(Rb / 3.84 * 106) + Eb/N0 [dB].

b.	For RACH requirements: Adjust the test signal mean power so the calibrated radiated Eb/N0 (or Ec/N0) value at the BS receiver is as specified in requirement’s specific subclause 8 of TS 25.141 [10].

i.	For RACH preamble detection in static propagation, multipath fading case 3, or in high speed train conditions: To achieve the specified Ec/N0, the ratio of the wanted signal level (of the preamble part) relative to the AWGN signal at the BS receiver should be adjusted to: Ec/N0 [dB].

ii.	For Demodulation of RACH message in static propagation conditions, multipath fading case 3, or in high speed train conditions: To achieve the specified Ec/N0, the ratio of the wanted signal level (of the message part) relative to the AWGN signal at the BS receiver should be adjusted to: 10 * Log10(TB / (TTI * 3.84 * 106)) + Eb/N0 [dB].

c.	For HS-DPCCH: Adjust the test signal mean power so the calibrated radiated Ec/N0 value at the BS receiver is as specified in requirement’s specific subclause 8 of TS 25.141 [10]. To achieve the specified Ec/N0, the ratio of the wanted signal level relative to the AWGN signal at the BS input should be adjusted to: Ec/N0 [dB].

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level quoted in the test procedure of TS 25.141 [10] minus ΔOTAREFSENS. Example of the AWGN levels for the UTRA FDD requiremetns, with the ΔOTAREFSENS correction are presented in table 8.4.5.2-1.

Table 8.3.5.2-1: AWGN power level at the BS input for UTRA FDD requirements

| BS class | AWGN power level |
| --- | --- |
| Wide Area | - 84 - ΔOTAREFSENS [dBm] / 3.84 MHz |
| Medium Range | - 74 - ΔOTAREFSENS [dBm] / 3.84 MHz |
| Local Area | - 70 - ΔOTAREFSENS [dBm] / 3.84 MHz |

While signal power adjustment, reassure that the Eb/N0 (or Ec/N0) at the BS receiver is not impacted by the noise floor.

9)	If RX diversity is not supported, ensure the polarisation match is achieved among test antenna(s) and the OTA AAS BS under test, in order to maximaze the power at the BS receiver.

10)	For reference channels applicable to the BS, measure the appropriate performance metric for the requirement as described in clause 8 of TS 25.141 [10].

### 8.3.6	Test requirements

Test requirements of radiated performance requirements in single RAT UTRA operation in FDD are the same as defined in TS 25.141 [10], in requirement’s specific “Test requirement” subclauses within 8.2 - 8.13 subcluases, with the exception that shall be derived based on the OTA test procedure, as in subclause 8.3.5.2.

Applicability of radiated test requirements is subject to limitations listed in subclause 8.3.2.

In the referenced test requirements in this section, the term "number of RX antennas" should be replaced by one if a single polarization is transmitted or by two if two polarizations are transmitted.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 8.4	Radiated performance requirements for E-UTRA

### 8.4.1	General

Radiated performance requirements for single RAT E-UTRA operation are specified for the fixed reference channels (FRC) and propagation conditions defined in TS 36.104 [4] annex A and annex B, respectively. The requirements only apply to those FRCs that are supported by the OTA AAS BS.

Unless stated otherwise, radiated performance requirements apply for a single carrier only. Radiated performance requirements for an OTA AAS BS in E-UTRA operation supporting carrier aggregation are defined in terms of single carrier requirements.

In the referred E-UTRA specification TS 36.104 [4], the term "RX antennas" refers to demodulation branches (and not physical antennas).

Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarizations supported by the BS, with the required SNR /SINR (in case of E-UTRA demodulation requirements), or required Eb/N0 (in case of UTRA FDD demodulation requirements) applied separately per polarization.

Only 2RX BS performance requirements apply when OTA AAS BS supports and is tested with dual polarizations, except where requirements are defined with 1RX only (e.g. HST) where the requirements shall be tested with single polarization.

NOTE:	OTA AAS BS can support more than 2 demodulation branches, however OTA conformance testing can only be performed for 1 or 2 demodulation branches.

For tests in subclause 8.4 the transmitters may be OFF.

The performance requirements for High Speed Train conditions are optional.

The performance requirements for UL timing adjustment scenario 2 are optional.

In tests performed with signal generators a synchronization signal may be provided, from the BS to the signal generator, to enable correct timing of the wanted signal.

For E-UTRA performance requirements the SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the subframe.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe.

For enhanced E-UTRA performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

SINR = S / N'

Where:

S is the total signal energy in the subframe.

N’is the summation of the received energy of the strongest interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe. The respective energy of each interferer relative to N’ is defined by its associated DIP value.

In the referenced conducted test requirements from TS 36.141 [12] the method to test describes connection to one or a number of BS antenna connectors. When applying these methods to the OTA AAS BS, connection shall be made to the RIB, based on one or two OTA demodulation branches.

### 8.4.2	Definitions and applicability

Definitions of radiated performance requirements in single RAT E-UTRA operation and their applicability are the same as defined in TS 36.141 [12], in requirement’s specific “Definition and applicability” subclauses within 8.2 - 8.4 subcluases.

The following limitation apply for the radiated performance requirements in single RAT E-UTRA operation:

-	All the radiated performance requirements referred from TS 36.141 [12] are subject to 2Rx limitation in the OTA test setup, as captured in subclause 8.1.1.

-	If the OTA AAS BS does not use polarisation diversity then performance requirements only apply to a single demodulation branch (i.e. 1TX-1RX test setup).

-	If the OTA AAS BS uses polarisation diversity and has the ability to maintain isolation between the performance requirements signals for each of the demodulation branches, then performance requirements can be applied to up to two demodulation branches (i.e. 1TX-2RX test setups).

NOTE:	for the list of BS demodulation requirements which were found to be feasible in OTA test setup with the above 2Rx limitation, refer to TR 37.843 [29], subclause 7.8.

-	A test for a specific channel bandwidth is only applicable if the BS supports it. For a BS supporting multiple channel bandwidths only the tests for the lowest and the highest channel bandwidths supported by the BS are applicable.

-	The performance requirements for High Speed Train conditions are optional.

-	The performance requirements for UL timing adjustment scenario 2 are optional.

-	Performance requirements for coverage enhancments are applicable only to the AAS BS supporting coverage enhancement configured with CEModeA. This applies to the following requirements:

-	Requirements for PUSCH supporting coverage enhancement

-	Requirements for PUSCH supporting Cat-M1 UEs

-	PUCCH performance requirements for supporting Cat-M1 UEs

-	PRACH missed detection, Cat-M1 mode

-	The enhaced performance requirements apply to AAS BS supporting the enhanced performance requirements type A and/or type B.

-	Performance requirements for NB-IoT are not applicable to AAS BS.

For PUSCH performance requiremetns, the FRCs for the throughput performance metric derivation are listed in TS 36.141 [12], annex A.

### 8.4.3	Minimum requirements

Minimum requirements for radiated performance requirements in single RAT E-UTRA operation are the same as defined in TS 36.104 [4], in requirement’s specific “Minimum requirement” subclauses within 8.2 - 8.4 subcluases, subject to limitations listed in subclause 8.4.2.

### 8.4.4	Test purposes

Test purposes for the radiated performance requirements in single RAT E-UTRA operation are the same as defined in TS 36.141 [12], in requirement’s specific “Test purpose” subclauses within subcluases 8.2 - 8.4.

The radiated test shall verify the OTA AAS BS receiver’s ability to achieve requirement’s specific performance metric under defined propagation conditions emulated in OTA test chamber, for a given SNR (or SINR).

### 8.4.5	Method of test

#### 8.4.5.1	Initial conditions

Initial conditions of radiated performance requirements in single RAT E-UTRA operation are the same as defined in TS 36.141 [12], in requirement’s specific “Initial conditions” subclauses within 8.2 - 8.4 subcluases, with the exception, that instead of connecting the BS tester(s) directly to antenna connector(s), either a single or two polarizations should be transmitted via test antenna(s) in the OTA chamber, where each polarization represents a demodulation branch.

The initial conditions for the radiated performance requirements in single RAT E-UTRA operation are generalized as follows:

Test environment:	normal, annex G.2.

RF channels to be tested:	M; see TS 36.141 [12], subclause 4.7.

Direction to be tested: receiver target reference direction (see table 4.10-1, D10.9).

#### 8.4.5.2	Procedure

1)	Place the OTA AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.2.1.

2)	Align the manufacturer declared coordinate system orientation of the OTA AAS BS with the test system.

3)	Set the OTA AAS BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, interference signal(s), multipath fading simulators and/or AWGN generators (depending on the required OTA test procedure) to a test antenna via a combining network in OTA test setup, as shown in appropriate subclause in annex D.2.2.

a.	For enhanced performance requirements and for ACK missed detection for multi user PUCCH format 1a: Interconnect attenuators for relative power setting purposes for all transmitting branches (wanted signal and all interferers, separately).

5)	Apply the conducted performance test procedure appropriate to the requirement as described in subclause 8 of TS 36.141 [12]. Instead of connection via an antenna connector, one of the RX antenna signals should be transmitted on each polarization of the test antenna(s).

6)	The characteristics of the wanted/interfering signal shall be configured according to the corresponding UL reference measurement channel defined in annex A in TS 36.141 [12], and according to additional test parameters listed in respective conducted performance test procedure in TS 36.141 [12]. In case of PUCCH requirements, the characteristics of the wanted signal shall be configured according to TS 36.211 [28].

7)	The multipath fading emulators shall be configured according to the corresponding channel model defined in TS 36.141 [12], annex B.

8)	Adjust the test signal mean power so the calibrated radiated SNR (or SINR) value at the BS receiver is as specified in requirement’s specific subclause 8 of TS 36.141 [12], and that the SNR (or SINR) at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level quoted in the test procedure of TS 36.141 [12] minus ΔOTAREFSENS. Examples of the AWGN levels for the E-UTRA PUSCH as well as for the PUCCH and PRACH requiremetns, with the ΔOTAREFSENS correction are presented in table 8.4.5.2-1 and table 8.4.5.2-2, respectively.

Table 8.4.5.2-1: AWGN power level at the BS input for PUSCH requirements

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -92.7 - ΔOTAREFSENS [dBm] / 1.08 MHz |
| 3 | -88.7 - ΔOTAREFSENS [dBm] / 2.7 MHz |
| 5 | -86.5 - ΔOTAREFSENS [dBm] / 4.5 MHz |
| 10 | -83.5 - ΔOTAREFSENS [dBm] / 9 MHz |
| 15 | -81.7 - ΔOTAREFSENS [dBm] / 13.5 MHz |
| 20 | -80.4 - ΔOTAREFSENS [dBm] / 18 MHz |

Table 8.4.5.2-2: AWGN power level at the BS input for PUCCH and PRACH requirements

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -89.7 - ΔOTAREFSENS [dBm] / 1.08 MHz |
| 3 | -85.7 - ΔOTAREFSENS [dBm] / 2.7 MHz |
| 5 | -83.5 - ΔOTAREFSENS [dBm] / 4.5 MHz |
| 10 | -80.5 - ΔOTAREFSENS [dBm] / 9 MHz |
| 15 | -78.7 - ΔOTAREFSENS [dBm] / 13.5 MHz |
| 20 | -77.4 - ΔOTAREFSENS [dBm] / 18 MHz |

9)	If RX diversity is not supported, ensure the polarisation match is achieved among test antenna(s) and the OTA AAS BS under test, in order to maximaze the power at the BS receiver.

10)	For reference channels applicable to the BS, measure the appropriate performance metric for the requirement as described in clause 8 of TS 36.141 [12].

### 8.4.6	Test requirements

Test requirements of radiated performance requirements in single RAT E-UTRA operation are the same as defined in TS 36.141 [12], in requirement’s specific “Test requirement” subclauses within 8.2 - 8.4 subcluases, with the exception that shall be derived based on the OTA test procedure, as in subclause 8.4.5.2.

Applicability of radiated test requirements is subject to limitations listed in subclause 8.4.2.

In the referenced test requirements in this section, the term "number of RX antennas" should be replaced by one if a single polarization is transmitted or by two if two polarizations are transmitted.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

Annex A (normative):
Test system characterization

The radiated measurement methods for AAS BS conformance testing were described in 3GPP TR 37.842 [7], including descriptions of their limitations and testing applicability.

Annex B (normative):
Calibration

The radiated measurement methods for AAS BS conformance testing were described in 3GPP TR 37.842 [7]. Calibration stage description was covered for each of the identified Test Systems in the measurement's procedure description, which covered two distinct stages: calibration stage and measurement stage. This has been separately covered for transmitter and for receiver requirements.

Annex C (informative):
Test tolerances and derivation of test requirements

# C.1	General

The test requirements explicitly defined in this specification have been calculated by relaxing the minimum requirements of the core specification using the Test Tolerances defined here. When the Test Tolerance is non-zero, the test requirements will differ from the minimum requirements, and the formula used for this relaxation is given in the following tables.

The Test Tolerances are derived from Test System uncertainties.

The Test Tolerances should not be modified for any reason e.g. to take account of commonly known test system errors (such as mismatch, cable loss, etc.).

Note that a formula for applying Test Tolerances is provided for all tests.

NOTE:	OTA test requirements for AAS BS are specified for Normal conditions, only.

# C.2	Measurement of transmitter (OTA)

Table C.2-1: Derivation of Test Requirements (OTA transmitter tests)

| Test | Minimum Requirement in 3GPP TS 37.105 [6] | Test Tolerance (TT) | Test Requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See 3GPP TS 37.105 [6], subclause 9.2 | 1.1 dB, f ≤ 3.0 GHz 1.3 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.2 Radiated transmit power (extreme conditions) | See 3GPP TS 37.105 [6], subclause 9.2 | 2.5 dB, f ≤ 3.0 GHz 2.6 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3.2 OTA Maximum output power | See 3GPP TS 37.105 [6], subclause 9.3.2 | 1.4 dB, f ≤ 3.0 GHz 1.5 dB, 3.0 GHz < f ≤ 4.2 | Formula: Upper limit + TT, Lower limit – TT |
| 6.3.3 OTA E-UTRA DL RS power | See 3GPP TS 37.105 [6], subclause 9.3.3 | 1.3 dB, f ≤ 3.0 GHz 1.5 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4.2 OTA UTRA Inner loop power control in the downlink | See 3GPP TS 37.105 [6], subclause 9.4.2 | 0.1 dB | Formula: Upper limit + TT, Lower limit – TT |
| 6.4.3 OTA Power control dynamic range | See 3GPP TS 37.105 [6], subclause 9.4.3 | 1.1 dB | Formula: Upper limit - TT, Lower limit + TT |
| 6.4.4 OTA Total power dynamic range | See 3GPP TS 37.105 [6], subclause 9.4.4 | 0.4 dB | Formula: limit - TT |
| 6.4.5 OTA IPDL time mask | See 3GPP TS 37.105 [6], subclause 9.4.5 | 0.7 dB | Formula: limit - TT |
| 6.5 OTA Transmit ON/OFF power | See 3GPP TS 37.105 [6], subclause 9.5 | 3.4 dB, f ≤ 3.0 GHz 3.6 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: limit - TT |
| 6.6.2 OTA Frequency Error | See 3GPP TS 37.105 [6] subclause 9.6.2 | 12 Hz | Limit + TT |
| 6.6.3 OTA Time alignment error | See 3GPP TS 37.105 [6], subclause 9.6.3 | 25 ns | Limit + TT |
| 6.6.4 OTA Modulation quality | See 3GPP TS 37.105 [6], subclause 9.6.3 | 1 % | Limit + TT |
| 6.7.2 OTA occupied bandwidth | See 3GPP TS 37.105 [6], subclause 9.7.2 | 0 Hz | Limit |
| 6.7.3 OTA Adjacent Channel Leakage power Ratio | See 3GPP TS 37.105 [6], subclause 9.7.3 | ACLR / CACLR 1.0 dB, f ≤ 3.0 GHz 1.2 dB, 3.0 GHz < f ≤ 4.2 Absolute limit 0 dB | Limit + TT |
| 6.7.4 OTA Spectrum emission mask | See 3GPP TS 37.105 [6], subclause 9.6.3 | 1.8 dB, f ≤ 3.0GHz 2.0 dB, 3.0GHz < f ≤ 4.2GHz FFS | Limit + TT |
| 6.7.5 OTA Operating band unwanted emission | See 3GPP TS 37.105 [6], subclause 9.6.3 | close to carrier (<10MHz) 1.8 dB, f ≤ 3.0GHz 2.0 dB, 3.0GHz < f ≤ 4.2GHz far from carrier ( ≥10MHz) 0dB | Limit + TT |
| 6.7.6.2 OTA Transmitter spurious emissions, Mandatory Requirements | See 3GPP TS 37.105 [6], subclause 9.7.6.2.1, 9.7.6.3.1 and 9.7.6.4.1 | 0dB | FFS |
| 6.7.6.3 Transmitter spurious emissions, Protection of BS receiver | See 3GPP TS 37.105 [6], subclause 9.7.6.2.2, 9.7.6.3.2 and 9.7.6.4.2 | 3.1 dB, f ≤ 3.0GHz 3.3 dB, 3.0GHz < f ≤ 4.2GHz | Limit + TT |
| 6.7.6.4 Transmitter spurious emissions, Additional spurious emission requirements | See 3GPP TS 37.105 [6], subclause 9.7.6.2.3, 9.7.6.3.3 and 9.7.6.4.3 | 2.6 dB, f ≤ 3.0GHz 3.0 dB, 3.0GHz < f ≤ 4.2GHz For co-existence with PHS and public safety bands. 0 dB | Limit + TT |
| 6.7.6.5 Transmitter spurious emissions, Co-location | See 3GPP TS 37.105 [6], subclause 9.7.6.2.4, 9.7.6.3.4 and 9.7.6.4.4 | 3.1 dB, f ≤ 3.0GHz 3.3 dB, 3.0GHz < f ≤ 4.2GHz | Limit + TT |
| 6.8 OTA Transmitter intermodulation | See 3GPP TS 37.105 [6] | 0dB |  |

# C.3	Measurement of receiver (OTA)

Table C.2-1: Derivation of Test Requirements (OTA receiver tests)

| Test | Minimum Requirement in 3GPP TS 37.105 [6] | Test Tolerance | Test Requirement in the present document |
| --- | --- | --- | --- |
| 7.2 OTA sensitivity | See 3GPP TS 37.105 [6], subclause 10.2 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Declared Minimum EIS + TT |
| 7.3 OTA Reference Sensitivity | See 3GPP TS 37.105 [6], subclause 10.3 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: EISREFSENS + TT |
| 7.4 OTA Dynamic range | See 3GPP TS 37.105 [6], subclause 10.4 | 0.3 dB | Formula: Wanted signal power + TT. Interferer signal power unchanged. |
| 7.5 OTA Adjacent channel selectivity, general blocking, and narrowband blocking | See 3GPP TS 37.105 [6], subclause 10.5 | 0 dB | Formula: Wanted signal power + TT. Interferer signal power unchanged. |
| 7.6 OTA Blocking - in-band | See 3GPP TS 37.105 [6], subclause 10.6 | 0 dB | Formula: Wanted signal power + TT. Interferer signal power unchanged. |
| 7.6 OTA Blocking - out of band | See 3GPP TS 37.105 [6], subclause 10.6 | 0 dB |  |
| 7.6 OTA blocking – co-location | See 3GPP TS 37.105 [6], subclause 10.6 | 0 dB | Formula: Wanted signal power unchanged Interferer signal power - TT. |
| 7.7 Receiver spurious emissions | See 3GPP TS 37.105 [6], subclause 10.7 | 2.5 dB. 30MHz < f ≤ 6 GHz 4.2 dB, 6 GHz < f ≤ 19 GHz | Formula: Wanted signal power + TT. |
| 7.8 OTA Receiver intermodulation | See 3GPP TS 37.105 [6], subclause 10.8 | 0 dB | Formula: Wanted signal power + TT. Interferer signal power unchanged. |
| 7.9 OTA In-channel selectivity | See 3GPP TS 37.105 [6], subclause 10.9 | 1.7 dB, f ≤ 3.0 GHz 2.1 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Wanted signal power + TT. Interferer signal power unchanged. |

Annex D (informative):
Test system set-up

# D.1	Transmitter

## D.1.1	Radiated Transmit Power, OTA E-UTRA DL RS power, output power dynamics and Transmitter signal quality

Figure D.1.1-1: Measurement set up for Radiated Transmit Power, OTA E-UTRA DL RS power, output power dynamics, Transmitter signal quality

The OTA chamber shown in figure D.1.1-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc..) .

## D.1.2	OTA Base Station output power, ACLR, OTA spectrum emissions mask, OTA operating band unwanted emissions

Figure D.1.2-1: Measurement set up for OTA Base Station output power, ACLR, OTA spectrum emissions mask, OTA operating band unwanted emissions

## D.1.3	OTA spurious emissions

Figure D.1.3-1: Measurement set up for OTA spurious emissions

## D.1.4	OTA Co-location emissions, TX OFF power

Figure D.1.4-1: Measurement set up for OTA Co-location emissions, TX OFF power

## D.1.5	OTA Transmitter Intermodulation

Figure D.1.5-1: Measurement set up for OTA Transmitter intermodulation

# D.2	Receiver

## D.2.1	OTA sensitivity and OTA Reference sensitivity

Figure D.2.1-1: Measurement set up for OTA sensitivity and OTA reference sensitivity

The OTA chamber shown in figure D.2.1-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc..).

## D.2.2	OTA Dynamic range

Figure D.2.2-1: Measurement set up for OTA Dynamic range

The OTA chamber shown in figure D.2.2-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc..).

## D.2.3	OTA Adjacent channel selectivity, general blocking, and narrowband blocking

Figure D.2.3-1: Measurement set up for OTA ACS and narrowband blocking

Test

The OTA chamber shown in figure D.2.3-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc..).

Figure D.2.3-2: Measurement set up for OTA general blocking

The OTA chamber shown in figure D.2.3-2 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc..).

## D.2.4	OTA Blocking

Figure D.2.4-1: Measurement set up for OTA Blocking

The OTA chamber shown in figure D.2.4-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc..).

Figure D.2.4-2: Measurement set up for OTA co-location blocking

## D.2.5	OTA Receiver spurious emissions

Figure D.2.5-1: Measurement set up for OTA receiver spurious emissions

## D.2.6	OTA Receiver intermodulation

Figure D.2.6-1: Measurement set up for OTA receiver intermodulation

The OTA chamber shown in figure D.2.6-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc..).

## D.2.7	OTA In-channel selectivity

Figure D.2.7-1: Measurement set up for OTA In-channel selectivity

The OTA chamber shown in figure D.2.7-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc..).

Annex E (normative):
Estimation of Measurement Uncertainty

# E.1	General

Common measurement uncertainty budget calculation principle, was described in 3GPP TR 37.842 [7].

# E.2	Measurement methodology descriptions

The AAS BS radiated measurement method descriptions, including description of the calibration stage, and the measurement stage, are described separately for each of the OTA tests, i.e. for radiated transmit power and for OTA sensitivity test in 3GPP TR 37.842 [7].

# E.3	Measurement uncertainty budget format

Each test methodology is characterized by different uncertainty contributors. Moreover, within single test methodology, certain uncertainty contributors will need to be modified, depending on the transmitter, or receiver test. Therefore, separate uncertainty budget format were collected for all considered Test Systems in 3GPP TR 37.842 [7], specifying uncertainty contributors for calibration stage, as well as measurement stage.

# E.4	Measurement uncertainty budgets

Descriptions of uncertainty assessment for radiated transmit power and for OTA sensitivity test were described separately for each of the considered Tests Systems as in 3GPP TR 37.842 [7].

# E.5	Measurement error contribution descriptions

Detailed descriptions of the uncertainty contributors for the radiated transmit power and OTA sensitivity test were described separately for each of the considered Tests Systems as in 3GPP TR 37.842 [7].

Annex F (normative): 
TRP measurement grids

# F.1	General

The annex describes various sampling grids and procedures for OTA TRP performance of AAS BS.

# F.2	Spherical equal angle grid

## F.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in F.2.2.

## F.2.2	Reference angular step criteria

The reference angular steps Δϕ and Δθ in are radians are defined as

Δϕref=min⁡(λDcyl,[π12])

Δθref=⁡(λD,[π12])

Dcyl and D are calculated as

DcylDcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in Figure F2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure F2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

In the case of Uniform Linear Array (ULA)  and the EUT is mounted along the yz plane as shown in Figure F2.2-2,the reference angular step can be determined by

∆θref=180°πarcsin⁡(λ/Dz)

Δϕref=180°πarcsin⁡(λDy)

Where Dy is the length of radiating  parts of EUT along y-axis, Dz is the length of radiating parts of EUTalong the z-axis and  is wavelength for the measured frequency.

Figure F2.2-2. Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor )of the grid is defined as

Where and  are the actual angular steps used in the measurement.

# F.3	Spherical equal area grid

TRPEstimate is defined as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified as

N ≥4π∆θ∆ϕ.

The sampling intervals Δθ and Δϕ are described in F.2.2. Each (θn,ϕn) is a sampling point.

# F.4	Spherical Fibonacci grid

TRPEstimate is defined as

TRPEstimate=1Ii=1IEIRP(θi,ϕi)

I is the total number of samples and specified as

N ≥4π∆θ∆ϕ

The sampling intervals Δθ and Δϕ are described in F.2.2. Each (θi,ϕi) is a sampling point.

# F.5	Orthogonal cut grid

EIRPav,cut-n=1Pj=1PEIRP(j)

TRPEstimate=1Nn=1NEIRPav,cut-n

Here, at least two cuts (default) shall be used, an optional third cut can be used. The alignment of the cuts must be along the symmetry planes of the antenna array. No alignment is required for spurious emissions.

When alignment is required:

1.	The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

2.	The second mandatory is a vertical cut passing through the peak direction of the main beam. Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

3.	The third optional cut is a vertical cut orthogonal to the first and the second cut.

When alignment is not required, the cuts can be aligned arbitrarily.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points in the cut. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the intersection points are measured multiple times and the repeated values can be removed from the samples before averaging.

When two cuts measurements are used, a conditional pattern multiplication can be applied. The following are the conditions for applying pattern multiplication:

i.	The vertical cut (and the main beam) is in the xz -plane

ii.	The frequency of the emission is within the downlink operating band.

iii.	The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

iv.	The emission appears/disappears when the Tx power is turned on/off.

v.	The antenna arrays of the EUT

      - 1.	Have rectangular grids of antenna element positions

      - 2.	Have symmetry planes that are vertical and horizontal.

      - 3.	Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRPcut1ϕ at θ=θH and a vertical cut with data EIRPcut2θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemispheres. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in Fig. F.1.2.-1 the u and v coordinates are defined as

u=sinθsinϕv=cosθ

Note that only the data on the cuts are measured.

Calculate power density/EIRP values outside the two cardinal cuts as

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

Note:	the numerical singularity at u2+v2=1 must be treated with care, e.g. by change of variables.

# F.6	Wave vector space grid

If EUT is mounted along the yz plane as shown in Figure F2.2-1, ,the reference step in wave vector space can be determined by

Δugrid=λDy

Δvgrid=λDzwhere Dy is the length of radiating  parts of EUT along  y-axis, Dz is the length of radiating parts of EUT along the z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be  represented as following:

u=sinθsinϕ, v=cos⁡(θ)

The total radiated power (TRP) in the wave vector space is determined by

TRP=ΔugridΔvgrid4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor)of the grid is defined as

Where and  are the actual steps used in the wave vector space in the measurement.

# F.7	Orthogonal 2 cuts with pattern multiplication

This method can be used when the antenna symmetries are compatible with pattern multiplication, see Subclause F.1.2.4. The procedure is as follows:

1.	Calculate the reference angular steps as described in Subclause F.1.2.

2.	Align the EUT to allow for proper pattern multiplication. See Subclause F.5. Measure EIRP on two orthogonal cuts with steps smaller or equal to the reference steps according to step 1.

3.	Apply pattern multiplication according to Subclause F.5 to extrapolate the two cuts data to full-sphere.

4.	Apply numerical integration to obtain the TRP estimate as described in Subclause F.5.

# F.8	Orthogonal 2 or 3 cut with dense sampling

The procedure is as follows:

1.	Follow steps described in F.5 and calculate the TRP estimate.

2.	Add the appropriate correction factor ΔTRP according to Table F.8-1 to ensure overestimation with 95% confidence.

3.	Compare the (TRP estimate + ΔTRP) to the limit.

4.	If the (TRP estimate + ΔTRP) is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 3.

Table F.8-1: The correction factor for two or three cuts dense sampling

|  | Three cuts | Two cuts |
| --- | --- | --- |
| Correction factor ΔTRP (dB) | [2.0] | [2.5] |

# F.9	Full sphere with sparse sampling

The procedure is as follows:

1.	Set the angular grid:

  - a.	Non-harmonic frequencies: choose the angular steps Δϕ and Δθ smaller than or equal to [15] degrees. Calculate the sparsity factor (SF) as

SF=maxΔϕΔϕref,ΔθΔθref

and the correction factor as:

ΔTRP=SF-1SFmax-1⋅1.0 dB,

where SFmax corresponds to 15 degrees angular step. If the sparsity factor is smaller than 1, the correction factor ΔTRP is 0 dB.

  - b.	Harmonic frequencies with fixed beam test signal: choose the angular steps smaller than or equal to the reference angular steps Δϕref and Δθref. Correction factor ΔTRP is 0 dB.

c.	[Harmonic frequencies with beam sweeping test signal: set the angular steps to [15] degrees. Correction factor is ΔTRP 0 dB].

2.	Apply a suitable numerical integration to calculate the TRP estimate.

3.	Add the appropriate correction factor ΔTRP according to step 1 to ensure an overestimation with 95% confidence.

4.	Compare the (TRP estimate + ΔTRP) with the limit. If the (TRP estimate + ΔTRP) is above the limit, choose a smaller angular step and repeat steps 2-4. If the sparsity factor is less than one, no significant improvement of accuracy is expected.

# F.10	Beam-based directions

Beam-based direction can be used if directivity of the EUT antenna is known for the base station operating band. TRPEstimate is defined as

TRPEstimate=EIRPpeakDEUT,

where EIRPpeak is the maximum EIRP in the beam peak direction within a particular beam direction pair and DEUT is directivity of the EUT antenna.

# F.11	Peak method

The peak method can be used when frequencies with unwanted peak emissions are identified during pre-scan. The method does not provide an estimate of TRP.

For each peak emission frequency identified during pre-scan, measure peak EIRP or power density as follows:

1.	Move EUT and test antenna to the same position where the peak emission is recorded during the pre-scan.

2.	Move the EUT around the position and test antenna orientation to find the final peak EIRP or power density.

3.	The measured peak power density or EIRP shall be used to demonstrate conformance.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarised components.

# F.12	Equal sector with peak average

Equal sector with peak average can be performed on frequencies with unwanted peak emission, which are considered by the peak method for further measurements.

The spherical angle ϕ is divided into K equal sectors. If the largest dimension of EUT is less than 60 cm, then each sector is a half quadrant of 45°.

For each peak emission frequency, measure peak EIRP of beams belonging to different sectors of the sphere as follows:

1.	Move EUT and test antenna to the same position where the emission peak is recorded during the pre-scan.

2.	Move EUT around the position and test antenna orientation to find the final peak EIRP.

3.	Repeat Steps 1 to 2 until all sectors are covered.

4.	Calculate TRPEstimate as

TRPEstimate=1Kk=1KEIRPk,

where EIRPk is the peak EIRP in the kth sector.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarised components.

# F.13	Pre-scan

Pre-scan is used to identify frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP.

The procedure for pre-scan is as follows:

1.	Scan the entire surface around EUT.

2.	Rotate test antenna to cover all possible polarisations of emissions to detect maximum emissions.

3.	Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

4.	Emissions which are [20 dB] or more below the specified limit shall not require further measurements.

Annex G (normative): 
Environmental requirements for the BS equipment

# G.1	General

For each test in the present document, the environmental conditions under which the AAS BS is to be tested are defined. The environmental conditions and class shall be from the relevant IEC specifications or the corresponding ETSI specifications.

For OTA requirements where it is not possible to environmentally control the entire calibrated OTA chamber either localised control of the AAS BS hardware or alternative OTA measurements which are then related to the original specification are acceptable.

# G.2	Normal test environment

When a normal test environment is specified for a test, the test should be performed within the minimum and maximum limits of the conditions stated in table G.2-1.

Table G.2-1: Limits of conditions for Normal Test Environment

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | 86 kPa | 106 kPa |
| Temperature | 15C | 30C |
| Relative Humidity | 20 % | 85 % |
| Power supply | Nominal, as declared by the manufacturer |  |
| Vibration | Negligible |  |

The ranges of barometric pressure, temperature and humidity represent the maximum variation expected in the uncontrolled environment of a test laboratory. If it is not possible to maintain these parameters within the specified limits, the actual values shall be recorded in the test report.

# G.3	Extreme test environment

## G.3.1	General

The manufacturer shall declare one of the following:

1)	the equipment class for the equipment under test, as defined in the IEC 60721-3-3 [21] or ETSI EN 300 019-1-3 [23] ("Stationary use at weather protected locations");

2)	the equipment class for the equipment under test, as defined in the IEC 60721-3-4 [22] or ETSI EN 300 019-1-4 [24] ("Stationary use at non weather protected locations");

3)	the equipment that does not comply to the mentioned classes, the relevant classes from IEC 60721 [20] documentation for Temperature, Humidity and Vibration shall be declared.

NOTE:	Reduced functionality for conditions that fall outside of the standard operational conditions are not tested in the present document. These may be stated and tested separately.

## G.3.2	Extreme temperature

When an extreme temperature test environment is specified for a test, the test shall be performed at the standard minimum and maximum operating temperatures defined by the manufacturer's declaration for the equipment under test.

Minimum temperature:

The test shall be performed with the environment test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60068-2-1 [25].

Maximum temperature:

The test shall be performed with the environmental test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60068-2-2 [26].

NOTE:	It is recommended that the equipment is made fully operational prior to the equipment being taken to its lower operating temperature.

# G.4	Vibration

When vibration conditions are specified for a test, the test shall be performed while the equipment is subjected to a vibration sequence as defined by the manufacturer's declaration for the equipment under test. This shall use the environmental test equipment and methods of inducing the required environmental phenomena in to the equipment, conforming to the test procedure of IEC 60068-2-6 [27]. Other environmental conditions shall be within the ranges specified in annex G.2.

NOTE:	The higher levels of vibration may induce undue physical stress in to equipment after a prolonged series of tests. The testing body should only vibrate the equipment during the RF measurement process.

# G.5	Power supply

When extreme power supply conditions are specified for a test, the test shall be performed at the standard upper and lower limits of operating voltage defined by manufacturer's declaration for the equipment under test.

Upper voltage limit:

The equipment shall be supplied with a voltage equal to the upper limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60068-2-1 [25] Test Ab/Ad and IEC 60068-2-2 [26] Test Bb/Bd: Dry Heat.

Lower voltage limit:

The equipment shall be supplied with a voltage equal to the lower limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60068-2-1 [25] Test Ab/Ad and IEC 60068-2-2 [26] Test Bb/Bd: Dry Heat.

# G.6	Measurement of test environments

The measurement accuracy of the BS test environments shall be:

Pressure:	5 kPa.

Temperature:	2 degrees.

Relative Humidity:	5 %.

DC Voltage:	1,0 %.

AC Voltage:	1,5 %.

Vibration:	10 %.

Vibration frequency:	0,1 Hz.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

# G.7	OTA extreme test methods

## G.7.1	Direct far field method

The AAS BS under test is placed inside a sealed RF transparent environmental enclosure , as showed in Figure G.7.1-1. This is connected to an environment control system which regulates the temperature inside the enclosure. The remaining equipment inside the OTA chamber (any suitable antenna test range chamber type is acceptable) is outside the environmental control and is at nominal temperature. Positioners, test antennas and all other OTA test equipment do not need to be specified over the extreme temperature range.

Figure G.7.1-1: Measurement set up for Extreme conditions for EIRP accuracy using direct far field method

The prescence of the environmental chamber inside the OTA chamber may affect the measurement accuracy due to additional reflections and refractions, also the loss through the environmental enclosure may not be consistant with direction as the path through the radome may vary with angle. Hence the system should be calibrated in all tested directions.

NOTE.	Currently only a single direction is specified for extreme testing so a single calibration directipon is sufficient.

Conformance may be demonstated by measuring the diference between the nominal measurement and the extreme measurement (Δsample) or by measuring Pmax,c,EIRP, extreme directly.

As the measurement is done in the far field (or measure in near field transformed to far field)

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

## G.7.2	Relative method

The AAS BS under test is placed inside a small (compared to a far field chamber) anachoic chamber which is both RF a screened and suitable for environmental conditioning. The RF conditionals inside the chamber are absorbative and capable of dispating the power the the AAS BS when radiating. A sample antenna or RF probe are placed in a location which gives a sample of the main beam EIRP but does not have to accuarctly measure the EIRP directly, instead the near-field response is measured. For this method test components are exposed to the full temperature range for example the test antenna/probe, cables, absorbers etc. may change as a function of temperature.

Using the relative method it is also necassary to measure the EIRP under nominal conditions using an appropriately calibrated far field (or near field) test range to obtain Pmax,c,EIRP.

Figure F.7.2-1: Measurement set up for Extreme conditions for EIRP accuracy using difference method

Measurements from the test antenna/ probe are taken under nominal conditions and extreme conditions to calculate (Δsample). The difference between the nominal and extreme conditions (Δsample) is then used along with the nominal EIRP measurement (Pmax,c,EIRP) made in the appropriate far field or near field chamber and compared against the extreme requirement. As follows:

Pmax,c,EIRP, extreme = Pmax,c,EIRP + Δsample.

To conserve test time it is beneficial to measure two orthogonal polarizations and add the result together. Otherwise, each polarization must be measured separately including a polarization matching procedure.

Annex H (informative): 
Measuring noise close to noise-floor

As the emission level seen by the meausurement receiver (PUEM) for co-location requirements are very low, it is suggested to measure relative noise change instead of absolute noise level. The relations between measured noise change 1, noise floor N0 and the relation to PUEM with respect to the noise floor denoted 2 is visualized in Figure H-1.

Figure H-1: Relative noise measurement

The absolute emission level in decibel scale is calculated as:

, where N0 is the noise floor of the measurement receiver and 2 is plotted s function of 1 in Figure H-1. The absolute noise floor of the measurement receiver, including probe antenna, cables, filter and LNA is determined by a calibration procedure. The calibration will determine the absolute emission level (N0) accuracy of measuring out-of-band unwanted emission close to the thermal noise floor.

Annex I (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-02 | RAN4#78 | R4-161118 |  |  |  | Specification structure | 0.1.0 |
| 2016-04 | RAN4#78bis | R4-162524 |  |  |  | R4-161370 - TP for TS 37.145-2: Adding Annex for relations between core and conformance requirements | 0.2.0 |
| 2016-05 | RAN4#79 | R4-164927 |  |  |  | R4-164717 - TP to TS 37.145 (part 2) sections 1-5 R4-164718 - TP for TS 37.145-2: Addition of radiated transmit power conformance test requirement in clause 6 R4-164719 - TP to TS 37.145 (part 2) section 7 - Radiated receiver characteristics | 0.3.0 |
| 2016-08 | RAN4#80 | R4-167179 |  |  |  | R4-166422 - TP to 3GPP TS 37.145-2 - clean up R4-166218 - TP for TS 37.145-2: Editorial correction on table numbers R4-166567 - TP to TS 37.145-2: Manufacturer declarations consistency improvements R4-166938 - TP for TS 37.145-2: Improvements of text in subclause 4.8 R4-166940 TP for TS 37.145-2: Improvements on text relating to the reference coordinate system R4-166939 TP to TR 37.145-2: Test Requirements derivation: Annexes A, B, C, E R4-166929 TP to TS 37.145-2: Measurement uncertainties and TT values R4-166931 TP for TS 37.145-2 - On OTA RX sensitivity requirements for AAS R4-167191 TP to TR 37.145-2: definition and test procedure clarifications for EIRP and EIS | 0.4.0 |
| 2016-09 | RP-73 | RP-161450 |  |  |  | Editorial corrections after review by ETSI editHelp | 1.0.0 |
| 2016-09 | RP-73 |  |  |  |  | TS was approved by RAN plenary | 13.0.0 |
| 2016-12 | RP-74 | RP-162422 | 0003 | - | A | Correction of Manufacturer declaration description list in TS 37.145-2 | 13.1.0 |
| 2017-03 | RP-75 | RP-170586 | 0005 | 1 | F | TS 37.145-2: Clarification of test procedure for radiated transmit power | 13.2.0 |
| 2017-03 | RP-75 | RP-170586 | 0006 | 1 | F | TS 37.145-2: Corrections | 13.2.0 |
| 2017-03 | RP-75 | - | - | - | - | Update to Rel-14 version (MCC) | 14.0.0 |
| 2017-06 | RP-76 | RP-171306 | 0011 |  | A | CR to TS 37.145-2: Isolation of Band 46 from the AAS BS specification | 14.1.0 |
| 2017-09 | RP-77 | RP-171968 | 0013 |  | A | CR to TS 37.145-2: Editorial corrections, Rel-14 | 14.2.0 |
| 2017-12 | RAN#78 | RP-172608 | 0015 | 1 | A | Correction of maximum steering directions declarations | 14.3.0 |
| 2018-03 | RAN#79 | RP-180280 | 0017 |  | A | CR to TS 37.145-2: corrections to the EIS test requirements (7.2) Rel-14 | 14.4.0 |
| 2018-06 | RAN#80 | RP-181109 | 0019 |  | A | CR to TR 37.145-2: Clarifications on OTA sensitivity requirement (7.2.1) | 14.5.0 |
| 2018-06 | SA#80 |  |  |  |  | Update to Rel-15 version (MCC) | 15.0.0 |
| 2018-09 | RAN#81 | RP-181907 | 0034 |  | B | CR to TS 37.145-2 | 15.1.0 |
| 2018-12 | RAN#82 | RP-182387 | 0035 | 1 | F | CR to TS 37.145-2 Correction on OTA test requirements | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0040 | 1 | F | CR to TS 37.145-2: CLTA definition | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0041 | 1 | F | CR to TS 37.145-2: Clarification on CLTA related MU | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0042 | 2 | F | CR to TS 37.145-2: Corrections on OTA Transmit ON/OFF power | 15.2.0 |
| 2018-12 | RAN#82 | RP-182380 | 0046 |  | A | CR to TS 37.145-2: corrections of declarations for the Radiated Transmit Power, Rel-15 | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0047 |  | F | CR to TS 37.145-2: correction of the "EIRP accuracy directions set" into "OTA peak directions set" | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0052 |  | F | CR to TS 37.145-2: terminology corrections for "Minimum requirements", Rel-15 | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0054 | 1 | F | CR to TS 37.145-2: Clarification on demodulation requirements (8.1) | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0055 | 1 | F | CR to TS 37.145-2: OTA Adjacent Channel Leakage Ratio (6.7.3) and OTA Operating band unwanted emissions (6.7.5) - corrections to text and tables | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0056 | 1 | F | CR to TS 37.145-2: adding TRP measurement procedures in Annex F. | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0060 | 1 | F | CR to 37.145-2: MU clarifications | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0061 | 1 | F | CR to 37.145-2: Corrections to co-location requirements | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0063 |  | F | CR to TS 37.145-2: UTRA TDD removal | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0064 | 1 | F | CR to TS 37.145-2: fix for the EUTRA demodulation requirements | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0065 | 1 | F | Cleanup to OTA requirements text | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0066 | 1 | F | Correction to RX receiver test directions | 15.2.0 |
| 2018-12 | RAN#82 | RP-182362 | 0067 | 1 | B | Introduction of NR to 37.145-2 | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0068 | 2 | F | CR to TS 37.145-2 - polarisation wording improvements for OTA reference sensitivity | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0070 |  | F | CR to TS 37.145-2 Removal of referencing error for in-band blocking | 15.2.0 |