3GPP TS 36.133 V14.11.0 (2019-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	78

1	Scope	79

2	References	79

3	Definitions, symbols and abbreviations	80

3.1	Definitions	80

3.2	Symbols	82

3.3	Abbreviations	83

3.4	Test tolerances	85

3.5	Additional notation	85

3.5.1	Groups of bands	85

3.6	General	87

3.6.1	Applicability of requirements in this specification version	87

4	E-UTRAN RRC_IDLE state mobility	93

4.1	Cell Selection	93

4.2	Cell Re-selection	93

4.2.1	Introduction	93

4.2.2	Requirements	93

4.2.2.1	Measurement and evaluation of serving cell	94

4.2.2.2	Void	95

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	95

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	97

4.2.2.5	Measurements of inter-RAT cells	99

4.2.2.5.1	Measurements of UTRAN FDD cells	99

4.2.2.5.2	Measurements of UTRAN TDD cells	101

4.2.2.5.3	Measurements of GSM cells	102

4.2.2.5.4	Measurements of HRPD cells	103

4.2.2.5.5	Measurements of cdma2000 1X	104

4.2.2.6	Evaluation of cell re-selection criteria	105

4.2.2.7	Maximum interruption in paging reception	106

4.2.2.8	void	106

4.2.2.9	UE measurement capability	106

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	107

4.2.2.10	Reselection to CSG cells	107

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	107

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	108

4.2.2.11	Void	109

4.2.2.12	Void	109

4.2.2.13	Void	109

4.3	Minimization of Drive Tests (MDT)	109

4.3.1	Introduction	110

4.3.2	Measurements	110

4.3.2.1	Requirements	110

4.3.3	Relative Time Stamp Accuracy	110

4.3.3.1	Requirements	110

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	110

4.3.4.1	 Requirements	110

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	111

4.3.5.1	Requirements for timeSinceFailure	111

4.4	MBSFN Measurements	111

4.4.1	Introduction	111

4.4.2	MBSFN RSRP measurements	111

4.4.3	MBSFN RSRQ measurements	111

4.4.4	MCH BLER measurements	112

4.5	Proximity-based Services	112

4.5.1	Introduction	112

4.5.2	Requirements	112

4.5.2.1	Interruptions with ProSe Direct Discovery	112

4.5.2.2	Interruptions with ProSe Direct Communication	112

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	112

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	113

4.6	Cell Selection and Re-selection Requirements for UE category NB1	113

4.6.1	Cell Selection	113

4.6.2	Cell Re-selection	113

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	114

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	114

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	116

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	116

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	118

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	119

4.6.2.7	Maximum interruption in paging reception in normal coverage	120

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	120

4.6.2.8	UE measurement capability	121

4.7	Cell Selection and Re-selection Requirements for UE category M1	121

4.7.1	Cell Selection	121

4.7.2	Cell Re-selection	121

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	121

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	121

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	121

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	123

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	124

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	124

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	125

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	125

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	126

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	127

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	129

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	129

4.8	Idle State Positioning Measurement Requirements for UE category NB1	130

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	130

4.8.1.1	RSTD Measurement Reporting Delay	131

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	132

4.8.2.1	RSTD Measurement Reporting Delay	133

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	133

4.8.3.1	RSTD Measurement Reporting Delay	135

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	135

4.8.4.1	RSTD Measurement Reporting Delay	136

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	137

4.8.5.1	Measurement Reporting Delay	138

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	138

4.8.6.1	Measurement Reporting Delay	139

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	139

4.8.7.1	Measurement Reporting Delay	140

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	141

4.8.8.1	Measurement Reporting Delay	142

5	E-UTRAN RRC_CONNECTED state mobility	142

5.1	E-UTRAN Handover	143

5.1.1	Introduction	143

5.1.2	Requirements	143

5.1.2.1	E-UTRAN FDD – FDD	143

5.1.2.1.1	Handover delay	143

5.1.2.1.2	Interruption time	144

5.2.2.2	E-UTRAN FDD – TDD	145

5.2.2.2.1	(Void)	145

5.2.2.2.2	(Void)	145

5.2.2.3	E-UTRAN TDD – FDD	145

5.2.2.3.1	(Void)	145

5.2.2.3.2	(Void)	145

5.2.2.4	E-UTRAN TDD – TDD	145

5.2.2.4.1	Handover delay	145

5.2.2.4.2	Interruption time	146

5.2.2.5	E-UTRAN HD–FDD	147

5.2.2.5.1	Handover delay	147

5.2.2.5.2	Interruption time	147

5.3	Handover to other RATs	149

5.3.1	E-UTRAN - UTRAN FDD Handover	149

5.3.1.1	Introduction	149

5.3.1.1.1	Handover delay	149

5.3.1.1.2	Interruption time	149

5.3.2	E-UTRAN - UTRAN TDD Handover	150

5.3.2.1	Introduction	150

5.3.2.2	Requirements	150

5.3.2.2.1	Handover delay	150

5.3.2.2.2	Interruption time	150

5.3.3	E-UTRAN - GSM Handover	151

5.3.3.1	Introduction	151

5.3.3.2	Requirements	151

5.3.3.2.1	Handover delay	151

5.3.3.2.2	Interruption time	151

5.4	Handover to Non-3GPP RATs	151

5.4.1	E-UTRAN – HRPD Handover	151

5.4.1.1	Introduction	151

5.4.1.1.1	Handover delay	152

5.4.1.1.2	Interruption time	152

5.4.2	E-UTRAN – cdma2000 1X Handover	152

5.4.2.1	Introduction	152

5.4.2.1.1	Handover delay	152

5.4.2.1.2	Interruption time	152

5.5	E-UTRAN Handover for Cat-M1 UEs	153

5.5.1	Introduction	153

5.5.2	Requirements in CEModeA	153

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	153

5.5.2.1.1	Handover delay	153

5.5.2.1.2	Interruption time	153

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	154

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	154

5.5.2.3.1	Void	154

5.5.2.3.2	Void	154

5.5.3	Requirements in CEModeB	154

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	154

5.5.3.1.1	Handover delay	154

5.5.3.1.2	Interruption time	155

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	155

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	155

5.6	Void	155

6	RRC Connection Mobility Control	155

6.1	RRC Re-establishment	155

6.1.1	Introduction	156

6.1.2	Requirements	156

6.1.2.1	UE Re-establishment delay requirement	156

6.2	Random Access	156

6.2.1	Introduction	156

6.2.2	Requirements	156

6.2.2.1	Contention based random access	157

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	157

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	157

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	157

6.2.2.1.4	Void	157

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	157

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	157

6.2.2.2	Non-Contention based random access	157

6.2.2.2.1	Correct behaviour when receiving Random Access Response	157

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	157

6.2.3	Requirements for Cat-M1 UEs	158

6.3	RRC Connection Release with Redirection	158

6.3.1	Introduction	158

6.3.2	Requirements	158

6.3.2.1	RRC connection release with redirection to UTRAN FDD	158

6.3.2.2	RRC connection release with redirection to GERAN	158

6.3.2.3	RRC connection release with redirection to UTRAN TDD	159

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	159

6.4.1	Introduction	159

6.4.2	Requirements	159

6.5	RRC Re-establishment for NB-IoT UEs	160

6.5.1	Introduction	160

6.5.2	Requirements	160

6.5.2.1	UE Re-establishment delay requirement in normal coverage	160

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	161

6.6	Random Access for UE category NB1	161

6.6.1	Introduction	161

6.6.2	Requirements	161

6.6.2.1	Correct behaviour when receiving Random Access Response reception	161

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	162

6.6.2.3	Correct behaviour when receiving a NACK on msg3	162

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	162

6.6.2.5	Correct behaviour when contention Resolution timer expires	162

6.6.2.6	MSG3-based channel quality report for UE Category NB1	162

6.6.3	Requirements for NPRACH configuration	162

6.7	RRC Re-establishment for Cat-M1 UEs	163

6.7.1	Introduction	163

6.7.2	Requirements	163

6.7.2.1	UE Re-establishment delay requirement for CEModeA	163

6.7.2.2	UE Re-establishment delay requirement for CEModeB	164

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	164

6.8.1	Introduction	164

6.8.2	Requirements	164

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	164

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	165

6.9.1	Introduction	165

6.9.2	Requirements	165

7	Timing and signalling characteristics	166

7.1	UE transmit timing	166

7.1.1	Introduction	166

7.1.2	Requirements	166

7.2	UE timer accuracy	167

7.2.1	Introduction	167

7.2.2	Requirements	167

7.3	Timing Advance	168

7.3.1	Introduction	168

7.3.2	Requirements	168

7.3.2.1	Timing Advance adjustment delay	168

7.3.2.2	Timing Advance adjustment accuracy	168

7.4	Cell phase synchronization accuracy (TDD)	168

7.4.1	Definition	168

7.4.2	Minimum requirements	168

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	169

7.5.1	Introduction	169

7.5.2	eNodeB Synchronization Requirements	169

7.5.2.1	Synchronized E-UTRAN	169

7.5.2.2	Non-Synchronized E-UTRAN	169

7.6	Radio Link Monitoring	169

7.6.1	Introduction	169

7.6.2	Requirements	171

7.6.2.1	Minimum requirement when no DRX is used	171

7.6.2.2	Minimum requirement when DRX is used	171

7.6.2.3	Minimum requirement at transitions	173

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	173

7.6.2.5	Minimum requirement under IDC Interference	173

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	173

7.7.1	Introduction	173

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	173

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	174

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells	174

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	175

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	176

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	176

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	177

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	177

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	177

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	178

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	179

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	179

7.8	Interruptions with Carrier Aggregation	179

7.8.1	Introduction	179

7.8.2	Requirements	180

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	180

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	180

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	180

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	180

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	180

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	181

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	181

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	181

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	181

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	183

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	183

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	183

7.8.2.13	Interruptions at SRS carrier based switching	184

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	184

7.9.1	Introduction	184

7.9.2	Minimum Requirements for Interband Carrier Aggregation	184

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	185

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	185

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	185

7.10.1	Introduction	185

7.10.2	Requirements	185

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	186

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	186

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	186

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	187

7.11	Radio Link Monitoring for UE Category 0	187

7.11.1	Introduction	187

7.11.2	Requirements for FD-FDD and TDD	188

7.11.2.1	Minimum requirement when no DRX is used	188

7.11.2.2	Minimum requirement when DRX is used	189

7.11.2.3	Minimum requirement at transitions	189

7.11.3	Requirements for HD-FDD	190

7.11.3.1	Minimum requirement when no DRX is used	190

7.11.3.2	Minimum requirement when DRX is used	190

7.11.3.3	Minimum requirement at transitions	191

7.12	Interruptions with Dual Connectivity	191

7.12.1	Introduction	191

7.12.2	Requirements	191

7.12.2.1	Interruptions at PSCell addition/release	191

7.12.2.2	Interruptions at transitions between active and non-active during DRX	191

7.12.2.3	Interruptions at transitions from non-DRX to DRX	192

7.12.2.4	Interruptions at SCell addition/release	192

7.12.2.5	Interruptions at SCell activation/deactivation	192

7.12.2.6	Interruptions during measurements on SCC	193

7.12.2.7	Interruptions at SRS carrier based switching	193

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	194

7.13.1	Definition	194

7.13.2	Minimum requirements	194

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	194

7.14.1	Introduction	194

7.14.2	PSCell Addition Delay Requirement	194

7.14.3	PSCell Release Delay Requirement	195

7.15	Maximum Receive Timing Difference in Dual Connectivity	195

7.15.1	Introduction	195

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	195

7.16	Proximity-based Services	195

7.16.1	Introduction	195

7.16.2	Requirements	195

7.16.2.1	ProSe UE transmission timing	195

7.16.2.1.1	Serving cell or PCell as timing reference	196

7.16.2.1.2	SCell or non-serving cell as timing reference	196

7.16.3	Interruptions with ProSe	196

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	196

7.16.3.2	Interruptions at ProSe Direct Communication configuration	196

7.16.3.3	Interruptions during ProSe Direct Discovery	197

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	197

7.16.3.5	Interruptions during ProSe Direct Communication	197

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	198

7.16.4.1	Measurement and evaluation of selected cell	198

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	198

7.16.5	Selection / Reselection of ProSe relay UE	199

7.16.6	ProSe operation under deactivated SCell	199

7.17	Maximum Transmission Timing Difference in Dual Connectivity	199

7.17.1	Introduction	199

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	200

7.18.1	Introduction	200

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	200

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	200

7.19	Radio Link Monitoring for UE Category M1	200

7.19.1	Introduction	200

7.19.2	Requirements for FD-FDD and TDD CE mode A	200

7.19.2.1	Minimum requirement when no DRX is used	201

7.19.2.2	Minimum requirement when DRX is used	202

7.19.2.3	Minimum requirement at transitions	203

7.19.3	Requirements for HD-FDD with CE mode A	203

7.19.3.1	Minimum requirement when no DRX is used	204

7.19.3.2	Minimum requirement when DRX is used	204

7.19.3.3	Minimum requirement at transitions	205

7.19.4	Requirements for FD-FDD and TDD with CE mode B	205

7.19.4.1	Minimum requirement when no DRX is used	206

7.19.4.2	Minimum requirement when DRX is used	207

7.19.4.3	Minimum requirement at transitions	208

7.19.5	Requirements for HD-FDD with CE mode B	208

7.19.5.1	Minimum requirement when no DRX is used	209

7.19.5.2	Minimum requirement when DRX is used	209

7.19.5.3	Minimum requirement at transitions	210

7.20	UE transmit timing for NB-IoT	210

7.20.1	Introduction	210

7.20.2	Requirements	210

7.21	 UE timer accuracy for NB-IoT	211

7.21.1	Introduction	211

7.21.2	Requirements	211

7.22	Timing Advance for NB-IoT	211

7.22.1	Introduction	211

7.22.2	Requirements	211

7.22.2.1	Timing Advance adjustment delay	211

7.22.2.2	Timing Advance adjustment accuracy	212

7.23	Radio Link Monitoring for Category NB1 UE	212

7.23.1	Introduction	212

7.23.2	Requirements for HD-FDD Category NB1 UE	212

7.23.2.1	Minimum requirement when no DRX is used	212

7.23.2.2	Minimum requirement when DRX is used	213

7.23.2.3	Minimum requirement at transitions	213

7.24	UE transmit timing for Category M1	214

7.24.1	Introduction	214

7.24.2	Requirements	214

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	215

7.25.1	Definition	215

7.25.2	Minimum requirements	215

7.26	UE transmit timing for Category M2	215

7.26.1	Introduction	215

7.26.2	Requirements	215

7.27	UE timer accuracy for category M1	216

7.27.1	Introduction	216

7.27.2	Requirements	216

7.28	Timing Advance for Category M1	216

7.28.1	Introduction	216

7.28.2	Requirements	216

7.29	Interruptions requirements with FeMBMS	216

7.29.1	Introduction	216

7.29.2	Requirements	216

7.30	Numerology switching delay requirements with FeMBMS	216

7.30.1	Introduction	216

7.30.2	Requirements	216

8	UE Measurements Procedures in RRC_CONNECTED State	217

8.1	General Measurement Requirements	217

8.1.1	Introduction	217

8.1.2	Requirements	217

8.1.2.1	UE measurement capability	217

8.1.2.1.1	Monitoring of multiple layers using gaps	221

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	222

8.1.2.1.2	Network controlled small gap	223

8.1.2.2	E-UTRAN intra frequency measurements	225

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	225

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	229

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	233

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	234

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	235

8.1.2.3	E-UTRAN inter frequency measurements	235

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	236

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	241

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	248

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	248

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	248

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	249

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	251

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	252

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	253

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	259

8.1.2.4	Inter RAT measurements	259

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	259

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	264

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	264

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	268

8.1.2.4.5	E-UTRAN FDD – GSM measurements	268

8.1.2.4.6	E-UTRAN TDD – GSM measurements	273

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	273

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	275

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	275

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	275

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	276

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	276

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	276

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	276

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	278

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	278

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	278

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	278

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	279

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	279

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	281

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	281

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	281

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	283

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	285

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	286

8.1.2.6.5	(Void)	288

8.1.2.6.6	(Void)	288

8.1.2.6.7	(Void)	288

8.1.2.6.8	(Void)	288

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	288

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	289

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	290

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	292

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	294

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	295

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	297

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	299

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	300

8.1.2.7	E-UTRAN E-CID Measurements	302

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	302

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	303

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	305

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	305

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	306

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	306

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	309

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	312

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	315

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	319

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	319

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	319

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	319

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	320

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	320

8.2.1	Introduction	320

8.2.2	Requirements	321

8.3	Measurements for E-UTRA carrier aggregation	323

8.3.1	Introduction	323

8.3.2	Measurements of the primary component carrier	323

8.3.3	Measurements of a secondary component carrier	323

8.3.3.1	Measurements of a secondary component carrier with active SCell	323

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	324

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	324

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	325

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	326

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	326

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	327

8.4.1	Introduction	327

8.4.2	Measurements on the primary component carrier	327

8.4.3	Measurements on a secondary component carrier	328

8.4.4	Measurements on both primary component carrier and a secondary component carrier	328

8.4.5	Measurements on different secondary component carriers	329

8.5	Measurements for UE category 0	330

8.5.1	Introduction	330

8.5.2 	Requirements	331

8.5.2.1 	E-UTRAN intra frequency measurements	331

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	331

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	334

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	336

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	340

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	341

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	341

8.6	Discovery signal measurements	342

8.6.1	Introduction	342

8.6.2	Requirements for CRS based discovery signal measurements	342

8.6.2.1	E-UTRAN intra frequency measurements	342

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	342

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	345

8.6.2.2	E-UTRAN inter frequency measurements	347

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	348

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	350

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	353

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	353

8.6.3	Requirements for CSI-RS based discovery signal measurements	353

8.6.3.1	E-UTRAN intra frequency measurements	353

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	353

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	356

8.6.3.2	E-UTRAN inter frequency measurements	358

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	358

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	361

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	363

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	364

8.7	Discovery signal measurements for E-UTRA carrier aggregation	364

8.7.1	Introduction	364

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	364

8.7.2.1	Measurements of the primary component carrier	364

8.7.2.2	Measurements of a secondary component carrier	364

8.7.2.3	Measurements of a secondary component carrier with active SCell	364

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	364

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	365

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	366

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	367

8.7.3.1	Measurements of the primary component carrier	367

8.7.3.2	Measurements of a secondary component carrier	367

8.7.3.3	Measurements of a secondary component carrier with active SCell	368

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	368

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	368

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	369

8.8	Measurements for E-UTRA dual connectivity	371

8.8.1	Introduction	371

8.8.2	Intra-frequency measurements requirements on PCell	371

8.8.3	Intra-frequency measurements requirements on PSCell	371

8.8.4	Inter-frequency and inter-RAT measurement requirements	371

8.8.5	Intra-frequency measurements with autonomous gaps	371

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	371

8.8.5.2	ECGI reporting delay	373

8.8.6	Inter-frequency measurements with autonomous gaps	373

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	373

8.8.6.2	ECGI reporting delay	374

8.8.7	SSTD Measurements	374

8.8.7.1	Introduction	374

8.8.7.2	SSTD Measurement requirements	374

8.8.7.3	SSTD Measurement Reporting Delay	374

8.8.8	Intra-frequency measurements requirements on SCell	375

8.9	MBSFN Measurements	375

8.9.1	Introduction	375

8.9.2	MBSFN RSRP Measurements	375

8.9.3	MBSFN RSRQ Measurements	375

8.9.4	MCH BLER Measurements	375

8.10	Proximity-based Services	376

8.10.1	Introduction	376

8.10.2	Requirements	376

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	376

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	376

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	377

8.11.1	Introduction	377

8.11.2	CRS based discovery signal measurements	377

8.11.2.1	E-UTRAN intra-frequency measurements	377

8.11.2.1.1	Requirements	377

8.11.2.1.1.1	Requirements when no DRX is used	377

8.11.2.1.1.1.1	Measurement Reporting Requirements	379

8.11.2.1.1.2	Requirements when DRX is used	380

8.11.2.1.1.2.1	Measurement Reporting Requirements	382

8.11.2.2	E-UTRAN inter-frequency measurements	382

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	382

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	386

8.11.3	CSI-RS based discovery signal measurements	386

8.11.3.1	E-UTRAN intra-frequency measurements	386

8.11.3.1.1	Requirements	387

8.11.3.1.1.1	Requirements when no DRX is used	387

8.11.3.1.1.1.1	Measurement Reporting Requirements	388

8.11.3.1.1.2	Requirements when DRX is used	389

8.11.3.1.1.2.1	Measurement Reporting Requirements	390

8.11.3.2	E-UTRAN inter-frequency measurements	391

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	391

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	395

8.11.4	RSSI measurements	395

8.11.4.1	E-UTRAN intra-frequency measurements	395

8.11.4.2	E-UTRAN inter-frequency measurements	395

8.11.5	Channel occupancy measurements	395

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	395

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	395

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	396

8.12.1	Introduction	396

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	396

8.12.2.1	Introduction	396

8.12.2.2	Measurements of a secondary component carrier	396

8.12.2.3	Measurements of a secondary component carrier with active SCell	396

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	396

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	396

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	399

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	401

8.12.3.1	Introduction	401

8.12.3.2	Measurements of a secondary component carrier	401

8.12.3.3	Measurements of a secondary component carrier with active SCell	401

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	401

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	401

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	403

8.13	Measurements for UE Category M1	405

8.13.1	Introduction	405

8.13.2	Requirements for UE category M1 with CE mode A	405

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	406

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	406

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	410

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	412

8.13.2.2	Void	415

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	415

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	415

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	418

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	420

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	421

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	421

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	423

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	425

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	426

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	427

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	427

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	428

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	428

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	429

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	430

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	430

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	430

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	434

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	436

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	440

8.13.3	Requirements for UE category M1 with CE mode B	440

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	441

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	441

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	445

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	448

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	453

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	454

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	454

8.13.3.2	Void	455

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	455

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	455

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	458

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	460

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	461

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	461

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	461

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	461

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	462

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	462

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	463

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	463

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	463

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	467

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	469

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	474

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	475

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	475

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	477

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	479

8.14	Measurements for UE category NB1	480

8.14.1	Introduction	480

8.14.2	NB-IoT intra frequency measurements under normal coverage	480

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	480

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	480

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	480

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	480

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	480

8.15	Void	481

8.16	Measurements for UE Category M2	481

8.16.1	Introduction	481

8.16.2	Requirements for UE category M2 with CE mode A	481

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	481

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	481

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	482

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	482

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	483

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	483

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	483

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	483

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	486

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	488

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	489

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	489

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	491

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	493

8.16.3	Requirements for UE category M2 with CE mode B	494

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	494

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	494

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	496

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	499

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	499

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	499

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	501

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	504

8.17	Void	504

8.18	Measurements for non-BL/CE UE	504

8.18.1	Introduction	504

8.18.2	Requirements for non-BL/CE UE with CE Mode B	504

8.18.2.1	E-UTRAN intra frequency measurements	504

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	504

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	505

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	505

9	Measurements performance requirements for UE	506

9.1	E-UTRAN measurements	507

9.1.1	Introduction	507

9.1.2	Intra-frequency RSRP Accuracy Requirements	507

9.1.2.1	Absolute RSRP Accuracy	507

9.1.2.2	Relative Accuracy of RSRP	508

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	508

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	509

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	510

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	511

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	512

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	513

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	514

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	514

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	514

9.1.3	Inter-frequency RSRP Accuracy Requirements	515

9.1.3.1	Absolute RSRP Accuracy	515

9.1.3.2	Relative Accuracy of RSRP	516

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	516

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	517

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	518

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	518

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	518

9.1.4	RSRP Measurement Report Mapping	519

9.1.5	Intra-frequency RSRQ Accuracy Requirements	519

9.1.5.1	Absolute RSRQ Accuracy	519

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	520

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	521

9.1.5.4	Absolute WB-RSRQ Accuracy	522

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	523

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	524

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	524

9.1.6	Inter-frequency RSRQ Accuracy Requirements	524

9.1.6.1	Absolute RSRQ Accuracy	524

9.1.6.2	Relative Accuracy of RSRQ	525

9.1.6.3	Absolute WB-RSRQ Accuracy	526

9.1.6.4	Relative WB-RSRQ Accuracy	526

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	527

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	528

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	528

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	528

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	529

9.1.7	RSRQ Measurement Report Mapping	530

9.1.8	Power Headroom	530

9.1.8.1	Period	530

9.1.8.2	Reporting Delay	530

9.1.8.3	Void	531

9.1.8.4	Report Mapping	531

9.1.9	UE Rx – Tx time difference	531

9.1.9.1	Measurement Requirement	531

9.1.9.2	Measurement Report mapping	532

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	532

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	533

9.1.10	Reference Signal Time Difference (RSTD)	534

9.1.10.1	Intra-Frequency Accuracy Requirement	534

9.1.10.2	Inter-Frequency Accuracy Requirement	536

9.1.10.3	RSTD Measurement Report Mapping	537

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	538

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	538

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	539

9.1.11	Carrier aggregation measurement accuracy	540

9.1.11.1	Primary component carrier accuracy requirement	541

9.1.11.2	Secondary component carrier accuracy requirement	541

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	541

9.1.11.4	Secondary component carrier relative accuracy requirement	541

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	541

9.1.13	Measurement accuracy for UE category 0	542

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	542

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	542

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	543

9.1.14	Accuracy requirements for Discovery Signal Measurements	544

9.1.14.1	Introduction	544

9.1.14.2	RSRP measurements in discovery signal occasions	544

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	544

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	544

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	544

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	545

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	546

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	546

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	547

9.1.14.3.3	CSI-RSRP measurement report mapping	548

9.1.14.4	RSRQ measurements in discovery signal occasions	548

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	548

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	548

9.1.15.1.1	Primary component carrier accuracy requirement	548

9.1.15.1.2	Secondary component carrier accuracy requirement	548

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	549

9.1.15.1.4	Secondary component carrier relative accuracy requirement	549

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	549

9.1.15.2.1	Primary component carrier accuracy requirement	549

9.1.15.2.2	Secondary component carrier accuracy requirement	549

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	549

9.1.15.2.4	Secondary component carrier relative accuracy requirement	549

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	549

9.1.17	RS-SINR Measurements	550

9.1.17.1	Measurement Report Mapping	550

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	550

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	550

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	551

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	551

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	552

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	552

9.1.18.1	Introduction	552

9.1.18.2	RSRP measurements	552

9.1.18.2.1	RSRP measurement report mapping	552

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	552

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	553

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	553

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	554

9.1.18.3	RSRQ measurements	554

9.1.18.3.1	RSRQ measurement report mapping	554

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	555

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	555

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	556

9.1.18.4	CSI-RSRP measurements	556

9.1.18.4.1	CSI-RSRP measurement report mapping	556

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	556

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	557

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	557

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	558

9.1.18.5	RSSI measurements	558

9.1.18.5.1	RSSI measurement report mapping	558

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	559

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	559

9.1.18.6	Channel occupancy measurements	559

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	559

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	560

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	560

9.1.19.1	Introduction	560

9.1.19.2	Accuracy requirements for measurements on SCC	560

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	560

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	561

9.1.20	SFN and Subframe Time Difference (SSTD)	561

9.1.20.1	SSTD Accuracy Requirement	561

9.1.20.2	SSTD Measurement Report Mapping	562

9.1.21	Measurement accuracy for UE category M1	563

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	563

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	564

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	565

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	566

9.1.21.5	RSRP Measurement Report Mapping	567

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	567

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	568

9.1.21.8	RSRQ Measurement Report Mapping	569

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	569

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	570

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	571

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	572

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	573

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	574

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	575

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	576

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	577

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	578

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	579

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	580

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	582

9.1.22	Measurement accuracy for UE Category NB1	584

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	584

9.1.22.2	Void	584

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode	584

9.1.22.4	Void	585

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	585

9.1.22.6	Void	585

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	585

9.1.22.8	Void	586

9.1.22.9	NRSRP Measurement Report Mapping	586

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	586

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	587

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	588

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	589

9.1.22.14	NRSRQ Measurement Report Mapping	590

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	591

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	591

9.1.23	Power Headroom for UE Category NB1	592

9.1.23.1	Period	592

9.1.23.2	Reporting Delay	592

9.1.23.3	Report Mapping for UE Category NB1	592

9.1.23.3.1	Void	593

9.1.23.3.2	Void	593

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	593

9.1.24	Void	593

9.1.25	Measurement accuracy for UE category M2	593

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	593

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	594

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	595

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	596

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	598

9.1.26	Measurement Accuracy for non-BL CE UE	599

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	600

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	600

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	601

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	602

9.1.26.5	RSRP Measurement Report Mapping	603

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	603

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	603

9.1.26.8	RSRQ Measurement Report Mapping	604

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	604

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	605

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	606

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	607

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	608

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	608

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	608

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	609

9.2	UTRAN FDD Measurements	610

9.2.1	UTRAN FDD CPICH RSCP	611

9.2.2	Void	611

9.2.3	UTRAN FDD CPICH Ec/No	611

9.3	UTRAN TDD Measurements	612

9.3.1	UTRAN TDD P-CCPCH RSCP	612

9.3.2	Void	612

9.3.3	Void	612

9.4	GSM Measurements	612

9.4.1	GSM carrier RSSI	612

9.5	CDMA2000 1x RTT Measurements	613

9.5.1	CDMA2000 1x RTT Pilot Strength	613

9.6	PCMAX,c	613

9.6.1	Report Mapping	613

9.6.2 	Estimation Period	613

9.6.3	Reporting Delay	613

9.7	IEEE802.11 Measurements	613

9.7.1	WLAN RSSI	614

9.7.2	WLAN RSSI Measurement Report Mapping	614

9.8	MBSFN Measurements	614

9.8.1	Introduction	614

9.8.2	MBSFN RSRP	614

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	614

9.8.2.2	MBSFN RSRP measurement report mapping	615

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	615

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	616

9.8.3	MBSFN RSRQ	616

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	616

9.8.3.2	MBSFN RSRQ measurement report mapping	616

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	617

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	617

9.8.4	MCH BLER	617

9.8.4.1	Measurement report mapping for MCH BLER	617

9.8.4.2	Measurement report mapping for MCH Block Number	618

9.9	ProSe Measurements	619

9.9.1	Introduction	619

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	619

9.9.2.1	Absolute S-RSRP Accuracy	619

9.9.2.2	Relative Accuracy of S-RSRP	620

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	621

9.9.3.1	Absolute SD-RSRP Accuracy	621

9.9.3.2	Relative Accuracy of SD-RSRP	621

9.10	V2X Measurements	622

9.10.1	Introduction	622

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	622

9.10.2.1	Absolute S-RSRP Accuracy	622

9.10.2.2	Relative Accuracy of S-RSRP	623

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	623

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	623

9.10.4	S-RSSI Measurement Accuracy Requirements	624

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	624

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	624

10	Measurements Performance Requirements for E-UTRAN	625

10.1	Received Interference Power	625

10.1.1	Absolute accuracy requirement	625

10.1.2	Relative accuracy requirement	625

10.1.3	Received Interference Power measurement report mapping	625

10.2	Angle of Arrival (AOA)	626

10.2.1	Range/mapping	626

10.3	Timing Advance (TADV)	626

10.3.1	Report mapping	626

11	ProSe Requirements in Any Cell Selection state	627

11.1	Introduction	627

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	627

11.2.1	Introduction	627

11.2.2	ProSe UE transmission timing	627

11.3	Initiation/Cease of SLSS Transmissions	627

11.3.1	Introduction	627

11.3.2	Requirements	627

11.4	Measurements for ProSe in Any Cell Selection State	628

11.4.1	Introduction	628

11.4.2	Requirements	628

11.4.2.1	E-UTRA FDD	628

11.4.2.2	E-UTRA TDD	628

11.5	Selection / Reselection of ProSe Synchronization Reference	629

11.5.1	Introduction	629

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	629

11.5.2.1	Introduction	629

11.5.2.2	Requirements	629

11.6	Void	629

11.7	Selection / Reselection of ProSe relay UE	629

11.7.1	Introduction	629

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	629

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	630

12.1	Introduction	630

12.2	Transmit Timing	630

12.2.1	GNSS as timing reference	630

12.3	Interruption	630

12.4	Reliability of GNSS signal	631

13	V2X Requirements	631

13.1	Introduction	631

13.2	UE Transmit Timing	631

13.2.1	Introduction	631

13.2.2	GNSS as synchronization reference source	631

13.2.3	Serving cell/PCell as synchronization reference source	632

13.2.4	SyncRef UE as synchronization reference source	632

13.3	Initiation/Cease of SLSS Transmissions	632

13.3.1	Introduction	632

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	632

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	633

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	633

13.4	Selection / Reselection of V2X Synchronization Reference Source	633

13.5	Autonomous Resource Selection/Reselection measurements	634

13.5.1	Introduction	634

13.5.2	PSSCH-RSRP measurements	634

13.5.3	S-RSSI measurements	634

13.6	Congestion Control measurements	635

13.7	Interruption	635

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	635

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	635

13.8	Reliability of GNSS signal	636

Annex A (normative):	Test Cases	637

A.1	Purpose of annex	637

A.2	Requirement classification for statistical testing	637

A.2.1	Types of requirements in TS 36.133	637

A.2.1.1	Time and delay requirements on UE higher layer actions	637

A.2.1.2	Measurements of power levels, relative powers and time	637

A.2.1.3	Implementation requirements	638

A.2.1.4	Physical layer timing requirements	638

A.3	RRM test configurations	639

A.3.1	Reference Measurement Channels	639

A.3.1.1	PDSCH	639

A.3.1.1.1	FDD	639

A.3.1.1.2	TDD	640

A.3.1.1.3	FDD for UE category 0	642

A.3.1.1.4	HD-FDD for UE category 0	643

A.3.1.1.5	TDD for UE category 0	644

A.3.1.1.6	Frame Structure 3	645

A.3.1.2	PCFICH/PDCCH/PHICH	646

A.3.1.2.1	FDD	646

A.3.1.2.2	TDD	646

A.3.1.2.3	HD-FDD for UE category 0	647

A.3.1.2.4	FS 3	647

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	647

A.3.1.3.1	FDD in CEModeA	648

A.3.1.3.2	HD-FDD in CEModeA	648

A.3.1.3.3	TDD in CEModeA	649

A.3.1.3.4	FDD in CEModeB	649

A.3.1.3.5	HD-FDD in CEModeB	650

A.3.1.3.6	TDD in CEModeB	650

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	651

A.3.1.4.1	FDD in CEModeA	651

A.3.1.4.2	HD-FDD in CEModeA	652

A.3.1.4.3	TDD in CEModeA	653

A.3.1.4.4	FDD in CEModeB	654

A.3.1.4.5	 HD-FDD in CEModeB	655

A.3.1.4.6	TDD in CEModeB	655

A.3.1.5	NPDSCH Reference Channel for UE category NB1	656

A.3.1.5.1	HD-FDD in-band operation	656

A.3.1.5.2	Void	657

A.3.1.5.3	HD-FDD standalone operation	657

A.3.1.5.4	Void	658

A.3.1.5.5	HD-FDD guard band operation	658

A.3.1.5.6	Void	658

A.3.1.6	NPDCCH Reference Channel for UE category NB1	658

A.3.1.6.1	HD-FDD in-band operation	658

A.3.1.6.2	Void	659

A.3.1.6.3	HD-FDD standalone operation	659

A.3.1.6.4	Void	659

A.3.1.6.5	HD-FDD guard band operation	659

A.3.1.6.6	Void	660

A.3.2	OFDMA Channel Noise Generator (OCNG)	660

A.3.2.1	OCNG Patterns for FDD	660

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	661

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	661

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	662

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	663

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	664

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	664

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	665

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	665

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	666

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	666

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	667

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	668

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	668

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	669

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	669

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	670

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	670

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	671

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	671

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	672

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	672

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	673

A.3.2.2	OCNG Patterns for TDD	673

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	674

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	674

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	675

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	675

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	676

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	677

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	678

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	679

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	679

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	680

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	681

A.3.2.3	OCNG Patterns for Narrowband IoT	681

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	683

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	684

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	684

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	685

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	686

A.3.2.4	OCNG Patterns for V2X sidelink	687

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	688

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	688

A.3.3	Reference DRX Configurations	688

A.3.4	ABS Transmission Configurations	689

A.3.4.1	Non-MBSFN ABS Transmission Configurations	689

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	689

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	689

A.3.4.2	MBSFN ABS Transmission Configurations	690

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	690

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	691

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	691

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	691

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	692

A.3.6.1	Introduction	692

A.3.7	Test Cases with Different Channel Bandwidths	692

A.3.7.1	Introduction	692

A.3.7.2	Principle of testing	692

A.3.8	Antenna Configuration	692

A.3.8.1	Antenna connection for 4 Rx capable UEs	692

A.3.8.1.1 Introduction	692

A.3.8.1.2 Principle of testing	692

A.3.8.1.2.1 Single carrier tests	692

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	693

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	694

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	694

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	694

A.3.9.1	Introduction	694

A.3.9.2	Principle of testing	694

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	695

A.3.10.1	Introduction	695

A.3.10.2	Principle of testing	695

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	695

A.3.11.1	Introduction	695

A.3.11.2	Principle of Testing	695

A.3.12	Proximity-based Services	695

A.3.12.1	Introduction	695

A.3.12.2	Reference DRX configurations for ProSe tests	695

A.3.12.3	Test Cases with Different Channel Bandwidths	696

A.3.12.3.1	Introduction	696

A.3.12.3.2	Principle of testing	696

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	696

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	702

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	704

A.3.12.6.1	FDD	704

A.3.12.7	Reference measurement channels for ProSe Direct Communication	704

A.3.12.7.1	FDD	704

A.3.12.8	ProSe Receive Traffic Generator	705

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	705

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	705

A.3.13	Time Offset between Cells	706

A.3.13.1	Introduction	706

A.3.13.2	Definition	706

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	706

A.3.14.1	Introduction	706

A.3.14.2	Principle of testing	706

A.3.15	Dual connectivity test cases with different combination of duplex mode	706

A.3.15.1	Introduction	706

A.3.15.2	Principle of testing	706

A.3.16	Reference PRACH Configurations	707

A.3.17	Listen before talk model	707

A.3.17.1	Introduction	707

A.3.17.2	Definition	707

A.3.18	Reference NPRACH Configurations	708

A.3.19	Dual connectivity test cases with different bandwidth combinations	709

A.3.19.1	Introduction	709

A.3.19.2	Principle of testing	709

A.3.20	Category M1 UE Test Cases	709

A.3.20.1	Introduction	709

A.3.20.2	Principle of Cat-M1 UE Testing	709

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	710

A.3.21.1	Introduction	710

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	711

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	712

A.3.22	Category 1bis UE Test Cases	713

A.3.22.1	Introduction	713

A.3.22.2	Principle of Category 1bis UE Testing	713

A.3.23	Category NB2 UE Test Cases	716

A.3.23.1	Introduction	716

A.3.23.2	Principle of Category NB2 UE Testing	716

A.3.24	V2X sidelink communication	716

A.3.24.1	Introduction	716

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	717

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	720

A.3.25	Category M2 UE Test Cases	721

A.3.25.1	Introduction	721

A.3.25.2	Principle of Cat-M2 UE Testing	721

A.4	E-UTRAN RRC_IDLE state	723

A.4.2	Cell Re-Selection	723

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	723

A.4.2.1.1	Test Purpose and Environment	723

A.4.2.1.2	Test Requirements	724

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	725

A.4.2.2.1	Test Purpose and Environment	725

A.4.2.2.2	Test Requirements	726

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	727

A.4.2.3.1	Test Purpose and Environment	727

A.4.2.3.2	Test Requirements	728

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	729

A.4.2.4.1	Test Purpose and Environment	729

A.4.2.4.2	Test Requirements	730

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	731

A.4.2.5.1	Test Purpose and Environment	731

A.4.2.5.2	Test Requirements	732

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	733

A.4.2.6.1	Test Purpose and Environment	733

A.4.2.6.2	Test Requirements	734

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	735

A.4.2.7.1	Test Purpose and Environment	735

A.4.2.7.2	Test Requirements	736

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	737

A.4.2.8.1	Test Purpose and Environment	737

A.4.2.8.2	Test Requirements	738

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	739

A.4.2.9.1	Test Purpose and Environment	739

A.4.2.9.2	Test Requirements	739

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	739

A.4.2.10.1	Test Purpose and Environment	739

A.4.2.10.2	Test Requirements	743

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	743

A.4.2.11.1	Test Purpose and Environment	743

A.4.2.11.2	Test Requirements	747

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	747

A.4.2.12.1	Test Purpose and Environment	747

A.4.2.12.2	Test Requirements	749

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	750

A.4.2.13.1	Test Purpose and Environment	750

A.4.2.13.2	Test Requirements	751

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	752

A.4.2.14.1	Test Purpose and Environment	752

A.4.2.14.2	Test Requirements	753

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	754

A.4.2.15.1	Test Purpose and Environment	754

A.4.2.15.2	Test Requirements	755

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	756

A.4.2.16.1	Test Purpose and Environment	756

A.4.2.16.2	Test Requirements	757

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	758

A.4.2.17.1	Test Purpose and Environment	758

A.4.2.17.2	Test Requirements	759

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	760

A.4.2.18.1	Test Purpose and Environment	760

A.4.2.18.2	Test Requirements	762

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	763

A.4.2.19.1	Test Purpose and Environment	763

A.4.2.19.2	Test Requirements	765

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	766

A.4.2.20.1	Test Purpose and Environment	766

A.4.2.20.2	Test Requirements	767

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	768

A.4.2.21.1	Test Purpose and Environment	768

A.4.2.21.2	Test Requirements	769

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	770

A.4.2.22.1	Test Purpose and Environment	770

A.4.2.22.2	Test Requirements	771

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	772

A.4.2.23.1	Test Purpose and Environment	772

A.4.2.23.2	Test Requirements	773

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	774

A.4.2.24.1	Test Purpose and Environment	774

A.4.2.24.2	Test Requirements	776

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	777

A.4.2.25.1	Test Purpose and Environment	777

A.4.2.25.2	Test Requirements	778

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	779

A.4.2.26.1	Test Purpose and Environment	779

A.4.2.26.2	Test Requirements	780

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	781

A.4.2.27.1	Test Purpose and Environment	781

A.4.2.27.2	Test Requirements	782

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	783

A.4.2.28.1	Test Purpose and Environment	783

A.4.2.28.2	Test Requirements	784

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	785

A.4.2.29.1	Test Purpose and Environment	785

A.4.2.29.2	Test Requirements	786

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	787

A.4.2.30.1	Test Purpose and Environment	787

A.4.2.30.2	Test Requirements	788

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	789

A.4.2.31.1	Test Purpose and Environment	789

A.4.2.31.2	Test Requirements	790

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	791

A.4.2.32.1	Test Purpose and Environment	791

A.4.2.32.2	Test Requirements	792

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	793

A.4.2.33.1	Test Purpose and Environment	793

A.4.2.33.2	Test Requirements	794

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	795

A.4.2.34.1	Test Purpose and Environment	795

A.4.2.34.2	Test Requirements	796

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	797

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	797

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	797

A.4.3.1.1.1	Test Purpose and Environment	797

A.4.3.1.1.2	Test Requirements	799

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	799

A.4.3.1.2.1	Test Purpose and Environment	799

A.4.3.1.2.2	Test Requirements	802

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	802

A.4.3.1.3.1	Test Purpose and Environment	802

A.4.3.1.3.2	Test Requirements	805

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	805

A.4.3.1.4.1	Test Purpose and Environment	805

A.4.3.1.4.2	Test Requirements	806

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	806

A.4.3.1.5.1	Test Purpose and Environment	806

A.4.3.1.5.2	Test Requirements	809

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	810

A.4.3.2.1	Test Purpose and Environment	810

A.4.3.2.1.1	Void	810

A.4.3.2.1.2	1.28Mcps TDD option	810

A.4.3.2.1.3	Void	812

A.4.3.2.2	Test Requirements	812

A.4.3.2.2.1	1.28Mcps TDD option	812

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	812

A.4.3.2A.1	Test Purpose and Environment	812

A.4.3.2A.2	Test Requirements	816

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	816

A.4.3.3.1	Test Purpose and Environment	816

A.4.3.3.2	Test Requirements	819

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	819

A.4.3.3A.1	Test Purpose and Environment	819

A.4.3.3A.2	Test Requirements	823

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	824

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	824

A.4.3.4.1.1	Test Purpose and Environment	824

A.4.3.4.1.2	Test Requirements	826

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	826

A.4.3.4.2.1	Test Purpose and Environment	826

A.4.3.4.2.2	Test Requirements	828

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	828

A.4.3.4.3.1	Test Purpose and Environment	828

A.4.3.4.3.2	Test Requirements	831

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	831

A.4.3.4.4.1	Test Purpose and Environment	831

A.4.3.4.4.2	Test Requirements	835

A.4.4	E-UTRAN to GSM Cell Re-Selection	835

A.4.4.1	E-UTRAN FDD – GSM:	835

A.4.4.1.1	Test Purpose and Environment	835

A.4.4.1.2	Test Requirements	837

A.4.4.2	E-UTRAN TDD – GSM:	837

A.4.4.2.1	Test Purpose and Environment	837

A.4.4.2.2	Test Requirements	839

A.4.5	E-UTRAN to HRPD Cell Re-Selection	840

A.4.5.1	E-UTRAN FDD – HRPD	840

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	840

A.4.5.1.1.1	Test Purpose and Environment	840

A.4.5.1.1.2	Test Requirements	842

A.4.5.2	E-UTRAN TDD – HRPD	842

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	842

A.4.5.2.1.1	Test Purpose and Environment	842

A.4.5.2.1.2	Test Requirements	845

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	845

A.4.6.1	E-UTRAN FDD – cdma2000 1X	845

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	845

A.4.6.1.1.1	Test Purpose and Environment	845

A.4.6.1.1.2	Test Requirements	848

A.4.6.2	E-UTRAN TDD – cdma2000 1X	848

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	848

A.4.6.2.1.1	Test Purpose and Environment	848

A.4.6.2.1.2	Test Requirements	851

A.4.7	Idle State Positioning Measurement for UE category NB1	851

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	851

A.4.7.1.1	Test Purpose and Environment	851

A.4.7.1.2	Test Requirements	855

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	856

A.4.7.2.1	Test Purpose and Environment	856

A.4.7.4.2	Test Requirements	859

A.5	E-UTRAN RRC CONNECTED Mode Mobility	860

A.5.1	E-UTRAN Handover	860

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	860

A.5.1.1.1	Test Purpose and Environment	860

A.5.1.1.2	Test Requirements	862

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	862

A.5.1.2.1	Test Purpose and Environment	862

A.5.1.2.2	Test Requirements	864

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	864

A.5.1.3.1	Test Purpose and Environment	864

A.5.1.3.2	Test Requirements	866

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	866

A.5.1.4.1	Test Purpose and Environment	866

A.5.1.4.2	Test Requirements	868

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	868

A.5.1.5.1	Test Purpose and Environment	868

A.5.1.5.2	Test Requirements	870

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	870

A.5.1.6.1	Test Purpose and Environment	870

A.5.1.6.2	Test Requirements	872

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	872

A.5.1.7.1	Test Purpose and Environment	872

A.5.1.7.2	Test Requirements	875

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	875

A.5.1.8.1	Test Purpose and Environment	875

A.5.1.8.2	Test Requirements	878

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	878

A.5.1.9.1	Test Purpose and Environment	878

A.5.1.9.2	Test Requirements	879

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	879

A.5.1.10.1	Test Purpose and Environment	879

A.5.1.10.2	Test Requirements	881

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	881

A.5.1.11.1	Test Purpose and Environment	881

A.5.1.11.2	Test Requirements	883

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	883

A.5.1.12.1	Test Purpose and Environment	883

A.5.1.12.2	Test Requirements	885

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	885

A.5.1.13.1	Test Purpose and Environment	885

A.5.1.13.2	Test Requirements	887

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	888

A.5.1.14.1	Test Purpose and Environment	888

A.5.1.14.2	Test Requirements	889

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	890

A.5.1.15.1	Test Purpose and Environment	890

A.5.1.15.2	Test Requirements	891

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	892

A.5.1.16.1	Test Purpose and Environment	892

A.5.1.16.2	Test Requirements	893

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	894

A.5.1.17.1	Test Purpose and Environment	894

A.5.1.17.2	Test Requirements	895

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	896

A.5.1.18.1	Test Purpose and Environment	896

A.5.1.18.2	Test Requirements	897

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	898

A.5.1.19.1	Test Purpose and Environment	898

A.5.1.19.2	Test Requirements	899

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	900

A.5.1.20.1	Test Purpose and Environment	900

A.5.1.20.2	Test Requirements	901

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	902

A.5.1.21.1	Test Purpose and Environment	902

A.5.1.21.2	Test Requirements	903

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	903

A.5.1.22.1	Test Purpose and Environment	903

A.5.1.22.2	Test Requirements	905

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	905

A.5.1.23.1	Test Purpose and Environment	905

A.5.1.23.2	Test Requirements	907

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	907

A.5.1.24.1	Test Purpose and Environment	907

A.5.1.24.2	Test Requirements	909

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	909

A.5.1.25.1	Test Purpose and Environment	909

A.5.1.25.2	Test Requirements	911

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	911

A.5.1.26.1	Test Purpose and Environment	911

A.5.1.26.2	Test Requirements	913

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	913

A.5.1.27.1	Test Purpose and Environment	913

A.5.1.27.2	Test Requirements	915

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	916

A.5.1.28.1	Test Purpose and Environment	916

A.5.1.28.2	Test Requirements	917

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	918

A.5.1.29.1	Test Purpose and Environment	918

A.5.1.29.2	Test Requirements	919

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	920

A.5.1.30.1	Test Purpose and Environment	920

A.5.1.30.2	Test Requirements	921

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	922

A.5.1.31.1	Test Purpose and Environment	922

A.5.1.31.2	Test Requirements	923

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	924

A.5.1.32.1	Test Purpose and Environment	924

A.5.1.32.2	Test Requirements	925

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	926

A.5.1.33.1	Test Purpose and Environment	926

A.5.1.13.2	Test Requirements	927

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	928

A.5.1.34.1	Test Purpose and Environment	928

A.5.1.34.2	Test Requirements	929

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	930

A.5.1.35.1	Test Purpose and Environment	930

A.5.1.35.2	Test Requirements	931

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	932

A.5.1.36.1	Test Purpose and Environment	932

A.5.1.36.2	Test Requirements	933

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	934

A.5.1.37.1	Test Purpose and Environment	934

A.5.1.37.2	Test Requirements	935

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	936

A.5.1.38.1	Test Purpose and Environment	936

A.5.1.38.2	Test Requirements	937

A.5.2	E-UTRAN Handover to other RATs	938

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	938

A.5.2.1.1	Test Purpose and Environment	938

A.5.2.1.2	Test Requirements	939

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	940

A.5.2.2.1	Test Purpose and Environment	940

A.5.2.2.2	Test Requirements	943

A.5.2.3 E-UTRAN FDD- GSM Handover	943

A.5.2.3.1	Test Purpose and Environment	943

A.5.2.3.2	Test Requirements	944

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	945

A.5.2.4.1	Test Purpose and Environment	945

A.5.2.4.1.1	Void	945

A.5.2.4.1.2	1.28 Mcps TDD option	945

A.5.2.4.1.3	Void	947

A.5.2.4.2	Test Requirements	947

A.5.2.4.2.1	Void	947

A.5.2.4.2.2	1.28 Mcps TDD option	947

A.5.2.4.2.3	Void	947

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	947

A.5.2.5.1	Test Purpose and Environment	947

A.5.2.5.1.3	Void	950

A.5.2.5.2	Test Requirements	950

A.5.2.5.2.1	Void	950

A.5.2.5.2.2	1.28 Mcps TDD option	950

A.5.2.5.2.3	Void	950

A.5.2.6	E-UTRAN TDD - GSM Handover	950

A.5.2.6.1	Test Purpose and Environment	950

A.5.2.6.2	Test Requirements	952

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	953

A.5.2.7.1	Test Purpose and Environment	953

A.5.2.7.2	Test Requirements	955

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	955

A.5.2.8.1	Test Purpose and Environment	955

A.5.2.8.2	Test Requirements	956

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	957

A.5.2.9.1	Test Purpose and Environment	957

A.5.2.9.2	Test Requirements	958

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	959

A.5.2.10.1	Test Purpose and Environment	959

A.5.2.10.2	Test Requirements	961

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	961

A.5.2.10A.1	Test Purpose and Environment	961

A.5.2.10A.2	Test Requirements	964

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	964

A.5.2.10B.1	Test Purpose and Environment	964

A.5.2.10B.2	Test Requirements	967

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	967

A.5.2.11.1	Test Purpose and Environment	967

A.5.2.11.2	Test Requirements	967

A.5.3	E-UTRAN Handover to Non-3GPP RATs	968

A.5.3.1	E-UTRAN FDD – HRPD Handover	968

A.5.3.1.1	Test Purpose and Environment	968

A.5.3.1.2	Test Requirements	970

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	970

A.5.3.2.1	Test Purpose and Environment	970

A.5.3.2.2	Test Requirements	973

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	973

A.5.3.3.1	Test Purpose and Environment	973

A.5.3.3.2	Test Requirements	975

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	976

A.5.3.4.1	Test Purpose and Environment	976

A.5.3.4.2	Test Requirements	977

A.5.3.5	E-UTRAN TDD – HRPD Handover	978

A.5.3.5.1	Test Purpose and Environment	978

A.5.3.5.2	Test Requirements	980

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	981

A.5.3.6.1	Test Purpose and Environment	981

A.5.3.6.2	Test Requirements	984

A.6	RRC Connection Control	984

A.6.1	RRC Re-establishment	984

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	984

A.6.1.1.1	Test Purpose and Environment	984

A.6.1.1.2	Test Requirements	985

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	986

A.6.1.2.1	Test Purpose and Environment	986

A.6.1.2.2	Test Requirements	987

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	988

A.6.1.3.1	Test Purpose and Environment	988

A.6.1.3.2	Test Requirements	989

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	990

A.6.1.4.1	Test Purpose and Environment	990

A.6.1.4.2	Test Requirements	991

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	992

A.6.1.5.1	Test Purpose and Environment	992

A.6.1.5.2	Test Requirements	992

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	992

A.6.1.6.1	Test Purpose and Environment	992

A.6.1.6.2	Test Requirements	994

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	995

A.6.1.7.1	Test Purpose and Environment	995

A.6.1.7.2	Test Requirements	996

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	997

A.6.1.8.1	Test Purpose and Environment	997

A.6.1.8.2	Test Requirements	998

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	999

A.6.1.9.1	Test Purpose and Environment	999

A.6.1.9.2	Test Requirements	1000

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1001

A.6.1.10.1	Test Purpose and Environment	1001

A.6.1.10.2	Test Requirements	1002

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1003

A.6.1.11.1	Test Purpose and Environment	1003

A.6.1.11.2	Test Requirements	1004

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1005

A.6.1.12.1	Test Purpose and Environment	1005

A.6.1.12.2	Test Requirements	1006

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1007

A.6.1.13.1	Test Purpose and Environment	1007

A.6.1.13.2	Test Requirements	1008

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1009

A.6.1.14.1	Test Purpose and Environment	1009

A.6.1.14.2	Test Requirements	1010

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1011

A.6.1.15.1	Test Purpose and Environment	1011

A.6.1.15.2	Test Requirements	1013

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1014

A.6.1.16.1	Test Purpose and Environment	1014

A.6.1.16.2	Test Requirements	1016

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1017

A.6.1.17.1	Test Purpose and Environment	1017

A.6.1.17.2	Test Requirements	1018

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1019

A.6.1.18.1	Test Purpose and Environment	1019

A.6.1.18.2	Test Requirements	1020

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1021

A.6.1.19.1	Test Purpose and Environment	1021

A.6.1.19.2	Test Requirements	1022

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1023

A.6.1.20.1	Test Purpose and Environment	1023

A.6.1.20.2	Test Requirements	1024

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1025

A.6.1.21.1	Test Purpose and Environment	1025

A.6.1.21.2	Test Requirements	1026

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1027

A.6.1.22.1	Test Purpose and Environment	1027

A.6.1.22.2	Test Requirements	1028

A.6.2	Random Access	1029

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1029

A.6.2.1.1	Test Purpose and Environment	1029

A.6.2.1.2	Test Requirements	1031

A.6.2.1.2.1	Random Access Response Reception	1031

A.6.2.1.2.2	No Random Access Response Reception	1031

A.6.2.1.2.3	Receiving a NACK on msg3	1031

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1031

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1032

A.6.2.1.2.6	Contention Resolution Timer expiry	1032

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1032

A.6.2.2.1	Test Purpose and Environment	1032

A.6.2.2.2	Test Requirements	1033

A.6.2.2.2.1	Random Access Response Reception	1034

A.6.2.2.2.2	No Random Access Response Reception	1034

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1034

A.6.2.3.1	Test Purpose and Environment	1034

A.6.2.3.2	Test Requirements	1036

A.6.2.3.2.1	Random Access Response Reception	1036

A.6.2.3.2.2	No Random Access Response reception	1036

A.6.2.3.2.3	Receiving a NACK on msg3	1036

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1037

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1037

A.6.2.3.2.6	Contention Resolution Timer expiry	1037

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1037

A.6.2.4.1	Test Purpose and Environment	1037

A.6.2.4.2	Test Requirements	1039

A.6.2.4.2.1	Random Access Response Reception	1039

A.6.2.4.2.2	No Random Access Response Reception	1039

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1039

A.6.2.5.1	Test Purpose and Environment	1039

A.6.2.5.2	Test Requirements	1040

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1040

A.6.2.6.1	Test Purpose and Environment	1040

A.6.2.6.2	Test Requirements	1040

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1041

A.6.2.7.1	Test Purpose and Environment	1041

A.6.2.7.2	Test Requirements	1043

A.6.2.7.2.1	Random Access Response Reception	1043

A.6.2.7.2.2	No Random Access Response Reception	1043

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1043

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1044

A.6.2.8.1	Test Purpose and Environment	1044

A.6.2.8.2	Test Requirements	1046

A.6.2.8.2.1	Random Access Response Reception	1046

A.6.2.8.2.2	No Random Access Response Reception	1046

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1046

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1047

A.6.2.9.1	Test Purpose and Environment	1047

A.6.2.9.2	Test Requirements	1049

A.6.2.9.2.1	Random Access Response Reception	1049

A.6.2.9.2.2	No Random Access Response Reception	1050

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1050

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1051

A.6.2.10.1	Test Purpose and Environment	1051

A.6.2.10.2	Test Requirements	1053

A.6.2.10.2.1	Random Access Response Reception	1053

A.6.2.10.2.2	No Random Access Response Reception	1053

A.6.2.10.2.3	Receiving a NACK on msg3	1054

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1054

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1054

A.6.2.10.2.6	Contention Resolution Timer expiry	1054

A.6.2.10.2.7	PRACH Resource Selection	1054

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1054

A.6.2.11.1	Test Purpose and Environment	1054

A.6.2.11.2	Test Requirements	1056

A.6.2.11.2.1	Random Access Response Reception	1056

A.6.2.11.2.2	No Random Access Response Reception	1057

A.6.2.11.2.3	Receiving a NACK on msg3	1057

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1057

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1057

A.6.2.11.2.6	Contention Resolution Timer expiry	1057

A.6.2.11.2.7	PRACH Resource Selection	1058

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1058

A.6.2.12.1	Test Purpose and Environment	1058

A.6.2.12.2	Test Requirements	1060

A.6.2.12.2.1	Random Access Response Reception	1060

A.6.2.12.2.2	No Random Access Response Reception	1061

A.6.2.12.2.3	Receiving a NACK on msg3	1061

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1061

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1061

A.6.2.12.2.6	Contention Resolution Timer expiry	1061

A.6.2.12.2.7	PRACH Resource Selection	1062

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1062

A.6.2.13.1	Test Purpose and Environment	1062

A.6.2.13.2	Test Requirements	1064

A.6.2.13.2.1	Random Access Response Reception	1064

A.6.2.13.2.2	No Random Access Response Reception	1064

A.6.2.13.2.3	Receiving a NACK on msg3	1065

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1065

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1065

A.6.2.13.2.6	Contention Resolution Timer expiry	1065

A.6.2.13.2.7	PRACH Resource Selection	1065

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1065

A.6.2.14.1	Test Purpose and Environment	1065

A.6.2.14.2	Test Requirements	1067

A.6.2.14.2.1	Random Access Response Reception	1067

A.6.2.14.2.2	No Random Access Response Reception	1068

A.6.2.14.2.3	Receiving a NACK on msg3	1068

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1068

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1068

A.6.2.14.2.6	Contention Resolution Timer expiry	1068

A.6.2.14.2.7	PRACH Resource Selection	1069

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1069

A.6.2.15.1	Test Purpose and Environment	1069

A.6.2.15.2	Test Requirements	1071

A.6.2.15.2.1	Random Access Response Reception	1071

A.6.2.15.2.2	No Random Access Response Reception	1072

A.6.2.15.2.3	Receiving a NACK on msg3	1072

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1072

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1072

A.6.2.15.2.6	Contention Resolution Timer expiry	1072

A.6.2.15.2.7	PRACH Resource Selection	1073

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1073

A.6.2.16.1	Test Purpose and Environment	1073

A.6.2.16.2	Test Requirements	1076

A.6.2.16.2.1	Random Access Response Reception	1076

A.6.2.16.2.2	No Random Access Response Reception	1077

A.6.2.16.2.3	Receiving a NACK on msg3	1077

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1077

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1077

A.6.2.16.2.6	Contention Resolution Timer expiry	1077

A.6.2.16.2.7	NPRACH Resource Selection	1077

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1078

A.6.2.17.1	Test Purpose and Environment	1078

A.6.2.17.2	Test Requirements	1081

A.6.2.17.2.1	Random Access Response Reception	1081

A.6.2.17.2.2	No Random Access Response Reception	1082

A.6.2.17.2.3	Receiving a NACK on msg3	1082

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1082

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1082

A.6.2.17.2.6	Contention Resolution Timer expiry	1082

A.6.2.17.2.7	NPRACH Resource Selection	1082

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1083

A.6.2.18.1	Test Purpose and Environment	1083

A.6.2.18.2	Test Requirements	1086

A.6.2.18.2.1	Random Access Response Reception	1086

A.6.2.18.2.2	No Random Access Response Reception	1087

A.6.2.18.2.3	Receiving a NACK on msg3	1087

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1087

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1087

A.6.2.18.2.6	Contention Resolution Timer expiry	1087

A.6.2.18.2.7	NPRACH Resource Selection	1087

A.6.3	RRC Connection Release with Redirection	1088

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1088

A.6.3.1.1	Test Purpose and Environment	1088

A.6.3.1.2	Test Requirements	1090

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1090

A.6.3.2.1	Test Purpose and Environment	1090

A.6.3.2.2	Test Requirements	1092

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1092

A.6.3.3.1	Test Purpose and Environment	1092

A.6.3.3.2	Test Requirements	1093

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1094

A.6.3.4.1	Test Purpose and Environment	1094

A.6.3.4.2	Test Requirements	1095

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1096

A.6.3.5.1	Test Purpose and Environment	1096

A.6.3.5.2	Test Requirements	1098

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1098

A.6.3.6.1	Test Purpose and Environment	1098

A. 6.3.6.2	Test Requirements	1100

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1100

A.6.3.7.1	Test Purpose and Environment	1100

A.6.3.7.2	Test Requirements	1103

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1103

A.6.3.8.1	Test Purpose and Environment	1103

A.6.3.8.2	Test Requirements	1106

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1106

A.6.3.9.1	Test Purpose and Environment	1106

A.6.3.9.2	Test Requirements	1108

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1108

A.6.3.10.1	Test Purpose and Environment	1108

A.6.3.10.2	Test Requirements	1110

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1110

A.6.3.11.1	Test Purpose and Environment	1110

A.6.3.11.2	Test Requirements	1112

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1112

A.6.3.12.1	Test Purpose and Environment	1112

A.6.3.12.2	Test Requirements	1115

A.7	Timing and Signalling Characteristics	1116

A.7.1	UE Transmit Timing	1116

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1116

A.7.1.1.1	Test Purpose and Environment	1116

A.7.1.1.2	Test Requirements	1117

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1118

A.7.1.2.1	Test Purpose and Environment	1118

A.7.1.2.2	Test Requirements	1120

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1121

A.7.1.3.1	Test Purpose and Environment	1121

A.7.1.3.2	Test Requirements	1123

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1124

A.7.1.4.1	Test Purpose and Environment	1124

A.7.1.4.2	Test Requirements	1126

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1127

A.7.1.4A.1	Test Purpose and Environment	1127

A.7.1.4A.2	Test Requirements	1127

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1127

A.7.1.5.1	Test Purpose and Environment	1127

A.7.1.5.2	Test Requirements	1128

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1128

A.7.1.6.1	Test Purpose and Environment	1128

A.7.1.6.2	Test Requirements	1130

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1131

A.7.1.7.1	Test Purpose and Environment	1131

A.7.1.7.2	Test Requirements	1132

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1133

A.7.1.7A.1	Test Purpose and Environment	1133

A.7.1.7A.2	Test Requirements	1133

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1133

A.7.1.7B.1	Test Purpose and Environment	1133

A.7.1.7B.2	Test Requirements	1133

A.7.1.8	Void	1134

A.7.1.8.1	Void	1134

A.7.1.8.2	Void	1134

A.7.1.9	Void	1134

A.7.1.9.1	Void	1134

A.7.1.9.2	Void	1134

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1134

A.7.1.10.1	Test Purpose and Environment	1134

A.7.1.10.2	Test Requirements	1136

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1136

A.7.1.11.1	Test Purpose and Environment	1136

A.7.1.11.2	Test Requirements	1138

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1138

A.7.1.12.1	Test Purpose and Environment	1138

A.7.1.12.2	Test Requirements	1140

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1140

A.7.1.13.1	Test Purpose and Environment	1140

A.7.1.13.2	Test Requirements	1142

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1143

A.7.1.14.1	Test Purpose and Environment	1143

A.7.1.14.2	Test Requirements	1144

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1144

A.7.1.15.1	Test Purpose and Environment	1144

A.7.1.15.2	Test Requirements	1145

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1146

A.7.1.16.1	Test Purpose and Environment	1146

A.7.1.16.2	Test Requirements	1147

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1148

A.7.1.17.1	Test Purpose and Environment	1148

A.7.1.17.2	Test Requirements	1150

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1151

A.7.1.18.1	Test Purpose and Environment	1151

A.7.1.18.2	Test Requirements	1153

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1154

A.7.1.19.1	Test Purpose and Environment	1154

A.7.1.19.2	Test Requirements	1155

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1155

A.7.1.20.1	Test Purpose and Environment	1155

A.7.1.20.2	Test Requirements	1157

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1157

A.7.1.21.1	Test Purpose and Environment	1157

A.7.1.21.2	Test Requirements	1159

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1159

A.7.1.22.1	Test Purpose and Environment	1159

A.7.1.22.2	Test Requirements	1161

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1161

A.7.1.23.1	Test Purpose and Environment	1161

A.7.1.23.2	Test Requirements	1163

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1163

A.7.1.24.1	Test Purpose and Environment	1163

A.7.1.24.2	Test Requirements	1164

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1165

A.7.1.25.1	Test Purpose and Environment	1165

A.7.1.25.2	Test Requirements	1166

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1167

A.7.1.26.1	Test Purpose and Environment	1167

A.7.1.26.2	Test Requirements	1168

A.7.2	UE Timing Advance	1169

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1169

A.7.2.1.1	Test Purpose and Environment	1169

A.7.2.1.2	Test Requirements	1171

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1171

A.7.2.2.1	Test Purpose and Environment	1171

A.7.2.2.2	Test Requirements	1173

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1173

A.7.2.3.1	Test Purpose and Environment	1173

A.7.2.3.2	Test Requirements	1174

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1174

A.7.2.4.1	Test Purpose and Environment	1174

A.7.2.4.2	Test Requirements	1176

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1176

A.7.2.5.1	Test Purpose and Environment	1176

A.7.2.5.2	Test Requirements	1178

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1178

A.7.2.5A.1	Test Purpose and Environment	1178

A.7.2.5A.2	Test Requirements	1179

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1179

A.7.2.5B.1	Test Purpose and Environment	1179

A.7.2.5B.2	Test Requirements	1179

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1179

A.7.2.6.1	Test Purpose and Environment	1179

A.7.2.6.2	Test Requirements	1182

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1182

A.7.2.7.1	Test Purpose and Environment	1182

A.7.2.7.2	Test Requirements	1184

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1184

A.7.2.8.1	Test Purpose and Environment	1184

A.7.2.8.2	Test Requirements	1186

A.7.2.9.2	Test Requirements	1188

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1188

A.7.2.10.1	Test Purpose and Environment	1188

A.7.2.10.2	Test Requirements	1189

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1189

A.7.2.11.1	Test Purpose and Environment	1189

A.7.2.11.2	Test Requirements	1191

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1191

A.7.2.12.1	Test Purpose and Environment	1191

A.7.2.12.2	Test Requirements	1193

A.7.3	Radio Link Monitoring	1193

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1194

A.7.3.1.1	Test Purpose and Environment	1194

A.7.3.1.2	Test Requirements	1198

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1198

A.7.3.2.1	Test Purpose and Environment	1198

A.7.3.2.2	Test Requirements	1202

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1202

A.7.3.3.1	Test Purpose and Environment	1202

A.7.3.3.2	Test Requirements	1206

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1206

A.7.3.4.1	Test Purpose and Environment	1206

A.7.3.4.2	Test Requirements	1210

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1210

A.7.3.5.1	Test Purpose and Environment	1210

A.7.3.5.2	Test Requirements	1213

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1214

A.7.3.6.1	Test Purpose and Environment	1214

A.7.3.6.2	Test Requirements	1217

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1217

A.7.3.7.1	Test Purpose and Environment	1217

A.7.3.7.2	Test Requirements	1220

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1220

A.7.3.8.1	Test Purpose and Environment	1220

A.7.3.8.2	Test Requirements	1223

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1223

A.7.3.9.2	Test Requirements	1226

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1226

A.7.3.10.1	Test Purpose and Environment	1226

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1230

A.7.3.11.1	Test Purpose and Environment	1230

A.7.3.11.2	Test Requirements	1235

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1235

A.7.3.12.1	Test Purpose and Environment	1235

A.7.3.12.2	Test Requirements	1240

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1240

A.7.3.13.1	Test Purpose and Environment	1240

A.7.3.13.2	Test Requirements	1243

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1243

A.7.3.14.1	Test Purpose and Environment	1243

A.7.3.14.2	Test Requirements	1246

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1246

A.7.3.15.1	Test Purpose and Environment	1246

A.7.3.15.2	Test Requirements	1250

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1250

A.7.3.16.1	Test Purpose and Environment	1250

A.7.3.16.2	Test Requirements	1255

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1255

A.7.3.17.1	Test Purpose and Environment	1255

A.7.3.17.2	Test Requirements	1259

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1259

A.7.3.18.1	Test Purpose and Environment	1259

A.7.3.18.2	Test Requirements	1263

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1263

A.7.3.19.1	Test Purpose and Environment	1263

A.7.3.19.2	Test Requirements	1268

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1268

A.7.3.20.1	Test Purpose and Environment	1268

A.7.3.20.2	Test Requirements	1273

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1273

A.7.3.21.1	Test Purpose and Environment	1273

A.7.3.21.2	Test Requirements	1278

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1278

A.7.3.22.1	Test Purpose and Environment	1278

A.7.3.22.2	Test Requirements	1283

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1283

A.7.3.23.1	Test Purpose and Environment	1283

A.7.3.23.2	Test Requirements	1284

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1284

A.7.3.24.1	Test Purpose and Environment	1284

A.7.3.24.2	Test Requirements	1285

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1285

A.7.3.25.1	Test Purpose and Environment	1285

A.7.3.25.2	Test Requirements	1286

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1286

A.7.3.26.1	Test Purpose and Environment	1286

A.7.3.26.2	Test Requirements	1289

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1289

A.7.3.27.1	Test Purpose and Environment	1289

A.7.3.27.2	Test Requirements	1292

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1292

A.7.3.28.1	Test Purpose and Environment	1292

A.7.3.28.2	Test Requirements	1295

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1295

A.7.3.29.1	Test Purpose and Environment	1295

A.7.3.29.2	Test Requirements	1298

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1298

A.7.3.30.1	Test Purpose and Environment	1298

A.7.3.30.2	Test Requirements	1301

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1301

A.7.3.31.1	Test Purpose and Environment	1301

A.7.3.31.2	Test Requirements	1304

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1304

A.7.3.32.1	Test Purpose and Environment	1304

A.7.3.32.2	Test Requirements	1307

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1307

A.7.3.33.1	Test Purpose and Environment	1307

A.7.3.33.2	Test Requirements	1310

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1310

A.7.3.34.1	Test Purpose and Environment	1310

A.7.3.34.2	Test Requirements	1313

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1313

A.7.3.35.1	Test Purpose and Environment	1313

A.7.3.35.2	Test Requirements	1316

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1316

A.7.3.36.1	Test Purpose and Environment	1316

A.7.3.36.2	Test Requirements	1319

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1319

A.7.3.37.1	Test Purpose and Environment	1319

A.7.3.37.2	Test Requirements	1322

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1322

A.7.3.38.1	Test Purpose and Environment	1322

A.7.3.38.2	Test Requirements	1325

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1325

A.7.3.39.1	Test Purpose and Environment	1325

A.7.3.39.2	Test Requirements	1328

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1329

A.7.3.40.1	Test Purpose and Environment	1329

A.7.3.40.2	Test Requirements	1331

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1332

A.7.3.41.1	Test Purpose and Environment	1332

A.7.3.41.2	Test Requirements	1335

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1335

A.7.3.42.1	Test Purpose and Environment	1335

A.7.3.42.2	Test Requirements	1338

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1338

A.7.3.43.1	Test Purpose and Environment	1338

A.7.3.43.2	Test Requirements	1341

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1341

A.7.3.44.1	Test Purpose and Environment	1341

A.7.3.44.2	Test Requirements	1344

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1345

A.7.3.45.1	Test Purpose and Environment	1345

A.7.3.45.2	Test Requirements	1347

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1348

A.7.3.46.1	Test Purpose and Environment	1348

A.7.3.46.2	Test Requirements	1351

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1351

A.7.3.47.1	Test Purpose and Environment	1351

A.7.3.47.2	Test Requirements	1354

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1354

A.7.3.48.1	Test Purpose and Environment	1354

A.7.3.48.2	Test Requirements	1357

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1357

A.7.3.49.1	Test Purpose and Environment	1357

A.7.3.49.2	Test Requirements	1360

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1360

A.7.3.50.1	Test Purpose and Environment	1360

A.7.3.50.2	Test Requirements	1363

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1363

A.7.3.51.1	Test Purpose and Environment	1363

A.7.3.51.2	Test Requirements	1366

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1366

A.7.3.52.1	Test Purpose and Environment	1366

A.7.3.52.2	Test Requirements	1369

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1369

A.7.3.53.1	Test Purpose and Environment	1369

A.7.3.53.2	Test Requirements	1372

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1372

A.7.3.54.1	Test Purpose and Environment	1372

A.7.3.54.2	Test Requirements	1375

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1375

A.7.3.55.1	Test Purpose and Environment	1375

A.7.3.55.2	Test Requirements	1378

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1378

A.7.3.56.1	Test Purpose and Environment	1378

A.7.3.56.2	Test Requirements	1381

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1381

A.7.3.57.1	Test Purpose and Environment	1381

A.7.3.57.2	Test Requirements	1384

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1384

A.7.3.58.1	Test Purpose and Environment	1384

A.7.3.58.2	Test Requirements	1387

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1387

A.7.3.59.1	Test Purpose and Environment	1387

A.7.3.59.2	Test Requirements	1390

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1390

A.7.3.60.1	Test Purpose and Environment	1390

A.7.3.60.2	Test Requirements	1394

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1394

A.7.3.61.1	Test Purpose and Environment	1394

A.7.3.61.2	Test Requirements	1398

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1398

A.7.3.62.1	Test Purpose and Environment	1398

A.7.3.62.2	Test Requirements	1402

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1402

A.7.3.63.1	Test Purpose and Environment	1402

A.7.3.63.2	Test Requirements	1406

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1406

A.7.3.64.1	Test Purpose and Environment	1406

A.7.3.64.2	Test Requirements	1409

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1410

A.7.3.65.1	Test Purpose and Environment	1410

A.7.3.65.2	Test Requirements	1413

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1414

A.7.3.66.1	Test Purpose and Environment	1414

A.7.3.66.2	Test Requirements	1416

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1417

A.7.3.67.1	Test Purpose and Environment	1417

A.7.3.67.2	Test Requirements	1420

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1421

A.7.3.68.1	Test Purpose and Environment	1421

A.7.3.68.2	Test Requirements	1423

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1423

A.7.3.69.1	Test Purpose and Environment	1423

A.7.3.69.2	Test Requirements	1425

The rate of correct events observed during repeated tests shall be at least 90%.A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1425

A.7.3.70.1	Test Purpose and Environment	1425

A.7.3.70.2	Test Requirements	1427

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1427

A.7.3.71.1	Test Purpose and Environment	1427

A.7.3.71.2	Test Requirements	1430

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1430

A.7.3.72.1	Test Purpose and Environment	1430

A.7.3.72.2	Test Requirements	1432

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1432

A.7.3.73.1	Test Purpose and Environment	1432

A.7.3.73.2	Test Requirements	1434

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1434

A.7.3.74.1	Test Purpose and Environment	1434

A.7.3.74.2	Test Requirements	1437

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1437

A.7.3.75.1	Test Purpose and Environment	1437

A.7.3.75.2	Test Requirements	1439

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1440

A.7.3.76.1	Test Purpose and Environment	1440

A.7.3.76.2	Test Requirements	1443

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1443

A.7.3.77.1	Test Purpose and Environment	1443

A.7.3.77.2	Test Requirements	1446

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1446

A.7.3.78.1	Test Purpose and Environment	1446

A.7.3.78.2	Test Requirements	1449

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1449

A.7.3.79.1	Test Purpose and Environment	1449

A.7.3.79.2	Test Requirements	1452

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1452

A.7.3.80.1	Test Purpose and Environment	1452

A.7.3.80.2	Test Requirements	1455

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1455

A.7.3.81.1	Test Purpose and Environment	1455

A.7.3.81.2	Test Requirements	1458

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1458

A.7.3.82.1	Test Purpose and Environment	1458

A.7.3.82.2	Test Requirements	1461

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1461

A.7.3.83.1	Test Purpose and Environment	1461

A.7.3.83.2	Test Requirements	1462

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1463

A.7.3.84.1	Test Purpose and Environment	1463

A.7.3.84.2	Test Requirements	1465

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1465

A.7.3.85.1	Test Purpose and Environment	1465

A.7.3.85.2	Test Requirements	1466

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1467

A.7.3.86.1	Test Purpose and Environment	1467

A.7.3.86.2	Test Requirements	1469

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1469

A.7.3.87.1	Test Purpose and Environment	1469

A.7.3.87.2	Test Requirements	1471

A.7.4	Interruption for Dual Connectivity	1472

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1472

A.7.4.1.1	Test Purpose and Environment	1472

A.7.4.1.2	Test Requirements	1474

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1474

A.7.4.2.1	Test Purpose and Environment	1474

A.7.4.2.2	Test Requirements	1476

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1476

A.7.4.3.1	Test Purpose and Environment	1476

A.7.4.3.2	Test Requirements	1478

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1478

A.7.4.4.1	Test Purpose and Environment	1478

A.7.4.4.2	Test Requirements	1481

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1481

A.7.4.5.1	Test Purpose and Environment	1481

A.7.4.5.2	Test Requirements	1483

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1483

A.7.4.6.1	Test Purpose and Environment	1483

A.7.4.6.2	Test Requirements	1485

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1485

A.7.4.7.1	Test Purpose and Environment	1485

A.7.4.7.2	Test Requirements	1489

A.7.5	Proximity-based Services	1489

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1489

A.7.5.1.1	Test Purpose and Environment	1489

A.7.5.1.2	Test Requirements	1490

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1491

A.7.5.2.1	Test Purpose and Environment	1491

A.7.5.1.2	Test Requirements	1491

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1492

A.7.5.3.1	Test Purpose and Environment	1492

A.7.5.3.2	Test Requirements	1493

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1494

A.7.5.4.1	Test Purpose and Environment	1494

A.7.5.4.2	Test Requirements	1495

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1496

A.7.5.5.1	Test Purpose and Environment	1496

A.7.5.5.2	Test Requirements	1498

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1499

A.7.5.6.1	Test Purpose and Environment	1499

A.7.5.6.2	Test Requirements	1500

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1501

A.7.5.7.1	Test Purpose and Environment	1501

A.7.5.7.2	Test Requirements	1503

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1503

A.7.5.8.1	Test Purpose and Environment	1503

A.7.5.8.2	Test Requirements	1505

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1506

A.7.5.9.1	Test Purpose and Environment	1506

A.7.5.9.2	Test Requirements	1508

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1509

A.7.5.10.1	Test Purpose and Environment	1509

A.7.5.10.2	Test Requirements	1511

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1512

A.7.5.11.1	Test Purpose and Environment	1512

A.7.5.11.2	Test Requirements	1514

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1514

A.7.5.12.1	Test Purpose and Environment	1514

A.7.5.12.2	Test Requirements	1518

A.7.6	Interruption for carrier aggregation	1519

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1519

A.7.6.1.1	Test Purpose and Environment	1519

A.7.6.1.2	Test Requirements	1522

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1522

A.7.6.2.1	Test Purpose and Environment	1522

A.7.6.2.2	Test Requirements	1525

A.8	UE Measurements Procedures	1525

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1525

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1525

A.8.1.1.1	Test Purpose and Environment	1525

A.8.1.1.2	Test Requirements	1527

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1527

A.8.1.2.1	Test Purpose and Environment	1527

A.8.1.2.2	Test Requirements	1529

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1529

A.8.1.3.1	Test Purpose and Environment	1529

A.8.1.3.2	Test Requirements	1532

A.8.1.4	Void	1532

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1532

A.8.1.5.1	Test Purpose and Environment	1532

A.8.1.5.2	Test Requirements	1534

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1534

A.8.1.6.1	Test Purpose and Environment	1534

A.8.1.6.2	Test Requirements	1536

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1536

A.8.1.7.1	Test Purpose and Environment	1536

A.8.1.7.2	Test Requirements	1538

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1539

A.8.1.8.1	Test Purpose and Environment	1539

A.8.1.8.2	Test Requirements	1541

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1542

A.8.1.9.1	Test Purpose and Environment	1542

A.8.1.9.2	Test Requirements	1542

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1542

A.8.1.10.1	Test Purpose and Environment	1542

A.8.1.10.2	Test Requirements	1543

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1543

A.8.1.11.1	Test Purpose and Environment	1543

A.8.1.11.2	Test Requirements	1545

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1546

A.8.1.12.1	Test Purpose and Environment	1546

A.8.1.12.2	Test Requirements	1547

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1548

A.8.1.13.1	Test Purpose and Environment	1548

A.8.1.13.2	Test Requirements	1550

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1550

A.8.1.14.1	Test Purpose and Environment	1550

A.8.1.14.2	Test Requirements	1552

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1553

A.8.1.15.1	Test Purpose and Environment	1553

A.8.1.15.2	Test Requirements	1554

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1555

A.8.1.16.1	Test Purpose and Environment	1555

A.8.1.16.2	Test Requirements	1557

A.8.1.17	Void	1557

A.8.1.18	Void	1557

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1557

A.8.1.19.1	Test Purpose and Environment	1557

A.8.1.19.2	Test Requirements	1559

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1560

A.8.1.20.1	Test Purpose and Environment	1560

A.8.1.20.2	Test Requirements	1562

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1562

A.8.1.21.1	Test Purpose and Environment	1562

A.8.1.21.2	Test Requirements	1564

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1565

A.8.1.22.1	Test Purpose and Environment	1565

A.8.1.22.2	Test Requirements	1567

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1567

A.8.1.23.1	Test Purpose and Environment	1567

A.8.1.23.2	Test Requirements	1568

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1569

A.8.1.24.1	Test Purpose and Environment	1569

A.8.1.24.2	Test Requirements	1571

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1571

A.8.1.25.1	Test Purpose and Environment	1571

A.8.1.25.2	Test Requirements	1573

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1573

A.8.1.26.1	Test Purpose and Environment	1573

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1576

A.8.1.27.1	Test Purpose and Environment	1576

A.8.1.27.2	Test Requirements	1578

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1578

A.8.1.28.1	Test Purpose and Environment	1578

A.8.1.28.2	Test Requirements	1580

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1580

A.8.1.29.1	Test Purpose and Environment	1580

A.8.1.29.2	Test Requirements	1582

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1583

A.8.1.30.1	Test Purpose and Environment	1583

A.8.1.30.2	Test Requirements	1585

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1585

A.8.1.31.1	Test Purpose and Environment	1585

A.8.1.31.2	Test Requirements	1587

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1588

A.8.1.32.1	Test Purpose and Environment	1588

A.8.1.32.2	Test Requirements	1589

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1590

A.8.1.33.1	Test Purpose and Environment	1590

A.8.1.33.2	Test Requirements	1591

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1592

A.8.1.34.1	Test Purpose and Environment	1592

A.8.1.34.2	Test Requirements	1593

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1594

A.8.1.35.1	Test Purpose and Environment	1594

A.8.1.35.2	Test Requirements	1595

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1596

A.8.1.36.1	Test Purpose and Environment	1596

A.8.1.36.2	Test Requirements	1597

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1598

A.8.1.37.1	Test Purpose and Environment	1598

A.8.1.37.2	Test Requirements	1600

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1600

A.8.1.38.1	Test Purpose and Environment	1600

A.8.1.38.2	Test Requirements	1601

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1602

A.8.1.39.1	Test Purpose and Environment	1602

A.8.1.39.2	Test Requirements	1604

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1604

A.8.1.40.1	Test Purpose and Environment	1604

A.8.1.40.2	Test Requirements	1606

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1606

A.8.1.41.1	Test Purpose and Environment	1606

A.8.1.41.2	Test Requirement	1608

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1608

A.8.1.42.1	Test Purpose and Environment	1608

A.8.1.42.2	Test Requirement	1609

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1610

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1610

A.8.2.1.1	Test Purpose and Environment	1610

A.8.2.1.2	Test Requirements	1611

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1611

A.8.2.2.1	Test Purpose and Environment	1611

A.8.2.2.2	Test Requirements	1614

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1614

A.8.2.3.1	Test Purpose and Environment	1614

A.8.2.3.2	Test Requirements	1616

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1617

A.8.2.4.1	Test Purpose and Environment	1617

A.8.2.4.2	Test Requirements	1619

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1619

A.8.2.5.1	Test Purpose and Environment	1619

A.8.2.5.2	Test Requirements	1621

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1622

A.8.2.6.1	Test Purpose and Environment	1622

A.8.2.6.2	Test Requirements	1625

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1626

A.8.2.7.1	Test Purpose and Environment	1626

A.8.2.7.2	Test Requirements	1627

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1628

A.8.2.8.1	Test Purpose and Environment	1628

A.8.2.8.2	Test Requirements	1630

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1630

A.8.2.9.1	Test Purpose and Environment	1630

A.8.2.9.2	Test Requirements	1632

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1633

A.8.2.10.1	Test Purpose and Environment	1633

A.8.2.10.2	Test Requirements	1635

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1635

A.8.2.11.1	Test Purpose and Environment	1635

A.8.2.11.2	Test Requirements	1637

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1637

A.8.2.12.1	Test Purpose and Environment	1637

A.8.2.12.2	Test Requirements	1639

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1640

A.8.2.13.1	Test Purpose and Environment	1640

A.8.2.13.2	Test Requirements	1642

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1642

A.8.2.14.1	Test Purpose and Environment	1642

A.8.2.14.2	Test Requirement	1644

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1645

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1645

A.8.3.1.1	Test Purpose and Environment	1645

A.8.3.1.2	Test Requirements	1646

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1646

A.8.3.2.1	Test Purpose and Environment	1646

A.8.3.2.2	Test Requirements	1649

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1649

A.8.3.3.1	Test Purpose and Environment	1649

A.8.3.3.2	Test Requirements	1652

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1652

A.8.3.4.1	Test Purpose and Environment	1652

A.8.3.4.2	Test Requirements	1654

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1654

A.8.3.5.2	Test Requirements	1656

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1656

A.8.3.6.1	Test Purpose and Environment	1656

A.8.3.6.2	Test Requirements	1658

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1659

A.8.3.7.1	Test Purpose and Environment	1659

A.8.3.7.2	Test Requirements	1661

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1662

A.8.3.8.1	Test Purpose and Environment	1662

A.8.3.8.2	Test Requirements	1665

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1665

A.8.3.9.1	Test Purpose and Environment	1665

A.8.3.9.2	Test Requirements	1669

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1670

A.8.3.10.1	Test Purpose and Environment	1670

A.8.3.10.2	Test Requirements	1671

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1671

A.8.3.11.1	Test Purpose and Environment	1671

A.8.3.11.2	Test Requirement	1673

A.8.3.12	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1674

A.8.3.12.1	Test Purpose and Environment	1674

A.8.3.12.2	Test Requirement	1675

A.8.3.13	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1676

A.8.3.13.1	Test Purpose and Environment	1676

A.8.3.13.2	Test Requirement	1677

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1678

A.8.3.14.1	Test Purpose and Environment	1678

A.8.3.14.2	Test Requirement	1679

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1680

A.8.3.15.1	Test Purpose and Environment	1680

A.8.3.15.2	Test Requirement	1681

A.8.3.16	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1682

A.8.3.16.1	Test Purpose and Environment	1682

A.8.3.16.2	Test Requirement	1684

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1684

A.8.3.17.1	Test Purpose and Environment	1684

A.8.3.17.2	Test Requirement	1687

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1687

A.8.3.18.1	Test Purpose and Environment	1687

A.8.3.18.2	Test Requirement	1690

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1690

A.8.3.19.1	Test Purpose and Environment	1690

A.8.3.19.2	Test Requirement	1693

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1693

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1693

A.8.4.1.1	Test Purpose and Environment	1693

A.8.4.1.2	Test Requirements	1695

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1695

A.8.4.2.1	Test Purpose and Environment	1695

A.8.4.2.2	Test Requirements	1698

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1698

A.8.4.3.1	Test Purpose and Environment	1698

A.8.4.3.2	Test Requirements	1700

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1701

A.8.4.4.1	Test Purpose and Environment	1701

A.8.4.4.2	Test Requirements	1703

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1704

A.8.4.5.1	Test Purpose and Environment	1704

A.8.4.5.2	Test Requirements	1706

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1706

A.8.4.6.1	Test Purpose and Environment	1706

A.8.4.6.2	Test Requirements	1707

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1707

A.8.4.7.1	Test Purpose and Environment	1707

A.8.4.7.2	Test Requirements	1711

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1711

A.8.4.8.1	Test Purpose and Environment	1711

A.8.4.8.2	Test Requirements	1714

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1714

A.8.4.9.1	Test Purpose and Environment	1714

A.8.4.9.2	Test Requirements	1719

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1719

A.8.4.10.1	Test Purpose and Environment	1719

A.8.4.10.2	Test Requirements	1721

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1721

A.8.4.11.1	Test Purpose and Environment	1721

A.8.4.11.2	Test Requirement	1723

A.8.4.12	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1723

A.8.4.12.1	Test Purpose and Environment	1723

A.8.4.12.2	Test Requirement	1725

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1726

A.8.4.13.1	Test Purpose and Environment	1726

A.8.4.13.2	Test Requirement	1727

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1728

A.8.4.14.1	Test Purpose and Environment	1728

A.8.4.14.2	Test Requirement	1730

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1730

A.8.4.15.1	Test Purpose and Environment	1730

A.8.4.15.2	Test Requirement	1733

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1733

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1733

A.8.5.1.1	Test Purpose and Environment	1733

A.8.5.1.2	Test Requirements	1735

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1735

A.8.5.2.1	Test Purpose and Environment	1735

A.8.5.2.2	Test Requirements	1737

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1737

A.8.5.3.1	Test Purpose and Environment	1737

A.8.5.3.2	Test Requirements	1740

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1740

A.8.5.4.1	Test Purpose and Environment	1740

A.8.5.4.2	Test Requirements	1742

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1742

A.8.5.5.1	Test Purpose and Environment	1742

A.8.5.5.2	Test Requirements	1745

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1745

A.8.5.6.1	Test Purpose and Environment	1745

A.8.5.6.2	Test Requirements	1746

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1747

A.8.5.7.1	Test Purpose and Environment	1747

A.8.5.7.2	Test Requirements	1747

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1747

A.8.5.8.1	Test Purpose and Environment	1747

A.8.5.8.2	Test Requirements	1750

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1750

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1750

A.8.6.1.1	Test Purpose and Environment	1750

A.8.6.1.2	Test Requirements	1752

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1753

A.8.6.2.1	Test Purpose and Environment	1753

A.8.6.2.2	Test Requirements	1755

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1755

A.8.6.3.1	Test Purpose and Environment	1755

A.8.6.3.2	Test Requirements	1758

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1758

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1758

A.8.7.1.1	Test Purpose and Environment	1758

A.8.7.1.1.1	Void	1758

A.8.7.1.1.2	1.28 Mcps TDD option	1758

A.8.7.1.1.3	Void	1760

A.8.7.1.2	Test Requirements	1760

A.8.7.1.2.1	Void	1760

A.8.7.1.2.2	1.28 Mcps TDD option	1760

A.8.7.1.2.3	Void	1760

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1760

A.8.7.2.1	Test Purpose and Environment	1760

A.8.7.2.2	Test Requirements	1763

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1764

A.8.7.3.1	Test Purpose and Environment	1764

A.8.7.3.2	Test Parameters	1764

A.8.7.3.3	Test Requirements	1765

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1766

A.8.7.4.1	Test Purpose and Environment	1766

A.8.7.4.2	Test Requirements	1768

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1768

A.8.7.5.1	Test Purpose and Environment	1768

A.8.7.5.2	Test Requirements	1770

A.8.7A	TBD	1770

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1770

A.8.7A.1.1	Test Purpose and Environment	1770

A.8.7A.1.2	Test Requirements	1773

A.8.8	E-UTRAN FDD – GSM Measurements	1773

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1773

A.8.8.1.1	Test Purpose and Environment	1773

A.8.8.1.2	Test Requirements	1775

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1775

A.8.8.2.1	Test Purpose and Environment	1775

A.8.8.2.2	Test Requirements	1777

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1778

A.8.8.3.1	Test Purpose and Environment	1778

A.8.8.3.2	Test Requirements	1779

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1780

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1780

A.8.9.1.1	Test Purpose and Environment	1780

A.8.9.1.2	Test Requirements	1781

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1782

A.8.9.2.1	Test Purpose and Environment	1782

A.8.9.2.2	Test Requirements	1784

A.8.10	E-UTRAN TDD – GSM Measurements	1784

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1784

A.8.10.1.1	Test Purpose and Environment	1784

A.8.10.1.2	Test Requirements	1785

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1786

A.8.10.2.1	Test Purpose and Environment	1786

A.8.10.2.2	Test Requirements	1788

A.8.11	Monitoring of Multiple Layers	1788

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1788

A.8.11.1.1	Test Purpose and Environment	1788

A.8.11.1.2	Test Requirements	1790

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1791

A.8.11.2.1	Test Purpose and Environment	1791

A.8.11.2.2	Test Requirements	1792

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1792

A.8.11.3.1	Test Purpose and Environment	1792

A.8.11.3.2	Test Requirements	1795

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1795

A.8.11.4.1	Test Purpose and Environment	1795

A.8.11.4.2	Test Requirements	1798

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1798

A.8.11.5.1	Test Purpose and Environment	1798

A.8.11.5.2	Test Requirements	1800

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1801

A.8.11.6.1	Test Purpose and Environment	1801

A.8.11.6.2	Test Requirements	1803

A.8.12	RSTD Intra-frequency Measurements	1804

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1804

A.8.12.1.1	Test Purpose and Environment	1804

A.8.12.1.2	Test Requirements	1808

A.8.12.1.2A	Test Requirements for UE Category 1bis	1808

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1808

A.8.12.2.1	Test Purpose and Environment	1808

A.8.12.2.2	Test Requirements	1812

A.8.12.2.2A	Test Requirements for UE Category 1bis	1813

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	1813

A.8.12.3.1	Test Purpose and Environment	1813

A.8.12.3.2	Test Requirements	1819

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	1819

A.8.12.4.1	Test Purpose and Environment	1819

A.8.12.4.2	Test Requirements	1824

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	1824

A.8.12.5.1	Test Purpose and Environment	1824

A.8.12.5.2	Test Requirements	1829

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	1829

A.8.12.6.1	Test Purpose and Environment	1829

A.8.12.6.2	Test Requirements	1834

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	1834

A.8.12.7.1	Test Purpose and Environment	1834

A.8.12.7.2	Test Requirements	1839

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	1839

A.8.12.8.1	Test Purpose and Environment	1839

A.8.12.8.2	Test Requirements	1844

A.8.13	RSTD Inter-frequency Measurements	1844

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1844

A.8.13.1.1	Test Purpose and Environment	1844

A.8.13.1.2	Test Requirements	1849

A.8.13.1.2A	Test Requirements for UE Category 1bis	1849

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1849

A.8.13.2.1	Test Purpose and Environment	1849

A.8.13.2.2	Test Requirements	1855

A.8.13.2.2A	Test Requirements for UE Category 1bis	1855

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	1855

A.8.13.3.1	Test Purpose and Environment	1855

A.8.13.3.2	Test Requirements	1861

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	1861

A.8.13.4.1	Test Purpose and Environment	1861

A.8.13.4.2	Test Requirements	1866

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	1866

A.8.13.5.1	Test Purpose and Environment	1866

A.8.13.5.2	Test Requirements	1872

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	1872

A.8.13.6.1	Test Purpose and Environment	1872

A.8.13.6.2	Test Requirements	1877

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	1877

A.8.13.7.1	Test Purpose and Environment	1877

A.8.13.7.2	Test Requirements	1882

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	1882

A.8.13.8.1	Test Purpose and Environment	1882

A.8.13.8.2	Test Requirements	1888

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	1888

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1888

A.8.14.1.1	Test Purpose and Environment	1888

A.8.14.1.2	Test Requirements	1890

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1890

A.8.14.2.1	Test Purpose and Environment	1890

A.8.14.2.2	Test Requirements	1893

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1893

A.8.14.3.1	Test Purpose and Environment	1893

A.8.14.3.2	Test Requirements	1895

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	1896

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1896

A.8.15.1.1	Test Purpose and Environment	1896

A.8.15.1.2	Test Requirements	1897

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1897

A.8.15.2.1	Test Purpose and Environment	1897

A.8.15.2.2	Test Requirements	1900

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1900

A.8.15.3.1	Test Purpose and Environment	1900

A.8.15.3.2	Test Requirements	1902

A.8.16	E-UTRAN Carrier Aggregation Measurements	1903

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	1903

A.8.16.1.1	Test Purpose and Environment	1903

A.8.16.1.2	Test Requirements	1905

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	1905

A.8.16.2.1	Test Purpose and Environment	1905

A.8.16.2.2	Test Requirements	1907

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1907

A.8.16.3.1	Test Purpose and Environment	1907

A.8.16.3.2	Test Requirements	1909

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	1910

A.8.16.3A.1	Test Purpose and Environment	1910

A.8.16.3A.2	Test Requirements	1911

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	1912

A.8.16.4.1	Test Purpose and Environment	1912

A.8.16.4.2	Test Requirements	1913

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1914

A.8.16.4A.1	Test Purpose and Environment	1914

A.8.16.4A.2	Test Requirements	1916

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1917

A.8.16.5.1	Test Purpose and Environment	1917

A.8.16.5.2	Test Requirements	1917

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1917

A.8.16.6.1	Test Purpose and Environment	1917

A.8.16.6.2	Test Requirements	1918

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1918

A.8.16.7.1	 Test Purpose and Environment	1918

A.8.16.7.2	 Test Requirements	1919

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1919

A.8.16.8.1	 Test Purpose and Environment	1919

A.8.16.8.2	 Test Requirements	1920

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1920

A.8.16.9.1	Test Purpose and Environment	1920

A.8.16.9.2	Test Requirements	1921

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1921

A.8.16.10.1	Test Purpose and Environment	1921

A.8.16.10.2	Test Requirements	1922

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1922

A.8.16.11.1	 Test Purpose and Environment	1922

A.8.16.11.2	 Test Requirements	1923

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1923

A.8.16.12.1	 Test Purpose and Environment	1923

A.8.16.12.2	 Test Requirements	1923

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	1923

A.8.16.13.1	Test Purpose and Environment	1923

A.8.16.13.2	Test Requirements	1924

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	1924

A.8.16.14.1	Test Purpose and Environment	1924

A.8.16.14.2	Test Requirements	1925

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	1925

A.8.16.15.1	 Test Purpose and Environment	1925

A.8.16.7.2	 Test Requirements	1925

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	1925

A.8.16.16.1	 Test Purpose and Environment	1925

A.8.16.16.2	 Test Requirements	1926

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	1926

A.8.16.17.1	Test Purpose and Environment	1926

A.8.16.17.2	Test Requirements	1928

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	1929

A.8.16.17A.1	Test Purpose and Environment	1929

A.8.16.17A.2	Test Requirements	1929

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1929

A.8.16.17B.1	Test Purpose and Environment	1929

A.8.16.17B.2	Test Requirements	1930

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1930

A.8.16.17C.1	Test Purpose and Environment	1930

A.8.16.17C.2	Test Requirements	1930

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	1930

A.8.16.18.1	Test Purpose and Environment	1930

A.8.16.18.2	Test Requirements	1932

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	1933

A.8.16.18A.1	Test Purpose and Environment	1933

A.8.16.18A.2	Test Requirements	1933

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1933

A.8.16.18B.1	Test Purpose and Environment	1933

A.8.16.18B.2	Test Requirements	1934

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1934

A.8.16.18C.1	Test Purpose and Environment	1934

A.8.16.18C.2	Test Requirements	1934

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	1934

A.8.16.18D.1	Test Purpose and Environment	1934

A.8.16.18D.2	Test Requirements	1935

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	1935

A.8.16.19.1	Test Purpose and Environment	1935

A.8.16.19.2	Test Requirements	1937

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1937

A.8.16.19A.1	Test Purpose and Environment	1937

A.8.16.19A.2	Test Requirements	1938

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1938

A.8.16.19B.1	Test Purpose and Environment	1938

A.8.16.19B.2	Test Requirements	1938

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1938

A.8.16.19C.1	Test Purpose and Environment	1938

A.8.16.19C.2	Test Requirements	1939

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	1939

A.8.16.20.1	Test Purpose and Environment	1939

A.8.16.20.2	Test Requirements	1941

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1942

A.8.16.20A.1	Test Purpose and Environment	1942

A.8.16.20A.2	Test Requirements	1942

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1942

A.8.16.20B.1	Test Purpose and Environment	1942

A.8.16.20B.2	Test Requirements	1943

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1943

A.8.16.20C.1	Test Purpose and Environment	1943

A.8.16.20C.2	Test Requirements	1943

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	1943

A.8.16.20D.1	Test Purpose and Environment	1943

A.8.16.20D.2	Test Requirements	1944

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	1944

A.8.16.21.1	Test Purpose and Environment	1944

A.8.16.21.2	Test Requirements	1946

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	1946

A.8.16.22.1	Test Purpose and Environment	1946

A.8.16.22.2	Test Requirements	1948

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	1948

A.8.16.23.1	Test Purpose and Environment	1948

A.8.16.23.2	Test Requirements	1951

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	1951

A.8.16.24.1	Test Purpose and Environment	1951

A.8.16.24.2	Test Requirements	1954

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	1954

A.8.16.25.1	Test Purpose and Environment	1954

A.8.16.25.2	Test Requirements	1957

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	1957

A.8.16.26.1	Test Purpose and Environment	1957

A.8.16.26.2	Test Requirements	1960

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1960

A.8.16.27.1	Test Purpose and Environment	1960

A.8.16.27.2	Test Requirements	1965

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1965

A.8.16.28.1	Test Purpose and Environment	1965

A.8.16.28.2	Test Requirements	1970

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1970

A.8.16.29.1	Test Purpose and Environment	1970

A.8.16.29.2	Test Requirements	1975

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1975

A.8.16.30.1	Test Purpose and Environment	1975

A.8.16.30.2	Test Requirements	1980

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	1980

A.8.16.31.1	Test Purpose and Environment	1980

A.8.16.31.2	Test Requirements	1985

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	1985

A.8.16.32.1	Test Purpose and Environment	1985

A.8.16.32.2	Test Requirements	1990

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1990

A.8.16.33.1	Test Purpose and Environment	1990

A.8.16.33.2	Test Requirements	1995

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1995

A.8.16.34.1	Test Purpose and Environment	1995

A.8.16.34.2	Test Requirements	2000

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2000

A.8.16.35.1	Test Purpose and Environment	2000

A.8.16.35.2	Test Requirements	2002

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2003

A.8.16.36.1	Test Purpose and Environment	2003

A.8.16.36.2	Test Requirements	2005

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2006

A.8.16.37.1	Test Purpose and Environment	2006

A.8.16.37.2	Test Requirements	2008

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2009

A.8.16.38.1	Test Purpose and Environment	2009

A.8.16.38.2	Test Requirements	2011

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2012

A.8.16.39.1	Test Purpose and Environment	2012

A.8.16.39.2	Test Requirements	2015

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2015

A.8.16.40.1	Test Purpose and Environment	2015

A.8.16.40.2	Test Requirements	2018

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2018

A.8.16.41.1	Test Purpose and Environment	2018

A.8.16.41.2	Test Requirements	2020

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2021

A.8.16.42.1	Test Purpose and Environment	2021

A.8.16.42.2	Test Requirements	2023

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2024

A.8.16.43.1	Test Purpose and Environment	2024

A.8.16.43.2	Test Requirements	2027

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2027

A.8.16.44.1	Test Purpose and Environment	2027

A.8.16.44.2	Test Requirements	2030

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2030

A.8.16.45.1	Test Purpose and Environment	2030

A.8.16.45.2	Test Requirements	2033

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2033

A.8.16.46.1	Test Purpose and Environment	2033

A.8.16.46.2	Test Requirements	2035

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2036

A.8.16.47.1	Test Purpose and Environment	2036

A.8.16.47.2	Test Requirements	2038

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2039

A.8.16.48.1	Test Purpose and Environment	2039

A.8.16.48.2	Test Requirements	2041

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2042

A.8.16.49.1	Test Purpose and Environment	2042

A.8.16.49.2	Test Requirements	2045

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2045

A.8.16.50.1	Test Purpose and Environment	2045

A.8.16.50.2	Test Requirements	2047

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2048

A.8.16.51.1	Test Purpose and Environment	2048

A.8.16.51.2	Test Requirements	2053

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2053

A.8.16.52.1	Test Purpose and Environment	2053

A.8.16.52.2	Test Requirements	2058

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2058

A.8.16.53.1	Test Purpose and Environment	2058

A.8.16.53.2	Test Requirements	2062

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2062

A.8.16.54.1	Test Purpose and Environment	2062

A.8.16.54.2	Test Requirements	2066

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2066

A.8.16.55.1	Test Purpose and Environment	2066

A.8.16.55.2	Test Requirements	2072

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2072

A.8.16.56.1	Test Purpose and Environment	2072

A.8.16.56.2	Test Requirements	2078

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2078

A.8.16.57.1	Test Purpose and Environment	2078

A.8.16.57.2	Test Requirements	2080

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2081

A.8.16.58.1	Test Purpose and Environment	2081

A.8.16.58.2	Test Requirements	2083

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2084

A.8.16.59.1	Test Purpose and Environment	2084

A.8.16.59.2	Test Requirements	2088

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2089

A.8.16.60.1	Test Purpose and Environment	2089

A.8.16.60.2	Test Requirements	2092

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2093

A.8.16.61.1	Test Purpose and Environment	2093

A.8.16.61.2	Test Requirements	2096

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2096

A.8.16.62.1	Test Purpose and Environment	2096

A.8.16.62.2	Test Requirements	2098

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2099

A.8.16.63.1	Test Purpose and Environment	2099

A.8.16.63.2	Test Requirements	2102

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2103

A.8.16.64.1	Test Purpose and Environment	2103

A.8.16.64.2	Test Requirements	2107

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2107

A.8.16.65.1	Test Purpose and Environment	2107

A.8.16.65.2	Test Requirements	2112

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2112

A.8.16.66.1	Test Purpose and Environment	2112

A.8.16.66.2	Test Requirements	2117

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2117

A.8.16.67.1	Test Purpose and Environment	2117

A.8.16.67.2	Test Requirements	2122

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2122

A.8.16.68.1	Test Purpose and Environment	2122

A.8.16.68.2	Test Requirements	2127

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2127

A.8.16.69.1	Test Purpose and Environment	2127

A.8.16.69.2	Test Requirements	2131

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2131

A.8.16.70.1	Test Purpose and Environment	2131

A.8.16.70.2	Test Requirements	2135

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2135

A.8.16.71.1	Test Purpose and Environment	2135

A.8.16.71.2	Test Requirements	2140

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2140

A.8.16.72.1	Test Purpose and Environment	2140

A.8.16.72.2	Test Requirements	2145

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2145

A.8.16.73.1	Test Purpose and Environment	2145

A.8.16.73.2	Test Requirements	2151

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2151

A.8.16.74.1	Test Purpose and Environment	2151

A.8.16.74.2	Test Requirements	2157

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2157

A.8.16.75.1	Test Purpose and Environment	2157

A.8.16.75.2	Test Requirements	2163

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2163

A.8.16.76.1	Test Purpose and Environment	2163

A.8.16.76.2	Test Requirements	2169

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2169

A.8.16.77.1	Test Purpose and Environment	2169

A.8.16.77.2	Test Requirements	2173

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2173

A.8.16.78.1	Test Purpose and Environment	2173

A.8.16.78.2	Test Requirements	2175

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2176

A.8.16.79.1	Test Purpose and Environment	2176

A.8.16.79.2	Test Requirements	2182

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2182

A.8.16.80.1	Test Purpose and Environment	2182

A.8.16.80.2	Test Requirements	2188

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2188

A.8.16.81.1	Test Purpose and Environment	2188

A.8.16.81.2	Test Requirements	2194

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2194

A.8.16.82.1	Test Purpose and Environment	2194

A.8.16.82.2	Test Requirements	2200

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2200

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2200

A.8.17.1.1	Test Purpose and Environment	2200

A.8.17.1.2	Test Requirements	2206

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2206

A.8.17.2.1	Test Purpose and Environment	2206

A.8.17.2.2	Test Requirements	2213

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2213

A.8.17.3.1	Test Purpose and Environment	2213

A.8.17.3.2	Test Requirements	2214

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2214

A.8.17.4.1	Test Purpose and Environment	2214

A.8.17.4.2	Test Requirements	2215

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2215

A.8.17.5.1	Test Purpose and Environment	2215

A.8.17.5.2	Test Requirements	2216

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2216

A.8.17.6.1	Test Purpose and Environment	2216

A.8.17.6.2	Test Requirements	2217

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2218

A.8.17.7.1	Test Purpose and Environment	2218

A.8.17.7.2	Test Requirements	2218

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2219

A.8.17.8.1	Test Purpose and Environment	2219

A.8.17.8.2	Test Requirements	2219

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2220

A.8.17.9.1	Test Purpose and Environment	2220

A.8.17.9.2	Test Requirements	2221

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2221

A.8.17.10.1	Test Purpose and Environment	2221

A.8.17.10.2	Test Requirements	2228

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2229

A.8.17.11.1	Test Purpose and Environment	2229

A.8.17.11.2	Test Requirements	2235

A.8.18	E-UTRAN TDD – HRPD Measurements	2236

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2236

A.8.18.1.1	Test Purpose and Environment	2236

A.8.18.1.2	Test Requirements	2238

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2238

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2238

A.8.19.1.1	Test Purpose and Environment	2238

A.8.19.1.2	Test Requirements	2239

A.8.20	Inter-frequency/RAT Measurements in CA mode	2240

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2240

A.8.20.1.1	Test Purpose and Environment	2240

A.8.20.1.2	Test Requirements	2241

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2242

A.8.20.2.1	Test Purpose and Environment	2242

A.8.20.2.2	Test Requirements	2243

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2244

A.8.20.2A.1	Test Purpose and Environment	2244

A.8.20.2A.2	Test Requirements	2244

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2244

A.8.20.2B.1	Test Purpose and Environment	2244

A.8.20.2B.2	Test Requirements	2247

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2247

A.8.20.3.1	Test Purpose and Environment	2247

A.8.20.3.2	Test Requirements	2249

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2249

A.8.20.4.1	Test Purpose and Environment	2249

A.8.20.4.1.1	1.28 Mcps TDD option	2249

A.8.20.4.2	Test Requirements	2251

A.8.20.4.2.1	1.28 Mcps TDD option	2251

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2251

A.8.20.4A.1	Test Purpose and Environment	2251

A.8.20.4A.1.1	1.28 Mcps TDD option	2251

A.8.20.4A.2	Test Requirements	2251

A.8.20.4A.2.1	1.28 Mcps TDD option	2251

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2252

A.8.20.4B.1	Test Purpose and Environment	2252

A.8.20.4B.1.1	1.28 Mcps TDD option	2252

A.8.20.4B.2	Test Requirements	2254

A.8.20.4B.2.1	1.28 Mcps TDD option	2254

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2254

A.8.21.1	Test Purpose and Environment	2254

A.8.21.2 Test Requirements	2258

A.8.22	E-UTRAN Discovery Signal Measurements	2258

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2258

A.8.22.1.1	Test Purpose and Environment	2258

A.8.22.1.2	Test Requirements	2261

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2261

A.8.22.2.1	Test Purpose and Environment	2261

A.8.22.2.2	Test Requirements	2264

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2264

A.8.22.3.1	Test Purpose and Environment	2264

A.8.22.3.2	Test Requirements	2267

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2267

A.8.22.4.1	Test Purpose and Environment	2267

A.8.22.4.2	Test Requirements	2270

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2270

A.8.22.5.1	Test Purpose and Environment	2270

A.8.22.5.2	Test Requirements	2274

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2274

A.8.22.6.1	Test Purpose and Environment	2274

A.8.22.6.2	Test Requirements	2278

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2278

A.8.22.7.1	Test Purpose and Environment	2278

A.8.22.7.2	Test Requirements	2282

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2282

A.8.22.8.1	Test Purpose and Environment	2282

A.8.22.8.2	Test Requirements	2286

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2286

A.8.22.9.1	Test Purpose and Environment	2286

A.8.22.9.2	Test Requirements	2288

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2289

A.8.22.10.1	Test Purpose and Environment	2289

A.8.22.10.2	Test Requirements	2291

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2292

A.8.22.11.1	Test Purpose and Environment	2292

A.8.22.11.2	Test Requirements	2295

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2295

A.8.22.12.1	Test Purpose and Environment	2295

A.8.22.12.2	Test Requirements	2299

A.8.23	E-UTRAN Dual Connectivity Measurements	2299

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2299

A.8.23.1.1	Test Purpose and Environment	2299

A.8.23.1.2	Test Requirements	2302

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2302

A.8.23.2.1	Test Purpose and Environment	2302

A.8.23.2.2	Test Requirements	2305

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2305

A.8.23.3.1	Test Purpose and Environment	2305

A.8.23.3.2	Test Requirements	2308

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2308

A.8.23.4.1	Test Purpose and Environment	2308

A.8.23.4.2	Test Requirements	2311

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2311

A.8.23.5.1	Test Purpose and Environment	2311

A.8.23.5.2	Test Requirements	2314

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2314

A.8.23.6.1	Test Purpose and Environment	2314

A.8.23.6.2	Test Requirements	2317

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2317

A.8.23.7.1	Test Purpose and Environment	2317

A.8.23.7.2	Test Requirements	2319

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2320

A.8.23.8.1	Test Purpose and Environment	2320

A.8.23.8.2	Test Requirements	2322

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2323

A.8.23.9.1	Test Purpose and Environment	2323

A.8.23.9.2	Test Requirements	2326

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2326

A.8.23.10.1	Test Purpose and Environment	2326

A.8.23.10.2	Test Requirements	2329

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2329

A.8.23.11.1	Test Purpose and Environment	2329

A.8.23.11.2	Test Requirements	2332

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2332

A.8.23.12.1	Test Purpose and Environment	2332

A.8.23.12.2	Test Requirements	2335

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2335

A.8.23.13.1	Test Purpose and Environment	2335

A.8.23.13.2	Test Requirements	2338

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2338

A.8.23.14.1	Test Purpose and Environment	2338

A.8.23.14.2	Test Requirements	2341

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2341

A.8.23.15.1	Test Purpose and Environment	2341

A.8.23.15.2	Test Requirements	2344

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2344

A.8.23.16.1	Test Purpose and Environment	2344

A.8.23.16.2	Test Requirements	2345

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2346

A.8.23.17.1	Test Purpose and Environment	2346

A.8.23.17.2	Test Requirements	2348

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2348

A.8.23.18.1	Test Purpose and Environment	2348

A.8.23.18.2	Test Requirements	2349

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2350

A.8.23.19.1	Test Purpose and Environment	2350

A.8.23.19.2	Test Requirements	2351

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2352

A.8.23.20.1	Test Purpose and Environment	2352

A.8.23.20.2	Test Requirements	2353

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2354

A.8.23.21.1	Test Purpose and Environment	2354

A.8.23.21.2	Test Requirements	2355

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2356

A.8.23.22.1	Test Purpose and Environment	2356

A.8.23.22.2	Test Requirements	2358

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2358

A.8.23.23.1	Test Purpose and Environment	2358

A.8.23.23.2	Test Requirements	2360

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2361

A.8.23.24.1	Test Purpose and Environment	2361

A.8.23.24.2	Test Requirements	2363

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2364

A.8.23.25.1	Test Purpose and Environment	2364

A.8.23.25.2	Test Requirements	2366

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2367

A.8.23.26.1	Test Purpose and Environment	2367

A.8.23.26.2	Test Requirements	2369

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2370

A.8.23.27.1	Test Purpose and Environment	2370

A.8.23.27.2	Test Requirements	2373

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2373

A.8.23.28.1	Test Purpose and Environment	2373

A.8.23.28.2	Test Requirements	2377

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2377

A.8.23.29.1	Test Purpose and Environment	2377

A.8.23.29.2	Test Requirements	2381

A.8.24	Proximity-based Services	2381

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2381

A.8.24.1.1	Test Purpose and Environment	2381

A.8.24.1.2	Test Requirements	2382

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2383

A.8.24.2.1	Test Purpose and Environment	2383

A.8.24.2.2	Test Requirements	2384

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2384

A.8.24.3.1	Test Purpose and Environment	2385

A.8.24.3.2	Test Requirements	2386

A.8.25	E-UTRAN-WLAN Measurements	2387

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2387

A.8.25.1.1	Test Purpose and Environment	2387

A.8.25.1.2	Test Requirements	2389

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2389

A.8.25.2.1	Test Purpose and Environment	2389

A.8.25.2.2	Test Requirements	2391

A.8.26 Frame Structure 3 (FS3)	2391

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2391

A.8.26.1.1	Test Purpose and Environment	2391

A.8.26.1.2	Test Requirements	2393

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2394

A.8.26.2.1	Test Purpose and Environment	2394

A.8.26.2.2	Test Requirements	2396

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2397

A.8.26.3.1	Test Purpose and Environment	2397

A.8.26.3.2	Test Requirements	2400

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2400

A.8.26.3A.1	Test Purpose and Environment	2400

A.8.26.3A.2	Test Requirements	2403

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2403

A.8.26.4.1	Test Purpose and Environment	2403

A.8.26.4.2	Test Requirements	2407

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2407

A.8.26.4A.1	Test Purpose and Environment	2407

A.8.26.4A.2	Test Requirements	2411

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2411

A.8.26.5.1	Test Purpose and Environment	2411

A.8.26.5.2	Test Requirements	2415

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2415

A.8.26.5A.1	Test Purpose and Environment	2415

A.8.26.5A.2	Test Requirements	2418

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2418

A.8.26.6.1	Test Purpose and Environment	2418

A.8.26.6.2	Test Requirements	2422

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2422

A.8.26.6A.1	Test Purpose and Environment	2422

A.8.26.6A.2	Test Requirements	2426

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2426

A.8.26.7.1	Test Purpose and Environment	2426

A.8.26.7.2	Test Requirements	2429

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2429

A.8.26.8.1	Test Purpose and Environment	2429

A.8.26.8.2	Test Requirements	2433

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2433

A.8.26.9.1	Test Purpose and Environment	2433

A.8.26.9.2	Test Requirements	2436

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2436

A.8.26.10.1	Test Purpose and Environment	2436

A.8.26.10.2	Test Requirements	2439

A.9	Measurement Performance Requirements	2440

A.9.1	RSRP	2440

A.9.1.1	FDD Intra frequency case	2440

A.9.1.1.1	Test Purpose and Environment	2440

A.9.1.1.2	Test parameters	2440

A.9.1.1.3	Test Requirements	2442

A.9.1.2	TDD Intra frequency case	2442

A.9.1.2.1	Test Purpose and Environment	2442

A.9.1.2.2	Test parameters	2442

A.9.1.2.3	Test Requirements	2443

A.9.1.3	FDD—FDD Inter frequency case	2444

A.9.1.3.1	Test Purpose and Environment	2444

A.9.1.3.2	Test parameters	2444

A.9.1.3.3	Test Requirements	2446

A.9.1.4	TDD—TDD Inter frequency case	2446

A.9.1.4.1	Test Purpose and Environment	2446

A.9.1.4.2	Test parameters	2446

A.9.1.4.3	Test Requirements	2448

A.9.1.5	FDD—TDD Inter frequency case	2449

A.9.1.5.1	Test Purpose and Environment	2449

A.9.1.5.2	Test parameters	2449

A.9.1.5.3	Test Requirements	2450

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2451

A.9.1.6.1	Test Purpose and Environment	2451

A.9.1.6.2	Test parameters	2451

A.9.1.6.3	Test Requirements	2453

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2453

A.9.1.7.1	Test Purpose and Environment	2453

A.9.1.7.2	Test parameters	2453

A.9.1.7.3	Test Requirements	2455

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2455

A.9.1.8.1	Test Purpose and Environment	2455

A.9.1.8.2	Test parameters	2455

A.9.1.8.3	Test Requirements	2458

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2458

A.9.1.9.1	Test Purpose and Environment	2458

A.9.1.9.2	Test parameters	2458

A.9.1.9.3	Test Requirements	2461

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2461

A.9.1.10.1	Test Purpose and Environment	2461

A.9.1.10.2	Test parameters	2461

A.9.1.10.3	Test Requirements	2463

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2463

A.9.1.11.1	Test Purpose and Environment	2463

A.9.1.11.2	Test parameters	2463

A.9.1.11.3	Test Requirements	2466

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2466

A.9.1.12.1	Test Purpose and Environment	2466

A.9.1.12.2	Test parameters	2466

A.9.1.12.3	Test Requirements	2467

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2467

A.9.1.13.1	Test Purpose and Environment	2467

A.9.1.13.2	Test parameters	2467

A.9.1.13.3	Test Requirements	2468

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2468

A.9.1.14.1	Test Purpose and Environment	2468

A.9.1.14.2	Test parameters	2468

A.9.1.14.3	Test Requirements	2471

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2471

A.9.1.15.1	Test Purpose and Environment	2471

A.9.1.15.2	Test parameters	2471

A.9.1.15.3	Test Requirements	2473

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2474

A.9.1.16.1	Test Purpose and Environment	2474

A.9.1.16.2	Test parameters	2474

A.9.1.16.3	Test Requirements	2475

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2475

A.9.1.17.1	Test Purpose and Environment	2475

A.9.1.17.2	Test parameters	2476

A.9.1.17.3	Test Requirements	2477

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2477

A.9.1.18.1	Test Purpose and Environment	2477

A.9.1.18.2	Test parameters	2477

A.9.1.18.3	Test Requirements	2478

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2478

A.9.1.19.1	Test Purpose and Environment	2479

A.9.1.19.2	Test parameters	2479

A.9.1.19.3	Test Requirements	2479

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2479

A.9.1.20.1	Test Purpose and Environment	2479

A.9.1.20.2	Test parameters	2479

A.9.1.20.3	Test Requirements	2480

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2481

A.9.1.21.1	Test Purpose and Environment	2481

A.9.1.21.2	Test parameters	2481

A.9.1.21.3	Test Requirements	2481

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2481

A.9.1.22.1	Test Purpose and Environment	2481

A.9.1.22.2	Test parameters	2482

A.9.1.22.3	Test Requirements	2485

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2485

A.9.1.23.1	Test Purpose and Environment	2485

A.9.1.23.2	Test parameters	2485

A.9.1.23.3	Test Requirements	2487

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2488

A.9.1.24.1	Test Purpose and Environment	2488

A.9.1.24.2	Test parameters	2488

A.9.1.24.3	Test Requirements	2488

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2489

A.9.1.25.1	Test Purpose and Environment	2489

A.9.1.25.2	Test parameters	2489

A.9.1.25.3	Test Requirements	2491

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2491

A.9.1.26.1	Test Purpose and Environment	2491

A.9.1.26.2	Test parameters	2491

A.9.1.26.3	Test Requirements	2493

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2493

A.9.1.27.1	Test Purpose and Environment	2493

A.9.1.27.2	Test parameters	2493

A.9.1.27.3	Test Requirements	2495

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2495

A.9.1.28.1	Test Purpose and Environment	2495

A.9.1.28.2	Test parameters	2495

A.9.1.28.3	Test Requirements	2497

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2497

A.9.1.29.1	Test Purpose and Environment	2497

A.9.1.29.2	Test parameters	2497

A.9.1.29.3	Test Requirements	2499

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2499

A.9.1.30.1	Test Purpose and Environment	2499

A.9.1.30.2	Test parameters	2499

A.9.1.30.3	Test Requirements	2501

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2501

A.9.1.31.1	Test Purpose and Environment	2501

A.9.1.31.2	Test parameters	2501

A.9.1.31.3	Test Requirements	2503

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2503

A.9.1.32.1	Test Purpose and Environment	2503

A.9.1.32.2	Test parameters	2503

A.9.1.32.3	Test Requirements	2506

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2506

A.9.1.33.1	Test Purpose and Environment	2506

A.9.1.33.2	Test parameters	2506

A.9.1.33.3	Test Requirements	2508

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2508

A.9.1.34.1	Test Purpose and Environment	2508

A.9.1.34.2	Test parameters	2508

A.9.1.34.3	Test Requirements	2510

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2510

A.9.1.35.1	Test Purpose and Environment	2510

A.9.1.35.2	Test parameters	2510

A.9.1.35.3	Test Requirements	2512

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2513

A.9.1.36.1	Test Purpose and Environment	2513

A.9.1.36.2	Test parameters	2513

A.9.1.36.3	Test Requirements	2515

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2515

A.9.1.37.1	Test Purpose and Environment	2515

A.9.1.37.2	Test parameters	2515

A.9.1.37.3	Test Requirements	2517

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2518

A.9.1.38.1	Test Purpose and Environment	2518

A.9.1.38.2	Test parameters	2518

A.9.1.38.3	Test Requirements	2520

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2521

A.9.1.39.1	Test Purpose and Environment	2521

A.9.1.39.2	Test parameters	2521

A.9.1.39.3	Test Requirements	2525

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2525

A.9.1.40.1	Test Purpose and Environment	2525

A.9.1.40.2	Test parameters	2525

A.9.1.40.3	Test Requirements	2530

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2530

A.9.1.41.1	Test Purpose and Environment	2530

A.9.1.41.2	Test parameters	2530

A.9.1.41.3	Test Requirements	2532

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2532

A.9.1.42.1	Test Purpose and Environment	2532

A.9.1.42.2	Test parameters	2532

A.9.1.42.3	Test Requirements	2534

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2534

A.9.1.43.1	Test Purpose and Environment	2534

A.9.1.43.2	Test parameters	2534

A.9.1.43.3	Test Requirements	2535

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2536

A.9.1.44.1	Test Purpose and Environment	2536

A.9.1.44.2	Test parameters	2536

A.9.1.44.3	Test Requirements	2540

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2540

A.9.1.45.1	Test Purpose and Environment	2540

A.9.1.45.2	Test parameters	2540

A.9.1.45.3	Test Requirements	2544

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2544

A.9.1.46.1	Test Purpose and Environment	2544

A.9.1.46.2	Test parameters	2544

A.9.1.46.3	Test Requirements	2550

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2550

A.9.1.47.1	Test Purpose and Environment	2550

A.9.1.47.2	Test parameters	2551

A.9.1.47.3	Test Requirements	2557

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2557

A.9.1.48.1	Test Purpose and Environment	2557

A.9.1.48.2	Test parameters	2558

A.9.1.48.3	Test Requirements	2565

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2566

A.9.1.49.1	Test Purpose and Environment	2566

A.9.1.49.2	Test parameters	2566

A.9.1.49.3	Test Requirements	2573

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2574

A.9.1.50.1	Test Purpose and Environment	2574

A.9.1.50.2	Test parameters	2574

A.9.1.50.3	Test Requirements	2579

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2579

A.9.1.51.1	Test Purpose and Environment	2579

A.9.1.51.2	Test parameters	2579

A.9.1.51.3	Test Requirements	2583

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2584

A.9.1.52.1	Test Purpose and Environment	2584

A.9.1.52.2	Test parameters	2584

A.9.1.52.3	Test Requirements	2586

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2586

A.9.1.52A.1	Test Purpose and Environment	2586

A.9.1.52A.2	Test parameters	2586

A.9.1.52A.3	Test Requirements	2587

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2587

A.9.1.53.1	Test Purpose and Environment	2587

A.9.1.53.2	Test parameters	2588

A.9.1.53.3	Test Requirements	2590

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2590

A.9.1.53A.1	Test Purpose and Environment	2590

A.9.1.53A.2	Test parameters	2590

A.9.1.53A.3	Test Requirements	2591

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2591

A.9.1.54.1	Test Purpose and Environment	2591

A.9.1.54.2	Test parameters	2592

A.9.1.54.3	Test Requirements	2594

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2594

A.9.1.55.1	Test Purpose and Environment	2594

A.9.1.55.2	Test parameters	2594

A.9.1.55.3	Test Requirements	2597

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2597

A.9.1.56.1	Test Purpose and Environment	2597

A.9.1.56.2	Test parameters	2597

A.9.1.56.3	Test Requirements	2599

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2600

A.9.1.57.1	Test Purpose and Environment	2600

A.9.1.57.2	Test parameters	2600

A.9.1.57.3	Test Requirements	2602

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2602

A.9.1.57A.1	Test Purpose and Environment	2602

A.9.1.57A.2	Test parameters	2602

A.9.1.57A.3	Test Requirements	2603

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2603

A.9.1.58.1	Test Purpose and Environment	2603

A.9.1.58.2	Test parameters	2604

A.9.1.58.3	Test Requirements	2606

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2606

A.9.1.58A.1	Test Purpose and Environment	2606

A.9.1.58A.2	Test parameters	2606

A.9.1.58A.3	Test Requirements	2607

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2607

A.9.1.59.1	Test Purpose and Environment	2607

A.9.1.59.2	Test parameters	2608

A.9.1.59.3	Test Requirements	2610

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2610

A.9.1.60.1	Test Purpose and Environment	2610

A.9.1.60.2	Test parameters	2610

A.9.1.60.3	Test Requirements	2612

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2613

A.9.1.61.1	Test Purpose and Environment	2613

A.9.1.61.2	Test parameters	2613

A.9.1.61.3	Test Requirements	2615

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2615

A.9.1.62.1	Test Purpose and Environment	2615

A.9.1.62.2	Test parameters	2615

A.9.1.62.3	Test Requirements	2617

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2617

A.9.1.63.1	Test Purpose and Environment	2617

A.9.1.63.2	Test parameters	2617

A.9.1.63.3	Test Requirements	2619

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2619

A.9.1.64.1	Test Purpose and Environment	2619

A.9.1.64.2	Test parameters	2619

A.9.1.64.3	Test Requirements	2620

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2621

A.9.1.65.1	Test Purpose and Environment	2621

A.9.1.65.2	Test parameters	2621

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2625

A.9.1.66.1	Test Purpose and Environment	2625

A.9.1.66.2	Test parameters	2625

A.9.1.66.3	Test Requirements	2629

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2629

A.9.1.67.1	Test Purpose and Environment	2629

A.9.1.67.2	Test parameters	2629

A.9.1.67.3	Test Requirements	2630

A.9.2	RSRQ	2631

A.9.2.1	FDD Intra frequency case	2631

A.9.2.1.1	Test Purpose and Environment	2631

A.9.2.1.2	Test parameters	2631

A.9.2.1.3	Test Requirements	2633

A.9.2.2	TDD Intra frequency case	2633

A.9.2.2.1	Test Purpose and Environment	2633

A.9.2.2.2	Test parameters	2633

A.9.2.2.3	Test Requirements	2635

A.9.2.3	FDD—FDD Inter frequency case	2635

A.9.2.3.1	Test Purpose and Environment	2635

A.9.2.3.2	Test parameters	2635

A.9.2.3.3	Test Requirements	2637

A.9.2.4	TDD—TDD Inter frequency case	2637

A.9.2.4.1	Test Purpose and Environment	2637

A.9.2.4.2	Test parameters	2637

A.9.2.4.3	Test Requirements	2639

A.9.2.4A	FDD—TDD Inter frequency case	2640

A.9.2.4A.1	Test Purpose and Environment	2640

A.9.2.4A.2	Test parameters	2640

A.9.2.4A.3	Test Requirements	2642

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2642

A.9.2.5.1	Test Purpose and Environment	2642

A.9.2.5.2	Test parameters	2643

A.9.2.5.3	Test Requirements	2645

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2645

A.9.2.6.1	Test Purpose and Environment	2645

A.9.2.6.2	Test parameters	2645

A.9.2.6.3	Test Requirements	2647

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2647

A.9.2.7.1	Test Purpose and Environment	2647

A.9.2.7.2	Test parameters	2647

A.9.2.7.3	Test Requirements	2650

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2650

A.9.2.8.1	Test Purpose and Environment	2650

A.9.2.8.2	Test parameters	2650

A.9.2.8.3	Test Requirements	2653

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2653

A.9.2.9.1	Test Purpose and Environment	2653

A.9.2.9.2	Test parameters	2653

A.9.2.9.3	Test Requirements	2656

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2656

A.9.2.10.1	Test Purpose and Environment	2656

A.9.2.10.2	Test parameters	2656

A.9.2.10.3	Test Requirements	2659

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2659

A.9.2.11.1	Test Purpose and Environment	2659

A.9.2.11.2	Test parameters	2659

A.9.2.11.3	Test Requirements	2660

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2660

A.9.2.12.1	Test Purpose and Environment	2660

A.9.2.12.2	Test parameters	2660

A.9.2.12.3	Test Requirements	2661

A.9.2.13	Void	2662

A.9.2.13.1	Void	2662

A.9.2.13.2	Void	2662

A.9.2.13.3	Void	2662

A.9.2.14	Void	2662

A.9.2.14.1	Void	2662

A.9.2.14.2	Void	2662

A.9.2.14.3	Void	2662

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2662

A.9.2.15.1	Test Purpose and Environment	2662

A.9.2.15.2	Test parameters	2662

A.9.2.15.3	Test Requirements	2665

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2665

A.9.2.16.1	Test Purpose and Environment	2665

A.9.2.16.2	Test parameters	2665

A.9.2.16.3	Test Requirements	2668

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2668

A.9.2.17.1	Test Purpose and Environment	2668

A.9.2.17.2	Test parameters	2668

A.9.2.17.3	Test Requirements	2670

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2670

A.9.2.18.1	Test Purpose and Environment	2670

A.9.2.18.2	Test parameters	2670

A.9.2.18.3	Test Requirements	2672

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2672

A.9.2.19.1	Test Purpose and Environment	2672

A.9.2.19.2	Test parameters	2672

A.9.2.19.3	Test Requirements	2674

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2674

A.9.2.20.1	Test Purpose and Environment	2674

A.9.2.20.2	Test parameters	2674

A.9.2.20.3	Test Requirements	2676

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2676

A.9.2.21.1	Test Purpose and Environment	2676

A.9.2.21.2	Test parameters	2676

A.9.2.21.3	Test Requirements	2678

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2678

A.9.2.22.1	Test Purpose and Environment	2678

A.9.2.22.2	Test parameters	2678

A.9.2.22.3	Test Requirements	2679

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2679

A.9.2.23.1	Test Purpose and Environment	2679

A.9.2.23.2	Test parameters	2679

A.9.2.23.3	Test Requirements	2681

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2681

A.9.2.24.1	Test Purpose and Environment	2681

A.9.2.24.2	Test parameters	2681

A.9.2.24.3	Test Requirements	2681

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2682

A.9.2.25.1	Test Purpose and Environment	2682

A.9.2.25.2	Test parameters	2682

A.9.2.25.3	Test Requirements	2685

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2685

A.9.2.26.1	Test Purpose and Environment	2685

A.9.2.26.2	Test parameters	2685

A.9.2.26.3	Test Requirements	2688

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2688

A.9.2.27.1	Test Purpose and Environment	2688

A.9.2.27.2	Test parameters	2688

A.9.2.27.3	Test Requirements	2689

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2689

A.9.2.28.1	Test Purpose and Environment	2689

A.9.2.28.2	Test parameters	2689

A.9.2.28.3	Test Requirements	2691

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2691

A.9.2.29.1	Test Purpose and Environment	2691

A.9.2.29.2	Test parameters	2691

A.9.2.29.3	Test Requirements	2693

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2693

A.9.2.30.1	Test Purpose and Environment	2693

A.9.2.30.2	Test parameters	2693

A.9.2.30.3	Test Requirements	2695

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2695

A.9.2.31.1	Test Purpose and Environment	2695

A.9.2.31.2	Test parameters	2695

A.9.2.31.3	Test Requirements	2697

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2697

A.9.2.32.1	Test Purpose and Environment	2697

A.9.2.32.2	Test parameters	2697

A.9.2.32.3	Test Requirements	2699

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2699

A.9.2.33.1	Test Purpose and Environment	2699

A.9.2.33.2	Test parameters	2699

A.9.2.33.3	Test Requirements	2701

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2701

A.9.2.34.1	Test Purpose and Environment	2701

A.9.2.34.2	Test parameters	2701

A.9.2.34.3	Test Requirements	2703

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2703

A.9.2.35.1	Test Purpose and Environment	2703

A.9.2.35.2	Test parameters	2703

A.9.2.35.3	Test Requirements	2705

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2705

A.9.2.36.1	Test Purpose and Environment	2705

A.9.2.36.2	Test parameters	2705

A.9.2.36.3	Test Requirements	2706

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2707

A.9.2.37.1	Test Purpose and Environment	2707

A.9.2.37.2	Test parameters	2707

A.9.2.37.3	Test Requirements	2708

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2709

A.9.2.38.1	Test Purpose and Environment	2709

A.9.2.38.2	Test parameters	2709

A.9.2.38.3	Test Requirements	2712

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2712

A.9.2.39.1	Test Purpose and Environment	2712

A.9.2.39.2	Test parameters	2712

A.9.2.39.3	Test Requirements	2715

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2715

A.9.2.40.1	Test Purpose and Environment	2715

A.9.2.40.2	Test parameters	2715

A.9.2.40.3	Test Requirements	2717

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2717

A.9.2.41.1	Test Purpose and Environment	2717

A.9.2.41.2	Test parameters	2718

A.9.2.41.3	Test Requirements	2720

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2720

A.9.2.42.1	Test Purpose and Environment	2720

A.9.2.42.2	Test parameters	2720

A.9.2.42.3	Test Requirements	2722

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2722

A.9.2.43.1	Test Purpose and Environment	2722

A.9.2.43.2	Test parameters	2722

A.9.2.43.3	Test Requirements	2724

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2724

A.9.2.44.1	Test Purpose and Environment	2724

A.9.2.44.2	Test parameters	2724

A.9.2.44.3	Test Requirements	2726

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2726

A.9.2.45.1	Test Purpose and Environment	2726

A.9.2.45.2	Test parameters	2726

A.9.2.45.3	Test Requirements	2729

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2729

A.9.2.46.1	Test Purpose and Environment	2730

A.9.2.46.2	Test parameters	2730

A.9.2.46.3	Test Requirements	2733

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2733

A.9.2.47.1	Test Purpose and Environment	2734

A.9.2.47.2	Test parameters	2734

A.9.2.47.3	Test Requirements	2737

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2737

A.9.2.48.1	Test Purpose and Environment	2737

A.9.2.48.2	Test parameters	2737

A.9.2.48.3	Test Requirements	2740

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2740

A.9.2.49.1	Test Purpose and Environment	2740

A.9.2.49.2	Test parameters	2740

A.9.2.49.3	Test Requirements	2744

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2745

A.9.2.50.1	Test Purpose and Environment	2745

A.9.2.50.2	Test parameters	2745

A.9.2.50.3	Test Requirements	2749

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2749

A.9.2.51.1	Test Purpose and Environment	2749

A.9.2.51.2	Test parameters	2750

A.9.2.51.3	Test Requirements	2753

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2753

A.9.2.52.1	Test Purpose and Environment	2753

A.9.2.52.2	Test parameters	2753

A.9.2.52.3	Test Requirements	2755

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2756

A.9.2.53.1	Test Purpose and Environment	2756

A.9.2.53.2	Test parameters	2756

A.9.2.53.3	Test Requirements	2760

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2761

A.9.2.54.1	Test Purpose and Environment	2761

A.9.2.54.2	Test parameters	2761

A.9.2.54.3	Test Requirements	2765

A.9.3	UTRAN FDD CPICH RSCP	2765

A.9.3.1	E-UTRAN FDD	2765

A.9.3.1.1	Test Purpose and Environment	2765

A.9.3.1.2	Parameters	2765

A.9.3.1.3	Test Requirements	2767

A.9.3.2 E-UTRAN TDD	2767

A.9.3.2.1	Test Purpose and Environment	2767

A.9.3.2.2	Parameters	2767

A.9.3.2.3	Test Requirements	2770

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2770

A.9.3.3.1	Test Purpose and Environment	2770

A.9.3.3.2	Parameters	2770

A.9.3.3.3	Test Requirements	2771

A.9.4	UTRAN FDD CPICH Ec/No	2772

A.9.4.1	E-UTRAN FDD	2772

A.9.4.1.1	Test Purpose and Environment	2772

A.9.4.1.2	Parameters	2772

A.9.4.1.3	Test Requirements	2774

A.9.4.2	E-UTRAN TDD	2775

A.9.4.2.1	Test Purpose and Environment	2775

A.9.4.2.2	Parameters	2775

A.9.4.2.3	Test Requirements	2777

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2778

A.9.4.3.1	Test Purpose and Environment	2778

A.9.4.3.2	Parameters	2778

A.9.4.3.3	Test Requirements	2779

A.9.5	UTRAN TDD measurement	2779

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2779

A.9.5.1.1	Test Purpose and Environment	2779

A.9.5.1.2	Test parameters	2779

A.9.5.1.3	Test Requirements	2781

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2781

A.9.5.2.1	Test Purpose and Environment	2781

A.9.5.2.2	Test parameters	2781

A.9.5.2.3	Test Requirements	2783

A.9.6	GSM Carrier RSSI	2784

A.9.6.1	E-UTRAN FDD	2784

A.9.6.1.1	Test Purpose and Environment	2784

A.9.6.1.2	Test Requirements	2785

A.9.6.2	E-UTRAN TDD	2785

A.9.6.2.1	Test Purpose and Environment	2785

A.9.6.2.2	Test Requirements	2788

A.9.7	UE Rx – Tx Time Difference	2788

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2788

A.9.7.1.1	Test Purpose and Environment	2788

A.9.7.1.2	Test parameters	2788

A.9.7.1.3	Test Requirements	2790

A.9.7.2	E-UTRA TDD	2790

A.9.7.2.1	Test Purpose and Environment	2790

A.9.7.2.2	Test parameters	2790

A.9.7.2.3	Test Requirements	2792

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2792

A.9.7.3.1	Test Purpose and Environment	2792

A.9.7.3.2	Test parameters	2792

A.9.7.3.3	Test Requirements	2795

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2795

A.9.7.4.1	Test Purpose and Environment	2795

A.9.7.4.2	Test Parameters	2795

A.9.7.4.3	Test Requirements	2798

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2798

A.9.7.5.1	Test Purpose and Environment	2798

A.9.7.5.2	Test parameters	2798

A.9.7.5.3	Test Requirements	2801

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2801

A.9.7.6.1	Test Purpose and Environment	2801

A.9.7.6.2	Test Parameters	2801

A.9.7.6.3	Test Requirements	2804

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2804

A.9.7.7.1	Test Purpose and Environment	2804

A.9.7.7.2	Test parameters	2804

A.9.7.7.3	Test Requirements	2805

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2806

A.9.7.8.1	Test Purpose and Environment	2806

A.9.7.8.2	Test parameters	2806

A.9.7.8.3	Test Requirements	2807

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2807

A.9.7.9.1	Test Purpose and Environment	2807

A.9.7.9.2	Test parameters	2807

A.9.7.9.3	Test Requirements	2808

A.9.8	RSTD	2809

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2809

A.9.8.1.1	Test Purpose and Environment	2809

A.9.8.1.2	Test Requirements	2811

A.9.8.1.2A	Test Requirements for UE Category 1bis	2811

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2812

A.9.8.2.1	Test Purpose and Environment	2812

A.9.8.2.2	Test Requirements	2814

A.9.8.2.2A	Test Requirements for UE Category 1bis	2814

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2815

A.9.8.3.1	Test Purpose and Environment	2815

A.9.8.3.2	Test Requirements	2817

A.9.8.3.2A	Test Requirements for UE Category 1bis	2817

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2818

A.9.8.4.1	Test Purpose and Environment	2818

A.9.8.4.2	Test Requirements	2820

A.9.8.4.2A	Test Requirements for UE Category 1bis	2820

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2821

A.9.8.5.1	Test Purpose and Environment	2821

A.9.8.5.2	Test Requirements	2823

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2823

A.9.8.6.1	Test Purpose and Environment	2823

A.9.8.6.2	Test Requirements	2826

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2826

A.9.8.7.1	Test Purpose and Environment	2826

A.9.8.7.2	Test Requirements	2827

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2827

A.9.8.8.1	Test Purpose and Environment	2827

A.9.8.8.2	Test Requirements	2828

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2828

A.9.8.9.1	Test Purpose and Environment	2828

A.9.8.9.2	Test Requirements	2829

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2829

A.9.8.10.1	Test Purpose and Environment	2829

A.9.8.10.2	Test Requirements	2829

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2829

A.9.8.11.1	Test Purpose and Environment	2829

A.9.8.11.2	Test Requirements	2830

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2830

A.9.8.12.1	Test Purpose and Environment	2830

A.9.8.12.2	Test Requirements	2831

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2831

A.9.8.13.1	Test Purpose and Environment	2831

A.9.8.13.2	Test Requirements	2832

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2832

A.9.8.14.1	Test Purpose and Environment	2832

A.9.8.14.2	Test Requirements	2837

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2837

A.9.8.15.1	Test Purpose and Environment	2837

A.9.8.15.2	Test Requirements	2841

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2841

A.9.8.16.1	Test Purpose and Environment	2841

A.9.8.16.2	Test Requirements	2844

A.9.8.17 HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2844

A.9.8.17.1	Test Purpose and Environment	2844

A.9.8.17.2	Test Requirements	2847

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2847

A.9.8.18.1	Test Purpose and Environment	2847

A.9.8.18.2	Test Requirements	2850

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2850

A.9.8.19.1	Test Purpose and Environment	2850

A.9.8.19.2	Test Requirements	2853

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2853

A.9.8.20.1	Test Purpose and Environment	2853

A.9.8.20.2	Test Requirements	2855

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2855

A.9.8.21.1	Test Purpose and Environment	2855

A.9.8.21.2	Test Requirements	2858

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2858

A.9.8.22.1	Test Purpose and Environment	2858

A.9.8.22.2	Test Requirements	2861

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2862

A.9.8.23.1	Test Purpose and Environment	2862

A.9.8.23.2	Test Requirements	2864

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2864

A.9.8.24.1	Test Purpose and Environment	2864

A.9.8.24.2	Test Requirements	2867

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2867

A.9.8.25.1	Test Purpose and Environment	2867

A.9.8.25.2	Test Requirements	2870

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2871

A.9.8.26.1	Test Purpose and Environment	2871

A.9.8.26.2	Test Requirements	2873

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2873

A.9.8.27.1	Test Purpose and Environment	2873

A.9.8.27.2	Test Requirements	2876

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2876

A.9.8.28.1	Test Purpose and Environment	2876

A.9.8.28.2	Test Requirements	2879

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2879

A.9.8.29.1	Test Purpose and Environment	2879

A.9.8.29.2	Test Requirements	2882

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2883

A.9.8.30.1	Test Purpose and Environment	2883

A.9.8.30.2	Test Requirements	2885

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2886

A.9.8.31.1	Test Purpose and Environment	2886

A.9.8.31.2	Test Requirements	2889

A.9.9	RSRP and RSRQ on the serving cell	2889

A.9.9.1	FDD Intra frequency serving cell case	2889

A.9.9.1.1	Test Purpose and Environment	2889

A.9.9.1.2	Test parameters	2889

A.9.9.1.3	Test Requirements	2891

A.9.9.2	TDD Intra frequency serving cell case	2891

A.9.9.2.1	Test Purpose and Environment	2891

A.9.9.2.2	Test parameters	2891

A.9.9.2.3	Test Requirements	2893

A.9.10	SSTD	2893

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2893

A.9.10.1.1	Test Purpose and Environment	2893

A.9.10.1.2	Test parameters	2893

A.9.10.1.3	Test Requirements	2895

A.9.10.3	FDD—FDD Inter frequency case	2895

A.9.10.3.1	Test Purpose and Environment	2895

A.9.10.3.2	Test parameters	2895

A.9.10.3.3	Test Requirements	2899

A.9.10.4	TDD—TDD Inter frequency case	2899

A.9.10.4.1	Test Purpose and Environment	2899

A.9.10.4.2	Test parameters	2899

A.9.10.4.3	Test Requirements	2904

A.9.11 RSSI	2904

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2904

A.9.11.1.1	Test Purpose and Environment	2904

A.9.11.1.2	Test parameters	2904

A.9.11.1.3	Test Requirements	2906

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2906

A.9.11.2.1	Test Purpose and Environment	2906

A.9.11.2.2	Test parameters	2906

A.9.12	Channel occupancy	2908

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2908

A.9.12.1.1	Test Purpose and Environment	2908

A.9.12.1.2	Test parameters	2908

A.9.12.1.3	Test Requirements	2910

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2910

A.9.12.2.1	Test Purpose and Environment	2910

A.9.12.2.2	Test parameters	2910

A.9.12.2.3	Test Requirements	2912

A.9.13	RS-SINR	2912

A.9.13.1	FDD Intra-Frequency Case	2912

A.9.13.1.1	Test Purpose and Environment	2912

A.9.13.1.2	Test parameters	2912

A.9.13.1.3	Test Requirements	2914

A.9.13.2	TDD Intra-Frequency Case	2914

A.9.13.2.1	Test Purpose and Environment	2914

A.9.13.2.2	Test parameters	2914

A.9.13.2.3	Test Requirements	2917

A.9.13.5	FDD—TDD Inter frequency case	2917

A.9.13.5.1	Test Purpose and Environment	2917

A.9.13.5.2	Test parameters	2917

A.9.13.5.3	Test Requirements	2921

A.9.13.6	TDD—FDD Inter frequency case	2921

A.9.13.6.1	Test Purpose and Environment	2921

A.9.13.6.2	Test parameters	2921

A.9.13.6.3	Test Requirements	2924

A.9.14	Channel quality reporting accuracy	2924

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2924

A.9.14.1.1	Test Purpose and Environment	2924

A.9.14.1.2	Test parameters	2924

A.9.14.1.3	Test Requirements	2925

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2925

A.9.14.2.1	Test Purpose and Environment	2925

A.9.14.2.2	Test parameters	2926

A.9.14.2.3	Test Requirements	2926

A.10	Proximity-based Services in Any Cell Selection State	2927

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2927

A.10.1.1	Test Purpose and Environment	2927

A.10.1.2	Test Requirements	2928

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2928

A.10.2.1	Test Purpose and Environment	2928

A.10.2.2	Test Requirements	2929

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2930

A.10.3.1	Test Purpose and Environment	2930

A.10.3.2	Test Requirements	2931

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2932

A.10.4.1	Test Purpose and Environment	2932

A.10.4.2	Test Requirements	2934

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2934

A.11.1	V2V UE Transmission Timing Accuracy Test	2934

A.11.1.1	Test Purpose and Environment	2934

A.11.1.2	Test requirements	2935

A.11.2	Interruptions due to V2V sidelink communication	2935

A.11.2.1	Test Purpose and Environment	2935

A.11.2.2	Test Requirements	2937

A.12	2937

A.12.1	V2X UE Transmission Timing Accuracy Test	2937

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2937

A.12.1.1.1	Test Purpose and Environment	2937

A.12.1.1.2	Test requirements	2938

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	2939

A.12.1.2.1	Test Purpose and Environment	2939

A.12.1.2.2	Test Requirements	2939

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2940

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	2940

A.12.2.1.1	Test Purpose and Environment	2940

A.12.2.1.2	Test Requirements	2941

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	2942

A.12.2.2.1	Test Purpose and Environment	2942

A.12.2.2.2	Test Requirements	2943

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	2943

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	2943

A.12.3.1.1	Test Purpose and Environment	2943

A.12.3.1.2	Test Requirements	2945

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	2946

A.12.3.2.1	Test Purpose and Environment	2946

A.12.3.1.2	Test Requirements	2948

A.12.4	Congestion Control Measurement Test for V2X UE	2948

A.12.4.1	Test Purpose and Environment	2948

A.12.4.2	Test Requirements	2950

A.12.5	Interruptions due to V2X Sidelink Communication	2950

A.12.5.1	Test Purpose and Environment	2950

A.12.5.1	Test Requirements	2952

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	2952

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	2952

A.12.6.1.1	Test Purpose and Environment	2952

A.12.6.1.2	Test Requirements	2954

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	2954

A.12.6.2.1	Test Purpose and Environment	2954

A.12.6.1.2	Test Requirements	2956

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	2957

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	2957

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	2957

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	2957

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	2957

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	2959

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	2959

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	2960

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	2960

B.1.7.1 Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	2960

B.1.7.2 Condition for measurements of inter-frequence E-UTRAN cells for cell selection	2961

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	2963

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	2963

B.2.1	Conditions for E-UTRAN intra-frequency measurements	2963

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	2964

B.2.3	Conditions for E-UTRAN inter-frequency measurements	2964

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	2964

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	2965

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	2965

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	2965

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	2966

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2966

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	2967

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	2967

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	2967

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	2968

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2968

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2968

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	2969

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	2969

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2969

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2969

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	2970

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	2971

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	2971

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	2972

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	2972

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	2974

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	2974

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	2974

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	2974

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	2975

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	2975

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	2977

B.3	Conditions for measurements performance requirements for UE	2977

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	2977

B.3.2	Void	2977

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	2977

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	2977

B.3.5	Conditions for UE Rx – Tx time difference	2977

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	2978

B.3.7	Conditions for inter-frequency RSTD measurements	2978

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	2978

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	2978

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	2978

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2979

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2979

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	2979

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2979

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	2979

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	2979

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2980

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	2980

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	2980

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2981

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	2981

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	2981

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2981

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	2981

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	2981

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	2981

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	2981

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	2981

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2982

B.3.21.1	Conditions for RSRP measurements	2982

B.3.21.2	Conditions for RSRQ measurements	2982

B.3.21.3	Conditions for CSI-RSRP measurements	2982

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2982

B.3.22.1	Conditions for RSRP measurements	2982

B.3.22.2	Void	2983

B.3.22.3	Conditions for CSI-RSRP measurements	2983

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2983

B.3.23.1	Conditions for RSRP measurements	2983

B.3.23.2	Conditions for RSRQ measurements	2983

B.3.23.3	Conditions for CSI-RSRP measurements	2983

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2984

B.3.24.1	Conditions for RSRP measurements	2984

B.3.24.2	Conditions for RSRQ measurements	2984

B.3.24.3	Conditions for CSI-RSRP measurements	2984

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2984

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2984

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	2984

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	2985

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	2985

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	2985

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	2985

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	2985

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	2986

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	2986

B.4	RRM Requirements Exceptions	2986

B.4.1	General	2986

B.4.2	Receiver sensitivity relaxation for UE supporting CA	2986

B.4.3	Receiver sensitivity relaxation for UE configured with CA	2986

B.4.3.1	Inter-band carrier aggregation	2986

B.4.3.2	Intra-band non-contiguous carrier aggregation	2986

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	2987

B.5	Conditions for Measurement Performance Requirements for ProSe UE	2987

B.5.1	Conditions for S-RSRP Accuracy Requirements	2987

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	2987

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2988

B.5.4	Conditions for SD-RSRP Accuracy Requirements	2988

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	2989

B.6	Conditions for V2X	2989

B.6.1	Test parameters for GNSS signals	2989

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	2989

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	2989

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2990

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	2990

Annex C (informative):	Change history:	2991