3GPP TR 37.843 V15.4.0 (2019-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA);

Radio Frequency (RF) requirement background for

Active Antenna System (AAS) Base Station (BS) radiated requirements

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	16

1	Scope	17

2	References	17

3	Definitions, symbols and abbreviations	20

3.1	Definitions	20

3.2	Symbols	22

3.3	Abbreviations	23

4	General	24

4.1	Work Item scope	24

4.2	Work Item objectives	24

4.3	General AAS BS RF architecture	24

4.4	Specification organization	26

4.4.1	TS 37.105: RF and demodulation requirements	26

4.4.2	TS 37.114: EMC requirements	26

4.5	Coordinate system	26

4.6	AAS BS classifications	27

4.6.1	Minimum coupling loss and minimum distance	27

4.6.2	BS classifications	27

4.7	Polarization	27

5	Radiated transmitter characteristics	28

5.1	General	28

5.1.1	Spatial requirements	28

5.1.2	Co-location requirements	32

5.1.2.1	Core requirements	32

5.1.2.2	Conformance requirements	34

5.1.2.2.1	General	34

5.1.2.2.2	Co-location test antenna	35

5.1.2.2.3	Standard test antenna	36

5.1.3	Transmit loss factor assumption	36

5.2	Base station output power	37

5.2.1	Background information on the conducted requirement	37

5.2.2	OTA requirement	37

5.2.3	Output power accuracy	37

5.2.3.1	Core requirement	37

5.2.3.2	Conformance requirement	38

5.2.3.3	Extreme conditions	38

5.2.4	BS class power limits	39

5.2.5	Control channel accuracy	40

5.2.5.1	Background	40

5.2.5.2	Core requirement	41

5.2.5.3	Conformance requirement	42

5.3	Output power dynamics	42

5.3.1	Background information on the conducted requirement	42

5.3.2	Output power dynamics OTA: Core requirement	43

5.3.3	Output power dynamics OTA: Conformance requirement	43

5.4	Transmit ON/OFF power	43

5.4.1	Background	43

5.4.2	OTA AAS BS – Core requirement	44

5.5	Transmitted signal quality	44

5.5.1	General	44

5.5.2	Frequency error	44

5.5.2.1	Background information on the conducted requirement	44

5.5.2.2	Frequency error OTA: Core requirement	44

5.5.2.3	Frequency error OTA: Conformance requirement	45

5.5.3	Time Alignment Error	45

5.5.4	Modulation Quality	46

5.5.4.1	Background information on the conducted requirement	46

5.5.4.2	Modulation Quality OTA: Core requirement	47

5.5.4.3	Modulation Quality OTA: Conformance requirement	48

5.5.5	Transmit pulse shape filter	48

5.5.5.1	Background information on the conducted requirement	48

5.5.5.2	Transmit pulse shape filter OTA: Core requirement	48

5.5.5.3	Transmit pulse shape filter OTA: Conformance requirement	48

5.6	Unwanted emissions	49

5.6.1	General	49

5.6.1.1	Background information on the conducted requirement	49

5.6.1.2	OTA unwanted emissions	49

5.6.2	Occupied bandwidth	50

5.6.2.1	Background information on the conducted requirement	50

5.6.2.2	Occupied bandwidth OTA: Core requirement	50

5.6.2.3	Occupied bandwidth OTA: Conformance requirement	50

5.6.3	Adjacent Channel Leakage Radio	50

5.6.3.1	Background information on the conducted requirement	50

5.6.3.2	ACLR OTA: Core requirement	51

5.6.3.3	ACLR OTA: Conformance requirement	52

5.6.4	Spectrum emission mask	52

5.6.4.1	Background information on the conducted requirement	52

5.6.4.2	Spectrum emission mask OTA: Core requirement	52

5.6.4.3	Spectrum emission mask OTA: Conformance requirement	52

5.6.5	Operating band unwanted emission	52

5.6.5.1	Background information on the conducted requirement	52

5.6.5.2	Operating band unwanted emission OTA: Core requirement	53

5.6.5.3	Operating band unwanted emission OTA: Conformance requirement	53

5.6.6	Transmitter spurious emissions	53

5.6.6.1	Mandatory Spurious emission requirements	53

5.6.6.1.1	Background information on the conducted requirement	53

5.6.6.1.2	Transmitter spurious emissions OTA: Core requirement	54

5.6.6.1.3	Transmitter spurious emissions OTA: Conformance requirement	54

5.6.6.2	Co-location requirements	55

5.6.6.2.1	Background	55

5.6.6.2.2	Core requirements	55

5.6.6.2.3	Conformance requirements	56

5.7	Transmitter intermodulation	57

5.7.1	General	57

5.7.2	Background information on the conducted requirement	57

6	Radiated receiver characteristics	58

6.1	General	58

6.1.1	Spatial requirements	58

6.1.2	Receive loss factor assumption	59

6.1.3	OTA offsets	59

6.2	Reference sensitivity level	59

6.2.1	Background information on the conducted requirement	59

6.2.2	OTA sensitivity requirements	60

6.2.2.1	General	60

6.2.2.2	Core requirement	60

6.2.2.2.1	OTA Sensitivity	60

6.2.2.2.2	OTA reference sensitivity	60

6.2.2.3	Conformance requirement	62

6.2.2.3.1	OTA Sensitivity	62

6.2.2.3.2	OTA reference sensitivity	62

6.2.2.4	Extreme conditions	62

6.3	Dynamic range	63

6.3.1	Background information on the conducted requirement	63

6.3.2	Core requirement: Receiver dynamic range	63

6.3.3	Conformance requirement: Receiver dynamic range	63

6.4	In-band selectivity and blocking	63

6.4.1	Background information on the conducted requirement	63

6.4.2	In-band selectivity and blocking: Core requirement	64

6.4.3	In-band selectivity and blocking: Conformance requirement	68

6.5	Out-of-band blocking	68

6.5.1	Background	68

6.5.2	Out-of-band blocking OTA core requirement	68

6.5.2.1	Interferer level	68

6.5.2.2	Wanted signal level and direction	70

6.5.2.3	Out of band region	70

6.5.2.4	Frequency range	71

6.5.3	Out-of-band blocking OTA conformance requirement	71

6.6	Receiver spurious emissions	71

6.6.1	Background information on the conducted requirement	71

6.6.2	Receiver spurious emissions OTA: Core requirement	71

6.6.3	Receiver spurious emissions OTA: Conformance requirement	72

6.7	Receiver intermodulation	72

6.7.1	Background information on the conducted requirement	72

6.7.2	Receiver intermodulation: Core requirement	72

6.7.3	Receiver intermodulation: Conformance requirement	73

6.8	In-channel selectivity	73

6.8.1	Background information on the conducted requirement	73

6.8.2	In-channel selectivity: Core requirement	73

6.8.3	In-channel selectivity: Conformance requirement	73

6.9	ACS and NB blocking	74

6.9.1	Background information on the conducted requirement	74

6.9.2	ACS and NB blocking: Core requirement	74

6.9.3	ACS and NB blocking: Conformance requirement	75

7	Performance requirements	75

7.1	General	75

7.2	Core requirements for OTA tests	75

7.3	UTRA FDD requirements	75

7.4	E-UTRA requirements	76

7.5	Propagation conditions	78

7.6	Conformance requirements	78

7.7	OTA test setup	78

7.8	BS demodulation requirements feasible OTA	80

8	EMC requirements	81

8.1	General	81

8.2	Regulatory EMC requirements	82

8.3	Field strength in EMC chamber	83

8.3.1	General	83

8.3.2	Conversion between dBm to V/m	84

8.4	Protection of measurement equipment	84

8.5	EMC rated emission requirement	85

8.6	Radiated Immunity	85

8.6.1	General	85

8.6.2	Measurement set-up for testing radiated immunity	86

8.6.3	Spatial exclusions for radiated immunity testing	86

9	Manufacturer declarations	87

9.1	General	87

9.2	Void	88

10	Conformance testing aspects	88

10.1	OTA conformance testing framework	88

10.1.1	General	88

10.1.2	Uncertainty budget calculation principle	88

10.2	Measurement uncertainty for TX directional requirements	88

10.2.1	General	88

10.2.2	EIRP accuracy	89

10.2.2.1	General	89

10.2.2.2	CATR	89

10.2.2.2.1	MU assessment	89

10.2.2.2.1.1	MU budget	89

10.2.2.2.1.2	MU value	89

10.2.3	OTA E-UTRA DL RS power	91

10.2.3.1	General	91

10.2.3.2	In-door anechoic chamber	91

10.2.3.2.1	General	91

10.2.3.2.2	Calibration	93

10.2.3.2.3	Procedure	94

10.2.3.2.4	MU assessment	94

10.2.3.2.4.1	MU Budget	94

10.2.3.2.4.2	MU Value	95

10.2.3.3	CATR	96

10.2.3.3.1	General	96

10.2.3.3.2	Calibration	97

10.2.3.3.3	Procedure	98

10.2.3.3.4	MU assessment	99

10.2.3.3.4.1	MU Budget	99

10.2.3.3.4.2	MU Value	100

10.2.3.4	Near Field	100

10.2.3.4.1	General	100

10.2.3.4.2	Calibration	101

10.2.3.4.3	Procedure	102

10.2.3.4.4	MU assessment	102

10.2.3.4.4.1	MU budget	102

10.2.3.4.4.2	MU value	103

10.2.3.5	Summary	105

10.2.4	OTA Output power dynamics	105

10.2.4.1	General	105

10.2.4.2	In-door anechoic chamber	106

10.2.4.2.1	General	106

10.2.4.2.2	Calibration	106

10.2.4.2.3	Procedure	106

10.2.4.2.4	MU assessment	106

10.2.4.2.4.1	MU Budget	106

10.2.4.2.4.2	MU Value	107

10.2.4.3	CATR	108

10.2.4.3.1	General	108

10.2.4.3.2	Calibration	108

10.2.4.3.3	Procedure	108

10.2.4.3.4	MU assessment	109

10.2.4.3.4.1	MU Budget	109

10.2.4.3.4.2	MU Value	109

10.2.4.4	Near Field Test range	110

10.2.4.4.1	Description	110

10.2.4.4.2	Calibration	110

10.2.4.4.3	Procedure	110

10.2.4.4.4	MU assessment	110

10.2.4.4.4.1	MU budget	110

10.2.4.4.4.2	MU value	110

10.2.4.5	Summary	111

10.2.5	OTA Transmitted signal quality –Frequency Error	112

10.2.5.1	General	112

10.2.5.2	In-door anechoic chamber	112

10.2.5.2.1	General	112

10.2.5.2.2	Calibration	113

10.2.5.2.3	Procedure	113

10.2.5.2.4	MU assessment	113

10.2.5.3	CATR	113

10.2.5.3.1	General	113

10.2.5.3.2	Calibration	113

10.2.5.3.3	Procedure	113

10.2.5.3.4	MU assessment	113

10.2.5.4	Near Field	114

10.2.5.4.1	General	114

10.2.5.4.2	Calibration	114

10.2.5.4.3	Procedure	114

10.2.5.4.4	MU assessment	114

10.2.5.5	Summary	114

10.2.6	OTA Transmitted signal quality –EVM	114

10.2.6.1	General	114

10.2.6.1.1	Test equipment MU	115

10.2.6.2	In-door anechoic chamber	115

10.2.6.2.1	General	115

10.2.6.2.2	Calibration	115

10.2.6.2.3	Procedure	115

10.2.6.2.3.1	MU Budget	116

10.2.6.2.3.2	MU Value	116

10.2.6.3	CATR	117

10.2.6.3.1	General	117

10.2.6.3.2	Calibration	117

10.2.6.3.3	Procedure	117

10.2.6.3.4	MU assessment	117

10.2.6.3.4.1	MU Budget	117

10.2.6.3.4.2	MU Value	118

10.2.6.4	Near Field	118

10.2.6.4.1	General	118

10.2.6.4.2	Test method limitations and scope	118

10.2.6.4.3	Calibration	119

10.2.6.4.4	Procedure	119

10.2.6.4.5	MU assessment	119

10.2.6.4.5.1	MU Budget	119

10.2.6.4.5.2	MU Value	119

10.2.6.5	Summary	120

10.2.7	OTA Transmitted signal quality –TAE	120

10.2.7.1	General	120

10.2.7.2	In-door anechoic chamber	121

10.2.7.2.1	General	121

10.2.7.2.2	Calibration	121

10.2.7.2.3	Procedure	121

10.2.7.2.4	MU assessment	121

10.2.7.3	CATR	121

10.2.7.3.1	General	121

10.2.7.3.2	Calibration	121

10.2.7.3.3	Procedure	122

10.2.7.3.4	MU assessment	122

10.2.7.4	Near Field	123

10.2.7.4.1	General	123

10.2.7.4.2	Calibration	123

10.2.7.4.3	Procedure	123

10.2.7.4.4	MU assessment	123

10.2.7.5	Summary	123

10.2.8	OTA Occupied bandwidth	123

10.2.8.1	General	123

10.2.8.2	In-door anechoic chamber	124

10.2.8.2.1	General	124

10.2.8.2.2	Calibration	124

10.2.8.2.3	Procedure	124

10.2.8.2.4	MU assessment	125

10.2.8.2.4.1	MU Budget	125

10.2.8.2.4.2	MU Value	125

10.2.8.3	CATR	125

10.2.8.3.1	General	125

10.2.8.3.2	Calibration	125

10.2.8.3.3	Procedure	125

10.2.8.3.4	MU assessment	125

10.2.8.3.4.1	MU Budget	125

10.2.8.3.4.2	MU Value	125

10.2.8.4	Near field	125

10.2.8.4.1	General	125

10.2.8.4.2	Calibration	125

10.2.8.4.3	Procedure	126

10.2.8.4.4	MU assessment	126

10.2.8.4.4.1	MU Budget	126

10.2.8.4.4.2	MU Value	126

10.2.8.5	Summary	126

10.2.9	EIRP accuracy – extreme conditions	126

10.2.9.1	General	126

10.2.9.2	In-door anechoic chamber	126

10.2.9.2.1	General	126

10.2.9.2.2	Calibration	127

10.2.9.2.3	Procedure	127

10.2.9.2.4	MU assessment	128

10.2.9.2.4.1	MU Budget	128

10.2.9.2.4.2	MU Value	128

10.2.9.3	CATR	130

10.2.9.3.1	General	130

10.2.9.3.2	Calibration	130

10.2.9.3.3	Procedure	130

10.2.9.3.4	MU assessment	131

10.2.9.3.4.1	MU Budget	131

10.2.9.3.4.2	MU Value	131

10.2.9.3	Summary	132

10.3	Measurement uncertainty for Rx Directional requirements	133

10.3.1	General	133

10.3.2	OTA sensitivity	133

10.3.3	OTA reference sensitivity	134

10.3.4	OTA Dynamic range	134

10.3.4.1	General	134

10.3.4.2	In-door anechoic chamber	134

10.3.4.2.1	General	134

10.3.4.2.2	Calibration	134

10.3.4.2.3	Procedure	135

10.3.4.2.4	MU assessment	135

10.3.4.3	CATR	135

10.3.4.3.1	General	135

10.3.4.3.2	Calibration	135

10.3.4.3.3	Procedure	136

10.3.4.3.4	MU assessment	136

10.3.4.4	Summary	136

10.3.5	OTA adjacent channel selectivity, general blocking and narrowband blocking	136

10.3.5.1	General	136

10.3.5.2	In-door anechoic chamber	136

10.3.5.2.1	General	136

10.3.5.2.2	Calibration	137

10.3.5.2.3	Procedure	137

10.3.5.2.4	MU assessment	138

10.3.5.2.4.1	MU Budget	138

10.3.5.2.4.2	MU Value	138

10.3.5.3	CATR	139

10.3.5.3.1	General	139

10.3.5.3.2	Calibration	139

10.3.5.3.3	Procedure	139

10.3.5.3.4	MU assessment	140

10.3.5.3.4.1	MU Budget	140

10.3.5.3.4.2	MU Value	140

10.3.5.4	Summary	141

10.3.6	OTA receiver intermodulation	141

10.3.6.1	General	141

10.3.6.2	In-door anechoic chamber	141

10.3.6.2.1	General	141

10.3.6.2.2	Calibration	142

10.3.6.2.3	Procedure	142

10.3.6.2.4	MU assessment	143

10.3.6.2.4.1	MU Budget	143

10.3.6.2.4.2	MU Value	143

10.3.6.3	CATR	144

10.3.6.3.1	General	144

10.3.6.3.2	Calibration	144

10.3.6.3.3	Procedure	144

10.3.6.3.4	MU assessment	145

10.3.6.3.4.1	MU Budget	145

10.3.6.3.4.2	MU Value	145

10.3.6.4	Summary	146

10.3.7	OTA in-channel selectivity	146

10.3.7.1	General	146

10.3.7.2	In-door anechoic chamber	146

10.3.7.2.1	General	146

10.3.7.2.2	Calibration	146

10.3.7.2.3	Procedure	147

10.3.7.2.4	MU assessment	147

10.3.7.2.4.1	MU Budget	147

10.3.7.2.4.2	MU Value	147

10.3.7.3	CATR	147

10.3.7.3.1	General	147

10.3.7.3.2	Calibration	147

10.3.7.3.3	Procedure	148

10.3.7.3.4	MU assessment	148

10.3.7.3.4.1	MU Budget	148

10.3.7.3.4.2	MU Value	148

10.3.7.4	Summary	148

10.4	Measurement uncertainty for In-band TRP requirements	148

10.4.1	OTA Base station output power requirement	148

10.4.1.1	General	148

10.4.1.2	In-door anechoic chamber	148

10.4.1.2.1	General	148

10.4.1.2.2	Calibration	149

10.4.1.2.3	Procedure	149

10.4.1.2.4	MU assessment	150

10.4.1.2.4.1	MU Budget	150

10.4.1.2.4.2	MU Value	150

10.4.1.3	CATR	152

10.4.1.3.1	General	152

10.4.1.3.2	Calibration	153

10.4.1.3.3	Procedure	153

10.4.1.3.4	MU assessment	154

10.4.1.3.4.1	MU Budget	154

10.4.1.3.4.2	MU Value	154

10.4.1.4	Near Field	156

10.4.1.4.1	General	156

10.4.1.4.2	Calibration	156

10.4.1.4.3	Procedure	156

10.4.1.4.4	MU assessment	156

10.4.1.4.4.1	MU Budget	156

10.4.1.4.4.2	MU Value	157

10.4.1.5	Summary	159

10.4.2	OTA Adjacent channel leakage power ratio	159

10.4.2.1	General	159

10.4.2.2	In-door anechoic chamber	159

10.4.2.2.1	General	159

10.4.2.2.2	Calibration	159

10.4.2.2.3	Procedure	160

10.4.2.2.4	MU assessment	160

10.4.2.2.4.1	MU Budget	160

10.4.2.2.4.2	MU Value	160

10.4.2.3.1	General	162

10.4.2.3.2	Calibration	162

10.4.2.4	Near Field	165

10.4.2.4.1	Description	165

10.4.2.4.2	Calibration	165

10.4.2.4.3	Procedure	165

10.4.2.4.4	MU assessment	166

10.4.2.4.4.1	MU Budget	166

10.4.2.4.4.2	MU Value	166

10.4.2.5	Summary	168

10.4.3	OTA Operating band unwanted emission	168

10.4.3.1	General	168

10.4.3.2	In-door anechoic chamber	168

10.4.3.2.1	General	168

10.4.3.2.2	Calibration	169

10.4.3.2.3	Procedure	169

10.4.3.2.4	MU assessment	169

10.4.3.2.4.1	MU Budget	169

10.4.3.2.4.2	MU Value	169

10.4.3.4	Near Field	173

10.4.3.4.1	General	173

10.4.3.4.2	Calibration	173

10.4.3.4.3	Procedure	173

10.4.3.4.4	MU assessment	173

10.4.3.4.4.1	MU Budget	173

10.4.3.4.4.2	MU Value	173

10.4.3.5	Summary	173

10.4.4	OTA Spectrum emission mask	174

10.4.4.1	General	174

10.4.4.2	In-door anechoic chamber	174

10.4.4.2.1	General	174

10.4.4.2.2	Calibration	174

10.4.4.2.3	Procedure	174

10.4.4.2.4	MU assessment	174

10.4.4.2.4.1	MU Budget	174

10.4.4.2.4.2	MU Value	174

10.4.4.3	CATR	176

10.4.4.3.1	General	176

10.4.4.3.2	Calibration	176

10.4.4.3.3	Procedure	176

10.4.4.3.4	MU assessment	177

10.4.4.3.4.1	MU Budget	177

10.4.4.3.4.2	MU Value	177

10.4.4.4	Near Field	179

10.4.4.4.1	General	179

10.4.4.4.2	Calibration	179

10.4.4.4.3	Procedure	179

10.4.4.4.4	MU assessment	179

10.4.4.4.4.1	MU Budget	179

10.4.4.4.4.2	MU Value	179

10.4.4.5	Summary	179

10.5	Measurement uncertainty for Out-of-band TRP requirements	180

10.5.1	General	180

10.5.2	Transmitter mandatory spurious emissions	180

10.5.2.1	General	180

10.5.2.2	General chamber	181

10.5.2.2.1	General	181

10.5.2.2.2	Calibration	181

10.5.2.2.3	Procedure	182

10.5.2.2.4	MU assessment	182

10.5.2.2.4.1	MU Budget	182

10.5.2.2.4.2	MU Value	183

10.5.2.3	Void	184

10.5.2.3A	Reverberation chamber	184

10.5.2.3A.1	General	184

10.5.2.3A.2	Chamber characterization	184

10.5.2.3A.3	Calibration	186

10.5.2.3A.4	Procedure	186

10.5.2.3A.5	MU assessment	187

10.5.2.4	Test Tolerance	187

10.5.2.5	Summary	188

10.5.3	Receiver spurious emissions	188

10.5.3.1	General	188

10.5.3.2	General chamber	188

10.5.3.2.1	General	188

10.5.3.2.2	Calibration	188

10.5.3.2.3	Procedure	188

10.5.3.2.4	MU assessment	189

10.5.3.2.4.1	MU Budget	189

10.5.3.2.4.2	MU Value	189

10.5.3.3	Void	190

10.5.3.4	Test Tolerance	190

10.5.3.5	Summary	191

10.5.4	Additional (co-existence) spurious emissions	191

10.5.4.1	General	191

10.5.4.2	CATR	192

10.5.4.2.1	General	192

10.5.4.2.2	Calibration	192

10.5.4.2.3	Procedure	192

10.5.4.2.4	MU assessment	192

10.5.4.2.4.1	MU Budget	192

10.5.4.2.4.2	MU Value	193

10.5.4.3	Void	194

10.5.4.4	Test Tolerance	194

10.5.4.4	Summary	194

10.6	Measurement uncertainty for Co-location requirements	194

10.6.1	General	194

10.6.2	OTA transmitter OFF power	194

10.6.2.1	General	194

10.6.2.2	In-door anechoic chamber	195

10.6.2.2.1	General	195

10.6.2.2.2	Calibration	195

10.6.2.2.3	Procedure	195

10.6.2.2.4	MU assessment	195

10.6.2.2.4.1	MU Budget	195

10.6.2.2.4.2	MU Value	195

10.6.2.3	CATR	195

10.6.2.3.1	General	195

10.6.2.3.2	Calibration	195

10.6.2.3.3	Procedure	195

10.6.2.3.4	MU assessment	196

10.6.2.3.4.1	MU Budget	196

10.6.2.3.4.2	MU Value	196

10.6.2.5	Summary	198

10.6.3	OTA co-location spurious emissions	198

10.6.3.1	General	198

10.6.3.2	In-door anechoic chamber	198

10.6.3.2.1	General	198

10.6.3.2.2	Calibration	198

10.6.3.2.3	Procedure	198

10.6.3.2.4	MU assessment	198

10.6.3.2.4.1	MU Budget	198

10.6.3.2.4.2	MU Value	198

10.6.3.3	CATR	198

10.6.3.3.1	General	198

10.6.3.3.2	Calibration	198

10.6.3.3.3	Procedure	198

10.6.3.3.4	MU assessment	199

10.6.3.3.4.1	MU Budget	199

10.6.3.3.4.2	MU Value	199

10.6.3.5	Summary	201

10.6.4	OTA Transmitter intermodulation	201

10.6.4.1	General	201

10.6.4.2	In-door anechoic chamber	201

10.6.4.2.1	General	201

10.6.4.2.2	Calibration	202

10.6.4.2.3	Procedure	202

10.6.4.2.4	MU assessment	202

10.6.4.2.4.1	MU Budget	202

10.6.4.2.4.2	MU Value	203

10.6.4.3	CATR	204

10.6.4.3.1	General	204

10.6.4.3.2	Calibration	204

10.6.4.3.3	Procedure	204

10.6.4.3.4	MU assessment	204

10.6.4.3.4.1	MU Budget	204

10.6.4.3.4.2	MU Value	204

10.6.4.4	Shielded Anechoic Chamber	204

10.6.4.4.1	General	204

10.6.4.4.2	Calibration	205

10.6.4.4.3	Procedure	205

10.6.4.4.4	MU assessment	206

10.6.4.4.4.1	MU Budget	206

10.6.4.4.4.2	MU Value	206

10.6.4.5	Summary	207

10.6.5	OTA co-location blocking	207

10.6.5.1	General	207

10.6.5.2	In-door anechoic chamber	207

10.6.5.2.1	General	207

10.6.5.2.2	Calibration	207

10.6.5.2.3	Procedure	207

10.6.5.2.4	MU assessment	208

10.6.5.2.4.1	MU Budget	208

10.6.5.2.4.2	MU Value	208

10.6.5.3	CATR	208

10.6.5.3.1	General	208

10.6.5.3.2	Calibration	208

10.6.5.3.3	Procedure	208

10.6.5.3.4	MU assessment	209

10.6.5.3.4.1	MU Budget	209

10.6.5.3.4.2	MU Value	209

10.6.5.5	Summary	210

10.7	Measurement uncertainty for Out-of-band Blocking requirements	210

10.7.1	General	210

10.7.2	General Chamber	211

10.7.2.4.2	MU Value	213

10.7.2.4.3	Interferer MU total	214

10.7.3	Summary	215

10.8.2.1	Spherical equal angle grid	216

10.8.2.2	Reference angular step criteria	217

10.8.2.3	Spherical equal area grids	222

10.8.2.4	Spherical Fibonacci grids	223

10.8.2.5	Orthogonal cuts grids	223

10.8.2.6	Wave vector space sampling grid	225

10.8.3	TRP measurement procedures	228

10.8.3.1	Procedures for base station output power	229

10.8.3.1.1	General	229

10.8.3.1.2	Two cuts with pattern multiplication	229

10.8.3.1.3	Full sphere	229

10.8.3.1.4	Beam-based directions	230

10.8.3.2	Procedures for SEM and OBUE	230

10.8.3.2.1	General	230

10.8.3.2.2	Two cuts with pattern multiplication	230

10.8.3.2.3	Two or Three cuts	230

10.8.3.2.4	Full sphere	230

10.8.3.2.5	Beam-based directions	230

10.8.3.3	Procedures for ACLR	231

10.8.3.3.1	General	231

10.8.3.3.2	TRP fraction method	231

10.8.3.4	Procedures for transmitter spurious emissions and EMC emissions	231

10.8.3.4.0	General	231

10.8.3.4.1	Pre-scan	231

10.8.3.4.2	Peak method	232

10.8.3.4.3	Equal Sector with peak average method	232

10.8.3.4.4	Two or three cuts with dense sampling	232

10.8.3.4.5	Full sphere with sparse sampling	233

10.8.3.4.6	Full sphere	233

10.11	Measurement uncertainty for demodulation performance requirements	233

10.11.1	General	233

10.11.2	In-door anechoic chamber	234

10.11.2.1	General	234

10.11.2.2	Calibration	234

10.11.2.3	Procedure	234

10.11.2.4	MU assessment	234

10.11.3	CATR	234

10.11.3.1	General	234

10.11.3.2	Calibration	235

10.11.3.3	Procedure	235

10.11.3.4	MU assessment	235

Annex A:	BS-to-BS co-location background	236

Annex B:	OTA sensitivity measurement error contribution descriptions	237

B.1	Indoor Anechoic Chamber	237

B.2	Compact Antenna Test Range	237

B.3	One Dimensional Compact Range	237

B.4	Near Field Test Range	237

Annex C:	Beam sweeping	238

C.1	Simulation results	238

C.2	Measurement results	239

C.3	Design of beam sweeping test signal	241

Annex D:	Sparse sampling for spurious emissions	242

Annex E:	Radiated transmit measurement error contribution descriptions	247

E.1	Indoor Anechoic Chamber	247

E.2	Compact Antenna Test Range	249

E.3	Near Field Test Range	252

E.4	General Chamber	255

E.5	Reverberation Chamber	258

Annex F:	Test equipment uncertainty values	259

F.1	Measurement Equipment uncertainty values	259

F.2	MU of TE derived from conducted specification	259

Annex G:	Power density measurements close to EUT	261

Annex H:	Change history	263

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is the Technical Report for the Work Item on Base Station (BS) radiated Radio Frequency (RF) requirements for Active Antenna System (AAS) [2]. The present document captures the background and the decisions on the specification of radiated RF requirements that are applicable to AAS BS.

The existing Rel-13 AAS specification contains requirements for both single RAT UTRA FDD, UTRA TDD and E-UTRA as well as MSR (including single RAT MSR requirements for each of the RATS), it was agreed that it may not be necessary to support so many variants for the OTA AAS BS hence a specification complexity reduction investigation was carried out. It was found the following was not needed for the OTA AAS BS requirements:

1.	Radiated requirements applicable for eAAS not support UTRA TDD.

2.	Radiated requirements applicable for eAAS not include UTRA pilot requirements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	RP-170844: "Work Item Description for Enhancements of Base Station (BS) RF and EMC requirements for Active Antenna System (AAS)"

[3]	3GPP TS 37.105: "Active Antenna System (AAS) Base Station (BS) transmission and reception"

[4]	3GPP TR 37.842: "Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)"

[5]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios"

[6]	3GPP TR 37.840: "Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station"

[7]	3GPP TS 37.104: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception"

[8]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

[9]	Recommendation ITU-R SM.328-11: "Spectra and bandwidth of emissions", ITU/R

[10]	3GPP TR 25.951: "FDD Base Station (BS) classification"

[11]	3GPP TR 37.809: "Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements"

[12]	3GPP TR 36.931: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) requirements for LTE Pico Node B"

[13]	3GPP TS 37.114: "Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)"

[14]	3GPP TS 37.113: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)"

[15]	3GPP TS 36.113: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[16]	3GPP TS 25.113: "Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[17]	Conversion of power to field strength in the book "Automotive Antenna Design and Applications", by Victor Rabinovich, Nikolai Alexandrov, Basim Alkhateeb; 2010

[18]	Recommendation ITU-R SM.329-10: "Unwanted emissions in the spurious domain"

[19]	3GPP TS 25.104: "Universal Terrestrial Radio Access (UTRA); Base Station (BS) radio transmission and reception (FDD)"

[20]	3GPP TS 25.105: "Universal Terrestrial Radio Access (UTRA); Base Station (BS) radio transmission and reception (TDD)"

[21]	3GPP TS 25.141: "Base Station (BS) conformance testing (FDD)"

[22]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing"

[23]	3GPP TS 37.145-1: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing"

[24]	3GPP TS 37.145-2: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: Radiated conformance testing"

[25]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE

[26]	TSGR4#8(99)631: "Antenna-to-Antenna Isolation Measurements", Allgon

[27]	Report ITU-R M.2244: "Isolation between antennas of IMT base stations in the land mobile service", ITU-R

[28]	IEC 61000-4-3: 2006+AMD1:2007+AMD2:2010: “Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test”

[29]	Hald, J.; Hansen, J. E.; Jensen, F. & Holm Larsen, F. Hansen, J. (Ed.) Spherical Near-Field Antenna Measurements, Peter Peregrinus Ltd., 1998, vol 26.

[30]	"Sparse Sampling Analysis Tool" Matlab code: https://se.mathworks.com/matlabcentral/fileexchange/67143-sparse-sampling-analysis-tool?s_tid=srchtitle

[31]	J. Fridén, A. Razavi, and A. Stjernman, “Angular sampling, Test Signal, and Near-Field Aspects for Over-the-Air Total Radiated Power Assessment in Anechoic Chambers”, IEEE Access, 2018, https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=8470084.

[32]	IEEE Std 145: “IEEE Standard Definitions of Terms for Antennas”, IEEE

[33]	IEC 61000-4-21. “Electromagnetic compatibility (EMC) - Part 4-21: Testing and measurement techniques - Reverberation chamber test methods” Edition 2.0 2011-01, The International Electrotechnical Commision (IEC), 2011

[34]	Hill, D.A., “Boundary fields in reverberation chambers,” IEEE Transactions on Electromagnetic Compatibility, vol. 47, no. 2, pp. 281-290, May 2005

[35]	Krauthäuser, H. G.; Winzerling, T.; J., N.; Eulig, N. & Enders, A. “Statistical interpretation of autocorrelation coefficients for fields in mode-stirred chambers” 2005 International Symposium on Electromagnetic Compatibility, 2005. EMC 2005., 2005, 2, 550-555

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

NOTE:	Multi-word definitions are treated as linguistic expressions and printed in italic font throughout this requirement specification. Linguistic expressions may not be split and are printed in their entirety.

active antenna system base station: base station system which combines an antenna array with a transceiver unit array and a radio distribution network

array element: subdivision of a passive antenna array, consisting of a single radiating element or a group of radiating elements, with a fixed radiation pattern

antenna array: group of radiating elements characterized by the geometry and the properties of the array elements

antenna gain: ratio of the radiation intensity, in a given direction, to the radiation intensity that would be obtained if the power accepted by the antenna were radiated isotropically

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

array factor: radiation pattern of an array antenna when each array element is considered to radiate isotropically

NOTE:	When the radiation pattern of individual array elements are identical, and the array elements are congruent under translation, then the product of the array factor and the array element radiation pattern gives the radiation pattern of the entire array.

angle of arrival: is the direction of propagation of electromagnetic wave incident on an AAS BS antenna array

basic limit: emissions limit taken from the non-AAS BS specifications that is converted into a per TAB connector TX min cell group AAS BS emissions limit, or into a per TAB connector RX min cell group AAS BS emissions limit by scaling, depending on the context

beam: main lobe of a radiation pattern from an AAS BS

NOTE:	For certain AAS BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is supposed to be found

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

co-location reference antenna: reference antenna used for co-location requirements.

directions diagram: two dimensional Cartesian diagram showing φ on the horizontal axis and minus θ on the vertical axis

directivity: ratio of the radiation intensity in a given direction from the antenna to the radiation intensity averaged over all directions

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

EIRP accuracy directions set: beam peak directions for which the EIRP accuracy requirement is intended to be met

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

EMC antenna port:	RF connector referred to as antenna port in EMC requirements

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE 1:	The sensitivity is the minimum received power level at which a RAT specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

hybrid AAS BS:	AAS BS which has both a conducted RF interface and a radiated RF interface in the far field and conforms to a hybrid requirements set

hybrid requirements set: Complete set of requirements applied to a hybrid AAS BS with both conducted and radiated  requirements

minSENS: the lowest declared EIS value for the OSDD’s declared for OTA sensitivity requirement

minSENS RoAoA:	the reference RoAoA associated with the OSDD with the lowest declared EIS value.

multi-band TAB connector: TAB connector supporting operation in multiple operating bands through common active electronic components(s)

NOTE:	For common TX and RX TAB connectors, the definition applies where common active electronic components are in the transmit path and/or in the receive path.

OTA AAS BS: AAS BS which has ≥8 transceiver units for E-UTRA or MSR and ≥4 transceiver units for UTRA per cell and has a radiated RF interface only and conforms to the OTA requirements set.

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA REFSENS RoAoA: is the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction.

NOTE:	This contour will be related to the average element/sub-array radiation pattern 3dB beam width

OTA requirements set:	complete set of OTA requirements applied to an OTA AAS BS

OTA sensitivity directions declaration: set of manufacturer declarations comprising at least one set of declared minimum EIS values (with related RAT and channel bandwidth), and related directions over which the EIS applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

radiating element: basic building block of an array element characterized by its radiation properties

radiation pattern: angular distribution of the radiated electromagnetic field or power level in the far field region

radio distribution network: linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

Reference RoAoA:	The sensitivity RoAoA associated with the receiver target reference direction for each OSDD.

receiver target: AoA in which reception is performed by AAS BS

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to particular OSDD

receiver target reference direction: direction inside the OTA sensitivity directions declaration declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA

sensitivity RoAoA: RoAoA within the OTA sensitivity directions declaration, within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific AAS BS direction setting

single direction requirement: AAS BS requirement which is applied in a specific direction within the OTA coverage range for the Tx and when the AoA of the incident wave of a received signal is within the OTA REFSENS RoAoA or the minSENS RoAoA as appropriate for the receiver.

TAB connector: transceiver array boundary connector

total radiated power: is the total power radiated by the antenna.

NOTE 1:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.

NOTE 2:	total radiated power is defined in both the near-field region and the far-field region.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transceiver unit: active unit consisting of transmitter and/or receiver which transmits and/or receives radio signals, and which may include passive RF filters

transceiver unit array: array of transceiver units which generate radio signals in the transmit direction and accept radio signals in the receive direction

transmitter OFF period: time period during which the transmitter is scheduled not to transmit

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter ON period: time period during which the transmitter is transmitting data and/or reference symbols

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter transient period: time period during which the transmitter unit belonging to the transceiver unit array is changing from the OFF period to the ON period or vice versa

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

TRP requirement: AAS BS requirements, which requires dual-polarized measurements of the figure of merit over the whole sphere around the DUT

TRP summation error: the relative difference between theoretical TRP and numerically estimated TRP

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BeWθ	The beamwidth in the θ-axis in degrees.

BeWφ	The beamwidth in the φ-axis in degrees.

BeWθ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees.

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees.

DRX_OTA_MARGIN	Directivity implementation margin in UL

EVM	Error Vector Magnitude

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular operating band.

PRated,c,TRP	The rated total radiated power when all the transmitter units are operating at their rated output power for a single carrier

The azimuth angle (defined between -180° and 180°)

Elevation angle of the signal direction (defined between -90° and 90°, 0° represents the direction perpendicular to the antenna array)

TRPEstimate	Numerically approximated TRP

TRPReference	Theoretical TRP

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

AAS BS	Active Antenna System Base Station

ACIR	Adjacent Channel Interference Ratio

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

BS	Base Station

CACLR	Cumulative ACLR

CATR	Compact Antenna Test Range

DUT	Device Under Test

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

EMC	Electromagnetic compatibility

EMC RE	EMC Radiated Emissions

ERP	Effective Radiated Power

EUT	Equipment Under Test

FRC	Fixed Reference Channel

FSF	Fixed Scaling Factor

LA	Local Area

MCL	Minimum Coupling Loss

MR	Medium Range

OBUE	Operating Band Unwanted Emissions

OSDD	OTA Sensitivity Direction Declaration

OTA	Over-the-Air

RDN	Radio Distribution Network

RF RSE	RF Radiated Spurious Emissions

RIB	Radiated Interface Boundary

RMC	Reference measurement channel

RoAoA	Range of Angles of Arrival

RXU	Receiver Unit

SEM	Spectrum Emission Mask

SGH	Standard Gain Horn

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TRP	Total Radiated Power

TRXU	Transceiver Unit

TRXUA	Transceiver Unit Array

TXU	Transmitter Unit

WA	Wide Area

# 4	General

## 4.1	Work Item scope

Rel-15 AAS BS Work Item (WI) is to specify radiated Radio Frequency (RF), BS demodulation and EMC requirements for Active Antenna System (AAS) Base Station (BS) covering single Radio Access Technology (RAT) capable BS supporting UTRA or E-UTRA, and multi-RAT capable BS operating in Band Category (BC) 1 and BC3, for Wide Area (WA), Medium Range (MR), and Local Area (LA) coverage to ensure necessary coexistence. Wide Area BS class will be prioritized.

## 4.2	Work Item objectives

The radiated AAS BS specification is based on translating the Rel-13 AAS BS conducted requirements from TS 37.105 [3] to over the air requirements which provide the same level of performance and protection as the Rel-13 AAS BS requirements do.

It is composed of the following work tasks:

a) Core RF requirements

b) Conformance testing requirements, derived from the core RF requirements, and demodulation testing requirements

Due to the necessary study required to arrive at requirement that does not require excessive measurement and will be practical (i.e. easy to measure) and robust, it is decided to include a study phase to identify the OTA core and conformance concepts before writing the specification text.

Detailed WID texts are presented in [2].

## 4.3	General AAS BS RF architecture

An AAS BS may have 2 different types of RF interface at which the RF and performance requirements are defined:

-	Conducted RF interface at the transceiver array boundary (TAB) and a radiated interface boundary (RIB) in the far field for a hybrid AAS BS.

-	Radiated interface boundary (RIB) only for an OTA AAS BS.

For both types of AAS BS the radio architecture is represented by three main functional blocks:

-	the transceiver unit array (TRXUA),

-	the radio distribution network (RDN), and

-	the antenna array (AA).

The transceiver units (TRXU) interface with the base band processing within the AAS BS.

The TRXUA consists of multiple transmitter units (TXU) and receiver units (RXUs). The TXU takes the baseband input from the AAS BS and provides the RF TX outputs. The OTA AAS BS is restricted to architectures which have ≥8 TRXUs for E_UTRA and ≥4 TRXUs for UTRA, restricting the minimum number of TRXUs in the OTA AAS BS architecture allows the emissions scaling factor to be fixed and avoids defining an OTA definition of an active transceiver unit which is difficult without the ability to count TAB connectors. AAS BS with small numbers of TRXUs should retain a conducted interface and are classified as hybrid AAS BS.

The RF TX outputs are distributed to the AA via a RDN. The RXU performs the reverse of the TXU operations. The RDN performs the distribution of the TX outputs into the corresponding antenna paths and antenna elements, and a distribution of RX inputs from antenna paths in the reverse direction. The TXU and RXU can be separated and can have different mapping through the RDN towards the AA.

For the hybrid AAS BS the transceiver array boundary (TAB) is the point(s) at which the TRXUA is connected to the RDN. The point where a TXU or RXU connects with the RDN is equivalent to an "antenna connector" of a non-AAS BS and is called TAB connector. The TAB connector is defined as conducted requirement reference point. The transmitted signal per carrier from one Transmitter Unit appear at one or more TAB connector(s) and the received signals per carrier from one or more TAB connector(s) appear at a single RXU.

Figure 4.3-1 shows a general architecture of a hybrid AAS BS radio architecture, where M is the total number of transceiver units and K is the total number of TAB connectors at the transceiver array boundary.

The OTA AAS BS has no transceiver array boundary or TAB connectors defined as it has no conducted requirements. The OTA AAS BS radiated requirements are defined at the radiated interface boundary (RIB).

Figure 4.3-2 shows a general architecture of an OTA AAS BS radio architecture, where P is the total number of transceiver units and P ≥ 8 (E-UTRA) or P ≥ 4 (UTRA) for OTA AAS BS.

NOTE 1:	The RDN may consist of a simple one to one mapping between the TXU(s)/RXU(s) and the passive antenna array. In this case, the RDN would be a logical entity but not necessarily a physical entity.

NOTE 2:	The antenna array includes various implementations and configurations e.g. polarization, array geometry (including element factor and element separation), etc.

NOTE 3:	The physical location of the TRXUA, the RDN, and the AA may differ from this logical representation and is implementation dependent.

NOTE 4:	No specific mapping in the RDN between TAB connectors and antenna elements is assumed. Further the number of separate receiver and transmitter units as well as the mapping in the RDN between TAB connectors and radiating elements can differ between the transmit and receive directions. Both AAS BS reference architectures allow for full asymmetry between receiver path and transmit path.

NOTE 5:	For AAS BS capable of supporting applications employing beamforming, all or subgroups of TAB connectors can be configured with designated amplitude and phase weights such that one or more beams are radiated from the antenna array.

NOTE 6:	If text of this TR and figures 4.3-1, 4.3-2 contradict each other, then the TR text applies.

NOTE 7:	In the case of the OTA AAS BS, the fixed scaling factor (FSF) equals 8 for E-UTRA and 4 for UTRA and is based on the maximum number of layers/streams specified in Rel-12. The FSF value may be further reconsidered for future releases if the maximum number of layers/streams supported by RAT(s) supported by the AAS BS is changed.

NOTE 8:	If an OTA AAS BS is declared to support more than 1 cell (Ncells>1) the total number of transceiver units is greater than FSF* Ncells , i.e. for 8* Ncells for E-UTRA or 4* Ncells for UTRA.

Figure 4.3-1: General architecture of hybrid AAS BS and the reference points for the conducted and radiated requirements

Figure 4.3-2: General architecture of OTA AAS BS and the reference point for the radiated requirements

## 4.4	Specification organization

### 4.4.1	TS 37.105: RF and demodulation requirements

The existing AAS BS technical specification TS 37.105 [3] will be updated to include both hybrid requirements set(s) and OTA requirements set.

The full OTA set of RF requirements will be added as additional sub-clauses to the existing radiated requirements in the Rel-15 version of the technical specification TS 37.105.

The updated technical specification TS 37.105 will contain all the existing conducted requirements as specified and the radiated requirements section will be modified to contain a full set of OTA requirements for OTA AAS BS.

The requirements in TS 37.105 can be used to describe either an OTA requirements set or a hybrid requirements set(s). There will be at least one hybrid requirements set which is the same as the existing AAS BS Rel-13 set of conducted and radiated requirements.

The applicability of requirements clause in TS 37.105 can be modified to define which sub-clauses are required to show compliance to either the OTA requirements set or the hybrid requirements set(s).

### 4.4.2	TS 37.114: EMC requirements

The existing EMC requirements defined in TS 37.114 [13] for hybrid AAS BS will be extended to also include EMC requirements for OTA AAS BS.

In the Rel-15 version of the TS 37.114 the set of EMC requirements for OTA AAS BS will be added as additional sub-clauses to the already existing EMC requirements defined for the hybrid AAS BS.

## 4.5	Coordinate system

OTA requirements are stated in terms of electromagnetic and spatial parameters. The electromagnetic parameters are specified either in terms of power (dBm) or field strength (dBV/m). The spatial parameters are specified in a Cartesian coordinate system (x, y, z) with an addition of spherical coordinates (r, ) for manufacturer declared parameters.

The orientation and location of the coordinate system is declared by the base station manufacturer as described in TR 37.842 [4], sub-clause 7.1.

For RF core requirements the vendor declares the location of this coordinate system origin in reference to an identifiable physical feature of the AAS BS enclosure. The vendor also declares the orientation of this coordinate system in reference to an identifiable physical feature of the AAS BS enclosure. However, for background information in this TR also other conventions for spatial angles may be used, e.g. definitions used in IEEE Standard Test Procedures for Antennas [25].

## 4.6	AAS BS classifications

### 4.6.1	Minimum coupling loss and minimum distance

AAS BS is classified according to the deployment scenarios defined in sub-clause 5.2.1 in TR 37.840 [6] and those scenarios are the same as for non-AAS BS. One of the key parameters that characterise each of the deployment scenarios is the MCL between BS and UE, which includes the path loss and the antenna gains of both UE and BS. For OTA AAS BS, without antenna connectors, minimum distance along the ground is used to characterise the deployment scenarios.

### 4.6.2	BS classifications

AAS BS classes for BS without antenna connectors are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

AAS BS classes for BS with antenna connectors are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to minimum coupling loss equal to 45 dB.

The deployment scenarios associated with BS classes and its definitions are exactly the same for BS both with and without connectors. An MCL of 70dB corresponds to a minimum distance of around 35m, 53dB to around 5m and 45dB to around 2m respectively for BS with connectors.

## 4.7	Polarization

Polarization is a vital characteristic for antennas used for radio communication. Both radio communication antennas and electromagnetic waves are said to have a polarization. According to eastablished and accepted standard definitions of terms for antennas [32], the viewpoint taken for polarization is that this term can be used in three related meanings;

-	Polarization can refer to a field vector at some point in space.

-	Polarization can refer to a plane wave.

-	Polarization can refer to an antenna.

Based on the geneal description above, polarization aspects for OTA requirements can be summarized as listed in table 4.7-1 for transmitter requirements and 4.7-2 for receiver requirements.

Table 4.7-1: Description of polarization aspects for transmitter requirements

| Requirement | Description |
| --- | --- |
| Radiated transmit power | The declared EIRP is defined as EIRP = EIRPp1 + EIRPp2, where p1 and p2 represents two orthogonal polarizations. |
| OTA Base Station output power | The declared output power based on TRP captures radiated power for two orthogonal polarizations. |
| OTA Output power dynamics | The EIRP level in this requirement is defined as EIRP = EIRPp1 + EIRPp2, where p1 and p2 represents two orthogonal polarizations. |
| OTA Transmit ON/OFF power | The TDD OFF power emission is measured at the CLTA outputs as a power sum of supported polarizations. |
| OTA Transmitter signal quality | The EIRP level in this requirement is defined as EIRP = EIRPp1 + EIRPp2, where p1 and p2 represents two orthogonal polarizations. |
| OTA Unwanted Emissions | Radiated unwanted emission based on TRP captures radiated power for two orthogonal polarizations. Except for co-location requirements, where the emission is measured at the CLTA outputs as a power sum of supported polarizations. |
| OTA Transmitter intermodulation | Radiated unwanted emission based on TRP captures radiated power for two orthogonal polarizations. The interferer signal is fed to the CLTA, where the power is divided per supported polarizations. |

Table 4.7-2: Description of polarization aspects for receiver requirements

| Requirement | Description |
| --- | --- |
| OTA sensitivity | The declared EIS level applies to each supported polarization, under assumption of polarization match. |
| OTA Reference sensitivity level | The wanted signal applies to all supported polarizations, under the assumption of polarization match. |
| OTA Dynamic range | The wanted and interferer signals apply to all supported polarizations, under the assumption of polarization match. |
| OTA Adjacent channel selectivity, general blocking, and narrowband blocking | The wanted and interferer signals apply to all supported polarizations, under the assumption of polarization match. |
| OTA Blocking | The wanted signal applies to all supported polarizations, under the assumption of polarization match. The interferer shall be polarization matched in band and the polarization maintained for out-of-band frequencies. Except for co-location requirements, where the interferer signal is fed to the CLTA per supported polarization. |
| OTA Receiver spurious emissions | OTA receiver emission based on TRP captures radiated power for two orthogonal polarizations. |
| OTA Receiver intermodulation | The wanted and interferer signals apply to all supported polarizations, under the assumption of polarization match. |
| OTA In-channel selectivity | The wanted and interferer signals apply to all supported polarizations, under the assumption of polarization match. |

# 5	Radiated transmitter characteristics

## 5.1	General

### 5.1.1	Spatial requirements

OTA requirements for the AAS BS are belonging to one of the two groups:

-	Directional requirements

-	The manufacturer to declare beam(s) and coverage ranges over which the beam can be steered.

-	An example of the directional requirements is the Rel-13 EIRP requirement.

-	Directional requirement type does not imply the requirement is only in one direction as many requirements have a number of compliance directions. It implies the requirement applies to a single direction at a time.

-	For all directional requirements defined within the OTA coverage range, testing using the narrowest declared beamwidth is sufficient for demonstrating conformance.

-	The number of directions that must be tested for conformance is captured in table 5.1-1 for each of the directional requirements.

-	TRP requirements

-	TRP is defined as:

, where PD(d,,)  is the total power density in W/m2 at a distance d of two orthogonal polarizations.

Correspondingly, and in order that the test requirement part of conformance test specification is measurable the above equation is approximated in the far-field region as the sum of the total EIRP at a number of discrete directions around the sphere as follows:

,where total EIRP of two orthogonal polarizations is sampled at N locations along the -axis and M locations along the φ-axis. For the equation above, 0≤n≤π≤m≤π and the boresight of the AAS antenna is the same as π/2 direction of the -axis.

All OTA requirements are met in either the OTA coverage range or OTA peak direction set(s) as shown in the overview table of the radiated Tx requirements in table 5.1-1.

Table 5.1-1: Overview of radiated Tx requirements

| AAS BS requirement |  | OTA requirement type | Coverage range | Notes |
| --- | --- | --- | --- | --- |
| Base station output power | Output power accuracy for EIRP | Directional requirement | OTA peak directions set | Output power accuracy for EIRP requirement is already included as a core requirement in TS 37.105. |
|  | Output power accuracy for TRP | TRP | n/a |  |
| E-UTRA DL RS power |  | Directional Requirement | OTA peak directions set | Conformance testing is carried out in the reference direction |
| Output power dynamics |  | Directional requirement | OTA peak directions set | Conformance testing is carried out in the reference direction. |
| Transmit ON/OFF power |  | co-location requirement | n/a |  |
| Frequency Error |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Time Alignment Error |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Modulation Quality (EVM) |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction and the maximum directions of the OTA coverage range on each axis. |
| Unwanted emissions |  |  |  |  |
| Occupied Bandwidth |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Adjacent Channel Leakage Radio (ACLR) |  | TRP | n/a |  |
| Spectrum emission mask |  | TRP | n/a |  |
| Spurious emissions |  |  |  |  |
| Mandatory Requirements |  | TRP | n/a |  |
| Protection of the BS receiver of own or different BS |  | co-location requirement | n/a |  |
| Additional spurious emissions requirements |  | TRP | n/a | Includes co-existence in same geographical area |
| Co-location with other base stations |  | co-location requirement | n/a |  |
| Transmitter intermodulation |  | co-location requirement | n/a | The interferer is applied as a co-location requirements, the radiated emissions requirements are specified in the appropriated referenced sub-clause. Generally TRP |

The OTA peak directions set(s) are the same as the EIRP accuracy directions set(s) used in the REL13 specification, the name has been changed as they are now used for other requirements. Examples of OTA peak directions sets remain the same as those in TR 37.842 [4].

Some further examples of the relationship between the OTA peak direction set(s) and the OTA coverage range are as follows:

Example 1.

This example shows the declarations that would be made for an OTA AAS BS with a passive antenna, the system hence has no beam steering capability (this is perhaps unlikely to be an OTA AAS BS however makes extreme example).

The declarations are: Vertical (θ) beam width = 10°, Horizontal (φ) beam width = 65°

Figure 5.1.1-1: Example OTA coverage range and OTA peak directions set for a passive antenna

Note as the beam cannot be steered the OTA peak directions set is a single point and the OTA coverage range is the same as the 3dB beam pattern of the passive antenna.

Example 2.

This example shows the declarations that would be made for an OTA AAS BS with multiple beam widths and beam steering capability. The declarations are defined as including an OTA peak directions set for both the widest beam width and the narrowest beam width in both dimensions. For simplicity it is assumed beam width in θ and ϕ are the same for this example.

-	For the minimum beam width case:  beam width (θ and ϕ) =10°, maximum steering (θ and ϕ) = ±32.5°

-	For the maximum beam width case: beam width (θ and ϕ) =35°, maximum steering (θ and ϕ) = ±25°

-	Note the larger beam width has a smaller declared extreme steering direction, but the potential coverage is greater.

The OTA peak directions sets for the 2 cases are different, however they require a common OTA coverage range, this can be seen in figure 5.1.1-2.

Figure 5.1.1-2: Example OTA coverage range and OTA peak directions set for a beam steering system with multiple beam widths

Note the OTA coverage range is the same for both cases as there is only a single OTA coverage range declared per OTA AAS BS, however there are multiple OTA peak directions sets allowed. In this case the OTA peak direction set for the minimum beam width case is the same as the OTA coverage range, for the maximum beam width case the OTA peak directions set is smaller than the OTA coverage range.

In both examples the OTA peak directions sets are a sub set of the OTA coverage range as required.

In both examples the beam widths for each of the maximum steering directions are shown for information, the beam width for each of the maximum steering direction is declared but it does not form part of the OTA coverage range. It can be seen in both examples that when the beam is steered to its maximum steering direction a portion of the beam is outside of the OTA coverage range. The maximum extent of the OTA coverage range on each axis is declared and used as test directions for the EVM requirement. The directions covered by this area on the directions diagram are not part of the OTA coverage range and hence are not part of the range of directions which are intended to meet the RF requirements, this is acceptable as performance beyond the OTA coverage range is not specified.

### 5.1.2	Co-location requirements

#### 5.1.2.1	Core requirements

There are a number of requirements which are based on the assumption of a co-located BS of the same specification as the BS under test.

These requirements include:

-	Transmitter spurious emissions- co-location with other base stations

-	Transmitter intermodulation

-	Tx OFF requirements

-	Co-location blocking

The non-AAS requirements at the antenna connector assume that the co-located BS have a coupling factor between them of 30 dB. This coupling factor is used to calculate the interferer level at a co-located victim in the case of the spurious emissions requirement, and the interferer level at the antenna connector from a co-located aggressor BS.

The figure of 30 dB is based on measurement of coupling levels between typical BS antennas in the same frequency band and is documented in [26] for the following scenarios:

d

d

I (90º)			II(120º)		III(180º)		IV(Horizontal)		V(Vertical)

Figure 5.1.2-1: The different scenarios used during the antenna coupling measurements. d denotes displacement

In most scenarios the reported coupling factor between antennas was greater than 30 dB. The figure of 30 dB comes from scenario IV when d is 0.25 m.

This figure agrees with the calculation for horizontal isolation coupling in [27].

There are a number of issues which make translating the existing conducted requirement into an OTA requirement difficult.

-	The required level of RX band spurious emissions is too low to measure in the far field.

-	The out-of-band antenna gain of the AAS BS is not known.

-	The interferer power level for the IMD requirement in the far field is very high.

However, OTA requirements have the advantage that the original co-location scenarios can be used to derive the requirements. The requirements can therefore be derived by defining both the type of co-located reference antenna and also how close it is to the AAS BS. This is referred to as the proximity method.

The co-location reference antenna can be regarded as having 2 interfaces, a proximity OTA interface which couples with the AAS BS and a conducted interface where the AAS BS requirements are set (and measured). For each of the co-location requirements the co-location power specified at the transceiver array boundary can be specified at the conducted output/input of the co-location reference antenna offset by 30 dB.

Some co-location requirements involve co-located BS of different frequency bands. As the coupling loss between antennas of different frequency ranges is higher than between antennas of the same frequency range, assuming antennas of the same frequency range can be considered worst case. This is consistent with the approach for the conducted co-location requirements where 30dB coupling loss was assumed for all antenna combinations.

For example, the co-located protection of other RX bands is based on the interfering power being low enough to not degrade the victim BS receiver by more than 0.5 dB, which equates to a power level in 100 kHz of -128 dBm. For the conducted requirement this is translated to the transceiver array boundary by adding the 30 dB isolation. So, the requirement at the output of the co-location reference antenna is -128 dBm (plus any scaling).

As the worst case is based on scenario IV in figure 5.1.2-1 the definition of the co-location reference antenna is based on this scenario. The simplest method for defining the co-location reference antenna is with a diagram as shown in figure 5.1.2-2.

The co-location reference antenna should be a single column passive antenna which has the same vertical radiating dimension (h), frequency range and polarization, as the composite antenna of AAS BS and nominal 65° horizontal half power beamwidth (suitable for 3-sector deployments). It is placed at a distance d from the edge of the AAS BS, as shown in figure 5.1.2-2.

Figure 5.1.2-2: Definition of the co-location reference antenna

Edge-to-edge separation d, between the AAS BS and the co-location reference antenna is set to 0.1 m.

The AAS BS and the co-location reference antenna are to be aligned in a common plane perpendicular to the mechanical bore-sight direction, as shown in figure 5.1.2-2.

The co-location reference antenna and the AAS can have different width.

The vertical radiating regions of the co-location reference antenna and the AAS composite antenna are aligned.

For co-location requirements where the frequency range of the signal at the co-location reference antenna is different from the AAS BS, a co-location reference antenna suitable for the frequency stated in the requirement is assumed.

OTA co-location requirements are based on the power at the conducted interface of a co-location reference antenna; depending on the requirement this interface is either an input or an output.

For AAS BS with dual polarization, the co-location reference antenna has two conducted interfaces each representing one polarization.

The area and dimensions of the radiating elements shown in figure 5.1.2-2 do not form part of the OTA core definition of the co-location reference antenna, they are included in the figure to demonstrate what is meant by the radiating dimension in the definition.

Based on the minimum separation of antennas used in the original scenarios in [26], the edge-to-edge separation d between the co-location reference antenna and the AAS BS is 0.10 m (this is derived from a 0.25 m centre to centre separation on 0.15 m wide antennas).

#### 5.1.2.2	Conformance requirements

##### 5.1.2.2.1	General

Co-location requirements are specified as power levels into or out of each conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of either a co-location test antenna (CLTA) or a standard test antenna (such as a dipole).

The CLTA is suitable for testing AAS BS implemented with a planar antenna array. The method for testing AAS BS with other antenna array implementations is FFS.

##### 5.1.2.2.2	Co-location test antenna

The co-location test antenna is a practical passive antenna based on the definition of the co-location reference antenna, and it is used for conformance testing of the co-location requirements. Thus the CLTA should be within a certain tolerance of the co-location reference antenna definition, so that commonly available passive BS antennas can be used to test the co-location requirements without having to use a large number of test antennas and without having to design specialized co-location test antennas.

For example, where the co-location reference antenna definition specifies that the co-location reference antenna, should be a single column passive antenna which has the same vertical radiating dimension (h) as the composite antenna of AAS, this would be very restrictive when selecting a test antenna. Hence the CLTA should be within a certain tolerance for the vertical radiating dimension (h) such, that a suitable practical BS antenna can be found for each AAS BS.

The CLTA is hence defined as follows:

The CLTA is a single column passive antenna which has a vertical radiating dimension (h) which is equal to the vertical radiating dimension of the AAS BS composite antenna ±30%.

For in-band requirements, the CLTA supports the same frequency range and polarizations as the AAS BS composite antenna.

For out-of-band co-location requirements, the half power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ±3°.

The polarization should match the in-band CLTA polarization

The CLTA has a horizontal half-power beamwidth of 65° ±10°.

The front faces of the AAS BS and the CLTA need to be aligned in a common plane perpendicular to the mechanical bore-sight direction, as shown in figure 5.1.2.2.2-1.

The centre of the vertical radiating regions of the CLTA and the AAS composite antenna need to be aligned.

This is summarised as follows:

Table 5.1.2.2.2-1: CLTA characteristics

| Parameter | in-band CLTA | out-of–band CLTAs |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | N/A |
| Horizontal beamwidth | 65° ± 10° | 65° ± 10° |
| Vertical beamwidth | N/A | The half-power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ± 3° |
| Polarization | match | match to in-band |
| Conducted interface return loss | > 10 dB | > 10 dB |
| NOTE 1: If a multi-column or multi-band antenna is used, the column closest to the AAS BS needs to be selected while other columns are terminated during testing. |  |  |

The alignment between the AAS BS under test and the co-location test antenna is described in Table 5.1.2.2.2-2 and Figure 5.1.2.2.2-1.

Table 5.1.2.2.2-2: CLTA alignment tolerances

| Parameter | in-band | out-of-band |
| --- | --- | --- |
| Edge-to-edge separation between the AAS BS and the CLTA (d) | 0.1 m ± 0.01 m | 0.1 m ± 0.01 m |
| Vertical alignment | centre ± 0.01 m | centre ± 0.01 m |
| Front to band alignment | Radome front ± 0.01 m | Radome front ± 0.01 m |

Figure 5.1.2.2.2-1: Alignment of AAS BS and CLTA

##### 5.1.2.2.3	Standard test antenna

It is captured in subclause 5.6.6.2 that co-location requirements may also be tested with a standardised test antenna such as a dipole. This has a number of advantages:

-	Standard test antennas are easily available and easier to specify.

-	Repeatability between different conformance testing runs will be greater.

-	Using standard test antennas, fewer antennas will be required for conformance testing.

-	A standard test antenna can have higher gain in the direction of the AAS BS than a CLTA, so that signal levels can be higher relative to the noise floor in emission measurements and lower test levels can be used in interference measurements.

In order to test with a standard test antenna, the translation between measured power levels in the standard test antenna and the co-location reference antenna must be shown. The method for translating the power levels in the co-location reference antenna to the standard test antenna and vice versa is FFS.

### 5.1.3	Transmit loss factor assumption

The transmitter loss factor is used to translate conducted power levels to OTA power levels

To align with the emissions requirements based on the recommendation in ITU-R SM.329 [18] the same limits are applied to both conducted and OTA, assuming the transmit loss factor accounting for antenna losses, distribution losses, integration losses etc. in the transmitter path(s) for the in-band, out-of-band and spurious region is assumed to be 0 dB.

## 5.2	Base station output power

### 5.2.1	Background information on the conducted requirement

The conducted single RAT specifications TS 25.104 [19], TS 25.105 [20] and TS 36.104 [8] as well as the MSR specification TS 37.104 [7] and the Rel-13 AAS BS specification TS 37.105 [3] all contain a set of manufacturer’s declarations and a requirement on conducted output power. The requirement is placed upon the rated carrier output power per carrier and applies to each antenna connector in non-AAS BS, and TAB connector in AAS BS. The rated carrier output power per carrier is declared, and the requirement is that the measured value is within ± 2dB (±2.5 dB in extreme conditions) of the declared value. In addition to the accuracy requirement on the carrier output power, a limit on the maximum declarable total rated output power per carrier, dependent on the BS class, is captured in the core specification TS 37.105 although not subject to any conformance test.

In the AAS BS conformance test specification TS 37.145-1 [23], the conducted power accuracy test is not carried out with a single carrier active, but with a set of carriers active such that the total output power at the TAB connector is at its maximum level and hence the power amplifier is operated at its maximum output power. Operation at maximum output power is an essential condition for a number of other transmitter tests, including in-band emissions, TX spurious emissions and EVM.

In the AAS BS specification TS 37.105, a requirement on EIRP accuracy was introduced. The requirement is based on a set of declarations that associate declared beams, beam steering ranges and EIRP achieved at certain points within the steering ranges. The declarations associated with the EIRP accuracy requirement are described in more detail in TR 37.842 [4]. The requirement itself states that the declared EIRP is achieved to within a ±2.2 dB accuracy interval at a number of (declared) directions.

### 5.2.2	OTA requirement

For the OTA requirements set, the possibility of complete OTA testing of output power is needed. This implies that where OTA testing is used, conformance to the conducted requirement cannot be verified. In order to decide on a suitable OTA solution, it is useful to review the purposes and uses of the radiated output power requirement.

-	Verifying the accuracy of the transmitter unit output power

-	The conducted output power requirement achieves this goal. The EIRP accuracy requirement verifies the accuracy of the transmitter unit output together with other factors relating to the antenna array. The EIRP accuracy requirement is a sufficient means to capture the accuracy of the whole system in delivering power.

-	A basis for the power limit for the BS classes

-	The BS class power limits have been derived based on co-existence simulations between different layers in heterogeneous network deployments. Due to the statistical nature of the co-existence simulations, it is to be expected to be the total power that will impact the co-existence statistics, independently of beamforming and the antenna pattern (as is the case for ACLR in general). This implies that an OTA based BS class power limit should be based on TRP.

-	A condition for other transmitter conformance requirements

During conformance testing, it is an important condition for other transmitter requirements that the OTA AAS BS is configured to and operating at maximum power.

### 5.2.3	Output power accuracy

#### 5.2.3.1	Core requirement

The existing EIRP accuracy requirements are adopted for the requirement in normal conditions. The accuracy requirement is hence ±2.2 dB.

As there are now a number of transmitter requirements which are specified as directional requirements the existing directions set name and definition over which the EIRP accuracy is met (i.e. EIRP accuracy directions set) is no longer suitable. EIRP accuracy is now be defined over the more general OTA peak directions set(s), the requirement is modified accordingly.

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within +2.2 dB and -2.2 dB of the claimed value.

EIRP accuracy was derived based on 3 methods outlined in sub-clause 7.2.3, TR 37.842 [4]:

1.	Impacts of accuracy on network performance,

2.	Non-AAS BS EIRP accuracy, and

3.	An estimate of the achievable accuracy by AAS BS.

Whilst much of the effort focused on the last of these the achievable accuracy of an AAS BS and the 3 error model used to estimate this, it was not the only basis for the final agreement.

The impact on network performance throughput is related to the wanted link between the AAS BS and the UE and hence is covered by the EIRP requirements so need not be considered when looking at TRP.

However, the BS class upper power limits are derived based on the interference the BS causes in co-existence scenarios. This was not considered when examining EIRP accuracy network performance. The upper power limits for the medium range and local area BS classes can be found in TR 25.951 [10] and are based on co-existence simulations with wide area networks and the macro layer downlink outage. From these simulations and the results it can be noted that:

-	The results for the medium range output power limit were between +20 to +30 dBm with +24 dBm agreed as a compromise.

-	The results for the local area output power limit were between +37 to +39 dBm with +38 dBm agreed as a compromise.

-	Simulations  in all cases did not take into account the BS output power variation, it can be assumed that this and the antenna network gain variation are assumed to be included as the channel and hence part of the slow fading (10 dB lognormal for the Macro layer and 6 dB for the micro/pico layers).

Considering the range of results found when deriving the power limit and the large variation considered in the slow fading and that the TRP accuracy to be a value between the conducted accuracy and the EIRP accuracy which are only 0.2 dB apart, it is unlikely that network simulation will provide a meaningful result. Hence methods 2 and 3 above were considered.

Unlike EIRP, TRP is not subject to the additional steering error caused by phase mismatch in between transceiver units when considering TRP. As there is no steering error, the other errors in the non AAS base station EIRP accuracy (method 2) and the estimate of the achievable accuracy by AAS BS (method 3) still applicable to TRP. For the latter, the TRP accuracy is expressed as:

TRPaccuracy  =σtx2+ σarray2              [dB]

Hence the TRP accuracy is bounded as follows:

conducted poweraccuracy≤TRPaccuracy≤ EIRPaccuracy

Finally RAN4 agreed to use ±2 dB as the TRP accuracy requirement with compromises by some vendors.

#### 5.2.3.2	Conformance requirement

The core requirement applies to any claimed power, however as the conducted requirement no longer exists it is important that the condition of all transceiver units operating at maximum power is defined for the conformance condition. The TRP accuracy requirement is valid for any operational condition, however if the condition for conformance is that all transceiver units are operating at full power, the TRP will not vary with beam steering. Hence it is only necessary to show conformance in a single beam direction.

#### 5.2.3.3	Extreme conditions

The only conducted transmitter requirement with an extreme requirement is conducted output power accuracy, in sub-clause 6.2.2.2.1 of TS 37.105 [3].

In normal conditions, Pmax,c,TABC shall remain within +2 dB and -2 dB of the configured carrier power for each TAB connector as declared by the manufacturer.

In extreme conditions, Pmax,c,TABC shall remain within +2.5 dB and -2.5 dB of the configured carrier power for each TAB connector as declared by the manufacturer.

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

For the OTA requirement set there are two out power accuracy requirements, one based on EIRP which is also in the hybrid requirement set, and one based on TRP.

The EIRP requirement in sub-clause 9.2.2 is currently only valid for nominal conditions:

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the EIRP accuracy directions set , a manufacturer  claimed EIRP level in the corresponding beam peak direction shall be achievable to within +2.2 dB and -2.2 dB of the claimed value.

The TRP accuracy requirement described in sub-clause 5.2.3.1 is the natural replacement of the conducted output power accuracy as they are directly related to each other. However the difficulty in measuring OTA at extreme temperatures means that practically extreme temperature measurements should be directional. This way the effect of the thermal chamber on the OTA accuracy and calibration can be limited to certain directions, this is not possible with a TRP measurement.

In addition any variation in TRP is included in the EIRP requirement, hence an extreme EIRP requirement will cover the TRP requirement.

The EIRP accuracy requirement will therefore have the extreme temperature requirement added to it for OTA AAS BS only.

The conducted allowance for extreme temperature is 0.5 dB, the same figure is added to the EIRP accuracy core and conformance requirements, for example:

Figure 5.2.3.3-1 example of EIRP accuracy core and conformance requirement, in nominal and extreme conditions

The core requirement for EIRP accuracy for an OTA AAS BS with therefore be ±2.7 dB. There is no extreme requirement for TRP accuracy.

The existing clarification stating that “In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal” also applies to the OTA requirement.

The EIRP accuracy requirement does not apply to the hybrid AAS BS.

### 5.2.4	BS class power limits

The AAS BS output power limits in existing requirements are based on the values derived for the non-AAS BS specifications [7][8][19][20] these power levels are specified as the rated output power per carrier at the antenna connector, i.e. per non-AAS transceiver. In the AAS BS specification for AAS BS with a conducted interface TS 37.105 [3] the requirement is based on both:

-	The rated system output power per carrier, PRated,c,sys

-	The rated output power per TAB connector, PRated,c,TABC

The limit per TAB connector is the same as that used in the non-AAS specifications, the system limit is the non-AAS limit scaled by the number of active transceiver units (capped at the appropriate number for E-UTRA or UTRA), NTXU,counted.

This ensures that the AAS BS does not exceed the power limit per carrier either for the system or per transceiver unit.

The equivalent output power limit for the OTA AAS BS specification is TRP. The AAS BS TRP is equivalent to the system output power (i.e. not the transceiver unit output power) by the following relationship:

PRated,c,TRP = PRated,c,sys – LTX

The OTA AAS BS output power limit can therefore be defined as:

PRated,c,TRP ≤ Non-AAS Power limit  – LTX + 10log(NTXU)

Where NTXU is the minimum number of active transceiver units, for an OTA AAS BS has ≥8 TRXU’s for E-UTRA or ≥4 TRXU’s for UTRA this number is fixed. As both LTX and NTXU have fixed values, the values in the power limit tables do not need to include variables and the adjusted fixed umber can be used.

For example for medium range E-UTRA

PRated,c,TRP ≤ 38 dBm  – 0 +10log(8) ≤ 47 dBm

Hence the power limits table is as follows:

Table 5.2.4-1: UTRA OTA AAS BS rated output power limits for BS classes

| AAS BS class |  | PRated,c,TRP |
| --- | --- | --- |
| Wide Area BS |  | (Note) |
| Medium Range BS |  | ≤ 44 dBm |
| Local Area BS |  | ≤ 30 dBm |
| NOTE: There is no upper limit for the PRated,c,TRP for the Wide Area Base Station. |  |  |

Table 5.2.4-2.: E-UTRA OTA AAS BS rated output power limits for BS classes

| AAS BS class |  | PRated,c,TRP |
| --- | --- | --- |
| Wide Area BS |  | (Note) |
| Medium Range BS |  | ≤ 47 dBm |
| Local Area BS |  | ≤ 33 dBm |
| NOTE: There is no upper limit for the PRated,c,TRP for the Wide Area Base Station. |  |  |

### 5.2.5	Control channel accuracy

#### 5.2.5.1	Background

There are a number of control channel power requirements in TS 37.105 [3]. The requirements are intended to confirm that power accuracy of the control channels is acceptable.

For UTRA

-	UTRA FDD primary CPICH power

-	UTRA TDD primary CCPCH power

-	UTRA FDD additional CPICH power for MIMO mode

And for E-UTRA

-	E-UTRA DL RS power

The UTRA FDD primary CPICH power, UTRA TDD primary CPICH power and the E-UTRA DL RS power are all very similar requirements. They all confirm the accuracy of the specified control channel is within x dB of the power indicated on the BCH (UTRA) or DL-SCH (E-UTRA), where x is different for each requirement and can be found in TS 37.105.

For hybrid AAS BS the possibility that these signals may be transmitted over a number of transceiver units so that the control channels could be beam formed was considered.  The requirement was modified so that the control channel power on a group of TAB connectors (those responsible for transmitting the control channel) was compared to the power indicted on the BCH (UTRA) or DL-SCH (E-UTRA).

The option to have a requirement per connector was also provided by using a declared set of beam forming weights for the TAB connector group and modifying the power indicated on the BCH (UTRA) or DL-SCH (E-UTRA) by the beam forming weight for each TAB connector.

The UTRA FDD addition CPICH power requirement for MIMO is slightly more complicated (both in AAS and non-AAS requirements) as it introduces the concept of “antenna 1”, “antenna 2”, “antenna 3” and “antenna 4”, P-CPICH is transmitted on “antenna 1” and S-CPICH on “antenna 2 - 3”. In non-AAS each “antenna” refers to an antenna connector, in AAS as the signals can be beam formed by a number of transceiver units the “antenna” applies to a declared group of TAB connectors. The requirement in this case is the difference between the P-CPICH power and the S-CPICH power.

As part of the specification simplification work, it was agreed that an UTRA OTA AAS BS would by design be capable of meeting the control channel accuracy requirements, these requirements are hence removed for UTRA OTA AAS BS.

#### 5.2.5.2	Core requirement

The E-UTRA DL RS power requirements is an accuracy requirements with respect to an absolute power indicted on the DL-SCH (E-UTRA).

The indicated power is provided by the BS, and clearly this is related to the BS rated output power. For non-AAS and hybrid AAS BS this power is a conducted power (PRated,c), however the OTA AAS BS does not have a conducted interface.

It is not clear if the power indicted on the DL_SCH for the OTA is related to the conducted power or to the OTA power, however the power indicted on the DL-SCH is controlled by the vendor and is effectively a declared parameter. In addition the control signal antenna gain may not be the same as the user signal beam forming gain hence it is necessary to declare the control signal antenna gain for the requirement. As both bits of information are variable depending on the AAS BS configuration and design the requirement is that the control signal power is within the required accuracy compared to the power level which is intended. The power level for the conformance test is declared.

As the core requirement E-UTRA DL RS power is an absolute requirement, the OTA accuracy requirement can be derived using the same methodology as the EIRP accuracy requirement. The EIRP accuracy requirement allows for an additional (non-correlated) 0.9 dB error.

Hence the control channel absolute accuracy requirement is given by:

For example for DL-RS the conduced requirement is 2.1 dB so the OTA requirement should be:

The core requirement for the E-UTRA DL RS power can hence be worded as:

The control channel power of each carrier shall be within dB of the configured control channel power.

#### 5.2.5.3	Conformance requirement

The control channel power accuracy requirement is intended to ensure that channels within the modulated signal maintain their power accuracy (essentially conforming that sufficient quantization is provided). The test condition for the almost all single direction requirements such as this is that the AAS BS is operating with the highest rated power, as such all the TRX units are operating at maximum power. In such a condition beam forming (for both steering and beam width) are done using phase manipulation and does not vary the accuracy of the power allocated to the control channels. Hence it is not necessary to show conformance in more than one direction.

Conformance testing of the control channel power accuracy is done at the reference steering direction when the system is operating at maximum power (PRated,c,TRP).

## 5.3	Output power dynamics

### 5.3.1	Background information on the conducted requirement

There are a number of output power dynamics requirements defined for AAS BS in TS 37.105 [3], in general they are RAT specific and are as follows:

UTRA FDD output power dynamics requirements are applicable to AAS BS in single RAT UTRA operation (TDD 1.28Mcps, or FDD) and to AAS BS in MSR operation using UTRA:

-	UTRA inner loop power control in the downlink: Inner loop power control in the downlink is the ability of the AAS BS to adjust the transmitted output power of a code channel in accordance with the corresponding TPC commands received in the uplink.

-	Power control dynamic range : The power control dynamic range is the difference between the maximum and the minimum code domain power of a code channel for a specified reference condition

-	Total power dynamic range: Total power dynamic range is the difference between the maximum and the minimum output power for a specified reference condition. The minimum power condition is modeled by using TM4 test model with fractional power allocation condition, while the maximum power condition is for the case of full power allocation specified by TM1.

-	UTRA FDD IPDL time mask: To support IPDL location method in UTRA FDD operation, the AAS BS interrupts all transmitted signals in the downlink (i.e. common and dedicated channels). The IPDL time mask specifies the limits of the TAB connector output power during these idle periods.

E-UTRA UTRA output power dynamics requirements are applicable to AAS BS in single RAT E-UTRA operation and to AAS BS in MSR operation using E-UTRA:

-	RE Power control dynamic range: The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (PRated,c,TABC) for a specified reference condition.

-	Total power dynamic range: Total power dynamic range is the difference between the maximum and the minimum output power for a specified reference condition. The minimum power condition is modeled by using E-TM2/2a test model with only one RB allocation, while the maximum power condition is for the case of all RB’s allocation (as per considered E-UTRA channel bandwidth) specified by E-TM3.1/3.1a.

All requirements are applied per each TAB connector and are differential between 2 measured powers.

### 5.3.2	Output power dynamics OTA: Core requirement

All the power dynamics requirements are in band and are concerned with controlling the power of modulated signals. These wanted signals are of interest between the BS and the UE. Such requirements should be applied as directional requirements and apply to the beam peak directions associated with the beam direction pairs.

As all the conducted output power dynamics requirements are differential they can be translated to OTA output power dynamics requirements with no further modification.

### 5.3.3	Output power dynamics OTA: Conformance requirement

As all output power dynamics requirements are relative between two measured power levels, any variation due to beam steering or beam shaping OTA is the same for both measured power values. The results are therefore unaffected by beam steering or beam shaping in case of OTA output power dynamics.

Hence it is only necessary to show conformance of the OTA output power dynamics requirements in a single direction, i.e. the reference steering direction for the narrowest (i.e. highest EIRP) beam, in a similar set up to the EIRP accuracy test in sub-clause 5.2.2 , i.e. to configure and operate at maximum transmit power during test. The narrowest beam can be determined based on the manufacturer declared beams as defined in declaration D9.3, in clause 9, where the narrowest intended BeWθ and BeWϕ are to be defined at the reference beam direction.

## 5.4	Transmit ON/OFF power

### 5.4.1	Background

The TDD TX OFF power ensures that receivers of other TDD system are not desensitized by the noise from an aggressor BS.

The TX OFF level requirement was first derived for the UTRA TDD requirement, the rationale for the UTRA TDD 1.28Mcps TX off level is given in TR 25.942

Assuming the Noise Figure(NF) of BS is 7dB and Minimum Couple Loss(MCL) is 30dB. For the victim receiver, the Tx OFF power should not exceed the thermal noise. If Tx OFF power is 6dB below thermal noise, it will introduce 1dB degradation. The proposal for Tx OFF power is as follows:

NOTE:	In 1.28 Mcps TDD option, The noise figure of BS is assumed to be 7dB, it is  based on that this parameter has been approved by CWTS yet and it is easier for implementation of BS.

For E-UTRA the power level is in 1MHz (rather than 1.28MHz) and the assumed NR is 5dB the requirement is hence:

And for E-UTRA (specified as -85dBm/MHz)

There are a number of different receivers which can be identified as being victims to the Tx OFF power:

-	Own receiver – this is possible but, it is unlikely a TDD system has separate Tx and Rx antennas so is identified by the RX sensitivity requirement even for a conducted system.

-	Co-located receivers – the requirement is measured for the Tx channel – however the same noise can be expected across the whole operational band. Other co-located receivers using other channels in the band are hence protected.

-	Same geographical area receivers – most co-location requirements in the same geographical area assume a reasonable separation between BS’s, hence the isolation is assumed to be greater.

For the release 13 AAS BS requirements the TX OFF level was applied to each TAB connector as the primary purpose of the requirement is to prevent desensitization of your own receiver. With a conducted interface the receiver sensitivity requirement alone is not sufficient to guarantee that, this is further documented in TR 37.842 [4].

### 5.4.2	OTA AAS BS – Core requirement

The OTA AAS BS has both radiated minimum sensitivity and radiated reference sensitivity requirements, hence any interference to your own receiver is fully covered by those requirements.

TX OFF level therefore ensures that co-located BS and BS co-existing in the same geographical area are protected. As the very low TX OFF levels cannot be measured accurately as a TRP the co-location method is used. The OTA TX OFF requirement is therefore specified a co-location requirement.

It is not possible to separate OTA emissions from separate transceiver units therefore OTA emissions requirements are specified as the sum of the emissions from all transceiver units in the AAS BS. For spurious emissions this has been the case for both conducted and OTA emissions. The sum of the total emissions from the AAS BS is intended to be no greater than that of a non-AAS system with equivalent number of MIMO branches and hence emission are capped at a level consistent with the maximum MIMO capability of the appropriate RAT (8 for E-UTRA, 4 for UTRA), however conducted TX OFF level is specified per TAB connector and as such is not subject to the same cap. However as the conducted requirement is primarily concerned with protection of your own receiver and OTA is concerned with co-location the parameters have somewhat changed, this was noted in the TR 37.842 when the conducted requirements were derived.

As a co-located TDD receiver should be offered the same protection as any other co-located receiver it is reasonable to apply the same cap on the TX OFF level as is applied to other unwanted emissions.

The TX OFF power level, from both polarisations, is less than:

Note: there is no requirement for UTRA TDD OTA AAS BS.

## 5.5	Transmitted signal quality

### 5.5.1	General

Unless otherwise stated, the requirements in sub-clause 5.5 apply during the transmitter(s) ON period.

### 5.5.2	Frequency error

#### 5.5.2.1	Background information on the conducted requirement

The frequency error requirement is defined to capture the maximum allowable difference between an assigned frequency and the actual generated frequency. The frequency error requirement is a regulatory requirement in some regions.

#### 5.5.2.2	Frequency error OTA: Core requirement

In order to meet the beam forming requirements of the AAS BS specifically EIRP accuracy, it is necessary that the transceiver units generating the beam have a known phase relationship between each other. To do this they need to be phase locked. There are two alternative ways to achieve this:

-	The RF LO originates from a common source and is distributed to each transceiver unit, or

-	The RF LO originates locally in each of the TRX units, where individual PLL’s use the same frequency reference, i.e. each individual RF source is locked to a common reference frequency.

In both cases, any frequency error is correlated among all TRX units forming the beam, hence the frequency error is coherent, has a ‘flat’ response in the spatial domain, i.e. OTA frequency error does not depend on the selection of the measurement point within beam’s compliance directions set.

As the intention of the eAAS WI is to provide the same performance and protection as the Rel-13 conducted AAS BS specification, the core requirement for the OTA frequency error requirement is reused from conducted frequency error requirement from TS 37.105 [3] specification, i.e. for the OTA frequency error requirement, the same frequency error values as defined for the conducted frequency error requirement in Rel-13 AAS BS specification is used.

Table 5.5.2.2-1: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

#### 5.5.2.3	Frequency error OTA: Conformance requirement

As the frequency error is flat in the spatial domain it is only necessary to show conformance in a single direction. The requirement needs to be defined so that all transmitter units are active and the system is operating at the declared maximum rated total radiated power.

For the sake of minimising the number of spatial declarations and as frequency error testing is generally done at the same time as EVM testing the ‘reference direction’ of the OTA peak directions set is the most suitable direction to define for the conformance testing. For conformance testing purposes, the frequency error is tested at the maximum and minimum power settings (together with the EVM test), in order to comply with the total power dynamic range requirement (refer to the Total power dynamic range requirement for E-UTRA in section 5.3).

### 5.5.3	Time Alignment Error

# 5.5.3.1	Background information on the conducted requirement

Timing Alignment Error (TAE) is a signal quality requirement specified for transmit diversity, MIMO transmission and CA. TAE is defined as the largest timing difference (i.e. error) between two reference signals, and is only applicable for base stations transmitting from multiple antennas via transmit diversity, MIMO, CA, or some combinations of these three. In case of NR, there is no TX diversity transmission mode defined. As a result, no TAE tests need to be developed for TX diversity for NR operation.

In the conducted test setup, the differences between two antenna ports is measured, depending of the functionality of the signal analyser, TAE between the two or more signals can be measured simultaneously. To achieve precise measurements, the RF cables being used during the conducted test should be equal in electrical length.

For AAS BS the number of TAB connectors could be large. Therefore, a concept for reducing number of test combinations was introduced in TS 37.105, sub-clause 6.5.3.4 [3]. In TAE requirement for AAS BS is designed as:

The TAE between any two TAB connectors from different transmitter groups shall not exceed the specified minimum requirements below.

, where transmitter groups are associated with the TAB connectors in the transceiver unit array corresponding to TX diversity, MIMO transmission, CA, etc.

# 5.5.3.2	Time Alignment Error OTA: Core requirement

For OTA TAE, the DUT is placed in an OTA test environment and is configured to generate beams carrying reference signals (e.g. CRS0 and CRS1 in case of E-UTRA, DMRS ports in case of NR) associated to logical antenna ports and different transmitter groups. The OTA TAE requirement is defined as a time difference between two radiated signals.

# 5.5.3.3	Time Alignment Error OTA: Conformance requirement

The conformance testing of OTA TAE can be done in an OTA test environment similar to the one defined for radiated transmit power. In terms of testing effort it is beneficial, to coordinate testing of OTA TAE with testing of other transmitter parameters such as OTA frequency error and radiated transmit power.

### 5.5.4	Modulation Quality

#### 5.5.4.1	Background information on the conducted requirement

EVM captures a maximum allowed distortion allowed due to degradations in the BS transmitter.

For UTRA, EVM is defined as:

Where Pe is the power of the error signal between a perfect RRC filtered transmitted signal without distortion and the actual transmitted signal and Pt is the power of the perfect transmitted signal without distortion.

For E-UTRA, EVM has been defined slightly differently the measurement device contains a simple equalization algorithm and the EVM is assessed after equalization (The simple equalization algorithm accounts for the fact that some static amplitude and phase deviations incurred in the BS transmitter is removed at the UE receiver). The EVM result is defined as the square root of the ratio of the mean error vector power to the mean reference power expressed in percent.

EVM is calculated as a root mean square average over 10 sub-frames.

For AAS BS in Rel-13 TS 37.105 [3], the MSR, E-UTRA and UTRA, EVM requirements are retained and applied per TAB connector.

The conducted EVM requirement applied per TAB connector needs to be translated to an OTA requirement.

Although EVM is expressed in terms of a percentage it is a ratio of 2 powers, that of the error vector and the reference power. As with ACLR whilst the reference (or wanted) signal may be subjected to beam forming the error vector power has unknown correlation level between transmitter units and hence may be beam formed or not.

If the error vector power is correlated then the EVM is constant over the OTA angular range, however if it is not correlated then it varies, as shown in figure 5.5.4.1-1.

Figure 5.5.4.1-1. 10 Element array, beam pattern and non-correlated EVM distribution (8% EVM)

#### 5.5.4.2	Modulation Quality OTA: Core requirement

The EVM requirement however is important only between the BS and its intended UE. Hence the EVM is only of importance in directions where the BS intends to communicate with the UE’s. It is therefore in a set of specific directions from the AAS BS which is of interest rather than the average or total EVM over the entire sphere. In this respect the EVM requirement is different in nature to the ACLR requirement.

The range of directions which the AAS BS meets the EVM requirement is declared by the manufacturer as OTA coverage range.

Figure 5.5.4.2-1. Example of OTA coverage range and 5 conformance points

The EVM is required to be met across the OTA coverage range.

#### 5.5.4.3	Modulation Quality OTA: Conformance requirement

For conformance testing the centre direction and the 4 extreme directions in θ and ϕ of the OTA coverage range, as shown in figure 5.5.4.2-1 are declared and tested.

For conformance testing purposes, the EVM is tested at the maximum and minimum power settings, in order to comply with the total power dynamic range requirement (refer to the Total power dynamic range requirement for E-UTRA in section 5.3).

### 5.5.5	Transmit pulse shape filter

#### 5.5.5.1	Background information on the conducted requirement

The transmit pulse shape filter for UTRA is defined as root-raised cosine (RRC) filter with the assumed roll-off factor. There is no minimum requirement directly defined for the transmit pulse shape filter in RAN4 specifications. Furthermore, there is no conformance requirement defined for the transmit pulse shape filter.

Transmit pulse shape filter requirement is applicable only for the AAS BS in single RAT UTRA operation (TDD, or FDD), or for AAS BS in MSR operation using UTRA (TDD, or FDD).

There is no transmit pulse shape filter requirement for an E-UTRA AAS BS.

#### 5.5.5.2	Transmit pulse shape filter OTA: Core requirement

For the transmit pulse shape filter OTA requirement, use the same approach as for the conducted transmit pulse shape filter requirement, by referring to the TS 25.104 [19] and TS 25.105 [20] specifications for UTRA FDD and UTRA TDD, respectively.

#### 5.5.5.3	Transmit pulse shape filter OTA: Conformance requirement

No conformance requirement to be defined for the transmit pulse shape filter OTA requirement.

## 5.6	Unwanted emissions

### 5.6.1	General

#### 5.6.1.1	Background information on the conducted requirement

In non-AAS BS, the unwanted emissions are specified in TS 25.104 [19], TS 25.105 [20], TS 36.104 [8] and TS 37.104 [7] and apply per antenna connector. The AAS BS has a TRXUA consisting of many transceiver units which is equal to, or potentially much higher than the number of non-AAS BS transmitter antenna connectors in a non-AAS BS with comparable functionality. As there is no upper limit on the number of transceiver units that an AAS BS system has, it is important to set the total unwanted emissions limits to an amount equivalent to Rel-13 non-AAS BS.

The total unwanted emissions from the AAS BS should be no higher than the unwanted emissions from a non-AAS BS conforming to the Rel-13 specifications with either the same number of transmitter units as the AAS BS or a maximum of 8 transmitter units (E-UTRA) or 4 transmitter units (UTRA).

#### 5.6.1.2	OTA unwanted emissions

The Rel-13 AAS BS specification which defined a hybrid AAS BS with a conducted interface applied a scaling factor which was based on the number of active transmitter units as counted at the transceiver array boundary. As the OTA AAS BS architecture is restricted to having a minimum number of transceiver units (8 for E-UTRA, 4 for UTRA) there is no need for a scaling variable as a fixed scaling factor (FSF) can be used based on the RAT-specific unwanted emissions requirements.

In Rel-13 requirements the emissions scaling was also multiplied by the minimum number of cells the AAS BS was declared to support (Ncells). This is necessary as if the system supports multiple cells at the conducted interface it is not always possible to directly map TAB connectors to cells. With an OTA AAS BS there is no conducted interface and requirements are applied radiated so no such mapping knowledge is required. The complication of scaling by the minimum number of cells therefore can also be removed as the requirement applies OTA per cell. It should be noted that the restriction on the minimum number of transceiver units should also apply per cell.

The OTA AAS BS emissions limits for E-UTRA therefore are applied per cell and are based on the basic limits used in the Rel-13 AAS BS requirements multiplied by the FSF equal to 8 (or plus 9dB).

The OTA AAS BS emissions limits for UTRA therefore are applied per cell and are based on the basic limits used in the Rel-13 AAS BS requirements multiplied by the FSF equal to 4 (or plus 6dB).

Some regional regulatory requirements for unwanted emissions are expressed as EIRP in the regulations. They implicitly assume a passive antenna. Since the gain between passive antenna systems does not vary significantly (i.e. only by a few dB, not 10s of dB), EIRP correlates directly to TRP in these cases.

For MSR, requirements are applied at an antenna connector. In order to transform the EIRP level stated in the regulations to a conducted level, vendors make a declaration of the passive antenna gain they assume.

In Rel-13 AAS these emissions requirements remain conducted. A declaration of the assumed passive antenna gain to relate the EIRP to a conducted level for the AAS is needed. It is important to note that what should be declared is the gain of an equivalent passive system.

OTA unwanted emissions requirements are defined as TRP. A means is needed to relate the EIRP values in the regulation to TRP. It is important to note that the EIRP does not correspond to the instantaneous EIRP of any beamforming scheme in the AAS, but was set based on the assumption of a passive antenna. In order to relate the EIRP values to OTA, a fixed assumption is made of the gain of a passive antenna system, in order that an OTA TRP requirement can be provided for AAS BS.

Since EIRP per antenna:  PEIRP = PTx + GAnt with GAnt equal to [17] dBi for both E-UTRA and UTRA.  To interpret existing EIRP regulations to TRP would be the follow:

UTRA: OTA AAS BS emissions limits = EIRP – [17] dBi + 6 dB

E-UTRA: OTA AAS BS emissions limits = EIRP – [17] dBi + 9 dB

In the above equations directivity value should be used in above equations, however as all antenna losses are assumed zero effective antenna gain can be used.

Since a fixed passive gain is assumed, the TRP limits can be stated directly as dBm numbers in the specifications.

### 5.6.2	Occupied bandwidth

#### 5.6.2.1	Background information on the conducted requirement

For UTRA and E-UTRA, if the ACLR requirements are met, then usually it is not possible for the system to fail the occupied bandwidth requirement, however the requirement still applies since some regulatory authorities expect an occupied bandwidth requirement for all systems.

According to SM.328 [9] occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. It defines the spectral properties of emission in a simple manner. The value of /2 is taken as 0.5%. This results in a power bandwidth of 99%, meaning that the occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated mean power of the transmitted spectrum on the assigned channel.

#### 5.6.2.2	Occupied bandwidth OTA: Core requirement

For occupied bandwidth, the beam characteristics are not important. The requirement should however cover the fact that all transmitter is active and the system is operating at the maximum declared rated total radiated power.

For the OTA requirement, the same occupied bandwidth value (i.e. /2= 0.5%) as for the conducted occupied bandwidth requirement for Rel-13 AAS BS is to be used.

As the occupied bandwidth is a measure of the wanted signal its performance is flat in space and can be completely characterized as a directional requirement or a TRP requirement. As performance of the wanted signal is generally characterized on the main beam as a directional requirement the same is done for occupied bandwidth.

Occupied bandwidth is therefore specified as a directional requirement valid over the OTA coverage range.

#### 5.6.2.3	Occupied bandwidth OTA: Conformance requirement

For conformance a beam should be identified which fulfils the core conditions.

### 5.6.3	Adjacent Channel Leakage Radio

#### 5.6.3.1	Background information on the conducted requirement

The Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency, expressed in dB. ACLR is defined for both the first adjacent channel and the second adjacent channel.

The Wide Area BS ACLR was originally derived by means of co-existence simulations as documented in the TR 36.942 [5]. The coexistence simulations captured DL network throughput loss due to interference from an aggressor network in the downlink vs. so-called ACIR. ACIR incorporates power in the UE receive band due to the transmitter unwanted emissions (ACLR) and the receiver (ACS), and is calculated as:

(assuming linear terms)

The coexistence simulation result for ACIR was split into ACLR and ACS, resulting in for wide area BS ACLR of 45dB and a UE ACS of 33dB.

An AAS BS may consist of multiple transmitters that perform beamforming. If the adjacent channel interference is not necessarily beam formed in the same manner as the wanted channel, hence ACLR may vary in space. Before setting a conducted requirement in the Release 13 AAS BS specification TS 37.105 [3], further co-existence simulations were performed in the AAS SI TR 37.840 [6] and AAS WI TR 37.842 [4]. These simulations revealed that it is the total power and not the spatial distribution of unwanted emissions (i.e. the correlation of adjacent channel noise between transmitter units) that impacts co-existence KPIs. Hence an ACLR based on total wanted channel and adjacent channel power was defined at TAB connectors.

In addition to the ACLR ratio of 45 dB, an absolute level of adjacent channel emissions is defined. The base station fulfils whichever is the less stringent of the ACLR ratio or the absolute level. The definition of an absolute level avoids specifying unnecessarily low levels on any BS that operate with a low wanted carrier transmit power. For wide area BS, the absolute level is defined in TS 37.104 [7] as either -13 dBm/MHz for category A, or -15dBm/MHz for category B emissions requirements, relating to regulatory frameworks.

In the AAS BS specification TS 37.105 [3] the ACLR requirement is set on the ratio of the total wanted signal power at all TAB connectors to the total power at all connectors of adjacent channel power. For the absolute requirement, the non-AAS requirement forms a basic limit, which is scaled up to an AAS BS requirement according to the number of active transceiver units, up to a maximum 8 times for E-UTRA (4 for UTRA), in the same manner as the scaling applied to other unwanted emissions requirements.

For medium range BS and local area BS, the ACLR requirement of 45 dB was retained in TS 37.104 [7] without any further simulation investigations. For TS 37.105 [3], the 45 dB ACLR was also retained for the other base station classes.

In the non-AAS specifications, the absolute limits for adjacent channel emissions are adjusted for the medium range and local area BS classes. For the local area BS class, the absolute limit are adjusted to -32 dBm/MHz. The limit of -32 dBm/MHz corresponds to an ACLR of roughly 45 dB for a local area BS with a 10 MHz carrier operating at the maximum permitted output power. For the medium range BS, the absolute limit are also adjusted. The absolute limit is set to -25 dBm/MHz. This again roughly corresponds to an ACLR of 45 dB for a medium range BS transmitting a 10MHz carrier near to maximum allowable output power, and also matches to the WCDMA SEM.

The absolute unwanted emissions requirements for medium range BS and local area BS were also used as basic limits in TS 37.105 [3], with the scaling mechanism applying in the same way as for wide area BS.

In addition to ACLR, a requirement on so-called Cumulative ACLR (CACLR) is applied for multicarrier and multiband BS. The CACLR requirement considers adjacent channel emissions falling onto a carrier that is adjacent to 2 active carriers; one at a lower frequency offset and one a higher frequency offset. The CACLR requirement is also 45 dB.

The CACLR requirements also apply in TS 37.105 [3] in the same manner as ACLR.

#### 5.6.3.2	ACLR OTA: Core requirement

Based on the findings of the simulation work done in TR 37.842 [4] the effect of adjacent channel interference on the throughput of adjacent networks is dependent on the total radiated adjacent channel power rather than the power in the main beam.

As the number of active transceiver units generating the beam and the coherence level of the adjacent channel noise from each active transceiver unit is unknown it is not possible to derive a relationship between the ACLR of a single point in the centre of the main beam (i.e. as ratio of 2 EIRP values) and the ACLR of the total radiated power.

The ACLR requirement is hence based on a ratio of the total radiated filtered mean power centred on the assigned channel frequency to the total radiated filtered mean power centred on an adjacent channel frequency:

.

The OTA ACLR requirement can be derived as:

OTA ACLR limit = Conducted ACLR limit + 10*log10(FSF)

Where:

-	Conducted ACLR limit: the AAS limit as defined in TS 37.105 [3] specification,

-	FSF: Fixed Scaling Factors for the OTA AAS BS. For UTRA the FSF = 4, for E-UTRA the FSF = 8,

Therefore the resulting OTA ACLR limit can be derived as:

-	UTRA: OTA ACLR limit = Conducted ACLR limit + 6 dB

-	E-UTRA: OTA ACLR limit = Conducted ACLR limit + 9 dB

#### 5.6.3.3	ACLR OTA: Conformance requirement

In order that the requirement is measurable the total radiated power may be defined as the sum of the EIRP on a grid of discrete directions around the sphere.The ACLR is calculated as described in sub-clause 10.8.

To properly capture the EIRP, two orthogonal polarizations must be summed together.

The points required for conformance of the ACLR OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion in sub-clause 10.8.

### 5.6.4	Spectrum emission mask

#### 5.6.4.1	Background information on the conducted requirement

Based on the conducted spectrum emission mask (SEM) requirement specified in TS 37.105 [3] for single RAT UTRA operation, the SEM limits are defined based on the basic limit definition over the frequency range starting with the carrier frequency offset of 2.5 MHz up to the fmax as defined in TS 37.105 [3].

There is no spectrum emission mask requirement for AAS BS in single RAT E-UTRA operation, nor for AAS BS in MSR operation using E-UTRA.

#### 5.6.4.2	Spectrum emission mask OTA: Core requirement

The metric used to capture SEM OTA is TRP.

The spectrum emission mask level for the OTA requirement can be derived as:

OTA SEM limit = Conducted SEM limit  + 10*log10(FSF)

Where:

-	Conducted SEM limit: the AAS limit as defined in TS 37.105 [3] specification,

-	FSF: Fixed Scaling Factors for the OTA AAS BS. For UTRA the FSF = 4,

Therefore the resulting OTA SEM limit can be derived as:

OTA SEM limit = Conducted SEM limit + 6 dB

#### 5.6.4.3	Spectrum emission mask OTA: Conformance requirement

Using TRP as a figure of merit to provide equivalent OTA SEM performance and protection to that of the existing conducted requirement, the conducted requirements can be adopted by translating the test procedure and requirement from TS 37.145-1 [23], sub clause 6.6.4 into the spatial domain. The TRP assessment grids and methods are described in sub-clause 10.8.

The points required for conformance of the SEM OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion in sub-clause 10.8.

### 5.6.5	Operating band unwanted emission

#### 5.6.5.1	Background information on the conducted requirement

Based on the conducted operating band unwanted emission (OBUE) requirement specified in TS 37.105 [3] for single RAT E-UTRA or for MSR operation, the OBUE limits are defined from 10 MHz below the lowest frequency of each supported downlink operating band to the lower Base Station RF Bandwidth edge located at FBW RF,low and from the upper Base Station RF Bandwidth edge located at FBW RF,high  up to 10 MHz above the highest frequency of each supported downlink operating band.

There is no operating band unwanted emission requirement for a single RAT UTRA FDD AAS BS, single RAT UTRA TDD AAS BS, nor MSR AAS BS operating in UTRA (FDD or TDD).

#### 5.6.5.2	Operating band unwanted emission OTA: Core requirement

The operating band unwanted emissions level for the OTA requirement can be derived as:

OTA OBUE limit = Conducted OBUE limit

, where conducted OBUE limit is the AAS limit as defined in TS 37.105 [3] specification.

Based on the discussion on the boundary between in-band and out of band region for NR FR1 operation, the ΔfUEM values for the boundary between OBUE and TX spurious emissions were agreed for AAS BS as presented in table 5.6.5.2-1. Those ΔfUEM values are applicable to conducted and radiated requirements for AAS BS in single RAT E-UTRA operation and to AAS BS in MSR operation using E-UTRA, and are applicable for all E-UTRA operating bands defined in TS 36.104 [7].

Table 5.6.5.2-1: ΔfUEM values for the boundary between OBUE and TX spurious emission regions

|  |  | E-UTRA AAS BS (Single-RAT, MSR) |
| --- | --- | --- |
| ΔfUEM | Operating band <100 MHz | 10 MHz |
|  | 100 MHz ≦ Operating band ≦ 200 MHz | 40 MHz |

#### 5.6.5.3	Operating band unwanted emission OTA: Conformance requirement

Using TRP as a figure of merit to provide equivalent OBUE performance and protection to that of the existing conducted requirement, the conducted requirements can be adopted by translating the test procedure and requirement from TS 37.145-1 [23], sub-clause 6.6.5 into the spatial domain. The TRP assessment grids and methods are described in sub-clause 10.8.

Thepoints required for conformance of the OBUE OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion in sub-clause 10.8.

### 5.6.6	Transmitter spurious emissions

#### 5.6.6.1	Mandatory Spurious emission requirements

##### 5.6.6.1.1	Background information on the conducted requirement

According to TS 37.105 [3], the transmitter spurious emission limits for the hybrid AAS BS operating bands apply in the frequency range from 9 kHz to 12.75 GHz, with consideration of the following exceptions:

-	Unwanted emissions: the spurious requirements apply at frequencies within the above specified frequency range with the exception of the UTRA spectrum emission mask (SEM) and E-UTRA operating band unwanted emissions (OBUE) ranges as specified below:

-	UTRA TDD BS 1.28 Mcps option as specified in TS 25.105 [20]: from 4 MHz below the lowest frequency of each operating band to 4 MHz above the highest frequency of each operating band.

-	UTRA TDD BS 3.84 Mcps option as specified in TS 25.105 [20]: from more than 12.5 MHz under the lowest carrier frequency used or more than 12.5 MHz above the highest carrier frequency used.

-	UTRA TDD BS 7.68 Mcps option as specified in TS 25.105 [20]: from more than 25 MHz under the lowest carrier frequency used or more than 25 MHz above the highest carrier frequency used.

-	UTRA FDD BS as specified in TS 25.104 [19]: from 12.5MHz below the lowest carrier frequency used up to 12.5MHz above the highest carrier frequency used.

-	E-UTRA BS as specified in TS 36.104 [8]: from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band.

-	MSR BS as specified in TS 37.104 [7]: from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band.

-	Upper frequency limit of the spurious range: in order to comply with the SM.329 recommendation [18], in case of spurious emissions requirement for certain operating bands, the upper limit of the spurious range is extended beyond 12.75GHz to the limit of 5th harmonic (but not higher than 26GHz). For more details on the upper limit setting in those cases, refer to SM.329 [18].

For AAS BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, this exclusion applies for each supported operating band.

##### 5.6.6.1.2	Transmitter spurious emissions OTA: Core requirement

The metric used to capture transmitter spurious emissions OTA is TRP.

The emission levels and the measurement bandwidths for the transmitter spurious emissions OTA requirement are based on the conducted requirements specified in TS 37.105 [3].

In case of OTA AAS BS requirement, emission levels are subject to fixed scaling factor (FSF), as described in sub-clause 5.6.1.2. The Tx spurious emissions level for the OTA requirement can be derived as:

OTA Tx spurious emissions limit = Conducted Tx spurious emissions limit + 10*log10(FSF)

Where:

-	Conducted Tx spurious emissions limit: the AAS limit as defined in TS 37.105 [3] specification,

-	FSF: Fixed Scaling Factors for the OTA AAS BS. For UTRA the FSF = 4, for E-UTRA the FSF = 8,

Therefore the resulting OTA Tx spurious emissions limit can be derived as:

UTRA: OTA Tx spurious emissions limit = Conducted Tx spurious emissions limit + 6 dB

E-UTRA: OTA Tx spurious emissions limit = Conducted Tx spurious emissions limit + 9 dB

The core requirement for the transmitter spurious emissions OTA is applied over 30 MHz to 12.75 GHz range, with consideration of the SEM and OBUE exceptions, and the operating band specific upper limit extensions of the spurious range,

Based on the discussion on the boundary between in-band and out of band region for NR FR1 operation, the ΔfUEM values for the boundary between OBUE and TX spurious emissions were agreed for AAS BS as presented in table 5.6.5.2-1. Those ΔfUEM values are applicable to conducted and radiated requirements for AAS BS in single RAT E-UTRA operation and to AAS BS in MSR operation using E-UTRA, and are applicable for all E-UTRA operating bands, except Band 46.

Where requirements for co-existence with other systems apply, the metric is TRP and the OTA requirement is based on the conducted requirements specified in TS 37.105 [3] for co-existence, subjected to fixed scaling as described in sub-clause 5.6.1.2. TRP has been identified as the most suitable metric for spurious emissions. This is due to the fact that decorrelation between emissions of different transmitter units, different antenna response in the spurious domain and time varying beamforming averages out the spatial pattern of the unwanted emissions. Coexistence studies indicate that degradation to other systems is generally related to the total amount of radiated interference. Furthermore the existing conducted requirement captures TRP.

##### 5.6.6.1.3	Transmitter spurious emissions OTA: Conformance requirement

In order that the requirement is measurable the total radiated power as defined in sub-clause 5.1.1 may be approximated as the sum of the EIRP on a grid of discrete directions around the sphere as described in sub-clause 10.8.

For the out-of-band region where the far field cannot be easily achieved other approximations based on power density should be considered,. Other measurement techniques are not precluded. For the upper part of the spurious region, the path-loss in the test chamber needs to be conserved, therefore power density should be considered.

An approximation of TRP measurement can be achieved by sampling discrete EIRP measurements surrounding the AAS BS.  The use of discrete sampling grid for the TRP approximation is motivated by the observation that in the spurious domain there is no coherent beam forming, as the AAS BS are designed to form beams based on superposition of signals from individual transceivers in TRXU array in their in-band frequency ranges, while in the spurious domain those signals are expected to be sufficiently uncorrelated. The points required for conformance of the SEM OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion in sub-clause 10.8.

Conformance requirement for the TX spurious requirement OTA is composed as a single requirement together with the EMC RE requirement OTA, as there is no possibility to separate those requirements in OTA test setup. The resulting test requirement will be subject to single test setup and single measurement procedure. Definition of the conformance requirement will be subject to TRP MU, where the MU definition is FFS. For more details on the EMC RE requirement, refer to clause 8.

{editors note: to be completed as part of conformance work}

#### 5.6.6.2	Co-location requirements

##### 5.6.6.2.1	Background

Spurious emission co-location requirements are applied for the protection of other BS receivers co-located with another BS.

The conducted requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The levels are specified so that the co-located victim BS receiver sensitivity does not degrade when co-located.

For example an E-UTRA wide area BS has a sensitivity of -101.5dBm for 5 MHz channel bandwidth.

The co-location noise requirement level is -96dBm/100kHz so this would cause a sensitivity degradation of:

Or a 0.6dB desensitization.

##### 5.6.6.2.2	Core requirements

The OTA spurious emission co-location requirements are specified as a co-location requirement as described in sub-clause 5.1.2.

The output of the co-location reference antenna is therefore the conducted requirement level minus the 30dB co-location isolation assumption.

The conducted spurious emission co-location requirements are specified in non-AAS per antenna connector and as a sum of all the power at the transceiver array boundary in the hybrid AAS requirements. As such the level specified in the non-AAS requirements is subject to scaling when applied to the AAS system requirements. As the OTA AAS BS architecture is limited to systems with 8 or more transceiver units (4 for UTRA) a fixed scaling factor is added to the non-AAS limit.

As the requirement is based on the total output from multiple transceivers if the AAS BS is radiating on two orthogonal polarisations then the power level out of the co-location reference antenna should be the sum of both polarisations.

The total power level, from both polarizations, at the output of the co-location reference antenna and is:

where X = 9dB for E-UTRA and X=6dB for UTRA

##### 5.6.6.2.3	Conformance requirements

During conformance testing the co-location reference antenna may be replaced with a measurement antenna with a known performance. The core requirement level is translated to a conformance requirement level considering the difference in OTA performance between the co-location reference antenna and the measurement antenna.

It is expected that as the measurement antenna has higher gain in the direction of the DUT than the co-location reference antenna the conformance power level is higher than the core level. This makes measurement more practical as the levels are close to the thermal noise floor.

The translation from the co-location reference antenna to the measurement antenna are FFS.

As the translation from the co-location reference antenna to a measurement antenna with higher gain requires detailed analysis and experimentation, it is probably that initial measurements are done with test antennas which are co-location reference antenna as described in subclause 5.1.2.

In sub-clause 5.1.2 the co-location reference antenna is described as a passive single column BS antenna suitable for the specified frequency range. As such the signal level measured during the test will be the same as the core requirement, which is very low:

-	OTA Transmit ON/OFF power (E-UTRA only),	-106 dBm/MHz

-	Protection of the BS receiver of own or different BS ,	-119dBm/100kHz (E-UTRA BC2), -120 dBm/100kHz (UTRA)

-	Co-location with other BS, -122dBm/100kHz (WA, GSM bands),  -120dBm/100kHz (WA UTRA and E-UTRA bands)

The worst case is the GSM co-location requirement which is only 2dB above the thermal noise floor at -124 dBm/100kHz.

As the emission received at the measurement receiver PUEM for co-location requirements are very low, it is suggested to measure the noise change. The relations between measured noise change 1, noise floor N0 and the relation to PUEM with respect to the noise floor denoted 2 is visualized in linear scale in Figure 5.6.6.2.3-1.

Figure 5.6.6.2.3-1: Relative noise measurement

The absolute emission level in decibel scale is calculated as:

, where N0 is the noise floor of the measurement receiver and 2 is found in Figure 5.6.6.2.3-2.

Figure 5.6.6.2.3-2: Measured noise change to relative noise level

The absolute noise floor of the measurement receiver, including probe antenna, cables, filter and LNA is determined by a calibration procedure. The calibration will determine the absolute emission level (N0) accuracy of measuring out-of-band unwanted emission close to the thermal noise floor.

## 5.7	Transmitter intermodulation

### 5.7.1	General

The transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array. The requirement applies during the transmitter ON period and the transmitter transient period. The interfering signal level is derived based on the co-location assumption that the co-located BS is of the same class and output power level as the BS under test. The OTA TX IMD interferer level can hence be regarded as a co-location requirement.

### 5.7.2	Background information on the conducted requirement

In Rel-13 AAS base station specification, TS 37.105 [3], sub-clause 6.7.1 there are two types of transmitter intermodulation cases captured by the transmitter intermodulation requirement:

1.	Co-location transmitter intermodulation in which the interfering signal is from a co-located base station.

2.	Intra-system transmitter intermodulation in which the interfering signal is from other transmitter units within the AAS BS.

# 5.7.3	Transmitter intermodulation OTA: Core requirement

The minimum requirement for the transmitter intermodulation requirement is that the emissions output level does not exceed the unwanted emission limits specified for operating band unwanted emission in sub-clause 5.6.5.1, transmitter spurious emission in sub-clause 5.6.6.2, and ACLR in sub-clause 5.6.3.2 in the presence of a wanted signal and an interfering signal in an OTA environment.

For the conducted requirement the transmitter spurious emissions requirement includes the mandatory emissions requirements, protection of own or different BS , additional spurious emission requirements (co-existence in same geographical area) and co-location requirements. As the protection of own or different BS and the co-location requirements are specified as an output of the co-location reference antenna and the TX IMD requirement uses the co-location reference antenna as an input these emissions requirements are not included as part of the OTA TX IMD requirement.

The interfering signal level is the same power level as the AAS BS (PRated,t,TRP) fed into a co-location reference antenna.

If the AAS BS has dual polarised antennas then the co-location reference antenna also needs to have dual polarised antennas and PRated,t,TRP / 2 is input to each polarisation.

When OTA ACLR, OTA OBUE and OTA spurious emission is defined, the relevance for having a separate intra-system requirement disappears. The intra-system case is covered by OTA unwanted emission requirement including the unwanted emission for the whole system, hence a separate intra-system IMD requirement is not needed when considering OTA.

# 5.7.4	Transmitter intermodulation OTA: Conformance requirement

[Text to be added]

# 6	Radiated receiver characteristics

## 6.1	General

### 6.1.1	Spatial requirements

Overview of the radiated Rx requirements of the AAS BS is presented in table 6.1.1-1.

Table 6.1.1-1: Overview of radiated Rx requirements

| AAS BS requirement | OTA requirement type | Applicability levels | Coverage range | Number of conformance directions |
| --- | --- | --- | --- | --- |
| OTA sensitivity | Single direction | - | Receiver target redirection range (D10.8) | 5 |
| OTA reference sensitivity | Single direction | OTA REFSENS | OTA REFSENS RoAoA | 5 |
| Dynamic range | Single direction | OTA REFSENS | OTA REFSENS RoAoA | 1 |
| In-band selectivity and blocking | Single direction | OTA REFSENS and minSENS | OTA REFSENS RoAoA and minSENS RoAoA | 5 |
| ACS and narrowband blocking | Single direction | OTA REFSENS (NB blocking only) and minSENS | OTA REFSENS RoAoA (NB blocking only) minSENS RoAoA (NB blocking and ACS) | 5 (blocking) 1 (ACS) |
| Out-of-band blocking | Single direction | minSENS | minSENS RoAoA | 1 |
| Receiver spurious emissions | TRP | - | n/a | - |
| Receiver intermodulation | Single direction | OTA REFSENS and minSENS | OTA REFSENS RoAoA and minSENS RoAoA | 1 |
| In-channel selectivity | Single direction | minSENS | minSENS RoAoA | 1 |

Note: Single direction implies a single direction requirement, i.e. the requirement is applied in a single direction at a time, not that the requirement is only specified in 1 direction.

For conformance testing of the radiated RX requirements, FRCs defined in annex A of UTRA FDD specification in TS 25.104 [19], or E-UTRA specification in TS 36.104 [8] is reused, where applicable.

### 6.1.2	Receive loss factor assumption

The receiver loss factor is used to translate conducted power levels to OTA power levels..

The receiver loss factor accounting for antenna losses, distribution losses, integration losses etc. in the receiver path(s) for the in-band, out-of-band and spurious region is assumed to be 0dB.

For emissions the same reasoning as the transmitter that assuming the same conducted and radiated power level as assumed in ITU-R SM.329 [18].

### 6.1.3	OTA offsets

Most of the conducted receiver requirements (with the exception of receiver spurious emissions) are based on wanted and interferer power levels being applied to the antenna connector. The OTA requirements are based on these conducted levels and are based on an OTA power level from a single direction where the AoA is within the specified RoAoA.

As there are 2 OTA sensitivity requirements described in sub-clauses 6.2 there are effectively 2 antenna gain offsets which are used for each of the receiver requirements:

OTA sensitivity requirement is declared, a number of declarations may be made with different EIS values and different RoAoA and receiver target redirection range(s). The lowest declared EIS value from all the OSDD declarations is used to represent the system minimum sensitivity (minSENS).

If receiver requirements include the effect of the receiver noise figure then  the OTA levels should be based on minSENS or offset by ΔminSENS where,

ΔminSENS = REFSENSConducted – minSENS

OTA REFSENS is based on the OTA REFSENS RoAoA which is intended to estimate the element antenna gain, assuming a low gain results in higher OTA power levels. If the receiver requirements are intended include the effect of receiver non-linearity’s due to high signal levels the OTA levels should be based on OTA REFSENS or offset by ΔREFSENS. ΔREFSENS is derived in sub-clause 6.2.2.2.

In some cases where receiver performance includes both the effect of receiver noise figure and of non-linearity the requirements are specified using both offsets.

## 6.2	Reference sensitivity level

### 6.2.1	Background information on the conducted requirement

The conducted reference sensitivity serves two purposes. The first purpose is as a minimum RF performance requirement for sensitivity. The second purpose is as a reference level from which other RX requirements are defined.

The wide area sensitivity requirement has been derived based on estimation and assumption of the achievable noise figure in the BS receiver and the SNR requirement for the reference measurement channel.

For the medium range BS class, system simulations were conducted based on 2 network layers of which one was a macro layer and a second a micro layer. The micro sensitivity is set such that the macro layer is not significantly degraded due to large power UL transmissions in the micro layer, whilst the micro layer sensitivity is somewhat relaxed due to the fact that interference from the macro layer renders achieving the same sensitivity level not useful. Based on the simulations, a different sensitivity level than for the wide area BS class was decided.

For the local area BS class, system simulations and calculations of interference to the pico cells due to other network layers was calculated. The presences of potentially strong interference due to UEs transmitting to micros or macros means that a tight sensitivity for a pico cell does not make sense from a performance or cost perspective.

Thus in effect the wide area sensitivity is based on achievable performance, whilst the sensitivity for the other layers is based on interference conditions in a heterogeneous network.

### 6.2.2	OTA sensitivity requirements

#### 6.2.2.1	General

In the Rel-13 AAS BS specification TS 37.105 [3] there are 2 requirements related to receiver sensitivity:

-	Reference sensitivity part of the conducted receiver requirements; applies per TAB connector.

-	Sub-cause 10.2 OTA sensitivity: Part of the radiated receiver requirements; applies in the far field region over the declared OSDD.

The Rel-13 OTA sensitivity requirement currently is based on meeting a declared EIS value over a declared RoAoA. This declared requirement is maintained for Rel-15 AAS BS OTA sensitivity requirement. In addition, in order to provide a minimum EIS requirement level which can replace the conducted reference sensitivity level, additional minimum requirement is set based on the existing conducted minimum requirement in Rel-13.

As the declared RoAoA for the Rel-13 OTA sensitivity requirement is not bounded by specific requirements (such as the 3dB contour) it can capture a much larger RoAoA and hence demonstrate the spatial performance of the receiver including the antenna. One important characteristic which can be captured is the scan loss of the system due to large steering angles and antenna geometry.

The OTA reference sensitivity (OTA REFSENS) requirement is the OTA equivalent of the conducted reference sensitivity, its intention is not to specifically capture antenna performance but rather to estimate an equivalent passive antenna gain if it were being used in the same deployment scenario and show equivalence to the conducted requirement.

The OTA REFSENS requirement does not replace the existing OTA sensitivity requirement in sub-clause 9.2 of TS 37.105 [3] but it is an additional new OTA requirement.

As OTA reference sensitivity is used as a reference for many of the interference requirements where it is important that the interferer is polarization matched to the receiver antenna it is also important that the wanted signal is polarization matched. Hence OTA reference sensitivity is specifically declared per polarization and the EIS values are under the assumption of polarisation match.

#### 6.2.2.2	Core requirement

##### 6.2.2.2.1	OTA Sensitivity

The declared OTA sensitivity requirement is based upon declaration of one or more OSDD, following the concept defined for Rel-13 AAS specification TS 37.105 [3], sub-clause 10.2.

##### 6.2.2.2.2	OTA reference sensitivity

OTA REFSENS is specified over the RoAoA which define a contour equivalent to the 3dB beam width of a non-AAS passive antenna covering the same RoAoA. Using the 3dB contour allows for simple estimation of equivalent directivity by use of standard formula.

The OTA REFSENS level (EISREFSENS) depends on the equivalent directivity of a passive antenna pattern which has the same beam width(s) as the AAS element or sub-array (declared via OTA REFSENS RoAoA) and can be calculated in dBm as follows:

Where:

-	DRX_OTA_MARGIN is an implementation margin to allow for errors associated with beam forming in the UL. The DRX_OTA_MARGIN value is 1dB.

-	PREFSENS is the conducted reference sensitivity level in dBm in the Rel-13 TS 37.105 [3].

-	D0 is the estimated antenna peak directivity in dBi of a non-AAS BS, which has a beam pattern related to the AAS BS OTA REFSENS RoAoA region.  D0 is approximated by the Elliot’s formula and expressed as,

Where,

-	BeWθ,REFSENS is the beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees.

-	BeWφ,REFSENS is the beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees.OTA REFSENS RoAoA is declared by the vendor.

-	Doff-peak is the peak directivity off-peak margin, it is defined as follows:

-	The OTA REFSENS RoAoA is defined as the contour where the EIS is 3dB higher than in the reference direction, therefore by definition Doff-peak in the reference direction is 0dB. In all other directions within the OTA REFSENS RoAoA Doff-peak is 3dB.

Redirection capability is a spatial performance characteristic of the antenna and hence is part of the OTA sensitivity requirement. The OTA REFSENS requirement demonstrates the minimum sensitivity of the OTA AAS BS receiver and hence requires only a single OTA REFSENS RoAoA to be declared per operating band.

DRX_OTA_MARGIN allows for degradation in receiver performance due to beam forming aspects, including (but not excluded to):

-	Steering error

-	Correlation of noise sources in RX units.

-	Baseband combining efficiency/implementation margin

Steering error is primarily due to errors in phase alignment of the receiver units causing a steering error, this is similar to the beam steering error estimated for the EIRP accuracy value. The average EIRP beam steering  error component of the EIRP accuracy documented in TR 37.842 [4] was 0.85dB.

Correlation of noise sources between the receivers may occur due to mutual coupling of the antenna elements resulting in correlated noise being present in each of the receiver units, correlation of common noise sources such as LO’s. With a mutual coupling of 15dB it is estimated that the effect is approximately a 0.4dB degradation in system noise figure.

As the contributors are independent the rms sum of the factors is approximately 1dB, hence the value of DRX_OTA_MARGIN is1dB.

The offset between conducted reference sensitivity and OTA REFSENS (ΔREFSENS) is used for a number of the receiver requirements and can be defined as:

In the reference direction:

In all other directions in the OTA REFSENS RoAoA:

So OTA REFSENS can be written as:

#### 6.2.2.3	Conformance requirement

##### 6.2.2.3.1	OTA Sensitivity

Conformance testing for OTA sensitivity is performed for the five directions same as the Rel-13 OTA sensitivity requirements as described in sub-clause 7.3.3 in TR 37.842 [4]. Same requirements for Rel-13 OTA sensitivity apply.

The conformance directions in minSENS RoAoA can be defined as the following five directions.

-	The receiver target reference direction.

-	The direction determined by the maximum  value achievable inside the minSENS RoAoA maintaining the receiver target reference direction  value.

-	The direction determined by the minimum  value achievable inside the minSENS RoAoA maintaining the receiver target reference direction  value.

-	The direction determined by the maximum  value achievable inside the minSENS RoAoA maintaining the receiver target reference direction  value.

-	The direction determined by the minimum  value achievable inside the minSENS RoAoA maintaining the receiver target reference direction  value.

##### 6.2.2.3.2	OTA reference sensitivity

Conformance testing for OTA reference sensitivity is performed for the following five directions.

-	The receiver target reference direction.

-	The direction determined by the maximum  value achievable inside the OTA REFSENS RoAoA maintaining	the receiver target reference direction  value.

-	The direction determined by the minimum  value achievable inside the OTA REFSENS RoAoA maintaining	the receiver target reference direction  value.

-	The direction determined by the maximum  value achievable inside the OTA REFSENS RoAoA	maintaining the receiver target reference direction  value.

-	The direction determined by the minimum  value achievable inside the OTA REFSENS RoAoA maintaining	the receiver target reference direction  value.

The manufacturer declares the five directions for conformance testing.

The measured EIS should be less than or equal to the EISREFSENS + Test Tolerance value.

#### 6.2.2.4	Extreme conditions

The exiting core conducted requirements in TS 37.105 [3] do not explicitly identify an extreme conditions receiver requirement, however in the conformance requirement TS 37.145 [24, 25] conducted reference sensitivity has a conformance requirement over the extreme conditions. The same is true for the 25, 36 and 37 series non-AAS requirements.

It is not cleat the reason why the core requirement is not captured, however to keep the specifications as similar is possible the same approach is used in the OTA part of the AAS specification.

Extreme temperature requirements for sensitivity could be applied to either OTA sensitivity or to OTA REFSENS, however as OTA REFSENS is intended primarily as the reference performance level for the interference requirements and it is larger or equal to the OTA sensitivity level, the extreme requirements are best applied to OTA sensitivity rather than OAT REFSENS. By passing OTA sensitivity requirements the extreme performance of OTA REFSENS is also demonstrated.

As there may be multiple OSDD’s declared for OTA sensitivity (OTA REFSENS has a single OTA EFSENS RoAoA) and hence a greater number of conformance directions. Extreme condition conformance directions should be minimised and restricted to reference directions only.

## 6.3	Dynamic range

### 6.3.1	Background information on the conducted requirement

The dynamic range requirement is related to the maximum co-channel input power than the receiver is expected to experience during operation. The receiver is exposed to a wanted signal and an interfering signal. The interfering level is set at around 20 dB above the receiver noise floor (assuming 5 dB NF in the receiver) and represents an estimated inter-cell interference scenario in an interference limited uplink (that was derived through simulations). The wanted signal level is set to a level at which 95% throughput is obtainable for a 16QAM based FRC. In effect, the RF level is that expected when the receiver is achieving a high SINR despite a large interference level.

Part of the design of the requirement involves deciding on an estimate for a worst-case level of inter-cell interference that is experienced by the BS receiver. The existing simulations of inter-cell interference performed for the existing requirement are based on a fixed passive BS antenna.

### 6.3.2	Core requirement: Receiver dynamic range

Under the assumption that there is no spatial rejection of the interference, in order to provide a reasonable conformance test, it was agreed that the requirement assumes that the wanted signal and interfering signal come from the same direction.

It is important to consider with this requirement that both wanted signal and AWGN interferer is transmitted from the test equipment during the OTA test. Since the AWGN interferer is much higher than the receiver noise floor, the AWGN interferer dominates the SINR. Both RF and in baseband provides the same combining gain for both the wanted signal and the AWGN, since both are co-channel and come from the same direction.

Thus, there is no impact of combining gain to the requirement. For this reason, a 2-level requirement, such as is discussed for the receiver in band blocking requirement is not necessary for receiver dynamic range.

As the wanted signal and the AWGN interferer are both above the receiver noise floor the core requirement is based on OTA REFSENS assumptions and both the wanted and interferer conducted power levels are offset by ΔREFSENS.

### 6.3.3	Conformance requirement: Receiver dynamic range

Testing is defined in the receiver target reference direction. The wanted signal and interfering signal are both aligned in space.

## 6.4	In-band selectivity and blocking

### 6.4.1	Background information on the conducted requirement

The in band receiver blocking requirement captures minimum requirements on several aspects of receiver behaviour. The conducted in band blocking requirement is based upon placing a high power interferer within the band on another carrier to a wanted signal. The Several types of interferer and several frequency relationships are defined in different blocking tests.

Figure 6.4.1-1 Example of an RX blocking requirement

The basic principle of the blocking test is that when the blocker is applied, a throughput target on the wanted signal should continue to be achieved. The wanted signal is set at 6 dB above reference sensitivity with the same RMC as for reference sensitivity. Thus in principle distortion arising from the blocker is allowed to desensitize a receiver operating at exactly reference sensitivity by around 6 dB.

It should be noted that the wanted signal level is set with respect to reference sensitivity, which is a minimum requirement and not the actual sensitivity level of the receiver, which may be lower.

Two signal levels are required for the blocking requirement; the blocker interferer level and the wanted signal level. To derive the blocker interferer level for wide area BS, uplink system level simulations were performed as documented in the TR 36.942 [5]. The blocker interferer level was decided partially by considering the 99.99th percentile of RX power observed from a non co-located aggressor LTE system on a victim system but mainly based on the UE transmit power and MCL. Later on, the AAS SI in 37.840 [6] investigated the dependency of the conducted blocker interferer level on antenna architecture and concluded that for the investigated array types, the conducted blocker interferer level was similar both for a receiver attached to a passive array and one attached to an individual antenna element. Based on this, in 37.105 [3] a per TAB connector conducted blocking interferer level is defined that is the same as the blocker interferer level defined in 37.104 [7].

For UTRA single RAT, blocker interferer levels were derived using system level simulations for the medium range and local area BS classes [10]. Higher blocker interferer levels are defined that for the wide area scenario. The wanted signal level is kept at 6dB above the reference sensitivity.

For E-UTRA single RAT, the blocker interferer level for the local area BS class was derived from system level simulations [12], and the wanted signal set at 6 dB above reference sensitivity. For the medium range BS class, the wanted signal was set at 6 dB above reference sensitivity, whilst the blocker interferer level was adjusted such that the difference between the blocker interferer level and the wanted signal level is equalized between E-UTRA wide area and medium range (The adjustment is needed because the reference sensitivity level differs between the two classes) [11].

Comparing UTRA and E-UTRA single RAT blocking requirements, there are differences in the blocker interferer levels (UTRA being larger). For MSR, there is a need for a single set of in band blocking requirements. To achieve this, the UTRA blocker interferer levels, which are more stringent than the E-UTRA were used for the MSR blocking requirement. However the difference between the blocker interferer level and the wanted signal level needed for the wide area BS class to pass blocking is also maintained for the other BS classes. To achieve this, the offset of the wanted signal from the reference sensitivity is adjusted and different to 6 dB for E-UTRA. Also the UTRA wanted signal levels are adjusted to keep a common difference between the blocker interferer level and the wanted signal level.

### 6.4.2	In-band selectivity and blocking: Core requirement

The conducted blocking level simulations assumptions use 3 UE’s in the interfering network, the interferer level recorded for the statistical analysis is the total of all 3 interfering UE’s. When translating this to an OTA requirement where the direction of the interferer also needs to be specified this can cause an added complication.

Directional information is distorted within the existing coexistence simulation framework due to the fact that multipath propagation is approximated as lognormal shadow fading. Whereas in the real world, several multipaths arrive from different directions in to the antenna, in the simulation the signal arrives from the line of sight direction, but a random fading margin is added. Nonetheless, coarse observations on likely locations of blocking UEs can be considered. Thus, using the same simulation parameters as the conducted blocking simulation but recording the data on the power level and location of the UE’s which defined the 99.99% point it was however found that the total power is dominated by a single UE.

Figure 6.4.2-1.Sample of 99.99% Blocking interference level and individual UE power

Therefore only a single interference direction is required to represent the 99.99% blocking case.

The same 99.99% blocking UE’s were investigated to see if they came from a predictable direction.

Figure 6.4.2-2 99.99% Blocking interfering UE location

The 99.99% UE’s direction was found in both azimuth and elevation to be approximately within the element beam width. Whether this is true when multipath propagation is considered is not clear from this type of simulation.

The simulation does not take into account the wanted signal and so it is not possible to find the direction of the wanted signal by the same analysis. It was judged to be feasible however to make a worst case assumption that the wanted UE and the aggressor UE are in the same direction. The assumption may have caused some over dimensioning of the blocking requirement, since the probability of the wanted signal and blocker arriving in the same direction is not 100%, but the potential over dimensioning is seen as acceptable for E-UTRA/UTRA requirements.

In sub-clause 6.2.2.2.2 the OTA reference sensitivity has been defined, this is based on the OTA REFSENS RoAoA which represents the average element/sub-array radiation pattern beam width and is intended to be the OTA equivalent of the conducted reference sensitivity. As such it provides a suitable basis for the OTA wanted signal power level when considering the maximum interference level at the input of each receiver unit.

As the wanted signal and the interferer are in the same location for the purpose of the requirement, and the purpose of the OTA requirements is to offer the same protection and performance as the conducted requirements the difference between the wanted signal and the interfering signal should be maintained. The conducted blocking interferer level can hence be translated to an OTA level using the same method:

Where:

-	PConducted_blocker is the conducted blocking interferer level is rel13

-	PREFSENS, D0 , Doff-peak, and DRX_OTA+MARGIN are the same as defined in sub-clause 6.2.2.2.2.

Or more simply

So

As OTA reference sensitivity and the OTA blocker level are both translated to OTA levels using the same method they retain the same difference and hence the requirement on the receiver unit hardware is similar to the existing conducted specification.

D0 is based on the OTA REFSENS RoAoA which is based on the element/sub array pattern as such is the minimum possible antenna gain for the AAS BS. The blocker interfere level is hence the largest level possible. This ensures that the linearity of the receiver for high signal levels at each of the receiver unit inputs.

However as the conducted requirements are per receiver unit and the OTA level is for the composite receive function the OTA REFSENS value does not include all of the beam forming gain the composite antenna provides. As such the receiver minimum sensitivity as not accuracy represented by the OTA REFSENS value.

When considering the conducted blocking requirement the receiver noise contributes to the receiver performance under blocking conditions as such the interference contribution due to the blocking interferer is only 4.7 dB rather than 6 dB. When only the element gain is considered the lower antenna gain results in higher blocker and wanted signal levels and the noise floor does not contribute significantly to the noise floor so the interference contribution due to the blocking interferer may be the full 6 dB.

For example consider an element of 65° by 65°, in a 10x1 array (0.9λ spacing) with the lowest declared EIS of -116 dBm.

For E-UTRA wide area

For the OTA REFSENS as a wanted power:

For the minSENS case:

As in both cases the system noise floor for the wanted signal is the same and is based on the composite receiver gain (i.e. -117dBm in this example), for the minimum gain case the interference contribution due to the blocking interferer may be:

or a contribution of 4.7dB

For the OTA REFSENS case the interference contribution due to the blocking interferer may be

or a contribution of 5.9dB

Although the OTA REFSENS case has a higher interferer level it may also generate a higher contribution of interference, hence it is difficult to define an absolute worst case as the 2 conditions cover 2 different cases:

-	OTA REFSENS covers the maximum interference level at the RX unit input

-	minSENS covers the case where the effect of the system noise figure is considered.

It is important to bear in mind that the ratio of wanted signal level to blocking level is related to the probability of users and RX power levels occurring in adjacent networks. The probabilities of the signal levels do not change as a result of BS architecture, and thus changing the blocking to wanted signal level would lead to a change in the blocking probability provided by the network.

As both these cases are important the OTA blocking requirement is set at 2 levels, in both cases the delta between the wanted signal and the blocking interferer (Δwanted-blocker) is the same as the difference between the two in the conducted requirement (e.g. 52.5 dB for wide area E-UTRA). This delta value is then applied at 2 OTA wanted signal EIS levels

1.	EISREFSENS + 6 dB

2.	minSENS + 6 dB

### 6.4.3	In-band selectivity and blocking: Conformance requirement

The OTA blocking requirement is tested as follows:

-	In the reference direction of the minSENS OSDD using the minSENS based requirement level

-	In each of the 4 conformance directions at the extremities of the OTA REFSENS RoAoA using the REFSENS based requirement level.

## 6.5	Out-of-band blocking

### 6.5.1	Background

The existing conducted out of band blocking requirement is based on a wanted signal level at PREFSENS + 6 dB, and a CW interfere at -15 dBm. It is valid from 1 MHz to 12.75 GHz excluding the operating band plus a guard (usually 20 MHz either side).

The original derivation of the -15 dBm level was done for the first release of the UTRA specification. The level of -15 dBm is based on analysis done on interference from a number of systems which are close in frequency to the 3GPP bands, notable other 3GPP bands in particular the TDD band 2010 to 2025 MHz and 2400 MHz ISM bands. In both cases the level of -15 dBm was derived by considering the interferer power level, the MCL and assuming that the BS antenna has the same gain for wanted and interfere signals. In some cases the figures were calculated for the UE and then applied to the BS at the same level.

The simple calculations were as follows:

TDD band 2010 to 2025 MHz:	BS to UE case	43 dBm - 60 = -17 dBm

WLAN: max emitted power = 30 dBm, requirement approx. -15 dBm

Based on the available documented information, no consideration was taken for systems operating at vastly different frequencies from the operating band. For a conducted requirement for a single transceiver it is reasonable to apply the same requirement level across the whole out of band spectrum as it likely the requirement is toughest close to the operational band.

### 6.5.2	Out-of-band blocking OTA core requirement

#### 6.5.2.1	Interferer level

There are a number of considerations which make translating the conducted requirement to an OTA requirement challenging:

-	The gain characteristics of an antenna is not known in the out-of-band frequency region.

-	The free space path-loss at high frequencies (i.e. up to 12.75 GHz) means that the radiated power levels required to provide -15 dBm at conducted point are unfeasibly high.

Clearly the interferers are present irrespective of the victim antenna gain, the original levels assumed that the victim antenna gain was the same as the in band gain, but no analysis was done at very high (or low) frequencies.  This was acceptable considering the conducted requirement in previous specifications and the fact it is difficult to estimate out of band antenna gain.

However, with the out-of-band blocking OTA requirement it is not necessary to estimate out-of-band antenna gain so instead another approach can be considered.

For the purposes of the OTA requirement it is useful to consider that the interferer power level is specified at a fixed distance from the AAS BS. This distance does not have to be representative of either the actual minimum distance of the interferer source or be at the same power level (of the interferer). The distance and the level can be adjusted to ensure the requirement presents a worst case requirement for the worst case interferer levels.

For frequencies outside the operating band for which the antenna is designed for the exact antenna gain is unknown, however the victim antenna aperture area is constant over the whole frequency range 30 MHz to 12.75 GHz.

In general, the power gain of an antenna is defined as:

, where Aeff is the effective antenna area in m2 and  is the wavelength in m.

Free space path loss for a given distance is calculated by

, where d is the distance in meters.

The resultant of the antenna gain and the path loss is given by:

means that electromagnetic radiation does not depend on frequency. In practice the antenna aperture Aeff is not constant over frequency, as antennas in general antennas are frequency tuned devices. But here it is reasonable to assume a worst-case scenario where the antenna aperture is constant. Hence it can be noted that the wavelength dependency of the path loss and the antenna gain cancel each out so that the worst-case for a fixed interferer power at a fixed distance results in a fixed equivalent conducted level.

Therefore the interferer power level (EIRP) can be determined at an arbitrary distance of 30m.

The assumed distance however it does not indicate that the conformance testing is done at 30m. The distance and power level may be adjusted considering the variation in FSPL and characteristics of the test facility.

Analysis shows that the worst-case interferers are other 3GPP BS and WLAN systems

For BS to BS the OTA interference power level is given by the co-existence scenario at 288m as:

EIRP = Ptx + Gtx = 43 dBm + 13 dBi - 6dB = 50 dBm, assuming 6 dB due to down-tilt.

The WLAN interference can be calculated based on BS to UE minimum distance and the WLAN Tx power, WLAN is specified in EIRP rather than total power, the value is 30 dB EIRP hence the interference power level can be calculated as 30 dBm (EIRP) at a distance of 42 m.

The historic analysis of the worst-case blocking does not provide the conducted interferer level currently in the specification.

i.e.

Prx = Pout + Gant_agressor – LDT – FSPL + Gant_victim - LDT = 43dBm + 13dBi  - 3dB - 87.65dB + 13dBi – 3dB = -24.65dBm

It could be argued that the existing conducted requirement is to high however, the conducted interferer blocking level of -15dB is more reasonable when it is considered that the antenna gain assumptions are low. Assuming an antenna again of 17dBi  is a more reasonable antenna again assumption.

i.e.

Prx = Pout + Gant_agressor – LDT – FSPL + Gant_victim - LDT = 43dBm + 17dBi  - 3dB - 87.65dB + 17dBi – 3dB = -16.65dBm

Using these assumptions as a basis for the OTA requirement gives:

EIRP(30m) = Prx – Gant + FSPL(30m) = -15dBm – 17dBi + 68dB(frequency=2 GHz, FSPL 30m) = 36dBm

The interferer EIRP level is hence specified as EIRP = 36dBm at 30m.

Polarization between the interferer and the AAS BS need to be taken into account.  As a pragmatic approach, at frequencies at several gigahertz away from the wanted signal polarization matching may be difficult to achieve.  However, polarization alignment where consideration of the physical AAS BS orientation and the interferer antenna polarization aligned need to be handled.  One approach is the requirement and test should use polarization matching in band and keep the same alignment even when performing the OOB measurement, even though the antenna may become detuned at the out of band frequency.

To take into account both polarizations of the AAS BS need to be excited equally, as the interferer is CW only a single polarization can be radiated hence the polarization of the interferer signal is split between the cross polarized victim antennas and therefore the input power needs to be doubled.

The interferer signal level at the aggressor can be converted to a RMS field-strength level at the victim base station as:

in V/m, where EIRP is in W and r is the distance in m.

The aggressor interferer level described as EIRP=36 dBm at a distance of 30m corresponds to a RMS field-strength level of 0.36 V/m. Using field-strength the interferer level the RF core requirement can be described at the base station, without assumptions related to the aggressor.

#### 6.5.2.2	Wanted signal level and direction

As with the in-band blocking requirements whist during operational conditions it is unlikely that the wanted signal and the interferer are in the same direction for the purposes of defining the RF requirement they can be considered to be in the same direction.

The impact of the blocking interfere on the system sensitivity is largest when the wanted signal is at its lowest level. The wanted signal is therefore defined at minSENS + 6 dB.

For practical purposes the OTA wanted signal level is only specified within the declared RoAoA, the direction of the RoAoA coincides with the direction of the antenna maximum gain, this can be assumed to apply for out of band performance as well as in band performance. The out of band blocking requirements are hence only specified over the receiver RoAoA associated with minSENS.

The wanted signal levels apply to all supported polarizations, under the assumption of polarization matching.

#### 6.5.2.3	Out of band region

The out of band requirement is applicable in the frequency range described in section 6.5.2.4, with the exception of the frequency range covered by the operating band plus/minus an additional ΔfOBUE MHz on either side. Based on the discussion on the boundary between in-band and out of band region for NR FR1 operation, the ΔfOBUE values for the out of band blocking were agreed for AAS BS as presented in table 6.5.2.3-1. Those ΔfOBUE values are applicable to conducted and radiated requirements for AAS BS in single RAT E-UTRA operation and to AAS BS in MSR operation using E-UTRA, and are applicable for all E-UTRA operating bands defined in TS 36.104 [7].

Table 6.5.2.3-1: ΔfOBUE values the out of band blocking

|  |  | E-UTRA AAS BS (Single-RAT, MSR) |
| --- | --- | --- |
| ΔfOBUE | Operating band <100 MHz | 20 MHz |
|  | 100 MHz ≦ Operating band ≦ 200 MHz | 60 MHz |

#### 6.5.2.4	Frequency range

The frequency range for the OTA out of band blocking requirement is 30MHz to 12.75GHz (excluding the operating band plus/minus ΔfOBUE as described in section 6.5.2.3). The lower limit of the frequency range is adjusted from 1MHz for the conducted requirement to 30MHz in order to be consistent with the out of band spurious emissions requirement, and considering that measurement at frequencies below 30MHz becomes unfeasible due to the large required chamber size.

### 6.5.3	Out-of-band blocking OTA conformance requirement

Out of band blocking is a long test and hence it is optimum to minimize the number of conformance test directions. Even though the antenna gain is not necessarily maximum at the reference direction above the 2nd harmonic of the designed antenna element center frequency, the practical measurement becomes excessively complex if wanted and interferer signals would be applied from different directions at different frequencies. Therefore it is considered sufficient to show conformance at the reference direction only.

It has been recognized that above 2nd harmonic frequency the antenna element response may be stronger in some other direction than in the reference direction. This means that there is a risk that testing in reference direction does not necessarily test the worst case, at least for some antenna types. Increasing the interferer level was considered to compensate for this effect. However, defining the breakpoint frequency and change in interferer level in a way which would treat different antenna implementations equally was not deemed feasible. Therefore, it was decided to apply a constant interferer level according to the core specification throughout the whole tested frequency range.

## 6.6	Receiver spurious emissions

### 6.6.1	Background information on the conducted requirement

Referring to the TS 37.105 [3], the RX spurious emissions power is defined as power of emissions generated or amplified in a receiver unit that appear at the TAB connector. The RX spurious emissions test purpose is to verify the ability of the AAS BS to limit the interference caused by receiver spurious emissions to other systems. Number of constraints are applicable for the RX spurious requirement depending on the TAB connectors capabilities:

1.	The requirements apply to all AAS BS with separate RX and TX TAB connectors. In this case for FDD AAS BS the test is performed when both TX and RX are ON, with the TX TAB connector terminated,

2.	For a TAB connector supporting both RX and TX in TDD, the requirements apply during the transmitter OFF period,

3.	For a TAB connector supporting both RX and TX in FDD, the receiver spurious requirements are superseded by the TX spurious requirements.

Based on the above limitations, it can be observed that the conducted RX spurious emissions requirement applicability is limited to the following cases:

1.	TDD case: applicable for RX TAB connectors and TX/RX TAB connectors, during TX OFF period,

2.	FDD case: applicable for RX TAB connectors, during TX ON period, when TX TAB connectors are terminated.

According to TS 37.105, the RX spurious emission limits for AAS BS apply in the frequency range from 30 MHz to 12.75 GHz, with consideration of the RAT-specific frequency exclusions for the OBUE/SEM, as well as the spurious range upper frequency limit extensions for selected operating bands, as defined in sub-clause 5.6.6.1 for the TX spurious emissions requirement.

In terms of emission levels, the basic limits and its scaling was also applied to the RX spurious emissions, similar to the TX spurious emissions.

### 6.6.2	Receiver spurious emissions OTA: Core requirement

The metric used to capture the receiver spurious emissions OTA is TRP, defined in the same way as for the transmitter spurious emissions OTA, in sub-clause 5.6.6.2.

The receiver spurious emission requirement is subject to the following limitations:

1.	For OTA AAS BS operating in FDD: OTA RX spurious emissions requirement do not apply to FDD duplex as being superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain.

- NOTE:	The OTA receiver spurious emission requirement applicability for the AAS BS with the RX-only capabilities is FFS.

2.	For OTA AAS BS operating in TDD: the OTA receiver spurious emissions requirement applies during the transmitter OFF period only.

The core requirement for the RX spurious emissions OTA is applied over the same spurious range as defined for the conducted RX spurious requirement in sub-clause 6.6.1.

Derivation of the emission levels for the OTA RX spurious emissions requirement is aligned with the methodology used for the derivation of TX spurious emission levels with the emissions limit, as captured in sub-clause 5.6.6.2.

### 6.6.3	Receiver spurious emissions OTA: Conformance requirement

In general, derivation of the OTA conformance requirement for the RX spurious emissions, uses the same principles as for the TX spurious emissions requirement OTA, as in sub-clause 5.6.6.3.

Definition of the emissions measurements for the conformance requirement is be based on the core requirement, subject to the TRP MU, where the MU definition should capture the whole spurious frequency range.

NOTE:	The OTA RX spurious emission requirement for the AAS BS with the RX-only capabilities is FFS.

For OTA AAS BS operating in TDD: applicability the OTA RX spurious emissions conformance requirement when all transmitters are OFF, is FFS.

{editors note: to be completed as part of conformance work}

## 6.7	Receiver intermodulation

### 6.7.1	Background information on the conducted requirement

The receiver intermodulation requirement relates to the ability of the receiver to avoid mixing (i.e. intermodulation) between 2 strong interferers leading to a significant noise floor rise in the receiver.

A scenario in which IM would arise would be one in which two UEs/transmitters from two interfering systems provide strong enough interference to the base station simultaneously. The interferer levels have been set to be the same in the EUTRA requirement for simplicity reasons; it is not necessarily the case that the interferer levels would be the same in real conditions. Neither is equal interferer levels necessarily the worst case.

### 6.7.2	Receiver intermodulation: Core requirement

Differing antenna architectures could have an impact to RX IM scenarios. However, simulation based investigation would be a highly complex and multidimensional problem for which there was insufficient time within the WI. Fortunately, several factors can be considered which to some extent mitigate the need for such an investigation:

-	It may be assumed that the two interfering systems are statistically independent

-	If it is also assumed that the two interfering systems operate the same RAT in the same deployment scenario (but on different frequencies), then their interference characteristics would be identical

-	For the blocking requirement, an assumption has been made that the conducted blocking level does not depend on the antenna element or module pattern. If the same assumption should be true for each of the interfering systems for RX intermodulation

-	The RX IM levels should be lower than the blocking level, since the probability of two interferers both having a high level simultaneously is lower than the probability of only one interferer having a high level

-	The justification in terms of simplification in assuming equal levels for the conducted requirements can be re-applied for OTA requirements

Taking these factors into account, the OTA RX IM requirement is defined based on the conducted RX IM requirement, with the conducted signal levels translated to OTA levels. Considering in particular the reasoning that the independent conducted levels should not depend on antenna element or module pattern, then a similar approach is adopted for receiver intermodulation as for receiver blocking; i.e. apply the requirement assuming all signals come from the same direction.

Since RX intermodulation relates to receiver non-linearity’s, it is important that the correct absolute conducted level is achieved by the OTA requirement for the receiver intermodulation. This is achieved by using the gain estimated from the OTA REFSENS RoAoA.

For the receiver blocking test, the requirement is also applied at lower absolute signal levels relating to the OTA minimum sensitivity. For RX IM, the signal levels are reduced then the IM effects are likely to reduce due to the LNA operating well within its linearity region. However in order to be consistent with the in band blocking requirement, RX IM is also at both OTA REFSENS and minSENS levels.

### 6.7.3	Receiver intermodulation: Conformance requirement

Since RX sensitivity and blocking already test at all conformance directions, it is sufficient to test RX IM only in a single direction.

## 6.8	In-channel selectivity

### 6.8.1	Background information on the conducted requirement

The in-channel selectivity requirement relates to the ability of the receiver to receive a low power signal whilst receiving a high-power signal in channel.

The selectivity is around 20 dB in 36.104 [8]; this happens to be roughly equal to the SINR range of the demodulation requirements.

### 6.8.2	In-channel selectivity: Core requirement

For setting the requirement, the same ICS as for the conducted requirements is assumed.

In regard to how to set the absolute levels it is worthwhile to consider that aspects of the receiver design that impact the selectivity include linearity, ADC range, phase noise, frequency error and others. The applicable aspects of receiver design divides into those which become of significance at high absolute power level and those that do not (e.g. phase noise). Other requirements, such as receiver blocking and dynamic range already capture the behavior of the receiver at high power level. It is important that the selectivity is achieved after combining. Thus, the most important receiver level to consider for this requirement is the level based on OTA minimum sensitivity.

Thus the receiver ICS requirement is specified at minimum sensitivity level:

-	Wanted signal level = Conducted wanted signal level + ΔminSENS Interferer level = Conducted interferer level + ΔminSENS

### 6.8.3	In-channel selectivity: Conformance requirement

The in channel selectivity requirement is tested in a single direction.

## 6.9	ACS and NB blocking

### 6.9.1	Background information on the conducted requirement

Adjacent channel selectivity (ACS) is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

ACS is derived by considering the results of coexistence system simulations that examine the impact of an aggressor system on the throughput attainable in a victim system. The ACIR, ACLR and ACS levels are set such that the average and 5th percentile throughput do not suffer a significant degradation in a victim network (usually no more than 5%), when adjacent channel interference from an aggressor network is modeled. The simulations are Monte Carlo simulations in which in effect all possible combinations of receiver SINR and interfering signal are modeled to find an ACIR requirement for the up-link and then the ACIR is shared between the BS ACS and the UR ACLR. Both these figures are relative.

It is worthwhile to note that the blocking requirement considers a particular situation in which the interfering signal is received with a very high level, whilst the wanted signal is low. However, the ACS may lead to degradation in many other situations, quite often in situations in which the wanted signal has high SINR and the interfering signal is received at a significantly lower power level than the blocking level. If the selectivity against the interfering signal is not sufficient, then high SINR wanted signals may suffer significant degradation.

The relative ACS for the BS requirement is 45 dBc.

The ACS requirement is in the form of an absolute interferer level and an absolute wanted signal level. The wanted signal level and the ACS relative requirement is used to find the ACS absolute value as follows:

For UTRA wide area BS:

And For E-UTRA wide area BS, 10MHz channel BW:

Note: The actual E-UTRA requirement is -52 dBm the same as UTRA.

The wanted power is an offset from REFSENS, generally it is REFSENS +6 dB however for E-UTRA as the REFSENS is different for different channel band widths the wanted power the wanted signal is fixed based on the 5-20 MHz channel band width requirement.

### 6.9.2	ACS and NB blocking: Core requirement

The ACS requirement is intended to specify the selectivity of the receiver, not the ability of the receiver to process a large signal level.

The conducted ACS has been derived based on co-existence simulations that take into account antenna selectivity considering a passive column. Thus, assuming that the AAS has the same type of antenna as the simulated scenario, then the task of the requirement definition and test is to re-create the conducted requirement. This implies that the test can be carried out with the wanted and interfering signals aligned in direction; there is no need to consider different directions for the signals, since selectivity has already been taken into account in the simulations used to derive the requirement.

For a beamforming system, the beamforming pattern is likely to provide additional spatial selectivity. During the eAAS WI, the impact of this additional spatial selectivity on ACS was not investigated. It is possible that for beamforming systems, the required ACS may be lower than 45 dBc; not higher.

The same 45 dBc is re-used for the OTA requirement.

It is agreed that the blocking requirements are tested using two OTA levels; one based on the OTA REFSENS RoAoA declaration and the second based on the OTA minimum sensitivity which is derived from the OSDD declarations. The purpose of the second level is to avoid that digital combining leads to the blocking being tested with the wanted signal at a large offset against the noise floor in the receiver.

For the ACS requirement, if due to combining the wanted signal would be significantly above the noise floor, then the selectivity would be relaxed somewhat. Thus, testing ACS using a 6 dB degradation against the minimum sensitivity level is a better requirement. On the other hand, testing of ACS based on the OTA REFSENS is not needed, since the blocking requirements tests the ability of the receiver to correctly manage absolute signal levels.

As the conducted absolute ACS level is an offset from REFSENS, the OTA ACS absolute value can be easily found as an offset from OTA minimum sensitivity value.

The requirement is valid over the OTA sensitivity OSDD.

The NB blocking requirement is defined using both the OTA REFSENS RoAoA and the OTA minimum sensitivity RoAoA in the same manner as the in band blocking requirement.

### 6.9.3	ACS and NB blocking: Conformance requirement

Conformance of the ACS requirements can be shown in the reference direction.

Conformance of the NB blocking requirement is shown in the same directions as OTA REFSENS.

# 7	Performance requirements

## 7.1	General

The objective of the performance part of the eAAS WID [2] for the AAS BS demodulation requirements is to clarify, which of the conducted BS demodulation requirements could be applied as radiated requirements for the OTA AAS BS and what methods could be used to perform conformance testing for such requirements. For OTA AAS, no new demodulation requirements are expected to be added beyond the set of AAS BS conducted demodulation requirements referred from single RAT UTRA FDD and E-UTRA specifications in TS 25.104 [19] and TS 36.104 [8] respectively.

NOTE: UTRA TDD BS demodulation requirements are out of scope of the work for OTA AAS BS.

## 7.2	Core requirements for OTA tests

In Rel-15 TS 37.105 [3], the AAS BS demodulation requirements for both hybrid AAS BS and OTA AAS BS have been specified, reusing the single RAT specifications:

1.	For single RAT UTRA FDD, demodulation requirements specified in TS 25.104 [19] for DCH and RACH apply.

2.	For single RAT E-UTRA, demodulation requirements specified in TS 36.104 [8] for PUSCH, PUCCH and PRACH apply. , except the PUSCH requirements for FS3, and except NPUSCH and NPRACH requirements.

Core requirements listed in the following subsections are subject for the discussion on which of the demodulation requirements could be applied as radiated requirements for the OTA AAS BS.

## 7.3	UTRA FDD requirements

In TS 25.104 [19], the following conducted BS demodulation requirements were defined:

Table 7.3-1: Overview of the conducted UTRA FDD demodulation requirements

|  | Tx (UE emulator) | Rx (AAS BS) | Propagation conditions |
| --- | --- | --- | --- |
| DCH |  |  |  |
| Demodulation in static propagation conditions | 1Tx | 1Rx, 2Rx | AWGN |
| Demodulation of DCH in multipath fading conditions | 1Tx | 1Rx, 2Rx | multipath fading (multiple cases defined) |
| Demodulation of DCH in moving propagation conditions | 1Tx | 1Rx, 2Rx | non-fading channel models with two equal power taps with pre-defined time difference between them |
| Demodulation of DCH in birth/death propagation conditions | 1Tx | 1Rx, 2Rx | non-fading propagation channel with two taps; which alternate between 'birth' and 'death'. The positions the paths appear are randomly selected with an equal probability rate. For BS with receiver diversity, the same path positions are applied to both Rx antenna connectors, and the path switching times are synchronized on the two receiver antenna connectors, but the AWGN signals applied to the two receiver antenna connectors are uncorrelated. |
| Demodulation of DCH in high speed train conditions | 1Tx | 1Rx, 2Rx | HST conditions: #1: open space: non-fading propagation channels #2: tunnel: fading propagation channel with one tap #3: tunnel: non-fading propagation channels |
| RACH |  |  |  |
| Performance requirement for RACH preamble detection | 1Tx | 1Rx, 2Rx | AWGN, multipath fading, HST conditions |
| Demodulation of RACH message | 1Tx | 1Rx, 2Rx | AWGN, multipath fading, HST conditions |
| HS-DPCCH |  |  |  |
| ACK false alarm | 1Tx | 2Rx | AWGN, multipath fading (multiple cases defined) |
| ACK mis-detection | 1Tx | 2Rx | AWGN, multipath fading (multiple cases defined) |
| ACK/NACK detection for 4C-HSDPA HS-DPCCH (ACK false alarm, ACK mis-detection) | 1Tx | 2Rx | AWGN, multipath fading |
| ACK/NACK detection for 8C-HSDPA HS-DPCCH | 1Tx | 2Rx | AWGN, multipath fading |
| E-DPDCH |  |  |  |
| E-DPDCH in multipath fading condition | 1Tx | 1Rx, 2Rx | multipath fading |
| E-DPDCH and S-E-DPDCH in multipath fading condition for UL MIMO | 1Tx | 2Rx | multipath fading |
| Signalling detection for E-DPCCH in multipath fading condition | 1Tx | 1Rx, 2Rx | multipath fading |

## 7.4	E-UTRA requirements

In TS 36.104 [8], the following conducted BS demodulation requirements were defined as listed in table 7.4-1.

NOTE: PUSCH requirements for FS3, NPUSCH and NPRACH requirements are not considered for OTA AAS BS.

Table 7.4-1: Overview of the conducted E-UTRA BS demodulation requirements

|  | Tx (UE emulator) | Rx (OTA AAS BS) | Propagation conditions | Notes |
| --- | --- | --- | --- | --- |
| PUSCH |  |  |  |  |
| PUSCH in multipath fading propagation conditions transmission | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA, ETU |  |
|  | 2Tx | 2Rx, 4Rx, 8Rx | EPA |  |
| UL timing adjustment | 1Tx | 2Rx | Moving propagation conditions: AWGN, ETU | 1 stationary UE, 1 moving UE |
| HARQ-ACK multiplexed on PUSCH | 1Tx | 2Rx | EVA, ETU |  |
| High Speed Train conditions | 1Tx | 1Rx, 2Rx | HST scenario#1: two non-fading propagation channels |  |
| PUSCH with TTI bundling and enhanced HARQ pattern | 1Tx | 2Rx, 4Rx, 8Rx | EVA, ETU |  |
| Enhanced performance requirement type A in multipath fading propagation conditions, synchronous interference | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA, ETU | 1 wanted UE, 1 interferer (2 Rx case) or 2 interfering UEs (4Rx or 8Rx case) |
| Enhanced performance requirement type A in multipath fading propagation conditions, asynchronous interference | 1Tx | 2Rx, 4Rx, 8Rx | EPA, ETU | 1 wanted UE, 2 interfering UEs |
| Enhanced performance requirement type B in multipath fading propagation conditions | 1Tx | 2Rx, 4Rx | EPA, EVA, ETU | 2 wanted UEs (2Rx case) or 4 wanted UEs (4Rx case); 1 interfering UE |
| Requirements for PUSCH supporting coverage enhancement | 1Tx | 2Rx | EPA, ETU |  |
| Requirements for PUSCH supporting Cat-M1 UEs | 1Tx | 2Rx | EPA, ETU |  |
| PUCCH |  |  |  |  |
| ACK missed detection for single user PUCCH format 1a transmission | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA, ETU |  |
|  | 2Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| CQI performance requirements for PUCCH format 2 transmission | 1Tx | 2Rx | EVA, ETU |  |
|  | 2Tx | 2Rx | EVA |  |
| ACK missed detection for multi user PUCCH format 1a | 1Tx | 2Rx | ETU | 1 wanted UE, 3 interfering UEs |
| ACK missed detection for PUCCH format 1b with Channel Selection | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| ACK missed detection for PUCCH format 3 | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| NACK to ACK detection for PUCCH format 3 | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| CQI performance requirements for PUCCH format 2 with DTX detection | 1Tx | 2Rx | EVA, ETU |  |
|  | 2Tx | 2Rx | EVA |  |
| PUCCH performance requirements for supporting Cat-M1 UEs | 1Tx | 2Rx | EPA, EVA |  |
| ACK missed detection requirements for PUCCH format 4 | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| ACK missed detection requirements for PUCCH format 5 | 1Tx | 2Rx, 4Rx, 8Rx | EPA, EVA |  |
| PRACH |  |  |  |  |
| PRACH missed detection, normal mode | 1Tx | 2Rx, 4Rx, 8Rx | AWGN, ETU |  |
| PRACH missed detection, high speed mode restricted set type A | 1Tx | 2Rx, 4Rx, 8Rx | AWGN, ETU |  |
| PRACH missed detection, Cat-M1 mode | 1Tx | 2Rx | AWGN, EPA |  |
| PRACH missed detection, high speed mode restricted set type B | 1Tx | 2Rx, 4Rx, 8Rx | AWGN, ETU |  |

For most of the requirements, multiple test cases were defined considering various number or Rx antennas at the BS.

## 7.5	Propagation conditions

The following propagation conditions were used in the BS demodulation requirements in TS 25.141 [21] and TS 36.141 [22]:

-	Static propagation conditions: static propagation consitions rely on AWGN channel with no fading or multi-paths. In case of the OTA test setup no channel emulators are required, and the test setup relies on test signal source and AWGN noise generator

-	Multipath fading propagation conditions: number of multipath channels were defined. In order to translate the fading profiles into OTA test setup, consideration of channel emulators is required on the transmitter side. Use of multiple instances of channel emulators over the same radio channel to model the conducted test setup is not precluded.

-	High Speed Train: three different scenarios were considered for the HST scenarios, using fading and non-fading profiles.

-	Moving propagation conditions: two UEs are modeled in this scenario, one being static, the other moving.

## 7.6	Conformance requirements

The OTA related parts of the conformance testing test procedures for the demodulation requirements are based upon the RX dynamic range requirement OTA procedure, as described in subclause 10.3.4. The setup and operation of the signal generator and measurement instruments are the same as for the corresponding conducted requirements.

Measurement Uncertainties for the OTA requirements are the same as for the conducted requirements, as described in subclause 10.11.

It is noted, that the possibility of OTA testing is considered also for those BS demodulation requirements, which were declared as Optional in TS 25.141 [21] and TS 36.141 [22].

## 7.7	OTA test setup

As the BS demodulation requirements rely on the link level performance between the UE and BS, in case of the requirements definition for the OTA AAS BS, it is required to establish radio link in uplink based on the test equipment (i.e. BS Tester test and reference antenna(s)) located within the RoAoA corresponding to the frequency band used for the BS demodulation testing.

For PUSCH testing purposes in OTA setup, the HARQ feedback can be delivered to the BS Tester via conducted interface.

Figure 7.7-1 shows OTA AAS BS measurement test setup for multipath fading propagation conditions, which is equivalent to the conducted 1TX – 1RX test setup.

The OTA AAS BS is comprised in an anechoic chamber and is configured to receive with single RX configuration. Test signal source is connected to the single TX test antenna. The radio propagation channel is emulated by connecting a fading channel emulator between the test signal generator and the TX antenna.

AWGN noise floor levels required in the conformance test setup are to be derived from conducted BS demodulation test setup procedures. Wanted signal generators are configured so that the SNR levels defined by the demodulation requirements are achievable at the DUT. For SNR derivation, the noise injection at the BS tester side is considered as the baseline approach.

Figure 7.7-1: Basic OTA test setup for multipath fading propagation conditions (equivalent to conducted 1TX – 1RX)

Figure 7.7-2 shows OTA AAS BS measurement test setup for multipath fading propagation conditions, which is equivalent to the conducted 1TX – 2RX test setup. The AAS BS is comprised in an anechoic chamber and is configured to receive with 2RX diversity in a cross-polarised configuration. Test signal source is connected to the 2TX tests antenna in cross-polarised configuration. The two orthogonal radio propagation channels are emulated by passing the same test signal trough one fading channel emulator for each of TX antennas.

Figure 7.7-2: Basic OTA test setup for multipath fading propagation conditions (equivalent to conducted 1TX – 2RX setup)

Figure 7.7-3: Basic OTA test setup for static propagation conditions (equivalent to conducted 1TX – 1RX setup)

Figure 7.7-4: Basic OTA test setup for static propagation conditions (equivalent to conducted 1TX – 2RX setup)

## 7.8	BS demodulation requirements feasible OTA

As per the eAAS WID [2], the objective of the work item is to clarify which type of conducted BS demodulation requirements can be applied as radiated requirements and how it might be tested.

The feasibility analysis of the OTA test cases modelling concluded that the conducted test case with one TX (UE side) and one or two RX chains (AAS BS side), based on UTRA FDD and E-UTRA conducted test cases overview in tables 7.3-1 and 7.4-1, were identified as candidates for feasible OTA demodulation cases.

For 2RX case the cross-polarisation is assumed to be used in order to achieve separation of streams at the RX side of OTA test setup. Most of the conducted test cases from tables 7.3-1 and 7.4-1 in 1TX-1RX, or 1TX-2RX subset, can be implemented in OTA test setups as illustrated in figures 7.7-1 and 7.7-2 for the multipath fading propagation conditions, and in figures 7.7-3 and 7.7-4 for the static propagation conditions.

Based on the analysis of the conducted BS demodulation requirements for UTRA FDD and E-UTRA, the following single RAT requirements were considered as feasible in OTA test setup, by reusing the core requirements defined in TS 25.104 [19] and TS 36.104 [8]:

Table 7.8-1: UTRA FDD BS demodulation requirements feasible OTA

| UTRA FDD BS demodulation requirements |  | Tx (UE emulator) | Rx (OTA AAS BS) |
| --- | --- | --- | --- |
| Demodulation in static propagation conditions | DCH | 1Tx | 1Rx, 2Rx |
| Demodulation of DCH in multipath fading conditions |  |  | 1Rx, 2Rx |
| Demodulation of DCH in moving propagation conditions |  |  | 1Rx, 2Rx |
| Demodulation of DCH in birth/death propagation conditions |  |  | 1Rx, 2Rx |
| Demodulation of DCH in high speed train conditions |  |  | 1Rx, 2Rx |
| Performance requirement for RACH preamble detection | RACH |  | 1Rx, 2Rx |
| Demodulation of RACH message |  |  | 1Rx, 2Rx |
| ACK false alarm | HS-DPCCH |  | 2Rx |
| ACK mis-detection |  |  | 2Rx |
| ACK/NACK detection for 4C-HSDPA HS-DPCCH (ACK false alarm, ACK mis-detection) |  |  | 2Rx |
| ACK/NACK detection for 8C-HSDPA HS-DPCCH |  |  | 2Rx |
| E-DPDCH in multipath fading condition | E-DPDCH |  | 1Rx, 2Rx |
| E-DPDCH and S-E-DPDCH in multipath fading condition for UL MIMO |  |  | 2Rx |
| Signalling detection for E-DPCCH in multipath fading condition |  |  | 1Rx, 2Rx |

Table 7.8-2: E-UTRA BS demodulation requirements feasible OTA

| E-UTRA BS demodulation requirements |  | Tx (UE emulator) | Rx (OTA AAS BS) |
| --- | --- | --- | --- |
| PUSCH in multipath fading propagation conditions transmission | PUSCH | 1Tx | 2Rx |
| UL timing adjustment |  |  | 2Rx |
| HARQ-ACK multiplexed on PUSCH |  |  | 2Rx |
| High Speed Train conditions |  |  | 1Rx, 2Rx |
| PUSCH with TTI bundling and enhanced HARQ pattern |  |  | 2Rx |
| Enhanced performance requirement type A in multipath fading propagation conditions, synchronous interference |  |  | 2Rx |
| Enhanced performance requirement type A in multipath fading propagation conditions, asynchronous interference |  |  | 2Rx |
| Enhanced performance requirement type B in multipath fading propagation conditions |  |  | 2Rx |
| Requirements for PUSCH supporting coverage enhancement |  |  | 2Rx |
| Requirements for PUSCH supporting Cat-M1 UEs |  |  | 2Rx |
| ACK missed detection for single user PUCCH format 1a transmission | PUCCH |  | 2Rx |
| CQI performance requirements for PUCCH format 2 transmission |  |  | 2Rx |
| ACK missed detection for multi user PUCCH format 1a |  |  | 2Rx |
| ACK missed detection for PUCCH format 1b with Channel Selection |  |  | 2Rx |
| ACK missed detection for PUCCH format 3 |  |  | 2Rx |
| NACK to ACK detection for PUCCH format 3 |  |  | 2Rx |
| CQI performance requirements for PUCCH format 2 with DTX detection |  |  | 2Rx |
| PUCCH performance requirements for supporting Cat-M1 UEs |  |  | 2Rx |
| ACK missed detection requirements for PUCCH format 4 |  |  | 2Rx |
| ACK missed detection requirements for PUCCH format 5 |  |  | 2Rx |
| PRACH missed detection, normal mode | PRACH |  | 2Rx |
| PRACH missed detection, high speed mode restricted set type A |  |  | 2Rx |
| PRACH missed detection, Cat-M1 mode |  |  | 2Rx |
| PRACH missed detection, high speed mode restricted set type B |  |  | 2Rx |

It is noted, that it is not within the scope of eAAS WI to extend the scope of functionality captured by the conducted BS demodulation requirements.

# 8	EMC requirements

## 8.1	General

The Rel-13 AAS EMC specification [13] was drafted with a simple principle of applying the existing EMC requirements to the AAS BS architecture hence it was possible to refer to the existing UTRA , E-UTRA and MSR EMC specifications [16][15][14]. It was possible to use this approach since release 13 of AAS BS specification [3] only allows for AAS BS architectures with a conducted interface, which means that the antenna array can be disconnected and TAB connectors can be terminated. Hence the system can be treated in the same way as in the existing specifications and therefore the EMC requirements remained applicable in exactly the same way.

The OTA AAS BS architecture does not have a conducted interface and so the antennas cannot be disconnected. This means that the conducted methods where the EMC antenna ports are terminated are no longer be possible and hence new methods for EMC testing are needed.

Figure 8.1-1 shows the prime difference in architecture from an EMC view point when comparing the ports on a conventional non-AAS BS to the ports on an AAS BS.

Figure 8.1-1: Comparison between BS architectures for EMC testing

Since the antenna array for the OTA AAS BS is integrated within the EUT, the transmissions from the EUT within the test chamber have antenna gain included in them.

Figure 8.1-2 illustrates the status of EMC antenna ports during EMC testing where it is noted that the ports are connected to a terminating load and the hybrid AAS BS is then set to transmit at full power.

Figure 8.1-2: EMC Antenna port status during EMC testing

## 8.2	Regulatory EMC requirements

The following table provides a summary of the existing EMC regulatory requirements in EU and US market. These requirements should serve as a reference when specifying the EMC requirements for the AAS BS.

Table 8.2-1: Overview of the regulatory EMC requirements for EU and US markets

|  | EU market | US market |
| --- | --- | --- |
| RF Radiated Spurious Emission (RSE) requirements | -36dBm/100kHz ERP below 1 GHz and -30dBm/1MHz ERP above 1 GHz Requirements specified in ETSI EN 301 908-1, and are tested with the antenna port connected in a terminating load. No TX exclusion bands defined for BS equipment today. | General requirement of -13 dBm/MHz ERP is applicable Requirements specified in FCC Part 2. FCC does not define any TX exclusion bands for BS equipment today. |
| EMC Radiated Emission (RE) requirements | -50dBm/MHz ERP general requirement. Not applicable for BS equipment Defined in ETSI EN 55032. | -50dBm/MHz above 1 GHz mandatory requirement. Specified in FCC Part 15, §15.35. |
| Radiated Immunity (RF electromagnetic field) requirements | Two type of requirements defined: 3 V/m, 80 MHz – 690 MHz, and 10 V/m 690 MHz – 6 GHz. Defined in ETSI EN 301 489-1 and ENSI EN 301 489-50 RX Exclusion bands defined 20 MHz above and below the operating band | No requirements today |

Note:	From the above regulatory requirements, the radiated spurious emission requirements (-36 dBm/100kHz and -30 dBm/1MHz) are reflected in the existing EMC specifications for AAS BS [13] and non-AAS BS [14]. Similarly, the 3 V/m immunity requirement is captured in the existing EMC specifications.

## 8.3	Field strength in EMC chamber

### 8.3.1	General

This sub-clause provides an example estimation of field strength within the EMC chamber. Assuming free space propagation loss in an anechoic chamber, the path loss can be calculated as:

, where Gt and Gr are the antenna gains (with respect to an isotropic radiator) of the transmitting and receiving antennas respectively,  is the wavelength, and R is the distance between the TX and RX.

Assuming a 0dBi antenna gain for the TX and RX, the isotropic free space path loss is plotted in figure 8.3.1-1 for 30, 1000, 2000, 4000, 6000 and 12000 MHz.

Figure 8.3.1-1: Free-space path loss, assuming Gt=Gr=0 dBi

It can be noted that for a typical distance between the EUT and the probe antenna within the chamber, (e.g. 5 meters), the path loss at 2 GHz is more than 50 dB. However, if the EUT has an integrated antenna which has gain, and the measurement probe antenna also has gain the total loss between the EUT and the measurement equipment may drop. For example, an EUT with  TX antenna gain = 10 dBi and assuming 10 dBi antenna gain for the probe antenna, the coupling loss at 5 meters is roughly 30 dB, i.e. 20 dB lower than the  coupling loss assuming 0 dBi antenna gains. With this observation, and the fact the AAS BS is transmitting and connected to its integrated antenna, it can be contemplated that an AAS BS EUT leads to significantly increased field strength within the chamber compared to the usual EMC measurement scenario where the transmitters are connected to terminating loads. This increased field strength and associated received power from the measurement probe could potentially damage the test equipment.

### 8.3.2	Conversion between dBm to V/m

Since the field strength within the EMC chamber is expressed in terms of electric field strength (i.e. V/m), it is important to establish the intensity of electric field strength generated when transmitting at a certain power (dBm). This conversion exists in prior art and is explained in [17]. In summary, for a free space wave impedance of 377 ohm, the electric field (V/m) can be expressed as:

Where R is the distance of EUT from the interferer (during the immunity testing).

This formula provides us a way to calculate the field strength within the chamber.

## 8.4	Protection of measurement equipment

In order to protect the measurement device from the increased field strength levels within the EMC chamber during the spurious emission testing. Traditionally in EMC testing as the BS transmitters are working into a load the received signal from the EMC text antenna is at a low power level which can be fed directly into the measurement equipment. With the AAS BS testing as the transmitter units are radiating through the AAS BS antenna the transmit signal received at the measurement device is of a high power. In order to protect the measurement device from this signal, the device needs to ensure additional attenuation by filtering out the wanting signal with a notch or band stop filter. Additionally, TX exclusion bands similar to those used in existing conducted spurious emissions requirements may need to be defined in certain areas to ensure that the increased field strength does not block/damage the measurement device.

The existing RAT-specific frequency boundary limits defined for out-of-band emissions by the SEM or OBUE requirements in TS 37.105 [3] is a sufficient boundary to limit exposure of the measurement’s equipment receiver from the high field strengths of the wanted signal.

## 8.5	EMC rated emission requirement

In terms of transmitter spurious emission, two types of core requirements specified for AAS BS are relevant:

1.	The RF spurious emission requirements specified in TS 37.105 [3] with different measurement bandwidths and spurious emission levels as detailed in SM.329 [18].

2.	The EMC radiated emission requirements specified in TS 37.114 [13] with the same measurement bandwidths as the RF spurious emission requirements.

In terms of the applicability of the RF Radiated Spurious Emission (RF RSE) and EMC Radiated Emission (EMC RE) requirements in the OTA setup, according to the existing conducted requirements for UTRA, E-UTRA and MSR, these requirements apply from 30 MHz to 12.75 GHz with consideration of exceptions:

-	Unwanted emissions: the RF RSE requirement apply at frequencies within the above specified frequency range with the exception of the UTRA spectrum emission mask (SEM) and E-UTRA operating band unwanted emissions (OBUE) ranges specified in single RAT and MSR specifications. For more details on the unwanted emissions exclusion ranges refer to subclause 5.6.6.1.

-	Upper frequency: in order for the RF RSE requirement to comply with the SM.329 recommendation [18], for certain operating bands the upper limit of the spurious range is extended beyond 12.75GHz to the limit of 5th harmonic (but not higher than 26GHz). For more details on the upper limit setting in those cases, refer to SM.329 [18].

The applicability of EMC RE and RF RSE requirements is illustrated in figure 8.5-1 below

Figure 8.5-1: Applicability of spurious emission requirements

It is to be noted that these EMC RE requirements are applicable at the enclosure port and the EMC antenna port is connected to a terminating load while the AAS BS is transmitting at full power. The EMC RE are then tested in this setup to make sure that the requirements are fulfilled.

For the OTA AAS BS architecture that does have the possibility of disconnecting the composite antenna, the EMC antenna ports can no longer be connected to a terminating load. This means that the RF RSE and EMC RE requirements cannot be differentiated any needs to be considered as single OTA requirement specified at the enclosure of the DUT. The single requirement contains a RF RSE component and an EMC RE component as shown in figure 8.5-1 and the total emission requirement is as specified in the SM.329 [18] recommendation.

Mechanism to specify the regional and additional co-existence requirements is for FFS.

## 8.6	Radiated Immunity

### 8.6.1	General

This sub-clause specifies EMC Radiated Immunity (RI) requirements for OTA AAS BS.

When performing EMC RI tests of an OTA AAS BS, due to lack of the antenna connectors a higher power level may, for certain test scenarios, enter the RX of the EUT, while for a hybrid AAS BS the antenna connectors would be terminated during test preventing this to happen. This may lead to the receiver of the OTA AAS BS being inoperable (in case the RF blocking requirements are not met). To avoid the OTA AAS BS receiver being inoperable during the EMC RI test, protection mechanisms need to be employed for BS RX during the RI testing.

### 8.6.2	Measurement set-up for testing radiated immunity

A high level example of EMC RI test setup for an OTA AAS BS is illustrated in figure 8.6.2-1. The Equipment Under Test (EUT) is placed in an anechoic chamber on top of a turn table. A range antenna or probe antenna generates the interfering RF electromagnetic field.

Figure 8.6.2-1: Example radiated immunity test setup for an OTA AAS BS

An important difference of the EMC radiated immunity test setup for an OTA AAS BS as compared with testing of the EUT equipped with the antenna connectors is the need for OTA performance parameter monitoring. During EMC RI test the EUT is rotated horizontally in order to expose it to the interfering RF electromagnetic field from all directions in horizontal plane.

For EUT equipped with the antenna connectors (i.e. the performance of the EUT is monitored via antenna connectors), the rotation of the EUT during the EMC RI test had no impact on the performance parameters being monitored. For an OTA AAS BS testing, the OTA link established between the EUT and the test equipment antenna may be impacted by the rotation of the test object during the test.

Another aspect when introducing the OTA performance parameter monitoring, is the need to introduce the test equipment antenna in such a way that the calibration of the interfering signal is not affected.

These two aspects are taken into account when setting up the EMC RI test for the OTA AAS BS.

According to IEC 61000-4-3 [28], all testing of equipment shall be performed in a configuration as close as possible to actual installation conditions. In order to allow electromagnetic susceptibility testing of the maximum surface for OTA AAS BS, the EUT shall be mounted using rack mount, if possible.

### 8.6.3	Spatial exclusions for radiated immunity testing

According to IEC 61000-4-3 [28], EMC Radiated Immunity test are normally performed with the generating antenna facing each side of the EUT. When technically justified, some EUTs can be tested by exposing fewer faces to the generating antenna. For OTA AAS BS the radiated immunity (RI) test need to be reduced to fewer angular incidences. This is to protect the EUT receiver from being blocked due to received input power levels exceeding the OOB blocking limits during the RI testing. Referring to the IEC 61000-4-3 [29] guidance above, the angular incidence where the front of the OTA AAS BS is facing the interferer range antenna needs to be omitted, as depicted in figure 8.6.3-1.

Figure 8.6.3-1: RI testing directions for OTA AAS BS (horizontal plane depicted)

# 9	Manufacturer declarations

## 9.1	General

The Rel-15 AAS BS manufacturer’s declarations are defined in TS 37.145-1 [23] for the conducted testing of the hybrid AAS BS over its conducted RF interface, and in TS 37.145-2 [24] for the radiated testing of the hybrid AAS BS or OTA AAS BS over their radiated RF interfaces.

The following groups of the manufacturer’s declarations were considered for the hybrid AAS BS conformance requirements:

1.	For conducted requirements:

the already existingRel-13/14 manufacturers declarations as defined in the conducted conformance testing specification TS 37.145-1 [23] are reused for Rel-15 hybrid AAS BS. Those declaration were subject to multiple corrections: some declarations were found redundant (e.g. CA related declarations), additional missing declarations were introduced (e.g. BS class), multiple corrections were implemented for consistence.

2.	For the OTA requirements:

the already existing Rel-13/14 manufacturers declarations as defined for the radiated EIS and EIRP requirements in the radiated conformance testing specification TS 37.145-2 [24] table 4.10-1, were reused for Rel-15 hybrid AAS BS.

The following groups of the manufacturer’s declarations were considered for the OTA AAS BS conformance requirements:

1.	The Rel-13/14 set of OTA manufacturers declarations as defined for EIRP and EIS requirements for the hybrid AAS BS (2a above) in the radiated conformance testing specification TS 37.145-2 [24], table 4.10-1.

2.	Set of radiated testing declarations for OTA AAS BS, which were mirrored from the conducted specifications in Rel-15, and captured in TS 37.145-2 [24], table 4.10-2. Conducted declarations mirroring was performed in order to obtain independent declaration sets for hybrid AAS BS and the OTA AAS BS.

3.	Newly defined Rel-15 manufacturers declarations, for OTA REFSENS test requirement, captured in TS 37.145-2 [24], table 4.10-2.

## 9.2	Void

# 10	Conformance testing aspects

## 10.1	OTA conformance testing framework

### 10.1.1	General

OTA conformance testing framework for the directional and TRP test requirements was compiled based on the one defined for Rel-13 AAS in TR 37.842 [4], sub-clause 10.1.2. The eAAS OTA conformance testing framework considers additional aspects of the TRP measurements, e.g. sampling grid, additional MU contributors.

The OTA conformance testing framework for the TX directional and TRP test requirements is defined as follows:

For TX directional requirements, testing using the narrowest declared beamwidth [in a single direction] is sufficient for demonstrating conformance, except for the case of EVM in which case testing should be performed in all conformance directions of the OTA coverage range.

### 10.1.2	Uncertainty budget calculation principle

The uncertainty budget calculation principles for estimation of the combined uncertainty for directional requirements was already defined in Rel-13 AAS WI in TR 37.842 [4].

The uncertainty budget calculation principle for directional and TRP requirements, with the division for transmitted and receiver requirements is expected to be required for eAAS WI. Details on the uncertainty budget calculation principles are {editors note: to be completed as part of conformance work}.

## 10.2	Measurement uncertainty for TX directional requirements

### 10.2.1	General

For the derivation of the measurement uncertainty for direction requirements, the approach already defined in the TR 37.842 [4] should be reused as much as possible.

Tx directional requirements where possible can use the same OTA test chambers and corresponding calibration and test procedures as the existing EIRP accuracy requirement. Most TX directional requirements are based upon the EIRP test procedures in TR 37.842 [4] where specific test models are referenced.  However, for simplicity since requirements here do not all use the same test model the procedures have been kept general by using “transmitting test signal” where the appropriate test model is found in TS 37.105 [3]

Most TX directional requirements require demodulation of the transmitted signal using the “global in channel TX test” described in Annex F of TS 36.141. As such the measurement equipment MU varies from the MU of the measurement equipment used for the EIRP accuracy requirement defined in TR 37.842 [4].

Some requirements are differential and hence many of the calibration and OTA measurement uncertainties may cancel. These are investigated on a case by case basis.

### 10.2.2	EIRP accuracy

#### 10.2.2.1	General

The EIRP accuracy uncertainty budget was re-assessed and adjusted slightly according as detailed in the following tables. Only the MU budget for the CATR is shown as it is this MU that determines the agreed MU and TT.

#### 10.2.2.2	CATR

##### 10.2.2.2.1	MU assessment

###### 10.2.2.2.1.1	MU budget

Table 10.2.2.3.1.1-1: CATR uncertainty budget format for EIRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | RF power measurement equipment | F1 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 5 | QZ ripple DUT | E2-5 |
| 19 | Miscellaneous uncertainty | E2-14 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | F1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | F1 |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |
| 23 | Frequency flatness | E2-16 |

###### 10.2.2.2.1.2	MU value

| EIRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value | Uncertainty value | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] | Standard uncertainty ui [dB] |
|  |  | f ≦ 3GHz | 3GHz ≦ f < 4.2 GHz |  |  |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 2 |  | 0.14 | 0.26 | Gaussian | 1 | 1 | 0,14 | 0,26 |
|  | RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  |  |  |  |  |  |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0,15 | 0,15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0,0012 | 0,0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0,0928 | 0,0928 |
| 23 | Frequency flatness | 0,25 | 0,25 | Normal | 1 | 1 | 0,25 | 0,25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0,13 | 0,2 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0,09 | 0,23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0,1 | 0,1 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0,0012 | 0,0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0,015 | 0,015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0,29 | 0,25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0,25 | 0,25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0,034 | 0,034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0,06 | 0,06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0,009 | 0,009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0,15 | 0,15 |
| Combined standard uncertainty (1σ) [dB]. | 0,568 | 0,646 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1,11 | 1,27 |

### 10.2.3	OTA E-UTRA DL RS power

{editors note: multiple test chamber solutions may be added here, Indoor anechoic chamber and CATR shown}

#### 10.2.3.1	General

DL RS power is an E-UTRA measurement only, it is defined as:

The DL RS power is the resource element power of the Downlink Reference Symbol at the RIB transmitting the DL RS for a cell.

The absolute DL RS power is indicated on the DL-SCH. The absolute accuracy is defined as the maximum deviation between the DL RS power indicated on the DL-SCH and the DL RS power of each E-UTRA carrier.

As such is an absolute power (EIRP) measurement of a single RE.

The 37.104 conducted requirement and hybrid BS requirement both assume that the value broadcast on DL-SCH is a conducted or TRP value, since the power is measured at the antenna connector. To facilitate efficient testing, OTA DL RS power is tested by measuring EIRP as a directional requirement, and compared with a declared DL RS EIRP derived from the power broadcast on the DL-SCH and the AAS BS directivity in the direction to be tested.. This is sufficient to demonstrate the ability of the physical layer to deliver accurate RS power.

As the requirement is based on absolute directional power the absolute level will be subject to the same calibration and measurement errors as the EIRP measurement which has previously been analyses in TR 37.842. However as the measurement is for a single RE it is necessary to demodulate the received signal this is done at the same time as the EVM test using the global in channel TX test described in Annex F of TS 36.141.

The demodulated power accuracy test is not as accurate as the power measurement using a power meter for the EIRP accuracy test.

The MU is calculated by using the existing conducted MU in the MU uncertainty budget in place of the RF measurement equipment.

#### 10.2.3.2	In-door anechoic chamber

##### 10.2.3.2.1	General

This method measures the EIRP in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.2.3.2.1-1.

Figure 10.2.3.2.1-1: Indoor Anechoic Chamber measurement system setup

##### 10.2.3.2.2	Calibration

Figure 10.2.3.2.2-1: Indoor Anechoic Chamber calibration system setup

1)	Connect the reference antenna and the receiving antenna to the measurement RF out port and RF in port of the network analyzer, respectively, as shown in figure 10.2.3.2.2-1. The amplifier may be installed between C and D if required.

2)	Install the reference antenna with its beam peak direction and the height of its phase centre aligned with the receiving antenna.

3)	Set the centre frequency of the network analyzer to the carrier centre frequency of the tested signal and measure LFTX_E→D , which is equivalent to 20log|S21| (dB) obtained by the network analyzer:

-	LFTX_E→D: Pathloss between E and D in figure 10.2.3.2.2-1.

4)	Measure the cable loss, LFTX_E→F between the reference antenna connector and the network analyzer connector:

-	LFTX_E→F: Cable loss between E and F in figure 10.2.3.2.2-1.

5)	Calculate the calibration value between A and D with the following formula:

-	LTX_cal, A→D = LFTX_E→D  + GREF_ANT, F→A - LFTX_E→F.

-	LTX_cal, A→D:  Calibration value between  A and D in figure10.2.3.2.2-1.

-	GREF_ANT, A→F: Antenna gain of the reference antenna.

##### 10.2.3.2.3	Procedure

1)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

2)	Set the AAS BS to generate the tested beam with the appropriate test model with the beam peak direction intended to be the same as the testing direction.

3)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

4)	Measure the appropriate test parameter

5)	Repeat steps 2-6 for all conformance test beam direction pairs and test conditions.

The appropriate test parameter in step 6 for DL RS power is PDL_RS the measured signal power of DL RS EIRP (in the beam peak direction), and calculation of EIRP EIRPDL_RS using the following equation:

EIRPDL_RS = PDL_RS + LTX_cal, A→D.

And

EIRPDL_RS = EIRPDL_RS_p1 + EIRPDL_RS_p2 where the declared beam is the measured signal at port 1 (p1) and port 2 (p2).

##### 10.2.3.2.4	MU assessment

###### 10.2.3.2.4.1	MU Budget

As the DL RS power is an absolute measurement most of the uncertainties form the EIRP accuracy remain the same. Also it can be noted that the measured signal is a wanted signal and hence will be beam formed in the same way as the wanted signal, hence any errors which may be dependent on the beam shape will be the same as for the EIRP accuracy measurement.

Table 10.2.3.2.4.1-1: Indoor Anechoic Chamber uncertainty contributions
for OTA E-UTRA DL RS power measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. | E1-2 |
| 3 | Quality of quiet zone | E1-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E1-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E1-5 |
| 6 | Phase curvature | E1-6 |
| 8 | Impedance mismatch in the receiving chain | E1-8 |
| 9 | Random uncertainty | E1-9 |
| 23 | MU of TE derived from conducted specification (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | E1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E1-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer | E1-12 |
| 13 | Quality of quiet zone | E1-3 |
| 14 | Polarization mismatch for reference antenna | E1-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E1-5 |
| 16 | Phase curvature | E1-6 |
| 17 | Uncertainty of the Network Analyzer | F1 |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E1-15 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | F1 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E1-18 |

###### 10.2.3.2.4.2	MU Value

Table 10.2.3.2.4.2-1: Indoor Anechoic Chamber uncertainty assessment for OTA E-UTRA DL RS power measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 23 | MU of TE derived from conducted specification | 0.41 | 0.56 | Gaussian | 1 | 1 | 0.41 | 0.56 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.59 | 0.73 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.15 | 1.43 |

#### 10.2.3.3	CATR

##### 10.2.3.3.1	General

The Compact Antenna Test Range (CATR) uses the DUT which radiates a wave front to a range antenna reflector which will then collimate the radiated spherical wave front into a feed antenna.  The sufficient separation between the DUT and the receiver (feed antenna shown in figure 10.2.3.3.1-1) so that the emanating spherical wave reaches nearly plane phase fronts from transmitter to receiver. The DUT transmits a wave front that will illuminate the range antenna reflector, which will then reflect the transmitted energy into the feed antenna.  The range feed antenna is connected to a vector network analyzer or other equivalent test equipment.

Figure 10.2.3.3.1-1: CATR measurement system setup for OTA E-UTRA DL RS power

Test Method limitations and scope

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is taken into account.

##### 10.2.3.3.2	Calibration

The calibration measurement is done by using a reference antenna (SGH used in figure 10.2.3.3.2 -1) with known efficiency or gain values. In the calibration measurement the reference antenna is measured in the same place as the DUT, and the attenuation of the complete transmission path (C↔A, as in figure 10.2.3.3.2 -1) from the DUT to the measurement receiver is calibrated out. Figure 10.2.3.3.2 -1 presents a setup of a typical compact antenna test range.

Figure 10.2.3.3.2-1: CATR calibration system setup for OTA E-UTRA DL RS power

1)	Path loss calibration C→A:

a)	Measure SGH (or other calibrated reference antenna) reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓSGH.

b)	Measure cable loss from point C to input of SGH, call this LC↔SGH which is the equivalent of 20log|S21| from the use of a network analyser.

c)	Calculate the combined total path loss from C→A by using the following expression:

-	LSGHcal = LC,SGH + 10log(1 - |ΓSGH|2) - GSGH;

-	where10log1-ΓSGH2  10log(1 - |ΓSGH|2)  is the compensation for SGH connector return loss, GSGH  is the known gain of the reference SGH.

2)	Connect SGH and C↔A cable.

3)	To remove polarization(s) mismatch between range antenna (labelled as feeder antenna in diagram) and SGH use positions to position the SGH in the boresight of range antenna.

4)	Measure path loss C→B with network analyzer LC→B = 20log|S21|.

5)	Calculate the test path loss compensation factor.  This is the total path loss between A↔B using the results from step 1c and 4. L = LSGHcal LSGHcal  - LC→B.

Where ΓSGH is the reflection coefficient (or mismatch) seen at the SGH connector (S11 with a network analyzer).

##### 10.2.3.3.3	Procedure

1)	Set up AAS BS in place of SGH from calibration stage.  Align DUT with boresight of range antenna.

2)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

3)	Configure TX branch and carrier according to maximum power requirement and test configuration.

4)	Set the AAS BS to transmit the test signal.

5)	Measure the appropriate test parameter

6)	Repeat steps 2-6 for all conformance test beam direction pairs and test conditions.

The appropriate test parameter in step 6 for DL RS power is PDL_RS the measured signal power of DL RS EIRP (in the beam peak direction), calculation of EIRP EIRPDL_RS using the following equation:

EIRPDL_RS_p(x) = PDL_RS_meas + LA→B.

And

EIRPDL_RS = EIRPDL_RS_p1 + EIRPDL_RS_p2 where the declared beam is the measured signal at port 1 (p1) and port 2 (p2).

##### 10.2.3.3.4	MU assessment

###### 10.2.3.3.4.1	MU Budget

As the DL RS power is an absolute measurement most of the uncertainties form the EIRP accuracy remain the same. Also it can be noted that the measured signal is a wanted signal and hence will be beam formed in the same way as the wanted signal, hence any errors which may be dependent on the beam shape will be the same as for the EIRP accuracy measurement.

Table 10.2.3.3.4.1-1: CATR uncertainty budget format for OTA E-UTRA DL RS power measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | RF power measurement equipment | F1 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 5 | QZ ripple DUT | E2-5 |
| 19 | Miscellaneous uncertainty | E2-2 |
| 23 | MU of TE derived from conducted specification (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | F1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | F1 |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |

###### 10.2.3.3.4.2	MU Value

Table 10.2.3.3.4.2-1: CATR uncertainty assessment for OTA E-UTRA DL RS power measurement

| EIRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 23 | MU of TE derived from conducted specification | 0.41 | 0.56 | Gaussian | 1 | 1 | 0.41 | 0.56 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.64 | 0.77 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.25 | 1.52 |

#### 10.2.3.4	Near Field

##### 10.2.3.4.1	General

The near field measurement techniques consist in measuring amplitude and phase of the modulated signal at the AAS BS under test, on some specific surfaces such as planar, cylindrical, and spherical. Each of the near field test methods can be implemented by one or more mechanical rotations of the probe and/or AAS BS under test as shown in figure 10.2.3.2.4.1-1. One or more mechanical movement can be substitute by a probe array. All the scanning methods will need an RF transmit and receive system equipped with an automated scanning, a data collection and control system, and computerized analysis ability.

Figure 10.2.3.4.1-1: Probe/scanner near field systems: spherical, cylindrical and planar

NOTE:	Although there are three methods available for obtaining the near field data, the spherical method is used as a working example.

In case the radiated field is sampled on a sphere surrounding completely the AAS BS under test, the 3D full sphere EIRP value can be measured in near field when the AAS BS is transmitting a defined modulated signals in a declared beam. The near field measurement technique would imply the use of mathematical artefact, NF to FF transform in order to have the EIRP value the in far field. The near field to far field transform is based on the well-known Huygens-Fresnel principle. The spherical modal wave expansion is the implicit application of the Huygens principle. A direct solution of the Helmholtz equations is found by applying boundary conditions on the surface S at infinity. From the tangential fields over the surface, the modal coefficients can be determined using the orthogonality of the modal expansion. Based on this formulation, the near field sampling criteria does play an important role

##### 10.2.3.4.2	Calibration

Stage 1 – Calibration:

Calibration accounts for the various factors affecting the measurements of the EIRP. These factors include components such as range length path loss, cable losses, gain of the receiving antenna, etc. Each measured data point for both radiated power and radiated sensitivity is transformed from a relative value in dB to an absolute value in dBm. For doing that the total path loss from the DUT to the measurement receiver, named L path loss is calibrated out. The calibration measurement is usually done by using a reference antenna with known gain. This approach is based on the so called gain-comparison method [25]. Figure 10.2.3.4.2 -1 shows the typical configuration for measuring path loss.

Figure 10.2.3.4.2-1: Typical Lpath loss measurement configuration

The Lpath loss can be determined from the power into the reference antenna by adding the gain of the reference antenna:

Piso = Pref + GrefPIsotropic Radiator=Preference antenna+Greference antenna

so that:

Lpath loss = Pref + Gref - PtestLpath loss=Preference antenna+ Greference antenna- Ptest equipment

In order to determine Pref, a cable reference measurement is performed in order to calibrate out the A, and B pathsLtest equipment-receiver. Assuming that the power at the source is fixed, it can be showed that:

Pref-Ptest=Prec'-Prec

Where Prec Preceiver and Prec' are the power measured at the receiver during the calibration measurement with the reference antenna and the power measured at the receiver during the cable reference measurement respectively. Lpath loss Lpath loss is then given by:

Lpath loss = Gref + Prec' – Prec

##### 10.2.3.4.3	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

1)	AAS BS near field radiation pattern measurement: this is performed with the AAS BS transmitting a defined modulated signal, as defined in appropriate conformance test specification [24].

a.	DL RS is measured during near field radiation pattern measurement and used as the basis for the NF to FF transformation.

2)	AAS BS near field to far field transformation: the near field power calibration is applied.

The near field to far field transformation is a mathematical computation which is applied to the near field measured radiation pattern in order to compute the far field radiation pattern. It is typically performed expanding the measured near field over a set of orthogonal basis functions. The near to far field transform is then performed in two steps:

1)	Expansion (or projection) of the measured near field (i.e. Emeas(r)) over a set of orthogonal basis functions (i.e. Fbasis(r)) in order to evaluate the transformed spectrum:

Emeas(r) = Spectrum * Fbasis(r)

2)	FF (i.e. EFF) computation using the previuosly calculated spectrum and with the basis functions evaluated at r∞ (i.e. Fbasis(r  ∞)):

EFF = Spectrum * Fbasis(r  ∞)

When performing the near field to far field transformation, the gain calibration is applied so that the near field pattern will be transformed from relative power (i.e. dB) to absolute power (i.e. dBm). In this specific case, the EIRP far field pattern is expressed in terms of the absolute power radiated by the AAS BS in the declared beam:

1)	DL RS power EIRP: once the full 3D far field EIRP pattern has been computed, the DL RS power EIRP can be derived at the beam peak direction according to the declared beam direction pair.

##### 10.2.3.4.4	MU assessment

###### 10.2.3.4.4.1	MU budget

Table 10.2.3.4.4-1: Near field test range uncertainty contributors in AAS OTA E-UTRA DL RS power measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2:, EIRP near field radiation pattern measurement and EIRP near field DUT power measurement |  |
| 1 | Axes Intersection | E3-1 |
| 2 | Axes Orthogonality | E3-2 |
| 3 | Horizontal Pointing | E3-3 |
| 4 | Probe Vertical Position | E3-4 |
| 5 | Probe H/V pointing | E3-5 |
| 6 | Measurement Distance | E3-6 |
| 7 | Amplitude and Phase Drift | E3-7 |
| 8 | Amplitude and Phase Noise | E3-8 |
| 9 | Leakage and Crosstalk | E3-9 |
| 10 | Amplitude Non-Linearity | E3-10 |
| 11 | Amplitude and Phase Shift in rotary joints | E3-11 |
| 12 | Channel Balance Amplitude and Phase | E3-12 |
| 13 | Probe Polarization Amplitude and Phase | E3-13 |
| 14 | Probe Pattern Knowledge | E3-14 |
| 15 | Multiple Reflections | E3-15 |
| 16 | Room Scattering | E3-16 |
| 17 | DUT support Scattering | E3-17 |
| 18 | Scan Area Truncation | E3-18 |
| 19 | Sampling Point Offset | E3-19 |
| 20 | Spherical Mode Truncation | E3-20 |
| 21 | Positioning | E3-21 |
| 22 | Probe Array Uniformity | E3-22 |
| 23 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | E3-23 |
| 24 | Insertion loss of receiver chain | E3-24 |
| 25 | Uncertainty of the absolute gain of the probe antenna | E3-25 |
| 36 | MU of TE derived from conducted specification | F2 |
| 27 | Measurement repeatability - Positioning Repeatability | E3-27 |
|  | Stage 1: Calibration measurement |  |
| 28 | Uncertainty of network analyser | E |
| 29 | Mismatch of receiver chain | B4-29 |
| 30 | Insertion loss of receiver chain | B4-30 |
| 31 | Mismatch in the connection of the calibration antenna | B4-31 |
| 32 | Influence of the calibration antenna feed cable | B4-32 |
| 33 | Influence of the probe antenna cable | B4-33 |
| 34 | Uncertainty of the absolute gain of the calibration antenna | E |
| 35 | Short term repeatability | B4-35 |

Note:	Refer to TR37.842 Annex B for description of each uncertainty term

###### 10.2.3.4.4.2	MU value

Table 10.2.3.4.4.2-1: Near field test range uncertainty assessment for OTA E-UTRA DL RS power measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Scan Area Truncation | 0.003 | 0.003 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 19 | Sampling Point Offset | 0.0058 | 0.0058 | Gaussian | 1.00 | 1 | 0.01 | 0.01 |
| 20 | Spherical Mode Truncation | 0.015 | 0.015 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 21 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 22 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 23 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 24 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 25 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 36 | MU of TE derived from conducted specification | 0.41 | 0.56 | Gaussian | 1.00 | 1 | 0.41 | 0.56 |
| 27 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 28 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 29 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 30 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 32 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 33 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 34 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 35 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.6 | 0.71 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.17 | 1.39 |

#### 10.2.3.5	Summary

{editors note: Last section at this level summarises MU estimates for all chambers and concludes final agreed MU}

The different OTA test chamber MU are summarised below:

Table 10.2.3.5-1: Test system specific measurement uncertainty values for the OTA E-UTRA DL RS power test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1.15 | 1.43 |
| Compact Antenna Test Range | 1.25 | 1.52 |
| Near Field chamber | 1.17 | 1.39 |
| … |  |  |
| Common maximum accepted test system uncertainty | 1.3 | 1.5 |

### 10.2.4	OTA Output power dynamics

#### 10.2.4.1	General

There are a number of UTRA and E-UTRA dynamic power requirements, they are all relative requirements which specify the dynamic range and step size accuracy of UTRA code domain channels and E-UTRA RE’s.

As the requirements are differential many of the measurement uncertainty values may cancel out however, in some cases demodulated signal power level (code domain power or RE power) are compared to maximum carrier power. As such the differential measurements may use different test equipment setting and hence not all test equipment uncertainties are cancelled out.

In all cases the measured signal are wanted signals and will be subject to the same beam forming as the main beam.

The conducted measurement has only 1 variable that being the measurement equipment. The measurement equipment MU can therefore be assumed to be the same as the conducted MU.

With a 95% confidence level (1.96σ) and a Gaussian distribution the test equipment MU is 0.4dB.

#### 10.2.4.2	In-door anechoic chamber

##### 10.2.4.2.1	General

This method measures the EIRP in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.2.3.2.1-1.

##### 10.2.4.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2

##### 10.2.4.2.3	Procedure

Reference procedure in subclause 10.2.3.2.3 where in step 6 the appropriate measurement is:

The appropriate test parameter in step 6 for the output power dynamics vary depending on the specific measurement as described for the conducted measurement in TS 37.145-1 in each case however the EIRP measurement is made on both polarisations and added as follows:

EIRPmeas_p(x) = Pmeas_p(x) + LA→B.

And

EIRPmeas = EIRPmeas_p1 + EIRPmeas_p2

where the declared beam is the measured signal at port 1 (p1) and port 2 (p2).

Furthermore, the measurement is performed twice; once with the BS transmitting at Prated,c,EIRP on all PRBs and then a second time with the BS transmitting on a single PRB. Both measurements are made in the same conformance direction in the same calibrated test setup. The ratio of these two EIRP levels is used to assess compliance for the OTA total power dynamic range.

##### 10.2.4.2.4	MU assessment

###### 10.2.4.2.4.1	MU Budget

As the output power dynamics are relative measurements most of the uncertainties form the EIRP accuracy cancel out as the same error will be applied to both of the measured OTA signals.

This includes all calibration errors, misalignment errors, impedance mismatch and mutual coupling.

As the both the measured OTA signal will have the same beam pattern quiet zone errors, phase curvature errors also can be expected to be the same for both signals.

The uncertainty budget descriptions are the same as those in table 10.2.3.2.4.1-1 with the addition descriptions in tble 10.2.4.2.4.1-1.

Table 10.2.4.2.4.1-1: Indoor Anechoic Chamber uuncertainty contributions
for AAS BS OTA E-UTRA Total power dynamic range measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna (note 1) | E1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. (note 1) | E1-2 |
| 3 | Quality of quiet zone (note 1) | E1-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna (note 1) | E1-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna (note 1) | E1-5 |
| 6 | Phase curvature (note 1) | E1-6 |
| 8 | Impedance mismatch in the receiving chain (note 1) | E1-8 |
| 9 | Random uncertainty (note 1) | E1-9 |
| 23 | Uncertainty of conducted measurement (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer (note 1) | E1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna (note 1) | E1-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer (note 1) | E1-12 |
| 13 | Quality of quiet zone (note 1) | E1-3 |
| 14 | Polarization mismatch for reference antenna (note 1) | E1-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E1-5 |
| 16 | Phase curvature (note 1) | E1-6 |
| 17 | Uncertainty of the Network Analyzer (note 1) | F1 |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability (note 1) | E1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty (note 1) | E1-15 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability (note 1) | E1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna (note 1) | F1 |
| 22 | Uncertainty of the absolute gain of the receiving antenna (note 1) | E1-18 |
| Note 1: These uncertainties cancel out as the requirement is differential |  |  |

###### 10.2.4.2.4.2	MU Value

The MU uncertainty assessment is shown in table 10.2.4.2.4.2-1, zero values have been omitted in the table for the sake of space, but still be considered as part of the budget.

Table 10.2.4.2.4.2-1: Indoor Anechoic Chamber uncertainty assessment for OTA Total power dynamic range measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 23 | Uncertainty of the RF Power Measurement Equipment | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.21 | 0.21 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.42 | 0.42 |

The same uncertainty assessments have been carried out for the UTRA dynamic range requirements (Power control steps, Power control dynamic range, Total power dynamic range, IPDL Time mask). In each case the uncertainty for the conducted measurement is the same as that for the conducted MU in TS 25.141 as follows:

Power control steps, Uncertainty of conducted measurement = 0.1 dB, Expanded OTA uncertainty = 0.15dB.

Power control dynamic range, Uncertainty of conducted measurement = 1.1 dB, Expanded OTA uncertainty = 1.11dB.

Total power dynamic range, Uncertainty of conducted measurement = 0.3 dB, Expanded OTA uncertainty = 0.32dB.

IPDL Time mask, Uncertainty of conducted measurement = 0.7dB, Expanded OTA uncertainty = 0.71dB.

#### 10.2.4.3	CATR

##### 10.2.4.3.1	General

The Compact Antenna Test Range (CATR) uses the DUT which radiates a wave front to a range antenna reflector which will then collimate the radiated spherical wave front into a feed antenna.  The sufficient separation between the DUT and the receiver (feed antenna shown in figure 10.2.3.3.1-1) so that the emanating spherical wave reaches nearly plane phase fronts from transmitter to receiver. The DUT transmits a wave front that will illuminate the range antenna reflector, which will then reflect the transmitted energy into the feed antenna.  The range feed antenna is connected to a vector network analyzer or other equivalent test equipment.

##### 10.2.4.3.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.3.2

##### 10.2.4.3.3	Procedure

Reference procedure in subclause 10.2.3.2.3 where in step 6 the appropriate measurement is:

The appropriate test parameter in step 6 for the output power dynamics vary depending on the specific measurement as described for the conducted measurement in TS 37.145-1 in each case however the EIRP measurement is made on both polarisations and added as follows:

EIRPmeas_p(x) = Pmeas_p(x) + LA→B.

And

EIRPmeas = EIRPmeas_p1 + EIRPmeas_p2

where the declared beam is the measured signal at port 1 (p1) and port 2 (p2).

Furthermore, the measurement is performed twice; once with the BS transmitting at Prated,c,EIRP on all PRBs and then a second time with the BS transmitting on a single PRB. Both measurements are made in the same conformance direction in the same calibrated test setup. The ratio of these two EIRP levels is used to assess compliance for the OTA total power dynamic range.

##### 10.2.4.3.4	MU assessment

###### 10.2.4.3.4.1	MU Budget

As the output power dynamics are relative measurements most of the uncertainties form the EIRP accuracy cancel out as the same error will be applied to both of the measured OTA signals.

This includes all calibration errors, misalignment errors, impedance mismatch and mutual coupling.

As the both the measured OTA signal will have the same beam pattern quiet zone errors, phase curvature errors also can be expected to be the same for both signals.

The uncertainty budget descriptions are the same as those in table 10.2.3.2.4.1-1 with the addition descriptions in tble 10.2.4.3.4.1-1.

Table 10.2.4.3.4.1-1: Indoor Anechoic Chamber uncertainty contributions
for OTA Total power dynamic range measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 23 | Uncertainty of conducted measurement (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |

###### 10.2.4.3.4.2	MU Value

The MU uncertainty assessment is shown in table 10.2.4.3.4.2-1, zero values have been omitted in the table for the sake of space, but still be considered as part of the budget.

Table 10.2.4.3.4.2-1: CATR uncertainty assessment for OTA Total power dynamic range measurement

| EIRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 23 | Uncertainty of conducted measurement | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.20 | 0.20 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.40 | 0.40 |

The same uncertainty assessments have been carried out for the UTRA dynamic range requirements (Power control steps, Power control dynamic range, Total power dynamic range, IPDL Time mask). In each case the uncertainty for the conducted measurement is the same as that for the conducted MU in TS 25.141 as follows:

Power control steps, Uncertainty of conducted measurement = 0.1 dB, Expanded OTA uncertainty = 0.1dB.

Power control dynamic range, Uncertainty of conducted measurement = 1.1 dB, Expanded OTA uncertainty = 1.1dB.

Total power dynamic range, Uncertainty of conducted measurement = 0.3 dB, Expanded OTA uncertainty = 0.3dB.

IPDL Time mask, Uncertainty of conducted measurement = 0.7dB, Expanded OTA uncertainty = 0.7dB.

#### 10.2.4.4	Near Field Test range

##### 10.2.4.4.1	Description

The near field measurement techniques consist in measuring amplitude and phase of the modulated signal at the AAS BS under test, on some specific surfaces such as planar, cylindrical, and spherical. The measurement setup is as depicted in clause 10.2.3.4.1.

##### 10.2.4.4.2	Calibration

Calibration shall be performed with the same procedure as in clause 10.2.3.4.2.

##### 10.2.4.4.3	Procedure

The measurement procedure consists of the following steps:

1)	Measure full DL RS pattern according to the procedure in clause 10.2.3.4.3.

2)	From the beam peak direction

a.	Measure the appropriate test parameter as specified for the conducted measurement in TS 37.145-1 for conducted measurement in each case however the signal power is measured for both polarizations.

##### 10.2.4.4.4	MU assessment

###### 10.2.4.4.4.1	MU budget

As the output power dynamics are relative measurements most of the uncertainties form the EIRP accuracy cancel out as the same error will be applied to both of the measured OTA signals.

The uncertainty budget descriptions are the same as those in table 10.2.3.3.4.1-1 (excluding uncertainties from the NF to FF transformation, since the transformation is not needed) with the addition descriptions in tble 10.2.4.4.4.1-1:

Table 10.2.4.4.4.1-1: Near Field uncertainty contributions for OTA E-UTRA Total power dynamic range measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 23 | Uncertainty of conducted measurement (conducted MU / 1.96) | F2 |
| Stage 1: Calibration measurement |  |  |

###### 10.2.4.4.4.2	MU value

The MU uncertainty assessment is shown in table 10.2.4.4 .4.2-1, zero values have been omitted in the table for the sake of space, but still be considered as part of the budget.

Table 10.2.4.4.4.2-1: Near Field uncertainty assessment for E-UTRA OTA E-UTRA Total power dynamic range measurement

| EIRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 23 | Uncertainty of conducted measurement | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.20 | 0.20 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.40 | 0.40 |

#### 10.2.4.5	Summary

The different OTA test chamber MU are summarised below:

Table 10.2.4.5-1: Test system specific measurement uncertainty values for the OTA E-UTRA dynamic range test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 0.42 | 0.42 |
| Compact Antenna Test Range | 0.4 | 0.4 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 0.4 | 0.4 |

Table 10.2.4.5-2: Test system specific measurement uncertainty values for the UTRA Power control steps test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 0.15 | 0.15 |
| Compact Antenna Test Range | 0.1 | 0.1 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 0.1 | 0.1 |

Table 10.2.4.5-3: Test system specific measurement uncertainty values for the UTRA Power control dynamic range test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1.11 | 1.11 |
| Compact Antenna Test Range | 1.1 | 1.1 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 1.1 | 1.1 |

Table 10.2.4.5-4: Test system specific measurement uncertainty values for the OTRA Total power dynamic range test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 0.32 | 0.32 |
| Compact Antenna Test Range | 0.3 | 0.3 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 0.3 | 0.3 |

Table 10.2.4.5-5: Test system specific measurement uncertainty values for the UTRA IPDL Time mask test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 0.71 | 0.71 |
| Compact Antenna Test Range | 0.7 | 0.7 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 0.7 | 0.7 |

### 10.2.5	OTA Transmitted signal quality –Frequency Error

#### 10.2.5.1	General

Frequency error is the measure of the difference between the actual BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

Frequency error is not affected by any  time or frequency varying amplitude errors. It is possible that some aspects of the OTA environment may impact the signal fidelity; for example, ripple in a quiet zone relates to reflections in the chamber and may create a frequency specific ripple. It is not expected that such effects would have any significant impact on the frequency error As long as the signal is large enough the only measurement uncertainty will be associated with the measurement equipment. It is important that considering the chamber path loss and gain of the measurement antenna and equipment, the test equipment is provided with a sufficient RX power level. As a guide, measurement equipment vendors quote EVM accuracy down to approx -20dBm input power for E-UTRA signals.

A quick look at the power levels for a AAS under test:

Looking a few typical output power levels and antenna arrangements for different BS classes:

| Class | Pout (dBm) | G_AAS (dBi) | L_ant (m) | d_FF (m) | FSPL (dB) | G_RX (dBi) | P_RX (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| wide | 43 | 17 | 1 | 13.33 | 60.96 | 9 | 8.04 |
| med | 38 | 9 | 0.5 | 6.67 | 54.94 | 9 | 1.06 |
| local | 24 | 5 | 0.2 | 2.67 | 46.98 | 9 | -8.98 |

It can be seen that there is considerable margin over -20dBm for the received power level, so it seems unlikely that this will cause any measurement problems.

As the OTA test system is not expected to substantially affect the measurement accuracy for frequency error the existing conducted MU can be used.

#### 10.2.5.2	In-door anechoic chamber

##### 10.2.5.2.1	General

This method measures the frequency error in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.2.3.2.1-1.

##### 10.2.5.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2 to ensure that the SNR at the measurement equipment input is appropriate and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.5.2.3	Procedure

Reference procedure in subclause 10.2.3.2.3 where in step 6 the appropriate measurement parameter is frequency error. In this case, however testing should be carried out in the OTA conformance reference direction, not the beam peak direction of the OTA peak directions set reference direction.

##### 10.2.5.2.4	MU assessment

The uncertainty in the power accuracy of the OTA test chamber will not affect the frequency error uncertainty.

Possible phase variation in the chamber due to variation in the signal BW is not significant to affect the frequency error measurement uncertainty.

The conducted MU value is adopted for the OTA MU.

#### 10.2.5.3	CATR

##### 10.2.5.3.1	General

##### 10.2.5.3.2	Calibration

Calibration of the CATR test setup should be carried out in the same manner as described for the EVM procedure in subclause 10.2.5.3.2.

##### 10.2.5.3.3	Procedure

As the frequency error is tested together with EVM, the measurement procedure is same with EVM.

1) Align DUT with boresight of range antenna.

2) Configure TX branch and carrier according to required test configuration

3) Measure frequency error of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument) denoted in figure [10.2.5.3.1-1].

4) Repeat steps 3-4 for all conformance test direction as described in 3GPP TS 37.145-2 [24], subclause 6.2.

6) Repeat steps 3-5 for all conformance test models listed in 3GPP TS 37.145-2 [24]. Note: all the discussions above are based on the measurement pre-condition of reference clock synchronized between measurement system with RF frequency signal denoted in figure [10.2.5.3.1-1].

##### 10.2.5.3.4	MU assessment

The uncertainty in the power accuracy of the OTA test chamber will not affect the frequency error uncertainty.

Possible phase variation in the chamber due to variation in the signal BW is not significant to affect the frequency error measurement uncertainty.

The conducted MU value is adopted for the OTA MU.

#### 10.2.5.4	Near Field

##### 10.2.5.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA Frequency Error type of measurements, NF to FF transform is not needed. Frequency Error is measured in Near Field for the declared direction.

##### 10.2.5.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in 10.2.3.4.2 to ensure that the SNR at the measurement equipment input is appropriate and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.5.4.3	Procedure

Stage 2 – Measurement:

1.	The testing procedure consists of the following steps: Align the AAS BS with (Theta, Phi) angles corresponding to the declared conformance direction to be measured

2.	Configure TX branch and carrier according to the manufacturer’s declared rated output power

3.	Set the AAS BS to transmit the test signal according to appropriate test models.

4.	Measure Frequency Error of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument).

5.	Repeat steps 3-5 for other appropriate test models.

For conformance tests, Frequency Error shall be measured at maximum power setting.

##### 10.2.5.4.4	MU assessment

The uncertainty in the power accuracy of the OTA test chamber will not affect the frequency error uncertainty.

Possible phase variation in the chamber due to variation in the signal BW is not significant to affect the frequency error measurement uncertainty.

The conducted MU value is adopted for the OTA MU.

#### 10.2.5.5	Summary

The uncertainty in the power accuracy of the OTA test chamber will not affect the frequency error uncertainty.

Possible phase variation in the chamber due to variation in the signal BW is not significant to affect the frequency error measurement uncertainty. The frequency error MU is ±12 Hz.

### 10.2.6	OTA Transmitted signal quality –EVM

#### 10.2.6.1	General

EVM is defined as:

The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector. The equaliser parameters are estimated as defined in Annex E. The EVM result is defined as the square root of the ratio of the mean error vector power to the mean reference power expressed in percent.

Although EVM is represented as a % it can also be thought of as a relative power ratio in dBc, when looked at as a power ratio the effect of potential amplitude errors in the OTA chamber can be seen

Figure 10.2.6.1-1: Example of EVM as a power ratio in dB.

In the example the co-channel noise is 45dBc which equates to an EVM of 0.56%, if the measurement system were to alter the ratio between the wanted signal and the co-channel signal then this would affect the EVM result in %.

##### 10.2.6.1.1	Test equipment MU

The existing conducted MU is 1% which is added in a linear fashion to the requirement, for example for 64QAM the 8% core requirement becomes a 9% test requirement.

Sources of EVM which are independent of each other however are added in an rms fashion, for an independent EVM contributor to change 8% to 9% would require an EVM of:

(2.8% for 256QAM)

The test equipment MU is quoted at approx [2%], so is greater than the MU value itself but smaller than allowable MU assuming an independent source.

#### 10.2.6.2	In-door anechoic chamber

##### 10.2.6.2.1	General

This method measures the EVM in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.2.3.2.1-1.

##### 10.2.6.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2 to ensure that the SNR at the measurement equipment input is appropriate and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.6.2.3	Procedure

Reference procedure in subclause 10.2.3.2.3 where in step 6 measure the EVM for the appropriate test case and the specific test models are the same as those for the equivalent conducted requirement in TS 37.145-1. In this case, however testing should be carried out in the OTA conformance reference direction, not the beam peak direction of the OTA peak directions set reference direction.10.2.6.2.4	MU assessment

###### 10.2.6.2.3.1	MU Budget

As both the wanted signal and the noise signal are at the same frequency they will be measured at the same time the requirement is effectively differential and most of the OTA chamber errors will cancel out.

The wanted signal will be beam formed and hence the errors used for the EIRP accuracy will be valid, however the co-channel noise may not be beam formed and hence could suffer different errors due to the chamber quite zone, and phase profile. These items are included in both the calibration error and the measurement error, as the requirement is differential if there is a difference between the wanted and the unwanted it will only be due to the measurement phase. The calibration errors will cancel as calibration is only done one so they will be the same for both wanted and unwanted signals.

Potentially, the EVM may vary in space due to different patterns of wanted signal and distortion. Thus for narrow beams, it may be possible that beam pointing and alignment errors could impact EVM results.

As EVM is also dependent on the phase of the calibrated path it is possible that phase ripple in the quite zone or elsewhere, which arises due to multipath reflections, may lead to frequency ripple and cause additional EVM errors which do not appear in a power accuracy analysis as done for EIRP accuracy.

The potential impacts of both beam pointing misalignment and scattering within the chamber on the received waveform and measurement accuracy were investigated. The potential deviation in the measured EVM arising from beam pointing errors was examined considering a worst case scenario, in which variation in space of EVM is maximal due to the ideal signal being correlated and the distortion uncorrelated; hence the impact of misalignment error would be the difference between array gain and element gain. Even in this circumstance, alignment errors of several degrees would not lead to a significant error in the measured EVM. Considering all likely chamber sizes, for E-UTRA any scattering would fall within the cyclic prefix of the OFDM symbol and hence not cause ISI. Furthermore, the likely delay spread of any scattering would relate to coherence bandwidths much larger than any UTRA/E-UTRA channel bandwidth. Even if the scattered energy would cause interference, the interference level would anyhow not lead to a significant EVM increase. Thus it was concluded that the impact scattering within the measurement chamber would be negligible.

###### 10.2.6.2.3.2	MU Value

The uncertainty causing by power variations when measuring EVM is indicated in table 10.2.6.2.4.2-1:

Table 10.2.6.2.3.2-1: Indoor Anechoic Chamber uncertainty assessment for power uncertainty aspects of OTA EVM

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.13 | 0.13 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.25 | 0.25 |

The indoor anechoic chamber budget is carried out without consideration of the measurement equipment as this MU is given in %, converting to dB gives, for example:

2% is equivalent to 20*log10(2/100) = -33.98dB

If the unwanted signal is 0.25dB higher than the wanted due to the test system then this will be degraded to -33.73dB, and

-33.73dB is equivalent to;	10(-33.73/20) *100 = 2.06%

Additional error due to potential phase error has not been considered however the potential increase due to then OTA test equipment is well within the contribution allowable with a 1% linear MU.

Note:	Analysis of the phase uncertainties indicates that the contributions are not significant to affect the final MU value, however if future work indicates that phase or any other errors not related to amplitude calibration may affect the EVM measurement uncertainty the MU analysis may be re-examined.

#### 10.2.6.3	CATR

##### 10.2.6.3.1	General

In a Compact Antenna Test Range (CATR) the DUT which radiates a wave front to a range antenna reflector which will then collimate the radiated spherical wave front into a feed antenna.  There is sufficient separation between the DUT and the receiver (feed antenna shown in figure 10.2.3.3.1-1) so that the emanating spherical wave achieves nearly plane phase fronts from transmitter to receiver. The range feed antenna is connected to a vector network analyzer or other equivalent test equipment.

Test method limitations and scope

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is considered.

##### 10.2.6.3.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.3.2 to ensure that the SNR at the measurement equipment input is appropriate and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.6.3.3	Procedure

Reference procedure in subclause 10.2.3.3.3 where in step 6 the appropriate measurement parameter is the time alignment error and the specific test models are the same as those for the equivalent conducted requirement in TS 37.145-1. In this case, however testing should be carried out in the OTA conformance reference direction, not the beam peak direction of the OTA peak directions set reference direction.

##### 10.2.6.3.4	MU assessment

###### 10.2.6.3.4.1	MU Budget

As both the wanted signal and the noise signal are at the same frequency they will be measured at the same time the requirement is effectively differential and most of the OTA chamber errors will cancel out.

The wanted signal will be beam formed and hence the errors used for the EIRP accuracy will be valid, however the co-channel noise may not be beam formed and hence could suffer different errors due to the chamber quite zone, and phase profile. These items are included in both the calibration error and the measurement error, as the requirement is differential if there is a difference between the wanted and the unwanted it will only be due to the measurement phase. The calibration errors will cancel as calibration is only done one so they will be the same for both wanted and unwanted signals.

Potentially, the EVM may vary in space due to different patterns of wanted signal and distortion. Thus for narrow beams, it may be possible that beam pointing and alignment errors could impact EVM results.

As EVM is also dependent on the phase of the calibrated path it is possible that phase ripple in the quite zone or elsewhere, which arises due to multipath reflections, may lead to frequency ripple and cause additional EVM errors which do not appear in a power accuracy analysis as done for EIRP accuracy.

The potential impacts of both beam pointing misalignment and scattering within the chamber on the received waveform and measurement accuracy were investigated. The potential deviation in the measured EVM arising from beam pointing errors was examined considering a worst case scenario, in which variation in space of EVM is maximal due to the ideal signal being correlated and the distortion uncorrelated; hence the impact of misalignment error would be the difference between array gain and element gain. Even in this circumstance, alignment errors of several degrees would not lead to a significant error in the measured EVM. Considering all likely chamber sizes, for E-UTRA any scattering would fall within the cyclic prefix of the OFDM symbol and hence not cause ISI. Furthermore, the likely delay spread of any scattering would relate to coherence bandwidths much larger than any UTRA/E-UTRA channel bandwidth. Even if the scattered energy would cause interference, the interference level would anyhow not lead to a significant EVM increase. Thus it was concluded that the impact scattering within the measurement chamber would be negligible.

###### 10.2.6.3.4.2	MU Value

The uncertainty causing by power variations when measuring EVM is indicated in table 10.2.6.2.4.2-1:

Table 10.2.6.3.4.2-1: CATR uncertainty assessment for power variation aspects for OTA E-UTRA DL RS power measurement

| EIRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.18 | 0.18 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.35 | 0.35 |

The CATR budget is carried out without consideration of the measurement equipment as this MU is given in %, converting to dB gives, for example:

2% is equivalent to 20*log10(2/100) = -33.98dB

If the unwanted signal is 0.35dB higher than the wanted due to the test system then this will be degraded to -33.63dB, and

-33.63dB is equivalent to;	10(-33.63/20) *100 = 2.08%

Additional error due to potential phase error has not been considered however the potential increase due to then OTA test equipment is well within the contribution allowable with a 1% linear MU.

Note:	Analysis of the phase uncertainties indicates that the contributions are not significant to affect the final MU value, however if future work indicates that phase or any other errors not related to amplitude calibration may affect the EVM measurement uncertainty the MU analysis may be re-examined.

#### 10.2.6.4	Near Field

##### 10.2.6.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA EVM type of measurements, NF to FF transform is not needed. EVM is measured in Near Field for the declared directions.

##### 10.2.6.4.2	Test method limitations and scope

For AAS BS implementations that point a beam in the EVM conformance direction during testing, the near field technique may report too high, but will never report too low EVM. In case the BS does not point a beam centre in the test direction, there is a risk of the near field technique report too low EVM

##### 10.2.6.4.3	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in 10.2.3.4.2. to ensure that the SNR at the measurement equipment input is appropriate and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.6.4.4	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

1.	Align the AAS BS with (Theta, Phi) angles corresponding to the declared conformance direction to be measured

2.	Configure TX beamforming and carrier according to the manufacturer's declared rated output power

3.	Set the AAS BS to transmit the test signal according to the appropriate test model.

4.	Measure EVM of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument).

5.	Repeat steps 3-4 for all conformance test beam direction pairs as described in 3GPP TS 37.145-2 [24], subclause 6.2.

6.	Repeat steps 3-5 for other appropriate test models.

The EVM of each carrier for different modulation schemes on (HS-)PDSCH to be less than the limits defined in 3GPP TS 37.105 [3], subclause 9.6.

For conformance tests, EVM shall be measured at maximum and minimum power settings while frequency error, and occupied BW at only maximum power setting.

##### 10.2.6.4.5	MU assessment

###### 10.2.6.4.5.1	MU Budget

EVM is a relative measurement given that the wanted signal and noise signal are at the same frequency and measured at the same time therefore most of the OTA anechoic chamber errors will cancel out.

Nearly all of uncertainty terms for the EVM measurement and EVM calibration are the same and hence EVM is a differential or relative measurement.

Potentially, the EVM may vary in space due to different patterns of wanted signal and distortion. Thus for narrow beams, it may be possible that beam pointing and alignment errors could impact EVM results. The importance and impact of such effects  is likely to be even smaller than for far field based measurements.

As EVM is also dependent on the phase of the calibrated path it is possible that phase ripple in the quiet zone or elsewhere, which arises due to multipath reflections, may lead to frequency ripple and cause additional EVM errors which do not appear in a power accuracy analysis as done for EIRP accuracy. Analysis of the effect of such effects is that the effects will be even smaller than for far field based measurements.

###### 10.2.6.4.5.2	MU Value

The Near Field budget is carried out without consideration of the measurement equipment therefore the MU is given in % and can be converted to dB , for example:

2% is equivalent to 20*log10(2/100) = -33.98dB

If the unwanted signal is 0.35dB higher than the wanted due to the test system then this will be degraded to -33.63dB, and

-33.63dB is equivalent to;	10(-33.63/20) *100 = 2.08%

Additional error due to potential phase error has not been considered however the potential increase due to then OTA test equipment is well within the contribution allowable with a 1% linear MU.

For Near Field Test Range a new measurement uncertainty term shall be added to the MU. This term will take into account the fact that in Near Field the phase pattern will sum up so that the signal level is increasing while the noise level is the same. This MU term will only cause an error in the direction of increasing the reported EVM value and not decreasing it, and will depend on the implementation of the DUT.

Thus for the near field test range the MU is decided to be 1% + increase in EVM due to different wanted signal/distortion beam patterns (dependent on DUT architecture).

Note:	Analysis of the phase uncertainties indicates that the contributions are not significant enough to affect the final MU value, however if future work indicates that phase or any other errors not related to amplitude calibration may affect the EVM measurement uncertainty, the MU analysis may need to be re-examined.

#### 10.2.6.5	Summary

Without consideration of any phase uncertainty, the amplitude error analysis shows the conducted MU of 1% can be maintained for the OTA MU (subject to the clarification noted in the limitations section that the reported EVM may be greater than the real EVM due to the difference between near field and far field EVM values. The extent of such a difference is dependent on the architecture of the DUT).

Note:	Analysis of the phase uncertainties indicates that the contributions are not significant to affect the final MU value, however if future work indicates that phase or any other errors not related to amplitude calibration may affect the EVM measurement uncertainty the MU analysis may be re-examined.

### 10.2.7	OTA Transmitted signal quality –TAE

#### 10.2.7.1	General

TAE is the timing difference between 2 modulated signals, either TX diversity, MIMO or CA carriers. Conducted TAE is measured as follows:

Figure 10.2.7.1-1 – Conducted TAE measurement set up

As the conducted signals are combined before being input to the test equipment the OTA test is simple to implement.

Figure 10.2.7.1-2 – OTA TAE measurement set up

The OTA signals are both transmitted from the AAS BS and added at the test receive antenna. As the test paths for the 2 signals are identical there are no additional timing errors added to the test system compared to the conducted test system.

As the TAE requirement is a measure of timing difference it is not affected by the accuracy of the test system amplitude calibration and measurement uncertainties.

As long as the signals fed into the measurement equipment are of a sufficient amplitude then the only measurement uncertainty will be that of the measurement equipment itself.

The measurement equipment requires a level of > -20dBm to accurately carry out the TAE.

As the requirement is done for the wanted beam at full power, even for low power AAS BS it is not anticipated the received test signal will be lower than -10dBm.

There may be a tiny impact to signal fidelity due to scattering in the chamber, however this will be insignificant and is not expected to impact TAE.

As the OTA test system does not affect the measurement accuracy for TAE the existing conducted MU can be used.

#### 10.2.7.2	In-door anechoic chamber

##### 10.2.7.2.1	General

This method measures the TAE in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.2.3.2.1-1.

##### 10.2.7.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.1.2 to ensure that the SNR at the measurement equipment input is appropriate for the measurement of the requirement and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.7.2.3	Procedure

Reference procedure in subclause 10.2.3.1.3 where in step 6 the appropriate measurement time alignment error for the testing condition. In this case, however testing should be carried out in the OTA conformance reference direction, not the beam peak direction of the OTA peak directions set reference direction.

##### 10.2.7.2.4	MU assessment

The uncertainty in the power accuracy of the OTA test chamber will not affect the TAE measurement uncertainty.

Possible phase variation in the chamber due to variation in the signal BW is not significant to affect the TAE measurement uncertainty.

The conducted MU value is adopted for the OTA MU.

#### 10.2.7.3	CATR

##### 10.2.7.3.1	General

The Compact Antenna Test Range (CATR) uses the DUT which radiates a wavefront to a range antenna reflector which will then collimate the radiated spherical wavefront into a feed antenna.  The sufficient separation between the DUT and the receiver (feed antenna shown in figure 10.2.3.3.1-1) so that the emanating spherical wave reaches nearly plane phase fronts from transmitter to receiver. The DUT transmits a wavefront that will illuminate the range antenna reflector, which will then reflect the transmitted energy into the feed antenna.  The range feed antenna is connected to a vector network analyzer or other equivalent test equipment.

##### 10.2.7.3.2	Calibration

Calibration of the CATR test setup should be carried out in the same manner as described for the EVM procedure in subclause 10.2.6.3.2.

##### 10.2.7.3.3	Procedure

Reference procedure in subclause 10.2.3.3.3 where in step 6 the appropriate measurement parameter is the time alignment error. In this case, however testing should be carried out in the OTA conformance reference direction, not the beam peak direction of the OTA peak directions set reference direction.

Figure 10.2.7.3.3-1: CATR measurement system setup for Timing Alignment Error

1)	Align DUT with boresight of the range antenna.

2)	Configure DUT to transmit signals carrying reference signals. All beams must be pointed at the same direction.

Note:	The transmitted signals should represent the beam configuration with the lowest number of beams. Each beam should be identifiable using a reference signal.

For an AAS BS declared to be capable of single carrier operation only, set DUT to transmit according to rated beam EIRP level.

If the AAS BS supports intra band contiguous or non-contiguous Carrier Aggregation, set DUT to transmit using the applicable test configuration and corresponding power setting specified in TS 37.145-2 subclauses 4.10 and 4.11.

If the AAS BS supports inter band carrier aggregation, set DUT to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in TS 37.145-2 subclauses 4.10 and 4.11.

3)	Measure the time alignment error between the reference signals with signal/spectrum analyser or equivalent equipment.

##### 10.2.7.3.4	MU assessment

The uncertainty in the power accuracy of the OTA test chamber will not affect the TAE measurement uncertainty.

Possible phase variation in the chamber due to variation in the signal BW is not significant to affect the TAE measurement uncertainty.

The conducted MU value is adopted for the OTA MU.

#### 10.2.7.4	Near Field

##### 10.2.7.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA Time Alignment Error type of measurements, NF to FF transform is not needed. TAE is measured in Near Field for the declared direction.

##### 10.2.7.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.3.4.2 to ensure that the SNR at the measurement equipment input is appropriate and the received signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.7.4.3	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

1)	Align the AAS BS with (Theta, Phi) angles corresponding to the declared conformance direction to be measured

2)	Configure DUT to transmit signals carrying reference signals. All beams must be pointed at the same direction.

Note:	The transmitted signals should represent the beam configuration with the lowest number of beams. Each beam should be identifiable using a reference signal.

For an AAS BS declared to be capable of single carrier operation only, set DUT to transmit according to rated beam EIRP level .

If the AAS BS supports intra band contiguous or non-contiguous Carrier Aggregation, set DUT to transmit using the applicable test configuration and corresponding power setting specified in TS 37.145-2 subclauses 4.10 and 4.11.

If the AAS BS supports inter band carrier aggregation, set DUT to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in TS 37.145-2 subclauses 4.10 and 4.11.

3)	Measure the time alignment error between the reference signals with signal/spectrum analyser or equivalent equipment.

##### 10.2.7.4.4	MU assessment

Refer to clause 10.2.7.3.4.

#### 10.2.7.5	Summary

The uncertainty in the power accuracy of the OTA test chamber will not affect the TAE measurement uncertainty.

Possible phase variation in the chamber due to variation in the signal BW is not significant to affect the TAE measurement uncertainty.

The conducted MU value is adopted for the OTA MU, and is ±25 ns.

### 10.2.8	OTA Occupied bandwidth

#### 10.2.8.1	General

Occupied BW is specified in Hz. It is defined as:

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power

It is measured by taking a number of narrow band power measurements across the modulated BW and finding the half power level and hence the frequency points which correspond to the half power level. The measurement is effectively a differential measurement as total power and hence half power is found from the measured data.

The only effect the measurement chamber would have on the accuracy of the measurement were if the frequency response of the chamber were not flat, however a 40MHZ BW is unlikely to introduce any significant frequency ripple in a RF chamber (40MHz is 4% of 1GHz) so this effect can be ignored.

The largest error it is due to the number of measurement points taken, looking at the gap between points compared to the conducted MU:

Table 10.2.8.1-1: Occupied BW conducted MU

| Channel bandwidth BWChannel [MHz] | 0.2 | 1.4 | 3 | 5 | 10 | 15 | 20 | >20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Span [MHz] | 0.4 | 10 | 10 | 10 | 20 | 30 | 40 |  |
| Minimum number of measurement points | 400 | 1429 | 667 | 400 | 400 | 400 | 400 |  |
| gap between samples (MHz) | 0.001 | 0.01 | 0.01 | 0.025 | 0.05 | 0.075 | 0.1 | 0.1 |
| conducted MU (MHz) |  | 0.03 | 0.03 | 0.1 | 0.1 | 0.3 | 0.3 | 0.3 |

The MU is 3 to 4 time larger than the distance between the measurement points. Which allows the estimation of the 3dB point to be 1.5 to 2 steps out on each side of the modulated band width.

As the OTA system will not introduce any additional frequency error and it will not introduce any additional differential amplitude error, the MU for the OTA measurement should be the same as for the conducted requirement.

It should be noted that the signal power level at the bandwidth edge required for meeting the OBW requirement is in general at least 20dB higher than the level that is required to meet unwanted emissions requirements. Thus there is very substantial room to accommodate measurement uncertainty for the OBW requirement.

#### 10.2.8.2	In-door anechoic chamber

##### 10.2.8.2.1	General

This method measures the occupied bandwidth in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.2.3.2.1-1.

##### 10.2.8.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2 to ensure that the SNR at the measurement equipment input is appropriate for the measurement of the requirement and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.8.2.3	Procedure

Reference procedure in subclause 10.2.3.1.3 where in step 6 the appropriate measurements needed for calculating occupied bandwidth.

##### 10.2.8.2.4	MU assessment

###### 10.2.8.2.4.1	MU Budget

###### 10.2.8.2.4.2	MU Value

#### 10.2.8.3	CATR

##### 10.2.8.3.1	General

In a Compact Antenna Test Range (CATR) the DUT which radiates a wave front to a range antenna reflector which will then collimate the radiated spherical wave front into a feed antenna.  There is sufficient separation between the DUT and the receiver (feed antenna shown in figure 10.2.3.3.1-1) so that the emanating spherical wave achieves nearly plane phase fronts from transmitter to receiver. The range feed antenna is connected to a vector network analyzer or other equivalent test equipment.

Test method limitations and scope

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is considered.

##### 10.2.8.3.2	Calibration

Calibration of the CATR test setup should be carried out in the same manner as described for the EVM procedure in subclause 10.2.6.3.2.

##### 10.2.8.3.3	Procedure

1)	Configure transmitter according to the manufacturer's declared EIRP at rated TRP.

2)	Set the AAS BS to transmit the test signal according to applicable test model for the tested carrier BW.

3)	Align DUT such that beam peak direction of probe antenna is aligned with the reference direction within the OTA coverage range.

4)	Measure the Occupied BW for a single carrier positioned at M channel.

##### 10.2.8.3.4	MU assessment

###### 10.2.8.3.4.1	MU Budget

###### 10.2.8.3.4.2	MU Value

#### 10.2.8.4	Near field

##### 10.2.8.4.1	General

The system is depicted in section 10.2.3.4.1 In case of OTA Occupied BW type of measurements, NF to FF transform is not needed. Occupied BW is measured in Near Field for the declared direction

##### 10.2.8.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in 10.2.3.4.2. to ensure that the SNR at the measurement equipment input is appropriate and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.8.4.3	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

1)	Align the AAS BS with (Theta, Phi) angles corresponding to the declared beam peak direction to be measured

2)	Configure TX branch and carrier according to the manufacturer's declared rated output power

3)	Set the AAS BS to transmit the test signal according to the appropriate test model.

4)	Measure Occupied BW of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument).

5)	Repeat steps 3-5 for other appropriate test models.

The Occupied Bandwidth of each carrier for different modulation schemes on (HS-)PDSCH to be less than the limits defined in 3GPP TS 37.105 [3], subclause 9.6.

For conformance tests, Occupied BW shall be measured at maximum power setting.

##### 10.2.8.4.4	MU assessment

###### 10.2.8.4.4.1	MU Budget

###### 10.2.8.4.4.2	MU Value

#### 10.2.8.5	Summary

The OTA occupied BW MU is the same as the conducted MU , as follows:

1.4MHz, 3MHz Channel BW: 30kHz

5MHz, 10MHz Channel BW: 100kHz

15MHz, ≥20MHz: Channel BW: 300kHz

### 10.2.9	EIRP accuracy – extreme conditions

#### 10.2.9.1	General

For the direct far field method (see sub-clause 5.2.3.4) the MU budget is very similar to the existing MU budget for the EIRP accuracy requirement in 3GPP TR 37.842. However there are a number of additional sources of uncertainty due to the environmental enclosure that need to be added to the budget.

#### 10.2.9.2	In-door anechoic chamber

##### 10.2.9.2.1	General

This method measures places the AAS BS under test inside a RF transparent environmentally sealed enclosure so that the DUT temperature can be controlled whilst the result of the OTA chamber facility is at nominal temperature.

The separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.2.9.2.1-1.

Figure 10.2.9.2.1-1: Indoor Anechoic Chamber measurement system setup for EIRP

##### 10.2.9.2.2	Calibration

Calibration shall be done with the same procedure shown in 10.2.3.2.2 to ensure that the SNR at the measurement equipment input is appropriate for the measurement of the requirement and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.9.2.3	Procedure

Reference procedure in subclause 10.2.3.2.3  where in step 6 the measured test parameter is the EIRP.

##### 10.2.9.2.4	MU assessment

###### 10.2.9.2.4.1	MU Budget

Table 10.2.9.2.4.1-1: Indoor anechoic chamber uncertainty contributions
for extreme EIRP accuracy measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. | E1-2 |
| 32 | Quality of quiet zone (extreme) | E1-20 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E1-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E1-5 |
| 6 | Phase curvature | E1-6 |
| 7 | Uncertainty of the RF power measurement equipment | F1 |
| 8 | Impedance mismatch in the receiving chain | E1-8 |
| 9 | Random uncertainty | E1.9 |
| 30 | radome loss variation | E1.22 |
| 31 | wet radome loss variation | E1.21 |
| 33 | Change in absorber behaviour | E1.23 |
| 34 | Test system frequency flatness | E1.19 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | E1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E1-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer | E1-12 |
| 13 | Quality of quiet zone | E1-3 |
| 14 | Polarization mismatch for reference antenna | E1-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E1-5 |
| 16 | Phase curvature | E1-6 |
| 17 | Uncertainty of the Network Analyzer | F1 |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E1-15 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | F1 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E1-18 |

###### 10.2.9.2.4.2	MU Value

Table 10.2.9.2.4.2-1: Indoor anechoic chamber uncertainty assessment for extreme EIRP accuracy measurement

| Indoor anechoic |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |
|  |  | f<3 GHz | 3<f<6 GHz |  |  |  | f≤3 GHz | 3<f≤6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | 1.73 | 1 | 0.17 | 0.17 |
| 32 | Quality of quiet zone (extreme) | 0.6 | 0.6 | Gaussian | 1.00 | 1 | 0.60 | 0.60 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.14 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | 1.41 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 |
| 30 | radome loss variation | 0.4 | 0.4 | Rectangular | 1.73 | 1 | 0.23 | 0.23 |
| 31 | wet radome loss variation | 0.95 | 0.95 | Normal | 1 | 1 | 0.95 | 0.95 |
| 33 | Change in absorber behaviour | 0.1 | 0.1 | Normal | 1.00 | 1 | 0.10 | 0.10 |
| 34 | Frequency flatness | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.035 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.006 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.035 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.100 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.006 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.000 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.050 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.2 | Gaussian | 1.00 | 1 | 0.13 | 0.200 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.029 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1.00 | 1 | 0.06 | 0.060 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.029 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.248 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.000 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.25 | 1.29 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.46 | 2.53 |

#### 10.2.9.3	CATR

##### 10.2.9.3.1	General

This method measures places the AAS BS under test inside a RF transparent environmentally sealed enclosure so that the DUT temperature can be controlled whilst the result of the OTA chamber facility is at nominal temperature.

The Compact Antenna Test Range (CATR) uses the DUT which radiates a wave front to a range antenna reflector which will then collimate the radiated spherical wave front into a feed antenna.  The sufficient separation between the DUT and the receiver (feed antenna shown in figure 10.3.1.1.3.1-1) so that the emanating spherical wave reaches nearly plane phase fronts from transmitter to receiver. The DUT transmits a wave front that will illuminate the range antenna reflector, which will then reflect the transmitted energy into the feed antenna.  The range feed antenna is connected to a vector network analyzer or other equivalent test equipment.

Figure 10.2.9.3.1-1: CATR measurement system setup for extreme conditions EIRP accuracy

##### 10.2.9.3.2	Calibration

Calibration shall be done with the same procedure shown in 10.2.3.3.2 to ensure that the SNR at the measurement equipment input is appropriate for the measurement of the requirement and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.9.3.3	Procedure

Reference procedure in subclause 10.2.3.3.3  where in step 6 the measured test parameter is the EIRP.

##### 10.2.9.3.4	MU assessment

###### 10.2.9.3.4.1	MU Budget

Table 10.2.9.3.4.1-1: CATR uncertainty contributions for extreme EIRP accuracy measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | RF power measurement equipment | F1 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 32 | QZ ripple DUT (extreme) | E2-17 |
| 30 | radome loss variation | E2-19 |
| 31 | wet radome loss variation | E2-18 |
| 33 | Change in absorber behaviour | E2-20 |
| 34 | Test system frequency flatness | E2-16 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | F1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | F1 |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |

###### 10.2.9.3.4.2	MU Value

Table 10.2.9.3.4.2-1: CATR uncertainty assessment for extreme EIRP accuracy measurement

| CATR |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value | Uncertainty value | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] | Standard uncertainty ui [dB] |
|  |  | f<3 GHz | 3<f<6 GHz |  |  |  | f≤3 GHz | 3<f≤6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | Exp. Normal | 2 | 1 | 0 | 0 |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.6 | 0.6 | Normal | 1 | 1 | 0.6 | 0.6 |
| 30 | radome loss variation | 0.4 | 0.4 | Rectangular | 1.73 | 1 | 0.23 | 0.23 |
| 31 | wet radome loss variation | 0.95 | 0.95 | Normal | 1 | 1 | 0.95 | 0.95 |
| 33 | Change in absorber behaviour | 0.1 | 0.1 | Normal | 1 | 1 | 0.10 | 0.10 |
| 34 | Frequency flatness | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.2 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | 1.41 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.00 | 0.00 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | 1.41 | 1 | 0.02 | 0.02 |
| 11 | SGH Calibration uncertainty | 0.5 | 0.433 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. Normal | 2 | 1 | 0.00 | 0.00 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. Normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | 1.41 | 1 | 0.03 | 0.03 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.01 | 0.01 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | 1.73 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.28 | 1.32 |
| Expanded uncertainty (1.96σ – confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.51 | 2.58 |

#### 10.2.9.3	Summary

The different OTA test chamber MU are summarised below:

Table 10.2.9.3-1: Test system specific measurement uncertainty values for the EIRP test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 2.46 | 2.53 |
| Compact Antenna Test Range | 2.46 | 2.58 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 2.5 | 2.6 |

## 10.3	Measurement uncertainty for Rx Directional requirements

### 10.3.1	General

Rx in-band directional requirements all use a wanted signal at a specified EIS power level and throughput/BER quality metric. Requirements which also include an in-band interferer have the wanted signal and the interferer coming from the same direction within the specified RoAoA.

For the derivation of the measurement uncertainty for direction requirements, the approach already defined in the TR 37.842 [4] is reused as much as possible.

Rx directional requirements use the same OTA test chambers and corresponding calibration and test procedures as the existing EIS accuracy requirement.

Requirements which require a wanted signal and an interferer use the conducted MU calculations in TS 36.141 [22] as guidance and are modified to take into account the OTA chamber.

Limitations and scope for each test method identified in TR 37.842[4] are the same when used for the OTA Rx in-band directional requirements.

Currently the near field test range is included as a test method for the hybrid AAS BS EIS requirement in TR 37.842, however there are some limitations on the use of that the AAS BS must not have any BB beam forming. Considering that OTA AAS BS must have 8TRX the probability of it meeting the restrictive conditions is small. In addition there are some issues surrounding the interference requirements when testing in the near field as the wanted and interfering signal may experience different beam forming. At this stage procedures and MU budgets are not provided for near field Rx directional requirements however if the issues are solved and the MUs are within the agreed values in this section use of the near field test method is not precluded in the future.

For the derivation of the measurement uncertainty for direction requirements, the approach already defined in the TR 37.842 [4] should be reused as much as possible.

Rx directional requirements where possible can use the same OTA test chambers and corresponding calibration and test procedures as the existing EIS accuracy requirement.  Most RX directional requirements are based upon the EIS test procedures in TR 37.842 [4] where specific test models are referenced.  However, for simplicity since requirements here do not all use the same test model the procedures have been kept general by using “receiving test signal” where the appropriate test model is found in TS 37.105 [3]

### 10.3.2	OTA sensitivity

The OTA sensitivity MU was derived in TR 37.842 [4] as follows:

Table 10.3.2-1: Test system specific measurement uncertainty values for the EIS test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1.22 | 1.25 |
| Compact Antenna Test Range | 1.33 | 1.40 |
| One Dimensional Compact Range Chamber | 1.29 | 1.43 |
| Near Field Test Range | 1.24 | 1.24 |
| Common maximum accepted test system uncertainty | 1.3 | 1.4 |

### 10.3.3	OTA reference sensitivity

The OTA REFSENS requirement is intended to ensure the OTA reference sensitivity level for a declared OTA REFSENS RoAoA.

The OTA reference sensitivity power level EISREFSENS is the mean power received at the radiated interface at which a reference performance requirement shall be met for a specified reference measurement channel.

The measurement methods, along with the corresponding calibration, procedure and MU assessment, are the same as those for the OTA sensitivity requirement in subclause 10.3.2, except that the required level is EISREFSENS in the measurement procedure.

### 10.3.4	OTA Dynamic range

#### 10.3.4.1	General

The OTA dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal inside the received channel bandwidth or the capability of receiving high level of wanted signal.

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

#### 10.3.4.2	In-door anechoic chamber

##### 10.3.4.2.1	General

The measurement system setup is as depicted in figure 10.3.4.2.1-1.

Figure 10.3.4.2.1-1: In-door Anechoic Chamber measurement system setup for dynamic range

##### 10.3.4.2.2	Calibration

Calibration shall be done with the procedure shown in subclause 10.3.2.1.1.2 in TR 37.842 [4] for the wanted and AWGN interfering signals with power combiner/coupler.

LWanted_cal, A→D:  Calibration value for wanted signal between A and D in figure 10.3.4.2.1-1.

LAWGN_cal, A→E:  Calibration value for AWGN signal between A and E in figure 10.3.4.2.1-1.

##### 10.3.4.2.3	Procedure

The measurement procedure is the same as those for the OTA sensitivity requirement in subclause 10.3.2.1.1.2 in TR 37.842 [4], except that step 11 is modified as follow.

11)	Set the test signal mean power at the RF signal source generator for wanted signal as the required level plus LWanted_cal, A→D and the reference measurement channel. Set the test signal mean power at the RF signal source generator for AWGN interfering signal as the required level plus LAWGN_cal, A→E and at the same frequency as wanted signal.

##### 10.3.4.2.4	MU assessment

The MU for receiver dynamic range requirement is not impacted by OTA chamber related uncertainties; this is further discussed in subclause 10.3.4.4.

#### 10.3.4.3	CATR

##### 10.3.4.3.1	General

The measurement system setup is as depicted in figure 10.3.4.3.1-1.

Figure 10.3.4.3.1-1: CATR measurement system setup for dynamic range

##### 10.3.4.3.2	Calibration

Calibration shall be done with the procedure shown in subclause 10.3.2.1.2.2 in TR 37.842 [4] for the wanted and AWGN interfering signals with power combiner/coupler.

LWanted_cal, A→B:  Calibration value for wanted signal between A and B in figure 10.3.4.3.1-1.

LAWGN_cal, A→E:  Calibration value for AWGN interfering signal between A and E in figure 10.3.4.3.1-1.

##### 10.3.4.3.3	Procedure

1)	Set up AAS BS in place of SGH from calibration stage. Align AAS BS with boresight of transmitting antenna.

2)	Configure RF signal generator for wanted signal to set the signal at the required level plus LA→B and at reference measurement channel.

3)	Configure RF signal generator for AWGN interferer to set the signal at the required level plus LA→E and at the same frequency as wanted signal.

4)	Measure the throughput of the wanted signal according to annex E of the 3GPP TS 36.141 [22].

5)	Repeat steps 2-4 for all conformance test bands and directions.

##### 10.3.4.3.4	MU assessment

The MU for receiver dynamic range requirement is not impacted by OTA chamber related uncertainties; this is further discussed in subclause 10.3.4.4.

#### 10.3.4.4	Summary

The receiver dynamic range requirement provides both a wanted signal and an in-channel AWGN interferer to the BS input. Both the wanted signal and interferer power levels are well above the receiver noise floor, to the extent that the SNR in the receiver is dependent only on the transmitter signal to AWGN level and is independent of the receiver noise level.

Small variations of the receiver power level due to uncertainty factors in the OTA environment will impact both the wanted signal and AWGN equally, and hence do not impact the SNR experienced by the receiver or change the measurement result. Thus, the receiver dynamic range requirement is not impacted by OTA chamber related uncertainties.

In 36.141 [22], the MU for receiver dynamic range is taken as the uncertainty in achieving the correct SNR at the test equipment transmitter. Considering the above argumentation that OTA chamber aspects will not impact MU for receiver dynamic range, the same MU of 0.3dB as for the conducted case can be assumed.

### 10.3.5	OTA adjacent channel selectivity, general blocking and narrowband blocking

#### 10.3.5.1	General

{editors note: general description of the test requirement placed here – non chamber specific}

#### 10.3.5.2	In-door anechoic chamber

##### 10.3.5.2.1	General

This method measures the adjacent channel selectivity, narrow-band blocking, and general blocking in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the transmitting antenna of no less than 2D2/λ, where D is the largest dimension of the transmitting antenna and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.5.2.1-1.

Figure 10.3.5.2.1-1: In-door Anechoic Chamber measurement system setup for  adjacent channel selectivity, general blocking and narrowband blocking

##### 10.3.5.2.2	Calibration

Calibration shall be done with the procedure shown in 10.3.2.4.2 for the frequencies of wanted and unwanted signals with power combiner/coupler. Calculate the calibration value for wanted and unwanted signal from A to D as well as from A to E.

LWanted_cal, A→D:  Calibration value for wanted signal between A and D in figure 10.3.5.2.1-1.

LUnanted_cal, A→E:  Calibration value for unwanted signal between A and E in figure 10.3.5.2.1-1.

##### 10.3.5.2.3	Procedure

1)	Uninstall the reference antenna and install the AAS BS with its manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with testing system.

2)	Set the AAS BS to be satisfied with the RoAoA desired by the requirement covering conformance testing receiving direction.

3)	Rotate the AAS BS to make the conformance testing receiving direction aligned with the boresight of the reference antenna at the calibration stage.

4)	For FDD AAS BS start BS transmission at the required condition.

5)	Set the test signal mean power at the RF signal source generator for wanted signal as the required level plus LWanted_cal, A→D  and the reference measurement channel.

6) Set the test signal mean power at the RF signal source generator for unwanted signal as the required level plus LUnwanted_cal, A→E  and the interference signal channel.

7)	Measure the throughput according to annex E of the 3GPP TS 36.141 [22].

8)	Repeat the above steps 1)~6) per conformance testing direction.

##### 10.3.5.2.4	MU assessment

###### 10.3.5.2.4.1	MU Budget

The MU is estimated based on MU in the conducted specification 36.141, the MU for EIS in section 10.3.3 and an estimated uncertainty for matching of test equipment and DUT or test antenna.

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

The uncertainty budget behind the EIS MU is captured in section 10.3.3.  The 2 sigma matching uncertainty is 0.196dB for   f ≦ 3GHz and 0.314dBfor  3GHz < f ≦ 4.2 GHz .

The ACLR effect from the interference signal is assumed to be 0.4dB, as in 36.141. This is under the assumption that no additional PA is needed for the test signal in the OTA test setup.

###### 10.3.5.2.4.2	MU Value

The MU for ACS is calculated as follows. Note that all uncertainties in the table are 1.96σ values.

Table 10.3.5.2.4.2-1: Test system uncertainty of ACS for in-door anechoic chamber

| Frequency range | f ≤ 3.0GHz |  | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- | --- |
| MUwanted_conducted | ±0.7 dB |  | ±1 dB |  |
| MUinterfer_conducted | ±0.7 dB |  | ±1 dB |  |
| MUmatching_conducted | ±0.196 dB |  | ±0.314 dB |  |
| MUwanted | ±0.672dB |  | ±0.949 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
| MUinterfer | ±0.672 dB |  | ±0.949 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
| MUEIS_OTA | 1.22 dB |  | 1.25 dB |  |
| MUTestEquipment_OTA | 0.9 dB |  | 0.9 dB |  |
| MUmatching_OTA | ±0.196 dB |  | ±0.314 dB |  |
| MUchamber_OTA | ±0.8 dB |  | ±0.804 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
| ACLR effect | 0.4 dB |  |  |  |
| MUACS_OTA | 1.7 dB | 2.00 dB |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |

#### 10.3.5.3	CATR

##### 10.3.5.3.1	General

This method measures the adjacent channel selectivity, narrow-band blocking, and general blocking in compact antenna test range with a sufficient separation between reference point of the AAS BS and the transmitting antenna by the range antenna reflector. The measurement system setup is as depicted in figure 10.3.5.3.1-1.

Figure 10.3.5.3.1-1: CATR measurement system setup for adjacent channel selectivity, general blocking and narrowband blocking

##### 10.3.5.3.2	Calibration

Calibration shall be done with the procedure shown in 37.842 [4] subclause 10.3.1.1.3.3 for the frequencies of wanted and interfering signals with power combiner/coupler. Calculate the calibration value for wanted and interfering signal from A to B as well as from A to E.

LWanted_cal, A→B:  Calibration value for wanted signal between A and B in figure 10.3.5.3.1-1.

Linterferer_cal, A→E:  Calibration value for interfering signal between A and E in figure 10.3.5.3.1-1.

##### 10.3.5.3.3	Procedure

1)	Set up AAS BS in place of SGH from calibration stage. Align AAS BS with boresight of transmitting antenna.

2)	Configure RF signal generator for wanted signal to set the signal at the required level plus LA→B and at reference measurement channel.

3)	Configure RF signal generator for interferer to set the signal at the required level plus LA→E and at reference measurement channel. The interferer’s centre frequency offset should be swept as required.

4)	Measure the throughput of the wanted signal according to annex E of the 3GPP TS 36.141 [22].

5)	Repeat steps 2-4 for all conformance test directions.

6)	Repeat steps above per involved band for multi-band RIB(s).

##### 10.3.5.3.4	MU assessment

###### 10.3.5.3.4.1	MU Budget

The MU is estimated based on MU in the conducted specification 36.141, the MU for EIS in section 10.3.3 and an estimated uncertainty for matching of test equipment and DUT or test antenna.

Table 10.3.5.3.4.1-1: CATR uncertainty budget format for mismatch error

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 2 | Standing wave between DUT and test range antenna | 3GPP 37.842 [4] B 2-2 |

###### 10.3.5.3.4.2	MU Value

The 2 sigma matching uncertainty is 0.294 dB for   f ≦ 3GHz and 0.294 dB for  3GHz < f ≦ 4.2 GHz .

Test system uncertainty is obtained by follwing equations:

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

And we assume  MUmatching_conducted= MUmatching_OTA

Test system uncertainty for ACS, narrow band blocking, and general band blocking is shown in Table 10.3.5.3.4.2-1. Note that all uncertainties in table are 1.96σ values.

Table 10.3.5.3.4.2-1: Test system uncertainty of ACS, narrow band blocking, and general band blocking for CATR.

| Frequency range |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- | --- |
| ACS and narrow band blocking | MUwanted_conducted | ±0.7 dB | ±1 dB |  |
|  | MUinterfer_conducted | ±0.7 dB | ±1 dB |  |
|  | MUmatching_conducted | ±0.294 dB | ±0.294 dB |  |
|  | MUwanted | ±0.635dB | ±0.956 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer | ±0.635 dB | ±0.956 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.33 dB | 1.40 dB |  |
|  | MUTestEquipiment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.294 dB | ±0.294 dB |  |
|  | MUchamber_OTA | ±0.933 dB | ±1.03 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |
|  | MUACS_OTA | 1.7 dB | 2.1 dB |  |

#### 10.3.5.4	Summary

The different OTA test chamber MU are summarised below:

Table 10.3.5.4-1: Test system specific measurement uncertainty values for the OTA adjacent channel selectivity, general blocking and narrowband blocking

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1.7 | 2.0 |
| Compact Antenna Test Range | 1.7 | 2.1 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 1.7 | 2.1 |

### 10.3.6	OTA receiver intermodulation

#### 10.3.6.1	General

{editors note: general description of the test requirement placed here – non chamber specific}

#### 10.3.6.2	In-door anechoic chamber

##### 10.3.6.2.1	General

This method measures the receiver intermodulation in an in-door anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the transmitting antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of transmitting antenna and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.6.2.1-1.

Figure 10.3.6.2.1-1: In-door Anechoic Chamber measurement system setup for  OTA receiver intermodulation

##### 10.3.6.2.2	Calibration

Calibration shall be done with the procedure shown in 37.842 [4] subclause 10.3.1.1.2.3 for the frequencies of wanted and unwanted signals with power combiner/coupler. Calculate the calibration value for wanted and unwanted signal from A to D, from A to E, and from A to F.

LWanted_cal, A→D:  Calibration value for wanted signal between A and D in figure 10.3.6.2.1-1.

LUnanted_cal, A→E:  Calibration value for unwanted signal between A and E in figure 10.3.6.2.1-1.

LUnanted_cal, A→F:  Calibration value for unwanted signal between A and F in figure 10.3.6.2.1-1.

##### 10.3.6.2.3	Procedure

1)	Uninstall the reference antenna and install the AAS BS with its manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with testing system.

2)	Set the AAS BS to be satisfied with the RoAoA desired by the requirement covering conformance testing receiving direction.

3)	Rotate the AAS BS to make the conformance testing receiving direction aligned with the boresight  of the reference antenna at the calibration stage.

4)	For FDD AAS BS start BS transmission at the required condition.

5)	Set the test signal mean power at the RF signal source generator for wanted signal as the required level plus LWanted_cal, A→D  and the reference measurement channel.

6)	Set the test signal mean power at the RF signal source generators for unwanted signal as the required level plus LUnwanted_cal, A→E  or plus LUnwanted_cal, A→F and the interference signal channel.

7)	Measure the throughput according to annex E of the 3GPP TS 36.141 [22].

8)	Repeat the above steps 1)~6) per conformance testing direction.

##### 10.3.6.2.4	MU assessment

###### 10.3.6.2.4.1	MU Budget

The MU is estimated based on MU in the conducted specification 36.141, the MU for EIS in section 10.3.3 and an estimated uncertainty for matching of test equipment and DUT or test antenna.

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

The uncertainty budget behind the EIS MU is captured in section 10.3.3.  The 2 sigma matching uncertainty is 0.196dB for   f ≦ 3GHz and 0.314dB for  3GHz < f ≦ 4.2 GHz .

The ACLR effect from the interference signal is assumed to be 0.4dB, as in 36.141 [22]. This is under the assumption that no additional PA is needed for the test signal in the OTA test setup.

###### 10.3.6.2.4.2	MU Value

The MU for RX IM is calculated as follows. Note that all uncertainties in the table are 1.96σ values.

Table 10.3.6.2.4.2-2: Test system uncertainty of RX IM for in-door anechoic chamber

| Frequency range | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- |
| MUwanted_conducted | ±0.7 dB | ±1.0 dB |  |
| MUCWinterfer_conducted | ±0.5 dB | ±0.7 dB |  |
| MUMODinterfer_conducted | ±0.7 dB | ±1.0 dB |  |
| MUmatching_conducted | ±0.196 dB | ±0.314 dB |  |
| MUwanted | ±0.672dB | ±0.949 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
| MUCWinterfer | ±0.460 dB | ±0.626 dB | MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2 |
| MUMODinterfer | ±0.672dB | ±0.949 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
| MUEIS_OTA | 1.22 dB | 1.25 dB |  |
| MUTestEquipment_OTA | 0.9 dB | 0.9 dB |  |
| MUmatching_OTA | ±0.196 dB | ±0.314 dB |  |
| MUchamber_OTA | ±0.8 dB | ±0.807 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
| ACLR effect | 0.4 dB |  |  |
| MURXIM_OTA | 2.0 dB | 2.5 dB | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

#### 10.3.6.3	CATR

##### 10.3.6.3.1	General

This method measures the receiver intermodulation in compact antenna test range with a sufficient separation between reference point of the AAS BS and the transmitting antenna by the range antenna reflector. The measurement system setup is as depicted in figure 10.3.6.3.1-1.

Figure 10.3.6.3.1-1: CATR measurement system setup for receiver intermodulation

##### 10.3.6.3.2	Calibration

Calibration shall be done with the procedure shown in 37.842 [4] subclause 10.3.1.1.3.3 for the frequencies of wanted and 2 interfering signals with power combiners/couplers. Calculate the calibration value for wanted and interfering signals from A to B as well as from A to E and A to H.

LWanted_cal, A→B:  Calibration value for wanted signal between A and B in figure 10.3.6.3.1-1.

Linterferer1_cal, A→E:  Calibration value for interfering signal between A and E in figure 10.3.6.3.1-1.

Linterferer2_cal, A→H:  Calibration value for interfering signal between A and H in figure 10.3.6.3.1-1.

##### 10.3.6.3.3	Procedure

1)	Set up AAS BS in place of SGH from calibration stage. Align AAS BS with boresight of transmitting antenna.

2)	Configure RF signal generator for wanted signal to set the signal at the required level plus LA→B and at reference measurement channel.

3)	Configure RF signal generator for interferer1 to set a modulated signal at the required level plus LA→E and at reference measurement channel. The interferer’s centre frequency offset should also fulfil the requirement.

4)	Configure RF signal generator for interferer2 to set a CW signal at the required level plus LA→H and at reference measurement channel. The interferer’s centre frequency offset should also fulfil the requirement.

5)	Measure the BER or throughput of the wanted signal according to annex E of the 3GPP TS 36.141 [22].

6)	Repeat steps all above per involved band for multi-band RIB(s).

##### 10.3.6.3.4	MU assessment

###### 10.3.6.3.4.1	MU Budget

The MU is estimated based on MU in the conducted specification 36.141, the MU for EIS in section 10.3.3 and an estimated uncertainty for matching of test equipment and DUT or test antenna is in section 10.3.5.

###### 10.3.6.3.4.2	MU Value

Test system uncertainty is obtained by follwing equations:

MUACS_OTA=		MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUMODinterfer=MUMODinterfer_conducted2-MUmatching_conducted2

MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

And we assume MUmatching_conducted= MUmatching_OTA

Test system uncertainty for receiver intermodulation is shown in Table 10.3.6.3.4.2-1. Note that all uncertainties in table are 1.96σ values.

Table 10.3.6.3.4.2-1: Test system uncertainty of receiver intermodulation for CATR

| Frequency range |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- | --- |
| RX IM | MUwanted_conducted | ±0.7 dB | ±1.0 dB |  |
|  | MUCWinterfer_conducted | ±0.5 dB | ±0.7 dB |  |
|  | MUMODinterfer_conducted | ±0.7 dB | ±1.0 dB |  |
|  | MUmatching_conducted | ±0.294 dB | ±0.294 dB |  |
|  | MUwanted | ±0.635 dB | ±0.956 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUCWinterfer | ±0.404 dB | ±0.635 dB | MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2 |
|  | MUMODinterfer | ±0.635 dB | ±0.956 dB | MUMODinterfer=MUMODinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.33 dB | 1.40 dB |  |
|  | MUTestEquipiment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.294 dB | ±0.294 dB |  |
|  | MUchamber_OTA | ±0.933 dB | ±1.03 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  |  |
|  | MURXIM_OTA | 2.0 dB | 2.6 dB | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

#### 10.3.6.4	Summary

The different OTA test chamber MU are summarised below:

Table 10.3.6.4-1: Test system specific measurement uncertainty values for the OTA receiver intermodulation

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 2.0 | 2.5 |
| Compact Antenna Test Range | 2.0 | 2.6 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 2.0 | 2.6 |

### 10.3.7	OTA in-channel selectivity

#### 10.3.7.1	General

{editors note: general description of the test requirement placed here – non chamber specific}

#### 10.3.7.2	In-door anechoic chamber

##### 10.3.7.2.1	General

##### 10.3.7.2.2	Calibration

The procedure for calibration for RX ICS is basically the same as for RX ACS, as described in section 10.5.2.3.1, however calibration must be carried out for wanted signal and interferer signals appropriate to ICS.

##### 10.3.7.2.3	Procedure

The procedure for RX ICS is based on the RX ACS procedure, described in section 10.3.5.2.3.1.

##### 10.3.7.2.4	MU assessment

###### 10.3.7.2.4.1	MU Budget

The MU budget for ICS is the same as that for RX ACS, described in section 10.3.5.2.4.1.

###### 10.3.7.2.4.2	MU Value

The MU value for ICS is the same as that for RX ACS, described in section 10.3.5.2.4.2.

#### 10.3.7.3	CATR

##### 10.3.7.3.1	General

This method measures the in channel selectivity in compact antenna test range with a sufficient separation between reference point of the AAS BS and the transmitting antenna by the range antenna reflector. The measurement system setup is as depicted in figure 10.3.7.3.1-1.

Figure 10.3.7.3.1-1: CATR measurement system setup for in channel selectivity

##### 10.3.7.3.2	Calibration

Calibration shall be done with the procedure shown in 37.842 [4] subclause 10.3.1.1.3.3 with power combiner/coupler. Calculate the calibration value for wanted and interfering signal from A to B as well as from A to D.

LWanted_cal, A→B:  Calibration value for wanted signal between A and B in figure 10.3.7.3.1-1.

Linterferer_cal, A→D:  Calibration value for interfering signal between A and D in figure 10.3.7.3.1-1.

##### 10.3.7.3.3	Procedure

1)	Set up AAS BS in place of SGH from calibration stage. Align AAS BS with boresight of transmitting antenna.

2)	Configure RF signal generator for wanted signal to set it at the required level plus LA→B and at one side of the frequency centre of reference measurement channel.

3)	Configure RF signal generator for interferer to set it at the required level plus LA→E and at the other side of the frequency centre of reference measurement channel. The interferer should be adjacent to the wanted signal.

4)	Measure the throughput of the wanted signal according to annex E of the 3GPP TS 36.141 [22].

5)	Repeat the steps 2-4 by exchange the locations of wanted signal and interfering signal in frequency domain at the same channel.

6)	Repeat steps above per involved band for multi-band RIB(s).

##### 10.3.7.3.4	MU assessment

###### 10.3.7.3.4.1	MU Budget

The MU budget for ICS is the same as that for RX ACS, described in section 10.3.5.3.4.1

###### 10.3.7.3.4.2	MU Value

The MU value for ICS is the same as that for RX ACS, described in section 10.3.5.3.4.1.

#### 10.3.7.4	Summary

The MU value for ICS is the same as that for RX ACS, described in section 10.3.5.4

## 10.4	Measurement uncertainty for In-band TRP requirements

### 10.4.1	OTA Base station output power requirement

#### 10.4.1.1	General

{editors note: general description of the test requirement placed here – non chamber specific}

#### 10.4.1.2	In-door anechoic chamber

##### 10.4.1.2.1	General

This method measures the base station output power in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in the figure 10.4.1.2.1-1.

NOTE: Whilst the TRP estimation does not require far-field conditions explicitly the MU budget below is based on errors under far-field conditions. If far-field conditions are not met an in-door anechoic chamber may be used but a separate MU analysis is necessary.

Figure 10.4.1.2.1-1: In-door Anechoic Chamber measurement system setup for OTA base station output power.

##### 10.4.1.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2.

##### 10.4.1.2.3	Procedure

1)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

2)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

3)	Set the AAS BS to transmit the test signal at the maximum power according to the appropriate test model

4)	Measure the appropriate test parameter.

5)	Repeat the above steps 3)~5) at a number of discrete directions around the sphere according to the chosen measurement grid, see subclause 10.8.3.

6)	Calculate TRPs from EIRP, as shown in subclause 10.8.

The appropriate test parameter in step 5 is PR_AAS_desired, D, measured mean power within the desired signal channel bandwidth for each carrier arriving at the measurement equipment connector at D in figure 10.4.1.2.1-1, and  calculation of EIRP EIRPd using the following equitation:

EIRPd = PR_AAS_desired, D+ LTX_cal, A→D

NOTE: 	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the AAS BS, then measure and sum the EIRP on both polarizations. If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations and calculate total radiated transmit power as the sum over both polarizations.

##### 10.4.1.2.4	MU assessment

###### 10.4.1.2.4.1	MU Budget

Table 10.4.1.2.4.1-1: In-door Anechoic Chamber uncertainty assessment condition for OTA BS output power EIRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E1-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna. | E1-2 |
| 3 | Quality of quiet zone | E1-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E1-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E1-5 |
| 6 | Phase curvature | E1-6 |
| 7 | Uncertainty of the RF power measurement equipment | E1-7, F.1 |
| 8 | Impedance mismatch in the receiving chain | E1-8 |
| 9 | Random uncertainty | E1-9 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | E1-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E1-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer | E1-12 |
| 13 | Quality of quiet zone | E1-3 |
| 14 | Polarization mismatch for reference antenna | E1-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E1-5 |
| 16 | Phase curvature | E1-6 |
| 17 | Uncertainty of the Network Analyzer | E1-13, F.1 |
| 18 | Influence of the reference antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E1-15 |
| 20 | Influence of the receiving antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | E1-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E1-17 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E1-18 |

###### 10.4.1.2.4.2	MU Value

Table 10.4.1.2.4.2-1: Indoor Anechoic Chamber uncertainty assessment for EIRP measurement of OTA base station output power

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.44 | 0.54 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.87 | 1.06 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,3dB

#### 10.4.1.3	CATR

##### 10.4.1.3.1	General

The Compact Antenna Test Range (CATR) uses the DUT which radiates a wave front to a range antenna reflector which will then collimate the radiated spherical wave front into a feed antenna.  The sufficient separation between the DUT and the receiver (feed antenna shown in figure 10.4.1.3.1-1) so that the emanating spherical wave reaches nearly plane phase fronts from transmitter to receiver. The DUT transmits a wave front that will illuminate the range antenna reflector, which will then reflect the transmitted energy into the feed antenna.  The range feed antenna is connected to a vector network analyzer or other equivalent test equipment.

Figure 10.4.1.3.1-1: CATR measurement system setup for OTA BS output power

Test Method limitations and scope

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is taken into account.

##### 10.4.1.3.2	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

Calibration should be carried out using the same procedure as in 10.2.3.3.2.

##### 10.4.1.3.3	Procedure

1)	Set up AAS BS in place of SGH from calibration stage.  Align the coordinates system of the DUT with that of the test system.

2)	Set the AAS BS to transmit the test signal at the maximum power according to the appropriate test model Measure the appropriate test parameter

3)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

4)	Measure the appropriate test parameter.

5)	Repeat the above steps 3)~5) at a number of discrete directions around the sphere according to the chosen measurement grid, see subclause 10.8.3.

6)	Calculate TRPs from power density, as shown in subclause 10.8.3.

The appropriate test parameter in step 5 is PR_AAS_desired, B, measured mean power within the desired signal channel bandwidth for each carrier arriving at the measurement equipment connector B in figure 10.4.1.3.1-1, and  calculation of power powerd using the following equitation:

PowerD = PR_AAS_desired, B+ LTX_cal, A→B

NOTE: If the test facility only supports single polarization, then measure power with the test facility's test antenna/probe polarization matched to the AAS BS, then measure and sum the power on both polarizations. If the test facility supports dual polarization then measure total power for two orthogonal polarizations and calculate total radiated transmit power as the sum over both polarizations.

##### 10.4.1.3.4	MU assessment

###### 10.4.1.3.4.1	MU Budget

Table 10.4.1.3.4.1-1: Compact antenna test range uncertainty contributions for OTA Base station output power measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | RF power measurement equipment | F.1 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 5 | QZ ripple DUT | E2-5 |
| 19 | Miscellaneous uncertainty | E2-14 |
|  | Test system frequency flatness | E2-16 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | E2-6, F.1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | E |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |

###### 10.4.1.3.4.2	MU Value

Table 10.4.1.3.4.2-1: Compact antenna test range uncertainty assessment for OTA Base station output power measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.594 | 0.669 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.16 | 1.31 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,4 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,5dB

#### 10.4.1.4	Near Field

##### 10.4.1.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA Base Station output power measurements, NF to FF transform is not needed since TRP is computed based on power density measured in Near Field by sampling properly the declared beam.

##### 10.4.1.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.3.4.2

##### 10.4.1.4.3	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

1)	Configure the beam of the AAS BS according to the required conditions for the TRP test

2)	Set the AAS BS to transmit the test signal at the maximum power according to the appropriate test model e.g. E-TM1.1.

3)	Measure the power density at a number of points (Theta; Phi) in the beam according to the chosen measurement grid. Refer to subclause 10.8 for details about measurement grids for TRP measurements

4)	Calculate TRP from the measured power density as in subclause 10.8

##### 10.4.1.4.4	MU assessment

###### 10.4.1.4.4.1	MU Budget

Table 10.4.1.4.4.1-1: Near field test range uncertainty contributors in power density pattern measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2: Near Field AAS power density pattern measurement |  |
| 1 | Axes Intersection | E3-1 |
| 2 | Axes Orthogonality | E3-2 |
| 3 | Horizontal Pointing | E3-3 |
| 4 | Probe Vertical Position | E3-4 |
| 5 | Probe H/V pointing | E3-5 |
| 6 | Measurement Distance | E3-6 |
| 7 | Amplitude and Phase Drift | E3-7 |
| 8 | Amplitude and Phase Noise | E3-8 |
| 9 | Leakage and Crosstalk | E3-9 |
| 10 | Amplitude Non-Linearity | E3-10 |
| 11 | Amplitude and Phase Shift in rotary joints | E3-11 |
| 12 | Channel Balance Amplitude and Phase | E3-12 |
| 13 | Probe Polarization Amplitude and Phase | E3-13 |
| 14 | Probe Pattern Knowledge | E3-14 |
| 15 | Multiple Reflections | E3-15 |
| 16 | Room Scattering | E3-16 |
| 17 | DUT support Scattering | E3-17 |
| 18 | Positioning | E3-21 |
| 19 | Probe Array Uniformity | E3-22 |
| 20 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | E3-23 |
| 21 | Insertion loss of receiver chain | E3-24 |
| 22 | Uncertainty of the absolute gain of the probe antenna | E3-25 |
| 23 | Measurement Equipment (spectrum Analyzer) | E3-26, F.1 |
| 24 | Measurement repeatability - Positioning Repeatability | E3-27 |
| 25 | Test System Frequency Flatness | E3-36 |
|  | Stage 1: Calibration measurement |  |
| 26 | Uncertainty of network analyser | E3.28, F.1 |
| 27 | Mismatch of receiver chain | E3-29 |
| 28 | Insertion loss of receiver chain | E3-30 |
| 29 | Mismatch in the connection of the calibration antenna | E3-31 |
| 30 | Influence of the calibration antenna feed cable | E3-32 |
| 31 | Influence of the probe antenna cable | E3-33 |
| 32 | Uncertainty of the absolute gain of the calibration antenna | F.1 |
| 33 | Short term repeatability | E3-35 |

Note: 	Refer to TR 37.842 for description of each uncertainty term

###### 10.4.1.4.4.2	MU Value

Table 10.4.1.4.4.2-1: Near field test range uncertainty assessment for power density pattern measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 19 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 20 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 21 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 22 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 23 | Measurement Equipment (Spectrum Analyzer) | 0.41 | 0.56 | Gaussian | 1.00 | 1 | 0.41 | 0.56 |
| 24 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| 25 | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1.00 | 1 | [0.25] | [0.25] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 26 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 27 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 28 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 29 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 30 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 32 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 33 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | [0.59] | [0.70] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | [1.15] | [1.37] |

MU for OTA BS Output Power is the RSS of MU per point measurement (table 10.4.1.4.4.2-1) and TRP summation error (subclause 10.8):

MUper-point2+SE2

For OTA BS Output Power measured in Near Field setup the MU is:

f≤3GHz

[MUTRP=0.592+0.752 =1.87dB]

3GHz≤f≤4.2GHz

[MUTRP=0.702+0.752 =2dB]

#### 10.4.1.4A	Reverberation chamber

##### 10.4.1.4A.1	General

The reverberation chamber test setup is described in subalcuse 10.5.2.3A.

##### 10.4.1.4A.2	Calibration

The calibration procedure is described in subclause 10.5.2.3A.3.

##### 10.4.1.4A.3	Procedure

The test procedure is described in subclause 10.5.2.3A.4.

##### 10.4.1.4A.4	MU assessment

###### 10.4.1.4A.4.1	MU Budget

The measurement error sources are described in subclause 10.5.2.3A.5.1.

###### 10.4.1.4A.4.2	MU Value

Table 10.4.1.4A.4.2-1: Reverberation chamber uncertainty assessment for output power

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| NOTE: This MU budget is applicable if the data tests in procedure step 11) of clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

#### 10.4.1.5	Summary

Table 10.4.1.5-1: Test system specific measurement uncertainty values for the OTA BS output power test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1,1 | 1,3 |
| Compact Antenna Test Range | 1,4 | 1,5 |
| Near Field chamber |  |  |
| Reverberation chamber | 1.4 | 1.46 |
| Common maximum accepted test system uncertainty | 1.4 | 1.5 |

### 10.4.2	OTA Adjacent channel leakage power ratio

#### 10.4.2.1	General

#### 10.4.2.2	In-door anechoic chamber

##### 10.4.2.2.1	General

This method measures the ACLR in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in the figure 10.2.3.2.1-1.

NOTE: 	Whilst the TRP estimation does not require far-field conditions explicitly the MU budget below is based on errors under far-field conditions. If far-field conditions are not met an in-door anechoic chamber may be used but a separate MU analysis is necessary.

##### 10.4.2.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2.

Calibration shall be performed individually for both the wanted signal and adjacent channel frequencies.

##### 10.4.2.2.3	Procedure

Reference procedure in subclause 10.4.1.2.3.

The appropriate parameters in step 5 are:

PR_AAS_desired, D: Measured mean power within the desired signal channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.2.3.2.1-1.

PR_AAS_emission, D: Measured mean emission power in the neighbouring channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.2.3.2.1-1.

Calculation of Powers Powerd and Powere using following formula:

Powerd = PR_AAS_desired, D+ LTX_cal, A→D

Powere = PR_AAS_emission, D+ LTX_cal, A→D

After calculation of TRP from Power as shown in subclause 10.8 calculate ACLR.

##### 10.4.2.2.4	MU assessment

###### 10.4.2.2.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.2.4.1.

Table 10.4.2.2.4.1-1: Void

###### 10.4.2.2.4.2	MU Value

Table 10.4.2.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA BS ACLR EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.444 | 0.538 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.871 | 1.055 |

The agreed summation error (SE) of 0,75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,3dB

#### 10.4.2.3	CATR

##### 10.4.2.3.1	General

The CATR is described in section 10.4.1.3.1.

##### 10.4.2.3.2	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

Calibration of the CATR test setup should be carried out in the same manner as described for the EVM procedure in subclause 10.2.6.3.2.

##### 10.4.2.3.3	Procedure

Stage 2 - Measurement:

1)	Align DUT with boresight of the range antenna.

2)	Configure carrier at a power level according to the manufacturer's declared rated output power and test configuration.

3)	Measure wanted and adjacent channel power for the frequency offsets both side of carrier frequency considering both polarizations of the range antenna. In multiple carrier case only offset frequencies below the lowest and above the offsets highest carrier frequency used shall be measured for ACLR; offsets in between carriers may be subject to CACLR according to the rules in 37.105..

4)	Repeat step 4 for additional points for all necessary points needed for full TRP for the wanted signal and adjacent channel emissions.

5)	Calculate ACLR or CACLR from the wanted signal TRP and the adjacent channel emissions TRP,

The appropriate parameters in step 5 are:

PR_AAS_desired, D: Measured mean power within the desired signal channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.2.3.2.1-1.

PR_AAS_emission, D: Measured mean emission power in the neighbouring channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.2.3.2.1-1.

Calculation of Powers Powerd and Powere using following formula:

Powerd = PR_AAS_desired, D+ LTX_cal, A→D

Powere = PR_AAS_emission, D+ LTX_cal, A→D

After calculation of TRP from power as shown in subclause 10.8, calculate ACLR.

##### 10.4.2.3.4	MU assessment

###### 10.4.2.3.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.3.4.1.

Table 10.4.2.3.4.1-1: VoidTable 10.4.2.3.4.1-2: Void

###### 10.4.2.3.4.2	MU Value

Table 10.4.2.3.4.2-1: Compact antenna test range uncertainty assessment for absolute ACLR measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | [0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.069 | 1.349 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.10 | 2.64 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

Table 10.4.2.3.4.2-2: CATR uncertainty assessment for relative ACLR measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.14 | Gaussian | 1 | 1 | 0.14 | 0.14 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.37 | 0.46 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.73 | 0.89 |

The agreed summation error (SE) of 0,75 dB is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1,0 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1,2dB

#### 10.4.2.4	Near Field

##### 10.4.2.4.1	Description

The system setup is depicted in section 10.2.3.4.1. In case of OTA ACLR measurement, the NF to FF transform is not needed since ACLR is based on TRP.

##### 10.4.2.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done by following the procedure in 10.2.3.4.2.

##### 10.4.2.4.3	Procedure

The procedure for relative ACLR OTA measurement consists of the following steps:

Stage 2 - Measurement:

The testing procedure consists of the following steps:

1.	Configure TX branch and carrier according to the required test configuration

2.	Measure in Near Field ACLR and CALCR for the frequency offsets both side of carrier frequency for both polarizations. In multiple carrier case only offset frequencies below the lowest and above the offsets highest carrier frequency used shall be measured.

a.	Power is measured Near Field – no NF to FF transform is applied to

b.	Full sphere power is measured with using a defined sampling grid (refer to subclause 10.8 for proper measurement grids)

3.	Calculate TRP by using the formula in [2]

##### 10.4.2.4.4	MU assessment

###### 10.4.2.4.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.4.4.1.

Table 10.4.2.4.4.1-1: Void

###### 10.4.2.4.4.2	MU Value

Table 10.4.2.4.4.2-2: Near Field uncertainty assessment for relative ACLR measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 19 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 20 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 21 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 22 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 23 | MU of TE derived from conducted specification | 0.41 | 0.56 | Gaussian | 1.00 | 1 | 0.41 | 0.56 |
| 24 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| 25 | Test System Frequency Flatness | [0.25] | [0.25] | Gaussian | 1.00 | 1 | [0.25] | [0.25] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 26 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 27 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 28 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 29 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 30 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 32 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 33 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | [0.59] | [0.7] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | [1.16] | [1.38] |

MU for OTA ACLR is TBD

TRP summation error is reported in subclause 10.8

#### 10.4.2.4A	Reverberation chamber

##### 10.4.2.4A.1	General

The reverberation chamber test setup is described in subalcuse 10.5.2.3A.

##### 10.4.2.4A.2	Calibration

The calibration procedure is described in subclause 10.5.2.3A.3.

##### 10.4.2.4A.3	Procedure

The test procedure is described in subclause 10.5.2.3A.4.

##### 10.4.2.4A.4	MU assessment

###### 10.4.2.4A.4.1	MU Budget

The measurement error sources are described in subclause 10.5.2.3A.5.1.

###### 10.4.2.4A.4.2	MU Value

Table 10.4.2.4A.4.2-1: Reverberation chamber uncertainty assessment for relative ACLR measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| NOTE: This MU budget is applicable if the data tests in procedure step 11) of clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

Table 10.4.2.4A.4.2-2: Reverberation chamber uncertainty assessment for absolute ACLR measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| NOTE: This MU budget is applicable if the data tests in procedure step 11) of clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

#### 10.4.2.5	Summary

Table 10.4.2.5-1: Test system specific measurement uncertainty values for the OTA absolute ACLR

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1,1 | 1,3 |
| Compact Antenna Test Range | 2,2 | 2,7 |
| Near Field chamber |  |  |
| Reverberation chamber | 1.40 | 1.46 |
| Common maximum accepted test system uncertainty | 2,2 | 2,7 |

Table 10.4.2.5-2: Test system specific measurement uncertainty values for the OTA relative ACLR

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber |  |  |
| Compact Antenna Test Range | 1,0 | 1,2 |
| Near Field chamber |  |  |
| Reverberation chamber | 1.40 | 1.46 |
| Common maximum accepted test system uncertainty | 1,0 | 1,2 |

### 10.4.3	OTA Operating band unwanted emission

#### 10.4.3.1	General

#### 10.4.3.2	In-door anechoic chamber

##### 10.4.3.2.1	General

This method measures the base station output power in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in the figure 10.2.3.2.1-1.

NOTE: 	Whilst the TRP estimation does not require far-field conditions explicitly the MU budget below is based on errors under far-field conditions. If far-field conditions are not met an in-door anechoic chamber may be used but a separate MU analysis is necessary.

##### 10.4.3.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2.

Calibration shall be performed individually for each frequency at which unwanted emissions are measured  (This may involve calibration measurement or interpolation between calibration points).

##### 10.4.3.2.3	Procedure

Reference procedure in subclause 10.4.1.2.3.

The appropriate The appropriate parameters in step 5 is the mean power for OBUE test for each carrier arriving at the measurement equipment connector, denoted by PR_AAS_OBUE, D, and calculation of EIRP EIRPe using following formula:

EIRPe = PR_AAS_OBUE_D+ LTX_cal, A→D

##### 10.4.3.2.4	MU assessment

###### 10.4.3.2.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.2.4.1.

Table 10.4.3.2.4.1-1: Void

###### 10.4.3.2.4.2	MU Value

Table 10.4.3.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA OBUE EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.444 | 0.538 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.871 | 1.055 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1.1 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1.3dB

#### 10.4.3.3	CATR

##### 10.4.3.3.1	General

The CATR is described in subclause 10.4.1.3.1.

##### 10.4.3.3.2	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

Calibration should be carried out using the same procedure as in 10.2.3.3.2.

Calibration shall be performed individually for each frequency at which unwanted emissions are measured. (This may involve calibration measurement or interpolation between calibration points).

##### 10.4.3.3.3	Procedure

The reference procedure can be found in subclause 10.4.1.3.3.

The appropriate The appropriate parameters in step 5 is the mean power for OBUE test for each carrier arriving at the measurement equipment connector, denoted by PR_AAS_OBUE, B, and calculation of power powere using following formula:

Epowere = PR_AAS_OBUE_B+ LTX_cal, A→B

##### 10.4.3.3.4	MU assessment

###### 10.4.3.3.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.3.4.1.

Table 10.4.3.3.4.1-1: Void

###### 10.4.3.3.4.2	MU Value

Table 10.4.3.3.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA operating band unwanted emissions measurement

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.83 | 0.93 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.63 | 1.83 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

#### 10.4.3.4	Near Field

##### 10.4.3.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA OBUE measurement, NF to FF transform is not needed since TRP is computed based on power density measured in Near Field by sampling properly the power density for OBUE.

##### 10.4.3.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.3.4.2

##### 10.4.3.4.3	Procedure

Stage 2 - Measurement:

Refer to section 10.4.1.4.3. The measured power in step 3 is the power density for OBUE test for each carrier arriving at the measurement equipment connector.

##### 10.4.3.4.4	MU assessment

###### 10.4.3.4.4.1	MU Budget

Refer to clause 10.4.1.4.4.1 for MU budget.

###### 10.4.3.4.4.2	MU Value

Refer to clause 10.4.1.4.4.2 for MU value per point measurement.

MU for OTA OBUE is the RSS of MU per point measurement (table 10.4.1.4.4.2-1) and TRP summation error (Subclause 10.8):

MUTRP=MUper-point2+SE2

For OTA OBUE measured in Near Field setup the MU is:

f≤3GHz

MUTRP=1.152+0.752 =1.4dB

3GHz≤f≤4.2GHz

MUTRP=1.372+0.752 =1.6dB

#### 10.4.3.4A	Reverberation chamber

##### 10.4.3.4A.1	General

The reverberation chamber test setup is described in subalcuse 10.5.2.3A.

##### 10.4.3.4A.2	Calibration

The calibration procedure is described in subclause 10.5.2.3A.3.

##### 10.4.3.4A.3	Procedure

The test procedure is described in subclause 10.5.2.3A.4.

##### 10.4.3.4A.4	MU assessment

###### 10.4.3.4A.4.1	MU Budget

The measurement error sources are described in subclause 10.5.2.3A.5.1.

###### 10.4.3.4A.4.2	MU Value

Table 10.4.3.4A.4.2-1: Reverberation chamber uncertainty assessment for OBUE

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| NOTE: This MU budget is applicable if the data tests in procedure step 11) of clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

#### 10.4.3.5	Summary

Table 10.4.3.5-1: Test system specific measurement uncertainty values for the OTA absolute ACLR

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1,1 | 1,3 |
| Compact Antenna Test Range | 1,8 | 2,0 |
| Near Field chamber |  |  |
| Reverberation chamber | 1.40 | 1.46 |
| Common maximum accepted test system uncertainty | 1,8 | 2,0 |

### 10.4.4	OTA Spectrum emission mask

#### 10.4.4.1	General

#### 10.4.4.2	In-door anechoic chamber

##### 10.4.4.2.1	General

This method measures the base station output power in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in the figure 10.2.3.2.1-1.

NOTE: 	Whilst the TRP estimation does not require far-field conditions explicitly the MU budget below is based on errors under far-field conditions. If far-field conditions are not met an in-door anechoic chamber may be used but a separate MU analysis is necessary.

##### 10.4.4.2.2	Calibration

Calibration shall be done with the procedure shown in 10.2.3.2.2.

Calibration shall be performed individually for each frequency at which unwanted emissions are measured. (This may involve calibration measurement or interpolation between calibration points).

##### 10.4.4.2.3	Procedure

Reference procedure in subclause 10.4.1.2.3.

The appropriate The appropriate parameters in step 5 is the mean power for SEM test at each carrier arriving at the measurement equipment connector, denoted by PR_AAS_SEM, D, and calculation of EIRP EIRPe using following formula:

EIRPe = PR_AAS_SEM_D+ LTX_cal, A→D

##### 10.4.4.2.4	MU assessment

###### 10.4.4.2.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.2.4.1.

Table 10.4.4.2.4.1-1: Void

###### 10.4.4.2.4.2	MU Value

Table 10.4.4.2.4.2-1: In-door anechoic chamber uncertainty assessment for OTA SEM EIRP measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.3 | 0.3 | Rectangular | √3 | 1 | 0.17 | 0.17 |
| 3 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 |  | 0.05 | 0.05 |
| 7 | Uncertainty of the RF Power Measurement Equipment | [0.14] | [0.26] | [Gaussian] | [1] | [1] | [0.14] | [0.26] |
| 8 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | U-shaped | √2 | 1 | 0.10 | 0.23 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.43 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | Rectangular | √3 | 1 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.493 | 0.560 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.966 | 1.098 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 1.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 1.3dB

#### 10.4.4.3	CATR

##### 10.4.4.3.1	General

The CATR is described in subclause 10.4.1.3.1.

##### 10.4.4.3.2	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

Calibration should be carried out using the same procedure as in 10.2.3.3.2.

Calibration shall be performed individually for each frequency at which unwanted emissions are measured (This may involve calibration measurement or interpolation between calibration points).

##### 10.4.4.3.3	Procedure

The reference procedure can be found in subclause 10.4.1.3.3.

The appropriate The appropriate parameters in step 5 is the mean power for SEM test at each carrier arriving at the measurement equipment connector, denoted by PR_AAS_SEM, B, and calculation of power powere using following formula:

powere = PR_AAS_SEM_B+ LTX_cal, A→B

##### 10.4.4.3.4	MU assessment

###### 10.4.4.3.4.1	MU Budget

The MU budget is the same as in subclause 10.4.1.3.4.1.

Table 10.4.4.3.4.1-1: Void

###### 10.4.4.3.4.2	MU Value

Table 10.4.4.3.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA SEM

| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
|  | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0,83 | 0,93 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1,63 | 1,83 |

The agreed summation error (SE) of 0.75 dB (see subclause 10.8) is then root square sum combined with the per point values to give the following result (with 95% confidence level):

Total uncertainty f ≦ 3GHz (95% confidence level): 2.2 dB

Total uncertainty 3GHz < f ≦ 4.2 GHz (95% confidence level): 2.7dB

#### 10.4.4.4	Near Field

##### 10.4.4.4.1	General

The system is depicted in section 10.2.3.4.1. In case of OTA Spectrum Emission Mask measurement, NF to FF transform is not needed since TRP is computed based on power density measured in Near Field by sampling properly the power density for SEM.

##### 10.4.4.4.2	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.3.4.2

##### 10.4.4.4.3	Procedure

Stage 2 - Measurement:

Refer to section 10.4.1.4.3. The measured power in step 3 is the power density for SEM test for each carrier arriving at the measurement equipment connector.

##### 10.4.4.4.4	MU assessment

###### 10.4.4.4.4.1	MU Budget

Refer to clause 10.4.1.4.4.1 for MU budget.

###### 10.4.4.4.4.2	MU Value

Refer to clause 10.4.1.4.4.2 for MU value per point measurement.

MU for OTA Spectrum Emission Mask is the RSS of MU per point measurement (table 10.4.1.4.4.2-1) and TRP summation error (Subclause 10.8):

MUTRP=MUper-point2+SE2

For OTA Spectrum Emission Mask measured in Near Field setup the MU is:

f≤3GHz

[MUTRP=1.152+0.752 =1.4dB

3GHz≤f≤4.2GHz

[MUTRP=1.372+0.752 =1.6dB]

#### 10.4.4.4A	Reverberation chamber

##### 10.4.4.4A.1	General

The reverberation chamber test setup is described in subalcuse 10.5.2.3A.

##### 10.4.4.4A.2	Calibration

The calibration procedure is described in subclause 10.5.2.3A.3.

##### 10.4.4.4A.3	Procedure

The test procedure is described in subclause 10.5.2.3A.4.

##### 10.4.4.4A.4	MU assessment

###### 10.4.4.4A.4.1	MU Budget

The measurement error sources are described in subclause 10.5.2.3A.5.1.

###### 10.4.4.4A.4.2	MU Value

Table 10.4.4.4A.4.2-1: Reverberation chamber uncertainty assessment for SEM

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 |
| 3 | Random uncertainty | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.15 | 0.15 | Gaussian | 1 | 1 | 0.15 | 0.15 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.71 | 0.75 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.40 | 1.46 |
| NOTE: This MU budget is applicable if the data tests in procedure step 11) of clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |

#### 10.4.4.5	Summary

Table 10.4.4.5-1: Test system specific measurement uncertainty values for the OTA SEM

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1,2 | 1,3 |
| Compact Antenna Test Range | 1,8 | 2,0 |
| Near Field chamber | [1.4] | [1.6] |
| Reverberation chamber | 1.40 | 1.46 |
| Common maximum accepted test system uncertainty | 1,8 | 2,0 |

## 10.5	Measurement uncertainty for Out-of-band TRP requirements

### 10.5.1	General

The TRP MU consists of an MU per point and a Summation Error (SE) which allows for errors in the calculation of the TRP from multiple directional power measurements and allows for a sparse grid to be used to reduce measurement time. The total MU is calculated as follows:

Refer to subclause 10.8 for the SE value.

### 10.5.2	Transmitter mandatory spurious emissions

#### 10.5.2.1	General

The conducted spurious emission requirement MU is split up into a number of frequency ranges

| Transmitter spurious emissions, Mandatory Requirements | dB | MU |
| --- | --- | --- |
| 30 MHz ≤ f ≤ 4 GHz |  | 2.0 |
| 4 GHz < f ≤ 19 GHz |  | 4.0 |

The conducted analysis based on UTRA/E-UTRA frequencies which were all below 4.2GHz (at the time), the break point in the MU is hence somewhat related to the in-band and out of band MU analysis. As in-band MU analysis is now being done up to 6GHz (for the LAA and NR bands) it is sensible to change the frequency break point to 6GHz.

The conducted and OTA test set up can be compared as follows:

Figure 10.5.2.1-1:	Conducted and OTA spurious emissions test set up

As the spurious emissions requirements cover a  large frequency span from 30MHz to 12.75GHz it is difficult to use the same measurement chambers as the in band measurements. The CATR for example is not suitable for low frequency measurements.

The chamber analysis is hence done on a general OTA chamber which is capable of all frequency ranges.

Other chambers may be used if their MU is within the final value and they are suitable for the frequencies being tested.

#### 10.5.2.2	General chamber

##### 10.5.2.2.1	General

As the AAS BS antenna radiating dimensions are fixed then the far field distance increases (FF≈2d2/λ). At 12.75 GHz the far field distance for a 1.5m AAS BS antenna array is almost 200m, this is clearly impractical in an indoor chamber (and the path loss would also make measurement difficult), so spurious emission testing will not always be in the far field. This is acceptable as the requirement is TRP and hence it is not necessary to measure in the far field however it needs to be considered when looking at MU.

Considerations of the large frequency range must also be considered, including the chamber performance (quiet zone), the calibration effectiveness and the available reference and test antennas over the frequency range.

##### 10.5.2.2.2	Calibration

Each frequency point can be calibrated using the EIRP calibration approach in sub-clause 10.2.3.2.2 .

The calibration should be repeated for each frequency being tested and each test antenna.

##### 10.5.2.2.3	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.7.6.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the AAS BS to transmit according to the applicable test configuration

6)	Align the BS and the test antenna such that measurements to determine TRP can be performed (see subclause 10.8.3)

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see subclause 10.8.3) and for frequency points to be tested.

Note 1: the TRP measurement grid may not be the same for all measurement frequencies.

Note 2: the frequency sweep or the TRP measurement grid sweep may be done in any order

9)	Calculate TRP at each specified frequency using the directional measurements.

##### 10.5.2.2.4	MU assessment

###### 10.5.2.2.4.1	MU Budget

Table 10.5.2.2.4.1-1: General  Chamber uncertainty contributions
Tx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E4-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | E4-2 |
| 3 | Quality of quiet zone | E4-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E4-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E4-5 |
| 6 | Phase curvature | E4-6 |
| 7 | Conducted measurement uncertainty (minus mismatch) | E4-22, F.2 |
| 8 | Impedance mismatch in the receiving chain | E4-8 |
| 9 | Random uncertainty | E4-9 |
| 23 | Measurement antenna frequency variation | E4-20 |
| 24 | FSPL estimation error | E4-21 |
| 25 | Test system frequency flatness | E4-19 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | E4-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E4-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | E4-12 |
| 13 | Quality of quiet zone | E4-3 |
| 14 | Polarization mismatch for reference antenna | E4-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E4-5 |
| 16 | Phase curvature | E4-6 |
| 17 | Uncertainty of the network analyzer | E4-13, F.1 |
| 18 | Influence of the reference antenna feed cable | E4-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E4-15 |
| 20 | Influence of the receiving antenna feed cable | E4-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E4-17 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E4-18 |

###### 10.5.2.2.4.2	MU Value

Table 10.5.2.2.4.2-1: General  Chamber uncertainty assessment
Tx spurious emissions

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |  |  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 7 | Conducted measurement uncertainty (minus mismatch | 1.02 | 2.04 | Gaussian | 1 | 1 | 1.02 | 2.04 |
| 8 | Impedance mismatch in the receiving chain | 0.2 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.32 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 23 | Measurement antenna frequency variation | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 24 | FSPL estimation error | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |
| 25 | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.2 | Gaussian | 1 | 1 | 0.13 | 0.2 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.12 | 2.11 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.20 | 4.15 |

#### 10.5.2.3	Void

#### 10.5.2.3A	Reverberation chamber

##### 10.5.2.3A.1	General

A reverberation chamber (RC) is an electrically large shielded metal enclosure that employs one or several “stirring” methods to randomize the fields, such as moving paddles, turntables, etc. In this way, a large number of uncorrelated samples is obtained. The volume in the room where the field is well-stirred is the working volume. Here the E-field, averaged over an entire stirring cycle, is independent of the location in the room, i.e. the field is spatially uniform.

For a proper analysis of the measured data a sufficient number of uncorrelated samples is required. The auto-correlation function is used to calculate the offset between statistically uncorrelated samples using the following expression [33]:

where the modulus operator mod(x,y) is the remainder of x/y, here performing  a circular shift of the measurement samples over a distance k. The symbols ⟨x ⟩ and σ= "std" (x) denote the average value and standard deviation. The threshold value for uncorrelated samples is defined as [33]:

The distance klimbetween uncorrelated samples is calculated as the minimum k-value satisfying rk≤rlim. The number of uncorrelated samples is calculated as:

When properly designed, this facility can be used for non-directional antenna measurements, such as TRP. When measuring TRP of sources with a directive pattern, special care must be taken to characterize the working volume of the chamber, see subclause 10.5.2.3A.2.

##### 10.5.2.3A.2	Chamber characterization

The purpose of the chamber characterization is to ensure that the effect of a non-uniform field distribution in the chamber has a negligible influence on the measurement result when the EUT is placed in the working volume. Lack of chamber uniformity is a major contributor to measurement uncertainty in reverberation chambers and should be handled with care.

The uniformity test can be quite time consuming and the test can be performed separate from the EUT measurement. Due to the non-negligible size of BS equipment the EUT can have a significant influence on the uniformity. To take this effect into account, either the EUT itself must be present in the room during characterization or an absorber with dimensions equal or larger than the EUT must be placed at the EUT’s location in the room.

The characterization procedure consists of placing a reference transmitter antenna (REF TX ant) at different locations and with different orientations in the room. The actual mode of RC operation shall be used, including stirrer movement, EUT movement, diversity antenna usage, etc. The directivity of the REF TX antenna will influence the spatial uniformity of the room.

Figure 10.5.2.3A.2-1:	Setup for characterization of a reverberation chamber

The working volume shall be at least half a wavelength from the chamber walls and other electromagnetic reflective objects according to [34]. According to [33] this distance may be restricted to 0.75 m below 100 MHz. The number of positions and orientations to use depends on the chamber size and the directivity of the REF TX. Measurements made at positions and orientations at the edges ofthe working volume are used to characterize the chamber and derive certain components of uncertainty.

The exact number of positions and orientations remain for further study, but at least (3) uncorrelated locations should be used and (6) uncorrelated orientations per position when directive spurs are to be detected.

The REF TX antenna and the measurement receive antenna (RX) are connected to a Vector Network Analyzer (VNA).  For each location/orientation n of the REF TX, RC sample, and desired frequency ff, the power transfer function

is measured. The explicit dependence on RC sample and frequency is not written out here. At least 250 uncorrelated samples shall be used per position/orientation. Using a lower number is not compatible with the underlying analysis on measurement uncertainty, see [33, 35].

The following tests are performed and shall be verified for each frequency:

a)	Uniformity of transfer function: For each location/orientation evaluate PnPn. The standard deviation of these average values shall be below (1.5 dB).

b)	Dynamic range: The dynamic range of each PnPn shall be at least 20 dB. The method to compute it is defined below.

c)	Uncorrelated samples: At least 250 uncorrelated samples shall be used.

It is important to note that spatial uniformity (and number of uncorrelated samples) in an RC is harder to achieve for more directive antenna patterns. As such, the measurement uncertainty of a sub-optimally configured room will be higher and additional measures to randomize the fields should be considered.

##### 10.5.2.3A.3	Calibration

To correctly determine the TRP by the EUT a calibration step to account for losses in cables, antennas, etc. is required. A reference PTF is determined at each frequency of interest by using a relevant reference TX antenna (REF TX). The chamber should be set up identically to the actual set up of the TRP measurement, which means the EUT has to be in the room, the same stirring procedure has to be used, etc.

A minimum Nref = 1 reference measurement is required, but it is noted that more measurements can be used to estimate the reference PTF and that this will lead to a reduction of the uncertainty by a factor 1/Nref. The radiation efficiency, η, of the reference antenna can be assumed to be the value declared by the manufacturer or can be determined in a separate antenna efficiency measurement.

Calibration procedure

1)	Place the EUT and the REF TX in the working volume of the RC. Turn off EUT power and EUT controls. See Figure 10.5.2.3A.4-1 (a). If more positions are used to determine the reference PTF the REF TX should be placed at uncorrelated positions within the test volume.

2)	Set the stirrers and turntables in the mode of operation used in the chamber characterization.

3)	Set the sampling rate as in the chamber characterization.

4)	Connect the REF TX and the RX antenna with a calibrated Network analyser (NA)

5)	Measure the scattering parameters over a complete stirring cycle for each frequency of interest and for each position and orientation.

6)	Calculate the reference transfer function Pr=S21r2

7)	Calculate the mis-match efficiency 	M1r=1-S11r2.

##### 10.5.2.3A.4	Procedure

(a)											(b)										(c)

Figure 10.5.2.3A.4-1:	The procedure for TRP measurements uses three consecutive setups: (a) Reference measurement, (b) ambient noise measurement (b), and (c) TRP measurement. The blue dots indicate the measurement planes

Calculate Ambient Power Level

1)	Connect the RX antenna to a calibrated Spectrum analyser (SA) using the same cables as in the calibration step. Turn on the EUT control. Keep the EUT RF power off. Terminate the REF TX in a 50Ω load. See Figure 10.5.2.3A.4-1 (b).

2)	Measure the voltage data Uamb and calculate the ambient power level as Pamb=Uamb250

3)	Calculate the ambient TRP level as TRPamb=ηM1rPrPamb

Calculate EUT TRP

1)	Turn on the EUT RF power and measure the received voltage UEUT, see Figure 10.5.2.3A.4-1 (c).

2)	Calculate the EUT power as PEUT=UEUT250

3)	Calculate the TRP from the EUT as

4)	The following tests shall be performed on the measurement data:

-	The dynamic range TRPEUTTRPambTRPEUT / TRPamb must be at least 20 dB

-	The number of uncorrelated samples, calculated via the auto-correlation function, see [35], shall be at least 250.

Note:	A reverberation chamber does not represent a real-life deployment for the EUT and can be considered as a hostile environment due to the potentially high field strengths. Therefore, it is important to make sure that the EUT is operating properly throughout the entire measurement.

##### 10.5.2.3A.4A	Test Method limitations and scope

The reverberation chamber test method is not suitable for testing spurious emissions at very low frequencies. The MU evaluation is applicable for the frequency above 380 MHz.

##### 10.5.2.3A.5	MU assessment

10.5.2.3A.5.1	MU Budget

Table 10.5.2.3A.5-1: Reverberation Chamber uncertainty contributions Tx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Uncertainty of the measurement equipment | E5-1 |
| 2 | Impedance mismatch in the receiving chain | E5-2 |
| 3 | Random uncertainty | E5-3 |
| Stage 1: Calibration measurement |  |  |
| 4 | Reference antenna radiation efficiency | E5-4 |
| 5 | Mean value estimation of reference antenna mis-match efficiency | E5-5 |
| 6 | Uncertainty of the Network Analyzer | E5-7 |
| 7 | Influence of the reference antenna feed cable | E5-8 |
| 8 | Mean value estimation of transfer function | E5-9 |
| 9 | Uniformity of transfer function | E5-10 |

10.5.2.3A.5.2	MU Value

Table 10.5.2.3A.5.2-1: Reverberation Chamber uncertainty assessment spurious emissions

| UID | Uncertainty source | Uncertainty value 380 MHz < f ≦ 3 GHz | Uncertainty value 3 GHz < f ≦ 3.8 GHz | Uncertainty value 3.8 GHz < f ≦ 12.75 GHz | Uncertainty value 12.75 GHz < f ≦ 19 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 380 MHz < f ≦ 3 GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 3.8 GHz | Standard uncertainty ui [dB] 3.8 GHz < f ≦ 12.75 GHz | Standard uncertainty ui [dB] 12.75 GHz < f ≦ 19 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.14 | 0.26 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.2 | 0.2 | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0.1 | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.13 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 | 1.5 | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.66 | 1.68 | 1.70 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.25 | 3.29 | 3.33 | 3.37 |
| NOTE: This MU budget is applicable if the data tests in procedure step 4) of sublause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 10.5.2.4	Test Tolerance

The conduced test tolerance for the mandatory spurious emissions requirements is zero. As the requirements are set by regulatory limits the same test tolerance is used for OTA.

TT=0.

#### 10.5.2.5	Summary

For general chamber the total MU is calculated by adding the MUperpoint and the SE (see subclause 10.8) we have:

,	30MHz<f≤6 GHz

,	6GHz<f≤19 GHz

By adding the MU per point and the SE values together we have the following total MU:

MUtotal = 2.3dB, 30MHz<f≤6GHz

MUtotal = 4.2dB, 6GHz<f≤19GHz

The test tolerance for mandatory spurious emissions is zero.

### 10.5.3	Receiver spurious emissions

#### 10.5.3.1	General

The conducted receiver spurious emission requirement MU is the same as for the TX spurious emissions, the measurement technique is the same and the power level is >-60dBm (where there is a break point for conducted power measurement accuracy), so this is reasonable.

For the OTA receiver emissions requirements however the lower power level of the requirement reduces the dynamic range of the TRP measurement and reduces measurement accuracy.

Considering that the loss in the chamber is based on the wanted signal being in the far field the per point noise floor is assumed to be approx -100dBm and the receiver emissions level translated to the test equipment is approx -90dBm. Hence the TRP calculation has only a 10dB dynamic range.

An uncertainty of 1dB is added to the TRP uncertainty budget to account for this additional uncertainty.

#### 10.5.3.2	General chamber

##### 10.5.3.2.1	General

As the AAS BS antenna radiating dimensions are fixed then the far field distance increases (FF≈2d2/λ). At 12.75 GHz the far field distance for a 1.5m AAS BS antenna array is almost 200m, this is clearly impractical in an indoor chamber (and the path loss would also make measurement difficult), so spurious emission testing will not always be in the far field. This is acceptable as the requirement is TRP and hence it is not necessary to measure in the far field however it needs to be considered when looking at MU.

Considerations of the large frequency range must also be considered, including the chamber performance (quiet zone), the calibration effectiveness and the available reference and test antennas over the frequency range.

##### 10.5.3.2.2	Calibration

The calibration is the same as described in sub-clause 10.5.2.2.2.

##### 10.5.3.2.3	Procedure

The measurement procedure is the same as described in sub-clause 10.5.2.2.3.

##### 10.5.3.2.4	MU assessment

###### 10.5.3.2.4.1	MU Budget

Table 10.5.3.2.4.1-1: General  Chamber uncertainty contributions
Rx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E4-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | E4-2 |
| 3 | Quality of quiet zone | E4-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E4-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E4-5 |
| 6 | Phase curvature | E4-6 |
| 7 | Conducted measurement uncertainty | E4-22, F.2 |
| 8 | Impedance mismatch in the receiving chain | E4-8 |
| 9 | Random uncertainty | E4-9 |
| 23 | Measurement antenna frequency variation | E4-20 |
| 24 | FSPL estimation error | E4-21 |
| 25 | Test system frequency flatness | E4-19 |
| 26 | Measurement system dynamic range uncertainty | E4-21 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer |  |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E4-10 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | E4-11 |
| 13 | Quality of quiet zone | E4-12 |
| 14 | Polarization mismatch for reference antenna | E4-3 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E4-4 |
| 16 | Phase curvature | E4-5 |
| 17 | Uncertainty of the network analyzer | E4-6 |
| 18 | Influence of the reference antenna feed cable | E4-13, F.1 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E4-14 |
| 20 | Influence of the receiving antenna feed cable | E4-15 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E4-16 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E4-17 |

###### 10.5.3.2.4.2	MU Value

Table 10.5.3.2.4.2-1: General Chamber uncertainty assessment
Rx spurious emissions

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |  |  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 7 | Conducted measurement uncertainty (minus mismatch) | 1 | 1.99 | Gaussian | 1 | 1 | 1 | 1.99 |
| 8 | Impedance mismatch in the receiving chain | 0.2 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.32 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 23 | Measurement antenna frequency variation | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 24 | FSPL estimation error | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |
| 25 | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| 26 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | Gaussian | 1 | 1 | 0.51 | 0.51 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.2 | Gaussian | 1 | 1 | 0.13 | 0.2 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.22 | 2.13 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.39 | 4.18 |

#### 10.5.3.3	Void

#### 10.5.3.4	Test Tolerance

The conduced test tolerance for the receiver spurious emissions requirements is zero. However for OTA AAS BS the receiver spurious emissions requirements only apply to TTD in OFF mode. As such the limit is set by RAN4 to be considerably lower than the equivalent regulatory requirement.

In addition due to the difficulty in measuring low levels of TRP close to the measurement system noise floor the risk of false failures is high. As the risk is due to the noise floor of the measurement system it cannot be mitigated by BS design

Hence it has been agreed that for receiver spurious emissions the TT=MU.

#### 10.5.3.5	Summary

Adding the MUperpoint and the SE (see subclause 10.8) we have:

,	30MHz<f≤6 GHz

,	6GHz<f≤19 GHz

By adding the MU per point and the SE values together we have the following total MU:MUtotal = 2.5dB,	30MHz<f≤6GHz

MUtotal = 4.2dB,	6GHz<f≤19GHz

The TT values are as follows:

TT = 2.5dB,	30MHz<f≤6GHz

TT = 4.2dB,	6GHz<f≤19GHz

### 10.5.4	Additional (co-existence) spurious emissions

#### 10.5.4.1	General

The additional spurious emissions requirements consist of the co-existence emissions requirements, and some additional regional requirements such as the protection of PHS and 700 and 800 public safety.

The conducted MU are consistent with the mandatory spurious emissions MU but are split into different frequency sub-divisions

Table 10.5.4.1-1: Conducted additional spurious emissions requirements

| Name | Unit | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| --- | --- | --- | --- | --- |
|  |  | AAS | AAS | E-UTRA |
| 6.7.6.4 Transmitter spurious emissions, Additional spurious emission requirements | dB |  |  |  |
| > -60dBm, f ≤ 3.0GHz |  | 2.0 |  |  |
| > -60dBm, 3.0GHz < f ≤ 4.2GHz |  | 2.5 |  |  |
| > -60dBm, 4.2GHz < f ≤ 6.0GHz |  | 3.0 |  |  |
| > -60dBm, 6.0GHz < f ≤ 26.0GHz |  | x | x | x |
| ≤ -60dBm, f ≤ 3.0GHz |  | 3.0 |  |  |
| ≤ -60dBm, 3.0GHz < f ≤ 4.2GHz |  | 3.5 |  |  |
| ≤ -60dBm, 4.2GHz < f ≤ 6.0GHz |  | 4.0 |  |  |
| ≤ -60dBm, 6.0GHz < f ≤ 26.0GHz |  | x | x | x |

The lower power levels (≤ -60dBm ) are only needed for home BS, as we have no home BS in AAS these can be ignored.

The test set up for the OTA additional emissions requirements is the same as that for the mandatory spurious emissions in Figure 10.5.2.1-1. However, the additional spurious emissions are at a much lower level than the mandatory requirements so the additional effect of the test system dynamic range must be considered in the same way as the receiver emissions requirements.

Unlike the other spurious emissions requirements the additional (co-existence) requirements are specified for other 3GPP bands and as such can be measured in the same chambers as the in-band measurements.

As the CATR MU budget results is the largest MU and is used for setting the in-band MU values only the CATR MU is analysed below. However any suitable IAC or near field) chamber can be used.

#### 10.5.4.2	CATR

##### 10.5.4.2.1	General

The CATR method only is described as it provides the worst MU budget for the additional requirements MU analysis.

##### 10.5.4.2.2	Calibration

The same calibration is carried out as the EIRP calibration described in TR 37.82 [4]

Note: This stage may be omitted provided calibration stage has been performed already during output power measurement

##### 10.5.4.2.3	Procedure

The reference procedure can be found in subclause 10.4.1.3.3.

The appropriate parameters in step 5 is the mean power of additional spurious emissions test over the measurement BW described in the test requirement

##### 10.5.4.2.4	MU assessment

###### 10.5.4.2.4.1	MU Budget

Table 10.5.4.2.4.1-1: Compact antenna test range uncertainty contributions for AAS BS OTA additional spurious emissions measurement

| UID | Description of uncertainty contribution | Details in annex of [1] |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | Conducted measurement uncertainty (minus mismatch) | E4-22, F.2 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 5 | QZ ripple DUT | E2-5 |
| 19 | Miscellaneous uncertainty | E2-14 |
| 25 | Measurement system dynamic range uncertainty | E4-22 |
| 26 | Test system frequency flatness | E2-16 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | F.1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | F.1 |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |

###### 10.5.4.2.4.2	MU Value

Table 10.5.4.2.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA additional spurious emissions measurement

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f≤3 GHz | 3GHz<f ≤4.2GHz | 4.2GHz<f ≤6GHz |  |  |  | f≤3 GHz | 3GHz<f ≤4.2GHz | 4.2GHz<f ≤6GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 | 0 |
| 2 | Conducted measurement uncertainty (minus mismatch) | 1.02 | 1.28 | 1.53 | Gaussian | 1 | 1 | 1.02 | 1.28 | 1.53 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 | 0.0928 |
| 25 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | 0.51 | Normal | 1 | 1 | 0.51 | 0.51 | 0.51 |
| 26 | Test system frequency flatness | 0.25 | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.2 | 0.2 | Normal | 1 | 1 | 0.13 | 0.2 | 0.2 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.1 | 0.1 | 0.1 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.5 | 0.433 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.27 | 1.50 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 2.48 | 2.93 | 3.37 |

#### 10.5.4.3	Void

#### 10.5.4.4	Test Tolerance

The conduced test tolerance for the additional spurious emissions requirements is zero.

However for OTA AAS BS, the difficulty in measuring TRP of the additional spurious emissions requirements at low levels close to the measurement system noise floor means the high risk of false failures. As the risk is due to the noise floor of the measurement system it cannot be mitigated by BS design.

As the 3GPP to 3GPP co-existence requirements are not regulatory but set by RAN4 to assist with co-existence of 3GPP systems in the same geographical area it is acceptable for RAN4 to set the TT value to be non-zero.

Hence it has been agreed that for 3GPP to 3GPP co-existence spurious emissions the TT=MU.

Some additional requirements such as the protection of PHS and the 700 and 800MHz  public safety bands, are regulatory so it is not possible to have anon zero TT, hence for these requirements the TT is zero.

#### 10.5.4.4	Summary

Adding the MUperpoint and the SE (see subclause 10.8) we have:

,	f≤3 GHz

,	3GHz<f≤4.2 GHz

,	4.2GHz<f≤6 GHz

By adding the MU per point and the SE values together we have the following total MU:

MUtotal = 2.6dB,								f≤3 GHz

MUtotal = 3.0dB,	3GHz<f≤4.2 GHz

MUtotal = 3.5dB,	4.2GHz<f≤6 GHz

For additional requirements for co-existence between 3GPP bands the TT values are as follows:

TT = 2.6dB,	f≤3 GHz

TT = 3.0dB,	3GHz<f≤4.2 GHz

TT = 3.5dB,	4.2GHz<f≤6 GHz

For PHS, and public safety additional requirements the TT=0dB.

## 10.6	Measurement uncertainty for Co-location requirements

### 10.6.1	General

Conformance to co-location requirements is shown using co-location proximity method which is described in 3GPP TS 37.145-2, subclause 4.15 [24].

### 10.6.2	OTA transmitter OFF power

#### 10.6.2.1	General

OTA transmitter ON/OFF power requirements apply only to TDD operation of E-UTRA.

The OTA Transmit ON/OFF power requirements are co-location requirements and specified as the power sum of the supported polarization(s) at the co-location reference antenna conducted output(s).

#### 10.6.2.2	In-door anechoic chamber

##### 10.6.2.2.1	General

This method measures the OTA transmitter OFF power in an anechoic chamber

##### 10.6.2.2.2	Calibration

See sub-clause 10.6.2.3.2.

##### 10.6.2.2.3	Procedure

See sub-clause 10.6.2.3.3.

##### 10.6.2.2.4	MU assessment

###### 10.6.2.2.4.1	MU Budget

###### 10.6.2.2.4.2	MU Value

#### 10.6.2.3	CATR

##### 10.6.2.3.1	General

This method measures the OTA transmitter OFF power in a compact antenna test range chamber.

##### 10.6.2.3.2	Calibration

Calibration for wanted signal power level is the same as in subclause 10.2.3.2.2.

Additionally, the losses in the signal chain between [co-location reference antenna] conducted output(s) and measurement equipment need to be calibrated out:

1)	Path loss calibration

a)	Measure the loss in signal chain from each output of [co-location reference antenna] to input of measurement equipment, call this L ANT↔C which is the equivalent of 20log|S21| from the use of a network analyser.

##### 10.6.2.3.3	Procedure

1)	Place AAS BS and [co-location reference antenna] as specified in subclause 4.15, 3GPP TS 37.145-2 [24].

2)	Place Range antenna in boresight direction (reference direction) at far-field distance, aligned in both polarizations with the AAS BS.

3)	The Range antenna shall be dual (or single) polarized with the same frequency range as the AAS BS for transmitter OFF power test case.

4)	Connect Range antenna and [co-location reference antenna] to the measurement equipment.

5)	OTA transmitter OFF power is measured at the [co-location reference antenna] conducted output(s).

6)	The measurement device (signal analyser) characteristics shall be:

-	Detection mode: True RMS.

7)	Set the AAS BS to transmit:

-	For AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit full beam power (rated beam EIRP), both polarizations, in boresight direction (reference direction), at manufacturer's declared rated output power, Prated,t,TRP.

-	For AAS BS declared to be capable of multi-carrier and/or CA operation, set the AAS BS to transmit full beam power (rated beam EIRP), both polarizations, in boresight direction (reference direction) on all carriers configured using the applicable test configuration and corresponding power setting.

-	Check that specified beam power (EIRP) is obtained at Range antenna RF output(s) (conducted side) for each polarization. Re-align if the specified beam power is not achieved.

8)	Measure the mean power spectral density from all [co-location reference antenna] conducted output(s) over 70μs filtered with a square filter of bandwidth equal to the RF bandwidth of the RIB centred on the central frequency of the RF bandwidth. 70μs average window centre is set from 35μs after end of one transmitter ON period + 17μs to 35μs before start of next transmitter ON period - 6.25μs.

Additional factor to be considered in the measurement is that the test requirement is very close to thermal noise floor, and the measurement setup needs to be able to tolerate both very high and low signal levels. This will impact measurement uncertainty.

##### 10.6.2.3.4	MU assessment

###### 10.6.2.3.4.1	MU Budget

###### 10.6.2.3.4.2	MU Value

#### 10.6.2.4	General chamber

##### 10.6.2.4.1	General

This method measures the OTA TDD OFF power level characteristics in a general OTA chamber.

##### 10.6.2.4.2	Calibration

##### 10.6.2.4.3	Procedure

##### 10.6.2.4.4	MU assessment

###### 10.6.2.4.4.1	MU Budget

###### 10.6.2.4.4.2	MU Value

Table 10.6.2.4.4.2-1: MU for TDD OFF power level

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
|  | Uncertainty related to the selection of the CLTA (Note) | 1.50 | 1.50 | 1.50 | Rectangular | √3 | 1 | 0.87 | 0.87 | 0.87 |
|  | Uncertainty related to the placement of the CLTA (Note) | 1.70 | 1.70 | 1.70 | Rectangular | √3 | 1 | 0.98 | 0.98 | 0.98 |
| 2 | Uncertainty related to measuring close to noise floor | 1.00 | 1.00 | 1.00 | Gaussian | 1 | 1 | 1.00 | 1.00 | 1.00 |
|  | Impedance mismatch between feeder cable and CLTA | 0.14 | 0.23 | 0.25 | U-Shaped | √2 | 1 | 0.10 | 0.16 | 0.18 |
|  | Gain variations in LNA | 0.10 | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 | 0.10 |
| 9 | Random uncertainty | 0.10 | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
|  | Measurement receiver accuracy | 0.41 | 0.74 | 0.80 | Gaussian | 1 | 1 | 0.41 | 0.74 | 0.80 |
|  | Reflections in anechoic chamber | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
|  | Gain variations in measurement amplifier | 0.10 | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 | 0.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 7 | Noise figure measurement accuracy | 0.2 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.72 | 1.83 | 1.86 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 3.36 | 3.58 | 3.64 |
| NOTE: It has been recognized that the selection and exact placement of the co-location test antenna will introduce uncertainty on the co-location measurement. Therefore, CLTA MU needs to be included to the MU budgets. Parameters for acceptable co-location test antennas and its placement are specified in the TS. Fulfilling these criteria for CLTA selection and placement is deemed sufficient for the test purposes. Therefore, measurement uncertainty related to the selection of the co-location test antenna and its alignment does not need to be analysed. Instead, the MU documented in the TR and TS can be directly used for total test system uncertainty evaluation and the test system is considered to meet the MU by meeting the CLTA selection and placement criteria. |  |  |  |  |  |  |  |  |  |  |

#### 10.6.2.5	Summary

### 10.6.3	OTA co-location spurious emissions

#### 10.6.3.1	General

#### 10.6.3.2	In-door anechoic chamber

##### 10.6.3.2.1	General

This method measures the OTA co-location spurious emissions in an anechoic chamber

##### 10.6.3.2.2	Calibration

See sub-clause 10.6.X.3.2.

##### 10.6.3.2.3	Procedure

See sub-clause 10.6.X.3.3.

##### 10.6.3.2.4	MU assessment

###### 10.6.3.2.4.1	MU Budget

###### 10.6.3.2.4.2	MU Value

#### 10.6.3.3	CATR

##### 10.6.3.3.1	General

This method measures the OTA co-location spurious emissions in a compact antenna test range chamber.

##### 10.6.3.3.2	Calibration

Calibration for wanted signal power level is the same as in subclause 10.2.3.3.2.

Additionally, the losses in the signal chain between [co-location reference antenna] conducted output(s) and measurement equipment need to be calibrated out:

1)	Path loss calibration

a)	Measure the loss in signal chain from each output of [co-location reference antenna] to input of measurement equipment, call this L ANT↔C which is the equivalent of 20log|S21| from the use of a network analyser.

##### 10.6.3.3.3	Procedure

1)	Place AAS BS and [co-location reference antenna] as specified in subclause 4.15, 3GPP TS 37.145-2 [24].

2)	Place Range antenna in boresight direction (reference direction) at far-field distance, aligned in both polarizations with the AAS BS.

3)	Connect Range antenna and [co-location reference antenna] to the measurement equipment.

4)	OTA co-location spurious emissions are measured at the [co-location reference antenna] conducted output(s).

5)	The measurement device (signal analyser) characteristics shall be:

-	Detection mode: True RMS.

6)	Set the AAS BS to transmit:

-	For AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit full beam power (rated beam EIRP), both polarizations, either simultaneously or sequentially, in boresight direction (reference direction), at manufacturer's declared rated output power, Prated,t,TRP.

-	For AAS BS declared to be capable of multi-carrier and/or CA operation, set the AAS BS to transmit full beam power (rated beam EIRP), both polarizations, in boresight direction (reference direction) on all carriers configured using the applicable test configuration and corresponding power setting.

-	Check that specified beam power (EIRP) is obtained at Range antenna RF output(s) (conducted side) for each polarization. Re-align if the specified beam power is not achieved.

7)	Measure the mean power spectral density from all [co-location reference antenna] conducted output(s) with each wanted signal polarization under test.

Additional factor to be considered in the measurement is that the test requirement is very close to thermal noise floor, and the measurement setup needs to be able to tolerate both very high and low signal levels. This will impact measurement uncertainty.

##### 10.6.3.3.4	MU assessment

###### 10.6.3.3.4.1	MU Budget

###### 10.6.3.3.4.2	MU Value

#### 10.6.3.4	General chamber

##### 10.6.3.4.1	General

This method measures the OTA co-location spurious emission characteristics in a general OTA chamber.

##### 10.6.3.4.2	Calibration

##### 10.6.3.4.3	Procedure

##### 10.6.3.4.4	MU assessment

###### 10.6.3.4.4.1	MU Budget

###### 10.6.3.4.4.2	MU Value

Table 10.6.3.4.4.2-1: MU for OTA co-location spurious emission

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
|  | Uncertainty related to the selection of the CLTA (Note) | 1.50 | 1.50 | 1.50 | Rectangular | √3 | 1 | 0.87 | 0.87 | 0.87 |
|  | Uncertainty related to the placement of the CLTA (Note) | 1.70 | 1.70 | 1.70 | Rectangular | √3 | 1 | 0.98 | 0.98 | 0.98 |
| 2 | Uncertainty related to measuring close to noise floor | 0.68 | 0.68 | 0.68 | Gaussian | 1 | 1 | 0.68 | 0.68 | 0.68 |
|  | Impedance mismatch between feeder cable and CLTA | 0.14 | 0.23 | 0.25 | U-Shaped | √2 | 1 | 0.10 | 0.16 | 0.18 |
|  | Gain variations in LNA | 0.10 | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 | 0.10 |
| 9 | Random uncertainty | 0.10 | 0.10 | 0.10 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
|  | Measurement receiver accuracy | 0.41 | 0.74 | 0.80 | Gaussian | 1 | 1 | 0.41 | 0.74 | 0.80 |
|  | Reflections in anechoic chamber | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
|  | Gain variations in measurement amplifier | 0.10 | 0.10 | 0.10 | Gaussian | 1 | 1 | 0.10 | 0.10 | 0.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 7 | Noise figure measurement accuracy | 0.2 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.56 | 1.68 | 1.71 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 3.10 | 3.29 | 3.35 |
| NOTE: It has been recognized that the selection and exact placement of the co-location test antenna will introduce uncertainty on the co-location measurement. Therefore, CLTA MU needs to be included to the MU budgets. Parameters for acceptable co-location test antennas and its placement are specified in the TS. Fulfilling these criteria for CLTA selection and placement is deemed sufficient for the test purposes. Therefore, measurement uncertainty related to the selection of the co-location test antenna and its alignment does not need to be analysed. Instead, the MU documented in the TR and TS can be directly used for total test system uncertainty evaluation and the test system is considered to meet the MU by meeting the CLTA selection and placement criteria. |  |  |  |  |  |  |  |  |  |  |

#### 10.6.3.5	Summary

{editors note: Last section at this level summarises MU estimates for all chambers and concludes final agreed MU}

### 10.6.4	OTA Transmitter intermodulation

#### 10.6.4.1	General

#### 10.6.4.2	In-door anechoic chamber

##### 10.6.4.2.1	General

This method measures the transmitter intermodulation in an anechoic chamber. The separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna depends on the unwanted emission requirements, ACLR, OBUE, SEM, and spurious emission, to be measured under the condition of transmitter intermodulation. See sub-clause 10.4.2, 10.4.3, 10.4.4, and 10.5.x. The measurement system setup is as depicted in figure 10.6.x.2.1-1.

{editor’s note: the separation is [] because the out-of-band spurious emission test may not satisfy the condition that the separation is no less than 2D2/λ.}

(a)	General set-up (top view)
(Positioner is not described here)

{editor’s note: detail of the co-location [test] antenna placement will be described here}

(b)	Detailed placement of co-location [test] antenna

Figure 10.6.4.2.1-1: In-door Anechoic Chamber measurement system setup for transmitter intermodulation

{editor’s note: Diagrams needs to be updated to show the co-location reference antenna and AAS BS are on the same position  and also the PA associated with the interferer signal. }

##### 10.6.4.2.2	Calibration

Calibration of the emission measurements shall be done with the procedure as shown in 10.2.3.2.2.

For calibration of [co-location reference antenna] part is FFS.

{editors note: we need to include calibration of the input to the collocation test antenna}

##### 10.6.4.2.3	Procedure

1)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

2)	Set the co-location test antenna] as specified in 3GPP TS37.145-2 sub-clause 4.15 [24], [at the declared worst case side.]

3)	Set the signal generator to generate the interfering signal to achieve the required level at the input port of co-location test antenna].

4)	Perform required ACLR, OBUE (for E-UTRA), SEM (for UTRA), occupied bandwidth, spurious emission tests according to the corresponding testing procedures.

##### 10.6.4.2.4	MU assessment

###### 10.6.4.2.4.1	MU Budget

[The MU of the unwanted emission (e.g. ACLR, OBUE, SEM, spurious emission) is the same as the MU of corresponding unwanted emission requirement. The MU of TX-IM is the uncertainty of the interference signal level at AAS BS.]

Table 10.6.4.2.4.1-1 MU contributors for the CLTA interferer signal

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Uncertainty of interference signal input level to co-location [test] antenna |  |  |
| 1 | Uncertainty of the RF signal generator |  |
| 2 | Uncertainty of the RF power measurement equipment |  |
| 3 | Impedance mismatch in the transmitting chain |  |
| 4 | Random uncertainty |  |
| 5 | Gain variation of power amplifier |  |
| 6 | Influence of the transmitting antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability |  |
| [7] | [Uncertainty of the network analyser at co-location [test] antenna calibration stage] |  |
| Uncertainty of co-location [test] antenna |  |  |
| [8] | Separation “d” error |  |
| [9] | Vertical alignment error |  |
| [10] | Front-back alignment error |  |
| [11] | Vertical radiation length “h” error |  |
| [12] | Polarization error |  |
| [13] | Beam width error |  |

{editor note: relationship between error sources need to be assessed regarding the CLTA variation}

###### 10.6.4.2.4.2	MU Value

Table 10.6.4.2.4.2-1: MU for the CLTA interferer signal

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uncertainty of interference signal input level to co-location [test] antenna |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the RF signal generator | 0.46 | 0.46 | Gaussian | 1.00 | 1 | 0.46 | 0.46 |
| 2 | Uncertainty of the RF power measurement equipment | 0.14 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 |
| 3 | Impedance mismatch in the transmitting chain | 0.14 | 0.23 | U-shaped | 1.41 | 1 | 0.10 | 0.16 |
| 4 | Random uncertainty | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 |
| 5 | Gain variation of power amplifier | [0.02] | [0.02] | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 6 | Influence of the transmitting antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 |
| [7] | [Uncertainty of the network analyser at co-location [test] antenna calibration stage] | [0.13] | [0.20] | [Gaussian] | [1.00] | [1] | [0.13] | [0.20] |
| Uncertainty of co-location [test] antenna |  |  |  |  |  |  |  |  |
| [8] | Separation “d” error |  |  |  |  |  |  |  |
| [9] | Vertical alignment error |  |  |  |  |  |  |  |
| [10] | Front-back alignment error |  |  |  |  |  |  |  |
| [11] | Vertical radiation length “h” error |  |  |  |  |  |  |  |
| [12] | Polarization error |  |  |  |  |  |  |  |
| [13] | Beam width error |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |
| Expanded uncertainty (1.96σ – confidence interval of 95%) [dB] ue = 1.96uc |  |  |  |  |  |  |  |  |

{editor note: relationship between error sources need to be assessed regarding the CLTA variation}

#### 10.6.4.3	CATR

##### 10.6.4.3.1	General

This method measures the OTA transmitter intermodulation in a compact antenna test range chamber. CATR can be used only within the frequency range where the test facility is functional.

##### 10.6.4.3.2	Calibration

Calibration for wanted signal power level is the same as in subclause 10.2.3.3.2.

Additionally, the losses in the signal chain between [co-location reference antenna] conducted input(s) and measurement equipment need to be calibrated out:

1)	able and matching loss calibration for co-location reference antenna

-	For each polarization supported by the co-location reference antenna

a)	Measure co-location reference antenna reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓANT.

b)	Measure cable loss from point F to input of co-location reference antenna, call this L ANT↔F which is the equivalent of 20log|S21| from the use of a network analyser.

c)	Calculate the combined total path loss from F to co-location reference antenna using the following equating

Lcal = L ANT↔F + 10log(1 - |ΓANT|2)

##### 10.6.4.3.3	Procedure

1)	Set up AAS BS (DUT) in place of SGH from the calibration stage, ensure that the [co-location reference antenna] and DUT are positioned as described in 3GPP TS 37.145-2, subclause 4.15 [24], [co-location reference antenna] placed at the declared worst-case side.

2)	Place Range antenna in boresight direction (reference direction) at far-field distance, aligned in both polarizations with the AAS BS.

3)	Connect Range antenna and co-location reference antenna to the measurement equipment.

4)	Configure measurement equipment to transmit the blocking signal through each polarization of the [co-location reference antenna] at appropriate level for the BS class as specified in section 6.X.X

5)	Configure all OTA AAS BS TX branches according to a maximum power requirement for a single carrier.

6)	Set the DUT to transmit the test signal according to appropriate test model.

7)	Measure unwanted emissions according to subclauses 10.4 and 10.5

##### 10.6.4.3.4	MU assessment

###### 10.6.4.3.4.1	MU Budget

###### 10.6.4.3.4.2	MU Value

#### 10.6.4.4	Shielded Anechoic Chamber

##### 10.6.4.4.1	General

This method measures the OTA transmitter intermodulation emissions in a shielded anechoic chamber.

Figure 10.6.4.4.1-1: OTA transmitter intermodulation

A power amplifier is needed to amplify the interferer signal generated by the signal generator to the level specified by the requirement.

##### 10.6.4.4.2	Calibration

The calibration of the test range is conducted as described for unwanted emission in sub-clause [x].

The calibration is conducted according to sub-clause 10.6.3.3.2.

##### 10.6.4.4.3	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place AAS BS and co-location reference antenna as specified, at the distance d=0.1m.

2)	Place test antenna at sufficient distance, aligned to supported polarizations with the AAS BS.

3)	The test antenna(s) shall be dual (or single) polarized covering the same frequency range as the AAS BS and the emission frequencies.

4)	Several test antennas are required to cover both the AAS BS and the whole emission frequency range.

5)	Connect test antenna and co-location reference antenna to the measurement equipment.

6)	During the OTA emission measurements at the test antenna conducted output(s), both AAS BS and co-location reference antenna are rotated around same axis.

7)	The OTA emission measurement method shall be TRP, according to the procedure described in subclause 10.8.

8)	The measurement device (signal analyzer) characteristics shall be:

-	Detection mode: True RMS.

9)	Set the AAS BS to transmit:

-	Set the AAS BS to transmit maximum power according to the applicable test configuration using the corresponding test models or set of physical channels.

-	Generate the interfering signal via the co-location reference antenna. The co-location reference antenna is fed with Prated,t,TRP, equally divided on all supported polarizations, from the same signal generator source.

10)	Perform the unwanted emission tests specified in OTA ACLR, OTA spectrum mask and OTA OBUE, for all third and fifth order intermodulation products which appear in the frequency ranges. The width of the intermodulation products shall be taken into account.

11)	Perform the transmitter spurious emissions test as specified in OTA spurious emission, except OTA co-location spurious emission, for all third and fifth order intermodulation products which appear in the frequency ranges. The width of the intermodulation products shall be taken into account.

12)	Verify that the emission level does not exceed the required level with the exception of interfering signal frequencies.

13)	Repeat the test for the remaining interfering signal centre frequency offsets.

14)	Repeat the test for the remaining interfering signals for requirements OTA ACLR, OTA spectrum mask, OTA OBUE and OTA spurious emission, except OTA co-location spurious emission.

In addition, for multi-band AAS BS, the following steps shall apply:

15)	For multi-band AAS BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 10.6.4.4.4	MU assessment

###### 10.6.4.4.4.1	MU Budget

###### 10.6.4.4.4.2	MU Value

#### 10.6.4.5	General chamber

##### 10.6.4.5.1	General

This method measures the OTA co-location blocking characteristics in a general OTA chamber.

##### 10.6.4.5.2	Calibration

##### 10.6.4.5.3	Procedure

##### 10.6.4.5.4	MU assessment

###### 10.6.4.5.4.1	MU Budget

###### 10.6.4.5.4.2	MU Value

Table 10.6.4.5.4.2-1 MU for transmitter intermodulation interferer signal

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
|  | Uncertainty related to the selection of the CLTA (Note) | 1.5 | 1.5 | 1.5 | Rectangular | √3 | 1 | 0.87 | 0.87 | 0.87 |
|  | Uncertainty related to the placement of the CLTA (Note) | 1.7 | 1.7 | 1.7 | Rectangular | √3 | 1 | 0.98 | 0.98 | 0.98 |
|  | Impedance mismatch between feeder cable and CLTA | 0.14 | 0.23 | 0.25 | U-Shaped | √2 | 1 | 0.10 | 0.16 | 0.18 |
| 9 | Random uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
|  | Reflections in anechoic chamber | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 7 | Conducted measurement uncertainty | 1.0 | 1.1 | 1.2 | Gaussian | 1 | 1 | 1.00 | 1.10 | 1.20 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.65 | 1.72 | 1.79 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 3.23 | 3.37 | 3.51 |
| NOTE: It has been recognized that the selection and exact placement of the co-location test antenna will introduce uncertainty on the co-location measurement. Therefore, CLTA MU needs to be included to the MU budgets. Parameters for acceptable co-location test antennas and its placement are specified in the TS. Fulfilling these criteria for CLTA selection and placement is deemed sufficient for the test purposes. Therefore, measurement uncertainty related to the selection of the co-location test antenna and its alignment does not need to be analysed. Instead, the MU documented in the TR and TS can be directly used for total test system uncertainty evaluation and the test system is considered to meet the MU by meeting the CLTA selection and placement criteria. |  |  |  |  |  |  |  |  |  |  |

#### 10.6.4.5	Summary

{Editors note: Last section at this level summarises MU estimates for all chambers and concludes final agreed MU}

### 10.6.5	OTA co-location blocking

#### 10.6.5.1	General

#### 10.6.5.2	In-door anechoic chamber

##### 10.6.5.2.1	General

This method measures the OTA co-location blocking in an anechoic chamber

##### 10.6.5.2.2	Calibration

See sub-clause 10.6.5.3.2.

##### 10.6.5.2.3	Procedure

See sub-clause 10.6.5.3.3.

##### 10.6.5.2.4	MU assessment

{Editor’s note: MU assessment for OTA co-location blocking in in-door anechoic chamber.}

###### 10.6.5.2.4.1	MU Budget

###### 10.6.5.2.4.2	MU Value

#### 10.6.5.3	CATR

##### 10.6.5.3.1	General

This method measures the OTA co-location blocking in a compact antenna test range chamber.

Figure 10.6.5.3.1-1: General OTA co-location blocking test set-up

##### 10.6.5.3.2	Calibration

Calibration for wanted signal power level is the same as in subclause 10.2.3.3.2

Additionally, the losses in the signal chain between co-location reference antenna conducted output(s) and measurement equipment need to be calibrated out:

a)	Measure the loss in signal chain from each output of co-location reference antenna to input of measurement equipment, call this L ANT↔C which is the equivalent of 20log|S21| from the use of a network analyser.

##### 10.6.5.3.3	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction of the BS according to declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit the beam(s) of the same operational band and RAT as the OSDD being tested according to the appropriate test configuration.

7)	Start the signal generator for the wanted signal to transmit: The test signal as specified.

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point as specified

9)	Set the AAS BS to transmit:

For AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit maximum power corresponding to manufacturer's declared rated output power, Prated,t,TRP.

10)	Configure measurement equipment to transmit the blocking signal at appropriate level and polarization as specified

11)	Measure: Throughput for each supported polarization with each interferer polarization

Repeat steps 9-10 for all frequency ranges to be tested. In case interferer antenna needs to be changed the calibration procedure for interferer power level needs to be repeated.

##### 10.6.5.3.4	MU assessment

###### 10.6.5.3.4.1	MU Budget

###### 10.6.5.3.4.2	MU Value

#### 10.6.5.5	General chamber

##### 10.6.5.5.1	General

This method measures the OTA co-location blocking characteristics in a general OTA chamber.

##### 10.6.5.5.2	Calibration

##### 10.6.5.5.3	Procedure

##### 10.6.5.5.4	MU assessment

###### 10.6.5.5.4.1	MU Budget

###### 10.6.5.5.4.2	MU Value

Table 10.6.5.5.4.2-1 MU for co-location blocking interferer signal

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
|  | Uncertainty related to the selection of the CLTA (Note) | 1.5 | 1.5 | 1.5 | Rectangular | √3 | 1 | 0.87 | 0.87 | 0.87 |
|  | Uncertainty related to the placement of the CLTA (Note) | 1.7 | 1.7 | 1.7 | Rectangular | √3 | 1 | 0.98 | 0.98 | 0.98 |
|  | Impedance mismatch between feeder cable and CLTA | 0.14 | 0.23 | 0.25 | U-Shaped | √2 | 1 | 0.10 | 0.16 | 0.18 |
| 9 | Random uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
|  | Reflections in anechoic chamber | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 7 | Conducted measurement uncertainty | 1.0 | 1.1 | 1.2 | Gaussian | 1 | 1 | 1.00 | 1.10 | 1.20 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.65 | 1.72 | 1.79 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 3.23 | 3.37 | 3.51 |
| NOTE: It has been recognized that the selection and exact placement of the co-location test antenna will introduce uncertainty on the co-location measurement. Therefore, CLTA MU needs to be included to the MU budgets. Parameters for acceptable co-location test antennas and its placement are specified in the TS. Fulfilling these criteria for CLTA selection and placement is deemed sufficient for the test purposes. Therefore, measurement uncertainty related to the selection of the co-location test antenna and its alignment does not need to be analysed. Instead, the MU documented in the TR and TS can be directly used for total test system uncertainty evaluation and the test system is considered to meet the MU by meeting the CLTA selection and placement criteria. |  |  |  |  |  |  |  |  |  |  |

#### 10.6.5.5	Summary

{editors note: Last section at this level summarises MU estimates for all chambers and concludes final agreed MU}

## 10.7	Measurement uncertainty for Out-of-band Blocking requirements

### 10.7.1	General

The out of band blocking requirement requires both a wanted in-band signal and an interferer out of band signal to be transmitted with the chamber. The wanted signal is defined at in the far field, the interferer is defined as a field strength, due to the large range for frequencies it will not always be in the far field.

Hence any acceptable measurement chamber for the OTA sensitivity requirement is also suitable for the out of band blocking requirement, it may be necessary that the interfering signal is transmitted from a separate antenna due to the large frequency range of the interferer.

AAS BS

Test system Calibrated point

θ

ϕ

0

AAS declared coordinate reference point and orientation

Test system enclosure

Test antenna (out of band)

Signal Generator for the interfering signal

Signal Generator for the wanted signal

Test antenna (in band)

Test antenna polarisation can be adjusted

Figure 10.7.1-1: General OTA blocking test set-up using a different antenna

Worst case the wanted and interfering signal are transmitted from separate test antennas, hence they each may have a different uncertainty associated with the OTA chamber. This differs from the in-band interference measurements where the wanted signal and the interferer are added together outside the chamber and applied to the same test antenna and hence have a common OTA chamber uncertainty.

For the wanted signal the calibration procedure and MU for the OTA sensitivity in sub-clause 10.3.2 can be assumed for each chamber type.

The unwanted signal the signal is defined as a field strength so can be applied in the near field with the assumption that the test antenna fully illuminated the AAS BS under test. For the purposes of calculating a MU value of the interfering signal the same general out of band OTA chamber used for the spurious emissions requirement in sub-clause 10.5.2 is assumed.

The out of band blocking is analysed using the same methodology as the other receiver interference requirements where:

The MU of the wanted signal is the same as that for the OTA sensitivity requirement.

The MU for the broad band noise effect is the same as the conducted requirement (0.1dB).

The uncertainty of the interferer is analysed in this section using a general chamber assumption. The requirement may be tested in any suitable chamber (IAC, CATR) that is capable of measuring EIS accurately and also applying the out of band interferer. For interferer frequencies where it can be applied from a common antenna (like the in-band requirements) this is acceptable but it is expected the MU will be lower so will not influence the final MU value. The chosen chamber must of course be specified to handle the frequency of both the interfere and the wanted signal. The complete out of band blocking test may be completed using multiple chambers for different frequency ranges if necessary.

### 10.7.2	General Chamber

#### 10.7.2.1	General

A general chamber is analysed for the interferer MU value, this is considered worst case for setting the MU value.

#### 10.7.2.2	Calibration

For calibration of the wanted signal path see sub-clause 10.3.2.

The interferer path is calibrated using the same method with appropriate antennas.

#### 10.7.2.3	Procedure

1)	Place test antenna(s) in [FFS direction(s)] at appropriate distance, aligned in all supported polarizations (single or dual) with the AAS BS.

2)	Connect test antenna(s) to the measurement equipment.

3)	The test antenna(s) shall be dual (or single) polarized covering the same frequency ranges as the AAS BS and the blocking frequencies. If the test antenna does not cover both the wanted and interfering signal frequencies, separate test antennas for the wanted and interfering signal are required.

4)	The OTA blocking interferer is injected into the test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. The interferer shall be polarization matched to the AAS BS in band and the position maintained for OOB measurements.

5)	The AAS BS receives the wanted signal and the interferer signal for all supported polarizations (single or dual), in the reference direction from the test antenna(s).

#### 10.7.2.4	MU assessment

##### 10.7.2.4.1	MU Budget

Table 10.7.2.4.1-1: uncertainty assessment for out of band blocking interferer level

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna |  |
| 2 | Pointing misalignment between the AAS BS and the test antenna |  |
| 3 | Quality of quiet zone |  |
| 4 | Polarization mismatch between the AAS BS and the test antenna |  |
| 5 | Mutual coupling between the AAS BS and the test antenna |  |
| 6 | Phase curvature |  |
| 8 | Impedance mismatch in the test chain |  |
| 9 | Random uncertainty |  |
| 23 | Measurement antenna frequency variation |  |
| 24 | FSPL estimation error |  |
| 25 | Test system frequency flatness |  |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the test antenna and the network analyzer |  |
| 11 | Positioning and pointing misalignment between the reference antenna and the test antenna |  |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. |  |
| 13 | Quality of quiet zone |  |
| 14 | Polarization mismatch for reference antenna |  |
| 15 | Mutual coupling between the reference antenna and the test antenna |  |
| 16 | Phase curvature |  |
| 17 | Uncertainty of the network analyzer |  |
| 18 | Influence of the reference antenna feed cable |  |
| 19 | Reference antenna feed cable loss measurement uncertainty |  |
| 20 | Influence of the test antenna feed cable |  |
| 21 | Uncertainty of the absolute gain of the reference antenna |  |
| 22 | Uncertainty of the absolute gain of the test antenna |  |

##### 10.7.2.4.2	MU Value

Table 10.7.2.4.2-1: uncertainty assessment for out of band blocking interferer

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤3 GHz | 3GHz<f ≤6GHz | 6GHz<f ≤12.75GHz |  |  |  | 30MHz<f≤3 GHz | 3GHz<f ≤6GHz | 6GHz<f ≤12.75GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0 | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 | 0 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 | 0.1 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 | 0 |
| 6 | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 | 0.05 |
| 8 | Impedance mismatch in the receiving chain | 0.2 | 0.2 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 | 0.32 |
| 9 | Random uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
| 23 | Measurement antenna frequency variation | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
| 24 | FSPL estimation error | 0 | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 | 0 |
| 25 | Test system frequency flatness | 0.25 | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 | 0.1 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | 0 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
| 16 | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.13 | 0.2 | Gaussian | 1 | 1 | 0.13 | 0.13 | 0.2 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 | 0.29 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | 0 | Rectangular | √3 | 1 | 0.00 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 0.47 | 0.47 | 0.57 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 0.93 | 0.93 | 1.12 |

##### 10.7.2.4.3	Interferer MU total

The OTA chamber is one part of the total interferer MU value. The rest of the MU is from the conducted signal accuracy, for this the conducted accuracy is used as a guide. In addition care is taken to ensure the mismatch is not added twice (from the chamber and the conducted value) and also it is considered if a PA is needed to achieve sufficient test signal power level.

The interferer is specified as a field strength of 0.36V/m, the required conducted power level depends one chamber size (the FSPL) the test antenna gain and the conducted signal path, and example is given in table 10.7.2.4.3-1.

Table 10.7.2.4.3-1: Example of required conducted interferer level

| OOB blocking interferer level |  |  |
| --- | --- | --- |
| field strength | 0.36 | V/m |
| d | 4 | m |
| EIRP | 18.40 | dBm |
| Ant | 5 | dBi |
| cables, filers etc | 10 | dB |
| Sig gen | 23.40 | dBm |

Some signal generators can provide 23.4dBm but it is at the top end so it is sensible to consider a PA in the test set up.

The MUinterferer value is hence given by:

For each of the frequency ranges this gives the value in table 10.7.2.4.3-2.

Table 10.7.2.4.3-2: MUinterferer values

| Freq range | Unit | cond. int. (mod) | PA | cond. Matching | out of band chamber | OTA interferer |
| --- | --- | --- | --- | --- | --- | --- |
| 30MHz<f≤3 GHz | dB | 1 | 0.2 | 0.294 | 0.93 | 1.35 |
| 3GHz<f ≤6GHz |  | 1.2 | 0.2 | 0.294 | 0.93 | 1.50 |
| 6GHz<f ≤12.75 GHz |  | 3 | 0.2 | 0.55 | 1.12 | 3.16 |

### 10.7.3	Summary

The final MU is calculated as follows:

The Noiseeffect from the signal generator is 0.1dB and the MUwantedsignal value is MUEIS from 3GPP TR 37.842 [4].

The final values are given in table 10.7.3-1

Table 10.7.3-1: MU values for out of band blocking

| OTA |  |  |  |  |
| --- | --- | --- | --- | --- |
| OOB blocking MU (dB) |  | Wanted signal operating band |  |  |
|  |  | 30MHz<f≤3 GHz | 3GHz<f ≤4.2GHz |  |
| Interferer frequency | 30MHz<f≤3 GHz | 2.0 | 2.0 |  |
|  | 3GHz<f ≤6GHz | 2.1 | 2.1 |  |
|  | 6GHz<f ≤12.75GHz | 3.5 | 3.6 |  |

## 10.8	TRP measurement

### 10.8.1	General

The TRP or the radiated power is simply the total power radiated by an EUT, as in subclause 5.1.1. In theory, due to energy conservation, TRP is independent of choice of test distance.

In theory, TRPReference is defined as the integral of the EUT’s far field radiation pattern over a spherical surface, that is

where U(θ, ϕ)U(θ,ϕ) is the radiation intensity at each angle in watts per Steradian. In the far field, the radiation intensity can be defined as

Uθ,ϕ=EIRP(θ,ϕ)4π

Thus, the definite integral for TRPReference becomes

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

Since EIRP is defined only in the far field, some occasions the power measurements can be performed at distances less than 2d2/λ (the traditional approximate far-field distance ). It may be possible to measure power density in near field considering only the magnitude of the tangential part of E field, Etr,θ,ϕ, with an acceptable level of accuracy assuming far field conditions. Thus equation TRPReference can be expressed as

where PDr,θ,ϕ=Etr,θ,ϕ2η and η=μ0ε0≈377 Ω (intrinsic impedance of vacuum).

The minimum distance between the measurement antenna and the smallest sphere enclosing the EUT must be at least 2λ. More details about the necessary conditions for accurate power density measurements close to EUT are included in Annex G.

In practice, discrete samples of EIRP or power density are measured at different directions (θ,ϕ) over the entire sphere, which are used to numerically approximate the surface TRPReference integral.  The obtained value, TRPEstimate, is an approximation of TRPReference and the difference between them is defined as the TRP summation error (SE) which is

SE = | TRPEstimate – TRPreference |   (in dB)

The SE is the error caused by the numerical integration of measured values on the grid to obtain TRPEstimate, given that the number of test points correspond to the reference angular steps, which are in turn dependent on antenna size and frequency, as described in subclause 10.8.2. A reasonable tradeoff between accuracy and sampling time is achieved when the SE = 0,75 dB.

When measuring radiated power, at each measurement point, two partial results for two orthogonal polarizations needs to be added. These can be the θ and φ polarizations or any pair of orthogonal polarizations.

The distribution of sampling points on the spherical surface depends the type of sampling grids applied. In following subclauses, we describe several spherical sampling grids which can be applied to EIRP or power density measurements.

### 10.8.2	TRP measurement grids

#### 10.8.2.1	Spherical equal angle grid

With the spherical equal angle grid, the grid spacing is uniform in the θ and ϕ directions. The range of θ angles from 0 to π is divided into N equally spaced subintervals and the range of ϕ angles from 0 to 2π is divided into M equally spaced subintervals. The width of each subinterval in the θ- and ϕ-angle is given as:

Δθ= πN and ∆ϕ= 2πM

The total number of angular sampling points is equal to (N+1)×M.

Let n and m be the indices used to denote the nth θ and mth ϕ angles, respectively. In practice, discrete samples of EIRP are measured at each sample point (θn, ϕm) by measuring its two orthogonally polarized components, EIRPp1(θn,ϕm) and EIRPp2(θn,ϕm). The EIRP sample are then used to approximate the definite integral for TRPReference as the discrete average sum of EIRP measured at different θ and ϕ angles.

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn.

The above equation can be simplified considering sinθ0 = sinθN = 0. Thus the total number of angular sampling points is equal to (N – 1)M.

Note TRPEstimate = TRPReference as N and M approach ∞.

There is a trade-off between the accuracy of the TRPEstimate and the total number of sampling points. A large number of sampling points leads to long measurement time. Thus, it is important to achieve short measurement time and fulfilling the minimum TRP summation error. Subclause 10.8.2.2 outlines the criteria for determining the minimum number of sampling points to characterize. Other means for set the number of sampling points are not precluded.

Figure 10.8.2.1-1: Void

One observation is that the equal angle grid points are not uniformly distributed on the sphere surface, and many are clustered towards the poles, as shown in Figure 10.8.2.1-2.

Figure 10.8.2.1-2: Spherical equal angle sampling grid

Figure 10.8.2.1-3: Void

#### 10.8.2.2	Reference angular step criteria

For each frequency, the reference angular steps ΔϕrefΔϕ and Δθref,Δθ in degrees, are calculated as [29]:

,

where D and Dcyl are defined in subclause 10.8.2.1. This implies a maximum angular step of 15 degrees. The upper limit for these reference angular steps of 15∘ ensures a low Summation Error (SE) when λ is large compared to the DUT dimensions.

The reference steps can be derived as follows. Consider two short vertical current elements separated a distance L along the z-axis. The EIRP pattern of this source is

EIRPθ,ϕ=sin2θcos2kcosθL2

Here, the element factor is sin2θ and cos2(kcosθ L/2) is the array factor contribution. To calculate the TRP value correctly, an angular sampling of Δθref=λ/L is required, see Figure 10.8.2.2-1. But a single ϕ is enough since the pattern is ϕ-independent (omni-directional).

Any current flowing on a line between the points x,y,z=(0,0,±L/2) will correspond to source separations less than or equal to L. Hence its EIRP pattern will correspond to the same angular resolution, i.e., the average value will be correctly predicted using the same angular step.

Figure 10.8.2.2-1: The average EIRP when using different angular steps Δθ and the EIRP pattern of two short vertical current elements separated a distance L=4, 8, 12, 16, and 32 wavelengths, respectively. The dashed lines depict the reference angular step λL=14,18,112,116,132 radians for the used source separations.

To proceed to more general sources two observations are useful:

1)	A rotation of a source will not change the required θ resolution, but the ϕ resolution must be set equal to the θ resolution.

2)	If the source distribution is stretched along the z-direction, the ϕ-resolution will not change.

Based on these two observations and the angular resolution of the line source of length L, the following can be deduced.

1)	If the line source is tilted 90 degrees down to the xy-plane, and then arbitrarily rotated around the z-axis, a flat disc of diameter L is generated. Based on observation 1, the angular resolution is Δθref=Δϕref=λ/L.

2)	If the disc is stretched a distance h along the z-axis (current elements are translated parallel to the z-axis), then the Δϕref=λ/L is unchanged, whereas the vertical angular resolution increases to Δθref=λ/L2+h2 to encompass the largest possible source separation within the cylinder.

The final shape of the source enclosure is hence a cylinder of diameter L and height h, and the angular steps required to get an accurate EIRP average (TRP value) are

Δθref=λL2+h2=λD , Δϕref=λL=λDcyl.

Here, D is the diameter of the source enclosure, i.e., the diameter of the smallest sphere enclosing all sources, and Dcyl is the diameter of the smallest z-directed circular cylinder that encloses all sources.

Other methods for determining the reference angular steps are not precluded.

Note: When sampling with the reference angular step, fine details of the radiation pattern are maybe not captured but the estimated TRP value is still accurate.

Spherical diameter is defined as the diameter of the smallest sphere enclosing the radiation source.

Cylindrical diameter is defined as the diameter of the smallest cylinder that encloses the radiation source along z-axis.

The spherical and cylindrical diameters are calculated as:

D=d2+w2+h2

Dcyl=d2+w2

The radiation source can be the antenna array or even the whole EUT, depending on the emissions we consider. This is further explained in subclause 10.8.3.

Some basic definitions and relations are given here for readability.

Figure 10.8.2.2-2: The dimensions of a radiation source are depth (d), width (w) and height (h)

Optionally, for the specific case of a Uniform Linear Array (ULA) system, the array spatial pattern could be defined as in the following equation.

Where spatial frequency  is defined as following:

Similar to Nyquist sampling in the time domain signal, the Rayleigh resolution for spatial domain signal to avoid the aliasing can be derived as:

where d is the separation distance between antenna elements and m is the number of antenna elements. If EUT is mounted along the yz plane as shown in Figure 10.8.2.2-3, based on the above considerations on the Rayleigh resolution for spatial domain signal, then subinterval in the φ and θ in degrees angle is calculated as:

Δθref=min⁡(180∘πarcsinλDZ, 15∘ )

Δϕref=min180∘ πarcsinλDY , 15∘

Where Dy is length of radiating part of the EUT along y-axis, Dz is length of radiated part of the EUT along the z-axis and  is wavelength for the measured frequency. Arcsine is in radians.

Figure 10.8.2.2-3: Spherical coordinate for OTA conformance testing of EUT

In the NR coexistence study, it was assumed that antenna configuration for wide area NR BS is 8x16 supporting two orthorgonal polarizations. If EUT mounted along y/z plane with antenna configuration 16x8 where 16 columns are assumed along the y-axis and 8 rows are assumed along the z-axis. Antenna elements are uniformly distributed with separation distance λ/2, therefore aperture size Dy≈8λ and Dz≈4λ. The uniform sampling in the spherical coordinate for this approach is demonstrated in the Figure 10.8.2.2-4.

Figure 10.8.2.2-4: Uniform sampling in the spherical coordinate, red crosses denotes the sampling points.

For a wanted signal, the reference angular steps are approximately equal to the beamwidth (in degrees) of the main beam.

∆ϕref≅BeWϕ≅FNBWϕ2Δθref≅BeWθ≅FNBWθ2

where BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively; and FNBWϕ and FNBWθare the first null beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

Using beamwidth of the wanted signal, the reference angular steps for each frequency within the downlink operating band including ΔfOBUE can be expressed as follows:

Δϕref=λλoBeWϕ=λλo FNBWϕ2Δθref=λλoBeWθ=λλo FNBWθ2where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth in the φ-axis and θ-axis, respectively.

For the OTA BS radiated transmit power requirement, beamwidths at five different directions is declared by manufacturers. For more details, see subclause 7.2 in 3GPP TR 37.842. The declared beamwidth may be used to set BeWφ and BeWθ in the above equations provided the same beam is applied to test in-band TRP requirements. If the beamwidth of a test beam is not declared, then the beamwidth can be obtained through measurements following the same procedure as the BS radiated transmit power requirement prior to TRP measurements.

In addition, the beamwidth of the wanted signal can be used to determine the physical dimensions of a radiation source as follows:

Dcyl=λoBeWϕand for the ULA case:

#### 10.8.2.3	Spherical equal area grids

With the spherical equal area sampling grid, the spherical surface is partitioned into N equal area regions. Let n be the index for the nth region and there is one point (θn,ϕn) located in the centre of each region. The definite integral for TRPReference can be approximated as:

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

The total number angular sampling points is N. Unlike the spherical equal angle grid, the TRPEstimate equation is not weighted by sin θ. As shown in Figure 10.8.2.3-1, the equal area grid points are distributed uniformly on the sphere but the pattern of θ and ϕ angles is irregular.

One possible way to estimate N is as follows:

N=4π∆θref∆ϕref

where Δθref and Δϕref are defined in 10.8.2.1. Other methods are possible and not precluded.

Figure 10.8.2.3-1: Spherical equal area sampling grid

#### 10.8.2.4	Spherical Fibonacci grids

The Fibonacci grid points are arranged along a generative spiral on the spherical surface. Similar to the equal area sampling grid, the Fibonacci grid generates points that are uniformly spaced in an isotropic way. Assume there are I points in the Fibonacci sampling grid, then the definite integral for TRPReference can be approximated as:

TRPEstimate=1Ii=0I-1EIRP(θi,ϕi)TRPEstimate=1Ii=1IEIRP(θi,ϕi)

where i = 0 .. I-1

θi=cos-11-2i+1Iθi=cos-11+2i+1I and

ϕi=2πiΨ,

where	Ψ=1+52

The total number of angular sampling points is I, which can be estimated in a similar manner as in subclause 10.8.2.3. Like the spherical equal area grid, the TRPEstimate equation is not weighted by sin θ.

Figure 10.8.2.4-1: Spherical Fibonacci sampling grid

#### 10.8.2.5	Orthogonal cuts grids

Compared to the TX spurious emissions the OBUE emissions and ACLR are likely to experience the similar beamforming pattern as for the wanted signal. Due to this reason, it is easily predictable where the maximum of the emissions is going to be, hence the possibility to enhance the measurement method to achieve better accuracy. Here we choose to apply the pattern multiplication method [31].

In this method, at least two cuts (default) shall be used, an optional third cut can be added if needed. The alignment of the cuts must be along the symmetry planes of the antenna array. Note that theta reference steps apply to the vertical cuts and phi reference steps to the horizontal cuts.

The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

The second mandatory is a vertical cut passing through the peak direction of the main beam.

Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

The third optional cut is a vertical cut orthogonal to the first and the second cut.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as:

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points. The final contribution for all cuts is calculated as:

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the intersection points are measured multiple times and the repeated values can be removed from the samples before averaging.

Figure 10.8.2.5-1: Example of orthogonal cuts geometry when the main lobe points along the x-axis. Two mandatory cuts grid (left) and the optional added third cut (right). The first two cuts are generated by rotating the EUT around its z-axis and y-axis, respectively, and the optional third cut is generated by rotating the EUT around its x-axis.

Two cuts cut data gives a conservative TRP estimate (an overestimation of the real TRP). Through pattern multiplication a less conservative estimate is obtained, based on the calculation of the antenna array factor as a product of two terms, corresponding to the two cuts.

The following conditions for being able to apply pattern multiplication method are mandatory:

i.	The vertical cut (and the main beam) is in the xz-plane

ii.	The frequency of the emission is within the downlink operating band.

iii.	The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

iv.	The emission appears/disappears when the Tx power is turned on/off.

v.	The antenna arrays of the EUT

1.	Have rectangular grids of antenna element positions

2.	Have symmetry planes that are vertical and horizontal.

3.	Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRPcut1ϕ at θ=θH and a vertical cut with data EIRPcut2θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemisphere. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in Figure 10.8.2.2-1 the u and v coordinates are defined as:

u=sinθsinϕv=cosθ

Note that only the data on the coordinate axes are measured, and hence only the data EIRPcut1u for vH=cosθH (the horizontal cut) and EIRPcut2v for u=0 (the vertical cut) are known. Moreover, only the points in the circular disc u2+v2≤1 , a.k.a. the visible region, contribute to the TRP.

The pattern multiplication is used to calculate power density values outside the two cardinal cuts as:

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

In Figure 10.8.2.5-2, the case where vH=0 is illustrated.

Figure 10.8.2.5-2: Example of pattern multiplication

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

Note: the numerical singularity at u2+v2=1 must be treated with care, e.g. by changing the coordinate system to polar as in [31].

#### 10.8.2.6	Wave vector space sampling grid

Similar as Rayleigh sampling approach, EUT is placed on the yz plane in the spherical coordinate and normal vector of EUT is pointing along the x-axis as shown in Figure 10.8.2.2-1. The angle φ and θ represent azimuth and elevation respectively, u and v represent the projection of normalized wave vector on y-axis and z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be represented as following:

u=sinθsinϕ, v=cos⁡(θ)

In the existing TR 37.843, TRP is defined in the spherical coordinate as following:

As TRP is defined in the wave vector coordinate, therefore TRP definition should be revised accordingly in the corresponding coordinate. For the TRP definition in normalized wave vector space, according to the 2D Jacobian transformation, the above equation could be adjusted as following, namely:

dudv=∂u,v∂θ,ϕdθdϕ=sin2θcosϕdθdϕ

Based on the above two equations, then we could get

TRP=14πfwdEIRPθ,ϕsinθcosϕdudv+14πbwdEIRPθ,ϕsinθcosϕdudv

where relationship between (θ,φ) and (u,v )is demonstrated in the equation before. Similar as discrete sampling process in TR37.843, the above equation is approximated in the far-field region as the sum of the total EIRP at a number of discrete directions as follows:

TRP=ΔugridΔvgrid4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

The above considerations could be applied for both polarization.

Uniform sampling in the wave vector coordinate as shown in Figure 10.8.2.6-1:

-	Rayleigh resolution in y-axis:   Δugrid=λDy

-	Rayleigh resolution in z-axis:    Δvgrid=λDz

Where Dy is length of radiating parts of EUT along y-axis, Dz is length of radiating parts of EUT along the z-axis.

Based on the uniform sampling grid on the yz plane, we could get the sampling point (um , vn). In addition, according to the transformation between (um,vn ) and (φ,θ ), then azimuth and elevation (φm,n, θn) in the spherical coordinate could be derived correspondingly as shown in Figure 10.8.2.6-2. Based on the (φm,n, θn) in the spherical coordinate, EIRP on the spherical coordinate could be measured.

Figure 10.8.2.6-1: Sampling grid in the wave vector space

Figure 10.8.2.6-2: Sampling grid in the spherical coordinate

### 10.8.3	TRP measurement procedures

Different procedures can be used to evaluate the TRP estimate. These procedures can provide either an accurate assessment or a controlled overestimate of the TRP. The choice of methods is based also on the available test setup, measurement equipment, and the measurement time. This section describes the methods which are suitable for each type of requirements. Other relevant methods are not precluded. For each method, the test purpose (accurate or overestimate) is pointed out. A summary of the requirement types and measurement procedures is shown in Table 10.8.3-1.

In the following clauses the measurement procedures for different parameters are described under the assumption of equal angle sampling. Similar procedures can be also used with other types of spherical grids, given that the proper reference steps Δθref and Δϕref are determined.

Table 10.8.3-1 - Applicability of TRP measurement methods to the type of emissions to be measured.

|  | Base station output power 5.2.2 | Unwanted emissions: Adjacent channel leakage ratio 5.6.3.3(ACLR) (Note 1) | Unwanted emissions: Spectrum emission mask 5.6.4.3 and Operating band unwanted emission OTA 5.6.5.3 (SEM & OBUE) | Transmitter spurious emissions OTA 5.6.6.1.3 (Note 2) |
| --- | --- | --- | --- | --- |
| Full sphere using reference steps (accurate) | X | X | X | X |
| Full sphere using sparse sampling (overestimate) [y] |  |  |  | X (Note 3) |
| Two cuts + Pattern multiplication (accurate) (Note 4) | X | X | X |  |
| Two/three cuts (overestimate) |  | X | X | X (Note 3) |
| Beam-based directions | X | X (Note 5) | X (Note 5) |  |
| Peak method |  |  | X | X |
| Equal sector with peak average |  |  | X | X |
| Pre-scan |  | X | X | X |
| Note 1: Two TRP measurements are needed. Note 2: Pre-scan is needed to identify the frequencies of interest. Pre-scan can also be applied to ACLR, OBUE and SEM. Note 3: At harmonic frequencies the use of this method is FFS due to risk of high beamforming gain Note 4: Pattern multiplication is conditional Note 5: Applicable if the directivity of corresponding requirement at the reference direction is equivalent to the directivity at the reference direction when EUT emits Prated,c,TRP and Prated,c,EIRP. Note 6: If box is blank the method is not excluded but the methodology has not been described in subclause 10.8, 3; if a suitable analysis is shown the method may be applied. |  |  |  |  |

#### 10.8.3.1	Procedures for base station output power

##### 10.8.3.1.1	General

Test purpose: Accurate TRP assessment.

The radiation source is assumed to be limited to the antennas on the EUT and it’s not necessary to take the dimensions of the whole EUT into account for calculations of the far-field distance and the reference angular resolution. The test choices are as follows.

##### 10.8.3.1.2	Two cuts with pattern multiplication

Use this method when the antenna symmetries are compatible with pattern multiplication, see subclause 10.8.2.5. Following steps are performed during the measurement.

1.	Calculate the reference angular steps as described in subclause 10.8.2.1.

2.	Align the EUT to allow for proper pattern multiplication. See Sec. 10.8.5. Measure EIRP on two orthogonal cuts with steps smaller or equal to the reference steps according to step 1.

3.	Apply pattern multiplication according to subclause 10.8.5 to extrapolate the two cuts data to full-sphere.

4.	Apply numerical integration to obtain the TRP estimate as described in subclause 10.8.5.

##### 10.8.3.1.3	Full sphere

Following steps are performed during the measurement:

1.	Calculate the reference angular steps as described in subclause 10.8.2.1. Other methods for determining the required angular steps are not precluded.

2.	Choose the angular steps smaller than or equal to the reference angular steps.

3.	Measure EIRP values on a spherical grid according to subclause 10.8.2.1. Having the poles of the measurement grid along the direction of the main beam shall be avoided.

Note:	Spherical grids of subclause 10.8.2.2, 10.8.2.3, 10.8.2.4 and 10.8.2.6 can also be used with proper angular steps.

4.	Apply suitable numerical integration to calculate the TRP estimate.

##### 10.8.3.1.4	Beam-based directions

This method is only applicable if directivity of EUT antenna is known for the operating frequency. For the Rel-13 AAS BS EIRP accuracy requirement, the peak EIRP of a beam is measured at the beam peak direction within the beam direction pair. Following the same approach, the peak EIRP of a beam can be obtained, which is used to derive TRP estimate using directivity of EUT antenna as follows:

TRPEstimate=EIRPpeakDEUT

, where DEUT is the directivity of EUT.

For the case of OTA BS output power, the directivity (DEUT,BSoutputpower ) shall be defined as:

DEUT,BSOutputpower= Prated,c,EIRPPrated,c,TRP

#### 10.8.3.2	Procedures for SEM and OBUE

##### 10.8.3.2.1	General

Test purpose: Accurate or controlled overestimate of TRP.

The radiation source is limited to the antennas on the EUT and it’s not necessary to take the dimension of the whole EUT into account for calculations of the far-field distance and the reference angular steps. The test choices are as follows.

##### 10.8.3.2.2	Two cuts with pattern multiplication

Use the same procedure as in subclause 10.8.3.1.2.

##### 10.8.3.2.3	Two or Three cuts

Use this method when the cuts described in subclause 10.8.2.5 can be identified, but the pattern multiplication is not applicable according to the requirements in subclause 10.8.2.5. This method will provide an overestimated value for TRP. Following steps are performed during the measurement.

1.	Calculate the reference angular steps.

2.	Measure EIRP on two orthogonal cuts with angular steps smaller than or equal to the reference steps according to step 3. Align the EUT such that the cardinal cuts are measured. See Figure 10.8.2.5-1.

3.	Calculate the average EIRP in each cut and then the TRP estimate according to subclause 10.8.2.5.

4.	If the TRP estimate is above the requirement limit, perform the measurement on a third cut (See Figure 10.8.2.5-1) and repeat step 3.

##### 10.8.3.2.4	Full sphere

Use the same procedure as in subclause 10.8.3.1.3.

##### 10.8.3.2.5	Beam-based directions

This method only applicable if directivity of EUT antenna is available for the downlink operating band plus ΔfOBUE on either side of the band edge. Refer to subclause 10.8.3.1.4 for more details.

The pre-scan, peak and equal sector with peak average methods are possible options. For more details, refer to subclause 10.8.3.4.1, 10.8.3.4.2 and 10.8.3.4.3.

#### 10.8.3.3	Procedures for ACLR

##### 10.8.3.3.1	General

Test purpose: Accurate or controlled underestimate of ACLR.

The methods described in this section are used for assessment of the TRP values for ACLR. Depending on the method, the result can be accurate or a controlled underestimate of the ACLR. The latter applies if the TRP of the radiation in the adjacent band is overestimated. The radiation source is limited to the antennas on the EUT and it’s not necessary to take the dimension of the whole EUT into account for calculations of the far-field distance and the reference angular steps.

##### 10.8.3.3.2	TRP fraction method

Following steps are performed during the measurement.

1.	For the TRP of the desired signal TRPd use a suitable method from subclause 10.8.3.1.

2.	For the TRP of the emission TRPe use a method from subclause 10.8.3.2.

3.	Evaluate the ratio  ACLR=TRPdTRPe.

#### 10.8.3.4	Procedures for transmitter spurious emissions and EMC emissions

##### 10.8.3.4.0	General

Test purpose: Accurate or controlled overestimate of TRP.

The methods described in this section are used for assessment of the TRP for spurious emissions. The methods are given in a sequence with increasing accuracy, but also increasing measurement time. There is no mandatory order, in which to use these methods nor all of the methods should be used for evaluating TRP. Some of these methods can be skipped if they are deemed not necessary, for instance, the pre-scan and peak methods.

The radiation source is not limited to the antennas on the EUT instead the entire physical dimensions of the EUT must be taken into account for calculations of the reference angular steps, see subclause 10.8.2.1. Note that full-sphere sampling using reference angular steps is the only method that aims to provide an accurate TRP.

When available, using a beam sweeping test signal with these methods can reduce the test time and improve the uncertainty.

##### 10.8.3.4.1	Pre-scan

Pre-scan is a fast measurement method, which is performed over the entire spurious frequency range to identify spurious frequencies with emission power levels above a threshold. Note, it is not necessary to do pre-scan before executing the test methods in this section, but it is recommended. It is also not meant to provide an estimate of TRP. Hence, other relevant test methods should be used for TRP estimation.

1.	The distance can be shorter than the intended measurement distance between EUT and the test antenna for evaluating TRP but should remain fixed throughout the scanning process.

2.	Scan the surface around the EUT.

3.	Rotate the measurement antenna to cover all polarizations of emissions to detect the maximum emission.

4.	Record the list of spurious emission frequencies and corresponding power levels, spatial positions of EUT and test antenna polarization where the maximum power levels occurred.

5.	For spurious frequencies with emission power levels more than 20 dB below the specified limit in then these spurious frequencies are considered compliant and no further measurements are required.

6.	For all the other spurious frequencies that do not meet the criterion in Step 5, further measurements are required.

##### 10.8.3.4.2	Peak method

This method is applicable when the pre-scan method indicates the presence of emission peaks. The peak method can be skipped if there exist no emission peaks. Further, the method does not provide a TRP estimate instead the highest absolute EIRP is measured at each spurious frequency identified in the pre-scan. If the absolute EIRP meets the specified TRP limit, then it implies that the TRP estimate would meet the limit too. As a result, it is not needed to perform further measurement using the other methods.   Following steps are

1.	Find the direction of the peak emission EIRP or peak power density

2.	Start with the spurious frequency that has the maximum power level recorded during pre-scan.

3.	The EUT and test antenna are oriented to the same position where the maximum power level is recorded during the pre-scan

Note: This set-up might not be applicable if pre-scan is performed in near-field.

4.	EUT and test antenna are moved around the position to identify and measure the peak EIRP.

5.	If the peak EIRP for a spurious frequency is less than the specified limit then no further measurements are required else use the methods below to evaluate TRP estimates.

6.	Repeat Steps 4 to 6 for the next strongest emission.

##### 10.8.3.4.3	Equal Sector with peak average method

The Equal sector with peak average method can be considered as an extension to the peak method. It is performed on the list of spurious frequencies which have not met the limit using the peak method. The method takes into account several peak EIRPs of beams belonging to different sectors of the sphere. TRP estimate is calculated as the average of the peak EIRP in different sectors.

1.	The measurement distance is in the far field.

2.	The sphere is divided into K equal sectors. The total number of sectors depend on the dimensions of EUT. If the largest dimension is less than 60 cm, then each sector is a half quadrant of 45 degrees. Other techniques for determining the sector size are not precluded such as using the angular step.

3.	For those spurious frequencies which need further measurements by the peak method, start with the spurious frequency that has the highest recorded power level.

4.	Perform Steps 4 and 5 as in the peak method.

5.	Move to the next sector with next higher emissions recorded and repeat Step 4 until all sectors are covered.

6.	Calculate TRP estimate as , where  is in linear units.

7.	Repeat steps 4 and 5 for at least 7 spurious frequencies with the next higher emission in descending order.

8.	If TRP estimate for each of the 8 spurious frequencies is less than the specified limit then no further measurements are required else use the other methods to evaluate TRP estimates.

##### 10.8.3.4.4	Two or three cuts with dense sampling

Following sequence can be used

1.	Follow steps described in subclause 10.8.3.2.3 and calculate the TRP estimate. Note that no alignment is needed for spurious emissions.

2.	Add the appropriate correction factor ΔTRP according to Table 10.8.3.4.4-1 to ensure overestimation with 95% confidence.

3.	Compare the (TRP estimate + ΔTRP) to the limit.

4.	If the (TRP estimate + ΔTRP) is above the limit, perform the measurement on an additional third cut (See Figure 10.8.2.5-1) and repeat steps 1 to 3.

Table 10.8.3.4.4-1: The correction factor for two or three cuts dense sampling.

|  | Three cuts | Two cuts |
| --- | --- | --- |
| Correction factor ΔTRP (dB) | 2.0 | 2.5 |

##### 10.8.3.4.5	Full sphere with sparse sampling

Sparse angular sampling with a correction factor can be used to save measurement time. The only difference is in the used angular steps. Following sequence can be used:

1.	Set the angular grid:

a.	Non-harmonic frequencies: choose the angular steps Δϕ and Δθ smaller than or equal to 15 degrees. Calculate the sparsity factor (SF) as:

SF=maxΔϕΔϕref,ΔθΔθref

and the correction factor as:

SF-1SFmax-1 dB,

b.	where SFmax corresponds to 15 degrees angular step. If the sparsity factor is smaller than 1, the correction factor ΔTRP is 0 dB. Harmonic frequencies with fixed beam test signal: choose the angular steps smaller than or equal to the reference angular steps Δϕref and Δθref. Correction factor ΔTRP is 0 dB.

c.	Harmonic frequencies with beam sweeping test signal: set the angular steps to [15] degrees. Correction factor is ΔTRP 0 dB.

2.	Measure EIRP (or power density multiplied by grid surface 4πd2)  on a spherical grid according to subclause 10.8.2. Having the poles of the measurement grid along the direction of the main beam shall be avoided.

Note:	Other spherical grids can also be used with proper angular sampling.

3.	Apply a suitable numerical integration to calculate the TRP estimate.

4.	Add the appropriate correction factor ΔTRP according to step 1 to ensure an overestimation with 95% confidence.

5.	Compare the (TRP estimate + ΔTRP) with the limit. If the (TRP estimate + ΔTRP) is above the limit, choose a smaller angular step and repeat steps 2-4. If the sparsity factor is less than one, no significant improvement of accuracy is expected.

##### 10.8.3.4.6	Full sphere

Use the same procedure as in subclause 10.8.3.1.3.

## 10.11	Measurement uncertainty for demodulation performance requirements

#### 10.11.1	General

This subclause describes the OTA aspects of the test procedure for assessing demodulation requirements, considering UTRA FDD and E-UTRA demodulation requirements which were found feasible OTA, as captured in subclause 7.8. The OTA part of the test procedure is the same for each requirement.

The demodulation requirements are in essence very similar to the RX dynamic range requirement; in both cases a wanted signal and AWGN is transmitted from the test equipment and should be reassured to be received at the absolute level well above the noise floor. The test equipment setup for demodulation requirements may include multiple wanted and interference signals and a fading channel emulators, as described in subclause 7.7.

#### 10.11.2	In-door anechoic chamber

##### 10.11.2.1	General

The indoor anechoic chamber setup for demodulation requirements is the same as that for RX dynamic range, as described in subclause 10.3.4.2.1.

##### 10.11.2.2	Calibration

The calibration procedure for demodulation requirements is the same as that for RX dynamic range, as described in subclause 10.3.4.2.2.

##### 10.11.2.3	Procedure

The OTA part of the procedure for demodulation requirements is based on that for RX dynamic range, as described in subclause 10.3.4.2.3. The test equipment setup is described in subclause 7.7.

The transmit signal power level may be set in the same manner as for RX dynamic range; i.e. by considering the AWGN absolute levels quoted in TS 36.141 [] together with an adjustment ΔOTAREFSENS to take into account BS antenna gain (path loss is taken account of by the calibration procedure). An example AWGN power levels for the E-UTRA PUSCH requirements are listed in table 10.11.2.3-1 (AWGN levels for other requirements can be found in the conducted single RAT specifications).

Table 10.11.2.3-1: AWGN power level at the BS input for E-UTRA demodulation requirements

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -92.7 - ΔOTAREFSENS dBm / 1.08MHz |
| 3 | -88.7 - ΔOTAREFSENS dBm / 2.7MHz |
| 5 | -86.5 - ΔOTAREFSENS dBm / 4.5MHz |
| 10 | -83.5 - ΔOTAREFSENS dBm / 9MHz |
| 15 | -81.7 - ΔOTAREFSENS dBm / 13.5MHz |
| 20 | -80.4 - ΔOTAREFSENS dBm / 18MHz |

##### 10.11.2.4	MU assessment

The BS demodulation requirements consist of a wanted signal and AWGN (potentially with fading modelling and additional interfering signals). In the conducted test setup, the received AWGN level is well above the noise floor, and so the SNR experienced at the BS receiver is dependent on the transmitter SNR/SINR (in case of E-UTRA) or Eb/N0 (in case of UTRA).

Small variations in the received power level due to additional uncertainties in the OTA measurement chamber will not impact the receiver SNR, and hence uncertainties due to OTA will not impact the demodulation requirement MU. The MU can be considered to depend on the TX SNR/SINR (in case of E-UTRA) or on the TX b/N0 (in case of UTRA) and, where applicable the fading channel generation uncertainty, in the same manner as for the conducted requirements.

Thus, the MU for OTA demodulation requirements is the same as the MU for conducted demodulation requirements, as derived in TS 25.141 [] for UTRA FDD, and in TS 36.141 [] for E-UTRA. The resulting test requirements for OTA AAS BS can be reused from conducted test specifications TS 25.141 [] and TS 36.141 [], as long as the SNR/SINR (in case of E-UTRA) or the TX b/N0 (in case of UTRA) at the BS receiver is not impacted by the noise floor due to the OTA test chamber path loss.

#### 10.11.3	CATR

##### 10.11.3.1	General

The CATR setup for demodulation requirements is the same as that for RX dynamic range, as described in subclause 10.3.4.3.1.

##### 10.11.3.2	Calibration

The calibration procedure for demodulation requirements is the same as that for RX dynamic range, as described in subclause 10.3.4.3.2.

##### 10.11.3.3	Procedure

The OTA part of the procedure for demodulation requirements is based on that for RX dynamic range, as described in subclause 10.3.4.3.3. The test equipment setup is described in subclause 7.7.

The transmit signal power level may be set in the same manner as described in section 10.11.2.3).

##### 10.11.3.4	MU assessment

The MU for OTA demodulation requirements is the same as for conducted demodulation requirements for the same reasons as applicable for the indoor anechoic chamber, described in subclause 10.11.2.4.

Annex A:	
Void

Annex B:
OTA sensitivity measurement error contribution descriptions

# B.1	Indoor Anechoic Chamber

The indoor anechoic chamber OTA sensitivity measurement error contribution descriptions can be found in 3GPP TR 37.842 [4] annex C.

# B.2	Compact Antenna Test Range

The compact antenna test range  OTA sensitivity measurement error contribution descriptions can be found in 3GPP TR 37.842 [4] annex C.

# B.3	One Dimensional Compact Range

The One Dimensional Compact Range OTA sensitivity measurement error contribution descriptions can be found in 3GPP TR 37.842 [4] annex C.

# B.4	Near Field Test Range

The Near Field Test Range OTA sensitivity measurement error contribution descriptions can be found in 3GPP TR 37.842 [4] annex C.

Annex C:
Beam sweeping

For emissions with a high beam-forming gain, correct assessment of TRP requires grid measurements at small angular steps. In a measurement where the EUT is mounted on a turntable this turns into a large number of mechanical rotations of the EUT and hence lengthy measurements. One way to mitigate this problem is to use beam sweeping as a complement to mechanical rotations. Instead of performing the TRP measurements using a fixed beam, the BS will be programmed to move the beam across several pre-defined positions, while the power samples are recorded for each point on the grid.

Such method could be advantageous not only for in-band signals, but also for spurious emissions characterized by high beamforming gain, such as harmonics.

The results below show that the calculated TRP from the beam average pattern is very close to the TRP of all individual beams that are included in the sweeping. The advantage of this method would be the possibility to use sparser spherical grid compared to reference angular steps, and without the need to find the peaks of the emissions.

# C.1	Simulation results

The effect of beam sweeping is illustrated by using a 10x10 Uniform Rectangular Array with 0.55λ element separation at the carrier frequency. A number of beams is realized by a constant phase-gradient steering. The beam sweeping range is selected to completely avoid the forming of grating lobes and the angular step between two lobes is selected as the Half Power Beamwidth (HPBW) of a broadside beam.

In the example of figure C.1-1 the reference angular step is 11.6 degrees (D=5λ=>Δϕref=λ/D=1/5 radians). For the harmonic in figure C.1-2 the reference step is 5.8 degrees. The full sphere average of the EIRP is the TRP. For simplicity, only sweeping in the horizontal plane is employed.. In the case where a fixed beam is used the average EIRP is not correct when the sampling angular step is larger than the reference angular step Δϕref. However, when the EIRP pattern is first averaged over all beams (blue curves) the average EIRP is virtually constant for any angular step up to 36 degrees, see figure C.1-3.

Figure C.1-1: In the left figure several beam patterns of the array are depicted, together with the corresponding element pattern. In the right picture two different settings of a spectrum analyser are simulated. Under the “Max hold” setting of the analyser the maximum EIRP over all beams is presented at every angular point. Under the “Average” setting the average EIRP over all beams is presented. The latter case is the intended setting to use for TRP assessment.

Figure C.1-2: The beam sweeping at the 2nd harmonic frequency is simulated using an embedded element pattern. Left plot depicts the EIRP patterns of individual beams. Right plot depicts the max hold and beam average EIRP patterns.

Fundamental frequency						2nd harmonic

Figure C.1-3: Calculation of average EIRP in the azimuth cut by using different angular step sizes. The yellow curve corresponds to a fixed broadside beam (perpendicular to the antenna array surface), and all the green curves correspond to steered, but fixed beams. The variation in TRP for different beams at the harmonic is due to grating lobe effects.

# C.2	Measurement results

The effect of beam sweeping measured on an AAS BS employing a 4x16 array, is depicted in figures C.2-1 and C.2-2. Measurements were performed in a CATR, in far-field conditions, with an AAS BS that transmits one single beam that can be moved across a number of pre-defined directions. For the results using beam sweeping, see right figure in Figure C.2-1, the following beam settings were used:

-	48 pre-defined beam directions (see fig. C.2-0) were swept, changing the beam direction every 15 usec

-	the range of beam directions is +/- 60 and +/- 15 degrees in azimuth and elevation, respectively.

-	The AAS BS is making a full rotation (360 degrees) mechanically into the horizontal plane, with a step of 1 degree. Each position was held for about 1 sec, while the spectrum analyser recorded a time averaged value of the received power (at least 1000 samples for each angular position were considered enough for good statistical average).

Figure C.2-0: Pre-defined possible directions of the main beam (red spots). Compared to the broadside beam (the middle one) the maximum supported steering is +/- 60 degrees in azimuth and +/- 15 degrees in elevation. The extreme directions are excluded in this experiment, so only 48 directions are swept.

.

Figure C.2-1: EIRP values for measured data in a horizontal cut versus frequency (in relative scale).

Figure C.2-2: To the left measured data for a horizontal cut are shown at a single frequency. A significantly broader pattern is detected for the beam sweeping signal (spectrum analyser in “Average” mode). To the right the angular average of the EIRP values are depicted for different angular step sizes.

# C.3	Design of beam sweeping test signal

The following design criteria have been identified:

1.	Full transmit power is used for all beams.

2.	Horizontal beam separation shall be the HPBW in the azimuth direction and the vertical beam separation shall be the HPBW in the vertical direction, evaluated for a broadside beam.

3.	Lobes with high loss compared to the broadside beam are avoided. By loss we mean the reduction in beamforming gain between different positions of the beam)

The motivation for the third criterion is to have a fairly constant TRP for the used beams. The loss can be due to both loss in element pattern gain, but also due to adverse mutual coupling effects. Removing lobes with high loss gives a beam average TRP that is close to the TRP for any individual beam.

Annex D:
Sparse sampling for spurious emissions

In the case of spurious emissions for large Equipments Under Test (EUTs), such as the AAS BS, the emissions do not necessarily radiate through the antenna elements. For this reason, to be on the conservative side, the entire EUT dimensions should be considered when calculating the reference angular steps on the spherical grid.

The emission lobes will be narrow and hard to locate for some emissions, especially in the higher part of the spurious emissions domain. Instead of searching for the direction of the emission lobes, it is proposed to perform the measurement on a grid without alignment considerations and add a correction factor to the TRP obtained from the grid, to ensure overestimation. This correction factor is obtained through the study of random rotations of the EUT.

In practice, the emissions are likely to have a low degree of correlation, except for harmonic emissions. For harmonic emissions a beam sweeping test signal is proposed in Annex Cand the description of such signal is FFS. For non-harmonic emissions, a correlation factor distributed uniformly in a given interval from 0 to  is assumed when calculating the correction factor ΔTRP for the TRPestimate. The angular variation of the emissions coming from low correlated sources is dominated by the array factor, i.e., the spatial distribution of the sources. Hence, emissions can be modelled by the array factor of a set of point sources, i.e., for simplicity the element pattern is isotropic.To quantify the degree of sparse sampling relative to the reference angular steps a Sparsity Factor (SF) is introduced. For a spherical equal angles grid

Where

The TRPestimate values are then calculated by using a given angular grid defined by Δϕand Δθ which are larger than the reference angular steps Δϕref and Δθref. The CDF of these TRP values are then created and the CDF is shifted so that the 5th percentile corresponds to 0 dB TRP error. This shift is introduced as ΔTRP, see figure D-1.

In the followings we propose a method to estimate ΔTRP for large antennas (D>4λ), based on generating a set of statistical samples of TRP values calculated on a set of randomly generated emission sources of a given size D.

In this investigation each statistical sample is generated as follows:

1.	Set up a quadratic uniform linear NxN array in the yz-plane, with horizontal and vertical element separation

N is randomly chosen such that the element separation is larger or equal to a half wave length.

2.	Rotate the source points by an Euler zyz-rotation with angles a, b, and c which are uniformly chosen in the intervals [0,360], [0,90] and [0,360] degrees, respectively. The rotated position of source n is denoted

3.	Pick a random correlation value ρ from the uniform distribution between 0 and , and generate source weights as

Where  are picked from a normal distribution with zero mean and unit standard deviation.

4.	Normalize the weights to TRP=1 by using a full sphere grid with sparsity factor 0.25.

5.	Generate EIRP values on the desired grid by using an array factor

6.	Calculate the TRP value as an appropriate mean value of the EIRP values for two/three cut grids and using full sphere numerical integration for the full sphere case.

A publicly available Matlab code [1] is available. https://se.mathworks.com/matlabcentral/fileexchange/67143-sparse-sampling-analysis-tool?s_tid=srchtitle and more results can be found in Ref [2].

The resulting 5th percentile values of the empirical CDFs are used as a practical lower bound on the TRP calculated directly from the points in the actual grid. If the 5th percentile value is positive, there is no need for a systematic correction, that means ΔTRP=0. On the other hand, if the 5th percentile value is negative, a systematic correction is needed to guarantee TRP overestimation with 95% confidence. For this purpose, the absolute value of the 5th percentile is used as systematic correction factor, denoted ΔTRP (see figure D-1). Some simulation results are found in figures D-2 (a) – (c) for two different choices of max correlation and the three grid types: full-sphere, three-cut, and two-cuts.

Figure D-1: Determination of ΔTRP based on the 5th percentile values. The dashed curve depicts the empirical CDF found from the statistical analysis, and the solid curve depicts the corrected CDF corresponding to over-estimation with 95% confidence.

Figure D-2: Correction factor  for 95% confidence overestimation of the TRP for three different electrical sizes  and correlation intervals.

Figure D-3: Full sphere correction factors  for max correlation up to 0.75 and D/λ=10.

Figure D-4: Three cuts correction factors  for max correlation up to 0.75 and D/λ=10.

Figure D-5: Two cuts correction factors  for max correlation up to 0.75 and D/λ=10.

The following systematic correction factors are proposed be used in the test procedure, given the results obtained for ρmax =0.5 , which is considered to be acceptable at this point:

Table D-1: For a ful sphere sparse grid the proposed correction factor is zero for angular sampling below or equal to the reference steps (SF<1). The maximum Sparsity Factor (SF) is the SF at 15 degree angular sampling.

|  | Full-sphere sparse grid | Three cuts using reference angular steps | Two cuts using reference angular steps |
| --- | --- | --- | --- |
| Correction factor (dB) | (SF-1)/(SFmax-1) | 2.0 | 2.5 |

Annex E:
Radiated transmit measurement error contribution descriptions

# E.1	Indoor Anechoic Chamber

E1-1 Positioning misalignment between the AAS BS and the reference antenna

This contribution originates from the misalignment of the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the reference antenna. The uncertainty makes the space propagation loss between the AAS BS and the receiving antenna at the AAS BS measurement stage (i.e. Stage 2) different from the space propagation loss between the reference antenna and the receiving antenna at the calibration stage (i.e. Stage 1).

E1-2 Pointing misalignment between the AAS BS and the receiving antenna

This contribution originates from the misalignment of the testing direction and the beam peak direction of the receiving antenna due to imperfect rotation operation. The pointing misalignment may happen in both azimuth and vertical directions and the effect of the misalignment depends highly on the beamwidth of the beam under test. The same level of misalignment results in a larger measurement error for a narrower beam.

E1-3 Quality of quiet zone

This contribution originates from a reflectivity level of an anechoic chamber. The reflectivity level is determined from the average standard deviation of the electric field in the quiet zone. By repeating a free space VSWR measurement in 15° grid in elevation and azimuth, 264 standard deviation values in both polarizations are determined. From these values an average standard deviation of electric field in the quiet zone can be calculated from the equation:

Where:

is the number of angular intervals in elevation,

is the number of angular intervals in azimuth, and

is elevation of single measurement .

If an efficiency calibration with omni-directional calibration antenna is performed, the effect of reflectivity level decreases in Stage 1 (i.e. calibration measurement) and  may be divided by factor 2. This is due to correcting impact of data averaging in this type of calibration. Efficiency calibration done with sampling step ≤ 30°, can be considered to have at least four independent samples.  may be divided by factor 2 also in Stage 2 (i.e. AAS BS measurement) for the same reason.

It's likely that asymmetry of the field probe will have a very small impact on this measurement uncertainty contributor, however, an upper bound to probe symmetry should be considered.

E1-4 Polarization mismatch between the AAS BS/reference antenna and the receiving antenna

This contribution originates from the misaligned polarization between the AAS BS/reference antenna and the receiving antenna.

E1-5 Mutual coupling between the AAS BS/reference antenna and the receiving antenna

This contribution originates from mutual coupling between the AAS BS/reference antenna and the receiving antenna. Mutual coupling degrades not just the antenna efficiency, i. e. the EIRP value, but it can alter the antenna's radiation pattern as well. For indoor anechoic chamber, usually the spacing between the AAS BS/reference antenna and the receiving antennas is large enough so that the level of mutual coupling might be negligible.

E1-6 Phase curvature

This contribution originates from the finite far field measurement distance, which causes phase curvature across the antenna of AAS BS/reference antenna.

E1-7 Uncertainty of the measurement equipment

This contribution originates from limited absolute level accuracy and non-linearity of the measurement equipment. The measurement equipment such as a BS simulator , spectrum analyzer, or power meter measures the received signal level in EIRP tests either as an absolute level or as a relative level. The uncertainty value will be indicated in the manufacturer's data sheet in logs. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E1-8 Impedance mismatch in the receiving chain

This contribution originates from multiple reflections between the receiving antenna and the power measurement equipment. The multiple reflections can produce an overall reflection that depends not only upon the individual reflections of each part but their reflective interactions as well. The combination loss by the overall reflection can be higher or lower than individual loss by multiple reflections. The combination loss is called the mismatch error and leads to the measurement uncertainty.

E1-9 Random uncertainty

The random uncertainty characterizes the undefined and miscellaneous effects which cannot be forecasted. One can estimate this type of uncertainty with a repeatability test by making a series of repeated measurement with a reference AAS BS without changing anything in the measurement set-up.

E1-10 Impedance mismatch between the receiving antenna and the network analyzer

This contribution originates from multiple reflections between the receiving antenna and the network analyzer. After appropriate calibration, the network analyzer may not introduce impedance mismatch error, but the error still happens between the receiving antenna feed cable and the receiving antenna.

E1-11 Positioning and pointing misalignment between the reference antenna and the receiving antenna

This contribution originates from reference antenna alignment and pointing error. In this measurement if the maximum gain directions of the reference antenna and the receiving antenna are aligned to each other, this contribution can be considered negligible and therefore set to zero.

E1-12 Impedance mismatch between the reference antenna and the network analyzer

This contribution originates from multiple reflections between the reference antenna and the network analyzer. After appropriate calibration, the network analyzer may not introduce impedance mismatch error, but the error still happens between the reference antenna feed cable and the reference antenna.

E1-13 Uncertainty of the network analyzer

This contribution originates from all uncertainties involved in the S21 measurement (including drift and frequency flatness) with a network analyzer. The uncertainty value will be indicated in the manufacturer's data sheet in logs. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E1-14 Influence of the reference antenna feed cable

In the calibration Stage 1, the influence of the calibration antenna feed cable is assessed by measurements. A measurement for calibration may be repeated with a reasonably differing routing of the feed cable. Largest difference among the results is entered to the uncertainty budget with a rectangular distribution.

E1-15 Reference antenna feed cable loss measurement uncertainty

Before performing the calibration, the reference antenna feed cable loss have to be measured. The measurement can be done with a network analyzer to measure its S21 and uncertainty is introduced.

E1-16 Influence of the receiving antenna feed cable

If the probe antenna is directional (i.e. peak gain >+5 dBi, e.g. horn, LPDA, etc.) and the same probe antenna cable configuration is used for both stages, the uncertainty is considered systematic and constant  0.00 dB value.

In other cases a technical study should be done.

E1-17 Uncertainty of the absolute gain of the reference antenna

This contribution originates from the gain/efficiency uncertainty of calibration antenna that is employed in Stage 2 only. This uncertainty value will be indicated in the manufacturer's data in logs with a rectangular distribution, unless otherwise informed. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E1-18 Uncertainty of the absolute gain of the receiving antenna

The uncertainty appears in both stages and it is thus considered systematic and constant  0.00 dB value.

E2-19 Frequency flatness

This uncertainty contribution to account for the frequency interpolation error caused by a finite frequency resolution during the calibration stage.

E1-20 Quality of quiet zone (extreme)

This contribution is related to the ambient quality of the quiet zone (C1-3, 3GPP TR 37.842 [4]) which originates from a reflectivity level of an anechoic chamber. The reflectivity level is determined from the average standard deviation of the electric field in the quiet zone. As the environmental enclosure is larger than the DUT and the material of the environmental chamber may cause some reflection and refraction the quite zone flatness will be effected. The quality of the quiet zone for the extreme test is therefore larger tan that for the ambient due to the environmental enclosures effect.

E1-21 Wet radome loss variation

The environmental chamber radome will be an ineffective thermal isolator and will have extreme temperatures on the inside and the OTA chamber ambient temperature on the outside. In such conditions condensation is inevitable. This uncertainty is due to the variation in the radome loss due to condensation on the environmental chamber radome.

E1-22 Radome loss variation

The environmental chamber radome will affect the path between the DUT and the test antenna due to both its insertion loss and also reflection and refraction from the materials surface. The loss is dependent on the material as well as its proximity to the DUT. The uncertainty is the residual uncertainly of the total loss after calibration

E1-23 Change in absorber behaviour

The environmental chamber will contain RF absorptive material to prevent reflections within the chamber. This RF absorptive material will be subjected to the extreme temperatures inside the environmental chamber and hence its properties will change. This uncertainty accounts for the effect of that change in behavior.

# E.2	Compact Antenna Test Range

E2-1 Misalignment DUT/calibration antenna & pointing error

This contribution denotes uncertainty in DUT/calibration antenna alignment and DUT/calibration antenna pointing error.  In this measurement the DUT/calibration antenna is aligned to maximum, also allowing for a zero contribution for polarization mismatch uncertainty. By adjusting for maximums to align, this contribution can be a small contribution. The calibration antenna's phase centre and polarization purity changes slightly according to the frequency. Therefore, there should be some uncertainty reserved for this. To ensure that the point error is at a minimal, this contribution should be captured using the antenna pattern cut which is broadest (in the case of the DUT this would most likely be in the azimuth domain).

E2-2 Measurement Receiver: uncertainty of the absolute level

The receiving device used to measure the received signal level in the EIRP tests either as an absolute level or as a relative level. These receiving devices to name a few are spectrum analyzers, network analyzers or power meter. These devices will have an uncertainty contribution of their own; this value declared by the test gear vendor should be recorded as this uncertainty contribution. If a power meter is used then both measurement uncertainty and out of band noise is considered as part of the contribution. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E2-3 Standing wave between DUT and test range antenna

This value is extracting the uncertainty value and standard deviation of gain ripple coming from standing waves between DUT and test range antenna. This value can be captured by moving the DUT towards the test range antenna as the standing waves go in and out of phase causing a ripple in measured gain.

E2-4 RF leakage (SGH connector terminated & test range antenna connector cable terminated)

This contribution denotes noise leaking in to connector and cable(s) between test range antenna and receiving equipment.  The contribution also includes the noise leakage between the connector and cable(s) between SGH/reference antenna and transmitting equipment.

E2-5	 QZ ripple DUT/calibration antenna

This is the quiet zone (QZ) ripple experienced by the AAS BS/calibration antenna during the measurement or calibration phase. The purpose of this component is to capture the contributions that the reflections from the walls, roof and floor that will add to the EIRP measurement. The sum of all these reflections from the walls, range reflector (if applicable), roof and floor will give the overall value for the QZ ripple. In other words, the uncertainty component from the wall will not be separated from the roof or the floor. The purpose of this uncertainty component is to capture the overall reflections from the chamber walls experienced by the AAS BS/calibration antenna. To capture the full effect of the QZ ripple a distance of  1λ should be measured from each of the AAS BS/calibration antenna physical aperture edges, i.e. total QZ distance = physical aperture length +2 λ, to ensure the full volume of the QZ is captured in the uncertainty measurement.

E2-6 Uncertainty of network analyser

a)	drift (temp, oscillators, filters, etc.) start-to-end time of  measurements.

This uncertainty includes all the uncertainties involved in the S21 measurement (including drift and frequency flatness) with a network analyzer, and will be calculated from the manufacturer's data in logs. This uncertainty also includes analyzer uncertainty for multi-polarization (2 or more ports) measured simultaneously. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E2-7 Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver)

This uncertainty is the residual uncertainty contribution coming from multiple reflections between the receiving antenna and the test receiver equipment. This value can be captured through measurement by measuring the S11 towards the receive antenna and also towards the test receiver. The mismatch between the antenna reflection and the receiver reflection can also be calculated. If the same cable is used for calibration Stage 1, this can be considered systematic and negligible.

E2-8 Insertion loss of receiver chain

This uncertainty is the residual uncertainty contribution coming from introducing an antenna at the end of the cable.  If this cable does not change/move between the calibration Stage 1 and the measurement Stage 2, the uncertainty is assumed to be systematic and negligible during the measurement stage. Alternatively, the insertion loss can also be calculated by taking the measurement of the cable where port 2 is the end of the cable connected to the AAS BS or calibration antenna.

IL = -20log10|S21| dB

E2-9 Influence of the calibration antenna feed cable

a)	Flexing cables, adapters, attenuators, extra pathloss cable & connector repeatability.

During the calibration phase this cable is used to feed the calibration antenna and any influence it may have upon the measurements is captured. This is assessed by repeated measurements while flexing the cables and rotary joints. The largest difference between the results is recorded as the uncertainty.

E2-10 Uncertainty of the absolute gain of the calibration antenna

This uncertainty consists of the uncertainty of the gain value associated with the gain value denoted from the antenna calibration. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E2-11 Misalignment positioning system

This contribution originates from uncertainty in sliding position and turn table angle accuracy. If the calibration antenna is aligned to maximum this contribution can be considered negligible and therefore set to zero.

E2-12 Rotary Joints

If applicable the contribution of this uncertainty the accuracy in changing from azimuth to vertical measurements.

E2-14 Miscellaneous Uncertainty

The term 'miscellaneous uncertainty' is used to define all the unknown, unquantifiable, etc. uncertainties associated with EIRP measurements. This term should include truly random effects as well as systematic uncertainties, such as that arising from dissimilarity between the patterns of the reference antenna (SGH) and the DUT.

E2-15 Switching Uncertainty

The purpose of the switching unit is to switch electromechanically different RF path to different measurement instruments of different measurement modes. The electromechanical switching clearly reduces the errors arising from manual switching work. Switching is also used to measure the path loss values of each polarization component.  Even though the electromechanical switching is preferable during path loss and antenna performance measurements, some minor uncertainties can occur when the switch states are programmed to change their polarity.

E2-16 Frequency flatness

This uncertainty contribution to account for the frequency interpolation error caused by a finite frequency resolution during the calibration stage.

E2-17 Quality of quiet zone (extreme)

This contribution is related to the ambient quality of the quiet zone (C1-3, 3GPP TR 37.842 [4]) which originates from a reflectivity level of an anechoic chamber. The reflectivity level is determined from the average standard deviation of the electric field in the quiet zone. As the environmental enclosure is larger than the DUT and the material of the environmental chamber may cause some reflection and refraction the quite zone flatness will be effected. The quality of the quiet zone for the extreme test is therefore larger tan that for the ambient due to the environmental enclosures effect.

E2-18 Wet radome loss variation

The environmental chamber radome will be an ineffective thermal isolator and will have extreme temperatures on the inside and the OTA chamber ambient temperature on the outside. In such conditions condensation is inevitable. This uncertainty is due to the variation in the radome loss due to condensation on the environmental chamber radome.

E2-19 Radome loss variation

The environmental chamber radome will affect the path between the DUT and the test antenna due to both its insertion loss and also reflection and refraction from the materials surface. The loss is dependent on the material as well as its proximity to the DUT. The uncertainty is the residual uncertainly of the total loss after calibration

E2-20 Change in absorber behaviour

The environmental chamber will contain RF absorptive material to prevent reflections within the chamber. This RF absorptive material will be subjected to the extreme temperatures inside the environmental chamber and hence its properties will change. This uncertainty accounts for the effect of that change in behavior.

# E.3	Near Field Test Range

E3-1	Axes Intersection

This is a mechanical uncertainty term and aim to find the uncertainty related with the lateral displacement between the horizontal and vertical axes of the AAS BS positioner. This can result in sampling the field on a non-ideal sphere. This uncertainty is assumed to have a Gaussian distribution.

E3-2	Axes Orthogonality

The difference from 90° of the angle between the horizontal and vertical axes also results in sampling the field on a non-ideal sphere. This uncertainty is assumed to have a Gaussian distribution.

E3-3	Horizontal Pointing

The horizontal mispointing of the horizontal axis to the probe reference point for Theta=0° also results in sampling the field on a non-ideal sphere. This uncertainty is assumed to have a Gaussian distribution.

E3-4	Probe Vertical position

The vertical displacement of the probe reference point from the horizontal axis results in sampling the field on a non-ideal sphere. This uncertainty is assumed to have a Gaussian distribution.

E3-5	Probe Horizontal/Vertical pointing

The horizontal or vertical mispointing of the probe z-axis from the intersection point of the horizontal/vertical axis. This uncertainty is assumed to have a Gaussian distribution.

E3-6	Measurement distance

This is the knowledge of the distance between the intersection point of the horizontal and vertical axis and probe reference point. This uncertainty is assumed to have a Gaussian distribution.

E3-7	Amplitude and phase drift

The system drift due to temperature variations the signal at AAS BS location to drift in amplitude and phase. This uncertainty is assumed to have a Gaussian distribution.

E3-8	Amplitude and phase noise

This uncertainty is due to the noise level of the test range so that the S/N ratio should be determined or measured at the AAS BS location. The noise level is usually measured with a Spectrum Analyzer. This uncertainty is assumed to have a Gaussian distribution.

E3-9	Leakage and Crosstalk

This uncertainty can be addressed by measurements on the actual system setup. The leakage and crosstalk cannot be separated from the random amplitude and phase errors so that the relative importance should be determined. This uncertainty is assumed to have a Gaussian distribution.

E3-10	Amplitude non-Linearity

This uncertainty is the linearity of the receiver used for the measurement. It can be taken from the data sheet of the receiver.

E3-11	Amplitude and phase shift in rotary joint

This uncertainty is due to the variation of the rotary joint. It can be measured and is assumed to have a Gaussian distribution.

E3-12	Channel balance amplitude and phase

This uncertainty is relevant for systems which are using dual polarized probes and polarization switches. The amplitude and phase difference between two signal channels of the receiver includes the difference between the probe ports, difference between the channels of the polarization switch, connecting cables and reflection coefficients. This uncertainty is assumed to have a Gaussian distribution.

E3-13	Probe polarization amplitude and phase

The amplitude and phase of the probe polarization coefficients should be measured. This uncertainty is assumed to have a Gaussian distribution.

E3-14	Probe pattern knowledge

The probe(s) pattern(s) is assumed to be known so that the AAS BS measurement in near field can be corrected when performing the near field to far field transform. There is no direct dependence between the AAS BS pattern and the probe pattern in near field measurements. This uncertainty is assumed to have a Gaussian distribution.

E3-15	Multiple reflections

The multiple reflections occur when a portion of the transmitted signal is reflected form the receiving antenna back to the transmitting antenna and re-reflected by the transmitting antenna back to the receiving antenna. This uncertainty can be determined by multiple measurements of the AAS BS when at different distance from the probes. This uncertainty is assumed to have a Gaussian distribution.

E3-16	Room scattering

As for the multiple reflections, a portion of the transmitted signal is reflected by either the absorbers or other structures in the measurement anechoic chamber before being received by the receiving antenna. This effect can be isolated from the multiple reflections by testing the AAS BS in different positions, separated by λ/4 with respect to the anechoic chamber and comparing these measurements with the reference. This uncertainty is assumed to have a Gaussian distribution.

E3-17	DUT support scattering

This is the uncertainty due to the AAS BS supporting structure on the signal level. This uncertainty is assumed to have a Gaussian distribution.

E3-18	Scan area truncation

This uncertainty does affect this near field measurement. It can be addressed by comparing the measurement result when scanning the full area. This uncertainty is assumed to have a Gaussian distribution.

E3-19	Sampling point offset

This uncertainty has an influence in near field and far field. It is assumed to have a Gaussian distribution.

E3-20	Mode truncation

The measured near field is expanded using a finite set of spherical modes. The number of modes is linked to number of samples. The filtering effect generated by the finite number of modes can improve measurement results by removing signals from outside the physical area of the AAS BS. Care should be taken in order to make sure the removed signals are not from the AAS BS itself. This uncertainty is usually negligible.

E3-21	Positioning

The relative position of the probe array is not ideal. This uncertainty is assumed to have a rectangular distribution.

E3-22	Probe array uniformity

This is the uncertainty due to the fact that different probes are used for each physical position. Different probes have different radiation patterns. Generally, the probe array is calibrated so that the uniformity of the probes is achieved.

E3-23	Mismatch of receiver chain

If the same chain configuration (including the measurement receiver; the probe antenna and other elements) is used in both stages, the uncertainty is considered systematic and constant  0.00 dB value.

If it is not the case, this uncertainty contribution has to be taken into account and should be measured or determined by the method described in [14]. This uncertainty is assumed to have a U-shaped distribution

E3-24	Insertion loss of receiver chain

It is composed of the following:

-	Insertion loss of the probe antenna cable.

-	Insertion loss of the probe antenna attenuator (if used).

-	Insertion loss of RF relays (if used).

If the same chain configuration is used for measurement and calibration, the uncertainty due to the above components is considered systematic and constant  0.00 dB value. This uncertainty is assumed to have a Gaussian distribution.

E3-25	Uncertainty of the absolute gain of the probe antenna

This uncertainty appears in the both stages and it is thus considered systematic and constant 0.00 dB value.

E3-26	Measurement Receiver

This uncertainty is composed of three uncertainties being:

-	absolute power level;

-	stability;

-	stability over temperature;

-	linearity.

The expanded uncertainty is the Root Square Sum (RSS) of the standard uncertainties for each uncertainty contributors. Each uncertainty can be taken from the instrument data sheet and is assumed to have a uniform distribution. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E3-27	Measurement Repeatability - Positioning Repeatability

This uncertainty is due to the repositioning of the AAS BS in the test setup. It can be addressed by repeating the corresponding measurement 5 times. Calculate the standard deviation of the metric obtained and use that as the measurement uncertainty. For tests that require multiple setups, the worst-case standard deviation is used. This uncertainty is assumed to have a Gaussian distribution.

E3-28	Uncertainty of Vector Network Analyzer

This uncertainty includes the all uncertainties involved in the S21 measurement with a vector network analyzer, and will be calculated from the manufacturer's data sheet. This uncertainty is assumed to have a uniform distribution. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E3-29	Mismatch of receiver chain

If the same chain configuration (including the measurement receiver; the probe antenna and other elements) is used in both stages, the uncertainty is considered systematic and constant  0.00 dB value.

If it is not the case, each uncertainty contribution has to be taken into account and should be measured or determined  and then taking the total of all non-zero mismatch uncertainty contribution from all parts by root-sum-squares (RSS) method. This uncertainty is assumed to have a Gaussian distribution.

E3-30	Insertion loss of receiver chain

If the same chain configuration is used for measurement and calibration, the uncertainty due to the above components is considered systematic and constant  0.00 dB value. This uncertainty is assumed to have a Gaussian distribution.

E3-31	Mismatch in the connection of the calibration antenna

This is the uncertainty from the mismatch in the connection between the system coax cable and the calibration antenna. This uncertainty is from the mismatch between the cable and the reference antenna that is used for calibration. It is determined by the S11 of the reference antenna and the S11 of the cable to which the antenna is connected i.e. if using an SGH antenna for calibration and 10dB pad is inserted on the cable connecting to the antenna this uncertainty contribution can be considered negligible. This uncertainty is assumed to have a U-shaped distribution.

E3-32	Influence of the calibration antenna feed cable

This uncertainty is due to the impact of the feeding cable on the radiation properties of the calibration antenna. In case of using either a standard horn or standard gain horn, the impact of the cable is to be considered negligible thus the uncertainty  0.00 dB value. In case of using a dipole-like antenna, the uncertainty should be addressed by measuring this impact. This uncertainty is assumed to have a Gaussian distribution.

E3-33	Influence of the probe antenna cable

If the same chain configuration is used for measurement and calibration, the uncertainty due to the above components is considered systematic and constant  0.00 dB value. This uncertainty is assumed to have a Gaussian distribution.

E3-34	Uncertainty of the absolute gain of the calibration antenna

The calibration antenna only appears in Stage 2. Therefore, the gain/efficiency uncertainty has to be taken into account. This uncertainty will be calculated from the manufacturer's calibration certificate and is assumed to have a uniform distribution, unless otherwise informed  This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E3-35 Short term repeatability

It can be addressed by performing a repeatability test of the calibration antenna. This uncertainty is assumed to have a Gaussian distribution.

E3-36 Frequency flatness

This uncertainty contribution to account for the frequency interpolation error caused by a finite frequency resolution during the calibration stage.

# E.4	General Chamber

This subclause describes the uncertainties for a general wide band chamber.

E4-1 Positioning misalignment between the AAS BS and the reference antenna

This contribution originates from the misalignment of the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the reference antenna. The uncertainty makes the space propagation loss between the AAS BS and the receiving antenna at the AAS BS measurement stage (i.e. Stage 2) different from the space propagation loss between the reference antenna and the receiving antenna at the calibration stage (i.e. Stage 1).

E4-2 Pointing misalignment between the AAS BS and the receiving antenna

This contribution originates from the misalignment of the testing direction and the beam peak direction of the receiving antenna due to imperfect rotation operation. The pointing misalignment may happen in both azimuth and vertical directions and the effect of the misalignment depends highly on the beamwidth of the beam under test. The same level of misalignment results in a larger measurement error for a narrower beam.

E4-3 Quality of quiet zone

This contribution originates from a reflectivity level of an anechoic chamber. The reflectivity level is determined from the average standard deviation of the electric field in the quiet zone. By repeating a free space VSWR measurement in 15° grid in elevation and azimuth, 264 standard deviation values in both polarizations are determined. From these values an average standard deviation of electric field in the quiet zone can be calculated from the equation:

Where:

is the number of angular intervals in elevation,

is the number of angular intervals in azimuth, and

is elevation of single measurement .

If an efficiency calibration with omni-directional calibration antenna is performed, the effect of reflectivity level decreases in Stage 1 (i.e. calibration measurement) and  may be divided by factor 2. This is due to correcting impact of data averaging in this type of calibration. Efficiency calibration done with sampling step ≤ 30°, can be considered to have at least four independent samples.  may be divided by factor 2 also in Stage 2 (i.e. AAS BS measurement) for the same reason.

It's likely that asymmetry of the field probe will have a very small impact on this measurement uncertainty contributor, however, an upper bound to probe symmetry should be considered.

E4-4 Polarization mismatch between the AAS BS/reference antenna and the receiving antenna

This contribution originates from the misaligned polarization between the AAS BS/reference antenna and the receiving antenna.

E4-5 Mutual coupling between the AAS BS/reference antenna and the receiving antenna

This contribution originates from mutual coupling between the AAS BS/reference antenna and the receiving antenna. Mutual coupling degrades not just the antenna efficiency, i. e. the EIRP value, but it can alter the antenna's radiation pattern as well. For indoor anechoic chamber, usually the spacing between the AAS BS/reference antenna and the receiving antennas is large enough so that the level of mutual coupling might be negligible.

E4-6 Phase curvature

This contribution originates from the finite far field measurement distance, which causes phase curvature across the antenna of AAS BS/reference antenna.

E4-7 Uncertainty of the measurement equipment

This contribution originates from limited absolute level accuracy and non-linearity of the measurement equipment. The measurement equipment such as a BS simulator , spectrum analyzer , or power meter  measures the received signal level in EIRP tests either as an absolute level or as a relative level. The uncertainty value will be indicated in the manufacturer's data sheet in logs. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E4-8 Impedance mismatch in the receiving chain

This contribution originates from multiple reflections between the receiving antenna and the power measurement equipment. The multiple reflections can produce an overall reflection that depends not only upon the individual reflections of each part but their reflective interactions as well. The combination loss by the overall reflection can be higher or lower than individual loss by multiple reflections. The combination loss is called the mismatch error and leads to the measurement uncertainty.

E4-9 Random uncertainty

The random uncertainty characterizes the undefined and miscellaneous effects which cannot be forecasted. One can estimate this type of uncertainty with a repeatability test by making a series of repeated measurement with a reference AAS BS without changing anything in the measurement set-up.

E4-10 Impedance mismatch between the receiving antenna and the network analyzer

This contribution originates from multiple reflections between the receiving antenna and the network analyzer. After appropriate calibration, the network analyzer may not introduce impedance mismatch error, but the error still happens between the receiving antenna feed cable and the receiving antenna.

E4-11 Positioning and pointing misalignment between the reference antenna and the receiving antenna

This contribution originates from reference antenna alignment and pointing error. In this measurement if the maximum gain directions of the reference antenna and the receiving antenna are aligned to each other, this contribution can be considered negligible and therefore set to zero.

E4-12 Impedance mismatch between the reference antenna and the network analyzer

This contribution originates from multiple reflections between the reference antenna and the network analyzer. After appropriate calibration, the network analyzer may not introduce impedance mismatch error, but the error still happens between the reference antenna feed cable and the reference antenna.

E4-13 Uncertainty of the network analyzer

This contribution originates from all uncertainties involved in the S21 measurement (including drift and frequency flatness) with a network analyzer. The uncertainty value will be indicated in the manufacturer's data sheet in logs. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E4-14 Influence of the reference antenna feed cable

In the calibration Stage 1, the influence of the calibration antenna feed cable is assessed by measurements. A measurement for calibration may be repeated with a reasonably differing routing of the feed cable. Largest difference among the results is entered to the uncertainty budget with a rectangular distribution.

E4-15 Reference antenna feed cable loss measurement uncertainty

Before performing the calibration, the reference antenna feed cable loss have to be measured. The measurement can be done with a network analyzer to measure its S21 and uncertainty is introduced.

E4-16 Influence of the receiving antenna feed cable

If the probe antenna is directional (i.e. peak gain >+5 dBi, e.g. horn, LPDA, etc.) and the same probe antenna cable configuration is used for both stages, the uncertainty is considered systematic and constant  0.00 dB value.

In other cases a technical study should be done.

E4-17 Uncertainty of the absolute gain of the reference antenna

This contribution originates from the gain/efficiency uncertainty of calibration antenna that is employed in Stage 2 only. This uncertainty value will be indicated in the manufacturer's data in logs with a rectangular distribution, unless otherwise informed. This uncertainty value can be found in Annex F and was a result of compromised value in order to align all test methods having this uncertainty contribution.

E4-18 Uncertainty of the absolute gain of the receiving antenna

The uncertainty appears in both stages and it is thus considered systematic and constant  0.00 dB value.

E2-19 Frequency flatness

This uncertainty contribution to account for the frequency interpolation error caused by a finite frequency resolution during the calibration stage.

E4-20 Measurement antenna frequency variation

For wide band measurement the measurement antenna gain will vary considerably over frequency. The gain can be calibrated howebr variation may still remain between calibration frequency steps. This uncertainty accounts for the variation between the calibrated steps.

E4-21 FSPL estimation error

For wide band measurement the measurement free space path loss in the chamber  will vary with frequency. The loss can be calibrated however this uncertainty accounts for the variation between the calibrated steps.

E4-21 - Measurement system dynamic range uncertainty

Uncertainty associated with the addition of each of the directional power measurements to calculate the TRP due to the limited dynamic range of the OTA test system causing an overestimation.

E4-22 - Conducted measurement uncertainty (minus mismatch)

Measurement uncertainty value taken from the conducted measurement uncertainty in 3GPP 36.141 [22]. Uncertainty includes all conducted uncertainties associated with the conducted part of the measurement (filters, limiters, switches etc.). The mismatch uncertainty is removed to prevent it being considered twice in the conducted part and the OTA chamber part of the analysis.

# E.5	Reverberation Chamber

E5-1 Uncertainty of the measurement equipment

This contribution originates from limited absolute level accuracy and non-linearity of the measurement equipment. The measurement equipment such as a network analyser, signal generator, spectrum analyzer, or power meter measures the received signal level in EIRP tests either as an absolute level or as a relative level. The uncertainty value will be indicated in the manufacturer's data sheet in logs.

E5-2 Impedance mismatch in the receiving chain

This contribution originates from multiple reflections between the receiving antenna and the power measurement equipment. The multiple reflections can produce an overall reflection that depends not only upon the individual reflections of each part but their reflective interactions as well. The combination loss by the overall reflection can be higher or lower than individual loss by multiple reflections. The combination loss is called the mismatch error and leads to the measurement uncertainty.

E5-3 Random uncertainty

The random uncertainty characterizes the undefined and miscellaneous effects which cannot be forecasted. One can estimate this type of uncertainty with a repeatability test by making a series of repeated measurement with a reference AAS BS without changing anything in the measurement set-up.

E5-4 Reference antenna radiation efficiency

Residue of uncertainty of reference antenna radiation efficiency after calibration.

E5-5 Mean value estimation of reference antenna mismatch efficiency

Error of the estimated mean related to the use of a finite number N of samples in the measurement of the reference antenna mismatch efficiency. The mean value estimation is calculated as σ/N where σ is the standard deviation of the series of measured values.

E5-6 Uncertainty of the network analyzer

This contribution originates from all uncertainties involved in the S21 measurement (including drift and frequency flatness) with a network analyzer. The uncertainty value will be indicated in the manufacturer's data sheet in logs.

E5-7 Influence of the reference antenna feed cable

Before performing the calibration, the reference antenna feed cable loss has to be measured. The measurement can be done with a network analyzer to measure its S21 and uncertainty is introduced.

E5-8 Mean value estimation of transfer function

Error of the estimated mean related to the use of a finite number N of samples in the measurement of the transfer function. The mean value estimation is calculated as σ/N where σ is the standard deviation of the series of measured values.

E5-9 Uniformity of transfer function

Standard deviation over DUT positions and rotations of the transfer function S212. This uncertainty expresses the variations of measured TRP values with respect to translations and rotations of the DUT. Ideally, the TRP does neither depend on translations nor rotations of the DUT.

Annex F:
Test equipment uncertainty values

# F.1	Measurement Equipment uncertainty values

The following uncertainty distribution and standard uncertainty (σ) values proposed by test vendors are adopted for the RF power measurement equipment, RF signal generator, and network analyzer in all EIRP and EIS test methods for AAS BS to calculate the uncertainty budget.

Standard uncertainty values captured in this annex are based on Way Forward agreements in R4-164720.

Table F.1-1: Test equipment uncertainty values

| Instrument | Use case | Measurement Uncertainty type | Standard uncertainty σ (dB) |  | Probability distribution |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≦ 3 GHz | 3 GHz < f ≦ 4.2 GHz |  |
| RF power measurement equipment (e.g. spectrum analyzer, power meter) | EIRP measurement stage | Total amplitude accuracy (with input levels down to -70 dBm) | 0.14 | 0.26 | Gaussian |
| RF signal generator | EIS measurement stage | Level error | 0.46 | 0.46 | Gaussian |
| Network analyzer | Calibration stage | Accuracy of transmission measurements | 0.13 | 0.20 | Gaussian |
| NOTE: Standard uncertainty values were derived from datasheets of mid-tier to high-end RF signal generators, spectrum analyzers, and VNAs. Standard uncertainty values of power measurement equipment were derived from datasheet of spectrum analyzers. |  |  |  |  |  |

The following uncertainty distribution and standard uncertainty (σ) value for the reference antenna derived as the maximum of companies' proposals are adopted in all test methods to calculate the uncertainty budget.

Table F.1-2: Reference antenna uncertainty value

| Instrument | Use case | Standard uncertainty σ (dB) |  | Probability distribution |
| --- | --- | --- | --- | --- |
|  |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |  |
| Reference antenna | Calibration stage | 0.29 | 0.25 | Rectangular |

# F.2	MU of TE derived from conducted specification

For a number of  test cases the conducted measurement uncertainty used in 3GPP 36.141 [22] is used to estimate the uncertainty contributing of the conducted part of the OTA test set up.

Where appropriate the mismatch uncertainty is removed from the conducted uncertainty so that it is not included twice in the calculation.

Table F.2-1: MU derived from the conducted specification

| Requirement | Use case | Standard uncertainty σ (dB) |  |  |  | Probability distribution |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz | 4.2GHz < f ≦ 6 GHz | 6GHz < f ≦ 19 GHz |  |
| OTA E-UTRA DL RS power | Measurement stage | 0.41 | 0.56 | N/A | N/A | Gaussian |  |
| OTA Output power dynamics | Measurement stage | 0.2 | 0.2 | N/A | N/A | Gaussian |  |
| Transmitter mandatory spurious emissions | Measurement stage | 1.02 |  |  | 2.04 | Gaussian |  |
| Receiver spurious emissions | Measurement stage | 1 |  |  | 1.99 | Gaussian |  |
| Additional (co-existence) spurious emissions | Measurement stage | 1.02 | 1.02 | 1.28 | N/A | Gaussian |  |

Annex G:
Power density measurements close to EUT

This annex contains guidelines for measurement of power density flux on a sphere surrounding the Equipment Under Test (EUT). The rationale for using a small test distance (compared to the far-field of EUT) is that any amount of power that radiates from the EUT also will radiate through any sphere enclosing the EUT, i.e., energy conservation.

Power flux density based on measurement of tangential electric field components can be used for correct TRP assessment [31]. For accurate power density assessment, the following conditions need to be met:

1. The test distance shall be at least the far-field distance of the measurement antenna, i.e., r0≥2DMA2/λ

2. The measurement antenna shall sample an approximately constant field.

3. The test distance must be at least two wavelengths from the the smallest sphere enclosing the EUT.

The second criterion is quantified as:

,	(G-1)

where Rsph is the radius of the smallest sphere enclosing the EUT. Furthermore, equstion (G-1) sets a minimum test distance for a given measurement antenna size w [31].

Figure G-1: The measurement distance and the dimensions of EUT and MA.

Example: for assessing TRP using power density data close to EUT, consider the patterns in Figure G-2, which are sampled at (a) 0.75 m, (b) 2.5 m and (c) 30 m from the same EUT. The far-field distance of the EUT is 30 m. The angular patterns vary with distance, but the full-sphere integration for TRP deviates less than 0.065 dB. The lower power density levels in the lower figure are balanced by the larger total area of the measurement sphere.

Figure G-2: Angular power density patterns measured at (a) 0.75 m, (b) 2.5 m and (c) 30 m from the EUT [31].

Annex H:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-08 | RAN4#80bis | R4-168871 |  |  |  | Skeleton | 0.0.1 |
| 2017-02 | RAN4#81 | R4-1701717 |  |  |  | Approved text proposals in the following documents were implemented: R4-1609938, TP to 37.843: Background information on conducted RX sensitivity requirement R4-1610393, TP to TR37.843 on EVM R4-1610400, TP to TR 37.843 definitions and abbreviations R4-1610799, TP to TR 37.843: Remaining OTA requirements for eAAS BS R4-1610802, TP to 37.843: Background information on conducted ACLR requirement R4-1610803, TP to TR 37.843 on ACLR R4-1610808, TP to TR 37.843 on min EIS | 0.1.0 |
| 2017-02 | RAN4#82 | R4-1703913 |  |  |  | R4-1701717 TR 37.843 v0.1.0 Cleanup TP in R4-1701873 R4-1701873 TP to the TR 37.843: cleanup R4-1701061 TP for TR 37.843: TR structure update R4-1701730 TP to TR37.843 Updating reference architecture diagrams R4-1702261 TP to 37.843: Coordinate system for eAAS R4-1702262 TP to TR37.843 Hybrid requirements structure Huawei R4-1702264 TP for TR 37.843: Adding text for Operating band unwanted emissions OTA requirement R4-1702265 TP for TR 37.843: Adding background and OTA information for spurious emissions R4-1702267 TP to 37.843. EVM background information R4-1702268 TP for TR 37.843: Adding background information on OTA frequency error requirement R4-1702269 TP for TR 37.843: Adding background information about occupied bandwidth requirement R4-1702272 TP to 37.843: Background information on conducted RX blocking requirement R4-1702273 TP on Spurious Emission & EMC Radiated Emission requirements for AAS R4-1702274 "TP on the protection of measurement devices for EMC testing Decision: The document was not treated. " R4-1702275 TP on Regulatory EMC requirements for AAS R4-1702276 TP on EMC field strength estimation for AAS BS R4-1702277 TP on general section for EMC requirements R4-1702280 TP to TR 37.843 – maximum rated output power limits for BS classes R4-1702282 TP to TR 37.843: Overview of the Tx and Rx requirements and their compliance ranges | 0.2.0 |
| 2017-04 | RAN4#82bis | R4-1705637 |  |  |  | R4-1703914 TP to TR37.843 – definition of gain R4-1703942 TP to TR 37.843: Minimum power requirements applicability R4-1704322 TP for TR 37.843: Adding background information for TAE requirement R4-1704324 TP for TR 37.843: Addition of background information about OTA transmitter intermodulation requirement R4- 1704329 TP for TR 37.843: General Section to Sub-clause 5.6 R4-1704330 TP to TR 37.843: Spurious frequency range correction for the RF spurious emissions requirement in OTA setup R4-1704331 TP to TR 37.843: Capturing agreements on the OBUE requirement in OTA setup R4- 1704332 TP Minimum EIS for eAAS OTA Sensitivity Requirements R4-1704334 TP to TR 37.843 – Blocking R4-1704335 TP to TR 37.843 Receiver ACS and NB blocking R4-1704338 TP to TR 37.843 - Transmitter direction range definitions and declarations R4-1704339 TP to TR 37.843 – Capturing EIRP accuracy requirement R4-1704341 TP for 37.843 - TX output power dynamics requirements R4-1704342 TP for TR 37.843 On exclusion band for the protection of measurement devices for spurious emission testing R4-1704343 TP for TR 37.843 On summing Sp Emission & EMC RE requirements for AAS R4-1704344 TP to TR 37.843: OTA EMC requirements structure R4-1704345 TP to TR 37.843: Frequency range of the EMC RE and RF RSE measurements in OTA setup R4-1704346 TP to TR 37.843: Conformance testing aspects for OTA requirements R4-1704347 TP for TR 37.843: OTA testing of BS demodulation requirements | 0.3.0 |
| 2017-05 | RAN4#83 | R4-1708390 |  |  |  | R4-1705806 TP to TR 37.843: Alignment of the requirements structure in the eAAS TR R4-1704879 TP for TR 37.843: Adding Annex for background information common for co-location requirements R4-1705315 Operating band unwanted emission for eAAS R4-1705802 TP to TR 37.843: Total power dynamic range requirement R4-1705803 TP to TR 37.843: Rx spurious requirement OTA R4-1705804 TP to TR 37.843: Further details on the frequency error requirement OTA R4-1705807 TP to TR 37.843: UTRA demodulation requirements analysis R4-1706124 TP for TR 37.843: Improvements of definitions of spatial angles in sub-clause 4.6 R4-1706125 TP to TR 37.843: BS classification for eAAS BS R4-1706131 TP to TR 37.843 – TRP accuracy R4-1706135 TP for TR 37.843 - TX Power requirements for control channels R4-1703138 TP for TR 37.843: Section 4.4 R4-1706139 TP to TR 37.843: Consideration of missing aspects for core and conformance Tx spurious requirement OTA R4-1706140 TP to TR 37.843 – Clarify example in sub-clause 5.1 R4-1706141 TP to TR 37.843: OTA sensitivity requirements for eAAS R4-1706144 TP for 37.843 – Clarifying Declarations for OTA sensitivity and OTA REFSENS R4-1706150 TP to TR 37.843: Consideration of the manufacturer's declarations for Hybrid AAS BS and OTA AAS BS R4-1706151 TP to TR 37.843: Additional considerations for the OTA test setups R4-1706152 TP to TR 37.843: Further details on the definition of core and conformance requirements for BS demodulation requirements R4-1706153 TP to TR 37.843: Placeholder for the BS performance requirements doable OTA R4-1706229 TP for TR 37.843 EMC immunity for OTA AAS BS – general Section R4-1706230 TP for TR 37.843 EMC immunity for OTA AAS BS – Measurement setup | 0.4.0 |
| 2017-08 | RAN4#84 | R4-1701784 |  |  |  | R4-1708878 TP to TR37.843: cleanup R4-1707481 TP for TR 37.843 on RI spatial exclusion R4-1707906 TP to TR37.843: new High Speed requirements for PRACH R4-1708401 TP to TR 37.843 - capture agreements on TX IMD R4-1708407 TP to TR 37.843 – Correcting BS power class limits R4-1708408 TP to TR 37.843 – Occupied BW OTA requirement type R4-1708879 TP to TR 37.843: General architecture update for hybrid and OTA AAS BS: RIB interface R4-1708891 TP to TR 37.843 - capture agreements on co-location requirements R4-1708950 TP for TR 37.843: Implementation of agreements on loss factor and emissions scaling R4-1708951 TP for TR 37.843 on co-existence in the same geographical area R4-1708953 TP to TR 37.843: fixed scaling and losses for the unwanted emissions R4-1708954 Transmit pulse shape filter requirement for OTA AAS BS R4-1708958 TP to TR 37.843 - Capturing agreements on OTA REFSENS R4-1709178 TP to TR 37.843: SEM for UTRA | 0.5.0 |
| 2017-10 | RAN4#84bis | R4-1713228 |  |  |  | R4-1711333 TP to TR37.843: correction of the RF RSE and EMC RE requirement R4-1711791 TP to TR 37.843: EIRP type Requirements R4-1711793 TP for TR 37.843: OTA modulation quality requirement range (5.5.4) R4-1711797 TP to TR 37.843 – Co-location requirements R4-1711801 TP to TR 37.843: OTA receiver blocking R4-1711802 TP to TR 37.843: OTA receiver ACS R4-1711803 TP to TR 37.843: OTA RX in channel selectivity R4-1711804 TP to TR 37.843: OTA Receiver Intermodulation R4-1711805 TP to TR 37.843: OTA receiver dynamic range R4-1711855 TP to TR 37.843 - extreme temperature power accuracy limits R4-1711862 TP to TR 37.843 - Out of band blocking | 0.6.0 |
| 2017-11 | RAN4#85 | R4-1713230 |  |  |  | R4-1713229 TP to TR 37.843 - clean up R4-1713251 TP to TR 37.843 update WI scope based on spec simplification agreements R4-1714236 TP to TR 37.843: in-band and out-of-band boundary for AAS BS R4-1714218 TP to TR 37.843 - TX IMD - co-location requirements R4-1714219 TP to TR 37.843 - co-location spurious emissions requirements R4-1714220 TP to TR 37.843 - co-location requirements definitions R4-1713255 TP to TR 37.843 - update transmitter radiated requirements table in sub-clause 5.1 R4-1713673 TP to TR 37.843: BS class power limits (5.2.4) R4-1713277 TP to TR 37.843: Out of band blocking frequency range R4-1712635 TP to TR 37.843: Reuse of the FRC’s for OTA Rx requirements R4-1713247 TP to TR 37.843 - Add agreements on receiver requirements R4-1714214 TP for TR 37.843: Extension to RIB definition in clause 3 and sub-clause 4.4 R4-1714223 TP to TR 37.843 - TX OFF level R4-1714225 TP to TR 37.843 - on out of band blocking. R4-1714226 TP for TR 37.843: Section 6.5.2 Out of band blocking OTA core requirement R4-1714229 TP to TR 37.843 - Add agreements on receiver sensitivity to sub-clause 6.2 R4-1714237 TP for TR 37.843: Addition of TRP in terminology in clause 3 and 5 R4-1714238 TP for TR 37.843: CATR Test Method Procedure for EVM R4-1714239 TP to TR 37.843: further refinements for OTA BS demodulation requirements R4-1714280 TP to TR 37.843 - update receiver radiated requirements table in sub-clause 6.1 R4-1714231 TP to TR 37.843 Introduction of Radiated Immunity and Spatial Exclusion | 0.7.0 |
| 2017-12 | RAN#78 | RP-172548 |  |  |  | Presented to RAN#78 for Approval | 1.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 under change control | 15.0.0 |
| 2018-09 | RAN#81 | RP-181907 | 0001 |  | B | CR to TR 37.843 | 15.1.0 |
| 2018-12 | RAN#82 | RP-182387 | 0005 |  | F | CR to TR 37.843 v15.1.0 | 15.2.0 |
| 2018-12 | RAN#82 | RP-182387 | 0006 | 2 | B | New Annex to TR 37.843: Power density measurements close to EUT | 15.2.0 |
| 2019-03 | RAN#83 | RP-190418 | 0008 |  | F | CR to TR 37.843: Addition of uncertainty sources relevant for RC in Annex E | 15.3.0 |
| 2019-03 | RAN#83 | RP-190419 | 0009 | 1 | F | CR to TR 37.843: Addition of RC MU evaluation and description in clause 2 and sub-clause 10.5 | 15.3.0 |
| 2019-03 | RAN#83 | RP-190419 | 0010 | 1 | F | CR to TR 37.843: Editorial clean-up of TRP measurement section in sub-clause 10.8 | 15.3.0 |
| 2019-03 | RAN#83 | RP-190419 | 0011 | 2 | F | CR to TR 37.843: Wave vector spacing sampling grid (Section 10.8.2.6) | 15.3.0 |
| 2019-06 | RAN#84 | RP-191263 | 0017 |  | F | CR to TR37.843, correct references to annex E | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0018 |  | F | CR to TR 37.843: Addition of RC MU evaluation for spurious emission in subclause 10.5.2.3A.5.2 | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0019 |  | F | CR to TR 37.843: Update of RC description in subclause 10.5.2.3A | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0020 |  | F | CR to TR 37.843: Addition of MU evaluation for testing output power, ACLR and OBUE in RC test method in subclause 10.4 | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0021 |  | F | CR to TR 37.843: Removal of information in Annex A | 15.4.0 |
| 2019-06 | RAN#84 | RP-191262 | 0022 |  | F | CR to TR 37.843: Upper limit for angular step for TRP calculation | 15.4.0 |
| 2019-06 | RAN#84 | RP-191262 | 0023 | 2 | F | CR to TP 37.843: Correction on usage of term EIRP | 15.4.0 |
| 2019-06 | RAN#84 | RP-191262 | 0024 | 2 | F | CR to TR 37.843 removal of Tx Diversity for TAE testing | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0025 | 1 | F | CR to TR 37.843: editorial corrections to spherical angle formula in subclause 10.8.2.4 | 15.4.0 |
| 2019-06 | RAN#84 | RP-191263 | 0026 | 2 | F | CR to TR37.843 Correction on beam based direcrtions | 15.4.0 |