Annex B (normative):
Conditions for RRM requirements applicability for operating bands

# B.1	Conditions for NR RRC_IDLE state mobility

## B.1.1	Introduction

In Annex B.1, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurements procedures and requirements in clause 4,

-	UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in clause 4.

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -122.3+Y2 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -125.3+Y1 | -122.3+Y2 | -109.1 | -124.8+Y4 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -122.3+Y2 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -111.3+Z2 | -105.2+Z3 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -117.3+Z1 | -111.3+Z2 | -105.2+Z3 | -115.8+Z4 |  |  |
|  |  | n260 | -114.3+Z1 |  | -100.9+Z3 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -111.3+Z2 | -105.2+Z3 | -115.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.1.2-2:

- The value of Y for Power classes 1, 2 and 4 is FFS, where Y1, Y2 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1, 2 and 4 respectively

- The value of Z for Power classes 1, 2, 3 and 4 is FFS, where Z1, Z2, Z3, and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1, 2, 3 and 4 respectively

## B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection

This clause defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.2-1 for FR1 NR intra-frequency cell re-selection shall also apply for FR1 NR inter-frequency cells in this clause.

The conditions defined in Table B.1.2-2 for FR2 NR intra-frequency cell re-selection shall also apply for FR2 NR inter-frequency cells in this clause.

# B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state

## B.2.1	Introduction

### B.2.1.1	General

In Annex B.2, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurements procedures and requirements in clause 9,

UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in clause 9,

-	UE conditions which shall apply for UE intra-frequency measurements performance requirements in clause 10,

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in clause 10.

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in clause 10,

-	The conditions for RRC connection release with redirection to NR requirements in clause 6.2.3.2.1,

-	The conditions for UE transmit timing adjustment in clause 7.1.

### B.2.1.2	Derivation of Minimum SSB_RP values for FR1

[FFS]

### B.2.1.3	Derivation of Minimum SSB_RP values for FR2

#### B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival

Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on Reference sensitivity for the Operating band and for the UE Power class, and taking a baseline of UE Power class 3 in Band n260 with 50 MHz channel bandwidth.

Minimum SSB_RP = Reference sensitivity PC3, n260, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ΣMBP

where:

EIS Reference sensitivity PC3, n260, 50MHz is the Reference sensitivity value in dBm specified for Power Class 3 in Band n260 for 50MHz Channel bandwidth in Table 7.3.2.3-1 of TS 38.101-2 [19].

Y is the gain difference between fine and rough beams, and is 7dB for a Power Class 3 UE. The value for other Power Classes is defined in Table B.2.1.3.1-1.

Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction

| Value “Y” in dB, for each UE Power class |  |  |  |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| FFS | [9.0] | 7.0 | FFS |

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [19] Table 5.3.2-1, and is 32.

12 is the number of subcarriers in a PRB.

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1dB.

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6dB for intra-frequency measurements and -4dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise.

ΣMBP is the UE multi-band relaxation factor value in dB specified in TS 38.101-2 [19] clause 6.2.1.

The calculated Minimum SSB_RP value for the baseline of UE Power class 3 in Band n260 is (-109.5+ΣMBP) dBm/120kHz for intra-frequency measurements and (-107.5+ΣMBP) dBm/120kHz for inter-frequency measurements.

The following methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is used:

For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = -109.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +ΣMBP

For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = -107.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +ΣMBP

Editor’s notes:

- May need to change Power Class and Band terminology (to A, B?) to avoid confusion with Y, Z dB values

- If changed, would also need to align TS 38.101-4 clause 4.5.3.3, and also TR 38.810

##### B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage

Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on EIS Spherical coverage for the Operating band and for the UE Power class, and taking a baseline of UE Power class 3 in Band n260 with 50 MHz channel bandwidth.

Minimum SSB_RP = EIS spherical coverage PC3, n260, 50MHz +Z -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ΣMBS

where:

EIS spherical coverage PC3, n260, 50MHz is the EIS spherical coverage value in dBm specified for Power Class 3 in Band n260 for 50MHz Channel bandwidth in TS 38.101-2 [19] Table 7.3.4.3-1.

Z is the gain difference between fine and rough beams, and is [FFS]dB for a Power Class 3 UE. The value for other Power Classes is defined in Table B.2.1.3.2-1.

Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions

| Value “Z” in dB, for each UE Power class |  |  |  |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| FFS | [9.0] | FFS | FFS |

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [19] Table 5.3.2-1, and is 32.

12 is the number of subcarriers in a PRB.

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1dB.

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6dB for intra-frequency measurements and -4dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise.

ΣMBS is the UE multi-band relaxation factor value in dB specified in TS 38.101-2 [19] clause 6.2.1.

The calculated Minimum SSB_RP value for the baseline of UE Power class 3 in Band n260 is (-103.9+ΣMBS +Z) dBm/120kHz for intra-frequency measurements and is (-101.9+ΣMBS +Z) dBm/120kHz for inter-frequency measurements.

The following methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is used:

For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = (-103.9+ΣMBS +Z) dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Z PC_X – Z PC3 +ΣMBS

For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = (-101.9+ΣMBS +Z) dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Z PC_X – Z PC3 +ΣMBS

Editor’s notes:

- May need to change Power Class and Band terminology (to A, B?) to avoid confusion with Y, Z dB values

- If changed, would also need to align TS 38.101-4 clause 4.5.3.3, and also TR 38.810

## B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | [-113.8] | -112.1 | -127.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | [-113.8] | -112.1 | -127.8+Y4 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | [-113.8] | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | [-102.8] | -108.2+Z3 | -118.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | [-102.8] | -108.2+Z3 | -118.8+Z4 |  |  |
|  |  | n260 | -117.3+Z1 |  | -103.9+Z3 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | [-102.8] | -108.2+Z3 | -118.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.2-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1, 3 and 4 is FFS, where Z1, Z3, and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1, 3 and 4 respectively

## B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | [-111.8] | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | [-111.8] | -110.1 | -125.8+Y4 |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | [-111.8] | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | [-100.8] | -106.2+Z3 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | [-100.8] | -106.2+Z3 | -116.8+Z4 |  |  |
|  |  | n260 | -115.3+Z1 |  | -101.9+Z3 | -111.8+Z4 |  |  |
|  |  | n261 | -118.3+Z1 | [-100.8] | -106.2+Z3 | -116.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.3-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1, 3 and 4 is FFS, where Z1, Z3, and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1, 3 and 4 respectively

## B.2.4	Conditions for NR L1-RSRP reporting

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.1-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

### B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  | SCSCSI-RS = 120 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.2-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

## B.2.5	Conditions for RRC connection release with redirection to NR

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.5-2: Conditions for RRC connection release with redirection to NR in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -114.3 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -114.3 | -110.1 | -125.8+Y4 |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -114.3 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -103.3 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -103.3 | -99.2 | -116.8+Z4 |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -114.3 | -103.3 | -99.2 | -116.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.5.2-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

## B.2.6	Conditions for UE transmit timing

### B.2.6.1	Conditions for SSB based UE transmit timing

This clause defines the following conditions for UE transmit timing adjustment performed based on SSBs: SSB_RP and SSB Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.1-1 for FR1 SSB.

Table B.2.6.1-1: Conditions for SSB based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=15 kHz | SCSSSB=30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | [-3] |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The conditions are defined in Table B.2.6.1-2 for FR2 SSB.

Table B.2.6.1-2: Conditions for SSB based UE transmit timing in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥[-3] |
|  |  | n258 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -113.3 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥[-3] |
|  |  | n258 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -102.3 | -98.2 | -115.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.6.1-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

### B.2.6.2	Conditions for CSI-RS based UE transmit timing

This clause defines the following conditions for UE transmit timing adjustment based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.2-1 for FR1 CSI-RS.

Table B.2.6.2-1: Conditions for CSI-RS based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS RSRP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS=15 kHz | SCSCSI-RS=30 kHz | SCSCSI-RS=60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | [-3] |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

The conditions are defined in Table B.2.6.2-2 for FR2 CSI-RS.

Table B.2.6.2-2: Conditions for CSI-RS based UE transmit timing in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  | SCSCSI-RS = 120 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥[-3] |
|  |  | n258 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -116.3 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥[-3] |
|  |  | n258 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -105.3 | -101.2 | -118.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.6.2-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1, 2 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1, 2, 3 and 4 respectively

# B.3	RRM Requirements Exceptions

## B.3.1	Introduction

Annex B.3 covers exceptions for side conditions based on receiver sensitivity for CA, DC, and SUL.

## B.3.2	Receiver sensitivity relaxation for CA

### B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1

For a UE supporting inter-band carrier aggregation configuration with uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in clause 7.3A.3 of TS 38.101-1 [18], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting CA configuration in FR1, the requirement in this clause applies for both SC and CA operation.

### B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1

#### B.3.2.2.1	Inter-band carrier aggregation

For a UE configured with inter-band carrier aggregation with active uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in clause 7.3A.3 of TS 38.101-1 [18], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

If the relaxation Δ specified in this clause applies, then the relaxation specified in clause B.3.2.1 should not be applied.

#### B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA

In this clause, requirements exceptions are described for the UE configured with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same CA configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in clause 7.3.2 of TS 38.101-1 [18], and L2 is the reference sensitivity level based on the requirements in clause 7.3A.4 of TS 38.101-1 [18], when the following conditions are fulfilled,

-	corresponding downlink component carriers on different NR bands are configured with CA and active,

-	the upling is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in clause 7.3A.4 of TS 38.101-1 [18], and

-	the exception requirements specified in clause 7.3A.4 of TS 38.101-1 [18] apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in clause B.3.2.1 should not be applied.

#### B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

In this clause, requirements exceptions are described for the UE with an inter-band carrier aggregation with uplink assigned to two NR bands.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in clause 7.3.2 of TS 38.101-1 [18], and L2 is the reference sensitivity level based on the requirements in clause 7.3A.5 of TS 38.101-1 [18], when the following conditions are fulfilled,

-	corresponding downlink component carriers on different bands are configured with CA and active,

-	uplinks are assigned to two NR bands,

-	the exception requirements specified in clause 7.3A.5 of TS 38.101-1 [18] apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in clause B.3.2.1 should not be applied.

### B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2

Editor’s note: TBD

### B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2

#### B.3.2.4.1	Intra-band contiguous carrier aggregation

For a UE configured with intra-band contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in clause 7.3A.2.1 of TS 38.101-2 [19] depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

#### B.3.2.4.2	Intra-band non-contiguous carrier aggregation

For a UE configured with intra-band non-contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in clause 7.3A.2.1 of TS 38.101-2 [19] depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

## B.3.3	Receiver sensitivity relaxation for DC

Editor’s note: TBD

## B.3.4	Receiver sensitivity relaxation for SUL

### B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1

For a UE supporting a SUL configuration in FR1, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in clause 7.3C.3 of TS 38.101-1 [18], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting a SUL configuration in FR1, the requirement in this clause applies for both SC and SUL operation.

### B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1

#### B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL

In this clause, requirements exceptions are described for the UE with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same SUL configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in clause 7.3.2 of TS 38.101-1 [18], and L2 is the reference sensitivity level based on the requirements in clause 7.3C.2 of TS 38.101-1 [18], when the following conditions are fulfilled,

-	a downlink component carrier is configured in NR band and is active,

-	the upling is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in clause 7.3C.2 of TS 38.101-1 [18], and

-	the exception requirements specified in clause 7.3C.2 of TS 38.101-1 [18] apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in clause B.3.4.1 should not be applied.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-05 | RAN4#83 | R4-1706324 |  |  |  | Specification skeleton | 0.0.1 |
| 2017-09 |  |  |  |  |  | Email approved | 0.1.0 |
| 2017-09 | RAN4-NR AH #3 | R4-1709413 |  |  |  | Capture TPs approved in the meeting | 0.2.0 |
| 2017-10 | RAN4#84-Bis | R4-1711985 |  |  |  | Capture TPs approved in the meeting | 0.3.0 |
| 2017-12 | RAN4#85 | R4-1714546 |  |  |  | Capture TPs approved in RAN4#85 | 0.4.0 |
| 2017-12 | RAN#78 | RP-172407 |  |  |  | v1.0.0 submitted for plenary approval | 1.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2018-03 | RAN#79 | RP-180264 | 0032 |  | B | CR to TS38.133 | 15.1.0 |
| 2018-06 | RAN#80 | RP-181075 | 0037 |  | B | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4 #86bis and RAN4 #87 | 15.2.0 |
| 2018-09 | RAN#81 | RP-181896 | 0043 |  | B | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4-AH-1807 and RAN4 #88 | 15.3.0 |
| 2018-12 | RAN#82 | RP-182763 | 0057 | 3 | B | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4-88bis and RAN4-89 | 15.4.0 |
| 2019-03 | RAN#83 | RP-190569 | 0064 | 1 | B | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4#90 | 15.5.0 |
| 2019-06 | RAN#84 | RP-191240 | 0072 | 1 | F | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4#90bis and RAN4#91 | 15.6.0 |
| 2019-06 | RAN#84 | RP-191248 | 0066 |  | B | Introduction of band n48 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191242 | 0067 |  | B | Introduction of band n14 - CR to TS 38.133 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191246 | 0068 |  | B | Introduction of band n30 - CR to TS 38.133 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191244 | 0069 |  | B | introduce n18 into TS38.133 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191250 | 0070 | 1 | B | n65 introduction to 38.133 | 16.0.0 |