3GPP TS 36.133 V15.9.0 (2019-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	87

1	Scope	88

2	References	88

3	Definitions, symbols and abbreviations	90

3.1	Definitions	90

3.2	Symbols	92

3.3	Abbreviations	93

3.4	Test tolerances	95

3.5	Additional notation	96

3.5.1	Groups of bands	96

3.6	General	98

3.6.1	Applicability of requirements in this specification version	98

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	105

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	106

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	107

3.6.2	Applicability of requirements for EN-DC operation	108

3.6.3	Applicability of requirements for NE-DC operation	109

3.6.4	Applicability of requirements for NGEN-DC operation	110

4	E-UTRAN RRC_IDLE state mobility	110

4.1	Cell Selection	110

4.2	Cell Re-selection	110

4.2.1	Introduction	110

4.2.2	Requirements	111

4.2.2.1	Measurement and evaluation of serving cell	112

4.2.2.2	Void	112

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	112

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	114

4.2.2.5	Measurements of inter-RAT cells	116

4.2.2.5.1	Measurements of UTRAN FDD cells	117

4.2.2.5.2	Measurements of UTRAN TDD cells	118

4.2.2.5.3	Measurements of GSM cells	119

4.2.2.5.4	Measurements of HRPD cells	120

4.2.2.5.5	Measurements of cdma2000 1X	121

4.2.2.5.6	Measurements of NR cells	122

4.2.2.6	Evaluation of cell re-selection criteria	123

4.2.2.7	Maximum interruption in paging reception	124

4.2.2.8	void	124

4.2.2.9	UE measurement capability	124

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	125

4.2.2.10	Reselection to CSG cells	125

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	126

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	126

4.2.2.11	Void	127

4.2.2.12	Void	127

4.2.2.13	Void	127

4.3	Minimization of Drive Tests (MDT)	127

4.3.1	Introduction	128

4.3.2	Measurements	128

4.3.2.1	Requirements	128

4.3.3	Relative Time Stamp Accuracy	128

4.3.3.1	Requirements	128

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	128

4.3.4.1	 Requirements	128

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	129

4.3.5.1	Requirements for timeSinceFailure	129

4.4	MBSFN Measurements	129

4.4.1	Introduction	129

4.4.2	MBSFN RSRP measurements	129

4.4.3	MBSFN RSRQ measurements	129

4.4.4	MCH BLER measurements	130

4.5	Proximity-based Services	130

4.5.1	Introduction	130

4.5.2	Requirements	130

4.5.2.1	Interruptions with ProSe Direct Discovery	130

4.5.2.2	Interruptions with ProSe Direct Communication	130

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	130

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	131

4.6	Cell Selection and Re-selection Requirements for UE category NB1	131

4.6.1	Cell Selection	131

4.6.2	Cell Re-selection	131

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	132

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	132

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	133

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	134

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	135

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	136

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	138

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	139

4.6.2.7	Maximum interruption in paging reception in normal coverage	140

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	140

4.6.2.8	UE measurement capability	140

4.6.2.9	WUS receptions for NB1	141

4.7	Cell Selection and Re-selection Requirements for UE category M1	141

4.7.1	Cell Selection	141

4.7.2	Cell Re-selection	141

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	141

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	141

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	142

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	143

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	145

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	145

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	145

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	145

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	146

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	148

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	149

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	150

4.7.2.3	WUS receptions for UE category M1	150

4.8	Idle State Positioning Measurement Requirements for UE category NB1	150

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	150

4.8.1.1	RSTD Measurement Reporting Delay	152

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	152

4.8.2.1	RSTD Measurement Reporting Delay	154

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	154

4.8.3.1	RSTD Measurement Reporting Delay	155

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	156

4.8.4.1	RSTD Measurement Reporting Delay	157

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	158

4.8.5.1	Measurement Reporting Delay	159

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	159

4.8.6.1	Measurement Reporting Delay	160

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	160

4.8.7.1	Measurement Reporting Delay	161

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	162

4.8.8.1	Measurement Reporting Delay	163

4.9	Idle Mode CA Measurement	164

4.9.1	Introduction	164

4.9.2	Requirement	164

4.9.2.1	Detected cell requirement during state transition and Idle mode	164

4.9.2.2	Measurements of inter-frequency CA candidate cells	164

4.9.2.3	Measurements on serving cell	165

4A	E-UTRAN RRC_INACTIVE state mobility	165

4A.1	Cell Re-selection	165

4A.1.1	Introduction	165

4A.1.2	Requirements	165

4A.1.2.1	UE measurement capability	165

4A.1.2.2	Measurement and evaluation of serving cell	165

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	165

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	165

4A.1.2.5	Evaluation of cell re-selection criteria	166

4A.1.2.6	Maximum interruption in paging reception	166

4A.1.2.7	Measurements of inter-RAT NR cells	166

5	E-UTRAN RRC_CONNECTED state mobility	166

5.1	E-UTRAN Handover	166

5.1.1	Introduction	166

5.1.2	Requirements	166

5.1.2.1	E-UTRAN FDD – FDD	166

5.1.2.1.1	Handover delay	166

5.1.2.1.2	Interruption time	167

5.1.2.2	E-UTRAN FDD – TDD	168

5.1.2.2.1	(Void)	168

5.1.2.2.2	(Void)	168

5.1.2.3	E-UTRAN TDD – FDD	168

5.1.2.3.1	(Void)	168

5.1.2.3.2	(Void)	168

5.1.2.4	E-UTRAN TDD – TDD	168

5.1.2.4.1	Handover delay	168

5.1.2.4.2	Interruption time	169

5.1.2.5	E-UTRAN HD–FDD	170

5.1.2.5.1	Handover delay	170

5.1.2.5.2	Interruption time	171

5.2	Void	172

5.3	Handover to other RATs	172

5.3.1	E-UTRAN - UTRAN FDD Handover	172

5.3.1.1	Introduction	172

5.3.1.1.1	Handover delay	172

5.3.1.1.2	Interruption time	172

5.3.2	E-UTRAN - UTRAN TDD Handover	173

5.3.2.1	Introduction	173

5.3.2.2	Requirements	173

5.3.2.2.1	Handover delay	173

5.3.2.2.2	Interruption time	173

5.3.3	E-UTRAN - GSM Handover	174

5.3.3.1	Introduction	174

5.3.3.2	Requirements	174

5.3.3.2.1	Handover delay	174

5.3.3.2.2	Interruption time	174

5.3.4	E-UTRAN - NR FR1 Handover	175

5.3.4.1	Introduction	175

5.3.4.2	Handover delay	175

5.3.4.3	Interruption time	175

5.3.5	E-UTRAN - NR FR2 Handover	175

5.3.5.1	Introduction	175

5.3.5.2	Handover delay	176

5.3.5.3	Interruption time	176

5.4	Handover to Non-3GPP RATs	177

5.4.1	E-UTRAN – HRPD Handover	177

5.4.1.1	Introduction	177

5.4.1.1.1	Handover delay	177

5.4.1.1.2	Interruption time	177

5.4.2	E-UTRAN – cdma2000 1X Handover	177

5.4.2.1	Introduction	177

5.4.2.1.1	Handover delay	177

5.4.2.1.2	Interruption time	178

5.5	E-UTRAN Handover for Cat-M1 UEs	178

5.5.1	Introduction	178

5.5.2	Requirements in CEModeA	178

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	178

5.5.2.1.1	Handover delay	178

5.5.2.1.2	Interruption time	178

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	179

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	179

5.5.2.3.1	Void	179

5.5.2.3.2	Void	179

5.5.3	Requirements in CEModeB	179

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	179

5.5.3.1.1	Handover delay	179

5.5.3.1.2	Interruption time	180

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	180

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	180

5.6	Void	180

6	RRC Connection Mobility Control	180

6.1	RRC Re-establishment	180

6.1.1	Introduction	181

6.1.2	Requirements	181

6.1.2.1	UE Re-establishment delay requirement	181

6.2	Random Access	181

6.2.1	Introduction	181

6.2.2	Requirements	181

6.2.2.1	Contention based random access	182

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	182

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	182

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	182

6.2.2.1.4	Void	182

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	182

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	182

6.2.2.2	Non-Contention based random access	182

6.2.2.2.1	Correct behaviour when receiving Random Access Response	182

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	182

6.2.3	Requirements for Cat-M1 UEs	183

6.3	RRC Connection Release with Redirection	183

6.3.1	Introduction	183

6.3.2	Requirements	183

6.3.2.1	RRC connection release with redirection to UTRAN FDD	183

6.3.2.2	RRC connection release with redirection to GERAN	183

6.3.2.3	RRC connection release with redirection to UTRAN TDD	184

6.3.2.4	RRC connection release with redirection to NR	184

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	185

6.4.1	Introduction	185

6.4.2	Requirements	185

6.5	RRC Re-establishment for NB-IoT UEs	185

6.5.1	Introduction	185

6.5.2	Requirements	186

6.5.2.1	UE Re-establishment delay requirement in normal coverage	186

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	186

6.6	Random Access for UE category NB1	187

6.6.1	Introduction	187

6.6.2	Requirements	187

6.6.2.1	Correct behaviour when receiving Random Access Response reception	187

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	187

6.6.2.3	Correct behaviour when receiving a NACK on msg3	187

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	187

6.6.2.5	Correct behaviour when contention Resolution timer expires	188

6.6.2.6	MSG3-based channel quality report for UE Category NB1	188

6.6.3	Requirements for NPRACH configuration	188

6.7	RRC Re-establishment for Cat-M1 UEs	188

6.7.1	Introduction	188

6.7.2	Requirements	189

6.7.2.1	UE Re-establishment delay requirement for CEModeA	189

6.7.2.2	UE Re-establishment delay requirement for CEModeB	189

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	190

6.8.1	Introduction	190

6.8.2	Requirements	190

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	190

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	191

6.9.1	Introduction	191

6.9.2	Requirements	191

7	Timing and signalling characteristics	191

7.1	UE transmit timing	191

7.1.1	Introduction	191

7.1.2	Requirements	192

7.2	UE timer accuracy	193

7.2.1	Introduction	193

7.2.2	Requirements	193

7.3	Timing Advance	193

7.3.1	Introduction	193

7.3.2	Requirements	193

7.3.2.1	Timing Advance adjustment delay	193

7.3.2.2	Timing Advance adjustment accuracy	194

7.4	Cell phase synchronization accuracy (TDD)	194

7.4.1	Definition	194

7.4.2	Minimum requirements	194

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	195

7.5.1	Introduction	195

7.5.2	eNodeB Synchronization Requirements	195

7.5.2.1	Synchronized E-UTRAN	195

7.5.2.2	Non-Synchronized E-UTRAN	195

7.6	Radio Link Monitoring	195

7.6.1	Introduction	195

7.6.2	Requirements	197

7.6.2.1	Minimum requirement when no DRX is used	197

7.6.2.2	Minimum requirement when DRX is used	198

7.6.2.3	Minimum requirement at transitions	199

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	199

7.6.2.5	Minimum requirement under IDC Interference	200

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	200

7.7.1	Introduction	200

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	200

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	201

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	202

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	204

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	205

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	205

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	206

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	206

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	206

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	208

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	208

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	209

7.7.14	SCell Activation Delay Requirement for Dormant SCell	210

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	211

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	212

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	213

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	213

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	215

7.8	Interruptions with Carrier Aggregation	217

7.8.1	Introduction	217

7.8.2	Requirements	218

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	218

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	218

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	218

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	218

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	218

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	218

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	219

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	219

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	219

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	220

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	220

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	221

7.8.2.13	Interruptions at SRS carrier based switching	221

7.8.2.14	Interruptions at SCell activation of dormant SCell for intra-band CA	222

7.8.2.15	Interruptions at SCell activation of dormant SCell for inter-band CA	222

7.8.2.16	Interruptions at SCell activation of multiple dormant SCells	222

7.8.2.17	Interruptions during CQI measurement on dormant SCell	223

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	223

7.9.1	Introduction	223

7.9.2	Minimum Requirements for Interband Carrier Aggregation	223

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	223

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	224

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	224

7.10.1	Introduction	224

7.10.2	Requirements	224

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	224

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	224

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	225

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	225

7.11	Radio Link Monitoring for UE Category 0	225

7.11.1	Introduction	225

7.11.2	Requirements for FD-FDD and TDD	227

7.11.2.1	Minimum requirement when no DRX is used	227

7.11.2.2	Minimum requirement when DRX is used	227

7.11.2.3	Minimum requirement at transitions	228

7.11.3	Requirements for HD-FDD	228

7.11.3.1	Minimum requirement when no DRX is used	228

7.11.3.2	Minimum requirement when DRX is used	228

7.11.3.3	Minimum requirement at transitions	229

7.12	Interruptions with Dual Connectivity	229

7.12.1	Introduction	229

7.12.2	Requirements	229

7.12.2.1	Interruptions at PSCell addition/release	229

7.12.2.2	Interruptions at transitions between active and non-active during DRX	230

7.12.2.3	Interruptions at transitions from non-DRX to DRX	230

7.12.2.4	Interruptions at SCell addition/release	230

7.12.2.5	Interruptions at SCell activation/deactivation	230

7.12.2.6	Interruptions during measurements on SCC	231

7.12.2.7	Interruptions at SRS carrier based switching	231

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	232

7.13.1	Definition	232

7.13.2	Minimum requirements	232

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	232

7.14.1	Introduction	232

7.14.2	PSCell Addition Delay Requirement	232

7.14.3	PSCell Release Delay Requirement	233

7.15	Maximum Receive Timing Difference in Dual Connectivity	233

7.15.1	Introduction	233

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	233

7.16	Proximity-based Services	234

7.16.1	Introduction	234

7.16.2	Requirements	234

7.16.2.1	ProSe UE transmission timing	234

7.16.2.1.1	Serving cell or PCell as timing reference	234

7.16.2.1.2	SCell or non-serving cell as timing reference	234

7.16.3	Interruptions with ProSe	234

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	235

7.16.3.2	Interruptions at ProSe Direct Communication configuration	235

7.16.3.3	Interruptions during ProSe Direct Discovery	235

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	235

7.16.3.5	Interruptions during ProSe Direct Communication	236

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	236

7.16.4.1	Measurement and evaluation of selected cell	236

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	236

7.16.5	Selection / Reselection of ProSe relay UE	237

7.16.6	ProSe operation under deactivated SCell	237

7.17	Maximum Transmission Timing Difference in Dual Connectivity	238

7.17.1	Introduction	238

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	238

7.18.1	Introduction	238

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	238

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	238

7.19	Radio Link Monitoring for UE Category M1	238

7.19.1	Introduction	238

7.19.2	Requirements for FD-FDD and TDD CE mode A	239

7.19.2.1	Minimum requirement when no DRX is used	240

7.19.2.2	Minimum requirement when DRX is used	241

7.19.2.3	Minimum requirement at transitions	242

7.19.3	Requirements for HD-FDD with CE mode A	242

7.19.3.1	Minimum requirement when no DRX is used	242

7.19.3.2	Minimum requirement when DRX is used	242

7.19.3.3	Minimum requirement at transitions	244

7.19.4	Requirements for FD-FDD and TDD with CE mode B	244

7.19.4.1	Minimum requirement when no DRX is used	245

7.19.4.2	Minimum requirement when DRX is used	246

7.19.4.3	Minimum requirement at transitions	247

7.19.5	Requirements for HD-FDD with CE mode B	247

7.19.5.1	Minimum requirement when no DRX is used	247

7.19.5.2	Minimum requirement when DRX is used	247

7.19.5.3	Minimum requirement at transitions	249

7.20	UE transmit timing for NB-IoT	249

7.20.1	Introduction	249

7.20.2	Requirements	249

7.21	 UE timer accuracy for NB-IoT	250

7.21.1	Introduction	250

7.21.2	Requirements	250

7.22	Timing Advance for NB-IoT	250

7.22.1	Introduction	250

7.22.2	Requirements	250

7.22.2.1	Timing Advance adjustment delay	250

7.22.2.2	Timing Advance adjustment accuracy	250

7.23	Radio Link Monitoring for Category NB1 UE	250

7.23.1	Introduction	250

7.23.2	Requirements for Category NB1 UE	250

7.23.2.1	Minimum requirement when no DRX is used	251

7.23.2.2	Minimum requirement when DRX is used	251

7.23.2.3	Minimum requirement at transitions	252

7.24	UE transmit timing for Category M1	252

7.24.1	Introduction	252

7.24.2	Requirements	252

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	253

7.25.1	Definition	253

7.25.2	Minimum requirements	253

7.26	UE transmit timing for Category M2	254

7.26.1	Introduction	254

7.26.2	Requirements	254

7.27	UE timer accuracy for category M1	254

7.27.1	Introduction	254

7.27.2	Requirements	254

7.28	Timing Advance for Category M1	254

7.28.1	Introduction	254

7.28.2	Requirements	254

7.29	Interruptions requirements with FeMBMS	254

7.29.1	Introduction	254

7.29.2	Requirements	255

7.30	Numerology switching delay requirements with FeMBMS	255

7.30.1	Introduction	255

7.30.2	Requirements	255

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	255

7.31.1	Introduction	255

7.31.2	NR PSCell Addition Delay Requirement	255

7.31.3	NR PSCell Release Delay Requirement	256

7.32	Interruptions with EN-DC	256

7.32.1	Introduction	256

7.32.2	Requirements	257

7.32.2.1	Interruptions at PSCell addition/release	257

7.32.2.2	Interruptions at transitions between active and non-active during DRX	257

7.32.2.3	Interruptions at transitions from non-DRX to DRX	257

7.32.2.4	Interruptions at SCell addition/release	257

7.32.2.5	Interruptions at SCell activation/deactivation	257

7.32.2.6	Interruptions during measurements on SCC	258

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	258

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	258

7.32.2.7	Interruptions at active BWP switching	258

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	259

7.33.1	Introduction	259

7.33.2	Requirements	259

7.34	Void	259

7.35	Interruptions with SFTD measurements	259

7.35.1	Introduction	259

7.35.2	Requirements	259

7.36	Interruptions with NE-DC	260

7.32.1	Introduction	260

7.36.2	Requirements	260

7.36.2.1	Interruptions at transitions between active and non-active during DRX	260

7.36.2.2	Interruptions at transitions from non-DRX to DRX	260

7.36.2.3	Interruptions at SCell addition/release	260

7.36.2.4	Interruptions at SCell activation/deactivation	261

7.36.2.5	Interruptions during measurements on SCC	261

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	261

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	261

7.36.2.6	Interruptions at active BWP switching	261

8	UE Measurements Procedures in RRC_CONNECTED State	262

8.1	General Measurement Requirements	262

8.1.1	Introduction	262

8.1.2	Requirements	263

8.1.2.1	UE measurement capability	263

8.1.2.1.1	Monitoring of multiple layers using gaps	270

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	271

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	272

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	274

8.1.2.1.2	Network controlled small gap	274

8.1.2.2	E-UTRAN intra frequency measurements	276

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	276

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	281

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	285

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	286

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	287

8.1.2.3	E-UTRAN inter frequency measurements	287

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	287

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	293

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	300

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	300

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	300

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	301

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	303

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	304

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	305

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	311

8.1.2.4	Inter RAT measurements	311

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	311

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	316

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	316

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	320

8.1.2.4.5	E-UTRAN FDD – GSM measurements	320

8.1.2.4.6	E-UTRAN TDD – GSM measurements	325

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	325

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	327

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	327

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	327

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	328

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	328

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	328

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	328

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	330

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	330

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	330

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	330

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	331

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	331

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	333

8.1.2.4.21	E-UTRAN FDD – NR measurements	333

8.1.2.4.22	E-UTRAN TDD – NR measurements	336

8.1.2.4.23	Void	336

8.1.2.4.24	Void	336

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	336

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	338

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	338

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	338

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	340

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	342

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	343

8.1.2.6.5	Void	345

8.1.2.6.6	Void	345

8.1.2.6.7	Void	345

8.1.2.6.8	Void	345

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	345

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	345

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	347

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	349

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	350

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	352

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	354

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	355

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	357

8.1.2.7	E-UTRAN E-CID Measurements	359

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	359

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	360

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	362

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	362

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	363

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	363

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	366

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	369

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	372

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	376

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	376

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	376

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	376

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	377

8.1.2.10	Void	377

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	377

8.2.1	Introduction	377

8.2.2	Requirements	378

8.3	Measurements for E-UTRA carrier aggregation	382

8.3.1	Introduction	382

8.3.2	Measurements of the primary component carrier	382

8.3.3	Measurements of a secondary component carrier	382

8.3.3.1	Measurements of a secondary component carrier with active SCell	382

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	383

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	383

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	384

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	385

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	386

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	386

8.4.1	Introduction	386

8.4.2	Measurements on the primary component carrier	386

8.4.3	Measurements on a secondary component carrier	387

8.4.4	Measurements on both primary component carrier and a secondary component carrier	388

8.4.5	Measurements on different secondary component carriers	389

8.5	Measurements for UE category 0	390

8.5.1	Introduction	390

8.5.2 	Requirements	390

8.5.2.1 	E-UTRAN intra frequency measurements	390

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	390

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	393

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	395

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	399

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	399

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	400

8.6	Discovery signal measurements	401

8.6.1	Introduction	401

8.6.2	Requirements for CRS based discovery signal measurements	401

8.6.2.1	E-UTRAN intra frequency measurements	401

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	401

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	404

8.6.2.2	E-UTRAN inter frequency measurements	406

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	407

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	409

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	412

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	412

8.6.3	Requirements for CSI-RS based discovery signal measurements	412

8.6.3.1	E-UTRAN intra frequency measurements	412

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	412

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	415

8.6.3.2	E-UTRAN inter frequency measurements	417

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	417

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	420

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	423

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	423

8.7	Discovery signal measurements for E-UTRA carrier aggregation	423

8.7.1	Introduction	423

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	423

8.7.2.1	Measurements of the primary component carrier	423

8.7.2.2	Measurements of a secondary component carrier	423

8.7.2.3	Measurements of a secondary component carrier with active SCell	423

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	424

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	424

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	425

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	427

8.7.3.1	Measurements of the primary component carrier	427

8.7.3.2	Measurements of a secondary component carrier	427

8.7.3.3	Measurements of a secondary component carrier with active SCell	427

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	427

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	427

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	428

8.8	Measurements for E-UTRA dual connectivity	430

8.8.1	Introduction	430

8.8.2	Intra-frequency measurements requirements on PCell	430

8.8.3	Intra-frequency measurements requirements on PSCell	430

8.8.4	Inter-frequency and inter-RAT measurement requirements	430

8.8.5	Intra-frequency measurements with autonomous gaps	431

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	431

8.8.5.2	ECGI reporting delay	432

8.8.6	Inter-frequency measurements with autonomous gaps	432

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	432

8.8.6.2	ECGI reporting delay	433

8.8.7	SSTD Measurements	433

8.8.7.1	Introduction	433

8.8.7.2	SSTD Measurement requirements	433

8.8.7.3	SSTD Measurement Reporting Delay	433

8.8.8	Intra-frequency measurements requirements on SCell	434

8.9	MBSFN Measurements	434

8.9.1	Introduction	434

8.9.2	MBSFN RSRP Measurements	434

8.9.3	MBSFN RSRQ Measurements	434

8.9.4	MCH BLER Measurements	435

8.10	Proximity-based Services	435

8.10.1	Introduction	435

8.10.2	Requirements	435

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	435

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	435

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	436

8.11.1	Introduction	436

8.11.2	CRS based discovery signal measurements	436

8.11.2.1	E-UTRAN intra-frequency measurements	436

8.11.2.1.1	Requirements	437

8.11.2.1.1.1	Requirements when no DRX is used	437

8.11.2.1.1.1.1	Measurement Reporting Requirements	438

8.11.2.1.1.2	Requirements when DRX is used	439

8.11.2.1.1.2.1	Measurement Reporting Requirements	441

8.11.2.2	E-UTRAN inter-frequency measurements	442

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	442

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	446

8.11.3	CSI-RS based discovery signal measurements	446

8.11.3.1	E-UTRAN intra-frequency measurements	446

8.11.3.1.1	Requirements	446

8.11.3.1.1.1	Requirements when no DRX is used	446

8.11.3.1.1.1.1	Measurement Reporting Requirements	448

8.11.3.1.1.2	Requirements when DRX is used	448

8.11.3.1.1.2.1	Measurement Reporting Requirements	449

8.11.3.2	E-UTRAN inter-frequency measurements	450

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	450

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	454

8.11.4	RSSI measurements	454

8.11.4.1	E-UTRAN intra-frequency measurements	454

8.11.4.2	E-UTRAN inter-frequency measurements	454

8.11.5	Channel occupancy measurements	454

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	454

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	455

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	455

8.12.1	Introduction	455

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	455

8.12.2.1	Introduction	455

8.12.2.2	Measurements of a secondary component carrier	455

8.12.2.3	Measurements of a secondary component carrier with active SCell	455

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	455

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	456

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	458

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	461

8.12.3.1	Introduction	461

8.12.3.2	Measurements of a secondary component carrier	461

8.12.3.3	Measurements of a secondary component carrier with active SCell	461

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	461

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	461

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	463

8.13	Measurements for UE Category M1	465

8.13.1	Introduction	465

8.13.2	Requirements for UE category M1 with CE mode A	465

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	466

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	466

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	470

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	472

8.13.2.2	Void	476

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	476

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	476

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	479

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	481

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	482

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	482

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	484

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	486

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	487

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	487

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	487

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	488

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	488

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	488

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	489

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	489

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	490

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	491

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	491

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	492

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	496

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	498

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	502

8.13.3	Requirements for UE category M1 with CE mode B	502

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	502

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	502

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	507

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	509

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	515

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	516

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	516

8.13.3.2	Void	517

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	517

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	517

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	520

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	522

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	523

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	523

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	523

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	523

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	524

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	524

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	525

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	525

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	525

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	529

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	532

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	536

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	537

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	537

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	539

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	541

8.14	Measurements for UE category NB1	542

8.14.1	Introduction	542

8.14.2	NB-IoT intra frequency measurements under normal coverage	542

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	542

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	542

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	542

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	542

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	542

8.15	Void	543

8.16	Measurements for UE Category M2	543

8.16.1	Introduction	543

8.16.2	Requirements for UE category M2 with CE mode A	543

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	543

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	544

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	544

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	545

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	545

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	545

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	546

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	546

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	548

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	551

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	551

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	551

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	553

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	556

8.16.3	Requirements for UE category M2 with CE mode B	556

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	556

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	556

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	559

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	562

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	562

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	562

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	564

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	567

8.17	Measurements for E-UTRA – NR Dual Connectivity	567

8.17.1	Introduction	567

8.17.1.1	Measurement Gap Sharing	567

8.17.1A	Intrafrequency Measurements	568

8.17.2	SFTD Measurements	568

8.17.2.1	Introduction	568

8.17.2.2	SFTD Measurement requirements	568

8.17.2.3	SFTD Measurement Reporting Delay	569

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	569

8.17.3.1	Introduction	569

8.17.3.2	E-UTRAN FDD inter frequency measurements	569

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	569

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	571

8.17.3.3	E-UTRAN TDD inter frequency measurements	573

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	573

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	574

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	576

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	576

8.17.4.1.1	NR Inter-RAT cell identification	577

8.17.4.1.2	NR Inter-RAT measurement	578

8.17.4.1.3	NR Inter-RAT measurement reporting	579

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	579

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	580

8.17.5.1	Introduction	580

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	580

8.17.5.2.1	Identification of a new UTRA FDD cell	580

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	580

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	580

8.17.5.2.4	Periodic Reporting	581

8.17.5.2.5	Event Triggered Reporting	581

8.17.5.2.6	Event-triggered Periodic Reporting	581

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	582

8.17.5.3.1	Periodic Reporting	583

8.17.5.3.2	Event Triggered Reporting	583

8.17.5.3.3	Event-triggered Periodic Reporting	583

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	584

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	584

8.17.7.1	Introduction	584

8.17.7.2	Identification of a new UTRA FDD cell for SON	584

8.17.7.2.1	Requirements when no DRX is used	584

8.17.7.2.2	Requirements when DRX is used	584

8.17.7.2.3	Reporting Delay	585

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	586

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	586

8.17.9.1	Introduction	586

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	586

8.17.9.2.1	Identification of a new UTRA FDD cell	586

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	586

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	586

8.17.9.2.4	Periodic Reporting	587

8.17.9.2.5	Event Triggered Reporting	587

8.17.9.2.6	Event-triggered Periodic Reporting	587

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	588

8.17.9.3.1	Periodic Reporting	589

8.17.9.3.2	Event Triggered Reporting	589

8.17.9.3.3	Event-triggered Periodic Reporting	589

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	589

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	589

8.17.11.1	Introduction	589

8.17.11.2	Identification of a new UTRA TDD cell for SON	590

8.17.11.2.1	Requirements when no DRX is used	590

8.17.11.2.2	Requirements when DRX is used	590

8.17.11.2.3	Reporting Delay	591

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	591

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	591

8.17.13.1	Introduction	591

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	591

8.17.13.2.1	GSM carrier RSSI	591

8.17.13.2.2	BSIC verification	592

8.17.13.2.3	Enhanced BSIC verification	593

8.17.13.2.4	Periodic Reporting	594

8.17.13.2.5	Event Triggered Reporting	594

8.17.13.2.6	Event-triggered Periodic Reporting	594

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	594

8.17.13.3.1	GSM carrier RSSI	594

8.17.13.3.2	BSIC verification	595

8.17.13.3.3	Periodic Reporting	596

8.17.13.3.4	Event Triggered Reporting	596

8.17.13.3.5	Event-triggered Periodic Reporting	596

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	597

8.18	Measurements for non-BL/CE UE	597

8.18.1	Introduction	597

8.18.2	Requirements for non-BL/CE UE with CE Mode B	597

8.18.2.1	E-UTRAN intra frequency measurements	597

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	597

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	598

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	598

8.19	Measurements for NR – E-UTRA Dual Connectivity	599

8.19.1	Introduction	599

8.19.2	Intra-frequency Measurements	599

8.19.3	Inter-frequency Measurements	599

8.19.4	Intra-frequency RSTD Measurements	600

8.19.5	Intra-frequency E-CID Measurements	600

9	Measurements performance requirements for UE	600

9.1	E-UTRAN measurements	600

9.1.1	Introduction	600

9.1.2	Intra-frequency RSRP Accuracy Requirements	601

9.1.2.1	Absolute RSRP Accuracy	601

9.1.2.2	Relative Accuracy of RSRP	601

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	602

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	603

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	604

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	605

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	606

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	607

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	608

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	608

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	608

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	609

9.1.2B.1	Introduction	609

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	609

9.1.3	Inter-frequency RSRP Accuracy Requirements	610

9.1.3.1	Absolute RSRP Accuracy	610

9.1.3.2	Relative Accuracy of RSRP	611

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	612

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	612

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	613

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	613

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	614

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	615

9.1.3B.1	Introduction	615

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	615

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	615

9.1.4	RSRP Measurement Report Mapping	616

9.1.5	Intra-frequency RSRQ Accuracy Requirements	616

9.1.5.1	Absolute RSRQ Accuracy	616

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	617

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	618

9.1.5.4	Absolute WB-RSRQ Accuracy	619

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	620

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	621

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	621

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	621

9.1.5B.1	Introduction	621

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	621

9.1.6	Inter-frequency RSRQ Accuracy Requirements	622

9.1.6.1	Absolute RSRQ Accuracy	622

9.1.6.2	Relative Accuracy of RSRQ	623

9.1.6.3	Absolute WB-RSRQ Accuracy	624

9.1.6.4	Relative WB-RSRQ Accuracy	624

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	625

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	626

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	626

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	626

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	627

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	628

9.1.6B.1	Introduction	628

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	628

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	628

9.1.7	RSRQ Measurement Report Mapping	629

9.1.8	Power Headroom	629

9.1.8.1	Period	630

9.1.8.2	Reporting Delay	630

9.1.8.3	Void	630

9.1.8.4	Report Mapping	630

9.1.9	UE Rx – Tx time difference	630

9.1.9.1	Measurement Requirement	630

9.1.9.2	Measurement Report mapping	631

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	632

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	633

9.1.10	Reference Signal Time Difference (RSTD)	634

9.1.10.1	Intra-Frequency Accuracy Requirement	634

9.1.10.2	Inter-Frequency Accuracy Requirement	635

9.1.10.3	RSTD Measurement Report Mapping	637

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	637

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	638

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	639

9.1.11	Carrier aggregation measurement accuracy	640

9.1.11.1	Primary component carrier accuracy requirement	641

9.1.11.2	Secondary component carrier accuracy requirement	641

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	641

9.1.11.4	Secondary component carrier relative accuracy requirement	641

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	641

9.1.13	Measurement accuracy for UE category 0	642

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	642

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	642

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	643

9.1.14	Accuracy requirements for Discovery Signal Measurements	644

9.1.14.1	Introduction	644

9.1.14.2	RSRP measurements in discovery signal occasions	644

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	644

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	644

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	644

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	645

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	646

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	646

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	646

9.1.14.3.3	CSI-RSRP measurement report mapping	647

9.1.14.4	RSRQ measurements in discovery signal occasions	647

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	647

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	648

9.1.15.1.1	Primary component carrier accuracy requirement	648

9.1.15.1.2	Secondary component carrier accuracy requirement	648

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	648

9.1.15.1.4	Secondary component carrier relative accuracy requirement	648

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	648

9.1.15.2.1	Primary component carrier accuracy requirement	648

9.1.15.2.2	Secondary component carrier accuracy requirement	648

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	648

9.1.15.2.4	Secondary component carrier relative accuracy requirement	648

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	649

9.1.17	RS-SINR Measurements	649

9.1.17.1	Measurement Report Mapping	649

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	650

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	650

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	650

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	650

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	651

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	652

9.1.18.1	Introduction	652

9.1.18.2	RSRP measurements	652

9.1.18.2.1	RSRP measurement report mapping	652

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	652

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	653

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	653

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	654

9.1.18.3	RSRQ measurements	654

9.1.18.3.1	RSRQ measurement report mapping	654

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	654

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	655

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	655

9.1.18.4	CSI-RSRP measurements	656

9.1.18.4.1	CSI-RSRP measurement report mapping	656

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	656

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	656

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	657

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	657

9.1.18.5	RSSI measurements	658

9.1.18.5.1	RSSI measurement report mapping	658

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	658

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	659

9.1.18.6	Channel occupancy measurements	659

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	659

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	659

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	660

9.1.19.1	Introduction	660

9.1.19.2	Accuracy requirements for measurements on SCC	660

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	660

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	660

9.1.20	SFN and Subframe Time Difference (SSTD)	661

9.1.20.1	SSTD Accuracy Requirement	661

9.1.20.2	SSTD Measurement Report Mapping	661

9.1.21	Measurement accuracy for UE category M1	662

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	662

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	663

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	664

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	665

9.1.21.5	RSRP Measurement Report Mapping	666

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	666

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	667

9.1.21.8	RSRQ Measurement Report Mapping	668

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	668

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	669

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	670

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	671

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	673

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	673

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	674

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	675

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	677

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	677

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	678

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	679

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	682

9.1.22	Measurement accuracy for UE Category NB1	685

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	685

9.1.22.2	Void	686

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	686

9.1.22.4	Void	687

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	687

9.1.22.6	Void	688

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	688

9.1.22.8	Void	689

9.1.22.9	NRSRP Measurement Report Mapping	689

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	689

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	690

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	691

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	692

9.1.22.14	NRSRQ Measurement Report Mapping	693

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	694

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	694

9.1.23	Power Headroom for UE Category NB1	695

9.1.23.1	Period	695

9.1.23.2	Reporting Delay	695

9.1.23.3	Report Mapping for UE Category NB1	695

9.1.23.3.1	Void	697

9.1.23.3.2	Void	697

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	697

9.1.24	Void	698

9.1.25	Measurement accuracy for UE category M2	698

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	698

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	699

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	700

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	701

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	703

9.1.26	Measurement Accuracy for non-BL CE UE	704

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	705

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	705

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	706

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	707

9.1.26.5	RSRP Measurement Report Mapping	708

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	708

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	708

9.1.26.8	RSRQ Measurement Report Mapping	709

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	709

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	710

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	711

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	712

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	713

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	713

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	713

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	714

9.1.27	SFN and frame Timing Difference (SFTD)	715

9.1.27.1	SFTD Accuracy Requirement	715

9.2	UTRAN FDD Measurements	717

9.2.1	UTRAN FDD CPICH RSCP	717

9.2.2	Void	718

9.2.3	UTRAN FDD CPICH Ec/No	718

9.3	UTRAN TDD Measurements	718

9.3.1	UTRAN TDD P-CCPCH RSCP	719

9.3.2	Void	719

9.3.3	Void	719

9.4	GSM Measurements	719

9.4.1	GSM carrier RSSI	719

9.5	CDMA2000 1x RTT Measurements	719

9.5.1	CDMA2000 1x RTT Pilot Strength	719

9.6	PCMAX,c	720

9.6.1	Report Mapping	720

9.6.2 	Estimation Period	720

9.6.3	Reporting Delay	720

9.7	IEEE802.11 Measurements	720

9.7.1	WLAN RSSI	720

9.7.2	WLAN RSSI Measurement Report Mapping	720

9.8	MBSFN Measurements	721

9.8.1	Introduction	721

9.8.2	MBSFN RSRP	721

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	721

9.8.2.2	MBSFN RSRP measurement report mapping	722

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	722

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	723

9.8.3	MBSFN RSRQ	723

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	723

9.8.3.2	MBSFN RSRQ measurement report mapping	724

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	724

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	725

9.8.4	MCH BLER	725

9.8.4.1	Measurement report mapping for MCH BLER	725

9.8.4.2	Measurement report mapping for MCH Block Number	726

9.9	ProSe Measurements	727

9.9.1	Introduction	727

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	727

9.9.2.1	Absolute S-RSRP Accuracy	727

9.9.2.2	Relative Accuracy of S-RSRP	728

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	729

9.9.3.1	Absolute SD-RSRP Accuracy	729

9.9.3.2	Relative Accuracy of SD-RSRP	729

9.10	V2X Measurements	730

9.10.1	Introduction	730

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	730

9.10.2.1	Absolute S-RSRP Accuracy	730

9.10.2.2	Relative Accuracy of S-RSRP	731

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	731

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	731

9.10.4	S-RSSI Measurement Accuracy Requirements	732

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	732

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	732

9.11	NR Measurements	733

9.11.1	NR SS-RSRP Measurements	733

9.11.2	NR SS-RSRQ Measurements	733

9.11.3	NR SS-SINR Measurements	734

10	Measurements Performance Requirements for E-UTRAN	734

10.1	Received Interference Power	734

10.1.1	Absolute accuracy requirement	734

10.1.2	Relative accuracy requirement	735

10.1.3	Received Interference Power measurement report mapping	735

10.2	Angle of Arrival (AOA)	735

10.2.1	Range/mapping	735

10.3	Timing Advance (TADV)	735

10.3.1	Report mapping	735

11	ProSe Requirements in Any Cell Selection state	736

11.1	Introduction	736

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	736

11.2.1	Introduction	736

11.2.2	ProSe UE transmission timing	736

11.3	Initiation/Cease of SLSS Transmissions	737

11.3.1	Introduction	737

11.3.2	Requirements	737

11.4	Measurements for ProSe in Any Cell Selection State	737

11.4.1	Introduction	737

11.4.2	Requirements	737

11.4.2.1	E-UTRA FDD	737

11.4.2.2	E-UTRA TDD	738

11.5	Selection / Reselection of ProSe Synchronization Reference	738

11.5.1	Introduction	738

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	738

11.5.2.1	Introduction	738

11.5.2.2	Requirements	738

11.6	Void	739

11.7	Selection / Reselection of ProSe relay UE	739

11.7.1	Introduction	739

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	739

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	739

12.1	Introduction	739

12.2	Transmit Timing	740

12.2.1	GNSS as timing reference	740

12.3	Interruption	740

12.4	Reliability of GNSS signal	740

13	V2X Requirements	740

13.1	Introduction	740

13.2	UE Transmit Timing	741

13.2.1	Introduction	741

13.2.2	GNSS as synchronization reference source	741

13.2.3	Serving cell/PCell as synchronization reference source	741

13.2.4	SyncRef UE as synchronization reference source	741

13.3	Initiation/Cease of SLSS Transmissions	741

13.3.1	Introduction	741

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	741

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	742

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	742

13.4	Selection / Reselection of V2X Synchronization Reference Source	743

13.5	Autonomous Resource Selection/Reselection measurements	744

13.5.1	Introduction	744

13.5.2	PSSCH-RSRP measurements	744

13.5.3	S-RSSI measurements	744

13.6	Congestion Control measurements	744

13.7	Interruption	744

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	744

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	744

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	745

13.8	Reliability of GNSS signal	745

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	745

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	746

Annex A (normative):	Test Cases	747

A.1	Purpose of annex	747

A.2	Requirement classification for statistical testing	747

A.2.1	Types of requirements in TS 36.133	747

A.2.1.1	Time and delay requirements on UE higher layer actions	747

A.2.1.2	Measurements of power levels, relative powers and time	747

A.2.1.3	Implementation requirements	748

A.2.1.4	Physical layer timing requirements	748

A.3	RRM test configurations	749

A.3.1	Reference Measurement Channels	749

A.3.1.1	PDSCH	749

A.3.1.1.1	FDD	749

A.3.1.1.2	TDD	753

A.3.1.1.3	FDD for UE category 0	756

A.3.1.1.4	HD-FDD for UE category 0	757

A.3.1.1.5	TDD for UE category 0	758

A.3.1.1.6	Frame Structure 3	759

A.3.1.2	PCFICH/PDCCH/PHICH	760

A.3.1.2.1	FDD	760

A.3.1.2.2	TDD	760

A.3.1.2.3	HD-FDD for UE category 0	761

A.3.1.2.4	FS 3	761

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	761

A.3.1.3.1	FDD in CEModeA	762

A.3.1.3.2	HD-FDD in CEModeA	762

A.3.1.3.3	TDD in CEModeA	763

A.3.1.3.4	FDD in CEModeB	763

A.3.1.3.5	HD-FDD in CEModeB	764

A.3.1.3.6	TDD in CEModeB	764

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	765

A.3.1.4.1	FDD in CEModeA	765

A.3.1.4.2	HD-FDD in CEModeA	766

A.3.1.4.3	TDD in CEModeA	767

A.3.1.4.4	FDD in CEModeB	768

A.3.1.4.5	 HD-FDD in CEModeB	769

A.3.1.4.6	TDD in CEModeB	769

A.3.1.5	NPDSCH Reference Channel for UE category NB1	770

A.3.1.5.1	HD-FDD in-band operation	770

A.3.1.5.2	Void	771

A.3.1.5.3	HD-FDD standalone operation	771

A.3.1.5.4	Void	772

A.3.1.5.5	HD-FDD guard band operation	772

A.3.1.5.6	Void	773

A.3.1.5.7	TDD in-band operation	773

A.3.1.5.8	TDD standalone operation	773

A.3.1.5.9	TDD guard band operation	774

A.3.1.6	NPDCCH Reference Channel for UE category NB1	774

A.3.1.6.1	In-band operation	774

A.3.1.6.2	Void	775

A.3.1.6.3	Standalone operation	775

A.3.1.6.4	Void	775

A.3.1.6.5	Guard band operation	775

A.3.1.6.6	Void	776

A.3.2	OFDMA Channel Noise Generator (OCNG)	776

A.3.2.1	OCNG Patterns for FDD	776

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	777

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	777

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	778

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	779

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	779

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	780

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	780

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	780

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	781

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	782

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	782

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	783

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	783

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	784

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	784

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	785

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	785

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	786

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	786

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	787

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	787

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	788

A.3.2.2	OCNG Patterns for TDD	788

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	789

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	789

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	790

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	790

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	791

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	792

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	793

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	794

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	794

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	795

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	795

A.3.2.3	OCNG Patterns for Narrowband IoT	796

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	797

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	798

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	798

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	799

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	800

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	801

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	802

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	803

A.3.2.4	OCNG Patterns for V2X sidelink	804

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	804

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	805

A.3.3	Reference DRX Configurations	805

A.3.4	ABS Transmission Configurations	806

A.3.4.1	Non-MBSFN ABS Transmission Configurations	806

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	806

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	806

A.3.4.2	MBSFN ABS Transmission Configurations	807

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	807

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	808

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	808

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	808

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	809

A.3.6.1	Introduction	809

A.3.7	Test Cases with Different Channel Bandwidths	809

A.3.7.1	Introduction	809

A.3.7.2	Principle of testing	809

A.3.8	Antenna Configuration	809

A.3.8.1	Antenna connection for 4 Rx capable UEs	809

A.3.8.1.1 Introduction	809

A.3.8.1.2 Principle of testing	809

A.3.8.1.2.1 Single carrier tests	809

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	810

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	811

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	811

A.3.8.2	Antenna connection for 8 Rx capable UEs	811

A.3.8.2.1	Introduction	811

A.3.8.2.2	Principle of testing	811

A.3.8.2.2.1	Single carrier tests	811

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	812

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	812

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	812

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	812

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	812

A.3.9.1	Introduction	812

A.3.9.2	Principle of testing	812

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	813

A.3.10.1	Introduction	813

A.3.10.2	Principle of testing	813

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	813

A.3.11.1	Introduction	813

A.3.11.2	Principle of Testing	813

A.3.12	Proximity-based Services	813

A.3.12.1	Introduction	813

A.3.12.2	Reference DRX configurations for ProSe tests	813

A.3.12.3	Test Cases with Different Channel Bandwidths	814

A.3.12.3.1	Introduction	814

A.3.12.3.2	Principle of testing	814

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	814

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	820

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	822

A.3.12.6.1	FDD	822

A.3.12.7	Reference measurement channels for ProSe Direct Communication	822

A.3.12.7.1	FDD	822

A.3.12.8	ProSe Receive Traffic Generator	823

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	823

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	823

A.3.13	Time Offset between Cells	824

A.3.13.1	Introduction	824

A.3.13.2	Definition	824

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	824

A.3.14.1	Introduction	824

A.3.14.2	Principle of testing	824

A.3.15	Dual connectivity test cases with different combination of duplex mode	824

A.3.15.1	Introduction	824

A.3.15.2	Principle of testing	824

A.3.16	Reference PRACH Configurations	825

A.3.17	Listen before talk model	825

A.3.17.1	Introduction	825

A.3.17.2	Definition	825

A.3.18	Reference NPRACH Configurations	826

A.3.19	Dual connectivity test cases with different bandwidth combinations	827

A.3.19.1	Introduction	827

A.3.19.2	Principle of testing	827

A.3.20	Category M1 UE Test Cases	828

A.3.20.1	Introduction	828

A.3.20.2	Principle of Cat-M1 UE Testing	828

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	829

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	830

A.3.21.1	Introduction	830

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	830

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	831

A.3.22	Category 1bis UE Test Cases	832

A.3.22.1	Introduction	832

A.3.22.2	Principle of Category 1bis UE Testing	832

A.3.23	Category NB2 UE Test Cases	835

A.3.23.1	Introduction	835

A.3.23.2	Principle of Category NB2 UE Testing	835

A.3.24	V2X sidelink communication	837

A.3.24.1	Introduction	837

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	838

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	841

A.3.25	Category M2 UE Test Cases	842

A.3.25.1	Introduction	842

A.3.25.2	Principle of Cat-M2 UE Testing	842

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	843

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	844

A.3.26.1	Introduction	844

A.3.26.2	Principle of testing	844

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	844

A.3.27.1	Introduction	844

A.3.27.2	Principle of INACTIVE cell re-selection Testing	844

A.4	E-UTRAN RRC_IDLE state	844

A.4.2	Cell Re-Selection	844

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	844

A.4.2.1.1	Test Purpose and Environment	844

A.4.2.1.2	Test Requirements	846

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	847

A.4.2.2.1	Test Purpose and Environment	847

A.4.2.2.2	Test Requirements	848

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	849

A.4.2.3.1	Test Purpose and Environment	849

A.4.2.3.2	Test Requirements	850

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	851

A.4.2.4.1	Test Purpose and Environment	851

A.4.2.4.2	Test Requirements	852

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	853

A.4.2.5.1	Test Purpose and Environment	853

A.4.2.5.2	Test Requirements	854

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	855

A.4.2.6.1	Test Purpose and Environment	855

A.4.2.6.2	Test Requirements	856

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	857

A.4.2.7.1	Test Purpose and Environment	857

A.4.2.7.2	Test Requirements	858

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	859

A.4.2.8.1	Test Purpose and Environment	859

A.4.2.8.2	Test Requirements	860

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	861

A.4.2.9.1	Test Purpose and Environment	861

A.4.2.9.2	Test Requirements	861

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	861

A.4.2.10.1	Test Purpose and Environment	861

A.4.2.10.2	Test Requirements	865

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	865

A.4.2.11.1	Test Purpose and Environment	865

A.4.2.11.2	Test Requirements	869

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	869

A.4.2.12.1	Test Purpose and Environment	869

A.4.2.12.2	Test Requirements	871

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	872

A.4.2.13.1	Test Purpose and Environment	872

A.4.2.13.2	Test Requirements	873

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	874

A.4.2.14.1	Test Purpose and Environment	874

A.4.2.14.2	Test Requirements	875

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	876

A.4.2.15.1	Test Purpose and Environment	876

A.4.2.15.2	Test Requirements	877

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	878

A.4.2.16.1	Test Purpose and Environment	878

A.4.2.16.2	Test Requirements	879

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	880

A.4.2.17.1	Test Purpose and Environment	880

A.4.2.17.2	Test Requirements	881

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	882

A.4.2.18.1	Test Purpose and Environment	882

A.4.2.18.2	Test Requirements	884

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	885

A.4.2.19.1	Test Purpose and Environment	885

A.4.2.19.2	Test Requirements	887

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	888

A.4.2.20.1	Test Purpose and Environment	888

A.4.2.20.2	Test Requirements	889

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	890

A.4.2.21.1	Test Purpose and Environment	890

A.4.2.21.2	Test Requirements	891

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	892

A.4.2.22.1	Test Purpose and Environment	892

A.4.2.22.2	Test Requirements	893

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	894

A.4.2.23.1	Test Purpose and Environment	894

A.4.2.23.2	Test Requirements	895

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	896

A.4.2.24.1	Test Purpose and Environment	896

A.4.2.24.2	Test Requirements	898

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	899

A.4.2.25.1	Test Purpose and Environment	899

A.4.2.25.2	Test Requirements	900

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	901

A.4.2.26.1	Test Purpose and Environment	901

A.4.2.26.2	Test Requirements	902

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	903

A.4.2.27.1	Test Purpose and Environment	903

A.4.2.27.2	Test Requirements	904

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	905

A.4.2.28.1	Test Purpose and Environment	905

A.4.2.28.2	Test Requirements	906

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	907

A.4.2.29.1	Test Purpose and Environment	907

A.4.2.29.2	Test Requirements	908

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	909

A.4.2.30.1	Test Purpose and Environment	909

A.4.2.30.2	Test Requirements	910

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	911

A.4.2.31.1	Test Purpose and Environment	911

A.4.2.31.2	Test Requirements	912

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	913

A.4.2.32.1	Test Purpose and Environment	913

A.4.2.32.2	Test Requirements	914

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	915

A.4.2.33.1	Test Purpose and Environment	915

A.4.2.33.2	Test Requirements	916

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	917

A.4.2.34.1	Test Purpose and Environment	917

A.4.2.34.2	Test Requirements	918

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	919

A.4.2.35.1	Test Purpose and Environment	919

A.4.2.35.2	Test Requirements	921

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	922

A.4.2.36.1	Test Purpose and Environment	922

A.4.2.36.2	Test Requirements	924

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	925

A.4.2.37.1	Test Purpose and Environment	925

A.4.2.37.2	Test Requirements	927

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	928

A.4.2.38.1	Test Purpose and Environment	928

A.4.2.38.2	Test Requirements	930

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	931

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	931

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	931

A.4.3.1.1.1	Test Purpose and Environment	931

A.4.3.1.1.2	Test Requirements	933

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	933

A.4.3.1.2.1	Test Purpose and Environment	933

A.4.3.1.2.2	Test Requirements	936

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	936

A.4.3.1.3.1	Test Purpose and Environment	936

A.4.3.1.3.2	Test Requirements	939

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	939

A.4.3.1.4.1	Test Purpose and Environment	939

A.4.3.1.4.2	Test Requirements	940

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	940

A.4.3.1.5.1	Test Purpose and Environment	940

A.4.3.1.5.2	Test Requirements	943

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	944

A.4.3.2.1	Test Purpose and Environment	944

A.4.3.2.1.1	Void	944

A.4.3.2.1.2	1.28Mcps TDD option	944

A.4.3.2.1.3	Void	946

A.4.3.2.2	Test Requirements	946

A.4.3.2.2.1	1.28Mcps TDD option	946

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	946

A.4.3.2A.1	Test Purpose and Environment	946

A.4.3.2A.2	Test Requirements	950

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	950

A.4.3.3.1	Test Purpose and Environment	950

A.4.3.3.2	Test Requirements	953

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	953

A.4.3.3A.1	Test Purpose and Environment	953

A.4.3.3A.2	Test Requirements	958

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	959

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	959

A.4.3.4.1.1	Test Purpose and Environment	959

A.4.3.4.1.2	Test Requirements	961

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	961

A.4.3.4.2.1	Test Purpose and Environment	961

A.4.3.4.2.2	Test Requirements	963

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	963

A.4.3.4.3.1	Test Purpose and Environment	963

A.4.3.4.3.2	Test Requirements	966

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	966

A.4.3.4.4.1	Test Purpose and Environment	966

A.4.3.4.4.2	Test Requirements	970

A.4.4	E-UTRAN to GSM Cell Re-Selection	970

A.4.4.1	E-UTRAN FDD – GSM:	970

A.4.4.1.1	Test Purpose and Environment	970

A.4.4.1.2	Test Requirements	972

A.4.4.2	E-UTRAN TDD – GSM:	972

A.4.4.2.1	Test Purpose and Environment	972

A.4.4.2.2	Test Requirements	974

A.4.5	E-UTRAN to HRPD Cell Re-Selection	975

A.4.5.1	E-UTRAN FDD – HRPD	975

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	975

A.4.5.1.1.1	Test Purpose and Environment	975

A.4.5.1.1.2	Test Requirements	977

A.4.5.2	E-UTRAN TDD – HRPD	977

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	977

A.4.5.2.1.1	Test Purpose and Environment	977

A.4.5.2.1.2	Test Requirements	980

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	980

A.4.6.1	E-UTRAN FDD – cdma2000 1X	980

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	980

A.4.6.1.1.1	Test Purpose and Environment	980

A.4.6.1.1.2	Test Requirements	983

A.4.6.2	E-UTRAN TDD – cdma2000 1X	983

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	983

A.4.6.2.1.1	Test Purpose and Environment	983

A.4.6.2.1.2	Test Requirements	986

A.4.7	Idle State Positioning Measurement for UE category NB1	986

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	986

A.4.7.1.1	Test Purpose and Environment	986

A.4.7.1.2	Test Requirements	990

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	991

A.4.7.2.1	Test Purpose and Environment	991

A.4.7.4.2	Test Requirements	994

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	995

A.4.7.3.1	Test Purpose and Environment	995

A.4.7.3.2	Test Requirements	998

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	999

A.4.7.4.1	Test Purpose and Environment	999

A.4.7.4.2	Test Requirements	1002

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1003

A.5.1	E-UTRAN Handover	1003

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1003

A.5.1.1.1	Test Purpose and Environment	1003

A.5.1.1.2	Test Requirements	1005

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1005

A.5.1.2.1	Test Purpose and Environment	1005

A.5.1.2.2	Test Requirements	1007

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1007

A.5.1.3.1	Test Purpose and Environment	1007

A.5.1.3.2	Test Requirements	1009

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1009

A.5.1.4.1	Test Purpose and Environment	1009

A.5.1.4.2	Test Requirements	1011

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1011

A.5.1.5.1	Test Purpose and Environment	1011

A.5.1.5.2	Test Requirements	1013

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1013

A.5.1.6.1	Test Purpose and Environment	1013

A.5.1.6.2	Test Requirements	1015

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1015

A.5.1.7.1	Test Purpose and Environment	1015

A.5.1.7.2	Test Requirements	1018

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1018

A.5.1.8.1	Test Purpose and Environment	1018

A.5.1.8.2	Test Requirements	1021

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1021

A.5.1.9.1	Test Purpose and Environment	1021

A.5.1.9.2	Test Requirements	1022

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1022

A.5.1.10.1	Test Purpose and Environment	1022

A.5.1.10.2	Test Requirements	1024

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1024

A.5.1.11.1	Test Purpose and Environment	1024

A.5.1.11.2	Test Requirements	1026

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1026

A.5.1.12.1	Test Purpose and Environment	1026

A.5.1.12.2	Test Requirements	1028

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1028

A.5.1.13.1	Test Purpose and Environment	1028

A.5.1.13.2	Test Requirements	1030

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1031

A.5.1.14.1	Test Purpose and Environment	1031

A.5.1.14.2	Test Requirements	1032

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1033

A.5.1.15.1	Test Purpose and Environment	1033

A.5.1.15.2	Test Requirements	1034

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1035

A.5.1.16.1	Test Purpose and Environment	1035

A.5.1.16.2	Test Requirements	1036

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1037

A.5.1.17.1	Test Purpose and Environment	1037

A.5.1.17.2	Test Requirements	1038

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1039

A.5.1.18.1	Test Purpose and Environment	1039

A.5.1.18.2	Test Requirements	1040

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1041

A.5.1.19.1	Test Purpose and Environment	1041

A.5.1.19.2	Test Requirements	1042

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1043

A.5.1.20.1	Test Purpose and Environment	1043

A.5.1.20.2	Test Requirements	1044

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1045

A.5.1.21.1	Test Purpose and Environment	1045

A.5.1.21.2	Test Requirements	1046

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1046

A.5.1.22.1	Test Purpose and Environment	1046

A.5.1.22.2	Test Requirements	1048

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1048

A.5.1.23.1	Test Purpose and Environment	1048

A.5.1.23.2	Test Requirements	1050

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1050

A.5.1.24.1	Test Purpose and Environment	1050

A.5.1.24.2	Test Requirements	1052

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1052

A.5.1.25.1	Test Purpose and Environment	1052

A.5.1.25.2	Test Requirements	1054

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1055

A.5.1.26.1	Test Purpose and Environment	1055

A.5.1.26.2	Test Requirements	1056

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1057

A.5.1.27.1	Test Purpose and Environment	1057

A.5.1.27.2	Test Requirements	1058

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1059

A.5.1.28.1	Test Purpose and Environment	1059

A.5.1.28.2	Test Requirements	1060

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1061

A.5.1.29.1	Test Purpose and Environment	1061

A.5.1.29.2	Test Requirements	1062

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1063

A.5.1.30.1	Test Purpose and Environment	1063

A.5.1.30.2	Test Requirements	1064

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1065

A.5.1.31.1	Test Purpose and Environment	1065

A.5.1.31.2	Test Requirements	1066

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1067

A.5.1.32.1	Test Purpose and Environment	1067

A.5.1.32.2	Test Requirements	1068

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1069

A.5.1.33.1	Test Purpose and Environment	1069

A.5.1.13.2	Test Requirements	1070

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1071

A.5.1.34.1	Test Purpose and Environment	1071

A.5.1.34.2	Test Requirements	1072

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1073

A.5.1.35.1	Test Purpose and Environment	1073

A.5.1.35.2	Test Requirements	1074

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1075

A.5.1.36.1	Test Purpose and Environment	1075

A.5.1.36.2	Test Requirements	1076

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1077

A.5.1.37.1	Test Purpose and Environment	1077

A.5.1.37.2	Test Requirements	1078

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1079

A.5.1.38.1	Test Purpose and Environment	1079

A.5.1.38.2	Test Requirements	1080

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1081

A.5.1.39.1	Test Purpose and Environment	1081

A.5.1.39.2	Test Requirements	1082

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1083

A.5.1.40.1	Test Purpose and Environment	1083

A.5.1.40.2	Test Requirements	1084

A.5.2	E-UTRAN Handover to other RATs	1085

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1085

A.5.2.1.1	Test Purpose and Environment	1085

A.5.2.1.2	Test Requirements	1087

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1088

A.5.2.2.1	Test Purpose and Environment	1088

A.5.2.2.2	Test Requirements	1091

A.5.2.3 E-UTRAN FDD- GSM Handover	1091

A.5.2.3.1	Test Purpose and Environment	1091

A.5.2.3.2	Test Requirements	1092

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1093

A.5.2.4.1	Test Purpose and Environment	1093

A.5.2.4.1.1	Void	1093

A.5.2.4.1.2	1.28 Mcps TDD option	1093

A.5.2.4.1.3	Void	1095

A.5.2.4.2	Test Requirements	1095

A.5.2.4.2.1	Void	1095

A.5.2.4.2.2	1.28 Mcps TDD option	1095

A.5.2.4.2.3	Void	1095

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1095

A.5.2.5.1	Test Purpose and Environment	1095

A.5.2.5.1.3	Void	1098

A.5.2.5.2	Test Requirements	1098

A.5.2.5.2.1	Void	1098

A.5.2.5.2.2	1.28 Mcps TDD option	1098

A.5.2.5.2.3	Void	1098

A.5.2.6	E-UTRAN TDD - GSM Handover	1098

A.5.2.6.1	Test Purpose and Environment	1098

A.5.2.6.2	Test Requirements	1100

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1101

A.5.2.7.1	Test Purpose and Environment	1101

A.5.2.7.2	Test Requirements	1103

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1103

A.5.2.8.1	Test Purpose and Environment	1103

A.5.2.8.2	Test Requirements	1104

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1105

A.5.2.9.1	Test Purpose and Environment	1105

A.5.2.9.2	Test Requirements	1106

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1107

A.5.2.10.1	Test Purpose and Environment	1107

A.5.2.10.2	Test Requirements	1109

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1109

A.5.2.10A.1	Test Purpose and Environment	1109

A.5.2.10A.2	Test Requirements	1112

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1112

A.5.2.10B.1	Test Purpose and Environment	1112

A.5.2.10B.2	Test Requirements	1115

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1115

A.5.2.11.1	Test Purpose and Environment	1115

A.5.2.11.2	Test Requirements	1115

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1116

A.5.3.1	E-UTRAN FDD – HRPD Handover	1116

A.5.3.1.1	Test Purpose and Environment	1116

A.5.3.1.2	Test Requirements	1118

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1118

A.5.3.2.1	Test Purpose and Environment	1118

A.5.3.2.2	Test Requirements	1121

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1121

A.5.3.3.1	Test Purpose and Environment	1121

A.5.3.3.2	Test Requirements	1123

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1124

A.5.3.4.1	Test Purpose and Environment	1124

A.5.3.4.2	Test Requirements	1125

A.5.3.5	E-UTRAN TDD – HRPD Handover	1126

A.5.3.5.1	Test Purpose and Environment	1126

A.5.3.5.2	Test Requirements	1128

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1129

A.5.3.6.1	Test Purpose and Environment	1129

A.5.3.6.2	Test Requirements	1132

A.6	RRC Connection Control	1132

A.6.1	RRC Re-establishment	1132

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1132

A.6.1.1.1	Test Purpose and Environment	1132

A.6.1.1.2	Test Requirements	1133

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1134

A.6.1.2.1	Test Purpose and Environment	1134

A.6.1.2.2	Test Requirements	1135

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1136

A.6.1.3.1	Test Purpose and Environment	1136

A.6.1.3.2	Test Requirements	1137

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1138

A.6.1.4.1	Test Purpose and Environment	1138

A.6.1.4.2	Test Requirements	1139

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1140

A.6.1.5.1	Test Purpose and Environment	1140

A.6.1.5.2	Test Requirements	1140

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1140

A.6.1.6.1	Test Purpose and Environment	1140

A.6.1.6.2	Test Requirements	1142

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1143

A.6.1.7.1	Test Purpose and Environment	1143

A.6.1.7.2	Test Requirements	1144

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1145

A.6.1.8.1	Test Purpose and Environment	1145

A.6.1.8.2	Test Requirements	1146

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1147

A.6.1.9.1	Test Purpose and Environment	1147

A.6.1.9.2	Test Requirements	1148

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1149

A.6.1.10.1	Test Purpose and Environment	1149

A.6.1.10.2	Test Requirements	1150

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1151

A.6.1.11.1	Test Purpose and Environment	1151

A.6.1.11.2	Test Requirements	1152

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1153

A.6.1.12.1	Test Purpose and Environment	1153

A.6.1.12.2	Test Requirements	1154

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1155

A.6.1.13.1	Test Purpose and Environment	1155

A.6.1.13.2	Test Requirements	1156

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1157

A.6.1.14.1	Test Purpose and Environment	1157

A.6.1.14.2	Test Requirements	1158

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1159

A.6.1.15.1	Test Purpose and Environment	1159

A.6.1.15.2	Test Requirements	1161

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1162

A.6.1.16.1	Test Purpose and Environment	1162

A.6.1.16.2	Test Requirements	1164

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1165

A.6.1.17.1	Test Purpose and Environment	1165

A.6.1.17.2	Test Requirements	1166

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1167

A.6.1.18.1	Test Purpose and Environment	1167

A.6.1.18.2	Test Requirements	1168

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1169

A.6.1.19.1	Test Purpose and Environment	1169

A.6.1.19.2	Test Requirements	1170

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1171

A.6.1.20.1	Test Purpose and Environment	1171

A.6.1.20.2	Test Requirements	1172

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1173

A.6.1.21.1	Test Purpose and Environment	1173

A.6.1.21.2	Test Requirements	1174

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1175

A.6.1.22.1	Test Purpose and Environment	1175

A.6.1.22.2	Test Requirements	1176

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1177

A.6.1.23.1	Test Purpose and Environment	1177

A.6.1.23.2	Test Requirements	1179

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1180

A.6.1.24.1	Test Purpose and Environment	1180

A.6.1.24.2	Test Requirements	1182

A.6.2	Random Access	1183

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1183

A.6.2.1.1	Test Purpose and Environment	1183

A.6.2.1.2	Test Requirements	1184

A.6.2.1.2.1	Random Access Response Reception	1184

A.6.2.1.2.2	No Random Access Response Reception	1184

A.6.2.1.2.3	Receiving a NACK on msg3	1184

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1185

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1185

A.6.2.1.2.6	Contention Resolution Timer expiry	1185

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1185

A.6.2.2.1	Test Purpose and Environment	1185

A.6.2.2.2	Test Requirements	1186

A.6.2.2.2.1	Random Access Response Reception	1187

A.6.2.2.2.2	No Random Access Response Reception	1187

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1187

A.6.2.3.1	Test Purpose and Environment	1187

A.6.2.3.2	Test Requirements	1189

A.6.2.3.2.1	Random Access Response Reception	1189

A.6.2.3.2.2	No Random Access Response reception	1189

A.6.2.3.2.3	Receiving a NACK on msg3	1189

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1190

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1190

A.6.2.3.2.6	Contention Resolution Timer expiry	1190

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1190

A.6.2.4.1	Test Purpose and Environment	1190

A.6.2.4.2	Test Requirements	1192

A.6.2.4.2.1	Random Access Response Reception	1192

A.6.2.4.2.2	No Random Access Response Reception	1192

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1192

A.6.2.5.1	Test Purpose and Environment	1192

A.6.2.5.2	Test Requirements	1193

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1193

A.6.2.6.1	Test Purpose and Environment	1193

A.6.2.6.2	Test Requirements	1193

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1194

A.6.2.7.1	Test Purpose and Environment	1194

A.6.2.7.2	Test Requirements	1196

A.6.2.7.2.1	Random Access Response Reception	1196

A.6.2.7.2.2	No Random Access Response Reception	1196

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1196

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1197

A.6.2.8.1	Test Purpose and Environment	1197

A.6.2.8.2	Test Requirements	1199

A.6.2.8.2.1	Random Access Response Reception	1199

A.6.2.8.2.2	No Random Access Response Reception	1199

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1199

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1200

A.6.2.9.1	Test Purpose and Environment	1200

A.6.2.9.2	Test Requirements	1202

A.6.2.9.2.1	Random Access Response Reception	1202

A.6.2.9.2.2	No Random Access Response Reception	1203

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1203

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1204

A.6.2.10.1	Test Purpose and Environment	1204

A.6.2.10.2	Test Requirements	1206

A.6.2.10.2.1	Random Access Response Reception	1206

A.6.2.10.2.2	No Random Access Response Reception	1207

A.6.2.10.2.3	Receiving a NACK on msg3	1207

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1207

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1207

A.6.2.10.2.6	Contention Resolution Timer expiry	1207

A.6.2.10.2.7	PRACH Resource Selection	1208

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1208

A.6.2.11.1	Test Purpose and Environment	1208

A.6.2.11.2	Test Requirements	1210

A.6.2.11.2.1	Random Access Response Reception	1210

A.6.2.11.2.2	No Random Access Response Reception	1211

A.6.2.11.2.3	Receiving a NACK on msg3	1211

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1211

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1211

A.6.2.11.2.6	Contention Resolution Timer expiry	1211

A.6.2.11.2.7	PRACH Resource Selection	1212

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1212

A.6.2.12.1	Test Purpose and Environment	1212

A.6.2.12.2	Test Requirements	1214

A.6.2.12.2.1	Random Access Response Reception	1214

A.6.2.12.2.2	No Random Access Response Reception	1215

A.6.2.12.2.3	Receiving a NACK on msg3	1215

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1215

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1215

A.6.2.12.2.6	Contention Resolution Timer expiry	1215

A.6.2.12.2.7	PRACH Resource Selection	1216

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1216

A.6.2.13.1	Test Purpose and Environment	1216

A.6.2.13.2	Test Requirements	1218

A.6.2.13.2.1	Random Access Response Reception	1218

A.6.2.13.2.2	No Random Access Response Reception	1219

A.6.2.13.2.3	Receiving a NACK on msg3	1219

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1219

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1219

A.6.2.13.2.6	Contention Resolution Timer expiry	1219

A.6.2.13.2.7	PRACH Resource Selection	1220

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1220

A.6.2.14.1	Test Purpose and Environment	1220

A.6.2.14.2	Test Requirements	1222

A.6.2.14.2.1	Random Access Response Reception	1222

A.6.2.14.2.2	No Random Access Response Reception	1223

A.6.2.14.2.3	Receiving a NACK on msg3	1223

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1223

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1223

A.6.2.14.2.6	Contention Resolution Timer expiry	1223

A.6.2.14.2.7	PRACH Resource Selection	1224

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1224

A.6.2.15.1	Test Purpose and Environment	1224

A.6.2.15.2	Test Requirements	1226

A.6.2.15.2.1	Random Access Response Reception	1226

A.6.2.15.2.2	No Random Access Response Reception	1227

A.6.2.15.2.3	Receiving a NACK on msg3	1227

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1227

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1227

A.6.2.15.2.6	Contention Resolution Timer expiry	1227

A.6.2.15.2.7	PRACH Resource Selection	1228

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1228

A.6.2.16.1	Test Purpose and Environment	1228

A.6.2.16.2	Test Requirements	1231

A.6.2.16.2.1	Random Access Response Reception	1231

A.6.2.16.2.2	No Random Access Response Reception	1232

A.6.2.16.2.3	Receiving a NACK on msg3	1232

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1232

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1232

A.6.2.16.2.6	Contention Resolution Timer expiry	1232

A.6.2.16.2.7	NPRACH Resource Selection	1232

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1233

A.6.2.17.1	Test Purpose and Environment	1233

A.6.2.17.2	Test Requirements	1236

A.6.2.17.2.1	Random Access Response Reception	1236

A.6.2.17.2.2	No Random Access Response Reception	1237

A.6.2.17.2.3	Receiving a NACK on msg3	1237

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1237

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1237

A.6.2.17.2.6	Contention Resolution Timer expiry	1237

A.6.2.17.2.7	NPRACH Resource Selection	1237

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1238

A.6.2.18.1	Test Purpose and Environment	1238

A.6.2.18.2	Test Requirements	1241

A.6.2.18.2.1	Random Access Response Reception	1241

A.6.2.18.2.2	No Random Access Response Reception	1242

A.6.2.18.2.3	Receiving a NACK on msg3	1242

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1242

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1242

A.6.2.18.2.6	Contention Resolution Timer expiry	1242

A.6.2.18.2.7	NPRACH Resource Selection	1242

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1243

A.6.2.19.1	Test Purpose and Environment	1243

A.6.2.19.2	Test Requirements	1245

A.6.2.19.2.1	Random Access Response Reception	1245

A.6.2.19.2.2	No Random Access Response Reception	1246

A.6.2.19.2.3	Receiving a NACK on msg3	1246

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1246

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1246

A.6.2.19.2.6	Contention Resolution Timer expiry	1246

A.6.2.19.2.7	NPRACH Resource Selection	1246

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1247

A.6.2.20.1	Test Purpose and Environment	1247

A.6.2.20.2	Test Requirements	1249

A.6.2.20.2.1	Random Access Response Reception	1249

A.6.2.20.2.2	No Random Access Response Reception	1250

A.6.2.20.2.3	Receiving a NACK on msg3	1250

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1250

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1250

A.6.2.20.2.6	Contention Resolution Timer expiry	1250

A.6.2.20.2.7	NPRACH Resource Selection	1250

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1251

A.6.2.21.1	Test Purpose and Environment	1251

A.6.2.21.2	Test Requirements	1253

A.6.2.21.2.1	Random Access Response Reception	1253

A.6.2.21.2.2	No Random Access Response Reception	1254

A.6.2.21.2.3	Receiving a NACK on msg3	1254

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1254

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1254

A.6.2.21.2.6	Contention Resolution Timer expiry	1254

A.6.2.21.2.7	NPRACH Resource Selection	1254

A.6.3	RRC Connection Release with Redirection	1255

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1255

A.6.3.1.1	Test Purpose and Environment	1255

A.6.3.1.2	Test Requirements	1257

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1257

A.6.3.2.1	Test Purpose and Environment	1257

A.6.3.2.2	Test Requirements	1259

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1259

A.6.3.3.1	Test Purpose and Environment	1259

A.6.3.3.2	Test Requirements	1260

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1261

A.6.3.4.1	Test Purpose and Environment	1261

A.6.3.4.2	Test Requirements	1262

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1263

A.6.3.5.1	Test Purpose and Environment	1263

A.6.3.5.2	Test Requirements	1265

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1265

A.6.3.6.1	Test Purpose and Environment	1265

A. 6.3.6.2	Test Requirements	1267

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1267

A.6.3.7.1	Test Purpose and Environment	1267

A.6.3.7.2	Test Requirements	1270

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1270

A.6.3.8.1	Test Purpose and Environment	1270

A.6.3.8.2	Test Requirements	1273

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1273

A.6.3.9.1	Test Purpose and Environment	1273

A.6.3.9.2	Test Requirements	1275

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1275

A.6.3.10.1	Test Purpose and Environment	1275

A.6.3.10.2	Test Requirements	1277

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1277

A.6.3.11.1	Test Purpose and Environment	1277

A.6.3.11.2	Test Requirements	1279

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1279

A.6.3.12.1	Test Purpose and Environment	1279

A.6.3.12.2	Test Requirements	1282

A.7	Timing and Signalling Characteristics	1283

A.7.1	UE Transmit Timing	1283

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1283

A.7.1.1.1	Test Purpose and Environment	1283

A.7.1.1.2	Test Requirements	1284

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1285

A.7.1.2.1	Test Purpose and Environment	1285

A.7.1.2.2	Test Requirements	1287

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1288

A.7.1.3.1	Test Purpose and Environment	1288

A.7.1.3.2	Test Requirements	1290

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1291

A.7.1.4.1	Test Purpose and Environment	1291

A.7.1.4.2	Test Requirements	1292

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1293

A.7.1.4A.1	Test Purpose and Environment	1293

A.7.1.4A.2	Test Requirements	1293

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1293

A.7.1.5.1	Test Purpose and Environment	1293

A.7.1.5.2	Test Requirements	1294

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1294

A.7.1.6.1	Test Purpose and Environment	1294

A.7.1.6.2	Test Requirements	1296

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1297

A.7.1.7.1	Test Purpose and Environment	1297

A.7.1.7.2	Test Requirements	1298

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1299

A.7.1.7A.1	Test Purpose and Environment	1299

A.7.1.7A.2	Test Requirements	1299

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1299

A.7.1.7B.1	Test Purpose and Environment	1299

A.7.1.7B.2	Test Requirements	1299

A.7.1.8	Void	1300

A.7.1.8.1	Void	1300

A.7.1.8.2	Void	1300

A.7.1.9	Void	1300

A.7.1.9.1	Void	1300

A.7.1.9.2	Void	1300

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1300

A.7.1.10.1	Test Purpose and Environment	1300

A.7.1.10.2	Test Requirements	1302

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1302

A.7.1.11.1	Test Purpose and Environment	1302

A.7.1.11.2	Test Requirements	1304

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1304

A.7.1.12.1	Test Purpose and Environment	1304

A.7.1.12.2	Test Requirements	1306

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1306

A.7.1.13.1	Test Purpose and Environment	1306

A.7.1.13.2	Test Requirements	1308

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1309

A.7.1.14.1	Test Purpose and Environment	1309

A.7.1.14.2	Test Requirements	1310

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1310

A.7.1.15.1	Test Purpose and Environment	1310

A.7.1.15.2	Test Requirements	1311

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1312

A.7.1.16.1	Test Purpose and Environment	1312

A.7.1.16.2	Test Requirements	1313

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1314

A.7.1.17.1	Test Purpose and Environment	1314

A.7.1.17.2	Test Requirements	1316

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1317

A.7.1.18.1	Test Purpose and Environment	1317

A.7.1.18.2	Test Requirements	1319

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1320

A.7.1.19.1	Test Purpose and Environment	1320

A.7.1.19.2	Test Requirements	1321

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1321

A.7.1.20.1	Test Purpose and Environment	1321

A.7.1.20.2	Test Requirements	1323

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1323

A.7.1.21.1	Test Purpose and Environment	1323

A.7.1.21.2	Test Requirements	1325

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1325

A.7.1.22.1	Test Purpose and Environment	1325

A.7.1.22.2	Test Requirements	1327

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1327

A.7.1.23.1	Test Purpose and Environment	1327

A.7.1.23.2	Test Requirements	1329

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1329

A.7.1.24.1	Test Purpose and Environment	1329

A.7.1.24.2	Test Requirements	1330

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1331

A.7.1.25.1	Test Purpose and Environment	1331

A.7.1.25.2	Test Requirements	1332

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1333

A.7.1.26.1	Test Purpose and Environment	1333

A.7.1.26.2	Test Requirements	1334

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1335

A.7.1.27.1	Test Purpose and Environment	1335

A.7.1.27.2	Test Requirements	1337

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1337

A.7.1.28.1	Test Purpose and Environment	1337

A.7.1.28.2	Test Requirements	1339

A.7.2	UE Timing Advance	1340

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1340

A.7.2.1.1	Test Purpose and Environment	1340

A.7.2.1.2	Test Requirements	1341

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1342

A.7.2.2.1	Test Purpose and Environment	1342

A.7.2.2.2	Test Requirements	1344

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1344

A.7.2.3.1	Test Purpose and Environment	1344

A.7.2.3.2	Test Requirements	1344

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1344

A.7.2.4.1	Test Purpose and Environment	1344

A.7.2.4.2	Test Requirements	1347

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1347

A.7.2.5.1	Test Purpose and Environment	1347

A.7.2.5.2	Test Requirements	1349

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1349

A.7.2.5A.1	Test Purpose and Environment	1349

A.7.2.5A.2	Test Requirements	1350

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1350

A.7.2.5B.1	Test Purpose and Environment	1350

A.7.2.5B.2	Test Requirements	1350

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1350

A.7.2.6.1	Test Purpose and Environment	1350

A.7.2.6.2	Test Requirements	1353

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1353

A.7.2.7.1	Test Purpose and Environment	1353

A.7.2.7.2	Test Requirements	1355

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1355

A.7.2.8.1	Test Purpose and Environment	1355

A.7.2.8.2	Test Requirements	1357

A.7.2.9.2	Test Requirements	1359

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1359

A.7.2.10.1	Test Purpose and Environment	1359

A.7.2.10.2	Test Requirements	1360

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1360

A.7.2.11.1	Test Purpose and Environment	1360

A.7.2.11.2	Test Requirements	1362

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1362

A.7.2.12.1	Test Purpose and Environment	1362

A.7.2.12.2	Test Requirements	1364

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1364

A.7.2.13.1	Test Purpose and Environment	1364

A.7.2.13.2	Test Requirements	1366

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1366

A.7.2.14.1	Test Purpose and Environment	1366

A.7.2.14.2	Test Requirements	1368

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1368

A.7.2.15.1	Test Purpose and Environment	1368

A.7.2.15.2	Test Requirements	1370

A.7.3	Radio Link Monitoring	1370

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1371

A.7.3.1.1	Test Purpose and Environment	1371

A.7.3.1.2	Test Requirements	1375

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1375

A.7.3.2.1	Test Purpose and Environment	1375

A.7.3.2.2	Test Requirements	1378

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1378

A.7.3.3.1	Test Purpose and Environment	1378

A.7.3.3.2	Test Requirements	1382

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1382

A.7.3.4.1	Test Purpose and Environment	1382

A.7.3.4.2	Test Requirements	1385

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1385

A.7.3.5.1	Test Purpose and Environment	1385

A.7.3.5.2	Test Requirements	1389

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1390

A.7.3.6.1	Test Purpose and Environment	1390

A.7.3.6.2	Test Requirements	1393

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1393

A.7.3.7.1	Test Purpose and Environment	1393

A.7.3.7.2	Test Requirements	1396

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1396

A.7.3.8.1	Test Purpose and Environment	1396

A.7.3.8.2	Test Requirements	1399

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1399

A.7.3.9.2	Test Requirements	1402

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1402

A.7.3.10.1	Test Purpose and Environment	1402

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1405

A.7.3.11.1	Test Purpose and Environment	1405

A.7.3.11.2	Test Requirements	1409

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1409

A.7.3.12.1	Test Purpose and Environment	1409

A.7.3.12.2	Test Requirements	1413

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1413

A.7.3.13.1	Test Purpose and Environment	1413

A.7.3.13.2	Test Requirements	1416

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1416

A.7.3.14.1	Test Purpose and Environment	1416

A.7.3.14.2	Test Requirements	1419

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1419

A.7.3.15.1	Test Purpose and Environment	1419

A.7.3.15.2	Test Requirements	1422

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1422

A.7.3.16.1	Test Purpose and Environment	1422

A.7.3.16.2	Test Requirements	1426

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1426

A.7.3.17.1	Test Purpose and Environment	1426

A.7.3.17.2	Test Requirements	1429

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1429

A.7.3.18.1	Test Purpose and Environment	1429

A.7.3.18.2	Test Requirements	1433

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1433

A.7.3.19.1	Test Purpose and Environment	1433

A.7.3.19.2	Test Requirements	1437

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1437

A.7.3.20.1	Test Purpose and Environment	1437

A.7.3.20.2	Test Requirements	1441

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1441

A.7.3.21.1	Test Purpose and Environment	1441

A.7.3.21.2	Test Requirements	1445

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1445

A.7.3.22.1	Test Purpose and Environment	1445

A.7.3.22.2	Test Requirements	1449

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1449

A.7.3.23.1	Test Purpose and Environment	1449

A.7.3.23.2	Test Requirements	1450

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1450

A.7.3.24.1	Test Purpose and Environment	1450

A.7.3.24.2	Test Requirements	1451

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1451

A.7.3.25.1	Test Purpose and Environment	1451

A.7.3.25.2	Test Requirements	1452

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1452

A.7.3.26.1	Test Purpose and Environment	1452

A.7.3.26.2	Test Requirements	1455

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1455

A.7.3.27.1	Test Purpose and Environment	1455

A.7.3.27.2	Test Requirements	1458

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1458

A.7.3.28.1	Test Purpose and Environment	1458

A.7.3.28.2	Test Requirements	1461

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1461

A.7.3.29.1	Test Purpose and Environment	1461

A.7.3.29.2	Test Requirements	1464

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1464

A.7.3.30.1	Test Purpose and Environment	1464

A.7.3.30.2	Test Requirements	1467

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1467

A.7.3.31.1	Test Purpose and Environment	1467

A.7.3.31.2	Test Requirements	1470

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1470

A.7.3.32.1	Test Purpose and Environment	1470

A.7.3.32.2	Test Requirements	1473

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1473

A.7.3.33.1	Test Purpose and Environment	1473

A.7.3.33.2	Test Requirements	1476

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1476

A.7.3.34.1	Test Purpose and Environment	1476

A.7.3.34.2	Test Requirements	1479

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1479

A.7.3.35.1	Test Purpose and Environment	1479

A.7.3.35.2	Test Requirements	1482

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1482

A.7.3.36.1	Test Purpose and Environment	1482

A.7.3.36.2	Test Requirements	1485

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1485

A.7.3.37.1	Test Purpose and Environment	1485

A.7.3.37.2	Test Requirements	1488

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1488

A.7.3.38.1	Test Purpose and Environment	1488

A.7.3.38.2	Test Requirements	1491

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1491

A.7.3.39.1	Test Purpose and Environment	1491

A.7.3.39.2	Test Requirements	1494

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1495

A.7.3.40.1	Test Purpose and Environment	1495

A.7.3.40.2	Test Requirements	1497

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1498

A.7.3.41.1	Test Purpose and Environment	1498

A.7.3.41.2	Test Requirements	1501

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1501

A.7.3.42.1	Test Purpose and Environment	1501

A.7.3.42.2	Test Requirements	1504

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1504

A.7.3.43.1	Test Purpose and Environment	1504

A.7.3.43.2	Test Requirements	1507

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1507

A.7.3.44.1	Test Purpose and Environment	1507

A.7.3.44.2	Test Requirements	1510

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1511

A.7.3.45.1	Test Purpose and Environment	1511

A.7.3.45.2	Test Requirements	1513

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1514

A.7.3.46.1	Test Purpose and Environment	1514

A.7.3.46.2	Test Requirements	1517

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1517

A.7.3.47.1	Test Purpose and Environment	1517

A.7.3.47.2	Test Requirements	1520

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1520

A.7.3.48.1	Test Purpose and Environment	1520

A.7.3.48.2	Test Requirements	1523

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1523

A.7.3.49.1	Test Purpose and Environment	1523

A.7.3.49.2	Test Requirements	1526

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1526

A.7.3.50.1	Test Purpose and Environment	1526

A.7.3.50.2	Test Requirements	1529

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1529

A.7.3.51.1	Test Purpose and Environment	1529

A.7.3.51.2	Test Requirements	1532

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1532

A.7.3.52.1	Test Purpose and Environment	1532

A.7.3.52.2	Test Requirements	1535

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1535

A.7.3.53.1	Test Purpose and Environment	1535

A.7.3.53.2	Test Requirements	1538

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1538

A.7.3.54.1	Test Purpose and Environment	1538

A.7.3.54.2	Test Requirements	1541

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1541

A.7.3.55.1	Test Purpose and Environment	1541

A.7.3.55.2	Test Requirements	1544

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1544

A.7.3.56.1	Test Purpose and Environment	1544

A.7.3.56.2	Test Requirements	1547

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1547

A.7.3.57.1	Test Purpose and Environment	1547

A.7.3.57.2	Test Requirements	1550

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1550

A.7.3.58.1	Test Purpose and Environment	1550

A.7.3.58.2	Test Requirements	1553

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1553

A.7.3.59.1	Test Purpose and Environment	1553

A.7.3.59.2	Test Requirements	1556

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1556

A.7.3.60.1	Test Purpose and Environment	1556

A.7.3.60.2	Test Requirements	1560

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1560

A.7.3.61.1	Test Purpose and Environment	1560

A.7.3.61.2	Test Requirements	1564

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1564

A.7.3.62.1	Test Purpose and Environment	1564

A.7.3.62.2	Test Requirements	1568

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1568

A.7.3.63.1	Test Purpose and Environment	1568

A.7.3.63.2	Test Requirements	1572

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1572

A.7.3.64.1	Test Purpose and Environment	1572

A.7.3.64.2	Test Requirements	1575

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1576

A.7.3.65.1	Test Purpose and Environment	1576

A.7.3.65.2	Test Requirements	1579

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1580

A.7.3.66.1	Test Purpose and Environment	1580

A.7.3.66.2	Test Requirements	1582

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1583

A.7.3.67.1	Test Purpose and Environment	1583

A.7.3.67.2	Test Requirements	1586

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1587

A.7.3.68.1	Test Purpose and Environment	1587

A.7.3.68.2	Test Requirements	1589

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1589

A.7.3.69.1	Test Purpose and Environment	1589

A.7.3.69.2	Test Requirements	1591

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1591

A.7.3.70.1	Test Purpose and Environment	1591

A.7.3.70.2	Test Requirements	1593

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1593

A.7.3.71.1	Test Purpose and Environment	1593

A.7.3.71.2	Test Requirements	1596

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1596

A.7.3.72.1	Test Purpose and Environment	1596

A.7.3.72.2	Test Requirements	1598

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1598

A.7.3.73.1	Test Purpose and Environment	1598

A.7.3.73.2	Test Requirements	1600

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1600

A.7.3.74.1	Test Purpose and Environment	1600

A.7.3.74.2	Test Requirements	1603

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1603

A.7.3.75.1	Test Purpose and Environment	1603

A.7.3.75.2	Test Requirements	1605

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1606

A.7.3.76.1	Test Purpose and Environment	1606

A.7.3.76.2	Test Requirements	1609

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1609

A.7.3.77.1	Test Purpose and Environment	1609

A.7.3.77.2	Test Requirements	1612

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1612

A.7.3.78.1	Test Purpose and Environment	1612

A.7.3.78.2	Test Requirements	1615

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1615

A.7.3.79.1	Test Purpose and Environment	1615

A.7.3.79.2	Test Requirements	1618

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1618

A.7.3.80.1	Test Purpose and Environment	1618

A.7.3.80.2	Test Requirements	1621

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1621

A.7.3.81.1	Test Purpose and Environment	1621

A.7.3.81.2	Test Requirements	1624

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1624

A.7.3.82.1	Test Purpose and Environment	1624

A.7.3.82.2	Test Requirements	1627

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1627

A.7.3.83.1	Test Purpose and Environment	1627

A.7.3.83.2	Test Requirements	1628

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1629

A.7.3.84.1	Test Purpose and Environment	1629

A.7.3.84.2	Test Requirements	1631

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1631

A.7.3.85.1	Test Purpose and Environment	1631

A.7.3.85.2	Test Requirements	1632

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1633

A.7.3.86.1	Test Purpose and Environment	1633

A.7.3.86.2	Test Requirements	1635

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1635

A.7.3.87.1	Test Purpose and Environment	1635

A.7.3.87.2	Test Requirements	1637

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1638

A.7.3.88.1	Test Purpose and Environment	1638

A.7.3.88.2	Test Requirements	1642

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1642

A.7.3.89.1	Test Purpose and Environment	1642

A.7.3.89.2	Test Requirements	1646

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1646

A.7.3.90.1	Test Purpose and Environment	1646

A.7.3.90.2	Test Requirements	1650

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1650

A.7.3.91.1	Test Purpose and Environment	1650

A.7.3.91.2	Test Requirements	1654

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1654

A.7.3.92.1	Test Purpose and Environment	1654

A.7.3.92.2	Test Requirements	1657

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1658

A.7.3.93.1	Test Purpose and Environment	1658

A.7.3.93.2	Test Requirements	1661

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1662

A.7.3.94.1	Test Purpose and Environment	1662

A.7.3.94.2	Test Requirements	1664

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1665

A.7.3.95.1	Test Purpose and Environment	1665

A.7.3.95.2	Test Requirements	1668

A.7.4	Interruption for Dual Connectivity	1669

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1669

A.7.4.1.1	Test Purpose and Environment	1669

A.7.4.1.2	Test Requirements	1671

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1671

A.7.4.2.1	Test Purpose and Environment	1671

A.7.4.2.2	Test Requirements	1673

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1673

A.7.4.3.1	Test Purpose and Environment	1673

A.7.4.3.2	Test Requirements	1675

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1675

A.7.4.4.1	Test Purpose and Environment	1675

A.7.4.4.2	Test Requirements	1678

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1678

A.7.4.5.1	Test Purpose and Environment	1678

A.7.4.5.2	Test Requirements	1680

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1680

A.7.4.6.1	Test Purpose and Environment	1680

A.7.4.6.2	Test Requirements	1682

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1682

A.7.4.7.1	Test Purpose and Environment	1682

A.7.4.7.2	Test Requirements	1685

A.7.5	Proximity-based Services	1685

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1685

A.7.5.1.1	Test Purpose and Environment	1685

A.7.5.1.2	Test Requirements	1686

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1687

A.7.5.2.1	Test Purpose and Environment	1687

A.7.5.1.2	Test Requirements	1687

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1688

A.7.5.3.1	Test Purpose and Environment	1688

A.7.5.3.2	Test Requirements	1689

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1690

A.7.5.4.1	Test Purpose and Environment	1690

A.7.5.4.2	Test Requirements	1691

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1692

A.7.5.5.1	Test Purpose and Environment	1692

A.7.5.5.2	Test Requirements	1694

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1695

A.7.5.6.1	Test Purpose and Environment	1695

A.7.5.6.2	Test Requirements	1696

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1697

A.7.5.7.1	Test Purpose and Environment	1697

A.7.5.7.2	Test Requirements	1699

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1699

A.7.5.8.1	Test Purpose and Environment	1699

A.7.5.8.2	Test Requirements	1701

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1702

A.7.5.9.1	Test Purpose and Environment	1702

A.7.5.9.2	Test Requirements	1704

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1705

A.7.5.10.1	Test Purpose and Environment	1705

A.7.5.10.2	Test Requirements	1707

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1708

A.7.5.11.1	Test Purpose and Environment	1708

A.7.5.11.2	Test Requirements	1710

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1710

A.7.5.12.1	Test Purpose and Environment	1710

A.7.5.12.2	Test Requirements	1714

A.7.6	Interruption for carrier aggregation	1715

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1715

A.7.6.1.1	Test Purpose and Environment	1715

A.7.6.1.2	Test Requirements	1717

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1717

A.7.6.2.1	Test Purpose and Environment	1717

A.7.6.2.2	Test Requirements	1720

A.8	UE Measurements Procedures	1720

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1720

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1720

A.8.1.1.1	Test Purpose and Environment	1720

A.8.1.1.2	Test Requirements	1722

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1722

A.8.1.2.1	Test Purpose and Environment	1722

A.8.1.2.2	Test Requirements	1724

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1724

A.8.1.3.1	Test Purpose and Environment	1724

A.8.1.3.2	Test Requirements	1727

A.8.1.4	Void	1727

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1727

A.8.1.5.1	Test Purpose and Environment	1727

A.8.1.5.2	Test Requirements	1729

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1729

A.8.1.6.1	Test Purpose and Environment	1729

A.8.1.6.2	Test Requirements	1731

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1731

A.8.1.7.1	Test Purpose and Environment	1731

A.8.1.7.2	Test Requirements	1733

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1734

A.8.1.8.1	Test Purpose and Environment	1734

A.8.1.8.2	Test Requirements	1736

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1737

A.8.1.9.1	Test Purpose and Environment	1737

A.8.1.9.2	Test Requirements	1737

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1737

A.8.1.10.1	Test Purpose and Environment	1737

A.8.1.10.2	Test Requirements	1738

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1738

A.8.1.11.1	Test Purpose and Environment	1738

A.8.1.11.2	Test Requirements	1740

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1741

A.8.1.12.1	Test Purpose and Environment	1741

A.8.1.12.2	Test Requirements	1742

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1743

A.8.1.13.1	Test Purpose and Environment	1743

A.8.1.13.2	Test Requirements	1745

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1745

A.8.1.14.1	Test Purpose and Environment	1745

A.8.1.14.2	Test Requirements	1747

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1748

A.8.1.15.1	Test Purpose and Environment	1748

A.8.1.15.2	Test Requirements	1749

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1750

A.8.1.16.1	Test Purpose and Environment	1750

A.8.1.16.2	Test Requirements	1752

A.8.1.17	Void	1752

A.8.1.18	Void	1752

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1752

A.8.1.19.1	Test Purpose and Environment	1752

A.8.1.19.2	Test Requirements	1754

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1755

A.8.1.20.1	Test Purpose and Environment	1755

A.8.1.20.2	Test Requirements	1757

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1757

A.8.1.21.1	Test Purpose and Environment	1757

A.8.1.21.2	Test Requirements	1759

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1760

A.8.1.22.1	Test Purpose and Environment	1760

A.8.1.22.2	Test Requirements	1762

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1762

A.8.1.23.1	Test Purpose and Environment	1762

A.8.1.23.2	Test Requirements	1763

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1764

A.8.1.24.1	Test Purpose and Environment	1764

A.8.1.24.2	Test Requirements	1766

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1766

A.8.1.25.1	Test Purpose and Environment	1766

A.8.1.25.2	Test Requirements	1768

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1768

A.8.1.26.1	Test Purpose and Environment	1768

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1771

A.8.1.27.1	Test Purpose and Environment	1771

A.8.1.27.2	Test Requirements	1773

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1773

A.8.1.28.1	Test Purpose and Environment	1773

A.8.1.28.2	Test Requirements	1775

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1775

A.8.1.29.1	Test Purpose and Environment	1775

A.8.1.29.2	Test Requirements	1777

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1778

A.8.1.30.1	Test Purpose and Environment	1778

A.8.1.30.2	Test Requirements	1780

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1780

A.8.1.31.1	Test Purpose and Environment	1780

A.8.1.31.2	Test Requirements	1782

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1783

A.8.1.32.1	Test Purpose and Environment	1783

A.8.1.32.2	Test Requirements	1784

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1785

A.8.1.33.1	Test Purpose and Environment	1785

A.8.1.33.2	Test Requirements	1786

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1787

A.8.1.34.1	Test Purpose and Environment	1787

A.8.1.34.2	Test Requirements	1788

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1789

A.8.1.35.1	Test Purpose and Environment	1789

A.8.1.35.2	Test Requirements	1790

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1791

A.8.1.36.1	Test Purpose and Environment	1791

A.8.1.36.2	Test Requirements	1792

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1793

A.8.1.37.1	Test Purpose and Environment	1793

A.8.1.37.2	Test Requirements	1795

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1795

A.8.1.38.1	Test Purpose and Environment	1795

A.8.1.38.2	Test Requirements	1796

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1797

A.8.1.39.1	Test Purpose and Environment	1797

A.8.1.39.2	Test Requirements	1799

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1799

A.8.1.40.1	Test Purpose and Environment	1799

A.8.1.40.2	Test Requirements	1801

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1801

A.8.1.41.1	Test Purpose and Environment	1801

A.8.1.41.2	Test Requirement	1803

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1804

A.8.1.42.1	Test Purpose and Environment	1804

A.8.1.42.2	Test Requirement	1805

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1806

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1806

A.8.2.1.1	Test Purpose and Environment	1806

A.8.2.1.2	Test Requirements	1807

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1808

A.8.2.2.1	Test Purpose and Environment	1808

A.8.2.2.2	Test Requirements	1810

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1810

A.8.2.3.1	Test Purpose and Environment	1810

A.8.2.3.2	Test Requirements	1812

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1813

A.8.2.4.1	Test Purpose and Environment	1813

A.8.2.4.2	Test Requirements	1815

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1815

A.8.2.5.1	Test Purpose and Environment	1815

A.8.2.5.2	Test Requirements	1817

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1818

A.8.2.6.1	Test Purpose and Environment	1818

A.8.2.6.2	Test Requirements	1820

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1821

A.8.2.7.1	Test Purpose and Environment	1821

A.8.2.7.2	Test Requirements	1822

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1823

A.8.2.8.1	Test Purpose and Environment	1823

A.8.2.8.2	Test Requirements	1825

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1825

A.8.2.9.1	Test Purpose and Environment	1825

A.8.2.9.2	Test Requirements	1827

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1828

A.8.2.10.1	Test Purpose and Environment	1828

A.8.2.10.2	Test Requirements	1830

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1830

A.8.2.11.1	Test Purpose and Environment	1830

A.8.2.11.2	Test Requirements	1832

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1832

A.8.2.12.1	Test Purpose and Environment	1832

A.8.2.12.2	Test Requirements	1834

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1835

A.8.2.13.1	Test Purpose and Environment	1835

A.8.2.13.2	Test Requirements	1837

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1837

A.8.2.14.1	Test Purpose and Environment	1837

A.8.2.14.2	Test Requirement	1839

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1840

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1840

A.8.3.1.1	Test Purpose and Environment	1840

A.8.3.1.2	Test Requirements	1841

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1841

A.8.3.2.1	Test Purpose and Environment	1841

A.8.3.2.2	Test Requirements	1844

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1844

A.8.3.3.1	Test Purpose and Environment	1844

A.8.3.3.2	Test Requirements	1847

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1847

A.8.3.4.1	Test Purpose and Environment	1847

A.8.3.4.2	Test Requirements	1849

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1849

A.8.3.5.2	Test Requirements	1851

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1851

A.8.3.6.1	Test Purpose and Environment	1851

A.8.3.6.2	Test Requirements	1853

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1854

A.8.3.7.1	Test Purpose and Environment	1854

A.8.3.7.2	Test Requirements	1856

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1857

A.8.3.8.1	Test Purpose and Environment	1857

A.8.3.8.2	Test Requirements	1860

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1860

A.8.3.9.1	Test Purpose and Environment	1860

A.8.3.9.2	Test Requirements	1863

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1864

A.8.3.10.1	Test Purpose and Environment	1864

A.8.3.10.2	Test Requirements	1865

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1865

A.8.3.11.1	Test Purpose and Environment	1865

A.8.3.11.2	Test Requirement	1867

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1868

A.8.3.12.1	Test Purpose and Environment	1868

A.8.3.12.2	Test Requirement	1869

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1870

A.8.3.13.1	Test Purpose and Environment	1870

A.8.3.13.2	Test Requirement	1871

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1872

A.8.3.14.1	Test Purpose and Environment	1872

A.8.3.14.2	Test Requirement	1873

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1874

A.8.3.15.1	Test Purpose and Environment	1874

A.8.3.15.2	Test Requirement	1875

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1876

A.8.3.16.1	Test Purpose and Environment	1876

A.8.3.16.2	Test Requirement	1878

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1878

A.8.3.17.1	Test Purpose and Environment	1878

A.8.3.17.2	Test Requirement	1881

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1881

A.8.3.18.1	Test Purpose and Environment	1881

A.8.3.18.2	Test Requirement	1884

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1884

A.8.3.19.1	Test Purpose and Environment	1884

A.8.3.19.2	Test Requirement	1887

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1887

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1887

A.8.4.1.1	Test Purpose and Environment	1887

A.8.4.1.2	Test Requirements	1889

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1889

A.8.4.2.1	Test Purpose and Environment	1889

A.8.4.2.2	Test Requirements	1892

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1892

A.8.4.3.1	Test Purpose and Environment	1892

A.8.4.3.2	Test Requirements	1894

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1895

A.8.4.4.1	Test Purpose and Environment	1895

A.8.4.4.2	Test Requirements	1897

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1898

A.8.4.5.1	Test Purpose and Environment	1898

A.8.4.5.2	Test Requirements	1900

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1900

A.8.4.6.1	Test Purpose and Environment	1900

A.8.4.6.2	Test Requirements	1901

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1901

A.8.4.7.1	Test Purpose and Environment	1901

A.8.4.7.2	Test Requirements	1905

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1905

A.8.4.8.1	Test Purpose and Environment	1905

A.8.4.8.2	Test Requirements	1908

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1908

A.8.4.9.1	Test Purpose and Environment	1908

A.8.4.9.2	Test Requirements	1912

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1912

A.8.4.10.1	Test Purpose and Environment	1912

A.8.4.10.2	Test Requirements	1914

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1914

A.8.4.11.1	Test Purpose and Environment	1914

A.8.4.11.2	Test Requirement	1916

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1916

A.8.4.12.1	Test Purpose and Environment	1916

A.8.4.12.2	Test Requirement	1918

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1919

A.8.4.13.1	Test Purpose and Environment	1919

A.8.4.13.2	Test Requirement	1920

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1921

A.8.4.14.1	Test Purpose and Environment	1921

A.8.4.14.2	Test Requirement	1923

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1923

A.8.4.15.1	Test Purpose and Environment	1923

A.8.4.15.2	Test Requirement	1926

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1926

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1926

A.8.5.1.1	Test Purpose and Environment	1926

A.8.5.1.2	Test Requirements	1928

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1928

A.8.5.2.1	Test Purpose and Environment	1928

A.8.5.2.2	Test Requirements	1930

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1930

A.8.5.3.1	Test Purpose and Environment	1930

A.8.5.3.2	Test Requirements	1933

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1933

A.8.5.4.1	Test Purpose and Environment	1933

A.8.5.4.2	Test Requirements	1935

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1935

A.8.5.5.1	Test Purpose and Environment	1935

A.8.5.5.2	Test Requirements	1938

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1938

A.8.5.6.1	Test Purpose and Environment	1938

A.8.5.6.2	Test Requirements	1939

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1940

A.8.5.7.1	Test Purpose and Environment	1940

A.8.5.7.2	Test Requirements	1940

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1940

A.8.5.8.1	Test Purpose and Environment	1940

A.8.5.8.2	Test Requirements	1943

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1943

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1943

A.8.6.1.1	Test Purpose and Environment	1943

A.8.6.1.2	Test Requirements	1945

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1945

A.8.6.2.1	Test Purpose and Environment	1945

A.8.6.2.2	Test Requirements	1948

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1948

A.8.6.3.1	Test Purpose and Environment	1948

A.8.6.3.2	Test Requirements	1951

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1951

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1951

A.8.7.1.1	Test Purpose and Environment	1951

A.8.7.1.1.1	Void	1951

A.8.7.1.1.2	1.28 Mcps TDD option	1951

A.8.7.1.1.3	Void	1953

A.8.7.1.2	Test Requirements	1953

A.8.7.1.2.1	Void	1953

A.8.7.1.2.2	1.28 Mcps TDD option	1953

A.8.7.1.2.3	Void	1953

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1953

A.8.7.2.1	Test Purpose and Environment	1953

A.8.7.2.2	Test Requirements	1956

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1957

A.8.7.3.1	Test Purpose and Environment	1957

A.8.7.3.2	Test Parameters	1957

A.8.7.3.3	Test Requirements	1958

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1959

A.8.7.4.1	Test Purpose and Environment	1959

A.8.7.4.2	Test Requirements	1961

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1961

A.8.7.5.1	Test Purpose and Environment	1961

A.8.7.5.2	Test Requirements	1963

A.8.7A	TBD	1963

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1963

A.8.7A.1.1	Test Purpose and Environment	1963

A.8.7A.1.2	Test Requirements	1966

A.8.8	E-UTRAN FDD – GSM Measurements	1966

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1966

A.8.8.1.1	Test Purpose and Environment	1966

A.8.8.1.2	Test Requirements	1968

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1968

A.8.8.2.1	Test Purpose and Environment	1968

A.8.8.2.2	Test Requirements	1970

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1971

A.8.8.3.1	Test Purpose and Environment	1971

A.8.8.3.2	Test Requirements	1972

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1973

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1973

A.8.9.1.1	Test Purpose and Environment	1973

A.8.9.1.2	Test Requirements	1974

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1975

A.8.9.2.1	Test Purpose and Environment	1975

A.8.9.2.2	Test Requirements	1977

A.8.10	E-UTRAN TDD – GSM Measurements	1977

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1977

A.8.10.1.1	Test Purpose and Environment	1977

A.8.10.1.2	Test Requirements	1978

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1979

A.8.10.2.1	Test Purpose and Environment	1979

A.8.10.2.2	Test Requirements	1981

A.8.11	Monitoring of Multiple Layers	1981

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1981

A.8.11.1.1	Test Purpose and Environment	1981

A.8.11.1.2	Test Requirements	1983

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1984

A.8.11.2.1	Test Purpose and Environment	1984

A.8.11.2.2	Test Requirements	1985

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1985

A.8.11.3.1	Test Purpose and Environment	1985

A.8.11.3.2	Test Requirements	1988

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1988

A.8.11.4.1	Test Purpose and Environment	1988

A.8.11.4.2	Test Requirements	1991

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1991

A.8.11.5.1	Test Purpose and Environment	1991

A.8.11.5.2	Test Requirements	1993

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1994

A.8.11.6.1	Test Purpose and Environment	1994

A.8.11.6.2	Test Requirements	1996

A.8.12	RSTD Intra-frequency Measurements	1997

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1997

A.8.12.1.1	Test Purpose and Environment	1997

A.8.12.1.2	Test Requirements	2000

A.8.12.1.2A	Test Requirements for UE Category 1bis	2001

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2001

A.8.12.2.1	Test Purpose and Environment	2001

A.8.12.2.2	Test Requirements	2005

A.8.12.2.2A	Test Requirements for UE Category 1bis	2006

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2006

A.8.12.3.1	Test Purpose and Environment	2006

A.8.12.3.2	Test Requirements	2010

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2011

A.8.12.4.1	Test Purpose and Environment	2011

A.8.12.4.2	Test Requirements	2014

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2015

A.8.12.5.1	Test Purpose and Environment	2015

A.8.12.5.2	Test Requirements	2018

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2019

A.8.12.6.1	Test Purpose and Environment	2019

A.8.12.6.2	Test Requirements	2022

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2023

A.8.12.7.1	Test Purpose and Environment	2023

A.8.12.7.2	Test Requirements	2026

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2027

A.8.12.8.1	Test Purpose and Environment	2027

A.8.12.8.2	Test Requirements	2030

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2031

A.8.12.9.1	Test Purpose and Environment	2031

A.8.12.9.2	Test Requirements	2034

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2035

A.8.12.10.1	Test Purpose and Environment	2035

A.8.12.10.2	Test Requirements	2038

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2039

A.8.12.11.1	Test Purpose and Environment	2039

A.8.12.11.2	Test Requirements	2043

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2044

A.8.12.12.1	Test Purpose and Environment	2044

A.8.12.12.2	Test Requirements	2047

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2048

A.8.12.13.1	Test Purpose and Environment	2048

A.8.12.13.2	Test Requirements	2051

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2052

A.8.12.14.1	Test Purpose and Environment	2052

A.8.12.14.2	Test Requirements	2056

A.8.13	RSTD Inter-frequency Measurements	2057

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2057

A.8.13.1.1	Test Purpose and Environment	2057

A.8.13.1.2	Test Requirements	2060

A.8.13.1.2A	Test Requirements for UE Category 1bis	2061

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2061

A.8.13.2.1	Test Purpose and Environment	2061

A.8.13.2.2	Test Requirements	2065

A.8.13.2.2A	Test Requirements for UE Category 1bis	2066

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2066

A.8.13.3.1	Test Purpose and Environment	2066

A.8.13.3.2	Test Requirements	2070

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2071

A.8.13.4.1	Test Purpose and Environment	2071

A.8.13.4.2	Test Requirements	2074

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2075

A.8.13.5.1	Test Purpose and Environment	2075

A.8.13.5.2	Test Requirements	2079

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2080

A.8.13.6.1	Test Purpose and Environment	2080

A.8.13.6.2	Test Requirements	2083

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2084

A.8.13.7.1	Test Purpose and Environment	2084

A.8.13.7.2	Test Requirements	2087

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2088

A.8.13.8.1	Test Purpose and Environment	2088

A.8.13.8.2	Test Requirements	2092

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2093

A.8.13.9.1	Test Purpose and Environment	2093

A.8.13.9.2	Test Requirements	2097

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2098

A.8.13.10.1	Test Purpose and Environment	2098

A.8.13.10.2	Test Requirements	2102

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2103

A.8.13.11.1	Test Purpose and Environment	2103

A.8.13.11.2	Test Requirements	2107

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2108

A.8.13.12.1	Test Purpose and Environment	2108

A.8.13.12.2	Test Requirements	2112

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2113

A.8.13.13.1	Test Purpose and Environment	2113

A.8.13.13.2	Test Requirements	2117

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2118

A.8.13.14.1	Test Purpose and Environment	2118

A.8.13.14.2	Test Requirements	2122

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2123

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2123

A.8.14.1.1	Test Purpose and Environment	2123

A.8.14.1.2	Test Requirements	2125

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2125

A.8.14.2.1	Test Purpose and Environment	2125

A.8.14.2.2	Test Requirements	2128

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2128

A.8.14.3.1	Test Purpose and Environment	2128

A.8.14.3.2	Test Requirements	2130

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2131

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2131

A.8.15.1.1	Test Purpose and Environment	2131

A.8.15.1.2	Test Requirements	2132

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2132

A.8.15.2.1	Test Purpose and Environment	2132

A.8.15.2.2	Test Requirements	2135

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2135

A.8.15.3.1	Test Purpose and Environment	2135

A.8.15.3.2	Test Requirements	2137

A.8.16	E-UTRAN Carrier Aggregation Measurements	2138

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2138

A.8.16.1.1	Test Purpose and Environment	2138

A.8.16.1.2	Test Requirements	2140

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2140

A.8.16.2.1	Test Purpose and Environment	2140

A.8.16.2.2	Test Requirements	2142

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2142

A.8.16.3.1	Test Purpose and Environment	2142

A.8.16.3.2	Test Requirements	2144

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2145

A.8.16.3A.1	Test Purpose and Environment	2145

A.8.16.3A.2	Test Requirements	2146

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2147

A.8.16.4.1	Test Purpose and Environment	2147

A.8.16.4.2	Test Requirements	2148

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2149

A.8.16.4A.1	Test Purpose and Environment	2149

A.8.16.4A.2	Test Requirements	2151

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2152

A.8.16.5.1	Test Purpose and Environment	2152

A.8.16.5.2	Test Requirements	2152

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2152

A.8.16.6.1	Test Purpose and Environment	2152

A.8.16.6.2	Test Requirements	2153

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2153

A.8.16.7.1	 Test Purpose and Environment	2153

A.8.16.7.2	 Test Requirements	2154

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2154

A.8.16.8.1	 Test Purpose and Environment	2154

A.8.16.8.2	 Test Requirements	2155

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2155

A.8.16.9.1	Test Purpose and Environment	2155

A.8.16.9.2	Test Requirements	2156

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2156

A.8.16.10.1	Test Purpose and Environment	2156

A.8.16.10.2	Test Requirements	2157

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2157

A.8.16.11.1	 Test Purpose and Environment	2157

A.8.16.11.2	 Test Requirements	2158

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2158

A.8.16.12.1	 Test Purpose and Environment	2158

A.8.16.12.2	 Test Requirements	2158

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2158

A.8.16.13.1	Test Purpose and Environment	2158

A.8.16.13.2	Test Requirements	2159

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2159

A.8.16.14.1	Test Purpose and Environment	2159

A.8.16.14.2	Test Requirements	2160

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2160

A.8.16.15.1	 Test Purpose and Environment	2160

A.8.16.7.2	 Test Requirements	2160

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2160

A.8.16.16.1	 Test Purpose and Environment	2160

A.8.16.16.2	 Test Requirements	2161

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2161

A.8.16.17.1	Test Purpose and Environment	2161

A.8.16.17.2	Test Requirements	2163

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2164

A.8.16.17A.1	Test Purpose and Environment	2164

A.8.16.17A.2	Test Requirements	2164

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2164

A.8.16.17B.1	Test Purpose and Environment	2164

A.8.16.17B.2	Test Requirements	2165

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2165

A.8.16.17C.1	Test Purpose and Environment	2165

A.8.16.17C.2	Test Requirements	2165

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2165

A.8.16.18.1	Test Purpose and Environment	2165

A.8.16.18.2	Test Requirements	2167

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2168

A.8.16.18A.1	Test Purpose and Environment	2168

A.8.16.18A.2	Test Requirements	2168

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2168

A.8.16.18B.1	Test Purpose and Environment	2168

A.8.16.18B.2	Test Requirements	2169

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2169

A.8.16.18C.1	Test Purpose and Environment	2169

A.8.16.18C.2	Test Requirements	2169

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2169

A.8.16.18D.1	Test Purpose and Environment	2169

A.8.16.18D.2	Test Requirements	2170

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2170

A.8.16.19.1	Test Purpose and Environment	2170

A.8.16.19.2	Test Requirements	2172

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2172

A.8.16.19A.1	Test Purpose and Environment	2172

A.8.16.19A.2	Test Requirements	2173

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2173

A.8.16.19B.1	Test Purpose and Environment	2173

A.8.16.19B.2	Test Requirements	2173

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2173

A.8.16.19C.1	Test Purpose and Environment	2173

A.8.16.19C.2	Test Requirements	2174

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2174

A.8.16.20.1	Test Purpose and Environment	2174

A.8.16.20.2	Test Requirements	2176

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2177

A.8.16.20A.1	Test Purpose and Environment	2177

A.8.16.20A.2	Test Requirements	2177

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2177

A.8.16.20B.1	Test Purpose and Environment	2177

A.8.16.20B.2	Test Requirements	2178

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2178

A.8.16.20C.1	Test Purpose and Environment	2178

A.8.16.20C.2	Test Requirements	2178

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2178

A.8.16.20D.1	Test Purpose and Environment	2178

A.8.16.20D.2	Test Requirements	2179

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2179

A.8.16.21.1	Test Purpose and Environment	2179

A.8.16.21.2	Test Requirements	2181

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2181

A.8.16.22.1	Test Purpose and Environment	2181

A.8.16.22.2	Test Requirements	2183

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2183

A.8.16.23.1	Test Purpose and Environment	2183

A.8.16.23.2	Test Requirements	2186

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2186

A.8.16.24.1	Test Purpose and Environment	2186

A.8.16.24.2	Test Requirements	2189

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2189

A.8.16.25.1	Test Purpose and Environment	2189

A.8.16.25.2	Test Requirements	2191

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2191

A.8.16.26.1	Test Purpose and Environment	2191

A.8.16.26.2	Test Requirements	2194

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2194

A.8.16.27.1	Test Purpose and Environment	2194

A.8.16.27.2	Test Requirements	2198

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2198

A.8.16.28.1	Test Purpose and Environment	2198

A.8.16.28.2	Test Requirements	2202

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2202

A.8.16.29.1	Test Purpose and Environment	2202

A.8.16.29.2	Test Requirements	2206

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2206

A.8.16.30.1	Test Purpose and Environment	2206

A.8.16.30.2	Test Requirements	2210

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2210

A.8.16.31.1	Test Purpose and Environment	2210

A.8.16.31.2	Test Requirements	2214

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2214

A.8.16.32.1	Test Purpose and Environment	2214

A.8.16.32.2	Test Requirements	2218

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2218

A.8.16.33.1	Test Purpose and Environment	2218

A.8.16.33.2	Test Requirements	2222

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2222

A.8.16.34.1	Test Purpose and Environment	2222

A.8.16.34.2	Test Requirements	2226

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2226

A.8.16.35.1	Test Purpose and Environment	2226

A.8.16.35.2	Test Requirements	2228

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2229

A.8.16.36.1	Test Purpose and Environment	2229

A.8.16.36.2	Test Requirements	2231

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2232

A.8.16.37.1	Test Purpose and Environment	2232

A.8.16.37.2	Test Requirements	2234

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2235

A.8.16.38.1	Test Purpose and Environment	2235

A.8.16.38.2	Test Requirements	2237

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2238

A.8.16.39.1	Test Purpose and Environment	2238

A.8.16.39.2	Test Requirements	2241

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2241

A.8.16.40.1	Test Purpose and Environment	2241

A.8.16.40.2	Test Requirements	2244

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2244

A.8.16.41.1	Test Purpose and Environment	2244

A.8.16.41.2	Test Requirements	2246

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2247

A.8.16.42.1	Test Purpose and Environment	2247

A.8.16.42.2	Test Requirements	2249

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2250

A.8.16.43.1	Test Purpose and Environment	2250

A.8.16.43.2	Test Requirements	2253

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2253

A.8.16.44.1	Test Purpose and Environment	2253

A.8.16.44.2	Test Requirements	2256

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2256

A.8.16.45.1	Test Purpose and Environment	2256

A.8.16.45.2	Test Requirements	2259

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2259

A.8.16.46.1	Test Purpose and Environment	2259

A.8.16.46.2	Test Requirements	2261

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2262

A.8.16.47.1	Test Purpose and Environment	2262

A.8.16.47.2	Test Requirements	2264

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2265

A.8.16.48.1	Test Purpose and Environment	2265

A.8.16.48.2	Test Requirements	2267

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2268

A.8.16.49.1	Test Purpose and Environment	2268

A.8.16.49.2	Test Requirements	2271

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2271

A.8.16.50.1	Test Purpose and Environment	2271

A.8.16.50.2	Test Requirements	2273

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2274

A.8.16.51.1	Test Purpose and Environment	2274

A.8.16.51.2	Test Requirements	2278

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2278

A.8.16.52.1	Test Purpose and Environment	2278

A.8.16.52.2	Test Requirements	2282

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2282

A.8.16.53.1	Test Purpose and Environment	2282

A.8.16.53.2	Test Requirements	2286

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2286

A.8.16.54.1	Test Purpose and Environment	2286

A.8.16.54.2	Test Requirements	2290

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2290

A.8.16.55.1	Test Purpose and Environment	2290

A.8.16.55.2	Test Requirements	2295

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2295

A.8.16.56.1	Test Purpose and Environment	2295

A.8.16.56.2	Test Requirements	2300

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2300

A.8.16.57.1	Test Purpose and Environment	2300

A.8.16.57.2	Test Requirements	2302

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2303

A.8.16.58.1	Test Purpose and Environment	2303

A.8.16.58.2	Test Requirements	2305

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2306

A.8.16.59.1	Test Purpose and Environment	2306

A.8.16.59.2	Test Requirements	2309

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2310

A.8.16.60.1	Test Purpose and Environment	2310

A.8.16.60.2	Test Requirements	2313

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2314

A.8.16.61.1	Test Purpose and Environment	2314

A.8.16.61.2	Test Requirements	2317

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2317

A.8.16.62.1	Test Purpose and Environment	2317

A.8.16.62.2	Test Requirements	2319

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2320

A.8.16.63.1	Test Purpose and Environment	2320

A.8.16.63.2	Test Requirements	2323

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2324

A.8.16.64.1	Test Purpose and Environment	2324

A.8.16.64.2	Test Requirements	2328

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2328

A.8.16.65.1	Test Purpose and Environment	2328

A.8.16.65.2	Test Requirements	2333

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2333

A.8.16.66.1	Test Purpose and Environment	2333

A.8.16.66.2	Test Requirements	2338

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2338

A.8.16.67.1	Test Purpose and Environment	2338

A.8.16.67.2	Test Requirements	2343

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2343

A.8.16.68.1	Test Purpose and Environment	2343

A.8.16.68.2	Test Requirements	2348

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2348

A.8.16.69.1	Test Purpose and Environment	2348

A.8.16.69.2	Test Requirements	2351

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2351

A.8.16.70.1	Test Purpose and Environment	2351

A.8.16.70.2	Test Requirements	2354

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2354

A.8.16.71.1	Test Purpose and Environment	2354

A.8.16.71.2	Test Requirements	2358

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2358

A.8.16.72.1	Test Purpose and Environment	2358

A.8.16.72.2	Test Requirements	2362

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2362

A.8.16.73.1	Test Purpose and Environment	2362

A.8.16.73.2	Test Requirements	2367

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2367

A.8.16.74.1	Test Purpose and Environment	2367

A.8.16.74.2	Test Requirements	2372

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2372

A.8.16.75.1	Test Purpose and Environment	2372

A.8.16.75.2	Test Requirements	2377

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2377

A.8.16.76.1	Test Purpose and Environment	2377

A.8.16.76.2	Test Requirements	2382

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2382

A.8.16.77.1	Test Purpose and Environment	2382

A.8.16.77.2	Test Requirements	2385

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2385

A.8.16.78.1	Test Purpose and Environment	2385

A.8.16.78.2	Test Requirements	2388

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2389

A.8.16.79.1	Test Purpose and Environment	2389

A.8.16.79.2	Test Requirements	2394

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2394

A.8.16.80.1	Test Purpose and Environment	2394

A.8.16.80.2	Test Requirements	2399

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2399

A.8.16.81.1	Test Purpose and Environment	2399

A.8.16.81.2	Test Requirements	2404

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2404

A.8.16.82.1	Test Purpose and Environment	2404

A.8.16.82.2	Test Requirements	2409

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2409

A.8.16.83.1	Test Purpose and Environment	2409

A.8.16.83.2	Test Requirements	2414

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2414

A.8.16.84.1	Test Purpose and Environment	2414

A.8.16.84.2	Test Requirements	2420

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2420

A.8.16.85.1	Test Purpose and Environment	2420

A.8.16.85.2	Test Requirements	2423

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2423

A.8.16.86.1	Test Purpose and Environment	2423

A.8.16.86.2	Test Requirements	2426

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2426

A.8.16.87.1	Test Purpose and Environment	2426

A.8.16.87.2	Test Requirements	2431

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2431

A.8.16.88.1	Test Purpose and Environment	2431

A.8.16.88.2	Test Requirements	2439

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2439

A.8.16.89.1	Test Purpose and Environment	2439

A.8.16.89.2	Test Requirements	2444

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2444

A.8.16.90.1	Test Purpose and Environment	2444

A.8.16.90.2	Test Requirements	2449

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2449

A.8.16.91.1	Test Purpose and Environment	2449

A.8.16.91.2	Test Requirements	2455

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2455

A.8.16.92.1	Test Purpose and Environment	2455

A.8.16.92.2	Test Requirements	2463

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2463

A.8.16.93.1	Test Purpose and Environment	2463

A.8.16.93.2	Test Requirements	2468

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2468

A.8.16.94.1	Test Purpose and Environment	2468

A.8.16.94.2	Test Requirements	2473

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2473

A.8.16.95.1	Test Purpose and Environment	2473

A.8.16.95.2	Test Requirements	2481

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2481

A.8.16.96.1	Test Purpose and Environment	2481

A.8.16.96.2	Test Requirements	2488

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2488

A.8.16.97.1	Test Purpose and Environment	2488

A.8.16.97.2	Test Requirements	2493

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2493

A.8.16.98.1	Test Purpose and Environment	2493

A.8.16.98.2	Test Requirements	2499

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2499

A.8.16.99.1	Test Purpose and Environment	2499

A.8.16.99.2	Test Requirements	2506

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2506

A.8.16.100.1	Test Purpose and Environment	2506

A.8.16.100.2	Test Requirements	2514

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2514

A.8.16.101.1	Test Purpose and Environment	2514

A.8.16.101.2	Test Requirements	2521

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2521

A.8.16.102.1	Test Purpose and Environment	2521

A.8.16.102.2	Test Requirements	2529

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2529

A.8.16.103.1	Test Purpose and Environment	2529

A.8.16.103.2	Test Requirements	2533

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2534

A.8.16.104.1	Test Purpose and Environment	2534

A.8.16.104.2	Test Requirements	2537

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2538

A.8.16.105.1	Test Purpose and Environment	2538

A.8.16.105.2	Test Requirements	2542

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2543

A.8.16.106.1	Test Purpose and Environment	2543

A.8.16.106.2	Test Requirements	2546

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2546

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2546

A.8.17.1.1	Test Purpose and Environment	2546

A.8.17.1.2	Test Requirements	2552

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2552

A.8.17.2.1	Test Purpose and Environment	2552

A.8.17.2.2	Test Requirements	2558

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2558

A.8.17.3.1	Test Purpose and Environment	2558

A.8.17.3.2	Test Requirements	2559

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2559

A.8.17.4.1	Test Purpose and Environment	2559

A.8.17.4.2	Test Requirements	2560

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2560

A.8.17.5.1	Test Purpose and Environment	2560

A.8.17.5.2	Test Requirements	2561

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2561

A.8.17.6.1	Test Purpose and Environment	2561

A.8.17.6.2	Test Requirements	2562

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2563

A.8.17.7.1	Test Purpose and Environment	2563

A.8.17.7.2	Test Requirements	2563

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2564

A.8.17.8.1	Test Purpose and Environment	2564

A.8.17.8.2	Test Requirements	2564

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2565

A.8.17.9.1	Test Purpose and Environment	2565

A.8.17.9.2	Test Requirements	2566

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2566

A.8.17.10.1	Test Purpose and Environment	2566

A.8.17.10.2	Test Requirements	2571

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2572

A.8.17.11.1	Test Purpose and Environment	2572

A.8.17.11.2	Test Requirements	2578

A.8.18	E-UTRAN TDD – HRPD Measurements	2579

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2579

A.8.18.1.1	Test Purpose and Environment	2579

A.8.18.1.2	Test Requirements	2581

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2581

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2581

A.8.19.1.1	Test Purpose and Environment	2581

A.8.19.1.2	Test Requirements	2582

A.8.20	Inter-frequency/RAT Measurements in CA mode	2583

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2583

A.8.20.1.1	Test Purpose and Environment	2583

A.8.20.1.2	Test Requirements	2584

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2584

A.8.20.2.1	Test Purpose and Environment	2585

A.8.20.2.2	Test Requirements	2586

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2587

A.8.20.2A.1	Test Purpose and Environment	2587

A.8.20.2A.2	Test Requirements	2587

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2587

A.8.20.2B.1	Test Purpose and Environment	2587

A.8.20.2B.2	Test Requirements	2590

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2590

A.8.20.3.1	Test Purpose and Environment	2590

A.8.20.3.2	Test Requirements	2592

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2592

A.8.20.4.1	Test Purpose and Environment	2592

A.8.20.4.1.1	1.28 Mcps TDD option	2592

A.8.20.4.2	Test Requirements	2594

A.8.20.4.2.1	1.28 Mcps TDD option	2594

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2594

A.8.20.4A.1	Test Purpose and Environment	2594

A.8.20.4A.1.1	1.28 Mcps TDD option	2594

A.8.20.4A.2	Test Requirements	2594

A.8.20.4A.2.1	1.28 Mcps TDD option	2594

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2595

A.8.20.4B.1	Test Purpose and Environment	2595

A.8.20.4B.1.1	1.28 Mcps TDD option	2595

A.8.20.4B.2	Test Requirements	2597

A.8.20.4B.2.1	1.28 Mcps TDD option	2597

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2597

A.8.21.1	Test Purpose and Environment	2597

A.8.21.2 Test Requirements	2601

A.8.22	E-UTRAN Discovery Signal Measurements	2601

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2601

A.8.22.1.1	Test Purpose and Environment	2601

A.8.22.1.2	Test Requirements	2604

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2604

A.8.22.2.1	Test Purpose and Environment	2604

A.8.22.2.2	Test Requirements	2607

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2607

A.8.22.3.1	Test Purpose and Environment	2607

A.8.22.3.2	Test Requirements	2610

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2610

A.8.22.4.1	Test Purpose and Environment	2610

A.8.22.4.2	Test Requirements	2613

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2613

A.8.22.5.1	Test Purpose and Environment	2613

A.8.22.5.2	Test Requirements	2616

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2616

A.8.22.6.1	Test Purpose and Environment	2616

A.8.22.6.2	Test Requirements	2619

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2619

A.8.22.7.1	Test Purpose and Environment	2619

A.8.22.7.2	Test Requirements	2622

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2622

A.8.22.8.1	Test Purpose and Environment	2622

A.8.22.8.2	Test Requirements	2625

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2625

A.8.22.9.1	Test Purpose and Environment	2625

A.8.22.9.2	Test Requirements	2627

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2628

A.8.22.10.1	Test Purpose and Environment	2628

A.8.22.10.2	Test Requirements	2630

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2631

A.8.22.11.1	Test Purpose and Environment	2631

A.8.22.11.2	Test Requirements	2633

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2633

A.8.22.12.1	Test Purpose and Environment	2633

A.8.22.12.2	Test Requirements	2636

A.8.23	E-UTRAN Dual Connectivity Measurements	2636

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2636

A.8.23.1.1	Test Purpose and Environment	2636

A.8.23.1.2	Test Requirements	2639

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2639

A.8.23.2.1	Test Purpose and Environment	2639

A.8.23.2.2	Test Requirements	2642

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2642

A.8.23.3.1	Test Purpose and Environment	2642

A.8.23.3.2	Test Requirements	2645

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2645

A.8.23.4.1	Test Purpose and Environment	2645

A.8.23.4.2	Test Requirements	2648

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2648

A.8.23.5.1	Test Purpose and Environment	2648

A.8.23.5.2	Test Requirements	2651

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2651

A.8.23.6.1	Test Purpose and Environment	2651

A.8.23.6.2	Test Requirements	2654

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2654

A.8.23.7.1	Test Purpose and Environment	2654

A.8.23.7.2	Test Requirements	2656

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2657

A.8.23.8.1	Test Purpose and Environment	2657

A.8.23.8.2	Test Requirements	2659

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2660

A.8.23.9.1	Test Purpose and Environment	2660

A.8.23.9.2	Test Requirements	2663

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2663

A.8.23.10.1	Test Purpose and Environment	2663

A.8.23.10.2	Test Requirements	2666

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2666

A.8.23.11.1	Test Purpose and Environment	2666

A.8.23.11.2	Test Requirements	2669

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2669

A.8.23.12.1	Test Purpose and Environment	2669

A.8.23.12.2	Test Requirements	2672

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2672

A.8.23.13.1	Test Purpose and Environment	2672

A.8.23.13.2	Test Requirements	2675

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2675

A.8.23.14.1	Test Purpose and Environment	2675

A.8.23.14.2	Test Requirements	2678

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2678

A.8.23.15.1	Test Purpose and Environment	2678

A.8.23.15.2	Test Requirements	2681

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2681

A.8.23.16.1	Test Purpose and Environment	2681

A.8.23.16.2	Test Requirements	2682

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2683

A.8.23.17.1	Test Purpose and Environment	2683

A.8.23.17.2	Test Requirements	2685

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2685

A.8.23.18.1	Test Purpose and Environment	2685

A.8.23.18.2	Test Requirements	2686

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2687

A.8.23.19.1	Test Purpose and Environment	2687

A.8.23.19.2	Test Requirements	2688

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2689

A.8.23.20.1	Test Purpose and Environment	2689

A.8.23.20.2	Test Requirements	2690

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2691

A.8.23.21.1	Test Purpose and Environment	2691

A.8.23.21.2	Test Requirements	2692

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2693

A.8.23.22.1	Test Purpose and Environment	2693

A.8.23.22.2	Test Requirements	2694

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2695

A.8.23.23.1	Test Purpose and Environment	2695

A.8.23.23.2	Test Requirements	2696

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2697

A.8.23.24.1	Test Purpose and Environment	2697

A.8.23.24.2	Test Requirements	2699

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2700

A.8.23.25.1	Test Purpose and Environment	2700

A.8.23.25.2	Test Requirements	2702

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2703

A.8.23.26.1	Test Purpose and Environment	2703

A.8.23.26.2	Test Requirements	2705

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2706

A.8.23.27.1	Test Purpose and Environment	2706

A.8.23.27.2	Test Requirements	2709

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2709

A.8.23.28.1	Test Purpose and Environment	2709

A.8.23.28.2	Test Requirements	2712

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2712

A.8.23.29.1	Test Purpose and Environment	2712

A.8.23.29.2	Test Requirements	2715

A.8.24	Proximity-based Services	2715

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2715

A.8.24.1.1	Test Purpose and Environment	2715

A.8.24.1.2	Test Requirements	2716

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2717

A.8.24.2.1	Test Purpose and Environment	2717

A.8.24.2.2	Test Requirements	2718

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2718

A.8.24.3.1	Test Purpose and Environment	2719

A.8.24.3.2	Test Requirements	2720

A.8.25	E-UTRAN-WLAN Measurements	2721

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2721

A.8.25.1.1	Test Purpose and Environment	2721

A.8.25.1.2	Test Requirements	2723

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2723

A.8.25.2.1	Test Purpose and Environment	2723

A.8.25.2.2	Test Requirements	2725

A.8.26 Frame Structure 3 (FS3)	2725

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2725

A.8.26.1.1	Test Purpose and Environment	2725

A.8.26.1.2	Test Requirements	2727

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2728

A.8.26.2.1	Test Purpose and Environment	2728

A.8.26.2.2	Test Requirements	2730

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2731

A.8.26.3.1	Test Purpose and Environment	2731

A.8.26.3.2	Test Requirements	2734

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2734

A.8.26.3A.1	Test Purpose and Environment	2734

A.8.26.3A.2	Test Requirements	2737

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2737

A.8.26.4.1	Test Purpose and Environment	2737

A.8.26.4.2	Test Requirements	2740

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2740

A.8.26.4A.1	Test Purpose and Environment	2740

A.8.26.4A.2	Test Requirements	2743

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2743

A.8.26.5.1	Test Purpose and Environment	2743

A.8.26.5.2	Test Requirements	2746

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2746

A.8.26.5A.1	Test Purpose and Environment	2746

A.8.26.5A.2	Test Requirements	2749

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2749

A.8.26.6.1	Test Purpose and Environment	2749

A.8.26.6.2	Test Requirements	2752

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2752

A.8.26.6A.1	Test Purpose and Environment	2752

A.8.26.6A.2	Test Requirements	2755

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2755

A.8.26.7.1	Test Purpose and Environment	2755

A.8.26.7.2	Test Requirements	2758

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2758

A.8.26.8.1	Test Purpose and Environment	2758

A.8.26.8.2	Test Requirements	2761

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2761

A.8.26.9.1	Test Purpose and Environment	2761

A.8.26.9.2	Test Requirements	2764

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2764

A.8.26.10.1	Test Purpose and Environment	2764

A.8.26.10.2	Test Requirements	2766

A.9	Measurement Performance Requirements	2767

A.9.1	RSRP	2767

A.9.1.1	FDD Intra frequency case	2767

A.9.1.1.1	Test Purpose and Environment	2767

A.9.1.1.2	Test parameters	2767

A.9.1.1.3	Test Requirements	2770

A.9.1.2	TDD Intra frequency case	2770

A.9.1.2.1	Test Purpose and Environment	2770

A.9.1.2.2	Test parameters	2770

A.9.1.2.3	Test Requirements	2771

A.9.1.3	FDD—FDD Inter frequency case	2772

A.9.1.3.1	Test Purpose and Environment	2772

A.9.1.3.2	Test parameters	2772

A.9.1.3.3	Test Requirements	2775

A.9.1.4	TDD—TDD Inter frequency case	2775

A.9.1.4.1	Test Purpose and Environment	2775

A.9.1.4.2	Test parameters	2775

A.9.1.4.3	Test Requirements	2777

A.9.1.5	FDD—TDD Inter frequency case	2778

A.9.1.5.1	Test Purpose and Environment	2778

A.9.1.5.2	Test parameters	2778

A.9.1.5.3	Test Requirements	2779

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2780

A.9.1.6.1	Test Purpose and Environment	2780

A.9.1.6.2	Test parameters	2780

A.9.1.6.3	Test Requirements	2783

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2783

A.9.1.7.1	Test Purpose and Environment	2783

A.9.1.7.2	Test parameters	2783

A.9.1.7.3	Test Requirements	2786

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2786

A.9.1.8.1	Test Purpose and Environment	2786

A.9.1.8.2	Test parameters	2786

A.9.1.8.3	Test Requirements	2790

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2790

A.9.1.9.1	Test Purpose and Environment	2790

A.9.1.9.2	Test parameters	2790

A.9.1.9.3	Test Requirements	2792

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2793

A.9.1.10.1	Test Purpose and Environment	2793

A.9.1.10.2	Test parameters	2793

A.9.1.10.3	Test Requirements	2796

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2796

A.9.1.11.1	Test Purpose and Environment	2796

A.9.1.11.2	Test parameters	2796

A.9.1.11.3	Test Requirements	2800

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2800

A.9.1.12.1	Test Purpose and Environment	2800

A.9.1.12.2	Test parameters	2800

A.9.1.12.3	Test Requirements	2801

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2801

A.9.1.13.1	Test Purpose and Environment	2801

A.9.1.13.2	Test parameters	2801

A.9.1.13.3	Test Requirements	2802

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2802

A.9.1.14.1	Test Purpose and Environment	2802

A.9.1.14.2	Test parameters	2802

A.9.1.14.3	Test Requirements	2806

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2806

A.9.1.15.1	Test Purpose and Environment	2806

A.9.1.15.2	Test parameters	2806

A.9.1.15.3	Test Requirements	2808

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2809

A.9.1.16.1	Test Purpose and Environment	2809

A.9.1.16.2	Test parameters	2809

A.9.1.16.3	Test Requirements	2810

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2810

A.9.1.17.1	Test Purpose and Environment	2810

A.9.1.17.2	Test parameters	2811

A.9.1.17.3	Test Requirements	2812

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2812

A.9.1.18.1	Test Purpose and Environment	2812

A.9.1.18.2	Test parameters	2812

A.9.1.18.3	Test Requirements	2813

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2814

A.9.1.19.1	Test Purpose and Environment	2814

A.9.1.19.2	Test parameters	2814

A.9.1.19.3	Test Requirements	2814

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2814

A.9.1.20.1	Test Purpose and Environment	2814

A.9.1.20.2	Test parameters	2814

A.9.1.20.3	Test Requirements	2816

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2816

A.9.1.21.1	Test Purpose and Environment	2816

A.9.1.21.2	Test parameters	2816

A.9.1.21.3	Test Requirements	2816

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2816

A.9.1.22.1	Test Purpose and Environment	2816

A.9.1.22.2	Test parameters	2817

A.9.1.22.3	Test Requirements	2821

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2821

A.9.1.23.1	Test Purpose and Environment	2821

A.9.1.23.2	Test parameters	2821

A.9.1.23.3	Test Requirements	2825

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2825

A.9.1.24.1	Test Purpose and Environment	2825

A.9.1.24.2	Test parameters	2825

A.9.1.24.3	Test Requirements	2826

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2826

A.9.1.25.1	Test Purpose and Environment	2826

A.9.1.25.2	Test parameters	2826

A.9.1.25.3	Test Requirements	2829

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2829

A.9.1.26.1	Test Purpose and Environment	2829

A.9.1.26.2	Test parameters	2829

A.9.1.26.3	Test Requirements	2832

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2832

A.9.1.27.1	Test Purpose and Environment	2832

A.9.1.27.2	Test parameters	2832

A.9.1.27.3	Test Requirements	2835

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2835

A.9.1.28.1	Test Purpose and Environment	2835

A.9.1.28.2	Test parameters	2835

A.9.1.28.3	Test Requirements	2838

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2838

A.9.1.29.1	Test Purpose and Environment	2838

A.9.1.29.2	Test parameters	2838

A.9.1.29.3	Test Requirements	2841

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2841

A.9.1.30.1	Test Purpose and Environment	2841

A.9.1.30.2	Test parameters	2842

A.9.1.30.3	Test Requirements	2845

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2845

A.9.1.31.1	Test Purpose and Environment	2845

A.9.1.31.2	Test parameters	2845

A.9.1.31.3	Test Requirements	2848

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2848

A.9.1.32.1	Test Purpose and Environment	2848

A.9.1.32.2	Test parameters	2849

A.9.1.32.3	Test Requirements	2852

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2852

A.9.1.33.1	Test Purpose and Environment	2852

A.9.1.33.2	Test parameters	2852

A.9.1.33.3	Test Requirements	2855

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2855

A.9.1.34.1	Test Purpose and Environment	2855

A.9.1.34.2	Test parameters	2855

A.9.1.34.3	Test Requirements	2858

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2858

A.9.1.35.1	Test Purpose and Environment	2858

A.9.1.35.2	Test parameters	2858

A.9.1.35.3	Test Requirements	2861

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2862

A.9.1.36.1	Test Purpose and Environment	2862

A.9.1.36.2	Test parameters	2862

A.9.1.36.3	Test Requirements	2865

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2865

A.9.1.37.1	Test Purpose and Environment	2865

A.9.1.37.2	Test parameters	2865

A.9.1.37.3	Test Requirements	2868

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2869

A.9.1.38.1	Test Purpose and Environment	2869

A.9.1.38.2	Test parameters	2869

A.9.1.38.3	Test Requirements	2872

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2873

A.9.1.39.1	Test Purpose and Environment	2873

A.9.1.39.2	Test parameters	2873

A.9.1.39.3	Test Requirements	2879

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2879

A.9.1.40.1	Test Purpose and Environment	2879

A.9.1.40.2	Test parameters	2880

A.9.1.40.3	Test Requirements	2886

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2886

A.9.1.41.1	Test Purpose and Environment	2886

A.9.1.41.2	Test parameters	2886

A.9.1.41.3	Test Requirements	2889

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2889

A.9.1.42.1	Test Purpose and Environment	2889

A.9.1.42.2	Test parameters	2889

A.9.1.42.3	Test Requirements	2892

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2892

A.9.1.43.1	Test Purpose and Environment	2892

A.9.1.43.2	Test parameters	2892

A.9.1.43.3	Test Requirements	2893

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2894

A.9.1.44.1	Test Purpose and Environment	2894

A.9.1.44.2	Test parameters	2894

A.9.1.44.3	Test Requirements	2899

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2899

A.9.1.45.1	Test Purpose and Environment	2899

A.9.1.45.2	Test parameters	2899

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2482

A.9.1.54.1	Test Purpose and Environment	2482

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.2	RSRQ	2582

A.9.2.1	FDD Intra frequency case	2582

A.9.2.1.1	Test Purpose and Environment	2582

A.9.2.1.2	Test parameters	2582

A.9.2.1.3	Test Requirements	2585

A.9.2.2	TDD Intra frequency case	2585

A.9.2.2.1	Test Purpose and Environment	2585

A.9.2.2.2	Test parameters	2585

A.9.2.2.3	Test Requirements	2587

A.9.2.3	FDD—FDD Inter frequency case	2587

A.9.2.3.1	Test Purpose and Environment	2587

A.9.2.3.2	Test parameters	2587

A.9.2.3.3	Test Requirements	2590

A.9.2.4	TDD—TDD Inter frequency case	2590

A.9.2.4.1	Test Purpose and Environment	2590

A.9.2.4.2	Test parameters	2590

A.9.2.4.3	Test Requirements	2592

A.9.2.4A	FDD—TDD Inter frequency case	2593

A.9.2.4A.1	Test Purpose and Environment	2593

A.9.2.4A.2	Test parameters	2593

A.9.2.4A.3	Test Requirements	2595

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2595

A.9.2.5.1	Test Purpose and Environment	2595

A.9.2.5.2	Test parameters	2596

A.9.2.5.3	Test Requirements	2599

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2599

A.9.2.6.1	Test Purpose and Environment	2599

A.9.2.6.2	Test parameters	2599

A.9.2.6.3	Test Requirements	2602

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2602

A.9.2.7.1	Test Purpose and Environment	2602

A.9.2.7.2	Test parameters	2602

A.9.2.7.3	Test Requirements	2606

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2606

A.9.2.8.1	Test Purpose and Environment	2606

A.9.2.8.2	Test parameters	2606

A.9.2.8.3	Test Requirements	2609

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2609

A.9.2.9.1	Test Purpose and Environment	2609

A.9.2.9.2	Test parameters	2609

A.9.2.9.3	Test Requirements	2613

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2613

A.9.2.10.1	Test Purpose and Environment	2613

A.9.2.10.2	Test parameters	2613

A.9.2.10.3	Test Requirements	2617

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2617

A.9.2.11.1	Test Purpose and Environment	2617

A.9.2.11.2	Test parameters	2617

A.9.2.11.3	Test Requirements	2618

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2618

A.9.2.12.1	Test Purpose and Environment	2618

A.9.2.12.2	Test parameters	2619

A.9.2.12.3	Test Requirements	2619

A.9.2.13	Void	2620

A.9.2.13.1	Void	2620

A.9.2.13.2	Void	2620

A.9.2.13.3	Void	2620

A.9.2.14	Void	2620

A.9.2.14.1	Void	2620

A.9.2.14.2	Void	2620

A.9.2.14.3	Void	2620

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2620

A.9.2.15.1	Test Purpose and Environment	2620

A.9.2.15.2	Test parameters	2620

A.9.2.15.3	Test Requirements	2624

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2624

A.9.2.16.1	Test Purpose and Environment	2624

A.9.2.16.2	Test parameters	2624

A.9.2.16.3	Test Requirements	2629

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2629

A.9.2.17.1	Test Purpose and Environment	2629

A.9.2.17.2	Test parameters	2629

A.9.2.17.3	Test Requirements	2631

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2631

A.9.2.18.1	Test Purpose and Environment	2631

A.9.2.18.2	Test parameters	2631

A.9.2.18.3	Test Requirements	2634

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2634

A.9.2.19.1	Test Purpose and Environment	2634

A.9.2.19.2	Test parameters	2634

A.9.2.19.3	Test Requirements	2636

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2636

A.9.2.20.1	Test Purpose and Environment	2636

A.9.2.20.2	Test parameters	2636

A.9.2.20.3	Test Requirements	2639

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2639

A.9.2.21.1	Test Purpose and Environment	2639

A.9.2.21.2	Test parameters	2639

A.9.2.21.3	Test Requirements	2642

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2642

A.9.2.22.1	Test Purpose and Environment	2643

A.9.2.22.2	Test parameters	2643

A.9.2.22.3	Test Requirements	2643

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2643

A.9.2.23.1	Test Purpose and Environment	2643

A.9.2.23.2	Test parameters	2643

A.9.2.23.3	Test Requirements	2646

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2646

A.9.2.24.1	Test Purpose and Environment	2646

A.9.2.24.2	Test parameters	2646

A.9.2.24.3	Test Requirements	2647

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2647

A.9.2.25.1	Test Purpose and Environment	2647

A.9.2.25.2	Test parameters	2647

A.9.2.25.3	Test Requirements	2652

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2652

A.9.2.26.1	Test Purpose and Environment	2652

A.9.2.26.2	Test parameters	2652

A.9.2.26.3	Test Requirements	2656

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2656

A.9.2.27.1	Test Purpose and Environment	2656

A.9.2.27.2	Test parameters	2656

A.9.2.27.3	Test Requirements	2657

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2657

A.9.2.28.1	Test Purpose and Environment	2657

A.9.2.28.2	Test parameters	2657

A.9.2.28.3	Test Requirements	2660

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2660

A.9.2.29.1	Test Purpose and Environment	2660

A.9.2.29.2	Test parameters	2660

A.9.2.29.3	Test Requirements	2663

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2663

A.9.2.30.1	Test Purpose and Environment	2663

A.9.2.30.2	Test parameters	2663

A.9.2.30.3	Test Requirements	2666

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2666

A.9.2.31.1	Test Purpose and Environment	2666

A.9.2.31.2	Test parameters	2666

A.9.2.31.3	Test Requirements	2669

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2669

A.9.2.32.1	Test Purpose and Environment	2669

A.9.2.32.2	Test parameters	2669

A.9.2.32.3	Test Requirements	2672

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2672

A.9.2.33.1	Test Purpose and Environment	2672

A.9.2.33.2	Test parameters	2672

A.9.2.33.3	Test Requirements	2676

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2676

A.9.2.34.1	Test Purpose and Environment	2676

A.9.2.34.2	Test parameters	2676

A.9.2.34.3	Test Requirements	2679

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2679

A.9.2.35.1	Test Purpose and Environment	2679

A.9.2.35.2	Test parameters	2679

A.9.2.35.3	Test Requirements	2682

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2682

A.9.2.36.1	Test Purpose and Environment	2682

A.9.2.36.2	Test parameters	2682

A.9.2.36.3	Test Requirements	2683

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2684

A.9.2.37.1	Test Purpose and Environment	2684

A.9.2.37.2	Test parameters	2684

A.9.2.37.3	Test Requirements	2685

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2686

A.9.2.38.1	Test Purpose and Environment	2686

A.9.2.38.2	Test parameters	2686

A.9.2.38.3	Test Requirements	2690

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2690

A.9.2.39.1	Test Purpose and Environment	2690

A.9.2.39.2	Test parameters	2690

A.9.2.39.3	Test Requirements	2694

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2694

A.9.2.40.1	Test Purpose and Environment	2694

A.9.2.40.2	Test parameters	2694

A.9.2.40.3	Test Requirements	2697

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2698

A.9.2.41.1	Test Purpose and Environment	2698

A.9.2.41.2	Test parameters	2698

A.9.2.41.3	Test Requirements	2701

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2701

A.9.2.42.1	Test Purpose and Environment	2701

A.9.2.42.2	Test parameters	2701

A.9.2.42.3	Test Requirements	2704

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2704

A.9.2.43.1	Test Purpose and Environment	2704

A.9.2.43.2	Test parameters	2704

A.9.2.43.3	Test Requirements	2707

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2707

A.9.2.44.1	Test Purpose and Environment	2707

A.9.2.44.2	Test parameters	2707

A.9.2.44.3	Test Requirements	2709

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2709

A.9.2.45.1	Test Purpose and Environment	2709

A.9.2.45.2	Test parameters	2709

A.9.2.45.3	Test Requirements	2713

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2714

A.9.2.46.1	Test Purpose and Environment	2714

A.9.2.46.2	Test parameters	2714

A.9.2.46.3	Test Requirements	2718

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2719

A.9.2.47.1	Test Purpose and Environment	2719

A.9.2.47.2	Test parameters	2719

A.9.2.47.3	Test Requirements	2723

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2723

A.9.2.48.1	Test Purpose and Environment	2723

A.9.2.48.2	Test parameters	2724

A.9.2.48.3	Test Requirements	2728

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2729

A.9.2.49.1	Test Purpose and Environment	2729

A.9.2.49.2	Test parameters	2729

A.9.2.49.3	Test Requirements	2735

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2736

A.9.2.50.1	Test Purpose and Environment	2736

A.9.2.50.2	Test parameters	2736

A.9.2.50.3	Test Requirements	2742

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2742

A.9.2.51.1	Test Purpose and Environment	2742

A.9.2.51.2	Test parameters	2743

A.9.2.51.3	Test Requirements	2747

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2747

A.9.2.52.1	Test Purpose and Environment	2747

A.9.2.52.2	Test parameters	2747

A.9.2.52.3	Test Requirements	2750

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2751

A.9.2.53.1	Test Purpose and Environment	2751

A.9.2.53.2	Test parameters	2751

A.9.2.53.3	Test Requirements	2757

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2758

A.9.2.54.1	Test Purpose and Environment	2758

A.9.2.54.2	Test parameters	2758

A.9.2.54.3	Test Requirements	2764

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2764

A.9.2.55.1	Test Purpose and Environment	2764

A.9.2.55.2	Test parameters	2765

A.9.2.55.3	Test Requirements	2770

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2770

A.9.2.56.1	Test Purpose and Environment	2770

A.9.2.56.2	Test parameters	2770

A.9.2.56.3	Test Requirements	2775

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2775

A.9.2.57.1	Test Purpose and Environment	2775

A.9.2.57.2	Test parameters	2776

A.9.2.57.3	Test Requirements	2780

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2781

A.9.2.58.1	Test Purpose and Environment	2781

A.9.2.58.2	Test parameters	2781

A.9.2.58.3	Test Requirements	2785

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2786

A.9.2.59.1	Test Purpose and Environment	2786

A.9.2.59.2	Test parameters	2786

A.9.2.59.3	Test Requirements	2791

A.9.3	UTRAN FDD CPICH RSCP	2792

A.9.3.1	E-UTRAN FDD	2792

A.9.3.1.1	Test Purpose and Environment	2792

A.9.3.1.2	Parameters	2792

A.9.3.1.3	Test Requirements	2794

A.9.3.2 E-UTRAN TDD	2794

A.9.3.2.1	Test Purpose and Environment	2794

A.9.3.2.2	Parameters	2794

A.9.3.2.3	Test Requirements	2797

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2797

A.9.3.3.1	Test Purpose and Environment	2797

A.9.3.3.2	Parameters	2797

A.9.3.3.3	Test Requirements	2798

A.9.4	UTRAN FDD CPICH Ec/No	2799

A.9.4.1	E-UTRAN FDD	2799

A.9.4.1.1	Test Purpose and Environment	2799

A.9.4.1.2	Parameters	2799

A.9.4.1.3	Test Requirements	2801

A.9.4.2	E-UTRAN TDD	2802

A.9.4.2.1	Test Purpose and Environment	2802

A.9.4.2.2	Parameters	2802

A.9.4.2.3	Test Requirements	2804

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2805

A.9.4.3.1	Test Purpose and Environment	2805

A.9.4.3.2	Parameters	2805

A.9.4.3.3	Test Requirements	2806

A.9.5	UTRAN TDD measurement	2806

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2806

A.9.5.1.1	Test Purpose and Environment	2806

A.9.5.1.2	Test parameters	2806

A.9.5.1.3	Test Requirements	2808

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2808

A.9.5.2.1	Test Purpose and Environment	2808

A.9.5.2.2	Test parameters	2808

A.9.5.2.3	Test Requirements	2810

A.9.6	GSM Carrier RSSI	2811

A.9.6.1	E-UTRAN FDD	2811

A.9.6.1.1	Test Purpose and Environment	2811

A.9.6.1.2	Test Requirements	2812

A.9.6.2	E-UTRAN TDD	2812

A.9.6.2.1	Test Purpose and Environment	2812

A.9.6.2.2	Test Requirements	2814

A.9.7	UE Rx – Tx Time Difference	2815

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2815

A.9.7.1.1	Test Purpose and Environment	2815

A.9.7.1.2	Test parameters	2815

A.9.7.1.3	Test Requirements	2816

A.9.7.2	E-UTRA TDD	2816

A.9.7.2.1	Test Purpose and Environment	2816

A.9.7.2.2	Test parameters	2816

A.9.7.2.3	Test Requirements	2818

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2818

A.9.7.3.1	Test Purpose and Environment	2818

A.9.7.3.2	Test parameters	2818

A.9.7.3.3	Test Requirements	2821

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2821

A.9.7.4.1	Test Purpose and Environment	2821

A.9.7.4.2	Test Parameters	2821

A.9.7.4.3	Test Requirements	2824

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2824

A.9.7.5.1	Test Purpose and Environment	2824

A.9.7.5.2	Test parameters	2824

A.9.7.5.3	Test Requirements	2827

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2827

A.9.7.6.1	Test Purpose and Environment	2827

A.9.7.6.2	Test Parameters	2827

A.9.7.6.3	Test Requirements	2830

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2830

A.9.7.7.1	Test Purpose and Environment	2830

A.9.7.7.2	Test parameters	2830

A.9.7.7.3	Test Requirements	2831

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2832

A.9.7.8.1	Test Purpose and Environment	2832

A.9.7.8.2	Test parameters	2832

A.9.7.8.3	Test Requirements	2833

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2833

A.9.7.9.1	Test Purpose and Environment	2833

A.9.7.9.2	Test parameters	2833

A.9.7.9.3	Test Requirements	2834

A.9.8	RSTD	2835

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2835

A.9.8.1.1	Test Purpose and Environment	2835

A.9.8.1.2	Test Requirements	2837

A.9.8.1.2A	Test Requirements for UE Category 1bis	2837

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2838

A.9.8.2.1	Test Purpose and Environment	2838

A.9.8.2.2	Test Requirements	2841

A.9.8.2.2A	Test Requirements for UE Category 1bis	2841

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2842

A.9.8.3.1	Test Purpose and Environment	2842

A.9.8.3.2	Test Requirements	2844

A.9.8.3.2A	Test Requirements for UE Category 1bis	2844

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2845

A.9.8.4.1	Test Purpose and Environment	2845

A.9.8.4.2	Test Requirements	2847

A.9.8.4.2A	Test Requirements for UE Category 1bis	2847

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2848

A.9.8.5.1	Test Purpose and Environment	2848

A.9.8.5.2	Test Requirements	2850

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2850

A.9.8.6.1	Test Purpose and Environment	2850

A.9.8.6.2	Test Requirements	2853

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2853

A.9.8.7.1	Test Purpose and Environment	2853

A.9.8.7.2	Test Requirements	2854

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2854

A.9.8.8.1	Test Purpose and Environment	2854

A.9.8.8.2	Test Requirements	2855

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2855

A.9.8.9.1	Test Purpose and Environment	2855

A.9.8.9.2	Test Requirements	2856

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2856

A.9.8.10.1	Test Purpose and Environment	2856

A.9.8.10.2	Test Requirements	2857

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2857

A.9.8.11.1	Test Purpose and Environment	2857

A.9.8.11.2	Test Requirements	2858

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2858

A.9.8.12.1	Test Purpose and Environment	2858

A.9.8.12.2	Test Requirements	2859

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2859

A.9.8.13.1	Test Purpose and Environment	2859

A.9.8.13.2	Test Requirements	2860

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2860

A.9.8.14.1	Test Purpose and Environment	2860

A.9.8.14.2	Test Requirements	2867

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2867

A.9.8.15.1	Test Purpose and Environment	2867

A.9.8.15.2	Test Requirements	2873

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2873

A.9.8.16.1	Test Purpose and Environment	2873

A.9.8.16.2	Test Requirements	2876

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2876

A.9.8.17.1	Test Purpose and Environment	2876

A.9.8.17.2	Test Requirements	2879

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2879

A.9.8.18.1	Test Purpose and Environment	2879

A.9.8.18.2	Test Requirements	2882

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2882

A.9.8.19.1	Test Purpose and Environment	2882

A.9.8.19.2	Test Requirements	2885

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2885

A.9.8.20.1	Test Purpose and Environment	2885

A.9.8.20.2	Test Requirements	2888

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2889

A.9.8.21.1	Test Purpose and Environment	2889

A.9.8.21.2	Test Requirements	2892

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2893

A.9.8.22.1	Test Purpose and Environment	2893

A.9.8.22.2	Test Requirements	2896

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2897

A.9.8.23.1	Test Purpose and Environment	2897

A.9.8.23.2	Test Requirements	2900

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2900

A.9.8.24.1	Test Purpose and Environment	2900

A.9.8.24.2	Test Requirements	2904

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2905

A.9.8.25.1	Test Purpose and Environment	2905

A.9.8.25.2	Test Requirements	2908

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2909

A.9.8.26.1	Test Purpose and Environment	2909

A.9.8.26.2	Test Requirements	2911

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2911

A.9.8.27.1	Test Purpose and Environment	2911

A.9.8.27.2	Test Requirements	2914

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2914

A.9.8.28.1	Test Purpose and Environment	2914

A.9.8.28.2	Test Requirements	2917

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2917

A.9.8.29.1	Test Purpose and Environment	2917

A.9.8.29.2	Test Requirements	2921

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2922

A.9.8.30.1	Test Purpose and Environment	2922

A.9.8.30.2	Test Requirements	2925

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2926

A.9.8.31.1	Test Purpose and Environment	2926

A.9.8.31.2	Test Requirements	2930

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2930

A.9.8.32.1	Test Purpose and Environment	2930

A.9.8.32.2	Test Requirements	2933

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2933

A.9.8.33.1	Test Purpose and Environment	2933

A.9.8.33.2	Test Requirements	2936

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2936

A.9.8.34.1	Test Purpose and Environment	2936

A.9.8.34.2	Test Requirements	2939

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2939

A.9.8.35.1	Test Purpose and Environment	2939

A.9.8.35.2	Test Requirements	2942

A.9.9	RSRP and RSRQ on the serving cell	2942

A.9.9.1	FDD Intra frequency serving cell case	2942

A.9.9.1.1	Test Purpose and Environment	2942

A.9.9.1.2	Test parameters	2942

A.9.9.1.3	Test Requirements	2945

A.9.9.2	TDD Intra frequency serving cell case	2945

A.9.9.2.1	Test Purpose and Environment	2945

A.9.9.2.2	Test parameters	2945

A.9.9.2.3	Test Requirements	2947

A.9.10	SSTD	2947

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2947

A.9.10.1.1	Test Purpose and Environment	2947

A.9.10.1.2	Test parameters	2947

A.9.10.1.3	Test Requirements	2949

A.9.10.2	Void	2949

A.9.10.3	Void	2949

A.9.10.4	Void	2949

A.9.11 RSSI	2949

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2949

A.9.11.1.1	Test Purpose and Environment	2949

A.9.11.1.2	Test parameters	2949

A.9.11.1.3	Test Requirements	2952

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2952

A.9.11.2.1	Test Purpose and Environment	2952

A.9.11.2.2	Test parameters	2952

A.9.12	Channel occupancy	2955

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2955

A.9.12.1.1	Test Purpose and Environment	2955

A.9.12.1.2	Test parameters	2955

A.9.12.1.3	Test Requirements	2958

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2958

A.9.12.2.1	Test Purpose and Environment	2958

A.9.12.2.2	Test parameters	2958

A.9.12.2.3	Test Requirements	2961

A.9.13	RS-SINR	2961

A.9.13.1	FDD Intra-Frequency Case	2961

A.9.13.1.1	Test Purpose and Environment	2961

A.9.13.1.2	Test parameters	2961

A.9.13.1.3	Test Requirements	2964

A.9.13.2	TDD Intra-Frequency Case	2964

A.9.13.2.1	Test Purpose and Environment	2964

A.9.13.2.2	Test parameters	2964

A.9.13.2.3	Test Requirements	2967

A.9.13.3	FDD—FDD Inter frequency case	2967

A.9.13.3.1	Test Purpose and Environment	2967

A.9.13.3.2	Test parameters	2967

A.9.13.3.3	Test Requirements	2970

A.9.13.4	TDD—TDD Inter frequency case	2970

A.9.13.4.1	Test Purpose and Environment	2970

A.9.13.4.2	Test parameters	2970

A.9.13.4.3	Test Requirements	2974

A.9.13.5	FDD—TDD Inter frequency case	2975

A.9.13.5.1	Test Purpose and Environment	2975

A.9.13.5.2	Test parameters	2975

A.9.13.5.3	Test Requirements	2981

A.9.13.6	TDD—FDD Inter frequency case	2981

A.9.13.6.1	Test Purpose and Environment	2981

A.9.13.6.2	Test parameters	2981

A.9.13.6.3	Test Requirements	2985

A.9.14	Channel quality reporting accuracy	2985

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2985

A.9.14.1.1	Test Purpose and Environment	2985

A.9.14.1.2	Test parameters	2986

A.9.14.1.3	Test Requirements	2986

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2987

A.9.14.2.1	Test Purpose and Environment	2987

A.9.14.2.2	Test parameters	2987

A.9.14.2.3	Test Requirements	2988

A.10	Proximity-based Services in Any Cell Selection State	2988

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2988

A.10.1.1	Test Purpose and Environment	2988

A.10.1.2	Test Requirements	2989

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2990

A.10.2.1	Test Purpose and Environment	2990

A.10.2.2	Test Requirements	2991

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2991

A.10.3.1	Test Purpose and Environment	2991

A.10.3.2	Test Requirements	2993

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2994

A.10.4.1	Test Purpose and Environment	2994

A.10.4.2	Test Requirements	2996

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2996

A.11.1	V2V UE Transmission Timing Accuracy Test	2996

A.11.1.1	Test Purpose and Environment	2996

A.11.1.2	Test requirements	2997

A.11.2	Interruptions due to V2V sidelink communication	2997

A.11.2.1	Test Purpose and Environment	2997

A.11.2.2	Test Requirements	2999

A.12	2999

A.12.1	V2X UE Transmission Timing Accuracy Test	2999

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2999

A.12.1.1.1	Test Purpose and Environment	2999

A.12.1.1.2	Test requirements	3000

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3001

A.12.1.2.1	Test Purpose and Environment	3001

A.12.1.2.2	Test Requirements	3001

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3002

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3002

A.12.2.1.1	Test Purpose and Environment	3002

A.12.2.1.2	Test Requirements	3003

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3004

A.12.2.2.1	Test Purpose and Environment	3004

A.12.2.2.2	Test Requirements	3005

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3005

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3005

A.12.3.1.1	Test Purpose and Environment	3005

A.12.3.1.2	Test Requirements	3008

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3009

A.12.3.2.1	Test Purpose and Environment	3009

A.12.3.1.2	Test Requirements	3011

A.12.4	Congestion Control Measurement Test for V2X UE	3011

A.12.4.1	Test Purpose and Environment	3011

A.12.4.2	Test Requirements	3013

A.12.5	Interruptions due to V2X Sidelink Communication	3013

A.12.5.1	Test Purpose and Environment	3013

A.12.5.1	Test Requirements	3015

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3015

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3015

A.12.6.1.1	Test Purpose and Environment	3015

A.12.6.1.2	Test Requirements	3017

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3017

A.12.6.2.1	Test Purpose and Environment	3017

A.12.6.1.2	Test Requirements	3019

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3019

A.12.7.1	Test Purpose and Environment	3019

A.12.7.2	Test Requirements	3022

A.12.8	Interruptions due to V2X Carrier Aggregation	3023

A.12.8.1	Interruptions on a FDD PCell	3023

A.12.8.1.1	Test Purpose and Environment	3023

A.12.8.1.2	Test Requirements	3024

A.12.8.2	Interruptions on a TDD PCell	3024

A.12.8.2.1	Test Purpose and Environment	3024

A.12.8.2.2	Test Requirements	3026

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	3027

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3027

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3027

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3027

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3027

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3029

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3030

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3030

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3030

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3030

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3032

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3033

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3033

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3033

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3034

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3034

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3035

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3035

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3035

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3036

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3036

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3037

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3037

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3037

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3037

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3038

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3038

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3038

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3039

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3039

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3039

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3040

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3040

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3041

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3042

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3043

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3044

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3045

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3045

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3045

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3045

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3046

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3046

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3048

B.3	Conditions for measurements performance requirements for UE	3048

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3048

B.3.2	Void	3048

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3048

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3048

B.3.5	Conditions for UE Rx – Tx time difference	3048

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3049

B.3.7	Conditions for inter-frequency RSTD measurements	3049

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3049

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3049

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3049

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3050

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3050

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3050

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3051

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3051

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3051

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3052

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3052

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3052

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3052

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3052

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3052

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3052

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3052

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3052

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3053

B.3.21.1	Conditions for RSRP measurements	3053

B.3.21.2	Conditions for RSRQ measurements	3053

B.3.21.3	Conditions for CSI-RSRP measurements	3053

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3053

B.3.22.1	Conditions for RSRP measurements	3053

B.3.22.2	Void	3054

B.3.22.3	Conditions for CSI-RSRP measurements	3054

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3054

B.3.23.1	Conditions for RSRP measurements	3054

B.3.23.2	Conditions for RSRQ measurements	3054

B.3.23.3	Conditions for CSI-RSRP measurements	3054

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3055

B.3.24.1	Conditions for RSRP measurements	3055

B.3.24.2	Conditions for RSRQ measurements	3055

B.3.24.3	Conditions for CSI-RSRP measurements	3055

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3055

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3055

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3055

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3056

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3056

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3056

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3056

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3056

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3057

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3057

B.4	RRM Requirements Exceptions	3057

B.4.1	General	3057

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3057

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3057

B.4.3.1	Inter-band carrier aggregation	3057

B.4.3.2	Intra-band non-contiguous carrier aggregation	3057

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3058

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3058

B.5.1	Conditions for S-RSRP Accuracy Requirements	3058

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3058

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3059

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3059

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3060

B.6	Conditions for V2X	3060

B.6.1	Test parameters for GNSS signals	3060

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3060

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3060

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3061

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3061

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3061

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3061

Annex C (informative):	Change history:	3063