Annex B (normative):
Conditions for RRM requirements applicability for operating bands

# B.1	Conditions for NR RRC_IDLE state mobility

## B.1.1	Introduction

In Annex B.1, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurements procedures and requirements in clause 4,

-	UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in clause 4.

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.1.2-2:

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

## B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection

This clause defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.2-1 for FR1 NR intra-frequency cell re-selection shall also apply for FR1 NR inter-frequency cells in this clause.

The conditions defined in Table B.1.2-2 for FR2 NR intra-frequency cell re-selection shall also apply for FR2 NR inter-frequency cells in this clause.

# B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state

## B.2.1	Introduction

### B.2.1.1	General

In Annex B.2, the following conditions are specified:

-	The conditions for RRC connection release with redirection to NR requirements in clause 6.2.3.2.1,

-	The conditions for UE transmit timing adjustment in clause 7.1

-	UE conditions which shall apply for UE intra-frequency measurements procedures and requirements in clause 9,

UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in clause 9,

-	UE conditions which shall apply for UE intra-frequency measurements performance requirements in clause 10,

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in clause 10.

### B.2.1.2	Derivation of Minimum SSB_RP values for FR1

[FFS]

### B.2.1.3	Derivation of Minimum SSB_RP values for FR2

#### B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival

Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on Reference sensitivity for the Operating band and for the UE power class, taking a baseline of UE Power class 3 in Band n260 with 50 MHz channel bandwidth.

Minimum SSB_RP = Reference sensitivity PC3, n260, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ΣMBP

where:

Reference sensitivity PC3, n260, 50MHz is the reference sensitivity value in dBm specified for power class 3 in Band n260 for 50 MHz Channel bandwidth in Table 7.3.2.3-1 of TS 38.101-2 [19];

Y is the gain difference between fine and rough beams, which is defined in Table B.2.1.3.1-1;

Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction

| Value “Y” in dB, for each UE power class |  |  |  |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| FFS | 9.0 | 7.0 | FFS |

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [19] Table 5.3.2-1, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

ΣMBP is the UE multi-band relaxation factor value in dB specified in TS 38.101-2 [19] clause 6.2.1.

The calculated Minimum SSB_RP value for the baseline of UE power class 3 in Band n260 is (-109.5+ΣMBP) dBm/120kHz for intra-frequency measurements and (-107.5+ΣMBP) dBm/120kHz for inter-frequency measurements.

The following methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is used:

For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = -109.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +ΣMBP,

For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = -107.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +ΣMBP.

##### B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage

Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on EIS spherical coverage for the Operating band and for the UE power class, taking a baseline of UE power class 3 in Band n260 with 50 MHz channel bandwidth.

Minimum SSB_RP = EIS spherical coverage PC3, n260, 50MHz +Z -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ΣMBS

where:

EIS spherical coverage PC3, n260, 50MHz is the EIS spherical coverage value in dBm specified for power class 3 in Band n260 for 50MHz Channel bandwidth in TS 38.101-2 [19] Table 7.3.4.3-1;

Z is the gain difference between fine and rough beams, and is defined in Table B.2.1.3.2-1;

Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions

| Value “Z” in dB, for each UE power class |  |  |  |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| FFS | 9.0 | 7.0 | FFS |

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [19] Table 5.3.2-1, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

ΣMBS is the UE multi-band relaxation factor value in dB specified in TS 38.101-2 [19] clause 6.2.1.

The calculated Minimum SSB_RP value for the baseline of UE power class 3 in Band n260 is (-96.9+ΣMBS) dBm/120kHz for intra-frequency measurements and is (-94.9+ΣMBS) dBm/120kHz for inter-frequency measurements.

The following methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is used:

For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = (-103.9+ΣMBS +Z) dBm/120 kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Z PC_X – Z PC3 +ΣMBS,

For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = (-101.9+ΣMBS +Z) dBm/120 kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Z PC_X – Z PC3 +ΣMBS

### B.2.1.4	Gain to SS-RSRP measurement point for FR1

In FR1 conducted requirements are specified at the UE antenna connector, which is also the SS-RSRP measurement point.

### B.2.1.5	Gain to SS-RSRP measurement point for FR2

#### B.2.1.5.1	Gain to SS-RSRP measurement point for Rx Beam Peak angle of arrival

In clause 5.1.1 of TS 38.215 [4] SS-RSRP is defined to be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reference point for requirement parameters from the UE perspective is the input of the UE antenna array. The gain “G” relates the combined signal from antenna elements corresponding to a given receiver branch to the reference point for requirement parameters.

The gain “G” affects absolute signal level values reported by the UE.

Figure B.2.1.5.1-1: Gain and Reference point for requirement parameters

The gain range for each power class is specified in Table B.2.1.5.1-1.

Table B.2.1.5.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Minimum, dBi | FFS | FFS | -10 | FFS |
| Maximum, dBi | FFS | FFS | +20 | FFS |

Gain range in spherical coverage directions may be lower than in Rx beam peak direction, according to the difference between the EIS spherical coverage value specified in TS 38.101-2 [19] clause 7.3.4 and the Reference sensitivity level specified in TS 38.101-2 [19] clause 7.3.2.

## B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.2-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

## B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.3-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1, and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

## B.2.4	Conditions for NR L1-RSRP reporting

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.1-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

### B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  | SCSCSI-RS = 120 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.2-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

## B.2.5	Conditions for RRC connection release with redirection to NR

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.5-2: Conditions for RRC connection release with redirection to NR in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -114.3 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.5.2-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine

## B.2.6	Conditions for UE transmit timing

### B.2.6.1	Conditions for SSB based UE transmit timing

This clause defines the following conditions for UE transmit timing adjustment performed based on SSBs: SSB_RP and SSB Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.1-1 for FR1 SSB.

Table B.2.6.1-1: Conditions for SSB based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=15 kHz | SCSSSB=30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The conditions are defined in Table B.2.6.1-2 for FR2 SSB.

Table B.2.6.1-2: Conditions for SSB based UE transmit timing in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOET 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.6.1-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

### B.2.6.2	Void

# B.3	RRM Requirements Exceptions

## B.3.1	Introduction

Annex B.3 covers exceptions for side conditions based on receiver sensitivity for CA, DC, and SUL.

## B.3.2	Receiver sensitivity relaxation for CA

### B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1

For a UE supporting inter-band carrier aggregation configuration with uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in clause 7.3A.3 of TS 38.101-1 [18], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting CA configuration in FR1, the requirement in this clause applies for both SC and CA operation.

### B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1

#### B.3.2.2.1	Inter-band carrier aggregation

For a UE configured with inter-band carrier aggregation with active uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in clause 7.3A.3 of TS 38.101-1 [18], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

If the relaxation Δ specified in this clause applies, then the relaxation specified in clause B.3.2.1 should not be applied.

#### B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA

In this clause, requirements exceptions are described for the UE configured with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same CA configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in clause 7.3.2 of TS 38.101-1 [18], and L2 is the reference sensitivity level based on the requirements in clause 7.3A.4 of TS 38.101-1 [18], when the following conditions are fulfilled,

-	corresponding downlink component carriers on different NR bands are configured with CA and active,

-	the upling is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in clause 7.3A.4 of TS 38.101-1 [18], and

-	the exception requirements specified in clause 7.3A.4 of TS 38.101-1 [18] apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in clause B.3.2.1 should not be applied.

#### B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

In this clause, requirements exceptions are described for the UE with an inter-band carrier aggregation with uplink assigned to two NR bands.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in clause 7.3.2 of TS 38.101-1 [18], and L2 is the reference sensitivity level based on the requirements in clause 7.3A.5 of TS 38.101-1 [18], when the following conditions are fulfilled,

-	corresponding downlink component carriers on different bands are configured with CA and active,

-	uplinks are assigned to two NR bands,

-	the exception requirements specified in clause 7.3A.5 of TS 38.101-1 [18] apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in clause B.3.2.1 should not be applied.

### B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2

### B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2

#### B.3.2.4.1	Intra-band contiguous carrier aggregation

For a UE configured with intra-band contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in clause 7.3A.2.1 of TS 38.101-2 [19] depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

#### B.3.2.4.2	Intra-band non-contiguous carrier aggregation

For a UE configured with intra-band non-contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in clause 7.3A.2.1 of TS 38.101-2 [19] depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

## B.3.3	Receiver sensitivity relaxation for DC

### B.3.3.1	Receiver sensitivity relaxation for EN-DC

Editor’s note: TBD

### B.3.3.2	Receiver sensitivity relaxation for NE-DC

Editor’s note: TBD

## B.3.4	Receiver sensitivity relaxation for SUL

### B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1

For a UE supporting a SUL configuration in FR1, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in clause 7.3C.3 of TS 38.101-1 [18], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting a SUL configuration in FR1, the requirement in this clause applies for both SC and SUL operation.

### B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1

#### B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL

In this clause, requirements exceptions are described for the UE with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same SUL configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in clause 7.3.2 of TS 38.101-1 [18], and L2 is the reference sensitivity level based on the requirements in clause 7.3C.2 of TS 38.101-1 [18], when the following conditions are fulfilled,

-	a downlink component carrier is configured in NR band and is active,

-	the upling is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in clause 7.3C.2 of TS 38.101-1 [18], and

-	the exception requirements specified in clause 7.3C.2 of TS 38.101-1 [18] apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in clause B.3.4.1 should not be applied.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-05 | RAN4#83 | R4-1706324 |  |  |  | Specification skeleton | 0.0.1 |
| 2017-09 |  |  |  |  |  | Email approved | 0.1.0 |
| 2017-09 | RAN4-NR AH #3 | R4-1709413 |  |  |  | Capture TPs approved in the meeting | 0.2.0 |
| 2017-10 | RAN4#84-Bis | R4-1711985 |  |  |  | Capture TPs approved in the meeting | 0.3.0 |
| 2017-12 | RAN4#85 | R4-1714546 |  |  |  | Capture TPs approved in RAN4#85 | 0.4.0 |
| 2017-12 | RAN#78 | RP-172407 |  |  |  | v1.0.0 submitted for plenary approval | 1.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2018-03 | RAN#79 | RP-180264 | 0032 |  | B | CR to TS38.133 | 15.1.0 |
| 2018-06 | RAN#80 | RP-181075 | 0037 |  | B | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4 #86bis and RAN4 #87 | 15.2.0 |
| 2018-09 | RAN#81 | RP-181896 | 0043 |  | B | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4-AH-1807 and RAN4 #88 | 15.3.0 |
| 2018-12 | RAN#82 | RP-182763 | 0057 | 3 | B | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4-88bis and RAN4-89 | 15.4.0 |
| 2019-03 | RAN#83 | RP-190569 | 0064 | 1 | B | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4#90 | 15.5.0 |
| 2019-06 | RAN#84 | RP-191240 | 0072 | 1 | F | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4#90bis and RAN4#91 | 15.6.0 |
| 2019-06 | RAN#84 | RP-191248 | 0066 |  | B | Introduction of band n48 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191242 | 0067 |  | B | Introduction of band n14 - CR to TS 38.133 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191246 | 0068 |  | B | Introduction of band n30 - CR to TS 38.133 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191244 | 0069 |  | B | introduce n18 into TS38.133 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191250 | 0070 | 1 | B | n65 introduction to 38.133 | 16.0.0 |
| 2019-09 | RAN#85 | RP-192034 | 0077 |  | B | n29 introduction to 38.133 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192022 | 0085 |  | A | CR to TS 38.133: Implementation of endorsed draft CRs from RAN4#92 (Rel-16) - Mirrors changes in R4-1910356 for Rel-15 TS 38.133 | 16.1.0 |
| 2019-12 | RAN#86 | RP-192997 | 0093 |  | A | Specification of UE antenna gain range | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0095 | 1 | A | Add RRM Test case setup for 1 AoA in Rx beam peak and 1 in non Rx beam peak | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0097 |  | A | Update of Parameters, Test case A.7.7.1.1 FR2 Intra-frequency SS-RSRP accuracy | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0099 |  | A | Update of Parameters, Test case A.5.7.1.1 FR2 Intra-frequency SS-RSRP accuracy | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0101 |  | A | Update of Parameters, Test case A.7.7.1.2 FR2 Inter-frequency SS-RSRP accuracy | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0103 |  | A | Update of Parameters, Test case A.5.7.1.2 FR2 Inter-frequency SS-RSRP accuracy | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0105 |  | A | Correction to Random access test case in FR1 for PSCell in EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0107 |  | A | CR on handover 38.133 - R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0112 | 1 | A | CR on the BWP switch test cases EN-DC FR1 (section A.4.5.6) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0113 | 1 | A | CR on the BWP switch test cases EN-DC FR2 (section A.5.5.6) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0114 | 1 | A | CR on the BWP switch test cases SA FR1 (section A.6.5.6) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0115 | 1 | A | CR on the BWP switch test cases SA FR2 (section A.7.5.6) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0117 |  | A | CR to TS38.133 on correction for BWP switching with SCS changing (Section 8.2.1.2.7, 8.2.2.2.5 and 8.6.2) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0121 |  | A | CR on handover RRM requirement (section 6.1.1.5) (R16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0123 |  | A | CR on test cases for EN-DC FR2 inter-frequency measurement (section A.5.6.2) (R16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0127 | 1 | A | CR on test cases for Redirection from NR in FR2 to NR in FR2 (section A.7.3.2.3) (R16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0129 | 1 | A | CR on test cases for FR2 handover (section A.7.3.1) (R16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0131 |  | A | CR to 38.133 on TCI state switching (Section 8.10) (R16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193009 | 0133 |  | F | CR on measurement gap applicability requirement for SRVCC | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0137 |  | A | CR on TC with monitoring PDCCH not in first 3 OFDM symbols R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193021 | 0139 |  | F | CR to add n90 in the NR operating bands in FR1 (3.5.2) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0148 | 1 | A | CR on inter-RAT measurement in TS38.133 (section 9.4.2, 9.4.3) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0151 |  | A | CR to 38.133 R16 Add the missing units to DRX cycle values (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193005 | 0152 | 1 | B | CR for Abbreviations for cross link interference (section 3) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193005 | 0153 | 1 | B | CR for cross link interference measurements (section 9) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0156 |  | A | CR on NR MTTD and MRTD definition for R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0157 | 1 | A | Editorial correction for SCell activation and deactivation delay | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0159 |  | A | CR for SCell activation delay in FR2 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0161 |  | A | CR for scheduling restriction due to L1-RSRP measurement | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0167 |  | A | CR on SSB setting for new gap and SMTC setting (Section A.3.10) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0169 |  | A | CR on TS38.133 for EN-DC SS-SINR tests with PSCell in FR1 (Section A.4.7.3) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0171 |  | A | CR on TS38.133 for SA SS-SINR tests with PCell in FR1 (Section A.6.7.3) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0185 |  | A | CR on cell-reselection test cases for NR SA FR2 R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0187 |  | A | endorsed CR on intra-frequency measurement and reporting for EN-DC FR2 R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0189 |  | A | endorsed CR on intra-frequency measurement and reporting for NR SA FR2 R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0191 |  | A | endorsed CR on RLM scheduling restrictions for EN-DC FR2 R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0193 |  | A | endorsed CR on RLM scheduling restrictions for NR SA FR2 R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0201 |  | A | Correction to PRACH configuration index in test cases_r16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193009 | 0205 |  | B | CR on UMTS inter-RAT measurement requirements | 16.2.0 |
| 2019-12 | RAN#86 | RP-193009 | 0206 |  | B | CR on CSSF for SRVCC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193009 | 0207 |  | B | CR on measurement capability for NR- UMTS for SRVCC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0209 |  | A | Correction on the TCI state switching (section 8.10) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0219 |  | A | CR for 38133 editorial for section 8.1,8.8,8.9,8.10,8.11 in Rel-16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0220 |  | A | CR for 38133 editorial for section 8.5 in Rel-16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0221 |  | A | CR for 38133 editorial for section 9.3 in Rel-16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0222 |  | A | CR on 38133 for removal the duplicated reference in section 2 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0223 |  | A | CR on 38133 for section 11 in Rel-16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0225 | 1 | A | CR on TC of UE transmit timing (A.4.4.1.1, A.5.4.1.1, A.6.4.1.1, A.7.4.1.1) Rel-16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0230 |  | A | Update on requirements related to inter-band EN-DC and NE-DC synchronous requirements | 16.2.0 |
| 2019-12 | RAN#86 | RP-193008 | 0231 | 1 | B | MRTD and MTTD requirements for asynchronous NR-NR DC | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0233 | 1 | A | Editorial corrections to measurement accuracy tests | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0235 |  | A | Corrections to SS-RSRQ and SS-SINR OTA tests with SA | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0237 | 1 | A | Corrections to SS-RSRQ and SS-SINR OTA tests with EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0239 | 1 | A | Editorial corrections to section 9.2 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193009 | 0240 |  | B | Introduction of handover requirements for SRVCC in section 6.1.2 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0242 |  | A | Corrections to band applicability of measurement accuracy tests | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0244 |  | A | Introduction of bandwidth limited OCNG for OTA testing | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0248 |  | A | Corrections to test cases for SA FR2 inter-frequency measurement (section A.7.6.2) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0250 |  | A | CR to 38.133 NR reporting criteria | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0264 |  | A | CR on correcting CSI-RS based BFD and link recovery tests for EN-DC in FR1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0266 |  | A | CR on correcting CSI-RS based BFD and link recovery tests for SA in FR1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0268 |  | A | CR on correcting CSI-RS based BFD and link recovery tests for EN-DC in FR2 | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0270 |  | A | CR on correcting CSI-RS based BFD and link recovery tests for SA in FR2 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193004 | 0274 | 1 | B | CR on introducing L1-SINR mapping in TS38.133 R16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0276 |  | A | CR on delay uncertainty of RRC Release with redirection requirements in TS 38.133 (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0278 |  | A | CR on known condition of PSCell addition requirement in NE-DC (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0280 |  | A | CR on known condition of PSCell addition requirement in NR DC (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0282 |  | A | CR on RRC Re-establishment requirements in TS 38.133 (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0284 |  | A | CR on scope of interruption requirements of EN-DC in TS 38.133 (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0286 |  | A | CR on scope of MTTD requirements in TS 38.133 (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0288 |  | A | CR on SSB-based RLM test case for EN-DC FR1 (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192994 | 0290 |  | A | CR on SSB-based RLM test case for NR SA FR1 (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0292 |  | A | Editorial CR on section 8.2 (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0296 |  | A | CR on NR inter-frequency identification (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0298 |  | A | CR on NR intra-frequency measurements (Cat A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0312 |  | A | Correction on CSSF within measurement gap (section 9.1.5.2) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0314 |  | A | CR on RLM scheduling restriction (section 8.1.7) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0316 |  | A | CR on SCell activation requirements (section 8.3.2) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0318 |  | A | CR to add QCL definition (section 3.6) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0320 |  | A | CR on power offset in TRS RMC (A.3.17) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0322 |  | A | CR to introduce new PDCCH RMC (A.3.1.3.2) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0324 |  | A | Maintenance CR for measurement accuracy (section 10.1) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0326 |  | A | FR1 CSI-RS RLM test OOS/IS non-DRX for EN-DC (section A.4.5.1) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0328 |  | A | FR2 CSI-RS RLM test OOS/IS non-DRX for EN-DC (section A.4.5.1) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0330 |  | A | FR1 CSI-RS RLM test OOS/IS non-DRX for SA (section A.6.5.1) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0332 |  | A | FR2 CSI-RS RLM test OOS/IS non-DRX for SA (section A.6.5.1) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0334 |  | A | L1-RSRP delay test FR1 EN-DC (section A.4.6.3) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0336 |  | A | L1-RSRP delay test FR2 EN-DC (section A.5.6.3) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0338 |  | A | L1-RSRP delay test FR1 SA (section A.6.6.4) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192997 | 0340 |  | A | L1-RSRP delay test FR2 SA (section A.7.6.3) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0344 |  | A | L1-RSRP accuracy test FR2 EN-DC (section A.5.7.4) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192996 | 0346 |  | A | L1-RSRP accuracy test FR2 SA (section A.7.7.4) (cat-A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193005 | 0347 | 1 | B | CR to introduce CLI measurement accuracy requirements | 16.2.0 |
| 2019-12 | RAN#86 | RP-193008 | 0348 |  | B | CR on measurement gap interruption due to async NR-DC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193008 | 0349 |  | B | CR on Interruptions at PSCell/SCell addition/release in async NR-DC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193008 | 0350 |  | B | Introducing euCA related interruption requirements for EN-DC in 38.133 (section 8.2.1) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193008 | 0351 |  | B | Introducing euCA related interruption requirements for NE-DC in 38.133 (section 8.2.3) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193008 | 0352 | 1 | B | CR on direct SCell activation delay | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0358 |  | A | CR 38.133 (8.3.2) Amendment of requirements depending on T_SMTC_Max | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0360 |  | A | CR 38.133 (8.3.3) Correction of SCell deactivation delay | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0362 | 1 | A | CR 38.133 (A.7.5.7) TCs for PSCell addition and release delay | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0366 |  | A | CR to TS 38.133: New common section with OTA related definitions for FR2 testing (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0368 |  | A | CR to TS 38.133: Configuration of NR FR1 cell in NR FR1-FR2 tests (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0370 |  | A | CR to TS 38.133: Clarificatins to Antenna Configurations for FR2 (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0372 |  | A | CR to TS 38.133: Corrections to CORESET RMCs (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192995 | 0374 |  | A | CR to TS 38.133: Corrections to FR2 test configurations (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0376 | 1 | A | Editorial updates (section 9.4) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0378 |  | A | Correction in interruption requirements (section 8.2) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193042 | 0380 | 1 | A | Editorial updates (Annex B) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0382 |  | A | CR on 38133 for MRTD and MTTD in intra-band EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193039 | 0390 |  | A | Correction to the starting point of the DRX cycle length interval | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0391 |  | A | CR for MAC-CE based TCI State switch for ENDC (Section A.5.5.8) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0392 |  | A | CR for MAC-CE based TCI State switch for NR SA (Section A.7.5.7) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0393 |  | A | CR for RRC based TCI State switch for NR SA (Section A.7.5.7) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192993 | 0394 |  | A | CR for RRC based TCI State switch for EN-DC (Section A.5.5.8) | 16.2.0 |
| 2019-12 | RAN#86 | RP-192992 | 0395 |  | A | CR for FR1 handover test cases (Section A.6.3.1.1, A.6.3.1.2, A.6.3.1.3) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193041 | 0396 |  | A | CR on MTTD for intra-band EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193040 | 0398 |  | A | CR on corrections on NR intra frequency measurement reporting requirements (Section 9.2.4) | 16.2.0 |