| 3GPP TS 36.101 V16.5.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	32

1	Scope	34

2	References	34

3	Definitions, symbols and abbreviations	34

3.1	Definitions	34

3.2	Symbols	36

3.3	Abbreviations	39

4	General	40

4.1	Relationship between minimum requirements and test requirements	40

4.2	Applicability of minimum requirements	41

4.3	Void	41

4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE)	41

4.4	RF requirements in later releases	43

5	Operating bands and channel arrangement	43

5.1	General	43

5.2	Void	43

5.3	Void	43

5.4	Void	43

5.5	Operating bands	43

5.5A	Operating bands for CA	46

5.5B	Operating bands for UL-MIMO	65

5.5C	Operating bands for Dual Connectivity	65

5.5D	Operating bands for ProSe	68

5.5E	Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis	69

5.5F	Operating bands for category NB1 and NB2	69

5.5G	Operating bands for V2X Communication	69

5.6	Channel bandwidth	70

5.6.1	Channel bandwidths per operating band	71

5.6A	Channel bandwidth for CA	74

5.6A.1	Channel bandwidths per operating band for CA	76

5.6B	Channel bandwidth for UL-MIMO	184

5.6B.1	Void	184

5.6C	Channel bandwidth for Dual Connectivity	184

5.6C.1	Void	184

5.6D	Channel bandwidth for ProSe	184

5.6D.1	Channel bandwidths per operating band for ProSe	184

5.6F	Channel bandwidth for category NB1 and NB2	185

5.6G	Channel bandwidth for V2X Communication	186

5.6G.1	Channel bandwidths per operating band for V2X Communication	186

5.7	Channel arrangement	188

5.7.1	Channel spacing	188

5.7.1A	Channel spacing for CA	188

5.7.1F	Channel spacing for category NB1 and NB2	189

5.7.2	Channel raster	189

5.7.2A	Channel raster for CA	189

5.7.2F	Channel raster for category NB1 and NB2	189

5.7.3	Carrier frequency and EARFCN	189

5.7.3F	Carrier frequency and EARFCN for category NB1 and NB2	192

5.7.4	TX–RX frequency separation	192

5.7.4A	TX–RX frequency separation for CA	193

5.7.4E	TX–RX frequency separation for category M1 and M2	193

5.7.4F	TX–RX frequency separation for category NB1 and NB2	194

6	Transmitter characteristics	194

6.1	General	194

6.2	Transmit power	194

6.2.1	Void	194

6.2.2	UE maximum output power	194

6.2.2A	UE maximum output power for CA	197

6.2.2B	UE maximum output power for UL-MIMO	202

6.2.2C	 Void	205

6.2.2D	UE maximum output power for ProSe	205

6.2.2E	UE maximum output power for Category M1 and M2 UE	205

6.2.2F	UE maximum output power for category NB1 and NB2	206

6.2.2G	UE maximum output power for V2X Communication	207

6.2.3	UE maximum output power for modulation / channel bandwidth	209

6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA	210

6.2.3B	UE maximum output power for modulation / channel bandwidth for UL-MIMO	215

6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe	215

6.2.3E	UE maximum output power for modulation / channel bandwidth for category M1 and M2	216

6.2.3F	UE maximum output power for modulation / channel bandwidth for category NB1 and NB2	218

6.2.3G	UE maximum output power for modulation / channel bandwidth for V2X Communication	218

6.2.3G.1	MPR for Power class 3 V2X UE	218

6.2.3G.2	MPR for Power class 2 V2X UE	220

6.2.4	UE maximum output power with additional requirements	220

6.2.4A	UE maximum output power with additional requirements for CA	239

6.2.4A.1	A-MPR for CA_NS_01 for CA_1C	241

6.2.4A.2	A-MPR for CA_NS_02 for CA_1C	241

6.2.4A.3	A-MPR for CA_NS_03 for CA_1C	242

6.2.4A.4	A-MPR for CA_NS_04	243

6.2.4A.5	A-MPR for CA_NS_05 for CA_38C	245

6.2.4A.6	A-MPR for CA_NS_06	246

6.2.4A.7	A-MPR for CA_NS_07	247

6.2.4A.8	A-MPR for CA_NS_08	248

6.2.4A.9	Void	249

6.2.4A.10	A-MPR for CA_NS_10	249

6.2.4B	UE maximum output power with additional requirements for UL-MIMO	252

6.2.4D	UE maximum output power with additional requirements for ProSe	253

6.2.4E	UE maximum output power with additional requirements for category M1 and M2 UE	253

6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE	262

6.2.4G	UE maximum output power with additional requirements for V2X Communication	263

6.2.5	Configured transmitted power	265

6.2.5A	Configured transmitted power for CA	296

6.2.5B	Configured transmitted power for UL-MIMO	300

6.2.5C	Configured transmitted power for Dual Connectivity	300

6.2.5D	Configured transmitted power for ProSe	302

6.2.5F	Configured transmitted Power for category NB1 and NB2	303

6.2.5G	Configured transmitted power for V2X Communication	304

6.3	Output power dynamics	308

6.3.1	(Void)	308

6.3.2	Minimum output power	308

6.3.2.1	Minimum requirement	308

6.3.2A	 UE Minimum output power for CA	308

6.3.2A.1	Minimum requirement for CA	309

6.3.2B	 UE Minimum output power for UL-MIMO	309

6.3.2B.1	Minimum requirement	309

6.3.2C	 Void	309

6.3.2D	UE Minimum output power for ProSe	309

6.3.2F	UE Minimum output power for category NB1 and NB2	310

6.3.2G	UE Minimum output power for V2X Communication	310

6.3.3	Transmit OFF power	310

6.3.3.1.	Minimum requirement	310

6.3.3A	 UE Transmit OFF power for CA	311

6.3.3A.1	Minimum requirement for CA	311

6.3.3B	 UE Transmit OFF power for UL-MIMO	311

6.3.3B.1	Minimum requirement	311

6.3.3D	Transmit OFF power for ProSe	312

6.3.3F	Transmit OFF power for category NB1 and NB2	312

6.3.3G	Transmit OFF power for V2X Communication	312

6.3.4	ON/OFF time mask	313

6.3.4.1	General ON/OFF time mask	313

6.3.4.2	PRACH and SRS time mask	314

6.3.4.2.1	PRACH time mask	314

6.3.4.2.2	SRS time mask	315

6.3.4.3	Slot / Sub frame boundary time mask for subframe TTI	317

6.3.4.4	PUCCH / PUSCH / SRS time mask for subframe TTI	319

6.3.4.5	Symbol / Subslot boundary time mask for subslot TTI	321

6.3.4.6	Subslot PUCCH / subslot PUSCH / SRS time mask for subslot TTI	322

6.3.4.7	Symbol / Slot boundary time mask for slot TTI	325

6.3.4.8	Slot PUCCH / slot PUSCH / SRS time mask for slot TTI	326

6.3.4.9	Consecutive subslot and slot TTI or consecutive subslot and subframe TTI  time mask	326

6.3.4.10	Consecutive subframe and subslot TTI or consecutive slot and subslot TTI time mask	326

6.3.4.11	Consecutive TTI and slot TTI or consecutive slot TTI and TTI time mask	327

6.3.4A	ON/OFF time mask for CA	328

6.3.4B	ON/OFF time mask for UL-MIMO	328

6.3.4D	ON/OFF time mask for ProSe	328

6.3.4D.1	 General time mask for ProSe	328

6.3.4D.2	 PSSS/SSSS time mask	329

6.3.4D.3	 PSSS / SSSS / PSBCH time mask	330

6.3.4D.4	 PSSCH / SRS time mask	330

6.3.4F	ON/OFF time mask for category NB1 and NB2	330

6.3.4F.1	General ON/OFF time mask	330

6.3.4F.2	NPRACH time mask	331

6.3.4G	ON/OFF time mask for V2X Communication	331

6.3.4G.1	PSSS / SSSS / PSBCH time mask	332

6.3.5	Power Control	332

6.3.5.1	Absolute power tolerance	332

6.3.5.1.1	 Minimum requirements	332

6.3.5.2	Relative Power tolerance	333

6.3.5.2.1	Minimum requirements	333

6.3.5.3	Aggregate power control tolerance	334

6.3.5.3.1	Minimum requirement	334

6.3.5A	Power control for CA	334

6.3.5A.1	Absolute power tolerance	334

6.3.5A.1.1	 Minimum requirements	334

6.3.5A.2	Relative power tolerance	335

6.3.5A.2.1	Minimum requirements	335

6.3.5A.3	Aggregate power control tolerance	335

6.3.5A.3.1	 Minimum requirements	335

6.3.5B	Power control for UL-MIMO	336

6.3.5D	Power Control for ProSe	336

6.3.5D.1	Absolute power tolerance	336

6.3.5E	Power control for category M1 and M2	336

6.3.5E.1	Absolute power tolerance	336

6.3.5E.2	Relative Power tolerance	336

6.3.5E.3	Aggregate power control tolerance	337

6.3.5E.3.1	Minimum requirement	337

6.3.5F	Power Control for category NB1 and NB2	337

6.3.5F.1	Absolute power tolerance	338

6.3.5F.2	Relative power tolerance	338

6.3.5F.3	Aggregate power control tolerance for category NB1 and NB2	339

6.3.5F.3.1	Minimum requirement	339

6.3.5G	Power Control for V2X Communication	339

6.3.5G.1	Absolute power tolerance	339

6.4	Void	340

6.5	Transmit signal quality	340

6.5.1	Frequency error	340

6.5.1A 	Frequency error for CA	340

6.5.1B	Frequency error for UL-MIMO	340

6.5.1D	Frequency error for ProSe	340

6.5.1E	Frequency error for UE category M1 and M2	340

6.5.1F	Frequency error for UE category NB1 and NB2	340

6.5.1G	Frequency error for V2X Communication	341

6.5.2	Transmit modulation quality	341

6.5.2.1	 Error Vector Magnitude	341

6.5.2.1.1	Minimum requirement	342

6.5.2.2	 Carrier leakage	342

6.5.2.2.1	 Minimum requirements	342

6.5.2.3	In-band emissions	343

6.5.2.3.1	 Minimum requirements	343

6.5.2.4	 EVM equalizer spectrum flatness	344

6.5.2.4.1	Minimum requirements	344

6.5.2A	Transmit modulation quality for CA	345

6.5.2A.1	Error Vector Magnitude	346

6.5.2A.2	Carrier leakage for CA	346

6.5.2A.2.1	Minimum requirements	346

6.5.2A.3	In-band emissions	346

6.5.2A.3.1	Minimum requirement for CA	346

6.5.2B	Transmit modulation quality for UL-MIMO	349

6.5.2B.1	Error Vector Magnitude	349

6.5.2B.2	Carrier leakage	349

6.5.2B.3	In-band emissions	349

6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO	349

6.5.2D	Transmit modulation quality for ProSe	349

6.5.2D.1	Error Vector Magnitude	349

6.5.2D.2	Carrier leakage	350

6.5.2D.3	In-band emissions	350

6.5.2D.4	EVM equalizer spectrum flatness for ProSe	350

6.5.2E	Transmit modulation quality for category M1 and M2	350

6.5.2E.1	Error Vector Magnitude	350

The Error Vector Magnitude is defined in section 6.5.2.1.	350

6.5.2E.2	Carrier leakage	350

6.5.2E.2.1	Minimum requirements	350

6.5.2E.3	In-band emissions	350

6.5.2E.3.1	Minimum requirements	350

6.5.2F	Transmit modulation quality for Category NB1 and NB2	352

6.5.2F.1	Error Vector Magnitude	352

6.5.2F.2	Carrier leakage	352

6.5.2F.3	In-band emissions	352

6.5.2G	Transmit modulation quality for V2X Communication	353

6.5.2G.1	Error Vector Magnitude	353

6.5.2G.2	Carrier leakage	354

6.5.2G.3	In-band emissions	354

6.5.2G.4	EVM equalizer spectrum flatness	354

6.6	Output RF spectrum emissions	354

6.6.1	Occupied bandwidth	354

6.6.1.1	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	355

6.6.1A	Occupied bandwidth for CA	355

6.6.1B	 Occupied bandwidth for UL-MIMO	355

6.6.1F	Occupied bandwidth for category NB1 and NB2	356

6.6.1G	Occupied bandwidth for V2X Communication	356

6.6.2	Out of band emission	356

6.6.2.1	Spectrum emission mask	356

6.6.2.1.1	Minimum requirement	356

6.6.2.1A	Spectrum emission mask for CA	357

6.6.2.2	Additional spectrum emission mask	358

6.6.2.2.1	Minimum requirement (network signalled value "NS_03", “NS_11”, "NS_20", and “NS_21”)	358

6.6.2.2.2	Minimum requirement (network signalled value "NS_04")	359

6.6.2.2.3	Minimum requirement (network signalled value "NS_06" or “NS_07”)	359

6.6.2.2.4	Minimum requirement (network signalled value "NS_33" or “NS_34”)	360

6.6.2.2.5	Minimum requirement (network signalled value “NS_27” and “NS_43”)	360

6.6.2.2.6	Minimum requirement (network signalled value "NS_28”)	361

6.6.2.2.7	Minimum requirement (network signalled value "NS_35")	361

6.6.2.2A	Additional Spectrum Emission Mask for CA	362

6.6.2.2A.1	Minimum requirement (network signalled value "CA_NS_04")	362

6.6.2.2A.2	Minimum requirement CA_66B (network signalled value "CA_NS_09")	362

6.6.2.2A.3	Minimum requirement CA_66C (network signalled value "CA_NS_09")	363

6.6.2.2A.4	Minimum requirement CA_48C (network signalled value "CA_NS_10")	363

6.6.2.3	Adjacent Channel Leakage Ratio	364

6.6.2.3.1	Minimum requirement E-UTRA	364

6.6.2.3.1a	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	365

6.6.2.3.1A	Void	366

6.6.2.3.1Aa	Void	366

6.6.2.3.2	Minimum requirements UTRA	366

6.6.2.3.2A	Minimum requirement UTRA for CA	367

6.6.2.3.3A	Minimum requirements for CA E-UTRA	369

6.6.2.4	Void	370

6.6.2.4.1	Void	370

6.6.2A	Void	370

6.6.2B	 Out of band emission for UL-MIMO	370

6.6.2C	 Void	370

6.6.2D	 Out of band emission for ProSe	370

6.6.2F	Out of band emission for category NB1 and NB2	371

6.6.2F.1	Spectrum emission mask	371

6.6.2F.2	Additional Spectrum Emission Mask for Category NB1 and NB2	371

6.6.2F.2.1	Minimum requirement (network signalled value "NS_02")	371

6.6.2F.2.2	Minimum requirement (network signalled value "NS_03")	371

6.6.2F.3	Adjacent Channel Leakage Ratio for category NB1 and NB2	372

6.6.2G	 Out of band emission for V2X Communication	372

6.6.3	Spurious emissions	373

6.6.3.1	Minimum requirements	373

6.6.3.1A	Minimum requirements for CA	374

6.6.3.2	Spurious emission band UE co-existence	375

6.6.3.2A	Spurious emission band UE co-existence for CA	383

6.6.3.3	Additional spurious emissions	398

6.6.3.3.1 	Minimum requirement (network signalled value "NS_05")	398

6.6.3.3.2 	Minimum requirement (network signalled value “NS_07”)	399

6.6.3.3.3 	Minimum requirement (network signalled value “NS_08”)	399

6.6.3.3.4 	Minimum requirement (network signalled value “NS_09”)	399

6.6.3.3.5 	Minimum requirement (network signalled value "NS_12")	400

6.6.3.3.6 	Minimum requirement (network signalled value “NS_13”)	400

6.6.3.3.7 	Minimum requirement (network signalled value “NS_14”)	400

6.6.3.3.8 	Minimum requirement (network signalled value “NS_15”)	400

6.6.3.3.9 	Minimum requirement (network signalled value “NS_16”)	401

6.6.3.3.10 	Minimum requirement (network signalled value “NS_17”)	401

6.6.3.3.11 	Minimum requirement (network signalled value “NS_18”)	401

6.6.3.3.12 	Minimum requirement (network signalled value “NS_19”)	401

6.6.3.3.13 	Minimum requirement (network signalled value “NS_11”)	402

6.6.3.3.14	Minimum requirement (network signalled value “NS_20”)	402

6.6.3.3.15	Minimum requirement (network signalled value “NS_21”)	402

6.6.3.3.16	Minimum requirement (network signalled value "NS_22")	403

6.6.3.3.17	Minimum requirement (network signalled value “NS_23”)	403

6.6.3.3.18	Void	404

6.6.3.3.19	Minimum requirement (network signalled value "NS_04")	404

6.6.3.3.20	Minimum requirement (network signalled value “NS_24”)	404

6.6.3.3.21	Minimum requirement (network signalled value “NS_25”)	405

6.6.3.3.22	Minimum requirement (network signalled value “NS_26”)	405

6.6.3.3.23	Minimum requirement (network signalled value “NS_27” and “NS_43”)	405

6.6.3.3.24	Minimum requirement (network signalled value “NS_28”)	405

6.6.3.3.25	Minimum requirement (network signalled value “NS_29”)	406

6.6.3.3.26	Minimum requirement (network signalled value “NS_30”)	406

6.6.3.3.27	Minimum requirement (network signalled value “NS_31”)	407

6.6.3.3.28	Minimum requirement (network signalled value “NS_36”)	408

6.6.3.3.29	Minimum requirement (network signalled value “NS_38”)	408

6.6.3.3.30	Minimum requirement (network signalled value “NS_39”)	408

6.6.3.3.31	Minimum requirement (network signalled value “NS_40” and “NS_41”)	409

6.6.3.3.32	Minimum requirement (network signalled value “NS_42”)	409

6.6.3.3.33	Minimum requirement (network signalled value “NS_44”)	409

6.6.3.3.34	Minimum requirement (network signalled value “NS_45”)	409

6.6.3.3A	Additional spurious emissions for CA	410

6.6.3.3A.1	Minimum requirement for CA_1C (network signalled value "CA_NS_01")	410

6.6.3.3A.2 	Minimum requirement for CA_1C (network signalled value "CA_NS_02")	410

6.6.3.3A.3	Minimum requirement for CA_1C (network signalled value "CA_NS_03")	411

6.6.3.3A.4	Minimum requirement for CA_38C (network signalled value "CA_NS_05")	411

6.6.3.3A.5	Minimum requirement for CA_7C (network signalled value "CA_NS_06")	411

6.6.3.3A.6	Minimum requirement for CA_39C and CA_39C-41A (network signalled value "CA_NS_07")	412

6.6.3.3A.7	Minimum requirement for CA_42C (network signalled value "CA_NS_08")	412

6.6.3.3A.8	Minimum requirement for CA_41C (network signalled value "CA_NS_04")	412

6.6.3.3A.9	Void	413

6.6.3.3A.10	Minimum requirement for CA_48C (network signalled value "CA_NS_10")	413

6.6.3A	Void	413

6.6.3B	 Spurious emission for UL-MIMO	413

6.6.3C	 Void	413

6.6.3D	 Spurious emission for ProSe	413

6.6.3F	Spurious emission for category NB1 and NB2	414

6.6.3G	Spurious emission for V2X Communication	414

6.6A	Void	418

6.6B	Void	418

6.7	Transmit intermodulation	418

6.7.1	Minimum requirement	418

6.7.1A	Minimum requirement for CA	419

6.7.1B	Minimum requirement for UL-MIMO	419

6.7.1F	Minimum requirement for category NB1 and NB2	419

6.7.1G	Minimum requirement for V2X Communication	420

6.8	Void	420

6.8A	Void	420

6.8B	 Time alignment error for UL-MIMO	420

6.8B.1	 Minimum Requirements	420

6.8C	Void	421

6.8D	Void	421

6.8E	Void	421

6.8F	Void	421

6.8G	Time alignment error	421

7	Receiver characteristics	421

7.1	General	421

7.2	Diversity characteristics	422

7.3	Reference sensitivity power level	422

7.3.1	Minimum requirements (QPSK)	422

7.3.1A	Minimum requirements (QPSK) for CA	457

7.3.1B	Minimum requirements (QPSK) for UL-MIMO	557

7.3.1D	Minimum requirements (QPSK) for ProSe	557

7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis	559

7.3.1F	Minimum requirements for UE category NB1 and NB2	567

7.3.1F.1	Reference sensitivity for UE category NB1 and NB2	567

7.3.1F.2	Void	568

7.3.1G	Minimum requirements (QPSK) for V2X	568

7.3.2	Void	570

7.4	Maximum input level	570

7.4.1	Minimum requirements	571

7.4.1A	Minimum requirements for CA	571

7.4.1B	Minimum requirements for UL-MIMO	572

7.4.1D	Minimum requirements for ProSe	572

7.4.1F	Minimum requirements for category NB1 and NB2	573

7.4.1G	Minimum requirements for V2X	573

7.4A	Void	574

7.4A.1	Void	574

7.5	Adjacent Channel Selectivity (ACS)	574

7.5.1	Minimum requirements	574

7.5.1A	Minimum requirements for CA	575

7.5.1B	Minimum requirements for UL-MIMO	579

7.5.1D	Minimum requirements for ProSe	579

7.5.1F	Minimum requirements for category NB1 and NB2	580

7.5.1G	Minimum requirements for V2X	581

7.6	Blocking characteristics	583

7.6.1	In-band blocking	583

7.6.1.1	Minimum requirements	583

7.6.1.1A	Minimum requirements for CA	585

7.6.1.1D	Minimum requirements for ProSe	588

7.6.1.1F	Minimum requirements for category NB1 and NB2	589

7.6.1.1G	Minimum requirements for V2X	589

7.6.2	Out-of-band blocking	591

7.6.2.1	Minimum requirements	591

7.6.2.1A	Minimum requirements for CA	593

7.6.2.1D	Minimum requirements for ProSe	596

7.6.2.1F	Minimum requirements for category NB1 and NB2	596

7.6.2.1G	Minimum requirements for V2X	597

7.6.3	Narrow band blocking	598

7.6.3.1	Minimum requirements	598

7.6.3.1A	Minimum requirements for CA	599

7.6.3.1D	Minimum requirements for ProSe	600

7.6A	Void	601

7.6B	Blocking characteristics for UL-MIMO	601

7.7	Spurious response	601

7.7.1	Minimum requirements	601

7.7.1A	Minimum requirements for CA	602

7.7.1B	Minimum requirements for UL-MIMO	603

7.7.1D	Minimum requirements for ProSe	603

7.7.1F	Minimum requirements for UE category NB1 and NB2	604

7.7.1G	Minimum requirements for V2X	604

7.8	Intermodulation characteristics	605

7.8.1	Wide band intermodulation	605

7.8.1.1	Minimum requirements	605

7.8.1A	Minimum requirements for CA	606

7.8.1B	Minimum requirements for UL-MIMO	609

7.8.1D	Minimum requirements for ProSe	609

7.8.1F	Minimum requirements for category NB1 and NB2	610

7.8.1G	Minimum requirements	610

7.8.2	Void	611

7.9	Spurious emissions	611

7.9.1	Minimum requirements	612

7.9.1A	Minimum requirements	612

7.10	Receiver image	613

7.10.1	Void	613

7.10.1A	Minimum requirements for CA	613

7.10.1G	Minimum requirements for V2X Communication	613

8	Performance requirement	568

8.1	General	568

8.1.1	Receiver antenna capability	568

8.1.1.1	Simultaneous unicast and MBMS operations	569

8.1.1.2	Dual-antenna receiver capability in idle mode	569

8.1.2	Applicability of requirements	569

8.1.2.1	Applicability of requirements for different channel bandwidths	569

8.1.2.2	Definition of CA capability	569

8.1.2.2A	Definition of dual connectivity capability	574

8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets	575

8.1.2.3A	Applicability and test rules for different dual connectivity configuration and bandwidth combination set	577

8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	578

8.1.2.3C	Applicability and test rules for SDR tests for 4Rx capable UEs	580

8.1.2.3D	Applicability and test rules for different CA with LAA SCell(s) configurations and bandwidth combination sets	580

8.1.2.3E	Applicability and test rules for SDR tests for 8Rx capable UEs	581

8.1.2.4	Test coverage for different number of component carriers	582

8.1.2.5	Applicability of performance requirements for Type B receiver	583

8.1.2.6	Applicability of performance requirements for 4Rx capable UEs	583

8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx	583

8.1.2.6.2	Applicability rule and antenna connection for CA and DC tests with 2Rx	585

8.1.2.6.3	Applicability rule and antenna connection for single carrier tests with 4Rx	585

8.1.2.6.4	Applicability rule for 256QAM tests	586

8.1.2.6.5	Applicability rule and antenna connection for CA and DC tests with 4Rx	586

8.1.2.6.6	Applicability rule for Type C with 4Rx	590

8.1.2.6.7	Applicability rule for 1024QAM tests	590

8.1.2.7	Applicability of Enhanced Downlink Control Channel Performance Requirements	590

8.1.2.8	Applicability of performance requirements for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations	592

8.1.2.8A	Applicability of performance requirements for UE supporting coverage enhancement	592

8.1.2.9	Applicability of SDR requirements for CA and LAA	593

8.1.2.10	Applicability of performance requirements for Multi-user Superposed Transmission	594

8.1.2.11	Applicability CRS interference mitigation receivers performance requirements	594

8.1.2.12	Applicability of performance requirements for 8Rx capable UEs	595

8.1.2.12.1	Applicability rule and antenna connection for single carrier PDSCH tests	595

8.1.2.12.2	Applicability rule and antenna connection for control channel tests	601

8.1.2.12.3	Applicability rule and antenna connection for CA and DC tests	601

8.1.3	UE category and UE DL category	602

8.2	Demodulation of PDSCH (Cell-Specific Reference Symbols)	602

8.2.1	FDD (Fixed Reference Channel)	602

8.2.1.1	Single-antenna port performance	603

8.2.1.1.1	Minimum Requirement	603

8.2.1.1.2	Void	609

8.2.1.1.3	Void	609

8.2.1.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	609

8.2.1.1.4A	Minimum Requirement 1 PRB allocation in presence of FeMBMS Unicast-mixed Cell under CA	609

8.2.1.2	Transmit diversity performance	610

8.2.1.2.1	Minimum Requirement 2 Tx Antenna Port	610

8.2.1.2.2	Minimum Requirement 4 Tx Antenna Port	611

8.2.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	611

8.2.1.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	613

8.2.1.2.4	Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	615

8.2.1.2.5	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	617

8.2.1.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	618

8.2.1.2.7	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	619

8.2.1.3	Open-loop spatial multiplexing performance	619

8.2.1.3.1	Minimum Requirement 2 Tx Antenna Port	619

8.2.1.3.1A	Soft buffer management test	623

8.2.1.3.1B	Enhanced Performance Requirement Type C –2Tx Antenna Ports	624

8.2.1.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	625

8.2.1.3.2	Minimum Requirement 4 Tx Antenna Port	626

8.2.1.3.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	626

8.2.1.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	630

8.2.1.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	632

8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	633

8.2.1.4	Closed-loop spatial multiplexing performance	634

8.2.1.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	634

8.2.1.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	635

8.2.1.4.1B	Enhanced Performance Requirement Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	635

8.2.1.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	637

8.2.1.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	640

8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	642

8.2.1.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	643

8.2.1.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	644

8.2.1.4.2A	Enhanced Performance Requirement Type C – Multi-layer Spatial Multiplexing 2Tx Antenna Ports	645

8.2.1.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	645

8.2.1.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	649

8.2.1.4.4	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	651

8.2.1.5	MU-MIMO	652

8.2.1.6	[Control channel performance: D-BCH and PCH]	652

8.2.1.7	Carrier aggregation with power imbalance	652

8.2.1.7.1	Minimum Requirement	652

8.2.1.8	Intra-band non-contiguous carrier aggregation with timing offset	653

8.2.1.8.1	Minimum Requirement	653

8.2.1.9	HST-SFN performance	654

8.2.1.9.1	Minimum Requirement	654

8.2.1.9.2	Minimum Requirement for Rel-16 further enhanced HST	657

8.2.2	TDD (Fixed Reference Channel)	657

8.2.2.1	Single-antenna port performance	658

8.2.2.1.1	Minimum Requirement	658

8.2.2.1.2	Void	663

8.2.2.1.3	Void	663

8.2.2.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	663

8.2.2.2	Transmit diversity performance	663

8.2.2.2.1	Minimum Requirement 2 Tx Antenna Port	663

8.2.2.2.2	Minimum Requirement 4 Tx Antenna Port	664

8.2.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	665

8.2.2.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	666

8.2.2.2.4	Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	668

8.2.2.2.5	Minimum Requirement 2 Tx Antenna Port (when EIMTA-MainConfigServCell-r12 is configured)	670

8.2.2.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	670

8.2.2.2.7	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	672

8.2.2.2.8	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	673

8.2.2.3	Open-loop spatial multiplexing performance	674

8.2.2.3.1	Minimum Requirement 2 Tx Antenna Port	674

8.2.2.3.1A	Soft buffer management test	677

8.2.2.3.1B	Enhanced Performance Requirement Type C - 2Tx Antenna Ports	677

8.2.2.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	678

8.2.2.3.2	Minimum Requirement 4 Tx Antenna Port	679

8.2.2.3.3	Minimum Requirement 2Tx antenna port (demodulation subframe overlaps with aggressor cell ABS)	680

8.2.2.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	684

8.2.2.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	686

8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	687

8.2.2.4	Closed-loop spatial multiplexing performance	688

8.2.2.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	688

8.2.2.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	689

8.2.2.4.1B	Enhanced Performance Requirement Type A – Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	689

8.2.2.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	691

8.2.2.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	693

8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	695

8.2.2.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	696

8.2.2.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	698

8.2.2.4.2A	Enhanced Performance Requirement Type C Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	698

8.2.2.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	699

8.2.2.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	703

8.2.2.4.4	Void	704

8.2.2.4.5	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	704

8.2.2.5	MU-MIMO	705

8.2.2.6	[Control channel performance: D-BCH and PCH]	705

8.2.2.7	Carrier aggregation with power imbalance	705

8.2.2.7.1	Minimum Requirement	705

8.2.2.8	Intra-band contiguous carrier aggregation with minimum channel spacing	706

8.2.2.8.1	Minimum Requirement	706

8.2.2.9	HST-SFN performance	707

8.2.2.9.1	Minimum Requirement	707

8.2.2.9.2	Minimum Requirement for Rel-16 further enhanced HST	709

8.2.3	TDD FDD CA (Fixed Reference Channel)	710

8.2.3.1	Single-antenna port performance	711

8.2.3.1.1	Minimum Requirement for FDD PCell	711

8.2.3.1.2	Minimum Requirement for TDD PCell	715

8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port	719

8.2.3.2.1	Minimum Requirement for FDD PCell	719

8.2.3.2.1A	Soft buffer management test for FDD PCell	723

8.2.3.2.2	Minimum Requirement for TDD PCell	724

8.2.3.2.2A	Soft buffer management test for TDD PCell	728

8.2.3.3	Closed-loop spatial multiplexing performance 4Tx Antenna Port	729

8.2.3.3.1	Minimum Requirement for FDD PCell	729

8.2.3.3.2	Minimum Requirement for TDD PCell	733

8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity	737

8.2.3.5	HST-SFN performance	739

8.2.3.5.0	General	739

8.2.3.5.1	Minimum Requirement for FDD PCell	739

8.2.3.5.2	Minimum Requirement for TDD PCell	742

8.2.4	LAA	745

8.2.4.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port	745

8.2.4.1.1	FDD PCell (FDD single carrier)	745

8.2.4.1.2	TDD PCell (TDD single carrier)	749

8.3	Demodulation of PDSCH (User-Specific Reference Symbols)	752

8.3.1	FDD	752

8.3.1.1	Single-layer Spatial Multiplexing	753

8.3.1.1A	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	755

8.3.1.1B	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	758

8.3.1.1C	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference model	761

8.3.1.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	763

8.3.1.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference model	764

8.3.1.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	765

8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)	767

8.3.1.1H	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	769

8.3.1.1I	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	770

8.3.1.2	Dual-Layer Spatial Multiplexing	772

8.3.1.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	773

8.3.1.3	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	774

8.3.1.3.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	774

8.3.1.3.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	776

8.3.1.3.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	778

8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	780

8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	782

8.3.1.3.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	785

8.3.1.4	Performance Requirements for semiOpenLoop transmission	787

8.3.2	TDD	789

8.3.2.1	Single-layer Spatial Multiplexing	789

8.3.2.1A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	791

8.3.2.1B	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	793

8.3.2.1C	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	796

8.3.2.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference	799

8.3.2.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	801

8.3.2.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference	803

8.3.2.1G	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	804

8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)	806

8.3.2.1I	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	808

8.3.2.1J	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	809

8.3.2.2	Dual-Layer Spatial Multiplexing	811

8.3.2.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	811

8.3.2.3	Dual-Layer Spatial Multiplexing (with multiple CSI-RS configurations)	812

8.3.2.4	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	813

8.3.2.4.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	813

8.3.2.4.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	815

8.3.2.4.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	817

8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	819

8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	821

8.3.2.4.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	824

8.3.2.5	Performance Requirements for semiOpenLoop transmission	826

8.3.3	LAA	828

8.3.3.1	Dual-Layer Spatial Multiplexing with DM-RS	828

8.3.3.1.1	FDD PCell (FDD single carrier)	828

8.3.3.1.2	TDD Pcell (TDD single carrier)	832

8.4	Demodulation of PDCCH/PCFICH	836

8.4.1	FDD	836

8.4.1.1	Single-antenna port performance	837

8.4.1.2	Transmit diversity performance	837

8.4.1.2.1	Minimum Requirement 2 Tx Antenna Port	837

8.4.1.2.2	Minimum Requirement 4 Tx Antenna Port	837

8.4.1.2.3 Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	838

8.4.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	843

8.4.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port under Asynchronous Network	849

8.4.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	850

8.4.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	851

8.4.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	852

8.4.1.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	853

8.4.2	TDD	854

8.4.2.1	Single-antenna port performance	855

8.4.2.2	Transmit diversity performance	855

8.4.2.2.1	Minimum Requirement 2 Tx Antenna Port	855

8.4.2.2.2	Minimum Requirement 4 Tx Antenna Port	856

8.4.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	856

8.4.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	860

8.4.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	864

8.4.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	865

8.4.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	866

8.4.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	867

8.4.2.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	868

8.4.3	LAA	869

8.4.3.1	Transmit diversity performance	869

8.4.3.1.1	FDD Pcell (FDD single carrier)	869

8.4.3.1.2	TDD Pcell (TDD single carrier)	870

8.5	Demodulation of PHICH	871

8.5.1	FDD	872

8.5.1.1	Single-antenna port performance	872

8.5.1.2	Transmit diversity performance	872

8.5.1.2.1	Minimum Requirement 2 Tx Antenna Port	872

8.5.1.2.2	Minimum Requirement 4 Tx Antenna Port	873

8.5.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	873

8.5.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	875

8.5.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports under Asynchronous Network	878

8.5.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	879

8.5.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	880

8.5.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	881

8.5.2	TDD	882

8.5.2.1	Single-antenna port performance	883

8.5.2.2	Transmit diversity performance	883

8.5.2.2.1	Minimum Requirement 2 Tx Antenna Port	883

8.5.2.2.2	Minimum Requirement 4 Tx Antenna Port	884

8.5.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	884

8.5.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	886

8.5.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	888

8.5.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	889

8.5.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	890

8.5.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	891

8.6	Demodulation of PBCH	892

8.6.1	FDD	892

8.6.1.1	Single-antenna port performance	892

8.6.1.2	Transmit diversity performance	893

8.6.1.2.1	Minimum Requirement 2 Tx Antenna Port	893

8.6.1.2.2	Minimum Requirement 4 Tx Antenna Port	893

8.6.1.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	893

8.6.2	TDD	895

8.6.2.1	Single-antenna port performance	895

8.6.2.2	Transmit diversity performance	895

8.6.2.2.1	Minimum Requirement 2 Tx Antenna Port	895

8.6.2.2.2	Minimum Requirement 4 Tx Antenna Port	895

8.6.2.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	896

8.7	Sustained downlink data rate provided by lower layers	897

8.7.1	FDD (single carrier and CA)	897

8.7.2	TDD (single carrier and CA)	914

8.7.3	FDD (EPDCCH scheduling)	918

8.7.4	TDD (EPDCCH scheduling)	920

8.7.5	TDD FDD CA	922

8.7.5.1	Minimum Requirement FDD PCell	923

8.7.5.2	Minimum Requirement TDD PCell	931

8.7.6	FDD (DC)	940

8.7.7	TDD (DC)	947

8.7.8	TDD FDD (DC)	950

8.7.9	FDD (4 Rx)	953

8.7.10	TDD (4 Rx)	955

8.7.11	TDD FDD CA (4 Rx)	957

8.7.11.1	Void	959

8.7.12	LAA	959

8.7.12.1	FDD CA in licensed bands	959

8.7.12.2	TDD CA in licensed bands	961

8.7.12.3	TDD-FDD CA in licensed bands	963

8.7.13	FDD DC (4 Rx)	966

8.7.14	TDD DC (4 Rx)	967

8.7.15	TDD FDD DC (4 Rx)	969

8.7.16	FDD (1024QAM and up to 4Rx supported)	971

8.7.17	TDD (1024QAM and up to 4 Rx supported)	974

8.7.18	TDD FDD CA (1024QAM and up to 4 Rx supported)	976

8.7.19	TDD (8 Rx)	978

8.8	Demodulation of EPDCCH	980

8.8.1	Distributed Transmission	980

8.8.1.1	FDD	980

8.8.1.1.1	Void	981

8.8.1.2	TDD	981

8.8.1.2.1	Void	982

8.8.2	Localized Transmission with TM9	982

8.8.2.1	FDD	982

8.8.2.1.1	Void	983

8.8.2.1.2	Void	984

8.8.2.2	TDD	984

8.8.2.2.1	Void	985

8.8.2.2.2	Void	985

8.8.3	Localized transmission with TM10 Type B quasi co-location type	985

8.8.3.1	FDD	985

8.8.3.2	TDD	988

8.8.4	Enhanced Downlink Control Channel Performance Requirements Type A - Localized Transmission with CRS Interference Model	991

8.8.4.1	FDD	991

8.8.4.2	TDD	992

8.8.5	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM9 Interference Model	994

8.8.5.1	TDD	994

8.8.6	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM3 Interference Model	995

8.8.6.1	FDD	995

8.9	Demodulation (single receiver antenna)	996

8.9.1	PDSCH	996

8.9.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	996

8.9.1.1.1	Transmit diversity performance (Cell-Specific Reference Symbols)	996

8.9.1.1.2	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	997

8.9.1.1.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	999

8.9.1.2	TDD (Fixed Reference Channel)	1002

8.9.1.2.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1002

8.9.1.2.2	 Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1003

8.9.1.2.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1006

8.9.2	PHICH	1008

8.9.2.1	FDD and half-duplex FDD	1008

8.9.2.1.1	Transmit diversity performance	1008

8.9.2.2	TDD	1008

8.9.2.2.1	Transmit diversity performance	1008

8.9.3	PBCH	1009

8.9.3.1	FDD and half-duplex FDD	1009

8.9.3.1.1	Transmit diversity performance	1009

8.9.3.2	TDD	1009

8.9.3.2.1	Transmit diversity performance	1009

8.9.4	PDCCH/PCFICH	1009

8.9.4.1	FDD and half-duplex FDD	1009

8.9.4.1.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1009

8.9.4.1.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1010

8.9.4.2	TDD	1011

8.9.4.2.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1011

8.9.4.2.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1012

8.10	Demodulation (4 receiver antenna ports)	1013

8.10.1	PDSCH	1013

8.10.1.1	FDD (Fixed Reference Channel)	1013

8.10.1.1.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1014

8.10.1.1.1A	Transmit diversity performance wit Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	1015

8.10.1.1.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1016

8.10.1.1.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1016

8.10.1.1.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (Cell-Specific Reference Symbols)	1017

8.10.1.1.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1018

8.10.1.1.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1019

8.10.1.1.5A	Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1022

8.10.1.1.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1023

8.10.1.1.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1024

8.10.1.1.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1026

8.10.1.1.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1027

8.10.1.1.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1028

8.10.1.1.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1028

8.10.1.1.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas (Cell-Specific Reference Symbols)	1029

8.10.1.1.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1029

8.10.1.1.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1031

8.10.1.1.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1032

8.10.1.1.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1033

8.10.1.1.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1034

8.10.1.1.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1035

8.10.1.1.14	HST-SFN performance	1039

8.10.1.2	TDD (Fixed Reference Channel)	1039

8.10.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1040

8.10.1.2.1A	Transmit diversity performance with Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	1041

8.10.1.2.2	Open-loop spatial multiplexing performance  with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1042

8.10.1.2.3	 Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1042

8.10.1.2.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1043

8.10.1.2.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1044

8.10.1.2.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1045

8.10.1.2.5A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	1048

8.10.1.2.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1049

8.10.1.2.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1050

8.10.1.2.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1052

8.10.1.2.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1053

8.10.1.2.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1054

8.10.1.2.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1054

8.10.1.2.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas	1055

8.10.1.2.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1056

8.10.1.2.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1057

8.10.1.2.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1058

8.10.1.2.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1060

8.10.1.2.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1061

8.10.1.2.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1063

8.10.1.2.14	HST-SFN performance	1067

8.10.2	PDCCH/PCFICH	1068

8.10.2.1	FDD	1068

8.10.2.1.1	Single-antenna port performance	1068

8.10.2.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1068

8.10.2.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1068

8.10.2.1.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1069

8.10.2.2	TDD	1070

8.10.2.2.1	Single-antenna port performance	1070

8.10.2.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1071

8.10.2.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1071

8.10.2.2.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1071

8.10.3	PHICH	1072

8.10.3.1	 FDD	1072

8.10.3.1.1	Single Tx Antenna Port performance	1073

8.10.3.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1073

8.10.3.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1073

8.10.3.2	TDD	1074

8.10.3.2.1	Single Tx Antenna Port performance	1074

8.10.3.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1075

8.10.3.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1075

8.10.4	ePDCCH	1075

8.10.4.1	Distributed Transmission with 4Rx	1075

8.10.4.1.1	FDD	1075

8.10.4.1.2	TDD	1076

8.10.4.2	Localized Transmission with TM9 and 4Rx	1077

8.10.4.2.1	FDD	1077

8.10.4.2.2	TDD	1078

8.11	Demodulation (UE supporting coverage enhancement)	1079

8.11.1	PDSCH	1080

8.11.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1080

8.11.1.1.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1080

8.11.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1084

8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1086

8.11.1.2	TDD (Fixed Reference Channel)	1091

8.11.1.2.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1091

8.11.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1096

8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1098

8.11.2	MPDCCH	1102

8.11.2.1	FDD and half-duplex FDD	1103

8.11.2.1.1	CE Mode A	1104

8.11.2.1.2	CE Mode B	1105

8.11.2.1.3	CE Mode A with TM9 interference model	1105

8.11.2.1.4	CE Mode A with CRS interference model	1107

8.11.2.2	TDD	1108

8.11.2.2.1	CE Mode A	1109

8.11.2.2.2	CE Mode B	1110

8.11.2.2.3	CE Mode A with TM9 interference model	1110

8.11.2.2.4	CE Mode A with CRS interference model	1111

8.11.3	PBCH	1112

8.11.3.1	FDD and half-duplex FDD	1113

8.11.3.1.1	Transmit diversity performance	1113

8.11.3.2	TDD	1114

8.11.3.2.1	Transmit diversity performance	1114

8.12	Demodulation of Narrowband IoT	1115

8.12.1	NPDSCH	1115

8.12.1.1	Half-duplex FDD	1115

8.12.1.1.1	Minimum Requirements for In-band	1115

8.12.1.1.2	Minimum Requirements for Standalone/Guard-band	1116

8.12.1.1.3	Minimum Requirements for Standalone for UE Category NB2	1117

8.12.1.2	TDD	1118

8.12.1.2.1	Minimum Requirements for In-band	1119

8.12.1.2.2	Minimum Requirements for Standalone/Guard-band	1120

8.12.1.2.3	Minimum Requirements for Standalone for UE Category NB2	1120

8.12.2	NPDCCH	1121

8.12.2.1	Half-duplex FDD	1121

8.12.2.1.1	Single-antenna performance	1122

8.12.2.1.2	Transmit diversity performance	1123

8.12.2.2	TDD	1123

8.12.2.2.1	Single-antenna performance	1125

8.12.2.2.2	Transmit diversity performance	1125

8.12.3	Demodulation of NPBCH	1125

8.12.3.1	HD-FDD	1126

8.12.3.1.1	Single-antenna port performance with single NPBCH TTI	1126

8.12.3.1.2	Transmit diversity performance	1126

8.12.3.2	TDD	1126

8.12.3.2.1	Single-antenna port performance with single NPBCH TTI	1127

8.12.3.2.2	Transmit diversity performance	1127

8.13	Demodulation of PDSCH CA and DC(4 receiver antenna ports)	1127

8.13.1	FDD (CA and DC)	1127

8.13.1.1	Closed-loop spatial multiplexing performance	1127

8.13.1.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1127

8.13.1.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1131

8.13.1.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1133

8.13.1.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1135

8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1136

8.13.1.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1136

8.13.1.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1139

8.13.1.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1139

8.13.1.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1141

8.13.1.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1141

8.13.2	TDD (CA and DC)	1144

8.13.2.1	Closed-loop spatial multiplexing performance	1145

8.13.2.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1145

8.13.2.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1147

8.13.2.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1148

8.13.2.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1149

8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1151

8.13.2.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1151

8.13.2.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1155

8.13.2.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1155

8.13.3	TDD-FDD (CA and DC)	1157

8.13.3.1	 Closed-loop spatial multiplexing performance 4Tx Antenna Port	1158

8.13.3.1.1	Minimum Requirement for FDD PCell	1158

8.13.3.1.2	Minimum Requirement for TDD PCell	1161

8.13.3.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1165

8.13.3.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for FDD PCell	1165

8.13.3.2.2	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for TDD PCell	1168

8.13.3.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1171

8.13.3.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for FDD PCell	1171

8.13.3.3.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for TDD PCell	1173

8.13.3.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1175

8.13.3.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for FDD PCell	1175

8.13.3.4.2	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for TDD PCell	1178

8.13.3.5	 Closed-loop spatial multiplexing performance 4Tx Antenna Port for DC	1181

8.13.3.5.1	Minimum Requirement for FDD PCell	1181

8.13.3.5.2	Minimum Requirement for TDD PCell	1183

8.13.3.6	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with 256QAM	1185

8.13.3.6.1	Minimum Requirement for FDD PCell	1185

8.13.3.6.2	Minimum Requirement for TDD PCell	1187

8.13.3.7	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with Four layers	1188

8.13.3.7.1	Minimum Requirement for FDD PCell	1188

8.13.3.7.2	Minimum Requirement for TDD PCell	1190

8.14	Demodulation (UE supporting Short TTI)	1192

8.14.1	Slot-PDSCH and Subslot-PDSCH	1192

8.14.1.1	FDD (Fixed Reference Channel)	1192

8.14.1.1.1	Open-loop spatial multiplexing performance	1192

8.14.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1193

8.14.1.2	TDD (Fixed Reference Channel)	1195

8.14.1.2.1	Open-loop spatial multiplexing performance	1195

8.14.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1196

8.14.2	SPDCCH	1197

8.14.2.1	FDD	1197

8.14.2.1.1	Mimimum requirement	1198

8.14.2.2	TDD	1198

8.14.2.2.1	Mimimum requirement	1199

8.15	Demodulation (8 receiver antenna ports)	1200

8.15.1	PDSCH	1200

8.15.1.1	Void	1200

8.15.1.2	TDD (Fixed Reference Channel)	1200

8.15.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1200

8.15.1.2.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1201

8.15.1.2.3	8 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1201

8.15.2	CA	1203

8.15.2.1	Void	1203

8.15.2.2	TDD	1203

8.15.2.2.1	Eight Layer Spatial Multiplexing (User-Specific Reference Symbols)	1203

9	Reporting of Channel State Information	1205

9.1	General	1205

9.1.1	Applicability of requirements	1205

9.1.1.1	Applicability of requirements for different channel bandwidths	1205

9.1.1.2	Applicability and test rules for different CA configurations and bandwidth combination sets	1205

9.1.1.2A	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	1206

9.1.1.3	Test coverage for different number of componenet carriers	1207

9.1.1.4	Applicability of performance requirements for 4Rx capable UEs	1207

9.1.1.4.1	Applicability rule and antenna connection for single carrier tests with 2Rx	1207

9.1.1.4.2	Applicability rule and antenna connection for CA tests with 2Rx	1209

9.1.1.4.3	Applicability rule and antenna connection for single carrier tests with 4Rx	1210

9.1.1.5	Applicability of requirements for UEs supporting coverage enhancement	1210

9.2	 CQI reporting definition under AWGN conditions	1210

9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols)	1210

9.2.1.1	FDD	1210

9.2.1.2	TDD	1211

9.2.1.3	FDD (CSI measurements in case two CSI subframe sets are configured)	1212

9.2.1.4	TDD (CSI measurements in case two CSI subframe sets are configured)	1215

9.2.1.5	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1218

9.2.1.6	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1221

9.2.1.7	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1224

9.2.1.8	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1225

9.2.1.9	FDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1225

9.2.1.10	TDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1226

9.2.2	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1227

9.2.2.1	FDD	1227

9.2.2.2	TDD	1228

9.2.3	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1229

9.2.3.1	FDD	1229

9.2.3.1A	FDD (With channelMeasRestriction configured)	1230

9.2.3.2	TDD	1231

9.2.3.2A	TDD (With channelMeasRestriction configured)	1232

9.2.4	Minimum requirement PUCCH 1-1 (With Single CSI Process)	1233

9.2.4.1	FDD	1234

9.2.4.1A	FDD (With interferenceMeasRestriction configured)	1236

9.2.4.2	TDD	1239

9.2.4.2A	TDD (With interferenceMeasRestriction configured)	1242

9.2.5	Minimum requirement PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured)	1245

9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1248

9.2.6.1	Frame structure type 3 with FDD Pcell	1248

9.2.6.2	Frame structure type 3 with TDD Pcell	1250

9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1252

9.2.7.1	Frame structure type 3 wth FDD Pcell	1252

9.2.7.2	Frame structure type 3 wth TDD Pcell	1254

9.3	CQI reporting under fading conditions	1258

9.3.1	Frequency-selective scheduling mode	1258

9.3.1.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1258

9.3.1.1.1	FDD	1258

9.3.1.1.2	TDD	1259

9.3.1.1.3	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1260

9.3.1.1.4	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1264

9.3.1.1.5	TDD (when csi-SubframeSet –r12 is configured)	1267

9.3.1.2	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1269

9.3.1.2.1	FDD	1269

9.3.1.2.2	TDD	1270

9.3.1.2.3	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1272

9.3.1.2.4	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1273

9.3.1.2.5	Void	1274

9.3.1.2.6	TDD (when csi-SubframeSet –r12 is configured with one CSI process)	1274

9.3.2	Frequency non-selective scheduling mode	1278

9.3.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1278

9.3.2.1.1	FDD	1278

9.3.2.1.2	TDD	1280

9.3.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1282

9.3.2.2.1	FDD	1282

9.3.2.2.2	TDD	1283

9.3.3	Frequency-selective interference	1285

9.3.3.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbol)	1285

9.3.3.1.1	FDD	1285

9.3.3.1.2	TDD	1286

9.3.3.2	Void	1287

9.3.3.2.1	Void	1287

9.3.3.2.2	Void	1287

9.3.4	UE-selected subband CQI	1287

9.3.4.1	Minimum requirement PUSCH 2-0 (Cell-Specific Reference Symbols)	1287

9.3.4.1.1	FDD	1287

9.3.4.1.2	TDD	1288

9.3.4.2	Minimum requirement PUCCH 2-0 (Cell-Specific Reference Symbols)	1289

9.3.4.2.1	FDD	1289

9.3.4.2.2	TDD	1291

9.3.5	Additional requirements for enhanced receiver Type A	1293

9.3.5.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1293

9.3.5.1.1	FDD	1293

9.3.5.1.2	TDD	1294

9.3.5.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1296

9.3.5.2.1	FDD	1296

9.3.5.2.2	TDD	1299

9.3.6	Minimum requirement (With multiple CSI processes)	1302

9.3.6.1	FDD	1302

9.3.6.2	TDD	1307

9.3.7	Minimum requirement PUSCH 3-2	1311

9.3.7.1	FDD	1311

9.3.7.2	TDD	1312

9.3.8	Additional requirements for enhanced receiver Type B	1314

9.3.8.1	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1314

9.3.8.1.1	FDD	1314

9.3.8.1.2	TDD	1315

9.3.8.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1317

9.3.8.2.1	FDD	1317

9.3.8.2.2	TDD	1319

9.3.8.3	Minimum requirement with CSI process	1322

9.3.8.3.1	FDD	1322

9.3.8.3.2	TDD	1325

9.4	Reporting of Precoding Matrix Indicator (PMI)	1328

9.4.1	Single PMI	1329

9.4.1.1	Minimum requirement PUSCH 3-1 (Cell-Specific Reference Symbols)	1329

9.4.1.1.1	FDD	1329

9.4.1.1.2	TDD	1330

9.4.1.2	 Minimum requirement PUCCH 2-1 (Cell-Specific Reference Symbols)	1331

9.4.1.2.1	FDD	1331

9.4.1.2.2	TDD	1333

9.4.1.3	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1334

9.4.1.3.1	FDD	1334

9.4.1.3.2	TDD	1337

9.4.1.3.3	FDD (with Class A 12Tx codebook)	1340

9.4.1.3.4	TDD (with Class A 12Tx codebook)	1342

9.4.1.3.5	FDD (with Class A 24Tx codebook)	1345

9.4.1.3.6	TDD (with Class A 24Tx codebook)	1347

9.4.1.4	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1350

9.4.1.4.1	FDD (with 4Tx enhanced codebook)	1350

9.4.1.4.2	TDD (with 4Tx enhanced codebook)	1352

9.4.1.4.3	FDD (with Class B alternative codebook for one CSI-RS resource configured)	1355

9.4.1.4.4	TDD (with Class B alternative codebook for one CSI-RS resource configured)	1357

9.4.1a	Void	1360

9.4.1a.1	Void	1360

9.4.1a.1.1	Void	1360

9.4.1a.1.2	Void	1360

9.4.2	Multiple PMI	1360

9.4.2.1	Minimum requirement PUSCH 1-2 (Cell-Specific Reference Symbols)	1360

9.4.2.1.1	FDD	1360

9.4.2.1.2	TDD	1361

9.4.2.2	 Minimum requirement PUSCH 2-2 (Cell-Specific Reference Symbols)	1362

9.4.2.2.1	FDD	1362

9.4.2.2.2	TDD	1363

9.4.2.3	Minimum requirement PUSCH 1-2 (CSI Reference Symbol)	1364

9.4.2.3.1	FDD	1364

9.4.2.3.2	TDD	1366

9.4.2.3.3	FDD (with 4Tx enhanced codebook)	1369

9.4.2.3.4	TDD (with 4Tx enhanced codebook)	1371

9.4.2.3.5	FDD (with Class A 16Tx codebook)	1373

9.4.2.3.6	TDD (with Class A 16Tx codebook)	1376

9.4.2.3.7	FDD (with Class A 32Tx codebook)	1379

9.4.2.3.8	TDD (with Class A 32Tx codebook)	1382

9.4.2.3.9	FDD (with Class A 16Tx advanced codebook)	1385

9.4.2.3.10	TDD (with Class A 16Tx advanced codebook)	1388

9.4.3	Void	1391

9.5	Reporting of Rank Indicator (RI)	1391

9.5.1	Minimum requirement (Cell-Specific Reference Symbols)	1391

9.5.1.1	FDD	1391

9.5.1.2	TDD	1392

9.5.2	Minimum requirement (CSI Reference Symbols)	1393

9.5.2.1	FDD	1393

9.5.2.2	TDD	1395

9.5.3	Minimum requirement (CSI measurements in case two CSI subframe sets are configured)	1397

9.5.3.1	FDD	1397

9.5.3.2	TDD	1400

9.5.4	Minimum requirement (CSI measurements in case two CSI subframe sets are configured and CRS assistance information are configured)	1403

9.5.4.1	FDD	1403

9.5.4.2	TDD	1406

9.5.5	Minimum requirement (with CSI process)	1409

9.5.5.1	FDD	1410

9.5.5.2	TDD	1413

9.6	 Additional requirements for carrier aggregation	1416

9.6.1	 Periodic reporting on multiple cells (Cell-Specific Reference Symbols)	1416

9.6.1.1	FDD	1416

9.6.1.2	TDD	1422

9.6.1.3	TDD-FDD CA with FDD PCell	1428

9.6.1.4	TDD-FDD CA with TDD PCell	1433

9.7	CSI reporting (Single receiver antenna)	1439

9.7.1	CQI reporting definition under AWGN conditions	1440

9.7.1.1	FDD and half-duplex FDD	1440

9.7.1.2	TDD	1440

9.7.1.3 FDD (Category 1bis UE)	1441

9.7.1.4 TDD (Category 1bis UE)	1442

9.7.2	CQI reporting under fading conditions	1443

9.7.2.1	FDD and half-duplex FDD	1443

9.7.2.2	TDD	1444

9.7.2.3 FDD (Category 1bis UE)	1445

9.7.2.4 TDD (Category 1bis UE)	1446

9.8	CSI reporting (UE supporting coverage enhancement)	1447

9.8.1	CQI reporting definition under AWGN conditions	1447

9.8.1.1	FDD and half-duplex FDD	1447

9.8.1.2	TDD	1448

9.8.2	UE-selected subband CQI	1449

9.8.2.1	FDD and half-duplex FDD	1449

9.8.2.2	TDD	1453

9.8.3	CQI reporting definition for UE supporting 64QAM under AWGN	1456

9.8.3.1	FDD and half-duplex FDD	1456

9.8.3.2	TDD	1457

9.8.4	CQI reporting definition for UE supporting alternative table under AWGN	1458

9.8.4.1	FDD and half-duplex FDD	1458

9.8.4.2	TDD	1459

9.9	CSI reporting for 4Rx UE	1460

9.9.1	CQI reporting definition under AWGN conditions	1460

9.9.1.1	Minimum requirement PUCCH 1-0 with Rank 1 (Cell-Specific Reference Symbols)	1461

9.9.1.1.1	FDD	1461

9.9.1.1.2	TDD	1461

9.9.1.2	Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)	1462

9.9.1.2.1	FDD	1462

9.9.1.2.2	TDD	1463

9.9.1.3	 Minimum requirement PUCCH 1-1 with Rank 4 (Cell-Specific Reference Symbols)	1464

9.9.1.3.1	FDD	1465

9.9.1.3.2	TDD	1465

9.9.1.4	Minimum requirement PUCCH 1-1 with Rank 3 (CSI Reference Symbols)	1466

9.9.1.4.1	FDD	1466

9.9.1.4.2	TDD	1467

9.9.2	CQI reporting definition under fading conditions	1468

9.9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol) for enhanced receiver Type A	1468

9.9.2.1.1	FDD	1469

9.9.2.1.2	TDD	1470

9.9.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol) for enhanced receiver Type A	1472

9.9.2.2.1	FDD	1472

9.9.2.2.2	TDD	1475

9.9.3	Reporting of Precoding Matrix Indicator (PMI) for 4Rx UE	1478

9.9.3.1	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1479

9.9.3.1.1	TDD	1479

9.9.4	 Reporting of Rank Indicator (RI)	1481

9.9.4.1	Minimum requirement (Cell-Specific Reference Symbols)	1481

9.9.4.1.1	FDD	1481

9.9.4.1.2	TDD	1483

9.9.4.2	Minimum requirement (CSI Reference Symbols)	1484

9.9.4.2.1	FDD	1484

9.9.4.2.2	TDD	1486

9.10	Reporting of CSI-RS Resource Indicator (CRI)	1488

9.10.1	Minimum requirement (PUSCH 3-1)	1489

9.10.1.1	FDD	1489

9.10.1.2	TDD	1491

9.10.2	Minimum requirement (PUSCH 3-1, QCL Type C)	1493

9.10.2.1	FDD	1493

9.10.2.2	TDD	1496

9.11	Reporting of Hybrid Channel state information	1499

9.11.1	Minimum requirement (with eMIMO-Type configured as Class B with more than one CSI-RS resource configured and eMIMO-Type2 as Class B with one CSI-RS resource configured)	1499

9.11.1.1	FDD	1500

9.11.1.2	TDD	1502

9.12	CSI reporting (UE supporting Short TTI)	1504

9.12.1	CQI reporting under fading conditions (Cell-Specific Reference Symbol)	1504

9.12.1.1	FDD	1504

9.12.1.2	TDD	1507

9.12.2	CQI reporting under fading conditions (CSI Reference Symbol)	1508

9.12.2.1	FDD	1508

9.12.2.2	TDD	1511

9.13	CSI reporting for 8Rx UE	1513

9.13.1	CQI reporting definition under AWGN conditions	1513

9.13.1.1	Minimum requirement PUCCH 1-1 with Rank 4 (CSI Reference Symbols)	1513

9.13.1.2.1	Void	1513

9.13.1.2.2	TDD	1513

10	Performance requirement (MBMS)	1514

10.1	FDD (Fixed Reference Channel)	1514

10.1.1	Minimum requirement	1515

10.2	TDD (Fixed Reference Channel)	1516

10.2.1	Minimum requirement	1516

10.3	FDD (Fixed Reference Channel) with FeMBMS	1517

10.3.1	Minimum requirement for FeMBMS Unicast-mixed Cell under CA	1517

10.3.1.1	Minimum requirement with 1.25kHz subcarrier spacing	1517

10.3.1.2	Minimum requirement with 7.5kHz subcarrier spacing	1518

10.3.2	Minimum requirement for FeMBMS Unicast-mixed Cell as Non-Serving Cell	1519

10.3.2.1	Minimum requirement with 1.25kHz subcarrier spacing	1519

10.3.2.2	Minimum requirement with 7.5kHz subcarrier spacing	1520

10.3.3	Minimum requirement for MBMS Dedicated cell	1521

10.3.3.1	Minimum requirement with 1.25kHz subcarrier spacing	1521

10.3.3.2	Minimum requirement with 7.5kHz subcarrier spacing	1522

10.3.3.3	Minimum requirement with 15kHz subcarrier spacing	1523

11	Performance requirement (ProSe Direct Discovery)	1524

11.1	General	1524

11.1.1	Applicability of requirements	1524

11.1.2	Reference DRX configuration	1525

11.2	Demodulation of PSDCH (single link performance)	1525

11.2.1	FDD (in-coverage)	1525

11.2.2	TDD (in-coverage)	1526

11.2.3	FDD (out-of-coverage)	1527

11.3	Power imbalance performance with two links	1527

11.3.1	FDD	1527

11.3.2	TDD	1528

11.4	Multiple timing reference test	1529

11.4.1	FDD	1530

11.5	Maximum Sidelink processes test	1531

11.5.1	FDD	1531

11.5.2	TDD	1532

12	Performance requirement (ProSe Direct Communication)	1534

12.1	General	1534

12.1.1	Applicability of requirements	1534

12.1.1.1	Applicability of requirements for different channel bandwidths	1534

12.1.1.2	Test coverage for different number of component carriers	1534

12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets	1534

12.1.2	Reference DRX configuration	1535

12.2	Demodulation of PSSCH	1535

12.2.1	FDD	1535

12.3	Demodulation of PSCCH	1536

12.3.1	FDD	1537

12.4	Demodulation of PSBCH	1538

12.4.1	FDD	1538

12.5	Power imbalance performance with two links	1538

12.5.1	 FDD	1538

12.6	Multiple timing reference test	1540

12.6.1	FDD	1540

12.7	Maximum Sidelink processes test	1543

12.7.1	FDD	1543

12.8	Sustained downlink data rate with active Sidelink	1544

13	Void	1546

14	Performance requirement (V2X Sidelink Communication)	1546

14.1	General	1546

14.1.1	Applicability of requirements	1546

14.2	Demodulation of PSSCH	1547

14.3	Demodulation of PSCCH	1548

14.4	Power imbalance performance with two links	1548

14.5	Demodulation of PSBCH	1549

14.6	Demodulation of PSSCH with eNB based synchronization	1550

14.7	Soft buffer test	1550

14.8	PSCCH/PSSCH decoding capability test	1551

14.9	Sustained downlink data rate with active sidelink	1552

14.10	Soft buffer test (CA)	1553

14.11	PSCCH/PSSCH decoding capability test (CA)	1554

Annex A (normative):	 Measurement channels	1471

A.1	General	1471

A.2	UL reference measurement channels	1471

A.2.1	General	1471

A.2.1.1	Applicability and common parameters	1471

A.2.1.2	Determination of payload size	1471

A.2.1.3	Overview of UL reference measurement channels	1472

A.2.2	Reference measurement channels for FDD	1495

A.2.2.1	Full RB allocation	1495

A.2.2.1.1	QPSK	1495

A.2.2.1.2	16-QAM	1497

A.2.2.1.3	64-QAM	1499

A.2.2.1.4	256 QAM	1500

A.2.2.2	Partial RB allocation	1500

A.2.2.2.1	QPSK	1501

A.2.2.2.2	16-QAM	1504

A.2.2.2.3	64-QAM	1506

A.2.2.2.4	256 QAM	1508

A.2.2.3	Void	1508

A.2.3	Reference measurement channels for TDD	1509

A.2.3.1	Full RB allocation	1509

A.2.3.1.1	QPSK	1509

A.2.3.1.2	16-QAM	1511

A.2.3.1.3	64-QAM	1514

A.2.3.1.4	256 QAM	1514

A.2.3.2	Partial RB allocation	1514

A.2.3.2.1	QPSK	1515

A.2.3.2.2	16-QAM	1519

A.2.3.2.3	64-QAM	1524

A.2.3.2.4	256 QAM	1526

A.2.3.3	Void	1526

A.2.4	Reference measurement channels for UE category NB1	1527

A.2.5	Reference measurement channels for LAA	1528

A.2.5.1	Full RB allocation	1528

A.2.5.1.1	QPSK	1528

A.2.5.1.2	16QAM	1528

A.2.5.1.3	64QAM	1529

A.2.5.2	Partial RB allocation	1529

A.2.5.2.1	QPSK	1529

A.2.5.2.2	16QAM	1530

A.2.5.2.3	64QAM	1530

A.3	DL reference measurement channels	1531

A.3.1	General	1531

A.3.1.1	Overview of DL reference measurement channels	1531

A.3.2	Reference measurement channel for receiver characteristics	1556

A.3.3	Reference measurement channels for PDSCH performance requirements (FDD)	1586

A.3.3.1	Single-antenna transmission (Common Reference Symbols)	1586

A.3.3.2	Multi-antenna transmission (Common Reference Symbols)	1591

A.3.3.2.1	Two antenna ports	1591

A.3.3.2.2	Four antenna ports	1599

A.3.3.3	Reference Measurement Channel for UE-Specific Reference Symbols	1604

A.3.3.3.0	Two antenna ports (no CSI-RS)	1604

A.3.3.3.1	Two antenna port (CSI-RS)	1605

A.3.3.3.2	Four antenna ports (CSI-RS)	1608

A.3.3.3.2A	Eight antenna ports (CSI-RS)	1615

A.3.3.3.3	Twelve antenna port (CSI-RS)	1616

A.3.3.3.4	Sixteen antenna port (CSI-RS)	1617

A.3.3.3.5	Twenty-four antenna port (CSI-RS)	1618

A.3.3.3.6	Thirty-two antenna port (CSI-RS)	1619

A.3.4	Reference measurement channels for PDSCH performance requirements (TDD)	1621

A.3.4.1	Single-antenna transmission (Common Reference Symbols)	1621

A.3.4.2	Multi-antenna transmission (Common Reference Signals)	1629

A.3.4.2.1	Two antenna ports	1629

A.3.4.2.2	Four antenna ports	1642

A.3.4.3	Reference Measurement Channels for UE-Specific Reference Symbols	1649

A.3.4.3.1	Single antenna port (Cell Specific)	1649

A.3.4.3.2	Two antenna ports (Cell Specific)	1650

A.3.4.3.3	Two antenna ports (CSI-RS)	1652

A.3.4.3.4	Four antenna ports (CSI-RS)	1660

A.3.4.3.5	Eight antenna ports (CSI-RS)	1667

A.3.4.3.6	Twelve antenna ports (CSI-RS)	1672

A.3.4.3.7	Sixteen antenna ports (CSI-RS)	1673

A.3.4.3.8	Twenty-four antenna ports (CSI-RS)	1674

A.3.4.3.9	Thirty-two antenna ports (CSI-RS)	1675

A.3.5	Reference measurement channels for PDCCH/PCFICH performance requirements	1677

A.3.5.1	FDD	1677

A.3.5.2	TDD	1677

A.3.5.3	LAA	1678

A.3.6	Reference measurement channels for PHICH performance requirements	1678

A.3.7	Reference measurement channels for PBCH performance requirements	1678

A.3.8	Reference measurement channels for MBMS performance requirements	1679

A.3.8.1	FDD	1679

A.3.8.2	TDD	1683

A.3.9	Reference measurement channels for sustained downlink data rate provided by lower layers	1685

A.3.9.1	FDD	1685

A.3.9.2	TDD	1689

A.3.9.3	FDD (EPDCCH scheduling)	1703

A.3.9.4	TDD (EPDCCH scheduling)	1704

A.3.9.5	LAA	1706

A.3.10	Reference Measurement Channels for EPDCCH performance requirements	1708

A.3.10.1	FDD	1708

A.3.10.2	TDD	1708

A.3.11	Reference Measurement Channels for MPDCCH performance requirements	1708

A.3.11.1	FDD and half-duplex FDD	1708

A.3.11.2	TDD	1709

A.3.12	Reference measurement channels for NPDSCH performance requirements	1710

A.3.12.1	In-band	1710

A.3.12.1.2	Two-antenna transmission	1710

A.3.12.2	Standalone/Guard-band	1712

A.3.12.2.1 Single-antenna transmission	1712

A.3.13	Reference measurement channels for NPDCCH performance requirements	1715

A.3.13.1	Half-duplex FDD	1715

A.3.13.2	TDD	1716

A.3.14	Reference measurement channels for NPBCH performance requirements for Cat NB1 UEs	1716

A.3.15	Reference Measurement Channels for LAA SCell with frame structure Type-3	1717

A.3.15.1	Multi-antenna transmission (Common Reference Symbols)	1717

A.3.15.1.1	Four antenna ports	1717

A.3.15.2	Reference Measurement Channel for UE-Specific Reference Symbols	1717

A.3.15.2.1	Two antenna ports (CSI-RS)	1717

A.3.16	Reference measurement channels for Slot-PDSCH and Subslot-PDSCH performance requirements	1719

A.3.16.1	FDD	1719

A.3.16.2	TDD	1723

A.3.17	Reference measurement channels for SPDCCH performance requirements	1724

A.3.17.1	FDD	1724

A.3.17.2	TDD	1725

A.4	CSI reference measurement channels	1725

A.5	OFDMA Channel Noise Generator (OCNG)	1740

A.5.1	OCNG Patterns for FDD	1740

A.5.1.1	OCNG FDD pattern 1: One sided dynamic OCNG FDD pattern	1740

A.5.1.2	OCNG FDD pattern 2: Two sided dynamic OCNG FDD pattern	1741

A.5.1.3	OCNG FDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1741

A.5.1.3A	OCNG FDD pattern 3A: 49 RB OCNG allocation with MBSFN enhancement in 10 MHz	1742

A.5.1.4	OCNG FDD pattern 4: One sided dynamic OCNG FDD pattern for MBMS transmission	1742

A.5.1.4A	OCNG FDD pattern 4A: One sided dynamic OCNG FDD pattern for enhanced MBMS transmission	1743

A.5.1.5	OCNG FDD pattern 5: One sided dynamic 16QAM modulated OCNG FDD pattern	1744

A.5.1.6	OCNG FDD pattern 6: dynamic OCNG FDD pattern when user data is in 2 non-contiguous blocks	1745

A.5.1.8	OCNG FDD pattern 8: Dynamic OCNG FDD pattern for TM10 transmission	1746

A.5.2	OCNG Patterns for TDD	1747

A.5.2.1	OCNG TDD pattern 1: One sided dynamic OCNG TDD pattern	1747

A.5.2.2	OCNG TDD pattern 2: Two sided dynamic OCNG TDD pattern	1748

A.5.2.3	OCNG TDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1749

A.5.2.4	OCNG TDD pattern 4: One sided dynamic OCNG TDD pattern for MBMS transmission	1749

A.5.2.5	OCNG TDD pattern 5: One sided dynamic 16QAM modulated OCNG TDD pattern	1750

A.5.2.6	OCNG TDD pattern 6: dynamic OCNG TDD pattern when user data is in 2 non-contiguous blocks	1751

A.5.2.8	OCNG TDD pattern 8: Dynamic OCNG TDD pattern for TM10 transmission	1752

A.5.3	OCNG Patterns for Narrowband IoT	1753

A.5.3.1	Narrowband IoT OCNG pattern 1	1753

A.5.4	OCNG Patterns for frame structure type 3	1754

A.5.4.1	OCNG FS3 pattern 1: One sided dynamic OCNG frame structure type 3 pattern	1754

A.5.4.2	OCNG FS3 pattern 2: Two sided dynamic OCNG frame structure 3 pattern	1755

A.6	Sidelink reference measurement channels	1756

A.6.1	General	1756

A.6.1.1	Overview of ProSe reference measurement channels	1756

A.6.2	Reference measurement channel for receiver characteristics	1757

A.6.3	Reference measurement channels for PSDCH performance requirements	1759

A.6.4	Reference measurement channels for PSCCH performance requirements	1760

A.6.5	Reference measurement channels for PSSCH performance requirements	1760

A.6.6	Reference measurement channels for PSBCH performance requirements	1761

A.7	Sidelink reference resource pool configurations	1762

A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests	1762

A.7.1.1	FDD	1762

A.7.1.2	TDD	1765

A.7.2	Reference resource pool configurations for ProSe Direct Communication demodulation tests	1767

A.7.2.1	FDD	1767

A.8	V2X reference measurement channels	1772

A.8.1	General	1772

A.8.1.1	Overview of V2X reference measurement channels	1773

A.8.2	Reference measurement channel for receiver characteristics	1773

A.8.3	Reference measurement channel for transmitter characteristics	1775

A.8.4	Reference measurement for PSCCH performance requirements	1778

A.8.5	Reference measurement for PSSCH performance requirements	1779

A.8.6	Reference measurement for PSBCH performance requirements	1779

A.9	V2X reference resource pool configurations	1780

Annex B (normative):	 Propagation conditions	1783

B.1	Static propagation condition	1783

B.1.1	UE Receiver with 2Rx	1783

B.1.2	UE Receiver with 4Rx	1783

B.1.3	UE Receiver with 8Rx	1784

B.2	Multi-path fading propagation conditions	1785

B.2.1	Delay profiles	1785

B.2.2	Combinations of channel model parameters	1786

B.2.3	MIMO Channel Correlation Matrices	1787

B.2.3.1	Definition of MIMO Correlation Matrices	1787

B.2.3.2	MIMO Correlation Matrices at High, Medium and Low Level	1793

B.2.3A	MIMO Channel Correlation Matrices using cross polarized antennas	1796

B.2.3A.1	Definition of MIMO Correlation Matrices using cross polarized antennas	1797

B.2.3A.2	Spatial Correlation Matrices using cross polarized antennas at eNB and UE sides	1797

B.2.3A.2.1	Spatial Correlation Matrices at eNB side	1797

B.2.3A.2.2	Spatial Correlation Matrices at UE side	1798

B.2.3A.4	Beam steering approach	1801

B.2.3B	MIMO Channel Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1801

B.2.3B.1	Definition of MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1802

B.2.3B.2	Spatial Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1803

B.2.3B.2.1	Spatial Correlation Matrices at eNB side	1803

B.2.3B.2.2	Spatial Correlation Matrices at UE side	1803

B.2.3B.3	MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1803

B.2.3B.4	Beam steering approach	1806

B.2.3B.4A	Beam steering approach with dual cluster beams	1807

B.2.4	Propagation conditions for CQI tests	1808

B.2.4.1	Propagation conditions for CQI tests with multiple CSI processes	1808

B.2.5	Void	1808

B.2.6	MBSFN Propagation Channel Profile	1808

B.2.6.1	Subcarrier spacing 15kHz or 7.5kHz	1808

B.2.6.2	Subcarrier spacing 1.25kHz	1809

B.3	High speed train scenario	1810

B.3A	HST-SFN scenario	1811

B.3B	HST-SFN scenario for 500 km/h speed	1815

B.3C	HST scenario for 500 km/h speed	1815

B.4	Beamforming Model	1815

B.4.1	Single-layer random beamforming (Antenna port 5, 7, or 8)	1815

B.4.1A	Single-layer random beamforming (Antenna port 7, 8, 11 or 13 with enhanced DMRS table configured)	1816

B.4.2	Dual-layer random beamforming (antenna ports 7 and 8)	1816

B.4.3	Generic beamforming model (antenna ports 7-14)	1817

B.4.4	Random beamforming for EPDCCH distributed transmission (Antenna port 107 and 109)	1817

B.4.5	Random beamforming for EPDCCH localized transmission (Antenna port 107, 108, 109 or 110)	1818

B.4.6	Beamforming model for CRI test	1818

B.5	Interference models for enhanced performance requirements Type-A	1819

B.5.1	Dominant interferer proportion	1820

B.5.2	Transmission mode 3 interference model	1820

B.5.3	Transmission mode 4 interference model	1820

B.5.4	Transmission mode 9 interference model	1821

B.6	Interference models for enhanced performance requirements Type-B	1821

B.6.1	Transmission mode 2 interference model	1821

B.6.2	Transmission mode 3 interference model	1821

B.6.3	Transmission mode 4 interference model	1822

B.6.4	Transmission mode 9 interference model	1822

B.6.5	CRS interference model	1823

B.6.6	Random interference model	1823

B.7	Interference models for enhanced downlink control channel performance requirements Type A and B	1824

B.7.1	PDCCH, PCFICH and PHICH interference model	1824

B.8	Burst transmission models for Frame structure type 3	1825

B.8.1	Burst transmission model for one LAA SCell	1825

B.8.2	Burst transmission model for multiple LAA SCell(s)	1825

Annex C (normative):	 Downlink Physical Channels	1827

C.1	General	1827

C.2	Set-up	1827

C.3	Connection	1827

C.3.1	Measurement of Receiver Characteristics	1827

C.3.2	Measurement of Performance requirements	1828

C.3.3	Aggressor cell power allocation for Measurement of Performance Requirements when ABS is Configured	1829

C.3.4	Power Allocation for Measurement of Performance Requirements when Quasi Co-location Type B: same Cell ID	1830

C.3.5	Simplified CA testing method	1830

C.3.6	Measurement of Receiver Characteristics for Narrowband IoT	1831

Annex D (normative):	 Characteristics of the interfering signal	1832

D.1	General	1832

D.2	Interference signals	1832

Annex E (normative):	 Environmental conditions	1833

E.1	 General	1833

E.2	 Environmental	1833

E.2.1	Temperature	1833

E.2.2	Voltage	1833

E.2.3	Vibration	1834

Annex F (normative):	 Transmit modulation	1835

F.1	Measurement Point	1835

F.2	Basic Error Vector Magnitude measurement	1835

F.3	Basic in-band emissions measurement	1836

F.4	Modified signal under test	1836

F.5	Window length	1838

F.5.1	Timing offset	1838

F.5.2	Window length	1838

F.5.3	Window length for normal CP	1838

F.5.4	Window length for Extended CP	1839

F.5.5	Window length for PRACH	1839

F.5.F	Window length for category NB1	1840

F.6	Averaged EVM	1840

F.6.F	Averaged EVM for category NB1	1841

F.7	Spectrum Flatness	1841

Annex G (informative):	 Reference sensitivity level in lower SNR	1842

G.1	General	1842

G.2	Typical receiver sensitivity performance (QPSK)	1842

G.3	Reference measurement channel for REFSENSE in lower SNR	1848

Annex H (normative):	 Modified MPR behavior	1851

H.1	Indication of modified MPR behavior	1851

Annex I (normative):	 Supported Post Antenna Gain	1852

I.1	Declared Supported Post Antenna Gain for UE	1852

Annex J (informative):	 Change history	1853