| 3GPP TS 36.133 V16.8.0 (2020-12) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	89

1	Scope	91

2	References	91

3	Definitions, symbols and abbreviations	93

3.1	Definitions	93

3.2	Symbols	95

3.3	Abbreviations	96

3.4	Test tolerances	99

3.5	Additional notation	99

3.5.1	Groups of bands	99

3.6	General	101

3.6.1	Applicability of requirements in this specification version	101

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	107

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	109

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	110

3.6.2	Applicability of requirements for EN-DC operation	111

3.6.3	Applicability of requirements for NE-DC operation	112

3.6.4	Applicability of requirements for NGEN-DC operation	113

4	E-UTRAN RRC_IDLE state mobility	113

4.1	Cell Selection	113

4.2	Cell Re-selection	113

4.2.1	Introduction	113

4.2.2	Requirements	114

4.2.2.1	Measurement and evaluation of serving cell	115

4.2.2.2	Void	115

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	115

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	117

4.2.2.5	Measurements of inter-RAT cells	120

4.2.2.5.1	Measurements of UTRAN FDD cells	120

4.2.2.5.2	Measurements of UTRAN TDD cells	121

4.2.2.5.3	Measurements of GSM cells	123

4.2.2.5.4	Measurements of HRPD cells	124

4.2.2.5.5	Measurements of cdma2000 1X	125

4.2.2.5.6	Measurements of NR cells	126

4.2.2.5.7	Measurements of NR cells subject to CCA	127

4.2.2.6	Evaluation of cell re-selection criteria	129

4.2.2.7	Maximum interruption in paging reception	129

4.2.2.8	void	130

4.2.2.9	UE measurement capability	130

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	131

4.2.2.10	Reselection to CSG cells	131

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	131

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	132

4.2.2.11	Void	133

4.2.2.12	Void	133

4.2.2.13	Void	133

4.3	Minimization of Drive Tests (MDT)	133

4.3.1	Introduction	134

4.3.2	Measurements	134

4.3.2.1	Requirements	134

4.3.3	Relative Time Stamp Accuracy	134

4.3.3.1	Requirements	134

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	134

4.3.4.1	 Requirements	134

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	135

4.3.5.1	Requirements for timeSinceFailure	135

4.4	MBSFN Measurements	135

4.4.1	Introduction	135

4.4.2	MBSFN RSRP measurements	135

4.4.3	MBSFN RSRQ measurements	135

4.4.4	MCH BLER measurements	136

4.5	Proximity-based Services	136

4.5.1	Introduction	136

4.5.2	Requirements	136

4.5.2.1	Interruptions with ProSe Direct Discovery	136

4.5.2.2	Interruptions with ProSe Direct Communication	136

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	136

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	137

4.6	Cell Selection and Re-selection Requirements for UE category NB1	137

4.6.1	Cell Selection	137

4.6.2	Cell Re-selection	137

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	138

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	139

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	140

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	141

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	142

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	143

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	145

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	146

4.6.2.7	Maximum interruption in paging reception in normal coverage	147

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	148

4.6.2.8	UE measurement capability	148

4.6.2.9	WUS receptions for NB1	148

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	148

4.6.3.1	Introduction	148

4.6.3.2	Requirements on UE synchronization for transmission using PUR	148

4.6.3.3	Requirements on TA validation for transmission using PUR	149

4.7	Cell Selection and Re-selection Requirements for UE category M1	149

4.7.1	Cell Selection	149

4.7.2	Cell Re-selection	149

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	150

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	150

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	151

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	153

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	154

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	155

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	155

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	155

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	156

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	157

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	159

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	161

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	162

4.7.2.3	WUS receptions for UE category M1	162

4.7.3	Channel quality report for UE Category M1 in idle mode	162

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	163

4.7.4.1	Introduction	163

4.7.4.2	Requirements on UE synchronization for transmission using PUR	163

4.7.4.3	Requirements on TA validation for transmission using PUR	163

4.8	Idle State Positioning Measurement Requirements for UE category NB1	164

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	164

4.8.1.1	RSTD Measurement Reporting Delay	166

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	166

4.8.2.1	RSTD Measurement Reporting Delay	167

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	168

4.8.3.1	RSTD Measurement Reporting Delay	169

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	170

4.8.4.1	RSTD Measurement Reporting Delay	171

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	172

4.8.5.1	Measurement Reporting Delay	173

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	173

4.8.6.1	Measurement Reporting Delay	174

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	175

4.8.7.1	Measurement Reporting Delay	176

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	176

4.8.8.1	Measurement Reporting Delay	177

4.9	Idle Mode CA Measurement	178

4.9.1	Introduction	178

4.9.2	Requirement	178

4.9.2.1	Detected cell requirement during state transition and Idle mode	178

4.9.2.2	Measurements of inter-frequency CA candidate cells	179

4.9.2.3	Measurements on serving cell	179

4A	E-UTRAN RRC_INACTIVE state mobility	180

4A.1	Cell Re-selection	180

4A.1.1	Introduction	180

4A.1.2	Requirements	180

4A.1.2.1	UE measurement capability	180

4A.1.2.2	Measurement and evaluation of serving cell	180

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	180

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	180

4A.1.2.5	Evaluation of cell re-selection criteria	180

4A.1.2.6	Maximum interruption in paging reception	180

4A.1.2.7	Measurements of inter-RAT NR cells	180

5	E-UTRAN RRC_CONNECTED state mobility	180

5.1	E-UTRAN Handover	181

5.1.1	Introduction	181

5.1.2	Requirements	181

5.1.2.1	E-UTRAN FDD – FDD	181

5.1.2.1.1	Handover delay	181

5.1.2.1.2	Interruption time	181

5.1.2.2	E-UTRAN FDD – TDD	183

5.1.2.2.1	(Void)	183

5.1.2.2.2	(Void)	183

5.1.2.3	E-UTRAN TDD – FDD	183

5.1.2.3.1	(Void)	183

5.1.2.3.2	(Void)	183

5.1.2.4	E-UTRAN TDD – TDD	183

5.1.2.4.1	Handover delay	183

5.1.2.4.2	Interruption time	183

5.1.2.5	E-UTRAN HD–FDD	185

5.1.2.5.1	Handover delay	185

5.1.2.5.2	Interruption time	185

5.1.2.6	E-UTRAN FDD – FDD conditional handover	187

5.1.2.6.1	Handover delay	187

5.1.2.6.2	Measurement time	187

5.1.2.6.3	Preparation time	187

5.1.2.6.4	Interruption time	187

5.1.2.7	E-UTRAN FDD – TDD conditional handover	188

5.1.2.8	E-UTRAN TDD – FDD conditional handover	188

5.1.2.9	E-UTRAN TDD – TDD conditional handover	188

5.2	Void	188

5.3	Handover to other RATs	188

5.3.1	E-UTRAN - UTRAN FDD Handover	188

5.3.1.1	Introduction	188

5.3.1.1.1	Handover delay	188

5.3.1.1.2	Interruption time	188

5.3.2	E-UTRAN - UTRAN TDD Handover	189

5.3.2.1	Introduction	189

5.3.2.2	Requirements	189

5.3.2.2.1	Handover delay	189

5.3.2.2.2	Interruption time	189

5.3.3	E-UTRAN - GSM Handover	190

5.3.3.1	Introduction	190

5.3.3.2	Requirements	190

5.3.3.2.1	Handover delay	190

5.3.3.2.2	Interruption time	190

5.3.4	E-UTRAN - NR FR1 Handover	191

5.3.4.1	Introduction	191

5.3.4.2	Handover delay	191

5.3.4.3	Interruption time	191

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	192

5.3.4A.1	Introduction	192

5.3.4A.2	Handover delay	192

5.3.4A.3	Interruption time	192

5.3.5	E-UTRAN - NR FR2 Handover	193

5.3.5.1	Introduction	193

5.3.5.2	Handover delay	193

5.3.5.3	Interruption time	193

5.4	Handover to Non-3GPP RATs	194

5.4.1	E-UTRAN – HRPD Handover	194

5.4.1.1	Introduction	194

5.4.1.1.1	Handover delay	194

5.4.1.1.2	Interruption time	194

5.4.2	E-UTRAN – cdma2000 1X Handover	195

5.4.2.1	Introduction	195

5.4.2.1.1	Handover delay	195

5.4.2.1.2	Interruption time	195

5.5	E-UTRAN Handover for Cat-M1 UEs	195

5.5.1	Introduction	195

5.5.2	Requirements in CEModeA	195

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	195

5.5.2.1.1	Handover delay	196

5.5.2.1.2	Interruption time	196

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	196

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	196

5.5.2.3.1	Void	197

5.5.2.3.2	Void	197

5.5.3	Requirements in CEModeB	197

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	197

5.5.3.1.1	Handover delay	197

5.5.3.1.2	Interruption time	197

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	198

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	198

5.6	Void	198

5.7	E-UTRAN DAPS Handover	198

5.7.1	Introduction	198

5.7.2	Requirements	198

5.7.2.1	E-UTRAN FDD – FDD	198

5.7.2.1.1	DAPS Handover delay	198

5.7.2.1.2	Interruption time	199

5.7.2.2	E-UTRAN FDD – TDD	199

5.7.2.3	E-UTRAN TDD – FDD	199

5.7.2.4	E-UTRAN TDD – TDD	200

6	RRC Connection Mobility Control	200

6.1	RRC Re-establishment	200

6.1.1	Introduction	200

6.1.2	Requirements	200

6.1.2.1	UE Re-establishment delay requirement	200

6.2	Random Access	201

6.2.1	Introduction	201

6.2.2	Requirements	201

6.2.2.1	Contention based random access	201

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	201

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	201

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	201

6.2.2.1.4	Void	201

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	201

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	201

6.2.2.2	Non-Contention based random access	202

6.2.2.2.1	Correct behaviour when receiving Random Access Response	202

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	202

6.2.3	Requirements for Cat-M1 UEs	202

6.3	RRC Connection Release with Redirection	202

6.3.1	Introduction	202

6.3.2	Requirements	202

6.3.2.1	RRC connection release with redirection to UTRAN FDD	202

6.3.2.2	RRC connection release with redirection to GERAN	203

6.3.2.3	RRC connection release with redirection to UTRAN TDD	203

6.3.2.4	RRC connection release with redirection to NR	204

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	204

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	205

6.4.1	Introduction	205

6.4.2	Requirements	205

6.5	RRC Re-establishment for NB-IoT UEs	206

6.5.1	Introduction	206

6.5.2	Requirements	206

6.5.2.1	UE Re-establishment delay requirement in normal coverage	206

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	206

6.6	Random Access for UE category NB1	207

6.6.1	Introduction	207

6.6.2	Requirements	207

6.6.2.1	Correct behaviour when receiving Random Access Response reception	207

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	207

6.6.2.3	Correct behaviour when receiving a NACK on msg3	208

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	208

6.6.2.5	Correct behaviour when contention Resolution timer expires	208

6.6.2.6	MSG3-based channel quality report for UE Category NB1	208

6.6.3	Requirements for NPRACH configuration	208

6.7	RRC Re-establishment for Cat-M1 UEs	209

6.7.1	Introduction	209

6.7.2	Requirements	209

6.7.2.1	UE Re-establishment delay requirement for CEModeA	209

6.7.2.2	UE Re-establishment delay requirement for CEModeB	209

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	210

6.8.1	Introduction	210

6.8.2	Requirements	210

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	210

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	211

6.9.1	Introduction	211

6.9.2	Requirements	211

7	Timing and signalling characteristics	212

7.1	UE transmit timing	212

7.1.1	Introduction	212

7.1.2	Requirements	212

7.2	UE timer accuracy	213

7.2.1	Introduction	213

7.2.2	Requirements	213

7.3	Timing Advance	214

7.3.1	Introduction	214

7.3.2	Requirements	214

7.3.2.1	Timing Advance adjustment delay	214

7.3.2.2	Timing Advance adjustment accuracy	214

7.4	Cell phase synchronization accuracy (TDD)	214

7.4.1	Definition	214

7.4.2	Minimum requirements	215

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	215

7.5.1	Introduction	215

7.5.2	eNodeB Synchronization Requirements	215

7.5.2.1	Synchronized E-UTRAN	215

7.5.2.2	Non-Synchronized E-UTRAN	216

7.6	Radio Link Monitoring	216

7.6.1	Introduction	216

7.6.2	Requirements	217

7.6.2.1	Minimum requirement when no DRX is used	217

7.6.2.2	Minimum requirement when DRX is used	218

7.6.2.3	Minimum requirement at transitions	219

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	219

7.6.2.5	Minimum requirement under IDC Interference	220

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	220

7.7.1	Introduction	220

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	220

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	222

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	223

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	224

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	225

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	226

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	226

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	226

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	227

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	228

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	229

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	230

7.7.14	SCell Activation Delay Requirement for Dormant SCell	230

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	231

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	232

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	234

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	234

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	236

7.8	Interruptions with Carrier Aggregation	238

7.8.1	Introduction	238

7.8.2	Requirements	238

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	238

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	238

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	238

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	238

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	239

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	239

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	239

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	239

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	240

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	241

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	241

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	241

7.8.2.13	Interruptions at SRS carrier based switching	242

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	242

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	243

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	243

7.8.2.17	Interruptions during CQI measurement on dormant SCell	243

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	243

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	244

7.8.2.20	Interruptions at SCell hibernation	244

7.8.2.21	Interruptions at direct SCell activation and hibernation	244

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	245

7.9.1	Introduction	245

7.9.2	Minimum Requirements for Interband Carrier Aggregation	245

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	245

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	246

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	246

7.10.1	Introduction	246

7.10.2	Requirements	246

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	246

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	246

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	247

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	247

7.11	Radio Link Monitoring for UE Category 0	247

7.11.1	Introduction	247

7.11.2	Requirements for FD-FDD and TDD	249

7.11.2.1	Minimum requirement when no DRX is used	249

7.11.2.2	Minimum requirement when DRX is used	249

7.11.2.3	Minimum requirement at transitions	250

7.11.3	Requirements for HD-FDD	250

7.11.3.1	Minimum requirement when no DRX is used	250

7.11.3.2	Minimum requirement when DRX is used	250

7.11.3.3	Minimum requirement at transitions	251

7.12	Interruptions with Dual Connectivity	251

7.12.1	Introduction	251

7.12.2	Requirements	251

7.12.2.1	Interruptions at PSCell addition/release	251

7.12.2.2	Interruptions at transitions between active and non-active during DRX	252

7.12.2.3	Interruptions at transitions from non-DRX to DRX	252

7.12.2.4	Interruptions at SCell addition/release	252

7.12.2.5	Interruptions at SCell activation/deactivation	252

7.12.2.6	Interruptions during measurements on SCC	253

7.12.2.7	Interruptions at SRS carrier based switching	253

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	254

7.13.1	Definition	254

7.13.2	Minimum requirements	254

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	254

7.14.1	Introduction	254

7.14.2	PSCell Addition Delay Requirement	254

7.14.3	PSCell Release Delay Requirement	255

7.15	Maximum Receive Timing Difference in Dual Connectivity	255

7.15.1	Introduction	255

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	255

7.16	Proximity-based Services	256

7.16.1	Introduction	256

7.16.2	Requirements	256

7.16.2.1	ProSe UE transmission timing	256

7.16.2.1.1	Serving cell or PCell as timing reference	256

7.16.2.1.2	SCell or non-serving cell as timing reference	256

7.16.3	Interruptions with ProSe	256

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	257

7.16.3.2	Interruptions at ProSe Direct Communication configuration	257

7.16.3.3	Interruptions during ProSe Direct Discovery	257

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	257

7.16.3.5	Interruptions during ProSe Direct Communication	258

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	258

7.16.4.1	Measurement and evaluation of selected cell	258

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	258

7.16.5	Selection / Reselection of ProSe relay UE	259

7.16.6	ProSe operation under deactivated SCell	259

7.17	Maximum Transmission Timing Difference in Dual Connectivity	260

7.17.1	Introduction	260

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	260

7.18.1	Introduction	260

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	260

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	260

7.19	Radio Link Monitoring for UE Category M1	260

7.19.1	Introduction	260

7.19.2	Requirements for FD-FDD and TDD CE mode A	261

7.19.2.1	Minimum requirement when no DRX is used	263

7.19.2.2	Minimum requirement when DRX is used	264

7.19.2.3	Minimum requirement at transitions	265

7.19.3	Requirements for HD-FDD with CE mode A	265

7.19.3.1	Minimum requirement when no DRX is used	265

7.19.3.2	Minimum requirement when DRX is used	265

7.19.3.3	Minimum requirement at transitions	267

7.19.4	Requirements for FD-FDD and TDD with CE mode B	267

7.19.4.1	Minimum requirement when no DRX is used	269

7.19.4.2	Minimum requirement when DRX is used	270

7.19.4.3	Minimum requirement at transitions	271

7.19.5	Requirements for HD-FDD with CE mode B	271

7.19.5.1	Minimum requirement when no DRX is used	271

7.19.5.2	Minimum requirement when DRX is used	271

7.19.5.3	Minimum requirement at transitions	273

7.20	UE transmit timing for NB-IoT	273

7.20.1	Introduction	273

7.20.2	Requirements	273

7.21	 UE timer accuracy for NB-IoT	274

7.21.1	Introduction	274

7.21.2	Requirements	274

7.22	Timing Advance for NB-IoT	274

7.22.1	Introduction	274

7.22.2	Requirements	274

7.22.2.1	Timing Advance adjustment delay	274

7.22.2.2	Timing Advance adjustment accuracy	274

7.23	Radio Link Monitoring for Category NB1 UE	274

7.23.1	Introduction	274

7.23.2	Requirements for Category NB1 UE	274

7.23.2.1	Minimum requirement when no DRX is used	275

7.23.2.2	Minimum requirement when DRX is used	275

7.23.2.3	Minimum requirement at transitions	276

7.24	UE transmit timing for Category M1	276

7.24.1	Introduction	276

7.24.2	Requirements	276

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	277

7.25.1	Definition	277

7.25.2	Minimum requirements	277

7.26	UE transmit timing for Category M2	278

7.26.1	Introduction	278

7.26.2	Requirements	278

7.27	UE timer accuracy for category M1	278

7.27.1	Introduction	278

7.27.2	Requirements	278

7.28	Timing Advance for Category M1	279

7.28.1	Introduction	279

7.28.2	Requirements	279

7.29	Interruptions requirements with FeMBMS	279

7.29.1	Introduction	279

7.29.2	Requirements	279

7.30	Numerology switching delay requirements with FeMBMS	279

7.30.1	Introduction	279

7.30.2	Requirements	279

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	279

7.31.1	Introduction	279

7.31.2	NR PSCell Addition Delay Requirement	280

7.31.3	NR PSCell Release Delay Requirement	280

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	281

7.31A.1	Introduction	281

7.31A.2	NR PSCell Addition Delay Requirement	281

7.31A.3	NR PSCell Release Delay Requirement	282

7.32	Interruptions with EN-DC	282

7.32.1	Introduction	282

7.32.2	Requirements	283

7.32.2.1	Interruptions at PSCell addition/release	283

7.32.2.2	Interruptions at transitions between active and non-active during DRX	283

7.32.2.3	Interruptions at transitions from non-DRX to DRX	283

7.32.2.4	Interruptions at SCell addition/release	283

7.32.2.5	Interruptions at SCell activation/deactivation	283

7.32.2.6	Interruptions during measurements on SCC	284

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	284

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	284

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	284

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	284

7.32.2.7	Interruptions at active BWP switching	284

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	285

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	285

7.32.2.10	Interruptions at SCell hibernation	285

7.32.2.11	Interruptions at direct SCell activation and hibernation	286

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	286

7.32.2.13	Interruptions at NR SRS carrier based switching	286

7.32.2.14	Interruptions at NR SCell dormancy	287

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	287

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	287

7.32.2.15	Interruption during NR measurement with autonomous gaps	287

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	288

7.33.1	Introduction	288

7.33.2	Requirements	288

7.34	Void	288

7.35	Interruptions with SFTD measurements	288

7.35.1	Introduction	288

7.35.2	Requirements	288

7.36	Interruptions with NE-DC	289

7.36.1	Introduction	289

7.36.2	Requirements	289

7.36.2.1	Interruptions at transitions between active and non-active during DRX	289

7.36.2.2	Interruptions at transitions from non-DRX to DRX	289

7.36.2.3	Interruptions at SCell addition/release	289

7.36.2.4	Interruptions at SCell activation/deactivation	290

7.36.2.5	Interruptions during measurements on SCC	290

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	290

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	290

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	291

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	291

7.36.2.6	Interruptions at active BWP switching	291

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	292

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	292

7.36.2.9	Interruptions at SCell hibernation	292

7.36.2.10	Interruptions at direct SCell activation and hibernation	292

7.36.2.11	Interruptions at NR SRS carrier based switching	292

7.36.2.12	Interruptions at NR SCell dormancy	293

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	293

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	293

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	293

7.36.2.14	Interruption during NR measurement with autonomous gaps	294

7.37	Interruptions during NR measurement with autonomous gaps	294

7.37.1	Introduction	294

7.37.2	Requirements	294

8	UE Measurements Procedures in RRC_CONNECTED State	295

8.1	General Measurement Requirements	295

8.1.1	Introduction	295

8.1.2	Requirements	295

8.1.2.1	UE measurement capability	295

8.1.2.1.1	Monitoring of multiple layers using gaps	302

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	303

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	304

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	306

8.1.2.1.2	Network controlled small gap	306

8.1.2.2	E-UTRAN intra frequency measurements	308

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	308

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	313

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	317

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	318

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	319

8.1.2.3	E-UTRAN inter frequency measurements	319

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	320

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	325

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	332

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	332

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	332

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	333

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	335

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	336

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	337

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	344

8.1.2.4	Inter RAT measurements	344

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	344

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	349

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	349

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	353

8.1.2.4.5	E-UTRAN FDD – GSM measurements	353

8.1.2.4.6	E-UTRAN TDD – GSM measurements	358

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	358

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	360

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	360

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	360

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	361

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	361

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	361

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	361

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	363

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	363

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	363

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	363

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	364

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	364

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	366

8.1.2.4.21	E-UTRAN FDD – NR measurements	366

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	370

8.1.2.4.22	E-UTRAN TDD – NR measurements	374

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	374

8.1.2.4.23	Void	374

8.1.2.4.24	Void	374

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	374

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	376

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	376

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	377

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	377

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	378

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	379

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	381

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	383

8.1.2.6.5	Void	384

8.1.2.6.6	Void	384

8.1.2.6.7	Void	384

8.1.2.6.8	Void	384

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	384

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	385

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	386

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	388

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	390

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	392

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	393

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	395

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	397

8.1.2.7	E-UTRAN E-CID Measurements	399

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	399

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	400

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	401

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	402

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	402

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	403

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	405

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	408

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	412

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	415

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	415

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	416

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	416

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	417

8.1.2.10	Void	417

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	417

8.2.1	Introduction	417

8.2.2	Requirements	417

8.3	Measurements for E-UTRA carrier aggregation	422

8.3.1	Introduction	422

8.3.2	Measurements of the primary component carrier	422

8.3.3	Measurements of a secondary component carrier	422

8.3.3.1	Measurements of a secondary component carrier with active SCell	422

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	423

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	423

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	424

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	426

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	426

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	426

8.4.1	Introduction	426

8.4.2	Measurements on the primary component carrier	427

8.4.3	Measurements on a secondary component carrier	428

8.4.4	Measurements on both primary component carrier and a secondary component carrier	428

8.4.5	Measurements on different secondary component carriers	429

8.5	Measurements for UE category 0	430

8.5.1	Introduction	430

8.5.2 	Requirements	431

8.5.2.1 	E-UTRAN intra frequency measurements	431

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	431

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	434

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	436

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	440

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	441

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	441

8.6	Discovery signal measurements	442

8.6.1	Introduction	442

8.6.2	Requirements for CRS based discovery signal measurements	442

8.6.2.1	E-UTRAN intra frequency measurements	442

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	442

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	445

8.6.2.2	E-UTRAN inter frequency measurements	447

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	448

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	450

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	453

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	453

8.6.3	Requirements for CSI-RS based discovery signal measurements	453

8.6.3.1	E-UTRAN intra frequency measurements	453

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	454

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	456

8.6.3.2	E-UTRAN inter frequency measurements	458

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	459

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	461

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	464

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	464

8.7	Discovery signal measurements for E-UTRA carrier aggregation	464

8.7.1	Introduction	464

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	465

8.7.2.1	Measurements of the primary component carrier	465

8.7.2.2	Measurements of a secondary component carrier	465

8.7.2.3	Measurements of a secondary component carrier with active SCell	465

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	465

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	465

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	466

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	468

8.7.3.1	Measurements of the primary component carrier	468

8.7.3.2	Measurements of a secondary component carrier	468

8.7.3.3	Measurements of a secondary component carrier with active SCell	468

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	468

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	468

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	470

8.8	Measurements for E-UTRA dual connectivity	471

8.8.1	Introduction	471

8.8.2	Intra-frequency measurements requirements on PCell	471

8.8.3	Intra-frequency measurements requirements on PSCell	472

8.8.4	Inter-frequency and inter-RAT measurement requirements	472

8.8.5	Intra-frequency measurements with autonomous gaps	472

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	472

8.8.5.2	ECGI reporting delay	473

8.8.6	Inter-frequency measurements with autonomous gaps	473

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	473

8.8.6.2	ECGI reporting delay	474

8.8.7	SSTD Measurements	474

8.8.7.1	Introduction	474

8.8.7.2	SSTD Measurement requirements	474

8.8.7.3	SSTD Measurement Reporting Delay	475

8.8.8	Intra-frequency measurements requirements on SCell	475

8.9	MBSFN Measurements	475

8.9.1	Introduction	475

8.9.2	MBSFN RSRP Measurements	475

8.9.3	MBSFN RSRQ Measurements	476

8.9.4	MCH BLER Measurements	476

8.10	Proximity-based Services	476

8.10.1	Introduction	476

8.10.2	Requirements	476

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	476

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	477

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	477

8.11.1	Introduction	477

8.11.2	CRS based discovery signal measurements	478

8.11.2.1	E-UTRAN intra-frequency measurements	478

8.11.2.1.1	Requirements	478

8.11.2.1.1.1	Requirements when no DRX is used	478

8.11.2.1.1.1.1	Measurement Reporting Requirements	480

8.11.2.1.1.2	Requirements when DRX is used	481

8.11.2.1.1.2.1	Measurement Reporting Requirements	483

8.11.2.2	E-UTRAN inter-frequency measurements	484

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	484

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	488

8.11.3	CSI-RS based discovery signal measurements	488

8.11.3.1	E-UTRAN intra-frequency measurements	488

8.11.3.1.1	Requirements	488

8.11.3.1.1.1	Requirements when no DRX is used	488

8.11.3.1.1.1.1	Measurement Reporting Requirements	490

8.11.3.1.1.2	Requirements when DRX is used	490

8.11.3.1.1.2.1	Measurement Reporting Requirements	491

8.11.3.2	E-UTRAN inter-frequency measurements	492

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	492

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	496

8.11.4	RSSI measurements	496

8.11.4.1	E-UTRAN intra-frequency measurements	496

8.11.4.2	E-UTRAN inter-frequency measurements	496

8.11.5	Channel occupancy measurements	497

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	497

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	497

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	497

8.12.1	Introduction	497

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	497

8.12.2.1	Introduction	497

8.12.2.2	Measurements of a secondary component carrier	497

8.12.2.3	Measurements of a secondary component carrier with active SCell	497

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	498

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	498

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	500

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	503

8.12.3.1	Introduction	503

8.12.3.2	Measurements of a secondary component carrier	503

8.12.3.3	Measurements of a secondary component carrier with active SCell	503

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	503

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	503

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	505

8.13	Measurements for UE Category M1	507

8.13.1	Introduction	507

8.13.2	Requirements for UE category M1 with CE mode A	507

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	508

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	508

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	512

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	514

8.13.2.2	Void	518

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	518

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	519

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	521

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	524

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	524

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	524

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	527

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	529

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	530

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	530

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	530

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	530

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	531

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	531

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	531

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	532

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	533

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	534

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	534

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	534

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	538

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	541

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	545

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	545

8.13.3	Requirements for UE category M1 with CE mode B	546

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	546

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	546

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	551

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	554

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	559

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	560

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	560

8.13.3.2	Void	561

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	561

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	561

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	564

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	566

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	567

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	567

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	567

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	568

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	568

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	568

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	569

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	569

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	569

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	574

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	576

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	580

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	581

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	581

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	583

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	585

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	586

8.14	Measurements for UE category NB1	586

8.14.1	Introduction	586

8.14.2	NB-IoT intra frequency measurements under normal coverage	587

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	587

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	587

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	587

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	587

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	587

8.14.4	Connected mode channel quality report for UE Category NB1	587

8.15	Void	588

8.16	Measurements for UE Category M2	588

8.16.1	Introduction	588

8.16.2	Requirements for UE category M2 with CE mode A	588

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	588

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	589

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	589

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	590

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	590

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	590

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	591

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	591

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	593

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	596

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	596

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	596

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	599

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	601

8.16.3	Requirements for UE category M2 with CE mode B	602

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	602

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	602

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	604

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	607

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	607

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	608

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	610

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	612

8.17	Measurements for E-UTRA – NR Dual Connectivity	613

8.17.1	Introduction	613

8.17.1.1	Measurement Gap Sharing	613

8.17.1A	Intrafrequency Measurements	613

8.17.2	SFTD Measurements	614

8.17.2.1	Introduction	614

8.17.2.2	SFTD Measurement requirements	614

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	614

8.17.2.3	SFTD Measurement Reporting Delay	616

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	616

8.17.3.1	Introduction	616

8.17.3.2	E-UTRAN FDD inter frequency measurements	616

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	616

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	618

8.17.3.3	E-UTRAN TDD inter frequency measurements	620

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	620

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	621

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	623

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	623

8.17.4.1.1	NR Inter-RAT cell identification	624

8.17.4.1.2	NR Inter-RAT measurement	625

8.17.4.1.3	NR Inter-RAT measurement reporting	626

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	627

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	627

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	627

8.17.4A.1.1	NR Inter-RAT cell identification	627

8.17.4A.1.2	NR Inter-RAT measurement	629

8.17.4A.1.3	NR Inter-RAT measurement reporting	629

8.17.4A.1.4	NR inter-RAT RSSI measurements	630

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	630

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	631

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	631

8.17.5.1	Introduction	631

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	631

8.17.5.2.1	Identification of a new UTRA FDD cell	631

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	631

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	632

8.17.5.2.4	Periodic Reporting	632

8.17.5.2.5	Event Triggered Reporting	632

8.17.5.2.6	Event-triggered Periodic Reporting	633

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	633

8.17.5.3.1	Periodic Reporting	634

8.17.5.3.2	Event Triggered Reporting	634

8.17.5.3.3	Event-triggered Periodic Reporting	634

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	635

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	635

8.17.7.1	Introduction	635

8.17.7.2	Identification of a new UTRA FDD cell for SON	635

8.17.7.2.1	Requirements when no DRX is used	635

8.17.7.2.2	Requirements when DRX is used	635

8.17.7.2.3	Reporting Delay	636

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	637

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	637

8.17.9.1	Introduction	637

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	637

8.17.9.2.1	Identification of a new UTRA FDD cell	637

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	637

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	638

8.17.9.2.4	Periodic Reporting	638

8.17.9.2.5	Event Triggered Reporting	638

8.17.9.2.6	Event-triggered Periodic Reporting	639

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	639

8.17.9.3.1	Periodic Reporting	640

8.17.9.3.2	Event Triggered Reporting	640

8.17.9.3.3	Event-triggered Periodic Reporting	640

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	641

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	641

8.17.11.1	Introduction	641

8.17.11.2	Identification of a new UTRA TDD cell for SON	641

8.17.11.2.1	Requirements when no DRX is used	641

8.17.11.2.2	Requirements when DRX is used	641

8.17.11.2.3	Reporting Delay	642

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	642

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	643

8.17.13.1	Introduction	643

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	643

8.17.13.2.1	GSM carrier RSSI	643

8.17.13.2.2	BSIC verification	643

8.17.13.2.3	Enhanced BSIC verification	645

8.17.13.2.4	Periodic Reporting	645

8.17.13.2.5	Event Triggered Reporting	645

8.17.13.2.6	Event-triggered Periodic Reporting	646

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	646

8.17.13.3.1	GSM carrier RSSI	646

8.17.13.3.2	BSIC verification	646

8.17.13.3.3	Periodic Reporting	648

8.17.13.3.4	Event Triggered Reporting	648

8.17.13.3.5	Event-triggered Periodic Reporting	648

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	648

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	648

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	648

8.17.15.1.1	RSTD Measurement Reporting Delay	649

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	649

8.17.15.2.1	RSTD Measurement Reporting Delay	649

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	649

8.17.15.3.1	RSTD Measurement Reporting Delay	650

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	650

8.17.15.4.1	RSTD Measurement Reporting Delay	650

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	651

8.17.16.1	Introduction	651

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	651

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	651

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	651

8.17.17.1	Introduction	651

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	651

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	651

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	651

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	651

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	651

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	651

8.18	Measurements for non-BL/CE UE	651

8.18.1	Introduction	651

8.18.2	Requirements for non-BL/CE UE with CE Mode B	652

8.18.2.1	E-UTRAN intra frequency measurements	652

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	652

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	652

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	653

8.19	Measurements for NR – E-UTRA Dual Connectivity	654

8.19.1	Introduction	654

8.19.2	Intra-frequency Measurements	654

8.19.3	Inter-frequency Measurements	654

8.19.4	Void	654

8.19.5	Intra-frequency E-CID Measurements	654

8.19.6	Intra-frequency measurements with autonomous gaps	655

8.19.6.1	Introduction	655

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	655

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	655

8.19.7	Inter-frequency measurements with autonomous gaps	655

8.19.7.1	Introduction	655

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	655

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	655

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	655

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	655

9	Measurements performance requirements for UE	655

9.1	E-UTRAN measurements	655

9.1.1	Introduction	655

9.1.2	Intra-frequency RSRP Accuracy Requirements	656

9.1.2.1	Absolute RSRP Accuracy	656

9.1.2.2	Relative Accuracy of RSRP	656

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	657

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	658

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	659

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	660

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	661

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	662

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	663

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	663

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	663

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	664

9.1.2B.1	Introduction	664

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	664

9.1.3	Inter-frequency RSRP Accuracy Requirements	665

9.1.3.1	Absolute RSRP Accuracy	665

9.1.3.2	Relative Accuracy of RSRP	666

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	667

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	667

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	668

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	668

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	669

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	670

9.1.3B.1	Introduction	670

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	670

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	670

9.1.4	RSRP Measurement Report Mapping	671

9.1.5	Intra-frequency RSRQ Accuracy Requirements	671

9.1.5.1	Absolute RSRQ Accuracy	671

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	672

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	673

9.1.5.4	Absolute WB-RSRQ Accuracy	674

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	675

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	676

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	676

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	676

9.1.5B.1	Introduction	676

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	676

9.1.6	Inter-frequency RSRQ Accuracy Requirements	677

9.1.6.1	Absolute RSRQ Accuracy	677

9.1.6.2	Relative Accuracy of RSRQ	678

9.1.6.3	Absolute WB-RSRQ Accuracy	679

9.1.6.4	Relative WB-RSRQ Accuracy	679

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	680

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	681

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	681

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	681

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	682

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	683

9.1.6B.1	Introduction	683

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	683

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	683

9.1.7	RSRQ Measurement Report Mapping	684

9.1.8	Power Headroom	684

9.1.8.1	Period	685

9.1.8.2	Reporting Delay	685

9.1.8.3	Void	685

9.1.8.4	Report Mapping	685

9.1.9	UE Rx – Tx time difference	685

9.1.9.1	Measurement Requirement	685

9.1.9.2	Measurement Report mapping	686

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	687

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	688

9.1.10	Reference Signal Time Difference (RSTD)	689

9.1.10.1	Intra-Frequency Accuracy Requirement	689

9.1.10.2	Inter-Frequency Accuracy Requirement	690

9.1.10.3	RSTD Measurement Report Mapping	692

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	692

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	693

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	694

9.1.11	Carrier aggregation measurement accuracy	695

9.1.11.1	Primary component carrier accuracy requirement	696

9.1.11.2	Secondary component carrier accuracy requirement	696

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	696

9.1.11.4	Secondary component carrier relative accuracy requirement	696

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	696

9.1.13	Measurement accuracy for UE category 0	697

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	697

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	697

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	698

9.1.14	Accuracy requirements for Discovery Signal Measurements	699

9.1.14.1	Introduction	699

9.1.14.2	RSRP measurements in discovery signal occasions	699

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	699

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	699

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	699

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	700

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	701

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	701

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	701

9.1.14.3.3	CSI-RSRP measurement report mapping	702

9.1.14.4	RSRQ measurements in discovery signal occasions	702

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	702

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	703

9.1.15.1.1	Primary component carrier accuracy requirement	703

9.1.15.1.2	Secondary component carrier accuracy requirement	703

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	703

9.1.15.1.4	Secondary component carrier relative accuracy requirement	703

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	703

9.1.15.2.1	Primary component carrier accuracy requirement	703

9.1.15.2.2	Secondary component carrier accuracy requirement	703

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	703

9.1.15.2.4	Secondary component carrier relative accuracy requirement	703

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	704

9.1.17	RS-SINR Measurements	704

9.1.17.1	Measurement Report Mapping	704

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	705

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	705

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	705

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	705

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	706

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	707

9.1.18.1	Introduction	707

9.1.18.2	RSRP measurements	707

9.1.18.2.1	RSRP measurement report mapping	707

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	707

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	708

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	708

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	709

9.1.18.3	RSRQ measurements	709

9.1.18.3.1	RSRQ measurement report mapping	709

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	709

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	710

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	710

9.1.18.4	CSI-RSRP measurements	711

9.1.18.4.1	CSI-RSRP measurement report mapping	711

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	711

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	711

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	712

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	712

9.1.18.5	RSSI measurements	713

9.1.18.5.1	RSSI measurement report mapping	713

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	713

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	714

9.1.18.6	Channel occupancy measurements	714

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	714

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	714

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	715

9.1.19.1	Introduction	715

9.1.19.2	Accuracy requirements for measurements on SCC	715

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	715

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	715

9.1.20	SFN and Subframe Time Difference (SSTD)	716

9.1.20.1	SSTD Accuracy Requirement	716

9.1.20.2	SSTD Measurement Report Mapping	716

9.1.21	Measurement accuracy for UE category M1	717

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	717

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	719

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	720

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	722

9.1.21.5	RSRP Measurement Report Mapping	723

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	723

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	724

9.1.21.8	RSRQ Measurement Report Mapping	725

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	725

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	726

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	727

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	728

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	729

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	730

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	731

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	732

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	733

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	734

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	735

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	736

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	739

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	742

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	742

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	745

9.1.22	Measurement accuracy for UE Category NB1	746

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	746

9.1.22.2	Void	747

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	747

9.1.22.4	Void	748

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	748

9.1.22.6	Void	749

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	749

9.1.22.8	Void	750

9.1.22.9	NRSRP Measurement Report Mapping	750

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	750

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	751

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	752

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	753

9.1.22.14	NRSRQ Measurement Report Mapping	754

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	755

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	755

9.1.23	Power Headroom for UE Category NB1	756

9.1.23.1	Period	756

9.1.23.2	Reporting Delay	756

9.1.23.3	Report Mapping for UE Category NB1	756

9.1.23.3.1	Void	758

9.1.23.3.2	Void	758

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	758

9.1.24	Void	759

9.1.25	Measurement accuracy for UE category M2	759

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	759

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	760

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	761

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	762

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	764

9.1.26	Measurement Accuracy for non-BL CE UE	765

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	766

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	768

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	768

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	770

9.1.26.5	RSRP Measurement Report Mapping	771

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	771

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	771

9.1.26.8	RSRQ Measurement Report Mapping	772

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	772

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	773

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	774

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	775

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	776

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	776

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	776

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	777

9.1.27	SFN and frame Timing Difference (SFTD)	778

9.1.27.1	SFTD Accuracy Requirement	778

9.2	UTRAN FDD Measurements	780

9.2.1	UTRAN FDD CPICH RSCP	780

9.2.2	Void	781

9.2.3	UTRAN FDD CPICH Ec/No	781

9.3	UTRAN TDD Measurements	781

9.3.1	UTRAN TDD P-CCPCH RSCP	782

9.3.2	Void	782

9.3.3	Void	782

9.4	GSM Measurements	782

9.4.1	GSM carrier RSSI	782

9.5	CDMA2000 1x RTT Measurements	782

9.5.1	CDMA2000 1x RTT Pilot Strength	782

9.6	PCMAX,c	783

9.6.1	Report Mapping	783

9.6.2 	Estimation Period	783

9.6.3	Reporting Delay	783

9.7	IEEE802.11 Measurements	783

9.7.1	WLAN RSSI	783

9.7.2	WLAN RSSI Measurement Report Mapping	783

9.8	MBSFN Measurements	784

9.8.1	Introduction	784

9.8.2	MBSFN RSRP	784

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	784

9.8.2.2	MBSFN RSRP measurement report mapping	785

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	785

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	786

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	786

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	786

9.8.3	MBSFN RSRQ	787

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	787

9.8.3.2	MBSFN RSRQ measurement report mapping	787

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	788

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	788

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	788

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	789

9.8.4	MCH BLER	789

9.8.4.1	Measurement report mapping for MCH BLER	789

9.8.4.2	Measurement report mapping for MCH Block Number	790

9.9	ProSe Measurements	791

9.9.1	Introduction	791

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	791

9.9.2.1	Absolute S-RSRP Accuracy	791

9.9.2.2	Relative Accuracy of S-RSRP	792

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	793

9.9.3.1	Absolute SD-RSRP Accuracy	793

9.9.3.2	Relative Accuracy of SD-RSRP	793

9.10	V2X Measurements	794

9.10.1	Introduction	794

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	794

9.10.2.1	Absolute S-RSRP Accuracy	794

9.10.2.2	Relative Accuracy of S-RSRP	795

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	795

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	795

9.10.4	S-RSSI Measurement Accuracy Requirements	796

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	796

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	796

9.11	NR Measurements	797

9.11.1	NR SS-RSRP Measurements	797

9.11.2	NR SS-RSRQ Measurements	797

9.11.3	NR SS-SINR Measurements	798

10	Measurements Performance Requirements for E-UTRAN	798

10.1	Received Interference Power	798

10.1.1	Absolute accuracy requirement	798

10.1.2	Relative accuracy requirement	799

10.1.3	Received Interference Power measurement report mapping	799

10.2	Angle of Arrival (AOA)	799

10.2.1	Range/mapping	799

10.3	Timing Advance (TADV)	799

10.3.1	Report mapping	799

11	ProSe Requirements in Any Cell Selection state	800

11.1	Introduction	800

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	800

11.2.1	Introduction	800

11.2.2	ProSe UE transmission timing	800

11.3	Initiation/Cease of SLSS Transmissions	801

11.3.1	Introduction	801

11.3.2	Requirements	801

11.4	Measurements for ProSe in Any Cell Selection State	801

11.4.1	Introduction	801

11.4.2	Requirements	801

11.4.2.1	E-UTRA FDD	801

11.4.2.2	E-UTRA TDD	802

11.5	Selection / Reselection of ProSe Synchronization Reference	802

11.5.1	Introduction	802

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	802

11.5.2.1	Introduction	802

11.5.2.2	Requirements	802

11.6	Void	803

11.7	Selection / Reselection of ProSe relay UE	803

11.7.1	Introduction	803

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	803

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	803

12.1	Introduction	803

12.2	Transmit Timing	804

12.2.1	GNSS as timing reference	804

12.3	Interruption	804

12.4	Reliability of GNSS signal	804

13	V2X Requirements	804

13.1	Introduction	804

13.2	UE Transmit Timing	805

13.2.1	Introduction	805

13.2.2	GNSS as synchronization reference source	805

13.2.3	Serving cell/PCell as synchronization reference source	805

13.2.4	SyncRef UE as synchronization reference source	805

13.3	Initiation/Cease of SLSS Transmissions	805

13.3.1	Introduction	805

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	805

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	806

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	806

13.4	Selection / Reselection of V2X Synchronization Reference Source	807

13.5	Autonomous Resource Selection/Reselection measurements	808

13.5.1	Introduction	808

13.5.2	PSSCH-RSRP measurements	808

13.5.3	S-RSSI measurements	808

13.6	Congestion Control measurements	808

13.7	Interruption	808

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	808

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	808

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	809

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	809

13.8	Reliability of GNSS signal	809

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	810

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	810

Annex A (normative):	Test Cases	788

A.1	Purpose of annex	788

A.2	Requirement classification for statistical testing	788

A.2.1	Types of requirements in TS 36.133	788

A.2.1.1	Time and delay requirements on UE higher layer actions	788

A.2.1.2	Measurements of power levels, relative powers and time	788

A.2.1.3	Implementation requirements	789

A.2.1.4	Physical layer timing requirements	789

A.3	RRM test configurations	790

A.3.1	Reference Measurement Channels	790

A.3.1.1	PDSCH	790

A.3.1.1.1	FDD	790

A.3.1.1.2	TDD	795

A.3.1.1.3	FDD for UE category 0	798

A.3.1.1.4	HD-FDD for UE category 0	799

A.3.1.1.5	TDD for UE category 0	800

A.3.1.1.6	Frame Structure 3	801

A.3.1.2	PCFICH/PDCCH/PHICH	802

A.3.1.2.1	FDD	802

A.3.1.2.2	TDD	802

A.3.1.2.3	HD-FDD for UE category 0	803

A.3.1.2.4	FS 3	803

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	803

A.3.1.3.1	FDD in CEModeA	804

A.3.1.3.2	HD-FDD in CEModeA	804

A.3.1.3.3	TDD in CEModeA	805

A.3.1.3.4	FDD in CEModeB	805

A.3.1.3.5	HD-FDD in CEModeB	806

A.3.1.3.6	TDD in CEModeB	806

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	807

A.3.1.4.1	FDD in CEModeA	807

A.3.1.4.2	HD-FDD in CEModeA	808

A.3.1.4.3	TDD in CEModeA	809

A.3.1.4.4	FDD in CEModeB	810

A.3.1.4.5	 HD-FDD in CEModeB	811

A.3.1.4.6	TDD in CEModeB	811

A.3.1.5	NPDSCH Reference Channel for UE category NB1	812

A.3.1.5.1	HD-FDD in-band operation	812

A.3.1.5.2	Void	813

A.3.1.5.3	HD-FDD standalone operation	813

A.3.1.5.4	Void	814

A.3.1.5.5	HD-FDD guard band operation	814

A.3.1.5.6	Void	815

A.3.1.5.7	TDD in-band operation	815

A.3.1.5.8	TDD standalone operation	815

A.3.1.5.9	TDD guard band operation	816

A.3.1.6	NPDCCH Reference Channel for UE category NB1	816

A.3.1.6.1	In-band operation	816

A.3.1.6.2	Void	817

A.3.1.6.3	Standalone operation	817

A.3.1.6.4	Void	817

A.3.1.6.5	Guard band operation	817

A.3.1.6.6	Void	818

A.3.2	OFDMA Channel Noise Generator (OCNG)	818

A.3.2.1	OCNG Patterns for FDD	818

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	819

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	819

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	820

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	820

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	821

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	822

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	822

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	822

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	823

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	824

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	824

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	825

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	825

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	826

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	826

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	827

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	827

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	828

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	828

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	829

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	829

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	830

A.3.2.2	OCNG Patterns for TDD	830

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	831

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	831

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	832

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	832

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	833

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	834

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	835

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	836

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	836

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	837

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	837

A.3.2.3	OCNG Patterns for Narrowband IoT	838

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	839

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	840

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	840

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	841

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	842

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	843

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	844

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	845

A.3.2.4	OCNG Patterns for V2X sidelink	846

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	846

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	847

A.3.3	Reference DRX Configurations	847

A.3.4	ABS Transmission Configurations	848

A.3.4.1	Non-MBSFN ABS Transmission Configurations	848

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	848

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	848

A.3.4.2	MBSFN ABS Transmission Configurations	849

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	849

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	850

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	850

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	850

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	851

A.3.6.1	Introduction	851

A.3.7	Test Cases with Different Channel Bandwidths	851

A.3.7.1	Introduction	851

A.3.7.2	Principle of testing	851

A.3.8	Antenna Configuration	851

A.3.8.1	Antenna connection for 4 Rx capable UEs	851

A.3.8.1.1 Introduction	851

A.3.8.1.2 Principle of testing	851

A.3.8.1.2.1 Single carrier tests	851

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	852

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	853

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	853

A.3.8.2	Antenna connection for 8 Rx capable UEs	853

A.3.8.2.1	Introduction	853

A.3.8.2.2	Principle of testing	853

A.3.8.2.2.1	Single carrier tests	853

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	854

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	854

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	854

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	854

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	854

A.3.9.1	Introduction	854

A.3.9.2	Principle of testing	854

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	855

A.3.10.1	Introduction	855

A.3.10.2	Principle of testing	855

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	855

A.3.11.1	Introduction	855

A.3.11.2	Principle of Testing	855

A.3.12	Proximity-based Services	855

A.3.12.1	Introduction	855

A.3.12.2	Reference DRX configurations for ProSe tests	855

A.3.12.3	Test Cases with Different Channel Bandwidths	856

A.3.12.3.1	Introduction	856

A.3.12.3.2	Principle of testing	856

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	856

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	863

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	866

A.3.12.6.1	FDD	866

A.3.12.7	Reference measurement channels for ProSe Direct Communication	866

A.3.12.7.1	FDD	866

A.3.12.8	ProSe Receive Traffic Generator	867

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	867

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	867

A.3.13	Time Offset between Cells	868

A.3.13.1	Introduction	868

A.3.13.2	Definition	868

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	868

A.3.14.1	Introduction	868

A.3.14.2	Principle of testing	868

A.3.15	Dual connectivity test cases with different combination of duplex mode	868

A.3.15.1	Introduction	868

A.3.15.2	Principle of testing	868

A.3.16	Reference PRACH Configurations	869

A.3.17	Listen before talk model	869

A.3.17.1	Introduction	869

A.3.17.2	Definition	869

A.3.18	Reference NPRACH Configurations	870

A.3.19	Dual connectivity test cases with different bandwidth combinations	871

A.3.19.1	Introduction	871

A.3.19.2	Principle of testing	871

A.3.20	Category M1 UE Test Cases	872

A.3.20.1	Introduction	872

A.3.20.2	Principle of Cat-M1 UE Testing	872

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	873

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	874

A.3.21.1	Introduction	874

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	874

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	875

A.3.22	Category 1bis UE Test Cases	876

A.3.22.1	Introduction	876

A.3.22.2	Principle of Category 1bis UE Testing	876

A.3.23	Category NB2 UE Test Cases	880

A.3.23.1	Introduction	880

A.3.23.2	Principle of Category NB2 UE Testing	880

A.3.24	V2X sidelink communication	883

A.3.24.1	Introduction	883

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	884

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	888

A.3.25	Category M2 UE Test Cases	889

A.3.25.1	Introduction	889

A.3.25.2	Principle of Cat-M2 UE Testing	889

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	890

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	891

A.3.26.1	Introduction	891

A.3.26.2	Principle of testing	891

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	891

A.3.27.1	Introduction	891

A.3.27.2	Principle of INACTIVE cell re-selection Testing	891

A.4	E-UTRAN RRC_IDLE state	891

A.4.2	Cell Re-Selection	891

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	891

A.4.2.1.1	Test Purpose and Environment	891

A.4.2.1.2	Test Requirements	893

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	894

A.4.2.2.1	Test Purpose and Environment	894

A.4.2.2.2	Test Requirements	895

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	896

A.4.2.3.1	Test Purpose and Environment	896

A.4.2.3.2	Test Requirements	897

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	898

A.4.2.4.1	Test Purpose and Environment	898

A.4.2.4.2	Test Requirements	899

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	900

A.4.2.5.1	Test Purpose and Environment	900

A.4.2.5.2	Test Requirements	901

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	902

A.4.2.6.1	Test Purpose and Environment	902

A.4.2.6.2	Test Requirements	903

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	904

A.4.2.7.1	Test Purpose and Environment	904

A.4.2.7.2	Test Requirements	905

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	906

A.4.2.8.1	Test Purpose and Environment	906

A.4.2.8.2	Test Requirements	907

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	908

A.4.2.9.1	Test Purpose and Environment	908

A.4.2.9.2	Test Requirements	908

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	908

A.4.2.10.1	Test Purpose and Environment	908

A.4.2.10.2	Test Requirements	912

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	912

A.4.2.11.1	Test Purpose and Environment	912

A.4.2.11.2	Test Requirements	916

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	916

A.4.2.12.1	Test Purpose and Environment	916

A.4.2.12.2	Test Requirements	918

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	919

A.4.2.13.1	Test Purpose and Environment	919

A.4.2.13.2	Test Requirements	920

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	921

A.4.2.14.1	Test Purpose and Environment	921

A.4.2.14.2	Test Requirements	922

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	923

A.4.2.15.1	Test Purpose and Environment	923

A.4.2.15.2	Test Requirements	924

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	925

A.4.2.16.1	Test Purpose and Environment	925

A.4.2.16.2	Test Requirements	926

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	927

A.4.2.17.1	Test Purpose and Environment	927

A.4.2.17.2	Test Requirements	928

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	929

A.4.2.18.1	Test Purpose and Environment	929

A.4.2.18.2	Test Requirements	931

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	932

A.4.2.19.1	Test Purpose and Environment	932

A.4.2.19.2	Test Requirements	934

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	935

A.4.2.20.1	Test Purpose and Environment	935

A.4.2.20.2	Test Requirements	936

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	937

A.4.2.21.1	Test Purpose and Environment	937

A.4.2.21.2	Test Requirements	938

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	939

A.4.2.22.1	Test Purpose and Environment	939

A.4.2.22.2	Test Requirements	940

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	941

A.4.2.23.1	Test Purpose and Environment	941

A.4.2.23.2	Test Requirements	942

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	943

A.4.2.24.1	Test Purpose and Environment	943

A.4.2.24.2	Test Requirements	946

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	947

A.4.2.25.1	Test Purpose and Environment	947

A.4.2.25.2	Test Requirements	948

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	949

A.4.2.26.1	Test Purpose and Environment	949

A.4.2.26.2	Test Requirements	950

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	951

A.4.2.27.1	Test Purpose and Environment	951

A.4.2.27.2	Test Requirements	952

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	953

A.4.2.28.1	Test Purpose and Environment	953

A.4.2.28.2	Test Requirements	954

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	955

A.4.2.29.1	Test Purpose and Environment	955

A.4.2.29.2	Test Requirements	956

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	957

A.4.2.30.1	Test Purpose and Environment	957

A.4.2.30.2	Test Requirements	958

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	959

A.4.2.31.1	Test Purpose and Environment	959

A.4.2.31.2	Test Requirements	960

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	961

A.4.2.32.1	Test Purpose and Environment	961

A.4.2.32.2	Test Requirements	962

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	963

A.4.2.33.1	Test Purpose and Environment	963

A.4.2.33.2	Test Requirements	964

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	965

A.4.2.34.1	Test Purpose and Environment	965

A.4.2.34.2	Test Requirements	966

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	967

A.4.2.35.1	Test Purpose and Environment	967

A.4.2.35.2	Test Requirements	969

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	970

A.4.2.36.1	Test Purpose and Environment	970

A.4.2.36.2	Test Requirements	972

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	973

A.4.2.37.1	Test Purpose and Environment	973

A.4.2.37.2	Test Requirements	975

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	976

A.4.2.38.1	Test Purpose and Environment	976

A.4.2.38.2	Test Requirements	978

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	979

A.4.2.39.1	Test Purpose and Environment	979

A.4.2.39.2	Test Requirements	980

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	981

A.4.2.40.1	Test Purpose and Environment	981

A.4.2.40.2	Test Requirements	982

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	983

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	983

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	983

A.4.3.1.1.1	Test Purpose and Environment	983

A.4.3.1.1.2	Test Requirements	986

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	986

A.4.3.1.2.1	Test Purpose and Environment	986

A.4.3.1.2.2	Test Requirements	989

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	989

A.4.3.1.3.1	Test Purpose and Environment	989

A.4.3.1.3.2	Test Requirements	992

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	992

A.4.3.1.4.1	Test Purpose and Environment	992

A.4.3.1.4.2	Test Requirements	993

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	993

A.4.3.1.5.1	Test Purpose and Environment	993

A.4.3.1.5.2	Test Requirements	996

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	997

A.4.3.2.1	Test Purpose and Environment	997

A.4.3.2.1.1	Void	997

A.4.3.2.1.2	1.28Mcps TDD option	997

A.4.3.2.1.3	Void	999

A.4.3.2.2	Test Requirements	999

A.4.3.2.2.1	1.28Mcps TDD option	999

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	999

A.4.3.2A.1	Test Purpose and Environment	999

A.4.3.2A.2	Test Requirements	1003

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1003

A.4.3.3.1	Test Purpose and Environment	1003

A.4.3.3.2	Test Requirements	1006

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1006

A.4.3.3A.1	Test Purpose and Environment	1006

A.4.3.3A.2	Test Requirements	1011

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1012

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1012

A.4.3.4.1.1	Test Purpose and Environment	1012

A.4.3.4.1.2	Test Requirements	1014

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1014

A.4.3.4.2.1	Test Purpose and Environment	1014

A.4.3.4.2.2	Test Requirements	1016

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1016

A.4.3.4.3.1	Test Purpose and Environment	1016

A.4.3.4.3.2	Test Requirements	1019

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1019

A.4.3.4.4.1	Test Purpose and Environment	1019

A.4.3.4.4.2	Test Requirements	1023

A.4.4	E-UTRAN to GSM Cell Re-Selection	1023

A.4.4.1	E-UTRAN FDD – GSM:	1023

A.4.4.1.1	Test Purpose and Environment	1023

A.4.4.1.2	Test Requirements	1025

A.4.4.2	E-UTRAN TDD – GSM:	1025

A.4.4.2.1	Test Purpose and Environment	1025

A.4.4.2.2	Test Requirements	1027

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1028

A.4.5.1	E-UTRAN FDD – HRPD	1028

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1028

A.4.5.1.1.1	Test Purpose and Environment	1028

A.4.5.1.1.2	Test Requirements	1030

A.4.5.2	E-UTRAN TDD – HRPD	1030

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1030

A.4.5.2.1.1	Test Purpose and Environment	1030

A.4.5.2.1.2	Test Requirements	1033

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1033

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1033

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1033

A.4.6.1.1.1	Test Purpose and Environment	1033

A.4.6.1.1.2	Test Requirements	1036

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1036

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1036

A.4.6.2.1.1	Test Purpose and Environment	1036

A.4.6.2.1.2	Test Requirements	1039

A.4.7	Idle State Positioning Measurement for UE category NB1	1039

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1039

A.4.7.1.1	Test Purpose and Environment	1039

A.4.7.1.2	Test Requirements	1044

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1045

A.4.7.2.1	Test Purpose and Environment	1045

A.4.7.4.2	Test Requirements	1049

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1050

A.4.7.3.1	Test Purpose and Environment	1050

A.4.7.3.2	Test Requirements	1054

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1055

A.4.7.4.1	Test Purpose and Environment	1055

A.4.7.4.2	Test Requirements	1059

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1060

A.5.1	E-UTRAN Handover	1060

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1060

A.5.1.1.1	Test Purpose and Environment	1060

A.5.1.1.2	Test Requirements	1062

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1062

A.5.1.2.1	Test Purpose and Environment	1062

A.5.1.2.2	Test Requirements	1064

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1064

A.5.1.3.1	Test Purpose and Environment	1064

A.5.1.3.2	Test Requirements	1066

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1066

A.5.1.4.1	Test Purpose and Environment	1066

A.5.1.4.2	Test Requirements	1068

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1068

A.5.1.5.1	Test Purpose and Environment	1068

A.5.1.5.2	Test Requirements	1070

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1070

A.5.1.6.1	Test Purpose and Environment	1070

A.5.1.6.2	Test Requirements	1072

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1072

A.5.1.7.1	Test Purpose and Environment	1072

A.5.1.7.2	Test Requirements	1075

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1075

A.5.1.8.1	Test Purpose and Environment	1075

A.5.1.8.2	Test Requirements	1078

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1078

A.5.1.9.1	Test Purpose and Environment	1078

A.5.1.9.2	Test Requirements	1079

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1079

A.5.1.10.1	Test Purpose and Environment	1079

A.5.1.10.2	Test Requirements	1081

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1081

A.5.1.11.1	Test Purpose and Environment	1081

A.5.1.11.2	Test Requirements	1083

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1083

A.5.1.12.1	Test Purpose and Environment	1083

A.5.1.12.2	Test Requirements	1085

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1085

A.5.1.13.1	Test Purpose and Environment	1085

A.5.1.13.2	Test Requirements	1087

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1088

A.5.1.14.1	Test Purpose and Environment	1088

A.5.1.14.2	Test Requirements	1089

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1090

A.5.1.15.1	Test Purpose and Environment	1090

A.5.1.15.2	Test Requirements	1091

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1092

A.5.1.16.1	Test Purpose and Environment	1092

A.5.1.16.2	Test Requirements	1093

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1094

A.5.1.17.1	Test Purpose and Environment	1094

A.5.1.17.2	Test Requirements	1095

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1096

A.5.1.18.1	Test Purpose and Environment	1096

A.5.1.18.2	Test Requirements	1097

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1098

A.5.1.19.1	Test Purpose and Environment	1098

A.5.1.19.2	Test Requirements	1099

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1100

A.5.1.20.1	Test Purpose and Environment	1100

A.5.1.20.2	Test Requirements	1101

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1102

A.5.1.21.1	Test Purpose and Environment	1102

A.5.1.21.2	Test Requirements	1103

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1103

A.5.1.22.1	Test Purpose and Environment	1103

A.5.1.22.2	Test Requirements	1105

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1105

A.5.1.23.1	Test Purpose and Environment	1105

A.5.1.23.2	Test Requirements	1107

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1107

A.5.1.24.1	Test Purpose and Environment	1107

A.5.1.24.2	Test Requirements	1109

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1109

A.5.1.25.1	Test Purpose and Environment	1109

A.5.1.25.2	Test Requirements	1111

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1112

A.5.1.26.1	Test Purpose and Environment	1112

A.5.1.26.2	Test Requirements	1113

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1114

A.5.1.27.1	Test Purpose and Environment	1114

A.5.1.27.2	Test Requirements	1115

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1116

A.5.1.28.1	Test Purpose and Environment	1116

A.5.1.28.2	Test Requirements	1117

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1118

A.5.1.29.1	Test Purpose and Environment	1118

A.5.1.29.2	Test Requirements	1119

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1120

A.5.1.30.1	Test Purpose and Environment	1120

A.5.1.30.2	Test Requirements	1121

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1122

A.5.1.31.1	Test Purpose and Environment	1122

A.5.1.31.2	Test Requirements	1123

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1124

A.5.1.32.1	Test Purpose and Environment	1124

A.5.1.32.2	Test Requirements	1125

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1126

A.5.1.33.1	Test Purpose and Environment	1126

A.5.1.13.2	Test Requirements	1127

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1128

A.5.1.34.1	Test Purpose and Environment	1128

A.5.1.34.2	Test Requirements	1129

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1130

A.5.1.35.1	Test Purpose and Environment	1130

A.5.1.35.2	Test Requirements	1131

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1132

A.5.1.36.1	Test Purpose and Environment	1132

A.5.1.36.2	Test Requirements	1133

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1134

A.5.1.37.1	Test Purpose and Environment	1134

A.5.1.37.2	Test Requirements	1135

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1136

A.5.1.38.1	Test Purpose and Environment	1136

A.5.1.38.2	Test Requirements	1137

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1138

A.5.1.39.1	Test Purpose and Environment	1138

A.5.1.39.2	Test Requirements	1139

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1140

A.5.1.40.1	Test Purpose and Environment	1140

A.5.1.40.2	Test Requirements	1141

A.5.2	E-UTRAN Handover to other RATs	1142

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1142

A.5.2.1.1	Test Purpose and Environment	1142

A.5.2.1.2	Test Requirements	1144

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1145

A.5.2.2.1	Test Purpose and Environment	1145

A.5.2.2.2	Test Requirements	1148

A.5.2.3 E-UTRAN FDD- GSM Handover	1148

A.5.2.3.1	Test Purpose and Environment	1148

A.5.2.3.2	Test Requirements	1149

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1150

A.5.2.4.1	Test Purpose and Environment	1150

A.5.2.4.1.1	Void	1150

A.5.2.4.1.2	1.28 Mcps TDD option	1150

A.5.2.4.1.3	Void	1152

A.5.2.4.2	Test Requirements	1152

A.5.2.4.2.1	Void	1152

A.5.2.4.2.2	1.28 Mcps TDD option	1152

A.5.2.4.2.3	Void	1152

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1152

A.5.2.5.1	Test Purpose and Environment	1152

A.5.2.5.1.3	Void	1155

A.5.2.5.2	Test Requirements	1155

A.5.2.5.2.1	Void	1155

A.5.2.5.2.2	1.28 Mcps TDD option	1155

A.5.2.5.2.3	Void	1155

A.5.2.6	E-UTRAN TDD - GSM Handover	1155

A.5.2.6.1	Test Purpose and Environment	1155

A.5.2.6.2	Test Requirements	1157

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1158

A.5.2.7.1	Test Purpose and Environment	1158

A.5.2.7.2	Test Requirements	1160

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1160

A.5.2.8.1	Test Purpose and Environment	1160

A.5.2.8.2	Test Requirements	1161

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1162

A.5.2.9.1	Test Purpose and Environment	1162

A.5.2.9.2	Test Requirements	1163

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1164

A.5.2.10.1	Test Purpose and Environment	1164

A.5.2.10.2	Test Requirements	1166

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1166

A.5.2.10A.1	Test Purpose and Environment	1166

A.5.2.10A.2	Test Requirements	1169

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1169

A.5.2.10B.1	Test Purpose and Environment	1169

A.5.2.10B.2	Test Requirements	1172

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1172

A.5.2.11.1	Test Purpose and Environment	1172

A.5.2.11.2	Test Requirements	1172

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1173

A.5.3.1	E-UTRAN FDD – HRPD Handover	1173

A.5.3.1.1	Test Purpose and Environment	1173

A.5.3.1.2	Test Requirements	1175

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1175

A.5.3.2.1	Test Purpose and Environment	1175

A.5.3.2.2	Test Requirements	1178

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1178

A.5.3.3.1	Test Purpose and Environment	1178

A.5.3.3.2	Test Requirements	1180

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1181

A.5.3.4.1	Test Purpose and Environment	1181

A.5.3.4.2	Test Requirements	1182

A.5.3.5	E-UTRAN TDD – HRPD Handover	1183

A.5.3.5.1	Test Purpose and Environment	1183

A.5.3.5.2	Test Requirements	1185

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1186

A.5.3.6.1	Test Purpose and Environment	1186

A.5.3.6.2	Test Requirements	1189

A.6	RRC Connection Control	1189

A.6.1	RRC Re-establishment	1189

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1189

A.6.1.1.1	Test Purpose and Environment	1189

A.6.1.1.2	Test Requirements	1190

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1191

A.6.1.2.1	Test Purpose and Environment	1191

A.6.1.2.2	Test Requirements	1192

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1193

A.6.1.3.1	Test Purpose and Environment	1193

A.6.1.3.2	Test Requirements	1194

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1195

A.6.1.4.1	Test Purpose and Environment	1195

A.6.1.4.2	Test Requirements	1196

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1197

A.6.1.5.1	Test Purpose and Environment	1197

A.6.1.5.2	Test Requirements	1197

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1197

A.6.1.6.1	Test Purpose and Environment	1197

A.6.1.6.2	Test Requirements	1199

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1200

A.6.1.7.1	Test Purpose and Environment	1200

A.6.1.7.2	Test Requirements	1201

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1202

A.6.1.8.1	Test Purpose and Environment	1202

A.6.1.8.2	Test Requirements	1203

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1204

A.6.1.9.1	Test Purpose and Environment	1204

A.6.1.9.2	Test Requirements	1205

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1206

A.6.1.10.1	Test Purpose and Environment	1206

A.6.1.10.2	Test Requirements	1207

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1208

A.6.1.11.1	Test Purpose and Environment	1208

A.6.1.11.2	Test Requirements	1209

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1210

A.6.1.12.1	Test Purpose and Environment	1210

A.6.1.12.2	Test Requirements	1211

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1212

A.6.1.13.1	Test Purpose and Environment	1212

A.6.1.13.2	Test Requirements	1213

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1214

A.6.1.14.1	Test Purpose and Environment	1214

A.6.1.14.2	Test Requirements	1215

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1216

A.6.1.15.1	Test Purpose and Environment	1216

A.6.1.15.2	Test Requirements	1218

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1219

A.6.1.16.1	Test Purpose and Environment	1219

A.6.1.16.2	Test Requirements	1221

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1222

A.6.1.17.1	Test Purpose and Environment	1222

A.6.1.17.2	Test Requirements	1223

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1224

A.6.1.18.1	Test Purpose and Environment	1224

A.6.1.18.2	Test Requirements	1225

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1226

A.6.1.19.1	Test Purpose and Environment	1226

A.6.1.19.2	Test Requirements	1227

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1228

A.6.1.20.1	Test Purpose and Environment	1228

A.6.1.20.2	Test Requirements	1229

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1230

A.6.1.21.1	Test Purpose and Environment	1230

A.6.1.21.2	Test Requirements	1231

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1232

A.6.1.22.1	Test Purpose and Environment	1232

A.6.1.22.2	Test Requirements	1233

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1234

A.6.1.23.1	Test Purpose and Environment	1234

A.6.1.23.2	Test Requirements	1236

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1237

A.6.1.24.1	Test Purpose and Environment	1237

A.6.1.24.2	Test Requirements	1239

A.6.2	Random Access	1240

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1240

A.6.2.1.1	Test Purpose and Environment	1240

A.6.2.1.2	Test Requirements	1241

A.6.2.1.2.1	Random Access Response Reception	1241

A.6.2.1.2.2	No Random Access Response Reception	1241

A.6.2.1.2.3	Receiving a NACK on msg3	1241

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1242

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1242

A.6.2.1.2.6	Contention Resolution Timer expiry	1242

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1242

A.6.2.2.1	Test Purpose and Environment	1242

A.6.2.2.2	Test Requirements	1243

A.6.2.2.2.1	Random Access Response Reception	1244

A.6.2.2.2.2	No Random Access Response Reception	1244

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1244

A.6.2.3.1	Test Purpose and Environment	1244

A.6.2.3.2	Test Requirements	1246

A.6.2.3.2.1	Random Access Response Reception	1246

A.6.2.3.2.2	No Random Access Response reception	1246

A.6.2.3.2.3	Receiving a NACK on msg3	1246

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1247

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1247

A.6.2.3.2.6	Contention Resolution Timer expiry	1247

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1247

A.6.2.4.1	Test Purpose and Environment	1247

A.6.2.4.2	Test Requirements	1249

A.6.2.4.2.1	Random Access Response Reception	1249

A.6.2.4.2.2	No Random Access Response Reception	1249

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1249

A.6.2.5.1	Test Purpose and Environment	1249

A.6.2.5.2	Test Requirements	1250

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1250

A.6.2.6.1	Test Purpose and Environment	1250

A.6.2.6.2	Test Requirements	1250

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1251

A.6.2.7.1	Test Purpose and Environment	1251

A.6.2.7.2	Test Requirements	1253

A.6.2.7.2.1	Random Access Response Reception	1253

A.6.2.7.2.2	No Random Access Response Reception	1253

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1253

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1254

A.6.2.8.1	Test Purpose and Environment	1254

A.6.2.8.2	Test Requirements	1256

A.6.2.8.2.1	Random Access Response Reception	1256

A.6.2.8.2.2	No Random Access Response Reception	1256

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1256

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1257

A.6.2.9.1	Test Purpose and Environment	1257

A.6.2.9.2	Test Requirements	1260

A.6.2.9.2.1	Random Access Response Reception	1260

A.6.2.9.2.2	No Random Access Response Reception	1261

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1261

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1262

A.6.2.10.1	Test Purpose and Environment	1262

A.6.2.10.2	Test Requirements	1264

A.6.2.10.2.1	Random Access Response Reception	1264

A.6.2.10.2.2	No Random Access Response Reception	1265

A.6.2.10.2.3	Receiving a NACK on msg3	1265

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1265

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1265

A.6.2.10.2.6	Contention Resolution Timer expiry	1265

A.6.2.10.2.7	PRACH Resource Selection	1266

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1266

A.6.2.11.1	Test Purpose and Environment	1266

A.6.2.11.2	Test Requirements	1268

A.6.2.11.2.1	Random Access Response Reception	1268

A.6.2.11.2.2	No Random Access Response Reception	1269

A.6.2.11.2.3	Receiving a NACK on msg3	1269

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1269

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1269

A.6.2.11.2.6	Contention Resolution Timer expiry	1269

A.6.2.11.2.7	PRACH Resource Selection	1270

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1270

A.6.2.12.1	Test Purpose and Environment	1270

A.6.2.12.2	Test Requirements	1272

A.6.2.12.2.1	Random Access Response Reception	1272

A.6.2.12.2.2	No Random Access Response Reception	1273

A.6.2.12.2.3	Receiving a NACK on msg3	1273

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1273

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1273

A.6.2.12.2.6	Contention Resolution Timer expiry	1273

A.6.2.12.2.7	PRACH Resource Selection	1274

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1274

A.6.2.13.1	Test Purpose and Environment	1274

A.6.2.13.2	Test Requirements	1276

A.6.2.13.2.1	Random Access Response Reception	1276

A.6.2.13.2.2	No Random Access Response Reception	1277

A.6.2.13.2.3	Receiving a NACK on msg3	1277

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1277

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1277

A.6.2.13.2.6	Contention Resolution Timer expiry	1277

A.6.2.13.2.7	PRACH Resource Selection	1278

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1278

A.6.2.14.1	Test Purpose and Environment	1278

A.6.2.14.2	Test Requirements	1280

A.6.2.14.2.1	Random Access Response Reception	1280

A.6.2.14.2.2	No Random Access Response Reception	1281

A.6.2.14.2.3	Receiving a NACK on msg3	1281

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1281

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1281

A.6.2.14.2.6	Contention Resolution Timer expiry	1281

A.6.2.14.2.7	PRACH Resource Selection	1282

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1282

A.6.2.15.1	Test Purpose and Environment	1282

A.6.2.15.2	Test Requirements	1284

A.6.2.15.2.1	Random Access Response Reception	1284

A.6.2.15.2.2	No Random Access Response Reception	1285

A.6.2.15.2.3	Receiving a NACK on msg3	1285

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1285

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1285

A.6.2.15.2.6	Contention Resolution Timer expiry	1285

A.6.2.15.2.7	PRACH Resource Selection	1286

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1286

A.6.2.16.1	Test Purpose and Environment	1286

A.6.2.16.2	Test Requirements	1289

A.6.2.16.2.1	Random Access Response Reception	1289

A.6.2.16.2.2	No Random Access Response Reception	1290

A.6.2.16.2.3	Receiving a NACK on msg3	1290

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1290

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1290

A.6.2.16.2.6	Contention Resolution Timer expiry	1290

A.6.2.16.2.7	NPRACH Resource Selection	1290

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1291

A.6.2.17.1	Test Purpose and Environment	1291

A.6.2.17.2	Test Requirements	1294

A.6.2.17.2.1	Random Access Response Reception	1294

A.6.2.17.2.2	No Random Access Response Reception	1295

A.6.2.17.2.3	Receiving a NACK on msg3	1295

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1295

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1295

A.6.2.17.2.6	Contention Resolution Timer expiry	1295

A.6.2.17.2.7	NPRACH Resource Selection	1295

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1296

A.6.2.18.1	Test Purpose and Environment	1296

A.6.2.18.2	Test Requirements	1299

A.6.2.18.2.1	Random Access Response Reception	1299

A.6.2.18.2.2	No Random Access Response Reception	1300

A.6.2.18.2.3	Receiving a NACK on msg3	1300

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1300

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1300

A.6.2.18.2.6	Contention Resolution Timer expiry	1300

A.6.2.18.2.7	NPRACH Resource Selection	1300

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1301

A.6.2.19.1	Test Purpose and Environment	1301

A.6.2.19.2	Test Requirements	1303

A.6.2.19.2.1	Random Access Response Reception	1303

A.6.2.19.2.2	No Random Access Response Reception	1304

A.6.2.19.2.3	Receiving a NACK on msg3	1304

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1304

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1304

A.6.2.19.2.6	Contention Resolution Timer expiry	1304

A.6.2.19.2.7	NPRACH Resource Selection	1304

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1305

A.6.2.20.1	Test Purpose and Environment	1305

A.6.2.20.2	Test Requirements	1307

A.6.2.20.2.1	Random Access Response Reception	1307

A.6.2.20.2.2	No Random Access Response Reception	1308

A.6.2.20.2.3	Receiving a NACK on msg3	1308

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1308

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1308

A.6.2.20.2.6	Contention Resolution Timer expiry	1308

A.6.2.20.2.7	NPRACH Resource Selection	1308

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1309

A.6.2.21.1	Test Purpose and Environment	1309

A.6.2.21.2	Test Requirements	1311

A.6.2.21.2.1	Random Access Response Reception	1311

A.6.2.21.2.2	No Random Access Response Reception	1312

A.6.2.21.2.3	Receiving a NACK on msg3	1312

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1312

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1312

A.6.2.21.2.6	Contention Resolution Timer expiry	1312

A.6.2.21.2.7	NPRACH Resource Selection	1312

A.6.3	RRC Connection Release with Redirection	1313

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1313

A.6.3.1.1	Test Purpose and Environment	1313

A.6.3.1.2	Test Requirements	1315

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1315

A.6.3.2.1	Test Purpose and Environment	1315

A.6.3.2.2	Test Requirements	1317

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1317

A.6.3.3.1	Test Purpose and Environment	1317

A.6.3.3.2	Test Requirements	1318

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1319

A.6.3.4.1	Test Purpose and Environment	1319

A.6.3.4.2	Test Requirements	1320

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1321

A.6.3.5.1	Test Purpose and Environment	1321

A.6.3.5.2	Test Requirements	1323

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1323

A.6.3.6.1	Test Purpose and Environment	1323

A. 6.3.6.2	Test Requirements	1326

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1326

A.6.3.7.1	Test Purpose and Environment	1326

A.6.3.7.2	Test Requirements	1329

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1329

A.6.3.8.1	Test Purpose and Environment	1329

A.6.3.8.2	Test Requirements	1332

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1332

A.6.3.9.1	Test Purpose and Environment	1332

A.6.3.9.2	Test Requirements	1334

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1334

A.6.3.10.1	Test Purpose and Environment	1334

A.6.3.10.2	Test Requirements	1336

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1336

A.6.3.11.1	Test Purpose and Environment	1336

A.6.3.11.2	Test Requirements	1338

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1338

A.6.3.12.1	Test Purpose and Environment	1338

A.6.3.12.2	Test Requirements	1341

A.7	Timing and Signalling Characteristics	1342

A.7.1	UE Transmit Timing	1342

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1342

A.7.1.1.1	Test Purpose and Environment	1342

A.7.1.1.2	Test Requirements	1343

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1344

A.7.1.2.1	Test Purpose and Environment	1344

A.7.1.2.2	Test Requirements	1346

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1347

A.7.1.3.1	Test Purpose and Environment	1347

A.7.1.3.2	Test Requirements	1349

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1350

A.7.1.4.1	Test Purpose and Environment	1350

A.7.1.4.2	Test Requirements	1351

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1352

A.7.1.4A.1	Test Purpose and Environment	1352

A.7.1.4A.2	Test Requirements	1352

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1352

A.7.1.5.1	Test Purpose and Environment	1352

A.7.1.5.2	Test Requirements	1353

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1353

A.7.1.6.1	Test Purpose and Environment	1353

A.7.1.6.2	Test Requirements	1355

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1356

A.7.1.7.1	Test Purpose and Environment	1356

A.7.1.7.2	Test Requirements	1357

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1358

A.7.1.7A.1	Test Purpose and Environment	1358

A.7.1.7A.2	Test Requirements	1358

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1358

A.7.1.7B.1	Test Purpose and Environment	1358

A.7.1.7B.2	Test Requirements	1358

A.7.1.8	Void	1359

A.7.1.8.1	Void	1359

A.7.1.8.2	Void	1359

A.7.1.9	Void	1359

A.7.1.9.1	Void	1359

A.7.1.9.2	Void	1359

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1359

A.7.1.10.1	Test Purpose and Environment	1359

A.7.1.10.2	Test Requirements	1361

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1361

A.7.1.11.1	Test Purpose and Environment	1361

A.7.1.11.2	Test Requirements	1363

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1363

A.7.1.12.1	Test Purpose and Environment	1363

A.7.1.12.2	Test Requirements	1365

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1365

A.7.1.13.1	Test Purpose and Environment	1365

A.7.1.13.2	Test Requirements	1367

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1368

A.7.1.14.1	Test Purpose and Environment	1368

A.7.1.14.2	Test Requirements	1369

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1369

A.7.1.15.1	Test Purpose and Environment	1369

A.7.1.15.2	Test Requirements	1370

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1371

A.7.1.16.1	Test Purpose and Environment	1371

A.7.1.16.2	Test Requirements	1372

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1373

A.7.1.17.1	Test Purpose and Environment	1373

A.7.1.17.2	Test Requirements	1375

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1376

A.7.1.18.1	Test Purpose and Environment	1376

A.7.1.18.2	Test Requirements	1378

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1379

A.7.1.19.1	Test Purpose and Environment	1379

A.7.1.19.2	Test Requirements	1380

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1380

A.7.1.20.1	Test Purpose and Environment	1380

A.7.1.20.2	Test Requirements	1382

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1382

A.7.1.21.1	Test Purpose and Environment	1382

A.7.1.21.2	Test Requirements	1384

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1384

A.7.1.22.1	Test Purpose and Environment	1384

A.7.1.22.2	Test Requirements	1386

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1386

A.7.1.23.1	Test Purpose and Environment	1386

A.7.1.23.2	Test Requirements	1388

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1388

A.7.1.24.1	Test Purpose and Environment	1388

A.7.1.24.2	Test Requirements	1389

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1390

A.7.1.25.1	Test Purpose and Environment	1390

A.7.1.25.2	Test Requirements	1391

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1392

A.7.1.26.1	Test Purpose and Environment	1392

A.7.1.26.2	Test Requirements	1393

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1394

A.7.1.27.1	Test Purpose and Environment	1394

A.7.1.27.2	Test Requirements	1396

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1396

A.7.1.28.1	Test Purpose and Environment	1396

A.7.1.28.2	Test Requirements	1398

A.7.2	UE Timing Advance	1399

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1399

A.7.2.1.1	Test Purpose and Environment	1399

A.7.2.1.2	Test Requirements	1400

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1401

A.7.2.2.1	Test Purpose and Environment	1401

A.7.2.2.2	Test Requirements	1403

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1403

A.7.2.3.1	Test Purpose and Environment	1403

A.7.2.3.2	Test Requirements	1403

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1403

A.7.2.4.1	Test Purpose and Environment	1403

A.7.2.4.2	Test Requirements	1406

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1406

A.7.2.5.1	Test Purpose and Environment	1406

A.7.2.5.2	Test Requirements	1408

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1408

A.7.2.5A.1	Test Purpose and Environment	1408

A.7.2.5A.2	Test Requirements	1409

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1409

A.7.2.5B.1	Test Purpose and Environment	1409

A.7.2.5B.2	Test Requirements	1409

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1409

A.7.2.6.1	Test Purpose and Environment	1409

A.7.2.6.2	Test Requirements	1412

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1412

A.7.2.7.1	Test Purpose and Environment	1412

A.7.2.7.2	Test Requirements	1414

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1414

A.7.2.8.1	Test Purpose and Environment	1414

A.7.2.8.2	Test Requirements	1416

A.7.2.9.2	Test Requirements	1417

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1418

A.7.2.10.1	Test Purpose and Environment	1418

A.7.2.10.2	Test Requirements	1419

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1419

A.7.2.11.1	Test Purpose and Environment	1419

A.7.2.11.2	Test Requirements	1421

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1421

A.7.2.12.1	Test Purpose and Environment	1421

A.7.2.12.2	Test Requirements	1423

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1423

A.7.2.13.1	Test Purpose and Environment	1423

A.7.2.13.2	Test Requirements	1425

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1425

A.7.2.14.1	Test Purpose and Environment	1425

A.7.2.14.2	Test Requirements	1427

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1427

A.7.2.15.1	Test Purpose and Environment	1427

A.7.2.15.2	Test Requirements	1429

A.7.3	Radio Link Monitoring	1429

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1430

A.7.3.1.1	Test Purpose and Environment	1430

A.7.3.1.2	Test Requirements	1434

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1434

A.7.3.2.1	Test Purpose and Environment	1434

A.7.3.2.2	Test Requirements	1437

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1437

A.7.3.3.1	Test Purpose and Environment	1437

A.7.3.3.2	Test Requirements	1441

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1441

A.7.3.4.1	Test Purpose and Environment	1441

A.7.3.4.2	Test Requirements	1444

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1444

A.7.3.5.1	Test Purpose and Environment	1444

A.7.3.5.2	Test Requirements	1448

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1449

A.7.3.6.1	Test Purpose and Environment	1449

A.7.3.6.2	Test Requirements	1452

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1452

A.7.3.7.1	Test Purpose and Environment	1452

A.7.3.7.2	Test Requirements	1455

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1455

A.7.3.8.1	Test Purpose and Environment	1455

A.7.3.8.2	Test Requirements	1458

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1458

A.7.3.9.2	Test Requirements	1461

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1461

A.7.3.10.1	Test Purpose and Environment	1461

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1464

A.7.3.11.1	Test Purpose and Environment	1464

A.7.3.11.2	Test Requirements	1468

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1468

A.7.3.12.1	Test Purpose and Environment	1468

A.7.3.12.2	Test Requirements	1472

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1472

A.7.3.13.1	Test Purpose and Environment	1472

A.7.3.13.2	Test Requirements	1475

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1475

A.7.3.14.1	Test Purpose and Environment	1475

A.7.3.14.2	Test Requirements	1478

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1478

A.7.3.15.1	Test Purpose and Environment	1478

A.7.3.15.2	Test Requirements	1481

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1481

A.7.3.16.1	Test Purpose and Environment	1481

A.7.3.16.2	Test Requirements	1485

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1485

A.7.3.17.1	Test Purpose and Environment	1485

A.7.3.17.2	Test Requirements	1488

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1488

A.7.3.18.1	Test Purpose and Environment	1488

A.7.3.18.2	Test Requirements	1492

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1492

A.7.3.19.1	Test Purpose and Environment	1492

A.7.3.19.2	Test Requirements	1496

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1496

A.7.3.20.1	Test Purpose and Environment	1496

A.7.3.20.2	Test Requirements	1500

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1500

A.7.3.21.1	Test Purpose and Environment	1500

A.7.3.21.2	Test Requirements	1504

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1504

A.7.3.22.1	Test Purpose and Environment	1504

A.7.3.22.2	Test Requirements	1508

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1508

A.7.3.23.1	Test Purpose and Environment	1508

A.7.3.23.2	Test Requirements	1509

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1509

A.7.3.24.1	Test Purpose and Environment	1509

A.7.3.24.2	Test Requirements	1510

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1510

A.7.3.25.1	Test Purpose and Environment	1510

A.7.3.25.2	Test Requirements	1511

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1511

A.7.3.26.1	Test Purpose and Environment	1511

A.7.3.26.2	Test Requirements	1514

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1514

A.7.3.27.1	Test Purpose and Environment	1514

A.7.3.27.2	Test Requirements	1517

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1517

A.7.3.28.1	Test Purpose and Environment	1517

A.7.3.28.2	Test Requirements	1520

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1520

A.7.3.29.1	Test Purpose and Environment	1520

A.7.3.29.2	Test Requirements	1523

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1523

A.7.3.30.1	Test Purpose and Environment	1523

A.7.3.30.2	Test Requirements	1526

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1526

A.7.3.31.1	Test Purpose and Environment	1526

A.7.3.31.2	Test Requirements	1529

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1529

A.7.3.32.1	Test Purpose and Environment	1529

A.7.3.32.2	Test Requirements	1532

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1532

A.7.3.33.1	Test Purpose and Environment	1532

A.7.3.33.2	Test Requirements	1535

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1535

A.7.3.34.1	Test Purpose and Environment	1535

A.7.3.34.2	Test Requirements	1538

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1538

A.7.3.35.1	Test Purpose and Environment	1538

A.7.3.35.2	Test Requirements	1541

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1541

A.7.3.36.1	Test Purpose and Environment	1541

A.7.3.36.2	Test Requirements	1544

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1544

A.7.3.37.1	Test Purpose and Environment	1544

A.7.3.37.2	Test Requirements	1547

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1547

A.7.3.38.1	Test Purpose and Environment	1547

A.7.3.38.2	Test Requirements	1550

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1550

A.7.3.39.1	Test Purpose and Environment	1550

A.7.3.39.2	Test Requirements	1553

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1554

A.7.3.40.1	Test Purpose and Environment	1554

A.7.3.40.2	Test Requirements	1556

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1557

A.7.3.41.1	Test Purpose and Environment	1557

A.7.3.41.2	Test Requirements	1560

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1560

A.7.3.42.1	Test Purpose and Environment	1560

A.7.3.42.2	Test Requirements	1563

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1563

A.7.3.43.1	Test Purpose and Environment	1563

A.7.3.43.2	Test Requirements	1566

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1566

A.7.3.44.1	Test Purpose and Environment	1566

A.7.3.44.2	Test Requirements	1569

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1570

A.7.3.45.1	Test Purpose and Environment	1570

A.7.3.45.2	Test Requirements	1572

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1573

A.7.3.46.1	Test Purpose and Environment	1573

A.7.3.46.2	Test Requirements	1576

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1576

A.7.3.47.1	Test Purpose and Environment	1576

A.7.3.47.2	Test Requirements	1579

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1579

A.7.3.48.1	Test Purpose and Environment	1579

A.7.3.48.2	Test Requirements	1582

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1582

A.7.3.49.1	Test Purpose and Environment	1582

A.7.3.49.2	Test Requirements	1585

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1585

A.7.3.50.1	Test Purpose and Environment	1585

A.7.3.50.2	Test Requirements	1588

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1588

A.7.3.51.1	Test Purpose and Environment	1588

A.7.3.51.2	Test Requirements	1591

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1591

A.7.3.52.1	Test Purpose and Environment	1591

A.7.3.52.2	Test Requirements	1594

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1594

A.7.3.53.1	Test Purpose and Environment	1594

A.7.3.53.2	Test Requirements	1597

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1597

A.7.3.54.1	Test Purpose and Environment	1597

A.7.3.54.2	Test Requirements	1600

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1600

A.7.3.55.1	Test Purpose and Environment	1600

A.7.3.55.2	Test Requirements	1603

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1603

A.7.3.56.1	Test Purpose and Environment	1603

A.7.3.56.2	Test Requirements	1606

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1606

A.7.3.57.1	Test Purpose and Environment	1606

A.7.3.57.2	Test Requirements	1609

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1609

A.7.3.58.1	Test Purpose and Environment	1609

A.7.3.58.2	Test Requirements	1612

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1612

A.7.3.59.1	Test Purpose and Environment	1612

A.7.3.59.2	Test Requirements	1615

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1615

A.7.3.60.1	Test Purpose and Environment	1615

A.7.3.60.2	Test Requirements	1619

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1619

A.7.3.61.1	Test Purpose and Environment	1619

A.7.3.61.2	Test Requirements	1623

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1623

A.7.3.62.1	Test Purpose and Environment	1623

A.7.3.62.2	Test Requirements	1627

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1627

A.7.3.63.1	Test Purpose and Environment	1627

A.7.3.63.2	Test Requirements	1631

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1631

A.7.3.64.1	Test Purpose and Environment	1631

A.7.3.64.2	Test Requirements	1634

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1635

A.7.3.65.1	Test Purpose and Environment	1635

A.7.3.65.2	Test Requirements	1638

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1639

A.7.3.66.1	Test Purpose and Environment	1639

A.7.3.66.2	Test Requirements	1641

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1642

A.7.3.67.1	Test Purpose and Environment	1642

A.7.3.67.2	Test Requirements	1645

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1646

A.7.3.68.1	Test Purpose and Environment	1646

A.7.3.68.2	Test Requirements	1648

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1648

A.7.3.69.1	Test Purpose and Environment	1648

A.7.3.69.2	Test Requirements	1650

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1650

A.7.3.70.1	Test Purpose and Environment	1650

A.7.3.70.2	Test Requirements	1652

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1652

A.7.3.71.1	Test Purpose and Environment	1652

A.7.3.71.2	Test Requirements	1655

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1655

A.7.3.72.1	Test Purpose and Environment	1655

A.7.3.72.2	Test Requirements	1657

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1657

A.7.3.73.1	Test Purpose and Environment	1657

A.7.3.73.2	Test Requirements	1659

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1659

A.7.3.74.1	Test Purpose and Environment	1659

A.7.3.74.2	Test Requirements	1662

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1662

A.7.3.75.1	Test Purpose and Environment	1662

A.7.3.75.2	Test Requirements	1664

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1665

A.7.3.76.1	Test Purpose and Environment	1665

A.7.3.76.2	Test Requirements	1668

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1668

A.7.3.77.1	Test Purpose and Environment	1668

A.7.3.77.2	Test Requirements	1671

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1671

A.7.3.78.1	Test Purpose and Environment	1671

A.7.3.78.2	Test Requirements	1674

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1674

A.7.3.79.1	Test Purpose and Environment	1674

A.7.3.79.2	Test Requirements	1677

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1677

A.7.3.80.1	Test Purpose and Environment	1677

A.7.3.80.2	Test Requirements	1680

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1680

A.7.3.81.1	Test Purpose and Environment	1680

A.7.3.81.2	Test Requirements	1683

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1683

A.7.3.82.1	Test Purpose and Environment	1683

A.7.3.82.2	Test Requirements	1686

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1686

A.7.3.83.1	Test Purpose and Environment	1686

A.7.3.83.2	Test Requirements	1687

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1688

A.7.3.84.1	Test Purpose and Environment	1688

A.7.3.84.2	Test Requirements	1690

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1690

A.7.3.85.1	Test Purpose and Environment	1690

A.7.3.85.2	Test Requirements	1691

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1692

A.7.3.86.1	Test Purpose and Environment	1692

A.7.3.86.2	Test Requirements	1694

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1694

A.7.3.87.1	Test Purpose and Environment	1694

A.7.3.87.2	Test Requirements	1696

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1697

A.7.3.88.1	Test Purpose and Environment	1697

A.7.3.88.2	Test Requirements	1701

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1701

A.7.3.89.1	Test Purpose and Environment	1701

A.7.3.89.2	Test Requirements	1705

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1705

A.7.3.90.1	Test Purpose and Environment	1705

A.7.3.90.2	Test Requirements	1709

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1709

A.7.3.91.1	Test Purpose and Environment	1709

A.7.3.91.2	Test Requirements	1713

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1713

A.7.3.92.1	Test Purpose and Environment	1713

A.7.3.92.2	Test Requirements	1716

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1717

A.7.3.93.1	Test Purpose and Environment	1717

A.7.3.93.2	Test Requirements	1720

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1721

A.7.3.94.1	Test Purpose and Environment	1721

A.7.3.94.2	Test Requirements	1723

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1724

A.7.3.95.1	Test Purpose and Environment	1724

A.7.3.95.2	Test Requirements	1727

A.7.4	Interruption for Dual Connectivity	1728

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1728

A.7.4.1.1	Test Purpose and Environment	1728

A.7.4.1.2	Test Requirements	1730

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1730

A.7.4.2.1	Test Purpose and Environment	1730

A.7.4.2.2	Test Requirements	1732

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1732

A.7.4.3.1	Test Purpose and Environment	1732

A.7.4.3.2	Test Requirements	1734

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1734

A.7.4.4.1	Test Purpose and Environment	1734

A.7.4.4.2	Test Requirements	1737

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1737

A.7.4.5.1	Test Purpose and Environment	1737

A.7.4.5.2	Test Requirements	1739

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1739

A.7.4.6.1	Test Purpose and Environment	1739

A.7.4.6.2	Test Requirements	1741

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1741

A.7.4.7.1	Test Purpose and Environment	1741

A.7.4.7.2	Test Requirements	1744

A.7.5	Proximity-based Services	1744

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1744

A.7.5.1.1	Test Purpose and Environment	1744

A.7.5.1.2	Test Requirements	1745

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1746

A.7.5.2.1	Test Purpose and Environment	1746

A.7.5.1.2	Test Requirements	1746

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1747

A.7.5.3.1	Test Purpose and Environment	1747

A.7.5.3.2	Test Requirements	1748

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1749

A.7.5.4.1	Test Purpose and Environment	1749

A.7.5.4.2	Test Requirements	1750

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1751

A.7.5.5.1	Test Purpose and Environment	1751

A.7.5.5.2	Test Requirements	1753

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1754

A.7.5.6.1	Test Purpose and Environment	1754

A.7.5.6.2	Test Requirements	1755

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1756

A.7.5.7.1	Test Purpose and Environment	1756

A.7.5.7.2	Test Requirements	1758

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1758

A.7.5.8.1	Test Purpose and Environment	1758

A.7.5.8.2	Test Requirements	1760

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1761

A.7.5.9.1	Test Purpose and Environment	1761

A.7.5.9.2	Test Requirements	1763

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1764

A.7.5.10.1	Test Purpose and Environment	1764

A.7.5.10.2	Test Requirements	1766

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1767

A.7.5.11.1	Test Purpose and Environment	1767

A.7.5.11.2	Test Requirements	1769

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1769

A.7.5.12.1	Test Purpose and Environment	1769

A.7.5.12.2	Test Requirements	1773

A.7.6	Interruption for carrier aggregation	1774

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1774

A.7.6.1.1	Test Purpose and Environment	1774

A.7.6.1.2	Test Requirements	1776

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1776

A.7.6.2.1	Test Purpose and Environment	1776

A.7.6.2.2	Test Requirements	1779

A.8	UE Measurements Procedures	1779

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1779

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1779

A.8.1.1.1	Test Purpose and Environment	1779

A.8.1.1.2	Test Requirements	1781

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1781

A.8.1.2.1	Test Purpose and Environment	1781

A.8.1.2.2	Test Requirements	1783

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1783

A.8.1.3.1	Test Purpose and Environment	1783

A.8.1.3.2	Test Requirements	1786

A.8.1.4	Void	1786

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1786

A.8.1.5.1	Test Purpose and Environment	1786

A.8.1.5.2	Test Requirements	1788

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1788

A.8.1.6.1	Test Purpose and Environment	1788

A.8.1.6.2	Test Requirements	1790

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1790

A.8.1.7.1	Test Purpose and Environment	1790

A.8.1.7.2	Test Requirements	1792

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1793

A.8.1.8.1	Test Purpose and Environment	1793

A.8.1.8.2	Test Requirements	1795

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1796

A.8.1.9.1	Test Purpose and Environment	1796

A.8.1.9.2	Test Requirements	1796

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1796

A.8.1.10.1	Test Purpose and Environment	1796

A.8.1.10.2	Test Requirements	1797

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1797

A.8.1.11.1	Test Purpose and Environment	1797

A.8.1.11.2	Test Requirements	1799

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1800

A.8.1.12.1	Test Purpose and Environment	1800

A.8.1.12.2	Test Requirements	1801

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1802

A.8.1.13.1	Test Purpose and Environment	1802

A.8.1.13.2	Test Requirements	1804

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1804

A.8.1.14.1	Test Purpose and Environment	1804

A.8.1.14.2	Test Requirements	1806

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1807

A.8.1.15.1	Test Purpose and Environment	1807

A.8.1.15.2	Test Requirements	1808

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1809

A.8.1.16.1	Test Purpose and Environment	1809

A.8.1.16.2	Test Requirements	1811

A.8.1.17	Void	1811

A.8.1.18	Void	1811

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1811

A.8.1.19.1	Test Purpose and Environment	1811

A.8.1.19.2	Test Requirements	1813

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1814

A.8.1.20.1	Test Purpose and Environment	1814

A.8.1.20.2	Test Requirements	1816

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1816

A.8.1.21.1	Test Purpose and Environment	1816

A.8.1.21.2	Test Requirements	1818

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1819

A.8.1.22.1	Test Purpose and Environment	1819

A.8.1.22.2	Test Requirements	1821

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1821

A.8.1.23.1	Test Purpose and Environment	1821

A.8.1.23.2	Test Requirements	1822

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1823

A.8.1.24.1	Test Purpose and Environment	1823

A.8.1.24.2	Test Requirements	1825

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1825

A.8.1.25.1	Test Purpose and Environment	1825

A.8.1.25.2	Test Requirements	1827

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1827

A.8.1.26.1	Test Purpose and Environment	1827

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1830

A.8.1.27.1	Test Purpose and Environment	1830

A.8.1.27.2	Test Requirements	1832

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1832

A.8.1.28.1	Test Purpose and Environment	1832

A.8.1.28.2	Test Requirements	1834

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1834

A.8.1.29.1	Test Purpose and Environment	1834

A.8.1.29.2	Test Requirements	1836

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1837

A.8.1.30.1	Test Purpose and Environment	1837

A.8.1.30.2	Test Requirements	1839

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1839

A.8.1.31.1	Test Purpose and Environment	1839

A.8.1.31.2	Test Requirements	1841

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1842

A.8.1.32.1	Test Purpose and Environment	1842

A.8.1.32.2	Test Requirements	1843

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1844

A.8.1.33.1	Test Purpose and Environment	1844

A.8.1.33.2	Test Requirements	1845

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1846

A.8.1.34.1	Test Purpose and Environment	1846

A.8.1.34.2	Test Requirements	1847

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1848

A.8.1.35.1	Test Purpose and Environment	1848

A.8.1.35.2	Test Requirements	1849

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1850

A.8.1.36.1	Test Purpose and Environment	1850

A.8.1.36.2	Test Requirements	1851

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1852

A.8.1.37.1	Test Purpose and Environment	1852

A.8.1.37.2	Test Requirements	1854

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1854

A.8.1.38.1	Test Purpose and Environment	1854

A.8.1.38.2	Test Requirements	1855

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1856

A.8.1.39.1	Test Purpose and Environment	1856

A.8.1.39.2	Test Requirements	1858

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1858

A.8.1.40.1	Test Purpose and Environment	1858

A.8.1.40.2	Test Requirements	1860

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1860

A.8.1.41.1	Test Purpose and Environment	1860

A.8.1.41.2	Test Requirement	1862

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1862

A.8.1.42.1	Test Purpose and Environment	1862

A.8.1.42.2	Test Requirement	1863

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1863

A.8.1.43.1	Test Purpose and Environment	1863

A.8.1.43.2	Test Requirements	1866

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1866

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1866

A.8.2.1.1	Test Purpose and Environment	1866

A.8.2.1.2	Test Requirements	1868

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1869

A.8.2.2.1	Test Purpose and Environment	1869

A.8.2.2.2	Test Requirements	1871

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1871

A.8.2.3.1	Test Purpose and Environment	1871

A.8.2.3.2	Test Requirements	1873

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1874

A.8.2.4.1	Test Purpose and Environment	1874

A.8.2.4.2	Test Requirements	1876

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1876

A.8.2.5.1	Test Purpose and Environment	1876

A.8.2.5.2	Test Requirements	1878

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1879

A.8.2.6.1	Test Purpose and Environment	1879

A.8.2.6.2	Test Requirements	1881

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1882

A.8.2.7.1	Test Purpose and Environment	1882

A.8.2.7.2	Test Requirements	1883

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1884

A.8.2.8.1	Test Purpose and Environment	1884

A.8.2.8.2	Test Requirements	1886

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1886

A.8.2.9.1	Test Purpose and Environment	1886

A.8.2.9.2	Test Requirements	1888

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1889

A.8.2.10.1	Test Purpose and Environment	1889

A.8.2.10.2	Test Requirements	1891

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1891

A.8.2.11.1	Test Purpose and Environment	1891

A.8.2.11.2	Test Requirements	1893

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1893

A.8.2.12.1	Test Purpose and Environment	1893

A.8.2.12.2	Test Requirements	1895

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1896

A.8.2.13.1	Test Purpose and Environment	1896

A.8.2.13.2	Test Requirements	1898

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1898

A.8.2.14.1	Test Purpose and Environment	1898

A.8.2.14.2	Test Requirement	1900

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1900

A.8.2.15.1	Test Purpose and Environment	1900

A.8.2.15.2	Test Requirements	1902

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1902

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1902

A.8.3.1.1	Test Purpose and Environment	1902

A.8.3.1.2	Test Requirements	1904

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1904

A.8.3.2.1	Test Purpose and Environment	1904

A.8.3.2.2	Test Requirements	1907

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1907

A.8.3.3.1	Test Purpose and Environment	1907

A.8.3.3.2	Test Requirements	1910

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1910

A.8.3.4.1	Test Purpose and Environment	1910

A.8.3.4.2	Test Requirements	1912

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1912

A.8.3.5.2	Test Requirements	1914

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1914

A.8.3.6.1	Test Purpose and Environment	1914

A.8.3.6.2	Test Requirements	1916

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1917

A.8.3.7.1	Test Purpose and Environment	1917

A.8.3.7.2	Test Requirements	1919

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1920

A.8.3.8.1	Test Purpose and Environment	1920

A.8.3.8.2	Test Requirements	1923

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1923

A.8.3.9.1	Test Purpose and Environment	1923

A.8.3.9.2	Test Requirements	1926

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1927

A.8.3.10.1	Test Purpose and Environment	1927

A.8.3.10.2	Test Requirements	1928

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1928

A.8.3.11.1	Test Purpose and Environment	1928

A.8.3.11.2	Test Requirement	1930

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1931

A.8.3.12.1	Test Purpose and Environment	1931

A.8.3.12.2	Test Requirement	1932

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1933

A.8.3.13.1	Test Purpose and Environment	1933

A.8.3.13.2	Test Requirement	1934

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1935

A.8.3.14.1	Test Purpose and Environment	1935

A.8.3.14.2	Test Requirement	1936

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1937

A.8.3.15.1	Test Purpose and Environment	1937

A.8.3.15.2	Test Requirement	1938

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1939

A.8.3.16.1	Test Purpose and Environment	1939

A.8.3.16.2	Test Requirement	1941

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1941

A.8.3.17.1	Test Purpose and Environment	1941

A.8.3.17.2	Test Requirement	1944

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1944

A.8.3.18.1	Test Purpose and Environment	1944

A.8.3.18.2	Test Requirement	1947

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1947

A.8.3.19.1	Test Purpose and Environment	1947

A.8.3.19.2	Test Requirement	1950

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1950

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1950

A.8.4.1.1	Test Purpose and Environment	1950

A.8.4.1.2	Test Requirements	1952

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1952

A.8.4.2.1	Test Purpose and Environment	1952

A.8.4.2.2	Test Requirements	1955

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1955

A.8.4.3.1	Test Purpose and Environment	1955

A.8.4.3.2	Test Requirements	1957

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1958

A.8.4.4.1	Test Purpose and Environment	1958

A.8.4.4.2	Test Requirements	1960

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1961

A.8.4.5.1	Test Purpose and Environment	1961

A.8.4.5.2	Test Requirements	1963

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1963

A.8.4.6.1	Test Purpose and Environment	1963

A.8.4.6.2	Test Requirements	1964

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1964

A.8.4.7.1	Test Purpose and Environment	1964

A.8.4.7.2	Test Requirements	1968

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1968

A.8.4.8.1	Test Purpose and Environment	1968

A.8.4.8.2	Test Requirements	1971

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1971

A.8.4.9.1	Test Purpose and Environment	1971

A.8.4.9.2	Test Requirements	1975

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1975

A.8.4.10.1	Test Purpose and Environment	1975

A.8.4.10.2	Test Requirements	1977

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1977

A.8.4.11.1	Test Purpose and Environment	1977

A.8.4.11.2	Test Requirement	1979

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1979

A.8.4.12.1	Test Purpose and Environment	1979

A.8.4.12.2	Test Requirement	1981

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1982

A.8.4.13.1	Test Purpose and Environment	1982

A.8.4.13.2	Test Requirement	1983

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1984

A.8.4.14.1	Test Purpose and Environment	1984

A.8.4.14.2	Test Requirement	1986

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1986

A.8.4.15.1	Test Purpose and Environment	1986

A.8.4.15.2	Test Requirement	1989

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1989

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1989

A.8.5.1.1	Test Purpose and Environment	1989

A.8.5.1.2	Test Requirements	1991

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1991

A.8.5.2.1	Test Purpose and Environment	1991

A.8.5.2.2	Test Requirements	1993

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1993

A.8.5.3.1	Test Purpose and Environment	1993

A.8.5.3.2	Test Requirements	1996

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1996

A.8.5.4.1	Test Purpose and Environment	1996

A.8.5.4.2	Test Requirements	1998

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1998

A.8.5.5.1	Test Purpose and Environment	1998

A.8.5.5.2	Test Requirements	2001

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2001

A.8.5.6.1	Test Purpose and Environment	2001

A.8.5.6.2	Test Requirements	2002

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2003

A.8.5.7.1	Test Purpose and Environment	2003

A.8.5.7.2	Test Requirements	2003

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2003

A.8.5.8.1	Test Purpose and Environment	2003

A.8.5.8.2	Test Requirements	2006

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2006

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2006

A.8.6.1.1	Test Purpose and Environment	2006

A.8.6.1.2	Test Requirements	2008

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2008

A.8.6.2.1	Test Purpose and Environment	2008

A.8.6.2.2	Test Requirements	2011

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2011

A.8.6.3.1	Test Purpose and Environment	2011

A.8.6.3.2	Test Requirements	2014

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2014

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2014

A.8.7.1.1	Test Purpose and Environment	2014

A.8.7.1.1.1	Void	2014

A.8.7.1.1.2	1.28 Mcps TDD option	2014

A.8.7.1.1.3	Void	2016

A.8.7.1.2	Test Requirements	2016

A.8.7.1.2.1	Void	2016

A.8.7.1.2.2	1.28 Mcps TDD option	2016

A.8.7.1.2.3	Void	2016

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2016

A.8.7.2.1	Test Purpose and Environment	2016

A.8.7.2.2	Test Requirements	2019

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2020

A.8.7.3.1	Test Purpose and Environment	2020

A.8.7.3.2	Test Parameters	2020

A.8.7.3.3	Test Requirements	2021

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2022

A.8.7.4.1	Test Purpose and Environment	2022

A.8.7.4.2	Test Requirements	2024

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2024

A.8.7.5.1	Test Purpose and Environment	2024

A.8.7.5.2	Test Requirements	2026

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2026

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2026

A.8.7A.1.1	Test Purpose and Environment	2026

A.8.7A.1.2	Test Requirements	2029

A.8.8	E-UTRAN FDD – GSM Measurements	2029

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2029

A.8.8.1.1	Test Purpose and Environment	2029

A.8.8.1.2	Test Requirements	2031

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2031

A.8.8.2.1	Test Purpose and Environment	2031

A.8.8.2.2	Test Requirements	2033

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2034

A.8.8.3.1	Test Purpose and Environment	2034

A.8.8.3.2	Test Requirements	2035

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2036

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2036

A.8.9.1.1	Test Purpose and Environment	2036

A.8.9.1.2	Test Requirements	2037

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2038

A.8.9.2.1	Test Purpose and Environment	2038

A.8.9.2.2	Test Requirements	2040

A.8.10	E-UTRAN TDD – GSM Measurements	2040

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2040

A.8.10.1.1	Test Purpose and Environment	2040

A.8.10.1.2	Test Requirements	2041

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2042

A.8.10.2.1	Test Purpose and Environment	2042

A.8.10.2.2	Test Requirements	2044

A.8.11	Monitoring of Multiple Layers	2044

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2044

A.8.11.1.1	Test Purpose and Environment	2044

A.8.11.1.2	Test Requirements	2046

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2047

A.8.11.2.1	Test Purpose and Environment	2047

A.8.11.2.2	Test Requirements	2048

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2049

A.8.11.3.1	Test Purpose and Environment	2049

A.8.11.3.2	Test Requirements	2051

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2051

A.8.11.4.1	Test Purpose and Environment	2051

A.8.11.4.2	Test Requirements	2054

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2054

A.8.11.5.1	Test Purpose and Environment	2054

A.8.11.5.2	Test Requirements	2056

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2057

A.8.11.6.1	Test Purpose and Environment	2057

A.8.11.6.2	Test Requirements	2059

A.8.12	RSTD Intra-frequency Measurements	2060

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2060

A.8.12.1.1	Test Purpose and Environment	2060

A.8.12.1.2	Test Requirements	2063

A.8.12.1.2A	Test Requirements for UE Category 1bis	2064

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2064

A.8.12.2.1	Test Purpose and Environment	2064

A.8.12.2.2	Test Requirements	2068

A.8.12.2.2A	Test Requirements for UE Category 1bis	2069

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2069

A.8.12.3.1	Test Purpose and Environment	2069

A.8.12.3.2	Test Requirements	2073

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2074

A.8.12.4.1	Test Purpose and Environment	2074

A.8.12.4.2	Test Requirements	2077

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2078

A.8.12.5.1	Test Purpose and Environment	2078

A.8.12.5.2	Test Requirements	2081

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2082

A.8.12.6.1	Test Purpose and Environment	2082

A.8.12.6.2	Test Requirements	2085

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2086

A.8.12.7.1	Test Purpose and Environment	2086

A.8.12.7.2	Test Requirements	2089

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2090

A.8.12.8.1	Test Purpose and Environment	2090

A.8.12.8.2	Test Requirements	2093

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2094

A.8.12.9.1	Test Purpose and Environment	2094

A.8.12.9.2	Test Requirements	2097

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2098

A.8.12.10.1	Test Purpose and Environment	2098

A.8.12.10.2	Test Requirements	2101

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2102

A.8.12.11.1	Test Purpose and Environment	2102

A.8.12.11.2	Test Requirements	2106

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2107

A.8.12.12.1	Test Purpose and Environment	2107

A.8.12.12.2	Test Requirements	2110

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2111

A.8.12.13.1	Test Purpose and Environment	2111

A.8.12.13.2	Test Requirements	2114

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2115

A.8.12.14.1	Test Purpose and Environment	2115

A.8.12.14.2	Test Requirements	2119

A.8.13	RSTD Inter-frequency Measurements	2120

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2120

A.8.13.1.1	Test Purpose and Environment	2120

A.8.13.1.2	Test Requirements	2123

A.8.13.1.2A	Test Requirements for UE Category 1bis	2124

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2124

A.8.13.2.1	Test Purpose and Environment	2124

A.8.13.2.2	Test Requirements	2128

A.8.13.2.2A	Test Requirements for UE Category 1bis	2129

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2129

A.8.13.3.1	Test Purpose and Environment	2129

A.8.13.3.2	Test Requirements	2133

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2134

A.8.13.4.1	Test Purpose and Environment	2134

A.8.13.4.2	Test Requirements	2137

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2138

A.8.13.5.1	Test Purpose and Environment	2138

A.8.13.5.2	Test Requirements	2142

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2143

A.8.13.6.1	Test Purpose and Environment	2143

A.8.13.6.2	Test Requirements	2146

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2147

A.8.13.7.1	Test Purpose and Environment	2147

A.8.13.7.2	Test Requirements	2150

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2151

A.8.13.8.1	Test Purpose and Environment	2151

A.8.13.8.2	Test Requirements	2155

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2156

A.8.13.9.1	Test Purpose and Environment	2156

A.8.13.9.2	Test Requirements	2160

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2161

A.8.13.10.1	Test Purpose and Environment	2161

A.8.13.10.2	Test Requirements	2165

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2166

A.8.13.11.1	Test Purpose and Environment	2166

A.8.13.11.2	Test Requirements	2170

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2171

A.8.13.12.1	Test Purpose and Environment	2171

A.8.13.12.2	Test Requirements	2175

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2176

A.8.13.13.1	Test Purpose and Environment	2176

A.8.13.13.2	Test Requirements	2180

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2181

A.8.13.14.1	Test Purpose and Environment	2181

A.8.13.14.2	Test Requirements	2185

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2186

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2186

A.8.14.1.1	Test Purpose and Environment	2186

A.8.14.1.2	Test Requirements	2188

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2188

A.8.14.2.1	Test Purpose and Environment	2188

A.8.14.2.2	Test Requirements	2191

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2191

A.8.14.3.1	Test Purpose and Environment	2191

A.8.14.3.2	Test Requirements	2193

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2194

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2194

A.8.15.1.1	Test Purpose and Environment	2194

A.8.15.1.2	Test Requirements	2195

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2195

A.8.15.2.1	Test Purpose and Environment	2195

A.8.15.2.2	Test Requirements	2198

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2198

A.8.15.3.1	Test Purpose and Environment	2198

A.8.15.3.2	Test Requirements	2200

A.8.16	E-UTRAN Carrier Aggregation Measurements	2201

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2201

A.8.16.1.1	Test Purpose and Environment	2201

A.8.16.1.2	Test Requirements	2203

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2203

A.8.16.2.1	Test Purpose and Environment	2203

A.8.16.2.2	Test Requirements	2205

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2205

A.8.16.3.1	Test Purpose and Environment	2205

A.8.16.3.2	Test Requirements	2207

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2208

A.8.16.3A.1	Test Purpose and Environment	2208

A.8.16.3A.2	Test Requirements	2209

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2210

A.8.16.4.1	Test Purpose and Environment	2210

A.8.16.4.2	Test Requirements	2211

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2212

A.8.16.4A.1	Test Purpose and Environment	2212

A.8.16.4A.2	Test Requirements	2214

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2215

A.8.16.5.1	Test Purpose and Environment	2215

A.8.16.5.2	Test Requirements	2215

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2215

A.8.16.6.1	Test Purpose and Environment	2215

A.8.16.6.2	Test Requirements	2216

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2216

A.8.16.7.1	 Test Purpose and Environment	2216

A.8.16.7.2	 Test Requirements	2217

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2217

A.8.16.8.1	 Test Purpose and Environment	2217

A.8.16.8.2	 Test Requirements	2218

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2218

A.8.16.9.1	Test Purpose and Environment	2218

A.8.16.9.2	Test Requirements	2219

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2219

A.8.16.10.1	Test Purpose and Environment	2219

A.8.16.10.2	Test Requirements	2220

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2220

A.8.16.11.1	 Test Purpose and Environment	2220

A.8.16.11.2	 Test Requirements	2221

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2221

A.8.16.12.1	 Test Purpose and Environment	2221

A.8.16.12.2	 Test Requirements	2221

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2221

A.8.16.13.1	Test Purpose and Environment	2221

A.8.16.13.2	Test Requirements	2222

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2222

A.8.16.14.1	Test Purpose and Environment	2222

A.8.16.14.2	Test Requirements	2223

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2223

A.8.16.15.1	 Test Purpose and Environment	2223

A.8.16.7.2	 Test Requirements	2223

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2223

A.8.16.16.1	 Test Purpose and Environment	2223

A.8.16.16.2	 Test Requirements	2224

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2224

A.8.16.17.1	Test Purpose and Environment	2224

A.8.16.17.2	Test Requirements	2226

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2227

A.8.16.17A.1	Test Purpose and Environment	2227

A.8.16.17A.2	Test Requirements	2227

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2227

A.8.16.17B.1	Test Purpose and Environment	2227

A.8.16.17B.2	Test Requirements	2228

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2228

A.8.16.17C.1	Test Purpose and Environment	2228

A.8.16.17C.2	Test Requirements	2228

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2228

A.8.16.18.1	Test Purpose and Environment	2228

A.8.16.18.2	Test Requirements	2230

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2231

A.8.16.18A.1	Test Purpose and Environment	2231

A.8.16.18A.2	Test Requirements	2231

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2231

A.8.16.18B.1	Test Purpose and Environment	2231

A.8.16.18B.2	Test Requirements	2232

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2232

A.8.16.18C.1	Test Purpose and Environment	2232

A.8.16.18C.2	Test Requirements	2232

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2232

A.8.16.18D.1	Test Purpose and Environment	2232

A.8.16.18D.2	Test Requirements	2233

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2233

A.8.16.19.1	Test Purpose and Environment	2233

A.8.16.19.2	Test Requirements	2235

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2235

A.8.16.19A.1	Test Purpose and Environment	2235

A.8.16.19A.2	Test Requirements	2236

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2236

A.8.16.19B.1	Test Purpose and Environment	2236

A.8.16.19B.2	Test Requirements	2236

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2236

A.8.16.19C.1	Test Purpose and Environment	2236

A.8.16.19C.2	Test Requirements	2237

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2237

A.8.16.20.1	Test Purpose and Environment	2237

A.8.16.20.2	Test Requirements	2239

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2240

A.8.16.20A.1	Test Purpose and Environment	2240

A.8.16.20A.2	Test Requirements	2240

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2240

A.8.16.20B.1	Test Purpose and Environment	2240

A.8.16.20B.2	Test Requirements	2241

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2241

A.8.16.20C.1	Test Purpose and Environment	2241

A.8.16.20C.2	Test Requirements	2241

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2241

A.8.16.20D.1	Test Purpose and Environment	2241

A.8.16.20D.2	Test Requirements	2242

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2242

A.8.16.21.1	Test Purpose and Environment	2242

A.8.16.21.2	Test Requirements	2244

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2244

A.8.16.22.1	Test Purpose and Environment	2244

A.8.16.22.2	Test Requirements	2246

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2246

A.8.16.23.1	Test Purpose and Environment	2246

A.8.16.23.2	Test Requirements	2249

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2249

A.8.16.24.1	Test Purpose and Environment	2249

A.8.16.24.2	Test Requirements	2252

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2252

A.8.16.25.1	Test Purpose and Environment	2252

A.8.16.25.2	Test Requirements	2254

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2254

A.8.16.26.1	Test Purpose and Environment	2254

A.8.16.26.2	Test Requirements	2257

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2257

A.8.16.27.1	Test Purpose and Environment	2257

A.8.16.27.2	Test Requirements	2261

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2261

A.8.16.28.1	Test Purpose and Environment	2261

A.8.16.28.2	Test Requirements	2265

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2265

A.8.16.29.1	Test Purpose and Environment	2265

A.8.16.29.2	Test Requirements	2269

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2269

A.8.16.30.1	Test Purpose and Environment	2269

A.8.16.30.2	Test Requirements	2273

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2273

A.8.16.31.1	Test Purpose and Environment	2273

A.8.16.31.2	Test Requirements	2277

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2277

A.8.16.32.1	Test Purpose and Environment	2277

A.8.16.32.2	Test Requirements	2281

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2281

A.8.16.33.1	Test Purpose and Environment	2281

A.8.16.33.2	Test Requirements	2285

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2285

A.8.16.34.1	Test Purpose and Environment	2285

A.8.16.34.2	Test Requirements	2289

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2289

A.8.16.35.1	Test Purpose and Environment	2289

A.8.16.35.2	Test Requirements	2291

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2292

A.8.16.36.1	Test Purpose and Environment	2292

A.8.16.36.2	Test Requirements	2294

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2295

A.8.16.37.1	Test Purpose and Environment	2295

A.8.16.37.2	Test Requirements	2297

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2298

A.8.16.38.1	Test Purpose and Environment	2298

A.8.16.38.2	Test Requirements	2300

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2301

A.8.16.39.1	Test Purpose and Environment	2301

A.8.16.39.2	Test Requirements	2304

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2304

A.8.16.40.1	Test Purpose and Environment	2304

A.8.16.40.2	Test Requirements	2307

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2307

A.8.16.41.1	Test Purpose and Environment	2307

A.8.16.41.2	Test Requirements	2309

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2310

A.8.16.42.1	Test Purpose and Environment	2310

A.8.16.42.2	Test Requirements	2312

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2313

A.8.16.43.1	Test Purpose and Environment	2313

A.8.16.43.2	Test Requirements	2316

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2316

A.8.16.44.1	Test Purpose and Environment	2316

A.8.16.44.2	Test Requirements	2319

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2319

A.8.16.45.1	Test Purpose and Environment	2319

A.8.16.45.2	Test Requirements	2322

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2322

A.8.16.46.1	Test Purpose and Environment	2322

A.8.16.46.2	Test Requirements	2324

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2325

A.8.16.47.1	Test Purpose and Environment	2325

A.8.16.47.2	Test Requirements	2327

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2328

A.8.16.48.1	Test Purpose and Environment	2328

A.8.16.48.2	Test Requirements	2330

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2331

A.8.16.49.1	Test Purpose and Environment	2331

A.8.16.49.2	Test Requirements	2334

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2334

A.8.16.50.1	Test Purpose and Environment	2334

A.8.16.50.2	Test Requirements	2336

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2337

A.8.16.51.1	Test Purpose and Environment	2337

A.8.16.51.2	Test Requirements	2341

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2341

A.8.16.52.1	Test Purpose and Environment	2341

A.8.16.52.2	Test Requirements	2345

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2345

A.8.16.53.1	Test Purpose and Environment	2345

A.8.16.53.2	Test Requirements	2349

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2349

A.8.16.54.1	Test Purpose and Environment	2349

A.8.16.54.2	Test Requirements	2353

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2353

A.8.16.55.1	Test Purpose and Environment	2353

A.8.16.55.2	Test Requirements	2358

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2358

A.8.16.56.1	Test Purpose and Environment	2358

A.8.16.56.2	Test Requirements	2363

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2363

A.8.16.57.1	Test Purpose and Environment	2363

A.8.16.57.2	Test Requirements	2365

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2366

A.8.16.58.1	Test Purpose and Environment	2366

A.8.16.58.2	Test Requirements	2368

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2369

A.8.16.59.1	Test Purpose and Environment	2369

A.8.16.59.2	Test Requirements	2372

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2373

A.8.16.60.1	Test Purpose and Environment	2373

A.8.16.60.2	Test Requirements	2376

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2377

A.8.16.61.1	Test Purpose and Environment	2377

A.8.16.61.2	Test Requirements	2380

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2380

A.8.16.62.1	Test Purpose and Environment	2380

A.8.16.62.2	Test Requirements	2382

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2383

A.8.16.63.1	Test Purpose and Environment	2383

A.8.16.63.2	Test Requirements	2386

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2387

A.8.16.64.1	Test Purpose and Environment	2387

A.8.16.64.2	Test Requirements	2391

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2391

A.8.16.65.1	Test Purpose and Environment	2391

A.8.16.65.2	Test Requirements	2396

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2396

A.8.16.66.1	Test Purpose and Environment	2396

A.8.16.66.2	Test Requirements	2401

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2401

A.8.16.67.1	Test Purpose and Environment	2401

A.8.16.67.2	Test Requirements	2406

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2406

A.8.16.68.1	Test Purpose and Environment	2406

A.8.16.68.2	Test Requirements	2411

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2411

A.8.16.69.1	Test Purpose and Environment	2411

A.8.16.69.2	Test Requirements	2414

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2414

A.8.16.70.1	Test Purpose and Environment	2414

A.8.16.70.2	Test Requirements	2417

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2417

A.8.16.71.1	Test Purpose and Environment	2417

A.8.16.71.2	Test Requirements	2421

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2421

A.8.16.72.1	Test Purpose and Environment	2421

A.8.16.72.2	Test Requirements	2425

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2425

A.8.16.73.1	Test Purpose and Environment	2425

A.8.16.73.2	Test Requirements	2430

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2430

A.8.16.74.1	Test Purpose and Environment	2430

A.8.16.74.2	Test Requirements	2435

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2435

A.8.16.75.1	Test Purpose and Environment	2435

A.8.16.75.2	Test Requirements	2440

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2440

A.8.16.76.1	Test Purpose and Environment	2440

A.8.16.76.2	Test Requirements	2445

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2445

A.8.16.77.1	Test Purpose and Environment	2445

A.8.16.77.2	Test Requirements	2448

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2448

A.8.16.78.1	Test Purpose and Environment	2448

A.8.16.78.2	Test Requirements	2451

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2452

A.8.16.79.1	Test Purpose and Environment	2452

A.8.16.79.2	Test Requirements	2457

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2457

A.8.16.80.1	Test Purpose and Environment	2457

A.8.16.80.2	Test Requirements	2462

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2462

A.8.16.81.1	Test Purpose and Environment	2462

A.8.16.81.2	Test Requirements	2467

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2467

A.8.16.82.1	Test Purpose and Environment	2467

A.8.16.82.2	Test Requirements	2472

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2472

A.8.16.83.1	Test Purpose and Environment	2472

A.8.16.83.2	Test Requirements	2477

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2477

A.8.16.84.1	Test Purpose and Environment	2477

A.8.16.84.2	Test Requirements	2483

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2483

A.8.16.85.1	Test Purpose and Environment	2483

A.8.16.85.2	Test Requirements	2486

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2486

A.8.16.86.1	Test Purpose and Environment	2486

A.8.16.86.2	Test Requirements	2489

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2489

A.8.16.87.1	Test Purpose and Environment	2489

A.8.16.87.2	Test Requirements	2494

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2494

A.8.16.88.1	Test Purpose and Environment	2494

A.8.16.88.2	Test Requirements	2502

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2502

A.8.16.89.1	Test Purpose and Environment	2502

A.8.16.89.2	Test Requirements	2507

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2507

A.8.16.90.1	Test Purpose and Environment	2507

A.8.16.90.2	Test Requirements	2512

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2512

A.8.16.91.1	Test Purpose and Environment	2512

A.8.16.91.2	Test Requirements	2518

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2518

A.8.16.92.1	Test Purpose and Environment	2518

A.8.16.92.2	Test Requirements	2526

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2526

A.8.16.93.1	Test Purpose and Environment	2526

A.8.16.93.2	Test Requirements	2531

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2531

A.8.16.94.1	Test Purpose and Environment	2531

A.8.16.94.2	Test Requirements	2536

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2536

A.8.16.95.1	Test Purpose and Environment	2536

A.8.16.95.2	Test Requirements	2544

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2544

A.8.16.96.1	Test Purpose and Environment	2544

A.8.16.96.2	Test Requirements	2551

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2551

A.8.16.97.1	Test Purpose and Environment	2551

A.8.16.97.2	Test Requirements	2556

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2556

A.8.16.98.1	Test Purpose and Environment	2556

A.8.16.98.2	Test Requirements	2562

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2562

A.8.16.99.1	Test Purpose and Environment	2562

A.8.16.99.2	Test Requirements	2569

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2569

A.8.16.100.1	Test Purpose and Environment	2569

A.8.16.100.2	Test Requirements	2577

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2577

A.8.16.101.1	Test Purpose and Environment	2577

A.8.16.101.2	Test Requirements	2584

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2584

A.8.16.102.1	Test Purpose and Environment	2584

A.8.16.102.2	Test Requirements	2592

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2592

A.8.16.103.1	Test Purpose and Environment	2592

A.8.16.103.2	Test Requirements	2596

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2597

A.8.16.104.1	Test Purpose and Environment	2597

A.8.16.104.2	Test Requirements	2600

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2601

A.8.16.105.1	Test Purpose and Environment	2601

A.8.16.105.2	Test Requirements	2605

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2606

A.8.16.106.1	Test Purpose and Environment	2606

A.8.16.106.2	Test Requirements	2609

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2609

A.8.16.107.1	Test Purpose and Environment	2609

A.8.16.107.2	Test Requirements	2611

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2612

A.8.16.108.1	Test Purpose and Environment	2612

A.8.16.108.2	Test Requirements	2614

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2615

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2615

A.8.17.1.1	Test Purpose and Environment	2615

A.8.17.1.2	Test Requirements	2620

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2620

A.8.17.2.1	Test Purpose and Environment	2620

A.8.17.2.2	Test Requirements	2626

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2626

A.8.17.3.1	Test Purpose and Environment	2626

A.8.17.3.2	Test Requirements	2627

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2627

A.8.17.4.1	Test Purpose and Environment	2627

A.8.17.4.2	Test Requirements	2628

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2628

A.8.17.5.1	Test Purpose and Environment	2628

A.8.17.5.2	Test Requirements	2629

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2629

A.8.17.6.1	Test Purpose and Environment	2629

A.8.17.6.2	Test Requirements	2630

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2631

A.8.17.7.1	Test Purpose and Environment	2631

A.8.17.7.2	Test Requirements	2631

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2632

A.8.17.8.1	Test Purpose and Environment	2632

A.8.17.8.2	Test Requirements	2632

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2633

A.8.17.9.1	Test Purpose and Environment	2633

A.8.17.9.2	Test Requirements	2634

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2634

A.8.17.10.1	Test Purpose and Environment	2634

A.8.17.10.2	Test Requirements	2639

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2640

A.8.17.11.1	Test Purpose and Environment	2640

A.8.17.11.2	Test Requirements	2646

A.8.18	E-UTRAN TDD – HRPD Measurements	2647

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2647

A.8.18.1.1	Test Purpose and Environment	2647

A.8.18.1.2	Test Requirements	2649

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2649

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2649

A.8.19.1.1	Test Purpose and Environment	2649

A.8.19.1.2	Test Requirements	2650

A.8.20	Inter-frequency/RAT Measurements in CA mode	2651

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2651

A.8.20.1.1	Test Purpose and Environment	2651

A.8.20.1.2	Test Requirements	2652

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2652

A.8.20.2.1	Test Purpose and Environment	2653

A.8.20.2.2	Test Requirements	2654

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2655

A.8.20.2A.1	Test Purpose and Environment	2655

A.8.20.2A.2	Test Requirements	2655

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2655

A.8.20.2B.1	Test Purpose and Environment	2655

A.8.20.2B.2	Test Requirements	2658

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2658

A.8.20.3.1	Test Purpose and Environment	2658

A.8.20.3.2	Test Requirements	2660

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2660

A.8.20.4.1	Test Purpose and Environment	2660

A.8.20.4.1.1	1.28 Mcps TDD option	2660

A.8.20.4.2	Test Requirements	2662

A.8.20.4.2.1	1.28 Mcps TDD option	2662

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2662

A.8.20.4A.1	Test Purpose and Environment	2662

A.8.20.4A.1.1	1.28 Mcps TDD option	2662

A.8.20.4A.2	Test Requirements	2662

A.8.20.4A.2.1	1.28 Mcps TDD option	2662

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2663

A.8.20.4B.1	Test Purpose and Environment	2663

A.8.20.4B.1.1	1.28 Mcps TDD option	2663

A.8.20.4B.2	Test Requirements	2665

A.8.20.4B.2.1	1.28 Mcps TDD option	2665

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2665

A.8.21.1	Test Purpose and Environment	2665

A.8.21.2 Test Requirements	2669

A.8.22	E-UTRAN Discovery Signal Measurements	2669

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2669

A.8.22.1.1	Test Purpose and Environment	2669

A.8.22.1.2	Test Requirements	2672

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2672

A.8.22.2.1	Test Purpose and Environment	2672

A.8.22.2.2	Test Requirements	2675

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2675

A.8.22.3.1	Test Purpose and Environment	2675

A.8.22.3.2	Test Requirements	2678

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2678

A.8.22.4.1	Test Purpose and Environment	2678

A.8.22.4.2	Test Requirements	2681

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2681

A.8.22.5.1	Test Purpose and Environment	2681

A.8.22.5.2	Test Requirements	2684

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2684

A.8.22.6.1	Test Purpose and Environment	2684

A.8.22.6.2	Test Requirements	2687

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2687

A.8.22.7.1	Test Purpose and Environment	2687

A.8.22.7.2	Test Requirements	2690

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2690

A.8.22.8.1	Test Purpose and Environment	2690

A.8.22.8.2	Test Requirements	2693

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2693

A.8.22.9.1	Test Purpose and Environment	2693

A.8.22.9.2	Test Requirements	2695

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2696

A.8.22.10.1	Test Purpose and Environment	2696

A.8.22.10.2	Test Requirements	2698

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2699

A.8.22.11.1	Test Purpose and Environment	2699

A.8.22.11.2	Test Requirements	2701

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2701

A.8.22.12.1	Test Purpose and Environment	2701

A.8.22.12.2	Test Requirements	2704

A.8.23	E-UTRAN Dual Connectivity Measurements	2704

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2704

A.8.23.1.1	Test Purpose and Environment	2704

A.8.23.1.2	Test Requirements	2707

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2707

A.8.23.2.1	Test Purpose and Environment	2707

A.8.23.2.2	Test Requirements	2710

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2710

A.8.23.3.1	Test Purpose and Environment	2710

A.8.23.3.2	Test Requirements	2713

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2713

A.8.23.4.1	Test Purpose and Environment	2713

A.8.23.4.2	Test Requirements	2716

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2716

A.8.23.5.1	Test Purpose and Environment	2716

A.8.23.5.2	Test Requirements	2719

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2719

A.8.23.6.1	Test Purpose and Environment	2719

A.8.23.6.2	Test Requirements	2722

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2722

A.8.23.7.1	Test Purpose and Environment	2722

A.8.23.7.2	Test Requirements	2724

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2725

A.8.23.8.1	Test Purpose and Environment	2725

A.8.23.8.2	Test Requirements	2727

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2728

A.8.23.9.1	Test Purpose and Environment	2728

A.8.23.9.2	Test Requirements	2731

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2731

A.8.23.10.1	Test Purpose and Environment	2731

A.8.23.10.2	Test Requirements	2734

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2734

A.8.23.11.1	Test Purpose and Environment	2734

A.8.23.11.2	Test Requirements	2737

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2737

A.8.23.12.1	Test Purpose and Environment	2737

A.8.23.12.2	Test Requirements	2740

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2740

A.8.23.13.1	Test Purpose and Environment	2740

A.8.23.13.2	Test Requirements	2743

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2743

A.8.23.14.1	Test Purpose and Environment	2743

A.8.23.14.2	Test Requirements	2746

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2746

A.8.23.15.1	Test Purpose and Environment	2746

A.8.23.15.2	Test Requirements	2749

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2749

A.8.23.16.1	Test Purpose and Environment	2749

A.8.23.16.2	Test Requirements	2750

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2751

A.8.23.17.1	Test Purpose and Environment	2751

A.8.23.17.2	Test Requirements	2753

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2753

A.8.23.18.1	Test Purpose and Environment	2753

A.8.23.18.2	Test Requirements	2754

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2755

A.8.23.19.1	Test Purpose and Environment	2755

A.8.23.19.2	Test Requirements	2756

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2757

A.8.23.20.1	Test Purpose and Environment	2757

A.8.23.20.2	Test Requirements	2758

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2759

A.8.23.21.1	Test Purpose and Environment	2759

A.8.23.21.2	Test Requirements	2760

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2761

A.8.23.22.1	Test Purpose and Environment	2761

A.8.23.22.2	Test Requirements	2762

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2763

A.8.23.23.1	Test Purpose and Environment	2763

A.8.23.23.2	Test Requirements	2764

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2765

A.8.23.24.1	Test Purpose and Environment	2765

A.8.23.24.2	Test Requirements	2767

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2768

A.8.23.25.1	Test Purpose and Environment	2768

A.8.23.25.2	Test Requirements	2770

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2771

A.8.23.26.1	Test Purpose and Environment	2771

A.8.23.26.2	Test Requirements	2773

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2774

A.8.23.27.1	Test Purpose and Environment	2774

A.8.23.27.2	Test Requirements	2777

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2777

A.8.23.28.1	Test Purpose and Environment	2777

A.8.23.28.2	Test Requirements	2780

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2780

A.8.23.29.1	Test Purpose and Environment	2780

A.8.23.29.2	Test Requirements	2783

A.8.24	Proximity-based Services	2783

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2783

A.8.24.1.1	Test Purpose and Environment	2783

A.8.24.1.2	Test Requirements	2784

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2785

A.8.24.2.1	Test Purpose and Environment	2785

A.8.24.2.2	Test Requirements	2786

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2786

A.8.24.3.1	Test Purpose and Environment	2787

A.8.24.3.2	Test Requirements	2788

A.8.25	E-UTRAN-WLAN Measurements	2789

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2789

A.8.25.1.1	Test Purpose and Environment	2789

A.8.25.1.2	Test Requirements	2791

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2791

A.8.25.2.1	Test Purpose and Environment	2791

A.8.25.2.2	Test Requirements	2793

A.8.26 Frame Structure 3 (FS3)	2793

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2793

A.8.26.1.1	Test Purpose and Environment	2793

A.8.26.1.2	Test Requirements	2795

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2796

A.8.26.2.1	Test Purpose and Environment	2796

A.8.26.2.2	Test Requirements	2798

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2799

A.8.26.3.1	Test Purpose and Environment	2799

A.8.26.3.2	Test Requirements	2802

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2802

A.8.26.3A.1	Test Purpose and Environment	2802

A.8.26.3A.2	Test Requirements	2805

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2805

A.8.26.4.1	Test Purpose and Environment	2805

A.8.26.4.2	Test Requirements	2808

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2808

A.8.26.4A.1	Test Purpose and Environment	2808

A.8.26.4A.2	Test Requirements	2811

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2811

A.8.26.5.1	Test Purpose and Environment	2811

A.8.26.5.2	Test Requirements	2814

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2814

A.8.26.5A.1	Test Purpose and Environment	2814

A.8.26.5A.2	Test Requirements	2817

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2817

A.8.26.6.1	Test Purpose and Environment	2817

A.8.26.6.2	Test Requirements	2820

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2820

A.8.26.6A.1	Test Purpose and Environment	2820

A.8.26.6A.2	Test Requirements	2823

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2823

A.8.26.7.1	Test Purpose and Environment	2823

A.8.26.7.2	Test Requirements	2826

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2826

A.8.26.8.1	Test Purpose and Environment	2826

A.8.26.8.2	Test Requirements	2829

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2829

A.8.26.9.1	Test Purpose and Environment	2829

A.8.26.9.2	Test Requirements	2832

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2832

A.8.26.10.1	Test Purpose and Environment	2832

A.8.26.10.2	Test Requirements	2834

A.9	Measurement Performance Requirements	2835

A.9.1	RSRP	2835

A.9.1.1	FDD Intra frequency case	2835

A.9.1.1.1	Test Purpose and Environment	2835

A.9.1.1.2	Test parameters	2835

A.9.1.1.3	Test Requirements	2838

A.9.1.2	TDD Intra frequency case	2838

A.9.1.2.1	Test Purpose and Environment	2838

A.9.1.2.2	Test parameters	2838

A.9.1.2.3	Test Requirements	2839

A.9.1.3	FDD—FDD Inter frequency case	2840

A.9.1.3.1	Test Purpose and Environment	2840

A.9.1.3.2	Test parameters	2840

A.9.1.3.3	Test Requirements	2843

A.9.1.4	TDD—TDD Inter frequency case	2843

A.9.1.4.1	Test Purpose and Environment	2843

A.9.1.4.2	Test parameters	2843

A.9.1.4.3	Test Requirements	2845

A.9.1.5	FDD—TDD Inter frequency case	2846

A.9.1.5.1	Test Purpose and Environment	2846

A.9.1.5.2	Test parameters	2846

A.9.1.5.3	Test Requirements	2847

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2848

A.9.1.6.1	Test Purpose and Environment	2848

A.9.1.6.2	Test parameters	2848

A.9.1.6.3	Test Requirements	2851

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2851

A.9.1.7.1	Test Purpose and Environment	2851

A.9.1.7.2	Test parameters	2851

A.9.1.7.3	Test Requirements	2854

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2854

A.9.1.8.1	Test Purpose and Environment	2854

A.9.1.8.2	Test parameters	2854

A.9.1.8.3	Test Requirements	2858

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2858

A.9.1.9.1	Test Purpose and Environment	2858

A.9.1.9.2	Test parameters	2858

A.9.1.9.3	Test Requirements	2860

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2861

A.9.1.10.1	Test Purpose and Environment	2861

A.9.1.10.2	Test parameters	2861

A.9.1.10.3	Test Requirements	2864

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2864

A.9.1.11.1	Test Purpose and Environment	2864

A.9.1.11.2	Test parameters	2864

A.9.1.11.3	Test Requirements	2868

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2868

A.9.1.12.1	Test Purpose and Environment	2868

A.9.1.12.2	Test parameters	2868

A.9.1.12.3	Test Requirements	2869

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2869

A.9.1.13.1	Test Purpose and Environment	2869

A.9.1.13.2	Test parameters	2869

A.9.1.13.3	Test Requirements	2870

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2870

A.9.1.14.1	Test Purpose and Environment	2870

A.9.1.14.2	Test parameters	2870

A.9.1.14.3	Test Requirements	2874

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2874

A.9.1.15.1	Test Purpose and Environment	2874

A.9.1.15.2	Test parameters	2874

A.9.1.15.3	Test Requirements	2876

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2877

A.9.1.16.1	Test Purpose and Environment	2877

A.9.1.16.2	Test parameters	2877

A.9.1.16.3	Test Requirements	2878

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2878

A.9.1.17.1	Test Purpose and Environment	2878

A.9.1.17.2	Test parameters	2879

A.9.1.17.3	Test Requirements	2880

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2880

A.9.1.18.1	Test Purpose and Environment	2880

A.9.1.18.2	Test parameters	2880

A.9.1.18.3	Test Requirements	2881

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2882

A.9.1.19.1	Test Purpose and Environment	2882

A.9.1.19.2	Test parameters	2882

A.9.1.19.3	Test Requirements	2882

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2882

A.9.1.20.1	Test Purpose and Environment	2882

A.9.1.20.2	Test parameters	2882

A.9.1.20.3	Test Requirements	2884

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2884

A.9.1.21.1	Test Purpose and Environment	2884

A.9.1.21.2	Test parameters	2884

A.9.1.21.3	Test Requirements	2884

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2884

A.9.1.22.1	Test Purpose and Environment	2884

A.9.1.22.2	Test parameters	2885

A.9.1.22.3	Test Requirements	2889

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2889

A.9.1.23.1	Test Purpose and Environment	2889

A.9.1.23.2	Test parameters	2889

A.9.1.23.3	Test Requirements	2893

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2893

A.9.1.24.1	Test Purpose and Environment	2893

A.9.1.24.2	Test parameters	2893

A.9.1.24.3	Test Requirements	2894

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2894

A.9.1.25.1	Test Purpose and Environment	2894

A.9.1.25.2	Test parameters	2894

A.9.1.25.3	Test Requirements	2897

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2897

A.9.1.26.1	Test Purpose and Environment	2897

A.9.1.26.2	Test parameters	2897

A.9.1.26.3	Test Requirements	2900

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2900

A.9.1.27.1	Test Purpose and Environment	2900

A.9.1.27.2	Test parameters	2900

A.9.1.27.3	Test Requirements	2903

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2903

A.9.1.28.1	Test Purpose and Environment	2903

A.9.1.28.2	Test parameters	2903

A.9.1.28.3	Test Requirements	2906

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2906

A.9.1.29.1	Test Purpose and Environment	2906

A.9.1.29.2	Test parameters	2906

A.9.1.29.3	Test Requirements	2909

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2909

A.9.1.30.1	Test Purpose and Environment	2909

A.9.1.30.2	Test parameters	2910

A.9.1.30.3	Test Requirements	2913

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2913

A.9.1.31.1	Test Purpose and Environment	2913

A.9.1.31.2	Test parameters	2913

A.9.1.31.3	Test Requirements	2916

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2917

A.9.1.32.1	Test Purpose and Environment	2917

A.9.1.32.2	Test parameters	2917

A.9.1.32.3	Test Requirements	2920

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2920

A.9.1.33.1	Test Purpose and Environment	2920

A.9.1.33.2	Test parameters	2920

A.9.1.33.3	Test Requirements	2923

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2923

A.9.1.34.1	Test Purpose and Environment	2923

A.9.1.34.2	Test parameters	2923

A.9.1.34.3	Test Requirements	2926

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2926

A.9.1.35.1	Test Purpose and Environment	2926

A.9.1.35.2	Test parameters	2926

A.9.1.35.3	Test Requirements	2929

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2930

A.9.1.36.1	Test Purpose and Environment	2930

A.9.1.36.2	Test parameters	2930

A.9.1.36.3	Test Requirements	2933

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2933

A.9.1.37.1	Test Purpose and Environment	2933

A.9.1.37.2	Test parameters	2933

A.9.1.37.3	Test Requirements	2936

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2937

A.9.1.38.1	Test Purpose and Environment	2937

A.9.1.38.2	Test parameters	2937

A.9.1.38.3	Test Requirements	2940

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2941

A.9.1.39.1	Test Purpose and Environment	2941

A.9.1.39.2	Test parameters	2941

A.9.1.39.3	Test Requirements	2947

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2947

A.9.1.40.1	Test Purpose and Environment	2947

A.9.1.40.2	Test parameters	2948

A.9.1.40.3	Test Requirements	2954

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2954

A.9.1.41.1	Test Purpose and Environment	2954

A.9.1.41.2	Test parameters	2954

A.9.1.41.3	Test Requirements	2957

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2957

A.9.1.42.1	Test Purpose and Environment	2957

A.9.1.42.2	Test parameters	2957

A.9.1.42.3	Test Requirements	2960

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2960

A.9.1.43.1	Test Purpose and Environment	2960

A.9.1.43.2	Test parameters	2960

A.9.1.43.3	Test Requirements	2961

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2962

A.9.1.44.1	Test Purpose and Environment	2962

A.9.1.44.2	Test parameters	2962

A.9.1.44.3	Test Requirements	2967

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2967

A.9.1.45.1	Test Purpose and Environment	2967

A.9.1.45.2	Test parameters	2967

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2482

A.9.1.54.1	Test Purpose and Environment	2482

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.2	RSRQ	2582

A.9.2.1	FDD Intra frequency case	2582

A.9.2.1.1	Test Purpose and Environment	2582

A.9.2.1.2	Test parameters	2582

A.9.2.1.3	Test Requirements	2585

A.9.2.2	TDD Intra frequency case	2585

A.9.2.2.1	Test Purpose and Environment	2585

A.9.2.2.2	Test parameters	2585

A.9.2.2.3	Test Requirements	2587

A.9.2.3	FDD—FDD Inter frequency case	2587

A.9.2.3.1	Test Purpose and Environment	2587

A.9.2.3.2	Test parameters	2587

A.9.2.3.3	Test Requirements	2590

A.9.2.4	TDD—TDD Inter frequency case	2590

A.9.2.4.1	Test Purpose and Environment	2590

A.9.2.4.2	Test parameters	2590

A.9.2.4.3	Test Requirements	2592

A.9.2.4A	FDD—TDD Inter frequency case	2593

A.9.2.4A.1	Test Purpose and Environment	2593

A.9.2.4A.2	Test parameters	2593

A.9.2.4A.3	Test Requirements	2595

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2595

A.9.2.5.1	Test Purpose and Environment	2595

A.9.2.5.2	Test parameters	2596

A.9.2.5.3	Test Requirements	2599

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2599

A.9.2.6.1	Test Purpose and Environment	2599

A.9.2.6.2	Test parameters	2600

A.9.2.6.3	Test Requirements	2603

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2603

A.9.2.7.1	Test Purpose and Environment	2603

A.9.2.7.2	Test parameters	2603

A.9.2.7.3	Test Requirements	2607

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2607

A.9.2.8.1	Test Purpose and Environment	2607

A.9.2.8.2	Test parameters	2607

A.9.2.8.3	Test Requirements	2610

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2610

A.9.2.9.1	Test Purpose and Environment	2610

A.9.2.9.2	Test parameters	2610

A.9.2.9.3	Test Requirements	2614

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2614

A.9.2.10.1	Test Purpose and Environment	2614

A.9.2.10.2	Test parameters	2614

A.9.2.10.3	Test Requirements	2618

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2618

A.9.2.11.1	Test Purpose and Environment	2618

A.9.2.11.2	Test parameters	2618

A.9.2.11.3	Test Requirements	2619

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2619

A.9.2.12.1	Test Purpose and Environment	2619

A.9.2.12.2	Test parameters	2620

A.9.2.12.3	Test Requirements	2620

A.9.2.13	Void	2621

A.9.2.13.1	Void	2621

A.9.2.13.2	Void	2621

A.9.2.13.3	Void	2621

A.9.2.14	Void	2621

A.9.2.14.1	Void	2621

A.9.2.14.2	Void	2621

A.9.2.14.3	Void	2621

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2621

A.9.2.15.1	Test Purpose and Environment	2621

A.9.2.15.2	Test parameters	2621

A.9.2.15.3	Test Requirements	2625

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2625

A.9.2.16.1	Test Purpose and Environment	2625

A.9.2.16.2	Test parameters	2625

A.9.2.16.3	Test Requirements	2630

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2630

A.9.2.17.1	Test Purpose and Environment	2630

A.9.2.17.2	Test parameters	2630

A.9.2.17.3	Test Requirements	2632

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2632

A.9.2.18.1	Test Purpose and Environment	2632

A.9.2.18.2	Test parameters	2632

A.9.2.18.3	Test Requirements	2635

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2635

A.9.2.19.1	Test Purpose and Environment	2635

A.9.2.19.2	Test parameters	2635

A.9.2.19.3	Test Requirements	2637

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2637

A.9.2.20.1	Test Purpose and Environment	2637

A.9.2.20.2	Test parameters	2637

A.9.2.20.3	Test Requirements	2640

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2640

A.9.2.21.1	Test Purpose and Environment	2640

A.9.2.21.2	Test parameters	2640

A.9.2.21.3	Test Requirements	2643

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2643

A.9.2.22.1	Test Purpose and Environment	2644

A.9.2.22.2	Test parameters	2644

A.9.2.22.3	Test Requirements	2644

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2644

A.9.2.23.1	Test Purpose and Environment	2644

A.9.2.23.2	Test parameters	2644

A.9.2.23.3	Test Requirements	2647

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2647

A.9.2.24.1	Test Purpose and Environment	2647

A.9.2.24.2	Test parameters	2647

A.9.2.24.3	Test Requirements	2648

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2648

A.9.2.25.1	Test Purpose and Environment	2648

A.9.2.25.2	Test parameters	2648

A.9.2.25.3	Test Requirements	2653

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2653

A.9.2.26.1	Test Purpose and Environment	2653

A.9.2.26.2	Test parameters	2653

A.9.2.26.3	Test Requirements	2657

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2657

A.9.2.27.1	Test Purpose and Environment	2657

A.9.2.27.2	Test parameters	2657

A.9.2.27.3	Test Requirements	2658

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2658

A.9.2.28.1	Test Purpose and Environment	2658

A.9.2.28.2	Test parameters	2658

A.9.2.28.3	Test Requirements	2661

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2661

A.9.2.29.1	Test Purpose and Environment	2661

A.9.2.29.2	Test parameters	2661

A.9.2.29.3	Test Requirements	2664

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2664

A.9.2.30.1	Test Purpose and Environment	2664

A.9.2.30.2	Test parameters	2664

A.9.2.30.3	Test Requirements	2667

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2667

A.9.2.31.1	Test Purpose and Environment	2667

A.9.2.31.2	Test parameters	2667

A.9.2.31.3	Test Requirements	2670

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2670

A.9.2.32.1	Test Purpose and Environment	2670

A.9.2.32.2	Test parameters	2670

A.9.2.32.3	Test Requirements	2673

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2673

A.9.2.33.1	Test Purpose and Environment	2673

A.9.2.33.2	Test parameters	2673

A.9.2.33.3	Test Requirements	2677

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2677

A.9.2.34.1	Test Purpose and Environment	2677

A.9.2.34.2	Test parameters	2677

A.9.2.34.3	Test Requirements	2680

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2680

A.9.2.35.1	Test Purpose and Environment	2680

A.9.2.35.2	Test parameters	2680

A.9.2.35.3	Test Requirements	2683

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2683

A.9.2.36.1	Test Purpose and Environment	2683

A.9.2.36.2	Test parameters	2683

A.9.2.36.3	Test Requirements	2684

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2685

A.9.2.37.1	Test Purpose and Environment	2685

A.9.2.37.2	Test parameters	2685

A.9.2.37.3	Test Requirements	2686

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2687

A.9.2.38.1	Test Purpose and Environment	2687

A.9.2.38.2	Test parameters	2687

A.9.2.38.3	Test Requirements	2691

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2691

A.9.2.39.1	Test Purpose and Environment	2691

A.9.2.39.2	Test parameters	2691

A.9.2.39.3	Test Requirements	2695

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2695

A.9.2.40.1	Test Purpose and Environment	2695

A.9.2.40.2	Test parameters	2695

A.9.2.40.3	Test Requirements	2698

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2699

A.9.2.41.1	Test Purpose and Environment	2699

A.9.2.41.2	Test parameters	2699

A.9.2.41.3	Test Requirements	2702

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2702

A.9.2.42.1	Test Purpose and Environment	2702

A.9.2.42.2	Test parameters	2702

A.9.2.42.3	Test Requirements	2705

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2705

A.9.2.43.1	Test Purpose and Environment	2705

A.9.2.43.2	Test parameters	2705

A.9.2.43.3	Test Requirements	2708

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2708

A.9.2.44.1	Test Purpose and Environment	2708

A.9.2.44.2	Test parameters	2708

A.9.2.44.3	Test Requirements	2710

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2710

A.9.2.45.1	Test Purpose and Environment	2710

A.9.2.45.2	Test parameters	2710

A.9.2.45.3	Test Requirements	2714

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2715

A.9.2.46.1	Test Purpose and Environment	2715

A.9.2.46.2	Test parameters	2715

A.9.2.46.3	Test Requirements	2719

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2720

A.9.2.47.1	Test Purpose and Environment	2720

A.9.2.47.2	Test parameters	2720

A.9.2.47.3	Test Requirements	2724

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2724

A.9.2.48.1	Test Purpose and Environment	2724

A.9.2.48.2	Test parameters	2725

A.9.2.48.3	Test Requirements	2729

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2730

A.9.2.49.1	Test Purpose and Environment	2730

A.9.2.49.2	Test parameters	2730

A.9.2.49.3	Test Requirements	2736

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2737

A.9.2.50.1	Test Purpose and Environment	2737

A.9.2.50.2	Test parameters	2737

A.9.2.50.3	Test Requirements	2743

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2743

A.9.2.51.1	Test Purpose and Environment	2743

A.9.2.51.2	Test parameters	2744

A.9.2.51.3	Test Requirements	2748

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2748

A.9.2.52.1	Test Purpose and Environment	2748

A.9.2.52.2	Test parameters	2748

A.9.2.52.3	Test Requirements	2751

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2752

A.9.2.53.1	Test Purpose and Environment	2752

A.9.2.53.2	Test parameters	2752

A.9.2.53.3	Test Requirements	2758

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2759

A.9.2.54.1	Test Purpose and Environment	2759

A.9.2.54.2	Test parameters	2759

A.9.2.54.3	Test Requirements	2765

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2765

A.9.2.55.1	Test Purpose and Environment	2765

A.9.2.55.2	Test parameters	2766

A.9.2.55.3	Test Requirements	2771

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2771

A.9.2.56.1	Test Purpose and Environment	2771

A.9.2.56.2	Test parameters	2771

A.9.2.56.3	Test Requirements	2776

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2776

A.9.2.57.1	Test Purpose and Environment	2776

A.9.2.57.2	Test parameters	2777

A.9.2.57.3	Test Requirements	2781

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2782

A.9.2.58.1	Test Purpose and Environment	2782

A.9.2.58.2	Test parameters	2782

A.9.2.58.3	Test Requirements	2786

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2787

A.9.2.59.1	Test Purpose and Environment	2787

A.9.2.59.2	Test parameters	2787

A.9.2.59.3	Test Requirements	2792

A.9.3	UTRAN FDD CPICH RSCP	2793

A.9.3.1	E-UTRAN FDD	2793

A.9.3.1.1	Test Purpose and Environment	2793

A.9.3.1.2	Parameters	2793

A.9.3.1.3	Test Requirements	2795

A.9.3.2 E-UTRAN TDD	2795

A.9.3.2.1	Test Purpose and Environment	2795

A.9.3.2.2	Parameters	2795

A.9.3.2.3	Test Requirements	2798

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2798

A.9.3.3.1	Test Purpose and Environment	2798

A.9.3.3.2	Parameters	2798

A.9.3.3.3	Test Requirements	2799

A.9.4	UTRAN FDD CPICH Ec/No	2800

A.9.4.1	E-UTRAN FDD	2800

A.9.4.1.1	Test Purpose and Environment	2800

A.9.4.1.2	Parameters	2800

A.9.4.1.3	Test Requirements	2802

A.9.4.2	E-UTRAN TDD	2803

A.9.4.2.1	Test Purpose and Environment	2803

A.9.4.2.2	Parameters	2803

A.9.4.2.3	Test Requirements	2805

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2806

A.9.4.3.1	Test Purpose and Environment	2806

A.9.4.3.2	Parameters	2806

A.9.4.3.3	Test Requirements	2807

A.9.5	UTRAN TDD measurement	2807

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2807

A.9.5.1.1	Test Purpose and Environment	2807

A.9.5.1.2	Test parameters	2807

A.9.5.1.3	Test Requirements	2809

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2809

A.9.5.2.1	Test Purpose and Environment	2809

A.9.5.2.2	Test parameters	2809

A.9.5.2.3	Test Requirements	2811

A.9.6	GSM Carrier RSSI	2812

A.9.6.1	E-UTRAN FDD	2812

A.9.6.1.1	Test Purpose and Environment	2812

A.9.6.1.2	Test Requirements	2813

A.9.6.2	E-UTRAN TDD	2813

A.9.6.2.1	Test Purpose and Environment	2813

A.9.6.2.2	Test Requirements	2815

A.9.7	UE Rx – Tx Time Difference	2816

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2816

A.9.7.1.1	Test Purpose and Environment	2816

A.9.7.1.2	Test parameters	2816

A.9.7.1.3	Test Requirements	2817

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2817

A.9.7.2.1	Test Purpose and Environment	2817

A.9.7.2.2	Test parameters	2817

A.9.7.2.3	Test Requirements	2819

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2819

A.9.7.3.1	Test Purpose and Environment	2819

A.9.7.3.2	Test parameters	2819

A.9.7.3.3	Test Requirements	2822

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2822

A.9.7.4.1	Test Purpose and Environment	2822

A.9.7.4.2	Test Parameters	2822

A.9.7.4.3	Test Requirements	2825

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2825

A.9.7.5.1	Test Purpose and Environment	2825

A.9.7.5.2	Test parameters	2825

A.9.7.5.3	Test Requirements	2828

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2828

A.9.7.6.1	Test Purpose and Environment	2828

A.9.7.6.2	Test Parameters	2828

A.9.7.6.3	Test Requirements	2831

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2831

A.9.7.7.1	Test Purpose and Environment	2831

A.9.7.7.2	Test parameters	2831

A.9.7.7.3	Test Requirements	2832

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2833

A.9.7.8.1	Test Purpose and Environment	2833

A.9.7.8.2	Test parameters	2833

A.9.7.8.3	Test Requirements	2834

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2834

A.9.7.9.1	Test Purpose and Environment	2834

A.9.7.9.2	Test parameters	2834

A.9.7.9.3	Test Requirements	2835

A.9.8	RSTD	2836

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2836

A.9.8.1.1	Test Purpose and Environment	2836

A.9.8.1.2	Test Requirements	2838

A.9.8.1.2A	Test Requirements for UE Category 1bis	2838

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2839

A.9.8.2.1	Test Purpose and Environment	2839

A.9.8.2.2	Test Requirements	2842

A.9.8.2.2A	Test Requirements for UE Category 1bis	2842

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2843

A.9.8.3.1	Test Purpose and Environment	2843

A.9.8.3.2	Test Requirements	2845

A.9.8.3.2A	Test Requirements for UE Category 1bis	2845

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2846

A.9.8.4.1	Test Purpose and Environment	2846

A.9.8.4.2	Test Requirements	2848

A.9.8.4.2A	Test Requirements for UE Category 1bis	2848

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2849

A.9.8.5.1	Test Purpose and Environment	2849

A.9.8.5.2	Test Requirements	2851

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2851

A.9.8.6.1	Test Purpose and Environment	2851

A.9.8.6.2	Test Requirements	2854

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2854

A.9.8.7.1	Test Purpose and Environment	2854

A.9.8.7.2	Test Requirements	2855

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2855

A.9.8.8.1	Test Purpose and Environment	2855

A.9.8.8.2	Test Requirements	2856

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2856

A.9.8.9.1	Test Purpose and Environment	2856

A.9.8.9.2	Test Requirements	2857

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2857

A.9.8.10.1	Test Purpose and Environment	2857

A.9.8.10.2	Test Requirements	2858

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2858

A.9.8.11.1	Test Purpose and Environment	2858

A.9.8.11.2	Test Requirements	2859

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2859

A.9.8.12.1	Test Purpose and Environment	2859

A.9.8.12.2	Test Requirements	2860

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2860

A.9.8.13.1	Test Purpose and Environment	2860

A.9.8.13.2	Test Requirements	2861

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2861

A.9.8.14.1	Test Purpose and Environment	2861

A.9.8.14.2	Test Requirements	2868

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2868

A.9.8.15.1	Test Purpose and Environment	2868

A.9.8.15.2	Test Requirements	2874

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2874

A.9.8.16.1	Test Purpose and Environment	2874

A.9.8.16.2	Test Requirements	2877

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2877

A.9.8.17.1	Test Purpose and Environment	2877

A.9.8.17.2	Test Requirements	2880

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2880

A.9.8.18.1	Test Purpose and Environment	2880

A.9.8.18.2	Test Requirements	2883

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2883

A.9.8.19.1	Test Purpose and Environment	2883

A.9.8.19.2	Test Requirements	2886

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2886

A.9.8.20.1	Test Purpose and Environment	2886

A.9.8.20.2	Test Requirements	2889

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2889

A.9.8.21.1	Test Purpose and Environment	2889

A.9.8.21.2	Test Requirements	2893

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2894

A.9.8.22.1	Test Purpose and Environment	2894

A.9.8.22.2	Test Requirements	2897

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2898

A.9.8.23.1	Test Purpose and Environment	2898

A.9.8.23.2	Test Requirements	2901

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2901

A.9.8.24.1	Test Purpose and Environment	2901

A.9.8.24.2	Test Requirements	2905

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2905

A.9.8.25.1	Test Purpose and Environment	2905

A.9.8.25.2	Test Requirements	2909

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2910

A.9.8.26.1	Test Purpose and Environment	2910

A.9.8.26.2	Test Requirements	2912

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2912

A.9.8.27.1	Test Purpose and Environment	2912

A.9.8.27.2	Test Requirements	2915

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2915

A.9.8.28.1	Test Purpose and Environment	2915

A.9.8.28.2	Test Requirements	2918

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2918

A.9.8.29.1	Test Purpose and Environment	2918

A.9.8.29.2	Test Requirements	2922

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2923

A.9.8.30.1	Test Purpose and Environment	2923

A.9.8.30.2	Test Requirements	2926

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2927

A.9.8.31.1	Test Purpose and Environment	2927

A.9.8.31.2	Test Requirements	2931

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2931

A.9.8.32.1	Test Purpose and Environment	2931

A.9.8.32.2	Test Requirements	2934

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2934

A.9.8.33.1	Test Purpose and Environment	2934

A.9.8.33.2	Test Requirements	2937

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2937

A.9.8.34.1	Test Purpose and Environment	2937

A.9.8.34.2	Test Requirements	2940

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2940

A.9.8.35.1	Test Purpose and Environment	2940

A.9.8.35.2	Test Requirements	2943

A.9.9	RSRP and RSRQ on the serving cell	2943

A.9.9.1	FDD Intra frequency serving cell case	2943

A.9.9.1.1	Test Purpose and Environment	2943

A.9.9.1.2	Test parameters	2943

A.9.9.1.3	Test Requirements	2946

A.9.9.2	TDD Intra frequency serving cell case	2946

A.9.9.2.1	Test Purpose and Environment	2946

A.9.9.2.2	Test parameters	2946

A.9.9.2.3	Test Requirements	2948

A.9.10	SSTD	2948

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2948

A.9.10.1.1	Test Purpose and Environment	2948

A.9.10.1.2	Test parameters	2948

A.9.10.1.3	Test Requirements	2950

A.9.10.2	Void	2950

A.9.10.3	Void	2950

A.9.10.4	Void	2950

A.9.11 RSSI	2950

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2950

A.9.11.1.1	Test Purpose and Environment	2950

A.9.11.1.2	Test parameters	2950

A.9.11.1.3	Test Requirements	2953

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2953

A.9.11.2.1	Test Purpose and Environment	2953

A.9.11.2.2	Test parameters	2953

A.9.12	Channel occupancy	2956

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2956

A.9.12.1.1	Test Purpose and Environment	2956

A.9.12.1.2	Test parameters	2956

A.9.12.1.3	Test Requirements	2959

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2959

A.9.12.2.1	Test Purpose and Environment	2959

A.9.12.2.2	Test parameters	2959

A.9.12.2.3	Test Requirements	2962

A.9.13	RS-SINR	2962

A.9.13.1	FDD Intra-Frequency Case	2962

A.9.13.1.1	Test Purpose and Environment	2962

A.9.13.1.2	Test parameters	2962

A.9.13.1.3	Test Requirements	2965

A.9.13.2	TDD Intra-Frequency Case	2965

A.9.13.2.1	Test Purpose and Environment	2965

A.9.13.2.2	Test parameters	2965

A.9.13.2.3	Test Requirements	2968

A.9.13.3	FDD—FDD Inter frequency case	2968

A.9.13.3.1	Test Purpose and Environment	2968

A.9.13.3.2	Test parameters	2968

A.9.13.3.3	Test Requirements	2971

A.9.13.4	TDD—TDD Inter frequency case	2971

A.9.13.4.1	Test Purpose and Environment	2971

A.9.13.4.2	Test parameters	2971

A.9.13.4.3	Test Requirements	2975

A.9.13.5	FDD—TDD Inter frequency case	2976

A.9.13.5.1	Test Purpose and Environment	2976

A.9.13.5.2	Test parameters	2976

A.9.13.5.3	Test Requirements	2982

A.9.13.6	TDD—FDD Inter frequency case	2982

A.9.13.6.1	Test Purpose and Environment	2982

A.9.13.6.2	Test parameters	2982

A.9.13.6.3	Test Requirements	2986

A.9.14	Channel quality reporting accuracy	2986

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2986

A.9.14.1.1	Test Purpose and Environment	2986

A.9.14.1.2	Test parameters	2987

A.9.14.1.3	Test Requirements	2987

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2987

A.9.14.2.1	Test Purpose and Environment	2987

A.9.14.2.2	Test parameters	2988

A.9.14.2.3	Test Requirements	2988

A.10	Proximity-based Services in Any Cell Selection State	2989

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2989

A.10.1.1	Test Purpose and Environment	2989

A.10.1.2	Test Requirements	2990

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2990

A.10.2.1	Test Purpose and Environment	2990

A.10.2.2	Test Requirements	2991

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2992

A.10.3.1	Test Purpose and Environment	2992

A.10.3.2	Test Requirements	2994

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2995

A.10.4.1	Test Purpose and Environment	2995

A.10.4.2	Test Requirements	2997

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2997

A.11.1	V2V UE Transmission Timing Accuracy Test	2997

A.11.1.1	Test Purpose and Environment	2997

A.11.1.2	Test requirements	2998

A.11.2	Interruptions due to V2V sidelink communication	2998

A.11.2.1	Test Purpose and Environment	2998

A.11.2.2	Test Requirements	3000

A.12	3000

A.12.1	V2X UE Transmission Timing Accuracy Test	3000

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3000

A.12.1.1.1	Test Purpose and Environment	3000

A.12.1.1.2	Test requirements	3001

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3002

A.12.1.2.1	Test Purpose and Environment	3002

A.12.1.2.2	Test Requirements	3002

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3003

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3003

A.12.2.1.1	Test Purpose and Environment	3003

A.12.2.1.2	Test Requirements	3004

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3005

A.12.2.2.1	Test Purpose and Environment	3005

A.12.2.2.2	Test Requirements	3006

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3006

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3006

A.12.3.1.1	Test Purpose and Environment	3006

A.12.3.1.2	Test Requirements	3009

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3010

A.12.3.2.1	Test Purpose and Environment	3010

A.12.3.1.2	Test Requirements	3012

A.12.4	Congestion Control Measurement Test for V2X UE	3012

A.12.4.1	Test Purpose and Environment	3012

A.12.4.2	Test Requirements	3014

A.12.5	Interruptions due to V2X Sidelink Communication	3014

A.12.5.1	Test Purpose and Environment	3014

A.12.5.2	Test Requirements	3016

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3016

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3016

A.12.6.1.1	Test Purpose and Environment	3016

A.12.6.1.2	Test Requirements	3018

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3018

A.12.6.2.1	Test Purpose and Environment	3018

A.12.6.1.2	Test Requirements	3020

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3020

A.12.7.1	Test Purpose and Environment	3020

A.12.7.2	Test Requirements	3023

A.12.8	Interruptions due to V2X Carrier Aggregation	3024

A.12.8.1	Interruptions on a FDD PCell	3024

A.12.8.1.1	Test Purpose and Environment	3024

A.12.8.1.2	Test Requirements	3025

A.12.8.2	Interruptions on a TDD PCell	3025

A.12.8.2.1	Test Purpose and Environment	3025

A.12.8.2.2	Test Requirements	3027

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	3028

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3028

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3028

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3028

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3028

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3030

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3031

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3031

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3031

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3031

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3033

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3034

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3034

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3034

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3035

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3035

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3036

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3036

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3036

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3037

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3037

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3038

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3038

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3038

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3038

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3039

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3039

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3039

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3040

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3040

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3040

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3041

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3041

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3042

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3043

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3044

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3045

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3046

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3046

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3046

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3046

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3047

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3047

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3049

B.3	Conditions for measurements performance requirements for UE	3049

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3049

B.3.2	Void	3049

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3049

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3049

B.3.5	Conditions for UE Rx – Tx time difference	3049

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3050

B.3.7	Conditions for inter-frequency RSTD measurements	3050

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3050

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3050

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3050

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3051

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3051

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3051

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3051

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3051

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3051

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3052

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3052

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3052

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3053

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3053

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3053

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3053

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3053

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3053

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3053

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3053

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3053

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3054

B.3.21.1	Conditions for RSRP measurements	3054

B.3.21.2	Conditions for RSRQ measurements	3054

B.3.21.3	Conditions for CSI-RSRP measurements	3054

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3054

B.3.22.1	Conditions for RSRP measurements	3054

B.3.22.2	Void	3055

B.3.22.3	Conditions for CSI-RSRP measurements	3055

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3055

B.3.23.1	Conditions for RSRP measurements	3055

B.3.23.2	Conditions for RSRQ measurements	3055

B.3.23.3	Conditions for CSI-RSRP measurements	3055

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3056

B.3.24.1	Conditions for RSRP measurements	3056

B.3.24.2	Conditions for RSRQ measurements	3056

B.3.24.3	Conditions for CSI-RSRP measurements	3056

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3056

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3056

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3056

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3057

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3057

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3057

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3057

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3057

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3058

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3058

B.4	RRM Requirements Exceptions	3058

B.4.1	General	3058

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3058

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3058

B.4.3.1	Inter-band carrier aggregation	3058

B.4.3.2	Intra-band non-contiguous carrier aggregation	3058

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3059

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3059

B.5.1	Conditions for S-RSRP Accuracy Requirements	3059

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3059

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3060

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3060

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3061

B.6	Conditions for V2X	3061

B.6.1	Test parameters for GNSS signals	3061

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3061

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3061

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3062

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3062

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3062

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3062

Annex C (informative):	Change history:	3064