| 3GPP TS 38.133 V16.6.0 (2020-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	31

1	Scope	33

2	References	33

3	Definitions, symbols and abbreviations	34

3.1	Definitions	34

3.2	Symbols	35

3.3	Abbreviations	35

3.4	Test tolerances	38

3.5	Frequency bands grouping	38

3.5.1	Introduction	38

3.5.2	NR operating bands in FR1	38

3.5.3	NR operating bands in FR2	38

3.6	Applicability of requirements in this specification version	39

3.6.1	RRC connected state requirements in DRX	39

3.6.2	Number of serving carriers	40

3.6.2.1	Number of serving carriers for SA	40

3.6.2.2	Number of serving carriers for EN-DC	40

3.6.2.3	Number of serving carriers for NE-DC	40

3.6.2.4	Number of serving carriers for NR-DC	40

3.6.3	Applicability for intra-band FR2	41

3.6.4	Applicability for FR2 UE power classes	41

3.6.5	Applicability for SDL bands	41

3.6.6	Applicability of requirements for NGEN-DC operation	41

3.6.7	Applicability of QCL	41

3.6.9	Applicability of requirements for scheduling availability	41

4	SA: RRC_IDLE state mobility	42

4.1	Cell Selection	42

4.2	Cell Re-selection	42

4.2.1	Introduction	42

4.2.2	Requirements	42

4.2.2.1	UE measurement capability	42

4.2.2.2	Measurement and evaluation of serving cell	43

4.2.2.3 	Measurements of intra-frequency NR cells	43

4.2.2.4	Measurements of inter-frequency NR cells	44

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	46

4.2.2.6	Maximum interruption in paging reception	47

4.2.2.7	General requirements	48

4.2.2.8	Minimum requirement at transitions	48

4.2.2.9 	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	48

4.2.2.9.1 	Introduction	48

4.2.2.9.2 	Measurements for UE fulfilling low mobility criterion	48

4.2.2.9.3 	Measurements for UE fulfilling not-at-cell edge criterion	49

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	50

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	50

4.2.2.10.1	Introduction	50

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	50

4.2.2.10.3 	Measurements for UE fulfilling not-at-cell edge criterion	51

4.2.2.10.4 	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	51

4.2.2.11 	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	52

4.2.2.11.2 	Measurements for UE fulfilling low mobility criterion	52

4.2.2.11.3 	Measurements for UE fulfilling with not-at-cell edge criterion	53

4.2.2.11.4 	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	53

4.2A	Cell Re-selection when subject to CCA	54

4.2A.1	Introduction	54

4.2A.2	Requirements	54

4.2A.2.1	UE measurement capability	54

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	54

4.2A.2.3 	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	55

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	56

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	58

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	58

4.2A.2.7	General requirements	58

4.3	Minimization of Drive Tests (MDT)	59

4.3.1	Introduction	59

4.3.2	Measurement Requirements	59

4.3.3	Requirements for Relative Time Stamp Accuracy	59

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	60

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	60

4.4	Idle Mode CA/DC Measurements	60

4.4.1	Introduction	60

4.4.2	Measurement Requirements	60

4.4.2.1	Detected cell requirement during state transition and Idle mode	60

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	61

4.4.2.3	Measurements on serving cell	61

5	SA: RRC_INACTIVE state mobility	62

5.1	Cell Re-selection	62

5.1.1	Introduction	62

5.1.2	Requirements	62

5.1.2.1	UE measurement capability	62

5.1.2.2	Measurement and evaluation of serving cell	62

5.1.2.3	Measurements of intra-frequency NR cells	62

5.1.2.4	Measurements of inter-frequency NR cells	62

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	62

5.1.2.6	Maximum interruption in paging reception	62

5.1.2.7	General requirements	62

5.1A	Cell Re-selection with CCA	62

5.1A.1	Introduction	62

5.1A.2	Requirements	63

5.1A.2.1	UE measurement capability	63

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	63

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	63

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	63

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	63

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	63

5.1A.2.7	General requirements	63

5.2	Void	63

5.3	Minimization of Drive Tests (MDT)	63

5.3.1	Introduction	63

5.3.2	Measurement Requirements	64

5.3.3	Requirements for Relative Time Stamp Accuracy	64

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	64

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	64

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	64

5.4	Idle Mode CA/DC Measurements	64

5.4.1	Introduction	64

5.4.2	Measurement Requirements	64

5.4.2.1	Detected cell requirement during state transition and Idle mode	65

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	65

5.4.2.3	Measurements on serving cell	65

6	RRC_CONNECTED state mobility	65

6.1	Handover	65

6.1.1	NR Handover	65

6.1.1.1	Introduction	65

6.1.1.2	NR FR1 - NR FR1 Handover	65

6.1.1.2.1	Handover delay	65

6.1.1.2.2	Interruption time	65

6.1.1.3	NR FR2- NR FR1 Handover	66

6.1.1.3.1	Handover delay	66

6.1.1.3.2	Interruption time	66

6.1.1.4	NR FR2- NR FR2 Handover	67

6.1.1.4.1	Handover delay	67

6.1.1.4.2	Interruption time	67

6.1.1.5	NR FR1- NR FR2 Handover	68

6.1.1.5.1	Handover delay	68

6.1.1.5.2	Interruption time	68

6.1.2	NR Handover to other RATs	69

6.1.2.1	NR – E-UTRAN Handover	69

6.1.2.1.1	Introduction	69

6.1.2.1.2	Handover delay	69

6.1.2.1.3	Interruption time	69

6.1.2.2	NR – UTRAN Handover	69

6.1.2.2.1	Introduction	69

6.1.2.2.2	Handover delay	69

6.1.2.2.3	Interruption time	70

6.1.3	NR DAPS Handover	70

6.1.3.1	Introduction	70

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	71

6.1.3.2.1	DAPS handover delay	71

6.1.3.2.2	Interruption time	72

6.1.3.3	NR FR2- NR FR1 DAPS Handover	73

6.1.3.3.1	DAPS handover delay	73

6.1.3.3.2	Interruption time	74

6.1.3.4	NR FR1- NR FR2 DAPS Handover	74

6.1.3.4.1	DAPS handover delay	75

6.1.3.4.2	Interruption time	75

6.1.4	NR Conditional Handover	75

6.1.4.1	Introduction	75

6.1.4.2	NR FR1 – NR FR1 conditional handover	76

6.1.4.3	NR FR2 – NR FR1 conditional handover	77

6.1.4.4	NR FR2 – NR FR2 conditional handover	77

6.1.4.4.1	Handover delay	77

6.1.4.4.2	Measurement time	77

6.1.4.4.3	Preparation time	78

6.1.4.4.4	Interruption time	78

6.1.4.5	NR FR1 – NR FR2 conditional handover	78

6.1A	Void	79

6.1A.1	Void	79

6.1A.1.1	Void	79

6.1A.1.2	Void	79

6.1A.1.2.1	Void	79

6.1A.1.2.2	Void	79

6.1B	Handover to target cell using CCA	79

6.1B.1	NR Handover	79

6.1B.1.1	Introduction	79

6.1B.1.2	NR FR1 - NR FR1 Handover	79

6.1B.1.2.1	Handover delay	79

6.1B.1.2.2	Interruption time	79

6.2	RRC Connection Mobility Control	80

6.2.1	SA: RRC Re-establishment	80

6.2.1.1	Introduction	80

6.2.1.2	Requirements	80

6.2.1.2.1	UE Re-establishment delay requirement	80

6.2.1A	RRC Re-establishment with CCA	82

6.2.1A.1	Introduction	82

6.2.1A.2	Requirements	82

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	82

6.2.2	Random access	84

6.2.2.1	Introduction	84

6.2.2.2	Requirements for 4-step RA type	84

6.2.2.2.1	Contention based random access	84

6.2.2.2.2	Non-Contention based random access	85

6.2.2.2.3	UE behaviour when configured with supplementary UL	86

6.2.2.3	Requirements for 2-step RA type	86

6.2.2.3.1	Contention based random access	87

6.2.2.3.2	Non-Contention based random access	87

6.2.2.3.3	UE behaviour when configured with supplementary UL	88

6.2.3	SA: RRC Connection Release with Redirection	88

6.2.3.1	Introduction	88

6.2.3.2	Requirements	88

6.2.3.2.1	RRC connection release with redirection to NR	88

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	89

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	90

7	Timing	91

7.1	UE transmit timing	91

7.1.1	Introduction	91

7.1.2	Requirements	91

7.1.2.1	Gradual timing adjustment	92

7.1.2.2	Void	93

7.2	UE timer accuracy	93

7.2.1	Introduction	93

7.2.2	Requirements	93

7.3	Timing advance	93

7.3.1	Introduction	93

7.3.2	Requirements	93

7.3.2.1	Timing Advance adjustment delay	93

7.3.2.2	Timing Advance adjustment accuracy	94

7.4	Cell phase synchronization accuracy	94

7.4.1	Definition	94

7.4.2	Minimum requirements	94

7.5	Maximum Transmission Timing Difference	94

7.5.1	Introduction	94

7.5.2	Minimum Requirements for inter-band EN-DC	94

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	95

7.5.3	Minimum Requirements for intra-band EN-DC	95

7.5.4	Minimum Requirements for NR Carrier Aggregation	95

7.5.5	Minimum Requirements for inter-band NE-DC	96

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	96

7.5.6	Minimum Requirements for inter-band NR DC	96

7.6	Maximum Receive Timing Difference	97

7.6.1	Introduction	97

7.6.2	Minimum Requirements for inter-band EN-DC	97

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	98

7.6.3	Minimum Requirements for intra-band EN-DC	98

7.6.4	Minimum Requirements for NR Carrier Aggregation	99

7.6.5	Minimum Requirements for inter-band NE-DC	99

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	100

7.6.6	Minimum Requirements for inter-band NR DC	100

7.7	deriveSSB-IndexFromCell tolerance	101

7.7.1	Minimum requirements	101

7.8	Void	101

8	Signalling characteristics	101

8.1	Radio Link Monitoring	101

8.1.1	Introduction	101

8.1.2	Requirements for SSB based radio link monitoring	102

8.1.2.1	Introduction	102

8.1.2.2	Minimum requirement	103

8.1.2.3	Measurement restrictions for SSB based RLM	105

8.1.3	Requirements for CSI-RS based radio link monitoring	105

8.1.3.1	Introduction	105

8.1.3.2	Minimum requirement	106

8.1.3.3	Measurement restrictions for CSI-RS based RLM	108

8.1.4	Minimum requirement at transitions	109

8.1.5	Minimum requirement for UE turning off the transmitter	109

8.1.6	Minimum requirement for L1 indication	109

8.1.7	Scheduling availability of UE during radio link monitoring	110

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	110

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	110

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	110

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	111

8.1A	Radio Link Monitoring with CCA on Target Frequency	111

8.1A.1	Introduction	111

8.1A.2	Requirements for SSB Based Radio Link Monitoring	112

8.1A.2.1	Introduction	112

8.1A.2.2	Minimum Requirement	113

8.1A.3	Minimum requirement at transitions	114

8.1A.4	Minimum requirement for UE turning off the transmitter	114

8.1A.5	Minimum requirement for L1 indication	114

8.1A.6	Scheduling availability of UE during radio link monitoring	114

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	114

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	115

8.2	Interruption	115

8.2.1	EN-DC Interruption	115

8.2.1.1	Introduction	115

8.2.1.2	Requirements	116

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	116

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	116

8.2.1.2.3	Interruptions at SCell addition/release	116

8.2.1.2.4	Interruptions at SCell activation/deactivation	117

8.2.1.2.5	Interruptions during measurements on SCC	118

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	119

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	120

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	121

8.2.1.2.9	Interruptions at SCell hibernation	121

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	122

8.2.1.2.11	Interruptions due to UE-specific CBW change	122

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	122

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	123

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	124

8.2.1.2.15	Interruptions due to SCell dormancy	125

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	125

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	126

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	127

8.2.2.1	Introduction	127

8.2.2.2	Requirements	127

8.2.2.2.1	Interruptions at SCell addition/release	127

8.2.2.2.2	Interruptions at SCell activation/deactivation	128

8.2.2.2.3	Interruptions during measurements on deactivated SCC	129

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	129

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	130

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	131

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	132

8.2.2.2.8	Interruptions due to UE-specific CBW change	132

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	132

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	134

8.2.2.2.11	Interruptions at direct SCell activation	134

8.2.2.2.12	Interruptions due to SCell dormancy	134

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	135

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	135

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	136

8.2.3	NE-DC Interruptions	136

8.2.3.1	Introduction	136

8.2.3.2	Requirements	137

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	137

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	137

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	137

8.2.3.2.4	Interruptions at SCell activation/deactivation	138

8.2.3.2.5	Interruptions during measurements on SCC	139

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	141

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	141

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	141

8.2.3.2.9	Interruptions at SCell hibernation	142

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	142

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	142

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	143

8.2.3.2.13	Interruptions due to SCell dormancy	144

8.2.3.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	145

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	145

8.2.4	NR-DC: Interruptions	146

8.2.4.1	Introduction	146

8.2.4.2	Requirements	147

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	147

8.2.4.2.2	Interruptions at SCell activation/deactivation	147

8.2.4.2.3	Interruptions during measurements on SCC	148

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	149

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	149

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	149

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	150

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	150

8.2.4.2.9	Interruptions at NR SRS carrier based switching	150

8.2.4.2.10	Interruptions at direct SCell activation	151

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	152

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	152

8.2.4.2A	Interruptions due to SCell dormancy	153

8.2.4.2A.1	Interruptions due to SCell dormancy switch	153

8.2.4.2A.2	Interruptions due to CQI measurements during SCell dormancy	153

8.2.4.2A.3	Interruptions due to RRM measurements during SCell dormancy	153

8.3	SCell Activation and Deactivation Delay	153

8.3.1	Introduction	153

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	154

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	157

8.3.4	Direct SCell Activation at SCell addition	157

8.3.5	Direct SCell Activation at Handover	158

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	159

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	162

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	162

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	164

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	165

8.3A.1	Introduction	165

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	165

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	167

8.4	UE UL carrier RRC reconfiguration delay	168

8.4.1	Introduction	168

8.4.2	UE UL carrier configuration delay requirement	168

8.4.3	UE UL carrier deconfiguration delay requirement	168

8.5	Link Recovery Procedures	168

8.5.1	Introduction	168

8.5.2	Requirements for SSB based beam failure detection	169

8.5.2.1	Introduction	169

8.5.2.2	Minimum requirement	169

8.5.2.3	Measurement restriction for SSB based beam failure detection	171

8.5.3	Requirements for CSI-RS based beam failure detection	171

8.5.3.1	Introduction	171

8.5.3.2	Minimum requirement	172

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	174

8.5.4	Minimum requirement for L1 indication	175

8.5.5	Requirements for SSB based candidate beam detection	175

8.5.5.1	Introduction	175

8.5.5.2	Minimum requirement	175

8.5.5.3	Measurement restriction for SSB based candidate beam detection	177

8.5.6	Requirements for CSI-RS based candidate beam detection	178

8.5.6.1	Introduction	178

8.5.6.2	Minimum requirement	178

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	180

8.5.7	Scheduling availability of UE during beam failure detection	181

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	181

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	181

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	181

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	182

8.5.8	Scheduling availability of UE during candidate beam detection	182

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	182

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	182

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	182

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	183

8.5.9	Requirements for Link Recovery with Link Recovery Request (LRR)	183

8.5.9.1	Introduction	183

8.5.9.2	Requirement	183

8.5A	Link Recovery Procedures when CCA is used on target frequency	183

8.5A.1	Introduction	183

8.5A.2	Requirements for SSB based beam failure detection	184

8.5A.2.1	Introduction	184

8.5A.2.2	Minimum requirement	184

8.5A.2.3	Measurement restriction for SSB based beam failure detection	185

8.5A.4	Minimum requirement for L1 indication	185

8.5A.5	Requirements for SSB based candidate beam detection	186

8.5A.5.1	Introduction	186

8.5A.5.2	Minimum requirement	186

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	186

8.5A.7	Scheduling availability of UE during beam failure detection	187

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	187

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	187

8.5A.8	Scheduling availability of UE during candidate beam detection	187

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	187

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	187

8.6	Active BWP switch delay	187

8.6.1	Introduction	187

8.6.2	DCI and timer based BWP switch delay on a single CC	187

8.6.2A	DCI based BWP switch delay on multiple CCs	189

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	189

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	190

8.6.2B	Timer based BWP switch delay on multiple CCs	190

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	190

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	191

8.6.3	RRC based BWP switch delay on a single CC	191

8.6.3A	RRC based BWP switch delay on multiple CCs	192

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	192

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	192

8.6.4	BWP switch delay on Consistent UL LBT recovery	193

8.7	Void	193

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	193

8.8.1	Introduction	193

8.8.2	E-UTRAN PSCell Addition Delay Requirement	193

8.8.3	E-UTRAN PSCell Release Delay Requirement	194

8.9	NR-DC: PSCell Addition and Release Delay	194

8.9.1	Introduction	194

8.9.2	PSCell Addition Delay Requirement	194

8.9.3	PSCell Release Delay Requirement	195

8.10	Active TCI state switching delay	195

8.10.4	DCI based TCI state switch delay	196

8.10.5	RRC based TCI state switch delay	196

8.10.6	Active TCI state list update delay	197

8.10A	Active TCI state switching delay with CCA	197

8.10A.1	Introduction	197

8.10A.2	Known conditions for TCI state	197

8.10A.3	MAC-CE based TCI state switch delay	197

8.10A.4	DCI based TCI state switch delay	198

8.10A.5	RRC based TCI state switch delay	199

8.10A.6	Active TCI state list update delay	199

8.11	PSCell Change	199

8.11A	void	200

8.11B	Conditional PSCell Change	200

8.11B.1	Introduction	200

8.11B.2	Conditoinal PSCell Change delay	200

8.11B.2.1	Measurement time	201

8.12	Uplink spatial relation switch delay	201

8.12.1	Introduction	201

8.12.2	Known conditions for spatial relation when associated with DL-RS	201

8.12.3	MAC-CE based spatial relation switch delay	201

8.12.4	DCI based spatial relation switch delay	202

8.12.5	RRC based spatial relation switch delay	202

8.13	UE-specific CBW change	202

8.13.1	Introduction	202

8.13.2	UE-specific CBW change delay	202

8.14	Pathloss reference signal switching delay	202

8.14.1	Introduction	202

8.14.2	Known conditions for pathloss reference signal	203

8.14.3	MAC-CE based pathloss reference signal switch delay	203

9	Measurement Procedure	203

9.1	General measurement requirement	203

9.1.1	Introduction	203

9.1.2	Measurement gap	204

9.1.2.1	EN-DC: Measurement Gap Sharing	214

9.1.2.1a	SA: Measurement Gap Sharing	215

9.1.2.1b	NE-DC: Measurement Gap Sharing	216

9.1.2.1c	NR-DC: Measurement Gap Sharing	217

9.1.3	UE Measurement capability	217

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	217

9.1.3.1a	SA: Monitoring of multiple layers using gaps	218

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	218

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	219

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	219

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	220

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	221

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	221

9.1A.3.2	EN-DC: Maximum allowed layers for multiple monitoring with CCA	222

9.1A.3.2a	SA: Maximum allowed layers for multiple monitoring with CCA	223

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	223

9.1.4.1	Introduction	223

9.1.4.2	Requirements	223

9.1.5	Carrier-specific scaling factor	227

9.1.5.1	Monitoring of multiple layers outside gaps	227

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	228

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	229

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	230

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	230

9.1.5.2	Monitoring of multiple layers within gaps	231

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed within gaps	232

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed within gaps	233

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps	234

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps	235

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	237

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	237

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	237

9.1.6	Minimum requirement at transitions	237

9.2	NR intra-frequency measurements	237

9.2.1	Introduction	237

9.2.2	Requirements applicability	238

9.2.3	Number of cells and number of SSB	238

9.2.3.1	Requirements for FR1	238

9.2.3.2	Requirements for FR2	239

9.2.4	Measurement Reporting Requirements	239

9.2.4.1	Periodic Reporting	239

9.2.4.2	Event-triggered Periodic Reporting	239

9.2.4.3	Event Triggered Reporting	239

9.2.5	Intrafrequency measurements without measurement gaps	240

9.2.5.1	Intrafrequency cell identification	240

9.2.5.2	Measurement period	242

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	243

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	243

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	244

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	244

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	245

9.2.5.4	SFTD Measurements between PCell and PSCell	245

9.2.5.4.1	Introduction	245

9.2.5.4.2	SFTD Measurement delay	245

9.2.5.4.3	SFTD Measurement Reporting Delay	246

9.2.6	Intra-frequency measurements with measurement gaps	246

9.2.6.1	Void	246

9.2.6.2	Intra-frequency cell identification	246

9.2.6.3	Intrafrequency Measurement Period	247

9.2A	NR intra-frequency measurements with CCA	248

9.2A.1	Introduction	248

9.2A.2	Requirements applicability	249

9.2A.3	Number of cells and number of SSB	249

9.2A.4	Measurement Reporting Requirements	249

9.2A.5	Intra-frequency measurements without measurement gaps	250

9.2A.5.2	Measurement period	252

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	253

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	253

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	253

9.2A.6	Intra-frequency measurements with measurement gaps	254

9.2A.6.1	Intra-frequency cell identification	254

9.2A.6.2	Intra-frequency Measurement Period	255

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	255

9.2A.7.1	Intra-frequency RSSI measurements	255

9.2A.7.2	Intra-frequency Channel occupancy measurements	256

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	257

9.3	NR inter-frequency measurements	257

9.3.1	Introduction	257

9.3.2	Requirements applicability	258

9.3.2.1	Void	258

9.3.2.2	Void	258

9.3.3	Number of cells and number of SSB	258

9.3.3.1	Requirements for FR1	258

9.3.3.2	Requirements for FR2	259

9.3.4	Inter-frequency measurement with measurement gaps	259

9.3.4.1	Void	260

9.3.4.2	Void	260

9.3.5	Inter-frequency measurements	260

9.3.5.1	Void	261

9.3.5.2	Void	261

9.3.5.3	Void	261

9.3.6	Inter-frequency measurements reporting requirements	261

9.3.6.1	Periodic Reporting	261

9.3.6.2	Event-triggered Periodic Reporting	261

9.3.6.3	Event-triggered Reporting	261

9.3.7	Void	262

9.3.8	Inter-frequency SFTD measurement requirements	262

9.3.8.1	Introduction	262

9.3.8.2	SFTD Measurement delay	262

9.3.8.3	SFTD Measurement reporting delay	263

9.3.9	Inter frequency measurements without measurement gaps	263

9.3.9.1	Inter frequency Cell identification	263

9.3.9.2	Measurement period	265

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	265

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	265

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	266

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	266

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	267

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	267

9.3A.1	Introduction	267

9.3A.2	Requirements applicability	267

9.3A.3	Number of cells and number of SSB	267

9.3A.3.1	Requirements	267

9.3A.4	Inter-frequency cell identification	268

9.3A.5	Inter-frequency measurements	269

9.3A.6	NR Inter-frequency measurements reporting requirements	269

9.3A.6.1	Periodic Reporting	269

9.3A.6.2	Event-triggered Periodic Reporting	269

9.3A.6.3	Event-triggered Reporting	269

9.3A.8	Inter-frequency RSSI measurements	270

9.3A.9	Inter-frequency channel occupancy measurements	270

9.4	Inter-RAT measurements	271

9.4.1	Introduction	271

9.4.2	NR − E-UTRAN FDD measurements	272

9.4.2.1	Introduction	272

9.4.2.2	Requirements when no DRX is used	273

9.4.2.3	Requirements when DRX is used	273

9.4.2.4	Measurement reporting requirements	274

9.4.2.4.1	Periodic Reporting	274

9.4.2.4.2	Event-Triggered Periodic Reporting	274

9.4.2.4.3	Event-Triggered Reporting	274

9.4.3	NR − E-UTRAN TDD measurements	275

9.4.3.1	Introduction	275

9.4.3.2	Requirements when no DRX is used	275

9.4.3.3	Requirements when DRX is used	276

9.4.3.4	Measurement reporting requirements	277

9.4.3.4.1	Periodic Reporting	277

9.4.3.4.2	Event-Triggered Periodic Reporting	277

9.4.3.4.3	Event-Triggered Reporting	278

9.4.4	Inter-RAT RSTD measurements	278

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	278

9.4.4.1.1	Introduction	278

9.4.4.1.2	Requirements	279

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	282

9.4.4.2.1	Introduction	282

9.4.4.2.2	Requirements	282

9.4.5	Inter-RAT E-CID measurements	286

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	286

9.4.5.1.1	Introduction	286

9.4.5.1.2	Requirements	286

9.4.5.1.3	Measurement Reporting Delay	286

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	286

9.4.5.2.1	Introduction	286

9.4.5.2.2	Requirements	286

9.4.5.2.3	Measurement Reporting Delay	286

9.4.6	NR − UTRAN FDD measurements	286

9.4.6.1	Introduction	286

9.4.6.2	Requirements when no DRX is used	287

9.4.6.3	Requirements when DRX is used	288

9.4.7	NR – E-UTRAN measurements with autonomous gaps	290

9.4.7.1	Reporting delay due to E-UTRAN CGI reading with autonomous gaps	290

9.5	L1-RSRP measurements for Reporting	290

9.5.1	Introduction	290

9.5.2	Requirements applicability	290

9.5.3	Measurement Reporting Requirements	291

9.5.3.1	Periodic Reporting	291

9.5.3.2	Semi-Persistent Reporting	291

9.5.3.3	Aperiodic Reporting	291

9.5.4	L1-RSRP measurement requirements	292

9.5.4.1	SSB based L1-RSRP Reporting	292

9.5.4.2	CSI-RS based L1-RSRP Reporting	293

9.5.4A	L1-RSRP measurement requirements (with CCA serving cell)	296

9.5.4A.1	SSB based L1-RSRP Reporting	296

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	297

9.5.5.1	Measurement restriction for SSB based L1-RSRP	297

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	297

9.5.6	Scheduling availability of UE during L1-RSRP measurement	298

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	298

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	298

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	298

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	299

9.6	NE-DC: Measurements	299

9.6.1	Introduction	299

9.6.2	SFTD Measurements	299

9.6.2.1	Introduction	299

9.6.2.2	SFTD Measurement requirements	300

9.7	Cross Link Interference measurements	300

9.7.1	Introduction	300

9.7.2	SRS-RSRP measurements	301

9.7.2.1	Introduction	301

9.7.2.2	Requirements applicability	301

9.7.2.3 	Measurement Reporting Requirements	301

9.7.2.3.1	Periodic Reporting	301

9.7.2.3.2	Event-triggered Periodic Reporting	301

9.7.2.3.3	Event Triggered Reporting	301

9.7.2.4	Measurement capability	302

9.7.2.5	SRS-RSRP measurement period	302

9.7.3	CLI-RSSI measurements	302

9.7.3.1	Introduction	302

9.7.3.2	Requirements applicability	302

9.7.3.3	Measurement Reporting Requirements	302

9.7.3.3.1	Periodic Reporting	302

9.7.3.3.2	Event-triggered Periodic Reporting	303

9.7.3.3.3	Event Triggered Reporting	303

9.7.3.4	Measurement capability	303

9.7.3.5	CLI-RSSI measurement period	303

9.7.4	Scheduling availability of UE during CLI measurements	303

9.7.4.1	Scheduling availability of UE performing measurement on FR1	303

9.7.4.2	Scheduling availability of UE performing measurement on FR2	304

9.8	L1-SINR measurements for Reporting	304

9.8.1	Introduction	304

9.8.2	Requirements applicability	305

9.8.3	Measurement Reporting Requirements	305

9.8.3.1	Periodic Reporting	305

9.8.3.2	Semi-Persistent Reporting	305

9.8.4	L1-SINR measurement requirements	306

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	306

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	309

9.8.5	Measurement restriction for L1-SINR measurement	311

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	311

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	311

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	312

9.8.6	Scheduling availability of UE during L1-SINR measurement	313

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	313

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	313

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	314

9.9	 NR measurements for positioning	314

9.9.1	Introduction	314

9.9.2	RSTD measurements	314

9.9.2.1	Introduction	314

9.9.2.2	Requirements Applicability	314

9.9.2.3	Measurement Capability	314

9.9.2.4	Measurement Reporting Requirements	314

9.9.2.4.1	Periodic Reporting	315

9.9.2.4.2	Event-triggered Periodic Reporting	315

9.9.2.4.3	Event Triggered Reporting	315

9.9.2.5	Measurements Period Requirements	315

9.9.2.6	Intra-frequency measurements without measurement gaps	316

9.9.3	PRS-RSRP measurements	317

9.9.3.1	Introduction	317

9.9.3.2	Requirements applicability	317

9.9.3.3	Measurement Capability	317

9.9.3.4	Measurement Reporting Requirements	318

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy as specified in the clauses 10.1.24.9.9.3.5	Measurement Period Requirements	318

9.9.4	UE Rx-Tx time difference measurements	319

9.9.4.1 Introduction	319

9.9.4.2 Requirements Applicability	319

9.9.4.3 Measurement Capability	319

9.9.4.4 Measurement Reporting Requirements	319

9.9.4.5 Measurement Period Requirements	319

Editors Note: The applicability of requirements if the time duration of a DL PRS resource exceeds the UE capability N.9.9.5	NR E-CID measurements	321

9.9.5.1	Introduction	321

9.9.5.2	Measurement Requirements	321

9.9.5.2.1	Intra-frequency Measurement Requirements	321

9.9.5.2.2	Inter-frequency Measurement Requirements	321

9.9.5.2.3	Measurement Reporting Delay	321

9.10	CSI-RS based L3 measurements	322

9.10.1	Introduction	322

9.10.2	CSI-RS based intra-frequency measurements	322

9.10.2.1	Introduction	322

9.10.2.2	Requirements applicability	322

9.10.2.3	Number of cells and number of CSI-RS	323

9.10.3	CSI-RS based Inter-frequency measurements	324

9.10.3.1	Introduction	324

9.10.3.2	Requirements applicability	324

9.10.3.3	Number of cells and number of CSI-RS resources	325

9.10.3.3.1	Requirements for FR1	325

9.10.3.3.2	Requirements for FR2	325

9.10.3.4	Measurements reporting requirements	325

9.10.3.4.1	Periodic Reporting	325

9.10.3.4.2	Event-triggered Periodic Reporting	325

9.10.3.4.3	Event-triggered Reporting	325

9.10.3.5	Inter frequency measurements with measurement gaps	325

9.11	NR measurements with autonomous gaps	326

9.11.1	Introduction	326

9.11.2	CGI identification of an NR cell with autonomous gaps	327

9.11.3	CGI reporting delay	327

10	Measurement Performance requirements	328

10.1	NR measurements	328

10.1.1	Introduction	328

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	328

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	328

10.1.2.1.1	Absolute SS-RSRP Accuracy	328

10.1.2.1.2	Relative SS-RSRP Accuracy	329

10.1.2.2	Void	330

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	330

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	330

10.1.3.1.1	Absolute SS-RSRP Accuracy	330

10.1.3.1.2	Relative SS-RSRP Accuracy	331

10.1.3.2	Void	332

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	332

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	332

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	332

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	332

10.1.4.2	Void	333

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	333

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	333

10.1.5.1.1	Absolute SS-RSRP Accuracy	333

10.1.5.1.2	Relative SS-RSRP Accuracy	334

10.1.5.2	Void	335

10.1.6 RSRP Measurement Report Mapping	335

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	337

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	337

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	337

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	338

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	338

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	338

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	339

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	339

10.1.9.1.1	Aboslute Accuracy of SS-RSRQ in FR1	339

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	340

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	341

10.1.11	RSRQ report mapping	343

10.1.12	Intra-frequency SINR accuracy requirements for FR1	343

10.1.13	Intra-frequency SINR accuracy requirements for FR2	344

10.1.14	Inter-frequency SINR accuracy requirements for FR1	345

10.1.15	Inter-frequency SINR accuracy requirements for FR2	347

10.1.16	SINR report mapping	349

10.1.17	Power Headroom	350

10.1.18	PCMAX,c,f	351

10.1.19	L1-RSRP accuracy requirements for FR1	351

10.1.20	L1-RSRP accuracy requirements for FR2	355

10.1.21	SFTD accuracy requirements	358

10.1.22	CLI measurement accuracy requirements	362

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	366

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	366

10.1.23	RSTD Measurements	368

10.1.23.1	Introduction	368

10.1.23.2	Measurement Accuracy Requirements	369

10.1.23.3	Report mapping	369

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	369

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	371

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	373

10.1.24	PRS-RSRP Measurements	375

10.1.24.1	Introduction	375

10.1.24.2	Measurement Accuracy Requirements	375

10.1.24.3	Report mapping	375

10.1.25	UE Rx-Tx Time Difference Measurements	375

10.1.25.1	Introduction	375

10.1.25.2	Measurement Accuracy Requirements	375

10.1.25.3	Report mapping	375

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	375

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	377

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	379

10.1.26	FR2 P-MPR report	381

10.1.26.1	Report mapping	381

10.2	E-UTRAN measurements	381

10.2.1	Introduction	381

10.2.2	E-UTRAN RSRP measurements	381

10.2.3	E-UTRAN RSRQ measurements	382

10.2.4	E-UTRAN RSTD measurements	382

10.2.5	E-UTRAN RS-SINR measurements	382

10.3	UTRAN FDD Measurements	382

10.3.1	UTRAN FDD CPICH RSCP	383

10.3.2	UTRAN FDD CPICH Ec/No	383

11	Void	384

12	V2X Requirements	384

12.1	Introduction	384

12.2	UE Transmit Timing	384

12.2.1	Introduction	384

12.2.2	GNSS as synchronization reference source	384

12.2.3	NR Cell as synchronization reference source	385

12.2.4	E-URTAN Cell as synchronization reference source	385

12.2.5	SyncRef UE as synchronization reference source	385

12.3	Initiation/Cease of SLSS Transmissions	386

12.3.1	Introduction	386

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	386

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	387

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	388

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	388

12.4	Selection / Reselection of V2X Synchronization Reference Source	388

12.5.1 Introduction	389

12.5.2 SL-RSRP measurements	389

12.6	Congestion Control measurements	390

12.7	Interruption	390

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	390

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	390

12.8	Reliability of GNSS signal	392

12.9	Scheduling availability	392

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	392

13	Measurement Performance Requirements for NR gNB	393

13.1	UL-RTOA	393

13.1.1	Report mapping	393

13.2	gNB Rx-Tx time difference	395

13.2.1	Report mapping	395

13.3	UL SRS RSRP measurement	397

13.3.1	Report mapping	397

13.4	AoA/ZoA	398

13.4.1	Report mapping	398

Annex A (normative):	Test Cases	245

A.1	Purpose of annex	245

A.2	Requirement classification for statistical testing	245

A.2.1	Types of requirements in TS 38.133	245

A.2.1.1	Time and delay requirements on UE higher layer actions	245

A.2.1.2	Measurements of power levels, relative powers and time	245

A.2.1.3	Implementation requirements	246

A.2.1.4	Physical layer timing requirements	246

A.3	RRM test configurations	247

A.3.1	Reference measurement channels	247

A.3.1.1	PDSCH	247

A.3.1.1.1	FDD	247

A.3.1.1.2	TDD	248

A.3.1.2	CORESET for RMSI scheduling	251

A.3.1.2.1	FDD	251

A.3.1.2.2	TDD	252

A.3.1.3	CORESET for RMC scheduling	255

A.3.1.3.1	FDD	255

A.3.1.3.2	TDD	256

A.3.1.4	TDD UL/DL configuration	258

A.3.2	OFDMA channel noise generator (OCNG)	259

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	259

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	260

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	260

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as PDSCH RMC	261

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	261

A.3.2.2	Void	262

A.3.3	Reference DRX configurations	262

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	262

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	262

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	262

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	263

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	263

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	263

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	264

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	264

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	264

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms	265

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	265

A.3.4	Test Cases with Different Channel Bandwidths	265

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	265

A.3.4.1.1	Introduction	265

A.3.4.1.2	Principle of testing	265

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	266

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	266

A.3.5.1.1	Introduction	266

A.3.5.1.2	Principle of Testing	266

A.3.6	Antenna configurations	266

A.3.6.1	Antenna configurations for FR1	266

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	266

A.3.6.1.1.1	Introduction	266

A.3.6.1.1.2	Principle of testing	266

A.3.6.2	Antenna configurations for FR2	269

A.3.7	EN-DC test setup	269

A.3.7.1	Introduction	269

A.3.7.2	E-UTRAN Serving Cell Parameters	269

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	269

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	270

A.3.7A	NR FR1-FR2 test setup	271

A.3.8	PRACH configurations	272

A.3.8.1	Introduction	272

A.3.8.2	PRACH configurations in FR1	272

A.3.8.2.1	FR1 PRACH configuration 1	272

A.3.8.2.2	FR1 PRACH configuration 2	272

A.3.8.2.3	FR1 PRACH configuration 3	273

A.3.8.2.4	FR1 PRACH configuration 4	274

A.3.8.3	PRACH configurations in FR2	275

A.3.8.3.1	FR2 PRACH configuration 1	275

A.3.8.3.2	FR2 PRACH configuration 2	276

A.3.8.3.3	FR2 PRACH configuration 3	277

A.3.8.3.4	FR2 PRACH configuration 4	278

A.3.9	BWP configurations	279

A.3.9.1	Introduction	279

A.3.9.2	Downlink BWP configurations	280

A.3.9.2.1	Initial BWP	280

A.3.9.2.2	Dedicated BWP	280

A.3.9.3	Uplink BWP configurations	280

A.3.9.3.1	Initial BWP	280

A.3.9.3.2	Dedicated BWP	281

A.3.10	SSB Configurations	281

A.3.10.1	SSB Configurations for FR1	281

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	281

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	283

A.3.11	SMTC Configurations	288

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	288

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	288

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	288

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	289

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	289

A.3.12	Test Cases with Different CC Configurations	289

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	289

A.3.12.1.1	Introduction	289

A.3.12.1.2	Principle of testing	289

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	289

A.3.12.2.1	Introduction	289

A.3.12.2.2	Principle of testing	289

A.3.13	Test Cases in SA and EN-DC Operations	290

A.3.13.1	Introduction	290

A.3.13.2	Principle of Testing	290

A.3.14	CSI-RS configurations	290

A.3.14.1	FDD	290

A.3.14.2	TDD	291

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	294

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	294

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	295

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	295

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	295

A.3.15.3	Setup 3: 2 AoAs	295

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	295

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	295

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	296

A.3.16	TCI State Configuration	296

A.3.16.1	Introduction	296

A.3.16.2	TCI states	296

A.3.17	Configurations of CSI-RS for tracking	297

A.3.17.1	Configuration of CSI-RS for tracking for FR1	297

A.3.17.1.1	FDD	297

A.3.17.1.2	TDD	298

A.3.17.2	Configuration of CSI-RS for tracking for FR2	299

A.3.17.2.1	TDD	299

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	300

A.3.18.1	Introduction	300

A.3.18.2	PRACH Power Measurement	300

A.4	EN-DC tests with all NR cells in FR1	300

A.4.1	Void	300

A.4.2	Void	300

A.4.3	RRC_CONNECTED state mobility	300

A.4.3.1	Void	300

A.4.3.2	RRC Connection Mobility Control	300

A.4.3.2.1	Void	300

A.4.3.2.2	Random Access	300

A.4.3.2.2.1	Contention based random access test in FR1 for PSCell in EN-DC	300

A.4.3.2.2.2	Non-contention based random access test in FR1 for PSCell in EN-DC	303

A.4.3.2.3	Void	307

A.4.4	Timing	307

A.4.4.1	UE transmit timing	307

A.4.4.1.1	NR UE Transmit Timing Test for FR1	307

A.4.4.1.1.1	Test Purpose and environment	307

A.4.4.1.1.2	Test requirements	312

A.4.4.2	UE timer accuracy	312

A.4.4.3	Timing advance	312

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	312

A.4.4.3.1.1	Test Purpose and Environment	312

A.4.4.3.1.2	Test Parameters	312

A.4.4.3.1.3	Test Requirements	316

A.4.5	Signaling characteristics	316

A.4.5.1	Radio link Monitoring	316

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	317

A.4.5.1.1.1	Test Purpose and Environment	317

A.4.5.1.1.2	Test Requirements	321

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	321

A.4.5.1.2.1	Test Purpose and Environment	321

A.4.5.1.2.2	Test Requirements	327

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	327

A.4.5.1.3.1	Test Purpose and Environment	327

A.4.5.1.3.2	Test Requirements	333

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	333

A.4.5.1.4.1	Test Purpose and Environment	333

A.4.5.1.4.2	Test Requirements	339

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	339

A.4.5.1.5.1	Test Purpose and Environment	339

A.4.5.1.5.2	Test Requirements	344

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	344

A.4.5.1.6.1	Test Purpose and Environment	344

A.4.5.1.6.2	Test Requirements	350

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	350

A.4.5.1.7.1	Test Purpose and Environment	350

A.4.5.1.7.2	Test Requirements	355

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	355

A.4.5.1.8.1	Test Purpose and Environment	355

A.4.5.1.8.2	Test Requirements	361

A.4.5.2	Interruption	361

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	361

A.4.5.2.1.1	Test Purpose and Environment	361

A.4.5.2.1.2	Test Requirements	365

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	365

A.4.5.2.2.1	Test Purpose and Environment	365

A.4.5.2.2.2	Test Requirements	369

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	369

A.4.5.2.3.1	Test Purpose and Environment	369

A.4.5.2.3.2	Test Requirements	373

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	373

A.4.5.2.4.1	Test Purpose and Environment	373

A.4.5.2.4.2	Test Requirements	377

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	377

A.4.5.2.5.1	Test Purpose and Environment	377

A.4.5.2.5.2	Test Requirements	381

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	381

A.4.5.2.6.1	Test Purpose and Environment	381

A.4.5.2.6.2	Test Requirements	385

A.4.5.2.7	Void	385

A.4.5.3	SCell Activation and Deactivation Delay	385

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	385

A.4.5.3.1.1	Test Purpose and Environment	385

A.4.5.3.1.2	Test Requirements	390

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	390

A.4.5.3.2.1	Test Purpose and Environment	390

A.4.5.3.2.2	Test Requirements	391

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	391

A.4.5.3.3.1	Test Purpose and Environment	391

A.4.5.3.3.2	Test Requirements	391

A.4.5.4	UE UL carrier RRC reconfiguration Delay	392

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	392

A.4.5.4.1.1 Test Purpose and Environment	392

A.4.5.4.1.2	Test Requirements	400

A.4.5.5	Beam Failure Detection and Link recovery procedures	400

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	400

A.4.5.5.1.1	Test Purpose and Environment	400

A.4.5.5.1.2	Test Requirements	406

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	406

A.4.5.5.2.1	Test Purpose and Environment	406

A.4.5.5.2.2	Test Requirements	412

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	412

A.4.5.5.3.1	Test Purpose and Environment	412

A.4.5.5.3.2	Test Requirements	418

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	418

A.4.5.5.4.1	Test Purpose and Environment	418

A.4.5.5.4.2	Test Requirements	424

A.4.5.6	Active BWP switch	424

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	424

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	424

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	430

A.4.5.6.2	RRC-based Active BWP Switch	434

A.4.5.7	PSCell addition and release delay	439

A.4.5.7.1	Addition and Release Delay of known NR PSCell	439

A.4.5.7.1.1	Test purpose and environment	439

A.4.5.7.1.2	Test Requirements	443

A.4.6	Measurement procedure	443

A.4.6.1	Intra-frequency Measurements	443

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	443

A.4.6.1.1.1	Test purpose and Environment	443

A.4.6.1.1.2	Test parameters	444

A.4.6.1.1.3	Test Requirements	447

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	447

A.4.6.1.2.1	Test purpose and Environment	447

A.4.6.1.2.2	Test parameters	447

A.4.6.1.2.2	Test Requirements	451

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	451

A.4.6.1.3.1	Test purpose and Environment	451

A.4.6.1.3.2	Test parameters	451

A.4.6.1.3.3	Test Requirements	455

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	455

A.4.6.1.4.1	Test purpose and Environment	455

A.4.6.1.4.2	Test parameters	455

A.4.6.1.4.3	Test Requirements	459

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	459

A.4.6.1.5.1	Test purpose and Environment	459

A.4.6.1.5.2	Test parameters	459

A.4.6.1.5.3	Test Requirements	461

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	462

A.4.6.1.6.1	Test purpose and Environment	462

A.4.6.1.6.2	Test parameters	462

A.4.6.1.6.3	Test Requirements	464

A.4.6.2	Inter-frequency Measurements	465

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	465

A.4.6.2.1.1	Test Purpose and Environment	465

A.4.6.2.1.2	Test Requirements	469

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	469

A.4.6.2.2.1	Test Purpose and Environment	469

A.4.6.2.2.2	Test Requirements	473

A.4.6.2.3	Void	474

A.4.6.2.4	Void	474

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	474

A.4.6.2.5.1	Test Purpose and Environment	474

A.4.6.2.5.2	Test Requirements	478

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	478

A.4.6.2.6.1	Test Purpose and Environment	478

A.4.6.2.6.2	Test Requirements	483

A.4.6.2.7	Void	484

A.4.6.2.8	Void	484

A.4.6.3	Void	484

A.4.6.4	L1-RSRP measurement for beam reporting	484

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	484

A.4.6.4.1.1	Test Purpose and Environment	484

A.4.6.4.1.2	Test parameters	484

A.4.6.4.1.3	Test Requirements	487

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	487

A.4.6.4.2.1	Test Purpose and Environment	487

A.4.6.4.2.2	Test parameters	488

A.4.6.4.2.3	Test Requirements	491

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	491

A.4.6.4.3.1	Test Purpose and Environment	491

A.4.6.4.3.2	Test parameters	492

A.4.6.4.3.3	Test Requirements	495

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	495

A.4.6.4.4.1	Test Purpose and Environment	495

A.4.6.4.4.2	Test parameters	496

A.4.6.4.4.3	Test Requirements	498

A.4.6.5	CLI measurements	498

A.4.6.5.1	SRS-RSRP measurement with non-DRX	498

A.4.6.5.1.1	Test Purpose and Environment	498

A.4.6.5.1.2	Test Parameters	499

A.4.6.5.1.3	Test Requirements	502

A.4.6.5.2	CLI-RSSI measurement with non-DRX	502

A.4.6.5.2.1	Test Purpose and Environment	502

A.4.6.5.2.2	Test Parameters	503

A.4.6.5.2.3	Test Requirements	504

A.4.7	Measurement Performance requirements	505

A.4.7.1	SS-RSRP	505

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	505

A.4.7.1.1.1	Test Purpose and Environment	505

A.4.7.1.1.2	Test parameters	505

A.4.7.1.1.3	Test Requirements	509

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	509

A.4.7.1.2.1	Test Purpose and Environment	509

A.4.7.1.2.2	Test parameters	510

A.4.7.1.2.3	Test Requirements	514

A.4.7.1.3	Void	514

A.4.7.2	SS-RSRQ	514

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	514

A.4.7.2.1.1	Test Purpose and Environment	514

A.4.7.2.1.2	Test Parameters	515

A.4.7.2.1.3	Test Requirements	520

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	520

A.4.7.2.2.1	Test Purpose and Environment	520

A.4.7.2.2.2	Test Parameters	520

A.4.7.2.2.3	Test Requirements	525

A.4.7.3	SS-SINR	525

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	525

A.4.7.3.1.1	Test Purpose and Environment	525

A.4.7.3.1.2	Test Parameters	525

A.4.7.3.1.3	Test Requirements	529

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	529

A.4.7.3.2.1	Test Purpose and Environment	529

A.4.7.3.2.2	Test Parameters	529

A.4.7.3.2.3	Test Requirements	534

A.4.7.4	L1-RSRP measurement for beam reporting	535

A.4.7.4.1	SSB based L1-RSRP measurement	535

A.4.7.4.1.1	Test Purpose and Environment	535

A.4.7.4.1.2	Test parameters	535

A.4.7.4.1.3	Test Requirements	538

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	538

A.4.7.4.2.1	Test Purpose and Environment	538

A.4.7.4.2.2	Test parameters	539

A.4.7.4.2.3	Test Requirements	542

A.4.7.5	SFTD accuracy	542

A.4.7.5.1	SFTD accuracy	542

A.4.7.5.1.1	Test Purpose and Environment	542

A.4.7.5.1.2	Test Parameters	542

A.4.7.5.1.3	Test Requirements	547

A.4.7.5.2	Void	547

A.4.7.5.3	Void	547

A.4.7.6	CLI measurements	547

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	547

A.4.7.6.1.1	Test Purpose and Environment	547

A.4.7.6.1.2	Test parameters	547

A.4.7.6.1.3	Test Requirements	553

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	553

A.4.7.6.2.1	Test Purpose and Environment	553

A.4.7.6.2.2	Test parameters	554

A.4.7.6.2.3	Test Requirements	557

A.4.8	Void	557

A.5	EN-DC tests with one or more NR cells in FR2	558

A.5.1	Void	558

A.5.2	Void	558

A.5.3	RRC_CONNECTED state mobility	558

A.5.3.1	Void	558

A.5.3.2	RRC Connection Mobility Control	558

A.5.3.2.1	Void	558

A.5.3.2.2	Random Access	558

A.5.3.2.2.1	Contention based random access test in FR2 for PSCell/SCell in EN-DC	558

A.5.3.2.2.2	Non-contention based random access test in FR2 for PSCell/SCell in EN-DC	562

A.5.3.2.3	Void	567

A.5.4	Timing	567

A.5.4.1	UE transmit timing	567

A.5.4.1.1	NR UE Transmit Timing Test for FR2	567

A.5.4.1.1.1	Test Purpose and environment	567

A.5.4.1.1.2	Test requirements	570

A.5.4.2	UE timer accuracy	571

A.5.4.3	Timing advance	571

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	571

A.5.4.3.1.1 Test Purpose and Environment	571

A.5.4.3.1.2 Test Parameters	571

A.5.4.3.1.3	Test Requirements	574

A.5.5	Signaling characteristics	574

A.5.5.1	Radio link Monitoring	574

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	574

A.5.5.1.1.1	Test Purpose and Environment	574

A.5.5.1.1.2	Test Requirements	577

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	578

A.5.5.1.2.1	Test Purpose and Environment	578

A.5.5.1.2.2	Test Requirements	581

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	582

A.5.5.1.3.1	Test Purpose and Environment	582

A.5.5.1.3.2	Test Requirements	585

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	585

A.5.5.1.4.1	Test Purpose and Environment	585

A.5.5.1.4.2	Test Requirements	590

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	590

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	596

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	601

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	606

A.5.5.1.8.2	Test Requirements	612

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	612

A.5.5.1.9.1	Test Purpose and Environment	612

A.5.5.1.9.2	Test Requirements	614

A.5.5.2	Interruption	614

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	614

A.5.5.2.1.1	Test Purpose and Environment	614

A.5.5.2.1.2	Test Requirements	617

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	617

A.5.5.2.2.1	Test Purpose and Environment	617

A.5.5.2.2.2	Test Requirements	620

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	620

A.5.5.2.3.1	Test Purpose and Environment	620

A.5.5.2.3.2	Test Requirements	624

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	624

A.5.5.2.4.1	Test Purpose and Environment	624

A.5.5.2.4.2	Test Requirements	627

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	628

A.5.5.2.5.1	Test Purpose and Environment	628

A.5.5.2.5.2	Test Requirements	631

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	632

A.5.5.2.6.1	Test Purpose and Environment	632

A.5.5.2.6.2	Test Requirements	634

A.5.5.3	SCell Activation and Deactivation Delay	635

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	635

A.5.5.3.1.1	Test Purpose and Environment	635

A.5.5.3.1.2	Test Requirements	637

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	637

A.5.5.3.2.1	Test Purpose and Environment	637

A.5.5.3.2.2	Test Requirements	640

A.5.5.3.3	Void	640

A.5.5.3.4	Void	640

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	640

A.5.5.3.5.1	Test Purpose and Environment	640

A.5.5.3.5.2	Test Requirements	643

A.5.5.4	Void	643

A.5.5.5	Beam Failure Detection and Link recovery procedures	643

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	643

A.5.5.5.1.1	Test Purpose and Environment	643

A.5.5.5.1.2	Test Requirements	649

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	649

A.5.5.5.2.1	Test Purpose and Environment	649

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	655

A.5.5.5.3.1	Test Purpose and Environment	655

A.5.5.5.3.2	Test Requirements	661

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	661

A.5.5.5.4.1	Test Purpose and Environment	661

A.5.5.5.4.2	Test Requirements	667

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	667

A.5.5.5.5.1	Test Purpose and Environment	667

A.5.5.5.5.2	Test Requirements	673

A.5.5.6	Active BWP switch	673

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	673

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	673

A.5.5.6.1.1.1	Test Purpose and Environment	673

A.5.5.6.1.1.2	Test Requirements	677

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC	678

A.5.5.6.2	RRC-based Active BWP Switch	682

A.5.5.7	PSCell addition and release delay	686

A.5.5.7.1	Addition and Release Delay of NR PSCell	686

A.5.5.7.1.1	Test purpose and environment	686

A.5.5.7.1.2	Test Requirements	689

A.5.5.8	Active TCI state switch delay	690

A.5.5.8.1	MAC-CE based active TCI state switch	690

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	690

A.5.5.8.1.1.1	Test Purpose and Environment	690

A.5.5.8.1.1.2	Test Requirements	692

A.5.5.8.2	RRC based active TCI state switch	692

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	692

A.5.5.8.2.1.1	Test Purpose and Environment	692

A.5.5.8.2.1.2	Test Requirements	695

A.5.6	Measurement procedure	695

A.5.6.1	Intra-frequency Measurements	695

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	695

A.5.6.1.1.1	Test purpose and Environment	695

A.5.6.1.1.2	Test Requirements	698

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	698

A.5.6.1.2.1	Test purpose and Environment	698

A.5.6.1.2.2	Test Requirements	701

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	701

A.5.6.1.3.1	Test purpose and Environment	701

A.5.6.1.3.2	Test Requirements	704

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	704

A.5.6.1.4.1	Test purpose and Environment	704

A.5.6.1.4.2	Test Requirements	708

A.5.6.2	Inter-frequency Measurements	709

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	709

A.5.6.2.1.1	Test Purpose and Environment	709

A.5.6.2.1.2	Test Requirements	712

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	712

A.5.6.2.2.1	Test Purpose and Environment	712

A.5.6.2.2.2	Test Requirements	716

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	716

A.5.6.2.3.1	Test Purpose and Environment	716

A.5.6.2.3.2	Test Requirements	720

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	720

A.5.6.2.4.1	Test Purpose and Environment	720

A.5.6.2.4.2	Test Requirements	723

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	723

A.5.6.2.5.1	Test Purpose and Environment	723

A.5.6.2.5.2	Test Requirements	728

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	729

A.5.6.2.6.1	Test Purpose and Environment	729

A.5.6.2.6.2	Test Requirements	732

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	733

A.5.6.2.7.1	Test Purpose and Environment	733

A.5.6.2.7.2	Test Requirements	736

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	736

A.5.6.2.8.1	Test Purpose and Environment	736

A.5.6.2.8.2	Test Requirements	740

A.5.6.3	L1-RSRP measurement for beam reporting	741

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	741

A.5.6.3.1.1	Test Purpose and Environment	741

A.5.6.3.1.2	Test parameters	741

A.5.6.3.1.3	Test Requirements	743

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	743

A.5.6.3.2.1	Test Purpose and Environment	743

A.5.6.3.2.2	Test parameters	744

A.5.6.3.2.3	Test Requirements	746

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	747

A.5.6.3.3.1	Test Purpose and Environment	747

A.5.6.3.3.2	Test parameters	747

A.5.6.3.3.3	Test Requirements	749

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	750

A.5.6.3.4.1	Test Purpose and Environment	750

A.5.6.3.4.2	Test parameters	750

A.5.6.3.4.3	Test Requirements	752

A.5.6.4	CLI measurements	753

A.5.6.4.1	SRS-RSRP measurement with DRX	753

A.5.6.4.1.1	Test Purpose and Environment	753

A.5.6.4.1.2	Test Parameters	753

A.5.6.4.1.3	Test Requirements	755

A.5.6.4.2	CLI-RSSI measurement with DRX	755

A.5.6.4.2.1	Test Purpose and Environment	755

A.5.6.4.2.2	Test Parameters	756

A.5.6.4.2.3	Test Requirements	758

A.5.7	Measurement Performance requirements	758

A.5.7.1	SS-RSRP	758

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	758

A.5.7.1.1.1	Test Purpose and Environment	758

A.5.7.1.1.2	Test parameters	758

A.5.7.1.1.3	Test Requirements	760

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	761

A.5.7.1.2.1	Test Purpose and Environment	761

A.5.7.1.2.2	Test parameters	761

A.5.7.1.2.3	Test Requirements	764

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	764

A.5.7.1.3.1	Test Purpose and Environment	764

A.5.7.1.3.2	Test parameters	765

A.5.7.1.3.3	Test Requirements	768

A.5.7.2	SS-RSRQ	768

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	768

A.5.7.2.1.1	Test Purpose and Environment	768

A.5.7.2.1.2	Test Parameters	769

A.5.7.2.1.3	Test Requirements	773

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	773

A.5.7.2.2.1	Test Purpose and Environment	773

A.5.7.2.2.2	Test Parameters	773

A.5.7.2.2.3	Test Requirements	776

A.5.7.3	SS-SINR	776

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	776

A.5.7.3.1.1	Test Purpose and Environment	776

A.5.7.3.1.2	Test Parameters	776

A.5.7.3.1.3	Test Requirements	779

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	779

A.5.7.3.2.1	Test Purpose and Environment	779

A.5.7.3.2.2	Test Parameters	779

A.5.7.3.2.3	Test Requirements	781

A.5.7.4	L1-RSRP measurement for beam reporting	782

A.5.7.4.1	SSB based L1-RSRP measurement	782

A.5.7.4.1.1	Test Purpose and Environment	782

A.5.7.4.1.2	Test parameters	782

A.5.7.4.1.3	Test Requirements	784

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	785

A.5.7.4.2.1	Test Purpose and Environment	785

A.5.7.4.2.2	Test parameters	785

A.5.7.4.2.3	Test Requirements	787

A.5.7.5	CLI measurements	788

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	788

A.5.7.5.1.1	Test Purpose and Environment	788

A.5.7.5.1.2	Test parameters	788

A.5.7.5.1.3	Test Requirements	791

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	792

A.5.7.5.2.1	Test Purpose and Environment	792

A.5.7.5.2.2	Test parameters	792

A.5.7.5.2.3	Test Requirements	794

A.5.8	Void	795

A.6	NR standalone tests with all NR cells in FR1	665

A.6.1	SA: RRC_IDLE state mobility	665

A.6.1.1	Cell re-selection to NR	665

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	665

A.6.1.1.1.1	Test Purpose and Environment	665

A.6.1.1.1.2	Test Parameters	665

A.6.1.1.1.3	Test Requirements	669

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	669

A.6.1.1.2.1	Test Purpose and Environment	669

A.6.1.1.2.2	Test Parameters	669

A.6.1.1.2.3	Test Requirements	673

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	673

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	673

A.6.1.2.1.1	Test Purpose and Environment	673

A.6.1.2.1.2	Test Parameters	673

A.6.1.2.1.3	Test Requirements	676

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	677

A.6.1.2.2.1	Test Purpose and Environment	677

A.6.1.2.2.2	Test Parameters	677

A.6.1.2.2.3	Test Requirements	680

A.6.2	SA: RRC_INACTIVE state mobility	681

A.6.3	RRC_CONNECTED state mobility	681

A.6.3.1	Handover	681

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	681

A.6.3.1.1.1	Test Purpose and Environment	681

A.6.3.1.1.2	Test Parameters	681

A.6.3.1.1.3 Test Requirements	685

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	685

A.6.3.1.2.1	Test Purpose and Environment	685

A.6.3.1.2.2	Test Parameters	685

A.6.3.1.2.3	Test Requirements	689

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	689

A.6.3.1.3.1	Test Purpose and Environment	689

A.6.3.1.3.2	Test Parameters	689

A.6.3.1.3.3 Test Requirements	692

A.6.3.1.4	 SA NR - E-UTRAN handover	692

A.6.3.1.4.1	Test Purpose and Environment	692

A.6.3.1.4.2	Test Requirements	697

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	697

A.6.3.1.5.1	Test Purpose and Environment	697

A.6.3.1.5.2	Test Requirements	703

A.6.3.1.6	 SA NR - UTRAN FDD handover	703

A.6.3.1.6.1	Test Purpose and Environment	703

A.6.3.1.6.2	Test Requirements	707

A.6.3.2	RRC Connection Mobility Control	707

A.6.3.2.1	SA: RRC Re-establishment	707

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	707

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	710

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	713

A.6.3.2.2	Random Access	716

A.6.3.2.2.1	Contention based random access test in FR1 for NR standalone	716

A.6.3.2.2.2	Non-Contention based random access test in FR1 for NR standalone	719

A.6.3.2.3	SA: RRC Connection Release with Redirection	723

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	723

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	727

A.6.4	Timing	734

A.6.4.1	UE transmit timing	734

A.6.4.1.1	NR UE Transmit Timing Test for FR1	734

A.6.4.1.1.1	Test Purpose and environment	734

A.6.4.1.1.2	Test requirements	738

A.6.4.2	UE timer accuracy	738

A.6.4.3	Timing advance	738

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	738

A.6.4.3.1.1	Test Purpose and Environment	738

A.6.4.3.1.2	Test Parameters	738

A.6.4.3.1.3	Test Requirements	742

A.6.5	Signalling characteristics	742

A.6.5.1	Radio link Monitoring	742

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	743

A.6.5.1.1.1	Test Purpose and Environment	743

A.6.5.1.1.2	Test Requirements	747

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	747

A.6.5.1.2.1	Test Purpose and Environment	747

A.6.5.1.2.2	Test Requirements	753

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	753

A.6.5.1.3.1	Test Purpose and Environment	753

A.6.5.1.3.2	Test Requirements	757

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	757

A.6.5.1.4.1	Test Purpose and Environment	757

A.6.5.1.4.2	Test Requirements	763

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	763

A.6.5.1.5.1	Test Purpose and Environment	763

A.6.5.1.5.2	Test Requirements	768

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	768

A.6.5.1.6.1	Test Purpose and Environment	768

A.6.5.1.6.2	Test Requirements	773

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	773

A.6.5.1.7.1	Test Purpose and Environment	773

A.6.5.1.7.2	Test Requirements	778

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	778

A.6.5.1.8.1	Test Purpose and Environment	778

A.6.5.1.8.2	Test Requirements	783

A.6.5.2	Interruption	783

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	783

A.6.5.2.1.2	Test Requirements	787

A.6.5.3	SCell Activation and Deactivation Delay	787

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	787

A.6.5.3.1.1	Test Purpose and Environment	787

A.6.5.3.1.2	Test Requirements	792

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	792

A.6.5.3.2.1	Test Purpose and Environment	792

A.6.5.3.2.2	Test Requirements	793

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	793

A.6.5.3.3.1	Test Purpose and Environment	793

A.6.5.3.3.2	Test Requirements	794

A.6.5.4	UE UL carrier RRC reconfiguration Delay	794

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	794

A.6.5.4.1.1	Test Purpose and Environment	794

A.6.5.4.1.2	Test Requirements	803

A.6.5.4.2	Void	803

A.6.5.5	Beam Failure Detection and Link recovery procedures	803

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	803

A.6.5.5.1.1	Test Purpose and Environment	803

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	809

A.6.5.5.2.1	Test Purpose and Environment	809

A.6.5.5.2.2	Test Requirements	816

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	816

A.6.5.5.3.1	Test Purpose and Environment	816

A.6.5.5.3.2	Test Requirements	822

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	822

A.6.5.5.4.1	Test Purpose and Environment	822

A.6.5.5.4.2	Test Requirements	827

A.6.5.6	Active BWP switch	828

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	828

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	828

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	832

A.6.5.6.2	RRC-based Active BWP Switch	837

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	837

A.6.6	Measurement procedure	841

A.6.6.1	Intra-frequency Measurements	841

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	841

A.6.6.1.1.1	Test purpose and Environment	841

A.6.6.1.1.2	Test parameters	841

A.6.6.1.1.3	Test Requirements	844

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	844

A.6.6.1.2.1	Test purpose and Environment	844

A.6.6.1.2.2	Test parameters	844

A.6.6.1.2.3	Test Requirements	846

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	847

A.6.6.1.3.1	Test purpose and Environment	847

A.6.6.1.3.2	Test parameters	847

A.6.6.1.3.3	Test Requirements	850

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	850

A.6.6.1.4.1	Test purpose and Environment	850

A.6.6.1.4.2	Test parameters	850

A.6.6.1.4.3	Test Requirements	853

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	853

A.6.6.1.5.1	Test purpose and Environment	853

A.6.6.1.5.2	Test parameters	853

A.6.6.1.5.3	Test Requirements	855

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	856

A.6.6.1.6.1	Test purpose and Environment	856

A.6.6.1.6.2	Test parameters	856

A.6.6.1.6.3	Test Requirements	857

A.6.6.2	Inter-frequency Measurements	858

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	858

A.6.6.2.1.1	Test Purpose and Environment	858

A.6.6.2.1.2	Test Requirements	862

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	862

A.6.6.2.2.1	Test Purpose and Environment	862

A.6.6.2.2.2	Test Requirements	866

A.6.6.2.3	Void	867

A.6.6.2.4	Void	867

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	867

A.6.6.2.5.1	Test Purpose and Environment	867

A.6.6.2.5.2	Test Requirements	870

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	870

A.6.6.2.6.1	Test Purpose and Environment	870

A.6.6.2.6.2	Test Requirements	874

A.6.6.2.7	Void	875

A.6.6.2.8	Void	875

A.6.6.3	Inter-RAT Measurements	875

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	875

A.6.6.3.1.1	Test Purpose and Environment	875

A.6.6.3.1.2	Test Requirements	881

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	881

A.6.6.3.2.1	Test Purpose and Environment	881

A.6.6.3.2.2	Test Requirements	888

A.6.6.4	L1-RSRP measurement for beam reporting	888

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	888

A.6.6.4.1.1	Test Purpose and Environment	888

A.6.6.4.1.2	Test parameters	888

A.6.6.4.1.3	Test Requirements	891

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	891

A.6.6.4.2.1	Test Purpose and Environment	891

A.6.6.4.2.2	Test parameters	892

A.6.6.4.2.3	Test Requirements	895

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	895

A.6.6.4.3.1	Test Purpose and Environment	895

A.6.6.4.3.2	Test parameters	896

A.6.6.4.3.3	Test Requirements	899

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	899

A.6.6.4.4.1	Test Purpose and Environment	899

A.6.6.4.4.2	Test parameters	900

A.6.6.4.4.3	Test Requirements	902

A.6.6.5	902

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	902

A.6.6.5.1.1	Test Purpose and Environment	902

A.6.6.5.1.2	Test Requirements	906

A.6.6.6	CLI measurements	906

A.6.6.6.1	SRS-RSRP measurement with DRX	906

A.6.6.6.1.1	Test Purpose and Environment	906

A.6.6.6.1.2	Test Parameters	907

A.6.6.6.1.3	Test Requirements	910

A.6.6.6.2	CLI-RSSI measurement with DRX	910

A.6.6.6.2.1	Test Purpose and Environment	910

A.6.6.6.2.2	Test Parameters	910

A.6.6.6.2.3	Test Requirements	912

A.6.7	Measurement Performance requirements	913

A.6.7.1	SS-RSRP	913

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	913

A.6.7.1.1.1	Test Purpose and Environment	913

A.6.7.1.1.2	Test parameters	913

A.6.7.1.1.3	Test Requirements	918

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	918

A.6.7.1.2.1	Test Purpose and Environment	918

A.6.7.1.2.2	Test parameters	918

A.6.7.1.2.3	Test Requirements	922

A.6.7.1.3	Void	922

A.6.7.2	SS-RSRQ	922

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	922

A.6.7.2.1.1	Test Purpose and Environment	922

A.6.7.2.1.2	Test Parameters	922

A.6.7.2.1.3	Test Requirements	928

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	928

A.6.7.2.2.1	Test Purpose and Environment	928

A.6.7.2.2.2	Test Parameters	928

A.6.7.2.2.3	Test Requirements	933

A.6.7.3	SS-SINR	934

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	934

A.6.7.3.1.1	Test Purpose and Environment	934

A.6.7.3.1.2	Test Parameters	934

A.6.7.3.1.3	Test Requirements	938

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	938

A.6.7.3.2.1	Test Purpose and Environment	938

A.6.7.3.2.2	Test Parameters	939

A.6.7.3.2.3	Test Requirements	943

A.6.7.4	L1-RSRP measurement for beam reporting	944

A.6.7.4.1	SSB based L1-RSRP measurement	944

A.6.7.4.1.1	Test Purpose and Environment	944

A.6.7.4.1.2	Test parameters	944

A.6.7.4.1.3	Test Requirements	948

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	948

A.6.7.4.2.1	Test Purpose and Environment	948

A.6.7.4.2.2	Test parameters	948

A.6.7.4.2.3	Test Requirements	953

A.6.7.5	E-UTRAN RSRP	953

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	953

A.6.7.5.1.1	Test Purpose and Environment	953

A.6.7.5.1.2	Test parameters	953

A.6.7.5.1.3	Test Requirements	958

A.6.7.6	E-UTRAN RSRQ	958

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	958

A.6.7.6.1.1	Test Purpose and Environment	958

A.6.7.6.1.2	Test parameters	959

A.6.7.6.1.3	Test Requirements	963

A.6.7.7	E-UTRAN RS-SINR	964

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	964

A.6.7.7.1.1	Test Purpose and Environment	964

A.6.7.7.1.2	Test parameters	964

A.6.7.7.1.3	Test Requirements	969

A.6.7.8	CLI measurements	969

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	969

A.6.7.8.1.1	Test Purpose and Environment	969

A.6.7.8.1.2	Test parameters	969

A.6.7.8.1.3	Test Requirements	973

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	973

A.6.7.8.2.1	Test Purpose and Environment	973

A.6.7.8.2.2	Test parameters	974

A.6.7.8.2.3	Test Requirements	977

A.7	NR standalone tests with one or more NR cells in FR2	977

A.7.1	SA: RRC_IDLE state mobility	977

A.7.1.1	Cell re-selection to NR	977

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	977

A.7.1.1.1.1	Test Purpose and Environment	977

A.7.1.1.1.2	Test Parameters	978

A.7.1.1.1.3	Test Requirements	981

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	981

A.7.1.1.2.1	Test Purpose and Environment	981

A.7.1.1.2.2	Test Parameters	981

A.7.1.1.2.3	Test Requirements	985

A.7.2	SA: RRC_INACTIVE state mobility	985

A.7.3	RRC_CONNECTED state mobility	985

A.7.3.1	Handover	985

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	985

A.7.3.1.1.1	Test Purpose and Environment	985

A.7.3.1.1.2	Test Parameters	985

A.7.3.1.1.3	Test Requirements	989

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	990

A.7.3.1.2.1	Test Purpose and Environment	990

A.7.3.1.2.2	Test Parameters	990

A.7.3.1.2.3	Test Requirements	991

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	992

A.7.3.1.3.1	Test Purpose and Environment	992

A.7.3.1.3.2	Test Parameters	992

A.7.3.1.3.3 Test Requirements	993

A.7.3.2	RRC Connection Mobility Control	994

A.7.3.2.1	SA: RRC Re-establishment	994

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	994

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	996

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	999

A.7.3.2.1.3.1	Test Purpose and Environment	999

A.7.3.2.1.3.2	Test Requirements	1001

A.7.3.2.2	Random Access	1002

A.7.3.2.2.1	Contention based random access test in FR2 for NR Standalone	1002

A.7.3.2.2.2	Non-contention based random access test in FR2 for NR Standalone	1006

A.7.3.2.3	SA: RRC Connection Release with Redirection	1010

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1010

A.7.4	Timing	1013

A.7.4.1	UE transmit timing	1013

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1013

A.7.4.1.1.1	Test Purpose and environment	1013

A.7.4.1.1.2	Test requirements	1017

A.7.4.2	UE timer accuracy	1018

A.7.4.3	Timing advance	1018

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1018

A.7.4.3.1.1	Test Purpose and Environment	1018

A.7.4.3.1.2	Test Parameters	1018

A.7.4.3.1.3 Test Requirements	1022

A.7.5	Signaling characteristics	1022

A.7.5.1	Radio link Monitoring	1022

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1022

A.7.5.1.1.1	Test Purpose and Environment	1022

A.7.5.1.1.2	Test Requirements	1025

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1026

A.7.5.1.2.1	Test Purpose and Environment	1026

A.7.5.1.2.2	Test Requirements	1030

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1031

A.7.5.1.3.1	Test Purpose and Environment	1031

A.7.5.1.3.2	Test Requirements	1034

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1034

A.7.5.1.4.1	Test Purpose and Environment	1034

A.7.5.1.4.2	Test Requirements	1039

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1039

A.7.5.1.5.1	Test Purpose and Environment	1039

A.7.5.1.5.2	Test Requirements	1043

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1043

A.7.5.1.6.1	Test Purpose and Environment	1043

A.7.5.1.6.2	Test Requirements	1047

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1047

A.7.5.1.7.1	Test Purpose and Environment	1047

A.7.5.1.7.2	Test Requirements	1050

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1050

A.7.5.1.8.1	Test Purpose and Environment	1050

A.7.5.1.8.2	Test Requirements	1054

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1054

A.7.5.1.9.1	Test Purpose and Environment	1054

A.7.5.1.9.2	Test Requirements	1055

A.7.5.2	Interruption	1056

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1056

A.7.5.3	SCell Activation and Deactivation Delay	1059

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1059

A.7.5.3.1.1	Test Purpose and Environment	1059

A.7.5.3.1.2	Test Requirements	1061

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1061

A.7.5.3.2.1	Test Purpose and Environment	1061

A.7.5.3.2.2	Test Requirements	1065

A.7.5.4	Void	1066

A.7.5.5	Beam Failure Detection and Link recovery procedures	1066

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1066

A.7.5.5.1.1	Test Purpose and Environment	1066

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1071

A.7.5.5.2.1	Test Purpose and Environment	1071

A.7.5.5.2.2	Test Requirements	1076

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1076

A.7.5.5.3.1	Test Purpose and Environment	1076

A.7.5.5.3.2	Test Requirements	1082

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1082

A.7.5.5.4.1	Test Purpose and Environment	1082

A.7.5.5.4.2	Test Requirements	1088

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1088

A.7.5.5.5.1	Test Purpose and Environment	1088

A.7.5.5.5.2	Test Requirements	1093

A.7.5.6	Active BWP switch	1093

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1093

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1093

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1098

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1102

A.7.5.6.1.3.1	Test Purpose and Environment	1102

A.7.5.6.1.3.2	Test Requirements	1105

A.7.5.6.2	RRC-based Active BWP Switch	1105

A.7.5.7	PSCell addition and release delay	1109

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1109

A.7.5.7.1.1	Test Purpose and Environment	1109

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1113

A.7.5.7.2.1	Test Purpose and Environment	1113

A.7.5.7.2.2	Test Requirements	1117

A.7.5.8	Active TCI state switch delay	1117

A.7.5.8.1	MAC-CE based active TCI state switch	1117

A.7.5.8.2	RRC based active TCI state switch	1119

A.7.6	Measurement procedure	1122

A.7.6.1	Intra-frequency Measurements	1122

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1122

A.7.6.1.1.1	Test purpose and Environment	1122

A.7.6.1.1.2	Test Requirements	1124

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1124

A.7.6.1.2.1	Test purpose and Environment	1124

A.7.6.1.2.2	Test Requirements	1127

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1128

A.7.6.1.3.1	Test purpose and Environment	1128

A.7.6.1.3.2	Test Requirements	1130

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1130

A.7.6.1.4.1	Test purpose and Environment	1130

A.7.6.1.4.2	Test Requirements	1133

A.7.6.2	Inter-frequency Measurements	1133

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1133

A.7.6.2.1.1	Test Purpose and Environment	1133

A.7.6.2.1.2	Test Requirements	1136

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1136

A.7.6.2.2.1	Test Purpose and Environment	1136

A.7.6.2.2.2	Test Requirements	1140

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1140

A.7.6.2.3.1	Test Purpose and Environment	1140

A.7.6.2.3.2	Test Requirements	1144

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1144

A.7.6.2.4.1	Test Purpose and Environment	1144

A.7.6.2.4.2	Test Requirements	1148

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1148

A.7.6.2.5.1	Test Purpose and Environment	1148

A.7.6.2.5.2	Test Requirements	1152

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1153

A.7.6.2.6.1	Test Purpose and Environment	1153

A.7.6.2.6.2	Test Requirements	1157

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1158

A.7.6.2.7.1	Test Purpose and Environment	1158

A.7.6.2.7.2	Test Requirements	1162

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1163

A.7.6.2.8.1	Test Purpose and Environment	1163

A.7.6.2.8.2	Test Requirements	1167

A.7.6.3	L1-RSRP measurement for beam reporting	1168

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1168

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1168

A.7.6.3.1.1	Test Purpose and Environment	1168

A.7.6.3.1.2	Test parameters	1168

A.7.6.3.1.3	Test Requirements	1170

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1171

A.7.6.3.2.1	Test Purpose and Environment	1171

A.7.6.3.2.2	Test parameters	1171

A.7.6.3.2.3	Test Requirements	1173

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1174

A.7.6.3.3.1	Test Purpose and Environment	1174

A.7.6.3.3.2	Test parameters	1174

A.7.6.3.3.3	Test Requirements	1176

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1177

A.7.6.3.4.1	Test Purpose and Environment	1177

A.7.6.3.4.2	Test parameters	1177

A.7.6.3.3.3	Test Requirements	1179

A.7.6.4	CLI measurements	1180

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1180

A.7.6.4.1.1	Test Purpose and Environment	1180

A.7.6.4.1.2	Test Parameters	1180

A.7.6.4.1.3	Test Requirements	1182

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1182

A.7.6.4.2.1	Test Purpose and Environment	1182

A.7.6.4.2.2	Test Parameters	1183

A.7.6.4.2.3	Test Requirements	1185

A.7.7	Measurement Performance requirements	1185

A.7.7.1	SS-RSRP	1185

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1185

A.7.7.1.1.1	Test Purpose and Environment	1185

A.7.7.1.1.3	Test Requirements	1189

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1189

A.7.7.1.2.1	Test Purpose and Environment	1189

A.7.7.1.2.2	Test parameters	1190

A.7.7.1.2.3	Test Requirements	1193

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1193

A.7.7.1.3.1	Test Purpose and Environment	1193

A.7.7.1.3.2	Test parameters	1194

A.7.7.1.3.3	Test Requirements	1197

A.7.7.2	SS-RSRQ	1197

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1197

A.7.7.2.1.1	Test Purpose and Environment	1197

A.7.7.2.1.2	Test Parameters	1198

A.7.7.2.1.3	Test Requirements	1200

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1200

A.7.7.2.2.1	Test Purpose and Environment	1200

A.7.7.2.2.2	Test Parameters	1200

A.7.7.2.2.3	Test Requirements	1202

A.7.7.3	SS-SINR	1202

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1202

A.7.7.3.1.1	Test Purpose and Environment	1202

A.7.7.3.1.2	Test Parameters	1202

A.7.7.3.1.3	Test Requirements	1205

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1205

A.7.7.3.2.1	Test Purpose and Environment	1205

A.7.7.3.2.2	Test Parameters	1205

A.7.7.3.2.3	Test Requirements	1207

A.7.7.4	L1-RSRP measurement for beam reporting	1207

A.7.7.4.1	SSB based L1-RSRP measurement	1207

A.7.7.4.1.1	Test Purpose and Environment	1207

A.7.7.4.1.2	Test parameters	1208

A.7.7.4.1.3	Test Requirements	1210

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1211

A.7.7.4.2.1	Test Purpose and Environment	1211

A.7.7.4.2.2	Test parameters	1211

A.7.7.4.2.3	Test Requirements	1213

A.7.7.5	CLI measurements	1214

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	1214

A.7.7.5.1.1	Test Purpose and Environment	1214

A.7.7.5.1.2	Test parameters	1214

A.7.7.5.1.3	Test Requirements	1217

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	1218

A.7.7.5.2.1	Test Purpose and Environment	1218

A.7.7.5.2.2	Test parameters	1218

A.7.7.5.2.3	Test Requirements	1220

A.8	E-UTRA standalone tests for NR RRM	1221

A.8.1	Void	1221

A.8.2	RRC_IDLE state mobility	1221

A.8.2.1	Inter-RAT NR Cell re-selection	1221

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	1221

A.8.2.1.1.1	Test Purpose and Environment	1221

A.8.2.1.1.2	Test Requirements	1226

A.8.3	RRC_CONNECTED state mobility	1227

A.8.3.1	Handover	1227

A.8.3.1.1	E-UTRAN - NR handover in FR1	1227

A.8.3.1.1.1	Test Purpose and Environment	1227

A.8.3.1.1.2	Test Requirements	1232

A.8.4	Measurement procedure	1232

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	1232

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	1232

A.8.4.1.1.1	Test Purpose and Environment	1232

A.8.4.1.1.2	Test Requirements	1235

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	1235

A.8.4.1.2.1	Test Purpose and Environment	1235

A.8.4.1.2.2	Test Requirements	1236

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	1236

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1236

A.8.4.2.1.1	Test Purpose and Environment	1236

A.8.4.2.1.2	Test Requirements	1240

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1241

A.8.4.2.2.1	Test Purpose and Environment	1241

A.8.4.2.2.2	Test Requirements	1244

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1245

A.8.4.2.3.1	Test Purpose and Environment	1245

A.8.4.2.3.2	Test Requirements	1248

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1249

A.8.4.2.4.1	Test Purpose and Environment	1249

A.8.4.2.4.2	Test Requirements	1252

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	1253

A.8.4.2.5.1	Test Purpose and Environment	1253

A.8.4.2.5.2	Test Requirements	1255

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	1256

A.8.4.2.6.1	Test Purpose and Environment	1256

A.8.4.2.6.2	Test Requirements	1258

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	1259

A.8.4.2.7.1	Test Purpose and Environment	1259

A.8.4.2.7.2	Test Requirements	1261

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	1262

A.8.4.2.8.1	Test Purpose and Environment	1262

A.8.4.2.8.2	Test Requirements	1264

A.8.5	Measurement performance	1265

A.8.5.1	SFTD accuracy	1265

A.8.5.1.1	SFTD accuracy	1265

A.8.5.1.1.1	Test Purpose	1265

A.8.5.1.1.2	Test Environment	1265

A.8.5.1.1.3	Test Requirements	1271

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	1271

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1271

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1275

A.8.5.2.1.2.1	Test Purpose and Environment	1275

A.8.5.2.1.2.2	Test Parameters	1275

A.8.5.2.1.2.3	Test Requirements	1277

A.8.5.2.2	SS-RSRQ	1277

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1277

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1281

A.8.5.2.2.2.1	Test Purpose and Environment	1281

A.8.5.2.2.2.2	Test Parameters	1282

A.8.5.2.2.2.3	Test Requirements	1284

A.8.5.2.3	SS-SINR	1284

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1284

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1287

A.8.5.2.3.2.1	Test Purpose and Environment	1287

A.8.5.2.3.2.2	Test Parameters	1287

A.8.5.2.3.2.3	Test Requirements	1289

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	1151

B.1	Conditions for NR RRC_IDLE state mobility	1151

B.1.1	Introduction	1151

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1151

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1152

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1153

B.2.1	Introduction	1153

B.2.1.1	General	1153

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1153

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1153

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1153

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1154

B.2.1.4	Gain to SS-RSRP measurement point for FR1	1154

B.2.1.5	Gain to SS-RSRP measurement point for FR2	1155

B.2.1.5.1	Gain to SS-RSRP measurement point for Rx Beam Peak angle of arrival	1155

B.2.2	Conditions for NR intra-frequency measurements	1155

B.2.3	Conditions for NR inter-frequency measurements	1156

B.2.4	Conditions for NR L1-RSRP reporting	1158

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1158

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1159

B.2.5	Conditions for RRC connection release with redirection to NR	1160

B.2.6	Conditions for UE transmit timing	1161

B.2.6.1	Conditions for SSB based UE transmit timing	1161

B.2.6.2	Void	1163

B.3	RRM Requirements Exceptions	1163

B.3.1	Introduction	1163

B.3.2	Receiver sensitivity relaxation for CA	1163

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1163

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1164

B.3.2.2.1	Inter-band carrier aggregation	1164

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1164

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1164

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1164

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1164

B.3.2.4.1	Intra-band contiguous carrier aggregation	1164

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1164

B.3.3	Receiver sensitivity relaxation for DC	1165

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1165

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1165

B.3.4	Receiver sensitivity relaxation for SUL	1165

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1165

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1165

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1165

Annex C (informative):	Change history	1166