| 3GPP TS 36.133 V17.1.0 (2021-03) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.6	General	104

3.6.1	Applicability of requirements in this specification version	104

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	110

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	112

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	113

3.6.2	Applicability of requirements for EN-DC operation	114

3.6.3	Applicability of requirements for NE-DC operation	115

3.6.4	Applicability of requirements for NGEN-DC operation	116

4	E-UTRAN RRC_IDLE state mobility	116

4.1	Cell Selection	116

4.2	Cell Re-selection	117

4.2.1	Introduction	117

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	118

4.2.2.2	Void	119

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	119

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	121

4.2.2.5	Measurements of inter-RAT cells	123

4.2.2.5.1	Measurements of UTRAN FDD cells	123

4.2.2.5.2	Measurements of UTRAN TDD cells	125

4.2.2.5.3	Measurements of GSM cells	126

4.2.2.5.4	Measurements of HRPD cells	127

4.2.2.5.5	Measurements of cdma2000 1X	128

4.2.2.5.6	Measurements of NR cells	129

4.2.2.5.7	Measurements of NR cells subject to CCA	131

4.2.2.6	Evaluation of cell re-selection criteria	133

4.2.2.7	Maximum interruption in paging reception	133

4.2.2.8	void	134

4.2.2.9	UE measurement capability	134

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	134

4.2.2.10	Reselection to CSG cells	135

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	135

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	136

4.2.2.11	Void	137

4.2.2.12	Void	137

4.2.2.13	Void	137

4.3	Minimization of Drive Tests (MDT)	137

4.3.1	Introduction	138

4.3.2	Measurements	138

4.3.2.1	Requirements	138

4.3.3	Relative Time Stamp Accuracy	138

4.3.3.1	Requirements	138

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	138

4.3.4.1	 Requirements	138

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	139

4.3.5.1	Requirements for timeSinceFailure	139

4.4	MBSFN Measurements	139

4.4.1	Introduction	139

4.4.2	MBSFN RSRP measurements	139

4.4.3	MBSFN RSRQ measurements	139

4.4.4	MCH BLER measurements	140

4.5	Proximity-based Services	140

4.5.1	Introduction	140

4.5.2	Requirements	140

4.5.2.1	Interruptions with ProSe Direct Discovery	140

4.5.2.2	Interruptions with ProSe Direct Communication	140

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	140

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	141

4.6	Cell Selection and Re-selection Requirements for UE category NB1	141

4.6.1	Cell Selection	141

4.6.2	Cell Re-selection	141

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	142

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	143

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	144

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	145

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	147

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	147

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	149

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	150

4.6.2.7	Maximum interruption in paging reception in normal coverage	152

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	152

4.6.2.8	UE measurement capability	152

4.6.2.9	WUS receptions for NB1	152

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	153

4.6.3.1	Introduction	153

4.6.3.2	Requirements on UE synchronization for transmission using PUR	153

4.6.3.3	Requirements on TA validation for transmission using PUR	153

4.7	Cell Selection and Re-selection Requirements for UE category M1	154

4.7.1	Cell Selection	154

4.7.2	Cell Re-selection	154

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	154

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	154

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	155

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	156

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	157

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	159

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	159

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	160

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	160

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	161

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	162

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	164

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	166

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	167

4.7.2.3	WUS receptions for UE category M1	167

4.7.3	Channel quality report for UE Category M1 in idle mode	167

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	168

4.7.4.1	Introduction	168

4.7.4.2	Requirements on UE synchronization for transmission using PUR	168

4.7.4.3	Requirements on TA validation for transmission using PUR	168

4.8	Idle State Positioning Measurement Requirements for UE category NB1	169

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	169

4.8.1.1	RSTD Measurement Reporting Delay	171

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	171

4.8.2.1	RSTD Measurement Reporting Delay	172

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	173

4.8.3.1	RSTD Measurement Reporting Delay	174

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	175

4.8.4.1	RSTD Measurement Reporting Delay	176

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	177

4.8.5.1	Measurement Reporting Delay	178

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	178

4.8.6.1	Measurement Reporting Delay	179

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	180

4.8.7.1	Measurement Reporting Delay	181

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	181

4.8.8.1	Measurement Reporting Delay	182

4.9	Idle Mode CA Measurement	183

4.9.1	Introduction	183

4.9.2	Requirement	183

4.9.2.1	Detected cell requirement during state transition and Idle mode	183

4.9.2.2	Measurements of inter-frequency CA candidate cells	184

4.9.2.3	Measurements on serving cell	184

4A	E-UTRAN RRC_INACTIVE state mobility	185

4A.1	Cell Re-selection	185

4A.1.1	Introduction	185

4A.1.2	Requirements	186

4A.1.2.1	UE measurement capability	186

4A.1.2.2	Measurement and evaluation of serving cell	186

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	186

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	186

4A.1.2.5	Evaluation of cell re-selection criteria	186

4A.1.2.6	Maximum interruption in paging reception	186

4A.1.2.7	Measurements of inter-RAT NR cells	186

4A.2	Requirements for UE Category M1	186

4A.2.1	Introduction	186

4A.2.2	Cell Selection	186

4A.2.3	Cell Reselection	186

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	186

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	186

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	187

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	187

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	187

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	187

4A.2.4	Channel quality report for UE Category M1 in idle mode	189

5	E-UTRAN RRC_CONNECTED state mobility	189

5.1	E-UTRAN Handover	190

5.1.1	Introduction	190

5.1.2	Requirements	190

5.1.2.1	E-UTRAN FDD – FDD	190

5.1.2.1.1	Handover delay	190

5.1.2.1.2	Interruption time	190

5.1.2.2	E-UTRAN FDD – TDD	191

5.1.2.2.1	(Void)	192

5.1.2.2.2	(Void)	192

5.1.2.3	E-UTRAN TDD – FDD	192

5.1.2.3.1	(Void)	192

5.1.2.3.2	(Void)	192

5.1.2.4	E-UTRAN TDD – TDD	192

5.1.2.4.1	Handover delay	192

5.1.2.4.2	Interruption time	192

5.1.2.5	E-UTRAN HD–FDD	194

5.1.2.5.1	Handover delay	194

5.1.2.5.2	Interruption time	194

5.1.2.6	E-UTRAN FDD – FDD conditional handover	195

5.1.2.6.1	Handover delay	195

5.1.2.6.2	Measurement time	196

5.1.2.6.3	Preparation time	196

5.1.2.6.4	Interruption time	196

5.1.2.7	E-UTRAN FDD – TDD conditional handover	196

5.1.2.8	E-UTRAN TDD – FDD conditional handover	197

5.1.2.9	E-UTRAN TDD – TDD conditional handover	197

5.2	Void	197

5.3	Handover to other RATs	197

5.3.1	E-UTRAN - UTRAN FDD Handover	197

5.3.1.1	Introduction	197

5.3.1.1.1	Handover delay	197

5.3.1.1.2	Interruption time	197

5.3.2	E-UTRAN - UTRAN TDD Handover	198

5.3.2.1	Introduction	198

5.3.2.2	Requirements	198

5.3.2.2.1	Handover delay	198

5.3.2.2.2	Interruption time	198

5.3.3	E-UTRAN - GSM Handover	199

5.3.3.1	Introduction	199

5.3.3.2	Requirements	199

5.3.3.2.1	Handover delay	199

5.3.3.2.2	Interruption time	199

5.3.4	E-UTRAN - NR FR1 Handover	199

5.3.4.1	Introduction	199

5.3.4.2	Handover delay	200

5.3.4.3	Interruption time	200

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	200

5.3.4A.1	Introduction	200

5.3.4A.2	Handover delay	201

5.3.4A.3	Interruption time	201

5.3.5	E-UTRAN - NR FR2 Handover	202

5.3.5.1	Introduction	202

5.3.5.2	Handover delay	202

5.3.5.3	Interruption time	202

5.4	Handover to Non-3GPP RATs	203

5.4.1	E-UTRAN – HRPD Handover	203

5.4.1.1	Introduction	203

5.4.1.1.1	Handover delay	203

5.4.1.1.2	Interruption time	203

5.4.2	E-UTRAN – cdma2000 1X Handover	204

5.4.2.1	Introduction	204

5.4.2.1.1	Handover delay	204

5.4.2.1.2	Interruption time	204

5.5	E-UTRAN Handover for Cat-M1 UEs	204

5.5.1	Introduction	204

5.5.2	Requirements in CEModeA	204

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	204

5.5.2.1.1	Handover delay	204

5.5.2.1.2	Interruption time	205

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	205

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	205

5.5.2.3.1	Void	206

5.5.2.3.2	Void	206

5.5.3	Requirements in CEModeB	206

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	206

5.5.3.1.1	Handover delay	206

5.5.3.1.2	Interruption time	206

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	207

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	207

5.6	Void	207

5.7	E-UTRAN DAPS Handover	207

5.7.1	Introduction	207

5.7.2	Requirements	207

5.7.2.1	E-UTRAN FDD – FDD	207

5.7.2.1.1	DAPS Handover delay	207

5.7.2.1.2	Interruption time	208

5.7.2.2	E-UTRAN FDD – TDD	208

5.7.2.3	E-UTRAN TDD – FDD	208

5.7.2.4	E-UTRAN TDD – TDD	209

6	RRC Connection Mobility Control	209

6.1	RRC Re-establishment	209

6.1.1	Introduction	209

6.1.2	Requirements	209

6.1.2.1	UE Re-establishment delay requirement	209

6.2	Random Access	210

6.2.1	Introduction	210

6.2.2	Requirements	210

6.2.2.1	Contention based random access	210

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	210

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	210

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	210

6.2.2.1.4	Void	210

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	210

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	210

6.2.2.2	Non-Contention based random access	211

6.2.2.2.1	Correct behaviour when receiving Random Access Response	211

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	211

6.2.3	Requirements for Cat-M1 UEs	211

6.3	RRC Connection Release with Redirection	211

6.3.1	Introduction	211

6.3.2	Requirements	211

6.3.2.1	RRC connection release with redirection to UTRAN FDD	211

6.3.2.2	RRC connection release with redirection to GERAN	212

6.3.2.3	RRC connection release with redirection to UTRAN TDD	212

6.3.2.4	RRC connection release with redirection to NR	213

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	213

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	214

6.4.1	Introduction	214

6.4.2	Requirements	215

6.5	RRC Re-establishment for NB-IoT UEs	215

6.5.1	Introduction	215

6.5.2	Requirements	215

6.5.2.1	UE Re-establishment delay requirement in normal coverage	215

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	216

6.6	Random Access for UE category NB1	216

6.6.1	Introduction	216

6.6.2	Requirements	216

6.6.2.1	Correct behaviour when receiving Random Access Response reception	216

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	217

6.6.2.3	Correct behaviour when receiving a NACK on msg3	217

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	217

6.6.2.5	Correct behaviour when contention Resolution timer expires	217

6.6.2.6	MSG3-based channel quality report for UE Category NB1	217

6.6.3	Requirements for NPRACH configuration	218

6.7	RRC Re-establishment for Cat-M1 UEs	218

6.7.1	Introduction	218

6.7.2	Requirements	218

6.7.2.1	UE Re-establishment delay requirement for CEModeA	218

6.7.2.2	UE Re-establishment delay requirement for CEModeB	219

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	219

6.8.1	Introduction	219

6.8.2	Requirements	219

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	219

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	220

6.9.1	Introduction	220

6.9.2	Requirements	220

7	Timing and signalling characteristics	221

7.1	UE transmit timing	221

7.1.1	Introduction	221

7.1.2	Requirements	221

7.2	UE timer accuracy	223

7.2.1	Introduction	223

7.2.2	Requirements	223

7.3	Timing Advance	223

7.3.1	Introduction	223

7.3.2	Requirements	223

7.3.2.1	Timing Advance adjustment delay	223

7.3.2.2	Timing Advance adjustment accuracy	224

7.4	Cell phase synchronization accuracy (TDD)	224

7.4.1	Definition	224

7.4.2	Minimum requirements	224

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	224

7.5.1	Introduction	224

7.5.2	eNodeB Synchronization Requirements	225

7.5.2.1	Synchronized E-UTRAN	225

7.5.2.2	Non-Synchronized E-UTRAN	225

7.6	Radio Link Monitoring	225

7.6.1	Introduction	225

7.6.2	Requirements	227

7.6.2.1	Minimum requirement when no DRX is used	227

7.6.2.2	Minimum requirement when DRX is used	227

7.6.2.3	Minimum requirement at transitions	228

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	229

7.6.2.5	Minimum requirement under IDC Interference	229

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	229

7.7.1	Introduction	229

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	229

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	231

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	232

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	233

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	234

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	235

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	235

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	235

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	236

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	237

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	238

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	239

7.7.14	SCell Activation Delay Requirement for Dormant SCell	239

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	240

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	241

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	243

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	243

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	245

7.8	Interruptions with Carrier Aggregation	247

7.8.1	Introduction	247

7.8.2	Requirements	247

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	247

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	247

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	247

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	247

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	248

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	248

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	248

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	248

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	249

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	250

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	250

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	250

7.8.2.13	Interruptions at SRS carrier based switching	251

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	251

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	252

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	252

7.8.2.17	Interruptions during CQI measurement on dormant SCell	252

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	252

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	253

7.8.2.20	Interruptions at SCell hibernation	253

7.8.2.21	Interruptions at direct SCell activation and hibernation	253

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	254

7.9.1	Introduction	254

7.9.2	Minimum Requirements for Interband Carrier Aggregation	254

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	254

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	255

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	255

7.10.1	Introduction	255

7.10.2	Requirements	255

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	255

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	255

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	256

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	256

7.11	Radio Link Monitoring for UE Category 0	256

7.11.1	Introduction	256

7.11.2	Requirements for FD-FDD and TDD	258

7.11.2.1	Minimum requirement when no DRX is used	258

7.11.2.2	Minimum requirement when DRX is used	258

7.11.2.3	Minimum requirement at transitions	259

7.11.3	Requirements for HD-FDD	259

7.11.3.1	Minimum requirement when no DRX is used	259

7.11.3.2	Minimum requirement when DRX is used	259

7.11.3.3	Minimum requirement at transitions	260

7.12	Interruptions with Dual Connectivity	260

7.12.1	Introduction	260

7.12.2	Requirements	260

7.12.2.1	Interruptions at PSCell addition/release	260

7.12.2.2	Interruptions at transitions between active and non-active during DRX	261

7.12.2.3	Interruptions at transitions from non-DRX to DRX	261

7.12.2.4	Interruptions at SCell addition/release	261

7.12.2.5	Interruptions at SCell activation/deactivation	261

7.12.2.6	Interruptions during measurements on SCC	262

7.12.2.7	Interruptions at SRS carrier based switching	262

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	263

7.13.1	Definition	263

7.13.2	Minimum requirements	263

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	263

7.14.1	Introduction	263

7.14.2	PSCell Addition Delay Requirement	263

7.14.3	PSCell Release Delay Requirement	264

7.15	Maximum Receive Timing Difference in Dual Connectivity	264

7.15.1	Introduction	264

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	264

7.16	Proximity-based Services	265

7.16.1	Introduction	265

7.16.2	Requirements	265

7.16.2.1	ProSe UE transmission timing	265

7.16.2.1.1	Serving cell or PCell as timing reference	265

7.16.2.1.2	SCell or non-serving cell as timing reference	265

7.16.3	Interruptions with ProSe	265

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	266

7.16.3.2	Interruptions at ProSe Direct Communication configuration	266

7.16.3.3	Interruptions during ProSe Direct Discovery	266

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	266

7.16.3.5	Interruptions during ProSe Direct Communication	267

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	267

7.16.4.1	Measurement and evaluation of selected cell	267

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	267

7.16.5	Selection / Reselection of ProSe relay UE	268

7.16.6	ProSe operation under deactivated SCell	268

7.17	Maximum Transmission Timing Difference in Dual Connectivity	269

7.17.1	Introduction	269

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	269

7.18.1	Introduction	269

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	269

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	269

7.19	Radio Link Monitoring for UE Category M1	269

7.19.1	Introduction	269

7.19.2	Requirements for FD-FDD and TDD CE mode A	270

7.19.2.1	Minimum requirement when no DRX is used	272

7.19.2.2	Minimum requirement when DRX is used	273

7.19.2.3	Minimum requirement at transitions	274

7.19.3	Requirements for HD-FDD with CE mode A	274

7.19.3.1	Minimum requirement when no DRX is used	274

7.19.3.2	Minimum requirement when DRX is used	274

7.19.3.3	Minimum requirement at transitions	276

7.19.4	Requirements for FD-FDD and TDD with CE mode B	276

7.19.4.1	Minimum requirement when no DRX is used	278

7.19.4.2	Minimum requirement when DRX is used	279

7.19.4.3	Minimum requirement at transitions	280

7.19.5	Requirements for HD-FDD with CE mode B	280

7.19.5.1	Minimum requirement when no DRX is used	280

7.19.5.2	Minimum requirement when DRX is used	280

7.19.5.3	Minimum requirement at transitions	282

7.20	UE transmit timing for NB-IoT	282

7.20.1	Introduction	282

7.20.2	Requirements	282

7.21	 UE timer accuracy for NB-IoT	283

7.21.1	Introduction	283

7.21.2	Requirements	283

7.22	Timing Advance for NB-IoT	283

7.22.1	Introduction	283

7.22.2	Requirements	283

7.22.2.1	Timing Advance adjustment delay	283

7.22.2.2	Timing Advance adjustment accuracy	283

7.23	Radio Link Monitoring for Category NB1 UE	283

7.23.1	Introduction	283

7.23.2	Requirements for Category NB1 UE	283

7.23.2.1	Minimum requirement when no DRX is used	284

7.23.2.2	Minimum requirement when DRX is used	284

7.23.2.3	Minimum requirement at transitions	285

7.24	UE transmit timing for Category M1	285

7.24.1	Introduction	285

7.24.2	Requirements	285

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	286

7.25.1	Definition	286

7.25.2	Minimum requirements	286

7.26	UE transmit timing for Category M2	287

7.26.1	Introduction	287

7.26.2	Requirements	287

7.27	UE timer accuracy for category M1	287

7.27.1	Introduction	287

7.27.2	Requirements	287

7.28	Timing Advance for Category M1	288

7.28.1	Introduction	288

7.28.2	Requirements	288

7.29	Interruptions requirements with FeMBMS	288

7.29.1	Introduction	288

7.29.2	Requirements	288

7.30	Numerology switching delay requirements with FeMBMS	288

7.30.1	Introduction	288

7.30.2	Requirements	288

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	288

7.31.1	Introduction	288

7.31.2	NR PSCell Addition Delay Requirement	289

7.31.3	NR PSCell Release Delay Requirement	289

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	290

7.31A.1	Introduction	290

7.31A.2	NR PSCell Addition Delay Requirement	290

7.31A.3	NR PSCell Release Delay Requirement	291

7.32	Interruptions with EN-DC	291

7.32.1	Introduction	291

7.32.2	Requirements	292

7.32.2.1	Interruptions at PSCell addition/release	292

7.32.2.2	Interruptions at transitions between active and non-active during DRX	292

7.32.2.3	Interruptions at transitions from non-DRX to DRX	292

7.32.2.4	Interruptions at SCell addition/release	292

7.32.2.5	Interruptions at SCell activation/deactivation	292

7.32.2.6	Interruptions during measurements on SCC	293

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	293

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	293

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	293

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	293

7.32.2.7	Interruptions at active BWP switching	293

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	294

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	294

7.32.2.10	Interruptions at SCell hibernation	295

7.32.2.11	Interruptions at direct SCell activation and hibernation	295

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	295

7.32.2.13	Interruptions at NR SRS carrier based switching	295

7.32.2.14	Interruptions at NR SCell dormancy	296

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	296

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	296

7.32.2.15	Interruption during NR measurement with autonomous gaps	297

7.32.2.16	Interruptions at SRS carrier based switching	297

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	297

7.33.1	Introduction	297

7.33.2	Requirements	298

7.34	Void	298

7.35	Interruptions with SFTD measurements	298

7.35.1	Introduction	298

7.35.2	Requirements	298

7.36	Interruptions with NE-DC	298

7.36.1	Introduction	298

7.36.2	Requirements	299

7.36.2.1	Interruptions at transitions between active and non-active during DRX	299

7.36.2.2	Interruptions at transitions from non-DRX to DRX	299

7.36.2.3	Interruptions at SCell addition/release	299

7.36.2.4	Interruptions at SCell activation/deactivation	299

7.36.2.5	Interruptions during measurements on SCC	300

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	300

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	300

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	300

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	300

7.36.2.6	Interruptions at active BWP switching	300

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	301

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	301

7.36.2.9	Interruptions at SCell hibernation	302

7.36.2.10	Interruptions at direct SCell activation and hibernation	302

7.36.2.11	Interruptions at NR SRS carrier based switching	302

7.36.2.12	Interruptions at NR SCell dormancy	303

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	303

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	303

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	303

7.36.2.14	Interruption during NR measurement with autonomous gaps	304

7.36.2.15	Interruptions at SRS carrier based switching	304

7.37	Interruptions during NR measurement with autonomous gaps	305

7.37.1	Introduction	305

7.37.2	Requirements	305

8	UE Measurements Procedures in RRC_CONNECTED State	305

8.1	General Measurement Requirements	305

8.1.1	Introduction	305

8.1.2	Requirements	305

8.1.2.1	UE measurement capability	305

8.1.2.1.1	Monitoring of multiple layers using gaps	314

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	315

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	316

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	318

8.1.2.1.2	Network controlled small gap	319

8.1.2.2	E-UTRAN intra frequency measurements	320

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	320

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	325

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	329

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	330

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	331

8.1.2.3	E-UTRAN inter frequency measurements	332

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	332

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	338

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	344

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	344

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	345

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	346

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	347

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	348

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	349

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	356

8.1.2.4	Inter RAT measurements	356

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	356

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	361

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	361

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	365

8.1.2.4.5	E-UTRAN FDD – GSM measurements	365

8.1.2.4.6	E-UTRAN TDD – GSM measurements	370

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	370

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	372

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	372

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	372

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	373

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	373

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	373

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	373

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	375

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	375

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	375

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	375

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	376

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	376

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	378

8.1.2.4.21	E-UTRAN FDD – NR measurements	378

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	382

8.1.2.4.22	E-UTRAN TDD – NR measurements	386

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	386

8.1.2.4.23	Void	386

8.1.2.4.24	Void	386

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	386

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	388

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	388

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	389

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	389

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	390

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	391

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	393

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	395

8.1.2.6.5	Void	396

8.1.2.6.6	Void	396

8.1.2.6.7	Void	396

8.1.2.6.8	Void	396

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	396

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	397

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	398

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	400

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	402

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	404

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	405

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	407

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	409

8.1.2.7	E-UTRAN E-CID Measurements	411

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	411

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	412

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	413

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	414

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	414

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	415

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	417

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	420

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	424

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	427

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	427

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	428

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	428

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	429

8.1.2.10	Void	429

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	429

8.2.1	Introduction	429

8.2.2	Requirements	429

8.3	Measurements for E-UTRA carrier aggregation	434

8.3.1	Introduction	434

8.3.2	Measurements of the primary component carrier	434

8.3.3	Measurements of a secondary component carrier	434

8.3.3.1	Measurements of a secondary component carrier with active SCell	434

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	435

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	435

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	436

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	438

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	438

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	438

8.4.1	Introduction	438

8.4.2	Measurements on the primary component carrier	439

8.4.3	Measurements on a secondary component carrier	440

8.4.4	Measurements on both primary component carrier and a secondary component carrier	440

8.4.5	Measurements on different secondary component carriers	441

8.5	Measurements for UE category 0	442

8.5.1	Introduction	442

8.5.2 	Requirements	443

8.5.2.1 	E-UTRAN intra frequency measurements	443

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	443

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	446

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	448

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	452

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	452

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	453

8.6	Discovery signal measurements	454

8.6.1	Introduction	454

8.6.2	Requirements for CRS based discovery signal measurements	454

8.6.2.1	E-UTRAN intra frequency measurements	454

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	454

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	457

8.6.2.2	E-UTRAN inter frequency measurements	459

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	460

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	462

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	465

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	465

8.6.3	Requirements for CSI-RS based discovery signal measurements	465

8.6.3.1	E-UTRAN intra frequency measurements	465

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	466

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	468

8.6.3.2	E-UTRAN inter frequency measurements	470

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	471

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	473

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	476

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	476

8.7	Discovery signal measurements for E-UTRA carrier aggregation	476

8.7.1	Introduction	476

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	477

8.7.2.1	Measurements of the primary component carrier	477

8.7.2.2	Measurements of a secondary component carrier	477

8.7.2.3	Measurements of a secondary component carrier with active SCell	477

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	477

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	477

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	478

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	480

8.7.3.1	Measurements of the primary component carrier	480

8.7.3.2	Measurements of a secondary component carrier	480

8.7.3.3	Measurements of a secondary component carrier with active SCell	480

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	480

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	480

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	482

8.8	Measurements for E-UTRA dual connectivity	483

8.8.1	Introduction	483

8.8.2	Intra-frequency measurements requirements on PCell	483

8.8.3	Intra-frequency measurements requirements on PSCell	484

8.8.4	Inter-frequency and inter-RAT measurement requirements	484

8.8.5	Intra-frequency measurements with autonomous gaps	484

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	484

8.8.5.2	ECGI reporting delay	485

8.8.6	Inter-frequency measurements with autonomous gaps	485

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	485

8.8.6.2	ECGI reporting delay	486

8.8.7	SSTD Measurements	486

8.8.7.1	Introduction	486

8.8.7.2	SSTD Measurement requirements	486

8.8.7.3	SSTD Measurement Reporting Delay	487

8.8.8	Intra-frequency measurements requirements on SCell	487

8.9	MBSFN Measurements	487

8.9.1	Introduction	487

8.9.2	MBSFN RSRP Measurements	487

8.9.3	MBSFN RSRQ Measurements	488

8.9.4	MCH BLER Measurements	488

8.10	Proximity-based Services	488

8.10.1	Introduction	488

8.10.2	Requirements	488

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	488

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	489

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	489

8.11.1	Introduction	489

8.11.2	CRS based discovery signal measurements	490

8.11.2.1	E-UTRAN intra-frequency measurements	490

8.11.2.1.1	Requirements	490

8.11.2.1.1.1	Requirements when no DRX is used	490

8.11.2.1.1.1.1	Measurement Reporting Requirements	492

8.11.2.1.1.2	Requirements when DRX is used	493

8.11.2.1.1.2.1	Measurement Reporting Requirements	495

8.11.2.2	E-UTRAN inter-frequency measurements	496

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	496

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	500

8.11.3	CSI-RS based discovery signal measurements	500

8.11.3.1	E-UTRAN intra-frequency measurements	500

8.11.3.1.1	Requirements	500

8.11.3.1.1.1	Requirements when no DRX is used	500

8.11.3.1.1.1.1	Measurement Reporting Requirements	502

8.11.3.1.1.2	Requirements when DRX is used	502

8.11.3.1.1.2.1	Measurement Reporting Requirements	503

8.11.3.2	E-UTRAN inter-frequency measurements	504

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	504

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	508

8.11.4	RSSI measurements	508

8.11.4.1	E-UTRAN intra-frequency measurements	508

8.11.4.2	E-UTRAN inter-frequency measurements	508

8.11.5	Channel occupancy measurements	509

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	509

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	509

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	509

8.12.1	Introduction	509

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	509

8.12.2.1	Introduction	509

8.12.2.2	Measurements of a secondary component carrier	509

8.12.2.3	Measurements of a secondary component carrier with active SCell	509

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	510

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	510

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	512

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	515

8.12.3.1	Introduction	515

8.12.3.2	Measurements of a secondary component carrier	515

8.12.3.3	Measurements of a secondary component carrier with active SCell	515

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	515

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	515

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	517

8.13	Measurements for UE Category M1	519

8.13.1	Introduction	519

8.13.2	Requirements for UE category M1 with CE mode A	519

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	520

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	520

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	524

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	527

8.13.2.2	Void	531

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	531

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	531

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	534

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	536

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	537

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	537

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	539

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	541

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	542

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	542

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	542

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	543

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	543

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	543

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	544

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	544

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	545

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	546

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	546

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	547

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	551

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	553

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	557

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	557

8.13.3	Requirements for UE category M1 with CE mode B	558

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	558

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	558

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	563

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	566

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	571

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	572

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	572

8.13.3.2	Void	573

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	573

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	573

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	576

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	578

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	579

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	579

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	579

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	580

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	580

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	580

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	581

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	581

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	581

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	586

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	588

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	592

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	593

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	593

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	595

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	597

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	598

8.14	Measurements for UE category NB1	598

8.14.1	Introduction	598

8.14.2	NB-IoT intra frequency measurements under normal coverage	599

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	599

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	599

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	599

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	599

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	599

8.14.4	Connected mode channel quality report for UE Category NB1	599

8.15	Void	600

8.16	Measurements for UE Category M2	600

8.16.1	Introduction	600

8.16.2	Requirements for UE category M2 with CE mode A	600

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	600

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	601

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	601

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	602

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	602

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	602

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	603

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	603

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	605

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	608

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	608

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	608

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	611

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	613

8.16.3	Requirements for UE category M2 with CE mode B	614

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	614

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	614

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	616

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	619

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	619

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	620

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	622

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	624

8.17	Measurements for E-UTRA – NR Dual Connectivity	625

8.17.1	Introduction	625

8.17.1.1	Measurement Gap Sharing	625

8.17.1A	Intrafrequency Measurements	625

8.17.2	SFTD Measurements	626

8.17.2.1	Introduction	626

8.17.2.2	SFTD Measurement requirements	626

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	626

8.17.2.3	SFTD Measurement Reporting Delay	627

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	628

8.17.3.1	Introduction	628

8.17.3.2	E-UTRAN FDD inter frequency measurements	628

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	628

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	630

8.17.3.3	E-UTRAN TDD inter frequency measurements	632

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	632

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	633

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	635

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	635

8.17.4.1.1	NR Inter-RAT cell identification	636

8.17.4.1.2	NR Inter-RAT measurement	637

8.17.4.1.3	NR Inter-RAT measurement reporting	638

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	639

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	639

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	639

8.17.4A.1.1	NR Inter-RAT cell identification	640

8.17.4A.1.2	NR Inter-RAT measurement	641

8.17.4A.1.3	NR Inter-RAT measurement reporting	642

8.17.4A.1.4	NR inter-RAT RSSI measurements	642

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	643

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	643

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	643

8.17.5.1	Introduction	643

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	644

8.17.5.2.1	Identification of a new UTRA FDD cell	644

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	644

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	644

8.17.5.2.4	Periodic Reporting	645

8.17.5.2.5	Event Triggered Reporting	645

8.17.5.2.6	Event-triggered Periodic Reporting	645

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	645

8.17.5.3.1	Periodic Reporting	647

8.17.5.3.2	Event Triggered Reporting	647

8.17.5.3.3	Event-triggered Periodic Reporting	647

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	647

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	648

8.17.7.1	Introduction	648

8.17.7.2	Identification of a new UTRA FDD cell for SON	648

8.17.7.2.1	Requirements when no DRX is used	648

8.17.7.2.2	Requirements when DRX is used	648

8.17.7.2.3	Reporting Delay	649

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	650

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	650

8.17.9.1	Introduction	650

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	650

8.17.9.2.1	Identification of a new UTRA FDD cell	650

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	650

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	651

8.17.9.2.4	Periodic Reporting	651

8.17.9.2.5	Event Triggered Reporting	651

8.17.9.2.6	Event-triggered Periodic Reporting	652

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	652

8.17.9.3.1	Periodic Reporting	653

8.17.9.3.2	Event Triggered Reporting	653

8.17.9.3.3	Event-triggered Periodic Reporting	653

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	654

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	654

8.17.11.1	Introduction	654

8.17.11.2	Identification of a new UTRA TDD cell for SON	654

8.17.11.2.1	Requirements when no DRX is used	654

8.17.11.2.2	Requirements when DRX is used	654

8.17.11.2.3	Reporting Delay	655

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	655

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	656

8.17.13.1	Introduction	656

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	656

8.17.13.2.1	GSM carrier RSSI	656

8.17.13.2.2	BSIC verification	656

8.17.13.2.3	Enhanced BSIC verification	658

8.17.13.2.4	Periodic Reporting	658

8.17.13.2.5	Event Triggered Reporting	658

8.17.13.2.6	Event-triggered Periodic Reporting	659

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	659

8.17.13.3.1	GSM carrier RSSI	659

8.17.13.3.2	BSIC verification	659

8.17.13.3.3	Periodic Reporting	661

8.17.13.3.4	Event Triggered Reporting	661

8.17.13.3.5	Event-triggered Periodic Reporting	661

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	661

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	661

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	661

8.17.15.1.1	RSTD Measurement Reporting Delay	662

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	662

8.17.15.2.1	RSTD Measurement Reporting Delay	662

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	662

8.17.15.3.1	RSTD Measurement Reporting Delay	663

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	663

8.17.15.4.1	RSTD Measurement Reporting Delay	663

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	664

8.17.16.1	Introduction	664

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	664

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	664

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	664

8.17.17.1	Introduction	664

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	664

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	664

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	664

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	664

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	664

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	664

8.18	Measurements for non-BL/CE UE	664

8.18.1	Introduction	664

8.18.2	Requirements for non-BL/CE UE with CE Mode B	665

8.18.2.1	E-UTRAN intra frequency measurements	665

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	665

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	665

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	666

8.19	Measurements for NR – E-UTRA Dual Connectivity	667

8.19.1	Introduction	667

8.19.2	Intra-frequency Measurements	667

8.19.3	Inter-frequency Measurements	667

8.19.4	Void	667

8.19.5	Intra-frequency E-CID Measurements	667

8.19.6	Intra-frequency measurements with autonomous gaps	668

8.19.6.1	Introduction	668

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	668

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	668

8.19.7	Inter-frequency measurements with autonomous gaps	668

8.19.7.1	Introduction	668

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	668

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	668

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	668

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	668

9	Measurements performance requirements for UE	668

9.1	E-UTRAN measurements	668

9.1.1	Introduction	668

9.1.2	Intra-frequency RSRP Accuracy Requirements	669

9.1.2.1	Absolute RSRP Accuracy	669

9.1.2.2	Relative Accuracy of RSRP	669

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	670

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	671

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	672

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	673

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	674

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	675

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	676

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	676

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	676

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	677

9.1.2B.1	Introduction	677

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	677

9.1.3	Inter-frequency RSRP Accuracy Requirements	678

9.1.3.1	Absolute RSRP Accuracy	678

9.1.3.2	Relative Accuracy of RSRP	679

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	680

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	680

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	681

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	681

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	682

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	683

9.1.3B.1	Introduction	683

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	683

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	683

9.1.4	RSRP Measurement Report Mapping	684

9.1.5	Intra-frequency RSRQ Accuracy Requirements	684

9.1.5.1	Absolute RSRQ Accuracy	684

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	685

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	686

9.1.5.4	Absolute WB-RSRQ Accuracy	687

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	688

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	689

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	689

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	689

9.1.5B.1	Introduction	689

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	689

9.1.6	Inter-frequency RSRQ Accuracy Requirements	690

9.1.6.1	Absolute RSRQ Accuracy	690

9.1.6.2	Relative Accuracy of RSRQ	691

9.1.6.3	Absolute WB-RSRQ Accuracy	692

9.1.6.4	Relative WB-RSRQ Accuracy	692

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	693

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	694

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	694

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	694

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	695

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	696

9.1.6B.1	Introduction	696

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	696

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	696

9.1.7	RSRQ Measurement Report Mapping	697

9.1.8	Power Headroom	697

9.1.8.1	Period	698

9.1.8.2	Reporting Delay	698

9.1.8.3	Void	698

9.1.8.4	Report Mapping	698

9.1.9	UE Rx – Tx time difference	698

9.1.9.1	Measurement Requirement	698

9.1.9.2	Measurement Report mapping	699

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	700

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	701

9.1.10	Reference Signal Time Difference (RSTD)	702

9.1.10.1	Intra-Frequency Accuracy Requirement	702

9.1.10.2	Inter-Frequency Accuracy Requirement	703

9.1.10.3	RSTD Measurement Report Mapping	705

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	705

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	706

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	707

9.1.11	Carrier aggregation measurement accuracy	708

9.1.11.1	Primary component carrier accuracy requirement	709

9.1.11.2	Secondary component carrier accuracy requirement	709

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	709

9.1.11.4	Secondary component carrier relative accuracy requirement	709

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	709

9.1.13	Measurement accuracy for UE category 0	710

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	710

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	710

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	711

9.1.14	Accuracy requirements for Discovery Signal Measurements	712

9.1.14.1	Introduction	712

9.1.14.2	RSRP measurements in discovery signal occasions	712

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	712

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	712

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	712

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	713

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	714

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	714

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	714

9.1.14.3.3	CSI-RSRP measurement report mapping	715

9.1.14.4	RSRQ measurements in discovery signal occasions	715

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	715

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	716

9.1.15.1.1	Primary component carrier accuracy requirement	716

9.1.15.1.2	Secondary component carrier accuracy requirement	716

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	716

9.1.15.1.4	Secondary component carrier relative accuracy requirement	716

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	716

9.1.15.2.1	Primary component carrier accuracy requirement	716

9.1.15.2.2	Secondary component carrier accuracy requirement	716

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	716

9.1.15.2.4	Secondary component carrier relative accuracy requirement	716

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	717

9.1.17	RS-SINR Measurements	717

9.1.17.1	Measurement Report Mapping	717

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	718

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	718

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	718

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	718

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	719

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	720

9.1.18.1	Introduction	720

9.1.18.2	RSRP measurements	720

9.1.18.2.1	RSRP measurement report mapping	720

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	720

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	721

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	721

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	722

9.1.18.3	RSRQ measurements	722

9.1.18.3.1	RSRQ measurement report mapping	722

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	722

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	723

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	723

9.1.18.4	CSI-RSRP measurements	724

9.1.18.4.1	CSI-RSRP measurement report mapping	724

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	724

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	724

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	725

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	725

9.1.18.5	RSSI measurements	726

9.1.18.5.1	RSSI measurement report mapping	726

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	726

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	727

9.1.18.6	Channel occupancy measurements	727

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	727

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	727

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	728

9.1.19.1	Introduction	728

9.1.19.2	Accuracy requirements for measurements on SCC	728

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	728

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	728

9.1.20	SFN and Subframe Time Difference (SSTD)	729

9.1.20.1	SSTD Accuracy Requirement	729

9.1.20.2	SSTD Measurement Report Mapping	729

9.1.21	Measurement accuracy for UE category M1	730

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	730

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	732

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	733

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	735

9.1.21.5	RSRP Measurement Report Mapping	736

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	736

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	737

9.1.21.8	RSRQ Measurement Report Mapping	738

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	738

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	739

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	740

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	741

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	742

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	743

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	744

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	745

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	746

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	747

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	748

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	749

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	752

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	755

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	755

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	758

9.1.22	Measurement accuracy for UE Category NB1	759

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	759

9.1.22.2	Void	760

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	760

9.1.22.4	Void	761

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	761

9.1.22.6	Void	762

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	762

9.1.22.8	Void	763

9.1.22.9	NRSRP Measurement Report Mapping	763

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	763

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	764

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	765

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	766

9.1.22.14	NRSRQ Measurement Report Mapping	767

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	768

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	768

9.1.23	Power Headroom for UE Category NB1	769

9.1.23.1	Period	769

9.1.23.2	Reporting Delay	769

9.1.23.3	Report Mapping for UE Category NB1	769

9.1.23.3.1	Void	771

9.1.23.3.2	Void	771

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	771

9.1.24	Void	772

9.1.25	Measurement accuracy for UE category M2	772

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	772

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	773

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	774

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	775

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	777

9.1.26	Measurement Accuracy for non-BL CE UE	778

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	779

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	781

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	781

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	783

9.1.26.5	RSRP Measurement Report Mapping	784

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	784

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	784

9.1.26.8	RSRQ Measurement Report Mapping	785

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	785

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	786

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	787

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	788

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	789

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	789

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	789

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	790

9.1.27	SFN and frame Timing Difference (SFTD)	791

9.1.27.1	SFTD Accuracy Requirement	791

9.2	UTRAN FDD Measurements	793

9.2.1	UTRAN FDD CPICH RSCP	793

9.2.2	Void	794

9.2.3	UTRAN FDD CPICH Ec/No	794

9.3	UTRAN TDD Measurements	794

9.3.1	UTRAN TDD P-CCPCH RSCP	795

9.3.2	Void	795

9.3.3	Void	795

9.4	GSM Measurements	795

9.4.1	GSM carrier RSSI	795

9.5	CDMA2000 1x RTT Measurements	795

9.5.1	CDMA2000 1x RTT Pilot Strength	795

9.6	PCMAX,c	796

9.6.1	Report Mapping	796

9.6.2 	Estimation Period	796

9.6.3	Reporting Delay	796

9.7	IEEE802.11 Measurements	796

9.7.1	WLAN RSSI	796

9.7.2	WLAN RSSI Measurement Report Mapping	796

9.8	MBSFN Measurements	797

9.8.1	Introduction	797

9.8.2	MBSFN RSRP	797

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	797

9.8.2.2	MBSFN RSRP measurement report mapping	798

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	798

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	799

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	799

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	799

9.8.3	MBSFN RSRQ	800

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	800

9.8.3.2	MBSFN RSRQ measurement report mapping	800

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	801

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	801

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	801

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	802

9.8.4	MCH BLER	802

9.8.4.1	Measurement report mapping for MCH BLER	802

9.8.4.2	Measurement report mapping for MCH Block Number	803

9.9	ProSe Measurements	804

9.9.1	Introduction	804

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	804

9.9.2.1	Absolute S-RSRP Accuracy	804

9.9.2.2	Relative Accuracy of S-RSRP	805

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	806

9.9.3.1	Absolute SD-RSRP Accuracy	806

9.9.3.2	Relative Accuracy of SD-RSRP	806

9.10	V2X Measurements	807

9.10.1	Introduction	807

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	807

9.10.2.1	Absolute S-RSRP Accuracy	807

9.10.2.2	Relative Accuracy of S-RSRP	808

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	808

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	808

9.10.4	S-RSSI Measurement Accuracy Requirements	809

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	809

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	809

9.11	NR Measurements	810

9.11.1	NR SS-RSRP Measurements	810

9.11.2	NR SS-RSRQ Measurements	810

9.11.3	NR SS-SINR Measurements	811

10	Measurements Performance Requirements for E-UTRAN	811

10.1	Received Interference Power	811

10.1.1	Absolute accuracy requirement	811

10.1.2	Relative accuracy requirement	812

10.1.3	Received Interference Power measurement report mapping	812

10.2	Angle of Arrival (AOA)	812

10.2.1	Range/mapping	812

10.3	Timing Advance (TADV)	812

10.3.1	Report mapping	812

11	ProSe Requirements in Any Cell Selection state	813

11.1	Introduction	813

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	813

11.2.1	Introduction	813

11.2.2	ProSe UE transmission timing	813

11.3	Initiation/Cease of SLSS Transmissions	814

11.3.1	Introduction	814

11.3.2	Requirements	814

11.4	Measurements for ProSe in Any Cell Selection State	814

11.4.1	Introduction	814

11.4.2	Requirements	814

11.4.2.1	E-UTRA FDD	814

11.4.2.2	E-UTRA TDD	815

11.5	Selection / Reselection of ProSe Synchronization Reference	815

11.5.1	Introduction	815

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	815

11.5.2.1	Introduction	815

11.5.2.2	Requirements	815

11.6	Void	816

11.7	Selection / Reselection of ProSe relay UE	816

11.7.1	Introduction	816

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	816

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	816

12.1	Introduction	816

12.2	Transmit Timing	817

12.2.1	GNSS as timing reference	817

12.3	Interruption	817

12.4	Reliability of GNSS signal	817

13	V2X Requirements	817

13.1	Introduction	817

13.2	UE Transmit Timing	818

13.2.1	Introduction	818

13.2.2	GNSS as synchronization reference source	818

13.2.3	Serving cell/PCell as synchronization reference source	818

13.2.4	SyncRef UE as synchronization reference source	818

13.3	Initiation/Cease of SLSS Transmissions	818

13.3.1	Introduction	818

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	818

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	819

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	819

13.4	Selection / Reselection of V2X Synchronization Reference Source	820

13.5	Autonomous Resource Selection/Reselection measurements	821

13.5.1	Introduction	821

13.5.2	PSSCH-RSRP measurements	821

13.5.3	S-RSSI measurements	821

13.6	Congestion Control measurements	821

13.7	Interruption	821

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	821

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	821

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	822

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	822

13.8	Reliability of GNSS signal	822

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	823

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	823

Annex A (normative): Test Cases	818

A.1	Purpose of annex	818

A.2	Requirement classification for statistical testing	818

A.2.1	Types of requirements in TS 36.133	818

A.2.1.1	Time and delay requirements on UE higher layer actions	818

A.2.1.2	Measurements of power levels, relative powers and time	818

A.2.1.3	Implementation requirements	819

A.2.1.4	Physical layer timing requirements	819

A.3	RRM test configurations	820

A.3.1	Reference Measurement Channels	820

A.3.1.1	PDSCH	820

A.3.1.1.1	FDD	820

A.3.1.1.2	TDD	825

A.3.1.1.3	FDD for UE category 0	828

A.3.1.1.4	HD-FDD for UE category 0	829

A.3.1.1.5	TDD for UE category 0	830

A.3.1.1.6	Frame Structure 3	831

A.3.1.2	PCFICH/PDCCH/PHICH	832

A.3.1.2.1	FDD	832

A.3.1.2.2	TDD	832

A.3.1.2.3	HD-FDD for UE category 0	833

A.3.1.2.4	FS 3	833

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	833

A.3.1.3.1	FDD in CEModeA	834

A.3.1.3.2	HD-FDD in CEModeA	834

A.3.1.3.3	TDD in CEModeA	835

A.3.1.3.4	FDD in CEModeB	835

A.3.1.3.5	HD-FDD in CEModeB	836

A.3.1.3.6	TDD in CEModeB	836

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	837

A.3.1.4.1	FDD in CEModeA	837

A.3.1.4.2	HD-FDD in CEModeA	838

A.3.1.4.3	TDD in CEModeA	839

A.3.1.4.4	FDD in CEModeB	840

A.3.1.4.5	 HD-FDD in CEModeB	841

A.3.1.4.6	TDD in CEModeB	841

A.3.1.5	NPDSCH Reference Channel for UE category NB1	842

A.3.1.5.1	HD-FDD in-band operation	842

A.3.1.5.2	Void	843

A.3.1.5.3	HD-FDD standalone operation	843

A.3.1.5.4	Void	844

A.3.1.5.5	HD-FDD guard band operation	844

A.3.1.5.6	Void	845

A.3.1.5.7	TDD in-band operation	845

A.3.1.5.8	TDD standalone operation	845

A.3.1.5.9	TDD guard band operation	846

A.3.1.6	NPDCCH Reference Channel for UE category NB1	846

A.3.1.6.1	In-band operation	846

A.3.1.6.2	Void	847

A.3.1.6.3	Standalone operation	847

A.3.1.6.4	Void	847

A.3.1.6.5	Guard band operation	847

A.3.1.6.6	Void	848

A.3.2	OFDMA Channel Noise Generator (OCNG)	848

A.3.2.1	OCNG Patterns for FDD	848

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	849

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	849

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	850

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	850

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	851

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	852

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	852

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	852

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	853

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	854

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	854

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	855

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	855

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	856

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	856

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	857

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	857

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	858

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	858

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	859

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	859

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	860

A.3.2.2	OCNG Patterns for TDD	860

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	861

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	861

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	862

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	862

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	863

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	864

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	865

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	866

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	866

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	867

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	867

A.3.2.3	OCNG Patterns for Narrowband IoT	868

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	869

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	870

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	870

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	871

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	872

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	873

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	874

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	875

A.3.2.4	OCNG Patterns for V2X sidelink	876

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	876

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	877

A.3.3	Reference DRX Configurations	877

A.3.4	ABS Transmission Configurations	878

A.3.4.1	Non-MBSFN ABS Transmission Configurations	878

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	878

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	878

A.3.4.2	MBSFN ABS Transmission Configurations	879

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	879

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	880

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	880

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	880

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	881

A.3.6.1	Introduction	881

A.3.7	Test Cases with Different Channel Bandwidths	881

A.3.7.1	Introduction	881

A.3.7.2	Principle of testing	881

A.3.8	Antenna Configuration	881

A.3.8.1	Antenna connection for 4 Rx capable UEs	881

A.3.8.1.1 Introduction	881

A.3.8.1.2 Principle of testing	881

A.3.8.1.2.1 Single carrier tests	881

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	882

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	883

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	883

A.3.8.2	Antenna connection for 8 Rx capable UEs	883

A.3.8.2.1	Introduction	883

A.3.8.2.2	Principle of testing	883

A.3.8.2.2.1	Single carrier tests	883

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	884

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	884

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	884

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	884

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	884

A.3.9.1	Introduction	884

A.3.9.2	Principle of testing	884

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	885

A.3.10.1	Introduction	885

A.3.10.2	Principle of testing	885

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	885

A.3.11.1	Introduction	885

A.3.11.2	Principle of Testing	885

A.3.12	Proximity-based Services	885

A.3.12.1	Introduction	885

A.3.12.2	Reference DRX configurations for ProSe tests	885

A.3.12.3	Test Cases with Different Channel Bandwidths	886

A.3.12.3.1	Introduction	886

A.3.12.3.2	Principle of testing	886

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	886

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	893

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	896

A.3.12.6.1	FDD	896

A.3.12.7	Reference measurement channels for ProSe Direct Communication	896

A.3.12.7.1	FDD	896

A.3.12.8	ProSe Receive Traffic Generator	897

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	897

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	897

A.3.13	Time Offset between Cells	898

A.3.13.1	Introduction	898

A.3.13.2	Definition	898

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	898

A.3.14.1	Introduction	898

A.3.14.2	Principle of testing	898

A.3.15	Dual connectivity test cases with different combination of duplex mode	898

A.3.15.1	Introduction	898

A.3.15.2	Principle of testing	898

A.3.16	Reference PRACH Configurations	899

A.3.17	Listen before talk model	899

A.3.17.1	Introduction	899

A.3.17.2	Definition	899

A.3.18	Reference NPRACH Configurations	900

A.3.19	Dual connectivity test cases with different bandwidth combinations	901

A.3.19.1	Introduction	901

A.3.19.2	Principle of testing	901

A.3.20	Category M1 UE Test Cases	902

A.3.20.1	Introduction	902

A.3.20.2	Principle of Cat-M1 UE Testing	902

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	903

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	904

A.3.21.1	Introduction	904

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	904

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	905

A.3.22	Category 1bis UE Test Cases	906

A.3.22.1	Introduction	906

A.3.22.2	Principle of Category 1bis UE Testing	906

A.3.23	Category NB2 UE Test Cases	910

A.3.23.1	Introduction	910

A.3.23.2	Principle of Category NB2 UE Testing	910

A.3.24	V2X sidelink communication	913

A.3.24.1	Introduction	913

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	914

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	918

A.3.25	Category M2 UE Test Cases	919

A.3.25.1	Introduction	919

A.3.25.2	Principle of Cat-M2 UE Testing	919

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	920

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	921

A.3.26.1	Introduction	921

A.3.26.2	Principle of testing	921

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	921

A.3.27.1	Introduction	921

A.3.27.2	Principle of INACTIVE cell re-selection Testing	921

A.4	E-UTRAN RRC_IDLE state	921

A.4.2	Cell Re-Selection	921

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	921

A.4.2.1.1	Test Purpose and Environment	921

A.4.2.1.2	Test Requirements	923

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	924

A.4.2.2.1	Test Purpose and Environment	924

A.4.2.2.2	Test Requirements	925

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	926

A.4.2.3.1	Test Purpose and Environment	926

A.4.2.3.2	Test Requirements	927

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	928

A.4.2.4.1	Test Purpose and Environment	928

A.4.2.4.2	Test Requirements	929

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	930

A.4.2.5.1	Test Purpose and Environment	930

A.4.2.5.2	Test Requirements	931

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	932

A.4.2.6.1	Test Purpose and Environment	932

A.4.2.6.2	Test Requirements	933

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	934

A.4.2.7.1	Test Purpose and Environment	934

A.4.2.7.2	Test Requirements	935

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	936

A.4.2.8.1	Test Purpose and Environment	936

A.4.2.8.2	Test Requirements	937

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	938

A.4.2.9.1	Test Purpose and Environment	938

A.4.2.9.2	Test Requirements	938

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	938

A.4.2.10.1	Test Purpose and Environment	938

A.4.2.10.2	Test Requirements	942

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	942

A.4.2.11.1	Test Purpose and Environment	942

A.4.2.11.2	Test Requirements	946

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	946

A.4.2.12.1	Test Purpose and Environment	946

A.4.2.12.2	Test Requirements	948

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	949

A.4.2.13.1	Test Purpose and Environment	949

A.4.2.13.2	Test Requirements	950

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	951

A.4.2.14.1	Test Purpose and Environment	951

A.4.2.14.2	Test Requirements	952

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	953

A.4.2.15.1	Test Purpose and Environment	953

A.4.2.15.2	Test Requirements	954

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	955

A.4.2.16.1	Test Purpose and Environment	955

A.4.2.16.2	Test Requirements	956

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	957

A.4.2.17.1	Test Purpose and Environment	957

A.4.2.17.2	Test Requirements	958

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	959

A.4.2.18.1	Test Purpose and Environment	959

A.4.2.18.2	Test Requirements	961

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	962

A.4.2.19.1	Test Purpose and Environment	962

A.4.2.19.2	Test Requirements	964

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	965

A.4.2.20.1	Test Purpose and Environment	965

A.4.2.20.2	Test Requirements	966

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	967

A.4.2.21.1	Test Purpose and Environment	967

A.4.2.21.2	Test Requirements	968

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	969

A.4.2.22.1	Test Purpose and Environment	969

A.4.2.22.2	Test Requirements	970

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	971

A.4.2.23.1	Test Purpose and Environment	971

A.4.2.23.2	Test Requirements	972

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	973

A.4.2.24.1	Test Purpose and Environment	973

A.4.2.24.2	Test Requirements	976

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	977

A.4.2.25.1	Test Purpose and Environment	977

A.4.2.25.2	Test Requirements	978

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	979

A.4.2.26.1	Test Purpose and Environment	979

A.4.2.26.2	Test Requirements	980

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	981

A.4.2.27.1	Test Purpose and Environment	981

A.4.2.27.2	Test Requirements	982

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	983

A.4.2.28.1	Test Purpose and Environment	983

A.4.2.28.2	Test Requirements	984

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	985

A.4.2.29.1	Test Purpose and Environment	985

A.4.2.29.2	Test Requirements	986

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	987

A.4.2.30.1	Test Purpose and Environment	987

A.4.2.30.2	Test Requirements	988

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	989

A.4.2.31.1	Test Purpose and Environment	989

A.4.2.31.2	Test Requirements	990

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	991

A.4.2.32.1	Test Purpose and Environment	991

A.4.2.32.2	Test Requirements	992

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	993

A.4.2.33.1	Test Purpose and Environment	993

A.4.2.33.2	Test Requirements	994

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	995

A.4.2.34.1	Test Purpose and Environment	995

A.4.2.34.2	Test Requirements	996

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	997

A.4.2.35.1	Test Purpose and Environment	997

A.4.2.35.2	Test Requirements	999

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1000

A.4.2.36.1	Test Purpose and Environment	1000

A.4.2.36.2	Test Requirements	1002

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1003

A.4.2.37.1	Test Purpose and Environment	1003

A.4.2.37.2	Test Requirements	1005

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1006

A.4.2.38.1	Test Purpose and Environment	1006

A.4.2.38.2	Test Requirements	1008

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1009

A.4.2.39.1	Test Purpose and Environment	1009

A.4.2.39.2	Test Requirements	1010

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1011

A.4.2.40.1	Test Purpose and Environment	1011

A.4.2.40.2	Test Requirements	1012

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1013

A.4.2.41.1	Test Purpose and Environment	1013

A.4.2.41.2	Test Requirements	1016

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1017

A.4.2.42.1	Test Purpose and Environment	1017

A.4.2.42.2	Test Requirements	1019

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1020

A.4.2.43.1	Test Purpose and Environment	1020

A.4.2.43.2	Test Requirements	1022

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1023

A.4.2.44.1	Test Purpose and Environment	1023

A.4.2.44.2	Test Requirements	1025

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1026

A.4.2.45.1	Test Purpose and Environment	1026

A.4.2.45.2	Test Requirements	1028

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1029

A.4.2.46.1	Test Purpose and Environment	1029

A.4.2.46.2	Test Requirements	1031

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1032

A.4.2.47.1	Test Purpose and Environment	1032

A.4.2.47.2	Test Requirements	1034

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1035

A.4.2.48.1	Test Purpose and Environment	1035

A.4.2.48.2	Test Requirements	1036

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1037

A.4.2.49.1	Test Purpose and Environment	1037

A.4.2.49.2	Test Requirements	1039

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1040

A.4.2.50.1	Test Purpose and Environment	1040

A.4.2.50.2	Test Requirements	1042

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1043

A.4.2.51.1	Test Purpose and Environment	1043

A.4.2.51.2	Test Requirements	1045

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1046

A.4.2.52.1	Test Purpose and Environment	1046

A.4.2.52.2	Test Requirements	1048

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1049

A.4.2.53.1	Test Purpose and Environment	1049

A.4.2.53.2	Test Requirements	1051

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1052

A.4.2.54.1	Test Purpose and Environment	1052

A.4.2.54.2	Test Requirements	1054

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1055

A.4.2.55.1	Test Purpose and Environment	1055

A.4.2.55.2	Test Requirements	1057

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1058

A.4.2.56.1	Test Purpose and Environment	1058

A.4.2.56.2	Test Requirements	1059

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1060

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1060

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1060

A.4.3.1.1.1	Test Purpose and Environment	1060

A.4.3.1.1.2	Test Requirements	1062

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1062

A.4.3.1.2.1	Test Purpose and Environment	1062

A.4.3.1.2.2	Test Requirements	1065

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1065

A.4.3.1.3.1	Test Purpose and Environment	1065

A.4.3.1.3.2	Test Requirements	1068

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1068

A.4.3.1.4.1	Test Purpose and Environment	1068

A.4.3.1.4.2	Test Requirements	1069

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1069

A.4.3.1.5.1	Test Purpose and Environment	1069

A.4.3.1.5.2	Test Requirements	1072

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1073

A.4.3.2.1	Test Purpose and Environment	1073

A.4.3.2.1.1	Void	1073

A.4.3.2.1.2	1.28Mcps TDD option	1073

A.4.3.2.1.3	Void	1075

A.4.3.2.2	Test Requirements	1075

A.4.3.2.2.1	1.28Mcps TDD option	1075

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1075

A.4.3.2A.1	Test Purpose and Environment	1075

A.4.3.2A.2	Test Requirements	1079

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1079

A.4.3.3.1	Test Purpose and Environment	1079

A.4.3.3.2	Test Requirements	1082

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1082

A.4.3.3A.1	Test Purpose and Environment	1082

A.4.3.3A.2	Test Requirements	1087

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1088

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1088

A.4.3.4.1.1	Test Purpose and Environment	1088

A.4.3.4.1.2	Test Requirements	1090

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1090

A.4.3.4.2.1	Test Purpose and Environment	1090

A.4.3.4.2.2	Test Requirements	1092

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1092

A.4.3.4.3.1	Test Purpose and Environment	1092

A.4.3.4.3.2	Test Requirements	1095

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1095

A.4.3.4.4.1	Test Purpose and Environment	1095

A.4.3.4.4.2	Test Requirements	1099

A.4.4	E-UTRAN to GSM Cell Re-Selection	1099

A.4.4.1	E-UTRAN FDD – GSM:	1099

A.4.4.1.1	Test Purpose and Environment	1099

A.4.4.1.2	Test Requirements	1101

A.4.4.2	E-UTRAN TDD – GSM:	1101

A.4.4.2.1	Test Purpose and Environment	1101

A.4.4.2.2	Test Requirements	1103

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1104

A.4.5.1	E-UTRAN FDD – HRPD	1104

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1104

A.4.5.1.1.1	Test Purpose and Environment	1104

A.4.5.1.1.2	Test Requirements	1106

A.4.5.2	E-UTRAN TDD – HRPD	1106

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1106

A.4.5.2.1.1	Test Purpose and Environment	1106

A.4.5.2.1.2	Test Requirements	1109

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1109

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1109

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1109

A.4.6.1.1.1	Test Purpose and Environment	1109

A.4.6.1.1.2	Test Requirements	1112

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1112

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1112

A.4.6.2.1.1	Test Purpose and Environment	1112

A.4.6.2.1.2	Test Requirements	1115

A.4.7	Idle State Positioning Measurement for UE category NB1	1115

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1115

A.4.7.1.1	Test Purpose and Environment	1115

A.4.7.1.2	Test Requirements	1120

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1121

A.4.7.2.1	Test Purpose and Environment	1121

A.4.7.4.2	Test Requirements	1125

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1126

A.4.7.3.1	Test Purpose and Environment	1126

A.4.7.3.2	Test Requirements	1130

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1131

A.4.7.4.1	Test Purpose and Environment	1131

A.4.7.4.2	Test Requirements	1135

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1136

A.5.1	E-UTRAN Handover	1136

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1136

A.5.1.1.1	Test Purpose and Environment	1136

A.5.1.1.2	Test Requirements	1138

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1138

A.5.1.2.1	Test Purpose and Environment	1138

A.5.1.2.2	Test Requirements	1140

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1140

A.5.1.3.1	Test Purpose and Environment	1140

A.5.1.3.2	Test Requirements	1142

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1142

A.5.1.4.1	Test Purpose and Environment	1142

A.5.1.4.2	Test Requirements	1144

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1144

A.5.1.5.1	Test Purpose and Environment	1144

A.5.1.5.2	Test Requirements	1146

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1146

A.5.1.6.1	Test Purpose and Environment	1146

A.5.1.6.2	Test Requirements	1148

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1148

A.5.1.7.1	Test Purpose and Environment	1148

A.5.1.7.2	Test Requirements	1151

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1151

A.5.1.8.1	Test Purpose and Environment	1151

A.5.1.8.2	Test Requirements	1154

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1154

A.5.1.9.1	Test Purpose and Environment	1154

A.5.1.9.2	Test Requirements	1155

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1155

A.5.1.10.1	Test Purpose and Environment	1155

A.5.1.10.2	Test Requirements	1157

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1157

A.5.1.11.1	Test Purpose and Environment	1157

A.5.1.11.2	Test Requirements	1159

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1159

A.5.1.12.1	Test Purpose and Environment	1159

A.5.1.12.2	Test Requirements	1161

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1161

A.5.1.13.1	Test Purpose and Environment	1161

A.5.1.13.2	Test Requirements	1163

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1164

A.5.1.14.1	Test Purpose and Environment	1164

A.5.1.14.2	Test Requirements	1165

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1166

A.5.1.15.1	Test Purpose and Environment	1166

A.5.1.15.2	Test Requirements	1167

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1168

A.5.1.16.1	Test Purpose and Environment	1168

A.5.1.16.2	Test Requirements	1169

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1170

A.5.1.17.1	Test Purpose and Environment	1170

A.5.1.17.2	Test Requirements	1171

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1172

A.5.1.18.1	Test Purpose and Environment	1172

A.5.1.18.2	Test Requirements	1173

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1174

A.5.1.19.1	Test Purpose and Environment	1174

A.5.1.19.2	Test Requirements	1175

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1176

A.5.1.20.1	Test Purpose and Environment	1176

A.5.1.20.2	Test Requirements	1177

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1178

A.5.1.21.1	Test Purpose and Environment	1178

A.5.1.21.2	Test Requirements	1179

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1179

A.5.1.22.1	Test Purpose and Environment	1179

A.5.1.22.2	Test Requirements	1181

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1181

A.5.1.23.1	Test Purpose and Environment	1181

A.5.1.23.2	Test Requirements	1183

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1183

A.5.1.24.1	Test Purpose and Environment	1183

A.5.1.24.2	Test Requirements	1185

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1185

A.5.1.25.1	Test Purpose and Environment	1185

A.5.1.25.2	Test Requirements	1187

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1188

A.5.1.26.1	Test Purpose and Environment	1188

A.5.1.26.2	Test Requirements	1189

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1190

A.5.1.27.1	Test Purpose and Environment	1190

A.5.1.27.2	Test Requirements	1191

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1192

A.5.1.28.1	Test Purpose and Environment	1192

A.5.1.28.2	Test Requirements	1193

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1194

A.5.1.29.1	Test Purpose and Environment	1194

A.5.1.29.2	Test Requirements	1195

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1196

A.5.1.30.1	Test Purpose and Environment	1196

A.5.1.30.2	Test Requirements	1197

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1198

A.5.1.31.1	Test Purpose and Environment	1198

A.5.1.31.2	Test Requirements	1199

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1200

A.5.1.32.1	Test Purpose and Environment	1200

A.5.1.32.2	Test Requirements	1201

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1202

A.5.1.33.1	Test Purpose and Environment	1202

A.5.1.13.2	Test Requirements	1203

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1204

A.5.1.34.1	Test Purpose and Environment	1204

A.5.1.34.2	Test Requirements	1205

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1206

A.5.1.35.1	Test Purpose and Environment	1206

A.5.1.35.2	Test Requirements	1207

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1208

A.5.1.36.1	Test Purpose and Environment	1208

A.5.1.36.2	Test Requirements	1209

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1210

A.5.1.37.1	Test Purpose and Environment	1210

A.5.1.37.2	Test Requirements	1211

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1212

A.5.1.38.1	Test Purpose and Environment	1212

A.5.1.38.2	Test Requirements	1213

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1214

A.5.1.39.1	Test Purpose and Environment	1214

A.5.1.39.2	Test Requirements	1215

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1216

A.5.1.40.1	Test Purpose and Environment	1216

A.5.1.40.2	Test Requirements	1217

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1218

A.5.1.41.1	Test Purpose and Environment	1218

A.5.1.41.2	Test Requirements	1220

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1221

A.5.1.42.1	Test Purpose and Environment	1221

A.5.1.42.2	Test Requirements	1222

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1223

A.5.1.43.1	Test Purpose and Environment	1223

A.5.1.43.2	Test Requirements	1224

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1225

A.5.1.44.1	Test Purpose and Environment	1225

A.5.1.44.2	Test Requirements	1226

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1227

A.5.1.45.1	Test Purpose and Environment	1227

A.5.1.45.1	Test Requirements	1228

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1229

A.5.1.46.1	Test Purpose and Environment	1229

A.5.1.46.2	Test Requirements	1230

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1231

A.5.1.47.1	Test Purpose and Environment	1231

A.5.1.47.2	Test Requirements	1232

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1233

A.5.1.48.1	Test Purpose and Environment	1233

A.5.1.48.2	Test Requirements	1234

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1235

A.5.1.49.1	Test Purpose and Environment	1235

A.5.1.49.2	Test Requirements	1236

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1237

A.5.1.50.1	Test Purpose and Environment	1237

A.5.1.50.2	Test Requirements	1238

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1239

A.5.1.51.1	Test Purpose and Environment	1239

A.5.1.51.2	Test Requirements	1241

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1241

A.5.152.1	Test Purpose and Environment	1241

A.5.1.52.2	Test Requirements	1244

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1245

A.5.1.53.1	Test Purpose and Environment	1245

A.5.1.53.2	Test Requirements	1247

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1248

A.5.1.54.1	Test Purpose and Environment	1248

A.5.1.54.2	Test Requirements	1250

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1251

A.5.1.55.1	Test Purpose and Environment	1251

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1255

A.5.1.56.1	Test Purpose and Environment	1255

A.5.1.56.2	Test Requirements	1258

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1259

A.5.1.57.1	Test Purpose and Environment	1259

A.5.1.57.2	Test Requirements	1261

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1261

A.5.1.58.1	Test Purpose and Environment	1261

A.5.1.58.2	Test Requirements	1264

A.5.2	E-UTRAN Handover to other RATs	1264

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1264

A.5.2.1.1	Test Purpose and Environment	1264

A.5.2.1.2	Test Requirements	1266

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1267

A.5.2.2.1	Test Purpose and Environment	1267

A.5.2.2.2	Test Requirements	1270

A.5.2.3 E-UTRAN FDD- GSM Handover	1270

A.5.2.3.1	Test Purpose and Environment	1270

A.5.2.3.2	Test Requirements	1271

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1272

A.5.2.4.1	Test Purpose and Environment	1272

A.5.2.4.1.1	Void	1272

A.5.2.4.1.2	1.28 Mcps TDD option	1272

A.5.2.4.1.3	Void	1274

A.5.2.4.2	Test Requirements	1274

A.5.2.4.2.1	Void	1274

A.5.2.4.2.2	1.28 Mcps TDD option	1274

A.5.2.4.2.3	Void	1274

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1274

A.5.2.5.1	Test Purpose and Environment	1274

A.5.2.5.1.3	Void	1277

A.5.2.5.2	Test Requirements	1277

A.5.2.5.2.1	Void	1277

A.5.2.5.2.2	1.28 Mcps TDD option	1277

A.5.2.5.2.3	Void	1277

A.5.2.6	E-UTRAN TDD - GSM Handover	1277

A.5.2.6.1	Test Purpose and Environment	1277

A.5.2.6.2	Test Requirements	1279

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1280

A.5.2.7.1	Test Purpose and Environment	1280

A.5.2.7.2	Test Requirements	1282

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1282

A.5.2.8.1	Test Purpose and Environment	1282

A.5.2.8.2	Test Requirements	1283

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1284

A.5.2.9.1	Test Purpose and Environment	1284

A.5.2.9.2	Test Requirements	1285

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1286

A.5.2.10.1	Test Purpose and Environment	1286

A.5.2.10.2	Test Requirements	1288

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1288

A.5.2.10A.1	Test Purpose and Environment	1288

A.5.2.10A.2	Test Requirements	1291

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1291

A.5.2.10B.1	Test Purpose and Environment	1291

A.5.2.10B.2	Test Requirements	1294

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1294

A.5.2.11.1	Test Purpose and Environment	1294

A.5.2.11.2	Test Requirements	1294

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1295

A.5.3.1	E-UTRAN FDD – HRPD Handover	1295

A.5.3.1.1	Test Purpose and Environment	1295

A.5.3.1.2	Test Requirements	1297

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1297

A.5.3.2.1	Test Purpose and Environment	1297

A.5.3.2.2	Test Requirements	1300

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1300

A.5.3.3.1	Test Purpose and Environment	1300

A.5.3.3.2	Test Requirements	1302

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1303

A.5.3.4.1	Test Purpose and Environment	1303

A.5.3.4.2	Test Requirements	1304

A.5.3.5	E-UTRAN TDD – HRPD Handover	1305

A.5.3.5.1	Test Purpose and Environment	1305

A.5.3.5.2	Test Requirements	1307

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1308

A.5.3.6.1	Test Purpose and Environment	1308

A.5.3.6.2	Test Requirements	1311

A.6	RRC Connection Control	1311

A.6.1	RRC Re-establishment	1311

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1311

A.6.1.1.1	Test Purpose and Environment	1311

A.6.1.1.2	Test Requirements	1312

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1313

A.6.1.2.1	Test Purpose and Environment	1313

A.6.1.2.2	Test Requirements	1314

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1315

A.6.1.3.1	Test Purpose and Environment	1315

A.6.1.3.2	Test Requirements	1316

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1317

A.6.1.4.1	Test Purpose and Environment	1317

A.6.1.4.2	Test Requirements	1318

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1319

A.6.1.5.1	Test Purpose and Environment	1319

A.6.1.5.2	Test Requirements	1319

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1319

A.6.1.6.1	Test Purpose and Environment	1319

A.6.1.6.2	Test Requirements	1321

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1322

A.6.1.7.1	Test Purpose and Environment	1322

A.6.1.7.2	Test Requirements	1323

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1324

A.6.1.8.1	Test Purpose and Environment	1324

A.6.1.8.2	Test Requirements	1325

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1326

A.6.1.9.1	Test Purpose and Environment	1326

A.6.1.9.2	Test Requirements	1327

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1328

A.6.1.10.1	Test Purpose and Environment	1328

A.6.1.10.2	Test Requirements	1329

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1330

A.6.1.11.1	Test Purpose and Environment	1330

A.6.1.11.2	Test Requirements	1331

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1332

A.6.1.12.1	Test Purpose and Environment	1332

A.6.1.12.2	Test Requirements	1333

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1334

A.6.1.13.1	Test Purpose and Environment	1334

A.6.1.13.2	Test Requirements	1335

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1336

A.6.1.14.1	Test Purpose and Environment	1336

A.6.1.14.2	Test Requirements	1337

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1338

A.6.1.15.1	Test Purpose and Environment	1338

A.6.1.15.2	Test Requirements	1340

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1341

A.6.1.16.1	Test Purpose and Environment	1341

A.6.1.16.2	Test Requirements	1343

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1344

A.6.1.17.1	Test Purpose and Environment	1344

A.6.1.17.2	Test Requirements	1345

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1346

A.6.1.18.1	Test Purpose and Environment	1346

A.6.1.18.2	Test Requirements	1347

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1348

A.6.1.19.1	Test Purpose and Environment	1348

A.6.1.19.2	Test Requirements	1349

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1350

A.6.1.20.1	Test Purpose and Environment	1350

A.6.1.20.2	Test Requirements	1351

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1352

A.6.1.21.1	Test Purpose and Environment	1352

A.6.1.21.2	Test Requirements	1353

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1354

A.6.1.22.1	Test Purpose and Environment	1354

A.6.1.22.2	Test Requirements	1355

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1356

A.6.1.23.1	Test Purpose and Environment	1356

A.6.1.23.2	Test Requirements	1358

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1359

A.6.1.24.1	Test Purpose and Environment	1359

A.6.1.24.2	Test Requirements	1361

A.6.2	Random Access	1362

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1362

A.6.2.1.1	Test Purpose and Environment	1362

A.6.2.1.2	Test Requirements	1363

A.6.2.1.2.1	Random Access Response Reception	1363

A.6.2.1.2.2	No Random Access Response Reception	1363

A.6.2.1.2.3	Receiving a NACK on msg3	1363

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1364

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1364

A.6.2.1.2.6	Contention Resolution Timer expiry	1364

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1364

A.6.2.2.1	Test Purpose and Environment	1364

A.6.2.2.2	Test Requirements	1365

A.6.2.2.2.1	Random Access Response Reception	1366

A.6.2.2.2.2	No Random Access Response Reception	1366

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1366

A.6.2.3.1	Test Purpose and Environment	1366

A.6.2.3.2	Test Requirements	1368

A.6.2.3.2.1	Random Access Response Reception	1368

A.6.2.3.2.2	No Random Access Response reception	1368

A.6.2.3.2.3	Receiving a NACK on msg3	1368

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1369

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1369

A.6.2.3.2.6	Contention Resolution Timer expiry	1369

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1369

A.6.2.4.1	Test Purpose and Environment	1369

A.6.2.4.2	Test Requirements	1371

A.6.2.4.2.1	Random Access Response Reception	1371

A.6.2.4.2.2	No Random Access Response Reception	1371

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1371

A.6.2.5.1	Test Purpose and Environment	1371

A.6.2.5.2	Test Requirements	1372

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1372

A.6.2.6.1	Test Purpose and Environment	1372

A.6.2.6.2	Test Requirements	1372

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1373

A.6.2.7.1	Test Purpose and Environment	1373

A.6.2.7.2	Test Requirements	1375

A.6.2.7.2.1	Random Access Response Reception	1375

A.6.2.7.2.2	No Random Access Response Reception	1375

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1375

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1376

A.6.2.8.1	Test Purpose and Environment	1376

A.6.2.8.2	Test Requirements	1378

A.6.2.8.2.1	Random Access Response Reception	1378

A.6.2.8.2.2	No Random Access Response Reception	1378

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1378

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1379

A.6.2.9.1	Test Purpose and Environment	1379

A.6.2.9.2	Test Requirements	1382

A.6.2.9.2.1	Random Access Response Reception	1382

A.6.2.9.2.2	No Random Access Response Reception	1383

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1383

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1384

A.6.2.10.1	Test Purpose and Environment	1384

A.6.2.10.2	Test Requirements	1386

A.6.2.10.2.1	Random Access Response Reception	1386

A.6.2.10.2.2	No Random Access Response Reception	1387

A.6.2.10.2.3	Receiving a NACK on msg3	1387

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1387

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1387

A.6.2.10.2.6	Contention Resolution Timer expiry	1387

A.6.2.10.2.7	PRACH Resource Selection	1388

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1388

A.6.2.11.1	Test Purpose and Environment	1388

A.6.2.11.2	Test Requirements	1390

A.6.2.11.2.1	Random Access Response Reception	1390

A.6.2.11.2.2	No Random Access Response Reception	1391

A.6.2.11.2.3	Receiving a NACK on msg3	1391

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1391

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1391

A.6.2.11.2.6	Contention Resolution Timer expiry	1391

A.6.2.11.2.7	PRACH Resource Selection	1392

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1392

A.6.2.12.1	Test Purpose and Environment	1392

A.6.2.12.2	Test Requirements	1394

A.6.2.12.2.1	Random Access Response Reception	1394

A.6.2.12.2.2	No Random Access Response Reception	1395

A.6.2.12.2.3	Receiving a NACK on msg3	1395

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1395

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1395

A.6.2.12.2.6	Contention Resolution Timer expiry	1395

A.6.2.12.2.7	PRACH Resource Selection	1396

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1396

A.6.2.13.1	Test Purpose and Environment	1396

A.6.2.13.2	Test Requirements	1398

A.6.2.13.2.1	Random Access Response Reception	1398

A.6.2.13.2.2	No Random Access Response Reception	1399

A.6.2.13.2.3	Receiving a NACK on msg3	1399

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1399

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1399

A.6.2.13.2.6	Contention Resolution Timer expiry	1399

A.6.2.13.2.7	PRACH Resource Selection	1400

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1400

A.6.2.14.1	Test Purpose and Environment	1400

A.6.2.14.2	Test Requirements	1402

A.6.2.14.2.1	Random Access Response Reception	1402

A.6.2.14.2.2	No Random Access Response Reception	1403

A.6.2.14.2.3	Receiving a NACK on msg3	1403

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1403

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1403

A.6.2.14.2.6	Contention Resolution Timer expiry	1403

A.6.2.14.2.7	PRACH Resource Selection	1404

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1404

A.6.2.15.1	Test Purpose and Environment	1404

A.6.2.15.2	Test Requirements	1406

A.6.2.15.2.1	Random Access Response Reception	1406

A.6.2.15.2.2	No Random Access Response Reception	1407

A.6.2.15.2.3	Receiving a NACK on msg3	1407

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1407

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1407

A.6.2.15.2.6	Contention Resolution Timer expiry	1407

A.6.2.15.2.7	PRACH Resource Selection	1408

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1408

A.6.2.16.1	Test Purpose and Environment	1408

A.6.2.16.2	Test Requirements	1411

A.6.2.16.2.1	Random Access Response Reception	1411

A.6.2.16.2.2	No Random Access Response Reception	1412

A.6.2.16.2.3	Receiving a NACK on msg3	1412

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1412

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1412

A.6.2.16.2.6	Contention Resolution Timer expiry	1412

A.6.2.16.2.7	NPRACH Resource Selection	1412

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1413

A.6.2.17.1	Test Purpose and Environment	1413

A.6.2.17.2	Test Requirements	1416

A.6.2.17.2.1	Random Access Response Reception	1416

A.6.2.17.2.2	No Random Access Response Reception	1417

A.6.2.17.2.3	Receiving a NACK on msg3	1417

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1417

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1417

A.6.2.17.2.6	Contention Resolution Timer expiry	1417

A.6.2.17.2.7	NPRACH Resource Selection	1417

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1418

A.6.2.18.1	Test Purpose and Environment	1418

A.6.2.18.2	Test Requirements	1421

A.6.2.18.2.1	Random Access Response Reception	1421

A.6.2.18.2.2	No Random Access Response Reception	1422

A.6.2.18.2.3	Receiving a NACK on msg3	1422

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1422

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1422

A.6.2.18.2.6	Contention Resolution Timer expiry	1422

A.6.2.18.2.7	NPRACH Resource Selection	1422

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1423

A.6.2.19.1	Test Purpose and Environment	1423

A.6.2.19.2	Test Requirements	1425

A.6.2.19.2.1	Random Access Response Reception	1425

A.6.2.19.2.2	No Random Access Response Reception	1426

A.6.2.19.2.3	Receiving a NACK on msg3	1426

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1426

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1426

A.6.2.19.2.6	Contention Resolution Timer expiry	1426

A.6.2.19.2.7	NPRACH Resource Selection	1426

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1427

A.6.2.20.1	Test Purpose and Environment	1427

A.6.2.20.2	Test Requirements	1429

A.6.2.20.2.1	Random Access Response Reception	1429

A.6.2.20.2.2	No Random Access Response Reception	1430

A.6.2.20.2.3	Receiving a NACK on msg3	1430

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1430

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1430

A.6.2.20.2.6	Contention Resolution Timer expiry	1430

A.6.2.20.2.7	NPRACH Resource Selection	1430

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1431

A.6.2.21.1	Test Purpose and Environment	1431

A.6.2.21.2	Test Requirements	1433

A.6.2.21.2.1	Random Access Response Reception	1433

A.6.2.21.2.2	No Random Access Response Reception	1434

A.6.2.21.2.3	Receiving a NACK on msg3	1434

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1434

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1434

A.6.2.21.2.6	Contention Resolution Timer expiry	1434

A.6.2.21.2.7	NPRACH Resource Selection	1434

A.6.3	RRC Connection Release with Redirection	1435

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1435

A.6.3.1.1	Test Purpose and Environment	1435

A.6.3.1.2	Test Requirements	1437

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1437

A.6.3.2.1	Test Purpose and Environment	1437

A.6.3.2.2	Test Requirements	1439

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1439

A.6.3.3.1	Test Purpose and Environment	1439

A.6.3.3.2	Test Requirements	1440

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1441

A.6.3.4.1	Test Purpose and Environment	1441

A.6.3.4.2	Test Requirements	1442

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1443

A.6.3.5.1	Test Purpose and Environment	1443

A.6.3.5.2	Test Requirements	1445

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1445

A.6.3.6.1	Test Purpose and Environment	1445

A. 6.3.6.2	Test Requirements	1448

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1448

A.6.3.7.1	Test Purpose and Environment	1448

A.6.3.7.2	Test Requirements	1451

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1451

A.6.3.8.1	Test Purpose and Environment	1451

A.6.3.8.2	Test Requirements	1454

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1454

A.6.3.9.1	Test Purpose and Environment	1454

A.6.3.9.2	Test Requirements	1456

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1456

A.6.3.10.1	Test Purpose and Environment	1456

A.6.3.10.2	Test Requirements	1458

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1458

A.6.3.11.1	Test Purpose and Environment	1458

A.6.3.11.2	Test Requirements	1460

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1460

A.6.3.12.1	Test Purpose and Environment	1460

A.6.3.12.2	Test Requirements	1463

A.7	Timing and Signalling Characteristics	1443

A.7.1	UE Transmit Timing	1443

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1443

A.7.1.1.1	Test Purpose and Environment	1443

A.7.1.1.2	Test Requirements	1444

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1445

A.7.1.2.1	Test Purpose and Environment	1445

A.7.1.2.2	Test Requirements	1447

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1448

A.7.1.3.1	Test Purpose and Environment	1448

A.7.1.3.2	Test Requirements	1450

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1451

A.7.1.4.1	Test Purpose and Environment	1451

A.7.1.4.2	Test Requirements	1452

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1453

A.7.1.4A.1	Test Purpose and Environment	1453

A.7.1.4A.2	Test Requirements	1453

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1453

A.7.1.5.1	Test Purpose and Environment	1453

A.7.1.5.2	Test Requirements	1454

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1454

A.7.1.6.1	Test Purpose and Environment	1454

A.7.1.6.2	Test Requirements	1456

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1457

A.7.1.7.1	Test Purpose and Environment	1457

A.7.1.7.2	Test Requirements	1458

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1459

A.7.1.7A.1	Test Purpose and Environment	1459

A.7.1.7A.2	Test Requirements	1459

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1459

A.7.1.7B.1	Test Purpose and Environment	1459

A.7.1.7B.2	Test Requirements	1459

A.7.1.8	Void	1460

A.7.1.8.1	Void	1460

A.7.1.8.2	Void	1460

A.7.1.9	Void	1460

A.7.1.9.1	Void	1460

A.7.1.9.2	Void	1460

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1460

A.7.1.10.1	Test Purpose and Environment	1460

A.7.1.10.2	Test Requirements	1462

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1462

A.7.1.11.1	Test Purpose and Environment	1462

A.7.1.11.2	Test Requirements	1464

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1464

A.7.1.12.1	Test Purpose and Environment	1464

A.7.1.12.2	Test Requirements	1466

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1466

A.7.1.13.1	Test Purpose and Environment	1466

A.7.1.13.2	Test Requirements	1468

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1469

A.7.1.14.1	Test Purpose and Environment	1469

A.7.1.14.2	Test Requirements	1470

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1470

A.7.1.15.1	Test Purpose and Environment	1470

A.7.1.15.2	Test Requirements	1471

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1472

A.7.1.16.1	Test Purpose and Environment	1472

A.7.1.16.2	Test Requirements	1473

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1474

A.7.1.17.1	Test Purpose and Environment	1474

A.7.1.17.2	Test Requirements	1476

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1477

A.7.1.18.1	Test Purpose and Environment	1477

A.7.1.18.2	Test Requirements	1479

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1480

A.7.1.19.1	Test Purpose and Environment	1480

A.7.1.19.2	Test Requirements	1481

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1481

A.7.1.20.1	Test Purpose and Environment	1481

A.7.1.20.2	Test Requirements	1483

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1483

A.7.1.21.1	Test Purpose and Environment	1483

A.7.1.21.2	Test Requirements	1485

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1485

A.7.1.22.1	Test Purpose and Environment	1485

A.7.1.22.2	Test Requirements	1487

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1487

A.7.1.23.1	Test Purpose and Environment	1487

A.7.1.23.2	Test Requirements	1489

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1489

A.7.1.24.1	Test Purpose and Environment	1489

A.7.1.24.2	Test Requirements	1490

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1491

A.7.1.25.1	Test Purpose and Environment	1491

A.7.1.25.2	Test Requirements	1492

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1493

A.7.1.26.1	Test Purpose and Environment	1493

A.7.1.26.2	Test Requirements	1494

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1495

A.7.1.27.1	Test Purpose and Environment	1495

A.7.1.27.2	Test Requirements	1497

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1497

A.7.1.28.1	Test Purpose and Environment	1497

A.7.1.28.2	Test Requirements	1499

A.7.2	UE Timing Advance	1500

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1500

A.7.2.1.1	Test Purpose and Environment	1500

A.7.2.1.2	Test Requirements	1501

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1502

A.7.2.2.1	Test Purpose and Environment	1502

A.7.2.2.2	Test Requirements	1504

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1504

A.7.2.3.1	Test Purpose and Environment	1504

A.7.2.3.2	Test Requirements	1504

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1504

A.7.2.4.1	Test Purpose and Environment	1504

A.7.2.4.2	Test Requirements	1507

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1507

A.7.2.5.1	Test Purpose and Environment	1507

A.7.2.5.2	Test Requirements	1509

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1509

A.7.2.5A.1	Test Purpose and Environment	1509

A.7.2.5A.2	Test Requirements	1510

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1510

A.7.2.5B.1	Test Purpose and Environment	1510

A.7.2.5B.2	Test Requirements	1510

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1510

A.7.2.6.1	Test Purpose and Environment	1510

A.7.2.6.2	Test Requirements	1513

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1513

A.7.2.7.1	Test Purpose and Environment	1513

A.7.2.7.2	Test Requirements	1515

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1515

A.7.2.8.1	Test Purpose and Environment	1515

A.7.2.8.2	Test Requirements	1517

A.7.2.9.2	Test Requirements	1518

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1519

A.7.2.10.1	Test Purpose and Environment	1519

A.7.2.10.2	Test Requirements	1520

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1520

A.7.2.11.1	Test Purpose and Environment	1520

A.7.2.11.2	Test Requirements	1522

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1522

A.7.2.12.1	Test Purpose and Environment	1522

A.7.2.12.2	Test Requirements	1524

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1524

A.7.2.13.1	Test Purpose and Environment	1524

A.7.2.13.2	Test Requirements	1526

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1526

A.7.2.14.1	Test Purpose and Environment	1526

A.7.2.14.2	Test Requirements	1528

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1528

A.7.2.15.1	Test Purpose and Environment	1528

A.7.2.15.2	Test Requirements	1530

A.7.3	Radio Link Monitoring	1530

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1531

A.7.3.1.1	Test Purpose and Environment	1531

A.7.3.1.2	Test Requirements	1535

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1535

A.7.3.2.1	Test Purpose and Environment	1535

A.7.3.2.2	Test Requirements	1538

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1538

A.7.3.3.1	Test Purpose and Environment	1538

A.7.3.3.2	Test Requirements	1542

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1542

A.7.3.4.1	Test Purpose and Environment	1542

A.7.3.4.2	Test Requirements	1545

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1545

A.7.3.5.1	Test Purpose and Environment	1545

A.7.3.5.2	Test Requirements	1549

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1550

A.7.3.6.1	Test Purpose and Environment	1550

A.7.3.6.2	Test Requirements	1553

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1553

A.7.3.7.1	Test Purpose and Environment	1553

A.7.3.7.2	Test Requirements	1556

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1556

A.7.3.8.1	Test Purpose and Environment	1556

A.7.3.8.2	Test Requirements	1559

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1559

A.7.3.9.2	Test Requirements	1562

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1562

A.7.3.10.1	Test Purpose and Environment	1562

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1565

A.7.3.11.1	Test Purpose and Environment	1565

A.7.3.11.2	Test Requirements	1569

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1569

A.7.3.12.1	Test Purpose and Environment	1569

A.7.3.12.2	Test Requirements	1573

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1573

A.7.3.13.1	Test Purpose and Environment	1573

A.7.3.13.2	Test Requirements	1576

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1576

A.7.3.14.1	Test Purpose and Environment	1576

A.7.3.14.2	Test Requirements	1579

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1579

A.7.3.15.1	Test Purpose and Environment	1579

A.7.3.15.2	Test Requirements	1582

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1582

A.7.3.16.1	Test Purpose and Environment	1582

A.7.3.16.2	Test Requirements	1586

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1586

A.7.3.17.1	Test Purpose and Environment	1586

A.7.3.17.2	Test Requirements	1589

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1589

A.7.3.18.1	Test Purpose and Environment	1589

A.7.3.18.2	Test Requirements	1593

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1593

A.7.3.19.1	Test Purpose and Environment	1593

A.7.3.19.2	Test Requirements	1597

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1597

A.7.3.20.1	Test Purpose and Environment	1597

A.7.3.20.2	Test Requirements	1601

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1601

A.7.3.21.1	Test Purpose and Environment	1601

A.7.3.21.2	Test Requirements	1605

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1605

A.7.3.22.1	Test Purpose and Environment	1605

A.7.3.22.2	Test Requirements	1609

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1609

A.7.3.23.1	Test Purpose and Environment	1609

A.7.3.23.2	Test Requirements	1610

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1610

A.7.3.24.1	Test Purpose and Environment	1610

A.7.3.24.2	Test Requirements	1611

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1611

A.7.3.25.1	Test Purpose and Environment	1611

A.7.3.25.2	Test Requirements	1612

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1612

A.7.3.26.1	Test Purpose and Environment	1612

A.7.3.26.2	Test Requirements	1615

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1615

A.7.3.27.1	Test Purpose and Environment	1615

A.7.3.27.2	Test Requirements	1618

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1618

A.7.3.28.1	Test Purpose and Environment	1618

A.7.3.28.2	Test Requirements	1621

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1621

A.7.3.29.1	Test Purpose and Environment	1621

A.7.3.29.2	Test Requirements	1624

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1624

A.7.3.30.1	Test Purpose and Environment	1624

A.7.3.30.2	Test Requirements	1627

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1627

A.7.3.31.1	Test Purpose and Environment	1627

A.7.3.31.2	Test Requirements	1630

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1630

A.7.3.32.1	Test Purpose and Environment	1630

A.7.3.32.2	Test Requirements	1633

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1633

A.7.3.33.1	Test Purpose and Environment	1633

A.7.3.33.2	Test Requirements	1636

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1636

A.7.3.34.1	Test Purpose and Environment	1636

A.7.3.34.2	Test Requirements	1639

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1639

A.7.3.35.1	Test Purpose and Environment	1639

A.7.3.35.2	Test Requirements	1642

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1642

A.7.3.36.1	Test Purpose and Environment	1642

A.7.3.36.2	Test Requirements	1645

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1645

A.7.3.37.1	Test Purpose and Environment	1645

A.7.3.37.2	Test Requirements	1648

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1648

A.7.3.38.1	Test Purpose and Environment	1648

A.7.3.38.2	Test Requirements	1651

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1651

A.7.3.39.1	Test Purpose and Environment	1651

A.7.3.39.2	Test Requirements	1654

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1655

A.7.3.40.1	Test Purpose and Environment	1655

A.7.3.40.2	Test Requirements	1657

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1658

A.7.3.41.1	Test Purpose and Environment	1658

A.7.3.41.2	Test Requirements	1661

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1661

A.7.3.42.1	Test Purpose and Environment	1661

A.7.3.42.2	Test Requirements	1664

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1664

A.7.3.43.1	Test Purpose and Environment	1664

A.7.3.43.2	Test Requirements	1667

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1667

A.7.3.44.1	Test Purpose and Environment	1667

A.7.3.44.2	Test Requirements	1670

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1671

A.7.3.45.1	Test Purpose and Environment	1671

A.7.3.45.2	Test Requirements	1673

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1674

A.7.3.46.1	Test Purpose and Environment	1674

A.7.3.46.2	Test Requirements	1677

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1677

A.7.3.47.1	Test Purpose and Environment	1677

A.7.3.47.2	Test Requirements	1680

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1680

A.7.3.48.1	Test Purpose and Environment	1680

A.7.3.48.2	Test Requirements	1683

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1683

A.7.3.49.1	Test Purpose and Environment	1683

A.7.3.49.2	Test Requirements	1686

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1686

A.7.3.50.1	Test Purpose and Environment	1686

A.7.3.50.2	Test Requirements	1689

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1689

A.7.3.51.1	Test Purpose and Environment	1689

A.7.3.51.2	Test Requirements	1692

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1692

A.7.3.52.1	Test Purpose and Environment	1692

A.7.3.52.2	Test Requirements	1695

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1695

A.7.3.53.1	Test Purpose and Environment	1695

A.7.3.53.2	Test Requirements	1698

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1698

A.7.3.54.1	Test Purpose and Environment	1698

A.7.3.54.2	Test Requirements	1701

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1701

A.7.3.55.1	Test Purpose and Environment	1701

A.7.3.55.2	Test Requirements	1704

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1704

A.7.3.56.1	Test Purpose and Environment	1704

A.7.3.56.2	Test Requirements	1707

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1707

A.7.3.57.1	Test Purpose and Environment	1707

A.7.3.57.2	Test Requirements	1710

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1710

A.7.3.58.1	Test Purpose and Environment	1710

A.7.3.58.2	Test Requirements	1713

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1713

A.7.3.59.1	Test Purpose and Environment	1713

A.7.3.59.2	Test Requirements	1716

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1716

A.7.3.60.1	Test Purpose and Environment	1716

A.7.3.60.2	Test Requirements	1720

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1720

A.7.3.61.1	Test Purpose and Environment	1720

A.7.3.61.2	Test Requirements	1724

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1724

A.7.3.62.1	Test Purpose and Environment	1724

A.7.3.62.2	Test Requirements	1728

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1728

A.7.3.63.1	Test Purpose and Environment	1728

A.7.3.63.2	Test Requirements	1732

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1732

A.7.3.64.1	Test Purpose and Environment	1732

A.7.3.64.2	Test Requirements	1735

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1736

A.7.3.65.1	Test Purpose and Environment	1736

A.7.3.65.2	Test Requirements	1739

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1740

A.7.3.66.1	Test Purpose and Environment	1740

A.7.3.66.2	Test Requirements	1742

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1743

A.7.3.67.1	Test Purpose and Environment	1743

A.7.3.67.2	Test Requirements	1746

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1747

A.7.3.68.1	Test Purpose and Environment	1747

A.7.3.68.2	Test Requirements	1749

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1749

A.7.3.69.1	Test Purpose and Environment	1749

A.7.3.69.2	Test Requirements	1751

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1751

A.7.3.70.1	Test Purpose and Environment	1751

A.7.3.70.2	Test Requirements	1753

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1753

A.7.3.71.1	Test Purpose and Environment	1753

A.7.3.71.2	Test Requirements	1756

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1756

A.7.3.72.1	Test Purpose and Environment	1756

A.7.3.72.2	Test Requirements	1758

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1758

A.7.3.73.1	Test Purpose and Environment	1758

A.7.3.73.2	Test Requirements	1760

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1760

A.7.3.74.1	Test Purpose and Environment	1760

A.7.3.74.2	Test Requirements	1763

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1763

A.7.3.75.1	Test Purpose and Environment	1763

A.7.3.75.2	Test Requirements	1765

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1766

A.7.3.76.1	Test Purpose and Environment	1766

A.7.3.76.2	Test Requirements	1769

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1769

A.7.3.77.1	Test Purpose and Environment	1769

A.7.3.77.2	Test Requirements	1772

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1772

A.7.3.78.1	Test Purpose and Environment	1772

A.7.3.78.2	Test Requirements	1775

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1775

A.7.3.79.1	Test Purpose and Environment	1775

A.7.3.79.2	Test Requirements	1778

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1778

A.7.3.80.1	Test Purpose and Environment	1778

A.7.3.80.2	Test Requirements	1781

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1781

A.7.3.81.1	Test Purpose and Environment	1781

A.7.3.81.2	Test Requirements	1784

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1784

A.7.3.82.1	Test Purpose and Environment	1784

A.7.3.82.2	Test Requirements	1787

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1787

A.7.3.83.1	Test Purpose and Environment	1787

A.7.3.83.2	Test Requirements	1788

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1789

A.7.3.84.1	Test Purpose and Environment	1789

A.7.3.84.2	Test Requirements	1791

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1791

A.7.3.85.1	Test Purpose and Environment	1791

A.7.3.85.2	Test Requirements	1792

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1793

A.7.3.86.1	Test Purpose and Environment	1793

A.7.3.86.2	Test Requirements	1795

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1795

A.7.3.87.1	Test Purpose and Environment	1795

A.7.3.87.2	Test Requirements	1797

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1798

A.7.3.88.1	Test Purpose and Environment	1798

A.7.3.88.2	Test Requirements	1802

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1802

A.7.3.89.1	Test Purpose and Environment	1802

A.7.3.89.2	Test Requirements	1806

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1806

A.7.3.90.1	Test Purpose and Environment	1806

A.7.3.90.2	Test Requirements	1810

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1810

A.7.3.91.1	Test Purpose and Environment	1810

A.7.3.91.2	Test Requirements	1814

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1814

A.7.3.92.1	Test Purpose and Environment	1814

A.7.3.92.2	Test Requirements	1817

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1818

A.7.3.93.1	Test Purpose and Environment	1818

A.7.3.93.2	Test Requirements	1821

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1822

A.7.3.94.1	Test Purpose and Environment	1822

A.7.3.94.2	Test Requirements	1824

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1825

A.7.3.95.1	Test Purpose and Environment	1825

A.7.3.95.2	Test Requirements	1828

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1828

A.7.3.96.1	Test Purpose and Environment	1828

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1830

A.7.3.97.1	Test Purpose and Environment	1830

A.7.3.97.2	Test Requirements	1833

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1833

A.7.3.98.1	Test Purpose and Environment	1833

A.7.3.98.2	Test Requirements	1836

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1836

A.7.3.99.1	Test Purpose and Environment	1836

A.7.3.99.2	Test Requirements	1839

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1839

A.7.3.100.1	Test Purpose and Environment	1839

A.7.3.100.2	Test Requirements	1841

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1841

A.7.3.101.1	Test Purpose and Environment	1841

A.7.3.101.2	Test Requirements	1843

A.7.4	Interruption for Dual Connectivity	1843

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1843

A.7.4.1.1	Test Purpose and Environment	1843

A.7.4.1.2	Test Requirements	1846

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1846

A.7.4.2.1	Test Purpose and Environment	1846

A.7.4.2.2	Test Requirements	1848

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1848

A.7.4.3.1	Test Purpose and Environment	1848

A.7.4.3.2	Test Requirements	1850

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1850

A.7.4.4.1	Test Purpose and Environment	1850

A.7.4.4.2	Test Requirements	1853

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1853

A.7.4.5.1	Test Purpose and Environment	1853

A.7.4.5.2	Test Requirements	1855

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1855

A.7.4.6.1	Test Purpose and Environment	1855

A.7.4.6.2	Test Requirements	1857

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1857

A.7.4.7.1	Test Purpose and Environment	1857

A.7.4.7.2	Test Requirements	1860

A.7.5	Proximity-based Services	1860

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1860

A.7.5.1.1	Test Purpose and Environment	1860

A.7.5.1.2	Test Requirements	1861

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1862

A.7.5.2.1	Test Purpose and Environment	1862

A.7.5.1.2	Test Requirements	1862

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1863

A.7.5.3.1	Test Purpose and Environment	1863

A.7.5.3.2	Test Requirements	1864

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1865

A.7.5.4.1	Test Purpose and Environment	1865

A.7.5.4.2	Test Requirements	1866

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1867

A.7.5.5.1	Test Purpose and Environment	1867

A.7.5.5.2	Test Requirements	1869

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1870

A.7.5.6.1	Test Purpose and Environment	1870

A.7.5.6.2	Test Requirements	1871

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1872

A.7.5.7.1	Test Purpose and Environment	1872

A.7.5.7.2	Test Requirements	1874

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1874

A.7.5.8.1	Test Purpose and Environment	1874

A.7.5.8.2	Test Requirements	1876

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1877

A.7.5.9.1	Test Purpose and Environment	1877

A.7.5.9.2	Test Requirements	1879

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1880

A.7.5.10.1	Test Purpose and Environment	1880

A.7.5.10.2	Test Requirements	1882

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1883

A.7.5.11.1	Test Purpose and Environment	1883

A.7.5.11.2	Test Requirements	1885

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1885

A.7.5.12.1	Test Purpose and Environment	1885

A.7.5.12.2	Test Requirements	1889

A.7.6	Interruption for carrier aggregation	1890

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1890

A.7.6.1.1	Test Purpose and Environment	1890

A.7.6.1.2	Test Requirements	1892

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1892

A.7.6.2.1	Test Purpose and Environment	1892

A.7.6.2.2	Test Requirements	1895

A.8	UE Measurements Procedures	1895

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1895

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1895

A.8.1.1.1	Test Purpose and Environment	1895

A.8.1.1.2	Test Requirements	1897

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1897

A.8.1.2.1	Test Purpose and Environment	1897

A.8.1.2.2	Test Requirements	1899

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1899

A.8.1.3.1	Test Purpose and Environment	1899

A.8.1.3.2	Test Requirements	1902

A.8.1.4	Void	1902

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1902

A.8.1.5.1	Test Purpose and Environment	1902

A.8.1.5.2	Test Requirements	1904

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1904

A.8.1.6.1	Test Purpose and Environment	1904

A.8.1.6.2	Test Requirements	1906

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1906

A.8.1.7.1	Test Purpose and Environment	1906

A.8.1.7.2	Test Requirements	1908

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1909

A.8.1.8.1	Test Purpose and Environment	1909

A.8.1.8.2	Test Requirements	1911

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1912

A.8.1.9.1	Test Purpose and Environment	1912

A.8.1.9.2	Test Requirements	1912

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1912

A.8.1.10.1	Test Purpose and Environment	1912

A.8.1.10.2	Test Requirements	1913

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1913

A.8.1.11.1	Test Purpose and Environment	1913

A.8.1.11.2	Test Requirements	1915

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1916

A.8.1.12.1	Test Purpose and Environment	1916

A.8.1.12.2	Test Requirements	1917

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1918

A.8.1.13.1	Test Purpose and Environment	1918

A.8.1.13.2	Test Requirements	1920

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1920

A.8.1.14.1	Test Purpose and Environment	1920

A.8.1.14.2	Test Requirements	1922

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1923

A.8.1.15.1	Test Purpose and Environment	1923

A.8.1.15.2	Test Requirements	1924

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1925

A.8.1.16.1	Test Purpose and Environment	1925

A.8.1.16.2	Test Requirements	1927

A.8.1.17	Void	1927

A.8.1.18	Void	1927

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1927

A.8.1.19.1	Test Purpose and Environment	1927

A.8.1.19.2	Test Requirements	1929

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1930

A.8.1.20.1	Test Purpose and Environment	1930

A.8.1.20.2	Test Requirements	1932

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1932

A.8.1.21.1	Test Purpose and Environment	1932

A.8.1.21.2	Test Requirements	1934

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1935

A.8.1.22.1	Test Purpose and Environment	1935

A.8.1.22.2	Test Requirements	1937

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1937

A.8.1.23.1	Test Purpose and Environment	1937

A.8.1.23.2	Test Requirements	1938

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1939

A.8.1.24.1	Test Purpose and Environment	1939

A.8.1.24.2	Test Requirements	1941

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1941

A.8.1.25.1	Test Purpose and Environment	1941

A.8.1.25.2	Test Requirements	1943

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1943

A.8.1.26.1	Test Purpose and Environment	1943

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1946

A.8.1.27.1	Test Purpose and Environment	1946

A.8.1.27.2	Test Requirements	1948

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1948

A.8.1.28.1	Test Purpose and Environment	1948

A.8.1.28.2	Test Requirements	1950

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1950

A.8.1.29.1	Test Purpose and Environment	1950

A.8.1.29.2	Test Requirements	1952

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1953

A.8.1.30.1	Test Purpose and Environment	1953

A.8.1.30.2	Test Requirements	1955

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1955

A.8.1.31.1	Test Purpose and Environment	1955

A.8.1.31.2	Test Requirements	1957

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1958

A.8.1.32.1	Test Purpose and Environment	1958

A.8.1.32.2	Test Requirements	1959

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1960

A.8.1.33.1	Test Purpose and Environment	1960

A.8.1.33.2	Test Requirements	1961

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1962

A.8.1.34.1	Test Purpose and Environment	1962

A.8.1.34.2	Test Requirements	1963

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1964

A.8.1.35.1	Test Purpose and Environment	1964

A.8.1.35.2	Test Requirements	1965

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1966

A.8.1.36.1	Test Purpose and Environment	1966

A.8.1.36.2	Test Requirements	1967

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1968

A.8.1.37.1	Test Purpose and Environment	1968

A.8.1.37.2	Test Requirements	1970

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1970

A.8.1.38.1	Test Purpose and Environment	1970

A.8.1.38.2	Test Requirements	1971

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1972

A.8.1.39.1	Test Purpose and Environment	1972

A.8.1.39.2	Test Requirements	1974

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1974

A.8.1.40.1	Test Purpose and Environment	1974

A.8.1.40.2	Test Requirements	1976

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1976

A.8.1.41.1	Test Purpose and Environment	1976

A.8.1.41.2	Test Requirement	1978

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1978

A.8.1.42.1	Test Purpose and Environment	1978

A.8.1.42.2	Test Requirement	1979

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1979

A.8.1.43.1	Test Purpose and Environment	1979

A.8.1.43.2	Test Requirements	1982

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1982

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1982

A.8.2.1.1	Test Purpose and Environment	1982

A.8.2.1.2	Test Requirements	1984

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1985

A.8.2.2.1	Test Purpose and Environment	1985

A.8.2.2.2	Test Requirements	1987

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1987

A.8.2.3.1	Test Purpose and Environment	1987

A.8.2.3.2	Test Requirements	1989

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1990

A.8.2.4.1	Test Purpose and Environment	1990

A.8.2.4.2	Test Requirements	1992

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1992

A.8.2.5.1	Test Purpose and Environment	1992

A.8.2.5.2	Test Requirements	1994

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1995

A.8.2.6.1	Test Purpose and Environment	1995

A.8.2.6.2	Test Requirements	1997

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1998

A.8.2.7.1	Test Purpose and Environment	1998

A.8.2.7.2	Test Requirements	1999

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2000

A.8.2.8.1	Test Purpose and Environment	2000

A.8.2.8.2	Test Requirements	2002

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2002

A.8.2.9.1	Test Purpose and Environment	2002

A.8.2.9.2	Test Requirements	2004

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2005

A.8.2.10.1	Test Purpose and Environment	2005

A.8.2.10.2	Test Requirements	2007

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2007

A.8.2.11.1	Test Purpose and Environment	2007

A.8.2.11.2	Test Requirements	2009

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2009

A.8.2.12.1	Test Purpose and Environment	2009

A.8.2.12.2	Test Requirements	2011

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2012

A.8.2.13.1	Test Purpose and Environment	2012

A.8.2.13.2	Test Requirements	2014

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2014

A.8.2.14.1	Test Purpose and Environment	2014

A.8.2.14.2	Test Requirement	2016

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2016

A.8.2.15.1	Test Purpose and Environment	2016

A.8.2.15.2	Test Requirements	2018

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2018

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2018

A.8.3.1.1	Test Purpose and Environment	2018

A.8.3.1.2	Test Requirements	2020

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2020

A.8.3.2.1	Test Purpose and Environment	2020

A.8.3.2.2	Test Requirements	2023

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2023

A.8.3.3.1	Test Purpose and Environment	2023

A.8.3.3.2	Test Requirements	2026

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2026

A.8.3.4.1	Test Purpose and Environment	2026

A.8.3.4.2	Test Requirements	2028

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2028

A.8.3.5.2	Test Requirements	2030

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2030

A.8.3.6.1	Test Purpose and Environment	2030

A.8.3.6.2	Test Requirements	2032

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2033

A.8.3.7.1	Test Purpose and Environment	2033

A.8.3.7.2	Test Requirements	2035

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2036

A.8.3.8.1	Test Purpose and Environment	2036

A.8.3.8.2	Test Requirements	2039

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2039

A.8.3.9.1	Test Purpose and Environment	2039

A.8.3.9.2	Test Requirements	2042

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2043

A.8.3.10.1	Test Purpose and Environment	2043

A.8.3.10.2	Test Requirements	2044

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2044

A.8.3.11.1	Test Purpose and Environment	2044

A.8.3.11.2	Test Requirement	2046

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2047

A.8.3.12.1	Test Purpose and Environment	2047

A.8.3.12.2	Test Requirement	2048

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2049

A.8.3.13.1	Test Purpose and Environment	2049

A.8.3.13.2	Test Requirement	2050

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2051

A.8.3.14.1	Test Purpose and Environment	2051

A.8.3.14.2	Test Requirement	2052

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2053

A.8.3.15.1	Test Purpose and Environment	2053

A.8.3.15.2	Test Requirement	2054

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2055

A.8.3.16.1	Test Purpose and Environment	2055

A.8.3.16.2	Test Requirement	2057

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2057

A.8.3.17.1	Test Purpose and Environment	2057

A.8.3.17.2	Test Requirement	2060

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2060

A.8.3.18.1	Test Purpose and Environment	2060

A.8.3.18.2	Test Requirement	2063

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2063

A.8.3.19.1	Test Purpose and Environment	2063

A.8.3.19.2	Test Requirement	2066

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2066

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2066

A.8.4.1.1	Test Purpose and Environment	2066

A.8.4.1.2	Test Requirements	2068

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2068

A.8.4.2.1	Test Purpose and Environment	2068

A.8.4.2.2	Test Requirements	2071

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2071

A.8.4.3.1	Test Purpose and Environment	2071

A.8.4.3.2	Test Requirements	2073

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2074

A.8.4.4.1	Test Purpose and Environment	2074

A.8.4.4.2	Test Requirements	2076

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2077

A.8.4.5.1	Test Purpose and Environment	2077

A.8.4.5.2	Test Requirements	2079

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2079

A.8.4.6.1	Test Purpose and Environment	2079

A.8.4.6.2	Test Requirements	2080

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2080

A.8.4.7.1	Test Purpose and Environment	2080

A.8.4.7.2	Test Requirements	2084

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2084

A.8.4.8.1	Test Purpose and Environment	2084

A.8.4.8.2	Test Requirements	2087

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2087

A.8.4.9.1	Test Purpose and Environment	2087

A.8.4.9.2	Test Requirements	2091

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2091

A.8.4.10.1	Test Purpose and Environment	2091

A.8.4.10.2	Test Requirements	2093

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2093

A.8.4.11.1	Test Purpose and Environment	2093

A.8.4.11.2	Test Requirement	2095

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2095

A.8.4.12.1	Test Purpose and Environment	2095

A.8.4.12.2	Test Requirement	2097

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2098

A.8.4.13.1	Test Purpose and Environment	2098

A.8.4.13.2	Test Requirement	2099

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2100

A.8.4.14.1	Test Purpose and Environment	2100

A.8.4.14.2	Test Requirement	2102

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2102

A.8.4.15.1	Test Purpose and Environment	2102

A.8.4.15.2	Test Requirement	2105

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2105

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2105

A.8.5.1.1	Test Purpose and Environment	2105

A.8.5.1.2	Test Requirements	2107

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2107

A.8.5.2.1	Test Purpose and Environment	2107

A.8.5.2.2	Test Requirements	2109

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2109

A.8.5.3.1	Test Purpose and Environment	2109

A.8.5.3.2	Test Requirements	2112

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2112

A.8.5.4.1	Test Purpose and Environment	2112

A.8.5.4.2	Test Requirements	2114

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2114

A.8.5.5.1	Test Purpose and Environment	2114

A.8.5.5.2	Test Requirements	2117

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2117

A.8.5.6.1	Test Purpose and Environment	2117

A.8.5.6.2	Test Requirements	2118

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2119

A.8.5.7.1	Test Purpose and Environment	2119

A.8.5.7.2	Test Requirements	2119

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2119

A.8.5.8.1	Test Purpose and Environment	2119

A.8.5.8.2	Test Requirements	2122

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2122

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2122

A.8.6.1.1	Test Purpose and Environment	2122

A.8.6.1.2	Test Requirements	2124

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2124

A.8.6.2.1	Test Purpose and Environment	2124

A.8.6.2.2	Test Requirements	2127

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2127

A.8.6.3.1	Test Purpose and Environment	2127

A.8.6.3.2	Test Requirements	2130

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2130

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2130

A.8.7.1.1	Test Purpose and Environment	2130

A.8.7.1.1.1	Void	2130

A.8.7.1.1.2	1.28 Mcps TDD option	2130

A.8.7.1.1.3	Void	2132

A.8.7.1.2	Test Requirements	2132

A.8.7.1.2.1	Void	2132

A.8.7.1.2.2	1.28 Mcps TDD option	2132

A.8.7.1.2.3	Void	2132

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2132

A.8.7.2.1	Test Purpose and Environment	2132

A.8.7.2.2	Test Requirements	2135

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2136

A.8.7.3.1	Test Purpose and Environment	2136

A.8.7.3.2	Test Parameters	2136

A.8.7.3.3	Test Requirements	2137

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2138

A.8.7.4.1	Test Purpose and Environment	2138

A.8.7.4.2	Test Requirements	2140

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2140

A.8.7.5.1	Test Purpose and Environment	2140

A.8.7.5.2	Test Requirements	2142

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2142

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2142

A.8.7A.1.1	Test Purpose and Environment	2142

A.8.7A.1.2	Test Requirements	2145

A.8.8	E-UTRAN FDD – GSM Measurements	2145

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2145

A.8.8.1.1	Test Purpose and Environment	2145

A.8.8.1.2	Test Requirements	2147

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2147

A.8.8.2.1	Test Purpose and Environment	2147

A.8.8.2.2	Test Requirements	2149

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2150

A.8.8.3.1	Test Purpose and Environment	2150

A.8.8.3.2	Test Requirements	2151

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2152

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2152

A.8.9.1.1	Test Purpose and Environment	2152

A.8.9.1.2	Test Requirements	2153

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2154

A.8.9.2.1	Test Purpose and Environment	2154

A.8.9.2.2	Test Requirements	2156

A.8.10	E-UTRAN TDD – GSM Measurements	2156

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2156

A.8.10.1.1	Test Purpose and Environment	2156

A.8.10.1.2	Test Requirements	2157

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2158

A.8.10.2.1	Test Purpose and Environment	2158

A.8.10.2.2	Test Requirements	2160

A.8.11	Monitoring of Multiple Layers	2160

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2160

A.8.11.1.1	Test Purpose and Environment	2160

A.8.11.1.2	Test Requirements	2162

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2163

A.8.11.2.1	Test Purpose and Environment	2163

A.8.11.2.2	Test Requirements	2164

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2165

A.8.11.3.1	Test Purpose and Environment	2165

A.8.11.3.2	Test Requirements	2167

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2167

A.8.11.4.1	Test Purpose and Environment	2167

A.8.11.4.2	Test Requirements	2170

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2170

A.8.11.5.1	Test Purpose and Environment	2170

A.8.11.5.2	Test Requirements	2172

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2173

A.8.11.6.1	Test Purpose and Environment	2173

A.8.11.6.2	Test Requirements	2175

A.8.12	RSTD Intra-frequency Measurements	2176

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2176

A.8.12.1.1	Test Purpose and Environment	2176

A.8.12.1.2	Test Requirements	2179

A.8.12.1.2A	Test Requirements for UE Category 1bis	2180

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2180

A.8.12.2.1	Test Purpose and Environment	2180

A.8.12.2.2	Test Requirements	2184

A.8.12.2.2A	Test Requirements for UE Category 1bis	2185

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2185

A.8.12.3.1	Test Purpose and Environment	2185

A.8.12.3.2	Test Requirements	2189

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2190

A.8.12.4.1	Test Purpose and Environment	2190

A.8.12.4.2	Test Requirements	2193

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2194

A.8.12.5.1	Test Purpose and Environment	2194

A.8.12.5.2	Test Requirements	2197

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2198

A.8.12.6.1	Test Purpose and Environment	2198

A.8.12.6.2	Test Requirements	2201

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2202

A.8.12.7.1	Test Purpose and Environment	2202

A.8.12.7.2	Test Requirements	2205

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2206

A.8.12.8.1	Test Purpose and Environment	2206

A.8.12.8.2	Test Requirements	2209

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2210

A.8.12.9.1	Test Purpose and Environment	2210

A.8.12.9.2	Test Requirements	2213

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2214

A.8.12.10.1	Test Purpose and Environment	2214

A.8.12.10.2	Test Requirements	2217

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2218

A.8.12.11.1	Test Purpose and Environment	2218

A.8.12.11.2	Test Requirements	2222

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2223

A.8.12.12.1	Test Purpose and Environment	2223

A.8.12.12.2	Test Requirements	2226

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2227

A.8.12.13.1	Test Purpose and Environment	2227

A.8.12.13.2	Test Requirements	2230

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2231

A.8.12.14.1	Test Purpose and Environment	2231

A.8.12.14.2	Test Requirements	2235

A.8.13	RSTD Inter-frequency Measurements	2236

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2236

A.8.13.1.1	Test Purpose and Environment	2236

A.8.13.1.2	Test Requirements	2239

A.8.13.1.2A	Test Requirements for UE Category 1bis	2240

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2240

A.8.13.2.1	Test Purpose and Environment	2240

A.8.13.2.2	Test Requirements	2244

A.8.13.2.2A	Test Requirements for UE Category 1bis	2245

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2245

A.8.13.3.1	Test Purpose and Environment	2245

A.8.13.3.2	Test Requirements	2249

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2250

A.8.13.4.1	Test Purpose and Environment	2250

A.8.13.4.2	Test Requirements	2253

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2254

A.8.13.5.1	Test Purpose and Environment	2254

A.8.13.5.2	Test Requirements	2258

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2259

A.8.13.6.1	Test Purpose and Environment	2259

A.8.13.6.2	Test Requirements	2262

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2263

A.8.13.7.1	Test Purpose and Environment	2263

A.8.13.7.2	Test Requirements	2266

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2267

A.8.13.8.1	Test Purpose and Environment	2267

A.8.13.8.2	Test Requirements	2271

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2272

A.8.13.9.1	Test Purpose and Environment	2272

A.8.13.9.2	Test Requirements	2276

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2277

A.8.13.10.1	Test Purpose and Environment	2277

A.8.13.10.2	Test Requirements	2281

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2282

A.8.13.11.1	Test Purpose and Environment	2282

A.8.13.11.2	Test Requirements	2286

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2287

A.8.13.12.1	Test Purpose and Environment	2287

A.8.13.12.2	Test Requirements	2291

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2292

A.8.13.13.1	Test Purpose and Environment	2292

A.8.13.13.2	Test Requirements	2296

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2297

A.8.13.14.1	Test Purpose and Environment	2297

A.8.13.14.2	Test Requirements	2301

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2302

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2302

A.8.14.1.1	Test Purpose and Environment	2302

A.8.14.1.2	Test Requirements	2304

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2304

A.8.14.2.1	Test Purpose and Environment	2304

A.8.14.2.2	Test Requirements	2307

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2307

A.8.14.3.1	Test Purpose and Environment	2307

A.8.14.3.2	Test Requirements	2309

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2310

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2310

A.8.15.1.1	Test Purpose and Environment	2310

A.8.15.1.2	Test Requirements	2311

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2311

A.8.15.2.1	Test Purpose and Environment	2311

A.8.15.2.2	Test Requirements	2314

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2314

A.8.15.3.1	Test Purpose and Environment	2314

A.8.15.3.2	Test Requirements	2316

A.8.16	E-UTRAN Carrier Aggregation Measurements	2317

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2317

A.8.16.1.1	Test Purpose and Environment	2317

A.8.16.1.2	Test Requirements	2319

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2319

A.8.16.2.1	Test Purpose and Environment	2319

A.8.16.2.2	Test Requirements	2321

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2321

A.8.16.3.1	Test Purpose and Environment	2321

A.8.16.3.2	Test Requirements	2323

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2324

A.8.16.3A.1	Test Purpose and Environment	2324

A.8.16.3A.2	Test Requirements	2325

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2326

A.8.16.4.1	Test Purpose and Environment	2326

A.8.16.4.2	Test Requirements	2327

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2328

A.8.16.4A.1	Test Purpose and Environment	2328

A.8.16.4A.2	Test Requirements	2330

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2331

A.8.16.5.1	Test Purpose and Environment	2331

A.8.16.5.2	Test Requirements	2331

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2331

A.8.16.6.1	Test Purpose and Environment	2331

A.8.16.6.2	Test Requirements	2332

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2332

A.8.16.7.1	 Test Purpose and Environment	2332

A.8.16.7.2	 Test Requirements	2333

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2333

A.8.16.8.1	 Test Purpose and Environment	2333

A.8.16.8.2	 Test Requirements	2334

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2334

A.8.16.9.1	Test Purpose and Environment	2334

A.8.16.9.2	Test Requirements	2335

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2335

A.8.16.10.1	Test Purpose and Environment	2335

A.8.16.10.2	Test Requirements	2336

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2336

A.8.16.11.1	 Test Purpose and Environment	2336

A.8.16.11.2	 Test Requirements	2337

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2337

A.8.16.12.1	 Test Purpose and Environment	2337

A.8.16.12.2	 Test Requirements	2337

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2337

A.8.16.13.1	Test Purpose and Environment	2337

A.8.16.13.2	Test Requirements	2338

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2338

A.8.16.14.1	Test Purpose and Environment	2338

A.8.16.14.2	Test Requirements	2339

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2339

A.8.16.15.1	 Test Purpose and Environment	2339

A.8.16.7.2	 Test Requirements	2339

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2339

A.8.16.16.1	 Test Purpose and Environment	2339

A.8.16.16.2	 Test Requirements	2340

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2340

A.8.16.17.1	Test Purpose and Environment	2340

A.8.16.17.2	Test Requirements	2342

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2343

A.8.16.17A.1	Test Purpose and Environment	2343

A.8.16.17A.2	Test Requirements	2343

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2343

A.8.16.17B.1	Test Purpose and Environment	2343

A.8.16.17B.2	Test Requirements	2344

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2344

A.8.16.17C.1	Test Purpose and Environment	2344

A.8.16.17C.2	Test Requirements	2344

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2344

A.8.16.18.1	Test Purpose and Environment	2344

A.8.16.18.2	Test Requirements	2346

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2347

A.8.16.18A.1	Test Purpose and Environment	2347

A.8.16.18A.2	Test Requirements	2347

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2347

A.8.16.18B.1	Test Purpose and Environment	2347

A.8.16.18B.2	Test Requirements	2348

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2348

A.8.16.18C.1	Test Purpose and Environment	2348

A.8.16.18C.2	Test Requirements	2348

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2348

A.8.16.18D.1	Test Purpose and Environment	2348

A.8.16.18D.2	Test Requirements	2349

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2349

A.8.16.19.1	Test Purpose and Environment	2349

A.8.16.19.2	Test Requirements	2351

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2351

A.8.16.19A.1	Test Purpose and Environment	2351

A.8.16.19A.2	Test Requirements	2352

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2352

A.8.16.19B.1	Test Purpose and Environment	2352

A.8.16.19B.2	Test Requirements	2352

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2352

A.8.16.19C.1	Test Purpose and Environment	2352

A.8.16.19C.2	Test Requirements	2353

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2353

A.8.16.20.1	Test Purpose and Environment	2353

A.8.16.20.2	Test Requirements	2355

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2356

A.8.16.20A.1	Test Purpose and Environment	2356

A.8.16.20A.2	Test Requirements	2356

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2356

A.8.16.20B.1	Test Purpose and Environment	2356

A.8.16.20B.2	Test Requirements	2357

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2357

A.8.16.20C.1	Test Purpose and Environment	2357

A.8.16.20C.2	Test Requirements	2357

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2357

A.8.16.20D.1	Test Purpose and Environment	2357

A.8.16.20D.2	Test Requirements	2358

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2358

A.8.16.21.1	Test Purpose and Environment	2358

A.8.16.21.2	Test Requirements	2360

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2360

A.8.16.22.1	Test Purpose and Environment	2360

A.8.16.22.2	Test Requirements	2362

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2362

A.8.16.23.1	Test Purpose and Environment	2362

A.8.16.23.2	Test Requirements	2365

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2365

A.8.16.24.1	Test Purpose and Environment	2365

A.8.16.24.2	Test Requirements	2368

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2368

A.8.16.25.1	Test Purpose and Environment	2368

A.8.16.25.2	Test Requirements	2370

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2370

A.8.16.26.1	Test Purpose and Environment	2370

A.8.16.26.2	Test Requirements	2373

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2373

A.8.16.27.1	Test Purpose and Environment	2373

A.8.16.27.2	Test Requirements	2377

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2377

A.8.16.28.1	Test Purpose and Environment	2377

A.8.16.28.2	Test Requirements	2381

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2381

A.8.16.29.1	Test Purpose and Environment	2381

A.8.16.29.2	Test Requirements	2385

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2385

A.8.16.30.1	Test Purpose and Environment	2385

A.8.16.30.2	Test Requirements	2389

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2389

A.8.16.31.1	Test Purpose and Environment	2389

A.8.16.31.2	Test Requirements	2393

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2393

A.8.16.32.1	Test Purpose and Environment	2393

A.8.16.32.2	Test Requirements	2397

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2397

A.8.16.33.1	Test Purpose and Environment	2397

A.8.16.33.2	Test Requirements	2401

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2401

A.8.16.34.1	Test Purpose and Environment	2401

A.8.16.34.2	Test Requirements	2405

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2405

A.8.16.35.1	Test Purpose and Environment	2405

A.8.16.35.2	Test Requirements	2407

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2408

A.8.16.36.1	Test Purpose and Environment	2408

A.8.16.36.2	Test Requirements	2410

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2411

A.8.16.37.1	Test Purpose and Environment	2411

A.8.16.37.2	Test Requirements	2413

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2414

A.8.16.38.1	Test Purpose and Environment	2414

A.8.16.38.2	Test Requirements	2416

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2417

A.8.16.39.1	Test Purpose and Environment	2417

A.8.16.39.2	Test Requirements	2420

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2420

A.8.16.40.1	Test Purpose and Environment	2420

A.8.16.40.2	Test Requirements	2423

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2423

A.8.16.41.1	Test Purpose and Environment	2423

A.8.16.41.2	Test Requirements	2425

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2426

A.8.16.42.1	Test Purpose and Environment	2426

A.8.16.42.2	Test Requirements	2428

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2429

A.8.16.43.1	Test Purpose and Environment	2429

A.8.16.43.2	Test Requirements	2432

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2432

A.8.16.44.1	Test Purpose and Environment	2432

A.8.16.44.2	Test Requirements	2435

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2435

A.8.16.45.1	Test Purpose and Environment	2435

A.8.16.45.2	Test Requirements	2438

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2438

A.8.16.46.1	Test Purpose and Environment	2438

A.8.16.46.2	Test Requirements	2440

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2441

A.8.16.47.1	Test Purpose and Environment	2441

A.8.16.47.2	Test Requirements	2443

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2444

A.8.16.48.1	Test Purpose and Environment	2444

A.8.16.48.2	Test Requirements	2446

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2447

A.8.16.49.1	Test Purpose and Environment	2447

A.8.16.49.2	Test Requirements	2450

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2450

A.8.16.50.1	Test Purpose and Environment	2450

A.8.16.50.2	Test Requirements	2452

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2453

A.8.16.51.1	Test Purpose and Environment	2453

A.8.16.51.2	Test Requirements	2457

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2457

A.8.16.52.1	Test Purpose and Environment	2457

A.8.16.52.2	Test Requirements	2461

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2461

A.8.16.53.1	Test Purpose and Environment	2461

A.8.16.53.2	Test Requirements	2465

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2465

A.8.16.54.1	Test Purpose and Environment	2465

A.8.16.54.2	Test Requirements	2469

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2469

A.8.16.55.1	Test Purpose and Environment	2469

A.8.16.55.2	Test Requirements	2474

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2474

A.8.16.56.1	Test Purpose and Environment	2474

A.8.16.56.2	Test Requirements	2479

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2479

A.8.16.57.1	Test Purpose and Environment	2479

A.8.16.57.2	Test Requirements	2481

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2482

A.8.16.58.1	Test Purpose and Environment	2482

A.8.16.58.2	Test Requirements	2484

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2485

A.8.16.59.1	Test Purpose and Environment	2485

A.8.16.59.2	Test Requirements	2488

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2489

A.8.16.60.1	Test Purpose and Environment	2489

A.8.16.60.2	Test Requirements	2492

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2493

A.8.16.61.1	Test Purpose and Environment	2493

A.8.16.61.2	Test Requirements	2496

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2496

A.8.16.62.1	Test Purpose and Environment	2496

A.8.16.62.2	Test Requirements	2498

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2499

A.8.16.63.1	Test Purpose and Environment	2499

A.8.16.63.2	Test Requirements	2502

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2503

A.8.16.64.1	Test Purpose and Environment	2503

A.8.16.64.2	Test Requirements	2507

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2507

A.8.16.65.1	Test Purpose and Environment	2507

A.8.16.65.2	Test Requirements	2512

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2512

A.8.16.66.1	Test Purpose and Environment	2512

A.8.16.66.2	Test Requirements	2517

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2517

A.8.16.67.1	Test Purpose and Environment	2517

A.8.16.67.2	Test Requirements	2522

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2522

A.8.16.68.1	Test Purpose and Environment	2522

A.8.16.68.2	Test Requirements	2527

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2527

A.8.16.69.1	Test Purpose and Environment	2527

A.8.16.69.2	Test Requirements	2530

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2530

A.8.16.70.1	Test Purpose and Environment	2530

A.8.16.70.2	Test Requirements	2533

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2533

A.8.16.71.1	Test Purpose and Environment	2533

A.8.16.71.2	Test Requirements	2537

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2537

A.8.16.72.1	Test Purpose and Environment	2537

A.8.16.72.2	Test Requirements	2541

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2541

A.8.16.73.1	Test Purpose and Environment	2541

A.8.16.73.2	Test Requirements	2546

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2546

A.8.16.74.1	Test Purpose and Environment	2546

A.8.16.74.2	Test Requirements	2551

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2551

A.8.16.75.1	Test Purpose and Environment	2551

A.8.16.75.2	Test Requirements	2556

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2556

A.8.16.76.1	Test Purpose and Environment	2556

A.8.16.76.2	Test Requirements	2561

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2561

A.8.16.77.1	Test Purpose and Environment	2561

A.8.16.77.2	Test Requirements	2564

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2564

A.8.16.78.1	Test Purpose and Environment	2564

A.8.16.78.2	Test Requirements	2567

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2568

A.8.16.79.1	Test Purpose and Environment	2568

A.8.16.79.2	Test Requirements	2573

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2573

A.8.16.80.1	Test Purpose and Environment	2573

A.8.16.80.2	Test Requirements	2578

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2578

A.8.16.81.1	Test Purpose and Environment	2578

A.8.16.81.2	Test Requirements	2583

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2583

A.8.16.82.1	Test Purpose and Environment	2583

A.8.16.82.2	Test Requirements	2588

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2588

A.8.16.83.1	Test Purpose and Environment	2588

A.8.16.83.2	Test Requirements	2593

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2593

A.8.16.84.1	Test Purpose and Environment	2593

A.8.16.84.2	Test Requirements	2599

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2599

A.8.16.85.1	Test Purpose and Environment	2599

A.8.16.85.2	Test Requirements	2602

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2602

A.8.16.86.1	Test Purpose and Environment	2602

A.8.16.86.2	Test Requirements	2605

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2605

A.8.16.87.1	Test Purpose and Environment	2605

A.8.16.87.2	Test Requirements	2610

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2610

A.8.16.88.1	Test Purpose and Environment	2610

A.8.16.88.2	Test Requirements	2618

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2618

A.8.16.89.1	Test Purpose and Environment	2618

A.8.16.89.2	Test Requirements	2623

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2623

A.8.16.90.1	Test Purpose and Environment	2623

A.8.16.90.2	Test Requirements	2628

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2628

A.8.16.91.1	Test Purpose and Environment	2628

A.8.16.91.2	Test Requirements	2634

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2634

A.8.16.92.1	Test Purpose and Environment	2634

A.8.16.92.2	Test Requirements	2642

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2642

A.8.16.93.1	Test Purpose and Environment	2642

A.8.16.93.2	Test Requirements	2647

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2647

A.8.16.94.1	Test Purpose and Environment	2647

A.8.16.94.2	Test Requirements	2652

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2652

A.8.16.95.1	Test Purpose and Environment	2652

A.8.16.95.2	Test Requirements	2660

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2660

A.8.16.96.1	Test Purpose and Environment	2660

A.8.16.96.2	Test Requirements	2667

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2667

A.8.16.97.1	Test Purpose and Environment	2667

A.8.16.97.2	Test Requirements	2672

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2672

A.8.16.98.1	Test Purpose and Environment	2672

A.8.16.98.2	Test Requirements	2678

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2678

A.8.16.99.1	Test Purpose and Environment	2678

A.8.16.99.2	Test Requirements	2685

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2685

A.8.16.100.1	Test Purpose and Environment	2685

A.8.16.100.2	Test Requirements	2693

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2693

A.8.16.101.1	Test Purpose and Environment	2693

A.8.16.101.2	Test Requirements	2700

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2700

A.8.16.102.1	Test Purpose and Environment	2700

A.8.16.102.2	Test Requirements	2708

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2708

A.8.16.103.1	Test Purpose and Environment	2708

A.8.16.103.2	Test Requirements	2712

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2713

A.8.16.104.1	Test Purpose and Environment	2713

A.8.16.104.2	Test Requirements	2716

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2717

A.8.16.105.1	Test Purpose and Environment	2717

A.8.16.105.2	Test Requirements	2721

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2722

A.8.16.106.1	Test Purpose and Environment	2722

A.8.16.106.2	Test Requirements	2725

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2725

A.8.16.107.1	Test Purpose and Environment	2725

A.8.16.107.2	Test Requirements	2727

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2728

A.8.16.108.1	Test Purpose and Environment	2728

A.8.16.108.2	Test Requirements	2730

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2731

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2731

A.8.17.1.1	Test Purpose and Environment	2731

A.8.17.1.2	Test Requirements	2736

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2736

A.8.17.2.1	Test Purpose and Environment	2736

A.8.17.2.2	Test Requirements	2742

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2742

A.8.17.3.1	Test Purpose and Environment	2742

A.8.17.3.2	Test Requirements	2743

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2743

A.8.17.4.1	Test Purpose and Environment	2743

A.8.17.4.2	Test Requirements	2744

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2744

A.8.17.5.1	Test Purpose and Environment	2744

A.8.17.5.2	Test Requirements	2745

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2745

A.8.17.6.1	Test Purpose and Environment	2745

A.8.17.6.2	Test Requirements	2746

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2747

A.8.17.7.1	Test Purpose and Environment	2747

A.8.17.7.2	Test Requirements	2747

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2748

A.8.17.8.1	Test Purpose and Environment	2748

A.8.17.8.2	Test Requirements	2748

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2749

A.8.17.9.1	Test Purpose and Environment	2749

A.8.17.9.2	Test Requirements	2750

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2750

A.8.17.10.1	Test Purpose and Environment	2750

A.8.17.10.2	Test Requirements	2755

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2756

A.8.17.11.1	Test Purpose and Environment	2756

A.8.17.11.2	Test Requirements	2762

A.8.18	E-UTRAN TDD – HRPD Measurements	2763

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2763

A.8.18.1.1	Test Purpose and Environment	2763

A.8.18.1.2	Test Requirements	2765

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2765

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2765

A.8.19.1.1	Test Purpose and Environment	2765

A.8.19.1.2	Test Requirements	2766

A.8.20	Inter-frequency/RAT Measurements in CA mode	2767

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2767

A.8.20.1.1	Test Purpose and Environment	2767

A.8.20.1.2	Test Requirements	2768

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2768

A.8.20.2.1	Test Purpose and Environment	2769

A.8.20.2.2	Test Requirements	2770

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2771

A.8.20.2A.1	Test Purpose and Environment	2771

A.8.20.2A.2	Test Requirements	2771

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2771

A.8.20.2B.1	Test Purpose and Environment	2771

A.8.20.2B.2	Test Requirements	2774

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2774

A.8.20.3.1	Test Purpose and Environment	2774

A.8.20.3.2	Test Requirements	2776

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2776

A.8.20.4.1	Test Purpose and Environment	2776

A.8.20.4.1.1	1.28 Mcps TDD option	2776

A.8.20.4.2	Test Requirements	2778

A.8.20.4.2.1	1.28 Mcps TDD option	2778

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2778

A.8.20.4A.1	Test Purpose and Environment	2778

A.8.20.4A.1.1	1.28 Mcps TDD option	2778

A.8.20.4A.2	Test Requirements	2778

A.8.20.4A.2.1	1.28 Mcps TDD option	2778

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2779

A.8.20.4B.1	Test Purpose and Environment	2779

A.8.20.4B.1.1	1.28 Mcps TDD option	2779

A.8.20.4B.2	Test Requirements	2781

A.8.20.4B.2.1	1.28 Mcps TDD option	2781

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2781

A.8.21.1	Test Purpose and Environment	2781

A.8.21.2 Test Requirements	2785

A.8.22	E-UTRAN Discovery Signal Measurements	2785

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2785

A.8.22.1.1	Test Purpose and Environment	2785

A.8.22.1.2	Test Requirements	2788

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2788

A.8.22.2.1	Test Purpose and Environment	2788

A.8.22.2.2	Test Requirements	2791

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2791

A.8.22.3.1	Test Purpose and Environment	2791

A.8.22.3.2	Test Requirements	2794

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2794

A.8.22.4.1	Test Purpose and Environment	2794

A.8.22.4.2	Test Requirements	2797

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2797

A.8.22.5.1	Test Purpose and Environment	2797

A.8.22.5.2	Test Requirements	2800

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2800

A.8.22.6.1	Test Purpose and Environment	2800

A.8.22.6.2	Test Requirements	2803

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2803

A.8.22.7.1	Test Purpose and Environment	2803

A.8.22.7.2	Test Requirements	2806

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2806

A.8.22.8.1	Test Purpose and Environment	2806

A.8.22.8.2	Test Requirements	2809

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2809

A.8.22.9.1	Test Purpose and Environment	2809

A.8.22.9.2	Test Requirements	2811

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2812

A.8.22.10.1	Test Purpose and Environment	2812

A.8.22.10.2	Test Requirements	2814

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2815

A.8.22.11.1	Test Purpose and Environment	2815

A.8.22.11.2	Test Requirements	2817

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2817

A.8.22.12.1	Test Purpose and Environment	2817

A.8.22.12.2	Test Requirements	2820

A.8.23	E-UTRAN Dual Connectivity Measurements	2820

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2820

A.8.23.1.1	Test Purpose and Environment	2820

A.8.23.1.2	Test Requirements	2823

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2823

A.8.23.2.1	Test Purpose and Environment	2823

A.8.23.2.2	Test Requirements	2826

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2826

A.8.23.3.1	Test Purpose and Environment	2826

A.8.23.3.2	Test Requirements	2829

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2829

A.8.23.4.1	Test Purpose and Environment	2829

A.8.23.4.2	Test Requirements	2832

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2832

A.8.23.5.1	Test Purpose and Environment	2832

A.8.23.5.2	Test Requirements	2835

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2835

A.8.23.6.1	Test Purpose and Environment	2835

A.8.23.6.2	Test Requirements	2838

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2838

A.8.23.7.1	Test Purpose and Environment	2838

A.8.23.7.2	Test Requirements	2840

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2841

A.8.23.8.1	Test Purpose and Environment	2841

A.8.23.8.2	Test Requirements	2843

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2844

A.8.23.9.1	Test Purpose and Environment	2844

A.8.23.9.2	Test Requirements	2847

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2847

A.8.23.10.1	Test Purpose and Environment	2847

A.8.23.10.2	Test Requirements	2850

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2850

A.8.23.11.1	Test Purpose and Environment	2850

A.8.23.11.2	Test Requirements	2853

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2853

A.8.23.12.1	Test Purpose and Environment	2853

A.8.23.12.2	Test Requirements	2856

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2856

A.8.23.13.1	Test Purpose and Environment	2856

A.8.23.13.2	Test Requirements	2859

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2859

A.8.23.14.1	Test Purpose and Environment	2859

A.8.23.14.2	Test Requirements	2862

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2862

A.8.23.15.1	Test Purpose and Environment	2862

A.8.23.15.2	Test Requirements	2865

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2865

A.8.23.16.1	Test Purpose and Environment	2865

A.8.23.16.2	Test Requirements	2866

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2867

A.8.23.17.1	Test Purpose and Environment	2867

A.8.23.17.2	Test Requirements	2869

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2869

A.8.23.18.1	Test Purpose and Environment	2869

A.8.23.18.2	Test Requirements	2870

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2871

A.8.23.19.1	Test Purpose and Environment	2871

A.8.23.19.2	Test Requirements	2872

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2873

A.8.23.20.1	Test Purpose and Environment	2873

A.8.23.20.2	Test Requirements	2874

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2875

A.8.23.21.1	Test Purpose and Environment	2875

A.8.23.21.2	Test Requirements	2876

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2877

A.8.23.22.1	Test Purpose and Environment	2877

A.8.23.22.2	Test Requirements	2878

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2879

A.8.23.23.1	Test Purpose and Environment	2879

A.8.23.23.2	Test Requirements	2880

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2881

A.8.23.24.1	Test Purpose and Environment	2881

A.8.23.24.2	Test Requirements	2883

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2884

A.8.23.25.1	Test Purpose and Environment	2884

A.8.23.25.2	Test Requirements	2886

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2887

A.8.23.26.1	Test Purpose and Environment	2887

A.8.23.26.2	Test Requirements	2889

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2890

A.8.23.27.1	Test Purpose and Environment	2890

A.8.23.27.2	Test Requirements	2893

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2893

A.8.23.28.1	Test Purpose and Environment	2893

A.8.23.28.2	Test Requirements	2896

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2896

A.8.23.29.1	Test Purpose and Environment	2896

A.8.23.29.2	Test Requirements	2899

A.8.24	Proximity-based Services	2899

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2899

A.8.24.1.1	Test Purpose and Environment	2899

A.8.24.1.2	Test Requirements	2900

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2901

A.8.24.2.1	Test Purpose and Environment	2901

A.8.24.2.2	Test Requirements	2902

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2902

A.8.24.3.1	Test Purpose and Environment	2903

A.8.24.3.2	Test Requirements	2904

A.8.25	E-UTRAN-WLAN Measurements	2905

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2905

A.8.25.1.1	Test Purpose and Environment	2905

A.8.25.1.2	Test Requirements	2907

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2907

A.8.25.2.1	Test Purpose and Environment	2907

A.8.25.2.2	Test Requirements	2909

A.8.26 Frame Structure 3 (FS3)	2909

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2909

A.8.26.1.1	Test Purpose and Environment	2909

A.8.26.1.2	Test Requirements	2911

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2912

A.8.26.2.1	Test Purpose and Environment	2912

A.8.26.2.2	Test Requirements	2914

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2915

A.8.26.3.1	Test Purpose and Environment	2915

A.8.26.3.2	Test Requirements	2918

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2918

A.8.26.3A.1	Test Purpose and Environment	2918

A.8.26.3A.2	Test Requirements	2921

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2921

A.8.26.4.1	Test Purpose and Environment	2921

A.8.26.4.2	Test Requirements	2924

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2924

A.8.26.4A.1	Test Purpose and Environment	2924

A.8.26.4A.2	Test Requirements	2927

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2927

A.8.26.5.1	Test Purpose and Environment	2927

A.8.26.5.2	Test Requirements	2930

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2930

A.8.26.5A.1	Test Purpose and Environment	2930

A.8.26.5A.2	Test Requirements	2933

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2933

A.8.26.6.1	Test Purpose and Environment	2933

A.8.26.6.2	Test Requirements	2936

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2936

A.8.26.6A.1	Test Purpose and Environment	2936

A.8.26.6A.2	Test Requirements	2939

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2939

A.8.26.7.1	Test Purpose and Environment	2939

A.8.26.7.2	Test Requirements	2942

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2942

A.8.26.8.1	Test Purpose and Environment	2942

A.8.26.8.2	Test Requirements	2945

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2945

A.8.26.9.1	Test Purpose and Environment	2945

A.8.26.9.2	Test Requirements	2948

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2948

A.8.26.10.1	Test Purpose and Environment	2948

A.8.26.10.2	Test Requirements	2950

A.9	Measurement Performance Requirements	2951

A.9.1	RSRP	2951

A.9.1.1	FDD Intra frequency case	2951

A.9.1.1.1	Test Purpose and Environment	2951

A.9.1.1.2	Test parameters	2951

A.9.1.1.3	Test Requirements	2954

A.9.1.2	TDD Intra frequency case	2954

A.9.1.2.1	Test Purpose and Environment	2954

A.9.1.2.2	Test parameters	2954

A.9.1.2.3	Test Requirements	2955

A.9.1.3	FDD—FDD Inter frequency case	2956

A.9.1.3.1	Test Purpose and Environment	2956

A.9.1.3.2	Test parameters	2956

A.9.1.3.3	Test Requirements	2959

A.9.1.4	TDD—TDD Inter frequency case	2959

A.9.1.4.1	Test Purpose and Environment	2959

A.9.1.4.2	Test parameters	2959

A.9.1.4.3	Test Requirements	2961

A.9.1.5	FDD—TDD Inter frequency case	2962

A.9.1.5.1	Test Purpose and Environment	2962

A.9.1.5.2	Test parameters	2962

A.9.1.5.3	Test Requirements	2963

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2964

A.9.1.6.1	Test Purpose and Environment	2964

A.9.1.6.2	Test parameters	2964

A.9.1.6.3	Test Requirements	2967

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2967

A.9.1.7.1	Test Purpose and Environment	2967

A.9.1.7.2	Test parameters	2967

A.9.1.7.3	Test Requirements	2970

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2970

A.9.1.8.1	Test Purpose and Environment	2970

A.9.1.8.2	Test parameters	2970

A.9.1.8.3	Test Requirements	2974

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2974

A.9.1.9.1	Test Purpose and Environment	2974

A.9.1.9.2	Test parameters	2974

A.9.1.9.3	Test Requirements	2977

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2977

A.9.1.10.1	Test Purpose and Environment	2977

A.9.1.10.2	Test parameters	2977

A.9.1.10.3	Test Requirements	2980

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2980

A.9.1.11.1	Test Purpose and Environment	2980

A.9.1.11.2	Test parameters	2980

A.9.1.11.3	Test Requirements	2984

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2984

A.9.1.12.1	Test Purpose and Environment	2984

A.9.1.12.2	Test parameters	2984

A.9.1.12.3	Test Requirements	2985

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2985

A.9.1.13.1	Test Purpose and Environment	2985

A.9.1.13.2	Test parameters	2985

A.9.1.13.3	Test Requirements	2986

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2986

A.9.1.14.1	Test Purpose and Environment	2986

A.9.1.14.2	Test parameters	2986

A.9.1.14.3	Test Requirements	2990

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2990

A.9.1.15.1	Test Purpose and Environment	2990

A.9.1.15.2	Test parameters	2990

A.9.1.15.3	Test Requirements	2992

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2993

A.9.1.16.1	Test Purpose and Environment	2993

A.9.1.16.2	Test parameters	2993

A.9.1.16.3	Test Requirements	2994

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2994

A.9.1.17.1	Test Purpose and Environment	2994

A.9.1.17.2	Test parameters	2995

A.9.1.17.3	Test Requirements	2996

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2996

A.9.1.18.1	Test Purpose and Environment	2996

A.9.1.18.2	Test parameters	2996

A.9.1.18.3	Test Requirements	2997

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2998

A.9.1.19.1	Test Purpose and Environment	2998

A.9.1.19.2	Test parameters	2998

A.9.1.19.3	Test Requirements	2998

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2998

A.9.1.20.1	Test Purpose and Environment	2998

A.9.1.20.2	Test parameters	2998

A.9.1.20.3	Test Requirements	3000

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3000

A.9.1.21.1	Test Purpose and Environment	3000

A.9.1.21.2	Test parameters	3000

A.9.1.21.3	Test Requirements	3000

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3000

A.9.1.22.1	Test Purpose and Environment	3000

A.9.1.22.2	Test parameters	3001

A.9.1.22.3	Test Requirements	3005

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3005

A.9.1.23.1	Test Purpose and Environment	3005

A.9.1.23.2	Test parameters	3005

A.9.1.23.3	Test Requirements	3009

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3009

A.9.1.24.1	Test Purpose and Environment	3009

A.9.1.24.2	Test parameters	3009

A.9.1.24.3	Test Requirements	3010

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3010

A.9.1.25.1	Test Purpose and Environment	3010

A.9.1.25.2	Test parameters	3010

A.9.1.25.3	Test Requirements	3013

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3013

A.9.1.26.1	Test Purpose and Environment	3013

A.9.1.26.2	Test parameters	3013

A.9.1.26.3	Test Requirements	3016

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3016

A.9.1.27.1	Test Purpose and Environment	3016

A.9.1.27.2	Test parameters	3016

A.9.1.27.3	Test Requirements	3019

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3019

A.9.1.28.1	Test Purpose and Environment	3019

A.9.1.28.2	Test parameters	3019

A.9.1.28.3	Test Requirements	3022

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3022

A.9.1.29.1	Test Purpose and Environment	3022

A.9.1.29.2	Test parameters	3022

A.9.1.29.3	Test Requirements	3025

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3025

A.9.1.30.1	Test Purpose and Environment	3025

A.9.1.30.2	Test parameters	3026

A.9.1.30.3	Test Requirements	3029

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3029

A.9.1.31.1	Test Purpose and Environment	3029

A.9.1.31.2	Test parameters	3029

A.9.1.31.3	Test Requirements	3032

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3033

A.9.1.32.1	Test Purpose and Environment	3033

A.9.1.32.2	Test parameters	3033

A.9.1.32.3	Test Requirements	3036

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3036

A.9.1.33.1	Test Purpose and Environment	3036

A.9.1.33.2	Test parameters	3036

A.9.1.33.3	Test Requirements	3039

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3039

A.9.1.34.1	Test Purpose and Environment	3039

A.9.1.34.2	Test parameters	3039

A.9.1.34.3	Test Requirements	3042

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3042

A.9.1.35.1	Test Purpose and Environment	3042

A.9.1.35.2	Test parameters	3042

A.9.1.35.3	Test Requirements	3045

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3046

A.9.1.36.1	Test Purpose and Environment	3046

A.9.1.36.2	Test parameters	3046

A.9.1.36.3	Test Requirements	3049

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3049

A.9.1.37.1	Test Purpose and Environment	3049

A.9.1.37.2	Test parameters	3049

A.9.1.37.3	Test Requirements	3052

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3053

A.9.1.38.1	Test Purpose and Environment	3053

A.9.1.38.2	Test parameters	3053

A.9.1.38.3	Test Requirements	3056

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3057

A.9.1.39.1	Test Purpose and Environment	3057

A.9.1.39.2	Test parameters	3057

A.9.1.39.3	Test Requirements	3063

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3063

A.9.1.40.1	Test Purpose and Environment	3063

A.9.1.40.2	Test parameters	3064

A.9.1.40.3	Test Requirements	3070

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3070

A.9.1.41.1	Test Purpose and Environment	3070

A.9.1.41.2	Test parameters	3070

A.9.1.41.3	Test Requirements	3073

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3073

A.9.1.42.1	Test Purpose and Environment	3073

A.9.1.42.2	Test parameters	3073

A.9.1.42.3	Test Requirements	3076

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3076

A.9.1.43.1	Test Purpose and Environment	3076

A.9.1.43.2	Test parameters	3076

A.9.1.43.3	Test Requirements	3077

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3078

A.9.1.44.1	Test Purpose and Environment	3078

A.9.1.44.2	Test parameters	3078

A.9.1.44.3	Test Requirements	3083

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3083

A.9.1.45.1	Test Purpose and Environment	3083

A.9.1.45.2	Test parameters	3083

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2486

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2486

A.9.1.55.1	Test Purpose and Environment	2486

A.9.1.55.2	Test parameters	2486

A.9.1.55.3	Test Requirements	2490

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2490

A.9.1.56.1	Test Purpose and Environment	2490

A.9.1.56.2	Test parameters	2490

A.9.1.56.3	Test Requirements	2493

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2494

A.9.1.57.1	Test Purpose and Environment	2494

A.9.1.57.2	Test parameters	2494

A.9.1.57.3	Test Requirements	2497

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2497

A.9.1.57A.1	Test Purpose and Environment	2497

A.9.1.57A.2	Test parameters	2497

A.9.1.57A.3	Test Requirements	2498

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2498

A.9.1.58.1	Test Purpose and Environment	2498

A.9.1.58.2	Test parameters	2499

A.9.1.58.3	Test Requirements	2502

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2502

A.9.1.58A.1	Test Purpose and Environment	2502

A.9.1.58A.2	Test parameters	2502

A.9.1.58A.3	Test Requirements	2503

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2503

A.9.1.59.1	Test Purpose and Environment	2503

A.9.1.59.2	Test parameters	2504

A.9.1.59.3	Test Requirements	2506

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2506

A.9.1.60.1	Test Purpose and Environment	2506

A.9.1.60.2	Test parameters	2506

A.9.1.60.3	Test Requirements	2509

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2510

A.9.1.61.1	Test Purpose and Environment	2510

A.9.1.61.2	Test parameters	2510

A.9.1.61.3	Test Requirements	2513

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2513

A.9.1.62.1	Test Purpose and Environment	2513

A.9.1.62.2	Test parameters	2513

A.9.1.62.3	Test Requirements	2516

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2516

A.9.1.53.1	Test Purpose and Environment	2516

A.9.1.53.2	Test parameters	2516

A.9.1.63.3	Test Requirements	2519

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2519

A.9.1.64.1	Test Purpose and Environment	2519

A.9.1.64.2	Test parameters	2519

A.9.1.64.3	Test Requirements	2520

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2521

A.9.1.65.1	Test Purpose and Environment	2521

A.9.1.65.2	Test parameters	2521

A.9.1.65.3	Test Requirements	2527

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2527

A.9.1.66.1	Test Purpose and Environment	2527

A.9.1.66.2	Test parameters	2527

A.9.1.66.3	Test Requirements	2533

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2533

A.9.1.67.1	Test Purpose and Environment	2533

A.9.1.67.2	Test parameters	2533

A.9.1.67.3	Test Requirements	2534

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2535

A.9.1.68.1	Test Purpose and Environment	2535

A.9.1.68.2	Test parameters	2535

A.9.1.68.3	Test Requirements	2539

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2539

A.9.1.69.1	Test Purpose and Environment	2539

A.9.1.69.2	Test parameters	2539

A.9.1.69.3	Test Requirements	2543

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2544

A.9.1.70.1	Test Purpose and Environment	2544

A.9.1.70.2	Test parameters	2544

A.9.1.70.3	Test Requirements	2556

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2557

A.9.1.71.1	Test Purpose and Environment	2557

A.9.1.71.2	Test parameters	2557

A.9.1.71.3	Test Requirements	2569

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2570

A.9.1.72.1	Test Purpose and Environment	2570

A.9.1.72.2	Test parameters	2570

A.9.1.72.3	Test Requirements	2582

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	2583

A.9.1.73.1	Test Purpose and Environment	2583

A.9.1.73.2	Test parameters	2583

A.9.1.73.3	Test Requirements	2586

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	2586

A.9.1.74.1	Test Purpose and Environment	2586

A.9.1.74.2	Test parameters	2586

A.9.1.74.3	Test Requirements	2587

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	2587

A.9.1.75.1	Test Purpose and Environment	2587

A.9.1.75.2	Test parameters	2588

A.9.1.75.3	Test Requirements	2589

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2589

A.9.1.76.1	Test Purpose and Environment	2589

A.9.1.76.2	Test parameters	2589

A.9.1.76.3	Test Requirements	2592

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2592

A.9.1.77.1	Test Purpose and Environment	2592

A.9.1.77.2	Test parameters	2592

A.9.1.77.3	Test Requirements	2595

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2595

A.9.1.78.1	Test Purpose and Environment	2595

A.9.1.78.2	Test parameters	2595

A.9.1.78.3	Test Requirements	2598

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2598

A.9.1.79.1	Test Purpose and Environment	2598

A.9.1.79.2	Test parameters	2598

A.9.1.79.3	Test Requirements	2601

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2601

A.9.1.80.1	Test Purpose and Environment	2601

A.9.1.80.2	Test parameters	2601

A.9.1.80.3	Test Requirements	2604

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2604

A.9.1.81.1	Test Purpose and Environment	2604

A.9.1.78.2	Test parameters	2604

A.9.1.81.3	Test Requirements	2607

A.9.2	RSRQ	2607

A.9.2.1	FDD Intra frequency case	2607

A.9.2.1.1	Test Purpose and Environment	2607

A.9.2.1.2	Test parameters	2607

A.9.2.1.3	Test Requirements	2610

A.9.2.2	TDD Intra frequency case	2610

A.9.2.2.1	Test Purpose and Environment	2610

A.9.2.2.2	Test parameters	2610

A.9.2.2.3	Test Requirements	2612

A.9.2.3	FDD—FDD Inter frequency case	2612

A.9.2.3.1	Test Purpose and Environment	2612

A.9.2.3.2	Test parameters	2612

A.9.2.3.3	Test Requirements	2615

A.9.2.4	TDD—TDD Inter frequency case	2615

A.9.2.4.1	Test Purpose and Environment	2615

A.9.2.4.2	Test parameters	2615

A.9.2.4.3	Test Requirements	2617

A.9.2.4A	FDD—TDD Inter frequency case	2618

A.9.2.4A.1	Test Purpose and Environment	2618

A.9.2.4A.2	Test parameters	2618

A.9.2.4A.3	Test Requirements	2620

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2620

A.9.2.5.1	Test Purpose and Environment	2620

A.9.2.5.2	Test parameters	2621

A.9.2.5.3	Test Requirements	2624

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2624

A.9.2.6.1	Test Purpose and Environment	2624

A.9.2.6.2	Test parameters	2625

A.9.2.6.3	Test Requirements	2628

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2628

A.9.2.7.1	Test Purpose and Environment	2628

A.9.2.7.2	Test parameters	2628

A.9.2.7.3	Test Requirements	2632

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2632

A.9.2.8.1	Test Purpose and Environment	2632

A.9.2.8.2	Test parameters	2632

A.9.2.8.3	Test Requirements	2636

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2636

A.9.2.9.1	Test Purpose and Environment	2636

A.9.2.9.2	Test parameters	2636

A.9.2.9.3	Test Requirements	2640

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2640

A.9.2.10.1	Test Purpose and Environment	2640

A.9.2.10.2	Test parameters	2640

A.9.2.10.3	Test Requirements	2644

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2644

A.9.2.11.1	Test Purpose and Environment	2644

A.9.2.11.2	Test parameters	2644

A.9.2.11.3	Test Requirements	2645

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2645

A.9.2.12.1	Test Purpose and Environment	2645

A.9.2.12.2	Test parameters	2646

A.9.2.12.3	Test Requirements	2646

A.9.2.13	Void	2647

A.9.2.13.1	Void	2647

A.9.2.13.2	Void	2647

A.9.2.13.3	Void	2647

A.9.2.14	Void	2647

A.9.2.14.1	Void	2647

A.9.2.14.2	Void	2647

A.9.2.14.3	Void	2647

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2647

A.9.2.15.1	Test Purpose and Environment	2647

A.9.2.15.2	Test parameters	2647

A.9.2.15.3	Test Requirements	2651

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2651

A.9.2.16.1	Test Purpose and Environment	2651

A.9.2.16.2	Test parameters	2651

A.9.2.16.3	Test Requirements	2656

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2656

A.9.2.17.1	Test Purpose and Environment	2656

A.9.2.17.2	Test parameters	2656

A.9.2.17.3	Test Requirements	2658

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2658

A.9.2.18.1	Test Purpose and Environment	2658

A.9.2.18.2	Test parameters	2658

A.9.2.18.3	Test Requirements	2661

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2661

A.9.2.19.1	Test Purpose and Environment	2661

A.9.2.19.2	Test parameters	2661

A.9.2.19.3	Test Requirements	2663

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2663

A.9.2.20.1	Test Purpose and Environment	2663

A.9.2.20.2	Test parameters	2663

A.9.2.20.3	Test Requirements	2666

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2666

A.9.2.21.1	Test Purpose and Environment	2666

A.9.2.21.2	Test parameters	2666

A.9.2.21.3	Test Requirements	2669

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2669

A.9.2.22.1	Test Purpose and Environment	2670

A.9.2.22.2	Test parameters	2670

A.9.2.22.3	Test Requirements	2670

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2670

A.9.2.23.1	Test Purpose and Environment	2670

A.9.2.23.2	Test parameters	2670

A.9.2.23.3	Test Requirements	2673

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2673

A.9.2.24.1	Test Purpose and Environment	2673

A.9.2.24.2	Test parameters	2673

A.9.2.24.3	Test Requirements	2674

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2674

A.9.2.25.1	Test Purpose and Environment	2674

A.9.2.25.2	Test parameters	2674

A.9.2.25.3	Test Requirements	2679

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2679

A.9.2.26.1	Test Purpose and Environment	2679

A.9.2.26.2	Test parameters	2679

A.9.2.26.3	Test Requirements	2683

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2683

A.9.2.27.1	Test Purpose and Environment	2683

A.9.2.27.2	Test parameters	2683

A.9.2.27.3	Test Requirements	2684

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2684

A.9.2.28.1	Test Purpose and Environment	2684

A.9.2.28.2	Test parameters	2684

A.9.2.28.3	Test Requirements	2687

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2687

A.9.2.29.1	Test Purpose and Environment	2687

A.9.2.29.2	Test parameters	2687

A.9.2.29.3	Test Requirements	2690

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2690

A.9.2.30.1	Test Purpose and Environment	2690

A.9.2.30.2	Test parameters	2690

A.9.2.30.3	Test Requirements	2693

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2693

A.9.2.31.1	Test Purpose and Environment	2693

A.9.2.31.2	Test parameters	2693

A.9.2.31.3	Test Requirements	2696

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2696

A.9.2.32.1	Test Purpose and Environment	2696

A.9.2.32.2	Test parameters	2696

A.9.2.32.3	Test Requirements	2699

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2699

A.9.2.33.1	Test Purpose and Environment	2699

A.9.2.33.2	Test parameters	2699

A.9.2.33.3	Test Requirements	2703

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2703

A.9.2.34.1	Test Purpose and Environment	2703

A.9.2.34.2	Test parameters	2703

A.9.2.34.3	Test Requirements	2706

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2706

A.9.2.35.1	Test Purpose and Environment	2706

A.9.2.35.2	Test parameters	2706

A.9.2.35.3	Test Requirements	2709

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2709

A.9.2.36.1	Test Purpose and Environment	2709

A.9.2.36.2	Test parameters	2709

A.9.2.36.3	Test Requirements	2710

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2711

A.9.2.37.1	Test Purpose and Environment	2711

A.9.2.37.2	Test parameters	2711

A.9.2.37.3	Test Requirements	2712

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2713

A.9.2.38.1	Test Purpose and Environment	2713

A.9.2.38.2	Test parameters	2713

A.9.2.38.3	Test Requirements	2717

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2717

A.9.2.39.1	Test Purpose and Environment	2717

A.9.2.39.2	Test parameters	2717

A.9.2.39.3	Test Requirements	2721

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2721

A.9.2.40.1	Test Purpose and Environment	2721

A.9.2.40.2	Test parameters	2721

A.9.2.40.3	Test Requirements	2724

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2725

A.9.2.41.1	Test Purpose and Environment	2725

A.9.2.41.2	Test parameters	2725

A.9.2.41.3	Test Requirements	2728

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2728

A.9.2.42.1	Test Purpose and Environment	2728

A.9.2.42.2	Test parameters	2728

A.9.2.42.3	Test Requirements	2731

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2731

A.9.2.43.1	Test Purpose and Environment	2731

A.9.2.43.2	Test parameters	2731

A.9.2.43.3	Test Requirements	2734

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2734

A.9.2.44.1	Test Purpose and Environment	2734

A.9.2.44.2	Test parameters	2734

A.9.2.44.3	Test Requirements	2736

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2736

A.9.2.45.1	Test Purpose and Environment	2736

A.9.2.45.2	Test parameters	2736

A.9.2.45.3	Test Requirements	2740

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2741

A.9.2.46.1	Test Purpose and Environment	2741

A.9.2.46.2	Test parameters	2741

A.9.2.46.3	Test Requirements	2745

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2746

A.9.2.47.1	Test Purpose and Environment	2746

A.9.2.47.2	Test parameters	2746

A.9.2.47.3	Test Requirements	2750

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2750

A.9.2.48.1	Test Purpose and Environment	2750

A.9.2.48.2	Test parameters	2751

A.9.2.48.3	Test Requirements	2755

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2756

A.9.2.49.1	Test Purpose and Environment	2756

A.9.2.49.2	Test parameters	2756

A.9.2.49.3	Test Requirements	2762

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2763

A.9.2.50.1	Test Purpose and Environment	2763

A.9.2.50.2	Test parameters	2763

A.9.2.50.3	Test Requirements	2769

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2769

A.9.2.51.1	Test Purpose and Environment	2769

A.9.2.51.2	Test parameters	2770

A.9.2.51.3	Test Requirements	2774

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2774

A.9.2.52.1	Test Purpose and Environment	2774

A.9.2.52.2	Test parameters	2774

A.9.2.52.3	Test Requirements	2777

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2778

A.9.2.53.1	Test Purpose and Environment	2778

A.9.2.53.2	Test parameters	2778

A.9.2.53.3	Test Requirements	2784

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2785

A.9.2.54.1	Test Purpose and Environment	2785

A.9.2.54.2	Test parameters	2785

A.9.2.54.3	Test Requirements	2791

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2791

A.9.2.55.1	Test Purpose and Environment	2791

A.9.2.55.2	Test parameters	2792

A.9.2.55.3	Test Requirements	2797

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2797

A.9.2.56.1	Test Purpose and Environment	2797

A.9.2.56.2	Test parameters	2797

A.9.2.56.3	Test Requirements	2802

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2802

A.9.2.57.1	Test Purpose and Environment	2802

A.9.2.57.2	Test parameters	2803

A.9.2.57.3	Test Requirements	2807

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2808

A.9.2.58.1	Test Purpose and Environment	2808

A.9.2.58.2	Test parameters	2808

A.9.2.58.3	Test Requirements	2812

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2813

A.9.2.59.1	Test Purpose and Environment	2813

A.9.2.59.2	Test parameters	2813

A.9.2.59.3	Test Requirements	2818

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	2819

A.9.2.60.1	Test Purpose and Environment	2819

A.9.2.60.2	Test parameters	2819

A.9.2.60.3	Test Requirements	2822

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	2822

A.9.2.61.1	Test Purpose and Environment	2822

A.9.2.61.2	Test parameters	2822

A.9.2.61.3	Test Requirements	2825

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	2825

A.9.2.62.1	Test Purpose and Environment	2825

A.9.2.62.2	Test parameters	2825

A.9.2.62.3	Test Requirements	2828

A.9.3	UTRAN FDD CPICH RSCP	2828

A.9.3.1	E-UTRAN FDD	2828

A.9.3.1.1	Test Purpose and Environment	2828

A.9.3.1.2	Parameters	2828

A.9.3.1.3	Test Requirements	2831

A.9.3.2 E-UTRAN TDD	2831

A.9.3.2.1	Test Purpose and Environment	2831

A.9.3.2.2	Parameters	2831

A.9.3.2.3	Test Requirements	2834

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2834

A.9.3.3.1	Test Purpose and Environment	2834

A.9.3.3.2	Parameters	2834

A.9.3.3.3	Test Requirements	2835

A.9.4	UTRAN FDD CPICH Ec/No	2836

A.9.4.1	E-UTRAN FDD	2836

A.9.4.1.1	Test Purpose and Environment	2836

A.9.4.1.2	Parameters	2836

A.9.4.1.3	Test Requirements	2838

A.9.4.2	E-UTRAN TDD	2839

A.9.4.2.1	Test Purpose and Environment	2839

A.9.4.2.2	Parameters	2839

A.9.4.2.3	Test Requirements	2841

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2842

A.9.4.3.1	Test Purpose and Environment	2842

A.9.4.3.2	Parameters	2842

A.9.4.3.3	Test Requirements	2843

A.9.5	UTRAN TDD measurement	2843

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2843

A.9.5.1.1	Test Purpose and Environment	2843

A.9.5.1.2	Test parameters	2843

A.9.5.1.3	Test Requirements	2845

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2845

A.9.5.2.1	Test Purpose and Environment	2845

A.9.5.2.2	Test parameters	2845

A.9.5.2.3	Test Requirements	2847

A.9.6	GSM Carrier RSSI	2848

A.9.6.1	E-UTRAN FDD	2848

A.9.6.1.1	Test Purpose and Environment	2848

A.9.6.1.2	Test Requirements	2849

A.9.6.2	E-UTRAN TDD	2849

A.9.6.2.1	Test Purpose and Environment	2849

A.9.6.2.2	Test Requirements	2851

A.9.7	UE Rx – Tx Time Difference	2852

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2852

A.9.7.1.1	Test Purpose and Environment	2852

A.9.7.1.2	Test parameters	2852

A.9.7.1.3	Test Requirements	2853

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2853

A.9.7.2.1	Test Purpose and Environment	2853

A.9.7.2.2	Test parameters	2853

A.9.7.2.3	Test Requirements	2855

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2855

A.9.7.3.1	Test Purpose and Environment	2855

A.9.7.3.2	Test parameters	2855

A.9.7.3.3	Test Requirements	2858

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2858

A.9.7.4.1	Test Purpose and Environment	2858

A.9.7.4.2	Test Parameters	2858

A.9.7.4.3	Test Requirements	2861

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2861

A.9.7.5.1	Test Purpose and Environment	2861

A.9.7.5.2	Test parameters	2861

A.9.7.5.3	Test Requirements	2864

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2864

A.9.7.6.1	Test Purpose and Environment	2864

A.9.7.6.2	Test Parameters	2864

A.9.7.6.3	Test Requirements	2867

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2867

A.9.7.7.1	Test Purpose and Environment	2867

A.9.7.7.2	Test parameters	2867

A.9.7.7.3	Test Requirements	2868

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2869

A.9.7.8.1	Test Purpose and Environment	2869

A.9.7.8.2	Test parameters	2869

A.9.7.8.3	Test Requirements	2870

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2870

A.9.7.9.1	Test Purpose and Environment	2870

A.9.7.9.2	Test parameters	2870

A.9.7.9.3	Test Requirements	2871

A.9.8	RSTD	2872

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2872

A.9.8.1.1	Test Purpose and Environment	2872

A.9.8.1.2	Test Requirements	2874

A.9.8.1.2A	Test Requirements for UE Category 1bis	2874

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2875

A.9.8.2.1	Test Purpose and Environment	2875

A.9.8.2.2	Test Requirements	2878

A.9.8.2.2A	Test Requirements for UE Category 1bis	2878

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2879

A.9.8.3.1	Test Purpose and Environment	2879

A.9.8.3.2	Test Requirements	2881

A.9.8.3.2A	Test Requirements for UE Category 1bis	2881

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2882

A.9.8.4.1	Test Purpose and Environment	2882

A.9.8.4.2	Test Requirements	2884

A.9.8.4.2A	Test Requirements for UE Category 1bis	2884

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2885

A.9.8.5.1	Test Purpose and Environment	2885

A.9.8.5.2	Test Requirements	2887

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2887

A.9.8.6.1	Test Purpose and Environment	2887

A.9.8.6.2	Test Requirements	2890

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2890

A.9.8.7.1	Test Purpose and Environment	2890

A.9.8.7.2	Test Requirements	2891

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2891

A.9.8.8.1	Test Purpose and Environment	2891

A.9.8.8.2	Test Requirements	2892

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2892

A.9.8.9.1	Test Purpose and Environment	2892

A.9.8.9.2	Test Requirements	2893

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2893

A.9.8.10.1	Test Purpose and Environment	2893

A.9.8.10.2	Test Requirements	2894

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2894

A.9.8.11.1	Test Purpose and Environment	2894

A.9.8.11.2	Test Requirements	2895

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2895

A.9.8.12.1	Test Purpose and Environment	2895

A.9.8.12.2	Test Requirements	2896

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2896

A.9.8.13.1	Test Purpose and Environment	2896

A.9.8.13.2	Test Requirements	2897

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2897

A.9.8.14.1	Test Purpose and Environment	2897

A.9.8.14.2	Test Requirements	2904

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2904

A.9.8.15.1	Test Purpose and Environment	2904

A.9.8.15.2	Test Requirements	2910

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2910

A.9.8.16.1	Test Purpose and Environment	2910

A.9.8.16.2	Test Requirements	2913

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2913

A.9.8.17.1	Test Purpose and Environment	2913

A.9.8.17.2	Test Requirements	2916

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2916

A.9.8.18.1	Test Purpose and Environment	2916

A.9.8.18.2	Test Requirements	2919

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2919

A.9.8.19.1	Test Purpose and Environment	2919

A.9.8.19.2	Test Requirements	2922

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2922

A.9.8.20.1	Test Purpose and Environment	2922

A.9.8.20.2	Test Requirements	2925

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2925

A.9.8.21.1	Test Purpose and Environment	2925

A.9.8.21.2	Test Requirements	2929

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2930

A.9.8.22.1	Test Purpose and Environment	2930

A.9.8.22.2	Test Requirements	2933

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2934

A.9.8.23.1	Test Purpose and Environment	2934

A.9.8.23.2	Test Requirements	2937

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2937

A.9.8.24.1	Test Purpose and Environment	2937

A.9.8.24.2	Test Requirements	2941

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2941

A.9.8.25.1	Test Purpose and Environment	2941

A.9.8.25.2	Test Requirements	2945

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2946

A.9.8.26.1	Test Purpose and Environment	2946

A.9.8.26.2	Test Requirements	2948

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2948

A.9.8.27.1	Test Purpose and Environment	2948

A.9.8.27.2	Test Requirements	2951

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2951

A.9.8.28.1	Test Purpose and Environment	2951

A.9.8.28.2	Test Requirements	2954

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2954

A.9.8.29.1	Test Purpose and Environment	2954

A.9.8.29.2	Test Requirements	2958

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2959

A.9.8.30.1	Test Purpose and Environment	2959

A.9.8.30.2	Test Requirements	2962

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2963

A.9.8.31.1	Test Purpose and Environment	2963

A.9.8.31.2	Test Requirements	2967

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2967

A.9.8.32.1	Test Purpose and Environment	2967

A.9.8.32.2	Test Requirements	2970

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2970

A.9.8.33.1	Test Purpose and Environment	2970

A.9.8.33.2	Test Requirements	2973

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2973

A.9.8.34.1	Test Purpose and Environment	2973

A.9.8.34.2	Test Requirements	2976

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2976

A.9.8.35.1	Test Purpose and Environment	2976

A.9.8.35.2	Test Requirements	2979

A.9.9	RSRP and RSRQ on the serving cell	2979

A.9.9.1	FDD Intra frequency serving cell case	2979

A.9.9.1.1	Test Purpose and Environment	2979

A.9.9.1.2	Test parameters	2979

A.9.9.1.3	Test Requirements	2982

A.9.9.2	TDD Intra frequency serving cell case	2982

A.9.9.2.1	Test Purpose and Environment	2982

A.9.9.2.2	Test parameters	2982

A.9.9.2.3	Test Requirements	2984

A.9.10	SSTD	2984

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2984

A.9.10.1.1	Test Purpose and Environment	2984

A.9.10.1.2	Test parameters	2984

A.9.10.1.3	Test Requirements	2986

A.9.10.2	Void	2986

A.9.10.3	Void	2986

A.9.10.4	Void	2986

A.9.11 RSSI	2986

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2986

A.9.11.1.1	Test Purpose and Environment	2986

A.9.11.1.2	Test parameters	2986

A.9.11.1.3	Test Requirements	2989

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2989

A.9.11.2.1	Test Purpose and Environment	2989

A.9.11.2.2	Test parameters	2989

A.9.12	Channel occupancy	2992

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2992

A.9.12.1.1	Test Purpose and Environment	2992

A.9.12.1.2	Test parameters	2992

A.9.12.1.3	Test Requirements	2995

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2995

A.9.12.2.1	Test Purpose and Environment	2995

A.9.12.2.2	Test parameters	2995

A.9.12.2.3	Test Requirements	2998

A.9.13	RS-SINR	2998

A.9.13.1	FDD Intra-Frequency Case	2998

A.9.13.1.1	Test Purpose and Environment	2998

A.9.13.1.2	Test parameters	2998

A.9.13.1.3	Test Requirements	3001

A.9.13.2	TDD Intra-Frequency Case	3001

A.9.13.2.1	Test Purpose and Environment	3001

A.9.13.2.2	Test parameters	3001

A.9.13.2.3	Test Requirements	3004

A.9.13.3	FDD—FDD Inter frequency case	3004

A.9.13.3.1	Test Purpose and Environment	3004

A.9.13.3.2	Test parameters	3004

A.9.13.3.3	Test Requirements	3007

A.9.13.4	TDD—TDD Inter frequency case	3007

A.9.13.4.1	Test Purpose and Environment	3007

A.9.13.4.2	Test parameters	3007

A.9.13.4.3	Test Requirements	3011

A.9.13.5	FDD—TDD Inter frequency case	3012

A.9.13.5.1	Test Purpose and Environment	3012

A.9.13.5.2	Test parameters	3012

A.9.13.5.3	Test Requirements	3018

A.9.13.6	TDD—FDD Inter frequency case	3018

A.9.13.6.1	Test Purpose and Environment	3018

A.9.13.6.2	Test parameters	3018

A.9.13.6.3	Test Requirements	3022

A.9.14	Channel quality reporting accuracy	3022

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3022

A.9.14.1.1	Test Purpose and Environment	3022

A.9.14.1.2	Test parameters	3023

A.9.14.1.3	Test Requirements	3023

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3024

A.9.14.2.1	Test Purpose and Environment	3024

A.9.14.2.2	Test parameters	3024

A.9.14.2.3	Test Requirements	3025

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3025

A.9.14.3.1	Test Purpose and Environment	3025

A.9.14.3.2	Test parameters	3025

A.9.14.3.3	Test Requirements	3026

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3026

A.9.14.4.1	Test Purpose and Environment	3026

A.9.14.4.2	Test parameters	3026

A.9.14.4.3	Test Requirements	3027

A.10	Proximity-based Services in Any Cell Selection State	3050

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3050

A.10.1.1	Test Purpose and Environment	3050

A.10.1.2	Test Requirements	3051

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3052

A.10.2.1	Test Purpose and Environment	3052

A.10.2.2	Test Requirements	3053

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3053

A.10.3.1	Test Purpose and Environment	3053

A.10.3.2	Test Requirements	3055

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3056

A.10.4.1	Test Purpose and Environment	3056

A.10.4.2	Test Requirements	3058

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3058

A.11.1	V2V UE Transmission Timing Accuracy Test	3058

A.11.1.1	Test Purpose and Environment	3058

A.11.1.2	Test requirements	3059

A.11.2	Interruptions due to V2V sidelink communication	3059

A.11.2.1	Test Purpose and Environment	3059

A.11.2.2	Test Requirements	3061

A.12	3061

A.12.1	V2X UE Transmission Timing Accuracy Test	3061

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3061

A.12.1.1.1	Test Purpose and Environment	3061

A.12.1.1.2	Test requirements	3062

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3063

A.12.1.2.1	Test Purpose and Environment	3063

A.12.1.2.2	Test Requirements	3063

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3064

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3064

A.12.2.1.1	Test Purpose and Environment	3064

A.12.2.1.2	Test Requirements	3065

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3066

A.12.2.2.1	Test Purpose and Environment	3066

A.12.2.2.2	Test Requirements	3067

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3067

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3067

A.12.3.1.1	Test Purpose and Environment	3067

A.12.3.1.2	Test Requirements	3070

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3071

A.12.3.2.1	Test Purpose and Environment	3071

A.12.3.1.2	Test Requirements	3073

A.12.4	Congestion Control Measurement Test for V2X UE	3073

A.12.4.1	Test Purpose and Environment	3073

A.12.4.2	Test Requirements	3075

A.12.5	Interruptions due to V2X Sidelink Communication	3075

A.12.5.1	Test Purpose and Environment	3075

A.12.5.2	Test Requirements	3077

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3077

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3077

A.12.6.1.1	Test Purpose and Environment	3077

A.12.6.1.2	Test Requirements	3079

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3079

A.12.6.2.1	Test Purpose and Environment	3079

A.12.6.1.2	Test Requirements	3081

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3081

A.12.7.1	Test Purpose and Environment	3081

A.12.7.2	Test Requirements	3084

A.12.8	Interruptions due to V2X Carrier Aggregation	3085

A.12.8.1	Interruptions on a FDD PCell	3085

A.12.8.1.1	Test Purpose and Environment	3085

A.12.8.1.2	Test Requirements	3086

A.12.8.2	Interruptions on a TDD PCell	3086

A.12.8.2.1	Test Purpose and Environment	3086

A.12.8.2.2	Test Requirements	3088

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3089

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3089

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3089

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3089

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3089

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3091

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3092

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3092

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3092

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3092

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3094

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3095

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3095

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3095

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3096

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3096

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3097

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3097

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3097

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3098

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3098

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3099

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3099

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3099

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3099

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3100

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3100

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3100

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3101

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3101

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3101

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3102

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3102

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3103

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3104

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3105

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3106

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3107

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3107

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3107

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3107

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3108

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3108

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3110

B.3	Conditions for measurements performance requirements for UE	3110

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3110

B.3.2	Void	3110

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3110

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3110

B.3.5	Conditions for UE Rx – Tx time difference	3110

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3111

B.3.7	Conditions for inter-frequency RSTD measurements	3111

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3111

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3111

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3111

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3112

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3112

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3112

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3113

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3113

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3113

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3114

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3114

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3114

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3114

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3114

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3114

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3114

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3114

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3114

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.21.1	Conditions for RSRP measurements	3115

B.3.21.2	Conditions for RSRQ measurements	3115

B.3.21.3	Conditions for CSI-RSRP measurements	3115

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.22.1	Conditions for RSRP measurements	3115

B.3.22.2	Void	3116

B.3.22.3	Conditions for CSI-RSRP measurements	3116

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3116

B.3.23.1	Conditions for RSRP measurements	3116

B.3.23.2	Conditions for RSRQ measurements	3116

B.3.23.3	Conditions for CSI-RSRP measurements	3116

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3117

B.3.24.1	Conditions for RSRP measurements	3117

B.3.24.2	Conditions for RSRQ measurements	3117

B.3.24.3	Conditions for CSI-RSRP measurements	3117

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3117

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3117

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3117

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3118

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3118

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3118

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3118

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3118

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3119

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3119

B.4	RRM Requirements Exceptions	3119

B.4.1	General	3119

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3119

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3119

B.4.3.1	Inter-band carrier aggregation	3119

B.4.3.2	Intra-band non-contiguous carrier aggregation	3119

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3120

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3120

B.5.1	Conditions for S-RSRP Accuracy Requirements	3120

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3120

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3121

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3121

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3122

B.6	Conditions for V2X	3122

B.6.1	Test parameters for GNSS signals	3122

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3122

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3122

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3123

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3123

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3123

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3123

Annex C (informative): Change history:	3125