3GPP TS 37.145-1 V14.10.0 (2021-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Active Antenna System (AAS) Base Station (BS)

conformance testing;

Part 1: Conducted conformance testing

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	12

1	Scope	13

2	References	13

3	Definitions, symbols and abbreviations	15

3.1	Definitions	15

3.2	Symbols	18

3.3	Abbreviations	20

4	General test conditions and declarations	22

4.1	Measurement uncertainties and test requirements	22

4.1.1	General	22

4.1.2	Acceptable uncertainty of Test System	22

4.1.2.1	General	22

4.1.2.2	Measurement of transmitter	23

4.1.2.3	Measurement of receiver	24

4.1.3	Interpretation of measurement results	26

4.2	Conducted and radiated requirement reference points	27

4.3	Base station classes for AAS BS	27

4.4	Regional requirements	28

4.5	Operating bands and band categories	28

4.6	Channel arrangements	29

4.7	Requirements for AAS BS capable of multi-band operation	29

4.8	AAS BS configurations	30

4.8.1	Transmit configurations	30

4.8.2	Receive configurations	30

4.8.3	Power supply options	31

4.8.4	BS with integrated Iuant BS modem	31

4.9	Capability sets	31

4.10	Manufacturer declarations for AAS BS testing	31

4.11	Test signal configurations for AAS BS	37

4.11.1	General	37

4.11.2	Test signal configurations	37

4.11.2.1	ATC1: UTRA multicarrier operation	37

4.11.2.1.1	General	37

4.11.2.1.2	ATC1a generation	37

4.11.2.1.3	ATC1b generation	37

4.11.2.1.4	ATC1 power allocation	38

4.11.2.2	ANTC1: UTRA FDD multicarrier non-contiguous operation	38

4.11.2.2.1	General	38

4.11.2.2.2	ANTC1 generation	38

4.11.2.2.3	ANTC1 power allocation	38

4.11.2.3	ATC2: E-UTRA multicarrier operation	38

4.11.2.3.1	General	38

4.11.2.3.2	ATC2a generation	38

4.11.2.3.3	ATC2b generation	39

4.11.2.3.4	ATC2 power allocation	39

4.11.2.4	ANTC2: E-UTRA multicarrier non-contiguous operation	39

4.11.2.4.1	General	39

4.11.2.4.2	ANTC2 generation	39

4.11.2.4.3	ANTC2 power allocation	40

4.11.2.5	ATC3: UTRA and E-UTRA multi-RAT operation	40

4.11.2.5.1	General	40

4.11.2.5.2	ATC3a generation	40

4.11.2.5.3	ATC3b generation	41

4.11.2.5.4	ATC3 power allocation	41

4.11.2.6	ANTC3: UTRA and E-UTRA multi RAT non-contiguous operation	41

4.11.2.6.1	General	41

4.11.2.6.2	ANTC3a generation	41

4.11.2.6.3	ANTC3 power allocation	42

4.11.2.7	ATC4: Single carrier for receiver tests	42

4.11.2.7.1	ATC4a generation	42

4.11.2.7.2	ATC4b generation	42

4.11.2.7.3	ATC4c generation	42

4.11.2.8	Generation of MB-MSR test configurations	42

4.11.2.8.1	ATC5a: MB-MSR test configuration for full carrier allocation	42

4.11.2.8.2	ATC5b: MB-MSR test configuration with high PSD per carrier	43

4.12	RF channels and test models	44

4.12.1	RF channels	44

4.12.2	Test models	45

4.13	Format and interpretation of tests	46

5	Applicability of Requirements	47

5.1	General	47

5.2	Test configurations for TAB connectors for operating bands where MSR is supported	48

5.3	Test configurations for multi-carrier capable TAB connector(s) in operating bands where one RAT capability sets are supported	50

5.3.1	General	50

5.3.2	TAB connector supporting one RAT only MSR in the operating band	50

5.3.3	TAB connector supporting Single-RAT UTRA in the operating band	54

5.3.4	TAB connector supporting Single-RAT E-UTRA in the operating band	55

5.4	Test configurations for Multi-band TAB connectors	56

5.4.1	Multi-band TAB connector supporting MSR operation	56

5.4.2	Multi-band TAB connector supporting Single-RAT only	58

6	Conducted transmitter characteristics	59

6.1	General	59

6.2	Base station output power	60

6.2.1	General	60

6.2.2	Maximum output power	60

6.2.2.1	Definition and applicability	60

6.2.2.2	Minimum Requirement	60

6.2.2.3	Test Purpose	60

6.2.2.4	Method of test	60

6.2.2.4.1	Initial conditions	60

6.2.2.4.2	Procedure	61

6.2.2.5	Test Requirements	61

6.2.3	UTRA FDD primary CPICH power	61

6.2.3.1	Definition and applicability	61

6.2.3.2	Minimum requirement	61

6.2.3.3	Test purpose	62

6.2.3.4	Method of test	62

6.2.3.4.1	Initial conditions	62

6.2.3.4.2	Procedure	62

6.2.3.5	Test requirements	62

6.2.4	UTRA TDD primary CCPCH power	63

6.2.4.1	Definition and applicability	63

6.2.4.2	Minimum requirement	63

6.2.4.3	Test purpose	63

6.2.4.4	Method of test	63

6.2.4.4.1	Initial conditions	63

6.2.4.4.2	Procedure	64

6.2.4.5	Test requirements	64

6.2.5	UTRA FDD additional CPICH power for MIMO mode	65

6.2.5.1	Definition and applicability	65

6.2.5.2	Minimum requirement	65

6.2.5.3	Test purpose	65

6.2.5.4	Method of test	65

6.2.5.4.1	Initial conditions	65

6.2.5.4.2	Procedure	66

6.2.5.5	Test requirements	66

6.2.6	E-UTRA DL RS power	67

6.2.6.1	Definition and applicability	67

6.2.6.2	Minimum requirement	67

6.2.6.3	Test purpose	67

6.2.6.4	Method of test	67

6.2.6.4.1	Initial conditions	67

6.2.6.4.2	Procedure	67

6.2.6.5	Test requirements	68

6.3	Output power dynamics	68

6.3.1	General	68

6.3.2	UTRA Inner loop power control in the downlink	68

6.3.2.1	Definition and applicability	68

6.3.2.2	Minimum requirement	68

6.3.2.3	Test purpose	68

6.3.2.4	Method of test	68

6.3.2.4.1	Initial conditions	68

6.3.2.4.2	Procedure	69

6.3.2.5	Test requirements	70

6.3.2.5.1	UTRA FDD	70

6.3.2.5.2	UTRA TDD	70

6. 3.3	Power control dynamic range	71

6.3.3.1	Definition and applicability	71

6.3.3.2	Minimum requirement	71

6.3.3.3	Test purpose	71

6.3.3.4	Method of test	71

6.3.3.4.1	Initial conditions	71

6.3.3.4.2	Procedure	72

6.3.3.5	Test requirements	73

6.3.3.5.1	UTRA FDD	73

6.3.3.5.2	UTRA TDD	73

6. 3.4	Total power dynamic range	73

6.3.4.1	Definition and applicability	73

6.3.4.2	Minimum requirement	74

6.3.4.3	Test purpose	74

6.3.4.4	Method of test	74

6.3.4.4.1	Initial conditions	74

6.3.4.4.2	Procedure	74

6.3.4.5	Test requirements	75

6.3.4.5.1	UTRA FDD	75

6.3.4.5.2	E-UTRA	75

6.3.5	IPDL time mask	75

6.3.5.1	Definition and applicability	75

6.3.5.2	Minimum requirement	75

6.3.5.3	Test purpose	76

6.3.5.4	Method of test	76

6.3.5.4.1	Initial conditions	76

6.3.5.4.2	Procedure	76

6.3.5.5	Test requirements	76

6.3.6	RE Power control dynamic range	77

6.3.6.1	Definition and applicability	77

6.3.6.2	Minimum requirement	77

6.3.6.3	Method of test	77

6.4	Transmit ON/OFF power	77

6.4.1	General	77

6.4.2	Transmitter OFF power	77

6.4.2.1	Definition and applicability	77

6.4.2.2	Minimum requirement	78

6.4.2.3	Test purpose	78

6.4.2.4	Method of test	78

6.4.2.4.1	Initial conditions	78

6.4.2.4.2	Procedure	78

6.4.2.5	Test requirements	78

6.4.3	Transmitter transient period	79

6.4.3.1	Definition and applicability	79

6.4.3.2	Minimum requirement	79

6.4.3.3	Test purpose	79

6.4.3.4	Method of test	79

6.4.3.4.1	Initial conditions	79

6.4.3.4.2	Procedure	80

6.4.3.5	Test requirements	81

6.4.3.5.1	MSR operation	81

6.4.3.5.1	UTRA TDD operation	81

6.4.3.5.1	E-UTRA operation	81

6.5	Transmitted signal quality	81

6.5.1	General	81

6.5.2	Frequency Error	82

6.5.2.1	Definition and applicability	82

6.5.2.2	Minimum Requirement	82

6.5.2.3	Test purpose	82

6.5.2.4	Method of test	82

6.5.2.5	Test Requirements	82

6.5.2.5.1	UTRA FDD test requirement	82

6.5.2.5.2	UTRA TDD test requirement	82

6.5.2.5.3	E-UTRA test requirement	82

6.5.3	Time alignment error	83

6.5.3.1	Definition and applicability	83

6.5.3.2	Minimum requirement	83

6.5.3.3	Test purpose	83

6.5.3.4	Method of test	83

6.5.3.4.1	Initial conditions	83

6.5.3.4.2	Procedure	84

6.5.3.5	Test requirement	85

6.5.3.5.1	UTRA FDD test requirement	85

6.5.3.5.2	UTRA TDD test requirement	85

6.5.3.5.3	E-UTRA test requirement	85

6.5.4	Modulation quality	86

6.5.4.1	Definition and applicability	86

6.5.4.2	Minimum Requirement	86

6.5.4.3	Test purpose	86

6.5.4.4	UTRA FDD Method of test	86

6.5.4.4.1	Initial conditions	86

6.5.4.4.2	Procedure	86

6.5.4.5	UTRA TDD Method of test	87

6.5.4.5.1	Initial conditions	87

6.5.4.5.2	Procedure	88

6.5.4.6	E-UTRA Method of test	90

6.5.4.6.1	Initial conditions	90

6.5.4.6.2	Procedure	90

6.5.4.7	Test Requirements	90

6.5.4.7.1	UTRA FDD test requirement	90

6.5.4.7.2	UTRA TDD test requirement	91

6.5.4.7.3	E-UTRA test requirement	91

6.6	Unwanted Emissions	92

6.6.1	General	92

6.6.2	Occupied bandwidth	92

6.6.2.1	Definition and applicability	92

6.6.2.2	Minimum requirement	92

6.6.2.3	Test purpose	92

6.6.2.4	Method of test	93

6.6.2.4.1	Initial conditions	93

6.6.2.4.2	Procedure	93

6.6.2.5	Test requirements	95

6.6.2.5.1	MSR	95

6.6.2.5.2	UTRA FDD	95

6.6.2.5.3	UTRA TDD	95

6.6.2.5.4	E-UTRA	96

6.6.3	Adjacent Channel Leakage power Ratio	96

6.6.3.1	Definition and applicability	96

6.6.3.2	Minimum requirement	96

6.6.3.3	Test purpose	96

6.6.3.4	Method of test	96

6.6.3.4.1	Initial conditions	96

6.6.3.4.2	Procedure	98

6.6.3.5	Test requirements	100

6.6.3.5.1	General Requirements	100

6.6.3.5.2	Absolute Limits	100

6.6.3.5.3	MSR	100

6.6.3.5.4	UTRA FDD	103

6.6.3.5.5	UTRA TDD, 1,28Mcps option	105

6.6.3.5.6	E-UTRA	105

6.6.4	Spectrum emission mask	108

6.6.4.1	Definition and applicability	108

6.6.4.2	Minimum requirement	108

6.6.4.3	Test purpose	108

6.6.4.4	Method of test	108

6.6.4.4.1	Initial conditions	108

6.6.4.4.2	Procedure	109

6.6.4.5	Test requirements	110

6.6.4.5.1	General	110

6.6.4.5.2	Basic Limits	110

6.6.5	Operating band unwanted emission	122

6.6.5.1	Definition and applicability	122

6.6.5.2	Minimum requirement	122

6.6.5.3	Test purpose	122

6.6.5.4	Method of test	122

6.6.5.4.1	Initial conditions	122

6.6.5.4.2	Procedure	123

6.6.5.5	Test requirements	123

6.6.5.5.1	General	123

6.6.5.5.2	Basic Limits for MSR Band Categories 1 and 3	124

6.6.5.5.3	Basic Limits for MSR Band Category 2	129

6.6.5.5.4	Basic Limits for MSR Additional requirements	134

6.6.5.5.5	Basic Limits for E-UTRA	136

6.6.6	Spurious emission	159

6.6.6.1	Definition and applicability	159

6.6.6.2	Minimum requirement	159

6.6.2.3	Test purpose	159

6.6.6.4	Method of test	159

6.6.6.4.1	Initial conditions	159

6.6.6.4.2	Procedure	160

6.6.6.5	Test requirements	160

6.6.6.5.1	General	160

6.6.6.5.2	Basic limits	161

6.7	Transmitter intermodulation	179

6.7.1	Definition and applicability	179

6.7.1.1	General	179

6.7.2	Minimum requirement	180

6.7.3	Test purpose	180

6.7.4	Method of test	180

6.7.4.1	Initial conditions	180

6.7.4.2	Procedure	180

6.7.5	Test requirements	183

6.7.5.1	MSR test requirements	183

6.7.5.1.1	General test requirement	183

6.7.5.1.2	Additional test requirement (BC1 and BC2)	183

6.7.5.1.3	Additional test requirement (BC3)	184

6.7.5.1.4	Intra-system test requirement	184

6.7.5.2	Single RAT UTRA operation	184

6.7.5.2.1	General test requirement for UTRA FDD	184

6.7.5.2.2	General test requirement for UTRA TDD	184

6.7.5.2.3	Intra-system test requirement	185

6.7.5.4	Single RAT E-UTRA operation	185

6.7.5.4.1	General test requirement	185

6.7.5.4.2	Additional test requirement for Band 41	185

6.7.5.4.3	Intra-system test requirement	185

7	Conducted receiver characteristics	185

7.1	General	185

7.2	Reference sensitivity level	186

7.2.1	Definition and applicability	186

7.2.2	Minimum Requirement	186

7.2.3	Test Purpose	186

7.2.4	Method of test	187

7.2.4.1	Initial conditions	187

7.2.4.2	Procedure	187

7.2.5	Test Requirements	187

7.2.5.1	UTRA FDD operation	187

7.2.5.2	UTRA TDD 1,28 Mcps option operation	188

7.2.5.3	E-UTRA operation	188

7.3	Dynamic range	189

7.3.1	Definition and applicability	189

7.3.2	Minimum requirement	189

7.3.3	Test purpose	190

7.3.4	Method of test	190

7.3.4.1	Initial conditions	190

7.3.4.2	Procedure	190

7.3.5	Test requirements	190

7.3.5.1	UTRA FDD operation	190

7.3.5.2	UTRA TDD 1,28 Mcps option operation	191

7.3.5.3	E-UTRA operation	191

7.4	Adjacent channel selectivity and narrowband blocking	192

7.4.1	Definition and applicability	192

7.4.2	Minimum requirement	193

7.4.4	Method of test	193

7.4.4.1	Initial conditions	193

7.4.4.2	Procedure	193

7.4.4.2.1	General procedure	193

7.4.4.2.2	MSR operation	194

7.4.4.2.3	Single RAT UTRA FDD operation	194

7.4.4.2.4	Single RAT UTRA TDD 1,28Mcps option operation	195

7.4.4.2.5	Single RAT E-UTRA operation	195

7.4.5	Test requirements	196

7.4.5.1	MSR operation	196

7.4.5.1.1	General blocking test requirement	196

7.4.5.1.2	General narrowband blocking test requirement	197

7.4.5.1.3	Additional BC3 blocking test requirement	198

7.4.5.2	Single RAT UTRA FDD operation	198

7.4.5.3	Single RAT UTRA TDD 1,28 Mcps option operation	199

7.4.5.4	Single RAT E-UTRA operation	199

7.5	Blocking	201

7.5.1	Definition and applicability	201

7.5.2	Minimum requirement	201

7.5.3	Test purpose	201

7.5.4	Method of test	202

7.5.4.1	Initial conditions	202

7.5.4.2	Procedure	202

7.5.4.2.1	General Procedure	202

7.5.4.2.2	MSR operation	202

7.5.4.2.3	Single RAT UTRA FDD operation	203

7.5.4.2.4	Single RAT UTRA TDD 1,28 Mcps option operation	203

7.5.4.2.5	Single RAT E-UTRA operation	203

7.5.5	Test requirements	204

7.5.5.1	MSR operation	204

7.5.5.1.1	General out-of-band blocking test requirements	204

7.5.5.1.2	Co-location test requirements	205

7.5.5.2	Single RAT UTRA FDD operation	209

7.5.5.3	Single RAT UTRA TDD 1,28 Mcps option operation	228

7.5.5.3.1	General requirements	228

7.5.5.3.2	Co-location with GSM, DCS, UTRA FDD and/or E-UTRA FDD, UTRA TDD and/or E-UTRA TDD	232

7.5.5.4	Single RAT E-UTRA operation	236

7.5.5.4.1	General test requirement	236

7.5.5.4.2	Co-location with other base stations	240

7.6	Receiver spurious emissions	249

7.6.1	Definition and applicability	249

7.6.2	Minimum Requirement	249

7.6.3	Test Purpose	250

7.6.4	Method of test	250

7.6.4.1	Initial conditions	250

7.6.4.2	Procedure	250

7.6.4.2.1	General procedure	250

7.6.4.2.2	MSR operation	250

7.6.4.2.3	Single RAT UTRA FDD operation	250

7.6.4.2.4	Single RAT UTRA TDD 1,28Mcps option operation	251

7.6.4.2.5	Single RAT E-UTRA operation	252

7.6.5	Test Requirements	252

7.6.5.1	General	252

7.6.5.2	Basic limits	253

7.6.5.2.1	MSR operation	253

7.6.5.2.2	Single RAT UTRA FDD operation	253

7.6.5.2.3	Single RAT UTRA TDD 1,28Mcps option operation	254

7.6.5.2.4	Single RAT E-UTRA operation	254

7.7	Receiver intermodulation	255

7.7.1	Definition and applicability	255

7.7.2	Minimum requirement	255

7.7.3	Test purpose	255

7.7.4	Method of test	255

7.7.4.1	Initial conditions	255

7.7.4.2	Procedure	256

7.7.4.2.1	General procedure	256

7.7.4.2.2	MSR operation	256

7.7.4.2.3	Single RAT UTRA FDD operation	256

7.7.4.2.4	Single RAT UTRA TDD 1,28Mcps option operation	257

7.7.4.2.5	Single RAT E-UTRA operation	257

7.7.5	Test requirements	257

7.7.5.1	MSR operation	257

7.7.5.1.1	General intermodulation test requirement	257

7.7.5.1.2	General narrowband intermodulation test requirement	258

7.7.5.2	Single RAT UTRA FDD operation	260

7.7.5.3	Single RAT UTRA TDD 1,28Mcps option operation	261

7.7.5.4	Single RAT E-UTRA operation	262

7.8	In-channel selectivity	265

7.8.1	Definition and applicability	265

7.8.2	Minimum requirement	265

7.8.3	Test purpose	265

7.8.4	Method of test	265

7.8.4.1	Initial conditions	265

7.8.4.2	Procedure	266

7.8.5	Test requirements	266

8	Performance requirements	267

8.1	General	267

8.2	Performance requirements MSR	268

8.3	Performance requirements UTRA FDD	268

8.3.1	Definition and applicability	268

8.3.2	Minimum Requirement	269

8.3.3	Test purpose	269

8.3.4	Method of test	269

8.3.4.1	Initial Conditions	269

8.3.4.2	Procedure	269

8.3.5	Test Requirement	269

8.4	Performance requirements UTRA TDD	270

8.4.1	Definition and applicability	270

8.4.2	Minimum Requirement	270

8.4.3	Test purpose	270

8.4.4	Method of test	270

8.4.4.1	Initial Conditions	270

8.4.4.2	Procedure	271

8.4.5	Test Requirement	271

8.5	Performance requirements E-UTRA	271

8.5.1	Definition and applicability	271

8.5.2	Minimum Requirement	272

8.5.3	Test purpose	272

8.5.4	Method of test	272

8.5.4.1	Initial Conditions	272

8.5.4.2	Procedure	272

8.5.5	Test Requirement	272

Annex A (normative):  Characteristics of interfering signals	273

Annex B (normative):  Environmental requirements for the BS equipment	274

B.1	General	274

B.2	Normal test environment	274

B.3	Extreme test environment	274

B.3.1	General	274

B.3.2	Extreme temperature	274

B.4	Vibration	275

B.5	Power supply	275

B.6	Measurement of test environments	275

Annex C (informative): Test tolerances and derivation of test requirements	276

Annex D (informative):  Measurement system set-up	277

D.1	Transmitter	277

D.1.1	AAS BS output power, transmitter ON/OFF power, modulation quality, frequency error and operating band unwanted emissions	277

D.1.2	Transmitter intermodulation	278

D.1.3	Transmitter spurious emissions	278

D.2	Receiver	280

D.2.1	Reference sensitivity level	280

D.2.2	Receiver Dynamic Range	280

D.2.3	Receiver Adjacent channel selectivity and narrowband blocking	281

D.2.4	Receiver spurious emissions	281

D.2.5	Receiver In-channel selectivity	282

D.2.6	Receiver Intermodulation	283

Annex E (informative): Change history	284

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the conducted test methods and conformance requirements for single RAT E-UTRA operation, single RAT UTRA operation (FDD and TDD) and Multi-Standard Radio (MSR) UTRA and E-UTRA Active Antenna System (AAS) Base Station (BS). These have been derived from, and are consistent with the non-AAS BS specifications in TS 25.104 [2], TS 25.105 [3], TS 36.104 [4] or TS 37.104 [5]. The technical specification TS 37.145 is in 2 parts, part TS 37.145-1  (the present document) covers conducted requirements and part TS 37.145-2 covers radiated requirements.

The present document does not establish conducted test methods and conformance requirements for Band 46 operation as it is not supported by AAS BS, but Band 46 test requirements are still applicable for AAS BS for protection of and against Band 46 operation.

The present document does not establish conducted test methods and conformance requirements for Narrow-Band Internet of Things (NB-IoT) in band, NB-IoT guard band, or standalone NB-IoT operation, for AAS BS in single RAT E-UTRA operation as defined in TS 36.141 [14], or for AAS BS in MSR operation using E-UTRA as defined in TS 37.141 [13].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD)"

[3]	3GPP TS 25.105: "Base Station (BS) radio transmission and reception (TDD)"

[4]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

[5]	3GPP TS 37.104: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception"

[6]	3GPP TS 37.105: "Active Antenna System (AAS) Base Station (BS) transmission and reception"

[7]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[8]	3GPP TS 37.105 (V14.1.0): "Active Antenna System (AAS) Base Station (BS) radio transmission and reception (Release 14)"

[9]	3GPP TS 25.104 (V14.2.0): "Base Station (BS) radio transmission and reception (FDD) (Release 14)"

[10]	3GPP TS 25.105 (V14.0.0): "Base Station (BS) radio transmission and reception (TDD) (Release 14)"

[11]	3GPP TS 36.104 (V14.4.0): "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception (Release 14)"

[12]	3GPP TS 37.104 (V14.4.0): "E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception (Release 14)"

[13]	3GPP TS 37.141:"E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing"

[14]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing"

[15]	3GPP TS 25.141: "Base Station (BS) conformance testing (FDD)"

[16]	3GPP TS 37.141 (V14.4.0): "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing (Release 14)"

[17]	3GPP TS 36.141 (V14.4.0): "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing (Release 14)"

[18]	3GPP TS 25.141 (V14.2.0): "Base Station (BS) conformance testing (FDD) (Release 14)"

[19]	3GPP TS 25.142: "Base Station (BS) conformance testing (TDD)"

[20]	3GPP TS 25.142 (V14.0.0): "Base Station (BS) conformance testing (TDD) (Release 14)"

[21]	3GPP TR 25.942: "Radio Frequency (RF) system scenarios"

[22]	3GPP TS 45.004: "Digital cellular telecommunications system (Phase 2+); Modulation"

[23]	3GPP TS 25.214: "Physical layer procedures (FDD)"

[24]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission

[25]	CEPT ECC Decision (13)03: "The harmonised use of the frequency band 1452-1492 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)"

[26]	IEC 60721: "Classification of environmental conditions"

[27]	IEC 60721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[28]	IEC 60721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 4: Stationary use at non-weather protected locations"

[29]	ETSI EN 300 019-1-3: "Environmental Engineering (EE); Environmental conditions and environmental tests for telecommunications equipment; Part 1-3: Classification of environmental conditions; Stationary use at weatherprotected locations"

[30]	ETSI EN 300 019-1-4: "Environmental Engineering (EE); Environmental conditions and environmental tests for telecommunications equipment; Part 1-4: Classification of environmental conditions; Stationary use at non-weatherprotected locations"

[31]	IEC 60068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[32]	IEC 60068-2-2 (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[33]	IEC 60068-2-6 (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[34]	Recommendation ITU-T O.153: "Basic parameters for the measurement of error performance at bit rates below the primary rate"

[35]	Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain"

[36]	FCC publication number 662911: "Emissions Testing of Transmitters with Multiple Outputs in the Same Band"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

active antenna system base station: base station system which combines an Antenna Array with an Active transceiver unit array and a Radio Distribution Network

active receiver unit: number of active receivers is the same as the [number of receiver diversity branches] to which compliance is declared for clause 8 performance requirements

active transmitter unit: transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to one or more TAB connectors at the transceiver array boundary

band category: group of operating bands for which the same MSR scenarios apply

Base Station RF Bandwidth: bandwidth in which a base station transmits and/or receives single or multiple carrier(s) and/or RATs simultaneously within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth

basic limit: emissions limit taken from the non AAS specifications that is converted into a per TAB connector TX cell group AAS emissions limit by scaling

beam: main lobe of a radiation pattern from an AAS BS

NOTE:	For certain AAS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the -3 dB EIRP contour of the beam

beam direction pair: data set consisting of  the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is supposed to be found

beamwidth: angles describing the major and minor axes of an ellipsoid closest fit to an essentially elliptic half-power contour of a beam

carrier: modulated waveform conveying the physical channels

carrier aggregation: aggregation of two or more E-UTRA component carriers in order to support wider transmission bandwidths

channel bandwidth: RF bandwidth supporting a single RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell

NOTE 1:	The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	For UTRA FDD, the channel bandwidth is the nominal channel spacing specified in TS 25.104 [2], For UTRA TDD 1,28 Mcps option, the channel bandwidth is the nominal channel spacing specified in TS 25.105 [3].

NOTE 3:	For E-UTRA, the channel bandwidths are specified in TS 36.104 [4]. Standalone NB-IoT channel bandwidths specified in TS 36.104 [4] are not applicable to AAS BS.

code domain power: part of the mean power which correlates with a particular (OVSF) code channel in a UTRA signal

NOTE:	The sum of all powers in the code domain equals the mean power in a bandwidth of (1+ ) times the chip rate of the radio access mode.

contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s)

downlink operating band: part of the (FDD) operating band designated for downlink

EIRP accuracy directions set: beam peak directions for which the EIRP accuracy requirement is intended to be met.

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE 1:	The sensitivity is the minimum received power level at which a RAT specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (0 dBi).

inter-band gap: frequency gap between two supported consecutive operating bands

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

maximum carrier output power per TAB connector: mean power level measured on a particular carrier at the array boundary antenna connectors, during the transmitter ON period in a specified reference condition

maximum throughput: maximum achievable throughput for a reference measurement channel

MSR operation: operation of AAS BS declared to be MSR in particular operating band(s)

multi-band requirements: requirements applying per one single operating band with exclusion bands or other multi-band provisions as defined for each requirement

multi-band TAB connector: TAB connector supporting operation in multiple operating bands through common active electronic component(s)

NOTE:	For common TX and RX TAB connectors, the definition applies where common active electronic components are in the transmit path and/or in the recieve path.

NB-IoT in-band operation: NB-IoT is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier

NB-IoT guard band operation: NB-IoT is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA carrier's guard-band

NB-IoT standalone operation: NB-IoT is operating standalone when it utilizes its own spectrum, for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment

non-AAS BS: BS  conforming to one of the RF requirement specifications TS 25.104 [2], TS 25.105 [3], TS 36.104 [4] or TS 37.104 [5]

NOTE:	For AAS BS in single RAT E-UTRA operation or in MSR operation using E-UTRA, the NB-IoT operation (including in-band, guard band and standalone operation) is excluded from the consideration in the performance comparison among AAS BS and non-AAS BS in this specification.

non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s)

OTA sensitivity directions declaration: set of manufacturer declarations comprising one or more EIS values (with related RAT and channel bandwidth), and the directions where it (they) applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier

radio distribution network: passive network which distributes radio signals generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping

rated carrier output power per TAB connector: mean power level associated with a particular carrier the manufacturer has declared to be available at the TAB connector, during the transmitter ON period in a specified reference condition

rated total output power per TAB connector: mean power level associated with a particular operating band the manufacturer has declared to be available at the TAB connector, during the transmitter ON period in a specified reference condition

receiver target: angles of arrival in which reception is performed

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD

receiver target reference direction: direction inside the receiver target redirection range declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA

reference beam direction: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy compliance directions set

sensitivity RoAoA: RoAoA within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific AAS BS direction setting

single band requirements: requirements applying per one single operating band without exclusion bands or other multi-band provisions

single band TAB connector: TAB connector supporting operation either in a single operating band only, or in multiple operating bands but without any common active electronic component(s)

single RAT E-UTRA operation: operation of AAS BS declared to be single RAT E-UTRA in the operating band

NOTE:	Single RAT E-UTRA operation does not cover in-band NB-IoT, nor guardband NB-IoT operation.

single RAT UTRA operation: operation of AAS BS declared to be single RAT UTRA in the operating band

sub-block: one contiguous allocated block of spectrum for use by the same Base Station

NOTE:	There may be multiple instances of sub-blocks within an Base StationRF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within an Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

TAB connector: transceiver array boundary connector

TAB connectors beam forming group: Group of TAB connectors associated with an EIRP beam declaration, comprising of the complete set of TAB connectors from which a declared beam is transmitted

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which requirements are applied

NOTE:	The group corresponds to the group of TAB connectors which are responsible for receiving a cell when the AAS BS setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which requirements are applied.

NOTE:	The group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the AAS BS setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

throughput: number of payload bits successfully received per second for a reference measurement channel in a specified reference condition

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: bandwidth of an instantaneous E-UTRA transmission from a UE or BS, measured in resource block units

transmitter OFF period: time period during which the transmitter is scheduled not to transmit

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter ON period: time period during which the transmitter is transmitting data and/or reference symbols

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

uplink operating band: part of the (FDD) operating band designated for uplink

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Roll-off factor

BWChannel	Channel bandwidth (for E-UTRA)

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz. BWChannel_CA= Fedge_high- Fedge_low

BWConfig	Transmission bandwidth configuration (for E-UTRA), expressed in MHz, where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink

DwPTS	Downlink part of the special subframe (for E-UTRA TDD operation

f	Frequency

f	Separation between the Base Station RF bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax	The largest value of f used for defining the requirement

FC	Carrier centre frequency

Ffilter	Filter centre frequency

f_offset	Separation between the Base Station RF bandwidth edge frequency and the centre of the measuring filter

f_offsetmax	The maximum value of f_offset used for defining the requirement

FBW RF,high	Upper RF bandwidth edge, where FBW RF,high = FC,high + Foffset, RAT

FBW RF,low	Lower RF bandwidth edge, where FBW RF,low = FC,low - Foffset, RAT

FC,high	Centre frequency of the highest transmitted/received carrier

FC,low	Centre frequency of the lowest transmitted/received carrier

Fedge_low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset

Fedge_high	The upper edge of aggregated channel bandwidth, expressed in MHz. Fedge_high = FC_high + FoffsetFoffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper RF bandwidth edge, sub-block edge or inter-RF bandwidth edge, or from the centre frequency of the lowest transmitted/received carrier to the lower RF bandwidth edge, sub-block edge or inter-RF bandwidth edge for a specific RAT

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fuw	Frequency offset of unwanted signal

Ncells	A declared number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all TAB connectors supporting the operating band

NRB	Transmission bandwidth configuration, expressed in units of resource blocks (for E-UTRA)

NRXU,active	The number of active receiver units. The same as the [number of receiver diversity branches] to which compliance is declared for chapter 8 performance requirements.

NRXU,counted	The number of active receiver units that are taken into account for unwanted emission scaling.

NRXU,countedpercell	The number of active receiver units that are taken into account for unwanted emissions scaling per cell

NTXU, active	The number of active transmitter units

NTXU,counted	The number of active transmitter units , as calculated in clause 6.1, that are taken into account for conducted TX power limit

NTXU,countedpercell	The number of active transmitter units  that are taken into account for emissions scaling per cell

PEM,B32,ind	Declared emission level in Band 32, ind=a, b, c, d, e (see table 4.10, D6.8)

Pmax,c,TABC	The maximum carrier output power per TAB connector

PRated,c,TABC	The rated carrier output power per TAB connector

PRated,c,sys	The sum of PRated,c,TABC for all TAB connectors for a single carrier

PRated,t,group	The sum of PRated,t,TABC for all TAB connectors belonging to a specified group

PRated,t,TABC	The rated total output power per TAB connector

PREFSENS	Reference Sensitivity power level

Wgap	Sub-block gap size or Inter RF Bandwidth gap size

Figure 3.2-1: Illustration of RF bandwidth related symbols and definitions for Multi-standard Radio

Figure 3.2-2: Illustration of RF bandwidth related symbols and definitions
for non-contiguous Multi-standard Radio

Figure 3.2-3: Illustration of maximum Radio Bandwidth and
Total RF bandwidth for Multi-band Multi-standard Radio

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AAS BS	Active Antenna System Base Station

ACLR	Adjacent Channel Leakage power Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

ARFCN	Absolute Radio Frequency Channel Number

B	Bottom RF channel (for testing purposes)

BC	Band Category

BER	Bit Error Rate

BLER	Block Error Rate

CA	Carrier Aggregation

CACLR	Cumulative ACLR

CP	Cyclic prefix

CRC	Cyclic Redundancy Check

CW	Continuous Wave (unmodulated signal)

DC-HSDPA	Dual Cell HSDPA

D-CPICH	Demodulation Common Pilot Channel

DIP	Dominant Interferer Proportion

EARFCN	E-UTRA Absolute Radio Frequency Channel Number

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

EVM	Error Vector Magnitude

FCC	Federal Communications Commission

FDD	Frequency Division Duplex

FRC	Fixed Reference Channel

HS-DSCH	High Speed Downlink Shared Channel

Iuant	E-Node B internal logical interface between the implementation specific O&M function and the RET antennas and TMAs control unit function of the E-Node B

ITU	International Telecommunication Union

ITU-R	Radio communication Sector of the ITU

LA	Local Area

M	Middle RF channel (for testing purposes)

MB-MSR	Multi-Band Multi-Standard Radio

MBT	Multi-Band Testing

MIMO	Multiple Inputs Multiple Outputs

MR	Medium Range

MSR	Multi-Standard Radio

NB-IoT	Narrowband – Internet of Things

OBW	Occupied Band Width

OFDM	Orthogonal Frequency Division Multiplex

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

PCCPCH	Primary Common Control Physical CHannel

QAM	Quadrature Amplitude Modulation

QPSK	Quadrature Phase-Shift Keying

RAT	Radio Access Technology

RB	Resource Block (for E-UTRA)

RDN	Radio Distribution Network

RE	Resource Element

RF	Radio Frequency

RoAoA	Range of Angles of Arrival

RRC	Root Raised Cosine

RS	Reference Symbol

SBT	Single Band Testing

SC	Single Carrier

SNR	Signal-to-Noise Ratio

T	Top RF channel (for testing purposes)

TAB	Tranceiver Array Boundary

TDD	Time Division Duplex

TT	Test Tolerance

UARFCN	UTRA Absolute Radio Frequency Channel Number

UE	User Equipment

WA	Wide Area

# 4	General test conditions and declarations

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in part 1 of this specification,, i.e. to all conducted test

The minimum requirements are given in TS 37.105 [6] and the references therein. Test requirements are given in this specification or are included by reference to TS 25.141 [15], TS 25.142 [19], TS 36.141 [14] or TS 37.141 [13]. Test Tolerances for the conducted test requirements explicitly stated in the present document are given in annex C of the present document. Test Tolerances for test requirements included by reference to TS 25.141 [15], TS 25.142 [19], TS 36.141 [14] or TS 37.141 [13] are defined in the respective referred test specification.

Test requirements and Test Tolerances for NB-IoT in-band, NB-IoT guard band, or standalone NB-IoT operation are not supported by AAS BS and not covered by this specification. When referring to standalone E-UTRA test requirements for single RAT operation in TS 36.141 [14] or to E-UTRA test requirements for MSR operation in TS 37.141 [13], any test requirements specified in those specifications for E-UTRA with NB-IoT (in-band or guard band) or for standalone NB-IoT, shall not be considered for the AAS BS. Unless otherwise stated, the exclusion of the NB-IoT test requirements in this specification applies to all operation modes (i.e. in-band NB-IoT operation, guard band NB-IoT operation and standalone NB-IoT operation).

Test Tolerances are individually calculated for each test. The Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

### 4.1.2	Acceptable uncertainty of Test System

#### 4.1.2.1	General

The maximum acceptable uncertainty of the Test System is specified below for each test defined explicitly in the present specification, where appropriate. The maximum acceptable uncertainty of the Test System for test requirements included by reference is defined in the respective referred test specification.

When a requirement is applied per TAB connector then the test uncertainty is applied to the measured value. When a requirement is applied for a group of TAB connectors then the test uncertainty is applied to sum of the measured power on each TAB connector in the group.

The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

For RF tests, it should be noted that the uncertainties in clause 4.1.2 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the DUT and the Test System.

#### 4.1.2.2	Measurement of transmitter

Table 4.1.2.2-1: Maximum Test System uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Base Station output power | ±0.7 dB for UTRA and E-UTRA, f ≤ 3.0 GHz ±1.0 dB, 3.0 GHz < f ≤ 4.2 GHz for UTRA and E-UTRA |  |
| 6.4 Transmit ON/OFF power | ±2.0 dB , f ≤ 3.0 GHz ±2.5 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.6.6 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB |  |
| 6.6.6 Transmitter spurious emissions, Additional BC2 Requirement | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 12.75 GHz: ±4.0 dB |  |
| 6.6.6 Transmitter spurious emissions, Protection of BS receiver | ±3.0 dB |  |
| 6.6.6 Transmitter spurious emissions, Additional spurious emission requirements | ±2.0 dB for > -60 dBm , f ≤ 3.0 GHz ±2.5 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.0 dB for ≤ -60 dBm , f ≤ 3.0 GHz ±3.5 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.6.6 Transmitter spurious emissions, Co-location | ±3.0 dB |  |
| 6.6.5 Operating band unwanted emissions | ±1.5 dB , f ≤ 3.0 GHz ±1.8 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.6.2 Occupied bandwidth | For E-UTRA: 1.4 MHz, 3 MHz Channel BW: ±30 kHz 5 MHz, 10 MHz Channel BW: ±100 kHz 15 MHz, 20 MHz: Channel BW: ±300 kHz For UTRA: ±100 kHz |  |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ACLR ±0.8 dB Absolute power ±2.0 dB, f ≤ 3.0 GHz Absolute power ±2.5 dB, 3.0 GHz < f ≤ 4.2 GHz CACLR ±0.8 dB CACLR absolute power ±2.0 dB , f ≤ 3.0 GHz CACLR absolute power ±2.5 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.7 Transmitter intermodulation (interferer requirements) This tolerance applies to the stimulus and not the measurements defined in 6.6.6, 6.6.5 and 6.6.3 | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests (6.6.1, 6.6.2 and 6.6.4) which have to be carried out in the presence of the interferer. ±1.0 dB | The uncertainty of interferer has double the effect on the result due to the frequency offset |

#### 4.1.2.3	Measurement of receiver

Table 4.1.2.3-1: Maximum Test System Uncertainty for receiver tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.4 Adjacent channel selectivity and narrowband blocking | ±1.4 dB , f ≤ 3.0 GHz ±1.8 dB, 3.0 GHz < f ≤ 4.2 GHz | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer leakage Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer leakage effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + leakage effect. f ≤ 3.0 GHz Wanted signal level ±0.7 dB Interferer signal level ±0.7 dB 3.0 GHz < f ≤ 4.2 GHz Wanted signal level ±1.0 dB Interferer signal level ±1.0 dB f ≤ 4.2 GHz Impact of interferer leakage 0.4 dB |
| 7.5 Blocking | 1 MHz finterferer 3 GHz: ±1.3 dB 3 GHz < finterferer 12.75 GHz: ±3.2 dB | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + Broadband noise effect. Out of band blocking, using CW interferer: Wanted signal level: ±0.7 dB up to 3 GHz ±1.0 dB up to 4.2 GHz Interferer signal level: ±1.0 dB up to 3 GHz ±3.0 dB up to 12.75 GHz Impact of interferer Broadband noise 0.1 dB |
| 7.6 Receiver spurious emissions | 30 MHz ≤ f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB |  |
| 7.7 Receiver intermodulation (General requirements) | ±1.8 dB , f ≤ 3.0 GHz ±2.4 dB, 3.0 GHz < f ≤ 4.2 GHz | Overall system uncertainty comprises four quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error 4. Impact of interferer ACLR The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] + ACLR effect. f ≤ 3.0 GHz Wanted signal level ± 0.7dB CW interferer level ± 0.5 dB Mod interferer level ± 0.7 dB 3.0 GHz < f ≤ 4.2 GHz Wanted signal level ± 1.0 dB CW Interferer level ± 0.7 dB Mod Interferer level ± 1.0 dB f ≤ 4.2 GHz Impact of interferer ACLR 0.4 dB |
| 7.7 Receiver intermodulation (Narrowband requirements) | ±1.8 dB , f ≤ 3.0 GHz ±2.4 dB, 3.0 GHz < f ≤ 4.2 GHz | Same as Receiver intermodulation (General requirements). |
| NOTE: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements or the BER/FER due to finite test duration is not considered. |  |  |

### 4.1.3	Interpretation of measurement results

The measurement results returned by the Test System are compared - without any modification - against the test requirements as defined by the Shared Risk principle.

The Shared Risk principle is defined in Recommendation ITU-R M.1545 [7].

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause 4.1.2 of the present document.

If the Test System for a test is known to have a measurement uncertainty greater than that specified in clause 4.1.2, it is still permitted to use this apparatus provided that an adjustment is made as follows.

Any additional uncertainty in the Test System over and above that specified in clause 4.1.2 shall be used to tighten the test requirement, making the test harder to pass. For some tests e.g. receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause 4.1.2 does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause 4.1.2 had been used.

## 4.2	Conducted and radiated requirement reference points

AAS BS requirements are defined for two points of reference, signified by radiated requirements and conducted requirements.

Figure 4.2-1: Radiated and conducted points of reference of AAS BS

Radiated characteristics are defined over the air (OTA) with a point of reference in the far field (Fraunhofer) region. Radiated requirements are also referred to as OTA requirements.

Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to receive/send parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network that distributes the RF power between the transceiver array boundary and the antenna array, in an implementation specific way.

How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement clause.

Part 1 of this specification details the test requirements of the conducted requirements only and hence only requires the conducted reference points.

## 4.3	Base station classes for AAS BS

The requirements in this specification apply to AAS BS of Wide Area BS, Medium Range BS and Local Area BS classes unless otherwise stated.

Wide Area BS are characterised by requirements derived from Macro Cell scenarios. For AAS BS of Wide Area BS class, the minimum coupling loss between any TAB connector and the UE is 70 dB.

NOTE:	Whenever WA BS is referred in this specification, the NB-IoT Wide Area BS and related requiremetns as defined in TS 36.104 [4], are not applicable for AAS BS.

Medium Range BS are characterised by requirements derived from Micro Cell scenarios. For AAS BS of Medium Range BS class, the minimum coupling loss between any TAB connector and the UE is 53 dB.

Local Area BS are characterised by requirements derived from Pico Cell scenarios. For AAS BS of Local Area BS class, the minimum coupling loss between any TAB connector and the UE is 45 dB.

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions. Non-AAS requirements are applicable as defined in the present document. In many cases, such requirements include regional requirements that are implicitly referenced from the present specification, and listed in the specification for the specifications concerned [2] [5].

Table 4.4-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 4.5 | Operating bands and Band Categories | Some bands may be applied regionally. |
| 6.2.2.2.1 | Base station output power | These requirements apply in Japan for an E-UTRA BS operating in Band 34 and Band 41. |
| 6.6.2 | Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the Occupied bandwidth according to the definition. |
| 6.6.4 | Spectrum emission mask | The mask specified may be mandatory in certain regions. In other regions this mask may not be applied. Additional spectrum protection requirements may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | The BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied and under the conditions declared by the manufacturer. |
| 6.6.5 | Operating band unwanted emissions | The requirements for protection of DTT may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | Regional requirement as defined in TS 37.104, clause 6.6.2.4.4 [12] may be applied for the protection of systems operating in frequency bands adjacent to Band 1 as defined in TS 37.104, clause 4.5, [12] in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed. |
| 6.6.5 | Operating band unwanted emissions | Additional requirements for Band 41 may apply in certain regions as additional Operating band unwanted emission limits. |
| 6.6.5 | Operating band unwanted emissions | Additional requirements defined for Band 24 in 3GPP TS 37.104, subclause 6.6.2.4.5 may apply in regions where FCC regulation applies. |
| 6.6.5 | Operating band unwanted emissions | Additional Band 32 unwanted emissions requirements may apply in certain regions. |
| 6.6.6 | Spurious emissions | Category A limits are mandatory for regions where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [35] apply. Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [35] apply. |
| 6.6.6 | Spurious emissions | Additional spurious emissions requirements may be applied for the protection of system operating in frequency ranges other than the AAS BS operating band as described in TS 37.104 [12] clause 6.6.1.3. |
| 6.6.6 | Spurious emissions | In addition to 3GPP requirements, the BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.6 | Spurious emissions | The emission limits specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |
| 6.7 | Transmitter intermodulation | Additional requirements may apply in certain regions. |
| 7.6 | Rx spurious emissions | The emission limits specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |

## 4.5	Operating bands and band categories

The operating bands and band categories for AAS BS are the same as for non-AAS BS, as described in TS 37.104 [5].

NOTE 1:	AAS BS does not support GSM, but BC2 is still applicable for protection of/against GSM operation in BC2 operating bands.

NOTE 2:	AAS BS does not support Band 46 (and all its sub-bands defined in TS 36.104 [12], clause 5.5) operation, but Band 46 test requirements are still applicable for AAS BS for protection of and against Band 46 operation.

NOTE 3:	Bands in BC1 and BC2 categories are also used for NB-IoT operation. NB-IoT is not applicable for AAS BS.

## 4.6	Channel arrangements

The channel arrangements for AAS BS are the same as those for UTRA non-AAS BS and E-UTRA non-AAS BS as described in TS 37.104 [5].

NOTE:	Test requirements for nominal carrier spacing of 19.8 MHz and 20.1 MHz for carriers in Band 46 as specified in 36.104 [12] clause 5.7.1, are not applicable for AAS BS.

## 4.7	Requirements for AAS BS capable of multi-band operation

For AAS BS capable of operation in multiple operating bands, the RF requirements in clause 6 and 7 apply separately to each supported operating band unless otherwise stated.

An AAS BS may be capable of supporting operation in multiple operating bands with one of the following implementations of TAB connectors in the transceiver array boundary:

-	All TAB connectors are single band TAB connectors:

-	Different sets of single band TAB connectors support different operating bands, but each TAB connector supports only operation in one single operating band.

-	Sets of single band TAB connectors support operation in multiple operating bands with some single band TAB connectors supporting more than one operating band.

-	All TAB connectors are multiband TAB connectors.

-	A combination of single band sets and multi-band sets of TAB connectors provides support of the AAS BS capability of operation in multiple operating bands.

Unless otherwise stated all requirements specified for an operating band apply only to the set of TAB connectors supporting that operating band.

In certain requirements it is explicitly stated that specific additions or exclusions to the requirement apply at multi-band TAB connectors as detailed in the requirement clause.

In the case of an operating band being supported only by single band TAB connectors in a TAB connector TX min cell group or a TAB connector RX min cell group, single band requirements apply to that set of TAB connectors.

NOTE:	Each supported operating band needs to be operated separately during conformance testing on single band TAB connectors.

In the case of an operating band being supported only by multi-band TAB connectors supporting the same operating band combination in a TAB connector TX min cell group or a TAB connector RX min cell group, multi-band requirements apply to that set of TAB connectors.

The case of an operating band being supported by both multi-band TAB connectors and single band TAB connectors in a TAB connector TX min cell group or a TAB connector RX min cell group is not covered by the present release of this specification.

The case of an operating band being supported by multi-band TAB connectors which are not all supporting the same operating band combination in a TAB connector TX min cell group or a TAB connector RX min cell group is not covered by the present release of this specification.

For multi-band TAB connectors supporting the bands for TDD, the RF requirements in the present specification assume no simultaneous uplink and downlink occur between the bands.

The RF requirements for multi-band TAB connectors supporting bands for both FDD and TDD are not covered by the present release of this specification.

## 4.8	AAS BS configurations

### 4.8.1	Transmit configurations

Unless otherwise stated, the conducted transmitter characteristics in clause 6 are specified at the AAS BS transceiver array boundary at the TAB connector(s) antenna connector with a full complement of transceiver units for the configuration in normal operating conditions.

Figure 4.8.1-1: Transmitter test ports

Unless otherwise stated, for the tests in clause 6 of the present document, the requirement applies for each transmit TAB connector.

### 4.8.2	Receive configurations

Unless otherwise stated, the conducted receiver characteristics in clause 7 are specified at the TAB connector with a full complement of transceiver units for the configuration in normal operating conditions.

Figure 4.8.2-1: Receiver test ports

For the tests in clause 7 of the present document, the requirement applies at each receive TAB connector.

Conducted receive requirements are tested at the TAB connector, with the remaining receiver units(s) disabled or their TAB connector(s) being terminated.

### 4.8.3	Power supply options

If the AAS BS is supplied with a number of different power supply configurations, it may not be necessary to test RF parameters for each of the power supply options, provided that it can be demonstrated that the range of conditions over which the equipment is tested is at least as great as the range of conditions due to any of the power supply configurations.

### 4.8.4	BS with integrated Iuant BS modem

Unless otherwise stated, for the tests in the present document, the integrated Iuant BS modem shall be switched off. Spurious emissions according to clauses 6.6.4 and 7.6 shall be measured only for frequencies above 20 MHz with the integrated Iuant BS modem switched on.

## 4.9	Capability sets

Capability set is defined as the TAB connectors capability to support certain RAT combinations in an operating band.

The manufacturer shall declare the supported capability set(s) according to table 4.9-1 for each supported TAB connector and supported operating band.

Table 4.9-1 Capability sets

| Capability Set supported by the BS | CSA1 | CSA2 | CSA3 | CSA4 | CSA5 |
| --- | --- | --- | --- | --- | --- |
| Supported RATs | TAB connector supports MSR operation of UTRA only in the band | TAB connector supports MSR operation of E-UTRA only in the band | TAB connector supports MSR in the band | TAB connector supports single-RAT UTRA in the band | TAB connector supports single-RAT E-UTRA in the band |
| Supported configurations | SR UTRA (SC, MC) | SR E-UTRA (SC, MC, CA) | MR UTRA + E-UTRA SR UTRA (SC, MC) SR E-UTRA (SC, MC, CA) | SR UTRA (SC, MC) | SR E-UTRA (SC, MC, CA) |
| Applicable BC | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 |

The applicable test configurations for each RF requirement are defined in clauses 5.1, 5.2 and 5.3 for the declared capability set(s). For a multi-band TAB connector the applicable test configurations for each RF requirement are defined in clause 5.4 for the declared capability set(s).

NOTE:	Not every supported configuration within a capability set is tested, but the tables in clauses 5.2, 5.3 and 5.4 provide a judicious choice among the supported configurations and test configurations to ensure proper test coverage.

## 4.10	Manufacturer declarations for AAS BS testing

The following declarations are required.

Table 4.10-1 Manufacturers declarations

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D6.1 | Operating bands and frequency ranges | List of operating band(s) supported by TAB connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Declarations shall be made per TAB connector. |
| D6.3 | Spurious emission category | Declare the AAS BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [35]. |
| D6.4 | Geographic area support | The manufacturer shall declare the regions the AAS BS may operate in. e.g. CEPT. |
| D6.5 | Band 20 or Band XX support, operating in geographical areas allocated to broadcasting (DTT) | If the AAS BS has TAB connectors declared to support Band 20 the manufacturer shall declare if the AAS BS may operate in geographical areas allocated to broadcasting (DTT). |
| D6.6 | Band 20 or Band XX support, emission level for channel N (PEM,N) | If the AAS BS has TAB connectors declared to support Band 20 or Band XX and has been declared to operate in geographical areas allocated to broadcasting (DTT), the emission level for channel N (Annex G of TS 36.104 [11]) shall be declared. |
| D6.7 | Band 20 or Band XX support, Maximum output Power in 10 MHz (P10MHz) | If the AAS BS has TAB connectors declared to support Band 20 or Band XX and has been declared to operate in geographical areas allocated to broadcasting (DTT), the maximum output power in 10 MHz (Annex G of TS 36.104 [11]) shall be declared. |
| D6.8 | Band 32 or Band XXXII support, Declared emission level in Band 32/XXXII (PEM,B32,ind) | If the AAS BS has TAB connectors declared to support Band 32 or Band XXXII the manufacturer shall declare if the AAS BS may operate in geographical areas allocated to broadcasting (DTT), the emission level in Band 32/XXXII (PEM,B32,ind, ind=a, b, c, d, e) shall be declared. |
| D6.9 | Band 24 support, Declared emission level in Band 24 (PEM,B24,ind) | If the AAS BS has TAB connectors declared to support Band 24 the manufacturer shall declare if the AAS BS may operate in geographical areas where FCC regulations apply, the emission level in Band 24 (PEM,B24,ind, ind=a, b, c, d, e, f) shall be declared. |
| D6.10 | Co-existence with other systems | The manufacturer shall declare whether the AAS BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another band are deployed. |
| D6.11 | Co-location with other base stations | The manufacturer shall declare whether the AAS BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD and/or E-UTRA operating in another band. |
| D6.12 | TAB connector capability set (CSA) | The manufacturer shall declare the supported capability set(s) according to table 4.9-1 for all TAB connector(s) and supported operating band. |
| D6.13 | Single or Multi-band TAB connector | Multi-band TAB connector or single band TAB connector. Declared for every TAB connector |
| D6.14 | Contiguous or non-contiguous spectrum | Ability to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier, per TAB connector, per band, per RAT. |
| D6.15 | Contiguous and non-contiguous parameters identical | If contiguous and non-contiguous operation is possible then parameters are the same. |
| D6.16 | Maximum Radio Bandwidth of the multi-band TAB connector. | Largest radio bandwidth that can be supported by the multi-band TAB connector. May be different for transmit and receive. Declared for each supported operating band and operating band combination (D6.41) supported for every multi-band TAB connector. |
| D6.17 | Maximum Base Station RF Bandwidth | Largest Base Station RF Bandwidth in the operating band, declared for each supported operating band for every TAB connector. |
| D6.18 | Maximum Base Station RF Bandwidth for contiguous operation. | Largest Base Station RF Bandwidth for contiguous spectrum operation, declared for each supported operating band and CS for every TAB connector. |
| D6.19 | Maximum Base Station RF Bandwidth for non- contiguous operation. | Largest Base Station RF Bandwidth for non-contiguous spectrum operation, declared for each supported operating band and CS for every TAB connector. |
| D6.20 | E-UTRA supported channel bandwidths | E-UTRA channel bandwidth supported for each supported operating band for every TAB connector. |
| D6.21 | TAB connector operating band support | List of operational bands and band combinations supported by the TAB connector, declared for every TAB connector. |
| D6.22 | CA only operation | Capable of operating with CA only but not multiple carriers. Declared per operating band per TAB connector. |
| D6.23 | Single or multiple carrier | Capable of operating with a single carrier (only) or multiple carriers per operating band, per RAT for all TAB connectors. |
| D6.24 | maximum number of supported carriers per band | Maximum number of supported carriers per supported band, made per band, per RAT for all TAB connectors. |
| D6.25 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported bands, declared for every TAB connector. |
| D6.26 | Reduced number of supported carriers at the rated total output power in Multi-RAT operations | Declared for each supported operating band for all TAB connectors. |
| D6.27 | Reduced total output power at the total number of supported carriers in Multi-RAT operations | Declared for each supported operating band for all TAB connectors. |
| D6.28 | Other band combination multi-band restrictions | Declare any other limitations under simultaneous operation in the declared band combinations (D6.41) for each multi-band TAB connector which have any impact on the test configuration generation. For every multi-band TAB connector. |
| D6.30 | The rated carrier output power for each TAB connector PRated,c,TABC | PRated,c,TABC, is declared per supported operating band, per supported RAT for all TAB connector(s). |
| D6.31 | The rated carrier output power per TAB connector, for contiguous spectrum operation | The rated carrier output power per TAB connector, for contiguous spectrum operation. Declared for each supported operational band and CS, for all TAB connectors |
| D6.32 | The rated carrier output power per TAB connector, for non-contiguous spectrum operation | The rated carrier output power per TAB connector, for non-contiguous spectrum operation. Declared for each supported operational band and CS, for all TAB connectors |
| D6.33 | The rated output power per RAT for each TAB connector, PRated,RAT,TABC | PRated,RAT,TABC is declared per supported operating band, per supported RAT for all TAB connector(s). |
| D6.34 | The rated total output power per TAB connector, PRated,t,TABC | The rated total output power PRated,t,TABC is declared for supported operating band, for all TAB connector(s). For multi-band TAB connectors PRated,t,TABC is declared for each supported band in each supported band combination. |
| D6.35 | The rated total output power per TAB connector, for contiguous spectrum operation | The rated total output power per TAB connector, for contiguous spectrum operation. Declared for each supported operational band and CS, for all TAB connectors. |
| D6.36 | The rated total output power per TAB connector, for non-contiguous spectrum operation | The rated total output power per TAB connector, for non-contiguous spectrum operation. Declared for each supported operational band and CS, for all TAB connectors. |
| D6.37 | The rated multi-band total output power per TAB connector, PRated,MB,TABC | The rated multi-band total output power (PRated,MB,TABC), declared for all declared operating band combinations for every multi-band TAB connector. |
| D6.38 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all TAB connectors supporting the operating band. |
| D6.39 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band, for all TAB connector(s). |
| D6.40 | Maximum supported power difference between carriers is different operating bands | Supported power difference between any two carriers in any two different supported operating bands, for all declared operating band combination for every multi-band TAB connector(s).. |
| D6.41 | AAS BS operating band combination support | List of operational bands combinations supported by the AAS BS. |
| D6.42 | Total number of supported carriers for the declared band combinations of the AAS BS | Total number of supported carriers for the declared band combinations (D6.41) of the AAS BS. |
| D6.43 | Intra-system interfering signal TAB connector declaration list | List of TAB connectors for which an intra-system interfering signal level is required to be declared. Declaration is required if the intra- system interfering signal level is larger than the co-location interfering signal level. |
| D6.44 | Intra-system interfering signal level | The interfering signal level in dBm per TAB connector declared for each supported operational band, for all TAB connectors covered by D6.43. |
| D6.45 | P-CPICH transmission group(s) | Groups of TAB connectors which are declared to transmit the P-CPICH. Declared per operating band, UTRA FDD only. |
| D6.48 | CCPCH transmission group(s) | Groups of TAB connectors which are declared to transmit the CCPCH. Declared per operating band, UTRA TDD only |
| D6.51 | UTRA FDD MIMO support. | Number of 'antennas' supported by the UTRA FDD MIMO mode (i.e. 2 or 4). The concept of "antenna 2", "antenna 3" and "antenna 4" is described in TS 25.104 [2]. Declared per operating band, UTRA FDD only. |
| D6.52 | S-CPICH transmission group(s) | Groups of TAB connectors which are declared to transmit the S-CPICH for each of the 'antennas' declared in D6.49 For UTRA FDD AAS BS operating only "antenna 1" and "antenna 2", mapping for 'antenna 2' is declared. For UTRA FDD AAS BS operating "antenna 1", "antenna 2", "antenna 3" and "antenna 4", mapping for "antenna 2", "antenna 3", and "antenna 4", is declared. Declared per operating band, UTRA FDD only. NOTE: Mapping for "antenna 1" is the same as D6.45. |
| D6.54 | DL RS transmission groups | Groups of TAB connectors which are declared to transmit the DL RS. Declared per operating band, E-UTRA only. |
| D6.57 | UTRA Inner loop power control dynamic range | Power control dynamic range for UTRA inner loop power control. Declared for each supported operating band, for all TAB connector(s). UTRA only. |
| D6.58 | TAE groups | Set of declared TAB connector beam forming groups on which the TAE requirements apply. All TAB connectors belong to at least 1 TAB connector beam forming group (even if it's a TAB connector beam forming group consisting of 1 connector). The smallest possible number of TAB connector beam forming groups need to be declared such that there is no TAB connector not contained in at least one of the declared TAB connector beam forming groups. Declared for each supported RAT and operating band. |
| D6.59 | Inter-band CA or HSDPA | Band combinations declared to support inter-band CA or multi-band HSDPA. Declared for every multi-band TAB connector which support CA or multi-band HSDPA. NOTE: Inter-band HSDPA is called multi-band HSDPA in UTRA specifications. Examples of the multi-band HSDPA are DB-DC-HSDPA or Dual band 4C-HSDPA. |
| D6.60 | Intra-band contiguous CA or HSDPA | Bands declared to support intra-band contiguous CA (per CA capable TAB connector, as in D6.22) or contiguous multi-carrier HSDPA. Declared for every TAB connector which support CA or multi-band HSDPA. NOTE: Intra-band HSDPA is called multi-carrier HSDPA in UTRA specifications. Examples of the contiguous multi-carrier HSDPA are DC-HSDPA, 4C-HSDPA, or 8C-HSDPA. |
| D6.61 | Intra-band non-contiguous CA or HSDPA | Bands declared to support intra-band non-contiguous CA (per CA capable TAB connector, as in D6.22) or non-contiguous multi-carrier HSDPA. Declared for every TAB connector which support CA or multi-band HSDPA. NOTE: Intra-band HSDPA is called multi-carrier HSDPA in UTRA specifications. Example of the non-contiguous multi-carrier HSDPA is NC-4C-HSDPA. |
| D6.70 | Equivalent TAB connectors | List of TAB connectors which have been declared equivalent. Equivalent TAB connectors imply that the TAB connectors are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the TAB connectors are identical and the transmitter unit and/or receiver unit driving the TAB connector are of identical design. |
| D6.71 | BS class | BS Class of the AAS BS, declared as Wide Area BS, Medium Range BS, or Local Area BS. |
| D6.72 | TAB connector RX min cell group | Declared as a group of TAB connectors to which RX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for receiving a cell when the AAS BS setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |
| D6.73 | TAB connector TX min cell group | Declared group of TAB connectors to which TX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for transmitting a cell when the AAS BS setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |
| D6.74 | TAB connectors used for performance requirement testing | To reduce test complexity, declaration of a representative (sub)set of TAB connectors to be used for performance requirement test purposes. At least one TAB connector mapped to each demodulation branch is declared. |

## 4.11	Test signal configurations for AAS BS

### 4.11.1	General

The test configurations shall be constructed using the methods defined below subject to the parameters declared by the manufacturer as listed in clause 4.10.

For test contiguous spectrum operation configurations used in receiver tests only the carriers in the outermost frequency positions in the TAB connector Base Station RF Bandwidth need to be generated by the test equipment. For non-contiguous spectrum operation test configurations used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment.

The applicable test models for generation of the carrier transmit test signal are defined in clause 4.12.2.

NOTE:	In case carriers are shifted to align with the channel raster Foffset.

### 4.11.2	Test signal configurations

#### 4.11.2.1	ATC1: UTRA multicarrier operation

##### 4.11.2.1.1	General

The purpose of ATC1 is to test UTRA multi-carrier aspects.

##### 4.11.2.1.2	ATC1a generation

ATC1 should be constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D6.18).

-	Place one UTRA FDD carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA FDD carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset shall apply.

-	For transmitter tests, alternately place a UTRA FDD carrier adjacent to the already placed carriers at the low and high Base Station RF Bandwidth edges until there is no more space to fit a carrier or the TAB connector does not support more carriers. The nominal carrier spacing defined in clause 4.6 shall apply.

-	The carrier(s) may be shifted maximum 100 kHz towards lower frequencies for BRFBW and MRFBW and towards higher frequencies for TRFBW to align with the channel raster.

##### 4.11.2.1.3	ATC1b generation

ATC1b is constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D6.18).

-	Place one UTRA TDD carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA TDD carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset shall apply.

-	For transmitter tests, alternately place a UTRA TDD carrier adjacent to the already placed carriers at the low and high Base Station RF Bandwidth edges until there is no more space to fit a carrier or the TAB connector does not support more carriers. The nominal carrier spacing defined in clause 4.6 shall apply.

##### 4.11.2.1.4	ATC1 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals Prated,RAT,TABC for UTRA according to the manufacturer's declaration in clause 4.10.

#### 4.11.2.2	ANTC1: UTRA FDD multicarrier non-contiguous operation

##### 4.11.2.2.1	General

The purpose of ANTC1 is to test UTRA FDD multicarrier non-contiguous aspects.

##### 4.11.2.2.2	ANTC1 generation

The purpose of ANTC1a is to test UTRA multicarrier non-contiguous aspects. ANTC1 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D6.19) of the TAB connector. The station RF bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth for non-contiguous operation.

-	For transmitter tests, place one UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	For receiver tests, place one UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. For single-band operation, if the maximum Base Station RF Bandwidth for non-contiguous operation is at least 35 MHz and the TAB connector supports at least 4 UTRA FDD carriers, place a UTRA FDD carrier adjacent to each already placed carrier for each sub-block. The nominal carrier spacing defined in clause 4.6 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset-RAT for the carrier adjacent to the sub-block gap.

-	The UTRA FDD carrier in the lower sub-block may be shifted maximum 100 kHz towards lower frequencies and the UTRA FDD carrier in the upper sub-block may be shifted maximum 100 kHz towards higher frequencies to align with the channel raster.

##### 4.11.2.2.3	ANTC1 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals PRated,RAT,TABC according to the manufacturer's declaration in clause 4.10.

#### 4.11.2.3	ATC2: E-UTRA multicarrier operation

##### 4.11.2.3.1	General

The purpose of ATC2a is to test E-UTRA multi-carrier aspects excluding CA occupied bandwidth.

The purpose of ATC2b is to test E-UTRA contiguous CA occupied bandwidth.

##### 4.11.2.3.2	ATC2a generation

ATC2a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D6.18) of the TAB connector.

-	Select the narrowest supported E-UTRA carrier and place it adjacent to the low Base Station RF Bandwidth edge. Place a 5 MHz E-UTRA carrier adjacent to the high Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	For transmitter tests, select as many 5 MHz E-UTRA carriers that the TAB connector supports and that fit in the rest of the Base Station RF Bandwidth. Place the carriers adjacent to each other starting from the high Base Station RF Bandwidth edge. The nominal carrier spacing defined in clause 4.6 shall apply. The specified FOffset-RAT shall apply.

-	If 5 MHz E-UTRA carriers are not supported by the TAB connector the narrowest supported channel bandwidth shall be selected instead.

The test configuration should be constructed on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the TAB connector (see table 4.10-1, D6.29). All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

##### 4.11.2.3.3	ATC2b generation

ATC2b is constructed on a per band basis using the following method:

-	All component carrier combinations supported by the TAB connector, which have different sum of channel bandwidth of component carrier, shall be tested. For all component carrier combinations which have the same sum of channel bandwidth of component carriers, only one of the component carrier combinations shall be tested.

-	Of all component carrier combinations which have same sum of channel bandwidth of component carrier, select those with the narrowest carrier at the lower Base Station RF Bandwidth edge.

-	Of the combinations selected in the previous step, select one with the narrowest carrier at the upper Base Station RF Bandwidth edge.

-	If there are multiple combinations fulfilling previous steps, select the one with the smallest number of component carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the lowest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the highest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the carrier which has been selected in the previous step.

-	If there are multiple combinations fulfilling previous steps, repeat the previous step until there is only one combination left.

-	The nominal carrier spacing defined in clause 4.6 shall apply.

##### 4.11.2.3.4	ATC2 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals Prated,RAT,TABC for E-UTRA (see table 4.10-1, D6.33).

For a TAB connector declared to support only CA operation (see table 4.10-1, D6.22), set the power spectral density of each carrier to the same level so that the sum of the carrier powers equals the rated total output power PRated,t,TABC (see table 4.10-1, D6.34).

#### 4.11.2.4	ANTC2: E-UTRA multicarrier non-contiguous operation

##### 4.11.2.4.1	General

The purpose of ANTC2 is to test E-UTRA multi-carrier non-contiguous aspects.

##### 4.11.2.4.2	ANTC2 generation

ANTC2 is constructed as NTC2 in TS 37.141 [16] clause 4.8.2a.1.

ANTC2 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D6.19) of the TAB connector. The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth (see table 4.10-1, D.17).

-	For transmitter tests, place a 5 MHz E-UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the TAB connector, the narrowest supported channel bandwidth shall be selected instead.

-	For receiver tests, place a 5 MHz E-UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. If 5 MHz E-UTRA carriers are not supported by the TAB connector, the narrowest supported channel bandwidth shall be selected instead.

-	For single-band operation receiver tests, if the remaining gap is at least 15 MHz plus two times the channel bandwidth used in the previous step and the TAB connector supports at least 4 E-UTRA carriers, place a E-UTRA carrier of this channel bandwidth adjacent to each already placed carrier for each sub-block. The nominal carrier spacing defined in clause 4.6 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset-RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.4.3	ANTC2 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals Prated,RAT,TABC for E-UTRA (see table 4.10-1, D6.33).

#### 4.11.2.5	ATC3: UTRA and E-UTRA multi-RAT operation

##### 4.11.2.5.1	General

The purpose of ATC3 is to test UTRA and E-UTRA multi-RAT aspects.

If the rated total output power per TAB connector PRated,t,TABC (see table 4.10-1, D.34) and total number of supported carriers (see table 4.10-1, D.25) are not simultaneously supported in multi-RAT operations, two instances of ATC3 shall be generated using the following values for rated total output power and the total number of supported carriers:

1)	The rated total output power per TAB connector PRated,t,TABC (see table 4.10-1, D6.34) and the reduced number of supported carriers at the rated total output power in multi-RAT operations (see table 4.10-1, D6.26).

2)	The reduced total output power at the total number of supported carriers in multi-RAT operations (see table 4.10-1, D6.27) and the total number of supported carriers (see table 4.10-1, D6.25).

Tests that use ATC3 shall be performed using both instances 1) and 2) of ATC3.

##### 4.11.2.5.2	ATC3a generation

ATC3a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (see table 4.10-1, D6.17) of the TAB connector.

-	Select an FDD UTRA carrier to be placed at the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	Place a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. If that is not possible use the narrowest E-UTRA carrier supported by the TAB connector. The specified FOffset-RAT shall apply.

-	For transmitter tests, alternately add FDD UTRA carriers at the low end and 5 MHz E-UTRA carriers at the high end adjacent to the already placed carriers until the Base Station RF Bandwidth is filled or the total number of supported carriers (see table 4.10-1, D6.25) is reached. The nominal carrier spacing defined in clause 4.6 shall apply.

##### 4.11.2.5.3	ATC3b generation

ATC3b is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (see table 4.10-1, D6.17) of the TAB connector.

-	Select a UTRA TDD carrier to be placed at the lower Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply.

-	Place a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. If that is not possible use the narrowest E-UTRA carrier supported by the TAB connector. The specified FOffset-RAT shall apply.

-	For transmitter tests, alternately add UTRA TDD carriers at the low end and 5 MHz E-UTRA carriers at the high end adjacent to the already placed carriers until the Base Station RF Bandwidth is filled or the total number of supported carriers (see table 4.10-1, D6.25) is reached. The nominal carrier spacing defined in clause 4.6 shall apply.

##### 4.11.2.5.4	ATC3 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals PRated,RAT,TABC according to the manufacturer's declaration in clause 4.10.

#### 4.11.2.6	ANTC3: UTRA and E-UTRA multi RAT non-contiguous operation

##### 4.11.2.6.1	General

The purpose of ANTC3 is to test UTRA and E-UTRA multi-RAT non-contiguous aspects.

If the rated total output power per TAB connector PRated,t,TABC (see table 4.10-1, D.34) and total number of supported carriers (see table 4.10-1, D.25) are not simultaneously supported in multi-RAT operations, two instances of ANTC3 shall be generated using the following values for PRated,t,TABC and the total number of supported carriers:

1)	The rated total output power per TAB connector PRated,t,TABc (see table 4.10-1, D6.34) and the reduced number of supported carriers at the rated total output power in multi-RAT operations (see table 4.10-1, D6.26).

2)	The reduced total output power at the total number of supported carriers in multi-RAT operationssee table 4.10-1, D6.27) at the total number of supported carriers (see table 4.10-1, D6.25).

If the reduced number of supported carriers is 4 or more, only instance 1) of ANTC3 shall be used in the tests, otherwise both instances 1) and 2) of ANTC3 shall be used in the tests.

##### 4.11.2.6.2	ANTC3a generation

ANTC3a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D6.19) of the TAB connector. The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D6.19).

-	For transmitter tests, place an UTRA carrier at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the TAB connector, the narrowest supported channel bandwidth shall be selected instead. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	For receiver tests, place an UTRA carrier at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified FOffset-RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the TAB connector, the narrowest supported channel bandwidth shall be selected instead. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	For single-band operation receiver tests, if the remaining gap is at least 20 MHz plus the channel bandwidth of the E-UTRA carrier used in the previous step and the TAB connector supports at least 2 UTRA and 2 E-UTRA carriers, place a E-UTRA carrier of this channel bandwidth adjacent to the carrier at the lower Base Station RF Bandwidth edge and UTRA carrier adjacent to the carrier at the upper Base Station RF Bandwidth edge. The nominal carrier spacing defined in clause 4.6 shall apply. The UTRA FDD may be shifted maximum 100 kHz towards higher frequencies to align with the channel raster.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified FOffset-RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.6.3	ANTC3 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals the PRated,t,TABC according to the cases in clause 4.11.2.6.1.

#### 4.11.2.7	ATC4: Single carrier for receiver tests

##### 4.11.2.7.1	ATC4a generation

ATC4a is constructed using the following method:

-	Place a single UTRA carrier in the middle of the maximum supported Base Station RF Bandwidth. The carrier may be shifted maximum 100 kHz towards lower frequencies for BRFBW and MRFBW and towards higher frequencies for TRFBW to align with the channel raster.

##### 4.11.2.7.2	ATC4b generation

ATC4b is constructed using the following method:

-	Place the narrowest supported E-UTRA carrier in the middle of the maximum supported Base Station RF Bandwidth.

##### 4.11.2.7.3	ATC4c generation

ATC4c is constructed using the following method:

-	Place a single UTRA TDD carrier in the middle of the maximum supported Base Station RF Bandwidth.

#### 4.11.2.8	Generation of MB-MSR test configurations

##### 4.11.2.8.1	ATC5a: MB-MSR test configuration for full carrier allocation

# 4.11.2.8.1.1	General

The purpose of ATC5a is to test multi-band TAB connectors, considering maximum supported number of carriers.

# 4.11.2.8.1.2	ATC5a generation

ATC5a is based on re-using the existing test configurations applicable per band on multi-band TAB connectors. ATC5a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (see table 4.10-1, D6.17) of the multi-band TAB connector.

-	The number of carriers of each supported operating band shall be the declared maximum number of supported carriers by the multi-band TAB connector in each band (see table 4.10-1, D6.25). Carriers shall first be placed at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16) Additional carriers shall next be placed at the edges of the Base Station RF Bandwidths, if possible.

-	The allocated Base Station RF Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to the test configuration referenced in Table 4.11.2.8.1.2-1, where the declared parameters for multi-band operation shall apply. The mirror image of the single band test configuration shall be used in the highest band being tested for the TAB connector.

-	If a multi-band TAB connector supports three carriers only, two carriers shall be placed in one band according to the relevant test configuration while the remaining carrier shall be placed at the edge of the maximum Radio Bandwidth (see table 4.10-1, D6.16) in the other band.

-	If the sum of the base Station RF bandwidths of each of the supported operating bands is greater than the declared Radio Bandwidth (see table 4.10-1, D6.16) for the declared band combinations (see table 4.10-1, D6.41) of the TAB connector then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared Radio Bandwidth (see table 4.10-1, D6.16) of the TAB connector is not exceeded and vice versa.

-	If the sum of the maximum number of supported carrier of each supported operating bands for the multi-band TAB connector is larger than the declared total number of supported carriers for the declared band combinations (see table 4.10-1, D6.42) of the AAS BS, repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not be exceeded and vice versa.

Table 4.11.2.8.1.2-1: The applicability of test configuration for carrier placement in each band

| BC | CSA 1 | CSA 2 | CSA 3 | CSA 4 | CSA 5 |
| --- | --- | --- | --- | --- | --- |
| BC1 | ATC1a | ATC2a | ATC3a | ATC1a | ATC2a |
| BC2 | ATC1a | ATC2a | ATC3a | ATC1a | ATC2a |
| BC3 | ATC1b | ATC2a | ATC3b | ATC1b | ATC2a |

# 4.11.2.8.1.3	ATC5a power allocation

Unless otherwise stated, set the power of each carrier (PRated,c,TABC) in all supported operating bands to the same power so that the sum of the carrier powers equals the rated total output power (PRated,MB,TABC) according to the manufacturer's declaration.

If the allocated power of a supported operating band(s) exceeds the declared rated total output power of the operating band(s) (PRated,MB,TABC ) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the power allocated for a carrier exceeds the rated carrier output power declared for that carrier (PRated,c,TABC), the exceeded power shall, if possible, be reallocated into the other carriers.

##### 4.11.2.8.2	ATC5b: MB-MSR test configuration with high PSD per carrier

# 4.11.2.8.2.1	General

The purpose of ATC5b is to test multi-band operation aspects considering higher PSD cases with reduced number of carriers and non-contiguous operation (if supported) in multi-band mode.

# 4.11.2.8.2.2	ATC5b generation

ATC5b is based on re-using the existing test configurations applicable per band on multi-band TAB connectors. ATC5b is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (see table 4.10-1, D6.16) of the multi-band TAB connector.

-	The allocated Radio Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	The maximum number of carriers is limited to two per band. Carriers shall be placed at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to the test configuration referenced in Table 4.11.2.8.2.2-1, where the declared parameters for multi-band operation shall apply. Narrowest supported E-UTRA channel bandwidth shall be used in the test configuration. The mirror image of the single band test configuration shall be used in the highest band being tested for the TAB connector.

-	For AAS BS supporting CSA4 in the band, if a multi-band TAB connector supports three carriers only, two carriers shall be placed in one band according to ATC2 while the remaining carrier shall be placed at the edge of the Maximum Base Station RF Bandwidth in the other band.

-	If the sum of the base Station RF bandwidths of each of the supported operating bands is greater than the declared Radio Bandwidth (see table 4.10-1, D6.16) for the declared band combinations (see table 4.10-1, D6.41) of the TAB connector then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared Radio Bandwidth (see table 4.10-1, D6.16) of the TAB connector is not exceeded and vice versa.

Table 4.11.2.8.2.2-1: The applicability of test configuration for carrier placement in each band

| BC | CSA 1 | CSA 2 | CSA 3 | CSA 4 | CSA 5 |
| --- | --- | --- | --- | --- | --- |
| BC1 | ANTC1a | ANTC2 | ANTC3a | ANTC1 | ANTC2 |
| BC2 | ANTC1a | ANTC2 | ANTC3a | ANTC1 | ANTC2 |
| BC3 | ATC1b | ANTC2 | ANTC3a | N/A | ANTC2 |

# 4.11.2.8.2.3	ATC5b power allocation

Unless otherwise stated, set the power of each carrier (PRated,c,TABC) in all supported operating bands to the same power so that the sum of the carrier powers equals the rated total output power (PRated,MB,TABC) according to the manufacturer's declaration.

If the allocated power of a supported operating band(s) exceeds the declared rated total output power of the operating band(s) (PRated,t,TABC) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the power allocated for a carrier exceeds the rated carrier output power declared for that carrier (PRated,c,TABC), the exceeded power shall, if possible, be reallocated into the other carriers.

## 4.12	RF channels and test models

### 4.12.1	RF channels

For single carrier tests unless otherwise stated the tests shall be performed with a single carrier at each of the RF channels B, M and T.

Many tests in this TS are performed with the maximum Base Station RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BRFBW (bottom), MRFBW (middle) and TRFBW (top).

Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum Base Station RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW: maximum Base Station RF Bandwidth located in the middle of the supported frequency range in the operating band. MRFBW may be shifted maximum 100 kHz towards lower frequencies to align carriers with the channel raster.

-	TRFBW: maximum Base Station RF Bandwidth located at the top of the supported frequency range in the operating band.

For the test of certain conducted RF requirements the present specification refers to test procedures defined in the single-RAT specifications [15], [14], [19]. In this case, the interpretation of the RF channels to be tested shall be according to the definitions in the corresponding single-RAT specifications [15], [14], [19].

Occupied bandwidth test in this TS is performed with the Aggregated Channel Bandwidth and sub-block bandwidths located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BBW Channel CA(bottom), MBW Channel CA (middle) and TBW Channel CA (top) for contiguous spectrum operation.

Unless otherwise stated, the test for contiguous spectrum operation shall be performed at BBW Channel CA, MBW Channel CA and TBW Channel CA defined as following:

-	BBW Channel CA: Aggregated Channel Bandwidth located at the bottom of the supported frequency range in each operating band;

-	MBW Channel CA: Aggregated Channel Bandwidth located close in the middle of the supported frequency range in each operating band, with the center frequency of each component carrier aligned to the channel raster;

-	TBW Channel CA: Aggregated Channel Bandwidth located at the top of the supported frequency range in each operating band.

For a multi-band TAB connector capable of dual-band operation, unless otherwise stated, the test shall be performed at BRFBW_T'RFBW and B'RFBW_TRFBW defined as following:

-	BRFBW_ T'RFBW: the Base Station RF Bandwidths located at the bottom of the supported frequency range in the lower operating band and at the highest possible simultaneous frequency position, within the maximum Radio Bandwidth, in the upper operating band.

-	B'RFBW_TRFBW: the Base Station RF Bandwidths located at the top of the supported frequency range in the upper operating band and at the lowest possible simultaneous frequency position, within the maximum Radio Bandwidth, in the lower operating band.

NOTE:	BRFBW_T'RFBW = B'RFBW_TRFBW = BRFBW_TRFBW when the declared maximum Radio Bandwidth (see table 4.10-1, D6.16) spans both operating bands. BRFBW_TRFBW means the Base Station RF Bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band.

When a test is performed by a test laboratory, the position of B, M and T for single carrier, BRFBW, MRFBW and TRFBW for single band operation, BBW Channel CA, MBW Channel CA and TBW Channel CA for contiguous spectrum operation in each supported operating band, the position of BRFBW_T'RFBW and B'RFBW_TRFBW in the supported operating band combinations shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

### 4.12.2	Test models

a) Unless otherwise stated, carriers used for transmitter tests shall be configured as follows:

-	UTRA FDD carriers shall be configured according to TM1 as defined in TS 25.141 [18], clause 6.1.1.1.

-	UTRA TDD carriers shall be configured according to table 6.1A as defined in TS 25.142 [20], clause 6.2.4.1.2.

-	E-UTRA carriers shall be configured according to E-TM1 as defined in TS 36.141 [17], clause  6.1.1.1. For BC3 CS3 BS testing, E-UTRA carriers shall be configured according to E-TM1_BC3CS3 defined in annex E of TS 37.141 [16].

b)	The configuration of the carriers in test configurations used for testing modulation quality and frequency error shall be as follows:

-	For the case that modulation accuracy is measured for UTRA FDD, the UTRA FDD carriers shall be configured according to the supported TM1, TM4 and if HS-PDSCH transmission using 16QAM is supported also TM5 as defined in TS 25.141 [18], clauses 6.1.1.1, 6.1.1.4 and 6.1.1.4A whilst any remaining carriers from other RAT(s) shall be configured according to a).

-	For the case that modulation accuracy is measured for UTRA TDD, the UTRA TDD carriers shall be configured according to the supported modulation in table 6.2A, table 6.39A, table 6.39B, table 6.39C, table 6.39D, table 6.40A, table 6.40B, table 6.41A, table 6.41B as defined in TS 25.142 [20], clauses 6.3.4, 6.8.1, 6.8.2 and 6.8.3 whilst any remaining carriers from other RAT(s) shall be configured according to a).

-	For the case that modulation accuracy is measured for E-UTRA, the E-UTRA carriers shall be configured according to the supported E-TM3.1, E-TM3.2, E-TM3.3 and E-TM2 as defined in TS 36.141 [17] clauses 6.1.1.4, 6.1.1.5, 6.1.1.6 and 6.1.1.3 whilst any remaining carriers from other RAT(s) shall be configured according to a). For BC3 CS3 BS testing, E-UTRA carriers shall be configured according to E-TM3.1_BC3CS3, E-TM3.2_BC3CS3, E-TM3.3_BC3CS3 and E-TM2_BC3CS3 defined in annex E of TS 37.141 [16].

c) Unless otherwise stated, transmitter carriers used for receiver tests shall be configured as follows:

-	UTRA FDD carriers shall be configured according to TM1 as defined in TS 25.141 [18], clause 6.1.1.1.

-	UTRA TDD carriers shall be configured according to table 6.1A as defined in TS 25.142 [20], clause 6.2.4.1.2.

-	E-UTRA carriers shall be configured according to E-TM1 as defined in TS 36.141 [17], clause 6.1.1.1. For BC3 CS3 BS testing, E-UTRA carriers shall be configured according to E-TM1_BC3CS3 defined in annex E of TS 37.141 [16].

For the test of certain RF requirements clause 5 refers to the test configurations as defined in the single-RAT specifications. In this case, the transmitter test signals and test models as defined within the referred test specification for the RF requirement shall be used.

## 4.13	Format and interpretation of tests

Each test in the following clauses has a standard format:

X	Title

All tests are applicable to all equipment within the scope of the present document, unless otherwise stated.

X.1	Definition and applicability

This clause gives the general definition of the parameter under consideration and specifies whether the test is applicable to all equipment or only to a certain subset. Required manufacturer declarations may be included here.

X.2	Minimum requirement

This clause contains the reference to the clause to the 3GPP reference (or core) specification which defines the minimum requirement. For each requirement, there are separate references for MSR and single RAT, where applicable in the core requirement. If the requirement does not apply to a particular RAT, this is explicitly stated here (rather than through a reference).

X.3	Test purpose

This clause defines the purpose of the test.

X.4	Method of test

X.4.1	General

In some cases there are alternative test procedures or initial conditions. In such cases, guidance for which initial conditions and test procedures can be applied are stated here. In the case only one test procedure is applicable, that is stated here. Guidance to which TAB connectors are subject to the test is also given here.

X.4.2y	First test method

X.4.2y.1	Initial conditions

This clause defines the initial conditions for each test, including the test environment, the RF channels to be tested and the basic measurement set-up.  The test system is assumed to be correctly calibrated as part of the initial conditions. Calibration is not explicitly mentioned.

X.4.2y.2	Procedure

This clause describes the steps necessary to perform the test and provides further details of the test definition like point of access (e.g. test port), domain (e.g. frequency-span), range, weighting (e.g. bandwidth), and algorithms (e.g. averaging).  The procedure may comprise data processing of the measurement result before comparison with the test requirement (e.g. average result from several measurement positions).

X.4.3y	Alternative test method (if any)

If there are alternative test methods, each is described with its initial conditions and procedures.

X.5	Test requirement

This clause defines the pass/fail criteria for the equipment under test, see clause 4.1.3 Interpretation of measurement results. Test requirements for every minimum requirement referred in clause X.2 are listed here. Cases where minimum requirements do not apply need not be mentioned.

The test requirements may be different depending on the test method applied. A test requirement for each test method applicable to the respective MSR/Single RAT requirement is given in separate clauses where applicable.

# 5	Applicability of Requirements

## 5.1	General

The present clause defines for each conducted test requirement the set of mandatory test configurations which shall be used for demonstrating conformance for each TAB connector.

Test configurations for TAB connectors supporting multiple RAT in the tested operating band are specified in clause 5.2.

Test configurations for TAB connectors declared to support single RAT requirements (see table 4.10-1, D6.13) by either  MSR requirements for UTRA only or E-UTRA only or with a single-RAT UTRA requirements or single RAT E-UTRA requirements are specified in clause 5.3.

Test configurations for Multi-band TAB connectors are specified in clause 5.4.

Requirements apply to TAB connectors according to the declared RAT Capability Set (see table 4.10-1, D6.12) within each supported operating band  and capability set of the TAB connector and the Band Category of the declared operating band  (see table 4.10-1, D6.1), as listed in the heading of each table. Some RF requirements listed in the tables may not be mandatory or they may apply only regionally. This is further specified for each requirement in clauses 6 and 7, and in table 4.4-1.

For a declared RAT Capability Set (see table 4.10-1, D6.12) in tables 5.2-1, 5.3.2-1, 5.3.3-1, 5.3.4-1, 5.4.1-1 or 5.4.2.1 only the requirements listed in the column for that Capability Set apply. Requirements listed under CSA other than the declared CSA(s) need not be tested.

For a TAB connector declared to support MSR and to be capable of contiguous spectrum operation only, the test configuration(s) in tables 5.2-1 and 5.3.2-1 denoted by a "C" and entries that refer to single-RAT specifications shall be used for testing.

For a TAB connector that is declared: to support MSR and to be capable of contiguous and non-contiguous spectrum operation (see table 4.10-1, D6.14), where the parameters according to clause 4.10 are identical for contiguous and non-contiguous operation (see table 4.10-1, D6.15), shall use for each declared operating band the test configuration(s) in tables 5.2-1 and 5.3.2-1 denoted by "CNC" and entries that refer to single-RAT specifications shall be used.

For a TAB connector that is declared: to support MSR and to be capable of contiguous and non-contiguous spectrum operation (see table 4.10-1, D6.14), where the parameters according to clause 4.10 are not identical for contiguous and non-contiguous operation (see table 4.10-1, D6.15), shall use for each declared operating band the test configuration(s) in tables 5.2-1 and 5.3.2-1 denoted by "C/NC" and entries that refer to single-RAT specifications shall be used.

For a MSR capable multi-band TAB connector the applicability of the requirement for each operating band is determined by the RAT configuration within that operating band as identified in tables 5.2-1 and 5.3.2-1, unless otherwise stated. The testing of MSR capable multi-band TAB connectors shall be according to table 5.4.1-1 as follows:

-	For requirements test denoted by SBT (Single Band Test), the test configuration(s) in tables 5.2-1 and 5.3.2-1 shall be used for each operating band depending on the RAT configuration within that band.

-	For requirements test denoted by MBT (Multi-Band Test), the test configuration(s) in table 5.4.1-1 shall be used depending on the Band Category of the declared operating band combination.

For a single-RAT UTRA only TAB connector for each declared operating band clause 5.3.3 defines for each conducted test requirement the set of mandatory test configurations which shall be used for demonstrating conformance. The applicable test configurations are specified in table 5.3.3-1 for each supported RF configuration, which shall be declared according to clause 4.10. The generation and power allocation for each test configuration is defined in clause 4.11.2.

For a single-RAT E-UTRA only TAB connector for each declared operating band clause 5.3.4 defines for each conducted test requirement the set of mandatory test configurations which shall be used for demonstrating conformance. The applicable test configurations are specified in table 5.3.4-1 for each supported RF configuration, which shall be declared according to clause 4.10. The generation and power allocation for each test configuration is defined in clause 4.11.2.

For a TAB connector declared to be capable of single carrier operation only in an operating band (see table 4.10-1, D6.23), a single carrier (SC) shall be used for testing.

## 5.2	Test configurations for TAB connectors for operating bands where MSR is supported

Table 5.2-1: Test configuration applicability to requirements and
capability sets for TAB connectors supporting MSR operation

| TAB connector test case |  | UTRA + E-UTRA (CSA3) |  |  |
| --- | --- | --- | --- | --- |
|  |  | BC1 | BC2 | BC3 |
| 6.2 | Base Station output power | - | - | - |
| 6.2.2 | Base Station maximum output power | C: ATC3a CNC: ATC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ATC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | Additional regional requirement (only for band 34) | N/A | N/A | (note) |
| 6.2.3 | UTRA FDD primary CPICH power | Clause 5.3.3 | Clause 5.3.3 | N/A |
| 6.2.3A | UTRA FDD secondary CPICH power | Clause 5.3.3 | Clause 5.3.3 | N/A |
| 6.2.4 | UTRA TDD primary CCPCH power | N/A | N/A | Clause 5.3.3 |
| 6.2.6 | E-UTRA DL RS power | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.3 | Output power dynamics | - | - | - |
|  | E-UTRA | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A |
|  | UTRA TDD | N/A | N/A | Clause 5.3.3 |
| 6.4 | Transmit ON/OFF power | - | - | - |
| 6.4.1 | Transmitter OFF power | N/A | N/A | C: ATC3b |
| 6.4.2 | Transmitter transient period | N/A | N/A | C: ATC3b |
| 6.5 | Transmitted signal quality | - | - | - |
| 6.5.2 | Frequency error | - | - | - |
|  | E-UTRA | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 |
|  | UTRA FDD | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 | N/A |
|  | UTRA TDD | N/A | N/A | Same TC as used in clause 6.5.4 |
| 6.5.3 | Time alignment error | - | - | - |
|  | E-UTRA | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A |
|  | UTRA TDD | N/A | N/A | Clause 5.3.3 |
| 6.5.4 | Modulation quality - EVM | - | - | - |
|  | E-UTRA | C: ATC3a CNC: ATC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ATC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | UTRA FDD | C: ATC3a CNC: ATC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ATC3a C/NC: ATC3a, ANTC3a | N/A |
|  | UTRA TDD | N/A | N/A | C: ATC3b |
| 6.5.5 | Transmit pulse shape filter | Not tested | Not tested | Not tested |
| 6.6 | Unwanted Emissions | - | - | - |
| 6.6.2 | Occupied bandwidth | - | - | - |
|  | Minimum requirement | Clause 5.3.3 Clause 5.3.4 | Clause 5.3.3 Clause 5.3.4 | Clause 5.3.3 Clause 5.3.4 |
| 6.6.3 | Adjacent Channel Leakage power Ratio | - | - | - |
|  | E-UTRA | C: ATC2a CNC: ANTC2 C/NC:ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC:ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC:ATC2a, ANTC2 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A |
|  | UTRA TDD | N/A | N/A | Clause 5.3.3 |
|  | Cumulative ACLR | CNC: ANTC3a C/NC:ANTC3a | CNC: ANTC3a C/NC:ANTC3a |  |
| 6.6.5 | Operating band unwanted emission | - | - | - |
|  | General requirement for Band Categories 1 and 3 | Clause 5.3.3 Clause 5.3.4 C: ATC3a CNC: ATC3a, ANTC3a C/NC: ATC3a, ANTC3a | N/A | Clause 5.3.3 Clause 5.3.4 C: ATC3b |
|  | General requirement for Band Category 2 | N/A | Clause 5.3.3 Clause 5.3.4 C: ATC3a CNC: ATC3a, ANTC3a C/NC: ATC3a, ANTC3a | N/A |
|  | Additional requirements | (note) | (note) | (note) |
| 6.6.6 | Spurious emission | - | - | - |
|  | (Category A) | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | (Category B) | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | Additional requirement for BC2 (Category B) | N/A | N/A | N/A |
|  | Protection of the BS receiver of own or different BS | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | Additional spurious emissions requirements | C: ATC3a, CNC: ANTC3a, C/NC: ATC3a, ATC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | Co-location with other Base Stations | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
| 6.7 | Transmitter intermodulation | - | - | - |
|  | General requirement | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 |
|  | Additional requirement (BC1 and BC2) | CNC: ANTC3a C/NC:ANTC3a | Same TC as used in clause 6.6 | N/A |
|  | Additional requirement (BC3) | N/A | N/A | Same TC as used in clause 6.6 |
| 7.2 | Reference sensitivity level | - | - | - |
|  | E-UTRA requirement | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD requirement | Clause 5.3.3 | Clause 5.3.3 | N/A |
|  | UTRA TDD requirement | N/A | N/A | Clause 5.3.3 |
| 7.3 | Dynamic range | - | - | - |
|  | E-UTRA | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A |
|  | UTRA TDD | N/A | N/A | Clause 5.3.3 |
| 7.4 | Adjacent channel selectivity and narrowband blocking | - | - | - |
|  | General blocking requirement | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | General narrowband blocking requirement | C: ATC3a, ATC4b CNC:ANTC3a, ATC4b C/NC: ATC3a, ANTC3a,ATC4b | C: ATC3a, ATC4b CNC:ANTC3a, ATC4b C/NC: ATC3a, ANTC3a,ATC4b | C: ATC3b, ATC4b |
|  | Additional BC3 blocking minimum requirement | N/A | N/A | C: ATC3b |
| 7.5 | Blocking | - | - | - |
|  | General requirement | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | Co-location requirement | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
| 7.6 | Receiver spurious emissions | - | - | - |
|  | General requirement | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | Additional requirement for BC2 (Category B) | N/A | N/A | N/A |
| 7.7 | Receiver intermodulation | - | - | - |
|  | General intermodulation requirement | C: ATC3a CNC: ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3a CNC ANTC3a C/NC: ATC3a, ANTC3a | C: ATC3b |
|  | General narrowband intermodulation requirement | C: ATC3a, ATC4b CNC:ANTC3a, ATC4b C/NC: ATC3a, ANTC3a, ATC4b | C: ATC3a ATC4b CNC:ANTC3a,ATC4b C/NC: ATC3a, ANTC3a; ATC4b | C: ATC3b, ATC4b |
| 7.8 | In-channel selectivity | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| NOTE: Compliance stated by manufacturer declaration. |  |  |  |  |

## 5.3	Test configurations for multi-carrier capable TAB connector(s) in operating bands where one RAT capability sets are supported

### 5.3.1	General

A TAB connector may support only one RAT operation in an operating band by fulfilling different sets of requirements. Both UTRA and E-UTRA have two complete sets of requirements that may be fulfilled depending on whether the TAB connector is declared to be MSR or single RAT in the operating band. MSR and single RAT requirements are addressed separately by separate test requirements (and corresponding core requirements). They are also identified by different capability sets as described in clauses 4.11 and 5.2.

### 5.3.2	TAB connector supporting one RAT only MSR in the operating band

This clause contains test configuration applicability to requirements and capability sets for TAB connectors supporting one RAT only MSR operation operating with multiple carriers (MC).

Table 5.3.2-1: Test configuration applicability to requirements and
capability sets for TAB connectors supporting one RAT only MSR operation

| Capability Set |  | UTRA (MC) capable BS (CSA1) |  |  | E-UTRA (MC) capable BS (CSA2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TAB connector test case |  | BC1 | BC2 | BC3 | BC1 | BC2 | BC3 |
| 6.2 | Base Station output power | - | - | - | - | - | - |
| 6.2.2 | Base Station maximum output power | C: ATC1a CNC: ATC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ATC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ATC2a C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ATC2a C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ATC2a C/NC: ATC2a, ANTC2 |
|  | Additional regional requirement (only for band 34) | N/A | N/A | N/A | N/A | N/A | NOTE1 |
| 6.2.3 | UTRA FDD primary CPICH power | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
| 6.2.3A | UTRA FDD secondary CPICH power | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
| 6.2.4 | UTRA TDD primary CCPCH power | N/A | N/A | Clause 5.3.3 | N/A | N/A | N/A |
| 6.2.5 | UTRA FDD additional CPICH power for MIMO mode | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
| 6.2.6 | E-UTRA DL RS power | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.3 | Output power dynamics | - | - | - | - | - | - |
| 6.3.2 | UTRA Inner loop power control in the downlink | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.3.3 | Power control dynamic range | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
| 6.3.4 | Total power dynamic range | N/A | N/A | Clause 5.3.3 | N/A | N/A | N/A |
| 6.3.5 | IPDL time mask | Clause 5.3.3 | Clause 5.3.3 | N/A |  |  |  |
| 6.3.6 | RE Power control dynamic range | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.4 | Transmit ON/OFF power | - | - | - | - | - | - |
| 6.4.1 | Transmitter OFF power | N/A | N/A | C: ATC1b | N/A | N/A | C: ATC2a CNC: ATC2a C/NC: ATC2a, ANTC2 |
| 6.4.2 | Transmitter transient period | N/A | N/A | C: ATC1b | N/A | N/A | C: ATC2a CNC: ATC2a C/NC: ATC2a, ANTC2 |
| 6.5 | Transmitted signal quality | - | - | - | - | - | - |
| 6.5.2 | Frequency error | - | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 |
|  | UTRA FDD | Same TC as used in clause 6.5.4 | Same TC as used in clause 6.5.4 | N/A | N/A | N/A | N/A |
|  | UTRA TDD | N/A | N/A | Same TC as used in clause 6.5.1 | N/A | N/A | N/A |
| 6.5.3 | Time alignment error | - | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
|  | UTRA TDD | N/A | N/A | Clause 5.3.3 | N/A | N/A | N/A |
| 6.5.4 | Modulation quality - EVM | - | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | C: ATC2a CNC: ATC2a C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ATC2a C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ATC2aC/NC: ATC2a, ANTC2 |
|  | UTRA FDD | C: ATC1a CNC: ATC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ATC1a C/NC: ATC1a, ANTC1a | N/A | N/A | N/A | N/A |
|  | UTRA TDD | N/A | N/A | C: ATC1b | N/A | N/A | N/A |
| 6.5.5 | Transmit pulse shape filter | Not tested | Not tested | Not tested | Not tested | Not tested | Not tested |
| 6.6 | Unwanted Emissions | - | - | - | - | - | - |
| 6.6.2 | Occupied bandwidth | - | - | - | - | - | - |
|  | Minimum requirement | Clause 5.3.3 | Clause 5.3.3 | Clause 5.3.3 | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| 6.6.3 | Adjacent Channel Leakage power Ratio |  | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
|  | UTRA TDD | N/A | N/A | Clause 5.3.3 | N/A | N/A | N/A |
|  | Cumulative ACLR | CNC: ANTC1a C/NC: ANTC1a | CNC: ANTC1a C/NC: ANTC1a | - | CNC: ANTC2 C/NC: ANTC2 | CNC: ANTC2 C/NC: ANTC2 | CNC: ANTC2 C/NC: ANTC2 |
| 6.6.4 | Spectrum emission mask |  |  |  |  |  |  |
| 6.6.5 | Operating band unwanted emission | - | - | - | - | - | - |
|  | General requirement for Band Categories 1 and 3 | Clause 5.3.3 C: ATC1a CNC: ATC1a, ANTC1a C/NC: ATC1a, ANTC1a | N/A | Clause 5.3.3 C: ATC1b | Clause 5.3.4 C: ATC2a CNC: ATC2a, ANTC2 C/NC: ATC2a, ANTC2 | N/A | Clause 5.3.4 C: ATC2a CNC: ATC2a, ANTC2 C/NC: ATC2a, ANTC2 |
|  | General requirement for Band Category 2 | N/A | Clause 5.3.3 C: ATC1a CNC: ATC1a, ANTC1a C/NC: ATC1a, ANTC1a | N/A | N/A | Clause 5.3.4 C: ATC2a CNC: ATC2a, ANTC2 C/NC: ATC2a, ANTC2 | N/A |
|  | Additional requirements | (note) | (note) | (note) | (note) | (note) | (note) |
| 6.6.6 | Spurious emission | - | - | - | - | - | - |
|  | (Category A) | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | (Category B) | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | Additional requirement for BC2 (Category B) | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Protection of the BS receiver of own or different BS | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | Additional spurious emissions requirements | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | Co-location with other Base Stations | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2C/NC: ATC2a, ANTC2 |
| 6.7 | Transmitter intermodulation | - | - | - | - | - | - |
|  | General requirement | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 | Same TC as used in clause 6.6 |
|  | Additional requirement (BC1 and BC2) | CNC: ANTC1a C/NC:ANTC1a | Same TC as used in 6.6 | N/A | CNC: ANTC2 C/NC: ANTC2 | Same TC as used in 6.6 | N/A |
|  | Additional requirement (BC3) | N/A | N/A | Same TC as used in clause 6.6 | N/A | N/A | Same TC as used in 6.6 |
| 7.2 | Reference sensitivity level | - | - | - | - | - | - |
|  | E-UTRA requirement | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD requirement | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
|  | UTRA TDD requirement | N/A | N/A | Clause 5.3.3 | N/A | N/A | N/A |
| 7.3 | Dynamic range | - | - | - | - | - | - |
|  | E-UTRA | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
|  | UTRA FDD | Clause 5.3.3 | Clause 5.3.3 | N/A | N/A | N/A | N/A |
|  | UTRA TDD | N/A | N/A | Clause 5.3.3 | N/A | N/A | N/A |
| 7.4 | Adjacent channel selectivity and narrowband blocking | - | - | - | - | - | - |
|  | General blocking requirement | C: ATC1a CNC: ANTC1a C/NC: ATC1a , ANTC1a | C: ATC1a, ATC4aCNC: ANTC1a C/NC: ATC1a , ANTC1a | C: ATC1b ATC4c | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | General narrowband blocking requirement | C: ATC1a, ATC4a CNC: ANTC1a, ATC4a C/NC: ATC1a, ANTC1a, ATC4a | C: ATC1a, ATC4a CNC:ANTC1a, ATC4a C/NC: ATC1a, ANTC1a, ATC4a | C: ATC1b ATC4c | C: ATC2a, ATC4b CNC:ANTC2, ATC4b C/NC: ATC2a, ANTC2 | C: ATC2a, ATC4b CNC: ANTC2, ATC4b C/NC: ATC2a, ANTC2, ATC4b | C: ATC2a, ATC4b CNC: ANTC2, ATC4b C/NC: ATC2a, ANTC2, ATC4b |
|  | Additional BC3 blocking minimum requirement | N/A | N/A | C: ATC1b | N/A | N/A | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
| 7.5 | Blocking | - | - | - | - | - | - |
|  | General requirement | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | Co-location requirement | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
| 7.6 | Receiver spurious emissions | - | - | - | - | - | - |
|  | General requirement | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a, ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | Additional requirement for BC2 (Category B) | N/A | N/A | N/A | N/A | N/A | N/A |
| 7.7 | Receiver intermodulation | - | - | - | - | - | - |
|  | General intermodulation requirement | C: ATC1a CNC: ANTC1a C/NC: ATC1a , ANTC1a | C: ATC1a CNC: ANTC1a C/NC: ATC1a , ANTC1a | C: ATC1b | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 | C: ATC2a CNC: ANTC2 C/NC: ATC2a, ANTC2 |
|  | General narrowband intermodulation requirement | C: ATC1a, ATC4a CNC:ANTC1a, ATC4a C/NC: ATC1a, ANTC1a, ATC4a | C: ATC1a, ATC4a CNC:ANTC1a, ATC4a C/NC: ATC1a, ANTC1a, ATC4a | C: ATC1b, ATC4c | C: ATC2a, ATC4b CNC: ANTC2, ATC4b C/NC: ATC2a, ANTC2, ATC4b | C: ATC2a, ATC4b CNC: ANTC2, ATC4b C/NC: ATC2a, ANTC2, ATC4b | C: ATC2a, ATC4b CNC: ANTC2, ATC4b C/NC: ATC2a, ANTC2, ATC4b |
| 7.8 | In-channel selectivity | - | - | - | - | - | - |
|  | E-UTRA requirement | N/A | N/A | N/A | Clause 5.3.4 | Clause 5.3.4 | Clause 5.3.4 |
| NOTE: Compliance stated by manufacturer. |  |  |  |  |  |  |  |

### 5.3.3	TAB connector supporting Single-RAT UTRA in the operating band

This clause contains the test configurations for TAB connectors supporting single-RAT UTRA in the operating band. The test configurations apply to TAB connectors operating with multiple carriers (MC).

For a TAB connector declared to support multi-carrier operation in contiguous spectrum operation in single band only, the test configurations in the second column of table 5.3.3-1 for FDD, and in the fifth column of table 5.3.3-1 for TDD, shall be used for testing.

NOTE:	The applicability of test configurations for TDD in this clause is only applicable to UTRA TDD 1,28 Mcps option.

For FDD a TAB connector declared to support multi-carrier operation in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to clause 4.10 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the third column of table 5.3.3-1 shall be used for testing.

For FDD a TAB connector declared to support multi-carrier operation in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to clause 4.10 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the fourth column of table 5.3.3-1 shall be used for testing.

Table 5.3.3-1: Test configurations for a TAB connector supporting single-RAT UTRA operation

| TAB connector test case |  | Single-RAT UTRA FDD MC capable TAB connector (CSA4) C capable only | Single-RAT UTRA FDD MC capable TAB connector (CSA4) C and NC capable with identical parameters | Single-RAT UTRA FDD MC capable TAB connector (CSA4) C and NC capable with different parameters | Single-RAT UTRA TDD MC capable TAB connector (CSA4) C capable only |
| --- | --- | --- | --- | --- | --- |
| 6.2 | Base Station output power | - | - | - | - |
| 6.2.2 | Base Station maximum output power | ATC1a | ATC1a | ATC1a, ANTC1 | ATC1b |
| 6.2.3 | UTRA FDD primary CPICH power | SC | SC | SC | N/A |
| 6.2.3A | UTRA FDD secondary CPICH power | SC | SC | SC | N/A |
| 6.2.4 | UTRA TDD primary CCPCH power | N/A | N/A | N/A | SC |
| 6.3 | Output power dynamics | - | - | - | - |
| 6.3.2 | UTRA Inner loop power control in the downlink | SC | SC | SC | SC |
| 6.3.3 | Power control dynamic range | SC | SC | SC | SC |
| 6.3.4 | Total power dynamic range | SC or ATC1a | SC or ATC1a | SC or ATC1a | N/A |
| 6.3.5 | IPDL time mask | SC | SC | SC | N/A |
| 6.4 | Transmit ON/OFF power | N/A | N/A | N/A | ATC1b |
| 6.5 | Transmitted signal quality | - | - | - | - |
| 6.5.2 | Frequency error | ATC1a | ATC1a | ATC1a, ANTC1 | ATC1b |
| 6.5.3 | Time alignment error | ATC1a | ATC1a | ATC1a, ANTC1 | ATC1b |
| 6.5.4 | Modulation quality - EVM | ATC1a | ATC1a | ATC1a, ANTC1 | ATC1b |
| 6.5.4 | Modulation quality - PCDE | ATC1a | ATC1a | ATC1a, ANTC1 | ATC1b |
| 6.5.4 | Modulation quality - RCDE | ATC1a | ATC1a | ATC1a | ATC1b |
| 6.5.5 | Transmit pulse shape filter | Not tested | Not tested | Not tested | Not tested |
| 6.6 | Unwanted Emissions | - | - | - | - |
| 6.6.2 | Occupied bandwidth | SC | SC | SC | SC |
| 6.6.3 | Adjacent Channel Leakage power Ratio | ATC1a | ANTC1 | ATC1a, ANTC1 | ATC1b |
|  | Cumulative ACLR | - | ANTC1 | ANTC1 | N/A |
| 6.6.4 | Spectrum emission mask | ATC1a | ATC1a, ANTC1 | ATC1a, ANTC1 | ATC1b |
| 6.6.6 | Spurious emission | ATC1a | ANTC1 | ATC1a, ANTC1 | ATC1b |
| 6.7 | Transmitter intermodulation | ATC1a | ATC1a, ANTC1 | ATC1a, ANTC1 | ATC1b |
| 7.2 | Reference sensitivity level | SC | SC | SC | SC |
| 7.3 | Dynamic range | SC | SC | SC | SC |
| 7.4 | Adjacent channel selectivity and narrowband blocking | ATC1a | ANTC1 | ATC1a, ANTC1 | ATC1b |
| 7.5 | Blocking | ATC1a | ANTC1 | ATC1a, ANTC1 | ATC1b |
| 7.6 | Receiver spurious emissions | ATC1a | ANTC1 | ATC1a, ANTC1 | ATC1b |
| 7.7 | Receiver intermodulation | ATC1a | ANTC1 | ATC1a, ANTC1 | ATC1b |

### 5.3.4	TAB connector supporting Single-RAT E-UTRA in the operating band

This clause contains the test configurations for TAB connectors supporting single-RAT E-UTRA in the operating band. The test configurations apply to TAB connectors operating with multiple carriers (MC).

For a TAB connector declared to support multi-carrier and/or CA operation in contiguous spectrum operation in single band only, the test configurations in the second column of table 5.3.4-1 shall be used for testing.

For a TAB connector declared to support multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to clause 4.10 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the third column of table 5.3.4-1 shall be used for testing.

For a TAB connector declared to support multi-carrier and/or CA in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to clause 4.10 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the fourth column of table 5.3.4-1 shall be used for testing.

Table 5.3.4-1: Test configurations for a TAB connector supporting single-RAT E-UTRA operation capable of both contiguous and non-contiguous spectrum in multi-carrier and/or CA operation in single band

| TAB connector test case |  | Single-RAT E-UTRA MC capable TAB connector (CSA5) C capable only | Single-RAT E-UTRA MC capable TAB connector (CSA5) C and NC capable BS with identical parameters (CNC) | Single-RAT E-UTRA MC capable TAB connector (CSA5) C and NC capable BS with different parameters (CNC) |
| --- | --- | --- | --- | --- |
| 6.2 | Base Station output power | - | - | - |
| 6.2.2 | Base Station maximum output power | ATC2a | ATC2a | ATC2a, ANTC2 |
| 6.2.6 | E-UTRA DL RS power | SC | SC | SC |
| 6.3 | Output power dynamics | - | - | - |
| 6.3.4 | Total power dynamic range | SC | SC | SC |
| 6.3.6 | RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.4 | Transmit ON/OFF power | ATC2a | ATC2a | ATC2a, ANTC2 |
| 6.5 | Transmitted signal quality | - | - | - |
| 6.5.2 | Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.5.3 | Time alignment error | ATC2a | ATC2a | ATC2a, ANTC2 |
| 6.5.4 | Modulation quality - EVM | ATC2a | ATC2a | ATC2a, ANTC2 |
| 6.5.5 | Transmit pulse shape filter | Not Tested | Not Tested | Not Tested |
| 6.6 | Unwanted Emissions | - | - | - |
| 6.6.2 | Occupied bandwidth | SC, ATC2b (Note) | SC, ATC2b (Note) | SC, ATC2b (Note) |
| 6.6.3 | Adjacent Channel Leakage power Ratio | ATC2a | ANTC2 | ATC2a, ANTC2 |
| 6.6.3 | Cumulative ACLR | - | ANTC2 | ANTC2 |
| 6.6.5 | Operating band unwanted emission | ATC2a | ATC2a, ANTC2 | ATC2a, ANTC2 |
| 6.6.6 | Spurious emission | ATC2a | ANTC2 | ATC2a, ANTC2 |
| 6.7 | Transmitter intermodulation | ATC2a | Same TC as used in 6.6 | Same TC as used in 6.6 |
| 7.2 | Reference sensitivity level | SC | SC | SC |
| 7.3 | Dynamic range | SC | SC | SC |
| 7.4 | Adjacent channel selectivity and narrowband blocking | ATC2a | ANTC2 | ATC2a, ANTC2 |
| 7.5 | Blocking | ATC2a | ANTC2 | ATC2a, ANTC2 |
| 7.6 | Receiver spurious emissions | ATC2a | ANTC2 | ATC2a, ANTC2 |
| 7.7 | Receiver intermodulation | ATC2a | ANTC2 | ATC2a, ANTC2 |
| 7.8 | In-channel selectivity | SC | SC | SC |
| NOTE: ATC2b is only applicable when contiguous CA is supported. |  |  |  |  |

## 5.4	Test configurations for Multi-band TAB connectors

### 5.4.1	Multi-band TAB connector supporting MSR operation

Table 5.4.1-1: Test configuration for multi-band TAB connectors supporting MSR operation

| TAB connector test case |  | Test for Multi-Band TAB connector | Test configuration for MBT |  |
| --- | --- | --- | --- | --- |
|  |  | CSA1, CSA2, CSA3 | BC1/BC2 | BC3 |
| 6.2 | Base Station output power | - | - | - |
| 6.2.2 | Base Station maximum output power | SBT, MBT | ATC5a | ATC5a |
|  | Additional regional requirement (only for band 34) | Compliance by declaration | N/A | - |
| 6.2.3 | UTRA FDD primary CPICH power | SBT | - | - |
| 6.2.3A | UTRA FDD secondary CPICH power |  |  |  |
| 6.2.4 | UTRA TDD primary CCPCH power | SBT | - | - |
| 6.2.5 | UTRA FDD additional CPICH power for MIMO mode |  |  |  |
| 6.2.6 | E-UTRA DL RS power | E-UTRA for DL RS power | SBT | - |
| 6.3 | Output power dynamics | - | - | - |
| 6.3.2 | UTRA Inner loop power control in the downlink | SBT | - | - |
| 6.3.3 | Power control dynamic range | SBT | - | - |
| 6.3.4 | Total power dynamic range | SBT | - | N/A |
| 6.3.5 | IPDL time mask | SBT | - | N/A |
| 6.3.6 | RE Power control dynamic range | SBT | - | N/A |
| 6.4 | Transmit ON/OFF power | - | - | - |
| 6.4.1 | Transmitter OFF power | MBT, SBT (note 3) | N/A | ATC5a |
| 6.4.2 | Transmitter transient period | MBT, SBT (note 3) | N/A | ATC5a |
| 6.5 | Transmitted signal quality | - | - | - |
| 6.5.2 | Frequency error | - | - | - |
|  | E-UTRA | SBT, MBT | ATC5a | ATC5a |
|  | UTRA FDD | SBT, MBT | ATC5a | N/A |
|  | UTRA TDD | SBT, MBT | N/A | ATC5a |
| 6.5.3 | Time alignment error | - | - | - |
|  | E-UTRA | SBT, MBT (note 1) | ATC5b | ATC5b |
|  | UTRA FDD | SBT, MBT (note 1) | ATC5b | N/A |
|  | UTRA TDD | SBT | N/A | - |
| 6.5.4 | Modulation quality - EVM | - | - | - |
|  | E-UTRA | SBT, MBT | ATC5a | ATC5a |
|  | UTRA FDD | SBT, MBT | ATC5a | N/A |
|  | UTRA TDD | SBT, MBT | N/A | ATC5a |
| 6.5.5 | Transmit pulse shape filter | Not tested | - | - |
| 6.6 | Unwanted Emissions | - | - | - |
| 6.6.2 | Occupied bandwidth | - | - | - |
|  | Minimum requirement | SBT | - | - |
| 6.6.3 | Adjacent Channel Leakage power Ratio | - | - | - |
|  | E-UTRA | SBT, MBT (note 2) | ATC5b | ATC5b |
|  | UTRA FDD | SBT, MBT (note 2) | ATC5b | N/A |
|  | UTRA TDD | SBT, MBT (note 2) | N/A | ATC5b |
|  | Cumulative ACLR | SBT, MBT (note 2) | ATC5b | ATC5b |
| 6.6.4 | Spectrum emission mask | - | - | - |
| 6.6.5 | Operating band unwanted emission | - | - | - |
|  | General requirement for Band Categories 1 and 3 | SBT, MBT | ATC5b | ATC5b |
|  | General requirement for Band Category 2 | SBT, MBT | ATC5b | N/A |
|  | Additional requirements | SBT, MBT | - | - |
| 6.6.6 | Spurious emission | - | - | - |
|  | (Category A) | SBT, MBT | ATC5b | ATC5b |
|  | (Category B) | SBT, MBT | ATC5b | ATC5b |
|  | Additional requirement for BC2 (Category B) | SBT, MBT | ATC5b | ATC5b |
|  | Protection of the BS receiver of own or different BS | SBT, MBT | ATC5b | ATC5b |
|  | Additional spurious emissions requirements | SBT, MBT | ATC5b | ATC5b |
|  | Co-location with other Base Stations | - | - | - |
| 6.7 | Transmitter intermodulation | - | - | - |
|  | General requirement | SBT | - | - |
|  | Additional requirement (BC1 and BC2) | SBT | - | N/A |
|  | Additional requirement (BC3) | SBT | N/A | - |
| 7.2 | Reference sensitivity level | - | - | - |
|  | E-UTRA requirement | SBT | - | - |
|  | UTRA FDD requirement | SBT | - | - |
|  | UTRA TDD requirement | SBT | - | - |
| 7.3 | Dynamic range | - | - | - |
|  | E-UTRA | SBT | - | - |
|  | UTRA FDD | SBT | - | - |
|  | UTRA TDD | SBT | - | - |
| 7.4 | Adjacent channel selectivity and narrowband blocking | - | - | - |
|  | General blocking requirement | MBT, SBT (note 3) | ATC5b | ATC5b |
|  | General narrowband blocking requirement | MBT, SBT (note 3) | ATC5b | ATC5b |
|  | Additional BC3 blocking minimum requirement | MBT, SBT (note 3) | N/A | ATC5b |
| 7.5 | Blocking | - | - | - |
|  | General requirement | MBT, SBT(note 3) | ATC5b | ATC5b |
|  | Co-location requirement | MBT, SBT(note 3) | ATC5b | ATC5b |
| 7.6 | Receiver spurious emissions | - | - | - |
|  | General requirement | SBT, MBT | ATC5b | ATC5b |
|  | Additional requirement for BC2 (Category B) | - | - | - |
| 7.7 | Receiver intermodulation | - | - | - |
|  | General intermodulation requirement | MBT, SBT(note 3) | ATC5b | ATC5b |
|  | General narrowband intermodulation requirement | MBT, SBT(note 3) | ATC5b | ATC5b |
| 7.8 | In-channel selectivity | - | - | - |
|  | E-UTRA requirement | SBT | - | - |
| NOTE 1: MBT is only applicable when DB-DC-HSDPA/inter-band CA is supported. NOTE 2: For ACLR, MBT shall be applied for the Inter RF bandwidth gap only. NOTE 3: SBT is only applicable if different Capability Sets are declared for single-band and multi-band operation. |  |  |  |  |

### 5.4.2	Multi-band TAB connector supporting Single-RAT only

For a multi-band TAB connector supporting single-RAT only in the operational band, the test configurations in table 5.4.2-1, shall be used for testing.

Table 5.4.2-1: Test configuration for multi-band TAB connectors supporting Single-RAT only

| TAB connector test case |  | UTRA FDD CSA4 | UTRA TDD CSA4 | E-UTRA Test CSA5 |
| --- | --- | --- | --- | --- |
| 6.2 | Base Station output power | - | - | - |
| 6.2.2 | Base Station maximum output power | ATC1a/ANTC1 (note 1) ATC5a | ATC1b (note 3), ATC5a | ATC2a/ANTC2 (note 5), ATC5a |
| 6.2.3 | UTRA FDD primary CPICH power | SC | N/A | N/A |
| 6.2.3A | UTRA FDD secondary CPICH power | SC | N/A | N/A |
| 6.2.4 | UTRA TDD primary CCPCH power | N/A | SC | N/A |
| 6.2.5 | UTRA FDD additional CPICH power for MIMO mode | SC | N/A | N/A |
| 6.2.6 | E-UTRA DL RS power | N/A | N/A | SC |
| 6.3 | Output power dynamics | - | - | - |
| 6.3.2 | UTRA Inner loop power control in the downlink | SC | SC | N/A |
| 6.3.3 | Power control dynamic range | SC | SC | SC |
| 6.3.4 | Total power dynamic range | SC or ATC1a | N/A | SC |
| 6.3.5 | IPDL time mask | SC | N/A | N/A |
| 6.3.6 | RE Power control dynamic range | N/A | N/A | Tested with Error Vector Magnitude |
| 6.4 | Transmit ON/OFF power | - | - | - |
| 6.4.1 | Transmitter OFF power | N/A | ATC5a | ATC5a (only applied for E-UTRA TDD BS) |
| 6.4.2 | Transmitter transient period | N/A | SC | SC |
| 6.5 | Transmitted signal quality | - | - | - |
| 6.5.2 | Frequency error | Tested with EVM | ATC1b (note 3), ATC5a | Tested with Error Vector Magnitude |
| 6.5.3 | Time alignment error | ATC1a/ANTC1 (note 1) ATC5b | ATC1b (note 3) | ATC2a/ANTC2 (note 5), ATC5b (note 6) |
| 6.5.4 | Modulation quality - EVM | ATC1a/ANTC1 (note 1), ATC5a | ATC1b (note 3), ATC5a | ATC2a/ANTC2 (note 5), ATC5a |
| 6.5.4 | Modulation quality - PCDE | ATC1a/ANTC1 (note 1) | ATC1b (note 3), ATC5a | N/A |
| 6.5.4 | Modulation quality - RCDE | AUTC1 | ATC1b (note 3), ATC5a | N/A |
| 6.5.5 | Transmit pulse shape filter | not tested | not tested | not tested |
| 6.6 | Unwanted Emissions | - | - | - |
| 6.6.2 | Occupied bandwidth | SC | SC | SC, ATC2b (note 7) |
| 6.6.3 | Adjacent Channel Leakage power Ratio | ATC1a/ANTC1 (note 1) ATC5b (note 2) | ATC1b (note 3), ATC5a (note 4) | ATC2a/ANTC2 (note 5), ATC5b (note 8) |
| 6.6.4 | Spectrum emission mask | ATC1a/ANTC1 (note 1) ATC5b | ATC1b (note 3), ATC5a | N/A |
| 6.6.5 | Operating band unwanted emission | N/A | N/A | ATC2a/ANTC2 (note 5), ATC5b |
| 6.6.6 | Spurious emission | ATC1a/ANTC1 (note 1) ATC5b | ATC1b (note 3), ATC5a | ATC2a/ANTC2 (note 5), ATC5b |
| 6.7 | Transmitter intermodulation | ATC1a/ANTC1 (note 1) | ATC1b (note 3) | ATC2a/ANTC2 (note 5) |
| 7.2 | Reference sensitivity level | SC | SC | SC |
| 7.3 | Dynamic range | SC | SC | SC |
| 7.4 | Adjacent channel selectivity and narrowband blocking | ATC5b | ATC5a | ATC5b |
| 7.5 | Blocking | ATC5b | ATC5a | ATC5b |
| 7.6 | Receiver spurious emissions | ATC1a/ANTC1 (note 1) ATC5b | ATC1b (note 3), ATC5a | ATC2a/ANTC2 (note 5), ATC5b |
| 7.7 | Receiver intermodulation | ATC5b | ATC5a | ATC5b |
| 7.8 | In-channel selectivity | N/A | N/A | SC |
| NOTE 1: ATC1a and/or ANTC1 shall be applied in each supported operating band according to table 5.3.3-1. NOTE 2: ATC5b may be applied for Inter RF Bandwidth gap only. NOTE 3: ATC1b shall be applied in each supported operating band according to table 5.3.3-1. NOTE 4: ATC5a may be applied for Inter RF bandwidth gap only. NOTE 5: ATC2 and/or ANTC2 shall be applied in each supported operating band according to table 5.3.4-1. NOTE 6: ATC5b is only applicable when inter-band CA is supported. NOTE 7: ATC2b is only applicable when contiguous CA is supported. NOTE 8: ATC5b may be applied for Inter RF bandwidth gap only. |  |  |  |  |

# 6	Conducted transmitter characteristics

## 6.1	General

General test conditions for transmitter tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in clause 4.8.

If a number of TAB connectors have been declared equivalent (see table 4.10-1, D6.70), only a representative one is necessary to be tested to demonstrate conformance.

In clause 6.6.3.1, if representative TAB connectors are used then per connector criteria (i.e. option 2) shall be applied.

The manufacturer shall declare the minimum number of supported geographical cells (i.e. geographical areas). The minimum number of supported geographical cells (Ncells) relates to the AAS BS setting with the minimum amount of cell splitting supported with transmission on all TAB connectors supporting the operating band. The manufacturer shall also declare TAB connector TX min cell groups. Every TAB connector supporting transmission in an operating band shall map to one TAB connector TX min cell group supporting the same operating band. The mapping of TAB connectors to cells is implementation dependent.

The number of active transmitter units that are considered when calculating the emissions limit (NTXU, counted) for an AAS BS is calculated as follows:

NTXU, counted = min(NTXU,active , 8·Ncells) for E-UTRA single RAT AAS BS and MSR AAS BS (except UTRA only MSR AAS BS); and

NTXU, counted = min(NTXU,active, 4·Ncells) for UTRA single RAT AAS BS and UTRA only MSR AAS BS

Further:

NTXU,countedpercell = NTXU,counted/Ncells

NTXU,countedpercell is used for scaling the basic limits as described in clause 6.6.

NOTE:	NTXU,active depends on the actual number of active transmitter units and is independent to the declaration of Ncells.

Any transmitter test requirement specified for Band 46 operation in TS 36.104 [4] for E-UTRA, or in TS 37.104 [5] for E-UTRA in MSR operation, and referred in clause 6, is not applicable for AAS BS.

Any transmitter test requirement specified for NB-IoT in-band, NB-IoT guard band, or standalone NB-IoT operation in TS 36.141 [14] for E-UTRA with NB-IoT (in-band or guard band) or for standalone NB-IoT, or in TS 37.141 [13] for E-UTRA with NB-IoT or standalone NB-IoT in MSR operation, and referred in clause 6, is not applicable for AAS BS.

## 6.2	Base station output power

### 6.2.1	General

The configured carrier power is the target maximum power for a specific carrier for the operating mode set in the BS within the limits given by the manufacturer's declaration.

### 6.2.2	Maximum output power

#### 6.2.2.1	Definition and applicability

The rated carrier output power of the AAS BS shall be as specified in table 6.2.2.1-1.

Table 6.2.2.1-1: AAS Base Station rated output power limits for BS classes

| AAS BS class | PRated,c,sys | PRated,c,TABC |
| --- | --- | --- |
| Wide Area BS | (note) | (note) |
| Medium Range BS | ≤ 38 dBm +10log(NTXU,counted) | ≤ 38 dBm |
| Local Area BS | ≤ 24 dBm +10log(NTXU,counted) | ≤ 24 dBm |
| NOTE: There is no upper limit for the PRated,c,sys or PRated,c,TABC of the Wide Area Base Station. |  |  |

The output power limit for the respective BS classes in table 6.2.2.1-1 shall be compared to the rated output power and the declared BS class. It is not subject to testing.

The requirement in clause 6.2.2.2 applies per TAB connector.

#### 6.2.2.2	Minimum Requirement

The minimum requirement is in TS 37.105 [8], clause 6.2.2.2.

#### 6.2.2.3	Test Purpose

The test purpose is to verify the accuracy of the maximum carrier output power per TAB connector (Pmax,c,TABC) across the frequency range and under normal and extreme conditions for all TAB connectors  in the AAS BS.

#### 6.2.2.4	Method of test

##### 6.2.2.4.1	Initial conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested:

-	B, M and T; see clause 4.12.1

Base Station RF Bandwidth positions to be tested:

-	BRFBW, MRFBW and TRFBW for single band TAB connector(s) , see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band TAB connector(s), see clause 4.12.1.

In addition, a single test case shall be performed under extreme test environment as defined in annex clause B.3 In this case, it is sufficient to test on a single combination of one ARFCN, UARFCN or EARFCN, one RF bandwidth position and with only one applicable test configuration defined in clause 5.

NOTE:	Tests under extreme power supply also test extreme temperature.

##### 6.2.2.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the power measuring equipment to the TAB connector(s) as shown in annex clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set each TAB connector to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.12.2. For single carrier set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

3)	Measure the mean power for each carrier at each TAB connector.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.2.2.5	Test Requirements

In normal conditions, the measurement result in step 3 of clause 6.2.2.4.3 shall be:

-	within +2.7 dB and -2.7 dB of the manufacturer's rated carrier output power per TAB connector (PRated,c,TABC) for carrier frequency f ≤ 3.0 GHz.

-	within +3.0 dB and -3.0 dB of the manufacturer's rated carrier output power per TAB connector (PRated,c,TABC) for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

In extreme conditions, measurement result in step 3 of clause 6.2.2.4.3 shall be:

-	within +3.2 dB and -3.2 dB of the manufacturer's rated carrier output power per TAB connector (PRated,c,TABC) for carrier frequency f ≤ 3.0 GHz.

-	within +3.5 dB and -3.5 dB of the manufacturer's rated carrier output power per TAB connector (PRated,c,TABC) for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

### 6.2.3	UTRA FDD primary CPICH power

#### 6.2.3.1	Definition and applicability

This requirement applies to the TAB connector group(s) transmitting primary CPICH.

Primary CPICH (P-CPICH) power is the code domain power of the Primary Common Pilot Channel summed over the TAB connectors transmitting the P-CPICH for a cell. P-CPICH power is indicated on the BCH.

NOTE 1:	A TAB connector group may comprise all TAB connectors.

NOTE 2:	A TAB connector may be mapped to several groups.

NOTE 3:	The manufacturer declares the TAB connector mapping to the P-CPICH transmission group(s) as specified in clause 4.10.

#### 6.2.3.2	Minimum requirement

The minimum requirement for UTRA FDD operation is in TS 37.105 [8], clause 6.2.3.

There is no P-CPICH power requirement for UTRA TDD 1,28 Mcps option operation.

There is no P-CPICH power requirement for E-UTRA operation.

#### 6.2.3.3	Test purpose

The test purpose is to verify that the UTRA FDD primary CPICH power is within the limits specified by the minimum requirement.

#### 6.2.3.4	Method of test

##### 6.2.3.4.1	Initial conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

Disable inner loop power control.

Set the TAB connectors in the P-CPICH transmission group (see table 4.10-1, D6.45) to output a signal in accordance to TM2, in TS 25.141 [18], clause 6.1.1.2.

##### 6.2.3.4.2	Procedure

The minimum requirement is applied to all TAB connectors in the TAB connector group(s) transmitting primary CPICH. TAB connectors may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to code domain analyser as shown in annex clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC). Measure the P-CPICH power in one timeslot on each of the TAB connector(s) transmitting the P-CPICH according to annex E in TS 25.141 [18].

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.2.3.5	Test requirements

For UTRA FDD the test requirement for CPICH power is:

Either:

The sum of the measured P-CPICH code domain power on each of the TAB connectors transmitting the P-CPICH shall be:

Within 2.9 dB of the configured absolute value for carrier frequency f ≤ 3.0 GHz.

Within 3.2 dB of the configured absolute value for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

Or

The measured P-CPICH code domain power on each of the TAB connectors transmitting the P-CPICH shall be:

Within 2.9 dB of the configured absolute value for carrier frequency f ≤ 3.0 GHz.

Within 3.2 dB of the configured absolute value for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.2.4	UTRA TDD primary CCPCH power

#### 6.2.4.1	Definition and applicability

This requirement applies to the TAB connector group(s) transmitting primary CCPCH. It comprises primary CCPCH (PCCPCH) absolute power accuracy, and differential accuracy.

Primary CCPCH power is the code domain power of the primary common control physical channel averaged over the transmit timeslot and summed over the TAB connectors transmitting the PCCPCH for a cell. Primary CCPCH power is signalled over the BCH.

The differential accuracy of the Primary CCPCH power is the relative transmitted power accuracy of PCCPCH in consecutive frames when the nominal PCCPCH power is not changed.

NOTE 1:	A TAB connector group may comprise all TAB connectors.

NOTE 2:	A TAB connector may be mapped to several groups.

NOTE 3:	The manufacturer declares the TAB connector mapping to the PCCPCH transmission group(s). See clause 4.10.

#### 6.2.4.2	Minimum requirement

The minimum requirement for UTRA TDD 1,28 Mcps option operation is in TS 37.105 [8], clause 6.2.4.

There is no PCCPCH power requirement for UTRA FDD operation.

There is no PCCPCH power requirement for E-UTRA operation.

#### 6.2.4.3	Test purpose

The test purpose is to verify that the UTRA TDD primary CCPCH power is within the limits specified by the minimum requirement.

#### 6.2.4.4	Method of test

##### 6.2.4.4.1	Initial conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1

Set the TAB connectors in the PCCPCH transmission group (see table 4.10-1, D6.47) to output a signal in accordance to table 6.2.4.4.1-1.

Table 6.2.4.4.1-1: Parameters of the BS transmitted signal
for Primary CCPCH power testing for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, ..., 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots carrying PCCPCH | TS 0 |
| Relative power of PCCPCH | 1/2 of BS output power |
| Data content of DPCH | real life (sufficient irregular) |

##### 6.2.4.4.2	Procedure

The minimum requirement is applied to all TAB connectors in the TAB connector group(s) transmitting primary CCPCH. TAB connectors may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

The requirement has both an absolute accuracy requirement and a differential accuracy requirement which are measured at the same time.

1)	Connect TAB connector to code domain analyser as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

3)	Measure the CCPCH power in one timeslot on each of the TAB connector(s) transmitting the PCCPCH according to annex E in TS 25.142 [20].

4)	Measure the PCCPCH code domain power in TS 0 of consecutive frames by applying the global in-channel Tx test method described in annex E in TS 25.142 [20].

5)	Calculate the differential accuracy of the Primary CCPCH power by taking the difference between the PCCPCH power measurement results of consecutive frames.

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.2.4.5	Test requirements

For UTRA TDD 1,28 Mcps option the test requirement for PCCPCH power is:

Either:

The sum of the measured PCCPCH code domain power on each of the TAB connectors transmitting the PCCPCH shall be within the limits defined in table 6.2.4.5-1.

Or

The measured PCCPCH code domain power on each of the TAB connectors transmitting the PCCPCH shall be within the tolerance indicated in table 6.2.4.5-1.

Table 6.2.4.5-1: Test Requirements for errors between
Primary CCPCH power and the broadcast value

| Output power in slot, dB | PCCPCH power tolerance |
| --- | --- |
| PRated,t,group - 3 < Pout PRated,t,group +2 | ±3.3 dB |
| PRated,t,group - 6 < Pout PRated,t,group -3 | ±4.3 dB |
| PRated,t,group - 13 < Pout PRated,t,group -6 | ±5.8 dB |
| NOTE: PRated,t,group is the power sum of PRated,t,TABC of all the TAB connectors in the group transmitting PCCPCH. |  |

The differential accuracy of the Primary CCPCH power, shall be within 0.6 dB.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.2.5	UTRA FDD additional CPICH power for MIMO mode

#### 6.2.5.1	Definition and applicability

This clause includes requirements on secondary CPICH power level, for two and four TAB connector groups, as well as Demodulation CPICH requirements. The requirements apply to all TAB connector groups associated with UTRA FDD MIMO transmission as "antenna 2", "antenna 3" or "antenna 4" in the AAS BS.

The concept of "antenna 2", "antenna 3" and "antenna 4" is described in TS 25.104 [2]. The group(s) of TAB connectors mapped to P-CPICH transmission (see table 4.10-1 D6.45) represents "antenna 1".

NOTE 1:	The manufacturer declares the mapping of TAB connectors to "antenna 1", "antenna 2", "antenna 3" and "antenna 4" (as defined in TS 25.331 [16]) where applicable for AAS BS capable of UTRA FDD operation. The required declarations are specified clause 4.10.

For UTRA FDD AAS BS operating only "antenna 1" and "antenna 2", the secondary CPICH (S-CPICH) power is the code domain power of the Secondary Common Pilot Channel. S-CPICH power is equal to the (dB) sum of the P-CPICH power and the power offset, which are signalled to the UE. The power offset is signalled in the IE "Power Offset for S-CPICH for MIMO", for MIMO mode as defined in clause 10.3.6.41b in TS 25.331 [16].

When the UE supports MIMO mode with four BS transmit antennas, the power offset of S-CPICH on antenna 2 is signalled in the IE "Power Offset for S-CPICH for MIMO mode with four transmit antennas on Antenna2" as defined in clause 10.3.6.143 in TS 25.331 [16]. The power offset of S-CPICH on antenna 3 and 4 is signalled in the IE "Common Power Offset for S-CPICH for MIMO mode with four transmit antennas on antenna 3 and 4", as defined in clause 10.3.6.143 in TS 25.331 [16].

Demodulation CPICH (D-CPICH) power is the code domain power of the Demodulation Common Pilot Channel. D-CPICH power is equal to the (dB) sum of the P-CPICH power and the power offset, which are signalled to the UE. The power offset of D-CPICH on antenna 3 and 4 is signalled in the IE "Common Power Offset for D-CPICH for MIMO mode with four transmit antennas on antenna 3 and 4", as defined in clause 10.3.6.143 in TS 25.331 [16].

NOTE 2:	The accuracy level of the power offset for S-CPICH may affect both MIMO HS-DSCH demodulation and CQI reporting performance.

NOTE 3:	The accuracy level of the power offset for D-CPICH transmitted on antennas 3 and 4 may affect both MIMO HS-DSCH demodulation and CQI reporting performance.

NOTE 4:	At high geometry level PDSCH performance may be affected if D-CPICH is not scheduled.

NOTE 5:	A TAB connector group may comprise all TAB connectors.

NOTE 6:	A TAB connector may be mapped to several groups.

#### 6.2.5.2	Minimum requirement

The minimum requirement for UTRA FDD operation is in TS 37.105 [8], clause 6.2.5.3.

There is no CPICH power requirement for UTRA TDD 1,28 Mcps option operation.

There is no CPICH power requirement for E-UTRA operation.

#### 6.2.5.3	Test purpose

The test purpose is to verify that the UTRA FDD CPICH power for MIMO mode is within the limits specified by the minimum requirement.

#### 6.2.5.4	Method of test

##### 6.2.5.4.1	Initial conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

Disable inner loop power control.

Set the TAB connectors in the P-CPICH transmission group (see table 4.10-1, D6.45) and the TAB connectors in the S-CPICH transmission groups (see table 4.10-1, D6.49), to output a signal in accordance to TM2, in TS 25.141 [18], clause 6.1.1.2.

NOTE:	Although the S-CPICH transmission groups are referred to as S-CPICH as the same TM2 is used for the test, the signal being measured on the S-CPICH transmission groups is actually a P-CPICH signal.

##### 6.2.5.4.2	Procedure

The minimum requirement is applied to all TAB connectors in the P-CPICH transmission group transmitting primary CPICH and the TAB connectors in the S-CPICH transmission groups. TAB connectors may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to code domain analyser as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

3)	Measure the P-CPICH power in one timeslot on each of the TAB connector(s) in the P-CPICH transmission groups according to annex E in TS 25.141 [18].

4)	Measure the P-CPICH power in the same timeslot as step 2 on each of the TAB connector(s) in each of the S-CPICH transmission groups according to annex E in TS 25.141 [18]. Depending on the MIMO support (see table 4.10-1, D6.49), for MIMO mode there is a single S-CPICH transmission group representing "antenna 2" and for MIMO with 4 transmit antennas there are three S-CPICH transmission groups representing "antenna 2", "antenna 3" and "antenna 4".

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.2.5.5	Test requirements

For UTRA FDD the test requirement for CPICH power for MIMO mode is:

Either:

The difference between the sums of the measured P-CPICH code domain power on each of the TAB connectors in the P-CPICH transmission group or the S-CPICH transmission groups shall be:

Within 2.7 dB for carrier frequency f ≤ 3.0 GHz.

Within 3.0 dB for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

Or

The difference between the measured P-CPICH code domain power on any 2 TAB connectors in either the P-CPICH transmission group or the S-CPICH transmission group(s) with corresponding beamforming weights shall be:

Within 2.7 dB for carrier frequency f ≤ 3.0 GHz.

Within 3.0 dB for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.2.6	E-UTRA DL RS power

#### 6.2.6.1	Definition and applicability

This requirement applies to the TAB connector group(s) transmitting primary DL RS.

The DL RS power is the resource element power of the Downlink Reference Symbol summed over the group of TAB connectors transmitting the DL RS for a cell.

The absolute DL RS power is indicated on the DL-SCH. The absolute accuracy is defined as the maximum deviation between the DL RS power indicated on the DL-SCH and the DL RS power of each E-UTRA carrier.

NOTE 1:	A TAB connector group may comprise all TAB connectors.

NOTE 2:	A TAB connector may be mapped to several groups.

NOTE 3:	The manufacturer declares the TAB connector mapping to the DL RS transmission group(s).

#### 6.2.6.2	Minimum requirement

The minimum requirement for E-UTRA operation is in TS 37.105 [8], clause 6.2.6.

There is no DL RS power requirement for UTRA FDD operation.

There is no DL RS power requirement for UTRA TDD 1,28 Mcps option operation.

#### 6.2.6.3	Test purpose

The test purpose is to verify that the E-UTRA FDD DL RS power is within the limits specified by the minimum requirement.

#### 6.2.6.4	Method of test

##### 6.2.6.4.1	Initial conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

Set the TAB connectors in the DL RS transmission group (see table 4.10-1, D6.54) to output a signal in accordance to E-TM 1.1, in TS 36.141 [17] clause 6.1.1.1.

##### 6.2.6.4.2	Procedure

The minimum requirement is applied to all TAB connectors in the TAB connector group(s) transmitting primary CPICH. TAB connectors may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to code domain analyser as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

3)	Measure the DL RS power on each of the TAB connector(s) transmitting the DL RS according to annex F in TS 36.141 [17].

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.2.6.5	Test requirements

The DL RS power of each E-UTRA carrier shall be:

within  2.9 dB of the DL RS power indicated on the DL-SCH for carrier frequency f ≤ 3.0 GHz.

within  3.2 dB of the DL RS power indicated on the DL-SCH for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

Alternatively, the DL RS power measured at each TAB connector shall be within ± 2.9 dB for f ≤ 3.0 GHz and within  3.2 dB for 3.0 GHz < f ≤ 4.2 GHz of the DL RS power level indicated on the DL-SCH multiplied by a TAB connector specific beamforming weight. Beamforming weights on P-CPICH are set by the AAS BS to achieve an intended radiated pattern.

## 6.3	Output power dynamics

### 6.3.1	General

The requirements in clause 6.3 apply during the transmitter ON period. Transmit signal quality (as specified in clause 6.5) shall be maintained for the output power dynamics requirements.

### 6.3.2	UTRA Inner loop power control in the downlink

#### 6.3.2.1	Definition and applicability

Inner loop power control in the downlink is the ability of the AAS BS to adjust the transmitted output power of a code channel in accordance with the corresponding TPC commands received in the uplink.

This requirement applies at each TAB connector supporting transmission in the operating band.

#### 6.3.2.2	Minimum requirement

The minimum requirement for UTRA FDD operation is in TS 37.105 [8], clause 6.3.2.

The minimum requirement for UTRA TDD 1,28 Mcps option operation is in TS 25.105 [10], clause 6.4.2.1.

There is no Inner loop power control requirement for E-UTRA operation.

#### 6.3.2.3	Test purpose

The test purpose is to verify that the Inner loop power control in the downlink is within the limits specified by the minimum requirement.

#### 6.3.2.4	Method of test

##### 6.3.2.4.1	Initial conditions

# 6.3.2.4.1.1	General test conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

Disable closed loop power control.

# 6.3.2.4.1.2	UTRA FDD

Set each TAB connector to output a signal in accordance to TM2, in TS 25.141 [18], clause 6.1.1.2.

The DPCH intended for power control is on channel 120 starting at -3 dB.

Establish downlink power control with parameters as specified in table 6.3.2.4.1.2-1.

Table 6.3.2.4.1.2-1: DL power control parameters

| Parameter | Level/status | Unit |
| --- | --- | --- |
| UL signal mean power | PREFSENS + 10 dB | dBm |
| Data sequence | PN9 |  |

# 6.3.2.4.1.3	UTRA TDD

Set the initial parameters of the TAB connector transmitted signal according to table 6.3.2.4.1.3-1.

Operate the TAB connector in such a mode that it is able to interpret received TPC commands.

NOTE:	The BS tester used for this test must have the ability:

-	to analyze the output signal of the TAB connector under test with respect to code domain power, by applying the global in-channel Tx test method described in annex E of TS 25.142 [20];

-	to simulate an UE with respect to the generation of TPC commands embedded in a valid UE signal.

Table 6.3.2.4.1.3-1: Initial parameters of the TAB connector signal
for power control steps test for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, ..., 6: transmit, if i is 0, 4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 1 |
| Data content of DPCH | real life (sufficient irregular) |

##### 6.3.2.4.2	Procedure

# 6.3.2.4.2.1	General procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers rated carrier output power per TAB connector (PRated,c,TABC).

# 6.3.2.4.2.2	UTRA FDD

1)	Set and send alternating TPC bits from the UE simulator or UL signal generator.

2)	Measure mean power level of the code under the test each time TPC command is transmitted. All steps within power control dynamic range declared by manufacturer (see table 4.10-1, D6.57) shall be measured. Use the code domain power measurement method defined in annex E in TS 25.141 [18].

3)	Measure the 10 highest and the 10 lowest power step levels within the power control dynamic range declared by manufacturer by sending 10 consecutive equal commands as described in TS 37.105 [8], clause 6.3.2. Table 6.3.2.3-2

In addition, for multi-band TAB connector(s), the following steps shall apply:

1)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.3.2.4.2.3	UTRA TDD

1)	Configure the TAB connector to enable power control steps of size 1 dB.

2)	Set the BS tester to produce a sequence of TPC commands related to the active DPCH. This sequence shall be transmitted to the AAS BS within receive time slots TS i of the AAS BS and shall consist of a series of TPC commands with content "Decrease Tx power", followed by a series of TPC commands with content "Increase Tx power". Each of these series should be sufficiently long so that the code domain power of the active DPCH is controlled to reach its minimum and its maximum, respectively.

3)	Measure the code domain power of the active DPCH over the 848 active chips of each transmit time slot AAS TS i of the AAS BS (this excludes the guard period) by applying the global in-channel Tx test method described in annex E in TS 25.142 [20].

4)	Based on the measurement made in step (5), calculate the power control step sizes and the average rate of change per 10 steps.

5)	Configure the BS transmitter to enable power control steps of 2 dB and of 3 dB, respectively, and repeat steps (4) to (6).

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.3.2.5	Test requirements

##### 6.3.2.5.1	UTRA FDD

For UTRA FDD the test requirement Inner loop power control is:

a)	TAB connector shall fulfil step size requirement shown in table 6.3.2.5.1-1 for all power control steps declared by manufacture in clause 4.10.

b)	For all measured Up/Down cycles, the difference of code domain power between before and after 10 equal commands (Up and Down), derived in step (3), shall not exceed the prescribed tolerance in table 6.3.2.5.1-2.

Table 6.3.2.5.1-1: UTRA FDD TAB connector power control step tolerance

| Power control commands in the downlink | Transmitter power control step tolerance |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 dB step size |  | 1.5 dB step size |  | 1 dB step size |  | 0.5 dB step size |  |
|  | Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| Up(TPC command "1") | +0.9 dB | +3.1 dB | +0.65 dB | +2.35 dB | +0.4 dB | +1.6 dB | +0.15 dB | +0.85 dB |
| Down(TPC command "0") | -0.9 dB | -3.1 dB | -0.65 dB | -2.35 dB | -0.4 dB | -1.6 dB | -0.15 dB | -0.85 dB |

Table 6.3.2.5.1-2: UTRA FDD TAB connector aggregated power control step range

| Power control commands in the downlink | Transmitter aggregated power control step range after 10 consecutive equal commands (up or down) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 dB step size |  | 1.5 dB step size |  | 1 dB step size |  | 0.5 dB step size |  |
|  | Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| Up(TPC command "1") | +15.9 dB | +24.1 dB | +11.9 dB | +18.1 dB | +7.9 dB | +12.1 dB | +3.9 dB | +6.1 dB |
| Down(TPC command "0") | -15.9 dB | -24.1 dB | -11.9 dB | -18.1 dB | -7.9 dB | -12.1 dB | -3.9 dB | -6.1 dB |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.3.2.5.2	UTRA TDD

For UTRA TDD 1,28 Mcps option the test requirement Inner loop power control is:

For all measurements, the tolerance of the power control step sizes and the average rate of change per 10 steps shall be within the limits given in table 6.3.2.5.2-1.

Table 6.3.2.5.2-1: Test Requirements for power control step size tolerance

| Step size | Single step tolerance | Range of average rate of change in code domain power per 10 steps |  |
| --- | --- | --- | --- |
|  |  | Minimum | maximum |
| 1 dB | 0,6 dB | 7,7 dB | 12,3 dB |
| 2 dB | 0,85 dB | 15,7 dB | 24,3 dB |
| 3 dB | 1,1 dB | 23,7 dB | 36,3 dB |

In case, the power control step size is set to 3 dB, the number of power control steps feasible within the power control dynamic range of the TAB connector under test may be less than 10. In this case, the evaluation of the average rate of change in code domain power shall be based on the number of power control steps actually feasible, and the permitted range of average rate of change shall be reduced compared to the values given in table 6.3.2.5.2-1 in proportion to the ratio (number of power control steps actually feasible /10).

EXAMPLE:	If the number of power control steps actually feasible is 9, the minimum and maximum value of the range of average rate of change in code domain power are given by 21,6 dB and 32,4 dB, respectively.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6. 3.3	Power control dynamic range

#### 6.3.3.1	Definition and applicability

The power control dynamic range is the difference between the maximum and the minimum code domain power of a code channel for a specified reference condition.

This requirement applies at each TAB connector supporting transmission in the operating band.

#### 6.3.3.2	Minimum requirement

The minimum requirement for UTRA FDD operation are defined in TS 25.104 [9], clause 6.4.2.1.

The minimum requirement for UTRA TDD 1,28 Mcps option operation is in TS 25.105 [10], clause 6.4.3.1

There is no power control dynamic range requirement for E-UTRA operation.

#### 6.3.3.3	Test purpose

The test purpose is to verify that the power control dynamic range is within the limits specified by the minimum requirement.

#### 6.3.3.4	Method of test

##### 6.3.3.4.1	Initial conditions

# 6.3.3.4.1.1	General test conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

# 6.3.3.4.1.2	UTRA FDD

Set each TAB connector to output a signal in accordance to TM2, in TS 25.141 [18], clause 6.1.1.2.

# 6.3.3.4.1.3	UTRA TDD

Set the initial parameters of the TAB connector transmitted signal according to table 6.3.3.4.1.3-1.

Operate the TAB connector in such a mode that it is able to interpret received TPC commands.

NOTE:	The BS tester used for this test must have the ability:

-	to analyze the output signal of the TAB connector under test with respect to code domain power, by applying the global in-channel Tx test method described in annex E of TS 25.142 [20];

-	to simulate an UE with respect to the generation of TPC commands embedded in a valid UE signal.

Table 6.3.3.4.1.3-1: Parameters of the BS transmitted signal
for power control dynamic range test for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, ..., 6: transmit, if i is 0, 4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 1 |
| Data content of DPCH | real life (sufficient irregular) |

##### 6.3.3.4.2	Procedure

# 6.3.3.4.2.1	General procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC)

# 6.3.3.4.2.1	UTRA FDD

1)	Using TM2, set the code domain power of the DPCH under test to Pmax,c,TABC - 3 dB. Power levels for other code channels may be adjusted if necessary.

2)	Measure the code domain power of the code channel under test. Use the code domain power measurement method defined in annex E in TS 25.141 [18].

3)	Set the code domain power of the DPCH under test to Pmax,c,TABC - 28 dB by means determined by the manufacturer. The power levels for the other code channels used in step 2 shall remain unchanged (the overall output power will drop by approximately 3 dB).

4)	Measure the code domain power of the code channel under test.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.3.3.4.2.2	UTRA TDD

1)	Configure the TAB connector transmitter unit to enable power control steps of size 1 dB.

2)	Set the BS tester to produce a sequence of TPC commands related to the active DPCH, with content "Decrease Tx power". This sequence shall be sufficiently long so that the code domain power of the active DPCH is controlled to reach its minimum, and shall be transmitted to the AAS BS within the receive time slots TS i of the BS.

3)	Measure the code domain power of the active DPCH over the 848 active chips of a transmit time slot TS i of the AAS BS (this excludes the guard period) by applying the global in-channel Tx test method described in annex E of TS 25.142 [20].

4)	Set the BS tester to produce a sequence of TPC commands related to the active DPCH, with content "Increase Tx power". This sequence shall be sufficiently long so that the code domain power of the active DPCH is controlled to reach its maximum, and shall be transmitted to the AAS BS within the receive time slots TS i of the AAS BS.

5)	Measure the code domain power of the active DPCH over the 848 active chips of a transmit time slot TS i of the AAS BS (this excludes the guard period) by applying the global in-channel Tx test method described in annex E in TS 25.142 [20].

6)	Determine the power control dynamic range by calculating the difference between the maximum code domain power measured in step (5) and the minimum code domain power measured in step (7).

7)	Configure the TAB connector transmitter to enable power control steps of 2 dB and of 3 dB, respectively, and repeat steps (4) to (8).

In addition, for multi-band TAB connector(s), the following steps shall apply:

8)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.3.3.5	Test requirements

##### 6.3.3.5.1	UTRA FDD

For UTRA FDD the test requirement Inner loop power control is:

Downlink (DL) power control dynamic range:

-	maximum code domain power: TAB connector maximum output power (Pmax,c,TABC) -4.1 dB or greater;

-	minimum code domain power: TAB connector maximum output power (Pmax,c,TABC) -26.9 dB or less.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.3.3.5.2	UTRA TDD

For UTRA TDD 1,28 Mcps option the power control dynamic range derived according to clause 6.3.2.4 shall be greater than or equal to 29,7dB.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6. 3.4	Total power dynamic range

#### 6.3.4.1	Definition and applicability

The total power dynamic range is the difference between the maximum and the minimum output power for a specified reference condition.

This requirement applies at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the dynamic range is the TAB connector maximum output power (PRated,c,TABC). The lower limit of the dynamic range is the lowest minimum power from the TAB connector when no traffic channels are activated.

Particularly for E-UTRA, the total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

NOTE 2:	The upper limit of the dynamic range at a TAB connector is the OFDM symbol power at maximum output power (PRated,c,TABC) when transmitting on all RBs. The lower limit of the dynamic range at a TAB connector is the OFDM symbol power when one resource block is transmitted. The OFDM symbol carries PDSCH and not contain RS, PBCH or synchronization signals.

#### 6.3.4.2	Minimum requirement

The minimum requirement for UTRA FDD operation are defined in TS 25.104 [9], clause 6.4.3.1.

There is no total power dynamic range requirement for UTRA TDD 1,28 Mcps option operation.

The minimum requirement for E-UTRA operation is in TS 36.104 [11], clause 6.3.2.1.

#### 6.3.4.3	Test purpose

The test purpose is to verify that the total power dynamic range is within the limits specified by the minimum requirement.

#### 6.3.4.4	Method of test

##### 6.3.4.4.1	Initial conditions

# 6.3.4.4.1.1	General test conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

# 6.3.4.4.1.2	UTRA FDD

Base Station RF Bandwidth positions to be tested for multi-carrier: BRFBW, MRFBW and TRFBW in single band operation; see clause 4.12.1.

Set each TAB connector to output a signal in accordance to TM2, in TS 25.141 [18], clause 6.1.1.2.

# 6.3.4.4.1.3	E-UTRA

Set the Channel set-up of the TAB connector transmitted signal according to E-TM 3.1.

##### 6.3.4.4.2	Procedure

# 6.3.4.4.2.1	General procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC)

# 6.3.4.4.2.2	UTRA FDD

The downlink total dynamic range is computed as the difference of the maximum carrier output power, measured as defined in step 3 in clause 6.2.2.4.3 and the carrier power measured at step 3 of the Error Vector Magnitude test, as described in clause 6.5.4.4.2.1.

In addition, for multi-band TAB connector(s), the following steps shall apply:

1)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.3.4.4.2.3	E-UTRA

1)	Measure the average OFDM symbol power as defined in annex F in TS 36.141 [17].

2)	Set the TAB connector to transmit a signal according to E-TM 2.

3)	Measure the average OFDM symbol power as defined in annex F TS 36.141 [17]. The measured OFDM symbols shall not contain RS, PBCH or synchronisation signals.

4)	If BS supports 256QAM, set the channel set-up of the TAB connector transmitted signal according to E-TM 3.1a.and repeat step 1. Set the TAB connector to transmit a signal according to E-TM 2a and repeat step 3.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.3.4.5	Test requirements

##### 6.3.4.5.1	UTRA FDD

For UTRA FDD the downlink total power dynamic range shall be 17.7 dB or greater.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.3.4.5.2	E-UTRA

The downlink (DL) total power dynamic range for each E-UTRA carrier shall be larger than or equal to the level in table 6.3.4.5.1-1.

Table 6.3.4.5.2-1 E-UTRA TAB connector total power dynamic range, paired spectrum

| E-UTRA channel bandwidth (MHz) | Total power dynamic range (dB) |
| --- | --- |
| 1.4 | 7.3 |
| 3 | 11.3 |
| 5 | 13.5 |
| 10 | 16.5 |
| 15 | 18.3 |
| 20 | 19.6 |

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

NOTE 2:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in clause 6.5.4.

### 6.3.5	IPDL time mask

#### 6.3.5.1	Definition and applicability

To support IPDL location method in UTRA FDD operation, the AAS BS shall interrupt all transmitted signals in the downlink (i.e. common and dedicated channels). The IPDL time mask specifies the limits of the TAB connector output power during these idle periods.

This requirement applies only to AAS BS supporting IPDL. The requirement applies at each TAB connector supporting transmission in the operating band.

#### 6.3.5.2	Minimum requirement

The minimum requirement for UTRA FDD operation are defined in TS 25.104 [9], clause 6.4.5.1.

There is no IPDL requirement for UTRA TDD 1,28 Mcps option operation.

There is no IPDL requirement for E-UTRA operation.

#### 6.3.5.3	Test purpose

The test purpose is to verify the ability of the AAS BS to temporarily reduce its output power on each TAB connector below a specified value to improve time difference measurements made by UE for location services.

#### 6.3.5.4	Method of test

##### 6.3.5.4.1	Initial conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

Set each TAB connector to output a signal in accordance to TM1, in clause 4.12.2.

Configure the TAB connector to produce idle periods in continuous mode. The IPDL parameters as defined in TS 25.214 [23] shall have the following values:

-	IP_Spacing = 5

-	IP_Length = 10 CPICH symbols

-	Seed = 0

##### 6.3.5.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

3)	Measure the mean power at the TAB connector over a period starting 27 chips after the beginning of the IPDL period and ending 27 chips before the expiration of the IPDL period.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.3.5.5	Test requirements

The mean power measured according to step (3) in clause 6.3.5.4.2 shall be equal to or less than

TAB connector  maximum output power (Pmax,c,TABC) - 34.3 dB.

See also figure 6.3.5.5-1.

Figure 6.3.5.5-1: IPDL Time Mask

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.3.6	RE Power control dynamic range

#### 6.3.6.1	Definition and applicability

The RE power control dynamic range is the difference between the power of an RE and the average RE power for a TAB connector at maximum output power (PRated,c,TABC) for a specified reference condition.

This requirement applies at each TAB connector supporting transmission in the operating band.

#### 6.3.6.2	Minimum requirement

There is no RE Power control dynamic range requirement for UTRA FDD operation.

There is no RE Power control dynamic range requirement for UTRA TDD 1,28 Mcps option operation.

The minimum requirement for E-UTRA operation are defined in TS 36.104 [11], clause 6.3.1.1.

#### 6.3.6.3	Method of test

No specific test or test requirements are defined for RE Power control dynamic range. The Error Vector Magnitude test, as described in clause 6.5.4 provides sufficient test coverage for this requirement.

## 6.4	Transmit ON/OFF power

### 6.4.1	General

Transmitter ON/OFF power requirements apply only to TDD operation of UTRA and E-UTRA.

### 6.4.2	Transmitter OFF power

#### 6.4.2.1	Definition and applicability

Transmitter OFF power is defined as the mean power measured over 70 s filtered with a square filter of bandwidth equal to the RF bandwidth(s) of the BS centred on the central frequency of the RF bandwidth(s) during the transmitter OFF period.

The requirement applies at each TAB connector supporting transmission in the operating band.

For multi-band TAB connectors and for single band TAB connectors supporting transmission in multiple operating bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

#### 6.4.2.2	Minimum requirement

There is no transmitter off power requirement for UTRA FDD operation.

The minimum requirement for UTRA TDD 1,28 Mcps option operation are defined in TS 37.105 [8], clause 6.4.2.3.

The minimum requirement for E-UTRA operation are defined in TS 37.105 [8], clause 6.4.2.4.

#### 6.4.2.3	Test purpose

The purpose of this test is to verify the TAB connector transmitter OFF power is within the limits of the minimum requirements.

#### 6.4.2.4	Method of test

##### 6.4.2.4.1	Initial conditions

Test environment:

-	normal; see annex B.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested:

-	MRFBW in single band operation, see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see clause 4.12.1.

##### 6.4.2.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set each TAB connector to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.12.2. For single carrier set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

3)	Measure the mean power spectral density measured over 70μs filtered with a square filter of bandwidth equal to the RF bandwidth of the TAB connector centred on the central frequency of the RF bandwidth. 70μs average window centre is set from 35μs after end of one transmitter ON period + 17μs to 35μs before start of next transmitter ON period - 6.25μs.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.4.2.5	Test requirements

The measured mean power spectral density according to clause 6.4.2.4.2 shall be less than -83 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The measured mean power spectral density according to clause 6.4.2.4.2 shall be less than -82.5 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

For multi-band TAB connector, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.4.3	Transmitter transient period

#### 6.4.3.1	Definition and applicability

The transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.3.1-1.

Figure 6.4.3.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at each TAB connector supporting transmission in the operating band.

#### 6.4.3.2	Minimum requirement

The minimum requirement for MSR operation is in TS 37.104 [12], clause 6.4.2.1.

There is no Transmitter transient period requirement for UTRA FDD operation.

The minimum requirement for single RAT UTRA TDD 1,28 Mcps option operation are defined in TS 25.105 [10], clause 6.5.2.1.2.

The minimum requirement for single RAT E-UTRA operation are defined in TS 36.104 [11], clause 6.4.2.1.

#### 6.4.3.3	Test purpose

The purpose of this test is to verify the TAB connector transmitter transient periods are within the limits of the minimum requirements.

#### 6.4.3.4	Method of test

##### 6.4.3.4.1	Initial conditions

# 6.4.3.4.1.1	MSR operation

For MSR operation the test for transmitter off power in clause 6.4.2 demonstrates compliance.

# 6.4.3.4.1.2	UTRA TDD operation

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier (SC):

-	B, M and T; see clause 4.12.1.

RF bandwidth positions to be tested:

-	BRFBW, MRFBW and TRFBW in single band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

Table 6.4.3.4.1.2-1: Parameters of the transmitted signal
for transmit ON/OFF time mask test for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is UpPCH,1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 8 |
| Data content of DPCH | real life (sufficient irregular) |

# 6.4.3.4.1.3	E-UTRA operation

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	M; see clause 4.12.1.

RF bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

##### 6.4.3.4.2	Procedure

# 6.4.3.4.2.1	MSR operation

For MSR operation the test for transmitter off power in clause 6.4.2 demonstrates compliance.

# 6.4.3.4.2.2	UTRA TDD operation

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	Set each TAB connector to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.12.2. For single carrier set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

3)	Measure the RRC filtered mean power of the TAB connector output signal chipwise (i.e. averaged over time intervals of one chip duration) over the transmit off power period starting 11 chips before the start of the receive time slot TS i = UpPCH, and ending 8 chips before the next transmit time slot TS i=4 starts.

# 6.4.3.4.2.3	E-UTRA operation

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

2)	For a Set each TAB connector to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.12.2. For single carrier set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

3)	Measure the mean power spectral density over 70μs filtered with a square filter of bandwidth equal to the RF bandwidth of the TAB connector centred on the central frequency of the RF bandwidth. 70μs average window centre is set from 35μs after end of one transmitter ON period + 17 μs to 35 μs before start of next transmitter ON period - 17 μs.

4)	For a TAB connector supporting contiguous CA, measure the mean power spectral density over 70 μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70 μs average window centre is set from 35 μs after end of one transmitter ON period + 17 μs to 35μs before start of next transmitter ON period - 17 μs.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.4.3.5	Test requirements

##### 6.4.3.5.1	MSR operation

For MSR operation the test for transmitter off power in clause 6.4.2 demonstrates compliance.

##### 6.4.3.5.1	UTRA TDD operation

Each value of the power measured according to clause 6.4.3.4.2.2 shall be below -41,3 dBm in the period from 85 chips to 88 chips after the burst and below -80 dBm in the period where the Tx OFF power specification is applicable.

For multi-band TAB connector, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

##### 6.4.3.5.1	E-UTRA operation

The measured mean power spectral density according to clause 6.3.5.4.3 shall be less than -83 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The measured mean power spectral density according to clause 6.3.5.4.3 shall be less than -82.5 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

For multi-band TAB connector, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 6.5	Transmitted signal quality

### 6.5.1	General

The requirements apply per TAB connector unless otherwise stated differently.

The requirement applies during the transmitter ON period.

### 6.5.2	Frequency Error

#### 6.5.2.1	Definition and applicability

Frequency error is the measure of the difference between the actual AAS BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

It is not possible to verify by testing that the data clock is derived from the same frequency source as used for RF generation. This may be confirmed by the manufacturer's declaration.

#### 6.5.2.2	Minimum Requirement

The minimum requirement is in TS 37.105 [8], clause 6.5.2.

#### 6.5.2.3	Test purpose

The test purpose is to verify that frequency error is within the limit specified by the minimum requirement.

#### 6.5.2.4	Method of test

Requirement is tested together with modulation quality test, as described in clause 6.5.4.

#### 6.5.2.5	Test Requirements

##### 6.5.2.5.1	UTRA FDD test requirement

The Frequency Error for every measured slot shall be between the minimum and maximum value specified in table 6.5.2.5.1-1.

Table 6.5.2.5.1-1: Frequency error test requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±(0.05 ppm + 12 Hz) |
| Medium Range BS | ±(0.1 ppm + 12 Hz) |
| Local Area BS | ±(0.1 ppm + 12 Hz) |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.5.2.5.2	UTRA TDD test requirement

The Frequency Error for every measured slot shall be between the minimum and maximum value specified in table 6.5.2.5.2-1.

Table 6.5.2.5.2-1: Frequency error test requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±(0.05 ppm + 12 Hz) |
| Medium Range BS | ±(0.1 ppm + 12 Hz) |

##### 6.5.2.5.3	E-UTRA test requirement

The modulated carrier frequency of each E-UTRA carrier configured by the AAS BS shall be accurate to within the accuracy range given in table 6.5.2.5.3-1 observed over a period of one subframe (1 ms).

Table 6.5.2.5.3-1: Frequency error test requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±(0.05 ppm + 12 Hz) |
| Medium Range BS | ±(0.1 ppm + 12 Hz) |
| Local Area BS | ±(0.1 ppm + 12 Hz) |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.5.3	Time alignment error

#### 6.5.3.1	Definition and applicability

This requirement applies to frame timing in:

-	UTRA single/multi-carrier transmissions and their combinations with MIMO or TX diversity.

-	E-UTRA single/multi-carrier transmissions and their combinations with MIMO or TX diversity.

-	E-UTRA carrier aggregation, with or without MIMO or TX diversity.

Frames of the UTRA/E-UTRA signals present at the TAB connectors are not perfectly aligned in time. In relation to each other, the RF signals present at the transceiver array boundary may experience certain timing differences.

For a specific set of signals/transmitter configuration/transmission mode, the Time Alignment Error (TAE) is defined as the largest timing difference between any two different E-UTRA signals or any two different UTRA signals belonging to different TAB Connectors belonging to different transmitter groups at the transceiver array boundary, where transmitter groups are associated with the TAB connectors in the transceiver unit array corresponding to TX diversity, MIMO transmission, carrier aggregation, etc.

#### 6.5.3.2	Minimum requirement

The minimum requirement is in TS 37.105 [8], clause 6.5.3.

#### 6.5.3.3	Test purpose

To verify that the time alignment error is within the limit specified by the minimum requirement.

#### 6.5.3.4	Method of test

##### 6.5.3.4.1	Initial conditions

# 6.5.3.4.1.1	General test conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	M; see clause 4.12.1.

# 6.5.3.4.1.2	UTRA FDD

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see clause 4.12.1.

Refer to clause D.1.3 for a functional block diagram of the test set-up.

# 6.5.3.4.1.3	UTRA TDD

RF bandwidth positions to be tested for multi-carrier: MRFBW in single band operation, see clause 4.12.1.

-	For a TAB connectors declared to be capable of single carrier operation only, set the base station to transmit according to table 6.5.3.4.1.3-1 on one cell using MIMO.

-	For a multi-carrier TAB connectors, set to transmit according to table 6.5.3.4.1.3-1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 5.3.

Table 6.5.3.4.1.3-1: Parameters of the BS transmitted signal for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, ..., 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS0 and DwPTS |
| Spreading factor | 16 |

# 6.5.3.4.1.4	E-UTRA

RF bandwidth positions to be tested for multi-carrier and/or CA: MRFBW in single-band operation, see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.7.

##### 6.5.3.4.2	Procedure

# 6.5.3.4.2.1	General Procedure

TAB connectors to be tested are identified from the declared sets of TAB connector beam forming groups (see clause 4.10 D.6.58).

Connect two representative TAB connectors one from each of the declared groups to the measurement equipment according to clause D.1.3. Terminate any unused TAB connector(s).

Compliance is to be demonstrated between all pairs of TAB connectors beam forming groups, however it is not required to exhaustively measure the time alignment error between every combination of pairs of representative TAB connectors. Compliance can be demonstrated by comparison of a reduced set of representative measurement results.

# 6.5.3.4.2.2	UTRA FDD Procedure

1)	If the AAS BS supports TX diversity or MIMO, set the TAB connectors to transmit TM1, clause 4.12.2, at manufacturer's declared rated output power, PRated,c,TABC on one cell using TX diversity or MIMO.

2)	Measure the time alignment error between the signals using the P-CPICH on one of the representative TAB connector from the main signal conveyed via a TAB connectors beam forming group and the CPICH on the TAB connector from the diversity signal conveyed via another TAB connectors beam forming group.

3)	If the AAS BS supports DC-HSDPA, 4C-HSDPA, NC-4C-HSDPA or 8C-HSDPA set the TAB connectors to transmit according to TM1, without using TX diversity or MIMO, on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

4)	Measure the time alignment error between the signals using the P-CPICH on one of the representative TAB connector and CPICH signals on representative TAB connector from another group.

5) If the AAS BS supports DB-DC-HSDPA or any of the multi-band 4C-HSDPA or 8C-HSDPA configurations set the TAB connectors to transmit TM1 on two carriers belonging to different frequency bands, without using TX diversity or MIMO on any of the carriers.

6) Measure the time alignment error between the signals using the P-CPICH and CPICH signals on the TAB connectors.

In addition, for multi-band TAB connector(s), the following steps shall apply:

7)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.5.3.4.2.3	UTRA TDD Procedure

1)	Start the TAB connector beam forming groups transmission at the manufacturer's specified rated output power, PRated,c,TABC at the TAB connector.

2)	Measure the time alignment error between the P-CCPCH and DwPTS on the representative TAB connectors under test.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.5.3.4.2.4	E-UTRA Procedure

1)	Set the AAS BS to transmit E-TM1.1 or any DL signal using TX diversity, MIMO transmission or carrier aggregation.

NOTE:	For TX diversity and MIMO transmission, different ports may be configured in E-TM (using p = 0 and 1).

For an AAS BS declared to be capable of single carrier operation only, set the representative TAB connectors to transmit according to manufacturer's declared rated output power, PRated,c,TABC.

If the AAS BS supports intra band contiguous or non-contiguous Carrier Aggregation set the representative TAB connectors to transmit using the applicable test configuration and corresponding power setting specified in clauses 4.10 and 4.11.

If the AAS BS supports inter band carrier aggregation set the representative TAB connectors to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in clauses 4.10 and 4.11.

2)	Measure the time alignment error between the reference symbols on the carrier(s) from the representative TAB connector(s).

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.5.3.5	Test requirement

##### 6.5.3.5.1	UTRA FDD test requirement

For Tx diversity and MIMO transmission, in the tested cell, TAE shall not exceed 0.35 Tc.

For transmission of multiple cells within a frequency band TAE shall not exceed 0.6 Tc.

For transmission of multiple cells in different frequency bands TAE shall not exceed 5.1 Tc.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.5.3.5.2	UTRA TDD test requirement

The time alignment error shall be less than 65 + 78 ns.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.5.3.5.3	E-UTRA test requirement

For MIMO or TX diversity transmissions, at each carrier frequency, TAE shall not exceed 90 ns.

For intra-band carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 155 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 285 ns.

For inter-band carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 285 ns.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.5.4	Modulation quality

#### 6.5.4.1	Definition and applicability

Modulation quality is defined by the difference between the measured carrier signal and a reference signal. Modulation quality can be expressed e.g. as Peak Code domain Error (PCDE) or Relative Code domain Error (RCDE) or Error Vector Magnitude (EVM) for UTRA and Error Vector Magnitude (EVM) for E-UTRA.

#### 6.5.4.2	Minimum Requirement

The minimum requirement for UTRA operation are defined in TS 37.105 [8], clause 6.5.4.3.

The minimum requirement for E-UTRA operation are defined in TS 37.105 [8], clause 6.5.4.4.

#### 6.5.4.3	Test purpose

The test purpose is to verify that modulation quality is within the limit specified by the minimum requirement.

#### 6.5.4.4	UTRA FDD Method of test

##### 6.5.4.4.1	Initial conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth position to be tested:

-	BRFBW, MRFBW and TRFBW single-band operation, see clause 4.12.1 single-band operation.

##### 6.5.4.4.2	Procedure

# 6.5.4.4.2.1	EVM procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to TM1 according to clause 4.12.2 at manufacturer's declared rated output power, PRated,c,TABC.

For a TAB connector declared to be capable of multi-carrier operation, set the TAB connector to transmit according to clause 4.12.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

2)	For each carrier, measure the Error Vector Magnitude and frequency error as defined in annex D.1.1 and the mean power of the signal. The measurement shall be performed on all 15 slots of the frame defined by the Test Model.

3)	Using the same setting as in step 1), set the TAB connector to transmit a signal according to TM4, clause 4.12.2, with X value equal to 18, and repeat step 2). If the requirement in clause 6.5.4.5 is not fulfilled, decrease the total output power by setting the base station to transmit a signal according to TM4 with X greater than 18, and repeat step 2).

The following test shall be additionally performed if the base station supports HS-PDSCH transmission using 16QAM:

4)	Using the same setting as in step 1), set the base station to transmit according to TM5, clause 4.12.2.

5)	Repeat step 2).

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.5.4.4.2.2	PCDE  procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to TM3, clause 4.12.2, at manufacturer's declared rated output power, PRated,c,TABC

For a TAB connector declared to be capable of multi-carrier operation, set the TAB connector to transmit according to TM3 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

2)	Measure Peak code domain error according to annexD.1.1. The measurement shall be performed on all 15 slots of the frame defined byTM3. For a TAB connector declared to be capable of multi-carrier operation the measurement is performed on one of the carriers under test.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.5.4.4.2.3	RCDE  procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to TM6, clause 4.12.2, at manufacturer's declared rated output power, PRated,c,TABC

For a TAB connector declared to be capable of multi-carrier operation, set the TAB connector to transmit according to TM6, clause 4.12.2, on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

2)	Measure average Relative code domain error according to annex E. The measurement shall be performed over one frame defined by TM6 and averaged as specified in clause 4.12.2. For a TAB connector declared to be capable of multi-carrier operation the measurement is performed on one of the carriers under test.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.5.4.5	UTRA TDD Method of test

##### 6.5.4.5.1	Initial conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth position to be tested:

-	BRFBW, MRFBW and TRFBW single-band operation, see clause 4.12.1 single-band operation.

Table 6.5.4.5.1-1: Parameters of the TAB connector transmitted signal
for modulation accuracy testing at maximum TAB connector output power for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, ..., 6: Transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 10 |
| Power of each DPCH | 1/10 of Base Station output power |
| Data content of DPCH | real life (sufficient irregular) |
| Spreading factor | 16 |

In addition the following test set up only applies for 16QAM capable BS.

Table 6.5.4.5.1-2: Parameters of the TAB connector transmitted signal for modulation accuracy testing at maximum TAB connector output power setting for 1,28 Mcps TDD - 16QAM capable BS

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| HS-PDSCH modulation | 16QAM |
| Number of HS-PDSCH in each time slot under test | 10 |
| Power of each HS-PDSCH | 1/10 of Base Station output power |
| Data content of HS-PDSCH | Real life (sufficient irregular) |
| Spreading factor | 16 |

##### 6.5.4.5.2	Procedure

# 6.5.4.5.2.1	EVM  Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only, set the parameters of the TAB connector transmitted signal according to table 6.5.4.5.1-1at manufacturer's declared output power, PRated,c,TABC.

For a TAB connector declared to be capable of multi-carrier operation, set the TAB connector to transmit according to table 6.5.4.5.1-1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 5.3.

2)	Measure the error vector magnitude (EVM) for each carrier by applying the global in-channel Tx test method described in annex E with the TAB connector transmitted signal set as described in table 6.5.4.5.1-1.

3)	Measure the error vector magnitude (EVM) for each carrier by applying the global in-channel Tx test method described in annex C with the TAB connector transmitted signal on each carrier set as described in table 6.5.4.5.2.1-1.

4)	For TAB connector declared to be capable of 16QAM repeat steps 2 and 3 using transmitted signal set as described in tables 6.5.4.5.1-2 and 6.5.4.5.2.1-2.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

Table 6.5.4.5.2.1-1: Parameters of the TAB connector transmitted signal
for modulation accuracy testing at minimum TAB connector output power for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, ..., 6: Transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slot under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 1 |
| BS output power setting on each carrier | Maximum output power - 30 dB |
| Data content of DPCH | Real life (sufficient irregular) |

Table 6.5.4.5.2.1-2: Parameters of the TAB connector transmitted signal for modulation accuracy
testing at minimum TAB connector output power setting for 1,28 Mcps TDD - 16QAM capable BS

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| HS-PDSCH modulation | 16QAM |
| Time slots under test | TS4, TS5 and TS6 |
| Number of HS-PDSCH in each time slot under test | 1 |
| BS output power setting on each carrier | Maximum output power - 30 dB |
| Data content of HS-PDSCH | Real life (sufficient irregular) |
| Spreading factor | 16 |

# 6.5.4.5.2.2	PCDE  Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only, set the parameters of the TAB connector transmitted signal according to table 6.5.4.5.2.1-1 at manufacturer's declared output power, PRated,c,TABC.

For a TAB connector declared to be capable of multi-carrier operation, set the TAB connector to transmit according to table 6.5.4.5.2.1-1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 5.3.

2)	Measure the Peak code domain error by applying the global in-channel Tx test method described in annex E.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.5.4.5.2.3	RCDE  Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only, set the parameters of the TAB connector transmitted signal according to table 6.5.4.5.2.1-1 at manufacturer's declared output power, PRated,c,TABC.

For a TAB connector declared to be capable of multi-carrier operation, set the TAB connector to transmit according to table 6.5.4.5.2.1-1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 5.3.

2)	Measure the Relative code domain error by applying the global in-channel Tx test method described in annex E.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.5.4.6	E-UTRA Method of test

##### 6.5.4.6.1	Initial conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW, MRFBW and TRFBW in single-band operation, see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

##### 6.5.4.6.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	For a TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to E-TM 3.1 at manufacturer's declared rated output power.

For a TAB connector declared to be capable of multi-carrier and/or CA operation, set the TAB connector to transmit according to E-TM3.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.10 and 4.11.

2)	Measure the EVM and frequency error as defined in annex F.

3)	Repeat steps 1 and 2 for E-TM 3.2, E-TM 3.3 and E-TM 2. Repeat steps 1 and 2 for E-TM3.1a and E-TM 2a for 256QAM, if supported by the BS. For E-TM2 and E-TM2a the OFDM symbol power shall be at the lower limit of the dynamic range according to the test procedure in clause 6.3.4.4.2.3 and test requirements in clause 6.3.4.5.2.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.5.4.7	Test Requirements

##### 6.5.4.7.1	UTRA FDD test requirement

The Error Vector Magnitude for each UTRA carrier and every measured slot shall be less than 17.5% when the TAB connector is transmitting a composite signal using only QPSK modulation and shall be less than 12.5 % when the TAB connector is transmitting a composite signal that includes 16QAM modulation.

The peak code domain error for every measured slot shall not exceed -32 dB at spreading factor 256.

The average Relative Code Domain Error for 64QAM modulated codes shall not exceed -20 dB at spreading factor 16.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.5.4.7.2	UTRA TDD test requirement

The error vector magnitude (EVM) for each carrier measured according to clause 6.5.4.5.2.1 shall not exceed 12,5 %.

The peak code domain error measured according to clause 6.5.4.5.2.2 shall not exceed -27 dB.

The Relative code domain error measured according to clause 6.5.4.5.2.3 shall not exceed -20.9 dB.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.5.4.7.3	E-UTRA test requirement

The EVM of each E-UTRA carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.5.4.7.3-1.

Table 6.5.4.7.3-1 EVM requirements

| Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 % |
| 16QAM | 13.5 % |
| 64QAM | 9 % |
| 256QAM | 4.5 % |

The EVM requirement shall be applicable within a time period around the centre of the CP therefore the EVM requirement is tested against the maximum of the RMS average of 10 subframes at the two window W extremities.

Table 6.5.4.7.3-2 specifies EVM window length (W) for normal CP, the cyclic prefix length  is 160 for symbols 0 and 144 for symbols 1-6.

Table 6.5.4.7.3-2 EVM window length for normal CP

| Channel Bandwidth MHz | FFT size | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6 (%) (Note) |
| --- | --- | --- | --- | --- | --- |
| 1.4 | 128 | 10 | 9 | 5 | 55.6 |
| 3 | 256 | 20 | 18 | 12 | 66.7 |
| 5 | 512 | 40 | 36 | 32 | 88.9 |
| 10 | 1024 | 80 | 72 | 66 | 91.7 |
| 15 | 1536 | 120 | 108 | 102 | 94.4 |
| 20 | 2048 | 160 | 144 | 136 | 94.4 |
| NOTE: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 6.6	Unwanted Emissions

### 6.6.1	General

Unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions [14]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

For E-UTRA single RAT BS and MSR BS, the out-of-band emissions requirement for the BS transmitter is specified in terms of an Operating band unwanted emissions requirement that defines limits for emissions in each supported downlink operating band plus the frequency ranges 10 MHz above and 10 MHz below each band. Emissions outside of this frequency range are limited by a spurious emissions requirement. For UTRA single RAT AAS BS, the out of band emission requirement for BS transmitter is specified in terms of Spectrum emission mask.

The unwanted emission level limit of a TAB connector TX min cell group is in general defined by the unwanted emission basic limit which is the same as the corresponding applicable Non-AAS BS per transmitter requirement specified in [2], [3], [4] or [5],and its scaling by NTXU,countedpercell. The unwanted emission requirements are applied per the TAB connector TX min cell groups for all the configurations supported by the AAS BS. The basic limits and corresponding scaling are defined in each relevant clause.

There are in addition a requirement for occupied bandwidth and an ACLR requirement.

### 6.6.2	Occupied bandwidth

#### 6.6.2.1	Definition and applicability

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also Recommendation ITU-R SM.328 [17].

The value of /2 shall be taken as 0.5%.

The occupied bandwidth requirement applies during the transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in the present clause.

#### 6.6.2.2	Minimum requirement

For MSR AAS BS, the minimum requirement for occupied bandwidth is the same as that stated in TS 37.104 [12], clause 6.6.3.

For single RAT UTRA FDD AAS BS, the minimum requirement for occupied bandwidth is the same as that stated in TS 25.104 [9], clause 6.6.1.

For single RAT UTRA TDD, 1,28Mcps option AAS BS, the minimum requirement for occupied bandwidth is the same as that stated in TS 25.105 [10], clause 6.6.1.

For single RAT E-UTRA AAS BS, the minimum requirement for occupied bandwidth is the same as that stated in TS 36.104 [11], clause 6.6.1.

#### 6.6.2.3	Test purpose

The test purpose is to verify that the emission of the TAB connector does not occupy an excessive bandwidth for the service to be provided and is, therefore, not likely to create interference to other users of the spectrum beyond undue limits.

#### 6.6.2.4	Method of test

##### 6.6.2.4.1	Initial conditions

# 6.6.2.4.1.1	General test conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested:

-	B, M and T; see clause 4.12.1.

# 6.6.2.4.1.2	UTRA FDD

Set the TAB connector to transmit a signal in accordance to TM1, clause 4.12.2.

# 6.6.2.4.1.3	UTRA TDD

Set the parameters of the BS transmitted signal according to table 6.6.2.4.1.3-1.

Table 6.6.2.4.1.3-1: Parameters of the TAB connector transmitted signal
for occupied bandwidth testing for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 8 |
| Power of each DPCH | 1/8 of Base Station output power |
| Data content of DPCH | real life (sufficient irregular) |

# 6.6.2.4.1.4	E-UTRA

Aggregated Channel Bandwidth positions to be tested for contiguous carrier aggregation:

-	BBW Channel CA, MBW Channel CA and TBW Channel CA; see clause 4.12.1.

For a AAS BS declared to be capable of single carrier operation, start transmission according to E- TM1.1, clause 4.12.2.

##### 6.6.2.4.2	Procedure

# 6.6.2.4.2.1	General Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

2)	For UTRA and  E-UTRA declared capable of single carrier operation set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

For E-UTRA declared capable of contiguous carrier aggregation operation set the TAB connector to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 5.

# 6.6.2.4.2.2	UTRA FDD

1)	Measure the spectrum of the transmitted signal across a span of 10 MHz, based on an occupied bandwidth requirement of 5 MHz. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less. The spectrum shall be measured at 400 or more points across the measurement span.

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode is power responding. There are at least two ways to be power responding. The spectrum analyser can be set to "sample" detection, with its video bandwidth setting at least three times its RBW setting. Or the analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

2)	Compute the total of the power, P0, (in power units, not decibel units) of all the measurement cells in the measurement span. Compute P1, the power outside the occupied bandwidth on each side. P1 is half of the total power outside the bandwidth. P1 is half of (100 % - (occupied percentage)) of P0. For the occupied percentage of 99 %, P1 is 0.005 times P0.

3)	Determine the lowest frequency, f1, for which the sum of all power in the measurement cells from the beginning of the span to f1 exceeds P1.

4)	Determine the highest frequency, f2, for which the sum of all power in the measurement cells from the end of the span to f2 exceeds P1.

5)	Compute the occupied bandwidth as f2 - f1.

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.6.2.4.2.3	UTRA TDD

1)	Measure the power of the transmitted signal with a measurement filter of bandwidth 30 kHz. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyser filter). The centre frequency of the filter shall be stepped in contiguous 30 kHz steps from a minimum frequency, which shall be (2,4 - 0,015) MHz below the assigned channel frequency of the transmitted signal, up to a maximum frequency, which shall be (2,4 - 0,015) MHz above the assigned channel frequency of the transmitted signal. The time duration of each step shall be sufficiently long to capture one active time slot. The measured power shall be recorded for each step.

2)	Determine the total output power by accumulating the recorded power measurement results of all steps.

3)	Sum up the recorded power measurement results, starting from the step at the minimum frequency defined in (3) up to the step at a lower limit frequency by which this sum is equal to or greater than 0,5 % of the total output power determined in (4). This limit frequency is recorded as "Lower Frequency".

4)	Sum up the recorded power measurement results, starting from the step at the maximum frequency defined in (3) down to the step at an upper limit frequency by which this sum is equal to or greater than 0,5 % of the total output power determined in (4). This limit frequency is recorded as "Upper Frequency".

5)	Calculate the occupied bandwidth as the difference between the "Upper Frequency" obtained in (5) and the "Lower Frequency" obtained in (6).

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.6.2.4.2.4	E-UTRA

1)	Measure the spectrum emission of the transmitted signal using at least the number of measurement points, and across a span, as listed in table 6.6.2.4.2.4-1. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less.

Table 6.6.2.4.2.4-1: Span and number of measurement points for OBW measurements

| Bandwidth | E-UTRA channel bandwidth BWChannel (MHz) |  |  |  |  |  | Aggregated channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 | 3 | 5 | 10 | 15 | 20 | > 20 |
| Span (MHz) | 10 | 10 | 10 | 20 | 30 | 40 |  |
| Minimum number of measurement points | 1429 | 667 | 400 | 400 | 400 | 400 |  |

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode is power responding.  The analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

2)	Compute the total of the power, P0, (in power units, not decibel units) of all the measurement cells in the measurement span. Compute P1, the power outside the occupied bandwidth on each side. P1 is half of the total power outside the bandwidth. P1 is half of (100 % - (occupied percentage)) of P0. For the occupied percentage of 99 %, P1 is 0.005 times P0.

3)	Determine the lowest frequency, f1, for which the sum of all power in the measurement cells from the beginning of the span to f1 exceeds P1.

4)	Determine the highest frequency, f2, for which the sum of all power in the measurement cells from f2 to the end of the span exceeds P1.

5)	Compute the occupied bandwidth as f2 - f1.

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.2.5	Test requirements

##### 6.6.2.5.1	MSR

The occupied bandwidth of a single carrier shall be less than the values listed in table 6.6.2.5.1-1. In addition, for E-UTRA intra-band contiguous carrier aggregation, test requirement in clause 6.6.1.5 of TS 36.141 [17] applies for the E-UTRA component carriers that are aggregated.

Table 6.6.2.5.1-1: Occupied bandwidth

| RAT | Occupied bandwidth limit |
| --- | --- |
| E-UTRA | BWChannel |
| UTRA FDD | 5 MHz |
| 1.28 Mcps UTRA TDD | 1.6 MHz |

##### 6.6.2.5.2	UTRA FDD

The occupied bandwidth shall be less than 5 MHz based on a chip rate of 3,84 Mcps.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.6.2.5.3	UTRA TDD

The occupied bandwidth shall be less than 1,6 MHz.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.6.2.5.4	E-UTRA

The occupied bandwidth for each E-UTRA carrier shall be less than the channel bandwidth. For contiguous CA, the occupied bandwidth shall be less than or equal to the Aggregated Channel Bandwidth as defined in TS 36.141 [17] clause 5.6.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.6.3	Adjacent Channel Leakage power Ratio

#### 6.6.3.1	Definition and applicability

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

NOTE:	Conformance to the AAS ACLR requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The ratio of the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the assigned channel frequency to the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency shall be greater than or equal to the AAS ACLR limit. This applies for each TAB connector TX min cell group.

Or

2)	The ratio of the filtered mean power at the TAB connector centred on the assigned channel frequency to the filtered mean power at each TAB connector centred on the adjacent channel frequency shall be greater than or equal to the AAS limit for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group.

#### 6.6.3.2	Minimum requirement

For single RAT UTRA FDD operation the AAS BS minimum requirements are the same as those specified in TS 37.105 [8], clause 6.6.3.2.

For single RAT UTRA FDD operation, the AAS BS minimum requirements are the same as those specified in TS 25.104 [9], clauses 6.6.2.2.1 and 6.6.2.2.2.

For single RAT UTRA TDD 1,28 Mcps option operation, the AAS BS minimum requirements are the same as those specified in TS 25.105 [10], clause 6.6.2.2.

For single RAT E-UTRA operation, the AAS BS minimum requirements are the same as those specified in TS 36.104 [11], clauses 6.6.2.1 and 6.6.2.2.

#### 6.6.3.3	Test purpose

To verify that the adjacent channel leakage power ratio requirement shall be met as specified by the minimum requirement.

#### 6.6.3.4	Method of test

##### 6.6.3.4.1	Initial conditions

# 6.6.3.4.1.1	General test conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

# 6.6.3.4.1.2	MSR

For E-UTRA ACLR requirement outside the Base Station RF Bandwidth edges and the ACLR requirement applied inside sub-block gap, in addition, for non-contiguous spectrum operation or Inter RF Bandwidth gap for multi-band operation using, the test configurations defined in clause 4.8, the method of test described in clauses 6.6.4.4.1 and 6.6.4.4.2 applies.

# 6.6.3.4.1.3	UTRA FDD

Set the base station to transmit a signal modulated in accordance to TM1, in clause 4.12.2.

For a TAB connector declared to be capable of multi-carrier operation, set the base station to transmit according to TM1 on all carriers configured

# 6.6.3.4.1.4	UTRA TDD

For a TAB connector declared to be capable of single carrier operation only, set the parameters of the transmitted signal according to table 6.6.3.4.1.4-1.

For a TAB connector declared to be capable of multi-carrier operation set the parameters of the transmitted signal according to table 6.6.3.4.1.4-1on all carriers.

Table 6.6.3.4.1.4-1: Parameters of the transmitted signal for ACLR testing for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 8 |
| Power of each DPCH | 1/8 of Base Station output power |
| Data content of DPCH | real life (sufficient irregular) |

For a TAB connector declared capable of supporting 16QAM capable.

For a TAB connector declared to be capable of single carrier operation only, set the parameters of the transmitted signal according to table 6.6.3.4.1.4-2.

For a TAB connector declared to be capable of multi-carrier operation, set the parameters of the transmitted signal according to table 6.6.3.4.1.4-2 on all carriers.

Table 6.6.3.4.1.4-2: Parameters of the transmitted signal
for ACLR testing for 1,28 Mcps TDD- 16QAM capable BS

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| HS-PDSCH modulation | 16QAM |
| Number of HS-PDSCH in each time slot under test | 8 |
| Power of each HS-PDSCH | 1/8 of Base Station output power |
| Data content of HS-PDSCH | real life (sufficient irregular) |
| Spreading factor | 16 |

# 6.6.3.4.1.3	E-UTRA

For a TAB connector declared to be capable of single carrier operation only set to transmit a signal according to E-TM1.1. in clause 4.12.2.

For a TAB connector declared to be capable of multi-carrier and/or CA operation, set to transmit according to E-TM1.1 on all carriers configured.

##### 6.6.3.4.2	Procedure

# 6.6.3.4.2.1	General procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

The measurement device characteristics shall be:

-	measurement filter bandwidth: defined in clause 6.6.3.5;

-	detection mode: true RMS voltage or true average power.

2)	For single carrier operation set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).

For a TAB connector declared to be capable of multi-carrier and/or CA operation set the TAB connector to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 5 using the corresponding test models or set of physical channels in clause 4.12.

# 6.6.3.4.2.2	MSR

1)	For E-UTRA, measure ACLR:

-	outside the Base Station RF Bandwidth edges;

-	inside sub-block gap for non-contiguous spectrum operation as specified in clause 6.6.4.5.1;

-	inside Inter RF Bandwidth gap for multi-band operation.

2)	For UTRA FDD, measure ACLR inside sub-block gap or Inter RF Bandwidth gap as specified in clause 6.6.4.5.2.

3)	Measure Cumulative Adjacent Channel Leakage power Ratio (CACLR) inside sub-block gap or the Inter RF Bandwidth gap as specified in clause 6.6.4.5.4.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.6.3.4.2.3	UTRA FDD

1)	Measure Adjacent channel leakage power ratio for 5 MHz and 10 MHz offsets both side of channel frequency. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

2)	For the ACLR requirement applied inside sub-block gap for non-contiguous spectrum operation or inside Inter RF Bandwidth gap for multi-band operation:

a)	Measure ACLR inside sub-block gap or Inter RF Bandwidth gap as specified in clause 6.6.3.5.4.1, if applicable.

b)	Measure Cumulative Adjacent Channel Leakage power Ratio (CACLR) inside sub-block gap or Inter RF Bandwidth gap as specified in clause 6.6.3.5.4.2, if applicable.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.6.3.4.2.4	UTRA TDD 1,28Mcps option

1)	Measure the RRC filtered mean power centred on the lowest assigned channel frequency of a operating band over the 848 active chips of the transmit time slots TS i (this excludes the guard period).

2)	Average over at least one time slot.

3)	Measure the RRC filtered mean power at the first lower adjacent RF channel (centre frequency 1,6 MHz below the lowest assigned channel frequency of the transmitted signal) over the useful part of the burst within the transmit time slots TS i (this excludes the guard period).

4)	Average over at least one time slot.

5)	Calculate the ACLR by the ratio:

ACLR = average power acc. to (4) / average interference power acc. to (6).

6)	Repeat steps (5), (6) and (7) for the second lower adjacent RF channel (centre frequency 3,2 MHz below the lowest assigned channel frequency of the transmitted signal) and also for the first and second upper adjacent RF channel (centre frequency 1,6 MHz and 3,2 MHz above the assigned channel frequency of the transmitted signal, respectively).

7)	In case of a multi-carrier TAB connector, repeat steps (3) and (4) for the highest assigned channel frequency. Otherwise, use the result obtained in step (4) above for further calculation in step (12).

8)	Measure the RRC filtered mean power at the first higher adjacent RF channel (centre frequency 1,6 MHz above the highest assigned channel frequency of the transmitted signal) over the useful part of the burst within the transmit time slots TS i (this excludes the guard period).

9)	Average over at least one time slot.

10)	Calculate the ACLR by the ratio

ACLR = average power acc. to (9) / average interference power acc. to (11).

11)	Repeat steps (10) to (12) for the second upper adjacent RF channel (centre frequency 3,2 MHz above the highest assigned channel frequency of the transmitted signal).

In addition, for multi-band TAB connector(s), the following steps shall apply:

12)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

In addition for a 16QAM capable TAB connector

The same procedure applies to 1,28 Mcps TDD option supporting 16QAM.

# 6.6.3.4.2.5	E-UTRA

1)	Measure Adjacent channel leakage power ratio for the frequency offsets both side of channel frequency as specified in table 6.6.3.5.6.1-1 (Paired spectrum case) or Table 6.6.3.5.6.1-2 (Unpaired spectrum case) respectively. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

2)	For the ACLR requirement applied inside sub-block gap for non-contiguous spectrum operation: or inside Inter RF Bandwidth gap for multi-band operation:

a)	Measure ACLR inside sub-block gap or Inter RF Bandwidth gap as specified in clause 6.6.3.5.6.1, if applicable.

b)	Measure CACLR inside sub-block gap or Inter RF Bandwidth gap as specified in clause 6.6.3.5.6.2, if applicable.

3)	Repeat the test with the channel set-up according to E- TM1.2 in clause 4.12.2.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.3.5	Test requirements

##### 6.6.3.5.1	General Requirements

For the ACLR requirement either the ACLR/CACLR limits in clauses 6.6.3.5.3, 6.6.3.5.4, 6.6.3.5.5 and 6.6.3.5.6, or the absolute limit in clause 6.6.3.5.2 shall apply, whichever is less stringent.

Conformance to the relative ACLR/CACLR requirement may be shown to either the measure and sum test requirement or the per TAB connector test requirement.

1)	The relative ACLR/CACLR test requirements for an AAS BS when using the measure and sum alternative are that for each TAB connector TX cell group and each applicable limit, the ratio of the power summation of wanted at the TAB connectors of the TAB connector TX cell group to the power sum of the emissions at the TAB connectors of the TAB connector TX cell group shall not exceed the limit.

2)	The relative ACL/CACLR R test requirements for an AAS BS when using the per TAB connector alternative are that for each TAB connector TX cell group and each applicable limit, ratio of the wanted signal to the emissions at each of the TAB connectors of the TAB connector TX cell group shall not exceed the limit.

##### 6.6.3.5.2	Absolute Limits

The absolute limits apply for ACLR and CACLR.

Conformance may be shown to either the measure and sum test requirement or the per TAB connector test requirement.

1)	The ACLR/CACLR test requirements for an AAS BS when using the measure and sum alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this clause, the power summation of emissions at the TAB connectors of the TAB connector TX cell group shall not exceed the specified basic limit + 10log10(NTXU,countedpercell).

2)	The ACLR/CACLR test requirements for an AAS BS when using the per TAB connector alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this clause, the emissions at each of the TAB connectors of the TAB connector TX cell group shall not exceed the specified basic limit + 10log10(NTXU,countedpercell) - 10log10(n) where n is the number of TAB connectors in the TAB connector TX cell group.

The basic limit for the ACLR/CACLR absolute value is specified in table 6.6.3.5.2-1.

Table 6.6.3.5.2-1: Base station ACLR/CACLR absolute basic limit

| BS category / BS class | ACLR/CACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

##### 6.6.3.5.3	MSR

# 6.6.3.5.3.1	MSR E-UTRA test requirement

For E-UTRA, the test requirement is specified in tables 6.6.3.5.3.1-1 and 6.6.3.5.3.1-2, and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

For a TAB connector operating in non-contiguous spectrum, the ACLR also applies for the first adjacent channel inside any sub-blockgap with a gap size Wgap ≥ 15 MHz. The ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz. The CACLR test requirement in clause 6.6.3.5.6.2 applies in sub block gaps for the frequency ranges defined in table 6.6.3.5.6.2-1.

For a multi-band TAB connector, the ACLR also applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15 MHz. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz. The CACLR requirement in clause 6.6.3.5.6.2 applies in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.6.3.5.6.2-1.

The requirement applies during the transmitter on period.

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For operation in paired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.5.3.1-1.

Table 6.6.3.5.3.1-1: Base Station ACLR in paired spectrum

| Channel bandwidth of E-UTRA Lowest/ Highest Carrier transmitted BWChannel (MHz) | adjacent channel centre frequency offset below the lower or above the upper Base Station RF Bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
|  | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
| 1.4, 3.0, 5, 10, 15, 20 | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA Lowest/Highest Carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |

For operation in unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.5.3.1-2.

Table 6.6.3.5.3.1-2: Base Station ACLR in unpaired spectrum with synchronized operation

| Channel bandwidth of E-UTRA Lowest/ Highest Carrier transmitted BWChannel (MHz) | adjacent channel centre frequency offset below the lower or above the upper Base StationRF Bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
|  | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
| 1.4, 3 | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | 0.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 1.5 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
| 5, 10, 15, 20 | 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | 5 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44.2 dB |
|  | 15 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44.2 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA Lowest/Highest Carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.105 [3], with a chip rate as defined in this table. |  |  |  |  |

For operation in non-contiguous paired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.5.3.1-3.

Table 6.6.3.5.3.1-3: Base Station ACLR in non-contiguous paired spectrum

| Sub-block gap size (Wgap) where the limit applies | adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |

For operation in non-contiguous unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.5.3.1-4.

Table 6.6.3.5.3.1-4: Base Station ACLR in non-contiguous unpaired spectrum

| Sub-block gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44.2 dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44.2 dB |

# 6.6.3.5.3.2	MSR UTRA FDD test requirement

For UTRA FDD, the test requirement is specified in TS 25.141 [18], clause 6.5.2.2.5, and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

For a TAB connector operating in non-contiguous spectrum, ACLR requirement also applies for the first adjacent channel, inside any sub-block gap with a gap size Wgap ≥ 15 MHz. The ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz. The CACLR test requirement in clause 6.6.3.5.6.2 applies in sub block gaps for the frequency ranges defined in table 6.6.3.5.6.2-1.

For a multi-band TAB connector ACLR requirement also applies for the first adjacent channel, inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15 MHz. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz. The CACLR requirement in clause 6.6.3.5.6.2 applies in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.6.3.5.6.2-1.

# 6.6.3.5.3.3	MSR UTRA TDD test requirement

For UTRA TDD, the test requirement is specified in clause 6.6.3.5.5, and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

# 6.6.3.5.3.4	Cumulative ACLR requirement in non-contiguous spectrum

The following test requirement applies for sub-block or Inter RF Bandwidth gap sizes listed in table 6.6.3.5.3.4-1:

-	Inside a sub-block gap within an operating band for a BS operating in non-contiguous spectrum.

-	Inside an Inter RF Bandwidth gap for a BS operating in multiple bands, where multiple bands are mapped on the same antenna connector.

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or the Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap; and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The requirement applies to adjacent channels of E-UTRA or UTRA carriers allocated adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap. The assumed filter for the adjacent channel frequency is defined in table 6.6.3.5.3.4-1 and the filters on the assigned channels are defined in table 6.6.3.5.3.4-2.

NOTE:	If the RAT on the assigned channel frequencies is different, the filters used are also different.

The CACLR for E-UTRA and UTRA carriers located on either side of the sub-block gap or the Inter RF Bandwidth gap shall be higher than the value specified in table 6.6.3.5.3.4-1.

Table 6.6.3.5.3.4-1: CACLR in non-contiguous spectrum or multiple bands

| Band Category | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| BC1, BC2 | 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| BC1, BC2 | 10 MHz ≤ Wgap < 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| BC3 | 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44.2 dB |
| BC3 | 10 MHz < Wgap < 20 MHz | 7.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44.2 dB |
| NOTE: For BC1 and BC2 the RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |  |

Table 6.6.3.5.3.4-2: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| E-UTRA | E-UTRA of same BW |
| UTRA FDD | RRC (3.84 Mcps) |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |

##### 6.6.3.5.4	UTRA FDD

# 6.6.3.5.4.1	ACLR

The measurement result shall not be less than the ACLR limit specified in tables 6.6.3.5.4.1-1.

Table 6.6.3.5.4.1-1: BS ACLR

| BS channel offset below the first or above the last carrier frequency used | ACLR limit |
| --- | --- |
| 5 MHz | 44.2 dB |
| 10 MHz | 49.2 dB |
| NOTE 1: In certain regions, the adjacent channel power (the RRC filtered mean power centred on an adjacent channel frequency) shall be less than or equal to -7.2 dBm/3.84 MHz (for Band I, III, IX, XI and XXI) or +2.8 dBm/3.84 MHz (for Band VI, VIII and XIX) or as specified by the ACLR limit, whichever is the higher. This note is not applicable for Home BS. NOTE 2: For Home BS, the adjacent channel power (the RRC filtered mean power centred on an adjacent channel frequency) shall be less than or equal to -42.7 dBm/3.84 MHz f≤ 3.0 GHz and -42.4 dBm/3.84 MHz for 3.0 GHz < f ≤ 4.2 GHz or as specified by the ACLR limit, whichever is the higher. |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

The measurement result in shall not be less than the ACLR limit specified in table 6.6.3.5.4.1-2.

Table 6.6.3.5.4.1-2: ACLR in non-contiguous spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |

# 6.6.3.5.4.2	Cumulative ACLR test requirement in non-contiguous spectrum or multiple-bands

The following test requirement applies for a TAB connector operating in non-contiguous spectrum or multiple bands.

The following requirement applies for the gap sizes listed in table 6.6.3.5.4.2-1:

-	inside a sub-block gap within an operating band for a TAB connector operating in non-contiguous spectrum;

-	inside an Inter RF Bandwidth gap for a multi-band TAB connector.

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or Inter RF Bandwidth gap; and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in table 6.6.3.5.4.2-1and the filters on the assigned channels are defined in table 6.6.3.5.4.2-2.

The CACLR for UTRA carriers located on either side of the sub-block gap or Inter RF Bandwidth gap shall be higher than the value specified in table 6.6.3.5.4.2-1.

Table 6.6.3.5.4.2-1: Base Station CACLR in non-contiguous spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- |
| 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| 10 MHz < Wgap < 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |

Table 6.6.3.5.4.2-2: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| UTRA FDD | RRC (3.84 Mcps) |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2],with a chip rate as defined in this table. |  |

##### 6.6.3.5.5	UTRA TDD, 1,28Mcps option

The ACLR shall be equal or greater than the limits given in table 6.6.3.5.5-1. In case the equipment is tested against the requirements defined for operation in the same geographic area or co-sited with unsynchronised TDD or FDD on adjacent channels, the adjacent channel leakage power measured shall not exceed the maximum levels specified in tables 6.27B, 6.27C, 6.27D, 6.28B, 6.28C or 6.28D, respectively.

Table 6.6.3.5.5-1: BS ACLR Test Requirements (1,28 Mcps option)

| BS adjacent channel offset below the first or above the last carrier frequency used | ACLR limit |
| --- | --- |
| 1,6 MHz | 39.2 dB |
| 3,2 MHz | 44.2 dB |

The requirements shall apply outside the Base Station RF bandwidth or maximum radio bandwidth edges whatever the type of transmitter considered (single carrier, multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

For a multi-band TAB connector the ACLR requirement also applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size Wgap ≥4.8MHz. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥6.4 MHz.

The same test requirements apply to 1,28 Mcps TDD option BS supporting 16QAM.

##### 6.6.3.5.6	E-UTRA

# 6.6.3.5.6.1	ACLR

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For operation in paired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.5.6.1-1.

Table 6.6.3.5.6.1-1: ACLR in paired spectrum

| Channel bandwidth of E-UTRA lowest/highest carrier transmitted BWChannel (MHz) | adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
|  | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
| 1.4, 3.0, 5, 10, 15, 20 | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | BWChannel /2 + 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | BWChannel /2 + 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA lowest/highest/ carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.014 [3] with a chip rate as defined in this table. |  |  |  |  |

For operation in unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.5.6.1-2.

Table 6.6.3.5.6.1-2: ACLR in unpaired spectrum with synchronized operation

| Channel bandwidth of E-UTRA lowest/highest carrier transmitted BWChannel (MHz) | adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
|  | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
| 1.4, 3.0 | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | BWChannel /2 + 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | BWChannel /2 + 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | BWChannel /2 + 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
| 5, 10, 15, 20 | BWChannel /2 + 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | BWChannel /2 + 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | BWChannel /2 + 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | BWChannel /2 + 5 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44.2 dB |
|  | BWChannel /2 + 15 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44.2 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.014 [3] with a chip rate as defined in this table. |  |  |  |  |

For operation in non-contiguous paired spectrum or multiple bands, the ACLR shall be higher than the value specified in table 6.6.3.5.6.1-3.

Table 6.6.3.5.6.1-3: Base Station ACLR in non-contiguous paired spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |

For operation in non-contiguous unpaired spectrum or multiple bands, the ACLR shall be higher than the value specified in table 6.6.3.5.6.1-4.

Table 6.6.3.5.6.1-4: ACLR in non-contiguous unpaired spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| Wgap ≥ 15 MHz | 2.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44.2 dB |
| Wgap ≥ 20 MHz | 7.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44.2 dB |

# 6.6.3.5.6.2	Cumulative ACLR test requirement in non-contiguous spectrum

The following test requirement applies for the sub-block or Inter RF Bandwidth gap sizes listed in table 6.6.3.5.6.2-1,

-	Inside a sub-block gap within an operating band for a BS operating in non-contiguous spectrum.

-	Inside an Inter RF Bandwidth gap for a multi-band TAB connector.

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or Inter RF Bandwidth gap; and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in tables 6.6.3.5.6.2-1 and 6.6.3.5.6.2-2. Filters on the assigned channels are defined in table 6.6.3.5.6.2-3.

For operation in non-contiguous spectrum or multiple bands, the CACLR for E-UTRA carriers located on either side of the sub-block gap or Inter RF Bandwidth gap shall be higher than the value specified in tables 6.6.3.5.6.2-1 and 6.6.3.5.6.2-2.

Table 6.6.3.5.6.2-1: Base Station CACLR in non-contiguous paired spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- |
| 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| 10 MHz < Wgap < 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |

Table 6.6.3.5.6.2-2: Base Station CACLR in non-contiguous unpaired spectrum or multiple bands

| Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | BS adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- |
| 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 5 MHz E-UTRA carrier | Square (BWConfig) | 44.2 dB |
| 10 MHz < Wgap < 20 MHz | 7.5 MHz | 5 MHz E-UTRA carrier | Square (BWConfig) | 44.2 dB |

Table 6.6.3.5.6.2-3: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| E-UTRA | E-UTRA of same BW |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 6.6.4	Spectrum emission mask

#### 6.6.4.1	Definition and applicability

This requirement is applicable for single RAT UTRA AAS BS operation only.

#### 6.6.4.2	Minimum requirement

The minimum requirement for UTRA operation are defined in TS 37.105 [8], clause 6.6.4.3.

#### 6.6.4.3	Test purpose

This test measures the emissions of the TAB connector, close to the assigned channel bandwidth of the wanted signal, while the transmitter unit associated with the TAB connector under test is in operation.

#### 6.6.4.4	Method of test

##### 6.6.4.4.1	Initial conditions

# 6.6.4.4.1.1	General test conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

# 6.6.4.4.1.2	UTRA FDD

For a TAB connector declared to be capable of single carrier operation only, set to transmit a signal according to TM1, in clause 4.12.2.

For a multi-carrier TAB connector, set to transmit according to TM1 on all carriers configured using the applicable test configuration.

# 6.6.4.4.1.3	UTRA TDD

For a TAB connector declared to be capable of single carrier operation only, set the parameters of the transmitted signal according to table 6.6.4.4.1.3-1.

For a multi-carrier TAB connector set to transmit according to table 6.6.4.4.1.3-1 on all carriers configured using the applicable test configuration.

Table 6.6.4.4.1.3-1: Parameters of the transmitted signal
for spectrum emission mask testing for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 8 |
| Power of each DPCH | 1/8 of Base Station output power |
| Data content of DPCH | real life (sufficient irregular) |

In addition for a TAB connector declared capable of 16QAM.

For a TAB connector declared to be capable of single carrier operation only, set the parameters of the transmitted signal according to table 6.6.4.4.1.3-2.

For a multi-carrier TAB connector set to transmit according to table 6.6.4.4.1.3-2 on all carriers configured using the applicable test configuration.

Table 6.6.4.4.1.3-2: Parameters of the transmitted signal
for spectrum emission mask testing for 1,28 Mcps TDD - 16QAM capable TAB connector

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| HS-PDSCH modulation | 16QAM |
| Number of HS-PDSCH in each time slot under test | 8 |
| Power of each HS-PDSCH | 1/8 of Base Station output power |
| Data content of HS-PDSCH | real life (sufficient irregular) |
| Spreading factor | 16 |

##### 6.6.4.4.2	Procedure

# 6.6.4.4.2.1	General procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

a)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The measurement device characteristics shall be:

-	Measurements with an offset from the carrier centre frequency between 2,515 MHz and 4.0 MHz shall use a 30 kHz measurement bandwidth.

-	Measurements with an offset from the carrier centre frequency between 4.0 MHz and (f_offsetmax - 500 kHz).shall use a 1 MHz measurement bandwidth.

-	Detection mode: True RMS.

b)	For single carrier operation set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC)

For a TAB connector declared to be capable of multi-carrier and/or CA operation set the TAB connector to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 5 using the corresponding test models or set of physical channels in clause 4.12.

# 6.6.4.4.2.1	UTRA FDD

1)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth. For multi-band TAB connector or TAB connector operating in non-contiguous spectrum, the emission within the Inter RF Bandwidth or sub-block gap shall be measured using the specified measurement bandwidth from the closest Base Station RF Bandwidth or sub block edge.

In addition, for multi-band TAB connector(s), the following steps shall apply:

2)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 6.6.4.4.2.2	UTRA TDD

1)	Measure the power of the TAB connector spectrum emissions by applying measurement filters with bandwidths as specified in the relevant table in clause 6.6.4.5.2.2. The characteristic of the filters shall be approximately Gaussian (typical spectrum analyzer filters). The centre frequency of the filter shall be stepped in contiguous steps over the ranges of frequency offsets f_offset as given in the tables. The step width shall be equal to the respective measurement bandwidth. The time duration of each step shall be sufficiently long to capture one active time slot.

2)	The measurement shall be performed by applying filters with measurement bandwidth of 50 kHz or less and integrating the measured results over the nominal measurement bandwidth 1 MHz specified in the tables in clause 6.6.4.5.2.2 when the measurement bandwidth is 1 MHz.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

The same procedure applies to TAB connectors declared to support 16QAM.

#### 6.6.4.5	Test requirements

##### 6.6.4.5.1	General

Conformance may be shown to either the measure and sum test requirement or the per TAB connector test requirement.

1)	The spurious emission test requirements for an AAS BS when using the measure and sum alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this clause, the power summation of emissions at the TAB connectors of the TAB connector TX cell group shall not exceed a limit specified as the basic limit + 10log10(NTXU,countedpercell).

2)	The spurious emission test requirements for an AAS BS when using the per TAB connector alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this clause, the emissions at each of the TAB connectors of the TAB connector TX cell group shall not exceed a limit specified as the basic limit + 10log10(NTXU,countedpercell) - 10log(n) where n is the number of TAB connectors in the TAB connector TX cell group.

The appropriate table for the basic limit is based on the same power level (PRated,c,sys) as used for the AAS BS rated power limits for BS classes in table 6.2.2.1-1 the same method of scaling the power level using NTXU,counted is used.

##### 6.6.4.5.2	Basic Limits

# 6.6.4.5.2.1	UTRA FDD

The basic limit is specified in tables 6.6.4.5.2.1-1 to 6.6.4.5.2.1-11 for the appropriate PRated,c,sys, where:

-	f is the separation between the carrier frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the carrier frequency and the centre of the measurement filter;

-	f_offsetmax is either 12.5 MHz or the offset to the UMTS Tx band edge as defined in clause 3.4.1, whichever is the greater.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Inside any Inter RF Bandwidth gap s with Wgap < 20 MHz for BS operating in multiple bands, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The basic limit for Base Station RF Bandwidth edge is specified in tables 6.6.4.5.2.1-1 to 6.6.4.5.2.1-11 below, where in this case:

-	f is equal to 2.5 MHz plus the separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the Base Station RF Bandwidth edge.

-	f_offset is equal to 2.5 MHz plus the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is either 12.5 MHz or the offset to the UMTS Tx band edge as defined in clause 5.2, whichever is the greater.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, the operating band unwanted emission limits apply also in a supported operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported operating band. In this case, no cumulative limit is applied in the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted and

-	In case the inter-band gap between a downlink band with carrier(s) transmitted and a downlink band without any carrier transmitted is less than 20 MHz, f_offsetmax shall be the offset to the frequency 10 MHz outside the outermost edges of the two downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present clause, shall apply across both downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (fmax), shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the downlink operating band without any carrier transmitted.

Inside any sub-block gap for a TAB connector operating in non-contiguous spectrum, the measurement results shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The basic limit for each sub block is specified in tables 6.6.4.5.2.1-1 to 6.6.4.5.2.1-11 below, where in this case:

-	f is equal to 2.5 MHz plus the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is equal to 2.5 MHz plus the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter plus 2.5 MHz.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.6.4.5.2.1-1: Spectrum emission mask values, Pmax,c,cell-10*log10(NTXU,countedpercell)  43 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -12.5 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz |  | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -24.5 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -11.5 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -11.5 dBm | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clauses 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-2: Spectrum emission mask values, Pmax,c,cell-10*log10(NTXU,countedpercell)  43 dBm UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -12.2 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz |  | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -24.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -11.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -11.2 dBm | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clause 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-3: Spectrum emission mask values,
39 dBm  Pmax,c,cell-10*log10(NTXU,countedpercell) < 43 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -12.5 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz |  | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -24.5 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -11.5 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 54.5 dB | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clauses 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-4: Spectrum emission mask values,
39 dBm  Pmax,c,cell-10*log10(NTXU,countedpercell) < 43 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -12.2 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz |  | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -24.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -11.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 54.2 dB | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clause 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-5: Spectrum emission mask values, 31 dBm  Pmax,c,cell-10*log10(NTXU,countedpercell) < 39 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point,f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 51.5 dB | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz |  | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 63.5 dB | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 50.5 dB | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 54.5 dB | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clauses 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-6: Spectrum emission mask values,
31 dBm  Pmax,c,cell-10*log10(NTXU,countedpercell) < 39 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point,f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 51.2 dB | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz |  | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 63.2 dB | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 50.2 dB | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 54.2 dB | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clauses 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-7: Spectrum emission mask values, Pmax,c,cell-10*log10(NTXU,countedpercell) < 31 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -20.5 dBm | 30 kHz |
| 2.7 f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz |  | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -32.5 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -19.5 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -23.5 dBm | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clauses 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-8: Spectrum emission mask values, Pmax,c,cell-10*log10(NTXU,countedpercell) < 31 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (notes 1 and 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515 MHz f_offset < 2.715 MHz | -20.2 dBm | 30 kHz |
| 2.7 f < 3.5 MHz | 2.715 MHz f_offset < 3.515 MHz |  | 30 kHz |
| (Note 4) | 3.515 MHz f_offset < 4.0 MHz | -32.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0 MHz | -19.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -23.2 dBm | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5 MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clauses 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

For operation in band II, IV, V, X, XII, XIII, XIV, XXV and XXVI, the applicable additional requirement in tables 6.6.4.5.2.1-9 to 6.6.4.5.2.1-11 apply in addition to the minimum requirements in tables 6.6.4.5.2.1-1 to 6.6.4.5.2.1-8.

Table 6.6.4.5.2.1-9: Additional spectrum emission limits for Bands II, IV, X, XXV

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 3.5 MHz | 2.515 MHz f_offset < 3.515 MHz | -15 dBm | 30 kHz |
| 3.5 MHz f fmax | 4.0 MHz f_offset < f_offsetmax | -13 dBm | 1 MHz |
| NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-10: Additional spectrum emission limits for Bands V, XXVI

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 3.5 MHz | 2.515 MHz f_offset < 3.515 MHz | -15 dBm | 30 kHz |
| 3.5 MHz f fmax | 3.55 MHz f_offset < f_offsetmax | -13 dBm | 100 kHz |
| NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

Table 6.6.4.5.2.1-11: Additional spectrum emission limits for Bands XII, XIII,  XIV

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.6 MHz | 2.515 MHz f_offset < 2.615 MHz | -13 dBm | 30 kHz |
| 2.6 MHz f fmax | 2.65 MHz f_offset < f_offsetmax | -13 dBm | 100 kHz |
| NOTE 5: As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |  |

In certain regions the following requirement may apply for protection of DTT. For a TAB connector operating in Band XX, the level of emissions in the band 470-790 MHz, measured in an 8MHz filter bandwidth on centre frequencies Ffilter according to table 6.21F, shall not exceed the  maximum emission level PEM,N declared by the manufacturer.

Table 6.6.4.5.2.1-12: Declared emissions levels for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission level (dBm) |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

NOTE 1:	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the basestation needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in annex D of [1].

In certain regions, the following requirements may apply to a TAB connector operating in Band XXXII within 1452-1492 MHz. The level of unwanted emissions, measured on centre frequencies f_offset with filter bandwidth, according to table 6.6.4.5.2.1-13, shall neither exceed the maximum emission level PEM,B32,a ,  PEM,B32,b nor PEM,B32,c declared by the manufacturer.

Table 6.6.4.5.2.1-13: Declared frequency band XXXII unwanted emission
within 1452-1492 MHz

| Frequency offset of measurement filter centre frequency, f_offset | Declared emission level (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 5 MHz | PEM,B32,a | 5 MHz |
| 10 MHz | PEM,B32,b | 5 MHz |
| 15 MHz ≤ f_offset ≤ f_offsetmax, B32 | PEM,B32,c | 5 MHz |
| NOTE: f_offsetmax, B32 denotes the frequency difference between the lower channel carrier frequency and 1454.5 MHz, and the frequency difference between the upper channel carrier frequency and 1489.5 MHz for the set channel position. |  |  |

NOTE 2:	The regional requirement, included in [17], is defined in terms of EIRP per antenna, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in annex H of TS 36.104 [11].

In certain regions, the following requirement may apply to TAB connector operating in Band XXXII within 1452-1492MHz for the protection of services in spectrum adjacent to the frequency range 1452-1492 MHz. The level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to table 6.6.4.5.2.1-14, shall neither exceed the maximum emission level PEM,B32,d nor PEM,B32,e declared by the manufacturer. This requirement applies in the frequency range 1429-1518MHz even though part of the range falls in the spurious domain.

Table 6.6.4.5.2.1-14: Frequency band XXXII declared emission outside 1452-1492 MHz

| Filter centre frequency, Ffilter | Declared emission level (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 1429.5 MHz ≤ Ffilter ≤ 1448.5 MHz | PEM,B32,d | 1 MHz |
| Ffilter = 1450.5 MHz | PEM,B32,e | 3 MHz |
| Ffilter = 1493.5 MHz | PEM,B32,e | 3 MHz |
| 1495.5 MHz ≤ Ffilter ≤ 1517.5 MHz | PEM,B32,d | 1 MHz |

NOTE 3:	The regional requirement, included in [17], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in annex H of TS 36.104 [11].

# 6.6.4.5.2.2	UTRA TDD

The basic limit is specified in tables 6.6.4.5.2.2-1 to 6.6.4.5.2.2-3 for the appropriate PRated,c,sy, where:

Table 6.6.4.5.2.2-1: basic limits for spectrum emission mask values,
Pmax,c,cell-10*log10(NTXU,countedpercell)  34 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | basic limit | Measurement bandwidth |
| --- | --- | --- |
| 0.815 MHz f_offset < 1.015 MHz | -18.5 dBm | 30 kHz |
| 1.015 MHz f_offset < 1.815 MHz |  | 30 kHz |
| 1.815 MHz f_offset < 2.3 MHz | -26.5 dBm | 30 kHz |
| 2.3 MHz f_offset < f_offsetmax | -11.5 dBm | 1 MHz |
| NOTE: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap. |  |  |

Table 6.6.4.5.2.2-2: basic limits for spectrum emission mask values,
26 dBm  Pmax,c,cell-10*log10(NTXU,countedpercell) < 34 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | basic limit | Measurement bandwidth |
| --- | --- | --- |
| 0.815 MHz f_offset < 1.015 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) -52.5 dB | 30 kHz |
| 1.015 MHz f_offset < 1.815 MHz |  | 30 kHz |
| 1.815 MHz f_offset < 2.3 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) -60.5 dB | 30 kHz |
| 2.3 MHz f_offset < f_offsetmax | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 45.5 dB | 1 MHz |
| NOTE: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap. |  |  |

Table 6.6.4.5.2.2-3: basic limits for spectrum emission mask values,
Pmax,c,cell-10*log10(NTXU,countedpercell) < 26 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measurement bandwidth |
| --- | --- | --- |
| 0.815 MHz f_offset < 1.015 MHz | -26.5 dBm | 30 kHz |
| 1.015 MHz f_offset < 1.815 MHz |  | 30 kHz |
| 1.815 MHz f_offset < 2.3 MHz | -34.5 dBm | 30 kHz |
| 2.3 MHz f_offset < f_offsetmax | -19.5 dBm | 1 MHz |
| NOTE: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap. |  |  |

The basic limits for a TAB connector declared capable of 16QAM are the same as those defined in the clause.

### 6.6.5	Operating band unwanted emission

#### 6.6.5.1	Definition and applicability

Unless otherwise stated, for E-UTRA single band and MSR the operating band unwanted emission limits are defined from 10 MHz below the lowest frequency of each supported downlink operating band to the lower Base Station RF Bandwidth edge located at FBW RF,low and from the upper Base Station RF Bandwidth edge located at FBW RF,high  up to 10 MHz above the highest frequency of each supported downlink operating band.

For AAS BS capable of operation in multiple operating bands, using single band TAB connectors, the single-band requirements apply to those connectors and the cumulative evaluation of the emission limit in the inter  RF bandwidth gap is not applicable.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification.

#### 6.6.5.2	Minimum requirement

The minimum requirement for MSR operation are defined in TS 37.105 [8], clause 6.6.5.2.

There is no Operating band unwanted emission requirement for UTRA operation.

The minimum requirement for E-UTRA operation is in TS 37.105 [8], clause 6.6.5.4.

#### 6.6.5.3	Test purpose

This test measures the emissions of the TAB connector, close to the assigned channel bandwidth of the wanted signal, while the transmitter is in operation.

#### 6.6.5.4	Method of test

##### 6.6.5.4.1	Initial conditions

# 6.6.5.4.1.1	General test conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

# 6.6.5.4.1.2	UTRA FDD

# 6.6.5.4.1.3	E-UTRA

##### 6.6.5.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.1. All TAB connectors not under test shall be terminated.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The measurement device characteristics shall be:

-	Detection mode: True RMS.

2)	Set the set the TAB connector to transmit:

a)	For MSR:

-	Set the TAB connector to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.12.

b)	For E-UTRA:

-	TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to E-TM1.1 (clause 4.12.2) at manufacturer's declared rated output power Prated,c,TABC..

-	For a TAB connector declared to be capable of multi-carrier and/or CA operation, set the set the TAB connector to transmit according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

3)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth.

4)	Repeat the test for the remaining test cases:

a)	For MSR with channel set-up according to clause 5 and clause 4.12.2.

b)	For E-UTRA with the channel set-up according to E-TM 1.2

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.5.5	Test requirements

##### 6.6.5.5.1	General

Conformance may be shown to either the measure and sum test requirement or the per TAB connector test requirement.

1)	The spurious emission test requirements for an AAS BS when using the measure and sum alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this clause, the power summation of emissions at the TAB connectors of the TAB connector TX cell group shall not exceed a limit specified as the basic limit + 10log10(NTXU,countedpercell).

2)	The spurious emission test requirements for an AAS BS when using the per TAB connector alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this clause, the emissions at each of the TAB connectors of the TAB connector TX cell group shall not exceed a limit specified as the basic limit + 10log10(NTXU,countedpercell) - 10log(n) where n is the number of TAB connectors in the TAB connector TX cell group.

The appropriate table for the basic limit is based on the same power level (PRated,c,sys) as used for the AAS BS rated power limits for BS classes in table 6.2.2.1-1 the same method of scaling the power level using NTXU,counted is used.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.5.5.2	Basic Limits for MSR Band Categories 1 and 3

For an AAS BS of Wide Area BS class operating in Band Category 1 or Band Category 3, the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Wide Area BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Wide Area BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

For an AAS BS of Medium Range BS class operating in Band Category 1 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Medium Range BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Medium Range BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

For an AAS BS of Local Area BS class operating in Band Category 1 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Local Area BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Local Area BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

Outside the Base Station RF Bandwidth edges, emissions basic limits are specified in tables 6.6.2.5.1-1 to 6.6.2.5.1-4 below, where:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10 MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, inside any Inter RF Bandwidth gaps with Wgap < 20 MHz, emissions a combined basic limit shall be applied which is the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The basic limit for Base Station RF Bandwidth edge is specified in tables 6.6.5.5.2-1 to 6.6.5.5.2-8, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in another operating band. In this case where there is no carrier transmitted in an operating band, no cumulative basic limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted and

-	In case the Inter RF Bandwidth gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted is less than 20MHz, f_offsetmax shall be the offset to the frequency 10 MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present clause, shall apply across both supported downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (fmax), shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

Inside any sub-block gap for a TAB connector operating in non-contiguous spectrum, emissions a combined basic limit shall be applied which is the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The basic limit for each sub block is specified in tables 6.6.5.5.2-1 to 6.6.5.5.2-8, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge frequency.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.6.5.5.2-1: Wide Area BS operating band unwanted emission mask (UEM)
for BC1 and BC3 bands ≤ 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015 MHz f_offset < 0.215 MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -11.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 5) | 1 MHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/MHz. NOTE 2: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.2-2: Wide Area BS operating band unwanted emission mask (UEM)
for BC1 and BC3 for bands > 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015 MHz f_offset < 0.215 MHz | -12.2 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | -24.2 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -11.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 5) | 1 MHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/MHz. NOTE 2: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.2-3: Medium Range BS operating band unwanted emission mask (UEM)
for BC1 for bands ≤ 3 GHz, 31 < Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 63.5 dB | 30 kHz |
| 1 MHz f 2.6 MHz | 1.5 MHz f_offset < 3.1 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 50.5 dB | 1 MHz |
| 2.6 MHz f 5 MHz | 3.1 MHz f_offset < 5.5 MHz | min(Pmax,c,cell - 10*log10(NTXU,countedpercell) - 50.5 dB, -13.5 dBm) | 1 MHz |
| 5 MHz f min(fmax, 10 MHz) | 5.5 MHz f_offset < min (f_offsetmax, 10.5 MHz) | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 54.5 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Pmax,c,cell - 10*log10(NTXU,countedpercell) -56 dB (Note 5) | 1 MHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be (Pmax,c,cell-10*log10(NTXU,countedpercell) - 56 dB)/MHz. NOTE 2: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.2-4: Medium Range BS operating band unwanted emission mask (UEM)
for BC1 for bands > 3 GHz, 31 < Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 63.2 dB | 30 kHz |
| 1 MHz f 2.6 MHz | 1.5 MHz f_offset < 3.1 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 50.2 dB | 1 MHz |
| 2.6 MHz f 5 MHz | 3.1 MHz f_offset < 5.5 MHz | min(Pmax,c,cell - 10*log10(NTXU,countedpercell) - 50.2 dB, -13.2dBm) | 1 MHz |
| 5 MHz f min(fmax, 10 MHz) | 5.5 MHz f_offset < min(f_offsetmax ,10.5 MHz) | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 54.2 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Pmax,c,cell - 10*log10(NTXU,countedpercell) -56 dB (Note 5) | 1 MHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be (Pmax,c,cell-10*log10(NTXU,countedpercell) - 56 dB)/MHz. NOTE 2: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.2-5: Medium Range BS operating band unwanted emission mask (UEM)
for BC1 for bands≤ 3 GHz, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | -32.5 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -19.5 dBm | 1 MHz |
| 5 MHz f min(fmax,10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5 MHz) | -23.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -25 dBm (Note 5) | 1 MHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -25 dBm/MHz. NOTE 2: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.2-6: Medium Range BS operating band unwanted emission mask (UEM)
for BC1 for bands> 3 GHz, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 3) | 1.015 MHz f_offset < 1.5 MHz | -32.2 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -19.2 dBm | 1 MHz |
| 5 MHz f min(fmax,10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5 MHz) | -23.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -25 dBm (Note 5) | 1 MHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -25 dBm/MHz. NOTE 2: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.2-7: Local Area operating band unwanted emission mask (UEM)
for BC1 for bands  3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -37 dBm/100 kHz. NOTE 2: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.2-8: Local Area operating band unwanted emission mask (UEM)
for BC1 for bands > 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -37 dBm/100 kHz. NOTE 2: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 3: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

##### 6.6.5.5.3	Basic Limits for MSR Band Category 2

For a TAB connector operating in Band Category 2 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a TAB connector operating in non-contiguous spectrum, it applies inside any sub-block gap.

Outside the Base Station RF Bandwidth edges, basic limits are specified in tables 6.6.5.5.3-1 to 6.6.5.5.3-8, where:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10 MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, inside any Inter RF Bandwidth gaps with Wgap < 20 MHz, emissions a combined basic limit shall be applied which is the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The basic limit for Base Station RF Bandwidth edge is specified in tables 6.6.5.5.3-1 to 6.6.5.5.3-8, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector and where there is no carrier transmitted in an operating band, no cumulative basic limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted and

-	In case the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted less than is 20MHz, f_offsetmax shall be the offset to the frequency 10 MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present clause, shall apply across both supported downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (fmax), shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

Inside any sub-block gap for a TAB connector operating in non-contiguous spectrum, emissions a combined basic limit shall be applied which is the cumulative sum of the test requirement specified for the adjacent sub blocks on each side of the sub block gap. The basic limit for each sub block is specified in tables 6.6.5.5.3-1 to 6.6.5.5.3-8, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.6.5.5.3-1: Wide Area BS operating band unwanted emission mask (UEM) for BC2

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz (Note 1) | 0.015 MHz f_offset < 0.215 MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 8) | 1.015 MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -11.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 10) | 1 MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.6.5.5.3-2 apply for 0 MHz f < 0.15 MHz. NOTE 2: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/MHz. NOTE 3: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz operation the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.6.5.5.3-2: Wide Area BS operating band unwanted emission limits for operation in BC2
with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Note 2, 3 and 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| NOTE 1: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 3: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 4: In case the carrier adjacent to the Base Station RF Bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier - 43, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the Base Station RF Bandwidth edge. In other cases, X = 0. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.6.5.5.3-3: Medium Range BS operating band unwanted emission mask (UEM)
for BC2, 31 < Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz (Note 1) | 0.015 MHz f_offset < 0.615 MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 8) | 1.015 MHz f_offset < 1.5 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 63.5 dB | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 50.5 dB | 1 MHz |
| 2.8 MHz f 5 MHz | 3.3 MHz f_offset < 5.5 MHz | min(Pmax,c,cell - 10*log10(NTXU,countedpercell) - 50.5 dB, -13.5 dBm) | 1 MHz |
| 5 MHz f min(fmax, 10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5 MHz) | Pmax,c,cell - 10*log10(NTXU,countedpercell) - 54.5 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Pmax,c,cell - 10*log10(NTXU,countedpercell) -56 dB (Note 10) | 1 MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.6.5.3-5 apply for 0 MHz f < 0.15 MHz. NOTE 2: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be (Pmax,c,cell-10*log10(NTXU,countedpercell) - 56 dB)/MHz. NOTE 3: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.6.5.5.3-4: Medium Range BS operating band unwanted emission mask (UEM)
for BC2, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz (Note 1) | 0.015 MHz f_offset < 0.615 MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 8) | 1.015 MHz f_offset < 1.5 MHz | -32.5 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -19.5 dBm | 1 MHz |
| 5 MHz f min(fmax,10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5 MHz) | -23.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -25 dBm (Note 10) | 1 MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.6.5.5.3-6 apply for 0 MHz f < 0.15 MHz. NOTE 2: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -25 dBm/MHz. NOTE 3: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.6.5.5.3-5: Medium Range operating band unwanted emission limits for operation in BC2
with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge,
31 < Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| NOTE 1: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 3: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.6.5.5.3-6: Medium Range operating band unwanted emission limits for operation in BC2
with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge,
Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 2, 3 and 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| NOTE 1: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 3: For multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 4: In case the carrier adjacent to the Base Station RF Bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier - 31, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the Base Station RF Bandwidth edge. In other cases, X = 0. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.6.5.5.3-7: Local Area operating band unwanted emission mask (UEM) for BC2

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 2 and 3) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz (Note 1) | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min (10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 7) | 100 kHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.6.5.5.3-8 apply for 0 MHz f < 0.16 MHz. NOTE 2: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -37 dBm/100 kHz. NOTE 3: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

Table 6.6.5.5.3-8: Local Area operating band unwanted emission limits for operation in BC2 with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the Base Station RF Bandwidth edge

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 2, 3 and 4) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.16 MHz | 0.065 MHz f_offset < 0.175 MHz |  | 30 kHz |
| NOTE 1: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 3: For MSR multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the Inter RF Bandwidth gap. NOTE 4: In case the carrier adjacent to the Base Station RF Bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier - 24, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the Base Station RF Bandwidth edge. In other cases, X = 0. NOTE 8: This frequency range ensures that the range of values of f_offset is continuous. NOTE 10: The requirement is not applicable when fmax < 10 MHz |  |  |  |

##### 6.6.5.5.4	Basic Limits for MSR Additional requirements

# 6.6.5.5.4.1	Limits in FCC Title 47

In addition to the requirements in clauses 6.6.5.5.2 and 6.6.5.5.3, the TAB connector may have to comply with the applicable basic limits established by FCC Title 47 [24], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

# 6.6.5.5.4.2	Unsynchronized operation for BC3

In certain regions, the following requirements may apply to a TDD TAB connector operating in BC3 in the same geographic area and in the same operating band as another TDD system without synchronisation. For this case the emissions basic limit is -52 dBm/MHz in each supported downlink operating band except in:

-	The frequency range from 10 MHz below the lower Base Station RF Bandwidth edge to the frequency 10 MHz above the upper Base Station RF Bandwidth edge of each supported band.

NOTE 1:	Local or regional regulations may specify another excluded frequency range, which may include frequencies where synchronised TDD systems operate.

NOTE 2:	TDD Base Stations that are synchronized and operating in BC3 can transmit without these additional co-existence requirements.

# 6.6.5.5.4.3	Protection of DTT

In certain regions the following requirement may apply for protection of DTT. For a TAB connector operating in Ban  20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to table 6.6.5.5.4.3-1, a basic limit PEM,N is declared by the manufacturer. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.6.5.5.4.3-1: Declared emissions basic limit for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission basic limit (dBm) |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

NOTE:	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the TAB connector and radiated in the far field. The requirement defined above provides the characteristics of the AAS BS  needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in annex G of TS 36.104 [11].

# 6.6.5.5.4.4	Co-existence with services in adjacent frequency bands

This requirement may be applied for the protection of systems operating in frequency bands adjacent to Band 1 as defined in clause 4.5, in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed.

The basic limit shall be.

Table 6.6.5.5.4.4-1: Emissions basic limits for protection of adjacent band services

| Operating Band | Frequency range | Basic limit | Measurement Bandwidth |
| --- | --- | --- | --- |
| 1 | 2100-2105 MHz | -30 + 3.4 × (f - 2100 MHz) dBm | 1 MHz |
|  | 2175-2180 MHz | -30 + 3.4 × (2180 MHz - f) dBm | 1 MHz |

# 6.6.5.5.4.5	Additional requirements for band 41

The following requirement may apply to a TAB connector operating in Band 41 in certain regions. Basic limits are specified in table 6.6.5.5.4.5-1, where:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

Table 6.6.5.5.4.5-1: Additional operating band unwanted emission basic limits Band 41

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 10 MHz | 10 MHz f < 20 MHz | 10.5 MHz f_offset < 19.5 MHz | -22 dBm | 1 MHz |
| 20 MHz | 20 MHz f < 40 MHz | 20.5 MHz f_offset < 39.5 MHz | -22 dBm | 1 MHz |
| NOTE: This requirement applies for E-UTRA carriers allocated within 2545-2575 MHz or 2595-2645 MHz. |  |  |  |  |

6.6.5.5.4.6	Additional band 32 unwanted emissions

In certain regions, the following requirements may apply to a TAB connector operating in Band 32 within 1452-1492 MHz. The maximum level of operating band unwanted emissions, measured on centre frequencies f_offset with filter bandwidth, according to table 6.6.5.5.4.6-1, shall be defined according to the basic limits PEM,B32,a ,  PEM,B32,b and PEM,B32,c declared by the manufacturer.

Table 6.6.5.5.4.6-1: Declared operating band 32 unwanted emission within 1452-1492 MHz

| Frequency offset of measurement filter centre frequency, f_offset | Declared emission basic limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2.5 MHz | PEM,B32,a | 5 MHz |
| 7.5 MHz | PEM,B32,b | 5 MHz |
| 12.5 MHz ≤ f_offset ≤ f_offsetmax, B32 | PEM,B32,c | 5 MHz |
| NOTE: f_offsetmax, B32 denotes the frequency difference between the lower Base Station RF Bandwidth edge and 1454.5 MHz, and the frequency difference between the upper Base Station RF Bandwidth edge and 1489.5 MHz for the set channel position. |  |  |

NOTE:	The regional requirement, included in [25], is defined in terms of EIRP per antenna, which is dependent on both the BS emissions at the TAB connector and radiated in the far field. The requirement defined above provides the characteristics of the AAS BS needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in annex H of TS 36.104 [11].

In certain regions, the following requirement may apply to a TAB connector operating in Band 32 within 1452-1492MHz for the protection of services in spectrum adjacent to the frequency range 1452-1492 MHz. The maximum level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to table 6.6.5.5.4.6-2, shall be defined according to the basic limits PEM,B32,d and PEM,B32,e declared by the manufacturer. This requirement applies in the frequency range 1429-1518MHz even though part of the range falls in the spurious domain.

Table 6.6.5.5.4.6-2: Operating band 32 declared emission outside 1452-1492 MHz

| Filter centre frequency, Ffilter | Declared emission basic limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 1429.5 MHz ≤ Ffilter ≤ 1448.5 MHz | PEM,B32,d | 1 MHz |
| Ffilter = 1450.5 MHz | PEM,B32,e | 3 MHz |
| Ffilter = 1493.5 MHz | PEM,B32,e | 3 MHz |
| 1495.5 MHz ≤ Ffilter ≤ 1517.5 MHz | PEM,B32,d | 1 MHz |

NOTE:	The regional requirement, included in [23], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and radiated in the far field. The requirement defined above provides the characteristics of the AAS BS needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in annex H of TS 36.104 [11].

6.6.5.5.4.7	Additional requirements for Band 48

The following requirement may apply to AAS BS operating in Band 48 in certain regions. Emissions shall not exceed the maximum levels specified in Table 6.6.2.5.4.7-1.

Table 6.6.2.5.4.7-1: Additional operating band unwanted emission limits for Band 48

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| All | 0 MHz f < 10 MHz | 0.5 MHz f_offset < 9.5 MHz | -13 dBm | 1 MHz |

##### 6.6.5.5.5	Basic Limits for E-UTRA

# 6.6.5.5.5.1	General

The measurement results in clause 6.6.5.4 shall not exceed the maximum levels based on the basic limits in the tables below, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10 MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, inside any Inter RF Bandwidth gaps with Wgap < 20 MHz, a combined basic limit shall be applied which is the cumulative sum the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The basic limit for Base Station RF Bandwidth edge is specified in tables 6.6.5.5.5.2-1 to 6.6.5.5.5.2-9, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the Base Station RF Bandwidth edge.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For multi-band TAB connector, the operating band unwanted emission limits apply also in a supported operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported operating band. In this case, no cumulative basic limit is applied in the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted and:

-	In case the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted is less than 20 MHz, f_offsetmax shall be the offset to the frequency 10 MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present clause, shall apply across both downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (fmax), shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

For a multicarrier E-UTRA TAB connector or a TAB connector configured for intra-band contiguous or non-contiguous carrier aggregation the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the upper edge of the carrier transmitted at the highest carrier frequency within a specified frequency band.

In addition inside any sub-block gap for a TAB connector operating in non-contiguous spectrum, measurement results shall not exceed a level based on a combined basic limit shall be applied which is the cumulative sum specified for the adjacent sub blocks on each side of the sub block gap. The basic limit for each sub block is specified in tables 6.6.5.5.5.2-1 to 6.6.5.5.5.2-9, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

# 6.6.5.5.5.2	Basic Limits for Wide Area BS (Category A)

For E-UTRA TAB connector operating in Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 26, 27, 28, 29, 31, 44, 68 basic limits are specified in tables 6.6.5.5.5.2-1 to 6.6.5.5.5.2-3.

Table 6.6.5.5.5.2-1: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.2-2: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.2-3: Wide Area BS operating band unwanted emission limits for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For E-UTRA TAB connector operating in Bands 1, 2, 3, 4, 7, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 45, 48, 65, 66, 69, 70, basic limits are specified in tables 6.6.5.5.5.2-4, 6.6.5.5.5.2-6 and 6.6.5.5.5.2-8.

For E-UTRA TAB connector operating in Bands 22, 42, 43, basic limits are specified in tables 6.6.5.5.5.2-5, 6.6.5.5.5.2-7 and 6.6.5.5.5.2-9.

Table 6.6.5.5.5.2-4: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz f fmax | 3.3 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.2-5: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -9.2 dBm | 100 kHz |
| 2.8 MHz f fmax | 3.3 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.2-6: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz f fmax | 6.5 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.2-7: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -13.2 dBm | 100 kHz |
| 6 MHz f fmax | 6.5 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.2-8: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.2-9: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

# 6.6.5.5.5.3	Basic limits for Wide Area BS Category B (Option1)

For Category B Operating band unwanted emissions, there are two options for the limits that may be applied regionally, option 1 is as follows.  For E-UTRA TAB connector operating in Bands 5, 8, 12, 13, 14, 17, 20, 26, 27, 28, 29, 31, 44, 67, 68 basic limits are specified in tables 6.6.5.5.5.3-1 to 6.6.5.5.5.3-3.

Table 6.6.5.5.5.3-1: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -16 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -16 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.3-2: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | -16 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -16 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

Table 6.6.5.5.5.3-3: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands < 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -16 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For E-UTRA TAB connector operating in Bands 1, 2, 3, 4, 7, 10, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 45, 48, 65, 66, 69, 70, basic limits are specified in tables 6.6.5.5.5.3-4, 6.6.5.5.5.3-6 and 6.6.5.5.5.3-8.

For E-UTRA TAB connector operating in Bands 22, 42, 43, basic limits are specified in tables 6.6.5.5.5.3-5, 6.6.5.5.5.3-7 and 6.6.5.5.5.3-9.

Table 6.6.5.5.5.3-4: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz f fmax | 3.3 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.3-5: Wide Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -9.2 dBm | 100 kHz |
| 2.8 MHz f fmax | 3.3 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.3-6: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz f fmax | 6.5 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.3-7: Wide Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -13.2 dBm | 100 kHz |
| 6 MHz f fmax | 6.5 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.3-8: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (1GHz < E-UTRA bands ≤ 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.3-9: Wide Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands > 3 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

# 6.6.5.5.5.4	Basic limits for Wide Area BS Category B (Option 2)

For Category B Operating band unwanted emissions, there are two options for the limits that may be applied regionally, option 2 is as follows.

The limits in this clause are intended for Europe and may be applied regionally for a TAB connector operating in band 1, 3, 8, 32, 33, 34 or 65.

For a TAB connector operating in band 1, 3, 8, 32, 33, 34 or 65, basic limits are specified in table 6.6.5.5.5.4-1 below for 5, 10, 15 and 20 MHz channel bandwidth.

Table 6.6.5.5.5.4-1: Regional Wide Area BS operating band unwanted emission limits
in band 1, 3, 8, 32, 33, 34 or 65 for 5, 10, 15 and 20 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015 MHz f_offset < 0.215 MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 7) | 1.015 MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f min( 10 MHz , fmax) | 1.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -11.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 7: This frequency range ensures that the range of values of f_offset is continuous. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For a TAB connector operating in band 3, 8 or 65, basic limits are specified in table 6.6.5.5.5.4-2 for 3 MHz channel bandwidth.

Table 6.6.5.5.5.4-2: Regional Wide Area BS operating band unwanted emission limits
in band 3, 8 or 65 for 3 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0. 065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.2 MHz | 0.165 MHz f_offset < 0.215 MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 7) | 1.015 MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f 6 MHz | 1.5 MHz f_offset < 6.5 MHz, | -11.5 dBm | 1 MHz |
| 6 MHz f fmax | 6.5 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 7: This frequency range ensures that the range of values of f_offset is continuous. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For a BS operating in band 3, 8 or 65, basic limits are specified in table 6.6.5.5.5.4-3 for 1.4 MHz channel bandwidth.

Table 6.6.5.5.5.4-3: Regional Wide Area BS operating band unwanted emission limits
in band 3, 8 or 65 for 1.4 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0. 065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.2 MHz | 0.165 MHz f_offset < 0.215 MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 7) | 1.015 MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | -11.5 dBm | 1 MHz |
| 2.8 MHz f fmax | 3.3 MHz f_offset < f_offsetmax | -15 dBm (Note 8) | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. NOTE 7: This frequency range ensures that the range of values of f_offset is continuous. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

# 6.6.5.5.5.5	Basic limits for Local Area BS (Category A and B)

For an AAS BS of Local Area BS class in E-UTRA bands ≤ 3 GHz, basic limits are specified in tables 6.6.5.5.5.5-1, 6.6.5.5.5.5-3 and 6.6.5.5.5.5-5.

For an AAS BS of Local Area BS class in E-UTRA bands > 3 GHz, basic limits are specified in tables 6.6.5.5.5.5-2, 6.6.5.5.5.5-4 and 6.6.5.5.5.5-6.

Table 6.6.5.5.5.5-1: Local Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -29.5 dBm | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -31 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -31 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.5-2: Local Area BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -29.2 dBm | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -31 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -31 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.5-3: Local Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -33.5 dBm | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | -35 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -35 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.5-4: Local Area BS operating band unwanted emission limits
for 3 MHz channel bandwidth (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -33.2 dBm | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | -35 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -35 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.5-5: Local Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -37 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.5-6: Local Area BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -37 dBm/100 kHz. NOTE 2: For BS a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

# 6.6.5.5.5.6	Basic limits for Medium Range BS (Category A and B)

For an AAS BS of Medium Range BS class in E-UTRA bands ≤ 3 GHz, basic limits are specified in tables 6.6.5.5.5.6-1, 6.6.5.5.5.6-3, 6.6.5.5.5.6-5, 6.6.5.5.5.6-7, 6.6.5.5.5.6-9 and 6.6.5.5.5.6-11.

For an AAS BS of Medium Range BS class in E-UTRA bands > 3 GHz, basic limits are specified in tables 6.6.5.5.5.6-2, 6.6.5.5.5.6-4, 6.6.5.5.5.6-6, 6.6.5.5.5.6-8, 6.6.5.5.5.6-10 and 6.6.5.5.5.6-12.

Table 6.6.5.5.5.6-1: Medium Range BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth, 31 < Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) -53.5dB | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -25 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -25 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-2: Medium Range BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth, 31 < Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) -53.2dB | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -25 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -25 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap basic limit < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-3: Medium Range BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -22.5 dBm | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -25 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -25 dBm/100 kHz. NOTE 2: For BS a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-4: Medium Range BS operating band unwanted emission limits
for 1.4 MHz channel bandwidth, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -22.2 dBm | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -25 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -25 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-5: Medium Range BS operating band unwanted emission limits
for 3 MHz channel bandwidth, 31 < Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) -57.5dB | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | Min(Pmax,c,cell - 10*log10(NTXU,countedpercell) -59dB, -25 dBm) (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be Min(Pmax,c,cell - 10*log10(NTXU,countedpercell) -59dB, -25 dBm)/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-6: Medium Range BS operating band unwanted emission limits
for 3 MHz channel bandwidth, 31 < Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | Pmax,c,cell - 10*log10(NTXU,countedpercell) -57.2dB | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | Min(Pmax,c,cell - 10*log10(NTXU,countedpercell) -59dB, -25 dBm) (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be Min(Pmax,c,cell - 10*log10(NTXU,countedpercell) -59dB, -25 dBm)/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-7: Medium Range BS operating band unwanted emission limits
for 3 MHz channel bandwidth, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -26.5 dBm | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | -28 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -28 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-8: Medium Range BS operating band unwanted emission limits
for 3 MHz channel bandwidth, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -26.2 dBm | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | -28 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -28 dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-9: Medium Range BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth, 31< Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Pmax,c,cell - 10*log10(NTXU,countedpercell) -58.5dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Pmax,c,cell - 10*log10(NTXU,countedpercell) -60 dB, -25 dBm) (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be Min(Pmax,c,cell - 10*log10(NTXU,countedpercell) -60 dB, -25 dBm)/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-10: Medium Range BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth, 31< Pmax,c,cell-10*log10(NTXU,countedpercell)  38 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Pmax,c,cell - 10*log10(NTXU,countedpercell) -58.2dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Pmax,c,cell - 10*log10(NTXU,countedpercell) -60 dB, -25 dBm) (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be Min(Pmax,c,cell - 10*log10(NTXU,countedpercell) -60 dB, -25 dBm)/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-11: Medium Range BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm (E-UTRA bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -27.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -29dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.6.5.5.5.6-12: Medium Range BS operating band unwanted emission limits
for 5, 10, 15 and 20 MHz channel bandwidth, Pmax,c,cell-10*log10(NTXU,countedpercell)  31 dBm (E-UTRA bands > 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Notes 1 and 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -27.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 8) | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -29dBm/100 kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20 MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

# 6.6.5.5.5.7	Basic limits for Additional requirements

In certain regions the following requirement may apply. For E-UTRA TAB connector operating in Bands 5, 26, 27 or 28, basic limits are specified in table 6.6.5.5.5.7-1.

Table 6.6.5.5.5.7-1: Additional operating band unwanted emission limits for E-UTRA bands < 1 GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz f < 1 MHz | 0.005 MHz f_offset < 0.995 MHz | -14 dBm | 10 kHz |
| 3 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -13 dBm | 30 kHz |
| 5 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -15 dBm | 30 kHz |
| 10 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 15 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 20 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| All | 1 MHz f < fmax | 1.05 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 100 kHz |
| NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |  |

In certain regions the following requirement may apply. For E-UTRA a TAB connector operating in Bands 2, 4, 10, 23, 25, 30, 35, 36, 41, 66, basic limits are specified in table 6.6.5.5.5.7-2.

Table 6.6.5.5.5.7-2: Additional operating band unwanted emission limits for E-UTRA bands > 1GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz f < 1 MHz | 0.005 MHz f_offset < 0.995 MHz | -14 dBm | 10 kHz |
| 3 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -13 dBm | 30 kHz |
| 5 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -15 dBm | 30 kHz |
| 10 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 15 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -15 dBm | 100 kHz |
| 20 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -16 dBm | 100 kHz |
| All | 1 MHz f < fmax | 1.5 MHz f_offset < f_offsetmax | -13 dBm (Note 8) | 1 MHz |
| NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |  |

In certain regions the following requirement may apply. For E-UTRA a TAB connector operating in Bands 12, 13, 14, 17, 29 basic limits are specified in table 6.6.5.5.5.7-3.

Table 6.6.5.5.5.7-3: Additional operating band unwanted emission limits
for E-UTRA (bands 12, 13, 14, 17 and 29)

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| All | 0 MHz f < 100 kHz | 0.015 MHz f_offset < 0.085 MHz | -13 dBm | 30 kHz |
| All | 100 kHz f < fmax | 150 kHz f_offset < f_offsetmax | -13 dBm (Note 8) | 100 kHz |
| NOTE 8: The requirement is not applicable when fmax < 10 MHz. |  |  |  |  |

In certain regions, the following requirements may apply to an E-UTRA TDD TAB connector operating in the same geographic area and in the same operating band as another E-UTRA TDD system without synchronisation. For this case the basic limit shall be -52 dBm/MHz in each supported downlink operating band, except in:

-	The frequency range from 10 MHz below the lower channel edge to the frequency 10 MHz above the upper channel edge of each supported band.

In certain regions the following requirement may apply for protection of DTT. For E-UTRA a TAB connector operating in Band 20, the maximum level of emissions in the band 470-790 MHz, measured in an 8MHz filter bandwidth on centre frequencies Ffilter according to table 6.6.5.5.5.7-4, shall be based upon a declared basic limit PEM,N declared by the manufacturer.  This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.6.5.5.5.7-4: Declared emissions basic limit for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission basic limit (dBm) |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

NOTE 1:	The regional requirement is defined in terms of EIRP, which is dependent on both the BS emissions at the TAB connector and the RND and antenna array. The EIRP level is calculated using: PEIRP = PE + Gant where PE denotes the TAB connector unwanted emission level at the TAB connector, Gant equals the RDN and antenna array gain. The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in annex G of TS 36.104 [11].

In certain regions the following requirement may apply for the protection of systems operating in frequency bands adjacent to band 1 as defined in clause 5.5, in geographic areas in which both an adjacent band service E-UTRA are deployed.

The basic limits shall be:

Table 6.6.5.5.5.7-5: Emissions limits for protection of adjacent band services

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth |
| --- | --- | --- | --- |
| 1 | 2100-2105 MHz | -30 + 3.4 × (f - 2100 MHz) dBm | 1 MHz |
|  | 2175-2180 MHz | -30 + 3.4 × (2180 MHz - f) dBm | 1 MHz |

In regions where FCC regulation applies, requirements for protection of GPS according to FCC Order DA 20-48applies for operation in Band 24. The following normative requirement covers the base station, to be used together with other information about the site installation to verify compliance with the requirement in FCC Order DA 20-48. The requirement applies to a TAB connector operating in Band 24 to ensure that appropriate interference protection is provided to the GPS. This requirement applies to the frequency range 1541-1650 MHz, even though part of this range falls within the spurious domain.

The maximum level of emissions in the 1541 - 1650 MHz band, measured in measurement bandwidth according to table 6.6.5.5.5.7-6 shall be based upon declared basic limits PEM_B24,a, PEM_B24,b, PEM_B24,c, PEM_B24d, PEM_B24,e and PEM_B24,f  declared by the manufacturer.

Table 6.6.5.5.5.7-6: Declared emissions basic limits for protection of the 1541-1650 MHz band

| Operating Band | Frequency range (MHz) | Declared emission level (dBW) (Measurement bandwidth = 1 MHz) | Declared emission basic limit (dBW) of discrete emissions of less than 700 Hz bandwidth (Measurement bandwidth = 1 kHz) | Declared emission basic limit (dBW) of discrete emissions of less than 2 kHz bandwidth (Measurement bandwidth = 1 kHz) |
| --- | --- | --- | --- | --- |
| 24 | 1541 - 1559 | PEM_B24,a |  | PEM_B24,f |
|  | 1559 - 1610 | PEM_B24,b | PEM_B24,d |  |
|  | 1610 - 1650 | PEM_B24,c | PEM_B24,e |  |

NOTE 2:	The regional requirements in FCC Order DA 20-48 are defined in terms of EIRP, which is dependent on both the BS emissions at the TAB connector and the RND and antenna array. The EIRP level is calculated using: PEIRP = PE + Gant where PE denotes the TAB connector unwanted emission level at the TAB connector, Gant equals the RDN and antenna array gain. The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in annex G of TS 36.104 [11].

The following requirement may apply to an E-UTRA TAB connector operating in Band 41 in certain regions. Basic limits are specified in table 6.6.5.5.5.7-7.

Table 6.6.5.5.5.7-7: Additional operating band unwanted emission basic limits for Band 41

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | basic limit | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 10 MHz | 10 MHz f < 20 MHz | 10.5 MHz f_offset < 19.5 MHz | -22 dBm | 1 MHz |
| 20 MHz | 20 MHz f < 40 MHz | 20.5 MHz f_offset < 39.5 MHz | -22 dBm | 1 MHz |
| NOTE: This requirement applies for E-UTRA carriers allocated within 2545-2575 MHz or 2595-2645 MHz. |  |  |  |  |

In certain regions, the following requirements may apply to E-UTRA TAB connector operating in Band 32 within 1452-1492 MHz. The maximum level of operating band unwanted emissions, measured on centre frequencies f_offset with filter bandwidth, according to table 6.6.5.5.5.7-8, shall be defined based on the declared basic limits PEM,B32,a ,  PEM,B32,b and PEM,B32,c declared by the manufacturer.

Table 6.6.5.5.5.7-8: Declared operating band 32 unwanted emission basic limits within 1452-1492 MHz

| Frequency offset of measurement filter centre frequency, f_offset | Declared emission basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2.5 MHz | PEM,B32,a | 5 MHz |
| 7.5 MHz | PEM,B32,b | 5 MHz |
| 12.5 MHz ≤ f_offset ≤ f_offsetmax,B32 | PEM,B32,c | 5 MHz |
| NOTE: f_offsetmax,B32 denotes the frequency difference between the lower channel edge and 1454.5 MHz, and the frequency difference between the upper channel edge and 1489.5 MHz for the set channel position. |  |  |

NOTE 3:	The regional requirement, included in [25], is defined in terms of EIRP which is dependent on both the BS emissions at the TAB connector and the RND and antenna array. The EIRP level is calculated using: PEIRP = PE + Gant where PE denotes the TAB connector unwanted emission level at the TAB connector, Gant equals the RDN and antenna array gain. The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in annex G of TS 36.104 [11].

In certain regions, the following requirement may apply to an E-UTRA TAB connector operating in Band 32 within 1452-1492 MHz for the protection of services in spectrum adjacent to the frequency range 1452-1492 MHz. The maximum level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to table 6.6.5.5.5.7-9, shall be based upon the declared basic limits PEM,B32,d and PEM,B32,e declared by the manufacturer. This requirement applies in the frequency range 1429-1518MHz even though part of the range falls in the spurious domain.

Table 6.6.5.5.5.7-9: Operating band 32 declared emission outside 1452-1492 MHz

| Filter centre frequency, Ffilter | Declared emission basic limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 1429.5 MHz ≤ Ffilter ≤ 1448.5 MHz | PEM,B32,d | 1 MHz |
| Ffilter = 1450.5 MHz | PEM,B32,e | 3 MHz |
| Ffilter = 1493.5 MHz | PEM,B32,e | 3 MHz |
| 1495.5 MHz ≤ Ffilter ≤ 1517.5 MHz | PEM,B32,d | 1 MHz |

NOTE 4:	The regional requirement, included in [25], is defined in terms of EIRP, which is dependent on both the BS emissions at the TAB connector and the RND and antenna array. The EIRP level is calculated using: PEIRP = PE + Gant where PE denotes the TAB connector unwanted emission level at the TAB connector, Gant equals the RDN and antenna array gain. The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in annex G of TS 36.104 [11].

In certain regions the following requirement may apply to an E-UTRA TAB connector operating in Band 45. Basic limits are specified in table 6.6.5.5.5.7-10.

Table 6.6.5.5.5.7-10: Emissions basic limits for protection of adjacent band services

| Operating Band | Filter centre frequency, Ffilter | Basic limit (dBm) | Measurement Bandwidth |
| --- | --- | --- | --- |
|  | Ffilter = 1467.5 | -20 | 1 MHz |
|  | Ffilter = 1468.5 | -23 | 1 MHz |
| 45 | Ffilter = 1469.5 | -26 | 1 MHz |
|  | Ffilter = 1470.5 | -33 | 1 MHz |
|  | Ffilter = 1471.5 | -40 | 1 MHz |
|  | 1472.5 MHz ≤ Ffilter ≤ 1491.5 MHz | -47 | 1 MHz |

The following requirement may apply to E-UTRA BS operating in Band 48 in certain regions. Emissions shall not exceed the maximum levels specified in Table 6.6.5.5.5.7-11.

Table 6.6.5.5.5.7-11: Additional operating band unwanted emission limits for Band 48

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 3) |
| --- | --- | --- | --- | --- |
| All | 0 MHz f < 10 MHz | 0.5 MHz f_offset < 9.5 MHz | -13 dBm | 1 MHz |

### 6.6.6	Spurious emission

#### 6.6.6.1	Definition and applicability

The transmitter spurious emission limits apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band. For some operating bands the upper frequency limit is higher than 12.75 GHz. In some exceptional cases, requirements apply also closer than 10 MHz from the downlink operating band; these cases are highlighted in the requirement tables. For operating bands supported by multi-band TAB connectors exclusion bands apply to each supported band.

The requirements applies for both single band and multiband TAB connectors (except for frequencies at which exclusion bands or other multi-band provisions apply) and for all transmission modes foreseen by the manufacturer's specification. Unless otherwise stated, all requirements are measured as mean power.

For operation in region 2, where the FCC guidance for MIMO systems in [36] is applicable, NTXU,countedpercell shall be equal to 1 for the purposes of calculating the spurious emissions limits in clauses 6.6.6. For all other unwanted emissions requirements, NTXU,countedpercell shall be the value calculated according to clause 6.1, unless stated differently in regional regulation.

The AAS BS test requirements for co-location spurious emissions limits which are specified for Band 46 in TS 37.104 [5], are applicable for AAS BS.

#### 6.6.6.2	Minimum requirement

The minimum requirement for MSR operation are defined in TS 37.105 [8], clause 6.6.6.2.

The minimum requirement for UTRA operation are defined in TS 37.105 [8], clause 6.6.6.3.

The minimum requirement for E-UTRA operation are defined in TS 37.105 [8], clause 6.6.6.4.

#### 6.6.2.3	Test purpose

This test measures conducted spurious emission from the AAS BS transmit TAB connector(s), while the transmitter unit associated with the TAB connector under test is in operation.

#### 6.6.6.4	Method of test

##### 6.6.6.4.1	Initial conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

##### 6.6.6.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.3. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in clause D.1.3. All TAB connectors not under test shall be terminated.

2)	Measurements shall use a measurement bandwidth in accordance to the conditions in TS 37.104 [12] clause 6.6.1.

3)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

4)	Set the TAB connector to transmit:

a)	For MSR:

-	Set the TAB connector to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.11.

b)	For UTRA:

-	For a TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to TM1, clause 4.12.2, at the manufacturer's declared rated output power, Prated,c,TABC.

-	For a TAB connector declared to be capable of multi-carrier operation, set the set the TAB connector to transmit according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

c)	For E-UTRA:

-	TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to E-TM1.1 in clause 4.12.2, at manufacturer's declared rated output power Prated,c,TABC.

-	For a TAB connector declared to be capable of multi-carrier and/or CA operation, set the set the TAB connector to transmit according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

5)	Measure the emission at the specified frequencies with specified measurement bandwidth and note that the measured value does not exceed the test requirement in clause 6.6.6.5.

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.6.5	Test requirements

##### 6.6.6.5.1	General

Conformance may be shown to either the measure and sum test requirement or the per TAB connector test requirement:

1)	The spurious emission test requirements for an AAS BS when using the measure and sum alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this clause, the power summation of emissions at the TAB connectors of the TAB connector TX cell group shall not exceed a limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell) , unless stated differently in regional regulation.

2)	The spurious emission test requirements for an AAS BS when using the per TAB connector alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this clause, the emissions at each of the TAB connectors of the TAB connector TX cell group shall not exceed a limit specified as the basic limit + X - 10log(n) where n is the number of TAB connectors in the TAB connector TX cell group and X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

The appropriate table for the basic limit is based on the same power level (PRated,c,sys) as used for the AAS BS rated power limits for BS classes in table 6.2.2.1-1 the same method of scaling the power level using NTXU,counted is used.

##### 6.6.6.5.2	Basic limits

# 6.6.6.5.2.1	General

The basic limits specified in tables 6.6.6.5.2.2-1 to table 6.6.6.5.2.6-3 are applicable for the TAB connector under test.

The test requirements of either clause 6.6.6.5.2.2 (Category A limits) or clause 6.6.6.5.2.3 (Category B limits) shall apply. In addition for a TAB connector operating in Band Category 2, the test requirements of 6.6.6.5.2.4 shall apply in case of Category B limits.

# 6.6.6.5.2.2	Spurious emissions (Category A)

The basic limit of any spurious emission shall not exceed the limits in table 6.6.6.5.2.2 -1

Table 6.6.6.5.2.2 -1: Spurious emission limits, Category A

| Frequency range | Maximum level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 9kHz - 150 kHz |  | 1 kHz | Note 1 |
| 150 kHz - 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz - 1GHz | -13 dBm | 100 kHz | Note 1 |
| 1GHz - 12.75 GHz |  | 1 MHz | Note 2 |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in Recommendation ITU-R SM.329 [13], s4.1 NOTE 2: Bandwidth as in Recommendation ITU-R SM.329 [13], s4.1. Upper frequency as in Recommendation ITU-R SM.329 [13], s2.5 table 1 NOTE 3: For E-UTRA applies only for Bands 22, 42, 43 and 48. NOTE 4: For UTRA applies only for Band XXII. |  |  |  |

# 6.6.6.5.2.3	Spurious emissions (Category B)

For MSR and E-UTRA the basic limits of is in table 6.6.6.5.2.3-1.

For UTRA FDD the basic limits of is in tables 6.6.6.5.2.3-2 and 6.6.6.5.2.3-3.

For UTRA FDD the basic limits of is in table 6.6.6.5.2.3-2.

Table 6.6.6.5.2.3-1: Spurious emissions limits, Category B

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz 150 kHz | -36 dBm | 1 kHz | Note 1 |
| 150 kHz 30 MHz | -36 dBm | 10 kHz | Note 1 |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -30 dBm | 1 MHz | Note 2 |
| 12.75 GHz 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in Recommendation ITU-R SM.329 [13], s4.1. NOTE 2: Bandwidth as in Recommendation ITU-R SM.329 [13], s4.1. Upper frequency as in Recommendation ITU-R SM.329 [13], s2.5 table 1. NOTE 3: Applies only for Bands 22, 42, 43 and 48. |  |  |  |

Table 6.6.6.5.2.3-2: Mandatory spurious emissions basic limits, UTRA FDD
in operating band I, II, III, IV, VII, X, XXV, XXXII (Category B) and UTRA TDD

| Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz 150 kHz | -36 dBm | 1 kHz | Note 1 |
| 150 kHz 30 MHz | -36 dBm | 10 kHz | Note 1 |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz Flow - 10 MHz | -30 dBm | 1 MHz | Note 1 |
| Flow - 10 MHz Fhigh + 10 MHz | -15 dBm | 1 MHz | Note 2 |
| Fhigh + 10 MHz 12.75 GHz | -30 dBm | 1 MHz | Note 3 |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 3, Note 4 |
| NOTE 1: Bandwidth as in Recommendation ITU-R SM.329 [35], s4.1. NOTE 2: Limit based on Recommendation ITU-R SM.329 [35], s4.3 and Annex 7. NOTE 3: Bandwidth as in Recommendation ITU-R SM.329 [35], s4.1. Upper frequency as in ITU-R SM.329 [35], s2.5 table 1. NOTE 4: UTRA FDD applies only for Band XXII. |  |  |  |
| Key: Flow: The lowest downlink frequency of the operating band as defined in clause 4.5. Fhigh: The highest downlink frequency of the operating band as defined in clause 4.5. |  |  |  |

Table 6.6.6.5.2.3-3: Mandatory spurious emissions basic limits,
UTRA in operating band V, VIII, XII, XIII, XIV, XX, XXVI (Category B)

| Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz 150 kHz | -36 dBm | 1 kHz | Note 1 |
| 150 kHz 30 MHz | -36 dBm | 10 kHz | Note 1 |
| 30 MHz Flow - 10 MHz | -36 dBm | 100 kHz | Note 1 |
| Flow - 10 MHz Fhigh + 10 MHz | -16 dBm | 100 kHz | Note 2 |
| Fhigh + 10 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1GHz 12.75GHz | -30 dBm | 1 MHz | Note 3 |
| NOTE 1: Bandwidth as in Recommendation ITU-R SM.329 [35], s4.1. NOTE 2: Limit based on Recommendation ITU-R SM.329 [35], s4.3 and Annex 7. NOTE 3: Bandwidth as in Recommendation ITU-R SM.329 [35], s4.1. Upper frequency as in ITU-R SM.329 [35], s2.5 table 1. |  |  |  |
| Key: Flow: The lowest downlink frequency of the operating band as defined in clause 4.5. Fhigh: The highest downlink frequency of the operating band as defined in clause 4.5. |  |  |  |

# 6.6.6.5.2.4	Protection of the BS receiver of own or different BS

This requirement shall be applied for FDD operation in order to prevent the receivers of Base Stations being desensitised by emissions from the transmitter TAB connector. It is measured at the transmit TAB connector for any type of TAB connector which has common or separate Tx/Rx antenna ports.

The basic limit of any spurious emission shall not exceed the limits in table 6.6.6.5.2.4 -1, depending on the declared Base Station class and Band Category.

Table 6.6.6.5.2.4-1: Spurious emissions basic limits for protection of the BS receiver

|  | Frequency range | UTRA basic limit | E-UTRA basic limit | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| Wide Area BS | FUL_low - FUL_high | -96 dBm | -96 dBm | 100 kHz |
| Medium Range BS | FUL_low - FUL_high | -86 dBm | -91 dBm | 100 kHz |
| Local Area BS | FUL_low - FUL_high | -82 dBm | -88 dBm | 100 kHz |
| Note: For E-UTRA Band 28 BS operating in regions where Band 28 is only partially allocated for E-UTRA operations, this requirement only applies in the UL frequency range of the partial allocation. |  |  |  |  |

# 6.6.6.5.2.5	Co-existence with other systems in the same geographical area

These requirements may be applied for the protection of system operating in frequency ranges other than the TAB connector downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the AAS BS, or they may be set by local or regional regulation as a mandatory requirement for an operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in clause 4.4.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM/EDGE, CDMA, UTRA, E-UTRA, etc.) as listed below. The basic limit any spurious emission are in table 6.6.6.5.2.5-1 for TAB connector(s) where requirements for co-existence with the system listed in the first column apply. For multi-band TAB connector(s), the exclusions and conditions in the Note column of table 6.6.6.5.2.5-1 apply for each supported operating band.

Table 6.6.6.5.2.5-1: Spurious emissions basic limits for co-existence
with systems operating in other frequency bands

| System type operating in the same geographical area | Band for co-existence requirement | Basic limit | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz | -57 dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in band VIII. This requirement does not apply to E-UTRA BS operating in band 8 |
|  | 876 - 915 MHz | -61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to UTRA FDD operating in band VIII, since it is already covered by the requirement in clause 6.6.6.5.2.4. For the frequency range 880-915 MHz, this requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| DCS1800 | 1805 - 1880 MHz | -47 dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in band III. This requirement does not apply to UTRA TDD operating in Band b and c. For UTRA TDD BS operating in Band f, it applies for 1805 - 1850 MHz This requirement does not apply to E-UTRA BS operating in band 3. |
|  | 1710 - 1785 MHz | -61 dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in band III, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD operating in Band b and c. For UTRA TDD BS operating in Band f, it applies for 1710 - 1755 MHz This requirement does not apply to E-UTRA BS operating in band 3, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| PCS1900 | 1930 - 1990 MHz | -47 dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band II or band XXV. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in frequency band 2, band 25 or band 36. |
|  | 1850 - 1910 MHz | -61 dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band II or band XXV, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in frequency band 2 or 25, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in frequency band 35. |
| GSM850 or CDMA850 | 869 - 894 MHz | -57 dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band V or XXVI. This requirement does not apply to E-UTRA BS operating in frequency band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -61 dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band V or XXVI, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in frequency band 5 or 26, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band I or E-UTRA Band 1 | 2110 - 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band I, This requirement does not apply to E-UTRA BS operating in band 1 or 65. |
|  | 1920 - 1980 MHz | -49 dBm (UTRA TDD -43 dBm for WA BS -40 dBm for LA BS) | 1 MHz (UTRA TDD 3.84 MHz) | This requirement does not apply to UTRA FDD BS operating in band I, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 1 or 65, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band II or E-UTRA Band 2 | 1930 - 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band II or band XXV4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 2 or 25. |
|  | 1850 - 1910 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band II or band XXV, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 2 or 25, since it is already covered by the requirement in clause 6.6.6.5.2.4 |
| UTRA FDD Band III or E-UTRA Band 3 | 1805 - 1880 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band III or band IX For UTRA TDD BS operating in Band f, it applies for 1805- 1850 MHz This requirement does not apply to E-UTRA BS operating in band 3. |
|  | 1710 - 1785 MHz | -49 dBm (UTRA TDD -43 dBm for WA BS -40 dBm for LA BS) | 1 MHz (UTRA TDD 3.84 MHz) | This requirement does not apply to UTRA FDD BS operating in band III, since it is already covered by the requirement in clause 6.6.6.5.2.4. For UTRA BS operating in band IX, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 3 or 9, since it is already covered by the requirement in clause 6.6.6.5.2.4. For UTRA TDD BS operating in Band f, it applies for 1710- 1755 MHz For E-UTRA BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in clause 6.6.6.5.2.4. |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band IV or band X This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66 |
|  | 1710 - 1755 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band IV or band X, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band V or E-UTRA Band 5 | 869 - 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band V or XXVI This requirement does not apply to E-UTRA BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -49 dBm (UTRA TDD -43 dBm for WA BS -40 dBm for LA BS) | 1 MHz (UTRA TDD 3.84 MHz) | This requirement does not apply to UTRA FDD BS operating in band V or XXVI, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 5 or 26, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band VI or XIX, or E-UTRA Band 6, 18 or 19 | 860 - 890 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VI or XIX For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 6, 18, 19. |
|  | 815 - 845 MHz | -49 dBm (UTRA TDD -43 dBm) | 1 MHz (UTRA TDD 3.84 MHz) | This requirement does not apply to UTRA FDD BS operating in band VI or XIX, since it is already covered by the requirement in clause 6.6.6.5.2.4. For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 18 between 815-830 MHz, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 6, 19 between 830-845 MHz, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band VII or E-UTRA Band 7 | 2620 - 2690 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VII, This requirement does not apply to E-UTRA BS operating in band 7. This requirement does not apply to E-UTRA BS operating in band 7, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
|  | 2500 - 2570 MHz | -49 dBm (UTRA TDD -43 dBm for WA BS -40 dBm for LA BS) | 1 MHz (UTRA TDD 3.84 MHz) | This requirement does not apply to UTRA FDD BS operating in band VII, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band VIII or E-UTRA Band 8 | 925 - 960 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VIII. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 8. |
|  | 880 - 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band VIII, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | -52 dBm (UTRA TDD -43 dBm) | 1 MHz (UTRA TDD 3.84 MHz) | This requirement does not apply to UTRA FDD BS operating in band III or band IX For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 3 or 9. |
|  | 1749. 9 - 1784.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band III or band IX, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 3 or 9, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band IV or band X This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66 |
|  | 1710 - 1770 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band X, since it is already covered by the requirement in clause 6.6.6.5.2.4. For UTRA FDD BS operating in Band IV, it applies for 1755 MHz to 1770 MHz, while the rest is covered in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 10 or 66, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in clause 6.6.6.5.2.4. |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 - 1510.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XI, XXI, or XXXII. For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 11, 21 or 32. |
|  | 1427.9 - 1447.9 MHz | -49 dBm (UTRA TDD -43 dBm) | 1 MHz (UTRA TDD 3.84 MHz) | This requirement does not apply to UTRA FDD BS operating in band XI, since it is already covered by the requirement in clause 6.6.6.5.2.4. For UTRA BS operating in band XXXII, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 11, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in band 32, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. |
|  | 1447.9 - 1462.9 MHz | -49 dBm (UTRA TDD -43 dBm) | 1 MHz (UTRA TDD 3.84 MHz) | This requirement does not apply to UTRA FDD BS operating in band XXI, since it is already covered by the requirement in clause 6.6.6.5.2.4. For UTRA BS operating in band XXXII, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. For UTRA TDD applicable in Japan up to 1462.9MHz. This requirement does not apply to E-UTRA BS operating in band 21, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in band 32, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. |
| UTRA FDD Band XII or E-UTRA Band 12 | 729 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 12. |
|  | 699 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 12, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6) |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 13. |
|  | 777 - 787 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIII, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 13, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIV This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 14. |
|  | 788 - 798 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIV, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 14, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| E-UTRA Band 17 | 734 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 17. |
|  | 704 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 17, since it is already covered by the requirement in clause 6.6.4.5.3. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6) |
| UTRA FDD Band XX or E-UTRA Band 20 | 791 - 821 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XX This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 20 or 28. |
|  | 832 - 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XX, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 20, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 -3590 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 22, 42 or 48. |
|  | 3410 -3490 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXII, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 22, since it is already covered by the requirement in clause 6.6.4.5.3. This requirement does not apply to E-UTRA BS operating in Band 42 |
| E-UTRA Band 23 | 2180 - 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 23 or 66. This requirement does not apply to UTRA TDD |
|  | 2000 - 2020 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band II or XXV, where the limits are defined separately. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 23, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to BS operating in Bands 2 or 25, where the limits are defined separately. |
|  | 2000 - 2010 MHz | -30 dBm | 1 MHz | This requirement only applies to UTRA FDD BS operating in Band II or Band XXV. This requirement applies starting 5 MHz above the Band XXV downlink operating band. (Note 3) This requirement does not apply to UTRA TDD This requirement only applies to E-UTRA BS operating in Band 2 or Band 25. This requirement applies starting 5 MHz above the Band 25 downlink operating band. (Note 4) |
|  | 2010 - 2020 MHZ | -49 dBm | 1 MHz |  |
| E-UTRA Band 24 | 1525 - 1559 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 24. This requirement does not apply to UTRA TDD |
|  | 1626.5 - 1660.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 24, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1930 - 1995 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band II or band XXV This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 2 or 25 |
|  | 1850 - 1915 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXV, since it is already covered by the requirement in clause 6.6.6.5.2.4. For UTRA FDD BS operating in Band II, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 25, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.6.6.5.2.4. |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859-894 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band V or band XXVI This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 814-849 MHz | -49 MHz | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XXVI, since it is already covered by the requirements in clause 6.6.6.5.2.4.For UTRA FDD BS operating in band V, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.3.2 This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 26, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| E-UTRA Band 27 | 852 - 869 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS operating in Band V or XXVI. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in Band 5, 26 or 27. |
|  | 807 - 824 MHz | -49 dBm | 1 MHz | For UTRA BS operating in Band XXVI, it applies for 807 MHz to 814 MHz, while the rest is covered in clause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in Band 27, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in clause 6.6.6.5.2.4. This requirement also applies to E-UTRA BS operating in Band 28, starting 4 MHz above the Band 28 downlink operating band (Note 5). |
| E-UTRA Band 28 | 758 - 803 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 20, 28, 44, 67 or 68. This requirement does not apply to UTRA TDD |
|  | 703 - 748 MHz | -49 MHz | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 28, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in Band 44. This requirement does not apply to UTRA TDD For E-UTRA BS operating in Band 67, it applies for 703 MHz to 736 MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733 MHz. |
| E-UTRA Band 29 | 717 - 728 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA TDD. This requirement does not apply to E-UTRA BS operating in Band 29 |
| E-UTRA Band 30 | 2350 - 2360 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA TDD. This requirement does not apply to E-UTRA BS operating in band 30 or 40. |
|  | 2305 - 2315 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA TDD. This requirement does not apply to E-UTRA BS operating in band 30, since it is already covered by the requirement in clause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in Band 40. |
| E-UTRA Band 31 | 462.5 -467.5 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA TDD. This requirement does not apply to E-UTRA BS operating in band 31. |
|  | 452.5 -457.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA TDD. This requirement does not apply to E-UTRA BS operating in band 31, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS operating in Band XI, XXI, or XXXII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 11, 21 or 32. |
| UTRA TDD in Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 33. |
| UTRA TDD in Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 34. |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 - 1910 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in Band 35. |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in Band 2 and 36. |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This is not applicable to E-UTRA BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD in Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 38 or 69. |
| UTRA TDD in Band f) or E-UTRA Band 39 | 1880 - 1920 MHz | -52 dBm | 1 MHz | Applicable in China for UTRA FDD. This is not applicable to E-UTRA BS operating in Band 39. |
| UTRA TDD in Band e) or E-UTRA Band 40 | 2300 - 2400 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 30 or 40. |
| E-UTRA Band 41 | 2496 - 2690 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 41. |
| E-UTRA Band 42 | 3400 - 3600 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 22, 42,43 or 48. |
| E-UTRA Band 43 | 3600 - 3800 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA TDD. This is not applicable to E-UTRA BS operating in Band 42, 43 or 48. |
| E-UTRA Band 44 | 703 - 803 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 28 or 44 |
| E-UTRA Band 45 | 1447 - 1467 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This is not applicable to E-UTRA BS operating in Band 45 |
| E-UTRA Band 46 | 5150 - 5925 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This is not applicable to E-UTRA BS operating in Band 46. |
| E-UTRA Band 48 | 3550 – 3700 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43 or 48. |
| E-UTRA Band 65 | 2110 - 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 1 or 65. |
|  | 1920 - 2010 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 65, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.6.6.5.2.4. |
| E-UTRA Band 66 | 2110 - 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 4, 10, 23 or 66. |
|  | 1710 - 1780 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 66, since it is already covered by the requirement in clause 6.6.4.5.3. For E-UTRA BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in clause 6.6.6.5.2.4. |
| E-UTRA Band 67 | 738 - 758 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in Band 28 or 67. |
| E-UTRA Band 68 | 753 -783 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 28, or 68. |
|  | 698-728 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 68, since it is already covered by the requirement in clause 6.6.6.5.2.4. For E-UTRA BS operating in Band 28, it applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.6.6.5.2.4. |
| E-UTRA Band 69 | 2570 - 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 38 or 69. |
| E-UTRA Band 70 | 1995 - 2020 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 2, 25 or 70. |
|  | 1695 – 1710 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 70, since it is already covered by the requirement in clause 6.6.6.5.2.4. |
| NOTE 1: The co-existence requirements do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see clause 4.5.). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 2: The table above assumes that two operating bands, where the frequency ranges would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band 25 or Band 29, the co-existence requirements in table 6.6.6.5.2.5-1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see tables 4.4-1 and 4.4-2). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.6.5.2.5-1 assumes that two operating bands, where the frequency ranges in table 4.4-1 or table 4.4-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III or E-UTRA Band 3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 - 1850 MHz and 1710 - 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 5:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band.

NOTE 6:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band or E-UTRA Band 17 UL operating band.

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between 10 MHz below the lowest BS transmitter frequency of the downlink operating band and 10 MHz above the highest BS transmitter frequency of the downlink operating band.

The basic limit for any spurious emission is:

Table 6.6.6.5.2.5-2: Spurious emissions basic limits for co-existence with PHS

| Frequency range | Basic limit | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -41 dBm | 300 kHz | Applicable for co-existence with PHS system operating in 1884.5-1915.7MHz |
| NOTE: The requirement is not applicable in China. |  |  |  |

The following requirement shall be applied to UTRA BS for co-existence with services in adjacent frequency bands. This requirement may be applied for the protection in bands adjacent to bands I or VII, as defined in clause 3.4.1, in geographic areas in which both an adjacent band service and UTRA FDD are deployed.

Table 6.6.6.5.2.5-3: spurious emissions basic limits for UTRA FDD protection
of adjacent band services

| Operating Band | Band | Basic limit | Measurement Bandwidth |
| --- | --- | --- | --- |
| I | 2100 - 2105 MHz | -30 + 3.4 (f - 2100 MHz) dBm | 1 MHz |
|  | 2175 - 2180 MHz | -30 + 3.4 (2180 MHz - f) dBm | 1 MHz |
| VII | 2610 - 2615 MHz | -30 + 3.4 (f - 2610 MHz) dBm | 1 MHz |
|  | 2695 - 2700 MHz | -30 +3.4 (2700 MHz - f) dBm | 1 MHz |

The following requirement shall be applied to TAB connectors operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS transmitter operating band up to 10 MHz above the highest frequency of the BS transmitter operating band. The basic limit for any spurious emission is:

Table 6.6.6.5.2.5-4: Spurious emissions basic limits for protection of 700 MHz public safety operations

| Operating Band | Band | Basic limit | Measurement Bandwidth |
| --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -46 dBm | 6.25 kHz |
| 13 | 793 - 805 MHz | -46 dBm | 6.25 kHz |
| 14 | 769 - 775 MHz | -46 dBm | 6.25 kHz |
| 14 | 799 - 805 MHz | -46 dBm | 6.25 kHz |

The following requirement shall be applied to TAB connectors operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The basic limit for any spurious emission is:

Table 6.6.6.5.2.5-5: BS Spurious emissions basic limits for protection of 800 MHz public safety operations

| Operating Band | Frequency range | Basic limit | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -13 dBm | 100 kHz | Applicable for offsets > 37.5 kHz from the channel edge |

The following requirement may apply to E-UTRA TAB connectors operating in Band 41 in certain regions. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The basic limit for any spurious emission is:

Table 6.6.6.5.2.5-6: Additional Spurious emissions basic limits for Band 41

| Frequency range | Basic limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 2505 MHz - 2535 MHz | -42dBm | 1 MHz |  |
| 2535 MHz - 2655 MHz | -22dBm | 1 MHz | Applicable at offsets ≥ 250 % of channel bandwidth from carrier frequency |
| NOTE: This requirement applies for 10 or 20 MHz E-UTRA carriers allocated within 2545 - 2575 MHz or 2595 - 2645 MHz. |  |  |  |

In addition to the requirements in clauses 6.6.6.5.2.1 to 6.6.6.5.2.5 and above in the present clause, the TAB connector may have to comply with the applicable emission limits established by FCC Title 47 [24], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

The following requirement may apply to a TAB connector operating in Band 30 in certain regions. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The basic limit for any spurious emission is:

Table 6.6.6.5.2.5-7: Additional Spurious emissions basic limits for Band 30

| Frequency range | Basic limit | Measurement Bandwidth |  |
| --- | --- | --- | --- |
| 2200 MHz - 2345 MHz | -45 dBm | 1 MHz |  |
| 2362.5 MHz - 2365 MHz | -25 dBm | 1 MHz |  |
| 2365 MHz - 2367.5 MHz | -40 dBm | 1 MHz |  |
| 2367.5 MHz - 2370 MHz | -42dBm | 1 MHz |  |
| 2370 MHz - 2395 MHz | -45 dBm | 1 MHz |  |

The following requirement may apply to E-UTRA BS operating in Band 48 in certain regions. The power of any spurious emission shall not exceed:

Table 6.6.6.5.2.5-8: Additional E-UTRA BS Spurious emissions limits for Band 48

| Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 3530MHz – 3720MHz | -25dBm | 1 MHz | Applicable 10MHz from the assigned channel edge |
| 3100MHz – 3530MHz 3720MHz – 4200MHz | -40dBm | 1 MHz |  |

# 6.6.6.5.2.6	Co-location with other Base Stations

These requirements may be applied for the protection of other BS receiver units when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD and/or E-UTRA BS are co-located with a BS.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The basic limit for any spurious emission are in table 6.6.6.5.2.6-1 for a MSR, E-UTRA or UTRA FDD TAB connector or tables 6.6.6.5.2.6-2 and 6.6.6.5.2.6-3 for UTRA TDD, where requirements for co-location with a BS type listed in the first column apply, depending on the declared Base Station class. For a multi-band TAB connector, the exclusions and conditions in the Notes column of table 6.6.6.5.2.6-1 apply for each supported operating band.

Table 6.6.6.5.2.6-1: Spurious emissions basic limits for MSR,
E-UTRA or UTRA FDD BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Basic limit (WA BS) | Basic limit (MR BS) | Basic limit (LA BS) | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876 - 915 MHz | -98 dBm | -91 dBm | MSR -88 dBm, UTRA, E-UTRA -70 dBm | 100 kHz |  |
| DCS1800 | 1710 - 1785 MHz | -98 dBm | -91 dBm (UTRA -96 dBm) | MSR -88 dBm, UTRA, E-UTRA -80 dBm | 100 kHz |  |
| PCS1900 | 1850 - 1910 MHz | -98 dBm | -91 dBm (UTRA -96 dBm) | MSR -88 dBm UTRA, E-UTRA -80 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 - 849 MHz | -98 dBm | -91 dBm | MSR -88 dBm UTRA, E-UTRA -70 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 | 1920 - 1980 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 | 1850 - 1910 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 | 1710 - 1785 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 | 824 - 849 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 845 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 | 2500 - 2570 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VIII or E-UTRA Band 8 | 880 - 915 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XII or E-UTRA Band 12 | 699 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 18 | 815 - 830 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 | 832 - 862 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 - 1462.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 - 3490 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42 |
| E-UTRA Band 23 | 2000 - 2020 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 24 | 1626.5 - 1660.5 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1850 - 1915 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814 - 849 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 27 | 807 - 824 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 28 | 703 - 748 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 44 |
| E-UTRA Band 30 | 2305 - 2315 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 40 |
| E-UTRA Band 31 | 452.5 - 457.5 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 - 1910 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 2 and 36 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 38. |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 33 and 39 |
| UTRA TDD Band e) or E-UTRA Band 40 | 2300 - 2400 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 30 or 40 |
| E-UTRA Band 41 | 2496 - 2690 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 41 |
| E-UTRA Band 42 | 3400 - 3600 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 22, 42 or 43, or 48 |
| E-UTRA Band 43 | 3600 - 3800 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 42 or 43, or 48 |
| E-UTRA Band 44 | 703 - 803 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 28 or 44 |
| E-UTRA Band 45 | 1447 – 1467 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to BS operating in Band 45 |
| E-UTRA Band 48 | 3550 – 3700 MHz | -96 dBm (UTRA -86 dBm) | -91 dBm Note 4 | -88 dBm (UTRA -78 dBm) | 100 kHz (UTRA 1 MHz) | This is not applicable to E-UTRA BS operating in Band 42, 43 or 48 |
| E-UTRA Band 65 | 1920 - 2010 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to AAS BS operating in Band 65 |
| E-UTRA Band 66 | 1710 - 1780 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 66 |
| E-UTRA Band 68 | 698 - 728 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 68 |
| E-UTRA Band 70 | 1695 – 1710 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.6.5.2.6-1 do not apply for the 10 MHz frequency range immediately outside the TAB connector transmit frequency range of a downlink operating band (clause 4.5). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21].

NOTE 2:	Table 6.6.6.5.2.6-1 assumes that two operating bands, where the corresponding TAB connector transmit and receive frequency ranges in clause 4.5 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD Base Stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized Base Stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	For UTRA MR BS the measurement bandwidth is the same as for E-UTRA (100 kHz).

Table 6.6.6.5.2.6-2: Spurious emissions basic limits
for UTRA TDD Wide Area BS co-located with another BS

| System type operating in the same geographical area | Band | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Macro GSM900 | 876 - 915 MHz | -98 dBm | 100 kHz |  |
| Macro DCS1800 | 1710 - 1785 MHz | -98 dBm | 100 kHz | This requirement does not apply to UTRA TDD operating in Band b and c. For UTRA TDD BS operating in Band f, it applies for 1710 - 1755 MHz |
| GSM850 or CDMA850 | 824 - 849 MHz | -98 dBm | 100 kHz |  |
| WA BS UTRA FDD Band I or | 1920 - 1980 MHz | -80 dBm (Note 1) | 3.84 MHz |  |
| E-UTRA Band 1 |  |  |  |  |
| WA BS UTRA FDD Band III or E-UTRA Band 3 | 1710 - 1785 MHz | -80 dBm | 3.84 MHz | For UTRA TDD BS operating in Band f, it applies for 1710 - 1755 MHz. |
| WA BS UTRA FDD Band V or E-UTRA Band 5 | 824 - 849 MHz | -80 dBm (Note 1) | 3.84 MHz |  |
| WA BS UTRA FDD Band VII or E-UTRA Band 7 | 2500 - 2570 MHz | - 80 dBm(Note 2) | 3.84 MHz |  |
| NOTE 1: The co-location requirements do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of the operating band (see clause 4.5). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 2: The requirements in table 6.17 are based on a minimum coupling loss of 30 dB between base stations. The co-location of different base station classes is not considered. NOTE 3: The table above assumes that two operating bands, where the frequency ranges would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

For UTRA TDD in geographic areas where 1,28 Mcps TDD is deployed, the RRC filtered mean power of any spurious emission in case of co-location shall not exceed the maximum level given in table 6.6.6.5.2.6-3.

For multi-band TAB connector, the exclusions and conditions in the Notes of table 6.6.6.5.2.6-3 for each supported operating band.

Table 6.6.6.5.2.6-3: Spurious emissions basic limits for co-location
with unsynchronised 1,28 Mcps UTRA TDD and/or E-UTRA TDD

| System type operating in the same geographic area | Frequency range | Basic limit | Measurement Bandwidth |
| --- | --- | --- | --- |
| WA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -96 dBm | 100 kHz |
| WA UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -96 dBm | 100 kHz |
| WA UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | -96 dBm | 100 kHz |
| WA UTRA TDD Band e) or E-UTRA Band 40 | 2300 - 2400 MHz | -96 dBm | 100 kHz |
| WA UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920 MHz | -96 dBm | 100 kHz |
| WA E-UTRA Band 41 | 2496 - 2690 MHz | -96 dBm | 100 kHz |
| WA E-UTRA Band 42 | 3400 - 3600 MHz | -96 dBm | 100 kHz |
| WA E-UTRA Band 44 | 703 - 803 MHz | -96 dBm | 100 kHz |
| LA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -88 dBm | 100 kHz |
| LA UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -88 dBm | 100 kHz |
| LA UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | -88 dBm | 100 kHz |
| LA UTRA TDD Band e) or E-UTRA Band 40 | 2300 - 2400 MHz | -88 dBm | 100 kHz |
| LA UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920 MHz | -88 dBm | 100 kHz |
| LA E-UTRA Band 41 | 2496 - 2690 MHz | -88 dBm | 100 kHz |
| LA E-UTRA Band 42 | 3400 - 3600 MHz | -88 dBm | 100 kHz |
| LA E-UTRA Band 44 | 703 - 803 MHz | -88 dBm | 100 kHz |
| NOTE 1: The requirement applies for frequencies more than 10 MHz below or above the supported frequency range declared by the vendor. The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 2: The requirements in this table are based on a minimum coupling loss of 30 dB between unsynchronised TDD base stations. The scenarios leading to these requirements are addressed in TR 25.942 [21]. NOTE 3: The table above assumes that two operating bands, where the frequency ranges would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. |  |  |  |

## 6.7	Transmitter intermodulation

### 6.7.1	Definition and applicability

#### 6.7.1.1	General

The transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array. The requirement applies during the transmitter ON period and the transmitter transient period.

The requirement applies at each TAB connector supporting transmission in the operating band.

The transmitter intermodulation level is the power of the intermodulation products when an interfering signal is injected into the TAB connector.

For AAS BS there are two types of transmitter intermodulation cases captured by the transmitter intermodulation requirement:

1)	Co-location transmitter intermodulation in which the interfering signal is from a co-located base station.

2)	Intra-system transmitter intermodulation in which the interfering signal is from other transmitter units within the AAS BS.

For AAS BS, the co-location transmitter intermodulation requirement is considered sufficient if the interfering signal for the co-location requirement is higher than the declared interfering signal for intra-system transmitter intermodulation requirement.

For TAB connectors capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the single-band requirements apply regardless of the interfering signals position relative to the Inter RF Bandwidth gap.

### 6.7.2	Minimum requirement

The minimum requirement for MSR operation are defined in TS 37.105 [8], clause 6.7.2.

The minimum requirement for UTRA operation are defined in TS 37.105 [8], clause 6.7.3.

The minimum requirement for UTRA operation are defined in TS 37.105 [8], clause 6.7.4.

### 6.7.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the TAB connector under test to restrict the generation of intermodulation products in its nonlinear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna to below specified levels.

### 6.7.4	Method of test

#### 6.7.4.1	Initial conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW in single-band operation; see clause 4.12.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

#### 6.7.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in annex D.1.2. All TAB connectors not under test shall be terminated.

2)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

3)	Set the set the TAB connector to transmit:

a)	For MSR:

-	Set the TAB connector to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.11.

b)	For UTRA FDD:

-	For a TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to TM1, clause 4.12.2, at the manufacturer's declared rated output power, Prated,c,TABC.

-	For a TAB connector declared to be capable of multi-carrier operation, set the set the TAB connector to transmit according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

c)	For UTRA TDD:

-	For a TAB connector declared to be capable of single carrier operation only, set the parameters of the transmitted signal according to table 6.7.4.2-1.

-	For a TAB connector declared to be capable of multi-carrier operation, set to transmit according to table 6.7.4.2-1on all carriers.

Table 6.7.4.2-1: Parameters of the transmitted signal for transmit intermodulation testing
for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; I = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 8 |
| Power of each DPCH | 1/8 of Base Station output power |
| Data content of DPCH | real life (sufficient irregular) |

d)	For E-UTRA:

-	TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to E- TM1.1 in clause 4.12.2, at manufacturer's declared rated output power Prated,c,TABC.

-	For a TAB connector declared to be capable of multi-carrier and/or CA operation, set the set the TAB connector to transmit according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

4)	Generate the interfering signal:

a)	For MSR:

-	using E-TM1.1 as defined in clause 4.12.2, with 5 MHz channel bandwidth, at a centre frequency offset according to the conditions in table 6.7.2.1-1 in TS 37.105 [8], but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

b)	For UTRA FDD:

-	in accordance to TM1, clause 4.12.2 with a frequency offset of according to the conditions of table 6.7.3.1-1 in TS 37.105 [8], but exclude interfering signal frequencies that are outside of the allocated downlink operating band or interfering signal frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

c)	For UTRA TDD:

-	The signal shall be like-modulated as the transmit signal and the active time slots of both signals shall be synchronized, with a frequency offset of according to the conditions of table 6.7.3.2-1 in TS 37.105 [8], but exclude interfering signal frequencies that are outside of the allocated downlink operating band or interfering signal frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

d)	For E-UTRA:

-	according to E-TM1.1, as defined in clause 4.12.2, with 5 MHz channel bandwidth and a centre frequency offset according to the conditions of table 6.7.4.1-1 in TS 37.105 [8], but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

5)	Adjust ATT1 so that level of the interfering signal is as defined in:

a)	For MSR:

i.	General co-location table 6.7.2.1-1 in TS 37.105 [8].

ii.	Additional co-location (BC1 and BC2) table 6.7.2.2-1 in TS 37.105 [8].

iii.	Additional co-location (BC3) table 6.7.2.3-1 in TS 37.105 [8].

iv.	Intra-system table 6.7.2.5-1 in TS 37.105 [8].

b)	For UTRA FDD:

i.	General co-location table 6.7.3.1-1 in TS 37.105 [8].

ii.	Intra-system table 6.7.3.3-1 in TS 37.105 [8].

c)	For UTRA TDD:

i.	General co-location for 1,28 Mcps TDD UTRA table 6.7.3.2-1 in TS 37.105 [8].

ii.	Intra-system table 6.7.3.3-1 in TS 37.105 [8].

d)	For E-UTRA:

i.	General co-location table 6.7.4.1-1 in TS 37.105 [8].

ii.	Additional requirement for Band 41 table 6.7.4.2-1 in TS 37.105 [8].

iii.	Intra-system table 6.7.4.131 in TS 37.105 [8].

6)	If the test signal is applicable according to clause 5, perform the unwanted emission tests specified in clauses 6.6.3, 6.6.4 and 6.6.5, for all third and fifth order intermodulation products which appear in the frequency ranges defined in clauses 6.6.3, 6.6.4 and 6.6.5. The width of the intermodulation products shall be taken into account.

7)	If the test signal is applicable according to clause 5, perform the Transmitter spurious emissions test as specified in clause 6.6.6, for all third and fifth order intermodulation products which appear in the frequency ranges defined in clause 6.6.6. The width of the intermodulation products shall be taken into account.

8)	Verify that the emission level does not exceed the required level in clause 6.7.5 with the exception of interfering signal frequencies.

9)	Repeat the test for the remaining interfering signal centre frequency offsets according to the conditions of:

a)	For MSR:

i.	General co-location table 6.7.2.1-1 in TS 37.105 [8].

ii.	Additional co-location (BC1 and BC2) table 6.7.2.2-1 in TS 37.105 [8].

iii.	Additional co-location (BC3) table 6.7.2.3-1 in TS 37.105 [8].

iv.	Intra-system table 6.7.2.5-1 in TS 37.105 [8].

b)	For UTRA FDD:

i.	General co-location table 6.7.3.1-1 in TS 37.105 [8].

ii.	Intra-system table 6.7.3.3-1 in TS 37.105 [8].

c)	For UTRA TDD:

i.	General co-location for 1,28 Mcps TDD UTRA table 6.7.3.2-1 in TS 37.105 [8].

ii.	Intra-system table 6.7.3.3-1 in TS 37.105 [8].

d)	For E-UTRA:

i.	General co-location table 6.7.4.1-1 in TS 37.105 [8].

ii.	Additional requirement for Band 41 table 6.7.4.2-1 in TS 37.105 [8].

iii.	Intra-system table 6.7.4.131 in TS 37.105 [8].

10)	Repeat the test for the remaining test signals defined in clause 5 for requirements 6.6.1, 6.6.2 and 6.6.4.

In addition, for multi-band TAB connector(s), the following steps shall apply:

11)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

NOTE:	The third order intermodulation products are centred at 2F1F2 and 2F2F1. The fifth order intermodulation products are centred at 3F12F2, 3F22F1, 4F1F2, and 4F2F1 where F1 represents the test signal centre frequency or centre frequency of each sub-block and F2 represents the interfering signal centre frequency. The widths of intermodulation products are:

-	(n*BWF1 + m*BWF2) for the nF1mF2 products;

-	(n*BWF2 + m*BWF1) for the nF2mF1 products;

where BWF1 represents the test signal RF bandwidth or channel bandwidth in case of single carrier, or sub-block bandwidth, and BWF2 represents interfering signal bandwidth.

### 6.7.5	Test requirements

#### 6.7.5.1	MSR test requirements

##### 6.7.5.1.1	General test requirement

In the frequency range relevant for this test the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in clause 6.6.6, operating band unwanted emission in clause 6.6.5 and ACLR in clause 6.6.3 in the presence of a wanted signal and an interfering signal according clause 6.7.4. for a TAB connector operating in BC1, BC2 and BC3.

The requirement is applicable outside the edges of the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or radio bandwidth edges. For TAB connectors supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For TAB connectors supporting operation in multiple operating bands, the requirement applies relative to the Base Station RF Bandwidth edges of each operating band. In case the inter Base Station RF Bandwidth gap is less than 15 MHz, the requirement in the gap applies only for interfering signal offsets where the interfering signal falls completely within the inter Base Station RF Bandwidth gap.

##### 6.7.5.1.2	Additional test requirement (BC1 and BC2)

In the frequency range relevant for this test  the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in clause 6.6.6, operating band unwanted emission in clause 6.6.5 and ACLR in clause 6.6.3 in the presence of a wanted signal and an interfering signal according clause 6.7.4. for a TAB connector operating in BC2.

The requirement is applicable outside the edges of the Base Station RF Bandwidth for BC2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

For TAB connectors supporting operation in non-contiguous spectrum in BC1 or BC2, the requirement is also applicable inside a sub-block gap with a gap size larger than or equal to two times the interfering signal centre frequency offset. For TAB connectors supporting operation in non-contiguous spectrum in BC1, the requirement is not applicable inside a sub-block gap with a gap size equal to or larger than 5 MHz. The interfering signal offset is defined relative to the sub-block edges.

For TAB connectors supporting operation in multiple operating bands, the requirement applies relative to the Base Station RF Bandwidth edges of a BC2 operating band. The requirement is also applicable for BC1 and BC2 inside an inter Base Station RF Bandwidth gap equal to or larger than two times the interfering signal centre frequency offset. For TAB connectors supporting operation in multiple operating bands, the requirement is not applicable for BC1 band inside an inter Base Station RF Bandwidth gap with a gap size equal to or larger than 5 MHz.

##### 6.7.5.1.3	Additional test requirement (BC3)

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in clause 6.6.6, operating band unwanted emission in clause 6.6.5 and ACLR in clause 6.6.3 in the presence of a wanted signal and an interfering signal according clause 6.7.4.

For TAB connectors supporting operation in multiple operating bands, the requirement applies relative to the Base Station RF Bandwidth edges of each operating band. In case the Inter RF Bandwidth gap is less than 3.2 MHz, the requirement in the gap applies only for interfering signal offsets where the interfering signal falls completely within the inter Base Station RF Bandwidth gap.

##### 6.7.5.1.4	Intra-system test requirement

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the unwanted emission limits specified for operating band unwanted emission in clause 6.6.5 and ACLR in clause 6.6.3 in the presence of a wanted signal and an interfering signal according clause 6.7.4 for a TAB connector operating in BC1, BC2 and BC3.

#### 6.7.5.2	Single RAT UTRA operation

##### 6.7.5.2.1	General test requirement for UTRA FDD

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the out of band emission or the spurious emission requirements of clause 6.6.5 and clause 6.6.6 in the presence of interfering signal according to according clause 6.7.4.

For TAB connectors supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For TAB connectors supporting operation in multiple operating bands, the requirement is also applicable inside an Inter RF Bandwidth gap for interfering signal offsets where the interfering signal falls completely within the Base Station RF Bandwidth gap.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 6.7.5.2.2	General test requirement for UTRA TDD

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the out of band emission or the spurious emission requirements of clause 6.6.5 and clause 6.6.6 in the presence of interfering signal according to according clause 6.7.4.

For TAB connectors supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For TAB connectors supporting operation in multiple operating bands, the requirement is also applicable inside an Inter RF Bandwidth gap for interfering signal offsets where the interfering signal falls completely within the Inter RF Bandwidth gap.

##### 6.7.5.2.3	Intra-system test requirement

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the unwanted emission limits specified for operating band unwanted emission in clause 6.6.5 and ACLR in clause 6.6.3 in the presence of a wanted signal and an interfering signal according to according clause 6.7.4.

#### 6.7.5.4	Single RAT E-UTRA operation

##### 6.7.5.4.1	General test requirement

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the unwanted emission limits in clauses 6.6.6, 6.6.5 and 6.6.3 in the presence of an E-UTRA interfering signal according to according clause 6.7.4.

The requirement is applicable outside the Base Station RF Bandwidth or radio bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or radio bandwidth edges.

For TAB connectors supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For TAB connectors supporting operation in multiple operating bands, the requirement applies relative to the Base Station RF Bandwidth edges of each supported operating band. In case the Inter RF Bandwidth gap is less than 15 MHz, the requirement in the gap applies only for interfering signal offsets where the interfering signal falls completely within the inter Base Station RF Bandwidth gap.

##### 6.7.5.4.2	Additional test requirement for Band 41

In certain regions the following requirement may apply: For E-UTRA single RAT AAS BS operating in operating band 41 in the presence of an interfering signal according clause 6.7.4. In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the maximum levels for spurious emission, and operating band unwanted emission specified additionally for operating band 41single RAT E-UTRA operation. Also the ACLR requirements for same carrier type assumed in adjacent channels shall be fulfilled in the presence of the interfering signal.

##### 6.7.5.4.3	Intra-system test requirement

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the unwanted emission limits specified for operating band unwanted emission in clause 6.6.5 and ACLR in clause 6.6.3 in the presence of a wanted signal and an interfering signal according to according clause 6.7.4.

# 7	Conducted receiver characteristics

## 7.1	General

Unless otherwise stated, the receiver characteristics are specified at the AAS BS TAB connector with full complement of transceivers for the configuration in normal operating condition.

The manufacturer shall declare the minimum number of supported geographical cells (i.e. geographical areas) in clause 4.10. The minimum number of supported geographical cells (Ncells) relates to the AAS BS setting with minimum amount of cell splitting. The manufacturer shall also declare TAB connector RX min cell groups for this minimum number of cells configuration. Every TAB connector supporting reception in an operating band shall map to one TAB connector RX min cell group supporting the same. The mapping of TAB connectors to cells is implementation dependent.

The number of active receiver units that are considered when calculating the emission limit (NRXU,counted) for an AAS base station is calculated as follows:

-	NRXU,counted = min(NRXU,active, 8 ·Ncells) for E-UTRA single RAT AAS BS and MSR AAS BS (excluding UTRA only MSR AAS BS).

And

-	NRXU,counted = min(NRXU, active, 4 ·Ncells) for UTRA single RAT AAS BS and UTRA only MSR AAS BS.

Further:

-	NRXU,countedpercell = NRXU,counted/Ncells

NRXU,countedpercell is used for scaling the basic limits as described in clause 7.6.

NOTE:	NRXU,active is the number of actually active receiver units and is independent to the declaration of Ncells.

If a number of TAB connectors have been declared equivalent (see table 4.10-1, D6.70), only a representative one is necessary to demonstrate conformance.

In clause 7.6.5.1, if representative TAB connectors are used then per connector criteria (option 2) shall be applied.

Any receiver test requirement specified for Band 46 operation in TS 36.104 [4] for E-UTRA, or in TS 37.104 [5] for E-UTRA in MSR operation, and referred in clause 6, is not applicable for AAS BS. The requirements for co-location blocking for Band 46 are applicable for AAS BS.

Any receiver requirement specified for NB-IoT in-band, NB-IoT guard band, or standalone NB-IoT operation in TS 36.104 [4] for E-UTRA with NB-IoT (in-band or guard band) or for standalone NB-IoT, or in TS 37.104 [5] for E-UTRA with NB-IoT or standalone NB-IoT in MSR operation, and referred in clause 7, is not applicable for AAS BS.

## 7.2	Reference sensitivity level

### 7.2.1	Definition and applicability

The reference sensitivity power level PREFSENS is the minimum mean power received at the TAB connector at which a reference performance requirement shall be met for a specified reference measurement channel.

### 7.2.2	Minimum Requirement

The single RAT UTRA FDD AAS BS of Wide Area BS class shall fulfil minimum requirements for reference sensitivity specified in TS 25.104 [9], clause 7.2.1.

The single RAT UTRA FDD AAS BS of Medium Range BS class shall fulfil minimum requirements for reference sensitivity specified in TS 25.104 [9], clause 7.2.1.

The single RAT UTRA FDD AAS BS of Local Area BS class shall fulfil minimum requirements for reference sensitivity specified in TS 25.104 [9], clause 7.2.1.

The single RAT UTRA TDD AAS BS of Wide Area BS class shall fulfil minimum requirements for reference sensitivity specified in TS 25.105 [10], clause 7.2.1.1.

The single RAT UTRA TDD AAS BS of Local Area BS class shall fulfil minimum requirements for reference sensitivity specified in TS 25.105 [10], clause 7.2.1.1.

The single RAT E-UTRA AAS BS of Wide Area BS class shall fulfil minimum requirements for reference sensitivity specified in TS 36.104 [11], clause 7.2.1.

The single RAT E-UTRA AAS BS of Medium Range BS class shall fulfil minimum requirements for reference sensitivity specified in TS 36.104 [11], clause 7.2.1.

The single RAT E-UTRA AAS BS of Local Area BS class shall fulfil minimum requirements for reference sensitivity specified in TS 36.104 [11], clause 7.2.1.

### 7.2.3	Test Purpose

To verify that at each TAB connector the reference sensitivity level the performance requirements shall be met for a specified reference measurement channel.

### 7.2.4	Method of test

#### 7.2.4.1	Initial conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

On each of B, M and T, the test shall be performed under extreme power supply as defined in annex B.5.

NOTE:	Tests under extreme power supply also test extreme temperature.

#### 7.2.4.2	Procedure

The minimum requirement is applied to all TAB connectors, the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect TAB connector to measurement equipment as shown in clause D.2.1. All TAB connectors not under test shall be terminated.

2)	Set all TAB connectors declared in the same RAT and operating band to transmit a signal according to clause 4.12.2 at manufacturers declared rated output power PRated,c,TABC.

3)	Start the signal generator for the wanted signal to transmit:

-	12,2kbps DPCH with reference measurement channel defined in annex A in TS 25.141 [18] (PN-9 data sequence or longer) for UTRA FDD,

-	UL reference measurement channel (12.2 kbps) defined in clause A.2.1 in TS 25.142 [20] for UTRA TDD 1,28Mcps operation

-	The test signal mean power as specified in clause 7.2.5.3 for E-UTRA to the TAB connector.

4)	For UTRA FDD disable the TPC function.

5)	Set the signal generator for the wanted signal power as specified in clause 7.2.5.

6)	Measure:

-	BER according to annex C in TS 25.141 [18] for FDD UTRA.

-	BER  according to annex F in TS 25.142 [20] for TDD UTRA.

-	Throughput according to annex E in TS 36.141 [17] for E-UTRA.

In addition, for multi-band TAB connector(s), the following steps shall apply:

7)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.2.5	Test Requirements

#### 7.2.5.1	UTRA FDD operation

The BER measurement result in step 6 of 7.2.4.2 shall not be greater than the limit specified in table 7.2.5.1-1.

Table 7.2.5.1-1: Reference sensitivity levels

| BS class | Reference measurement channel data rate | reference sensitivity level (dBm) |  | BER |
| --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |
| Wide Area BS | 12.2 kbps | -120.3 | -120.0 | BER shall not exceed 0.001 |
| Medium Range BS | 12.2 kbps | -110.3 | -110.0 | BER shall not exceed 0.001 |
| Local Area BS | 12.2 kbps | -106.3 | -106.0 | BER shall not exceed 0.001 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.2.5.2	UTRA TDD 1,28 Mcps option operation

The BER measurement result in step 4 of 7.2.4.2.2 shall not be greater than the limit specified in table 7.2.5.1-1.

Table 7.2.5.2-1: Test Requirement for BS reference sensitivity level for 1,28 Mcps option

| BS class | Reference measurement channel data rate | reference sensitivity level | BER |
| --- | --- | --- | --- |
| Wide Area BS | 12.2 kbps | -109.3 dBm | BER shall not exceed 0.001 |
| Local Area BS | 12.2 kbps | -95.3 dBm | BER shall not exceed 0.001 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.2.5.3	E-UTRA operation

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in clause A.1 in TS 36.141 [17] with parameters specified in table 7.2.5.3-1 for Wide Area BS, in table 7.2.5.3-2 for Local Area BS, in table 7.2-3 and in table 7.2.5.3-3 for Medium Range BS.

Table 7.2.5.3-1: Wide Area BS class reference sensitivity levels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel (Note 2) | Reference sensitivity power level, PREFSENS (dBm) |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 1.4 | FRC A1-1 | -106.1 | -105.8 |
| 3 | FRC A1-2 | -102.3 | -102.0 |
| 5 | FRC A1-3 | -100.8 | -100.5 |
| 10 | FRC A1-3 (Note 1) | -100.8 | -100.5 |
| 15 | FRC A1-3 (Note 1) | -100.8 | -100.5 |
| 20 | FRC A1-3 (Note 1) | -100.8 | -100.5 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 in TS 36.141 [17] mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 2: FRC reference measurement channels as defined in annex A.1 in TS 36.141 [17]. |  |  |  |

Table 7.2.5.3-2: Local Area BS class, reference sensitivity levels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel (Note 2) | Reference sensitivity power level, PREFSENS (dBm) |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 1.4 | FRC A1-1 | -98.1 | -97.8 |
| 3 | FRC A1-2 | -94.3 | -94.0 |
| 5 | FRC A1-3 | -92.8 | -92.5 |
| 10 | FRC A1-3 (Note 1) | -92.8 | -92.5 |
| 15 | FRC A1-3 (Note 1) | -92.8 | -92.5 |
| 20 | FRC A1-3 (Note 1) | -92.8 | -92.5 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 in TS 36.141 [17] mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 2: FRC reference measurement channels as defined in annex A.1 in TS 36.141 [17]. |  |  |  |

Table 7.2.5.3-3: Medium Range BS class, reference sensitivity levels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel (Note 2) | Reference sensitivity power level, PREFSENS (dBm) |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 1.4 | FRC A1-1 | -101.1 | -100.8 |
| 3 | FRC A1-2 | -97.3 | -97.0 |
| 5 | FRC A1-3 | -95.8 | -95.5 |
| 10 | FRC A1-3 (Note 1) | -95.8 | -95.5 |
| 15 | FRC A1-3 (Note 1) | -95.8 | -95.5 |
| 20 | FRC A1-3 (Note 1) | -95.8 | -95.5 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 in TS 36.141 [17] mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 2: FRC reference measurement channels as defined in annex A.1 in TS 36.141 [17]. |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 7.3	Dynamic range

### 7.3.1	Definition and applicability

The dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal at the TAB connector inside the received channel bandwidth or the capability of receiving high level of wanted signal.

### 7.3.2	Minimum requirement

The single RAT UTRA FDD AAS BS of Wide Area BS class shall fulfil minimum requirements for dynamic range specified in TS 25.104 [9], clause 7.3.1.

The single RAT UTRA FDD AAS BS of Medium Range BS class shall fulfil minimum requirements for dynamic range specified in TS 25.104 [9], clause 7.3.1.

The single RAT UTRA FDD AAS BS of Local Area BS class shall fulfil minimum requirements for dynamic range specified in TS 25.104 [9], clause 7.3.1.

The single RAT UTRA TDD AAS BS of Wide Area BS class shall fulfil minimum requirements for dynamic range specified in TS 25.105 [10], clause 7.3.1.1.

The single RAT UTRA TDD AAS BS of Local Area BS class shall fulfil minimum requirements for dynamic range specified in TS 25.105 [10], clause 7.3.1.1.

The single RAT E-UTRA AAS BS of A Wide Area BS class shall fulfil minimum requirements for dynamic range specified in TS 36.104 [11], clause 7.3.1.

The single RAT E-UTRA AAS BS of Medium Range BS class shall fulfil minimum requirements for dynamic range specified in TS 36.104 [11], clause 7.3.1.

The single RAT E-UTRA AAS BS of Local Area BS class shall fulfil minimum requirements for dynamic range specified in TS 36.104 [11], clause 7.3.1.

### 7.3.3	Test purpose

To verify that at the dynamic range of the receiver unit associated with each TAB connector shall fulfil the specified limit.

### 7.3.4	Method of test

#### 7.3.4.1	Initial conditions

Test environment:

-	normal; see clause B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

#### 7.3.4.2	Procedure

The minimum requirement is applied to all TAB connectors, the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect TAB connector to measurement equipment as shown in annex D.2.1. All TAB connectors not under test shall be terminated.

2)	Set the signal generator for the wanted signal to transmit:

-	as specified in table 7.3.5.1-1 for UTRA.

-	as specified in table 7.3.5.1-2 for UTRA TDD 1,28Mcps operation.

-	as specified in table 7.3.5.1-3 to table 7.3.5.1-5  for E-UTRA.

3)	Set the Signal generator for the AWGN interfering signal at the same frequency as the wanted signal to transmit:

-	as specified in table 7.3.5.1-1 for UTRA.

-	as specified in table 7.3.5.1-2 for UTRA TDD 1,28Mcps operation.

-	as specified in table 7.3.5.1-3 to table 7.3.5.1-5  for E-UTRA.

4)	Measure:

-	BER according to annex C in TS 25.141 [18] for FDD UTRA.

-	BER according to annex F in TS 25.142 [20] for TDD UTRA.

-	Throughput according to annex E in TS 36.141 [17] for E-UTRA.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.3.5	Test requirements

#### 7.3.5.1	UTRA FDD operation

The BER shall not exceed 0,001 for the parameters specified in table 7.3.5.1-1.

Table 7.3.5.1-1: Dynamic range

| Parameter | Level Wide Area BS | Level Medium Range BS | Level Local Area BS | Unit |
| --- | --- | --- | --- | --- |
| Reference measurement channel data rate | 12,2 | 12.2 | 12.2 | Kbps |
| Wanted signal mean power | -89.8 | -79.8 | -75.8 | dBm |
| Interfering AWGN signal | -73 | -63 | -59 | dBm/3.84 MHz |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.3.5.2	UTRA TDD 1,28 Mcps option operation

The BER shall not exceed 0,001 for the parameters specified in table 7.3.5.2-1.

Table 7.3.5.2-1: Test Requirements for Dynamic Range for 1,28 Mcps TDD option

| Parameter |  | Level | Unit |
| --- | --- | --- | --- |
| Reference measurement channel data rate |  | 12,2 | kbit/s |
| Wanted signal | Wide Area BS | -78,8 | dBm |
| mean power | Local Area BS | -64,8 | dBm |
| Interfering | Wide Area BS | -76 | dBm/1,28 MHz |
| AWGN signal | Local Area BS | -62 | dBm/1,28 MHz |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.3.5.3	E-UTRA operation

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A in TS 36.141 [17] with parameters specified in table 7.3.5.3-1 for an AAS BS of Wide Area BS class, in Table7.3.5.3-2 for an AAS BS of Local Area BS class and in table 7.3.5.3-3 for AAS BS of Medium Range BS class.

Table 7.3.5.3-1: AAS BS of Wide Area BS class dynamic range

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in annex A.2 | -76.0 | -88.7 | AWGN |
| 3 | FRC A2-2 in annex A.2 | -72.1 | -84.7 | AWGN |
| 5 | FRC A2-3 in annex A.2 | -69.9 | -82.5 | AWGN |
| 10 | FRC A2-3 in annex A.2 (Note) | -69.9 | -79.5 | AWGN |
| 15 | FRC A2-3 in annex A.2 (Note) | -69.9 | -77.7 | AWGN |
| 20 | FRC A2-3 in annex A.2 (Note) | -69.9 | -76.4 | AWGN |
| NOTE: The wanted signal mean power is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A2-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |  |

Table 7.3.5.3-2: AAS BS of Local Area BS class  dynamic range

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in annex A.2 | -68.0 | -80.7 | AWGN |
| 3 | FRC A2-2 in annex A.2 | -64.1 | -76.7 | AWGN |
| 5 | FRC A2-3 in annex A.2 | -61.9 | -74.5 | AWGN |
| 10 | FRC A2-3 in annex A.2 (Note) | -61.9 | -71.5 | AWGN |
| 15 | FRC A2-3 in annex A.2 (Note) | -61.9 | -69.7 | AWGN |
| 20 | FRC A2-3 in annex A.2 (Note) | -61.9 | -68.4 | AWGN |
| NOTE: The wanted signal mean power is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A2-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |  |

Table 7.3.5.3-3: AAS BS of Medium Range BS class dynamic range

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in annex A.2 | -71.0 | -83.7 | AWGN |
| 3 | FRC A2-2 in annex A.2 | -67.1 | -79.7 | AWGN |
| 5 | FRC A2-3 in annex A.2 | -64.9 | -77.5 | AWGN |
| 10 | FRC A2-3 in annex A.2 (Note) | -64.9 | -74.5 | AWGN |
| 15 | FRC A2-3 in annex A.2 (Note) | -64.9 | -72.7 | AWGN |
| 20 | FRC A2-3 in annex A.2 (Note) | -64.9 | -71.4 | AWGN |
| NOTE: The wanted signal mean power is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A2-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 7.4	Adjacent channel selectivity and narrowband blocking

### 7.4.1	Definition and applicability

The adjacent channel selectivity, general blocking and narrowband blocking characteristics are measures of the receiver unit ability to receive a wanted signal at its assigned channel at the TAB connector in the presence of an unwanted interferer inside the operating band.

NOTE	For Single RAT requirements, the in-band selectivity characteristics is referred to as "adjacent channel selectivity", whereas for the MSR requirements, the corresponding property is referred to as "general blocking" since the adjacent frequency range may not carry a channel addressable from the interfered carrier.

### 7.4.2	Minimum requirement

The minimum requirement for MSR operation is in TS 37.105 [8], clause 7.4.2.

The single RAT UTRA FDD AAS BS of Wide Area BS class shall fulfil minimum requirements for adjacent channel selectivity and narrow-band blocking specified in TS 25.104 [9], clause 7.4.

The single RAT UTRA FDD AAS BS of Medium Range BS class shall fulfil minimum requirements for adjacent channel selectivity and narrow-band blocking specified in TS 25.104 [9], clause 7.4.

The single RAT UTRA FDD Local Area BS class shall fulfil minimum requirements for adjacent channel selectivity and narrow-band blocking specified in TS 25.104 [9], clause 7.4.

The single RAT UTRA TDD AAS BS of Wide Area BS class shall fulfil minimum requirements for adjacent channel selectivity and narrow-band blocking specified in TS 25.105 [10], clause 7.4.

The single RAT UTRA TDD AAS BS of Local Area BS class shall fulfil minimum requirements for adjacent channel selectivity and narrow-band blocking specified in TS 25.105 [10], clause 7.4.

The single RAT E-UTRA AAS BS of Wide Area BS class shall fulfil minimum requirements for adjacent channel selectivity and narrow-band blocking specified in TS 36.104 [11], clause 7.5.

The single RAT E-UTRA AAS BS of Medium Range BS class shall fulfil minimum requirements for adjacent channel selectivity and narrow-band blocking specified in TS 36.104 [11], clause 7.5.

The single RAT E-UTRA AAS BS of Local Area BS class shall fulfil minimum requirements for adjacent channel selectivity and narrow-band blocking specified in TS 36.104 [11], clause 7.5.

# 7.4.3	Test purpose

The test stresses the receiver unit ability to withstand high-level interference from unwanted signals at specified frequency offsets at the TAB connector without undue degradation of its sensitivity.

### 7.4.4	Method of test

#### 7.4.4.1	Initial conditions

Test environment:

-	Normal; see clause B.2.

RF channels to be tested for single carrier (SC):

-	B, M and T; see clause 4.12.1

Base Station RF Bandwidth positions to be tested for multi-carrier (MC):

-	MRFBW for single-band TAB connector(s), see clause 4.12.1, BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band TAB connector(s), see clause 4.12.1.

#### 7.4.4.2	Procedure

##### 7.4.4.2.1	General procedure

The general procedure steps apply to the procedures for all the RATs.

The minimum requirement is applied to all TAB connectors, the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect TAB connector to measurement equipment as shown in annex D.2.3. All TAB connectors not under test shall be terminated.

2)	Generate the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the TAB connector under test as follows:

-	For E-UTRA see clause A.1 in TS 36.141 [17].

-	For UTRA FDD see clause A.2 in TS 25.141 [18].

-	For UTRA TDD see clause A.2.1 in TS 25.142 [20].

3)	Set the transmitter unit associated with the TAB connector under test to transmit with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

##### 7.4.4.2.2	MSR operation

# 7.4.4.2.2.1	Procedure for general blocking

1)	Adjust the signal generators to the type of interfering signal, levels and the frequency offsets as specified in table 7.4.5.1.1-1.

2)	The interfering signal shall be swept with a step size of 1 MHz starting from the minimum offset to the channel edges of the wanted signals as specified in table 7.4.5.1.1-1.

3)	Measure the performance of the wanted signal at the receiver unit associated with the TAB connector under test, as defined in clause 7.4.5.1, for the relevant carriers specified by the test configuration in clause 4.11.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 7.4.4.2.2.2	Procedure for narrowband blocking

1)	Adjust the signal generators to the type of interfering signal, levels and the frequency offsets as specified in table 7.4.5.1.2-1.

2)	Set-up and sweep the interfering RB centre frequency offset to the channel edge of the wanted signal according to table 7.4.5.1.2-1.

3)	Measure the performance of the wanted signal at the receiver unit associated with the TAB connector under test, as defined in clause 7.4.5.1, for the relevant carriers specified by the test configuration in clause 4.11.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 7.4.4.2.2.3	Procedure for additional BC3 blocking requirement

1)	Adjust the signal generators to the type of interfering signal, levels and the frequency offsets as specified in table 7.4.5.5-1.

2)	Measure the performance of the wanted signal at the receiver unit associated with the TAB connector under test, as defined in clause 7.4.5, for the relevant carriers specified by the test configuration in clause 4.8.

##### 7.4.4.2.3	Single RAT UTRA FDD operation

1)	Generate the wanted signal and adjust the ATT1 to set the input level to the base station under test to the level specified in table 7.4.5.2-1 For a TAB connector supporting multi-carrier operation, generate the wanted signal according to the applicable test configuration (see clause 4.11) using applicable reference measurement channel to the TAB connector under test. Power settings are specified in table 7.4.5.2-1.

2)	Set-up the interfering signal at the adjacent channel frequency and adjust the ATT2 to obtain the specified level of interfering signal at the base station input defined in table 7.4.5.2-1. Note that the interfering signal shall have an ACLR of at least 63 dB in order to eliminate the impact of interfering signal adjacent channel leakage power on the ACS measurement.

3)	Measure the BER of the wanted signal at the receiver unit associated with the TAB connecter under test.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.4.4.2.4	Single RAT UTRA TDD 1,28Mcps option operation

1)	Generate the wanted signal according to the test configurations in clause 4.11 and adjust the input level to the TAB connector under test according to table 7.4.5.3-1. The UL reference measurement channel (12,2 kbps) defined in clause A.2.1 in TS 25.142 [20] shall be used for each wanted carrier.

2)	Set-up the interfering signal at the adjacent channel frequency and adjust the interfering signal level at the TAB connector according to table 7.4.5.3-1. The interfering signal is equivalent to a continuous CDMA signal with one code of chip frequency 1,28 Mchip/s, filtered by an RRC transmit pulse-shaping filter with roll-off  = 0,22.

3)	Measure the BER of the wanted signal at the receiver unit associated with the TAB connecter under test.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.4.4.2.5	Single RAT E-UTRA operation

# 7.4.4.2.5.1	Procedure for adjacent channel selectivity

1)	Generate the wanted signal using the applicable test configuration specified in clause 5.3.4 and adjust the input level to the TAB connector under test to the level specified in table 7.4.5.4-1 for the appropriate BS class.

2)	Set-up the interfering signal at the adjacent channel frequency and adjust the interfering signal level at the TAB connector under test to the level defined in table 7.4.5.4-1 for the appropriate BS class.

3)	Measure the throughput, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clause 5.3.4.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

# 7.4.4.2.5.2	Procedure for narrow-band blocking

1)	For TAB connector operating E-UTRA FDD declared to be capable of single carrier operation only in the operating band, set the transmitter unit associated with the TAB connector under test to transmit according to clause 4.12.2 at manufacturers declared rated output power PRated,c,TABC.

For a TAB connector operating E-UTRA FDD declared to be capable of multi-carrier and/or CA operation in the operating band, set the transmitter unit associated with the TAB connector under test to transmit according to clause 4.12.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 5.3.4.

2)	Generate the wanted signal using the applicable test configuration specified in clause 5.3.4 and adjust the input level to the TAB connector under test to the level specified in table 7.4.5.4-1.

3)	Adjust the interfering signal level at the TAB connector input to the level defined in table 7.4.5.4-1. Set-up and sweep the interfering RB centre frequency offset to the channel edge of the wanted signal according to table 7.4.5.4-2.

4)	Measure the throughput, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clause 5.3.4.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.4.5	Test requirements

#### 7.4.5.1	MSR operation

##### 7.4.5.1.1	General blocking test requirement

For the general blocking requirement, the interfering signal shall be a UTRA FDD signal as specified in clause A.1 in TS 25.141 [18].

For TAB connector operating in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band TAB connector the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 15 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the TAB connector, using the parameters in tables 7.4.5.1.1-1 and 7.4.5.1.1-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.1.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

For a multi-band TAB connector, the requirement applies according to table 7.4.5.1-1 for the in-band blocking frequency ranges of each supported operating band.

Table 7.4.5.1.1-1: General blocking requirement

| Base Station class | Mean power of interfering signal (dBm) | Wanted Signal mean power (dBm) (Note 1) | Centre Frequency of Interfering Signal | Interfering signal centre frequency minimum frequency offset from the Base Station RF Bandwidth edge or sub-block edge inside a gap (MHz) |
| --- | --- | --- | --- | --- |
| Wide Area BS | -40 | PREFSENS + x dB (Note 2) |  |  |
| Medium Range BS | -35 | PREFSENS + x dB (Note 3) | See Table 7.4.5.1-2 | ±7.5 |
| Local Area BS | -30 | PREFSENS + x dB (Note 4) |  |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104 [12]. NOTE 2: For AAS BS of wide area class "x" is equal to 6 in case of E-UTRA or UTRA wanted signals. NOTE 3: For AAS BS of medium range class, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA wanted signal. NOTE 4: For AAS BS of local area class, "x" is equal to 11 in case of E-UTRA wanted signal, 6 in case of UTRA wanted signal. NOTE 5: For a multi-band TAB connector, "x" in Note 2, 3, 4 apply in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present, or in an adjacent or overlapping band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. |  |  |  |  |

Table 7.4.5.1.1-2: Interfering signal for the general blocking requirement

| Operating Band Number | Centre Frequency of Interfering Signal (MHz) |  |  |
| --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18, 19, 21-23, 24, 27, 30, 33-45, 65, 66, 68 | (FUL_low -20) | to | (FUL_high +20) |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) |
| 12 | (FUL_low -20) | to | (FUL_high +13) |
| 17 | (FUL_low -20) | to | (FUL_high +18) |
| 20 | (FUL_low -11) | to | (FUL_high +20) |
| 25 | (FUL_low -20) | to | (FUL_high +15) |
| 31 | (FUL_low -20) | to | (FUL_high +5) |

NOTE:	The requirement in tables 7.4.5.1.1-1 and 7.4.5.1.1-2 assumes that two operating bands, where the downlink operating band (see table 4.4-1 and table 4.4-2 in TS 37.141 [16].) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

##### 7.4.5.1.2	General narrowband blocking test requirement

For the narrowband blocking requirement, the interfering signal shall be an E-UTRA 1RB signal as specified in clause A.3 in TS 37.141 [16].

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For a TAB connector operating in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 3 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least 3 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the TAB connector, using the parameters in table 7.4.5.1.2-1 the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.1.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

Table 7.4.5.1.2-1: Narrowband blocking requirement

| Base Station Class | RAT of the carrier | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering RB (Note 3) centre frequency offset from the Base Station RF Bandwidth edge or sub-block edge inside a gap (kHz) |
| --- | --- | --- | --- | --- |
| Wide Area BS |  |  | -49 |  |
| Medium Range BS | E-UTRA and UTRA | PREFSENS + x dB (Note 2) | -44 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 |
| Local Area BS |  |  | -41 |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104 [12]. NOTE 2: "x" is equal to 6 in case of E-UTRA or UTRA wanted signals. NOTE 3: Interfering signal (E-UTRA 3 MHz) consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. |  |  |  |  |

##### 7.4.5.1.3	Additional BC3 blocking test requirement

The interfering signal is a 1,28Mcps UTRA TDD modulated signal as specified in clause A.2 in TS 37.141 [16].

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For a multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 4.8MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the TAB connector, using the parameters in table 7.4.5.1.3-1, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

Table 7.4.5.1.3-1: Additional blocking requirement for Band Category 3

| Operating Band | Centre Frequency of Interfering Signal (MHz) |  |  | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Interfering signal centre frequency minimum frequency offset from the Base Station RF Bandwidth edge (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 33 - 40 | (FUL_low - 20) | to | (FUL_high + 20) | -40, | PREFSENS + 6 dB (Note) | ±2.4 |
| NOTE: PREFSENS depends on the RAT and on the channel bandwidth, see clause 7.2. |  |  |  |  |  |  |

#### 7.4.5.2	Single RAT UTRA FDD operation

For each measured carrier, the BER shall not exceed 0,001 for the parameters specified in table 7.4.5.2-1.

For multi-carrier TAB connector the ACS requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the lower/upper Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For a TAB connector operating in non-contiguous spectrum within any operating band, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 5 MHz. The interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -2.5 MHz/+2.5 MHz, respectively.

For a multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 5 MHz. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -2.5 MHz/+2.5 MHz, respectively.

Table 7.4.5.2-1: Adjacent channel selectivity

| Parameter | Level Wide Area BS | Level Medium Range BS | Level Local Area BS | Unit |
| --- | --- | --- | --- | --- |
| Reference measurement channel data rate | 12.2 | 12.2 | 12.2 | kbps |
| Wanted signal mean power | -115 | -105 | -101 | dBm |
| Interfering signal mean power | -52 | -42 | -38 | dBm |
| Fuw (Modulated) | 5 | 5 | 5 | MHz |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.4.5.3	Single RAT UTRA TDD 1,28 Mcps option operation

The BER, measured on the wanted signal in the presence of an interfering signal, shall not exceed 0,001 for the parameters specified in table 7.4.5.3-1.

The ACS requirement is always applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth edges. The interfering signal offset is defined relative to the lower / upper Base Station RF Bandwidth  edges or Maximum Radio Bandwidth edges.

For multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap as long as the Inter RF Bandwidth gap size is at least 1.6MHz. The interfering signal offset is defined relative to the lower / upper  Base Station RF Bandwidth  edges inside the Inter RF Bandwidth gap and is equal to -0.8MHz/+0.8MHz, respectively.

Table 7.4.5.3-1: Parameters of the wanted signal and the interfering signal
for ACS testing for 1,28 Mcps TDD

| Parameter |  |  | Level | Unit |
| --- | --- | --- | --- | --- |
| Reference measurement channel data rate |  |  | 12,2 | kbit/s |
| Wanted signal | Wide Area BS |  | -104 | dBm |
| mean power | Local Area BS |  | -90 | dBm |
| Interfering signal |  | Wide Area BS | -55 | dBm |
| mean power |  | Local Area BS | -41 | dBm |
| Fuw (modulated) |  |  | ±1,6 | MHz |
| NOTE: Fuw is the frequency offset of the unwanted interfering signal from the assigned channel frequency of the wanted signal. |  |  |  |  |

#### 7.4.5.4	Single RAT E-UTRA operation

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For AAS BS of wide area BS class the wanted and the interfering signal coupled to the TAB connector are specified in table 7.4.5.4-1 and 7.4.5.4-2 for narrowband blocking and 7.4.5.4-3 for ACS. The reference measurement channel for the wanted signal is specified in table 7.2.5.3-1for each channel bandwidth and further specified in annex A in TS 36.141 [17].

For AAS BS of Medium Range BS class, the wanted and the interfering signal coupled to the TAB connector are specified in tables 7.4.5.4-1 and 7.4.5.4-2 for narrowband blocking and in table 7.4.5.4-5 for ACS. The reference measurement channel for the wanted signal is specified in table 7.2.5.3-3 for each channel bandwidth and further specified in annex A in TS 36.141 [17].

For AAS BS of Local Area BS class, the wanted and the interfering signal coupled to the TAB connector are specified in tables 7.4.5.4-1 and 7.4.5.4-2 for narrowband blocking and 7.4.5.4-4 for ACS. The reference measurement channel for the wanted signal is specified in table 7.2.5.3-2 for each channel bandwidth and further specified in annex A in TS 36.141 [17].

The ACS and narrowband blocking requirement is always applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For a TAB connector operating in non-contiguous spectrum within any operating band, the ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the E-UTRA interfering signal in tables 7.4.5.4-3, 7.4.5.4-4 and 7.4.5.4-5. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band TAB connector, the ACS requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the E-UTRA interfering signal in tables 7.4.5.4-3, 7.4.5.4-4 and 7.4.5.4-5. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For a TAB connector operating in non-contiguous spectrum within any operating band, the narrowband blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.4.5.4-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band TAB connector, the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the E-UTRA interfering signal in table 7.4.5.4-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.4.5.4-1: Narrowband blocking requirement

|  | Wanted signal mean power (dBm) (Note) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS + 6 dB | -49 | See table 7.4.5.4-2 |
| Medium Range BS | PREFSENS + 6 dB | -44 | See table 7.4.5.4-2 |
| Local Area BS | PREFSENS +6 dB | -41 | See table 7.4.5.4-2 |
| NOTE: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. |  |  |  |

Table 7.4.5.4-2: Interfering signal for Narrowband blocking requirement

| E-UTRA channel BW of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) | Type of interfering signal (Note) |
| --- | --- | --- |
| 1.4 | ±(252.5+m*180), m=0, 1, 2, 3, 4, 5 | 1.4 MHz E-UTRA signal, 1 RB |
| 3 | ±(247.5+m*180), m=0, 1, 2, 3, 4, 7, 10, 13 | 3 MHz E-UTRA signal, 1 RB |
| 5 | ±(342.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB |
| 10 | ±(347.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB |
| 15 | ±(352.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB |
| 20 | ±(342.5+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz E-UTRA signal, 1 RB |
| NOTE: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |

Table 7.4.5.4-3: Adjacent channel selectivity for Wide Area BS

| E-UTRA channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note) | Interfering signal mean power (dBm) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 11 dB | -52 | ±0.7025 | 1.4 MHz E-UTRA signal |
| 3 | PREFSENS + 8 dB | -52 | ±1.5075 | 3 MHz E-UTRA signal |
| 5 | PREFSENS + 6 d | -52 | ±2.5025 | 5 MHz E-UTRA signal |
| 10 | PREFSENS + 6 dB | -52 | ±2.5075 | 5 MHz E-UTRA signal |
| 15 | PREFSENS + 6 dB | -52 | ±2.5125 | 5 MHz E-UTRA signal |
| 20 | PREFSENS + 6 dB | -52 | ±2.5025 | 5 MHz E-UTRA signal |
| NOTE: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. |  |  |  |  |

Table 7.4.5.4-4: Adjacent channel selectivity for Local Area BS

| E-UTRA channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note) | Interfering signal mean power (dBm) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 11 dB | -44 | ±0.7025 | 1.4 MHz E-UTRA signal |
| 3 | PREFSENS + 8 dB | -44 | ±1.5075 | 3 MHz E-UTRA signal |
| 5 | PREFSENS + 6 dB | -44 | ±2.5025 | 5 MHz E-UTRA signal |
| 10 | PREFSENS + 6 dB | -44 | ±2.5075 | 5 MHz E-UTRA signal |
| 15 | PREFSENS + 6 dB | -44 | ±2.5125 | 5 MHz E-UTRA signal |
| 20 | PREFSENS + 6 dB | -44 | ±2.5025 | 5 MHz E-UTRA signal |
| NOTE: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. |  |  |  |  |

Table 7.4.5.4-5: Adjacent channel selectivity for Medium Range BS

| E-UTRA channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 11 dB | -47 | ±0.7025 | 1.4 MHz E-UTRA signal |
| 3 | PREFSENS + 8 dB | -47 | ±1.5075 | 3 MHz E-UTRA signal |
| 5 | PREFSENS + 6 dB | -47 | ±2.5025 | 5 MHz E-UTRA signal |
| 10 | PREFSENS + 6 dB | -47 | ±2.5075 | 5 MHz E-UTRA signal |
| 15 | PREFSENS + 6 dB | -47 | ±2.5125 | 5 MHz E-UTRA signal |
| 20 | PREFSENS + 6 dB | -47 | ±2.5025 | 5 MHz E-UTRA signal |
| NOTE: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 7.5	Blocking

### 7.5.1	Definition and applicability

The out-of-band blocking characteristic is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer outside the uplink operating band.

### 7.5.2	Minimum requirement

The minimum requirement for MSR operation is in TS 37.105 [8], clause 7.5.2.

The minimum requirement for single RAT UTRA operation is in TS 37.105 [8], clause 7.5.3.

The minimum requirement for single RAT E-UTRA operation is in TS 37.105 [8], clause 7.5.4.

### 7.5.3	Test purpose

The test stresses the ability of the receiver unit associated with the TAB connector under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

### 7.5.4	Method of test

#### 7.5.4.1	Initial conditions

Test environment:

-	Normal; see clause B.2.

RF channels to be tested for single carrier (SC):

-	M; see clause 4.12.1

Base Station RF Bandwidth positions to be tested for multi-carrier (MC):

-	MRFBW for single-band TAB connector(s), see clause 4.12.1, BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band TAB connector(s), see clause 4.12.1.

In addition, for multi-band TAB connectors:

-	For BRFBW_T'RFBW, out-of-band blocking testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, out-of-band blocking testing below the lowest operating band may be omitted.

#### 7.5.4.2	Procedure

##### 7.5.4.2.1	General Procedure

The general procedure steps apply to the procedures for all the RATs.

The minimum requirement is applied to all TAB connectors, the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1:

1)	Connect TAB connector to measurement equipment as shown in annex D.2.3. All TAB connectors not under test shall be terminated.

##### 7.5.4.2.2	MSR operation

1)	Generate the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the TAB connector under test as follows:

-	For E-UTRA see clause A.1 in TS 36.141 [17].

-	For UTRA FDD see clause A.2 in TS 25.141 [18].

-	For UTRA TDD see clause A.2.1 in TS 25.142 [20].

2)	Set the transmitter unit associated with the TAB connector under test to transmit with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

The transmitter unit associated with the TAB connector under test may be turned off for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

3)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.5.5.1.1-1 and, when applicable, for co-location test requirements in table 7.5.5.1.2-1.

4)	The CW interfering signal shall be swept with a step size of 1 MHz within the specified range.

5)	Measure the performance of the wanted signal at the receiver unit associated with the TAB connector, as defined in the clause 7.5.5, for the relevant carriers specified by the test configuration in clause 4.11.

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.5.4.2.3	Single RAT UTRA FDD operation

1)	Generate the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the TAB connector under test as shown in clause A.2.1 in TS 25.141 [18].

2)	Set the transmitter unit associated with the TAB connector under test to transmit with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

The transmitter unit associated with the TAB connector under test may be turned off for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

3)	Adjust the signal generators to the type of interfering signals and the frequency offsets as specified in tables 7.5.5.2-1 to 7.5.5.2-1 . Note that the GMSK modulated interfering signal shall have an ACLR of at least 72 dB in order to eliminate the impact of interfering signal adjacent channel leakage power on the blocking characteristics measurement. For the tests defined in tables 7.5.5.2-1 to 7.5.5.2-3, the interfering signal shall be at a frequency offset Fuw from the assigned channel frequency of the wanted signal which is given by:

Fuw =  (n x 1 MHz),

where n shall be increased in integer steps from n = 10 up to such a value that the centre frequency of the interfering signal covers the range from 1 MHz to 12,75 GHz.

4)	Measure the BER of the wanted signal at the receiver unit associated with the TAB connecter under test.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.5.4.2.4	Single RAT UTRA TDD 1,28 Mcps option operation

1)	Generate the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the TAB connector under test as shown in clause A.2.1 in TS 25.142 [20].

2)	Set the transmitter unit associated with the TAB connector under test to transmit with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

The transmitter unit associated with the TAB connector under test may be turned off for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

3)	Set the signal generator to produce an interfering signal at a frequency offset Fuw from the assigned channel frequency of the wanted signal which is given by

Fuw=3,2 +/- n) x 1 MHz

where n shall be increased in integer steps from n = 0 up to such a value that the centre frequency of the interfering signal covers the range from 1 MHz to 12,75 GHz. The interfering signal level measured at the TAB connector shall be set in dependency of its centre frequency, as specified in tables 7.5.5.3.1-1 to 7.5.5.3.1-2 and 7.5.5.3.2-1. The type of the interfering signal is either equivalent to a continuous wideband CDMA signal with one code of chip frequency 1,28 Mcps, filtered by an RRC transmit pulse-shaping filter with roll-off  = 0,22, or a CW signal; see tables 7.5.5.3.1-1 to 7.5.5.3.1-2 and 7.5.5.3.2-1.

4)	Measure the BER of the wanted signal at the receiver unit associated with the TAB connecter under test.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.5.4.2.5	Single RAT E-UTRA operation

1)	Generate the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the TAB connector under test as shown in clause A.1 in TS 36.141 [17].

2)	Set the transmitter unit associated with the TAB connector under test to transmit with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

The transmitter unit associated with the TAB connector under test may be turned off for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

3)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.5.5.4.1-1 and Table 7.5.5.4.2-1 for AAS BS of wide area BS class, tables 7.5.5.4.1-2 and 7.5.5.4.2-2 for AAS BS of local area BS class and tables 7.5.5.4.1-3 and 7.5.5.4.2-3 for AAS BS of medium range BS class.

4)	The CW interfering signal shall be swept with a step size of 1 MHz within the specified range.

5)	Measure the performance of the wanted signal at the receiver unit associated with the TAB connector, as defined in the clause 7.5.5, for the relevant carriers specified by the test configuration in clause 4.11.

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.5.5	Test requirements

#### 7.5.5.1	MSR operation

##### 7.5.5.1.1	General out-of-band blocking test requirements

For a wanted and an interfering signal coupled to a TAB connector using the parameters in table 7.5.5.1.1-1, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.1.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

For a multi-band TAB connector, the requirement applies for each supported operating band. The in-band blocking frequency ranges of all supported operating bands according to table 7.4.5.1.1-2 shall be excluded from the requirement.

Table 7.5.5.1.1-1: Blocking performance requirement

| Operating Band Number | Centre Frequency of Interfering Signal (MHz) |  |  | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) (Note) | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18, 19, 21-23, 24, 27, 30, 33-45, 48, 65, 66, 68, 70 | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS + x dB | CW carrier |
| 8, 26, 28 | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS + x dB | CW carrier |
| 12 | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS + x dB | CW carrier |
| 17 | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS + x dB | CW carrier |
| 20 | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS + x dB | CW carrier |
| 25 | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 | PREFSENS + x dB | CW carrier |
| 31 | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS + x dB | CW carrier |
| NOTE: PREFSENS depends on the RAT, the BS class and the channel bandwidth, see clause 7.2. "x" is equal to 6 dB in case of E-UTRA or UTRA wanted signals. |  |  |  |  |  |  |

##### 7.5.5.1.2	Co-location test requirements

This additional blocking requirement may be applied for the protection of receiver units associated with TAB connectors when a E-UTRA, UTRA, CDMA or GSM/EDGE BS operating in a different frequency band are co-located with the AAS BS.

The requirements in this clause assume a 30 dB coupling loss between the interfering transmitter and the TAB connector and are based on co-location with base stations of the same class.

For a wanted and an interfering signal coupled to TAB connector using the parameters in table 7.5.5.1.2-1, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.1.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

Table 7.5.5.1.2-1: Blocking requirement for co-location with BS in other frequency bands

| Type of co-located BS |  | Centre Frequency of Interfering Signal (MHz) |  | Interfering Signal mean power for WA BS (dBm) |  | Interfering Signal mean power for MR BS (dBm) |  | Interfering Signal mean power for LA BS (dBm) |  | Wanted Signal mean power (dBm) (Note 1) |  | Type of Interfering Signal |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 |  | 869 - 894 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| GSM900 |  | 921 - 960 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| DCS1800 |  | 1805 - 1880 (Note 4) |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| PCS1900 |  | 1930 - 1990 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band I or E-UTRA Band 1 |  | 2110 - 2170 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band II or E-UTRA Band 2 |  | 1930 - 1990 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band III or E-UTRA Band 3 |  | 1805 - 1880 (Note 4) |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band IV or E-UTRA Band 4 |  | 2110 - 2155 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band V or E-UTRA Band 5 |  | 869 - 894 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band VI or E-UTRA Band 6 |  | 875 - 885 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band VII or E-UTRA Band 7 |  | 2620 - 2690 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band VIII or E-UTRA Band 8 |  | 925 - 960 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band IX or E-UTRA Band 9 |  | 1844.9 - 1879.9 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band X or E-UTRA Band 10 |  | 2110 - 2170 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XI or E-UTRA Band 11 |  | 1475.9 - 1495.9 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XII or E-UTRA Band 12 |  | 729 - 746 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XIIII or E-UTRA Band 13 |  | 746 - 756 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XIV or E-UTRA Band 14 |  | 758 - 768 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 17 |  | 734 - 746 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 18 |  | 860 - 875 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XIX or E-UTRA Band 19 |  | 875 - 890 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XX or E-UTRA Band 20 |  | 791 - 821 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XXI or E-UTRA Band 21 |  | 1495.9 - 1510.9 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XXII or E-UTRA Band 22 |  | 3510 - 3590 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 23 |  | 2180 - 2200 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 24 |  | 1525 - 1559 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XXV or E-UTRA Band 25 |  | 1930 - 1995 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 |  | 859 - 894 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 27 |  | 852 - 869 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 28 |  | 758 - 803 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 29 |  | 717 - 728 |  | +16 |  | +8 |  | -6 |  | PREFSENS + 6 dB |  | CW carrier |  |
| E-UTRA Band 30 |  | 2350 - 2360 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 31 |  | 462.5 - 467.5 |  | +16 |  | +8 |  | -6 |  | PREFSENS + 6 dB |  | CW carrier |  |  |
| UTRA FDD Band XXXII or E-UTRA Band 32 |  | 1452 - 1496 (Note 5) |  | +16 |  | +8 |  | -6 |  | PREFSENS + 6 dB |  | CW carrier |  |  |
| UTRA TDD Band a) or E-UTRA Band 33 |  | 1900-1920 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA TDD Band a) or E-UTRA Band 34 |  | 2010-2025 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA TDD Band b) or E-UTRA Band 35 |  | 1850-1910 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA TDD Band b) or E-UTRA Band 36 |  | 1930-1990 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA TDD Band c) or E-UTRA Band 37 |  | 1910-1930 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA TDD Band d) or E-UTRA Band 38 |  | 2570-2620 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA TDD Band f) or E-UTRA Band 39 |  | 1880-1920 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| UTRA TDD Band e) or E-UTRA Band 40 |  | 2300-2400 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 41 |  | 2496 - 2690 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 42 |  | 3400 - 3600 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 43 |  | 3600 - 3800 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 44 |  | 703 - 803 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 45 |  | 1447 - 1467 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 46 |  | 5150 - 5925 |  | N/A |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 48 |  | 3550 - 3700 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 65 |  | 2110 - 2200 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 66 |  | 2110 - 2200 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 67 |  | 738 - 758 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 68 |  | 753 - 783 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB* |  | CW carrier |  |  |
| E-UTRA Band 69 |  | 2570 - 2620 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| E-UTRA Band 70 |  | 1995 - 2020 |  | +16 |  | +8 |  | -6 |  | PREFSENS + x dB |  | CW carrier |  |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and the channel bandwidth, see clause 7.2. "x" is equal to 6 in case of UTRA or E-UTRA wanted signals. NOTE 2: Except for a TAB connector operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a TAB connector operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797MHz. NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805-1850 MHz. NOTE 5: For a TAB connector operating in band 11 or 21, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 6: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 7.5.5.2	Single RAT UTRA FDD operation

For each measured carrier, the BER shall not exceed 0.001 for the parameters specified in tables 7.5.5.2-1 to 7.5.5.2-9 if applicable for the TAB connector under test.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the lower/upper Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For a TAB connector operating in non-contiguous spectrum within any operating band, the blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15 MHz. The interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -7.5 MHz/+7.5 MHz, respectively.

For a TAB connector operating in non-contiguous spectrum within any operating band, the narrowband blocking requirements in tables 7.4R-7.4T apply in addition inside any sub-block gap, in case the sub-block gap size is at least 400 kHz or 600 kHz, depending on the operating band. The interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -200 kHz/+200 kHz or -300 kHz/+300 kHz, respectively.

For a multi-band TAB connector, the requirement in the in-band blocking frequency range applies for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 15 MHz. The interfering signal offset is defined relative to lower/upper Base Station RF bandwidth edges inside the Inter RF Bandwidth gap and is equal to -7.5 MHz/+7.5 MHz, respectively.

For a multi-band TAB connector, the requirement in the out-of-band blocking frequency ranges apply for each operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to tables 7.4K - 7.4M shall be excluded from the out-of-band blocking requirement.

For a multi-band TAB connector, the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 400 kHz or 600 kHz, depending on the operating band. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -200 kHz/+200 kHz or -300 kHz/+300 kHz, respectively.

Table 7.5.5.2-1: Blocking characteristics for Wide Area BS

| Operating Band | Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| V | 824-849 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 804-824 MHz 849-869 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 860 - 880 MHz 915 - 925 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 679 - 699 MHz 716 - 729 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 679 MHz 729 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 757 - 777 MHz 787 - 807 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 768 - 788 MHz 798 - 818 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 810 - 830 MHz 845 - 865 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 821 - 832 MHz 862 - 882 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 794-814 MHz 849-859 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| NOTE 1: The characteristics of the W-CDMA interfering signal are specified in annex I of TS 25.141 [18]. NOTE 2: For a multi-band TAB connector, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, or in an adjacent or overlapping band, the wanted signal mean power is equal to -119.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.5.5.2-1 assumes that two operating bands, where the downlink frequencies (see table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.5.5.2-2: Blocking characteristics for Medium Range BS

| Operating Band | Centre Frequency of Interfering Signal | Interfering Signal Level | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| V | 824-849 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 804-824 MHz 849-869 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 860 - 880 MHz 915 - 925 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 679 - 699 MHz 716 - 729 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 679 MHz 729 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 757 - 777 MHz 787 - 807 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 768 - 788 MHz 798 - 818 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 810 - 830 MHz 845 - 865 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 821 - 832 MHz 862 - 882 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 794-814 MHz 849-859 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal (Note) |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| NOTE 1: The characteristics of the WCDMA interfering signal are specified in annex I of TS 25.141 [18]. NOTE 2: For a multi-band TAB connector, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, or in an adjacent or overlapping band, the wanted signal mean power is equal to -109.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.5.5.2-2 assumes that two operating bands, where the downlink frequencies (see Table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.5.5.2-3: Blocking characteristics for Local Area

| Operating Band | Centre Frequency of Interfering Signal | Interfering Signal Level | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| V | 824-849 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 804-824 MHz 849-869 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 860 - 880 MHz 915 - 925 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 679 - 699 MHz 716 - 729 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 679 MHz 729 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 757 - 777 MHz 787 - 807 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 768 - 788 MHz 798 - 818 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 810 - 830 MHz 845 - 865 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 821 - 832 MHz 862 - 882 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal (Note 1) |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal (Note 1) |
|  | 794-814 MHz 849-859 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal (Note 1) |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| NOTE 1: The characteristics of the WCDMA interfering signal are specified in annex I of TS 25.141 [18]. NOTE 2: For a multi-band TAB connector, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, or in an adjacent or overlapping band, the wanted signal mean power is equal to -105.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.5.5.2-3 assumes that two operating bands, where the downlink frequencies (see table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.5.5.2-4: Blocking performance requirement for Wide Area BS
when co-located with BS in other bands

| Co-located BS type | Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Macro GSM900 | 921 - 960 MHz | +16 dBm | -115 dBm | CW carrier |
| Macro DCS1800 | 1805 - 1880 MHz | +16 dBm | -115 dBm | CW carrier |
| Macro PCS1900 | 1930 - 1990 MHz | +16 dBm | -115 dBm | CW carrier |
| Macro GSM850 or CDMA850 | 869 - 894 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band I or E-UTRA Band 1 | 2110 - 2170 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band II or E-UTRA Band 2 | 1930 - 1990 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band III or E-UTRA Band 3 | 1805 - 1880 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band V or E-UTRA Band 5 | 869 - 894 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band VI or E-UTRA Band 6 | 875 - 885 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band VII or E-UTRA Band 7 | 2620 - 2690 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band VIII or E-UTRA Band 8 | 925 - 960 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XII or E-UTRA Band 12 | 729 - 746 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 17 | 734 - 746 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 18 | 860 - 875 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XIX or E-UTRA Band 19 | 875 - 890 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XX or E-UTRA Band 20 | 791 - 821 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXII or E-UTRA Band 22 | 3510 - 3590 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 23 | 2180 - 2200 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 24 | 1525 - 1559 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXV or E-UTRA Band 25 | 1930 - 1995 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXVI or E-UTRA Band 26 | 859 - 894 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 27 | 852 - 859 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 28 | 758 - 803 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 29 | 717 - 728 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 30 | 2350 - 2360 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 31 | 462.5 - 467.5 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 MHz (NOTE 3) | + 16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band e) E-UTRA Band 40 | 2300 - 2400 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 41 | 2496 - 2690 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 42 | 3400 - 3600 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 43 | 3600 - 3800 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 44 | 703 - 803 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 45 | 1447 - 1467 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 48 | 3550 - 3700 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 65 | 2110 - 2200 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 66 | 2110 - 2200 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 67 | 738 - 758 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 68 | 753 - 783 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 69 | 2570 - 2620 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 70 | 1995 - 2020 MHz | +16 dBm | -115 dBm | CW carrier |
| NOTE 1: Except for a TAB connector operating in Band XIII, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported e uplink operating band. For a TAB connector operating in band XIII the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA FDD with UTRA TDD or E-UTRA TDD on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 3: For a TAB connector operating in band XI or XXI, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. |  |  |  |  |

Table 7.5.5.2-5: Blocking performance requirement for Medium Range BS
when co-located with BS in other bands

| Co-located BS type | Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Micro GSM900 | 921 - 960 MHz | -3 dBm | -105 dBm | CW carrier |
| Micro DCS1800 | 1805 - 1880 MHz | +5 dBm | -105 dBm | CW carrier |
| Micro PCS1900 | 1930 - 1990 MHz | +5 dBm | -105 dBm | CW carrier |
| Micro GSM850 | 869 - 894 MHz | -3 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band I or E-UTRA Band 1 | 2110 - 2170 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band II or E-UTRA Band 2 | 1930 - 1990 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band III or E-UTRA Band 3 | 1805 - 1880 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band V or E-UTRA Band 5 | 869 - 894 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band VI or E-UTRA Band 6 | 875 - 885 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band VII or E-UTRA Band 7 | 2620 - 2690 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band VIII or E-UTRA Band 8 | 925 - 960 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XII or E-UTRA Band 12 | 729 - 746 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 17 | 734 - 746 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 18 | 860 - 875 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XIX or E-UTRA Band 19 | 875 - 890 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XX or E-UTRA Band 20 | 791 - 821 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XXII or E-UTRA Band 22 | 3510 - 3590 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 23 | 2180 - 2200 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 24 | 1525 - 1559 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XXV or E-UTRA Band 25 | 1930 - 1995 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XXVI or E-UTRA Band 26 | 859 - 894 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 27 | 852 - 859 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 28 | 758 - 803 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 29 | 717 - 728 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 30 | 2350 - 2360 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 31 | 462.5 - 467.5 MHz | +8 dBm | -105 dBm | CW carrier |
| MR UTRA-FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 MHz (NOTE 3) | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 33 | 1900 - 1920 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 34 | 2010 - 2025 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 38 | 2570 - 2620 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 39 | 1880 - 1920 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 40 | 2300 - 2400 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 41 | 2496 - 2690 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 42 | 3400 - 3600 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 43 | 3600 - 3800 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 44 | 703 - 803 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 45 | 1447 - 1467 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 48 | 3550 - 3700 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 65 | 2110 - 2200 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 66 | 2110 - 2200 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 67 | 738 - 758 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 68 | 753 - 783 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 69 | 2570 - 2620 MHz | +8 dBm | -105 dBm | CW carrier |
| MR E-UTRA Band 70 | 1995 - 2020 MHz | +8 dBm | -105 dBm | CW carrier |
| NOTE 1: Except for a TAB connector operating in Band XIII, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a TAB connector operating in band XIII the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA FDD with UTRA TDD or E-UTRA TDD on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 3: For a TAB connector operating in band XI or XXI, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. |  |  |  |  |

Table 7.5.5.2-6: Blocking performance requirement for Local Area BS
when co-located with BS in other bands

| Co-located BS type | Centre Frequency of Interfering Signal |  | Interfering Signal mean power |  | Wanted Signal mean power |  | Type of Interfering Signal |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pico GSM900 | 921 - 960 MHz |  | -7 dBm |  | -101 dBm |  | CW carrier |  |
| Pico DCS1800 | 1805 - 1880 MHz |  | -4 dBm |  | -101 dBm |  | CW carrier |  |
| Pico PCS1900 | 1930 - 1990 MHz |  | -4 dBm |  | -101 dBm |  | CW carrier |  |
| Pico GSM850 | 869 - 894 MHz |  | -7 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band I or E-UTRA Band 1 | 2110 - 2170 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band II or E-UTRA Band 2 | 1930 - 1990 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band III or E-UTRA Band 3 | 1805 - 1880 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band V or E-UTRA Band 5 | 869 - 894 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band VI or E-UTRA Band 6 | 875 - 885 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band VII or E-UTRA Band 7 | 2620 - 2690 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band VIII or E-UTRA Band 8 | 925 - 960 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band IX or E-UTRA Band 9 |  | 1844.9 - 1879.9 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band X or E-UTRA Band 10 |  | 2110 - 2170 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XI or E-UTRA Band 11 |  | 1475.9 - 1495.9 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XII or E-UTRA Band 12 |  | 729 - 746 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XIII or E-UTRA Band 13 |  | 746 - 756 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XIV or E-UTRA Band 14 |  | 758 - 768 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 17 |  | 734 - 746 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XIX or E-UTRA Band 19 |  | 875 - 890 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XX or E-UTRA Band 20 |  | 791 - 821 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XXI or E-UTRA Band 21 |  | 1495.9 - 1510.9 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XXII or E-UTRA Band 22 |  | 3510 - 3590 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 23 |  | 2180 - 2200 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 24 |  | 1525 - 1559 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XXV or E-UTRA Band 25 |  | 1930 - 1995 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA-FDD Band XXVI or E-UTRA Band 26 |  | 859 - 894 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 27 |  | 852 - 869 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 28 |  | 758 - 803 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 29 |  | 717 - 728 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 30 |  | 2350 - 2360 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 31 |  | 462.5 - 467.5 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA_FDD Band XXXII or E-UTRA Band 32 |  | 1452 - 1496 MHz (NOTE 3) |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA TDD Band a) or E-UTRA Band 33 |  | 1900 - 1920 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA TDD Band a) or E-UTRA Band 34 |  | 2010 - 2025 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA TDD Band d) or E-UTRA Band 38 |  | 2570 - 2620 MHz |  | -4 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA TDD Band f) or E-UTRA Band 39 |  | 1880 - 1920 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA UTRA TDD Band e) E-UTRA Band 40 |  | 2300 - 2400 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 41 |  | 2496 - 2690 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 42 |  | 3400 - 3600 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 43 |  | 3600 - 3800 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 44 |  | 703 - 803 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 45 |  | 1447 - 1467 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 48 |  | 3550 - 3700 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 65 |  | 2110 - 2200 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 66 |  | 2110 - 2200 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 67 |  | 738 - 758 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 68 |  | 753 - 783 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 69 |  | 2570 - 2620 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| LA E-UTRA Band 70 |  | 1995 - 2020 MHz |  | -6 dBm |  | -101 dBm |  | CW carrier |  |
| NOTE 1: Except for a TAB connector operating in Band XIII, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported the uplink operating band. For a TAB connector operating in band XIII the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA FDD with UTRA TDD or E-UTRA TDD on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 3: For a TAB connector operating in band XI or XXI, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. |  |  |  |  |  |  |  |  |  |

Table 7.5.5.2-7: Blocking performance requirement (narrowband) for Wide Area BS

| Operating Band | Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal (Note) |
| --- | --- | --- | --- | --- | --- |
| II | 1850 - 1910 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated |
| III | 1710 - 1785 MHz | - 47 dBm | -115 dBm | 2.8 MHz | GMSK modulated |
| IV | 1710 - 1755 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated |
| V | 824 - 849 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated |
| VIII | 880 - 915 MHz | - 47 dBm | -115 dBm | 2.8 MHz | GMSK modulated |
| X | 1710 - 1770 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated |
| XII | 699 - 716 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated |
| XIII | 777 - 787 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated |
| XIV | 788 - 798 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated |
| XXV | 1850 - 1915 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated |
| XXVI | 814-849 MHz | -47 dBm | -115 dBm | ±2.7 MHz | GMSK modulated |
| NOTE: GMSK modulation as defined in TS 45.004 [22]. |  |  |  |  |  |

Table 7.5.5.2-8: Blocking performance requirement (narrowband) for Medium range BS

| Operating Band | Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| II | 1850 - 1910 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated |
| III | 1710 - 1785 MHz | - 42 dBm | -105 dBm | 2.8 MHz | GMSK modulated |
| IV | 1710 - 1755 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated |
| V | 824 - 849 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated |
| VIII | 880 - 915 MHz | - 42 dBm | -105 dBm | 2.8 MHz | GMSK modulated |
| X | 1710 - 1770 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated |
| XII | 699 - 716 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated |
| XIII | 777 - 787 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated |
| XIV | 788 - 798 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated |
| XXV | 1850 - 1915 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated |
| XXVI | 814-849 MHz | - 42 dBm | -105 dBm | ±2.7 MHz | GMSK modulated |
| NOTE: GMSK modulation as defined in TS 45.004 [22]. |  |  |  |  |  |

Table 7.5.5.2-9: Blocking performance requirement (narrowband) for Local Area

| Operating Band | Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| II | 1850 - 1910 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated |
| III | 1710 - 1785 MHz | - 37 dBm | -101 dBm | 2.8 MHz | GMSK modulated |
| IV | 1710 - 1755 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated |
| V | 824 - 849 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated |
| VIII | 880 - 915 MHz | - 37 dBm | -101 dBm | 2.8 MHz | GMSK modulated |
| X | 1710 - 1770 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated |
| XII | 699 - 716 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated |
| XIII | 777 - 787 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated |
| XIV | 788 - 798 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated |
| XXV | 1850 - 1915 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated |
| XXVI | 814-849 MHz | - 37 dBm | -101 dBm | ±2.7 MHz | GMSK modulated |
| NOTE: GMSK modulation as defined in TS 45.004 [22]. |  |  |  |  |  |

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

NOTE 2:	Annex C of TS 25.141 [18] describes the procedure for BER tests taking into account the statistical consequence of frequent repetition of BER measurements within the blocking test. The consequence is: a DUT exactly on the limit may fail due to the statistical nature 2.55 times (mean value) in 12750 BER measurements using the predefined wrong decision probability of 0.02%. If the fail cases are ≤ 12, it is allowed to repeat the fail cases 1 time before the final verdict.

#### 7.5.5.3	Single RAT UTRA TDD 1,28 Mcps option operation

##### 7.5.5.3.1	General requirements

The static reference performance as specified in clause 7.2 shall be met with a wanted and an interfering signal coupled to the TAB connector using the parameters specified in table 7.5.5.3.1-1 to table 7.5.5.3.1-12, respectively.

The blocking requirement is always applicable outside the Base Station RF Bandwidth  or Maximum Radio Bandwidth edges. The interfering signal offset is defined relative to the lower / upper  Base Station RF Bandwidth  edges or Maximum Radio Bandwidth edges.

For multi-band TAB connector, the requirement in the in-band blocking frequency range applies for each supported operating band. The requirements applies in addition inside any Inter RF Bandwidth gap as long as the Inter RF Bandwidth gap size is at least 4.8MHz. The interfering signal offset is defined relative to the lower / upper  Base Station RF Bandwidth  edges inside the Inter RF Bandwidth gap and is equal to -2.4 MHz/+2.4 MHz, respectively.

For multi-band TAB connector, the requirement in the out-of-band blocking frequency ranges apply for each supported operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands shall be excluded from the out-of-band blocking requirement.

Table 7.5.5.3.1-1: Blocking requirements for Wide Area BS
in operating bands defined in clause 4.5 a) for 1,28 Mcps TDD

| Centre frequency of interfering signal | Interfering signal mean power | Wanted signal mean power | Minimum offset of interfering signal | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1900 - 1920 MHz, 2010 - 2025 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1880 - 1900 MHz, 1990 - 2010 MHz, 2025 - 2045 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1920 - 1980 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 1880 MHz, 1980 - 1990 MHz, 2045 - 12750 MHz | -15 dBm | -104 dBm |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -108.6 dBm. |  |  |  |  |

Table 7.5.5.3.1-2: Blocking requirements for Wide Area BS
in operating bands defined in clause 4.5 b) for 1,28 Mcps TDD

| Centre frequency of interfering signal | Interfering signal mean power | Wanted signal mean power | Minimum offset of interfering signal | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1850 - 1990 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1830 - 1850 MHz, 1990 - 2010 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 1830 MHz, 2010 - 12750 MHz | -15 dBm | -104 dBm |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -108.6 dBm. |  |  |  |  |

Table 7.5.5.3.1-3: Blocking requirements for Wide Area BS
in operating bands defined in clause 4.5 c) for 1,28 Mcps TDD

| Centre frequency of interfering signal | Interfering signal mean power | Wanted signal mean power | Minimum offset of interfering signal | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1910 - 1930 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1890 - 1910 MHz, 1930 - 1950 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 1890 MHz, 1950 - 12750 MHz | -15 dBm | -104 dBm |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -108.6 dBm. |  |  |  |  |

Table 7.5.5.3.1-4: Blocking requirements for Wide Area BS
in operating bands defined in clause 4.5 d) for 1,28 Mcps TDD

| Centre Frequency of Interfering Signal | Interfering Signal Mean Power | Wanted Signal Mean Power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 2570 - 2620 MHz | -40 dBm | -104 dBm | ±3.2MHz | 1,28 Mcps TDD signal with one code |
| 2500 - 2570 MHz, 2620 - 2690 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 2500 MHz, 2690 - 12750 MHz | -15 dBm | -104 dBm |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -108.6 dBm. |  |  |  |  |

Table 7.5.5.3.1-5: Blocking requirements for Wide Area BS
in operating bands defined in clause 4.5 e) for 1,28 Mcps TDD

| Centre Frequency of Interfering Signal | Interfering Signal Mean Power | Wanted Signal Mean Power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 2300 - 2400 MHz | -40 dBm | -104 dBm | ±3.2MHz | 1,28 Mcps TDD signal with one code |
| 2280 - 2300 MHz, 2400 - 2420 MHz | -40 dBm | -104 dBm | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 2280 MHz, 2420 - 12750 MHz | -15 dBm | -104 dBm |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -108.6 dBm. |  |  |  |  |

Table 7.5.5.3.1-6: Blocking requirements for Wide Area BS
in operating bands defined in clause 4.5 f) for 1,28Mcps TDD

| Centre Frequency of Interfering Signal | Interfering Signal Mean Power | Wanted Signal Mean Power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 1880 - 1920 MHz | -40 dBm | -104 dBm | ±3.2 MHz | Narrow band CDMA signal with one code |
| 1860 - 1880 MHz, 1920 - 1940 MHz | -40 dBm | -104 dBm | ±3.2 MHz | Narrow band CDMA signal with one code |
| 1 - 1860 MHz, 1940 - 12750 MHz | -15 dBm | -104 dBm |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -108.6 dBm. |  |  |  |  |

Table 7.5.5.3.1-7: Blocking requirements for Local Area BS
in operating bands defined in clause 4.5 a) for 1,28 Mcps TDD

| Centre frequency of interfering signal | Interfering signal level | Wanted signal level | Minimum offset of interfering signal | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1900 - 1920 MHz, 2010 - 2025 MHz | -30 dBm | PREFSENS + 6 dB | ±3,2 MHz | 1,28 Mcps TDD signal with one code |
| 1880 - 1900 MHz, 1990 - 2010 MHz, 2025 - 2045 MHz | -30 dBm | PREFSENS + 6 dB | ±3,2 MHz | 1,28 Mcps TDD signal with one code |
| 1920 - 1980 MHz | -30 dBm | PREFSENS + 6 dB | ±3,2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 1880 MHz, 1980 - 1990 MHz, 2045 - 12750 MHz | -15 dBm | PREFSENS + 6 dB |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -94.6 dBm for Local Area BS. |  |  |  |  |

Table 7.5.5.3.1-8: Blocking requirements for Local Area BS
in operating bands defined in clause 4.5 b) for 1,28 Mcps TDD

| Centre frequency of interfering signal | Interfering signal level | Wanted signal level | Minimum offset of interfering signal | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1850 - 1990 MHz | -30 dBm | PREFSENS + 6 dB | ±3,2 MHz | 1,28 Mcps TDD signal with one code |
| 1830 - 1850 MHz, 1990 - 2010 MHz | -30 dBm | PREFSENS + 6 dB | ±3,2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 1830 MHz, 2010 - 12750 MHz | -15 dBm | PREFSENS + 6 dB |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -94.6 dBm for Local Area BS. |  |  |  |  |

Table 7.5.5.3.1-9: Blocking requirements for Local Area BS
in operating bands defined in clause 4.5 c) for 1,28 Mcps TDD

| Centre frequency of interfering signal | Interfering signal level | Wanted signal level | Minimum offset of interfering signal | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1910 - 1930 MHz | -30 dBm | PREFSENS + 6 dB | ±3,2 MHz | 1,28 Mcps TDD signal with one code |
| 1890 - 1910 MHz, 1930 - 1950 MHz | -30 dBm | PREFSENS + 6 dB | ±3,2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 1890 MHz, 1950 - 12750 MHz | -15 dBm | PREFSENS + 6 dB |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -94.6 dBm for Local Area BS. |  |  |  |  |

Table 7.5.5.3.1-10: Blocking requirements for Local Area BS
in operating bands defined in clause 4.5 d) for 1,28 Mcps TDD

| Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 2570 - 2620 MHz | -30 dBm | PREFSENS + 6 dB | ±3.2MHz | 1,28 Mcps TDD signal with one code |
| 2500 - 2570 MHz, 2620 - 2690 MHz | -30 dBm | PREFSENS + 6 dB | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 2500 MHz, 2690 - 12750 MHz | -15 dBm | PREFSENS + 6 dB |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -94.6 dBm for Local Area BS. |  |  |  |  |

Table 7.5.5.3.1-11: Blocking requirements for Local Area BS
in operating bands defined in clause 4.5 e) for 1,28 Mcps TDD

| Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 2300 - 2400 MHz | -30 dBm | PREFSENS + 6 dB | ±3.2MHz | 1,28 Mcps TDD signal with one code |
| 2280 - 2300 MHz, 2400 - 2420 MHz | -30 dBm | PREFSENS + 6 dB | ±3.2 MHz | 1,28 Mcps TDD signal with one code |
| 1 - 2280 MHz, 2420 - 12750 MHz | -15 dBm | PREFSENS + 6 dB |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -94.6 dBm for Local Area BS. |  |  |  |  |

Table 7.5.5.3.1-12: Blocking requirements for Local Area BS
in operating bands defined in clause 4.5 f) for 1,28 Mcps TDD

| Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 1880-1920 MHz | -30 dBm | -90 dBm | ±3.2 MHz | Narrow band CDMA signal with one code |
| 1860 - 1880 MHz, 1920 - 1940 MHz | -30 dBm | -90 dBm | ±3.2 MHz | Narrow band CDMA signal with one code |
| 1 - 1860 MHz, 1940 - 12750 MHz | -15 dBm | -90 dBm |  | CW carrier |
| NOTE: For multi-band TAB connector, in case the interfering signal for in-band blocking is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power shall not exceed -94.6 dBm for Local Area BS. |  |  |  |  |

##### 7.5.5.3.2	Co-location with GSM, DCS, UTRA FDD and/or E-UTRA FDD, UTRA TDD and/or E-UTRA TDD

This additional blocking requirement may be applied for the protection of receiver units associated with TAB connectors with TDD operation when GSM, DCS, UTRA FDD, E-UTRA FDD, FDD unsynchronized UTRA TDD and/or unsynchronized E-UTRA TDD BTS operating in a different frequency band are co-located with UTRA TDD Wide Area BS.

The blocking performance requirement applies to interfering signals with centre frequency within the ranges specified in the tables below, using a 1 MHz step size.

In case this additional blocking requirement is applied, the static reference performance as specified in clause 7.2.1 shall be met with a wanted and an interfering signal coupled to BS antenna input using the following parameters.

Table 7.5.5.3.2-1: Additional blocking requirements for Wide Area BS

| System type operating in the same geographic area | Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Macro GSM900 | 921 - 960 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| Macro DCS1800 | 1805 - 1880 MHz | +16 dBm | -104 dBm |  | CW carrier | For TAB connector operating UTRA TDD in Band 5.2(f), it applies for 1805 - 1850 MHz |
| GSM850 or CDMA850 | 869 - 894 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA BS UTRA FDD Band I or E-UTRA Band 1 | 2110 - 2170 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA BS UTRA FDD Band iii or E-UTRA Band 3 | 1805 - 1880 MHz | +16 dBm | -104 dBm |  | CW carrier | (NOTE 3) |
| WA BS UTRA FDD Band V or E-UTRA Band 5 | 869 - 894 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA BS UTRA FDD Band VII or E-UTRA Band 7 | 2620 - 2690 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA UTRA TDD Band e) or E-UTRA Band 40 | 2300 - 2400 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA E-UTRA Band 41 | 2496 - 2690 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA E-UTRA Band 42 | 3400 - 3600 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| WA E-UTRA Band 44 | 703 - 803 MHz | +16 dBm | -104 dBm |  | CW carrier |  |
| Pico GSM850 | 869 - 894 | -7 dBm | -104 dBm |  | CW carrier |  |
| Pico GSM900 | 921 - 960 | -7 dBm | -104 dBm |  | CW carrier |  |
| Pico DCS1800 | 1805 - 1880 | -7 dBm | -104 dBm |  | CW carrier |  |
| LA BS UTRA FDD Band I or E-UTRA Band 1 | 2110 - 2170 | -6 dBm | -104 dBm |  | CW carrier |  |
| LA BS UTRA FDD Band III or E-UTRA Band 3 | 1805 - 1880 MHz | -6 dBm | -104 dBm |  | CW carrier | (NOTE 3) |
| LA BS UTRA FDD Band V or E-UTRA Band 5 | 869 - 894 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| LA BS UTRA FDD Band VII or E-UTRA Band 7 | 2620 - 2690 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| LA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| LA UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| LA UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| LA UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| LA UTRA TDD Band e) or E-UTRA Band 40 | 2300 - 2400 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| LA E-UTRA Band 41 | 2496 - 2690 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| LA E-UTRA Band 42 | 3400 - 3600 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| WA E-UTRA Band 44 | 703 - 803 MHz | -6 dBm | -104 dBm |  | CW carrier |  |
| NOTE 1: These requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz frequency range immediately outside the any of the supported uplink operating band. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD with UTRA FDD or E-UTRA FDD on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 3: For TAB connector operating UTRA TDD in Band 5.2(f), the requirement is not covered by the present release of this specification. For the source of this requirement, refer to co-location requirements in TS 25.142 [19]. |  |  |  |  |  |  |

#### 7.5.5.4	Single RAT E-UTRA operation

##### 7.5.5.4.1	General test requirement

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS antenna input using the parameters in tables 7.5.5.4.1-1, 7.5.5.4.1-2, 7.5.5.4.1-3 and 7.5.5.4.1-4. The reference measurement channel for the wanted signal is specified in tables 7.2-1, 7.2-2, 7.2-3 and 7.2-4 for each channel bandwidth and further specified in annex A.

The blocking requirement is always applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For a TAB connector operating in non-contiguous spectrum within any operating band, the blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 7.6-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band TAB connector, the requirement in the in-band blocking frequency ranges applies for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in table 7.6-2.

For a multi-band TAB connector, the requirement in the out-of-band blocking frequency ranges apply for each operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to tables 7.5.5.4.1-1, 7.5.5.4.1-2 and 7.5.5.4.1-3 shall be excluded from the out-of-band blocking requirement.

Table 7.5.5.4.1-1: Blocking performance requirement for Wide Area BS

| Operating Band | Centre Frequency of Interfering Signal (MHz) |  |  | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) (Note 1) | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18, 19, 21-23, 24, 27, 30, 33-45, 48, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -43 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -43 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -43 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 20 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -43 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 31 | (FUL_low -20) | to | (FUL_high +5) | -43 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| NOTE 1: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. NOTE 2: For a multi-band TAB connector, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, or in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE 1:	Table 7.5.5.4.1-1 assumes that two operating bands, where the downlink operating band see clause 4.5 of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.5.5.4.1-2: Blocking performance requirement for Local Area BS

| Operating Band | Centre Frequency of Interfering Signal (MHz) |  |  | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) (Note 1) | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13-14, 18,19,21-23, 24, 27, 30, 33-45, 48, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -35 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -35 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -35 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 20 | (FUL_low -11) | to | (FUL_high +20) | -35 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -35 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +15) | to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 31 | (FUL_low -20) | to | (FUL_high +5) | -35 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| NOTE 1: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. NOTE 2: For a multi-band TAB connector, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, or in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE 2:	Table 7.5.5.4.1-2 assumes that two operating bands, where the downlink operating band see clause 4.5 of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.5.5.4.1-3: Blocking performance requirement for Medium Range BS

| Operating Band | Centre Frequency of Interfering Signal (MHz) |  |  | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) (Note 1) | Interfering signal centre frequency minimum frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -38 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -38 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 20 | (FUL_low -11) | to | (FUL_high +20) | -38 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -38 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| 31 | (FUL_low -20) | to | (FUL_high +5) | -38 | PREFSENS +6 dB (Note 2) | See table 7.5.5.4.1-4 | See table 7.5.5.4.1-4 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6 dB |  | CW carrier |
| NOTE 1: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. NOTE 2: For a multi-band TAB connector, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, or in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE 3:	Table 7.5.5.4.1-3 assumes that two operating bands, where the downlink operating band see clause 4.5 of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.5.5.4.1-4: Interfering signals for blocking performance requirement

| E-UTRA channel BW of the lowest/highest carrier received (MHz) | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | 1.4 MHz E-UTRA signal |
| 3 | ±4.5 | 3 MHz E-UTRA signal |
| 5 | ±7.5 | 5 MHz E-UTRA signal |
| 10 | ±7.5 | 5 MHz E-UTRA signal |
| 15 | ±7.5 | 5 MHz E-UTRA signal |
| 20 | ±7.5 | 5 MHz E-UTRA signal |

NOTE 4:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 7.5.5.4.2	Co-location with other base stations

This additional blocking requirement may be applied for the protection of E-UTRA receiver units associated with the TAB connectors under test when GSM, CMDA, UTRA or E-UTRA BS operating in a different frequency band are co-located with an E-UTRA BS. The requirement is applicable to all channel bandwidths supported by the E-UTRA BS.

The requirements in this clause assume a 30 dB coupling loss between interfering transmitter and E-UTRA BS receiver and are based on co-location with base stations of the same class.

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to the TAB connector using the parameters in table 7.5.5.4.2-1 for AAS BS of Wide Area BS class, in table 7.5.5.4.2-2 for AAS BS of Local Area BS class and in table 7.5.5.4.2-3 for AAS BS of Medium Range BS class. The reference measurement channel for the wanted signal is specified in tables 7.2.5.3-1, 7.2.5.3-2 and 7.2.5.3-4 for each channel bandwidth and further specified in annex A of TS 36.141 [17].

Table 7.5.5.4.2-1: Blocking performance requirement
for E-UTRA Wide Area BS when co-located with BS in other frequency bands

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) (Note 5) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Macro GSM850 or CDMA850 | 869 - 894 | +16 | PREFSENS + 6 dB | CW carrier |
| Macro GSM900 | 921 - 960 | +16 | PREFSENS + 6 dB | CW carrier |
| Macro DCS1800 | 1805 - 1880 | +16 | PREFSENS + 6 dB | CW carrier |
| Macro PCS1900 | 1930 - 1990 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band I or E-UTRA Band 1 | 2110 - 2170 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band II or E-UTRA Band 2 | 1930 - 1990 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band III or E-UTRA Band 3 | 1805 - 1880 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band V or E-UTRA Band 5 | 869 - 894 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band VI or E-UTRA Band 6 | 875 - 885 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band VII or E-UTRA Band 7 | 2620 - 2690 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band VIII or E-UTRA Band 8 | 925 - 960 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 -1495.9 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XII or E-UTRA Band 12 | 729 - 746 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XIIII or E-UTRA Band 13 | 746 - 756 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 17 | 734 - 746 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 18 | 860 - 875 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XIX or E-UTRA Band 19 | 875 - 890 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XX or E-UTRA Band 20 | 791 - 821 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 24 | 1525 - 1559 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XXII or E-UTRA Band 22 | 3510 - 3590 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 23 | 2180 - 2200 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XXV or E-UTRA Band 25 | 1930 - 1995 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XXVI or E-UTRA Band 26 | 859 - 894 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 27 | 852 - 869 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 28 | 758 - 803 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 29 | 717 - 728 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 30 | 2350 - 2360 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 31 | 462.5 - 467.5 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 (Note 3) | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA TDD Band a) or E-UTRA in Band 33 | 1900 - 1920 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA TDD Band a) or E-UTRA in Band 34 | 2010 - 2025 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA TDD Band b) or E-UTRA in Band 35 | 1850 - 1910 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA TDD Band b) or E-UTRA in Band 36 | 1930 - 1990 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920 | +16 | PREFSENS + 6 dB | CW carrier |
| WA UTRA TDD Band e) or E-UTRA Band 40 | 2300 - 2400 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 41 | 2496 - 2690 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 42 | 3400 - 3600 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 43 | 3600 - 3800 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 44 | 703 - 803 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 45 | 1447 - 1467 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 48 | 3550 - 3700 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 65 | 2110 - 2200 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 66 | 2110 - 2200 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 67 | 738 - 758 | +16 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 68 | 753 - 783 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 69 | 2570 - 2620 | +16 | PREFSENS + 6dB | CW carrier |
| WA E-UTRA Band 70 | 1995 - 2020 | +16 | PREFSENS + 6dB | CW carrier |
| NOTE 1: Except for a TAB connector operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a TAB connector operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 3: For a TAB connector operating in band 11 or 21, this requirement applies for interfering signal within the frequency range 1475.9 - 1495.9 MHz. NOTE 4: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 5: PREFSENS is related to the channel bandwidth and specified in TS 36.104 [11], clause 7.2.1. |  |  |  |  |

Table 7.5.5.4.2-2: Blocking performance requirement
for Local Area BS when co-located with BS in other frequency bands

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) (Note 5) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Pico GSM850 | 869 - 894 | -7 | PREFSENS + 6 dB | CW carrier |
| Pico GSM900 | 921 - 960 | -7 | PREFSENS + 6 dB | CW carrier |
| Pico DCS1800 | 1805 - 1880 | -4 | PREFSENS + 6 dB | CW carrier |
| Pico PCS1900 | 1930 - 1990 | -4 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band I or E-UTRA Band 1 | 2110 - 2170 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band II or E-UTRA Band 2 | 1930 - 1990 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band III or E-UTRA Band 3 | 1805 - 1880 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band V or E-UTRA Band 5 | 869 - 894 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band VI or E-UTRA Band 6 | 875 - 885 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band VII or E-UTRA Band 7 | 2620 - 2690 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band VIII or E-UTRA Band 8 | 925 - 960 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XII or E-UTRA Band 12 | 729 - 746 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XIIII or E-UTRA Band 13 | 746 - 756 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 17 | 734 - 746 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 18 | 860 - 875 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XIX or E-UTRA Band 19 | 875 - 890 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XX or E-UTRA Band 20 | 791 - 821 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XXII or E-UTRA Band 22 | 3510 - 3590 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 23 | 2180 - 2200 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 24 | 1525 - 1559 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XXV or E-UTRA Band 25 | 1930 - 1995 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XXVI or E-UTRA Band 26 | 859 - 894 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 27 | 852 - 869 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 28 | 758 - 803 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 29 | 717 - 728 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 30 | 2350 - 2360 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 31 | 462.5 - 467.5 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 (Note 3) | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA TDD Band b) or E-UTRA Band 35 | 1850 - 1910 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA TDD in Band d) or E-UTRA Band 38 | 2570 - 2620 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA TDD in Band f) or E-UTRA Band 39 | 1880 - 1920 | -6 | PREFSENS + 6 dB | CW carrier |
| LA UTRA TDD in Band e) or E-UTRA Band 40 | 2300 - 2400 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 41 | 2496 - 2690 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 42 | 3400 - 3600 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 43 | 3600 - 3800 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 44 | 703 - 803 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 45 | 1447 - 1467 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 48 | 3550 - 3700 | -6 | PREFSENS + 6dB | CW carrier |
| LA E-UTRA Band 65 | 2110 - 2200 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 66 | 2110 - 2200 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 67 | 738 - 758 | -6 | PREFSENS + 6 dB | CW carrier |
| LA E-UTRA Band 68 | 753 - 783 | -6 | PREFSENS + 6dB | CW carrier |
| LA E-UTRA Band 69 | 2570 - 2620 | -6 | PREFSENS + 6dB | CW carrier |
| LA E-UTRA Band 70 | 1995 - 2020 | -6 | PREFSENS + 6dB | CW carrier |
| NOTE 1: Except for a TAB connector operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a TAB connector operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 3: For a TAB connector operating in band 11 or 21, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 4: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 5: PREFSENS is related to the channel bandwidth and specified in TS 36.104 [11], clause 7.2.1. |  |  |  |  |

Table 7.5.5.4.2-3: Blocking performance requirement
for E-UTRA Medium Range BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) (Note 5) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Micro/MR GSM850 | 869 - 894 | +8 | PREFSENS + 6 dB | CW carrier |
| Micro/MR GSM900 | 921 - 960 | +8 | PREFSENS + 6 dB | CW carrier |
| Micro/MR DCS1800 | 1805 - 1880 | +8 | PREFSENS + 6 dB | CW carrier |
| Micro/MR PCS1900 | 1930 - 1990 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band I or E-UTRA Band 1 | 2110 - 2170 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band II or E-UTRA Band 2 | 1930 - 1990 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band III or E-UTRA Band 3 | 1805 - 1880 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band V or E-UTRA Band 5 | 869 - 894 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band VI or E-UTRA Band 6 | 875 - 885 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band VII or E-UTRA Band 7 | 2620 - 2690 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band VIII or E-UTRA Band 8 | 925 - 960 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 -1495.9 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XII or E-UTRA Band 12 | 729 - 746 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XIIII or E-UTRA Band 13 | 746 - 756 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 17 | 734 - 746 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 18 | 860 - 875 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XIX or E-UTRA Band 19 | 875 - 890 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XX or E-UTRA Band 20 | 791 - 821 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XXII or E-UTRA Band 22 | 3510 - 3590 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 23 | 2180 - 2200 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 24 | 1525 - 1559 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XXV or E-UTRA Band 25 | 1930 - 1995 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XXVI or E-UTRA Band 26 | 859 - 894 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 27 | 852 - 869 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 28 | 758 - 803 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 29 | 717 - 728 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 30 | 2350 - 2360 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 31 | 462.5 - 467.5 | +8 | PREFSENS + 6 dB | CW carrier |
| MR UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 (Note 3) | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 33 | 1900 - 1920 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 34 | 2010 - 2025 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 35 | 1850 - 1910 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 36 | 1930 - 1990 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 37 | 1910 - 1930 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 38 | 2570 - 2620 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 39 | 1880 - 1920 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 40 | 2300 - 2400 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 41 | 2496 - 2690 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 42 | 3400 - 3600 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 43 | 3600 - 3800 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 44 | 703 - 803 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 45 | 1447 - 1467 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 48 | 3550 - 3700 | +8 | PREFSENS + 6dB | CW carrier |
| MR E-UTRA Band 65 | 2110 - 2200 | +8 | PREFSENS + 6 dB | CW carrier |
| WA E-UTRA Band 66 | 2110 - 2200 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 67 | 738 - 758 | +8 | PREFSENS + 6 dB | CW carrier |
| MR E-UTRA Band 68 | 753 - 783 | +8 | PREFSENS + 6dB | CW carrier |
| MR E-UTRA Band 69 | 2570 - 2620 | +8 | PREFSENS + 6dB | CW carrier |
| MR E-UTRA Band 70 | 1995 - 2020 | +8 | PREFSENS + 6dB | CW carrier |
| NOTE 1: Except for a TAB connector operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a TAB connector operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [21]. NOTE 3: For a TAB connector operating in band 11 or 21, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 4: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 5: PREFSENS is related to the channel bandwidth and specified in TS 36.104 [11], clause 7.2.1. |  |  |  |  |

## 7.6	Receiver spurious emissions

### 7.6.1	Definition and applicability

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the TAB connector. The requirements apply to all AAS BS with separate RX and TX TAB connectors.

NOTE:	In this case for FDD AAS BS the test is performed when both TX and RX are on, with the TX TAB connector terminated.

For a TAB connector supporting both RX and TX in TDD, the requirements apply during the transmitter OFF period. For a TAB connector supporting both RX and TX in FDD, the receiver spurious requirements are superseded by the TX spurious requirements in TS 37.105 [8], clause 6.6.6.

For RX only multi-band TAB connector(s), the RX spurious emissions requirements are subject to exclusion zones in each supported operating band. For multi-band TAB connector(s) that both transmit and receive in operating bands supporting TDD, RX spurious emissions requirements are applicable during the TX OFF period, and are subject to exclusion zones in each supported operating band. The unwanted emission level limit of a TAB connector RX min cell group is in general defined by the unwanted emission basic limit which is the same as the corresponding applicable Non-AAS BS per transmitter requirement specified in [3], [4], or [5], and its scaling by NRXU,countedpercell . The basic limits and corresponding scaling are defined in each relevant clause. The receiver spurious emission requirements are applied per the TAB connector RX min cell groups for all the configurations supported by the AAS BS.

### 7.6.2	Minimum Requirement

The minimum requirement for MSR operation is in TS 37.105 [8], clause 7.6.2.

The minimum requirement for single RAT UTRA operation is defined in TS 37.105 [8], clause 7.6.3.

The minimum requirement for single RAT E-UTRA operation is defined in TS 37.105 [8], clause 7.6.4.

### 7.6.3	Test Purpose

The test purpose is to verify the ability of the AAS BS to limit the interference caused by receiver spurious emissions to other systems.

### 7.6.4	Method of test

#### 7.6.4.1	Initial conditions

Test environment:

-	normal; see clause B.2

RF channels to be tested for single carrier:

-	M; see clause 4.12.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band operation, see clause 4.12.1, BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.12.1.

#### 7.6.4.2	Procedure

##### 7.6.4.2.1	General procedure

The general procedure steps apply to the procedures for all the RATs.

The minimum requirement is applied to all TAB connectors described in clause 7.6.1, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.2.4. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect TAB connector to measurement equipment as shown in annex D.2.4. All TAB connectors not under test shall be terminated.

##### 7.6.4.2.2	MSR operation

1)	Set the measurement equipment parameters as specified in table 7.6.5.2.1-1.

2)	Set the transmitter unit associated with the TAB connector under test to transmit with the carrier set-up and power allocation according to the applicable test configuration(s), see clause 5.

3)	Measure the spurious emissions over each frequency range described in clause 7.6.5.2.1.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.6.4.2.3	Single RAT UTRA FDD operation

1)	For TAB connector(s) capable of single carrier operation only, set each TAB connector  declared in the same RAT and operating band to transmit a signal according to TM1, (clause 4.12.2), at the manufacturer's declared rated output power, PRated,c,TABC.

For TAB connector(s) declared to be capable of multi-carrier operation, set each TAB connector  declared in the same RAT and operating band to transmit a signal according to TM1on all carriers configured, using the applicable test configuration and corresponding power setting for receiver tests, as specified in clause 4.11.

2)	Set measurement equipment parameters as specified in table 7.6.4.2.3-1.

3)	Measure the spurious emissions over each frequency range described in clause 7.6.5.2.2

Table 7.6.4.2.3-1: Measurement equipment parameters

| Measurement Band width | 3.84 MHz (Root raised cosine,0.22) / 100 kHz/ 1 MHz (note) |
| --- | --- |
| Sweep frequency range | 30 MHz to 12.75GHz |
| Detection | True RMS |
| NOTE: As defined in clause 7.6.2. |  |

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.6.4.2.4	Single RAT UTRA TDD 1,28Mcps option operation

1)	For TAB connector(s) capable of single carrier operation only, set each TAB connector  declared in the same RAT and operating band to transmit a signal according to table 7.6.4.2.4-1, at the manufacturer's declared rated output power, PRated,c,TABC.

For TAB connector(s) declared to be capable of multi-carrier operation, set each TAB connector  declared in the same RAT and operating band to transmit a signal according to table 7.6.4.2.4-1on all carriers configured, using the applicable test configuration and corresponding power setting for receiver tests, as specified in clause 4.11.

Table 7.6.4.2.4-1: Parameters of the transmitted signal
for Rx spurious emissions test for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; i = 0, 1, 2, ..., 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS1, TS2 and TS3 |
| Number of DPCH in each time slot under test | 8 |
| Power of each DPCH | 1/8 of Base Station output power |
| Data content of DPCH | real life (sufficient irregular) |

2)	Measure the power of the spurious emissions by applying the measuring equipment with the settings as specified in table 7.6.4.2.4-2. The characteristics of the measurement filter with the bandwidth 1,28 MHz shall be RRC with roll-off  = 0,22. The characteristics of the measurement filters with bandwidths 100 kHz and 1 MHz shall be approximately Gaussian (typical spectrum analyzer filter). The centre frequency of the filters shall be stepped in contiguous steps over the frequency bands as specified in table 7.6.4.2.4-2. The time duration of each step shall be sufficiently long to capture one even (transmit) time slot.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

Table 7.6.4.2.4-2: Measurement equipment settings

| Stepped frequency range | Measurement bandwidth | Step width | Note | Detection mode |
| --- | --- | --- | --- | --- |
| 30 MHz - 1 GHz | 100 kHz | 100 kHz |  | true RMS |
| 1 GHz - 1,880 GHz | 1 MHz | 1 MHz | With the exception of frequencies between 4 MHz below the first carrier frequency and 4 MHz above the last carrier frequency used by the BS |  |
| 1,880 GHz - 1,980 GHz | 1,28 MHz | 200 kHz |  |  |
| 1,980 GHz - 2,010 GHz | 1 MHz | 1 MHz |  |  |
| 2,010 GHz - 2,025 GHz | 1,28 MHz | 200 kHz |  |  |
| 2,025 - 2,300 GHz | 1 MHz | 1 MHz |  |  |
| 2,300 GHz -2,400 GHz | 1,28 MHz | 200 kHz |  |  |
| 2,400 GHz -2,500 GHz | 1 MHz | 1 MHz |  |  |
| 2,500 GHz - 2,620 GHz | 1,28 MHz | 200 kHz |  |  |
| 2,620 GHz - 12,75 GHz | 1 MHz | 1 MHz |  |  |

##### 7.6.4.2.5	Single RAT E-UTRA operation

1)	Set the measurement equipment parameters as specified in table 7.6.5.2.5-1.

2)	Set the TAB connector(s) to transmit with the carrier set-up and power allocation according to the applicable test configuration(s), see clause 5.

3)	Measure the spurious emissions over each frequency range described in clause 7.6.5.2.4.

In addition, for multi-band TAB connector(s), the following steps shall apply:

4)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.6.5	Test Requirements

#### 7.6.5.1	General

Conformance may be shown to either the measure and sum test requirement or the per TAB connector test requirement.

1)	The spurious emission test requirements for an AAS BS when using the measure and sum alternative are that for each TAB connector RX cell group and each applicable basic limit as specified in clause 7.6.5.2, the power summation of emissions at the TAB connectors of the TAB connector RX cell group shall not exceed a limit specified as the basic limit + X, where X = 10log10(NRXU,countedpercell), unless stated differently in regional regulation.

2)	The spurious emission test requirements for an AAS BS when using the per TAB connector alternative are that for each TAB connector RX cell group and each applicable basic limit as specified in clause 7.6.5.2, the emissions at each of the TAB connectors of the TAB connector RX cell group shall not exceed a limit specified as the basic X - 10log(n) where n is the number of TAB connectors in the TAB connector RX cell group and X = 10log10(NRXU,countedpercell), unless stated differently in regional regulation.

#### 7.6.5.2	Basic limits

##### 7.6.5.2.1	MSR operation

The basic limit for MSR operation is given below:

Table 7.6.5.2.1-1: General spurious emission test requirement

| Frequency range | Maximum level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz - 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz - 12.75 GHz | -47 dBm | 1 MHz |  |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Applies only for Bands 22, 42, 43 and 48. |
| NOTE: The frequency range from FBW RF,DL,low -10 MHz to FBW RF,_,DLhigh + 10 MHz may be excluded from the requirement. For BS capable of multi-band operation, the exclusion applies for all supported operating bands. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the single-band requirements apply and the excluded frequency range is only applicable for the operating band supported on each antenna connector. |  |  |  |

In addition to the basic limits in table 7.6.5.2.1-1, Additional spurious emissions requirements in clause 6.6.6.5.2.5 form basic limits for additional receiver spurious emission requirements.

In case of FDD BS (for BC1 and BC2), the levels specified for Protection of the BS receivers of own or different BS in clause 6.6.6.5.2.4, form basic levels for additional receiver spurious emission requirements.

In addition, the requirements for co-location with other base stations specified in clause 6.6.6.5.2.6, may also form basic levels for co-location spurious emission requirements.

##### 7.6.5.2.2	Single RAT UTRA FDD operation

The basic limit for UTRA FDD operation is given below:

For multi-band TAB connectors, the exclusions and conditions in the Notes column of table 7.6.5.2.1-1 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the single-band requirements apply and the excluded frequency range is only applicable for the operating band supported on each antenna connector.

Table 7.6.5.2.2-1: Spurious emission minimum requirement

| Band | Maximum level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz - 1 GHz | -57 dBm | 100 kHz | With the exception of frequencies between 12.5 MHz below the first carrier frequency and 12.5 MHz above the last carrier frequency transmitted used by the BS. |
| 1 GHz - 12.75 GHz | -47 dBm | 1 MHz | With the exception of frequencies between 12.5 MHz below the first carrier frequency and 12.5 MHz above the last carrier frequency transmitted used by the BS. |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | NOTE: Applies only for Band XXII |

In addition to the requirements in table 7.6.5.2.2-1, the power of any spurious emission shall not exceed the levels specified for Protection of the BS receiver of own or different BS in clause 6.6.6.5.2.4 and for Co-existence with other systems in the same geographical area in clause 6.6.6.5.2.5 and 6.6.6.5.2.6. In addition, the co-existence requirements for co-located base stations specified in clause 6.5.3.7.5 and 6.5.3.7.8.2 may also be applied.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 7.6.5.2.3	Single RAT UTRA TDD 1,28Mcps option operation

The basic limits for UTRA TDD operation are given below:

Table 7.6.5.2.3-1: General receiver spurious emission minimum requirements

| Band | Maximum level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz - 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz - 12.75 GHz | -47 dBm | 1 MHz | With the exception of frequencies between 4 MHz below the first carrier frequency and 4 MHz above the last carrier frequency used by the BS. |

In addition to the requirements in table 7.6.5.2.3-1, the power of any spurious emission shall not exceed the levels specified for Co-existence with other systems in the same geographical area in clause 6.6.6.5.2.4. In addition, the co-existence requirements for co-located base stations specified in clause 6.6.6.5.2.5 and 6.6.6.5.2.6 may also be applied.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

##### 7.6.5.2.4	Single RAT E-UTRA operation

The basic limit for E-UTRA operation is given below:

Table 7.6.5.2.4-1: General spurious emission test requirement

| Frequency range | Maximum level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz - 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz - 12.75 GHz | -47 dBm | 1 MHz |  |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Applies only for Bands 22, 42, 43 and 48. |
| NOTE: The frequency range from FBW RF,DL,low -10 MHz to FBW RF,_,DLhigh + 10 MHz may be excluded from the requirement. For a multi-band TAB connector , the exclusion applies for all supported operating bands. |  |  |  |

In addition to the requirements in table 7.6.5.2.4-1, the power of any spurious emission shall not exceed the additional spurious emissions requirements in clause 6.6.6.5.2.5 and in case of a TAB connector operating FDD (for BC1 and BC2) emission shall not exceed the levels specified for protection of the BS receivers of own or different BS in clause 6.6.6.5.2.4. In addition, the requirements for co-location with other Base Stations specified in clause 6.6.6.5.2.6 may also be applied.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 7.7	Receiver intermodulation

### 7.7.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver unit to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. The requirement applies per TAB connector.

### 7.7.2	Minimum requirement

The minimum requirement for MSR operation is defined in TS 37.105 [8], clause 7.7.2.

The single RAT UTRA FDD AAS BS of Wide Area BS class shall fulfil minimum requirements for receiver intermodulation specified in TS 25.104 [9], clause 7.6.1.

The single RAT UTRA FDD AAS BS of Medium Range BS class shall fulfil minimum requirements for receiver intermodulation specified in TS 25.104 [9], clause 7.6.1.

The single RAT UTRA FDD AAS BS of Local Area BS class shall fulfil minimum requirements for receiver intermodulation specified in TS 25.104 [9], clause 7.6.1.

The single RAT UTRA TDD AAS BS of Wide Area BS class shall fulfil minimum requirements for receiver intermodulation specified in TS 25.105 [10], clause 7.6.1.2.

The single RAT UTRA TDD AAS BS of Local Area BS class shall fulfil minimum requirements for receiver intermodulation specified in TS 25.105 [10], clause 7.6.1.2.

The single RAT E-UTRA AAS BS of Wide Area BS class shall fulfil minimum requirements for receiver intermodulation specified in TS 36.104 [11], clause 7.8.

The single RAT E-UTRA AAS BS of Medium Range BS class shall fulfil minimum requirements for receiver intermodulation specified in TS 36.104 [11], clause 7.8.

The single RAT E-UTRA AAS BS of Local Area BS class shall fulfil minimum requirements for receiver intermodulation specified in TS 36.104 [11], clause 7.8.

### 7.7.3	Test purpose

The test purpose is to verify the ability of the receiver unit associated with the TAB connector under test to inhibit the generation of intermodulation products in its non-linear elements caused by the presence of two high-level interfering signals at frequencies with a specific relationship to the frequency of the wanted signal.

### 7.7.4	Method of test

#### 7.7.4.1	Initial conditions

Test environment:

-	Normal; see clause B.2.

RF channels to be tested for single carrier (SC):

-	B, M and T; see clause 4.12.1

Base Station RF Bandwidth positions to be tested:

-	For single-band TAB connector(s),: MRFBW if ATC4 is applicable; BRFBW and TRFBW for other ATC, see clause 4.12.1. For multi-band TAB connector(s): BRFBW_T'RFBW and B'RFBW_TRFBW, see clause 4.12.1.

#### 7.7.4.2	Procedure

##### 7.7.4.2.1	General procedure

The general procedure steps apply to the procedures for all the RATs.

The minimum requirement is applied to all TAB connectors, the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect TAB connector to measurement equipment as shown in annex D.2.6. All TAB connectors not under test shall be terminated.

2)	Generate the wanted signal according to the applicable test configuration (see clause 5) using applicable reference measurement channel to the TAB connector under test as follows:

-	For E-UTRA see clause A.1 in TS 36.141 [17].

-	For UTRA FDD see clause A.2 in TS 25.141 [18].

-	For UTRA TDD see clause A.2.1 in TS 25.142 [20].

3)	Set the transmitter unit associated with the TAB connector under test to transmit with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 5).

##### 7.7.4.2.2	MSR operation

7.7.4.4.2.1	Procedure for general and narrowband intermodulation

1)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified in table 7.7.5.1.1-1 and Table 7.7.5.1.1-2 for general intermodulation requirement, and Table 7.7.5.2.1-1 and Table 7.7.5.2.1-2 for narrowband intermodulation requirement.

2)	Measure the performance of the wanted signal at the receiver unit associated with the TAB connector under test, as defined in clause 7.7.5.1.1 and 7.7.5.1.2, for the relevant carriers specified by the test configuration in clause 5.

In addition, for multi-band TAB connector(s), the following steps shall apply:

3)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.7.4.2.3	Single RAT UTRA FDD operation

1)	Generate the wanted signal (reference signal) and adjust ATT1 to set the signal level to the TAB connector under test to the level specified in table 7.7.5.2-1. For a TAB connector supporting multi-carrier operation, generate the wanted signal according to the applicable test configuration (see clause 4.11) using applicable reference measurement channel to the TAB connector under test. Power settings are specified in table 7.7.5.2-1.

2)	Adjust the signal generators to the type of interfering signals and the frequency offsets as specified in tables 7.7.5.2-1and 7.7.5.2-2. Note that the GMSK modulated interfering signal shall have an ACLR of at least 72 dB in order to eliminate the impact of interfering signal adjacent channel leakage power on the intermodulation characteristics measurement.

3)	Adjust the ATT2 and ATT3 to obtain the specified level of interfering signal at the TAB connector.

4)	Measure the BER of the wanted signal. For a TAB connector supporting multi-carrier operation the BER shall be measured for all relevant carriers specified by the test configuration.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.7.4.2.4	Single RAT UTRA TDD 1,28Mcps option operation

1)	Start transmission from the BS tester to the TAB connector using the UL reference measurement channel (12,2 kbps) defined in clause A.2.1 of TS 25.142 [20]. For a TAB connector supporting multi-carrier operation, generate the wanted signal using the applicable test configuration specified in clause 4.11, and the UL reference measurement channel in clause A.2.1 of TS 25.142 [20] shall be used on all carriers for the TAB connector under test. The level of the UE simulator signal measured at the TAB connector shall be set to 6 dB above the reference sensitivity level specified in clause 7.2.2.

2)	Set the first signal generator to produce a CW signal with a level measured at the TAB connector as specified in table 7.7.5.3-1.

3)	Set the second signal generator to produce an interfering signal equivalent to a wideband CDMA signal with one code of chip frequency 1,28 MHz, filtered by an RRC transmit pulse-shaping filter with roll-off  = 0,22. The level of the signal measured at the TAB connector shall be set as specified in table 7.7.5.3-1.

4)	Measure the BER of the wanted signal.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

##### 7.7.4.2.5	Single RAT E-UTRA operation

1)	Generate the wanted signal using the applicable test configuration specified in clause 5 and adjust the signal level to the TAB connector under test to the level specified in table 7.7.5.4-1.

2)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified in table 7.7.5.4-2 for intermodulation requirement and Table 7.7.5.4-3, Table 7.7.5.4-4 and Table 7.7.5.4-5 for narrowband intermodulation requirement.

3)	Adjust the signal generators to obtain the specified level of interfering signal at the TAB connector.

4)	Measure the throughput, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clause 5.

In addition, for multi-band TAB connector(s), the following steps shall apply:

5)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.7.5	Test requirements

#### 7.7.5.1	MSR operation

##### 7.7.5.1.1	General intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA or UTRA signal, as specified in annex A of TS 37.141 [16].

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the RF bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the TAB connector, using the parameters in table 7.7.5.1.1-1 and 7.7.5.1.1-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.1.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

Table 7.7.5.1.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals (dBm) | Wanted Signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48 | PREFSENS +x dB (Note 2) |  |
| Medium Range BS | -44 | PREFSENS +x dB (Note 3) | See Table 7.7.5.1.1-2 |
| Local Area BS | -38 | PREFSENS +x dB (Note 4) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104 [12]. For E-UTRA channel bandwidths 10, 15 and 20 MHz this requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 2: For WA BS, "x" is equal to 6 in case of E-UTRA or UTRA wanted signals NOTE 3: For MR BS, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA wanted signal. NOTE 4: For LA BS, "x" is equal to 12 in case of E-UTRA wanted signals, 6 in case of UTRA wanted signal. |  |  |  |

Table 7.7.5.1.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the high/low Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2.0 (BC1 and BC3) / ±2.1 (BC2) | CW |
|  | ±4.9 | 1.4 MHz E-UTRA signal |
| E-UTRA 3 MHz | ±4.4 (BC1 and BC3) / ±4.5 (BC2) | CW |
|  | ±10.5 | 3 MHz E-UTRA signal |
| UTRA FDD and E-UTRA 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| E-UTRA 10 MHz | ±7.375 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| E-UTRA 15 MHz | ±7.25 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| E-UTRA 20 MHz | ±7.125 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| GSM/EDGE | ±7.575 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| 1,28 Mcps UTRA TDD | ±2.3 (BC3) | CW |
|  | ±5.6 (BC3) | 1,28Mcps UTRA TDD signal |

##### 7.7.5.1.2	General narrowband intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal, as specified in annex A of TS 37.141 [16].

The requirement is applicable outside the Base Station RF Bandwidth or maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF bandwidth edges or maximum Radio Bandwidth edges.

For a TAB connector operating in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.7.5.1.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For a multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in table 7.7.5.1.2-2. The interfering signal offset is defined relative to the RF bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the Base Station antenna input, using the parameters in table 7.7.5.2-1 and 7.7.5.2-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.1.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

Table 7.7.5.1.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals (dBm) | Wanted Signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 | PREFSENS +x dB (Note) | See Table 7.7.5.1.2-2 |
| Medium Range BS | -47 |  |  |
| Local Area BS | -44 |  |  |
| NOTE: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104 [12]. "x" is equal to 6 in case of E-UTRA or UTRA wanted signals. |  |  |  |

Table 7.7.5.1.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the high/low Base Station RF Bandwidth edge or edge of the sub-block | Interfering signal centre frequency offset from the Base Station RF bandwidth edge or edge of sub-block inside a gap (kHz) | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1.4 MHz E-UTRA signal, 1 RB (Note 1) |
| E-UTRA 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3.0 MHz E-UTRA signal, 1 RB (Note 1) |
| E-UTRA 5 MHz | ±360 | CW |
|  | ±1060 | 5 MHz E-UTRA signal, 1 RB (Note 1) |
| E-UTRA 10 MHz (Note 2) | ±325 | CW |
|  | ±1240 | 5 MHz E-UTRA signal, 1 RB (Note 1) |
| E-UTRA 15 MHz (Note 2) | ±380 | CW |
|  | ±1600 | 5 MHz E-UTRA signal, 1 RB (Note 1) |
| E-UTRA 20 MHz (Note 2) | ±345 | CW |
|  | ±1780 | 5 MHz E-UTRA signal, 1 RB (Note 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1780 (BC1 and BC2) | 5 MHz E-UTRA signal, 1 RB (Note 1) |
| GSM/EDGE | ±340 | CW |
|  | ±880 | 5 MHz E-UTRA signal, 1 RB (Note 1) |
| 1,28Mcps UTRA TDD | ±190 (BC3) | CW |
|  | ±970 (BC3) | 1.4 MHz E-UTRA signal, 1 RB (Note 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF bandwidth edge or edge of sub-block inside a gap. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

#### 7.7.5.2	Single RAT UTRA FDD operation

For each measured carrier, the BER shall not exceed 0,001 for the parameters specified in tables 7.7.5.2-1 and 7.7.5.2-2.

The intermodulation performance shall be met when the following signals are applied to the receiver.

For a TAB connector operating in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 6.8MHz. The CW interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -1 MHz/+1 MHz, respectively. The GMSK modulated interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -3.4 MHz/+3.4 MHz, respectively. The requirement applies separately for both sub-blocks.

For a multi-band TAB connector, the narrowband intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 6.8MHz. The CW interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -1 MHz/+1 MHz, respectively. The GMSK modulated interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -3.4 MHz/+3.4 MHz, respectively.

Table 7.7.5.2-1: Intermodulation performance requirement

| Operating | Type of Signal | Offset | Signal mean power |  |  |
| --- | --- | --- | --- | --- | --- |
| Band |  |  | Wide Area BS | Medium Range BS | Local Area |  |
| All bands | Wanted signal | - | -115 dBm | -105 dBm | -101 dBm |
|  | CW signal | 10 MHz | -48 dBm | -44 dBm | -38 dBm |  |
|  | WCDMA signal (Note) | 20 MHz | -48 dBm | -44 dBm | -38 dBm |  |
| NOTE: The characteristics of the W-CDMA interfering signal are specified in annex I of TS 25.141 [18] |  |  |  |  |  |  |

Table 7.7.5.2.-2: Narrowband intermodulation performance requirement

| Operating | Type of Signal | Offset | Signal mean power |  |  |
| --- | --- | --- | --- | --- | --- |
| band |  |  | Wide Area BS | Medium Range BS | Local Area |
| II, III, IV, V, | Wanted signal | - | -115 dBm | -105 dBm | -101 dBm |
| VIII, X, XII, | CW signal | 3.5 MHz | - 47 dBm | - 43 dBm | -37 dBm |
| XIII, XIV, XXV, XXVI | GMSK modulated (Note) | 5.9 MHz | - 47 dBm | - 43 dBm | -37 dBm |
| NOTE: GMSK as defined in TS 45.004 [22]. |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.7.5.3	Single RAT UTRA TDD 1,28Mcps option operation

The BER measured shall not exceed 0,001.

The static reference performance as specified in clause 7.2.5.2 should be met when the following signals are coupled to the TAB connector.

-	A wanted signal at the assigned channel frequency, with mean power 6 dB above the static reference level.

-	Two interfering signals with the parameters specified in table 7.7.5.3-1.

The blocking requirement is always applicable outside the Base Station RF bandwidth or maximum Radio Bandwidth edges. The interfering signal offset is defined relative to the lower (upper) or maximum Radio Bandwidth edges.

For a multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 11.2MHz. The CW interfering signal offset is defined relative to lower/upper Base Station RF bandwidth edges inside the Inter RF Bandwidth gap and is equal to -2.4 MHz/+2.4 MHz, respectively. The modulated interfering signal offset is defined relative to lower/upper Base Station RF bandwidth edges inside the Inter RF Bandwidth gap and is equal to -5.6MHz/+5.6MHz, respectively.

Table 7.7.5.3-1: Parameters of the interfering signals
for intermodulation characteristics testing for 1,28 Mcps TDD

| Interfering Signal mean power |  | Offset | Type of Interfering Signal |
| --- | --- | --- | --- |
| Wide Area BS | Local Area BS |  |  |
| - 48 dBm | - 38 dBm | ±3,2 MHz | CW signal |
| - 48 dBm | - 38 dBm | ±6,4 MHz | 1,28 Mcps TDD signal with one code |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

#### 7.7.5.4	Single RAT E-UTRA operation

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals with the conditions specified in tables 7.7.5.4-1 and 7.7.5.4-2 for intermodulation performance and in tables 7.7.5.4-3, 7.7.5.4-4 and 7.7.5.4-5 for narrowband intermodulation performance. The reference measurement channel for the wanted signal is specified in tables 7.2.5.3-1, 7.2.5.3-2 and 7.2.5.3-3 for each channel bandwidth and further specified in annex A of TS 36.141 [17].

The receiver intermodulation requirement is always applicable outside the Base Station RF bandwidth or maximum Radio Bandwidth edges. The interfering signal offset is defined relative to the lower (upper) or maximum Radio Bandwidth edges.

For a TAB connector operating in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.7.5.4-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap. The requirement applies separately for both sub-blocks.

For a multi-band TAB connector, the intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the E-UTRA interfering signal centre frequency offset from the Base Station RF bandwidth edge.

For a multi-band TAB connector, the narrowband intermodulation requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in tables 7.7.5.4-3, 7.7.5.4-4 and 7.7.5.4-5. The interfering signal offset is defined relative to the Base Station RF bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.7.5.4-1: Intermodulation performance requirement

| BS type | Wanted signal mean power (dBm) (Note) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS + 6 dB | -52 |  |
| Medium Range BS | PREFSENS + 6 dB | -47 | See Table 7.7.5.4-2 |
| Local Area BS | PREFSENS + 6 dB | -44 |  |
| NOTE: PREFSENS depends on the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. For E-UTRA channel bandwidths 10, 15 and 20 MHz this requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |  |

Table 7.7.5.4-2: Interfering signal for Intermodulation performance requirement

| E-UTRA channel bandwidth of the lowest (highest) carrier received (MHz) | Interfering signal centre frequency offset from the lower (upper) edge (MHz) | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | CW |
|  | ±4.9 | 1.4 MHz E-UTRA signal |
| 3 | ±4.5 | CW |
|  | ±10.5 | 3 MHz E-UTRA signal |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| 10 | ±7. 375 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| 15 | ±7.25 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| 20 | ±7.125 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |

Table 7.7.5.4-3: Narrowband intermodulation performance requirement for Wide Area BS

| E-UTRA channel bandwidth of the lowest (highest) carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering RB centre frequency offset from the lower (upper) edge or sub-block edge inside a sub-block gap (kHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 6 dB | -52 | ±270 | CW |
|  |  | -52 | ±790 | 1.4 MHz E-UTRA signal, 1 RB (Note 2) |
| 3 | PREFSENS + 6 | -52 | ±270 | CW |
|  |  | -52 | ±780 | 3.0 MHz E-UTRA signal, 1 RB (Note 2) |
| 5 | PREFSENS + 6 dB | -52 | ±360 | CW |
|  |  | -52 | ±1060 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 10 | PREFSENS + 6 dB | -52 | ±325 | CW |
|  | (Note 3) | -52 | ±1240 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 15 | PREFSENS + 6 dB | -52 | ±380 | CW |
|  | (Note 3) | -52 | ±1600 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 20 | PREFSENS + 6 dB | -52 | ±345 | CW |
|  | (Note 3) | -52 | ±1780 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| NOTE 1: PREFSENS is related to the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower (upper) edge. NOTE 3: This requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals |  |  |  |  |

Table 7.7.5.4-4: Narrowband intermodulation performance requirement for Local Area BS

| E-UTRA channel bandwidth of the lowest (highest) carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering RB centre frequency offset from the lower (upper) edge or sub-block edge inside a sub-block (kHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 6 dB | -44 | ±270 | CW |
|  |  | -44 | ±790 | 1.4 MHz E-UTRA signal, 1 RB (Note 2) |
| 3 | PREFSENS + 6 | -44 | ±275 | CW |
|  |  | -44 | ±790 | 3.0 MHz E-UTRA signal, 1 RB (Note 2) |
| 5 | PREFSENS + 6 dB | -44 | ±360 | CW |
|  |  | -44 | ±1060 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 10 | PREFSENS + 6 dB | -44 | ±415 | CW |
|  | (Note 3) | -44 | ±1420 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 15 | PREFSENS + 6 dB | -44 | ±380 | CW |
|  | (Note 3) | -44 | ±1600 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 20 | PREFSENS + 6 dB | -44 | ±345 | CW |
|  | (Note 3) | -44 | ±1780 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| NOTE 1: PREFSENS is related to the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower (upper) edge. NOTE 3: This requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |  |  |

Table 7.7.5.4-5: Narrowband intermodulation performance requirement for Medium Range BS

| E-UTRA channel bandwidth of the lowest (highest) carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering RB centre frequency offset to the lower (higher) edge or sub-block edge inside a sub-block gap (kHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 6 dB | −47 | ±270 | CW |
|  |  | −47 | ±790 | 1.4 MHz E-UTRA signal, 1 RB (Note 2) |
| 3 | PREFSENS + 6 | −47 | ±270 | CW |
|  |  | −47 | ±780 | 3.0 MHz E-UTRA signal, 1 RB (Note 2) |
| 5 | PREFSENS + 6 dB | −47 | ±360 | CW |
|  |  | −47 | ±1060 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 10 | PREFSENS + 6 dB | −47 | ±325 | CW |
|  | (Note 3) | −47 | ±1240 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 15 | PREFSENS + 6 dB | −47 | ±380 | CW |
|  | (Note 3) | −47 | ±1600 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 20 | PREFSENS + 6 dB | −47 | ±345 | CW |
|  | (Note 3) | −47 | ±1780 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| NOTE 1: PREFSENS is related to the channel bandwidth as specified in TS 36.104 [11], clause 7.2.1. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower (higher) edge. NOTE 3: This requirement shall apply only for a FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

## 7.8	In-channel selectivity

### 7.8.1	Definition and applicability

In-channel selectivity (ICS) is a measure of the receiver unit ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel. The requirement applies per TAB connector.

### 7.8.2	Minimum requirement

The minimum requirement for E-UTRA operation is defined in TS 37.105 [8], clause 7.8.4.

### 7.8.3	Test purpose

The purpose of this test is to verify the ability of the receiver unit associated with the TAB connector under test to suppress the IQ leakage.

### 7.8.4	Method of test

#### 7.8.4.1	Initial conditions

Test environment:

-	normal; see clause B.2

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.12.1.

#### 7.8.4.2	Procedure

The minimum requirement is applied to all TAB connectors, the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect TAB connector to measurement equipment as shown in annex D.2.5. All TAB connectors not under test shall be terminated.

For each supported E-UTRA channel BW:

2)	Adjust the signal generator for the wanted signal as specified in table 7.8.5-1 for AAS BS of  Wide Area BS class, in table 7.8.5-2 for AAS BS of  Local Area BS class and in table 7.8.5-3 for AAS BS of  Medium Range BS class on one side of the FC.

3)	Adjust the signal generator for the interfering signal as specified in table 7.8.5-1 for AAS BS of  Wide Area BS class, in table 7.8.5-2 for AAS BS of  Local Area BS class and in table 7.8.5-3 for AAS BS of  Medium Range BS class at opposite side of the FC and adjacent to the wanted signal.

4)	Measure throughput.

5)	Repeat the measurement with the wanted signal on the other side of the FC, and the interfering signal at opposite side of the FC and adjacent to the wanted signal.

In addition, for multi-band TAB connector(s), the following steps shall apply:

6)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.8.5	Test requirements

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in [Annex A] with parameters specified in table 7.8.5-1 for AAS BS of Wide Area BS, class in Table7.8.5-2 for AAS BS of Local Area BS class and in table 7.8.5-3 for AAS BS of Medium Range BS class.

Table 7.8.5-1: Wide Area BS in-channel selectivity

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in clause A.1 | -105.5 | -105.1 | -87 | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in clause A.1 | -100.7 | -100.3 | -84 | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in clause A.1 | -98.6 | -98.2 | -81 | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in clause A.1 | -97.1 | -96.7 | -77 | 10 MHz E-UTRA signal, 25 RBs |
| 15 | A1-3 in clause A.1 (Note) | -97.1 | -96.7 | -77 | 15 MHz E-UTRA signal, 25 RBs (Note) |
| 20 | A1-3 in clause A.1 (Note) | -97.1 | -96.7 | -77 | 20 MHz E-UTRA signal, 25 RBs (Note) |
| NOTE: Wanted and interfering signal are placed adjacently around FC. |  |  |  |  |  |

Table 7.8.5-2: Local Area BS in-channel selectivity

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in clause A.1 | -97.5 | -97.1 | -79 | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in clause A.1 | -92.7 | -92.3 | -76 | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in clause A.1 | -90.6 | -90.2 | -73 | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in clause A.1 | -89.1 | -88.7 | -69 | 10 MHz E-UTRA signal, 25 RBs |
| 15 | A1-3 in clause A.1 (Note) | -89.1 | -88.7 | -69 | 15 MHz E-UTRA signal, 25 RBs (Note) |
| 20 | A1-3 in clause A.1 (Note) | -89.1 | -88.7 | -69 | 20 MHz E-UTRA signal, 25 RBs (Note) |
| NOTE: Wanted and interfering signal are placed adjacently around Fc. |  |  |  |  |  |

Table 7.8.5-3: Medium Range BS in-channel selectivity

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in clause A.1 | -100.5 | -100.1 | -82 | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in clause A.1 | -95.7 | -95.3 | -79 | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in clause A.1 | -93.6 | -93.2 | -76 | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in clause A.1 | -92.1 | -91.7 | -72 | 10 MHz E-UTRA signal, 25 RBs |
| 15 | A1-3 in clause A.1 (Note) | -92.1 | -91.7 | -72 | 15 MHz E-UTRA signal, 25 RBs (Note) |
| 20 | A1-3 in clause A.1 (Note) | -92.1 | -91.7 | -72 | 20 MHz E-UTRA signal, 25 RBs (Note) |
| NOTE: Wanted and interfering signal are placed adjacently around Fc. |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

# 8	Performance requirements

## 8.1	General

Performance requirements specify the ability of the AAS BS to correctly demodulate signals in various conditions and configurations.

The demodulation requirements for an AAS BS are the same as non-AAS BS demodulation requirements specified for:

-	Single RAT UTRA operation in TS 25.104 [9] clause 8 for FDD operation, and in TS 25.105 [10] clause 8 for TDD operation,

-	Single RAT E-UTRA operation in TS 36.104 [11], clauses 8.2 – 8.4.

The test requirements for the AAS BS performance requirements have hence been referenced from TS 25.141 [18] clause 8 for UTRA FDD, TS 25.142 [20] clause 8 for UTRA TDD, and TS 36.141 [17] clauses 8.2 – 8.4 for E-UTRA respectively. Necessary amendments for the AAS BS architecture are made in this clause.

## 8.2	Performance requirements MSR

For Single RAT UTRA operation in FDD, the minimum performance requirements are specified in clause 8.3.

For Single RAT UTRA operation in TDD 1,28Mcps option, the minimum performance requirements are specified in clause 8.4.

For Single RAT E-UTRA operation, the minimum performance requirements are specified in clause 8.5.

## 8.3	Performance requirements UTRA FDD

### 8.3.1	Definition and applicability

Performance requirements for single RAT UTRA operation in FDD are specified for the measurement channels defined in TS 25.104 [2]. The requirements only apply to those measurement channels that are supported by AAS BS. For FRC8 in TS 25.104 [2] the non E-DPCCH boosting and E-DPCCH boosting requirement only apply for the option supported by the AAS BS. The performance requirements for the high speed train scenarios defined in TS 25.104 [2] and TS 25.105 [3] are optional.

Unless stated otherwise, performance requirements apply for a single cell only. Performance requirements for an AAS BS supporting DC-HSUPA or DB-DC-HSUPA are defined in terms of single carrier requirements. The requirements in clause 8 shall be met with the transmitter unit(s) associated with the TAB connectors(s) in the operating band ON.

NOTE:	In normal operating conditions the TAB connector(s) are configured to transmit and receive at the same time. The transmitter unit(s) associated with the TAB connectors may be OFF for some of the tests.

In the referred UTRA specifications and in this section, the term BS with RX diversity refers to performance requirements for two demodulation branches, and BS without RX diversity refers to performance requirements for one demodulation branch.

For AAS BS with RX diversity, only the BS performance requirements with RX diversity apply, the required Eb/N0 shall be applied separately for each demodulation branch.

For AAS BS without RX diversity, only the BS performance requirements without RX diversity apply. The required Eb/N0 shall be applied for each AAS BS demodulation branch.

The Eb/N0 used is defined as:

Where:

is the received total energy of DPDCH, DPCCH, S-DPCCH, HS-DPCCH, E-DPDCH, S-E-DPDCH, E-DPCCH and S-E-DPCCH per PN chip per demodulation branch from all branches

is the total one-sided noise power spectral density due to all noise sources

is the number of chips per frame

is the number of information bits in DTCH excluding CRC bits per frame

Table 8.3.1-1: Summary of AAS BS performance targets for single RAT UTRA operation in FDD

| Physical channel | Measurement channel | Static | Multi-path Case 1 | Multi-path Case 2 | Multi-path Case 3 | Moving | Birth / Death | High Speed Train (Note) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Performance metric |  |  |  |  |  |  |
|  | 12.2 kbps | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER<10-2 | BLER <10-2 | BLER <10-2 | BLER <10-2 |
| DCH | 64 kbps | BLER< 10-1,10-2 | BLER< 10-1, 10-2 | BLER< 10-1,10-2 | BLER < 10-1, 10-2,10-3 | BLER < 10-1,10-2 | BLER < 10-1,10-2 | - |
|  | 144 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER < 10-1, 10-2,10-3 | - | - | - |
|  | 384 kbps | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER< 10-1,10-2 | BLER < 10-1, 10-2,10-3 | - | - | - |
| NOTE: Optional condition, not applicable for all BSs. |  |  |  |  |  |  |  |  |

In the referenced test requirements from TS 25.141 [15] the method to test describes connection to one or a number of BS antenna connectors. When applying these methods to the AAS BS connection shall be made to the declared TAB connectors (see table 4.10-1, D8.1) which represent the demodulation branches.

### 8.3.2	Minimum Requirement

The minimum requirements for the single RAT UTRA operation in FDD for each of the tests specified in clauses 8.2 to 8.13 of TS 25.141 [18] are the demodulation performance requirements specified in clauses 8.2 to 8.12 of TS 25.104 [9].

### 8.3.3	Test purpose

The test shall verify the receiver's ability to achieve the specified performance metrics under the measurement channels and conditions for a given Ec/N0 or Eb/N0 for each of the demodulation performance requirements specified in clauses 8.2 to 8.12 of TS 25.104 [9].

### 8.3.4	Method of test

#### 8.3.4.1	Initial Conditions

The initial conditions for each of the tests as specified in clauses 8.2 to 8.13 of TS 25.141 [18], shall be the same as those specified in clauses 8.2 to 8.13 of TS 25.141 [18].

With the exception that instead of connecting to BS antenna connectors as stated in TS 25.141 [18], clause 8 connection shall be made to the declared TAB connectors which represent the demodulation branches (see table 4.10-1, D6.74).

All unused TAB connectors shall be terminated. Receiver units associated with unused TAB connectors may be turned OFF.

#### 8.3.4.2	Procedure

The initial conditions shall be the same as those specified in clauses 8.2 to 8.13 of TS 25.141 [18].

### 8.3.5	Test Requirement

The test requirements for each of the tests, as specified in clauses 8.2 to 8.13 of TS 25.141 [18], shall be the same as those in clauses 8.2 to 8.13 of TS 25.141 [18].

In the referenced test requirements in this section, the term "BS with RX diversity" should be replaced with performance requirements for two demodulation branches, and the term "BS without RX diversity" should be replaced with performance requirements for one demodulation branch.

## 8.4	Performance requirements UTRA TDD

### 8.4.1	Definition and applicability

Performance requirements for single RAT UTRA operation in TDD are specified for the measurement channels defined in TS 25.105 [3]. The requirements only apply to those measurement channels that are supported by AAS BS. The performance requirements for the high speed train conditions defined in TS 25.105 [3] are optional. All Bit Error Ratio (BER) and Block Error Rate (BLER) measurements shall be carried out according to the general rules for statistical testing defined in recommendation ITU-T O.153 [34] and annex F of TS 25.142 [20].

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for a AAS BS supporting MC-HSUPA are defined in terms of single carrier requirements.

The characteristics of the white noise source, simulating interference from other cells (Ioc), shall comply with the AWGN interferer definition in TS 25.105 [10], clause 5.18.

In the referred UTRA specifications and in this section, the term BS with dual RX diversity refers to performance requirements for two demodulation branches, and BS without RX diversity refers to performance requirements for one demodulation branch.

For AAS BS with RX diversity, only the BS performance requirements with dual RX diversity apply, the required Îor/Ioc shall be applied separately for each demodulation branch.

For AAS BS without RX diversity, only the BS performance requirements without dual RX diversity apply. The required Îor/Ioc shall be applied for each AAS BS demodulation branch.

Table 8.4.1-1: Summary of AAS BS performance targets for single RAT UTRA operation in TDD

| Physical channel | Measurement channel | Static | Multi-path Case 1 | Multi-path Case 2 (Note) | Multi-path Case 3 (Note) | High speed train (Note) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Performance metric |  |  |  |  |
|  | 12,2 kbps | BLER < 10-2 | BLER < 10-2 | BLER < 10-2 | BLER < 10-2 | BLER<10-2 |
| DCH | 64 kbps | BLER < 10-1, 10-2 | BLER < 10-1, 10-2 | BLER < 10-1, 10-2 | BLER < 10-1, 10-2, 10-3 | BLER< 10-1, 10-2 |
|  | 144 kbps | BLER < 10-1, 10-2 | BLER < 10-1, 10-2 | BLER < 10-1, 10-2 | BLER < 10-1, 10-2, 10-3 | - |
|  | 384 kbps | BLER < 10-1, 10-2 | BLER < 10-1, 10-2 | BLER < 10-1, 10-2 | BLER < 10-1, 10-2, 10-3 | - |
| NOTE: Optional condition, not applicable for all BSs. |  |  |  |  |  |  |

In the referenced test requirements from TS 25.142 [19] the method to test describes connection to one or a number of BS antenna connectors. When applying these methods to the AAS BS connection shall be made to the declared TAB connectors (see table 4.10-1, D8.1) which represent the demodulation branches.

### 8.4.2	Minimum Requirement

The minimum requirements for the single RAT UTRA operation in TDD for each of the tests specified in clauses 8.2 to 8.5 of TS 25.142 [20] are the demodulation performance requirements specified in clauses 8.2 to 8.5 of TS 25.105 [10].

### 8.4.3	Test purpose

The test shall verify the receiver's ability to achieve the specified performance metrics under the measurement channels and conditions for a given Îor/Ioc for each of the demodulation performance requirements specified in clauses 8.2 to 8.5 of TS 25.105 [10].

### 8.4.4	Method of test

#### 8.4.4.1	Initial Conditions

The initial conditions for each of the tests as specified in clauses 8.2 to 8.5 of TS 25.142 [20], shall be the same as those specified in clauses 8.2 to 8.5 of TS 25.142 [20].

With the exception that instead of connecting to BS antenna connectors as stated in clauses 8.2 to 8.5 of TS 25.142 [20] connection shall be made to the declared TAB connectors which represent the demodulation branches (see table 4.10-1, D6.74).

All unused TAB connectors shall be terminated. Receiver units associated with unused TAB connectors may be turned OFF.

#### 8.4.4.2	Procedure

The initial conditions shall be the same as those specified in clauses 8.2 to 8.5 of TS 25.142 [20].

### 8.4.5	Test Requirement

The test requirements for each of the tests, as specified in clauses 8.2 to 8.5 of TS 25.142 [20], shall be the same as those in clauses 8.2 to 8.5 of TS 25.142 [20].

In the referenced test requirements in this section, the term "BS with RX diversity" should be replaced with performance requirements for two demodulation branches, and the term "BS without RX diversity" should be replaced with performance requirements for one demodulation branch.

## 8.5	Performance requirements E-UTRA

### 8.5.1	Definition and applicability

Performance requirements for single RAT E-UTRA operation are specified for the fixed reference channels (FRC) and propagation conditions defined in TS 36.104 [8] annex A and annex B, respectively. The requirements only apply to those FRCs that are supported by the AAS BS.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for a single RAT E-UTRA operation supporting carrier aggregation are defined in terms of single carrier requirements. For FDD operation the requirements shall be met with the transmitter unit(s) associated with the TAB connectors(s) in the operating band ON.

NOTE:	In normal operating conditions the TAB connector(s) in FDD operation are configured to transmit and receive at the same time. The transmitter unit(s) associated with the TAB connectors may be OFF for some of the tests.

In the referred E-UTRA specification, the term "RX antennas" refers to demodulation branches (i.e.  not physical antennas).

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the subframe on a single TAB connector.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe.

For enhanced performance requirements type A in TS 36.104 [4], the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy in the subframe on a single TAB connector.

is the summation of the received energy of the strongest interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single TAB connector. The respective energy of each interferer relative to  is defined by its associated DIP value.

In the referenced test requirements from TS 36.141 [14] the method to test describes connection to one or a number of BS antenna connectors. When applying these methods to the AAS BS connection shall be made to the declared TAB connectors (see table 4.10-1, D8.1) which represent the demodulation branches.

### 8.5.2	Minimum Requirement

The minimum requirements for single RAT E-UTRA operation for each of the tests specified in clauses 8.2 to 8.4 of TS 36.141 [17] are the demodulation performance requirements specified in clauses 8.2 to 8.4 of TS 36.104 [11].

### 8.5.3	Test purpose

The test shall verify the receiver's ability to achieve the specified performance metrics under the measurement channels and conditions for a given SNR or SINR for each of the demodulation performance requirements specified in clauses 8.2 to 8.4 of TS 36.104 [11].

### 8.5.4	Method of test

#### 8.5.4.1	Initial Conditions

The initial conditions for each of the tests as specified in clauses 8.2 to 8.4 of TS 36.141 [17], shall be the same as those specified in clauses 8.2 to 8.4 of TS 36.141 [17].

With the exception that instead of connecting to BS antenna connectors as stated in TS 36.141 [17] clause 8 connection shall be made to the declared TAB connectors which represent the demodulation branches (see table 4.10-1, D6.74).

All unused TAB connectors shall be terminated. Receiver units associated with unused TAB connectors may be turned OFF.

#### 8.5.4.2	Procedure

The procedure shall be the same as those specified in clauses 8.2 to 8.4 of TS 36.141 [17].

### 8.5.5	Test Requirement

The test requirements for each of the tests, as specified in clauses 8.2 to 8.4 of TS 36.141 [17], shall be the same as those in clauses 8.2 to 8.4 of TS 36.141 [17].

In the referenced test requirements in this section, the term "number of RX antennas" should be replaced by the number of demodulation branches.

Annex A (normative): 
Characteristics of interfering signals

Annex B (normative): 
Environmental requirements for the BS equipment

# B.1	General

For each test in the present document, the environmental conditions under which the AAS BS is to be tested are defined. The environmental conditions and class shall be from the relevant IEC specifications or the corresponding ETSI specifications.

# B.2	Normal test environment

When a normal test environment is specified for a test, the test should be performed within the minimum and maximum limits of the conditions stated in table B.2-1.

Table B.2-1: Limits of conditions for Normal Test Environment

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | 86 kPa | 106 kPa |
| Temperature | 15C | 30C |
| Relative Humidity | 20 % | 85 % |
| Power supply | Nominal, as declared by the manufacturer |  |
| Vibration | Negligible |  |

The ranges of barometric pressure, temperature and humidity represent the maximum variation expected in the uncontrolled environment of a test laboratory. If it is not possible to maintain these parameters within the specified limits, the actual values shall be recorded in the test report.

# B.3	Extreme test environment

## B.3.1	General

The manufacturer shall declare one of the following:

1)	the equipment class for the equipment under test, as defined in the IEC 60721-3-3 [27] or ETSI EN 300 019-1-3 [29] ("Stationary use at weather protected locations");

2)	the equipment class for the equipment under test, as defined in the IEC 60721-3-4 [28] or ETSI EN 300 019-1-4 [30] ("Stationary use at non weather protected locations");

3)	the equipment that does not comply to the mentioned classes, the relevant classes from IEC 60721 [26] documentation for Temperature, Humidity and Vibration shall be declared.

NOTE:	Reduced functionality for conditions that fall outside of the standard operational conditions are not tested in the present document. These may be stated and tested separately.

## B.3.2	Extreme temperature

When an extreme temperature test environment is specified for a test, the test shall be performed at the standard minimum and maximum operating temperatures defined by the manufacturer's declaration for the equipment under test.

Minimum temperature:

The test shall be performed with the environment test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60068-2-1 [31].

Maximum temperature:

The test shall be performed with the environmental test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60068-2-2 [32].

NOTE:	It is recommended that the equipment is made fully operational prior to the equipment being taken to its lower operating temperature.

# B.4	Vibration

When vibration conditions are specified for a test, the test shall be performed while the equipment is subjected to a vibration sequence as defined by the manufacturer's declaration for the equipment under test. This shall use the environmental test equipment and methods of inducing the required environmental phenomena in to the equipment, conforming to the test procedure of IEC 60068-2-6 [33]. Other environmental conditions shall be within the ranges specified in annex B.2.

NOTE:	The higher levels of vibration may induce undue physical stress in to equipment after a prolonged series of tests. The testing body should only vibrate the equipment during the RF measurement process.

# B.5	Power supply

When extreme power supply conditions are specified for a test, the test shall be performed at the standard upper and lower limits of operating voltage defined by manufacturer's declaration for the equipment under test.

Upper voltage limit:

The equipment shall be supplied with a voltage equal to the upper limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60068-2-1 [31] Test Ab/Ad and IEC 60068-2-2 [32] Test Bb/Bd: Dry Heat.

Lower voltage limit:

The equipment shall be supplied with a voltage equal to the lower limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60068-2-1 [31] Test Ab/Ad and IEC 60068-2-2 [32] Test Bb/Bd: Dry Heat.

# B.6	Measurement of test environments

The measurement accuracy of the BS test environments shall be:

Pressure:	5 kPa.

Temperature:	2 degrees.

Relative Humidity:	5 %.

DC Voltage:	1,0 %.

AC Voltage:	1,5 %.

Vibration:	10 %.

Vibration frequency:	0,1 Hz.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

Annex C (informative):
Test tolerances and derivation of test requirements

Test requirements which are included by reference to TS 25.141 [15], TS 25.142 [19], TS 36.141 [14] or TS 37.141 [13] have been calculated within the referred test specification using the Test Tolerances defined therein.

Annex D (informative): 
Measurement system set-up

# D.1	Transmitter

## D.1.1	AAS BS output power, transmitter ON/OFF power, modulation quality, frequency error and operating band unwanted emissions

TAB connectors may be connected to the measurement equipment singularly and tested one at a time (figure D.1.1-1), or may be tested simultaneously in groups (figure D.1.1-2) where the group size may range from two to all the TAB connectors which are subject to particular transmitter test in this test setup.

In all cases the measurement is per TAB connector but the measurement may be done in parallel.

Figure D.1.1-1: Measuring system set-up for AAS BS output power, transmitter ON/OFF power,
modulation quality, frequency error and operating band unwanted emissions for a single TAB connector

Figure D.1.1-2: Measuring system set-up for AAS BS output power, transmitter ON/OFF power,
modulation quality, frequency error and operating band unwanted emissions for multiple TAB connectors

## D.1.2	Transmitter intermodulation

Figure D.1.2-1: Measuring system set-up for transmitter intermodulation

## D.1.3	Transmitter spurious emissions

TAB connectors may be connected to the measurement equipment singularly and tested one at a time (figure D.1.3-1), or may be tested simultaneously in groups (figure D.1.3-2) where the group size may range from two to all the TAB connectors which are subject to transmitter spurious emissions test.

In all cases the measurement is per TAB connector but the measurement may be done in parallel.

Figure D.1.3-1: Measuring system set-up for transmitter spurious emissions for a single TAB connector

Figure D.1.3-2: Measuring system set-up for transmitter spurious emissions for multiple TAB connectors in parallel test

# D.2	Receiver

## D.2.1	Reference sensitivity level

Figure D.2.1-1: Measuring system Set-up for Reference sensitivity level Test

## D.2.2	Receiver Dynamic Range

Figure D.2.2-1: Measuring system Set-up for Dynamic Range Test

## D.2.3	Receiver Adjacent channel selectivity and narrowband blocking

Figure D.2.3-1: Measuring system Set-up for Adjacent channel selectivity
and narrowband blocking Test

## D.2.4	Receiver spurious emissions

TAB connector(s) may be connected to the measurement equipment singularly and tested one at a time (figure D.2.2-1), or may be tested simultaneously in groups (figure D.2.2-2) where the group size may range from 2 to all the TAB connectors.

In all cases the measurement is per TAB connector but the measurement may be done in parallel.

Figure D.2.4-1: Measuring system set-up for receiver spurious emissions for a single TAB connector

Figure D.2.4-2: Measuring system set-up for receiver spurious emissions for multiple TAB connectors

## D.2.5	Receiver In-channel selectivity

Figure D.2.5-1: Measuring system Set-up for In-channel selectivity Test

## D.2.6	Receiver Intermodulation

Figure D.2.6-1: Measuring system Set-up for receiver intermodulation Test

Annex E (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-02 | RAN4#78 | R4-161117 |  |  |  | Specification structure | 0.1.0 |
| 2016-04 | RAN4#79 | R4-164273 |  |  |  | Approved text proposals in the following documents were implemented: R4-162956 - TP to TS 37.145 (part 1) - Sections 1-5 R4-162957 - TP to TS 37.145 (part 1) - Sections 6.1, 6.2, 6.3, 6.4 R4-162958 - TP to TS 37.145 (part 1) - Section 6.5 R4-162961 - TP to TS 37.145 (part 1) - Section 7 R4-162962 - TP to TS 37.145 (part 1) - Section 8 R4-162532 - TP to TS 37.145 (part 1) - Annex B | 0.2.0 |
| 2016-05 | RAN4#79 | R4-164926 |  |  |  | R4-164711 - TP to TS 37.145 (part 1) R4-164715 - TP to TS 37.145 (part 1) - Sections 6.5.3 (TAE) R4-164712 - TP to TS 37.145 (part 1) 6.6 and 6.7 | 0.3.0 |
| 2016-08 | RAN4#80 | R4-167178 |  |  |  | R4-166423 - TP to TS37.145-1 - reference corrections R4-166217 - TP for TS 37.145-1: Editorial correction on table numbers R4-166935 TP for TS 37.145-1: AAS BS Performance Target R4-166936 TP to TS 37.145-1: Annex C (Test tolerances and derivation of test requirements) R4-166937 TP to TS 37.145-1 - clean-up | 0.4.0 |
| 2016-09 | RANP#73 | RP-161449 |  |  |  | Editorial corrections after review by ETSI editHelp | 1.0.0 |
| 2016-09 | RP#73 |  |  |  |  | TS Approved by RAN plenary | 13.0.0 |
| 2016-12 | RP-74 | RP-162422 | 0001 | - | D | TS 37.145 part 1: Editorial corrections | 13.1.0 |
| 2016-12 | RP-74 | RP-162422 | 0002 | - | F | TS 37.145 part 1: Corrections related to band 65 | 13.1.0 |
| 2016-12 | RP-74 | RP-162422 | 0003 | 1 | F | MB MSR related corrections on receiver blocking | 13.1.0 |
| 2016-12 | RP-74 | RP-162422 | 0005 | - | F | Correction of Manufacturer declaration description list in TS 37.145-1 | 13.1.0 |
| 2016-12 | RP-74 | RP-162422 | 0006 | 1 | F | TS 37.145 part 1: Corrections | 13.1.0 |
| 2016-12 | RP-74 | RP-162422 | 0007 | 1 | D | CR to TS 37.145-1: Frequency error corrections | 13.1.0 |
| 2017-03 | RP-75 | RP-170586 | 0008 | - | F | RF channels for blocking test | 13.2.0 |
| 2017-03 | RP-75 | RP-170586 | 0009 | - | F | Corrections of the power range for SEM and OBUE requirement. | 13.2.0 |
| 2017-03 | RP-75 | RP-170586 | 0010 | 1 | F | TS 37.145-1: Corrections on test procedure related to support for 256QAM | 13.2.0 |
| 2017-03 | RP-75 | RP-170586 | 0011 | - | F | CR to TS 37.145-1: Removal of FFS and TBD's | 13.2.0 |
| 2017-03 | RP-75 | RP-170586 | 0012 | 1 | D | CR to TS 37.145-1: Implementation comments from MCC and drafting rules implementation | 13.2.0 |
| 2017-03 | RP-75 | - | - | - | - | Update to Rel-14 version (MCC) | 14.0.0 |
| 2017-06 | RP-76 | RP-171306 | 0058 |  | A | CR to TS 37.145-1: Isolation of Band 46 from the AAS BS specification | 14.1.0 |
| 2017-06 | RP-76 | RP-171306 | 0059 |  | A | CR to TS 37.145-1: Isolation of the NB-IoT from the AAS BS specification | 14.1.0 |
| 2017-06 | RP-76 | RP-171306 | 0060 |  | A | CR to TS 37.145-1: BS demodulation requirements update | 14.1.0 |
| 2017-06 | RP-76 | RP-171306 | 0061 |  | A | CR to TS 37.145-1: Correction of the 256QAM test requirement for EVM | 14.1.0 |
| 2017-06 | RP-76 | RP-171306 | 0062 |  | A | CR to TS 37.145-1: Update of Rel-13 references to the UTRA, EUTRA and MSR specifications | 14.1.0 |
| 2017-06 | RP-76 | RP-171306 | 0063 |  | A | CR to TS 37.145-1: Introduction of Rel-13 bands: 45, 65, 66 and 68 | 14.1.0 |
| 2017-06 | RP-76 | RP-171306 | 0064 |  | A | CR to TS 37.145-1: Correction of the test setup for the Tx spurious emissions requirement | 14.1.0 |
| 2017-06 | RP-76 | RP-171306 | 0065 |  | A | CR to TS 37.145-1: MB-MSR update | 14.1.0 |
| 2017-09 | RP-77 | RP-171968 | 0067 |  | A | CR to TS 37.145-1: Editorial corrections, Rel-14 | 14.2.0 |
| 2017-09 | RP-77 | RP-171968 | 0068 | 1 | B | CR to TS 37.145-1: introduction of bands 47, 48, 69, 70 | 14.2.0 |
| 2017-09 | RP-77 | RP-171968 | 0069 |  | F | CR to TS 37.145-1: versioned references updates to Rel-14 non-AAS specifications | 14.2.0 |
| 2018-06 | RAN#80 | RP-181109 | 0071 |  | A | CR to TS 37.145-1: corrections to the existing manufacturers declarations (4.10), Rel-14 | 14.3.0 |
| 2018-06 | RAN#80 | RP-181109 | 0073 |  | A | CR to TS 37.145-1: correction of the performance metrics for BS demod, Rel-14 | 14.3.0 |
| 2018-06 | RAN#80 | RP-181109 | 0075 |  | F | CR to TS 37.145-1: Correction of regional requirements - removal of co-location and co-existance (4.4), Rel-14 | 14.3.0 |
| 2018-06 | RAN#80 | RP-181109 | 0077 |  | A | Correction of test scope for ACLR tests | 14.3.0 |
| 2018-09 | RAN#81 | RP-181910 | 0087 |  | A | CR to TS 37.145-1: Clarification on manufacturers declarations | 14.4.0 |
| 2018-09 | RAN#81 | RP-181910 | 0095 |  | A | CR to TS 37.145-1: Correction of Occupied BS test applicability for CA, Rel-14 | 14.4.0 |
| 2018-09 | RAN#81 | RP-181910 | 0098 |  | A | CR to TS 37.145-1: E-UTRA DL RS power test requirement correction, Rel-14 | 14.4.0 |
| 2018-12 | RAN#82 | RP-182380 | 0101 |  | A | CR to TS37.145-1_Adding RF channel for CA OBW (section 4.12.1) Rel.14 | 14.5.0 |
| 2018-12 | RAN#82 | RP-182380 | 0115 |  | A | Correction of hybrid BS demodulation declarations | 14.5.0 |
| 2018-12 | RAN#82 | RP-182380 | 0118 |  | A | Cleanup to conducted requirements text | 14.5.0 |
| 2018-12 | RAN#82 | RP-182380 | 0121 |  | A | Clarification to ACLR test requirements | 14.5.0 |
| 2019-03 | RAN#83 | RP-190412 | 0130 | 1 | A | CR to TS 37.145-1: Intermodulation product bandwidth in Tx IMD test | 14.6.0 |
| 2019-03 | RAN#83 | RP-190412 | 0134 | 1 | A | TS 37.145-1: Editorial corrections | 14.6.0 |
| 2019-03 | RAN#83 | RP-190412 | 0137 |  | A | CR to TS 37.145-1 on Correction of unwanted emissions scaling | 14.6.0 |
| 2019-03 | RAN#83 | RP-190412 | 0142 |  | A | CR to TS 37.145-1: BS Spurious emissions limits for protection of the BS receiver for B28 in Europe, Rel-14 | 14.6.0 |
| 2019-03 | RAN#83 | RP-190412 | 0146 | 1 | F | CR to TS 37.145-1: Band 48 colocation correction, Rel-14 | 14.6.0 |
| 2019-06 | RAN#84 | RP-191258 | 0151 |  | A | CR to TS 37.145-1: correction of the throughput calculation in test procedures, Rel-14 | 14.7.0 |
| 2019-06 | RAN#84 | RP-191258 | 0154 |  | A | CR to TS 37.145-1: removal of "hybrid" terminology, Rel-14 | 14.7.0 |
| 2019-06 | RAN#84 | RP-191258 | 0165 |  | A | CR to TS 37.145-1: mirror of operating band and frequency range declaration from NR, Rel-14 | 14.7.0 |
| 2019-09 | RAN#85 | RP-192050 | 0193 |  | A | CR to 37.145-1: correction of equivalent TAB connectors testing, Rel-14 | 14.8.0 |
| 2020-12 | RAN#90 | RP-202513 | 0222 |  | A | CR to TS 37.145-1: correction of manufacturer | 14.9.0 |
| 2021-03 | RAN#91 | RP-210111 | 0238 |  | A | CR for 37.145-1: Corrections related to Band 24 regulatory updates | 14.10.0 |