| 3GPP TS 38.101-1 V17.1.0 (2021-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	16

Foreword	18

1	Scope	20

2	References	20

3	Definitions, symbols and abbreviations	20

3.1	Definitions	20

3.2	Symbols	21

3.3	Abbreviations	23

4	General	25

4.1	Relationship between minimum requirements and test requirements	25

4.2	Applicability of minimum requirements	25

4.3	Specification suffix information	25

5	Operating bands and channel arrangement	26

5.1	General	26

5.2	Operating bands	26

5.2A	Operating bands for CA	28

5.2A.0	General	28

5.2A.1	Intra-band CA	28

5.2A.2	Inter-band CA	29

5.2A.2.1	Inter-band CA (two bands)	29

5.2A.2.2	Inter-band CA (three bands)	33

5.2A.2.3	Inter-band CA (four bands)	35

5.2B	Operating bands for DC	35

5.2C	Operating band combination for SUL	35

5.2D	Operating bands for UL MIMO	37

5.2E	Operating band for V2X	38

5.2E.1	V2X operating bands	38

5.2E.2	V2X operating bands for con-current operation	38

5.3	UE channel bandwidth	39

5.3.1	General	39

5.3.2	Maximum transmission bandwidth configuration	39

5.3.3	Minimum guardband and transmission bandwidth configuration	40

5.3.4	RB alignment	42

5.3.5	UE channel bandwidth per operating band	42

5.3.6	Asymmetric channel bandwidths	45

5.3A	UE channel bandwidth for CA	46

5.3A.1	General	46

5.3A.2	Maximum transmission bandwidth configuration for CA	46

5.3A.3	Minimum guardband and transmission bandwidth configuration for CA	46

5.3A.4	Void	48

5.3A.5	UE channel bandwidth per operating band for CA	48

5.3E	Channel bandwidth for V2X	49

5.3E.1	General	49

5.3E.2	Channel bandwidth for V2X concurrent operation	49

5.4	Channel arrangement	51

5.4.1	Channel spacing	51

5.4.1.1	Channel spacing for adjacent NR carriers	51

5.4.2	Channel raster	51

5.4.2.1	NR-ARFCN and channel raster	51

5.4.2.2	Channel raster to resource element mapping	52

5.4.2.3	Channel raster entries for each operating band	52

5.4.3	Synchronization raster	55

5.4.3.1	Synchronization raster and numbering	55

5.4.3.3	Synchronization raster entries for each operating band	56

5.4.4	TX–RX frequency separation	59

5.4A	Channel arrangement for CA	60

5.4A.1	Channel spacing for CA	60

5.4A.2	Channel raster for CA	60

5.4A.3	Synchronization raster for CA	60

5.4A.4	Tx-Rx frequency separation for CA	60

5.4B	Reservedz	61

5.4C	Reserved	61

5.4D	Reserved	61

5.4E	Channel arrangement for V2X	61

5.4E.1	Channel spacing	61

5.4E.2	Channel raster	61

5.4E.2.1	NR-ARFCN and channel raster	61

5.4E.2.2	Channel raster to resource element mapping	61

5.4E.2.3	Channel raster entries for each operating band	61

5.4E.3	Synchronization raster for V2X	61

5.5	Void	62

5.5A	Configurations for CA	62

5.5A.0	General	62

5.5A.1	Configurations for intra-band contiguous CA	63

5.5A.2	Configurations for intra-band non-contiguous CA	66

5.5A.3	Configurations for inter-band CA	68

5.5A.3.1	Configurations for inter-band CA (two bands)	68

5.5A.3.2	Configurations for inter-band CA (three bands)	88

5.5A.3.3	Configurations for inter-band CA (four bands)	110

5.5B	Configurations for DC	115

5.5C	Configurations for SUL	117

6	Transmitter characteristics	122

6.1	General	122

6.1A	General	122

6.2	Transmitter power	122

6.2.1	UE maximum output power	122

6.2.2	UE maximum output power reduction	124

6.2.3	UE additional maximum output power reduction	127

6.2.3.1	General	127

6.2.3.2	A-MPR for NS_04	132

6.2.3.3	A-MPR for NS_10	133

6.2.3.4	A-MPR for NS_05 and NS_05U	133

6.2.3.5	A-MPR for NS_40	136

6.2.3.6	A-MPR for NS_43 and NS_43U	136

6.2.3.7	A-MPR for NS_03 and NS_03U	137

6.2.3.8	A-MPR for NS_37	138

6.2.3.9	A-MPR for NS_38	138

6.2.3.10	A-MPR for NS_39	139

6.2.3.11	A-MPR for NS_41	139

6.2.3.12	A-MPR for NS_42	140

6.2.3.13	A-MPR for NS_18	140

6.2.3.14	A-MPR for NS_21	141

6.2.3.15	A-MPR for NS_24	141

6.2.3.16	A-MPR for NS_27	143

6.2.3.17	A-MPR for NS_46	144

6.2.3.18	A-MPR for NS_47	145

6.2.3.19	A-MPR for NS_50	146

6.2.3.20	A-MPR for NS_44	146

6.2.3.21	A-MPR for NS_12	147

6.2.3.22	A-MPR for NS_13	147

6.2.3.23	A-MPR for NS_14	148

6.2.3.24	A-MPR for NS_15	149

6.2.3.25	A-MPR for NS_45	149

6.2.3.26	A-MPR for NS_48	150

6.2.3.27	A-MPR for NS_49	150

6.2.3.28	A-MPR for NS_51	151

6.2.3.29	A-MPR for NS_07	152

6.2.3.30	A-MPR for NS_52	152

6.2.4	Configured transmitted power	153

6.2A	Transmitter power for CA	155

6.2A.1	UE maximum output power for CA	155

6.2A.1.1	UE maximum output power for Intra-band contiguous CA	155

6.2A.1.2	UE maximum output power for Intra-band non-contiguous CA	155

6.2A.1.3	UE maximum output power for Inter-band CA	156

6.2A.1.4	Void	159

6.2A.1.5	Void	159

6.2A.2	UE maximum output power reduction for CA	159

6.2A.2.1	 UE maximum output power reduction for Intra-band contiguous CA	159

6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA	161

6.2A.2.2.1	MPRIM3 to meet -30dBm/MHz	162

6.2A.2.2.2	MPRIM3 to meet -13dBm/MHz	162

6.2A.2.3	UE maximum output power reduction for Inter-band CA	163

6.2A.2.4	Void	163

6.2A.3	UE additional maximum output power reduction for CA	163

6.2A.3.1.1	 UE additional maximum output power reduction for Intra-band contiguous CA	163

6.2A.3.1.1.1	A-MPR for CA_NS_04	164

6.2A.3.1.2	UE additional maximum output power reduction for Intra-band non-contiguous CA	169

6.2A.3.1.3	UE additional maximum output power reduction for Inter-band CA	171

6.2A.4	Configured output power for CA	172

6.2A.4.1	Configured transmitted power level	172

6.2A.4.1.1	Configured transmitted power for Intra-band contiguous CA	172

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA	174

6.2A.4.1.3	Configured transmitted power for Inter-band CA	176

6.2A.4.1.4	Void	179

6.2A.4.2	ΔTIB,c for CA	179

6.2A.4.2.1	Void	179

6.2A.4.2.2	Void	179

6.2A.4.2.3	ΔTIB,c for Inter-band CA (two bands)	179

6.2A.4.2.4	ΔTIB,c for Inter-band CA (three bands)	185

6.2A.4.2.5	ΔTIB,c for Inter-band CA (four bands)	190

6.2B	Transmitter power for NR-DC	190

6.2B.0	General	190

6.2B.1	UE maximum output power for NR-DC	191

6.2B.2	UE maximum output power reduction for NR-DC	191

6.2B.3	UE additional maximum output power reduction for NR-DC	191

6.2B.4.1	Configured transmitted power level for NR-DC	191

6.2B.4.2	ΔTIB,c for NR-DC	194

6.2C	Transmitter power for SUL	195

6.2C.1	Configured transmitted power for SUL	195

6.2C.2	ΔTIB,c	195

6.2D	Transmitter power for UL MIMO	196

6.2D.1	UE maximum output power for UL MIMO	196

6.2D.2	UE maximum output power reduction for UL MIMO	197

6.2D.3	UE additional maximum output power reduction for UL MIMO	198

6.2D.4	Configured transmitted power for UL MIMO	198

6.2E	Transmitter power for V2X	199

6.2E.1	UE maximum output power for V2X	199

6.2E.1.1	General	199

6.2E.1.2	UE maximum output power for V2X con-current operation	199

6.2E.2	UE maximum output power reduction for V2X	200

6.2E.2.1	General	200

6.2E.2.2	MPR for Power class 3 V2X UE	200

6.2E.2.3	MPR for Power class 3 V2X con-current operation	201

6.2E.3	UE additional maximum output power reduction for V2X	201

6.2E.3.1	General	201

6.2E.3.2	A-MPR for Power class 3 V2X UE by NS_33	202

6.2E.3.3	A-MPR for Power class 3 V2X UE by NS_52	204

6.2E.3.4	A-MPR for power class 3 V2X con-current operation	205

6.2E.4	Configured transmitted power for V2X	205

6.2E.4.1	General	205

6.2E.4.2	Configured transmitted power for V2X con-current operation	206

6.2F	Transmitter power for shared spectrum channel access	207

6.2F.1	UE maximum output power	207

6.2F.1A	UE maximum output power for CA	208

6.2F.1A.1	UE maximum output power for inter-band CA	208

6.2F.2	UE maximum output power reduction	208

6.2F.2A	UE maximum output power reduction for CA	209

6.2F.2A.1	UE maximum output power reduction for inter-band CA	209

6.2F.3	UE additional maximum output power reduction	209

6.2F.3.1	General	209

6.2F.3.2	A-MPR for NS_28	210

6.2F.3.3	A-MPR for NS_29	211

6.2F.3.4	A-MPR for NS_30	211

6.2F.3.5	A-MPR for NS_31	212

6.2F.3.6	A-MPR for NS_53	212

6.2F.3.7	A-MPR for NS_54	213

6.2F.3A	UE additional maximum output power reduction for CA	213

6.2F.3A.1	UE additional maximum output power reduction for inter-band CA	213

6.2F.4	Configured transmitted power	214

6.3	Output power dynamics	214

6.3.1	Minimum output power	214

6.3.2	Transmit OFF power	214

6.3.3	Transmit ON/OFF time mask	215

6.3.3.1	General	215

6.3.3.2	General ON/OFF time mask	215

6.3.3.3	Transmit power time mask for slot and short or long subslot boundaries	215

6.3.3.4	PRACH time mask	216

6.3.3.5	Void	217

6.3.3.6	SRS time mask	217

6.3.3.7	PUSCH-PUCCH and PUSCH-SRS time masks	218

6.3.3.8	Transmit power time mask for consecutive slot or long subslot transmission and short subslot transmission boundaries	220

6.3.3.9	Transmit power time mask for consecutive short subslot  transmissions boundaries	220

6.3.4	Power control	221

6.3.4.1	General	221

6.3.4.2	Absolute power tolerance	221

6.3.4.3	Relative power tolerance	221

6.3.4.4	Aggregate power tolerance	222

6.3A	Output power dynamics for CA	222

6.3A.1.1	Minimum output power for intra-band contiguous CA	222

6.3A.1.2	Minimum output power for intra-band non-contiguous CA	222

6.3A.1.3	Minimum output power for inter-band CA	222

6.3A.1.4	Void	223

6.3A.2	Transmit OFF power for CA	223

6.3A.2.1	Transmit OFF power for intra-band contiguous CA	223

6.3A.2.2	Transmit OFF power for intra-band non-contiguous CA	223

6.3A.2.3	Transmit OFF power for inter-band CA	223

6.3A.2.4	Void	223

6.3A.3	Transmit ON/OFF time mask for CA	223

6.3A.3.1	Transmit ON/OFF time mask for intra-band contiguous CA	223

6.3A.3.2	Transmit ON/OFF time mask for intra-band non-contiguous CA	223

6.3A.3.3	Transmit ON/OFF time mask for inter-band CA	223

6.3A.3.3.1	General	223

6.3A.3.3.2	Time mask for switching between two uplink carriers	223

6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors	224

6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	225

6.3A.3.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors	226

6.3A.3.4	Void	227

6.3A.4	Power control for CA	227

6.3A.4.1	Power control for intra-band contiguous CA	227

6.3A.4.1.1	Absolute power tolerance	227

6.3A.4.1.2	Relative power tolerance	227

6.3A.4.1.3	Aggregate power control tolerance	228

6.3A.4.2	Power control for intra-band non-contiguous CA	228

6.3A.4.2.1	Absolute power tolerance	228

6.3A.4.2.1.1	 Minimum requirements	228

6.3A.4.2.2	Relative power tolerance	228

6.3A.4.2.2.1	Minimum requirements	228

6.3A.4.2.3	Aggregate power control tolerance	229

6.3A.4.3	Power control for inter-band CA	229

6.3A.4.4	Void	229

6.3B	Output power dynamics for NR-DC	229

6.3C	Output power dynamics for SUL	229

6.3C.1	Void	229

6.3C.2	Void	229

6.3C.3	Transmit ON/OFF time mask for SUL	229

6.3C.3.1	Time mask for switching between two uplink carriers	229

6.3C.3.2	Time mask for switching between two uplink carriers with two transmit antenna connectors	230

6.3C.3.3	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	231

6.3C.3.4	Time mask for switching between two uplink bands with two transmit antenna connectors	232

6.3D	Output power dynamics for UL MIMO	233

6.3D.1	Minimum output power for UL MIMO	233

6.3D.2	Transmit OFF power for UL MIMO	233

6.3D.3	Transmit ON/OFF time mask for UL MIMO	233

6.3D.4	Power control for UL MIMO	233

6.3E	Output power dynamics for V2X	234

6.3E.1	Minimum output power for V2X	234

6.3E.1.1	General	234

6.3E.1.2	Minimum output power for V2X con-current operation	234

6.3E.2	Transmit OFF power for V2X	234

6.3E.2.1	General	234

6.3E.2.2	Transmit OFF power for V2X con-current operation	234

6.3E.3	Transmit ON/OFF time mask for V2X	235

6.3E.3.1	General	235

6.3E.3.2	General time mask	235

6.3E.3.3	S-SSB time mask	235

6.3E.3.4	Transmit ON/OFF time mask for V2X con-current operation	236

6.3E.4	Power control for V2X	236

6.3E.4.1	General	236

6.3E.4.2	Absolute power tolerance	236

6.3E.4.3	Power control for V2X con-current operation	236

6.3F	Output power dynamics for shared spectrum channel access	236

6.3F.1	Minimum output power	236

6.3F.2	Transmit OFF power	236

6.3F.3	Transmit ON/OFF time mask	236

6.3F.3.1	General	236

6.3F.3.2	General ON/OFF time mask	236

6.3F.3A	General ON/OFF mask for CA	237

6.3F.3A.1	General ON/OFF mask for inter-band CA	237

6.3F.4	Power control	237

6.3F.4.1	General	237

6.3F.4.2	Absolute power tolerance	237

6.3F.4.3	Relative power tolerance	237

6.3F.4.4	Aggregate power tolerance	237

6.4	Transmit signal quality	237

6.4.1	Frequency error	237

6.4.2	Transmit modulation quality	238

6.4.2.1	Error Vector Magnitude	238

6.4.2.1a	Error Vector Magnitude including symbols with transient period	239

6.4.2.2	Carrier leakage	239

6.4.2.3	In-band emissions	240

6.4.2.4	EVM equalizer spectrum flatness	241

6.4.2.4.1	Requirements for Pi/2 BPSK modulation	242

6.4A	Transmit signal quality for CA	243

6.4A.1	Frequency error for CA	243

6.4A.1.1	Frequency error for intra-band contiguous CA	243

6.4A.1.2	Frequency error for intra-band non-contiguous CA	243

6.4A.1.3	Frequency error for inter-band CA	243

6.4A.1.4	Void	244

6.4A.2	Transmit modulation quality for CA	244

6.4A.2.1	Transmit modulation quality for intra-band contiguous CA	244

6.4A.2.1.1	Error Vector Magnitude	244

6.4A.2.1.2	In-band emissions	244

6.4A.2.1.3	Carrier leakage	247

6.4A.2.2	Transmit modulation quality for intra-band non-contiguous CA	247

6.4A.2.2.1	Error Vector Magnitude	247

6.4A.2.2.2	In-band emissions	248

6.4A.2.3	Transmit modulation quality for inter-band CA	248

6.4A.2.4	Void	248

6.4B	Transmit signal quality for NR-DC	248

6.4D	Transmit signal quality for UL MIMO	248

6.4D.1	Frequency error for UL MIMO	248

6.4D.2	Transmit modulation quality for UL MIMO	248

6.4D.2.1	Error Vector Magnitude	249

6.4D.2.2	Carrier leakage	249

6.4D.2.3	In-band emissions	249

6.4D.2.4	EVM equalizer spectrum flatness for UL MIMO	249

6.4D.3	Time alignment error for UL MIMO	249

6.4D.4	Requirements for coherent UL MIMO	249

6.4E	Transmit signal quality for V2X	250

6.4E.1	Frequency error for V2X	250

6.4E.1.1	General	250

6.4E.1.2	Frequency error for V2X con-current operation	250

6.4E.2	Transmit modulation quality for V2X	250

6.4E.2.1	General	250

6.4E.2.2	Error Vector Magnitude for V2X	250

6.4E.2.3	Carrier leakage for V2X	250

6.4E.2.4	In-band emissions for V2X	250

6.4E.2.5	EVM equalizer spectrum flatness for V2X	251

6.4E.2.6	Transmit modulation quality for V2X con-current operation	251

6.4F	Transmit signal quality for shared spectrum channel access	251

6.4F.1	Frequency error	251

6.4F.2	Transmit modulation quality	251

6.4F.2.1	Error Vector Magnitude	251

6.4F.2.2	Carrier leakage	251

6.4F.2.3	In-band emissions	251

6.4F.2.4	EVM equalizer spectrum flatness	252

6.4F.2A	Transmit modulation quality for CA	252

6.4F.2A.1	Transmit modulation quality for inter-band CA	252

6.5	Output RF spectrum emissions	253

6.5.1	Occupied bandwidth	253

6.5.2	Out of band emission	253

6.5.2.1	General	253

6.5.2.2	Spectrum emission mask	253

6.5.2.3	Additional spectrum emission mask	254

6.5.2.3.1	Requirements for network signalling value "NS_35"	254

6.5.2.3.2	Requirements for network signalling value "NS_04"	255

6.5.2.3.3	Requirements for network signalling value "NS_03" and "NS_21"	255

6.5.2.3.4	Requirements for network signalling value "NS_06" or “NS_07”	256

6.5.2.3.5	Void	257

6.5.2.3.6	Void	257

6.5.2.3.7	Void	257

6.5.2.3.8	Requirements for network signalled value "NS_27"	257

6.5.2.4	Adjacent channel leakage ratio	257

6.5.2.4.1	NR ACLR	257

6.5.2.4.2	UTRA ACLR	258

6.5.3	Spurious emissions	258

6.5.3.1	General spurious emissions	259

6.5.3.2	Spurious emissions for UE co-existence	259

6.5.3.3	Additional spurious emissions	267

6.5.3.3.1	Requirement for network signalling value "NS_04"	267

6.5.3.3.2	Requirement for network signalling value "NS_17"	268

6.5.3.3.3	Requirement for network signalling value "NS_18"	268

6.5.3.3.4	Requirement for network signalling value "NS_05"	268

6.5.3.3.5	Requirement for network signalling value "NS_43"	268

6.5.3.3.6	Requirement for network signalling value "NS_37"	269

6.5.3.3.7	Requirement for network signalling value "NS_38"	269

6.5.3.3.8	Requirement for network signalling value "NS_39"	269

6.5.3.3.9	Requirement for network signalling value "NS_40"	269

6.5.3.3.10	Requirement for network signalling value "NS_41"	270

6.5.3.3.11	Requirement for network signalling value "NS_42"	270

6.5.3.3.12	Requirement for network signalling value "NS_21"	270

6.5.3.3.13	Requirement for network signalling value "NS_24"	270

6.5.3.3.14	Requirement for network signalling value "NS_27"	271

6.5.3.3.15	Requirement for network signalling value "NS_47"	271

6.5.3.3.16	Requirement for network signalling value "NS_50"	271

6.5.3.3.17	Requirement for network signalled value "NS_12"	272

6.5.3.3.18	Requirement for network signalled value "NS_13"	272

6.5.3.3.19	Requirement for network signalled value "NS_14"	272

6.5.3.3.20	Requirement for network signalled value "NS_15"	272

6.5.3.3.21	Requirement for network signalled value "NS_45"	273

6.5.3.3.22	Requirement for network signalled values "NS_48" and "NS_51"	273

6.5.3.3.23	Requirement for network signalled value "NS_49"	273

6.5.3.3.24	Requirement for network signalled value "NS_44"	274

6.5.3.3.25	Requirement for network signalled value "NS_46"	274

6.5.3.3.26	Requirement for network signalled value "NS_07"	274

6.5.3.3.27 	Requirement for network signalled value “NS_52”	275

6.5.4	Transmit intermodulation	275

6.5A	Output RF spectrum emissions for CA	276

6.5A.1	Occupied bandwidth for CA	276

6.5A.1.1	Void	276

6.5A.1.1a	Occupied bandwidth for Intra-band contiguous CA	276

6.5A.1.2	Occupied bandwidth for Intra-band non-contiguous CA	276

6.5A.1.3	Occupied bandwidth for Inter-band CA	276

6.5A.2	Out of band emission for CA	276

6.5A.2.1	General	276

6.5A.2.2	Spectrum emission mask	276

6.5A.2.2.1	Spectrum emission mask for intra-band contiguous C	276

6.5A.2.2.2	Spectrum emission mask for intra-band non-contiguous CA	277

6.5A.2.2.3	Spectrum emission mask for Inter-band CA	277

6.5.A.2.2.4	Void	277

6.5A.2.3	Additional spectrum emission mask for CA	277

6.5A.2.3.1	Additional spectrum emission mask for intra-band contiguous CA	277

6.5A.2.3.2	Additional spectrum emission mask for Intra-band non-contiguous CA	278

6.5A.2.3.2.1 Minimum requirement (network signalled value “CA_NC_NS_04”)	278

6.5A.2.3.3	Additional spectrum emission mask for Inter-band CA	278

6.5A.2.4	Adjacent channel leakage ratio	278

6.5A.2.4.1	NR ACLR	278

6.5A.2.4.2	UTRA ACLR	279

6.5A.3	Spurious emission for CA	279

6.5A.3.1	General spurious emissions	279

6.5A.3.2	Spurious emissions for UE co-existence	280

6.5A.3.2.1	Spurious emissions for UE co-existence for intra-band contiguous CA	280

6.5A.3.2.2	Spurious emissions for UE co-existence for intra-band non-contiguous CA	281

6.5A.3.2.3	Spurious emissions for UE co-existence for Inter-band CA	282

6.5A.3.2.4	Void	294

6.5A.3.2.5	Void	294

6.5A.3.2.6	Void	294

6.5A.3.3	Additional spurious emissions for CA	294

6.5A.3.3.1	Additional spurious emissions for intra-band contiguous  CA	294

6.5A.4	Transmit intermodulation for CA	295

6.5A.4.2.1	Transmit intermodulation for intra-band contiguous CA	295

6.5A.4.2.2	Void	295

6.5B	Output RF spectrum emissions for NR-DC	295

6.5D	Output RF spectrum emissions for UL MIMO	295

6.5D.1	Occupied bandwidth for UL MIMO	295

6.5D.2	Out of band emission for UL MIMO	295

6.5D.3	Spurious emission for UL MIMO	296

6.5D.4	Transmit intermodulation for UL MIMO	296

6.5E	Output RF spectrum emissions for V2X	296

6.5E.1	Occupied bandwidth for V2X	296

6.5E.1.1	General	296

6.5E.1.2	Occupied bandwidth for V2X con-current operation	296

6.5E.2	Out of band emission for V2X	297

6.5E.2.1	General	297

6.5E.2.2	Spectrum emission mask	297

6.5E.2.2.1	General	297

6.5E.2.2.2	Spectrum emission mask for V2X con-current operation	297

6.5E.2.3	Additional Spectrum emission mask	297

6.5E.2.3.1	Requirements for network signalled value "NS_33"	297

6.5E.2.3.2	Requirements for network signalled value "NS_52"	298

6.5E.2.4	Adjacent channel leakage ratio	298

6.5E.2.4.1	General	298

6.5E.2.4.2	ACLR for V2X con-current operation	298

6.5E.3	Spurious emissions for V2X	298

6.5E.3.1	General spurious emissions	298

6.5E.3.2	Spurious emissions for UE co-existence	299

6.5E.3.3	Spurious emissions for UE co-existence for V2X con-current operation	299

6.5E.3.4	Additional spurious emissions requirements for V2X	300

6.5E.3.4.1	General	300

6.5E.3.4.2	Requirements for network signalled value "NS_33"	300

6.5E.3.4.3	Void	300

6.5E.4	Transmit intermodulation	300

6.5E.4.1	General	300

6.5E.4.2	Transmit intermodulation for V2X con-current operation	300

6.5F	Output RF spectrum emissions	301

6.5F.1	Occupied bandwidth	301

6.5F.2	Out of band emission	301

6.5F.2.1	General	301

6.5F.2.2	Spectrum emission mask for operation with shared spectrum channel access	301

6.5F.2.2.1	Spectrum emission mask for non-transmitted channels	302

6.5F.2.3	Additional spectrum emission mask	302

6.5F.2.4	Adjacent channel leakage ratio	302

6.5F.2.4.1	Shared spectrum channel access ACLR	302

6.5F.2.4.2	Additional requirement for network signaled value "NS_29"	302

6.5F.3	Spurious emissions	303

6.5F.3.1	General spurious emissions	303

6.5F.3.2	Spurious emissions for UE co-existence	303

6.5F.3.3	Additional spurious emissions	303

6.5F.3.3.1	Requirement for network signalled value "NS_28"	303

6.5F.3.3.2	Requirement for network signalled value "NS_29"	303

6.5F.3.3.3	Requirement for network signalled value "NS_30"	305

6.5F.3.3.4	Requirement for network signalled value "NS_31"	306

6.5F.3.3.5	Requirements for network signalled value "NS_53" or "NS_54"	307

6.5F.4	Transmit intermodulation	307

6.6	Void	307

6.6E	Time alignment error	307

7	Receiver characteristics	308

7.1	General	308

7.1A	General	308

7.2	Diversity characteristics	309

7.3	Reference sensitivity	309

7.3.1	General	309

7.3.2	Reference sensitivity power level	309

7.3.3	ΔRIB,c	319

7.3A	Reference sensitivity for CA	320

7.3A.1	General	320

7.3A.2	Reference sensitivity power level for CA	320

7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA	320

7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA	320

7.3A.2.3	Reference sensitivity power level for Inter-band CA	321

7.3A.2.4	Void	321

7.3A.3	ΔRIB,c for CA	321

7.3A.3.1	General	321

7.3A.3.2	ΔRIB,c for Inter-band CA	322

7.3A.3.2.1	ΔRIB,c for two bands	323

7.3A.3.2.2	Void	326

7.3A.3.2.3	ΔRIB,c for three bands	326

7.3A.3.2.4	ΔRIB,c for four bands	330

7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA	330

7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	336

7.3A.6	Reference sensitivity exceptions due to cross band isolation for CA	347

7.3B	Reference sensitivity for NR-DC	348

7.3C	Reference sensitivity for SUL	348

7.3C.1	General	348

7.3C.2	Reference sensitivity power level for SUL	348

7.3C.3	ΔRIB,c for SUL	351

7.3C.3.1	General	351

7.3C.3.2	SUL band combination	351

7.3C.3.2.1	ΔRIB,c  for two bands	352

7.3C.3.2.2	ΔRIB,c  for three bands	352

7.3D	Reference sensitivity for UL MIMO	352

7.3E	Reference sensitivity for V2X	353

7.3E.1	General	353

7.3E.2	Minimum requirements	353

7.3E.3	Reference sensitivity power level for V2X con-current operation	353

7.3F	Reference sensitivity for shared spectrum channel access	356

7.3F.1	General	356

7.3F.2	Reference sensitivity power level	356

7.3F.3	ΔRIB,c	357

7.3F.4	Intra-band contiguous shared spectrum channel access CA	357

7.3G.5	Inter-band CA with shared spectrum channel access	357

7.3G.5.1	Reference sensitivity exceptions due to UL harmonic interference	357

7.3F.5.2	Reference sensitivity exceptions due to cross band isolation	358

7.4	Maximum input level	358

7.4A	Maximum input level for CA	359

7.4A.1	Maximum input level for Intra-band contiguous CA	359

7.4A.2	Maximum input level for Intra-band non-contiguous CA	359

7.4A.3	Maximum input level for Inter-band CA	359

7.4B	Maximum input level for NR-DC	360

7.4D	Maximum input level for UL MIMO	360

7.4E	Maximum input level for V2X	360

7.4E.1	General	360

7.4E.2	Maximum input level for V2X con-current operation	360

7.5	Adjacent channel selectivity	360

7.5A	Adjacent channel selectivity for CA	365

7.5A.1	Adjacent channel selectivity for Intra-band contiguous CA	365

7.5A.2	Adjacent channel selectivity Intra-band non-contiguous CA	367

7.5A.3	Adjacent channel selectivity Inter-band CA	368

7.5B	Adjacent channel selectivity for NR-DC	368

7.5D	Adjacent channel selectivity for UL MIMO	368

7.5E	Adjacent channel selectivity for V2X	368

7.5E.1	General	368

7.5E.2	Adjacent channel selectivity for V2X con-current operation	369

7.5F	Adjacent channel selectivity	369

7.5F.1	General	369

7.5F.2	Intra-band contiguous shared spectrum channel access CA	370

7.6	Blocking characteristics	371

7.6.1	General	371

7.6.2	In-band blocking	371

7.6.3	Out-of-band blocking	375

7.6.4	Narrow band blocking	378

7.6A	Blocking characteristics for CA	378

7.6A.1	General	378

7.6A.2	In-band blocking for CA	378

7.6A.2.1	In-band blocking for Intra-band contiguous CA	378

7.6A.2.2	In-band blocking for Intra-band non-contiguous CA	380

7.6A.2.3	In-band blocking for Inter-band CA	380

7.6A.3	Out-of-band blocking for CA	381

7.6A.3.1	Out-of-band blocking for Intra-band contiguous CA	381

7.6A.3.2	Out-of-band blocking for Intra-band non-contiguous CA	382

7.6A.3.3	Out-of-band blocking for Inter-band CA	382

7.6A.4	Narrow band blocking for CA	383

7.6A.4.1	Narrow band blocking for Intra-band contiguous CA	383

7.6A.4.2	Narrow band blocking for Intra-band non-contiguous CA	384

7.6A.4.3	Narrow band blocking for Inter-band CA	384

7.6B	Blocking characteristics for NR-DC	385

7.6C	Blocking characteristics for SUL	385

7.6C.1	General	385

7.6C.2	In-band blocking for SUL	385

7.6C.3	Out-of-band blocking for SUL	385

7.6C.4	Narrow band blocking for SUL	385

7.6D	Blocking characteristics for UL MIMO	386

7.6E	Blocking characteristics for V2X	386

7.6E.1	General	386

7.6E.2	In-band blocking	386

7.6E.2.1	General	386

7.6E.2.2	In-band blocking for V2X con-current operation	387

7.6E.3	Out-of-band blocking	387

7.6E.3.1	General	387

7.6E.3.2	Out-of-band blocking for V2X con-current operation	387

7.6F	Blocking characteristics	387

7.6F.1	General	387

7.6F.2	In-band blocking	388

7.6F.2.1	General	388

7.6F.2.2	Intra-band contiguous shared spectrum channel access CA	388

7.6F.3	Out-of-band blocking	389

7.6F.3.1	General	389

7.6F.3.2	Intra-band contiguous shared spectrum channel access CA	390

clause7.7	Spurious response	391

7.7A	Spurious response for CA	392

7.7A.1	Spurious response for Intra-band contiguous CA	392

7.7A.2	Spurious response for Intra-band non-contiguous CA	393

7.7A.3	Spurious response for Inter-band CA	393

7.7B	Spurious response for NR-DC	393

7.7D	Spurious response for UL MIMO	393

7.7E	Spurious response for V2X	393

7.7.E.1	General	393

7.7E.2	Spurious response for V2X con-current operation	394

7.7F	Spurious response for shared spectrum channel access	394

7.7F.1	General	394

7.7F.2	Intra-band contiguous shared spectrum channel access CA	394

7.8	Intermodulation characteristics	395

7.8.1	General	395

7.8.2	Wide band Intermodulation	395

7.8A	Intermodulation characteristics for CA	397

7.8A.1	General	397

7.8A.2	Wide band intermodulation for CA	397

7.8A.2.1	Wide band intermodulation for Intra-band contiguous CA	397

7.8A.2.2	Wide band intermodulation for Intra-band non-contiguous CA	398

7.8A.2.3	Wide band intermodulation for Inter-band CA	398

7.8B	Intermodulation characteristics for NR-DC	398

7.8D	Intermodulation characteristics for UL MIMO	399

7.8E	Intermodulation characteristics for V2X	399

7.8E.1	General	399

7.8E.2	Wide band Intermodulation	399

7.8E.3	Intermodulation for V2X con-current operation	399

7.8F	Intermodulation characteristics for shared spectrum channel access	399

7.8F.1	General	399

7.8F.2	Wide band Intermodulation	400

7.9	Spurious emissions	400

7.9A	Spurious emissions for CA	401

7.9A.1	Void	401

7.9A.2	Void	401

7.9A.3	Spurious emissions for Inter-band CA	401

7.9B	Spurious emissions for NR-DC	401

Annex A (normative): Measurement channels	402

A.1	General	402

A.2	UL reference measurement channels	402

A.2.1	General	402

A.2.2	Reference measurement channels for FDD	403

A.2.2.1	DFT-s-OFDM Pi/2-BPSK	403

A.2.2.2	DFT-s-OFDM QPSK	406

A.2.2.3	DFT-s-OFDM 16QAM	409

A.2.2.4	DFT-s-OFDM 64QAM	412

A.2.2.5	DFT-s-OFDM 256QAM	415

A.2.2.6	CP-OFDM QPSK	418

A.2.2.7	CP-OFDM 16QAM	421

A.2.2.8	CP-OFDM 64QAM	424

A.2.2.9	CP-OFDM 256QAM	427

A.2.3	Reference measurement channels for TDD	429

A.2.3.1	DFT-s-OFDM Pi/2-BPSK	430

A.2.3.2	DFT-s-OFDM QPSK	433

A.2.3.3	DFT-s-OFDM 16QAM	437

A.2.3.4	DFT-s-OFDM 64QAM	440

A.2.3.5	DFT-s-OFDM 256QAM	443

A.2.3.6	CP-OFDM QPSK	446

A.2.3.7	CP-OFDM 16QAM	449

A.2.3.8	CP-OFDM 64QAM	452

A.2.3.9	CP-OFDM 256QAM	455

A.3	DL reference measurement channels	458

A.3.1	General	458

A.3.2	DL reference measurement channels for FDD	459

A.3.2.1	General	459

A.3.2.2	FRC for receiver requirements for QPSK	459

A.3.2.3	FRC for maximum input level for 64QAM	462

A.3.2.4	FRC for maximum input level for 256 QAM	465

A.3.3	DL reference measurement channels for TDD	468

A.3.3.1	General	468

A.3.3.2	FRC for receiver requirements for QPSK	469

A.3.3.3	FRC for maximum input level for 64QAM	472

A.3.3.4	FRC for maximum input level for 256 QAM	475

A.4	CSI reference measurement channels	478

A.5	OFDMA Channel Noise Generator (OCNG)	478

A.5.1	OCNG Patterns for FDD	478

A.5.1.1	OCNG FDD pattern 1: Generic OCNG FDD Pattern for all unused REs	478

A.5.2	OCNG Patterns for TDD	478

A.5.2.1	OCNG TDD pattern 1: Generic OCNG TDD Pattern for all unused REs	478

A.6	Void	479

A.7	V2X reference measurement channels	479

A.7.1	General	479

A.7.2	FRC for maximum input level for QPSK	479

A.7.3	FRC for maximum input level for 64QAM	481

A.7.4	FRC for maximum input level for 256QAM	482

Annex B (informative): Void	484

Annex C (informative): Downlink physical channels	485

C.1	General	485

C.2	Setup	485

C.3	Connection	485

C.3.1	Measurement of Receiver Characteristics	485

Annex D (normative): Characteristics of the interfering signal	487

D.1 General	487

D.2	Interference signals	487

Annex E (normative):  Environmental conditions	488

E.1	General	488

E.2	Environmental	488

E.2.1	Temperature	488

E.2.2	Voltage	489

E.2.3	Vibration	489

Annex F (normative):  Transmit modulation	490

F.0	General	490

F.1	Measurement Point	490

F.2	Basic Error Vector Magnitude measurement	490

F.3	Basic in-band emissions measurement	491

F.4	Modified signal under test	491

F.5	Window length	494

F.5.1	Timing offset	494

F.5.2	Window length	494

F.5.3	Window length for normal CP	494

F.5.4	Window length for Extended CP	495

F.5.5	Window length for PRACH	496

F.6	Averaged EVM	497

F.7	Spectrum Flatness	498

Annex G (informative): Void	500

Annex H (informative): Void	500

Annex I (informative): Void	500

Annex J (informative): Void	500

Annex K (informative): Void	500

Annex L (normative): ModifiedMPR-Behavior	500

L.1	Indication of modified MPR behavior	500

Annex M (informative): Change history	502

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document establishes the minimum RF requirements for NR User Equipment (UE) operating on frequency Range 1.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

For a specific reference, subsequent revisions do not apply.

For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[3]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[4]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone".

[5]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[6]	3GPP TS 38.211: "NR; Physical channels and modulation".

[7]	3GPP TS 38.331: "Radio Resource Control (RRC) protocol specification".

[8]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[9]	ITU-R Recommendation SM.329-10, "Unwanted emissions in the spurious domain".

[10]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[11]	3GPP TS 36.101: Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception;

[12]	ETSI TS 102 792: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (CEN DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gaps.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Sub-band:  For a UE that supports shared spectrum channel access in wideband operation, a sub-band is the set of RBs within an approximately 20 MHz segment of the channel where the wideband channel is uniformly divided into an integer number of 20 MHz sub-bands.  Sub-bands may be separately allocated in uplink and downlink.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Δ Frequency of default TX-RX separation of the FDD operating band

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RBΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band EN-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max( BWGB,Channel(k) )

BWGB,Channel(k)	Minimum guard band defined in clause 5.3A.1 of carrier k

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,Shift	RF reference frequency for Supplementary Uplink (SUL) bands and for the uplink for all FDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

GBChannel	Minimum guard band defined in clause 5.3.3

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation

expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj*2μj for carrier 1 to j, where μ is defined in TS 38.211 [6]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211 [6]

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance)

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart	Indicates the lowest RB index of transmitted resource blocks

RBstart_CA	Indicates the lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.1

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.1

tp		Transient Period value signalled by the UE

tpstart		Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in clause 6.2.1

SSREF	SS block reference frequency position

UTRAACLR	UTRA ACLR

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BS	Base Station

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable NR operating band

CC	Component Carriers

CG	Carrier Group

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DC	Dual Connectivity

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

E-UTRA	Evolved UTRA

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FR	Frequency Range

FRC	Fixed Reference Channel

FWA	Fixed Wireless Access

GSCN	Global Synchronization Channel Number

IBB	In-band Blocking

IDFT	Inverse Discrete Fourier Transformation

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MCG	Master Cell Group

MOP	Maximum Output Power

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NS	Network Signalling

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

P-MPR	Power Management Maximum Power Reduction

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

QAM	Quadrature Amplitude Modulation

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

RMS	Root Mean Square (value)

RSRP	Reference Signal Receiving PowerRx	Receiver

SC	Single Carrier

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SDL	Supplementary Downlink

SEM	Spectrum Emission Mask

SL	Sidelink

SL-MIMO	Sidelink-Multiple Antenna transmission

SNR	Signal-to-Noise Ratio

SRS	Sounding Reference Symbol

SUL	Supplementary uplink

SS	Synchronization Symbol

TAE	Time Alignment Error

TAG	Timing Advance Group

Tx	Transmitter

UL MIMO	Uplink Multiple Antenna transmission

ULFPTx	Uplink Full Power Transmission

V2X	Vehicle to Everything

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

The present document is a Single-RAT specification for NR UE, covering RF characteristics and minimum performance requirements. Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 38.521-1 [4].

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification TS 38.521-1 [4] defines test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in Recommendation ITU-R M.1545 [5].

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigurationDedicated in the PCell and SCells for NR SA.

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffixes A to F) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes A to F) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause.

A terminal which supports more than one feature in clauses 5, 6 and 7 shall meet all of the separate corresponding requirements.

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly. For a terminal that supports SUL, the current version of the specification assumes the terminal is not configured with UL MIMO on SUL carrier.

For a terminal that supports operation in shared spectrum, the current version of this specification assumes in the uplink sub-bands within a wideband channel shall be contiguously allocated to the UE.  The uplink requirements for one or more non-transmitted sub-bands between two transmitted sub-bands does not form a part of the current version of this specification.

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The present specification covers FR1 operating bands.

## 5.2	Operating bands

NR is designed to operate in the FR1 operating bands defined in Table 5.2-1.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n13 | 777 MHz – 787 MHz | 746 MHz – 756 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n2416 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n26 | 814 MHz – 849 MHz | 859 MHz – 894 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n303 | 2305 MHz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n3810 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD13 |
| n4711 | 5855 MHz – 5925 MHz | 5855 MHz – 5925 MHz | TDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n7712 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n89 | 824 MHz – 849 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| n91 | 832 MHz – 862 MHz | 1427 MHz – 1432 MHz | FDD9 |
| n92 | 832 MHz – 862 MHz | 1432 MHz – 1517 MHz | FDD9 |
| n93 | 880 MHz – 915 MHz | 1427 MHz – 1432 MHz | FDD9 |
| n94 | 880 MHz – 915 MHz | 1432 MHz – 1517 MHz | FDD9 |
| n958 | 2010 MHz – 2025 MHz | N/A | SUL |
| n9614 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD13 |
| n9715 | 2300 MHz – 2400 MHz | N/A | SUL |
| n9815 | 1880 MHz – 1920 MHz | N/A | SUL |
| n9916 | 1626.5 MHz – 1660.5 MHz | N/A | SUL |
| NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. NOTE 3: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 4: A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements. NOTE 5: Unless otherwise stated, the applicability of requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the requirements for Band n41. A UE supporting Band n90 shall also support band n41. NOTE 6: A UE that supports NR Band n66 shall receive in the entire DL operating band. NOTE 7: A UE that supports NR Band n66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_n66B and CA_n66(2A) in the current version of the specification. NOTE 8: This band is applicable in China only. NOTE 9: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 10: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. NOTE 11: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. NOTE 12: In the USA this band is restricted to 3700 – 3980 MHz. NOTE 13: This band is restricted to operation with shared spectrum channel access as defined in 37.213. NOTE 14: This band is applicable in the USA only subject to FCC Report and Order FCC 20-51 NOTE 15: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 16: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. |  |  |  |

## 5.2A	Operating bands for CA

### 5.2A.0	General

CA operating bands including Band n90 are defined by the corresponding CA operating bands including Band n41 with Band n90 replacing Band n41. For brevity the said CA operating bands including Band n90 are not listed in the tables below but are covered by this specification.

### 5.2A.1	Intra-band CA

NR intra-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2, where all operating bands are within FR1.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1 | n1 |
| CA_n7 | n7 |
| CA_n40 | n40 |
| CA_n41 | n41 |
| CA_n461 | n46 |
| CA_n48 | n48 |
| CA_n66 | n66 |
| CA_n71 | n71 |
| CA_n77 | n77 |
| CA_n78 | n78 |
| CA_n79 | n79 |
| NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. |  |

Table 5.2A.1-2: Intra-band non-contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3(*) | n3 |
| CA_n7(*) | n7 |
| CA_n25(*) | n25 |
| CA_n41(*) | n41 |
| CA_n48(*) | n48 |
| CA_n66(*) | n66 |
| CA_n77(*) | n77 |
| CA_n78(*) | n78 |
| NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 2: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in 5.5A.2. |  |

### 5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, 5.2A.2.2-1 and Table 5.2A.2.3-1, where all operating bands are within FR1.

Table 5.2A.2-1: Void

Table 5.2A.2-2: Void

Table 5.2A.2-3: Void

#### 5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n1-n3 | n1, n3 |  |
| CA_n1-n7 | n1, n7 |  |
| CA_n1-n8 | n1, n8 |  |
| CA_n1-n28 | n1, n28 |  |
| CA_n1-n40 | n1, n40 |  |
| CA_n1-n41 | n1, n41 |  |
| CA_n1-n77 | n1, n77 | No |
| CA_n1-n78 | n1, n78 | No |
| CA_n1-n79 | n1, n79 | No |
| CA_n2-n5 | n2, n5 |  |
| CA_n2-n48 | n2, n48 |  |
| CA_n2-n66 | n2, n66 |  |
| CA_n2-n77 | n2, n77 |  |
| CA_n2-n78 | n2, n78 |  |
| CA_n3-n7 | n3, n7 |  |
| CA_n3-n8 | n3, n8 |  |
| CA_n3-n18 | n3, n18 |  |
| CA_n3-n28 | n3, n28 |  |
| CA_n3-n38 | n3, n38 |  |
| CA_n3-n40 | n3, n40 |  |
| CA_n3-n41 | n3, n41 | No |
| CA_n3-n771 | n3, n77 | No |
| CA_n3-n781 | n3, n78 | No |
| CA_n3-n791 | n3, n79 | No |
| CA_n5-n7 | n5, n7 |  |
| CA_n5-n25 | n5, n25 |  |
| CA_n5-n48 | n5, n48 |  |
| CA_n5-n66 | n5, n66 |  |
| CA_n5-n77 | n5, n77 |  |
| CA_n5-n78 | n5, n78 | No |
| CA_n5-n79 | n5, n79 | No |
| CA_n7-n25 | n7, n25 |  |
| CA_n7-n28 | n7, n28 |  |
| CA_n7-n66 | n7, n66 |  |
| CA_n7-n77 | n7, n77 |  |
| CA_n7-n78 | n7, n78 |  |
| CA_n8-n20 | n8, n20 |  |
| CA_n8-n28 | n8, n28 |  |
| CA_n8-n391 | n8, n39 |  |
| CA_n8-n40 | n8, n40 |  |
| CA_n8-n41 | n8, n41 | No |
| CA_n8-n751 | n8, n75 |  |
| CA n8-n781 | n8, n78 | No |
| CA_n8-n791 | n8, n79 | No |
| CA_n13-n25 | n13, n25 |  |
| CA_n13-n66 | n13, n66 |  |
| CA_n18-n41 | n18, n41 |  |
| CA_n20-n282 | n20, n28 |  |
| CA_n20-n75 | n20, n75 |  |
| CA_n20-n78 | n20, n78 |  |
| CA_n25-n29 | n25, n29 |  |
| CA_n25-n38 | n25, n38 |  |
| CA_n25-n41 | n25, n41 |  |
| CA_n25-n466 | n25, n46 |  |
| CA_n25-n48 | n25, n48 |  |
| CA_n25-n66 | n25, n66 |  |
| CA_n25-n71 | n25, n71 |  |
| CA_n25-n77 | n25, n77 |  |
| CA_n25-n78 | n25,n78 |  |
| CA_n28-n40 | n28, n40 |  |
| CA_n28-n41 | n28, n41 |  |
| CA_n28-n50 | n28, n50 |  |
| CA_n28-n752 | n28, n75 |  |
| CA_n28-n77 | n28, n77 | No |
| CA_n28-n781 | n28, n78 | No |
| CA_n28-n79 | n28, n79 |  |
| CA_n29-n66 | n29, n66 |  |
| CA_n29-n70 | n29, n70 |  |
| CA_n34-n791 | n34, n79 |  |
| CA_n38-n66 | n38, n66 |  |
| CA_n38-n781 | n38, n78 |  |
| CA_n39-n40 | n39, n40 |  |
| CA_n39-n41 | n39, n41 | No |
| CA_n39-n791 | n39, n79 | No |
| CA_n40-n41 | n40, n41 |  |
| CA_n40-n78 | n40, n78 |  |
| CA_n40-n791,4 | n40, n79 | No |
| CA_n41-n501 | n41, n50 |  |
| CA_n41-n66 | n41, n66 |  |
| CA_n41-n711 | n41, n71 |  |
| CA_n41-n771 | n41, n77 |  |
| CA_n41-n78 | n41, n78 |  |
| CA_n41-n791,3 | n41, n79 | No |
| CA_n46-n486 | n46, n48 |  |
| CA_n46-n666 | n46, n66 |  |
| CA_n48-n66 | n48, n66 |  |
| CA_n50-n78 | n50, n78 |  |
| CA_n66-n70 | n66, n70 |  |
| CA_n66-n71 | n66, n71 |  |
| CA_n66-n77 | n66, n77 |  |
| CA_n66-n78 | n66, n78 |  |
| CA_n70-n71 | n70, n71 |  |
| CA_n71-n77 | n71, n77 |  |
| CA_n71-n78 | n71, n78 |  |
| CA_n75-n781 | n75, n78 |  |
| CA_n76-n781 | n76, n78 |  |
| CA_n77-n795,7 | n77, n79 |  |
| CA_n78-n795 | n78, n79 |  |
| CA_n78-n92 | n78, n92 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 3: The frequency range below 2506 MHz for Band n41 is not used in this combination. NOTE 4: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 6: The PCell is allocated in the licensed band in this combination. NOTE 7: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 8: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

#### 5.2A.2.2	Inter-band CA (three bands)

Table 5.2A.2.2-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3-n7 | n1, n3, n7 |
| CA_n1-n3-n8 | n1, n3, n8 |
| CA_n1-n3-n28 | n1, n3, n28 |
| CA_n1-n3-n41 | n1, n3, n41 |
| CA_n1-n3-n78 | n1, n3, n78 |
| CA_n1-n7-n28 | n1, n7, n28 |
| CA_n1-n7-n78 | n1，n7, n78 |
| CA_n1-n8-n78 | n1, n8, n78 |
| CA_n1-n8-n79 | n1, n8, n79 |
| CA_n1-n28-n78 | n1, n28, n78 |
| CA_n1-n40-n78 | n1, n40, n78 |
| CA_n1-n77-n79 | n1, n77, n79 |
| CA_n1-n78-n79 | n1, n78, n79 |
| CA_n2-n66-n77 | n2, n66, n77 |
| CA_n3-n5-n7 | n3, n5, n7 |
| CA_n3-n7-n28 | n3, n7, n28 |
| CA_n3-n7-n78 | n3, n7, n78 |
| CA_n3-n8-n78 | n3, n8, n78 |
| CA_n3-n18-n41 | n3, n18, n41 |
| CA_n3-n28-n41 | n3, n28, n41 |
| CA_n3-n28-n77 | n3, n28, n77 |
| CA_n3-n28-n78 | n3, n28, n78 |
| CA_n3-n40-n41 | n3, n40, n41 |
| CA_n3-n41-n77 | n3, n41, n77 |
| CA_n3-n41-n78 | n3, n41, n78 |
| CA_n3-n41-n79 | n3, n41, n79 |
| CA_n5-n7-n78 | n5, n7, n78 |
| CA_n5-n25-n66 | n5, n25, n66 |
| CA_n5-n25-n77 | n5, n25, n77 |
| CA_n5-n25-n78 | n5, n25, n78 |
| CA_n5-n66-n77 | n5, n66, n77 |
| CA_n5-n66-n78 | n5, n66, n78 |
| CA_n7-n25-n66 | n7, n25, n66 |
| CA_n7-n28-n78 | n7, n28, n78 |
| CA_n7-n66-n78 | n7, n66, n78 |
| CA_n8-n39-n41 | n8, n39, n41 |
| CA_n8-n40-n41 | n8, n40, n41 |
| CA_n8-n41-n79 | n8, n41, n79 |
| CA_n8-n78-n79 | n8, n78, n79 |
| CA_n13-n25-n66 | n13, n25, n66 |
| CA_n20-n28-n78 | n20, n28, n78 |
| CA_n25-n29-n66 | n25, n29, n66 |
| CA_n25-n38-n78 | n25, n38, n78 |
| CA_n25-n41-n66 | n25, n41, n66 |
| CA_n25-n41-n71 | n25, n41, n71 |
| CA_n25-n41-n77 | n25, n41, n77 |
| CA_n25-n48-n66 | n25, n48, n66 |
| CA_n25-n66-n71 | n25, n66, n71 |
| CA_n25-n66-n77 | n25, n66, n77 |
| CA_n25-n66-n78 | n25, n66, n78 |
| CA_n25-n71-n77 | n25, n71, n77 |
| CA_n28-n77-n79 | n28, n77, n79 |
| CA_n28-n78-n79 | n28, n78, n79 |
| CA_n28-n40-n78 | n28, n40, n78 |
| CA_n28-n41-n77 | n28, n41, n77 |
| CA_n28-n41-n78 | n28, n41, n78 |
| CA_n29-n66-n70 | n29, n66, n70 |
| CA_n38-n66-n78 | n38, n66, n78 |
| CA_n39-n40-n41 | n39, n40, n41 |
| CA_n39-n40-n79 | n39, n40, n79 |
| CA_n39-n41-n79 | n39, n41, n79 |
| CA_n40-n41-n791,2 | n40, n41, n79 |
| CA_ n41-n66-n71 | n41, n66, n71 |
| CA_ n41-n66-n77 | n41, n66, n77 |
| CA_n41-n71-n77 | n41, n71, n77 |
| CA_n66-n70-n71 | n66, n70, n71 |
| CA_n66-n71-n77 | n66, n71, n77 |
| CA_n66-n71-n78 | n66, n71, n78 |
| NOTE 1: The frequency range below 2506 MHz for Band n41 is not used in this band combination. NOTE 2: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. |  |

#### 5.2A.2.3	Inter-band CA (four bands)

Table 5.2A.2.3-1: Inter-band CA operating bands involving FR1 (four bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3-n7-n28 | n1, n3, n7, n28 |
| CA_n1-n3-n7-n78 | n1, n3, n7, n78 |
| CA_n1-n3-n8-n78 | n1, n3, n8, n78 |
| CA_n1-n3-n28-n78 | n1, n3, n28, n78 |
| CA_n3-n5-n7-n78 | n3, n5, n7, n78 |
| CA_n3-n7-n28-n78 | n3, n7, n28, n78 |
| CA_n3-n28-n41-n77 | n3, n28, n41, n77 |
| CA_n3-n28-n41-n78 | n3, n28, n41, n78 |
| CA_n7-n25-n66-n78 | n7, n25, n66, n78 |
| CA_n25-n41-n66-n71 | n25, n41, n66, n71 |
| CA_n41-n66-n71-n77 | n41, n66, n71, n77 |

## 5.2B	Operating bands for DC

The operating bands are specified in clause 5.5B for operation with NR dual connectivity configured, where all operating bands are within FR1.

## 5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, Table 5.2C-2, Table 5.2C-3 and Table 5.2C-4, where all operating bands are within FR1.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41-n80 | n41, n80 |
| SUL_n41-n81 | n41, n81 |
| SUL_n41-n832 | n41, n83 |
| SUL_n41-n95 | n41, n95 |
| SUL_n41-n98 | n41, n98 |
| SUL_n77-n802 | n77, n80 |
| SUL_n77-n842 | n77, n84 |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| SUL_n79-n832 | n79, n83 |
| SUL_n79-n84 | n79, n84 |
| SUL_n79-n95 | n79, n95 |
| SUL_n79-n97 | n79, n97 |
| SUL_n79-n98 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.2C-2: Operating SUL band combination with intra-band non-contiguous CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n78(*)-n862 | n78, n86 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in table 5.5C-2. |  |

Table 5.2C-3: Operating SUL band combination with intra-band contiguous CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41-n80 | n41, n80 |
| SUL_n41-n83 | n41, n83 |
| SUL_n41-n95 | n41, n95 |
| SUL_n78-n80 | n78, n80 |
| SUL_n78-n84 | n78, n84 |
| SUL_n79-n80 | n79, n80 |
| SUL_n79-n83 | n79, n83 |
| SUL_n79-n95 | n79, n95 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.2C-4: Operating SUL band combination with inter-band CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_SUL_n78-n80 | n1, n78, n80 |
| CA_n1_SUL_n78-n84 | n1, n78, n84 |
| CA_n3_SUL_n78-n80 | n3, n78, n80 |
| CA_n28_SUL_n41-n83 | n28, n41, n83 |
| CA_n28_SUL_n79-n83 | n28, n79, n83 |
| CA_n41_SUL_n79-n80 | n41, n79, n80 |
| CA_n41_SUL_n79-n83 | n41, n79, n83 |
| CA_n79_SUL_n41-n80 | n41, n79, n80 |
| CA_n79_SUL_n41-n83 | n41, n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

## 5.2D	Operating bands for UL MIMO

NR is designed to support UL MIMO where all of the operating bands are in FR1 defined in Table 5.2D-1.

Table 5.2D-1: NR operating bands for UL MIMO in FR1

NR operating band

n1

n2

n3

n7

n25

n301

n34

n38

n39

n40

n41

n46

n48

n66

n70

n712

n77

n78

n79

n80

n84

n95

n96

n97

n98

NOTE 1: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 2: UL MIMO is targeted for FWA form factor.

## 5.2E	Operating band for V2X

### 5.2E.1	V2X operating bands

NR V2X is designed to operate in the operating bands in FR1 defined in Table 5.2E.1-1.

Table 5.2E.1-1 V2X operating bands in FR1

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n381 | 2570 MHz | - | 2620 MHz | 2570 MHz | - | 2620 MHz | HD | PC5 |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | HD | PC5 |
| Note 1: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. |  |  |  |  |  |  |  |  |

### 5.2E.2	V2X operating bands for con-current operation

NR V2X operation is designed to operate concurrent with NR uplink/downlink on the operating bands combinations listed in Table 5.2E.2-1.

Table 5.2E.2-1 Inter-band con-current V2X operating bands

| V2X con-current operating Band | NR or V2X Operating Band | Interface |
| --- | --- | --- |
| V2X_n39-n47 | n39 | Uu |
|  | n47 | PC5 |
| V2X_n40-n47 | n40 | Uu |
|  | n47 | PC5 |
| V2X_n41-n47 | n41 | Uu |
|  | n47 | PC5 |
| V2X_n71-n47 | n71 | Uu |
|  | n47 | PC5 |

## 5.3	UE channel bandwidth

### 5.3.1	General

The UE channel bandwidth supports a single NR RF carrier in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the maximum transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

### 5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-1,

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 552.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 905 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1610 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1.

Figure 5.3.3-1: Void

The number of RBs configured in any channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.3-2: UE PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol due to BS transmission of SSB, the minimum guardband on each side of the carrier is the guardband applied at the configured channel bandwidth for the numerology that is received immediately adjacent to the guard.

If multiple numerologies are multiplexed in the same symbol and the UE channel bandwidth is >50 MHz, the minimum guardband applied adjacent to 15 kHz SCS shall be the same as the minimum guardband defined for 30 kHz SCS for the same UE channel bandwidth.

Figure 5.3.3-3 Guard band definition when transmitting multiple numerologies

NOTE:	Figure 5.3.3-3 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

[For a UE supporting wideband operation], the nominal intra-cell guard bands and the corresponding sizes of the RB sets separated by the said guard bands are as specified in Table 5.3.3-2 for each UE channel bandwidth and sub-carrier spacing for the downlink and uplink. The nominal intra-cell guard bands in Table 5.3.3-2 are applicable when the respective IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [7] for the uplink and downlink are not provided, as specified in [10] clause 7.

Table 5.3.3-2: Nominal intra-cell guard bands for wideband operation

| SCS (kHz) | 40 MHz | 60 MHz | 80 MHz |
| --- | --- | --- | --- |
| 15 | 105-6-105 (216) | N/A | N/A |
| 30 | 50-6-50 (106) | 50-6-50-6-50 (162) | 50-6-50-5-50-6-50 (217) |
| 60 | 23-5-23 (51) | 23-5-23-5-23 (79) | 23-5-23-5-23-5-23 (107) |
| NOTE 1: The intra-cell guard band is denoted TBW0-GB0-…-GBN_RBset-2-TBWN_RBset-1 for N_RBset > 1 number of RB-sets with TBWr the maximum transmission bandwidth (PRB) of RB-set r and GBr the guard band (PRB) above the upper edge of RB-set r. The RB-set 0 is starting at the first common resource block (CRB) of the carrier as indicated by offsetToCarrier. The total transmission bandwidth configuration (size of resource grid) including guard bands is given in between parentheses. |  |  |  |

For a UE that supports shared spectrum channel access, there are no uplink or downlink intra-cell guard bands for operation with 10 MHz and 20 MHz channel bandwidths; the maximum transmission bandwidth configurations for these channel bandwidths are in accordance with clause 5.3.2.

For each UE channel bandwidth and sub-carrier spacing given by Table 5.3.3-2, the maximum transmission bandwidth configuration of the carrier including intra-cell guard bands, if configured for the uplink and downlink by the respective IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [7], and corresponding RB-set(s) shall be in accordance with clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1 for the uplink and downlink. Minimum requirements specified for wideband operation in Clause 6 and Clause 7 also apply for intra-cell guard bands larger than the nominal sizes in Table 5.3.3-2 as listed in Table 5.3.3-3 for each sub-carrier spacing; each guard band in order of CRB index must be larger than or equal to the corresponding nominal guard band specified in Table 5.3.3-2 for each channel bandwidth.

Table 5.3.3-3: Applicable intra-cell guard bands for wideband operation

| Parameter | Unit | SCS |  |
| --- | --- | --- | --- |
|  |  | 15 kHz | 30 kHz |
| Intra-cell guard band (size) | PRB | 6,7 | 5,6,7 |
| Transmission bandwidth (size) of RB-set | PRB | 104,105 | 49,50,51 |

If the UE is configured with zero width intra-cell guard bands for the uplink and downlink by the IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [7] on a carrier greater than 20 MHz, the maximum transmission bandwidth configuration for the uplink and downlink shall be in accordance with clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1.

### 5.3.4	RB alignment

For each numerology, its common resource blocks are specified in Clause 4.4.4.3 in TS 38.211 [6], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to "Reference point A" in the unit of the numerology. The UE transmission bandwidth configuration is indicated by the higher layer parameter carrierBandwidth [7] and will fulfil the minimum UE guardband requirement specified in Clause 5.3.3.

### 5.3.5	UE channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the TX and RX path.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n26 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n28 | 15 | Yes | Yes | Yes | Yes7 |  | Yes7 |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes7 |  | Yes7 |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | Yes | Yes10 | Yes | Yes10 | Yes | Yes10 | Yes10 |  |  |  |  |  |  |
|  | 30 |  | Yes10 | Yes | Yes10 | Yes | Yes10 | Yes10 |  |  |  |  |  |  |
|  | 60 |  | Yes10 | Yes | Yes10 | Yes | Yes10 | Yes10 |  |  |  |  |  |  |
| n39 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
| n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| n46 | 15 |  | Yes5 |  | Yes |  |  | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes5 |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |
|  | 60 |  | Yes5 |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |
| n48 | 15 | Yes5 | Yes | Yes | Yes |  | Yes | Yes | Yes6 |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes6 | Yes6 | Yes6 | Yes6 | Yes6,4 | Yes6 |
|  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes6 | Yes6 | Yes6 | Yes6 | Yes6,4 | Yes6 |
| n47 | 15 |  | Yes10 |  | Yes10 |  | Yes10 | Yes10 |  |  |  |  |  |  |
|  | 30 |  | Yes10 |  | Yes10 |  | Yes10 | Yes10 |  |  |  |  |  |  |
|  | 60 |  | Yes10 |  | Yes10 |  | Yes10 | Yes10 |  |  |  |  |  |  |
| n50 | 15 | Yes9 | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes3 |  |  |
|  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes3 |  |  |
| n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |  |
| n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n70 | 15 | Yes | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |  |  |
| n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n76 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |
| n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |
| n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |
|  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |
| n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n82 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n83 | 15 | Yes | Yes | Yes | Yes |  | Yes7 |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes7 |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n84 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n86 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
| n89 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n90 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |
| n91 | 15 | Yes | Yes8 |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 | Yes | Yes8 |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n95 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| n96 | 15 |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |
|  | 30 |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |
|  | 60 |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |
| n97 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
| n98 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n99 | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 10: These UE channel bandwidths are applicable to sidelink operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.3.6	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. |  |  |  |

In TDD, the operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-2.

Table 5.3.6-2: TDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) |
| --- | --- | --- |
| n50 | 60 | 80 |
| NOTE 1: Both centre frequency and BWP-ID shall match between DL and UL carriers as defined in TS 38.331 [7] cl. 6.3.2 and TS 38.213 [8] clause 12. NOTE 2: In a case a UE is configured with a full width of BWP within both UL/ DL channels, the centre frequency of UL/ DL channels shall be same. NOTE 3: A position of Point A is common between UL and DL carriers as defined in TS 38.331 [7] cl. 6.3.2. |  |  |

## 5.3A	UE channel bandwidth for CA

### 5.3A.1	General

Figure 5.3A.1-1: Void

Figure 5.3A.1-2: Void

### 5.3A.2	Maximum transmission bandwidth configuration for CA

For carrier aggregation, the maximum transmission bandwidth configuration is defined per component carrier and the requirement is specified in clause 5.3.2.

### 5.3A.3	Minimum guardband and transmission bandwidth configuration for CA

For intra-band contiguous carrier aggregation, Aggregated Channel Bandwidth and Guard Bands are defined as follows, see Figure 5.3A.3-1.

FC, low

Lower Edge

Upper  Edge

Lowest Carrier Transmission Bandwidth Configuration [RB]

FC, high

Foffset, low

Highest Carrier Transmission Bandwidth Configuration [RB]

Resource block

Aggregated Channel Bandwidth, BWchannel_CA (MHz)

Fedge, low

Fedge, high

Foffset, high

Figure 5.3A.3-1: Definition of Aggregated Channel Bandwidth for intra-band carrier aggregation

The aggregated channel bandwidth, BWChannel_CA, is defined as

BWChannel_CA = Fedge,high - Fedge,low (MHz).

The lower bandwidth edge Fedge, low and the upper bandwidth edge Fedge,high of the aggregated channel bandwidth are used as frequency reference points for transmitter and receiver requirements and are defined by

Fedge,low = FC,low - Foffset,low

Fedge,high = FC,high + Foffset,high

The lower and upper frequency offsets depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carrier and are defined as

Foffset,low = (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz)

BWGB = max(BWGB,Channel(k))

NRB,low and NRB,high are the transmission bandwidth configurations according to Table 5.3.2-1 for the lowest and highest assigned component carrier, SCSlow and SCShigh are the sub-carrier spacing for the lowest and highest assigned component carrier respectively. SCSlow, SCShigh, NRB,low, NRB,high, and BWGB,Channel(k) use the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and BWGB,Channel(k) is the minimum guard band for carrier k according to Table 5.3.3-1 for the said μ value.

In case there is no common μ value for both of the channel bandwidths, μ=1 is used for SCSlow, SCShigh, NRB,low, NRB,high, and BWGB,Channel(k).

For intra-band non-contiguous carrier aggregation Sub-block Bandwidth and Sub-block edges are defined as follows, see Figure 5.3A.3-2.

...

Sub block n

Transmission Bandwidth Configuration of the highest carrier in a sub-block [RB]

Transmission Bandwidth Configuration of the lowest carrier in a sub-block  [RB]

Fedge,block n, low

FC,block n,high

Fedge,block n,high

Foffset,high

Foffset,low

FC,block n,low

Sub-block Bandwidth, BWChannel,block n (MHz)

Lower Sub-block Edge

Upper Sub-block Edge

Resource block

Sub block n+1

Foffset, low

Fedge,block n+1, low

FC,block n+1,low

FC,block n+1,high

Fedge,block n+1,high

Foffset,high

Sub-block Bandwidth, BWChannel,block n+1  (MHz)

Lower Sub-block Edge

Upper Sub-block Edge

Transmission Bandwidth Configuration of the highest carrier in a sub-block [RB]

Transmission Bandwidth Configuration of the lowest carrier in a sub-block  [RB]

Resource block

Figure 5.3A.3-2: Definition of sub-block bandwidth for intra-band non-contiguous spectrum

The lower sub-block edge of the Sub-block Bandwidth (BWChannel,block) is defined as

Fedge,block, low = FC,block,low - Foffset, low.

The upper sub-block edge of the Sub-block Bandwidth is defined as

Fedge,block,high = FC,block,high + Foffset,high.

The Sub-block Bandwidth, BWChannel,block, is defined as follows:

BWChannel,block = Fedge,block,high - Fedge,block,low (MHz)

The lower and upper frequency offsets Foffset,block,low and Foffset,block,high depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carriers within a sub-block and are defined as

Foffset,block,low =  (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

Foffset,block,high =  (NRB,high*12 - 1)*SCShigh/2 + BWGB(MHz)

BWGB = max(BWGB,Channel(k))

where NRB,low and NRB,high are the transmission bandwidth configurations according to Table 5.3.2-1 for the lowest and highest assigned component carrier within a sub-block, respectively. SCSlow and SCShigh are the sub-carrier spacing for the lowest and highest assigned component carrier within a sub-block, respectively.  SCSlow, SCShigh, NRB,low, NRB,high, and BWGB,Channel(k) use the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and BWGB,Channel(k) is the minimum guard band for carrier k according to Table 5.3.3-1 for the said μ value. In case there is no common μ value for both of the channel bandwidths, μ=1 is used for SCSlow, SCShigh, NRB,low, NRB,high, and BWGB,Channel(k).

The sub-block gap size between two consecutive sub-blocks Wgap is defined as

Wgap = Fedge,block n+1,low - Fedge,block n,high (MHz)

### 5.3A.4	Void

### 5.3A.5	UE channel bandwidth per operating band for CA

The requirements for carrier aggregation in this specification are defined for carrier aggregation configurations.

For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class with associated bandwidth combination sets specified in clause 5.5A.1. For each carrier aggregation configuration, requirements are specified for all aggregated channel bandwidths contained in a bandwidth combination set, a UE can indicate support of several bandwidth combination sets per carrier aggregation configuration. For intra-band non-contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting two or more sub-blocks, each supporting a carrier aggregation bandwidth class.

For intra-band non-contiguous uplink carrier aggregation, frequency separation class (Fs) specified in Table 5.3A.5-2 indicates the maximum frequency span between lower edge of lowest component carrier and upper edge of highest component carrier that UE can support per band combination in uplink in non-contiguous intra-band operation when the signalling is absent for dualPA-Architecture IE.

For inter-band carrier aggregation, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class.

Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 3 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 3 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 3 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 3 |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. |  |  |  |

Table 5.3A.5-2: NR intra-band non-contiguous UL CA frequency separation classes

| NR NC UL CA frequency separation class | Maximum allowed frequency separation |
| --- | --- |
| I | 100 MHz |
| II | 200 MHz |
| III | [600MHz] |

## 5.3E	Channel bandwidth for V2X

### 5.3E.1	General

NR V2X operation channel bandwidths for each operating band is specified in Table 5.3.5-1 in clause 5.3.5. The same (symmetrical) channel bandwidth is specified for both the transmission and reception path.

### 5.3E.2	Channel bandwidth for V2X concurrent operation

For NR V2X inter-band con-current operation in FR1, the NR V2X channel bandwidths for each operating band is specified in Table 5.3E.2-1.

Table 5.3E.2-1: Inter-band con-current V2X configurations

| V2X con-current operating band Configuration | NR Bands | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz |  | Maximum bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n39A-n47A | n39 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 80 | 0 |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n47 | 15 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |
| V2X_n40A-n47A | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 | 0 |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n47 | 15 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |
| 140V2X_n41A-n47A | n40 | 15 | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  | Yes |  |  |  | 140 | 0 |
|  |  | 30 | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
|  |  | 60 | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
|  | n47 | 15 | Yes |  | Yes |  | Yes | Yes |  |  |  | Yes |  |  |  |  |  |
|  |  | 30 | Yes |  | Yes |  | Yes | Yes |  |  |  | Yes |  |  |  |  |  |
|  |  | 60 | Yes |  | Yes |  | Yes | Yes |  |  |  | Yes |  |  |  |  |  |
| V2X_n71A-n47A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n47 | 15 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |

## 5.4	Channel arrangement

### 5.4.1	Channel spacing

#### 5.4.1.1	Channel spacing for adjacent NR carriers

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent NR carriers is defined as following:

-	For NR operating bands with 100 kHz channel raster,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

-	For NR operating bands with 15 kHz channel raster,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-5 kHz, 0 kHz, 5 kHz} for ∆FRaster equals 15 kHz

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals 30 kHz

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

### 5.4.2	Channel raster

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

For SUL bands except n95, n97, n98 and for the uplink of all FDD bands defined in Table 5.2-1, and for TDD bands n48, n90, n38 and n40

FREF, shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz.

where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [7]. For Band n48, FREF, shift is only applicable to uplink transmissions using a 15 kHz SCS. For Band n38, FREF, shift is only applicable to uplink transmissions using a 15 kHz SCS. For Band n40, FREF, shift is only applicable to uplink transmissions using a 15 kHz SCS.

The mapping between the channel raster and corresponding resource element is given in Clause 5.4.2.2. The applicable entries for each operating band are defined in Clause 5.4.2.3

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

, nPRB, NRB are as defined in TS 38.211[6].

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NR operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20>.

For NR operating bands with 15 kHz channel raster below 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {3,6}. Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as < I >.

For NR operating bands with 15 kHz channel raster above 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

In frequency bands with two ΔFRaster, the higher ΔFRaster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ∆FRaster .

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n13 | 100 | 155400 – <20> – 157400 | 149200 – <20> – 151200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n24 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n26 | 100 | 162800 – <20> – 169800 | 171800 – <20> – 178800 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n29 | 100 | N/A | 143400 – <20> – 145600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n462 | 15 | 743333 – <1> – 795000 | 743333 – <1> – 795000 |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n53 | 100 | 496700 – <20> – 499000 | 496700 – <20> – 499000 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| n89 | 100 | 164800 – <20> – 169800 | N/A |
| n90 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |
| n91 | 100 | 166400 – <20> – 172400 | 285400 – <20> – 286400 |
| n92 | 100 | 166400 – <20> – 172400 | 286400 – <20> – 303400 |
| n93 | 100 | 176000 – <20> – 183000 | 285400 – <20> – 286400 |
| n94 | 100 | 176000 – <20> – 183000 | 286400 – <20> – 303400 |
| n95 | 100 | 402000 – <20> – 405000 | N/A |
| n963 | 15 | 795000 – <1> – 875000 | 795000 – <1> – 875000 |
| n97 | 100 | 460000 – <20> – 480000 | N/A |
| n98 | 100 | 376000 – <20> – 384000 | N/A |
| n99 | 100 | 325300 – <20> – 332100 | N/A |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. |  |  |  |

Table 5.4.2.3-2: Allowed NREF (NR-ARFCN) for operation in Band n46

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 10 MHz | 782000, 788668 |
| 20 MHz | 744000, 745332, 746668, 748000, 749332, 750668, 752000, 753332, 754668, 756000, 765332, 766668, 768000, 769332, 770668, 772000, 773332, 774668, 776000, 777332, 778668, 780000, 781332, 783000, 784332, 785668, 787000, 788332, 789668, 791000, 792332, 793668 |
| 40 MHz | 744668, 746000, 748668, 751332, 754000, 755332, 766000, 767332, 770000, 772668, 775332, 778000, 780668, 783668, 786332, 787668, 790332, 793000 |
| 60 MHz | 745332, 746668, 748000, 752000, 753332, 754668, 766668, 768000, 769332, 773332, 774668, 778668, 780000, 784332, 785668, 787000, 791000, 792332 |
| 80 MHz | 746000, 747332, 752668, 754000, 767332, 768668, 774000, 779332, 785000, 786332, 791668 |
| NOTE: 10 MHz channel bandwidth shall only apply in certain regions where the absence of non 3GPP technologies can be guaranteed on a long-term basis in this version of specification. |  |

Table 5.4.2.3-3: Allowed NREF (NR-ARFCN) for operation in Band n96

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668, 829000, 830332, 831668, 833000, 834332, 835668, 837000, 838332, 839668, 841000, 842332, 843668, 845000, 846332, 847668, 849000, 850332, 851668, 853000, 854332, 855668, 857000, 858332, 859668, 861000, 862332, 863668, 865000, 866332, 867668, 869000, 870332, 871668, 873000, 874332 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000, 829668, 832332, 835000, 837668, 840332, 843000, 845668, 848332, 851000, 853668, 856332, 859000, 861668, 864332, 867000, 869668, 872332 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332, 830332, 831668, 835668, 837000, 841000, 842332, 846332, 847668, 851668, 853000, 857000, 858332, 862332, 863668, 867668, 869000, 873000 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668, 831000, 836332, 841668, 847000, 852332, 857668, 863000, 868332 |

### 5.4.3	Synchronization raster

#### 5.4.3.1	Synchronization raster and numbering

The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present.

A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The parameters defining the SSREF and GSCN for all the frequency ranges are in Table 5.4.3.1-1.

The resource element corresponding to the SS block reference frequency SSREF is given in clause 5.4.3.2. The synchronization raster and the subcarrier spacing of the synchronization block is defined separately for each band.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} (Note 1) | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

#### 5.4.3.2	Synchronization raster to synchronization block resource element mapping

The mapping between the synchronization raster and the corresponding resource element of the SS block is given in Table 5.4.3.2-1.

Table 5.4.3.2-1: Synchronization raster to SS block resource element mapping

| Resource element index | 120 |
| --- | --- |

is the subcarrier number of SS/PBCH block defined in TS 38.211 clause 7.4.3.1 [6].

#### 5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| n3 | 15 kHz | Case A | 4517 – <1> – 4693 |
| n5 | 15 kHz | Case A | 2177 – <1> – 2230 |
|  | 30 kHz | Case B | 2183 – <1> – 2224 |
| n7 | 15 kHz | Case A | 6554 – <1> – 6718 |
| n8 | 15 kHz | Case A | 2318 – <1> – 2395 |
| n12 | 15 kHz | Case A | 1828 – <1> – 1858 |
| n13 | 15 kHz | Case A | 1871 – <1> – 1885 |
| n14 | 15 kHz | Case A | 1901 – <1> – 1915 |
| n18 | 15 kHz | Case A | 2156 – <1> – 2182 |
| n20 | 15 kHz | Case A | 1982 – <1> – 2047 |
| n24 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n25 | 15 kHz | Case A | 4829 – <1> – 4981 |
| n26 | 15 kHz | Case A | 2153 – <1> – 2230 |
| n28 | 15 kHz | Case A | 1901 – <1> – 2002 |
| n29 | 15 kHz | Case A | 1798 – <1> – 1813 |
| n30 | 15 kHz | Case A | 5879 – <1> – 5893 |
| n34 | 15 kHz | Case A | NOTE 5 |
|  | 30 kHz | Case C | 5036 – <1> – 5050 |
| n38 | 15 kHz | Case A | NOTE 2 |
|  | 30 kHz | Case C | 6437 – <1> – 6538 |
| n39 | 15 kHz | Case A | NOTE 6 |
|  | 30 kHz | Case C | 4712 – <1> – 4789 |
| n40 | 30 kHz | Case C | 5762 – <1> – 5989 |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n463 | 30 kHz | Case C | 8993 – <1> – 9530 |
| n48 | 30 kHz | Case C | 7884 – <1> – 7982 |
| n50 | 30 kHz | Case C | 3590 – <1> – 3781 |
| n51 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n53 | 15 kHz | Case A | 6215 – <1> – 6232 |
| n65 | 15 kHz | Case A | 5279 – <1> – 5494 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |
| n70 | 15 kHz | Case A | 4993 – <1> – 5044 |
| n71 | 15 kHz | Case A | 1547 – <1> – 1624 |
| n74 | 15 kHz | Case A | 3692 – <1> – 3790 |
| n75 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n76 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n77 | 30 kHz | Case C | 7711 – <1> – 8329 |
| n78 | 30 kHz | Case C | 7711 – <1> – 8051 |
| n79 | 30 kHz | Case C | 8480 – <16> – 8880 |
| n90 | 15 kHz | Case A | 6246 – <1> – 6717 |
|  | 30 kHz | Case C | 6252 – <1> – 6714 |
| n91 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n92 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n93 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n94 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n964 | 30 kHz | Case C | 9531 – <1> – 10363 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8]. NOTE 2: The applicable SS raster entries are GSCN = {6432, 6443, 6457, 6468, 6479, 6493, 6507, 6518, 6532, 6543}. NOTE 3: The following GSCN are allowed for operation in band n46: GSCN = {8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9288, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458, 9472, 9485, 9499, 9513}. NOTE 4: The following GSCN are allowed for operation in band n96: GSCN = {9548, 9562, 9576, 9590, 9603, 9617,9631, 9645, 9659, 9673, 9687, 9701, 9714, 9728, 9742, 9756, 9770, 9784, 9798, 9812, 9826, 9840, 9853, 9867, 9881, 9895, 9909, 9923, 9937, 9951, 9964, 9978, 9992, 10006, 10020, 10034, 10048, 10062, 10076, 10090, 10103, 10117, 10131, 10145, 10159, 10173, 10187, 10201, 10214, 10228, 10242, 10256, 10270, 10284, 10298, 10312, 10325, 10339, 10353{. NOTE 5: The applicable SS raster entries are GSCN = {5032, 5043, 5054} NOTE 6: The applicable SS raster entries are GSCN = {4707, 4715, 4718, 4729, 4732, 4743, 4747, 4754, 4761, 4768, 4772, 4782, 4786, 4793} |  |  |  |

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1.

Table 5.4.4-1: UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n1 | 190 MHz |
| n2 | 80 MHz |
| n3 | 95 MHz |
| n5 | 45 MHz |
| n7 | 120 MHz |
| n8 | 45 MHz |
| n12 | 30 MHz |
| n13 | -31 MHz |
| n14 | -30 MHz |
| n18 | 45 MHz |
| n20 | -41 MHz |
| n24 | -101.5, -120.5 MHz |
| n25 | 80 MHz |
| n26 | 45 MHz |
| n28 | 55 MHz |
| n30 | 45 MHz |
| n65 | 190 MHz |
| n66 | 400 MHz |
| n70 | 300 MHz |
| n71 | -46 MHz |
| n74 | 48 MHz |
| n91 | 570 MHz – 595 MHz (NOTE 2) |
| n92 | 575 MHz – 680 MHz (μ = 0) 580 MHz – 675 MHz (μ = 1) (NOTE 2) |
| n93 | 517 MHz – 547 MHz (NOTE 2) |
| n94 | 522 MHz – 632 MHz (μ = 0) 527 MHz – 627 MHz (μ = 1) (NOTE 2) |
| NOTE 1: Void NOTE 2: The range of TX-RX frequency separation given paired UL and DL channel bandwidths BWUL and BWDL is given by the respective lower and upper limit FDL_low – FUL_high + 0.5(BWDL + BWUL) and FDL_high – FUL_low – 0.5(BWDL + BWUL). The UL and DL channel bandwidth combinations specified in Table 5.3.5-1 and 5.3.6-1 depend on the subcarrier spacing configuration μ [6]. |  |

## 5.4A	Channel arrangement for CA

### 5.4A.1	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise:

For NR operating bands with a 100 kHz channel raster:

while for NR operating bands without a 100 kHz channel raster:

with

n = µ0

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR component carriers according to Table 5.3.2-1 with values in MHz, μ0  is the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for the said μ value with μ as defined in TS 38.211. In case there is no common μ value for both of the channel bandwidths, μ0=1 is selected and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for μ=1 with μ as defined in TS 38.211.

The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of least common multiple of channel raster and sub-carrier spacing less than the nominal channel spacing to optimize performance in a particular deployment scenario.

For intra-band non-contiguous carrier aggregation, the channel spacing between two NR component carriers in different sub-blocks shall be larger than the nominal channel spacing defined in this clause.

### 5.4A.2	Channel raster for CA

For inter-band and intra-band contiguous carrier aggregation, the channel raster requirements in clause 5.4.2 apply for each operating band.

### 5.4A.3	Synchronization raster for CA

For inter-band and intra-band  contiguous carrier aggregation, the synchronization raster requirements in clause 5.4.3 apply for each operating band.

### 5.4A.4	Tx-Rx frequency separation for CA

For inter-band carrier aggregation, the Tx-Rx frequency separation requirements in clause 5.4.4 apply for each operating band.

For intra-band contiguous carrier aggregation, the same TX-RX frequency separation as specified in Table 5.4.4-1 is applied to PCC and SCC, respectively.

## 5.4B	Reservedz

## 5.4C	Reserved

## 5.4D	Reserved

## 5.4E	Channel arrangement for V2X

### 5.4E.1	Channel spacing

For NR V2X, the channel spacing requirements in clause 5.4.1 apply for each operating band.

### 5.4E.2	Channel raster

#### 5.4E.2.1	NR-ARFCN and channel raster

For NR V2X, the NR-ARFCN and channel raster requirements in clause 5.4.2.1 apply for each operating band.

For NR V2X UE, the reference frequency can be shifted by configuration.

FREF_V2X = FREF + Δshift + N * 5 kHz

where

Δshift = 0 kHz or 7.5 kHz indicated in IE (frequencyShift7p5khz), and

N can be set as one of following values {-1, 0, 1}, which are signalled by the network in higher layer parameters or configured by pre-configuration parameters.

#### 5.4E.2.2	Channel raster to resource element mapping

For NR V2X, the channel raster to resource element mapping requirements in clause 5.4.2.2 apply for each operating band.

#### 5.4E.2.3	Channel raster entries for each operating band

For NR V2X, the channel raster entries, the channel raster entries requirements in clause 5.4.2.3 apply for each operating band.

The RF channel positions on the channel raster in each NR V2X operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4E.2.2.

For NR V2X operating band n47, ΔFRaster = I × ΔFGlobal, where I ϵ {1}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <I>.

### 5.4E.3	Synchronization raster for V2X

There is no synchronization raster definition for NR V2X for both licensed bands and unlicensed bands.

## 5.5	Void

## 5.5A	Configurations for CA

### 5.5A.0	General

The configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for CA operating bands with Band n90 are not listed in the tables below but are covered by this specification.

Non-contiguous resource allocation and almost contiguous allocation are not applicable for each NR carrier of intra-band contiguous and non-contiguous CA configurations.

### 5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n1B | - | 10 | 10,15 |  |  |  | 40 | 0 |
|  |  | 15 | 15,20 |  |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |  |
| CA_n7B | CA_n7B | 10 | 10, 15, 20, 30, 40 |  |  |  | 50 | 0 |
|  |  | 15 | 15, 20, 30 |  |  |  |  |  |
|  |  | 20 | 20, 30 |  |  |  |  |  |
| CA_n40B | - | 20 | 80 |  |  |  | 100 | 0 |
|  |  | 50 | 50 |  |  |  |  |  |
| CA_n41B | CA_n41B | 10, 20, 30, 40, 50 | 10, 20, 30, 40, 50 |  |  |  | 100 | 0 |
| CA_n41C | CA_n41C | 40 | 80, 100 |  |  |  | 180 | 0 |
|  |  | 50, 60, 80 | 60, 80, 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 190 | 1 |
|  |  | 15, 20 | 90, 100 |  |  |  |  |  |
|  |  | 40 | 80, 90, 100 |  |  |  |  |  |
|  |  | 50, 60, 80, 90 | 60, 80, 90, 100 |  |  |  |  |  |
| CA_n46B | - | 20, 40, 60 | 20, 40 |  |  |  | 100 | 0 |
| CA_n46C | - | 60, 80 | 60, 80 |  |  |  | 160 | 0 |
| CA_n46D | - | 60, 80 | 80 | 80 |  |  | 240 | 0 |
| CA_n46E | - | 80 | 80 | 80 | 80 |  | 320 | 0 |
| CA_n46M | - | 20, 40, 60 | 20, 40 | 20, 40 |  |  | 140 | 0 |
| CA_n46N | - | 20, 40, 80 | 20, 40 | 20, 40 | 20, 40 |  | 200 | 0 |
| CA_n46O | - | 20, 60 | 20, 40 | 20, 40 | 20, 40 | 20, 40 | 220 | 0 |
| CA_n48B | CA_n48B | 5 | 15, 20 |  |  |  | 40 | 0 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  |  |  |  |
|  |  | 15, 20 | 15, 20 |  |  |  |  |  |
|  | - | 10 | 50, 60, 80, 90 |  |  |  | 100 | 1 |
|  |  | 15, 20 | 40, 50, 60, 80 |  |  |  |  |  |
|  |  | 40 | 40, 50, 60 |  |  |  |  |  |
| CA_n48C | - | 10 | 100 |  |  |  | 140 | 0 |
|  |  | 15 | 90,100 |  |  |  |  |  |
|  |  | 20 | 90, 100 |  |  |  |  |  |
|  |  | 40 | 80, 90, 100 |  |  |  |  |  |
| CA_n66B | - | 5 1 | 20, 40 |  |  |  | 50 | 0 |
|  |  | 10 | 15, 20, 40 |  |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |  |
| CA_n71B | - | 5 | 20 |  |  |  | 25 | 0 |
|  |  | 10 | 15 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 35 | 1 |
|  |  | 15 | 15, 20 |  |  |  |  |  |
|  |  | 5, 10, 15 | 15, 20 |  |  |  | 35 | 2 |
| CA_n77C | CA_n77C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 200 | 1 |
|  |  | 15, 20 | 90, 100 |  |  |  |  |  |
|  |  | 25, 30 | 80, 90, 100 |  |  |  |  |  |
|  |  | 40 | 70, 80, 90, 100 |  |  |  |  |  |
|  |  | 50, 60, 70, 80, 90, 100 | 60, 70, 80, 90, 100 |  |  |  |  |  |
| CA_n77D | - | 100 | 100 | 100 |  |  | 300 | 0 |
| CA_n78B | - | 20 | 50 |  |  |  | 70 | 0 |
| CA_n78C | CA_n78C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 200 | 1 |
|  |  | 15, 20 | 90, 100 |  |  |  |  |  |
|  |  | 25, 30 | 80, 90, 100 |  |  |  |  |  |
|  |  | 40 | 70, 80, 90, 100 |  |  |  |  |  |
|  |  | 50, 60, 70, 80, 90, 100 | 60, 70, 80, 90, 100 |  |  |  |  |  |
| CA_n78D | - | 100 | 100 | 100 |  |  | 300 | 0 |
| CA_n79C | CA_n79C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
| CA_n79D | - | 100 | 100 | 100 |  |  | 300 | 0 |
| NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. |  |  |  |  |  |  |  |  |

Table 5.5A.1-2: Void

### 5.5A.2	Configurations for intra-band non-contiguous CA

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
| CA_n3(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
| CA_n5(2A) |  | - |  | 5 |  | 5, 10, 15, 20 |  |  |  |  |  | 25 |  | 0 |  |
|  |  |  |  | 10 |  | 10, 15 |  |  |  |  |  |  |  |  |  |
| CA_n7(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
| CA_n25(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
| CA_n41(2A) | CA_n41(2A) | 40, 50, 60, 80 | 40, 50, 60, 80, 100 |  |  | 180 | 0 |
|  |  | 10, 15, 20, 40, 50, 60, 80, 90 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 190 | 1 |
| CA_n48(2A) |  | 10, 15, 20, 40, 50, 60 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 1402 | 0 |
| CA_n48(3A) | - | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 |  | 1402 | 0 |
| CA_n48(4A) | - | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 1352 | 0 |
| CA_n66(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  |  | 60 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  |  | 80 | 1 |
| CA_n71(2A) |  | - |  | 5,10 |  | 5,10,15, 20 |  |  |  |  |  | 30 |  | 0 |  |
|  |  |  |  | 15 |  | 15 |  |  |  |  |  |  |  |  |  |
| CA_n77(2A) |  | CA_n77(2A) |  | 20, 40, 80, 100 |  | 20, 40, 80, 100 |  |  |  |  |  | 200 |  | 0 |  |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 200 | 1 |
| CA_n77(3A) | - | 20, 40, 80, 100 | 20, 40, 80, 100 | 20, 40, 80, 100 |  | 300 | 0 |
| CA_n78(2A) | CA_n78(2A) | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 |  |  | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 200 | 1 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 200 | 2 |
| NOTE 1: Void. NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150 MHz), and the minimum frequency gaps in between NR non-contiguous component carriers. |  |  |  |  |  |  |  |

Table 5.5A.2-2: NR CA configurations and bandwidth combination sets defined for mixed intra-band contiguous and non-contiguous CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n48(A-B) | CA_n48B | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 501 | 601 |  | 801 | 901 | 1001 | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48(A-C) | - | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 501 | 601 |  | 801 | 901 | 1001 | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.5A.3	Configurations for inter-band CA

Table 5.5A.3-1: Void

Table 5.5A.3-2: Void

Table 5.5A.3-3: Void

#### 5.5A.3.1	Configurations for inter-band CA (two bands)

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A | CA_n1A-n3A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n1B-n3A | CA_n1A-n3A | n1 | See CA_n1B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
| CA_n1A-n3(2A) | CA_n1A-n3A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | See CA_n3(2A) bandwidth combination set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n7A | CA_n1A-n7A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n1A-n7B | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A | CA_n1A-n8A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n28A | CA_n1A-n28A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n40A | CA_n1A-n40A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n1A-n41A | CA_n1A-n41A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n77A | CA_n1A-n77A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n78A | CA_n1A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 2 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78C | CA_n1A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n79A | CA_n1A-n79A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n1A-n79C | CA_n1A-n79A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n5A | CA_n2A-n5A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48A | CA_n2A-n48A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 501 | 601 |  | 801 | 901 | 1001 |  |
| CA_n2A-n48B | CA_n2A-n48A CA_n48B | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48C | CA_n2A-n48A CA_n48C | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48(2A) | CA_n2A-n48A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48(A-C) | CA_n2A-n48A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(A-C) Bandwidth Combination Set 0 in Table 5.5A.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  | CA_n2A-n66A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n66B | CA_n2A-n66B CA_n66B | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n77A | CA_n2A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n77(2A) | CA_n2A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n77C | CA_n2A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n78A | CA_n2A-n78A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n2A-n78(2A) | CA_n2A-n78A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7A | CA_n3A-n7A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n3A-n7B | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A | CA_n3A-n8A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n18A | CA_n3A-n18A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A | CA_n3A-n28A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n38A | CA_n3A-n38A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n38 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n3A-n40A | CA_n3A-n40A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n3A-n41A | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  |  |  |  |  |
| CA_n3A-n41C | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n41(2A) | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A | CA_n3A-n77A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n77(2A) | CA_n77(2A) CA_n3A-n77A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A | CA_n3A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n78C | CA_n3A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78(2A) | CA_n3A-n78A CA_n78(2A) | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n79A | CA_n3A-n79A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n3A-n79C | CA_n3A-n79A | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n7A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n5A-n7B | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A | CA_n5A-n25A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n25(2A) | CA_n5A-n25A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48A | CA_n5A-n48A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n5A-n48(2A) | CA_n5A-n48A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48B | CA_n5A-n48A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48C | CA_n5A-n48A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A | CA_n5A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n66(2A) | CA_n5A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n77A | CA_n5A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n77(2A) | CA_n5A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n77C | CA_n5A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n78A | CA_n5A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n78C | CA_n5A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n79A | CA_n5A-n79A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n5A-n79C | CA_n5A-n79A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25A | CA_n7A-n25A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n7A-n25(2A) | CA_n7A-n25A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25A | CA_n7A-n25A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A) | CA_n7A-n25A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n28A | CA_n7A-n28A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n7B-n28A | - | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n66A | CA_n7A-n66A | n7 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 |  | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 60 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 60 | 40 |  |  |  |  |  |  |  |
| CA_n7A-n66(2A) | CA_n7A-n66A | n7 | 5 | 10 | 15 | 20 | 25 | 60 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66A | CA_n7A-n66A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 60 | 40 |  |  |  |  |  |  |  |
| CA_n7(2A)-n66(2A) | CA_n7A-n66A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n77A | CA_n7A-n77A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n77A | CA_n7A-n77A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n77(2A) | CA_n7A-n77A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n77(2A) | CA_n7A-n77A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n78A | CA_n7A-n78A | n7 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n7A-n78(2A) | CA_n7A-n78A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n78A | CA_n7A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n7(2A)-n78(2A) | CA_n7A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n20A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n28A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| CA_n8A-n39A | CA_n8A-n39A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n8A-n40A | CA_n8A-n40A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n8A-n41A | CA_n8A-n41A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  |  |  |  |  |
| CA_n8A-n75A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n78A | CA_n8A-n78A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n79A | CA_n8A-n79A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n13A-n25A | CA_n13A-n25A | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n13A-n66A | CA_n13A-n66A | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n18A-n41A | CA_n18A-n41A | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n20A-n28A | CA_n20A-n28A | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| CA_n20A-n75A | - | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n20A-n78A | CA_n20A-n78A | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n25A-n29A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n38A | CA_n25A-n38A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n38 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25(2A)-n38A | CA_n25A-n38A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n38 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n41A | CA_n25A-n41A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25(2A)-n41A | CA_n25A-n41A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n25A-n41C | CA_n25A-n41A CA_n41C | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A) | CA_n25A-n41A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n48A | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n25A-n48(2A) | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n48C | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66A | CA_n25A-n66A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n66(2A) | CA_n25A-n66A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n66A | CA_n25A-n66A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 |  | 10 | 15 | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25(2A)-n66(2A) | CA_n25A-n66A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n71A | CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n71(2A) | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n77A | CA_n25A-n77A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n78A | CA_n25A-n78A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n78(2A) | CA_n25A-n78A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n78A | CA_n25A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25(2A)-n78(2A) | CA_n25A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n46A | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n46 |  |  |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  |  |
| CA_n28A-n40A | CA_n28A-n40A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n28A-n41A | CA_n28A-n41A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n28A-n50A | CA_n28A-n50A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n50 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 801 |  |  |  |
| CA_n28A-n75A | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n75 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n28A-n77A | CA_n28A-n77A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n28A-n77(2A) | CA_n77(2A) CA_n28A-n77A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A | CA_n28A-n78A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n78(2A) | CA_n78(2A) CA_n28A-n78A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66A | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n29A-n66B | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66(2A) | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n70A | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |
| CA_n38A-n66A | CA_n38A-n66A | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n38 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n38A-n66(2A) | CA_n38A-n66A | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n38A-n78A | CA_n38A-n78A | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n38A-n78(2A) | CA_n38A-n78A | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n79A | CA_n28A-n79A | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n34A-n79A | CA_n34A-n79A | n34 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n39A-n40A | CA_n39A-n40A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n39A-n41A | CA_n39A-n41A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n39A-n41C | CA_n39A-n41A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n39A-n41(2A) | CA_n39A-n41A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n39A-n79A | CA_n39A-n79A | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n40A-n41A | CA_n40A-n41A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  |  |  |  |  |
| CA_n40A-n41C | CA_n41C CA_n40A-n41A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n40A-n78A | CA_n40A-n78A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n40A-n78(2A) | CA_n40A-n78A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n40A-n79A | CA_n40A-n79A | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n41A-n50A | CA_n41A-n50A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n50 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 801 |  |  |  |
| CA_n41A-n66A | CA_n41A-n66A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n41(2A)-n66A | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 inTable 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n41A-n66(2A) | CA_n41A-n66A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 1 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in inTable 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n66A | - | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  | CA_n41C CA_n41A-n66A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n41A-n71A | CA_n41A-n71A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n71B | CA_n41A-n71A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 1 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n71(2A) | CA_n41A-n71A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 1 |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n71A | CA_n41C CA_n41A-n71A | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n71A | CA_n41A-n71A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n71B | CA_n41A-n71A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n71B | CA_n41A-n71A | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n71 | See CA_n71B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n77A | CA_n41A-n77A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 1 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41(2A)-n77A | CA_n41A-n77A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41C-n77A | CA_n41A-n77A CA_n41C | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n77(2A) | CA_n41A-n77A | n41 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n78A | CA_n41A-n78A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n78(2A) | CA_n41A-n78A | n41 |  | 10 | 15 | 50 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n79A | CA_n41A-n79A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  |  |  |  | 1 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n41C-n79A | CA_n41A-n79A CA_n41C | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n46A-n48A | CA_n46A-n48A | n46 |  |  |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48A | CA_n46A-n48A | n46 | See CA_n46B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48A | CA_n46A-n48A | n46 | See CA_n46C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48A | CA_n46A-n48A | n46 | See CA_n46D Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46E-n48A | CA_n46A-n48A | n46 | See CA_n46E Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48B | CA_n46A-n48A CA_n46A-n48B | n46 |  |  |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48C | CA_n46A-n48A CA_n46A-n48B | n46 |  |  |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | See CA_n46B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48C | CA_n46A-n48A CA_n46A-n48B | n46 | See CA_n46B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | See CA_n46C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48C | CA_n46A-n48A CA_n46A-n48B | n46 | See CA_n46C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | See CA_n46D Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48C | CA_n46A-n48A CA_n46A-n48B | n46 | See CA_n46D Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46E-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | See CA_n46E Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46E-n48C |  | n46 | See CA_n46E Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n46A-n48A CA_n46A-n48B | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n66A | - | n46 |  |  |  | 20 |  |  | 40 |  | 60 |  | 80 |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n48A-n66A | CA_n48A-n66A | n48 | 5 | 10 | 15 | 20 |  |  | 40 | 501 | 601 |  | 801 | 901 | 1001 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n48B-n66A | CA_n48A-n66A CA_n48B | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n48C-n66A | CA_n48A-n66A CA_n48C | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n48(2A)-n66A | CA_n48A-n66A | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n48(A-C)-n66A | CA_n48A-n66A | n48 | See CA_n48(A-C) Bandwidth Combination Set 0 in Table 5.5A.2-2 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| CA_n50A-n78A | CA_n50A-n78A | n50 | 5 | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 801 |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n66A-n70A | - | n66 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |
| CA_n66B-n70A | - | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n70A | - | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71A | CA_n66A-n71A | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71(2A) | - | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  | 0 |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n71A | CA_n66A-n71A | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n66B-n71A | CA_n66A-n71A | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77A | CA_n66A-n77A | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66(2A)-n77A | CA_n66A-n77A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66A-n77(2A) | CA_n66A-n77A | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n66A-n77A CA_n77(2A) | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n77(2A) | CA_n66A-n77A CA_n77(2A) | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77C | CA_n66A-n77A | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n77 | See CA_n77C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n78A | CA_n66A-n78A | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66A-n78(2A) | CA_n66A-n78A | n66 | 5 | 10 | 15 | 20 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n78A | CA_n66A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66(2A)-n78(2A) | CA_n66A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n70A-n71A | CA_n70A-n71A | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n71A-n77A | CA_n71A-n77A | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n71A-n77(2A) | CA_n71A-n77A | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n71A-n78A | CA_n71A-n78A | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n71A-n78(2A) | CA_n71A-n78A | n71 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n75A-n78A | - | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n75A-n78(2A) | - | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n76A-n78A | - | n76 | 5 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n77A-n78A2 |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n77A-n79A | CA_n77A-n79A | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n78A-n79A | CA_n78A-n79A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 1 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n78(2A)-n79A | CA_n78A-n79A | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n78A-n92A | CA_n78A-n92A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n92 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n78(2A)-n92A | CA_n78A-n92A | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n92 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink. NOTE 2: The minimum requirements for intra-band contiguous or non-contiguous CA apply. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 MHz |  | 80 |  | 90 |  | 100 |  |
| CA_n1A-n3A-n7A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n7B | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n8A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n28A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 202 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n41A | CA_n1A-n3A CA_n1A-n41A CA_n3A-n41A | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n1A-n3A-n78A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n1A-n8A-n78A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n1A-n8A-n79A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n1A-n7A-n28A | CA_n1A-n7A CA_n1A-n28A CA_n7A-n28A | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n78(2A) | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n7A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 901 |  | 100 |  |
| CA_n1A-n7A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n28A-n78A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 202 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n1A-n40A-n78A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n1A-n77A-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n1A-n78A-n79A5 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
|  |  | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n1A-n78(2A)-n79A | - | n1 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n2A-n66A-n77A | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n5A-n7A | - | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n5A-n7B | - | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7A-n28A | - | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7B-n28A | - | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7A-n78A | - | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n7B-n78A | - | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n8A-n78A | CA_n3A-n8A CA_3A-n78A CA_n8A-n78A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n18A-n41A | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n18 | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n28A-n41A | CA_n3A-n28A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n28A-n77A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n28A-n77(2A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n77(2A) | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A | CA_n3A-n78A CA_n28A-n78A | n3 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 202 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n28A-n78(2A) | CA_n3A-n78A CA_n28A-n78A | n3 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 202 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n40A-n41A | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  |  |  |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n41A-n79A | - | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
|  |  | n3 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n3A-n41A-n77A | CA_n3A-n41A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n3A-n77A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  | CA_n41A-n77A | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n41A-n77(2A) | CA_n3A-n41A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n3A-n77A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  | CA_n41A-n77A | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n41A-n78A | - | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
|  | CA_n3A-n41A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n3A-n78A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  | CA_n41A-n78A | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n3A-n41A-n78(2A) | CA_n3A-n41A | n3 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n3A-n78A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  | CA_n41A-n78A | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n7A-n78A | - | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n5A-n7B-n78A | - | n5 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n5A-n25A-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A-n77A | CA_n5A-n25A | n5 | 5 |  | 10 |  | 15 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n5A-n77A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n77A | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n5A-n25A-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n5A-n25(2A)-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n5A-n25A-n78(2A) | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A-n77A | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n5A-n66A-n77(2A) | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n7A-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n28A-n78A | - | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n7B-n28A-n78A | - | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n7A-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n7A-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n7A-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n7(2A)-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n7(2A)-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66(2A)-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n39A-n41A | - | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n39 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
|  |  | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n39 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  |  |  |  |  |  |  |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  |  |  |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n8A-n41A-n79A | - | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
|  |  | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n8A-n78A-n79A | - | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n8A-n78 (2A)-n79A | - | n8 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n13A-n25A-n66A | CA_n13A-n25A | n13 | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n13A-n66A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n66A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n20A-n28A-n78A | - | n20 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n29A-n66A | CA_n25-n66A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n29 | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n38A-n78A | CA_n25A-n38A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n25A-n78A | n38 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n38A-n78A | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n38A-n78(2A) | CA_n25A-n38A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n25A-n78A | n38 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n38A-n78A | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n38A-n78A | CA_n25A-n38A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n25A-n78A | n38 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n38A-n78A | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n25(2A)-n38A-n78(2A) | CA_n25A-n38A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n25A-n78A | n38 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n38A-n78A | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n66A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n41A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n25A-n66A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
|  | CA_n41A-n66A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41C-n66A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in 38.101-1 Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n41A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n25A-n66A | n41 | See CA_n41C Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n66A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A)-n66A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n41A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n25A-n66A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n66A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n71A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n41A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n41A-n71A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
|  | CA_n25A-n71A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A)-n71A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41C-n71A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in 38.101-1 Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n41A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n41A-n71A | n41 | See CA_n41C bandwidth combination set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n71A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n41(2A)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
|  |  | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n41C-n77A | CA_n41C CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n48A-n66A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n48(2A)-n66A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n48C-n66A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66A-n71A | - | n25 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n66A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n25A-n71A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n66A-n71A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n25A-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A - | n25 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n28A-n40A-n78A | - | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n28A-n41A-n77A | CA_n28A-n41A | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n28A-n77A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  | CA_n41A-n77A | n77 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n28A-n41A-n77(2A) | CA_n28A-n41A | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n28A-n77A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  | CA_n41A-n77A | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n41A-n78A CA_n28A-n78A | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  |  |  | 90 |  | 100 |  |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n28A-n41A-n78(2A) | CA_n78(2A) | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n79A | - | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n28A-n78A-n79A | - | n28 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n29A-n66A-n70A | - | n29 | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n70 | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66B-n70A | - | n29 | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n70 | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n70 | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  |  |  |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n40A-n79A CA_n39A-n79A | n39 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n38A-n66A-n78A | CA_n38A-n66A | n38 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n38A-n78A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n66A-n78A | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n38A-n66A-n78(2A) | CA_n38A-n66A | n38 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n38A-n78A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n66A-n78A | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n38A-n66(2A)-n78A | CA_n38A-n66A | n38 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n38A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n66A-n78A | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n38A-n66(2A)-n78(2A) | CA_n38A-n66A | n38 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n38A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n66A-n78A | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n39A-n41A-n79A | - | n39 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  |  |  |
|  |  | n39 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  |  |  |
| CA_n40A-n41A-n79A | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | n40 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  |  | 0 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
|  |  | n40 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |
| CA_n41A-n66A-n71A | - | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n71A | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 | 1 |
|  | CA_n66A-n71A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n66A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n66A-n71A | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n71A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n66A-n71A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n66A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n66A-n71A | - | n41 | See CA_n41C Bandwidth Combination Set 0 in 38.101-1 Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n71A | n41 | See CA_n41C Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | CA_n66A-n71A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n66A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66A-n77A | - | n41 |  |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n41(2A)-n66A-n77A | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n41C-n66A-n77A | CA_41C | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n41A-n71A-n77A | - | n41 |  | 10 |  | 15 |  | 20 |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  | 0 |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n41(2A)-n71A-n77A | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n41C-n71A-n77A | CA_41C | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n70 | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n66A-n71A-n78A | CA_n66A-n78A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n66A-n71A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n71A-n78A | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n66A-n71A-n78(2A) | CA_n66A-n78A | n66 | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n66A-n71A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n71A-n78A | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n71A-n78A | CA_n66A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n66A-n71A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n71A-n78A | n78 |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
| CA_n66(2A)-n71A-n78(2A) | CA_n66A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n66A-n71A | n71 | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n71A-n78A | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 5: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.5A.3.3	Configurations for inter-band CA (four bands)

Table 5.5A.3.3-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A-n7A-n28A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n7B-n28A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n7A-n78A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A-n7B-n78A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A-n8A-n78A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 901 | 100 |  |
| CA_n1A-n3A-n28A-n78A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 202 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 901 | 100 |  |
| CA_n3A-n5A-n7A-n78A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n5A-n7B-n78A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n7A-n28A-n78A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n7B-n28A-n78A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n28A-n41A-n77A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n28A-n41A-n78A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n78A CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n25A-n66A-n78A | - | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41A-n66A-n71A | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n66A CA_n66A-n71A CA_n71A-n77A CA_n41A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A)-n66A-n71A | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n66A CA_n66A-n71A CA_n71A-n77A CA_n41A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41C-n66A-n71A | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | CA_n41A-n66A CA_n66A-n71A CA_n71A-n77A CA_n41A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66A-n71A-n77A | CA_n41A-n66A CA_n66A-n71A CA_n71A-n77A CA_n41A-n71A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41C-n66A-n71A-n77A | CA_n41A-n66A CA_n66A-n71A CA_n71A-n77A CA_n41A-n71A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41(2A)-n66A-n71A-n77A | CA_n41A-n66A CA_n66A-n71A CA_n71A-n77A CA_n41A-n71A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.5B	Configurations for DC

For an NR DC configuration specified in 5.5B.1-1, the bandwidth combination sets for the corresponding NR CA configuration in 5.5A.3,i.e.,dual uplink inter-band carrier aggregation with uplink assigned to two NR bands, are applicable to Dual Connectivity.

Table 5.5B.1-1: Inter-band NR DC configurations  (two bands)

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n2A-n5A | DC_n2A-n5A |
| DC_n2A-n48A | DC_n2A-n48A |
| DC_n2A-n48B | DC_n2A-n48A |
| DC_n2A-n48C | DC_n2A-n48A |
| DC_n2A-n48(2A) | DC_n2A-n48A |
| DC_n2A-n48(A-C) | DC_n2A-n48A |
| DC_n2A-n66A | DC_n2A-n66A |
| DC_n2A-n66B | DC_n2A-n66A |
| DC_n2A-n77A | DC_n2A-n77A |
| DC_n2A-n77(2A) | DC_n2A-n77A |
| DC_n2A-n77C | DC_n2A-n77A |
| DC_n3A-n28A | DC_n3A-n28A |
| DC_n3A-n41A | DC_n3A-n41A |
| DC_n3A-n77A | DC_n3A-n77A |
| DC_n3A-n77(2A) | DC_n3A-n77A |
| DC_n3A-n78A | DC_n3A-n78A |
| DC_n5A-n48A | DC_n5A-n48A |
| DC_n5A-n48(2A) | DC_n5A-n48A |
| DC_n5A-n48B | DC_n5A-n48A |
| DC_n5A-n48C | DC_n5A-n48A |
| DC_n5A-n66A | DC_n5A-n66A |
| DC_n5A-n66(2A) | DC_n5A-n66A |
| DC_n5A-n77A | DC_n5A-n77A |
| DC_n5A-n77(2A) | DC_n5A-n77A |
| DC_n5A-n77C | DC_n5A-n77A |
| DC_n28A-n41A | DC_n28A-n41A |
| DC_n28A-n77A | DC_n28A-n77A |
| DC_n28A-n78A | DC_n28A-n78A |
| DC_n28A-n77(2A) | DC_n28A-n77A |
| DC_n41A-n77A | DC_n41A-n77A |
| DC_n41A-n78A | DC_n41A-n78A |
| DC_n46A-n48A | DC_n46A-n48A |
| DC_n46B-n48A | DC_n46A-n48A |
| DC_n46C-n48A | DC_n46A-n48A |
| DC_n46D-n48A | DC_n46A-n48A |
| DC_n46E-n48A | DC_n46A-n48A |
| DC_n48A-n66A | DC_n48A-n66A |
| DC_n48B-n66A | DC_n48A-n66A |
| DC_n48(2A)-n66A | DC_n48A-n66A |
| DC_n48C-n66A | DC_n48A-n66A |
| DC_n48(A-C)-n66A) | DC_n48A-n66A |
| DC_n66A-n77A | DC_n66A-n77A |
| DC_n66A-n77(2A) | DC_n66A-n77A |
| DC_n66(2A)-n77(2A) | DC_n66A-n77A |
| DC_n66A-n77C | DC_n66A-n77A |

Table 5.5B.1-2: Inter-band NR DC configurations  (three bands)

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n3A-n28A-n77A | DC_n3A-n28A DC_n3A-n77A DC_n28A-n77A |
| DC_n3A-n28A-n77(2A) | DC_n3A-n28A DC_n3A-n77A DC_n28A-n77A |

## 5.5C	Configurations for SUL

Table 5.5C-1: Supported channel bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 MHz | 80 | 90 | 100 |  |
| SUL_n41A-n80A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n80 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 1 |
|  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| SUL_n41A-n81A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n81 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| SUL_n41A-n83A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| SUL_n41A-n95A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n95 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n41A-n98A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n95 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| SUL_n77A-n80A | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n80 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
| SUL_n77A-n84A | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n84 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n80A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n80 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 1 |
|  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| SUL_n78A-n81A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n81 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n82A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n82 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n83A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n83 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 1 |
|  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| SUL_n78A-n84A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  | n84 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 1 |
|  | n84 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| SUL_n78A-n86A | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  | n86 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n80A | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  | n80 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 1 |
|  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| SUL_n79A-n81A | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  | n81 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n83A | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| SUL_n79A-n84A | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  | n84 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n95A | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  | n95 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n97A | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  | n97 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| SUL_n79A-n98A | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 | 0 |
|  | n98 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5C-2: Supported channel bandwidths per SUL band combination with intra-band non-contiguous CA

| SUL band combination with intra-band non-contiguous CA | SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |  |
| SUL_n78(2A)-n86A | SUL_n78A-n86A | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n86 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5C-3: Supported channel bandwidths per SUL band combination with intra-band contiguous CA

| SUL band combination with CA | SUL configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n41C-n80A | SUL_n41A-n80A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| SUL_n41C-n83A | SUL_n41A-n83A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| SUL_n41C-n95A | SUL_n41A-n95A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n95 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n78C-n80A | SUL_n78A-n80A | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| SUL_n78C-n84A | SUL_n78A-n84A | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n84 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| SUL_n79C-n80A | SUL_n79A-n80A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| SUL_n79C-n83A | SUL_n79A-n83A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| SUL_n79C-n95A | SUL_n79A-n95A | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n95 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5C-4: Supported channel bandwidths per SUL band combination with inter-band CA

| SUL band combination with CA | SUL configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n80A | SUL_n78A-n80A | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n1A_SUL_n78A-n84A | SUL_n78A-n84A | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n84 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n3A_SUL_n78A-n80A | SUL_n78A-n80A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n28A_SUL_n41A-n83A | SUL_n41A-n83A | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| CA_n28A_SUL_n79A-n83A | SUL_n79A-n83A | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| CA_n41A_SUL_n79A-n80A | SUL_n79A-n80A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n41A_SUL_n79A-n83A | SUL_n79A-n83A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |
| CA_n79A_SUL_n41A-n80A | SUL_n41A-n80A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n80 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n79A_SUL_n41A-n83A | SUL_n41A-n83A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  | n83 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  |  |

# 6	Transmitter characteristics

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

The applicability of transmitter requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the minimum requirements for Band n41.

## 6.1A	General

The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | ±2 |
| n2 |  |  |  |  |  |  | 23 | ±23 |
| n3 |  |  |  |  |  |  | 23 | ±23 |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 |  |  |  |  |  |  | 23 | ±23 |
| n8 |  |  |  |  |  |  | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | ±2 |
| n25 |  |  |  |  |  |  | 23 | ±23 |
| n26 |  |  |  |  |  |  | 23 | ±23 |
| n28 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n34 |  |  |  |  |  |  | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  |  |  |  |  | 23 | ±2 |
| n40 |  |  |  |  | 26 | +2/-33 | 23 | ±2 |
| n41 |  |  | 295 | _2/-33 | 26 | +2/-33 | 23 | ±23 |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  |  |  | 23 | ±2 |
| n70 |  |  |  |  |  |  | 23 | ±2 |
| n71 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  |  |  | 23 | ±2 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | ±2/-2.5 |
| n84 |  |  |  |  |  |  | 23 | ±2 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  |  |  | 23 | ±2 |
| n98 |  |  |  |  |  |  | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB NOTE 5: Achieved via dual Tx NOTE 6: Generally, PC1 UE for Band n14 is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle/2 (The exact evaluation period is no less than one radio frame); or
if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the power class 2 or lower;
shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

### 6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 1.5, 2 and 3 and UE power class 1 in Band n14, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-4, Table 6.2.2-2, Table 6.2.2-1 and Table 6.2.2-5, respectively for channel bandwidths  ≤ 100 MHz.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth > 4% for TDD bands or > 3% for FDD bands, the ∆MPR is defined in Table 6.2.2-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high)

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | ≤ 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2.2-3: ∆MPR

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 | 30 MHz | 0.5 |

Table 6.2.2-4 Maximum power reduction (MPR) for power class 1.5 with  dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6.5 | ≤ 3.5 | ≤ 1.5 |
|  | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 5 | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 6 | ≤ 3 |
|  | 16 QAM | ≤ 6.5 | ≤ 6 | ≤ 3.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 5 |
|  | 256 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | ≤ 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ 2 RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap ) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in clause 6.1.2.2 of TS 38.214 [10]. For these almost contiguous signals in power class 2 and 3, the allowed maximum power reduction defined in Table 6.2.2-1 is increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameters of RBStart,Low and RBStart,High to specify valid RB allocation ranges for Outer and Inner RB allocations are defined as following:

RBStart,Low = max(1, floor((NRB_alloc + NRB_gap)/2))

RBStart,High = NRB – RBStart,Low – NRB_alloc –NRB_gap

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2.4 apply.

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2 In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK  is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 |  |  | Clause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.4 |
| NS_06 | 6.5.2.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5,10 |  |  |
| NS_07 | 6.5.3.3.26 | n13 | 5,10 | Table 6.2.3.29-1 | Table 6.2.3.29-2 |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 |  |  |  |
| NS_13 | 6.5.3.3.18 | n26 |  |  |  |
| NS_14 | 6.5.3.3.19 | n26 |  |  |  |
| NS_15 | 6.5.3.3.20 | n26 |  |  |  |
| NS_17 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.3.3.12 | n30 | 5, 10 |  | Clause 6.2.3.14 |
| NS_24 | 6.5.3.3.13 | n65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.3.15-1 | Clause 6.2.3.15 |
| NS_27 | 6.5.2.3.8 6.5.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.16-1 | Table 6.2.3.16-2 |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.6 | n74 (NOTE 3) | 10, 15 | Table 6.2.3.8-1 | Table 6.2.3.8-1 |
| NS_38 | 6.5.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.9-1 | Table 6.2.3.9-1 |
| NS_39 | 6.5.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.10-1 | Table 6.2.3.10-1 |
| NS_40 | 6.5.3.3.9 | n51 | 5 |  | Table 6.2.3.5-1 |
| NS_41 | 6.5.3.3.10 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.11-1 |
| NS_42 | 6.5.3.3.11 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.12-1 |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25, 30, 40 | Table 6.2.3.20-1 | Table 6.2.3.20-1 |
| NS_45 | 6.5.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.25 |
| NS_46 | 6.5.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.17-1 | Table 6.2.3.17-2 |
| NS_47 | 6.5.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.18-1 | Table 6.2.3.18-2 |
| NS_48 | 6.5.3.3.22 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.26-1 | Table 6.2.3.26-1 |
| NS_49 | 6.5.3.3.23 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.27-1 | Table 6.2.3.27-1 |
| NS_50 | 6.5.3.3.16 | n39, n98 | 25, 30, 40 |  | Clause 6.2.3.19 |
| NS_51 | 6.5.3.3.22 | n65 | 50 | Table 6.2.3.28-1 | Table 6.2.3.28-2 |
| NS_52 | 6.5.3.3.27 | n24, n99 | 5, 10 |  | Clause 6.2.3.30 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the lower channel edge is ≥ 1930 MHz,10 MHz BWChannel where the lower channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the lower channel edge is ≥ 1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHNOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n18 | NS_01 | NS_100 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_52 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n89 | NS_01 | NS_100 |  |  |  |  |  |  |
| n91 | NS_01 |  |  |  |  |  |  |  |
| n92 | NS_01 |  |  |  |  |  |  |  |
| n93 | NS_01 |  |  |  |  |  |  |  |
| n94 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n97 | NS_01 |  |  |  |  |  |  |  |
| n98 | NS_01 | NS_50 |  |  |  |  |  |  |
| n99 | NS_01 | NS_52 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

#### 6.2.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double counting MPR.

Allowed maximum power reduction is defined as A-MPR = max(MPR, A-MPR'),

Note that A-MPR' = 0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A2 relative to 23 dBm for power class 3,  PC2_A4 relative to 26 dBm for power class 2, and PC1.5_A6 relative to 29 dBm for power class 1.5,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A1 relative to 23 dBm for power class 3,  PC2_A3 relative to 26 dBm for power class 2, , and PC1.5_A5 relative to 29 dBm for power class 1.5,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.2-1.

Table 6.2.3.2-1: Parameters for region edges and frequency offsets

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | CP-OFDM |  | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) |  | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.2-2: A-MPR' values Access

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16 QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256 QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256 QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |  |

#### 6.2.3.3	A-MPR for NS_10

Table 6.2.3.3-1: A-MPR for NS_10

| Channel bandwidth (MHz) | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB (RBs) | 1 – 20 |
|  | A (dB) | ≤ 36 |
| 20 | RBstart | 0 – 15 |
|  | LCRB (RBs) | 1 – 20 |
|  | A (dB) | ≤ 66 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. NOTE 5: The A-MPR for DFT-s-OFDM is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3-1 and Table 6.2.4-1 in TS 36.101 and A value specified in Table 6.2.3.3-1. NOTE 6: The A-MPR for CP-OFDM is the total backoff and is obtained by adding the A value in Table 6.2.3.3-1 to the corresponding MPR specified in Table 6.2.2-1. |  |  |

#### 6.2.3.4	A-MPR for NS_05 and NS_05U

Table 6.2.3.4-1: A-MPR regions for NS_05 and NS_05U

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | < 1.62 MHz/12/SCS | > 2.52 MHz/12/SCS | A3 |  |  |  |  |  |  |
| 10 | 1925 ≤ FC < 1935 | < 1.62 MHz/12/SCS | > 0 | A1 | > 1.62 MHz/12/SCS ≤ 3.60 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 | ≥ 7.2 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 10 | 1935 ≤ FC < 1945 |  | > 4.5 MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | < 3.24MHz/12/SCS | > 0 | A1 | > 3.24 MHz/12/SCS ≤ 5.40 MHz/12/SCS | > 8.1 MHz/12/SCS | A7 | ≥ 10.08 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 15 | 1932.5 ≤ FC < 1942.5 | < 1.62 MHz/12/SCS | > 0 | A1 |  |  |  | ≥ 12.24 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | > 7.2 MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | < 4.86 MHz/12/SCS | > 0 | A1 | > 4.86 MHz/12/SCS ≤ 7.20 MHz/12/SCS | > 9.0 MHz/12/SCS | A7 | ≥ 13.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 20 | 1950 ≤ FC < 1960 |  | > 9.0 MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.4-2, 6.2.3.4-3 and 6.2.3.4-10. NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-2: A-MPR for NS_05 and NS_05U

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 10 | ≤ 5 | ≤ 4 |  |
|  | QPSK | ≤ 10 | ≤ 5 | ≤ 4.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 6 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 6 |  |
|  | 256 QAM | ≤ 13 | ≤ 5 | ≤ 7 |  |
| CP-OFDM | QPSK | ≤ 10 | ≤ 5 | ≤ 7.5 | ≤ 2 |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 8 |  |
|  | 256 QAM | ≤ 13 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.4-3: A-MPR for NS_05

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1 |  | ≤ 1 | N/A | ≤ 6 |
|  | QPSK |  |  | ≤ 1.5 |  | ≤ 1.5 |  | ≤ 6 |
|  | 16 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 64 QAM |  |  |  | ≤ 4 |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.4-4 - Table 6.2.3.4-9: Void

Table 6.2.3.4-10: A-MPR for modulation and waveform type for NS_05U

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 2 |  | ≤ 2 | N/A | ≤ 6 |
|  | QPSK | ≤ 2 |  | ≤ 2 |  | ≤ 2 |  | ≤ 6 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 6 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |  | ≤ 3 |  | ≤ 6 |
|  | 256 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 16 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 64 QAM | ≤ 4 |  | ≤ 4 | ≤ 4 | ≤ 4 |  | ≤ 6 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

#### 6.2.3.5	A-MPR for NS_40

Table 6.2.3.5-1: A-MPR for NS_40

| Modulation/ Waveform |  | A (dB) |  |
| --- | --- | --- | --- |
|  |  | Channel bandwidth: 5 MHz |  |
|  |  | Outer | Inner |
| DFT-s-OFDM | QPSK | ≤ 15.5 | ≤ 12 |
|  | 16 QAM | ≤ 14.5 | ≤ 11 |
|  | 64 QAM | ≤ 14.5 | ≤ 10 |
|  | 256 QAM | ≤ 12.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 14.5 | ≤ 10 |
|  | 16 QAM | ≤ 14.5 | ≤ 10 |
|  | 64 QAM | ≤ 14 | ≤ 8 |
|  | 256 QAM | ≤ 11 | ≤ 5.5 |
| NOTE 1: The A-MPR for NS_40 is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3-1 and Table 6.2.4-30a in TS 36.101 and MPR + A specified in Table 6.2.2-1 and Table 6.2.3.5-1. |  |  |  |

#### 6.2.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.6-1: A-MPR regions for NS_43

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 15 | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 40 | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 45 | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | < 1.8 MHz /12/SCS > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2. NOTE 2: 15 kHz SCS unless otherwise stated NOTE 3: Void |  |  |  |  |  |  |  |

Table 6.2.3.6-2: A-MPR for NS_43

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 6.2.3.6-3: Void

When NS_43U is signalled for 5 and 10 MHz channel bandwidths A-MPR is defined in Table 6.2.3.1-2 except for DFT-s-OFDM QPSK when LCRB > 5.4 MHz/12/SCS the A-MPR is 2.5 dB. For 15 MHz channel bandwidth Table 6.2.3.6-4 applies.

Table 6.2.3.6-4: A-MPR for NS_43U

| Modulation/Waveform |  | 15 MHz |
| --- | --- | --- |
|  |  | Outer / Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9 |
|  | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |
| CP-OFDM | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |

#### 6.2.3.7	A-MPR for NS_03 and NS_03U

Table 6.2.3.7-1 A-MPR for NS_03

Table 6.2.3.7-1 A-MPR for NS_03

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
|  | 256 QAM | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.5 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |

In case UE operates in a band where NS_03U applies and it receives additionalSpectrumEmission value of 3 then A-MPR values specified in Table 6.2.3.7-1 apply with an exception that DFT-s-OFDM Pi/2 BPSK A-MPR is 2 dB.

#### 6.2.3.8	A-MPR for NS_37

Table 6.2.3.8-1: A-MPR regions for B11/B21 protection (NS_37) (1447.9 - 1462.9 MHz)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |  | Region C (Outer/Inner) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 10 | 1452.9 < FC ≤ 1457.9 | ≥ 0 | > 7.2 MHz/12/SCS | ≤ A1 | N/A | N/A | N/A | N/A | N/A | N/A |
| 15 | FC = 1455.4 | ≥ 0 | > 9.9 MHz/12/SCS | ≤ A1 | < 0.54 MHz/12/SCS | < 1.08 MHz/12/SCS | ≤ A2 | > 13.86 MHz/12/SCS | < 1.08 MHz/12/SCS | ≤ A2 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.8-2 NOTE 2: Void NOTE 3: Void NOTE 4: No A-MPR for SCS = 60 kHz for region B and C only. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.8-2: A-MPR for NS_37

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 3 |
|  | QPSK | ≤ 1.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 3 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3 |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |

#### 6.2.3.9	A-MPR for NS_38

Table 6.2.3.9-1: A-MPR for EESS (NS_38) Protection (1430 – 1470 MHz)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A Outer/Inner |  |  | Region B Outer/Inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR (dB) | RBstart+LCRB | A-MPR (dB) |
| 5 | 1432.5 ≤ FC < 1437.5 | ≤ -1.8 MHz/12/SCS + LCRB /2 | > 3.6 MHz/12/SCS | ≤ 7 | ≤ 2.16 MHz/12/SCS | ≤ 5.5 |
| 10 | 1435 ≤ FC < 1442 | ≤ -1.8 MHz/12/SCS + LCRB /2 | > 3.6 MHz/12/SCS | ≤ 12 | ≤ 2.16 MHz/12/SCS | ≤ 9 |
| 15 | 1437.5 ≤ FC < 1447.5 | ≤ -1.8 MHz/12/SCS + LCRB /2 | > 3.6 MHz/12/SCS | ≤ 13 | ≤ 3.6 MHz/12/SCS | ≤ 10 |
| 20 | 1440 ≤ FC < 1450 | ≤ -1.8 MHz/12/SCS + LCRB /2 | > 3.6 MHz/12/SCS | ≤ 13 | ≤ 5.4 MHz/12/SCS | ≤ 10 |
| NOTE 1 - 4: Void |  |  |  |  |  |  |

#### 6.2.3.10	A-MPR for NS_39

Table 6.2.3.10-1: A-MPR for own RX (NS_39) Protection (1440 – 1470 MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A (Outer/Inner) |  |
| --- | --- | --- | --- |
|  |  | RBstart+LCRB | A-MPR (dB) |
| 10 | 1462 < FC ≤ 1465 | > 7.9 MHz/12/SCS | ≤ 6 |
| 15 | 1456.3 < FC ≤ 1462.5 | > 11.2 MHz/12/SCS | ≤ 6 |
| 20 | 1450.8 < FC ≤ 1460 | > 14.4 MHz/12/SCS | ≤ 6 |
| NOTE 1 - 4: Void |  |  |  |

#### 6.2.3.11	A-MPR for NS_41

Table 6.2.3.11-1: A-MPR for NS_41

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A Outer/Inner |  |  | Region B Outer/Inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR (dB) | RBstart+LCRB | A-MPR (dB) |
| 5 | - | - | - | - | - | - |
| 10 | 1437 ≤ FC < 1442 | ≤ -4.5 MHz/12/SCS + LCRB | > 4.5 MHz/12/SCS | ≤ 9 | < 1.8 MHz/12/SCS | ≤ 9 |
| 15 | 1439.5 ≤ FC < 1447.5 | ≤ -5.4 MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 11 | < 3.42 MHz/12/SCS | ≤ 9 |
| 20 | 1442 ≤ FC < 1450 | ≤ -5.4 MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 12 | < 5.04 MHz/12/SCS | ≤ 9 |
| 30 | 1452 ≤ FC < 1502 | ≤ -7.2MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 11.7 MHz/12/SCS | ≤ 13.5 |
| 40 | 1452 ≤ FC < 1497 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 11.7 MHz/12/SCS | ≤ 13.5 |
| 50 | 1457 ≤ FC < 1492 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 15.12 MHz/12/SCS | ≤ 13.5 |
| 60 | 1462 ≤ FC < 1487 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 18.72 MHz/12/SCS | ≤ 13.5 |
| NOTE 1 - 4: Void |  |  |  |  |  |  |

#### 6.2.3.12	A-MPR for NS_42

Table 6.2.3.12-1: A-MPR for NS_42

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart+LCRB | A-MPR Outer/Inner (dB) | RBstart | RBstart+LCRB | A-MPR Inner (dB) | A-MPR Outer (dB) |
| 5 | 1512 ≤ FC ≤ 1514.5 | > 3.1 MHz / 12 / SCS | ≤ 7 | < 0.90 MHz / 12 / SCS | ≤ 3.1 MHz / 12 / SCS | ≤ 1.5 | ≤ 4 |
| 10 | 1497 ≤ FC ≤ 1512 | > 6.2 MHz / 12 / SCS | ≤ 8 | < 0.90 MHz / 12 / SCS | ≤ 6.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 15 | 1502 ≤ FC ≤ 1509.5 | > 9.3 MHz / 12 / SCS | ≤ 8 | < 3.06 MHz / 12 / SCS | ≤ 9.3 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 20 | 1497 ≤ FC ≤ 1507 | > 12.4 MHz / 12 / SCS | ≤ 8 | < 4.50 MHz / 12 / SCS | ≤ 12.4 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 30 | 1477 ≤ FC ≤ 1502 | > 24.8 MHz / 12 / SCS | ≤ 8 | < 5.40 MHz / 12 / SCS | ≤ 24.8 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 40 | 1477 ≤ FC ≤ 1497 | > 24.8 MHz / 12 / SCS | ≤ 8 | < 5.40 MHz / 12 / SCS | ≤ 24.8 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 50 | 1467 ≤ FC ≤ 1492 | > 31 MHz / 12 / SCS | ≤ 8 | < 7.20 MHz / 12 / SCS | ≤ 31 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 60 | 1462 ≤ FC ≤ 1487 | > 37.2 MHz / 12 / SCS | ≤ 8 | < 7.20 MHz / 12 / SCS | ≤ 37.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| NOTE 1 - 5: Void |  |  |  |  |  |  |  |

#### 6.2.3.13	A-MPR for NS_18

Table 6.2.3.13-0: Band n28 and n83 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |

Table 6.2.3.13-1: A-MPR for NS_18

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

#### 6.2.3.14	A-MPR for NS_21

Table 6.2.3.14-1: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 1.44MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS | Region A1b RBstart ≤ 1.44MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A2 LCRB > 5.4MHz/12/SCS | Region A3b RBend ≥ 7.74MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A3a RBend ≥ 7.74MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 10 | DFT-s-OFDM | PI/2 BPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | QPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | 16 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 64 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 256 QAM | 6 | 3 | 4 | 3 | 6 |
|  | CP-OFDM | QPSK | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 16 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 64 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 256 QAM | 6 | 4 | 5.5 | 4 | 6 |

#### 6.2.3.15	A-MPR for NS_24

Table 6.2.3.15-1: A-MPR for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | Fc=1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | Fc=1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | Fc=2002.5 |  | >2.16 | A1 | >3.78 | ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
| 10MHz | Fc=1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | Fc=1995 |  | >4.5 | A1 | >7.56 | ≤4.32 | A2 | ≤7.38 | ≤4.32 | A3 |
| 10MHz | Fc=2000 | >6.84 |  | A5 | <2.88 |  | A5 | ≥3.06 ≤6.66 | >1.44 | A6 |
| 15MHz | Fc=1987.5 |  | >7.02 | A1 | >11.52 | ≤6.84 | A2 | ≤11.34 | ≤6.84 | A3 |
| 15MHz | Fc=1997.5 | >9.36 |  | A5 | <3.6 |  | A5 | ≥3.78 ≤9.18 | >1.44 | A6 |
| 20MHz | Fc=1990 | >13.5 |  | A5 | <4.5 |  | A5 | ≥4.68 ≤13.32 | >2.16 | A6 |
| 20MHz | Fc=1995 | >12.6 |  | A5 | <5.4 |  | A5 | ≥5.58 ≤12.42 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.15-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 11 | ≤ 5 |  | ≤ 8.5 | ≤ 20 | ≤ 10 |  |
| CP-OFDM QPSK | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13 | ≤ 6.5 |  | ≤ 8.5 | ≤ 20 | ≤ 12 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |

#### 6.2.3.16	A-MPR for NS_27

Table 6.2.3.16-1: A-MPR for NS_27

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  | Region B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR | LCRB*12*SCS | A-MPR |
| 15 MHz | 3557.5 ≤ FC < 3562.5 | <1.8 MHz |  |  | A3 | ≥10.8 MHz | A3 |
|  | 3687.5 < FC ≤ 3692.5 | >11.52 MHz |  |  |  |  |  |
| 15 MHz | 3562.5 ≤ FC < 3567.5 | ≤1.08 MHz |  | <1.44 MHz | A4 | ≥11.52 MHz | [2] |
|  | 3682.5 < FC ≤ 3687.5 |  | ≥13.22 MHz |  |  |  |  |
| 20 MHz | 3560 ≤ FC < 3570 | <3.6 MHz |  |  | A5 | ≥10.8 MHz | A5 |
|  | 3680 < FC ≤ 3690 | >12.96 MHz |  |  |  |  |  |
| 20 MHz | 3570 ≤ FC < 3580 | ≤2.16 MHz |  | <1.44 MHz | A6 | ≥14.4 MHz | [2] |
|  | 3670 < FC ≤ 3680 |  | ≥16.92 |  |  |  |  |
| 30 MHz | 3565 ≤ FC < 3585 | < [7.38] MHz |  |  | A7 |  |  |
|  |  | ≥ [7.38] MHz |  | ≥ [15.3] MHz | A2 |  |  |
|  |  | ≤[24.48] MHz |  | ≤ [15.3] MHz | A1 |  |  |
|  |  |  |  | < [2.7] MHz | A7 |  |  |
|  |  | ≥[24.48] MHz | > [19.44] MHz |  | A7 |  |  |
|  | 3665 < FC ≤ 3685 |  | ≤[19.44] MHz | ≥ [15.3] MHz | A2 |  |  |
|  |  |  | ≥[3.24] MHz | < [15.3] MHz | A1 |  |  |
|  |  |  |  | < [2.7] MHz | A7 |  |  |
|  | 3585 ≤ FC ≤ 3665 |  | <[3.24] MHz | < [1.44] MHz | A8 | ≥[19.44] MHz | [4] |
|  |  | ≤[3.96] MHz |  |  | A8 |  |  |
| 40 MHz | 3570 ≤ FC < 3600 | <11.34 MHz |  |  | A7 |  |  |
|  |  | ≥11.34 MH, ≤31.0 MHz |  | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  | >31.0 MHz |  | <3.6 MHz | A7 |  |  |
|  | 3650 < FC ≤ 3680 |  | >24.48 MHz |  | A7 |  |  |
|  |  |  | ≤24.48 MHz, ≥6.48 MHz | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  |  | <6.48 MHz | <3.6 MHz | A7 |  |  |
| 40 MHz | 3600 ≤ FC ≤ 3650 | ≤6.12 MHz |  | <1.44 MHz | A8 | >[20] MHz | [4.5] |
|  |  |  | ≥ 32.76 |  |  |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

Table 6.2.3.16-2: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | [4.5] | [6] | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | QPSK | [4.5] | [6] | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | 16 QAM | [4.5] | [6] | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 64 QAM | [4.5] | [6] | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 256 QAM |  | [6] |  |  |  |  | 11 |  |
| CP-OFDM | QPSK | [5.5] | [7] | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 16 QAM | [5.5] | [7] | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 64 QAM | [5.5] | [7] | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 256 QAM |  | [7] |  |  |  |  | 11.5 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |  |

#### 6.2.3.17	A-MPR for NS_46

Table 6.2.3.17-1: A-MPR regions for NS_46

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2534.5 ≤ FC < 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: > 9.72 MHz for DFT-s-OFDM, > 16.02 MHz for CP-OFDM. |  |  |  |  |

Table 6.2.3.17-2: A-MPR for NS_46

| Modulation/Waveform |  | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM | 6 |  |  |  | 7 | 11 |

#### 6.2.3.18	A-MPR for NS_47

Table 6.2.3.18-1: A-MPR regions and types for NS_47

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.18-2. |  |  |  |  |

Table 6.2.3.18-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1(dB) |  | A2(dB) |  | A3(dB) |  | A4(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 |  | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 |  | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  |  |  | ≤ 7 |

#### 6.2.3.19	A-MPR for NS_50

Table 6.2.3.19-1: A-MPR regions for NS_50

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 20 | ≤ 1.44 | A8 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 25 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-2. |  |  |  |

Table 6.2.3.19-2: A-MPR for NS_50

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 2 |
|  | QPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2 |
|  | 16 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2.5 |
|  | 64 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 |  | ≤ 5 |  |
|  | 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5 |  | ≤ 5 |  |
| CP-OFDM | QPSK | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  |
|  | 16 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  |
|  | 64 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 |  | ≤ 6.5 |  |
|  | 256 QAM | ≤ 12 | ≤ 8 |  |  |  | ≤ 6.5 |  |

#### 6.2.3.20	A-MPR for NS_44

Table 6.2.3.20-1: A-MPR regions for NS_44

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2582.5≤ FC ≤ 2602.5 | <18.0 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥18.0 | <7.2 | A3 |
|  |  | ≥18.0 | ≥7.2 | A6 |
| 30 MHz | 2585 ≤ FC ≤ 2600 | <21.6 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥21.6 | <12.6 | A3 |
|  |  | ≥21.6 | ≥12.6 | A6 |
| 40 MHz | 2590 ≤ FC ≤ 2595 | ≥0, <2.88 | >0 | A1 |
|  |  | ≥2.88, <14.4 | >max (0, 12*SCS*RBend - 3.6) | A2 |
|  |  | ≥14.4, <23.4 | >10.8 | A3 |
|  |  | ≥23.4, <32.4 | >16.2 | A4 |
|  |  | ≥32.4 | >0 | A5 |

Table 6.2.3.20-2: A-MPR for NS_44

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | QPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 16 QAM | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 64 QAM | 5 |  | 3 | 7 | 12 | 4 |
|  | 256 QAM | 5 |  |  | 7 | 12 |  |
| CP-OFDM | QPSK | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 16 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 64 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 256 QAM |  |  |  | 8 | 12 |  |

#### 6.2.3.21	A-MPR for NS_12

Table 6.2.3.21-1: A-MPR regions for NS_12

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 5MHz | ≤1.8 | >0 | A1 |
| 10MHz | ≤3.6 | >0 | A1 |

Table 6.2.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 |
| CP-OFDM QPSK | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 9.5 |

#### 6.2.3.22	A-MPR for NS_13

Table 6.2.3.22-1: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |

Table 6.2.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |

#### 6.2.3.23	A-MPR for NS_14

Table 6.2.3.23-1: A-MPR regions for NS_14

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | >0 | ≥9 | A2 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | >0 | ≥9 | A2 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | >0 | ≥9 | A2 |

Table 6.2.3.23-2: A-MPR for NS_14

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3 |  | ≤ 3 |
| DFT-s-OFDM 256 QAM |  |  | ≤ 8 |
| CP-OFDM QPSK | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 6 |  | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 |  | ≤ 8 |

#### 6.2.3.24	A-MPR for NS_15

Table 6.2.3.24-1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >0 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >0 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >0 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >0 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  | Fc ≤ 835 | ≥13.68 | >0 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |

Table 6.2.3.24-2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 |

#### 6.2.3.25	A-MPR for NS_45

Table 6.2.3.25-1: A-MPR for NS_45

| Modulation/Waveform |  | Outer |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.5 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |

#### 6.2.3.26	A-MPR for NS_48

Table 6.2.3.26-1: A-MPR regions for NS_48

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥13.5 | A3 |
|  |  | ≥21.6 | <1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A2 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 3.6) | A3 |
|  |  | ≥17.1, <27.36 | ≥13.5 | A4 |
|  |  | ≥27.36, <34.56 | ≥13.5 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <6.12 | >0 | A2 |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥20.7, <41.04 | ≥17.1 | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 6.2.3.26-2: A-MPR for NS_48

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | QPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 16 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 64 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 256 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
| CP-OFDM | QPSK | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 16 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 64 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 256 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |

#### 6.2.3.27	A-MPR for NS_49

Table 6.2.3.27-1: A-MPR regions for NS_49

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤3.96 | <1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <6.48 | ≥0 | A5 |
|  |  | ≥6.48, <14.4 | ≥max (0,12*SCS* RBend - 3.6) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.8 | A4 |
|  |  | ≥21.6 | ≥10.8 | A2 |
|  |  | ≥21.6 | <1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <7.2 | ≥0 | A1 |
|  |  | ≥7.2, <10.44 | <1.08 | A5 |
|  |  | ≥7.2, <18 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥18, <34.56 | ≥14.4, <28.8 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥28.8 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥7.74, <14.4 | < min [1.08, max(0,12*SCS* RBend-7.74)] | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥18, <max (0, 12*SCS*RBend – 7.74) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 7.74) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6.2.3.27-2: A-MPR for NS_49

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | QPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 16 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 64 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 256 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
| CP-OFDM | QPSK | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 16 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 64 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 256 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |

#### 6.2.3.28	A-MPR for NS_51

Table 6.2.3.28-1: A-MPR regions for NS_51

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 50 MHz | Fc ≤ 1945 | ≤ 4.5 | > 0 | A7 |
|  |  | >4.5, < 32.4 | ≥ max(0, 12*SCS*RBend - 14.4) | A4 |
|  |  | < 32.4 | < max(0, 12*SCS*RBend - 14.4) | A5 |
|  |  | ≥ 32.4 | > 0 | A6 |
| 50 MHz | 1945 < Fc ≤ 1980 | < 27 | ≥ max(0, 12*SCS*RBend - 14.4) | A1 |
|  |  | < 27 | < max(0, 12*SCS*RBend - 14.4) | A2 |
|  |  | ≥ 27 | > 0 | A3 |

Table 6.2.3.28-2: A-MPR for NS_51

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A5 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 15 | 12.5 | 22 | 7 | 4.5 | 16 | 14 |
|  | QPSK | 15 | 12.5 | 22 | 7 | 4.5 | 16 | 14 |
|  | 16 QAM | 15 | 12.5 | 22 | 7 | 4.5 | 16 | 14 |
|  | 64 QAM | 15 | 12.5 | 22 | 7 | 4.5 | 16 | 14 |
|  | 256 QAM | 15 | 12.5 | 22 | 7 | 4.5 | 16 | 14 |
| CP-OFDM | QPSK | 15 | 12.5 | 22 | 8.5 | 4.5 | 17 | 14 |
|  | 16 QAM | 15 | 12.5 | 22 | 8.5 | 4.5 | 17 | 14 |
|  | 64 QAM | 15 | 12.5 | 22 | 8.5 | 4.5 | 17 | 14 |
|  | 256 QAM | 15 | 12.5 | 22 | 8.5 | 4.5 | 17 | 14 |

#### 6.2.3.29	A-MPR for NS_07

Table 6.2.3.29-1: A-MPR regions for NS_07

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 | >0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | >3.42 | ≤ 0.36 | A4 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |

Table 6.2.3.29-2: A-MPR for NS_07

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM QPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 16 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 64 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 256 QAM | 12 | 9 | 6 | 3 |
| CP-OFDM QPSK | 14 | 10 | 7 | 3 |
| CP-OFDM 16 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 64 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 256 QAM | 14 | 10 | 7 | 3 |

#### 6.2.3.30	A-MPR for NS_52

For 5 MHz channel centered on frequencies (FC) = 1630.0, 1630.3 MHz, A-MPR is defined as

if RBstart <= ceil{[3/SCS/15 kHz])}and LCRB <= ceil{[17/SCS/15 kHz)]},

then

the A-MPR = [14] dB for SCS = 15 kHz and AMPR = [8] dB for SCS >= 30 kHz,

else,

if RBstart <= ceil{[3/(SCS/15 kHz)]} and LCRB > ceil{[17/(SCS/15 kHz)]},

then

the A-MPR = [6] dB,

else,

if RBstart <= ceil{[8/(SCS/15 kHz)]},

then

the A-MPR = [4] dB.

For 5 MHz channel centered on frequencies (Fc) = 1635.0, 1649.0, 1654.0 MHz, no A-MPR is needed.

For Channel 10 MHz with center frequency of 1632.5 MHz, A-MPR is defined as

if RBstart < ceil{[3/(SCS/15 kHz)]} and LCRB <= ceil{[8/(SCS/15 kHz)]},

then

the A-MPR = [12] dB for SCS = 15 kHz and AMPR = [8] dB for SCS >= 30 kHz,

else,

if RBstart < ceil{[9/(SCS/15 kHz)]}, and LCRB > ceil{[8/(SCS/15 kHz)]},

then

the A-MPR = [8] dB,

else,

if RBstart <= ceil{[18/(SCS/15 kHz)]},

then

the A-MPR = [6] dB

else,

if RBstart [>= ceil{TBD}], and LCRB [> ceil{TBD}],

then

the A-MPR = [TBD] dB.

For 10 MHz channel centered on frequency of 1651.5 MHz, no A-MPR is needed.

Note that the additional emission requirements in Table 6.5.3.3.27 are specified in EIRP.  The above A-MPR values to meet these additional emission requirements have been estimated using a 0 dBi antenna gain.  The A-MPR may have to be adjusted if the supported antenna gain Gant declared by the UE manufacturer is different from 0 dBi.

### 6.2.4	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1-1 without taking into account the tolerance specified in the Table 6.2.1-1;

When  the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 capable UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less symbols in certain evaluation period are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 capable UE operating in TDD bands n40, n41, n77, n78, and n79 with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass = 3 dB for a power class 2 capable UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); 3 dB for a power class 1.5 capable UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); otherwise ΔPPowerClass = 0 dB;

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

MPRc and A-MPRc for serving cell c are specified in clause 6.2.2 and clause 6.2.3, respectively;

∆MPRc for serving cell c is specified in clause 6.2.2.

∆TRxSRS is applied when

a)	UE transmits SRS to other than first SRS port when the SRS-TxSwitch capability is indicated as  '1T2R', '1T4R' or, '1T4R/2T4R'

b)	UE transmits SRS to other than first or second SRS port when the SRS-TxSwitch capability is indicated as  '2T4R' or '1T4R/2T4R', or

c)	UE transmits SRS to a DL-only carrier

The value of ∆TRxSRS is 4.5dB for n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 in the band.  The value of ∆TRxSRS is 7.5dB for n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 2 in the band.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the power management maximum power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum  value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4-1. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1-1.

Table 6.2.4-1: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

## 6.2A	Transmitter power for CA

### 6.2A.1	UE maximum output power for CA

#### 6.2A.1.1	UE maximum output power for Intra-band contiguous CA

For uplink intra-band contiguous carrier aggregation, the maximum output power is specified in Table 6.2A.1.1-1. For downlink intra-band contiguous carrier aggregation with a single uplink component carrier configured in the NR band, the maximum output power is specified in Table 6.2.2-1.

Table 6.2A.1.1-1: UE Power Class for intra-band contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7B |  |  |  |  | 23 | +2/-2 |  |  |
| CA_n41C |  |  |  |  | 23 | +2/-21 |  |  |
| CA_n48B |  |  |  |  | 23 | +2/-2 |  |  |
| CA_n77C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_n78C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_n79C |  |  |  |  | 23 | +2/-2 |  |  |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For intra-band contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

#### 6.2A.1.2	UE maximum output power for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements in clause 6.2.2 apply. For intra-band non-contiguous carrier aggregation with two uplink carriers the maximum output power is specified in Table 6.2A.1.2-1.

Table 6.2A.1.2-1: UE Power Class for intraband non-contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41(2A) |  |  |  |  | 23 | +2/-31 |  |  |
| CA_n77(2A) |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78(2A) |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For intra-band non-contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

#### 6.2A.1.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in clause 6.2 apply.

For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.3-1.

For PC3 inter-band carrier aggregation with one uplink component carrier assigned to one NR band in NR band n41, n77, n78, and n79, the requirements for power class 2 are not applicable and the corresponding requirements for a power class 3 UE shall apply.

Table 6.2A.1.3-1 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n78A |  |  | 266 | +2/-32 | 23 | +2/-32 |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n18A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3-n38A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n25A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n48A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n25A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n39A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n13A-n25A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n13A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n18A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n20A-n28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n20A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n25A-n38A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n25A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n25A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n25A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n25A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n50A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n34A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n38A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n38A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n39A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n39A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n40A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n40A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n40A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n71A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n50A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n46A-n48A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n46A-n48B |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n66A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n66A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n71A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n71A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n77A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n78A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n78A-n92A |  |  |  |  | 23 | +2/-32 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. |  |  |  |  |  |  |  |  |

#### 6.2A.1.4	Void

#### 6.2A.1.5	Void

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.1		UE maximum output power reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.4-1 with contiguous RB allocation is specified in Table 6.2A.2.4-1 for UE power class 3 CA bandwidth classes B and C.

In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

Table 6.2A.2.1-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

For CA bandwidth class B and bandwidth class C with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2^µ1 + LCRB2 ∙ 2^µ2

NRB_alloc= (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2,

NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2.

If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2,

if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.5-1 with non-contiguous RB allocation is specified in Table 6.2A.2.4-2 for UE power class 3 CA bandwidth classes B and C.

Table 6.2A.2.1-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

For CA bandwidth classes B and C with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0, when both uplink CCs are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of CC1 and CC2 respectively.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_CA , BWgap and BWGB,low are as defined for the Inner region.

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

#### 6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPRIM3 defined in Clause 6.2A.2.2.2

Else If AND( FIM3,low_block,low > SEM-25,low ,  FIM3,high_block,high < SEM-25,high )

MPRIM3 defined in Clause 6.2A.2.2.1

Else

MPRIM3 defined in Clause 6.2A.2.2.1

where

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-25,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-25,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.2.

##### 6.2A.2.2.1	MPRIM3 to meet -30dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15; 	0 ≤ B < 1.08

14.5; 	1.08 ≤ B < 2.16

13.5; 	2.16 ≤ B < 3.24

12.5;       3.24 ≤ B < 5.04

11.5; 	5.04≤ B < 10.08

10.5; 	10.08 ≤ B < 16.38

10;        16.38 ≤ B < 21.78

9; 	      21.78 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

##### 6.2A.2.2.2	MPRIM3 to meet -13dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

#### 6.2A.2.3	UE maximum output power reduction for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.2 apply for each uplink component carrier.

#### 6.2A.2.4	Void

### 6.2A.3	UE additional maximum output power reduction for CA

#### 6.2A.3.1.1	 UE additional maximum output power reduction for Intra-band contiguous CA

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7]. Relation between NR CA band and NR frequency band is specified in Table 5.2A.1-1.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2A.1.5-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2.4. In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2A.3.1.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NS value. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1.1-2.

Table 6.2A.3.1.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR CA Band | Aggregated channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_01 |  | Table 5.2A.1-1 | All applicaple NR CA bands | All applicaple NR CA configurations | N/A |
| CA_NS_04 | 6.5A.2.3.1.1 6.5A.3.3.1.1 | CA_n41 | Table 5.5A.1-1 | 6.2A.3.1.1.1 | 6.2A.3.1.1.1 |
| CA_NS_27 | 6.5A.2.3.1.2 6.5A.3.3.1.2 | CA_n48 | Table 5.5A.1-1 | 6.2A.3.1.1.2 | 6.2A.3.1.1.2 |
| CA_NS_46 | 6.5A.3.3.1.3 | CA_n7 | Table 5.5A.1-1 | 6.2A.3.1.1.3 | 6.2A.3.1.1.3 |

[The CA_NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2.3.1.1-2: Mapping of network signaling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NS_01 | CA_NS_04 |  |  |  |  |  |  |
| CA_n48 | CA_NS_01 | CA_NS_27 |  |  |  |  |  |  |
| CA_n7 | CA_NS_01 | CA_NS_46 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

##### 6.2A.3.1.1.1	A-MPR for CA_NS_04

6.2A.3.1.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and scs when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in Table 6.2A.2.4-1 then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in Table 6.2A.2.4-2,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C.

Where

-	MPR is the MPR as defined in Table 6.2A.2.4-1 for the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2 AMPR in table 6.2.3.2-2.

6.2A.3.1.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.1-1 or Table 6.5A.3.3.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

if allocation is an inner or outer 1 allocation as defined in Table 6.2A.2.4-2 then A-MPR = MPR

if allocation is an outer 2 allocation as defined in Table 6.2A.2.4-2 then A-MPR = MPR-1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

if RB allocation is an inner or outer 1 allocation as defined in Table 6.2A.2.4-1 then A-MPR = MPR

if RB allocation is an outer 2 allocation as defined in Table 6.2A.2.4-2 then A-MPR = MPR-1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.2.2.1

where

-	MPR is the MPR as defined in Table 6.2A.2.4-2 for the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

6.2A.3.1.1.1.3	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;        16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000,000

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

6.2A.3.1.1.2	A-MPR for CA_NS_27

6.2A.3.1.1.2.1	Contiguous allocations

For all modulations and scs when Fedge, low - BWChannel_CA ≥ 3540 MHz AND Fedge, high + BWChannel_CA ≤ 3710 MHz

if allocation is inner 1 then A-MPR = 0 dB where inner 1 is defined as

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB_agg – RBStart,Low – LCRB

with following conditions

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_agg /2)

Inner 1 region exceptions thresholds are for LCRB < 8 and

RBstart ≤ 30 and RBend ≥ 164 for BWChannel_CA = 40MHz, and

when 3540 MHz + BWChannel_CA ≤ Fedge, low < 3530 MHz + 2*BWChannel_CA,

RBstart ≤ 25 for BWChannel_CA = 35MHz, and

RBstart ≤ 19 for BWChannel_CA = 30MHz, and

RBstart ≤ 14 for BWChannel_CA = 25MHz, and

RBstart ≤ 9 for BWChannel_CA = 20MHz, and

RBstart ≤ 3 for BWChannel_CA = 15MHz, and

when 3720 MHz – 2*BWChannel_CA < Fedge, high ≤ 3710 MHz - BWChannel_CA,

RBend ≥ 144 for BWChannel_CA = 35MHz, and

RBend ≥ 124 for BWChannel_CA = 30MHz, and

RBend ≥ 104 for BWChannel_CA = 25MHz, and

RBend ≥  80 for BWChannel_CA = 20MHz, and

RBend ≥ 68 for BWChannel_CA = 15MHz,

For which AMPR = 5dB.

else A-MPR= 5 dB

For all modulations and scs when 3550 MHz ≤ Fedge, low < 3540 MHz + BWChannel_CA

if allocation is inner 3 then A-MPR = 0 dB.

Inner 3 region exceptions thresholds are

RBstart ≤ 63 for BWChannel_CA = 40MHz, and

RBstart ≤ 52 for BWChannel_CA = 35MHz, and

RBstart ≤ 42 for BWChannel_CA = 30MHz, and

For which AMPR = 7dB for BWChannel_CA ≤ 20MHz and 11.5dB for BWChannel_CA > 20MHz

where inner 3 is defined as

RBStart = NRB_agg /4

LCRB = NRB_agg/4

RBStart = NRB_agg 3/4  LCRB

with following conditions

NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

else when BWagg ≤ 20 MHz, A-MPR = 7 dB or when BWagg > 20 MHz, A-MPR = 11.5dB.

For all modulations and scs when 3710 MHz - BWChannel_CA < Fedge, high ≤ 3700

if allocation is inner 3 then A-MPR = 0 dB.

Inner 3 region exceptions thresholds are

RBend ≥ 132 for BWChannel_CA = 40MHz, and

RBend ≥ 121 for BWChannel_CA = 35MHz, and

RBend ≥ 110 for BWChannel_CA = 30MHz, and

For which AMPR = 7dB for BWChannel_CA ≤ 20MHz and 11.5dB for BWChannel_CA > 20MHz

where inner 3 is defined as

RBStart = NRB_agg /4

LCRB = NRB_agg/4

RBStart = NRB_agg 3/4  LCRB

with following conditions

NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

else when BWagg ≤ 20 MHz, A-MPR = 7 dB or when BWagg > 20 MHz, A-MPR = 11.5dB.

6.2A.3.1.1.2.2	Non-contiguous allocations

For all modulations and scs when Fedge, low - BWChannel_CA ≥ 3540 MHz AND Fedge, high + BWChannel_CA ≤ 3710 MHz

A-MPRCA_IM5=

13; 	0 ≤B<1.08

12; 	1.08 ≤B<2.16

11; 	2.16 ≤B<3.24

10.5; 	3.24 ≤ B < 5.04

9.5; 	5.04≤B< 10.08

8; 		10.08 ≤B< 16.56

7; 		16.56 ≤ B < 21.96

6.5; 		21.96 ≤B

For all modulations and scs when 3550 MHz ≤ Fedge, low < 3540 MHz + BWChannel_CA or 3710 MHz - BWChannel_CA < Fedge, high ≤ 3700

when BWagg ≤ 20 MHz

A-MPRCA_IM5=

13; 	0 ≤B<1.08

12; 	1.08 ≤B<2.16

11; 	2.16 ≤B<3.24

10.5; 	3.24 ≤ B < 5.04

9.5; 	5.04 ≤B< 10.08

8; 		10.08 ≤B< 16.56

7; 		16.56 ≤ B < 21.96

6.5; 	21.96 ≤B

or when BWagg > 20 MHz

A-MPRCA_IM3 =

20; 	0 ≤B<1.08

19.5; 	1.08 ≤B<2.16

19; 	2.16 ≤B<3.24

18.5; 	3.24 ≤ B < 5.04

18; 	5.04 ≤B< 10.08

17; 	10.08 ≤B< 16.56

16; 	16.56 ≤ B < 21.96

13; 	21.96 ≤B.

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000,000

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

6.2A.3.1.1.3	A-MPR for CA_NS_46

6.2A.3.1.1.3.1	Contiguous allocations

[For all modulations and scs when BWChannel_CA > 25 MHz

IF			RBend > NRB_agg 5/6 with the exception of NRB_agg 3/4 for BWChannel_CA = 50MHz OR RBend > 4/3 NRB_agg - LCRB

THEN		A-MPR = 11dB

ELSE IF 	RBend < NRB_agg /6 AND LCRB < 5

THEN		A-MPR = 5dB

ELSE IF	LCRB 3/2< RBend < NRB_agg 3/4 AND LCRB < NRB_agg /4

THEN 	A-MPR = 0 dB,

OTHERWISE	A-MPR = [7] dB.

For all modulations and scs when BWChannel_CA <= 25 MHz and 2595 MHz – 2*BWChannel_CA < Fedge,high ≤ 2570 MHz

IF 			RBend ≥ 4/3 NRB_agg - LCRB

THEN 	A-MPR = 6 dB.

OTHERWISE	A-MPR = 0 dB.

For all modulations and scs when BWChannel_CA <= 25 MHz and Fedge_high <= 2595 MHz – 2*BWChannel_CA,

A-MPR = 0 dB.]

6.2A.3.1.1.3.2	Non-contiguous allocations

[For all modulations and scs when BWChannel_CA > 25 MHz and 2595 MHz - BWChannel_CA ≤ Fedge_high ≤ 2570 MHz

A-MPRCA_IM3 =

20; 	0 ≤B<1.08

19.5; 	1.08 ≤B<2.16

19; 	2.16 ≤B<3.24

18.5; 	3.24 ≤ B < 5.04

18; 	5.04 ≤B< 10.08

17; 	10.08 ≤B< 16.56

16; 	16.56 ≤ B < 21.96

13; 	21.96 ≤B

For all modulations and scs when BWChannel_CA > 25 MHz and Fedge_high < 2595 MHz - BWChannel_CA

A-MPRCA_IM5 =

13; 	0 ≤B<1.08

12; 	1.08 ≤B<2.16

11; 	2.16 ≤B<3.24

10.5; 	3.24 ≤ B < 5.04

9.5; 	5.04 ≤B< 10.08

8; 		10.08 ≤B< 16.56

7.5; 	16.56 ≤ B < 21.96

7; 		21.96 ≤B

For all modulations and scs when BWChannel_CA <= 25 MHz and 2595 MHz – 2*BWChannel_CA ≤ Fedge_high ≤ 2570 MHz

A-MPRCA_IM5 =

13; 	0 ≤B<1.08

12; 	1.08 ≤B<2.16

11; 	2.16 ≤B<3.24

10.5; 	3.24 ≤ B < 5.04

9.5; 	5.04 ≤B< 10.08

8; 		10.08 ≤B< 16.56

7.5; 	16.56 ≤ B < 21.96

7; 		21.96 ≤B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000,000

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.]

##### 6.2A.3.1.2	UE additional maximum output power reduction for Intra-band non-contiguous CA

Table 6.2A.3.1.2-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NC_NS value. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2A.3.1.2-2.

Table 6.2A.3.1.2-1: Additional Maximum Power Reduction (A-MPR) for intra-band non-contiguous CA

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 |  | All applicaple NR CA configurations | N/A |
| CA_NC_NS_04 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |

Table 6.2A.3.1.2-2: Mapping of network signaling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NC_NS_01 | CA_NC_NS_04 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

6.2A.3.1.2.1	AMPR for CA_NC_NS_04 (CA_n41(2A))

For intra-band non-contiguous CA_n41(2A) and it receives IE CA_NC_NS_04, the UE determins the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified into 2 types: AMPR to meet -25dBm/MHz and -13dBm/MHz. The A-MPR defined in this clause is used instead of MPR defined in 6.2A.2.2, not additively, so CA MPR=0 when CA_NC_NS_04 is signaled.

The UE determins the AMPR type as follows:

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

A-MPRIM3 defined in Clause 6.2A.3.2.1.2

Else

A-MPRIM3 defined in Clause 6.2A.3.2.1.1

where

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = [2480 MHz]

-	Ffilter,high = [2745 MHz]

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.2.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.2.

6.2A.3.1.2.1.1	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band non-contiguous CA_n41(2A) power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

AMPRIM3=MAWhere MA is defined as follows

MA = 	12; 	0 ≤ B < 1.08

12; 	1.08 ≤ B < 2.16

11; 	2.16 ≤ B < 3.24

10;       3.24 ≤ B < 5.04

9; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.38

7;        16.38 ≤ B < 21.78

6; 	      21.78 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

6.2A.3.1.2.1.2	AMPRIM3 to meet -13dBm/MHz

AMPR in this clause is for intra-band non-contiguous CA_n41(2A) power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

AMPRIM3=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

##### 6.2A.3.1.3	UE additional maximum output power reduction for Inter-band CA

Unless otherwise stated, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply for each uplink component carrier.

### 6.2A.4	Configured output power for CA

#### 6.2A.4.1	Configured transmitted power level

##### 6.2A.4.1.1	Configured transmitted power for Intra-band contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in clause 6.2.4, MPRc and A-MPRc are determined by clause 6.2.2. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,PPowerClass,CA – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA}

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2A.1.1-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in clause 6.2A.2 and 6.2A.3, respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c;

-	∆TRxSRS is the highest value among all serving cells c;

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.

For uplink intra-band contiguous carrier aggregation, when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in clause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different or same bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.

TREF and Teval are specified in Table 6.2A.4.1.3-0 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

Table 6.2A.4.1.1-0: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.1.3-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1-2 for inter-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or slot pattern, shall be within the following range:

P'CMAX_L–  MAX{TL, TLOW (P'CMAX_L)} ≤  P'UMAX  ≤  P'CMAX_H + THIGH (P'CMAX_H)

P'UMAX = 10 log10 ∑ p'UMAX,c

where p'UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P'CMAX) and THIGH(P'CMAX) for applicable values of P'CMAX are specified in Table 6.2A.4.1.3-1 for inter-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1 for inter-band carrier aggregation.

where:

P'CMAX_L  = MIN{ MIN {10log10∑( pCMAX_L,f,c(i),i), PPowerClass,CA} over all overlapping slots in TREF}

P'CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c , PPowerClass,CA} over all overlapping slots in TREF}

Table 6.2A.4.1.1-1: PCMAX tolerance for uplink intra-band contiguous CA

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

##### 6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.

[The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4, MPRc and A-MPRc are determined by subclause 6.2.2. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.]

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band non-contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,PPowerClass,CA – MAX(MAX(MPRc, A-MPRc) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA}

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.2-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.2A.2 and subclause 6.2A.3 respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in Table 6.2A.4.2.3-1;

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c;

-	∆TRxSRS is the highest value among all serving cells c;

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.

[For uplink intra-band non-contiguous carrier aggregation, when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in subclause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different or same bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.]

TREF and Teval are specified in Table 6.2A.4.1.3-0 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

Table 6.2A.4.1.2-1: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.1.2-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.2-1 for intra-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or slot pattern, shall be within the following range:

P'CMAX_L–  MAX{TL, TLOW (P'CMAX_L)} ≤  P'UMAX  ≤  P'CMAX_H + THIGH (P'CMAX_H)

P'UMAX = 10 log10 ∑ p'UMAX,c

where p'UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P'CMAX) and THIGH(P'CMAX) for applicable values of P'CMAX are specified in Table 6.2A.4.1.2-1 for intra-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.2-2 for intra-band carrier aggregation.

where:

P'CMAX_L  = MIN{ MIN {10log10∑( pCMAX_L,f,c(i),i), PPowerClass,CA} over all overlapping slots in TREF}

P'CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c , PPowerClass,CA} over all overlapping slots in TREF}

Table 6.2A.4.1.2-2: PCMAX tolerance for uplink intra-band non-contiguous CA

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

##### 6.2A.4.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in clause 6.2.4.

For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in clause 6.2.2 and clause 6.2.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink inter-band carrier aggregation with one serving cell c per operating band when same slot symbol pattern is used in all aggregated serving cells,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (tC,c),  pPowerClass.c/(MAX(mprc·∆mprc, a-mprc)·tC,c ·tIB,c·tRxSRS,c) , pPowerClass,c/pmprc], PEMAX,CA, PPowerClass,CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.3-1 without taking into account the tolerance specified in the Table 6.2A.1.3-1;

-	pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 without taking into account the tolerance;

-	mpr c and a-mpr c are the linear values of MPR c and A-MPR c as specified in clause 6.2.2 and clause 6.2.3, respectively;

-	∆mpr c is the linear value of ∆MPR c as specified in clause 6.2.2;

-	pmprc is the linear value of P-MPR c;

-	∆tRxSRS,c  is the linear value of ∆TRxSRS,c;

-	tC,c is the linear value of TC,ctC,c = 1.41 when NOTE 2 in Table 6.2A.1.3-1 applies for a serving cell c, otherwise tC,c = 1;

-	tIB,c  is the linear value of the inter-band relaxation term TIB,c of the serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; otherwise tIB,c In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.

For uplink inter-band carrier aggregation with one serving cell c per operating band when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in clause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.

For combinations of intra-band and inter-band carrier aggregation with UE configured for transmission on three serving cells (up to two contiguously aggregated carriers per operating band), the following apply:

For the case when p and q belong to the same band and k belongs to a different band, but p, q and k are of the same numerology and slot patterns.

PCMAX_L = MIN {10log10∑( pCMAX_L, Bi), PEMAX,CA, PPowerClass}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass}

Where

-	pCMAX_L, Bi is the linear values of PCMAX_L specified for the specific operating band Bi.

-	The linear value of PCMAX_L specified for uplink intra-band contiguous carrier aggregation in subclause 6.2A.4.1.1 applies for operating band supporting two contiguous serving cells, designated by its band index Bi. The linear value of PCMAX_L specified for single carrier in subclause 6.2.4 applies for operating band Bj supporting one serving cell.

For the case when p and q belong to the same band and are of the same numerology i and slot patterns (p,q),while k belong to a different band and is of different numerology j and/or slot pattern on the 3rd cell then:

PCMAX_L (p,q,k) = MIN {10 log10 [pCMAX_L,Bi,i(p,q) + pCMAX_L,c(3),Bj,j(k)], PEMAX,CA, PPowerClass}

PCMAX_H (p,q,k) = MIN {10 log10 [pCMAX_ H,Bi,i (p,q) + pCMAX_ H,c(3), Bj,j(k)], PEMAX,CA, PPowerClass}

Where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PEMAX,CA is p-UE-FR1 value signalled by RRC and defined in [38.331];

-	PPowerClass is the maximum UE power specified in Table 6.2A.1.3-1 without taking into account the tolerance specified in the Table 6.2A.1.3-1 or Table 6.2F.1A.1-1 for shared spectrum bands;

-	pCMAX_L,c(3),Bj,j(k) and pCMAX_ H,c(3), Bj,j(k) are the linear values of PCMAX_L and PCMAX_H respectively, specified for single carrier in subclause 6.2.4 and applies for operating band supporting one serving cell in the Bj band on numerology j, using slot pattern k;

-	pCMAX_L,Bi,i(p,q)  and pCMAX_ H,Bi,i (p,q) are the linear values of PCMAX_L respectively PCMAX_H for uplink intra-band contiguous carrier aggregation specified in subclause 6.2A.4.1.1 which applies for operating band Bi on numerology i, supporting two contiguous serving cells, using the same slot pattern (p,q).

TREF and Teval are specified in Table 6.2A.4.1.3-0 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

Table 6.2A.4.1.3-0: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.1.3-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1-2 for inter-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or symbol pattern, shall be within the following range:

P'CMAX_L–  MAX{TL, TLOW (P'CMAX_L)} ≤  P'UMAX  ≤  P'CMAX_H + THIGH (P'CMAX_H)

P'UMAX = 10 log10 ∑ p'UMAX,c

where p'UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P'CMAX) and THIGH(P'CMAX) for applicable values of P'CMAX are specified in Table 6.2A.4.1.3-1 for inter-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1 for inter-band carrier aggregation.

where:

P'CMAX_L  = MIN{ MIN {10log10∑( pCMAX_L,f,c(i),i), PPowerClass,CA} over all overlapping slots in TREF}

P'CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c , PPowerClass,CA} over all overlapping slots in TREF}

Table 6.2A.4.1.3-1: PCMAX tolerance for uplink inter-band CA (two bands)

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| PCMAX = 23 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

##### 6.2A.4.1.4	Void

#### 6.2A.4.2	ΔTIB,c for CA

For the UE which supports inter-band NR CA configuration, ΔTIB,c in tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

##### 6.2A.4.2.1	Void

##### 6.2A.4.2.2	Void

##### 6.2A.4.2.3	ΔTIB,c for Inter-band CA (two bands)

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3 | n1 | 0.3 |
|  | n3 | 0.3 |
| CA_n1-n7 | n1 | 0.5 |
|  | n7 | 0.6 |
| CA_n1-n8 | n1 | 0.3 |
|  | n8 | 0.3 |
| CA_n1-n28 | n1 | 0.3 |
|  | n28 | 0.6 |
| CA_n1-n40 | n1 | 0.5 |
|  | n40 | 0.5 |
| CA_n1-n41 | n1 | 0.5 |
|  | n41 | 0.5 |
| CA_n1-n77 | n1 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n78 | n1 | 0.3 |
|  | n78 | 0.8 |
| CA_n2-n5 | n2 | 0.3 |
|  | n5 | 0.3 |
| CA_n2-n48 | n2 | 0.6 |
|  | n48 | 0.8 |
| CA_n2-n66 | n2 | 0.5 |
|  | n66 | 0.5 |
| CA_n2-n77 | n2 | 0.6 |
|  | n77 | 0.8 |
| CA_n2-n78 | n2 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n7 | n3 | 0.5 |
|  | n7 | 0.5 |
| CA_n3-n8 | n3 | 0.3 |
|  | n8 | 0.3 |
| CA_n3-n18 | n3 | 0.3 |
|  | n18 | 0.3 |
| CA_n3-n28 | n3 | 0.3 |
|  | n28 | 0.3 |
| CA_n3-n38 | n3 | 0.5 |
|  | n38 | 0.5 |
| CA_n3-n40 | n3 | 0.5 |
|  | n40 | 0.5 |
| CA_n3-n41 | n3 | 0.5 |
|  | n41 | 0.34 |
|  |  | 0.85 |
| CA_n3-n77 | n3 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n78 | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n79 | n3 | 0.3 |
|  | n79 | 0.8 |
| CA_n5-n7 | n5 | 0.3 |
|  | n7 | 0.3 |
| CA_n5-n25 | n5 | 0.3 |
|  | n25 | 0.3 |
| CA_n5-n48 | n5 | 0.3 |
|  | n48 | 0.3 |
| CA_n5-n66 | n5 | 0.3 |
|  | n66 | 0.3 |
| CA_n5-n77 | n5 | 0.6 |
|  | n77 | 0.8 |
| CA_n5-n78 | n5 | 0.6 |
|  | n78 | 0.8 |
| CA_n7-n25 | n7 | 0.5 |
|  | n25 | 0.5 |
| CA_n7-n28 | n7 | 0.3 |
|  | n28 | 0.3 |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |
| CA_n7-n77 | n7 | 0.5 |
|  | n77 | 0.8 |
| CA_n7-n78 | n7 | 0.5 |
|  | n78 | 0.8 |
| CA_n8-n20 | n8 | 0.4 |
|  | n20 | 0.4 |
| CA_n8-n28 | n8 | 0.6 |
|  | n28 | 0.5 |
| CA_n8-n39 | n8 | 0.3 |
|  | n39 | 0.3 |
| CA_n8-n40 | n8 | 0.3 |
|  | n40 | 0.3 |
| CA_n8-n41 | n8 | 0.6 |
|  | n41 | 0.3 |
| CA n8-n75 | n8 | 0.3 |
| CA n8-n78 | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n8-n79 | n8 | 0.3 |
|  | n79 | 0.8 |
| CA_n13-n25 | n13 | 0.3 |
|  | n25 | 0.3 |
| CA_n13-n66 | n13 | 0.3 |
|  | n66 | 0.3 |
| CA_n18-n41 | n18 | 0.3 |
|  | n41 | 0.3 |
| CA_n20-n28 | n20 | 0.5 |
|  | n28 | 0.5 |
| CA_n20-n75 | n20 | 0.3 |
| CA_n20-n78 | n20 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n29 | n25 | 0.3 |
| CA_n25-n38 | n25 | 0.5 |
|  | n38 | 0.5 |
| CA_n25-n48 | n25 | 0.6 |
|  | n48 | 0.8 |
| CA_n25-n41 | n25 | 0.5 |
|  | n41 | 0.46 |
|  |  | 0.97 |
| CA_n25-n66 | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n25-n71 | n25 | 0.3 |
|  | n71 | 0.6 |
| CA_n25-n77 | n25 | 0.6 |
|  | n77 | 0.8 |
| CA_n28-n40 | n28 | 0.3 |
|  | n40 | 0.3 |
| CA_n28-n41 | n28 | 0.3 |
|  | n41 | 0.3 |
| CA_n28-n50 | n28 | 0.3 |
|  | n50 | 0.4 |
| CA_n28-n75 | n28 | 0.3 |
| CA_n28-n77 | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n28-n78 | n28 | 0.5 |
|  | n78 | 0.8 |
| CA_n28-n79 | n28 | 0.5 |
|  | n79 | 0.8 |
| CA_n29-n66 | n66 | 0.3 |
| CA_n29-n70 | n70 | 0.3 |
| CA_n34-n79 | n34 | 0.3 |
|  | n79 | 0.8 |
| CA_n38-n66 | n38 | 0.5 |
|  | n66 | 0.5 |
| CA_n38-n78 | n38 | 0.3 |
|  | n78 | 0.8 |
| CA_n39-n41 | n39 | 02 |
|  | n41 | 02 |
|  | n39 | 0.53 |
|  | n41 | 0.53 |
| CA_n39-n79 | n39 | 0.3 |
|  | n79 | 0.8 |
| CA_n40-n41 | n40 | 0.53 |
|  | n41 | 0.53 |
| CA_n40-n78 | n40 | 0 |
|  | n78 | 0.5 |
| CA_n40-n79 | n40 | 0.3 |
|  | n79 | 0.8 |
| CA_n41-n50 | n41 | 0.3 |
|  | n50 | 0.4 |
| CA_n41-n66 | n41 | 0.86 |
|  |  | 1.37 |
|  | n66 | 0.5 |
| CA_n41-n71 | n41 | 0.3 |
|  | n71 | 0.6 |
| CA_n41-n771 | n41 | 0.3 |
|  | n77 | 0.8 |
| CA_n41-n781 | n41 | 0.3 |
|  | n78 | 0.8 |
| CA_n41-n79 | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n48-n66 | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n50-n78 | n50 | 02 |
|  | n78 | 02 |
|  | n50 | 0.53 |
|  | n78 | 0.53 |
| CA_n66-n70 | n66 | 0.5 |
|  | n70 | 0.5 |
| CA_n66-n71 | n66 | 0.3 |
|  | n71 | 0.3 |
| CA_n66-n77 | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n66-n78 | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n70-n71 | n70 | 0.3 |
|  | n71 | 0.6 |
| CA_n71-n77 | n71 | 0.5 |
|  | n77 | 0.8 |
| CA_n71-n78 | n71 | 0.5 |
|  | n78 | 0.8 |
| CA_n75-n78 | n78 | 0.8 |
| CA_n76-n78 | n78 | 0.8 |
| CA n77-n79 | n77 | 0.5 |
|  | n79 | 0.5 |
| CA_n78-n79 | n78 | 0.5 |
|  |  | 1.58 |
|  | n79 | 0.5 |
|  |  | 1.58 |
| CA_n78-n92 | n78 | 0.8 |
|  | n92 | 0.6 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

Table 6.2A.4.2.3-2: Void

Table 6.2A.4.2.3-3: Void

##### 6.2A.4.2.4	ΔTIB,c for Inter-band CA (three bands)

Table 6.2A.4.2.4-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n7 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n7 | 0.6 |
| CA_n1-n3-n8 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n8 | 0.3 |
| CA_n1-n3-n28 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |
| CA_n1-n3-n41 | n1 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.35 |
|  |  | 0.86 |
| CA_n1-n3-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n8-n78 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n8-n79 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n79 | 0.8 |
| CA_n1-n28-n78 | n1 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n77-n79 | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |
| CA_n1-n78-n79 | n1 | 0.3 |
|  | n78 | 0.8 |
|  |  | 1.57 |
|  | n79 | 0.5 |
|  |  | 1.57 |
| CA_n2-n66-n77 | n2 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n8-n78 | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n7-n28 | n1 | 0.5 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
| CA_n1-n7-n78 | n1 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n40-n78 | n1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |
| CA_n3-n5-n7 | n3 | 0.5 |
|  | n5 | 0.3 |
|  | n7 | 0.5 |
| CA_n3-n7-n28 | n3 | 0.5 |
|  | n7 | 0.5 |
|  | n28 | 0.3 |
| CA_n3-n7-n78 | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n5-n7 | n3 | 0.5 |
|  | n5 | 0.3 |
|  | n7 | 0.5 |
| CA_n3-n18-n41 | n3 | 0.5 |
|  | n18 | 0.3 |
|  | n41 | 0.31/0.82 |
| CA_n3-n28-n77 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n3-n28-n78 | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| CA_n3-n40-n41 | n3 | 0.5 |
|  | n40 | 0.5 |
|  | n41 | 0.51,3 |
|  |  | 0.82,3 |
| CA_n3-n41-n77 | n3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| CA_n3-n41-n78 | n3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n78 | 0.8 |
| CA_n3-n41-n79 | n3 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
| CA_n5-n7-n78 | n5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n5-n25-n66 | n5 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n5-n25-n77 | n5 | 0.6 |
|  | n25 | 0.6 |
|  | n77 | 0.8 |
| CA_n5-n25-n78 | n5 | 0.6 |
|  | n25 | 0.6 |
|  | n78 | 0.8 |
| CA_n5-n66-n77 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n5_n66-n78 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n7_n25-n66 | n7 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n7_n28-n78 | n7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| CA_n7_n66-n78 | n7 | 0.5 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n8-n39-n41 | n8 | 0.6 |
|  | n39 | 0.54 |
|  | n41 | 0.54 |
| CA_n8-n40-n41 | n8 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |
| CA_n8-n41-n79 | n8 | 0.6 |
|  | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n8-n78-n79 | n8 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.8 |
| CA_n13-n25-n66 | n13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n20-n28-n78 | n20 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| CA_n25-n29-n66 | n25 | 0.5 |
|  | n29 | 0 |
|  | n66 | 0.5 |
| CA_n25-n38-n78 | n25 | 0.5 |
|  | n38 | 0.4 |
|  | n78 | 0.8 |
| CA_n25-n41-n66 | n25 | 0.5 |
|  | n41 | 0.85 |
|  |  | 1.36 |
|  | n66 | 0.5 |
| CA_n25-n41-n71 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.6 |
| CA_n25-n41-n77 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.6 |
| CA_n25-n48-n66 | n25 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n25-n66-n71 | n25 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.6 |
| CA_n25-n66-n77 | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n25-n66-n78 | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n71-n77 | n25 | 0.6 |
|  | n71 | 0.6 |
|  | n77 | 0.8 |
| CA_n28-n77-n79 | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |
| CA_n28-n78-n79 | n28 | 0.5 |
|  | n78 | 0.8 |
|  |  | 1.57 |
|  | n79 | 0.5 |
|  |  | 1.57 |
| CA_n28-n40-n78 | n28 | 0.5 |
|  | n40 | 0.3 |
|  | n78 | 0.8 |
| CA_n28-n41-n77 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |
| CA_n28-n41-n78 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |
| CA_n29-n66-n70 | n29 | 0 |
|  | n66 | 0.5 |
|  | n70 | 0.5 |
| CA_n38-n66-n78 | n38 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.8 |
| CA_n39-n40-n41 | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |
| CA_n39-n40-n79 | n39 | 0.3 |
|  | n40 | 0 |
|  | n79 | 0.8 |
| CA_n39-n41-n79 | n39 | 0.3 |
|  | n41 | 0.34 |
|  | n79 | 0.84 |
| CA_n40-n41-n79 | n40 | 0.53 |
|  | n41 | 0.53 |
|  | n79 | 0.8 |
| CA_n41-n66-n71 | n41 | 0.81 |
|  |  | 1.32 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |
| CA_n41-n66-n77 | n41 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n41-n71-n77 | n41 | 0.3 |
|  | n71 | 0.5 |
|  | n77 | 0.8 |
| CA_n66-n70-n71 | n66 | 0.5 |
|  | n70 | 0.5 |
|  | n71 | 0.6 |
| CA_n66-n71-n77 | n66 | 0.6 |
|  | n71 | 0.6 |
|  | n77 | 0.8 |
| CA_n66-n71-n78 | n66 | 0.6 |
|  | n71 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. NOTE 4: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx between n39 and n41. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 7: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

##### 6.2A.4.2.5	ΔTIB,c for Inter-band CA (four bands)

Table 6.2A.4.2.5-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n7-n28 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
| CA_n1-n3-n7-n78 | n1 | 0.7 |
|  | n3 | 0.7 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |
| CA_n1-n3-n8-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n3-n28-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n5-n7-n78 | n3 | 0.6 |
|  | n5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n7-n28-n78 | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.6 |
| CA_n3-n28-n41-n77 | n3 | 1 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| CA_n3-n28-n41-n78 | n3 | 1 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
|  | n78 | 0.8 |
| CA_n7-n25-n66-n78 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n41-n66-n71 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |
| CA_n41-n66-n71-n77 | n41 | 0.31/0.82 |
|  | n66 | 1 |
|  | n71 | 0.5 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

## 6.2B	Transmitter power for NR-DC

### 6.2B.0	General

The requirements apply for inter-band NR-DC with one uplink serving cell configured per CG.

### 6.2B.1	UE maximum output power for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the transmitter power requirements in clause 6.2 apply per band.

For inter-band NR-DC with one uplink assigned per band, the UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, the maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2B.1.3-1.

Table 6.2B.1.3-1 UE Power Class for inter-band NR-DC

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_n2A-n5A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n2A-n48A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n2A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n3A-n41A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n3A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n5A-n48A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n5A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n28A-n41A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n28A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n41A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n41A-n78A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n48A-n66A |  |  |  |  | 23 | +2/-31 |  |  |
| DC_n66A-n77A |  |  |  |  | 23 | +2/-31 |  |  |
| NOTE 1: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high within each CG, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass is the maximum UE power specified without account of the tolerance NOTE 3: The maximum power requirement applies to the total transmitted power over both the MCG and SCG. NOTE 4: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

### 6.2B.2	UE maximum output power reduction for NR-DC

For inter-band NR-DC with one uplink assigned per band, the requirements in clause 6.2.2 apply for each uplink component carrier.

### 6.2B.3	UE additional maximum output power reduction for NR-DC

For inter-band NR-DC with one uplink assigned per band, the requirements in clause 6.2.3 apply for each uplink component carrier.

### 6.2B.4	Configured output power for NR-DC

#### 6.2B.4.1	Configured transmitted power level for NR-DC

The UE is allowed to set its configured maximum output power PCMAX,f,c,MCG and PCMAX,f,c,SCG for the respective MCG and SCG and its total configured maximum output power for NR-DC operation  PTotalNR-DC=10log10(PTotalNR-DC) with PTotalNR-DC as specified in clause 7.6.2 of [8]. The UE is configured with an inter-CG power sharing mode by NR-DC-PC-mode.The requirements apply for one uplink serving cell configured per CG and for asynchronous and synchronous NR-DC if not otherwise stated.

Unless otherwise stated, the configured maximum output power PCMAX,f,c,MCG (q) in physical-channel q for carrier f of serving cell c shall be set within the bounds if contained in the MCG,

PCMAX_L,f,c,MCG (q) ≤  PCMAX,f,c,MCG (q) ≤  PCMAX_H,f,c,MCG (q)

and the corresponding PCMAX_L,f,c,SCG (q) for a serving cell contained in the SCG,

PCMAX_L,f,c,SCG (q) ≤  PCMAX,f,c,SCG (q) ≤  PCMAX_H,f,c,SCG (q)

where PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,SCG and PCMAX_H,f,c,SCG are the limits for a serving cell c as specified in clause 6.2.4 modified as follows:

PCMAX_L,f,c,MCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc)}

PCMAX_H,f,c,MCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass – ΔPPowerClass}

for the MCG and

PCMAX_L,f,c,SCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc)}

PCMAX_H,f,c,SCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass – ΔPPowerClass}

for the SCG, where

-	PEMAX,NR-DC is the value given by the field p-UE-FR1 of the PhysicalCellGroupConfig IE for the MCG as defined in [7];

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2B.1.3-1 without taking into account the tolerance specified in the Table 6.2B.1.3-1;

-	∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2B.4.2 for NR-DC; ∆TIB,c = 0 dB otherwise;

-	∆TC,c = 1.5dB when NOTE 2 in Table 6.2B.1.3-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

-	∆MPRc for serving cell c is specified in clause 6.2.2.

-	ΔPPowerClass = 0 dB for a power class 3 capable UE.

For a UE provided with NR-DC-PC-mode = Semi-static-mode1,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass} + 0.3 dB

with PPowerClass set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG.

If for synchronous NR-DC operation a UE is provided NR-DC-PC-mode = Semi-static-mode2, the PTotalNR-DC is determined as above and

-	if at least one symbol of slot i1 of the MCG/SCG is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG/MCG, the UE determines a maximum power for the transmission on the SCG/MCG overlapping with slot i2 using the configured maximum power PCMAX,f,c,SCG or PCMAX,f,c,MCG for the SCG or MSG, respectively,

-	otherwise (i.e. an ongoing transmission overlapping with slot i2 of the SCG/MCG overlaps with only semi-static downlink symbols within slot i1 of the MCG/SCG), the UE determines a maximum power for the transmission on MCG or the SCG overlapping with slot i2 using the configured maximum power as specified in clause 6.2.4.

If a UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided with NR-DC-PC-mode = Dynamic,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass}

with PPowerClass set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG except

-	if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a maximum transmission power in slot i1 of the MCG or in slot i2 of the SCG using the configured maximum power as specified in clause 6.2.4.

If a UE indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG as specified in clause 7.6.2 of [8], and is provided with NR-DC-PC-mode = Dynamic,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass}

with PPowerClass set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG.

The measured total maximum output power PUMAX over both CGs measured over the transmission reference time duration is

PUMAX = 10 log10 (pUMAX,c,MCG + pUMAX,c,SCG),

where pUMAX,c,MSG and pUMAX,c,SCG denote the measured output power of serving cells c contained in the respective MSG and SCG expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

When a subframe p on the MSG overlap with a physical-channel q on the SCG, then for PUMAX evaluation, the subframe p on the MCG is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframe and physical-channel durations are used on the carriers. The PPowerClass shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| Transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different CG carriers | MCG subframe | MIN(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX{PCMAX_NR-DC_H(p,q), PCMAX_NR-DC_H(p,q+1), … , PCMAX_NR-DC_H(p,q+n)}

where PCMAX_NR-DC_H entries are the applicable upper limits for each overlapping scheduling unit pairs (p,q), (p, q+1), up to (p, q+n) for each applicable Teval duration, where q+n is the last physical-channel on the SCG overlapping with subframe p on the MCG, while PCMAX_L is computed as follows:

PCMAX_L = MIN{PCMAX_NR-DC_L(p,q), PCMAX_NR-DC_L(p,q+1), … , PCMAX_NR-DC_L(p,q+n)}

where PCMAX_NR-DC_L entries are the applicable lower limits for each overlapping scheduling unit pairs (p,q), (p, q+1) up to (p, q+n) for each applicable Teval duration, where q+n is the last physical-channel on the SCG overlapping with subframe p on the MCG.

For a UE provided with NR-DC-PC-mode = Semi-static-mode1 and configured with pNR,MCG + pNR,SCG ≤ PTotalNR-DC with pNR,MCG and pNR,SCG the values of the PNR for the respective MCG and SCG expressed in linear scale

PCMAX_NR-DC_L(p,q) = 10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)]

PCMAX_NR-DC_H(p,q) = 10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)]

with pCMAX_L,f,c,MCG, pCMAX_H,f,c,MCG, pCMAX_L,f,c,SCG, and pCMAX_H,f,c,SCG the values of the respective PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,MCG, and PCMAX_H,f,c,SCG expressed in linear scale, while the measured configured maximum power PUMAX  for each CG shall meet the requirements as specified in clause 6.2.4 but with bounds for PCMAX,f,c,MCG (p) and PCMAX,f,c,SCG  as specified in this clause.

If for synchronized NR-DC a UE is provided with NR-DC-PC-mode = Semi-static-mode2 and configured with pNR,MCG + pNR,SCG ≤ PTotalNR-DC with pNR,MCG and pNR,SCG the linear-scale values of the PNR for the respective MCG and SCG

PCMAX_NR-DC_L(p,q) = 10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)]

PCMAX_NR-DC_H(p,q) = 10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)]

while the measured configured maximum power PUMAX for each CG shall meet the requirements specified in Table 6.2.4-2 but with bounds for PCMAX,f,c,MCG (p) and PCMAX,f,c,SCG  as specified in this clause except

-	if an ongoing transmission overlapping with physical channel q of the SCG or subframe p of the MCG overlaps with only semi-static downlink symbols within the respective subframe p of the MCG or physical channel q of the SCG as indicated to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided,

then the measured configured maximum power PUMAX for the transmission subframe p on the MCG or physical channel q on the SCG shall meet the requirements as specified in clause 6.2.4 and with bounds for PCMAX,f,c,MCG (p) or PCMAX,f,c,SCG  as specified in clause 6.2.4.

For a UE provided with NR-DC-PC-mode = Dynamic,

PCMAX_NR-DC_L(p,q) = MIN{10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)], PTotalNR-DC}

PCMAX_NR-DC_H(p,q) = MIN{10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)], PTotalNR-DC}

while the measured configured maximum power PUMAX on the MCG shall meet the requirements as specified in clause 6.2.4-2 but with bounds for PCMAX,f,c,MCG (p) as specified in this clause, and the PUMAX on the SCG shall be within

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where

PCMAX_L,f,c = MIN{PCMAX_L,f,c,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

PCMAX_H,f,c = MIN{PCMAX_H,f,c,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

with limits as specified in Table 6.2.4-2 and pNR,MCG the value of the PNR for the MCG expressed in linear scale.

Table 6.2B.4.1.3-2: PCMAX tolerance for NR-DC

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs provided with NR-DC-PC-mode = Semi-static-mode1 or with NR-DC-PC-mode = Semi-static-mode2, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

#### 6.2B.4.2	ΔTIB,c for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the ΔTIB,c for the corresponding inter-band CA configuration as specified in clause 6.2A.4.2 applies.

## 6.2C	Transmitter power for SUL

### 6.2C.1	Configured transmitted power for SUL

When a UE is configured with both NR UL and NR SUL carriers in a serving cell with active transmission either on the UL carrier or SUL carrier, the configured transmit power requirements specified in clause 6.2.4 are applicable for the UL carrier and the SUL carrier, respectively.

### 6.2C.2	ΔTIB,c

For the UE which supports SUL band combination, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2C.2-1: ΔTIB,c due to SUL

| Band combination for SUL | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| SUL_n41-n80 | n41 | 0.31 |
|  |  | 0.82 |
|  | n80 | 0.5 |
| SUL_n41-n81 | n41 | 0.3 |
|  | n81 | 0.3 |
| SUL_n41-n83 | n41 | 0.3 |
|  | n83 | 0.3 |
| SUL_n41-n98 | n41 | 0.5 |
|  | n98 | 0.5 |
| SUL_n77-n80 | n77 | 0.8 |
|  | n80 | 0.6 |
| SUL_n77-n84 | n77 | 0.8 |
|  | n84 | 0.6 |
| SUL_n78-n80 | n78 | 0.8 |
|  | n80 | 0.6 |
| SUL_n78-n81 | n78 | 0.8 |
|  | n81 | 0.6 |
| SUL_n78-n82 | n78 | 0.8 |
|  | n82 | 0.6 |
| SUL_n78-n83 | n78 | 0.8 |
|  | n83 | 0.5 |
| SUL_n78-n84 | n78 | 0.8 |
|  | n84 | 0.3 |
| SUL_n78-n86 | n78 | 0.8 |
|  | n86 | 0.6 |
| SUL_n79-n83 | n79 | 0.8 |
|  | n83 | 0.5 |
| SUL_n79-n97 | n79 | 0.8 |
|  | n98 | 0.3 |
| SUL_n79-n98 | n79 | 0.8 |
|  | n98 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515 MHz. |  |  |

Table 6.2C.2-2: ΔTIB,c for SUL band combination (Three bands)

| Band combination for SUL | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1_SUL_n78-n80 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| CA_n1_SUL_n78-n84 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n84 | 0.6 |
| CA_n3_SUL_n78-n80 | n3 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| CA_n28_SUL_n41-n83 | n28 | 0.3 |
|  | n41 | 0.3 |
|  | n83 | 0.3 |
| CA_n28_SUL_n79-n83 | n28 | 0.5 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |
| CA_n41_SUL_n79-n80 | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
|  | n80 | 0.3 |
| CA_n41_SUL_n79-n83 | n41 | 0.3 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |
| CA_n79_SUL_n41-n80 | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
|  | n80 | 0.3 |
| CA_n79_SUL_n41-n83 | n41 | 0.3 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | +2/-3 |  |  |
| n2 |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  | 23 | +2/-31 |  |  |
| n25 |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 23 | +2/-3 |  |  |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 23 | +2/-3 |  |  |
| n40 |  |  |  |  | 23 | +2/-3 |  |  |
| n41 | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 23 | +2/-3 |  |  |
| n70 |  |  |  |  | 23 | +2/-3 |  |  |
| n71 |  |  |  |  | 23 | +2/-3 |  |  |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n80 |  |  |  |  | 23 | +2/-31 |  |  |
| n84 |  |  |  |  | 23 | +2/-3 |  |  |
| n95 |  |  |  |  | 23 | +2/-3 |  |  |
| n97 |  |  |  |  | 23 | +2/-3 |  |  |
| n98 |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

Table 6.2D.1-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. |  |  |  |

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1-1 shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ULFPTxModes set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.1 apply for the power class as indicated by the ue-PowerClass field in capability signalling.

### 6.2D.2	UE maximum output power reduction for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the allowed MPR for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.2 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

### 6.2D.3	UE additional maximum output power reduction for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2D.4 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.4 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

### 6.2D.4	Configured transmitted power for UL MIMO

For UE supporting UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in clause 6.2.4 shall apply to UE supporting UL MIMO, where

-	PPowerClass, ΔPPowerClass and ∆TC,c are specified in clause 6.2.4 unless otherwise stated;

-	MPRc is specified in clause 6.2D.2;

-	A-MPRc is specified in clause 6.2D.3.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2D.1-1 for the applicable operating band.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the tolerance is specified in Table 6.2D.4-1. The requirements shall be met with UL MIMO configurations specified in Table 6.2D.1-2.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the tolerance is specified in Table 6.2D.4-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.4-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.4 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

## 6.2E	Transmitter power for V2X

### 6.2E.1	UE maximum output power for V2X

#### 6.2E.1.1	General

When NR V2X UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the allowed NR V2X UE maximum output power is specified in Table 6.2.1-1 in clause 6.2.1.

When a UE is configured for NR V2X sidelink transmissions in NR Band n47, the V2X UE shall meet the following additional requirements for transmission within the frequency ranges 5855-5925 MHz:

-	The maximum mean power spectral density shall be restricted to 23 dBm/MHz EIRP when the network signaling value NS_33 is indicated.

where the network signaling values are specified in clause 6.2E.3.

NOTE:	The PSD limit in EIRP shall be converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11].

The requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1-2. For NR V2X UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms).

Table 6.2E.1.1-1: NR V2X UE Power Class for SL-MIMO

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n47 |  |  |  |  | 23 | +2/-3 |  |  |

If the UE transmits on one antenna connector at a time, the requirements in Table 6.2.1-1 shall apply to the active antenna connector.

#### 6.2E.1.2	UE maximum output power for V2X con-current operation

For the inter-band con-current NR V2X operation, the maximum output power is specified in Table 6.2E.1.1-1. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.2-1: NR V2X UE Power Class for inter-band con-current combination (two bands)

| NR V2X con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n39A-n47A |  |  |  |  | 23 | +2/-32 |  |  |
| V2X_n40A-n47A |  |  |  |  | 23 | +2/-32 |  |  |
| V2X_n41A-n47A |  |  |  |  | 23 | +2/-32 |  |  |
| V2X_n71A-n47A |  |  |  |  | 23 | +2/-34 |  |  |
| NOTE 1: The con-current band combinations are used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

### 6.2E.2	UE maximum output power reduction for V2X

#### 6.2E.2.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, this clause specifies the allowed Maximum Power Reduction (MPR) power for V2X physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

#### 6.2E.2.2	MPR for Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2E.2.2-1 for Power class 3 NR V2X UE.

Table 6.2E.2.2-1: Maximum Power Reduction (MPR) for power class 3 NR V2X

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 2.5 |
|  | 16QAM | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.0 |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

For PSFCH with single RB transmission for PC3 NR V2X UE, the required MPR is defined as follow

MPR_PSFCH =  3.5 dB

For contiguous and non-contiguous allocation for simultaneous PSFCH transmission for PC3 NR V2X UE, the required MPR are specified as follow

MPR_PSFCH = CEIL {MA_PSFCH, 0.5}

Where MA_PSFCH is defined as follows

MA_PSFCH =	7.5		; 0.00< NGap/NRB ≤ 0.55

=    12.0	; 0.55< NGap/NRB ≤1.0

Where,

NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart)

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed MPR for the maximum output power for NR V2X physical channels on S-SSB transmission shall be specified in Table 6.2E.2.2-2.

Table 6.2E.2.2-2: Maximum Power Reduction (MPR) for S-SSB transmission for power class 3 NR V2X

| Channel | MPRS-SSB (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| S-SSB | ≤ 6.0 | ≤ 2.5 |

For NR V2X UE with two transmit antenna connectors, the allowed Maximum Power Reduction (MPR) values specified in clause 6.2E.2 shall apply to the maximum output power specified in Table 6.2E.1.1-1. The requirements shall be met with SL MIMO configurations defined in Table 6.2D.1-2. For UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from each UE antenna connector.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2E.4 apply.

#### 6.2E.2.3	MPR for Power class 3 V2X con-current operation

For the inter-band con-current NR V2X operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The MPR requirements in clause 6.2.2 apply for NR Uu operation in licensed band, and the MPR requirements in in clause 6.2E.2 apply for NR sidelink operation in Band n47.

### 6.2E.3	UE additional maximum output power reduction for V2X

#### 6.2E.3.1	General

For the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in clause 6.2E.2 and A-MPR requirements specified in clause 6.2E.3.

Additional emission requirements can be indicated by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field [additionalSpectrumEmission]. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR V2X frequency band number of the applicable operating band, the IE field [freqBandIndicatorNR] and an associated value of [additionalSpectrumEmission] in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2E.2. Outer and inner allocation notation used in clause 6.2E.3 is defined in clause 6.2E.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2E.3.1-1: Additional Maximum Power Reduction (A-MPR) for PC3 NR V2X

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2E.1-1 | 10, 20, 30, 40 | Table 5.3.2-1 | N/A |
| NS_33 | 6.5E.2.2.1 (A-SEM) 6.5.3.2 (A-SE) | n47 | 10 | Clause 6.2E.3.2 |  |
| NS_52 | 6.5E.2.2.2 (A-SEM) | n47 | 40 | Clause 6.2E.3.3 |  |

Table 6.2E.3.1-2: Mapping of network signaling label

| NR V2X operating bands | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n38 | NS_01 |  |  |  |  |  |  |  |
| n47 | NS_01 | NS_33 | NS_52 |  |  |  |  |  |
| NOTE: [additionalSpectrumEmission] corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |  |

For UE with two transmit antenna connectors, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2E.1-1. The requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1-2. For UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2E.4 apply.

#### 6.2E.3.2	A-MPR for Power class 3 V2X UE by NS_33

When NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector* A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase  and A-MPRStep  are specified in Tables 6.2E.3-1, 6.2E.3-2 is allowed when network signalling value is provided. A-MPRBase is the default A-MPR value when no Gpost connector is declared. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in annex I in [11]. The A-MPRstep is the increase in A-MPR allowance to allow UE to meet tighter conducted A-SE and A-SEM requirements with higher value of declared Gpost connector.

For the contiguous PSSCH and PSCCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 6.2E.3.2-1: A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz)

| Carrier frequency [MHz] | Resources Blocks (LCRB) | Start Resource Block | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 | ≤ 24 |  |  |
|  |  | ≥ 1 and ≤ 3 | ≤19 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 26 and ≤ 38 | ≤6 |  |  |
|  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤11 |  |  |
|  |  | ≥ 15 and ≤ 19 | ≤9.5 |  |  |
|  |  | ≥ 20 and ≤ 25 | ≤8.0 |  |  |
|  | ≥ 10 and ≤ 30 | ≥ 4 and ≤7 | ≤ 16 |  |  |
|  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 20 and ≤ 30 | ≥ 0 and ≤ 3 | ≤ 22 |  |  |
|  | 25 and 30 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  | ≥ 22 and ≤ 27 | ≤ 8.0 |  |  |
|  | ≥ 25 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  | 40 and 45 | 0 and 1 | ≤ 19 |  |  |
|  |  | ≥ 2 and ≤ 5 | ≤ 16 |  |  |
|  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 50 | ≥ 0 | ≤ 16 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Table 6.2E.3.2-2: A-MPR for PSSCH/PSCCH by NS_33 (at other carrier frequency)

| Carrier frequency [MHz] | RB allocations | A-MPRBase (dB) |  |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 | 0.5 |
|  | Outer | ≤ 4.5 |  |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.1 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |  |

For the simultaneous PSFCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 6.2E.3.2-3: A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.15 | 0.15≤ NGap / NRB < 0.3 | 0.3≤ NGap / NRB ≤ 1 |  |
| 10 | 5860 | NRB =1 | 19.0 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910, 5920 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 14 | 7 | 18.5 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

For the S-SSB transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 6.2E.3.2-4: A-MPR for S-SSB transmission by NS_33

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 9 |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |

#### 6.2E.3.3	A-MPR for Power class 3 V2X UE by NS_52

When NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPR

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

For the contiguous PSSCH and PSCCH transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 6.2E.3.3-1: A-MPR for PSSCH/PSCCH by NS_52

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Region 1 | Region 2 | Region 3 |
| 5885 | QPSK | ≤ 13.5 | ≤ 8.0 | ≤ 5.5 |
|  | 16QAM |  | ≤ 8.0 | ≤ 5.5 |
|  | 64QAM |  | ≤ 8.5 | ≤ 5.5 |
|  | 256QAM |  | ≤ 8.5 | ≤ 6.0 |
| Note1: Void. |  |  |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Region1, Region2 and Region3 according to RB allocations:

Table 6.2E.3.3-1a: A-MPR Region definitions for PSSCH/PSCCH by NS_52

| Channel Bandwidth, MHz | Carrier frequency (MHz) | A-MPR parameters for region definitions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart or RBend | LCRB |  |
| 40 | 5885 | RBstart ≤ floor(NRB*0.2) or RBend ≥ NRB - floor(NRB*0.2) | LCRB ≤floor(NRB*0.2) | Region 1 |
|  |  | The RB allocation is in Region 2 allocation for all other allocations which are not a Region1 or Region3 allocation. |  | Region 2 |
|  |  | floor(NRB /3.5) ≤ RBstart ≤ NRB –floor(NRB /3.5) – LCRB | LCRB ≤ceil(NRB/3.5) | Region 3 |

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

For the simultaneous PSFCH transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 6.2E.3.3-2: A-MPR for simultaneous PSFCH by NS_52

| Channel Bandwidth [MHz] | Carrier frequency [MHz] | A-MPR (dB) |
| --- | --- | --- |
| 40 MHz | 5885 | 23.5 |

For the S-SSB transmission when NS_52 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 6.2E.3.2-3: A-MPR for S-SSB transmission by NS_52

| Carrier Frequency [MHz] | RBStart * 12*SCS [MHz] | A-MPR (dB) |
| --- | --- | --- |
| 5885 | ≤ 7 | ≤ 16 |
|  | > 7 and ≤ 12 | ≤ 10.5 |
|  | > 12 and ≤ 19 | ≤ 4.0 |
|  | > 19 and ≤ 25 | ≤ 10.5 |
|  | > 25 | ≤ 16 |

#### 6.2E.3.4	A-MPR for power class 3 V2X con-current operation

For the inter-band con-current NR V2X operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in clause 6.2.3 apply for NR Uu operation in licensed band, and the A-MPR requirements in in clause 6.2E.3 apply for NR sidelink operation in Band n47.

### 6.2E.4	Configured transmitted power for V2X

#### 6.2E.4.1	General

The NR V2X UE is allowed to set its configured maximum output power PCMAX,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c – TC,c,  (PPowerClass – ΔPPowerClass) –– MAX(MAX(MPRc , A-MPRc)+ ΔTIB,c + TC,c + ∆TRxSRS, P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c,  (PPowerClass– ΔPPowerClass),  PRegulatory,c }

where

-	PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	For the total transmitted power PCMAX,PSSCH/PSCCH, PCMAX,S-SSB and PCMAX,PSFCH, PEMAX,c is the value given by IE maxTxPower, defined by [TS 38.331], when the UE is not associated with a serving cell on the NR V2X carrier .

-	PPowerClass is the maximum UE power specified in Table 6.2.1-1 without taking into account the tolerance specified in the Table 6.2.1-1;

-	MPRc and A-MPRc for serving cell c are specified in clause 6.2E.2 and clause 6.2E.3 for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	TIB,c, TC,c, ∆TRxSRS, PPoweclass and P-MPRc are specified in clause 6.2.4

-	PRegulatory,c= 10 - Gpost connector dBm the V2X UE is within the protected zone [12] of CEN DSRC tolling system and operating in Band n47; PRegulatory,c= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in clause 6.2.4 shall be applied.

For NR V2X UE supporting SL MIMO, the transmitted power is configured per each UE.

For NR V2X UE with two transmit antenna connectors, the tolerance is specified in Table 6.2E.4.1-1. The requirements shall be met with SL MIMO configurations specified in Table 6.2D.1-2.

If the UE transmits on two antenna connectors at the same time, the tolerance is specified in Table 6.2E.4.1-1.

Table 6.2E.4.1-1: PCMAX,c tolerance schemes for MIMO

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

#### 6.2E.4.2	Configured transmitted power for V2X con-current operation

When a UE is configured for simultaneous NR V2X sidelink and NR uplink transmissions for inter-band con-current operation, the UE is allowed to set its configured maximum output power PCMAX,c,NR and PCMAX,c,V2X for the configured NR uplink carrier and the configured NR V2X carrier, respectively, and its total configured maximum output power PCMAX,c.

The configured maximum output power PCMAX c,NR(p) in slot p for the configured NR uplink carrier shall be set within the bounds:

PCMAX_L,c,NR (p) ≤  PCMAX,c,NR (p) ≤  PCMAX_H,c,NR (p)

where PCMAX_L,c,NR and PCMAX_H,c,NR are the limits for a serving cell c as specified in clause 6.2.4.

The configured maximum output power PCMAX c,V2X (q) in slot q for the configured NR V2X carrier shall be set within the bounds:

PCMAX,c,V2X (q) ≤  PCMAX_H,c,V2X (q)

where PCMAX_H,c,V2X is the limit as specified in clause 6.2E.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of NR uplink carrier and a slot q of NR V2X sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  PCMAX_L,c,NR (p)

PCMAX_H (p,q) = 10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,V2X (q)]

where pCMAX_H,c,V2X and pCMAX_H,c,NR are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,NR (p) expressed in linear scale.

The measured total maximum output power PUMAX over both the NR uplink and NR V2X carriers is

PUMAX = 10 log10 [pUMAX,c,NR + pUMAX,c,V2X],

where pUMAX,c,NR  denotes the measured output power of serving cell c for the configured NR uplink carrier, and pUMAX,c,V2X  denotes the measured output power for the configured NR V2X carrier expressed in linear scale.

When a UE is configured for synchronous V2X sidelink and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2E.4-1. PCMAX_L may be modified for any overlapping portion of slots (p, q) and (p +1, q+1).

## 6.2F	Transmitter power for shared spectrum channel access

### 6.2F.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of shared spectrum channel access carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2F.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

The UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1-2 when NS is signaled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_28 | 20, 40, 60, 80 | 5150 – 5350 | 10 |
|  |  |  | 5470 – 5725 |  |
|  | NS_29 | 20 | 5170 – 5330 | 10 |
|  |  |  | 5490 – 5730 |  |
|  |  | 40 | 5170 – 5330 | 7 |
|  |  |  | 5490 – 5730 |  |
|  |  | 60, 80 | 5170 – 5330 | 4 |
|  |  |  | 5490 – 5730 |  |
|  | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
|  | NS_31 | 20 | 5150 - 5230 | 10 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 40 | 5150 - 5230 | 7 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 60, 80 | 5150 - 5230 | 4 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 |  |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

### 6.2F.1A	UE maximum output power for CA

#### 6.2F.1A.1	UE maximum output power for inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in clause 6.2 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2F.1.3A-1.

Table 6.2F.1A.1-1 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46A-n48A |  |  |  |  | 23 | +2/-32 |  |  |

### 6.2F.2	UE maximum output power reduction

For UE maximum output power reduction, the general requirements of clause 6.2.2 do not apply but instead the UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations for power class 5 according to Table 6.2F.2-1 and Table 6.2F.2-2.

Table 6.2F.2-1 Maximum power reduction (MPR) for shared spectrum access UE power class 5

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |
|  | QPSK | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 2.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Table 6.2F.2-2 MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration |  |
| --- | --- | --- |
|  | A | B |
| 40 | 11 | 10, 01 |
| 60 | 111, 011, 110, 001, 010, 100 | None |
| 80 | 1111, 0111, 1110, 0110, 0001, 1000 | 1100, 0011, 0100, 0010 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2F.4 apply.

### 6.2F.2A	UE maximum output power reduction for CA

#### 6.2F.2A.1	UE maximum output power reduction for inter-band CA

For inter-band carrier aggregation with uplink assigned to two bands, the requirements in clause 6.2.2 apply for the NR uplink carrier and clause 6.2F.2 for the carrier operating with shared spectrum access.

### 6.2F.3	UE additional maximum output power reduction

#### 6.2F.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2F.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2F.2.

Table 6.2F.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2F.3.1-1A.

Table 6.2F.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (clause) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | n46, n96 | 20, 40, 60, 80 |  | N/A |
| NS_28 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.2 |
| NS_29 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3 |
| NS_30 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.4 |
| NS_31 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.5 |
| NS_53 |  | n96 | 20, 40, 60, 80 |  | 6.2F.3.6 |
| NS_54 |  | n96 | 20, 40, 60, 80 |  | 6.2F.3.7 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2F.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  |  |
| n96 | NS_01 | NS_53 | NS_54 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

#### 6.2F.3.2	A-MPR for NS_28

When "NS_28" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.2-1.

Table 6.2F.3.2-1: A-MPR for NS_28 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-ODFM | QPSK | ≤ 4.0 | ≤ 6.0 | See Table 6.2F.2-1 |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 |  |
|  | 64 QAM | ≤ 4.5 | ≤ 6.5 |  |
|  | 256 QAM | ≤ 5.5 | ≤ 6.5 |  |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 |  |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.5 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. |  |  |  |  |

#### 6.2F.3.3	A-MPR for NS_29

When "NS_29" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3-1.

Table 6.2F.3.3-1: A-MPR for NS_29 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz |  | 60 MHz, 80 MHz |  |
|  |  | Full/Partial | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ 2.0 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 2.5 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 4.0 | ≤ 4.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 3.5 | ≤ 4.5 | ≤ 4.0 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 6.5 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 6.5 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |

#### 6.2F.3.4	A-MPR for NS_30

When "NS_30" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.4-1.

Table 6.2F.3.4-1: A-MPR for NS_30 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 15.0 | ≤ 2.5 | ≤ 5.0 | See Table 6.2F.2-1 |
|  | 16 QAM | ≤ 9.0 | ≤ 15.5 | ≤ 3.0 | ≤ 5.0 |  |
|  | 64 QAM | ≤ 9.0 | ≤ 15.5 | ≤ 4.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 9.0 | ≤ 16.0 | ≤ 5.5 | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 14.0 | ≤ 4.0 | ≤ 6.0 |  |
|  | 16 QAM | ≤ 9.5 | ≤ 14.5 | ≤ 4.0 | ≤ 6.0 |  |
|  | 64 QAM | ≤ 9.5 | ≤ 15.0 | ≤ 5.5 | ≤ 6.5 |  |
|  | 256 QAM | ≤ 9.5 | ≤ 15.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 2 and NOTE 3. |  |  |  |  |  |  |

#### 6.2F.3.5	A-MPR for NS_31

When "NS_31" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.5-1.

Table 6.2F.3.5-1: A-MPR for NS_31 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ 4.0 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 4.0 | ≤ 6.5 |
|  | 64 QAM |  | ≤ 4.0 | ≤ 6.5 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.5 |
| CP-OFDM | QPSK |  | ≤ 5.5 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

#### 6.2F.3.6	A-MPR for NS_53

When "NS_53" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.6-1.

Table 6.2F.3.6-1: A-MPR for NS_53 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.0 | ≤ 7.0 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |

#### 6.2F.3.7	A-MPR for NS_54

When "NS_54" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.7-1.

Table 6.2F.3.7-1: A-MPR for NS_54 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 5.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. |  |  |  |  |

### 6.2F.3A	UE additional maximum output power reduction for CA

#### 6.2F.3A.1	UE additional maximum output power reduction for inter-band CA

For inter-band carrier aggregation with uplink assigned to two bands, the requirements in clause 6.2.3 apply for the NR uplink carrier and clause 6.2F.3 for the carrier operating with shared spectrum access.

### 6.2F.4	Configured transmitted power

The requirements for configured maximum output power in clause 6.2.4 apply.

## 6.3	Output power dynamics

### 6.3.1	Minimum output power

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value.

The minimum output power is defined as the mean power in at least one sub-frame 1 ms. The minimum output power shall not exceed the values specified in Table 6.3.1-1.

Table 6.3.1-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40 | 4.515 |
| 10 | -40 | 9.375 |
| 15 | -40 | 14.235 |
| 20 | -40 | 19.095 |
| 25 | -39 | 23.955 |
| 30 | -38.2 | 28.815 |
| 40 | -37 | 38.895 |
| 50 | -36 | 48.615 |
| 60 | -35.2 | 58.35 |
| 70 | -34.6 | 68.07 |
| 80 | -34 | 78.15 |
| 90 | -33.5 | 88.23 |
| 100 | -33 | 98.31 |

### 6.3.2	Transmit OFF power

Transmit OFF power is defined as the mean power in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports..

The transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1 ms) excluding any transient periods. The transmit OFF power shall not exceed the values specified in Table 6.3.2-1.

Table 6.3.2-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50 | 4.515 |
| 10 | -50 | 9.375 |
| 15 | -50 | 14.235 |
| 20 | -50 | 19.095 |
| 25 | -50 | 23.955 |
| 30 | -50 | 28.815 |
| 40 | -50 | 38.895 |
| 50 | -50 | 48.615 |
| 60 | -50 | 58.35 |
| 70 | -50 | 68.07 |
| 80 | -50 | 78.15 |
| 90 | -50 | 88.23 |
| 100 | -50 | 98.31 |

### 6.3.3	Transmit ON/OFF time mask

#### 6.3.3.1	General

The transmit power time mask defines the transient period(s) allowed

-	between transmit OFF power as defined in clause 6.3.2 and transmit ON power symbols (transmit ON/OFF)

-	between continuous ON-power transmissions with powerchange or RB hopping is applied. When a UE signals the transient period capability, the transient period value (tp) can be 2, 4, or 7 usec.  If no capability is signalled, the default transient period value of 10 usec applies.

In case of RB hopping, transition period is shared symmetrically when the transient period is 10usec (tpstart would then be equal to -5s in following figures). If the UE signals a transient period (tp) of 2, 4 or 7usec, the transient period start position is given by tpstart in Table 6.3.3.1-1.

Table 6.3.3.1-1 tpstart  values

| tp (s) | tpstart (s) |
| --- | --- |
| 2 | [-0.5] |
| 4 | [-1] |
| 7 | [-2] |
| NOTE 1: Negative values mean that the transient period starts before the symbol boundary |  |

Unless otherwise stated the requirements in clause 6.5 apply also in transient periods.

In the following clauses, following definitions apply:

-	A slot or long subslot transmission is a transmission with more than 2 symbols.

-	A short subslot transmission is a transmission with 1 or 2 symbols.

#### 6.3.3.2	General ON/OFF time mask

The general ON/OFF time mask defines the observation period between transmit OFF and ON power and between transmit ON and OFF power for each SCS. ON/OFF scenarios include: contiguous, and non-contiguous transmission, etc

The OFF power measurement period is defined in a duration of at least one slot excluding any transient periods. The ON power is defined as the mean power over one slot excluding any transient period.

Figure 6.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR1

#### 6.3.3.3	Transmit power time mask for slot and short or long subslot boundaries

The transmit power time mask for slot and a long subslot transmission boundaries defines the transient periods allowed between slot and long subslot PUSCH transmissions. For PUSCH-PUCCH and PUSCH-SRS transitions and multiplexing the time masks in clause 6.3.3.7 apply.

The transmit power time mask for slot or long subslot and short subslot transmission boundaries defines the transient periods allowed between slot or long subslot and short subslot transmissions. The time masks in clause 6.3.3.8 apply.

The transmit power time mask for short subslot transmission boundaries defines the transient periods allowed between short subslot transmissions. The time masks in clause 6.3.3.9 apply.

#### 6.3.3.4	PRACH time mask

The PRACH ON power is specified as the mean power over the PRACH measurement period excluding any transient periods as shown in Figure 6.3.3.4-1. The measurement period for different PRACH preamble format is specified in Table 6.3.3.4-1.

Table 6.3.3.4-1: PRACH ON power measurement period

| PRACH preamble format | SCS (kHz) | Measurement period (ms) |
| --- | --- | --- |
| 0 | 1.25 | 0.903125 |
| 1 | 1.25 | 2.284375 |
| 2 | 1.25 | 3.352604 |
| 3 | 5 | 0.903125 |
| A1 | 15 | 0.142708 |
|  | 30 | 0.071354 |
| A2 | 15 | 0.285417 |
|  | 30 | 0.142708 |
| A3 | 15 | 0.428125 |
|  | 30 | 0.2140625 |
| B1 | 15 | 0.140365 |
|  | 30 | 0.070182 |
| B4 | 15 | 0.83046875 |
|  | 30 | 0.415234375 |
| A1/B1 | 15 | 0.142708 ms for first six occasion 0.140365 ms for the last occasion |
|  | 30 | 0.071354 ms for first six occasion 0.070182 ms for the last occasion |
| A2/B2 | 15 | 0.285417 ms for first two occasion 0.278385 ms for the third occasion |
|  | 30 | 0.142708 ms for first two occasion 0.1391925 ms for the third occasion |
| A3/B3 | 15 | 0.428125 ms for the first occasion 0.41640625 ms for the second occasion |
|  | 30 | 0.2140625 ms for the first occasion 0.208203125 ms for the second occasion |
| C0 | 15 | 0.10703125 |
|  | 30 | 0.053515625 |
| C2 | 15 | 0.333333 |
|  | 30 | 0.166667 |
| NOTE: For PRACH on PRACH occasion start from the beginning of 0.5 ms or span the boundary of 0.5 ms of the subframe, the measurement period will plus 0.032552 μs |  |  |

Figure 6.3.3.4-1: PRACH ON/OFF time mask

#### 6.3.3.5	Void

#### 6.3.3.6	SRS time mask

For SRS transmission mapped to one OFDM symbol, the ON power is defined as the mean power over the symbol duration excluding any transient period; See Figure 6.3.3.6-1

Figure 6.3.3.6-1: Single SRS time mask for NR UL transmission

For SRS transmission mapped to two or more OFDM symbols the ON power is defined as the mean power for each symbol duration excluding any transient period. For consecutive SRS transmissions without power change, Figure 6.3.3.6-2 applies.

Figure 6.3.3.6-2: Consecutive SRS time mask for the case when no power change is required with SRS usage other than antenna switching.

When power change between consecutive SRS transmissions is required, then Figure 6.3.3.6-3 and Figure 6.3.3.6-4 apply.

Figure 6.3.3.6-3: Consecutive SRS time mask for the case when power change is required and when 15 kHz and 30 kHz SCS is used in FR1 with SRS usage other than antenna switching.

Figure 6.3.3.6-4: Consecutive SRS time mask for the case when power change is required and when 60 kHz SCS is used in FR1, when the transient period is 10 µs

Figure 6.3.3.6-5: FR1 Time mask for 15 kHz and 30 kHz SCS for the case when consecutive SRS switching usage is between antenna switching & other sets

where "other sets" belongs to a "usage set" other than the set for antenna switching. The usage sets for SRS switching are defined in clause 6.2.1 of TS 38.214 [10].

NOTE:	Guard period of one symbol is defined between two SRS resources of an SRS resource set for antenna switching for 15kHz, 30kHz and 60kHz SCS in Table 6.2.1.2-1 of TS 38.214 [10].

The above transient period applies to all the transmit CCs in CA with the CC sounding SRS. UE RF requirements do not apply during this transient period.

#### 6.3.3.7	PUSCH-PUCCH and PUSCH-SRS time masks

The PUCCH/PUSCH/SRS time mask defines the observation period between sounding reference symbol (SRS) and an adjacent PUSCH/PUCCH symbol and subsequent UL transmissions. The time masks apply for all types of frame structures and their allowed PUCCH/PUSCH/SRS transmissions unless otherwise stated.

Figure 6.3.3.7-1: PUCCH/PUSCH/SRS time mask when there is a transmission before or after or both before and after SRS, when sounded on the same antenna (Ant 'x')

Figure 6.3.3.7-2: PUCCH/PUSCH/SRS time mask when there is a transmission before or after or both before and after SRS, when sounded on a different antenna (Ant 'x' and Ant 'y' are different antenna ports)

Figure 6.3.3.7-3: Consecutive long subslot transmission and long subslot transmission time mask

This transient period of 15 µsec applies before and after SRS transmission to all the transmit CCs in CA with the CC sounding SRS. UE RF requirements do not apply during this transient period.

When there is no transmission preceding SRS transmission or succeeding SRS transmission, then the same time mask applies as shown in Figure 6.3.3.7-1.

#### 6.3.3.8	Transmit power time mask for consecutive slot or long subslot transmission and short subslot transmission boundaries

The transmit power time mask for consecutive slot or long subslot transmission and short slot transmission boundaries defines the transient periods allowed between such transmissions.

Figure 6.3.3.8-1: Consecutive slot or long subslot transmission and short subslot transmission time mask

#### 6.3.3.9	Transmit power time mask for consecutive short subslot  transmissions boundaries

The transmit power time mask for consecutive short subslot transmission boundaries defines the transient periods allowed between short subslot transmissions.

The transient period shall be equally shared as shown on Figure 6.3.3.9-2.

Figure 6.3.3.9-1: Void

Figure 6.3.3.9-2: Consecutive short subslot transmissions time mask

Figure 6.3.3.9-3: Consecutive short subslot (1 symbol gap) time mask for the case when transient period is required on both sides of the symbol and when 60 kHz SCS is used in FR1, where the transient period is 10 µs

### 6.3.4	Power control

#### 6.3.4.1	General

The requirements on power control accuracy apply under normal conditions.

#### 6.3.4.2	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1 ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error.

The minimum requirement specified in Table 6.3.4.2-1 apply in the power range bounded by the minimum output power as specified in clause 6.3.1 and the maximum output power as specified in clause 6.2.1.

Table 6.3.4.2-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

#### 6.3.4.3	Relative power tolerance

The relative power tolerance is the ability of the UE transmitter to set its output power in a target sub-frame (1 ms) relatively to the power of the most recently transmitted reference sub-frame (1 ms) if the transmission gap between these sub-frames is less than or equal to 20 ms.

The minimum requirements specified in Table 6.3.4.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in clause 6.3.1 and the measured PUMAX as defined in clause 6.2.1.

To account for RF Power amplifier mode changes, 2 exceptions are allowed for each of two test patterns. The test patterns are a monotonically increasing power sweep and a monotonically decreasing power sweep over a range bounded by the requirements of minimum power and maximum power specified in clauses 6.3.1 and 6.2.1, respectively. For those exceptions, the power tolerance limit is a maximum of ± 6.0 dB in Table 6.3.4.3-1.

Table 6.3.4.3-1: Relative power tolerance

| Power step P (Up or down) (dB) | All combinations of PUSCH and PUCCH transitions (dB) | All combinations of PUSCH/PUCCH and SRS transitions between sub-frames (dB) | PRACH (dB) |
| --- | --- | --- | --- |
| ΔP < 2 | ± 2.0 (NOTE) | ± 2.5 | ± 2.0 |
| 2 ≤ ΔP < 3 | ± 2.5 | ± 3.5 | ± 2.5 |
| 3 ≤ ΔP < 4 | ± 3.0 | ± 4.5 | ± 3.0 |
| 4 ≤ ΔP < 10 | ± 3.5 | ± 5.5 | ± 3.5 |
| 10 ≤ ΔP < 15 | ± 4.0 | ± 7.0 | ± 4.0 |
| 15 ≤ ΔP | ± 5.0 | ± 8.0 | ± 5.0 |
| NOTE: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods: for a power step ΔP ≤ 1 dB, the relative power tolerance for transmission is ± 0.7 dB. |  |  |  |

#### 6.3.4.4	Aggregate power tolerance

The aggregate power control tolerance is the ability of the UE transmitter to maintain its power in a sub-frame (1 ms) during non-contiguous transmissions within 21 ms in response to 0 dB commands with respect to the first UE transmission and all other power control parameters as specified in TS 38.213 [8] kept constant.

The minimum requirement specified in Table 6.3.4.4-1 apply in the power range bounded by the minimum output power as specified in clause 6.3.1 and the maximum output power as specified in clause 6.2.2.

Table 6.3.4.4-1: Aggregate power tolerance

| TPC command | UL channel | Aggregate power tolerance within 21 ms |
| --- | --- | --- |
| 0 dB | PUCCH | ± 2.5 dB |
| 0 dB | PUSCH | ± 3.5 dB |

## 6.3A	Output power dynamics for CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the output power dynamics requirements in clause 6.3 apply.

#### 6.3A.1.1	Minimum output power for intra-band contiguous CA

For intra-band contiguous carrier aggregation, the minimum output power is defined per carrier and the requirement is specified in clause 6.3.1.

#### 6.3A.1.2	Minimum output power for intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation, the minimum output power is defined per carrier and the requirement is specified in clause 6.3.1.

#### 6.3A.1.3	Minimum output power for inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the minimum output power is defined per carrier and the requirement is specified in clause 6.3.1.

#### 6.3A.1.4	Void

### 6.3A.2	Transmit OFF power for CA

#### 6.3A.2.1	Transmit OFF power for intra-band contiguous CA

For intra-band contiguous carrier aggregation, the transmit OFF power specified in clause 6.3.2.1 is applicable for each component carrier when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

#### 6.3A.2.2	Transmit OFF power for intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation, the transmit OFF power specified in clause 6.3.2.1 is applicable for each component carrier when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

#### 6.3A.2.3	Transmit OFF power for inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the transmit OFF power specified in clause 6.3.2.1 is applicable for each component carrier when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

#### 6.3A.2.4	Void

### 6.3A.3	Transmit ON/OFF time mask for CA

#### 6.3A.3.1	Transmit ON/OFF time mask for intra-band contiguous CA

For s intra-band contiguous carrier aggregation, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

#### 6.3A.3.2	Transmit ON/OFF time mask for intra-band non-contiguous CA

For s intra-band non-contiguous carrier aggregation, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

#### 6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

##### 6.3A.3.3.1	General

For inter-band carrier aggregation with uplink assigned to two NR bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

##### 6.3A.3.3.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors with 3dB boosting on the maximum output power for CA power class 3 when the capability uplinkTxSwitchingPowerBoosting is present and the IE uplinkTxSwitchingPowerBoosting is enabled, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

##### 6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability [TBD] is present, and is only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10], where NR UL carrier 1 is capable of two transmit antenna connectors and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1 and carrier 2.

The switching periods described in Figure 6.3A.3.3.3-1a and Figure 6.3A.3.3.3-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling [TBD] [7], and the length of uplink switching period X is less than the value indicated by UE capability [TBD].

Figure 6.3A.3.3.3-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.3-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

##### 6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability [TBD] is present, and is only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10], where NR UL carrier 1 in band A is capable of one transmit antenna connector, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink bands following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 and carrier 3 in band B.

The switching periods described in Figure 6.3A.3.3.4-1a and Figure 6.3A.3.3.4-1b are located in either NR band A or band B as indicated in RRC signalling [TBD] [7], and the length of uplink switching period X is less than the value indicated by UE capability [TBD].

Figure 6.3A.3.3.4-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A

Figure 6.3A.3.3.4-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

The requirements apply for the case of co-located and synchronized network deployment for the three uplink carriers.

The requirements apply for the case of single TAG for the three uplink carriers, i.e., the same uplink timing for the three carriers as described in clause 4.2 of TS 38.213 [8].

##### 6.3A.3.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability [TBD] is present, and is only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10], where NR UL carrier 1 in band A is capable of two transmit antenna connectors, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink bands following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1, carrier 2 and carrier 3 in the two bands.

The switching periods described in Figure 6.3A.3.3.5-1a and Figure 6.3A.3.3.5-1b are located in either NR band A or band B as indicated in RRC signalling [TBD] [7], and the length of uplink switching period X is less than the value indicated by UE capability [TBD].

Figure 6.3A.3.3.5-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A

Figure 6.3A.3.3.5-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

The requirements apply for the case of co-located and synchronized network deployment for the three uplink carriers.

The requirements apply for the case of single TAG for the three uplink carriers, i.e., the same uplink timing for the three carriers as described in clause 4.2 of TS 38.213 [8].

#### 6.3A.3.4	Void

### 6.3A.4	Power control for CA

#### 6.3A.4.1	Power control for intra-band contiguous CA

##### 6.3A.4.1.1	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms. The requirement can be tested by time aligning any transmission gaps on the component carriers.

6.3A.4.1.1.1		Minimum requirements

For intra-band contiguous carrier aggregation the absolute power control tolerance per component carrier is given in Table 6.3.4.2-1.

##### 6.3A.4.1.2	Relative power tolerance

6.3A.4.1.2.1	Minimum requirements

For intra-band contiguous carrier aggregation, the requirements apply when the power of the target and reference sub-frames on each component carrier exceed the minimum output power as defined in clause 6.3A.1 and the total power is limited by PUMAX as defined in clause 6.2A.4. The UE shall meet the following requirements for transmission on both assigned component carriers when the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame:

a)	for all possible combinations of PUSCH and PUCCH transitions per component carrier, the corresponding requirements given in Table 6.3.4.2-1;

b)	for SRS transitions on each component carrier, the requirements for combinations of PUSCH/PUCCH and SRS transitions given in Table 6.3.4.2-1 with simultaneous SRS of constant SRS bandwidth allocated in the target and reference subrames;

c)	for RACH on the primary component carrier, the requirements given in Table 6.3.4.2-1 for PRACH.

For a) and b) above, the power step P between the reference and target subframes shall be set by a TPC command and/or an uplink scheduling grant transmitted by means of an appropriate DCI Format.

##### 6.3A.4.1.3	Aggregate power control tolerance

For intra-band contiguous carrier aggregation, the aggregate power tolerance per component carrier is given in Table 6.3.4.2-1. The average power per PRB shall be aligned across both assigned carriers before the start of the test. The requirement can be tested with the transmission gaps time aligned between component carriers.

#### 6.3A.4.2	Power control for intra-band non-contiguous CA

#### 6.3A.4.2.1	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms. The requirement can be tested by time aligning any transmission gaps on the component carriers.

##### 6.3A.4.2.1.1		Minimum requirements

For intra-band non-contiguous carrier aggregation the absolute power control tolerance per component carrier is given in Table 6.3.4.2-1.

#### 6.3A.4.2.2	Relative power tolerance

##### 6.3A.4.2.2.1	Minimum requirements

For intra-band non-contiguous carrier aggregation, the requirements apply when the power of the target and reference sub-frames on each component carrier exceed the minimum output power as defined in subclause 6.3A.1 and the total power is limited by PUMAX as defined in subclause 6.2A.4. The UE shall meet the following requirements for transmission on both assigned component carriers when the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame:

a)	for all possible combinations of PUSCH and PUCCH transitions per component carrier, the corresponding requirements given in Table 6.3.4.2-1;

b)	for SRS transitions on each component carrier, the requirements for combinations of PUSCH/PUCCH and SRS transitions given in Table 6.3.4.2-1 with simultaneous SRS of constant SRS bandwidth allocated in the target and reference subrames;

c)	for RACH on the primary component carrier, the requirements given in Table 6.3.4.2-1 for PRACH.

For a) and b) above, the power step P between the reference and target subframes shall be set by a TPC command and/or an uplink scheduling grant transmitted by means of an appropriate DCI Format.

#### 6.3A.4.2.3	Aggregate power control tolerance

For intra-band non-contiguous carrier aggregation, the aggregate power tolerance per component carrier is given in Table 6.3.4.2-1. The average power per PRB shall be aligned across both assigned carriers before the start of the test. The requirement can be tested with the transmission gaps time aligned between component carriers.

#### 6.3A.4.3	Power control for inter-band CA

No requirements unique to CA operation are defined.

#### 6.3A.4.4	Void

## 6.3B	Output power dynamics for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the output power dynamics for the corresponding inter-band CA configuration as specified in clause 6.3A applies.

## 6.3C	Output power dynamics for SUL

### 6.3C.1	Void

### 6.3C.2	Void

### 6.3C.3	Transmit ON/OFF time mask for SUL

#### 6.3C.3.1	Time mask for switching between two uplink carriers

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability uplinkTxSwitchingPeriod is present, is only applicable for uplink switching mechanisms specified in clause 6.16 of TS 38.214 [10], where NR SUL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3C.3.1-1a and Figure 6.3C.3.1-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3C.3.1-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3C.3.1-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

#### 6.3C.3.2	Time mask for switching between two uplink carriers with two transmit antenna connectors

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability [TBD] is present, is only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10], where NR SUL carrier 1 is capable of two transmit antenna connectors and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1 and carrier 2.

The switching periods described in Figure 6.3C.3.2-1a and Figure 6.3C.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling [TBD] [7], and the length of uplink switching period X is less than the value indicated by UE capability [TBD].

Figure 6.3C.3.2-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3C.3.2-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

#### 6.3C.3.3	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability [TBD] is present, is only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10], where NR SUL carrier 1 in band A is capable of one transmit antenna connector and NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink bands following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 and carrier 3 in band B.

The switching periods described in Figure 6.3C.3.3-1a and Figure 6.3C.3.3-1b are located in either NR band A or band B as indicated in RRC signalling [TBD] [7], and the length of uplink switching period X is less than the value indicated by UE capability [TBD].

Figure 6.3C.3.3-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A

Figure 6.3C.3.3-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

The requirements apply for the case of co-located and synchronized network deployment for the three uplink carriers.

The requirements apply for the case of single TAG for the three uplink carriers, i.e., the same uplink timing for the three carriers as described in clause 4.2 of TS 38.213 [8].

#### 6.3C.3.4	Time mask for switching between two uplink bands with two transmit antenna connectors

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability [TBD] is present, is only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10], where NR SUL carrier 1 in band A is capable of two transmit antenna connectors and NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink bands following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1, carrier 2 and carrier 3 in the two bands.

The switching periods described in Figure 6.3C.3.4-1a and Figure 6.3C.3.4-1b are located in either NR band A or band B as indicated in RRC signalling [TBD] [7], and the length of uplink switching period X is less than the value indicated by UE capability [TBD].

Figure 6.3C.3.4-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A

Figure 6.3C.3.4-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

The requirements apply for the case of co-located and synchronized network deployment for the three uplink carriers.

The requirements apply for the case of single TAG for the three uplink carriers, i.e., the same uplink timing for the three carriers as described in clause 4.2 of TS 38.213 [8].

## 6.3D	Output power dynamics for UL MIMO

### 6.3D.1	Minimum output power for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the minimum output power is defined as the sum of the mean power from both transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3.1-1.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.3.1 apply.

### 6.3D.2	Transmit OFF power for UL MIMO

The transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3.2-1.

### 6.3D.3	Transmit ON/OFF time mask for UL MIMO

For UE supporting UL MIMO, the ON/OFF time mask requirements in clause 6.3.3 apply at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements specified in clause 6.3.3.1 apply to each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in clause 6.2D.1.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.3.3 apply.

### 6.3D.4	Power control for UL MIMO

For UE supporting UL MIMO, the power control tolerance applies to the sum of output powers from both transmit antenna connector.

The power control requirements specified in clause 6.3.4 apply to UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.3.4 apply.

## 6.3E	Output power dynamics for V2X

### 6.3E.1	Minimum output power for V2X

#### 6.3E.1.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the minimum output power is specified in Table 6.3E.1.1-1. The minimum output power is defined as the mean power in at least one sub-frame 1 ms.

Table 6.3E.1.1-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -30 | 9.375 |
| 20 | -30 | 19.095 |
| 30 | -28.2 | 28.815 |
| 40 | -27 | 38.895 |

For NR V2X UE with two transmit antenna connectors, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified for single carrier.

If the UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

#### 6.3E.1.2	Minimum output power for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.1 shall apply for the sidelink in Band n47.

### 6.3E.2	Transmit OFF power for V2X

#### 6.3E.2.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the requirements specified in clause 6.3.2 apply.

Table 6.3E.2.1-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -50 | 9.375 |
| 20 | -50 | 19.095 |
| 30 | -50 | 28.815 |
| 40 | -50 | 38.895 |

For NR V2X UE supporting SL MIMO, the transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3E.2.1-1 for single carrier. Transmit off power is defined as the mean power in at least one sub-frame 1 ms.

#### 6.3E.2.2	Transmit OFF power for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.2 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.2 shall apply for the sidelink in Band n47.

### 6.3E.3	Transmit ON/OFF time mask for V2X

#### 6.3E.3.1	General

For NR V2X UE, additional requirements on ON/OFF time masks for V2X physical channels and signals are specified in this clause.

#### 6.3E.3.2	General time mask

The General ON/OFF time mask defines the observation period between the Transmit OFF and ON power and between Transmit ON and OFF power for PSCCH, and PSSCH transmissions in a slot wherein the last symbol is punctured to create a guard period.

Figure 6.3E.3.2-1: General PSCCH/PSSCH time mask for NR V2X UE

#### 6.3E.3.3	S-SSB time mask

The S-PSS/S-SSS/PSBCH time mask for NR V2X UE defines the observation period between transmit OFF and ON S-PSS power and between transmit ON PSBCH and OFF power in a slot wherein the last symbol is punctured to create a guard period.

Figure 6.3E.3.3-1: S-SSB time mask for NR V2X UE

For NR V2X UE supporting SL MIMO, the ON/OFF time mask requirements apply at each transmit antenna connector.

For UE with two transmit antenna connectors, the general ON/OFF time mask requirements specified in subclause 6.3E.3 apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in subclause 6.2D.1.

If the UE transmits on one antenna connector at a time, the general ON/OFF time mask requirements apply to the active antenna connector.

#### 6.3E.3.4	Transmit ON/OFF time mask for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.3 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.3 shall apply for the sidelink in Band n47.

### 6.3E.4	Power control for V2X

#### 6.3E.4.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the following requirements are applied for NR V2X sidelink transmission.

For NR V2X UE supporting SL MIMO, the power control tolerance for single carrier shall apply to the sum of output power at each transmit antenna connector.

If the UE transmits on one antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

#### 6.3E.4.2	Absolute power tolerance

The requirements in clause 6.3.4.2 shall apply for NR V2X transmission.

#### 6.3E.4.3	Power control for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.4 shall apply for the sidelink in Band n47.

## 6.3F	Output power dynamics for shared spectrum channel access

### 6.3F.1	Minimum output power

The requirements for minimum output power in clause 6.3.1 apply.

### 6.3F.2	Transmit OFF power

The requirements for Transmit OFF power in clause 6.3.2 apply.

### 6.3F.3	Transmit ON/OFF time mask

#### 6.3F.3.1	General

The transmit power time mask defines the transient period(s) allowed between transmit OFF power as defined in clause 6.3F.2 and transmit ON power symbols (transmit ON/OFF).  The transmit power ON/OFF time mask specified in clause 6.3F.3.2 supercedes the ON/OFF masks specified in clause 6.3.3; however, between continuous ON-power transmissions the requirements in clause 6.3.3 apply. Unless otherwise stated the requirements in clause 6.5F apply also in transient periods.

#### 6.3F.3.2	General ON/OFF time mask

The general ON/OFF time mask defines the observation period between transmit OFF and ON power and between transmit ON and OFF power for each SCS as illustrated below in Figure 6.3F.3.2-1. ON/OFF scenarios include: contiguous, and non-contiguous transmission, etc.

The OFF power measurement period is defined in a duration of at least one slot excluding any transient periods. The ON power is defined as the mean power over the duration of at least one slot excluding any transient period and non-transmitted symbols.  The leading transient period starts 5us before the beginning of the first symbol of transmission and extends 10us into the transmission including the CP extension if applicable.  The trailing transient period starts 5us before the end of transmssion and extends 5us beyond the end of transmission.

CP-E

End of OFF power

5µs

5µs

Transient period

Transient period

Start of OFF power

Start of ON power

requirement

Start of transmission

End of transmission

End of ON power

requirement

* The OFF power requirements does not

apply for DTX and measurement gaps

10µs

5µs

Figure 6.3F.3.2-1: General ON/OFF time mask for shared spectrum channel access

### 6.3F.3A	General ON/OFF mask for CA

#### 6.3F.3A.1	General ON/OFF mask for inter-band CA

For inter-band carrier aggregation with uplink assigned to two bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for the NR uplink carrier while the general output power ON/OFF time mask specified in clause 6.3F.3 is applicable for the carrier operating with shared spectrum access. The OFF period as specified in clause 6.3.3.1 and clause 6.3F.3 shall only be applicable for each component carrier when all the component carriers are OFF.

### 6.3F.4	Power control

#### 6.3F.4.1	General

The requirements on power control accuracy apply under normal conditions.

#### 6.3F.4.2	Absolute power tolerance

The absolute power tolerance requirements of clause 6.3.4.2 apply at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 40 ms.

#### 6.3F.4.3	Relative power tolerance

The relative power tolerace requirements of clause 6.3.4.3 apply if the transmission gap between the target sub-frame and the reference sub-frame is less than or equal to 40 ms.

#### 6.3F.4.4	Aggregate power tolerance

The aggregate power tolerance requirements of clause 6.3.4.4 apply during non-contiguous transmissions within 41ms with respect to the first UE transmission.

## 6.4	Transmit signal quality

### 6.4.1	Frequency error

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

### 6.4.2	Transmit modulation quality

Transmit modulation quality defines the modulation quality for expected in-channel RF transmissions from the UE. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage

-	In-band emissions for the non-allocated RB

All the parameters defined in clause 6.4.2 are defined using the measurement methodology specified in Annex F.

In case the parameter 3300 or 3301 is reported from UE via the parameter txDirectCurrentLocation in UplinkTxDirectCurrentList IE (as defined in TS 38.331 [7]), carrier leakage measurement requirement in clause 6.4.2.2 and 6.4.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

#### 6.4.2.1	Error Vector Magnitude

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further equalised using the channel estimates subjected to the EVM equaliser spectrum flatness requirement specified in clause 6.4.2.4. For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and one slotfor PUCCH and PUSCH in the time domain. The EVM measurement interval is reduced by any symbols that contains an allowable power transient in the measurement interval, as defined in clause 6.3.3.

The RMS average of the basic EVM measurements over 10 subframes for the average EVM case, and over 60 subframes for the reference signal EVM case, for the different modulation schemes shall not exceed the values specified in Table 6.4.2.1-1 for the parameters defined in Table 6.4.2.1-2. For EVM evaluation purposes, all 13 PRACH preamble formats and all 5 PUCCH formats are considered to have the same EVM requirement as QPSK modulated..

Table 6.4.2.1-1: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

Table 6.4.2.1-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | Table 6.3.1-1 |
| UE Output Power for 256 QAM | dBm | Table 6.3.1-1 + 10 dB |
| Operating conditions |  | Normal conditions |

#### 6.4.2.1a	Error Vector Magnitude including symbols with transient period

In 6.4.2.1, EVM has been defined by excluding the symbols which have a transient period. In this section, measurement interval is defined for the symbols with a transient period to include these symbols in the RMS average EVM computation when the UE reports a transient period capability other than the default. Before calculating the EVM, the measured waveform is corrected for sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM. The symbols with transient period should not be used for equalization. Only CP-OFDM waveform is used for conformance testing.”

In the case of PUSCH or PUCCH transmissions when the mean power, modulation or RB allocation across slot or subslot boundaries is expected to change the EVM result over the symbols where the transient occurs is calculated according to Table 6.4.2.1a-1.

Table 6.4.2.1a-1: EVM definition for reported transient period

| Reported transient capability (us) | EVM definition | tpstart (µs) | SCS4 |
| --- | --- | --- | --- |
| 2 | EVMafter=max⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | [-0.5] | 15kHz or 30kHz5 |
| 4 | EVMafter=max⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | [-1] | 15kHz |
| 7 | EVMafter=min⁡(EVMl_tp,EVMh) EVMbefore=max⁡(EVMl,EVMh_tp) | [-2] | 15kHz |
| NOTE 1: EVMl , EVMh, EVMl_tp,and EVMh_tp are defined in Annex F NOTE 2: EVMafter is the EVM for a symbol right after a transition; EVMbefore is the EVM for a symbol right before a transition NOTE 3: tpstart denotes the start position of the EVM exclusion window as shown in Annex F.4 NOTE 4: SCS denotes the SCS that can be used in the conformance test NOTE 5: 30kHz shall be used in the conformance test unless the UE signals in supportedSubCarrierSpacingUL in FeatureSetPerCC that it only supports 15kHz in the corresponding band |  |  |  |

The RMS average of the basic EVM measurements over [108] subframes for the symbols where the transient occurs for the different modulation schemes shall not exceed the values  specified in Table 6.4.2.1a-2 for the parameters defined in Table 6.4.2.1a-3.  This requirement can be verified with 64 QAM and 256 QAM modulation.

Table 6.4.2.1a-2: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | [10] |
| 256 QAM | % | [8] |

Table 6.4.2.1a-3: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | Table 6.3.1-1 |
| UE Output Power for 256 QAM | dBm | Table 6.3.1-1 + 10 dB |
| Operating conditions |  | Normal conditions |

#### 6.4.2.2	Carrier leakage

Carrier leakage is an additive sinusoid waveform whose frequency is the same as the modulated waveform carrier frequency. The measurement interval is one slot in the time domain.

In the case that uplink sharing, the carrier leakage may have 7.5 kHz shift with the carrier frequency.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2-1.

Table 6.4.2.2-1: Requirements for Carrier Leakage

| Parameter | Relative Limit (dBc) |
| --- | --- |
| Output power > 10 dBm | -28 |
| 0 dBm ≤ Output power ≤ 10 dBm | -25 |
| -30 dBm ≤ Output power < 0 dBm | -20 |
| -40 dBm ≤ Output power < -30 dBm | -10 |

#### 6.4.2.3	In-band emissions

The in-band emission is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is defined over one slot in the time domain; however, the minimum requirement applies when the in-band emission measurement is averaged over 10 sub-frames. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one or more symbols, accordingly.

The average of the basic in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4.2.3-1.

Table 6.4.2.3-1: Requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see clause 5.3). NOTE 8: EVM is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: For almost contiguous allocations defined in clause 6.2.2, LCRB = NRB_alloc + NRB_gap with no in-gap emission requirement. |  |  |  |  |

#### 6.4.2.4	EVM equalizer spectrum flatness

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex F) must meet a spectral flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the allocated uplink block. The basic measurement interval is the same as for EVM.

The peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4.2.4-1 for normal conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 5 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 7 dB (see Figure 6.4.2.4-1).

The EVM equalizer spectral flatness shall not exceed the values specified in Table 6.4.2.4-2 for extreme conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 10 dB (see Figure 6.4.2.4-1).

Table 6.4.2.4-1: Requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple (dB) |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each NR frequency band specified in Table 5.2-1 |  |

Table 6.4.2.4-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| Frequency range | Maximum Ripple (dB) |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each NR frequency band specified in Table 5.2-1 |  |

f

FUL_High

FUL_High – 3(5) MHz

< 4(4) dBp-p

Range 1

Range 2

max(Range 1)-min(Range 2) < 5(6) dB

max(Range 2)-min(Range 1) < 7(10) dB

< 8(12) dBp-p

Figure 6.4.2.4-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement are within brackets).

##### 6.4.2.4.1	Requirements for Pi/2 BPSK modulation

These requirements apply if the IE powerBoostPi2BPSK is set to 1 for power class 3 capable UE operating in TDD bands n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40 % or less slots in radio frame are used for UL transmission. These requirements also apply if the IE [DMRSPi2BPSK] is set to 1 and UE indicates support for UE capability [	DMRS-pi2BPSK-supported]. Otherwise the requirements for EVM equalizer spectrum flatness defined in clause 6.4.2.4 apply.

The EVM equalizer coefficients across the allocated uplink block shall be modified to fit inside the mask specified in Table 6.4.2.4.1-1 for normal conditions, prior to the calculation of EVM. The limiting mask shall be placed to minimize the change in equalizer coefficients in a sum of squares sense.

Table 6.4.2.4.1-1: Mask for EVM equalizer coefficients for Pi/2 BPSK, normal conditions

| Frequency range | Parameter | Maximum ripple (dB) |
| --- | --- | --- |
| ¦FUL_Meas – Fcenter¦ ≤ X MHz (Range 1) | X1 | 6 (p-p) |
| ¦FUL_Meas – Fcenter¦ > X MHz (Range 2) | X2 | 14 (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: Fcenter refers to the center frequency of an allocated block of PRBs NOTE 3: X, in MHz, is equal to 25% of the bandwidth of the PRB allocation NOTE 4: See Figure 6.4.2.4.1-1 for description of X1, X2 |  |  |

Figure 6.4.2.4.1-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation.  .

For Pi/2 BPSK modulation the UE shall be allowed to employ spectral shaping and the shaping filter shall be restricted so that the impulse response of the shaping filter itself shall meet

│ãt(t,0)│ ≥ │ãt(t, τ)│    ∀τ ≠ 0

20log10│ãt(t,τ)│< -15 dB    1< τ < M - 1,

where│ãt(t, τ)│=IDFT{│ãt(t,f)│ejφ (t,f)},   f  is the frequency of the M allocated subcarriers , ã(t,f) and φ(t,f) are the amplitude and phase response.

0 dB reference is defined as 20log10│ãt(t,0)│.

## 6.4A	Transmit signal quality for CA

### 6.4A.1	Frequency error for CA

#### 6.4A.1.1	Frequency error for intra-band contiguous CA

For intra-band contiguous carrier aggregation the UE modulated carrier frequencies per band shall be accurate to within ±0.1 PPM observed over a period of one timeslot compared to the carrier frequency of primary component carrier received in the corresponding band

#### 6.4A.1.2	Frequency error for intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation the requirements in Section 6.4.1 applies per component carrier.

#### 6.4A.1.3	Frequency error for inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the frequency error requirements defined in clause 6.4.1 shall apply on each component carrier with all component carriers active.

#### 6.4A.1.4	Void

### 6.4A.2	Transmit modulation quality for CA

#### 6.4A.2.1	Transmit modulation quality for intra-band contiguous CA

For intra-band contiguous carrier aggregation, the requirements in clauses 6.4A.2.1.1, 6.4A.2.1.2 applies.

The requirements in this clause apply with PCC and SCC in the UL configured and activated: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [13]), carrier leakage measurement requirement in clause 6.4A.2.4.2 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

##### 6.4A.2.1.1	Error Vector Magnitude

For the intra-band contiguous carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers. Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-clause 6.4.2.1.

When a single component carrier is configured Table 6.4.2.1-1 apply.

The EVM requirements are according to Table 6.4A.2.1.1-1 if CA is configured in uplink with the parameters defined in Table 6.4.2.1-2.

Table 6.4A.2.1.1-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level per CC |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

##### 6.4A.2.1.2	In-band emissions

For intra-band contiguous carrier aggregation, the requirements in Table 6.4A.2.1.2-1 and 6.4A.2.1.2-2 apply within the aggregated transmission bandwidth configuration with both component carrier (s) active and one single contiguous PRB allocation of bandwidth  at the edge of the aggregated transmission bandwidth configuration.

The inband emission is defined as the interference falling into the non allocated resource blocks for all component carriers. The measurement method for the inband emissions in the component carrier with PRB allocation is specified in annex F.3. For a non allocated component carrier a spectral measurement is specified.

Table 6.4A.2.1.2-1: Minimum requirements for in-band emissions (allocated component carrier)

| Parameter | Unit | Limit |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Output power > 10 dBm | Image frequencies (NOTE 3) |
|  |  | -25 | 0≤ Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTE 4,5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. The limit is evaluated in each non-allocated RB. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in the non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE indicated in active uplink carrier(s). For band combinations with supporting additional DC location reporting for intra-band CA, the applicable LO leakage frequency depend on the txDirectCurrentLocation indicated in the additional reporting IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. Otherwise, the applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE. For only one uplink carrier is activated, the applicable LO leakage frequency follow definition in clause 6.4.2. NOTE 6: is the Transmission Bandwidth (see clause 5.3) not exceeding . NOTE 7: is the Transmission Bandwidth Configuration (see clause 5.3) of the component carrier with RBs allocated. NOTE 8: is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. |  |  |  |  |

Table 6.4A.2.1.2-2: Minimum requirements for in-band emissions (not allocated component carrier)

| Para-meter | Unit | Meas BW NOTE 1 | Limit |  |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB |  |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB | NOTE 2 |  |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of the contiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -28 | Output power > 10 dBm |  |  |  |
|  |  |  | -25 | 0≤ Output power ≤ 10 dBm |  |  |  |
| Carrier leakage | dBc | BW of 1 RB | NOTE 3 |  |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -28 |  | Output power > 10 dBm |  |  |
|  |  |  | -25 |  | 0 dBm ≤ Output power ≤ 10 dBm |  |  |
|  |  |  | -20 |  | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -10 |  | -40 dBm Output power < -30 dBm |  |  |
| NOTE1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. NOTE 2: Exceptions to the general limit is are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. NOTE 3: Two Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs NOTE 4: NOTES 1, 5, 6, 7, 8, 9 from Table 6.4A.2.3.1-1 apply for Table 6.4A.2.3.2-2 as well. NOTE 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. |  |  |  |  |  |  |  |

##### 6.4A.2.1.3	Carrier leakage

Carrier leakage is an additive sinusoid waveform that is confined within the aggrecated transmission bandwidth configuration. For intra-band contiguous CA, the carrier leakage requirement is defined with applicable frequencies dependent on parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE indicated in active uplink carrier(s). For band combinations with supporting additional DC location reporting for intra-band CA, the applicable LO leakage frequency depend on the txDirectCurrentLocation indicated in the additional reporting IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. Otherwise, the applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE. For only one uplink carrier is activated, the applicable LO leakage frequency follow definition in clause 6.4.2.The measurement interval is one slot in the time domain.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.4A.2.4.3-1.

Table 6.4A.2.1.3-1: Minimum requirements for Relative Carrier Leakage Power

| Parameters | Relative Limit (dBc) |
| --- | --- |
| Output power > 10 dBm | -28 |
| 0 dBm ≤ Output power ≤ 10 dBm | -25 |
| -30 dBm ≤ Output power < 0 dBm | -20 |
| -40 dBm ≤ Output power < -30 dBm | -10 |

#### 6.4A.2.2	Transmit modulation quality for intra-band non-contiguous CA

For intra-band non-coutiguous carrier aggregation, the requirements in subclauses 6.4A.2.2.1, 6.4A.2.2.2 applies.

The requirements in this clause apply with PCC and SCC in the UL configured and activated: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [13]), carrier leakage measurement requirement in subclause 6.4A.2.2.2 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

#### 6.4A.2.2.1	Error Vector Magnitude

For the intra-band non-contiguous carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers. Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.4.2.1.

When a single component carrier is configured Table 6.4.2.1-1 apply.

The EVM requirements are according to Table 6.4A.2.2.1-1 if CA is configured in uplink with the parameters defined in Table 6.4.2.1-2.

Table 6.4A.2.2.1-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level per CC |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

#### 6.4A.2.2.2	In-band emissions

For intra-band non-contiguous carrier aggregation the requirements for in-band emissions should be defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers according to Table 6.4.2.3-1.

For intra-band non-contiguous CA, the carrier leakage requirement is defined with applicable frequencies dependent on parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE indicated in active uplink carrier(s). For band combinations with supporting additional DC location reporting for intra-band CA, the applicable LO leakage frequency depend on the txDirectCurrentLocation indicated in the additional reporting IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency  but excluding any allocated RB. Otherwise, The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

For intra-band non-contiguous CA, the IQ image requirement is defined with the applicable frequencies based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs.

#### 6.4A.2.3	Transmit modulation quality for inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the transmit modulation quality requirements shall apply on each component carrier as defined in clause 6.4.2 with all component carriers active: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

#### 6.4A.2.4	Void

## 6.4B	Transmit signal quality for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the transmit signal quality for the corresponding inter-band CA configuration as specified in clause 6.4A applies.

## 6.4D	Transmit signal quality for UL MIMO

### 6.4D.1	Frequency error for UL MIMO

For UE(s) supporting UL MIMO, the basic measurement interval of modulated carrier frequency is 1 UL slot.  The mean value of basic measurements of UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

### 6.4D.2	Transmit modulation quality for UL MIMO

For UE supporting UL MIMO, the transmit modulation quality requirements are specified at each transmit antenna connector.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.4.2 apply.

The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage (caused by IQ offset)

-	In-band emissions for the non-allocated RB

In case the parameter 3300 or 3301 is reported from UE via the parameter txDirectCurrentLocation in UplinkTxDirectCurrentList IE (as defined in TS 38.331 [7]), carrier leakage measurement requirement in clause 6.4D.2.2 and 6.4D.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

#### 6.4D.2.1	Error Vector Magnitude

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in Table 6.4.2.1-1 which is defined in clause 6.4.2.1 apply at each transmit antenna connector. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.2	Carrier leakage

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Relative Carrier Leakage Power requirements specified in Table 6.4.2.2-1 which is defined in clause 6.4.2.2 apply at each transmit antenna connector. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.3	In-band emissions

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the In-band Emission requirements specified in Table 6.4.2.3-1 which is defined in clause 6.4.2.3 apply at each transmit antenna connector. The requirements shall be met with the uplink MIMO configurations specified in Table 6.2D.1-2

#### 6.4D.2.4	EVM equalizer spectrum flatness for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the EVM Equalizer Spectrum Flatness requirements specified in Table 6.4.2.4-1 and Table 6.4.2.4-2 which are defined in clause 6.4.2.4 apply at each transmit antenna connector. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2

### 6.4D.3	Time alignment error for UL MIMO

For UE(s) with multiple transmit antenna connectors supporting UL MIMO, this requirement applies to frame timing differences between transmissions on multiple transmit antenna connectors in the closed-loop spatial multiplexing scheme.

The time alignment error (TAE) is defined as the average frame timing difference between any two transmissions on different transmit antenna connectors.

For UE(s) with multiple transmit antenna connectors, the Time Alignment Error (TAE) shall not exceed 130 ns.

### 6.4D.4	Requirements for coherent UL MIMO

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between different antenna ports in any slot within the specified time window from the last transmitted SRS on the same antenna ports, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4-1 apply when the UL transmission power at each antenna port is larger than 0 dBm for SRS transmission and for the duration of time window.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |

The above requirements when all the following conditions are met within the specified time window:

-	UE is not signaled with a change in number of SRS ports in SRS-config, or a change in PUSCH-config

-	UE remains in DRX active time (UE does not enter DRX OFF time)

-	No measurement gap occurs

-	No instance of SRS transmission with the usage antenna switching occurs

-	Active BWP remains the same

-	EN-DC and CA configuration is not changed for the UE (UE is not configured or de-configured with PSCell or SCell(s))

## 6.4E	Transmit signal quality for V2X

### 6.4E.1	Frequency error for V2X

#### 6.4E.1.1	General

The UE modulated carrier frequency for NR V2X sidelink transmissions in Table 5.2E.1-1, shall be accurate to within ±0.1 PPM observed over a period of 1 ms compared to the absolute frequency in case of using GNSS synchronization source. The same requirements applied over a period of 1 ms compared to the carrier frequency received from the gNB or V2X synchronization reference UE in case of using the gNB or V2X synchronization reference UE sidelink synchronization signals.

For NR V2X UE supporting SL MIMO, the UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ±0.1 PPM observed over a period of 0.5 ms in case of using GNSS synchronization source. The same requirements apply over a period of 0.5 ms compared to the relative frequency in case of using the NR gNode B or V2X synchronization reference UE sidelink synchronization signals.

If the UE transmits on one antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

#### 6.4E.1.2	Frequency error for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.4.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.4E.1 shall apply for the sidelink in Band n47.

### 6.4E.2	Transmit modulation quality for V2X

#### 6.4E.2.1	General

The transmit modulation quality requirements in this clause apply to V2X sidelink transmissions.

For NR V2X UE supporting SL MIMO, the transmit modulation quality requirements for single carrier shall apply to each transmit antenna connector.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier apply to the active antenna connector.

#### 6.4E.2.2	Error Vector Magnitude for V2X

For V2X sidelink physical channels PSCCH and PSSCH, the Error Vector Magnitude requirements shall be as specified for PUSCH in Table 6.4.2.1-1 except pi/2-BPSK for NR V2X operating bands in Table 5.2E.1-1. When sidelink transmissions are shortened due to transmission gap of one symbol at the end of the slot, the EVM measurement interval is reduced by one symbol, accordingly.

#### 6.4E.2.3	Carrier leakage for V2X

Carrier leakage of NR V2X sidelink transmission, the requirements for NR PUSCH in Table 6.4.2.2-1 shall be applied.

#### 6.4E.2.4	In-band emissions for V2X

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the In-band emissions requirements shall be as specified for PUSCH in subclause 6.4.2.3 for the corresponding modulation and transmission bandwidth. When V2X transmissions are shortened due to transmission gap of one symbol at the end of the subframe, the In-band emissions measurement interval is reduced by one symbol, accordingly.

#### 6.4E.2.5	EVM equalizer spectrum flatness for V2X

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the EVM equalizer spectrum flatness requirements shall be as specified for PUSCH in clause 6.4.2.4 for the corresponding modulation and transmission bandwidth.

#### 6.4E.2.6	Transmit modulation quality for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.4.2 shall apply for the uplink in licensed band and the requirements specified in clause 6.4E.2 shall apply for the sidelink in Band n47.

## 6.4F	Transmit signal quality for shared spectrum channel access

### 6.4F.1	Frequency error

The requirements for frequency error in  clause 6.4.1 apply.

### 6.4F.2	Transmit modulation quality

Transmit modulation quality defines the modulation quality for expected in-channel RF transmissions from the UE. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage

-	In-band emissions for the non-allocated RB

All the parameters defined in clause 6.4.2 are defined using the measurement methodology specified in Annex F.

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [7]), carrier leakage measurement requirement in clause 6.4F.2.2 and 6.4F.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

#### 6.4F.2.1	Error Vector Magnitude

The requirements for Error Vector Magnitude in clause 6.4.2.1 apply.

#### 6.4F.2.2	Carrier leakage

The requirements for carrier leakage in clause 6.4.2.2 apply.

#### 6.4F.2.3	In-band emissions

The in-band emission is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is defined over one slot in the time domain; however, the minimum requirement applies when the in-band emission measurement is averaged over 10 sub-frames. When the PUSCH or PUCCH transmission slot is shortened, the in-band emissions measurement interval is reduced by one or more symbols, accordingly.  The requirement applies for power class 5 UE for 20 MHz channel bandwidth and 15 kHz SCS,

Instead of the general requirement in clause 6.4.2.3, the average of the basic in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4F.2.3-1.

Table 6.4F.2.3-1: Minimum requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-10-6∆RB-1,-57dBm180kHz-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. The requirement applies with for any non-allocated RB with RIV=1 and RIV=5 in the uplink scheduling grant where RIV is specified in [10]. NOTE 3: [The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated RBs, based on symmetry with respect to the reported carrier frequency location in txDirectCurrentLocation field of the UplinkTxDirectCurrentBWP, but excluding any allocated RBs. If txDirectCurrentLocation is not available or is reported with value 3300 or 3301, applicable frequencies shall be calculated with an assumed carrier frequency location at the center of the channel.] NOTE 4: [The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs with RIV=1 and RIV=5 in the uplink scheduling grant.] NOTE 5: [The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. The location of the DC frequency is given by txDirectCurrentLocation field of the UplinkTxDirectCurrentBWP. If txDirectCurrentLocation is not available or is reported with value 3300 or 3301, applicable frequencies shall be those that are enclosed in the RB(s) in the center of the channel.] NOTE 6: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 7: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180*2 kHz in allocated RBs, measured in dBm. |  |  |  |  |

#### 6.4F.2.4	EVM equalizer spectrum flatness

The requirements for EVM equalizer spectrum flatness in clause 6.4.2.4 apply.

### 6.4F.2A	Transmit modulation quality for CA

#### 6.4F.2A.1	Transmit modulation quality for inter-band CA

For inter-band carrier aggregation with uplink assigned to two bands, the transmit modulation quality requirements shall apply on the NR carrier as defined in clause 6.4.2 and on the carrier operating with shared spectrum access as defind in clause 6.4F.2.  The requirements apply with all component carrier active: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

## 6.5	Output RF spectrum emissions

### 6.5.1	Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 6.5.1-1.

Table 6.5.1-1: Occupied channel bandwidth

|  | NR channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Occupied channel bandwidth (MHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |

### 6.5.2	Out of band emission

#### 6.5.2.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2.2	Spectrum emission mask

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth.

Table 6.5.2.2-1: General NR spectrum emission mask

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

#### 6.5.2.3	Additional spectrum emission mask

##### 6.5.2.3.1	Requirements for network signalling value "NS_35"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_35" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.1-1.

Table 6.5.2.3.1-1: Additional requirements for "NS_35"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 |  |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| NOTE 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5.2.3.2	Requirements for network signalling value "NS_04"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The n41 SEM transition point from -13 dBm/MHz to -25 dBm/MHz is based on the emission bandwidth. The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.  Since the 26-dB emission bandwidth is implementation dependent, the maximum transmission bandwidths in MHz (NRB * SCS * 12 / 1,000,000) is used for the SEM.

Table 6.5.2.3.2-1: n41 maximum transmission bandwidth for CP-OFDM

| SCS (kHz) | Channel bandwidth (MHz) / Maximum transmission bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 | 9.36 | 14.22 | 19.08 | 28.80 | 38.88 | 48.6 | N/A | N/A | N/A | N/A | N/A |
| 30 | 8.64 | 13.68 | 18.36 | 28.08 | 38.16 | 47.88 | 58.32 | 68.04 | 78.12 | 88.02 | 98.28 |
| 60 | 7.92 | 12.96 | 17.28 | 27.36 | 36.72 | 46.8 | 56.88 | 66.96 | 77.04 | 87.12 | 97.20 |

Table 6.5.2.3.2-2: n41 maximum transmission bandwidth for DFT-S-OFDM

| SCS (kHz) | Channel bandwidth (MHz) / Maximum transmission bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 | 9.00 | 13.50 | 18.00 | 28.80 | 38.88 | 48.60 | N/A | N/A | N/A | N/A | N/A |
| 30 | 8.64 | 12.96 | 18.00 | 27.00 | 36.00 | 46.08 | 58.32 | 64.80 | 77.76 | 87.48 | 97.20 |
| 60 | 7.20 | 12.96 | 17.28 | 25.92 | 36.00 | 46.08 | 54.00 | 64.80 | 72.00 | 86.40 | 97.20 |

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.2-3.

Table 6.5.2.3.2-3: n41 SEM with "NS_04"

| ΔfOOB MHz |  | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| ± 0 - 1 | -10 | -10 | -10 | -10 | -10 |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  |  | -10 |  |  |  |  | 1 MHz |
| ± 1 - 5 |  | -10 |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X |  | -13 |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) |  | -25 |  |  |  |  |  |  |  |  |  |  |
| NOTE: X is defined in Table 6.5.2.3.2-1 for CP-OFDM and 6.5.2.3.2-2 for DFT-S-OFDM |  |  |  |  |  |  |  |  |  |  |  |  |

##### 6.5.2.3.3	Requirements for network signalling value "NS_03" and "NS_21"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_03" or "NS_21", is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3-1.

Table 6.5.2.3.3-1: Additional requirements for "NS_03" and "NS_21"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |
| 0-1 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | 1 % of channel BW |
| 1-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 | -25 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 15-20 |  |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 20-25 |  |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 25-30 |  |  |  |  | -25 | -13 | -13 | 1 MHz |
| 30-35 |  |  |  |  |  | -25 | -13 | 1 MHz |
| 35-40 |  |  |  |  |  |  | -13 | 1 MHz |
| 40-45 |  |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Table 6.5.2.3.3-2: Void

##### 6.5.2.3.4	Requirements for network signalling value "NS_06" or “NS_07”

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_06" or "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.4-1.

Table 6.5.2.3.4-1: Additional requirements for "NS_06" or "NS_07"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 |  |
| ± 0 – 0.1 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |  |
| ± 10 – 15 |  | -25 |  |  |
| ± 15 – 20 |  |  | -25 |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5.2.3.5	Void

##### 6.5.2.3.6	Void

##### 6.5.2.3.7	Void

##### 6.5.2.3.8	Requirements for network signalled value "NS_27"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.8-1.

Table 6.5.2.3.8-1: Additional requirements for "NS_27"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 40 |  |
| ± 0 - 1 | -13 |  |  |  |  | 1 % channel bandwidth |
| ± 1 - X | -13 |  |  |  |  | 1 MHz |
| < – X or > X | -25 |  |  |  |  |  |
| NOTE 1: X is occupied channel bandwidth as defined in Table 6.5.1-1. NOTE 2: The requirements apply only at the frequency range from 3540 MHz to 3710 MHz. |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2.4	Adjacent channel leakage ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5.2.4.1	NR ACLR

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1-2.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth (MHz) | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 11 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB1 | 31 dB | 31 dB | 30 dB |
| NOTE 1: Applicable for power class 1 UE operating in Band n14. |  |  |  |  |

##### 6.5.2.4.2	UTRA ACLR

UTRA adjacent channel leakage power ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRAACLR is specified for the first adjacent UTRA channel (UTRAACLR1) which center frequency is ± 2.5 MHz from NR channel edge and for the 2nd adjacent UTRA channel (UTRAACLR2) which center frequency is ± 7.5 MHz from NR channel edge.

The UTRA channel power is measured with a RRC filter with roll-off factor = 0.22 and bandwidth of 3.84 MHz. The assigned NR channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than – 50 dBm then the UTRAACLR1 and UTRAACLR2 shall be higher than the value specified in Table 6.5.2.4.2-1.

UTRAACLR is not applicable to the power class 3 UE operating in Band n12, n14, n17, and n30.

UTRAACLR is not applicable to the power class 1 UE operating in Band n14.

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

### 6.5.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [9] and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

#### 6.5.3.1	General spurious emissions

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.1-2 apply for all transmitter band configurations (NRB) and channel bandwidths.

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel | BWChannel + 5 |

Table 6.5.3.1-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

#### 6.5.3.2	Spurious emissions for UE co-existence

This clause specifies the requirements for NR bands for coexistence with protected bands.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n3, n80 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n5, n89 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 18, 19, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 52, 53 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| n8, n81 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 50, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| n13 | E-UTRA Band 2, 4, 5,12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
| n14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 12, 15 |
| n18 | E-UTRA Band 1, 3, 11, 21, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| n20, n82 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| n24, n99 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 53, 65, 66, 70, 71, 73,74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n28, n83 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 72, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| n30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 66, 70, 71, 85, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| n34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| n39, n98 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 74, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 15, 26, 33 |
| n40, n97 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8 |
| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n71, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| n50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 65, 66, 67, 68 | FDL_low | - | FDL_high | -50 | 1 |  |
| n51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| n53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| n65 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 43 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| n66, n86 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n47, n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| n74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 41, 53, 65, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n95 | E-UTRA Band 1, 3 , 5, 8, 28, 39, 40, 41, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: 15 kHz SCS is assumed when RB is mentioned in the note when channel bandwidth is less than or equal to 50 MHz, lowest SCS is assumed when channel bandwidth is larger than 50 MHz. The transmission bandwidth in terms of RB position and range is not limited to 15 kHz SCS and shall scale with SCS accordingly. NOTE 4: Void NOTE 5: For non-synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Void NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 9: Void NOTE 10: Void NOTE 11: Void NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: Void NOTE 14: Void NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 16: Void NOTE 17: Void NOTE 18: Void NOTE 19: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: Void NOTE 21: This requirement is applicable for any channel bandwidths up to 20MHz within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths up to 20 MHz. For channel bandwidth within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. . For carriers overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: Void NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for channel bandwidths up to 20 MHz within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: Void NOTE 29: Void NOTE 30: Void NOTE 31: Void NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth up to 20MHz and confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718-728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 35: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: Void NOTE 37: Void NOTE 38: Void NOTE 39: Void NOTE 40: Void NOTE 41: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 42: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: This requirement is applicable for NR channel bandwidth allocated within 1920-1980 MHz. |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5.3.2-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

#### 6.5.3.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

##### 6.5.3.3.1	Requirement for network signalling value "NS_04"

When "NS 04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.1-1: Additional requirements for "NS_04"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 MHz |  |
| 2495 ≤ f < 2496 | -13 | 1 % of Channel BW |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

##### 6.5.3.3.2	Requirement for network signalling value "NS_17"

When "NS_17" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-1: Additional requirements for "NS_17"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10 |  |  |
| 470 ≤ f ≤ 710 | -26.2 | 6 MHz | 1 |
| NOTE 1: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |

##### 6.5.3.3.3	Requirement for network signalling value "NS_18"

When "NS_18" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3. 3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3-1: Additional requirements for "NS_18"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20, 30 |  |  |
| 692-698 | -26.2 | 6 MHz |  |

##### 6.5.3.3.4	Requirement for network signalling value "NS_05"

When "NS_05" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.4-1: Additional requirements for "NS_05"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 |  |  |
| 1884.5 f 1915.7 | -41 | 300 kHz |  |

##### 6.5.3.3.5	Requirement for network signalling value "NS_43"

When "NS 43" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.5-1: Additional requirement for "NS_43"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15 |  |
| 860 ≤ f ≤ 890 | -40 | 1 MHz |
| NOTE 1: Applicable for 5 MHz and 15 MHz channel BW confined between 900 MHz and 915 MHz and for 10 MHz channel BW confined between 905 MHz and 915 MHz |  |  |

##### 6.5.3.3.6	Requirement for network signalling value "NS_37"

When "NS_37" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.6-1: Additional requirement for "NS_37"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15,20 |  |
| 1475.9 ≤ f ≤ 1510.9 | -35 | 1 MHz |

##### 6.5.3.3.7	Requirement for network signalling value "NS_38"

When "NS_38" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.7-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.7-1: Additional requirements for NR channels assigned within 1430-1452MHz for "NS_38"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20 |  |
| 1400 ≤ f ≤ 1427 | -32 | 27 MHz |
| NOTE 1: This requirement shall be verified with UE transmission power of 15 dBm. |  |  |

##### 6.5.3.3.8	Requirement for network signalling value "NS_39"

When "NS_39" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.8-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.8-1: Additional requirements for "NS_39"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20 |  |
| 1475 ≤ f ≤ 1488 | -28 | 1 MHz |

##### 6.5.3.3.9	Requirement for network signalling value "NS_40"

When "NS_40" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.9-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.9-1: Additional requirements for NR channels assigned within 1427-1432MHz for "NS_40"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 |  |
| 1400 ≤ f ≤ 1427 | -32 | 27 MHz |
| NOTE 1: This requirement shall be verified with UE transmission power of 15 dBm. |  |  |

##### 6.5.3.3.10	Requirement for network signalling value "NS_41"

When "NS_41" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.10-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.10-1: Additional requirements for NR channels assigned within 1432-1517 MHz for "NS_41"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 40, 50, 60 |  |
| 1400 ≤ f ≤ 1427 | -32 | 27 MHz |
| NOTE 1: This requirement shall be verified with UE transmission power of 15 dBm. |  |  |

##### 6.5.3.3.11	Requirement for network signalling value "NS_42"

When "NS_42" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.11-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.11-1: Additional requirements for NR channels assigned within 1432-1517 MHz for "NS_42"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 40, 50, 60 MHz |  |
| 1518 ≤ f ≤ 1520 | -0.8 | 1 MHz |
| 1520 < f ≤ 1559 | -30 | 1 MHz |

##### 6.5.3.3.12	Requirement for network signalling value "NS_21"

When "NS_21" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.12-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.12-1: Additional requirements for "NS_21"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10 |  |
| 2200 ≤ f < 2288 | -40 | 1 MHz |
| 2288 ≤ f < 2292 | -37 | 1 MHz |
| 2292 ≤ f < 2296 | -31 | 1 MHz |
| 2296 ≤ f < 2300 | -25 | 1 MHz |
| 2320 ≤ f < 2324 | -25 | 1 MHz |
| 2324 ≤ f < 2328 | -31 | 1 MHz |
| 2328 ≤ f < 2332 | -37 | 1 MHz |
| 2332 ≤ f ≤ 2395 | -40 | 1 MHz |

##### 6.5.3.3.13	Requirement for network signalling value "NS_24"

When "NS 24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.13-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.13-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 2010 ≤ f ≤ 2025 | -50 | 1 MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

##### 6.5.3.3.14	Requirement for network signalling value "NS_27"

When "NS 27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.14-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.14-1: Additional requirements for "NS_27"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 40 |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

##### 6.5.3.3.15	Requirement for network signalling value "NS_47"

When "NS_47" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.15-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.15-1: Additional requirements for NR channels assigned within 2545 - 2575 MHz for "NS_47"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 30 |  |
| 2530 ≤ f ≤ 2535 | -25 | 1 MHz |
| 2505 ≤ f ≤ 2530 | -30 | 1 MHz |

##### 6.5.3.3.16	Requirement for network signalling value "NS_50"

When "NS_50" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.16-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.16-1: Additional requirements for "NS_50"

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 1805 | - | 1855 | -40 | 1 | 1 |
| Frequency range | 1855 | - | 1880 | -15.5 | 5 | 1, 2, 3 |
| NOTE 1: This requirement is applicable for carriers with aggregated channel bandwidths confined in 1885-1920 MHz for 25MHz and 30MHz channel BWs and confined in 1880-1920 MHz for 40MHz channel BW. NOTE 2: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.5.3.3.17	Requirement for network signalled value "NS_12"

When "NS_12" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.17-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.17-1: Additional requirements NS_12

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

##### 6.5.3.3.18	Requirement for network signalled value "NS_13"

When "NS_13" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.18-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.18-1: Additional requirements NS_13

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 817 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

##### 6.5.3.3.19	Requirement for network signalled value "NS_14"

When "NS_13" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.19-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.19-1: Additional requirements NS_14

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 817 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

##### 6.5.3.3.20	Requirement for network signalled value "NS_15"

When "NS_13" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.20-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.20-1: Additional requirements NS_15

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

##### 6.5.3.3.21	Requirement for network signalled value "NS_45"

When "NS_45" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.21-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.21-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 MHz | 10 MHz |  |
| 0.009 < f ≤ 2473.5 | -25 | -25 | 1 MHz |
| 2473.5 < f ≤ 2477.5 | -25 | -13 | 1 MHz |
| 2477.5 < f ≤ 2478.5 | -13 | -13 | 1 MHz |
| 2478.5< f ≤ 2483.5 | -10 | -10 | 1 MHz |
| 2495 ≤ f < 2496 | -13 | -13 | 1% of Channel Bandwidth |
| 2496 ≤ f < 2501 | -13 | -13 | 1 MHz |
| 2501 < f ≤ 2505 | -25 | -13 | 1 MHz |
| 2505 ≤ f ≤ 5th harmonic of the upper frequency edge of the UL operating band | -25 | -25 | 1 MHz |

##### 6.5.3.3.22	Requirement for network signalled values "NS_48" and "NS_51"

When "NS_48" or "NS_51" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.22-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.22-1: Additional requirements for "NS_48"

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 – NR band n34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1900 | - | 1915 | -15.5 | 5 | 1 |
| Frequency range | 1915 | - | 1920 | +1.6 | 5 | 1 |
| NOTE 1: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.5.3.3.23	Requirement for network signalled value "NS_49"

When "NS_49" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.23-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.23-1: Additional requirements for "NS_49"

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 - NR band n34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1880 | - | 1895 | -40 | 1 |  |
| Frequency range | 1895 |  | 1915 | -15.5 | 5 | 1 |
| Frequency range | 1915 | - | 1920 | 1.6 | 5 | 1 |
| NOTE 1: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.5.3.3.24	Requirement for network signalled value "NS_44"

When "NS_44" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.24-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.24-1: Additional requirements for "NS_44"

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2620 | - | 2645 | -15.5 | 5 | 1, 2 |
| Frequency range | 2645 | - | 2690 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for carriers confined in 2570-2615 MHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.5.3.3.25	Requirement for network signalled value "NS_46"

When "NS_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.25-1: Additional requirements for “NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for all carriers confined in 2500-2570 MHz. Sepcial restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.5.3.3.26	Requirement for network signalled value "NS_07"

When "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.26-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.26-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 769 ≤ f ≤ 775 | -57 | 6.25 kHz |
| NOTE: The emissions measurement shall be sufficiently power averaged to ensure standard standard deviation < 0.5 dB. |  |  |

##### 6.5.3.3.27 	Requirement for network signalled value “NS_52”

When "NS_52" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.27-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.27-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit in EIRP1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | EIRP of discrete emissions of less than 700 Hz bandwidth, averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | EIRP and averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement is converted to conducted requirement depending on the supported antenna gain Gant declared by the UE manufacturer. If not declared by the UE manufacturer, a 0 dBi antenna gain shall be used. |  |  |  |

### 6.5.4	Transmit intermodulation

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

UE transmit intermodulation is defined by the ratio of the mean power of the wanted signal to the mean power of the intermodulation product when an interfering CW signal is added at a level below the wanted signal at each transmitter antenna port with the other antenna port(s) if any terminated. Both the wanted signal power and the intermodulation product power are measured through NR rectangular filter with measurement bandwidth shown in Table 6.5.4-1.

The requirement of transmit intermodulation is specified in Table 6.5.4-1.

Table 6.5.4-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel center | BWChannel | 2*BWChannel |
| Interference CW signal level | -40 dBc |  |
| Intermodulation product | < -29 dBc | < -35 dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCS's for the channel BW as defined in Table 6.5.2.4.1-1 |  |
| Measurement offset from channel center | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |

## 6.5A	Output RF spectrum emissions for CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the output RF spectrum emissions requirements in clause 6.5 apply.

### 6.5A.1	Occupied bandwidth for CA

#### 6.5A.1.1	Void

#### 6.5A.1.1a	Occupied bandwidth for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated power of the transmitted spectrum. The occupied bandwidth shall be less than the aggregated channel bandwidth defined in clause 5.3A.3.

#### 6.5A.1.2	Occupied bandwidth for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation, the OBW requirement is met when the ratio of the transmitted power in all sub-blocks of the uplink CA configuration to the total integrated power of the transmitted spectrum is greater than 99%.

#### 6.5A.1.3	Occupied bandwidth for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the occupied bandwidth is defined per component carrier. Occupied bandwidth is the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on assigned channel bandwidth on the component carrier. The occupied bandwidth shall be less than the channel bandwidth specified in Table 6.5.1-1.

### 6.5A.2	Out of band emission for CA

#### 6.5A.2.1	General

This clause contains requirements for out of band emissions for UE configured of carrier aggregation.

#### 6.5A.2.2	Spectrum emission mask

##### 6.5A.2.2.1	Spectrum emission mask for intra-band contiguous C

For intra-band contiguous carrier aggregation the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the aggregated channel bandwidth. For intra-band contiguous carrier aggregation, the power of any UE emission shall not exceed the levels specified in Table 6.5A.2.2.1-1 for the specified channel bandwidth.

Table 6.5A.2.2.1-1: General NR CA spectrum emission mask

| ΔfOOB (MHz) | Spectrum emission limit(dBm) | MBW(MHz) |
| --- | --- | --- |
| ± 0 - 1 | -13 | Min(0.01*BWchannel_CA, 0.4) |
| ± 1 - 5 | -10 | 1MHz |
| ± 5 – BWchannel_CA | -13 | 1MHz |
| ±BWchannel_CA- BWchannel_CA+5 | -25 | 1MHz |

##### 6.5A.2.2.2	Spectrum emission mask for intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation the spectrum emission mask requirement is defined as a composite spectrum emissions mask. Composite spectrum emission mask applies to frequencies up to ΔfOOB starting from the edges of the sub-blocks. Composite spectrum emission mask is defined as follows

a)	Composite spectrum emission mask is a combination of individual sub-block spectrum emissions masks

b)	In case the sub-block consist of one component carrier the sub-lock general spectrum emission mask is defined in subclause 6.5.2.1

c)	If for some frequency sub-block spectrum emission masks overlap then spectrum emission mask allowing higher power spectral density applies for that frequency

d)	If for some frequency a sub-block spectrum emission mask overlaps with the sub-block bandwidth of another sub-block, then the emission mask does not apply for that frequency.

For the signalling is absent for dualPA-Architecture IE, if carrier leakage or I/Q image lands inside the gap spectrum between 2 UL CCs when UL CCs are synchronized with frequencies in the gap, exception to the SEM requirement applies.

##### 6.5A.2.2.3	Spectrum emission mask for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the spectrum emission mask of the UE is defined per component carrier while both component carriers are active and the requirements are specified in clauses 6.5.2.1 and 6.5.2.2. If for some frequency spectrum emission masks of component carriers overlap then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency a component carrier spectrum emission mask overlaps with the channel bandwidth of another component carrier, then the emission mask does not apply for that frequency.

##### 6.5.A.2.2.4	Void

#### 6.5A.2.3	Additional spectrum emission mask for CA

##### 6.5A.2.3.1	Additional spectrum emission mask for intra-band contiguous CA

6.5A.2.3.1.1	Requirements for network signalled value "CA_NS_04"

When "CA_NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.2.3.1.1-1.

Table 6.5A.2.3.1.1-1: Additional requirements for "CA_NS_04"

| ΔfOOB MHz | BWChannel_CA (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | ≤50 | >50 |  |
| ± 0 – 1 | -10 |  | 2 % of BWChannel_CA |
|  |  | -10 | 1 MHz |
| ± 1 – 5 | -10 |  | 1 MHz |
| ± 5 – X | -13 |  |  |
| ± X - (BWChannel_CA + 5 MHz) | -25 |  |  |
| NOTE: X is aggregated bandwidth |  |  |  |

6.5A.2.3.1.2	Requirements for network signalled value "CA_NS_27"

When "CA_NS_27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.2A.2.3.2.1.-1.

Table 6.2A.2.3.2.1-1: Additional requirements for "CA_NS_27"

| Spectrum emission limit (dBm) / measurement bandwidth for each aggregated channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB MHz | Aggregated channel bandwidth of max 40 MHz | Measurement bandwidth |
| ± 0 – 1 | -13 | 1 % of X |
| ± 1 – X | -13 | 1 MHz |
| < – X or > X | -25 |  |
| NOTE 1: X is the aggregated channel bandwidth NOTE 2: The requirements apply only at the frequency range from 3540 MHz to 3710 MHz. |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5A.2.3.2	Additional spectrum emission mask for Intra-band non-contiguous CA

##### 6.5A.2.3.2.1 Minimum requirement (network signalled value “CA_NC_NS_04”)

For intra-band non-cotiguous CA_n41(2A), the additional SEM requirements in subclause 6.5.2.3.2 (indicated by NS_04) applies in each uplink CC.

##### 6.5A.2.3.3	Additional spectrum emission mask for Inter-band CA

#### 6.5A.2.4	Adjacent channel leakage ratio

##### 6.5A.2.4.1	NR ACLR

6.5A.2.4.1.1	NR ACLR for intra-band contiguous CA

For intra-band contiguous carrier aggregation the carrier aggregation the Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent aggregated channel bandwidth at nominal channel spacing. The assigned aggregated channel bandwidth power and adjacent aggregated channel bandwidth power are measured with rectangular filters with measurement bandwidths specified in Table 6.5A.2.4.1.1-1. If the measured adjacent channel power is greater than –50dBm then the NRACLR shall be higher than the value specified in Table 6.5A.2.4.1.1-1.

Table 6.5A.2.4.1.1-1: General requirements for intra-band contiguous CA ACLR

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 30 dB |
| CA Measurement bandwidth (NOTE 1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |
| Difference between ACLR MBW center and Fc,low | MBWshift= (MBWACLR_CA-MBWACLR,low)/2 |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1, respectively. |  |

6.5A.2.4.1.2	NR ACLR for intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation, CA Adjacent Channel Leakage power Ratio(CAACLR) is the ratio of the sum of the filtered mean power centred on each assigned channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing. In case the gap bandwidth Wgap between 2 uplink CCs is smaller than maximum of the 2 uplink channel bandwidths then no CAACLR requirement is set for the gap. Each assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1. If the measured adjacent channel power is greater than –50dBm then the ACLR shall be higher than the value specified in Table 6.5A.2.4.1.2-1.

Table 6.5A.2.4.1.2-1: General requirements for intra-band non-contiguous CA ACLR

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 30 dB |
| CA Measurement bandwidth for each sub block (NOTE 1) | MBWACLR |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel / - BWChannel |
| NOTE 1: MBWACLR is the single-channel ACLR measurement bandwidths specified in 6.5.2.4.1. |  |

When the signalling is absent for dualPA-Architecture IE, if carrier leakage or I/Q image lands inside the gap spectrum between 2 UL CCs when UL CCs are synchronized with frequencies in the gap , exception to the ACLR requirement with 3dB relaxation applies.

6.5A.2.4.1.3	NR ACLR for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the NR Adjacent Channel Leakage power Ratio (NRACLR) is defined per component carrier while both component carriers are active and the requirement is specified in clause 6.5.2.4.1.

6.5A.2.4.1.4	Void

##### 6.5A.2.4.2	UTRA ACLR

6.5A.2.4.2.1	Void

6.5A.2.4.2.2	Void

6.5A.2.4.2.3	UTRA ACLR for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is defined per component carrier while both component carrier are active and the requirement is specified in clause 6.5.2.4.2.

### 6.5A.3	Spurious emission for CA

#### 6.5A.3.1	General spurious emissions

For inter-band carrier aggregation with uplink assigned to two NR bands, the spurious emission requirement Table 6.5.3.1-2 apply for the frequency ranges that are more than FOOB as defined in Table 6.5.3.1-1 away from edges of the assigned channel bandwidth on a component carrier. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier then it does not apply.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands the requirements in Table 6.5.3.1-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5.3.1-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA spurious emission requirement.

For intra-band contiguous carrier aggregation the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB as specified in Table 6.5A.3.1-1 the spurious emission requirements in Table 6.5.3.1-2 are applicable.

Table 6.5A.3.1-1: Boundary between out of band and spurious emission domain for intra-band contiguous carrier aggregation

| Aggregated Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel_CA | BWChannel_CA + 5 |

For intra-band non-contiguous carrier aggregation transmission the spurious emission requirement is defined as a composite spurious emission requirement. Composite spurious emission requirement applies to frequency ranges that are more than FOOB away from the edges of each carrier in the gap and out of the gap. Composite spurious emission requirement is defined as follows

a)	Composite spurious emission requirement is a combination of individual sub-block spurious emission requirements

b)	In case the sub-block consist of one component carrier the sub-lock spurious emission requirement and FOOB are defined in subclause 6.5.3.1

c)	If for some frequency an individual sub-block spurious emission requirement overlaps with the general spectrum emission mask or the sub-block bandwidth of another sub-block then it does not apply

For the signalling is absent for dualPA-Architecture IE, if carrier leakage or I/Q image lands inside the gap spectrum between 2 UL CCs when UL CCs are synchronized with frequencies in the gap, exception to the general spurious requirement applies.

#### 6.5A.3.2	Spurious emissions for UE co-existence

##### 6.5A.3.2.1	Spurious emissions for UE co-existence for intra-band contiguous CA

This clause specifies the requirements for the specified intra-band contiguous carrier aggregation configurations for coexistence with protected bands, the requirements in Table 6.5A.3.2.1-1 apply.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.2.1-1: Requirements for uplink intra-band contiguous carrier aggregation

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
| CA_n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 6: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |

##### 6.5A.3.2.2	Spurious emissions for UE co-existence for intra-band non-contiguous CA

This clause specifies the requirements for the specified intra-band non-contiguous carrier aggregation configurations for coexistence with protected bands, the requirements in Table 6.5A.3.2.2-1 apply.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.2.2-1: Requirements for uplink intra-band non-contiguous carrier aggregation

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |

##### 6.5A.3.2.3	Spurious emissions for UE co-existence for Inter-band CA

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.3-1 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands the requirements in Table 6.5A.3.2.3-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5A.3.2.3-1 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.5A.3.2.3-1: Requirements for uplink inter-band carrier aggregation (two bands)

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4,6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n7 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 68, 72, 74, 75, 76 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 4. 7, 6 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 4. 7, 6 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n1-n8 | E-UTRA Band 20, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n40 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 4, 14 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 4, 7, 14 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 4, 7, 14 |
| CA_n1-n41 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4,6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n77 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n2-n5 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 28, 29, 30, 42, 48, 50, 51 66, 70, 71, 74, 85, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n48 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n2-n66 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n77 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n78 | E-UTRA Band 5, 7, 12, 13，26, 28, 41, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n3-n7 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 42, 52 NR-band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n3-n8 | E-UTRA Band 1, 11, 20, 21, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n18 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n3-n38 | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| CA_n3-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31 38, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 32, 42, 43, 50, 51, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n3-n40 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | UTRA Band 22, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n3-n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n25 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 50, 51, 53, 66, 70, 71,74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n5An48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 65, 66, 70, 71, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n5-n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 7, 2 |
| CA_n5-n79 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 | FDL_low | - | FDL_high |  |  |  |
|  | E-UTRA Band 41, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n7-n25 | E-UTRA Band 4, 5, 7, 12, 13, 14 17, 26, 27, 28, 29, 30, 42, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n28 | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band n1 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 66, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 6 |
| CA_n7-n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n8-n39 | E-UTRA Band 1, 34, 40, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n8-n40 | E-UTRA Bands 1, 20, 28, 31, 32, 33, 34, 38, 39, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 3, 7, 22, 41, 42, 43, 52 NR Bands n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n8-n41 | E-UTRA Band 1, 11, 12, 28, 34, 39, 40, 45, 50, 51, 65, 73,74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 42, 52 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n79 | E-UTRA Band 1, 8, 11, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 41, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n13-n25 | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2,14, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n13-n66 | Bands 2, 4, 5, 7, 12, 13, 17, 25, 26, 27, 29, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 24, 30, 46, 48, | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n77 | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n18-n41 | E-UTRA Band 1, 3, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n20-n28 | E-UTRA Band 1, 3, 7, 22, 28, 31, 32, 34, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n20-n78 | E-UTRA Band 1, 3, 7, 8, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n25-n41 | E-UTRA Band 4, 5, 12, 13 , 14, 17, 24, 26, 27, 28, 29, 30, 42, 48, 66, 70, 71,85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n25-n66 | E-UTRA Band 4, 5, 7, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 38, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 43, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n25-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n25-n77 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n25-n78 | E-UTRA Band 5, 7, 12, 13, 25, 26, 28, 41，66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n28-n40 | E-UTRA Band 3, 5, 7, 8, 20, 26, 27, 31, 34, 38, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n50 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 27, 31, 34, 38, 39, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 22, 42, 43, 48, 52, 65, 66, 73 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n77 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 10, 11 |
| CA_n34-n79 | E-UTRA Band 1, 3, 8, 11, 18, 19, 21, 28, 39, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n38-n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 19 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 19, |
| CA_n38-n78 | E-UTRA Band 1, 3, 5, 8, 20, 28, 34, 40, 65, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 8 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4, 8 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n40-n41 | E-UTRA Band 1, 3, 5, 8, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n40-n78 | UTRA Band 1, 3, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n40-n79 | E-UTRA Band 1, 3, 5, 8, 28, 34, 39, 41, 42, 65, | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n41-n50 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 20, 25, 26, 27, 28, 29, 30, 31, 34, 39, 40, 42, 43, 44, 48, 52, 65, 66, 67, 68, 70, 71, 73, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n41-n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n41-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n41-n77 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 44, 45, 50, 51, 53, 65, 66, 70, 71, 73, 74, 85, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n78 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n50-n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 25, 26, 27, 28, 29, 31, 33, 34, 38, 39, 40, 41, 44, 65, 66, 67, 68, 69, 72, 73, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n66-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n66-n78 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 29, 26, 28, 41, 66, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n71-n77 | E-UTRA Band 1, 3, 4, 5, 7, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 30, 34, 39, 40, 44, 45, 50, 51, 53, 65, 66, 71, 73, 74, 85, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n71-n78 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n77-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n78-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n78-n92 | E-UTRA Band 1, 3, 7, 8, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 17: Void. NOTE 18: This requirement is applicable for any channel bandwidths within the range 2500 – 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 – 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 19: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.3.2.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

##### 6.5A.3.2.4	Void

##### 6.5A.3.2.5	Void

##### 6.5A.3.2.6	Void

#### 6.5A.3.3	Additional spurious emissions for CA

##### 6.5A.3.3.1	Additional spurious emissions for intra-band contiguous  CA

6.5A.3.3.1.1		Requirement for network signalling value "CA_ NS_04"

When "CA_NS04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5A.3.3.1.1-1: Additional requirements for "CA_ NS_04"

| Frequency range (MHz) | BWChannel_CA (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 to 190 MHz |  |
| 2495 ≤ f < 2496 | -13 | Max(1 % of BWChannel_CA, 1 MHz) |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

6.5A.3.3.1.2		Requirement for network signalling value "CA_ NS_27"

When "CA_NS 27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5A.3.3.1.2-1: Additional requirements for "CA_ NS_27"

| Frequency range (MHz) | Spectrum emission limit (dBm) for aggregated channel bandwidth of max 40 MHz | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

6.5A.3.3.1.3		Requirement for network signalling value "CA_ NS_46"

When "CA_NS 46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1.3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5A.3.3.1.3-1: Additional requirements for “CA_NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for carriers confined in 2500-2570 MHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

### 6.5A.4	Transmit intermodulation for CA

##### 6.5A.4.2.1	Transmit intermodulation for intra-band contiguous CA

For intra-band contiguous carrier aggregation the requirement of transmitting intermodulation is specified in Table 6.5A.4.2.1-1.

Table 6.5A.4.2.1-1: Transmit Intermodulation

| CA bandwidth class(UL) | B and C |  |  |
| --- | --- | --- | --- |
| Interference Signal Frequency Offset | BWChannel_CA | 2*BWChannel_CA |  |
| Interference CW Signal Level | -40dBc |  |  |
| Intermodulation Product | -29dBc | -35dBc |  |
| Measurement bandwidth (NOTE1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |  |  |
| Measurement offset from channel center | BWChannel_CA and 2*BWChannel_CA |  | 2*BWChannel_CA and 4*BWChannel_CA |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1, respectively. |  |  |  |

##### 6.5A.4.2.2	Void

6.5A.4.2.3	Transmit intermodulation for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the transmit intermodulation requirement is specified in Table 6.5.4-1 which shall apply on each component carrier with both component carriers active.

## 6.5B	Output RF spectrum emissions for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the output RF spectrum emissions for the corresponding inter-band CA configuration as specified in clause 6.5A applies.

## 6.5D	Output RF spectrum emissions for UL MIMO

### 6.5D.1	Occupied bandwidth for UL MIMO

For UE supporting UL MIMO, the requirements for occupied bandwidth apply to  the sum of the powers from both UE transmit antenna connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth shall be less than the channel bandwidth specified in table 6.5.1-1. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.1 apply.

### 6.5D.2	Out of band emission for UL MIMO

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UEtransmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclasuse 6.5.2 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.2 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.2 apply.

### 6.5D.3	Spurious emission for UL MIMO

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in subclasuse 6.5.3 apply. The requirements shall be met with the UL MIMO configurations described in clause 6.2D.1.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.3 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.3 apply.

### 6.5D.4	Transmit intermodulation for UL MIMO

For UE supporting UL MIMO, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output powers from both UE transmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in clause 6.5.4 apply to each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in clause 6.2D.1.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.4 apply.

## 6.5E	Output RF spectrum emissions for V2X

### 6.5E.1	Occupied bandwidth for V2X

#### 6.5E.1.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.1 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in Table 6.5.1-1. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

#### 6.5E.1.2	Occupied bandwidth for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.5.1 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.1 shall apply for the sidelink in Band n47.

### 6.5E.2	Out of band emission for V2X

#### 6.5E.2.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5E.2 apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.

#### 6.5E.2.2	Spectrum emission mask

##### 6.5E.2.2.1	General

For NR V2X UE, the existing NR general spectrum emission mask in subclause 6.5.2.2 applies for all supporting NR V2X channel bandwidths. The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies greater than (ΔfOOB), the power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth for NR V2X operating bands in Table 5.2E.1-1.

##### 6.5E.2.2.2	Spectrum emission mask for V2X con-current operation

For the inter-band con-current NR V2X operation, the general/additional SEM requirements specified in clause 6.5.2 shall apply for the uplink in licensed band and the general/additional SEM requirements specified in clause 6.5E.2 shall apply for the sidelink in Band n47.

#### 6.5E.2.3	Additional Spectrum emission mask

##### 6.5E.2.3.1	Requirements for network signalled value "NS_33"

The additional spectrum mask in Table 6.5E.2.2.1-1 applies for NR V2X UE within 5 855 MHz to 5 950 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.2.1-1.

Table 6.5E.2.2.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for NR V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11].

##### 6.5E.2.3.2	Requirements for network signalled value "NS_52"

The additional spectrum mask in Table 6.5E.2.3.2-1 applies for NR V2X UE within 5 765 MHz to 6 005 MHz according to FCC regulation. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_52" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.2-1.

Table 6.5E.2.3.2-1: Additional spectrum mask requirements for 40MHz channel bandwidth (fc = 5885MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0-2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40-100 | -50 | 100kHz |

#### 6.5E.2.4	Adjacent channel leakage ratio

##### 6.5E.2.4.1	General

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

For NR V2X UE, the existing ACLR requirement for NR uplink transmission in clause 6.5.2.4 are applied for NR V2X UE for NR V2X operating bands in 5.2E.1-1.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

##### 6.5E.2.4.2	ACLR for V2X con-current operation

For the inter-band con-current NR V2X operation, the ACLR requirement specified in clause 6.5.2.4 shall apply for the uplink in licensed band and the ACLR requirement specified in clause 6.5E.2.4 shall apply for the sidelink in Band n47.

### 6.5E.3	Spurious emissions for V2X

#### 6.5E.3.1	General spurious emissions

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the general spurious emission requirements in clause 6.5.3.1 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

#### 6.5E.3.2	Spurious emissions for UE co-existence

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.3.2 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

#### 6.5E.3.3	Spurious emissions for UE co-existence for V2X con-current operation

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.1.1-1 apply for the corresponding inter-band con-current operation with transmission assigned to both uplink in licensed band and sidelink in Band n47.

Table 6.5E.3.3.1-1: Requirements for inter-band con-current V2X operation

| V2X | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| con-current operating band cofiguration | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n39A-n47A | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n40A-n47A | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 42, 44, 45, 68, 72 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
|  | E-UTRA Band 1, 3, 5, 8, 26, 28, 34, 39, 42, 44, 45, 65, 73 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n71A-n47A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

#### 6.5E.3.4	Additional spurious emissions requirements for V2X

##### 6.5E.3.4.1	General

This clause specifies additional spurious emission requirements for V2X operation

##### 6.5E.3.4.2	Requirements for network signalled value "NS_33"

Table 6.5.3.4.2-1: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5950 | -30 | 1 | 1 |
| Frequency range | 5815 |  | - | 5855 | -30 | 1 | 3 |
| NOTE 1: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy. |  |  |  |  |  |  |  |

When "NS_33" is configured from pre-configured radio parameters or the cell, and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any NR V2X UE emission shall fulfil either one of the two sets of conditions.

Table 6.5.3.4.2-2: Requirements for spurious emissions to protect CEN DSRC for V2X UE

|  | Maximum Transmission Power (dBm EIRP1) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP1) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |
| NOTE 1: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. |  |  |

##### 6.5E.3.4.3	Void

### 6.5E.4	Transmit intermodulation

#### 6.5E.4.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.4 apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

#### 6.5E.4.2	Transmit intermodulation for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.5.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.4 shall apply for the sidelink in Band n47.

## 6.5F	Output RF spectrum emissions

### 6.5F.1	Occupied bandwidth

The requirements for occupied bandwidth in clause 6.5.1 apply for the specified NR-U channel bandwidths in Table 5.3.5-1.

### 6.5F.2	Out of band emission

#### 6.5F.2.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5F.2.2	Spectrum emission mask for operation with shared spectrum channel access

Instead of the general spectrum emission mask requirement in clause 6.5.2.2, when operating with shared spectrum channel access the relative power of any UE emission shall not exceed the levels specified in Table 6.5F.2.2-1 for the specified channel bandwidth or -30 dBm/MHz whichever is the greatest. The spectrum emission mask for operation with shared spectrum channel access is defined relative to the maximum power density in a 1 MHz measurement bandwidth within the channel bandwidth.

The spectrum emission mask for operation with shared spectrum channel access applies to frequencies (ΔfOOB) starting from the  edge of the assigned channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

Table 6.5F.2.2-1: Spectrum emission mask for operation with shared spectrum channel access

| Spectrum emission limit (dBr) / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz |  | 20 MHz |  | 40 MHz |  | 60 MHz | 80 MHz | Measurement bandwidth (MBW) |
| ± 0-1 | -20 ∆fOOB |  |  |  |  |  |  |  | [100kHz]3 |
| ± 1-5 | NOTE 1 | NOTE 1 |  | NOTE 1 |  | NOTE 1 |  | NOTE 1 | 1 MHz |
| ± 5-10 | NOTE 2 |  |  |  |  |  |  |  |  |
| ± 10-20 | -40 | NOTE 2 |  |  |  |  |  |  |  |
| ± 20-30 |  | -40 |  | NOTE 2 |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  | NOTE 2 |  |  |  |
| ± 40-50 |  |  |  | -40 |  |  |  | NOTE 2 |  |
| ± 50-60 |  |  |  |  |  |  |  |  |  |
| ± 60-70 |  |  |  |  |  | -40 |  |  |  |
| ± 70-80 |  |  |  |  |  |  |  |  |  |
| ± 80-100 |  |  |  |  |  |  |  | -40 |  |
| NOTE 1: Given as: -20-8A ∆fOOB-1 where A= Channel Bandwidth2-1 NOTE 2: Given as: -16-12B ∆fOOB where B= Channel Bandwidth2 NOTE 3: The measured value shall be scaled by a factor equal to the ratio of the reference bandwidth (1 MHz) to the measurement bandwidth before the emission limit (dBr) is applied. NOTE 4: The carrier leakage exceptions from Table 6.4F.2.3-1 apply and carrier leakage contribution shall be removed prior to setting the 0dBr level of the mask, the reported carrier frequency location in txDirectCurrentLocation field of the UplinkTxDirectCurrentBWP can be used to cancel the carrier leakage contribution. If txDirectCurrentLocation is not available or is reported with value 3300 or 3301, a carrier frequency location at the center of the channel shall be assumed. |  |  |  |  |  |  |  |  |  |

For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2.

##### 6.5F.2.2.1	Spectrum emission mask for non-transmitted channels

In the case of non-transmitted 20 MHz channel(s) on the edges of an assigned channel bandwidth the spectrum emission mask for operation with shared spectrum channel access, specified in Table 6.5F.2.2-1, is applied by using the total bandwidth of the remaining transmitted channels. The spectrum emission mask for non-transmitted channels is floored at -28dBr.

The relative power of any UE emission shall not exceed the most stringent levels given by the spectrum emission mask for operation with shared spectrum channel access with full channel bandwidth and the spectrum emission mask for non-transmitted channels with the channel bandwidth of the transmitted channels in the case of non-transmitted channels at the edge of an assigned channel bandwidth.

An exception to the spectrum emission mask for non-transmitted channels allows a single [2] MHz bandwidth to extend to [-28] dBc relative to total transmit power, or [-20] dBm, whichever is the greatest.

#### 6.5F.2.3	Additional spectrum emission mask

There are no additional spectrum emission mask requirements in this version of the specification.

#### 6.5F.2.4	Adjacent channel leakage ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5F.2.4.1	Shared spectrum channel access ACLR

The Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing.  The assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

Instead of the general ACLR requirement in clause 6.5.2.4, if the measured adjacent channel power is greater than –47 dBm then the ACLR shall be higher than the value specified in Table 6.5F.2.4.1-1.

Table 6.5F.2.4.1-1: Shared spectrum channel acess ACLR requirement

|  | Power class 5 |
| --- | --- |
| ACLR | 27 dB |

##### 6.5F.2.4.2	Additional requirement for network signaled value "NS_29"

When "NS_29" is indicated in the cell, the UE emission shall meet the additional requirements specified in Table 6.5F.2.4.2-1 for shared spectrum channels assigned within 5150 – 5350 MHz and 5470 – 5730 MHz.

Table 6.5F.2.4.2-1: ACLR2 requirement for "NS_29"

| Power class 5 | 20 MHz | 40 MHz | 60, 80 MHz |
| --- | --- | --- | --- |
| ACLR2 | 40 dB | 40 dB | N/A |
| Measurement bandwidth | 20 MHz | 40 MHz | N/A |
| Adjacent channel center frequency offset (MHz) | +40 / -40 | +80 / -80 | N/A |

### 6.5F.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [9] and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

#### 6.5F.3.1	General spurious emissions

The requirements for general spurious emission requirements in clause 6.5.3.1 apply.

#### 6.5F.3.2	Spurious emissions for UE co-existence

Spurious emissions requirements for UE coexistence are not applicable to bands restricted to stand-alone operation with shared spectrum channel access as identified in Table 5.2-1.

#### 6.5F.3.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

##### 6.5F.3.3.1	Requirement for network signalled value "NS_28"

When "NS_28" is indicated in the cell, the power of any UE emission for channels assigned within 5150-5350 and 5470-5725 MHz shall not exceed the levels specified in Table 6.5F.3.3.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.1-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80, [100] MHz |  |
| 47 ≤ f ≤ 74 | -54 | 100 kHz |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 862 | -54 | 100 kHz |
| 1000 ≤ f ≤ 5150 | -30 | 1 MHz |
| 5350 ≤ f ≤ 5470 | -30 | 1 MHz |
| 5725 ≤ f ≤ 26000 | -30 | 1 MHz |

##### 6.5F.3.3.2	Requirement for network signalled value "NS_29"

When "NS_29" is indicated in the cell, the power of any UE emission for channels assigned within 5150-5350 and 5470-5730 MHz shall not exceed the levels specified in Table 6.5F.3.3.2-1, Table 6.5F.3.3.2-2, and Table 6.F.3.3.2-3. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.2-1: Additional requirements for 20 MHz channel bandwidth

| Center Frequency Fc [MHz] | Protected range [MHz] | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 5179.98 ≤ Fc ≤ 5239.98 | 5135 ≤ f ≤ 5142 | -26 | 1 MHz |
|  | 5142 < f ≤ 5150 | -18 |  |
|  | 5250 ≤ f < 5250.2 | 3 to -2 |  |
|  | 5250.2 ≤ f < 5251 | -2 to -10 |  |
|  | 5251 ≤ f < 5260 | -10 to -18 |  |
|  | 5260 ≤ f < 5266.7 | -18 to -26 |  |
|  | 5266.7 ≤ f ≤ 5365 | -26 |  |
| 5260.02 ≤ Fc ≤ 5320.02 | 5135 ≤ f ≤ 5233.3 | -26 |  |
|  | 5233.3 < f ≤ 5240 | -26 to -18 |  |
|  | 5240 < f ≤ 5249 | -18 to -10 |  |
|  | 5249 < f ≤ 5249.8 | -10 to -2 |  |
|  | 5249.8 < f ≤ 5250 | -2 to 3 |  |
|  | 5350 ≤ f ≤ 5365 | -26 |  |
| 5500.02 ≤ Fc ≤ 5719.98 | 5420 ≤ f ≤ 5460 | -26 |  |
|  | 5460 < f ≤ 5470 | -19 |  |
|  | 5745 ≤ f < 5765 | -19 |  |
|  | 5765 ≤ f ≤ 5800 | -26 |  |
| NOTE: The minimum requirement when specified as a range denotes the emission requirement at the end points of the protected range. The requirement within the protected range is obtained by linear interpolation between the requirements at the end points. |  |  |  |

Table 6.5F.3.3.2-2: Additional requirements for 40 MHz channel bandwidth

| Center Frequency Fc [MHz] | Protected range [MHz] | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 5190 ≤ Fc ≤ 5230.02 | 5100 ≤ f ≤ 5141.6 | -26 | 1 MHz |
|  | 5141.6 < f ≤ 5150 | -18 |  |
|  | 5250 ≤ f < 5251 | -3 to -13 |  |
|  | 5251 ≤ f < 5270 | -13 to -21 |  |
|  | 5270 ≤ f < 5278.4 | -21 to -26 |  |
|  | 5278.4 ≤ f ≤ 5400 | -26 |  |
| 5269.98 ≤ Fc ≤ 5310 | 5210 < f ≤ 5221.6 | -26 |  |
|  | 5221.6 < f ≤ 5230 | -26 to -21 |  |
|  | 5230 < f ≤ 5249 | -21 to -13 |  |
|  | 5249 ≤ f ≤ 5250 | -13 to -3 |  |
|  | 5350 ≤ f ≤ 5358.4 | -18 |  |
|  | 5358.4 < f ≤ 5400 | -26 |  |
| 5509.98 ≤ Fc ≤ 5670 | 5420 ≤ f ≤ 5460 | -19 |  |
|  | 5460 < f ≤ 5470 | -13 |  |
|  | 5770 ≤ f ≤ 5800 | -19 |  |
| NOTE: The minimum requirement when specified as a range denotes the emission requirement at the end points of the protected range. The requirement within the protected range is obtained by linear interpolation between the requirements at the end points. |  |  |  |

Table 6.5F.3.3.2-3: Additional requirements for 60 and 80 MHz channel bandwidth

| Center Frequency Fc [MHz] | Protected range [MHz] | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 5200.02 ≤ Fc ≤ 5220 | 5020 ≤ f ≤ 5123.2 | -26 | 1 MHz |
|  | 5123.2 < f ≤ 5150 | -18 |  |
|  | 5250 ≤ f < 5251 | -6 to -16 |  |
|  | 5251 ≤ f < 5290 | -16 to -24 |  |
|  | 5290 ≤ f < 5296.7 | -24 to -26 |  |
|  | 5296.7 ≤ f ≤ 5480 | -26 |  |
| 5280 ≤ Fc ≤ 5299.98 | 5020 ≤ f ≤ 5203.3 | -26 |  |
|  | 5203.3 < f ≤ 5210 | -26 to -24 |  |
|  | 5210 < f ≤ 5249 | -24 to -16 |  |
|  | 5249 < f ≤ 5250 | -16 to -6 |  |
|  | 5350 ≤ f < 5376.8 | -18 |  |
|  | 5376.8 ≤ f ≤ 5480 | -26 |  |
| 5520 ≤ Fc ≤ 5689.98 | 5340 ≤ f ≤ 5460 | -19 |  |
|  | 5460 < f ≤ 5469.5 | -13 |  |
|  | 5469.5 < f ≤ 5470 | -13 |  |
|  | 5770 ≤ f ≤ 5800 | -19 |  |
| NOTE: The minimum requirement when specified as a range denotes the emission requirement at the end points of the protected range. The requirement within the protected range is obtained by linear interpolation between the requirements at the end points. |  |  |  |

##### 6.5F.3.3.3	Requirement for network signalled value "NS_30"

When "NS_30" is indicated in the cell, the power of any UE emission for channels assigned within 5150-5350 MHz, 5470-5725 MHz and 5725-5850 MHz shall not exceed the levels specified in Table 6.5F.3.3.3-1-1, Table 6.5F.3.3.3-1-2 and Table 6.5F.3.3.3-1-3, respectively. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.3-1: Additional requirements for shared access channels assigned within 5150-5350 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 ≤ f ≤ 5460 | -41 |  |

Table 6.5F.3.3.3-2: Additional requirements for shared access channels assigned within 5470-5725 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 ≤ f ≤ 5460 | -41 |  |
| 5460 < f ≤ 5470 | -27 |  |
| 5725 ≤ f | -27 |  |

Table 6.5F.3.3.3-3: Additional requirements for shared access channels assigned within 5725-5850 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80, [100] MHz |  |
| f < 5650 | -27 | 1 MHz |
| 5650 ≤ f < 5700 | -27 to 10 |  |
| 5700 ≤ f < 5720 | 10 to 15.6 |  |
| 5720 < f ≤ 5725 | 15.6 to 27 |  |
| 5850 ≤ f ≤ 5855 | 27 to 15.6 |  |
| 5855 < f ≤ 5875 | 15.6 to 10 |  |
| 5875 < f ≤ 5925 | 10 to -27 |  |
| 5925 < f | -27 |  |
| NOTE: The minimum requirement when specified as a range denotes the emission requirement at the end points of the protected range. The requirement within the protected range is obtained by linear interpolation between the requirements at the end points. |  |  |

##### 6.5F.3.3.4	Requirement for network signalled value "NS_31"

When "NS_31" is indicated in the cell, the power of any UE emission for channels assigned within 5150-5250 MHz, 5250-5350 MHz, 5470-5725 MHz and 5725-5850 MHz shall not exceed the levels specified in Table 6.5F.3.3.4-1, Table 6.5F.3.3.4-2, Table 6.5F.3.3.4-3 and Table 6.5F.3.3.4-4, respectively. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.4-1: Additional requirements for NR-U channels assigned within 5150-5250 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| f ≤ 5150 | -27 | 1 MHz |
| f ≥ 5250 | -27 |  |

Table 6.5F.3.3.4-2: Additional requirements for NR-U channels assigned within 5250-5350 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| f ≤ 5250 | -27 | 1 MHz |
| f ≥ 5350 | -27 |  |

Table 6.5F.3.3.4-3: Additional requirements for NR-U channels assigned within 5470-5725 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| f ≤ 5470 | -27 | 1 MHz |
| f ≥ 5725 | -27 |  |

Table 6.5F.3.3.4-4: Additional requirements for NR-U channels assigned within 5725-5850 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| f ≤ 5725 | -27 | 1 MHz |
| f ≥ 5850 | -27 |  |

##### 6.5F.3.3.5	Requirements for network signalled value "NS_53" or "NS_54"

When "NS_53" or "NS_54" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3.3.5-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

### 6.5F.4	Transmit intermodulation

The requirements for transmit intermodulation in clause 6.5F.4 apply.

## 6.6	Void

## 6.6E	Time alignment error

For V2X UE(s) with two  transmit antenna connectors in SL MIMO, this requirement applies to slot timing differences between transmissions on two transmit antenna connectors.The Time Alignment Error (TAE) shall not exceed 260 ns.

# 7	Receiver characteristics

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective clauses below.

The applicability of receiver requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the minimum requirements for Band n41.

With the exception of clause 7.3, the requirements shall be verified with the network signalling value NS_01 configured (Table 6.2.3-1).

All the parameters in clause 7 are defined using the UL reference measurement channels specified in Annexes A.2.2 and A.2.3, the DL reference measurement channels specified in Annex A.3.2 and using the set-up specified in Annex C.3.1.

The minium requirements specified in clauses 7.5, 7.6, 7.7 and 7.8 for NR band n48 refer to the minimum requirements for NR bands < 2.7 GHz.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks, an in-gap test refers to the case when the interfering signal is located at a negative offset with respect to the assigned lowest channel frequency of the highest sub-block and located at a positive offset with respect to the assigned highest channel frequency of the lowest sub-block.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks, an out-of-gap test refers to the case when the interfering signal(s) is (are) located at a positive offset with respect to the assigned channel frequency of the highest carrier frequency, or located at a negative offset with respect to the assigned channel frequency of the lowest carrier frequency.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing adjacent channel selectivity requirements, in-band blocking requirements (for each case), and narrow band blocking requirements apply for in-gap tests only if the corresponding interferer frequency offsets with respect to the two measured carriers satisfy the following condition in relation to the sub-block gap size Wgap for at least one of these carriers j = 1,2, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

where FInterferer (offset),j for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier j as specified in clause 7.5, clause 7.6.2 and clause 7.6.4 for the respective requirement and BWChannel(j) the channel bandwidth of carrier j. FInterferer (offset),j for a sub-block with two or more contiguous component carriers is the interference frequency offset with respect to the carrier adjacent to the gap is specified in clause 7.5A, 7.6A.2 and 7.6A.3. The interferer frequency offsets for adjacent channel selectivity, each in-band blocking case and narrow- band blocking shall be tested separately with a single in-gap interferer at a time.

For the additional requirements for operation with shared spectrum channel access, the receiver requirements apply under the assumption that all 20 MHz sub-bands and all RB’s of each sub-band within the downlink channel are allocated with intra-cell guard bands configured to zero.

## 7.1A	General

The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

The minium requirements specified in clauses 7.5A, 7.6A, 7.7A and 7.8A for NR band n48 refer to the minimum requirements for NR bands < 2.7 GHz.

## 7.2	Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n77, n78, n79 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination.

For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports.

For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.

The above rules apply for all clauses with the exception of clause 7.9.

## 7.3	Reference sensitivity

### 7.3.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

in all bands, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3.2-1 with 2 Rx antenna ports tested;

for bands where the UE is required to be equipped with 4 Rx antenna ports, the UE shall additionally be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2 with 4 Rx antenna ports tested.

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1 and Table 7.3.2-2.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |  |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 | -89.6 |  |  |  |  |  | FDD |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 | -89.7 |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 | -89.7 |  |  |  |  |  |  |  |
| n2 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  |  |  |  |  |  |  |  |  |  |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -82.3 |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -82.4 |  |  |  |  |  |  |  |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -82.6 |  |  |  |  |  |  |  |  |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -95.1 | -93.1 | -88.6 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 | -88.6 | -81.5 |  |  |  |  |  | FDD |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 | -90.8 | -90.0 | -88.7 | -81.5 |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 | -91.0 | -90.1 | -88.9 | -81.5 |  |  |  |  |  |  |  |
| n8 | 15 | -97.0 | -93.8 | -91.4 | -85.8 |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 | -100.0 | -96.8 |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -97.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -79.5 |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -79.6 |  |  |  |  |  |  |  |  |
|  | 60 |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 | -79.7 |  |  |  |  |  |  |  |  |
| n26 | 15 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  | -78.5 |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  | -78.6 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29x | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |  |  |  | SDL |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -99.0 | -95.8 |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -96.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |  |  |  | TDD |  |
|  | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 |  |  |  |  |  |  |  |  |  |  |  |  |
| n381 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 |  |  |  |  |  |  | TDD |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 |  |  |  |  |  |  |  |  |
| n39 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 |  |  |  |  |  |  | TDD |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 |  |  |  |  |  |  |  |  |
| n40 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 | -89.6 |  |  |  |  |  | TDD |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 | -89.7 | -88.9 |  | -87.6 |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 | -89.8 | -89.1 |  | -87.6 |  |  |  |  |
| n411 | 15 |  | -94.8 | -93.0 | -91.8 |  | -89.9 | -88.6 | -87.6 |  |  |  |  |  | TDD |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  | -90.0 | -88.7 | -87.7 | -86.9 | -86.2 | -85.6 | -85.1 | -84.7 |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  | -90.1 | -88.9 | -87.8 | -87.1 | -86.3 | -85.6 | -85.1 | -84.7 |  |  |
| n481 | 15 | -99 | -95.8 | -94.0 | -92.7 |  | -90.9 | -89.6 | -88.65 |  |  |  |  |  | TDD |  |
|  | 30 |  | -96.1 | -94.1 | -92.9 |  | -91.0 | -89.7 | -88.75 | -87.95 | -87.25 | -86.65 | -86.15 | -85.65 |  |  |
|  | 60 |  | -96.5 | -94.4 | -93.1 |  | -91.1 | -89.9 | -88.85 | -88.05 | -87.35 | -86.75 | -86.25 | -85.75 |  |  |
| n50 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  | -91.9 | -90.6 | -89.6 |  |  |  |  |  | TDD |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  | -92.0 | -90.7 | -89.7 | -88.9 |  | -87.6 |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  | -92.1 | -90.9 | -89.8 | -89.1 |  | -87.6 |  |  |  |  |
| n51 | 15 | -100.0 |  |  |  |  |  |  |  |  |  |  |  |  | TDD |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 | -100.0 | -96.8 |  |  |  |  |  |  |  |  |  |  |  | TDD |  |
|  | 30 |  | -97.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  | -89.2 |  |  |  |  |  | FDD |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  | -89.3 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  | -89.4 |  |  |  |  |  |  |  |
| n66 | 15 | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 | -90.1 |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 | -90.2 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 | -90.4 |  |  |  |  |  |  |  |  |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |  |  |  |  |  |
| n757 | 15 | -100 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 | -89.6 |  |  |  |  |  | SDL |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 | -89.7 |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 | -89.8 |  |  |  |  |  |  |  |
| n767 | 15 | -100 |  |  |  |  |  |  |  |  |  |  |  |  | SDL |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n771,4 | 15 |  | -95.3 | -93.5 | -92.2 | -91.2 | -90.4 | -89.1 | -88.1 |  |  |  |  |  | TDD |  |
|  | 30 |  | -95.6 | -93.6 | -92.4 | -91.3 | -90.5 | -89.2 | -88.2 | -87.4 | -86.7 | -86.1 | -85.6 | -85.1 |  |  |
|  | 60 |  | -96.0 | -93.9 | -92.6 | -91.5 | -90.6 | -89.4 | -88.3 | -87.5 | -86.8 | -86.2 | -85.7 | -85.2 |  |  |
| n781 | 15 |  | -95.8 | -94.0 | -92.7 | -91.7 | -90.9 | -89.6 | -88.6 |  |  |  |  |  | TDD |  |
|  | 30 |  | -96.1 | -94.1 | -92.9 | -91.8 | -91 | -89.7 | -88.7 | -87.9 | -87.2 | -86.6 | -86.1 | -85.6 |  |  |
|  | 60 |  | -96.5 | -94.4 | -93.1 | -92 | -91.1 | -89.9 | -88.8 | -88.0 | -87.3 | -86.7 | -86.2 | -85.7 |  |  |
| n791 | 15 |  |  |  |  |  |  | -89.6 | -88.6 |  |  |  |  |  | TDD |  |
|  | 30 |  |  |  |  |  |  | -89.7 | -88.7 | -87.9 |  | -86.6 |  | -85.6 |  |  |
|  | 60 |  |  |  |  |  |  | -89.9 | -88.8 | -88.0 |  | -86.7 |  | -85.7 |  |  |
| n91 | 15 | -100 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 | -100 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |  |  |  | FDD |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1 shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n28, n71 | -2.71 |
| n1, n2, n3, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79 | -2.2 |
| NOTE 1: 4 Rx operation is targeted for FWA form factor |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.2-1 and Table 7.3.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 | 1281 | 1281 | 1281 | 1281 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 641 | 641 | 641 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 301 | 301 | 301 |  |  |  |  |  |  |
| n2 | 15 | 25 | 501 | 501 | 501 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 | 101 | 24 | 241 | 241 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
| n3 | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |
| n5 | 15 | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | 25 | 501 | 751 | 751 | 721 | 641 | 451 | 451 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 361 | 361 | 321 | 201 | 201 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 181 | 181 | 161 | 101 | 101 |  |  |  |  |  |  |
| n8 | 15 | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n18 | 15 | 25 | 251 | 251 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | 25 | 201 | 202 | 202 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 102 | 102 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | 25 | 501 | 501 | 501 | 501 | 481 | 401 |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 | 201 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |
| n26 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 121 | 121 |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 25 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 216 |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 | 100 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 | 50 |  |  |  |  |  |  |  |
| n39 | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 216 |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 | 100 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 | 50 |  |  |  |  |  |  |  |
| n40 | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 216 | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 |  | 216 |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 |  | 100 |  |  |  |
| n41 | 15 |  | 50 | 75 | 100 |  | 160 | 216 | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  | 75 | 100 | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 |  | 36 | 50 | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n48 | 15 | 25 | 50 | 75 | 100 |  | 160 | 216 |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  | 75 | 100 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 |  | 36 | 50 |  |  |  |  |  |  |  |
| n50 | 15 | 25 | 50 | 75 | 100 |  | 160 | 216 | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  | 75 | 100 | 128 | 162 |  | NOTE 3 |  |  |  |
|  | 60 |  | 10 | 18 | 24 |  | 36 | 50 | 64 | 75 |  | NOTE 3 |  |  |  |
| n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | 25 | 501 | 751 | 1001 |  |  |  | 1281 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 |  |  |  | 641 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 |  |  |  | 301 |  |  |  |  |  |  |
| n66 | 15 | 25 | 501 | 751 | 1001 | 1281 | 160 | 216 |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 751 | 1001 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 361 | 501 |  |  |  |  |  |  |  |
| n70 | 15 | 25 | 501 | 751 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 51 | 51 | 51 |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | 50 | 75 | 100 | 128 | 160 | 216 | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n78 | 15 |  | 50 | 75 | 100 | 128 | 160 | 216 | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n79 | 15 |  |  |  |  |  |  | 216 | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  | 100 | 128 | 162 |  | 216 |  | 270 |  |
|  | 60 |  |  |  |  |  |  | 50 | 64 | 75 |  | 100 |  | 135 |  |
| n91 | 15 | 254 | 201,4 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 | 25 | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 | 254 | 251,4 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used. NOTE 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Unless given by Table 7.3.2-4, the minimum requirements specified in Tables 7.3.2-1 and 7.3.2-2 shall be verified with the network signalling value NS_01 (Table 6.2.3-1) configured.

Table 7.3.2-4: Network signaling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n2 | NS_03 |
| n12 | NS_06 |
| n13 | NS_06 |
| n14 | NS_06 |
| n24 | NS_52 |
| n25 | NS_03 |
| n30 | NS_21 |
| n48 | NS_27 |
| n53 | NS_45 |
| n66 | NS_03 |
| n70 | NS_03 |
| n71 | NS_35 |

### 7.3.3	ΔRIB,c

For a UE supporting CA, SUL or DC band combination, the minimum requirement for reference sensitivity in Table 7.3.2-1 shall be increased by the amount given by ΔRIB,c defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

## 7.3A	Reference sensitivity for CA

### 7.3A.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3A.2	Reference sensitivity power level for CA

#### 7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1, Table 7.3.2-2, and Table 7.3.2-3.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency.

For aggregation of two or more downlink FDD carriers with one or two uplink carriers, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.1-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.1-1: Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration |  | SCS (kHz) |  | Aggregated channel bandwidth (PCC+SCC) |  | UL PCC allocation (LCRB) |  | UL SCC allocation (LCRB) |  | PCC ΔRIBNC (dB) |  | SCC ΔRIBNC (dB) |  | Duplex mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7B |  | 15+15 |  | 216RB+52RB |  | 25 (RBstart = 191) |  | 20 (RBstart = 32) |  | [25] |  | [34] |  | FDD |  |
|  |  |  |  | 216RB+52RB |  | 64 (RBstart = 152) |  | 0 |  | [5.5] |  | [8.5] |  |  |  |
|  |  |  |  | 160RB+106RB |  | 64 (RBstart = 96) |  | 0 |  | [4] |  | [8.5] |  |  |  |
|  |  |  |  | 160RB+79RB |  | 64 (RBstart = 96) |  | 0 |  | [0] |  | [8] |  |  |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1, Table 7.3.2-2, and Table 7.3A.2.2-1 with the reference sensitivity power level increased by ΔRIBNC  given in Table 7.3A.2.2-1 for the SCC(s). For aggregation of two or more downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | 15 | 25RB+25RB | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15 | 25RB+25RB | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15 | 75RB + 25RB | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15 | 52RB+25RB | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n25(2A) | 15 | 25RB+25RB | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n41(2A) | N/A | NOTE 1 | NOTE 2 | NOTE 3 | 0.0 | TDD |
| CA_n66(2A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15 | 25RB+25RB | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 50RB+25RB | Wgap = 20.0 | 58 | 4.6 |  |
|  |  |  | Wgap = 5.0 | 208 | 2.3 |  |
|  |  | 75RB+50RB | Wgap = 10.0 | 59 | 22.2 |  |
|  |  |  | Wgap = 5.0 | 2010 | 5.2 |  |
| CA_n77(2A) |  | NOTE 1 | NOTE 2 | NOTE 3 | 0.0 | TDD |
| CA_n78(2A) |  | NOTE 1 | NOTE 2 | NOTE 3 | 0.0 | TDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: Uplink resource block starts at RB postion [9] for SCS=15KHz. NOTE 9: Uplink resource block starts at RB postion [2] for SCS=15KHz. NOTE 10: Uplink resource block starts at RB postion [19] for SCS=15KHz. |  |  |  |  |  |  |

#### 7.3A.2.3	Reference sensitivity power level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1, Table 7.3.2-2 and Table 7.3.2-3 modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4.

#### 7.3A.2.4	Void

clause

### 7.3A.3	ΔRIB,c for CA

#### 7.3A.3.1	General

For a UE supporting a CA configuration, the ΔRIB,c applies for both SC and CA operation.

#### 7.3A.3.2	ΔRIB,c for Inter-band CA

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in clause 7.3A.2 shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

##### 7.3A.3.2.1	ΔRIB,c for two bands

Table 7.3A.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n28 | n28 | 0.2 |
| CA_n1-n77 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78 | n78 | 0.5 |
| CA_n2-n48 | n2 | 0.2 |
|  | n48 | 0.5 |
| CA_n2-n66 | n2 | 0.3 |
|  | n66 | 0.3 |
| CA_n2-n77 | n2 | 0.2 |
|  | n77 | 0.5 |
| CA_n2-n78 | n2 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n41 | n41 | 04 |
|  |  | 0.55 |
| CA_n3-n77 | n3 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n79 | n79 | 0.5 |
| CA_n5-n77 | n5 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n78 | n5 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |
| CA_n7-n77 | n77 | 0.5 |
| CA_n7-n78 | n7 | 0.5 |
|  | n78 | 0.5 |
| CA_n8-n28 | n8 | 0.2 |
|  | n28 | 0.2 |
| CA_n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n8-n79 | n79 | 0.5 |
| CA_n20-n78 | n78 | 0.5 |
| CA_n25-n48 | n25 | 0.2 |
|  | n48 | 0.5 |
| CA_n25-n66 | n25 | 0.3 |
|  | n66 | 0.3 |
| CA_n25-n71 | n71 | 0.3 |
| CA_n25-n77 | n25 | 0.2 |
|  | n77 | 0.5 |
| CA_n25-n78 | n25 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n75 | n28 | 0.2 |
| CA_n28-n77 | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n79 | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n34-n79 | n79 | 0.5 |
| CA_n38-n66 | n38 | 0.5 |
|  | n66 | 0.5 |
| CA_n38-n78 | n38 | 0.4 |
|  | n78 | 0.5 |
| CA_n39-n40 | n39 | 0.3 |
|  | n40 | 0.3 |
| CA_n39-n41 | n39 | 0.22 |
|  | n41 | 0.22 |
|  | n39 | 0.23 |
|  | n41 | 0.23 |
| CA_n39-n79 | n79 | 0.5 |
| CA_n40-n78 | n40 | 0.4 |
|  | n78 | 0.5 |
| CA_n40-n79 | n79 | 0.5 |
| CA_n41-n66 | n41 | 0.56 |
|  |  | 17 |
|  | n66 | 0.5 |
| CA_n41-n71 | n71 | 0.2 |
| CA_n41-n771 | n77 | 0.5 |
| CA_n41-n781 | n78 | 0.5 |
| CA_n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n48-n66 | n48 | 0.5 |
|  | n66 | 0.2 |
| CA_n50-n78 | n50 | 0.22 |
|  | n78 | 0.22 |
|  | n50 | 0.23 |
|  | n78 | 0.23 |
| CA_n66-n77 | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n78 | n66 | 0.2 |
|  | n78 | 0.5 |
| CA_n71-n77 | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n71-n78 | n71 | 0.2 |
|  | n78 | 0.5 |
| CA_n75-n78 | n78 | 0.5 |
| CA_n76-n78 | n78 | 0.5 |
| CA_n78-n92 | n78 | 0.5 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz |  |  |

Table 7.3A.3.2.1-2: void

##### 7.3A.3.2.2	Void

##### 7.3A.3.2.3	ΔRIB,c for three bands

Table 7.3A.3.2.3-1: ΔRIB,c due to CA (three bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n7 | n1 | 0 |
|  | n3 | 0 |
|  | n7 | 0 |
| CA_n1-n3-n28 | n28 | 0.2 |
| CA_n1-n3-n41 | n41 | 05 |
|  |  | 0.56 |
| CA_n1-n3-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n7-n28 | n1 | 0 |
|  | n7 | 0 |
|  | n28 | 0.2 |
| CA_n1-n7-n78 | n1 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n8-n78 | n1 | 0 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n8-n79 | n1 | 0 |
|  | n8 | 0.2 |
|  | n79 | 0.5 |
| CA_n1-n28-n78 | n1 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n40-n78 | n1 | 0 |
|  | n40 | 0 |
|  | n78 | 0.5 |
| CA_n1-n77-n79 | n1 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |
| CA_n1-n78-n79 | n1 | 0 |
|  | n78 | 0.5 |
|  | n79 | 0 |
| CA_n2-n66-n77 | n2 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n7-n28 | n3 | 0 |
|  | n7 | 0 |
|  | n28 | 0 |
| CA_n3-n7-n78 | n3 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n8-n78 | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n18-n41 | n3 | 0 |
|  | n28 | 0 |
|  | n41 | 01/0.52 |
| CA_n3-n28-n41 | n3 | 0 |
|  | n28 | 0 |
|  | n41 | 01/0.52 |
| CA_n3-n28-n77 | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n28-n78 | n3 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n40-n41 | n41 | 01,3 |
|  |  | 0.52,3 |
| CA_n3-n41-n77 | n3 | 0.2 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| CA_n3-n41-n78 | n3 | 0.2 |
|  | n41 | 01/0.52 |
|  | n78 | 0.5 |
| CA_n3-n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n5-n7-n78 | n5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n5-n25-n77 | n5 | 0.2 |
|  | n25 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n25-n78 | n5 | 0.2 |
|  | n25 | 0.2 |
|  | n78 | 0.5 |
| CA_n5-n66-n77 | n5 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n5_n66-n78 | n5 | 0.5 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |
| CA_n7_n25-n66 | n7 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |
| CA_n7_n28-n78 | n7 | 0 |
|  | n28 | 0 |
|  | n78 | 0.5 |
| CA_n7_n66-n78 | n7 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |
| CA_n8-n39-n41 | n39 | 0.24 |
|  | n41 | 0.24 |
| CA_n8-n41-n79 |  |  |
|  | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n8-n78-n79 | n8 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.5 |
| CA_n13-n25-n66 | n13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |
| CA_n20-n28-n78 | n20 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n25-n29-n66 | n25 | 0.3 |
|  | n29 | 0 |
|  | n66 | 0.3 |
| CA_n25-n38-n78 | n25 | 0.2 |
|  | n38 | 0.4 |
|  | n78 | 0.5 |
| CA_n25-n41-n66 | n25 | 0.3 |
|  | n41 | 0.55 |
|  |  | 16 |
|  | n66 | 0.3 |
| CA_n25-n41-n71 | n25 | 0 |
|  | n41 | 0 |
|  | n71 | 0.2 |
| CA_n25-n48-n66 | n25 | 0.3 |
|  | n48 | 0.5 |
|  | n66 | 0.3 |
| CA_n25-n66-n71 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0.3 |
| CA_n25-n66-n78 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |
| CA_n25-n66-n77 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |
| CA_n25-n71-n77 | n25 | 0.2 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n40-n78 | n28 | 0 |
|  | n40 | 0 |
|  | n78 | 0.5 |
| CA_n28-n41-n77 | n28 | 0.2 |
|  | n41 | 0 |
|  | n77 | 0.5 |
| CA_n28-n77-n79 | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |
| CA_n28-n78-n79 | n28 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |
| CA_n28-n41-n78 | n28 | 0.2 |
|  | n41 | 0 |
|  | n78 | 0.5 |
| CA_n28-n41-n77 | n28 | 0.2 |
|  | n41 | 0 |
|  | n77 | 0.5 |
| CA_n39-n40-n79 | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.5 |
| CA_n39-n41-n79 | n39 | 0.34 |
|  | n41 | 0.34 |
|  | n79 | 0.8 |
| CA_n40-n41-n79 | n40 | 08 |
|  | n41 | 0.58 |
|  | n79 | 0.5 |
| CA_n41-n66-n71 | n41 | 0.51 |
|  |  | 12 |
|  | n66 | 0.5 |
|  | n71 | 0 |
| CA_n41-n66-n77 | n41 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n41-n71-n77 | n41 | 0 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n71-n77 | n66 | 0.2 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n71-n78 | n66 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. NOTE 4: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx between n39 and n41. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 7: Void. NOTE 8: Void. |  |  |

##### 7.3A.3.2.4	ΔRIB,c for four bands

Table 7.3A.3.2.4-1: ΔRIB,c due to CA (four bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n7-n28 | n28 | 0.2 |
| CA_n1-n3-n7-n78 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.5 |
| CA_n1-n3-n8-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n3-n28-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n5-n7-n78 | n3 | 0.2 |
|  | n5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n7-n28-n78 | n3 | 0.2 |
|  | n7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n28-n41-n77 | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| CA_n3-n28-n41-n78 | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
|  | n78 | 0.5 |
| CA_n7-n25-n66-n78 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n41-n66-n71 | n25 | 0.3 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
| CA_n41-n66-n71-n77 | n41 | 01/0.52 |
|  | n66 | 0.5 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

### 7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for a band in frequency range 1 if it is impacted by UL harmonic interference from another band in frequency range 1 of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-1 with uplink configuration specified in Table 7.3A.4-2.

Table 7.3A.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |  | dB | dB | dB |
| n1 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n2 | n481, 2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n2 | n771,2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 |  | 15.5 | 14.8 | 14.3 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| 2 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n3 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
|  | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.9 | 16.1 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n5 | n774,5 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 |  | 2.8 | 2.3 | 2.1 |
| n5 | n776,7 |  | 10.4 | 8.9 | 7.8 | 7.4 | 6.5 | 4.7 | 3.7 | 3 |  | 2.35 | 1.7 | 1.2 |
| n5 | n784,5 |  | 10.5 | 8.9 | 7.8 |  |  | 5.4 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| n8 | n311 | N/A | N/A | N/A | N/A | N/A | N/A |  |  |  |  |  |  |  |
|  | n418,9 |  | 13.0 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 |  | 4.3 | 3.9 | 3.5 |
|  | n784,5 |  | 10.8 | 9.1 | 8.0 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
|  | n796,7 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| n20 | n784,5 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n25 | n481,2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n25 | n771,2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 |  | 15.5 | 14.8 | 14.3 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| 25 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n28 | n18,9 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
|  | n501,2 |  | 19.8 | 18.0 | 16.8 |  |  | 13.8 | 12.8 | 12.0 |  | 10.8 |  |  |
|  | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 | 21.8 | 21.0 | 19.7 | 18.7 |  |  |  |  |  |
|  | n776,7 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
|  | n786,7 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| n66 | n481,2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 |  | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n66 | n771,2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 |  | 15.3 | 14.8 | 14.3 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |  |
| n66 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n71 | n2510 | 10 | 7.5 | 6 | 5.1 | 4.1 | 3.0 | 2.1 |  |  |  |  |  |  |
|  | n414,5 |  | 10.8 | 9.1 | 8.0 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
|  | n708,9 | 9.9 | 7.1 | 6.7 | 4.9 | 4.1 |  |  |  |  |  |  |  |  |
| n92 | n784,5 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 7: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.2 apply unless otherwise specified). NOTE 12: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n77 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n2 | n48 | 25 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n2 | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n2 | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n3 | n77 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n3 | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n5 | n77 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n5 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 | n78 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n8 | n79 |  |  |  |  |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| n20 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n25 | n48 | 25 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n25 | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | n2 |
| n25 | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n28 | n1 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n28 | n50 |  | 25 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 |  |  |
| n28 | n75 | 12 | 25 | 36 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |
| n28 | n77 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n28 | n78 |  | 10 | 15 | 20 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| n66 | n48 | 12 | 25 | 36 | 50 |  |  | 100 | 128 | 160 |  | 200 | 200 | 200 |
| n66 | n77 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n66 | n78 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n71 | n25 | 84 | 84 | 84 | 84 | 84 | 84 | 84 |  |  |  |  |  |  |
| n71 | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n71 | n70 | 8 | 16 | 20 | 20 | 20 |  |  |  |  |  |  |  |  |
| n92 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-3: Void

Table 7.3A.4-3a: Void

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-4 with uplink configuration specified in Table 7.3A.4-4a.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | n713,4 | 26.5 | 23.3 | 20.9 | 15.3 |  |  |  |  |  |  |  |  |  |
| n40 | n284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |  |  |
| n40 | n781 |  | 8.3 | 8.0 | 6.9 |  |  | 3.9 | 3 | 2.3 |  | 1.2 |  | 0.4 |
| n413,4 | n18 | [24.3] | [24.3] | [22.5] |  |  |  |  |  |  |  |  |  |  |
| n41 | n781 |  | 8.3 | 8.0 | 6.9 |  |  | 3.9 | 3 | 2.3 |  | 1.2 |  | 0.4 |
| n77 | n2 | 6.7 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |  |  |
| n77 | n5 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |  |  |
| n77 | n25 | 6.7 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |  |  |
| n77 | 412 |  | 10.4 | 10.4 | 10.4 |  | 10.4 | 10.4 | 10.4 | 10.4 |  | 10.4 | 10.4 | 10.4 |
| n78 | n402 | 10.4 | 10.4 | 10.4 | 10.4 |  |  | 7.2 | 6.2 | 5.5 |  | 4.5 |  |  |
| n78 | n412 |  | 10.4 | 10.4 | 10.4 |  |  | 8.2 | 7.6 | 7.3 |  | 6.6 | 6.4 | 6.3 |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n25 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | n28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | n78 | 30 |  | 24 | 24 | 24 |  |  | 24 | 24 | 24 |  | 24 |  | 24 |
| n41 | n18 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |  |  |
| n41 | n78 | 30 |  | 24 | 24 | 24 |  |  | 24 | 24 | 24 |  | 24 |  | 24 |
| n77 | n2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n77 | n5 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |  |
| n77 | n25 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n77 | 41 | 15 |  | 25 | 36 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n78 | n40 | 30 | 50 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 |  |  |
| n78 | n41 | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands given in Table 7.3A.5-1, Table 7.3A.5-1a and Table 7.3A.5-2 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.5-1, Table 7.3A.5-1a and Table 7.3A.5-2. For these test points the reference sensitivity requirement specified in Table 7.3.2-1 and Table 7.3.2-2 are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.5-1, Table 7.3A.5-1a and Table 7.3A.5-2.

Table 7.3A.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
| CA_n1-n77 | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  |  |  |  |  |  | 32.5 5 |  |  |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 8.0 | FDD | IMD4 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n2-n78 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
| CA_n3-n18 | n18 | 818 | 5 | 25 | 863 | N/A | FDD | N/A |
|  | n3 | 1731 | 5 | 25 | 1826 | 4 | FDD | IMD4 |
| CA_n3-n38 | n3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | n38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  |  |  |  |  |  | 28.74 |  |  |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.74 |  |  |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
| CA_n5-n66 ) | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n5-n77 | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | 5 | 829 | 5 | 25 | 875 | 5.5 | FDD | IMD5 |
|  | n77 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| CA_n7-n66 | n7 | 2535 | 10 | 50 | 2655 | 15 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
| CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 7.1 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n79 | n8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |
| CA_n20-n78 | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n28-n50 | n28 | 730 | 10 | 50 | 775 | 15.3 | FDD | IMD2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
|  | n28 | 740 | 10 | 50 | 785 | 6.0 | FDD | IMD44 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
| CA_n28-n77 | n28 | N/A | N/A | N/A | N/A | N/A7 | FDD | IMD2 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n77/n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 11 | FDD | IMD4 |
| CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |
| CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |
| CA_n70-n71 | n70 | 1697.5 | 5 | 25 | 1997.5 | 5 | FDD | IMD4 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD5 |
|  | n77 | 3300 | 10 | 50 | 3300 | N/A | TDD | N/A |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 7: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD2 result will fall in Rx frequency range of band n28. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). |  |  |  |  |  |  |  |  |

Table 7.3A.5-1a: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1A-n78A | n1 | 1950 | 5 | 25 | 2140 | [17.8] | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |

Table 7.3A.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n41 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n41 | 2507.5 | 10 | 25 | 2507.5 | 5.0 | TDD | IMD5 |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 |  | IMD2 |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |
| CA_n1-n7-n28 | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2533 | 10 | 50 | 2653 | 30.0 | FDD | IMD2 |
|  | n28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |
|  | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n28 | 730 | 10 | 50 | 785 | 4.5 | FDD | IMD5 |
| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | FDD | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 8.7 | FDD | IMD4 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | TDD | IMD4 |
| CA_n1-n77-n79 | n1 | 1950 | 5 | 25 | 2140 | 6.0 | FDD | IMD31,2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 15.6 | FDD | IMD31,2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n2-n66-n77 | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 31.2 | FDD | IMD2 |
|  | n77 | 4010 | 10 | 50 | 4010 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.3 | FDD | IMD4 |
|  | n77 | 3480 | 10 | 50 | 3480 | N/A | TDD | N/A |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 2.8 | FDD | IMD5 |
|  | n77 | 3860 | 10 | 50 | 3860 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| CA_n3-n8-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | 4.5 | TDD | IMD5 |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.7 | FDD | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
| CA_n3-n18-n41 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2540 | 10 | 50 | 2540 | [N/A]1 | TDD | IMD2 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | 16.0 | TDD | IMD3 |
|  | n18 | 820 | 5 | 25 | 865 | 28.9 | FDD | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n18 | 830 | 5 | 25 | 875 | [19.0] | FDD | IMD3 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 28.8 | FDD | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
| CA_n3-n28-n41 | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 |
| CA_n3-n28-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | TDD | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | FDD | IMD3 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 15.3 | FDD | IMD3 |
| CA_n3-n28-n78 | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | IMD3 |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.3 | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | TDD | IMD5 |
| CA_n3-40-n41 | n3 | 1747.5 | 5 | 25 | 1842.5 | 1.0 | FDD | IMD5 |
|  | n40 | 2347.5 | 5 | 25 | 2347.5 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n41 | 2640 | 5 | 25 | 2640 | 5.3 | TDD | IMD5 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 16.4 | FDD | IMD3 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | 16.8 | TDD | IMD31 |
| CA_n3-n41-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | TDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.4 | TDD | IMD3 |
|  | n3 | 1745 | 5 | 25 | 1840 | 16.4 | TDD | IMD3 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | FDD | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
| CA_n5-n25-n66 | n5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1712 | 5 | 25 | 2132 | 7.2 | FDD | IMD4 |
| CA_n5-n25-n77 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | TDD | IMD3 |
|  | n5 | 844 | 5 | 25 | 889 | 3.8 | FDD | IMD5 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | 1907 | 5 | 25 | 1987 | 16.5 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| CA_n5-n25-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n78 | 3560 | 10 | 50 | 3560 | 16.1 | TDD | IMD3 |
| CA_n5-n66-n77 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | 16.1 | TDD | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4192 | 10 | 50 | 4192 | 8.2 | TDD | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n77 | 3590 | 10 | 50 | 3590 | 3.3 | TDD | IMD5 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 14.4 | FDD | IMD3 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n5-n66-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | 16.1 | TDD | IMD3 |
| CA_n5-n66-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | 13.2 | FDD | IMD3 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n7-n66-n78 | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD3 |
| CA_n7-n66-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | FDD | IMD4 |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
| CA_n13-n25-n66 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | 1736 | 5 | 25 | 2156 | 7..2 | FDD | IMD4 |
|  | n25 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n13 | 780 | 10 | 50 | 749 | N/A | FDD | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 6.2 | FDD | IMD4 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
| CA_n25-n38-n78 | n25 | 1852.5 | 5 | 25 | 1932.5 | 16.4 | FDD | IMD3 |
|  | n38 | 2617.5 | 5 | 25 | 2617.5 | N/A | TDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | 14.8 | TDD | IMD3 |
|  | n25 | 1880 | 5 | 25 | 1960 | 8.6 | TDD | IMD4 |
|  | n38 | 2570 | 5 | 25 | 2570 | N/A | FDD | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | 14.8 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n77 | 3775 | 10 | 50 | 3775 | 4.2 | TDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2640 | 5 | 25 | 2640 | 5.3 | TDD | IMD5 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 17.6 | FDD | IMD3 |
|  | n41 | 2565 | 5 | 25 | 2565 | N/A | TDD | N/A |
|  | n77 | 3180 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 8.6 | FDD | IMD4 |
|  | n41 | 2550 | 5 | 25 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3475 | N/A | TDD | N/A |
| CA_n25-n66-n77 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | FDD | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | 10 | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
| CA_n25-n71-n77 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | 8.0 | TDD | IMD31,2 |
|  | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| CA_n28-n41-n77 | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD24 |
|  | n41 | 2567.5 | 10 | 50 | 2567.5 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n28 | 727.5 | 5 | 25 | 782.5 | 3.0 | FDD | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | 3323 | 10 | 50 | 3323 | 28.2 | TDD | IMD24 |
| CA_n28-n41-n78 | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |
|  | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD21 |
|  | n41 | 2565 | 5 | 25 | 2565 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | TDD | IMD22 |
| CA_n38-n66-n78 | n38 | 2550 | 5 | 25 | 2550 | N/A | TDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | FDD | IMD4 |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3460 | 10 | 50 | 3460 | 15.0 | TDD | IMD3 |
| CA_n39-n40-n79 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 5.8 | TDD | IMD4 |
| CA_n40-n41-n79 | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | 30.5 | TDD | IMD2 |
| CA_n41-n66-n77 | n41 | 2560 | 5 | 25 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD31,2 |
|  | n41 | 2670 | 5 | 25 | 2670 | 5.2 | TDD | IMD5 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2530 | 5 | 25 | 2530 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21 |
|  | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | 16.3 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
|  | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2615 | 5 | 25 | 2615 | 15.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | N/A | TDD | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n71 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n77 | 4108 | 10 | 50 | 4108 | 15.9 | TDD | IMD31,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | FDD | IMD3 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| CA_n66-n71-n78 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n78 | 3724 | 10 | 50 | 3724 | 9 | TDD | IMD41 |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 7.3A.6	Reference sensitivity exceptions due to cross band isolation for CA

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same NR CA configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3A.6-1 with uplink configuration of the agressor band specified in Table 7.3A.6-2.

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n1 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 |  |  |  |  |  |  |  |
| n1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  | 6.6 |  |  |
| n1 | n41 |  | 6.1 | 6.1 | 6.1 |  |  | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 |
| n3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 |
| n38 | n78 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 |  | 4.0 | 3.9 | 3.8 |
| n40 | n1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |  |
| n41 | n1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |  |
| n41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |
| n7 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |
| n41 | n25 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |
| n38 | n25 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |
| n411 | n66 | 3.5 | 3.5 | 3.5 | 3.5 |  |  | 3.5 |  |  |  |  |  |  |
| n41 | n77 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n41 | n78 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n78 | n71 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |
| n78 | n38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |  |
| n78 | n401 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |  | 4.5 |  |  |
| n78 | n411 |  | 4.5 | 4.5 | 4.5 |  | 4.5 | 4.5 | 4.5 | 4.5 |  | 4.5 | 4.5 | 4.5 |
| n783 | n79 |  |  |  |  |  |  | 2 | 2 | 2 |  | 2 |  | 2 |
| n79 | n783 |  | 2.6 | 2.6 | 2.6 |  |  | 2.6 | 2.6 | 2.6 |  | 2.6 | 2.6 | 2.6 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.6.2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |
| n1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 |  |  |
| n1 | n41 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n3 | n41 | 15 |  | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n38 | n78 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n40 | n1 | 30 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n41 | n1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| n41 | n3 | 30 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |
| n7 | n3 | 15 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |
| n41 | n25 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| n38 | n25 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| n41 | n66 | 30 | 128 | 128 | 128 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |
| n41 | n77 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n41 | n78 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n7 | 30 | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  |  |  |  |  |
| n78 | n38 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | n40 | 30 | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  | 270 |  |  |
| n78 | n41 | 30 |  | 270 | 270 | 270 |  | 270 | 270 | 270 | 270 |  | 270 | 270 | 270 |
| n783 | n79 | 30 |  |  |  |  |  | 270 | 270 | 270 | 270 |  | 270 |  | 270 |
| n79 | n783 | 30 |  | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  | 270 | 270 | 270 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. NOTE 2: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 7.3B	Reference sensitivity for NR-DC

For inter-band NR-DC configurations, the reference sensitivity for the corresponding inter-band CA configuration as specified in clause 7.3A applies.

## 7.3C	Reference sensitivity for SUL

### 7.3C.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3C.2	Reference sensitivity power level for SUL

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in Table 7.3.2-1 and Table 7.3.2-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 or supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.2-1 with reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1), unless sensitivity degradation is allowed in this clause of this specification. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 or supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.2-1 with reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1), unless sensitivity degradation is allowed in this clause of this specification. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

Table 7.3C.2-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n80 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| n1 | n841 | 15 | 25 | 50 | 75 | 100 | 128 | 128 | 128 | 128 |  |  |  |  |  |
| n3 | n801 | 15 | 25 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |  |
| n28 | n831 | 15 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |  |
| n41 | n80 | 15 |  | 160 | 160 | 160 |  | 160 | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n41 | n81 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n41 | n83 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n41 | n95 | 15 |  | 75 | 75 | 75 |  | 75 | 75 | 75 | 75 |  | 75 | 75 | 75 |
| n41 | n98 | 15 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 |  | 216 | 216 | 216 |
| n77 | n80 | 15 |  | 160 | 160 | 160 |  |  | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n77 | n84 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n78 | n80 | 15 |  | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |
| n78 | n81 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n78 | n82 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n78 | n83 | 15 |  | 100 | 100 | 100 | 160 | 160 | 100 | 100 | 100 | 160 | 100 | 100 | 100 |
| n78 | n84 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n86 | 15 |  | 216 | 216 | 216 |  |  | 216 | 216 | 216 |  | 216 | 216 | 216 |
| n79 | n80 | 15 |  |  |  |  |  |  | 160 | 160 | 160 |  | 160 |  | 160 |
| n79 | n83 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
|  |  | 30 |  |  |  |  |  |  | 50 | 50 | 50 |  | 50 |  | 50 |
| n79 | n81 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
| n79 | n84 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
| n79 | n95 | 15 |  |  |  |  |  |  | 75 | 75 | 75 |  | 75 |  | 75 |
| n79 | n97 | 15 |  |  |  |  |  |  | 270 | 270 | 270 |  | 270 |  | 270 |
| n79 | n98 | 15 |  |  |  |  |  |  | 216 | 216 | 216 |  | 216 |  | 216 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE that supports any of the SUL operation given in Table 7.3C.2-2, exceptions to the requirements specified in Table 7.3.2-1are allowed when the uplink is active in a lower frequency band and is within a specified frequency range such that transmitter harmonics fall within the downlink transmission bandwidth assigned in a higher band as noted in Table 7.3C.2-2. For these exceptions, the UE shall meet the requirements specified in Table 7.3C.2-2 and Table 7.3C.2-3.

Table 7.3C.2-2: Reference sensitivity for SUL operation (exceptions due to harmonic issue)

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB |  | dB | dB | dB |
| n80 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n80 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n81 | n418,9 |  | 13 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 |  | 4.3 | 3.9 | 3.5 |
|  | n784,5 |  | 10.8 | 9.1 | 8 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 1.5 | 1.4 |
|  | n796,7 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| n82 | n784,5 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n83 | n786,7 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| n84 | n771,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n86 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n97 | n791,2 |  |  |  |  |  |  | 29.4 | 28.4 | 27.6 |  | 26.3 |  | 25.3 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for SUL_n78-n80, SUL_n78-n86. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3C.2-3: Supplementary uplink configuration (exceptions due to harmonic issue)

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (NRB) | 10 MHz (NRB) | 15 MHz (NRB) | 20 MHz (NRB) | 25 MHz (NRB) | 30 MHz (NRB) | 40 MHz (NRB) | 50 MHz (NRB) | 60 MHz (NRB) | 70 MHz (NRB) | 80 MHz (NRB) | 90 MHz (NRB) | 100 MHz (NRB) |
| n80 | n77 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n80 | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| n81 | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n81 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n81 | n79 |  |  |  |  |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| n82 | n78 |  | 16 | 20 | 20 |  |  | 20 | 20 | 20 |  | 20 | 20 | 20 |
| n83 | n78 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n84 | n77 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n86 | n78 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n97 | n79 |  |  |  |  |  |  | 100 | 135 | 160 |  | 216 |  | 270 |
| NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band NOTE 3: Unless stated otherwise, UL resource blocks shall be centered within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same SUL configuration due to cross band isolation issues. Reference sensitivity exceptions are specified in Table 7.3C.2-4 with uplink configuration specified in Table 7.3C.2-5.

Table 7.3C.2-4: Reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 | n41 |  | 4.3 | 4.0 | 3.9 |  | 3.9 | 3.9 | 3.5 | 3.3 | 3.2 | 3.1 | 3.0 |
| n95 | n41 |  | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| NOTE 1: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3C.2-5: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 | n41 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| n95 | n41 |  | 75 | 75 | 75 |  | 75 | 75 | 75 | 75 | 75 | 75 | 75 |
| NOTE: 15 kHz SCS is assumed for UL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.3C.3	ΔRIB,c for SUL

#### 7.3C.3.1	General

For a UE supporting a SUL configuration, the ΔRIB,c applies for both SC and SUL operation.

#### 7.3C.3.2	SUL band combination

For the UE which supports SUL band combiantion, the minimum requirement for reference sensitivity in clause 7.3C.2 shall be increased by the amount given in ΔRIB,c defined in clause 7.3C.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

##### 7.3C.3.2.1	ΔRIB,c  for two bands

Table 7.3C.3.2.1-1: ΔRIB,c due to SUL (two bands)

| Band combination for SUL | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| SUL_n41-n80 | n41 | 0.5 (note) |
| SUL_n41-n95 | n41 | 0.2 |
| SUL_n41-n98 | n41 | 0.2 |
| SUL_n77-n80 | n77 | 0.5 |
| SUL_n77-n84 | n77 | 0.5 |
| SUL_n78-n80 | n78 | 0.5 |
| SUL_n78-n81 | n78 | 0.5 |
| SUL_n78-n82 | n78 | 0.5 |
| SUL_n78-n83 | n78 | 0.5 |
| SUL_n78-n84 | n78 | 0.5 |
| SUL_n78-n86 | n78 | 0.5 |
| SUL_n79-n83 | n79 | 0.5 |
| SUL_n79-n97 | n79 | 0.5 |
| SUL_n79-n98 | n79 | 0.5 |
| NOTE: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

##### 7.3C.3.2.2	ΔRIB,c  for three bands

Table 7.3C.3.2.2-1: ΔRIB,c due to SUL (three bands)

| Band combination for SUL | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1_SUL_n78-n80 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n1_SUL_n78-n84 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n3_SUL_n78-n80 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n28_SUL_n41-n83 | n28 | 0.2 |
| CA_n28_SUL_n79-n83 | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n41_SUL_n79-n80 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n41_SUL_n79-n83 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n79_SUL_n41-n80 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n79_SUL_n41-n83 | n41 | 0.5 |
|  | n79 | 0.5 |

## 7.3D	Reference sensitivity for UL MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in clause 7.3 shall be met with the UL MIMO configurations described in clause 6.2D.1 and the reference measurement channels as specified in Annexes A.2.2 and A.2.3 for CP-OFDM waveforms shall apply. For UL MIMO, the parameter PUMAX is the total transmitter power over the two transmits power over the two transmit antenna connectors.

## 7.3E	Reference sensitivity for V2X

### 7.3E.1	General

The reference sensitivity power level PREFSENS_V2X is the minimum mean power applied to each one of the UE antenna port for V2X UE, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3E.2	Minimum requirements

When UE is configured for NR V2X reception non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2-1.

Table 7.3E.2-1: Reference sensitivity of NR V2X Bands (PC5)

|  |  | Channel bandwidth / PREFSENS_V2X(dBm) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n38 | 15 | -96.5 | -93.2 | -91.4 | -90.1 | HD |
|  | 30 | -96.1 | -93.4 | -91.7 | -90.2 | HD |
|  | 60 | -96.9 | -93.1 | -91.9 | -90.4 | HD |
| n47 | 15 | -92.5 | -89.2 | -87.4 | -86.1 | HD |
|  | 30 | -92.1 | -89.4 | -87.7 | -86.2 | HD |
|  | 60 | -92.9 | -89.1 | -87.9 | -86.4 | HD |
| NOTE 1: Reference measurement channel is defined in A.8. NOTE 2: The signal power is specified per antenna port. NOTE 3: Void. |  |  |  |  |  |  |

Table 7.3E.2-2: Sidelink TX configuration for reference sensitivity of NR V2X Bands (PC5)

| NR Band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n38 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 | HD |
|  | 60 | 102 | 24 | 36 | 50 | HD |
| n47 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 | HD |
|  | 60 | 102 | 24 | 36 | 50 | HD |
| NOTE 1: The sidelink allocated RB (LCRB) size could be adjusted according to resource pool configuration in [7]. NOTE 2: For the case, 11 RB is allowed for S-SSB Block. |  |  |  |  |  |  |

### 7.3E.3	Reference sensitivity power level for V2X con-current operation

When UE is configured for NR V2X reception on V2X carrier con-current with NR uplink and downlink, NR V2X sidelink throughput for the carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes 8.2 with parameters specified in Table 7.3E.3-1. Also the NR downlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.

For the inter-band con-current NR V2X operation, and the UE also supports an NR downlink inter-band con-current configuration in Table 7.3E.3-2, the minimum requirement for reference sensitivity shall be increased by the amount given in ΔRIB,V2X in Table 7.3E.3-2 for the corresponding NR V2X inter-band combinations.

Table 7.3E.3-1: Reference sensitivity for V2X Communication QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | NR band | NR Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100Mhz (dBm) | Duplex Mode |
| n47 | n39 | n39 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 |  |  |  |  |  | TDD |
|  |  |  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 |  |  |  |  |  |  |
|  |  |  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 |  |  |  |  |  |  |
|  |  | n47 | 15 |  | -92.5 |  | -89.2 |  | -87.4 | -86.1 |  |  |  |  |  | HD |
|  |  |  | 30 |  | -92.1 |  | -89.4 |  | -87.7 | -86.2 |  |  |  |  |  |  |
|  |  |  | 60 |  | -92.9 |  | -89.1 |  | -87.9 | -86.4 |  |  |  |  |  |  |
| n47 | n40 | n40 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 | -89.6 |  |  |  |  | TDD |
|  |  |  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 | -89.7 | -88.9 | -87.6 |  |  |  |
|  |  |  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 | -89.8 | -89.1 | -87.6 |  |  |  |
|  |  | n47 | 15 |  | -92.5 |  | -89.2 |  | -87.4 | -86.1 |  |  |  |  |  | HD |
|  |  |  | 30 |  | -92.1 |  | -89.4 |  | -87.7 | -86.2 |  |  |  |  |  |  |
|  |  |  | 60 |  | -92.9 |  | -89.1 |  | -87.9 | -86.4 |  |  |  |  |  |  |
| n47 | n41 | n41 | 15 |  | -94.8 | -93.0 | -91.8 |  | -89.9 | -88.6 | -87.6 |  |  |  |  | TDD |
|  |  |  | 30 |  | -95.1 | -93.1 | -92.0 |  | -90.0 | -88.7 | -87.7 | -86.9 | -85.6 | -85.1 | -84.7 |  |
|  |  |  | 60 |  | -95.5 | -93.4 | -92.2 |  | -90.1 | -88.9 | -87.8 | -87.1 | -85.6 | -85.1 | -84.7 |  |
|  |  | n47 | 15 |  | -92.5 |  | -89.2 |  | -87.4 | -86.1 |  |  |  |  |  | HD |
|  |  |  | 30 |  | -92.1 |  | -89.4 |  | -87.7 | -86.2 |  |  |  |  |  |  |
|  |  |  | 60 |  | -92.9 |  | -89.1 |  | -87.9 | -86.4 |  |  |  |  |  |  |
| n47 | n71 | n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  |  |  | FDD |
|  |  |  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n47 | 15 |  | -92.5 |  | -89.2 |  | -87.4 | -86.1 |  |  |  |  |  | HD |
|  |  |  | 30 |  | -92.1 |  | -89.4 |  | -87.7 | -86.2 |  |  |  |  |  |  |
|  |  |  | 60 |  | -92.9 |  | -89.1 |  | -87.9 | -86.4 |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is defined in A.7.2. NOTE 2: The signal power is specified per antenna port. NOTE 3: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3E.3-2: ΔRIB,V2X (two bands)

| V2X inter-band con-current band Combination | NR Band | ΔRIB,V2X [dB] |
| --- | --- | --- |
| V2X_n71-n47 | n71 | 0.0 |

The reference sensitivity is defined to be met with NR uplink assigned to one band (that differs from the V2X operating band) and all NR downlink carriers active. The NR uplink resource blocks shall be located as close as possible to NR V2X operating band but confined within the transmission bandwidth configuration for the channel. The uplink configuration for the NR operating band is specified in Table 7.3E.3-3 and 7.3E.3-4. The REFSENS of Uu downlink and PC5 sidelink will be tested at the same time.

Table 7.3E.3-3: Uplink configuration for REFSENS of NR V2X Bands (PC5)

| Inter-band NR V2X con-current band configuration |  | NR UL band / SCS/ Channel BW / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR V2X band (Uu) | NR V2X UL band (Uu) | SCS (kHz) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | n39 | n39 | 15 | 10 | 52 | TDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 11 |  |
| n47 | n40 | n40 | 15 | 10 | 52 | TDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 11 |  |
| n47 | n41 | n41 | 15 | 10 | 52 | TDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 11 |  |
| n47 | n71 | n71 | 15 | 10 | 52 | FDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 11 |  |

Table 7.3E.3-4: Sidelink TX configuration for REFSENS of NR V2X Bands (Uu)

| Inter-band NR V2X con-current band configuration |  | NR UL band / SCS/ Channel BW / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR V2X band (Uu) | NR V2X band (PC5) | SCS (kHz) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | n39 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| n47 | n40 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| n47 | n41 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| n47 | n71 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| NOTE 1: The sidelink allocated RB (LCRB) size could be adjusted according to resource pool configuration in [7]. |  |  |  |  |  |  |

clause

## 7.3F	Reference sensitivity for shared spectrum channel access

### 7.3F.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3G.2-1 with 2 Rx antenna ports tested.

### 7.3F.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3F.2-1, Table 7.3F.2-2, and Table 7.3F.2-3.

Table 7.3F.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 20 MHz (dBm) | 40 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) |
| n46 | 15 | -89.7 | -86.6 |  |  |
|  | 30 | -89.9 | -86.7 | -84.8 | -83.6 |
|  | 60 | -90.1 | -86.9 | -85.0 | -83.6 |
| n96 | 15 | -89.2 | -86.1 |  |  |
|  | 30 | -89.4 | -86.2 | -84.3 | -83.1 |
|  | 60 | -89.6 | -86.4 | -84.5 | -83.1 |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3F.2-1 shall be modified by the amount given in ΔRIB,4R in Table 7.3F.2-2 for the applicable operating bands.

Table 7.3F.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n46, n96 | -2.2 |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3F.2-1 and Table 7.3F.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3F.2-3.

Table 7.3F.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 20 MHz (dBm) | 40 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) |
| n46 | 15 | 100 | 216 |  |  |
|  | 30 | 50 | 100 | 162 | 216 |
|  | 60 | 24 | 50 | 75 | 100 |
| n96 | 15 | 100 | 216 |  |  |
|  | 30 | 50 | 100 | 162 | 216 |
|  | 60 | 24 | 50 | 75 | 100 |

Unless given by Table 7.3F.2-4, the minimum requirements specified in Tables 7.3F.2-1 and 7.3F.2-2 shall be verified with the network signalling value NS_01 (Table 6.2F.3.1-1) configured.

Table 7.3F.2-4: Network signaling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n46 | NS_01 |
| n96 | NS_53 |

### 7.3F.3	ΔRIB,c

For a UE supporting CA or DC band combination, the minimum requirement for reference sensitivity in Table 7.3F.2-1 shall be increased by the amount given by ΔRIB,c defined in Table 7.3F.3-1.  Unless otherwise stated, ΔRIB,c is set to zero.

Table 7.3F.3-1: ΔRIB,c due to CA (two bands)

| Inter-band CA combination | Operating Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n46-n48 | n46 | 0 |
|  | n48 | 0.5 |

In case the UE supports more than one of band combinations for CA or DC, and an operating band belongs to more than one band combinations then the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A and 7.3F.3 in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

### 7.3F.4	Intra-band contiguous shared spectrum channel access CA

For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3F.2-1, Table 7.3F.2-2, and Table 7.3F.2-3.

### 7.3G.5	Inter-band CA with shared spectrum channel access

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput of the NR carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1, Table 7.3.2-2 and Table 7.3.2-3 modified in accordance with clause 7.3F.3. The throughput of the NR-U carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3F.2-1, Table 7.3F.2-2, and Table 7.3F.2-3 modified in accordance with clause 7.3F.3.  The reference sensitivity is defined to be met with all downlink component carriers active and the PCell uplink carrier active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3F.5.1 and clause 7.3F.5.2.

#### 7.3G.5.1	Reference sensitivity exceptions due to UL harmonic interference

The reference sensitivity for the shared access band does not apply when there is at least one individual RE within the shared access downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3F.5.1-1.

Table 7.3F.5.1-1: NR-U reference sensitivity measurement exclusion region in MHz.

| NR Band / Harmonic order / Channel BW in UL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | Harmonic order | 5MHz | 10MHz | 15MHz | 20 MHz | 40MHz |
| n25 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 | +/- 90 |
| n66 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 | +/- 90 |
| NOTE 1: Even though UL harmonic does not fall directly into NR-U band the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |  |

#### 7.3F.5.2	Reference sensitivity exceptions due to cross band isolation

For unsynchronized operation, Rx de-sensing in one band will be caused by another band due to lack of isolation in the band filters. Reference sensitivity exceptions for cross band are specified in Table 7.3F.5.2-1 with uplink configuration specified in Table 7.3F.5.2-2-2.

Table 7.3F.5.2-1: MSD for cross band isolation

| Operating Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| CA_n46A-n48A | n46 | n48 | 13.3 | 10.4 | 8.8 | 7.8 | - | - | 7.8 | 7 | 6.5 | 5.7 | 5.4 | 5.1 |
|  | n48 | n46 | - | - | - | 13.5 | - | - | 10.9 | - | 9.4 | 8.7 | - | - |

Table 7.3F.5.2-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| Operating Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n46 | n48 | 30 | 216 | 216 | 216 | 216 |  |  | 216 | 216 | 216 | 216 | 216 | 216 |
| n48 | n46 | 15 |  |  |  | 216 |  |  | 216 |  | 216 | 216 |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. NOTE 2: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 7.4	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4-1.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252 |  |  |  | -242 | -232 | -222 | -212 | -202 |  |  |  |  |
|  |  | -273 |  |  |  | -263 | -253 | -243 | -233 | -223 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 7.4A	Maximum input level for CA

### 7.4A.1	Maximum input level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation maximum input level is defined as the maximum mean power received at the UE antenna port, over the Transmission bandwidth configuration of each CC.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4A.1-1 for each component carrier.

Table 7.4A.1-1: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232 | -232 | -252 |  |
|  |  | -253 | -253 | -273 |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |  |  |

### 7.4A.2	Maximum input level for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the maximum input level requirements are defined with the uplink configuration in accordance with 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in Table 7.4-1 and Table 7.4A.1-1 for one component carrier and two component carriers per sub-block, respectively. The throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the specified reference measurement channel as specified in Annex A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1 and A.5.2.1. The requirements apply with all downlink carriers active.

### 7.4A.3	Maximum input level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the maximum input level is defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.4 for each component carrier while all downlink carriers are active.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) for each component carrier.

## 7.4B	Maximum input level for NR-DC

For inter-band NR-DC configurations, the maximum input level for the corresponding inter-band CA configuration as specified in clause 7.4A applies.

## 7.4D	Maximum input level for UL MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing, the minimum requirements specified in clause 7.4 shall be met with the UL MIMO configurations described in clause 6.2D.1. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

## 7.4E	Maximum input level for V2X

### 7.4E.1	General

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.7.3 and A.7.4 with parameters specified in Table 7.4E.1-1.

Table 7.4E.1-1: Maximum input level of NR V2X

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -251 | -251 | -231 | -221 |
|  |  | -272 | -272 | -252 | -242 |
| NOTE 1: Reference measurement channel is A.7.3 for 64 QAM. NOTE 2: Reference measurement channel is A.7.4 for 256 QAM. |  |  |  |  |  |

### 7.4E.2	Maximum input level for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.4E shall apply for the NR sidelink reception in Band n47 and the requirements specified in clause 7.4 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

## 7.5	Adjacent channel selectivity

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The UE shall fulfil the minimum requirements specified in Table 7.5-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and the minimum requirements specified in Table 7.5-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5-3 and Table 7.5-4 for verification of the requirements specified in Table 7.5-1, and as in Table 7.5-5 and Table 7.5-6 for verification of the requirements specified in Table 7.5-2. For these test parameters, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.5-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| ACS | dB | 33 | 33 | 30 | 27 | 26 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| ACS | dB | 25.5 | 24 | 23 | 22.5 | 21 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| ACS | dB | 20.5 | 20 |  |  |  |

Table 7.5-2: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| ACS | dB | 33 | 33 | 33 | 33 | 33 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| ACS | dB | 33 | 33 | 33 | 33 | 33 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| ACS | dB | 33 | 33 |  |  |  |

Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 dB | REFSENS + 38.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 38 dB | REFSENS + 36.5 dB | REFSENS + 35.5 dB | REFSENS + 35 dB | REFSENS + 33.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 33 dB | REFSENS + 32.5 dB |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.5-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | -56.5 | -56.5 | -53.5 | -50.5 | -49.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | -49 | -47 | -46.5 | -46 | -44.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | -44 | -43.5 |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |

Table 7.5-5: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 25 / -25 | 30 / -30 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 45.5 dB |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| Finterferer (offset) | MHz | 40 / -40 | 50 / -50 | 60 / -60 | 70 / -70 | 80 / -80 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB | REFSENS + 45.5 dB |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| Finterferer (offset) | MHz | 100 / -90 | 100 / -100 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.5-6: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 25 / -25 | 30 / -30 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 | -25 | -25 | -25 |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| Finterferer (offset) | MHz | 40 / -40 | 50 / -50 | 60 / -60 | 70 / -70 | 80 / -80 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| Finterferer (offset) | MHz | 90 / -90 | 100 / -100 |  |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

## 7.5A	Adjacent channel selectivity for CA

### 7.5A.1	Adjacent channel selectivity for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.5A.1-1 and 7.5A.1-1a for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5A.1-2, 7.5A.1-2a, 7.5A.1-3 and 7.5A.1-3a.

Table 7.5A.1-1: ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D |  |
| ACS | dB | 26.0 | 33.0 | 25.2 |  |

Table 7.5A.1-1a: ACS for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

|  |  | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C |
| ACS | dB | 20.0 | 17.0 |

Table 7.5A.1-2: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |
| PInterferer | dBm | Aggregated power + 24.5 dB | Aggregated power + 31.5 dB | Aggregated power + 23.7 dB |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FInterferer (offset) | MHz | 10 + Foffset / -10 - Foffset | BWchannel CA / -BWchannel CA | 25 + Foffset / -25 -Foffset |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |

Table 7.5A.1-2a: Test parameters for intra-band contiguous CA with FDL_low<2700 MHz and FUL_low<2700 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 18.5 dB | Aggregated power + 15.5 dB |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Table 7.5A.1-3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -49.5 + 10log(NRB,c/NRB_agg) | -56.5 | -48.7 + 10log(NRB,c/NRB_agg) |  |
| PInterferer | dBm | -25 | -25 | -25 |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FInterferer (offset) | MHz | 10 + Foffset / -10 -Foffset | BWchannel CA / -BWchannel CA | 25 + Foffset / -25 -Foffset |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |

Table 7.5A.1-3a: Test parameters for intra-band contiguous CA with FDL_low <2700 MHz and FUL_low<2700 MHz, case 2

| Rx Parameter | Units | NR CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -43.5 + 10log(NRB,c/NRB_agg) | -40.5 + 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 | -25 |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

### 7.5A.2	Adjacent channel selectivity Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with FDL_low < 2700 MHz and FUL_low < 2700 MHz with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the adjacent channel selectivity requirements are defined with the uplink configuration in accordance with Table 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clauses 7.5 and 7.5A.1 for one component carrier and two component carriers per sub-block, respectively. The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active. For the lower range of test parameters (Case 1), the interferer power Pinterferer  shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5-3 and Table 7.5A.1-2a for one component carrier and two component carriers per sub-block, respectively. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer  in accordance with the ACS requirement for each sub-block (Table 7.5-1 and Table 7.5A.1-1a). For the upper range of test parameters (Case 2) for which the interferer power Pinterferer  is -25 dBm (Table 7.5-4 and Table 7.5A.1-3a) the wanted signal power levels for the carriers of each sub-block shall be adjusted relative to Pinterferer  like for Case 1.

For intra-band non-contiguous carrier aggregation with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the adjacent channel selectivity requirements are defined with the uplink configuration in accordance with Table 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clauses 7.5 and 7.5A.1 for one component carrier and two component carriers per sub-block, respectively. The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active. For the lower range of test parameters (Case 1), the interferer power Pinterferer  shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5-5 and Table 7.5A.1-2 for one component carrier and two component carriers per sub-block, respectively. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer  in accordance with the ACS requirement for each sub-block (Table 7.5-2 and Table 7.5A.1-1). For the upper range of test parameters (Case 2) for which the interferer power Pinterferer  is -25 dBm (Table 7.5-6 and Table 7.5A.1-3) the wanted signal power levels for the carriers of each sub-block shall be adjusted relative to Pinterferer  like for Case 1.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

### 7.5A.3	Adjacent channel selectivity Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the adjacent channel requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.5 for each component carrier while all downlink carriers are active.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

## 7.5B	Adjacent channel selectivity for NR-DC

For inter-band NR-DC configurations, the adjacent channel selectivity for the corresponding inter-band CA configuration as specified in clause 7.5A applies.

## 7.5D	Adjacent channel selectivity for UL MIMO

For UE(s) with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in clause 7.5 shall be met with the UL MIMO configurations described in clause 6.2D.1. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

## 7.5E	Adjacent channel selectivity for V2X

### 7.5E.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The UE shall fulfil the minimum requirements specified in Table 7.5E.1-1 for NR V2X UE. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5E.1-2 and Table 7.5E.1-3 for verification of the requirements specified in Table 7.5E.1-1. For these test parameters, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2.

In licensed band, the minimum requirements shall reuse the same ACS values with NR UE.

Table 7.5E.1-1: Adjacent channel selectivity for NR V2X

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| ACS | dB | 33.0 | 27.0 | 25.5 | 24.0 |

Table 7.5E.1-2: Test parameters for Adjacent channel selectivity for V2X, Case 1

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in transmission bandwidth configuration | dBm | PREFSENS_V2X + 14 dB |  |  |  |
| Pinterferer | dBm | PREFSENS_V2X + 45.5 dB | PREFSENS_V2X + 39.5 dB | PREFSENS_V2X + 38.0 dB | PREFSENS_V2X + 36.5 dB |
| BWinterferer | MHz | 10 | 10 | 10 | 10 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 25 / -25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |

Table 7.5E.1-3: Test parameters for Adjacent channel selectivity for V2X, Case 2

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in transmission bandwidth configuration | dBm | -56.5 | -50.5 | -49.0 | -47.5 |
| Pinterferer | dBm | -25 |  |  |  |
| BWinterferer | MHz | 10 | 10 | 10 | 10 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 25 / -25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |

### 7.5E.2	Adjacent channel selectivity for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.5E shall apply for the NR sidelink reception in Band n47 and the requirements specified in clause 7.5 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

## 7.5F	Adjacent channel selectivity

### 7.5F.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

Instead of the general ACS requirements specified in clause 7.5, the UE shall fulfil the minimum requirements specified in Table 7.5F.1-1. These requirements apply for any SCS specified for the channel bandwidth of the wanted signal.  For the test parameters specified in Table 7.5F.1-2, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.5F.1-1: ACS for shared spectrum channel access bands

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| ACS | dB | 24 | 21 | 19.2 | 18 |

Table 7.5F.1-2: Test parameters for shared spectrum channel acess bands

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer | dBm | REFSENS + 36.5 dB | REFSENS + 33.5 dB | REFSENS + 31.7 dB | REFSENS + 30.5 dB |
| BWinterferer | MHz | 20 |  |  |  |
| Finterferer (offset) | MHz | 20 / -20 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |

### 7.5F.2	Intra-band contiguous shared spectrum channel access CA

ACS for intra-band contiguous shared access CA requirements are specified in Table 7.5F.2-1.  These requirements apply for any SCS specified for the channel bandwidth of the wanted signal.  For the test parameters specified in Table 7.5F.2-2, the throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.5F.2-1: ACS for intra-band contiguous shared access CA

|  |  | NR-U CA bandwidth class |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D | E | I | M | N | O |
| ACS | dB | 24 – 10log10(BWChannel_CA/20) |  |  |  |  |  |  |  |

Table 7.5F.1-2: Test parameters for intra-band contiguous NR-U CA

| Rx Parameter | Units | NR-U CA bandwidth class |
| --- | --- | --- |
|  |  | B, C, D, E, M, N, O |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 22.5 – 10log10(BWChannel_CA/20) dB |
| BWInterferer | MHz | 20 |
| FInterferer (offset) | MHz | 10 + Foffset / -10 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

## 7.6	Blocking characteristics

### 7.6.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

For shared spectrum channel access and band combinations with operating bands intended for shared spectrum channel access, the blocking characteristics is specified in clause 7.6F.

### 7.6.2	In-band blocking

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz in-band blocking (IBB) is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.  The throughput of the wanted signal shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.2-1 and Table 7.6.2-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.2-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. |  |  |  |  |  |  |

Table 7.6.2-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28,n34, n38,n39, n40, n41, n483, n50, n51, n53, n65, n66, n70, n74, n75, n76, n91, n92, n93, n94 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. |  |  |  |  |  |  |

For NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz in-band blocking (IBB) is defined for an unwanted interfering signal falling into the UE receive band or into an immediately adjacent frequency range up to 3*BWChannel below or above the UE receive band where BWChannel is the bandwidth of the wanted signal. The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1)] with parameters specified in Table 7.6.2-3 and Table 7.6.2-4. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.2-3: In-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| FIoffset, case 1 | MHz | 15 | 22.5 | 30 | 37.5 | 45 |
| FIoffset, case 2 | MHz | 25 | 37.5 | 50 | 62.5 | 75 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| FIoffset, case 1 | MHz | 60 | 75 | 90 | 105 | 120 |
| FIoffset, case 2 | MHz | 100 | 125 | 150 | 175 | 200 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| FIoffset, case 1 | MHz | 135 | 150 |  |  |  |
| FIoffset, case 2 | MHz | 225 | 250 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |

Table 7.6.2-4: In-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3*BWChannel to FDL_high + 3*BWChannel |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: BWChannel denotes the channel bandwidth of the wanted signal |  |  |  |  |

### 7.6.3	Out-of-band blocking

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band. The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3-1 and Table 7.6.3-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.3-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.6.3-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, | Pinterferer | dBm | -44 | -30 | -15 |
| n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n485, n50, n51, n536, n65, n66, n70, n71, n74, n75, n76, n91, n92, n93, n94 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 5: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz. NOTE 6: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2580 MHz and FInterferer < 2775 MHz. NOTE 7 For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. NOTE8: For bands 91 and 93 the FDL_high of bands 92 and 94 are applied as FDL_high for bands 91 and 93. For bands 92 and 94, the FDL_low of bands 91 and 93 are applied as FDL_low for bands 92 and 94 |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-2, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

For NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range up to 3*BWChannel below or from 3*BWChannel above the UE receive band, where BWChannel is the channel bandwidth. The throughput of the wanted signal shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3-3 and Table 7.6.3-4. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.3-3: Out-of-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 |  |  |  |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.6.3-4: Out of-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low ≤ -3*BWChannel or 3*BWChannel ≤ f – FDL_high < 60 | -200 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*BWChannel) or FDL_high + MAX(200,3*BWChannel) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel from the band edge. For BWChannel larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-4, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

### 7.6.4	Narrow band blocking

This requirement is measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The relative throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.4-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.4-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n70, n71, n74, n75, n76 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 | 5.2125 | 7.7025 | 10.2075 | 13.0275 | 15.6075 | 20.5575 | 25.7025 | NA | NA | NA | NA |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA | NA | NA | NA | NA | NA | NA | NA | 30.855 | 40.935 | 45.915 | 50.865 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 7.6A	Blocking characteristics for CA

### 7.6A.1	General

### 7.6A.2	In-band blocking for CA

#### 7.6A.2.1	In-band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.6A.2.1-1 and 7.6A.2.1-1a for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6A.2.1-1: In-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dB | REFSENS + CA bandwidth class specific value below |  |  |  |
|  |  | 10.0 | 6 | 13.8 |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FIoffset, case 1 | MHz | 30 | BWchannel CA+ BWchannel CA/2 | 75 |  |
| FIoffset, case 2 | MHz | 50 | BWInterferer + FIoffset, case 1 | 125 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattrn OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |

Table 7.6A.2.1-1a: In-band blocking parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below |  |
|  |  | 16.0 | 19.0 |
| BWInterferer | MHz | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |

Table 7.6A.2.1-2: In-band blocking for intra-band contiguous CA  with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 3BWchannel CA to FDL_high + 3BWchannel CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.1-2a: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n41, n66, n484,n40 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

#### 7.6A.2.2	In-band blocking for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the in-band blocking requirements are defined with the uplink configuration in accordance with Table 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clause 7.6.2 and 7.6A.2.1 for one component carrier and two component carriers per sub-block, respectively. The requirements apply for in-gap and out-of-gap interferers while all downlink carriers are active.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

#### 7.6A.2.3	In-band blocking for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the in-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in clause 7.6.2 for each component carrier while all downlink carriers are active.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2, Pinterferer power defined in Table 7.6.2-2 and 7.6.2-4 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.

For E-UTRA CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The requirements for the component carrier configured in the operating band without uplink operation are specified in Table 7.6A.2.3-1.

Table 7.6A.2.3-1: In-band blocking parameters for additional NR operating bands for carrier aggregation with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |
| n29 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal NOTE 4: BWChannel denotes the channel bandwidth of the wanted signal |  |  |  |  |

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

### 7.6A.3	Out-of-band blocking for CA

#### 7.6A.3.1	Out-of-band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggreagation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test.

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.6A.3-1 and Table 7.6A.3-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6A.3-1: Out-of-band blocking parameters for intra-band contiguous CA

| RX parameter | Units | CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |  |
|  | dB | 9 | 9 | 9 |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |

Table 7.6A.3-1a: Void

Table 7.6A.3-2: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n41,n66,n71,n485,n40 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n77, n78 (NOTE 3) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3*BWChannel_CA) or FDL_high+ MAX(200,3*BWChannel_CA) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel_CA) or FDL_high + MAX(150,3*BWChannel_CA) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel_CA denotes the aggregated channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel_CA > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel_CA from the band edge. For BWChannel_CA larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel_CA≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 5: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz |  |  |  |  |  |

Table 7.6A.3-2a: Void

#### 7.6A.3.2	Out-of-band blocking for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the out-of-band blocking requirements are defined with the uplink configuration in accordance with table 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clauses 7.6.3 and 7.6A.3.1 for one component carrier and two component carriers per sub-block, respectively. The requirements apply with all downlink carriers active.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

#### 7.6A.3.3	Out-of-band blocking for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the out-of-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.6.3 for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with component carriers in operating bands < 2.7GHz including n48, and for FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz, the appropriate adjacent channel selectivity and in-band blocking requirements in the respective clauses 7.5 and 7.6.2 shall be applied for carrier j. For inter-band carrier aggregation with component carriers in operating bands > 2.7GHz excluding n48, and for FDL_Low(j) – 3* BWchannel ≤ f ≤ FDL_High(j) + 3* BWchannel, the appropriate adjacent channel selectivity and in-band blocking requirements in the respective clauses 7.5 and 7.6.2 shall be applied for carrier j. FDL_Low(j) and FDL_High(j) denote the respective lower and upper frequency limits of the operating band containing carrier j, j = 1,…,X, with carriers numbered in increasing order of carrier frequency and X the number of component carriers in the band combination. BWchannel denotes the channel bandwidth of the wanted signal component carrier j. If CW interferer falls in a gap between FDL_High(j) and FDL_Low(j+1) where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

For inter-band carrier aggregation with uplink assigned to two NR bands, the out-of-band blocking requirements specified in clause 7.6.3 shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,f,c  for each serving cell c.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.1-1, Pinterferer power defined in Table 7.6.3-2 and 7.6.3-4 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.1-1.

For inter-band CA combination listed in Table 7.6A.3.3-1, exceptions to the requirement specified in Table 7.6A.3.3-2 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 7.6A.3.3-1: CA band combination with exceptions allowed

CA band combination

CA_n5-n77

CA_n5-n78

CA_n5-n79

CA_n8-n78

CA_n8-n79

CA_n20-n78

CA_n28-n77

CA_n28-n78

CA_n28-n79

CA_n71-n77

CA_n71-n78

CA_n78-n92

Table 7.6A.3.3-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when fInterferer± fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

For all interferer frequency ranges specified in clause 7.6.3 a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of   MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

### 7.6A.4	Narrow band blocking for CA

#### 7.6A.4.1	Narrow band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6A.4.1-1 with the uplink configuration. For UE(s) supporting one uplink, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.6A.4.1-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6A.4.1-1.

Table 7.6A.4.1-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |  |
| --- | --- | --- | --- | --- |
|  |  |  | B | C |
| n1, n41, n66, n71,n48, n40 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA Bandwidth Class specific value below |  |
|  |  |  | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 |
|  | Fuw (offset forf = 15 kHz, 30 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

#### 7.6A.4.2	Narrow band blocking for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with FDL_low < 2700 MHz and FUL_low < 2700 MHz with one uplink carrier and two or more downlink sub-blocks, the narrow band blocking requirements are defined with the uplink configuration in accordance with Table 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clauses 7.6.4 and 7.6A.4.1 for one component carrier and two component carriers per sub-block, respectively. The requirements apply for in-gap and out-of-gap interferers while all downlink carriers are active.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

#### 7.6A.4.3	Narrow band blocking for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the narrow band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.6.4 for each component carrier while all downlink carriers are active.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.1-1, PUW power defined in Table 7.6.4-1 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.1-1.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

## 7.6B	Blocking characteristics for NR-DC

For inter-band NR-DC configurations, the blocking characterstics for the corresponding inter-band CA configuration as specified in clause 7.6A applies.

## 7.6C	Blocking characteristics for SUL

### 7.6C.1	General

### 7.6C.2	In-band blocking for SUL

For SUL operation, the in-band blocking requirement for downlink bands specified in clause 7.6.2 shall be met.

For SUL operation with downlink CA, the in-band blocking requirement for downlink bands specified in clause 7.6A.2 shall be met.

### 7.6C.3	Out-of-band blocking for SUL

For SUL operation, the out-of-band blocking requirement for downlink bands specified in clause 7.6.3 shall be met. For SUL operation with downlink CA, the out-of-band blocking requirement for downlink bands specified in clause 7.6A.3 shall be met. For operation band combination listed in Table 7.6C.3-1, exceptions to the requirement specified in Table 7.6C.3-2 are allowed when the second order intermodulation product of the SUL carrier and the CW interfering signal fully or partially overlaps with the DL carrier.

Table 7.6C.3-1: SUL operating band combination with exceptions allowed

NR Band combination for SUL

SUL_n78-n81

SUL_n78-n82

SUL_n78-n83

SUL_n79-n81

SUL_n79-n83

Table 7.6C.3-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when ¦fInterferer± fSUL- fDL¦≤(BWSUL+ BWDL)/2, where BWSUL and BWDL are the channel bandwidths configured for SUL and DL (victim) bands in MHz, respectively. |  |  |

For all interferer frequency ranges specified in clause 7.6.3 a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

### 7.6C.4	Narrow band blocking for SUL

Narrow band blocking is not specified for SUL band combination.

## 7.6D	Blocking characteristics for UL MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in clause 7.6 shall be met with the UL MIMO configurations described in clause 6.2D.1. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

## 7.6E	Blocking characteristics for V2X

### 7.6E.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

### 7.6E.2	In-band blocking

#### 7.6E.2.1	General

The throughput of the wanted signal shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with parameters specified in Table 7.6E.2.1-1 and Table 7.6E.2.1-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6E.2.1-1: In-band blocking parameters for NR V2X

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in transmission bandwidth configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |
|  | dB | 6 | 9 | 11 | 12 |
| BWinterferer | MHz | 10 |  |  |  |
| FIoffset, case 1 | MHz | 15 |  |  |  |
| FIoffset, case 2 | MHz | 25 |  |  |  |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |  |  |  |

Table 7.6E.2.1-2: In-band blocking for NR V2X

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
| n38, n47 | Pinterferer | dBm | -44 | -44 |
|  | Finterferer (offset) | MHz | -BW/2 – FIoffset, case 1 and BW/2 + FIoffset, case 1 | ≤ -BW/2 – FIoffset, case 2 and ≥ BW/2 + FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 30 to FDL_high + 30 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 – FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies NOTE 4: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |

#### 7.6E.2.2	In-band blocking for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.6E2 shall apply for the NR sidelink reception in Band n47 and the requirements specified in clause 7.6.2 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

### 7.6E.3	Out-of-band blocking

#### 7.6E.3.1	General

For NR V2X bands out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 30 MHz below or above the UE receive band. The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.6E.3.1-1 and Table 7.6E.3.1-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6E.3.1-1: Out-of-band blocking parameters for NR V2X

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in transmission bandwidth configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |
|  | dB | 6 | 9 | 11 | 12 |
| NOTE: Reference measurement channel is A.7.2. |  |  |  |  |  |

Table 7.6E.3.1-2: Out of-band blocking for NR V2X

| NR band | Parameter | Units | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n47 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | FDL_low -30 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| n38 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | FDL_low -30 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4400 MHz. |  |  |  |  |  |

#### 7.6E.3.2	Out-of-band blocking for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.6E3 shall apply for the NR sidelink reception in Band n47 and the requirements specified in clause 7.6.3 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

## 7.6F	Blocking characteristics

### 7.6F.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

### 7.6F.2	In-band blocking

#### 7.6F.2.1	General

In-band blocking (IBB) is defined for an unwanted interfering signal falling into the UE receive band or into the first 60 MHz below or above the UE receive band.  clauseThe throughput of the wanted signal shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6F.2.1-1 and Table 7.6F.2.1-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6F.2.1-1: In-band blocking parameters for shared access bands

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |
|  | dB | 9 | 12 | 13.8 | 15 |
| BWinterferer | MHz | 20 |  |  |  |
| FIoffset, case 1 | MHz | 30 |  |  |  |
| FIoffset, case 2 | MHz | ≥ 50 |  |  |  |

Table 7.6F.2.1-2: In-band blocking for shared access bands

| Operating band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
|  | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
| n46, n96 | Finterferer |  | NOTE 2 | FDL_low – 3*CBW to FDL_high + 3*CBW, NOTE 4 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal NOTE 4: Interferer carrier frequencies in the frequency range for Case 2 shall be located at discrete frequencies in integer multiples of 20 MHz offset from -CBW/2 – FIoffset, case 2 and CBW/2 + FIoffset, case 2 |  |  |  |  |

#### 7.6F.2.2	Intra-band contiguous shared spectrum channel access CA

In-band blocking for intra-band contiguous shared access CA requirements are specified in Table 7.6F.2.2-1.  These requirements apply for any SCS specified for the channel bandwidth of the wanted signal.  For the test parameters specified in Table 7.6F.2.2-2, the throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6F.2.2-1: In-band blocking parameters for intra-band contiguous shared access CA

| Rx Parameter | Units | Shared access CA bandwidth class |
| --- | --- | --- |
|  |  | B, C, D, E, M, N, O |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + aggregated channel bandwidth value below |
|  | dB | 9 + 10log10(BWChannel_CA/20) |
| BWInterferer | MHz | 20 |
| FIoffset, case 1 | MHz | 30 |
| FIoffset, case 2 | MHz | ≥ 50 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |

Table 7.6F.2.2-2: In-band blocking for intra-band contiguous shared access CA

| Operating band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
|  | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
| n46 | Finterferer | MHz | NOTE 2 | FDL_low – 3* BWchannel CA to FDL_high + 3* BWchannel CA NOTE 4 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: Interferer carrier frequencies in the frequency range for Case 2 shall be located at discrete frequencies in integer multiples of 20 MHz offset from - BWchannel CA /2 – FIoffset, case 2 and BWchannel CA /2 + FIoffset, case 2 |  |  |  |  |

### 7.6F.3	Out-of-band blocking

#### 7.6F.3.1	General

Out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 60 MHz or greater below or above the UE receive band. clauseThe throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6F.3.1-1 and Table 7.6F.3.1-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6F.3.1-1: Out-of-band blocking parameters for shared access bands

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |
|  | dB | 9 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |

Table 7.6F.3.1-2: Out of-band blocking for shared access bands

| Operating band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n46, n96 | Finterferer (CW) | MHz | N/A | -200 < f – FDL_low ≤ -3*CBW or 3*CBW ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*CBW) or FDL_high + MAX(200,3*CBW) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4200 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6F.3-2, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7F apply.

#### 7.6F.3.2	Intra-band contiguous shared spectrum channel access CA

Out-of-band blocking for intra-band contiguous shared access CA requirements are specified in Table 7.6F.3.2-1.  These requirements apply for any SCS specified for the channel bandwidth of the wanted signal.  For the test parameters specified in Table 7.6F.3.2-2, the throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6F.3.2-1: Out-of-band blocking parameters for intra-band contiguous shared access CA

| Rx Parameter | Units | Shared access CA bandwidth class |
| --- | --- | --- |
|  |  | B, C, D, E, M, N, O |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA bandwidth class specific value below |
|  | dB | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |

Table 7.6F.3.2-2: Out of-band blocking for intra-band contiguous CA

| Operating band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n46 | Finterferer (CW) | MHz | N/A | -200 < f – FDL_low ≤ -3*BWChannel_CA or 3*BWChannel_CA ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*BWChannel_CA) or FDL_high + MAX(200,3*BWChannel_CA) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 4200 MHz. |  |  |  |  |  |

## 7.7	Spurious response

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in clause 7.6.3 is not met.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2  and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters for the wanted signal as specified in Table 7.7-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and in Table 7.7-1a for NR bands with FDL_high ≥ 3300 MHz and FUL_high ≥ 3300 MHz and for the interferer as specified in Table 7.7-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.7-1: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.7.1-1a: Spurious response parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

Table 7.7-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

## 7.7A	Spurious response for CA

### 7.7A.1	Spurious response for Intra-band contiguous CA

Table 7.7A-1: Spurious response parameters for intra-band contiguous CA

| RX parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |  |
|  | dB | 9 | 9 | 9 |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |

Table 7.7A-2: Spurious response for CA

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

Table 7.7A-3: Void

Table 7.7A-4: void

### 7.7A.2	Spurious response for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the spurious response requirements are defined with the uplink configuration in accordance with Table 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clauses 7.7 and 7.7A.1 for one component carrier and two component carriers per sub-block, respectively. The requirements apply with all downlink carriers active.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

### 7.7A.3	Spurious response for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the spurious response are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in clause 7.7 for each component carrier while all downlink carriers are active.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.1-1, Pinterferer power defined in Table 7.7-2 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.1-1.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

## 7.7B	Spurious response for NR-DC

For inter-band NR-DC configurations, the spurious response for the corresponding inter-band CA configuration as specified in clause 7.7B applies.

## 7.7D	Spurious response for UL MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in clause 7.7 shall be met with the UL MIMO configurations described in clause 6.2D.1. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

## 7.7E	Spurious response for V2X

### 7.7.E.1	General

Spurious response is a measure of the receiver’s ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in clause 7.6E.3 is not met.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters for the wanted signal as specified in Table 7.7E.1-1 and Table 7.7E.1-2 for NR V2X bands. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.7E.1-1: Spurious response parameters for NR V2X

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in transmission bandwidth configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |
|  | dB | 6 | 9 | 11 | 12 |
| NOTE 1: Reference measurement channel is A.7.2 |  |  |  |  |  |

Table 7.7E.1-2: Spurious response for NR V2X

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

### 7.7E.2	Spurious response for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.7E shall apply for the NR sidelink reception in Band n47 and the requirements specified in clause 7.7 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

## 7.7F	Spurious response for shared spectrum channel access

### 7.7F.1	General

For spurious responses, the throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.7F.1-1 and Table 7.7F.1-2. The relative throughput requirement shall be met for any SCS at any other frequency at which a response is obtained i.e. for which the limit as specified in clause 7.6F.3.1 is not met.

Table 7.7F.1-1: Spurious response parameters for shared access bands

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |
|  | dB | 9 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |

Table 7.7F.1-2: Spurious response for shared spectrum channel access

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

### 7.7F.2	Intra-band contiguous shared spectrum channel access CA

For spurious responses, the throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.7F.2-1 and Table 7.7F.2-2. The relative throughput requirement shall be met for any SCS at any other frequency at which a response is obtained i.e. for which the limit as specified in clause 7.6F.3.2 is not met.

Table 7.7F.2-1: Spurious response parameters for intra-band contiguous shared access CA

| Rx Parameter | Units | Shared access CA bandwidth class |
| --- | --- | --- |
|  |  | B, C, D, E, I, M, N,O |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA bandwidth class specific value below |
|  | dB | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |

Table 7.7F.2-2: Spurious response for intra-band contiguous shared access CA

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

## 7.8	Intermodulation characteristics

### 7.8.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal

### 7.8.2	Wide band Intermodulation

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8.2-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and Table 7.8.2-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.8.2-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter |  | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC |  | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 6 | 6 | 7 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 15 | 16 |
| PInterferer 1 (CW) |  | dBm |  | -46 |  |  |  |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) |  | dBm |  | -46 |  |  |  |  |  |  |  |  |  |  |  |
| BWInterferer 2 |  | MHz |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) |  | MHz |  | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |  |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) |  | MHz |  | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.8.2-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 |  |  |  |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |
| BWInterferer 2 | MHz | BW |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz | -2BW / +2BW |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |

## 7.8A	Intermodulation characteristics for CA

### 7.8A.1	General

### 7.8A.2	Wide band intermodulation for CA

#### 7.8A.2.1	Wide band intermodulation for Intra-band contiguous CA

Table 7.8A.2.1-1: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 10 | REFSENS + 6 | REFSENS + 13.8 |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |
| BWInterferer 2 | MHz | 20 | BWChannel_CA | 50 |  |
| FInterferer 1 (Offset) | MHz | -Foffset-30 / Foffset+30 | -2BWChannel_CA / +2BWChannel_CA | -Foffset-75 / Foffset+75 |  |
| FInterferer 2 (Offset) | MHz |  | 2*FInterferer 1 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the closest carrier. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |

Table 7.8A.2.1-2: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| Rx parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 16 | REFSENS + 19 |
| PInterferer 1 (CW) | dBm | -46 | -46 |
| PInterferer 2 (Modulated) | dBm | -46 | -46 |
| BWInterferer 2 | MHz | 5 | 5 |
| FInterferer 1 (Offset) | MHz | -Foffset-7.5 / Foffset+7.5 | -Foffset-7.5 / Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |

#### 7.8A.2.2	Wide band intermodulation for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the wide band intermodulation requirements are defined with the uplink configuration in accordance with Table 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clause 7.8.2 and 7.8A.2.1 for one component carrier and two component carriers per sub-block, respectively. The requirements apply for out-of-gap interferers while all downlink carriers are active.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

#### 7.8A.2.3	Wide band intermodulation for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in clause 7.8 for each component carrier while all downlink carriers are active.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.1-1, Pinterferer power defined in Table 7.8.2-1 and 7.8.2-2 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.1-1.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

## 7.8B	Intermodulation characteristics for NR-DC

For inter-band NR-DC configurations, the intermodulation characteristics for the corresponding inter-band CA configuration as specified in clause 7.8A applies.

## 7.8D	Intermodulation characteristics for UL MIMO

For UE(s) with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements in clause 7.8 shall be met with the UL MIMO configurations described in clause 6.2D.1. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

## 7.8E	Intermodulation characteristics for V2X

### 7.8E.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

### 7.8E.2	Wide band Intermodulation

The wide band intermodulation requirement is defined using modulated NR carrier and a CW signal as interferer 1 and interferer 2 respectively. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.8E.2-1 for NR V2X bands. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.8E.2-1: Wide band intermodulation parameters for NR V2X

| NR band | Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| n38, n47 | Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |
|  |  |  | 6 | 9 | 11 | 12 |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |
|  | BWInterferer 2 | MHz | 10MHz |  |  |  |
|  | FInterferer 1 (Offset) | MHz | -BW/2 – 15 / +BW/2 + 15 |  |  |  |
|  | FInterferer 2 (Offset) | MHz | 2 * FInterferer 1 |  |  |  |
| NOTE 1: Reference measurement channel is A.7.2 NOTE 2: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |  |  |  |  |

### 7.8E.3	Intermodulation for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 7.8E shall apply for the NR sidelink reception in Band n47 and the requirements specified in clause 7.8 shall apply for the NR downlink reception in licensed band while all downlink carriers are active.

## 7.8F	Intermodulation characteristics for shared spectrum channel access

### 7.8F.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal

### 7.8F.2	Wide band Intermodulation

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively.

Instead of the general wideband intermodulation requirements specified in clause 7.8.2, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8F.2-1. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.8F.2-1: Wide band intermodulation parameters for shared spectrum channel access

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + channel bandwidth specific value below |  |  |  |
|  |  | 9 | 12 | 13.8 | 15 |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |
| BWInterferer 2 | MHz | 20 |  |  |  |
| FInterferer 1 (Offset) | MHz | -BW/2 - 30 / +BW/2 + 30 |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |

## 7.9	Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9-1

Table 7.9-1: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30 MHz f < 1 GHz | 100 kHz | -57 dBm |  |
| 1 GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 2 |
| 12.75 GHz – 26 GHz | 1 MHz | -47 dBm | 3 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 2.69 GHz. NOTE 3: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. |  |  |  |

## 7.9A	Spurious emissions for CA

### 7.9A.1	Void

### 7.9A.2	Void

### 7.9A.3	Spurious emissions for Inter-band CA

For inter-band carrier aggregation including an operating band without uplink band, the UE shall meet the Rx spurious emissions requirements specified in clause 7.9 for each component carrier while all downlink carriers are active.

## 7.9B	Spurious emissions for NR-DC

For inter-band NR-DC configurations, the spurious emissions for the corresponding inter-band CA configuration as specified in clause 7.9A applies.

Annex A (normative):
Measurement channels

# A.1	General

The throughput values defined in the measurement channels specified in Annex A, are calculated and are valid per datastream (codeword). For multi-stream (more than one codeword) transmissions, the throughput referenced in the minimum requirements is the sum of throughputs of all datastreams (codewords).

The UE category entry in the definition of the reference measurement channel in Annex A is only informative and reveals the UE categories, which can support the corresponding measurement channel. Whether the measurement channel is used for testing a certain UE category or not is specified in the individual minimum requirements.

# A.2	UL reference measurement channels

## A.2.1	General

The measurement channels in the following clauses are defined to derive the requirements in clause 6 (Transmitter Characteristics) and clause 7 (Receiver Characteristics). The measurement channels represent example configurations of physical channels for different data rates.

## A.2.2	Reference measurement channels for FDD

### A.2.2.1	DFT-s-OFDM Pi/2-BPSK

Table A.2.2.1-1: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 5 | 15 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 5 | 15 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 10 | 15 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 10 | 15 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 15 | 15 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 15 | 15 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 20 | 15 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 20 | 15 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 25 | 15 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 25 | 15 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 30 | 15 | 80 | 11 | pi/2 BPSK | 0 | 1/4 | 2472 | 16 | 2 | 1 | 10560 | 10560 |
|  | 30 | 15 | 160 | 11 | pi/2 BPSK | 0 | 1/4 | 4872 | 24 | 2 | 2 | 21120 | 21120 |
|  | 40 | 15 | 108 | 11 | pi/2 BPSK | 0 | 1/4 | 3368 | 16 | 2 | 1 | 14256 | 14256 |
|  | 40 | 15 | 216 | 11 | pi/2 BPSK | 0 | 1/4 | 6664 | 24 | 2 | 2 | 28512 | 28512 |
|  | 50 | 15 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
|  | 50 | 15 | 270 | 11 | pi/2 BPSK | 0 | 1/4 | 8448 | 24 | 2 | 3 | 35640 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.1-2: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 30 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 5 | 30 | 5 | 11 | pi/2 BPSK | 0 | 1/4 | 160 | 16 | 2 | 1 | 660 | 660 |
|  | 5 | 30 | 10 | 11 | pi/2 BPSK | 0 | 1/4 | 320 | 16 | 2 | 1 | 1320 | 1320 |
|  | 10 | 30 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 10 | 30 | 24 | 11 | pi/2 BPSK | 0 | 1/4 | 768 | 16 | 2 | 1 | 3168 | 3168 |
|  | 15 | 30 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 15 | 30 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 20 | 30 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 20 | 30 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 25 | 30 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 25 | 30 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 30 | 30 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 30 | 30 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 40 | 30 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 40 | 30 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 50 | 30 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 50 | 30 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 60 | 30 | 81 | 11 | pi/2 BPSK | 0 | 1/4 | 2536 | 16 | 2 | 1 | 10692 | 10692 |
|  | 60 | 30 | 162 | 11 | pi/2 BPSK | 0 | 1/4 | 5000 | 24 | 2 | 2 | 21384 | 21384 |
|  | 80 | 30 | 108 | 11 | pi/2 BPSK | 0 | 1/4 | 3368 | 16 | 2 | 1 | 14256 | 14256 |
|  | 80 | 30 | 216 | 11 | pi/2 BPSK | 0 | 1/4 | 6664 | 24 | 2 | 2 | 28512 | 28512 |
|  | 90 | 30 | 120 | 11 | pi/2 BPSK | 0 | 1/4 | 3752 | 16 | 2 | 1 | 15840 | 15840 |
|  | 90 | 30 | 243 | 11 | pi/2 BPSK | 0 | 1/4 | 7560 | 24 | 2 | 2 | 32076 | 32076 |
|  | 100 | 30 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
|  | 100 | 30 | 270 | 11 | pi/2 BPSK | 0 | 1/4 | 8448 | 24 | 2 | 3 | 35640 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.1-3: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 10 | 60 | 5 | 11 | pi/2 BPSK | 0 | 1/4 | 160 | 16 | 2 | 1 | 660 | 660 |
|  | 10 | 60 | 10 | 11 | pi/2 BPSK | 0 | 1/4 | 320 | 16 | 2 | 1 | 1320 | 1320 |
|  | 15 | 60 | 9 | 11 | pi/2 BPSK | 0 | 1/4 | 288 | 16 | 2 | 1 | 1188 | 1188 |
|  | 15 | 60 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 20 | 60 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 20 | 60 | 24 | 11 | pi/2 BPSK | 0 | 1/4 | 768 | 16 | 2 | 1 | 3168 | 3168 |
|  | 25 | 60 | 15 | 11 | pi/2 BPSK | 0 | 1/4 | 480 | 16 | 2 | 1 | 1980 | 1980 |
|  | 25 | 60 | 30 | 11 | pi/2 BPSK | 0 | 1/4 | 984 | 16 | 2 | 1 | 3960 | 3960 |
|  | 30 | 60 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 30 | 60 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 40 | 60 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 40 | 60 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 50 | 60 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 50 | 60 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 60 | 60 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 60 | 60 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 80 | 60 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 80 | 60 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 90 | 60 | 60 | 11 | pi/2 BPSK | 0 | 1/4 | 1864 | 16 | 2 | 1 | 7920 | 7920 |
|  | 90 | 60 | 120 | 11 | pi/2 BPSK | 0 | 1/4 | 3752 | 16 | 2 | 1 | 15840 | 15840 |
|  | 100 | 60 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 100 | 60 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.2	DFT-s-OFDM QPSK

Table A.2.2.2-1: Reference Channels for DFT-s-OFDM QPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 5 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 10 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 15 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 15 | 15 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 15 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 20 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 20 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 20 | 15 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 25 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 15 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 25 | 15 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 30 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2-2: Reference Channels for DFT-s-OFDM QPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 5 | 30 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 15 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 20 | 30 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 30 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 25 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 30 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 30 | 30 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 40 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 40 | 30 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 50 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 50 | 30 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 80 | 30 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 80 | 30 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 90 | 30 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 90 | 30 | 243 | 11 | QPSK | 2 | 1/6 | 12040 | 24 | 2 | 4 | 64152 | 32076 |
|  | 100 | 30 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 100 | 30 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2-3: Reference Channels for DFT-s-OFDM QPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 10 | 60 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 15 | 11 | QPSK | 2 | 1/6 | 768 | 16 | 2 | 1 | 3960 | 1980 |
|  | 25 | 60 | 30 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 7920 | 3960 |
|  | 30 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 30 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 40 | 60 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 40 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 50 | 60 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 50 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 60 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 60 | 60 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 80 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 80 | 60 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 90 | 60 | 60 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 15840 | 7920 |
|  | 90 | 60 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 100 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.3	DFT-s-OFDM 16QAM

Table A.2.2.3-1: Reference Channels for DFT-s-OFDM 16QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 15 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 5 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 15 | 15 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 15 | 15 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 20 | 15 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 20 | 15 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 25 | 15 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 25 | 15 | 128 | 11 | 16QAM | 10 | 1/3 | 22536 | 24 | 1 | 3 | 67584 | 16896 |
|  | 30 | 15 | 80 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42240 | 10560 |
|  | 30 | 15 | 160 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 84480 | 21120 |
|  | 40 | 15 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 40 | 15 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 50 | 15 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 50 | 15 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.3-2: Reference Channels for DFT-s-OFDM 16QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 30 | 5 | 11 | 16QAM | 10 | 1/3 | 888 | 16 | 2 | 1 | 2640 | 660 |
|  | 5 | 30 | 10 | 11 | 16QAM | 10 | 1/3 | 1800 | 16 | 2 | 1 | 5280 | 1320 |
|  | 10 | 30 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 10 | 30 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 15 | 30 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 15 | 30 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 20 | 30 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 20 | 30 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 25 | 30 | 32 | 11 | 16QAM | 10 | 1/3 | 5632 | 24 | 1 | 1 | 16896 | 4224 |
|  | 25 | 30 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 30 | 30 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 30 | 30 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 40 | 30 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 40 | 30 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 50 | 30 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 50 | 30 | 128 | 11 | 16QAM | 10 | 1/3 | 22536 | 24 | 1 | 3 | 67584 | 16896 |
|  | 60 | 30 | 81 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42768 | 10692 |
|  | 60 | 30 | 162 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 85536 | 21384 |
|  | 80 | 30 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 80 | 30 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 90 | 30 | 120 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63360 | 15840 |
|  | 90 | 30 | 243 | 11 | 16QAM | 10 | 1/3 | 43032 | 24 | 1 | 6 | 128304 | 32076 |
|  | 100 | 30 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 100 | 30 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.3-3: Reference Channels for DFT-s-OFDM 16QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 10 | 60 | 5 | 11 | 16QAM | 10 | 1/3 | 888 | 16 | 2 | 1 | 2640 | 660 |
|  | 10 | 60 | 10 | 11 | 16QAM | 10 | 1/3 | 1800 | 16 | 2 | 1 | 5280 | 1320 |
|  | 15 | 60 | 9 | 11 | 16QAM | 10 | 1/3 | 1608 | 16 | 2 | 1 | 4752 | 1188 |
|  | 15 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 20 | 60 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 20 | 60 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 25 | 60 | 15 | 11 | 16QAM | 10 | 1/3 | 2664 | 16 | 2 | 1 | 7920 | 1980 |
|  | 25 | 60 | 30 | 11 | 16QAM | 10 | 1/3 | 5248 | 24 | 1 | 1 | 15840 | 3960 |
|  | 30 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 30 | 60 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 40 | 60 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 40 | 60 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 50 | 60 | 32 | 11 | 16QAM | 10 | 1/3 | 5632 | 24 | 1 | 1 | 16896 | 4224 |
|  | 50 | 60 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 60 | 60 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 60 | 60 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 80 | 60 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 80 | 60 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 90 | 60 | 60 | 11 | 16QAM | 10 | 1/3 | 10504 | 24 | 1 | 2 | 31680 | 7920 |
|  | 90 | 60 | 120 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63360 | 15840 |
|  | 100 | 60 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 100 | 60 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.4	DFT-s-OFDM 64QAM

Table A.2.2.4-1: Reference Channels for DFT-s-OFDM 64QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 64QAM | 18 | 1/2 | 9992 | 24 | 1 | 2 | 19800 | 3300 |
|  | 10 | 15 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 15 | 15 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 20 | 15 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 25 | 15 | 128 | 11 | 64QAM | 18 | 1/2 | 51216 | 24 | 1 | 7 | 101376 | 16896 |
|  | 30 | 15 | 160 | 11 | 64QAM | 18 | 1/2 | 63528 | 24 | 1 | 8 | 126720 | 21120 |
|  | 40 | 15 | 216 | 11 | 64QAM | 18 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 50 | 15 | 270 | 11 | 64QAM | 18 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.4-2: Reference Channels for DFT-s-OFDM 64QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 10 | 11 | 64QAM | 18 | 1/2 | 3968 | 24 | 1 | 1 | 7920 | 1320 |
|  | 10 | 30 | 24 | 11 | 64QAM | 18 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 15 | 30 | 36 | 11 | 64QAM | 18 | 1/2 | 14344 | 24 | 1 | 2 | 28512 | 4752 |
|  | 20 | 30 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 25 | 30 | 64 | 11 | 64QAM | 18 | 1/2 | 25608 | 24 | 1 | 4 | 50688 | 8448 |
|  | 30 | 30 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 40 | 30 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 50 | 30 | 128 | 11 | 64QAM | 18 | 1/2 | 51216 | 24 | 1 | 7 | 101376 | 16896 |
|  | 60 | 30 | 162 | 11 | 64QAM | 18 | 1/2 | 64552 | 24 | 1 | 8 | 128304 | 21384 |
|  | 80 | 30 | 216 | 11 | 64QAM | 18 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 90 | 30 | 243 | 11 | 64QAM | 18 | 1/2 | 96264 | 24 | 1 | 12 | 192456 | 32076 |
|  | 100 | 30 | 270 | 11 | 64QAM | 18 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.4-3: Reference Channels for DFT-s-OFDM 64QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 10 | 11 | 64QAM | 18 | 1/2 | 3968 | 24 | 1 | 1 | 7920 | 1320 |
|  | 15 | 60 | 18 | 11 | 64QAM | 18 | 1/2 | 7168 | 24 | 1 | 1 | 14256 | 2376 |
|  | 20 | 60 | 24 | 11 | 64QAM | 18 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 25 | 60 | 30 | 11 | 64QAM | 18 | 1/2 | 12040 | 24 | 1 | 2 | 23760 | 3960 |
|  | 30 | 60 | 36 | 11 | 64QAM | 18 | 1/2 | 14344 | 24 | 1 | 2 | 28512 | 4752 |
|  | 40 | 60 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 50 | 60 | 64 | 11 | 64QAM | 18 | 1/2 | 25608 | 24 | 1 | 4 | 50688 | 8448 |
|  | 60 | 60 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 80 | 60 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 90 | 60 | 120 | 11 | 64QAM | 18 | 1/2 | 48168 | 24 | 1 | 6 | 95040 | 15840 |
|  | 100 | 60 | 135 | 11 | 64QAM | 18 | 1/2 | 54296 | 24 | 1 | 7 | 106920 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.5	DFT-s-OFDM 256QAM

Table A.2.2.5-1: Reference Channels for DFT-s-OFDM 256QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 256QAM | 20 | 2/3 | 17424 | 24 | 1 | 3 | 26400 | 3300 |
|  | 10 | 15 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 15 | 15 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 20 | 15 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 25 | 15 | 128 | 11 | 256QAM | 20 | 2/3 | 90176 | 24 | 1 | 11 | 135168 | 16896 |
|  | 30 | 15 | 160 | 11 | 256QAM | 20 | 2/3 | 112648 | 24 | 1 | 14 | 168960 | 21120 |
|  | 40 | 15 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 50 | 15 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.5-2: Reference Channels for DFT-s-OFDM 256QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 10 | 11 | 256QAM | 20 | 2/3 | 7040 | 24 | 1 | 1 | 10560 | 1320 |
|  | 10 | 30 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 15 | 30 | 36 | 11 | 256QAM | 20 | 2/3 | 25104 | 24 | 1 | 3 | 38016 | 4752 |
|  | 20 | 30 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 25 | 30 | 64 | 11 | 256QAM | 20 | 2/3 | 45096 | 24 | 1 | 6 | 67584 | 8448 |
|  | 30 | 30 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 40 | 30 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 50 | 30 | 128 | 11 | 256QAM | 20 | 2/3 | 90176 | 24 | 1 | 11 | 135168 | 16896 |
|  | 60 | 30 | 162 | 11 | 256QAM | 20 | 2/3 | 114776 | 24 | 1 | 14 | 171072 | 21384 |
|  | 80 | 30 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 90 | 30 | 243 | 11 | 256QAM | 20 | 2/3 | 172176 | 24 | 1 | 21 | 256608 | 32076 |
|  | 100 | 30 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.5-3: Reference Channels for DFT-s-OFDM 256QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 10 | 11 | 256QAM | 20 | 2/3 | 7040 | 24 | 1 | 1 | 10560 | 1320 |
|  | 15 | 60 | 18 | 11 | 256QAM | 20 | 2/3 | 12552 | 24 | 1 | 2 | 19008 | 2376 |
|  | 20 | 60 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 25 | 60 | 30 | 11 | 256QAM | 20 | 2/3 | 21000 | 24 | 1 | 3 | 31680 | 3960 |
|  | 30 | 60 | 36 | 11 | 256QAM | 20 | 2/3 | 25104 | 24 | 1 | 3 | 38016 | 4752 |
|  | 40 | 60 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 50 | 60 | 64 | 11 | 256QAM | 20 | 2/3 | 45096 | 24 | 1 | 6 | 67584 | 8448 |
|  | 60 | 60 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 80 | 60 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 90 | 60 | 120 | 11 | 256QAM | 20 | 2/3 | 83976 | 24 | 1 | 10 | 126720 | 15840 |
|  | 100 | 60 | 135 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 142560 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.6	CP-OFDM QPSK

Table A.2.2.6-1: Reference Channels for CP-OFDM QPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 13 | 11 | QPSK | 2 | 1/6 | 672 | 16 | 2 | 1 | 3432 | 1716 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 10 | 15 | 52 | 11 | QPSK | 2 | 1/6 | 2600 | 16 | 2 | 1 | 13728 | 6864 |
|  | 15 | 15 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 15 | 15 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 20 | 15 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 20 | 15 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 25 | 15 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 25 | 15 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.6-2: Reference Channels for CP-OFDM QPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 5 | 30 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 15 | 30 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 20 | 30 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 20 | 30 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 25 | 30 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 25 | 30 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 30 | 30 | 39 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10296 | 5148 |
|  | 30 | 30 | 78 | 11 | QPSK | 2 | 1/6 | 3848 | 24 | 2 | 2 | 20592 | 10296 |
|  | 40 | 30 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 40 | 30 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 50 | 30 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 50 | 30 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 80 | 30 | 109 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28776 | 14388 |
|  | 80 | 30 | 217 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57288 | 28644 |
|  | 90 | 30 | 123 | 11 | QPSK | 2 | 1/6 | 6152 | 24 | 2 | 2 | 32472 | 16236 |
|  | 90 | 30 | 245 | 11 | QPSK | 2 | 1/6 | 12296 | 24 | 2 | 4 | 64680 | 32340 |
|  | 100 | 30 | 137 | 11 | QPSK | 2 | 1/6 | 6792 | 24 | 2 | 2 | 36168 | 18084 |
|  | 100 | 30 | 273 | 11 | QPSK | 2 | 1/6 | 13576 | 24 | 2 | 4 | 72072 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.6-3: Reference Channels for CP-OFDM QPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 10 | 60 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 16 | 11 | QPSK | 2 | 1/6 | 808 | 16 | 2 | 1 | 4224 | 2112 |
|  | 25 | 60 | 31 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 8184 | 4092 |
|  | 30 | 60 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 30 | 60 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 40 | 60 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 40 | 60 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 50 | 60 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 50 | 60 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 60 | 60 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 60 | 60 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 80 | 60 | 54 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 14256 | 7128 |
|  | 80 | 60 | 107 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 28248 | 14124 |
|  | 90 | 60 | 61 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 16104 | 8052 |
|  | 90 | 60 | 121 | 11 | QPSK | 2 | 1/6 | 6024 | 24 | 2 | 2 | 31944 | 15972 |
|  | 100 | 60 | 68 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17952 | 8976 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.7	CP-OFDM 16QAM

Table A.2.2.7-1: Reference Channels for CP-OFDM 16QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 15 | 13 | 11 | 16QAM | 10 | 1/3 | 2280 | 16 | 2 | 1 | 6864 | 1716 |
|  | 5 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 10 | 15 | 52 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27456 | 6864 |
|  | 15 | 15 | 40 | 11 | 16QAM | 10 | 1/3 | 7040 | 24 | 1 | 1 | 21120 | 5280 |
|  | 15 | 15 | 79 | 11 | 16QAM | 10 | 1/3 | 13832 | 24 | 1 | 2 | 41712 | 10428 |
|  | 20 | 15 | 53 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27984 | 6996 |
|  | 20 | 15 | 106 | 11 | 16QAM | 10 | 1/3 | 18432 | 24 | 1 | 3 | 55968 | 13992 |
|  | 25 | 15 | 67 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35376 | 8844 |
|  | 25 | 15 | 133 | 11 | 16QAM | 10 | 1/3 | 23040 | 24 | 1 | 3 | 70224 | 17556 |
|  | 30 | 15 | 80 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42240 | 10560 |
|  | 30 | 15 | 160 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 84480 | 21120 |
|  | 40 | 15 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 40 | 15 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 50 | 15 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 50 | 15 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.7-2: Reference Channels for CP-OFDM 16QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 30 | 6 | 11 | 16QAM | 10 | 1/3 | 1064 | 16 | 2 | 1 | 3168 | 792 |
|  | 5 | 30 | 11 | 11 | 16QAM | 10 | 1/3 | 1928 | 16 | 2 | 1 | 5808 | 1452 |
|  | 10 | 30 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 10 | 30 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 15 | 30 | 19 | 11 | 16QAM | 10 | 1/3 | 3368 | 16 | 2 | 1 | 10032 | 2508 |
|  | 15 | 30 | 38 | 11 | 16QAM | 10 | 1/3 | 6656 | 24 | 1 | 1 | 20064 | 5016 |
|  | 20 | 30 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 20 | 30 | 51 | 11 | 16QAM | 10 | 1/3 | 8968 | 24 | 1 | 2 | 26928 | 6732 |
|  | 25 | 30 | 33 | 11 | 16QAM | 10 | 1/3 | 5760 | 24 | 1 | 1 | 17424 | 4356 |
|  | 25 | 30 | 65 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 34320 | 8580 |
|  | 30 | 30 | 39 | 11 | 16QAM | 10 | 1/3 | 6784 | 24 | 1 | 1 | 20592 | 5148 |
|  | 30 | 30 | 78 | 11 | 16QAM | 10 | 1/3 | 13576 | 24 | 1 | 2 | 41184 | 10296 |
|  | 40 | 30 | 53 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27984 | 6996 |
|  | 40 | 30 | 106 | 11 | 16QAM | 10 | 1/3 | 18432 | 24 | 1 | 3 | 55968 | 13992 |
|  | 50 | 30 | 67 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35376 | 8844 |
|  | 50 | 30 | 133 | 11 | 16QAM | 10 | 1/3 | 23040 | 24 | 1 | 3 | 70224 | 17556 |
|  | 60 | 30 | 81 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42768 | 10692 |
|  | 60 | 30 | 162 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 85536 | 21384 |
|  | 80 | 30 | 109 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57552 | 14388 |
|  | 80 | 30 | 217 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114576 | 28644 |
|  | 90 | 30 | 123 | 11 | 16QAM | 10 | 1/3 | 21504 | 24 | 1 | 3 | 64944 | 16236 |
|  | 90 | 30 | 245 | 11 | 16QAM | 10 | 1/3 | 43032 | 24 | 1 | 6 | 129360 | 32340 |
|  | 100 | 30 | 137 | 11 | 16QAM | 10 | 1/3 | 24072 | 24 | 1 | 3 | 72336 | 18084 |
|  | 100 | 30 | 273 | 11 | 16QAM | 10 | 1/3 | 48168 | 24 | 1 | 6 | 144144 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.7-3: Reference Channels for CP-OFDM 16QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 10 | 60 | 6 | 11 | 16QAM | 10 | 1/3 | 1064 | 16 | 2 | 1 | 3168 | 792 |
|  | 10 | 60 | 11 | 11 | 16QAM | 10 | 1/3 | 1928 | 16 | 2 | 1 | 5808 | 1452 |
|  | 15 | 60 | 9 | 11 | 16QAM | 10 | 1/3 | 1608 | 16 | 2 | 1 | 4752 | 1188 |
|  | 15 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 20 | 60 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 20 | 60 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 25 | 60 | 16 | 11 | 16QAM | 10 | 1/3 | 2792 | 16 | 2 | 1 | 8448 | 2112 |
|  | 25 | 60 | 31 | 11 | 16QAM | 10 | 1/3 | 5376 | 24 | 1 | 1 | 16368 | 4092 |
|  | 30 | 60 | 19 | 11 | 16QAM | 10 | 1/3 | 3368 | 16 | 2 | 1 | 10032 | 2508 |
|  | 30 | 60 | 38 | 11 | 16QAM | 10 | 1/3 | 6656 | 24 | 1 | 1 | 20064 | 5016 |
|  | 40 | 60 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 40 | 60 | 51 | 11 | 16QAM | 10 | 1/3 | 8968 | 24 | 1 | 2 | 26928 | 6732 |
|  | 50 | 60 | 33 | 11 | 16QAM | 10 | 1/3 | 5760 | 24 | 1 | 1 | 17424 | 4356 |
|  | 50 | 60 | 65 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 34320 | 8580 |
|  | 60 | 60 | 40 | 11 | 16QAM | 10 | 1/3 | 7040 | 24 | 1 | 1 | 21120 | 5280 |
|  | 60 | 60 | 79 | 11 | 16QAM | 10 | 1/3 | 13832 | 24 | 1 | 2 | 41712 | 10428 |
|  | 80 | 60 | 54 | 11 | 16QAM | 10 | 1/3 | 9480 | 24 | 1 | 2 | 28512 | 7128 |
|  | 80 | 60 | 107 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 56496 | 14124 |
|  | 90 | 60 | 61 | 11 | 16QAM | 10 | 1/3 | 10760 | 24 | 1 | 2 | 32208 | 8052 |
|  | 90 | 60 | 121 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63888 | 15972 |
|  | 100 | 60 | 68 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35904 | 8976 |
|  | 100 | 60 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.8	CP-OFDM 64QAM

Table A.2.2.8-1: Reference Channels for CP-OFDM 64QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 64QAM | 19 | 1/2 | 9992 | 24 | 1 | 2 | 19800 | 3300 |
|  | 10 | 15 | 52 | 11 | 64QAM | 19 | 1/2 | 21000 | 24 | 1 | 3 | 41184 | 6864 |
|  | 15 | 15 | 79 | 11 | 64QAM | 19 | 1/2 | 31752 | 24 | 1 | 4 | 62568 | 10428 |
|  | 20 | 15 | 106 | 11 | 64QAM | 19 | 1/2 | 42016 | 24 | 1 | 5 | 83952 | 13992 |
|  | 25 | 15 | 133 | 11 | 64QAM | 19 | 1/2 | 53288 | 24 | 1 | 7 | 105336 | 17556 |
|  | 30 | 15 | 160 | 11 | 64QAM | 19 | 1/2 | 63528 | 24 | 1 | 8 | 126720 | 21120 |
|  | 40 | 15 | 216 | 11 | 64QAM | 19 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 50 | 15 | 270 | 11 | 64QAM | 19 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.8-2: Reference Channels for CP-OFDM 64QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 11 | 11 | 64QAM | 19 | 1/2 | 4352 | 24 | 1 | 1 | 8712 | 1452 |
|  | 10 | 30 | 24 | 11 | 64QAM | 19 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 15 | 30 | 38 | 11 | 64QAM | 19 | 1/2 | 15112 | 24 | 1 | 2 | 30096 | 5016 |
|  | 20 | 30 | 51 | 11 | 64QAM | 19 | 1/2 | 20496 | 24 | 1 | 3 | 40392 | 6732 |
|  | 25 | 30 | 65 | 11 | 64QAM | 19 | 1/2 | 26120 | 24 | 1 | 4 | 51480 | 8580 |
|  | 30 | 30 | 78 | 11 | 64QAM | 19 | 1/2 | 31240 | 24 | 1 | 4 | 61776 | 10296 |
|  | 40 | 30 | 106 | 11 | 64QAM | 19 | 1/2 | 42016 | 24 | 1 | 5 | 83952 | 13992 |
|  | 50 | 30 | 133 | 11 | 64QAM | 19 | 1/2 | 53288 | 24 | 1 | 7 | 105336 | 17556 |
|  | 60 | 30 | 162 | 11 | 64QAM | 19 | 1/2 | 64552 | 24 | 1 | 8 | 128304 | 21384 |
|  | 80 | 30 | 217 | 11 | 64QAM | 19 | 1/2 | 86040 | 24 | 1 | 11 | 171864 | 28644 |
|  | 90 | 30 | 245 | 11 | 64QAM | 19 | 1/2 | 98376 | 24 | 1 | 12 | 194040 | 32340 |
|  | 100 | 30 | 273 | 11 | 64QAM | 19 | 1/2 | 108552 | 24 | 1 | 13 | 216216 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.8-3: Reference Channels for CP-OFDM 64QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 11 | 11 | 64QAM | 19 | 1/2 | 4352 | 24 | 1 | 1 | 8712 | 1452 |
|  | 15 | 60 | 18 | 11 | 64QAM | 19 | 1/2 | 7168 | 24 | 1 | 1 | 14256 | 2376 |
|  | 20 | 60 | 24 | 11 | 64QAM | 19 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 25 | 60 | 31 | 11 | 64QAM | 19 | 1/2 | 12296 | 24 | 1 | 2 | 24552 | 4092 |
|  | 30 | 60 | 38 | 11 | 64QAM | 19 | 1/2 | 15112 | 24 | 1 | 2 | 30096 | 5016 |
|  | 40 | 60 | 51 | 11 | 64QAM | 19 | 1/2 | 20496 | 24 | 1 | 3 | 40392 | 6732 |
|  | 50 | 60 | 65 | 11 | 64QAM | 19 | 1/2 | 26120 | 24 | 1 | 4 | 51480 | 8580 |
|  | 60 | 60 | 79 | 11 | 64QAM | 19 | 1/2 | 31752 | 24 | 1 | 4 | 62568 | 10428 |
|  | 80 | 60 | 107 | 11 | 64QAM | 19 | 1/2 | 43032 | 24 | 1 | 6 | 84744 | 14124 |
|  | 90 | 60 | 121 | 11 | 64QAM | 19 | 1/2 | 48168 | 24 | 1 | 6 | 95832 | 15972 |
|  | 100 | 60 | 135 | 11 | 64QAM | 19 | 1/2 | 54296 | 24 | 1 | 7 | 106920 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.2.9	CP-OFDM 256QAM

Table A.2.2.9-1: Reference Channels for CP-OFDM 256QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 256QAM | 20 | 2/3 | 17424 | 24 | 1 | 3 | 26400 | 3300 |
|  | 10 | 15 | 52 | 11 | 256QAM | 20 | 2/3 | 36896 | 24 | 1 | 5 | 54912 | 6864 |
|  | 15 | 15 | 79 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 83424 | 10428 |
|  | 20 | 15 | 106 | 11 | 256QAM | 20 | 2/3 | 73776 | 24 | 1 | 9 | 111936 | 13992 |
|  | 25 | 15 | 133 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 140448 | 17556 |
|  | 30 | 15 | 160 | 11 | 256QAM | 20 | 2/3 | 112648 | 24 | 1 | 14 | 168960 | 21120 |
|  | 40 | 15 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 50 | 15 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.9-2: Reference Channels for CP-OFDM 256QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 11 | 11 | 256QAM | 20 | 2/3 | 7680 | 24 | 1 | 1 | 11616 | 1452 |
|  | 10 | 30 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 15 | 30 | 38 | 11 | 256QAM | 20 | 2/3 | 26632 | 24 | 1 | 4 | 40128 | 5016 |
|  | 20 | 30 | 51 | 11 | 256QAM | 20 | 2/3 | 35856 | 24 | 1 | 5 | 53856 | 6732 |
|  | 25 | 30 | 65 | 11 | 256QAM | 20 | 2/3 | 46104 | 24 | 1 | 6 | 68640 | 8580 |
|  | 30 | 30 | 78 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 82368 | 10296 |
|  | 40 | 30 | 106 | 11 | 256QAM | 20 | 2/3 | 73776 | 24 | 1 | 9 | 111936 | 13992 |
|  | 50 | 30 | 133 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 140448 | 17556 |
|  | 60 | 30 | 162 | 11 | 256QAM | 20 | 2/3 | 114776 | 24 | 1 | 14 | 171072 | 21384 |
|  | 80 | 30 | 217 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 229152 | 28644 |
|  | 90 | 30 | 245 | 11 | 256QAM | 20 | 2/3 | 172176 | 24 | 1 | 21 | 258720 | 32340 |
|  | 100 | 30 | 273 | 11 | 256QAM | 20 | 2/3 | 192624 | 24 | 1 | 23 | 288288 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.9-3: Reference Channels for CP-OFDM 256QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 11 | 11 | 256QAM | 20 | 2/3 | 7680 | 24 | 1 | 1 | 11616 | 1452 |
|  | 15 | 60 | 18 | 11 | 256QAM | 20 | 2/3 | 12552 | 24 | 1 | 2 | 19008 | 2376 |
|  | 20 | 60 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 25 | 60 | 31 | 11 | 256QAM | 20 | 2/3 | 22032 | 24 | 1 | 3 | 32736 | 4092 |
|  | 30 | 60 | 38 | 11 | 256QAM | 20 | 2/3 | 26632 | 24 | 1 | 4 | 40128 | 5016 |
|  | 40 | 60 | 51 | 11 | 256QAM | 20 | 2/3 | 35856 | 24 | 1 | 5 | 53856 | 6732 |
|  | 50 | 60 | 65 | 11 | 256QAM | 20 | 2/3 | 46104 | 24 | 1 | 6 | 68640 | 8580 |
|  | 60 | 60 | 79 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 83424 | 10428 |
|  | 80 | 60 | 107 | 11 | 256QAM | 20 | 2/3 | 75792 | 24 | 1 | 9 | 112992 | 14124 |
|  | 90 | 60 | 121 | 11 | 256QAM | 20 | 2/3 | 86040 | 24 | 1 | 11 | 127776 | 15972 |
|  | 100 | 60 | 135 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 142560 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

## A.2.3	Reference measurement channels for TDD

TDD slot patterns defined for reference sensitivity tests will be used for UL RMCs defined below.

### A.2.3.1	DFT-s-OFDM Pi/2-BPSK

Table A.2.3.1-1: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 5 | 15 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 5 | 15 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 10 | 15 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 10 | 15 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 15 | 15 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 15 | 15 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 20 | 15 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 20 | 15 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 25 | 15 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 25 | 15 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 30 | 15 | 80 | 11 | pi/2 BPSK | 0 | 1/4 | 2472 | 16 | 2 | 1 | 10560 | 10560 |
|  | 30 | 15 | 160 | 11 | pi/2 BPSK | 0 | 1/4 | 4872 | 24 | 2 | 2 | 21120 | 21120 |
|  | 40 | 15 | 108 | 11 | pi/2 BPSK | 0 | 1/4 | 3368 | 16 | 2 | 1 | 14256 | 14256 |
|  | 40 | 15 | 216 | 11 | pi/2 BPSK | 0 | 1/4 | 6664 | 24 | 2 | 2 | 28512 | 28512 |
|  | 50 | 15 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
|  | 50 | 15 | 270 | 11 | pi/2 BPSK | 0 | 1/4 | 8448 | 24 | 2 | 3 | 35640 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.1-2: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 5 | 30 | 5 | 11 | pi/2 BPSK | 0 | 1/4 | 160 | 16 | 2 | 1 | 660 | 660 |
|  | 5 | 30 | 10 | 11 | pi/2 BPSK | 0 | 1/4 | 320 | 16 | 2 | 1 | 1320 | 1320 |
|  | 10 | 30 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 10 | 30 | 24 | 11 | pi/2 BPSK | 0 | 1/4 | 768 | 16 | 2 | 1 | 3168 | 3168 |
|  | 15 | 30 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 15 | 30 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 20 | 30 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 20 | 30 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 25 | 30 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 25 | 30 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 30 | 30 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 30 | 30 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 40 | 30 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 40 | 30 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 50 | 30 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 50 | 30 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 60 | 30 | 81 | 11 | pi/2 BPSK | 0 | 1/4 | 2536 | 16 | 2 | 1 | 10692 | 10692 |
|  | 60 | 30 | 162 | 11 | pi/2 BPSK | 0 | 1/4 | 5000 | 24 | 2 | 2 | 21384 | 21384 |
|  | 70 | 30 | 90 | 11 | pi/2 BPSK | 0 | 1/4 | 2792 | 16 | 2 | 1 | 11880 | 11880 |
|  | 70 | 30 | 180 | 11 | pi/2 BPSK | 0 | 1/4 | 5512 | 24 | 2 | 2 | 23760 | 23760 |
|  | 80 | 30 | 108 | 11 | pi/2 BPSK | 0 | 1/4 | 3368 | 16 | 2 | 1 | 14256 | 14256 |
|  | 80 | 30 | 216 | 11 | pi/2 BPSK | 0 | 1/4 | 6664 | 24 | 2 | 2 | 28512 | 28512 |
|  | 90 | 30 | 120 | 11 | pi/2 BPSK | 0 | 1/4 | 3752 | 16 | 2 | 1 | 15840 | 15840 |
|  | 90 | 30 | 243 | 11 | pi/2 BPSK | 0 | 1/4 | 7560 | 24 | 2 | 2 | 32076 | 32076 |
|  | 100 | 30 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
|  | 100 | 30 | 270 | 11 | pi/2 BPSK | 0 | 1/4 | 8448 | 24 | 2 | 3 | 35640 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.1-3: Reference Channels for DFT-s-OFDM Pi/2-BPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 10 | 60 | 5 | 11 | pi/2 BPSK | 0 | 1/4 | 160 | 16 | 2 | 1 | 660 | 660 |
|  | 10 | 60 | 10 | 11 | pi/2 BPSK | 0 | 1/4 | 320 | 16 | 2 | 1 | 1320 | 1320 |
|  | 15 | 60 | 9 | 11 | pi/2 BPSK | 0 | 1/4 | 288 | 16 | 2 | 1 | 1188 | 1188 |
|  | 15 | 60 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 20 | 60 | 12 | 11 | pi/2 BPSK | 0 | 1/4 | 384 | 16 | 2 | 1 | 1584 | 1584 |
|  | 20 | 60 | 24 | 11 | pi/2 BPSK | 0 | 1/4 | 768 | 16 | 2 | 1 | 3168 | 3168 |
|  | 25 | 60 | 15 | 11 | pi/2 BPSK | 0 | 1/4 | 480 | 16 | 2 | 1 | 1980 | 1980 |
|  | 25 | 60 | 30 | 11 | pi/2 BPSK | 0 | 1/4 | 984 | 16 | 2 | 1 | 3960 | 3960 |
|  | 30 | 60 | 18 | 11 | pi/2 BPSK | 0 | 1/4 | 576 | 16 | 2 | 1 | 2376 | 2376 |
|  | 30 | 60 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 40 | 60 | 25 | 11 | pi/2 BPSK | 0 | 1/4 | 808 | 16 | 2 | 1 | 3300 | 3300 |
|  | 40 | 60 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 50 | 60 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 50 | 60 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 60 | 60 | 36 | 11 | pi/2 BPSK | 0 | 1/4 | 1128 | 16 | 2 | 1 | 4752 | 4752 |
|  | 60 | 60 | 75 | 11 | pi/2 BPSK | 0 | 1/4 | 2408 | 16 | 2 | 1 | 9900 | 9900 |
|  | 70 | 60 | 45 | 11 | pi/2 BPSK | 0 | 1/4 | 1416 | 16 | 2 | 1 | 5940 | 5940 |
|  | 70 | 60 | 90 | 11 | pi/2 BPSK | 0 | 1/4 | 2792 | 16 | 2 | 1 | 11880 | 11880 |
|  | 80 | 60 | 50 | 11 | pi/2 BPSK | 0 | 1/4 | 1544 | 16 | 2 | 1 | 6600 | 6600 |
|  | 80 | 60 | 100 | 11 | pi/2 BPSK | 0 | 1/4 | 3104 | 16 | 2 | 1 | 13200 | 13200 |
|  | 90 | 60 | 60 | 11 | pi/2 BPSK | 0 | 1/4 | 1864 | 16 | 2 | 1 | 7920 | 7920 |
|  | 90 | 60 | 120 | 11 | pi/2 BPSK | 0 | 1/4 | 3752 | 16 | 2 | 1 | 15840 | 15840 |
|  | 100 | 60 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 100 | 60 | 135 | 11 | pi/2 BPSK | 0 | 1/4 | 4104 | 24 | 2 | 2 | 17820 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.2	DFT-s-OFDM QPSK

Table A.2.3.2-1: Reference channels for DFT-s-OFDM QPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 5 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 10 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 15 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 15 | 15 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 15 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 20 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 20 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 20 | 15 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 25 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 15 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 25 | 15 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 30 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2-2: Reference channels for DFT-s-OFDM QPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 5 | 30 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 15 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 20 | 30 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 30 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 25 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 30 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 30 | 30 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 40 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 40 | 30 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 50 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 50 | 30 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 70 | 30 | 90 | 11 | QPSK | 2 | 1/6 | 4488 | 24 | 2 | 2 | 23760 | 11880 |
|  | 70 | 30 | 180 | 11 | QPSK | 2 | 1/6 | 8976 | 24 | 2 | 3 | 47520 | 23760 |
|  | 80 | 30 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 80 | 30 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 90 | 30 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 90 | 30 | 243 | 11 | QPSK | 2 | 1/6 | 12040 | 24 | 2 | 4 | 64152 | 32076 |
|  | 100 | 30 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 100 | 30 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2-3: Reference channels for DFT-s-OFDM QPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 10 | 60 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 15 | 11 | QPSK | 2 | 1/6 | 768 | 16 | 2 | 1 | 3960 | 1980 |
|  | 25 | 60 | 30 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 7920 | 3960 |
|  | 30 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 30 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 40 | 60 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 40 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 50 | 60 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 50 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 60 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 60 | 60 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 70 | 60 | 45 | 11 | QPSK | 2 | 1/6 | 2208 | 16 | 2 | 1 | 11880 | 5940 |
|  | 70 | 60 | 90 | 11 | QPSK | 2 | 1/6 | 4488 | 24 | 2 | 2 | 23760 | 11880 |
|  | 80 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 80 | 60 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 90 | 60 | 60 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 15840 | 7920 |
|  | 90 | 60 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 100 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.3	DFT-s-OFDM 16QAM

Table A.2.3.3-1: Reference channels for DFT-s-OFDM 16QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 15 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 5 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 15 | 15 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 15 | 15 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 20 | 15 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 20 | 15 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 25 | 15 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 25 | 15 | 128 | 11 | 16QAM | 10 | 1/3 | 22536 | 24 | 1 | 3 | 67584 | 16896 |
|  | 30 | 15 | 80 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42240 | 10560 |
|  | 30 | 15 | 160 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 84480 | 21120 |
|  | 40 | 15 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 40 | 15 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 50 | 15 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 50 | 15 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.3-2: Reference channels for DFT-s-OFDM 16QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 30 | 5 | 11 | 16QAM | 10 | 1/3 | 888 | 16 | 2 | 1 | 2640 | 660 |
|  | 5 | 30 | 10 | 11 | 16QAM | 10 | 1/3 | 1800 | 16 | 2 | 1 | 5280 | 1320 |
|  | 10 | 30 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 10 | 30 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 15 | 30 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 15 | 30 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 20 | 30 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 20 | 30 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 25 | 30 | 32 | 11 | 16QAM | 10 | 1/3 | 5632 | 24 | 1 | 1 | 16896 | 4224 |
|  | 25 | 30 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 30 | 30 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 30 | 30 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 40 | 30 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 40 | 30 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 50 | 30 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 50 | 30 | 128 | 11 | 16QAM | 10 | 1/3 | 22536 | 24 | 1 | 3 | 67584 | 16896 |
|  | 60 | 30 | 81 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42768 | 10692 |
|  | 60 | 30 | 162 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 85536 | 21384 |
|  | 70 | 30 | 90 | 11 | 16QAM | 10 | 1/3 | 15880 | 24 | 1 | 2 | 47520 | 11880 |
|  | 70 | 30 | 180 | 11 | 16QAM | 10 | 1/3 | 31752 | 24 | 1 | 4 | 95040 | 23760 |
|  | 80 | 30 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 80 | 30 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 90 | 30 | 120 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63360 | 15840 |
|  | 90 | 30 | 243 | 11 | 16QAM | 10 | 1/3 | 43032 | 24 | 1 | 6 | 128304 | 32076 |
|  | 100 | 30 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 100 | 30 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.3-3: Reference channels for DFT-s-OFDM 16QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 10 | 60 | 5 | 11 | 16QAM | 10 | 1/3 | 888 | 16 | 2 | 1 | 2640 | 660 |
|  | 10 | 60 | 10 | 11 | 16QAM | 10 | 1/3 | 1800 | 16 | 2 | 1 | 5280 | 1320 |
|  | 15 | 60 | 9 | 11 | 16QAM | 10 | 1/3 | 1608 | 16 | 2 | 1 | 4752 | 1188 |
|  | 15 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 20 | 60 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 20 | 60 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 25 | 60 | 15 | 11 | 16QAM | 10 | 1/3 | 2664 | 16 | 2 | 1 | 7920 | 1980 |
|  | 25 | 60 | 30 | 11 | 16QAM | 10 | 1/3 | 5248 | 24 | 1 | 1 | 15840 | 3960 |
|  | 30 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 30 | 60 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 40 | 60 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 40 | 60 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 50 | 60 | 32 | 11 | 16QAM | 10 | 1/3 | 5632 | 24 | 1 | 1 | 16896 | 4224 |
|  | 50 | 60 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 60 | 60 | 36 | 11 | 16QAM | 10 | 1/3 | 6272 | 24 | 1 | 1 | 19008 | 4752 |
|  | 60 | 60 | 75 | 11 | 16QAM | 10 | 1/3 | 13064 | 24 | 1 | 2 | 39600 | 9900 |
|  | 70 | 60 | 45 | 11 | 16QAM | 10 | 1/3 | 7808 | 24 | 1 | 1 | 23760 | 5940 |
|  | 70 | 60 | 90 | 11 | 16QAM | 10 | 1/3 | 15880 | 24 | 1 | 2 | 47520 | 11880 |
|  | 80 | 60 | 50 | 11 | 16QAM | 10 | 1/3 | 8712 | 24 | 1 | 2 | 26400 | 6600 |
|  | 80 | 60 | 100 | 11 | 16QAM | 10 | 1/3 | 17424 | 24 | 1 | 3 | 52800 | 13200 |
|  | 90 | 60 | 60 | 11 | 16QAM | 10 | 1/3 | 10504 | 24 | 1 | 2 | 31680 | 7920 |
|  | 90 | 60 | 120 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63360 | 15840 |
|  | 100 | 60 | 64 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 33792 | 8448 |
|  | 100 | 60 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.4	DFT-s-OFDM 64QAM

Table A.2.3.4-1: Reference channels for DFT-s-OFDM 64QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 64QAM | 18 | 1/2 | 9992 | 24 | 1 | 2 | 19800 | 3300 |
|  | 10 | 15 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 15 | 15 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 20 | 15 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 25 | 15 | 128 | 11 | 64QAM | 18 | 1/2 | 51216 | 24 | 1 | 7 | 101376 | 16896 |
|  | 30 | 15 | 160 | 11 | 64QAM | 18 | 1/2 | 63528 | 24 | 1 | 8 | 126720 | 21120 |
|  | 40 | 15 | 216 | 11 | 64QAM | 18 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 50 | 15 | 270 | 11 | 64QAM | 18 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.4-2: Reference channels for DFT-s-OFDM 64QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 10 | 11 | 64QAM | 18 | 1/2 | 3968 | 24 | 1 | 1 | 7920 | 1320 |
|  | 10 | 30 | 24 | 11 | 64QAM | 18 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 15 | 30 | 36 | 11 | 64QAM | 18 | 1/2 | 14344 | 24 | 1 | 2 | 28512 | 4752 |
|  | 20 | 30 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 25 | 30 | 64 | 11 | 64QAM | 18 | 1/2 | 25608 | 24 | 1 | 4 | 50688 | 8448 |
|  | 30 | 30 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 40 | 30 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 50 | 30 | 128 | 11 | 64QAM | 18 | 1/2 | 51216 | 24 | 1 | 7 | 101376 | 16896 |
|  | 60 | 30 | 162 | 11 | 64QAM | 18 | 1/2 | 64552 | 24 | 1 | 8 | 128304 | 21384 |
|  | 70 | 30 | 180 | 11 | 64QAM | 18 | 1/2 | 71688 | 24 | 1 | 9 | 142560 | 23760 |
|  | 80 | 30 | 216 | 11 | 64QAM | 18 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 90 | 30 | 243 | 11 | 64QAM | 18 | 1/2 | 96264 | 24 | 1 | 12 | 192456 | 32076 |
|  | 100 | 30 | 270 | 11 | 64QAM | 18 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.4-3: Reference channels for DFT-s-OFDM 64QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 10 | 11 | 64QAM | 18 | 1/2 | 3968 | 24 | 1 | 1 | 7920 | 1320 |
|  | 15 | 60 | 18 | 11 | 64QAM | 18 | 1/2 | 7168 | 24 | 1 | 1 | 14256 | 2376 |
|  | 20 | 60 | 24 | 11 | 64QAM | 18 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 25 | 60 | 30 | 11 | 64QAM | 18 | 1/2 | 12040 | 24 | 1 | 2 | 23760 | 3960 |
|  | 30 | 60 | 36 | 11 | 64QAM | 18 | 1/2 | 14344 | 24 | 1 | 2 | 28512 | 4752 |
|  | 40 | 60 | 50 | 11 | 64QAM | 18 | 1/2 | 19968 | 24 | 1 | 3 | 39600 | 6600 |
|  | 50 | 60 | 64 | 11 | 64QAM | 18 | 1/2 | 25608 | 24 | 1 | 4 | 50688 | 8448 |
|  | 60 | 60 | 75 | 11 | 64QAM | 18 | 1/2 | 30216 | 24 | 1 | 4 | 59400 | 9900 |
|  | 70 | 60 | 90 | 11 | 64QAM | 18 | 1/2 | 35856 | 24 | 1 | 5 | 71280 | 11880 |
|  | 80 | 60 | 100 | 11 | 64QAM | 18 | 1/2 | 39936 | 24 | 1 | 5 | 79200 | 13200 |
|  | 90 | 60 | 120 | 11 | 64QAM | 18 | 1/2 | 48168 | 24 | 1 | 6 | 95040 | 15840 |
|  | 100 | 60 | 135 | 11 | 64QAM | 18 | 1/2 | 54296 | 24 | 1 | 7 | 106920 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.5	DFT-s-OFDM 256QAM

Table A.2.3.5-1: Reference channels for DFT-s-OFDM 256QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 256QAM | 20 | 2/3 | 17424 | 24 | 1 | 3 | 26400 | 3300 |
|  | 10 | 15 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 15 | 15 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 20 | 15 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 25 | 15 | 128 | 11 | 256QAM | 20 | 2/3 | 90176 | 24 | 1 | 11 | 135168 | 16896 |
|  | 30 | 15 | 160 | 11 | 256QAM | 20 | 2/3 | 112648 | 24 | 1 | 14 | 168960 | 21120 |
|  | 40 | 15 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 50 | 15 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.5-2: Reference channels for DFT-s-OFDM 256QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 10 | 11 | 256QAM | 20 | 2/3 | 7040 | 24 | 1 | 1 | 10560 | 1320 |
|  | 10 | 30 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 15 | 30 | 36 | 11 | 256QAM | 20 | 2/3 | 25104 | 24 | 1 | 3 | 38016 | 4752 |
|  | 20 | 30 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 25 | 30 | 64 | 11 | 256QAM | 20 | 2/3 | 45096 | 24 | 1 | 6 | 67584 | 8448 |
|  | 30 | 30 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 40 | 30 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 50 | 30 | 128 | 11 | 256QAM | 20 | 2/3 | 90176 | 24 | 1 | 11 | 135168 | 16896 |
|  | 60 | 30 | 162 | 11 | 256QAM | 20 | 2/3 | 114776 | 24 | 1 | 14 | 171072 | 21384 |
|  | 70 | 30 | 180 | 11 | 256QAM | 20 | 2/3 | 127080 | 24 | 1 | 16 | 190080 | 23760 |
|  | 80 | 30 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 90 | 30 | 243 | 11 | 256QAM | 20 | 2/3 | 172176 | 24 | 1 | 21 | 256608 | 32076 |
|  | 100 | 30 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.5-3: Reference channels for DFT-s-OFDM 256QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 10 | 11 | 256QAM | 20 | 2/3 | 7040 | 24 | 1 | 1 | 10560 | 1320 |
|  | 15 | 60 | 18 | 11 | 256QAM | 20 | 2/3 | 12552 | 24 | 1 | 2 | 19008 | 2376 |
|  | 20 | 60 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 25 | 60 | 30 | 11 | 256QAM | 20 | 2/3 | 21000 | 24 | 1 | 3 | 31680 | 3960 |
|  | 30 | 60 | 36 | 11 | 256QAM | 20 | 2/3 | 25104 | 24 | 1 | 3 | 38016 | 4752 |
|  | 40 | 60 | 50 | 11 | 256QAM | 20 | 2/3 | 34816 | 24 | 1 | 5 | 52800 | 6600 |
|  | 50 | 60 | 64 | 11 | 256QAM | 20 | 2/3 | 45096 | 24 | 1 | 6 | 67584 | 8448 |
|  | 60 | 60 | 75 | 11 | 256QAM | 20 | 2/3 | 53288 | 24 | 1 | 7 | 79200 | 9900 |
|  | 70 | 60 | 90 | 11 | 256QAM | 20 | 2/3 | 63528 | 24 | 1 | 8 | 95040 | 11880 |
|  | 80 | 60 | 100 | 11 | 256QAM | 20 | 2/3 | 69672 | 24 | 1 | 9 | 105600 | 13200 |
|  | 90 | 60 | 120 | 11 | 256QAM | 20 | 2/3 | 83976 | 24 | 1 | 10 | 126720 | 15840 |
|  | 100 | 60 | 135 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 142560 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.6	CP-OFDM QPSK

Table A.2.3.6-1: Reference channels for CP-OFDM QPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 13 | 11 | QPSK | 2 | 1/6 | 672 | 16 | 2 | 1 | 3432 | 1716 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 10 | 15 | 52 | 11 | QPSK | 2 | 1/6 | 2600 | 16 | 2 | 1 | 13728 | 6864 |
|  | 15 | 15 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 15 | 15 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 20 | 15 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 20 | 15 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 25 | 15 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 25 | 15 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.6-2: Reference channels for CP-OFDM QPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 5 | 30 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 15 | 30 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 20 | 30 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 20 | 30 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 25 | 30 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 25 | 30 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 30 | 30 | 39 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10296 | 5148 |
|  | 30 | 30 | 78 | 11 | QPSK | 2 | 1/6 | 3848 | 24 | 2 | 2 | 20592 | 10296 |
|  | 40 | 30 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 40 | 30 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 50 | 30 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 50 | 30 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 70 | 30 | 95 | 11 | QPSK | 2 | 1/6 | 4744 | 24 | 2 | 2 | 25080 | 12540 |
|  | 70 | 30 | 189 | 11 | QPSK | 2 | 1/6 | 9480 | 24 | 2 | 3 | 49896 | 24948 |
|  | 80 | 30 | 109 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28776 | 14388 |
|  | 80 | 30 | 217 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57288 | 28644 |
|  | 90 | 30 | 123 | 11 | QPSK | 2 | 1/6 | 6152 | 24 | 2 | 2 | 32472 | 16236 |
|  | 90 | 30 | 245 | 11 | QPSK | 2 | 1/6 | 12296 | 24 | 2 | 4 | 64680 | 32340 |
|  | 100 | 30 | 137 | 11 | QPSK | 2 | 1/6 | 6792 | 24 | 2 | 2 | 36168 | 18084 |
|  | 100 | 30 | 273 | 11 | QPSK | 2 | 1/6 | 13576 | 24 | 2 | 4 | 72072 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.6-3: Reference channels for CP-OFDM QPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 10 | 60 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 16 | 11 | QPSK | 2 | 1/6 | 808 | 16 | 2 | 1 | 4224 | 2112 |
|  | 25 | 60 | 31 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 8184 | 4092 |
|  | 30 | 60 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 30 | 60 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 40 | 60 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 40 | 60 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 50 | 60 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 50 | 60 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 60 | 60 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 60 | 60 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 70 | 60 | 47 | 11 | QPSK | 2 | 1/6 | 2408 | 16 | 2 | 1 | 12408 | 6204 |
|  | 70 | 60 | 93 | 11 | QPSK | 2 | 1/6 | 4616 | 24 | 2 | 2 | 24552 | 12276 |
|  | 80 | 60 | 54 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 14256 | 7128 |
|  | 80 | 60 | 107 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 28248 | 14124 |
|  | 90 | 60 | 61 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 16104 | 8052 |
|  | 90 | 60 | 121 | 11 | QPSK | 2 | 1/6 | 6024 | 24 | 2 | 2 | 31944 | 15972 |
|  | 100 | 60 | 68 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17952 | 8976 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.7	CP-OFDM 16QAM

Table A.2.3.7-1: Reference channels for CP-OFDM 16QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 15 | 13 | 11 | 16QAM | 10 | 1/3 | 2280 | 16 | 2 | 1 | 6864 | 1716 |
|  | 5 | 15 | 25 | 11 | 16QAM | 10 | 1/3 | 4352 | 24 | 1 | 1 | 13200 | 3300 |
|  | 10 | 15 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 10 | 15 | 52 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27456 | 6864 |
|  | 15 | 15 | 40 | 11 | 16QAM | 10 | 1/3 | 7040 | 24 | 1 | 1 | 21120 | 5280 |
|  | 15 | 15 | 79 | 11 | 16QAM | 10 | 1/3 | 13832 | 24 | 1 | 2 | 41712 | 10428 |
|  | 20 | 15 | 53 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27984 | 6996 |
|  | 20 | 15 | 106 | 11 | 16QAM | 10 | 1/3 | 18432 | 24 | 1 | 3 | 55968 | 13992 |
|  | 25 | 15 | 67 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35376 | 8844 |
|  | 25 | 15 | 133 | 11 | 16QAM | 10 | 1/3 | 23040 | 24 | 1 | 3 | 70224 | 17556 |
|  | 30 | 15 | 80 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42240 | 10560 |
|  | 30 | 15 | 160 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 84480 | 21120 |
|  | 40 | 15 | 108 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57024 | 14256 |
|  | 40 | 15 | 216 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114048 | 28512 |
|  | 50 | 15 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
|  | 50 | 15 | 270 | 11 | 16QAM | 10 | 1/3 | 47112 | 24 | 1 | 6 | 142560 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.7-2: Reference channels for CP-OFDM 16QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 5 | 30 | 6 | 11 | 16QAM | 10 | 1/3 | 1064 | 16 | 2 | 1 | 3168 | 792 |
|  | 5 | 30 | 11 | 11 | 16QAM | 10 | 1/3 | 1928 | 16 | 2 | 1 | 5808 | 1452 |
|  | 10 | 30 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 10 | 30 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 15 | 30 | 19 | 11 | 16QAM | 10 | 1/3 | 3368 | 16 | 2 | 1 | 10032 | 2508 |
|  | 15 | 30 | 38 | 11 | 16QAM | 10 | 1/3 | 6656 | 24 | 1 | 1 | 20064 | 5016 |
|  | 20 | 30 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 20 | 30 | 51 | 11 | 16QAM | 10 | 1/3 | 8968 | 24 | 1 | 2 | 26928 | 6732 |
|  | 25 | 30 | 33 | 11 | 16QAM | 10 | 1/3 | 5760 | 24 | 1 | 1 | 17424 | 4356 |
|  | 25 | 30 | 65 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 34320 | 8580 |
|  | 30 | 30 | 39 | 11 | 16QAM | 10 | 1/3 | 6784 | 24 | 1 | 1 | 20592 | 5148 |
|  | 30 | 30 | 78 | 11 | 16QAM | 10 | 1/3 | 13576 | 24 | 1 | 2 | 41184 | 10296 |
|  | 40 | 30 | 53 | 11 | 16QAM | 10 | 1/3 | 9224 | 24 | 1 | 2 | 27984 | 6996 |
|  | 40 | 30 | 106 | 11 | 16QAM | 10 | 1/3 | 18432 | 24 | 1 | 3 | 55968 | 13992 |
|  | 50 | 30 | 67 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35376 | 8844 |
|  | 50 | 30 | 133 | 11 | 16QAM | 10 | 1/3 | 23040 | 24 | 1 | 3 | 70224 | 17556 |
|  | 60 | 30 | 81 | 11 | 16QAM | 10 | 1/3 | 14088 | 24 | 1 | 2 | 42768 | 10692 |
|  | 60 | 30 | 162 | 11 | 16QAM | 10 | 1/3 | 28168 | 24 | 1 | 4 | 85536 | 21384 |
|  | 70 | 30 | 95 | 11 | 16QAM | 10 | 1/3 | 16392 | 24 | 1 | 2 | 50160 | 12540 |
|  | 70 | 30 | 189 | 11 | 16QAM | 10 | 1/3 | 32776 | 24 | 1 | 4 | 99792 | 24948 |
|  | 80 | 30 | 109 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 57552 | 14388 |
|  | 80 | 30 | 217 | 11 | 16QAM | 10 | 1/3 | 37896 | 24 | 1 | 5 | 114576 | 28644 |
|  | 90 | 30 | 123 | 11 | 16QAM | 10 | 1/3 | 21504 | 24 | 1 | 3 | 64944 | 16236 |
|  | 90 | 30 | 245 | 11 | 16QAM | 10 | 1/3 | 43032 | 24 | 1 | 6 | 129360 | 32340 |
|  | 100 | 30 | 137 | 11 | 16QAM | 10 | 1/3 | 24072 | 24 | 1 | 3 | 72336 | 18084 |
|  | 100 | 30 | 273 | 11 | 16QAM | 10 | 1/3 | 48168 | 24 | 1 | 6 | 144144 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.7-3: Reference channels for CP-OFDM 16QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | 16QAM | 10 | 1/3 | 176 | 16 | 2 | 1 | 528 | 132 |
|  | 10 | 60 | 6 | 11 | 16QAM | 10 | 1/3 | 1064 | 16 | 2 | 1 | 3168 | 792 |
|  | 10 | 60 | 11 | 11 | 16QAM | 10 | 1/3 | 1928 | 16 | 2 | 1 | 5808 | 1452 |
|  | 15 | 60 | 9 | 11 | 16QAM | 10 | 1/3 | 1608 | 16 | 2 | 1 | 4752 | 1188 |
|  | 15 | 60 | 18 | 11 | 16QAM | 10 | 1/3 | 3240 | 16 | 2 | 1 | 9504 | 2376 |
|  | 20 | 60 | 12 | 11 | 16QAM | 10 | 1/3 | 2088 | 16 | 2 | 1 | 6336 | 1584 |
|  | 20 | 60 | 24 | 11 | 16QAM | 10 | 1/3 | 4224 | 24 | 1 | 1 | 12672 | 3168 |
|  | 25 | 60 | 16 | 11 | 16QAM | 10 | 1/3 | 2792 | 16 | 2 | 1 | 8448 | 2112 |
|  | 25 | 60 | 31 | 11 | 16QAM | 10 | 1/3 | 5376 | 24 | 1 | 1 | 16368 | 4092 |
|  | 30 | 60 | 19 | 11 | 16QAM | 10 | 1/3 | 3368 | 16 | 2 | 1 | 10032 | 2508 |
|  | 30 | 60 | 38 | 11 | 16QAM | 10 | 1/3 | 6656 | 24 | 1 | 1 | 20064 | 5016 |
|  | 40 | 60 | 26 | 11 | 16QAM | 10 | 1/3 | 4480 | 24 | 1 | 1 | 13728 | 3432 |
|  | 40 | 60 | 51 | 11 | 16QAM | 10 | 1/3 | 8968 | 24 | 1 | 2 | 26928 | 6732 |
|  | 50 | 60 | 33 | 11 | 16QAM | 10 | 1/3 | 5760 | 24 | 1 | 1 | 17424 | 4356 |
|  | 50 | 60 | 65 | 11 | 16QAM | 10 | 1/3 | 11272 | 24 | 1 | 2 | 34320 | 8580 |
|  | 60 | 60 | 40 | 11 | 16QAM | 10 | 1/3 | 7040 | 24 | 1 | 1 | 21120 | 5280 |
|  | 60 | 60 | 79 | 11 | 16QAM | 10 | 1/3 | 13832 | 24 | 1 | 2 | 41712 | 10428 |
|  | 70 | 60 | 47 | 11 | 16QAM | 10 | 1/3 | 8192 | 24 | 1 | 1 | 24816 | 6204 |
|  | 70 | 60 | 93 | 11 | 16QAM | 10 | 1/3 | 16392 | 24 | 1 | 2 | 49104 | 12276 |
|  | 80 | 60 | 54 | 11 | 16QAM | 10 | 1/3 | 9480 | 24 | 1 | 2 | 28512 | 7128 |
|  | 80 | 60 | 107 | 11 | 16QAM | 10 | 1/3 | 18960 | 24 | 1 | 3 | 56496 | 14124 |
|  | 90 | 60 | 61 | 11 | 16QAM | 10 | 1/3 | 10760 | 24 | 1 | 2 | 32208 | 8052 |
|  | 90 | 60 | 121 | 11 | 16QAM | 10 | 1/3 | 21000 | 24 | 1 | 3 | 63888 | 15972 |
|  | 100 | 60 | 68 | 11 | 16QAM | 10 | 1/3 | 11784 | 24 | 1 | 2 | 35904 | 8976 |
|  | 100 | 60 | 135 | 11 | 16QAM | 10 | 1/3 | 23568 | 24 | 1 | 3 | 71280 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.8	CP-OFDM 64QAM

Table A.2.3.8-1: Reference channels for CP-OFDM 64QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 64QAM | 19 | 1/2 | 9992 | 24 | 1 | 2 | 19800 | 3300 |
|  | 10 | 15 | 52 | 11 | 64QAM | 19 | 1/2 | 21000 | 24 | 1 | 3 | 41184 | 6864 |
|  | 15 | 15 | 79 | 11 | 64QAM | 19 | 1/2 | 31752 | 24 | 1 | 4 | 62568 | 10428 |
|  | 20 | 15 | 106 | 11 | 64QAM | 19 | 1/2 | 42016 | 24 | 1 | 5 | 83952 | 13992 |
|  | 25 | 15 | 133 | 11 | 64QAM | 19 | 1/2 | 53288 | 24 | 1 | 7 | 105336 | 17556 |
|  | 30 | 15 | 160 | 11 | 64QAM | 19 | 1/2 | 63528 | 24 | 1 | 8 | 126720 | 21120 |
|  | 40 | 15 | 216 | 11 | 64QAM | 19 | 1/2 | 86040 | 24 | 1 | 11 | 171072 | 28512 |
|  | 50 | 15 | 270 | 11 | 64QAM | 19 | 1/2 | 108552 | 24 | 1 | 13 | 213840 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.8-2: Reference channels for CP-OFDM 64QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 11 | 11 | 64QAM | 19 | 1/2 | 4352 | 24 | 1 | 1 | 8712 | 1452 |
|  | 10 | 30 | 24 | 11 | 64QAM | 19 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 15 | 30 | 38 | 11 | 64QAM | 19 | 1/2 | 15112 | 24 | 1 | 2 | 30096 | 5016 |
|  | 20 | 30 | 51 | 11 | 64QAM | 19 | 1/2 | 20496 | 24 | 1 | 3 | 40392 | 6732 |
|  | 25 | 30 | 65 | 11 | 64QAM | 19 | 1/2 | 26120 | 24 | 1 | 4 | 51480 | 8580 |
|  | 30 | 30 | 78 | 11 | 64QAM | 19 | 1/2 | 31240 | 24 | 1 | 4 | 61776 | 10296 |
|  | 40 | 30 | 106 | 11 | 64QAM | 19 | 1/2 | 42016 | 24 | 1 | 5 | 83952 | 13992 |
|  | 50 | 30 | 133 | 11 | 64QAM | 19 | 1/2 | 53288 | 24 | 1 | 7 | 105336 | 17556 |
|  | 60 | 30 | 162 | 11 | 64QAM | 19 | 1/2 | 64552 | 24 | 1 | 8 | 128304 | 21384 |
|  | 70 | 30 |  | 11 | 64QAM | 19 | 1/2 | 75792 | 24 | 1 | 9 | 149688 | 24948 |
|  | 80 | 30 | 217 | 11 | 64QAM | 19 | 1/2 | 86040 | 24 | 1 | 11 | 171864 | 28644 |
|  | 90 | 30 | 245 | 11 | 64QAM | 19 | 1/2 | 98376 | 24 | 1 | 12 | 194040 | 32340 |
|  | 100 | 30 | 273 | 11 | 64QAM | 19 | 1/2 | 108552 | 24 | 1 | 13 | 216216 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.8-3: Reference channels for CP-OFDM 64QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 11 | 11 | 64QAM | 19 | 1/2 | 4352 | 24 | 1 | 1 | 8712 | 1452 |
|  | 15 | 60 | 18 | 11 | 64QAM | 19 | 1/2 | 7168 | 24 | 1 | 1 | 14256 | 2376 |
|  | 20 | 60 | 24 | 11 | 64QAM | 19 | 1/2 | 9480 | 24 | 1 | 2 | 19008 | 3168 |
|  | 25 | 60 | 31 | 11 | 64QAM | 19 | 1/2 | 12296 | 24 | 1 | 2 | 24552 | 4092 |
|  | 30 | 60 | 38 | 11 | 64QAM | 19 | 1/2 | 15112 | 24 | 1 | 2 | 30096 | 5016 |
|  | 40 | 60 | 51 | 11 | 64QAM | 19 | 1/2 | 20496 | 24 | 1 | 3 | 40392 | 6732 |
|  | 50 | 60 | 65 | 11 | 64QAM | 19 | 1/2 | 26120 | 24 | 1 | 4 | 51480 | 8580 |
|  | 60 | 60 | 79 | 11 | 64QAM | 19 | 1/2 | 31752 | 24 | 1 | 4 | 62568 | 10428 |
|  | 70 | 60 | 93 | 11 | 64QAM | 19 | 1/2 | 36896 | 24 | 1 | 5 | 73656 | 12276 |
|  | 80 | 60 | 107 | 11 | 64QAM | 19 | 1/2 | 43032 | 24 | 1 | 6 | 84744 | 14124 |
|  | 90 | 60 | 121 | 11 | 64QAM | 19 | 1/2 | 48168 | 24 | 1 | 6 | 95832 | 15972 |
|  | 100 | 60 | 135 | 11 | 64QAM | 19 | 1/2 | 54296 | 24 | 1 | 7 | 106920 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.9	CP-OFDM 256QAM

Table A.2.3.9-1: Reference channels for CP-OFDM 256QAM for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 15 | 25 | 11 | 256QAM | 20 | 2/3 | 17424 | 24 | 1 | 3 | 26400 | 3300 |
|  | 10 | 15 | 52 | 11 | 256QAM | 20 | 2/3 | 36896 | 24 | 1 | 5 | 54912 | 6864 |
|  | 15 | 15 | 79 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 83424 | 10428 |
|  | 20 | 15 | 106 | 11 | 256QAM | 20 | 2/3 | 73776 | 24 | 1 | 9 | 111936 | 13992 |
|  | 25 | 15 | 133 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 140448 | 17556 |
|  | 30 | 15 | 160 | 11 | 256QAM | 20 | 2/3 | 112648 | 24 | 1 | 14 | 168960 | 21120 |
|  | 40 | 15 | 216 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 228096 | 28512 |
|  | 50 | 15 | 270 | 11 | 256QAM | 20 | 2/3 | 188576 | 24 | 1 | 23 | 285120 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.9-2: Reference channels for CP-OFDM 256QAM for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5 | 30 | 11 | 11 | 256QAM | 20 | 2/3 | 7680 | 24 | 1 | 1 | 11616 | 1452 |
|  | 10 | 30 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 15 | 30 | 38 | 11 | 256QAM | 20 | 2/3 | 26632 | 24 | 1 | 4 | 40128 | 5016 |
|  | 20 | 30 | 51 | 11 | 256QAM | 20 | 2/3 | 35856 | 24 | 1 | 5 | 53856 | 6732 |
|  | 25 | 30 | 65 | 11 | 256QAM | 20 | 2/3 | 46104 | 24 | 1 | 6 | 68640 | 8580 |
|  | 30 | 30 | 78 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 82368 | 10296 |
|  | 40 | 30 | 106 | 11 | 256QAM | 20 | 2/3 | 73776 | 24 | 1 | 9 | 111936 | 13992 |
|  | 50 | 30 | 133 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 140448 | 17556 |
|  | 60 | 30 | 162 | 11 | 256QAM | 20 | 2/3 | 114776 | 24 | 1 | 14 | 171072 | 21384 |
|  | 70 | 30 | 189 | 11 | 256QAM | 20 | 2/3 | 131176 | 24 | 1 | 16 | 199584 | 24948 |
|  | 80 | 30 | 217 | 11 | 256QAM | 20 | 2/3 | 151608 | 24 | 1 | 18 | 229152 | 28644 |
|  | 90 | 30 | 245 | 11 | 256QAM | 20 | 2/3 | 172176 | 24 | 1 | 21 | 258720 | 32340 |
|  | 100 | 30 | 273 | 11 | 256QAM | 20 | 2/3 | 192624 | 24 | 1 | 23 | 288288 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.9-3: Reference channels for CP-OFDM 256QAM for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10 | 60 | 11 | 11 | 256QAM | 20 | 2/3 | 7680 | 24 | 1 | 1 | 11616 | 1452 |
|  | 15 | 60 | 18 | 11 | 256QAM | 20 | 2/3 | 12552 | 24 | 1 | 2 | 19008 | 2376 |
|  | 20 | 60 | 24 | 11 | 256QAM | 20 | 2/3 | 16896 | 24 | 1 | 3 | 25344 | 3168 |
|  | 25 | 60 | 31 | 11 | 256QAM | 20 | 2/3 | 22032 | 24 | 1 | 3 | 32736 | 4092 |
|  | 30 | 60 | 38 | 11 | 256QAM | 20 | 2/3 | 26632 | 24 | 1 | 4 | 40128 | 5016 |
|  | 40 | 60 | 51 | 11 | 256QAM | 20 | 2/3 | 35856 | 24 | 1 | 5 | 53856 | 6732 |
|  | 50 | 60 | 65 | 11 | 256QAM | 20 | 2/3 | 46104 | 24 | 1 | 6 | 68640 | 8580 |
|  | 60 | 60 | 79 | 11 | 256QAM | 20 | 2/3 | 55304 | 24 | 1 | 7 | 83424 | 10428 |
|  | 70 | 60 | 93 | 11 | 256QAM | 20 | 2/3 | 65576 | 24 | 1 | 8 | 98208 | 12276 |
|  | 80 | 60 | 107 | 11 | 256QAM | 20 | 2/3 | 75792 | 24 | 1 | 9 | 112992 | 14124 |
|  | 90 | 60 | 121 | 11 | 256QAM | 20 | 2/3 | 86040 | 24 | 1 | 11 | 127776 | 15972 |
|  | 100 | 60 | 135 | 11 | 256QAM | 20 | 2/3 | 94248 | 24 | 1 | 12 | 142560 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 5.1.3.1-2 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

# A.3	DL reference measurement channels

## A.3.1	General

Unless otherwise stated, Tables A.3.2.2-1, A.3.2.2-2, A.3.2.2-3, A.3.3.2-1, A.3.3.2-2 and A.3.3.2-3 are applicable for measurements of the Receiver Characteristics (clause 7) with the exception of clauses 7.4 (Maximum input level).

Unless otherwise stated, Tables A.3.2.3-1, A.3.2.3-2, A.3.2.3-3, A.3.3.3-1, A.3.3.3-2 and A.3.3.3-3 are applicable for clauses 7.4 (Maximum input level) and for UE not supporting PDSCH 256QAM,

Unless otherwise stated, Tables A.3.2.4-1, A.3.2.4-2, A.3.2.4-3, A.3.3.4-1, A.3.3.4-2 and A.3.3.4-3 are applicable for clauses 7.4 (Maximum input level) and for UE supporting PDSCH 256QAM,

Unless otherwise stated, Tables A.3.2.2-1, A.3.2.2-2, A.3.2.2-3, A.3.3.2-1, A.3.3.2-2 and A.3.3.2-3 also apply for the modulated interferer used in Clauses 7.5, 7.6 and 7.8 with test specific bandwidths.

Table A.3.1-1. Common reference channel parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| CORESET frequency domain allocation |  |  | Full BW |
| CORESET time domain allocation |  |  | 2 OFDM symbols at the begin of each slot |
| PDSCH mapping type |  |  | Type A |
| PDSCH start symbol index (S) |  |  | 2 |
| Number of consecutive PDSCH symbols (L) |  |  | 12 |
| PDSCH PRB bundling |  | PRBs | 2 |
| Dynamic PRB bundling |  |  | false |
| Overhead value for TBS determination |  |  | 0 |
| First DMRS position for Type A PDSCH mapping |  |  | 2 |
| DMRS type |  |  | Type 1 |
| Number of additional DMRS |  |  | 2 |
| FDM between DMRS and PDSCH |  |  | Disable |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity | Slots | 15 kHz SCS: 10 for CSI-RS resource 1,2,3,4 30 kHz SCS: 20 for CSI-RS resource 1,2,3,4 60 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 0 for CSI-RS resource 1 and 2 1 for CSI-RS resource 3 and 4 30 kHz SCS: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 60 kHz SCS: 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  | TCI state #0 |
| PTRS configuration |  |  | PTRS is not configured |

## A.3.2	DL reference measurement channels for FDD

### A.3.2.1	General

Table A.3.2.1-1 Additional reference channels parameters for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ Processes |  | 4 |
| K1 value |  | 2 for all slots |

### A.3.2.2	FRC for receiver requirements for QPSK

Table A.3.2.2-1 Fixed reference channel for receiver requirements (SCS 15 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| Max. Throughput averaged over 1 frame | Mbps | 1.338 | 2.694 | 4.096 | 5.530 | 6.970 | 8.403 | 11.270 | 13.9392 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.2.2-2 Fixed reference channel for receiver requirements (SCS 30 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 245 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 736 | 1608 | 2472 | 3368 | 4224 | 4992 | 6912 | 8712 | 10504 | 14088 | 15880 | 17928 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 2376 | 5184 | 8208 | 11016 | 14040 | 16848 | 22896 | 28728 | 34992 | 46872 | 52920 | 58968 |
| Max. Throughput averaged over 1 frame | Mbps | 1.251 | 2.734 | 4.202 | 5.726 | 7.181 | 8.486 | 11.750 | 14.810 | 17.857 | 23.950 | 26.996 | 30.478 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2.2-3 Fixed reference channel for receiver requirements (SCS 60 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 121 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS Determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | Bits | 736 | 1192 | 1608 | 2024 | 2472 | 3368 | 4224 | 5120 | 6912 | 7808 | 8712 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slot 0,1,2,3 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slot 0,1,2,3 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | Bits | 2376 | 3888 | 5184 | 6696 | 8208 | 11016 | 14040 | 17064 | 23112 | 26136 | 29160 |
| Max. Throughput averaged over 1 frame | Mbps | 2.650 | 4.291 | 5.789 | 7.286 | 8.899 | 12.125 | 15.206 | 18.432 | 24.883 | 28.109 | 31.363 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

### A.3.2.3	FRC for maximum input level for 64QAM

Table A.3.2.3-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 12296 | 25608 | 38936 | 52224 | 64552 | 77896 | 106576 | 131176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slot 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 2 | 4 | 5 | 7 | 8 | 10 | 13 | 16 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slot 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 16200 | 33696 | 51192 | 68688 | 86184 | 103680 | 139968 | 174960 |
| Max. Throughput averaged over 1 frame | Mbps | 9.837 | 20.486 | 31.149 | 41.779 | 51.642 | 62.317 | 85.261 | 104.941 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.2.3-2 Fixed reference channel for maximum input level receiver requirements (SCS 30 kHz, FDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 5376 | 11784 | 18432 | 25104 | 31752 | 37896 | 52224 | 64552 | 79896 | 106576 | 135296 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slot2 0,1,2 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | CBs | 1 | 2 | 3 | 3 | 4 | 5 | 7 | 8 | 10 | 13 | 17 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 7128 | 15552 | 24624 | 33048 | 42120 | 50544 | 68688 | 86184 | 104976 | 140616 | 176904 |
| Max. Throughput averaged over 1 frame | Mbps | 9.139 | 20.033 | 31.334 | 42.677 | 53.978 | 64.423 | 88.781 | 109.738 | 135.823 | 181.179 | 230.003 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2.3-3 Fixed Reference Channel for Maximum input level receiver requirements (SCS 60 kHz, FDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | Bits | 5376 | 8712 | 11784 | 15112 | 18432 | 25104 | 31752 | 38936 | 52224 | 65576 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | CBs | 1 | 2 | 2 | 2 | 3 | 3 | 4 | 5 | 7 | 8 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | Bits | 7128 | 11664 | 15552 | 20088 | 24624 | 33048 | 42120 | 51192 | 69336 | 87480 |
| Max. Throughput averaged over 1 frame | Mbps | 19.354 | 31.363 | 42.422 | 54.403 | 66.355 | 90.374 | 114.307 | 140.170 | 188.006 | 236.074 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |

### A.3.2.4	FRC for maximum input level for 256 QAM

Table A.3.2.4-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 16896 | 34816 | 53288 | 71688 | 90176 | 108552 | 143400 | 180376 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slot 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 3 | 5 | 7 | 9 | 12 | 14 | 18 | 23 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 21600 | 44928 | 68256 | 91584 | 114912 | 138240 | 186624 | 233280 |
| Max. Throughput averaged over 1 frame | Mbps | 13.517 | 27.853 | 42.630 | 57.350 | 72.141 | 86.842 | 114.720 | 144.310 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.2.4-2 Fixed reference channel for maximum input level receiver requirements (SCS 30 kHz, FDD, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 17 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 7424 | 16136 | 25608 | 33816 | 44040 | 52224 | 71688 | 90176 | 108552 | 147576 | 184424 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | CBs | 1 | 3 | 4 | 5 | 6 | 7 | 9 | 12 | 14 | 19 | 23 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 3,…,19 | Bits | 9504 | 20736 | 32832 | 44064 | 56160 | 67392 | 91584 | 114912 | 139968 | 187488 | 235872 |
| Max. Throughput averaged over 1 frame | Mbps | 12.621 | 27.431 | 43.534 | 57.487 | 74.868 | 88.781 | 121.870 | 153.299 | 184.538 | 250.879 | 313.521 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2.4-3 Fixed reference channel for maximum input level receiver requirements (SCS 60 kHz, FDD, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | Bits | 7424 | 12040 | 16136 | 21000 | 25608 | 33816 | 44040 | 53288 | 71688 | 90176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | CBs | 1 | 2 | 3 | 3 | 4 | 5 | 6 | 7 | 9 | 12 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |
| For Slot 0,1,2,3 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 4,…,39 | Bits | 9504 | 15552 | 20736 | 26784 | 32832 | 44064 | 56160 | 68256 | 92448 | 116640 |
| Max. Throughput averaged over 1 frame | Mbps | 26.726 | 43.344 | 58.090 | 75.600 | 92.189 | 121.738 | 158.544 | 191.837 | 258.077 | 324.634 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |

## A.3.3	DL reference measurement channels for TDD

### A.3.3.1	General

Table A.3.3.1-1 Additional reference channels parameters for TDD

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSU | 7DS2U | 14DS1S24U |
| Special Slot Configuration (Note 2) |  | 10D+2G+2U | 6D+4G+4U | S1=12D+2G, S2=6G+8U |
| referenceSubcarrierSpacing |  | 15 kHz | 30 kHz | 60 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
|  | nrofDownlinkSlots | 3 | 7 | 14 |
|  | nrofDownlinkSymbols | 10 | 6 | 12 |
|  | nrofUplinkSlot | 1 | 2 | 4 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
| Number of HARQ Processes |  | 8 | 8 | 16 |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} | K1 = 8 if mod(i,10) = 0 K1 = 7 if mod(i,10) = 1 K1 = 6 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 3 K1 = 4 if mod(i,10) = 4 K1 = 3 if mod(i,10) = 5 K1 = 2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} | K1 = 13 if mod(i,20) = 2 K1 = 12 if mod(i,20) = 3 K1 = 11 if mod(i,20) = 4 K1 = 10 if mod(i,20) = 5 K1 = 9 if mod(i,20) = 6 K1 = 8 if mod(i,20) = 7 K1 = 7 if mod(i,20) = 8 K1 = 6 if mod(i,20) = 9 K1 = 6 if mod(i,20) = 10 K1 = 6 if mod(i,20) = 11 K1 = 6 if mod(i,20) = 12 K1 = 6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. NOTE 3: i is the slot index per frame. |  |  |  |  |

### A.3.3.2	FRC for receiver requirements for QPSK

Table A.3.3.2-1 Fixed reference channel for receiver requirements (SCS 15 kHz, TDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | CBs | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| Max. Throughput averaged over 1 frame | Mbps | 0.669 | 1.347 | 2.048 | 2.765 | 3.485 | 4.202 | 5.635 | 6.970 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.3.2-2 Fixed reference channel for receiver requirements (SCS 30 kHz, TDD, QPSK 1/3)

| Parameter | Unit |  | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 162 | 217 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 13 | 11 | 11 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | Bits | 736 | 1608 | 2472 | 3368 | 4224 | 4992 | 6912 | 8712 | 10504 | 12296 | 14088 | 17928 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | Bits | 2376 | 5184 | 8208 | 11016 | 14040 | 16848 | 22896 | 28728 | 34992 | 40824 | 46872 | 58968 |
| Max. Throughput averaged over 1 frame | Mbps | 0.810 | 2.1.769 | 2.719 | 3.705 | 4.646 | 5.491 | 7.603 | 9.583 | 11.554 | 13.526 | 15.497 | 19.721 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.2-3 Fixed reference channel for receiver requirements (SCS 60 kHz, TDD, QPSK 1/3)

| Parameter | Unit |  | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 26 | 24 | 24 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…, 13} for i from {4,…,39} | Bits | 736 | 1192 | 1608 | 2024 | 2472 | 3368 | 4224 | 5120 | 6016 | 6912 | 8712 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…, 13} for i from {4,…,39} | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…,13} for i from {4,…,39} | Bits | 2376 | 3888 | 5184 | 6696 | 8208 | 11016 | 14040 | 17064 | 20088 | 23112 | 29160 |
| Max. Throughput averaged over 1 frame | Mbps | 1.766 | 3.2.861 | 3.859 | 4.858 | 5.933 | 8.083 | 10.138 | 12.288 | 14.438 | 16.589 | 20.909 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

### A.3.3.3	FRC for maximum input level for 64QAM

Table A.3.3.3-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, TDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 12296 | 25608 | 38936 | 52224 | 64552 | 77896 | 106576 | 131176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | CBs | 2 | 4 | 5 | 7 | 8 | 10 | 13 | 16 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 16200 | 33696 | 51192 | 68688 | 86184 | 103680 | 139968 | 174960 |
| Max. Throughput averaged over 1 frame | Mbps | 4.918 | 10.243 | 15.574 | 20.890 | 20.890 | 31.158 | 42.630 | 52.470 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.3.3-2 Fixed reference channel for maximum input level receiver requirements (SCS 30 kHz, TDD, 64QAM)

| Parameter | Unit |  | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 100 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 273 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 13 | 11 | 11 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | Bits | 5376 | 11784 | 18432 | 25104 | 31752 | 37896 | 52224 | 64552 | 79896 | 92200 | 106576 | 135296 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | CBs | 1 | 2 | 3 | 3 | 4 | 5 | 7 | 8 | 10 | 11 | 13 | 17 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | Bits | 7128 | 15552 | 24624 | 33048 | 42120 | 50544 | 68688 | 86184 | 104976 | 122472 | 140616 | 176904 |
| Max. Throughput averaged over 1 frame | Mbps | 5.914 | 12.962 | 20.275 | 27.614 | 34.927 | 41.686 | 57.446 | 71.007 | 87.886 | 101.42 | 117.234 | 148.826 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.3-3. Fixed reference channel for maximum input level receiver requirements (SCS 60 kHz, TDD, 64QAM)

| Parameter | Unit |  | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 26 | 24 | 24 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…, 13} for i from {4,…,39} | Bits | 5376 | 8712 | 11784 | 15112 | 18432 | 25104 | 31752 | 38936 | 45096 | 52224 | 65576 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…, 13} for i from {4,…,39} | CBs | 1 | 2 | 2 | 2 | 3 | 3 | 4 | 5 | 6 | 7 | 8 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…, 13} for i from {4,…,39} | Bits | 7128 | 11664 | 15552 | 20088 | 24624 | 33048 | 42120 | 51192 | 60264 | 69336 | 87480 |
| Max. Throughput averaged over 1 frame | Mbps | 12.902 | 20.909 | 28.282 | 36.269 | 44.237 | 60.250 | 76.205 | 93.446 | 108.23 | 125.338 | 157.382 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

### A.3.3.4	FRC for maximum input level for 256 QAM

Table A.3.3.4-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, TDD, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 16896 | 34816 | 53288 | 71688 | 90176 | 108552 | 143400 | 180376 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | CBs | 3 | 5 | 7 | 9 | 12 | 14 | 18 | 23 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 21600 | 44928 | 68256 | 91584 | 114912 | 138240 | 186624 | 233280 |
| Max. Throughput averaged over 1 frame | Mbps | 6.758 | 13.926 | 21.315 | 28.675 | 36.070 | 43.421 | 57.360 | 72.150 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Table A.3.3.4-2 Fixed Reference channel for maximum input level receiver requirements (SCS 30 kHz, TDD, 256QAM)

| Parameter |  | Unit |  | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 100 |
| Subcarrier spacing configuration |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 273 |
| Subcarriers per resource block |  |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  |  | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 13 | 11 | 11 |
| MCS Index |  |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination |  |  | 256QAM |  |  |  |  |  |  |  |  |  |  |  |
| Modulation |  |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} |  | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} |  | Bits | 7424 | 16136 | 25608 | 33816 | 44040 | 52224 | 71688 | 90176 | 108552 | 127080 | 147576 | 184424 |
| Transport block CRC |  | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} |  | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} |  | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} |  | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} |  | Bits | 9504 | 20736 | 32832 | 44064 | 56160 | 67392 | 91584 | 114912 | 139968 | 163296 | 187488 | 235872 |
| Max. Throughput averaged over 1 frame |  | Mbps | 8.166 | 17.750 | 28.169 | 37.198 | 48.444 | 57.446 | 78.857 | 99.194 | 119.407 | 139.788 | 162.334 | 202.866 |
|  | NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.4-3 Fixed reference channel for maximum input level receiver requirements (SCS 60 kHz, TDD, 256QAM)

| Parameter | Unit |  | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 100 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 135 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 26 | 24 | 24 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination |  | 256QAM |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM | 256 QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…, 13} for i from {4,…,39} | Bits | 7424 | 12040 | 16136 | 21000 | 25608 | 33816 | 44040 | 53288 | 62504 | 71688 | 90176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…, 13} for i from {4,…,39} | CBs | 1 | 2 | 3 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 12 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3 and Slot i, if mod(i, 20) = {14,15,16,17,18,19} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 20) = {0,…, 13} for i from {4,…,39} | Bits | 9504 | 15552 | 20736 | 26784 | 32832 | 44064 | 56160 | 68256 | 80352 | 92448 | 116640 |
| Max. Throughput averaged over 1 frame | Mbps | 17.818 | 28.896 | 38.726 | 50.400 | 61.459 | 81.158 | 105.696 | 127.891 | 150.010 | 172.051 | 216.422 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |  |  |  |

# A.4	CSI reference measurement channels

# A.5	OFDMA Channel Noise Generator (OCNG)

## A.5.1	OCNG Patterns for FDD

### A.5.1.1	OCNG FDD pattern 1: Generic OCNG FDD Pattern for all unused REs

Table A.5.1.1-1: OP.1 FDD: Generic OCNG FDD Pattern for all unused REs

| OCNG Appliance OCNG Parameters | Control Region (Core Set) | Data Region |
| --- | --- | --- |
| Resources allocated | All unused REs (Note 1) | All unused REs (Note 2) |
| Structure | PDCCH | PDSCH |
| Content | Uncorrelated pseudo random QPSK modulated data | Uncorrelated pseudo random QPSK modulated data |
| Transmission scheme for multiple antennas ports transmission | Single Tx port transmission | Spatial multiplexing using any precoding matrix with dimensions same as the precoding matrix for PDSCH |
| Subcarrier Spacing | Same as for RMC PDCCH in the active BWP | Same as for RMC PDSCH in the active BWP |
| Power Level | Same as for RMC PDCCH | Same as for RMC PDSCH |
| NOTE 1: All unused REs in the active CORESETS appointed by the search spaces in use. NOTE 2: Unused available REs refer to REs in PRBs not allocated for any physical channels, CORESETs, synchronization signals or reference signals in channel bandwidth. |  |  |

## A.5.2	OCNG Patterns for TDD

### A.5.2.1	OCNG TDD pattern 1: Generic OCNG TDD Pattern for all unused REs

Table A.5.2.1-1: OP.1 TDD: Generic OCNG TDD Pattern for all unused REs

| OCNG Appliance OCNG Parameters | Control Region (Core Set) | Data Region |
| --- | --- | --- |
| Resources allocated | All unused REs (Note 1) | All unused REs (Note 2) |
| Structure | PDCCH | PDSCH |
| Content | Uncorrelated pseudo random QPSK modulated data | Uncorrelated pseudo random QPSK modulated data |
| Transmission scheme for multiple antennas ports transmission | Single Tx port transmission | Spatial multiplexing using any precoding matrix with dimensions same as the precoding matrix for PDSCH |
| Subcarrier Spacing | Same as for RMC PDCCH in the active BWP | Same as for RMC PDSCH in the active BWP |
| Power Level | Same as for RMC PDCCH | Same as for RMC PDSCH |
| NOTE 1: All unused REs in the active CORESETS appointed by the search spaces in use. NOTE 2: Unused available REs refer to REs in PRBs not allocated for any physical channels, CORESETs, synchronization signals or reference signals in channel bandwidth. |  |  |

# A.6	Void

# A.7	V2X reference measurement channels

## A.7.1	General

The algorithm for determining the payload size A is as follows; given a desired coding rate R and radio block allocation NRB

1.	Calculate the RE number of 2nd stage SCI Q_SCI2^' that can be transmitted in a given sub-frame, where in order to make sure that the code-rate of 2-A is approximate to SCI 1-A, a beta offset is selected based on MCS, and vacant resource elements γ value is determined based on NRB and DMRS frequency density.

2.	Transport Block Size is determined according to clause 8.1.3.2 of TS 38.214 [13] based on Table A.7.1-1.

3.	Calculate Binary Channel Bits per Slot for PSSCH as below

Binary Channel Bits per Slot  = (NRB* Subcarriers per resource block*CP-OFDM symbols per slot – DMRS resource REs – PSCCH resource Res - Q_SCI2^') * Qm

Where Qm is the modulation order corresponding to MCS.

In Table A.7.1-1 Common reference channel parameters are listed the Sidelink reference measurement channels specified in annexes A.7.2 to A.7.6.

Table A.7.1-1: Common reference channel parameters

| Parameter | Value | remark |
| --- | --- | --- |
| Number of HARQ Processes | 1 |  |
| Channel state | AWGN |  |
| Subcarriers per resource block | 12 |  |
| sl-PSSCH-DMRS-TimePatternList | 2 | symbol4 and symbol 10 in each slot FDMed with PSSCH within DMRS symbol Frequency density is ½ |
| CP-OFDM symbols per slot (Note1) | 12 for all slots | Excluding the first OFDM symbol in one SL slot used for AGC |
| PSCCH resource | 10 PRBs, 3 symbols in time domain |  |
| Slot number in 10ms | 10*2μ | μ=0,1,2 for 15kHz, 30kHz, 60kHz |
| PT-RS | disable |  |
| CSI-RS | disable |  |
| x-overhead | 0 |  |
| PSFCH period | 0 |  |
| 2nd stage SCI payload size | 59 | 35bits SCI-2A + 24bits CRC |
| Redundancy Version | RV0 | For channel coding |
| Alpha value for SCI-2 | 1 |  |

## A.7.2	FRC for maximum input level for QPSK

For V2X transmission over PC5, Table A.7.2-1, Table A.7.2-2 and Table A.7.2-3 are applicable for measurements on the Receiver Characteristics with the exception of Maximum input level.

Table A.7.2-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subchannel size |  | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 50 | 105 | 160 | 216 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 3624 | 7936 | 12296 | 16896 |
| Transport block CRC | Bits | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 2 | 3 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 12036 | 26556 | 41076 | 55860 |
| Max. Throughput averaged over 100ms | Mbps | 0.3624 | 0.7936 | 1.2296 | 1.6896 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

Table A.7.2-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 1608 | 3624 | 5632 | 7936 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 1 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 5160 | 12036 | 18636 | 26556 |
| Max. Throughput averaged over 100ms | Mbps | 0.3216 | 0.7248 | 1.1264 | 1.5872 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

Table A.7.2-3: Fixed reference channel for V2X receiver requirements (SCS 60 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 456 | 1608 | 2536 | 3624 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 |
| LDPC base graph |  | 2 | 2 | 2 | 2 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 1 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 7 | 7 | 1 |
| Binary Channel Bits per Slot |  | 1464 | 5160 | 8328 | 12036 |
| Max. Throughput averaged over 100ms | Mbps | 0.1824 | 0.6432 | 1.0144 | 1.4496 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

## A.7.3	FRC for maximum input level for 64QAM

For V2X transmission over PC5, Table A.7.3-1, Table A.7.3-2 and TableA.7.3-3 are applicable for Maximum input level when the maximum modulation order is 64QAM.

Table A.7.3-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subchannel size |  | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 50 | 105 | 160 | 216 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 27144 | 60456 | 92200 | 127080 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 4 | 8 | 11 | 16 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 35964 | 79524 | 123084 | 167436 |
| Max. Throughput averaged over 100ms | Mbps | 2.7144 | 6.0456 | 9.22 | 12.708 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

Table A.7.3-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 11528 | 27144 | 42016 | 60456 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 4 | 5 | 8 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 15336 | 35964 | 55764 | 79524 |
| Max. Throughput averaged over 100ms | Mbps | 2.3056 | 5.4288 | 8.4032 | 12.091 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

TableA.7.3-3: Fixed reference channel for V2X receiver requirements (SCS 60 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 3240 | 11528 | 18960 | 27144 |
| Transport block CRC | Bits | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 2 | 3 | 4 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 7 | 7 | 1 |
| Binary Channel Bits per Slot |  | 4248 | 15336 | 24840 | 35964 |
| Max. Throughput averaged over 100ms | Mbps | 1.296 | 4.6112 | 7.584 | 10.858 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

## A.7.4	FRC for maximum input level for 256QAM

For V2X transmission over PC5, Table A.7.4-1, Table A.7.4-2 and Table A.7.4-3 are applicable for Maximum input level when the 256QAM is supported.

Table A.7.4-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subchannel size |  | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 50 | 105 | 160 | 216 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 36896 | 81976 | 127080 | 172176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 5 | 10 | 16 | 21 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 |
| Binary Channel Bits per Slot |  | 48000 | 106080 | 164160 | 223296 |
| Max. Throughput averaged over 100ms | Mbps | 3.6896 | 8.1976 | 12.708 | 17.218 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

Table A.7.4-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 15880 | 36896 | 58384 | 81976 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 5 | 7 | 10 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 |
| Binary Channel Bits per Slot |  | 20544 | 48000 | 74400 | 106080 |
| Max. Throughput averaged over 100ms | Mbps | 3.176 | 7.3792 | 11.677 | 16.395 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

Table A.7.4-3: Fixed reference channel for V2X receiver requirements (SCS 60kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 4480 | 15880 | 25608 | 36896 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 2 | 4 | 5 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 |
| Binary Channel Bits per Slot |  | 5760 | 20544 | 33216 | 48000 |
| Max. Throughput averaged over 100ms | Mbps | 1.792 | 6.352 | 10.243 | 14.758 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |

Annex B (informative): Void

Annex C (informative):
Downlink physical channels

# C.1	General

The following clauses, describes the downlink Physical Channels that are transmitted during a connection i.e., when measurements are done.

# C.2	Setup

Table C.2-1 describes the downlink Physical Channels that are required for connection set up.

Table C.2-1: Downlink Physical Channels required
for connection set-up

Physical Channel

PBCH

SSS

PSS

PDCCH

PDSCH

PBCH DMRS

PDCCH DMRS

PDSCH DMRS

CSI-RS

# C.3	Connection

## C.3.1	Measurement of Receiver Characteristics

Unless otherwise stated, Table C.3.1-1 is applicable for measurements on the Receiver Characteristics (clause 7).

Table C.3.1-1: Downlink Physical Channels transmitted during a connection (FDD and TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| SSS transmit power | W | Test specific |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH to SSS | dB | 0 |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |
| EPRE ratio of PDCCH to SSS | dB | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |
| EPRE ratio of PDSCH to SSS | dB | 0 |
| EPRE ratio of PDSCH to PDSCH DMRS (Note 1) | dB | -3 |
| EPRE ratio of CSI-RS to SSS | dB | 0 |
| EPRE ratio of PTRS to PDSCH | dB | Test specific |
| EPRE ratio of OCNG DMRS to SSS | dB | 0 |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) | dB | 0 |
| NOTE 1: No boosting is applied to any of the channels except PDSCH DMRS. For PDSCH DMRS, 3 dB power boosting is applied assuming DMRS Type 1 configuration when DMRS and PDSCH are TDM'ed and only half of the DMRS REs are occupied. NOTE 2: Number of DMRS CDM groups without data for PDSCH DMRS configuration for OCNG is set to 1. |  |  |

Annex D (normative):
Characteristics of the interfering signal

# D.1 General

Some RF performance requirements for the NR UE receiver are defined with interfering signals present in addition to the wanted signal.

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, a modulated 5 MHz full bandwidth NR down link signal, and in some cases an additional CW signal, are used as interfering signal. For intra-band contiguous CA bandwidth class B and C, a modulated 5 MHz NR downlink signal is used. And for some cases an additional CW signal is used.

For NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, a modulated NR downlink signal which equals to channel bandwidth of the wanted signal for single carrier and inter-band CA cases is used as interfering signal. For intra-band contiguous CA bandwidth Class C, a modulated NR downlink signal which equals to the aggregated channel bandwidth of the wanted signal is used. For intra-band contiguous CA bandwidth class D and E cases, a modulated 50 MHz NR downlink signal is used. And for some cases an additional CW signal is used.

# D.2	Interference signals

Table D.2-1 and Table D.2-4 describes the modulated interferer for different channel bandwidth options for NR band lower than 2700MHz.

Table D.2-1: Description of modulated NR interferer for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

|  | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| RB | NOTE 1 |  |  |  |  |  |
| BWInterferer | 5 MHz |  |  |  |  |  |
|  | Channel bandwidth |  |  |  |  |  |
|  | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| RB | NOTE 1 |  |  |  |  |  |
| BWInterferer | 5 MHz |  |  |  |  |  |
| NOTE 1: The RB configured for interfering signal is the same as maximum RB number defined in Table 5.3.2-1 for each sub-carrier spacing. |  |  |  |  |  |  |

Table D.2-2 and Table D.2-3 describe the modulated interferer for different channel bandwidth options for NR band higher than 3300MHz.

Table D.2-2: Description of modulated NR interferer for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| RB | NOTE 1 |  |  |  |  |  |  |  |  |
| BWInterferer | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| NOTE 1: The RB configured for interfering signal is the same as maximum RB number defined in Table 5.3.2-1 for each sub-carrier spacing. |  |  |  |  |  |  |  |  |  |

Table D.2-3: Description of modulated NR interferer for NR bands with FDL_low≥ 3300 MHz and FUL_low≥ 3300 MHz for Intra-band contiguous CA

|  | Aggregated Channel bandwidth of Bandwdith Class C |  |  |  |  |  |  |  | Bandwidth Class D/E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz | 160 MHz | 180 MHz | 200 MHz |  |
| RB(SCS=30 kHz) | NOTE 1 |  |  |  |  |  |  |  | 133 |
| RB(SCS=60 kHz) | NOTE 1 |  |  |  |  |  |  |  | 65 |
| BWInterferer | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz | 160 MHz | 180 MHz | 200 MHz | 50MHz |
| NOTE 1: The interfering signal shall be configured in the same way as the aggregated bandwidth of the wanted signal. The RB configurations for each component carrier are defined in Table 5.3.2-1 for each sub-carrier spacing. |  |  |  |  |  |  |  |  |  |

Table D.2-4: Description of modulated NR interferer for NR bands with FDL_low < 2700 MHz and FUL_low < 2700 MHz for Intra-band contiguous CA

|  | Bandwidth Class B | Bandwidth Class C |
| --- | --- | --- |
| RB | NOTE 1 | NOTE 1 |
| BWInterferer | 5 MHz | 5 MHz |
| NOTE 1: The RB configured for interfering signal is the same as maximum RB number defined in Table 5.3.2-1 for each sub-carrier spacing. |  |  |

Annex E (normative): 
Environmental conditions

# E.1	General

This normative annex specifies the environmental requirements of the UE. Within these limits the requirements of the present documents shall be fulfilled.

# E.2	Environmental

The requirements in this clause apply to all types of UE(s).

## E.2.1	Temperature

The UE shall fulfil all the requirements in the full temperature range of:

Table E.2.1-1: Temperature conditions

| +15C to +35C | For normal conditions (with relative humidity of 25 % to 75 %) |
| --- | --- |
| -10C to +55C | For extreme conditions (see IEC publications 68-2-1 and 68-2-2) |

Outside this temperature range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in clause 6.2 for extreme operation.

## E.2.2	Voltage

The UE shall fulfil all the requirements in the full voltage range, i.e. the voltage range between the extreme voltages.

The manufacturer shall declare the lower and higher extreme voltages and the approximate shutdown voltage. For the equipment that can be operated from one or more of the power sources listed below, the lower extreme voltage shall not be higher, and the higher extreme voltage shall not be lower than that specified below.

Table E.2.2-1: Voltage conditions

| Power source | Lower extreme voltage | Higher extreme voltage | Normal conditions voltage |
| --- | --- | --- | --- |
| AC mains | 0,9 * nominal | 1,1 * nominal | nominal |
| Regulated lead acid battery | 0,9 * nominal | 1,3 * nominal | 1,1 * nominal |
| Non regulated batteries: Leclanché Lithium Mercury/nickel & cadmium | 0,85 * nominal 0,95 * nominal 0,90 * nominal | Nominal 1,1 * Nominal | Nominal 1,1 * Nominal Nominal |

Outside this voltage range the UE if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in clause 6.2 for extreme operation. In particular, the UE shall inhibit all RF transmissions when the power supply voltage is below the manufacturer declared shutdown voltage.

## E.2.3	Vibration

The UE shall fulfil all the requirements when vibrated at the following frequency/amplitudes.

Table E.2.3-1: Vibration conditions

| Frequency | ASD (Acceleration Spectral Density) random vibration |
| --- | --- |
| 5 Hz to 20 Hz | 0.96 m2/s3 |
| 20 Hz to 500 Hz | 0.96 m2/s3 at 20 Hz, thereafter –3 dB/Octave |

Outside the specified frequency range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 38.101-1 for extreme operation.

Annex F (normative): 
Transmit modulation

# F.0	General

While measuring the transmit modulation quality of carriers, an existence of the carrier leakage needs to be taken into account indicated by the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

# F.1	Measurement Point

Figure F.1-1 shows the measurement point for the unwanted emission falling into non-allocated RB(s) and the EVM for the allocated RB(s).

DFT

IFFT

TX

Front

-

-end

Channel

RF

correction

FFT

Tx

-

Rx chain

equalizer

In

-

band

emissions

meas.

DFT-s-OFDM PUSCH

0

0

IDFT

DUT

Test equipment

CP-OFDM PUSCH, PUCCH and   DM-RS

CP-OFDM PUSCH, PUCCH and DM-RS

Tone  map

DFT-s-OFDM PUSCH, PUCCH

Figure F.1-1: EVM measurement points

# F.2	Basic Error Vector Magnitude measurement

The EVM is the difference between the ideal waveform and the measured waveform for the allocated RB(s)

,

where

is a set of  modulation symbols with the considered modulation scheme being active within the measurement period,

are the samples of the signal evaluated for the EVM,

is the ideal signal reconstructed by the measurement equipment, and

is the average power of the ideal signal. For normalized modulation symbols  is equal to 1.

The basic EVM measurement interval is defined over one slot in the time domain for PUCCH and PUSCH and over one preamble sequence for the PRACH.

# F.3	Basic in-band emissions measurement

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks. The in-band emission requirement is evaluated for PUCCH and PUSCH transmissions. The in-band emission requirement is not evaluated for PRACH transmissions.

The in-band emissions are measured as follows

,

where

is a set of OFDM symbols with the considered modulation scheme being active within the measurement period,

is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g.  or  for the first adjacent RB),

(resp. ) is the lower (resp. upper) edge of the UL UE channel bandwidth,

and  are the lower and upper edge of the allocated BW, and

is the frequency domain signal evaluated for in-band emissions as defined in the clause (ii)

The relative in-band emissions are, given by

where

is the number of allocated RBs

The basic in-band emissions measurement interval is defined over one slot in the time domain. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one OFDM symbol, accordingly.

In the evaluation of in-band emissions, the timing is set according to , where sample time offsets  and  are defined in clause F.4.

# F.4	Modified signal under test

Implicit in the definition of EVM is an assumption that the receiver is able to compensate a number of transmitter impairments.

The DFT-s-OFDM modulated signals or PRACH signal under test is modified and, in the case of DFT-s-OFDM modulated signals, decoded according to:

where

is the time domain samples of the signal under test.

The CP-OFDM modulated signals or PUSCH demodulation reference signal or PUCCH data signal under test is equalised and, in the case of CP-OFDM modulated signals decoded according to:

where

is the time domain samples of the signal under test.

To minimize the error, the signal under test should be modified with respect to a set of parameters following the procedure explained below.

Notation:

is the sample timing difference between the FFT processing window in relation to nominal timing of the ideal signal.

is the RF frequency offset.

is the phase response of the TX chain.

is the amplitude response of the TX chain.

In the following  represents the middle sample of the EVM window of length  (defined in the next clauses) or the last sample of the first window half if is even.

The EVM analyser shall

-	detect the start of each slot and estimate  and ,

-	determine  so that the EVM window of length  is centred

-	on the time interval determined by the measured cyclic prefix minus 16κ samples of the considered OFDM symbol for symbol l for subcarrier spacing configuration µ in a subframe, with l = 0 or l = 7*2^µ for normal CP, i.e. the first 16κ samples of the CP should not be taken into account for this step. In the determination of the number of excluded samples, a sampling rate of 1/Tc is assumed. If a different sampling rate is used, the number of excluded samples is scaled linearly.

-	on the measured cyclic prefix of the considered OFDM symbol symbol for all other symbols for normal CP and for symbol 0 to 11 for extended CP.

-	on the measured preamble cyclic prefix for the PRACH

To determine the other parameters a sample timing offset equal to  is corrected from the signal under test. The EVM analyser shall then

-	correct the RF frequency offset for each time slot, and

-	apply an FFT of appropriate size. The chosen FFT size shall ensure that in the case of an ideal signal under test, there is no measured inter-subcarrier interference.

The carrier leakage shall be removed from the evaluated signal before calculating the EVM and the in-band emissions; however, the removed relative carrier leakage power also has to satisfy the applicable requirement.

At this stage the allocated RBs shall be separated from the non-allocated RBs. In the case of PUCCH and PUSCH EVM, the signal on the non-allocated RB(s), , is used to evaluate the in-band emissions.

Moreover, the following procedure applies only to the signal on the allocated RB(s).

-	In the case of PUCCH and PUSCH, the UL EVM analyzer shall estimate the TX chain equalizer coefficients and  used by the ZF equalizer for all subcarriers by time averaging at each signal subcarrier of the amplitude and phase of the reference and data symbols. The time-averaging length is 1 slot. This process creates an average amplitude and phase for each signal subcarrier used by the ZF equalizer. The knowledge of data modulation symbols may be required in this step because the determination of symbols by demodulation is not reliable before signal equalization.

-	In the case of PRACH, the UL EVM analyzer shall estimate the TX chain coefficients and  used for phase and amplitude correction and are seleted so as to minimize the resulting EVM. The TX chain coefficients are not dependent on frequency, i.e.  and . The TX chain coefficient are chosen independently for each preamble transmission and for each .

At this stage estimates of , ,  and  are available.  is one of the extremities of the window , i.e. can be  or , where  if  is odd and  if is even. The EVM analyser shall then

-	calculate EVMl with  set to ,

-	calculate EVMh with  set to .

For the EVM calculation on the symbols with a transient period when the UE signals a transient period capability (tp) of 2, 4 or 7usec, Δt is is given below.

-	calculate EVMl_tp with Δt set totp+tpstartTc+1, where is 1/Tc the sampling rate

-	calculate EVMh_tp with Δt set toCP+tpstartTc-1, where 1/Tc is the sampling rate and the CP is the cyclic prefix of the symbol on which EVM is calculated(e.g. long CP for the first symbol of the slot) in seconds

A pictorial representation of the EVM measurement windows is given in Figure F.4-1.

Figure F.4-1 EVM measruement window

# F.5	Window length

## F.5.1	Timing offset

As a result of using a cyclic prefix, there is a range of, which, at least in the case of perfect Tx signal quality, would give close to minimum error vector magnitude. As a first order approximation, that range should be equal to the length of the cyclic prefix. Any time domain windowing or FIR pulse shaping applied by the transmitter reduces the  range within which the error vector is close to its minimum.

## F.5.2	Window length

The window length W affects the measured EVM and is expressed as a function of the configured cyclic prefix length. In the case where equalization is present, as with frequency domain EVM computation, the effect of FIR is reduced. This is because the equalization can correct most of the linear distortion introduced by the FIR. However, the time domain windowing effect can't be removed.

## F.5.3	Window length for normal CP

Table F.5.3-1, F.5.3-2, F.5.3-3 below specify the EVM window length (W) for normal CP.

Table F.5.3-1: EVM window length for normal CP for NR, FR1, 15 kHz SCS

| Channel Bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-131 (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 18 | 50 |
| 10 | 1024 | 72 | 36 | 50 |
| 15 | 1536 | 108 | 54 | 50 |
| 20 | 2048 | 144 | 72 | 50 |
| 25 | 2048 | 144 | 72 | 50 |
| 30 | 3072 | 216 | 108 | 50 |
| 40 | 4096 | 288 | 144 | 50 |
| 50 | 4096 | 288 | 144 | 50 |
| NOTE 1: These percentages are informative and apply to a slot's symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Table F.5.3-2: EVM window length for normal CP for NR, FR1, 30 kHz SCS

| Channel Bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-131 (%) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 9 | 50 |
| 10 | 512 | 36 | 18 | 50 |
| 15 | 768 | 54 | 27 | 50 |
| 20 | 1024 | 72 | 36 | 50 |
| 25 | 1024 | 72 | 36 | 50 |
| 30 | 1536 | 108 | 54 | 50 |
| 40 | 2048 | 144 | 72 | 50 |
| 50 | 2048 | 144 | 72 | 50 |
| 60 | 3072 | 216 | 108 | 50 |
| 70 | 3072 | 216 | 108 | 50 |
| 80 | 4096 | 288 | 144 | 50 |
| 90 | 4096 | 288 | 144 | 50 |
| 100 | 4096 | 288 | 144 | 50 |
| NOTE 1: These percentages are informative and apply to a slot's symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Table F.5.3-3: EVM window length for normal CP for NR (60 kHz SCS)

| Channel Bandwidth (MHz) | FFT size | Cyclic prefix length for symbols in FFT samples | EVM window length W | Ratio of W to total CP length 1 (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | 9 | 50 |
| 15 | 384 | 27 | 14 | 50 |
| 20 | 512 | 36 | 18 | 50 |
| 25 | 512 | 36 | 18 | 50 |
| 30 | 768 | 54 | 27 | 50 |
| 40 | 1024 | 72 | 36 | 50 |
| 50 | 1024 | 72 | 36 | 50 |
| 60 | 1536 | 108 | 54 | 50 |
| 70 | 1536 | 108 | 54 | 50 |
| 80 | 2048 | 144 | 72 | 50 |
| 90 | 2048 | 144 | 72 | 50 |
| 100 | 2048 | 144 | 72 | 50 |
| NOTE 1: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

## F.5.4	Window length for Extended CP

Table F.5.4-1 below specifies the EVM window length (W) for extended CP. The number of CP samples excluded from the EVM window is the same as for normal CP length.

Table F.5.4-1: EVM window length for extended CP for NR, FR1, 60 kHz SCS

| Channel Bandwidth (MHz) | FFT size | Cyclic prefix length in FFT samples | EVM window length W | Ratio of W to total CP length1 (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 64 | 54 | 84.4 |
| 15 | 384 | 96 | 80 | 83.3 |
| 20 | 512 | 128 | 106 | 82.8 |
| 25 | 512 | 128 | 110 | 85.9 |
| 30 | 768 | 192 | 164 | 85.4 |
| 40 | 1024 | 256 | 220 | 85.9 |
| 50 | 1024 | 256 | 220 | 85.9 |
| 60 | 1536 | 384 | 330 | 85.9 |
| 70 | 1536 | 384 | 330 | 85.9 |
| 80 | 2048 | 512 | 440 | 85.9 |
| 90 | 2048 | 512 | 440 | 85.9 |
| 100 | 2048 | 512 | 440 | 85.9 |
| NOTE 1: These percentages are informative. |  |  |  |  |

## F.5.5	Window length for PRACH

The table below specifies the EVM window length for PRACH preamble formats for LRA= 839 and .

Table F.5.5-1 EVM window length for PRACH formats for  LRA= 839

| Preamble format | Cyclic prefix length NCP | Nominal FFT size1 | EVM window length W in FFT samples | Ratio of W to CP2 |
| --- | --- | --- | --- | --- |
| 0 | 3168 | 24576 | 2307 | 72.8% |
| 1 | 21024 | 24576 | 20163 | 95.9% |
| 2 | 4688 | 24576 | 3827 | 81.6% |
| 3 | 3168 | 6144 | 2952 | 93.2% |
| NOTE 1: The use of other FFT sizes is possible as long as appropriate scaling of the window length is applied NOTE 2: These percentages are informative |  |  |  |  |

The table below specifies the EVM window length for PRACH preamble formats for  LRA= 139  and  where.

Table F.5.5-2 EVM window length for PRACH formats for  LRA= 139

| Preamble format | Cyclic prefix length NCP | Nominal FFT size1 | EVM window length W in FFT samples | Ratio of W to CP2 |
| --- | --- | --- | --- | --- |
| A1 | 2882- | 20482- | 1442- | 50.0% |
| A2 | 5762- | 20482- | 4322- | 75.0% |
| A3 | 8642- | 20482- | 7202- | 83.3% |
| B1 | 2162- | 20482- | 722- | 33.3% |
| B2 | 3602- | 20482- | 2162- | 60.0% |
| B3 | 5042- | 20482- | 3602- | 71.4% |
| B4 | 9362- | 20482- | 7922- | 84.6% |
| C0 | 12402- | 20482- | 10962- | 88.4% |
| C2 | 20482- | 20482- | 19042- | 93.0% |
| NOTE 1: The use of other FFT sizes is possible as long as appropriate scaling of the window length is applied NOTE 2: These percentages are informative |  |  |  |  |

# F.6	Averaged EVM

The general EVM is averaged over basic EVM measurements for n slots in the time domain.

,

where n is

n=10, for 15 kHz SCS20, for 30 kHz SCS40, for 60 kHz SCS

for PUCCH, PUSCH.

The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

Thus  is calculated using in the expressions above and is calculated using .

Thus we get:

The calculation of the EVM for the demodulation reference signal, , follows the same procedure as calculating the general EVM, with the exception that the modulation symbol set  defined in clause F.2 is restricted to symbols containing uplink demodulation reference signals.

The basic  measurements are first averaged over n slots in the time domain to obtain an intermediate average .

EVMDMRS=1ni=1nEVMDMRS,i2

In the determination of each , the timing is set to  if , and it is set to  otherwise, where  and  are the general average EVM values calculated in the same n slots over which the intermediate average  is calculated. Note that in some cases, the general average EVM may be calculated only for the purpose of timing selection for the demodulation reference signal EVM.

Then the results are further averaged to get the EVM for the demodulation reference signal, ,

The PRACH EVM, , is averaged over 2 preamble sequence measurements for long preamble formats as defined in table 6.3.3.1-1 in [6] and averaged over 10 preamble sequence measurements for short preamble formats as defined in table 6.3.3.1-2 in [6].

The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

Thus  is calculated using  and is calculated using .

Thus we get:

# F.7	Spectrum Flatness

The data shall be taken from FFT coded data symbols and the demodulation reference symbols of the allocated resource block.

Annex G (informative): Void

Annex H (informative): Void

Annex I (informative): Void

Annex J (informative): Void

Annex K (informative): Void

Annex L (normative):
ModifiedMPR-Behavior

# L.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [7] by an 8-bit bitmap per supported NR band.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n41 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | - EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | - EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
| n71 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |

Annex M (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN4#84 | R4-1708909 |  |  |  | Initial Skeleton | 0.0.1 |
| 2017-10 | RAN4#84Bis | R4-1709958 |  |  |  | Added approved TPs in RAN4-NR-AH#3 R4-1709948, TP for TS 38.101-1: minimum output power, Huawei R4-1709454, TP for TS 38.101-1:UE Tx spurious emission for range 1, ZTE Corporation | 0.1.0 |
| 2017-10 | RAN4#84Bis | R4-1711978 |  |  |  | Embedded approved TPs in RAN4#84Bis R4-1711556, "TP to TS 38.101: Draft CR to Transmitter power clause", Nokia R4-1710962, "TP to TS 38.101-1: Draft CR to Output RF spectrum emissions" Nokia R4-1711608, "TP for TS38.101-1 on conducted UE transmitter intermodulation for FR1(section 6.5)" ZTE Corporation Number of TPs by editors | 0.2.0 |
| 2017-12 | RAN4#85 | R4-1713805 |  |  |  | Approved TPs in RAN4#85 R4-1713204, TP on general parts for 38.101-1 NR FR1, Ericsson R4-1714047, WF on MPR for sub6GHz, NTT DOCOMO, INC. R4-1714052, TP for TS 38.101-1 introduction of band n71 for transmitter characteristics, T-Mobile USA Inc. R4-1714162, TP to 38.101-1: ACS, Ericsson R4-1714163, TP to 36.101-1: In-band blocking, Ericsson R4-1714446, TP to 36.101-1: Out-of-band blocking and exceptions for spurious response, Ericsson R4-1714369, TP for NBB requirement for FR1, Intel Corporation R4-1714529, TP on introducing operating bands for NR-LTE DC including SUL band combinations in 38.101-1, Huawei R4-1714097, TP for TS 38.101-1: UE RF requirements for standalone SUL, Huawei R4-1714536, TP for TS 38.101-1: Channel Bandwidth Definition, Qualcomm Incorporated (Note, this TP was further discussed and edited in the reflector) R4-1714114, TP for TS 38.101-1: Channel Arrangement, Qualcomm Incorporated (Note, this TP was further discussed and edited in the reflector) R4-1714029, Sub6 Reference Sensitivity, Qualcomm Incorporated R4-1714329, TP to TR 38.101-01 v0.2.0: ON/OFF mask design for NR UE transmissions for FR1, Ericsson Band list according to R4-1714542, List of bands and band combinations to be introduced into RAN4 NR core requirements by December 2017, RAN4 Chairmen Input from: R4-1714479, TP for TR 38.817-01 NR channel bandwidth, Huawei, HiSilicon | 0.3.0 |
| 2017-12 | RAN4#85 | R4-1714569 |  |  |  | Further corrections and alignments with 38.104 after email review | 0.4.0 |
| 2017-12 | RAN#78 | RP-172475 |  |  |  | v1.0.0 submitted for plenary approval. Contents same as 0.4.0 | 1.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2018-03 | RAN#79 | RP-180264 | 0003 |  | F | Implementation of endorced CRs to 38.101-1 Endorsed draft CRs F: R4-1800400, Editorial corrections for 38.101-1, Qualcomm B: R4-1801102, Draft CR for 30 MHz CBW support, Huawei F: R4-1800032, 38.101-1 n71 draft CR for section 6.2.3 - UE A-MPR - NS values, T-Mobile USA Inc. B: R4-1801121, Draft pCR for TS 38.101-1 version 15.0.0: Remaining ON/OFF masks for FR1 NR UE transmissions, Ericsson F: R4-1800417, Correction of NR SEM table and additional requirements table, vivo F: R4-1800033, 38.101-1 n71 draft CR for section 6.5.3.2 Spurious emissions for UE co-existence, T-Mobile USA Inc. F: R4-1801114, Proposal on protected band numbering in UE specs, Sprint Corporation F: R4-1800407, Draft CR for TS 38.101-1: Mandatory 4Rx antenna performance for NR UE, Vodafone Group Plc F: R4-1800451 Draft CR for TS 38.101-1: Clarification of 4Rx NR bands, Huawei, HiSilicon F: R4-1801136, Draft CR for TS 38.101-1: REFSENS for NR bands, Huawei, HiSilicon F: R4-1801137, Draft CR: n71 REFSENS, Dish Network F: R4-1800395, Draft CR to 38.101-1: corrections to ACS and in-band blocking, Ericsson F: R4-1800396, Draft CR to 38.101-1: corrections to out-of-band blocking, Ericsson F: R4-1800397, Draft CR to 38.101-1: corrections to spurious response, Ericsson F: R4-1800305, Draft CR for NR FR1 wide band intermodulation requirements, MediaTek Inc. F: R4-1800320, Draft CR to 38.101-1: Rx Spurious emission for NR FR1 (section 7.9), ZTE Corporation F: R4-1800473, Draft CR on UE RF requirements for SUL in TS 38.101-1, Huawei F: R4-1800965, Draft CR to TS 38.101-1: Asymmetric CH BW operation, Dish Network F: R4-1800882, Draft CR for correction of UE channel bandwidth for Bands n77 and n78 for TS 38.101-1, Orange UK F: R4-1801012, Draft CR to 38.101-1: Clarifications to UE spectrum utilization section 5.3, Ericsson F: R4-1800030, 38.101-1 n71 draft CR for section 5.4.4 - TX–RX frequency separation, T-Mobile USA Inc F: R4-1801228, Draft CR to 38.101-1: Channel spacing for CA for NR FR1(section 5.4.1.2), ZTE Corporation F: R4-1801231, Correction CR for channel spacing:38.101-1, Samsung F: R4-1801235, Draft CR to TS 38.101-1: Corrections on channel raster calculation in section 5.4.2, ZTE Corporation F: R4-1801318, Draft CR on synchronization raster, Huawei RAN4#86: R4-1803053, Draft CR for new spec structure of 38.101-1, Ericsson R4-1801479, Draft CR to 38.101-1: Default Tx-RX frequency separation for NR FR1(section 5.4.4), ZTE R4-1801581, Draft CR for TS 38.101-1 update of 4Rx bands, Huawei Technologies France R4-1802211, draft CR TS 38.101-1 Uplink configuration for FR1 NR REFSENS, Skyworks Solutions Inc. R4-1802342, Draft CR for NR FR1 ACS case 2 transmitter power setting correction (Note 1), MediaTek Inc. R4-1802509, Draft CR on 38.101-1 v15.0.0: Remaining ON/OFF masks for FR1 NR UE transmissions, Ericsson R4-1802566, Draft CR to TS 38.101-1: Clarification of mixed numerology guardband size, Ericsson R4-1802978, Draft CR to TS 38.101-1: Corrections on channel raster in Section 5.4.2.3, Intel Corporation R4-1803064, Draft CR for 38.101-1: Correction of errors, Sprint Corporation R4-1803065, Draft CR for 38.101-1 Introduction of n41requirements, Sprint Corporation R4-1803242, Draft CR to 38.101-1: Corrections to n66, Dish Network R4-1803285, Draft CR to 38.101-1: Correction to CH BWs without symmetric uplink Dish Network, Skyworks Solutions Inc. R4-1803436, Introduction of UL subcarrier alignment for additional bands, AT&T R4-1803456, Draft CR for 38.101-1: Spurious Emissions for UE Coexistence, Sprint Corporation R4-1803461, CR on configured transmitted power for TS 38.101-1, Huawei R4-1803452, draft CR for introduction of completed band combinations from 37.865-01-01 into 38.101-1, Ericsson R4-1803567, Draft CR for TS 38.101-1: Sync raster offset in re-farming bands (5.4.3), Ericsson R4-1803365, CR to introduce MPR for PC2 and PC3 and A-MPR for UTRA protection, Nokia | 15.1.0 |
| 2018-06 | RAN#80 | RP-181262 | 0011 |  | F | CR to TS 38.101-1: Implementation of endorsed draft CRs from RAN4 #86bis and RAN4 #87 R4-1803900, Draft CR into TS 38.101-1 Introduction of band combinations for SUL, Huawei R4-1804021 CR for clarifications for NR FR1 CA BW Classes Nokia, Nokia Shanghai Bell R4-1804140 CR for Narrow Band Blocking requirement for FR1 Intel Corporation R4-1804219 Draft CR for 38.101-1: n41 SEM and additional spurious emissions SPRINT Corporation R4-1804266 Draft CR to 38.101-1 MPR channel bandwidth criteria Skyworks Solutions Inc. R4-1804267 Draft CR to 38.101-1 n3,n5,n8 REFSENS levels Skyworks Solutions Inc. R4-1804268 Draft CR to 38.101-1: Corrrection to n41 uplink configuration for reference sensitivity Skyworks Solutions Inc. R4-1804370 Draft CR to add missing NR inter-band DL CA in FR1 for TS 38.101-1 NTT DOCOMO, INC. R4-1804581 Draft CR to 38.101-1: On EVM Wording Qualcomm, Inc. R4-1804948 Corrections to 5.3.3 in TS 38.101-1 Nokia, Nokia Shanghai Bell R4-1804877 draft CR introduction completed band combinations 37.865-01-01 -> 38.101-1 Ericsson R4-1805444 Draft CR to TS 38.101-1: Asymmetric CH BW operation Dish Network R4-1805447 drfat CR for including SRS antenna switching in configured output power Qualcomm Incorporated R4-1805462 Editorial corrections to UE RF requirements in 38.101-1 Qualcomm Incorporated R4-1805659 Draft CR for CBW for n50 for 38.101-1 Huawei R4-1805664 Draft CR to 38.101-1: Addition of Annex F Rohde & Schwarz R4-1805665 Correction to inner and outer definitions for MPR Qualcomm Incorporated R4-1805684 Draft CR to TS38.101-1: Channel Raster to Resource Element Mapping (Section 5.4.2.2) and RB alignment with different numerologies (Section 5.3.4) ZTE Corporation R4-1805698 Draft CR for 38.101-1 for Rx(Ch7) of Band n77, n78 and n79 RF requirements CMCC R4-1805699 Draft CR to 38.101-1:introduction of Tx/Rx requirements for inter-band CA ZTE Corporation R4-1805751 Draft CR on UE-to-UE coexistence requirements to protect band 29 from NR band 71 LG Electronics France R4-1805783 Draft CR for 38.101-1 for Tx(Ch6) of Band n77, n78 and n79 RF requirements CMCC R4-1805902 Draft CR into TS 38.101-1 Correction on SUL_n78-n80 Huawei, HiSilicon R4-1805904 Draft CR into TS 38.101-1 Introduction of new band combinations for SUL Huawei, HiSilicon R4-1805921 Draft CR on NR UE REFSENS SNR FRC for FR1 Intel Corporation R4-1805981 Draft CR for TS38.101-1:Sync raster Samsung R4-1804548 Draft CR for CA BW class for FR1 NTT DOCOMO, INC. R4-1806170 Draft CR on frequency error for TS 38.101-1 ZTE Corporation R4-1806481 Draft CR for Environmental conditions in TS 38.101-1 Annex NTT DOCOMO, INC. R4-1806657 Draft CR to 38.101-1: Measurement BW for min and off power Skyworks Solutions Inc. R4-1806669 Draft CR to TS38.101-1_introduction of completed band combinations for inter-band 2UL CA ZTE Corporation R4-1806673 Draft CR to TS38.101-1_Remove brackets from Tx and Rx spurious emission table ZTE Corporation R4-1806677 Draft CR on including CA bandwidth class and band combinations for intra-band CA LG Electronics France R4-1806719 Introduction of 7.5 kHz frequency shift for Band n71 Ericsson, T-Mobile R4-1806844 Draft CR for 38.101-1 for Tx(Ch6): missing maximum power requirements for n1 and n8 SoftBank Corp. R4-1806945 Draft CR for TS 38.101-1: Channel raster and NR-ARFCN clarification (5.4.2) Ericsson R4-1807039 Intra-band CA terminology for UE ZTE Corporation R4-1807178 Corrections to n70 TX/RX frequency separation Dish Network R4-1807181 Corrections to spurious emissions UE co-existence table Dish Network R4-1807234 Draft CR into TS 38.101-1 Some Corrections for SUL Huawei, HiSilicon R4-1807269 Corrections to Wide band intermodulation table <2700MHz Dish Network R4-1807392 to remove the brackets for SU in 38.101-1 Huawei, HiSilicon R4-1807647 Draft CR to TS 38.101-1: Correction to Asymmetric CH BW operation Dish Network R4-1807680 Draft CR on 38.101-1 on channel raster to achieve alignment of data and SSB subcarrier grids Nokia, Nokia Shanghai Bell, Ericsson R4-1807705 CR for TS 38.101-1 A-MPR for n51 Huawei, Hisilicon R4-1807814 Draft CR for 38.101-1: SEM correction for n41 Sprint Corporation R4-1807851 Draft CR for 38.101-1: UE spurious emission protection requirements for n5 Sprint Corporation R4-1807920 General requirements for downlink inter-band CA Qualcomm Incorporated R4-1807923 Resolution bandwidth for ACLR Qualcomm Incorporated R4-1808084 Introduction of n12 into TS 38.101-1 Nokia R4-1808087 Draft CR 38.101-1: Introduction of n2, n25, n66 and n70 Sprint Corporation, Dishnetwork R4-1808090 Draft CR to TS 38.101-1: Inclusion of Simultaneous RxTx UE capability for some band combinations Ericsson, Vodafone, Orange R4-1808107 Draft CR to TS38.101-1_corrections on UE coexistence ZTE Corporation R4-1808111 TP to TS38.101-1 - UE ON/OFF masks Ericsson R4-1808116 Draft CR to 38.101-1: introduction of Band n34,n39 and n40 RF requirements ZTE Corporation,CMCC R4-1808136 Draft CR to 38.101-1: FR1 UE Power Control Qualcomm Incorporated R4-1808141 Correction to MPR for PC2 and spectrum emission mask measurement bandwidth Qualcomm Incorporated R4-1808142 Draft CR for 38.101-1 n41 A-MPR Sprint Corporation, Nokia, Nokia Shanghai Bell, Ericsson R4-1808143 Draft CR for TS 38.101-1 A-MPR for n20 Huawei, HiSilicon R4-1808155 Draft CR for TS 38.101-1: to correct reqirements for n71 Samsung R4-1808178 Addition parameters about n50 & n51 in TS 38.101-1 Huawei, Hisilicon, Etisalat (editors note: n50 not implemented per chairmans aggreement) R4-1808182 Draft CR for TS 38.101-1 A-MPR for n28 Huawei, HiSilicon R4-1808187 CR for RF requirements for Coherent UL MIMO for FR1 Qualcomm Austria RFFE GmbH R4-1808207 Draft CR to 38.101-1: On EVM Averaging Length, Wording , Qualcomm R4-1808209 Draft CR for 38.101-1 for Tx (Ch6) of HPUE Qualcomm R4-1808466 Draft CR on UL RMC and OCNG pattern for FDD REFSENS tests RD session R4-1808493 Draft CR for TS 38.101-1: Channel and sync raster corrections (5.4) Ericsson R4-1808507 Draft CR for TS38.101-1 on addition of new 90MHz UE CBW for n41/n78 LG Electronics Inc., LG Uplus, Samsung R4-1808176, Draft CR for 38.101-1 : Introduction of A-MPR for n8, SoftBank R4-1808201, Draft CR for 38.101-1 : Introduction of A-MPR for n1, SoftBank R4-1807101, draft CR introduction completed band combinations 37.865-01-01 -> 38.101-1, Ericsson | 15.2.0 |
| 2018-09 | RAN#81 | RP-181896 | 0025 |  | F | Big CR for 38.101-1 Endorced draft CRs from RAN4#NR-AH-1807 R4-1809335, Draft CR on UL RMC for FR1 RF tests , Qualcomm Incorporated R4-1809337, Draft CR on NR UE REFSENS SNR FRC for FR1, Intel Corporation R4-1809339, Draft CR on measurement of receiver characteristics for FR1 RF Tests, Qualcomm Incorporated R4-1809396, Draft CR on NR UE maximum input level FRC for FR1, Intel R4-1809567, Draft CR on OCNG pattern for FR1 REFSENS tests, Qualcomm Incorporated, Rohde & Schwarz Endorced draft CRs from RAN4#88 R4-1809714, Draft CR to correct in-band blocking parameters for FR1, Anritsu Corporation R4-1809784, Draft CR to 38.101-1: Corrections on CA bandwidth classes for FR1, ZTE Corporation R4-1809785, Draft CR to TS 38.101-1 for Corrections on UE transmitter power, ZTE Corporation R4-1809793, Draft CR to 38.101-1: Corrections on additional spectrum emission mask, ZTE Corporation R4-1809919, Correction on UE receiver requirement for FR1, CATT R4-1810091, Draft CR TS 38.101-1 - UE ON-OFF mask clean up, Ericsson R4-1810210, Draft CR for TS 38.101-1: MPR inner and outer RB allocations formula correction, MediaTek, Inc. R4-1810229, Draft CR for TS 38.101-1: Spurious emission for UE coexistence table corrections, MediaTek, Inc. R4-1810230, Draft CR for TS38.101-1 to correct 90MHz UE CBW, LG Electronics, Inc. R4-1810232, Draft CR for TS 38.101-1: Table 7.3.2-1 n77 reference sensitivity corrections, MediaTek, Inc. R4-1810369, Draft CR to 38.101-1: Corrections on symbols and abbreviations in section 3, ZTE Corporation R4-1810376, Draft CR: General corrections to n71 requirements, Dish Network R4-1810428, Draft CR on TS38.101-1 for UE maximum output power for UL MIMO, OPPO R4-1810552, Correction of reference tables, OPPO R4-1810729, Draft CR for introduction of Band n74 for TS 38.101-1, NTT DOCOMO, Inc. R4-1810862, Draft CR to 38.101-1: Updates to Transmit Modulation Annex, Rohde & Schwarz R4-1810892, CR to update Table 6.2D.1-2 for FR1, Qualcomm Incorporated R4-1810961, CR on ACS minimum requirement, Intel Corporation R4-1810965, CR on Out-of-Band Blocking minimum requirement, Intel Corporation R4-1810967, CR on Rx Intermodulation characteristics for CA, Intel Corporation R4-1810974, Annex lettering change for 38.101-1, Qualcomm Incorporated R4-1811189, CR to add more details to Coherent UL MIMO spec for FR1, Qualcomm Incorporated R4-1811280, Corrections of NR receiver characteristics titles, Vivo R4-1811455, Draft CR on DL Physical Channel for FR1 RF tests, Qualcomm Europe Inc. (Spain) R4-1811457, NS numbering, Qualcomm Incorporated R4-1811459, Correction on UE transmitter requirement for FR1, CATT R4-1811463, Draft CR for 38.101-1: Addition of missing NR CA configurations n8-n75 and n28-n75, VodafoneItalia SpA R4-1811472, Addition parameters about n51 in TS 38.101-1, Huawei, Hisilicon, Etisalat R4-1811474, CR CP- OFDM almost contiguous allocation, Nokia, Nokia Shanghai Bell R4-1811477, Draft CR to 38.101-1: FR1 Power Control, Qualcomm Incorporated R4-1811478, A-MPR correction for n20 and n28, Huawei, HiSilicon R4-1811490, Draft CR to 38.101-1: Addition of Carrier Leakage table, Rohde & Schwarz R4-1811491, Draft CR for TS38.101-1 on transmit signal quality, OPPO R4-1811493, CR to TS 38.101-1: pi/2 BPSK with Spectrum Shaping, Indian Institute of Tech (M),Indian Institute of Tech (H), CEWiT, Nokia R4-1811513, A proposal on 2UL co-ex table modification, SoftBank Corp. R4-1811514, Draft CR to TS 38.101-1: Clarification on OCNG, Keysight Technologies UK Ltd R4-1811516, Draft CR on NR DL FRCs for FR1 UE RF requirements, Intel Corporation R4-1811550, Draft CR to TS 38.101-1 on channel bandwidth and spacing descriptions, Ericsson R4-1811553, Draft CR to 38.101-1: Corrections on description of channel raster entries, ZTE Corporation R4-1811783, Measurement period of PRACH time mask, CATT R4-1811792, Draft CR for A-MPR revision for n1, NTT DOCOMO, INC. R4-1811798, Draft CR for Pcmax for FR1, Qualcomm Incorporated R4-1811799, Pcmax for inter-band NR CA FR1 draft CR, InterDigital, Inc. R4-1811812, Draft CR to 38.101-1: On FR1 AMPR Band n41 NS_04, Qualcomm Incorporated R4-1811816, CR to update the definition of Long and Short subslot for FR1, Qualcomm R4-1811894, Addition parameters about n50 in TS 38.101-1, Huawei R4-1811896, Draft CR for TS 38.101-1: n41 GSCN range modification, MediaTek Inc. R4-1811285, Draft CR TS 38.101-1: NS_04 A-MPR' and spurious emisison corrections, Sprint | 15.3.0 |
| 2018-12 | RAN#82 | RP-182836 | 0029 | 1 | F | Endorced draft CRs from RAN4#88Bis: R4-1812050, CR Simplification of NR NS_08, Nokia R4-1812054, Correction for Inter-band CA operating bands table in TS 38.101-1, Nokia. R4-1812079, draft CR to introduce asymmetric UL DL channel BW combinations for n71, T-Mobile USA Inc. R4-1812121, Draft CR on Note1 Corrections in 38.101 RX tests, Qualcomm R4-1812128, draftCR on 256QAM UL power requirement, Intel Corporation R4-1812200, Draft CR to TS 38.101-1 Add clarification note to PC3 MPR table, Intel Corporation R4-1812217, Draft CR to 38.101-1: Corrections on the descriptions of UE channel bandwidth for CA, ZTE Corporation R4-1812319, Draft CR for TS 38.101-1: REFSENS UL configuration corrections, MediaTek Inc. R4-1812320, Draft CR for TS 38.101-1: Out-of-band blocking exceptions for CA, MediaTek Inc. R4-1812322, Draft CR for TS 38.101-1: Blocking characteristics for SUL, MediaTek Inc. R4-1812397, Clarification for almost contiguous CP-OFDM, Qualcomm Incorporated R4-1812508, Draft CR to 38.101-1: Corrections on channel raster & SS raster for operating bands, ZTE Corporation R4-1812611, Draft CR to 38.101-1: Some corrections for inter-band CA combinations, ZTE Corporation R4-1813459, Draft CR for TS 38.101-1: Support 4Rx for n38, Huawei R4-1813469, draftCR on applicability of TDD configuratiin for CA in TS 38.101-1, Huawei R4-1813521, Addition of ?TC,c for single carrier Pcmax for FR1, vivo R4-1813798, Draft CR to 38.101-1: Corrections on UE additional maximum output power reduction, ZTE Corporation R4-1813811, Draft CR to 38.101-1: Corrrection to n12 reference sensitivity power levels, Skyworks Solutions Inc. R4-1813812, Band n41 spurious emission limits , Qualcomm Incorporated R4-1813813, Draft CR for TS 38.101-1: P-Max for 5G NR HPUE, CMCC R4-1814158, CR on Spurious emissions for UE co-existence, Intel Corporation R4-1814159, Draft CR for CA ACS/IBB for Bandwidth class C, Qualcomm R4-1813843, Draft CR to 38.101-1: Update of Annex F, Rohde & Schwarz R4-1813845, Correction for PI/2 PBSK requriements, Nokia Endorsed draft CR's from RAN4#89 R4-1815950, dCR on TS38.101-1 merging draft CRs from RAN4#88Bis, Qualcomm Incorporated R4-1814752, DraftCR to TS 38.101-1 pi/2 BPSK in n41, CMCC R4-1814824, n50 A-MPR, Qualcomm Incorporated R4-1814959, Changes to Max input power UL and DL configuratgions in FR1, OPPO R4-1814970, NR FR1 relative power tolerance CR, Nokia R4-1814972, A-MPR for NS_03 and NS_03U and re-formulation of NS_100, Nokia R4-1815060, draft CR for adding note about the fallback of NR CA in FR1 for TS 38.101-1, NTT DOCOMO, INC. R4-1815392, Draft CR to 38.101-1: Update to NS_04 requirements, Rohde & Schwarz R4-1815563, Draft CR to 38.101-1 on Clarification on 7.5 KHz raster shift in NR re-farmed bands, Ericsson R4-1815863, Draft CR for 38.101-1: Nominal carrier spacing for 30 kHz raster, SPRINT Corporation R4-1815898, draft CR on CA configuration on bandwidth class F, Huawei R4-1815917, draftCR on DL RMC for TS 38.101-1, Huawei R4-1816162, Draft CR on introduction of SRS switch IL in FR1, OPPO R4-1816199, Draft CR on FR1-FR2 UE-to-UE coexistence for TS38.101-1, LG Electronics France R4-1816200, Draft CR to 38.101-1 on intra-band contiguous CA configurations for FR1, ZTE Corporation R4-1816240, Transient period for SRS Antenna Switching for FR1, Qualcomm R4-1816243, Draft CR to TS38.101-1_Clarifications on MSD and UL configuration tables for inter-band CA, ZTE Corporation R4-1816466, Draft CR on some changes for SUL band combinations to TS 38.101-1, Huawei R4-1816468, Support of 7.5 kHz carrier shift for additional operating bands, Ericsson R4-1816604, TDD configuration for UE Tx test in FR1, Ericsson R4-1816663, Draft CR to 38.101-1 (5.3.4) RB alignment, Huawei R4-1816755, CR to 38.101-1: ACS and IBB intra-band contiguous CA, Intel Corporation Further changes in RAN#82 - 7.5 kHz frequency shift is specified for all FDD bands in clause 5.4.2.1 | 15.4.0 |
| 2018-12 | RAN#82 | RP-182814 | 0030 | 2 | F | Company CR on 2Rx exception for NR vehicular UE at FR1 | 15.4.0 |
| 2019-03 | RAN#83 | RP-190403 | 0034 |  | F | CR to TS 38.101-1: Implementation of endorsed draft CRs from RAN4#90 Endorced draft CR from Ran4#90 R4-1900032, Editorial corrections for 38.101-1, Qualcomm Incorporated R4-1900031, draftCR on SRS IL for CA, Qualcomm Incorporated R4-1900161, CR on Relative power tolerance, Intel Corporation R4-1900162, CR on Minimum output power, Intel Corporation R4-1900274, Draft CR to TS 38.101-1 on NR general spectrum emission mask, ZTE Corporation R4-1900275, Draft CR to TS 38.101-1 on spurious emisssion for network signalled value NS_40, NS_41 and NS_42, ZTE Corporation R4-1900424, Correction of table references and other typos, Ericsson R4-1900508, Draft CR to TS 38.101-1 on UE transmitter power and some other editorial corrections, ZTE Corporation R4-1900723, Draft CR on editorial error of TS38.101-1, LG Electronics Inc. R4-1900727, Update to PRACH EVM window length for FR1, Rohde & Schwarz R4-1900840, Draft CR for 38.101-1 modification of Transmit intermodulation requirement, Huawei R4-1900848, [RAN5 LS]Draft CR for 38.101-1: adding note for inter-band CA spurious emissions, Huawei R4-1901033, Alignment of Foob related description for 38.101-1, vivo R4-1901273, Correction of HARQ-ACK transmission timing for DL RMC for FR1 TDD SCS=60kHz, Ericsson R4-1901766, draft_CR TS 38.101-1 Correction to UL configuration for reference sensitivity, Skyworks Solutions Inc. R4-1901823, draft CR on spurious requirment for TS 38.101-1, Huawei, HiSilicon R4-1901835, draftCR on MSD for CA_n41-n78 for TS 38.101-1, Huawei R4-1901847, Draft CR for 38.101-1: Addition of default power class, Sprint Corporation R4-1901873, Receiver requirement RMC references, Qualcomm Incorporated R4-1901925, Draft CR to 38.101-1 to update and clarify Rx wide band intermod and spurious requirments for BW class C, D, E, Qualcomm Incorporated R4-1901992, Draft CR to 38.101-1. Correct FR1 NS_41 AMPR for n50 , Huawei R4-1902001, Draft CR to 38.101-1 on n41 – B40 coexistence, Qualcomm Incorporated R4-1902150, Draft CR to TS38.101-1_Clarifications on MSD and UL configuration tables for inter-band CA, ZTE Corporation R4-1902166, Tx ON/OFF time mask for FR1, Qualcomm Inc R4-1902174, Draft CR to 38.101-1: On FR1 A-MPR NS_08 for n8, Qualcomm Incorporated R4-1902175, Draft CR on AMPR requirements for NS_05U and NS_08U to TS 38.101-1, Huawei R4-1902194, [41 DL]Draft CR for 38.101-1 adding DL intra-band CA requirements for frequency less than 2700MHz, Huawei R4-1902196, Draft CR for 7.9A Spurious emissions for CA, CMCC R4-1902223, UE optional bandwidth for FR1, Nokia R4-1902225, CR to 38.101-1 on CA BW Classes fallback groups, Intel Corporation R4-1902233, Draft CR to 38.101-1: SUL clarifications, Nokia R4-1902339, Draft CR to TS 38.101-1 on FR1 extension, Ericsson R4-1902455, Completion of the Pcmax specification: additional P-max and P_NR, Ericsson R4-1902468, Draft CR: Introduction of Annex on Characteristics of the Interfering Signal, Samsung R4-1902479, Draft CR on some errors to TS 38.101-1, Huawei R4-1902480, Draft CR for 38.101-1 modification of requirements for network signalled value NS_04, Huawei R4-1902655, CR to 38.101-1 on NR Uplink RBs location, Intel Corporation R4-1901610, Draft CR for 38.101-1 REFSENS for UL MIMO , Huawei Editorial changes after RAN#83 To align the annex numbering with other specifications (TS 38.101-x series), annexes J and K were added and Change history was numbered as annex L. | 15.5.0 |
| 2019-06 | RAN#84 | RP-191240 | 0047 |  | F | CR to TS 38.101-1: Implementation of endorsed draft CRs from RAN4#90bis and RAN4#91 Endorced draft CRs from RAN4#90Bis R4-1902826, Draft CR for 38.101-1 modification of ACS test parameters case 2 for intra-band contiguous CA , Huawei R4-1902926, Draft CR to TS 38.101-1 Correction to Pcmax, Intel Corporation R4-1902975, Draft CR on PRACH and PUCCH format description for EVM in FR1, Anritsu corporation R4-1903032, Draft CR on editorial error of TS38.101-1, LG Electronics France R4-1903120, Draft CR on DL power allocation for TS 38.101-1, Intel Corporation R4-1903124, Draft CR on b41-n40 coexistence, Intel Corporation R4-1903151, Draft CR to TS38.101-1_removing DC sections, ZTE Corporation R4-1903195, Draft CR for 38.101-1: remove the bracket of UE capability "powerBoosting-pi2BPSK", Huawei R4-1903392, Draft CR for TS 38.101-1: Corrections to EVM equalizer spectrum flatness requirements, MediaTek Inc. R4-1903473, Draft CR on FREF,Shift, CMCC R4-1903508, Draft CR to TS 38.101-1 on spurious emissions for UE co-existence, ZTE Corporation R4-1904335, DraftCR TS 38.101 Corrections to NS_100 UTRA ACLR frequency band list, Skyworks Solutions Inc. R4-1904460, Draft CR for 38.101-1 CA Pcmax, Huawei R4-1904537, Draft CR for TR 38.101-1 correction of A-MPR for NS_04, Huawei R4-1904554, Draft CR to 38.101-1: FR1 power dynamics DTX removal, Qualcomm Incorporated R4-1904927, Draft CR to clarify frequency of carrier leakage in RBs for FR1, Anritsu corporation R4-1904928, Draft CR to TS 38.101-1 on description of UE additional output power reduction, ZTE Corporation R4-1904929, draft Rel-15 CR for editorial corrections in 38.101-1, Ericsson R4-1904941, draft CR to 38.101-1 Correction to Pi/2 BPSK power boosting, Intel Corporation R4-1904957, Draft CR for TR38.101-1 – Update to EVM averaging, Rohde & Schwarz R4-1904958, Draft CR for TR38.101-1 – Update to spectrum flatness, Rohde & Schwarz R4-1904967, Draft CR for 38.101-1 definition of Maximum input level for intra-band contiguous CA, Huawei R4-1904969, Draft CR for 38.101-1: editoral correction, Huawei R4-1904987, Draft CR for correction on TS38.101-1, CATT Endorced draft CRs from RAN4#91 R4-1905339 removal of A-MPR brackets in FR1 Nokia R4-1905503 Change description 4.2(d) in Applicability of minimum requirements for TS 38.101-1 vivo R4-1905524 [Rx]Draft CR for 38.101-1 Removing the brackets in Rx requirements Huawei R4-1905526 [Rx]Draft CR for 38.101-1 defining NBB requirements<2.7GHz Huawei R4-1905772 Draft CR to TS38.101-1 Almost contiguous MPR Intel Corporation R4-1905795 Correction to a description of PRB for in-band emission in FR1 Anritsu Corporation R4-1905797 Correction to power control in FR1 Anritsu Corporation R4-1906140 draft CR for TS 38.101-1 Rx requirement for CA Huawei R4-1906153 Draft CR for TS 38.101-1: Editorial corrections to intra-band contiguous CA ACS and in-band blocking requirements MediaTek Inc. R4-1906154 Draft CR for TS 38.101-1: Adding symbol definitions for intra-band contiguous CA Rx maximum input level and ACS requirements MediaTek Inc. R4-1906871 Draft CR for TS 38.101-1 UE optional bandwidth for FR1 Huawei R4-1907131 Draft CR to 38.101-1. Clarification to FR1 NS_43 AMPR frequency ranges Qualcomm Incorporated R4-1907135 Draft CR to 38.101-1 rel. 15 to fix missing Exceptions for Out-of-band Blocking Apple R4-1907419 Draft CR for TS 38.101-1: Editorial improvement to EVM equalizer spectrum flatness requirements for Pi/2 BPSK MediaTek Inc. R4-1907429 Draft CR to TS38.101-1 A-MPR for Inter-band CA Intel Corporation R4-1907434 [Rx]Draft CR for 38.101-1 modifying characteristics of the interfering signal in Annex D Huawei R4-1907435 Draft CR to TS38.101-1_introduction of n41C and corrections on Rx requirements for NR intra-band contiguous CA ZTE Corporation R4-1907439 Draft CR to TS 38.101-1 on CA bandwidth class description ZTE Corporation R4-1907471 Draft CR to 38.101-1. Clarify all RB reference so transmission BW applies for all SCS Qualcomm Incorporated R4-1907474 Draft CR for TS 38.101-1 Correction of channel bandwidth set for NR CA Huawei R4-1907477 Draft CR to TS 38.101-1 on maximum aggregated bandwidth for NR CA configurations ZTE Corporation R4-1907481 Correction of RefSens exceptions due to UL harmonic interference for NR CA in 38.101-1 vivo R4-1907687 Correction to CA carrier spacing Ericsson | 15.6.0 |
| 2019-06 | RAN#84 | RP-191248 | 0037 | 1 | B | Introduction of n48 in to TS 38.101-1 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0040 |  | B | CR to REL-16 TS 38.101-1: Implementation of endorsed draft CRs on NR combinations and dual Connectivity combinations | 16.0.0 |
| 2019-06 | RAN#84 | RP-191242 | 0041 | 1 | B | CR to TS 38.101-1: Introduction of band n14 – Endorsed R4-1904008 in RAN4#90b | 16.0.0 |
| 2019-06 | RAN#84 | RP-191246 | 0042 | 1 | B | CR to TS 38.101-1: Introduction of band n30 + editorial in table 7.6.2-2 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191244 | 0043 | 1 | B | CR to introduce n18 to TS 38.101-1 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191250 | 0044 | 1 | B | n65 introduction to 38.101-1 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191251 | 0045 |  | B | Addition channel bandwidth of 30MHz for n50 in TS 38.101-1 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191252 | 0046 | 1 | B | Introduction of a new NR band for LTE/NR spectrum sharing in Band 41/n41 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0048 |  | B | CR on introducing NR inter-band CA of 3DL Bands and 1UL band | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0049 |  | B | CR to reflect the completed NR inter-band CA/DC combinations into Rel16 TS38.101-1 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0050 |  | B | CR to reflect the completed NR inter-band CA/DC combinations for 3 bands DL with 2 bands UL into Rel16 TS38.101-1 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0051 |  | B | CR introduction completed band combinations 38.716-01-01 -> 38.101-1 | 16.0.0 |
| 2019-09 | RAN#85 | RP-192038 | 0052 |  | F | Correction to FR1 ASEM NS_27 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192032 | 0053 |  | B | Addition of NS information on 30MHz support for n41 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192031 | 0054 | 1 | B | Addition of new channel bandwidths for n7 into TS 38.101-1 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0055 |  | B | CR on introducing NR intra-band CA for 3DL Bands and 1UL band | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0057 | 1 | F | Minor corrections of intra-band non-contiguous CA operating bands in TS 38.101-1 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0058 | 1 | F | Adding DeltaFHD for CA_n1-n77 refersense requirments | 16.1.0 |
| 2019-09 | RAN#85 | RP-192032 | 0060 |  | B | CR to introduce 30MHz bandwidth of n41 into TS 38.101-1 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192026 | 0061 | 1 | B | Characteristics of Interfering signal for Contiguous Intra-band CA Class B | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0062 | 1 | F | Correction Inter-band CA configurations | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0063 | 1 | F | Finalizing Generic Intra-band Contiguous CA Class B requirements | 16.1.0 |
| 2019-09 | RAN#85 | RP-192034 | 0064 | 1 | B | n29 introduction to 38.101 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0065 |  | F | [SUL] CR on SUL band combinations into Rel-16 TS 38.101-1 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192029 | 0066 |  | B | CR on Introduction of SUL band n89 into Rel-16 TS 38.101-1 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192046 | 0068 | 2 | F | Correction to Band n66 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192026 | 0070 | 1 | F | CR to 38.101-1. Revamp CA ACS and IBB tables to differentiate by band numbers and not frequency | 16.1.0 |
| 2019-09 | RAN#85 | RP-192038 | 0071 |  | F | CR to 38.101-1. Add missing AMPR to NS27 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192026 | 0072 |  | B | CR for 38.101-1 Rx requirement for NR intra-band non-contiguous CA | 16.1.0 |
| 2019-09 | RAN#85 | RP-192036 | 0073 |  | F | CR for 38.101-1: Correction to the Spurious Emission for UE Coexistence table for n14 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192037 | 0074 |  | F | CR for 38.101-1: Correction to the Spurious Emission for UE Coexistence table for n30 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0075 |  | B | CR introduction completed band combinations 38.716-01-01 -> 38.101-1 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0076 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-1 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0077 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-1 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192049 | 0079 |  | A | CR to TS 38.101-1: Implementation of endorsed draft CRs from RAN4#92 (Rel-16) - Mirrors changes in R4-1910350 (of RAN4#92) for Rel-15 TS 38.101-1 | 16.1.0 |
| 2019-12 | RAN#86 | RP-193022 | 0097 |  | F | CR to align NS27 AMPR to CA_NS_10 AMPR for 40MHz BW at the center of band 48. | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0099 |  | A | CR for 38.101- RX Out-of-Band Blocking for B38 and B41 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0103 |  | A | CR for 38.101-1 n39 AMPR | 16.2.0 |
| 2019-12 | RAN#86 | RP-193013 | 0105 | 1 | B | Introduction of 2010-2025MHz SUL band into Rel-16 TS 38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193015 | 0110 |  | B | Addition of 25, 30 and 40 MHz to NR band n25 in TS 38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0112 |  | A | Sync raster to SSB resource element mapping | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0114 |  | A | CR to TS 38.101-1 Almost contiguous A-MPR (R16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0118 |  | A | CR to 38.101-1 (Rel-16) to clarify measurement interval and observation window on frequency error | 16.2.0 |
| 2019-12 | RAN#86 | RP-193020 | 0119 |  | D | Format misalignment on NS_47 protection requirement table | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0121 |  | A | CR to TS 38.101-1: Replace CBW with symbols defined in the specification | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0124 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0125 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0126 |  | F | CR to remove square brackets for n90 in TS38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0128 |  | A | CR for TS38.101-1, Clarification and Editorial corrections | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0132 |  | B | Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0133 |  | B | Adding band n71 and n28 to 4 Rx antenna ports support in 38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0137 |  | A | CR for TS 38.101-1: Editorial correction for n2 uplink configuration note index in Table 7.3.2-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0138 |  | A | CR to TS 38.101-1 on A-MPR table cleanup (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0140 |  | A | CR for TS 38.101-1: Removing CA configurations for CA_n77D/E, CA_n78D/E, and CA_n79D/E | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0144 |  | A | CR for TS 38.101-1: Fix out-of-band blocking issue for n50 and n75 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0146 |  | A | CR to TS 38.101-1 on corrections to channel raster entries for NR band (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0150 |  | A | CR to transmit modulation quality in FR1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0151 |  | F | Corrections Intra-band CA simultaneous TX/RX requirements | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0153 |  | F | Removal of brackets from reciever requirements in 38.101-1 REL-16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0155 |  | B | Extension of CA BW class B | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0157 |  | A | CR to 38.101-1: Editorial correction of UL RMCs | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0164 |  | B | CR for 38.101-1 introduce SUL band combination CA_n78(2A)_SUL_n78A-n86A | 16.2.0 |
| 2019-12 | RAN#86 | RP-193010 | 0165 |  | F | CR for 38.101-1: add BCS1 configurations for CA_n78(2A) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193017 | 0166 |  | B | CR to 38.101-1 - Band n75 - wider CBW | 16.2.0 |
| 2019-12 | RAN#86 | RP-193018 | 0167 |  | B | CR for TS 38.101: adding wider channel bandwidths | 16.2.0 |
| 2019-12 | RAN#86 | RP-193016 | 0168 |  | B | CR to 38.101-1: Addition of channel bandwidth for band n38 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0169 |  | B | CR introduction completed band combinations 38.716-01-01 -> 38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0170 |  | B | CR introduction completed band combinations 38.716-04-01 -> 38.101-1 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193021 | 0171 |  | C | CR for 38.101-1: Making 90 MHz channel bandwidth mandatory for n41, n78 and n90 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193020 | 0172 |  | B | CR for 38.101-1: adding 30 MHz CHBW to NS_04 for n41 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0174 |  | A | CR to 38.101-1-g10 Corrections to Transient Time Masks | 16.2.0 |
| 2019-12 | RAN#86 | RP-193010 | 0176 | 1 | F | CR for intra-band DL contiguous CA RF requirements | 16.2.0 |
| 2019-12 | RAN#86 | RP-193010 | 0179 |  | B | Introduction of almost contiguous MPR for PC2 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193029 | 0180 |  | A | CR for asynchronous operation for NR CA n78-n79 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193028 | 0182 |  | A | CR to 38.101-1: DMRS Exceptions | 16.2.0 |
| 2020-03 | RAN#87 | RP-200408 | 0191 |  | F | Corrections to n65 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200377 | 0201 | 1 | F | CR for 38.101-1 to introduce BCS1 for CA_n77C and CA_n78C | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0203 |  | A | CR to TS 38.101-1 on corrections to network signalling value (Rel-16) | 16.3.0 |
| 2020-03 | RAN#87 | RP-200484 | 0208 |  | A | CR for 38.101- n39 NS flag change due to conflict | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0210 |  | A | Mirror CR for 38.101-1: n41 and n25 corrections | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0211 | 2 | F | CR for 38.101-1: Corrections to intra-band CA tables | 16.3.0 |
| 2020-03 | RAN#87 | RP-200387 | 0212 |  | F | CR for 38.101-1: Missing 70 MHz for NS_01 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200381 | 0215 |  | B | CR for 38.101-1: Introduction of n26 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0216 |  | F | CR to TS 38.101-1: Corrections on MSD tables for CA_n20-n78 and CA_n66-n78 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0218 |  | A | CR to TS 38.101-1: corrections on ACS for intra-band contiguous CA | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0219 | 1 | F | CR to TS 38.101-1: Improvement on NR 3DL inter-band CA combination | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0221 |  | A | CR to TS 38.101-1: Replace CBW with symbols defined in the specification. NOTE: The CR is based on something else than the latest version of the specification and therefore it is not implemented, e.g. Tables 6.2.3.1-1, 7.6.2-2 and Table 7.6.2-4 in CR0221 are different compared to those in 38.101-1 v16.2.0. | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0222 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-1 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0223 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-1 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0224 | 1 | B | Introduction of n53 into TS 38.101-1 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0229 |  | A | CR for TS38.101-1, Remove notes for UE channel bandwidth | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0231 |  | A | CR for TS38.101-1, Correction of IE RF-Parameters name of maxUplinkDutyCycle | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0234 | 1 | B | Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200377 | 0239 | 1 | F | CR for TS 38.101-1: Corrections for n48 receiver requirements | 16.3.0 |
| 2020-03 | RAN#87 | RP-200386 | 0240 | 1 | B | CR for TS 38.101: adding wider channel bandwidths for n66 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200392 | 0241 | 1 | F | Maintenance on the UE BW for n92 and n94 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200392 | 0242 |  | F | Maintenance on the Rx-Tx separation terms | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0244 |  | A | CR for 38.101-1: to remove fallback group 1 in table 5.5A.1-1 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200389 | 0247 |  | F | CR for 38.101-1: to correct CA_n8A-n75A REFSENS | 16.3.0 |
| 2020-03 | RAN#87 | RP-200384 | 0249 | 1 | B | CR for 38.101-1: to introduce UE RF requirements for adding wider channel bandwidth in band n28 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200383 | 0250 | 1 | B | CR to 38.101-1 Band n1 - wider CBW - Additional Channel BW | 16.3.0 |
| 2020-03 | RAN#87 | RP-200385 | 0252 | 1 | B | CR to 38.101-1 Band n38 - wider CBW - Additional Channel BW | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0260 | 1 | F | Editorial corrections | 16.3.0 |
| 2020-03 | RAN#87 | RP-200377 | 0263 |  | F | CR for alomost contiguous allocation applicability | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0265 | 1 | A | CR for inter-band CA Tx requirement | 16.3.0 |
| 2020-03 | RAN#87 | RP-200377 | 0266 | 1 | F | CR for intra-band CA configuration and DL RF requirements | 16.3.0 |
| 2020-03 | RAN#87 | RP-200391 | 0273 |  | F | CR for 38.101-1: Mandatory support for n41 by UEs that support n90 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0275 |  | A | CR for [agreed] asynchronous operation for NR CA n78-n79 NOTE: The CR is based on something else than the latest version of the specification and therefore it is not implemented, e.g. Tables 6.2A.4.2.3-1, Table 7.3A.6-1, 7.3A.6.2 and table notes are different compared to those in 38.101-1 v16.2.0. | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0280 |  | F | CR for 38.101-1: delta Tib corrections | 16.3.0 |
| 2020-03 | RAN#87 | RP-200394 | 0281 |  | A | Removal of unnecessary definition of offsetmax,IMD3 from Table 6.2.3.2-1 | 16.3.0 |
| 2020-06 | RAN#88 | RP-201338 | 0293 | 4 | B | CR to TS 38.101-1: Switching time mask between two uplink carriers in UL CA and SUL | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0294 |  | F | Corrections to CA n48 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0300 |  | A | CR to asymmetric CBW operation in FR1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0302 |  | A | CR on ACLR MBW definition in FR1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0305 |  | B | Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0307 |  | F | CR Coexistence cleanup for 38101-1 Rel16 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0310 |  | A | CR to TS 38.101-1 R16: corrections on ACS for intra-band contiguous CA | 16.4.0 |
| 2020-06 | RAN#88 | RP-200966 | 0311 |  | F | CR for TS 38.101-1: UL harmonic MSD and OOBB exception | 16.4.0 |
| 2020-06 | RAN#88 | RP-200981 | 0315 |  | F | Update 4Rx Requirement for Band n30 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200958 | 0317 |  | B | CR on NR V2X UE RF requirements for single carrier in TS38.101-1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0327 |  | A | Maintenance CR to 38101-1 on relative power tolerance R16 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200974 | 0329 |  | F | Endorsed CR on default AMPR signaling for n91 n92 n93 and n94 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0331 |  | A | Update of CSI-RS definition for FR1 DL RMCs | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0335 |  | A | Correction to FR1 QPSK UL RMC | 16.4.0 |
| 2020-06 | RAN#88 | RP-200966 | 0336 |  | B | CR to TS38.101-1: Introduction of NR DC(Clauses 3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0338 |  | A | CR to TS 38.101-1: Correction on the CA nominal channel spacing | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0340 |  | A | CR to TS 38.101-1: Replace CBW with symbols defined in the specification. | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0341 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0345 |  | A | 30k SSB SCS for n50 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0347 |  | A | Addition of 30k SSB SCS for Band n38 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0354 |  | A | IBE measurements for Pi/2 BPSK with spectrum shaping | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0357 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0360 |  | B | CR introduction completed band combinations 38.716-01-01 - | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0361 |  | B | CR introduction completed band combinations 38.716-04-01 - | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0364 |  | B | CR on Introduction of completed SUL band combinations into TS 38.101-1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-201045 | 0365 |  | F | CR for 38.101-1 to introduce BCS2 for CA_n78(2A). | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0367 |  | A | CR for 38.101-1 to remove the NR CA configuration for REFSENS exception due to cross band isolation for CA (mirror CR) | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0369 |  | A | CR for 38.101-1 to add the REFSENS exception for inter band CA with SDL (mirror CR) | 16.4.0 |
| 2020-06 | RAN#88 | RP-200979 | 0373 |  | F | CR on introduce delta-MPR for inter-band CA in band n28 and review value with brackets | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0379 |  | A | IBE requirement for almost contiguous allocations | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0385 |  | A | OOB blocking for n70 adjacent to n25 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0394 |  | F | CR for TS 38.101-1 UE co-existence correction (R16) | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0396 |  | F | CR for 38.101-1 RFC corrections (R16) | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0400 |  | A | TS38.101-1 CR on 30KHz SSB SCS for n40(Rel-16) | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0318 | 1 | F | CR to add simultaneous RXTX capability for CA_n41-n79 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0404 |  | A | CR for 38.101-1: to add some missing sub-clause title for NR inter-band CA | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0343 | 1 | A | CR for [agreed] asynchronous operation for NR CA n78-n79 | 16.4.0 |
| 2020-06 | RAN#88 | RP-201045 | 0387 | 1 | B | CR on FR1 UL contiguous CA requirement | 16.4.0 |
| 2020-06 | RAN#88 | RP-200974 | 0325 | 1 | F | CR on blocking requirements for n91 n92 n93 and n94 | 16.4.0 |
| 2020-06 | RAN#88 | RP-201045 | 0380 | 1 | B | Addition of mutual UE coexistence between US bands and NR Band n77 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200977 | 0356 | 1 | B | CR for TS 38.101: adding 50 MHz CBW for n1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200980 | 0358 | 1 | B | CR to TS 38.101-1 - Add 40 MHz CBW in band n3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200982 | 0359 | 1 | B | CR to TS 38.101-1 - Add 50 MHz CBW in band n65 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0405 |  | F | Corrections of UE co-ex tables for Japan-related bands (R16) | 16.4.0 |
| 2020-06 | RAN#88 | RP-201045 | 0320 | 2 | B | CR to 38.101-1: Introduce an operating band list and NR bands to UL MIMO | 16.4.0 |
| 2020-06 | RAN#88 | RP-200966 | 0362 | 1 | B | CR to 38.101-1 for Introduction of requirements for NR-DC | 16.4.0 |
| 2020-09 | RAN#89 | RP-201495 | 0407 | 1 | F | Correction to FR1 UL contiguous CA MPR regions | 16.5.0 |
| 2020-09 | RAN#89 | RP-201506 | 0409 |  | F | CR for n26 AMPR for 256QAM | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0411 |  | A | OOB blocking for Inter-band CA | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0416 | 1 | F | Correction to ASEM for NS_27 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201507 | 0419 |  | F | Introduction of UE PC2 for NR band n40 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201502 | 0422 | 1 | B | Introduction of LTE/NR spectrum sharing in band 48/n48 frequency range | 16.5.0 |
| 2020-09 | RAN#89 | RP-201507 | 0423 |  | F | Coexistence cleanup for 38101-1 Rel16 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201506 | 0424 |  | D | CR Editorial cleanup of band combination tables for 38101-1 Rel16 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0426 |  | A | CR to TS 38.101-1: corrections on narrow band blocking for intra-band contiguous CA | 16.5.0 |
| 2020-09 | RAN#89 | RP-201492 | 0428 | 1 | F | CR for TS 38.101-1: Removal of table 6.5E.3.4.3-1 and table 6.5E.3.4.3-2 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201503 | 0432 | 1 | B | CR for 38.101-1: Introduction of Power Class 1.5 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201488 | 0433 | 1 | B | CR to TS38.101-1 on introduction of Uplink Full Power Transmission | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0435 |  | A | Corrections of Japan-related CA co-ex tables for REL-15 combo | 16.5.0 |
| 2020-09 | RAN#89 | RP-201492 | 0437 | 1 | F | Correction on 5G V2X UE RF requirements in rel-16 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201495 | 0438 | 2 | B | A-MPR definition for CA_n48B, CA_n41B and CA_n41C | 16.5.0 |
| 2020-09 | RAN#89 | RP-201495 | 0439 |  | F | CR Restoring the clause structure of NR FR1 uplink contiguous intraband CA | 16.5.0 |
| 2020-09 | RAN#89 | RP-201492 | 0440 | 1 | F | CR on TS38.101-1 for NR V2X | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0442 |  | A | 30k SSB SCS for Band n34 and n39 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0444 |  | F | Correction for 5 MHz channel bandwidth for n50 and introduction of Annex H | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0458 |  | A | CR for 38.101-1 FRC corrections (R16) | 16.5.0 |
| 2020-09 | RAN#89 | RP-201506 | 0459 | 1 | F | CR for 38.101-1 to remove PHS system and 860~890 protection for NR CA band combination with band n1 and band n8 | 16.5.0 |
| 2020-09 | RAN#89 | RP-201506 | 0460 | 1 | F | CR for 38.101-1 to add the missing region for NS_18 and maintenance the ?mprc | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0462 |  | A | CR for 38.101-1 to add the missing MSD for CA_n41A-n78A | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0465 |  | A | Correction to configured power with allowance for SRS switching | 16.5.0 |
| 2020-09 | RAN#89 | RP-202117 | 0466 |  | B | Introduce UE NR-U requirements to 38.101-1 including Band n46 (5 GHz) and Band n96 (6 GHz) | 16.5.0 |
| 2020-09 | RAN#89 | RP-201495 | 0468 | 1 | F | CR for intra-band UL CA non-contiguous CA requirement | 16.5.0 |
| 2020-09 | RAN#89 | RP-201495 | 0469 | 1 | F | CR for correction on intra-band UL CA contiguous CA requirement | 16.5.0 |
| 2020-09 | RAN#89 | RP-201495 | 0470 | 1 | F | CR for intra-band UL contiguous CA DC location | 16.5.0 |
| 2020-09 | RAN#89 | RP-201495 | 0471 | 1 | B | CR for intra-band UL CA non-contiguous CA requirement | 16.5.0 |
| 2020-09 | RAN#89 | RP-201507 | 0480 | 1 | F | CR to 38.101-1 - Correction to CA BCS and cross band isolation MSD tables | 16.5.0 |
| 2020-09 | RAN#89 | RP-201512 | 0483 |  | A | Correction of applicability of 2Rx requirements | 16.5.0 |
| 2020-09 | RAN#89 | RP-201488 | 0486 | 2 | B | CR to add PC3 Pi/2 BPSK DMRS for IE powerBoostPi2BPSK = 0 | 16.5.0 |
| 2020-09 | RAN#89 | RP-202098 | 0499 | 1 | C | 7.5 kHz UL shift for LTE/NR spectrum sharing in Band 38/n38 | 16.5.0 |
| 2020-12 | RAN#90 | RP-202440 | 0492 | 1 | F | CR CatF n7 NS_46 AMPR and coexistence | 16.6.0 |
| 2020-12 | RAN#90 | RP-202427 | 0498 | 1 | F | Correction on 5G V2X UE RF requirements in TS38.101-1 in rel-16 | 16.6.0 |
| 2020-12 | RAN#90 | RP-202438 | 0506 |  | F | n53 bracket removal | 16.6.0 |
| 2020-12 | RAN#90 | RP-202442 | 0507 | 2 | F | A-MPR definition for CA_n7B, CA_n48B, CA_n41B and CA_n41C | 16.6.0 |
| 2020-12 | RAN#90 | RP-202485 | 0512 |  | A | CR to TS38.101-1 on DC location correction | 16.6.0 |
| 2020-12 | RAN#90 | RP-202509 | 0518 |  | F | Coexistence cleanup for 38.101-1 Rel16 | 16.6.0 |
| 2020-12 | RAN#90 | RP-202509 | 0524 | 1 | F | CR to TS 38.101-1 on simplification for inter-band CA configuration | 16.6.0 |
| 2020-12 | RAN#90 | RP-202427 | 0525 |  | F | CR on TS38.101-1 for NR V2X | 16.6.0 |
| 2020-12 | RAN#90 | RP-202485 | 0527 |  | A | CR to TS 38.101-1[R16]: Clarification of non-simultaneous Rx/Tx operation for CA_n77-n79 and CA_n78-n79 in TS 38.101-1. | 16.6.0 |
| 2020-12 | RAN#90 | RP-202442 | 0533 | 1 | F | CR to 38.101-1 Add requirement on the UL CA configurations with no DL interruption | 16.6.0 |
| 2020-12 | RAN#90 | RP-202509 | 0534 |  | F | Editorial correction on section 5.2C to 38.101-1 R16 | 16.6.0 |
| 2020-12 | RAN#90 | RP-202427 | 0535 | 1 | F | CR on V2X bands reference table | 16.6.0 |
| 2020-12 | RAN#90 | RP-202509 | 0536 | 1 | F | CR on sum of power for multiple transmit connectors | 16.6.0 |
| 2020-12 | RAN#90 | RP-202428 | 0540 |  | F | CR for 38.101-1 to correct the notation of SUL band combinations in order to be aligned with 38.101-3 | 16.6.0 |
| 2020-12 | RAN#90 | RP-202485 | 0542 |  | A | CR for 38.101-1 to adjust the structure of NR CA REFSENS (Rel-16) | 16.6.0 |
| 2020-12 | RAN#90 | RP-202509 | 0544 |  | F | Reference measurement channels for 70 MHz CBW | 16.6.0 |
| 2020-12 | RAN#90 | RP-202428 | 0547 |  | F | Correction to supported channel bandwidths per SUL_n41A-n81A | 16.6.0 |
| 2020-12 | RAN#90 | RP-202414 | 0550 | 3 | F | Correction to the intra-cell guard band definition for wideband operation | 16.6.0 |
| 2020-12 | RAN#90 | RP-202414 | 0552 | 1 | F | Correction to receiver requirements for shared spectrum channel access | 16.6.0 |
| 2020-12 | RAN#90 | RP-202442 | 0556 |  | F | CR Correction to NS_27 and Band 10 protection 38101-1 Rel16 | 16.6.0 |
| 2020-12 | RAN#90 | RP-202428 | 0557 | 1 | F | CR for editorial corrections 38.101-1 | 16.6.0 |
| 2020-12 | RAN#90 | RP-202414 | 0558 | 2 | F | Removal of square brackets for 38.101-1 NR-U | 16.6.0 |
| 2020-12 | RAN#90 | RP-202509 | 0562 |  | F | CR to for 38.101-1: CA uplink power clarification | 16.6.0 |
| 2020-12 | RAN#90 | RP-202509 | 0563 |  | D | CR for 38.101-1: Editorial corrections | 16.6.0 |
| 2020-12 | RAN#90 | RP-202427 | 0566 | 1 | F | CR for 38.101-1 NR V2X FRC | 16.6.0 |
| 2020-12 | RAN#90 | RP-202485 | 0571 |  | A | CR for TS 38.101-1: correction of delta Tib for UE supporting multiple band combinations (R16) | 16.6.0 |
| 2020-12 | RAN#90 | RP-202442 | 0574 | 1 | B | CR for intra-band UL CA non-contiguous CA requirement | 16.6.0 |
| 2020-12 | RAN#90 | RP-202485 | 0581 |  | A | CR for 38.101-1 on corrections for AMPR-Rel-16 | 16.6.0 |
| 2020-12 | RAN#90 | RP-202485 | 0584 |  | A | CR to DMRS position in UL RMC for FR1 | 16.6.0 |
| 2020-12 | RAN#90 | RP-202456 | 0408 | 3 | B | LTE/NR spectrum sharing in Band 40/n40 | 17.0.0 |
| 2020-12 | RAN#90 | RP-202451 | 0499 |  | B | Introduction of 1880-1920MHz SUL band into Rel-17 TS 38.101-1 | 17.0.0 |
| 2020-12 | RAN#90 | RP-202452 | 0500 |  | B | introduction of 2300-2400MHz SUL band into Rel-17 TS 38.101-1 | 17.0.0 |
| 2020-12 | RAN#90 | RP-202450 | 0503 | 1 | B | CR for TS 38.101-1, Introduce new band combination of V2X_n39A-n47A and V2X_n40A-n47A | 17.0.0 |
| 2020-12 | RAN#90 | RP-202468 | 0504 |  | B | CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1 | 17.0.0 |
| 2020-12 | RAN#90 | RP-202471 | 0513 |  | B | CR on introduction of completed NR CA/DC combs with 4DL/2UL within FR1 | 17.0.0 |
| 2020-12 | RAN#90 | RP-202472 | 0514 |  | B | CR on Introduction of completed SUL band combinations into TS 38.101-1 | 17.0.0 |
| 2020-12 | RAN#90 | RP-202448 | 0543 | 1 | B | CR to TS 38.101-1: introduction of NR band n13 | 17.0.0 |
| 2020-12 | RAN#90 | RP-202455 | 0545 |  | B | CR to 38.101-1 Introduce band combination requirements for PC2 CA_n1A-n78A | 17.0.0 |
| 2020-12 | RAN#90 | RP-202453 | 0546 | 1 | B | Big CR to 38.101-1 - Additional Channel BW | 17.0.0 |
| 2020-12 | RAN#90 | RP-202466 | 0548 |  | B | CR introduction completed band combinations Rel-17 NR Intra-band - | 17.0.0 |
| 2020-12 | RAN#90 | RP-202470 | 0549 |  | B | CR introduction completed band combinations NR Inter-band 4 bands CA - | 17.0.0 |
| 2020-12 | RAN#90 | RP-202467 | 0585 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1 | 17.0.0 |
| 2020-12 | RAN#90 | RP-202469 | 0586 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-1 | 17.0.0 |
| 2021-03 | RAN#91 | RP-210190 | 0589 |  | A | PC1 and PC3 Updates for Band n14 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0594 | 1 | A | 38.101 Void clean up R17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210097 | 0604 | 2 | B | CR for TS 38.101-1 introduction of NR band n24 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210072 | 0606 |  | A | CR on editorial correction on V2X operation in TS38.101-1 in Rel-17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210178 | 0607 |  | B | CR on Introduction of completed SUL band combinations into TS 38.101-1 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210096 | 0609 | 2 | B | CR to 38101-1 on introducing new SUL band n99 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0612 |  | A | CR for TS38 101-1 Rel-17 Correction for definition of P-MPR | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0614 |  | A | CR for TS38 101-1 Rel-17 Correction of condition for MPR and delta MPR | 17.1.0 |
| 2021-03 | RAN#91 | RP-210094 | 0616 | 1 | B | CR for TS 38.101-1, Introduce new band combination of V2X_n41A-n47A | 17.1.0 |
| 2021-03 | RAN#91 | RP-210179 | 0617 |  | B | CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210094 | 0625 |  | F | Revision of inter-band V2X con-currency table for V2X_n39A-n47A and V2X_n40A-n47A | 17.1.0 |
| 2021-03 | RAN#91 | RP-210092 | 0628 | 1 | B | Switching time mask for 2Tx-2Tx switching between two carriers and 1Tx-2Tx/2Tx-2Tx switching between two bands in Rel-17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210082 | 0630 |  | F | CR for TS 38.101-1: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-17) | 17.1.0 |
| 2021-03 | RAN#91 | RP-210092 | 0631 |  | B | CR on introducing NR SUL bands n80 to UL-MIMO configuration | 17.1.0 |
| 2021-03 | RAN#91 | RP-210091 | 0633 |  | A | CR for 38.101-1: Update of missing fallback NR-DC combinations Rel-17 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210184 | 0636 | 1 | B | CR on introduction of completed NR CA/DC combs with 4DL/2UL within FR1 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210091 | 0642 |  | A | CR on introduction of shorter Transient Period Capability | 17.1.0 |
| 2021-03 | RAN#91 | RP-210100 | 0657 |  | B | CR introduction completed band combinations Rel-17 NR Intra-band - | 17.1.0 |
| 2021-03 | RAN#91 | RP-210091 | 0660 |  | A | CR for 38.101-1 to add missing spurious emissions for band n38 UE co-existence (Rel-17) | 17.1.0 |
| 2021-03 | RAN#91 | RP-210084 | 0663 |  | A | CR to TS 38.101-1: system parameters maintenance for NR-U | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0665 |  | A | Simplification of n70 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210074 | 0669 |  | A | CR for 38.101-1: Add CA_n25A-n41(2A)-n71A which was missing in the CR implementation | 17.1.0 |
| 2021-03 | RAN#91 | RP-210189 | 0670 | 1 | F | Big CR to TS 38.101-1 - New CBW Basket WI | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0674 |  | A | CR to TS38.101-1: Correction on applicability of minimum requirements | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0677 |  | A | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0679 |  | A | CR to TS38.101-1: Correction on configured transmitted power requirement | 17.1.0 |
| 2021-03 | RAN#91 | RP-210100 | 0680 | 1 | B | Rel-17 CR 38.101-1 for improvements Intra-band tables | 17.1.0 |
| 2021-03 | RAN#91 | RP-210101 | 0681 |  | F | Rel-17 CR 38.101-1 for corrections NR CA 2, 3 and 4 band configuration tables | 17.1.0 |
| 2021-03 | RAN#91 | RP-210099 | 0688 |  | B | CR for TS 38.101-1 Introduce NR SUL bands to PC3 UL-MIMO configuration | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0690 |  | A | Missing parent clause for NR-DC PCMAX | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0692 |  | A | Corrections to PCMAX for UL CA | 17.1.0 |
| 2021-03 | RAN#91 | RP-210082 | 0695 |  | A | CR CA_n7B REFSENS | 17.1.0 |
| 2021-03 | RAN#91 | RP-210117 | 0699 |  | A | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | 17.1.0 |
| 2021-03 | RAN#91 | RP-210082 | 0701 |  | A | CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements | 17.1.0 |
| 2021-03 | RAN#91 | RP-210091 | 0703 |  | A | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | 17.1.0 |
| 2021-03 | RAN#91 | RP-210091 | 0713 |  | A | CR on TS 38.101-1 NS_49 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210176 | 0715 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210181 | 0716 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-1 | 17.1.0 |
| 2021-03 | RAN#91 | RP-210101 | 0717 |  | B | Introduction of specific Pcmax requirements for inter-band CA category A-B combos | 17.1.0 |