| 3GPP TS 38.133 V16.7.0 (2021-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	50

3.6	Applicability of requirements in this specification version	51

3.6.1	RRC connected state requirements in DRX	51

3.6.2	Number of serving carriers	52

3.6.2.1	Number of serving carriers for SA	52

3.6.2.2	Number of serving carriers for EN-DC	52

3.6.2.3	Number of serving carriers for NE-DC	52

3.6.2.4	Number of serving carriers for NR-DC	52

3.6.3	Applicability for intra-band FR2	53

3.6.4	Applicability for FR2 UE power classes	53

3.6.5	Applicability for SDL bands	53

3.6.6	Applicability of requirements for NGEN-DC operation	53

3.6.7	Applicability of QCL	53

3.6.9	Applicability of requirements for scheduling availability	54

3.6.10	Applicability of requirements for measurement restrictions	54

4	SA: RRC_IDLE state mobility	54

4.1	Cell Selection	54

4.2	Cell Re-selection	54

4.2.1	Introduction	54

4.2.2	Requirements	54

4.2.2.1	UE measurement capability	54

4.2.2.2	Measurement and evaluation of serving cell	55

4.2.2.3	Measurements of intra-frequency NR cells	55

4.2.2.4	Measurements of inter-frequency NR cells	57

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	58

4.2.2.6	Maximum interruption in paging reception	60

4.2.2.7	General requirements	60

4.2.2.8	Minimum requirement at transitions	61

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	61

4.2.2.9.1	Introduction	61

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	61

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	62

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	62

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	63

4.2.2.10.1	Introduction	63

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	63

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	64

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	64

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	65

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	65

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	66

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	66

4.2A	Cell Re-selection when subject to CCA	67

4.2A.1	Introduction	67

4.2A.2	Requirements	67

4.2A.2.1	UE measurement capability	67

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	67

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	68

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	69

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	71

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	71

4.2A.2.7	General requirements	71

4.3	Minimization of Drive Tests (MDT)	72

4.3.1	Introduction	72

4.3.2	Measurement Requirements	72

4.3.3	Requirements for Relative Time Stamp Accuracy	72

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	73

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	73

4.4	Idle Mode CA/DC Measurements	73

4.4.1	Introduction	73

4.4.2	Measurement Requirements	73

4.4.2.1	Detected cell requirement during state transition and Idle mode	73

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	74

4.4.2.3	Measurements on serving cell	75

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	75

5	SA: RRC_INACTIVE state mobility	77

5.1	Cell Re-selection	77

5.1.1	Introduction	77

5.1.2	Requirements	77

5.1.2.1	UE measurement capability	77

5.1.2.2	Measurement and evaluation of serving cell	77

5.1.2.3	Measurements of intra-frequency NR cells	77

5.1.2.4	Measurements of inter-frequency NR cells	77

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	77

5.1.2.6	Maximum interruption in paging reception	77

5.1.2.7	General requirements	77

5.1A	Cell Re-selection with CCA	77

5.1A.1	Introduction	77

5.1A.2	Requirements	78

5.1A.2.1	UE measurement capability	78

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	78

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	78

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	78

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	78

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	78

5.1A.2.7	General requirements	78

5.2	Void	78

5.3	Minimization of Drive Tests (MDT)	78

5.3.1	Introduction	78

5.3.2	Measurement Requirements	79

5.3.3	Requirements for Relative Time Stamp Accuracy	79

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	79

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	79

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	79

5.4	Idle Mode CA/DC Measurements	79

5.4.1	Introduction	79

5.4.2	Measurement Requirements	79

5.4.2.1	Detected cell requirement during state transition and Idle mode	80

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	80

5.4.2.3	Measurements on serving cell	80

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	80

6	RRC_CONNECTED state mobility	80

6.1	Handover	80

6.1.1	NR Handover	80

6.1.1.1	Introduction	80

6.1.1.2	NR FR1 - NR FR1 Handover	80

6.1.1.2.1	Handover delay	80

6.1.1.2.2	Interruption time	80

6.1.1.3	NR FR2- NR FR1 Handover	81

6.1.1.3.1	Handover delay	81

6.1.1.3.2	Interruption time	81

6.1.1.4	NR FR2- NR FR2 Handover	82

6.1.1.4.1	Handover delay	82

6.1.1.4.2	Interruption time	82

6.1.1.5	NR FR1- NR FR2 Handover	83

6.1.1.5.1	Handover delay	83

6.1.1.5.2	Interruption time	83

6.1.2	NR Handover to other RATs	84

6.1.2.1	NR – E-UTRAN Handover	84

6.1.2.1.1	Introduction	84

6.1.2.1.2	Handover delay	84

6.1.2.1.3	Interruption time	84

6.1.2.2	NR – UTRAN Handover	84

6.1.2.2.1	Introduction	84

6.1.2.2.2	Handover delay	85

6.1.2.2.3	Interruption time	85

6.1.3	NR DAPS Handover	85

6.1.3.1	Introduction	85

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	86

6.1.3.2.1	DAPS handover delay	86

6.1.3.2.2	Interruption time	87

6.1.3.3	NR FR2- NR FR1 DAPS Handover	88

6.1.3.3.1	DAPS handover delay	88

6.1.3.3.2	Interruption time	89

6.1.3.4	NR FR1- NR FR2 DAPS Handover	89

6.1.3.4.1	DAPS handover delay	90

6.1.3.4.2	Interruption time	90

6.1.4	NR Conditional Handover	90

6.1.4.1	Introduction	90

6.1.4.2	NR FR1 – NR FR1 conditional handover	91

6.1.4.3	NR FR2 – NR FR1 conditional handover	92

6.1.4.4	NR FR2 – NR FR2 conditional handover	92

6.1.4.4.1	Handover delay	92

6.1.4.4.2	Measurement time	92

6.1.4.4.3	Preparation time	93

6.1.4.4.4	Interruption time	93

6.1.4.5	NR FR1 – NR FR2 conditional handover	93

6.1A	Void	94

6.1A.1	Void	94

6.1A.1.1	Void	94

6.1A.1.2	Void	94

6.1A.1.2.1	Void	94

6.1A.1.2.2	Void	94

6.1B	Handover to target cell using CCA	94

6.1B.1	NR Handover	94

6.1B.1.1	Introduction	94

6.1B.1.2	NR FR1 - NR FR1 Handover	94

6.1B.1.2.1	Handover delay	94

6.1B.1.2.2	Interruption time	94

6.2	RRC Connection Mobility Control	95

6.2.1	SA: RRC Re-establishment	95

6.2.1.1	Introduction	95

6.2.1.2	Requirements	95

6.2.1.2.1	UE Re-establishment delay requirement	96

6.2.1A	RRC Re-establishment with CCA	97

6.2.1A.1	Introduction	97

6.2.1A.2	Requirements	97

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	98

6.2.2	Random access	99

6.2.2.1	Introduction	99

6.2.2.2	Requirements for 4-step RA type	99

6.2.2.2.1	Contention based random access	100

6.2.2.2.2	Non-Contention based random access	100

6.2.2.2.3	UE behaviour when configured with supplementary UL	101

6.2.2.3	Requirements for 2-step RA type	101

6.2.2.3.1	Contention based random access	102

6.2.2.3.2	Non-Contention based random access	103

6.2.2.3.3	UE behaviour when configured with supplementary UL	103

6.2.2A	Random access when CCA is used on target frequency	103

6.2.2A.1	Introduction	103

6.2.2A.2	Requirements for 4-step RA type	104

6.2.2A.2.1	Contention based random access	104

6.2.2A.2.2	Non-Contention based random access	105

6.2.2A.3	Requirements for 2-step RA type	106

6.2.2A.3.1	Contention based random access	106

6.2.2A.3.2	Non-Contention based random access	107

6.2.3	SA: RRC Connection Release with Redirection	108

6.2.3.1	Introduction	108

6.2.3.2	Requirements	108

6.2.3.2.1	RRC connection release with redirection to NR	108

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	109

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	110

7	Timing	111

7.1	UE transmit timing	111

7.1.1	Introduction	111

7.1.2	Requirements	111

7.1.2.1	Gradual timing adjustment	112

7.1.2.2	Void	113

7.2	UE timer accuracy	113

7.2.1	Introduction	113

7.2.2	Requirements	113

7.3	Timing advance	114

7.3.1	Introduction	114

7.3.2	Requirements	114

7.3.2.1	Timing Advance adjustment delay	114

7.3.2.2	Timing Advance adjustment accuracy	114

7.4	Cell phase synchronization accuracy	114

7.4.1	Definition	114

7.4.2	Minimum requirements	114

7.5	Maximum Transmission Timing Difference	114

7.5.1	Introduction	114

7.5.2	Minimum Requirements for inter-band EN-DC	115

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	115

7.5.3	Minimum Requirements for intra-band EN-DC	115

7.5.4	Minimum Requirements for NR Carrier Aggregation	116

7.5.5	Minimum Requirements for inter-band NE-DC	116

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	117

7.5.6	Minimum Requirements for inter-band NR DC	117

7.6	Maximum Receive Timing Difference	117

7.6.1	Introduction	117

7.6.2	Minimum Requirements for inter-band EN-DC	118

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	118

7.6.3	Minimum Requirements for intra-band EN-DC	118

7.6.4	Minimum Requirements for NR Carrier Aggregation	119

7.6.5	Minimum Requirements for inter-band NE-DC	119

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	120

7.6.6	Minimum Requirements for inter-band NR DC	120

7.7	deriveSSB-IndexFromCell tolerance	121

7.7.1	Minimum requirements	121

7.8	Void	121

8	Signalling characteristics	121

8.1	Radio Link Monitoring	121

8.1.1	Introduction	121

8.1.2	Requirements for SSB based radio link monitoring	122

8.1.2.1	Introduction	122

8.1.2.2	Minimum requirement	123

8.1.2.3	Measurement restrictions for SSB based RLM	125

8.1.3	Requirements for CSI-RS based radio link monitoring	125

8.1.3.1	Introduction	125

8.1.3.2	Minimum requirement	126

8.1.3.3	Measurement restrictions for CSI-RS based RLM	128

8.1.4	Minimum requirement at transitions	129

8.1.5	Minimum requirement for UE turning off the transmitter	129

8.1.6	Minimum requirement for L1 indication	129

8.1.7	Scheduling availability of UE during radio link monitoring	130

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	130

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	130

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	130

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	131

8.1A	Radio Link Monitoring with CCA on Target Frequency	131

8.1A.1	Introduction	131

8.1A.2	Requirements for SSB Based Radio Link Monitoring	132

8.1A.2.1	Introduction	132

8.1A.2.2	Minimum Requirement	133

8.1A.3	Minimum requirement at transitions	134

8.1A.4	Minimum requirement for UE turning off the transmitter	135

8.1A.5	Minimum requirement for L1 indication	135

8.1A.6	Scheduling availability of UE during radio link monitoring	135

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	135

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	135

8.2	Interruption	136

8.2.1	EN-DC Interruption	136

8.2.1.1	Introduction	136

8.2.1.2	Requirements	136

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	136

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	137

8.2.1.2.3	Interruptions at SCell addition/release	137

8.2.1.2.4	Interruptions at SCell activation/deactivation	138

8.2.1.2.5	Interruptions during measurements on SCC	139

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	140

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	141

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	142

8.2.1.2.9	Interruptions at SCell hibernation	142

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	143

8.2.1.2.11	Interruptions due to UE-specific CBW change	143

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	143

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	144

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	145

8.2.1.2.15	Interruptions due to SCell dormancy	146

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	146

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	147

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	147

8.2.2.1	Introduction	147

8.2.2.2	Requirements	148

8.2.2.2.1	Interruptions at SCell addition/release	148

8.2.2.2.2	Interruptions at SCell activation/deactivation	149

8.2.2.2.3	Interruptions during measurements on deactivated SCC	150

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	150

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	151

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	152

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	153

8.2.2.2.8	Interruptions due to UE-specific CBW change	153

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	153

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	155

8.2.2.2.11	Interruptions at direct SCell activation	155

8.2.2.2.12	Interruptions due to SCell dormancy	156

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	156

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	156

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	157

8.2.3	NE-DC Interruptions	157

8.2.3.1	Introduction	157

8.2.3.2	Requirements	158

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	158

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	158

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	158

8.2.3.2.4	Interruptions at SCell activation/deactivation	159

8.2.3.2.5	Interruptions during measurements on SCC	161

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	162

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	162

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	162

8.2.3.2.9	Interruptions at SCell hibernation	163

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	163

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	163

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	165

8.2.3.2.13	Interruptions due to SCell dormancy	165

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	166

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	166

8.2.4	NR-DC: Interruptions	167

8.2.4.1	Introduction	167

8.2.4.2	Requirements	168

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	168

8.2.4.2.2	Interruptions at SCell activation/deactivation	168

8.2.4.2.3	Interruptions during measurements on SCC	169

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	170

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	170

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	170

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	171

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	171

8.2.4.2.9	Interruptions at NR SRS carrier based switching	171

8.2.4.2.10	Interruptions at direct SCell activation	172

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	173

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	173

8.2.4.2.13	 Interruptions due to SCell dormancy	174

8.2.4.2A	Void	175

8.2.4.2A.1	Void	175

8.2.4.2A.2	Void	175

8.2.4.2A.3	Void	175

8.3	SCell Activation and Deactivation Delay	175

8.3.1	Introduction	175

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	175

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	179

8.3.4	Direct SCell Activation at SCell addition	179

8.3.5	Direct SCell Activation at Handover	180

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	181

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	184

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	185

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	186

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	187

8.3A.1	Introduction	187

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	187

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	190

8.4	UE UL carrier RRC reconfiguration delay	190

8.4.1	Introduction	190

8.4.2	UE UL carrier configuration delay requirement	190

8.4.3	UE UL carrier deconfiguration delay requirement	190

8.5	Link Recovery Procedures	191

8.5.1	Introduction	191

8.5.2	Requirements for SSB based beam failure detection	191

8.5.2.1	Introduction	191

8.5.2.2	Minimum requirement	192

8.5.2.3	Measurement restriction for SSB based beam failure detection	193

8.5.3	Requirements for CSI-RS based beam failure detection	194

8.5.3.1	Introduction	194

8.5.3.2	Minimum requirement	195

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	197

8.5.4	Minimum requirement for L1 indication	198

8.5.5	Requirements for SSB based candidate beam detection	198

8.5.5.1	Introduction	198

8.5.5.2	Minimum requirement	198

8.5.5.3	Measurement restriction for SSB based candidate beam detection	200

8.5.6	Requirements for CSI-RS based candidate beam detection	201

8.5.6.1	Introduction	201

8.5.6.2	Minimum requirement	201

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	203

8.5.7	Scheduling availability of UE during beam failure detection	204

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	204

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	204

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	204

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	205

8.5.8	Scheduling availability of UE during candidate beam detection	205

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	205

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	205

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	205

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	206

8.5.9	Requirements for Link Recovery with Link Recovery Request (LRR)	206

8.5.9.1	Introduction	206

8.5.9.2	Requirement	206

8.5A	Link Recovery Procedures when CCA is used on target frequency	206

8.5A.1	Introduction	206

8.5A.2	Requirements for SSB based beam failure detection	207

8.5A.2.1	Introduction	207

8.5A.2.2	Minimum requirement	207

8.5A.2.3	Measurement restriction for SSB based beam failure detection	208

8.5A.4	Minimum requirement for L1 indication	208

8.5A.5	Requirements for SSB based candidate beam detection	209

8.5A.5.1	Introduction	209

8.5A.5.2	Minimum requirement	209

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	209

8.5A.7	Scheduling availability of UE during beam failure detection	210

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	210

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	210

8.5A.8	Scheduling availability of UE during candidate beam detection	210

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	210

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	210

8.6	Active BWP switch delay	210

8.6.1	Introduction	210

8.6.2	DCI and timer based BWP switch delay on a single CC	210

8.6.2A	DCI based BWP switch delay on multiple CCs	212

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	212

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	213

8.6.2B	Timer based BWP switch delay on multiple CCs	214

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	214

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	214

8.6.3	RRC based BWP switch delay on a single CC	215

8.6.3A	RRC based BWP switch delay on multiple CCs	215

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	215

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	216

8.6.4	BWP switch delay on Consistent UL LBT recovery	216

8.7	Void	217

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	217

8.8.1	Introduction	217

8.8.2	E-UTRAN PSCell Addition Delay Requirement	217

8.8.3	E-UTRAN PSCell Release Delay Requirement	217

8.9	NR-DC: PSCell Addition and Release Delay	218

8.9.1	Introduction	218

8.9.2	PSCell Addition Delay Requirement	218

8.9.3	PSCell Release Delay Requirement	218

8.10	Active TCI state switching delay	219

8.10.4	DCI based TCI state switch delay	220

8.10.5	RRC based TCI state switch delay	220

8.10.6	Active TCI state list update delay	221

8.10A	Active TCI state switching delay with CCA	221

8.10A.1	Introduction	221

8.10A.2	Known conditions for TCI state	221

8.10A.3	MAC-CE based TCI state switch delay	221

8.10A.4	DCI based TCI state switch delay	222

8.10A.5	RRC based TCI state switch delay	222

8.10A.6	Active TCI state list update delay	223

8.11	PSCell Change	223

8.11A	void	223

8.11B	Conditional PSCell Change	223

8.11B.1	Introduction	223

8.11B.2	Conditoinal PSCell Change delay	223

8.11B.2.1	Measurement time	224

8.12	Uplink spatial relation switch delay	224

8.12.1	Introduction	224

8.12.2	Known conditions for spatial relation when associated with DL-RS	225

8.12.3	MAC-CE based spatial relation switch delay	225

8.12.4	DCI based spatial relation switch delay	226

8.12.5	RRC based spatial relation switch delay	226

8.13	UE-specific CBW change	226

8.13.1	Introduction	226

8.13.2	UE-specific CBW change delay	226

8.14	Pathloss reference signal switching delay	226

8.14.1	Introduction	226

8.14.2	Known conditions for pathloss reference signal	227

8.14.3	MAC-CE based pathloss reference signal switch delay	227

9	Measurement Procedure	227

9.1	General measurement requirement	227

9.1.1	Introduction	227

9.1.2	Measurement gap	228

9.1.2.1	EN-DC: Measurement Gap Sharing	238

9.1.2.1a	SA: Measurement Gap Sharing	239

9.1.2.1b	NE-DC: Measurement Gap Sharing	240

9.1.2.1c	NR-DC: Measurement Gap Sharing	241

9.1.3	UE Measurement capability	241

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	241

9.1.3.1a	SA: Monitoring of multiple layers using gaps	242

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	242

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	243

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	243

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	244

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	245

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	246

9.1A.3.2	Void	247

9.1.3A	UE Measurement capability under operation mode with CCA	247

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	247

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	247

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	247

9.1A.3.2a	Void9.1.3A.2A SA: Maximum allowed layers for multiple monitoring under CCA	247

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	248

9.1.4.1	Introduction	248

9.1.4.2	Requirements	248

9.1.5	Carrier-specific scaling factor	252

9.1.5.1	Monitoring of multiple layers outside gaps	252

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	254

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	254

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	255

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	256

9.1.5.2	Monitoring of multiple layers within gaps	257

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB and CSI-RS-based L3 measurements performed within gaps	258

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB and CSI-RS-based L3 measurements performed within gaps	259

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	261

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	262

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	264

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	264

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	264

9.1.6	Minimum requirement at transitions	264

9.2	NR intra-frequency measurements	264

9.2.1	Introduction	264

9.2.2	Requirements applicability	265

9.2.3	Number of cells and number of SSB	265

9.2.3.1	Requirements for FR1	265

9.2.3.2	Requirements for FR2	265

9.2.4	Measurement Reporting Requirements	266

9.2.4.1	Periodic Reporting	266

9.2.4.2	Event-triggered Periodic Reporting	266

9.2.4.3	Event Triggered Reporting	266

9.2.5	Intrafrequency measurements without measurement gaps	267

9.2.5.1	Intrafrequency cell identification	267

9.2.5.2	Measurement period	269

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	270

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	271

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	271

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	271

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	272

9.2.5.4	SFTD Measurements between PCell and PSCell	272

9.2.5.4.1	Introduction	272

9.2.5.4.2	SFTD Measurement delay	272

9.2.5.4.3	SFTD Measurement Reporting Delay	273

9.2.6	Intra-frequency measurements with measurement gaps	273

9.2.6.1	Void	273

9.2.6.2	Intra-frequency cell identification	273

9.2.6.3	Intrafrequency Measurement Period	275

9.2A	NR intra-frequency measurements with CCA	276

9.2A.1	Introduction	276

9.2A.2	Requirements applicability	277

9.2A.3	Number of cells and number of SSB	277

9.2A.4	Measurement Reporting Requirements	277

9.2A.5	Intra-frequency measurements without measurement gaps	278

9.2A.5.2	Measurement period	280

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	281

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	281

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	282

9.2A.6	Intra-frequency measurements with measurement gaps	282

9.2A.6.1	Intra-frequency cell identification	282

9.2A.6.2	Intra-frequency Measurement Period	283

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	284

9.2A.7.1	Intra-frequency RSSI measurements	284

9.2A.7.2	Intra-frequency Channel occupancy measurements	285

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	286

9.3	NR inter-frequency measurements	286

9.3.1	Introduction	286

9.3.2	Requirements applicability	287

9.3.2.1	Void	287

9.3.2.2	Void	287

9.3.3	Number of cells and number of SSB	287

9.3.3.1	Requirements for FR1	287

9.3.3.2	Requirements for FR2	287

9.3.4	Inter-frequency measurement with measurement gaps	287

9.3.4.1	Void	289

9.3.4.2	Void	289

9.3.5	Inter-frequency measurements	289

9.3.5.1	Void	289

9.3.5.2	Void	289

9.3.5.3	Void	289

9.3.6	Inter-frequency measurements reporting requirements	289

9.3.6.1	Periodic Reporting	289

9.3.6.2	Event-triggered Periodic Reporting	290

9.3.6.3	Event-triggered Reporting	290

9.3.7	Void	290

9.3.8	Inter-frequency SFTD measurement requirements	290

9.3.8.1	Introduction	290

9.3.8.2	SFTD Measurement delay	290

9.3.8.3	SFTD Measurement reporting delay	291

9.3.9	Inter frequency measurements without measurement gaps	292

9.3.9.1	Inter frequency Cell identification	292

9.3.9.2	Measurement period	293

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	294

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	294

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	295

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	295

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	295

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	296

9.3A.1	Introduction	296

9.3A.2	Requirements applicability	296

9.3A.3	Number of cells and number of SSB	296

9.3A.3.1	Requirements	296

9.3A.4	Inter-frequency cell identification	297

9.3A.5	Inter-frequency measurements	298

9.3A.6	NR Inter-frequency measurements reporting requirements	299

9.3A.6.1	Periodic Reporting	299

9.3A.6.2	Event-triggered Periodic Reporting	299

9.3A.6.3	Event-triggered Reporting	299

9.3A.8	Inter-frequency RSSI measurements	299

9.3A.9	Inter-frequency channel occupancy measurements	300

9.4	Inter-RAT measurements	300

9.4.1	Introduction	300

9.4.2	NR − E-UTRAN FDD measurements	302

9.4.2.1	Introduction	302

9.4.2.2	Requirements when no DRX is used	302

9.4.2.3	Requirements when DRX is used	302

9.4.2.4	Measurement reporting requirements	303

9.4.2.4.1	Periodic Reporting	303

9.4.2.4.2	Event-Triggered Periodic Reporting	304

9.4.2.4.3	Event-Triggered Reporting	304

9.4.3	NR − E-UTRAN TDD measurements	304

9.4.3.1	Introduction	304

9.4.3.2	Requirements when no DRX is used	304

9.4.3.3	Requirements when DRX is used	305

9.4.3.4	Measurement reporting requirements	306

9.4.3.4.1	Periodic Reporting	306

9.4.3.4.2	Event-Triggered Periodic Reporting	306

9.4.3.4.3	Event-Triggered Reporting	307

9.4.4	Inter-RAT RSTD measurements	307

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	307

9.4.4.1.1	Introduction	307

9.4.4.1.2	Requirements	308

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	311

9.4.4.2.1	Introduction	311

9.4.4.2.2	Requirements	311

9.4.5	Inter-RAT E-CID measurements	315

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	315

9.4.5.1.1	Introduction	315

9.4.5.1.2	Requirements	315

9.4.5.1.3	Measurement Reporting Delay	315

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	315

9.4.5.2.1	Introduction	315

9.4.5.2.2	Requirements	315

9.4.5.2.3	Measurement Reporting Delay	315

9.4.6	NR − UTRAN FDD measurements	315

9.4.6.1	Introduction	315

9.4.6.2	Requirements when no DRX is used	316

9.4.6.3	Requirements when DRX is used	317

9.4.7	NR – E-UTRAN measurements with autonomous gaps	319

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	319

9.4.7.2	CGI reporting delay	319

9.5	L1-RSRP measurements for Reporting	319

9.5.1	Introduction	319

9.5.2	Requirements applicability	319

9.5.3	Measurement Reporting Requirements	320

9.5.3.1	Periodic Reporting	320

9.5.3.2	Semi-Persistent Reporting	320

9.5.3.3	Aperiodic Reporting	321

9.5.4	L1-RSRP measurement requirements	321

9.5.4.1	SSB based L1-RSRP Reporting	321

9.5.4.2	CSI-RS based L1-RSRP Reporting	322

9.5.4A	Void	325

9.5.4A.1	Void	325

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	325

9.5.5.1	Measurement restriction for SSB based L1-RSRP	325

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	326

9.5.6	Scheduling availability of UE during L1-RSRP measurement	326

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	326

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	327

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	327

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	328

9.5A	L1-RSRP measurements for Reporting under CCA	328

9.5A.1	Introduction	328

9.5A.2	Requirements applicability	328

9.5A.3	Measurement Reporting Requirements	329

9.5A.3.1	Periodic Reporting	329

9.5A.3.2	Semi-Persistent Reporting	329

9.5A.3.3	Aperiodic Reporting	329

9.5A.4	L1-RSRP measurement requirements	330

9.5A.4.1	SSB based L1-RSRP Reporting	330

9.5A.5	Measurement restriction for L1-RSRP measurement	330

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	330

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	331

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	331

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	331

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	331

9.6	NE-DC: Measurements	331

9.6.1	Introduction	331

9.6.2	SFTD Measurements	331

9.6.2.1	Introduction	331

9.6.2.2	SFTD Measurement requirements	332

9.7	Cross Link Interference measurements	332

9.7.1	Introduction	332

9.7.2	SRS-RSRP measurements	333

9.7.2.1	Introduction	333

9.7.2.2	Requirements applicability	333

9.7.2.3	Measurement Reporting Requirements	333

9.7.2.3.1	Periodic Reporting	333

9.7.2.3.2	Event-triggered Periodic Reporting	333

9.7.2.3.3	Event Triggered Reporting	333

9.7.2.4	Measurement capability	334

9.7.2.5	SRS-RSRP measurement period	334

9.7.3	CLI-RSSI measurements	334

9.7.3.1	Introduction	334

9.7.3.2	Requirements applicability	334

9.7.3.3	Measurement Reporting Requirements	334

9.7.3.3.1	Periodic Reporting	334

9.7.3.3.2	Event-triggered Periodic Reporting	335

9.7.3.3.3	Event Triggered Reporting	335

9.7.3.4	Measurement capability	335

9.7.3.5	CLI-RSSI measurement period	335

9.7.4	Scheduling availability of UE during CLI measurements	335

9.7.4.1	Scheduling availability of UE performing measurement on FR1	335

9.7.4.2	Scheduling availability of UE performing measurement on FR2	336

9.8	L1-SINR measurements for Reporting	336

9.8.1	Introduction	336

9.8.2	Requirements applicability	337

9.8.3	Measurement Reporting Requirements	337

9.8.3.1	Periodic Reporting	338

9.8.3.2	Semi-Persistent Reporting	338

9.8.4	L1-SINR measurement requirements	338

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	338

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	341

9.8.5	Measurement restriction for L1-SINR measurement	343

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	343

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	344

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	345

9.8.6	Scheduling availability of UE during L1-SINR measurement	345

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	345

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	345

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	346

9.9	 NR measurements for positioning	346

9.9.1	Introduction	346

9.9.2	RSTD measurements	347

9.9.2.1	Introduction	347

9.9.2.2	Requirements Applicability	347

9.9.2.3	Measurement Capability	347

9.9.2.4	Measurement Reporting Requirements	347

9.9.2.4.1	Void	347

9.9.2.4.2	Void	347

9.9.2.4.3	Void	347

9.9.2.5	Measurements Period Requirements	347

9.9.2.6	Void	349

9.9.3	PRS-RSRP measurements	349

9.9.3.1	Introduction	349

9.9.3.2	Requirements applicability	349

9.9.3.3	Measurement Capability	349

9.9.3.4	Measurement Reporting Requirements	349

9.9.3.5	Measurement Period Requirements	349

9.9.4	UE Rx-Tx time difference measurements	351

9.9.4.1 Introduction	351

9.9.4.2 Requirements Applicability	351

9.9.4.3 Measurement Capability	351

9.9.4.4 Measurement Reporting Requirements	351

9.9.4.5 Measurement Period Requirements	351

9.9.5	NR E-CID measurements	353

9.9.5.1	Introduction	353

9.9.5.2	Measurement Requirements	353

9.9.5.2.1	Intra-frequency Measurement Requirements	353

9.9.5.2.2	Inter-frequency Measurement Requirements	353

9.9.5.2.3	Measurement Reporting Delay	354

9.10	CSI-RS based L3 measurements	354

9.10.1	Introduction	354

9.10.2	CSI-RS based intra-frequency measurements	354

9.10.2.1	Introduction	354

9.10.2.2	Requirements applicability	355

9.10.2.3	Number of cells and number of CSI-RS	355

9.10.2.3.1	Requirements for FR1	355

9.10.2.3.2	Requirements for FR2	356

9.10.2.4	Measurement Reporting Requirements	356

9.10.2.4.1	Periodic Reporting	356

9.10.2.4.2	Event-triggered Periodic Reporting	356

9.10.2.4.3	Event Triggered Reporting	356

9.10.2.5	Intra-frequency measurements without measurement gaps	357

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	358

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	358

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	358

9.10.3	CSI-RS based Inter-frequency measurements	358

9.10.3.1	Introduction	358

9.10.3.2	Requirements applicability	358

9.10.3.3	Number of cells and number of CSI-RS resources	359

9.10.3.3.1	Requirements for FR1	359

9.10.3.3.2	Requirements for FR2	359

9.10.3.4	Measurements reporting requirements	359

9.10.3.4.1	Periodic Reporting	359

9.10.3.4.2	Event-triggered Periodic Reporting	359

9.10.3.4.3	Event-triggered Reporting	360

9.10.3.5	Inter frequency measurements with measurement gaps	360

9.11	NR measurements with autonomous gaps	361

9.11.1	Introduction	361

9.11.2	CGI identification of an NR cell with autonomous gaps	361

9.11.3	CGI reporting delay	362

10	Measurement Performance requirements	362

10.1	NR measurements	362

10.1.1	Introduction	362

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	363

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	363

10.1.2.1.1	Absolute SS-RSRP Accuracy	363

10.1.2.1.2	Relative SS-RSRP Accuracy	364

10.1.2.2	Void	365

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	365

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	365

10.1.3.1.1	Absolute SS-RSRP Accuracy	365

10.1.3.1.2	Relative SS-RSRP Accuracy	366

10.1.3.2	Void	367

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	367

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	367

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	367

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	367

10.1.4.2	Void	368

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	368

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	368

10.1.5.1.1	Absolute SS-RSRP Accuracy	368

10.1.5.1.2	Relative SS-RSRP Accuracy	369

10.1.5.2	Void	370

10.1.6 RSRP Measurement Report Mapping	370

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	372

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	372

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	372

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	373

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	373

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	373

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	374

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	374

10.1.9.1.1	Aboslute Accuracy of SS-RSRQ in FR1	374

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	375

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	376

10.1.11	RSRQ report mapping	378

10.1.12	Intra-frequency SINR accuracy requirements for FR1	378

10.1.13	Intra-frequency SINR accuracy requirements for FR2	379

10.1.14	Inter-frequency SINR accuracy requirements for FR1	380

10.1.15	Inter-frequency SINR accuracy requirements for FR2	382

10.1.16	SINR report mapping	384

10.1.17	Power Headroom	385

10.1.18	PCMAX,c,f	386

10.1.19	L1-RSRP accuracy requirements for FR1	386

10.1.20	L1-RSRP accuracy requirements for FR2	390

10.1.21	SFTD accuracy requirements	393

10.1.22	CLI measurement accuracy requirements	397

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	401

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	401

10.1.23	RSTD Measurements	403

10.1.23.1	Introduction	403

10.1.23.2	Measurement Accuracy Requirements	404

10.1.23.3	Report mapping	404

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	404

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	406

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	408

10.1.24	PRS-RSRP Measurements	410

10.1.24.1	Introduction	410

10.1.24.2	Measurement Accuracy Requirements	410

10.1.24.3	Report mapping	410

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	410

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	411

10.1.25	UE Rx-Tx Time Difference Measurements	413

10.1.25.1	Introduction	413

10.1.25.2	Measurement Accuracy Requirements	414

10.1.25.3	Report mapping	414

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	414

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	416

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	417

10.1.26	FR2 P-MPR report	419

10.1.26.1	Report mapping	419

10.1.27	L1-SINR accuracy requirements for FR1	419

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	419

10.1.27.1.1	Absolute Accuracy	419

10.1.27.1.2	Relative Accuracy	420

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	421

10.1.27.2.1	Absolute Accuracy	421

10.1.27.2.2	Relative Accuracy	422

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	424

10.1.27.3.1	Absolute Accuracy	424

10.1.27.3.2	Relative Accuracy	426

10.1.28	L1-SINR accuracy requirements for FR2	427

10.1.28.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	427

10.1.28.1.1	Absolute Accuracy	427

10.1.28.1.2	Relative Accuracy	428

10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	429

10.1.28.2.1	Absolute Accuracy	429

10.1.28.2.2	Relative Accuracy	430

10.1.28.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	431

10.1.28.3.1	Absolute Accuracy	431

10.1.28.3.2	Relative Accuracy	432

10.2	E-UTRAN measurements	434

10.2.1	Introduction	434

10.2.2	E-UTRAN RSRP measurements	434

10.2.3	E-UTRAN RSRQ measurements	434

10.2.4	E-UTRAN RSTD measurements	434

10.2.5	E-UTRAN RS-SINR measurements	435

10.3	UTRAN FDD Measurements	435

10.3.1	UTRAN FDD CPICH RSCP	435

10.3.2	UTRAN FDD CPICH Ec/No	436

10.4	V2X measurements	436

10.4.1	Introduction	436

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	436

10.4.2.1	PSBCH-RSRP Absolute Accuracy	436

10.4.2.2	PSBCH-RSRP Relative Accuracy	437

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	438

10.4.3.1	Absolute SL-RSSI Accuracy	438

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	438

10.4.4.1	Absolute L1 SL-RSRP Accuracy	438

11	Void	439

12	V2X Requirements	439

12.1	Introduction	439

12.2	UE Transmit Timing	439

12.2.1	Introduction	439

12.2.2	GNSS as synchronization reference source	439

12.2.3	NR Cell as synchronization reference source	440

12.2.4	E-URTAN Cell as synchronization reference source	440

12.2.5	SyncRef UE as synchronization reference source	441

12.3	Initiation/Cease of SLSS Transmissions	441

12.3.1	Introduction	441

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	441

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	442

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	443

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	443

12.4	Selection / Reselection of V2X Synchronization Reference Source	443

12.5	L1 SL-RSRP measurements	444

12.5.1	Introduction	444

12.5.2	SL-RSRP measurements	444

12.6	Congestion Control measurements	445

12.7	Interruption	445

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	445

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	445

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	447

12.8	Reliability of GNSS signal	447

12.9	Scheduling availability	448

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	448

13	Measurement Performance Requirements for NR gNB	448

13.1	UL-RTOA	448

13.1.1	Report mapping	448

13.2	gNB Rx-Tx time difference	450

13.2.1	Report mapping	450

13.3	UL SRS RSRP measurement	452

13.3.1	Report mapping	452

13.4	AoA/ZoA	453

13.4.1	Report mapping	453

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1.2	CORESET for RMSI scheduling	429

A.3.1.2.1	FDD	429

A.3.1.2.2	TDD	430

A.3.1.3	CORESET for RMC scheduling	433

A.3.1.3.1	FDD	433

A.3.1.3.2	TDD	434

A.3.1.4	TDD UL/DL configuration	437

A.3.2	OFDMA channel noise generator (OCNG)	438

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	438

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	438

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	439

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	439

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	440

A.3.2.2	Void	441

A.3.3	Reference DRX configurations	441

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	441

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	441

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	441

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	442

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	442

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	442

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	443

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	443

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	443

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms	444

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	444

A.3.4	Test Cases with Different Channel Bandwidths	444

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	444

A.3.4.1.1	Introduction	444

A.3.4.1.2	Principle of testing	444

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	445

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	445

A.3.5.1.1	Introduction	445

A.3.5.1.2	Principle of Testing	445

A.3.6	Antenna configurations	445

A.3.6.1	Antenna configurations for FR1	445

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	445

A.3.6.1.1.1	Introduction	445

A.3.6.1.1.2	Principle of testing	445

A.3.6.2	Antenna configurations for FR2	448

A.3.7	EN-DC test setup	448

A.3.7.1	Introduction	448

A.3.7.2	E-UTRAN Serving Cell Parameters	448

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	448

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	449

A.3.7A	NR FR1-FR2 test setup	450

A.3.8	PRACH configurations	451

A.3.8.1	Introduction	451

A.3.8.2	PRACH configurations in FR1	451

A.3.8.2.1	FR1 PRACH configuration 1	451

A.3.8.2.2	FR1 PRACH configuration 2	451

A.3.8.2.3	FR1 PRACH configuration 3	452

A.3.8.2.4	FR1 PRACH configuration 4	453

A.3.8.3	PRACH configurations in FR2	454

A.3.8.3.1	FR2 PRACH configuration 1	454

A.3.8.3.2	FR2 PRACH configuration 2	455

A.3.8.3.3	FR2 PRACH configuration 3	456

A.3.8.3.4	FR2 PRACH configuration 4	457

A.3.9	BWP configurations	458

A.3.9.1	Introduction	458

A.3.9.2	Downlink BWP configurations	459

A.3.9.2.1	Initial BWP	459

A.3.9.2.2	Dedicated BWP	459

A.3.9.3	Uplink BWP configurations	459

A.3.9.3.1	Initial BWP	459

A.3.9.3.2	Dedicated BWP	460

A.3.10	SSB Configurations	460

A.3.10.1	SSB Configurations for FR1	460

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	460

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	462

A.3.11	SMTC Configurations	467

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	467

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	467

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	467

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	468

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	468

A.3.12	Test Cases with Different CC Configurations	468

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	468

A.3.12.1.1	Introduction	468

A.3.12.1.2	Principle of testing	468

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	468

A.3.12.2.1	Introduction	468

A.3.12.2.2	Principle of testing	468

A.3.13	Test Cases in SA and EN-DC Operations	469

A.3.13.1	Introduction	469

A.3.13.2	Principle of Testing	469

A.3.14	CSI-RS configurations	469

A.3.14.1	FDD	469

A.3.14.2	TDD	471

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	476

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	476

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	476

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	476

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	477

A.3.15.3	Setup 3: 2 AoAs	477

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	477

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	477

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	477

A.3.16	TCI State Configuration	478

A.3.16.1	Introduction	478

A.3.16.2	TCI states	478

A.3.17	Configurations of CSI-RS for tracking	478

A.3.17.1	Configuration of CSI-RS for tracking for FR1	478

A.3.17.1.1	FDD	478

A.3.17.1.2	TDD	479

A.3.17.2	Configuration of CSI-RS for tracking for FR2	480

A.3.17.2.1	TDD	480

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	481

A.3.18.1	Introduction	481

A.3.18.2	PRACH Power Measurement	481

A.3.19	Test applicability for DAPS handover	481

A.3.19.1	Introduction	481

A.3.19.2	Principle of testing	481

A.3.20	MsgA configurations	482

A.3.20.1	Introduction	482

A.3.20.2	MsgA configurations in FR1	482

A.3.20.2.1	FR1 MsgA configuration 1	482

A.3.20.2.2	FR1 MsgA configuration 2	483

A.3.20.3	MsgA configurations in FR2	485

A.3.20.3.1	FR2 MsgA configuration 1	485

A.3.20.3.2	FR2 MsgA configuration 2	486

A.3.21	V2X sidelink communication	488

A.3.21.1	Introduction	488

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	488

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	495

A.3.22	CSI-IM configurations	495

A.3.22.1	FDD	495

A.3.22.2	TDD	496

A.3.23	Spatial Relation Configuration	498

A.3.23.1	Introduction	498

A.3.23.2	Spatial Relation	498

A.3.24	SRS configuration	499

A.3.25	Channel bandwidth (CBW) configurations	501

A.3.25.1	DL UE specific CBW	501

A.3.25.2	UL UE specific CBW	501

A.4	EN-DC tests with all NR cells in FR1	502

A.4.1	Void	502

A.4.2	Void	502

A.4.3	RRC_CONNECTED state mobility	502

A.4.3.1	Void	502

A.4.3.2	RRC Connection Mobility Control	502

A.4.3.2.1	Void	502

A.4.3.2.2	Random Access	502

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	502

A.4.3.2.2.2	4-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	505

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	509

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	512

A.4.3.2.3	Void	516

A.4.4	Timing	516

A.4.4.1	UE transmit timing	516

A.4.4.1.1	NR UE Transmit Timing Test for FR1	516

A.4.4.1.1.1	Test Purpose and environment	516

A.4.4.1.1.2	Test requirements	520

A.4.4.2	UE timer accuracy	520

A.4.4.3	Timing advance	520

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	520

A.4.4.3.1.1	Test Purpose and Environment	520

A.4.4.3.1.2	Test Parameters	520

A.4.4.3.1.3	Test Requirements	524

A.4.5	Signaling characteristics	524

A.4.5.1	Radio link Monitoring	524

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	525

A.4.5.1.1.1	Test Purpose and Environment	525

A.4.5.1.1.2	Test Requirements	529

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	529

A.4.5.1.2.1	Test Purpose and Environment	529

A.4.5.1.2.2	Test Requirements	535

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	535

A.4.5.1.3.1	Test Purpose and Environment	535

A.4.5.1.3.2	Test Requirements	541

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	541

A.4.5.1.4.1	Test Purpose and Environment	541

A.4.5.1.4.2	Test Requirements	547

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	547

A.4.5.1.5.1	Test Purpose and Environment	547

A.4.5.1.5.2	Test Requirements	553

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	553

A.4.5.1.6.1	Test Purpose and Environment	553

A.4.5.1.6.2	Test Requirements	559

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	559

A.4.5.1.7.1	Test Purpose and Environment	559

A.4.5.1.7.2	Test Requirements	564

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	565

A.4.5.1.8.1	Test Purpose and Environment	565

A.4.5.1.8.2	Test Requirements	570

A.4.5.2	Interruption	570

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	570

A.4.5.2.1.1	Test Purpose and Environment	570

A.4.5.2.1.2	Test Requirements	574

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	574

A.4.5.2.2.1	Test Purpose and Environment	574

A.4.5.2.2.2	Test Requirements	578

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	578

A.4.5.2.3.1	Test Purpose and Environment	578

A.4.5.2.3.2	Test Requirements	582

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	582

A.4.5.2.4.1	Test Purpose and Environment	582

A.4.5.2.4.2	Test Requirements	586

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	586

A.4.5.2.5.1	Test Purpose and Environment	586

A.4.5.2.5.2	Test Requirements	590

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	590

A.4.5.2.6.1	Test Purpose and Environment	590

A.4.5.2.6.2	Test Requirements	594

A.4.5.2.7	Void	594

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	594

A.4.5.2.8.1	Test Purpose and Environment	594

A.4.5.2.8.2	Test Requirements	597

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	598

A.4.5.2.9.1	Test Purpose and Environment	598

A.4.5.2.9.2	Test Requirements	602

A.4.5.3	SCell Activation and Deactivation Delay	603

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	603

A.4.5.3.1.1	Test Purpose and Environment	603

A.4.5.3.1.2	Test Requirements	608

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	609

A.4.5.3.2.1	Test Purpose and Environment	609

A.4.5.3.2.2	Test Requirements	609

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	609

A.4.5.3.3.1	Test Purpose and Environment	609

A.4.5.3.3.2	Test Requirements	610

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	610

A.4.5.3.4.1	Test Purpose and Environment	610

A.4.5.3.4.2	Test Requirements	614

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	614

A.5.5.3.6.1	Test Purpose and Environment	614

A.5.5.3.6.2	Test Requirements	617

A.4.5.4	UE UL carrier RRC reconfiguration Delay	618

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	618

A.4.5.4.1.1 Test Purpose and Environment	618

A.4.5.4.1.2	Test Requirements	626

A.4.5.5	Beam Failure Detection and Link recovery procedures	626

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	626

A.4.5.5.1.1	Test Purpose and Environment	626

A.4.5.5.1.2	Test Requirements	632

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	632

A.4.5.5.2.1	Test Purpose and Environment	632

A.4.5.5.2.2	Test Requirements	638

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	638

A.4.5.5.3.1	Test Purpose and Environment	638

A.4.5.5.3.2	Test Requirements	644

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	644

A.4.5.5.4.1	Test Purpose and Environment	644

A.4.5.5.4.2	Test Requirements	650

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	650

A.4.5.5.5.1	Test Purpose and Environment	650

A.4.5.5.5.2	Test Requirements	656

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	656

A.4.5.5.6.1	Test Purpose and Environment	656

A.4.5.5.6.2	Test Requirements	663

A.4.5.6	Active BWP switch	664

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	664

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	664

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	669

A.4.5.6.2	RRC-based Active BWP Switch	673

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	678

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	678

A.4.5.6.3.1.1	Test Purpose and Environment	678

A.4.5.6.3.1.2	Test Requirements	683

A.4.5.7	PSCell addition and release delay	684

A.4.5.7.1	Addition and Release Delay of known NR PSCell	684

A.4.5.7.1.1	Test purpose and environment	684

A.4.5.7.1.2	Test Requirements	687

A.4.5.8	DL Interruptions at switching between two uplink carriers	687

A.4.5.8.1	Test Purpose and Environment	687

A.4.5.8.2	Test Requirements	691

A.4.5.9	UE specific CBW change	691

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	691

A.4.5.9.1.1	Test Purpose and Environment	691

A.4.5.9.1.2	Test Requirements	695

A.4.6	Measurement procedure	695

A.4.6.1	Intra-frequency Measurements	695

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	695

A.4.6.1.1.1	Test purpose and Environment	695

A.4.6.1.1.2	Test parameters	695

A.4.6.1.1.3	Test Requirements	699

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	699

A.4.6.1.2.1	Test purpose and Environment	699

A.4.6.1.2.2	Test parameters	699

A.4.6.1.2.2	Test Requirements	703

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	703

A.4.6.1.3.1	Test purpose and Environment	703

A.4.6.1.3.2	Test parameters	703

A.4.6.1.3.3	Test Requirements	707

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	707

A.4.6.1.4.1	Test purpose and Environment	707

A.4.6.1.4.2	Test parameters	707

A.4.6.1.4.3	Test Requirements	711

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	711

A.4.6.1.5.1	Test purpose and Environment	711

A.4.6.1.5.2	Test parameters	711

A.4.6.1.5.3	Test Requirements	713

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	714

A.4.6.1.6.1	Test purpose and Environment	714

A.4.6.1.6.2	Test parameters	714

A.4.6.1.6.3	Test Requirements	716

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	717

A.4.6.1.7.1	Test purpose and Environment	717

A.4.6.1.7.2	Test parameters	717

A.4.6.1.7.3	Test Requirements	721

A.4.6.2	Inter-frequency Measurements	721

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	721

A.4.6.2.1.1	Test Purpose and Environment	721

A.4.6.2.1.2	Test Requirements	725

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	725

A.4.6.2.2.1	Test Purpose and Environment	725

A.4.6.2.2.2	Test Requirements	729

A.4.6.2.3	Void	730

A.4.6.2.4	Void	730

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	730

A.4.6.2.5.1	Test Purpose and Environment	730

A.4.6.2.5.2	Test Requirements	734

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	734

A.4.6.2.6.1	Test Purpose and Environment	734

A.4.6.2.6.2	Test Requirements	739

A.4.6.2.7	Void	740

A.4.6.2.8	Void	740

A.4.6.3	Void	740

A.4.6.4	L1-RSRP measurement for beam reporting	740

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	740

A.4.6.4.1.1	Test Purpose and Environment	740

A.4.6.4.1.2	Test parameters	740

A.4.6.4.1.3	Test Requirements	743

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	743

A.4.6.4.2.1	Test Purpose and Environment	743

A.4.6.4.2.2	Test parameters	744

A.4.6.4.2.3	Test Requirements	747

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	747

A.4.6.4.3.1	Test Purpose and Environment	747

A.4.6.4.3.2	Test parameters	748

A.4.6.4.3.3	Test Requirements	751

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	751

A.4.6.4.4.1	Test Purpose and Environment	751

A.4.6.4.4.2	Test parameters	752

A.4.6.4.4.3	Test Requirements	754

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	754

A.4.6.4.5.1	Test Purpose and Environment	754

A.4.6.4.5.2	Test parameters	755

A.4.6.4.2.3	Test Requirements	758

A.4.6.5	CLI measurements	758

A.4.6.5.1	SRS-RSRP measurement with non-DRX	758

A.4.6.5.1.1	Test Purpose and Environment	758

A.4.6.5.1.2	Test Parameters	759

A.4.6.5.1.3	Test Requirements	762

A.4.6.5.2	CLI-RSSI measurement with non-DRX	762

A.4.6.5.2.1	Test Purpose and Environment	762

A.4.6.5.2.2	Test Parameters	763

A.4.6.5.2.3	Test Requirements	764

A.4.6.6	Measurements with autonomous gaps	765

A.4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	765

A.4.6.6.1.1	Test Purpose and Environment	765

A.4.6.6.1.2	Test Requirements	769

A.4.6.7	L1-SINR measurement for beam reporting	769

A.4.6.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR when DRX is not used	769

A.4.6.7.1.1	Test Purpose and Environment	769

A.4.6.7.1.2	Test parameters	770

A.4.6.7.1.3	Test Requirements	773

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	773

A.4.6.7.2.1	Test Purpose and Environment	773

A.4.6.7.2.2	Test parameters	774

A.4.6.7.2.3	Test Requirements	777

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	777

A.4.6.7.3.1	Test Purpose and Environment	777

A.4.6.7.3.2	Test parameters	778

A.4.6.7.3.3	Test Requirements	781

A.4.7	Measurement Performance requirements	781

A.4.7.1	SS-RSRP	782

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	782

A.4.7.1.1.1	Test Purpose and Environment	782

A.4.7.1.1.2	Test parameters	782

A.4.7.1.1.3	Test Requirements	786

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	786

A.4.7.1.2.1	Test Purpose and Environment	786

A.4.7.1.2.2	Test parameters	787

A.4.7.1.2.3	Test Requirements	791

A.4.7.1.3	Void	791

A.4.7.2	SS-RSRQ	791

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	791

A.4.7.2.1.1	Test Purpose and Environment	791

A.4.7.2.1.2	Test Parameters	792

A.4.7.2.1.3	Test Requirements	797

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	797

A.4.7.2.2.1	Test Purpose and Environment	797

A.4.7.2.2.2	Test Parameters	797

A.4.7.2.2.3	Test Requirements	802

A.4.7.3	SS-SINR	802

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	802

A.4.7.3.1.1	Test Purpose and Environment	802

A.4.7.3.1.2	Test Parameters	802

A.4.7.3.1.3	Test Requirements	806

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	806

A.4.7.3.2.1	Test Purpose and Environment	806

A.4.7.3.2.2	Test Parameters	806

A.4.7.3.2.3	Test Requirements	811

A.4.7.4	L1-RSRP measurement for beam reporting	811

A.4.7.4.1	SSB based L1-RSRP measurement	811

A.4.7.4.1.1	Test Purpose and Environment	811

A.4.7.4.1.2	Test parameters	812

A.4.7.4.1.3	Test Requirements	815

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	815

A.4.7.4.2.1	Test Purpose and Environment	815

A.4.7.4.2.2	Test parameters	816

A.4.7.4.2.3	Test Requirements	819

A.4.7.5	SFTD accuracy	819

A.4.7.5.1	SFTD accuracy	819

A.4.7.5.1.1	Test Purpose and Environment	819

A.4.7.5.1.2	Test Parameters	819

A.4.7.5.1.3	Test Requirements	824

A.4.7.5.2	Void	824

A.4.7.5.3	Void	824

A.4.7.6	CLI measurements	824

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	824

A.4.7.6.1.1	Test Purpose and Environment	824

A.4.7.6.1.2	Test parameters	825

A.4.7.6.1.3	Test Requirements	830

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	830

A.4.7.6.2.1	Test Purpose and Environment	830

A.4.7.6.2.2	Test parameters	831

A.4.7.6.2.3	Test Requirements	834

A.4.7.7	L1-SINR measurement for beam reporting	834

A.4.7.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	834

A.4.7.7.1.1	Test Purpose and Environment	834

A.4.7.7.1.2	Test parameters	835

A.4.7.7.1.3	Test Requirements	839

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	839

A.4.7.7.2.1	Test Purpose and Environment	839

A.4.7.7.2.2	Test parameters	840

A.4.7.7.2.3	Test Requirements	844

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	844

A.4.7.7.3.1	Test Purpose and Environment	844

A.4.7.7.3.2	Test parameters	845

A.4.7.7.3.3	Test Requirements	848

A.4.8	Void	848

A.5	EN-DC tests with one or more NR cells in FR2	849

A.5.1	Void	849

A.5.2	Void	849

A.5.3	RRC_CONNECTED state mobility	849

A.5.3.1	Void	849

A.5.3.2	RRC Connection Mobility Control	849

A.5.3.2.1	Void	849

A.5.3.2.2	Random Access	849

A.5.3.2.2.1	4-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	849

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	853

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	857

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	861

A.5.3.2.3	Void	865

A.5.4	Timing	865

A.5.4.1	UE transmit timing	865

A.5.4.1.1	NR UE Transmit Timing Test for FR2	865

A.5.4.1.1.1	Test Purpose and environment	865

A.5.4.1.1.2	Test requirements	868

A.5.4.2	UE timer accuracy	869

A.5.4.3	Timing advance	869

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	869

A.5.4.3.1.1 Test Purpose and Environment	869

A.5.4.3.1.2 Test Parameters	869

A.5.4.3.1.3	Test Requirements	872

A.5.5	Signaling characteristics	872

A.5.5.1	Radio link Monitoring	872

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	872

A.5.5.1.1.1	Test Purpose and Environment	872

A.5.5.1.1.2	Test Requirements	875

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	876

A.5.5.1.2.1	Test Purpose and Environment	876

A.5.5.1.2.2	Test Requirements	879

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	880

A.5.5.1.3.1	Test Purpose and Environment	880

A.5.5.1.3.2	Test Requirements	883

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	883

A.5.5.1.4.1	Test Purpose and Environment	883

A.5.5.1.4.2	Test Requirements	888

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	888

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	894

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	899

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	905

A.5.5.1.8.2	Test Requirements	911

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	912

A.5.5.1.9.1	Test Purpose and Environment	912

A.5.5.1.9.2	Test Requirements	913

A.5.5.2	Interruption	913

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	913

A.5.5.2.1.1	Test Purpose and Environment	913

A.5.5.2.1.2	Test Requirements	916

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	916

A.5.5.2.2.1	Test Purpose and Environment	916

A.5.5.2.2.2	Test Requirements	919

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	919

A.5.5.2.3.1	Test Purpose and Environment	919

A.5.5.2.3.2	Test Requirements	923

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	923

A.5.5.2.4.1	Test Purpose and Environment	923

A.5.5.2.4.2	Test Requirements	926

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	927

A.5.5.2.5.1	Test Purpose and Environment	927

A.5.5.2.5.2	Test Requirements	930

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	931

A.5.5.2.6.1	Test Purpose and Environment	931

A.5.5.2.6.2	Test Requirements	933

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	934

A.5.5.2.7.1	Test Purpose and Environment	934

A.5.5.2.7.2	Test Requirements	937

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	937

A.5.5.2.8.1 Test Purpose and Environment	937

A.5.5.2.8.3	Test Requirements	941

A.5.5.3	SCell Activation and Deactivation Delay	941

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	941

A.5.5.3.1.1	Test Purpose and Environment	941

A.5.5.3.1.2	Test Requirements	943

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	943

A.5.5.3.2.1	Test Purpose and Environment	943

A.5.5.3.2.2	Test Requirements	946

A.5.5.3.3	Void	946

A.5.5.3.4	Void	946

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	946

A.5.5.3.5.1	Test Purpose and Environment	946

A.5.5.3.5.2	Test Requirements	950

A.5.5.4	Void	951

A.5.5.5	Beam Failure Detection and Link recovery procedures	951

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	951

A.5.5.5.1.1	Test Purpose and Environment	951

A.5.5.5.1.2	Test Requirements	957

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	957

A.5.5.5.2.1	Test Purpose and Environment	957

A.5.5.5.2.2	Test Requirements	963

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	963

A.5.5.5.3.1	Test Purpose and Environment	963

A.5.5.5.3.2	Test Requirements	969

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	969

A.5.5.5.4.1	Test Purpose and Environment	969

A.5.5.5.4.2	Test Requirements	975

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	975

A.5.5.5.5.1	Test Purpose and Environment	975

A.5.5.5.5.2	Test Requirements	981

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	981

A.5.5.5.6.1	Test Purpose and Environment	981

A.5.5.5.6.2	Test Requirements	986

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	986

A.5.5.5.7.1	Test Purpose and Environment	986

A.5.5.5.7.2	Test Requirements	992

A.5.5.6	Active BWP switch	992

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	992

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	992

A.5.5.6.1.1.1	Test Purpose and Environment	992

A.5.5.6.1.1.2	Test Requirements	996

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC	997

A.5.5.6.2	RRC-based Active BWP Switch	1001

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1005

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1005

A.5.5.6.3.1.1	Test Purpose and Environment	1005

A.5.5.6.3.1.2	Test Requirements	1009

A.5.5.7	PSCell addition and release delay	1010

A.5.5.7.1	Addition and Release Delay of NR PSCell	1010

A.5.5.7.1.1	Test purpose and environment	1010

A.5.5.7.1.2	Test Requirements	1013

A.5.5.8	Active TCI state switch delay	1014

A.5.5.8.1	MAC-CE based active TCI state switch	1014

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1014

A.5.5.8.1.1.1	Test Purpose and Environment	1014

A.5.5.8.1.1.2	Test Requirements	1016

A.5.5.8.2	RRC based active TCI state switch	1016

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1016

A.5.5.8.2.1.1	Test Purpose and Environment	1016

A.5.5.8.2.1.2	Test Requirements	1019

A.5.5.9	Uplink spatial relation switch delay	1019

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1019

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1019

A.5.5.9.1.1.1	Test Purpose and Environment	1019

A.5.5.9.1.1.2	Test Requirements	1022

A.5.5.9.2	RRC based spatial relation switch	1022

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1022

A.5.5.9.2.1.1	Test Purpose and Environment	1022

A.5.5.9.2.1.2	Test Requirements	1025

A.5.5.10	UE specific CBW change	1025

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1025

A.5.5.10.1.1	Test Purpose and Environment	1025

A.5.5.10.1.2	Test Requirements	1028

A.5.6	Measurement procedure	1029

A.5.6.1	Intra-frequency Measurements	1029

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1029

A.5.6.1.1.1	Test purpose and Environment	1029

A.5.6.1.1.2	Test Requirements	1031

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1031

A.5.6.1.2.1	Test purpose and Environment	1031

A.5.6.1.2.2	Test Requirements	1034

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1035

A.5.6.1.3.1	Test purpose and Environment	1035

A.5.6.1.3.2	Test Requirements	1038

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1038

A.5.6.1.4.1	Test purpose and Environment	1038

A.5.6.1.4.2	Test Requirements	1042

A.5.6.2	Inter-frequency Measurements	1043

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1043

A.5.6.2.1.1	Test Purpose and Environment	1043

A.5.6.2.1.2	Test Requirements	1046

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1046

A.5.6.2.2.1	Test Purpose and Environment	1046

A.5.6.2.2.2	Test Requirements	1050

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1050

A.5.6.2.3.1	Test Purpose and Environment	1050

A.5.6.2.3.2	Test Requirements	1054

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1054

A.5.6.2.4.1	Test Purpose and Environment	1054

A.5.6.2.4.2	Test Requirements	1057

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1057

A.5.6.2.5.1	Test Purpose and Environment	1057

A.5.6.2.5.2	Test Requirements	1062

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1063

A.5.6.2.6.1	Test Purpose and Environment	1063

A.5.6.2.6.2	Test Requirements	1066

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1066

A.5.6.2.7.1	Test Purpose and Environment	1066

A.5.6.2.7.2	Test Requirements	1071

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1071

A.5.6.2.8.1	Test Purpose and Environment	1071

A.5.6.2.8.2	Test Requirements	1075

A.5.6.3	L1-RSRP measurement for beam reporting	1076

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1076

A.5.6.3.1.1	Test Purpose and Environment	1076

A.5.6.3.1.2	Test parameters	1076

A.5.6.3.1.3	Test Requirements	1078

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1078

A.5.6.3.2.1	Test Purpose and Environment	1078

A.5.6.3.2.2	Test parameters	1079

A.5.6.3.2.3	Test Requirements	1081

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1082

A.5.6.3.3.1	Test Purpose and Environment	1082

A.5.6.3.3.2	Test parameters	1082

A.5.6.3.3.3	Test Requirements	1084

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1085

A.5.6.3.4.1	Test Purpose and Environment	1085

A.5.6.3.4.2	Test parameters	1085

A.5.6.3.4.3	Test Requirements	1087

A.5.6.4	CLI measurements	1088

A.5.6.4.1	SRS-RSRP measurement with DRX	1088

A.5.6.4.1.1	Test Purpose and Environment	1088

A.5.6.4.1.2	Test Parameters	1088

A.5.6.4.1.3	Test Requirements	1090

A.5.6.4.2	CLI-RSSI measurement with DRX	1090

A.5.6.4.2.1	Test Purpose and Environment	1090

A.5.6.4.2.2	Test Parameters	1091

A.5.6.4.2.3	Test Requirements	1093

A.5.6.4	Measurements with autonomous gaps	1093

A.5.6.4.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1093

A.5.6.4.1.1	Test Purpose and Environment	1093

A.5.6.4.1.2	Test Requirements	1096

A.5.6.5	L1-SINR measurement for beam reporting	1097

A.5.6.5.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1097

A.5.6.5.1.1	Test Purpose and Environment	1097

A.5.6.5.1.2	Test parameters	1097

A.5.6.5.1.3	Test Requirements	1099

A.5.6.5.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1099

A.5.6.5.2.1	Test Purpose and Environment	1099

A.5.6.5.2.2	Test parameters	1100

A.5.6.5.2.3	Test Requirements	1103

A.5.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1103

A.5.6.5.3.1	Test Purpose and Environment	1103

A.5.6.5.3.2	Test parameters	1103

A.5.6.5.3.3	Test Requirements	1105

A.5.7	Measurement Performance requirements	1105

A.5.7.1	SS-RSRP	1106

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1106

A.5.7.1.1.1	Test Purpose and Environment	1106

A.5.7.1.1.2	Test parameters	1106

A.5.7.1.1.3	Test Requirements	1108

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1109

A.5.7.1.2.1	Test Purpose and Environment	1109

A.5.7.1.2.2	Test parameters	1109

A.5.7.1.2.3	Test Requirements	1113

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1114

A.5.7.1.3.1	Test Purpose and Environment	1114

A.5.7.1.3.2	Test parameters	1114

A.5.7.1.3.3	Test Requirements	1117

A.5.7.2	SS-RSRQ	1117

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1117

A.5.7.2.1.1	Test Purpose and Environment	1117

A.5.7.2.1.2	Test Parameters	1118

A.5.7.2.1.3	Test Requirements	1122

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1122

A.5.7.2.2.1	Test Purpose and Environment	1122

A.5.7.2.2.2	Test Parameters	1122

A.5.7.2.2.3	Test Requirements	1125

A.5.7.3	SS-SINR	1125

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1125

A.5.7.3.1.1	Test Purpose and Environment	1125

A.5.7.3.1.2	Test Parameters	1125

A.5.7.3.1.3	Test Requirements	1128

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1128

A.5.7.3.2.1	Test Purpose and Environment	1128

A.5.7.3.2.2	Test Parameters	1128

A.5.7.3.2.3	Test Requirements	1130

A.5.7.4	L1-RSRP measurement for beam reporting	1131

A.5.7.4.1	SSB based L1-RSRP measurement	1131

A.5.7.4.1.1	Test Purpose and Environment	1131

A.5.7.4.1.2	Test parameters	1131

A.5.7.4.1.3	Test Requirements	1133

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1134

A.5.7.4.2.1	Test Purpose and Environment	1134

A.5.7.4.2.2	Test parameters	1134

A.5.7.4.2.3	Test Requirements	1136

A.5.7.5	CLI measurements	1137

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1137

A.5.7.5.1.1	Test Purpose and Environment	1137

A.5.7.5.1.2	Test parameters	1137

A.5.7.5.1.3	Test Requirements	1140

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1141

A.5.7.5.2.1	Test Purpose and Environment	1141

A.5.7.5.2.2	Test parameters	1141

A.5.7.5.2.3	Test Requirements	1143

A.5.7.6	L1-SINR measurement for beam reporting	1144

A.5.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1144

A.5.7.6.1.1	Test Purpose and Environment	1144

A.5.7.6.1.2	Test parameters	1144

A.5.7.6.1.3	Test Requirements	1146

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1146

A.5.7.6.2.1	Test Purpose and Environment	1146

A.5.7.6.2.2	Test parameters	1147

A.5.7.6.2.3	Test Requirements	1149

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1150

A.5.7.6.3.1	Test Purpose and Environment	1150

A.5.7.6.3.2	Test parameters	1150

A.5.7.6.3.3	Test Requirements	1152

A.5.8	Void	1152

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1028

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1028

A.6.1.1.5.1	Test Purpose and Environment	1028

A.6.1.1.5.2	Test Parameters	1028

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1032

A.6.1.1.6.1	Test Purpose and Environment	1032

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1042

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1042

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1042

A.6.1.2.1.1	Test Purpose and Environment	1042

A.6.1.2.1.2	Test Parameters	1042

A.6.1.2.1.3	Test Requirements	1045

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1046

A.6.1.2.2.1	Test Purpose and Environment	1046

A.6.1.2.2.2	Test Parameters	1046

A.6.1.2.2.3	Test Requirements	1049

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1050

A.6.1.2.3.1	Test Purpose and Environment	1050

A.6.1.2.3.2	Test Parameters	1050

A.6.1.2.3.3	Test Requirements	1053

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1054

A.6.1.2.4.1	Test Purpose and Environment	1054

A.6.1.2.4.2	Test Parameters	1054

A.6.1.2.4.3	Test Requirements	1058

A.6.2	SA: RRC_INACTIVE state mobility	1061

A.6.3	RRC_CONNECTED state mobility	1061

A.6.3.1	Handover	1061

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1061

A.6.3.1.1.1	Test Purpose and Environment	1061

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1065

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1065

A.6.3.1.2.1	Test Purpose and Environment	1065

A.6.3.1.2.2	Test Parameters	1065

A.6.3.1.2.3	Test Requirements	1069

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1069

A.6.3.1.3.1	Test Purpose and Environment	1069

A.6.3.1.3.2	Test Parameters	1069

A.6.3.1.3.3 Test Requirements	1073

A.6.3.1.4	 SA NR - E-UTRAN handover	1073

A.6.3.1.4.1	Test Purpose and Environment	1073

A.6.3.1.4.2	Test Requirements	1079

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1079

A.6.3.1.5.1	Test Purpose and Environment	1079

A.6.3.1.5.2	Test Requirements	1085

A.6.3.1.6	 SA NR - UTRAN FDD handover	1085

A.6.3.1.6.1	Test Purpose and Environment	1085

A.6.3.1.6.2	Test Requirements	1089

A.6.3.1.7	Intra-frequency handover in FR1; synchronous scenario	1089

A.6.3.1.7.1	Test Purpose and Environment	1089

A.6.3.1.7.2	Test Parameters	1089

A.6.3.1.7.3 Test Requirements	1093

A.6.3.1.8	Intra-frequency handover in FR1; asynchronous scenario	1094

A.6.3.1.8.1	Test Purpose and Environment	1094

A.6.3.1.8.2	Test Parameters	1094

A.6.3.1.8.3 Test Requirements	1098

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1099

A.6.3.1.9.1	Test Purpose and Environment	1099

A.6.3.1.9.2	Test Parameters	1099

A.6.3.1.9.3 Test Requirements	1103

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1103

A.6.3.1.10.1	Test Purpose and Environment	1103

A.6.3.1.10.2	Test Parameters	1103

A.6.3.1.10.3 Test Requirements	1107

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1107

A.6.3.1.11.1	Test Purpose and Environment	1107

A.6.3.1.11.2	Test Parameters	1107

A.6.3.1.11.3 Test Requirements	1114

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1114

A.6.3.1.12.1	Test Purpose and Environment	1114

A.6.3.1.12.2	Test Parameters	1114

A.6.3.1.12.3 Test Requirements	1121

A.6.3.2	RRC Connection Mobility Control	1121

A.6.3.2.1	SA: RRC Re-establishment	1121

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1121

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1125

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1129

A.6.3.2.2	Random Access	1132

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1132

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1136

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1141

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1146

A.6.3.2.3	SA: RRC Connection Release with Redirection	1150

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1150

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1154

A.6.3.3 Conditional handover	1161

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1161

A.6.3.3.1.1	Test Purpose and Environment	1161

A.6.3.3.1.2	Test Parameters	1161

A.6.3.3.1.3 Test Requirements	1165

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1165

A.6.3.3.2.1	Test Purpose and Environment	1165

A.6.3.3.2.2	Test Parameters	1165

A.6.3.3.2.3 Test Requirements	1169

A.6.4	Timing	1169

A.6.4.1	UE transmit timing	1169

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1169

A.6.4.1.1.1	Test Purpose and environment	1169

A.6.4.1.1.2	Test requirements	1173

A.6.4.2	UE timer accuracy	1173

A.6.4.3	Timing advance	1173

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1173

A.6.4.3.1.1	Test Purpose and Environment	1173

A.6.4.3.1.2	Test Parameters	1173

A.6.4.3.1.3	Test Requirements	1177

A.6.5	Signalling characteristics	1177

A.6.5.1	Radio link Monitoring	1177

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1178

A.6.5.1.1.1	Test Purpose and Environment	1178

A.6.5.1.1.2	Test Requirements	1183

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1183

A.6.5.1.2.1	Test Purpose and Environment	1183

A.6.5.1.2.2	Test Requirements	1189

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1189

A.6.5.1.3.1	Test Purpose and Environment	1189

A.6.5.1.3.2	Test Requirements	1195

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1195

A.6.5.1.4.1	Test Purpose and Environment	1195

A.6.5.1.4.2	Test Requirements	1201

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1201

A.6.5.1.5.1	Test Purpose and Environment	1201

A.6.5.1.5.2	Test Requirements	1207

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1207

A.6.5.1.6.1	Test Purpose and Environment	1207

A.6.5.1.6.2	Test Requirements	1213

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1213

A.6.5.1.7.1	Test Purpose and Environment	1213

A.6.5.1.7.2	Test Requirements	1217

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1217

A.6.5.1.8.1	Test Purpose and Environment	1217

A.6.5.1.8.2	Test Requirements	1223

A.6.5.2	Interruption	1223

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1223

A.6.5.2.1.2	Test Requirements	1227

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1227

A.6.5.2.2.1	Test Purpose and Environment	1227

A.6.5.2.2.2	Test Parameters	1228

A.6.5.2.2.3	Test Requirements	1230

A.6.5.3	SCell Activation and Deactivation Delay	1230

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1230

A.6.5.3.1.1	Test Purpose and Environment	1230

A.6.5.3.1.2	Test Requirements	1235

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	1235

A.6.5.3.2.1	Test Purpose and Environment	1235

A.6.5.3.2.2	Test Requirements	1236

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1236

A.6.5.3.3.1	Test Purpose and Environment	1236

A.6.5.3.3.2	Test Requirements	1237

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1237

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1237

A.6.5.4.1.1	Test Purpose and Environment	1237

A.6.5.4.1.2	Test Requirements	1246

A.6.5.4.2	Void	1246

A.6.5.5	Beam Failure Detection and Link recovery procedures	1246

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1246

A.6.5.5.1.1	Test Purpose and Environment	1246

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1252

A.6.5.5.2.1	Test Purpose and Environment	1252

A.6.5.5.2.2	Test Requirements	1259

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1259

A.6.5.5.3.1	Test Purpose and Environment	1259

A.6.5.5.3.2	Test Requirements	1265

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1265

A.6.5.5.4.1	Test Purpose and Environment	1265

A.6.5.5.4.2	Test Requirements	1271

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1271

A.6.5.5.5.1	Test Purpose and Environment	1271

A.6.5.5.5.2	Test Requirements	1275

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1276

A.6.5.5.6.1	Test Purpose and Environment	1276

A.6.5.5.6.2	Test Requirements	1280

A.6.5.6	Active BWP switch	1280

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1280

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	1280

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1285

A.6.5.6.2	RRC-based Active BWP Switch	1290

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1290

A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1294

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1294

A.6.5.6.3.1.1	Test Purpose and Environment	1294

A.6.5.6.3.1.2	Test Requirements	1299

A.6.5.7	DL interruptions at switching between two uplink carriers	1300

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1300

A.6.5.7.1.1	Test Purpose and Environment	1300

A.6.5.7.1.2	Test Requirements	1304

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1304

A.6.5.7.2.1	Test Purpose and Environment	1304

A.6.5.7.2.2	Test Requirements	1308

A.6.5.8	UE specific CBW change	1308

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1308

A.6.5.8.1.1	Test Purpose and Environment	1308

A.6.5.8.1.2	Test Requirements	1312

A.6.6	Measurement procedure	1312

A.6.6.1	Intra-frequency Measurements	1312

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1312

A.6.6.1.1.1	Test purpose and Environment	1312

A.6.6.1.1.2	Test parameters	1312

A.6.6.1.1.3	Test Requirements	1316

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1316

A.6.6.1.2.1	Test purpose and Environment	1316

A.6.6.1.2.2	Test parameters	1316

A.6.6.1.2.3	Test Requirements	1320

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1320

A.6.6.1.3.1	Test purpose and Environment	1320

A.6.6.1.3.2	Test parameters	1320

A.6.6.1.3.3	Test Requirements	1324

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1324

A.6.6.1.4.1	Test purpose and Environment	1324

A.6.6.1.4.2	Test parameters	1324

A.6.6.1.4.3	Test Requirements	1328

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1328

A.6.6.1.5.1	Test purpose and Environment	1328

A.6.6.1.5.2	Test parameters	1328

A.6.6.1.5.3	Test Requirements	1330

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1331

A.6.6.1.6.1	Test purpose and Environment	1331

A.6.6.1.6.2	Test parameters	1331

A.6.6.1.6.3	Test Requirements	1332

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1333

A.6.6.1.7.1	Test purpose and Environment	1333

A.6.6.1.7.2	Test parameters	1333

A.6.6.1.7.3	Test Requirements	1337

A.6.6.2	Inter-frequency Measurements	1337

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1337

A.6.6.2.1.1	Test Purpose and Environment	1337

A.6.6.2.1.2	Test Requirements	1341

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1341

A.6.6.2.2.1	Test Purpose and Environment	1341

A.6.6.2.2.2	Test Requirements	1345

A.6.6.2.3	Void	1346

A.6.6.2.4	Void	1346

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1346

A.6.6.2.5.1	Test Purpose and Environment	1346

A.6.6.2.5.2	Test Requirements	1350

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1350

A.6.6.2.6.1	Test Purpose and Environment	1350

A.6.6.2.6.2	Test Requirements	1354

A.6.6.2.7	Void	1355

A.6.6.2.8	Void	1355

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1355

A.6.6.2.9.1	Test Purpose and Environment	1355

A.6.6.2.9.2	Test Requirements	1358

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1358

A.6.6.2.10.1	Test Purpose and Environment	1358

A.6.6.2.10.2	Test Requirements	1362

A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	1363

A.6.6.2.11.1	Test Purpose and Environment	1363

A.6.6.2.11.2	Test Requirements	1366

A.6.6.3	Inter-RAT Measurements	1366

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1366

A.6.6.3.1.1	Test Purpose and Environment	1366

A.6.6.3.1.2	Test Requirements	1372

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1372

A.6.6.3.2.1	Test Purpose and Environment	1372

A.6.6.3.2.2	Test Requirements	1379

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1379

A.6.6.3.3.1	Test Purpose and Environment	1379

A.6.6.3.3.2	Test Requirements	1386

A.6.6.4	L1-RSRP measurement for beam reporting	1386

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1386

A.6.6.4.1.1	Test Purpose and Environment	1386

A.6.6.4.1.2	Test parameters	1386

A.6.6.4.1.3	Test Requirements	1389

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1389

A.6.6.4.2.1	Test Purpose and Environment	1389

A.6.6.4.2.2	Test parameters	1390

A.6.6.4.2.3	Test Requirements	1393

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1393

A.6.6.4.3.1	Test Purpose and Environment	1393

A.6.6.4.3.2	Test parameters	1394

A.6.6.4.3.3	Test Requirements	1397

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1397

A.6.6.4.4.1	Test Purpose and Environment	1397

A.6.6.4.4.2	Test parameters	1398

A.6.6.4.4.3	Test Requirements	1401

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1401

A.6.6.4.5.1	Test Purpose and Environment	1401

A.6.6.4.5.2	Test parameters	1402

A.6.6.4.5.3	Test Requirements	1405

A.6.6.5	1405

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1405

A.6.6.5.1.1	Test Purpose and Environment	1405

A.6.6.5.1.2	Test Requirements	1409

A.6.6.6	CLI measurements	1409

A.6.6.6.1	SRS-RSRP measurement with DRX	1409

A.6.6.6.1.1	Test Purpose and Environment	1409

A.6.6.6.1.2	Test Parameters	1410

A.6.6.6.1.3	Test Requirements	1413

A.6.6.6.2	CLI-RSSI measurement with DRX	1413

A.6.6.6.2.1	Test Purpose and Environment	1413

A.6.6.6.2.2	Test Parameters	1414

A.6.6.6.2.3	Test Requirements	1416

A.6.6.7	NR measurements with autonomous gaps	1416

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1416

A.6.6.7.1.1	Test Purpose and Environment	1416

A.6.6.7.1.2	Test Parameters	1416

A.6.6.7.1.3	Test Requirements	1419

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1419

A.6.6.7.2.1	Test Purpose and Environment	1419

A.6.6.7.2.2	Test Requirements	1422

A.6.6.8	L1-SINR measurement for beam reporting	1423

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1423

A.6.6.8.1.1	Test Purpose and Environment	1423

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1426

A.6.6.8.2.1	Test Purpose and Environment	1426

A.6.6.8.2.2	Test parameters	1427

A.6.6.8.2.3	Test Requirements	1431

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1431

A.6.6.8.3.1	Test Purpose and Environment	1431

A.6.6.8.3.2	Test parameters	1432

A.6.6.8.3.3	Test Requirements	1435

A.6.7	Measurement Performance requirements	1435

A.6.7.1	SS-RSRP	1436

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1436

A.6.7.1.1.1	Test Purpose and Environment	1436

A.6.7.1.1.2	Test parameters	1436

A.6.7.1.1.3	Test Requirements	1441

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1441

A.6.7.1.2.1	Test Purpose and Environment	1441

A.6.7.1.2.2	Test parameters	1441

A.6.7.1.2.3	Test Requirements	1445

A.6.7.1.3	Void	1445

A.6.7.2	SS-RSRQ	1445

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1445

A.6.7.2.1.1	Test Purpose and Environment	1445

A.6.7.2.1.2	Test Parameters	1446

A.6.7.2.1.3	Test Requirements	1451

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1451

A.6.7.2.2.1	Test Purpose and Environment	1451

A.6.7.2.2.2	Test Parameters	1451

A.6.7.2.2.3	Test Requirements	1456

A.6.7.3	SS-SINR	1456

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1456

A.6.7.3.1.1	Test Purpose and Environment	1456

A.6.7.3.1.2	Test Parameters	1457

A.6.7.3.1.3	Test Requirements	1461

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1461

A.6.7.3.2.1	Test Purpose and Environment	1461

A.6.7.3.2.2	Test Parameters	1461

A.6.7.3.2.3	Test Requirements	1467

A.6.7.4	L1-RSRP measurement for beam reporting	1468

A.6.7.4.1	SSB based L1-RSRP measurement	1468

A.6.7.4.1.1	Test Purpose and Environment	1468

A.6.7.4.1.2	Test parameters	1468

A.6.7.4.1.3	Test Requirements	1472

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1472

A.6.7.4.2.1	Test Purpose and Environment	1472

A.6.7.4.2.2	Test parameters	1473

A.6.7.4.2.3	Test Requirements	1477

A.6.7.5	E-UTRAN RSRP	1477

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1477

A.6.7.5.1.1	Test Purpose and Environment	1477

A.6.7.5.1.2	Test parameters	1478

A.6.7.5.1.3	Test Requirements	1483

A.6.7.6	E-UTRAN RSRQ	1484

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1484

A.6.7.6.1.1	Test Purpose and Environment	1484

A.6.7.6.1.2	Test parameters	1484

A.6.7.6.1.3	Test Requirements	1489

A.6.7.7	E-UTRAN RS-SINR	1490

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1490

A.6.7.7.1.1	Test Purpose and Environment	1490

A.6.7.7.1.2	Test parameters	1490

A.6.7.7.1.3	Test Requirements	1495

A.6.7.8	CLI measurements	1496

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1496

A.6.7.8.1.1	Test Purpose and Environment	1496

A.6.7.8.1.2	Test parameters	1496

A.6.7.8.1.3	Test Requirements	1502

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1502

A.6.7.8.2.1	Test Purpose and Environment	1502

A.6.7.8.2.2	Test parameters	1503

A.6.7.8.2.3	Test Requirements	1506

A.6.7.9	L1-SINR measurement for beam reporting	1507

A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1507

A.6.7.9.1.1	Test Purpose and Environment	1507

A.6.7.9.1.2	Test parameters	1507

A.6.7.9.1.3	Test Requirements	1511

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1511

A.6.7.9.2.1	Test Purpose and Environment	1511

A.6.7.9.2.2	Test parameters	1512

A.6.7.9.2.3	Test Requirements	1517

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1517

A.6.7.9.3.1	Test Purpose and Environment	1517

A.6.7.9.3.2	Test parameters	1517

A.6.7.9.3.3	Test Requirements	1523

The L1-SINR measurement accuracy for CSI-RS#0+CSI-IM#0 and CSI-RS#1+CSI-IM# of Cell 1 shall fulfil the requirements in clause 10.1.27.3.	1523

A.7	NR standalone tests with one or more NR cells in FR2	1523

A.7.1	SA: RRC_IDLE state mobility	1523

A.7.1.1	Cell re-selection to NR	1523

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1523

A.7.1.1.1.1	Test Purpose and Environment	1523

A.7.1.1.1.2	Test Parameters	1523

A.7.1.1.1.3	Test Requirements	1527

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1527

A.7.1.1.2.1	Test Purpose and Environment	1527

A.7.1.1.2.2	Test Parameters	1527

A.7.1.1.2.3	Test Requirements	1531

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1531

A.7.1.1.3.1	Test Purpose and Environment	1531

A.7.1.1.3.2	Test Parameters	1531

A.7.1.1.3.3	Test Requirements	1534

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1534

A.7.1.1.4.1	Test Purpose and Environment	1534

A.7.1.1.4.2	Test Parameters	1534

A.7.1.1.4.3	Test Requirements	1537

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1537

A.7.1.1.5.1	Test Purpose and Environment	1537

A.7.1.1.5.2	Test Parameters	1537

A.7.1.1.5.3	Test Requirements	1541

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1541

A.7.1.1.6.1	Test Purpose and Environment	1541

A.7.1.1.6.2	Test Parameters	1541

A.7.1.1.6.3	Test Requirements	1544

A.7.2	SA: RRC_INACTIVE state mobility	1544

A.7.3	RRC_CONNECTED state mobility	1544

A.7.3.1	Handover	1544

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1544

A.7.3.1.1.1	Test Purpose and Environment	1544

A.7.3.1.1.2	Test Parameters	1544

A.7.3.1.1.3	Test Requirements	1548

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1548

A.7.3.1.2.1	Test Purpose and Environment	1548

A.7.3.1.2.2	Test Parameters	1548

A.7.3.1.2.3	Test Requirements	1551

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1551

A.7.3.1.3.1	Test Purpose and Environment	1551

A.7.3.1.3.2	Test Parameters	1551

A.7.3.1.3.3 Test Requirements	1552

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1553

A.7.3.1.4.1	Test Purpose and Environment	1553

A.7.3.1.4.2	Test Parameters	1553

A.7.3.1.4.3 Test Requirements	1560

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1560

A.7.3.1.5.1	Test Purpose and Environment	1560

A.7.3.1.5.2	Test Parameters	1560

A.7.3.1.5.3 Test Requirements	1566

A.7.3.2	RRC Connection Mobility Control	1566

A.7.3.2.1	SA: RRC Re-establishment	1566

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1566

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1569

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1572

A.7.3.2.1.3.1	Test Purpose and Environment	1572

A.7.3.2.1.3.2	Test Requirements	1574

A.7.3.2.2	Random Access	1575

A.7.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	1575

A.7.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	1579

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1583

A.7.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	1586

A.7.3.2.3	SA: RRC Connection Release with Redirection	1589

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1589

A.7.3.3 Conditional Handover	1592

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1592

A.7.3.3.1.1	Test Purpose and Environment	1592

A.7.3.3.1.2	Test Parameters	1592

A.7.3.3.1.2.3	Test Requirements	1595

A.7.3.3.2	Inter-frequency handover from FR2 to FR2; unknown target cell	1595

A.7.3.3.2.1	Test Purpose and Environment	1595

A.7.3.3.2.2	Test Parameters	1595

A.7.3.3.2.3 Test Requirements	1597

A.7.4	Timing	1597

A.7.4.1	UE transmit timing	1597

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1597

A.7.4.1.1.1	Test Purpose and environment	1597

A.7.4.1.1.2	Test requirements	1600

A.7.4.2	UE timer accuracy	1600

A.7.4.3	Timing advance	1600

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1600

A.7.4.3.1.1	Test Purpose and Environment	1600

A.7.4.3.1.2	Test Parameters	1600

A.7.4.3.1.3 Test Requirements	1603

A.7.5	Signaling characteristics	1604

A.7.5.1	Radio link Monitoring	1604

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1604

A.7.5.1.1.1	Test Purpose and Environment	1604

A.7.5.1.1.2	Test Requirements	1607

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1608

A.7.5.1.2.1	Test Purpose and Environment	1608

A.7.5.1.2.2	Test Requirements	1612

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1613

A.7.5.1.3.1	Test Purpose and Environment	1613

A.7.5.1.3.2	Test Requirements	1616

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1616

A.7.5.1.4.1	Test Purpose and Environment	1616

A.7.5.1.4.2	Test Requirements	1621

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1621

A.7.5.1.5.1	Test Purpose and Environment	1621

A.7.5.1.5.2	Test Requirements	1625

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1625

A.7.5.1.6.1	Test Purpose and Environment	1625

A.7.5.1.6.2	Test Requirements	1629

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1629

A.7.5.1.7.1	Test Purpose and Environment	1629

A.7.5.1.7.2	Test Requirements	1632

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1632

A.7.5.1.8.1	Test Purpose and Environment	1632

A.7.5.1.8.2	Test Requirements	1637

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1637

A.7.5.1.9.1	Test Purpose and Environment	1637

A.7.5.1.9.2	Test Requirements	1638

A.7.5.2	Interruption	1639

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1639

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1642

A.7.5.2.2.1	Test Purpose and Environment	1642

A.7.5.2.2.2	Test Parameters	1642

A.7.5.2.2.3	Test Requirements	1644

A.7.5.3	SCell Activation and Deactivation Delay	1644

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1644

A.7.5.3.1.1	Test Purpose and Environment	1644

A.7.5.3.1.2	Test Requirements	1646

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1646

A.7.5.3.2.1	Test Purpose and Environment	1646

A.7.5.3.2.2	Test Requirements	1651

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1652

A.7.5.3.3.1	Test Purpose and Environment	1652

A.7.5.3.3.2	Test Requirements	1655

A.7.5.4	Void	1656

A.7.5.5	Beam Failure Detection and Link recovery procedures	1656

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1656

A.7.5.5.1.1	Test Purpose and Environment	1656

A.7.5.5.1.2	Test Requirements	1661

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1661

A.7.5.5.2.1	Test Purpose and Environment	1661

A.7.5.5.2.2	Test Requirements	1666

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1666

A.7.5.5.3.1	Test Purpose and Environment	1666

A.7.5.5.3.2	Test Requirements	1672

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1672

A.7.5.5.4.1	Test Purpose and Environment	1672

A.7.5.5.4.2	Test Requirements	1678

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1678

A.7.5.5.5.1	Test Purpose and Environment	1678

A.7.5.5.5.2	Test Requirements	1683

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1683

A.7.5.5.6.1	Test Purpose and Environment	1683

A.7.5.5.6.2	Test Requirements	1687

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1687

A.7.5.5.7.1	Test Purpose and Environment	1687

A.7.5.5.7.2	Test Requirements	1692

A.7.5.6	Active BWP switch	1692

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1692

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1692

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1697

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1702

A.7.5.6.1.3.1	Test Purpose and Environment	1702

A.7.5.6.1.3.2	Test Requirements	1705

A.7.5.6.2	RRC-based Active BWP Switch	1705

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1709

A.7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	1709

A.7.5.6.3.1.1	Test Purpose and Environment	1709

A.7.5.6.3.1.2	Test Requirements	1712

A.7.5.7	PSCell addition and release delay	1712

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1712

A.7.5.7.1.1	Test Purpose and Environment	1712

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1716

A.7.5.7.2.1	Test Purpose and Environment	1716

A.7.5.7.2.2	Test Requirements	1720

A.7.5.8	Active TCI state switch delay	1720

A.7.5.8.1	MAC-CE based active TCI state switch	1720

A.7.5.8.2	RRC based active TCI state switch	1722

A.7.5.9	Uplink spatial relation switch delay	1725

A.7.5.9.1	MAC-CE based Spatial Relation switch	1725

A.7.5.9.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	1725

A.7.5.9.1.1.2	Test Requirements	1728

A.7.5.9.2	RRC based spatial relation switch	1728

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1728

A.7.5.9.2.1.1	Test Purpose and Environment	1728

A.7.5.9.2.1.2	Test Requirements	1731

A.7.5.10	UE specific CBW change	1731

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1731

A.7.5.10.1.1	Test Purpose and Environment	1731

A.7.5.10.1.2	Test Requirements	1734

A.7.6	Measurement procedure	1735

A.7.6.1	Intra-frequency Measurements	1735

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1735

A.7.6.1.1.1	Test purpose and Environment	1735

A.7.6.1.1.2	Test Requirements	1737

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1737

A.7.6.1.2.1	Test purpose and Environment	1737

A.7.6.1.2.2	Test Requirements	1740

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1741

A.7.6.1.3.1	Test purpose and Environment	1741

A.7.6.1.3.2	Test Requirements	1743

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1743

A.7.6.1.4.1	Test purpose and Environment	1743

A.7.6.1.4.2	Test Requirements	1746

A.7.6.2	Inter-frequency Measurements	1747

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1747

A.7.6.2.1.1	Test Purpose and Environment	1747

A.7.6.2.1.2	Test Requirements	1750

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1750

A.7.6.2.2.1	Test Purpose and Environment	1750

A.7.6.2.2.2	Test Requirements	1754

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1754

A.7.6.2.3.1	Test Purpose and Environment	1754

A.7.6.2.3.2	Test Requirements	1758

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1758

A.7.6.2.4.1	Test Purpose and Environment	1758

A.7.6.2.4.2	Test Requirements	1762

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1762

A.7.6.2.5.1	Test Purpose and Environment	1762

A.7.6.2.5.2	Test Requirements	1767

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1768

A.7.6.2.6.1	Test Purpose and Environment	1768

A.7.6.2.6.2	Test Requirements	1772

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1773

A.7.6.2.7.1	Test Purpose and Environment	1773

A.7.6.2.7.2	Test Requirements	1777

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1778

A.7.6.2.8.1	Test Purpose and Environment	1778

A.7.6.2.8.2	Test Requirements	1782

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1783

A.7.6.2.9.1	Test Purpose and Environment	1783

A.7.6.2.9.2	Test Requirements	1786

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1786

A.7.6.2.10.1	Test Purpose and Environment	1786

A.7.6.2.10.2	Test Requirements	1788

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1788

A.7.6.2.11.1	Test Purpose and Environment	1788

A.7.6.2.11.2	Test Requirements	1791

A.7.6.3	L1-RSRP measurement for beam reporting	1792

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1792

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1792

A.7.6.3.1.1	Test Purpose and Environment	1792

A.7.6.3.1.2	Test parameters	1792

A.7.6.3.1.3	Test Requirements	1794

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1794

A.7.6.3.2.1	Test Purpose and Environment	1794

A.7.6.3.2.2	Test parameters	1795

A.7.6.3.2.3	Test Requirements	1797

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1797

A.7.6.3.3.1	Test Purpose and Environment	1797

A.7.6.3.3.2	Test parameters	1798

A.7.6.3.3.3	Test Requirements	1800

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1801

A.7.6.3.4.1	Test Purpose and Environment	1801

A.7.6.3.4.2	Test parameters	1801

A.7.6.3.3.3	Test Requirements	1803

A.7.6.4	CLI measurements	1804

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1804

A.7.6.4.1.1	Test Purpose and Environment	1804

A.7.6.4.1.2	Test Parameters	1804

A.7.6.4.1.3	Test Requirements	1806

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1806

A.7.6.4.2.1	Test Purpose and Environment	1806

A.7.6.4.2.2	Test Parameters	1807

A.7.6.4.2.3	Test Requirements	1808

A.7.6.5	NR Measurements with autonomous gaps	1809

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1809

A.7.6.5.1.1	Test Purpose and Environment	1809

A.7.6.5.1.2	Test Requirements	1812

A.7.6.6	L1-SINR measurement for beam reporting	1812

A.7.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	1812

A.7.6.6.1.1	Test Purpose and Environment	1812

A.7.6.6.1.2	Test parameters	1812

A.7.6.6.1.3	Test Requirements	1814

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1814

A.7.6.6.2.1	Test Purpose and Environment	1814

A.7.6.6.2.2	Test parameters	1815

A.7.6.6.2.3	Test Requirements	1817

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1817

A.7.6.6.3.1	Test Purpose and Environment	1817

A.7.6.6.3.2	Test parameters	1818

A.7.6.6.3.3	Test Requirements	1820

A.7.7	Measurement Performance requirements	1820

A.7.7.1	SS-RSRP	1821

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1821

A.7.7.1.1.1	Test Purpose and Environment	1821

A.7.7.1.1.2	Test parameters	1821

A.7.7.1.1.3	Test Requirements	1825

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1825

A.7.7.1.2.1	Test Purpose and Environment	1825

A.7.7.1.2.2	Test parameters	1826

A.7.7.1.2.3	Test Requirements	1829

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1830

A.7.7.1.3.1	Test Purpose and Environment	1830

A.7.7.1.3.2	Test parameters	1830

A.7.7.1.3.3	Test Requirements	1833

A.7.7.2	SS-RSRQ	1834

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1834

A.7.7.2.1.1	Test Purpose and Environment	1834

A.7.7.2.1.2	Test Parameters	1834

A.7.7.2.1.3	Test Requirements	1836

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1836

A.7.7.2.2.1	Test Purpose and Environment	1836

A.7.7.2.2.2	Test Parameters	1836

A.7.7.2.2.3	Test Requirements	1838

A.7.7.3	SS-SINR	1838

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1838

A.7.7.3.1.1	Test Purpose and Environment	1838

A.7.7.3.1.2	Test Parameters	1838

A.7.7.3.1.3	Test Requirements	1841

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1841

A.7.7.3.2.1	Test Purpose and Environment	1841

A.7.7.3.2.2	Test Parameters	1841

A.7.7.3.2.3	Test Requirements	1843

A.7.7.4	L1-RSRP measurement for beam reporting	1843

A.7.7.4.1	SSB based L1-RSRP measurement	1843

A.7.7.4.1.1	Test Purpose and Environment	1843

A.7.7.4.1.2	Test parameters	1844

A.7.7.4.1.3	Test Requirements	1846

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1847

A.7.7.4.2.1	Test Purpose and Environment	1847

A.7.7.4.2.2	Test parameters	1847

A.7.7.4.2.3	Test Requirements	1849

A.7.7.5	CLI measurements	1850

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	1850

A.7.7.5.1.1	Test Purpose and Environment	1850

A.7.7.5.1.2	Test parameters	1850

A.7.7.5.1.3	Test Requirements	1853

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	1854

A.7.7.5.2.1	Test Purpose and Environment	1854

A.7.7.5.2.2	Test parameters	1854

A.7.7.5.2.3	Test Requirements	1856

A.7.7.6	L1-SINR measurement for beam reporting	1857

A.7.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1857

A.7.7.6.1.1	Test Purpose and Environment	1857

A.7.7.6.1.2	Test parameters	1857

A.7.7.6.1.3	Test Requirements	1859

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1859

A.7.7.6.2.1	Test Purpose and Environment	1859

A.7.7.6.2.2	Test parameters	1860

A.7.7.6.2.3	Test Requirements	1862

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1862

A.7.7.6.3.1	Test Purpose and Environment	1862

A.7.7.6.3.2	Test parameters	1863

A.7.7.6.3.3	Test Requirements	1865

A.8	E-UTRA standalone tests for NR RRM	1865

A.8.1	Void	1866

A.8.2	RRC_IDLE state mobility	1866

A.8.2.1	Inter-RAT NR Cell re-selection	1866

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	1866

A.8.2.1.1.1	Test Purpose and Environment	1866

A.8.2.1.1.2	Test Requirements	1871

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	1872

A.8.2.1.2.1	Test Purpose and Environment	1872

A.8.2.1.2.2	Test Requirements	1877

A.8.3	RRC_CONNECTED state mobility	1878

A.8.3.1	Handover	1878

A.8.3.1.1	E-UTRAN - NR handover in FR1	1878

A.8.3.1.1.1	Test Purpose and Environment	1878

A.8.3.1.1.2	Test Requirements	1883

A.8.4	Measurement procedure	1883

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	1883

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	1883

A.8.4.1.1.1	Test Purpose and Environment	1883

A.8.4.1.1.2	Test Requirements	1885

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	1886

A.8.4.1.2.1	Test Purpose and Environment	1886

A.8.4.1.2.2	Test Requirements	1887

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	1887

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1887

A.8.4.2.1.1	Test Purpose and Environment	1887

A.8.4.2.1.2	Test Requirements	1892

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1892

A.8.4.2.2.1	Test Purpose and Environment	1892

A.8.4.2.2.2	Test Requirements	1896

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1896

A.8.4.2.3.1	Test Purpose and Environment	1896

A.8.4.2.3.2	Test Requirements	1901

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1901

A.8.4.2.4.1	Test Purpose and Environment	1901

A.8.4.2.4.2	Test Requirements	1905

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	1905

A.8.4.2.5.1	Test Purpose and Environment	1905

A.8.4.2.5.2	Test Requirements	1907

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	1908

A.8.4.2.6.1	Test Purpose and Environment	1908

A.8.4.2.6.2	Test Requirements	1910

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	1911

A.8.4.2.7.1	Test Purpose and Environment	1911

A.8.4.2.7.2	Test Requirements	1913

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	1914

A.8.4.2.8.1	Test Purpose and Environment	1914

A.8.4.2.8.2	Test Requirements	1916

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	1917

A.8.4.2.9.1	Test Purpose and Environment	1917

A.8.4.2.9.2	Test Requirements	1923

A.8.5	Measurement performance	1923

A.8.5.1	SFTD accuracy	1923

A.8.5.1.1	SFTD accuracy	1923

A.8.5.1.1.1	Test Purpose	1923

A.8.5.1.1.2	Test Environment	1923

A.8.5.1.1.3	Test Requirements	1929

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	1929

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1929

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1934

A.8.5.2.1.2.1	Test Purpose and Environment	1934

A.8.5.2.1.2.2	Test Parameters	1934

A.8.5.2.1.2.3	Test Requirements	1936

A.8.5.2.2	SS-RSRQ	1936

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1936

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1941

A.8.5.2.2.2.1	Test Purpose and Environment	1941

A.8.5.2.2.2.2	Test Parameters	1941

A.8.5.2.2.2.3	Test Requirements	1943

A.8.5.2.3	SS-SINR	1943

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1943

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1947

A.8.5.2.3.2.1	Test Purpose and Environment	1947

A.8.5.2.3.2.2	Test Parameters	1947

A.8.5.2.3.2.3	Test Requirements	1949

A.9	V2X Tests	1950

A.9.1	V2X Tests in FR1	1950

A.9.1.1	Test for V2X UE Transmit Timing	1950

A.9.1.1.1	Test for GNSS as Synchronization Reference Source	1950

A.9.1.1.1.1	Test Purpose and Environment	1950

A.9.1.1.1.2	Test requirements	1950

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	1950

A.9.1.1.2.1	Test Purpose and Environment	1950

A.9.1.1.2.2	Test requirements	1951

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	1951

A.9.1.1.3.1	Test Purpose and Environment	1951

A.9.1.1.3.2	Test requirements	1954

A.9.1.2	Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	1954

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	1954

A.9.1.2.1.1	Test Purpose and Environment	1954

A.9.1.2.1.2	Test Requirements	1958

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	1958

A.9.1.2.2.1	Test Purpose and Environment	1958

A.9.1.2.2.2	Test Requirements	1959

A.9.1.3	Test for V2X Synchronization Reference Selection/Reselection	1960

A.9.1.3.1	Test for GNSS configured as the highest priority	1960

A.9.1.3.1.1	Test Purpose and Environment	1960

A.9.1.3.1.2	Test Requirements	1962

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	1963

A.9.1.3.2.1	Test Purpose and Environment	1963

A.9.1.3.2.2	Test Requirements	1966

A.9.1.4	Test for L1 SL-RSRP Measurement	1967

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	1967

A.9.1.4.1.1	Test Purpose and Environment	1967

A.9.1.4.1.2	Test Requirements	1969

A.9.1.4.2	Test for V2X UE Resource Pre-emption	1969

A.9.1.4.2.1	Test Purpose and Environment	1969

A.9.1.4.2.2	Test Requirements	1973

A.9.1.4.3	Test for V2X UE Resource Re-evaluation	1973

A.9.1.4.3.1	Test Purpose and Environment	1973

A.9.1.4.3.2	Test Requirements	1978

A.9.1.5	Test for Congestion Control Measurement	1978

A.9.1.5.1	Test Purpose and Environment	1978

A.9.1.5.2	Test Requirements	1981

A.9.1.6	Test for Interruption	1981

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	1981

A.9.1.6.1.1	Test Purpose and Environment	1981

A.9.1.6.1.2	Test Requirements	1985

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1818

B.1	Conditions for NR RRC_IDLE state mobility	1818

B.1.1	Introduction	1818

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1818

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1819

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1820

B.2.1	Introduction	1820

B.2.1.1	General	1820

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1820

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1820

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1820

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1821

B.2.1.4	Gain to SS-RSRP measurement point for FR1	1821

B.2.1.5	Gain to SS-RSRP measurement point for FR2	1822

B.2.1.5.1	Gain to SS-RSRP measurement point for Rx Beam Peak angle of arrival	1822

B.2.2	Conditions for NR intra-frequency measurements	1822

B.2.3	Conditions for NR inter-frequency measurements	1823

B.2.4	Conditions for NR L1-RSRP reporting	1825

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1825

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1826

B.2.5	Conditions for RRC connection release with redirection to NR	1827

B.2.6	Void	1829

B.2.6.1	Void	1829

B.2.6.2	Void	1829

B.3	RRM Requirements Exceptions	1830

B.3.1	Introduction	1830

B.3.2	Receiver sensitivity relaxation for CA	1830

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1830

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1830

B.3.2.2.1	Inter-band carrier aggregation	1830

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1830

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1831

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1831

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1831

B.3.2.4.1	Intra-band contiguous carrier aggregation	1831

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1831

B.3.3	Receiver sensitivity relaxation for DC	1831

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1831

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1831

B.3.4	Receiver sensitivity relaxation for SUL	1831

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1831

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1832

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1832

B.4	Conditions for V2X	1832

B.4.1	Test parameters for GNSS signals	1832

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1832

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1833

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1833

Annex C (informative): Change history	1834