| 3GPP TS 38.133 V17.2.0 (2021-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	52

3.6.1	RRC connected state requirements in DRX	53

3.6.2	Number of serving carriers	53

3.6.2.1	Number of serving carriers for SA	53

3.6.2.2	Number of serving carriers for EN-DC	53

3.6.2.3	Number of serving carriers for NE-DC	53

3.6.2.4	Number of serving carriers for NR-DC	54

3.6.3	Applicability for intra-band FR2	54

3.6.4	Applicability for FR2 UE power classes	54

3.6.5	Applicability for SDL bands	54

3.6.6	Applicability of requirements for NGEN-DC operation	54

3.6.7	Applicability of QCL	54

3.6.9	Applicability of requirements for scheduling availability	55

3.6.10	Applicability of requirements for measurement restrictions	55

4	SA: RRC_IDLE state mobility	55

4.1	Cell Selection	55

4.2	Cell Re-selection	55

4.2.1	Introduction	55

4.2.2	Requirements	56

4.2.2.1	UE measurement capability	56

4.2.2.2	Measurement and evaluation of serving cell	56

4.2.2.3	Measurements of intra-frequency NR cells	56

4.2.2.4	Measurements of inter-frequency NR cells	58

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	60

4.2.2.6	Maximum interruption in paging reception	62

4.2.2.7	General requirements	62

4.2.2.8	Minimum requirement at transitions	63

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	63

4.2.2.9.1	Introduction	63

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	63

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	64

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	64

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	65

4.2.2.10.1	Introduction	65

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	65

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	66

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	66

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	67

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	67

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	68

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	68

4.2A	Cell Re-selection when subject to CCA	69

4.2A.1	Introduction	69

4.2A.2	Requirements	69

4.2A.2.1	UE measurement capability	69

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	69

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	70

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	71

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	73

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	73

4.2A.2.7	General requirements	73

4.3	Minimization of Drive Tests (MDT)	74

4.3.1	Introduction	74

4.3.2	Measurement Requirements	74

4.3.3	Requirements for Relative Time Stamp Accuracy	74

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	75

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	75

4.4	Idle Mode CA/DC Measurements	75

4.4.1	Introduction	75

4.4.2	Measurement Requirements	75

4.4.2.1	Detected cell requirement during state transition and Idle mode	75

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	76

4.4.2.3	Measurements on serving cell	77

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	77

5	SA: RRC_INACTIVE state mobility	79

5.1	Cell Re-selection	79

5.1.1	Introduction	79

5.1.2	Requirements	79

5.1.2.1	UE measurement capability	79

5.1.2.2	Measurement and evaluation of serving cell	79

5.1.2.3	Measurements of intra-frequency NR cells	79

5.1.2.4	Measurements of inter-frequency NR cells	79

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	79

5.1.2.6	Maximum interruption in paging reception	79

5.1.2.7	General requirements	79

5.1A	Cell Re-selection with CCA	79

5.1A.1	Introduction	79

5.1A.2	Requirements	80

5.1A.2.1	UE measurement capability	80

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	80

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	80

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	80

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	80

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	80

5.1A.2.7	General requirements	80

5.2	Void	80

5.3	Minimization of Drive Tests (MDT)	80

5.3.1	Introduction	80

5.3.2	Measurement Requirements	81

5.3.3	Requirements for Relative Time Stamp Accuracy	81

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	81

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	81

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	81

5.4	Idle Mode CA/DC Measurements	81

5.4.1	Introduction	81

5.4.2	Measurement Requirements	81

5.4.2.1	Detected cell requirement during state transition and Idle mode	82

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	82

5.4.2.3	Measurements on serving cell	82

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	82

6	RRC_CONNECTED state mobility	82

6.1	Handover	82

6.1.1	NR Handover	82

6.1.1.1	Introduction	82

6.1.1.2	NR FR1 - NR FR1 Handover	82

6.1.1.2.1	Handover delay	82

6.1.1.2.2	Interruption time	82

6.1.1.3	NR FR2- NR FR1 Handover	83

6.1.1.3.1	Handover delay	83

6.1.1.3.2	Interruption time	83

6.1.1.4	NR FR2- NR FR2 Handover	84

6.1.1.4.1	Handover delay	84

6.1.1.4.2	Interruption time	84

6.1.1.5	NR FR1- NR FR2 Handover	85

6.1.1.5.1	Handover delay	85

6.1.1.5.2	Interruption time	85

6.1.2	NR Handover to other RATs	86

6.1.2.1	NR – E-UTRAN Handover	86

6.1.2.1.1	Introduction	86

6.1.2.1.2	Handover delay	86

6.1.2.1.3	Interruption time	86

6.1.2.2	NR – UTRAN Handover	86

6.1.2.2.1	Introduction	86

6.1.2.2.2	Handover delay	87

6.1.2.2.3	Interruption time	87

6.1.3	NR DAPS Handover	87

6.1.3.1	Introduction	87

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	88

6.1.3.2.1	DAPS handover delay	88

6.1.3.2.2	Interruption time	89

6.1.3.3	NR FR2- NR FR1 DAPS Handover	90

6.1.3.3.1	DAPS handover delay	91

6.1.3.3.2	Interruption time	91

6.1.3.4	NR FR1- NR FR2 DAPS Handover	92

6.1.3.4.1	DAPS handover delay	92

6.1.3.4.2	Interruption time	92

6.1.4	NR Conditional Handover	93

6.1.4.1	Introduction	93

6.1.4.2	NR FR1 – NR FR1 conditional handover	93

6.1.4.3	NR FR2 – NR FR1 conditional handover	94

6.1.4.4	NR FR2 – NR FR2 conditional handover	94

6.1.4.4.1	Handover delay	94

6.1.4.4.2	Measurement time	95

6.1.4.4.3	Preparation time	95

6.1.4.4.4	Interruption time	95

6.1.4.5	NR FR1 – NR FR2 conditional handover	96

6.1A	Void	96

6.1A.1	Void	96

6.1A.1.1	Void	96

6.1A.1.2	Void	96

6.1A.1.2.1	Void	96

6.1A.1.2.2	Void	96

6.1B	Handover to target cell using CCA	96

6.1B.1	NR Handover	96

6.1B.1.1	Introduction	96

6.1B.1.2	NR FR1 - NR FR1 Handover	96

6.1B.1.2.1	Handover delay	96

6.1B.1.2.2	Interruption time	97

6.2	RRC Connection Mobility Control	97

6.2.1	SA: RRC Re-establishment	97

6.2.1.1	Introduction	97

6.2.1.2	Requirements	98

6.2.1.2.1	UE Re-establishment delay requirement	98

6.2.1A	RRC Re-establishment with CCA	99

6.2.1A.1	Introduction	99

6.2.1A.2	Requirements	99

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	100

6.2.2	Random access	101

6.2.2.1	Introduction	101

6.2.2.2	Requirements for 4-step RA type	101

6.2.2.2.1	Contention based random access	102

6.2.2.2.2	Non-Contention based random access	103

6.2.2.2.3	UE behaviour when configured with supplementary UL	104

6.2.2.3	Requirements for 2-step RA type	104

6.2.2.3.1	Contention based random access	104

6.2.2.3.2	Non-Contention based random access	105

6.2.2.3.3	UE behaviour when configured with supplementary UL	105

6.2.2A	Random access when CCA is used on target frequency	106

6.2.2A.1	Introduction	106

6.2.2A.2	Requirements for 4-step RA type	106

6.2.2A.2.1	Contention based random access	106

6.2.2A.2.2	Non-Contention based random access	107

6.2.2A.3	Requirements for 2-step RA type	108

6.2.2A.3.1	Contention based random access	109

6.2.2A.3.2	Non-Contention based random access	110

6.2.3	SA: RRC Connection Release with Redirection	110

6.2.3.1	Introduction	110

6.2.3.2	Requirements	111

6.2.3.2.1	RRC connection release with redirection to NR	111

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	112

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	112

7	Timing	113

7.1	UE transmit timing	113

7.1.1	Introduction	113

7.1.2	Requirements	114

7.1.2.1	Gradual timing adjustment	115

7.1.2.2	Void	115

7.2	UE timer accuracy	115

7.2.1	Introduction	115

7.2.2	Requirements	116

7.3	Timing advance	116

7.3.1	Introduction	116

7.3.2	Requirements	116

7.3.2.1	Timing Advance adjustment delay	116

7.3.2.2	Timing Advance adjustment accuracy	116

7.4	Cell phase synchronization accuracy	116

7.4.1	Definition	116

7.4.2	Minimum requirements	116

7.5	Maximum Transmission Timing Difference	117

7.5.1	Introduction	117

7.5.2	Minimum Requirements for inter-band EN-DC	117

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	117

7.5.3	Minimum Requirements for intra-band EN-DC	118

7.5.4	Minimum Requirements for NR Carrier Aggregation	118

7.5.5	Minimum Requirements for inter-band NE-DC	118

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	119

7.5.6	Minimum Requirements for inter-band NR DC	119

7.6	Maximum Receive Timing Difference	120

7.6.1	Introduction	120

7.6.2	Minimum Requirements for inter-band EN-DC	120

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	120

7.6.3	Minimum Requirements for intra-band EN-DC	121

7.6.4	Minimum Requirements for NR Carrier Aggregation	121

7.6.5	Minimum Requirements for inter-band NE-DC	122

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	122

7.6.6	Minimum Requirements for inter-band NR DC	123

7.7	deriveSSB-IndexFromCell tolerance	123

7.7.1	Minimum requirements	123

7.8	Void	124

8	Signalling characteristics	124

8.1	Radio Link Monitoring	124

8.1.1	Introduction	124

8.1.2	Requirements for SSB based radio link monitoring	125

8.1.2.1	Introduction	125

8.1.2.2	Minimum requirement	126

8.1.2.3	Measurement restrictions for SSB based RLM	127

8.1.3	Requirements for CSI-RS based radio link monitoring	128

8.1.3.1	Introduction	128

8.1.3.2	Minimum requirement	129

8.1.3.3	Measurement restrictions for CSI-RS based RLM	131

8.1.4	Minimum requirement at transitions	131

8.1.5	Minimum requirement for UE turning off the transmitter	132

8.1.6	Minimum requirement for L1 indication	132

8.1.7	Scheduling availability of UE during radio link monitoring	132

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	132

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	132

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	133

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	133

8.1A	Radio Link Monitoring with CCA on Target Frequency	133

8.1A.1	Introduction	133

8.1A.2	Requirements for SSB Based Radio Link Monitoring	134

8.1A.2.1	Introduction	134

8.1A.2.2	Minimum Requirement	135

8.1A.3	Minimum requirement at transitions	136

8.1A.4	Minimum requirement for UE turning off the transmitter	136

8.1A.5	Minimum requirement for L1 indication	137

8.1A.6	Scheduling availability of UE during radio link monitoring	137

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	137

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	137

8.2	Interruption	137

8.2.1	EN-DC Interruption	137

8.2.1.1	Introduction	137

8.2.1.2	Requirements	138

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	138

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	138

8.2.1.2.3	Interruptions at SCell addition/release	139

8.2.1.2.4	Interruptions at SCell activation/deactivation	140

8.2.1.2.5	Interruptions during measurements on SCC	141

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	142

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	143

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	144

8.2.1.2.9	Interruptions at SCell hibernation	144

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	144

8.2.1.2.11	Interruptions due to UE-specific CBW change	145

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	145

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	146

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	147

8.2.1.2.15	Interruptions due to SCell dormancy	148

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	148

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	149

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	149

8.2.2.1	Introduction	149

8.2.2.2	Requirements	150

8.2.2.2.1	Interruptions at SCell addition/release	150

8.2.2.2.2	Interruptions at SCell activation/deactivation	151

8.2.2.2.3	Interruptions during measurements on deactivated SCC	152

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	153

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	153

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	154

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	155

8.2.2.2.8	Interruptions due to UE-specific CBW change	155

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	156

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	157

8.2.2.2.11	Interruptions at direct SCell activation	158

8.2.2.2.12	Interruptions due to SCell dormancy	158

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	158

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	159

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	159

8.2.3	NE-DC Interruptions	160

8.2.3.1	Introduction	160

8.2.3.2	Requirements	160

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	160

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	161

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	161

8.2.3.2.4	Interruptions at SCell activation/deactivation	162

8.2.3.2.5	Interruptions during measurements on SCC	163

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	164

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	165

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	165

8.2.3.2.9	Interruptions at SCell hibernation	165

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	166

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	166

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	167

8.2.3.2.13	Interruptions due to SCell dormancy	168

8.2.3.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	169

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	169

8.2.4	NR-DC: Interruptions	170

8.2.4.1	Introduction	170

8.2.4.2	Requirements	170

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	170

8.2.4.2.2	Interruptions at SCell activation/deactivation	171

8.2.4.2.3	Interruptions during measurements on SCC	172

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	172

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	173

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	173

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	173

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	173

8.2.4.2.9	Interruptions at NR SRS carrier based switching	174

8.2.4.2.10	Interruptions at direct SCell activation	175

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	175

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	176

8.2.4.2.13	 Interruptions due to SCell dormancy	177

8.2.4.2A	Void	177

8.2.4.2A.1	Void	177

8.2.4.2A.2	Void	177

8.2.4.2A.3	Void	177

8.3	SCell Activation and Deactivation Delay	177

8.3.1	Introduction	177

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	177

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	182

8.3.4	Direct SCell Activation at SCell addition	182

8.3.5	Direct SCell Activation at Handover	183

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	185

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	188

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	189

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	190

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	191

8.3A.1	Introduction	191

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	192

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	194

8.4	UE UL carrier RRC reconfiguration delay	194

8.4.1	Introduction	194

8.4.2	UE UL carrier configuration delay requirement	195

8.4.3	UE UL carrier deconfiguration delay requirement	195

8.5	Link Recovery Procedures	195

8.5.1	Introduction	195

8.5.2	Requirements for SSB based beam failure detection	196

8.5.2.1	Introduction	196

8.5.2.2	Minimum requirement	196

8.5.2.3	Measurement restriction for SSB based beam failure detection	198

8.5.3	Requirements for CSI-RS based beam failure detection	198

8.5.3.1	Introduction	198

8.5.3.2	Minimum requirement	199

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	201

8.5.4	Minimum requirement for L1 indication	202

8.5.5	Requirements for SSB based candidate beam detection	202

8.5.5.1	Introduction	202

8.5.5.2	Minimum requirement	202

8.5.5.3	Measurement restriction for SSB based candidate beam detection	204

8.5.6	Requirements for CSI-RS based candidate beam detection	205

8.5.6.1	Introduction	205

8.5.6.2	Minimum requirement	205

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	207

8.5.7	Scheduling availability of UE during beam failure detection	208

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	208

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	208

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	208

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	209

8.5.8	Scheduling availability of UE during candidate beam detection	209

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	209

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	209

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	209

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	210

8.5.9	Requirements for Beam Failure Recovery in SCell	210

8.5.9.1	Introduction	210

8.5.9.2	Requirement	210

8.5A	Link Recovery Procedures when CCA is used on target frequency	211

8.5A.1	Introduction	211

8.5A.2	Requirements for SSB based beam failure detection	211

8.5A.2.1	Introduction	211

8.5A.2.2	Minimum requirement	212

8.5A.2.3	Measurement restriction for SSB based beam failure detection	212

8.5A.4	Minimum requirement for L1 indication	213

8.5A.5	Requirements for SSB based candidate beam detection	213

8.5A.5.1	Introduction	213

8.5A.5.2	Minimum requirement	213

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	214

8.5A.7	Scheduling availability of UE during beam failure detection	214

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	214

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	214

8.5A.8	Scheduling availability of UE during candidate beam detection	214

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	214

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	215

8.6	Active BWP switch delay	215

8.6.1	Introduction	215

8.6.2	DCI and timer based BWP switch delay on a single CC	215

8.6.2A	DCI based BWP switch delay on multiple CCs	216

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	216

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	218

8.6.2B	Timer based BWP switch delay on multiple CCs	218

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	218

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	219

8.6.3	RRC based BWP switch delay on a single CC	219

8.6.3A	RRC based BWP switch delay on multiple CCs	220

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	220

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	220

8.6.4	BWP switch delay on Consistent UL CCA recovery	221

8.7	Void	221

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	221

8.8.1	Introduction	221

8.8.2	E-UTRAN PSCell Addition Delay Requirement	221

8.8.3	E-UTRAN PSCell Release Delay Requirement	222

8.9	NR-DC: PSCell Addition and Release Delay	222

8.9.1	Introduction	222

8.9.2	PSCell Addition Delay Requirement	222

8.9.3	PSCell Release Delay Requirement	223

8.10	Active TCI state switching delay	223

8.10.4	DCI based TCI state switch delay	224

8.10.5	RRC based TCI state switch delay	224

8.10.6	Active TCI state list update delay	225

8.10A	Active TCI state switching delay with CCA	225

8.10A.1	Introduction	225

8.10A.2	Known conditions for TCI state	225

8.10A.3	MAC-CE based TCI state switch delay	226

8.10A.4	DCI based TCI state switch delay	226

8.10A.5	RRC based TCI state switch delay	227

8.10A.6	Active TCI state list update delay	227

8.11	PSCell Change	227

8.11A	void	228

8.11B	Conditional PSCell Change	228

8.11B.1	Introduction	228

8.11B.2	Conditoinal PSCell Change delay	228

8.11B.2.1	Measurement time	228

8.12	Uplink spatial relation switch delay	229

8.12.1	Introduction	229

8.12.2	Known conditions for spatial relation when associated with DL-RS	229

8.12.3	MAC-CE based spatial relation switch delay	229

8.12.4	DCI based spatial relation switch delay	230

8.12.5	RRC based spatial relation switch delay	230

8.13	UE-specific CBW change	230

8.13.1	Introduction	230

8.13.2	UE-specific CBW change delay	231

8.14	Pathloss reference signal switching delay	231

8.14.1	Introduction	231

8.14.2	Known conditions for pathloss reference signal	231

8.14.3	MAC-CE based pathloss reference signal switch delay	231

9	Measurement Procedure	232

9.1	General measurement requirement	232

9.1.1	Introduction	232

9.1.2	Measurement gap	232

9.1.2.1	EN-DC: Measurement Gap Sharing	243

9.1.2.1a	SA: Measurement Gap Sharing	244

9.1.2.1b	NE-DC: Measurement Gap Sharing	245

9.1.2.1c	NR-DC: Measurement Gap Sharing	246

9.1.3	UE Measurement capability	247

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	247

9.1.3.1a	SA: Monitoring of multiple layers using gaps	247

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	248

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	248

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	249

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	250

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	250

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	251

9.1A.3.2	Void	252

9.1.3A	UE Measurement capability under operation mode with CCA	252

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	252

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	252

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	252

9.1A.3.2a	Void	253

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	253

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	253

9.1.4.1	Introduction	253

9.1.4.2	Requirements	253

9.1.5	Carrier-specific scaling factor	257

9.1.5.1	Monitoring of multiple layers outside gaps	257

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	259

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	260

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	261

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	262

9.1.5.2	Monitoring of multiple layers within gaps	263

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	264

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	266

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	267

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	269

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	271

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	271

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	271

9.1.6	Minimum requirement at transitions	271

9.2	NR intra-frequency measurements	271

9.2.1	Introduction	271

9.2.2	Requirements applicability	272

9.2.3	Number of cells and number of SSB	272

9.2.3.1	Requirements for FR1	272

9.2.3.2	Requirements for FR2	272

9.2.4	Measurement Reporting Requirements	273

9.2.4.1	Periodic Reporting	273

9.2.4.2	Event-triggered Periodic Reporting	273

9.2.4.3	Event Triggered Reporting	273

9.2.5	Intrafrequency measurements without measurement gaps	274

9.2.5.1	Intrafrequency cell identification	274

9.2.5.2	Measurement period	276

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	278

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	278

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	278

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	279

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	279

9.2.5.4	SFTD Measurements between PCell and PSCell	280

9.2.5.4.1	Introduction	280

9.2.5.4.2	SFTD Measurement delay	280

9.2.5.4.3	SFTD Measurement Reporting Delay	280

9.2.6	Intra-frequency measurements with measurement gaps	281

9.2.6.1	Void	281

9.2.6.2	Intra-frequency cell identification	281

9.2.6.3	Intrafrequency Measurement Period	282

9.2A	NR intra-frequency measurements with CCA	283

9.2A.1	Introduction	283

9.2A.2	Requirements applicability	284

9.2A.3	Number of cells and number of SSB	284

9.2A.4	Measurement Reporting Requirements	284

9.2A.5	Intra-frequency measurements without measurement gaps	285

9.2A.5.2	Measurement period	287

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	288

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	288

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	289

9.2A.6	Intra-frequency measurements with measurement gaps	289

9.2A.6.1	Intra-frequency cell identification	289

9.2A.6.2	Intra-frequency Measurement Period	290

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	291

9.2A.7.1	Intra-frequency RSSI measurements	291

9.2A.7.2	Intra-frequency Channel occupancy measurements	292

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	293

9.3	NR inter-frequency measurements	293

9.3.1	Introduction	293

9.3.2	Requirements applicability	294

9.3.2.1	Void	294

9.3.2.2	Void	294

9.3.3	Number of cells and number of SSB	294

9.3.3.1	Requirements for FR1	294

9.3.3.2	Requirements for FR2	294

9.3.4	Inter-frequency measurement with measurement gaps	294

9.3.4.1	Void	296

9.3.4.2	Void	296

9.3.5	Inter-frequency measurements	296

9.3.5.1	Void	296

9.3.5.2	Void	296

9.3.5.3	Void	296

9.3.6	Inter-frequency measurements reporting requirements	296

9.3.6.1	Periodic Reporting	296

9.3.6.2	Event-triggered Periodic Reporting	297

9.3.6.3	Event-triggered Reporting	297

9.3.7	Void	297

9.3.8	Inter-frequency SFTD measurement requirements	297

9.3.8.1	Introduction	297

9.3.8.2	SFTD Measurement delay	297

9.3.8.3	SFTD Measurement reporting delay	298

9.3.9	Inter frequency measurements without measurement gaps	299

9.3.9.1	Inter frequency Cell identification	299

9.3.9.2	Measurement period	300

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	301

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	301

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	302

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	302

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	302

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	303

9.3A.1	Introduction	303

9.3A.2	Requirements applicability	303

9.3A.3	Number of cells and number of SSB	303

9.3A.3.1	Requirements	303

9.3A.4	Inter-frequency cell identification	304

9.3A.5	Inter-frequency measurements	305

9.3A.6	NR Inter-frequency measurements reporting requirements	306

9.3A.6.1	Periodic Reporting	306

9.3A.6.2	Event-triggered Periodic Reporting	306

9.3A.6.3	Event-triggered Reporting	306

9.3A.8	Inter-frequency RSSI measurements	306

9.3A.9	Inter-frequency channel occupancy measurements	307

9.4	Inter-RAT measurements	307

9.4.1	Introduction	307

9.4.2	NR − E-UTRAN FDD measurements	309

9.4.2.1	Introduction	309

9.4.2.2	Requirements when no DRX is used	309

9.4.2.3	Requirements when DRX is used	310

9.4.2.4	Measurement reporting requirements	311

9.4.2.4.1	Periodic Reporting	311

9.4.2.4.2	Event-Triggered Periodic Reporting	311

9.4.2.4.3	Event-Triggered Reporting	311

9.4.3	NR − E-UTRAN TDD measurements	311

9.4.3.1	Introduction	311

9.4.3.2	Requirements when no DRX is used	311

9.4.3.3	Requirements when DRX is used	312

9.4.3.4	Measurement reporting requirements	314

9.4.3.4.1	Periodic Reporting	314

9.4.3.4.2	Event-Triggered Periodic Reporting	314

9.4.3.4.3	Event-Triggered Reporting	314

9.4.4	Inter-RAT RSTD measurements	314

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	314

9.4.4.1.1	Introduction	314

9.4.4.1.2	Requirements	315

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	318

9.4.4.2.1	Introduction	318

9.4.4.2.2	Requirements	319

9.4.5	Inter-RAT E-CID measurements	322

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	322

9.4.5.1.1	Introduction	322

9.4.5.1.2	Requirements	322

9.4.5.1.3	Measurement Reporting Delay	322

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	322

9.4.5.2.1	Introduction	322

9.4.5.2.2	Requirements	322

9.4.5.2.3	Measurement Reporting Delay	323

9.4.6	NR − UTRAN FDD measurements	323

9.4.6.1	Introduction	323

9.4.6.2	Requirements when no DRX is used	323

9.4.6.3	Requirements when DRX is used	324

9.4.7	NR – E-UTRAN measurements with autonomous gaps	326

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	326

The requirement for identifying a new CGI of an E-UTRA cell within Tidentify_CGI, E-UTRAN is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.9.4.7.2	CGI reporting delay	326

9.5	L1-RSRP measurements for Reporting	327

9.5.1	Introduction	327

9.5.2	Requirements applicability	327

9.5.3	Measurement Reporting Requirements	327

9.5.3.1	Periodic Reporting	328

9.5.3.2	Semi-Persistent Reporting	328

9.5.3.3	Aperiodic Reporting	328

9.5.4	L1-RSRP measurement requirements	328

9.5.4.1	SSB based L1-RSRP Reporting	328

9.5.4.2	CSI-RS based L1-RSRP Reporting	330

9.5.4A	Void	333

9.5.4A.1	Void	333

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	333

9.5.5.1	Measurement restriction for SSB based L1-RSRP	333

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	334

9.5.6	Scheduling availability of UE during L1-RSRP measurement	334

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	334

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	334

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	335

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	336

9.5A	L1-RSRP measurements for Reporting under CCA	336

9.5A.1	Introduction	336

9.5A.2	Requirements applicability	336

9.5A.3	Measurement Reporting Requirements	336

9.5A.3.1	Periodic Reporting	337

9.5A.3.2	Semi-Persistent Reporting	337

9.5A.3.3	Aperiodic Reporting	337

9.5A.4	L1-RSRP measurement requirements	337

9.5A.4.1	SSB based L1-RSRP Reporting	337

9.5A.5	Measurement restriction for L1-RSRP measurement	338

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	338

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	339

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	339

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	339

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	339

9.6	NE-DC: Measurements	339

9.6.1	Introduction	339

9.6.2	SFTD Measurements	339

9.6.2.1	Introduction	339

9.6.2.2	SFTD Measurement requirements	339

9.7	Cross Link Interference measurements	340

9.7.1	Introduction	340

9.7.2	SRS-RSRP measurements	340

9.7.2.1	Introduction	340

9.7.2.2	Requirements applicability	341

9.7.2.3	Measurement Reporting Requirements	341

9.7.2.3.1	Periodic Reporting	341

9.7.2.3.2	Event-triggered Periodic Reporting	341

9.7.2.3.3	Event Triggered Reporting	341

9.7.2.4	Measurement capability	341

9.7.2.5	SRS-RSRP measurement period	341

9.7.3	CLI-RSSI measurements	342

9.7.3.1	Introduction	342

9.7.3.2	Requirements applicability	342

9.7.3.3	Measurement Reporting Requirements	342

9.7.3.3.1	Periodic Reporting	342

9.7.3.3.2	Event-triggered Periodic Reporting	342

9.7.3.3.3	Event Triggered Reporting	342

9.7.3.4	Measurement capability	343

9.7.3.5	CLI-RSSI measurement period	343

9.7.4	Scheduling availability of UE during CLI measurements	343

9.7.4.1	Scheduling availability of UE performing measurement on FR1	343

9.7.4.2	Scheduling availability of UE performing measurement on FR2	344

9.8	L1-SINR measurements for Reporting	344

9.8.1	Introduction	344

9.8.2	Requirements applicability	345

9.8.3	Measurement Reporting Requirements	345

9.8.3.1	Periodic Reporting	345

9.8.3.2	Semi-Persistent Reporting	346

9.8.4	L1-SINR measurement requirements	346

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	346

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	349

9.8.5	Measurement restriction for L1-SINR measurement	351

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	351

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	352

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	352

9.8.6	Scheduling availability of UE during L1-SINR measurement	353

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	353

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	353

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	354

9.9	 NR measurements for positioning	354

9.9.1	Introduction	354

9.9.2	RSTD measurements	355

9.9.2.1	Introduction	355

9.9.2.2	Requirements Applicability	355

9.9.2.3	Measurement Capability	355

9.9.2.4	Measurement Reporting Requirements	355

9.9.2.4.1	Void	355

9.9.2.4.2	Void	355

9.9.2.4.3	Void	355

9.9.2.5	Measurements Period Requirements	355

9.9.2.6	Void	357

9.9.3	PRS-RSRP measurements	357

9.9.3.1	Introduction	357

9.9.3.2	Requirements applicability	357

9.9.3.3	Measurement Capability	357

9.9.3.4	Measurement Reporting Requirements	358

9.9.3.5	Measurement Period Requirements	358

9.9.4	UE Rx-Tx time difference measurements	360

9.9.4.1 Introduction	360

9.9.4.2 Requirements Applicability	360

9.9.4.3 Measurement Capability	360

9.9.4.4 Measurement Reporting Requirements	360

9.9.4.5 Measurement Period Requirements	360

9.9.5.1	Introduction	362

9.9.5.2	Measurement Requirements	362

9.9.5.2.1	Intra-frequency Measurement Requirements	362

9.9.5.2.2	Inter-frequency Measurement Requirements	363

9.9.5.2.3	Measurement Reporting Delay	363

9.10	CSI-RS based L3 measurements	363

9.10.1	Introduction	363

9.10.2	CSI-RS based intra-frequency measurements	364

9.10.2.1	Introduction	364

9.10.2.2	Requirements applicability	364

9.10.2.3	Number of cells and number of CSI-RS	365

9.10.2.3.1	Requirements for FR1	365

9.10.2.3.2	Requirements for FR2	365

9.10.2.4	Measurement Reporting Requirements	366

9.10.2.4.1	Periodic Reporting	366

9.10.2.4.2	Event-triggered Periodic Reporting	366

9.10.2.4.3	Event Triggered Reporting	366

9.10.2.5	Intra-frequency measurements without measurement gaps	366

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	367

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	368

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	368

9.10.3	CSI-RS based Inter-frequency measurements	368

9.10.3.1	Introduction	368

9.10.3.2	Requirements applicability	368

9.10.3.3	Number of cells and number of CSI-RS resources	369

9.10.3.3.1	Requirements for FR1	369

9.10.3.3.2	Requirements for FR2	369

9.10.3.4	Measurements reporting requirements	369

9.10.3.4.1	Periodic Reporting	369

9.10.3.4.2	Event-triggered Periodic Reporting	369

9.10.3.4.3	Event-triggered Reporting	370

9.10.3.5	Inter frequency measurements with measurement gaps	370

9.11	NR measurements with autonomous gaps	371

9.11.1	Introduction	371

9.11.2	CGI identification of an NR cell with autonomous gaps	371

9.11.3	CGI reporting delay	372

10	Measurement Performance requirements	372

10.1	NR measurements	372

10.1.1	Introduction	372

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	373

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	373

10.1.2.1.1	Absolute SS-RSRP Accuracy	373

10.1.2.1.2	Relative SS-RSRP Accuracy	374

10.1.2.2	Void	375

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	375

10.1.2.3.1	Absolute CSI-RSRP Accuracy	375

10.1.2.3.2	Relative CSI-RSRP Accuracy	376

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	377

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	377

10.1.2B.1.1	Absolute SS-RSRP Accuracy	377

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	378

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	378

10.1.3.1.1	Absolute SS-RSRP Accuracy	378

10.1.3.1.2	Relative SS-RSRP Accuracy	379

10.1.3.2	Void	380

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	380

10.1.3.3.1	Absolute CSI-RSRP Accuracy	380

10.1.3.3.2	Relative CSI-RSRP Accuracy	380

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	381

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	381

10.1.3B.1.1	Absolute SS-RSRP Accuracy	381

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	382

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	382

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	382

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	383

10.1.4.2	Void	384

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	384

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	384

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	385

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	386

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	386

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	386

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	387

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	387

10.1.5.1.1	Absolute SS-RSRP Accuracy	387

10.1.5.1.2	Relative SS-RSRP Accuracy	388

10.1.5.2	Void	389

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	389

10.1.5.3.1	Absolute CSI-RSRP Accuracy	389

10.1.5.3.2	Relative CSI-RSRP Accuracy	389

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	390

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	390

10.1.5B.1.1	Absolute SS-RSRP Accuracy	390

10.1.6 RSRP Measurement Report Mapping	391

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	393

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	393

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	393

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	394

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	394

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	395

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	395

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	395

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	396

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	396

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	396

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	397

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	397

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	398

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	398

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	398

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	399

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	399

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	399

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	400

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	401

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	401

10.1.9.2.2	Relative CSI-RSRQ Accuracy	402

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	403

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	403

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	403

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	404

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	406

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	406

10.1.10.2.2	Relative CSI-RSRQ Accuracy	406

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	407

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	407

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	407

10.1.11	RSRQ report mapping	408

10.1.12	Intra-frequency SINR accuracy requirements for FR1	408

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	409

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	409

10.1.13	Intra-frequency SINR accuracy requirements for FR2	410

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	411

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	411

10.1.14	Inter-frequency SINR accuracy requirements for FR1	412

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	414

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	414

10.1.15	Inter-frequency SINR accuracy requirements for FR2	416

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	418

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	418

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	418

10.1.16	SINR report mapping	419

10.1.17	Power Headroom	420

10.1.18	PCMAX,c,f	421

10.1.19	L1-RSRP accuracy requirements for FR1	421

10.1.20	L1-RSRP accuracy requirements for FR2	425

10.1.21	SFTD accuracy requirements	428

10.1.22	CLI measurement accuracy requirements	432

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	436

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	436

10.1.23	RSTD Measurements	438

10.1.23.1	Introduction	438

10.1.23.2	Measurement Accuracy Requirements	439

10.1.23.3	Report mapping	443

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	443

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	445

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	447

10.1.24	PRS-RSRP Measurements	449

10.1.24.1	Introduction	449

10.1.24.2	Measurement Accuracy Requirements	449

10.1.24.2.1 Absolute PRS RSRP accuracy	449

10.1.24.2.2 Relative PRS RSRP accuracy	451

10.1.24.3	Report mapping	453

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	453

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	454

10.1.25	UE Rx-Tx Time Difference Measurements	456

10.1.25.1	Introduction	456

10.1.25.2	Measurement Accuracy Requirements	457

10.1.25.3	Report mapping	457

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	457

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	459

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	460

10.1.26	FR2 P-MPR report	462

10.1.26.1	Report mapping	462

10.1.27	L1-SINR accuracy requirements for FR1	462

10.1.28	L1-SINR accuracy requirements for FR2	470

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	477

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	477

10.1.29.1.1	Absolute SS-RSRQ Accuracy	477

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	477

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	477

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	477

10.1.30.1.2	Relative Accuracy of SS-RSRQ	478

10.1.31	Intra-frequency SINR accuracy requirements under CCA	479

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	479

10.1.31.1.1	Absolute SS-SINR Accuracy	479

10.1.32	Inter-frequency SINR accuracy requirements under CCA	479

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	479

10.1.32.1.1	Aboslute Accuracy of SS-SINR	479

10.1.32.1.2	Relative Accuracy of SS-SINR	480

10.1.33	L1-RSRP accuracy requirements under CCA	481

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	481

10.1.33.1.1	Absolute Accuracy	481

10.1.33.1.2	Relative Accuracy	481

10.1.34	RSSI measurements under CCA	482

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	482

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	482

10.1.34.3	RSSI measurement report mapping	482

10.1.35	Channel occupancy measurements under CCA	483

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	483

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	483

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	483

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	483

10.1.36.1.1	Absolute SS-RSRP Accuracy	483

10.1.36.1.2	Relative SS-RSRP Accuracy	484

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	485

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	485

10.1.37.1.1	Absolute Accuracy of SS-RSRP	485

10.1.37.1.2	Relative Accuracy of SS-RSRP	485

10.2	E-UTRAN measurements	486

10.2.1	Introduction	486

10.2.2	E-UTRAN RSRP measurements	486

10.2.3	E-UTRAN RSRQ measurements	486

10.2.4	E-UTRAN RSTD measurements	487

10.2.5	E-UTRAN RS-SINR measurements	487

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	487

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	488

10.3	UTRAN FDD Measurements	488

10.3.1	UTRAN FDD CPICH RSCP	488

10.3.2	UTRAN FDD CPICH Ec/No	489

10.4	V2X measurements	489

10.4.1	Introduction	489

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	489

10.4.2.1	PSBCH-RSRP Absolute Accuracy	489

10.4.2.2	PSBCH-RSRP Relative Accuracy	490

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	491

10.4.3.1	Absolute SL-RSSI Accuracy	491

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	491

10.4.4.1	Absolute L1 SL-RSRP Accuracy	491

11	Void	492

12	V2X Requirements	492

12.1	Introduction	492

12.2	UE Transmit Timing	492

12.2.1	Introduction	492

12.2.2	GNSS as synchronization reference source	493

12.2.3	NR Cell as synchronization reference source	493

12.2.4	E-URTAN Cell as synchronization reference source	493

12.2.5	SyncRef UE as synchronization reference source	494

12.3	Initiation/Cease of SLSS Transmissions	494

12.3.1	Introduction	494

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	494

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	495

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	496

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	496

12.4	Selection / Reselection of V2X Synchronization Reference Source	497

12.5	L1 SL-RSRP measurements	498

12.5.1	Introduction	498

12.5.2	SL-RSRP measurements	498

12.6	Congestion Control measurements	498

12.7	Interruption	498

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	498

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	499

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	501

12.8	Reliability of GNSS signal	501

12.9	Scheduling availability	501

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	501

13	Measurement Performance Requirements for NR gNB	502

13.1	UL-RTOA	502

13.1.1	Report mapping	502

13.2	gNB Rx-Tx time difference	504

13.2.1	Report mapping	504

13.2.2	Measurement Accuracy Requirements	506

13.2.2.1	Introduction	506

13.3	UL SRS RSRP measurement	507

13.3.1	Report mapping	507

13.3.2	Measurement accuracy requirements	508

13.3.2.1	Introduction	508

13.3.2.2	Requirements	508

13.4	AoA/ZoA	509

13.4.1	Report mapping	509

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1.2	CORESET for RMSI scheduling	429

A.3.1.2.1	FDD	429

A.3.1.2.2	TDD	430

A.3.1.3	CORESET for RMC scheduling	433

A.3.1.3.1	FDD	433

A.3.1.3.2	TDD	434

A.3.1.4	TDD UL/DL configuration	437

A.3.1A	Reference measurement channels under CCA	439

A.3.1A.1	PDSCH	439

A.3.1A.1.1	TDD	439

A.3.1A.2	CORESET for RMSI scheduling	440

A.3.1A.2.1	TDD	440

A.3.1A.3	CORESET for RMC scheduling	441

A.3.1A.3.1	TDD	441

A.3.1A.4	TDD UL/DL configuration	442

A.3.1A.5	RMC burst transmission model	442

RMC burst transmission is scheduled outside discovery burst transmission window. If transmission occurred in the previous slot, transmission is muted for a duration of one slot. Additionaly, if the start time of the candidate RMC burst transmission is within 5 slots of the start of the discovery burst transmission window, RMC transmission is not performed.	442

A.3.2	OFDMA channel noise generator (OCNG)	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	443

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	444

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	446

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	446

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	447

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	448

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	448

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	449

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	449

A.3.4	Test Cases with Different Channel Bandwidths	449

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	449

A.3.4.1.1	Introduction	449

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	450

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	450

A.3.5.1.1	Introduction	450

A.3.5.1.2	Principle of Testing	450

A.3.6	Antenna configurations	450

A.3.6.1	Antenna configurations for FR1	450

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	450

A.3.6.1.1.1	Introduction	450

A.3.6.1.1.2	Principle of testing	450

A.3.6.2	Antenna configurations for FR2	453

A.3.6A	Antenna configurations with unlicensed bands	453

A.3.6A.1	Antenna configurations for FR1	453

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	453

A.3.6A.1.1.1	Introduction	453

A.3.6A.1.1.2	Principle of testing	453

A.3.7	EN-DC test setup	455

A.3.7.1	Introduction	455

A.3.7.2	E-UTRAN Serving Cell Parameters	455

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	455

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	456

A.3.7A	NR FR1-FR2 test setup	457

A.3.7B	EN-DC test setup with unlicensed bands	458

A.3.7B.1	Introduction	458

A.3.7B.2	E-UTRAN Serving Cell Parameters	458

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	458

A.3.8	PRACH configurations	459

A.3.8.1	Introduction	459

A.3.8.2	PRACH configurations in FR1	459

A.3.8.2.1	FR1 PRACH configuration 1	459

A.3.8.2.2	FR1 PRACH configuration 2	460

A.3.8.2.3	FR1 PRACH configuration 3	461

A.3.8.2.4	FR1 PRACH configuration 4	462

A.3.8.3	PRACH configurations in FR2	463

A.3.8.3.1	FR2 PRACH configuration 1	463

A.3.8.3.2	FR2 PRACH configuration 2	464

A.3.8.3.3	FR2 PRACH configuration 3	465

A.3.8.3.4	FR2 PRACH configuration 4	466

A.3.8A	PRACH configurations under CCA	467

A.3.8A.1	Introduction	467

A.3.8A.2	PRACH configurations in FR1	467

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	467

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	468

A.3.9	BWP configurations	469

A.3.9.1	Introduction	469

A.3.9.2	Downlink BWP configurations	470

A.3.9.2.1	Initial BWP	470

A.3.9.2.2	Dedicated BWP	470

A.3.9.3	Uplink BWP configurations	470

A.3.9.3.1	Initial BWP	470

A.3.9.3.2	Dedicated BWP	471

A.3.10	SSB Configurations	471

A.3.10.1	SSB Configurations for FR1	471

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	471

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	473

A.3.10A	SSB Configurations under CCA	478

A.3.10A.1	SSB Configurations under CCA for FR1	478

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	479

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	480

A.3.11	SMTC Configurations	480

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	481

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	481

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	481

A.3.12	Test Cases with Different CC Configurations	481

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	481

A.3.12.1.1	Introduction	481

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	482

A.3.12.2.1	Introduction	482

A.3.12.2.2	Principle of testing	482

A.3.13	Test Cases in SA and EN-DC Operations	482

A.3.13.1	Introduction	482

A.3.13.2	Principle of Testing	482

A.3.14	CSI-RS configurations	482

A.3.14.1	FDD	482

A.3.14.2	TDD	486

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	492

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	492

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	492

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	492

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	493

A.3.15.3	Setup 3: 2 AoAs	493

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	493

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	493

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	493

A.3.16	TCI State Configuration	494

A.3.16.1	Introduction	494

A.3.16.2	TCI states	494

A.3.17	Configurations of CSI-RS for tracking	494

A.3.17.1	Configuration of CSI-RS for tracking for FR1	494

A.3.17.1.1	FDD	494

A.3.17.1.2	TDD	495

A.3.17.2	Configuration of CSI-RS for tracking for FR2	496

A.3.17.2.1	TDD	496

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	497

A.3.18.1	Introduction	497

A.3.18.2	PRACH Power Measurement	497

A.3.19	Test applicability for DAPS handover	497

A.3.19.1	Introduction	497

A.3.19.2	Principle of testing	497

A.3.20	MsgA configurations	498

A.3.20.1	Introduction	498

A.3.20.2	MsgA configurations in FR1	498

A.3.20.2.1	FR1 MsgA configuration 1	498

A.3.20.2.2	FR1 MsgA configuration 2	499

A.3.20.3	MsgA configurations in FR2	501

A.3.20.3.1	FR2 MsgA configuration 1	501

A.3.20.3.2	FR2 MsgA configuration 2	502

A.3.20A	MsgA configurations under CCA	504

A.3.20A.1	Introduction	504

A.3.20A.2	MsgA configurations in FR1	504

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	504

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	505

A.3.21	V2X sidelink communication	508

A.3.21.1	Introduction	508

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	508

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	514

A.3.22	CSI-IM configurations	514

A.3.22.1	FDD	514

A.3.22.2	TDD	515

A.3.23	Spatial Relation Configuration	517

A.3.23.1	Introduction	517

A.3.23.2	Spatial Relation	517

A.3.24	SRS configuration	518

A.3.25	Channel bandwidth (CBW) configurations	520

A.3.25.1	DL UE specific CBW	520

A.3.25.2	UL UE specific CBW	520

A.3.26	CCA model	520

A.3.26.1	Introduction	520

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	520

A.3.26.2.1	DL CCA model	520

A.3.26.2.2	UL CCA model	521

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	522

A.3.27.1	Introduction	522

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	522

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	522

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	522

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	522

A.3.28	Discovery Burst Transmission Window configuration under CCA	523

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	523

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	523

A.3.29.1	Introduction	523

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	523

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	523

A.3.30	CSI-RS configurations for RRM	524

A.3.30.1	FDD	524

A.3.30.2	TDD	525

A.3.31	PRS Configurations	527

A.3.31.1.	PRS Configurations for FR1	527

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz in 10 MHz	527

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz in 40 MHz	528

A.3.31.2.	PRS Configurations for FR2	528

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz in 100 MHz	528

A.4	EN-DC tests with all NR cells in FR1	529

A.4.1	Void	529

A.4.2	Void	529

A.4.3	RRC_CONNECTED state mobility	529

A.4.3.1	Void	529

A.4.3.2	RRC Connection Mobility Control	529

A.4.3.2.1	Void	529

A.4.3.2.2	Random Access	529

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	529

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	532

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	536

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	539

A.4.3.2.3	Void	543

A.4.4	Timing	543

A.4.4.1	UE transmit timing	543

A.4.4.1.1	NR UE Transmit Timing Test for FR1	543

A.4.4.1.1.1	Test Purpose and environment	543

A.4.4.1.1.2	Test requirements	547

A.4.4.2	UE timer accuracy	547

A.4.4.3	Timing advance	547

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	547

A.4.4.3.1.1	Test Purpose and Environment	547

A.4.4.3.1.2	Test Parameters	547

A.4.4.3.1.3	Test Requirements	551

A.4.5	Signaling characteristics	551

A.4.5.1	Radio link Monitoring	551

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	552

A.4.5.1.1.1	Test Purpose and Environment	552

A.4.5.1.1.2	Test Requirements	556

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	556

A.4.5.1.2.1	Test Purpose and Environment	556

A.4.5.1.2.2	Test Requirements	562

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	562

A.4.5.1.3.1	Test Purpose and Environment	562

A.4.5.1.3.2	Test Requirements	568

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	568

A.4.5.1.4.1	Test Purpose and Environment	568

A.4.5.1.4.2	Test Requirements	574

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	574

A.4.5.1.5.1	Test Purpose and Environment	574

A.4.5.1.5.2	Test Requirements	580

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	580

A.4.5.1.6.1	Test Purpose and Environment	580

A.4.5.1.6.2	Test Requirements	586

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	586

A.4.5.1.7.1	Test Purpose and Environment	586

A.4.5.1.7.2	Test Requirements	591

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	592

A.4.5.1.8.1	Test Purpose and Environment	592

A.4.5.1.8.2	Test Requirements	597

A.4.5.2	Interruption	598

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	598

A.4.5.2.1.1	Test Purpose and Environment	598

A.4.5.2.1.2	Test Requirements	601

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	601

A.4.5.2.2.1	Test Purpose and Environment	601

A.4.5.2.2.2	Test Requirements	605

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	605

A.4.5.2.3.1	Test Purpose and Environment	605

A.4.5.2.3.2	Test Requirements	609

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	610

A.4.5.2.4.1	Test Purpose and Environment	610

A.4.5.2.4.2	Test Requirements	613

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	613

A.4.5.2.5.1	Test Purpose and Environment	613

A.4.5.2.5.2	Test Requirements	617

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	617

A.4.5.2.6.1	Test Purpose and Environment	617

A.4.5.2.6.2	Test Requirements	621

A.4.5.2.7	Void	621

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	621

A.4.5.2.8.1	Test Purpose and Environment	621

A.4.5.2.8.2	Test Requirements	624

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	625

A.4.5.2.9.1	Test Purpose and Environment	625

A.4.5.2.9.2	Test Requirements	628

A.4.5.3	SCell Activation and Deactivation Delay	629

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	629

A.4.5.3.1.1	Test Purpose and Environment	629

A.4.5.3.1.2	Test Requirements	634

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	635

A.4.5.3.2.1	Test Purpose and Environment	635

A.4.5.3.2.2	Test Requirements	635

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	635

A.4.5.3.3.1	Test Purpose and Environment	635

A.4.5.3.3.2	Test Requirements	636

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	636

A.4.5.3.4.1	Test Purpose and Environment	636

A.4.5.3.3.2	Test Requirements	639

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	639

A.4.5.3.5.1	Test Purpose and Environment	639

A.4.5.3.5.2	Test Requirements	644

A.4.5.4	UE UL carrier RRC reconfiguration Delay	644

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	644

A.4.5.4.1.1 Test Purpose and Environment	644

A.4.5.4.1.2	Test Requirements	654

A.4.5.5	Beam Failure Detection and Link recovery procedures	654

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	654

A.4.5.5.1.1	Test Purpose and Environment	654

A.4.5.5.1.2	Test Requirements	659

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	659

A.4.5.5.2.1	Test Purpose and Environment	659

A.4.5.5.2.2	Test Requirements	665

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	665

A.4.5.5.3.1	Test Purpose and Environment	665

A.4.5.5.3.2	Test Requirements	671

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	671

A.4.5.5.4.1	Test Purpose and Environment	671

A.4.5.5.4.2	Test Requirements	677

A.4.5.6	Active BWP switch	691

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	691

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	691

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	696

A.4.5.6.2	RRC-based Active BWP Switch	700

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	705

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	705

A.4.5.6.3.1.1	Test Purpose and Environment	705

A.4.5.6.3.1.2	Test Requirements	710

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	711

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	711

A.4.5.6.4.1.1	Test Purpose and Environment	711

A.4.5.6.4.1.2	Test Requirements	715

A.4.5.6.5	SCell dormancy switch	715

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	715

A.4.5.6.5.1.1	Test Purpose and Environment	715

A.4.5.6.5.1.2	Test Requirements	721

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	721

A.4.5.6.5.2.1	Test Purpose and Environment	721

A.4.5.6.5.2.2	Test Requirements	726

A.4.5.7	PSCell addition and release delay	726

A.4.5.7.1	Addition and Release Delay of known NR PSCell	726

A.4.5.7.1.1	Test purpose and environment	726

A.4.5.7.1.2	Test Requirements	730

A.4.5.8	DL Interruptions at switching between two uplink carriers	730

A.4.5.8.1	Test Purpose and Environment	730

A.4.5.8.2	Test Requirements	734

A.4.6	Measurement procedure	734

A.4.6.1	Intra-frequency Measurements	734

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	734

A.4.6.1.1.1	Test purpose and Environment	734

A.4.6.1.1.2	Test parameters	734

A.4.6.1.1.3	Test Requirements	738

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	738

A.4.6.1.2.1	Test purpose and Environment	738

A.4.6.1.2.2	Test parameters	738

A.4.6.1.2.2	Test Requirements	742

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	742

A.4.6.1.3.1	Test purpose and Environment	742

A.4.6.1.3.2	Test parameters	742

A.4.6.1.3.3	Test Requirements	746

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	746

A.4.6.1.4.1	Test purpose and Environment	746

A.4.6.1.4.2	Test parameters	746

A.4.6.1.4.3	Test Requirements	750

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	750

A.4.6.1.5.1	Test purpose and Environment	750

A.4.6.1.5.2	Test parameters	750

A.4.6.1.5.3	Test Requirements	752

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	753

A.4.6.1.6.1	Test purpose and Environment	753

A.4.6.1.6.2	Test parameters	753

A.4.6.1.6.3	Test Requirements	755

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	756

A.4.6.1.7.1	Test purpose and Environment	756

A.4.6.1.7.2	Test parameters	756

A.4.6.1.7.3	Test Requirements	760

A.4.6.2	Inter-frequency Measurements	760

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	760

A.4.6.2.1.1	Test Purpose and Environment	760

A.4.6.2.1.2	Test Requirements	764

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	764

A.4.6.2.2.1	Test Purpose and Environment	764

A.4.6.2.2.2	Test Requirements	768

A.4.6.2.3	Void	769

A.4.6.2.4	Void	769

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	769

A.4.6.2.5.1	Test Purpose and Environment	769

A.4.6.2.5.2	Test Requirements	773

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	773

A.4.6.2.6.1	Test Purpose and Environment	773

A.4.6.2.6.2	Test Requirements	778

A.4.6.2.7	Void	779

A.4.6.2.8	Void	779

A.4.6.3	Void	779

A.4.6.4	L1-RSRP measurement for beam reporting	779

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	779

A.4.6.4.1.1	Test Purpose and Environment	779

A.4.6.4.1.2	Test parameters	779

A.4.6.4.1.3	Test Requirements	782

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	782

A.4.6.4.2.1	Test Purpose and Environment	782

A.4.6.4.2.2	Test parameters	783

A.4.6.4.2.3	Test Requirements	786

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	786

A.4.6.4.3.1	Test Purpose and Environment	786

A.4.6.4.3.2	Test parameters	787

A.4.6.4.3.3	Test Requirements	790

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	790

A.4.6.4.4.1	Test Purpose and Environment	790

A.4.6.4.4.2	Test parameters	791

A.4.6.4.4.3	Test Requirements	793

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	793

A.4.6.4.5.1	Test Purpose and Environment	793

A.4.6.4.5.2	Test parameters	794

A.4.6.4.2.3	Test Requirements	797

A.4.6.5	CLI measurements	797

A.4.6.5.1	SRS-RSRP measurement with non-DRX	797

A.4.6.5.1.1	Test Purpose and Environment	797

A.4.6.5.1.2	Test Parameters	798

A.4.6.5.1.3	Test Requirements	801

A.4.6.5.2	CLI-RSSI measurement with non-DRX	801

A.4.6.5.2.1	Test Purpose and Environment	801

A.4.6.5.2.2	Test Parameters	802

A.4.6.5.2.3	Test Requirements	803

A.4.6.6.1.2	Test Requirements	808

A.4.6.7	L1-SINR measurement for beam reporting	808

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	811

A.4.6.7.2.1	Test Purpose and Environment	811

A.4.6.7.2.2	Test parameters	811

A.4.6.7.2.3	Test Requirements	814

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	814

A.4.6.7.3.1	Test Purpose and Environment	814

A.4.6.7.3.2	Test parameters	814

A.4.6.7.3.3	Test Requirements	816

A.4.6.8	CSI-RS based intra-frequency Measurement	817

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	817

A.4.6.8.1.1	Test purpose and Environment	817

A.4.6.8.1.2	Test Requirements	821

A.4.6.9	CSI-RS based inter-frequency Measurement	821

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	821

A.4.6.9.1.1	Test Purpose and Environment	821

A.4.6.9.1.2	Test Requirements	825

A.4.7	Measurement Performance requirements	826

A.4.7.1	SS-RSRP	826

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	826

A.4.7.1.1.1	Test Purpose and Environment	826

A.4.7.1.1.2	Test parameters	826

A.4.7.1.1.3	Test Requirements	830

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	830

A.4.7.1.2.1	Test Purpose and Environment	830

A.4.7.1.2.2	Test parameters	831

A.4.7.1.2.3	Test Requirements	835

A.4.7.1.3	Void	835

A.4.7.2	SS-RSRQ	835

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	835

A.4.7.2.1.1	Test Purpose and Environment	835

A.4.7.2.1.2	Test Parameters	836

A.4.7.2.1.3	Test Requirements	841

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	841

A.4.7.2.2.1	Test Purpose and Environment	841

A.4.7.2.2.2	Test Parameters	841

A.4.7.2.2.3	Test Requirements	846

A.4.7.3	SS-SINR	846

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	846

A.4.7.3.1.1	Test Purpose and Environment	846

A.4.7.3.1.2	Test Parameters	846

A.4.7.3.1.3	Test Requirements	850

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	850

A.4.7.3.2.1	Test Purpose and Environment	850

A.4.7.3.2.2	Test Parameters	850

A.4.7.3.2.3	Test Requirements	855

A.4.7.4	L1-RSRP measurement for beam reporting	855

A.4.7.4.1	SSB based L1-RSRP measurement	855

A.4.7.4.1.1	Test Purpose and Environment	855

A.4.7.4.1.2	Test parameters	856

A.4.7.4.1.3	Test Requirements	859

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	859

A.4.7.4.2.1	Test Purpose and Environment	859

A.4.7.4.2.2	Test parameters	860

A.4.7.4.2.3	Test Requirements	863

A.4.7.5	SFTD accuracy	863

A.4.7.5.1	SFTD accuracy	863

A.4.7.5.1.1	Test Purpose and Environment	863

A.4.7.5.1.2	Test Parameters	863

A.4.7.5.1.3	Test Requirements	868

A.4.7.5.2	Void	868

A.4.7.5.3	Void	868

A.4.7.6	CLI measurements	868

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	868

A.4.7.6.1.1	Test Purpose and Environment	868

A.4.7.6.1.2	Test parameters	869

A.4.7.6.1.3	Test Requirements	874

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	874

A.4.7.6.2.1	Test Purpose and Environment	874

A.4.7.6.2.2	Test parameters	875

A.4.7.6.2.3	Test Requirements	878

A.4.7.7	L1-SINR measurement for beam reporting	878

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	882

A.4.7.7.2.1	Test Purpose and Environment	882

A.4.7.7.2.2	Test parameters	882

A.4.7.7.2.3	Test Requirements	886

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	886

A.4.7.7.3.1	Test Purpose and Environment	886

A.4.7.7.3.2	Test parameters	886

A.4.7.7.3.3	Test Requirements	889

A.4.7.8	CSI-RSRP	889

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	889

A.4.7.8.1.1	Test Purpose and Environment	889

A.4.7.8.1.2	Test parameters	890

A.4.7.8.1.3	Test Requirements	895

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	895

A.4.7.8.2.1	Test Purpose and Environment	895

A.4.7.8.2.2	Test parameters	895

A.4.7.8.2.3	Test Requirements	900

A.4.7.9	CSI-RSRQ	900

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	900

A.4.7.9.1.1	Test Purpose and Environment	900

A.4.7.9.1.2	Test Parameters	900

A.4.7.9.1.3	Test Requirements	905

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	905

A.4.7.9.2.1	Test Purpose and Environment	905

A.4.7.9.2.2	Test Parameters	905

A.4.7.9.2.3	Test Requirements	910

A.4.7.10	CSI-SINR	910

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	910

A.4.7.10.1.1	Test Purpose and Environment	910

A.4.7.10.1.2	Test Parameters	911

A.4.7.10.1.3	Test Requirements	915

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	915

A.4.7.10.2.1	Test Purpose and Environment	915

A.4.7.10.2.2	Test Parameters	916

A.4.7.10.2.3	Test Requirements	920

A.4.8	Void	920

A.5	EN-DC tests with one or more NR cells in FR2	921

A.5.1	Void	921

A.5.2	Void	921

A.5.3	RRC_CONNECTED state mobility	921

A.5.3.1	Void	921

A.5.3.2	RRC Connection Mobility Control	921

A.5.3.2.1	Void	921

A.5.3.2.2	Random Access	921

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	921

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	925

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	929

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	933

A.5.3.2.3	Void	937

A.5.4	Timing	937

A.5.4.1	UE transmit timing	937

A.5.4.1.1	NR UE Transmit Timing Test for FR2	937

A.5.4.1.1.1	Test Purpose and environment	937

A.5.4.1.1.2	Test requirements	940

A.5.4.2	UE timer accuracy	941

A.5.4.3	Timing advance	941

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	941

A.5.4.3.1.1 Test Purpose and Environment	941

A.5.4.3.1.2 Test Parameters	941

A.5.4.3.1.3	Test Requirements	944

A.5.5	Signaling characteristics	944

A.5.5.1	Radio link Monitoring	944

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	944

A.5.5.1.1.1	Test Purpose and Environment	944

A.5.5.1.1.2	Test Requirements	947

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	948

A.5.5.1.2.1	Test Purpose and Environment	948

A.5.5.1.2.2	Test Requirements	951

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	952

A.5.5.1.3.1	Test Purpose and Environment	952

A.5.5.1.3.2	Test Requirements	955

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	955

A.5.5.1.4.1	Test Purpose and Environment	955

A.5.5.1.4.2	Test Requirements	960

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	960

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	966

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	971

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	977

A.5.5.1.8.2	Test Requirements	983

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	984

A.5.5.1.9.1	Test Purpose and Environment	984

A.5.5.1.9.2	Test Requirements	986

A.5.5.2	Interruption	986

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	986

A.5.5.2.1.1	Test Purpose and Environment	986

A.5.5.2.1.2	Test Requirements	989

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	989

A.5.5.2.2.1	Test Purpose and Environment	989

A.5.5.2.2.2	Test Requirements	992

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	992

A.5.5.2.3.1	Test Purpose and Environment	992

A.5.5.2.3.2	Test Requirements	996

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	996

A.5.5.2.4.1	Test Purpose and Environment	996

A.5.5.2.4.2	Test Requirements	999

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1000

A.5.5.2.5.1	Test Purpose and Environment	1000

A.5.5.2.5.2	Test Requirements	1003

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1004

A.5.5.2.6.1	Test Purpose and Environment	1004

A.5.5.2.6.2	Test Requirements	1006

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1007

A.5.5.2.7.1	Test Purpose and Environment	1007

A.5.5.2.7.2	Test Requirements	1009

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1010

A.5.5.2.8.1 Test Purpose and Environment	1010

A.5.5.2.8.3	Test Requirements	1012

A.5.5.3	SCell Activation and Deactivation Delay	1012

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1012

A.5.5.3.1.1	Test Purpose and Environment	1012

A.5.5.3.1.2	Test Requirements	1014

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1014

A.5.5.3.2.1	Test Purpose and Environment	1014

A.5.5.3.2.2	Test Requirements	1017

A.5.5.3.3	Void	1017

A.5.5.3.4	Void	1017

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1017

A.5.5.3.5.1	Test Purpose and Environment	1017

A.5.5.3.5.2	Test Requirements	1021

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1022

A.5.5.3.6.1	Test Purpose and Environment	1022

A.5.5.3.6.2	Test Requirements	1025

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1026

A.5.5.3.7.1	Test Purpose and Environment	1026

A.5.5.3.7.2	Test Requirements	1029

A.5.5.4	Void	1029

A.5.5.5	Beam Failure Detection and Link recovery procedures	1029

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1029

A.5.5.5.1.1	Test Purpose and Environment	1029

A.5.5.5.1.2	Test Requirements	1035

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1035

A.5.5.5.2.1	Test Purpose and Environment	1035

A.5.5.5.2.2	Test Requirements	1041

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1041

A.5.5.5.3.1	Test Purpose and Environment	1041

A.5.5.5.3.2	Test Requirements	1047

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1047

A.5.5.5.4.1	Test Purpose and Environment	1047

A.5.5.5.4.2	Test Requirements	1053

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1053

A.5.5.5.5.1	Test Purpose and Environment	1053

A.5.5.5.5.2	Test Requirements	1059

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1059

A.5.5.5.6.1	Test Purpose and Environment	1059

A.5.5.5.6.2	Test Requirements	1064

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1064

A.5.5.5.7.1	Test Purpose and Environment	1064

A.5.5.5.7.2	Test Requirements	1070

A.5.5.6	Active BWP switch	1070

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1070

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1070

A.5.5.6.1.1.1	Test Purpose and Environment	1070

A.5.5.6.1.1.2	Test Requirements	1074

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC	1075

A.5.5.6.2	RRC-based Active BWP Switch	1079

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1083

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1083

A.5.5.6.3.1.1	Test Purpose and Environment	1083

A.5.5.6.3.1.2	Test Requirements	1087

A.5.5.6.4	SCell dormancy switch	1088

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1088

A.5.5.6.4.1.1	Test Purpose and Environment	1088

A.5.5.6.4.1.2	Test Requirements	1092

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1092

A.5.5.6.4.2.1	Test Purpose and Environment	1092

A.5.5.6.4.2.2	Test Requirements	1099

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1099

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1099

A.5.5.7	PSCell addition and release delay	1103

A.5.5.7.1	Addition and Release Delay of NR PSCell	1103

A.5.5.7.1.1	Test purpose and environment	1103

A.5.5.7.1.2	Test Requirements	1106

A.5.5.8	Active TCI state switch delay	1107

A.5.5.8.1	MAC-CE based active TCI state switch	1107

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1107

A.5.5.8.1.1.1	Test Purpose and Environment	1107

A.5.5.8.1.1.2	Test Requirements	1109

A.5.5.8.2	RRC based active TCI state switch	1109

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1109

A.5.5.8.2.1.1	Test Purpose and Environment	1109

A.5.5.8.2.1.2	Test Requirements	1112

A.5.5.9	Uplink spatial relation switch delay	1112

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1112

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1112

A.5.5.9.1.1.1	Test Purpose and Environment	1112

A.5.5.9.1.1.2	Test Requirements	1115

A.5.5.9.2	RRC based spatial relation switch	1115

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1115

A.5.5.9.2.1.1	Test Purpose and Environment	1115

A.5.5.9.2.1.2	Test Requirements	1118

A.5.5.10	UE specific CBW change	1118

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1118

A.5.5.10.1.1	Test Purpose and Environment	1118

A.5.5.10.1.2	Test Requirements	1121

A.5.6	Measurement procedure	1122

A.5.6.1	Intra-frequency Measurements	1122

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1122

A.5.6.1.1.1	Test purpose and Environment	1122

A.5.6.1.1.2	Test Requirements	1126

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1126

A.5.6.1.2.1	Test purpose and Environment	1126

A.5.6.1.2.2	Test Requirements	1128

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1129

A.5.6.1.3.1	Test purpose and Environment	1129

A.5.6.1.3.2	Test Requirements	1133

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1133

A.5.6.1.4.1	Test purpose and Environment	1133

A.5.6.1.4.2	Test Requirements	1136

A.5.6.2	Inter-frequency Measurements	1137

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1137

A.5.6.2.1.1	Test Purpose and Environment	1137

A.5.6.2.1.2	Test Requirements	1140

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1140

A.5.6.2.2.1	Test Purpose and Environment	1140

A.5.6.2.2.2	Test Requirements	1144

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1144

A.5.6.2.3.1	Test Purpose and Environment	1144

A.5.6.2.3.2	Test Requirements	1148

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1148

A.5.6.2.4.1	Test Purpose and Environment	1148

A.5.6.2.4.2	Test Requirements	1151

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1151

A.5.6.2.5.1	Test Purpose and Environment	1151

A.5.6.2.5.2	Test Requirements	1156

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1156

A.5.6.2.6.1	Test Purpose and Environment	1156

A.5.6.2.6.2	Test Requirements	1160

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1160

A.5.6.2.7.1	Test Purpose and Environment	1160

A.5.6.2.7.2	Test Requirements	1165

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1165

A.5.6.2.8.1	Test Purpose and Environment	1165

A.5.6.2.8.2	Test Requirements	1170

A.5.6.3	L1-RSRP measurement for beam reporting	1171

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1171

A.5.6.3.1.1	Test Purpose and Environment	1171

A.5.6.3.1.2	Test parameters	1171

A.5.6.3.1.3	Test Requirements	1173

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1173

A.5.6.3.2.1	Test Purpose and Environment	1173

A.5.6.3.2.2	Test parameters	1174

A.5.6.3.2.3	Test Requirements	1176

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1177

A.5.6.3.3.1	Test Purpose and Environment	1177

A.5.6.3.3.2	Test parameters	1177

A.5.6.3.3.3	Test Requirements	1179

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1180

A.5.6.3.4.1	Test Purpose and Environment	1180

A.5.6.3.4.2	Test parameters	1180

A.5.6.3.4.3	Test Requirements	1182

A.5.6.4	CLI measurements	1183

A.5.6.4.1	SRS-RSRP measurement with DRX	1183

A.5.6.4.1.1	Test Purpose and Environment	1183

A.5.6.4.1.2	Test Parameters	1183

A.5.6.4.1.3	Test Requirements	1185

A.5.6.4.2	CLI-RSSI measurement with DRX	1185

A.5.6.4.2.1	Test Purpose and Environment	1185

A.5.6.4.2.2	Test Parameters	1186

A.5.6.4.2.3	Test Requirements	1188

A.5.6.4	Measurements with autonomous gaps	1188

A.5.6.4.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1188

A.5.6.4.1.1	Test Purpose and Environment	1188

A.5.6.4.1.2	Test Requirements	1191

A.5.6.5	L1-SINR measurement for beam reporting	1192

A.5.6.5.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1194

A.5.6.5.2.1	Test Purpose and Environment	1194

A.5.6.5.2.2	Test parameters	1195

A.5.6.5.2.3	Test Requirements	1198

A.5.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1198

A.5.6.5.3.1	Test Purpose and Environment	1198

A.5.6.5.3.2	Test parameters	1198

A.5.6.5.3.3	Test Requirements	1200

A.5.6.6	CSI-RS based Intra-frequency Measurements	1200

A.5.6.6.1	EN-DC event triggered reporting test without gap under non-DRX	1200

A.5.6.6.1.1	Test purpose and Environment	1200

A.5.6.6.1.2	Test Requirements	1203

A.5.6.7	CSI-RS based Inter-frequency Measurements	1203

A.5.6.7.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1203

A.5.6.7.1.1	Test Purpose and Environment	1203

A.5.6.7.1.2	Test Requirements	1208

A.5.7	Measurement Performance requirements	1208

A.5.7.1	SS-RSRP	1208

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1208

A.5.7.1.1.1	Test Purpose and Environment	1208

A.5.7.1.1.2	Test parameters	1209

A.5.7.1.1.3	Test Requirements	1211

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1212

A.5.7.1.2.1	Test Purpose and Environment	1212

A.5.7.1.2.2	Test parameters	1212

A.5.7.1.2.3	Test Requirements	1216

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1217

A.5.7.1.3.1	Test Purpose and Environment	1217

A.5.7.1.3.2	Test parameters	1217

A.5.7.1.3.3	Test Requirements	1220

A.5.7.2	SS-RSRQ	1221

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1221

A.5.7.2.1.1	Test Purpose and Environment	1221

A.5.7.2.1.2	Test Parameters	1221

A.5.7.2.1.3	Test Requirements	1225

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1225

A.5.7.2.2.1	Test Purpose and Environment	1225

A.5.7.2.2.2	Test Parameters	1225

A.5.7.2.2.3	Test Requirements	1228

A.5.7.3	SS-SINR	1228

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1228

A.5.7.3.1.1	Test Purpose and Environment	1228

A.5.7.3.1.2	Test Parameters	1228

A.5.7.3.1.3	Test Requirements	1231

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1231

A.5.7.3.2.1	Test Purpose and Environment	1231

A.5.7.3.2.2	Test Parameters	1231

A.5.7.3.2.3	Test Requirements	1233

A.5.7.4	L1-RSRP measurement for beam reporting	1234

A.5.7.4.1	SSB based L1-RSRP measurement	1234

A.5.7.4.1.1	Test Purpose and Environment	1234

A.5.7.4.1.2	Test parameters	1234

A.5.7.4.1.3	Test Requirements	1236

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1237

A.5.7.4.2.1	Test Purpose and Environment	1237

A.5.7.4.2.2	Test parameters	1237

A.5.7.4.2.3	Test Requirements	1239

A.5.7.5	CLI measurements	1240

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1240

A.5.7.5.1.1	Test Purpose and Environment	1240

A.5.7.5.1.2	Test parameters	1240

A.5.7.5.1.3	Test Requirements	1243

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1244

A.5.7.5.2.1	Test Purpose and Environment	1244

A.5.7.5.2.2	Test parameters	1244

A.5.7.5.2.3	Test Requirements	1246

A.5.7.6	L1-SINR measurement for beam reporting	1247

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1249

A.5.7.6.2.1	Test Purpose and Environment	1249

A.5.7.6.2.2	Test parameters	1250

A.5.7.6.2.3	Test Requirements	1252

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1253

A.5.7.6.3.1	Test Purpose and Environment	1253

A.5.7.6.3.2	Test parameters	1253

A.5.7.6.3.3	Test Requirements	1255

A.5.7.7	CSI-RSRP	1255

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1255

A.5.7.7.1.1	Test Purpose and Environment	1255

A.5.7.7.1.2	Test parameters	1256

A.5.7.7.1.3	Test Requirements	1258

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1259

A.5.7.7.2.1	Test Purpose and Environment	1259

A.5.7.7.2.2	Test parameters	1259

A.5.7.7.2.3	Test Requirements	1263

A.5.7.8	CSI-RSRQ	1264

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1264

A.5.7.8.1.1	Test Purpose and Environment	1264

A.5.7.8.1.2	Test Parameters	1264

A.5.7.8.1.3	Test Requirements	1266

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1266

A.5.7.8.2.1	Test Purpose and Environment	1266

A.5.7.8.2.2	Test Parameters	1266

A.5.7.8.2.3	Test Requirements	1268

A.5.7.9	CSI-SINR	1268

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1268

A.5.7.9.1.1	Test Purpose and Environment	1268

A.5.7.9.1.2	Test Parameters	1268

A.5.7.9.1.3	Test Requirements	1271

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1271

A.5.7.9.2.1	Test Purpose and Environment	1271

A.5.7.9.2.2	Test Parameters	1271

A.5.7.9.2.3	Test Requirements	1274

A.5.8	Void	1274

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1027

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1027

A.6.1.1.5.1	Test Purpose and Environment	1027

A.6.1.1.5.2	Test Parameters	1027

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1032

A.6.1.1.6.1	Test Purpose and Environment	1032

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1041

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1041

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1041

A.6.1.2.1.1	Test Purpose and Environment	1041

A.6.1.2.1.2	Test Parameters	1041

A.6.1.2.1.3	Test Requirements	1044

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1045

A.6.1.2.2.1	Test Purpose and Environment	1045

A.6.1.2.2.2	Test Parameters	1045

A.6.1.2.2.3	Test Requirements	1048

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1049

A.6.1.2.3.1	Test Purpose and Environment	1049

A.6.1.2.3.2	Test Parameters	1049

A.6.1.2.3.3	Test Requirements	1052

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1053

A.6.1.2.4.1	Test Purpose and Environment	1053

A.6.1.2.4.2	Test Parameters	1053

A.6.1.2.4.3	Test Requirements	1057

A.6.1.1.7	Void	1061

A.6.2	SA: RRC_INACTIVE state mobility	1061

A.6.3	RRC_CONNECTED state mobility	1061

A.6.3.1	Handover	1061

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1061

A.6.3.1.1.1	Test Purpose and Environment	1061

A.6.3.1.1.2	Test Parameters	1061

A.6.3.1.1.3 Test Requirements	1064

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1064

A.6.3.1.2.1	Test Purpose and Environment	1064

A.6.3.1.2.2	Test Parameters	1064

A.6.3.1.2.3	Test Requirements	1068

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1068

A.6.3.1.3.1	Test Purpose and Environment	1068

A.6.3.1.3.2	Test Parameters	1068

A.6.3.1.3.3 Test Requirements	1072

A.6.3.1.4	 SA NR - E-UTRAN handover	1072

A.6.3.1.4.1	Test Purpose and Environment	1072

A.6.3.1.4.2	Test Requirements	1078

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1078

A.6.3.1.5.1	Test Purpose and Environment	1078

A.6.3.1.5.2	Test Requirements	1084

A.6.3.1.6	 SA NR - UTRAN FDD handover	1084

A.6.3.1.6.1	Test Purpose and Environment	1084

A.6.3.1.6.2	Test Requirements	1088

A.6.3.1.7	Intra-frequency handover in FR1; synchronous scenario	1088

A.6.3.1.7.1	Test Purpose and Environment	1088

A.6.3.1.7.2	Test Parameters	1088

A.6.3.1.7.3 Test Requirements	1092

A.6.3.1.8	Intra-frequency handover in FR1; asynchronous scenario	1093

A.6.3.1.8.1	Test Purpose and Environment	1093

A.6.3.1.8.2	Test Parameters	1093

A.6.3.1.8.3 Test Requirements	1097

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1098

A.6.3.1.9.1	Test Purpose and Environment	1098

A.6.3.1.9.2	Test Parameters	1098

A.6.3.1.9.3 Test Requirements	1102

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1102

A.6.3.1.10.1	Test Purpose and Environment	1102

A.6.3.1.10.2	Test Parameters	1102

A.6.3.1.10.3 Test Requirements	1106

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1106

A.6.3.1.11.1	Test Purpose and Environment	1106

A.6.3.1.11.2	Test Parameters	1106

A.6.3.1.11.3 Test Requirements	1113

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1113

A.6.3.1.12.1	Test Purpose and Environment	1113

A.6.3.1.12.2	Test Parameters	1113

A.6.3.1.12.3 Test Requirements	1120

A.6.3.2	RRC Connection Mobility Control	1120

A.6.3.2.1	SA: RRC Re-establishment	1120

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1120

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1124

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1128

A.6.3.2.2	Random Access	1131

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1131

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1135

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1140

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1145

A.6.3.2.3	SA: RRC Connection Release with Redirection	1149

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1149

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1153

A.6.3.3 Conditional handover	1160

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1160

A.6.3.3.1.1	Test Purpose and Environment	1160

A.6.3.3.1.2	Test Parameters	1160

A.6.3.3.1.3 Test Requirements	1164

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1164

A.6.3.3.2.1	Test Purpose and Environment	1164

A.6.3.3.2.2	Test Parameters	1164

A.6.3.3.2.3 Test Requirements	1168

A.6.4	Timing	1168

A.6.4.1	UE transmit timing	1168

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1168

A.6.4.1.1.1	Test Purpose and environment	1168

A.6.4.1.1.2	Test requirements	1172

A.6.4.2	UE timer accuracy	1172

A.6.4.3	Timing advance	1172

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1172

A.6.4.3.1.1	Test Purpose and Environment	1172

A.6.4.3.1.2	Test Parameters	1172

A.6.4.3.1.3	Test Requirements	1176

A.6.5	Signalling characteristics	1176

A.6.5.1	Radio link Monitoring	1176

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1177

A.6.5.1.1.1	Test Purpose and Environment	1177

A.6.5.1.1.2	Test Requirements	1182

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1182

A.6.5.1.2.1	Test Purpose and Environment	1182

A.6.5.1.2.2	Test Requirements	1188

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1188

A.6.5.1.3.1	Test Purpose and Environment	1188

A.6.5.1.3.2	Test Requirements	1194

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1194

A.6.5.1.4.1	Test Purpose and Environment	1194

A.6.5.1.4.2	Test Requirements	1200

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1200

A.6.5.1.5.1	Test Purpose and Environment	1200

A.6.5.1.5.2	Test Requirements	1206

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1206

A.6.5.1.6.1	Test Purpose and Environment	1206

A.6.5.1.6.2	Test Requirements	1212

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1212

A.6.5.1.7.1	Test Purpose and Environment	1212

A.6.5.1.7.2	Test Requirements	1217

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1217

A.6.5.1.8.1	Test Purpose and Environment	1217

A.6.5.1.8.2	Test Requirements	1223

A.6.5.2	Interruption	1223

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1223

A.6.5.2.1.2	Test Requirements	1227

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1228

A.6.5.2.2.1	Test Purpose and Environment	1228

A.6.5.2.2.2	Test Parameters	1228

A.6.5.2.2.3	Test Requirements	1230

A.6.5.3	SCell Activation and Deactivation Delay	1230

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1230

A.6.5.3.1.1	Test Purpose and Environment	1230

A.6.5.3.1.2	Test Requirements	1234

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	1235

A.6.5.3.2.1	Test Purpose and Environment	1235

A.6.5.3.2.2	Test Requirements	1235

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1235

A.6.5.3.3.1	Test Purpose and Environment	1235

A.6.5.3.3.2	Test Requirements	1236

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1236

A.6.5.3.4.1	Test Purpose and Environment	1236

A.6.5.3.4.2	Test Requirements	1240

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1240

A.6.5.3.5.1	Test Purpose and Environment	1240

A.6.5.3.5.2	Test Requirements	1244

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1245

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1245

A.6.5.4.1.1	Test Purpose and Environment	1245

A.6.5.4.1.2	Test Requirements	1254

A.6.5.4.2	Void	1255

A.6.5.5	Beam Failure Detection and Link recovery procedures	1255

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1255

A.6.5.5.1.1	Test Purpose and Environment	1255

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1260

A.6.5.5.2.1	Test Purpose and Environment	1260

A.6.5.5.2.2	Test Requirements	1267

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1267

A.6.5.5.3.1	Test Purpose and Environment	1267

A.6.5.5.3.2	Test Requirements	1273

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1273

A.6.5.5.4.1	Test Purpose and Environment	1273

A.6.5.5.4.2	Test Requirements	1279

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1279

A.6.5.5.5.1	Test Purpose and Environment	1279

A.6.5.5.5.2	Test Requirements	1283

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1284

A.6.5.5.6.1	Test Purpose and Environment	1284

A.6.5.5.6.2	Test Requirements	1288

A.6.5.6	Active BWP switch	1288

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1288

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	1288

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1293

A.6.5.6.2	RRC-based Active BWP Switch	1298

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1298

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1302

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1302

A.6.5.6.3.1.1	Test Purpose and Environment	1302

A.6.5.6.3.1.2	Test Requirements	1307

A.6.5.6.4	SCell dormancy switch	1308

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1308

A.6.5.6.4.1.1	Test Purpose and Environment	1308

A.6.5.6.4.1.2	Test Requirements	1313

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1314

A.6.5.6.4.2.1	 Test Purpose and Environment	1314

A.6.5.6.4.2.2	 Test Requirements	1318

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1318

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1318

A.6.5.7	DL interruptions at switching between two uplink carriers	1322

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1322

A.6.5.7.1.1	Test Purpose and Environment	1322

A.6.5.7.1.2	Test Requirements	1326

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1326

A.6.5.7.2.1	Test Purpose and Environment	1326

A.6.5.7.2.2	Test Requirements	1330

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1330

A.6.5.8.1.1	Test Purpose and Environment	1330

A.6.5.8.1.2	Test Requirements	1334

A.6.6	Measurement procedure	1334

A.6.6.1	Intra-frequency Measurements	1334

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1334

A.6.6.1.1.1	Test purpose and Environment	1334

A.6.6.1.1.2	Test parameters	1334

A.6.6.1.1.3	Test Requirements	1338

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1338

A.6.6.1.2.1	Test purpose and Environment	1338

A.6.6.1.2.2	Test parameters	1338

A.6.6.1.2.3	Test Requirements	1342

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1342

A.6.6.1.3.1	Test purpose and Environment	1342

A.6.6.1.3.2	Test parameters	1342

A.6.6.1.3.3	Test Requirements	1346

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1346

A.6.6.1.4.1	Test purpose and Environment	1346

A.6.6.1.4.2	Test parameters	1346

A.6.6.1.4.3	Test Requirements	1350

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1350

A.6.6.1.5.1	Test purpose and Environment	1350

A.6.6.1.5.2	Test parameters	1350

A.6.6.1.5.3	Test Requirements	1352

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1353

A.6.6.1.6.1	Test purpose and Environment	1353

A.6.6.1.6.2	Test parameters	1353

A.6.6.1.6.3	Test Requirements	1354

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1355

A.6.6.1.7.1	Test purpose and Environment	1355

A.6.6.1.7.2	Test parameters	1355

A.6.6.1.7.3	Test Requirements	1359

A.6.6.2	Inter-frequency Measurements	1359

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1359

A.6.6.2.1.1	Test Purpose and Environment	1359

A.6.6.2.1.2	Test Requirements	1363

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1363

A.6.6.2.2.1	Test Purpose and Environment	1363

A.6.6.2.2.2	Test Requirements	1367

A.6.6.2.3	Void	1368

A.6.6.2.4	Void	1368

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1368

A.6.6.2.5.1	Test Purpose and Environment	1368

A.6.6.2.5.2	Test Requirements	1372

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1372

A.6.6.2.6.1	Test Purpose and Environment	1372

A.6.6.2.6.2	Test Requirements	1376

A.6.6.2.7	Void	1377

A.6.6.2.8	Void	1377

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1377

A.6.6.2.9.1	Test Purpose and Environment	1377

A.6.6.2.9.2	Test Requirements	1380

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1380

A.6.6.2.10.1	Test Purpose and Environment	1380

A.6.6.2.10.2	Test Requirements	1384

A.6.6.3	Inter-RAT Measurements	1388

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1388

A.6.6.3.1.1	Test Purpose and Environment	1388

A.6.6.3.1.2	Test Requirements	1394

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1394

A.6.6.3.2.1	Test Purpose and Environment	1394

A.6.6.3.2.2	Test Requirements	1401

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1401

A.6.6.3.3.1	Test Purpose and Environment	1401

A.6.6.3.3.2	Test Requirements	1405

A.6.6.4	L1-RSRP measurement for beam reporting	1405

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1405

A.6.6.4.1.1	Test Purpose and Environment	1405

A.6.6.4.1.2	Test parameters	1406

A.6.6.4.1.3	Test Requirements	1409

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1409

A.6.6.4.2.1	Test Purpose and Environment	1409

A.6.6.4.2.2	Test parameters	1410

A.6.6.4.2.3	Test Requirements	1413

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1413

A.6.6.4.3.1	Test Purpose and Environment	1413

A.6.6.4.3.2	Test parameters	1414

A.6.6.4.3.3	Test Requirements	1417

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1417

A.6.6.4.4.1	Test Purpose and Environment	1417

A.6.6.4.4.2	Test parameters	1418

A.6.6.4.4.3	Test Requirements	1421

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1421

A.6.6.4.5.1	Test Purpose and Environment	1421

A.6.6.4.5.2	Test parameters	1422

A.6.6.4.5.3	Test Requirements	1424

A.6.6.5	1424

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1424

A.6.6.5.1.1	Test Purpose and Environment	1424

A.6.6.5.1.2	Test Requirements	1428

A.6.6.6	CLI measurements	1428

A.6.6.6.1	SRS-RSRP measurement with DRX	1428

A.6.6.6.1.1	Test Purpose and Environment	1428

A.6.6.6.1.2	Test Parameters	1429

A.6.6.6.1.3	Test Requirements	1432

A.6.6.6.2	CLI-RSSI measurement with DRX	1432

A.6.6.6.2.1	Test Purpose and Environment	1432

A.6.6.6.2.2	Test Parameters	1433

A.6.6.6.2.3	Test Requirements	1435

A.6.6.7	NR measurements with autonomous gaps	1435

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1435

A.6.6.7.1.1	Test Purpose and Environment	1435

A.6.6.7.1.2	Test Parameters	1435

A.6.6.7.1.3	Test Requirements	1438

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1438

A.6.6.7.2.1	Test Purpose and Environment	1438

A.6.6.7.2.2	Test Requirements	1441

A.6.6.8	L1-SINR measurement for beam reporting	1442

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1444

A.6.6.8.2.1	Test Purpose and Environment	1444

A.6.6.8.2.2	Test parameters	1444

A.6.6.8.2.3	Test Requirements	1449

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1449

A.6.6.8.3.1	Test Purpose and Environment	1449

A.6.6.8.3.2	Test parameters	1450

A.6.6.8.3.3	Test Requirements	1452

A.6.6.9	Idle Mode CA/DC Measurements	1452

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1452

A.6.6.9.1.1	Test Purpose and Environment	1452

A.6.6.9.1.2	Test Requirements	1459

A.6.6.10	CSI-RS based intra-frequency Measurements	1459

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1459

A.6.6.10.1.1	Test purpose and Environment	1459

A.6.6.10.1.2	Test Requirements	1462

A.6.6.11	CSI-RS based inter-frequency Measurements	1462

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1462

A.6.6.11.1.1	Test Purpose and Environment	1462

A.6.6.11.1.2	Test Requirements	1466

A.6.6.12	RSTD measurements	1466

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1466

A.6.6.12.1.1	Test Purpose and Environment	1466

A.6.6.12.1.2	Test Requirements	1472

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1473

A.6.6.12.2.1	Test Purpose and Environment	1473

A.6.6.12.2.2	Test Requirements	1478

A.6.6.13 PRS-RSRP measurements	1479

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1479

A.6.6.13.1.1	Test purpose and Environment	1479

A.6.6.13.1.2	Test Requirements	1482

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1482

A.6.6.13.2.1	Test purpose and Environment	1482

A.6.6.13.2.2	Test Requirements	1485

A.6.6.14	UE Rx-Tx time difference measurements	1485

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1485

A.6.6.14.1.1	Test purpose and environment	1485

A.6.6.14.1.2	Test requirements	1488

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1488

A.6.6.14.2.1	Test purpose and environment	1488

A.6.6.14.2.2	Test requirements	1491

A.6.7	Measurement Performance requirements	1491

A.6.7.1	SS-RSRP	1491

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1491

A.6.7.1.1.1	Test Purpose and Environment	1491

A.6.7.1.1.2	Test parameters	1492

A.6.7.1.1.3	Test Requirements	1497

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1497

A.6.7.1.2.1	Test Purpose and Environment	1497

A.6.7.1.2.2	Test parameters	1497

A.6.7.1.2.3	Test Requirements	1501

A.6.7.1.3	Void	1501

A.6.7.2	SS-RSRQ	1501

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1501

A.6.7.2.1.1	Test Purpose and Environment	1501

A.6.7.2.1.2	Test Parameters	1502

A.6.7.2.1.3	Test Requirements	1507

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1507

A.6.7.2.2.1	Test Purpose and Environment	1507

A.6.7.2.2.2	Test Parameters	1507

A.6.7.2.2.3	Test Requirements	1512

A.6.7.3	SS-SINR	1512

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1512

A.6.7.3.1.1	Test Purpose and Environment	1512

A.6.7.3.1.2	Test Parameters	1513

A.6.7.3.1.3	Test Requirements	1517

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1517

A.6.7.3.2.1	Test Purpose and Environment	1517

A.6.7.3.2.2	Test Parameters	1518

A.6.7.3.2.3	Test Requirements	1522

A.6.7.4	L1-RSRP measurement for beam reporting	1523

A.6.7.4.1	SSB based L1-RSRP measurement	1523

A.6.7.4.1.1	Test Purpose and Environment	1523

A.6.7.4.1.2	Test parameters	1523

A.6.7.4.1.3	Test Requirements	1527

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1527

A.6.7.4.2.1	Test Purpose and Environment	1527

A.6.7.4.2.2	Test parameters	1528

A.6.7.4.2.3	Test Requirements	1532

A.6.7.5	E-UTRAN RSRP	1532

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1532

A.6.7.5.1.1	Test Purpose and Environment	1532

A.6.7.5.1.2	Test parameters	1533

A.6.7.5.1.3	Test Requirements	1538

A.6.7.6	E-UTRAN RSRQ	1539

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1539

A.6.7.6.1.1	Test Purpose and Environment	1539

A.6.7.6.1.2	Test parameters	1539

A.6.7.6.1.3	Test Requirements	1544

A.6.7.7	E-UTRAN RS-SINR	1545

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1545

A.6.7.7.1.1	Test Purpose and Environment	1545

A.6.7.7.1.2	Test parameters	1545

A.6.7.7.1.3	Test Requirements	1550

A.6.7.8	CLI measurements	1551

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1551

A.6.7.8.1.1	Test Purpose and Environment	1551

A.6.7.8.1.2	Test parameters	1551

A.6.7.8.1.3	Test Requirements	1557

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1557

A.6.7.8.2.1	Test Purpose and Environment	1557

A.6.7.8.2.2	Test parameters	1558

A.6.7.8.2.3	Test Requirements	1561

A.6.7.9	L1-SINR measurement for beam reporting	1562

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1566

A.6.7.9.2.1	Test Purpose and Environment	1566

A.6.7.9.2.2	Test parameters	1567

A.6.7.9.2.3	Test Requirements	1572

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1572

A.6.7.9.3.1	Test Purpose and Environment	1572

A.6.7.9.3.2	Test parameters	1572

A.6.7.9.3.3	Test Requirements	1577

A.6.7.10	CSI-RSRP	1577

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1577

A.6.7.9.10.1	Test Purpose and Environment	1577

A.6.7.9.10.2	Test parameters	1578

A.6.7.10.1.3	Test Requirements	1583

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1583

A.6.7.9.10.1	Test Purpose and Environment	1583

A.6.7.10.2.2	Test parameters	1583

A.6.7.10.2.3	Test Requirements	1588

A.6.7.11	CSI-RSRQ	1588

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1588

A.6.7.11.1.1	Test Purpose and Environment	1588

A.6.7.11.1.2	Test Parameters	1588

A.6.7.11.1.3	Test Requirements	1593

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1593

A.6.7.11.2.1	Test Purpose and Environment	1593

A.6.7.11.2.2	Test Parameters	1593

A.6.7.11.2.3	Test Requirements	1599

A.6.7.12	CSI-SINR	1600

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1600

A.6.7.12.1.1	Test Purpose and Environment	1600

A.6.7.12.1.2	Test Parameters	1600

A.6.7.12.1.3	Test Requirements	1604

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1605

A.6.7.12.2.1	Test Purpose and Environment	1605

A.6.7.12.2.2	Test Parameters	1605

A.6.7.12.2.3	Test Requirements	1611

A.6.7.13	RSTD measurements	1611

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1611

A.6.7.13.1.1	Test purpose and Environment	1611

A.6.7.13.1.2	Test Requirements	1614

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1614

A.6.7.13.2.1	Test purpose and Environment	1614

A.6.7.13.2.2	Test Requirements	1617

A.6.7.14	PRS-RSRP measurements	1617

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1617

A.6.7.14.1.1	Test Purpose and Environment	1617

A.6.7.14.1.2	Test parameters	1617

A.6.7.14.1.3	Test Requirements	1622

A.6.7.15	UE Rx-Tx time difference measurements	1622

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1622

A.6.7.15.1.1	Test purpose and environment	1622

A.6.7.15.1.2	Test parameters	1623

A.6.7.15.1.3	Test requirements	1626

A.7	NR standalone tests with one or more NR cells in FR2	1626

A.7.1	SA: RRC_IDLE state mobility	1626

A.7.1.1	Cell re-selection to NR	1626

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1626

A.7.1.1.1.1	Test Purpose and Environment	1626

A.7.1.1.1.2	Test Parameters	1626

A.7.1.1.1.3	Test Requirements	1630

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1630

A.7.1.1.2.1	Test Purpose and Environment	1630

A.7.1.1.2.2	Test Parameters	1630

A.7.1.1.2.3	Test Requirements	1634

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1634

A.7.1.1.3.1	Test Purpose and Environment	1634

A.7.1.1.3.2	Test Parameters	1634

A.7.1.1.3.3	Test Requirements	1637

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1637

A.7.1.1.4.1	Test Purpose and Environment	1637

A.7.1.1.4.2	Test Parameters	1637

A.7.1.1.4.3	Test Requirements	1640

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1640

A.7.1.1.5.1	Test Purpose and Environment	1640

A.7.1.1.5.2	Test Parameters	1640

A.7.1.1.5.3	Test Requirements	1644

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1644

A.7.1.1.6.1	Test Purpose and Environment	1644

A.7.1.1.6.2	Test Parameters	1644

A.7.1.1.6.3	Test Requirements	1646

A.7.2	SA: RRC_INACTIVE state mobility	1647

A.7.3	RRC_CONNECTED state mobility	1647

A.7.3.1	Handover	1647

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1647

A.7.3.1.1.1	Test Purpose and Environment	1647

A.7.3.1.1.2	Test Parameters	1647

A.7.3.1.1.3	Test Requirements	1651

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1651

A.7.3.1.2.1	Test Purpose and Environment	1651

A.7.3.1.2.2	Test Parameters	1651

A.7.3.1.2.3	Test Requirements	1654

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1654

A.7.3.1.3.1	Test Purpose and Environment	1654

A.7.3.1.3.2	Test Parameters	1654

A.7.3.1.3.3 Test Requirements	1655

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1656

A.7.3.1.4.1	Test Purpose and Environment	1656

A.7.3.1.4.2	Test Parameters	1656

A.7.3.1.4.3 Test Requirements	1663

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1663

A.7.3.1.5.1	Test Purpose and Environment	1663

A.7.3.1.5.2	Test Parameters	1663

A.7.3.1.5.3 Test Requirements	1670

A.7.3.2	RRC Connection Mobility Control	1670

A.7.3.2.1	SA: RRC Re-establishment	1670

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1670

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1673

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1676

A.7.3.2.1.3.1	Test Purpose and Environment	1676

A.7.3.2.1.3.2	Test Requirements	1678

A.7.3.2.2	Random Access	1679

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	1679

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	1683

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1687

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	1690

A.7.3.2.3	SA: RRC Connection Release with Redirection	1693

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1693

A.7.3.3 Conditional Handover	1697

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1697

A.7.3.3.1.1	Test Purpose and Environment	1697

A.7.3.3.1.2	Test Parameters	1697

A.7.3.3.1.2.3	Test Requirements	1700

A.7.3.3.2	Inter-frequency handover from FR2 to FR2; unknown target cell	1700

A.7.3.3.2.1	Test Purpose and Environment	1700

A.7.3.3.2.2	Test Parameters	1700

A.7.3.3.2.3 Test Requirements	1702

A.7.4	Timing	1702

A.7.4.1	UE transmit timing	1702

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1702

A.7.4.1.1.1	Test Purpose and environment	1702

A.7.4.1.1.2	Test requirements	1705

A.7.4.2	UE timer accuracy	1706

A.7.4.3	Timing advance	1706

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1706

A.7.4.3.1.1	Test Purpose and Environment	1706

A.7.4.3.1.2	Test Parameters	1706

A.7.4.3.1.3 Test Requirements	1710

A.7.5	Signaling characteristics	1710

A.7.5.1	Radio link Monitoring	1710

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1710

A.7.5.1.1.1	Test Purpose and Environment	1710

A.7.5.1.1.2	Test Requirements	1713

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1714

A.7.5.1.2.1	Test Purpose and Environment	1714

A.7.5.1.2.2	Test Requirements	1718

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1719

A.7.5.1.3.1	Test Purpose and Environment	1719

A.7.5.1.3.2	Test Requirements	1722

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1722

A.7.5.1.4.1	Test Purpose and Environment	1722

A.7.5.1.4.2	Test Requirements	1727

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1727

A.7.5.1.5.1	Test Purpose and Environment	1727

A.7.5.1.5.2	Test Requirements	1731

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1731

A.7.5.1.6.1	Test Purpose and Environment	1731

A.7.5.1.6.2	Test Requirements	1735

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1735

A.7.5.1.7.1	Test Purpose and Environment	1735

A.7.5.1.7.2	Test Requirements	1738

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1738

A.7.5.1.8.1	Test Purpose and Environment	1738

A.7.5.1.8.2	Test Requirements	1743

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1743

A.7.5.1.9.1	Test Purpose and Environment	1743

A.7.5.1.9.2	Test Requirements	1746

A.7.5.2	Interruption	1746

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1746

A.7.5.2.1.1	Test Purpose and Environment	1746

A.7.5.2.1.2	Test Requirements	1749

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1750

A.7.5.2.2.1	Test Purpose and Environment	1750

A.7.5.2.2.2	Test Parameters	1750

A.7.5.2.2.3	Test Requirements	1752

A.7.5.3	SCell Activation and Deactivation Delay	1752

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1752

A.7.5.3.1.1	Test Purpose and Environment	1752

A.7.5.3.1.2	Test Requirements	1754

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1754

A.7.5.3.2.1	Test Purpose and Environment	1754

A.7.5.3.2.2	Test Requirements	1759

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1760

A.7.5.3.3.1	Test Purpose and Environment	1760

A.7.5.3.3.2	Test Requirements	1763

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1764

A.7.5.3.4.1	Test Purpose and Environment	1764

A.7.5.3.4.2	Test Requirements	1767

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1768

A.7.5.3.5.1	Test Purpose and Environment	1768

A.7.5.3.5.2	Test Requirements	1770

A.7.5.4	Void	1771

A.7.5.5	Beam Failure Detection and Link recovery procedures	1771

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1771

A.7.5.5.1.1	Test Purpose and Environment	1771

A.7.5.5.1.2	Test Requirements	1776

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1776

A.7.5.5.2.1	Test Purpose and Environment	1776

A.7.5.5.2.2	Test Requirements	1781

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1781

A.7.5.5.3.1	Test Purpose and Environment	1781

A.7.5.5.3.2	Test Requirements	1787

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1787

A.7.5.5.4.1	Test Purpose and Environment	1787

A.7.5.5.4.2	Test Requirements	1793

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1793

A.7.5.5.5.1	Test Purpose and Environment	1793

A.7.5.5.5.2	Test Requirements	1798

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1798

A.7.5.5.6.1	Test Purpose and Environment	1798

A.7.5.5.6.2	Test Requirements	1802

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1802

A.7.5.5.7.1	Test Purpose and Environment	1802

A.7.5.5.7.2	Test Requirements	1806

A.7.5.6	Active BWP switch	1806

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1806

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1806

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1811

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1816

A.7.5.6.1.3.1	Test Purpose and Environment	1816

A.7.5.6.1.3.2	Test Requirements	1819

A.7.5.6.2	RRC-based Active BWP Switch	1819

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1823

A.7.5.6.3.1.1	Test Purpose and Environment	1823

A.7.5.6.3.1.2	Test Requirements	1826

A.7.5.6.4	SCell dormancy switch	1826

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1826

A.7.5.6.4.1.1	Test Purpose and Environment	1826

A.7.5.6.4.1.2	Test Requirements	1831

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1831

A.7.5.6.4.2.1	 Test Purpose and Environment	1831

A.7.5.6.4.2.2	 Test Requirements	1836

A.7.5.7	PSCell addition and release delay	1836

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1836

A.7.5.7.1.1	Test Purpose and Environment	1836

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1840

A.7.5.7.2.1	Test Purpose and Environment	1840

A.7.5.7.2.2	Test Requirements	1844

A.7.5.8	Active TCI state switch delay	1844

A.7.5.8.1	MAC-CE based active TCI state switch	1844

A.7.5.8.2	RRC based active TCI state switch	1846

A.7.5.9	Uplink spatial relation switch delay	1849

A.7.5.9.1.1.1	Test Purpose and Environment	1849

A.7.5.9.1.1.2	Test Requirements	1852

A.7.5.9.2	RRC based spatial relation switch	1852

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1852

A.7.5.9.2.1.1	Test Purpose and Environment	1852

A.7.5.9.2.1.2	Test Requirements	1855

A.7.5.10	UE specific CBW change	1855

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1855

A.7.5.10.1.1	Test Purpose and Environment	1855

A.7.5.10.1.2	Test Requirements	1858

A.7.6	Measurement procedure	1859

A.7.6.1	Intra-frequency Measurements	1859

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1859

A.7.6.1.1.1	Test purpose and Environment	1859

A.7.6.1.1.2	Test Requirements	1862

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1862

A.7.6.1.2.1	Test purpose and Environment	1862

A.7.6.1.2.2	Test Requirements	1865

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1866

A.7.6.1.3.1	Test purpose and Environment	1866

A.7.6.1.3.2	Test Requirements	1869

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1869

A.7.6.1.4.1	Test purpose and Environment	1869

A.7.6.1.4.2	Test Requirements	1872

A.7.6.2	Inter-frequency Measurements	1873

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1873

A.7.6.2.1.1	Test Purpose and Environment	1873

A.7.6.2.1.2	Test Requirements	1876

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1876

A.7.6.2.2.1	Test Purpose and Environment	1876

A.7.6.2.2.2	Test Requirements	1880

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1880

A.7.6.2.3.1	Test Purpose and Environment	1880

A.7.6.2.3.2	Test Requirements	1884

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1884

A.7.6.2.4.1	Test Purpose and Environment	1884

A.7.6.2.4.2	Test Requirements	1888

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1888

A.7.6.2.5.1	Test Purpose and Environment	1888

A.7.6.2.5.2	Test Requirements	1892

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1892

A.7.6.2.6.1	Test Purpose and Environment	1892

A.7.6.2.6.2	Test Requirements	1897

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1898

A.7.6.2.7.1	Test Purpose and Environment	1898

A.7.6.2.7.2	Test Requirements	1902

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1902

A.7.6.2.8.1	Test Purpose and Environment	1902

A.7.6.2.8.2	Test Requirements	1907

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1908

A.7.6.2.9.1	Test Purpose and Environment	1908

A.7.6.2.9.2	Test Requirements	1912

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1912

A.7.6.2.10.1	Test Purpose and Environment	1912

A.7.6.2.10.2	Test Requirements	1914

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1914

A.7.6.2.11.1	Test Purpose and Environment	1914

A.7.6.2.11.2	Test Requirements	1917

A.7.6.3	L1-RSRP measurement for beam reporting	1918

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1918

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1918

A.7.6.3.1.1	Test Purpose and Environment	1918

A.7.6.3.1.2	Test parameters	1918

A.7.6.3.1.3	Test Requirements	1920

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1920

A.7.6.3.2.1	Test Purpose and Environment	1920

A.7.6.3.2.2	Test parameters	1921

A.7.6.3.2.3	Test Requirements	1923

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1923

A.7.6.3.3.1	Test Purpose and Environment	1923

A.7.6.3.3.2	Test parameters	1924

A.7.6.3.3.3	Test Requirements	1926

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1927

A.7.6.3.4.1	Test Purpose and Environment	1927

A.7.6.3.4.2	Test parameters	1927

A.7.6.3.3.3	Test Requirements	1929

A.7.6.4	CLI measurements	1930

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1930

A.7.6.4.1.1	Test Purpose and Environment	1930

A.7.6.4.1.2	Test Parameters	1930

A.7.6.4.1.3	Test Requirements	1932

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1932

A.7.6.4.2.1	Test Purpose and Environment	1932

A.7.6.4.2.2	Test Parameters	1933

A.7.6.4.2.3	Test Requirements	1934

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1935

A.7.6.5.1.1	Test Purpose and Environment	1935

A.7.6.5.1.2	Test Requirements	1938

A.7.6.6	L1-SINR measurement for beam reporting	1938

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1940

A.7.6.6.2.1	Test Purpose and Environment	1940

A.7.6.6.2.2	Test parameters	1941

A.7.6.6.2.3	Test Requirements	1943

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1944

A.7.6.6.3.1	Test Purpose and Environment	1944

A.7.6.6.3.2	Test parameters	1944

A.7.6.6.3.3	Test Requirements	1946

A.7.6.7	CSI-RS based intra-frequency Measurements	1946

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1946

A.7.6.7.1.1	Test purpose and Environment	1946

A.7.6.7.1.2	Test Requirements	1949

A.7.6.8	CSI-RS based inter-frequency Measurements	1949

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1949

A.7.6.8.1.1	Test Purpose and Environment	1949

A.7.6.8.1.2	Test Requirements	1952

A.7.6.9	RSTD measurements	1952

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1952

A.7.6.9.1.1	Test Purpose and Environment	1952

A.7.6.9.1.2	Test Requirements	1958

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1958

A.7.6.9.2.1	Test Purpose and Environment	1958

A.7.6.9.2.2	Test Requirements	1965

A.7.6.10 PRS-RSRP measurements	1965

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	1965

A.7.6.10.1.1	Test Purpose and Environment	1965

A.7.6.10.1.2	Test Requirements	1969

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1969

A.7.6.10.2.1	Test Purpose and Environment	1969

A.7.6.10.2.2	Test Requirements	1973

A.7.6.11	UE Rx-Tx time difference measurements	1973

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	1973

A.7.6.11.1.1	Test purpose and environment	1973

A.7.6.11.1.2	Test requirements	1976

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	1976

A.7.6.11.2.1	Test purpose and environment	1976

A.7.6.11.2.2	Test requirements	1979

A.7.7	Measurement Performance requirements	1979

A.7.7.1	SS-RSRP	1979

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1979

A.7.7.1.1.1	Test Purpose and Environment	1979

A.7.7.1.1.2	Test parameters	1979

A.7.7.1.1.3	Test Requirements	1984

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1985

A.7.7.1.2.1	Test Purpose and Environment	1985

A.7.7.1.2.2	Test parameters	1985

A.7.7.1.2.3	Test Requirements	1989

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1990

A.7.7.1.3.1	Test Purpose and Environment	1990

A.7.7.1.3.2	Test parameters	1990

A.7.7.1.3.3	Test Requirements	1993

A.7.7.2	SS-RSRQ	1994

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1994

A.7.7.2.1.1	Test Purpose and Environment	1994

A.7.7.2.1.2	Test Parameters	1994

A.7.7.2.1.3	Test Requirements	1996

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1996

A.7.7.2.2.1	Test Purpose and Environment	1996

A.7.7.2.2.2	Test Parameters	1996

A.7.7.2.2.3	Test Requirements	1998

A.7.7.3	SS-SINR	1998

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1998

A.7.7.3.1.1	Test Purpose and Environment	1998

A.7.7.3.1.2	Test Parameters	1998

A.7.7.3.1.3	Test Requirements	2001

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2001

A.7.7.3.2.1	Test Purpose and Environment	2001

A.7.7.3.2.2	Test Parameters	2001

A.7.7.3.2.3	Test Requirements	2003

A.7.7.4	L1-RSRP measurement for beam reporting	2003

A.7.7.4.1	SSB based L1-RSRP measurement	2003

A.7.7.4.1.1	Test Purpose and Environment	2003

A.7.7.4.1.2	Test parameters	2004

A.7.7.4.1.3	Test Requirements	2006

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2007

A.7.7.4.2.1	Test Purpose and Environment	2007

A.7.7.4.2.2	Test parameters	2007

A.7.7.4.2.3	Test Requirements	2009

A.7.7.5	CLI measurements	2010

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2010

A.7.7.5.1.1	Test Purpose and Environment	2010

A.7.7.5.1.2	Test parameters	2010

A.7.7.5.1.3	Test Requirements	2013

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2014

A.7.7.5.2.1	Test Purpose and Environment	2014

A.7.7.5.2.2	Test parameters	2014

A.7.7.5.2.3	Test Requirements	2016

A.7.7.6	L1-SINR measurement for beam reporting	2017

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2019

A.7.7.6.2.1	Test Purpose and Environment	2019

A.7.7.6.2.2	Test parameters	2020

A.7.7.6.2.3	Test Requirements	2022

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2022

A.7.7.6.3.1	Test Purpose and Environment	2022

A.7.7.6.3.2	Test parameters	2023

A.7.7.6.3.3	Test Requirements	2025

A.7.7.7	CSI-RSRP	2025

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2025

A.7.7.7.1.1	Test Purpose and Environment	2025

A.7.7.7.1.2	Test parameters	2026

A.7.7.7.1.3	Test Requirements	2029

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2030

A.7.7.7.2.1	Test Purpose and Environment	2030

A.7.7.7.2.2	Test parameters	2030

A.7.7.7.2.3	Test Requirements	2033

A.7.7.8	CSI-RSRQ	2033

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2033

A.7.7.8.1.1	Test Purpose and Environment	2033

A.7.7.8.1.2	Test Parameters	2034

A.7.7.8.1.3	Test Requirements	2036

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2036

A.7.7.8.2.1	Test Purpose and Environment	2036

A.7.7.8.2.2	Test Parameters	2036

A.7.7.8.2.3	Test Requirements	2038

A.7.7.9	CSI-SINR	2038

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2038

A.7.7.9.1.1	Test Purpose and Environment	2038

A.7.7.9.1.2	Test Parameters	2038

A.7.7.9.1.3	Test Requirements	2040

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2040

A.7.7.9.2.1	Test Purpose and Environment	2040

A.7.7.9.2.2	Test Parameters	2041

A.7.7.9.2.3	Test Requirements	2043

A.7.7.10	RSTD measurements	2043

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2043

A.7.7.10.1.1	Test purpose and Environment	2043

A.7.7.10.1.2	Test Requirements	2045

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2045

A.7.7.10.2.1	Test purpose and Environment	2045

A.7.7.10.2.2	Test Requirements	2047

A.7.7.11	PRS-RSRP measurements	2047

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2047

A.7.7.11.1.1	Test Purpose and Environment	2047

A.7.7.11.1.2	Test parameters	2047

A.7.7.11.1.3	Test Requirements	2050

A.7.7.12	UE Rx-Tx time difference measurements	2051

A.7.7.12.1	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2051

A.7.7.12.1.1	Test purpose and environment	2051

A.7.7.12.1.2	Test parameters	2051

A.7.7.12.1.3	Test requirements	2053

A.8	E-UTRA standalone tests for NR RRM	2053

A.8.1	Void	2053

A.8.2	RRC_IDLE state mobility	2053

A.8.2.1	Inter-RAT NR Cell re-selection	2053

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2053

A.8.2.1.1.1	Test Purpose and Environment	2053

A.8.2.1.1.2	Test Requirements	2058

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2059

A.8.2.1.2.1	Test Purpose and Environment	2059

A.8.2.1.2.2	Test Requirements	2062

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2063

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2063

A.8.2.2.1.1	Test Purpose and Environment	2063

A.8.2.2.1.2	Test Requirements	2066

A.8.3	RRC_CONNECTED state mobility	2067

A.8.3.1	Handover	2067

A.8.3.1.1	E-UTRAN - NR handover in FR1	2067

A.8.3.1.1.1	Test Purpose and Environment	2067

A.8.3.1.1.2	Test Requirements	2072

A.8.4	Measurement procedure	2072

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2072

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2072

A.8.4.1.1.1	Test Purpose and Environment	2072

A.8.4.1.1.2	Test Requirements	2074

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2075

A.8.4.1.2.1	Test Purpose and Environment	2075

A.8.4.1.2.2	Test Requirements	2076

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2076

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2076

A.8.4.2.1.1	Test Purpose and Environment	2076

A.8.4.2.1.2	Test Requirements	2081

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2081

A.8.4.2.2.1	Test Purpose and Environment	2081

A.8.4.2.2.2	Test Requirements	2085

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2085

A.8.4.2.3.1	Test Purpose and Environment	2085

A.8.4.2.3.2	Test Requirements	2090

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2090

A.8.4.2.4.1	Test Purpose and Environment	2090

A.8.4.2.4.2	Test Requirements	2094

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2094

A.8.4.2.5.1	Test Purpose and Environment	2094

A.8.4.2.5.2	Test Requirements	2096

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2097

A.8.4.2.6.1	Test Purpose and Environment	2097

A.8.4.2.6.2	Test Requirements	2099

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2100

A.8.4.2.7.1	Test Purpose and Environment	2100

A.8.4.2.7.2	Test Requirements	2102

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2103

A.8.4.2.8.1	Test Purpose and Environment	2103

A.8.4.2.8.2	Test Requirements	2105

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2106

A.8.4.2.9.1	Test Purpose and Environment	2106

A.8.4.2.9.2	Test Requirements	2110

A.8.5	Measurement performance	2110

A.8.5.1	SFTD accuracy	2110

A.8.5.1.1	SFTD accuracy	2110

A.8.5.1.1.1	Test Purpose	2110

A.8.5.1.1.2	Test Environment	2111

A.8.5.1.1.3	Test Requirements	2117

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2117

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2117

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2122

A.8.5.2.1.2.1	Test Purpose and Environment	2122

A.8.5.2.1.2.2	Test Parameters	2122

A.8.5.2.1.2.3	Test Requirements	2124

A.8.5.2.2	SS-RSRQ	2124

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2124

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2129

A.8.5.2.2.2.1	Test Purpose and Environment	2129

A.8.5.2.2.2.2	Test Parameters	2129

A.8.5.2.2.2.3	Test Requirements	2131

A.8.5.2.3	SS-SINR	2131

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2131

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2135

A.8.5.2.3.2.1	Test Purpose and Environment	2135

A.8.5.2.3.2.2	Test Parameters	2135

A.8.5.2.3.2.3	Test Requirements	2137

A.9	V2X Tests	2138

A.9.1	V2X Tests in FR1	2138

A.9.1.1	Test for V2X UE Transmit Timing	2138

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	2138

A.9.1.1.1.1	Test Purpose and Environment	2138

A.9.1.1.1.2	Test requirements	2138

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2138

A.9.1.1.2.1	Test Purpose and Environment	2138

A.9.1.1.2.2	Test requirements	2139

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2139

A.9.1.1.3.1	Test Purpose and Environment	2139

A.9.1.1.3.2	Test requirements	2142

A.9.1.2	Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2142

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2142

A.9.1.2.1.1	Test Purpose and Environment	2142

A.9.1.2.1.2	Test Requirements	2145

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2145

A.9.1.2.2.1	Test Purpose and Environment	2145

A.9.1.2.2.2	Test Requirements	2146

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	2147

A.9.1.3.1	 Test for GNSS configured as the highest priority	2147

A.9.1.3.1.1	Test Purpose and Environment	2147

A.9.1.3.1.2	Test Requirements	2149

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2150

A.9.1.3.2.1	Test Purpose and Environment	2150

A.9.1.3.2.2	Test Requirements	2152

A.9.1.4	Test for L1 SL-RSRP Measurement	2153

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2153

A.9.1.4.1.1	Test Purpose and Environment	2153

A.9.1.4.1.2	Test Requirements	2155

A. 9.1.4.2 Test for V2X UE Resource Pre-emption	2155

A. 9.1.4.2.1	Test Purpose and Environment	2155

A.9.1.4.2.2	Test Requirements	2159

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	2159

A.9.1.4.3.1	Test Purpose and Environment	2159

A.9.1.4.3.2	Test Requirements	2163

A.9.1.5	Test for Congestion Control Measurement	2163

A.9.1.5.1	Test Purpose and Environment	2163

A.9.1.5.2	Test Requirements	2166

A.9.1.6	Test for Interruption	2166

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2166

A.9.1.6.1.1	Test Purpose and Environment	2166

A.9.1.6.1.2	Test Requirements	2170

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2170

A.10.1	RRC_CONNECTED state mobility	2170

A.10.1.1	RRC connection mobility control	2170

A.10.1.1.1	Random Access	2170

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2170

A.10.1.1.1.1.1	Test Purpose and Environment	2170

A.10.1.1.1.1.2	Test Requirements	2173

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2173

A.10.1.1.1.1.2.2	Random Access Response Reception	2173

A.10.1.1.1.1.2.3	No Random Access Response Reception	2174

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2174

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2174

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2175

A.10.1.1.1.2.1	Test Purpose and Environment	2175

A.10.1.1.1.2.2	Test Requirements	2178

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2178

A.10.1.1.1.2.2.2	Random Access Response Reception	2179

A.10.1.1.1.2.2.3	No Random Access Response Reception	2179

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2179

A.10.1.1.1.3.1	Test Purpose and Environment	2179

A.10.1.1.1.3.2	Test Requirements	2183

A.10.1.1.1.3.2.1	MsgA Transmission	2183

A.10.1.1.1.3.2.2	MsgB Reception	2183

A.10.1.1.1.3.2.3	No MsgB Reception	2184

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2184

A.10.1.1.1.4.1	Test Purpose and Environment	2184

A.10.1.1.1.4.2	Test Requirements	2188

A.10.1.1.1.4.2.1	MsgA Transmission	2188

A.10.1.1.1.4.2.2	MsgB Reception	2189

A.10.1.1.1.4.2.3	No MsgB Reception	2189

A.10.2	Timing	2190

A.10.2.1	UE transmit timing	2190

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	2190

A.10.2.1.1.1	Test Purpose and environment	2190

A.10.2.1.1.2	Test requirements	2193

A.10.2.2	UE timing advance	2194

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	2194

A.10.2.2.1.1	Test Purpose and Environment	2194

A.10.2.2.1.2	Test Parameters	2194

A.10.2.2.1.3	Test Requirements	2197

A.10.3	Signalling characteristics	2197

A.10.3.1	Radio link monitoring	2197

A.10.3.1.1	Introduction	2197

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2198

A.10.3.1.2.1	Test purpose and environment	2198

A.10.3.1.2.2	Test requirements	2202

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2202

A.10.3.1.3.1	Test purpose and environment	2202

A.10.3.1.3.2	Test requirements	2208

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2208

A.10.3.1.4.1	Test purpose and environment	2208

A.10.3.1.4.2	Test requirements	2208

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2208

A.10.3.1.5.1	Test purpose and environment	2208

A.10.3.1.5.2	Test requirements	2208

A.10.3.2	Interruption	2208

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2208

A.10.3.2.1.1	Test Purpose and Environment	2208

A.10.3.2.1.2	Test Requirements	2213

A.10.3.3	SCell activation and deactivation delay	2213

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	2213

A.10.3.3.1.1	Test Purpose and Environment	2213

A.10.3.3.1.2	Test Requirements	2218

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 320 ms SCell measurement cycle	2218

A.10.3.3.2.1	Test Purpose and Environment	2218

A.10.3.3.2.2	Test Requirements	2219

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2219

A.10.3.3.3.1	Test Purpose and Environment	2219

A.10.3.3.3.2	Test Requirements	2220

A.10.3.4	Beam failure detection and link recovery procedures	2220

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2220

A.10.3.4.1.1	Test Purpose and Environment	2220

A.10.3.4.1.2	Test Requirements	2226

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2226

A.10.3.4.2.1	Test Purpose and Environment	2226

A.10.3.4.2.2	Test Requirements	2233

A.10.3.5	Active BWP switching	2233

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2233

A.10.3.5.1.2	Test Requirements	2238

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2239

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2239

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2242

A.10.3.5.3	RRC-based Active BWP Switch	2246

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2246

A.10.3.6	PSCell addition and release delay	2249

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2249

A.10.3.6.1.1	Test purpose and environment	2249

A.10.3.6.1.2	Test Requirements	2253

A.10.3.7	Active TCI state switching delay	2254

A.10.4	Measurement procedure	2254

A.10.4.1	Intra-frequency measurements	2254

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2254

A.10.4.1.1.1	Test purpose and environment	2254

A.10.4.1.1.2	Test parameters	2254

A.10.4.1.1.3	Test Requirements	2254

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2255

A.10.4.1.2.1	Test purpose and environment	2255

A.10.4.1.2.2	Test parameters	2255

A.10.4.1.2.3	Test Requirements	2255

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2255

A.10.4.1.3.1	Test purpose and environment	2255

A.10.4.1.3.2	Test parameters	2255

A.10.4.1.3.3	Test Requirements	2255

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2256

A.10.4.1.4.1	Test purpose and environment	2256

A.10.4.1.4.2	Test parameters	2256

A.10.4.1.4.3	Test Requirements	2256

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2256

A.10.4.1.5.1	Test purpose and environment	2256

A.10.4.1.5.2	Test parameters	2256

A.10.4.1.5.3	Test Requirements	2256

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2257

A.10.4.1.6.1	Test purpose and environment	2257

A.10.4.1.6.2	Test parameters	2257

A.10.4.1.6.3	Test Requirements	2257

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2257

A.10.4.1.7.1	Test purpose and environment	2257

A.10.4.1.7.2	Test parameters	2257

A.10.4.1.7.3	Test Requirements	2257

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2258

A.10.4.1.8.1	Test purpose and environment	2258

A.10.4.1.8.2	Test parameters	2258

A.10.4.1.8.3	Test Requirements	2258

A.10.4.1.9	RSSI measurement reporting on PSCC	2258

A.10.4.1.9.1	Test purpose and environment	2258

A.10.4.1.9.2	Test parameters	2258

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2259

A.10.4.1.10.1	Test purpose and environment	2259

A.10.4.1.10.2	Test parameters	2259

A.10.4.1.11	RSSI measurement reporting on SCC	2259

A.10.4.1.11.1	Test purpose and environment	2259

A.10.4.1.11.2	Test parameters	2259

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2260

A.10.4.1.12.1	Test purpose and environment	2260

A.10.4.1.12.2	Test parameters	2260

A.10.4.2	Inter-frequency measurements	2260

A.10.4.2.1	RSSI measurement reporting	2260

A.10.4.2.1.1	Test purpose and environment	2260

A.10.4.2.1.2	Test parameters	2260

A.10.4.2.2	Channel occupancy measurement reporting	2261

A.10.4.2.2.1	Test purpose and environment	2261

A.10.4.2.2.2	Test parameters	2261

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2261

A.10.4.2.3.1	Test Purpose and Environment	2261

A.10.4.2.3.2	Test Requirements	2266

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2267

A.10.4.2.4.1	Test Purpose and Environment	2267

A.10.4.2.4.2	Test Requirements	2271

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2272

A.10.4.2.5.1	Test Purpose and Environment	2272

A.10.4.2.5.2	Test Requirements	2276

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2277

A.10.4.2.6.1	Test Purpose and Environment	2277

A.10.4.2.6.2	Test Requirements	2281

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2282

A.10.4.2.7.1	Test Purpose and Environment	2282

A.10.4.2.7.2	Test Requirements	2288

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2288

A.10.4.2.8.1	Test Purpose and Environment	2288

A.10.4.2.8.2	Test Requirements	2294

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2295

A.10.4.2.9.1	Test Purpose and Environment	2295

A.10.4.2.9.2	Test Requirements	2300

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2300

A.10.4.2.10.1	Test Purpose and Environment	2300

A.10.4.2.10.2	Test Requirements	2306

A.10.4.3	L1-RSRP measurements for beam reporting	2307

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2307

A.10.4.3.1.1	Test Purpose and Environment	2307

A.10.4.3.1.2	Test parameters	2307

A.10.4.3.1.3	Test Requirements	2309

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2310

A.10.4.3.2.1	Test Purpose and Environment	2310

A.10.4.3.2.2	Test parameters	2310

A.10.4.3.2.3	Test Requirements	2312

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2313

A.10.4.3.3.1	Test Purpose and Environment	2313

A.10.4.3.3.2	Test parameters	2313

A.10.4.3.3.3	Test Requirements	2316

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2317

A.10.4.3.4.1	Test Purpose and Environment	2317

A.10.4.3.4.2	Test parameters	2317

A.10.4.3.4.3	Test Requirements	2320

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2321

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2321

A.10.4.4.1.1	Test Purpose and Environment	2321

A.10.4.4.1.2	Test Requirements	2325

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2326

A.10.4.4.2.1	Test Purpose and Environment	2326

A.10.4.4.2.2	Test Requirements	2331

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2332

A.10.4.4.3.1	Test Purpose and Environment	2332

A.10.4.4.3.2	Test Requirements	2336

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2337

A.10.4.4.4.1	Test Purpose and Environment	2337

A.10.4.4.4.2	Test Requirements	2342

A.10.5	Measurement performance	2343

A.10.5.1	SS-RSRP	2343

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2343

A.10.5.1.1.1	Test Purpose and Environment	2343

A.10.5.1.1.2	Test parameters	2343

A.10.5.1.1.3	Test Requirements	2346

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2346

A.10.5.1.2.1	Test Purpose and Environment	2346

A.10.5.1.2.2	Test parameters	2346

A.10.5.1.2.3	Test Requirements	2349

A.10.5.2	SS-RSRQ	2349

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2349

A.10.5.2.1.1	Test Purpose and Environment	2349

A.10.5.2.1.2	Test Parameters	2349

A.10.5.2.1.3	Test Requirements	2352

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2352

A.10.5.2.2.1	Test Purpose and Environment	2352

A.10.5.2.2.2	Test Parameters	2352

A.10.5.2.2.3	Test Requirements	2355

A.10.5.3	SS-SINR	2355

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2355

A.10.5.4.1	SSB based L1-RSRP measurement	2355

A.10.5.4.1.1	Test Purpose and Environment	2355

A.10.5.4.1.2	Test parameters	2355

A.10.5.4.1.3	Test Requirements	2358

A.10.5.5	RSSI	2358

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2358

A.10.5.5.1.1	Test Purpose and Environment	2358

A.10.5.5.1.2	Test parameters	2358

A.10.5.5.1.3	Test Requirements	2361

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2361

A.10.5.5.2.1	Test Purpose and Environment	2361

A.10.5.5.2.2	Test parameters	2361

A.10.5.5.2.3	Test Requirements	2364

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2364

A.10.5.5.3.1	Test Purpose and Environment	2364

A.10.5.5.3.2	Test parameters	2364

A.10.5.5.3.3	Test Requirements	2367

A.10.5.6	Channel occupancy	2367

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2367

A.10.5.6.1.1	Test Purpose and Environment	2367

A.10.5.6.1.2	Test parameters	2367

A.10.5.6.1.3	Test Requirements	2371

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2371

A.10.5.6.2.1	Test Purpose and Environment	2371

A.10.5.6.2.2	Test parameters	2371

A.10.5.6.2.3	Test Requirements	2374

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2374

A.10.5.6.3.1	Test Purpose and Environment	2374

A.10.5.6.3.2	Test parameters	2374

A.10.5.6.3.3	Test Requirements	2378

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2378

A.11.1	RRC_IDLE state mobility	2378

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2378

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2378

A.11.1.1.1.1	Test Purpose and Environment	2378

A.11.1.1.1.2	Test Parameters	2378

A.11.1.1.1.3	Test Requirements	2381

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2381

A.11.1.1.2.1	Test Purpose and Environment	2381

A.11.1.1.2.2	Test Parameters	2381

A.11.1.1.2.3	Test Requirements	2385

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2385

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2385

A.11.1.2.1.1	Test Purpose and Environment	2385

A.11.1.2.1.2	Test Parameters	2386

A.11.1.2.1.3	Test Requirements	2392

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2393

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2393

A.11.1.3.1.1	Test Purpose and Environment	2393

A.11.1.3.1.2	Test Parameters	2393

A.11.1.3.1.3	Test Requirements	2399

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2400

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2400

A.11.1.4.1.1	Test Purpose and Environment	2400

A.11.1.4.1.2	Test Parameters	2400

A.11.1.4.1.3	Test Requirements	2403

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2404

A.11.1.4.2.1	Test Purpose and Environment	2404

A.11.1.4.2.2	Test Requirements	2406

A.11.2	RRC_CONNECTED state mobility	2407

A.11.2.1	Handover	2407

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2407

A.11.2.1.1.1	Test Purpose and Environment	2407

A.11.2.1.1.2	Test Parameters	2407

A.11.2.1.1.3 Test Requirements	2410

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2410

A.11.2.1.2.1	Test Purpose and Environment	2410

A.11.2.1.2.2	Test Parameters	2410

A.11.2.1.2.3	Test Requirements	2413

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2413

A.11.2.1.3.1	Test Purpose and Environment	2413

A.11.2.1.3.2	Test Parameters	2413

A.11.2.1.3.3 Test Requirements	2416

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2416

A.11.2.1.4.1	Test Purpose and Environment	2416

A.11.2.1.4.2	Test Parameters	2416

A.11.2.1.4.3 Test Requirements	2420

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2420

A.11.2.1.5.1	Test Purpose and Environment	2420

A.11.2.1.5.2	Test Parameters	2420

A.11.2.1.5.3 Test Requirements	2424

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2425

A.11.2.1.6.1	Test Purpose and Environment	2425

A.11.2.1.6.2	Test Parameters	2425

A.11.2.1.6.3 Test Requirements	2428

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2429

A.11.2.1.7.1	Test Purpose and Environment	2429

A.11.2.1.7.2	Test Requirements	2435

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2435

A.11.2.1.8.1	Test Purpose and Environment	2435

A.11.2.1.8.2	Test Requirements	2440

A.11.2.2	RRC connection mobility control	2440

A.11.2.2.1	RRC re-establishment	2440

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2440

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2443

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2451

A.11.2.2.2	Random Access	2454

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2454

A.11.2.2.2.1.1	Test Purpose and Environment	2454

A.11.2.2.2.1.2	Test Requirements	2458

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2458

A.11.2.2.2.1.2.2	Random Access Response Reception	2458

A.11.2.2.2.1.2.3	No Random Access Response Reception	2459

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2459

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2459

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2460

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2460

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2460

A.11.2.2.2.2.1	Test Purpose and Environment	2460

A.11.2.2.2.2.2	Test Requirements	2463

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2463

A.11.2.2.2.2.2.2	Random Access Response Reception	2464

A.11.2.2.2.2.2.3	No Random Access Response Reception	2464

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2464

A.11.2.2.2.3.1	Test Purpose and Environment	2464

A.11.2.2.2.3.2	Test Requirements	2468

A.11.2.2.2.3.2.1	MsgA Transmission	2468

A.11.2.2.2.3.2.2	MsgB Reception	2468

A.11.2.2.2.3.2.3	No MsgB Reception	2469

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2469

A.11.2.2.2.4.1	Test Purpose and Environment	2469

A.11.2.2.2.4.2	Test Requirements	2473

A.11.2.2.2.4.2.1	MsgA Transmission	2473

A.11.2.2.2.4.2.2	MsgB Reception	2474

A.11.2.2.2.4.2.3	No MsgB Reception	2474

A.11.2.2.3	RRC connection release with redirection	2475

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2475

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	2479

A.11.3	Timing	2483

A.11.3.1	UE transmit timing	2483

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	2483

A.11.3.1.1.1	Test Purpose and environment	2483

A.11.3.1.1.2	Test requirements	2487

A.11.3.2	UE timing advance	2488

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2488

A.11.3.2.1.1	Test Purpose and Environment	2488

A.11.3.2.1.2	Test Parameters	2488

A.11.3.2.1.3	Test Requirements	2491

A.11.4	Signalling characteristics	2491

A.11.4.1	Radio link monitoring	2491

A.11.4.1.1	Introduction	2491

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2492

A.11.4.1.2.1	Test purpose and environment	2492

A.11.4.1.2.2	Test requirements	2496

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2496

A.11.4.1.3.1	Test purpose and environment	2496

A.11.4.1.3.2	Test requirements	2502

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2502

A.11.4.1.4.1	Test purpose and environment	2502

A.11.4.1.4.2	Test requirements	2502

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2502

A.11.4.1.5.1	Test purpose and environment	2502

A.11.4.1.5.2	Test requirements	2502

A.11.4.2	Interruption	2503

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2503

A.11.4.2.1.1 Test Purpose and Environment	2503

A.11.4.2.1.2	Test Requirements	2507

A.11.4.3	SCell activation and deactivation delay	2507

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	2507

A.11.4.3.1.1	Test Purpose and Environment	2507

A.11.4.3.1.2	Test Requirements	2512

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 320 ms SCell measurement cycle	2512

A.11.4.3.2.1	Test Purpose and Environment	2512

A.11.4.3.2.2	Test Requirements	2513

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	2513

A.11.4.3.3.1	Test Purpose and Environment	2513

A.11.4.3.3.2	Test Requirements	2514

A.11.4.4	Beam failure detection and link recovery procedures	2514

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2514

A.11.4.4.1.1	Test Purpose and Environment	2514

A.11.4.4.1.2	Test Requirements	2520

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2520

A.11.4.4.2.1	Test Purpose and Environment	2520

A.11.4.4.2.2	Test Requirements	2527

A.11.4.5	Active BWP switching	2527

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2527

A.11.4.5.1.1	Test Purpose and Environment	2527

A.11.4.5.1.2	Test Requirements	2532

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	2533

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	2533

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	2536

A.11.4.5.3	RRC-based Active BWP Switch	2539

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2539

A.11.4.6	Active TCI state switching delay	2542

A.11.5	Measurement procedure	2542

A.11.5.1	Intra-frequency measurements	2542

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2542

A.11.5.1.1.1	Test purpose and environment	2542

A.11.5.1.1.2	Test parameters	2542

A.11.5.1.1.3	Test Requirements	2543

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2543

A.11.5.1.2.1	Test purpose and environment	2543

A.11.5.1.2.2	Test parameters	2543

A.11.5.1.2.3	Test Requirements	2543

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2544

A.11.5.1.3.1	Test purpose and environment	2544

A.11.5.1.3.2	Test parameters	2544

A.11.5.1.3.3	Test Requirements	2544

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2544

A.11.5.1.4.1	Test purpose and environment	2544

A.11.5.1.4.2	Test parameters	2544

A.11.5.1.4.3	Test Requirements	2544

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2545

A.11.5.1.5.1	Test purpose and environment	2545

A.11.5.1.5.2	Test parameters	2545

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2545

A.11.5.1.6.1	Test purpose and environment	2545

A.11.5.1.6.2	Test parameters	2545

A.11.5.1.6.3	Test Requirements	2545

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2546

A.11.5.1.7.1	Test purpose and environment	2546

A.11.5.1.7.2	Test parameters	2546

A.11.5.1.7.3	Test Requirements	2546

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2546

A.11.5.1.8.1	Test purpose and environment	2546

A.11.5.1.8.2	Test parameters	2546

A.11.5.1.8.3	Test Requirements	2546

A.11.5.1.9	RSSI measurement reporting on PCC	2547

A.11.5.1.9.1	Test purpose and environment	2547

A.11.5.1.9.2	Test parameters	2547

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2547

A.11.5.1.10.1	Test purpose and environment	2547

A.11.5.1.10.2	Test parameters	2547

A.11.5.1.11	RSSI measurement reporting on SCC	2547

A.11.5.1.11.1	Test purpose and environment	2547

A.11.5.1.11.2	Test parameters	2548

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2548

A.11.5.1.12.1	Test purpose and environment	2548

A.11.5.1.12.2	Test parameters	2548

A.11.5.2	Inter-frequency measurements	2548

A.11.5.2.1	RSSI measurement reporting	2548

A.11.5.2.1.1	Test purpose and environment	2548

A.11.5.2.1.2	Test parameters	2549

A.11.5.2.2	Channel occupancy measurement reporting	2549

A.11.5.2.2.1	Test purpose and environment	2549

A.11.5.2.2.2	Test parameters	2549

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2549

A.11.5.2.3.1	Test Purpose and Environment	2549

A.11.5.2.3.2	Test Requirements	2553

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2554

A.11.5.2.4.1	Test Purpose and Environment	2554

A.11.5.2.4.2	Test Requirements	2558

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2559

A.11.5.2.5.1	Test Purpose and Environment	2559

A.11.5.2.5.2	Test Requirements	2563

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2564

A.11.5.2.6.1	Test Purpose and Environment	2564

A.11.5.2.6.2	Test Requirements	2568

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2569

A.11.5.2.7.1	Test Purpose and Environment	2569

A.11.5.2.7.2	Test Requirements	2573

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2574

A.11.5.2.8.1	Test Purpose and Environment	2574

A.11.5.2.8.2	Test Requirements	2579

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2579

A.11.5.2.9.1	Test Purpose and Environment	2579

A.11.5.2.9.2	Test Requirements	2584

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2585

A.11.5.2.10.1	Test Purpose and Environment	2585

A.11.5.2.10.2	Test Requirements	2590

A.11.5.3	Inter-RAT E-UTRAN measurements	2591

A.11.5.4	L1-RSRP measurements for beam reporting	2591

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2591

A.11.5.4.1.1	Test Purpose and Environment	2591

A.11.5.4.1.2	Test parameters	2591

A.11.5.4.1.3	Test Requirements	2593

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2593

A.11.5.4.2.1	Test Purpose and Environment	2593

A.11.5.4.2.2	Test parameters	2594

A.11.5.4.2.3	Test Requirements	2596

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2596

A.11.5.4.3.1	Test Purpose and Environment	2596

A.11.5.4.3.2	Test parameters	2597

A.11.5.4.3.3	Test Requirements	2600

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2601

A.11.5.4.4.1	Test Purpose and Environment	2601

A.11.5.4.4.2	Test parameters	2601

A.11.5.4.4.3	Test Requirements	2604

A.11.6	Measurement performance	2605

A.11.6.1	SS-RSRP	2605

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2605

A.11.6.1.1.1	Test Purpose and Environment	2605

A.11.6.1.1.2	Test parameters	2605

A.11.6.1.1.3	Test Requirements	2607

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2607

A.11.6.1.2.1	Test Purpose and Environment	2607

A.11.6.1.2.2	Test parameters	2607

A.11.6.1.2.3	Test Requirements	2609

A.11.6.2	SS-RSRQ	2609

A.11.6.3	SS-SINR	2609

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2609

A.11.6.4.1	SSB based L1-RSRP measurement	2609

A.11.6.4.1.1	Test Purpose and Environment	2609

A.11.6.4.1.2	Test parameters	2609

A.11.6.4.1.3	Test Requirements	2612

A.11.6.5	RSSI	2612

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2612

A.11.6.5.1.1	Test Purpose and Environment	2612

A.11.6.5.1.2	Test parameters	2612

A.11.6.5.1.3	Test Requirements	2615

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2615

A.11.6.5.2.1	Test Purpose and Environment	2615

A.11.6.5.2.2	Test parameters	2615

A.11.6.5.2.3	Test Requirements	2618

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2618

A.11.6.5.3.1	Test Purpose and Environment	2618

A.11.6.5.3.2	Test parameters	2618

A.11.6.5.3.3	Test Requirements	2621

A.11.6.6	Channel occupancy	2621

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2621

A.11.6.6.1.1	Test Purpose and Environment	2621

A.11.6.6.1.2	Test parameters	2621

A.11.6.6.1.3	Test Requirements	2624

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2624

A.11.6.6.2.1	Test Purpose and Environment	2624

A.11.6.6.2.2	Test parameters	2624

A.11.6.6.2.3	Test Requirements	2627

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2627

A.11.6.6.3.1	Test Purpose and Environment	2627

A.11.6.6.3.2	Test parameters	2627

A.11.6.6.3.3	Test Requirements	2630

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.	2630

A.11.6.7	E-UTRAN RSRP	2630

A.11.6.8	E-UTRAN RSRQ	2630

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2630

A.12.1	RRC_IDLE state mobility	2630

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2630

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2630

A.12.1.1.1.1	Test Purpose and Environment	2630

A.12.1.1.1.2	Test Requirements	2635

A.12.2	RRC_CONNECTED state mobility	2636

A.12.2.1	Handover	2636

A.12.2.1.1	E-UTRAN - NR with CCA handover	2636

A.12.2.1.1.1	Test Purpose and Environment	2636

A.12.2.1.1.2	Test Requirements	2641

A.12.3	Signalling characteristics	2641

A.12.3.1	Interruptions	2641

A.12.4	Measurement procedure	2641

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2641

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2641

A.12.4.1.1.1	Test Purpose and Environment	2641

A.12.4.1.1.2	Test Requirements	2645

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2645

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2645

A.12.4.2.1.1	Test Purpose and Environment	2645

A.12.4.2.1.2	Test Requirements	2649

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2650

A.12.4.2.2.1	Test Purpose and Environment	2650

A.12.4.2.2.2	Test Requirements	2654

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2654

A.12.4.2.3.1	Test Purpose and Environment	2654

A.12.4.2.3.2	Test Requirements	2658

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2659

A.12.4.2.4.1	Test Purpose and Environment	2659

A.12.4.2.4.2	Test Requirements	2663

A.12.4.2.5	RSSI measurement reporting	2663

A.12.4.2.5.1	Test purpose and environment	2663

A.12.4.2.5.2	Test parameters	2663

A.12.4.2.6	Channel occupancy measurement reporting	2664

A.12.4.2.6.1	Test purpose and environment	2664

A.12.4.2.6.2	Test parameters	2664

A.12.5	Measurement performance	2664

A.12.5.1	E-UTRANNR SFTD	2664

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	2664

A.12.5.1.1.1	Test Purpose	2664

A.12.5.1.1.2	Test Environment	2665

A.12.5.1.1.3	Test Requirements	2670

A.12.5.2	E-UTRANNR SS-RSRP	2670

A.12.5.3	E-UTRANNR SS-RSRQ	2670

A.12.5.4	E-UTRANNR SS-SINR	2670

A.12.5.5	E-UTRANNR RSSI	2670

A.12.5.6	E-UTRANNR channel occupancy	2670

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2670

A.13.1	Timing	2670

A.13.1.1	UE transmit timing	2670

A.13.1.2	Timing advance	2670

A.13.2	Signalling characteristics	2670

A.13.2.1	Interruption	2670

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2670

A.13.2.1.1.1	Test Purpose and Environment	2670

A.13.2.1.1.2	Test Requirements	2674

A.13.2.2	SCell activation and deactivation delay	2674

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	2674

A.13.2.2.1.1	Test Purpose and Environment	2674

A.13.2.2.1.2	Test Requirements	2679

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 320 ms SCell measurement cycle	2679

A.13.2.2.2.1	Test Purpose and Environment	2679

A.13.2.2.2.2	Test Requirements	2680

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	2680

A.13.2.2.3.1	Test Purpose and Environment	2680

A.13.2.2.3.2	Test Requirements	2681

A.13.2.3	Active TCI state switching delay	2681

A.13.3	Measurement procedure	2681

A.13.3.1	Intra-frequency measurements	2681

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2681

A.13.3.1.1.1	Test purpose and environment	2681

A.13.3.1.1.2	Test parameters	2681

A.13.3.1.1.3	Test Requirements	2686

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2686

A.13.3.1.2.1	Test purpose and environment	2686

A.13.3.1.2.2	Test parameters	2686

A.13.3.1.2.3	Test Requirements	2690

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2690

A.13.3.1.3.1	Test purpose and environment	2690

A.13.3.1.3.2	Test parameters	2690

A.13.3.1.3.3	Test Requirements	2695

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2695

A.13.3.1.4.1	Test purpose and environment	2695

A.13.3.1.4.2	Test parameters	2695

A.13.3.1.4.3	Test Requirements	2700

A.13.3.1.5	RSSI measurement reporting on SCC	2700

A.13.3.1.5.1	Test purpose and environment	2700

A.13.3.1.5.2	Test parameters	2700

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2701

A.13.3.1.6.1	Test purpose and environment	2701

A.13.3.1.6.2	Test parameters	2702

A.13.3.2	Inter-frequency measurements	2702

A.13.3.2.1	RSSI measurement reporting	2702

A.13.3.2.1.1	Test purpose and environment	2702

A.13.3.2.1.2	Test parameters	2702

A.13.3.2.2	Channel occupancy measurement reporting	2703

A.13.3.2.2.1	Test purpose and environment	2703

A.13.3.2.2.2	Test parameters	2703

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2703

A.13.3.2.3.1	Test Purpose and Environment	2703

A.13.3.2.3.2	Test Requirements	2708

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2708

A.13.3.2.4.1	Test Purpose and Environment	2708

A.13.3.2.4.2	Test Requirements	2713

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2714

A.13.3.2.5.1	Test Purpose and Environment	2714

A.13.3.2.5.2	Test Requirements	2719

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2719

A.13.3.2.6.1	Test Purpose and Environment	2719

A.13.3.2.6.2	Test Requirements	2725

A.13.3.3	L1-RSRP measurements for beam reporting	2726

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2726

A.13.3.3.1.1	Test Purpose and Environment	2726

A.13.3.3.1.2	Test parameters	2726

A.13.3.3.1.3	Test Requirements	2730

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2730

A.13.3.3.2.1	Test Purpose and Environment	2730

A.13.3.3.2.2	Test parameters	2730

A.13.3.3.2.3	Test Requirements	2734

A.13.4	Measurement performance	2734

A.13.4.1	SS-RSRP	2734

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2734

A.13.4.1.1.1	Test Purpose and Environment	2734

A.13.4.1.1.2	Test parameters	2734

A.13.4.1.1.3	Test Requirements	2736

A.13.4.2	SS-RSRQ	2736

A.13.4.3	SS-SINR	2736

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2736

A.13.4.4.1	SSB based L1-RSRP measurement	2736

A.13.4.4.1.1	Test Purpose and Environment	2736

A.13.4.4.1.2	Test parameters	2736

A.13.4.4.1.3	Test Requirements	2741

A.13.4.5	RSSI	2741

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2741

A.13.4.5.1.1	Test Purpose and Environment	2741

A.13.4.5.1.2	Test parameters	2741

A.13.4.5.1.3	Test Requirements	2744

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2744

A.13.4.5.2.1	Test Purpose and Environment	2744

A.13.4.5.2.2	Test parameters	2744

A.13.4.5.2.3	Test Requirements	2748

A.13.4.6	Channel occupancy	2748

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2748

A.13.4.6.1.1	Test Purpose and Environment	2748

A.13.4.6.1.2	Test parameters	2748

A.13.4.6.1.3	Test Requirements	2752

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2752

A.13.4.6.2.1	Test Purpose and Environment	2752

A.13.4.6.2.2	Test parameters	2752

A.13.4.6.2.3	Test Requirements	2756

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1787

B.1	Conditions for NR RRC_IDLE state mobility	1787

B.1.1	Introduction	1787

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1787

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1788

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1788

B.2.1	Introduction	1788

B.2.1.1	General	1788

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1789

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1789

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1789

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1790

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1790

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1790

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1790

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1791

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1791

B.2.2	Conditions for NR intra-frequency measurements	1792

B.2.3	Conditions for NR inter-frequency measurements	1793

B.2.4	Conditions for NR L1-RSRP reporting	1794

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1794

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1795

B.2.5	Conditions for RRC connection release with redirection to NR	1796

B.2.6	Void	1797

B.2.6.1	Void	1797

B.2.6.2	Void	1797

B.2.8	Conditions for NR L1-SINR reporting	1798

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1798

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1799

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1799

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1800

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1801

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1801

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1802

B.2.9	Conditions for NR intra-frequency measurements under CCA	1804

B.2.10	Conditions for NR inter-frequency measurements under CCA	1804

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1804

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1804

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1805

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1806

B.2.14	Conditions for NR PRS-based measurements	1807

B.3	RRM Requirements Exceptions	1809

B.3.1	Introduction	1809

B.3.2	Receiver sensitivity relaxation for CA	1809

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1809

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1809

B.3.2.2.1	Inter-band carrier aggregation	1809

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1809

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1809

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1810

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1810

B.3.2.4.1	Intra-band contiguous carrier aggregation	1810

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1810

B.3.3	Receiver sensitivity relaxation for DC	1810

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1810

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1810

B.3.4	Receiver sensitivity relaxation for SUL	1810

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1810

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1810

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1810

B.4	Conditions for V2X	1811

B.4.1	Test parameters for GNSS signals	1811

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1811

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1811

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1811

Annex C (informative): Change history	1813