| 3GPP TS 38.133 V18.0.0 (2022-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	82

1	Scope	84

2	References	84

3	Definitions, symbols and abbreviations	85

3.1	Definitions	85

3.2	Symbols	86

3.3	Abbreviations	87

3.4	Test tolerances	90

3.5	Frequency bands grouping	90

3.5.1	Introduction	90

3.5.2	NR operating bands in FR1	90

3.5.2A	NR operating bands for satellite access in FR1	91

3.5.3	NR operating bands in FR2	91

3.6	Applicability of requirements in this specification version	92

3.6.1	RRC connected state requirements in DRX	93

3.6.2	Number of serving carriers	93

3.6.2.1	Number of serving carriers for SA	93

3.6.2.2	Number of serving carriers for EN-DC	93

3.6.2.3	Number of serving carriers for NE-DC	93

3.6.2.4	Number of serving carriers for NR-DC	94

3.6.3	Applicability for intra-band FR2	94

3.6.4	Applicability for FR2 UE power classes	94

3.6.5	Applicability for SDL bands	94

3.6.6	Applicability of requirements for NGEN-DC operation	94

3.6.7	Applicability of QCL	94

3.6.9	Applicability of requirements for scheduling availability	95

3.6.10	Applicability of requirements for measurement restrictions	95

3.6.11	Applicability of requirements for Redcap UEs	95

3.6.11.1	RRC connected state requirements in DRX	95

3.6.11.2	Applicability for FR2 Redcap UE power classes	95

3.6.11.3	Applicability of QCL	95

3.6.12	Applicability of requirements for Satellite Access	96

3.6.13	Applicability of requirements for FR2	96

3.6.14	Applicability of requirements for FR2 Power Class 6	96

4	SA: RRC_IDLE state mobility	96

4.1	Cell Selection	96

4.2	Cell Re-selection	96

4.2.1	Introduction	96

4.2.2	Requirements	97

4.2.2.1	UE measurement capability	97

4.2.2.2	Measurement and evaluation of serving cell	97

4.2.2.3	Measurements of intra-frequency NR cells	97

4.2.2.4	Measurements of inter-frequency NR cells	99

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	102

4.2.2.6	Maximum interruption in paging reception	104

4.2.2.7	General requirements	104

4.2.2.8	Minimum requirement at transitions	104

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	105

4.2.2.9.1	Introduction	105

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	105

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	106

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	106

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	106

4.2.2.10.1	Introduction	106

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	107

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	108

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	109

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	109

4.2.2.11.1	Introduction	109

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	109

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	110

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	111

4.2A	Cell Re-selection when subject to CCA	112

4.2A.1	Introduction	112

4.2A.2	Requirements	112

4.2A.2.1	UE measurement capability	112

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	113

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	113

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	115

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	116

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	116

4.2A.2.7	General requirements	117

4.2B	Cell Re-selection for RedCap	117

4.2B.1	Introduction	117

4.2B.2	Requirements	117

4.2B.2.1	UE measurement capability for RedCap	117

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	117

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	117

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	117

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	119

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	122

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	125

4.2B.2.6	Maximum interruption in paging reception for RedCap	127

4.2B.2.7	General requirements for RedCap	127

4.2B.2.8	Minimum requirement at transitions	127

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	128

4.2B.2.9.1	Introduction	128

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	129

4.2B.2.9.3	Measurements for a UE fulfilling stationary and not-at-cell edge criteria	132

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criteria	132

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	132

4.2B.2.9.5	Measurements for a UE fulfilling low mobility criterion and stationary and not-at-cell edge criteria	133

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary and not-at-cell edge criteria	133

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary and notatcell edge criteria	133

4.2B.2.9.8	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary criterion	133

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	134

4.2B.2.10.1	Introduction	134

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	134

4.2B.2.10.3	Measurements for a UE fulfilling stationary not at cell edge criterion	137

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	137

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	137

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	138

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	138

4.2B.2.10.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	138

4.2B.2.10.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	138

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	139

4.2B.2.11.1	Introduction	139

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	139

4.2B.2.11.3	Measurements for a UE fulfilling stationary not at cell edge criterion	140

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	141

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	141

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	141

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	142

4.2B.2.11.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	142

4.2B.2.11.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	142

4.2C	Cell Re-selection for NR UE for Satellite Access	142

4.2C.1	Introduction	142

4.2C.2	Requirements	143

4.2C.2.1	UE measurement capability	143

4.2C.2.2	Measurement and evaluation of serving cell	143

4.2C.2.3	Measurements of intra-frequency NR cells	144

4.2C.2.4	Measurements of inter-frequency NR cells	146

4.2C.2.5	Maximum interruption in paging reception	149

4.2C.2.6	Minimum requirement at transitions	149

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	149

4.2C.2.8	General requirements	149

4.3	Minimization of Drive Tests (MDT)	149

4.3.1	Introduction	149

4.3.2	Measurement Requirements	150

4.3.3	Requirements for Relative Time Stamp Accuracy	150

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	150

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	151

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	151

4.3C.1	Introduction	151

4.3C.2	Measurement Requirements	152

4.3C.3	Requirements for Relative Time Stamp Accuracy	152

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	152

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	152

4.4	Idle Mode CA/DC Measurements	153

4.4.1	Introduction	153

4.4.2	Measurement Requirements	153

4.4.2.1	Detected cell requirement during state transition and Idle mode	153

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	154

4.4.2.3	Measurements on serving cell	155

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	155

5	SA: RRC_INACTIVE state mobility	157

5.1	Cell Re-selection	157

5.1.1	Introduction	157

5.1.2	Requirements	157

5.1.2.1	UE measurement capability	157

5.1.2.2	Measurement and evaluation of serving cell	157

5.1.2.3	Measurements of intra-frequency NR cells	157

5.1.2.4	Measurements of inter-frequency NR cells	157

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	157

5.1.2.6	Maximum interruption in paging reception	157

5.1.2.7	General requirements	158

5.1A	Cell Re-selection with CCA	158

5.1A.1	Introduction	158

5.1A.2	Requirements	158

5.1A.2.1	UE measurement capability	158

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	158

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	158

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	158

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	158

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	158

5.1A.2.7	General requirements	158

5.1B	Cell Re-selection for RedCap	159

5.1B.1	Introduction	159

5.1B.2	Requirements	159

5.1B.2.1	UE measurement capability	159

5.1B.2.2	Measurement and evaluation of serving cell	159

5.1B.2.3	Measurements of intra-frequency NR cells	160

5.1B.2.4	Measurements of inter-frequency NR cells	160

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	161

5.1B.2.6	Maximum interruption in paging reception	161

5.1B.2.7	General requirements	161

5.1B.2.8	Minimum requirement at transitions	161

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	161

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	162

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	162

5.2	Void	162

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	162

5.2B.1	Introduction	162

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	162

5.2B.2.2	Scheduling restriction	163

5.3	Minimization of Drive Tests (MDT)	163

5.3.1	Introduction	163

5.3.2	Measurement Requirements	164

5.3.3	Requirements for Relative Time Stamp Accuracy	164

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	164

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	164

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	164

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	164

5.3C.1	Introduction	164

5.3C.2	Measurement Requirements	165

5.3C.3	Requirements for Relative Time Stamp Accuracy	165

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	165

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	165

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	165

5.4	Idle Mode CA/DC Measurements	165

5.4.1	Introduction	165

5.4.2	Measurement Requirements	165

5.4.2.1	Detected cell requirement during state transition and Idle mode	165

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	166

5.4.2.3	Measurements on serving cell	166

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	166

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	166

5.5.1	Introduction	166

5.5.2	Requirements on UE synchronization for small data transmissions	166

5.5.3	TA validation requirements	166

5.5.4	Scheduling restriction	167

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	167

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	167

5.5.4.3	Scheduling availability of UE performing measurements on FR2	168

5.5.5	Applicability conditions for SDT	168

5.6	NR measurements for positioning	168

5.6.1	Introduction	168

5.6.2	RSTD measurements	169

5.6.2.1	Introduction	169

5.6.2.2	Requirements Applicability	169

5.6.2.3	Measurement Capability	169

5.6.2.5	Measurements Period Requirements	170

5.6.3	PRS-RSRP measurements	173

5.6.3.1	Introduction	173

5.6.3.2	Requirements applicability	173

5.6.3.3	Measurement Capability	173

5.6.3.4	Measurement Reporting Requirements	173

5.6.3.5	Measurement Period Requirements	173

5.6.4	UE Rx-Tx time difference measurements	176

5.6.4.1	Introduction	176

5.6.4.2	Requirements Applicability	176

5.6.4.3	Measurement Capability	176

5.6.4.4	Measurement Reporting Requirements	176

5.6.4.5	Measurement Period Requirements	176

5.6.5	PRS-RSRPP measurements	179

5.6.5.1	Introduction	179

5.6.5.2	Requirements applicability	179

5.6.5.3	Measurement capability	179

5.6.5.4	Measurement reporting requirements	179

5.6.5.5	Measurement period requirements	180

5.7	Random access based Small Data Transmissions (RA-SDT)	180

5.7.1	Introduction	180

5.7.2	Requirements for small data transmissions based on 2-step RA	180

5.7.3	Requirements for small data transmissions based on 4-step RA	180

6	RRC_CONNECTED state mobility	180

6.1	Handover	180

6.1.1	NR Handover	180

6.1.1.1	Introduction	180

6.1.1.2	NR FR1 - NR FR1 Handover	180

6.1.1.2.1	Handover delay	180

6.1.1.2.2	Interruption time	181

6.1.1.3	NR FR2- NR FR1 Handover	181

6.1.1.3.1	Handover delay	181

6.1.1.3.2	Interruption time	181

6.1.1.4	NR FR2- NR FR2 Handover	182

6.1.1.4.1	Handover delay	182

6.1.1.4.2	Interruption time	182

6.1.1.5	NR FR1- NR FR2 Handover	183

6.1.1.5.1	Handover delay	183

6.1.1.5.2	Interruption time	183

6.1.2	NR Handover to other RATs	184

6.1.2.1	NR – E-UTRAN Handover	184

6.1.2.1.1	Introduction	184

6.1.2.1.2	Handover delay	184

6.1.2.1.3	Interruption time	185

6.1.2.2	NR – UTRAN Handover	185

6.1.2.2.1	Introduction	185

6.1.2.2.2	Handover delay	185

6.1.2.2.3	Interruption time	185

6.1.3	NR DAPS Handover	186

6.1.3.1	Introduction	186

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	186

6.1.3.2.1	DAPS handover delay	187

6.1.3.2.2	Interruption time	187

6.1.3.3	NR FR2- NR FR1 DAPS Handover	189

6.1.3.3.1	DAPS handover delay	189

6.1.3.3.2	Interruption time	190

6.1.3.4	NR FR1- NR FR2 DAPS Handover	190

6.1.3.4.1	DAPS handover delay	191

6.1.3.4.2	Interruption time	191

6.1.4	NR Conditional Handover	191

6.1.4.1	Introduction	191

6.1.4.2	NR FR1 – NR FR1 conditional handover	192

6.1.4.3	NR FR2 – NR FR1 conditional handover	193

6.1.4.4	NR FR2 – NR FR2 conditional handover	193

6.1.4.4.1	Handover delay	193

6.1.4.4.2	Measurement time	193

6.1.4.4.3	Preparation time	194

6.1.4.4.4	Interruption time	194

6.1.4.5	NR FR1 – NR FR2 conditional handover	194

6.1.5	NR Handover with PSCell	195

6.1.5.1	Introduction	195

6.1.5.2	Handover with PSCell from NR SA to EN-DC	195

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	195

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	195

6.1.5.3	HO with PSCell from NE-DC to NE-DC	196

6.1.5.3.1	Handover delay	196

6.1.5.3.2	HO with PSCell - PCell Interruption time	196

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	197

6.1.5.4	HO with PSCell from NR-DC to NR-DC	197

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	198

6.1.5.5.1	Introduction	198

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	198

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	199

6.1A	Void	200

6.1A.1	Void	200

6.1A.1.1	Void	200

6.1A.1.2	Void	200

6.1A.1.2.1	Void	200

6.1A.1.2.2	Void	200

6.1B	Handover to target cell using CCA	200

6.1B.1	NR Handover	200

6.1B.1.1	Introduction	200

6.1B.1.2	NR FR1 - NR FR1 Handover	200

6.1B.1.2.1	Handover delay	200

6.1B.1.2.2	Interruption time	201

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	201

6.1B.1.3.1	Handover delay	201

6.1B.1.3.2	Interruption time	202

6.1B.1.4	NR FR1- NR FR2-2 Handover	202

6.1B.1.4.1	Handover delay	203

6.1B.1.4.2	Interruption time	203

6.1C	Handover for SAN	204

6.1C.1	NR SAN Handover	204

6.1C.1.1	Introduction	204

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	204

6.1C.1.2.1	Handover delay	204

6.1C.1.2.2	Interruption time	204

6.1C.2	NR SAN Conditional Handover	205

6.1C.2.1	Introduction	205

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	205

6.1C.2.2.1	Handover delay	205

6.1C.2.2.2	Measurement time	206

6.1C.2.2.3	Preparation time	207

6.1C.2.2.4	Interruption time	207

6.1D	Handover for RedCap	207

6.1D.1	NR Handover	207

6.1D.1.1	Introduction	207

6.1D.1.2	NR FR1 - NR FR1 Handover	207

6.1D.1.3	NR FR2- NR FR2 Handover	208

6.1D.2	NR Handover to other RATs	208

6.1D.2.1	NR – E-UTRAN Handover	208

6.2	RRC Connection Mobility Control	208

6.2.1	SA: RRC Re-establishment	208

6.2.1.1	Introduction	208

6.2.1.2	Requirements	209

6.2.1.2.1	UE Re-establishment delay requirement	209

6.2.1A	RRC Re-establishment with CCA	210

6.2.1A.1	Introduction	210

6.2.1A.2	Requirements	211

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	211

6.2.1B	SA: RRC Re-establishment for RedCap	213

6.2.1B.1	Introduction	213

6.2.1B.2	Requirements	213

6.2.2	Random access	213

6.2.2.1	Introduction	213

6.2.2.2	Requirements for 4-step RA type	213

6.2.2.2.1	Contention based random access	214

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	214

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	214

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	214

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	214

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	214

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	214

6.2.2.2.2	Non-Contention based random access	215

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	215

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	215

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	215

6.2.2.2.3	UE behaviour when configured with supplementary UL	216

6.2.2.3	Requirements for 2-step RA type	216

6.2.2.3.1	Contention based random access	216

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	216

6.2.2.3.1.2	Correct behaviour when receiving MsgB	216

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	217

6.2.2.3.2	Non-Contention based random access	217

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	217

6.2.2.3.2.2	Correct behaviour when receiving MsgB	217

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	217

6.2.2.3.3	UE behaviour when configured with supplementary UL	217

6.2.2A	Random access when CCA is used on target frequency	218

6.2.2A.1	Introduction	218

6.2.2A.2	Requirements for 4-step RA type	218

6.2.2A.2.1	Contention based random access	218

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	218

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	219

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	219

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	219

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	219

6.2.2A.2.2	Non-Contention based random access	219

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	219

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	220

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	220

6.2.2A.3	Requirements for 2-step RA type	220

6.2.2A.3.1	Contention based random access	221

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	221

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	221

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	221

6.2.2A.3.2	Non-Contention based random access	222

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	222

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	222

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	222

6.2.2B	Random access for RedCap	222

6.2.2B.1	Introduction	222

6.2.2B.2	Requirements	223

6.2.3	SA: RRC Connection Release with Redirection	223

6.2.3.1	Introduction	223

6.2.3.2	Requirements	223

6.2.3.2.1	RRC connection release with redirection to NR	223

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	224

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	225

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	226

6.2.3A.1	Introduction	226

6.2.3A.2	Requirements	226

6.2.3A.2.1	RRC connection release with redirection to NR	226

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	226

6.2C	RRC Connection Mobility Control for Satellite Access	226

6.2C.1	SA: RRC Re-establishment for Satellite Access	226

6.2C.1.1	Introduction	226

6.2C.1.2	Requirements	227

6.2C.1.2.1	UE Re-establishment delay requirement	227

6.2C.2	Random access for satellite access	228

6.2C.2.1	Introduction	228

6.2C.2.2	Requirements for 4-step RA type	228

6.2C.2.2.1	Contention based random access	229

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	229

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	229

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	229

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	229

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	229

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	229

6.2C.2.2.2	Non-Contention based random access	229

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	229

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	230

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	230

6.2C.2.3	Requirements for 2-step RA type	230

6.2C.2.3.1	Contention based random access	231

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	231

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	231

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	231

6.2C.2.3.2	Non-Contention based random access	231

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	231

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	232

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	232

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	232

6.2C.3.1	Introduction	232

6.2C.3.2	Requirements	232

6.2C.3.2.1	RRC connection release with redirection to NR	232

7	Timing	233

7.1	UE transmit timing	233

7.1.1	Introduction	233

7.1.2	Requirements	233

7.1.2.1	Gradual timing adjustment	235

7.1.2.2	Void	236

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	236

7.1A	UE transmit timing for RedCap	236

7.1A.1	Introduction	236

7.1A.2	Requirements	236

7.1A.2.1	Gradual timing adjustment	238

7.1C	UE transmit timing for Satellite Access	238

7.1C.1	Introduction	238

7.1C.2	Requirements	238

7.1C.2.1	Gradual timing adjustment	239

7.2	UE timer accuracy	239

7.2.1	Introduction	239

7.2.2	Requirements	240

7.2A	UE timer accuracy for RedCap	240

7.2A.1	Introduction	240

7.2A.2	Requirements	240

7.2C	UE timer accuracy for satellite access	240

7.2C.1	Introduction	240

7.2C.2	Requirements	241

7.3	Timing advance	241

7.3.1	Introduction	241

7.3.2	Requirements	241

7.3.2.1	Timing Advance adjustment delay	241

7.3.2.2	Timing Advance adjustment accuracy	241

7.3A	Timing Advance for RedCap	241

7.3A.1	Introduction	241

7.3A.2	Requirements	242

7.3A.2.1	Timing Advance adjustment delay	242

7.3A.2.2	Timing Advance adjustment accuracy	242

7.3C	Timing advance for satellite access	242

7.3C.1	Introduction	242

7.3C.2	Requirements	242

7.3C.2.1	Timing Advance adjustment delay	242

7.3C.2.2	Timing Advance adjustment accuracy	242

7.4	Cell phase synchronization accuracy	243

7.4.1	Definition	243

7.4.2	Minimum requirements	243

7.5	Maximum Transmission Timing Difference	243

7.5.1	Introduction	243

7.5.2	Minimum Requirements for inter-band EN-DC	243

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	244

7.5.3	Minimum Requirements for intra-band EN-DC	244

7.5.4	Minimum Requirements for NR Carrier Aggregation	245

7.5.5	Minimum Requirements for inter-band NE-DC	245

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	246

7.5.6	Minimum Requirements for inter-band NR DC	246

7.6	Maximum Receive Timing Difference	247

7.6.1	Introduction	247

7.6.2	Minimum Requirements for inter-band EN-DC	247

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	248

7.6.3	Minimum Requirements for intra-band EN-DC	248

7.6.4	Minimum Requirements for NR Carrier Aggregation	248

7.6.5	Minimum Requirements for inter-band NE-DC	249

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	250

7.6.6	Minimum Requirements for inter-band NR DC	250

7.7	deriveSSB-IndexFromCell tolerance	251

7.7.1	Minimum requirements	251

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	251

7.7A.1	Minimum requirements	251

7.8	Void	252

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	252

7.9.1	Minimum requirements	252

8	Signalling characteristics	252

8.1	Radio Link Monitoring	252

8.1.1	Introduction	252

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	253

8.1.2	Requirements for SSB based radio link monitoring	254

8.1.2.1	Introduction	254

8.1.2.2	Minimum requirement	255

8.1.2.3	Measurement restrictions for SSB based RLM	258

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	258

8.1.3	Requirements for CSI-RS based radio link monitoring	259

8.1.3.1	Introduction	259

8.1.3.2	Minimum requirement	260

8.1.3.3	Measurement restrictions for CSI-RS based RLM	263

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	264

8.1.4	Minimum requirement at transitions	265

8.1.5	Minimum requirement for UE turning off the transmitter	265

8.1.6	Minimum requirement for L1 indication	265

8.1.7	Scheduling availability of UE during radio link monitoring	266

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	266

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	266

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	266

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	267

8.1A	Radio Link Monitoring with CCA on Target Frequency	267

8.1A.1	Introduction	267

8.1A.2	Requirements for SSB Based Radio Link Monitoring	268

8.1A.2.1	Introduction	268

8.1A.2.2	Minimum Requirement	269

8.1A.3	Minimum requirement at transitions	272

8.1A.4	Minimum requirement for UE turning off the transmitter	272

8.1A.5	Minimum requirement for L1 indication	272

8.1A.6	Scheduling availability of UE during radio link monitoring	273

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	273

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	274

8.1B	Radio Link Monitoring for RedCap	274

8.1B.1	Introduction	274

8.1B.2	Requirements for SSB based radio link monitoring	275

8.1B.2.1	Introduction	275

8.1B.2.2	Minimum requirement	276

8.1B.2.3	Measurement restrictions for SSB based RLM	278

8.1B.3	Requirements for CSI-RS based radio link monitoring	278

8.1B.3.1	Introduction	278

8.1B.3.2	Minimum requirement	279

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	282

8.1B.4	Minimum requirement at transitions	282

8.1B.5	Minimum requirement for UE turning off the transmitter	283

8.1B.6	Minimum requirement for L1 indication	283

8.1B.7	Scheduling availability of UE during radio link monitoring	283

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	283

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	283

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	284

8.1C	Radio Link Monitoring for Satellite Access	284

8.1C.1	Introduction	284

8.1C.2	Requirements for SSB based radio link monitoring	285

8.1C.2.1	Introduction	285

8.1C.2.2	Minimum requirement	286

8.1C.2.3	Measurement restrictions for SSB based RLM	286

8.1C.3	Requirements for CSI-RS based radio link monitoring	287

8.1C.3.1	Introduction	287

8.1C.3.2	Minimum requirement	288

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	288

8.1C.4	Minimum requirement at transitions	289

8.1C.5	Minimum requirement for UE turning off the transmitter	289

8.1C.6	Minimum requirement for L1 indication	289

8.1C.7	Scheduling availability of UE during radio link monitoring	289

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	290

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	290

8.2	Interruption	290

8.2.1	EN-DC Interruption	290

8.2.1.1	Introduction	290

8.2.1.2	Requirements	291

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	291

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	291

8.2.1.2.3	Interruptions at SCell addition/release	291

8.2.1.2.4	Interruptions at SCell activation/deactivation	292

8.2.1.2.5	Interruptions during measurements on SCC	294

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	294

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	294

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	294

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	294

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	295

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	295

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	296

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	296

8.2.1.2.8.2	Interruptions during direct SCell activation	297

8.2.1.2.9	Interruptions at SCell hibernation	297

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	297

8.2.1.2.11	Interruptions due to UE-specific CBW change	297

8.2.1.2.12	Interruptions at NR SRS carrier based switching	297

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	299

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	300

8.2.1.2.15	Interruptions due to SCell dormancy	300

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	300

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	300

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	301

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	301

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	301

8.2.1.2.18	Interruptions at NR SRS antenna port switching	302

8.2.1.2.19	Interruptions at fast SCell activation	303

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	304

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	304

8.2.2.1	Introduction	304

8.2.2.2	Requirements	305

8.2.2.2.1	Interruptions at SCell addition/release	305

8.2.2.2.2	Interruptions at SCell activation/deactivation	306

8.2.2.2.3	Interruptions during measurements on deactivated SCC	307

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	308

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	308

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	309

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	311

8.2.2.2.8	Interruptions due to UE-specific CBW change	311

8.2.2.2.9	Interruptions at NR SRS carrier based switching	311

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	313

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	313

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	314

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	314

8.2.2.2.11	Interruptions at direct SCell activation	314

8.2.2.2.12	Interruptions due to SCell dormancy	315

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	315

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	315

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	315

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	315

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	315

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	316

8.2.2.2.16	Interruptions at NR SRS antenna port switching	316

8.2.2.2.17	Interruptions at fast SCell activation	317

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	318

8.2.3	NE-DC Interruptions	318

8.2.3.1	Introduction	318

8.2.3.2	Requirements	319

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	319

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	319

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	319

8.2.3.2.4	Interruptions at SCell activation/deactivation	321

8.2.3.2.5	Interruptions during measurements on SCC	322

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	322

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	322

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	322

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	323

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	323

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	323

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	324

8.2.3.2.9	Interruptions at SCell hibernation	324

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	324

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	324

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	326

8.2.3.2.13	Interruptions due to SCell dormancy	327

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	327

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	328

8.2.3.2.17	Interruptions at fast SCell activation	329

8.2.3.2.18	Interruptions due to UE-specific CBW change	330

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	330

8.2.4	NR-DC: Interruptions	330

8.2.4.1	Introduction	330

8.2.4.2	Requirements	331

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	331

8.2.4.2.2	Interruptions at SCell activation/deactivation	332

8.2.4.2.3	Interruptions during measurements on SCC	333

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	334

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	334

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	334

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	335

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	335

8.2.4.2.9	Interruptions at NR SRS carrier based switching	335

8.2.4.2.10	Interruptions at direct SCell activation	337

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	337

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	338

8.2.4.2.13	 Interruptions due to SCell dormancy	338

8.2.4.2.14	Interruptions at NR SRS antenna port switching	339

8.2.4.2.15	Interruptions at fast SCell activation	340

8.2.4.2.16	Interruptions at SCG activation/deactivation	340

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	341

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	341

8.2.4.2.19	Interruptions due to UE-specific CBW change	341

8.2.4.2A	Void	342

8.2.4.2A.1	Void	342

8.2.4.2A.2	Void	342

8.2.4.2A.3	Void	342

8.3	SCell Activation and Deactivation Delay	342

8.3.1	Introduction	342

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	342

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	346

8.3.4	Direct SCell Activation at SCell addition	346

8.3.5	Direct SCell Activation at Handover	348

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	350

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	354

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	354

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	355

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	357

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	359

8.3.13.1	Introduction	359

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	361

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	362

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	362

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	365

8.3A.1	Introduction	365

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	365

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	369

8.4	UE UL carrier RRC reconfiguration delay	370

8.4.1	Introduction	370

8.4.2	UE UL carrier configuration delay requirement	370

8.4.3	UE UL carrier deconfiguration delay requirement	370

8.5	Link Recovery Procedures	370

8.5.1	Introduction	370

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	371

8.5.2	Requirements for SSB based beam failure detection	372

8.5.2.1	Introduction	372

8.5.2.2	Minimum requirement	373

8.5.2.3	Measurement restriction for SSB based beam failure detection	376

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	376

8.5.3	Requirements for CSI-RS based beam failure detection	377

8.5.3.1	Introduction	377

8.5.3.2	Minimum requirement	378

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	381

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	382

8.5.4	Minimum requirement for L1 indication	383

8.5.5	Requirements for SSB based candidate beam detection	383

8.5.5.1	Introduction	383

8.5.5.2	Minimum requirement	383

8.5.5.3	Measurement restriction for SSB based candidate beam detection	386

8.5.6	Requirements for CSI-RS based candidate beam detection	387

8.5.6.1	Introduction	387

8.5.6.2	Minimum requirement	387

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	390

8.5.7	Scheduling availability of UE during beam failure detection	390

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	391

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	391

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	391

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	392

8.5.8	Scheduling availability of UE during candidate beam detection	392

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	392

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	392

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	392

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	393

8.5.9	Requirements for Beam Failure Recovery in SCell	393

8.5.9.1	Introduction	393

8.5.9.2	Requirement	393

8.5.10	Minimum requirement at transitions for beam failure detection	394

8.5A	Link Recovery Procedures when CCA is used on target frequency	394

8.5A.1	Introduction	394

8.5A.2	Requirements for SSB based beam failure detection	395

8.5A.2.1	Introduction	395

8.5A.2.2	Minimum requirement	395

8.5A.2.3	Measurement restriction for SSB based beam failure detection	397

8.5A.3	Void	398

8.5A.4	Minimum requirement for L1 indication	398

8.5A.5	Requirements for SSB based candidate beam detection	398

8.5A.5.1	Introduction	398

8.5A.5.2	Minimum requirement	398

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	400

8.5A.6	Void	401

8.5A.7	Scheduling availability of UE during beam failure detection	401

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	401

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	401

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	401

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	401

8.5A.8	Scheduling availability of UE during candidate beam detection	401

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	402

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	402

8.5B	Link Recovery Procedures for Redcap	402

8.5B.1	Introduction	402

8.5B.2	Requirements for SSB based beam failure detection for Redcap	402

8.5B.2.1	Introduction	402

8.5B.2.2	Minimum requirement	403

8.5B.2.3	Measurement restriction for SSB based beam failure detection	404

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	405

8.5B.3.1	Introduction	405

8.5B.3.2	Minimum requirement	406

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	408

8.5B.4	Minimum requirement for L1 indication for Redcap	408

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	409

8.5B.5.1	Introduction	409

8.5B.5.2	Minimum requirement	409

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	411

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	411

8.5B.6.1	Introduction	411

8.5B.6.2	Minimum requirement	411

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	414

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	414

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	414

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	414

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	414

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	415

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	415

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	415

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	415

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	416

8.5C	Link Recovery Procedures for Satellite Access	416

8.5C.1	Introduction	416

8.5C.2	Requirements for SSB based beam failure detection	417

8.5C.2.1	Introduction	417

8.5C.2.2	Minimum requirement	417

8.5C.2.3	Measurement restriction for SSB based beam failure detection	418

8.5C.3	Requirements for CSI-RS based beam failure detection	418

8.5C.3.1	Introduction	418

8.5C.3.2	Minimum requirement	418

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	419

8.5C.4	Minimum requirement for L1 indication	420

8.5C.5	Requirements for SSB based candidate beam detection	420

8.5C.5.1	Introduction	420

8.5C.5.2	Minimum requirement	420

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	421

8.5C.6	Requirements for CSI-RS based candidate beam detection	421

8.5C.6.1	Introduction	421

8.5C.6.2	Minimum requirement	421

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	422

8.5C.7	Scheduling availability of UE during beam failure detection	422

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	423

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	423

8.5C.8	Scheduling availability of UE during candidate beam detection	423

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	423

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	423

8.5C.9	Minimum requirement at transitions for beam failure detection	423

8.6	Active BWP switch delay	424

8.6.1	Introduction	424

8.6.2	DCI and timer based BWP switch delay on a single CC	424

8.6.2A	DCI based BWP switch delay on multiple CCs	425

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	426

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	427

8.6.2B	Timer based BWP switch delay on multiple CCs	427

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	427

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	428

8.6.3	RRC based BWP switch delay on a single CC	429

8.6.3A	RRC based BWP switch delay on multiple CCs	429

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	429

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	430

8.6.4	BWP switch delay on Consistent UL CCA recovery	430

8.6A	Active BWP switch delay for RedCap	431

8.6A.1	Introduction	431

8.6A.2	DCI and timer based BWP switch delay on a single CC	431

8.6A.3	RRC based BWP switch delay on a single CC	432

8.6C	Active BWP switch delay for satellite access	433

8.6C.1	Introduction	433

8.6C.2	DCI and timer based BWP switch delay on a single CC	433

8.6C.3	RRC based BWP switch delay on a single CC	434

8.7	Void	435

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	435

8.8.1	Introduction	435

8.8.2	E-UTRAN PSCell Addition Delay Requirement	435

8.8.3	E-UTRAN PSCell Release Delay Requirement	436

8.9	NR-DC: PSCell Addition and Release Delay	436

8.9.1	Introduction	436

8.9.2	PSCell Addition Delay Requirement	436

8.9.3	PSCell Release Delay Requirement	437

8.9A	Conditional PSCell Addition Delay	437

8.9A.1	Introduction	437

8.9A.2	Conditional PSCell Addition Delay Requirement	437

8.9A.2.1	Measurement time	438

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	438

8.9B.1	Introduction	438

8.9B.2	PSCell Addition Delay Requirement	438

8.9B.3	PSCell Release Delay Requirement	439

8.10	Active TCI state switching delay	439

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	441

8.10.4	DCI based TCI state switch delay	441

8.10.5	RRC based TCI state switch delay	441

8.10.6	Active TCI state list update delay	442

8.10A	Active TCI state switching delay with CCA	442

8.10A.1	Introduction	442

8.10A.2	Known conditions for TCI state	442

8.10A.3	MAC-CE based TCI state switch delay	443

8.10A.4	DCI based TCI state switch delay	444

8.10A.5	RRC based TCI state switch delay	444

8.10A.6	Active TCI state list update delay	445

8.10B	Active TCI state switching delay for RedCap	445

8.10B.1	Introduction	445

8.10B.2	Known conditions for TCI state	445

8.10B.3	MAC-CE based TCI state switch delay	445

8.10B.4	DCI based TCI state switch delay	446

8.10B.5	RRC based TCI state switch delay	446

8.10B.6	Active TCI state list update delay	447

8.10C	Active TCI state switching delay for satellite access	447

8.10C.1	Introduction	447

8.10C.2	MAC-CE based TCI state switch delay	447

8.10C.4	DCI based TCI state switch delay	447

8.10C.5	RRC based TCI state switch delay	448

8.10C.6	Active TCI state list update delay	448

8.11	PSCell Change	448

8.11A	PSCell Change in Carriers with CCA	448

8.11B	Conditional PSCell Change	449

8.11B.1	Introduction	449

8.11B.2	Conditoinal PSCell Change delay	449

8.11B.2.1	Measurement time	450

8.11D	Conditional PSCell Change in Carriers with CCA	450

8.11D.1	Introduction	450

8.11D.2	Conditional PSCell Change delay	450

8.11D.2.1	Measurement time	451

8.12	Uplink spatial relation switch delay	452

8.12.1	Introduction	452

8.12.2	Known conditions for spatial relation when associated with DL-RS	452

8.12.3	MAC-CE based spatial relation switch delay	452

8.12.4	DCI based spatial relation switch delay	453

8.12.5	RRC based spatial relation switch delay	453

8.12A	Uplink spatial relation switch delay for RedCap	454

8.12A.1	Introduction	454

8.12A.2	Known conditions for spatial relation when associated with DL-RS	454

8.12A.3	MAC-CE based spatial relation switch delay	454

8.12A.4	DCI based spatial relation switch delay	455

8.12A.5	RRC based spatial relation switch delay	455

8.12C	Uplink spatial relation switch delay for satellite access	456

8.12C.1	Introduction	456

8.12C.2	Known conditions for spatial relation when associated with DL-RS	456

8.12C.3	MAC-CE based spatial relation switch delay	456

8.12C.4	DCI based spatial relation switch delay	457

8.12C.5	RRC based spatial relation switch delay	457

8.13	UE-specific CBW change	457

8.13.1	Introduction	457

8.13.2	UE-specific CBW change delay	457

8.13A	UE-specific CBW change for RedCap	458

8.13A.1	Introduction	458

8.13A.2	UE-specific CBW change delay	458

8.13C	UE-specific CBW change for satellite access	458

8.13C.1	Introduction	458

8.13C.2	UE-specific CBW change delay	459

8.14	Pathloss reference signal switching delay	459

8.14.1	Introduction	459

8.14.2	Known conditions for pathloss reference signal	459

8.14.3	MAC-CE based pathloss reference signal switch delay	459

8.14C	Pathloss reference signal switching delay for satellite access	460

8.14C.1	Introduction	460

8.14C.2	Known conditions for pathloss reference signal	460

8.14C.3	MAC-CE based pathloss reference signal switch delay	461

8.15	Active downlink TCI state switching delay for unified TCI	461

8.15.1	Introduction	461

8.15.4	DCI based downlink TCI state switch delay	463

8.15.5	Active Downlink TCI state list update delay	463

8.16	Active uplink TCI state switching delay for unified TCI	464

8.16.1	Introduction	464

8.16.4	DCI based uplink TCI state switch delay	466

8.16.5	Active Uplink TCI state list update delay	466

8.17	SCG Activation and Deactivation Delay	467

8.17.1	Introduction	467

8.17.2	SCG Activation Delay Requirement	467

8.17.3	SCG Deactivation Delay Requirement	468

8.18	TRP specific Link Recovery Procedures	469

8.18.1	Introduction	469

8.18.2	Requirements for TRP specific SSB based beam failure detection	469

8.18.2.1	Introduction	469

8.18.2.2	Minimum requirement	470

8.18.2.3	Measurement restriction for SSB based beam failure detection	472

8.18.3	Requirements for CSI-RS based beam failure detection	472

8.18.3.1	Introduction	472

8.18.3.2	Minimum requirement	473

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	475

8.18.4	Minimum requirement for L1 indication	476

8.18.5	Requirements for SSB based candidate beam detection	476

8.18.5.1	Introduction	476

8.18.5.2	Minimum requirement	476

8.18.5.3	Measurement restriction for SSB based candidate beam detection	478

8.18.6	Requirements for CSI-RS based candidate beam detection	478

8.18.6.1	Introduction	478

8.18.6.2	Minimum requirement	479

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	481

8.18.7	Requirements for TRP specific Beam Failure Recovery	482

8.18.7.1	Introduction	482

8.18.7.2	Requirement	482

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	482

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	482

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	482

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	483

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	483

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	484

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	484

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	484

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	484

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	485

8.19	Pre-configured measurement gap activation/deactivation delay	485

8.19.1	Introduction	485

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	485

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	485

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	485

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	485

9	Measurement Procedure	486

9.1	General measurement requirement	486

9.1.1	Introduction	486

9.1.2	Measurement gap	486

9.1.2.1	EN-DC: Measurement Gap Sharing	498

9.1.2.1a	SA: Measurement Gap Sharing	499

9.1.2.1b	NE-DC: Measurement Gap Sharing	500

9.1.2.1c	NR-DC: Measurement Gap Sharing	501

9.1.3	UE Measurement capability	502

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	502

9.1.3.1a	SA: Monitoring of multiple layers using gaps	502

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	503

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	503

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	504

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	505

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	505

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	506

9.1A.3.2	Void	507

9.1.3A	UE Measurement capability under operation mode with CCA	507

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	507

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	507

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	507

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	508

9.1.3C	UE Measurement capability under operation mode with satellite access	508

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	508

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	509

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	509

9.1.4.1	Introduction	509

9.1.4.2	Requirements	509

9.1.5	Carrier-specific scaling factor	513

9.1.5.1	Monitoring of multiple layers outside gaps	514

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	516

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	517

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	518

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	519

9.1.5.2	Monitoring of multiple layers within gaps	520

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	522

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	523

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	525

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	527

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	529

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	529

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	529

9.1.5.3	Monitoring of multiple layers within NCSG	529

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	530

9.1.6	Minimum requirement at transitions	531

9.1.7	Pre-configured measurement gap	531

9.1.7.1	Introduction	531

9.1.7.2	Requirements applicability	531

9.1.7.3	Requirements	532

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	532

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	533

9.1.8	Concurrent measurement gaps	533

9.1.8.1	Introduction	533

9.1.8.2	Requirements	533

9.1.8.3	Collision between concurrent measurement gaps	535

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	535

9.1.9	Network controlled small gap	535

9.1.9.1	Introduction	535

9.1.9.2	Requirements applicability	536

9.1.10	MUSIM gaps	539

9.1.11	UL gap for Tx power management	540

9.1A	General measurement requirement for RedCap	541

9.1A.1	Introduction	541

9.1A.2	Measurement gap	541

9.1A.2.1	SA: Measurement Gap Sharing	545

9.1A.3	UE Measurement capability	546

9.1A.3.1	SA: Monitoring of multiple layers using gaps	546

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	546

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	546

9.1A.4.1	Introduction	546

9.1A.4.2	Requirements	547

9.1A.5	Carrier-specific scaling factor	547

9.1A.5.1	Monitoring of multiple layers outside gaps	547

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	548

9.1A.5.2	Monitoring of multiple layers within gaps	548

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	548

9.1A.6	Minimum requirement at transitions	549

9.1C	General measurement requirement	550

9.1C.1	Introduction	550

9.1C.2	Measurement gap	550

9.1C.8	Concurrent measurement gaps for SAN	552

9.1C.8.1	Introduction	552

9.1C.8.2	Requirements	552

9.1C.8.3	Collision between concurrent measurement gaps	553

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	553

9.2	NR intra-frequency measurements	554

9.2.1	Introduction	554

9.2.2	Requirements applicability	555

9.2.3	Number of cells and number of SSB	555

9.2.3.1	Requirements for FR1	555

9.2.3.2	Requirements for FR2	555

9.2.4	Measurement Reporting Requirements	556

9.2.4.1	Periodic Reporting	556

9.2.4.2	Event-triggered Periodic Reporting	556

9.2.4.3	Event Triggered Reporting	556

9.2.5	Intrafrequency measurements without measurement gaps	557

9.2.5.1	Intrafrequency cell identification	557

9.2.5.2	Measurement period	562

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	565

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	565

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	565

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	566

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	567

9.2.5.4	SFTD Measurements between PCell and PSCell	568

9.2.5.4.1	Introduction	568

9.2.5.4.2	SFTD Measurement delay	568

9.2.5.4.3	SFTD Measurement Reporting Delay	568

9.2.6	Intra-frequency measurements with measurement gaps	569

9.2.6.1	Void	569

9.2.6.2	Intra-frequency cell identification	569

9.2.6.3	Intrafrequency Measurement Period	571

9.2.7	Intra-frequency measurements with NCSG	573

9.2.7.1	Intra-frequency cell identification	573

9.2.7.2	Measurement period	575

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	576

9.2A	NR intra-frequency measurements with CCA	576

9.2A.1	Introduction	576

9.2A.2	Requirements applicability	577

9.2A.3	Number of cells and number of SSB	577

9.2A.3.1	Requirements for FR1	577

9.2A.3.2	Requirements for FR2-2	577

9.2A.4	Measurement Reporting Requirements	578

9.2A.5	Intra-frequency measurements without measurement gaps	579

9.2A.5.2	Measurement period	583

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	585

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	586

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	586

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	586

9.2A.6	Intra-frequency measurements with measurement gaps	586

9.2A.6.1	Intra-frequency cell identification	586

9.2A.6.2	Intra-frequency Measurement Period	589

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	590

9.2A.7.1	Intra-frequency RSSI measurements	590

9.2A.7.2	Intra-frequency Channel occupancy measurements	592

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	593

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	593

9.2B	NR intra-frequency measurements for RedCap	594

9.2B.1	Introduction	594

9.2B.2	Requirements applicability	594

9.2B.3	Number of cells and number of SSB	595

9.2B.3.1	Requirements for FR1	595

9.2B.3.2	Requirements for FR2	595

9.2B.4	Measurement Reporting Requirements	595

9.2B.4.1	Periodic Reporting	595

9.2B.4.2	Event-triggered Periodic Reporting	595

9.2B.4.3	Event Triggered Reporting	595

9.2B.5	Intra-frequency measurements without measurement gaps for RedCap	596

9.2B.5.1	Intra-frequency cell identification	596

9.2B.5.2	Measurement period	598

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	599

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	599

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	600

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	600

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	601

9.2B.6	Intra-frequency measurements with measurement gaps	601

9.2B.6.1	Intra-frequency cell identification	601

9.2B.6.2	Intra-frequency Measurement Period	602

9.2C	NR intra-frequency measurements for SAN	603

9.2C.1	Introduction	603

9.2C.2	Requirements applicability	604

9.2C.3	Number of cells and number of SSB	604

9.2C.3.1	Requirements for FR1	604

9.2C.4	Measurement Reporting Requirements	604

9.2C.4.1	Periodic Reporting	604

9.2C.4.2	Event-triggered Periodic Reporting	605

9.2C.4.3	Event Triggered Reporting	605

9.2C.5	Intrafrequency measurements without measurement gaps	605

9.2C.5.1	Intrafrequency cell identification	605

9.2C.5.2	Measurement period	607

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	607

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	608

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	608

9.2C.6	Intra-frequency measurements with measurement gaps	608

9.2C.6.1	Intra-frequency cell identification	608

9.2C.6.3	Intrafrequency Measurement Period	609

9.3	NR inter-frequency measurements	610

9.3.1	Introduction	610

9.3.2	Requirements applicability	611

9.3.2.1	Void	611

9.3.2.2	Void	611

9.3.3	Number of cells and number of SSB	611

9.3.3.1	Requirements for FR1	611

9.3.3.2	Requirements for FR2	611

9.3.4	Inter-frequency measurement with measurement gaps	612

9.3.4.1	Void	614

9.3.4.2	Void	614

9.3.5	Inter-frequency measurements	614

9.3.5.1	Void	615

9.3.5.2	Void	615

9.3.5.3	Void	615

9.3.6	Inter-frequency measurements reporting requirements	615

9.3.6.1	Periodic Reporting	615

9.3.6.2	Event-triggered Periodic Reporting	616

9.3.6.3	Event-triggered Reporting	616

9.3.7	Void	616

9.3.8	Inter-frequency SFTD measurement requirements	616

9.3.8.1	Introduction	616

9.3.8.2	SFTD Measurement delay	616

9.3.8.3	SFTD Measurement reporting delay	617

9.3.9	Inter frequency measurements without measurement gaps	618

9.3.9.1	Inter frequency Cell identification	618

9.3.9.2	Measurement period	620

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	621

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	621

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	621

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	622

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	622

9.3.10	Inter-frequency measurement with NCSG	622

9.3.10.1	Inter-frequency cell identification	622

9.3.10.2	Measurement period	624

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	624

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	625

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	625

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	626

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	628

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	628

9.3A.1	Introduction	628

9.3A.2	Requirements applicability	629

9.3A.3	Number of cells and number of SSB	629

9.3A.3.1	Requirements for FR1	629

9.3A.3.2	Requirements for FR2-2	629

9.3A.4	Inter-frequency cell identification	629

9.3A.5	Inter-frequency measurements	632

9.3A.6	NR Inter-frequency measurements reporting requirements	633

9.3A.6.1	Periodic Reporting	633

9.3A.6.2	Event-triggered Periodic Reporting	633

9.3A.6.3	Event-triggered Reporting	633

9.3A.8	Inter-frequency RSSI measurements	634

9.3A.9	Inter-frequency channel occupancy measurements	634

9.3B	NR inter-frequency measurements for RedCap	635

9.3B.1	Introduction	635

9.3B.2	Requirements applicability	635

9.3B.3	Number of cells and number of SSB	636

9.3B.3.1	Requirements for FR1	636

9.3B.3.2	Requirements for FR2	636

9.3B.4	Inter-frequency measurement with measurement gaps	636

9.3B.5	Inter-frequency measurements	638

9.3B.6	Inter-frequency measurements reporting requirements	639

9.3B.6.1	Periodic Reporting	639

9.3B.6.2	Event-triggered Periodic Reporting	639

9.3B.6.3	Event-triggered Reporting	639

9.3B.7	Inter frequency measurements without measurement gaps	640

9.3B.7.1	Inter frequency Cell identification	640

9.3B.7.2	Measurement period	642

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	643

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	643

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	643

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	644

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	644

9.3C	NR inter-frequency measurements for SAN	644

9.3C.1	Introduction	644

9.3C.2	Requirements applicability	645

9.3C.3	Number of cells and number of SSB	645

9.3C.3.1	Requirements for FR1	645

9.3C.4	Inter-frequency measurement with measurement gaps	645

9.3C.5	Inter-frequency measurements	646

9.3C.6	Inter-frequency measurements reporting requirements	647

9.3C.6.1	Periodic Reporting	647

9.3C.6.2	Event-triggered Periodic Reporting	647

9.3C.6.3	Event-triggered Reporting	647

9.3C.7	Inter frequency measurements without measurement gaps	648

9.3C.7.1	Inter frequency Cell identification	648

9.3C.7.2	Measurement period	649

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	649

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	650

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	650

9.4	Inter-RAT measurements	650

9.4.1	Introduction	650

9.4.2	NR − E-UTRAN FDD measurements	653

9.4.2.1	Introduction	653

9.4.2.2	Requirements when no DRX is used	653

9.4.2.3	Requirements when DRX is used	654

9.4.2.4	Measurement reporting requirements	656

9.4.2.4.1	Periodic Reporting	656

9.4.2.4.2	Event-Triggered Periodic Reporting	656

9.4.2.4.3	Event-Triggered Reporting	656

9.4.3	NR − E-UTRAN TDD measurements	656

9.4.3.1	Introduction	656

9.4.3.2	Requirements when no DRX is used	657

9.4.3.3	Requirements when DRX is used	658

9.4.3.4	Measurement reporting requirements	660

9.4.3.4.1	Periodic Reporting	660

9.4.3.4.2	Event-Triggered Periodic Reporting	660

9.4.3.4.3	Event-Triggered Reporting	660

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	661

9.4.4	Inter-RAT RSTD measurements	661

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	661

9.4.4.1.1	Introduction	661

9.4.4.1.2	Requirements	661

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	664

9.4.4.2.1	Introduction	664

9.4.4.2.2	Requirements	665

9.4.5	Inter-RAT E-CID measurements	668

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	668

9.4.5.1.1	Introduction	668

9.4.5.1.2	Requirements	668

9.4.5.1.3	Measurement Reporting Delay	669

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	669

9.4.5.2.1	Introduction	669

9.4.5.2.2	Requirements	669

9.4.5.2.3	Measurement Reporting Delay	669

9.4.6	NR − UTRAN FDD measurements	669

9.4.6.1	Introduction	669

9.4.6.2	Requirements when no DRX is used	669

9.4.6.3	Requirements when DRX is used	671

9.4.7	NR – E-UTRAN measurements with autonomous gaps	673

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	673

9.4A	Inter-RAT measurements for RedCap	673

9.4A.1	Introduction	673

9.4A.2	NR − E-UTRAN FDD measurements	674

9.4A.2.1	Introduction	674

9.4A.2.2	Requirements when no DRX is used	675

9.4A.2.3	Requirements when DRX is used	676

9.4A.2.4	Measurement reporting requirements	677

9.4A.2.4.1	Periodic Reporting	677

9.4A.2.4.2	Event-Triggered Periodic Reporting	677

9.4A.2.4.3	Event-Triggered Reporting	677

9.4A.3	NR − E-UTRAN TDD measurements	678

9.4A.3.1	Introduction	678

9.4A.3.2	Requirements when no DRX is used	678

9.4A.3.3	Requirements when DRX is used	680

9.4A.3.4	Measurement reporting requirements	681

9.4A.3.4.1	Periodic Reporting	681

9.4A.3.4.2	Event-Triggered Periodic Reporting	681

9.4A.3.4.3	Event-Triggered Reporting	681

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	682

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	682

9.4A.4.2	CGI reporting delay	682

9.4A.4.3	CGI reporting scheduling restriction	683

9.5	L1-RSRP measurements for Reporting	683

9.5.1	Introduction	683

9.5.2	Requirements applicability	684

9.5.3	Measurement Reporting Requirements	684

9.5.3.1	Periodic Reporting	684

9.5.3.2	Semi-Persistent Reporting	684

9.5.3.3	Aperiodic Reporting	685

9.5.4	L1-RSRP measurement requirements	685

9.5.4.1	SSB based L1-RSRP Reporting	685

9.5.4.2	CSI-RS based L1-RSRP Reporting	688

9.5.4A	Void	691

9.5.4A.1	Void	691

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	692

9.5.5.1	Measurement restriction for SSB based L1-RSRP	692

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	692

9.5.6	Scheduling availability of UE during L1-RSRP measurement	693

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	693

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	693

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	693

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	695

9.5A	L1-RSRP measurements for Reporting under CCA	695

9.5A.1	Introduction	695

9.5A.2	Requirements applicability	695

9.5A.3	Measurement Reporting Requirements	696

9.5A.3.1	Periodic Reporting	696

9.5A.3.2	Semi-Persistent Reporting	696

9.5A.3.3	Aperiodic Reporting	696

9.5A.4	L1-RSRP measurement requirements	697

9.5A.4.1	SSB based L1-RSRP Reporting	697

9.5A.5	Measurement restriction for L1-RSRP measurement	699

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	699

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	700

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	700

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	700

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	700

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	700

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	701

9.5B	L1-RSRP measurements for Reporting for RedCap	701

9.5B.1	Introduction	701

9.5B.2	Requirements applicability	701

9.5B.3	Measurement Reporting Requirements	702

9.5B.3.1	Periodic Reporting	702

9.5B.3.2	Semi-Persistent Reporting	702

9.5B.3.3	Aperiodic Reporting	703

9.5B.4	L1-RSRP measurement requirements	703

9.5B.4.1	SSB based L1-RSRP Reporting	703

9.5B.4.2	CSI-RS based L1-RSRP Reporting	705

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	708

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	708

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	708

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	709

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	709

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	709

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	709

9.5C	L1-RSRP measurements for Reporting for satellite access	710

9.5C.1	Introduction	710

9.5C.3	Measurement Reporting Requirements	710

9.5C.3.1	Periodic Reporting	710

9.5C.3.2	Semi-Persistent Reporting	710

9.5C.3.3	Aperiodic Reporting	711

9.5C.4	L1-RSRP measurement requirements	711

9.5C.4.1	SSB based L1-RSRP Reporting	711

9.5C.5	Measurement restriction for L1-RSRP measurement	711

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	711

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	711

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	711

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	711

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	711

9.6	NE-DC: Measurements	712

9.6.1	Introduction	712

9.6.2	SFTD Measurements	712

9.6.2.1	Introduction	712

9.6.2.2	SFTD Measurement requirements	712

9.7	Cross Link Interference measurements	713

9.7.1	Introduction	713

9.7.2	SRS-RSRP measurements	713

9.7.2.1	Introduction	713

9.7.2.2	Requirements applicability	713

9.7.2.3	Measurement Reporting Requirements	713

9.7.2.3.1	Periodic Reporting	713

9.7.2.3.2	Event-triggered Periodic Reporting	714

9.7.2.3.3	Event Triggered Reporting	714

9.7.2.4	Measurement capability	714

9.7.2.5	SRS-RSRP measurement period	714

9.7.3	CLI-RSSI measurements	714

9.7.3.1	Introduction	714

9.7.3.2	Requirements applicability	714

9.7.3.3	Measurement Reporting Requirements	715

9.7.3.3.1	Periodic Reporting	715

9.7.3.3.2	Event-triggered Periodic Reporting	715

9.7.3.3.3	Event Triggered Reporting	715

9.7.3.4	Measurement capability	715

9.7.3.5	CLI-RSSI measurement period	715

9.7.4	Scheduling availability of UE during CLI measurements	716

9.7.4.1	Scheduling availability of UE performing measurement on FR1	716

9.7.4.2	Scheduling availability of UE performing measurement on FR2	716

9.8	L1-SINR measurements for Reporting	717

9.8.1	Introduction	717

9.8.2	Requirements applicability	717

9.8.3	Measurement Reporting Requirements	718

9.8.3.1	Periodic Reporting	718

9.8.3.2	Semi-Persistent Reporting	718

9.8.4	L1-SINR measurement requirements	719

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	719

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	723

9.8.5	Measurement restriction for L1-SINR measurement	725

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	725

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	726

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	727

9.8.6	Scheduling availability of UE during L1-SINR measurement	727

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	727

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	727

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	728

9.9	 NR measurements for positioning	728

9.9.1	Introduction	728

9.9.1.1	General Aspects of Gap-based Measurement	729

9.9.1.2	General Aspects of Gapless Measurement	729

9.9.2	RSTD measurements	730

9.9.2.1	Introduction	730

9.9.2.2	Requirements Applicability	730

9.9.2.3	Measurement Capability	730

9.9.2.4	Measurement Reporting Requirements	730

9.9.2.4.1	Void	731

9.9.2.4.2	Void	731

9.9.2.4.3	Void	731

9.9.2.5	Measurements Period Requirements	731

9.9.2.6	Void	734

9.9.2.7	Measurements Period Requirements without Measurement Gaps	734

9.9.2.8	Scheduling Availability of UE during RSTD Measurement	736

9.9.3	PRS-RSRP measurements	737

9.9.3.1	Introduction	737

9.9.3.2	Requirements applicability	737

9.9.3.3	Measurement Capability	737

9.9.3.4	Measurement Reporting Requirements	737

9.9.3.5	Measurement Period Requirements	737

9.9.3.6	Measurement Period Requirements without Measurement Gaps	740

9.9.3.7	Scheduling Availability of UE during PRS-RSRP Measurement	742

9.9.4	UE Rx-Tx time difference measurements	743

9.9.4.1 Introduction	743

9.9.4.2 Requirements Applicability	743

9.9.4.3 Measurement Capability	743

9.9.4.4 Measurement Reporting Requirements	743

9.9.4.5	Measurement Period Requirements	743

9.9.4.6	Measurement Period Requirements without Measurement Gaps	747

9.9.4.7	Scheduling Availability of UE during UE Rx-Tx Time Difference Measurement	749

9.9.5	E-CID measurements	750

9.9.5.1	Introduction	750

9.9.5.2	Measurement Requirements	750

9.9.5.2.1	Intra-frequency Measurement Requirements	750

9.9.5.2.2	Inter-frequency Measurement Requirements	751

9.9.5.2.3	Measurement Reporting Delay	751

9.9.6	PRS-RSRPP measurements	752

9.9.6.1	Introduction	752

9.9.6.2	Requirements applicability	752

9.9.6.3	Measurement capability	752

9.9.6.4	Measurement reporting requirements	752

9.9.6.5	Measurement period requirements	752

9.9.6.6	Measurement Period Requirements without Measurement Gaps	752

9.9.6.7	Scheduling Availability of UE during PRS-RSRPP Measurement	752

9.10	CSI-RS based L3 measurements	753

9.10.1	Introduction	753

9.10.2	CSI-RS based intra-frequency measurements	753

9.10.2.1	Introduction	753

9.10.2.2	Requirements applicability	754

9.10.2.3	Number of cells and number of CSI-RS	755

9.10.2.3.1	Requirements for FR1	755

9.10.2.3.2	Requirements for FR2	755

9.10.2.4	Measurement Reporting Requirements	755

9.10.2.4.1	Periodic Reporting	755

9.10.2.4.2	Event-triggered Periodic Reporting	756

9.10.2.4.3	Event Triggered Reporting	756

9.10.2.5	Intra-frequency measurements without measurement gaps	756

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	758

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	758

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	758

9.10.3	CSI-RS based Inter-frequency measurements	758

9.10.3.1	Introduction	758

9.10.3.2	Requirements applicability	759

9.10.3.3	Number of cells and number of CSI-RS resources	759

9.10.3.3.1	Requirements for FR1	759

9.10.3.3.2	Requirements for FR2	760

9.10.3.4	Measurements reporting requirements	760

9.10.3.4.1	Periodic Reporting	760

9.10.3.4.2	Event-triggered Periodic Reporting	760

9.10.3.4.3	Event-triggered Reporting	760

9.10.3.5	Inter frequency measurements with measurement gaps	760

9.11	NR measurements with autonomous gaps	762

9.11.1	Introduction	762

9.11.2	CGI identification of an NR cell with autonomous gaps	762

9.11.3	CGI reporting delay	763

9.11A	NR measurements with autonomous gaps for RedCap	764

9.11A.1	Introduction	764

9.11A.2	CGI identification of an NR cell with autonomous gaps	764

9.11A.3	CGI reporting delay	765

9.11A.4	CGI reporting scheduling restriction	765

9.12	Measurement for Propagation Delay Compensation	766

9.12.1	Introduction	766

9.12.2	Requirements Applicability	766

9.12.3	Measurement Capability	766

9.12.4	Measurement period requirements	766

9.12.4.1	PRS Measurement Period	766

9.12.4.2	TRS Measurement Period	767

9.12.5	Measurement Reporting Requirements	767

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	767

9.13.1	Introduction	767

9.13.2	Requirements Applicability	768

9.13.3	Measurement Reporting Requirements	768

9.13.3.1	Periodic Reporting	769

9.13.3.2	Semi-Persistent Reporting	769

9.13.3.3	Aperiodic Reporting	769

9.13.4	L1-RSRP measurement requirements	769

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	769

9.13.5	Measurement restriction for L1-RSRP measurement	771

9.13.5.1	Measurement restriction for SSB based L1-RSRP	771

9.13.6	Scheduling availability of UE during L1-RSRP measurement	772

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	772

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	772

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	773

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	773

10	Measurement Performance requirements	773

10.1	NR measurements	773

10.1.1	Introduction	773

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	774

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	774

10.1.2.1.1	Absolute SS-RSRP Accuracy	774

10.1.2.1.2	Relative SS-RSRP Accuracy	775

10.1.2.2	Void	776

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	776

10.1.2.3.1	Absolute CSI-RSRP Accuracy	776

10.1.2.3.2	Relative CSI-RSRP Accuracy	777

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	778

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	778

10.1.2B.1.1	Absolute SS-RSRP Accuracy	778

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	779

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	779

10.1.2C.1.1	Absolute SS-RSRP Accuracy	779

10.1.2C.1.2	Relative SS-RSRP Accuracy	780

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	780

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	780

10.1.3.1.1	Absolute SS-RSRP Accuracy	780

10.1.3.1.2	Relative SS-RSRP Accuracy	781

10.1.3.2	Void	782

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	782

10.1.3.3.1	Absolute CSI-RSRP Accuracy	782

10.1.3.3.2	Relative CSI-RSRP Accuracy	783

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	784

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	784

10.1.3B.1.1	Absolute SS-RSRP Accuracy	784

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	785

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	785

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	785

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	786

10.1.4.2	Void	787

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	787

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	787

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	788

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	789

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	789

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	789

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	790

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	790

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	790

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	791

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	791

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	791

10.1.5.1.1	Absolute SS-RSRP Accuracy	791

10.1.5.1.2	Relative SS-RSRP Accuracy	792

10.1.5.2	Void	793

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	793

10.1.5.3.1	Absolute CSI-RSRP Accuracy	793

10.1.5.3.2	Relative CSI-RSRP Accuracy	794

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	795

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	795

10.1.5B.1.1	Absolute SS-RSRP Accuracy	795

10.1.6	RSRP Measurement Report Mapping	796

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	798

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	798

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	798

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	799

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	799

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	800

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	800

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	800

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	801

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	801

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	801

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	802

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	802

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	802

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	803

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	803

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	803

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	803

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	804

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	804

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	804

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	804

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	805

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	806

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	806

10.1.9.2.2	Relative CSI-RSRQ Accuracy	807

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	808

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	808

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	808

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	809

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	809

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	809

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	810

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	811

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	811

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	811

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	811

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	812

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	812

10.1.10.2.2	Relative CSI-RSRQ Accuracy	813

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	814

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	814

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	814

10.1.11	RSRQ report mapping	815

10.1.12	Intra-frequency SINR accuracy requirements for FR1	815

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	815

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	815

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	816

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	816

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	817

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	817

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	817

10.1.13	Intra-frequency SINR accuracy requirements for FR2	818

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	818

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	818

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	819

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	819

10.1.14	Inter-frequency SINR accuracy requirements for FR1	819

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	819

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	819

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	820

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	821

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	821

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	822

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	823

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	823

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	823

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	824

10.1.15	Inter-frequency SINR accuracy requirements for FR2	825

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	825

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	825

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	825

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	826

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	826

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	827

10.1.16	SINR report mapping	828

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	828

10.1.17	Power Headroom	829

10.1.17.1	Power Headroom Report	829

10.1.17.1.1	Power Headroom Report Mapping	829

10.1.18	PCMAX,c,f	830

10.1.18.1	Report Mapping	830

10.1.19	L1-RSRP accuracy requirements for FR1	830

10.1.19.1	SSB based L1-RSRP accuracy requirements	830

10.1.19.1.1	Absolute Accuracy	830

10.1.19.1.2	Relative Accuracy	831

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	832

10.1.19.2.1	Absolute Accuracy	832

10.1.19.2.2	Relative Accuracy	833

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	834

10.1.19C.1	SSB based L1-RSRP accuracy requirements	834

10.1.19C.1.1	Absolute Accuracy	834

10.1.19C.1.2	Relative Accuracy	835

10.1.20	L1-RSRP accuracy requirements for FR2	835

10.1.20.1	SSB based L1-RSRP accuracy requirements	835

10.1.20.1.1	Absolute Accuracy	835

10.1.20.1.2	Relative Accuracy	836

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	837

10.1.20.2.1	Absolute Accuracy	837

10.1.20.2.2	Relative Accuracy	838

10.1.21	SFTD accuracy requirements	839

10.1.21.1	SFTD acuracy requirements for NE-DC	839

10.1.21.2	SFTD acuracy requirements for NR-DC	841

10.1.21.3	Inter frequency SFTD acuracy requirements	842

10.1.22	CLI measurement accuracy requirements	843

10.1.22.1	SRS-RSRP	843

10.1.22.1.1	SRS-RSRP Accuracy	843

10.1.22.1.2	SRS-RSRP report mapping	845

10.1.22.2	CLI-RSSI	845

10.1.22.2.1	CLI-RSSI Accuracy	845

10.1.22.2.2	CLI-RSSI report mapping	846

10.1.23	RSTD Measurements	847

10.1.23.1	Introduction	847

10.1.23.2	Measurement Accuracy Requirements	847

10.1.23.3	Report mapping	852

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	852

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	854

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	856

10.1.24	PRS-RSRP Measurements	858

10.1.24.1	Introduction	858

10.1.24.2	Measurement Accuracy Requirements	858

10.1.24.2.1	Absolute PRS RSRP accuracy	858

10.1.24.2.2	Relative PRS RSRP accuracy	862

10.1.24.3	Report mapping	866

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	866

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	867

10.1.25	UE Rx-Tx Time Difference Measurements	869

10.1.25.1	Introduction	869

10.1.25.2	Measurement Accuracy Requirements	869

10.1.25.3	Report mapping	881

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	881

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	883

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	885

10.1.26	FR2 P-MPR report	887

10.1.26.1	Report mapping	887

10.1.27	L1-SINR accuracy requirements for FR1	887

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	887

10.1.27.1.1	Absolute Accuracy	887

10.1.27.1.2	Relative Accuracy	888

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	889

10.1.27.2.1	Absolute Accuracy	889

10.1.27.2.2	Relative Accuracy	890

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	892

10.1.27.3.1	Absolute Accuracy	892

10.1.27.3.2	Relative Accuracy	894

10.1.28	L1-SINR accuracy requirements for FR2	895

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	902

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	902

10.1.29.1.1	Absolute SS-RSRQ Accuracy	902

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	902

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	902

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	902

10.1.30.1.2	Relative Accuracy of SS-RSRQ	903

10.1.31	Intra-frequency SINR accuracy requirements under CCA	904

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	904

10.1.31.1.1	Absolute SS-SINR Accuracy	904

10.1.32	Inter-frequency SINR accuracy requirements under CCA	904

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	904

10.1.32.1.1	Aboslute Accuracy of SS-SINR	904

10.1.32.1.2	Relative Accuracy of SS-SINR	905

10.1.33	L1-RSRP accuracy requirements under CCA	906

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	906

10.1.33.1.1	Absolute Accuracy	906

10.1.33.1.2	Relative Accuracy	906

10.1.34	RSSI measurements under CCA	907

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	907

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	907

10.1.34.3	RSSI measurement report mapping	907

10.1.35	Channel occupancy measurements under CCA	908

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	908

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	908

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	908

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	908

10.1.36.1.1	Absolute SS-RSRP Accuracy	908

10.1.36.1.2	Relative SS-RSRP Accuracy	909

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	910

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	910

10.1.37.1.1	Absolute Accuracy of SS-RSRP	910

10.1.37.1.2	Relative Accuracy of SS-RSRP	910

10.1.38	PRS-RSRPP Measurements	911

10.1.38.1	Introduction	911

10.1.38.2	Measurement Accuracy Requirements	911

10.1.38.2.1	Absolute PRS RSRPP accuracy	911

10.1.38.3	Report mapping	915

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	915

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	916

10.1A	NR measurements for RedCap	917

10.1A.1	Introduction	917

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	918

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	918

10.1A.2.1.1	Absolute SS-RSRP Accuracy	918

10.1A.2.1.2	Relative SS-RSRP Accuracy	919

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	919

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	919

10.1A.3.1.1	Absolute SS-RSRP Accuracy	919

10.1A.3.1.2	Relative SS-RSRP Accuracy	919

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	920

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	920

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	920

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	920

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	921

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	921

10.1A.5.1.1	Absolute SS-RSRP Accuracy	921

10.1A.5.1.2	Relative SS-RSRP Accuracy	921

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	921

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	921

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	921

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	922

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	922

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	922

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	922

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	922

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	922

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	923

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	924

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	924

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	924

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	924

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	924

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	924

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	924

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	925

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	925

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	925

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	925

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	925

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	925

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	926

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	927

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	927

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	927

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	927

10.1A.14	 L1-RSRP accuracy requirements for FR1	927

10.1A.14.1	SSB based L1-RSRP accuracy requirements	927

10.1A.14.1.1	Absolute Accuracy	927

10.1A.14.1.2	Relative Accuracy	928

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	929

10.1A.14.2.1	Absolute Accuracy	929

10.1A.14.2.2	Relative Accuracy	930

10.1A.15	 L1-RSRP accuracy requirements for FR2	931

10.1A.15.1	SSB based L1-RSRP accuracy requirements	931

10.1A.15.1.1	Absolute Accuracy	931

10.1A.15.1.2	Relative Accuracy	931

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	931

10.1A.15.2.1	Absolute Accuracy	931

10.1A.15.2.2	Relative Accuracy	931

10.2	E-UTRAN measurements	932

10.2.1	Introduction	932

10.2.2	E-UTRAN RSRP measurements	932

10.2.3	E-UTRAN RSRQ measurements	932

10.2.4	E-UTRAN RSTD measurements	932

10.2.5	E-UTRAN RS-SINR measurements	933

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	933

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	933

10.3	UTRAN FDD Measurements	933

10.3.1	UTRAN FDD CPICH RSCP	934

10.3.2	UTRAN FDD CPICH Ec/No	934

10.4	V2X measurements	935

10.4.1	Introduction	935

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	935

10.4.2.1	PSBCH-RSRP Absolute Accuracy	935

10.4.2.2	PSBCH-RSRP Relative Accuracy	935

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	936

10.4.3.1	Absolute SL-RSSI Accuracy	936

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	937

10.4.4.1	Absolute L1 SL-RSRP Accuracy	937

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	937

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	937

11	Void	938

12	V2X Requirements	938

12.1	Introduction	938

12.2	UE Transmit Timing	939

12.2.1	Introduction	939

12.2.2	GNSS as synchronization reference source	939

12.2.3	NR Cell as synchronization reference source	939

12.2.4	E-URTAN Cell as synchronization reference source	940

12.2.5	SyncRef UE as synchronization reference source	940

12.3	Initiation/Cease of SLSS Transmissions	940

12.3.1	Introduction	940

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	940

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	941

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	942

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	942

12.4	Selection / Reselection of V2X Synchronization Reference Source	943

12.5	L1 SL-RSRP measurements	945

12.5.1	Introduction	945

12.5.2	SL-RSRP measurements	945

12.6	Congestion Control measurements	945

12.7	Interruption	946

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	946

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	946

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	948

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	948

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	949

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	949

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	949

12.7.8	Interruptions at NR sidelink discovery configuration	949

12.8	Reliability of GNSS signal	950

12.9	Scheduling availability	950

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	950

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	950

12.10	Selection / Reselection of relay UE	951

12.10.1	Introduction	951

12.10.2	Selection / Reselection of relay UE	951

13	Measurement Performance Requirements for NR gNB	952

13.1	UL-RTOA	952

13.1.1	Report mapping	952

13.2	gNB Rx-Tx time difference	954

13.2.1	Report mapping	954

13.2.2	Measurement Accuracy Requirements	956

13.2.2.1	Introduction	956

13.3	UL SRS RSRP measurement	957

13.3.1	Report mapping	957

13.3.2	Measurement accuracy requirements	958

13.3.2.1	Introduction	958

13.3.2.2	Requirements	958

13.4	AoA/ZoA	959

13.4.1	Report mapping	959

13.5	Timing advance (TADV)	960

13.5.1	Report mapping	960

13.6	UL SRS RSRPP measurement	961

13.6.1	Report mapping	961

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	963

13.7.1	Report mapping	963

13.7.2	Measurement Accuracy Requirements	963

13.7.2.1	Introduction	963

13.7.2.2	Requirements	963

Annex A (normative): Test Cases	967

A.1	Purpose of annex	967

A.2	Requirement classification for statistical testing	967

A.2.1	Types of requirements in TS 38.133	967

A.2.1.1	Time and delay requirements on UE higher layer actions	967

A.2.1.2	Measurements of power levels, relative powers and time	967

A.2.1.3	Implementation requirements	968

A.2.1.4	Physical layer timing requirements	968

A.2.1.5	Requirements under CCA	968

A.3	RRM test configurations	969

A.3.1	Reference measurement channels	969

A.3.1.1	PDSCH	969

A.3.1.1.1	FDD	969

A.3.1.1.2	TDD	970

A.3.1.2	CORESET for RMSI scheduling	973

A.3.1.2.1	FDD	973

A.3.1.2.2	TDD	974

A.3.1.3	CORESET for RMC scheduling	977

A.3.1.3.1	FDD	977

A.3.1.3.2	TDD	978

A.3.1.4	TDD UL/DL configuration	981

A.3.1A	Reference measurement channels under CCA	983

A.3.1A.1	PDSCH	983

A.3.1A.1.1	TDD	983

A.3.1A.2	CORESET for RMSI scheduling	984

A.3.1A.2.1	TDD	984

A.3.1A.3	CORESET for RMC scheduling	985

A.3.1A.3.1	TDD	985

A.3.1A.4	TDD UL/DL configuration	986

A.3.1A.5	RMC burst transmission model	986

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	986

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	987

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	987

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	988

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	988

A.3.2.2	Void	989

A.3.3	Reference DRX configurations	989

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	989

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	990

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	990

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	990

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	991

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	991

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	991

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	992

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	992

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	992

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	993

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	993

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	993

A.3.4	Test Cases with Different Channel Bandwidths	994

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	994

A.3.4.1.1	Introduction	994

A.3.4.1.2	Principle of testing	994

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	994

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	994

A.3.5.1.1	Introduction	994

A.3.5.1.2	Principle of Testing	994

A.3.6	Antenna configurations	994

A.3.6.1	Antenna configurations for FR1	994

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	994

A.3.6.1.1.1	Introduction	994

A.3.6.1.1.2	Principle of testing	995

A.3.6.2	Antenna configurations for FR2	998

A.3.6A	Antenna configurations with unlicensed bands	998

A.3.6A.1	Antenna configurations for FR1	998

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	998

A.3.6A.1.1.1	Introduction	998

A.3.6A.1.1.2	Principle of testing	998

A.3.7	EN-DC test setup	1000

A.3.7.1	Introduction	1000

A.3.7.2	E-UTRAN Serving Cell Parameters	1000

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1000

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1001

A.3.7A	NR FR1-FR2 test setup	1002

A.3.7B	EN-DC test setup with unlicensed bands	1003

A.3.7B.1	Introduction	1003

A.3.7B.2	E-UTRAN Serving Cell Parameters	1003

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1003

A.3.7C	LTE-FR1/FR2 test setup	1004

A.3.7D	NE-DC test setup	1004

A.3.7D.1	Introduction	1004

A.3.7D.2	E-UTRAN Serving Cell Parameters	1004

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1004

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1004

A.3.8	PRACH configurations	1004

A.3.8.1	Introduction	1004

A.3.8.2	PRACH configurations in FR1	1005

A.3.8.2.1	FR1 PRACH configuration 1	1005

A.3.8.2.2	FR1 PRACH configuration 2	1005

A.3.8.2.3	FR1 PRACH configuration 3	1006

A.3.8.2.4	FR1 PRACH configuration 4	1007

A.3.8.3	PRACH configurations in FR2	1008

A.3.8.3.1	FR2 PRACH configuration 1	1008

A.3.8.3.2	FR2 PRACH configuration 2	1009

A.3.8.3.3	FR2 PRACH configuration 3	1010

A.3.8.3.4	FR2 PRACH configuration 4	1011

A.3.8A	PRACH configurations under CCA	1012

A.3.8A.1	Introduction	1012

A.3.8A.2	PRACH configurations in FR1	1012

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1012

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1013

A.3.9	BWP configurations	1014

A.3.9.1	Introduction	1014

A.3.9.2	Downlink BWP configurations	1015

A.3.9.2.1	Initial BWP	1015

A.3.9.2.2	Dedicated BWP	1015

A.3.9.3	Uplink BWP configurations	1016

A.3.9.3.1	Initial BWP	1016

A.3.9.3.2	Dedicated BWP	1016

A.3.10	SSB Configurations	1017

A.3.10.1	SSB Configurations for FR1	1017

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1017

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1019

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1019

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1020

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1020

A.3.10.2	SSB Configurations for FR2	1021

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1021

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1021

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1022

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1022

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1023

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1023

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1024

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1024

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1025

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1025

A.3.10A	SSB Configurations under CCA	1030

A.3.10A.1	SSB Configurations under CCA for FR1	1030

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1030

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1030

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1031

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1031

A.3.10B	SSB Configurations for RedCap	1032

A.3.10B.1	SSB Configurations for FR1	1032

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1032

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1032

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1033

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1033

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1034

A.3.10B.2	SSB Configurations for FR2	1034

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1034

A.3.11	SMTC Configurations	1035

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1035

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1035

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1035

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1035

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1036

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1036

A.3.11.7	SMTC pattern Y: SMTC period = 20 ms with SMTC duration = 5 ms	1036

A.3.11A	SMTC Configurations for RedCap	1036

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1036

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1036

A.3.12	Test Cases with Different CC Configurations	1037

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1037

A.3.12.1.1	Introduction	1037

A.3.12.1.2	Principle of testing	1037

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1037

A.3.12.2.1	Introduction	1037

A.3.12.2.2	Principle of testing	1037

A.3.13	Test Cases in SA and EN-DC Operations	1037

A.3.13.1	Introduction	1037

A.3.13.2	Principle of Testing	1038

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1038

A.3.13A.1	Introduction	1038

A.3.13A.2	Principle of Testing in EN-DC	1038

A.3.13A.3	Principle of Testing in SA	1038

A.3.13A.4	Principle of Testing in E-UTRA	1039

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1039

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1039

A.3.13B.1.1	Introduction	1039

A.3.13B.1.2	Principle of Testing	1040

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1040

A.3.13B.2.1	Introduction	1040

A.3.13B.2.2	Principle of Testing	1040

A.3.14	CSI-RS configurations	1040

A.3.14.1	FDD	1040

A.3.14.2	TDD	1042

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1048

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1048

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1048

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1048

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1049

A.3.15.3	Setup 3: 2 AoAs	1049

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1049

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1049

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1049

A.3.16	TCI State Configuration	1050

A.3.16.1	Introduction	1050

A.3.16.2	TCI states	1050

A.3.16A	Unified TCI State Configuration	1050

A.3.16A.1	Introduction	1050

A.3.16A.2	DLorJoint TCI states	1051

A.3.16A.3	UL TCI states	1051

A.3.17	Configurations of CSI-RS for tracking	1052

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1052

A.3.17.1.1	FDD	1052

A.3.17.1.2	TDD	1054

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1056

A.3.17.2.1	TDD	1056

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1057

A.3.18.1	Introduction	1057

A.3.18.2	PRACH Power Measurement	1057

A.3.19	Test applicability for DAPS handover	1057

A.3.19.1	Introduction	1057

A.3.19.2	Principle of testing	1057

A.3.20	MsgA configurations	1058

A.3.20.1	Introduction	1058

A.3.20.2	MsgA configurations in FR1	1058

A.3.20.2.1	FR1 MsgA configuration 1	1058

A.3.20.2.2	FR1 MsgA configuration 2	1059

A.3.20.3	MsgA configurations in FR2	1061

A.3.20.3.1	FR2 MsgA configuration 1	1061

A.3.20.3.2	FR2 MsgA configuration 2	1062

A.3.20A	MsgA configurations under CCA	1064

A.3.20A.1	Introduction	1064

A.3.20A.2	MsgA configurations in FR1	1064

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1064

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1065

A.3.21	V2X sidelink communication	1068

A.3.21.1	Introduction	1068

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1068

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1072

A.3.21.4	Reference SL-DRX configurations	1073

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1073

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1073

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1073

A.3.22	CSI-IM configurations	1073

A.3.22.1	FDD	1073

A.3.22.2	TDD	1074

A.3.23	Spatial Relation Configuration	1075

A.3.23.1	Introduction	1075

A.3.23.2	Spatial Relation	1076

A.3.24	SRS configuration	1077

A.3.25	Channel bandwidth (CBW) configurations	1079

A.3.25.1	DL UE specific CBW	1079

A.3.25.2	UL UE specific CBW	1080

A.3.26	CCA model	1080

A.3.26.1	Introduction	1080

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1080

A.3.26.2.1	DL CCA model	1080

A.3.26.2.2	UL CCA model	1081

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1082

A.3.27.1	Introduction	1082

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1082

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1082

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1082

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1082

A.3.28	Discovery Burst Transmission Window configuration under CCA	1083

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1083

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1083

A.3.29.1	Introduction	1083

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1083

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1083

A.3.30	CSI-RS configurations for RRM	1084

A.3.30.1	FDD	1084

A.3.30.2	TDD	1085

A.3.31	PRS Configurations	1087

A.3.31.1.	PRS Configurations for FR1	1087

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1087

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1088

A.3.31.2.	PRS Configurations for FR2	1088

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1088

A.3.32	NR sidelink discovery	1088

A.3.32.1	Introduction	1088

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1088

A.3.32.3	Principle of Testing	1089

A.3.33	PRS Processing Window (PPW) configurations	1089

A.3.34	Testing principles for test cases related to PRS measurements	1089

A.3.34.1	Introduction	1089

A.3.34.2	Test cases in RRC_INACTIVE state	1090

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1090

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1090

A.3.35	Testing principle for RedCap UE	1091

A.3.35.1	Introduction	1091

A.3.35.2	Principle of testing for FR1	1091

A.3.35.3	Principle of testing for FR2	1091

A.4	EN-DC tests with all NR cells in FR1	1092

A.4.1	Void	1092

A.4.2	Void	1092

A.4.3	RRC_CONNECTED state mobility	1092

A.4.3.1	Void	1092

A.4.3.2	RRC Connection Mobility Control	1092

A.4.3.2.1	Void	1092

A.4.3.2.2	Random Access	1092

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1092

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1096

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1100

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1104

A.4.3.2.3	Void	1108

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1108

A.4.3.3.1	Test Purpose and Environment	1108

A.4.3.3.2	Test Requirements	1114

A.4.4	Timing	1115

A.4.4.1	UE transmit timing	1115

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1115

A.4.4.1.1.1	Test Purpose and environment	1115

A.4.4.1.1.2	Test requirements	1119

A.4.4.2	UE timer accuracy	1119

A.4.4.3	Timing advance	1119

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1119

A.4.4.3.1.1	Test Purpose and Environment	1119

A.4.4.3.1.2	Test Parameters	1119

A.4.4.3.1.3	Test Requirements	1123

A.4.5	Signaling characteristics	1123

A.4.5.1	Radio link Monitoring	1123

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1124

A.4.5.1.1.1	Test Purpose and Environment	1124

A.4.5.1.1.2	Test Requirements	1128

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1128

A.4.5.1.2.1	Test Purpose and Environment	1128

A.4.5.1.2.2	Test Requirements	1134

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1134

A.4.5.1.3.1	Test Purpose and Environment	1134

A.4.5.1.3.2	Test Requirements	1139

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1139

A.4.5.1.4.1	Test Purpose and Environment	1139

A.4.5.1.4.2	Test Requirements	1145

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1145

A.4.5.1.5.1	Test Purpose and Environment	1145

A.4.5.1.5.2	Test Requirements	1150

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1150

A.4.5.1.6.1	Test Purpose and Environment	1150

A.4.5.1.6.2	Test Requirements	1156

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1156

A.4.5.1.7.1	Test Purpose and Environment	1156

A.4.5.1.7.2	Test Requirements	1161

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1162

A.4.5.1.8.1	Test Purpose and Environment	1162

A.4.5.1.8.2	Test Requirements	1167

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1167

A.4.5.1.9.1	Test Purpose and Environment	1167

A.4.5.2	Interruption	1173

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1173

A.4.5.2.1.1	Test Purpose and Environment	1173

A.4.5.2.1.2	Test Requirements	1177

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1177

A.4.5.2.2.1	Test Purpose and Environment	1177

A.4.5.2.2.2	Test Requirements	1181

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1181

A.4.5.2.3.1	Test Purpose and Environment	1181

A.4.5.2.3.2	Test Requirements	1186

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1187

A.4.5.2.4.1	Test Purpose and Environment	1187

A.4.5.2.4.2	Test Requirements	1191

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1192

A.4.5.2.5.1	Test Purpose and Environment	1192

A.4.5.2.5.2	Test Requirements	1196

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1196

A.4.5.2.6.1	Test Purpose and Environment	1196

A.4.5.2.6.2	Test Requirements	1200

A.4.5.2.7	Void	1200

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1200

A.4.5.2.8.1	Test Purpose and Environment	1200

A.4.5.2.8.2	Test Requirements	1203

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1204

A.4.5.2.9.1	Test Purpose and Environment	1204

A.4.5.2.9.2	Test Requirements	1207

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1208

A.4.5.2.10.1	Test Purpose and Environment	1208

A.4.5.2.10.2	Test Requirements	1211

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1211

A.4.5.2.11.1	Test Purpose and Environment	1211

A.4.5.2.11.2	Test Requirements	1216

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1217

A.4.5.2.12.1	Test Purpose and Environment	1217

A.4.5.3	SCell Activation and Deactivation Delay	1225

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1225

A.4.5.3.1.1	Test Purpose and Environment	1225

A.4.5.3.1.2	Test Requirements	1231

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1232

A.4.5.3.2.1	Test Purpose and Environment	1232

A.4.5.3.2.2	Test Requirements	1232

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1232

A.4.5.3.3.1	Test Purpose and Environment	1232

A.4.5.3.3.2	Test Requirements	1233

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1234

A.4.5.3.4.1	Test Purpose and Environment	1234

A.4.5.3.3.2	Test Requirements	1236

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1236

A.4.5.3.5.1	Test Purpose and Environment	1236

A.4.5.3.5.2	Test Requirements	1244

A.4.5.3.3	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1244

A.4.5.3.3.1	Test Purpose and Environment	1244

A.4.5.3.3.2	Test Requirements	1249

A.4.5.3.4	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1250

A.4.5.3.4.1	Test Purpose and Environment	1250

A.4.5.3.4.2	Test Requirements	1250

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1251

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1251

A.4.5.4.1.1 Test Purpose and Environment	1251

A.4.5.4.1.2	Test Requirements	1261

A.4.5.5	Beam Failure Detection and Link recovery procedures	1261

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1261

A.4.5.5.1.1	Test Purpose and Environment	1261

A.4.5.5.1.2	Test Requirements	1267

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1267

A.4.5.5.2.1	Test Purpose and Environment	1267

A.4.5.5.2.2	Test Requirements	1273

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1273

A.4.5.5.3.1	Test Purpose and Environment	1273

A.4.5.5.3.2	Test Requirements	1279

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1279

A.4.5.5.4.1	Test Purpose and Environment	1279

A.4.5.5.4.2	Test Requirements	1285

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1285

A.4.5.5.5.1	Test Purpose and Environment	1285

A.4.5.5.5.2	Test Requirements	1291

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1291

A.4.5.5.6.1	Test Purpose and Environment	1291

A.4.5.5.6.2	Test Requirements	1298

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1298

A.4.5.5.7.1	Test Purpose and Environment	1298

A.4.5.5.7.2	Test Requirements	1303

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1304

A.4.5.5.8.1	Test Purpose and Environment	1304

A.4.5.5.8.2	Test Requirements	1310

A.4.5.6	Active BWP switch	1311

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1311

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1311

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1316

A.4.5.6.2	RRC-based Active BWP Switch	1322

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1326

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1326

A.4.5.6.3.1.1	Test Purpose and Environment	1326

A.4.5.6.3.1.2	Test Requirements	1332

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1332

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1332

A.4.5.6.4.1.1	Test Purpose and Environment	1332

A.4.5.6.4.1.2	Test Requirements	1339

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1339

A.4.5.6.4.2.1	Test Purpose and Environment	1339

A.4.5.6.4.2.2	Test Requirements	1349

A.4.5.6.5	SCell dormancy switch	1350

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1350

A.4.5.6.5.1.1	Test Purpose and Environment	1350

A.4.5.6.5.1.2	Test Requirements	1355

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1355

A.4.5.6.5.2.1	Test Purpose and Environment	1355

A.4.5.6.5.2.2	Test Requirements	1361

A.4.5.7	PSCell addition and release delay	1361

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1361

A.4.5.7.1.1	Test purpose and environment	1361

A.4.5.7.1.2	Test Requirements	1365

A.4.5.8	DL Interruptions at switching between two uplink carriers	1366

A.4.5.8.1	Test Purpose and Environment	1366

A.4.5.8.2	Test Requirements	1369

A.4.5.9	PSCell activation and deactivation delay	1370

A.4.5.9.1	PSCell activation and deactivation delay	1370

A.4.5.9.1.1	Test purpose and environment	1370

A.4.5.9.1.2	Test Requirements	1374

A.4.5.10	Conditional PSCell addition and release delay (FR1 EN-DC)	1375

A.4.5.10.1	Addition and Release Delay of PSCell	1375

A.4.5.10.1.1	Test purpose and environment	1375

A.4.5.10.1.2	Test Parameters	1375

A.4.5.10.1.3	Test Requirements	1379

A.4.6	Measurement procedure	1379

A.4.6.1	Intra-frequency Measurements	1379

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1379

A.4.6.1.1.1	Test purpose and Environment	1379

A.4.6.1.1.2	Test parameters	1379

A.4.6.1.1.3	Test Requirements	1383

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1383

A.4.6.1.2.1	Test purpose and Environment	1383

A.4.6.1.2.2	Test parameters	1383

A.4.6.1.2.2	Test Requirements	1387

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1387

A.4.6.1.3.1	Test purpose and Environment	1387

A.4.6.1.3.2	Test parameters	1387

A.4.6.1.3.3	Test Requirements	1391

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1391

A.4.6.1.4.1	Test purpose and Environment	1391

A.4.6.1.4.2	Test parameters	1391

A.4.6.1.4.3	Test Requirements	1395

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1395

A.4.6.1.5.1	Test purpose and Environment	1395

A.4.6.1.5.2	Test parameters	1395

A.4.6.1.5.3	Test Requirements	1397

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1398

A.4.6.1.6.1	Test purpose and Environment	1398

A.4.6.1.6.2	Test parameters	1398

A.4.6.1.6.3	Test Requirements	1400

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1401

A.4.6.1.7.1	Test purpose and Environment	1401

A.4.6.1.7.2	Test parameters	1401

A.4.6.1.7.3	Test Requirements	1405

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1405

A.4.6.1.8.1	Test Purpose and Environment	1405

A.4.6.1.8.2	Test Requirements	1409

A.4.6.2	Inter-frequency Measurements	1409

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1409

A.4.6.2.1.1	Test Purpose and Environment	1409

A.4.6.2.1.2	Test Requirements	1414

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1414

A.4.6.2.2.1	Test Purpose and Environment	1414

A.4.6.2.2.2	Test Requirements	1418

A.4.6.2.3	Void	1419

A.4.6.2.4	Void	1419

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1419

A.4.6.2.5.1	Test Purpose and Environment	1419

A.4.6.2.5.2	Test Requirements	1423

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1423

A.4.6.2.6.1	Test Purpose and Environment	1423

A.4.6.2.6.2	Test Requirements	1428

A.4.6.2.7	Void	1429

A.4.6.2.8	Void	1429

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1429

A.4.6.2.9.1	Test Purpose and Environment	1429

A.4.6.2.9.2	Test Requirements	1432

A.4.6.3	Void	1433

A.4.6.4	L1-RSRP measurement for beam reporting	1433

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1433

A.4.6.4.1.1	Test Purpose and Environment	1433

A.4.6.4.1.2	Test parameters	1433

A.4.6.4.1.3	Test Requirements	1436

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1436

A.4.6.4.2.1	Test Purpose and Environment	1436

A.4.6.4.2.2	Test parameters	1437

A.4.6.4.2.3	Test Requirements	1440

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1440

A.4.6.4.3.1	Test Purpose and Environment	1440

A.4.6.4.3.2	Test parameters	1441

A.4.6.4.3.3	Test Requirements	1444

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1444

A.4.6.4.4.1	Test Purpose and Environment	1444

A.4.6.4.4.2	Test parameters	1445

A.4.6.4.4.3	Test Requirements	1447

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1447

A.4.6.4.5.1	Test Purpose and Environment	1447

A.4.6.4.5.2	Test parameters	1448

A.4.6.4.5.3	Test Requirements	1451

A.4.6.5	CLI measurements	1451

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1451

A.4.6.5.1.1	Test Purpose and Environment	1451

A.4.6.5.1.2	Test Parameters	1452

A.4.6.5.1.3	Test Requirements	1455

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1455

A.4.6.5.2.1	Test Purpose and Environment	1455

A.4.6.5.2.2	Test Parameters	1456

A.4.6.5.2.3	Test Requirements	1457

A.4.6.6.1.2	Test Requirements	1462

A.4.6.7	L1-SINR measurement for beam reporting	1462

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1465

A.4.6.7.2.1	Test Purpose and Environment	1465

A.4.6.7.2.2	Test parameters	1465

A.4.6.7.2.3	Test Requirements	1468

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1468

A.4.6.7.3.1	Test Purpose and Environment	1468

A.4.6.7.3.2	Test parameters	1468

A.4.6.7.3.3	Test Requirements	1470

A.4.6.8	CSI-RS based intra-frequency Measurement	1471

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1471

A.4.6.8.1.1	Test purpose and Environment	1471

A.4.6.8.1.2	Test Requirements	1475

A.4.6.9	CSI-RS based inter-frequency Measurement	1475

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1475

A.4.6.9.1.1	Test Purpose and Environment	1475

A.4.6.9.1.2	Test Requirements	1479

A.4.7	Measurement Performance requirements	1480

A.4.7.1	SS-RSRP	1480

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1480

A.4.7.1.1.1	Test Purpose and Environment	1480

A.4.7.1.1.2	Test parameters	1480

A.4.7.1.1.3	Test Requirements	1484

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1484

A.4.7.1.2.1	Test Purpose and Environment	1484

A.4.7.1.2.2	Test parameters	1485

A.4.7.1.2.3	Test Requirements	1489

A.4.7.1.3	Void	1489

A.4.7.2	SS-RSRQ	1489

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1489

A.4.7.2.1.1	Test Purpose and Environment	1489

A.4.7.2.1.2	Test Parameters	1490

A.4.7.2.1.3	Test Requirements	1495

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1495

A.4.7.2.2.1	Test Purpose and Environment	1495

A.4.7.2.2.2	Test Parameters	1495

A.4.7.2.2.3	Test Requirements	1500

A.4.7.3	SS-SINR	1500

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1500

A.4.7.3.1.1	Test Purpose and Environment	1500

A.4.7.3.1.2	Test Parameters	1500

A.4.7.3.1.3	Test Requirements	1504

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1504

A.4.7.3.2.1	Test Purpose and Environment	1504

A.4.7.3.2.2	Test Parameters	1504

A.4.7.3.2.3	Test Requirements	1509

A.4.7.4	L1-RSRP measurement for beam reporting	1509

A.4.7.4.1	SSB based L1-RSRP measurement	1509

A.4.7.4.1.1	Test Purpose and Environment	1509

A.4.7.4.1.2	Test parameters	1510

A.4.7.4.1.3	Test Requirements	1513

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1513

A.4.7.4.2.1	Test Purpose and Environment	1513

A.4.7.4.2.2	Test parameters	1514

A.4.7.4.2.3	Test Requirements	1517

A.4.7.5	SFTD accuracy	1517

A.4.7.5.1	SFTD accuracy	1517

A.4.7.5.1.1	Test Purpose and Environment	1517

A.4.7.5.1.2	Test Parameters	1517

A.4.7.5.1.3	Test Requirements	1522

A.4.7.5.2	Void	1522

A.4.7.5.3	Void	1522

A.4.7.6	CLI measurements	1522

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1522

A.4.7.6.1.1	Test Purpose and Environment	1522

A.4.7.6.1.2	Test parameters	1523

A.4.7.6.1.3	Test Requirements	1528

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1528

A.4.7.6.2.1	Test Purpose and Environment	1528

A.4.7.6.2.2	Test parameters	1529

A.4.7.6.2.3	Test Requirements	1532

A.4.7.7	L1-SINR measurement for beam reporting	1532

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1536

A.4.7.7.2.1	Test Purpose and Environment	1536

A.4.7.7.2.2	Test parameters	1536

A.4.7.7.2.3	Test Requirements	1540

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1540

A.4.7.7.3.1	Test Purpose and Environment	1540

A.4.7.7.3.2	Test parameters	1540

A.4.7.7.3.3	Test Requirements	1543

A.4.7.8	CSI-RSRP	1543

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1543

A.4.7.8.1.1	Test Purpose and Environment	1543

A.4.7.8.1.2	Test parameters	1544

A.4.7.8.1.3	Test Requirements	1549

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1549

A.4.7.8.2.1	Test Purpose and Environment	1549

A.4.7.8.2.2	Test parameters	1549

A.4.7.8.2.3	Test Requirements	1554

A.4.7.9	CSI-RSRQ	1554

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1554

A.4.7.9.1.1	Test Purpose and Environment	1554

A.4.7.9.1.2	Test Parameters	1554

A.4.7.9.1.3	Test Requirements	1559

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1559

A.4.7.9.2.1	Test Purpose and Environment	1559

A.4.7.9.2.2	Test Parameters	1559

A.4.7.9.2.3	Test Requirements	1564

A.4.7.10	CSI-SINR	1564

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1564

A.4.7.10.1.1	Test Purpose and Environment	1564

A.4.7.10.1.2	Test Parameters	1565

A.4.7.10.1.3	Test Requirements	1569

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1569

A.4.7.10.2.1	Test Purpose and Environment	1569

A.4.7.10.2.2	Test Parameters	1570

A.4.7.10.2.3	Test Requirements	1574

A.4.8	Void	1574

A.4A	NE-DC test with all NR cells in FR1	1575

A.4A.1	Signaling characteristics	1575

A.4A.1.1	E-UTRAN PSCell addition	1575

A.4A.1.1.1	Test purpose and environment	1575

A.4A.1.1.2	Test Requirements	1579

A.4A.1.2	Active BWP switch	1580

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1580

A.4A.1.2.1.1	Test Purpose and Environment	1580

A.4A.1.2.1.2	Test Requirements	1584

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1585

A.4A.1.3.1	Test purpose and environment	1585

A.4A.1.3.2	Test Requirements	1589

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1590

A.4A.1.4.1	Test Purpose and Environment	1590

A.4A.1.4.2	Test Parameters	1590

A.4A.1.4.3	Test Requirements	1595

A.4A.1.4.3.1	Test Requirements for NR HO	1595

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1595

A.4A.2	Measurement performance	1596

A.4A.2.1	SFTD accuracy	1596

A.4A.2.1.1	SFTD accuracy	1596

A.4A.2.1.1.1	Test Purpose	1596

A.4A.2.1.1.2	Test Environment	1596

A.4A.2.1.1.3	Test Requirements	1599

A.5	EN-DC tests with one or more NR cells in FR2	1600

A.5.1	Void	1600

A.5.2	Void	1600

A.5.3	RRC_CONNECTED state mobility	1600

A.5.3.1	Void	1600

A.5.3.2	RRC Connection Mobility Control	1600

A.5.3.2.1	Void	1600

A.5.3.2.2	Random Access	1600

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1600

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1603

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1609

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1612

A.5.3.2.3	Void	1616

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1616

A.5.3.3.1	Test purpose and environment	1616

A.5.3.3.2	Test Requirements	1620

A.5.5.3.3	PUCCH SCell Activation and deactivation of known SCell in FR2	1621

A.5.5.3.3.1	Test Purpose and Environment	1621

A.5.5.3.3.2	Test Requirements	1624

A.5.5.3.4	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1624

A.5.5.3.4.1	Test Purpose and Environment	1624

A.5.5.3.4.2	Test Requirements	1628

A.5.5.3.5	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1628

A.5.5.3.5.1	Test Purpose and Environment	1628

A.5.5.3.5.2	Test Requirements	1632

A.5.5.3.6	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1633

A.5.5.3.6.1	Test Purpose and Environment	1633

A.5.5.3.6.2	Test Requirements	1636

A.5.4	Timing	1637

A.5.4.1	UE transmit timing	1637

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1637

A.5.4.1.1.1	Test Purpose and environment	1637

A.5.4.1.1.2	Test requirements	1640

A.5.4.2	UE timer accuracy	1641

A.5.4.3	Timing advance	1641

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1641

A.5.4.3.1.1 Test Purpose and Environment	1641

A.5.4.3.1.2 Test Parameters	1641

A.5.4.3.1.3	Test Requirements	1645

A.5.5	Signaling characteristics	1645

A.5.5.1	Radio link Monitoring	1645

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1645

A.5.5.1.1.1	Test Purpose and Environment	1645

A.5.5.1.1.2	Test Requirements	1649

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1650

A.5.5.1.2.1	Test Purpose and Environment	1650

A.5.5.1.2.2	Test Requirements	1653

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1654

A.5.5.1.3.1	Test Purpose and Environment	1654

A.5.5.1.3.2	Test Requirements	1658

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1658

A.5.5.1.4.1	Test Purpose and Environment	1658

A.5.5.1.4.2	Test Requirements	1662

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1662

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1666

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1670

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1675

A.5.5.1.8.2	Test Requirements	1679

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1680

A.5.5.1.9.1	Test Purpose and Environment	1680

A.5.5.1.9.2	Test Requirements	1682

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1682

A.5.5.1.10.1	Test Purpose and Environment	1682

A.5.5.1.10.2	Test Requirements	1685

A.5.5.2	Interruption	1685

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1685

A.5.5.2.1.1	Test Purpose and Environment	1685

A.5.5.2.1.2	Test Requirements	1688

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1688

A.5.5.2.2.1	Test Purpose and Environment	1688

A.5.5.2.2.2	Test Requirements	1691

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1691

A.5.5.2.3.1	Test Purpose and Environment	1691

A.5.5.2.3.2	Test Requirements	1695

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1695

A.5.5.2.4.1	Test Purpose and Environment	1695

A.5.5.2.4.2	Test Requirements	1698

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1699

A.5.5.2.5.1	Test Purpose and Environment	1699

A.5.5.2.5.2	Test Requirements	1702

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1702

A.5.5.2.6.1	Test Purpose and Environment	1702

A.5.5.2.6.2	Test Requirements	1705

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1705

A.5.5.2.7.1	Test Purpose and Environment	1705

A.5.5.2.7.2	Test Requirements	1709

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1709

A.5.5.2.8.1 Test Purpose and Environment	1709

A.5.5.2.8.3	Test Requirements	1711

A.5.5.3	SCell Activation and Deactivation Delay	1711

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1711

A.5.5.3.1.1	Test Purpose and Environment	1711

A.5.5.3.1.2	Test Requirements	1713

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1713

A.5.5.3.2.1	Test Purpose and Environment	1713

A.5.5.3.2.2	Test Requirements	1717

A.5.5.3.3	Void	1717

A.5.5.3.4	Void	1717

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1717

A.5.5.3.5.1	Test Purpose and Environment	1717

A.5.5.3.5.2	Test Requirements	1720

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1721

A.5.5.3.6.1	Test Purpose and Environment	1721

A.5.5.3.6.2	Test Requirements	1725

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1726

A.5.5.3.7.1	Test Purpose and Environment	1726

A.5.5.3.7.2	Test Requirements	1729

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1729

A.5.5.3.8.1	Test Purpose and Environment	1729

A.5.5.3.8.2	Test Requirements	1733

A.5.5.4	Void	1734

A.5.5.5	Beam Failure Detection and Link recovery procedures	1734

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1734

A.5.5.5.1.1	Test Purpose and Environment	1734

A.5.5.5.1.2	Test Requirements	1738

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1738

A.5.5.5.2.1	Test Purpose and Environment	1738

A.5.5.5.2.2	Test Requirements	1742

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1742

A.5.5.5.3.1	Test Purpose and Environment	1742

A.5.5.5.3.2	Test Requirements	1746

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1746

A.5.5.5.4.1	Test Purpose and Environment	1746

A.5.5.5.4.2	Test Requirements	1750

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1750

A.5.5.5.5.1	Test Purpose and Environment	1750

A.5.5.5.5.2	Test Requirements	1754

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1754

A.5.5.5.6.1	Test Purpose and Environment	1754

A.5.5.5.6.2	Test Requirements	1758

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1758

A.5.5.5.7.1	Test Purpose and Environment	1758

A.5.5.5.7.2	Test Requirements	1762

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1762

A.5.5.5.8.1	Test Purpose and Environment	1762

A.5.5.5.8.2	Test Requirements	1767

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1767

A.5.5.5.9.1	Test Purpose and Environment	1767

A.5.5.5.9.2	Test Requirements	1771

A.5.5.6	Active BWP switch	1771

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1771

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1771

A.5.5.6.1.1.1	Test Purpose and Environment	1771

A.5.5.6.1.1.2	Test Requirements	1775

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1775

A.5.5.6.2	RRC-based Active BWP Switch	1780

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1780

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1784

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1784

A.5.5.6.3.1.1	Test Purpose and Environment	1784

A.5.5.6.3.1.2	Test Requirements	1788

A.5.5.6.4	SCell dormancy switch	1788

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1788

A.5.5.6.4.1.1	Test Purpose and Environment	1788

A.5.5.6.4.1.2	Test Requirements	1792

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1792

A.5.5.6.4.2.1	Test Purpose and Environment	1792

A.5.5.6.4.2.2	Test Requirements	1799

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1799

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1799

A.5.5.7	PSCell addition and release delay	1802

A.5.5.7.1	Addition and Release Delay of NR PSCell	1802

A.5.5.7.1.1	Test purpose and environment	1802

A.5.5.7.1.2	Test Requirements	1805

A.5.5.8	Active TCI state switch delay	1806

A.5.5.8.1	MAC-CE based active TCI state switch	1806

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1806

A.5.5.8.1.1.1	Test Purpose and Environment	1806

A.5.5.8.1.1.2	Test Requirements	1809

A.5.5.8.2	RRC based active TCI state switch	1809

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1809

A.5.5.8.2.1.1	Test Purpose and Environment	1809

A.5.5.8.2.1.2	Test Requirements	1813

A.5.5.9	Uplink spatial relation switch delay	1813

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1813

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1813

A.5.5.9.1.1.1	Test Purpose and Environment	1813

A.5.5.9.1.1.2	Test Requirements	1816

A.5.5.9.2	RRC based spatial relation switch	1816

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1816

A.5.5.9.2.1.1	Test Purpose and Environment	1816

A.5.5.9.2.1.2	Test Requirements	1819

A.5.5.10	UE specific CBW change	1819

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1819

A.5.5.10.1.1	Test Purpose and Environment	1819

A.5.5.10.1.2	Test Requirements	1822

A.5.5.11	Unified TCI state switch delay	1823

A.5.5.11.1	MAC-CE based active joint TCI state switch	1823

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	1823

A.5.5.11.1.1.1	Test Purpose and Environment	1823

A.5.5.11.1.1.2	Test parameters	1823

A.5.5.11.1.1.3	Test Requirements	1825

A.5.5.11.2	MAC-CE based active uplink TCI state switch	1825

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	1825

A.5.5.11.2.1.1	Test Purpose and Environment	1825

A.5.5.11.2.1.2	Test parameters	1826

A.5.5.11.2.1.3	Test Requirements	1828

A.5.5.11.3	MAC-CE based active downlink TCI state switch	1828

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	1828

A.5.5.11.3.1.1	Test Purpose and Environment	1828

A.5.5.11.3.1.2	Test Parameters	1829

A.5.5.11.3.1.3	Test Requirements	1832

A.5.5.12	PSCell activation and deactivation delay	1832

A.5.5.12.1	PSCell activation and deactivation delay	1832

A.5.5.12.1.1	Test purpose and environment	1832

A.5.5.12.1.2	Test Requirements	1835

A.5.5.13	Conditional PSCell addition and release delay	1836

A.5.5.13.1	Addition and Release Delay of NR PSCell	1836

A.5.5.13.1.1	Test purpose and environment	1836

A.5.5.13.1.2	Test Requirements	1839

A.5.6	Measurement procedure	1840

A.5.6.1	Intra-frequency Measurements	1840

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1840

A.5.6.1.1.1	Test purpose and Environment	1840

A.5.6.1.1.2	Test Requirements	1843

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1843

A.5.6.1.2.1	Test purpose and Environment	1843

A.5.6.1.2.2	Test Requirements	1845

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1846

A.5.6.1.3.1	Test purpose and Environment	1846

A.5.6.1.3.2	Test Requirements	1850

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1850

A.5.6.1.4.1	Test purpose and Environment	1850

A.5.6.1.4.2	Test Requirements	1853

A.5.6.2	Inter-frequency Measurements	1854

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1854

A.5.6.2.1.1	Test Purpose and Environment	1854

A.5.6.2.1.2	Test Requirements	1857

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1857

A.5.6.2.2.1	Test Purpose and Environment	1857

A.5.6.2.2.2	Test Requirements	1861

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1861

A.5.6.2.3.1	Test Purpose and Environment	1861

A.5.6.2.3.2	Test Requirements	1865

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1865

A.5.6.2.4.1	Test Purpose and Environment	1865

A.5.6.2.4.2	Test Requirements	1869

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1869

A.5.6.2.5.1	Test Purpose and Environment	1869

A.5.6.2.5.2	Test Requirements	1874

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1874

A.5.6.2.6.1	Test Purpose and Environment	1874

A.5.6.2.6.2	Test Requirements	1878

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1878

A.5.6.2.7.1	Test Purpose and Environment	1878

A.5.6.2.7.2	Test Requirements	1883

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1883

A.5.6.2.8.1	Test Purpose and Environment	1883

A.5.6.2.8.2	Test Requirements	1888

A.5.6.3	L1-RSRP measurement for beam reporting	1889

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1889

A.5.6.3.1.1	Test Purpose and Environment	1889

A.5.6.3.1.2	Test parameters	1889

A.5.6.3.1.3	Test Requirements	1891

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1891

A.5.6.3.2.1	Test Purpose and Environment	1891

A.5.6.3.2.2	Test parameters	1892

A.5.6.3.2.3	Test Requirements	1894

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1894

A.5.6.3.3.1	Test Purpose and Environment	1894

A.5.6.3.3.2	Test parameters	1895

A.5.6.3.3.3	Test Requirements	1897

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1897

A.5.6.3.4.1	Test Purpose and Environment	1897

A.5.6.3.4.2	Test parameters	1898

A.5.6.3.4.3	Test Requirements	1900

A.5.6.4	CLI measurements	1901

A.5.6.4.1	SRS-RSRP measurement with DRX	1901

A.5.6.4.1.1	Test Purpose and Environment	1901

A.5.6.4.1.2	Test Parameters	1901

A.5.6.4.1.3	Test Requirements	1903

A.5.6.4.2	CLI-RSSI measurement with DRX	1903

A.5.6.4.2.1	Test Purpose and Environment	1903

A.5.6.4.2.2	Test Parameters	1904

A.5.6.4.2.3	Test Requirements	1906

A.5.6.5	Measurements with autonomous gaps	1906

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1906

A.5.6.5.1.1	Test Purpose and Environment	1906

A.5.6.5.1.2	Test Requirements	1909

A.5.6.6	L1-SINR measurement for beam reporting	1910

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1912

A.5.6.6.2.1	Test Purpose and Environment	1912

A.5.6.6.2.2	Test parameters	1913

A.5.6.6.2.3	Test Requirements	1916

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1916

A.5.6.6.3.1	Test Purpose and Environment	1916

A.5.6.6.3.2	Test parameters	1917

A.5.6.6.3.3	Test Requirements	1919

A.5.6.7	CSI-RS based Intra-frequency Measurements	1919

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1919

A.5.6.7.1.1	Test purpose and Environment	1919

A.5.6.7.1.2	Test Requirements	1922

A.5.6.8	CSI-RS based Inter-frequency Measurements	1922

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1922

A.5.6.8.1.1	Test Purpose and Environment	1922

A.5.6.8.1.2	Test Requirements	1927

A.5.7	Measurement Performance requirements	1927

A.5.7.1	SS-RSRP	1927

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1927

A.5.7.1.1.1	Test Purpose and Environment	1927

A.5.7.1.1.2	Test parameters	1928

A.5.7.1.1.3	Test Requirements	1930

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1931

A.5.7.1.2.1	Test Purpose and Environment	1931

A.5.7.1.2.2	Test parameters	1931

A.5.7.1.2.3	Test Requirements	1935

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1936

A.5.7.1.3.1	Test Purpose and Environment	1936

A.5.7.1.3.2	Test parameters	1936

A.5.7.1.3.3	Test Requirements	1939

A.5.7.2	SS-RSRQ	1940

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1940

A.5.7.2.1.1	Test Purpose and Environment	1940

A.5.7.2.1.2	Test Parameters	1940

A.5.7.2.1.3	Test Requirements	1943

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1943

A.5.7.2.2.1	Test Purpose and Environment	1943

A.5.7.2.2.2	Test Parameters	1943

A.5.7.2.2.3	Test Requirements	1945

A.5.7.3	SS-SINR	1946

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1946

A.5.7.3.1.1	Test Purpose and Environment	1946

A.5.7.3.1.2	Test Parameters	1946

A.5.7.3.1.3	Test Requirements	1949

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1949

A.5.7.3.2.1	Test Purpose and Environment	1949

A.5.7.3.2.2	Test Parameters	1949

A.5.7.3.2.3	Test Requirements	1951

A.5.7.4	L1-RSRP measurement for beam reporting	1951

A.5.7.4.1	SSB based L1-RSRP measurement	1951

A.5.7.4.1.1	Test Purpose and Environment	1951

A.5.7.4.1.2	Test parameters	1952

A.5.7.4.1.3	Test Requirements	1954

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1955

A.5.7.4.2.1	Test Purpose and Environment	1955

A.5.7.4.2.2	Test parameters	1955

A.5.7.4.2.3	Test Requirements	1957

A.5.7.5	CLI measurements	1958

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1958

A.5.7.5.1.1	Test Purpose and Environment	1958

A.5.7.5.1.2	Test parameters	1958

A.5.7.5.1.3	Test Requirements	1961

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1962

A.5.7.5.2.1	Test Purpose and Environment	1962

A.5.7.5.2.2	Test parameters	1962

A.5.7.5.2.3	Test Requirements	1964

A.5.7.6	L1-SINR measurement for beam reporting	1965

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1967

A.5.7.6.2.1	Test Purpose and Environment	1967

A.5.7.6.2.2	Test parameters	1968

A.5.7.6.2.3	Test Requirements	1970

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1971

A.5.7.6.3.1	Test Purpose and Environment	1971

A.5.7.6.3.2	Test parameters	1971

A.5.7.6.3.3	Test Requirements	1973

A.5.7.7	CSI-RSRP	1973

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1973

A.5.7.7.1.1	Test Purpose and Environment	1973

A.5.7.7.1.2	Test parameters	1974

A.5.7.7.1.3	Test Requirements	1976

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1977

A.5.7.7.2.1	Test Purpose and Environment	1977

A.5.7.7.2.2	Test parameters	1977

A.5.7.7.2.3	Test Requirements	1981

A.5.7.8	CSI-RSRQ	1982

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1982

A.5.7.8.1.1	Test Purpose and Environment	1982

A.5.7.8.1.2	Test Parameters	1982

A.5.7.8.1.3	Test Requirements	1984

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1984

A.5.7.8.2.1	Test Purpose and Environment	1984

A.5.7.8.2.2	Test Parameters	1984

A.5.7.8.2.3	Test Requirements	1986

A.5.7.9	CSI-SINR	1986

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1986

A.5.7.9.1.1	Test Purpose and Environment	1986

A.5.7.9.1.2	Test Parameters	1986

A.5.7.9.1.3	Test Requirements	1989

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1989

A.5.7.9.2.1	Test Purpose and Environment	1989

A.5.7.9.2.2	Test Parameters	1989

A.5.7.9.2.3	Test Requirements	1992

A.5.8	Void	1992

A.6	NR standalone tests with all NR cells in FR1	1989

A.6.1	SA: RRC_IDLE state mobility	1989

A.6.1.1	Cell re-selection to NR	1989

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1989

A.6.1.1.1.1	Test Purpose and Environment	1989

A.6.1.1.1.2	Test Parameters	1989

A.6.1.1.1.3	Test Requirements	1993

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1993

A.6.1.1.2.1	Test Purpose and Environment	1993

A.6.1.1.2.2	Test Parameters	1993

A.6.1.1.2.3	Test Requirements	1997

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1997

A.6.1.1.3.1	Test Purpose and Environment	1997

A.6.1.1.3.2	Test Parameters	1997

A.6.1.1.3.3	Test Requirements	2002

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2002

A.6.1.1.4.1	Test Purpose and Environment	2002

A.6.1.1.4.2	Test Parameters	2002

A.6.1.1.4.3	Test Requirements	2005

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2005

A.6.1.1.5.1	Test Purpose and Environment	2005

A.6.1.1.5.2	Test Parameters	2005

A.6.1.1.5.3	Test Requirements	2010

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2010

A.6.1.1.6.1	Test Purpose and Environment	2010

A.6.1.1.6.2	Test Parameters	2011

A.6.1.1.6.3	Test Requirements	2015

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2016

A.6.1.1.7.1	Test Purpose and Environment	2016

A.6.1.1.7.2	Test Parameters	2016

A.6.1.1.7.3	Test Requirements	2019

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2019

A.6.1.1.8.1	Test Purpose and Environment	2019

A.6.1.1.8.2	Test Parameters	2019

A.6.1.1.8.3	Test Requirements	2023

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2023

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2023

A.6.1.2.1.1	Test Purpose and Environment	2023

A.6.1.2.1.2	Test Parameters	2023

A.6.1.2.1.3	Test Requirements	2026

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2027

A.6.1.2.2.1	Test Purpose and Environment	2027

A.6.1.2.2.2	Test Parameters	2027

A.6.1.2.2.3	Test Requirements	2030

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2031

A.6.1.2.3.1	Test Purpose and Environment	2031

A.6.1.2.3.2	Test Parameters	2031

A.6.1.2.3.3	Test Requirements	2034

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2035

A.6.1.2.4.1	Test Purpose and Environment	2035

A.6.1.2.4.2	Test Parameters	2035

A.6.1.2.4.3	Test Requirements	2038

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2039

A.6.1.2.5.1	Test Purpose and Environment	2039

A.6.1.2.5.2	Test Parameters	2039

A.6.1.2.5.3	Test Requirements	2043

A.6.1.1.7	Void	2044

A.6.2	SA: RRC_INACTIVE state mobility	2044

A.6.3	RRC_CONNECTED state mobility	2044

A.6.3.1	Handover	2044

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2044

A.6.3.1.1.1	Test Purpose and Environment	2044

A.6.3.1.1.2	Test Parameters	2044

A.6.3.1.1.3 Test Requirements	2048

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2048

A.6.3.1.2.1	Test Purpose and Environment	2048

A.6.3.1.2.2	Test Parameters	2048

A.6.3.1.2.3	Test Requirements	2052

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2052

A.6.3.1.3.1	Test Purpose and Environment	2052

A.6.3.1.3.2	Test Parameters	2052

A.6.3.1.3.3	Test Requirements	2056

A.6.3.1.4	SA NR - E-UTRAN handover	2056

A.6.3.1.4.1	Test Purpose and Environment	2056

A.6.3.1.4.2	Test Requirements	2062

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2062

A.6.3.1.5.1	Test Purpose and Environment	2062

A.6.3.1.5.2	Test Requirements	2068

A.6.3.1.6	 SA NR - UTRAN FDD handover	2068

A.6.3.1.6.1	Test Purpose and Environment	2068

A.6.3.1.6.2	Test Requirements	2072

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2072

A.6.3.1.7.1	Test Purpose and Environment	2072

A.6.3.1.7.2	Test Parameters	2072

A.6.3.1.7.3	Test Requirements	2076

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2077

A.6.3.1.8.1	Test Purpose and Environment	2077

A.6.3.1.8.2	Test Parameters	2077

A.6.3.1.8.3	Test Requirements	2081

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2082

A.6.3.1.9.1	Test Purpose and Environment	2082

A.6.3.1.9.2	Test Parameters	2082

A.6.3.1.9.3	Test Requirements	2086

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2086

A.6.3.1.10.1	Test Purpose and Environment	2086

A.6.3.1.10.2	Test Parameters	2086

A.6.3.1.10.3	Test Requirements	2089

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2089

A.6.3.1.11.1	Test Purpose and Environment	2089

A.6.3.1.11.2	Test Parameters	2089

A.6.3.1.11.3 Test Requirements	2096

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2096

A.6.3.1.12.1	Test Purpose and Environment	2096

A.6.3.1.12.2	Test Parameters	2096

A.6.3.1.12.3 Test Requirements	2104

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2104

A.6.3.1.13.1	Test Purpose and Environment	2104

A.6.3.1.13.2	Test Requirements	2113

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2113

A.6.3.1.14.1	Test Purpose and Environment	2113

A.6.3.1.14.2	Test Requirements	2122

A.6.3.2	RRC Connection Mobility Control	2123

A.6.3.2.1	SA: RRC Re-establishment	2123

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2123

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2127

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2131

A.6.3.2.2	Random Access	2134

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2134

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2138

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2143

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2148

A.6.3.2.3	SA: RRC Connection Release with Redirection	2152

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2152

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2156

A.6.3.3 Conditional handover	2163

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2163

A.6.3.3.1.1	Test Purpose and Environment	2163

A.6.3.3.1.2	Test Parameters	2163

A.6.3.3.1.3 Test Requirements	2167

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2167

A.6.3.3.2.1	Test Purpose and Environment	2167

A.6.3.3.2.2	Test Parameters	2167

A.6.3.3.2.3 Test Requirements	2171

A.6.4	Timing	2171

A.6.4.1	UE transmit timing	2171

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2171

A.6.4.1.1.1	Test Purpose and environment	2171

A.6.4.1.1.2	Test requirements	2175

A.6.4.2	UE timer accuracy	2175

A.6.4.3	Timing advance	2175

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2175

A.6.4.3.1.1	Test Purpose and Environment	2175

A.6.4.3.1.2	Test Parameters	2175

A.6.4.3.1.3	Test Requirements	2179

A.6.5	Signalling characteristics	2179

A.6.5.1	Radio link Monitoring	2179

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2180

A.6.5.1.1.1	Test Purpose and Environment	2180

A.6.5.1.1.2	Test Requirements	2185

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2185

A.6.5.1.2.1	Test Purpose and Environment	2185

A.6.5.1.2.2	Test Requirements	2191

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2191

A.6.5.1.3.1	Test Purpose and Environment	2191

A.6.5.1.3.2	Test Requirements	2197

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2197

A.6.5.1.4.1	Test Purpose and Environment	2197

A.6.5.1.4.2	Test Requirements	2203

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2203

A.6.5.1.5.1	Test Purpose and Environment	2203

A.6.5.1.5.2	Test Requirements	2209

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2209

A.6.5.1.6.1	Test Purpose and Environment	2209

A.6.5.1.6.2	Test Requirements	2214

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2214

A.6.5.1.7.1	Test Purpose and Environment	2214

A.6.5.1.7.2	Test Requirements	2219

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2219

A.6.5.1.8.1	Test Purpose and Environment	2219

A.6.5.1.8.2	Test Requirements	2224

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2224

A.6.5.1.9.1	Test Purpose and Environment	2224

A.6.5.1.9.2	Test Requirements	2230

A.6.5.2	Interruption	2230

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2230

A.6.5.2.1.2	Test Requirements	2234

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2235

A.6.5.2.2.1	Test Purpose and Environment	2235

A.6.5.2.2.2	Test Parameters	2235

A.6.5.2.2.3	Test Requirements	2237

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2238

A.6.5.2.3.1	Test Purpose and Environment	2238

A.6.5.2.3.2	Test Parameters	2238

A.6.5.2.3.3	Test Requirements	2240

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2241

A.6.5.2.4.1	Test Purpose and Environment	2241

A.6.5.2.4.2	Test Parameters	2241

A.6.5.2.4.3	Test Requirements	2243

A.6.5.3	SCell Activation and Deactivation Delay	2244

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2244

A.6.5.3.1.1	Test Purpose and Environment	2244

A.6.5.3.1.2	Test Requirements	2249

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2250

A.6.5.3.2.1	Test Purpose and Environment	2250

A.6.5.3.2.2	Test Requirements	2250

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2250

A.6.5.3.3.1	Test Purpose and Environment	2250

A.6.5.3.3.2	Test Requirements	2251

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2251

A.6.5.3.4.1	Test Purpose and Environment	2251

A.6.5.3.4.2	Test Requirements	2255

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2255

A.6.5.3.5.1	Test Purpose and Environment	2255

A.6.5.3.5.2	Test Requirements	2260

A.6.5.3.4	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2260

A.6.5.3.4.1	Test Purpose and Environment	2260

A.6.5.3.4.2	Test Requirements	2264

A.6.5.3.5	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2264

A.6.5.3.5.1	Test Purpose and Environment	2264

A.6.5.3.5.2	Test Requirements	2265

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2265

A.6.5.3.6.1	Test Purpose and Environment	2265

A.6.5.3.6.2	Test Requirements	2269

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2269

A.6.5.3.7.1	Test Purpose and Environment	2269

A.6.5.3.7.2	Test Requirements	2273

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2274

A.6.5.3.8.1	Test Purpose and Environment	2274

A.6.5.3.8.2	Test Requirements	2278

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2279

A.6.5.3.9.1	Test Purpose and Environment	2279

A.6.5.3.9.2	Test Requirements	2283

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2284

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2284

A.6.5.4.1.1	Test Purpose and Environment	2284

A.6.5.4.1.2	Test Requirements	2294

A.6.5.4.2	Void	2295

A.6.5.5	Beam Failure Detection and Link recovery procedures	2295

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2295

A.6.5.5.1.1	Test Purpose and Environment	2295

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2300

A.6.5.5.2.1	Test Purpose and Environment	2300

A.6.5.5.2.2	Test Requirements	2307

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2307

A.6.5.5.3.1	Test Purpose and Environment	2307

A.6.5.5.3.2	Test Requirements	2313

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2313

A.6.5.5.4.1	Test Purpose and Environment	2313

A.6.5.5.4.2	Test Requirements	2319

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2319

A.6.5.5.5.1	Test Purpose and Environment	2319

A.6.5.5.5.2	Test Requirements	2323

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2324

A.6.5.5.6.1	Test Purpose and Environment	2324

A.6.5.5.6.2	Test Requirements	2328

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2328

A.6.5.5.7.1	Test Purpose and Environment	2328

A.6.5.5.7.2	Test Requirements	2334

A.6.5.6	Active BWP switch	2335

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2335

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2335

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2341

A.6.5.6.2	RRC-based Active BWP Switch	2346

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2346

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2350

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2350

A.6.5.6.3.1.1	Test Purpose and Environment	2350

A.6.5.6.3.1.2	Test Requirements	2357

A.6.5.6.4	SCell dormancy switch	2357

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2357

A.6.5.6.4.1.1	Test Purpose and Environment	2357

A.6.5.6.4.1.2	Test Requirements	2363

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2363

A.6.5.6.4.2.1	 Test Purpose and Environment	2363

A.6.5.6.4.2.2	Test Requirements	2370

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2370

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2370

A.6.5.7	DL interruptions at switching between two uplink carriers	2375

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2375

A.6.5.7.1.1	Test Purpose and Environment	2375

A.6.5.7.1.2	Test Requirements	2379

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2379

A.6.5.7.2.1	Test Purpose and Environment	2379

A.6.5.7.2.2	Test Requirements	2383

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2383

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2383

A.6.5.7A.1.1	Test Purpose and Environment	2383

A.6.5.7A.1.2	Test Requirements	2387

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2387

A.6.5.7A.2.1	Test Purpose and Environment	2387

A.6.5.7A.2.2	Test Requirements	2391

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2391

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2391

A.6.5.7B.1.1	Test Purpose and Environment	2391

A.6.5.7B.1.2	Test Requirements	2395

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2395

A.6.5.7B.2.1	Test Purpose and Environment	2395

A.6.5.7B.2.2	Test Requirements	2399

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2399

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2399

A.6.5.7C.1.1	Test Purpose and Environment	2399

A.6.5.7C.1.2	Test Requirements	2403

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2404

A.6.5.7C.2.1	Test Purpose and Environment	2404

A.6.5.7C.2.2	Test Requirements	2408

A.6.5.8	UE specific CBW change	2409

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2409

A.6.5.8.1.1	Test Purpose and Environment	2409

A.6.5.8.1.2	Test Requirements	2413

A.6.5.9	Pathloss reference signal switching delay	2413

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2413

A.6.5.9.1.1	Test Purpose and Environment	2413

A.6.5.9.1.2	Test Requirements	2416

A.6.6	Measurement procedure	2417

A.6.6.1	Intra-frequency Measurements	2417

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2417

A.6.6.1.1.1	Test purpose and Environment	2417

A.6.6.1.1.2	Test parameters	2417

A.6.6.1.1.3	Test Requirements	2421

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2421

A.6.6.1.2.1	Test purpose and Environment	2421

A.6.6.1.2.2	Test parameters	2421

A.6.6.1.2.3	Test Requirements	2425

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2425

A.6.6.1.3.1	Test purpose and Environment	2425

A.6.6.1.3.2	Test parameters	2425

A.6.6.1.3.3	Test Requirements	2429

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2429

A.6.6.1.4.1	Test purpose and Environment	2429

A.6.6.1.4.2	Test parameters	2429

A.6.6.1.4.3	Test Requirements	2433

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2433

A.6.6.1.5.1	Test purpose and Environment	2433

A.6.6.1.5.2	Test parameters	2433

A.6.6.1.5.3	Test Requirements	2435

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2436

A.6.6.1.6.1	Test purpose and Environment	2436

A.6.6.1.6.2	Test parameters	2436

A.6.6.1.6.3	Test Requirements	2437

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2438

A.6.6.1.7.1	Test purpose and Environment	2438

A.6.6.1.7.2	Test parameters	2438

A.6.6.1.7.3	Test Requirements	2442

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2442

A.6.6.1.8.1	Test purpose and Environment	2442

A.6.6.1.8.2	Test parameters	2442

A.6.6.1.8.3	Test Requirements	2446

A.6.6.2	Inter-frequency Measurements	2446

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2446

A.6.6.2.1.1	Test Purpose and Environment	2446

A.6.6.2.1.2	Test Requirements	2450

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2450

A.6.6.2.2.1	Test Purpose and Environment	2450

A.6.6.2.2.2	Test Requirements	2454

A.6.6.2.3	Void	2455

A.6.6.2.4	Void	2455

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2455

A.6.6.2.5.1	Test Purpose and Environment	2455

A.6.6.2.5.2	Test Requirements	2459

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2459

A.6.6.2.6.1	Test Purpose and Environment	2459

A.6.6.2.6.2	Test Requirements	2463

A.6.6.2.7	Void	2464

A.6.6.2.8	Void	2464

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2464

A.6.6.2.9.1	Test Purpose and Environment	2464

A.6.6.2.9.2	Test Requirements	2467

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2467

A.6.6.2.10.1	Test Purpose and Environment	2467

A.6.6.2.10.2	Test Requirements	2471

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2475

A.6.6.2.12.1	Test Purpose and Environment	2475

A.6.6.2.12.2	Test Requirements	2479

A.6.6.3	Inter-RAT Measurements	2479

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2479

A.6.6.3.1.1	Test Purpose and Environment	2479

A.6.6.3.1.2	Test Requirements	2485

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2485

A.6.6.3.2.1	Test Purpose and Environment	2485

A.6.6.3.2.2	Test Requirements	2492

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2492

A.6.6.3.3.1	Test Purpose and Environment	2492

A.6.6.3.3.2	Test Requirements	2496

A.6.6.4	L1-RSRP measurement for beam reporting	2496

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2496

A.6.6.4.1.1	Test Purpose and Environment	2496

A.6.6.4.1.2	Test parameters	2497

A.6.6.4.1.3	Test Requirements	2500

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2500

A.6.6.4.2.1	Test Purpose and Environment	2500

A.6.6.4.2.2	Test parameters	2501

A.6.6.4.2.3	Test Requirements	2504

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2504

A.6.6.4.3.1	Test Purpose and Environment	2504

A.6.6.4.3.2	Test parameters	2505

A.6.6.4.3.3	Test Requirements	2508

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2508

A.6.6.4.4.1	Test Purpose and Environment	2508

A.6.6.4.4.2	Test parameters	2509

A.6.6.4.4.3	Test Requirements	2512

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2512

A.6.6.4.5.1	Test Purpose and Environment	2512

A.6.6.4.5.2	Test parameters	2513

A.6.6.4.5.3	Test Requirements	2515

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2515

A.6.6.4.6.1	Test Purpose and Environment	2515

A.6.6.4.6.2	Test parameters	2515

A.6.6.4.6.3	Test Requirements	2518

A.6.6.5	Inter-RAT UTRAN FDD measurements	2519

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2519

A.6.6.5.1.1	Test Purpose and Environment	2519

A.6.6.5.1.2	Test Requirements	2522

A.6.6.6	CLI measurements	2522

A.6.6.6.1	SRS-RSRP measurement with DRX	2522

A.6.6.6.1.1	Test Purpose and Environment	2522

A.6.6.6.1.2	Test Parameters	2523

A.6.6.6.1.3	Test Requirements	2526

A.6.6.6.2	CLI-RSSI measurement with DRX	2526

A.6.6.6.2.1	Test Purpose and Environment	2526

A.6.6.6.2.2	Test Parameters	2527

A.6.6.6.2.3	Test Requirements	2529

A.6.6.7	NR measurements with autonomous gaps	2529

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2529

A.6.6.7.1.1	Test Purpose and Environment	2529

A.6.6.7.1.2	Test Parameters	2529

A.6.6.7.1.3	Test Requirements	2532

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2532

A.6.6.7.2.1	Test Purpose and Environment	2532

A.6.6.7.2.2	Test Requirements	2535

A.6.6.8	L1-SINR measurement for beam reporting	2536

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2538

A.6.6.8.2.1	Test Purpose and Environment	2538

A.6.6.8.2.2	Test parameters	2538

A.6.6.8.2.3	Test Requirements	2542

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2542

A.6.6.8.3.1	Test Purpose and Environment	2542

A.6.6.8.3.2	Test parameters	2543

A.6.6.8.3.3	Test Requirements	2545

A.6.6.9	Idle Mode CA/DC Measurements	2545

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2545

A.6.6.9.1.1	Test Purpose and Environment	2545

A.6.6.9.1.2	Test Requirements	2552

A.6.6.10	CSI-RS based intra-frequency Measurements	2552

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2552

A.6.6.10.1.1	Test purpose and Environment	2552

A.6.6.10.1.2	Test Requirements	2555

A.6.6.11	CSI-RS based inter-frequency Measurements	2555

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2555

A.6.6.11.1.1	Test Purpose and Environment	2555

A.6.6.11.1.2	Test Requirements	2559

A.6.6.12	RSTD measurements	2559

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2559

A.6.6.12.1.1	Test Purpose and Environment	2559

A.6.6.12.1.2	Test Requirements	2566

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2567

A.6.6.12.2.1	Test Purpose and Environment	2567

A.6.6.12.2.2	Test Requirements	2573

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2573

A.6.6.12.3.1	Test Purpose and Environment	2573

A.6.6.12.3.2	Test Requirements	2579

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2579

A.6.6.12.4.1	Test Purpose and Environment	2579

A.6.6.12.4.2	Test Requirements	2583

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2584

A.6.6.12.5.1	Test Purpose and Environment	2584

A.6.6.12.5.2	Test Requirements	2588

A.6.6.13 PRS-RSRP measurements	2588

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2588

A.6.6.13.1.1	Test purpose and Environment	2588

A.6.6.13.1.2	Test Requirements	2592

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2592

A.6.6.13.2.1	Test purpose and Environment	2592

A.6.6.13.2.2	Test Requirements	2596

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2596

A.6.6.13.3.1	Test purpose and Environment	2596

A.6.6.13.3.2	Test Requirements	2600

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2600

A.6.6.13.4.1	Test purpose and Environment	2600

A.6.6.14	UE Rx-Tx time difference measurements	2604

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2604

A.6.6.14.1.1	Test purpose and environment	2604

A.6.6.14.1.2	Test requirements	2608

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2608

A.6.6.14.2.1	Test purpose and environment	2608

A.6.6.14.2.2	Test requirements	2612

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2612

A.6.6.14.3.1	Test purpose and environment	2612

A.6.6.14.3.2	Test requirements	2616

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2616

A.6.6.14.4.1	Test purpose and environment	2616

A.6.6.14.4.2	Test requirements	2620

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2620

A.6.6.14.4.1	Test purpose and environment	2620

A.6.6.14.4.2	Test requirements	2624

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	2624

A.6.6.15.1	Test Purpose and Environment	2624

A.6.6.15.2	Test Requirements	2631

A.6.6.16	PRS-RSRPP measurements	2632

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2632

A.6.6.16.1.1	Test purpose and Environment	2632

A.6.6.16.1.2	Test Requirements	2634

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2634

A.6.6.16.2.1	Test purpose and Environment	2634

A.6.6.16.2.2	Test Requirements	2637

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2637

A.6.6.16.3.1	Test purpose and Environment	2637

A.6.6.16.3.2	Test Requirements	2641

A.6.6.17	SA event triggered reporting tests with Pre-MG	2641

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2641

A.6.6.17.1.1	Test purpose and Environment	2641

A.6.6.17.1.2	Test parameters	2641

A.6.6.17.1.3	Test Requirements	2645

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2645

A.6.6.17.2.1	Test purpose and Environment	2645

A.6.6.17.2.2	Test parameters	2645

A.6.6.17.2.3	Test Requirements	2650

A.6.6.17.3	Intra-frequency measurement with SA event triggered reporting tests under non-DRX: with autonomous activation/deactivation of Pre-MG in FR1 triggered by MO addition/release	2650

A.6.6.17.3.1	Test purpose and Environment	2650

A.6.6.17.3.2	Test parameters	2650

A.6.6.17.3.3	Test Requirements	2654

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2655

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2655

A.6.6.18.1.1	Test Purpose and Environment	2655

A.6.6.18.1.2	Test Requirements	2658

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2658

A.6.6.18.2.1	Test Purpose and Environment	2658

A.6.6.18.2.2	Test Requirements	2662

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2662

A.6.6.18.3.1	Test Purpose and Environment	2662

A.6.6.18.3.2	Test Requirements	2669

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2669

A.6.6.18.4.1	Test Purpose and Environment	2669

A.6.6.18.4.2	Test Requirements	2673

A.6.6.19	SA event triggered reporting tests with NCSG	2673

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2673

A.6.6.19.1.1	Test purpose and Environment	2673

A.6.6.19.1.2	Test parameters	2673

A.6.6.19.1.3	Test Requirements	2678

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2678

A.6.6.19.2.1	Test Purpose and Environment	2678

A.6.6.19.2.2	Test parameters	2678

A.6.6.19.2.3	Test Requirements	2682

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2682

A.6.6.19.3.1	Test Purpose and Environment	2682

A.6.6.19.3.2	Test parameters	2683

A.6.6.19.3.3	Test Requirements	2688

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2688

A.6.6.19.4.1	Test purpose and Environment	2688

A.6.6.19.4.2	Test parameters	2688

A.6.6.19.4.3	Test Requirements	2691

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2691

A.6.6.20.1	Test purpose and environment	2691

A.6.6.20.2	Test requirements	2695

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2695

A.6.6.21.1	Test purpose and environment	2695

A.6.6.21.2	Test requirements	2697

A.6.7	Measurement Performance requirements	2698

A.6.7.1	SS-RSRP	2698

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2698

A.6.7.1.1.1	Test Purpose and Environment	2698

A.6.7.1.1.2	Test parameters	2698

A.6.7.1.1.3	Test Requirements	2703

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2703

A.6.7.1.2.1	Test Purpose and Environment	2703

A.6.7.1.2.2	Test parameters	2703

A.6.7.1.2.3	Test Requirements	2707

A.6.7.1.3	Void	2707

A.6.7.2	SS-RSRQ	2707

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2707

A.6.7.2.1.1	Test Purpose and Environment	2707

A.6.7.2.1.2	Test Parameters	2708

A.6.7.2.1.3	Test Requirements	2713

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2713

A.6.7.2.2.1	Test Purpose and Environment	2713

A.6.7.2.2.2	Test Parameters	2713

A.6.7.2.2.3	Test Requirements	2718

A.6.7.3	SS-SINR	2718

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2718

A.6.7.3.1.1	Test Purpose and Environment	2718

A.6.7.3.1.2	Test Parameters	2719

A.6.7.3.1.3	Test Requirements	2723

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2723

A.6.7.3.2.1	Test Purpose and Environment	2723

A.6.7.3.2.2	Test Parameters	2724

A.6.7.3.2.3	Test Requirements	2728

A.6.7.4	L1-RSRP measurement for beam reporting	2729

A.6.7.4.1	SSB based L1-RSRP measurement	2729

A.6.7.4.1.1	Test Purpose and Environment	2729

A.6.7.4.1.2	Test parameters	2729

A.6.7.4.1.3	Test Requirements	2733

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2733

A.6.7.4.2.1	Test Purpose and Environment	2733

A.6.7.4.2.2	Test parameters	2734

A.6.7.4.2.3	Test Requirements	2738

A.6.7.5	E-UTRAN RSRP	2738

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2738

A.6.7.5.1.1	Test Purpose and Environment	2738

A.6.7.5.1.2	Test parameters	2739

A.6.7.5.1.3	Test Requirements	2745

A.6.7.6	E-UTRAN RSRQ	2746

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2746

A.6.7.6.1.1	Test Purpose and Environment	2746

A.6.7.6.1.2	Test parameters	2746

A.6.7.6.1.3	Test Requirements	2751

A.6.7.7	E-UTRAN RS-SINR	2752

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2752

A.6.7.7.1.1	Test Purpose and Environment	2752

A.6.7.7.1.2	Test parameters	2752

A.6.7.7.1.3	Test Requirements	2758

A.6.7.8	CLI measurements	2759

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2759

A.6.7.8.1.1	Test Purpose and Environment	2759

A.6.7.8.1.2	Test parameters	2759

A.6.7.8.1.3	Test Requirements	2765

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2765

A.6.7.8.2.1	Test Purpose and Environment	2765

A.6.7.8.2.2	Test parameters	2766

A.6.7.8.2.3	Test Requirements	2769

A.6.7.9	L1-SINR measurement for beam reporting	2770

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2774

A.6.7.9.2.1	Test Purpose and Environment	2774

A.6.7.9.2.2	Test parameters	2775

A.6.7.9.2.3	Test Requirements	2780

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2780

A.6.7.9.3.1	Test Purpose and Environment	2780

A.6.7.9.3.2	Test parameters	2780

A.6.7.9.3.3	Test Requirements	2785

A.6.7.10	CSI-RSRP	2785

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2785

A.6.7.9.10.1	Test Purpose and Environment	2785

A.6.7.9.10.2	Test parameters	2786

A.6.7.10.1.3	Test Requirements	2791

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2791

A.6.7.9.10.1	Test Purpose and Environment	2791

A.6.7.10.2.2	Test parameters	2791

A.6.7.10.2.3	Test Requirements	2796

A.6.7.11	CSI-RSRQ	2796

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2796

A.6.7.11.1.1	Test Purpose and Environment	2796

A.6.7.11.1.2	Test Parameters	2796

A.6.7.11.1.3	Test Requirements	2801

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2801

A.6.7.11.2.1	Test Purpose and Environment	2801

A.6.7.11.2.2	Test Parameters	2801

A.6.7.11.2.3	Test Requirements	2807

A.6.7.12	CSI-SINR	2808

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2808

A.6.7.12.1.1	Test Purpose and Environment	2808

A.6.7.12.1.2	Test Parameters	2808

A.6.7.12.1.3	Test Requirements	2812

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2813

A.6.7.12.2.1	Test Purpose and Environment	2813

A.6.7.12.2.2	Test Parameters	2813

A.6.7.12.2.3	Test Requirements	2819

A.6.7.13	RSTD measurements	2819

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	2819

A.6.7.13.1.1	Test purpose and Environment	2819

A.6.7.13.1.2	Test Requirements	2822

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	2822

A.6.7.13.2.1	Test purpose and Environment	2822

A.6.7.13.2.2	Test Requirements	2826

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2826

A.6.7.13.3.1	Test purpose and Environment	2826

A.6.7.13.3.2	Test Requirements	2828

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	2828

A.6.7.14	PRS-RSRP measurements	2832

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	2832

A.6.7.14.1.1	Test Purpose and Environment	2832

A.6.7.14.1.2	Test parameters	2833

A.6.7.14.1.3	Test Requirements	2837

A.6.7.14.2	SA: measurement accuracy with PRS in FR1	2837

A.6.7.14.2.1	Test Purpose and Environment	2837

A.6.7.14.2.2	Test parameters	2837

A.6.7.14.2.3	Test Requirements	2842

A.6.7.14.3	SA: measurement accuracy with reduced PRS samples in FR1	2842

A.6.7.14.3.1	Test Purpose and Environment	2842

A.6.7.14.3.2	Test parameters	2842

A.6.7.15	UE Rx-Tx time difference measurements	2845

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	2845

A.6.7.15.1.1	Test purpose and environment	2845

A.6.7.15.1.2	Test parameters	2846

A.6.7.15.1.3	Test requirements	2849

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	2850

A.6.7.15.2.1	Test purpose and environment	2850

A.6.7.15.2.2	Test parameters	2850

A.6.7.15.2.3	Test requirements	2853

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	2853

A.6.7.15.3.1	Test purpose and environment	2853

A.6.7.15.3.2	Test parameters	2854

A.6.7.15.3.3	Test requirements	2857

A.6.7.16	PRS-RSRPP measurements	2857

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	2857

A.6.7.16.1.1	Test Purpose and Environment	2857

A.6.7.16.1.2	Test parameters	2858

A.6.7.16.1.3	Test Requirements	2863

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	2863

A.6.7.16.2.1	Test Purpose and Environment	2863

A.6.7.16.2.2	Test parameters	2863

A.6.8	Measurement procedure in RRC_INACTIVE	2867

A.6.8.1	RSTD measurements	2867

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	2867

A.6.8.1.1.1	Test Purpose and Environment	2867

A.6.8.1.1.2	Test Requirements	2870

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	2870

A.6.8.1.2.1	Test Purpose and Environment	2870

A.6.8.1.2.2	Test Requirements	2875

A.6.8.2	PRS-RSRP measurements	2875

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	2875

A.6.8.2.1.1	Test purpose and Environment	2875

A.6.8.2.1.2	Test Requirements	2877

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	2878

A.6.8.2.2.1	Test purpose and Environment	2878

A.6.8.2.2.2	Test Requirements	2881

A.6.8.3	UE Rx-Tx time difference measurements	2881

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2881

A.6.8.3.1.1	Test purpose and environment	2881

A.6.8.3.1.2	Test requirements	2885

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	2885

A.6.8.3.2.1	Test purpose and environment	2885

A.6.8.3.2.2	Test requirements	2888

A.6.8.4	PRS-RSRPP measurements	2888

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	2888

A.6.8.4.1.1	Test purpose and Environment	2888

A.6.8.4.1.2	Test Requirements	2891

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	2891

A.6.8.4.2.1	Test purpose and Environment	2891

A.6.8.4.2.2	Test Requirements	2893

A.6.9	Measurement performance requirements in RRC_INACTIVE	2893

A.6.9.1	RSTD measurements	2893

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	2893

A.6.9.1.1.1	Test purpose and Environment	2893

A.6.9.1.1.2	Test Requirements	2895

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	2895

A.6.9.1.2.1	Test purpose and Environment	2895

A.6.9.1.2.2	Test Requirements	2897

A.6.9.2	PRS-RSRP measurements	2897

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	2897

A.6.9.2.1.1	Test Purpose and Environment	2897

A.6.9.2.1.2	Test parameters	2898

A.6.9.2.1.3	Test Requirements	2903

A.6.9.2.2	SA: measurement accuracy with PRS in FR1	2903

A.6.9.2.2.1	Test Purpose and Environment	2903

A.6.9.2.2.2	Test parameters	2903

A.6.9.2.2.3 Test Requirements	2906

A.6.9.3	UE Rx-Tx time difference measurements	2907

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	2907

A.6.9.3.1.1.1	Test purpose and environment	2907

A.6.9.3.1.1.2 Test parameters	2907

A.6.9.3.1.1.3	Test requirements	2910

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	2910

A.6.9.3.2.1	Test purpose and environment	2910

A.6.9.3.2.2	Test parameters	2910

A.6.9.3.2.3	Test requirements	2913

A.6.9.4	PRS-RSRPP measurements	2913

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	2913

A.6.9.4.1.1	Test Purpose and Environment	2913

A.6.9.4.1.2	Test parameters	2913

A.6.9.4.1.3	Test Requirements	2918

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	2918

A.6.9.4.2.1	Test Purpose and Environment	2918

A.6.9.4.2.2	Test parameters	2918

A.6.9.4.2.3	Test Requirements	2921

A.7	NR standalone tests with one or more NR cells in FR2	2949

A.7.1	SA: RRC_IDLE state mobility	2949

A.7.1.1	Cell re-selection to NR	2949

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	2949

A.7.1.1.1.1	Test Purpose and Environment	2949

A.7.1.1.1.2	Test Parameters	2949

A.7.1.1.1.3	Test Requirements	2952

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	2952

A.7.1.1.2.1	Test Purpose and Environment	2952

A.7.1.1.2.2	Test Parameters	2953

A.7.1.1.2.3	Test Requirements	2955

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2955

A.7.1.1.3.1	Test Purpose and Environment	2955

A.7.1.1.3.2	Test Parameters	2955

A.7.1.1.3.3	Test Requirements	2958

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2958

A.7.1.1.4.1	Test Purpose and Environment	2958

A.7.1.1.4.2	Test Parameters	2958

A.7.1.1.4.3	Test Requirements	2961

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2961

A.7.1.1.5.1	Test Purpose and Environment	2961

A.7.1.1.5.2	Test Parameters	2961

A.7.1.1.5.3	Test Requirements	2964

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2964

A.7.1.1.6.1	Test Purpose and Environment	2964

A.7.1.1.6.2	Test Parameters	2964

A.7.1.1.6.3	Test Requirements	2967

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	2967

A.7.1.1.7.1	Test Purpose and Environment	2967

A.7.1.1.7.2	Test Parameters	2967

A.7.1.1.7.3	Test Requirements	2971

A.7.2	SA: RRC_INACTIVE state mobility	2971

A.7.3	RRC_CONNECTED state mobility	2971

A.7.3.1	Handover	2971

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	2971

A.7.3.1.1.1	Test Purpose and Environment	2971

A.7.3.1.1.2	Test Parameters	2972

A.7.3.1.1.3	Test Requirements	2975

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	2975

A.7.3.1.2.1	Test Purpose and Environment	2975

A.7.3.1.2.2	Test Parameters	2975

A.7.3.1.2.3	Test Requirements	2978

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	2978

A.7.3.1.3.1	Test Purpose and Environment	2978

A.7.3.1.3.2	Test Parameters	2978

A.7.3.1.3.3	Test Requirements	2980

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	2981

A.7.3.1.4.1	Test Purpose and Environment	2981

A.7.3.1.4.2	Test Parameters	2981

A.7.3.1.4.3 Test Requirements	2988

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	2988

A.7.3.1.5.1	Test Purpose and Environment	2988

A.7.3.1.5.2	Test Parameters	2988

A.7.3.1.5.3 Test Requirements	2995

A.7.3.1.6	Handover with PSCell from SA to EN-DC; unknown FR2 target cell	2995

A.7.3.1.6.1	Test Purpose and Environment	2995

A.7.3.1.6.2	Test Parameters	2995

A.7.3.1.6.3	Test Requirements	3000

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3000

A.7.3.1.7.1	Test purpose and environment	3000

A.7.3.1.7.2	Test Requirements	3006

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3006

A.7.3.1.8.1	Test Purpose and Environment	3006

A.7.3.1.8.2	Test Requirements	3012

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3012

A.7.3.1.9.1	Test Purpose and Environment	3012

A.7.3.1.9.2	Test Parameters	3012

A.7.3.1.9.3	Test Requirements	3015

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3015

A.7.3.1.10.1	Test Purpose and Environment	3015

A.7.3.1.10.2	Test Parameters	3015

A.7.3.1.10.3	Test Requirements	3019

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3019

A.7.3.1.11.1	Test Purpose and Environment	3019

A.7.3.1.11.2	Test Parameters	3019

A.7.3.1.11.3	Test Requirements	3023

A.7.3.2	RRC Connection Mobility Control	3023

A.7.3.2.1	SA: RRC Re-establishment	3023

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3023

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3026

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3029

A.7.3.2.1.3.1	Test Purpose and Environment	3029

A.7.3.2.1.3.2	Test Requirements	3031

A.7.3.2.2	Random Access	3032

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3032

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3036

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3040

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3043

A.7.3.2.3	SA: RRC Connection Release with Redirection	3046

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3046

A.7.3.3 Conditional Handover	3050

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3050

A.7.3.3.1.1	Test Purpose and Environment	3050

A.7.3.3.1.2	Test Parameters	3050

A.7.3.3.1.2.3	Test Requirements	3053

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3053

A.7.3.3.2.1	Test Purpose and Environment	3053

A.7.3.3.2.2	Test Parameters	3053

A.7.3.3.2.3 Test Requirements	3055

A.7.4	Timing	3055

A.7.4.1	UE transmit timing	3055

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3055

A.7.4.1.1.1	Test Purpose and environment	3055

A.7.4.1.1.2	Test requirements	3058

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3059

A.7.4.1.2.1	Test Purpose and environment	3059

A.7.4.1.2.2	Test requirements	3063

A.7.4.2	UE timer accuracy	3064

A.7.4.3	Timing advance	3064

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3064

A.7.4.3.1.1	Test Purpose and Environment	3064

A.7.4.3.1.2	Test Parameters	3064

A.7.4.3.1.3 Test Requirements	3067

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3068

A.7.4.3.2.1	Test Purpose and Environment	3068

A.7.4.3.2.2	Test Parameters	3068

A.7.4.3.2.3	Test Requirements	3072

A.7.5	Signaling characteristics	3072

A.7.5.1	Radio link Monitoring	3072

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3072

A.7.5.1.1.1	Test Purpose and Environment	3072

A.7.5.1.1.2	Test Requirements	3075

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3076

A.7.5.1.2.1	Test Purpose and Environment	3076

A.7.5.1.2.2	Test Requirements	3080

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3081

A.7.5.1.3.1	Test Purpose and Environment	3081

A.7.5.1.3.2	Test Requirements	3085

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3085

A.7.5.1.4.1	Test Purpose and Environment	3085

A.7.5.1.4.2	Test Requirements	3090

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3090

A.7.5.1.5.1	Test Purpose and Environment	3090

A.7.5.1.5.2	Test Requirements	3095

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3095

A.7.5.1.6.1	Test Purpose and Environment	3095

A.7.5.1.6.2	Test Requirements	3099

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3099

A.7.5.1.7.1	Test Purpose and Environment	3099

A.7.5.1.7.2	Test Requirements	3103

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3103

A.7.5.1.8.1	Test Purpose and Environment	3103

A.7.5.1.8.2	Test Requirements	3108

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3108

A.7.5.1.9.1	Test Purpose and Environment	3108

A.7.5.1.9.2	Test Requirements	3111

A.7.5.2	Interruption	3111

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3111

A.7.5.2.1.1	Test Purpose and Environment	3111

A.7.5.2.1.2	Test Requirements	3114

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3115

A.7.5.2.2.1	Test Purpose and Environment	3115

A.7.5.2.2.2	Test Parameters	3115

A.7.5.2.2.3	Test Requirements	3117

A.7.5.3	SCell Activation and Deactivation Delay	3117

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3117

A.7.5.3.1.1	Test Purpose and Environment	3117

A.7.5.3.1.2	Test Requirements	3119

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3119

A.7.5.3.2.1	Test Purpose and Environment	3119

A.7.5.3.2.2	Test Requirements	3123

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3124

A.7.5.3.3.1	Test Purpose and Environment	3124

A.7.5.3.3.2	Test Requirements	3127

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3128

A.7.5.3.4.1	Test Purpose and Environment	3128

A.7.5.3.4.2	Test Requirements	3131

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3132

A.7.5.3.5.1	Test Purpose and Environment	3132

A.7.5.3.5.2	Test Requirements	3134

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3135

A.7.5.3.6.1	Test Purpose and Environment	3135

A.7.5.3.6.2	Test Requirements	3139

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3140

A.7.5.3.7.1	Test Purpose and Environment	3140

A.7.5.3.7.2	Test Requirements	3144

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3145

A.7.5.3.8.1	Test Purpose and Environment	3145

A.7.5.3.8.2	Test Requirements	3149

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3150

A.7.5.3.9.1	Test Purpose and Environment	3150

A.7.5.3.9.2	Test Requirements	3153

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3154

A.7.5.3.10.1	Test Purpose and Environment	3154

A.7.5.3.10.2	Test Requirements	3158

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3159

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3159

A.7.5.3.11.1.1	Test Purpose and Environment	3159

A.7.5.3.11.1.2	Test Requirements	3163

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3164

A.7.5.3.11.2.1	Test Purpose and Environment	3164

A.7.5.3.11.2.2	Test Requirements	3168

A.7.5.3.12	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3169

A.7.5.3.12.1	Test Purpose and Environment	3169

A.7.5.3.12.2	Test Requirements	3173

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3174

A.7.5.3.13.1	Test Purpose and Environment	3174

A.7.5.3.13.2	Test Requirements	3176

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3176

A.7.5.3.14.1	Test Purpose and Environment	3176

A.7.5.3.14.2	Test Requirements	3179

A.7.5.15	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3180

A.7.5.15.3.1	Test Purpose and Environment	3180

A.7.5.3.2.2	Test Requirements	3182

A.7.5.4	Void	3183

A.7.5.5	Beam Failure Detection and Link recovery procedures	3183

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3183

A.7.5.5.1.1	Test Purpose and Environment	3183

A.7.5.5.1.2	Test Requirements	3187

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3187

A.7.5.5.2.1	Test Purpose and Environment	3187

A.7.5.5.2.2	Test Requirements	3191

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3191

A.7.5.5.3.1	Test Purpose and Environment	3191

A.7.5.5.3.2	Test Requirements	3195

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3195

A.7.5.5.4.1	Test Purpose and Environment	3195

A.7.5.5.4.2	Test Requirements	3199

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3199

A.7.5.5.5.1	Test Purpose and Environment	3199

A.7.5.5.5.2	Test Requirements	3203

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3203

A.7.5.5.6.1	Test Purpose and Environment	3203

A.7.5.5.6.2	Test Requirements	3207

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3207

A.7.5.5.7.1	Test Purpose and Environment	3207

A.7.5.5.7.2	Test Requirements	3211

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3211

A.7.5.5.8.1	Test Purpose and Environment	3211

A.7.5.5.8.2	Test Requirements	3216

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3216

A.7.5.5.9.1	Test Purpose and Environment	3216

A.7.5.5.9.2	Test Requirements	3221

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3221

A.7.5.5.10.1	Test Purpose and Environment	3221

A.7.5.5.10.2	Test Requirements	3225

A.7.5.6	Active BWP switch	3225

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3225

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3225

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3230

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3235

A.7.5.6.1.3.1	Test Purpose and Environment	3235

A.7.5.6.1.3.2	Test Requirements	3238

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3238

A.7.5.6.1.4.1	Test Purpose and Environment	3238

A.7.5.6.1.4.2	Test Requirements	3243

A.7.5.6.2	RRC-based Active BWP Switch	3244

A.7.5.6.2.1.1	Test Purpose and Environment	3244

A.7.5.6.2.1.2	Test Requirements	3247

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3248

A.7.5.6.2.2.1	Test Purpose and Environment	3248

A.7.5.6.2.2.2	Test Requirements	3252

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3253

A.7.5.6.3.1.1	Test Purpose and Environment	3253

A.7.5.6.3.1.2	Test Requirements	3256

A.7.5.6.4	SCell dormancy switch	3256

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3256

A.7.5.6.4.1.1	Test Purpose and Environment	3256

A.7.5.6.4.1.2	Test Requirements	3261

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3261

A.7.5.6.4.2.1	 Test Purpose and Environment	3261

A.7.5.6.4.2.2	 Test Requirements	3266

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3266

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3266

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3269

A.7.5.6.5.2.1	Test Purpose and Environment	3269

A.7.5.6.5.2.2	Test Requirements	3273

A.7.5.7	PSCell addition and release delay	3274

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3274

A.7.5.7.1.1	Test Purpose and Environment	3274

A.7.5.7.1.2	Test Requirements	3277

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	3277

A.7.5.7.2.1	Test Purpose and Environment	3277

A.7.5.7.2.2	Test Requirements	3280

A.7.5.8	Active TCI state switch delay	3280

A.7.5.8.1	MAC-CE based active TCI state switch	3280

A.7.5.8.2	RRC based active TCI state switch	3284

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3288

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3288

A.7.5.8.3.1.1	Test Purpose and Environment	3288

A.7.5.8.3.1.2	Test Requirements	3292

A.7.5.9	Uplink spatial relation switch delay	3293

A.7.5.9.1.1.1	Test Purpose and Environment	3293

A.7.5.9.1.1.2	Test Requirements	3296

A.7.5.9.2	RRC based spatial relation switch	3296

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3296

A.7.5.9.2.1.1	Test Purpose and Environment	3296

A.7.5.9.2.1.2	Test Requirements	3299

A.7.5.10	UE specific CBW change	3299

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3299

A.7.5.10.1.1	Test Purpose and Environment	3299

A.7.5.10.1.2	Test Requirements	3302

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3302

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3302

A.7.5.11.1.1	Test Purpose and Environment	3302

A.7.5.11.1.2	Test Requirements	3306

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3306

A.7.5.12.1	Addition and Release Delay of PSCell	3306

A.7.5.12.1.1	Test purpose and environment	3306

A.7.5.12.1.2	Test Parameters	3306

A.7.5.12.1.3	Test Requirements	3309

A.7.5.13	 Unified TCI state switching delay	3309

A.7.5.13.1	MAC-CE based active joint TCI state switching	3309

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3309

A.7.5.13.1.1.1	Test Purpose and Environment	3309

A.7.5.13.1.1.2	Test parameters	3310

A.7.5.13.1.1.3	Test Requirements	3312

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3312

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3312

A.7.5.13.2.1.1	Test Purpose and Environment	3312

A.7.5.13.2.1.2	Test parameters	3313

A.7.5.13.2.1.3	Test Requirements	3315

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3315

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3315

A.7.5.13.3.1.1	Test Purpose and Environment	3315

A.7.5.13.3.1.2	Test Parameters	3316

A.7.5.13.3.1.3	Test Requirements	3319

A.7.6	Measurement procedure	3319

A.7.6.1	Intra-frequency Measurements	3319

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3319

A.7.6.1.1.1	Test purpose and Environment	3319

A.7.6.1.1.2	Test Requirements	3323

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3323

A.7.6.1.2.1	Test purpose and Environment	3323

A.7.6.1.2.2	Test Requirements	3326

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3327

A.7.6.1.3.1	Test purpose and Environment	3327

A.7.6.1.3.2	Test Requirements	3330

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3330

A.7.6.1.4.1	Test purpose and Environment	3330

A.7.6.1.4.2	Test Requirements	3333

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasCA-Scell-r17	3334

A.7.6.1.5.1	Test purpose and Environment	3334

A.7.6.1.5.2	Test Requirements	3337

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX	3337

A.7.6.1.6.1	Test purpose and Environment	3337

A.7.6.1.6.2	Test Requirements	3340

A.7.6.1.7	SA event triggered reporting test without gap under DRX	3341

A.7.6.1.7.1	Test purpose and Environment	3341

A.7.6.1.7.2	Test Requirements	3343

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX	3344

A.7.6.1.8.1	Test purpose and Environment	3344

A.7.6.1.8.2	Test Requirements	3347

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX	3348

A.7.6.1.9.1	Test purpose and Environment	3348

A.7.6.1.9.2	Test Requirements	3350

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX	3351

A.7.6.1.10.1	Test purpose and Environment	3351

A.7.6.1.10.2	Test Requirements	3354

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX	3355

A.7.6.1.11.1	Test purpose and Environment	3355

A.7.6.1.11.2	Test Requirements	3357

A.7.6.2	Inter-frequency Measurements	3358

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3358

A.7.6.2.1.1	Test Purpose and Environment	3358

A.7.6.2.1.2	Test Requirements	3361

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	3361

A.7.6.2.2.1	Test Purpose and Environment	3361

A.7.6.2.2.2	Test Requirements	3365

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3365

A.7.6.2.3.1	Test Purpose and Environment	3365

A.7.6.2.3.2	Test Requirements	3369

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	3369

A.7.6.2.4.1	Test Purpose and Environment	3369

A.7.6.2.4.2	Test Requirements	3373

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3373

A.7.6.2.5.1	Test Purpose and Environment	3373

A.7.6.2.5.2	Test Requirements	3377

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	3378

A.7.6.2.6.1	Test Purpose and Environment	3378

A.7.6.2.6.2	Test Requirements	3382

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3383

A.7.6.2.7.1	Test Purpose and Environment	3383

A.7.6.2.7.2	Test Requirements	3387

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3388

A.7.6.2.8.1	Test Purpose and Environment	3388

A.7.6.2.8.2	Test Requirements	3392

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3393

A.7.6.2.9.1	Test Purpose and Environment	3393

A.7.6.2.9.2	Test Requirements	3397

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3397

A.7.6.2.10.1	Test Purpose and Environment	3397

A.7.6.2.10.2	Test Requirements	3399

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3399

A.7.6.2.11.1	Test Purpose and Environment	3399

A.7.6.2.11.2	Test Requirements	3402

A.7.6.2.12	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3402

A.7.6.2.12.1	Test Purpose and Environment	3402

A.7.6.2.12.2	Test Requirements	3406

A.7.6.2.13	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2-2)	3407

A.7.6.2.13.1	Test Purpose and Environment	3407

A.7.6.2.13.2	Test Requirements	3411

A.7.6.2.14	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3412

A.7.6.2.14.1	Test Purpose and Environment	3412

A.7.6.2.14.2	Test Requirements	3416

A.7.6.2.15	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2-2)	3417

A.7.6.2.15.1	Test Purpose and Environment	3417

A.7.6.2.15.2	Test Requirements	3421

A.7.6.2.16	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3422

A.7.6.2.16.1	Test Purpose and Environment	3422

A.7.6.2.16.2	Test Requirements	3428

A.7.6.2.17	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	3428

A.7.6.2.17.1	Test Purpose and Environment	3428

A.7.6.2.17.2	Test Requirements	3435

A.7.6.2.18	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3436

A.7.6.2.18.1	Test Purpose and Environment	3436

A.7.6.2.18.2	Test Requirements	3441

A.7.6.2.19	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3442

A.7.6.2.19.1	Test Purpose and Environment	3442

A.7.6.2.19.2	Test Requirements	3450

A.7.6.3	L1-RSRP measurement for beam reporting	3451

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3451

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3451

A.7.6.3.1.1	Test Purpose and Environment	3451

A.7.6.3.1.2	Test parameters	3451

A.7.6.3.1.3	Test Requirements	3453

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3453

A.7.6.3.2.1	Test Purpose and Environment	3453

A.7.6.3.2.2	Test parameters	3454

A.7.6.3.2.3	Test Requirements	3456

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3456

A.7.6.3.3.1	Test Purpose and Environment	3456

A.7.6.3.3.2	Test parameters	3457

A.7.6.3.3.3	Test Requirements	3459

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3460

A.7.6.3.4.1	Test Purpose and Environment	3460

A.7.6.3.4.2	Test parameters	3460

A.7.6.3.3.3	Test Requirements	3462

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3463

A.7.6.3.5.1	Test Purpose and Environment	3463

A.7.6.3.5.2	Test parameters	3463

A.7.6.3.5.3	Test Requirements	3465

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3465

A.7.6.3.6.1	Test Purpose and Environment	3465

A.7.6.3.6.2	Test parameters	3466

A.7.6.3.6.3	Test Requirements	3469

A.7.6.4	CLI measurements	3469

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3469

A.7.6.4.1.1	Test Purpose and Environment	3469

A.7.6.4.1.2	Test Parameters	3470

A.7.6.4.1.3	Test Requirements	3472

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3472

A.7.6.4.2.1	Test Purpose and Environment	3472

A.7.6.4.2.2	Test Parameters	3473

A.7.6.4.2.3	Test Requirements	3474

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3475

A.7.6.5.1.1	Test Purpose and Environment	3475

A.7.6.5.1.2	Test Requirements	3478

A.7.6.6	L1-SINR measurement for beam reporting	3478

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3480

A.7.6.6.2.1	Test Purpose and Environment	3480

A.7.6.6.2.2	Test parameters	3481

A.7.6.6.2.3	Test Requirements	3483

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3483

A.7.6.6.3.1	Test Purpose and Environment	3483

A.7.6.6.3.2	Test parameters	3484

A.7.6.6.3.3	Test Requirements	3486

A.7.6.7	CSI-RS based intra-frequency Measurements	3486

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3486

A.7.6.7.1.1	Test purpose and Environment	3486

A.7.6.7.1.2	Test Requirements	3489

A.7.6.8	CSI-RS based inter-frequency Measurements	3489

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3489

A.7.6.8.1.1	Test Purpose and Environment	3489

A.7.6.8.1.2	Test Requirements	3492

A.7.6.9	RSTD measurements	3492

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3492

A.7.6.9.1.1	Test Purpose and Environment	3492

A.7.6.9.1.2	Test Requirements	3500

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3500

A.7.6.9.2.1	Test Purpose and Environment	3500

A.7.6.9.2.2	Test Requirements	3507

A.7.6.10 PRS-RSRP measurements	3507

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3507

A.7.6.10.1.1	Test Purpose and Environment	3507

A.7.6.10.1.2	Test Requirements	3511

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3511

A.7.6.10.2.1	Test Purpose and Environment	3511

A.7.6.10.2.2	Test Requirements	3515

A.7.6.11	UE Rx-Tx time difference measurements	3515

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3515

A.7.6.11.1.1	Test purpose and environment	3515

A.7.6.11.1.2	Test requirements	3519

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3519

A.7.6.11.2.1	Test purpose and environment	3519

A.7.6.11.2.2	Test requirements	3523

A.7.6.12	PRS-RSRPP measurements	3523

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3523

A.7.6.12.1.1	Test Purpose and Environment	3523

A.7.6.12.1.2	Test Requirements	3526

A.7.6.12.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3526

A.7.6.12.2.1	Test Purpose and Environment	3526

A.7.6.12.2.2	Test Requirements	3529

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3529

A.7.6.12.3.1	Test Purpose and Environment	3529

A.7.6.12.3.2	Test Requirements	3532

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3532

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3532

A.7.6.13.1.1	Test purpose and environment13	3532

A.7.6.13.1.2	Test requirements	3534

A.7.6.14	SA event triggered reporting tests with Pre-MG	3535

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3535

A.7.6.14.1.1	Test purpose and Environment	3535

A.7.6.14.1.2	Test parameters	3535

A.7.6.14.1.3	Test Requirements	3537

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3538

A.7.6.14.2.1	Test purpose and Environment	3538

A.7.6.14.2.2	Test parameters	3538

A.7.6.14.2.3	Test Requirements	3540

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3541

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3541

A.7.6.15.1.1	Test Purpose and Environment	3541

A.7.6.15.1.2	Test Requirements	3544

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3544

A.7.6.15.2.1	Test Purpose and Environment	3544

A.7.6.15.2.2	Test Requirements	3547

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overalpping scenario for SSB-based measurements and PRS-based measurement	3547

A.7.6.15.3.1	Test Purpose and Environment	3547

A.7.6.15.3.2	Test Requirements	3551

A.7.6.16	SA event triggered reporting tests with NCSG	3551

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3551

A.7.6.16.1.1	Test purpose and Environment	3551

A.7.6.16.1.2	Test Requirements	3555

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3555

A.7.6.16.2.1	Test Purpose and Environment	3555

A.7.6.16.2.2	Test Requirements	3558

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3558

A.7.6.16.3.1	Test Purpose and Environment	3558

A.7.6.16.3.2	Test Requirements	3561

A.7.7	Measurement Performance requirements	3561

A.7.7.1	SS-RSRP	3562

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3562

A.7.7.1.1.1	Test Purpose and Environment	3562

A.7.7.1.1.2	Test parameters	3562

A.7.7.1.1.3	Test Requirements	3566

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3566

A.7.7.1.2.1	Test Purpose and Environment	3566

A.7.7.1.2.2	Test parameters	3567

A.7.7.1.2.3	Test Requirements	3571

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3572

A.7.7.1.3.1	Test Purpose and Environment	3572

A.7.7.1.3.2	Test parameters	3572

A.7.7.1.3.3	Test Requirements	3576

A.7.7.2	SS-RSRQ	3576

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3576

A.7.7.2.1.1	Test Purpose and Environment	3576

A.7.7.2.1.2	Test Parameters	3576

A.7.7.2.1.3	Test Requirements	3578

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3578

A.7.7.2.2.1	Test Purpose and Environment	3578

A.7.7.2.2.2	Test Parameters	3578

A.7.7.2.2.3	Test Requirements	3580

A.7.7.3	SS-SINR	3580

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3580

A.7.7.3.1.1	Test Purpose and Environment	3580

A.7.7.3.1.2	Test Parameters	3580

A.7.7.3.1.3	Test Requirements	3582

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3582

A.7.7.3.2.1	Test Purpose and Environment	3582

A.7.7.3.2.2	Test Parameters	3582

A.7.7.3.2.3	Test Requirements	3584

A.7.7.4	L1-RSRP measurement for beam reporting	3584

A.7.7.4.1	SSB based L1-RSRP measurement	3584

A.7.7.4.1.1	Test Purpose and Environment	3584

A.7.7.4.1.2	Test parameters	3585

A.7.7.4.1.3	Test Requirements	3587

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3588

A.7.7.4.2.1	Test Purpose and Environment	3588

A.7.7.4.2.2	Test parameters	3588

A.7.7.4.2.3	Test Requirements	3590

A.7.7.5	CLI measurements	3591

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3591

A.7.7.5.1.1	Test Purpose and Environment	3591

A.7.7.5.1.2	Test parameters	3591

A.7.7.5.1.3	Test Requirements	3594

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3595

A.7.7.5.2.1	Test Purpose and Environment	3595

A.7.7.5.2.2	Test parameters	3595

A.7.7.5.2.3	Test Requirements	3597

A.7.7.6	L1-SINR measurement for beam reporting	3598

A.7.7.6.1.1	Test Purpose and Environment	3598

A.7.7.6.1.2	Test parameters	3598

A.7.7.6.1.3	Test Requirements	3600

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3601

A.7.7.6.2.1	Test Purpose and Environment	3601

A.7.7.6.2.2	Test parameters	3601

A.7.7.6.2.3	Test Requirements	3603

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3603

A.7.7.6.3.1	Test Purpose and Environment	3603

A.7.7.6.3.2	Test parameters	3604

A.7.7.6.3.3	Test Requirements	3606

A.7.7.7	CSI-RSRP	3606

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3606

A.7.7.7.1.1	Test Purpose and Environment	3606

A.7.7.7.1.2	Test parameters	3607

A.7.7.7.1.3	Test Requirements	3610

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3611

A.7.7.7.2.1	Test Purpose and Environment	3611

A.7.7.7.2.2	Test parameters	3611

A.7.7.7.2.3	Test Requirements	3614

A.7.7.8	CSI-RSRQ	3614

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3614

A.7.7.8.1.1	Test Purpose and Environment	3614

A.7.7.8.1.2	Test Parameters	3615

A.7.7.8.1.3	Test Requirements	3617

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3617

A.7.7.8.2.1	Test Purpose and Environment	3617

A.7.7.8.2.2	Test Parameters	3617

A.7.7.8.2.3	Test Requirements	3619

A.7.7.9	CSI-SINR	3619

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3619

A.7.7.9.1.1	Test Purpose and Environment	3619

A.7.7.9.1.2	Test Parameters	3619

A.7.7.9.1.3	Test Requirements	3621

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3621

A.7.7.9.2.1	Test Purpose and Environment	3621

A.7.7.9.2.2	Test Parameters	3622

A.7.7.9.2.3	Test Requirements	3624

A.7.7.10	RSTD measurements	3624

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3624

A.7.7.10.1.1	Test purpose and Environment	3624

A.7.7.10.1.2	Test Requirements	3627

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3627

A.7.7.10.2.1	Test purpose and Environment	3627

A.7.7.10.2.2	Test Requirements	3631

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3632

A.7.7.10.3.1	Test purpose and Environment	3632

A.7.7.10.3.2	Test Requirements	3633

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3634

A.7.7.10.4.1	Test purpose and Environment	3634

A.7.7.10.4.2	Test Requirements	3636

A.7.7.11	PRS-RSRP measurements	3636

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3636

A.7.7.11.1.1	Test Purpose and Environment	3636

A.7.7.11.1.2	Test parameters	3636

A.7.7.11.1.3	Test Requirements	3640

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3641

A.7.7.11.2.1	Test Purpose and Environment	3641

A.7.7.11.2.2	Test parameters	3641

A.7.7.11.2.3	Test Requirements	3643

A.7.7.12	UE Rx-Tx time difference measurements	3644

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3644

A.7.7.12.1.1	Test purpose and environment	3644

A.7.7.12.1.2	Test parameters	3644

A.7.7.12.1.3	Test requirements	3647

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3647

A.7.7.12.2.1	Test purpose and environment	3647

A.7.7.12.2.2	Test parameters	3648

A.7.7.12.2.3	Test requirements	3651

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3651

A.7.7.12.3.1	Test purpose and environment	3651

A.7.7.12.3.2	Test parameters	3651

A.7.7.12.3.3	Test requirements	3654

A.7.7.13	PRS-RSRPP measurements	3654

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3654

A.7.7.13.1.1	Test Purpose and Environment	3654

A.7.7.13.1.2	Test parameters	3654

A.7.7.13.1.3	Test Requirements	3657

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3658

A.7.7.13.2.1	Test Purpose and Environment	3658

A.7.7.13.2.2	Test parameters	3658

A.7.7.13.2.3	Test Requirements	3660

A.7.8	Measurement procedure in RRC_INACTIVE	3661

A.7.8.1	RSTD measurements	3661

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3661

A.7.8.1.1.1	Test Purpose and Environment	3661

A.7.8.1.1.2	Test Requirements	3664

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3664

A.7.8.1.2.1	Test Purpose and Environment	3664

A.7.8.1.2.2	Test Requirements	3670

A.7.8.2	PRS-RSRP measurements	3671

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3671

A.7.8.2.1.1	Test Purpose and Environment	3671

A.7.8.2.1.2	Test Requirements	3675

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3675

A.7.8.2.2.1	Test purpose and Environment	3675

A.7.8.2.2.2	Test Requirements	3679

A.7.8.3	UE Rx-Tx time difference measurements	3679

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3679

A.7.8.3.1.1	Test purpose and environment	3679

A.7.8.3.1.2	Test requirements	3683

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3683

A.7.8.3.2.1	Test purpose and environment	3683

A.7.8.3.2.2	Test requirements	3685

A.7.8.4	PRS-RSRPP measurements	3686

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3686

A.7.8.4.1.1	Test Purpose and Environment	3686

A.7.8.4.1.2	Test Requirements	3688

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3688

A.7.8.4.2.1	Test Purpose and Environment	3688

A.7.8.4.2.2	Test Requirements	3691

A.7.9	Measurement performance requirements in RRC_INACTIVE	3691

A.7.9.1	RSTD measurements	3691

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3691

A.7.9.1.1.1	Test purpose and Environment	3691

A.7.9.1.1.2	Test Requirements	3693

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3693

A.7.9.1.2.1	Test purpose and Environment	3693

A.7.9.1.2.2	Test Requirements	3695

A.7.9.2	PRS-RSRP measurements	3695

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3695

A.7.9.2.1.1	Test Purpose and Environment	3695

A.7.9.2.1.2	Test parameters	3695

A.7.9.2.1.3	Test Requirements	3697

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3698

A.7.9.2.2.1	Test Purpose and Environment	3698

A.7.9.2.2.2	Test parameters	3698

A.7.9.2.2.3	Test Requirements	3700

A.7.9.3	UE Rx-Tx time difference measurements	3701

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3701

A.7.9.3.1.1	Test purpose and environment	3701

A.7.9.3.1.2	Test parameters	3701

A.7.9.3.1.3	Test requirements	3703

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3703

A.7.9.3.2.1	Test purpose and environment	3703

A.7.9.3.2.2	Test parameters	3703

A.7.9.3.2.3	Test requirements	3706

A.7.9.4	PRS-RSRPP measurements	3706

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3706

A.7.9.4.1.1	Test Purpose and Environment	3706

A.7.9.4.1.2	Test parameters	3706

A.7.9.4.1.3	Test Requirements	3708

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3709

A.7.9.4.2.1	Test Purpose and Environment	3709

A.7.9.4.2.2	Test parameters	3709

A.7.9.4.2.3	Test Requirements	3711

A.8	E-UTRA standalone tests for NR RRM	3717

A.8.1	Void	3717

A.8.2	RRC_IDLE state mobility	3717

A.8.2.1	Inter-RAT NR Cell re-selection	3717

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3717

A.8.2.1.1.1	Test Purpose and Environment	3717

A.8.2.1.1.2	Test Requirements	3722

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3723

A.8.2.1.2.1	Test Purpose and Environment	3723

A.8.2.1.2.2	Test Requirements	3727

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3728

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	3728

A.8.2.2.1.1	Test Purpose and Environment	3728

A.8.2.2.1.2	Test Requirements	3731

A.8.2.2.2	E-UTRA – NR Early Measruement Reporting for NR in FR2	3732

A.8.2.2.2.1	Test Purpose and Environment	3732

A.8.2.2.2.2	Test Requirements	3734

A.8.3	RRC_CONNECTED state mobility	3735

A.8.3.1	Handover	3735

A.8.3.1.1	E-UTRAN - NR handover in FR1	3735

A.8.3.1.1.1	Test Purpose and Environment	3735

A.8.3.1.1.2	Test Requirements	3740

A.8.4	Measurement procedure	3740

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3740

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3740

A.8.4.1.1.1	Test Purpose and Environment	3740

A.8.4.1.1.2	Test Requirements	3742

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3743

A.8.4.1.2.1	Test Purpose and Environment	3743

A.8.4.1.2.2	Test Requirements	3744

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3744

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3744

A.8.4.2.1.1	Test Purpose and Environment	3744

A.8.4.2.1.2	Test Requirements	3749

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3749

A.8.4.2.2.1	Test Purpose and Environment	3749

A.8.4.2.2.2	Test Requirements	3753

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3753

A.8.4.2.3.1	Test Purpose and Environment	3753

A.8.4.2.3.2	Test Requirements	3758

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3758

A.8.4.2.4.1	Test Purpose and Environment	3758

A.8.4.2.4.2	Test Requirements	3762

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3762

A.8.4.2.5.1	Test Purpose and Environment	3762

A.8.4.2.5.2	Test Requirements	3764

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3765

A.8.4.2.6.1	Test Purpose and Environment	3765

A.8.4.2.6.2	Test Requirements	3767

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3768

A.8.4.2.7.1	Test Purpose and Environment	3768

A.8.4.2.7.2	Test Requirements	3770

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3771

A.8.4.2.8.1	Test Purpose and Environment	3771

A.8.4.2.8.2	Test Requirements	3773

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3774

A.8.4.2.9.1	Test Purpose and Environment	3774

A.8.4.2.9.2	Test Requirements	3778

A.8.5	Measurement performance	3778

A.8.5.1	SFTD accuracy	3778

A.8.5.1.1	SFTD accuracy	3778

A.8.5.1.1.1	Test Purpose	3778

A.8.5.1.1.2	Test Environment	3779

A.8.5.1.1.3	Test Requirements	3785

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	3785

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3785

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3790

A.8.5.2.1.2.1	Test Purpose and Environment	3790

A.8.5.2.1.2.2	Test Parameters	3790

A.8.5.2.1.2.3	Test Requirements	3792

A.8.5.2.2	SS-RSRQ	3792

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3792

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3797

A.8.5.2.2.2.1	Test Purpose and Environment	3797

A.8.5.2.2.2.2	Test Parameters	3797

A.8.5.2.2.2.3	Test Requirements	3799

A.8.5.2.3	SS-SINR	3800

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3800

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3803

A.8.5.2.3.2.1	Test Purpose and Environment	3803

A.8.5.2.3.2.2	Test Parameters	3803

A.8.5.2.3.2.3	Test Requirements	3805

A.9	V2X Tests	3806

A.9.1	V2X Tests in FR1	3806

A.9.1.1	Test for V2X UE Transmit Timing	3806

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	3806

A.9.1.1.1.1	Test Purpose and Environment	3806

A.9.1.1.1.2	Test requirements	3806

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	3806

A.9.1.1.2.1	Test Purpose and Environment	3806

A.9.1.1.2.2	Test requirements	3807

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	3807

A.9.1.1.3.1	Test Purpose and Environment	3807

A.9.1.1.3.2	Test requirements	3810

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	3810

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	3810

A.9.1.2.1.1	Test Purpose and Environment	3810

A.9.1.2.1.2	Test Requirements	3813

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	3813

A.9.1.2.2.1	Test Purpose and Environment	3813

A.9.1.2.2.2	Test Requirements	3814

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	3815

A.9.1.2.3.1	Test Purpose and Environment	3815

A.9.1.2.3.2	Test Requirements	3817

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	3817

A.9.1.3.1	 Test for GNSS configured as the highest priority	3817

A.9.1.3.1.1	Test Purpose and Environment	3817

A.9.1.3.1.2	Test Requirements	3819

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	3820

A.9.1.3.2.1	Test Purpose and Environment	3820

A.9.1.3.2.2	Test Requirements	3822

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	3823

A.9.1.3.3.1	Test Purpose and Environment	3823

A.9.1.3.3.2	Test Requirements	3825

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	3826

A.9.1.3.4.1	Test Purpose and Environment	3826

A.9.1.3.4.2	Test Requirements	3828

A.9.1.4	Test for L1 SL-RSRP Measurement	3829

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	3829

A.9.1.4.1.1	Test Purpose and Environment	3829

A.9.1.4.1.2	Test Requirements	3832

A.9.1.4.2	Test for V2X UE Resource Pre-emption	3832

A.9.1.4.2.1	Test Purpose and Environment	3832

A.9.1.4.2.2	Test Requirements	3836

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	3836

A.9.1.4.3.1	Test Purpose and Environment	3836

A.9.1.4.3.2	Test Requirements	3843

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	3843

A.9.1.4.4.1	Test Purpose and Environment	3843

A.9.1.4.4.2	Test Requirements	3846

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	3847

A.9.1.4.5.1	Test Purpose and Environment	3847

A.9.1.4.5.2	Test Requirements	3849

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	3849

A.9.1.4.6.1	Test Purpose and Environment	3849

A.9.1.4.6.2	Test Requirements	3852

A.9.1.5	Test for Congestion Control Measurement	3852

A.9.1.5.1	Test Purpose and Environment	3852

A.9.1.5.2	Test Requirements	3858

A.9.1.6	Test for Interruption	3858

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	3858

A.9.1.6.1.1	Test Purpose and Environment	3858

A.9.1.6.1.2	Test Requirements	3861

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	3861

A.9.1.6.2.1	Test Purpose and Environment	3861

A.9.1.6.2.2	Test Requirements	3865

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	3865

A.9.1.6.3.1	Test Purpose and Environment	3865

A.9.1.6.3.2	Test Requirements	3868

A.9.1.7	Selection / Reselection of relay UE	3868

A.9.1.7.1	Test Purpose and Environment	3868

A.9.1.7.2	Test Requirements	3874

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	3877

A.10.1	RRC_CONNECTED state mobility	3877

A.10.1.1	RRC connection mobility control	3877

A.10.1.1.1	Random Access	3877

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	3877

A.10.1.1.1.1.1	Test Purpose and Environment	3877

A.10.1.1.1.1.2	Test Requirements	3880

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	3880

A.10.1.1.1.1.2.2	Random Access Response Reception	3880

A.10.1.1.1.1.2.3	No Random Access Response Reception	3881

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	3881

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	3881

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	3882

A.10.1.1.1.2.1	Test Purpose and Environment	3882

A.10.1.1.1.2.2	Test Requirements	3885

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	3885

A.10.1.1.1.2.2.2	Random Access Response Reception	3886

A.10.1.1.1.2.2.3	No Random Access Response Reception	3886

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	3886

A.10.1.1.1.3.1	Test Purpose and Environment	3886

A.10.1.1.1.3.2	Test Requirements	3890

A.10.1.1.1.3.2.1	MsgA Transmission	3890

A.10.1.1.1.3.2.2	MsgB Reception	3890

A.10.1.1.1.3.2.3	No MsgB Reception	3891

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	3891

A.10.1.1.1.4.1	Test Purpose and Environment	3891

A.10.1.1.1.4.2	Test Requirements	3895

A.10.1.1.1.4.2.1	MsgA Transmission	3895

A.10.1.1.1.4.2.2	MsgB Reception	3896

A.10.1.1.1.4.2.3	No MsgB Reception	3896

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	3896

A.10.1.2.1	Test Purpose and Environment	3896

A.10.1.2.2	Test Requirements	3902

A.10.2	Timing	3903

A.10.2.1	UE transmit timing	3903

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	3903

A.10.2.1.1.1	Test Purpose and environment	3903

A.10.2.1.1.2	Test requirements	3906

A.10.2.2	UE timing advance	3907

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	3907

A.10.2.2.1.1	Test Purpose and Environment	3907

A.10.2.2.1.2	Test Parameters	3907

A.10.2.2.1.3	Test Requirements	3911

A.10.3	Signalling characteristics	3911

A.10.3.1	Radio link monitoring	3911

A.10.3.1.1	Introduction	3911

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	3912

A.10.3.1.2.1	Test purpose and environment	3912

A.10.3.1.2.2	Test requirements	3916

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	3917

A.10.3.1.3.1	Test purpose and environment	3917

A.10.3.1.3.2	Test requirements	3922

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	3922

A.10.3.1.4.1	Test purpose and environment	3922

A.10.3.1.4.2	Test requirements	3922

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	3922

A.10.3.1.5.1	Test purpose and environment	3922

A.10.3.1.5.2	Test requirements	3922

A.10.3.2	Interruption	3922

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	3922

A.10.3.2.1.1	Test Purpose and Environment	3922

A.10.3.2.1.2	Test Requirements	3927

A.10.3.3	SCell activation and deactivation delay	3927

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	3927

A.10.3.3.1.1	Test Purpose and Environment	3927

A.10.3.3.1.2	Test Requirements	3932

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	3932

A.10.3.3.2.1	Test Purpose and Environment	3932

A.10.3.3.2.2	Test Requirements	3933

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	3933

A.10.3.3.3.1	Test Purpose and Environment	3933

A.10.3.3.3.2	Test Requirements	3934

A.10.3.4	Beam failure detection and link recovery procedures	3934

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	3934

A.10.3.4.1.1	Test Purpose and Environment	3934

A.10.3.4.1.2	Test Requirements	3939

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	3939

A.10.3.4.2.1	Test Purpose and Environment	3939

A.10.3.4.2.2	Test Requirements	3945

A.10.3.5	Active BWP switching	3945

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	3945

A.10.3.5.1.2	Test Requirements	3950

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	3951

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	3951

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	3954

A.10.3.5.3	RRC-based Active BWP Switch	3958

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	3958

A.10.3.6	PSCell addition and release delay	3962

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	3962

A.10.3.6.1.1	Test purpose and environment	3962

A.10.3.6.1.2	Test Requirements	3966

A.10.3.7	Void	3967

A.10.4	Measurement procedure	3967

A.10.4.1	Intra-frequency measurements	3967

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	3967

A.10.4.1.1.1	Test purpose and environment	3967

A.10.4.1.1.2	Test parameters	3967

A.10.4.1.1.3	Test Requirements	3967

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	3968

A.10.4.1.2.1	Test purpose and environment	3968

A.10.4.1.2.2	Test parameters	3968

A.10.4.1.2.3	Test Requirements	3968

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	3968

A.10.4.1.3.1	Test purpose and environment	3968

A.10.4.1.3.2	Test parameters	3968

A.10.4.1.3.3	Test Requirements	3968

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	3969

A.10.4.1.4.1	Test purpose and environment	3969

A.10.4.1.4.2	Test parameters	3969

A.10.4.1.4.3	Test Requirements	3969

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	3969

A.10.4.1.5.1	Test purpose and environment	3969

A.10.4.1.5.2	Test parameters	3969

A.10.4.1.5.3	Test Requirements	3969

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	3970

A.10.4.1.6.1	Test purpose and environment	3970

A.10.4.1.6.2	Test parameters	3970

A.10.4.1.6.3	Test Requirements	3970

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	3970

A.10.4.1.7.1	Test purpose and environment	3970

A.10.4.1.7.2	Test parameters	3970

A.10.4.1.7.3	Test Requirements	3970

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	3971

A.10.4.1.8.1	Test purpose and environment	3971

A.10.4.1.8.2	Test parameters	3971

A.10.4.1.8.3	Test Requirements	3971

A.10.4.1.9	RSSI measurement reporting on PSCC	3971

A.10.4.1.9.1	Test purpose and environment	3971

A.10.4.1.9.2	Test parameters	3971

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	3972

A.10.4.1.10.1	Test purpose and environment	3972

A.10.4.1.10.2	Test parameters	3972

A.10.4.1.11	RSSI measurement reporting on SCC	3972

A.10.4.1.11.1	Test purpose and environment	3972

A.10.4.1.11.2	Test parameters	3972

A.10.4.1.12	Channel occupancy measurement reporting on SCC	3973

A.10.4.1.12.1	Test purpose and environment	3973

A.10.4.1.12.2	Test parameters	3973

A.10.4.2	Inter-frequency measurements	3973

A.10.4.2.1	RSSI measurement reporting	3973

A.10.4.2.1.1	Test purpose and environment	3973

A.10.4.2.1.2	Test parameters	3973

A.10.4.2.2	Channel occupancy measurement reporting	3974

A.10.4.2.2.1	Test purpose and environment	3974

A.10.4.2.2.2	Test parameters	3974

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	3974

A.10.4.2.3.1	Test Purpose and Environment	3974

A.10.4.2.3.2	Test Requirements	3979

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	3980

A.10.4.2.4.1	Test Purpose and Environment	3980

A.10.4.2.4.2	Test Requirements	3984

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	3985

A.10.4.2.5.1	Test Purpose and Environment	3985

A.10.4.2.5.2	Test Requirements	3989

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	3990

A.10.4.2.6.1	Test Purpose and Environment	3990

A.10.4.2.6.2	Test Requirements	3994

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	3995

A.10.4.2.7.1	Test Purpose and Environment	3995

A.10.4.2.7.2	Test Requirements	4001

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4001

A.10.4.2.8.1	Test Purpose and Environment	4001

A.10.4.2.8.2	Test Requirements	4007

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4008

A.10.4.2.9.1	Test Purpose and Environment	4008

A.10.4.2.9.2	Test Requirements	4013

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4013

A.10.4.2.10.1	Test Purpose and Environment	4013

A.10.4.2.10.2	Test Requirements	4019

A.10.4.3	L1-RSRP measurements for beam reporting	4020

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4020

A.10.4.3.1.1	Test Purpose and Environment	4020

A.10.4.3.1.2	Test parameters	4020

A.10.4.3.1.3	Test Requirements	4022

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4023

A.10.4.3.2.1	Test Purpose and Environment	4023

A.10.4.3.2.2	Test parameters	4023

A.10.4.3.2.3	Test Requirements	4025

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4026

A.10.4.3.3.1	Test Purpose and Environment	4026

A.10.4.3.3.2	Test parameters	4026

A.10.4.3.3.3	Test Requirements	4029

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4030

A.10.4.3.4.1	Test Purpose and Environment	4030

A.10.4.3.4.2	Test parameters	4030

A.10.4.3.4.3	Test Requirements	4033

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4034

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4034

A.10.4.4.1.1	Test Purpose and Environment	4034

A.10.4.4.1.2	Test Requirements	4038

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4039

A.10.4.4.2.1	Test Purpose and Environment	4039

A.10.4.4.2.2	Test Requirements	4044

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4045

A.10.4.4.3.1	Test Purpose and Environment	4045

A.10.4.4.3.2	Test Requirements	4049

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4050

A.10.4.4.4.1	Test Purpose and Environment	4050

A.10.4.4.4.2	Test Requirements	4055

A.10.5	Measurement performance	4056

A.10.5.1	SS-RSRP	4056

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4056

A.10.5.1.1.1	Test Purpose and Environment	4056

A.10.5.1.1.2	Test parameters	4056

A.10.5.1.1.3	Test Requirements	4059

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4059

A.10.5.1.2.1	Test Purpose and Environment	4059

A.10.5.1.2.2	Test parameters	4059

A.10.5.1.2.3	Test Requirements	4062

A.10.5.2	SS-RSRQ	4062

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4062

A.10.5.2.1.1	Test Purpose and Environment	4062

A.10.5.2.1.2	Test Parameters	4062

A.10.5.2.1.3	Test Requirements	4065

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4065

A.10.5.2.2.1	Test Purpose and Environment	4065

A.10.5.2.2.2	Test Parameters	4065

A.10.5.2.2.3	Test Requirements	4068

A.10.5.3	SS-SINR	4068

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4068

A.10.5.3.1.1	Test Purpose and Environment	4068

A.10.5.3.1.2	Test Parameters	4068

A.10.5.3.1.3	Test Requirements	4071

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4071

A.10.5.3.2.1	Test Purpose and Environment	4071

A.10.5.3.2.2	Test Parameters	4071

A.10.5.3.2.3	Test Requirements	4074

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4074

A.10.5.3.3.1	Test Purpose and Environment	4074

A.10.5.3.3.2	Test Parameters	4074

A.10.5.3.3.3	Test Requirements	4077

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4077

A.10.5.4.1	SSB based L1-RSRP measurement	4077

A.10.5.4.1.1	Test Purpose and Environment	4077

A.10.5.4.1.2	Test parameters	4078

A.10.5.4.1.3	Test Requirements	4080

A.10.5.5	RSSI	4080

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4080

A.10.5.5.1.1	Test Purpose and Environment	4080

A.10.5.5.1.2	Test parameters	4080

A.10.5.5.1.3	Test Requirements	4083

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4083

A.10.5.5.2.1	Test Purpose and Environment	4083

A.10.5.5.2.2	Test parameters	4083

A.10.5.5.2.3	Test Requirements	4086

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4086

A.10.5.5.3.1	Test Purpose and Environment	4086

A.10.5.5.3.2	Test parameters	4086

A.10.5.5.3.3	Test Requirements	4089

A.10.5.6	Channel occupancy	4089

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4089

A.10.5.6.1.1	Test Purpose and Environment	4089

A.10.5.6.1.2	Test parameters	4089

A.10.5.6.1.3	Test Requirements	4093

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4093

A.10.5.6.2.1	Test Purpose and Environment	4093

A.10.5.6.2.2	Test parameters	4093

A.10.5.6.2.3	Test Requirements	4096

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4096

A.10.5.6.3.1	Test Purpose and Environment	4096

A.10.5.6.3.2	Test parameters	4096

A.10.5.6.3.3	Test Requirements	4100

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4100

A.11.1	RRC_IDLE state mobility	4100

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4100

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4100

A.11.1.1.1.1	Test Purpose and Environment	4100

A.11.1.1.1.2	Test Parameters	4100

A.11.1.1.1.3	Test Requirements	4103

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4103

A.11.1.1.2.1	Test Purpose and Environment	4103

A.11.1.1.2.2	Test Parameters	4103

A.11.1.1.2.3	Test Requirements	4107

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4107

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4107

A.11.1.2.1.1	Test Purpose and Environment	4107

A.11.1.2.1.2	Test Parameters	4108

A.11.1.2.1.3	Test Requirements	4114

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4115

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4115

A.11.1.3.1.1	Test Purpose and Environment	4115

A.11.1.3.1.2	Test Parameters	4115

A.11.1.3.1.3	Test Requirements	4121

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4122

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4122

A.11.1.4.1.1	Test Purpose and Environment	4122

A.11.1.4.1.2	Test Parameters	4122

A.11.1.4.1.3	Test Requirements	4125

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4126

A.11.1.4.2.1	Test Purpose and Environment	4126

A.11.1.4.2.2	Test Requirements	4129

A.11.2	RRC_CONNECTED state mobility	4130

A.11.2.1	Handover	4130

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4130

A.11.2.1.1.1	Test Purpose and Environment	4130

A.11.2.1.1.2	Test Parameters	4130

A.11.2.1.1.3 Test Requirements	4133

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4133

A.11.2.1.2.1	Test Purpose and Environment	4133

A.11.2.1.2.2	Test Parameters	4133

A.11.2.1.2.3	Test Requirements	4136

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4136

A.11.2.1.3.1	Test Purpose and Environment	4136

A.11.2.1.3.2	Test Parameters	4136

A.11.2.1.3.3 Test Requirements	4139

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4139

A.11.2.1.4.1	Test Purpose and Environment	4139

A.11.2.1.4.2	Test Parameters	4139

A.11.2.1.4.3 Test Requirements	4143

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4144

A.11.2.1.5.1	Test Purpose and Environment	4144

A.11.2.1.5.2	Test Parameters	4144

A.11.2.1.5.3 Test Requirements	4147

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4148

A.11.2.1.6.1	Test Purpose and Environment	4148

A.11.2.1.6.2	Test Parameters	4148

A.11.2.1.6.3 Test Requirements	4151

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4152

A.11.2.1.7.1	Test Purpose and Environment	4152

A.11.2.1.7.2	Test Requirements	4158

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4158

A.11.2.1.8.1	Test Purpose and Environment	4158

A.11.2.1.8.2	Test Requirements	4163

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4163

A.11.2.1.9.1	Test Purpose and Environment	4163

A.11.2.1.9.2	Test Requirements	4172

A.11.2.2	RRC connection mobility control	4173

A.11.2.2.1	RRC re-establishment	4173

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4173

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4176

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4183

A.11.2.2.2	Random Access	4188

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4188

A.11.2.2.2.1.1	Test Purpose and Environment	4188

A.11.2.2.2.1.2	Test Requirements	4191

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4191

A.11.2.2.2.1.2.2	Random Access Response Reception	4191

A.11.2.2.2.1.2.3	No Random Access Response Reception	4192

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4192

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4192

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4193

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4193

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4193

A.11.2.2.2.2.1	Test Purpose and Environment	4193

A.11.2.2.2.2.2	Test Requirements	4196

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4196

A.11.2.2.2.2.2.2	Random Access Response Reception	4197

A.11.2.2.2.2.2.3	No Random Access Response Reception	4197

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4197

A.11.2.2.2.3.1	Test Purpose and Environment	4197

A.11.2.2.2.3.2	Test Requirements	4201

A.11.2.2.2.3.2.1	MsgA Transmission	4201

A.11.2.2.2.3.2.2	MsgB Reception	4201

A.11.2.2.2.3.2.3	No MsgB Reception	4202

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4202

A.11.2.2.2.4.1	Test Purpose and Environment	4202

A.11.2.2.2.4.2	Test Requirements	4206

A.11.2.2.2.4.2.1	MsgA Transmission	4206

A.11.2.2.2.4.2.2	MsgB Reception	4207

A.11.2.2.2.4.2.3	No MsgB Reception	4207

A.11.2.2.3	RRC connection release with redirection	4208

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4208

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4212

A.11.3	Timing	4217

A.11.3.1	UE transmit timing	4217

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4217

A.11.3.1.1.1	Test Purpose and environment	4217

A.11.3.1.1.2	Test requirements	4220

A.11.3.2	UE timing advance	4221

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4221

A.11.3.2.1.1	Test Purpose and Environment	4221

A.11.3.2.1.2	Test Parameters	4221

A.11.3.2.1.3	Test Requirements	4225

A.11.4	Signalling characteristics	4225

A.11.4.1	Radio link monitoring	4225

A.11.4.1.1	Introduction	4225

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4226

A.11.4.1.2.1	Test purpose and environment	4226

A.11.4.1.2.2	Test requirements	4230

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4230

A.11.4.1.3.1	Test purpose and environment	4230

A.11.4.1.3.2	Test requirements	4236

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	4236

A.11.4.1.4.1	Test purpose and environment	4236

A.11.4.1.4.2	Test requirements	4236

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	4236

A.11.4.1.5.1	Test purpose and environment	4236

A.11.4.1.5.2	Test requirements	4236

A.11.4.2	Interruption	4237

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4237

A.11.4.2.1.1 Test Purpose and Environment	4237

A.11.4.2.1.2	Test Requirements	4241

A.11.4.3	SCell activation and deactivation delay	4241

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4241

A.11.4.3.1.1	Test Purpose and Environment	4241

A.11.4.3.1.2	Test Requirements	4246

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4246

A.11.4.3.2.1	Test Purpose and Environment	4246

A.11.4.3.2.2	Test Requirements	4247

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4247

A.11.4.3.3.1	Test Purpose and Environment	4247

A.11.4.3.3.2	Test Requirements	4248

A.11.4.4	Beam failure detection and link recovery procedures	4248

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4248

A.11.4.4.1.1	Test Purpose and Environment	4248

A.11.4.4.1.2	Test Requirements	4253

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4253

A.11.4.4.2.1	Test Purpose and Environment	4253

A.11.4.4.2.2	Test Requirements	4259

A.11.4.5	Active BWP switching	4259

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4259

A.11.4.5.1.1	Test Purpose and Environment	4259

A.11.4.5.1.2	Test Requirements	4263

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4264

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4264

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4267

A.11.4.5.3	RRC-based Active BWP Switch	4270

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4270

A.11.4.6	Void	4273

A.11.5	Measurement procedure	4273

A.11.5.1	Intra-frequency measurements	4273

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4273

A.11.5.1.1.1	Test purpose and environment	4273

A.11.5.1.1.2	Test parameters	4273

A.11.5.1.1.3	Test Requirements	4274

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4274

A.11.5.1.2.1	Test purpose and environment	4274

A.11.5.1.2.2	Test parameters	4274

A.11.5.1.2.3	Test Requirements	4274

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4275

A.11.5.1.3.1	Test purpose and environment	4275

A.11.5.1.3.2	Test parameters	4275

A.11.5.1.3.3	Test Requirements	4275

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4275

A.11.5.1.4.1	Test purpose and environment	4275

A.11.5.1.4.2	Test parameters	4275

A.11.5.1.4.3	Test Requirements	4275

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4276

A.11.5.1.5.1	Test purpose and environment	4276

A.11.5.1.5.2	Test parameters	4276

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4276

A.11.5.1.6.1	Test purpose and environment	4276

A.11.5.1.6.2	Test parameters	4276

A.11.5.1.6.3	Test Requirements	4276

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4277

A.11.5.1.7.1	Test purpose and environment	4277

A.11.5.1.7.2	Test parameters	4277

A.11.5.1.7.3	Test Requirements	4277

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4277

A.11.5.1.8.1	Test purpose and environment	4277

A.11.5.1.8.2	Test parameters	4277

A.11.5.1.8.3	Test Requirements	4277

A.11.5.1.9	RSSI measurement reporting on PCC	4278

A.11.5.1.9.1	Test purpose and environment	4278

A.11.5.1.9.2	Test parameters	4278

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4278

A.11.5.1.10.1	Test purpose and environment	4278

A.11.5.1.10.2	Test parameters	4278

A.11.5.1.11	RSSI measurement reporting on SCC	4278

A.11.5.1.11.1	Test purpose and environment	4278

A.11.5.1.11.2	Test parameters	4279

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4279

A.11.5.1.12.1	Test purpose and environment	4279

A.11.5.1.12.2	Test parameters	4279

A.11.5.2	Inter-frequency measurements	4279

A.11.5.2.1	RSSI measurement reporting	4279

A.11.5.2.1.1	Test purpose and environment	4279

A.11.5.2.1.2	Test parameters	4280

A.11.5.2.2	Channel occupancy measurement reporting	4280

A.11.5.2.2.1	Test purpose and environment	4280

A.11.5.2.2.2	Test parameters	4280

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4280

A.11.5.2.3.1	Test Purpose and Environment	4280

A.11.5.2.3.2	Test Requirements	4284

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4285

A.11.5.2.4.1	Test Purpose and Environment	4285

A.11.5.2.4.2	Test Requirements	4290

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4290

A.11.5.2.5.1	Test Purpose and Environment	4290

A.11.5.2.5.2	Test Requirements	4294

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4295

A.11.5.2.6.1	Test Purpose and Environment	4295

A.11.5.2.6.2	Test Requirements	4300

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4300

A.11.5.2.7.1	Test Purpose and Environment	4300

A.11.5.2.7.2	Test Requirements	4304

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4305

A.11.5.2.8.1	Test Purpose and Environment	4305

A.11.5.2.8.2	Test Requirements	4310

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4310

A.11.5.2.9.1	Test Purpose and Environment	4310

A.11.5.2.9.2	Test Requirements	4315

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4316

A.11.5.2.10.1	Test Purpose and Environment	4316

A.11.5.2.10.2	Test Requirements	4321

A.11.5.3	Inter-RAT E-UTRAN measurements	4322

A.11.5.4	L1-RSRP measurements for beam reporting	4322

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4322

A.11.5.4.1.1	Test Purpose and Environment	4322

A.11.5.4.1.2	Test parameters	4322

A.11.5.4.1.3	Test Requirements	4324

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4325

A.11.5.4.2.1	Test Purpose and Environment	4325

A.11.5.4.2.2	Test parameters	4325

A.11.5.4.2.3	Test Requirements	4327

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4328

A.11.5.4.3.1	Test Purpose and Environment	4328

A.11.5.4.3.2	Test parameters	4328

A.11.5.4.3.3	Test Requirements	4331

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4332

A.11.5.4.4.1	Test Purpose and Environment	4332

A.11.5.4.4.2	Test parameters	4332

A.11.5.4.4.3	Test Requirements	4335

A.11.6	Measurement performance	4336

A.11.6.1	SS-RSRP	4336

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4336

A.11.6.1.1.1	Test Purpose and Environment	4336

A.11.6.1.1.2	Test parameters	4336

A.11.6.1.1.3	Test Requirements	4338

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4338

A.11.6.1.2.1	Test Purpose and Environment	4338

A.11.6.1.2.2	Test parameters	4338

A.11.6.1.2.3	Test Requirements	4340

A.11.6.2	SS-RSRQ	4340

A.11.6.2.1	Intra-frequency measurement accuracy	4340

A.11.6.2.1.1	Test Purpose and Environment	4340

A.11.6.2.1.2	Test Parameters	4340

A.11.6.2.1.3	Test Requirements	4343

A.11.6.2.2	Inter-frequency measurement accuracy	4343

A.11.6.2.2.1	Test Purpose and Environment	4343

A.11.6.2.2.2	Test Parameters	4343

A.11.6.2.2.3	Test Requirements	4346

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4346

A.11.6.2.3.1	Test Purpose and Environment	4346

A.11.6.2.3.2	Test Parameters	4346

A.11.6.2.3.3	Test Requirements	4349

A.11.6.2.4	Inter-frequency measurement accuracy	4349

A.11.6.2.4.1	Test Purpose and Environment	4349

A.11.6.2.4.2	Test Parameters	4349

A.11.6.2.4.3	Test Requirements	4358

A.11.6.3	SS-SINR	4358

A.11.6.3.1	Intra-frequency measurement accuracy	4358

A.11.6.3.1.1	Test Purpose and Environment	4358

A.11.6.3.1.2	Test Parameters	4358

A.11.6.3.1.3	Test Requirements	4361

A.11.6.3.2	Inter-frequency measurement accuracy	4361

A.11.6.3.2.1	Test Purpose and Environment	4361

A.11.6.3.2.2	Test Parameters	4361

A.11.6.3.2.3	Test Requirements	4364

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4364

A.11.6.3.3.1	Test Purpose and Environment	4364

A.11.6.3.3.2	Test Parameters	4364

A.11.6.3.3.3	Test Requirements	4367

A.11.6.3.4	Inter-frequency measurement accuracy	4367

A.11.6.3.4.1	Test Purpose and Environment	4367

A.11.6.3.4.2	Test Parameters	4367

A.11.6.3.4.3	Test Requirements	4375

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4375

A.11.6.4.1	SSB based L1-RSRP measurement	4375

A.11.6.4.1.1	Test Purpose and Environment	4375

A.11.6.4.1.2	Test parameters	4376

A.11.6.4.1.3	Test Requirements	4379

A.11.6.5	RSSI	4379

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4379

A.11.6.5.1.1	Test Purpose and Environment	4379

A.11.6.5.1.2	Test parameters	4379

A.11.6.5.1.3	Test Requirements	4382

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4382

A.11.6.5.2.1	Test Purpose and Environment	4382

A.11.6.5.2.2	Test parameters	4382

A.11.6.5.2.3	Test Requirements	4385

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4385

A.11.6.5.3.1	Test Purpose and Environment	4385

A.11.6.5.3.2	Test parameters	4385

A.11.6.5.3.3	Test Requirements	4388

A.11.6.6	Channel occupancy	4388

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4388

A.11.6.6.1.1	Test Purpose and Environment	4388

A.11.6.6.1.2	Test parameters	4388

A.11.6.6.1.3	Test Requirements	4391

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4391

A.11.6.6.2.1	Test Purpose and Environment	4391

A.11.6.6.2.2	Test parameters	4391

A.11.6.6.2.3	Test Requirements	4394

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4394

A.11.6.6.3.1	Test Purpose and Environment	4394

A.11.6.6.3.2	Test parameters	4394

A.11.6.6.3.3	Test Requirements	4397

A.11.6.7	E-UTRAN RSRP	4397

A.11.6.8	E-UTRAN RSRQ	4397

A.11.6.9	E-UTRAN SINR	4397

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4397

A.12.1	RRC_IDLE state mobility	4397

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4397

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4397

A.12.1.1.1.1	Test Purpose and Environment	4397

A.12.1.1.1.2	Test Requirements	4402

A.12.2	RRC_CONNECTED state mobility	4403

A.12.2.1	Handover	4403

A.12.2.1.1	E-UTRAN - NR with CCA handover	4403

A.12.2.1.1.1	Test Purpose and Environment	4403

A.12.2.1.1.2	Test Requirements	4408

A.12.3	Signalling characteristics	4409

A.12.3.1	Interruptions	4409

A.12.4	Measurement procedure	4409

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4409

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4409

A.12.4.1.1.1	Test Purpose and Environment	4409

A.12.4.1.1.2	Test Requirements	4413

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4413

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4413

A.12.4.2.1.1	Test Purpose and Environment	4413

A.12.4.2.1.2	Test Requirements	4417

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4418

A.12.4.2.2.1	Test Purpose and Environment	4418

A.12.4.2.2.2	Test Requirements	4422

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4422

A.12.4.2.3.1	Test Purpose and Environment	4422

A.12.4.2.3.2	Test Requirements	4426

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4427

A.12.4.2.4.1	Test Purpose and Environment	4427

A.12.4.2.4.2	Test Requirements	4431

A.12.4.2.5	RSSI measurement reporting	4431

A.12.4.2.5.1	Test purpose and environment	4431

A.12.4.2.5.2	Test parameters	4431

A.12.4.2.6	Channel occupancy measurement reporting	4432

A.12.4.2.6.1	Test purpose and environment	4432

A.12.4.2.6.2	Test parameters	4432

A.12.5	Measurement performance	4432

A.12.5.1	E-UTRANNR SFTD	4432

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4432

A.12.5.1.1.1	Test Purpose	4432

A.12.5.1.1.2	Test Environment	4433

A.12.5.1.1.3	Test Requirements	4438

A.12.5.2	E-UTRANNR SS-RSRP	4438

A.12.5.3	E-UTRANNR SS-RSRQ	4438

A.12.5.4	E-UTRANNR SS-SINR	4438

A.12.5.5	E-UTRANNR RSSI	4438

A.12.5.6	E-UTRANNR channel occupancy	4438

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4442

A.13.1	Timing	4442

A.13.1.1	UE transmit timing	4442

A.13.1.2	Timing advance	4442

A.13.2	Signalling characteristics	4442

A.13.2.1	Interruption	4442

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4442

A.13.2.1.1.1	Test Purpose and Environment	4442

A.13.2.1.1.2	Test Requirements	4446

A.13.2.2	SCell activation and deactivation delay	4446

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4446

A.13.2.2.1.1	Test Purpose and Environment	4446

A.13.2.2.1.2	Test Requirements	4451

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4451

A.13.2.2.2.1	Test Purpose and Environment	4451

A.13.2.2.2.2	Test Requirements	4452

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4452

A.13.2.2.3.1	Test Purpose and Environment	4452

A.13.2.2.3.2	Test Requirements	4453

A.13.2.3	Void	4453

A.13.3	Measurement procedure	4453

A.13.3.1	Intra-frequency measurements	4453

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4453

A.13.3.1.1.1	Test purpose and environment	4453

A.13.3.1.1.2	Test parameters	4453

A.13.3.1.1.3	Test Requirements	4458

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4458

A.13.3.1.2.1	Test purpose and environment	4458

A.13.3.1.2.2	Test parameters	4458

A.13.3.1.2.3	Test Requirements	4463

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4463

A.13.3.1.3.1	Test purpose and environment	4463

A.13.3.1.3.2	Test parameters	4463

A.13.3.1.3.3	Test Requirements	4468

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4468

A.13.3.1.4.1	Test purpose and environment	4468

A.13.3.1.4.2	Test parameters	4468

A.13.3.1.4.3	Test Requirements	4473

A.13.3.1.5	RSSI measurement reporting on SCC	4473

A.13.3.1.5.1	Test purpose and environment	4473

A.13.3.1.5.2	Test parameters	4473

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4474

A.13.3.1.6.1	Test purpose and environment	4474

A.13.3.1.6.2	Test parameters	4475

A.13.3.2	Inter-frequency measurements	4475

A.13.3.2.1	RSSI measurement reporting	4475

A.13.3.2.1.1	Test purpose and environment	4475

A.13.3.2.1.2	Test parameters	4475

A.13.3.2.2	Channel occupancy measurement reporting	4476

A.13.3.2.2.1	Test purpose and environment	4476

A.13.3.2.2.2	Test parameters	4476

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4476

A.13.3.2.3.1	Test Purpose and Environment	4476

A.13.3.2.3.2	Test Requirements	4481

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4481

A.13.3.2.4.1	Test Purpose and Environment	4481

A.13.3.2.4.2	Test Requirements	4486

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4487

A.13.3.2.5.1	Test Purpose and Environment	4487

A.13.3.2.5.2	Test Requirements	4492

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4492

A.13.3.2.6.1	Test Purpose and Environment	4492

A.13.3.2.6.2	Test Requirements	4498

A.13.3.3	L1-RSRP measurements for beam reporting	4499

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4499

A.13.3.3.1.1	Test Purpose and Environment	4499

A.13.3.3.1.2	Test parameters	4499

A.13.3.3.1.3	Test Requirements	4503

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4503

A.13.3.3.2.1	Test Purpose and Environment	4503

A.13.3.3.2.2	Test parameters	4503

A.13.3.3.2.3	Test Requirements	4507

A.13.4	Measurement performance	4507

A.13.4.1	SS-RSRP	4507

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4507

A.13.4.1.1.1	Test Purpose and Environment	4507

A.13.4.1.1.2	Test parameters	4507

A.13.4.1.1.3	Test Requirements	4509

A.13.4.2	SS-RSRQ	4509

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4509

A.13.4.2.1.1	Test Purpose and Environment	4509

A.13.4.2.1.2	Test Parameters	4509

A.13.4.2.1.3	Test Requirements	4517

A.13.4.3	SS-SINR	4517

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4517

A.13.4.3.1.1	Test Purpose and Environment	4517

A.13.4.3.1.2	Test Parameters	4517

A.13.4.3.1.3	Test Requirements	4524

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4524

A.13.4.4.1	SSB based L1-RSRP measurement	4524

A.13.4.4.1.1	Test Purpose and Environment	4524

A.13.4.4.1.2	Test parameters	4524

A.13.4.4.1.3	Test Requirements	4528

A.13.4.5	RSSI	4528

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4528

A.13.4.5.1.1	Test Purpose and Environment	4528

A.13.4.5.1.2	Test parameters	4528

A.13.4.5.1.3	Test Requirements	4531

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4531

A.13.4.5.2.1	Test Purpose and Environment	4531

A.13.4.5.2.2	Test parameters	4531

A.13.4.5.2.3	Test Requirements	4535

A.13.4.6	Channel occupancy	4535

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4535

A.13.4.6.1.1	Test Purpose and Environment	4535

A.13.4.6.1.2	Test parameters	4535

A.13.4.6.1.3	Test Requirements	4539

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4539

A.13.4.6.2.1	Test Purpose and Environment	4539

A.13.4.6.2.2	Test parameters	4539

A.13.4.6.2.3	Test Requirements	4543

A.14	NR standalone tests for Satellite access	4543

A.14.1	RRC_IDLE state mobility	4543

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4543

A.14.1.1.1	Test Purpose and Environment	4543

A.14.1.1.2	Test Parameters	4543

A.14.1.1.3	Test Requirements	4545

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements]	4546

A.14.1.2.1	Test Purpose and Environment	4546

A.14.1.2.2	Test Parameters	4546

A.14.1.2.3	Test Requirements	4548

A.14.1.3	Time-based cell reselection to FR1 intra-frequency NR cell	4549

A.14.1.3.1	Test Purpose and Environment	4549

A.14.1.3.2	Test Parameters	4549

A.14.1.3.3	Test Requirements	4551

A.14.1.4	Location-based cell reselection to FR1 intra-frequency NR cell	4552

A.14.1.4.1	Test Purpose and Environment	4552

A.14.1.4.2	Test Parameters	4552

A.14.1.4.3	Test Requirements	4554

A.14.2	RRC_CONNECTED state mobility	4555

A.14.2.1	Handover	4555

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4555

A.14.2.1.1.1	Test Purpose and Environment	4555

A.14.2.1.1.2	Test Parameters	4555

A.14.2.1.1.3	Test Requirements	4558

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4558

A.14.2.1.2.1	Test Purpose and Environment	4558

A.14.2.1.2.2	Test Parameters	4558

A.14.2.1.2.3	Test Requirements	4560

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4560

A.14.2.1.3.1	Test Purpose and Environment	4560

A.14.2.1.3.2	Test Parameters	4560

A.14.2.1.3.3	Test Requirements	4562

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4562

A.14.2.1.4.1	Test Purpose and Environment	4562

A.14.2.1.4.2	Test Parameters	4562

A.14.2.1.4.3	Test Requirements	4565

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4565

A.14.2.1.5.1	Test Purpose and Environment	4565

A.14.2.1.5.2	Test Parameters	4565

A.14.2.1.5.3	Test Requirements	4568

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4568

A.14.2.1.6.1	Test Purpose and Environment	4568

A.14.2.1.6.2	Test Parameters	4568

A.14.2.1.6.3	Test Requirements	4571

A.14.2.2	RRC Connection Mobility Control	4571

A.14.2.2.1	SA: RRC Re-establishment for SAN	4571

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4571

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4574

A.14.3	Timing for Satellite Access	4577

A.14.3.1	UE transmit timing for Satellite Access	4577

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4577

A.14.3.1.1.1	Test Purpose and environment	4577

A.14.3.1.1.2	Test requirements	4580

A.14.4	Signalling characteristics	4581

A.14.4.1	Radio link Monitoring	4581

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4581

A.14.4.1.1.1	Test Purpose and Environment	4581

A.14.4.1.1.2	Test Requirements	4586

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4586

A.14.4.1.2.1	Test Purpose and Environment	4586

A.14.4.1.2.2	Test Requirements	4592

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4592

A.14.4.1.3.1	Test Purpose and Environment	4592

A.14.4.1.3.2	Test Requirements	4596

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4596

A.14.4.1.4.1	Test Purpose and Environment	4596

A.14.4.1.4.2	Test Requirements	4601

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4601

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4601

A.14.4.2.1.1	Test Purpose and Environment	4601

A.14.4.2.1.2	Test Requirements	4607

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4607

A.14.4.2.2.1	Test Purpose and Environment	4607

A.14.4.2.2.2	Test Requirements	4614

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4614

A.14.4.2.3.1	Test Purpose and Environment	4614

A.14.4.2.3.2	Test Requirements	4620

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4620

A.14.4.2.4.1	Test Purpose and Environment	4620

A.14.4.2.4.2	Test Requirements	4626

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	4626

A.14.4.2.5.1	Test Purpose and Environment	4626

A.14.4.2.5.2	Test Requirements	4630

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	4631

A.14.4.2.6.1	Test Purpose and Environment	4631

A.14.4.2.6.2	Test Requirements	4635

A.14.4.3	Active BWP switch for satellite access	4635

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4635

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4635

A.14.4.3.1.1.1	Test Purpose and Environment	4635

A.14.4.3.1.1.2	Test Requirements	4640

A.14.4.3.2	RRC-based Active BWP Switch	4640

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4640

A.14.4.3.2.1.1	Test Purpose and Environment	4640

A.14.4.3.2.1.2	Test Requirements	4644

A.14.4.4	UE specific CBW change for satellite access	4644

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4644

A.14.4.4.1.1	Test Purpose and Environment	4644

A.14.4.4.1.2	Test Requirements	4648

A.14.5	Measurement procedure	4648

A.14.5.1	Intra-frequency Measurements	4648

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4648

A.14.5.1.1.1	Test purpose and Environment	4648

A.14.5.1.1.2	Test parameters	4648

A.14.5.1.1.3	Test Requirements	4650

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4651

A.14.5.1.2.1	Test purpose and Environment	4651

A.14.5.1.2.2	Test parameters	4651

A.14.5.1.2.3	Test Requirements	4653

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4654

A.14.5.1.3.1	Test purpose and Environment	4654

A.14.5.1.3.2	Test parameters	4654

A.14.5.1.3.3	Test Requirements	4656

A.14.5.2	Intra-frequency Measurements	4657

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4657

A.14.5.2.1.1	Test Purpose and Environment	4657

A.14.5.2.1.2	Test Requirements	4661

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4661

A.14.5.2.2.1	Test Purpose and Environment	4661

A.14.5.2.2.2	Test Requirements	4665

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4665

A.14.5.2.3.1	Test Purpose and Environment	4665

A.14.5.2.3.2	Test Requirements	4669

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4669

A.14.5.2.4.1	Test Purpose and Environment	4669

A.14.5.2.4.2	Test Requirements	4672

A.14.5.2.5	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with two gaps in fully non-overlapped for satellite access	4672

A.14.5.2.5.1	Test Purpose and Environment	4672

A.14.5.2.5.2	Test Requirements	4676

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4677

A.14.5.2.6.1	Test Purpose and Environment	4677

A.14.5.2.6.2	Test Requirements	4681

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4681

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4681

A.14.5.3.1.1	Test Purpose and Environment	4681

A.14.5.3.1.2	Test parameters	4681

A.14.5.3.1.3	Test Requirements	4684

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4684

A.14.5.3.2.1	Test Purpose and Environment	4684

A.14.5.3.2.2	Test parameters	4684

A.14.5.3.2.3	Test Requirements	4688

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	4688

A.14.5.3.3.1	Test Purpose and Environment	4688

A.14.5.3.3.2	Test parameters	4688

A.14.5.3.3.3	Test Requirements	4692

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	4692

A.14.5.3.4.1	Test Purpose and Environment	4692

A.14.5.3.4.2	Test parameters	4693

A.14.5.3.4.3	Test Requirements	4696

A.14.6	Measurement Performance requirements	4696

A.14.6.1	 SS-RSRP for SAN	4696

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4696

A.14.6.1.1.1	Test Purpose and Environment	4696

A.14.6.1.1.2	Test parameters	4696

A.14.6.1.1.3	Test Requirements	4700

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4700

A.14.6.1.2.1	Test Purpose and Environment	4700

A.14.6.1.2.2	Test parameters	4700

A.14.6.1.2.3	Test Requirements	4703

A.14.6.2	SS-RSRQ	4703

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4703

A.14.6.2.1.1	Test Purpose and Environment	4703

A.14.6.2.1.2	Test Parameters	4703

A.14.6.2.1.3	Test Requirements	4706

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4706

A.14.6.2.2.1	Test Purpose and Environment	4706

A.14.6.2.2.2	Test Parameters	4706

A.14.6.2.2.3	Test Requirements	4708

A.15	NR standalone tests with one or more NR cells in FR2-2	4708

A.15.1	SA: RRC_IDLE state mobility	4708

A.15.1.1	Cell re-selection to NR	4708

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	4708

A.15.1.1.1.1	Test Purpose and Environment	4708

A.15.1.1.1.2	Test Parameters	4708

A.15.1.1.1.3	Test Requirements	4712

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	4712

A.15.1.2.1	Test Purpose and Environment	4712

A.15.1.2.2	Test Parameters	4712

A.15.1.2.3	Test Requirements	4716

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4716

A.15.1.3.1	Test Purpose and Environment	4716

A.15.1.3.2	Test Parameters	4716

A.15.1.3.3	Test Requirements	4719

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4719

A.15.1.4.1	Test Purpose and Environment	4719

A.15.1.4.2	Test Parameters	4719

A.15.1.4.3	Test Requirements	4722

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4722

A.15.1.5.1	Test Purpose and Environment	4722

A.15.1.5.2	Test Parameters	4722

A.15.1.5.3	Test Requirements	4726

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4726

A.15.1.6.1	Test Purpose and Environment	4726

A.15.1.6.2	Test Parameters	4726

A.15.1.6.3	Test Requirements	4729

A.15.2	Signaling characteristics	4730

A.15.2.1	SCell Activation and Deactivation Delay	4730

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	4730

A.15.2.1.1.1	Test Purpose and Environment	4730

A.15.2.1.1.2	Test Requirements	4732

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	4732

A.15.2.1.2.1	Test Purpose and Environment	4732

A.15.2.1.2.2	Test Requirements	4737

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	4738

A.15.2.1.3.1	Test Purpose and Environment	4738

A.15.2.1.3.2	Test Requirements	4741

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	4742

A.15.2.1.4.1	Test Purpose and Environment	4742

A.15.2.1.4.2	Test Requirements	4745

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	4746

A.15.2.1.5.1	Test Purpose and Environment	4746

A.15.2.1.5.2	Test Requirements	4750

A.16	NR standalone tests with all NR cells in FR1 for RedCap	4751

A.16.1	SA: RRC_IDLE state mobility for RedCap	4751

A.16.1.1	Cell re-selection to NR	4751

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	4751

A.16.1.1.1.1	Test Purpose and Environment	4751

A.16.1.1.1.2	Test Parameters	4752

A.16.1.1.1.3	Test Requirements	4756

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	4756

A.16.1.1.2.1	Test Purpose and Environment	4756

A.16.1.1.2.2	Test Parameters	4756

A.16.1.1.2.3	Test Requirements	4760

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	4760

A.16.1.1.3.1	Test Purpose and Environment	4760

A.16.1.1.3.2	Test Parameters	4760

A.16.1.1.3.3	Test Requirements	4765

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	4765

A.16.1.1.4.1	Test Purpose and Environment	4765

A.16.1.1.4.2	Test Parameters	4765

A.16.1.1.4.3	Test Requirements	4770

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4770

A.16.1.1.5.1	Test Purpose and Environment	4770

A.16.1.1.5.2	Test Parameters	4770

A.16.1.1.5.3	Test Requirements	4775

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4775

A.16.1.1.6.1	Test Purpose and Environment	4775

A.16.1.1.6.2	Test Parameters	4775

A.16.1.1.6.3	Test Requirements	4779

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4779

A.16.1.1.7.1	Test Purpose and Environment	4779

A.16.1.1.7.2	Test Parameters	4779

A.16.1.1.7.3	Test Requirements	4783

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4783

A.16.1.1.8.1	Test Purpose and Environment	4783

A.16.1.1.8.2	Test Parameters	4784

A.16.1.1.8.3	Test Requirements	4788

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	4789

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	4789

A.16.1.2.1.1	Test Purpose and Environment	4789

A.16.1.2.1.2	Test Parameters	4789

A.16.1.2.1.3	Test Requirements	4792

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	4793

A.16.1.2.2.1	Test Purpose and Environment	4793

A.16.1.2.2.2	Test Parameters	4793

A.16.1.2.2.3	Test Requirements	4796

A.16.1.2.3.1	Test Purpose and Environment	4797

A. 16.1.2.3.2	Test Parameters	4797

A.16.1.2.3.3	Test Requirements	4800

A.16.1.2.4.1	Test Purpose and Environment	4801

A.16.1.2.4.2	Test Parameters	4801

A.16.1.3.1.3	Test Requirements	4804

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4805

A.16.1.2.5.1	Test Purpose and Environment	4805

A.16.1.2.5.2	Test Parameters	4805

A.16.1.2.5.3	Test Requirements	4808

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4809

A.16.1.2.6.1	Test Purpose and Environment	4809

A.16.1.2.6.2	Test Parameters	4809

A.16.1.2.6.3	Test Requirements	4812

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	4813

A.16.3	RRC_CONNECTED state mobility for RedCap	4813

A.16.3.1	Handover	4813

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	4813

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	4813

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	4813

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	4813

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	4813

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	4813

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	4813

A.16.3.1.7.1	Test Purpose and Environment	4813

A.16.3.1.7.2	Test Requirements	4819

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	4819

A.16.3.1.8.1	Test Purpose and Environment	4819

A.16.3.1.8.2	Test Requirements	4825

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	4825

A.16.3.1.9.1	Test Purpose and Environment	4825

A.16.3.1.9.2	Test Requirements	4831

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	4831

A.16.3.1.10.1	Test Purpose and Environment	4831

A.16.3.1.10.2	Test Requirements	4837

A.16.3.2	RRC Connection Mobility Control	4837

A.16.3.2.1	SA: RRC Re-establishment	4837

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	4837

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	4841

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	4845

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	4850

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	4855

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	4859

A.16.3.2.2	Random Access	4863

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	4863

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	4884

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	4888

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	4892

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	4896

A.16.3.2.3	SA: RRC Connection Release with Redirection	4900

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	4900

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	4904

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	4908

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	4915

A.16.4	Timing for RedCap	4922

A.16.4.1	UE transmit timing	4922

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	4922

A.16.4.1.1.1	Test Purpose and environment	4922

A.16.4.1.1.2	Test requirements	4924

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	4925

A.16.4.1.2.1	Test Purpose and environment	4925

A.16.4.1.2.2	Test requirements	4927

A.16.4.2	UE timer accuracy	4927

A.16.4.3	Timing advance	4927

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	4927

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	4927

A.16.5	Signalling characteristics for RedCap	4927

A.16.5.1	Radio link Monitoring	4927

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	4927

A.16.5.1.1.1	Test Purpose and Environment	4927

A.16.5.1.1.2	Test Requirements	4932

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	4932

A.16.5.1.2.1	Test Purpose and Environment	4932

A.16.5.1.2.2	Test Requirements	4938

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	4938

A.16.5.1.3.1	Test Purpose and Environment	4938

A.16.5.1.3.2	Test Requirements	4944

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	4944

A.16.5.1.4.1	Test Purpose and Environment	4944

A.16.5.1.4.2	Test Requirements	4950

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	4950

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	4950

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	4950

A.16.5.1.7.1	Test Purpose and Environment	4950

A.16.5.1.7.2	Test Requirements	4956

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	4956

A.16.5.1.8.1	Test Purpose and Environment	4956

A.16.5.1.8.2	Test Requirements	4962

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	4963

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	4963

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	4963

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	4963

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	4963

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	4963

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	4963

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	4963

A.16.5.2	Beam Failure Detection and Link recovery procedures	4963

A.16.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	4963

A.16.5.5.1.1	Test Purpose and Environment	4963

A.16.5.5.1.2	Test Requirements	4969

A.16.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	4969

A.16.5.5.2.1	Test Purpose and Environment	4969

A.16.5.5.2.2	Test Requirements	4975

A.16.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	4976

A.16.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	4976

A.16.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	4976

A.16.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	4976

A.16.5.5.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	4976

A.16.5.5.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	4976

A.16.5.3	Active BWP switch	4976

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	4976

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	4976

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	4976

A.16.5.3.2	RRC-based Active BWP Switch	4976

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	4976

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	4976

A.16.5.8	UE specific CBW change	4976

A.16.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	4976

A.16.5.8.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	4976

A.16.6	Measurement procedure for RedCap	4977

A.16.6.1	Intra-frequency Measurements	4977

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	4977

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	4977

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	4977

A.16.6.1.3.1	Test purpose and Environment	4977

A.16.6.1.3.2	Test parameters	4977

A.16.6.1.3.3	Test Requirements	4981

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	4981

A.16.6.1.4.1	Test purpose and Environment	4981

A.16.6.1.4.2	Test parameters	4981

A.16.6.1.4.3	Test Requirements	4985

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	4985

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	4985

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	4985

A.16.6.1.7.1	Test purpose and Environment	4985

A.16.6.1.7.2	Test parameters	4985

A.16.6.1.7.3	Test Requirements	4989

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	4989

A.16.6.1.8.1	Test purpose and Environment	4989

A.16.6.1.8.2	Test parameters	4989

A.16.6.1.8.3	Test Requirements	4993

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	4993

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	4993

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	4993

A.16.6.1.11.1	Test purpose and Environment	4993

A.16.6.1.11.2	Test parameters	4993

A.16.6.1.11.3	Test Requirements	4995

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	4995

A.16.6.1.12.1	Test purpose and Environment	4995

A.16.6.1.12.2	Test parameters	4996

A.16.6.1.11.3	Test Requirements	4997

A.16.6.2	Inter-frequency Measurements	4998

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	4998

A.16.6.2.1.1	Test Purpose and Environment	4998

A.16.6.2.1.2	Test Requirements	5003

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5003

A.16.6.2.2.1	Test Purpose and Environment	5003

A.16.6.2.2.2	Test Requirements	5008

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5008

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5008

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5008

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5008

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5008

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5008

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5008

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5008

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5008

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5008

A.16.6.3	Inter-RAT Measurements	5009

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5009

A.16.6.3.1.1	Test purpose and Environment	5009

A.16.6.3.1.2	Test Requirements	5014

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5014

A.16.6.3.2.1	Test purpose and Environment	5014

A.16.6.3.2.2	Test Requirements	5019

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5019

A.16.6.3.3.1	Test purpose and Environment	5019

A.16.6.3.3.2	Test Requirements	5024

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5024

A.16.6.3.4.1	Test purpose and Environment	5024

A.16.6.3.4.2	Test Requirements	5029

A.16.6.4	L1-RSRP measurement for beam reporting	5029

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5029

A.16.6.4.1.1	Test Purpose and Environment	5029

A.16.6.4.1.2	Test parameters	5029

A.16.6.4.1.3	Test Requirements	5032

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5032

A.16.6.4.2.1	Test Purpose and Environment	5032

A.16.6.4.2.2	Test parameters	5033

A.16.6.4.2.3	Test Requirements	5036

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5036

A.16.6.4.3.1	Test Purpose and Environment	5036

A.16.6.4.3.2	Test parameters	5037

A.16.6.4.3.3	Test Requirements	5040

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5040

A.16.6.4.4.1	Test Purpose and Environment	5040

A.16.6.4.4.2	Test parameters	5041

A.16.6.4.4.3	Test Requirements	5044

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5044

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5044

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5044

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5044

A.16.6.7	NR measurements with autonomous gaps	5045

A.16.6.7. 1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5045

A.16.6.7. 2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5045

A.16.6.7.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5045

A.16.6.7.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5045

A.16.7	Measurement Performance requirements for RedCap	5045

A.16.7.1	SS-RSRP	5045

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5045

A.16.7.1.1.1	Test Purpose and Environment	5045

A.16.7.1.1.2	Test parameters	5045

A.16.7.1.1.3	Test Requirements	5050

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5050

A.16.7.1.2.1	Test Purpose and Environment	5050

A.16.7.1.2.2	Test parameters	5050

A.16.7.1.2.3	Test Requirements	5055

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5055

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5055

A.16.7.2	SS-RSRQ	5055

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5055

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5055

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5055

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5055

A.16.7.3	SS-SINR	5056

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5056

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5056

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5056

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5056

A.16.7.4	L1-RSRP measurement for beam reporting	5056

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5056

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5056

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5056

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5056

A.16.7.5	E-UTRAN RSRP	5056

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5056

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5056

A.16.7.6	E-UTRAN RSRQ	5056

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5056

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5056

A.16.7.7	E-UTRAN RS-SINR	5056

A.16.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5056

A.16.7.7.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5056

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5057

A.17.1	SA: RRC_IDLE state mobility for RedCap	5057

A.17.1.1	Cell re-selection to NR	5057

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5057

A.17.1.1.1.1	Test Purpose and Environment	5057

A.17.1.1.1.2	Test Parameters	5057

A.17.1.1.1.3	Test Requirements	5061

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5061

A.17.1.1.2.1	Test Purpose and Environment	5061

A.17.1.1.2.2	Test Parameters	5061

A.17.1.1.2.3	Test Requirements	5065

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5065

A.17.1.1.3.1	Test Purpose and Environment	5065

A.17.1.1.3.2	Test Parameters	5065

A.17.1.1.3.3	Test Requirements	5068

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5068

A.17.1.1.4.1	Test Purpose and Environment	5068

A.17.1.1.4.2	Test Parameters	5068

A.17.1.1.4.3	Test Requirements	5071

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5071

A.17.3	RRC_CONNECTED state mobility for RedCap	5071

A.17.3.1	Handover for RedCap	5071

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5071

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5071

A.17.3.2	RRC Connection Mobility Control for RedCap	5072

A.17.3.2.1	SA: RRC Re-establishment	5072

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5072

A.17.3.2.1.1.1	Test Purpose and Environment	5072

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5075

A.17.3.2.1.2.1	Test Purpose and Environment	5075

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5078

A.17.3.2.1.3.1	Test Purpose and Environment	5078

A.17.3.2.1.3.2	Test Requirements	5080

A.17.3.2.2	Random Access	5081

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5081

A.17.3.2.2.1.1	Test Purpose and Environment	5081

A.17.3.2.2.1.2	Test Requirements	5083

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5085

A.17.3.2.2.2.1	Test Purpose and Environment	5085

A.17.3.2.2.2.2	Test Requirements	5087

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5089

A.17.3.2.2.3.1	Test Purpose and Environment	5089

A.17.3.2.2.3.2	Test Requirements	5091

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5092

A.17.3.2.2.4.1	Test Purpose and Environment	5092

A.17.3.2.2.4.2	Test Requirements	5094

A.17.3.2.3	SA: RRC Connection Release with Redirection	5095

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5095

A.17.3.2.3.1.1	Test Purpose and Environment	5095

A.17.3.2.3.1.2	Test Parameters	5095

A.17.3.2.3.1.3	Test Requirements	5099

A.17.4	Timing	5099

A.17.4.1	UE transmit timing	5099

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5099

A.17.4.2	UE timer accuracy	5099

A.17.4.3	Timing advance	5099

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5099

A.17.5	Signaling characteristics for RedCap	5099

A.17.5.1	Radio link Monitoring for RedCap	5099

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5099

A.17.5.1.1.1	Test Purpose and Environment	5099

A.17.5.1.1.2	Test Requirements	5103

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5104

A.17.5.1.2.1	Test Purpose and Environment	5104

A.17.5.1.2.2	Test Requirements	5108

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5109

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5109

A.17.5.1.4.1	Test Purpose and Environment	5109

A.17.5.1.4.2	Test Requirements	5113

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5113

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5113

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5113

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5113

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5113

A.17.5.2	Beam Failure Detection and Link recovery procedures	5114

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5114

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5114

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5114

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5114

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5114

A.17.5.3	Active BWP switch for RedCap	5114

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5114

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5114

A.17.5.3.2	RRC-based Active BWP Switch	5114

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5114

A.17.5.4	Active TCI state switch delay	5114

A.17.5.4.1	MAC-CE based active TCI state switch	5114

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5114

A.17.5.4.2	RRC based active TCI state switch	5114

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5114

A.17.5.5	Uplink spatial relation switch delay	5115

A.17.5.5.1	MAC-CE based Spatial Relation switch	5115

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5115

A.17.5.5.2	RRC based spatial relation switch	5115

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5115

A.17.5.6	UE specific CBW change	5115

A.17.5.5.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5115

A.17.6	Measurement procedure for RedCap	5115

A.17.6.1	Intra-frequency Measurements	5115

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5115

A.17.6.1.1.1	Test purpose and Environment	5115

A.17.6.1.1.2	Test Requirements	5118

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5118

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5118

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5118

A.17.6.2	Inter-frequency Measurements	5118

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5118

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5118

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5118

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	5118

A.17.6.3	L1-RSRP measurement for beam reporting	5118

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5118

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5118

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5118

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5118

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	5119

A.17.7	Measurement Performance requirements	5119

A.17.7.1	SS-RSRP	5119

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5119

A.17.7.1.1.1	Test Purpose and Environment	5119

A.17.7.1.1.2	Test parameters	5119

A.17.7.1.1.3	Test Requirements	5122

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5123

A.17.7.2	SS-RSRQ	5123

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5123

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5123

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5123

A.17.7.3	L1-RSRP measurement for beam reporting	5123

A.17.7.3.1	SSB based L1-RSRP measurement	5123

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5123

A.18	E-UTRA standalone tests for NR RRM for RedCap	5124

A.18.1	RRC_IDLE state mobility	5124

A.18.1.1	Inter-RAT NR Cell re-selection	5124

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5124

A.18.1.1.1.1	Test Purpose and Environment	5124

A.18.2.1.1.2	Test Requirements	5129

A.18.2	RRC_CONNECTED state mobility	5129

A.18.2.1	Handover	5129

A.18.2.1.1	E-UTRAN - NR handover in FR1	5129

A.18.2.1.1.1	Test Purpose and Environment	5129

A.18.2.1.1.2	Test Requirements	5134

A.18.2.2	RRC connection release with redirection	5134

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5134

A.18.2.2.1.1	Test Purpose and Environment	5134

A.18.2.2.1.2	Test Parameters	5134

A.18.2.2.1.3	Test Requirements	5141

A.18.3	Measurement procedure	5142

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5142

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5142

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5142

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5142

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5142

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5142

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5142

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5142

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5142

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5149

B.1	Conditions for NR RRC_IDLE state mobility	5149

B.1.1	Introduction	5149

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5149

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5150

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5150

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5152

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5152

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5153

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5153

B.2.1	Introduction	5153

B.2.1.1	General	5153

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5153

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5153

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5153

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5154

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5155

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5155

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5155

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5156

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5156

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5156

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5156

B.2.2	Conditions for NR intra-frequency measurements	5157

B.2.3	Conditions for NR inter-frequency measurements	5158

B.2.4	Conditions for NR L1-RSRP reporting	5159

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5159

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5160

B.2.5	Conditions for RRC connection release with redirection to NR	5162

B.2.6	Void	5163

B.2.6.1	Void	5163

B.2.6.2	Void	5163

B.2.7	Conditions for SRS-RSRP measurements	5163

B.2.8	Conditions for NR L1-SINR reporting	5164

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	5164

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	5165

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	5165

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	5165

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	5167

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	5167

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	5168

B.2.9	Conditions for NR intra-frequency measurements under CCA	5169

B.2.10	Conditions for NR inter-frequency measurements under CCA	5169

B.2.11	Conditions for NR L1-RSRP reporting under CCA	5169

B.2.11.1	Conditions for SSB based L1-RSRP reporting	5169

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	5170

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	5171

B.2.14	Conditions for NR PRS-based measurements	5173

B.2.15	Conditions for NR intra-frequency measurements	5174

B.2.16	Conditions for NR inter-frequency measurements	5176

B.2.17	Conditions for NR intra-frequency measurements for satellite access	5178

B.2.18	Conditions for NR inter-frequency measurements for satellite access	5179

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	5179

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	5179

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	5179

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	5180

B.3	RRM Requirements Exceptions	5180

B.3.1	Introduction	5180

B.3.2	Receiver sensitivity relaxation for CA	5180

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	5180

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	5180

B.3.2.2.1	Inter-band carrier aggregation	5180

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	5180

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	5181

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	5181

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	5181

B.3.2.4.1	Intra-band contiguous carrier aggregation	5181

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	5181

B.3.3	Receiver sensitivity relaxation for DC	5181

B.3.3.1	Receiver sensitivity relaxation for EN-DC	5181

B.3.3.2	Receiver sensitivity relaxation for NE-DC	5181

B.3.4	Receiver sensitivity relaxation for SUL	5181

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	5181

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	5182

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	5182

B.4	Conditions for V2X	5182

B.4.1	Test parameters for GNSS signals	5182

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	5182

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	5183

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	5183

Annex C (informative): Change history	5184