3GPP TSG-RAN WG4 #100-e	     	          													 R4-2111735

# Electronic Meeting, Aug 16-27, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS38.101-1 | CR | 0880 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR CatA N65 NS_24 AMPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_band_n65-Core |  |  |  |  |  |  | Date: |  | 2021-07-27 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | AMPR is insufficient and there are RB allocations where AMPR is not defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify 5MHz, Fc=2002.5MHz LCRB > 2.16 to LCRB > 1.98MHz in Region A to cover AMPR for LCRB = 2.16MHz - Modify 5MHz, Fc=2002.5MHz RBend > 3.78 to RBend > 3.6MHz in Region B to cover AMPR for RBend = 3.78 MHz - Modify 10MHz, Fc=1995MHz RBend > 7.56 to RBend > 7.38MHz in Region B to cover AMPR for RBend = 7.56MHz - Modify 10MHz, Fc=1995MHz LCRB > 4.5 to LCRB > 4.32MHz in Region A to cover AMPR for LCRB = 4.5MHz - Modify 10MHz, Fc=2000MHz RBend > 6.84 to RBend > 6.66MHz in Region A to cover AMPR for RBend = 6.84MHz - Modify 10MHz, Fc=2000MHz RBend < 2.88 to RBend < 3.06MHz in Region B to cover AMPR for RBend = 2.88MHz - Modify 15MHz, Fc=1987.5MHz LCRB > 7.02 to LCRB > 6.84MHz in Region A to cover AMPR for LCRB = 7.02MHz - Modify 15MHz, Fc=1987.5MHz RBend > 11.52 to RBend > 11.34 MHz in Region B to cover AMPR for RBend = 11.52 MHz - Modify 15MHz, Fc=1997.5MHz RBend > 9.36 to RBend > 9.18MHz in Region A to cover AMPR for RBend = 9.36MHz - Modify 15MHz, Fc=1997.5MHz RBend < 3.6 to RBend < 3.78MHz in Region B to cover AMPR for RBend = 3.6MHz - Modify 20MHz, Fc=1990MHz RBend > 13.5 to RBend > 13.32MHz in Region A to cover AMPR for RBend = 13.5MHz - Modify 20MHz, Fc=1990MHz RBend < 4.5 to RBend < 4.68MHz in Region B to cover AMPR for RBend = 4.5MHz - Modify 20MHz, Fc=1995MHz RBend > 12.6 to RBend > 12.42MHz in Region A to cover AMPR for RBend = 12.6MHz - Modify 20MHz, Fc=1995MHz RBend < 5.4 to RBend < 5.58MHz in Region B to cover AMPR for RBend = 5.4MHz - Add AMPR A6 to region B for 5MHz Fc=2002.5MHz. - Add AMPR A6 to region B for 10MHz Fc=1995MHz. - Add AMPR A6 to region B for 15MHz Fc=1987.5MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot meet NS requirement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | 38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.2.3.15	A-MPR for NS_24

Table 6.2.3.15-1: A-MPR for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | Fc=1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | Fc=1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz |  | Fc=2002.5 |  |  |  |  | >1.98 |  | A1 |  | >3.6 |  | >1.08 ≤1.98 |  | A2 |  | ≤3.6 |  | ≤1.98 | A3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | ≤1.08 |  | A6 |  |  |  |  |  |
| 10MHz | Fc=1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz |  | Fc=1995 |  |  |  |  | >4.32 |  | A1 |  | >7.38 |  | >1.08 ≤4.32 |  | A2 |  | ≤7.38 |  | ≤4.32 | A3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | ≤1.08 |  | A6 |  |  |  |  |  |
| 10MHz | Fc=2000 | >6.66 |  | A5 | <3.06 |  | A5 | ≥3.06 ≤6.66 | >1.44 | A6 |
| 15MHz |  | Fc=1987.5 |  |  |  |  | >6.84 |  | A1 |  | >11.34 |  | >1.08 ≤6.84 |  | A2 |  | ≤11.34 |  | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | Fc=1997.5 | >9.18 |  | A5 | <3.78 |  | A5 | ≥3.78 ≤9.18 | >1.44 | A6 |
| 20MHz | Fc=1990 | >13.32 |  | A5 | <4.68 |  | A5 | ≥4.68 ≤13.32 | >2.16 | A6 |
| 20MHz | Fc=1995 | >12.42 |  | A5 | <5.58 |  | A5 | ≥5.58 ≤12.42 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.15-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 11 | ≤ 5 |  | ≤ 8.5 | ≤ 20 | ≤ 10 |  |
| CP-OFDM QPSK | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13 | ≤ 6.5 |  | ≤ 8.5 | ≤ 20 | ≤ 12 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |

< end of changes >