3GPP TSG-RAN WG4 Meeting #100-e	R4-2111752

Electronic Meeting, 16th – 27th August 2021

Source:	AT&T

Title:	TP for TR 37.717-21-11 Addition of DC_2-29_n77

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-21-11 to add DC_2A-29A_n77A.

# 2. Text Proposal

---Start of changes---

## 5.X	DC_2-29_n77

### 5.X.1	Configurations for DC

Table 5.X.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-29A_n77A | DC_2A_n77A |

### 5.X.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback mode, DC_2A_n77A. It can be seen that:

-	There are no IMD products produced by Band 2 and n77 that might fall in Rx of band 29.

Although DC_29_n77 is not defined, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

### 5.X.3	∆TIB and ∆RIB values

For DC_2-29_n77, the TIB,c and RIB,c values are given in the tables below. The values are proposed to be reused from the values for CA_n2-n12-n77 with the exception that TIB,c would not be applicable for band 29.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29_n77 | 2 | 0.6 |
|  | n77 | 0.8 |

Table 5.X.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29_n77 | 2 | 0.2 |
|  | 29 | 0.2 |
|  | n77 | 0.5 |

### 5.X.4	Reference sensitivity exceptions

DC_2-29_n77 5th order harmonic mixing impact to band 29 can be derived from DC_12-n77 values.

Table 5.X.4-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 292 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.X.4-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

---End of changes---

# References