3GPP TSG-RAN WG4 Meeting # 100-e	R4-2111765

Electronic Meeting, 16-27 Aug., 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Clarification of BCS4/5 scope

Agenda Item: 			8.28.2

Document for:	Approval

# 1	Introduction

According to a WID [1], BCS4/5 applies to SUL, inter-band and intra-band NR-CA. Practically, these band configuration types may be used together with the other RAT, i.e., LTE or used as NR DC. This contribution addresses to clarify for which band configuration types BCS4/5 can be exploited

# 2	Discussion

Regarding NR DC, BCS4/5 can be used for NR DC since a bandwidth combination set for an NR DC configuration refers to that for a corresponding NR CA configuration as TS38.101-1 mentions that “the bandwidth combination sets for the corresponding NR CA configuration in 5.5A.3,i.e.,dual uplink inter-band carrier aggregation with uplink assigned to two NR bands, are applicable to Dual Connectivity.”

Regarding MR DC, a bandwidth combination set for an NR CA configuration is used for a part of inter band MR DC since a bandwidth combination set for an inter band MR DC is a product of that for the NR CA configuration and that for LTE or LTE CA configuration. This principle, however, does not apply to a bandwidth combination for intra band MR DC. In summary, BCS4/5 can apply to NR CA, NR DC and NR CA part of inter band MR DC. And it does not apply to intra-band MR DC. Note that the last clarification is necessary since inter band MR DC may include intra band MR DC.

Proposal: RAN4 should confirm that BCS4/5 applies to SUL, NR CA, NR DC and SUL and/or NR CA part of inter band MR-DC while it does not apply to intra band MR DC.

# 3	Conclusion

This contribution aimed at clarifying of applicability of BCS4/5. As a result, we propose the following.

Proposal: RAN4 should confirm that BCS4/5 applies to SUL, NR CA, NR DC and SUL and/or NR CA part of inter band MR-DC while it does not apply to intra band MR DC.

# 4	References

[1] RP-202832, New WID: Introduction of bandwidth combination set 4 (BCS4) for NR, Ericsson, T-Mobile USA