3GPP TSG-RAN WG4 Meeting #100-e	R4-2111934

Electronic Meeting, Aug. 16-27, 2021

Title: 	Discussion on UE Rx RF requirements for NTN

Source: 	CATT

Agenda item:	9.13.4.2

Document for:	Discussion

- Introduction

In RAN4#99e meeting, the UE RF requirements was discussed initially in [1], but no consensus has been reached so far. This document will further discuss NTN UE Rx RF requirements.

- Discussion

In last meeting, a way forward for NTN general part was approved [2]. It is agreed that UE NTN FR1 may use similar specification as TS 38.101-1 (with different clauses for NTN). It is proposed NTN UE Rx RF requirements can be defined in specification TS38.101 using clause suffix G.

Proposal 1: NTN UE Rx RF requirements can be defined in specification TS38.101 using clause suffix G.

REFSENS

For NTN UE Rx RF requirements, the most important is to determine reference sensitivity requirement and the reference measurement channel. The reference measurement channel will be defined based on channel coding and TBS in RAN1 specification. From the WID of NTN [3], RAN1 will work on the following enhancement for NTN.

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821):

- Timing relationship enhancements[RAN1,RAN2]

- Enhancements on UL time and frequency synchronization [RAN1,RAN2]

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

In addition, the following topics should be specified if beneficial and needed

- Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2].

- Feeder link switch [RAN2,RAN1]

- Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2]

  - Including signalling of polarization mode

So the physical channel coding and TBS table will reuse Rel-15 design. Since REFSENS testing does not consider HARQ, the RAN1 enhancement does not impact REFSENS requirement and the reference measurement channel design in TS38.101-1.

Proposal 2: The REFSENS and reference measurement channel defined in TS38.101-1 are reused for NTN UE.

Maximum input leval

For maximum input level requirements, it is defined up to -25dBm for TN. This value is determined by the minimum coupling loss between BS and UE. In NTN system,  The MCL between satellite and UE will be very large than TN system. So maximum input level requirements for NTN UE should be re-evaluated. Of course, reusing the current requirement is ok for us.

Blocking

For blocking characteristics requirement, it includes in-band blocking and out-of-band blocking. The in-band blocking interference level may be different for NTN UE. But it should be small than that for TN UE. Reusing the current requirement should be enough.

The out-of-band blocking requirements for TN UE can be reused for NTN UE.

Rx intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal. We think this requirement is meaningless for NTN system.

Spurious emission

The spurious emissions requirement for TN UE can be reused for NTN UE.

Table 2-1 Summary of transmitter requirement for NTN

| Requirements in 38.101 section 7 | Remarks |
| --- | --- |
| 7.3 Reference sensitivity | Reused |
| 7.4 Maximum input level | FFS |
| 7.5 Adjacent channel selectivity | FFS |
| 7.6.2 In-band blocking | FFS |
| 7.6.3 Out-of-band blocking | Reused |
| 7.7 Spurious response | Reused |
| 7.8 Intermodulation characteristics | no need to define |
| 7.9 Spurious emissions | Reused |

- Conclusion

This document discussed Rx RF requirements for NTN UE and concluded the following.

Table 2-1 Summary of transmitter requirement for NTN

| Requirements in 38.101 section 7 | Remarks |
| --- | --- |
| 7.3 Reference sensitivity | Reused |
| 7.4 Maximum input level | FFS |
| 7.5 Adjacent channel selectivity | FFS |
| 7.6.2 In-band blocking | FFS |
| 7.6.3 Out-of-band blocking | Reused |
| 7.7 Spurious response | Reused |
| 7.8 Intermodulation characteristics | no need to define |
| 7.9 Spurious emissions | Reused |

- Reference

[1]	R4-2108465, Discussion on UE RF requirements for NTN, CATT

[2]	R4-2108099, Way Forward on NTN_solutions_Part1, Thales

[3]	RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales