3GPP TSG-RAN WG4 Meeting # 100-e		R4-2111958

Electronic Meeting, August 16-27, 2021

Agenda item:	9.9.4.6

Source: 	CATT

Title: 	Discussion on measurement procedure requirements for HST RRM in FR2

Document for:	Discussion

Introduction

In RAN4#99-e meeting, a way forward on RRM requirements for NR FR2 HST was approved in [1].

In this contribution, we provide the further consideration of measurement procedure requirement for HST RRM in FR2 and give our proposals.

Discussion

In way forward [1], it is listed as below:

- PSS/SSS detection - FFS whether reusing the Rel-16 FR1 HST scaling factor M2 for FR2 HST is acceptable. - FFS reduction of Mpss/sss_sync_w/o_gaps, which is proportional to the number of samples (S) and of receiver sweeping beams (N) - Intra-frequency measurements - FFS whether reusing the Rel-16 FR1 HST scaling factor M2 for FR2 HST is acceptable. - FFS reduction of Mmeas_period_w/o_gaps, which is proportional to the number of samples (S) and of receiver sweeping beams (N)

For PSS/SSS detection and measurement period for FR2 are shown as below:

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps(FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Proposal 1: Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps can be enhanced by reducing RX beams from 8 in PSS/SSS detection and Measurement period. The RX beams can be used the decision of discussion of RX beam number.

Proposal 2: The same M2 in Rel-16 HST can be reused for FR2 HST.

- Restriction on SMTC periodicity in measurement requirements

In way forward [1], it is listed as below:

- FFS: the upper bound of SMTC periodicity, with [40] ms as a basis - FFS: impact on the specification if upper bound of SMTC periodicity is agreed.

We think we can discuss the feasible SMTC in HST scenario such as 40ms is a reasonable value for HST but do not add the restriction in spec.

Proposal 3: 40ms is a reasonable value for HST for upper bound. But do not add the restriction in the specification.

Conclusion

In this contribution, we provide the further consideration of measurement procedure requirement and present our proposals as below:

Proposal 1: Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps can be enhanced by reducing RX beams from 8 in PSS/SSS detection and Measurement period. The RX beams can be used the decision of discussion of RX beam number.

Proposal 2: The same M2 in Rel-16 HST can be reused for FR2 HST.

Proposal 3: 40ms is a reasonable value for HST for upper bound. But do not add the restriction in the specification.

Reference

[1] R4-2108342, WF on FR2 HST RRM requirements, Nokia, Nokia Shanghai Bell