3GPP TSG-RAN WG4 Meeting #100-e                                                           R4-2111973

Electronic Meeting, August 16 - 27, 2021

Source: 	CATT

Title: 	Discussion on the BS RX RF requirements for 52.6-71GHz

Agenda item:	9.16.5.2

Document for:	Approval

- Introduction

In RAN4#99-e, a WF [1] on BS RF RX requirements for 52.6 – 71 GHz was approved.

In this contribution, we provide our views on remaining issues in WF [1].

- Discussion

OTA reference sensitivity level

As per the WF [1], the agreements and the remaining issues concerning reference sensitivity are shown as below:

- Sensitivity levels - Use the same principle for derivation of sensitivity level requirements as for current FR2 (i.e. declaration of sensitivity level), - Further check if sensitivity declarations based on 100 MHz reference measurement channel can be the basis instead of 50MHz (as in existing FR2), - Further check whether the allowed range of sensitivity values to be declared needs an update.

EISREFSENS_50M is a declared parameter, and EISREFSENS_50M itself is not a requirement. Although, 50MHz bandwidth/60kHz SCS is not supported for 52.6-71GHz, EISREFSENS_50M also can be used for sensitivity declaration for 52.6-71GHz. There is no need to define the 100MHz CBW/120kHz SCS as a new declared parameter.

Proposal 1: Re-use FR2 EISREFSENS_50M for reference sensitivity declaration for 52.6-71GHz.

The allowed declared range of EISREFSENS_50M is calculated using the following formula.

For WA BS for 24.25 – 33.4 GHz,

Max. EISREFSENS_50M = -174dBm +10*log10(66*12*60*1000)+10+2+(-1.1)-10=-96.33 =-96 dBm

Min. EISREFSENS_50M  = -174dBm +10*log10(66*12*60*1000)+10+2+(-1.1)-33= -119.3312=-119 dBm

Where, for 24.25 – 33.4 GHz, the NF is assumed to be 10dB, and minimum and maximum antenna gain is assumed to be 10dBi and 33dBi respectively.

For WA BS for 37 – 52.6 GHz,

Max. EISREFSENS_50M   = -174dBm +10*log10(66*12*60*1000)+12+2+(-1.1)-12=-96.33 =-96 dBm

Min. EISREFSENS_50M   = -174dBm +10*log10(66*12*60*1000)+12+2+(-1.1)-35= -119.3312=-119 dBm

Where, for 37 – 52.6 GHz, the NF is assumed to be 12dB, and minimum and maximum antenna gain is assumed to be 12dBi and 35dBi respectively.

The declared range of EISREFSENS_50M for 24.25 – 33.4 GHz is same with the declared range of EISREFSENS_50M for 37 – 52.6 GHz. So for 52.6-71GHz, to maintain the same OTA REFSENS value, the the declared range of EISREFSENS_50M can be reused for 52.6-71GHz.

Proposal 2: Re-use the FR2 EISREFSENS_50M declared range for 52.6-71GHz.

FRC

As per the WF [1], the agreements and the remaining issues concerning FRC are shown as below:

- FRCs - FRCs for 52.6 GHz – 71 GHz include 100 MHz/120 kHz, 400 MHz/480 kHz, 400 MHz/960 MHz are to be defined - Further check on the possibility to reuse parameters from FR2.

The existing G-FR2-A1-3 and G-FR2-A1-2 can be used for 100MHz/120kHz for sensitivity and ICS respectively. There is need to define G-FR2-A1-6 and G-FR2-A1-7 for sensitivity for 400MHz/480kHz and 400MHz/960kHz respectively. And there is also need to define G-FR2-A1-8 and G-FR2-A1-9 for ICS for 400MHz/480kHz and 400MHz/960kHz respectively.  The TBS (bits) for new FRCs are shown in the following Table 2-1, and the TBS (bits) can be further discussed once the NRB for 400MHz/480kHz and 400MHz/960kHz is decided.

Table 2-1 TBS for new FRCs for 52.6-71GHz

| FRC | CBW(MHz) | SCS(kHz) | TBS(bits) |
| --- | --- | --- | --- |
| G-FR2-A1-6 | 400([66PRBs]) | 480 | [5632] |
| G-FR2-A1-7 | 400([32PRBs]) | 960 | [2792] |
| G-FR2-A1-8 | [33PRBS] | 480 | [2856] |
| G-FR1-A1-9 | [16PRBs] | 960 | [1416] |

Proposal 3: Define G-FR2-A1-6 and G-FR2-A1-7 for sensitivity for 400MHz/480kHz and 400MHz/960kHz respectively.

Proposal 4: Define G-FR2-A1-8 and G-FR2-A1-9 for ICS for 400MHz/480kHz and 400MHz/960kHz respectively.

The SNR value for each FRC shall be specified by link level simulation, the Simulation assumptions in R4-1801031[2] for FR2 FRC simulation can be as starting point for 52.6-71GHz. In this contribution, such as the MCS index 4 and target coding rate = 308/1024, MMSE receiver, DMRS pattern: 1+1 (2, 11), no data in DMRS symbol, PTRS configuration: KPTRS=2, LPTRS=1, No phase noise simulation, no data in PTRS REs for FR2 FRC can be reused for 52.6-71GHz.

Proposal 5: The Simulation assumptions in R4-1801031[2] for FR2 FRC simulation can be as starting point for 52.6-71GHz.

- Conclusion

This contribution provides analysis on BS RX RF requirements for 52.6-71GHz. The following proposals and observations are concluded as follows:

Proposal 1: Re-use FR2 EISREFSENS_50M for reference sensitivity declaration for 52.6-71GHz.

Proposal 2: Re-use the FR2 EISREFSENS_50M declared range for 52.6-71GHz.

Proposal 3: Define G-FR2-A1-6 and G-FR2-A1-7 for sensitivity for 400MHz/480kHz and 400MHz/960kHz respectively.

Proposal 4: Define G-FR2-A1-8 and G-FR2-A1-9 for ICS for 400MHz/480kHz and 400MHz/960kHz respectively.

Proposal 5: The Simulation assumptions in R4-1801031[2] for FR2 FRC simulation can be as starting point for 52.6-71GHz.

- References

- R4-2108639, WF on BS RX RF requirements for 52.6 – 71 GHz, Huawei, RAN4#99-e

- R4-1801031,WF on NR BS RF FRCs, Ericsson, Huawei, ZTE, CATT, Nokia, Nokia Shanghai Bell, RAN4#85