3GPP TSG-RAN WG4 Meeting # 100-e 		                       R4-2112011

Electronic Meeting, August 16th - 27th, 2021

Title: 	Rx requirements for NTN gNB

Source: 	CATT

Agenda item:	9.13.3.2

Document for:	Discussion

- Introduction

It has been agreed to specify NTN network side requirements by taking 38.104 as the starting point [1, 2]. This contribution will discuss the applicability of TN BS Rx RF requirements for NTN gNB based on Type 1-H and type 1-C.

- Discussion

  - NF and REFSENS

As analyzed in our paper [3], we propose to introduce 3 NTN BS classes due to different output power. But we propose to use 5dB noise figure as starting point for all 3 NTN BS classes. NTN-Gateway should have similar or small value than WA BS. It is FFS whether additional NF for NTN-payload should be considered.

Table 2-1 REFSENS for NTN BS class A/B/C

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5/10/15/20 | 15/30/60 | TBD | TBD |

  - Dynamic range

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the antenna connector for BS type 1-C or TAB connector for BS type 1-H inside the received BS channel bandwidth. The interfering signal level can be determined by system level simulation under NTN operating scenario.

  - In-band selectivity and blocking

This requirement usually depends on inter-operator co-existence scenario. It should be determined based on NTN-NTN co-existence study.

  - Out-of-band blocking

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

It needs further check whether such scenario exists for NTN BS.

  - Receiver spurious emissions

How to define this requirement need to be considered in line with Tx spurious emission requirement.

  - Receiver intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

We think this requirement might be meaningless for NTN BS considering its operating scenario.

  - In-channel selectivity

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an interfering signal received at a larger power spectral density.

The same requirement can be reused for NTN BS.

- Conclusion

This paper gave some initial consideration on NTN BS Rx RF requirements.

- Reference

[1]	R4-2108099, WF on [312] NTN_Solutions_Part1, Thales

[2]	R4-2108100, WF on NTN RF Aspect, CATT

[3]	R4-2112009, Discussion on NTN gNB type/class, CATT