3GPP TSG-RAN WG4 Meeting #100-e	R4-2112017

# Electronic Meeting, 16th to 27th Aug, 2021

Agenda Item:	8.6.1

Source: 	MediaTek Inc.

Title: 	Further discussion on MSD due to IMD5 for CA_n41C-n66A

Document for:	Approval

- 1. Introduction

In last RAN4 meeting, it has been discussed MSD due to IMDs for band combinations. However there’s large difference on MSD due to IMD5 for CA_n41C-n66A for the contributions [2][3] in last meeting. Following the WF[1], the contribution provide our analysis and view for the case.

- 2. Discussion

NR UL CCA configurations has started to be introduced in Rel-17 and MSD due to IMDs were discussed in previous meetings. One of the undetermined issue was MSD due to IMD5 for CA_n41C-n66A since there was 10dB difference between analyses of company contributions. We would like to provide analyses and our view here.

Case 1 Separated antenna

The parameters assumption are listed in table 1. It is assumed the spur level arrive victim PRX and DRX are equal. The analysis result is listed in table 2.

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n41 filter rejection at n66 DL range | 30 | dB |
| IMD5 spur level at n41 PA output | -30 | dBm/MHz |
| Front-end loss | 4 | dB |
| PA RXBN noise | -125 | dBm/Hz |
| Thermal noise at n66 RX ANT port | -166 | dBm/Hz |
| Transceiver effective phase noise | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n41 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n41 PA output IMD5 spur level at n66 freq | -23 | -23 | dBm |
| n41 TX IMD5 spur level at n66 PRX/DRX LNA port | -67 | -67 | dBm |
| n41 noise power at n66 DUP- COM port | -99 | NA | dBm/5MHz |
| n41 noise power at n66 DRX ANT port | NA | -99 | dBm/5MHz |
| Thermal noise at RX ant port | -166 | -166 | dBm/Hz |
| Total noise level refers to receiver LNA port | -67 | -67 | dBm |
| Combined |  |  |  |
| Combined B66 REFSENS (5MHz BW) | -67 |  | dBm |
| MSD (5MHz BW) | 32.5 |  | dB |

Table 6.1.x.5-4 Link analysis for n66 5MHz REFSENS calculation

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41C-n66A | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | [32.5] | FDD | IMD5 |

Case 2 Shared antenna

The primary path is using a diplexer for CA_n41C-n66A implementation. According to the data provided by our vendor pool, the isolation of the diplexer is around 10dB (typ 15dB and worst 8dB). Thus there’s only small MSD difference than separated antenna architecture in PRX while it still provides better MSD with the advantage of DRX. The calculated MSD is ~25dB.

Considering previous RAN4 work only specify worst case scenario for MSD and specs need to accommodate different UE implementations, we would propose

Proposal 1: For the CA_n41C-n66A MSD relaxation, we propose worst case scenario as below:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41C-n66A | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | [32.5] | FDD | IMD5 |

Conclusion

Proposal 1: For the CA_n41C-n66A MSD relaxation, we propose worst case scenario as below:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41C-n66A | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | [32.5] | FDD | IMD5 |

Reference:

- R4-2107802	WF on MSD due to IMD of intra-band UL CA UL configurations, RAN4#99-e

- R4-2107625 Revision of R4-2111476 MSD due to IMD from ULCA, Qualcomm Incorporated, R4#99-e

- R4-2111016 MSD Due to NR Intra-band ULCA IMD within Inter-band Combinations, Skyworks Solutions Inc., R4#99-e