3GPP RAN WG4 Meeting #100e		R4-2112033

Electronic Meeting, Aug 16-27, 2021

Agenda item:	9.16.4.1

Source:	Murata Manufacturing Co., Ltd.

Title:	Views on UE antenna array size and link budget at 70 GHz

Document for:	Discussion

1	Introduction

In RAN4#99-e meeting a power class parameter framework for 52.6-71 GHz UEs was agreed to along with an initial set of UE types to be considered [1]. In discussing the TX power class parameters of minimum peak EIRP and maximum TRP, it is thought beneficial to discuss antenna array sizes. This contribution shows our views on UE array size and link budget for 70 GHz Handheld UEs

2	Discussion

There are several deployment scenarios and ISDs in TR38.807. In each scenario, maximum communication range between UE and BS will be half of the ISD. From this we can calculate pathloss at 70GHz by equations in TR38.901. Table 1 shows the result of this calculation.

Table 1: maximum communication range and pathloss

| scenario | ISD [m] | max range [m] | Pathloss [dB] | Calculation condition |
| --- | --- | --- | --- | --- |
| Dense Urban | 200 | 100 | 128.7 | UMa NLOS |
| Urban Micro | 100 | 50 | 121.7 | UMi NLOS |
| Urban Macro | 500 | 250 | 117.7 | UMa LOS |

Observation 1:	Pathloss will be 128.7 dB in Dense Urban, 121.7 dB in Urban Micro, 117.7 dB in Urban Macro.

In order to estimate feasible antenna array size, we made uplink and downlink link budgets. Table 2 and Table 3 show sample link budgets.

Table 2: Link budget of uplink

| Common | Frequency | 70.0 | 70.0 | 70.0 | 70.0 | 70.0 | 70.0 | GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bandwidth | 400 | 400 | 400 | 400 | 400 | 400 | MHz |
|  | Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |  |
| UE | # of antennas in an array | 4 | 4 | 8 | 8 | 16 | 16 |  |
|  | Pout per element | 7.0 | 7.0 | 7.0 | 7.0 | 7.0 | 7.0 | dBm |
|  | Total conducted power per polarization | 13.0 | 13.0 | 16.0 | 16.0 | 19.0 | 19.0 | dBm |
|  | Average antenna element gain | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | dBi |
|  | Antenna roll-off loss versus frequency | -2.0 | -2.0 | -2.0 | -2.0 | -2.0 | -2.0 | dB |
|  | Realized antenna array gain | 8.0 | 8.0 | 11.0 | 11.0 | 14.0 | 14.0 | dBi |
|  | Polarization gain | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | dB |
|  | Total implementation loss | -6.5 | -6.5 | -6.5 | -6.5 | -6.5 | -6.5 | dB |
|  | Peak EIRP | 17.3 | 17.3 | 23.4 | 23.4 | 29.4 | 29.4 | dBm |
| BS | # of antennas in an array | 128 | 1024 | 128 | 1024 | 128 | 1024 |  |
|  | Average antenna element gain | 6.0 | 6.0 | 6.0 | 6.0 | 6.0 | 6.0 | dBi |
|  | Antenna roll-off loss versus frequency | -2.0 | -2.0 | -2.0 | -2.0 | -2.0 | -2.0 | dB |
|  | Realized antenna array gain | 25.1 | 34.1 | 25.1 | 34.1 | 25.1 | 34.1 | dBi |
|  | Total implementation loss | -6.5 | -6.5 | -6.5 | -6.5 | -6.5 | -6.5 | dB |
|  | Thermal noise | -87.8 | -87.8 | -87.8 | -87.8 | -87.8 | -87.8 | dBm |
|  | Rx NF | 10.0 | 10.0 | 10.0 | 10.0 | 10.0 | 10.0 | dB |
|  | Required C/N | -1.0 | -1.0 | -1.0 | -1.0 | -1.0 | -1.0 | dB |
|  | EIS | -97.4 | -106.4 | -97.4 | -106.4 | -97.4 | -106.4 | dBm |
| Peak EIRP - EIS |  | 114.8 | 123.8 | 120.8 | 129.8 | 126.8 | 135.8 | dB |

Table 3: Link budget of downlink

| Common | Frequency | 70.0 | 70.0 | 70.0 | 70.0 | 70.0 | 70.0 | GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bandwidth | 400 | 400 | 400 | 400 | 400 | 400 | MHz |
|  | Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |  |
| BS | # of antennas in an array | 128 | 1024 | 128 | 1024 | 128 | 1024 |  |
|  | Pout per element | 7.0 | 7.0 | 7.0 | 7.0 | 7.0 | 7.0 | dBm |
|  | Total conducted power per polarization | 28.1 | 37.1 | 28.1 | 37.1 | 28.1 | 37.1 | dBm |
|  | Average antenna element gain | 6.0 | 6.0 | 6.0 | 6.0 | 6.0 | 6.0 | dBi |
|  | Antenna roll-off loss versus frequency | -2.0 | -2.0 | -2.0 | -2.0 | -2.0 | -2.0 | dB |
|  | Realized antenna array gain | 25.1 | 34.1 | 25.1 | 34.1 | 25.1 | 34.1 | dBi |
|  | Polarization gain | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | dB |
|  | Total implementation loss | -6.5 | -6.5 | -6.5 | -6.5 | -6.5 | -6.5 | dB |
|  | Peak EIRP | 46.6 | 64.7 | 46.6 | 64.7 | 46.6 | 64.7 | dBm |
| UE | # of antennas in an array | 4 | 4 | 8 | 8 | 16 | 16 |  |
|  | Average antenna element gain | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | dBi |
|  | Antenna roll-off loss versus frequency | -2.0 | -2.0 | -2.0 | -2.0 | -2.0 | -2.0 | dB |
|  | Realized antenna array gain | 8.0 | 8.0 | 11.0 | 11.0 | 14.0 | 14.0 | dBi |
|  | Total implementation loss | -6.5 | -6.5 | -6.5 | -6.5 | -6.5 | -6.5 | dB |
|  | Thermal noise | -87.8 | -87.8 | -87.8 | -87.8 | -87.8 | -87.8 | dBm |
|  | Rx NF | 14.0 | 14.0 | 14.0 | 14.0 | 14.0 | 14.0 | dB |
|  | Required C/N | -1.0 | -1.0 | -1.0 | -1.0 | -1.0 | -1.0 | dB |
|  | EIS | -76.4 | -76.4 | -79.4 | -79.4 | -82.4 | -82.4 | dBm |
| Peak EIRP - EIS |  | 123.0 | 141.1 | 126.0 | 144.1 | 129.0 | 147.1 | dB |

We made some other link budgets. Table 4 shows all values of “Peak EIRP – EIS”.

Table 4: “Peak EIRP – EIS” [dB] values with some antenna array sizes

|  |  | BS antenna array size |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Uplink |  |  | Downlink |  |  |
|  |  | BS=8x16 | BS=16x16 | BS=32x32 | BS=8x16 | BS=16x16 | BS=32x32 |
| UE antennas | 4 | 114.8 | 117.8 | 123.8 | 123.0 | 129.0 | 141.1 |
|  | 8 | 120.8 | 123.8 | 129.8 | 126.0 | 132.0 | 144.1 |
|  | 16 | 126.8 | 129.8 | 135.8 | 129.0 | 135.0 | 147.1 |

Comparing Table 1 and Table 4, we can know the required number of UE antenna elements to support the communication link range defined by the ISD. Table 5 shows the summary of required number of UE antenna elements in order to communicate the half-ISD range.

Table 5: Required number of UE antenna elements for each scenario

| Scenario | ISD [m] | max range [m] | BS antenna array size |  |  | reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 8x16 | 16x16 | 32x32 | Max pathloss [dB] | Pathloss calculation condition |
| Dense Urban | 200 | 100 | Over 16 | 16 | 8 | 128.7 | UMa NLOS |
| Urban Micro | 100 | 50 | 16 | 8 | 4 | 121.7 | UMi NLOS |
| Urban Macro | 500 | 250 | 8 | 4 | 4 | 117.7 | UMa LOS |

If we use 4 antenna elements in the UE, we need base stations with 32x32 antenna elements, but only urban micro and macro will be supported, not dense urban. Using 8 antenna elements for handheld UE would assure connectivity for all three scenarios.

On the other hand, increasing the number of antenna elements (i.e. the number of PA) makes power consumption higher, and we should investigate of its influence. However, we believe connectivity is important as well as power consumption. Recommend considering 8 antenna elements for the handheld UE as baseline architecture.

Observation 2:	If we use 4 antenna elements in UE, we need base stations with 32x32 antenna elements to support urban micro and urban macro – dense urban will not be supported.

Proposal 1:	Recommend considering 8 antenna elements for the handheld UE as baseline architecture.

3	Conclusions

This contribution provides our view on the antenna array size for handheld UE:

Observation 1:	Pathloss will be 128.7 dB in Dense Urban, 121.7 dB in Urban Micro, 117.7 dB in Urban Macro.

Observation 2:	If we use 4 antenna elements in UE, we need base stations with 32x32 antenna elements to support urban micro and urban macro – dense urban will not be supported.

Proposal 1:	Recommend considering 8 antenna elements for the handheld UE as baseline architecture.

4	References

R4-2107973, “WF on 60 GHz UE Tx requirements,” Intel.