3GPP TSG-RAN WG4 Meeting #100-e	R4-2112147

Electronic Meeting, August 16-27, 2021

Source:	China Telecom

Title:	Initial simulation results and discussion on PUSCH FR1 256QAM demodulation requirements

Agenda Item:	9.12.3.1

Document for:	Discussion

- Introduction

In the RAN4#99-e meeting, we discussed the test parameter, simulation assumptions and applicability for the PUSCH FR1 256QAM demodulation test cases. We approved the WF in [1].

This paper we provide initial simulation results for PUSCH 256QAM demodulation for 10MHz 15kHz SCS and 40MHz 30kHz SCS, and give our views on the remain open issues.

- Initial Simulation Results

Based on the agreed test parameters in [1], we provide initial simulation results for PUSCH 256QAM demodulation for 10MHz 15kHz SCS and 40MHz 30kHz SCS as follows:

Table 1 Ideal and impairment SNR @ 70% max TP (1T2R)

|  |  |  | Ideal | Impairment |
| --- | --- | --- | --- | --- |
| PUSCH Mapping Type A | 15kHz, 10MHz | MCS20 | 17.15 | 19.15 |
|  |  | MCS21 | 18.13 | 20.13 |
|  |  | MCS22 | 18.70 | 20.70 |
|  |  | MCS24 | 20.74 | 22.74 |
|  | 30kHz, 40MHz | MCS20 | 17.36 | 19.36 |
|  |  | MCS21 | 18.23 | 20.23 |
|  |  | MCS22 | 19.07 | 21.07 |
|  |  | MCS24 | 20.76 | 22.76 |

Table 2 Ideal and impairment SNR @ 70% max TP (1T4R)

|  |  |  | Ideal | Impairment |
| --- | --- | --- | --- | --- |
| PUSCH Mapping Type A | 15kHz, 10MHz | MCS20 | 13.23 | 15.23 |
|  |  | MCS21 | 14.03 | 16.03 |
|  |  | MCS22 | 14.90 | 16.90 |
|  |  | MCS24 | 16.88 | 18.88 |
|  | 30kHz, 40MHz | MCS20 | 13.29 | 15.29 |
|  |  | MCS21 | 14.13 | 16.13 |
|  |  | MCS22 | 15.01 | 17.01 |
|  |  | MCS24 | 17.07 | 19.07 |

Table 3 Ideal and impairment SNR @ 70% max TP (1T8R)

|  |  |  | Ideal | Impairment |
| --- | --- | --- | --- | --- |
| PUSCH Mapping Type A | 15kHz, 10MHz | MCS20 | 9.81 | 11.81 |
|  |  | MCS21 | 10.62 | 12.62 |
|  |  | MCS22 | 11.30 | 13.30 |
|  |  | MCS24 | 13.42 | 15.42 |
|  | 30kHz, 40MHz | MCS20 | 9.70 | 11.70 |
|  |  | MCS21 | 10.60 | 12.60 |
|  |  | MCS22 | 11.54 | 13.54 |
|  |  | MCS24 | 13.51 | 15.51 |

- Discussion

On MCS Index

WF Status in [1]:

- MCS

  - Option 1: MCS 22

  - Option 2: MCS 24

  - Option 3: MCS 20 or MCS 21 if there is testability issue for OTA test

Since we are considering both conducted and radiated performance requirements for 2Rx, and there is maximum SNR limit for OTA test requirement definition, based on our simulation results in Table 1, it seems MCS24 is not suitable for requirement definition for OTA test.

Therefore, we are fine with MCS22 or MCS21 for PUSCH 256QAM requirement definition.

Proposal 1: We are fine with MCS22 or MCS21 for PUSCH 256QAM requirement definition.

Tx EVM

WF Status in [1]:

- Tx EVM

  - Option 1: Consider 3.5%Tx EVM modelling for alignment results

  - Option 2: Consider 3.5% Tx EVM impact in the impairment results

    - Option 2a: add a certain margin on top of the averaged impairment results

    - Option 2b: consider it in the impaired results submitted by companies

  - Option 3: Not consider 3.5% Tx EVM impact if the target SNR is 20dB or less

For Tx EVM, to align with Rel-15 PUSCH demodulation requirements defined for other modulation orders, we support not to consider 3.5% Tx EVM impact, or only consider the Tx EVM impact in the impairment results submitted by companies.

Proposal 2: Not to consider 3.5% Tx EVM impact, or only consider the Tx EVM impact in the impairment results submitted by companies.

Requirement and Applicability Rule for SCS and CBW

WF Status in [1]:

- SCS and Bandwidth combination

  - 15kHz SCS:

    - Option 1: 5MHz and 10MHz

    - Option 2: 5MHz, 10MHz and 20MHz

  - 30kHz SCS:

    - Option 1: 10MHz and 40MHz

    - Option 2: 10MHz, 20MHz, 40MHz and 100MHz

With the agreed applicability rule for different CBW which does not increase the test case number, we prefer to reuse the same CBW configurations for Rel-15 PUSCH demodulation tests, i.e., option 2 for both 15kHz SCS and 30kHz SCS. Some typical CBW configurations are missing in option 1.

Proposal 3: Reuse the same CBW configurations for Rel-15 PUSCH demodulation tests, i.e., option 2 for both 15kHz SCS and 30kHz SCS.

- Conclusion

In this paper, our views on the test parameter configuration and applicability rules for PUSCH FR1 256QAM demodulation requirements are given:

Proposal 1: We are fine with MCS22 or MCS21 for PUSCH 256QAM requirement definition.

Proposal 2: Not to consider 3.5% Tx EVM impact, or only consider the Tx EVM impact in the impairment results submitted by companies.

Proposal 3: Reuse the same CBW configurations for Rel-15 PUSCH demodulation tests, i.e., option 2 for both 15kHz SCS and 30kHz SCS.

- Reference

R4-2108667, Way forward on PUSCH 256QAM performance requirements, Huawei, HiSilicon, RAN4#99-e, May 2021.