3GPP TSG-RAN WG4 Meeting # 100-e      		R4-2112208

Electronic Meeting, August. 16-27, 2021

Agenda Item:	9.12.2.1.1

Source: 	CMCC

Title: 	Simulation results for inter-cell interference suppressing

Document for:	Discussion

- Introduction

In RAN4#99-e meeting, we discussed the test parameters and the interference model for inter-cell interference MMSE-IRC receiver, a way forward was agreed in [1]. In this contribution, we provide our simulation results for inter-cell interference suppressing.

- Simulation Results

Table 1. Simulation results

| Case 1 | 2T2R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR 3.1 dB in case of 1 interference cell | SINR=4.4 |
| --- | --- | --- |
| Case 2 | 2T2R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR 3.1 dB in case of 1 interference cell | SINR=-2 |
| Case 3 | 2T4R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR 3.1 dB in case of 1 interference cell | SINR=-0.9 |
| Case 4 | 2T4R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR 3.1 dB in case of 1 interference cell | SINR=-6.9 |
| Case 5 | 2T4R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SINR=0.04 |
| Case 6 | 2T4R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SINR=-7.2 |

Observation 1: For 2T4R MCS 4 configuration, the SINR is lower than -6dB.

- Conclusion

In this contribution, we provide our simulation results for inter-cell interference suppressing. The observations are:

Observation 1: For 2T4R MCS 4 configuration, the SINR is lower than -6dB.

- Reference

[1] R4-2108664, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel Corporation.

[2] R4-2106116, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel Corporation.

[3] RP-202000, New WID: Further enhancement on NR demodulation performance, China telecom.

- Annex

Table 2. Simulation assumptions

| Network type | FDD sync |
| --- | --- |
| Antenna configuration | 2 Tx as baseline for serving cell and interfering cells UE with 2 and 4 RX ULA low correlation as baseline |
| SCS | FDD 15kHz TDD 30kHz |
| Channel bandwidth | FDD: 10MHz，[40MHz] TDD: 40MHz，[100MHz] |
| TDD DL/UL configuration for 30kHz SCS | 7D1S2U(S=6D+4G+4U) |
| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID i for interferer i (where i ≥ 1, maximum number of i is FFS) |
| PDCCH and PDSCH allocation in time domain | Use symbols #0 and #1 of each slot for PDCCH PDSCH mapping type A, Start symbol 2, Duration 12 |
| PDSCH allocation in frequency domain | Full PRB |
| SSB configuration | All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources |
| TRS/CSI-RS configuration | TRS/CSI-RS colliding with TRS/CSI-RS interference |
| DMRS configuration | For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interferer |
| Propagation condition | TDLA30-10 |
| Transmission rank | rank 1 |
| MCS | Use MCS 4 (QPSK, CR=0.3) and MCS 13 (16QAM, CR=0.5) for initial simulation purpose |
| Precoding model | Random precoder with Type I SP codebook |
| PRB bundle size | Set PRB bundle size as 2 for target PDSCH |
| HARQ process number | 4 for FDD 8 for TDD |
| Performance measurement point | SINR at 70% TP |
| DIP values for Homogeneous deployment assumptions initial simulations | DIP values for Homogeneous deployment assumptions for initial simulations - Consider DIP1/2 = -1.73/-8.66 dB (INR1/2 = 5.43/-1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell) as baseline for initial link level analysis for Synchronous network |
| INR values for HetNet deployment assumptions for initial simulations (in case HetNet is agreed) | INRs 11.39 and 5.45 dB (DIPs -1.23 and -7.16 dB) |
| Transmission rank of interfering PDSCH | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| Precoding of interfering PDSCH | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |
| Modulation order of interfering PDSCH | Use 16QAM randomly modulated symbols for initial simulation assumption |
| Number of explicitly modeled interference cells | 1 and 2 interference cells |