3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112246

Electronic Meeting, 16-27 Aug., 2021

Agenda Item: 	8.28.1

Source: 			Qualcomm Incorporated

Title: 				Proposals on BCS4 and BCS5

Document for: 	Approval

# Introduction

The WF on BCS4 excluding MSD was approved in [1] with the following agreements.

- BCS5 is the same as BCS4 except that there is new signalling introduced for BCS5. How to design the new signalling depends on RAN2’s input.

- BCS4 is optional for a given combination, allocated as requested.

- Option 3 which covers inter-band and intra-band to represent BCS4

- Continue discussions for how to represent BCS4 for intra-band NR-CA at RAN4#100.

In this paper, we provide our views on how to apply BCS4 and BCS5.

# Discussion

  - How to request and represent BCS4 and BCS5

According to the agreements in [1], BCS5 is the same as BCS4 only except that a new signalling will be introduced for BCS5. And the request for BCS4 is optional for a given combination. When BCS4 and BCS5 were discussed, RAN4 has the common understanding that both BCS4 and BCS5 could reduce the workload when specifying band combinations. From RAN4 perspective, there should be no difference to request and represent BCS4/BCS5. We therefore propose to treat BCS4 and BCS5 equally when requesting the new band combination, i.e., if BCS4 is requested, then it is equivalent to get BCS5 requested and vice versa.

Proposal 1: If BCS4 is requested for a band combination then it is equivalent to get BCS5 requested, and vice versa.

After BCS4/BCS5 is completed, BCS4/5 can be represented in the same table.

Proposal 2: RAN4 to represent BCS4 and BCS5 with the same manner as Table 1.

Table 1 NR CA configurations and bandwidth combination sets defined for inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_nXA-nYA | CA_nXA-nYA | nX | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | nY | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | See nX and nY channel bandwidths in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 4/5 |
| CA_nXA-nYC | CA_nXA-nYA | nX | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | nY | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | See nX channel bandwidths in Table 5.3.5-1 and nYC channel bandwidths in Table 5.5A.1-1. |  |  |  |  |  |  |  |  |  |  |  |  |  | 4/5 |

  - How to indicate BCS4/BCS5 for intra-band NR CA

According to [2], inter-band NR CA, SUL and intra-band NR CA are in the scope. For inter-band CA and SUL, option 3 in [1] was agreed. But for intra-band NR CA, how to indicate BCS4/BCS5 was not decided since the aggregated bandwidths need to be considered.

For BCS4, UE supports all the channel bandwidths permutations below maximum bandwidth. So there is no need to reflect all supported channel bandwidths in the tables. The key information would be maximum aggregated bandwidth which is restricted by CA bandwidths class. For BCS5, it is the same as BCS4 only except additional minimum channel bandwidth will be indicated per CC. With that, we propose to use the following Table 1 and Table 2 to represent intra-band contiguous and non-contiguous respectively.

Proposal 3: RAN4 to use Table 2 and Table 3 to represent intra-band contiguous NR CA and intra-band non-contiguous NR CA respectively.

Table 2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_nXC | CA_nXC | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | see nX channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  |  | 200 | 4/5 |
| NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. |  |  |  |  |  |  |  |  |

Table 3: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) |
| CA_nX(2A) | CA_nX(2A) | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 |  |  | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 200 | 1 |
|  |  | see nX channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 200 | 4/5 |
| NOTE 1: Void. NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150 MHz), and the minimum frequency gaps in between NR non-contiguous component carriers. |  |  |  |  |  |  |  |

# Conclusion

In this paper, we provide our views on how to apply BCS4 and BCS5. The following proposals were made based on the analysis.

Proposal 1: If BCS4 is requested for a band combination then it is equivalent to get BCS5 requested, and vice versa.

Proposal 2: RAN4 to represent BCS4 and BCS5 with the same manner as Table 1.

Proposal 3: RAN4 to use Table 2 and Table 3 to represent intra-band contiguous NR CA and intra-band non-contiguous NR CA respectively.

# References

[1]		R4-2107821	Way forward for general questions for BCS4 (excluding MSD), T-Mobile USA, Ericsson

[2]		RP-202832	New WID: Introduction of bandwidth combination set 4 (BCS4) for NR, Ericsson, T-Mobile USA