3GPP TSG-RAN4 #100-e	R4-2112254

Electronic Meeting, 16th August – 27th August 2021

Title:	FFT window starting point values for EVM measurements for transient period capability

Source:			Anritsu Limited

Agenda Item:			6.1.9.2.1.

Document for:			Approval

# 1.	Introduction

The transient period capability which allows the UE to report its transient period for ON to ON cases for power change and frequency hopping was approved. With such feature, BS will be able to consider the effective supported transient time for each individual UE, and not the maximum specified one in TS 38.101-1 (10µs) and TS 38.101-2 (5µs).

An EVM test allows to determine which transient period (tp) is supported by UE among the non-mandatory values of 2, 4 and 7 µs in FR1. To determine the EVM, the sample timing difference Δt ̃ between the FFT processing window in relation to nominal timing of the ideal signal needs to be calculated.

The calculation of Δt ̃ for one of the four FFT processing windows EVMh_tp involves the cyclic prefix (CP) which is a floating-point value when expressed in µs as tpstart is. As shown in [1], due to the rounding of CP in µs, there may be significant differences between TE vendors based on how are calculated Δt ̃ values for EVMh_tp, and so impacting the measured EVM.

It was then recommended in contribution [1] and accepted during RAN4 #99-e to put precise “hard-coded” Δt ̃values expressed in number of samples using the sampling unit Tc in the Annex F of [2]. The calculation of those tabulated Δt ̃ values being made with accurate CP values and using formulas from Annex F of [2].

# 2.	Discussion

8 “hard-coded” Δt ̃ values in total for EVMl_tp and EVMh_tp explicitly added in the annex F.4 of the 38.101-1. [2], those 8 Δt ̃ values corresponding to the {SCS, tp} cases used in the conformance test as described in Table 6.4.2.1a-1 of the 38.101-1 [2].

The current tpstart values used in [2] are shown in the extract below. In the annex A are shown the corresponding 8 “hard-coded” Δt ̃ values.

Table 6.3.3.1-1 tpstart values tp (s); tpstart (s). 2; [-0.5]. 4; [-1]. 7; [-2]. NOTE 1: Negative values mean that the transient period starts before the symbol boundary. […] For the EVM calculation on the symbols with a transient period when the UE signals a transient period capability (tp) of 2, 4 or 7usec, Δt is is given below. - calculate EVMl_tp with Δt set totp+tpstartTc+1, where is 1/Tc the sampling rate - calculate EVMh_tp with Δt set toCP+tpstartTc-1, where 1/Tc is the sampling rate and the CP is the cyclic prefix of the symbol on which EVM is calculated (e.g. long CP for the first symbol of the slot) in seconds

Proposal 1: Add Δt ̃values from annex A of the present contribution in annex F.4 of 38.101-1 corresponding to tpstart values {-0.5, -1, -2}.

Based on [3] and the WF [4], two of the tpstart values may change and would lead to other Δt ̃ values as shown in annex B.

Table 1 – tpstart values

| tp (s) | tpstart (s) |
| --- | --- |
| 2 | [-0.7] |
| 4 | [-1] |
| 7 | [-2.7] |
| NOTE 1: Negative values mean that the transient period starts before the symbol boundary |  |

Proposal 2: Add Δt ̃values from annex B of the present contribution in annex F.4 of 38.101-1 corresponding to tpstart values {-0.7, -1, -2.7}.

- Conclusion

In this contribution is given two proposals of “hard-coded” Δt ̃ values to be added in annex F.4 of 38.101-1 to guarantee that TE vendors use identical Δt ̃values and do not lead to EVM measurement discrepancies.

Proposal 1: Add Δt ̃values from annex A of the present contribution in annex F.4 of 38.101-1 corresponding to tpstart values {-0.5, -1, -2}.

Proposal 2: Add Δt ̃values from annex A of the present contribution in annex F.4 of 38.101-1 corresponding to tpstart values {-0.7, -1, -2.7}.

# 4.	References

[1] R4-2108857, “FFT window starting point values for EVM measurements for transient period capability”, Anritsu Limited, RAN4 #99-e, May 2021

[2] 38-101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, version 17.2.0

[3] R4-2111539, “Transient Period Capability Measurements”, Skyworks Solutions Inc., RAN4 #99-e, May 2021

[4] R4-2107783, “WF on Transient period capability”, RAN4 #99-e, May 2021

## Annex A - Add Δt ̃values in annex F.4 of 38.101-1

The number of possibilities for Δt for both EVMl_tp and EVMh_tp relevant for assessing the transient period (tp) supported by a UE being limited to 8 in total, Δt ̃values could be added in annex F-4 of 38.101-1 as shown below.

## Annex B - Add Δt ̃values in annex F.4 of 38.101-1

The number of possibilities for Δt for both EVMl_tp and EVMh_tp relevant for assessing the transient period (tp) supported by a UE being limited to 8 in total, Δt ̃values could be added in annex F-4 of 38.101-1 as shown below.