3GPP TSG-RAN WG4 Meeting #100-e	R4-2112292

# Electronic Meeting, 16 – 27 August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR |  | rev |  | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 37.141: MSR band table update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | AT RAN4#99e, the band tables in the MSR specifications were corrected to properly describe the support of NR in different bands. It was also proposed to create a new more streamlined band table with fewer notes in Rel-17, but that discussion was postponed. The present band table has one column with MSR/E-UTRA band number and separate columns for NR, UTRA and GSM/EDGE band numbers. Since there are no columns for E-UTRA and NB-IoT, it is not possible to express the support of those RATs through table entries and it is instead done with notes, but not consistently. Notes are also used for the other RATs however, which duplicates information in the table. The number of notes has also grown with new RAT combinations added and there are presently 13 notes. This may get even more complicated with new bands added and the band table should be updated to become less ambiguous and more future proof. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revised band tables are introduced where the RAT support is described in separate columns for each RAT, which also reduces the number of table notes from 13 to 6. <EDITOR’S NOTE: In this Draft CR, there are three Options proposed for discussion, with ten example bands introduced in table 4.5-1 for each option. Table 4.5-2 should be updated in the same way, once an option is agreed. Option 1 is most similar to the original table, by simply adding new columns for E-UTRA and NB-IoT. Any notes restricting operation is kept at the respective band number. Option 2 is a more compact version, where there is room for a new column that will include any notes restricting operation. Notes 1 and 2 then needs a slight re-drafting. Option 3 is the most compact version, where the supported RAT is marked with an “X” for all RATs and the band number is not listed for each RAT.> |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAT support in different bands for MSR BS would remain ambiguous and difficult to update. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 37.104 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.4	Operating bands and band categories

MSR requirements are applicable for band definitions and band numbering as defined in the specifications TS 45.005 [6], TS25.104 [3], TS 25.105 [4], TS 36.104 [5] and TS 38.104 [27]. For the purpose of defining the BS requirements, the operating bands are divided into three band categories as follows:

-	Band Category 1 (BC1): Bands for NR FDD, E-UTRA FDD and/or UTRA FDD operation. Bands in this category are also used for NB-IoT operation (all modes).

-	Band Category 2 (BC2): Bands for NR FDD, E-UTRA FDD, UTRA FDD and/or GSM/EDGE operation. Bands in this category are also used for NB-IoT operation (all modes).

-	Band Category 3 (BC3): Bands for NR TDD, E-UTRA TDD and/or UTRA TDD operation. Bands in this category are also used for NB-IoT operation (all modes).

NOTE:	For UTRA TDD, requirements in the present document cover the 1.28 Mcps UTRA TDD option.

The paired and unpaired bands for the three Band Categories are shown in Table 4.4-1 and 4.4-2, together with the corresponding NR, E-UTRA, UTRA and GSM/EDGE band designations. In the present specification, the operating band of an MSR Base Stations is designated using the E-UTRA band number according to the tables.

<OPTION 1>

Table 4.4-1: Paired bands in NR, E-UTRA, UTRA and GSM/EDGE.

| MSR Band number | NR Band number | E-UTRA Band number | NB-Iot support | UTRA Band number | GSM/EDGE Band designation | Uplink (UL) BS receive UE transmit | Downlink (DL) BS transmit UE receive | BC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n1 | 1 | X | I | - | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | 1 |
| 2 | n2 | 2 | X | II | PCS 1900 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | 2 |
| 3 | n3 | 3 | X | III | DCS 1800 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | 2 |
| 4 | - | 4 | X | IV | - | 1710 MHz – 1755 MHz | 2110 MHz – 2155 MHz | 1 |
| 5 | n5 | 5 | X | V | GSM 850 | 824 MHz – 849 MHz | 869 MHz – 894MHz | 2 |
| 6 | - | - | - | VI | - | 830 MHz – 840 MHz | 875 MHz – 885 MHz | 1 |
| 29 | n29 | 29 (Note 1) | - | - | - | N/A | 717 MHz – 728 MHz | 1 |
| 30 | n30 | 30 | - | - | - | 2305 MHz – 2315 MHz | 2350 MHz – 2360 MHz | 1 |
| 31 | - | 31 | X | - | - | 452.5 MHz – 457.5 MHz | 462.5 MHz – 467.5 MHz | 1 |
| 32 | - | 32 (Note 1) | - | XXXII (Note 2) | - | N/A | 1452 MHz – 1496 MHz | 1 |
| NOTE 1: Restricted to NR and/or E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 2: Restricted to UTRA operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequenc(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 3: In E-UTRA operation, the range 2180-2200 MHz of the DL operating band is restricted to operation when carrier aggregation is configured. NOTE 4: Band 23 is not applicable. NOTE 5: In E-UTRA operation, the range 2010-2020 MHz of the DL operating band is restricted to operation when carrier aggregation is configured, and TX-RX separation is 300 MHz. In E-UTRA operation, the range 2005-2020 MHz of the DL operating band is restricted to operation when carrier aggregation is configured, and TX-RX separation is 295 MHz. NOTE 6: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. |  |  |  |  |  |  |  |  |

<OPTION 2>

Table 4.4-1: Paired bands in NR, E-UTRA, UTRA and GSM/EDGE.

| MSR Band number | Supported RATs and Band Numbers |  |  |  |  | Uplink (UL) BS receive, UE transmit | Downlink (DL) BS transmit, UE receive | BC | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA | NB-IoT | UTRA | GSM/EDGE |  |  |  |  |
| 1 | n1 | 1 | X | I | - | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | 1 |  |
| 2 | n2 | 2 | X | II | PCS 1900 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | 2 |  |
| 3 | n3 | 3 | X | III | DCS 1800 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | 2 |  |
| 4 | - | 4 | X | IV | - | 1710 MHz – 1755 MHz | 2110 MHz – 2155 MHz | 1 |  |
| 5 | n5 | 5 | X | V | GSM 850 | 824 MHz – 849 MHz | 869 MHz – 894MHz | 2 |  |
| 6 | - | - | - | VI | - | 830 MHz – 840 MHz | 875 MHz – 885 MHz | 1 |  |
| 29 | n29 | 29 | - | - | - | N/A | 717 MHz – 728 MHz | 1 | Note 1 |
| 30 | n30 | 30 | - | - | - | 2305 MHz – 2315 MHz | 2350 MHz – 2360 MHz | 1 |  |
| 31 | - | 31 | X | - | - | 452.5 MHz – 457.5 MHz | 462.5 MHz – 467.5 MHz | 1 |  |
| 32 | - | 32 | - | XXXII | - | N/A | 1452 MHz – 1496 MHz | 1 | Note1, Note 2 |
| NOTE 1: For NR and/or E-UTRA, the band is restricted to operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 2: For UTRA, the band is restricted to operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequenc(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 3: In E-UTRA operation, the range 2180-2200 MHz of the DL operating band is restricted to operation when carrier aggregation is configured. NOTE 4: Band 23 is not applicable. NOTE 5: In E-UTRA operation, the range 2010-2020 MHz of the DL operating band is restricted to operation when carrier aggregation is configured, and TX-RX separation is 300 MHz. In E-UTRA operation, the range 2005-2020 MHz of the DL operating band is restricted to operation when carrier aggregation is configured, and TX-RX separation is 295 MHz. NOTE 6: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. |  |  |  |  |  |  |  |  |  |

<OPTION 3>

Table 4.4-1: Paired bands in NR, E-UTRA, UTRA and GSM/EDGE.

| MSR Band number | Supported RATs |  |  |  |  | Uplink (UL) BS receive, UE transmit | Downlink (DL) BS transmit, UE receive | BC | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA | NB-IoT | UTRA | GSM/EDGE |  |  |  |  |
| 1 | X | X | X | X | - | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | 1 |  |
| 2 | X | X | X | X | X | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | 2 |  |
| 3 | X | X | X | X | X | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | 2 |  |
| 4 | - | X | X | X | - | 1710 MHz – 1755 MHz | 2110 MHz – 2155 MHz | 1 |  |
| 5 | X | X | X | X | X | 824 MHz – 849 MHz | 869 MHz – 894MHz | 2 |  |
| 6 | - | - | - | X | - | 830 MHz – 840 MHz | 875 MHz – 885 MHz | 1 |  |
| 29 | X | X | - | - | - | N/A | 717 MHz – 728 MHz | 1 | Note 1 |
| 30 | X | X | - | - | - | 2305 MHz – 2315 MHz | 2350 MHz – 2360 MHz | 1 |  |
| 31 | - | X | X | - | - | 452.5 MHz – 457.5 MHz | 462.5 MHz – 467.5 MHz | 1 |  |
| 32 | - | X | - | X | - | N/A | 1452 MHz – 1496 MHz | 1 | Note1, Note 2 |
| NOTE 1: For NR and/or E-UTRA, the band is restricted to operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 2: For UTRA, the band is restricted to operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequenc(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 3: In E-UTRA operation, the range 2180-2200 MHz of the DL operating band is restricted to operation when carrier aggregation is configured. NOTE 4: Band 23 is not applicable. NOTE 5: In E-UTRA operation, the range 2010-2020 MHz of the DL operating band is restricted to operation when carrier aggregation is configured, and TX-RX separation is 300 MHz. In E-UTRA operation, the range 2005-2020 MHz of the DL operating band is restricted to operation when carrier aggregation is configured, and TX-RX separation is 295 MHz. NOTE 6: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. |  |  |  |  |  |  |  |  |  |

| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

NOTE:	For BS capable of multi-band operation, the supported operating bands may belong to different Band Categories.

Table 4.4-2: Unpaired bands in NR, E-UTRA and UTRA

| MSR and E-UTRA Band number | NR Band number | UTRA Band number | Uplink (UL) BS receive UE transmit |  |  | Downlink (DL) BS transmit UE receive |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 |  | a) | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | 3 |
| 34 | n34 | a) | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | 3 |
| 35 |  | b) | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | 3 |
| 36 |  | b) | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | 3 |
| 37 |  | c) | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | 3 |
| 38 | n38 | d) | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | 3 |
| 39 | n39 | f) | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | 3 |
| 40 | n40 | e) | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | 3 |
| 41 | n41 | - | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | 3 (NOTE 1) |
| 42 |  | - | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | 3 (NOTE 1) |
| 43 |  | - | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | 3 (NOTE 1) |
| 44 |  | - | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | 3 |
| 45 |  | - | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | 3 |
| 48 | n48 | - | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | 3 |
| 50 | n50 | - | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | 3 |
| 51 | n51 | - | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | 3 |
| 52 |  | - | 3300 MHz | – | 3400 MHz | 3300 MHz | – | 3400 MHz | 3 |
| 53 | n53 | - | 2483.5 MHz | – | 2495 MHz | 2483.5 MHz | – | 2495 MHz | 3 |
| 77 | n77 | - | 3300 MHz | - | 4200 MHz | 3300 MHz | - | 4200 MHz | 3 (NOTE 2) |
| 78 | n78 | - | 3300 MHz | - | 3800 MHz | 3300 MHz | - | 3800 MHz | 3 (NOTE 2) |
| NOTE 1: The band 41 supports NB-IoT (in certain regions). The band 42 and 43 support NB-IoT. NOTE 2: The band is for NR only. |  |  |  |  |  |  |  |  |  |

Table 4.4-3. Void

Table 4.4-4. Void

E-UTRA is designed to operate for the carrier aggregation bands defined in TS 36.101 [28]. The E-UTRA channel bandwidth BWChannel for a single carrier and the Aggregated Channel Bandwidth BWChannel_CA for E-UTRA carrier aggregation are specified in clause 5.6 of TS 36.104 [5].

The NB-IoT channel bandwidth BWChannel is specified in clause 5.6 of TS 36.104 [5].

The NR BS channel bandwidth and PRB utilization is specified in clause 5.3 of TS 38.104 [27].