3GPP TSG-RAN WG4 Meeting #100e	R4-2112321

Electronic Meeting, 16th - 27th Aug, 2021

Source:	ZTE Corporation

Title:	Unified REFSENS equation for all FR1 bands

Agenda Item:	8.27.2

Document for:	Discussion

# Introduction

In RAN4#99e, RAN4 has discussed several ways to simplify the REFSENS table to resolve the displaying issue , and a consensus was reached that equation-based approach is to be used to simplify the FR1 REFSENS table format [1]. And there are still several options to implement this approach:

- Option 1: One table for TDD bands (equation-based REFSENS) and the other table for FDD and SDL bands (explicit REFSENS values) - Option 2: One table for all FR1 bands without MSD (equation-based REFSENS) and the other table for FDD bands with MSD (MSD is additive to the same band with equation-based REFSENS) - Option 3: One table for all FR1 bands (unified equation-based REFSENS) - Option 4: Other proposals

In this contribution, we continue to advertise the unified equation for all TDD and FDD bands in FR1 as the proponent.

# Discussion

In our previous contribution, we identified that it is possible to use a unified equation to specify REFSENS for each TDD and FDD band in FR1 [1]:

REFSENS=-100+10∙log10NRB∙SCSkHz∙124500+∆R

Where

- NRB is the maximum transmission bandwidth configuration for the concerned channel bandwidth and SCS.

- the REFSENS adjustment ∆R is a band-specific offset as shown in the table in Appendix:

During the discussions, a concern was raised that whether or not the band-specific offset ∆R is applicable to all channel bandwidths for the band. Actually, we have verified that it is applicable to all channel bandwidths supported in the band.

Observation 1: The band specific offset ∆R is applicable to all channel bandwidths supported in the band.

Furthermore, if more channel bandwidths (no matter existing or brand new) are added, what is needed is just to add the channel bandwidth to the list cell in the table for the concerned band, so there is no displaying issue anymore, which means the single table based on the unified equation approach for REFSENS is future proof.

Observation 2: The single table for REFSENS for all bands including TDD and FDD bands in FR1 based on the unified equation approach is a future-proof solution.

So we suggest RAN4 to take the unified equation approach for simplifying the REFSENS table to one single table:

Proposal: Simplify REFSENS table to one single table for all bands including TDD and FDD bands in FR1 based on the unified equation.

And we also provide a draft CR based on this proposal in our companion paper.

# Conclusion

In this paper, we have the following observations and proposal for simplifying the REFSENS table based on the unified equation:

Observation 1: The band specific offset ∆R is applicable to all channel bandwidths supported in the band.

Observation 2: The single table for REFSENS for all bands including TDD and FDD bands in FR1 based on the unified equation approach is a future-proof solution.

Proposal: Simplify REFSENS table to one single table for all bands including TDD and FDD bands in FR1 based on the unified equation.

# Appendix

The band specific offsets are shown in the below table:

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| REFSENS (dBm) = 10*log10(N_RB*12*SCS(kHz)/4500)+∆R-100 |  |  |  |  |
| Operating Band | SCS kHz | Channel bandwidth (MHz) | ∆R | Duplex Mode |
| n1 | 15 | 5,10,15,20,25,30,40,50 | 0 | FDD |  |
|  | 30 | 10,15,20,25,30,40,50 |  |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |  |
| n2 | 15 | 5,10,15,20 | 2 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 | 10,15,20 |  |  |  |
| n3 | 15 | 5,10,15,20,25,30,40 | 3 | FDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n5 | 15 | 5,10,15,20 | 2 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n71 | 15 | 5,10,15,20,25,30,40,50 | 2 | FDD |  |
|  | 30 | 10,15,20,25,30,40,50 |  |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |  |
| n8 | 15 | 5,10,15,20 | 3 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n12 | 15 | 5,10,15 | 3 | FDD |  |
|  | 30 | 10,15 |  |  |  |
|  | 60 |  |  |  |  |
| n14 | 15 | 5,10 | 3 | FDD |  |
|  | 30 | 10 |  |  |  |
|  | 60 |  |  |  |  |
| n18 | 15 | 5,10,15 | 0 | FDD |  |
|  | 30 | 10,15 |  |  |  |
|  | 60 |  |  |  |  |
| n20 | 15 | 5,10,15,20 | 3 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n25 | 15 | 5,10,15,20,25,30,40 | 3.5 | FDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n26 | 15 | 5,10,15,20 | 2.44 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
| n28 | 15 | 5,10,15,20,30 | 1.5 | FDD |  |
|  | 30 | 10,15,20,30 |  |  |  |
|  | 60 |  |  |  |  |
| n29x | 15 | 5,10 | 3 | SDL |  |
|  | 30 | 10 |  |  |  |
|  | 60 |  |  |  |  |
| n30 | 15 | 5,10 | 1 | FDD |  |
|  | 30 | 10 |  |  |  |
|  | 60 |  |  |  |  |
| n34 | 15 | 5,10,15 | 0 | TDD |  |
|  | 30 | 10,15 |  |  |  |
|  | 60 | 10,15 |  |  |  |
| n381 | 15 | 5,10,15,20,25,30,40 | 0 | TDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n39 | 15 | 5,10,15,20,25,30,40 | 0 | TDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n40 | 15 | 5,10,15,20,25,30,40,50 | 0 | TDD |  |
|  | 30 | 10,15,20,25,30,40,50,60,80 |  |  |  |
|  | 60 | 10,15,20,25,30,40,50,60,80 |  |  |  |
| n411 | 15 | 10,15,20,30,40,50 | 2 | TDD |  |
|  | 30 | 10,15,20,30,40,50,60,80,90,10 |  |  |  |
|  | 60 | 10,15,20,30,40,50,60,80,90,10 |  |  |  |
| n481 | 15 | 5, 10, 15, 20, 40, 505 | 1 | TDD |  |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 |  |  |  |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 |  |  |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | 0 | TDD |  |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 |  |  |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 |  |  |  |
| n51 | 15 | 5 | 0 | TDD |  |
|  | 30 |  |  |  |  |
|  | 60 |  |  |  |  |
| n53 | 15 | 5, 10 | 0 | TDD |  |
|  | 30 | 10 |  |  |  |
|  | 60 | 10 |  |  |  |
| n65 | 15 | 5,10,15,20,50 | 0.5 | FDD |  |
|  | 30 | 10,15,20,50 |  |  |  |
|  | 60 | 10,15,20,50 |  |  |  |
| n66 | 15 | 5,10,15,20,25,30,40 | 0.5 | FDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n70 | 15 | 5,10,15,20,25 | 0 | FDD |  |
|  | 30 | 10,15,20,25 |  |  |  |
|  | 60 | 10,15,20,25 |  |  |  |
| n71 | 15 | 5,10,15,20 | 2.8 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n74 | 15 | 5,10,15,20 | 0.47 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 | 10,15,20 |  |  |  |
| n757 | 15 | 5,10,15,20,25,30,40,50 | 0 | SDL |  |
|  | 30 | 10,15,20,25,30,40,50 |  |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |  |
| n767 | 15 | 5 | 0 | SDL |  |
|  | 30 |  |  |  |  |
|  | 60 |  |  |  |  |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | 1.5 | TDD |  |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | 1 | TDD |  |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| n791 | 15 | 40, 50 | 1 | TDD |  |
|  | 30 | 40, 50, 60, 80, 100 |  |  |  |
|  | 60 | 40, 50, 60, 80, 100 |  |  |  |
| n91 | 15 | 5 | 0 | FDD |  |
|  | 30 |  |  |  |  |
|  | 60 |  |  |  |  |
| n92 | 15 | 5,10,15,20 | 0 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n93 | 15 | 5 | 0 | FDD |  |
|  | 30 |  |  |  |  |
|  | 60 |  |  |  |  |
| n94 | 15 | 5,10,15,20 | 0 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. |  |  |  |  |  |
| NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |
| NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. |  |  |  |  |  |
| NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. |  |  |  |  |  |
| NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. |  |  |  |  |  |
| NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. |  |  |  |  |  |
| NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. |  |  |  |  |  |
| NOTE 8: NRB in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: The REFSENS value is rounded to the nearest number down to one decimal point. |  |  |  |  |  |

# Reference

R4-2107820, WF on REFSENS Table Split and Simplification, Apple/Skyworks/ZTE

R4-2109418, A unified equation for specifying REFSENS for both TDD and FDD bands, ZTE