3GPP RAN WG4 Meeting #100-e		R4-2112361

# Electronic Meeting, 16 – 27 August, 2021

Agenda item:	8.32.2.1

Source:	Apple Inc.

Title:	Considerations and A-MPR results for PC2 n39

WI/SI:	NR_UE_PC2_n39-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The WF for PC2 n39 [1] asks interested companies to provide their input for PC2 n39 A-MPR for CBW equal and below 20MHz. This contribution provides an analysis on A-MPR and also considerations on general coexistence requirements with PC2.

# 2  Discussion

The following assumptions for the simulations were used:

- Power Class 2

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

##### 2.2 NS_50 for CBW ≤ 20MHz

The simulations were done for 5MHz, 10MHz, 15MHz and 20MHz channel bandwidth for NS_50. The aggregated channel bandwidth was confined in 1885MHz to 1920MHz. The actual carrier frequencies are provided in the result section 3. Regions are proposed in Table 1.

Proposal 1: Use allocations regions found in Table 1 for 10MHz, 15MHz and 20MHz CBW NS_50.

It was observed that in Table 2 the region A8 allows 3dB A-MPR for 16QAM DFT-s-OFDM. This is due to the increased need for outer allocations. As the MPR for 64QAM is 2.5dB (which is 0.5dB below) it should be considered to also include 3dB A-MPR allowance for 64QAM. The proposed change is marked in Table 2 with red color.

Proposal 2: As DFT-s-OFDM for 16QAM has 3dB A-MPR in region A8 and Table 2, it should be considered to also provide 3dB for 64QAM.

Table 1: PC2 A-MPR regions for NS_50 Channel Bandwidth (MHz); RBstart*12*SCS (MHz); LCRB*12*SCS (MHz); A-MPR. 10 MHz; ≤ LCRB*12*SCS – 7.2; > 7.2; A8. > LCRB*12*SCS – 7.2 ≤ 1.08; > 0; A8. 15 MHz; ≤ LCRB*12*SCS – 7.2; > 7.2; A6. > LCRB*12*SCS – 7.2 ≤ 2.88; > 0; A8. 20 MHz; ≤ LCRB*12*SCS – 7.2; > 7.2; A3. > LCRB*12*SCS – 7.2 ≤ 4.32; > 0; A8. NOTE 1: The A-MPR values are specified in Table xxx.. Table 2: PC2 A-MPR for NS_50 Modulation/Waveform; A1 (dB); A2 (dB); A3 (dB); A4 (dB); A6 (dB); A7 (dB); A8 (dB); A9 (dB). Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ [11.5]; ≤ [8.0]; ≤ [4.0]; ≤ [2.5]; ≤ [2.5]; ≤ [4.5]; ≤ [2.5]; ≤ [1.0]. QPSK; ≤ [11.5]; ≤ [8.0]; ≤ [4.0]; ≤ [2.5]; ≤ [2.5]; ≤ [5.5]; ≤ [2.5]; ≤ [1.0]. 16 QAM; ≤ [11.5]; ≤ [8.0]; ≤ [4.0]; ≤ [2.5]; ≤ [2.5]; ≤ [5.5]; ≤ [3.0]; ≤ [1.5]. 64 QAM; ≤ [11.5]; ≤ [8.0]; ≤ [4.0]; ≤ [5.5]; ≤ [3.0]. 256 QAM; ≤ [11.5]; ≤ [8.0]; ≤ [5.5]. CP-OFDM; QPSK; ≤ [12.5]; ≤ [9.0]; ≤ [5.5]; ≤ [4.0]; ≤ [4.0]; ≤ [7.0]; ≤ [2.0]; ≤ [2.0]. 16 QAM; ≤ [12.5]; ≤ [9.0]; ≤ [5.5]; ≤ [4.0]; ≤ [4.0]; ≤ [7.0]. 64 QAM; ≤ [12.5]; ≤ [9.0]; ≤ [5.5]; ≤ [4.0]; ≤ [7.0]. 256 QAM; ≤ [12.5]; ≤ [9.0]; ≤ [7.0].

# 3  Simulations

##### 3.1 NS_50

# Conclusions

This paper presents A-MPR simulation results for n39 with NS_50 and CBW 20MHz and below.

Proposal 1: Use allocations regions found in Table 1 for 10MHz, 15MHz and 20MHz CBW NS_50.

Proposal 2: As DFT-s-OFDM for 16QAM has 3dB A-MPR in region A8 and Table 2, it should be considered to also provide 3dB for 64QAM.

# 4	References

- R4-2107738, “WF on A-MPR for PC2 n39”, Huawei, HiSilicon, Apple, Nokia, 3GPP TSG RAN Meeting #99-e