3GPP RAN WG4 Meeting #100-e		R4-2112363

Electronic Meeting, 16 – 27 August, 2021

Agenda item:	9.2.2

Source:	Apple

Title:	Views on test cases which employ hand phantoms

Work Item:	NR_FR1_TRP_TRS-Core

Document for:	Discussion

# 1	Introduction

A new work item to define the OTA test methodology and TRP/TRS requirements for UEs operating in NR FR1 stand-alone and EN-DC within FR1 configurations was approved during the RAN #91 [1] and further revised during RAN #92 [2].  During the RAN4 #99 meeting a liaison was sent to CTIA with the following request [3]:

1 Overall description 3GPP RAN Plenary approved the WI on FR1 TRP TRS OTA in RAN#91e meeting, March 2021. This WI is to define FR1 OTA test methodology and specify corresponding FR1 TRP and TRS OTA requirements, for both SA and EN-DC UEs. Based on objectives in the WID, phantom-based testing has been agreed as the 1st priority test condition for smartphone UEs. CTIA is expertized on phantoms development and has been successfully defined the human phantoms for 5G OTA testing. Using the same set of phantoms for UE antenna performance measurement in the industry is not only beneficial to UE conformance testing but also for product development. 3GPP RAN4 would like to cooperate with CTIA on FR1 OTA test methods and try to define a harmonized test method for the industry. Therefore, 3GPP would like to ask for the permission of head&hand phantoms defined in CTIA for 3GPP FR1 OTA test method development, feedback from CTIA on the following question is much appreciated: - Can 3GPP reference the head and hand phantoms defined in CTIA OTA test plan to develop FR1 OTA test methods? 2 Actions To CTIA: ACTION: 3GPP RAN4 respectfully asks CTIA to share feedback on the above information and encourage CTIA to further collaborate with 3GPP RAN4 on 5G FR1 OTA test method and requirement.

The agreements following the RAN4 #99 meeting, captured in [4], indicated the development of head and hand phantoms as one of the open issues.

This contribution provides our views on test cases which employ hand phantoms.

# 2	Discussion

Considering the scope of TRP/TRS testing methodology for smartphones, two phantom configurations can be applicable:  talk mode beside head and hand (BHH) and data mode in hand (H).

According to TR25.914 [5], the quiet zone dimension and minimum distance to the DUT are defined as follows:

By applying the TR37.914 definiton, the minimum radius of the quiet zone of 150 mm implies that the maximum extension of the radiating structure, when the DUT is placed in a hand phantom and next to the head phantom, spans the diameter of the quiet zone, or 300 mm.  Table 1 below provides the minimum measurement criterion calculations for several NR bands.

Table 1: Minimum measurement distance criterion

| NR Band | Uplink frequency (MHz) |  | Downlink frequency (MHz) |  | Wavelength (m) |  | Phase limit 2D2/λ | Amplitude limit 3D | Reactive near-field limit 3λ | min R (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low frequency | High frequncy | Low frequency | High frequency | Low frequency | High frequency |  |  |  |  |
| n71 | 663 | 698 | 617 | 652 | 0.49 | 0.43 | 0.42 | 0.90 | 1.46 | 1.46 |
| n28 | 703 | 748 | 758 | 803 | 0.43 | 0.37 | 0.48 | 0.90 | 1.28 | 1.28 |
| n66 | 1710 | 1780 | 2110 | 2200 | 0.18 | 0.14 | 1.32 | 0.90 | 0.53 | 1.32 |
| n1 | 1920 | 1980 | 2110 | 2170 | 0.16 | 0.14 | 1.30 | 0.90 | 0.47 | 1.30 |
| n40 | 2300 | 2400 | 2300 | 2400 | 0.13 | 0.12 | 1.44 | 0.90 | 0.39 | 1.44 |
| n41 | 2496 | 2690 | 2496 | 2690 | 0.12 | 0.11 | 1.62 | 0.90 | 0.36 | 1.62 |
| n77 | 3300 | 4200 | 3300 | 4200 | 0.09 | 0.07 | 2.52 | 0.90 | 0.27 | 2.52 |
| n78 | 3300 | 3800 | 3300 | 3800 | 0.09 | 0.08 | 2.28 | 0.90 | 0.27 | 2.28 |
| n79 | 4400 | 5000 | 4400 | 5000 | 0.07 | 0.06 | 3.00 | 0.90 | 0.20 | 3.00 |
| n46 | 5150 | 5925 | 5150 | 5925 | 0.06 | 0.05 | 3.56 | 0.90 | 0.17 | 3.56 |
| n96 | 5925 | 7125 | 5925 | 7125 | 0.05 | 0.04 | 4.28 | 0.90 | 0.15 | 4.28 |

We observe that the minimum measurement distance increases with frequency up to 4.28 m for band n96.  One area for potential investigation can be to determine whether the assumption placed on D is valid for higher frequencies:  i.e. is the maximum extension of the radiating structure a constant value of D=300 mm?  In addition, is it possible to determine whether the radiating structure needs to enclose the DUT and hand phantom for all test setups?

Observation 1:	RAN4 should study whether the assumption D=300 mm is valid for higher frequencies.

Observation 2:	RAN4 should study whether the radiating structure needs to enclose the DUT and hand phantom for all test setups.

Two distinct hand phantom designs accommodate device width ≤ 72 mm and device width > 72 mm.  While developing TRP/TRS requirements for LTE handsets, RAN4 recognized this distinction and focused the effort on device width ≤ 72 mm [6].

Observation 3:	RAN4 should define separate BHH and H requirements for devices with width ≤ 72 mm and with width > 72 mm.

# 3	Conclusions

This contribution provides our views on test cases which employ hand phantoms and makes the following observations:

Observation 1:	RAN4 should study whether the assumption D=300 mm is valid for higher frequencies.

Observation 2:	RAN4 should study whether the radiating structure needs to enclose the DUT and hand phantom for all test setups.

Observation 3:	RAN4 should define separate BHH and H requirements for devices with width ≤ 72 mm and width > 72 mm.

# 4	References

RP-210807, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, CAICT, Rohde & Schwarz, 3GPP RAN #91, March 2021

RP-211158, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, 3GPP RAN #92, June 2021

R4-2108621, "LS on Head and Hand Phantoms for 5G FR1 OTA testing," 3GPP RAN4 #99, May 2021

R4-2108620, "WF on FR1 TRP TRS," vivo, 3GPP RAN4 #99

3GPP TR25.914, "Measurements of radio performances for UMTS terminals in speech mode"

R4-75AH-TRPS-0005, "Text proposal for TS.37.144 on TRP/TRS BHH testing conditions," Sony Mobile, 3GPP RAN4 #75 OTA TRP/TRS adhoc, July 2013