3GPP TSG-RAN WG4 Meeting #100-e                                R4-2112450

Online, 16th-27th Aug, 2021

Source:	Samsung, Verizon

Title:	TP for TR 37.717-21-11: DC_48-66_n2

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_48-66_n2 according to the request in [1].

2. Reference

- R4-2110666 WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

- R4-2110665 TR 37.717-21-11 v0.5.0_DC of 2 LTE band and 1 NR band

# 3. Text Proposal

<Start of Text Proposal>

5.x	DC_48-66_n2

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | DC_66A_n2A |

### 5.x.2	Co-existence studies

Based on the co-existence study of UL DC_66A_n2A, IMD2 of b66 UL and n2 UL falls into band 48 RX.

### 5.x.3	∆TIB and ∆RIB values

For DC_48-66_n2, the TIB,c and RIB,c values are reused from DC_2-48_n66 shown below:

Table 5.x.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n2 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |

Table 5.x.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n2 | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.x.4	REFSENS requirements

Based on 5.x.2 discussion above, MSD values due to IMD are reused from DC_2-48_n66 shown below:

Table 5.x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |

<End of Text Proposal>