3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112523

Electronic Meeting, August 16-27, 2021

Agenda Item: 9.8.2.1.1

Source: MediaTek Inc.

Title: Discussion on Rel-17 HST in FR1 for intra-frequency measurement

Document for: Discussion

- Introduction

In last meeting, one remaining issue regarding the Kp factor for deactivated SCell measurement is still under discussion without conclusion. Our views on this open issue are provided in this paper.

- Discussion

The Kp for the requirements on deactivated SCell are on-going discussion as the WF in RAN4 #99e.

- Kp for the requirements on deactivated SCell - FFS whether Rel-16 Kp requirements modifications (if any) shall also apply for R17 HST

For the non-HST case, Kp factor has been introduced for deactivated SCell measurement in last meeting, as the agreement in RAN4 #99e:

Issue 1-1-2: Kp factor for measurement on deactivated SCC - Agreements: - Kp shall also apply for measurement requirements on SCC with deactivated SCell, where Kp = 1/(1- (SMTC period /MGRP)) - Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra

Observation 1: For deactivated SCell in non-HST, Kp factor is introduced.

On the other hand, for HST intra-frequency without MG, Kp has also been introduced. The intra-frequency measurement for activated SCell in R17 HST is provided for reference as follows.

The measurement period of PCell for intra-frequency measurements without gaps in clause 9.2.5 in TS 38.133 Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1.. Table 9.2.5.1-3: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps(FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2.5.2-5: T SSB_measurement_period_intra When highSpeedMeasFlag-r16 is configured (Frequency range FR1) DRX cycle; T SSB_measurement_period_intra. No DRX Note 2; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 160ms; max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. 160ms < DRX cycle≤ 320ms; ceil(4 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle). DRX cycle>320ms; ceil( Y Note 3 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms.

As shown in the above tables, it can be observed that the Kp factor has been applied for both HST and non-HST scenarios.

Observation 2: For intra frequency without MG, Kp factor has been introduced for both HST and non-HST scenarios.

Thus, according to Observation 1 and Observation 2, we suggest the Kp factor should also be introduced for deactivated SCell in HST case to align with intra-frequency measurement requirement without MG.

Proposal 1: For R17 HST, scaling factor Kp should be introduced in PSS/SSS detection, time index detection and measurement requirement for deactivated SCells.

- Summary

In this paper, the discussion of R17 HST in FR1 is provided. We have the following proposal:

Observation 1: For deactivated SCell in non-HST, Kp factor is introduced.

Observation 2: For intra frequency without MG, Kp factor has been introduced for both HST and non-HST scenarios.

Proposal 1: For R17 HST, scaling factor Kp should be introduced in PSS/SSS detection, time index detection and measurement requirement for deactivated SCells.