3GPP TSG-RAN4 WG4 Meeting #100-e	R4-2112559

# Electronic meeting, 16 August – 27 August, 2021

Source: 	vivo

Title: 	Discussion on timing requirements for 52.6-71GHz

Agenda Item:	9.16.7.2

Document for:	Discussion

Introduction

The item on impact of higher SCS on RRM requirements from 52.6GHz to 71GHz was discussed at RAN4 #99e. A WF [1] related to Timing was duplicated as below:

- Define new RRM requirements due to higher data/SSB SCS for at least the following topics: - Timing - UE transmit timing - Timing advance (TA) - Interruptions - Active BWP switching delay - Measurement gaps - Interruption time

Specifying new SCS, 480kHz and 960kHz, timing aspects including UE transmit timing and Timing advance needs to be considered. The existing requirements for UE transmit timing and Timing advance are specified as below [TS38.133].

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. Table 7.1.2-1: Te Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te. 1; 15; 15; 12*64*Tc. 30; 10*64*Tc. 60; 10*64*Tc. 30; 15; 8*64*Tc. 30; 8*64*Tc. 60; 7*64*Tc. 2; 120; 60; 3.5*64*Tc. 120; 3.5*64*Tc. 240; 60; 3*64*Tc. 120; 3*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 7.3.2.2-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. Table 7.3.2.2-1: UE Timing Advance adjustment accuracy UL Sub Carrier Spacing(kHz); 15; 30; 60; 120. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc.

In this paper, we review the UE initial transmission timing error and timing advance adjustment accuracy for NR and discuss the requirements for timing from 52.6GHz to 71GHz.

Discussion

Initial transmit timing error

In [2], it was pointed out that Te without margin depend on the DL BW of the signal used for DL timing estimation. The minimum timing error would be the inverse of signal BW. In NR, UE can derive its timing by using SS/PBCH block which consists of 240 contiguous subcarriers. Table 1 shows both minimum timing error and margin for different combinations of SSB signals and uplink signals.

Table 1: initial transmission timing error for NR

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te | SS/PBCH block BW (MHz) | Minimum timing error | Margin (Te- Minimum timing error) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc | 15*240=3.6 | 4.2*64*Tc | 7.8*64*Tc |
|  |  | 30 | 10*64*Tc | 15*240=3.6 | 4.2*64*Tc | 5.8*64*Tc |
|  |  | 60 | 10*64*Tc | 15*240=3.6 | 4.2*64*Tc | 5.8*64*Tc |
|  | 30 | 15 | 8*64*Tc | 30*240=7.2 | 2.1*64*Tc | 5.9*64*Tc |
|  |  | 30 | 8*64*Tc | 30*240=7.2 | 2.1*64*Tc | 5.9*64*Tc |
|  |  | 60 | 7*64*Tc | 30*240=7.2 | 2.1*64*Tc | 4.9*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc | 120*240=28.8 | 0.53*64*Tc | 2.97*64*Tc |
|  |  | 120 | 3.5*64*Tc | 120*240=28.8 | 0.53*64*Tc | 2.97*64*Tc |
|  | 240 | 60 | 3*64*Tc | 240*240=57.6 | 0.26*64*Tc | 2.74*64*Tc |
|  |  | 120 | 3*64*Tc | 240*240=57.6 | 0.26*64*Tc | 2.74*64*Tc |

From the above table, it is observed that for FR1 and FR2, the minimum margin is about 5Ts and 2.75Ts respectively. We consider for 52.6-71GHz, the margin should not change much compared with the existing FR2. Meanwhile, it was agreed that the margin decrease with the increasing SCS in [3].

According to the above discussion and analysis, we suggest the margin is about 2.5Ts and 2.25Ts for 480kHz and 960kHz SSB SCS. The final values of initial transmission timing error are computed as the sum of minimum timing error and margin. The result for 52.6-71GHz is shown in Table 2.

Table 2: initial transmission timing error for 52.6-71GHz

| SCS of SSB signals(kHz) | SCS of uplink signals(kHz) | SS/PBCH block BW (MHz) | Minimum timing error (Ts) | Margin (Ts) | initial transmission timing error (Ts) | Round off values (Ts) |
| --- | --- | --- | --- | --- | --- | --- |
| 120 | 120 | 120*240=28.8 | 0.53 | 2.97 | 3.5 | 3.5 |
|  | 480 | 120*240=28.8 | 0.53 | 2.97 | 3.5 | 3.5 |
|  | 960 | 120*240=28.8 | 0.53 | 2.97 | 3.5 | 3.5 |
| 480 | 120 | 480*240=115.2 | 0.1356 | 2.5 | 2.6356 | 2.75 |
|  | 480 | 480*240=115.2 | 0.1356 | 2.5 | 2.6356 | 2.75 |
|  | 960 | 480*240=115.2 | 0.1356 | 2.5 | 2.6356 | 2.75 |
| 960 | 120 | 960*240=230.4 | 0.0678 | 2.25 | 2.3178 | 2.5 |
|  | 480 | 960*240=230.4 | 0.0678 | 2.25 | 2.3178 | 2.5 |
|  | 960 | 960*240=230.4 | 0.0678 | 2.25 | 2.3178 | 2.5 |

For higher SCS, tighter timing requirements need to be applied in order to guarantee UL performance. From the above table, it is observed that when UL SCS is greater than SSB SCS, it would cause UL performance degradation. So it should be considered that the uplink signal SCS should be no greater than SSB SCS for 52.6-71GHz. In other words, 120kHz SSB SCS only support 120kHz UL SCS and 480kHz SSB SCS only support 120kHz and 480kHz UL SCS. We have revised Table 2 and the result is shown in Table 3.

Table 3: revised initial transmission timing error for 52.6-71GHz

| SCS of SSB signals(kHz) | SCS of uplink signals(kHz) | SS/PBCH block BW (MHz) | Minimum timing error (Ts) | Margin (Ts) | initial transmission timing error (Ts) | Round off values (Ts) |
| --- | --- | --- | --- | --- | --- | --- |
| 120 | 120 | 120*240=28.8 | 0.53 | 2.97 | 3.5 | 3.5 |
| 480 | 120 | 480*240=115.2 | 0.1356 | 2.5 | 2.6356 | 2.75 |
|  | 480 | 480*240=115.2 | 0.1356 | 2.5 | 2.6356 | 2.75 |
| 960 | 120 | 960*240=230.4 | 0.0678 | 2.25 | 2.3178 | 2.5 |
|  | 480 | 960*240=230.4 | 0.0678 | 2.25 | 2.3178 | 2.5 |
|  | 960 | 960*240=230.4 | 0.0678 | 2.25 | 2.3178 | 2.5 |

Proposal 1: The uplink signal SCS should be no greater than SSB SCS for 52.6-71GHz.

In [4], it was agreed that FR2 is extended to cover 24.25GHz to 71GHz with FR2-1 for 24.25-52.6GHz and FR2-2 for 52.6-71GHz. For FR2-1, UE support 120kHz and 240kHz SSB SCS for 60kHz and 120kHz uplink signal SCS. For FR2-2, UE support 120kHz, 480kHz and 960kHz SCS for both SSB and uplink signal. FR2-1 and FR2-2 are different in content, therefore we discuss the requirements separately. Combine the above result with the existing content in TS38.133 and the final initial transmission timing error can be shown in Table 4.

Proposal 2: Since FR2-1 and FR2-2 is different in content, the requirements for 24.25-52.6GHz and 52.6-71GHz need to be discussed separately.

From the result, it is observed that for larger SCS of uplink signals, 480kHz and 960kHz, Te may exceeds the cyclic prefix length and it could cause UL performance degradation. In our opinion, scheduling restriction may be needed on the first symbol.

Proposal 3: The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 4.

Table 4: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  | 480 | 120 | 2.75*64*Tc |
|  |  | 480 | 2.75*64*Tc |
|  | 960 | 120 | 2.5*64*Tc |
|  |  | 480 | 2.5*64*Tc |
|  |  | 960 | 2.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Timing advance adjustment accuracy

According to [5], The timing adjustment accuracy in existing requirements can be derived from minimum sampling rate which is equal to the product of FFT size and SCS (see Table 5). When oversampling is not used, the timing adjustment accuracy can be guaranteed to at most one sample.

Table 5: timing adjustment accuracy in existing requirements

| SCS (kHz) | minimum Bandwidth (MHz) | FFT size | minimum sampling rate (MHz) | Timing adjustment accuracy (+/-) |
| --- | --- | --- | --- | --- |
| 15 | 5 | 512 | 7.68 | 256Tc |
| 30 | 5 | 256 | 7.68 | 256Tc |
| 60 | 10 | 256 | 15.36 | 128Tc |
| 120 | 50 | 512 | 61.44 | 32Tc |

In [6], RAN4 agreed on the minimum channel bandwidths for NR operation in 52.6GHz to 71GHz as follow:

- For 120 kHz SCS, 100MHz

- For 480 kHz SCS, 400MHz

- For 960 kHz SCS, 400MHz

Based on the above discussion and analysis, we scale the timing adjustment accuracy requirements from 52.6GHz to 71GHz in Table 6.

Table 6: UE Timing Advance adjustment accuracy from 52.6GHz to 71GHz

| SCS (kHz) | minimum Bandwidth (MHz) | FFT size | minimum sampling rate (MHz) | Timing adjustment accuracy (+/-) |
| --- | --- | --- | --- | --- |
| 120 | 100 | 1024 | 122.88 | 16Tc |
| 480 | 400 | 1024 | 491.52 | 4Tc |
| 960 | 400 | 512 | 491.52 | 4Tc |

We combine the above result with the existing content in TS38.133 and the final UE Timing Advance adjustment accuracy can be shown in Table 7.

Proposal 4: The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 7.

Table 7: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | ±4 Tc | ±4 Tc |

Conclusion

Proposal 1: The uplink signal SCS should be no greater than SSB SCS for 52.6-71GHz.

Proposal 2: Since FR2-1 and FR2-2 is different in content, the initial transmission timing requirements for 24.25-52.6GHz and 52.6-71GHz need to be discussed separately.

Proposal 3: The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 4.

Table 4: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  | 480 | 120 | 2.75*64*Tc |
|  |  | 480 | 2.75*64*Tc |
|  | 960 | 120 | 2.5*64*Tc |
|  |  | 480 | 2.5*64*Tc |
|  |  | 960 | 2.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Proposal 4: The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 7.

Table 7: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | ±4 Tc | ±4 Tc |

- References

[1] R4-2108354

[2] R4-1711514

[3] R4-1801401

[4] RP-211584

[5] R4-1709758

[6] R4-2105410