3GPP TSG-RAN4 WG4 Meeting #100-e	R4-2112562

# Electronic meeting, 16 August – 27 August, 2021

Source: 	vivo

Title: 	Discussion on measurement gap interruption for 52.6-71GHz

Agenda Item:	9.16.7.5

Document for:	Discussion

Introduction

The item on impact of higher SCS on RRM requirements from 52.6GHz to 71GHz was discussed at RAN4 #99e. A WF [1] related to measurement gap was duplicated as below:

- Define new RRM requirements due to higher data/SSB SCS for at least the following topics: - Timing - UE transmit timing - Timing advance (TA) - Interruptions - Active BWP switching delay - Measurement gaps - Interruption time

Specifying new SCS, 480kHz and 960kHz, measurement gaps interruption time need to be considered. The existing requirements are specified as below [TS38.133].

The corresponding total number of interrupted slots on serving cells is listed in Table 9.1.2-4 for all serving cells in synchronous EN-DC, NR standalone (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and for serving cells in MCG in NR standalone operation (with asynchronous NR-DC configuration). The corresponding total number of interrupted slots on serving cells is listed in Table 9.1.2-4a for asynchronous EN-DC, and for serving cells in SCG in NR standalone operation (with asynchronous NR-DC configuration). Table 9.1.2-4: Total number of interrupted slots on all serving cells during MGL for Synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and on all serving cells in MCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1 NR; Total number of interrupted slots on serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.5ms is applied. (kHz); MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms; MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms. 15; 20; 10; 6; 4; 3; 21Note3; 11Note3; 7Note3; 5Note3; 4Note3. 30; 40; 20; 12; 8; 6; 40; 20; 12; 8; 6. 60; 80; 40; 24; 16; 12; 80; 40; 24; 16; 12. 120; 160; 80; 48; 32; 24; 160; 80; 48; 32; 24. NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation.. Table 9.1.2-4a: Total number of interrupted slots on serving cells during MGL for Asynchronous EN-DC, and on all serving cells in SCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1 NR; Total number of interrupted slots on serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.5ms is applied. (kHz); MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms; MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms. 15; 21; 11; 7; 5; 4; 21; 11; 7; 5; 4. 30; 41; 21; 13; 9; 7; 41; 21; 13; 9; 7. 60; 81; 41; 25; 17; 13; 81; 41; 25; 17; 13. 120; 161; 81; 49; 33; 25; 161; 81; 49; 33; 25. NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz is only applicable to the case with per-UE measurement gap.. In case that UE capable of per-FR measurement gap is configured with per-FR measurement gap for FR2 serving cells, total number of interrupted slots on FR2 serving cells during MGL is listed in Table9.1.2-4b. Table 9.1.2-4b: Total number of interrupted slots on FR2 serving cells during MGL for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC with per-UE measurement gap or per-FR measurement gap for FR2 NR; Total number of interrupted slots on FR2 serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.25ms is applied. (kHz); MGL= 20ms; MGL= 10ms; MGL= 5.5ms; MGL= 3.5ms; MGL= 1.5ms; MGL= 20ms; MGL= 10ms; MGL= 5.5ms; MGL= 3.5ms; MGL= 1.5ms. 60; 80; 40; 22; 14; 6; 80; 40; 22; 14; 6. 120; 160; 80; 44; 28; 12; 160; 80; 44; 28; 12. NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell..

In this paper, we review measurement gaps for NR and discuss the requirements for measurement gap interruption from 52.6GHz to 71GHz.

Discussion

In [2], it was mentioned that if the SMTC starting time is consistent with the MG starting time, some initial SSBs could be missed due to RF retuning time such as 0.5ms for FR1 and 0.25ms for FR2. Therefore, RAN4 discussed the measurement gap offset shift for efficient MG configuration and reach a consensus that MG timing advance is 0.5ms for FR1 and 0.25ms for FR2. There is no much difference for RF tuning time and we consider reusing 0.25ms as MG timing advance for 52.6-71GHz.

Proposal 1: RAN4 should reuse 0.25ms as MG timing advance for 52.6-71GHz.

For new SCS, 480kHz and 960kHz, the slot duration is 0.03125ms and 0.015625ms respectively. For the synchronous operation, in case that the MG timing advance of 0ms, 0.5ms or 0.25ms is applied, the values of interrupted slots are calculated as measurement gap length divided by slot duration. We add two rows for 480kHz and 960kHz in existing requirements. In order to allow the starting and ending point of interruption not to align with slot boundaries, asynchronous requirements are 1 slot longer than synchronous requirements.

For the case with per-UE gap or per-FR gap for FR1, NOTE 2 points out that NR SCS of 120 kHz is only applicable to the case with per-UE measurement gap in current requirements. When new SCS is introduced, NOTE 2 needs to be modified for 52.6-71GHz.

Proposal 2: RAN4 should modify NOTE 2 for the case with per-UE gap or per-FR gap for FR1 from 52.6GHz to 71GHz.

According to the above discussion and analysis, total number of interrupted slots on serving cells can be summarized for different operations as Table 1-3.

Proposal 3: Measurement gap interruption requirements shall be shown as following tables to support 480kHz and 960kHz.

Table 1: Total number of interrupted slots on all serving cells during MGL for Synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and on all serving cells in MCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 20 | 10 | 6 | 4 | 3 | 21Note3 | 11Note3 | 7Note3 | 5Note3 | 4Note3 |
| 30 | 40 | 20 | 12 | 8 | 6 | 40 | 20 | 12 | 8 | 6 |
| 60 | 80 | 40 | 24 | 16 | 12 | 80 | 40 | 24 | 16 | 12 |
| 120 | 160 | 80 | 48 | 32 | 24 | 160 | 80 | 48 | 32 | 24 |
| 480 | 640 | 320 | 192 | 128 | 96 | 640 | 320 | 192 | 128 | 96 |
| 960 | 1280 | 640 | 384 | 256 | 192 | 1280 | 640 | 384 | 256 | 192 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation. |  |  |  |  |  |  |  |  |  |  |

Table 2: Total number of interrupted slots on serving cells during MGL for Asynchronous EN-DC, and on all serving cells in SCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 21 | 11 | 7 | 5 | 4 | 21 | 11 | 7 | 5 | 4 |
| 30 | 41 | 21 | 13 | 9 | 7 | 41 | 21 | 13 | 9 | 7 |
| 60 | 81 | 41 | 25 | 17 | 13 | 81 | 41 | 25 | 17 | 13 |
| 120 | 161 | 81 | 49 | 33 | 25 | 161 | 81 | 49 | 33 | 25 |
| 480 | 641 | 321 | 193 | 129 | 97 | 641 | 321 | 193 | 129 | 97 |
| 960 | 1281 | 641 | 385 | 257 | 193 | 1281 | 641 | 385 | 257 | 193 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap. |  |  |  |  |  |  |  |  |  |  |

Table 3: Total number of interrupted slots on FR2 serving cells during MGL for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC with per-UE measurement gap or per-FR measurement gap for FR2

| NR | Total number of interrupted slots on FR2 serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.25ms is applied |  |  |  |  |
| (kHz) | MGL= 20ms | MGL= 10ms | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms | MGL= 20ms | MGL= 10ms | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms |
| 60 | 80 | 40 | 22 | 14 | 6 | 80 | 40 | 22 | 14 | 6 |
| 120 | 160 | 80 | 44 | 28 | 12 | 160 | 80 | 44 | 28 | 12 |
| 480 | 640 | 320 | 176 | 112 | 48 | 640 | 320 | 176 | 112 | 48 |
| 960 | 1280 | 640 | 352 | 224 | 96 | 1280 | 640 | 352 | 224 | 96 |
| NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell. |  |  |  |  |  |  |  |  |  |  |

Conclusion

Proposal 1: RAN4 should reuse 0.25ms as MG timing advance for 52.6-71GHz.

Proposal 2: RAN4 should modify NOTE 2 for the case with per-UE gap or per-FR gap for FR1 from 52.6GHz to 71GHz.

Proposal 3: Measurement gap interruption requirements shall be shown as Table 1-3 to support 480kHz and 960kHz.

- References

[1] R4-2108354

[2] R4-1802424