3GPP TSG-RAN WG4 Meeting #100-e	R4-2112581

# Elecrtronic Meeting,  Aug. 16 - 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | XXXX | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on additional UE co-ex requirements for 2 Band UL CA/DC for Japan(R16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SoftBank Corp., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Due to the clean-up of general/additional UE co-ex requirements, the additional requirements for 2 band UL UE co-ex were deleted and left uncovered for some time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The new section and table are added to make the requirements clear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relevant requirements remain missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.4.2/2A(New), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR38.521-3 ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Unaffected Portions Skipped]

6.5B.4	Additional spurious emissions

6.5B.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5B.4.1.1	Minimum requirement (network signalled value "NS_04")

When "NS 04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.1.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5B.4.1.1-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2495 ≤ f < 2496 | -13 | 1 % of Channel BW for contiguous BW up to 100 MHz, 1 MHz for contiguous BW > 100 MHz |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

#### 6.5B.4.2	Inter-band EN-DC within FR1

For inter-band EN-DC within FR1, the requirements in Table 6.5B.4.2-1 are specified in terms of an additional spectrum emission requirement. These requirements also apply for the frequency ranges that are less than FOOB (MHz) from the edge of the channel bandwidth specified in TS 36.101 [4] and TS 38.101-1 [2], respectively.

Table 6.5B.4.2A-1: Additional Requirements for inter-band EN-DC (two-bands)

| NR CA combination | Band | Applied_NS | Note |
| --- | --- | --- | --- |
| DC_1_n3 | 1 | 05 | 1 |
|  | n3 | 100 |  |
| DC_1_n5 | 1 | 05 | 1 |
|  | n5 | 100 |  |
| DC_1_n8 | 1 | 05 | 1 |
|  | n8 | 43/43U |  |
| DC_1_n28 | 1 | 05 | 2 |
|  | n28 | 17 |  |
| DC_1_n40 | 1 | 05 |  |
| DC_1_n41 | 1 | 05 |  |
|  | n41 | 47 |  |
| DC_1_n77 DC_1_n84_ULSUP-TDM_n77 | 1 | 05 | 1 |
|  | n84 | 05/05U |  |
| DC_1_n78 DC_1_n84_ULSUP-TDM_n78 | 1 | 05 | 1 |
|  | n84 | 05/05U |  |
| DC_1_n79 DC_1_n84_ULSUP-TDM_n79 | 1 | 05 | 1 |
|  | n84 | 05/05U |  |
| DC_3_n5 | n5 | 100 | 1 |
| DC_3_n8 | n8 | 43/43U | 1 |
| DC_3_n28 | n28 | 17 | 2 |
| DC_3_n41, DC_3_n80_ULSUP-TDM_n41 | n41 | 47 | 1 |
|  | n80 | 100 |  |
| DC_8_n1 | n1 | 05/05U | 1, 3 |
| DC_8_n3 | n3 | 100 | 1, 3 |
| DC_8_n28 | n28 | 17 | 2, 3 |
| DC_8_n41, DC_8_n81_ULSUP-TDM_n41 | n41 | 47 | 3 |
|  | n81 | 43/43U |  |
| DC_11_n3 | n3 | 100 | 1 |
| DC_11_n28 | n28 | 17 | 2 |
| DC_18_n3 | n3 | 100 | 1 |
| DC_19_n77 | 19 | 08 |  |
| DC_19_n78 | 19 | 08 |  |
| DC_19_n79 | 19 | 08 |  |
| DC_21_n77 | 21 | 09 |  |
| DC_21_n78 | 21 | 09 |  |
| DC_21_n79 | 21 | 09 |  |
| DC_28_n3 | n3 | 100 | 2 |
|  | 28 | 17 |  |
| DC_28_n5 | 28 | 17 | 1, 2 |
|  | n5 | 100 |  |
| DC_28_n8 | 28 | 17 | 1, 2 |
|  | n8 | 43/43U |  |
| DC_28_n40 | 28 | 17 | 2 |
| DC_28_n41 | 28 | 17 | 2 |
|  | n41 | 47 |  |
| DC_28_n77 | 28 | 17 | 2 |
| DC_28_n78 | 28 | 17 | 2 |
| DC_28_n79 | 28 | 17 | 2 |
| DC_40_n1 | n1 | 05/05U | 1 |
| DC_40_n41 | n41 | 47 |  |
| DC_41_n3 | n3 | 100 | 1 |
| DC_41_n28 | n28 | 17 | 2 |
| DC_42_n28 | n28 | 17 | 2 |
| NOTE 1: NS_05U, NS_43U and NS_100 can be signalled for NR bands that have UTRA services deployed and protected range is specified in clause 6.5.2.4.2 of TS38.101-1[2]. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 3: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. |  |  |  |

#### 6.5B.4.2A	Inter-band NE-DC within FR1

For inter-band NE-DC within FR1, the requirements in Table 6.5B.4.2A-1 are specified in terms of an additional spectrum emission requirement. These requirements also apply for the frequency ranges that are less than FOOB (MHz) from the edge of the channel bandwidth specified in TS 38.101-1 [2] and TS 36.101 [4], respectively.

Table 6.5B.4.2A-1: Additional Requirements for inter-band NE-DC (two-bands)

| NR CA combination | Band | Applied_NS | Note |
| --- | --- | --- | --- |
| DC_n1_28 | n1 | 05/05U | 1, 2 |
|  | 28 | 17 |  |
| NOTE 1: NS_05U can be signalled for NR bands that have UTRA services deployed and protected range is specified in clause 6.5.2.4.2 of TS38.101-1[2]. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |

[Unaffected Portions Skipped]