| 3GPP TR 36.717-03-02 V0.4.0 (2021-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Background	10

4.1	TR Maintenance	11

5	LTE-A inter-band CA for x bands DL with 2 bands UL: General part	11

5.1	UE RF part	11

5.1.1	UE general RF architecture	11

5.1.2	General treatment of ∆TIB and ∆RIB values	11

5.1.3	Fallback CA mode and mandatory support of all paired 2 UL CA configurations	11

5.1.4	General TX/RX requirements	11

5.1.4.1 Transmitter requirements	11

5.1.4.2 Receiver requirements	12

5.2	RRM part	16

6	LTE-A inter-band CA for 3 bands DL with 2 bands UL: Specific Band Combination Part	16

6.1	LTE-A inter-band CA: Band 1 and Band 8 and Band 38 DL with 2 bands UL	16

6.1.1	List of specific combination issues	16

6.1.1.1	Channel bandwidths per operating band for CA	16

6.1.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-3A-38A	17

6.1.1.3	MSD	18

6.1.1.4	∆TIB and ∆RIB values	18

6.2	LTE-A inter-band CA: Band 3 and Band 8 and Band 20 DL with 2 bands UL	18

6.2.1	List of specific combination issues	18

6.2.1.1	Channel bandwidths per operating band for CA	18

6.2.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-8A-20A	18

6.2.1.3	MSD	20

6.2.1.4	∆TIB and ∆RIB values	20

6.3	LTE-A inter-band CA: Band 1 and Band 8 and Band 20 DL with 2 bands UL	21

6.3.1	List of specific combination issues	21

6.3.1.1	Channel bandwidths per operating band for CA	21

6.3.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-8A-20A	21

6.3.1.3	MSD	22

6.3.1.4	∆TIB and ∆RIB values	22

6.4	 LTE-A inter-band CA: Band 1 and Band 20 and Band 38 DL with 2 bands UL	23

6.4.1	List of specific combination issues	23

6.4.1.1	Channel bandwidths per operating band for CA	23

6.4.1.2	Co-existence studies for LTE-A UL CA_1A-20A and DL CA_1A-20A-38A	23

6.4.1.3	MSD	24

6.4.1.4	∆TIB and ∆RIB values	24

6.5	 LTE-A inter-band CA: Band 3 and Band 20 and Band 38 DL with 2 bands UL	25

6.5.1	List of specific combination issues	25

6.5.1.1	Channel bandwidths per operating band for CA	25

6.5.1.2	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_3A-20A-38A	25

6.5.1.3	MSD	26

6.x.1.4	∆TIB and ∆RIB values	26

7	LTE-A inter-band CA for 4 bands DL with 2 bands UL: Specific Band Combination Part	27

7.1	LTE-A inter-band CA: Band 1 and Band 8 and Band 20 and Band 38 with 2 bands UL	27

7.1.1	List of specific combination issues	27

7.1.1.1	Channel bandwidths per operating band for CA	27

7.1.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-8A-20A-38A	27

7.1.1.3	MSD	28

7.1.1.4	∆TIB and ∆RIB values	28

7.2	LTE-A inter-band CA: Band 1 and Band 3 and Band 20 and Band 38 DL with 2 bands UL	29

7.2.1	List of specific combination issues	29

7.2.1.1	Channel bandwidths per operating band for CA	29

7.2.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-20A-38A	29

7.2.1.3	MSD	30

7.2.1.4	∆TIB and ∆RIB values	30

7.3	LTE-A inter-band CA: Band 1 and Band 3 and Band 7 and Band 38 DL with 2 bands UL	31

7.3.1	List of specific combination issues	31

7.3.1.1	Channel bandwidths per operating band for CA	31

7.3.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7A-38A	31

7.3.1.3	MSD	32

7.3.1.4	∆TIB and ∆RIB values	32

7.4	LTE-A inter-band CA: Band 1 and Band 3 and Band 8 and Band 20 DL with 2 bands UL	32

7.4.1	List of specific combination issues	32

7.4.1.1	Channel bandwidths per operating band for CA	32

7.4.1.2	Co-existence studies for LTE-A UL CA_1A-3A ,UL CA_1A-8A ,UL CA_3A-8A and DL CA_1A-3A-8A-20A	33

7.4.1.3	MSD	37

7.4.1.4	∆TIB and ∆RIB values	37

7.5	LTE-A inter-band CA: Band 1 and Band 7 and Band 8 and Band 38 DL with 2 bands UL	37

7.5.1	List of specific combination issues	37

7.5.1.1	Channel bandwidths per operating band for CA	37

7.5.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-7A-8A-38A	37

7.5.1.3	MSD	38

7.5.1.4	∆TIB and ∆RIB values	38

7.6	LTE-A inter-band CA: Band 3 and Band 7 and Band 8 and Band 38 DL with 2 bands UL	39

7.6.1	List of specific combination issues	39

7.6.1.1	Channel bandwidths per operating band for CA	39

7.6.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-7A-8A-38A	39

7.6.1.3	MSD	40

7.6.1.4	∆TIB and ∆RIB values	40

7.7	LTE-A inter-band CA: Band 3 and Band 8 and Band 20 and Band 38 DL with 2 bands UL	40

7.7.1	List of specific combination issues	40

7.7.1.1	Channel bandwidths per operating band for CA	40

7.7.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-8A-20A-38A	41

7.7.1.3	MSD	42

7.7.1.4	∆TIB and ∆RIB values	42

7.8	  LTE-A inter-band CA: Band 1 and Band 3 and Band 20 and Band 38 with 2 bands UL	42

7.8.1	List of specific combination issues	42

7.8.1.1	Channel bandwidths per operating band for CA	42

7.8.1.2	Co-existence studies for LTE-A inter-band UL CA_1A-20A and DL CA_1A-3A-20A-38A	43

7.8.1.3	Co-existence studies for LTE-A inter-band UL CA_3A-20A and DL CA_1A-3A-20A-38A	44

7.8.1.4	MSD	45

7.8.1.5	∆TIB and ∆RIB values	45

7.9	 LTE-A inter-band CA: Band 1 and Band 7 and Band 20 and Band 38 with 2 bands UL	45

7.9.1	List of specific combination issues	45

7.9.1.1	Channel bandwidths per operating band for CA	45

7.9.1.2	Co-existence studies for LTE-A UL CA_1A-20A and DL CA_1A-7A-20A-38A	45

7.9.1.3	 MSD	45

7.9.1.4	 ∆TIB and ∆RIB values	46

8	LTE-A inter-band CA for 5 bands DL with 2 bands UL: Specific Band Combination Part	46

8.1	LTE-A inter-band CA: Band 1 and Band 3 and Band 8 and Band 8 and Band 38 DL with 2 bands UL	46

8.1.1	List of specific combination issues	46

8.1.1.1	Channel bandwidths per operating band for CA	46

8.1.1.2	Co-existence studies for LTE-A UL CA_1A-3A, UL CA_1A-8A, UL CA_3A-8A and DL CA_1A-3A-7A-8A-38A	46

8.1.1.3	MSD	50

8.1.1.4	∆TIB and ∆RIB values	50

8.2	LTE-A inter-band CA: Band 1 and Band 3 and Band 8 and Band 20 and Band 38 DL with 2 bands UL	51

8.2.1	List of specific combination issues	51

8.2.1.1	Channel bandwidths per operating band for CA	51

8.2.1.2	Co-existence studies for LTE-A UL CA_1A-3A ,UL CA_1A-8A ,UL CA_3A-8A and DL CA_1A-3A-8A-20A-38A	51

8.2.1.3	MSD	55

8.2.1.4	∆TIB and ∆RIB values	55

Annex A: Change history	56

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for LTE-Advanced inter-band carrier aggregation for x bands DL (x=3, 4, 5) with 2 bands UL under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address Rel-17 band combinations that are related to x bands (x=3, 4, 5) DL with 2 bands UL CA requirements in Table 1-1, 1-2, and 1-3.

Table 1-1: 3 bands DL/2 bands UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configuration | E-UTRA Bands | 1.4MHz | 3 MHz | 5 MHz | 10 MH z | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3C-7A-8A | CA_3A-8A | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3C-38A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-8A-38A | CA_3A-8A | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C-20A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-8A-20A | CA_3A-8A | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-8A-20A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-8A-20A | CA_3A-8A | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-8A-38A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

Table 1-2: 4 bands DL/2 bands UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configuration | E-UTRA Bands | 1.4MHz | 3 MHz | 5 MH z | 10 MH z | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3C-7A-8A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3C-8A-38A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-8A-20A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C-8A-20A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-20A-38A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C-20A-38A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-7A-38A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-7A-8A-38A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-7A-8A-38A | CA_3A-8A | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-8A-20A-38A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-8A-20A-38A | CA_3A-8A | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

Table 1-3: 5 bands DL/2 bands UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configuration | E-UTRA Bands | 1.4MHz | 3 MHz | 5 MH z | 10 MH z | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A_3A_7A_8A_38A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A_3A_8A_20A_38A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

This TR contains a general part and band specific combination part and the actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[4]		RP-201247, “New WID on Rel-17 LTE-A inter-band CA for x bands (x=3, 4, 5) DL with 2 bands UL”, LG Electronics”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

Mixed intra-band and inter-band carrier aggregation: Carrier aggregation of component carriers in same operating band and different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

4	Background

The present document is a technical report for LTE-A inter-band CA for x bands (x=3, 4, 5) DL with 2 bands UL under Rel-17 time frame and the document is divided into two different parts:

- 	General part: this part covers some common issues of x bands (x=3, 4, 5) DL with 2 bands UL regardless of band combinations

- 	Specific band combination part:  this part covers each band combination of x bands (x=3, 4, 5) DL with 2 bands UL and its own specific issues independently from each other (i.e. one sub-clause is defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	LTE-A inter-band CA for x bands DL with 2 bands UL: General part

5.1	UE RF part

5.1.1	UE general RF architecture

The UE RF architecture of the general part in TR 36.716-03-02 (Rel-16) can be reused to define Maximum Sensitivity Degradation (MSD) for LTE-A inter-band CA for x bands (x = 3, 4, 5) DL with 2 bands in Rel-17. This UE general RF architecture is for TDD-FDD Carrier Aggregation (CA) band combinations in Rel-17 and it only applies to LTE. Non-Standalone (NSA) DC/CA operation isn’t considered in this technical report.

Figure 5.1.1- 1 Example of UE RF architecture using cascaded diplexer

5.1.2	General treatment of ∆TIB and ∆RIB values

Additional insertion loss from LTE-A inter-band CA for x bands DL / 1 UL can be reused when new RF components are not necessary to be introduced. If there will be a need for introducing the new RF components, then more detail information shall be provided for a certain band combination.

5.1.3	Fallback CA mode and mandatory support of all paired 2 UL CA configurations

Based on the agreed Way forward (R4-168853) in Rel-14, fallback CA mode has been defined by RAN4.

5.1.4	General TX/RX requirements

5.1.4.1 Transmitter requirements

For dual uplink inter-band CA operation, transmitter requirements have been analyzed in TR36.860 and UEs supporting LTE-A inter-band CA for x bands (x= 3, 4, 5) DL with 2 bands UL can apply the common Tx requirements in Table 5.1.4.1-1.

Table 5.1.4.1-1: UE RF TX requirements for dual uplink inter-band CA

| Section / Clause in TS36.101 | Description |
| --- | --- |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.5A | Configured transmitted Power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.3 | Adjacent Channel Leakage Ratio |
| 6.6.3.1A | Spurious Emission for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 6.7.1A | Transmit intermodulation for CA |

5.1.4.2 Receiver requirements

Like the transmitter requirements in the section 5.1.4.1, receiver requirements have been analysed in TR36.860 for dual uplink inter-band CA operation and additional requirements of UE RF receiver for LTE-A inter-band CA for x bands (x = 3, 4, 5) DL with 2 bands UL can be found in Table 5.1.4.2-1. For some band combinations that are related to x bands (x = 3, 4, 5) DL with 2 bands UL without self-interference problem, the same requirements of x bands DL/ 1 bands UL CA can be reused. The requirements (out-of-band blocking and spurious response) of 2 bands DL / 2 bands UL can be covered by applying the test method of one uplink (i.e. single carrier LTE and 2 bands DL / 1 band UL CA. Therefore, there will be no required to test the requirements (out-of-band blocking and spurious response) for x bands DL with 2 bands UL. Only REFSENS test will be required when all x bands DL and 2 bands UL are activated at the same time. Lastly, a new MSD value shall be defined for a band combination which has self-interference problem when two uplinks are activated.

Table 5.1.4.2-1: UE RF Rx requirements for dual uplink inter-band CA

| Section / Clause in TS36.101 | Description | Comments |
| --- | --- | --- |
| 7.3.1A | Reference sensitivity for CA | Need to be tested for all x bands DL/ 2 bands UL |
| 7.6.2.1A | Out-of-band blocking for CA | No need to be tested for x bands DL/ 2 bands UL |
| 7.7.1A | Spurious response for CA | No need to be tested for x bands DL/ 2 bands UL |

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes a result of self-interference studies of proposed LTE-A inter-band CA combination for 3 bands DL with 2 bands UL in Rel-17.

Table 5.1.5-1: Summary of the self-interference studies for 3 bands DL with 2 bands UL

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | IMD to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3C-7A-8A | CA_3A-8A | 3rd harmonic | 2nd and 3rd IMDs | - | No need to study for harmonic and IMD impacts since they have been already analyzed in TS36.101. |
| CA_1A-3C-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_3C-8A-38A | CA_3A-8A | - | 2nd and 3rd IMDs | - | No need to study for IMD impacts since they have been already analyzed in TS36.101. |
| CA_1A-3C-20A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_3A-8A-20A CA_3C-8A-20A | CA_3A-8A | - | 2nd , 4th and 5th IMDs | - | MSD (27dB) for 2nd IMD can follow from DC_3A-20A_n8A in TS38.101-3 |
| CA_1A-8A-20A | CA_1A-8A | - | 4th IMD | - | MSD (11.5dB) for 4th IMD can follow from DC_1A-20A_n8A in TS38.101-3 |
| CA_1A-8A-38A | CA_1A-8A | - | - | - | - |
| CA_1A-20A-38A | CA_1A-20A | 3rd harmonic from B20 fall into B38 | - | - | Harmonic issue already solved in CA_20A-38A |
| CA_3A-20A-38A | CA_3A-20A | 3rd harmonic from B20 fall into B38 | 2nd & 3rd IMDs | - | - Harmonic issue already solved in CA_20A-38A - 28.4dB MSD level for 2nd IMD by UL_CA_3A-20A which was proposed in R4-2112925 |

Table 5.1.5-2 summarizes a result of self-interference studies of proposed LTE-A inter-band CA combination for 4 bands DL with 2 bands UL in Rel-17.

Table 5.1.5-2: Summary of the self-interference studies for 4 bands DL with 2 bands UL

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3C-7A-8A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 and B7 | - | - | No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-8A | 3rd harmonic impact from B8 to B7 | 2nd and 3rd IMDs in B7 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - 2nd and 3rd IMD problems have been already covered in CA_3A-7A-8A/CA_3A-8A. |
| CA_1A-3C-8A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 | - | - | No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-8A | - | 2nd and 3rd IMDs in B38 | - | 2nd and 3rd IMD problems have been already covered in CA_3A-8A-38A/CA_3A-8A. |
| CA_1A-3A-8A-20A, CA_1A-3C-8A-20A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 | 4th IMD in B20 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-8A | - | 2nd and 5th IMDs in B20 | - | The MSD will study for IMD impacts in in lower order DL_CA_3A-8A-20A with UL_CA_3A-8A |
| CA_1A-3A-20A-38A, CA_1A-3C-20A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_1A-3A-7A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_1A-7A-8A-38A | CA_1A-8A | 3rd harmonic impact from B8 to B7 | - | - | No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
| CA_3A-7A-8A-38A | CA_3A-8A | 3rd harmonic impact from B8 to B7 | 2nd and 3rd IMDs in B7, 2nd and 3rd IMDS in B38 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - All IMD problems have been already covered in CA_3A-7A-8A/CA_3A-8A and CA_3A-8A-38A/CA_3A-8A. |
| CA_1A-8A-20A-38A | CA_1A-8A | - | 4th IMD in B20 | - | The MSD will be covered in lower order DL_CA_1A-8A-20A with UL_CA_1A-8A |
| CA_3A-8A-20A-38A | CA_3A-8A | - | 2nd and 5th IMDs in B20, 2nd and 3rd IMDs in B38 | - | - The MSD for 2nd and 5th IMD problems will be covered in lower order DL_CA_3A-8A-20A with UL_CA_3A-8A. - 2nd and 3rd IMD problems have been already covered in CA_3A-8A-38A/CA_3A-8A. |
| CA_3A-7A-20A-38A | CA_3A-20A | 3rd harmonic from B20 into B38 | 2nd IMD into B7 & B38, 3rd IMD into B7 & B38 | - | - The 2nd IMD already covered in DL_CA_3A-7A-20A with UL_CA_3A-20A. Also follow same MSD of DL_CA_3A-7A-20A for CA_3A-20A-38A |
| CA_1A-7A-20A-38A | CA_1A-20A | 3rd harmonic from B20 into B38 | - | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. |

Table 5.1.5-3 summarizes a result of self-interference studies of proposed LTE-A inter-band CA combination for 5 bands DL with 2 bands UL in Rel-17.

Table 5.1.5-3: Summary of the self-interference studies for 5 bands DL with 2 bands UL

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | IMD to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3A-7A-8A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 and 3rd harmonic impact from B8 to B7 | - | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-8A | 3rd harmonic impact from B8 to B7 | 2nd IMD in B7, 3rd IMD in B38 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - All IMD problems have been already covered in CA_3A-7A-8A/CA_3A-8A and CA_3A-8A-38A/CA_3A-8A. |
| CA_1A-3A-8A-20A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 | 4th IMD in B20 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - MSD for IMD impacts will be covered in DL_CA_1A-8A-20A with UL_CA_1A-8A. |
|  | CA_3A-8A | - | 2nd and 5th IMDs in B20, 2nd and 3rd IMDs in B38, | - | - 2nd and 5th IMD problems for CA_3A-8A-20A/CA_3A-8A need to be studied. - 2nd and 3rd IMD problems have been already covered in CA_3A-8A-38A/CA_3A-8A. |
| CA_1A-3A-7A-20A-38A | CA_1A-3A |  | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-20A | 3rd harmonic from B20 into B38 | - | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-20A | 3rd harmonic from B20 into B38 | 2nd & 3rd IMD into B7 & B38 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - These IMD issues are already covered in lower fallback mode. |

According to Table 5.1.5.1, 5.1.5.2, and 5.1.5.3, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3 bands DL with 2 bands UL CA combinations that have some IMD issues.

Table 5.1.5.4 REFSENS exception and test configuration for proposed 3 bands DL with 2 bands UL CA combination in Rel-17

| E-UTRA band/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA DL Configuration | E-UTRA CA UL Configuration | E-UTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_1A-8A-20A | CA_1A-8A | 1 | 1925 | 5 | 25 | 2115 | 5 | N/A | FDD | N/A |
|  |  | 8 | 910 | 5 | 25 | 955 | 5 |  |  |  |
|  |  | 20 | 846 | 5 | 25 | 805 | 5 | 11.5 |  | IMD4 |
| CA_3A-8A-20A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 8 | 910 | 5 | 25 | 955 | 5 |  |  |  |
|  |  | 20 | 851 | 5 | 25 | 810 | 5 | 27.0 |  | IMD2 |

5.2	RRM part

[Editor Note] It will be updated in future

6	LTE-A inter-band CA for 3 bands DL with 2 bands UL: Specific Band Combination Part

6.1	LTE-A inter-band CA: Band 1 and Band 8 and Band 38 DL with 2 bands UL

6.1.1	List of specific combination issues

6.1.1.1	Channel bandwidths per operating band for CA

Table 6.1.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-8A-38A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

6.1.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-3A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.1.2-1

Table 6.1.1.2-1: Harmonic and IMD analysis for UL_CA_1A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on Table 6.1.1.2-1, no IMD products by Band 1 and Band 8 falls into the own Rx frequency of Band 38.

6.1.1.3	MSD

When uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-8A-38A, there is no MSD requirements

6.1.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

6.2	LTE-A inter-band CA: Band 3 and Band 8 and Band 20 DL with 2 bands UL

6.2.1	List of specific combination issues

6.2.1.1	Channel bandwidths per operating band for CA

Table 6.2.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-8A-20A | CA_3A-8A | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-8A-20A | CA_3A-8A | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

6.2.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-8A-20A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.2.1.2-1

Table 6.2.1.2-1: Harmonic and IMD analysis for UL_CA_3A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

Based on Table 6.2.1.2-1, the 2nd harmonics of Band 8, 4th & 5th IMDs fall into the own Rx frequency of Band 3. And also the 4th & 5th IMDs fall into the own Rx frequency of Band 8. The 5th IMD falls into the own Rx frequency of Band 20.

6.2.1.3	MSD

When uplink CA_3A_8A is paired with downlink CA_3A-8A-20A, the 2nd order IMD and 5th order IMD product by band 3 and band 8 falls into the own Rx frequency band 20. The MSD for 2nd IMD can be reused from DC_3A-20A_n8A. The 5th IMD do not defined in TS36.101.

Table 6.2.1.3-1: MSD summary for 2UL 3DL CA_3A-8A-20A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |  | Source of IMD |
| CA_3A-8A-20A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | IMD2 |  |
|  |  | 8 | 910 | 5 | 25 | 955 | 5 |  |  |  |  |
|  |  | 20 | 851 | 5 | 25 | 810 | 5 | 27.0 |  |  |  |

When uplink CA_3A_8A is paired with downlink CA_3A-8A-20A, the 4th & 5th order IMD product by band 3 and band 8 falls into the own Rx frequency band 3 and band 8. It has been solved by fall-back combination CA_3A_8A.

6.2.1.4	∆TIB and ∆RIB values

For CA_3A-8A-20A, the TIB,c and RIB,c values are shown in table 6.2.1.4-1 and in table 6.2.1.4-2.

Table 6.2.1.4-1: TIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-8A-20A | 3 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |

Table 6.2.1.4-2: RIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3A-8A-20A | 3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |

6.3	LTE-A inter-band CA: Band 1 and Band 8 and Band 20 DL with 2 bands UL

6.3.1	List of specific combination issues

6.3.1.1	Channel bandwidths per operating band for CA

Table 6.3.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-8A-20A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

6.3.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-8A-20A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.3.1.2-1

Table 6.3.1.2-1: Harmonic and IMD analysis for UL_CA_1A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on Table 6.3.1.2-1, 4th IMD products by Band 1 and Band 8 falls into the own Rx frequency of Band 1 and Band 20.

6.3.1.3	MSD

When uplink CA_1A_8A is paired with downlink CA_1A-8A-20A, the 4th order IMD product by band 1 and band 8 falls into the own Rx frequency band 20. The MSD for 4th IMD can be used from DC_1A-20A_n8A.

Table 6.3.1.3-1: MSD summary for 2UL 3DL CA_1A-8A-20A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_1A-8A-20A | CA_1A-8A | 1 | 1925 | 5 | 25 | 2115 | 5 | N/A | FDD | IMD4 |
|  |  | 8 | 910 | 5 | 25 | 955 | 5 |  |  |  |
|  |  | 20 | 846 | 5 | 25 | 805 | 5 | 11.5 |  |  |

When uplink CA_1A_8A is paired with downlink CA_1A-8A-20A, the 4th order IMD product by band 1 and band 8 falls into the own Rx frequency band 1. It has been solved by fallback combination CA_1A_8A.

6.3.1.4	∆TIB and ∆RIB values

For CA_1A-8A-20A, the TIB,c and RIB,c values are shown in table 6.3.1.4-1 and in table 6.3.1.4-2.

Table 6.3.1.4-1: TIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-20A | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 20 | 0.4 |

Table 6.3.1.4-2: RIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-20A | 1 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |

  - 6.4		LTE-A inter-band CA: Band 1 and Band 20 and Band 38 DL with 2 bands UL

    - 6.4.1	List of specific combination issues

      - 6.4.1.1	Channel bandwidths per operating band for CA

Table 6.4.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-20A-38A | CA_1A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 6.4.1.2	Co-existence studies for LTE-A UL CA_1A-20A and DL CA_1A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.4.1.2-1

Table 6.4.1.2-1: Harmonic and IMD analysis for UL_CA_1A-20A

| UL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 1920 | 1980 | 832 | 862 |
| 2nd harmonic products | 3840 | 3960 | 1664 | 1724 |
| 3rd harmonic products | 5760 | 5940 | 2496 | 2586 |
| 4th harmonic products | 7680 | 7920 | 3328 | 3448 |
| 5th harmonic products | 9600 | 9900 | 4160 | 4310 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1148 | 1058 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 | 3128 | 316 | 196 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 | 4822 | 3584 | 3704 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 | 5108 | 516 | 666 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 | 6802 | 4416 | 4506 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 | 1348 | 7088 | 6818 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 | 5428 | 8512 | 8782 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 | 1464 | 4276 | 4036 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6546 | 7424 | 7664 |

Based on Table 6.4.1.2-1:

- no IMD products by Band 1 and Band 20  falls into the own Rx frequency of Band 38.

- the 3rd harmonic of band 20 falls into the own Rx frequency of Band 38.

      - 6.4.1.3	MSD

According to the co-existence studies as above, there is no MSD requirements caused by IMD when uplink CA configurations CA_1A-20A is paired with downlink CA configuration CA_1A-20A-38A.

For the harmonic MSD requirements, it have already been solved in the fallback combination of CA_20A-38A. Thus there are no additional MSD requirement for the uplink CA configurations CA_1A-20A paired with downlink CA configuration CA_1A-20A-38A.

      - 6.4.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

  - 6.5		LTE-A inter-band CA: Band 3 and Band 20 and Band 38 DL with 2 bands UL

    - 6.5.1	List of specific combination issues

      - 6.5.1.1	Channel bandwidths per operating band for CA

Table 6.5.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-20A-38A | CA_3A-20A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 6.5.1.2	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_3A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.5.1.2-1

Table 6.5.1.2-1: Harmonic and IMD analysis for UL_CA_3A-20A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1785 | 832 | 862 |
| 2nd order harmonics frequency range (MHz) | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd order harmonics frequency range (MHz) | 3420 | 3570 | 1664 | 1724 |
| 3rd order harmonics frequency range (MHz) | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd order harmonics frequency range (MHz) | 5130 | 5355 | 2496 | 2586 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 953 | 848 | 2542 | 2647 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 | 2738 | 121 | 14 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 | 4432 | 3374 | 3509 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1696 | 1906 | 5084 | 5294 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 | 4523 | 711 | 876 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 | 6217 | 4206 | 4371 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 | 1543 | 6308 | 5978 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 | 5233 | 7672 | 8002 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 834 | 1074 | 3691 | 3406 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6156 | 6794 | 7079 |

Based on Table 6.5.1.2-1:

- IMD2, IMD3 produce caused by  CA_3_20 uplink may fall into own Rx of band 38.

      - 6.5.1.3	MSD

According to co-existence studies, both IMD2 and IMD3 may fall into the Rx of band 38. Here we only define MSD caused by IMD2 for this band combination in table 6.5.1.3-1. For MSD caused by IMD3, Note 1 in the table is specified which is already existed in the Table 7.3.1A-0g in TS36.101-1.

Table 6.5.1.3-1: MSD summary for 2UL/3DL CA_3A-20A-38A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_3A-20A-38A | CA_3A-20A | 3 | 1760 | 5 | 25 | 1855 | 5 | N/A | FDD | N/A |
|  |  | 20 | 850 | 5 | 25 | 809 | 5 | N/A | FDD | N/A |
|  |  | 38 | 2610 | 5 | 25 | 2610 | 5 | 28.4 | TDD | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

      - 6.x.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

7	LTE-A inter-band CA for 4 bands DL with 2 bands UL: Specific Band Combination Part

7.1	LTE-A inter-band CA: Band 1 and Band 8 and Band 20 and Band 38 with 2 bands UL

7.1.1	List of specific combination issues

7.1.1.1	Channel bandwidths per operating band for CA

Table 7.1.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-8A-20A-38A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

7.1.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-8A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.2-1

Table 7.1.1.2-1: Harmonic and IMD analysis for UL_CA_1A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on Table 7.1.1.2-1, 4th IMD products by Band 1 and Band 8 falls into the own Rx frequency of Band 1 and Band 20.

7.1.1.3	MSD

When uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-8A-20A-38A there are no additional analysis of MSD because it is solved in fallback band combination.

7.1.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

7.2	LTE-A inter-band CA: Band 1 and Band 3 and Band 20 and Band 38 DL with 2 bands UL

7.2.1	List of specific combination issues

7.2.1.1	Channel bandwidths per operating band for CA

Table 7.2.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-20A-38A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C-20A-38A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

7.2.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.2.1.2-1

Table 7.2.1.2-1: Harmonic and IMD analysis for UL_CA_1A-3A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Based on Table 7.2.1.2-1, the 3rd IMD will interfere own Rx frequency of Band 1. There are no harmonic or IMD products from band 1 and band 3 uplinks which would desensitize band 20 and band 38 downlink.

7.2.1.3	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-20A-38A there are no interference components from 2 uplink operation which would interfere the downlink of the third band, the 3rd IMD will interfere own Rx frequency of Band 1 and it has been solved by fallback combination CA_1A_3A.

7.2.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

7.3	LTE-A inter-band CA: Band 1 and Band 3 and Band 7 and Band 38 DL with 2 bands UL

7.3.1	List of specific combination issues

7.3.1.1	Channel bandwidths per operating band for CA

Table 7.3.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-38A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

7.3.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.3.1.2-1

Table 7.3.1.2-1: Harmonic and IMD analysis for UL_CA_1A-3A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Based on Table 7.3.1.2-1, the 3rd IMD will interfere own Rx frequency of Band 1. There are no harmonic or IMD products from band 1 and band 3 uplinks which would desensitize band 7 and band 38 downlink.

7.3.1.3	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-7A-38A there are no interference components from 2 uplink operation which would interfere the downlink of the third band, the 3rd IMD will interfere own Rx frequency of Band 1 and it has been solved by fallback combination CA_1A_3A.

7.3.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

7.4	LTE-A inter-band CA: Band 1 and Band 3 and Band 8 and Band 20 DL with 2 bands UL

7.4.1	List of specific combination issues

7.4.1.1	Channel bandwidths per operating band for CA

Table 7.4.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-8A-20A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C-8A-20A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

7.4.1.2	Co-existence studies for LTE-A UL CA_1A-3A ,UL CA_1A-8A ,UL CA_3A-8A and DL CA_1A-3A-8A-20A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.4.1.2-1, Table 7.4.1.2-2 and Table 7.4.1.2-3.

Table 7.4.1.2-1: Harmonic and IMD analysis for UL_CA_1A-3A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Based on Table 7.4.1.2-1, the 3rd IMD products by Band 1 and Band 3  will interfere own Rx frequency of Band 1. There are no harmonic or IMD products from Band 1 and Band 3 uplinks which would desensitize Band 8 and Band 20 downlink.

Table 7.4.1.2-2: Harmonic and IMD analysis UL_CA_1A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on Table 7.4.1.2-2, 4th IMD products by Band 1 and Band 8 falls into the own Rx frequency of Band 1 and Band 20,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3.

Table 7.4.1.2-3: Harmonic and IMD analysis UL_CA_3A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

Based on Table 7.4.1.2-3, 2nd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 20, and 5th IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 20,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3.

7.4.1.3	MSD

When uplink CA configurations CA_1A-3A , CA_1A-8A and CA_3A-8A is paired with downlink CA configuration CA_1A-3A-8A-20A there is no additional analysis of MSD because it is solved in fallback band combination.

7.4.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

7.5	LTE-A inter-band CA: Band 1 and Band 7 and Band 8 and Band 38 DL with 2 bands UL

7.5.1	List of specific combination issues

7.5.1.1	Channel bandwidths per operating band for CA

Table 7.5.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7A-8A-38A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

7.5.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-7A-8A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.5.1.2-1

Table 7.5.1.2-1: Harmonic and IMD analysis for UL_CA_1A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on Table 7.5.1.2-1, 4th IMD products by Band 1 and Band 8 falls into the own Rx frequency of Band 1, and the 3nd harmonic from Band 8 falls into Rx frequency of Band 7.

7.5.1.3	MSD

When uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-7A-8A-38A there are no additional analysis of MSD because it is solved in fallback band combination.

7.5.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

7.6	LTE-A inter-band CA: Band 3 and Band 7 and Band 8 and Band 38 DL with 2 bands UL

7.6.1	List of specific combination issues

7.6.1.1	Channel bandwidths per operating band for CA

Table 7.6.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-7A-8A-38A | CA_3A-8A | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

7.6.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-7A-8A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.6.1.2-1

Table 7.6.1.2-1: Harmonic and IMD analysis for UL_CA_3A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

Based on Table 7.6.1.2-1, 2nd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 7,and 3th IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 38,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3, and the 3nd harmonic from Band 8 falls into Rx frequency of Band 7.

7.6.1.3	MSD

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3A-7A-8A-38A there is no additional analysis of MSD because it is solved in fallback band combination.

7.6.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

7.7	LTE-A inter-band CA: Band 3 and Band 8 and Band 20 and Band 38 DL with 2 bands UL

7.7.1	List of specific combination issues

7.7.1.1	Channel bandwidths per operating band for CA

Table 7.7.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-8A-20A-38A | CA_3A-8A | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

7.7.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-8A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.7.1.2-1

Table 7.7.1.2-1: Harmonic and IMD analysis for UL_CA_3A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

Based on Table 7.7.1.2-1, 2nd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 20 and Band 38,and 3rd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 38, and 5th IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 20,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3.

7.7.1.3	MSD

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3A-8A-20A-38A there is no additional analysis of MSD because it is solved in fallback band combination.

7.7.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

  - 7.8			LTE-A inter-band CA: Band 1 and Band 3 and Band 20 and Band 38 with 2 bands UL

    - 7.8.1	List of specific combination issues

      - 7.8.1.1	Channel bandwidths per operating band for CA

Table 7.8.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-20A-38A | CA_1A-20A CA_3A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 7.8.1.2	Co-existence studies for LTE-A inter-band UL CA_1A-20A and DL CA_1A-3A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.8.1.2-1

Table 7.8.1.2-1: Harmonic and IMD analysis for UL_CA_1A-20A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2496 | 2586 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 | 1058 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 | 3128 | 316 | 196 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 | 4822 | 3584 | 3704 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4898 | 5108 | 516 | 666 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6592 | 6802 | 4416 | 4566 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 | 1348 | 7088 | 6818 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 | 5428 | 8512 | 8782 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 | 1464 | 4276 | 4036 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6546 | 7424 | 7664 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 | 2000 | 812 | 882 |

Based on Table 7.8.1.2-1:

-There is no harmonics and IMDs products fall into own Rx of Band 3.

-3rd harmonic of band 20 may fall into own Rx of Band 38, and  no IMDs products fall into own Rx of Band 38.

      - 7.8.1.3	Co-existence studies for LTE-A inter-band UL CA_3A-20A and DL CA_1A-3A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.8.1.3-1

Table 7.8.1.3-1: Harmonic and IMD analysis for UL_CA_3A-20A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1785 | 832 | 862 |
| 2nd order harmonics frequency range (MHz) | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd order harmonics frequency range (MHz) | 3420 | 3570 | 1664 | 1724 |
| 3rd order harmonics frequency range (MHz) | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd order harmonics frequency range (MHz) | 5130 | 5355 | 2496 | 2586 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 953 | 848 | 2542 | 2647 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 | 2738 | 121 | 14 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 | 4432 | 3374 | 3509 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1696 | 1906 | 5084 | 5294 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 | 4523 | 711 | 876 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 | 6217 | 4206 | 4371 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 | 1543 | 6308 | 5978 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 | 5233 | 7672 | 8002 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 834 | 1074 | 3691 | 3406 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6156 | 6794 | 7079 |

Based on Table 7.8.1.3-1:

-There is no harmonics and IMDs products fall into own Rx of Band 1.

-3rd harmonic of band 20 may fall into own Rx of Band 38 and 2rd IMDs products fall into own Rx of Band 38.

      - 7.8.1.4	MSD

According to the co-existence study, some MSD issue needs to be considered. However, for harmonic MSD, it have already been captured in 1UL/2DL, and for IMD MSD, it also have been captured in the corresponding fallback 2UL/3DL configuration, which means there is no need to consider additional MSD requirements for this combination.

      - 7.8.1.5	∆TIB and ∆RIB values

The same requirements of fallback combination can be applied.

  - 7.9		LTE-A inter-band CA: Band 1 and Band 7 and Band 20 and Band 38 with 2 bands UL

    - 7.9.1	List of specific combination issues

      - 7.9.1.1	Channel bandwidths per operating band for CA

Table 7.9.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7A-20A-38A | CA_1A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 7.9.1.2	Co-existence studies for LTE-A UL CA_1A-20A and DL CA_1A-7A-20A-38A

For 2UL/4DL LTE CA band combination, the additional MSD caused by the intermodulation products of two ULs from CA_1A-20A falls into band 7 or band 38 would be defined. However, such MSD are discussed in the corresponding fall back 2UL/3DL LTE CA band combination of CA_1A-7A-20A and CA_1A-20A-38A, where no intermodulation products of two ULs from CA_1A-20A falls into band 7 or band 38.

      - 7.9.1.3		MSD

According to the co-existence studies analysis, when uplink CA configurations CA_1A-20A is paired with downlink CA configuration CA_1A-7A-20A-38A there are no additional analysis of MSD because they are solved in fallback band combination.

      - 7.9.1.4		∆TIB and ∆RIB values

The same requirements of lower order combination can be applied.

8	LTE-A inter-band CA for 5 bands DL with 2 bands UL: Specific Band Combination Part

8.1	LTE-A inter-band CA: Band 1 and Band 3 and Band 8 and Band 8 and Band 38 DL with 2 bands UL

8.1.1	List of specific combination issues

8.1.1.1	Channel bandwidths per operating band for CA

Table 8.1.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-8A-38A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

8.1.1.2	Co-existence studies for LTE-A UL CA_1A-3A, UL CA_1A-8A, UL CA_3A-8A and DL CA_1A-3A-7A-8A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.2-1,Table 8.1.1.2-2 and Table 8.1.1.2-3.

Table 8.1.1.2-1: Harmonic and IMD analysis for UL_CA_1A-3A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Based on Table 8.1.1.2-1, the 3rd IMD products by Band 1 and Band 3 will interfere own Rx frequency of Band 1. There are no harmonic or IMD products from Band 1 and Band 3 uplinks which would desensitize Band 7 and Band 8 and Band 38 downlink.

Table 8.1.1.2-2: Harmonic and IMD analysis for UL_CA_1A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on Table 8.1.1.2-2, 4th IMD products by Band 1 and Band 8 falls into the own Rx frequency of Band 1,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3, and the 3nd harmonic from Band 8 falls into Rx frequency of Band 7.

Table 8.1.1.2-3: Harmonic and IMD analysis for UL_CA_3A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

Based on Table 8.1.1.2-3, 2nd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 7,and 3th IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 38,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3, and the 3nd harmonic from Band 8 falls into Rx frequency of Band 7.

8.1.1.3	MSD

When uplink CA configurations CA_1A-3A, CA_1A-8A, CA_3A-8A is paired with downlink CA configuration CA_1A-3A-7A-8A-38A there are no additional analysis of MSD because it is solved in fallback band combination..

8.1.1.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are shown in table 8.1.1.4-1 and table 8.1.1.4-2 according to CA_1-3-7-8-40 relaxation values, respectively.

Table 8.1.1.4-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-8A-38A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 8 | 0.3 |
|  | 38 | 0.9 |

Table 8.1.1.4-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-8A-38A | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0 |
|  | 38 | 0 |

8.2	LTE-A inter-band CA: Band 1 and Band 3 and Band 8 and Band 20 and Band 38 DL with 2 bands UL

8.2.1	List of specific combination issues

8.2.1.1	Channel bandwidths per operating band for CA

Table 8.2.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-8A-20A-38A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

8.2.1.2	Co-existence studies for LTE-A UL CA_1A-3A ,UL CA_1A-8A ,UL CA_3A-8A and DL CA_1A-3A-8A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.2.1.2-1, Table 8.2.1.2-2 and Table 8.2.1.2-3.

Table 8.2.1.2-1: Harmonic and IMD analysis for UL_CA_1A-3A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Based on Table 8.2.1.2-1, the 3rd IMD will interfere own Rx frequency of Band 1. There are no harmonic or IMD products from Band 1 and Band 3 uplinks which would desensitize Band 8 and Band 20 and Band 38 downlink.

Table 8.2.1.2-2: Harmonic and IMD analysis for UL_CA_1A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on Table 8.2.1.2-2, 4th IMD products by Band 1 and Band 8 falls into the own Rx frequency of Band 1 and Band 20,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3.

Table 8.2.1.2-3: Harmonic and IMD analysis for UL_CA_3A-8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

Based on Table 8.2.1.2-3, 2nd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 20 and Band 38,and 3rd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 38, and 5th IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 20,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3.

8.2.1.3	MSD

When uplink CA configurations CA_1A-3A, CA_1A-8A and CA_3A-8A is paired with downlink CA configuration CA_1A-3A-8A-20A-38A there is no additional analysis of MSD because it is solved in fallback band combination.

8.2.1.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are shown in table 8.2.1.4-1 and table 8.2.1.4-2 according to CA_1-3-7-8-20 relaxation values, respectively.

Table 8.2.1.4-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-20A-38A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 38 | 0.6 |

Table 8.2.1.4-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-20A-38A | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |

Annex A: Change history

|  | Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | Old version | New version |
| 2020-08 | 3GPP RAN4#96e | R4-2010091 |  |  |  | TR skeleton for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17 | N/A | 0.0.1 |
|  |  | R4-2011894 |  |  |  | TR 36.717-03-02 v0.1.0 for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17 | 0.0.1 | 0.1.0 |
| 2021-02 | 3GPP RAN4#98e | R4-2100265 |  |  |  | TR 36.717-03-02 v0.2.0 for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17 | 0.1.0 | 0.2.0 |
|  |  | R4-2103098 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-8A for CA_1A-8A-38A |  |  |
| 2021-04 | 3GPP RAN4#98BIS-e | R4-2104962 |  |  |  | TR 36.717-03-02 v0.3.0 for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17 | 0.2.0 | 0.3.0 |
|  |  | R4-2104963 |  |  |  | TP on summary of self-interference analysis for new x bands (x=3,4,5) DL with 2 bands UL CA |  |  |
|  |  | R4-2105147 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-3A for CA_1A-3A-20A-38A and CA_1A-3C-20A-38A |  |  |
|  |  | R4-2105148 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-3A for CA_1A-3A-7A-38A |  |  |
|  |  | R4-2105155 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-8A for CA_1A-7A-8A-38A |  |  |
|  |  | R4-2105157 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-8A for CA_1A-8A-20A-38A |  |  |
|  |  | R4-2105161 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_3A-8A for CA_3A-7A-8A-38A |  |  |
|  |  | R4-2105163 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_3A-8A for CA_3A-8A-20A-38A |  |  |
|  |  | R4-2105231 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_3A-8A for CA_3A-8A-20A and CA_3C-8A-20A |  |  |
|  |  | R4-2105227 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-3A for CA_1A-3A-7A-8A-38A |  |  |
|  |  | R4-2105228 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-3A for CA_1A-3A-8A-20A and CA_1A-3C-8A-20A |  |  |
|  |  | R4-2105229 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-3A for CA_1A-3A-8A-20A-38A |  |  |
|  |  | R4-2105230 |  |  |  | TP for TR 36.717-03-02: to add UL configuration CA_1A-8A for CA_1A-8A-20A |  |  |
|  |  | R4-2106376 |  |  |  | TP for TR 36.717-03-02_CA_1A-3A-20A-38A |  |  |
| 2021-08 | 3GPP RAN4#100-e | R4-2112711 |  |  |  | TR 36.717-03-02 v0.4.0 for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17 | 0.3.0 | 0.4.0 |
|  |  | R4-2114755 |  |  |  | TP on summary of self-interference analysis for new x bands (x=3,4,5) DL with 2 bands UL |  |  |
|  |  | R4-2112923 |  |  |  | TP for 36.717-03-02: CA_1A-7A-20A-38A |  |  |
|  |  | R4-2114757 |  |  |  | TP for 36.717-03-02: CA_1A-20A-38A |  |  |
|  |  | R4-2112925 |  |  |  | TP for 36.717-03-02: CA_3A-20A-38A |  |  |