3GPP TSG-RAN WG4 Meeting # 100-e                              R4-2112733

Electronic Meeting, 16– 27 August, 2021

Source: 	LG Electronics, SK Telecom, LG Uplus, Ericsson

Title: 	TP for TR 37.717-21-11 DC_1-5_n77, DC_1-7_n77, DC_3-5_n77, DC_3-7_n77 and DC_5-7_n77

Agenda Item:	8.15.2

Document for:	Approval

Introduction

The WID for NR DC was updated in RAN #92 meeting. This contribution provides a TP for TR 37.717-21-11 to introduce the UE RF requirements for the new DC band combinations.

- Text Proposal

<TP for TR 37.717-21-11>

- 5.x	DC_1-5_n77

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-5A_n77A | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n77(2A) | DC_1A_n77A DC_5A_n77A |

5.x.2	Co-existence studies

Based on co-existence studies of DC_1_n77 and DC_5_n77, own Rx impact of the 3rd band is the followings.

-	5th order IMD products generated by DC_1_n77 uplink may fall into own Rx of Band 5.

-	3rd order IMD products generated by DC_5_n77 uplink may fall into own Rx of Band 1.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.x.3	∆TIB and ∆RIB values

For DC_1-5_n77, the TIB,c and RIB,c values are reused from DC_1-5_n78, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-5_n77 | B1 | 0.3 |
|  | B5 | 0.6 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-5_n77 | B1 | 0.2 |
|  | B5 | 0.2 |
|  | n77 | 0.5 |

5.x.4	Reference sensitivity exceptions

For the DC_1A-5A_n77A and DC_1A-5A_n77(2A), RAN4 can reuse the MSD results of the DC_1A-5A_n78A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.x.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-5A_n77A DC_1A-5A_n77(2A) | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n77 | 3405 | 10 | 50 | 3405 | N/A | N/A |

- 5.x	DC_1-7_n77

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n77(2A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A-n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A-n77(2A) | DC_1A_n77A DC_7A_n77A |

5.x.2	Co-existence studies

Based on co-existence studies of DC_1_n77 and DC_7_n77, own Rx impact of the 3rd band is the followings.

-	4th & 5th order IMD products generated by DC_1_n77 uplink may fall into own Rx of Band 7.

-	4th order IMD products generated by DC_7_n77 uplink may fall into own Rx of Band 1.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.x.3	∆TIB and ∆RIB values

For DC_1-7_n77, the TIB,c and RIB,c values are reused from DC_1-7_n78, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n77 | B1 | 0.6 |
|  | B7 | 0.6 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n77 | B1 | 0.2 |
|  | B7 | 0.2 |
|  | n77 | 0.5 |

5.x.4	Reference sensitivity exceptions

For the DC_1A-7A_n77A, DC_1A-7A_n77(2A), DC_1A-7A-7A_n77A and DC_1A-7A-7A_n77(2A), RAN4 can reuse the MSD results of the DC_1A-7A_n78A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.x.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n77A DC_1A-7A_n77(2A) DC_1A-7A-7A_n77A DC_1A-7A-7A_n77(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | N/A |

- 5.x	DC_3-5_n77

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-5A_n77A | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n77(2A) | DC_3A_n77A DC_5A_n77A |

5.x.2	Co-existence studies

Based on co-existence studies of DC_3_n77 and DC_5_n77, own Rx impact of the 3rd band is the followings.

-	No IMD problems by DC_3_n77 uplink in the own Rx of Band 5.

-	3rd order IMD products generated by DC_5_n77 uplink may fall into own Rx of Band 3.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.x.3	∆TIB and ∆RIB values

For DC_3-5_n77, the TIB,c and RIB,c values are reused from DC_3-5_n78, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-5_n77 | B3 | 0.6 |
|  | B5 | 0.6 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-5_n77 | B3 | 0.2 |
|  | B5 | 0.2 |
|  | n77 | 0.5 |

5.x.4	Reference sensitivity exceptions

For the DC_3A-5A_n77A and DC_3A-5A_n77(2A), RAN4 can reuse the MSD results of the DC_3A-20A_n78A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.x.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-5A_n77A DC_3A-5A_n77(2A) | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 5 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n77 | 3510 | 10 | 50 | 3510 | N/A | N/A |

- 5.x	DC_3-7_n77

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-7A_n77(2A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77(2A) | DC_3A_n77A DC_7A_n77A |

5.x.2	Co-existence studies

Based on co-existence studies of DC_3_n77 and DC_7_n77, own Rx impact of the 3rd band is the followings.

-	5th order IMD products generaged by DC_3_n77 uplink may fall into own Rx of Band 7.

-	3rd & 4th order IMD products generated by DC_7_n77 uplink may fall into own Rx of Band 3.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.x.3	∆TIB and ∆RIB values

For DC_3-7_n77, the TIB,c and RIB,c values are already specified in TS38.101-3 as follow.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n77 | B3 | 0.6 |
|  | B7 | 0.6 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n77 | B3 | 0.2 |
|  | B7 | 0.2 |
|  | n77 | 0.5 |

5.x.4	Reference sensitivity exceptions

For the DC_3A-7A_n77(2A) and DC_3A-7A-7A_n77(2A), RAN4 can reuse the MSD results of the DC_3A-7A_n77A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.x.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |  |
| DC_3A-7A_n77(2A) DC_3A-7A-7A_n77(2A) | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | N/A |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |

- 5.x	DC_5-7_n77

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-7A_n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A_n77(2A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A-n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A-n77(2A) | DC_5A_n77A DC_7A_n77A |

5.x.2	Co-existence studies

Based on co-existence studies of DC_5_n77 and DC_7_n77, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_5_n77 uplink may fall into own Rx of Band 7.

-	2nd & 3rd order IMD products generated by DC_7_n77 uplink may fall into own Rx of Band 5.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.x.3	∆TIB and ∆RIB values

For DC_5-7_n77, the TIB,c and RIB,c values are reused from DC_5-7_n78, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7_n77 | B5 | 0.6 |
|  | B7 | 0.6 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7_n77 | B5 | 0.2 |
|  | B7 | 0.2 |
|  | n77 | 0.5 |

5.x.4	Reference sensitivity exceptions

For the DC_5A-7A_n77A, DC_5A-7A_n77(2A), DC_5A-7A-7A_n77A and DC_5A-7A-7A_n77(2A), RAN4 can reuse the MSD results of the DC_5A-7A_n78A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.x.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-7A_n77A DC_5A-7A_n77(2A) DC_5A-7A-7A_n77A DC_5A-7A-7A_n77(2A) | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | N/A |

<End of TP >