3GPP TSG-RAN WG4 Meeting #100-e	R4-2112756

Electronic Meeting, 16 August – 27 August 2021

Source:	Ericsson, Telstra

Title:	TP for TR 38.717-04-02: CA_n3-n5-n7-n78

Agenda item:			8.12.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-04-02 to include CA_n3A-n5A-n7A-n78A and CA_n3A-n5A-n7B-n78A as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 5.2.x	CA_n3-n5-n7-n78

#### 5.2.x.1	Operating bands for CA

Table 5.2.x.1-1: Inter-band CA operating bands of CA_n3-n5-n7-n78

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n3-n5-n7-n78 | n3, n5, n7, n78 |

#### 5.2.x.1	Channel bandwidths per operating bands for CA

Table 5.x.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n5A-n7A-n78A | CA_n3A-n5A CA_n3A-n7A CA_n3A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n5A-n7B-n78A | CA_n3A-n5A CA_n3A-n7A CA_n3A-n78A CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

#### 5.2.x.3		UE co-existence studies

The coexistence studies have been captured into the constituent fallback modes in TR 38.717-03-02, there is no additional harmonic and intermodulation impact for the additional band receiver.

#### 5.2.x.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c  could reuse the values for CA_n3-n5-n7-n78 with 1UL that have been captured  into TR 38.717-04-01.

#### 5.2.x.5	REFSENS requirements

There is no need to specify additional MSD requirement for this UL CA configuration.

---End of changes---

# Reference

[1] 		RP-211194, “Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for DL 4 bands and 2UL bands”, Samsung