3GPP TSG-RAN4 Meeting #100-e	R4-2112777

# Online, , 16th May 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on ambiguity in deciding TL,C R15 CATF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-06-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 received an LS on ambiguity in deciding TL,C from RAN5 R5-206676. This CR adresses the double counting of band edge relaxation which was discuvered by RAN5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | dTc is removed from relevant PCMAX_L,f,c formulas. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Double counting of band edge relaxation remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.2.4, 6.2A.4.1.3, 6.2D.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************** Start of changes ******************************************

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Δ Frequency of default TX-RX separation of the FDD operating band

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RBΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max( BWGB,Channel(k) )

BWGB,Channel(k)	Minimum guard band defined in clause 5.3A.1 of carrier k

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,Shift	RF reference frequency for Supplementary Uplink (SUL) bands and for the uplink for all FDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

GBChannel	Minimum guard band defined in clause 5.3.3

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj*2μj for carrier 1 to j, where μ is defined in TS 38.211 [6]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211 [6]

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in section 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in section 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance)

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart	Indicates the lowest RB index of transmitted resource blocks

SCSc	SCS for the component carrier c

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

SCSlow	SCS for the lowest assigned component carrier in section 5.3A.1

SCShigh	SCS for the highest assigned component carrier in section 5.3A.1

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in section 6.2.1

SSREF	SS block reference frequency position

UTRAACLR	UTRA ACLR

****************************** No changes ******************************************

### 6.2.4	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1-1 without taking into account the tolerance specified in the Table 6.2.1-1;

When  the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 capable UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less symbols in certain evaluation period are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 capable UE operating in TDD bands n40, n41, n77, n78, and n79 with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass = 3 dB for a power class 2 capable UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evalutation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); otherwise ΔPPowerClass = 0 dB;

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

MPRc and A-MPRc for serving cell c are specified in clause 6.2.2 and clause 6.2.3, respectively;

∆TRxSRS is applied when

a)	UE transmits SRS to other than first SRS port when the SRS-TxSwitch capability is indicated as  '1T2R', '1T4R' or, '1T4R/2T4R'

b)	UE transmits SRS to other than first or second SRS port when the SRS-TxSwitch capability is indicated as '2T4R' or '1T4R/2T4R, or'

c)	UE transmits SRS to a DL-only carrier.

The value of ∆TRxSRS is 4.5dB for n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 in the band.  The value of ∆TRxSRS is 7.5dB for n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 2 in the band.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the power management maximum power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum  value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4-1. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1-1.

Table 6.2.4-1: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

****************************** No changes ******************************************

##### 6.2A.4.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in clause 6.2.4.

For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in clause 6.2.2 and clause 6.2.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink inter-band carrier aggregation with one serving cell c per operating band when same slot symbol pattern is used in all aggregated serving cells,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c,  pPowerClass,c/(MAX(mprc,a-mprc) ·tIB,c·tRxSRS,c) , pPowerClass,c/pmprc], PEMAX,CA, PPowerClass,CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.3-1 without taking into account the tolerance specified in the Table 6.2A.1.3-1;

-	pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 without taking into account the tolerance;

-	mpr c and a-mpr c are the linear values of MPR c and A-MPR c as specified in clause 6.2.2 and clause 6.2.3, respectively;

-	pmprc is the linear value of P-MPR c;

-	∆tRxSRS,c  is the linear value of ∆TRxSRS,c;

-	tIB,c  is the linear value of the inter-band relaxation term TIB,c of the serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; otherwise tIB,c In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.For uplink inter-band carrier aggregation with one serving cell c per operating band when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in clause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA }

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA }

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.

TREF and Teval are specified in Table 6.2A.4.1.3-0 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

Table 6.2A.4.1.3-0: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.1.3-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1-2 for inter-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or symbol pattern, shall be within the following range:

P'CMAX_L–  MAX{TL, TLOW (P'CMAX_L)} ≤  P'UMAX  ≤  P'CMAX_H + THIGH (P'CMAX_H)

P'UMAX = 10 log10 ∑ p'UMAX,c

where p'UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P'CMAX) and THIGH(P'CMAX) for applicable values of P'CMAX are specified in Table 6.2A.4.1.3-1 for inter-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1 for inter-band carrier aggregation.

where:

P'CMAX_L  = MIN{ MIN {10log10∑( pCMAX_L,f,c(i),i), PPowerClass,CA} over all overlapping slots in TREF}

P'CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c , PPowerClass,CA} over all overlapping slots in TREF}

Table 6.2A.4.1.3-1: PCMAX tolerance for uplink inter-band CA (two bands)

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| PCMAX = 23 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

****************************** No changes ******************************************

### 6.2D.4	Configured transmitted power for UL MIMO

For UE supporting UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in clause 6.2.4 shall apply to UE supporting UL MIMO, where

-	PPowerClass and ΔPPowerClass are specified in clause 6.2D.1;

-	MPRc is specified in clause 6.2D.2;

-	A-MPRc is specified in clause 6.2D.3.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2D.1-1 for the applicable operating band.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the tolerance is specified in Table 6.2D.4-1. The requirements shall be met with UL MIMO configurations specified in Table 6.2D.1-2.

Table 6.2D.4-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.4 apply.

****************************** End of changes ******************************************