3GPP TSG-RAN WG4 Meeting#100e   	R4-2112782

E-meeting, 16th – 27th  Aug, 2021

Title:	Release independence aspect of 6-band LTE CA

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	11.3.1

Document for:	Approval

# 1	Introduction

Current 36.101 has already 6-band LTE CA specified but 36.307 does not mention release independence aspect yet.

# 2	Discussion

As can be seen from table below two six band LTE CA configurations have been specified. Both of them are also six CC configurations.

Table 5.6A.1-2d: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (six bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-8A-20A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 110 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-7A-8A-20A-28A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 110 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

From next table below (from 36.307) we can see that currently release independent aspect has been defined for up to 5-bands and 7-CCs which are release independent from REL-14

Table 3A.2-2: Inter-band CAconfigurations

| Feature | DL/UL | number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 36.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations | DL | 2 | 2-4 | A, B, C | FDD, TDD | Rel-10 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 2-5 | D, E | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 2-5 | A, B, C, D, E | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 | A, C, D, E, F | FDD, TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 |  | FDD and TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  | 3 | 3 | A | FDD, TDD | Rel-10 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 3-5 | B, C, D | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 3 | A | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 | A, C, D, E, F | FDD, TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 |  | FDD and TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  | 4 | 4-5 | A, C | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 4-5 |  | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 | A, C, D, E | FDD, TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 |  | FDD and TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  | 5 | 5 | A | FDD, TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 5 |  | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 | A, C, D | FDD, TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 |  | FDD and TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  | UL | 2 | 2-4 | A, C | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 2-3 | A, C | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |

As it is already agreed that up to 7-CCs is release independent from REL14 we propose following.

Proposal: 6-band and 6-CC LTE CA configurations are release independent from REL-14.

# 3	Conclusion

In this contribution we have made following proposal.

Proposal: 6-band and 6-CC LTE CA configurations are release independent from REL-14.