3GPP TSG-RAN WG4 Meeting#100-e	R4-2112818

# Electronic meeting, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to modified MPR behaviour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Incorrect conditions for bits indicating funtionality in v16.2.0 in the field modifiedMPRbehavior (all defined in Rel-15). Modified MPR behaviour introduced in an earlier release (Rel-15) is mandatory in a later release (Rel-16), the functionality indicated by the said bits must be supported by a UE compliant with this version (v16.8.0) of the specification. The modified behaviour for any one of the said bits does not address early Rel-16 implementations but is a change that can be supported in the earlier release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex H: “may set” is changed to “shall set” for the bits defined for n257, n258, n260 and n261 all introduced in the Rel-15 version of 38.101-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mandatoty functionality may not be indicated for a given band. The optionallity of setting the bits would contradict the main body of this (Rel-16) specification in which the functionality is specified (mandatory if the band is supported). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

Annex H (Normative): Modified MPR behavior

# H.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [13] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination. Moreover, the bits in the field can explicitly indicate NS value(s) supported by a UE.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [13] by an 8-bit bitmap per supported NR band.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit shall be set to 1 by a UE supporting n257 |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit shall be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.11.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
| n260 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit shall be set to 1 by a UE supporting n260 |
| n261 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit shall be set to 1 by a UE supporting n261 |

Annex I (informative): Void

< end of changes >