3GPP TSG-RAN WG4 Meeting # 100-e		R4-2112871

# Electronic Meeting, 16-27 August 2021

Source:	Sony, Ericsson

Title:	Remaining core part requirement for FWA

Agenda item:	8.40.1

Document for:	Approval

- Background

The WI is to specify the requirements on FWA UE, maintaining the max EIRP of 43dBm and max TRP of 23dBm upper power limitation, and to specify corresponding RF requirements for such kind of UE type [1]. The WI was discussed in in RAN4 #99-e [3] and a WF was agreed [4] where Multi-band relaxations and spherical coverage requirement (gain drop) were agreed but Min. peak EIRP, REFSENS and Beam correspondence requirements were not agreed (FFS).

#### 2.1.2	Remaining Open issues

- Remaining core part (UE RF requirements):

  - Min. peak EIRP requirement

  - REFSENS requirement

- Completion of requirements in 38.101-3

- Completion of requirements in 38.101-4

- Completion of requirements 38.133

Completion of requirements 38.307

Remaining items are summarized in the Status Report to TSG [2] (copied below).

In this tdoc we will focus on remaining core part (UE RF requirements): Min. peak EIRP and REFSENS requirements.  Beam correspondence requirements will also be discussed.

- Discussion

## RF architecture

When specifying PC5 FWA it is appropriate to look at PC1 specification, and also FWA PC1 devices on the market, which could be based on architectures of 32 or even 64 antenna elements. An additional constraint for PC5 (compared to PC1) is the lower maximum allowed TRP (23dBm) in comparison to expected peak EIRP, and thus, a higher antenna gain is preferred. Therefore, our estimation of min peak EIRP (and REFSENS) is based on16 respectively 32 antenna elements where we believe 16 elements should be used as the baseline in RAN4.

Observation 1	Due to the maximum allowed TRP of 23 dBm it is reasonable to use 16 antenna elements as the baseline for min Peak EIRP estimation.

## Min Peak EIRP

The two options to derive min Peak EIRP: based on scaling from agreed values or from budget-based proposals remain from previous discussions [4].

We base the estimation on link budget calculation. Min Peak EIRP for PC5 for band n259 based on 16 respectively 32 antenna elements is shown in Table 1.

| Parameter | Unit | Nom. | Tol. | Nom. | Tol. |
| --- | --- | --- | --- | --- | --- |
|  |  | PC5 |  | PC5 |  |
| Frequency range | GHz | 39.5 43.5 |  |  |  |
| Pout per element | dBm | 9 | 0.5 | 8 | 0.5 |
| # of antennas in an array |  | 16 |  | 32 |  |
| Total conducted power per polarization | dBm | 21 |  | 23 |  |
| Avg antenna element gain | dBi | 5.5 | 0.5 | 5.5 | 0.5 |
| Antenna roll off loss versus frequency | dB | 2 |  | 2 |  |
| Array factor correction (loss) | dB | 0.5 |  | 0.5 |  |
| Realized antenna array gain | dBi | 15 | 0.6 | 18.1 | 0.6 |
| Polarization gain | dB | 2.5 |  | 2.5 |  |
| Mismatch and transmission line loss including load pull | dB | 2 | 0.5 | 2 | 0.5 |
| Beam forming loss (phase shifter and amplitude error) | dB | 1.5 | 0.5 | 1.5 | 0.5 |
| Finite beam table | dB |  |  |  |  |
| Beam forming loss (one beam table fits all) | dB |  |  |  |  |
| Form factor integration loss (incl. radome loss) | dB | 2.5 | 1.5 | 2.5 | 1.5 |
| Total implementation loss (nominal) | dB | 6 |  | 6 |  |
| Total implementation loss (worst case) | dB | 8.5 |  | 8.5 |  |
| Peak EIRP (Nominal) | dBm | 33.1 |  | 37.6 |  |
| Tolerance (+/-) | dB | 4.1 |  | 4.1 |  |
| Peak EIRP (Minimum) | dBm | 28.5 |  | 33.5 |  |
| TRP (Maximum) | dBm | 21.0 |  | 23.0 |  |

Table 1 Estimation on Peak EIRP for PC5: n259.

Proposal 1	According to our estimate minimum peak EIRP for PC5, n262, shall be no less than 28.5 dBm.

A conservative calculation of maximum TRP shows that based on 16 or 32 antenna elements it is possible to fulfill the regulatory requirement of maximum 23 dBm TRP.

Observation 2	 It is possible to fulfill maximum TRP 23 dBm even with a minimum peak EIRP of 33.5 dBm.

## Reference sensitivity power level

Calculation of REFSENS for 50MHz band n259 PC5 is shown in Table 1 for both 16 element array and 32 element array.

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
|  |  | PC5 | PC5 |
| Band number |  | n259 | n259 |
| Frequency range | GHz | 39.5 – 43.5 | 39.5 – 43.5 |
| Modulation |  | QPSK | QPSK |
| SNR requirement | dB | -1 | -1 |
| Implementation margin (IM) | dB | 1 | 1 |
| Bandwidth | MHz | 50 | 50 |
| Thermal noise | dBm/Hz | -174 | -174 |
| Noise Figure (NF) | dB | 12 | 12 |
| Number of elements in an array |  | 16 | 32 |
| Array gain | dB | 10.9 | 14.0 |
| Element gain | dBi | 5 | 5 |
| Diversity gain | dB | 0 | 0 |
| Antenna gain roll-off over frequency | dB | 2 | 2 |
| Beamforming loss | dB | 2 | 2 |
| Mismatch and Transmission line loss | dB | 2.5 | 2.5 |
| Form factor integration losses | dB | 4 | 4 |
| REFSENS | dBm | -90.5 | -93.5 |

Table 2 Estimation on REFSENS for PC1, PC2 and PC4: n262.

Proposal 2	According to our estimate REFSENS for PC5, n259, shall be -90.5 dBm.

## Beam correspondence

Beam correspondence requirements were also discussed in RAN4 #99-e without consensus [4]. In this section we give our view, starting with some background.

The bit 0/1 concept was introduced in RAN #82 [6], to facilitate the design of the early generation the Rel-15 FR2 handheld devices. It was extensively discussed in the following RAN4 meetings. The problem description (described in e.g., [7]) led to agreed simulation assumptions [8] where main contribution to error in BC (wrong beam decision by UE) was based on poor RSRP estimation. For the conformance test of BC, RAN4 decided to ensure good enough SNR in the test procedure, in order for a too poor SNR not to infer conformance test. If the SNR drops below this value (6dB), which is likely to happen sometimes in reality, the ability of the UE to do a proper BC is strongly related to the low SNR.

For the handheld UE, the motivation for defining bit-0 BC is due to that the L1-RSRP is highly impacted by the SNR condition where the UE cannot estimate the L1-RSRP accurately, which may lead to an error in beam selection. However, FWA devices generally operate under a clear line of sight (LOS condition), which shall be accompanied by a good SNR condition. In addition, the wireless channel between the gNB and FWA UE is usually relatively stable and predictable. Therefore, we don’t foresee there would be an issue for the FWA devices to obtain an accurate RSRP estimation supporting bit-1 BC solely.

Observation 3 	The SNR condition for FWA devices is likely to be good and stable, and thus an FWA device should obtain a good RSRP estimation.

Another factor that may degrade the BC is the RF component impairment, typically errors in phase shifters. However, that error has already been included in the peak EIRP and spherical coverage requirement. For example, the phase shifter errors have been included in the peak EIRP derivation for PC1 according to Table 7.2.1.1.1-1 in TR38.817. Concerns have been raised that the beam forming error will grow when the number of phase shifters (i.e. number of antenna elements) increase. We don’t believe the total error introduces by the phase shifters necessarily will grow when the number of antenna elements increases, rather the contrary. Taking all this into account, we believe an FWA UE shall be able to achieve spherical coverage and peak EIRP solely relying on bit-1 BC.

Observation 4	The degradation due to the phase shifter errors have been included in the peak EIRP and spherical coverage requirement.

Finally, we would like to question how meaningful the bit-0/bit-1 BC capability is in general? In real networks, in poor SNR/SINR conditions, the probability that a UE may make RSRP estimation errors increases, and thus the UE may fail to select an optimal uplink beam autonomously, regardless of the UE BC capability. On the other hand, the SNR/SINR may also sometime be very high in a real network scenario, and a UE that has set its UE BC capability bit to 0 may be capable of selecting an optimal uplink beam autonomously under such a condition. The beam correspondence tolerance was introduced in Rel-15 in order to accommodate the lower capability of some UEs to select the uplink beam autonomously and ease the way of early launching of FR2 UEs. However, in light of the discussion above, it is questionable whether it is useful for the network to know such a UE capability.

Observation 5	The beam correspondence depends on the SNR condition. Therefore, it is questionable whether it is useful for the network to know a UE BC capability with bit-1 or bit-0.

Proposal 3	Define only BC bit 1 requirement for new FWA UE.

Currently, only bit-1 is defined for PC5/PC1. We believe the beam correspondence requirement for n259 shall remain the same as n258 and n257 for PC5.

Proposal 4	Adopt the same beam correspondence requirement (only bit 1) for n259 as for n257 and n258 for PC5.

- Conclusion

In this contribution, we have shared our views on the open issues related to the RF requirement for PC5 FWA for band n259. The following observations and proposals have been given:

Observation 1	Due to the maximum allowed TRP of 23 dBm it is reasonable to use 16 antenna elements as the baseline for min Peak EIRP estimation.

Observation 2	 It is possible to fulfill maximum TRP 23 dBm even with a minimum peak EIRP of 33.5 dBm.

Observation 3 	The SNR condition for FWA devices is likely to be good and stable, and thus an FWA device should obtain a good RSRP estimation.

Observation 4	The degradation due to the phase shifter errors have been included in the peak EIRP and spherical coverage requirement.

Observation 5	The beam correspondence depends on the SNR condition. Therefore, it is questionable whether it is useful for the network to know a UE BC capability with bit-1 or bit-0.

Proposal 1	According to our estimate minimum peak EIRP for PC5, n262, shall be no less than 28.5 dBm.

Proposal 2	According to our estimate REFSENS for PC5, n259, shall be -90.5 dBm.

Proposal 3	Define only BC bit 1 requirement for new FWA UE.

Proposal 4	Adopt the same beam correspondence requirement (only bit 1) for n259 as for n257 and n258 for PC5.

- Reference

RP-210875,	“New WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259”, SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm

RP-211508,	“Status Report to TSG”, RAN4 (Qualcomm)

R4-2107942,	“Email discussion summary for [99-e][132] NR_NewRAT_SysParameters”, Moderator (Qualcomm Incorporated)

R4-2107839,	“WF on PC5 requirements in n259”, Qualcomm Incorporated

R4-2109006, 	“Views on RF requirement for FWA”, Sony, Ericsson

RP-182879,	“WF on Beam Correspondence”, Samsung, Apple, Nokia, Intel, ZTE, Sanechips, Qualcomm, MediaTek, Panasonic, Verizon, CATT, AT&T, OPPO, CMCC, Huawei, HiSilicon, CAICT, vivo, LG Electronics, KT Corp.

R4-1901796, 	“Beam Correspondence, DL reference signal”, Sony

R4-1902684,	“WF on simulation assumptions for BC tolerance requirements”, LG Electronics